3GPP TSG-SA WG6 Meeting #64	S6-245259

Orlando (FL), USA, 18th – 22nd November 2024

Source:	Ericsson

Title:	Enhancement to Spatial Anchor Creation and Discovery

Spec:	3GPP TS 23.437 v0.3.0

Agenda item:	9.11

Document for:	Approval

Contact:	ashish.sanjay.sharma@ericsson.com

1. Introduction

This CR brings enhancements related to the spatial anchor creation procedure.

2. Reason for Change

Enhancing the SA with prioritization information.

Why we need Priority information for Spatial Anchor:

Consider a use case of an industry warehouse order fulfilment centre.

The warehouse relies on AR headsets and spatial anchors to efficiently manage thousands of products. It needs to handle both urgent shipments, like fast and normal shipping, so the warehouse is divided into areas designated for different shipping priorities. Goods for fast shipping are stored in specific zones, while normal shipping goods are in separate areas. These zones may be adjusted dynamically based on warehouse load.

Workers or robots must identify the correct areas for different fulfillment sections, either to store items or pick up the appropriate ones for shipping. Spatial anchors are essential here, as they allow workers to locate, for example, the fast shipping area. It's important that workers or robots can differentiate between spatial anchors—such as identifying the anchor for a fast shipping area versus one for normal shipping. This distinction can be made by assigning priority levels to each spatial anchor.

The sequence below illustrates how anchor prioritization assists in the fulfillment center. During setup, the inventory manager assigns priority information to each spatial anchor. In the warehouse, workers or robots are tasked with handling both urgent deliveries (like prime or one-day shipping) and non-urgent ones (like free shipping).

For instance, the manager assigns high-priority anchors to urgent, fast-moving items, such as electronics or goods requiring fast shipping, while lower-priority anchors are used for less in-demand items or those designated for normal shipping. This prioritization helps workers navigate the warehouse more efficiently. With these prioritized anchors, a worker handling urgent shipping items can quickly locate the correct shelf or designated location to store or retrieve goods.

When a worker puts on the AR headset, the system detects the spatial anchors. High-priority spatial anchors guide the worker directly to shelves or locations for urgent orders (such as smartphones) to be stored or picked up. This enhances operational efficiency, improves flexibility, and minimizes delays in fulfilling warehouse orders.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.437 v0.3.0.

* * * First Change * * * *

#### 8.3.1.2	Procedure

Figure 8.3.1.2-1 illustrates the spatial anchor creation procedure.

Pre-conditions:

1.	The SAn client or VAL server has received information (e.g. URI, IP address) related to the SAn server;

2.	The SAn client or VAL server has received security credentials authorizing it to communicate with the SAn server.

3.	For SAn client to trigger this procedure, the SAn client has received the required information from the VAL client over SAn-C interface.

Figure 8.3.1.2-1: Spatial anchor creation

1.	The requestor (e.g., SAn client or VAL server) sends a spatial anchor create request to the SAn server. The request includes a requestor identifier, security credentials, and list of spatial anchors to be created and priority requirements of the spatial anchor(s) as described in clause 8.3.1.3.1. The request includes access control rules defining which entities are permitted to discover and access the spatial anchor, and discoverability information (e.g., indicating whether the spatial anchor is discoverable by any user). If the request includes spatial anchor specific information, that information remains transparent to the SAn server and can be stored with the spatial anchor. The request can indicate to create multiple spatial anchors.

NOTE:	The spatial anchor specific information is out of scope of standardization. For example, the spatial anchor specific information may contain application specific service information like a name, a URL, a media data, a type, a manufacturing company, a date, an expiry date, a ratings, and customer premise information (e.g. a residence, office or shop), etc. Such information may be stored in key-value pair and is transparent to the SAn server.

2.	Upon receiving the request, the SAn server validates if the requestor is authorized for the request. If the requestor is authorized, the SAn server creates the spatial anchor(s), assigns spatial anchor identifier to the newly created spatial anchor(s) and stores the spatial anchor information.

3.	The SAn server sends a spatial anchor create response to the requestor. If the SAn server created spatial anchors, the response includes an indication of success and spatial anchor identifier(s) of newly created spatial anchor(s). Otherwise, the response includes an indication of failure and can include a reason for failure.

* * * Next Change * * * *

#### 8.3.4.2	Procedure

Figure 8.3.4.2-1 illustrates the procedure for spatial anchor discovery following the request-response model.

Pre-conditions:

1.	The SAn client or VAL server has received information (e.g. URI, IP address) related to the SAn server; and

2.	The SAn client or VAL server has received security credentials authorizing it to communicate with the SAn server.

3.	For SAn client to trigger this procedure, the SAn client has received the required information from the VAL client over SAn-C interface.

Figure 8.3.4.2-1: Spatial anchor discovery

1.	The requestor (e.g., SAn client or VAL server) sends a spatial anchor discovery request message to the SAn server to obtain information about spatial anchors of interest. The request includes a requestor identifier, security credentials, and can include spatial anchor discovery filters as described in clause 8.3.4.3.1. If the discovery request was triggered by a VAL client on a UE, the spatial anchor discovery request can additionally include a VAL client identifier.

2.	Upon receiving the request, the SAn server validates if the requestor is authorized for the request. If the requestor is authorized, the SAn server identifies spatial anchor(s) that satisfies the spatial anchor discovery filters.

NOTE 1:	User preferences in the spatial anchor discovery filters can be compared with spatial anchor specific information.

For each matching spatial anchor, the SAn server checks whether the requestor is authorized to obtain the spatial anchor based on the identity and location of the requestor, and based on the access control rules. To determine the location of interest of the requestor, the SAn server may use location information provided in the request. If location information is not provided in the request, the SAn server may obtain location information by invoking the 3GPP Core Network Location Services as described in 3GPP TS 23.273 [r23273] and 3GPP TS 23.502 [5] directly or via the NEF, or by invoking the SEAL Location Management APIs as described in 3GPP TS 23.434 [4].

3.	The SAn server sends a spatial anchor discovery response to the requestor. If the SAn server has identified spatial anchor(s), the response includes an indication of success and the identified spatial anchors. The discovery response includes the prioritization information of the spatial anchor.  If the recommended spatial anchor indication is provided in the request, then the SAn server uses the discovery filter information and stored spatial anchor information to provide the recommended list of spatial anchors in the discovery response. Otherwise, the response includes an indication of failure and may include a reason for failure. If universal discoverable visibility is indicated in request discovery filter, then the List of spatial anchors IE in the response corresponds to the Discovered universal spatial anchor List.

NOTE 2:	The logic to generate a recommended list of spatial anchors is out of scope and implementation specific.

4.	Upon receiving the spatial anchor discovery response, if the response indicates success, the requestor can store the spatial anchor information in a spatial anchor cache. The requestor evaluates the validity conditions associated with the discovered spatial anchors prior to using the spatial anchor. If the requestor is the SAn client, the SAn client can provide spatial anchor information to the VAL client(s). Upon receiving valid spatial anchor information, VAL client(s) can access the service(s) associated with each spatial anchor.

* * * End of Changes * * * *

#### 8.3.4.3	Information flows

##### 8.3.4.3.1	Spatial anchor discovery request

Table 8.3.4.3.1-1 describes information elements for the spatial anchor discovery request from the SAn client or VAL server to the SAn server.

Table 8.3.4.3.1-1: Spatial anchor discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server or VAL UE and VAL user). |
| Security credentials | M | The security credentials of the requestor. |
| VAL client ID | O | The identifier of the VAL client when the requestor is a SAn client. |
| Discovery filters | O | The filters used for determining the spatial anchors. |
| > List of spatial anchor ID | O | The list of spatial anchor identifiers to discover. |
| > Area of interest | O | The area of interest (e.g., topological area and/or geographical area). |
| > Service ID | O | The identifier of a VAL service. |
| > Universal discoverable visibility | O | Indicates the request to discover spatial anchor marked as universal. Possible values are Yes or No. |
| Recommended spatial anchor indication | O | Indicates the request to get recommended spatial anchors. Possible values areYes or No. Default value is Yes. |

##### 8.3.4.3.2	Spatial anchor discovery response

Table 8.3.4.3.2-1 describes information elements for the spatial anchor discovery response from the SAn server.

Table 8.3.4.3.2-1: Spatial anchor discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the spatial anchor discovery request was successful. |
| List of spatial anchors | O | List of discovered spatial anchors. |
| > Spatial anchor ID | M | Identifier of the discovered spatial anchor. |
| > Position | M | The 3D position of the spatial anchor in space (x, y and z coordinates). |
| > VAL service information | M | Information about the application service associated with the spatial anchor. |
| >> Service ID | M | The identifier of the application service. |
| >> Service endpoint | M | Endpoint information (e.g., URI, FQDN, IP address) used to communicate with the application service. |
| >> Service connection information | O | The connection information (e.g., DNN, DNAI(s)) for establishing connectivity to the application service. |
| > Spatial anchor specific information | O | A container to contain application specific information. |
| > Priority | O | Indicates the prioritization information of the spatial anchor. |
| Recommended spatial anchor List | O | Indicates the recommended spatial anchor(s) |
| Failure response | O | Indicates that the spatial anchor discovery request has failed. |
| > Cause | O | The cause for the request failure. |

* * * Next Change * * * *

##### 8.3.1.3.1	Spatial anchor create request

Table 8.3.1.3.1-1 describes information elements for the spatial anchor create request from the SAn client or VAL server to the SAn server.

Table 8.3.1.3.1-1: Spatial anchor create request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server or VAL UE and VAL user). |
| Security credentials | M | The security credentials of the requestor. |
| VAL service information | M | Information about the application service associated with the spatial anchor. |
| > Service ID | M | The identifier of the application service. |
| > Service endpoint | M | Endpoint information (e.g., URI, FQDN, IP address) used to communicate with the application service. |
| > Service connection information | O | The connection information (e.g., DNN, DNAI(s)) for establishing connectivity to the application service. |
| List of spatial anchors | M | List of spatial anchors to be created. |
| > Position | M | The 3D position of the spatial anchor in space (x, y and z coordinates). |
| > Service area | O | The service area where the spatial anchor is accessible. |
| > Access control rules | O | Access control rules defining which entities are permitted to discover the spatial anchor. If none, the spatial anchor is marked as universal, and it is discoverable by all. |
| > Spatial anchor specific information | O | A container to contain application specific information. |
| > Priority | O | Indicates the prioritization information of the spatial anchor. |

* * * End of Changes * * * *