3GPP TSG-SA6 Meeting #64	S6-245292

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0629 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding recording replay procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Motorola Solutions UK Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2024-11-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Common replay procedures are not specified in the MC specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding common replay procedures in MC specifications. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification for recording and replay feature. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Procedure was discussed in SA#63 |  |  |  |  |  |  |  |  |

# * * * First change * * * *

### 10.18.X	Recording replay procedures

#### 10.18.X.1	Replay procedure for recording content.

Replay service allows fetch and replay of the recorded content from the recording server. Following section lists procedure for supporting replay of recorded contents.

Figure 10.18.X.1-1:  replay procedure

1. 	The MC replay UE is provided with required recording service configuration; this is as per the procedures documented in section 10.1.x.x

2. 	The MC replay UE is provided with required replay user profile; this is as per the procedures documented in section 10.1.x.x

3.	Authentication and authorization of replay user shall be done as per the procedures documented in section 10.6

4.	Post service authentication and authorization, replay user shall be involved in replay service operations.

NOTE:	Details of step-4 is out of scope of this specification as it involves interface between replay client and recording server.

# * * * * End of changes * * * *