3GPP TSG-SA WG6 Meeting #64	S6-245310

Orlando, USA, 18th – 22nd November 2024	(revision of S6-245xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.436 | CR | 0058 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to A-ADRF Service on Analytics Storage |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_ADAES |  |  |  |  |  | Date: |  |  | 2024-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There is an EN left under the data storage subscription request table: Editor’s Note: What IEs be contained in the request and response tables 8.10.3.2-1 and 8.2.4.5-1is FFS. Table 8.10.3.2-1 is for data storage subscription request from consumer (e.g. A-DCCF or ADAE server) to A-ADRF, Table 8.2.4.5-1 is data collection subscription response from Data Producer (e.g. A-DCCF) to the ADAE server. The EN can be removed. Errors exist in the procedure and information flows. Thus, this paper proposes to remove the EN, and correct errors in the procedure and information flows. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes include: - Remove the EN in clause 8.10.3.2. - Correct errors in the procedure and information flows. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Errors exist in the procedure and informaiton flows of A-ADRF Service on Analytics Storage. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.10.2.1, 8.10.2.2, 8.10.3, 8.10.3.1, 8.10.3.2, 8.10.3.4, 8.10.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.10.2.1	Notification based data storage

Figure 8.10.2.1-1 illustrates the procedure for consumer to request A-ADRF to initiate request for data or analytics to store.

Pre-conditions:

1.	Analytics or data consumer requests ADAE server, A-DCCF or data sources to store analytics or data to A-ADRF.

2.	ADAE server, A-DCCF or A-ADRF are configured with default operator storage policies.

Figure 8.10.2.1-1: Support for data storage to A-ADRF

1.	The consumer (A-DCCF or ADAE server) determines that data and/or analytics needs to be stored to the A-ADRF, or requests the A-ADRF subscribes to Data Producer for receiving notifications. The determination may be made based on configuration or information supplied by the data or analytics. As specified in Table 8.10.3.2-1, the request to the A-ADRF specifies the data and/or analytics to which the A-ADRF will subscribe. The request includes identity of the consumer, security credential(s) for authorization and verification, Storage Handling information (e.g. a lifetime for how long the data or analytics should be stored, indicate that a notification alerting the consumer be sent prior to data deletion from the A-ADRF).

2.	The A-ADRF may, based on implementation, determine whether the same data and/or analytics is already stored or being stored, based on the information sent in step 1 by the consumer. Based on Storage Handling information, the A-ADRF determines the Storage Approach (e.g. lifetime for storing data and whether consumer is notified prior to data deletion).

3.	If the data and/or analytics is already stored and/or being stored in the A-ADRF, the A-ADRF sends data storage subscription response message to the consumer indicating that data and/or analytics is stored. As specified in Table 8.10.3.3-1, the A-ADRF includes the Storage Approach if determined in step 2.

4.	The A-ADRF subscribes to the data producer (e.g. A-DCCF, ADAE server) to receive notifications with data or analytics, providing its notification endpoint address and a notification correlation ID.

5.	The data producer sends data or analytics notifications containing the notification correlation ID provided by the A-ADRF to notification endpoint address. The analytics or data notifications shall contain timestamp. The ADRF stores the notifications.

* * * Next Change * * * *

#### 8.10.2.2	Direct data storage

Figure 8.10.2.2-1 illustrates the procedure for the consumer to request A-ADRF for data or analytics storage.

Pre-conditions:

1.	ADAE server or A-DCCF has data or analytics need to store to A-ADRF.

2.	ADAE server, A-DCCF or A-ADRF are configured with default operator storage policies.

Figure 8.10.2.2-1: Support for data storage to A-ADRF

1.	The consumer (e.g. ADAE server, A-DCCF) sends data storage request to the A-ADRF for storing data and/or analytics. The request message includes identity of the consumer, security credential(s) for authorization and verification, collected data with timestamp and/or analytics with timestamp, Storage Handling information (e.g. a lifetime for how long the data or analytics should be stored, indicate that a notification alerting the consumer be sent prior to data deletion from the A-ADRF), as specified in Table 8.10.3.4-1.

2.	Based on Storage Handling information (if available), the A-ADRF determines the Storage Approach (e.g. lifetime for storing data and whether consumer is notified prior to data deletion).

3.	The A-ADRF stores the data and/or analytics sent by the consumer. The A-ADRF may, based on implementation, determine whether the same data and/or analytics is already stored or being stored based on the information sent in step 1 by the consumer and, if the data and/or analytics is already stored or being stored in the A-ADRF, the A-ADRF decides to not store again the data and/or analytics sent by the consumer.

4.	The ADRF sends data storage response message to the consumer indicating that data and/or analytics is stored, whether the A-ADRF determined at step 3 that data or analytics is already stored and the Storage Approach, as specified in Table 8.10.3.5-1.

* * * Next Change * * * *

### 8.10.3	Information flows

#### 8.10.3.1	General

The following information flows are specified for data storage based on 8.10.2.

* * * Next Change * * * *

#### 8.10.3.2	Data storage subscription request

Table 8.10.3.2-1 describes information elements for the data storage subscription request from the consumer (ADAE server, A-DCCF) to the A-ADRF.

Table 8.10.3.2-1: Data storage subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Consumer ID | M | The identifier of the consumer (ADAE server, A-DCCF). |
| Data Producer ID | O | The identifier of the data producer (e.g. ADAE server, A-DCCF). |
| Analytics ID | O | The identifier of the analytics event. This IE shall be provided when Analytics Producer ID is present. |
| Analytics Type | O | The type of analytics for the event, which include category (e.g. statistics or predictive analytics), may include mode (e.g. offline or online) and indication on ML-enabled analytics. This IE shall be provided when Analytics Producer ID is present. |
| Target Data Producer Profile Criteria | O | Characteristics of the data producers to be used. This IE shall be provided when Data Producer ID is present. |
| Area of Interest | O | The geographical or service area for which the subscription request applies. |
| Time Validity | O | The time validity of the subscription request to the A-ADRF. |
| Storage Handling information | O | The information for storage handling. |
| > Lifetime | O | A lifetime for how long the data or analytics should be stored. |
| > Alert Indicator | O | To indicate that a notification alerting the consumer be sent prior to data deletion from the A-ADRF. |

* * * Next Change * * * *

#### 8.10.3.4	Data storage request

Table 8.10.3.4-1 describes information elements for the data storage request from the consumer (ADAE server, A-DCCF) to the A-ADRF.

Table 8.10.3.4-1: Data storage request

| Information element | Status | Description |
| --- | --- | --- |
| Consumer ID | M | The identifier of the consumer (ADAE server, A-DCCF). |
| Data Type | M | To indicate the data type (e.g. data or analytics). |
| Content | M | The data or analytics to be stored. |
| Timestamp | M | The timestamp of the data or analytics. |
| Analytics ID | O | The identifier of the analytics event. This IE shall be provided if the Storage Type is analytics. |
| Analytics Type | O | The type of analytics, which include category (e.g. statistics or predictive analytics), may include mode (e.g. offline or online) and indication on ML-enabled analytics. This IE shall be provided if the Data Type is analytics. |
| Data Source Information | O | Information of the data source. This IE shall be provided if the Data Type is data. |
| Storage Handling information | O | The information for storage handling. |
| > Lifetime | O | A lifetime for how long the data or analytics should be stored. |
| > Alert Indicator | O | To indicate that a notification alerting the consumer be sent prior to data deletion from the A-ADRF. |

* * * Next Change * * * *

#### 8.10.3.5	Data storage response

Table 8.10.3.5-1 describes information elements for the data storage response from the A-ADRF to the consumer (ADAE server, A-DCCF).

Table 8.10.3.5-1: Data storage response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the data storage request (positive or negative acknowledgement). |
| Storage Approach | M | Represent the Storage Approach (e.g. lifetime for storing data, and whether consumer is notified prior to data deletion). |

* * * END of Changes * * * *