3GPP TSG-SA6 Meeting #64	S6-245536

# Orlando, USA, 18th - 22nd November 2024	(revision of 5063, 4680, 4553, 4202)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0205 | rev | 4 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Requesting of collective resource owner authorization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple (Ulanqab), Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2024-10-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Solution#12 & #13 of 3GPP TR 23.700-22 were concluded to be considered for the normative work for addressing the communication between resource owner and Authorization function (CCF) over CAPIF-8. This CR proposes the implement them 3GPP TS 23.222. The two solution are motivated by reducing the required number of interactions between the CCF and RO, for instance when multiple API invokers seek access to a particular resource in a similar time period. The decision to make a request with a wider authorization scope is based on CCF configuration, which includes the case where there is an association between certain the API exposing functions and API invokers, or when an interaction with a RO should only be performed at certain times. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to add a new clause (8.x) regarding obtaining collective authorization combining both solution#12 & #13 of 3GPP TR 23.700-22. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The required number of interactions between the CCF (Authorization Function) and resource owner via the ROF over CAPIF-8 will be needlessly high. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 8.x (new), 8.x.1 (new), 8.x.2 (new) & 8.x.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | In revision 1: Highlighted that the CCF must support the mechanism to obtain bulk authorization from a resource owner for the procedure to apply. Replaced the term consent with authorization with respect to resource owner authorization. Moved CCF configuration to pre-condition and updated the figure accordingly. Highlighted that the ROF is UE hosted. Condensed the description of step 4 (step 3 in the revision). Add Timeperiod in the configuration period. In revision 2: In note 2, replaced “out-of-scope of 3GPP” with “out-of-scope of the present specification”. In revision 3 (SA6#64): Replaced “bulk” with “collective” and replaced the figure to be editable. In revision 4: Replaced “bulk” with “collective” in the figure. Change “critierian” to “criteria” in step 3 (and associated “has” to “have”). |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 8.x	Obtaining collective authorization from a resource owner

### 8.x.1	General

Using the procedure described in clause 8.31 the CAPIF may authorize an API invoker to invoke a service API based on the authorization information from the resource owner given before the API invocation. This clause describes an extension to that procedure to enable the CAPIF to obtain authorization information from the resource owner relating to multiple API invokers and service APIs in a single interaction.

Clause 8.x.3 shows the procedure for obtaining collective authorization information from a resource owner via their resource owner function.

### 8.x.2	Information flows

Editor’s note:	The security aspects of this procedure are to be specified in TS 33.122 [12].

### 8.x.3	Procedure

Figure 8.x.3-1 illustrates the procedure for API invoker obtaining collective authorization from resource owner.

The mechanism to obtain collective authorization from a resource owner is supported by the CAPIF core function.

Pre-conditions:

1.	The resource owner function can communicate with the CCF (Authorization function).

2.	Access to an API exposing function offered service API requires obtaining authorization from a resource owner.

3. 	The API invoker is successfully onboarded as described in clause 8.1.

4.	The CCF is configured for requesting collective authorization from the resource owner, see table 8.x.3-1.

Figure 8.x.3-1: Procedure for API invoker obtaining collective authorization from resource owner

1.	An API invoker sends obtain service API authorization request to the CAPIF core function for obtaining permission to access a service API by including its API invoker identity information and any information required for authentication of the API invoker. The API invoker includes in the request information about the purpose and the targeted resources and operations to be performed on those resources.

2.	The CCF validates the authentication of the API invoker (using authentication information) and checks whether it is permitted to access the requested service API by checking whether a resource owner authorization record is already in place for that API invoker. If the authorization for accessing the requested resources for the signalled purpose is already granted, the procedure continues with step 5.

NOTE 1:	Whether the CCF is allowed to retain resource owner authorization records depends on local regulations.

3. 	If authorization information is not available regarding whether the API invoker is permitted to access the requested service API (i.e., there is no applicable resource owner authorization record available at the CCF) the collective resource owner authorization configuration (table 8.x.3-1) is assessed.

If authorization trigger criteria have been specified as part of the collective resource owner authorization configuration, interactions with the resource owner are only initiated once the trigger conditions are met.

If trigger criteria have not been set or once the trigger conditions are met, the authorization function determines the authorization by interacting with the resource owner via the ROF deployed on a UE. Based on the configured collective resource owner authorization configuration at the CCF and any cached “obtain service API authorization requests”, the scope of the request towards the ROF can be expanded to include additional API invokers and or service APIs based on the assumption that there is an association between the requesting API invoker and the API exposing functions and API invokers listed in the CCF configuration.

Editor’s Note: The detailed procedure to obtain the resource owner's authorization information is in scope of SA3. However, it is recommended that the resource owner is provided with at least the option to only grant authorization for only the API invoker initiating the authorization request and then only for the specific service being requested.

NOTE 2:	The details of the interaction between RO and ROF for capturing the resource owner authorization are considered out-of-scope of the present specification.

4.	Based on the RO provided authorization, the CCF sends an API invoker specific service API authorization response to the API invoker that initially triggered the collective resource owner authorization information requested and to each API invoker for which a obtain service API authorization request was received (for the same service API) whilst the collective resource owner authorization configuration duration timer was running, i.e., the cached requests.

5.	An API invoker that receives the service API authorization response can send service API invocation requests to the API exposing function with their API invoker specific resource owner authorization information.

6.	The API exposing function will send a service API invocation response once it has successfully checked that the API invoker is authorized to invoke that service API based on the authorization information.

Table 8.x.3-1: Collective resource owner authorization configuration

| Information element | Status | Description |
| --- | --- | --- |
| List of API invoker associations (NOTE 1) | O | Lists of API invokers with their associated service APIs |
| > API invoker identity information list | M | List of API invoker identities |
| > Service API identification list | M | List of AEF service API identities. |
| List of trigger criteria (NOTE 1) | O | Criteria for triggering requesting of resource owner authorization |
| > Duration (NOTE 2) | O | Initiate request towards RO via ROF after the given duration (in seconds) after receiving a obtain service authorization request from an API invoker |
| > Timeperiod (NOTE 2) | O | Time period during which notifications may be sent towards RO via ROF. |
| > Resource owner identifier | O | Can only be included if a duration or timeslot is included. Indicates that the resource owner authorization trigger criteria is to be applied to the specific resource owner identified by their GPSI, i.e., no other resource owner authorization trigger criteria shall be applied for that resource owner. |
| NOTE 1: At least one of these attributes shall be present NOTE 2: Only one of these attributes shall be present |  |  |

* * * End of Changes * * * *