3GPP TSG-SA WG6 Meeting #64	S6-245677

Orlando, USA, 18th – 22nd November 2024	(revision of S6-245640, S6-245115)

Source:	Samsung

Title:	Pseudo-CR on implicit subscription for spatial map ready event

Spec:	3GPP TS 23.437 (0.3.0)

Agenda item:	9.11

Document for:	Approval

Contact:	Sapan Shah (sapan.shah@samsung.com)

1. Introduction

This pCR introduces implicit subscription for spatial map ready event.

2. Reason for Change

Following EN was added in previous meeting:

Editor’s Note:	The implcit subscription for notification after "in-progress" response is FFS.

In order to resolve the EN, procedure changes are required to send one time notification to the requestor to notify about spatial map ready event.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.437 (0.3.0).

* * * First Change * * * *

#### 9.3.1.2	Procedure

Figure 9.3.1.2-1 depicts the procedure for creating a spatial map. For the request from VAL server or SEAL SM client, as a spatial map consumer, SEAL SM server creates a spatial map. A spatial map can be structured in layers where each layer signifies a specific aspect of the spatial information (e.g. three dimensional space of area of interest, objects).

Figure 9.3.1.2-1: Create spatial map procedure

1)	The requestor (e.g., VAL server or SEAL SM client) sends a request message to the SEAL SM server to create a spatial map. The request includes requestor ID, security credentials, three-dimensional area of interest, information to be included in the spatial map such as access control rules defining which entities are permitted to discover and access the spatial map. The request may include spatial map layering information parameters and may also include augmented layer information that can be requested with the spatial map.

NOTE:	SEAL SM server can obtain augmented layer information and can provide the obtained augmented layer information to the requestor in the response message.

2)	The SEAL SM server authorizes the requestor, and validates the request. The SEAL SM server produces a requested spatial map using layering information and processed sensor data.

a)	If the augmented layer information indicates to include VAL UE information, the SEAL SM server fetches the list of VAL UEs in the area of interest from LM server using clause 9.3.10 of 3GPP TS 23.434 [4] and/or from NEF as specified in 3GPP TS 23.502 [5].

b)	If the augmented layer information indicates to include spatial anchors information within area of interest, the SEAL SM server fetches the list of spatial anchors in the area of interest as specified in clause 8.3.4.

NOTE:	For the current release, the processed sensor data is stored at VAL layer (e.g., in database at VAL layer) and the SEAL SM server gets access to such database to create a spatial map via application specific way which is out of scope.

3)	The SEAL SM server sends response message to the requestor with a indication of success, in-progress or fail. If the requested spatial map has been created sussesfully, the response message includes assigned spatial map ID and information which includes three-dimensional space defined by the spatial map and can include a list of spatial map layers with their corresponding layer ID, objects belong to the layer, etc.. If the response indicates "in-progress", an assigned spatial map ID is included in the message. Further, if the response indicates "in-progress" and the requestor has not provided a notification target address, then the requestor  subscribes for spatial map event to receive notification when the spatial map is created. Otherwise, the response includes an indication of failure and may include a reason for the failure.

4)	If the response indicates "in-progress", the SM server detects when the SEAL SM server has sufficient information to map all the objects related to area of interest and the spatial map is ready to provide service (e.g. employed for localization). If the requestor has provided a notification target address in the request or has subscribed for spatial map events, the SEAL SM server notifies the requestor with the spatial map ready notification message. The notification includes parameters as specifed in Table 9.3.6.3.3-1.

* * * Next Change * * * *

#### 9.3.1.3	Information flows

#### 9.3.1.3.1	Create spatial map request

Table 9.3.1.3.1-1 descibes the information elements from VAL server (or SEAL SM client) to SM server for create spatial map request.

Table 9.3.1.3.1-1: Create spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or SM client) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Application service ID | M | ID of the requesting VAL application service |
| Area of interest | M | Three-dimensional area information to produce the spatial map |
| Notification Target Address | O | Notification target address (e.g. URL, URI, IP) where the notifications should be sent. |
| Information to include | O | Information to be included in the spatial map |
| > Access control rules | O | Access control rules defining which entities are permitted to discover and access the spatial map |
| > List of spatial map layers | O | List of spatial map layer information. Each element includes the information described below. |
| >> Spatial map layer information | O | Information to specify the corresponding spatial map layer specific information |
| > Augmented layer information | O | List of required augmented layer information. This IE can contain multiple values. Possible values are: - VAL_USERS; - SPATIAL_ANCHORS. |

NOTE 1:	For the current release, the spatial map layer information is implementation specific and out of scope.

Editor’s note: Details of Access control rules along with new terminology is FFS.

##### 9.3.1.3.2	Create spatial map request

Table 9.3.1.3.2-1 descibes the information elements for create spatial map response from SM server to VAL server or SM client.

Table 9.3.1.3.2-1: Create spatial map response

| Information element | Status | Description |
| --- | --- | --- |
| Application service ID | O | ID of the requesting VAL application service |
| Success response | O (NOTE) | Information on the created spatial map. It contains information as specified in Table 9.3.1.3.2-2. |
| > Spatial map information | M | The information about the created spatial map as described in Table 9.3.1.3.2-2. |
| > Augmented layer information | O | Augmented layer information provided by SM server |
| Failure response | O (NOTE) | Indicates failed create spatial map |
| > Cause | O | Cause of the failure |
| NOTE: At least one of the information elements shall be provided. |  |  |

Table 9.3.1.3.2-2: Spatial map information

| Information element | Status | Description |
| --- | --- | --- |
| Spatial map ID | M | The identifier of the spatial map |
| Spatial map coverage | M | The three-dimensional space coordinates information of the spatial map |
| Access control rules | O | Access control rules defining which entities are permitted to discover and access the spatial map |
| List of spatial map layers | M | The list of spatial map layer information. At least one or more spatial map layers should be included. Each element includes the information described below. |
| > Spatial map layer ID | M | The identifier of the corresponding spatial map layer. |
| > Spatial map layer information | O | A container to carry the corresponding spatial map layer specific information |
| >> area | O | The area covered by this layer. This area can be the same or a subset of the Spatial map coverage space. |
| >> associated spatial map | O | The details about a spatial map associated with this layer. For example, if the layer is for a particular floor of the building, the detailed spatial map of that particular floor can be associated with this layer. |
| > List of objects | O | List of object information. Each element includes the information below. |
| >> Object ID | M | The identifier of the corresponding object |
| >> Object type | M | The type of object |
| >> Object position | O | The 3D position of the object in the layer |
| >> Object specific information | O | Object specific attribute information |

NOTE 2:	For the current release, the object specific information is implementation specific and out of scope.

* * * Next Change * * * *

#### 9.3.6.2.1	Subscribe spatial map event

Figure 9.3.6.2.1-1 depicts the procedure for the authorized spatial map consumer (VAL server or SEAL SM client) to subscribe the spatial map event. The service is provided by SEAL SM server. Spatial map consumer can get notification information when the the SM server detects subscribed event.

Figure 9.3.6.2.1-1: Subscribe spatial map event procedure

1)	The VAL server (or SEAL SM client) sends a request message to the SEAL SM server to subscribe spatial map event. The request includes requestor ID, security credentials, a notification endpoint, a spatial map ID, event to subscribe which triggers the notification. The request message may also include a list of spatial map layers with its corresponding layer IDs to subscribe spatial map layering information related events. The requestor (VAL server or SEAL SM client) may subscribe to one or more of the following events:

a)	"changes of objects" events to indicate the SM server to notify the requestor when changes of the objects are detected. The notification triggering criteria for "changes of objects" event may include added or removed objects, position or direction changes of the spatial map objects.

b)	"layers modification" events to indicate the SM server to notify the requestor when any layer in the spatial map is added or updated or removed.

c)	"spatial map ready" events to indicate the SM server to notify the requestor when the SEAL SM server has sufficient information to map all the objects related to area of interest the spatial map and the spatial map is ready to provide service (e.g. employed for localization). This event is used when the requestor VAL server  may want to use the spatial map for localization service when spatial map is ready.

Editor’s Note: Objects are limited to UEs and objects other than UE is FFS.

2)	The SEAL SM server authorizes VAL server (or SEAL SM client), and validates the request. Then server add the requestor into the subscriber list of the event. SM server starts monitoring the events.

3)	The SEAL SM server sends response message with the result. For the successful subscribe request, a subscription ID is included in the message. Otherwise, the response message includes an indication of failure and may include a reason for failure.