| 3GPP TS 38.133 V17.12.0 (2023-12) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Requirements for support of radio resource management (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	119

1	Scope	121

2	References	121

3	Definitions, symbols and abbreviations	122

3.1	Definitions	122

3.2	Symbols	123

3.3	Abbreviations	124

3.4	Test tolerances	127

3.5	Frequency bands grouping	127

3.5.1	Introduction	127

3.5.2	NR operating bands in FR1	127

3.5.2A	NR operating bands for satellite access in FR1	128

3.5.3	NR operating bands in FR2	128

3.6	Applicability of requirements in this specification version	129

3.6.1	RRC connected state requirements in DRX	130

3.6.2	Number of serving carriers	130

3.6.2.1	Number of serving carriers for SA	130

3.6.2.2	Number of serving carriers for EN-DC	130

3.6.2.3	Number of serving carriers for NE-DC	130

3.6.2.4	Number of serving carriers for NR-DC	131

3.6.3	Applicability for intra-band FR2	131

3.6.4	Applicability for FR2 UE power classes	131

3.6.5	Applicability for SDL bands	131

3.6.6	Applicability of requirements for NGEN-DC operation	131

3.6.7	Applicability of QCL	131

3.6.9	Applicability of requirements for scheduling availability	132

3.6.10	Applicability of requirements for measurement restrictions	132

3.6.11	Applicability of requirements for Redcap UEs	132

3.6.11.1	RRC connected state requirements in DRX	132

3.6.11.2	Applicability for FR2 Redcap UE power classes	132

3.6.11.3	Applicability of QCL	132

3.6.12	Applicability of requirements for Satellite Access	133

3.6.13	Applicability of requirements for FR2	133

3.6.14	Applicability of requirements for FR2 Power Class 6	133

3.6.15	Applicability of requirements for per-FR gap	133

4	SA: RRC_IDLE state mobility	133

4.1	Cell Selection	133

4.2	Cell Re-selection	134

4.2.1	Introduction	134

4.2.2	Requirements	134

4.2.2.1	UE measurement capability	134

4.2.2.2	Measurement and evaluation of serving cell	134

4.2.2.3	Measurements of intra-frequency NR cells	137

4.2.2.4	Measurements of inter-frequency NR cells	140

4.2.2.5	Measurements of inter-RAT E-UTRAN cells	145

4.2.2.6	Maximum interruption in paging reception	147

4.2.2.7	General requirements	148

4.2.2.8	Minimum requirement at transitions	148

4.2.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	148

4.2.2.9.1	Introduction	148

4.2.2.9.2	Measurements for UE fulfilling low mobility criterion	149

4.2.2.9.3	Measurements for UE fulfilling not-at-cell edge criterion	151

4.2.2.9.4	Measurements for UE fulfilling low mobility and not-at-cell edge criteria	153

4.2.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	154

4.2.2.10.1	Introduction	154

4.2.2.10.2	Measurements for UE fulfilling low mobility criterion	154

4.2.2.10.3	Measurements for UE fulfilling not-at-cell edge criterion	157

4.2.2.10.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	160

4.2.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	160

4.2.2.11.1	Introduction	160

4.2.2.11.2	Measurements for UE fulfilling low mobility criterion	161

4.2.2.11.3	Measurements for UE fulfilling with not-at-cell edge criterion	162

4.2.2.11.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	164

4.2A	Cell Re-selection when subject to CCA	165

4.2A.1	Introduction	165

4.2A.2	Requirements	165

4.2A.2.1	UE measurement capability	165

4.2A.2.2	Measurement and evaluation when subject to CCA on the serving cell	166

4.2A.2.3	Measurements of intra-frequency NR cells when subject to CCA on the serving cell and target cell	166

4.2A.2.4	Measurements of inter-frequency NR cells when subject to CCA on the target cell	168

4.2A.2.5	Measurements of inter-RAT E-UTRAN cells when subject to CCA on the serving cell	170

4.2A.2.6	Maximum interruption in paging reception when subject to CCA on the target cell	170

4.2A.2.7	General requirements	170

4.2B	Cell Re-selection for RedCap	171

4.2B.1	Introduction	171

4.2B.2	Requirements	171

4.2B.2.1	UE measurement capability for RedCap	171

4.2B.2.1.1	UE measurement capability for 1 Rx RedCap	171

4.2B.2.1.2	UE measurement capability for 2 Rx RedCap	171

4.2B.2.2	Measurement and evaluation of serving cell for RedCap UE	171

4.2B.2.3	Measurements of intra-frequency NR cells for RedCap UE	173

4.2B.2.4	Measurements of inter-frequency NR cells for RedCap UE	176

4.2B.2.5	Measurements of inter-RAT E-UTRAN cells for RedCap UE	179

4.2B.2.6	Maximum interruption in paging reception for RedCap	181

4.2B.2.7	General requirements for RedCap	181

4.2B.2.8	Minimum requirement at transitions	181

4.2B.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion for RedCap	182

4.2B.2.9.1	Introduction	182

4.2B.2.9.2	Measurements for UE fulfilling stationary criterion	183

4.2B.2.9.3	Measurements for a UE fulfilling not-at-cell edge while stationary criterion	186

4.2B.2.9.3A	Measurements for a UE fulfilling stationary and not-at-cell-edge criteria	186

4.2B.2.9.4	Measurements for a UE fulfilling low mobility and stationary criteria	186

4.2B.2.9.5	Measurements for a UE fulfilling low mobility and not-at-cell-edge while stationary criteria	187

4.2B.2.9.6	Measurements for a UE fulfilling not-at-cell edge and not-at-cell edge while stationary criteria	187

4.2B.2.9.7	Measurements for a UE fulfilling low mobility and not-at-cell edge criteria and not-at-cell-edge while stationary criteria	187

4.2B.2.9.8	Measurements for a UE fulfilling low mobility, not-at-cell edge and stationary criterion	187

4.2B.2.9.9	Measurements for UE fulfilling low mobility criterion	188

4.2B.2.9.10	Measurements for UE fulfilling not-at-cell edge criterion	191

4.2B.2.9.11	Measurements for UE fulfilling low mobility and not-at-cell edge criteria	193

4.2B.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	193

4.2B.2.10.1	Introduction	193

4.2B.2.10.2	Measurements for UE fulfilling stationary criterion	194

4.2B.2.10.3	Measurements for a UE fulfilling not-at-cell edge while stationary criterion	196

4.2B.2.10.3A	Measurements for a UE fulfilling stationary and not-at-cell-edge-criterion	197

4.2B.2.10.4	Measurements for a UE fulfilling low mobility and stationary criteria	197

4.2B.2.10.5	Measurements for a UE fulfilling low mobility and not-at-cell-edge while stationary criteria	198

4.2B.2.10.6	Measurements for a UE fulfilling not-at-cell edge and not-at-cell edge while stationary criteria	198

4.2B.2.10.7	Measurements for a UE fulfilling low mobility and not-at-cell edge criteria and not-at-cell-edge while stationary criteria	198

4.2B.2.10.8	Measurements for a UE fulfilling low mobility, not-at-cell edge and stationary criteria	198

4.2B.2.10.9	Measurements for UE fulfilling low mobility criterion	199

4.2B.2.10.10	Measurements for UE fulfilling not-at-cell edge criterion	201

4.2B.2.10.11	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	204

4.2B.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	204

4.2B.2.11.1	Introduction	204

4.2B.2.11.2	Measurements for UE fulfilling stationary criterion	205

4.2B.2.11.3	Measurements for a UE not-at-cell edge while stationary criterion	206

4.2B.2.11.3A	Measurements for a UE fulfilling stationary and not-at-cell-edge criterion	206

4.2B.2.11.4	Measurements for a UE fulfilling low mobility and stationary criteria	207

4.2B.2.11.5	Measurements for a UE fulfilling low mobility and stationary not-at-cell-edge while stationary criteria	207

4.2B.2.11.6	Measurements for a UE fulfilling not-at-cell edge and not-at-cell edge while stationary criteria	207

4.2B.2.11.7	Measurements for a UE fulfilling low mobility and not-at-cell edge criteria and not-at-cell-edge while stationary criteria	208

4.2B.2.11.8	Measurements for a UE fulfilling low mobility, not-at-cell edge and stationary criteria	208

4.2B.2.11.9	Measurements for UE fulfilling low mobility criterion	208

4.2B.2.11.10	Measurements for UE fulfilling with not-at-cell edge criterion	210

4.2B.2.11.11	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	211

4.2C	Cell Re-selection for NR UE for Satellite Access	211

4.2C.1	Introduction	211

4.2C.2	Requirements	211

4.2C.2.1	UE measurement capability	211

4.2C.2.2	Measurement and evaluation of serving cell	212

4.2C.2.3	Measurements of intra-frequency NR cells	212

4.2C.2.4	Measurements of inter-frequency NR cells	215

4.2C.2.5	Maximum interruption in paging reception	218

4.2C.2.6	Minimum requirement at transitions	218

4.2C.2.7	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	218

4.2C.2.8	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	219

4.2C.2.9	General requirements	219

4.3	Minimization of Drive Tests (MDT)	219

4.3.1	Introduction	219

4.3.2	Measurement Requirements	219

4.3.3	Requirements for Relative Time Stamp Accuracy	220

4.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	220

4.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	220

4.3C	Minimization of Drive Tests (MDT) for Satellite Access	220

4.3C.1	Introduction	220

4.3C.2	Measurement Requirements	221

4.3C.3	Requirements for Relative Time Stamp Accuracy	221

4.3C.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	221

4.3C.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	222

4.4	Idle Mode CA/DC Measurements	222

4.4.1	Introduction	222

4.4.2	Measurement Requirements	222

4.4.2.1	Detected cell requirement during state transition and Idle mode	222

4.4.2.2	Measurements of inter-frequency CA/DC candidate cells	223

4.4.2.3	Measurements on serving cell	224

4.4.2.4	Measurements of E-UTRAN inter-RAT DC candidate cells	224

5	SA: RRC_INACTIVE state mobility	226

5.1	Cell Re-selection	226

5.1.1	Introduction	226

5.1.2	Requirements	226

5.1.2.1	UE measurement capability	226

5.1.2.2	Measurement and evaluation of serving cell	226

5.1.2.3	Measurements of intra-frequency NR cells	227

5.1.2.4	Measurements of inter-frequency NR cells	228

5.1.2.5	Measurements of inter-RAT E-UTRAN cells	229

5.1.2.6	Maximum interruption in paging reception	230

5.1.2.7	General requirements	230

5.1A	Cell Re-selection with CCA	230

5.1A.1	Introduction	230

5.1A.2	Requirements	230

5.1A.2.1	UE measurement capability	230

5.1A.2.2	Measurement and evaluation when CCA is used on the serving cell	230

5.1A.2.3	Measurements of intra-frequency NR cells when CCA is used on the serving cell and target cell	230

5.1A.2.4	Measurements of inter-frequency NR cells when CCA is used on the target cell	230

5.1A.2.5	Measurements of inter-RAT E-UTRAN cells when CCA is used on the serving cell	230

5.1A.2.6	Maximum interruption in paging reception when CCA is used on the target cell	230

5.1A.2.7	General requirements	231

5.1B	Cell Re-selection for RedCap	231

5.1B.1	Introduction	231

5.1B.2	Requirements	231

5.1B.2.1	UE measurement capability	231

5.1B.2.2	Measurement and evaluation of serving cell	231

5.1B.2.3	Measurements of intra-frequency NR cells	232

5.1B.2.4	Measurements of inter-frequency NR cells	233

5.1B.2.5	Measurements of inter-RAT E-UTRAN cells	234

5.1B.2.6	Maximum interruption in paging reception	234

5.1B.2.7	General requirements	234

5.1B.2.8	Minimum requirement at transitions	234

5.1B.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	235

5.1B.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	235

5.1B.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	235

5.1C	Cell Re-selection	235

5.1C.1	Introduction	235

5.1C.2	Requirements	235

5.1C.2.1	UE measurement capability	235

5.1C.2.2	Measurement and evaluation of serving cell	235

5.1C.2.3	Measurements of intra-frequency NR cells	235

5.1C.2.4	Measurements of inter-frequency NR cells	235

5.1C.2.5	Maximum interruption in paging reception	235

5.1C.2.6	General requirements	236

5.2	Void	236

5.2B	Configured Grant based Small Data Transmissions (CG-SDT) for RedCap	236

5.2B.1	Introduction	236

5.2B.2	Requirements on UE synchronization for small data transmissions for RedCap	236

5.2B.2.1	Void	236

5.2B.3	TA validation requirements for RedCap	236

5.2B.2.2	Void	237

5.2B.2.3	Void	237

5.2B.4	Scheduling restriction	237

5.2B.5	Applicability conditions for CG-SDT for RedCap	238

5.3	Minimization of Drive Tests (MDT)	238

5.3.1	Introduction	238

5.3.2	Measurement Requirements	238

5.3.3	Requirements for Relative Time Stamp Accuracy	238

5.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	238

5.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	238

5.3.6	Requirements for Relative Time Stamp Accuracy for RRC Resume Failure Log Reporting	239

5.3C	Minimization of Drive Tests (MDT) for Satellite Access	239

5.3C.1	Introduction	239

5.3C.2	Measurement Requirements	239

5.3C.3	Requirements for Relative Time Stamp Accuracy	239

5.3C.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	239

5.3C.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	239

5.3C.6	Requirements for Relative Time Stamp Accuracy for RRC Resume Failure Log Reporting	240

5.4	Idle Mode CA/DC Measurements	240

5.4.1	Introduction	240

5.4.2	Measurement Requirements	240

5.4.2.1	Detected cell requirement during state transition and Idle mode	240

5.4.2.2	Measurements of inter-frequency CA/DC candidate cells	240

5.4.2.3	Measurements on serving cell	240

5.4.2.4	Measurements on E-UTRAN inter-RAT DC candidate cells	240

5.5	Configured Grant based Small Data Transmissions (CG-SDT)	240

5.5.1	Introduction	240

5.5.2	Requirements on UE synchronization for small data transmissions	240

5.5.3	TA validation requirements	241

5.5.4	Scheduling restriction	242

5.5.4.1	Scheduling availability of UE performing measurements in TDD bands on FR1	242

5.5.4.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	242

5.5.4.3	Scheduling availability of UE performing measurements on FR2	242

5.5.5	Applicability conditions for SDT	243

5.6	NR measurements for positioning	243

5.6.1	Introduction	243

5.6.2	RSTD measurements	244

5.6.2.1	Introduction	244

5.6.2.2	Requirements Applicability	244

5.6.2.3	Measurement Capability	244

5.6.2.5	Measurements Period Requirements	244

5.6.3	PRS-RSRP measurements	247

5.6.3.1	Introduction	247

5.6.3.2	Requirements applicability	247

5.6.3.3	Measurement Capability	247

5.6.3.4	Measurement Reporting Requirements	247

5.6.3.5	Measurement Period Requirements	248

5.6.4	UE Rx-Tx time difference measurements	250

5.6.4.1	Introduction	250

5.6.4.2	Requirements Applicability	250

5.6.4.3	Measurement Capability	250

5.6.4.4	Measurement Reporting Requirements	250

5.6.4.5	Measurement Period Requirements	251

5.6.5	PRS-RSRPP measurements	254

5.6.5.1	Introduction	254

5.6.5.2	Requirements Applicability	254

5.6.5.3	Measurement Capability	254

5.6.5.4	Measurement Reporting Requirements	254

5.6.5.5	Measurement Period Requirements	254

5.7	Random access based Small Data Transmissions (RA-SDT)	255

5.7.1	Introduction	255

5.7.2	Requirements for small data transmissions based on 2-step RA	256

5.7.3	Requirements for small data transmissions based on 4-step RA	256

5.7.4	Applicability conditions for SDT	256

5.7B	Random access based Small Data Transmissions (RA-SDT) for RedCap	256

5.7B.1	Introduction	256

5.7B.2	Requirements for small data transmissions based on 2-step RA	256

5.7B.3	Requirements for small data transmissions based on 4-step RA	256

5.7B.4	Applicability conditions for RA-SDT for RedCap	256

6	RRC_CONNECTED state mobility	257

6.1	Handover	257

6.1.1	NR Handover	257

6.1.1.1	Introduction	257

6.1.1.2	NR FR1 - NR FR1 Handover	257

6.1.1.2.1	Handover delay	257

6.1.1.2.2	Interruption time	257

6.1.1.3	NR FR2- NR FR1 Handover	258

6.1.1.3.1	Handover delay	258

6.1.1.3.2	Interruption time	258

6.1.1.4	NR FR2- NR FR2 Handover	259

6.1.1.4.1	Handover delay	259

6.1.1.4.2	Interruption time	259

6.1.1.5	NR FR1- NR FR2 Handover	260

6.1.1.5.1	Handover delay	260

6.1.1.5.2	Interruption time	260

6.1.2	NR Handover to other RATs	261

6.1.2.1	NR – E-UTRAN Handover	261

6.1.2.1.1	Introduction	261

6.1.2.1.2	Handover delay	261

6.1.2.1.3	Interruption time	261

6.1.2.2	NR – UTRAN Handover	261

6.1.2.2.1	Introduction	261

6.1.2.2.2	Handover delay	262

6.1.2.2.3	Interruption time	262

6.1.3	NR DAPS Handover	262

6.1.3.1	Introduction	262

6.1.3.2	NR FR1 - NR FR1 DAPS Handover	263

6.1.3.2.1	DAPS handover delay	263

6.1.3.2.2	Interruption time	264

6.1.3.3	NR FR2- NR FR1 DAPS Handover	265

6.1.3.3.1	DAPS handover delay	266

6.1.3.3.2	Interruption time	266

6.1.3.4	NR FR1- NR FR2 DAPS Handover	267

6.1.3.4.1	DAPS handover delay	267

6.1.3.4.2	Interruption time	267

6.1.4	NR Conditional Handover	268

6.1.4.1	Introduction	268

6.1.4.2	NR FR1 – NR FR1 conditional handover	268

6.1.4.3	NR FR2 – NR FR1 conditional handover	269

6.1.4.4	NR FR2 – NR FR2 conditional handover	269

6.1.4.4.1	Handover delay	269

6.1.4.4.2	Measurement time	270

6.1.4.4.3	Preparation time	270

6.1.4.4.4	Interruption time	270

6.1.4.5	NR FR1 – NR FR2 conditional handover	271

6.1.5	NR Handover with PSCell	271

6.1.5.1	Introduction	271

6.1.5.2	Handover with PSCell from NR SA to EN-DC	271

6.1.5.2.1	Interruption time for inter-RAT HO from NR to E-UTRAN	271

6.1.5.2.2	PSCell addition in HO with PSCell for NR SA to EN-DC	272

6.1.5.3	HO with PSCell from NE-DC to NE-DC	272

6.1.5.3.1	Handover delay	272

6.1.5.3.2	HO with PSCell - PCell Interruption time	273

6.1.5.3.3	PSCell addition/change in NE-DC to NE-DC HO with PSCell	273

6.1.5.4	HO with PSCell from NR-DC to NR-DC	273

6.1.5.5	Handover with PSCell from NR SA to EN-DC with PSCell using CCA	274

6.1.5.5.1	Introduction	274

6.1.5.5.2	NR SA to EN-DC HO with PSCell- NR to E-UTRA HO Interruption time	275

6.1.5.5.3	NR SA to EN-DC HO with PSCell - NR PSCell Addition Delay requirements	275

6.1A	Void	276

6.1A.1	Void	276

6.1A.1.1	Void	276

6.1A.1.2	Void	276

6.1A.1.2.1	Void	276

6.1A.1.2.2	Void	276

6.1B	Handover to target cell using CCA	276

6.1B.1	NR Handover	276

6.1B.1.1	Introduction	276

6.1B.1.2	NR FR1 - NR FR1 Handover	277

6.1B.1.2.1	Handover delay	277

6.1B.1.2.2	Interruption time	277

6.1B.1.3	NR FR2-2 NR FR2-2 Handover	278

6.1B.1.3.1	Handover delay	278

6.1B.1.3.2	Interruption time	278

6.1B.1.4	NR FR1- NR FR2-2 Handover	279

6.1B.1.4.1	Handover delay	279

6.1B.1.4.2	Interruption time	279

6.1C	Handover for SAN	280

6.1C.1	NR SAN Handover	280

6.1C.1.1	Introduction	280

6.1C.1.2	NR SAN FR1 – NR SAN FR1 Handover	280

6.1C.1.2.1	Handover delay	280

6.1C.1.2.2	Interruption time	280

6.1C.2	NR SAN Conditional Handover	281

6.1C.2.1	Introduction	281

6.1C.2.2	NR SAN FR1 – NR SAN FR1 conditional handover	281

6.1C.2.2.1	Handover delay	281

6.1C.2.2.2	Measurement time	282

6.1C.2.2.3	Preparation time	283

6.1C.2.2.4	Interruption time	283

6.1D	Handover for RedCap	283

6.1D.1	NR Handover	283

6.1D.1.1	Introduction	283

6.1D.1.2	NR FR1 - NR FR1 Handover	284

6.1D.1.2.1	Handover delay	284

6.1D.1.2.2	Interruption time	284

6.1D.1.3	NR FR2- NR FR2 Handover	285

6.1D.1.3.1	Handover delay	286

6.1D.1.3.2	Interruption time	286

6.1D.2	NR Handover to other RATs	287

6.1D.2.1	NR – E-UTRAN Handover	287

6.2	RRC Connection Mobility Control	287

6.2.1	SA: RRC Re-establishment	287

6.2.1.1	Introduction	287

6.2.1.2	Requirements	287

6.2.1.2.1	UE Re-establishment delay requirement	288

6.2.1A	RRC Re-establishment with CCA	289

6.2.1A.1	Introduction	289

6.2.1A.2	Requirements	290

6.2.1A.2.1	UE Re-establishment with CCA delay requirement	290

6.2.1B	SA: RRC Re-establishment for RedCap	292

6.2.1B.1	Introduction	292

6.2.1B.2	Requirements	292

6.2.2	Random access	292

6.2.2.1	Introduction	292

6.2.2.2	Requirements for 4-step RA type	293

6.2.2.2.1	Contention based random access	293

6.2.2.2.1.1	Correct behaviour when transmitting Random Access Preamble	293

6.2.2.2.1.2	Correct behaviour when receiving Random Access Response	293

6.2.2.2.1.3	Correct behaviour when not receiving Random Access Response	293

6.2.2.2.1.4	Correct behaviour when receiving an UL grant for msg3 retransmission	293

6.2.2.2.1.5	SA: Correct behaviour when receiving a message over Temporary C-RNTI	293

6.2.2.2.1.6	Correct behaviour when contention Resolution timer expires	294

6.2.2.2.2	Non-Contention based random access	294

6.2.2.2.2.1	Correct behaviour when transmitting Random Access Preamble	294

6.2.2.2.2.2	Correct behaviour when receiving Random Access Response	294

6.2.2.2.2.3	Correct behaviour when not receiving Random Access Response	294

6.2.2.2.3	UE behaviour when configured with supplementary UL	295

6.2.2.3	Requirements for 2-step RA type	295

6.2.2.3.1	Contention based random access	295

6.2.2.3.1.1	Correct behaviour when transmitting MsgA	295

6.2.2.3.1.2	Correct behaviour when receiving MsgB	295

6.2.2.3.1.3	Correct behaviour when not receiving MsgB	296

6.2.2.3.2	Non-Contention based random access	296

6.2.2.3.2.1	Correct behaviour when transmitting MsgA	296

6.2.2.3.2.2	Correct behaviour when receiving MsgB	296

6.2.2.3.2.3	Correct behaviour when not receiving MsgB	296

6.2.2.3.3	UE behaviour when configured with supplementary UL	296

6.2.2A	Random access when CCA is used on target frequency	297

6.2.2A.1	Introduction	297

6.2.2A.2	Requirements for 4-step RA type	297

6.2.2A.2.1	Contention based random access	297

6.2.2A.2.1.1	Correct behaviour when transmitting Random Access Preamble	297

6.2.2A.2.1.2	Correct behaviour when receiving Random Access Response	298

6.2.2A.2.1.3	Correct behaviour when not receiving Random Access Response	298

6.2.2A.2.1.4	Correct behaviour when receiving an UL grant for msg3 retransmission	298

6.2.2A.2.1.6	Correct behaviour when contention Resolution timer expires	298

6.2.2A.2.2	Non-Contention based random access	298

6.2.2A.2.2.1	Correct behaviour when transmitting Random Access Preamble	298

6.2.2A.2.2.2	Correct behaviour when receiving Random Access Response	299

6.2.2A.2.2.3	Correct behaviour when not receiving Random Access Response	299

6.2.2A.3	Requirements for 2-step RA type	299

6.2.2A.3.1	Contention based random access	300

6.2.2A.3.1.1	Correct behaviour when transmitting MsgA	300

6.2.2A.3.1.2	Correct behaviour when receiving MsgB	300

6.2.2A.3.1.3	Correct behaviour when not receiving MsgB	300

6.2.2A.3.2	Non-Contention based random access	301

6.2.2A.3.2.1	Correct behaviour when transmitting MsgA	301

6.2.2A.3.2.2	Correct behaviour when receiving MsgB	301

6.2.2A.3.2.3	Correct behaviour when not receiving MsgB	301

6.2.2B	Random access for RedCap	301

6.2.2B.1	Introduction	301

6.2.2B.2	Requirements	302

6.2.3	SA: RRC Connection Release with Redirection	302

6.2.3.1	Introduction	302

6.2.3.2	Requirements	303

6.2.3.2.1	RRC connection release with redirection to NR	303

6.2.3.2.2	RRC connection release with redirection to E-UTRAN	303

6.2.3.2.3	RRC connection release with redirection to NR carrier subject to CCA	304

6.2.3A	SA: RRC Connection Release with Redirection for RedCap	305

6.2.3A.1	Introduction	305

6.2.3A.2	Requirements	305

6.2.3A.2.1	RRC connection release with redirection to NR	305

6.2.3A.2.2	RRC connection release with redirection to E-UTRAN	306

6.2C	RRC Connection Mobility Control for Satellite Access	306

6.2C.1	SA: RRC Re-establishment for Satellite Access	306

6.2C.1.1	Introduction	306

6.2C.1.2	Requirements	306

6.2C.1.2.1	UE Re-establishment delay requirement	306

6.2C.2	Random access for satellite access	307

6.2C.2.1	Introduction	307

6.2C.2.2	Requirements for 4-step RA type	308

6.2C.2.2.1	Contention based random access	308

6.2C.2.2.1.1	Correct behaviour when transmitting Random Access Preamble	308

6.2C.2.2.1.2	Correct behaviour when receiving Random Access Response	308

6.2C.2.2.1.3	Correct behaviour when not receiving Random Access Response	308

6.2C.2.2.1.4	Correct behaviour when receiving an UL grant for msg3 retransmission	308

6.2C.2.2.1.5	SA: Correct behaviour when receiving a message over Temporary C-RNTI	308

6.2C.2.2.1.6	Correct behaviour when contention Resolution timer expires	309

6.2C.2.2.2	Non-Contention based random access	309

6.2C.2.2.2.1	Correct behaviour when transmitting Random Access Preamble	309

6.2C.2.2.2.2	Correct behaviour when receiving Random Access Response	309

6.2C.2.2.2.3	Correct behaviour when not receiving Random Access Response	310

6.2C.2.3	Requirements for 2-step RA type	310

6.2C.2.3.1	Contention based random access	310

6.2C.2.3.1.1	Correct behaviour when transmitting MsgA	310

6.2C.2.3.1.2	Correct behaviour when receiving MsgB	310

6.2C.2.3.1.3	Correct behaviour when not receiving MsgB	311

6.2C.2.3.2	Non-Contention based random access	311

6.2C.2.3.2.1	Correct behaviour when transmitting MsgA	311

6.2C.2.3.2.2	Correct behaviour when receiving MsgB	311

6.2C.2.3.2.3	Correct behaviour when not receiving MsgB	311

6.2C.3	SA: RRC Connection Release with Redirection for Satellite Access	311

6.2C.3.1	Introduction	311

6.2C.3.2	Requirements	312

6.2C.3.2.1	RRC connection release with redirection to NR	312

7	Timing	313

7.1	UE transmit timing	313

7.1.1	Introduction	313

7.1.2	Requirements	313

7.1.2.1	Gradual timing adjustment	315

7.1.2.2	Void	315

7.1.2.3	One shot large UL timing adjustment for FR2 Power Class 6 UE	315

7.1A	UE transmit timing for RedCap	316

7.1A.1	Introduction	316

7.1A.2	Requirements	316

7.1A.2.1	Gradual timing adjustment	317

7.1C	UE transmit timing for Satellite Access	318

7.1C.1	Introduction	318

7.1C.2	Requirements	318

7.1C.2.1	Gradual timing adjustment	319

7.2	UE timer accuracy	319

7.2.1	Introduction	319

7.2.2	Requirements	319

7.2A	UE timer accuracy for RedCap	320

7.2A.1	Introduction	320

7.2A.2	Requirements	320

7.2C	UE timer accuracy for satellite access	320

7.2C.1	Introduction	320

7.2C.2	Requirements	320

7.3	Timing advance	321

7.3.1	Introduction	321

7.3.2	Requirements	321

7.3.2.1	Timing Advance adjustment delay	321

7.3.2.2	Timing Advance adjustment accuracy	321

7.3A	Timing Advance for RedCap	321

7.3A.1	Introduction	321

7.3A.2	Requirements	321

7.3A.2.1	Timing Advance adjustment delay	321

7.3A.2.2	Timing Advance adjustment accuracy	321

7.3C	Timing advance for satellite access	322

7.3C.1	Introduction	322

7.3C.2	Requirements	322

7.3C.2.1	Timing Advance adjustment delay	322

7.3C.2.2	Timing Advance adjustment accuracy	322

7.4	Cell phase synchronization accuracy	322

7.4.1	Definition	322

7.4.2	Minimum requirements	322

7.5	Maximum Transmission Timing Difference	323

7.5.1	Introduction	323

7.5.2	Minimum Requirements for inter-band EN-DC	323

7.5.2.1	Minimum Requirements for inter-band synchronous EN-DC	323

7.5.3	Minimum Requirements for intra-band EN-DC	324

7.5.4	Minimum Requirements for NR Carrier Aggregation	324

7.5.5	Minimum Requirements for inter-band NE-DC	325

7.5.5.1	Minimum Requirements for inter-band synchronous NE-DC	325

7.5.6	Minimum Requirements for inter-band NR DC	326

7.6	Maximum Receive Timing Difference	326

7.6.1	Introduction	326

7.6.2	Minimum Requirements for inter-band EN-DC	327

7.6.2.1	Minimum Requirements for inter-band synchronous EN-DC	327

7.6.3	Minimum Requirements for intra-band EN-DC	328

7.6.4	Minimum Requirements for NR Carrier Aggregation	328

7.6.5	Minimum Requirements for inter-band NE-DC	329

7.6.5.1	Minimum Requirements for inter-band synchronous NE-DC	329

7.6.6	Minimum Requirements for inter-band NR DC	330

7.7	deriveSSB-IndexFromCell tolerance	331

7.7.1	Minimum requirements	331

7.7A	deriveSSB-IndexFromCell tolerance for RedCap	331

7.7A.1	Minimum requirements	331

7.8	Void	331

7.9	deriveSSB-IndexFromCellInter-r17 tolerance	331

7.9.1	Minimum requirements	331

8	Signalling characteristics	332

8.1	Radio Link Monitoring	332

8.1.1	Introduction	332

8.1.1.1	Introduction of Requirement on Radio Link Monitoring for UE Configured with Relaxed Measurement Criteria	333

8.1.2	Requirements for SSB based radio link monitoring	333

8.1.2.1	Introduction	334

8.1.2.2	Minimum requirement	334

8.1.2.3	Measurement restrictions for SSB based RLM	338

8.1.2.4	Minimum requirement of SSB based radio link monitoring for UE fulfilling relaxed measurement criteria	338

8.1.3	Requirements for CSI-RS based radio link monitoring	339

8.1.3.1	Introduction	339

8.1.3.2	Minimum requirement	340

8.1.3.3	Measurement restrictions for CSI-RS based RLM	343

8.1.3.4	Minimum requirement of CSI-RS based radio link monitoring for UE fulfilling relaxed measurement criteria	344

8.1.4	Minimum requirement at transitions	345

8.1.5	Minimum requirement for UE turning off the transmitter	345

8.1.6	Minimum requirement for L1 indication	345

8.1.7	Scheduling availability of UE during radio link monitoring	346

8.1.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	346

8.1.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	346

8.1.7.3	Scheduling availability of UE performing radio link monitoring on FR2	346

8.1.7.4	Scheduling availability of UE performing radio link monitoring on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	347

8.1A	Radio Link Monitoring with CCA on Target Frequency	347

8.1A.1	Introduction	347

8.1A.2	Requirements for SSB Based Radio Link Monitoring	348

8.1A.2.1	Introduction	348

8.1A.2.2	Minimum Requirement	349

8.1A.3	Minimum requirement at transitions	352

8.1A.4	Minimum requirement for UE turning off the transmitter	353

8.1A.5	Minimum requirement for L1 indication	353

8.1A.6	Scheduling availability of UE during radio link monitoring	353

8.1A.6.3	Scheduling availability of UE performing radio link monitoring on FR2-2	353

8.1A.6.4	Scheduling availability of UE performing radio link monitoring on FR1 or FR2-2 in case of FR1-FR2-2 inter-band CA and NR-DC	354

8.1B	Radio Link Monitoring for RedCap	354

8.1B.1	Introduction	354

8.1B.2	Requirements for SSB based radio link monitoring	355

8.1B.2.1	Introduction	355

8.1B.2.2	Minimum requirement	356

8.1B.2.3	Measurement restrictions for SSB based RLM	358

8.1B.3	Requirements for CSI-RS based radio link monitoring	359

8.1B.3.1	Introduction	359

8.1B.3.2	Minimum requirement	359

8.1B.3.3	Measurement restrictions for CSI-RS based RLM	362

8.1B.4	Minimum requirement at transitions	363

8.1B.5	Minimum requirement for UE turning off the transmitter	363

8.1B.6	Minimum requirement for L1 indication	363

8.1B.7	Scheduling availability of UE during radio link monitoring	363

8.1B.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	364

8.1B.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	364

8.1B.7.3	Scheduling availability of UE performing radio link monitoring on FR2	364

8.1C	Radio Link Monitoring for Satellite Access	364

8.1C.1	Introduction	364

8.1C.2	Requirements for SSB based radio link monitoring	365

8.1C.2.1	Introduction	365

8.1C.2.2	Minimum requirement	366

8.1C.2.3	Measurement restrictions for SSB based RLM	367

8.1C.3	Requirements for CSI-RS based radio link monitoring	367

8.1C.3.1	Introduction	367

8.1C.3.2	Minimum requirement	368

8.1C.3.3	Measurement restrictions for CSI-RS based RLM	369

8.1C.4	Minimum requirement at transitions	370

8.1C.5	Minimum requirement for UE turning off the transmitter	370

8.1C.6	Minimum requirement for L1 indication	370

8.1C.7	Scheduling availability of UE during radio link monitoring	370

8.1C.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	371

8.1C.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	371

8.2	Interruption	371

8.2.1	EN-DC Interruption	371

8.2.1.1	Introduction	371

8.2.1.2	Requirements	372

8.2.1.2.1	Interruptions at transitions between active and non-active during DRX	372

8.2.1.2.2	Interruptions at transitions from non-DRX to DRX	372

8.2.1.2.3	Interruptions at SCell addition/release	372

8.2.1.2.4	Interruptions at SCell activation/deactivation	373

8.2.1.2.5	Interruptions during measurements on SCC	375

8.2.1.2.5.1	Interruptions during measurements on deactivated NR SCC	375

8.2.1.2.5.2	Interruptions during measurements on deactivated E-UTRAN SCC	375

8.2.1.2.5.3	Interruptions during CQI measurements on dormant E-UTRAN SCell	375

8.2.1.2.5.4	Interruptions during RRM measurements on dormant E-UTRAN SCC	376

8.2.1.2.6	Interruptions at UL carrier RRC reconfiguration	376

8.2.1.2.7	Interruptions due to Active BWP switching Requirement	377

8.2.1.2.8	Interruptions at direct SCell activation and hibernation	378

8.2.1.2.8.1	Interruptions during direct SCell activation and hibernation of E-UTRA SCell	378

8.2.1.2.8.2	Interruptions during direct SCell activation	378

8.2.1.2.9	Interruptions at SCell hibernation	378

8.2.1.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	379

8.2.1.2.11	Interruptions due to UE-specific CBW change	379

8.2.1.2.12	Interruptions at NR SRS carrier based switching	379

8.2.1.2.13	Interruptions at E-UTRA SRS carrier based switching	381

8.2.1.2.14	DL Interruptions at switching between two uplink carriers	382

8.2.1.2.15	Interruptions due to SCell dormancy	382

8.2.1.2.15.1	Interruptions due to SCell dormancy switch	382

8.2.1.2.15.2	Interruptions due to CQI measurements during SCell dormancy	382

8.2.1.2.15.3	Interruptions due to RRM measurements during SCell dormancy	383

8.2.1.2.16	Interruptions when identifying CGI of an NR cell with autonomous gaps	383

8.2.1.2.17	Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	383

8.2.1.2.18	Interruptions at NR SRS antenna port switching	384

8.2.1.2.19	Interruptions at fast SCell activation	385

8.2.1.2.20	Interruptions due to PUCCH SCell activation/deactivation	386

8.2.2	SA: Interruptions with Standalone NR Carrier Aggregation	386

8.2.2.1	Introduction	386

8.2.2.2	Requirements	387

8.2.2.2.1	Interruptions at SCell addition/release	387

8.2.2.2.2	Interruptions at SCell activation/deactivation	388

8.2.2.2.3	Interruptions during measurements on deactivated SCC	389

8.2.2.2.4	Interruptions at UL carrier RRC reconfiguration	390

8.2.2.2.5	Interruptions due to Active BWP switching Requirement	390

8.2.2.2.6	Interruptions at inter-frequency SFTD measurement	391

8.2.2.2.7	Interruptions at SCell activation/deactivation with multiple downlink SCells	393

8.2.2.2.8	Interruptions due to UE-specific CBW change	393

8.2.2.2.9	Interruptions at NR SRS carrier based switching	393

8.2.2.2.10	DL Interruptions at UE switching between two uplink carriers	395

8.2.2.2.10A	DL Interruptions at UE switching between two uplink carriers with two transmit antenna connectors	395

8.2.2.2.10B	DL Interruptions at UE switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors	396

8.2.2.2.10C	DL Interruptions at UE switching between two uplink bands with two transmit antenna connectors	396

8.2.2.2.11	Interruptions at direct SCell activation	396

8.2.2.2.12	Interruptions due to SCell dormancy	397

8.2.2.2.12.1	Interruptions due to SCell dormancy switch	397

8.2.2.2.12.2	Interruptions due to CQI measurements during SCell dormancy	397

8.2.2.2.12.3	Interruptions due to RRM measurements during SCell dormancy	397

8.2.2.2.13	Interruptions at transitions between active and non-active during DRX	397

8.2.2.2.14	Interruptions when identifying CGI of an NR cell with autonomous gaps	397

8.2.2.2.15	Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	398

8.2.2.2.16	Interruptions at NR SRS antenna port switching	398

8.2.2.2.17	Interruptions at fast SCell activation	399

8.2.2.2.18	Interruptions due to PUCCH SCell activation/deactivation	400

8.2.3	NE-DC Interruptions	400

8.2.3.1	Introduction	400

8.2.3.2	Requirements	401

8.2.3.2.1	Interruptions at transitions between active and non-active during DRX	401

8.2.3.2.2	Interruptions at transitions from non-DRX to DRX	401

8.2.3.2.3	Interruptions at PSCell/SCell addition/release	401

8.2.3.2.4	Interruptions at SCell activation/deactivation	403

8.2.3.2.5	Interruptions during measurements on SCC	404

8.2.3.2.5.1	Interruptions during measurements on deactivated NR SCC	404

8.2.3.2.5.2	Interruptions during measurements on deactivated E-UTRAN SCC	404

8.2.3.2.5.3	Interruptions during CQI measurements on dormant E-UTRAN SCC	405

8.2.3.2.5.4	Interruptions during RRM measurements on dormant E-UTRAN SCC	405

8.2.3.2.6	Interruptions at UL carrier RRC reconfiguration	405

8.2.3.2.7	Interruptions due to Active BWP switching Requirement	406

8.2.3.2.8	Interruptions at direct SCell activation and hibernation	406

8.2.3.2.9	Interruptions at SCell hibernation	406

8.2.3.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	407

8.2.3.2.11	 Interruptions at NR SRS carrier based switching	407

8.2.3.2.12	 Interruptions at E-UTRA SRS carrier based switching	408

8.2.3.2.13	Interruptions due to SCell dormancy	409

8.2.3.2.14	Interruptions when identifying CGI of an NR cell with autonomous gaps	410

8.2.3.2.15	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	410

8.2.3.2.17	Interruptions at fast SCell activation	412

8.2.3.2.18	Interruptions due to UE-specific CBW change	412

8.2.3.2.19	Interruptions due to PUCCH SCell activation/deactivation	413

8.2.4	NR-DC: Interruptions	413

8.2.4.1	Introduction	413

8.2.4.2	Requirements	414

8.2.4.2.1	Interruptions at PSCell/SCell addition/release	414

8.2.4.2.2	Interruptions at SCell activation/deactivation	415

8.2.4.2.3	Interruptions during measurements on SCC	416

8.2.4.2.4	Interruptions at UL carrier RRC reconfiguration	416

8.2.4.2.5	Interruptions due to Active BWP switching Requirement	417

8.2.4.2.6	Interruptions at transitions between active and non-active during DRX	417

8.2.4.2.7	Interruptions at transitions from non-DRX to DRX	417

8.2.4.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	418

8.2.4.2.9	Interruptions at NR SRS carrier based switching	418

8.2.4.2.10	Interruptions at direct SCell activation	419

8.2.4.2.11	Interruptions when identifying CGI of an NR cell with autonomous gaps	420

8.2.4.2.12	Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	420

8.2.4.2.13	 Interruptions due to SCell dormancy	421

8.2.4.2.14	Interruptions at NR SRS antenna port switching	421

8.2.4.2.15	Interruptions at fast SCell activation	422

8.2.4.2.16	Interruptions at SCG activation/deactivation	423

8.2.4.2.17	Interruptions due to RRM measurements on deactivated SCG	423

8.2.4.2.18 Interruptions during RLM/BFD measurements on deactivated PSCell	424

8.2.4.2.19	Interruptions due to UE-specific CBW change	424

8.2.4.2A	Void	424

8.2.4.2A.1	Void	424

8.2.4.2A.2	Void	424

8.2.4.2A.3	Void	424

8.3	SCell Activation and Deactivation Delay	424

8.3.1	Introduction	424

8.3.2	SCell Activation Delay Requirement for Deactivated SCell	424

8.3.3	SCell Deactivation Delay Requirement for Activated SCell	429

8.3.4	Direct SCell Activation at SCell addition	429

8.3.5	Direct SCell Activation at Handover	431

8.3.7	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	432

8.3.8	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	436

8.3.9	Direct SCell Activation of Multiple Downlink SCells at SCell addition	437

8.3.10	Direct SCell Activation of Multiple Downlink SCells at Handover	438

8.3.12	SCell Activation Delay Requirement for Deactivated PUCCH SCell	439

8.3.13	SCell activation delay Requirement for Deactivated PUCCH SCell with Multiple SCells	442

8.3.13.1	Introduction	442

8.3.14	SCell Deactivation Delay Requirement for Activated PUCCH SCell	444

8.3.15	SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple Downlink SCells	444

8.3.16	Fast SCell Activation Delay Requirement for Deactivated SCell	445

8.3A	SCell Activation and Deactivation Delay in Carriers with CCA	447

8.3A.1	Introduction	447

8.3A.2	SCell Activation Delay Requirement for Deactivated SCell	448

8.3A.3	SCell Deactivation Delay Requirement for Activated SCell	452

8.4	UE UL carrier RRC reconfiguration delay	452

8.4.1	Introduction	452

8.4.2	UE UL carrier configuration delay requirement	453

8.4.3	UE UL carrier deconfiguration delay requirement	453

8.5	Link Recovery Procedures	453

8.5.1	Introduction	453

8.5.1.1	Introduction of Requirement on Link Recovery Procedures for UE configured with relaxed measurement criteria	454

8.5.2	Requirements for SSB based beam failure detection	455

8.5.2.1	Introduction	455

8.5.2.2	Minimum requirement	455

8.5.2.3	Measurement restriction for SSB based beam failure detection	458

8.5.2.4	Minimum requirement of SSB based beam failure detection for UE fulfilling relaxed measurement criteria	459

8.5.3	Requirements for CSI-RS based beam failure detection	460

8.5.3.1	Introduction	460

8.5.3.2	Minimum requirement	460

8.5.3.3	Measurement restrictions for CSI-RS beam failure detection	464

8.5.3.4	Minimum requirement of CSI-RS based beam failure detection for UE fulfilling relaxed measurement criteria	465

8.5.4	Minimum requirement for L1 indication	465

8.5.5	Requirements for SSB based candidate beam detection	466

8.5.5.1	Introduction	466

8.5.5.2	Minimum requirement	466

8.5.5.3	Measurement restriction for SSB based candidate beam detection	469

8.5.6	Requirements for CSI-RS based candidate beam detection	469

8.5.6.1	Introduction	469

8.5.6.2	Minimum requirement	470

8.5.6.3	Measurement restriction for CSI-RS based candidate beam detection	473

8.5.7	Scheduling availability of UE during beam failure detection	473

8.5.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	473

8.5.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	474

8.5.7.3	Scheduling availability of UE performing beam failure detection on FR2	474

8.5.7.4	Scheduling availability of UE performing beam failure detection on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR DC	475

8.5.8	Scheduling availability of UE during candidate beam detection	475

8.5.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	475

8.5.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	475

8.5.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	475

8.5.8.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	476

8.5.9	Requirements for Beam Failure Recovery in SCell	476

8.5.9.1	Introduction	476

8.5.9.2	Requirement	476

8.5.10	Minimum requirement at transitions for beam failure detection	476

8.5A	Link Recovery Procedures when CCA is used on target frequency	477

8.5A.1	Introduction	477

8.5A.2	Requirements for SSB based beam failure detection	478

8.5A.2.1	Introduction	478

8.5A.2.2	Minimum requirement	478

8.5A.2.3	Measurement restriction for SSB based beam failure detection	480

8.5A.3	Void	480

8.5A.4	Minimum requirement for L1 indication	480

8.5A.5	Requirements for SSB based candidate beam detection	481

8.5A.5.1	Introduction	481

8.5A.5.2	Minimum requirement	481

8.5A.5.3	Measurement restriction for SSB based candidate beam detection	483

8.5A.6	Void	484

8.5A.7	Scheduling availability of UE during beam failure detection	484

8.5A.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH	484

8.5A.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH	484

8.5A.7.3	Scheduling availability of UE performing beam failure detection on FR2-2	484

8.5A.7.4	Scheduling availability of UE performing beam failure detection on FR1 or FR2-2 in case of FR1-FR2-2 inter-band CA and NR DC	484

8.5A.8	Scheduling availability of UE during candidate beam detection	484

8.5A.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2-2	484

8.5A.8.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2-2 in case of FR1-FR2-2 inter-band CA and NR-DC	485

8.5B	Link Recovery Procedures for Redcap	485

8.5B.1	Introduction	485

8.5B.2	Requirements for SSB based beam failure detection for Redcap	485

8.5B.2.1	Introduction	485

8.5B.2.2	Minimum requirement	486

8.5B.2.3	Measurement restriction for SSB based beam failure detection	487

8.5B.3	Requirements for CSI-RS based beam failure detection for Redcap	488

8.5B.3.1	Introduction	488

8.5B.3.2	Minimum requirement	488

8.5B.3.3	Measurement restrictions for CSI-RS beam failure detection	490

8.5B.4	Minimum requirement for L1 indication for Redcap	491

8.5B.5	Requirements for SSB based candidate beam detection for Redcap	492

8.5B.5.1	Introduction	492

8.5B.5.2	Minimum requirement	492

8.5B.5.3	Measurement restriction for SSB based candidate beam detection	494

8.5B.6	Requirements for CSI-RS based candidate beam detection for Redcap	494

8.5B.6.1	Introduction	494

8.5B.6.2	Minimum requirement	494

8.5B.6.3	Measurement restriction for CSI-RS based candidate beam detection	496

8.5B.7	Scheduling availability of UE during beam failure detection for Redcap	497

8.5B.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	497

8.5B.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	497

8.5B.7.3	Scheduling availability of UE performing beam failure detection on FR2	497

8.5B.8	Scheduling availability of UE during candidate beam detection for Redcap	498

8.5B.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	498

8.5B.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	498

8.5B.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	498

8.5B.9	Minimum requirement at transitions for beam failure detection for Redcap	498

8.5C	Link Recovery Procedures for Satellite Access	499

8.5C.1	Introduction	499

8.5C.2	Requirements for SSB based beam failure detection	499

8.5C.2.1	Introduction	499

8.5C.2.2	Minimum requirement	500

8.5C.2.3	Measurement restriction for SSB based beam failure detection	500

8.5C.3	Requirements for CSI-RS based beam failure detection	501

8.5C.3.1	Introduction	501

8.5C.3.2	Minimum requirement	501

8.5C.3.3	Measurement restrictions for CSI-RS beam failure detection	502

8.5C.4	Minimum requirement for L1 indication	503

8.5C.5	Requirements for SSB based candidate beam detection	503

8.5C.5.1	Introduction	503

8.5C.5.2	Minimum requirement	503

8.5C.5.3	Measurement restriction for SSB based candidate beam detection	504

8.5C.6	Requirements for CSI-RS based candidate beam detection	505

8.5C.6.1	Introduction	505

8.5C.6.2	Minimum requirement	505

8.5C.6.3	Measurement restriction for CSI-RS based candidate beam detection	506

8.5C.7	Scheduling availability of UE during beam failure detection	506

8.5C.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	506

8.5C.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	506

8.5C.8	Scheduling availability of UE during candidate beam detection	507

8.5C.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	507

8.5C.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	507

8.5C.9	Minimum requirement at transitions for beam failure detection	507

8.6	Active BWP switch delay	507

8.6.1	Introduction	507

8.6.2	DCI and timer based BWP switch delay on a single CC	508

8.6.2A	DCI based BWP switch delay on multiple CCs	509

8.6.2A.1	Simultaneous DCI based BWP switch delay on multiple CCs	509

8.6.2A.2	Non-simultaneous DCI based BWP switch delay on multiple CCs	511

8.6.2B	Timer based BWP switch delay on multiple CCs	511

8.6.2B.1	Simultaneous timer based BWP switch delay on multiple CCs	511

8.6.2B.2	Non-simultaneous timer based BWP switch delay on multiple CCs	511

8.6.3	RRC based BWP switch delay on a single CC	512

8.6.3A	RRC based BWP switch delay on multiple CCs	513

8.6.3A.1	Simultaneous RRC based BWP switch delay on multiple CCs	513

8.6.3A.2	Non-simultaneous RRC based BWP switch delay on multiple CCs	513

8.6.4	BWP switch delay on Consistent UL CCA recovery	514

8.6A	Active BWP switch delay for RedCap	514

8.6A.1	Introduction	514

8.6A.2	DCI and timer based BWP switch delay on a single CC	514

8.6A.3	RRC based BWP switch delay on a single CC	516

8.6C	Active BWP switch delay for satellite access	516

8.6C.1	Introduction	516

8.6C.2	DCI and timer based BWP switch delay on a single CC	516

8.6C.3	RRC based BWP switch delay on a single CC	518

8.7	Void	518

8.8	NE-DC: E-UTRAN PSCell Addition and Release Delay	518

8.8.1	Introduction	518

8.8.2	E-UTRAN PSCell Addition Delay Requirement	518

8.8.3	E-UTRAN PSCell Release Delay Requirement	519

8.9	NR-DC: PSCell Addition and Release Delay	519

8.9.1	Introduction	519

8.9.2	PSCell Addition Delay Requirement	519

8.9.3	PSCell Release Delay Requirement	520

8.9A	Conditional PSCell Addition Delay	520

8.9A.1	Introduction	520

8.9A.2	Conditional PSCell Addition Delay Requirement	520

8.9A.2.1	Measurement time	521

8.9B	NR-DC: PSCell Addition and Release Delay in Carriers with CCA	521

8.9B.1	Introduction	521

8.9B.2	PSCell Addition Delay Requirement	522

8.9B.3	PSCell Release Delay Requirement	522

8.10	Active TCI state switching delay	523

8.10.3A	MAC-CE based TCI state switch delay in HST FR2 scenarios	524

8.10.4	DCI based TCI state switch delay	524

8.10.5	RRC based TCI state switch delay	525

8.10.6	Active TCI state list update delay	525

8.10A	Active TCI state switching delay with CCA	525

8.10A.1	Introduction	525

8.10A.2	Known conditions for TCI state	526

8.10A.3	MAC-CE based TCI state switch delay	526

8.10A.4	DCI based TCI state switch delay	527

8.10A.5	RRC based TCI state switch delay	527

8.10A.6	Active TCI state list update delay	528

8.10B	Active TCI state switching delay for RedCap	528

8.10B.1	Introduction	528

8.10B.2	Known conditions for TCI state	528

8.10B.3	MAC-CE based TCI state switch delay	529

8.10B.4	DCI based TCI state switch delay	529

8.10B.5	RRC based TCI state switch delay	530

8.10B.6	Active TCI state list update delay	530

8.10C	Active TCI state switching delay for satellite access	531

8.10C.1	Introduction	531

8.10C.2	MAC-CE based TCI state switch delay	531

8.10C.4	DCI based TCI state switch delay	531

8.10C.5	RRC based TCI state switch delay	531

8.10C.6	Active TCI state list update delay	531

8.11	PSCell Change	532

8.11A	PSCell Change in Carriers with CCA	532

8.11B	Conditional PSCell Change	532

8.11B.1	Introduction	532

8.11B.2	Conditoinal PSCell Change delay	533

8.11B.2.1	Measurement time	533

8.11D	Conditional PSCell Change in Carriers with CCA	534

8.11D.1	Introduction	534

8.11D.2	Conditional PSCell Change delay	534

8.11D.2.1	Measurement time	535

8.12	Uplink spatial relation switch delay	535

8.12.1	Introduction	535

8.12.2	Known conditions for spatial relation when associated with DL-RS	535

8.12.3	MAC-CE based spatial relation switch delay	536

8.12.4	DCI based spatial relation switch delay	536

8.12.5	RRC based spatial relation switch delay	537

8.12A	Uplink spatial relation switch delay for RedCap	537

8.12A.1	Introduction	537

8.12A.2	Known conditions for spatial relation when associated with DL-RS	537

8.12A.3	MAC-CE based spatial relation switch delay	538

8.12A.4	DCI based spatial relation switch delay	538

8.12A.5	RRC based spatial relation switch delay	539

8.12C	Uplink spatial relation switch delay for satellite access	539

8.12C.1	Void	540

8.12C.2	Void	540

8.12C.3	Void	540

8.12C.4	Void	540

8.12C.5	Void	540

8.13	UE-specific CBW change	540

8.13.1	Introduction	540

8.13.2	UE-specific CBW change delay	540

8.13A	UE-specific CBW change for RedCap	540

8.13A.1	Introduction	540

8.13A.2	UE-specific CBW change delay	540

8.13C	UE-specific CBW change for satellite access	541

8.13C.1	Introduction	541

8.13C.2	UE-specific CBW change delay	541

8.14	Pathloss reference signal switching delay	541

8.14.1	Introduction	541

8.14.2	Known conditions for pathloss reference signal	542

8.14.3	MAC-CE based pathloss reference signal switch delay	542

8.14C	Pathloss reference signal switching delay for satellite access	543

8.14C.1	Introduction	543

8.14C.2	Known conditions for pathloss reference signal	543

8.14C.3	MAC-CE based pathloss reference signal switch delay	543

8.15	Active downlink TCI state switching delay for unified TCI	544

8.15.1	Introduction	544

8.15.4	DCI based downlink TCI state switch delay	546

8.15.5	Active Downlink TCI state list update delay	546

8.16	Active uplink TCI state switching delay for unified TCI	547

8.16.1	Introduction	547

8.16.4	DCI based uplink TCI state switch delay	549

8.16.5	Active Uplink TCI state list update delay	550

8.17	SCG Activation and Deactivation Delay	551

8.17.1	Introduction	551

8.17.2	SCG Activation Delay Requirement	551

8.17.3	SCG Deactivation Delay Requirement	552

8.18	TRP specific Link Recovery Procedures	553

8.18.1	Introduction	553

8.18.2	Requirements for TRP specific SSB based beam failure detection	553

8.18.2.1	Introduction	553

8.18.2.2	Minimum requirement	554

8.18.2.3	Measurement restriction for SSB based beam failure detection	556

8.18.3	Requirements for CSI-RS based beam failure detection	556

8.18.3.1	Introduction	556

8.18.3.2	Minimum requirement	557

8.18.3.3	Measurement restrictions for CSI-RS beam failure detection	559

8.18.4	Minimum requirement for L1 indication	560

8.18.5	Requirements for SSB based candidate beam detection	560

8.18.5.1	Introduction	560

8.18.5.2	Minimum requirement	560

8.18.5.3	Measurement restriction for SSB based candidate beam detection	562

8.18.6	Requirements for CSI-RS based candidate beam detection	563

8.18.6.1	Introduction	563

8.18.6.2	Minimum requirement	563

8.18.6.3	Measurement restriction for CSI-RS based candidate beam detection	565

8.18.7	Requirements for TRP specific Beam Failure Recovery	566

8.18.7.1	Introduction	566

8.18.7.2	Requirement	566

8.18.8	Scheduling availability of UE during TRP specific beam failure detection	566

8.18.8.1	Scheduling availability of UE performing TRP specific beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	566

8.18.8.2	Scheduling availability of UE performing TRP specific beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	567

8.18.8.3	Scheduling availability of UE performing TRP specific beam failure detection on FR2	567

8.18.8.4	Scheduling availability of UE performing TRP specific beam failure detection on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR DC	568

8.18.9	Scheduling availability of UE during TRP specific candidate beam detection	568

8.18.9.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	568

8.18.9.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	568

8.18.9.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	568

8.18.9.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	569

8.19	Pre-configured measurement gap activation/deactivation delay	569

8.19.1	Introduction	569

8.19.2	Pre-configured measurement gap activation/deactivation upon DCI/timer-based BWP switch	569

8.19.2.1	Activation/deactivation upon DCI/timer-based BWP switch delay on a single CC	569

8.19.3	Pre-configured measurement gap activation/deactivation upon SCell activation/deactivation	569

8.19.4	Pre-configured measurement gap activation/deactivation upon RRC reconfiguration	570

9	Measurement Procedure	573

9.1	General measurement requirement	573

9.1.1	Introduction	573

9.1.2	Measurement gap	573

9.1.2.1	EN-DC: Measurement Gap Sharing	585

9.1.2.1a	SA: Measurement Gap Sharing	586

9.1.2.1b	NE-DC: Measurement Gap Sharing	587

9.1.2.1c	NR-DC: Measurement Gap Sharing	588

9.1.3	UE Measurement capability	589

9.1.3.1	EN-DC: Monitoring of multiple layers using gaps	589

9.1.3.1a	SA: Monitoring of multiple layers using gaps	589

9.1.3.1b	NE-DC: Monitoring of multiple layers using gaps	590

9.1.3.1c	NR-DC: Monitoring of multiple layers using gaps	590

9.1.3.2	EN-DC: Maximum allowed layers for multiple monitoring	591

9.1.3.2a	SA: Maximum allowed layers for multiple monitoring	592

9.1.3.2b	NE-DC: Maximum allowed layers for multiple monitoring	592

9.1.3.2c	NR-DC: Maximum allowed layers for multiple monitoring	593

9.1A.3.2	Void	594

9.1.3A	UE Measurement capability under operation mode with CCA	594

9.1.3A.1	EN-DC: Monitoring of multiple layers using gaps under CCA	594

9.1.3A.1a	SA: Monitoring of multiple layers using gaps under CCA	594

9.1.3A.2	EN-DC: Maximum allowed layers for multiple monitoring under CCA	594

9.1.3A.2a	SA: Maximum allowed layers for multiple monitoring under CCA	595

9.1.3C	UE Measurement capability under operation mode with satellite access	595

9.1.3C.1a	SA: Monitoring of multiple layers using gaps under satellite access	595

9.1.3C.2a	SA: Maximum allowed layers for multiple monitoring for SAN	596

9.1.4	Capabilities for Support of Event Triggering and Reporting Criteria	596

9.1.4.1	Introduction	596

9.1.4.2	Requirements	596

9.1.5	Carrier-specific scaling factor	600

9.1.5.1	Monitoring of multiple layers outside gaps	601

9.1.5.1.1	EN-DC mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	603

9.1.5.1.2	SA mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	604

9.1.5.1.3	NR-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed outside gaps	605

9.1.5.1.4	NE-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based measurements performed outside gaps	605

9.1.5.2	Monitoring of multiple layers within gaps	606

9.1.5.2.1	EN-DC mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	608

9.1.5.2.2	SA mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	610

9.1.5.2.3	NE-DC: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed within gaps	612

9.1.5.2.4	NR-DC: carrier-specific scaling factor for SSB-based and CSI-RS-based L3 measurements performed within gaps	614

9.1.5.2.5	SA mode: carrier-specific scaling factor for PRS-based measurements performed within gaps	615

9.1.5.2.6	NE-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	616

9.1.5.2.7	NR-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	616

9.1.5.3	Monitoring of multiple layers within NCSG	616

9.1.5.3.1	SA mode: carrier-specific scaling factor for measurements performed within NCSG	617

9.1.6	Minimum requirement at transitions	617

9.1.7	Pre-configured measurement gap	618

9.1.7.1	Introduction	618

9.1.7.2	Requirements applicability	618

9.1.7.3	Requirements	618

9.1.7.3.1	Requirements for autonomous activation/deactivation mechanism	619

9.1.7.3.2	Requirements for network-controlled activation/deactivation mechanism	619

9.1.7.3.3	Requirements for reception/transmission during activation/deactivation	620

9.1.8	Concurrent measurement gaps	620

9.1.8.1	Introduction	620

9.1.8.2	Requirements	620

9.1.8.3	Collision between concurrent measurement gaps	622

9.1.8.4	Measurement gap related requirements of concurrent measurement gaps	622

9.1.9	Network controlled small gap	622

9.1.9.1	Introduction	622

9.1.9.2	Requirements applicability	623

9.1.10	MUSIM gaps	625

9.1.11	UL gap for Tx power management	626

9.1A	General measurement requirement for RedCap	627

9.1A.1	Introduction	627

9.1A.2	Measurement gap	627

9.1A.2.1	SA: Measurement Gap Sharing	631

9.1A.3	UE Measurement capability	632

9.1A.3.1	SA: Monitoring of multiple layers using gaps	632

9.1A.3.2	SA: Maximum allowed layers for multiple monitoring	632

9.1A.4	Capabilities for Support of Event Triggering and Reporting Criteria	632

9.1A.4.1	Introduction	632

9.1A.4.2	Requirements	633

9.1A.5	Carrier-specific scaling factor	633

9.1A.5.1	Monitoring of multiple layers outside gaps	633

9.1A.5.1.1	SA mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps	634

9.1A.5.2	Monitoring of multiple layers within gaps	634

9.1A.5.2.1	SA mode: carrier-specific scaling factor for SSB measurements performed within gaps	634

9.1A.6	Minimum requirement at transitions	635

9.1C	General measurement requirement for SAN	636

9.1C.1	Introduction	636

9.1C.2	Measurement gap	636

9.1C.8	Concurrent measurement gaps for SAN	638

9.1C.8.1	Introduction	638

9.1C.8.2	Requirements	638

9.1C.8.3	Collision between concurrent measurement gaps	638

9.1C.8.4	Measurement gap related requirements of concurrent measurement gaps	639

9.1C.9	Collision between SMTC and measurement gap for SAN	639

9.1C.9.1	Introduction	639

9.1C.9.2	Collision between SMTCs and measurement gap	639

9.1C.9.3	Collision between multiple SMTCs on a SAN carrier	640

9.2	NR intra-frequency measurements	640

9.2.1	Introduction	640

9.2.2	Requirements applicability	642

9.2.3	Number of cells and number of SSB	642

9.2.3.1	Requirements for FR1	642

9.2.3.2	Requirements for FR2	642

9.2.4	Measurement Reporting Requirements	643

9.2.4.1	Periodic Reporting	643

9.2.4.2	Event-triggered Periodic Reporting	643

9.2.4.3	Event Triggered Reporting	643

9.2.5	Intrafrequency measurements without measurement gaps	644

9.2.5.1	Intrafrequency cell identification	644

9.2.5.2	Measurement period	648

9.2.5.3	Scheduling availability of UE during intra-frequency measurements	651

9.2.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	651

9.2.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	652

9.2.5.3.3	Scheduling availability of UE performing measurements on FR2	652

9.2.5.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	654

9.2.5.4	SFTD Measurements between PCell and PSCell	654

9.2.5.4.1	Introduction	654

9.2.5.4.2	SFTD Measurement delay	654

9.2.5.4.3	SFTD Measurement Reporting Delay	655

9.2.6	Intra-frequency measurements with measurement gaps	655

9.2.6.1	Void	655

9.2.6.2	Intra-frequency cell identification	655

9.2.6.3	Intrafrequency Measurement Period	658

9.2.7	Intra-frequency measurements with NCSG	659

9.2.7.1	Intra-frequency cell identification	659

9.2.7.2	Measurement period	661

9.2.7.3	Scheduling availability during intra-frequency measurement with NCSG	662

9.2A	NR intra-frequency measurements with CCA	662

9.2A.1	Introduction	662

9.2A.2	Requirements applicability	663

9.2A.3	Number of cells and number of SSB	664

9.2A.3.1	Requirements for FR1	664

9.2A.3.2	Requirements for FR2-2	664

9.2A.4	Measurement Reporting Requirements	664

9.2A.5	Intra-frequency measurements without measurement gaps	665

9.2A.5.2	Measurement period	670

9.2A.5.3	Scheduling availability of UE during intra-frequency measurements	672

9.2A.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	672

9.2A.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	673

9.2A.5.3.3	Scheduling availability of UE performing measurements in TDD bands on FR2-2	673

9.2A.6	Intra-frequency measurements with measurement gaps	673

9.2A.6.1	Intra-frequency cell identification	673

9.2A.6.2	Intra-frequency Measurement Period	675

9.2A.7	Intra-frequency RSSI and Channel occupancy measurements	676

9.2A.7.1	Intra-frequency RSSI measurements	676

9.2A.7.2	Intra-frequency Channel occupancy measurements	678

9.2A.7.3	Scheduling restriction during RSSI and Channel Occupancy measurements in FR1	680

9.2A.7.4	Scheduling restriction during RSSI measurements in FR2-2	680

9.2B	NR intra-frequency measurements for RedCap	680

9.2B.1	Introduction	680

9.2B.2	Requirements applicability	681

9.2B.3	Number of cells and number of SSB	681

9.2B.3.1	Requirements for FR1	681

9.2B.3.2	Requirements for FR2	681

9.2B.4	Measurement Reporting Requirements	682

9.2B.4.1	Periodic Reporting	682

9.2B.4.2	Event-triggered Periodic Reporting	682

9.2B.4.3	Event Triggered Reporting	682

9.2B.5	Intra-frequency measurements without measurement gaps	682

9.2B.5.1	Intra-frequency cell identification	682

9.2B.5.2	Measurement period	685

9.2B.5.3	Scheduling availability of UE during intra-frequency measurements	686

9.2B.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	686

9.2B.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	686

9.2B.5.3.3	Scheduling availability of UE performing measurements on FR2	687

9.2B.5.3.4	Scheduling availability of UE performing measurements in HD-FDD bands on FR1	687

9.2B.6	Intra-frequency measurements with measurement gaps	688

9.2B.6.1	Intra-frequency cell identification	688

9.2B.6.2	Intra-frequency Measurement Period	689

9.2C	NR intra-frequency measurements for SAN	690

9.2C.1	Introduction	690

9.2C.2	Requirements applicability	690

9.2C.3	Number of cells and number of SSB	691

9.2C.3.1	Requirements for FR1	691

9.2C.4	Measurement Reporting Requirements	691

9.2C.4.1	Periodic Reporting	691

9.2C.4.2	Event-triggered Periodic Reporting	691

9.2C.4.3	Event Triggered Reporting	691

9.2C.5	Intra frequency measurements without measurement gaps	692

9.2C.5.1	Intra frequency cell identification	692

9.2C.5.2	Measurement period	694

9.2C.5.3	Scheduling availability of UE during intra-frequency measurements	694

9.2C.5.3.1	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	695

9.2C.5.3.2	Scheduling availability of UE performing measurements on a neighbor cell served by a different satellite in LEO	695

9.2C.6	Intra-frequency measurements with measurement gaps	695

9.2C.6.1	Intra-frequency cell identification	695

9.2C.6.3	Intrafrequency Measurement Period	697

9.3	NR inter-frequency measurements	697

9.3.1	Introduction	697

9.3.2	Requirements applicability	699

9.3.2.1	Void	699

9.3.2.2	Void	699

9.3.3	Number of cells and number of SSB	699

9.3.3.1	Requirements for FR1	699

9.3.3.2	Requirements for FR2	699

9.3.4	Inter-frequency measurement with measurement gaps	700

9.3.4.1	Void	703

9.3.4.2	Void	703

9.3.5	Inter-frequency measurements	703

9.3.5.1	Void	704

9.3.5.2	Void	704

9.3.5.3	Void	704

9.3.6	Inter-frequency measurements reporting requirements	704

9.3.6.1	Periodic Reporting	704

9.3.6.2	Event-triggered Periodic Reporting	705

9.3.6.3	Event-triggered Reporting	705

9.3.7	Void	705

9.3.8	Inter-frequency SFTD measurement requirements	705

9.3.8.1	Introduction	705

9.3.8.2	SFTD Measurement delay	705

9.3.8.3	SFTD Measurement reporting delay	706

9.3.9	Inter frequency measurements without measurement gaps	707

9.3.9.1	Inter frequency Cell identification	707

9.3.9.2	Measurement period	709

9.3.9.3	Scheduling availability of UE during inter-frequency measurements when the SSB is completely contained in the active BWP of the UE	711

9.3.9.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	711

9.3.9.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	711

9.3.9.3.3	Scheduling availability of UE performing measurements on FR2	712

9.3.9.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	712

9.3.9.4	Scheduling availability of UE during inter-frequency measurements when the SSB is not completely contained in the active BWP of the UE	712

9.3.9.4.1	Scheduling availability of UE performing measurements in TDD bands on FR1	713

9.3.9.4.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	713

9.3.9.4.3	Scheduling availability of UE performing measurements on FR2	714

9.3.9.4.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	716

9.3.10	Inter-frequency measurement with NCSG	716

9.3.10.1	Inter-frequency cell identification	716

9.3.10.2	Measurement period	717

9.3.10.3	Scheduling availability during inter-frequency measurement with NCSG	718

9.3.10.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	718

9.3.10.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	719

9.3.10.3.3	Scheduling availability of UE performing measurements on FR2	719

9.3.10.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	721

9.3A	NR inter-frequency measurements in carrier frequencies with CCA	721

9.3A.1	Introduction	721

9.3A.2	Requirements applicability	722

9.3A.3	Number of cells and number of SSB	722

9.3A.3.1	Requirements for FR1	722

9.3A.3.2	Requirements for FR2-2	722

9.3A.4	Inter-frequency cell identification	723

9.3A.5	Inter-frequency measurements	725

9.3A.6	NR Inter-frequency measurements reporting requirements	726

9.3A.6.1	Periodic Reporting	726

9.3A.6.2	Event-triggered Periodic Reporting	726

9.3A.6.3	Event-triggered Reporting	726

9.3A.8	Inter-frequency RSSI measurements	727

9.3A.9	Inter-frequency channel occupancy measurements	728

9.3B	NR inter-frequency measurements for RedCap	728

9.3B.1	Introduction	728

9.3B.2	Requirements applicability	729

9.3B.3	Number of cells and number of SSB	729

9.3B.3.1	Requirements for FR1	729

9.3B.3.2	Requirements for FR2	729

9.3B.4	Inter-frequency measurement with measurement gaps	730

9.3B.5	Inter-frequency measurements	731

9.3B.6	Inter-frequency measurements reporting requirements	732

9.3B.6.1	Periodic Reporting	732

9.3B.6.2	Event-triggered Periodic Reporting	732

9.3B.6.3	Event-triggered Reporting	732

9.3B.7	Inter frequency measurements without measurement gaps	733

9.3B.7.1	Inter frequency Cell identification	733

9.3B.7.2	Measurement period	735

9.3B.7.3	Scheduling availability of UE during inter-frequency measurements	736

9.3B.7.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	736

9.3B.7.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	736

9.3B.7.3.3	Scheduling availability of UE performing measurements on FR2	737

9.3B.7.3.4	Scheduling availability of UE performing measurements in HD-FDD bands on FR1	737

9.3C	NR inter-frequency measurements for SAN	737

9.3C.1	Introduction	737

9.3C.2	Requirements applicability	738

9.3C.3	Number of cells and number of SSB	738

9.3C.3.1	Requirements for FR1	738

9.3C.4	Inter-frequency measurement with measurement gaps	738

9.3C.5	Inter-frequency measurements	740

9.3C.6	Inter-frequency measurements reporting requirements	740

9.3C.6.1	Periodic Reporting	740

9.3C.6.2	Event-triggered Periodic Reporting	740

9.3C.6.3	Event-triggered Reporting	740

9.3C.7	Inter frequency measurements without measurement gaps	741

9.3C.7.1	Inter frequency Cell identification	741

9.3C.7.2	Measurement period	743

9.3C.7.3	Scheduling availability of UE during inter-frequency measurements	743

9.3C.7.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	743

9.3C.7.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	743

9.4	Inter-RAT measurements	744

9.4.1	Introduction	744

9.4.2	NR − E-UTRAN FDD measurements	746

9.4.2.1	Introduction	746

9.4.2.2	Requirements when no DRX is used	746

9.4.2.3	Requirements when DRX is used	747

9.4.2.4	Measurement reporting requirements	749

9.4.2.4.1	Periodic Reporting	749

9.4.2.4.2	Event-Triggered Periodic Reporting	749

9.4.2.4.3	Event-Triggered Reporting	750

9.4.2.5	Scheduling Availability During NR − E-UTRAN FDD measurements with NCSG	750

9.4.3	NR − E-UTRAN TDD measurements	750

9.4.3.1	Introduction	750

9.4.3.2	Requirements when no DRX is used	750

9.4.3.3	Requirements when DRX is used	752

9.4.3.4	Measurement reporting requirements	753

9.4.3.4.1	Periodic Reporting	753

9.4.3.4.2	Event-Triggered Periodic Reporting	754

9.4.3.4.3	Event-Triggered Reporting	754

9.4.3.5	Scheduling Availability During NR − E-UTRAN TDD measurements with NCSG	754

9.4.4	Inter-RAT RSTD measurements	754

9.4.4.1	NR − E-UTRAN FDD RSTD measurements	754

9.4.4.1.1	Introduction	754

9.4.4.1.2	Requirements	755

9.4.4.2	NR − E-UTRAN TDD RSTD measurements	758

9.4.4.2.1	Introduction	758

9.4.4.2.2	Requirements	759

9.4.5	Inter-RAT E-CID measurements	762

9.4.5.1	NR−E-UTRAN FDD E-CID RSRP and RSRQ measurements	762

9.4.5.1.1	Introduction	762

9.4.5.1.2	Requirements	762

9.4.5.1.3	Measurement Reporting Delay	762

9.4.5.2	NR−E-UTRAN TDD E-CID RSRP and RSRQ measurements	763

9.4.5.2.1	Introduction	763

9.4.5.2.2	Requirements	763

9.4.5.2.3	Measurement Reporting Delay	763

9.4.6	NR − UTRAN FDD measurements	763

9.4.6.1	Introduction	763

9.4.6.2	Requirements when no DRX is used	763

9.4.6.3	Requirements when DRX is used	764

9.4.7	NR – E-UTRAN measurements with autonomous gaps	766

9.4.7.1	CGI identification of an E-UTRA cell with autonomous gaps	766

9.4.7.2	CGI reporting delay	766

9.4A	Inter-RAT measurements for RedCap	767

9.4A.1	Introduction	767

9.4A.2	NR − E-UTRAN FDD measurements	768

9.4A.2.1	Introduction	768

9.4A.2.2	Requirements when no DRX is used	768

9.4A.2.3	Requirements when DRX is used	769

9.4A.2.4	Measurement reporting requirements	770

9.4A.2.4.1	Periodic Reporting	770

9.4A.2.4.2	Event-Triggered Periodic Reporting	770

9.4A.2.4.3	Event-Triggered Reporting	770

9.4A.3	NR − E-UTRAN TDD measurements	771

9.4A.3.1	Introduction	771

9.4A.3.2	Requirements when no DRX is used	771

9.4A.3.3	Requirements when DRX is used	773

9.4A.3.4	Measurement reporting requirements	774

9.4A.3.4.1	Periodic Reporting	774

9.4A.3.4.2	Event-Triggered Periodic Reporting	774

9.4A.3.4.3	Event-Triggered Reporting	774

9.4A.4	NR – E-UTRAN measurements with autonomous gaps	775

9.4A.4.1	CGI identification of an E-UTRA cell with autonomous gaps	775

9.4A.4.2	CGI reporting delay	775

9.4A.4.3	CGI reporting scheduling restriction	775

9.5	L1-RSRP measurements for Reporting	776

9.5.1	Introduction	776

9.5.2	Requirements applicability	776

9.5.3	Measurement Reporting Requirements	777

9.5.3.1	Periodic Reporting	777

9.5.3.2	Semi-Persistent Reporting	777

9.5.3.3	Aperiodic Reporting	777

9.5.4	L1-RSRP measurement requirements	778

9.5.4.1	SSB based L1-RSRP Reporting	778

9.5.4.2	CSI-RS based L1-RSRP Reporting	781

9.5.4A	Void	784

9.5.4A.1	Void	784

9.5.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	784

9.5.5.1	Measurement restriction for SSB based L1-RSRP	784

9.5.5.2	Measurement restriction for CSI-RS based L1-RSRP	785

9.5.6	Scheduling availability of UE during L1-RSRP measurement	786

9.5.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	786

9.5.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	786

9.5.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	786

9.5.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	788

9.5A	L1-RSRP measurements for Reporting under CCA	788

9.5A.1	Introduction	788

9.5A.2	Requirements applicability	788

9.5A.3	Measurement Reporting Requirements	788

9.5A.3.1	Periodic Reporting	789

9.5A.3.2	Semi-Persistent Reporting	789

9.5A.3.3	Aperiodic Reporting	789

9.5A.4	L1-RSRP measurement requirements	789

9.5A.4.1	SSB based L1-RSRP Reporting	789

9.5A.5	Measurement restriction for L1-RSRP measurement	792

9.5A.5.1	Measurement restriction for SSB based L1-RSRP	792

9.5A.6	Scheduling availability of UE during L1-RSRP measurement	792

9.5A.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	792

9.5A.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	793

9.5A.6.3	Scheduling availability of UE performing L1-RSRP measurement in case of FR1-FR2 inter-band CA	793

9.5A.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2-2	793

9.5A.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	794

9.5B	L1-RSRP measurements for Reporting for RedCap	794

9.5B.1	Introduction	794

9.5B.2	Requirements applicability	794

9.5B.3	Measurement Reporting Requirements	795

9.5B.3.1	Periodic Reporting	795

9.5B.3.2	Semi-Persistent Reporting	795

9.5B.3.3	Aperiodic Reporting	795

9.5B.4	L1-RSRP measurement requirements	796

9.5B.4.1	SSB based L1-RSRP Reporting	796

9.5B.4.2	CSI-RS based L1-RSRP Reporting	797

9.5B.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	800

9.5B.5.1	Measurement restriction for SSB based L1-RSRP	800

9.5B.5.2	Measurement restriction for CSI-RS based L1-RSRP	801

9.5B.6	Scheduling availability of UE during L1-RSRP measurement	801

9.5B.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	801

9.5B.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	801

9.5B.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	802

9.5C	L1-RSRP measurements for Reporting for satellite access	802

9.5C.1	Introduction	802

9.5C.3	Measurement Reporting Requirements	803

9.5C.3.1	Periodic Reporting	803

9.5C.3.2	Semi-Persistent Reporting	803

9.5C.3.3	Aperiodic Reporting	804

9.5C.4	L1-RSRP measurement requirements	804

9.5C.4.1	SSB based L1-RSRP Reporting	804

9.5C.5	Measurement restriction for L1-RSRP measurement	806

9.5C.5.1	Measurement restriction for SSB based L1-RSRP	806

9.5C.5.2	Measurement restriction for CSI-RS based L1-RSRP	806

9.5C.6	Scheduling availability of UE during L1-RSRP measurement	807

9.5C.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	807

9.5C.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	807

9.6	NE-DC: Measurements	807

9.6.1	Introduction	807

9.6.2	SFTD Measurements	807

9.6.2.1	Introduction	807

9.6.2.2	SFTD Measurement requirements	807

9.7	Cross Link Interference measurements	808

9.7.1	Introduction	808

9.7.2	SRS-RSRP measurements	808

9.7.2.1	Introduction	808

9.7.2.2	Requirements applicability	809

9.7.2.3	Measurement Reporting Requirements	809

9.7.2.3.1	Periodic Reporting	809

9.7.2.3.2	Event-triggered Periodic Reporting	809

9.7.2.3.3	Event Triggered Reporting	809

9.7.2.4	Measurement capability	809

9.7.2.5	SRS-RSRP measurement period	810

9.7.3	CLI-RSSI measurements	810

9.7.3.1	Introduction	810

9.7.3.2	Requirements applicability	810

9.7.3.3	Measurement Reporting Requirements	810

9.7.3.3.1	Periodic Reporting	810

9.7.3.3.2	Event-triggered Periodic Reporting	810

9.7.3.3.3	Event Triggered Reporting	811

9.7.3.4	Measurement capability	811

9.7.3.5	CLI-RSSI measurement period	811

9.7.4	Scheduling availability of UE during CLI measurements	811

9.7.4.1	Scheduling availability of UE performing measurement on FR1	811

9.7.4.2	Scheduling availability of UE performing measurement on FR2	812

9.8	L1-SINR measurements for Reporting	812

9.8.1	Introduction	812

9.8.2	Requirements applicability	813

9.8.3	Measurement Reporting Requirements	813

9.8.3.1	Periodic Reporting	813

9.8.3.2	Semi-Persistent Reporting	814

9.8.4	L1-SINR measurement requirements	814

9.8.4.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	814

9.8.4.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	819

9.8.5	Measurement restriction for L1-SINR measurement	820

9.8.5.1	Measurement restriction if SSB configured for L1-SINR Measurement	821

9.8.5.2	Measurement restriction if CSI-RS configured for L1-SINR measurement	821

9.8.5.3	Measurement restriction if CSI-IM configured for L1-SINR measurement	822

9.8.6	Scheduling availability of UE during L1-SINR measurement	822

9.8.6.1	Scheduling availability of UE performing L1-SINR measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	822

9.8.6.2	Scheduling availability of UE performing L1-SINR measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	822

9.8.6.4	Scheduling availability of UE performing L1-SINR measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	823

9.9	 NR measurements for positioning	824

9.9.1	Introduction	824

9.9.1.1	General Aspects of Gap-based Measurement	824

9.9.1.2	General Aspects of Gapless Measurement	824

9.9.1.3	Scheduling Availability of UE during PRS Measurement without Measurement Gaps	825

9.9.2	RSTD measurements	826

9.9.2.1	Introduction	826

9.9.2.2	Requirements Applicability	826

9.9.2.3	Measurement Capability	826

9.9.2.4	Measurement Reporting Requirements	826

9.9.2.4.1	Void	827

9.9.2.4.2	Void	827

9.9.2.4.3	Void	827

9.9.2.5	Measurements Period Requirements	827

9.9.2.6	Void	830

9.9.2.7	Measurements Period Requirements without Measurement Gaps	830

9.9.2.8	Void	833

9.9.2.9	Measurements Period Requirements with both MG and PPW	833

9.9.3	PRS-RSRP measurements	833

9.9.3.1	Introduction	833

9.9.3.2	Requirements applicability	833

9.9.3.3	Measurement Capability	834

9.9.3.4	Measurement Reporting Requirements	834

9.9.3.5	Measurement Period Requirements	834

9.9.3.6	Measurement Period Requirements without Measurement Gaps	836

9.9.3.7	Void	839

9.9.3.8	Measurements Period Requirements with both MG and PPW	839

9.9.4	UE Rx-Tx time difference measurements	840

9.9.4.1 Introduction	840

9.9.4.2 Requirements Applicability	840

9.9.4.3 Measurement Capability	840

9.9.4.4 Measurement Reporting Requirements	840

9.9.4.5	Measurement Period Requirements	840

9.9.4.6	Measurement Period Requirements without Measurement Gaps	844

9.9.4.7	Void	847

9.9.4.8	Measurements Period Requirements with both MG and PPW	847

9.9.5	E-CID measurements	848

9.9.5.1	Introduction	848

9.9.5.2	Measurement Requirements	848

9.9.5.2.1	Intra-frequency Measurement Requirements	848

9.9.5.2.2	Inter-frequency Measurement Requirements	848

9.9.5.2.3	Measurement Reporting Delay	849

9.9.6	PRS-RSRPP measurements	849

9.9.6.1	Introduction	849

9.9.6.2	Requirements applicability	849

9.9.6.3	Measurement capability	849

9.9.6.4	Measurement reporting requirements	849

9.9.6.5	Measurement period requirements	849

9.9.6.6	Measurement Period Requirements without Measurement Gaps	850

9.9.6.7	Void	850

9.9.6.8	Measurements Period Requirements with both MG and PPW	850

9.10	CSI-RS based L3 measurements	850

9.10.1	Introduction	850

9.10.2	CSI-RS based intra-frequency measurements	850

9.10.2.1	Introduction	850

9.10.2.2	Requirements applicability	851

9.10.2.3	Number of cells and number of CSI-RS	852

9.10.2.3.1	Requirements for FR1	852

9.10.2.3.2	Requirements for FR2	852

9.10.2.4	Measurement Reporting Requirements	852

9.10.2.4.1	Periodic Reporting	852

9.10.2.4.2	Event-triggered Periodic Reporting	853

9.10.2.4.3	Event Triggered Reporting	853

9.10.2.5	Intra-frequency measurements without measurement gaps	853

9.10.2.6	Scheduling availability of UE during CSI-RS based intra-frequency measurements	854

9.10.2.6.1	Scheduling availability of UE performing CSI-RS based measurements in TDD bands	855

9.10.2.6.2	Scheduling availability of UE performing CSI-RS based measurements in FR2	855

9.10.3	CSI-RS based Inter-frequency measurements	855

9.10.3.1	Introduction	855

9.10.3.2	Requirements applicability	855

9.10.3.3	Number of cells and number of CSI-RS resources	856

9.10.3.3.1	Requirements for FR1	856

9.10.3.3.2	Requirements for FR2	856

9.10.3.4	Measurements reporting requirements	856

9.10.3.4.1	Periodic Reporting	857

9.10.3.4.2	Event-triggered Periodic Reporting	857

9.10.3.4.3	Event-triggered Reporting	857

9.10.3.5	Inter frequency measurements with measurement gaps	857

9.11	NR measurements with autonomous gaps	859

9.11.1	Introduction	859

9.11.2	CGI identification of an NR cell with autonomous gaps	859

9.11.3	CGI reporting delay	860

9.11A	NR measurements with autonomous gaps for RedCap	860

9.11A.1	Introduction	860

9.11A.2	CGI identification of an NR cell with autonomous gaps	860

9.11A.3	CGI reporting delay	861

9.11A.4	CGI reporting scheduling restriction	862

9.12	Measurement for Propagation Delay Compensation	862

9.12.1	Introduction	862

9.12.2	Requirements Applicability	862

9.12.3	Measurement Capability	863

9.12.4	Measurement period requirements	863

9.12.4.1	PRS Measurement Period	863

9.12.4.2	TRS Measurement Period	864

9.12.5	Measurement Reporting Requirements	865

9.12.6	Scheduling availability during measurement for Propagation Delay Compensation	865

9.12.7	Measurement restriction for measurement for Propagation Delay Compensation	865

9.13	L1-RSRP measurements for a cell with different PCI from serving cell	865

9.13.1	Introduction	865

9.13.2	Requirements Applicability	866

9.13.3	Measurement Reporting Requirements	866

9.13.3.1	Periodic Reporting	866

9.13.3.2	Semi-Persistent Reporting	867

9.13.3.3	Aperiodic Reporting	867

9.13.4	L1-RSRP measurement requirements	867

9.13.4.1	Inter-cell SSB based L1-RSRP Reporting	867

9.13.5	Measurement restriction for L1-RSRP measurement	869

9.13.5.1	Measurement restriction for SSB based L1-RSRP	869

9.13.6	Scheduling availability of UE during L1-RSRP measurement	870

9.13.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	870

9.13.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	870

9.13.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	871

9.13.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	871

9.13.6.5	Scheduling availability of UE performing L1-RSRP measurement in TDD bands on FR1	871

10	Measurement Performance requirements	872

10.1	NR measurements	872

10.1.1	Introduction	872

10.1.2	Intra-frequency RSRP accuracy requirements for FR1	872

10.1.2.1	Intra-frequency SS-RSRP accuracy requirements	872

10.1.2.1.1	Absolute SS-RSRP Accuracy	872

10.1.2.1.2	Relative SS-RSRP Accuracy	873

10.1.2.2	Void	874

10.1.2.3	Intra-frequency CSI-RSRP accuracy requirements	874

10.1.2.3.1	Absolute CSI-RSRP Accuracy	874

10.1.2.3.2	Relative CSI-RSRP Accuracy	875

10.1.2B	Intra-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	876

10.1.2B.1	Intra-frequency SS-RSRP accuracy requirements	876

10.1.2B.1.1	Absolute SS-RSRP Accuracy	876

10.1.2C	Intra-frequency RSRP accuracy requirements for FR1 SAN	877

10.1.2C.1	Intra-frequency SS-RSRP accuracy requirements	877

10.1.2C.1.1	Absolute SS-RSRP Accuracy	877

10.1.2C.1.2	Relative SS-RSRP Accuracy	878

10.1.3	Intra-frequency RSRP accuracy requirements for FR2	878

10.1.3.1	Intra-frequency SS-RSRP accuracy requirements	878

10.1.3.1.1	Absolute SS-RSRP Accuracy	878

10.1.3.1.2	Relative SS-RSRP Accuracy	879

10.1.3.2	Void	880

10.1.3.3	Intra-frequency CSI-RSRP accuracy requirements	880

10.1.3.3.1	Absolute CSI-RSRP Accuracy	880

10.1.3.3.2	Relative CSI-RSRP Accuracy	881

10.1.3B	Intra-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	882

10.1.3B.1	Intra-frequency SS-RSRP accuracy requirements	882

10.1.3B.1.1	Absolute SS-RSRP Accuracy	882

10.1.4	Inter-frequency RSRP accuracy requirements for FR1	883

10.1.4.1	Inter-frequency SS-RSRP accuracy requirements	883

10.1.4.1.1	Absolute Accuracy of SS-RSRP in FR1	883

10.1.4.1.2	Relative Accuracy of SS-RSRP in FR1	884

10.1.4.2	Void	885

10.1.4.3	Inter-frequency CSI-RSRP accuracy requirements	885

10.1.4.3.1	Absolute Accuracy of CSI-RSRP in FR1	885

10.1.4.3.2	Relative Accuracy of CS-RSRP in FR1	886

10.1.4B	Inter-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	887

10.1.4B.1	Inter-frequency SS-RSRP accuracy requirements	887

10.1.4B.1.1	Absolute Accuracy of SS-RSRP in FR1	887

10.1.4C	Inter-frequency RSRP accuracy requirements for FR1 SAN	888

10.1.4C.1	Inter-frequency SS-RSRP accuracy requirements	888

10.1.4C.1.1	Absolute Accuracy of SS-RSRP in FR1	888

10.1.4C.1.2	Relative Accuracy of SS-RSRP in FR1	889

10.1.5	Inter-frequency RSRP accuracy requirements for FR2	889

10.1.5.1	Inter-frequency SS-RSRP accuracy requirements	889

10.1.5.1.1	Absolute SS-RSRP Accuracy	889

10.1.5.1.2	Relative SS-RSRP Accuracy	890

10.1.5.2	Void	891

10.1.5.3	Inter-frequency CSI-RSRP accuracy requirements	891

10.1.5.3.1	Absolute CSI-RSRP Accuracy	891

10.1.5.3.2	Relative CSI-RSRP Accuracy	892

10.1.5B	Inter-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	893

10.1.5B.1	Inter-frequency SS-RSRP accuracy requirements	893

10.1.5B.1.1	Absolute SS-RSRP Accuracy	893

10.1.6	RSRP Measurement Report Mapping	894

10.1.7	Intra-frequency RSRQ accuracy requirements for FR1	896

10.1.7.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	896

10.1.7.1.1	Absolute SS-RSRQ Accuracy in FR1	896

10.1.7.2	Intra-frequency CSI-RSRQ accuracy requirements	897

10.1.7.2.1	Absolute CSI-RSRQ Accuracy	897

10.1.7B	Intra-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	898

10.1.7B.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	898

10.1.7B.1.1	Absolute SS-RSRQ Accuracy in FR1	898

10.1.7C	Intra-frequency RSRQ accuracy requirements for FR1 SAN	899

10.1.7C.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	899

10.1.7C.1.1	Absolute SS-RSRQ Accuracy in FR1	899

10.1.8	Intra-frequency RSRQ accuracy requirements for FR2	900

10.1.8.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	900

10.1.8.1.1	Absolute SS-RSRQ Accuracy in FR2	900

10.1.8.2	Intra-frequency CSI-RSRQ accuracy requirements	901

10.1.8.2.1	Absolute CSI-RSRQ Accuracy	901

10.1.8B	Intra-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	901

10.1.8B.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	901

10.1.8B.1.1	Absolute SS-RSRQ Accuracy in FR2	902

10.1.9	Inter-frequency RSRQ accuracy requirements for FR1	902

10.1.9.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	902

10.1.9.1.1	Absolute Accuracy of SS-RSRQ in FR1	902

10.1.9.1.2	Relative Accuracy of SS-RSRQ in FR1	903

10.1.9.2	Inter-frequency CSI-RSRQ accuracy requirements	904

10.1.9.2.1	Absolute CSI-RSRQ Accuracy	904

10.1.9.2.2	Relative CSI-RSRQ Accuracy	905

10.1.9B	Inter-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	906

10.1.9B.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	906

10.1.9B.1.1	Absolute Accuracy of SS-RSRQ in FR1	906

10.1.9C	Inter-frequency RSRQ accuracy requirements for FR1 SAN	907

10.1.9C.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	907

10.1.9C.1.1	Absolute Accuracy of SS-RSRQ in FR1	907

10.1.9C.1.2	Relative Accuracy of SS-RSRQ in FR1	908

10.1.10	Inter-frequency RSRQ accuracy requirements for FR2	909

10.1.10.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	909

10.1.10.1.1	Absolute Accuracy of SS-RSRQ in FR2	909

10.1.10.1.2	Relative Accuracy of SS-RSRQ in FR2	909

10.1.10.2	Inter-frequency CSI-RSRQ accuracy requirements	910

10.1.10.2.1	Absolute CSI-RSRQ Accuracy	910

10.1.10.2.2	Relative CSI-RSRQ Accuracy	911

10.1.10B	 Inter-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	912

10.1.10B.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	912

10.1.10B.1.1	Absolute Accuracy of SS-RSRQ in FR2	912

10.1.11	RSRQ report mapping	913

10.1.12	Intra-frequency SINR accuracy requirements for FR1	913

10.1.12.1	Intra-frequency SS-SINR accuracy requirements in FR1	913

10.1.12.1.1	Absolute SS-SINR Accuracy in FR1	913

10.1.12.2	Intra-frequency CSI-SINR accuracy requirements in FR1	914

10.1.12.2.1	Absolute CSI-SINR Accuracy in FR1	914

10.1.12C	Intra-frequency SINR accuracy requirements for FR1 SAN	915

10.1.12C.1	Intra-frequency SS-SINR accuracy requirements in FR1	915

10.1.12C.1.1	Absolute SS-SINR Accuracy in FR1	915

10.1.13	Intra-frequency SINR accuracy requirements for FR2	916

10.1.13.1	Intra-frequency SS-SINR accuracy requirements in FR2	916

10.1.13.1.1	Absolute SS-SINR Accuracy in FR2	916

10.1.13.2	Intra-frequency CSI-SINR accuracy requirements in FR2	917

10.1.13.2.1	Absolute CSI-SINR Accuracy in FR2	917

10.1.14	Inter-frequency SINR accuracy requirements for FR1	917

10.1.14.1	Inter-frequency SS-SINR accuracy requirements in FR1	917

10.1.14.1.1	Aboslute Accuracy of SS-SINR in FR1	917

10.1.14.1.2	Relative Accuracy of SS-SINR in FR1	918

10.1.14.2	Inter-frequency CSI-SINR accuracy requirements in FR1	919

10.1.14.2.1	Aboslute Accuracy of CSI-SINR in FR1	919

10.1.14.2.2	Relative Accuracy of CSI-SINR in FR1	920

10.1.14C	Inter-frequency SINR accuracy requirements for FR1 SAN	921

10.1.14C.1	Inter-frequency SS-SINR accuracy requirements in FR1	921

10.1.14C.1.1	Aboslute Accuracy of SS-SINR in FR1	921

10.1.14C.1.2	Relative Accuracy of SS-SINR in FR1	922

10.1.15	Inter-frequency SINR accuracy requirements for FR2	923

10.1.15.1	Inter-frequency SS-SINR accuracy requirements in FR2	923

10.1.15.1.1	Aboslute Accuracy of SS-SINR in FR2	923

10.1.15.1.2	Relative Accuracy of SS-SINR in FR2	923

10.1.15.2	Inter-frequency CSI-SINR accuracy requirements in FR2	924

10.1.15.2.1	Aboslute Accuracy of CSI-SINR in FR2	924

10.1.15.2.2	Relative Accuracy of CSI-SINR in FR2	925

10.1.16	SINR report mapping	926

10.1.16.1	SS-SINR and CSI-SINR measurement report mapping	926

10.1.17	Power Headroom	927

10.1.17.1	Power Headroom Report	927

10.1.17.1.1	Power Headroom Report Mapping	927

10.1.18	PCMAX,c,f	928

10.1.18.1	Report Mapping	928

10.1.19	L1-RSRP accuracy requirements for FR1	928

10.1.19.1	SSB based L1-RSRP accuracy requirements	928

10.1.19.1.1	Absolute Accuracy	928

10.1.19.1.2	Relative Accuracy	929

10.1.19.2	CSI-RS based L1-RSRP accuracy requirements	930

10.1.19.2.1	Absolute Accuracy	930

10.1.19.2.2	Relative Accuracy	931

10.1.19C	L1-RSRP accuracy requirements for FR1 SAN	932

10.1.19C.1	SSB based L1-RSRP accuracy requirements	932

10.1.19C.1.1	Absolute Accuracy	932

10.1.19C.1.2	Relative Accuracy	933

10.1.20	L1-RSRP accuracy requirements for FR2	933

10.1.20.1	SSB based L1-RSRP accuracy requirements	933

10.1.20.1.1	Absolute Accuracy	933

10.1.20.1.2	Relative Accuracy	934

10.1.20.2	CSI-RS based L1-RSRP accuracy requirements	935

10.1.20.2.1	Absolute Accuracy	935

10.1.20.2.2	Relative Accuracy	936

10.1.21	SFTD accuracy requirements	937

10.1.21.1	SFTD acuracy requirements for NE-DC	937

10.1.21.2	SFTD acuracy requirements for NR-DC	939

10.1.21.3	Inter frequency SFTD acuracy requirements	940

10.1.22	CLI measurement accuracy requirements	941

10.1.22.1	SRS-RSRP	941

10.1.22.1.1	SRS-RSRP Accuracy	941

10.1.22.1.2	SRS-RSRP report mapping	943

10.1.22.2	CLI-RSSI	943

10.1.22.2.1	CLI-RSSI Accuracy	943

10.1.22.2.2	CLI-RSSI report mapping	944

10.1.23	RSTD Measurements	945

10.1.23.1	Introduction	945

10.1.23.2	Measurement Accuracy Requirements	945

10.1.23.3	Report mapping	955

10.1.23.3.1	Absolute DL RSTD Measurement Reporting	955

10.1.23.3.2	Differential Reporting for DL RSTD Measurement	957

10.1.23.3.3	Additional Path Report Mapping for DL RSTD	959

10.1.24	PRS-RSRP Measurements	961

10.1.24.1	Introduction	961

10.1.24.2	Measurement Accuracy Requirements	961

10.1.24.2.1	Absolute PRS RSRP accuracy	961

10.1.24.2.2	Relative PRS RSRP accuracy	965

10.1.24.3	Report mapping	969

10.1.24.3.1	Absolute PRS-RSRP Measurement Report Mapping	969

10.1.24.3.2	Differential Report Mapping for PRS-RSRP Measurement	970

10.1.25	UE Rx-Tx Time Difference Measurements	972

10.1.25.1	Introduction	972

10.1.25.2	Measurement Accuracy Requirements	972

10.1.25.3	Report mapping	985

10.1.25.3.1	Absolute UE Rx-Tx Measurement Report Mapping	985

10.1.25.3.2	Differential UE Rx-Tx Measurement Report Mapping	987

10.1.25.3.3	Additional Path Report Mapping for UE Rx-Tx Time Difference	988

10.1.26	FR2 P-MPR report	990

10.1.26.1	Report mapping	990

10.1.27	L1-SINR accuracy requirements for FR1	990

10.1.27.1	L1-SINR accuracy requirements with CSI-RS based CMR and no dedicated IMR configured	990

10.1.27.1.1	Absolute Accuracy	990

10.1.27.1.2	Relative Accuracy	991

10.1.27.2	L1-SINR accuracy requirements with SSB based CMR and dedicated IMR configured	992

10.1.27.2.1	Absolute Accuracy	992

10.1.27.2.2	Relative Accuracy	993

10.1.27.3	L1-SINR accuracy requirements with CSI-RS based CMR and dedicated IMR configured	995

10.1.27.3.1	Absolute Accuracy	995

10.1.27.3.2	Relative Accuracy	997

10.1.28	L1-SINR accuracy requirements for FR2	998

10.1.29	Intra-frequency RSRQ accuracy requirements under CCA	1005

10.1.29.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	1005

10.1.29.1.1	Absolute SS-RSRQ Accuracy	1005

10.1.30	Inter-frequency RSRQ accuracy requirements under CCA	1005

10.1.30.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	1005

10.1.30.1.1	Aboslute Accuracy of SS-RSRQ	1005

10.1.30.1.2	Relative Accuracy of SS-RSRQ	1006

10.1.31	Intra-frequency SINR accuracy requirements under CCA	1007

10.1.31.1	Intra-frequency SS-SINR accuracy requirements in FR1	1007

10.1.31.1.1	Absolute SS-SINR Accuracy	1007

10.1.32	Inter-frequency SINR accuracy requirements under CCA	1007

10.1.32.1	Inter-frequency SS-SINR accuracy requirements in FR1	1007

10.1.32.1.1	Aboslute Accuracy of SS-SINR	1007

10.1.32.1.2	Relative Accuracy of SS-SINR	1008

10.1.33	L1-RSRP accuracy requirements under CCA	1009

10.1.33.1	SSB based L1-RSRP accuracy requirements in FR1	1009

10.1.33.1.1	Absolute Accuracy	1009

10.1.33.1.2	Relative Accuracy	1009

10.1.34	RSSI measurements under CCA	1010

10.1.34.1	Intra-frequency absolute RSSI measurement accuracy requirements in FR1	1010

10.1.34.2	Inter-frequency absolute RSSI measurement accuracy requirements in FR1	1010

10.1.34.3	RSSI measurement report mapping	1010

10.1.35	Channel occupancy measurements under CCA	1011

10.1.35.1	Intra-frequency channel occupancy measurement accuracy requirements in FR1	1011

10.1.35.2	Inter-frequency channel occupancy measurement accuracy requirements in FR1	1011

10.1.36	Intra-frequency RSRP accuracy requirements under CCA	1011

10.1.36.1	Intra-frequency SS-RSRP accuracy requirements in FR1	1011

10.1.36.1.1	Absolute SS-RSRP Accuracy	1011

10.1.36.1.2	Relative SS-RSRP Accuracy	1012

10.1.37	Inter-frequency RSRP accuracy requirements under CCA	1013

10.1.37.1	Inter-frequency SS-RSRP accuracy requirements in FR1	1013

10.1.37.1.1	Absolute Accuracy of SS-RSRP	1013

10.1.37.1.2	Relative Accuracy of SS-RSRP	1013

10.1.38	PRS-RSRPP Measurements	1014

10.1.38.1	Introduction	1014

10.1.38.2	Measurement Accuracy Requirements	1014

10.1.38.2.1	Absolute PRS RSRPP accuracy	1014

10.1.38.3	Report mapping	1018

10.1.38.3.1	Absolute PRS-RSRPP Measurement Report Mapping	1018

10.1.38.3.2	Differential Report Mapping for PRS-RSRPP Measurement	1019

10.1.39	UE Rx-Tx time difference measurements for RTT-based PDC	1020

10.1.39.1	 Void	1020

10.1.39.2	 Measurement Accuracy Requirements for PRS	1020

10.1.39.3	 Measurement Accuracy Requirements for TRS	1024

10.1.40	Void	1027

10.1A	NR measurements for RedCap	1027

10.1A.1	Introduction	1027

10.1A.2	Intra-frequency RSRP accuracy requirements for FR1	1027

10.1A.2.1	Intra-frequency SS-RSRP accuracy requirements	1027

10.1A.2.1.1	Absolute SS-RSRP Accuracy	1027

10.1A.2.1.2	Relative SS-RSRP Accuracy	1028

10.1A.3	Intra-frequency RSRP accuracy requirements for FR2	1029

10.1A.3.1	Intra-frequency SS-RSRP accuracy requirements	1029

10.1A.3.1.1	Absolute SS-RSRP Accuracy	1029

10.1A.3.1.2	Relative SS-RSRP Accuracy	1029

10.1A.4	Inter-frequency RSRP accuracy requirements for FR1	1029

10.1A.4.1	Inter-frequency SS-RSRP accuracy requirements	1029

10.1A.4.1.1	Absolute Accuracy of SS-RSRP in FR1	1029

10.1A.4.1.2	Relative Accuracy of SS-RSRP in FR1	1030

10.1A.5	Inter-frequency RSRP accuracy requirements for FR2	1031

10.1A.5.1	Inter-frequency SS-RSRP accuracy requirements	1031

10.1A.5.1.1	Absolute SS-RSRP Accuracy	1031

10.1A.5.1.2	Relative SS-RSRP Accuracy	1031

10.1A.6	Intra-frequency RSRQ accuracy requirements for FR1	1031

10.1A.6.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	1031

10.1A.6.1.1	Absolute SS-RSRQ Accuracy in FR1	1031

10.1A.7	Intra-frequency RSRQ accuracy requirements for FR2	1032

10.1A.7.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	1032

10.1A.7.1.1	Absolute SS-RSRQ Accuracy in FR2	1032

10.1A.8	Inter-frequency RSRQ accuracy requirements for FR1	1032

10.1A.8.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	1032

10.1A.8.1.1	Absolute Accuracy of SS-RSRQ in FR1	1032

10.1A.8.1.2	Relative Accuracy of SS-RSRQ in FR1	1033

10.1A.9	Inter-frequency RSRQ accuracy requirements for FR2	1034

10.1A.9.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	1034

10.1A.9.1.1	Absolute Accuracy of SS-RSRQ in FR2	1034

10.1A.9.1.2	Relative Accuracy of SS-RSRQ in FR2	1034

10.1A.10	 Intra-frequency SINR accuracy requirements for FR1	1034

10.1A.10.1	Intra-frequency SS-SINR accuracy requirements in FR1	1034

10.1A.10.1.1	Absolute SS-SINR Accuracy in FR1	1034

10.1A.11	Intra-frequency SINR accuracy requirements for FR2	1035

10.1A.11.1	Intra-frequency SS-SINR accuracy requirements in FR2	1035

10.1A.11.1.1	Absolute SS-SINR Accuracy in FR2	1035

10.1A.12 	Inter-frequency SINR accuracy requirements for FR1	1035

10.1A.12.1	Inter-frequency SS-SINR accuracy requirements in FR1	1035

10.1A.12.1.1	Aboslute Accuracy of SS-SINR in FR1	1035

10.1A.12.1.2	Relative Accuracy of SS-SINR in FR1	1036

10.1A.13	 Inter-frequency SINR accuracy requirements for FR2	1037

10.1A.13.1	Inter-frequency SS-SINR accuracy requirements in FR2	1037

10.1A.13.1.1	Aboslute Accuracy of SS-SINR in FR2	1037

10.1A.13.1.2	Relative Accuracy of SS-SINR in FR2	1037

10.1A.14	 L1-RSRP accuracy requirements for FR1	1037

10.1A.14.1	SSB based L1-RSRP accuracy requirements	1037

10.1A.14.1.1	Absolute Accuracy	1037

10.1A.14.1.2	Relative Accuracy	1038

10.1A.14.2	CSI-RS based L1-RSRP accuracy requirements	1039

10.1A.14.2.1	Absolute Accuracy	1039

10.1A.14.2.2	Relative Accuracy	1040

10.1A.15	 L1-RSRP accuracy requirements for FR2	1041

10.1A.15.1	SSB based L1-RSRP accuracy requirements	1041

10.1A.15.1.1	Absolute Accuracy	1041

10.1A.15.1.2	Relative Accuracy	1041

10.1A.15.2	CSI-RS based L1-RSRP accuracy requirements	1041

10.1A.15.2.1	Absolute Accuracy	1041

10.1A.15.2.2	Relative Accuracy	1041

10.2	E-UTRAN measurements	1042

10.2.1	Introduction	1042

10.2.2	E-UTRAN RSRP measurements	1042

10.2.3	E-UTRAN RSRQ measurements	1042

10.2.4	E-UTRAN RSTD measurements	1042

10.2.5	E-UTRAN RS-SINR measurements	1043

10.2.6	E-UTRAN RSRP measurements for CA/DC Idle Mode Measurements	1043

10.2.7	E-UTRAN RSRQ measurements for CA/DC Idle Mode Measurements	1043

10.2A	E-UTRAN measurements for RedCap	1044

10.2A.1	Introduction	1044

10.2A.2	E-UTRAN RSRP measurements	1044

10.2A.3	E-UTRAN RSRQ measurements	1044

10.2A.4	E-UTRAN RS-SINR measurements	1045

10.3	UTRAN FDD Measurements	1045

10.3.1	UTRAN FDD CPICH RSCP	1045

10.3.2	UTRAN FDD CPICH Ec/No	1046

10.4	V2X measurements	1046

10.4.1	Introduction	1046

10.4.2	Intra-frequency PSBCH-RSRP accuracy requirements for FR1	1046

10.4.2.1	PSBCH-RSRP Absolute Accuracy	1046

10.4.2.2	PSBCH-RSRP Relative Accuracy	1047

10.4.3	Intra-Frequency SL-RSSI Measurement Accuracy Requirements for FR1	1048

10.4.3.1	Absolute SL-RSSI Accuracy	1048

10.4.4	Intra-Frequency L1 SL-RSRP Measurement Accuracy Requirements for FR1	1048

10.4.4.1	Absolute L1 SL-RSRP Accuracy	1048

10.4.5	Intra-Frequency Discovery Signal Measurement Accuracy Requirements	1049

10.4.5.1	Absolute Discovery Signal Measurement Accuracy	1049

11	Void	1050

12	V2X Requirements	1051

12.1	Introduction	1051

12.2	UE Transmit Timing	1051

12.2.1	Introduction	1051

12.2.2	GNSS as synchronization reference source	1051

12.2.3	NR Cell as synchronization reference source	1052

12.2.4	E-URTAN Cell as synchronization reference source	1052

12.2.5	SyncRef UE as synchronization reference source	1052

12.3	Initiation/Cease of SLSS Transmissions	1053

12.3.1	Introduction	1053

12.3.1.1	Initiation/Cease of SLSS transmissions with NR cell as synchronization reference source	1053

12.3.1.2	Initiation/Cease of SLSS transmissions with EUTRAN cell as synchronization reference source	1054

12.3.1.3	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	1055

12.3.1.4	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	1055

12.4	Selection / Reselection of V2X Synchronization Reference Source	1056

12.5	L1 SL-RSRP measurements	1057

12.5.1	Introduction	1057

12.5.2	SL-RSRP measurements	1057

12.6	Congestion Control measurements	1058

12.7	Interruption	1058

12.7.1	Interruptions to WAN due to V2X Sidelink Communication	1058

12.7.2	V2X Sidelink Communication Dropping due to synchronization source change	1059

12.7.3	Interruptions to WAN due to switching between E-UTRA V2X Sidelink and NR V2X Sidelink	1060

12.7.4	Interruptions to WAN at transitions between active and non-active during SL-DRX	1061

12.7.5	Interruptions to V2X sidelink at transitions between active and non-active during DRX	1061

12.7.6	Interruptions to V2X sidelink due to Active BWP switching Requirement	1062

12.7.7	Interruptions to WAN due to SyncRef UE detection and/or Sensing during SL DRX off duration	1062

12.7.8	Interruptions at NR sidelink discovery configuration	1062

12.8	Reliability of GNSS signal	1063

12.9	Scheduling availability	1063

12.9.1	Scheduling availability of UE switching between E-UTRA sidelink and NR sidelink	1063

12.9.2	Scheduling availability of UE switching between Uu uplink  and V2X sidelink	1063

12.10	Selection / Reselection of relay UE	1064

12.10.1	Introduction	1064

12.10.2	Selection / Reselection of relay UE	1064

13	Measurement Performance Requirements for NR gNB	1064

13.1	UL-RTOA	1064

13.1.1	Report mapping	1064

13.1.1A	Additional Path Report Mapping for UL-RTOA	1066

13.2	gNB Rx-Tx time difference	1068

13.2.1	Report mapping	1068

13.2.1A	 Additional Path Report Mapping for gNB Rx-Tx	1070

13.2.2	Measurement Accuracy Requirements	1072

13.2.2.1	Introduction	1072

13.3	UL SRS RSRP measurement	1073

13.3.1	Report mapping	1073

13.3.2	Measurement accuracy requirements	1074

13.3.2.1	Introduction	1074

13.3.2.2	Requirements	1074

13.4	AoA/ZoA	1075

13.4.1	Report mapping	1075

13.5	Timing advance (TADV)	1076

13.5.1	Report mapping	1076

13.6	UL SRS RSRPP measurement	1077

13.6.1	Report mapping	1077

13.7	gNB Rx-Tx time difference measurements for RTT-based PDC	1079

13.7.1	Report mapping	1079

13.7.2	Measurement Accuracy Requirements	1079

13.7.2.1	Introduction	1079

13.7.2.2	Requirements	1079

Annex A (normative): Test Cases	1082

A.1	Purpose of annex	1082

A.2	Requirement classification for statistical testing	1082

A.2.1	Types of requirements in TS 38.133	1082

A.2.1.1	Time and delay requirements on UE higher layer actions	1082

A.2.1.2	Measurements of power levels, relative powers and time	1082

A.2.1.3	Implementation requirements	1083

A.2.1.4	Physical layer timing requirements	1083

A.2.1.5	Requirements under CCA	1083

A.3	RRM test configurations	1084

A.3.1	Reference measurement channels	1084

A.3.1.1	PDSCH	1084

A.3.1.1.1	FDD	1084

A.3.1.1.2	TDD	1085

A.3.1.2	CORESET for RMSI scheduling	1088

A.3.1.2.1	FDD	1088

A.3.1.2.2	TDD	1089

A.3.1.3	CORESET for RMC scheduling	1092

A.3.1.3.1	FDD	1092

A.3.1.3.2	TDD	1093

A.3.1.4	TDD UL/DL configuration	1096

A.3.1A	Reference measurement channels under CCA	1098

A.3.1A.1	PDSCH	1098

A.3.1A.1.1	TDD	1098

A.3.1A.2	CORESET for RMSI scheduling	1099

A.3.1A.2.1	TDD	1099

A.3.1A.3	CORESET for RMC scheduling	1100

A.3.1A.3.1	TDD	1100

A.3.1A.4	TDD UL/DL configuration	1101

A.3.1A.5	RMC burst transmission model	1101

A.3.2.1	Generic OFDMA Channel Noise Generator (OCNG)	1101

A.3.2.1.1	OCNG pattern 1: Generic OCNG pattern for all unused REs	1102

A.3.2.1.2	OCNG pattern 2: Generic OCNG pattern for all unused REs for 2AoA setup	1102

A.3.2.1.3	OCNG pattern 3: Generic OCNG pattern for unused REs in the same bandwidth as CORESET	1103

A.3.2.1.4	OCNG pattern 4: Generic OCNG pattern for all unused REs outside SSB slot(s)	1103

A.3.2.2	Void	1104

A.3.3	Reference DRX configurations	1104

A.3.3.1	DRX Configuration 1: DRX cycle = 40 ms and TAT = 500 ms	1104

A.3.3.2	DRX Configuration 2: DRX cycle = 640 ms and TAT = 500 ms	1105

A.3.3.3	DRX Configuration 3: DRX cycle = 40 ms and TAT = Infinity	1105

A.3.3.4	DRX Configuration 4: DRX cycle = 160 ms and TAT = Infinity	1105

A.3.3.5	DRX Configuration 5: DRX cycle = 320 ms and TAT = Infinity	1106

A.3.3.6	DRX Configuration 6: DRX cycle = 320 ms and TAT = 500 ms	1106

A.3.3.7	DRX Configuration 7: DRX cycle = 640 ms and TAT = Infinity	1106

A.3.3.8	DRX Configuration 8: DRX cycle = 320 ms and TAT = Infinity	1107

A.3.3.9	DRX Configuration 9: DRX cycle = 40 ms and TAT = 500 ms	1107

A.3.3.10	DRX Configuration 10: DRX cycle = 640 ms and TAT = 500 ms	1107

A.3.3.11	DRX Configuration 11: DRX cycle = 20 ms and TAT = Infinity	1108

A.3.3.12	DRX Configuration 12: DRX cycle = 640 ms and TAT = Infinity	1108

A.3.3.13	DRX Configuration X1: DRX cycle = 80 ms and TAT = Infinity	1108

A.3.4	Test Cases with Different Channel Bandwidths	1109

A.3.4.1	Test Cases with Different E-UTRA Channel Bandwidths	1109

A.3.4.1.1	Introduction	1109

A.3.4.1.2	Principle of testing	1109

A.3.5	Test Cases for Synchronous and Asynchronous DC Operations	1109

A.3.5.1	EN-DC Test Cases for Synchronous and Asynchronous EN-DC Operations	1109

A.3.5.1.1	Introduction	1109

A.3.5.1.2	Principle of Testing	1109

A.3.6	Antenna configurations	1109

A.3.6.1	Antenna configurations for FR1	1109

A.3.6.1.1	Antenna connection for 4 Rx capable UEs	1109

A.3.6.1.1.1	Introduction	1109

A.3.6.1.1.2	Principle of testing	1110

A.3.6.2	Antenna configurations for FR2	1113

A.3.6A	Antenna configurations with unlicensed bands	1113

A.3.6A.1	Antenna configurations for FR1	1113

A.3.6A.1.1	Antenna connection for 4 Rx capable UEs	1113

A.3.6A.1.1.1	Introduction	1113

A.3.6A.1.1.2	Principle of testing	1113

A.3.7	EN-DC test setup	1115

A.3.7.1	Introduction	1115

A.3.7.2	E-UTRAN Serving Cell Parameters	1115

A.3.7.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	1115

A.3.7.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	1116

A.3.7A	NR FR1-FR2 test setup	1117

A.3.7B	EN-DC test setup with unlicensed bands	1118

A.3.7B.1	Introduction	1118

A.3.7B.2	E-UTRAN Serving Cell Parameters	1118

A.3.7B.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) under CCA in FR1	1118

A.3.7C	LTE-FR1/FR2 test setup	1119

A.3.7D	NE-DC test setup	1119

A.3.7D.1	Introduction	1119

A.3.7D.2	E-UTRAN Serving Cell Parameters	1119

A.3.7D.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	1119

A.3.7D.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	1119

A.3.8	PRACH configurations	1120

A.3.8.1	Introduction	1120

A.3.8.2	PRACH configurations in FR1	1120

A.3.8.2.1	FR1 PRACH configuration 1	1120

A.3.8.2.2	FR1 PRACH configuration 2	1120

A.3.8.2.3	FR1 PRACH configuration 3	1121

A.3.8.2.4	FR1 PRACH configuration 4	1122

A.3.8.3	PRACH configurations in FR2	1123

A.3.8.3.1	FR2 PRACH configuration 1	1123

A.3.8.3.2	FR2 PRACH configuration 2	1124

A.3.8.3.3	FR2 PRACH configuration 3	1125

A.3.8.3.4	FR2 PRACH configuration 4	1126

A.3.8A	PRACH configurations under CCA	1127

A.3.8A.1	Introduction	1127

A.3.8A.2	PRACH configurations in FR1	1127

A.3.8A.2.1	FR1 PRACH configuration 1 under CCA	1127

A.3.8A.2.2	FR1 PRACH configuration 2 under CCA	1128

A.3.9	BWP configurations	1129

A.3.9.1	Introduction	1129

A.3.9.2	Downlink BWP configurations	1130

A.3.9.2.1	Initial BWP	1130

A.3.9.2.2	Dedicated BWP	1130

A.3.9.3	Uplink BWP configurations	1131

A.3.9.3.1	Initial BWP	1131

A.3.9.3.2	Dedicated BWP	1131

A.3.9A	BWP configurations for RedCap	1131

A.3.9A.1	Introduction	1131

A.3.9A.2	Downlink BWP configurations	1132

A.3.9A.2.1	Dedicated BWP	1132

A.3.9A.3	Uplink BWP configurations	1132

A.3.9A.3.1	Dedicated BWP	1132

A.3.10	SSB Configurations	1133

A.3.10.1	SSB Configurations for FR1	1133

A.3.10.1.1	SSB pattern 1 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1133

A.3.10.1.5	SSB pattern 5 in FR1: SSB allocation for SSB SCS=15 kHz starting from odd SFN in 10 MHz	1135

A.3.10.1.6	SSB pattern 6 in FR1: SSB allocation for SSB SCS=30 kHz starting from odd SFN in 40 MHz	1135

A.3.10.1.7	SSB pattern 7 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1136

A.3.10.1.8	SSB pattern 8 in FR1: SSB allocation for SSB SCS=30 kHz in 40 MHz	1136

A.3.10.2	SSB Configurations for FR2	1137

A.3.10.2.1	SSB pattern 1 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1137

A.3.10.2.2	SSB pattern 2 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1137

A.3.10.2.3	SSB pattern 3 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1138

A.3.10.2.4	SSB pattern 4 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1138

A.3.10.2.5	SSB pattern 5 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1139

A.3.10.2.6	SSB pattern 6 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1139

A.3.10.2.7	SSB pattern 7 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1140

A.3.10.2.8	SSB pattern 8 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1140

A.3.10.2.9	SSB pattern 9 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1141

A.3.10.2.10	SSB pattern 10 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1141

A.3.10A	SSB Configurations under CCA	1146

A.3.10A.1	SSB Configurations under CCA for FR1	1146

A.3.10A.1.1	SSB pattern 1 under CCA for semi-static channel access: SSB allocation for SSB SCS=30kHz in 40MHz	1146

A.3.10A.1.2	SSB pattern 2 under CCA for dynamic channel access: SSB allocation for SSB SCS=30kHz in 40MHz	1146

A.3.10A.1.3	SSB pattern 3 under CCA for semi-static channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	1147

A.3.10A.1.4	SSB pattern 4 under CCA for dynamic channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	1147

A.3.10B	SSB Configurations for RedCap	1148

A.3.10B.1	SSB Configurations for FR1	1148

A.3.10B.1.1	SSB pattern 1 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1148

A.3.10B.1.2	SSB pattern 2 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1148

A.3.10B.1.3	SSB pattern 3 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz starting from odd SFN in 20 MHz	1149

A.3.10B.1.4	SSB pattern 4 for RedCap in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1149

A.3.10B.1.5	SSB pattern 5 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1150

A.3.10B.1.6	SSB pattern 6 for RedCap in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1150

A.3.10B.1.7	SSB pattern 7 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1151

A.3.10B.2	SSB Configurations for FR2	1151

A.3.10B.2.1	SSB pattern 1 for RedCap in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1151

A.3.10B.2.2	SSB pattern 2 for RedCap in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1152

A.3.10B.2.3	SSB pattern 3 for RedCap in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1152

A.3.10B.2.4	SSB pattern 4 for RedCap in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1153

A.3.10B.2.5	SSB pattern 5 for RedCap in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1153

A.3.11	SMTC Configurations	1154

A.3.11.1	SMTC pattern 1: SMTC period = 20 ms with SMTC duration = 1 ms	1154

A.3.11.2	SMTC pattern 2: SMTC period = 20 ms with SMTC duration = 5 ms	1154

A.3.11.3	SMTC pattern 3: SMTC period = 160 ms with SMTC duration = 1 ms	1154

A.3.11.4	SMTC pattern 4: SMTC period = 20 ms with SMTC duration = 1 ms	1154

A.3.11.5	SMTC pattern 5: SMTC period = 20 ms with SMTC duration = 5 ms	1155

A.3.11.6	SMTC pattern 6: SMTC period = 20 ms with SMTC duration = 5 ms	1155

A.3.11.7	SMTC pattern 7: SMTC period = 20 ms with SMTC duration = 5 ms	1155

A.3.11.8	SMTC pattern 8: SMTC period = 10 ms with SMTC duration = 1 ms	1155

A.3.11A	SMTC Configurations for RedCap	1156

A.3.11A.1	SMTC pattern 1 for RedCap: SMTC period = 40 ms with SMTC duration = 1 ms	1156

A.3.11A.2	SMTC pattern 2 for RedCap: SMTC period = 80 ms with SMTC duration = 1 ms	1156

A.3.11A.3	SMTC pattern 3 for RedCap: SMTC period = 40 ms with SMTC duration = 1 ms	1156

A.3.11A.4	SMTC pattern 4 for RedCap: SMTC period = 80 ms with SMTC duration = 5 ms	1156

A.3.12	Test Cases with Different CC Configurations	1156

A.3.12.1 EN-DC Test Cases with Different EN-DC Configurations	1156

A.3.12.1.1	Introduction	1156

A.3.12.1.2	Principle of testing	1157

A.3.12.2	Carrier Aggregation Test Cases with Different CA Configurations	1157

A.3.12.2.1	Introduction	1157

A.3.12.2.2	Principle of testing	1157

A.3.13	Test Cases in SA and EN-DC Operations	1157

A.3.13.1	Introduction	1157

A.3.13.2	Principle of Testing	1157

A.3.13A	Test Cases involving E-UTRA/FR1 and FR2 carriers	1158

A.3.13A.1	Introduction	1158

A.3.13A.2	Principle of Testing in EN-DC	1158

A.3.13A.3	Principle of Testing in SA	1158

A.3.13A.4	Principle of Testing in E-UTRA	1159

A.3.13B	Test Cases for EN-DC and NE-DC Operations	1160

A.3.13B.1	Active BWP switch Test Cases for EN-DC and NE-DC Operations	1160

A.3.13B.1.1	Introduction	1160

A.3.13B.1.2	Principle of Testing	1160

A.3.13B.2	SFTD accuracy Test Cases for EN-DC and NE-DC Operations	1160

A.3.13B.2.1	Introduction	1160

A.3.13B.2.2	Principle of Testing	1160

A.3.14	CSI-RS configurations	1161

A.3.14.1	FDD	1161

A.3.14.2	TDD	1163

A.3.15	Angle of Arrival (AoA) for FR2 RRM test cases	1169

A.3.15.1	Setup 1: Single AoA in Rx beam peak direction	1169

A.3.15.2	Setup 2: Single AoA in non Rx beam peak direction	1170

A.3.15.2.1	Setup 2a: Single AoA in non Rx beam peak direction without change in direction	1170

A.3.15.2.2	Setup 2b: Single AoA in non Rx beam peak direction with change in direction	1170

A.3.15.3	Setup 3: 2 AoAs	1170

A.3.15.4	Setup 4: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak	1170

A.3.15.4.1	Setup 4a: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak without change in direction	1170

A.3.15.4.2	Setup 4b: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak with change in direction	1171

A.3.16	TCI State Configuration	1171

A.3.16.1	Introduction	1171

A.3.16.2	TCI states	1171

A.3.16A	Unified TCI State Configuration	1171

A.3.16A.1	Introduction	1171

A.3.16A.2	DLorJoint TCI states	1172

A.3.16A.3	UL TCI states	1172

A.3.17	Configurations of CSI-RS for tracking	1173

A.3.17.1	Configuration of CSI-RS for tracking for FR1	1173

A.3.17.1.1	FDD	1173

A.3.17.1.2	TDD	1175

A.3.17.2	Configuration of CSI-RS for tracking for FR2	1177

A.3.17.2.1	TDD	1177

A.3.18	Additional definitions related to OTA testing for FR2 RRM test cases	1178

A.3.18.1	Introduction	1178

A.3.18.2	PRACH Power Measurement	1178

A.3.19	Test applicability for DAPS handover	1178

A.3.19.1	Introduction	1178

A.3.19.2	Principle of testing	1178

A.3.20	MsgA configurations	1179

A.3.20.1	Introduction	1179

A.3.20.2	MsgA configurations in FR1	1179

A.3.20.2.1	FR1 MsgA configuration 1	1179

A.3.20.2.2	FR1 MsgA configuration 2	1180

A.3.20.3	MsgA configurations in FR2	1182

A.3.20.3.1	FR2 MsgA configuration 1	1182

A.3.20.3.2	FR2 MsgA configuration 2	1183

A.3.20A	MsgA configurations under CCA	1185

A.3.20A.1	Introduction	1185

A.3.20A.2	MsgA configurations in FR1	1185

A.3.20A.2.1	FR1 MsgA configuration 1 under CCA	1185

A.3.20A.2.2	FR1 MsgA configuration 2 under CCA	1186

A.3.21	V2X sidelink communication	1189

A.3.21.1	Introduction	1189

A.3.21.2	Reference resource pool configurations for V2X Sidelink Communication	1189

A.3.21.3	Reference measurement channels for V2X Sidelink Communication	1193

A.3.21.4	Reference SL-DRX configurations	1194

A.3.21.4.1	SL-DRX Configuration 1: SL-DRX cycle = 40 ms	1194

A.3.21.4.2	SL-DRX Configuration 2: SL-DRX cycle = 320 ms	1194

A.3.21.4.3	SL-DRX Configuration 3: SL-DRX cycle = 640 ms	1194

A.3.22	CSI-IM configurations	1194

A.3.22.1	FDD	1194

A.3.22.2	TDD	1195

A.3.23	Spatial Relation Configuration	1196

A.3.23.1	Introduction	1196

A.3.23.2	Spatial Relation	1197

A.3.24	SRS configuration	1198

A.3.25	Channel bandwidth (CBW) configurations	1200

A.3.25.1	DL UE specific CBW	1200

A.3.25.2	UL UE specific CBW	1201

A.3.26	CCA model	1201

A.3.26.1	Introduction	1201

A.3.26.2	CCA model for operation on a carrier frequency with CCA in FR1	1201

A.3.26.2.1	DL CCA model	1201

A.3.26.2.2	UL CCA model	1203

A.3.26.3	CCA model for operation on a carrier frequency with CCA in FR2-2	1203

A.3.26.3.1	DL CCA model	1203

A.3.26.3.2	UL CCA model	1204

A.3.27	Test Cases with at Least One Cell on a Carrier Frequency with CCA	1204

A.3.27.1	Introduction	1204

A.3.27.2	 NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	1204

A.3.27.3	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	1205

A.3.27.4	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	1205

A.3.27.5	E-UTRA Standalone Tests with at Least One NR Cell under CCA	1205

A.3.28	Discovery Burst Transmission Window configuration under CCA	1205

A.3.28.1	DBT Window pattern 1: DBT Window period = 20 ms with DBT Window duration = 1 ms	1205

A.3.29	Testing principles for UE capable of only NR bands with shared spectrum access	1205

A.3.29.1	Introduction	1205

A.3.29.2	Principle of testing for UE capable of EN-DC with only NR bands with shared spectrum access	1205

A.3.29.3	Principle of testing for UE capable of SA operation with only NR bands with shared spectrum access	1206

A.3.30	CSI-RS configurations for RRM	1207

A.3.30.1	FDD	1207

A.3.30.2	TDD	1208

A.3.31	PRS Configurations	1210

A.3.31.1.	PRS Configurations for FR1	1210

A.3.31.1.1.	PRS pattern 1 in FR1: SCS=15 KHz	1210

A.3.31.1.2.	PRS pattern 2 in FR1: SCS=30 KHz	1211

A.3.31.2.	PRS Configurations for FR2	1211

A.3.31.2.1.	PRS pattern 1 in FR2: SCS=120 KHz	1211

A.3.32	NR sidelink discovery	1211

A.3.32.1	Introduction	1211

A.3.32.2	Reference resource pool configurations for NR Sidelink Discovery	1211

A.3.32.3	Principle of Testing	1212

A.3.33	PRS Processing Window (PPW) configurations	1212

A.3.34	Testing principles for test cases related to PRS measurements	1212

A.3.34.1	Introduction	1212

A.3.34.2	Test cases in RRC_INACTIVE state	1213

A.3.34.3	Test cases for PRS measurements with gaps in RRC_CONNECTED state	1213

A.3.34.4	Test cases for PRS measurements without gaps in RRC_CONNECTED state	1213

A.3.35	Testing principle for RedCap UE	1214

A.3.35.1	Introduction	1214

A.3.35.2	Principle of testing for FR1	1214

A.3.35.3	Principle of testing for FR2	1214

A.3.36	Testing related to Satellite access	1214

A.3.36.1	Introduction	1214

A.3.36.2	Principle of testing GSO and NGSO scenarios	1214

A.3.36.2	Principle of testing different RRM requirements	1215

A.3.36.3	Principle of testing different ephemeris formats	1215

A.3.36.4	General setup for SIB19	1217

A.3.36.5	Satellite specific parameters configuration	1217

A.3.36.5.1	Satellite specific configuration for serving cell	1217

A.3.36.5.2	Satellite specific configuration for neighbour cell	1218

A.4	EN-DC tests with all NR cells in FR1	1219

A.4.1	Void	1219

A.4.2	Void	1219

A.4.3	RRC_CONNECTED state mobility	1219

A.4.3.1	Void	1219

A.4.3.2	RRC Connection Mobility Control	1219

A.4.3.2.1	Void	1219

A.4.3.2.2	Random Access	1219

A.4.3.2.2.1	4-step RA type contention based random access test in FR1 for PSCell in EN-DC	1219

A.4.3.2.2.2	4-step RA type n on-contention based random access test in FR1 for PSCell in EN-DC	1223

A.4.3.2.2.3	2-step RA type contention based random access test in FR1 for PSCell in EN-DC	1227

A.4.3.2.2.4	2-step RA type n on-contention based random access test in FR1 for PSCell in EN-DC	1231

A.4.3.2.3	Void	1235

A.4.3.3	Handover with PSCell from EN-DC to EN-DC with known target PSCell in FR1	1235

A.4.3.3.1	Test Purpose and Environment	1235

A.4.3.3.2	Test Requirements	1241

A.4.4	Timing	1242

A.4.4.1	UE transmit timing	1242

A.4.4.1.1	NR UE Transmit Timing Test for FR1	1242

A.4.4.1.1.1	Test Purpose and environment	1242

A.4.4.1.1.2	Test requirements	1246

A.4.4.2	UE timer accuracy	1246

A.4.4.3	Timing advance	1246

A.4.4.3.1	EN-DC FR1 timing advance adjustment accuracy	1246

A.4.4.3.1.1	Test Purpose and Environment	1246

A.4.4.3.1.2	Test Parameters	1246

A.4.4.3.1.3	Test Requirements	1250

A.4.5	Signaling characteristics	1250

A.4.5.1	Radio link Monitoring	1250

A.4.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	1251

A.4.5.1.1.1	Test Purpose and Environment	1251

A.4.5.1.1.2	Test Requirements	1255

A.4.5.1.2	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	1255

A.4.5.1.2.1	Test Purpose and Environment	1255

A.4.5.1.2.2	Test Requirements	1261

A.4.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	1261

A.4.5.1.3.1	Test Purpose and Environment	1261

A.4.5.1.3.2	Test Requirements	1266

A.4.5.1.4	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	1266

A.4.5.1.4.1	Test Purpose and Environment	1266

A.4.5.1.4.2	Test Requirements	1272

A.4.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	1272

A.4.5.1.5.1	Test Purpose and Environment	1272

A.4.5.1.5.2	Test Requirements	1277

A.4.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	1277

A.4.5.1.6.1	Test Purpose and Environment	1277

A.4.5.1.6.2	Test Requirements	1283

A.4.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	1283

A.4.5.1.7.1	Test Purpose and Environment	1283

A.4.5.1.7.2	Test Requirements	1288

A.4.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	1289

A.4.5.1.8.1	Test Purpose and Environment	1289

A.4.5.1.8.2	Test Requirements	1294

A.4.5.1.9	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion	1294

A.4.5.1.9.1	Test Purpose and Environment	1294

A.4.5.2	Interruption	1300

A.4.5.2.1	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	1300

A.4.5.2.1.1	Test Purpose and Environment	1300

A.4.5.2.1.2	Test Requirements	1304

A.4.5.2.2	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	1304

A.4.5.2.2.1	Test Purpose and Environment	1304

A.4.5.2.2.2	Test Requirements	1308

A.4.5.2.3	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	1308

A.4.5.2.3.1	Test Purpose and Environment	1308

A.4.5.2.3.2	Test Requirements	1313

A.4.5.2.4	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	1314

A.4.5.2.4.1	Test Purpose and Environment	1314

A.4.5.2.4.2	Test Requirements	1318

A.4.5.2.5	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	1319

A.4.5.2.5.1	Test Purpose and Environment	1319

A.4.5.2.5.2	Test Requirements	1323

A.4.5.2.6	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	1323

A.4.5.2.6.1	Test Purpose and Environment	1323

A.4.5.2.6.2	Test Requirements	1327

A.4.5.2.7	Void	1327

A.4.5.2.8	E-UTRAN - NR FR1 interruptions at NR SRS carrier based switching in asynchronous EN-DC	1327

A.4.5.2.8.1	Test Purpose and Environment	1327

A.4.5.2.8.2	Test Requirements	1330

A.4.5.2.9	E-UTRAN – NR interruptions at E-UTRA SRS carrier based switching	1331

A.4.5.2.9.1	Test Purpose and Environment	1331

A.4.5.2.9.2	Test Requirements	1334

A.4.5.2.10	E-UTRAN – NR FR1 interruptions due to RRM and RLM/BFD measurements on deactivated NR PSCell	1335

A.4.5.2.10.1	Test Purpose and Environment	1335

A.4.5.2.10.2	Test Requirements	1338

A.4.5.2.11	E-UTRAN - NR FR1 interruptions at NR SRS antenna port switching with 1 SRS symbol in a slot in synchronous EN-DC	1338

A.4.5.2.11.1	Test Purpose and Environment	1338

A.4.5.2.11.2	Test Requirements	1343

A.4.5.2.12	E-UTRAN - NR FR1 interruptions at NR SRS antenna port switching in asynchronous EN-DC	1344

A.4.5.2.12.1	Test Purpose and Environment	1344

A.4.5.3	SCell Activation and Deactivation Delay	1351

A.4.5.3.1	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	1351

A.4.5.3.1.1	Test Purpose and Environment	1351

A.4.5.3.1.2	Test Requirements	1357

A.4.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 640ms SCell measurement cycle	1358

A.4.5.3.2.1	Test Purpose and Environment	1358

A.4.5.3.2.2	Test Requirements	1358

A.4.5.3.3	SCell Activation and deactivation of unknown SCell in FR1	1358

A.4.5.3.3.1	Test Purpose and Environment	1358

A.4.5.3.3.2	Test Requirements	1359

A.4.5.3.4	SCell Activation and deactivation of multiple unknown SCells in FR1 with single activation/deactivation command	1360

A.4.5.3.4.1	Test Purpose and Environment	1360

A.4.5.3.3.2	Test Requirements	1362

A.4.5.3.5	Direct SCell activation at SCell addition of known SCell in FR1	1362

A.4.5.3.5.1	Test Purpose and Environment	1362

A.4.5.3.5.2	Test Requirements	1370

A.4.5.3.6	Fast SCell Activation of known SCell in FR1 for 160ms SCell measurement cycle	1371

A.4.5.3.6.1	Test Purpose and Environment	1371

A.4.5.3.6.2	Test Requirements	1375

A.4.5.3.7	Fast SCell Activation of known SCell in FR1 for 640 ms SCell measurement cycle	1376

A.4.5.3.7.1	Test Purpose and Environment	1376

A.4.5.3.7.2	Test Requirements	1376

A.4.5.4	UE UL carrier RRC reconfiguration Delay	1377

A.4.5.4.1	UE UL carrier RRC reconfiguration Delay	1377

A.4.5.4.1.1 Test Purpose and Environment	1377

A.4.5.4.1.2	Test Requirements	1387

A.4.5.5	Beam Failure Detection and Link recovery procedures	1387

A.4.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	1387

A.4.5.5.1.1	Test Purpose and Environment	1387

A.4.5.5.1.2	Test Requirements	1393

A.4.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	1394

A.4.5.5.2.1	Test Purpose and Environment	1394

A.4.5.5.2.2	Test Requirements	1399

A.4.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	1400

A.4.5.5.3.1	Test Purpose and Environment	1400

A.4.5.5.3.2	Test Requirements	1405

A.4.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in DRX mode	1406

A.4.5.5.4.1	Test Purpose and Environment	1406

A.4.5.5.4.2	Test Requirements	1411

A.4.5.5.5	EN-DC Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	1412

A.4.5.5.5.1	Test Purpose and Environment	1412

A.4.5.5.5.2	Test Requirements	1417

A.4.5.5.6	EN-DC Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode	1418

A.4.5.5.6.1	Test Purpose and Environment	1418

A.4.5.5.6.2	Test Requirements	1424

A.4.5.5.7	EN-DC TRP specific Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	1425

A.4.5.5.7.1	Test Purpose and Environment	1425

A.4.5.5.7.2	Test Requirements	1430

A.4.5.5.8	EN-DC TRP specific Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	1431

A.4.5.5.8.1	Test Purpose and Environment	1431

A.4.5.5.8.2	Test Requirements	1437

A.4.5.6	Active BWP switch	1438

A.4.5.6.1	DCI-based and Timer-based Active BWP Switch	1438

A.4.5.6.1.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	1438

A.4.5.6.1.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	1443

A.4.5.6.2	RRC-based Active BWP Switch	1449

A.4.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1453

A.4.5.6.3.1	Simultaneous E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in EN-DC on multiple CCs	1453

A.4.5.6.3.1.1	Test Purpose and Environment	1453

A.4.5.6.4	Simultaneous RRC-based Active BWP Switch on multiple CCs	1459

A.4.5.6.4.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC on multiple CCs	1459

A.4.5.6.4.1.1	Test Purpose and Environment	1459

A.4.5.6.4.1.2	Test Requirements	1466

A.4.5.6.4.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR1 SCells inside active time	1466

A.4.5.6.4.2.1	Test Purpose and Environment	1466

A.4.5.6.4.2.2	Test Requirements	1477

A.4.5.6.5	SCell dormancy switch	1478

A.4.5.6.5.1	E-UTRAN – NR FR1 PSCell SCell dormancy switch of single FR1 SCell outside active time	1478

A.4.5.6.5.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR1 SCells inside active time	1483

A.4.5.6.5.2.1	Test Purpose and Environment	1483

A.4.5.6.5.2.2	Test Requirements	1489

A.4.5.7	PSCell addition and release delay	1489

A.4.5.7.1	Addition and Release Delay of known NR PSCell	1489

A.4.5.7.1.1	Test purpose and environment	1489

A.4.5.7.1.2	Test Requirements	1494

A.4.5.8	DL Interruptions at switching between two uplink carriers	1495

A.4.5.8.1	Test Purpose and Environment	1495

A.4.5.8.2	Test Requirements	1498

A.4.5.9	UE specific CBW change	1499

A.4.5.9.1	UE specific CBW change on FR1 NR PSCell with non-DRX in synchronous EN- DC	1499

A.4.5.9.1.1	Test Purpose and Environment	1499

A.4.5.9.1.2	Test Requirements	1503

A.4.5.10	PSCell activation and deactivation delay	1503

A.4.5.10.1	PSCell activation and deactivation delay	1503

A.4.5.10.1.1	Test purpose and environment	1503

A.4.5.10.1.2	Test Requirements	1507

A.4.5.11	Conditional PSCell addition and release delay (FR1 EN-DC)	1508

A.4.5.11.1	Conditional PSCell Addition and Release Delay	1508

A.4.5.11.1.1	Test purpose and environment	1508

A.4.5.11.1.2	Test Parameters	1508

A.4.5.11.1.3	Test Requirements	1512

A.4.6	Measurement procedure	1512

A.4.6.1	Intra-frequency Measurements	1512

A.4.6.1.1	EN-DC event triggered reporting tests without gap under non-DRX	1512

A.4.6.1.1.1	Test purpose and Environment	1512

A.4.6.1.1.2	Test parameters	1512

A.4.6.1.1.3	Test Requirements	1516

A.4.6.1.2	EN-DC event triggered reporting tests without gap under DRX	1516

A.4.6.1.2.1	Test purpose and Environment	1516

A.4.6.1.2.2	Test parameters	1516

A.4.6.1.2.2	Test Requirements	1520

A.4.6.1.3	EN-DC event triggered reporting tests with per-UE gaps under non-DRX	1520

A.4.6.1.3.1	Test purpose and Environment	1520

A.4.6.1.3.2	Test parameters	1520

A.4.6.1.3.3	Test Requirements	1524

A.4.6.1.4	EN-DC event triggered reporting tests with per-UE gaps under DRX	1524

A.4.6.1.4.1	Test purpose and Environment	1524

A.4.6.1.4.2	Test parameters	1524

A.4.6.1.4.3	Test Requirements	1528

A.4.6.1.5	EN-DC event triggered reporting tests without gap under non-DRX with SSB index reading	1528

A.4.6.1.5.1	Test purpose and Environment	1528

A.4.6.1.5.2	Test parameters	1528

A.4.6.1.5.3	Test Requirements	1530

A.4.6.1.6	EN-DC event triggered reporting tests with SSB index reading with per-UE gaps	1531

A.4.6.1.6.1	Test purpose and Environment	1531

A.4.6.1.6.2	Test parameters	1531

A.4.6.1.6.3	Test Requirements	1533

A.4.6.1.7	EN-DC event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	1534

A.4.6.1.7.1	Test purpose and Environment	1534

A.4.6.1.7.2	Test parameters	1534

A.4.6.1.7.3	Test Requirements	1538

A.4.6.1.8	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used for UE configured with highSpeedMeasCA-Scell-r17	1538

A.4.6.1.8.1	Test Purpose and Environment	1538

A.4.6.1.8.2	Test Requirements	1543

A.4.6.2	Inter-frequency Measurements	1544

A.4.6.2.1	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	1544

A.4.6.2.1.1	Test Purpose and Environment	1544

A.4.6.2.1.2	Test Requirements	1548

A.4.6.2.2	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	1548

A.4.6.2.2.1	Test Purpose and Environment	1548

A.4.6.2.2.2	Test Requirements	1552

A.4.6.2.3	Void	1552

A.4.6.2.4	Void	1552

A.4.6.2.5	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	1552

A.4.6.2.5.1	Test Purpose and Environment	1552

A.4.6.2.5.2	Test Requirements	1557

A.4.6.2.6	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	1557

A.4.6.2.6.1	Test Purpose and Environment	1557

A.4.6.2.6.2	Test Requirements	1562

A.4.6.2.7	Void	1562

A.4.6.2.8	Void	1562

A.4.6.2.9	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used for UE configured with highSpeedMeasInterFreq-r17	1563

A.4.6.2.9.1	Test Purpose and Environment	1563

A.4.6.2.9.2	Test Requirements	1566

A.4.6.3	Void	1566

A.4.6.4	L1-RSRP measurement for beam reporting	1566

A.4.6.4.1	SSB based L1-RSRP measurement when DRX is not used	1566

A.4.6.4.1.1	Test Purpose and Environment	1566

A.4.6.4.1.2	Test parameters	1567

A.4.6.4.1.3	Test Requirements	1570

A.4.6.4.2	SSB based L1-RSRP measurement when DRX is used	1570

A.4.6.4.2.1	Test Purpose and Environment	1570

A.4.6.4.2.2	Test parameters	1571

A.4.6.4.2.3	Test Requirements	1574

A.4.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	1574

A.4.6.4.3.1	Test Purpose and Environment	1574

A.4.6.4.3.2	Test parameters	1575

A.4.6.4.3.3	Test Requirements	1578

A.4.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	1578

A.4.6.4.4.1	Test Purpose and Environment	1578

A.4.6.4.4.2	Test parameters	1579

A.4.6.4.4.3	Test Requirements	1581

A.4.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	1581

A.4.6.4.5.1	Test Purpose and Environment	1581

A.4.6.4.5.2	Test parameters	1582

A.4.6.4.5.3	Test Requirements	1585

A.4.6.5	CLI measurements	1585

A.4.6.5.1	SRS-RSRP measurement with non-DRX	1585

A.4.6.5.1.1	Test Purpose and Environment	1585

A.4.6.5.1.2	Test Parameters	1586

A.4.6.5.1.3	Test Requirements	1589

A.4.6.5.2	CLI-RSSI measurement with non-DRX	1589

A.4.6.5.2.1	Test Purpose and Environment	1589

A.4.6.5.2.2	Test Parameters	1590

A.4.6.5.2.3	Test Requirements	1591

A.4.6.6.1.2	Test Requirements	1596

A.4.6.7	L1-SINR measurement for beam reporting	1596

A.4.6.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	1599

A.4.6.7.2.1	Test Purpose and Environment	1599

A.4.6.7.2.2	Test parameters	1599

A.4.6.7.2.3	Test Requirements	1602

A.4.6.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	1602

A.4.6.7.3.1	Test Purpose and Environment	1602

A.4.6.7.3.2	Test parameters	1602

A.4.6.7.3.3	Test Requirements	1604

A.4.6.8	CSI-RS based intra-frequency Measurement	1605

A.4.6.8.1	EN-DC event triggered reporting tests without gap under DRX	1605

A.4.6.8.1.1	Test purpose and Environment	1605

A.4.6.8.1.2	Test Requirements	1609

A.4.6.9	CSI-RS based inter-frequency Measurement	1609

A.4.6.9.1	EN-DC event triggered reporting tests for FR1 cell when non-DRX is used	1609

A.4.6.9.1.1	Test Purpose and Environment	1609

A.4.6.9.1.2	Test Requirements	1613

A.4.7	Measurement Performance requirements	1614

A.4.7.1	SS-RSRP	1614

A.4.7.1.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1614

A.4.7.1.1.1	Test Purpose and Environment	1614

A.4.7.1.1.2	Test parameters	1614

A.4.7.1.1.3	Test Requirements	1618

A.4.7.1.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1618

A.4.7.1.2.1	Test Purpose and Environment	1618

A.4.7.1.2.2	Test parameters	1619

A.4.7.1.2.3	Test Requirements	1623

A.4.7.1.3	Void	1623

A.4.7.2	SS-RSRQ	1623

A.4.7.2.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1623

A.4.7.2.1.1	Test Purpose and Environment	1623

A.4.7.2.1.2	Test Parameters	1624

A.4.7.2.1.3	Test Requirements	1628

A.4.7.2.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1628

A.4.7.2.2.1	Test Purpose and Environment	1628

A.4.7.2.2.2	Test Parameters	1628

A.4.7.2.2.3	Test Requirements	1633

A.4.7.3	SS-SINR	1633

A.4.7.3.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1633

A.4.7.3.1.1	Test Purpose and Environment	1633

A.4.7.3.1.2	Test Parameters	1633

A.4.7.3.1.3	Test Requirements	1636

A.4.7.3.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1636

A.4.7.3.2.1	Test Purpose and Environment	1636

A.4.7.3.2.2	Test Parameters	1636

A.4.7.3.2.3	Test Requirements	1641

A.4.7.4	L1-RSRP measurement for beam reporting	1641

A.4.7.4.1	SSB based L1-RSRP measurement	1641

A.4.7.4.1.1	Test Purpose and Environment	1641

A.4.7.4.1.2	Test parameters	1642

A.4.7.4.1.3	Test Requirements	1645

A.4.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	1645

A.4.7.4.2.1	Test Purpose and Environment	1645

A.4.7.4.2.2	Test parameters	1646

A.4.7.4.2.3	Test Requirements	1649

A.4.7.5	SFTD accuracy	1649

A.4.7.5.1	SFTD accuracy	1649

A.4.7.5.1.1	Test Purpose and Environment	1649

A.4.7.5.1.2	Test Parameters	1649

A.4.7.5.1.3	Test Requirements	1654

A.4.7.5.2	Void	1654

A.4.7.5.3	Void	1654

A.4.7.6	CLI measurements	1654

A.4.7.6.1	EN-DC SRS-RSRP measurement accuracy with FR1 serving cell	1654

A.4.7.6.1.1	Test Purpose and Environment	1654

A.4.7.6.1.2	Test parameters	1655

A.4.7.6.1.3	Test Requirements	1660

A.4.7.6.2	EN-DC CLI-RSSI measurement accuracy with FR1 serving cell	1660

A.4.7.6.2.1	Test Purpose and Environment	1660

A.4.7.6.2.2	Test parameters	1661

A.4.7.6.2.3	Test Requirements	1664

A.4.7.7	L1-SINR measurement for beam reporting	1664

A.4.7.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR	1668

A.4.7.7.2.1	Test Purpose and Environment	1668

A.4.7.7.2.2	Test parameters	1668

A.4.7.7.2.3	Test Requirements	1672

A.4.7.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	1672

A.4.7.7.3.1	Test Purpose and Environment	1672

A.4.7.7.3.2	Test parameters	1672

A.4.7.7.3.3	Test Requirements	1675

A.4.7.8	CSI-RSRP	1675

A.4.7.8.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1675

A.4.7.8.1.1	Test Purpose and Environment	1675

A.4.7.8.1.2	Test parameters	1676

A.4.7.8.1.3	Test Requirements	1681

A.4.7.8.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1681

A.4.7.8.2.1	Test Purpose and Environment	1681

A.4.7.8.2.2	Test parameters	1681

A.4.7.8.2.3	Test Requirements	1686

A.4.7.9	CSI-RSRQ	1686

A.4.7.9.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1686

A.4.7.9.1.1	Test Purpose and Environment	1686

A.4.7.9.1.2	Test Parameters	1686

A.4.7.9.1.3	Test Requirements	1691

A.4.7.9.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1691

A.4.7.9.2.1	Test Purpose and Environment	1691

A.4.7.9.2.2	Test Parameters	1691

A.4.7.9.2.3	Test Requirements	1696

A.4.7.10	CSI-SINR	1696

A.4.7.10.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1696

A.4.7.10.1.1	Test Purpose and Environment	1696

A.4.7.10.1.2	Test Parameters	1697

A.4.7.10.1.3	Test Requirements	1701

A.4.7.10.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1701

A.4.7.10.2.1	Test Purpose and Environment	1701

A.4.7.10.2.2	Test Parameters	1702

A.4.7.10.2.3	Test Requirements	1706

A.4.8	Void	1707

A.4A	NE-DC test with all NR cells in FR1	1707

A.4A.1	Signaling characteristics	1707

A.4A.1.1	E-UTRAN PSCell addition	1707

A.4A.1.1.1	Test purpose and environment	1707

A.4A.1.1.2	Test Requirements	1711

A.4A.1.2	Active BWP switch	1712

A.4A.1.2.1	E-UTRAN PSCell – NR PCell FR1 DCI-based and Timer-based DL active BWP switch in non-DRX in synchronous NE-DC	1712

A.4A.1.2.1.1	Test Purpose and Environment	1712

A.4A.1.2.1.2	Test Requirements	1716

A.4A.1.3	Intra-frequency handover with E-UTRAN PSCell	1717

A.4A.1.3.1	Test purpose and environment	1717

A.4A.1.3.2	Test Requirements	1721

A.4A.1.4	Handover with PSCell from NE-DC to NE-DC with unknown target PSCell	1722

A.4A.1.4.1	Test Purpose and Environment	1722

A.4A.1.4.2	Test Parameters	1722

A.4A.1.4.3	Test Requirements	1727

A.4A.1.4.3.1	Test Requirements for NR HO	1727

A.4A.1.4.3.2	Test Requirements for LTE PSCell Change	1727

A.4A.2	Measurement performance	1728

A.4A.2.1	SFTD accuracy	1728

A.4A.2.1.1	SFTD accuracy	1728

A.4A.2.1.1.1	Test Purpose	1728

A.4A.2.1.1.2	Test Environment	1728

A.4A.2.1.1.3	Test Requirements	1731

A.5	EN-DC tests with one or more NR cells in FR2	1732

A.5.1	Void	1732

A.5.2	Void	1732

A.5.3	RRC_CONNECTED state mobility	1732

A.5.3.1	Void	1732

A.5.3.2	RRC Connection Mobility Control	1732

A.5.3.2.1	Void	1732

A.5.3.2.2	Random Access	1732

A.5.3.2.2.1	4-step RA type c ontention based random access test in FR2 for PSCell/SCell in EN-DC	1732

A.5.3.2.2.2	4-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	1735

A.5.3.2.2.3	2-step RA type contention based random access test in FR2 for PSCell/SCell in EN-DC	1741

A.5.3.2.2.4	2-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	1744

A.5.3.2.3	Void	1748

A.5.3.3	Handover with PSCell with known FR2 target PSCell	1748

A.5.3.3.1	Test purpose and environment	1748

A.5.3.3.2	Test Requirements	1752

A.5.5.3.3	void	1753

A.5.5.3.4	void	1753

A.5.5.3.5	void	1753

A.5.5.3.6	void	1753

A.5.4	Timing	1753

A.5.4.1	UE transmit timing	1753

A.5.4.1.1	NR UE Transmit Timing Test for FR2	1753

A.5.4.1.1.1	Test Purpose and environment	1753

A.5.4.1.1.2	Test requirements	1756

A.5.4.2	UE timer accuracy	1757

A.5.4.3	Timing advance	1757

A.5.4.3.1 EN-DC FR2 timing advance adjustment accuracy	1757

A.5.4.3.1.1 Test Purpose and Environment	1757

A.5.4.3.1.2 Test Parameters	1757

A.5.4.3.1.3	Test Requirements	1761

A.5.5	Signaling characteristics	1761

A.5.5.1	Radio link Monitoring	1761

A.5.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	1761

A.5.5.1.1.1	Test Purpose and Environment	1761

A.5.5.1.1.2	Test Requirements	1765

A.5.5.1.2	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	1766

A.5.5.1.2.1	Test Purpose and Environment	1766

A.5.5.1.2.2	Test Requirements	1769

A.5.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	1770

A.5.5.1.3.1	Test Purpose and Environment	1770

A.5.5.1.3.2	Test Requirements	1774

A.5.5.1.4	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	1774

A.5.5.1.4.1	Test Purpose and Environment	1774

A.5.5.1.4.2	Test Requirements	1778

A.5.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	1778

A.5.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	1782

A.5.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	1786

A.5.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	1791

A.5.5.1.8.2	Test Requirements	1795

A.5.5.1.9	EN-DC Radio Link Monitoring UE Scheduling Restrictions on FR2	1796

A.5.5.1.9.1	Test Purpose and Environment	1796

A.5.5.1.9.2	Test Requirements	1798

A.5.5.1.10	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion	1798

A.5.5.1.10.1	Test Purpose and Environment	1798

A.5.5.1.10.2	Test Requirements	1801

A.5.5.2	Interruption	1801

A.5.5.2.1	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	1801

A.5.5.2.1.1	Test Purpose and Environment	1801

A.5.5.2.1.2	Test Requirements	1804

A.5.5.2.2	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	1804

A.5.5.2.2.1	Test Purpose and Environment	1804

A.5.5.2.2.2	Test Requirements	1807

A.5.5.2.3	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	1807

A.5.5.2.3.1	Test Purpose and Environment	1807

A.5.5.2.3.2	Test Requirements	1811

A.5.5.2.4	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	1811

A.5.5.2.4.1	Test Purpose and Environment	1811

A.5.5.2.4.2	Test Requirements	1814

A.5.5.2.5	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	1815

A.5.5.2.5.1	Test Purpose and Environment	1815

A.5.5.2.5.2	Test Requirements	1818

A.5.5.2.6	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	1818

A.5.5.2.6.1	Test Purpose and Environment	1818

A.5.5.2.6.2	Test Requirements	1821

A.5.5.2.7	E-UTRAN – NR FR2 interruptions at E-UTRA SRS carrier based switching	1821

A.5.5.2.7.1	Test Purpose and Environment	1821

A.5.5.2.7.2	Test Requirements	1825

A.5.5.2.8 E-UTRAN – NR FR2 interruptions at NR SRS carrier based switching	1825

A.5.5.2.8.1 Test Purpose and Environment	1825

A.5.5.2.8.3	Test Requirements	1827

A.5.5.3	SCell Activation and Deactivation Delay	1827

A.5.5.3.1	SCell Activation and deactivation of SCell in FR2 intra-band	1827

A.5.5.3.1.1	Test Purpose and Environment	1827

A.5.5.3.1.2	Test Requirements	1829

A.5.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	1829

A.5.5.3.2.1	Test Purpose and Environment	1829

A.5.5.3.2.2	Test Requirements	1833

A.5.5.3.3	Void	1833

A.5.5.3.4	Void	1833

A.5.5.3.5	SCell Activation and deactivation of SCell in FR2	1833

A.5.5.3.5.1	Test Purpose and Environment	1833

A.5.5.3.5.2	Test Requirements	1836

A.5.5.3.6	Multiple SCell Activation and deactivation of one unknown SCell and one known SCell in FR2	1837

A.5.5.3.6.1	Test Purpose and Environment	1837

A.5.5.3.6.2	Test Requirements	1841

A.5.5.3.7	Direct SCell activation at SCell addition of known SCell in FR2	1842

A.5.5.3.7.1	Test Purpose and Environment	1842

A.5.5.3.7.2	Test Requirements	1845

A.5.5.3.8	Fast SCell Activation of SCell in FR2 intra-band	1845

A.5.5.3.8.1	Test Purpose and Environment	1845

A.5.5.3.8.2	Test Requirements	1849

A.5.5.3.9	PUCCH SCell Activation and deactivation of known SCell in FR2	1849

A.5.5.3.9.1	Test Purpose and Environment	1849

A.5.5.3.9.2	Test Requirements	1853

A.5.5.3.10	PUCCH SCell Activation and deactivation of unknown SCell in FR2	1853

A.5.5.3.10.1	Test Purpose and Environment	1853

A.5.5.3.10.2	Test Requirements	1856

A.5.5.3.11	Multiple SCell activation and deactivation of one known PUCCH SCell and one unknown SCell in FR2	1857

A.5.5.3.11.1	Test Purpose and Environment	1857

A.5.5.3.11.2	Test Requirements	1860

A.5.5.3.12	SCell Activation and deactivation of unknown PUCCH SCell and unknown DL SCell in FR2 in non-DRX	1861

A.5.5.3.12.1	Test Purpose and Environment	1861

A.5.5.3.12.2	Test Requirements	1864

A.5.5.4	Void	1865

A.5.5.5	Beam Failure Detection and Link recovery procedures	1865

A.5.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	1865

A.5.5.5.1.1	Test Purpose and Environment	1865

A.5.5.5.1.2	Test Requirements	1869

A.5.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in DRX mode	1870

A.5.5.5.2.1	Test Purpose and Environment	1870

A.5.5.5.2.2	Test Requirements	1874

A.5.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	1875

A.5.5.5.3.1	Test Purpose and Environment	1875

A.5.5.5.3.2	Test Requirements	1879

A.5.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX mode	1880

A.5.5.5.4.1	Test Purpose and Environment	1880

A.5.5.5.4.2	Test Requirements	1884

A.5.5.5.5	EN-DC scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	1885

A.5.5.5.5.1	Test Purpose and Environment	1885

A.5.5.5.5.2	Test Requirements	1889

A.5.5.5.6	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	1890

A.5.5.5.6.1	Test Purpose and Environment	1890

A.5.5.5.6.2	Test Requirements	1894

A.5.5.5.7	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	1895

A.5.5.5.7.1	Test Purpose and Environment	1895

A.5.5.5.7.2	Test Requirements	1899

A.5.5.5.8	EN-DC TRP specific Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX mode	1900

A.5.5.5.8.1	Test Purpose and Environment	1900

A.5.5.5.8.2	Test Requirements	1905

A.5.5.5.9	Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in DRX mode for UE fulfilling relaxed measurement criterion	1905

A.5.5.5.9.1	Test Purpose and Environment	1905

A.5.5.5.9.2	Test Requirements	1909

A.5.5.6	Active BWP switch	1909

A.5.5.6.1	DCI-based and Timer-based Active BWP Switch	1909

A.5.5.6.1.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	1909

A.5.5.6.1.1.1	Test Purpose and Environment	1909

A.5.5.6.1.1.2	Test Requirements	1913

A.5.5.6.1.2	E-UTRAN – NR PSCell FR2 with FR2 SCell DL active BWP switch in non-DRX in synchronous EN-DC	1913

A.5.5.6.2	RRC-based Active BWP Switch	1918

A.5.5.6.2.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	1918

A.5.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1922

A.5.5.6.3.1	E-UTRAN – NR PSCell FR2 and NR SCell FR2 DL active BWP switch on multiple CCs in synchronous EN-DC	1922

A.5.5.6.4	SCell dormancy switch	1926

A.5.5.6.4.1	E-UTRAN – NR FR2 PSCell SCell dormancy switch of single FR2 SCell inside active time	1926

A.5.5.6.4.1.1	Test Purpose and Environment	1926

A.5.5.6.4.1.2	Test Requirements	1930

A.5.5.6.4.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR2 SCells outside active time	1930

A.5.5.6.4.2.1	Test Purpose and Environment	1930

A.5.5.6.4.2.2	Test Requirements	1937

A.5.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	1937

A.5.5.6.5.1	E-UTRAN – NR PSCell FR2  and NR SCell FR2 DL active BWP switch on multiple CCs with non-DRX in synchronous EN-DC	1937

A.5.5.7	PSCell addition and release delay	1941

A.5.5.7.1	Addition and Release Delay of NR PSCell	1941

A.5.5.7.1.1	Test purpose and environment	1941

A.5.5.7.1.2	Test Requirements	1944

A.5.5.8	Active TCI state switch delay	1945

A.5.5.8.1	MAC-CE based active TCI state switch	1945

A.5.5.8.1.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	1945

A.5.5.8.1.1.1	Test Purpose and Environment	1945

A.5.5.8.1.1.2	Test Requirements	1948

A.5.5.8.2	RRC based active TCI state switch	1948

A.5.5.8.2.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	1948

A.5.5.8.2.1.1	Test Purpose and Environment	1948

A.5.5.8.2.1.2	Test Requirements	1952

A.5.5.9	Uplink spatial relation switch delay	1952

A.5.5.9.1	MAC-CE based uplink spatial relation switch	1952

A.5.5.9.1.1	E-UTRAN – NR PSCell FR2 uplink spatial relation switch for a known spatial relation	1952

A.5.5.9.1.1.1	Test Purpose and Environment	1952

A.5.5.9.1.1.2	Test Requirements	1955

A.5.5.9.2	RRC based spatial relation switch	1955

A.5.5.9.2.1	E-UTRAN – NR PSCell FR2 spatial relation switch associated with a known DL-RS	1955

A.5.5.9.2.1.1	Test Purpose and Environment	1955

A.5.5.9.2.1.2	Test Requirements	1958

A.5.5.10	UE specific CBW change	1958

A.5.5.10.1	UE specific CBW change on FR2 NR PSCell	1958

A.5.5.10.1.1	Test Purpose and Environment	1958

A.5.5.10.1.2	Test Requirements	1961

A.5.5.11	Unified TCI state switch delay	1962

A.5.5.11.1	MAC-CE based active joint TCI state switch	1962

A.5.5.11.1.1	E-UTRAN – NR PSCell FR2 active joint TCI state switch for a known TCI state	1962

A.5.5.11.1.1.1	Test Purpose and Environment	1962

A.5.5.11.1.1.2	Test parameters	1962

A.5.5.11.1.1.3	Test Requirements	1965

A.5.5.11.2	MAC-CE based active uplink TCI state switch	1965

A.5.5.11.2.1	E-UTRAN – NR PSCell FR2 active uplink TCI state switch for a known TCI state	1965

A.5.5.11.2.1.1	Test Purpose and Environment	1965

A.5.5.11.2.1.2	Test parameters	1966

A.5.5.11.2.1.3	Test Requirements	1968

A.5.5.11.3	MAC-CE based active downlink TCI state switch	1968

A.5.5.11.3.1	E-UTRAN – NR PSCell FR2 downlink TCI state switch to cell with additional PCI for a known TCI state	1968

A.5.5.11.3.1.1	Test Purpose and Environment	1968

A.5.5.11.3.1.2	Test Parameters	1969

A.5.5.11.3.1.3	Test Requirements	1973

A.5.5.12	PSCell activation and deactivation delay	1973

A.5.5.12.1	PSCell activation and deactivation delay	1973

A.5.5.12.1.1	Test purpose and environment	1973

A.5.5.12.1.2	Test Requirements	1976

A.5.5.13	Conditional PSCell addition and release delay	1977

A.5.5.13.1	Addition and Release Delay of NR PSCell	1977

A.5.5.13.1.1	Test purpose and environment	1977

A.5.5.13.1.2	Test Requirements	1980

A.5.6	Measurement procedure	1981

A.5.6.1	Intra-frequency Measurements	1981

A.5.6.1.1	EN-DC event triggered reporting test without gap under non-DRX	1981

A.5.6.1.1.1	Test purpose and Environment	1981

A.5.6.1.1.2	Test Requirements	1984

A.5.6.1.2	EN-DC event triggered reporting test without gap under DRX	1984

A.5.6.1.2.1	Test purpose and Environment	1984

A.5.6.1.2.2	Test Requirements	1986

A.5.6.1.3	EN-DC event triggered reporting test with per-UE gaps under non-DRX	1987

A.5.6.1.3.1	Test purpose and Environment	1987

A.5.6.1.3.2	Test Requirements	1991

A.5.6.1.4	EN-DC event triggered reporting test with per-UE gaps under DRX	1991

A.5.6.1.4.1	Test purpose and Environment	1991

A.5.6.1.4.2	Test Requirements	1994

A.5.6.2	Inter-frequency Measurements	1995

A.5.6.2.1 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	1995

A.5.6.2.1.1	Test Purpose and Environment	1995

A.5.6.2.1.2	Test Requirements	1998

A.5.6.2.2 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	1998

A.5.6.2.2.1	Test Purpose and Environment	1998

A.5.6.2.2.2	Test Requirements	2002

A.5.6.2.3 	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	2002

A.5.6.2.3.1	Test Purpose and Environment	2002

A.5.6.2.3.2	Test Requirements	2007

A.5.6.2.4	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	2007

A.5.6.2.4.1	Test Purpose and Environment	2007

A.5.6.2.4.2	Test Requirements	2011

A.5.6.2.5	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	2011

A.5.6.2.5.1	Test Purpose and Environment	2011

A.5.6.2.5.2	Test Requirements	2016

A.5.6.2.6	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	2016

A.5.6.2.6.1	Test Purpose and Environment	2016

A.5.6.2.6.2	Test Requirements	2020

A.5.6.2.7	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	2020

A.5.6.2.7.1	Test Purpose and Environment	2020

A.5.6.2.7.2	Test Requirements	2025

A.5.6.2.8	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	2025

A.5.6.2.8.1	Test Purpose and Environment	2025

A.5.6.2.8.2	Test Requirements	2030

A.5.6.3	L1-RSRP measurement for beam reporting	2031

A.5.6.3.1	SSB based L1-RSRP measurement when DRX is not used	2031

A.5.6.3.1.1	Test Purpose and Environment	2031

A.5.6.3.1.2	Test parameters	2031

A.5.6.3.1.3	Test Requirements	2033

A.5.6.3.2	SSB based L1-RSRP measurement when DRX is used	2033

A.5.6.3.2.1	Test Purpose and Environment	2033

A.5.6.3.2.2	Test parameters	2034

A.5.6.3.2.3	Test Requirements	2036

A.5.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	2036

A.5.6.3.3.1	Test Purpose and Environment	2036

A.5.6.3.3.2	Test parameters	2037

A.5.6.3.3.3	Test Requirements	2039

A.5.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	2039

A.5.6.3.4.1	Test Purpose and Environment	2039

A.5.6.3.4.2	Test parameters	2040

A.5.6.3.4.3	Test Requirements	2042

A.5.6.4	CLI measurements	2043

A.5.6.4.1	SRS-RSRP measurement with DRX	2043

A.5.6.4.1.1	Test Purpose and Environment	2043

A.5.6.4.1.2	Test Parameters	2043

A.5.6.4.1.3	Test Requirements	2045

A.5.6.4.2	CLI-RSSI measurement with DRX	2045

A.5.6.4.2.1	Test Purpose and Environment	2045

A.5.6.4.2.2	Test Parameters	2046

A.5.6.4.2.3	Test Requirements	2048

A.5.6.5	Measurements with autonomous gaps	2048

A.5.6.5.1 	EN-DC inter-frequency CGI identification of NR neighbor cell in FR2	2048

A.5.6.5.1.1	Test Purpose and Environment	2048

A.5.6.5.1.2	Test Requirements	2051

A.5.6.6	L1-SINR measurement for beam reporting	2052

A.5.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	2054

A.5.6.6.2.1	Test Purpose and Environment	2054

A.5.6.6.2.2	Test parameters	2055

A.5.6.6.2.3	Test Requirements	2058

A.5.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	2058

A.5.6.6.3.1	Test Purpose and Environment	2058

A.5.6.6.3.2	Test parameters	2059

A.5.6.6.3.3	Test Requirements	2061

A.5.6.7	CSI-RS based Intra-frequency Measurements	2061

A.5.6.7.1	EN-DC event triggered reporting test without gap under non-DRX	2061

A.5.6.7.1.1	Test purpose and Environment	2061

A.5.6.7.1.2	Test Requirements	2064

A.5.6.8	CSI-RS based Inter-frequency Measurements	2064

A.5.6.8.1 	EN-DC event triggered reporting tests for NR FR2 cell when DRX is used	2064

A.5.6.8.1.1	Test Purpose and Environment	2064

A.5.6.8.1.2	Test Requirements	2069

A.5.7	Measurement Performance requirements	2069

A.5.7.1	SS-RSRP	2069

A.5.7.1.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2069

A.5.7.1.1.1	Test Purpose and Environment	2069

A.5.7.1.1.2	Test parameters	2070

A.5.7.1.1.3	Test Requirements	2072

A.5.7.1.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2073

A.5.7.1.2.1	Test Purpose and Environment	2073

A.5.7.1.2.2	Test parameters	2073

A.5.7.1.2.3	Test Requirements	2077

A.5.7.1.3	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	2078

A.5.7.1.3.1	Test Purpose and Environment	2078

A.5.7.1.3.2	Test parameters	2078

A.5.7.1.3.3	Test Requirements	2081

A.5.7.2	SS-RSRQ	2082

A.5.7.2.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2082

A.5.7.2.1.1	Test Purpose and Environment	2082

A.5.7.2.1.2	Test Parameters	2082

A.5.7.2.1.3	Test Requirements	2085

A.5.7.2.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2085

A.5.7.2.2.1	Test Purpose and Environment	2085

A.5.7.2.2.2	Test Parameters	2085

A.5.7.2.2.3	Test Requirements	2087

A.5.7.3	SS-SINR	2088

A.5.7.3.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2088

A.5.7.3.1.1	Test Purpose and Environment	2088

A.5.7.3.1.2	Test Parameters	2088

A.5.7.3.1.3	Test Requirements	2091

A.5.7.3.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2091

A.5.7.3.2.1	Test Purpose and Environment	2091

A.5.7.3.2.2	Test Parameters	2091

A.5.7.3.2.3	Test Requirements	2093

A.5.7.4	L1-RSRP measurement for beam reporting	2093

A.5.7.4.1	SSB based L1-RSRP measurement	2093

A.5.7.4.1.1	Test Purpose and Environment	2093

A.5.7.4.1.2	Test parameters	2094

A.5.7.4.1.3	Test Requirements	2096

A.5.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	2097

A.5.7.4.2.1	Test Purpose and Environment	2097

A.5.7.4.2.2	Test parameters	2097

A.5.7.4.2.3	Test Requirements	2099

A.5.7.5	CLI measurements	2100

A.5.7.5.1	EN-DC SRS-RSRP measurement accuracy with FR2 serving cell	2100

A.5.7.5.1.1	Test Purpose and Environment	2100

A.5.7.5.1.2	Test parameters	2100

A.5.7.5.1.3	Test Requirements	2103

A.5.7.5.2	EN-DC CLI-RSSI measurement accuracy with FR2 serving cell	2104

A.5.7.5.2.1	Test Purpose and Environment	2104

A.5.7.5.2.2	Test parameters	2104

A.5.7.5.2.3	Test Requirements	2106

A.5.7.6	L1-SINR measurement for beam reporting	2107

A.5.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	2110

A.5.7.6.2.1	Test Purpose and Environment	2110

A.5.7.6.2.2	Test parameters	2110

A.5.7.6.2.3	Test Requirements	2112

A.5.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	2113

A.5.7.6.3.1	Test Purpose and Environment	2113

A.5.7.6.3.2	Test parameters	2113

A.5.7.6.3.3	Test Requirements	2115

A.5.7.7	CSI-RSRP	2116

A.5.7.7.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2116

A.5.7.7.1.1	Test Purpose and Environment	2116

A.5.7.7.1.2	Test parameters	2116

A.5.7.7.1.3	Test Requirements	2118

A.5.7.7.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2119

A.5.7.7.2.1	Test Purpose and Environment	2119

A.5.7.7.2.2	Test parameters	2119

A.5.7.7.2.3	Test Requirements	2123

A.5.7.8	CSI-RSRQ	2124

A.5.7.8.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	2124

A.5.7.8.1.1	Test Purpose and Environment	2124

A.5.7.8.1.2	Test Parameters	2124

A.5.7.8.1.3	Test Requirements	2126

A.5.7.8.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2126

A.5.7.8.2.1	Test Purpose and Environment	2126

A.5.7.8.2.2	Test Parameters	2126

A.5.7.8.2.3	Test Requirements	2128

A.5.7.9	CSI-SINR	2128

A.5.7.9.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2128

A.5.7.9.1.1	Test Purpose and Environment	2128

A.5.7.9.1.2	Test Parameters	2128

A.5.7.9.1.3	Test Requirements	2131

A.5.7.9.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2131

A.5.7.9.2.1	Test Purpose and Environment	2131

A.5.7.9.2.2	Test Parameters	2131

A.5.7.9.2.3	Test Requirements	2134

A.5.8	Void	2134

A.6	NR standalone tests with all NR cells in FR1	2135

A.6.1	SA: RRC_IDLE state mobility	2135

A.6.1.1	Cell re-selection to NR	2135

A.6.1.1.1	Cell reselection to FR1 intra-frequency NR case	2135

A.6.1.1.1.1	Test Purpose and Environment	2135

A.6.1.1.1.2	Test Parameters	2135

A.6.1.1.1.3	Test Requirements	2139

A.6.1.1.2	Cell reselection to FR1 inter-frequency NR case	2139

A.6.1.1.2.1	Test Purpose and Environment	2139

A.6.1.1.2.2	Test Parameters	2139

A.6.1.1.2.3	Test Requirements	2143

A.6.1.1.3	Cell reselection to FR1 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	2143

A.6.1.1.3.1	Test Purpose and Environment	2143

A.6.1.1.3.2	Test Parameters	2143

A.6.1.1.3.3	Test Requirements	2148

A.6.1.1.4	Cell reselection to FR1 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	2148

A.6.1.1.4.1	Test Purpose and Environment	2148

A.6.1.1.4.2	Test Parameters	2148

A.6.1.1.4.3	Test Requirements	2151

A.6.1.1.5	Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	2151

A.6.1.1.5.1	Test Purpose and Environment	2151

A.6.1.1.5.2	Test Parameters	2151

A.6.1.1.5.3	Test Requirements	2156

A.6.1.1.6	Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	2157

A.6.1.1.6.1	Test Purpose and Environment	2157

A.6.1.1.6.2	Test Parameters	2157

A.6.1.1.6.3	Test Requirements	2161

A.6.1.1.7	Cell reselection to FR1 intra-frequency NR case for UE configured with highSpeedMeasFlag-r16	2162

A.6.1.1.7.1	Test Purpose and Environment	2162

A.6.1.1.7.2	Test Parameters	2162

A.6.1.1.7.3	Test Requirements	2165

A.6.1.1.8	Cell reselection to FR1 inter-frequency NR case for UE configured with highSpeedMeasInterFreq-r17	2165

A.6.1.1.8.1	Test Purpose and Environment	2165

A.6.1.1.8.2	Test Parameters	2165

A.6.1.1.8.3	Test Requirements	2169

A.6.1.2	Inter-RAT E-UTRAN cell re-selection	2169

A.6.1.2.1	Cell reselection to higher priority E-UTRAN	2169

A.6.1.2.1.1	Test Purpose and Environment	2169

A.6.1.2.1.2	Test Parameters	2169

A.6.1.2.1.3	Test Requirements	2172

A.6.1.2.2	Cell reselection to lower priority E-UTRAN	2173

A.6.1.2.2.1	Test Purpose and Environment	2173

A.6.1.2.2.2	Test Parameters	2173

A.6.1.2.2.3	Test Requirements	2176

A.6.1.2.3	Cell reselection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion	2177

A.6.1.2.3.1	Test Purpose and Environment	2177

A.6.1.2.3.2	Test Parameters	2177

A.6.1.2.3.3	Test Requirements	2180

A.6.1.2.4	Cell reselection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion	2181

A.6.1.2.4.1	Test Purpose and Environment	2181

A.6.1.2.4.2	Test Parameters	2181

A.6.1.2.4.3	Test Requirements	2184

A.6.1.2.5	Cell reselection to lower priority E-UTRAN cell for UE configured with highSpeedMeasFlag-r16	2185

A.6.1.2.5.1	Test Purpose and Environment	2185

A.6.1.2.5.2	Test Parameters	2185

A.6.1.2.5.3	Test Requirements	2189

A.6.1.1.7	Void	2190

A.6.2	SA: RRC_INACTIVE state mobility	2190

A.6.2.1	Configured Grant based Small Data Transmissions (CG-SDT)	2190

A.6.2.1.1	Test purpose and Environment	2190

A.6.2.1.2	Test Parameters	2191

A.6.2.1.3	Test requirements	2193

A.6.3	RRC_CONNECTED state mobility	2193

A.6.3.1	Handover	2193

A.6.3.1.1	Intra-frequency handover from FR1 to FR1; known target cell	2193

A.6.3.1.1.1	Test Purpose and Environment	2193

A.6.3.1.1.2	Test Parameters	2193

A.6.3.1.1.3 Test Requirements	2197

A.6.3.1.2	Intra-frequency handover from FR1 to FR1; unknown target cell	2197

A.6.3.1.2.1	Test Purpose and Environment	2197

A.6.3.1.2.2	Test Parameters	2197

A.6.3.1.2.3	Test Requirements	2201

A.6.3.1.3	Inter-frequency handover from FR1 to FR1; unknown target cell	2201

A.6.3.1.3.1	Test Purpose and Environment	2201

A.6.3.1.3.2	Test Parameters	2201

A.6.3.1.3.3	Test Requirements	2205

A.6.3.1.4	SA NR - E-UTRAN handover	2205

A.6.3.1.4.1	Test Purpose and Environment	2205

A.6.3.1.4.2	Test Requirements	2211

A.6.3.1.5	SA NR - E-UTRAN handover with unknown target cell	2211

A.6.3.1.5.1	Test Purpose and Environment	2211

A.6.3.1.5.2	Test Requirements	2217

A.6.3.1.6	 SA NR - UTRAN FDD handover	2217

A.6.3.1.6.1	Test Purpose and Environment	2217

A.6.3.1.6.2	Test Requirements	2221

A.6.3.1.7	Intra-frequency synchronous DAPS handover in FR1	2221

A.6.3.1.7.1	Test Purpose and Environment	2221

A.6.3.1.7.2	Test Parameters	2221

A.6.3.1.7.3	Test Requirements	2225

A.6.3.1.8	Intra-frequency asynchronous DAPS handover in FR1	2226

A.6.3.1.8.1	Test Purpose and Environment	2226

A.6.3.1.8.2	Test Parameters	2226

A.6.3.1.8.3	Test Requirements	2230

A.6.3.1.9	Intra-band inter-frequency synchronous DAPS handover test in SA for FR1	2231

A.6.3.1.9.1	Test Purpose and Environment	2231

A.6.3.1.9.2	Test Parameters	2231

A.6.3.1.9.3	Test Requirements	2235

A.6.3.1.10	Intra-band inter-frequency asynchronous DAPS handover test in SA for FR1	2235

A.6.3.1.10.1	Test Purpose and Environment	2235

A.6.3.1.10.2	Test Parameters	2235

A.6.3.1.10.3	Test Requirements	2238

A.6.3.1.11	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR1	2238

A.6.3.1.11.1	Test Purpose and Environment	2238

A.6.3.1.11.2	Test Parameters	2238

A.6.3.1.11.3 Test Requirements	2245

A.6.3.1.12	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR1	2245

A.6.3.1.12.1	Test Purpose and Environment	2245

A.6.3.1.12.2	Test Parameters	2245

A.6.3.1.12.3 Test Requirements	2253

A.6.3.1.13	SA NR - E-UTRAN with NR PSCell addition in FR1	2253

A.6.3.1.13.1	Test Purpose and Environment	2253

A.6.3.1.13.2	Test Requirements	2262

A.6.3.1.14	SA NR - E-UTRAN handover with NR FR1 PScell addition	2262

A.6.3.1.14.1	Test Purpose and Environment	2262

A.6.3.1.14.2	Test Requirements	2271

A.6.3.2	RRC Connection Mobility Control	2272

A.6.3.2.1	SA: RRC Re-establishment	2272

A.6.3.2.1.1	Intra-frequency RRC Re-establishment in FR1	2272

A.6.3.2.1.2	Inter-frequency RRC Re-establishment in FR1	2276

A.6.3.2.1.3	Intra-frequency RRC Re-establishment in FR1 without serving cell timing	2280

A.6.3.2.2	Random Access	2283

A.6.3.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone	2283

A.6.3.2.2.2	4-step RA type non-contention based random access test in FR1 for NR standalone	2287

A.6.3.2.2.3	2-step RA type contention based random access test in FR1 for NR standalone	2292

A.6.3.2.2.4	2-step RA type non-contention based test in FR1 for NR standalone	2297

A.6.3.2.3	SA: RRC Connection Release with Redirection	2301

A.6.3.2.3.1	Redirection from NR in FR1 to NR in FR1	2301

A.6.3.2.3.2	Redirection from NR in FR1 to E-UTRAN	2305

A.6.3.3 Conditional handover	2312

A.6.3.3.1	Intra-frequency conditional handover from FR1 to FR1	2312

A.6.3.3.1.1	Test Purpose and Environment	2312

A.6.3.3.1.2	Test Parameters	2312

A.6.3.3.1.3 Test Requirements	2316

A.6.3.3.2	Inter-frequency conditional handover from FR1 to FR1	2316

A.6.3.3.2.1	Test Purpose and Environment	2316

A.6.3.3.2.2	Test Parameters	2316

A.6.3.3.2.3 Test Requirements	2320

A.6.4	Timing	2320

A.6.4.1	UE transmit timing	2320

A.6.4.1.1	NR UE Transmit Timing Test for FR1	2320

A.6.4.1.1.1	Test Purpose and environment	2320

A.6.4.1.1.2	Test requirements	2324

A.6.4.2	UE timer accuracy	2324

A.6.4.3	Timing advance	2324

A.6.4.3.1	SA FR1 timing advance adjustment accuracy	2324

A.6.4.3.1.1	Test Purpose and Environment	2324

A.6.4.3.1.2	Test Parameters	2324

A.6.4.3.1.3	Test Requirements	2328

A.6.5	Signalling characteristics	2328

A.6.5.1	Radio link Monitoring	2328

A.6.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	2329

A.6.5.1.1.1	Test Purpose and Environment	2329

A.6.5.1.1.2	Test Requirements	2334

A.6.5.1.2	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	2334

A.6.5.1.2.1	Test Purpose and Environment	2334

A.6.5.1.2.2	Test Requirements	2340

A.6.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	2340

A.6.5.1.3.1	Test Purpose and Environment	2340

A.6.5.1.3.2	Test Requirements	2346

A.6.5.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	2346

A.6.5.1.4.1	Test Purpose and Environment	2346

A.6.5.1.4.2	Test Requirements	2352

A.6.5.1.5	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	2352

A.6.5.1.5.1	Test Purpose and Environment	2352

A.6.5.1.5.2	Test Requirements	2358

A.6.5.1.6	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	2358

A.6.5.1.6.1	Test Purpose and Environment	2358

A.6.5.1.6.2	Test Requirements	2363

A.6.5.1.7	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	2363

A.6.5.1.7.1	Test Purpose and Environment	2363

A.6.5.1.7.2	Test Requirements	2368

A.6.5.1.8	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	2368

A.6.5.1.8.1	Test Purpose and Environment	2368

A.6.5.1.8.2	Test Requirements	2373

A.6.5.1.9	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM for UE fulfilling relaxed measurement criterion	2373

A.6.5.1.9.1	Test Purpose and Environment	2373

A.6.5.1.9.2	Test Requirements	2379

A.6.5.2	Interruption	2379

A.6.5.2.1	Interruptions during measurements on deactivated NR SCC in FR1	2379

A.6.5.2.1.2	Test Requirements	2383

A.6.5.2.2	SA interruptions at NR SRS carrier based switching	2384

A.6.5.2.2.1	Test Purpose and Environment	2384

A.6.5.2.2.2	Test Parameters	2384

A.6.5.2.2.3	Test Requirements	2386

A.6.5.2.3	SA interruptions at NR SRS antenna port switching with 1 SRS symbol in a slot in NR-CA	2387

A.6.5.2.3.1	Test Purpose and Environment	2387

A.6.5.2.3.2	Test Parameters	2387

A.6.5.2.3.3	Test Requirements	2389

A.6.5.2.4	SA interruptions at NR SRS antenna port switching	2390

A.6.5.2.4.1	Test Purpose and Environment	2390

A.6.5.2.4.2	Test Parameters	2390

A.6.5.2.4.3	Test Requirements	2392

A.6.5.3	SCell Activation and Deactivation Delay	2393

A.6.5.3.1	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160ms SCell measurement cycle	2393

A.6.5.3.1.1	Test Purpose and Environment	2393

A.6.5.3.1.2	Test Requirements	2398

A.6.5.3.2	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle	2399

A.6.5.3.2.1	Test Purpose and Environment	2399

A.6.5.3.2.2	Test Requirements	2399

A.6.5.3.3	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX	2399

A.6.5.3.3.1	Test Purpose and Environment	2399

A.6.5.3.3.2	Test Requirements	2400

A.6.5.3.4	Direct SCell activation at SCell addition of known SCell in FR1	2400

A.6.5.3.4.1	Test Purpose and Environment	2400

A.6.5.3.4.2	Test Requirements	2407

A.6.5.3.5	Direct SCell activation at handover with known SCell in FR1	2407

A.6.5.3.5.1	Test Purpose and Environment	2407

A.6.5.3.5.2	Test Requirements	2412

A.6.5.3.6	PUCCH SCell Activation and deactivation of known SCell in FR1	2412

A.6.5.3.6.1	Test Purpose and Environment	2412

A.6.5.3.6.2	Test Requirements	2416

A.6.5.3.7	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX	2416

A.6.5.3.7.1	Test Purpose and Environment	2416

A.6.5.3.7.2	Test Requirements	2420

A.6.5.3.8	SCell Activation and Deactivation of one FR1 known PUCCH SCell and one FR1 unknown SCell with single activation/deactivation command	2421

A.6.5.3.8.1	Test Purpose and Environment	2421

A.6.5.3.8.2	Test Requirements	2425

A.6.5.3.9	SCell Activation and deactivation of unknown PUCCH SCell and unknown DL SCell in FR1 in non-DRX	2426

A.6.5.3.9.1	Test Purpose and Environment	2426

A.6.5.3.9.2	Test Requirements	2430

A.6.5.3.10	Fast SCell Activation of known SCell in FR1 in non-DRX for 160ms SCell measurement cycle	2431

A.6.5.3.10.1	Test Purpose and Environment	2431

A.6.5.3.10.2	Test Requirements	2434

A.6.5.3.11	SCell Activation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle	2435

A.6.5.3.11.1	Test Purpose and Environment	2435

A.6.5.3.11.2	Test Requirements	2435

A.6.5.4	UE UL carrier RRC reconfiguration Delay	2435

A.6.5.4.1	UE UL carrier RRC reconfiguration Delay	2435

A.6.5.4.1.1	Test Purpose and Environment	2435

A.6.5.4.1.2	Test Requirements	2445

A.6.5.4.2	Void	2446

A.6.5.5	Beam Failure Detection and Link recovery procedures	2446

A.6.5.5.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	2446

A.6.5.5.1.1	Test Purpose and Environment	2446

A.6.5.5.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	2452

A.6.5.5.2.1	Test Purpose and Environment	2452

A.6.5.5.2.2	Test Requirements	2458

A.6.5.5.3	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	2459

A.6.5.5.3.1	Test Purpose and Environment	2459

A.6.5.5.3.2	Test Requirements	2464

A.6.5.5.4	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode	2465

A.6.5.5.4.1	Test Purpose and Environment	2465

A.6.5.5.4.2	Test Requirements	2470

A.6.5.5.5	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	2471

A.6.5.5.5.1	Test Purpose and Environment	2471

A.6.5.5.5.2	Test Requirements	2475

A.6.5.5.6	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode	2475

A.6.5.5.6.1	Test Purpose and Environment	2475

A.6.5.5.6.2	Test Requirements	2480

A.6.5.5.7	TRP Specific Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode	2480

A.6.5.5.7.1	Test Purpose and Environment	2480

A.6.5.5.7.2	Test Requirements	2486

A.6.5.6	Active BWP switch	2487

A.6.5.6.1	DCI-based and Timer-based Active BWP Switch	2487

A.6.5.6.1.1	NR FR1- NR FR1 DL active BWP switch of SCell with non-DRX in SA	2487

A.6.5.6.1.2	NR FR1 DL active BWP switch with non-DRX in SA	2493

A.6.5.6.2	RRC-based Active BWP Switch	2498

A.6.5.6.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	2498

A.6.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	2502

A.6.5.6.3.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	2502

A.6.5.6.4	SCell dormancy switch	2509

A.6.5.6.4.1	NR FR1 PCell SCell dormancy switch of single FR1 SCell outside active time	2509

A.6.5.6.4.1.1	Test Purpose and Environment	2509

A.6.5.6.4.1.2	Test Requirements	2515

A.6.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR1 SCells inside active time	2515

A.6.5.6.4.2.1	 Test Purpose and Environment	2515

A.6.5.6.4.2.2	Test Requirements	2522

A.6.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	2522

A.6.5.6.5.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	2522

A.6.5.7	DL interruptions at switching between two uplink carriers	2527

A.6.5.7.1	DL interruptions at switching between two uplink carriers in FDD-TDD CA	2527

A.6.5.7.1.1	Test Purpose and Environment	2527

A.6.5.7.1.2	Test Requirements	2531

A.6.5.7.2	DL interruptions at switching between two uplink carriers in TDD-TDD CA	2531

A.6.5.7.2.1	Test Purpose and Environment	2531

A.6.5.7.2.2	Test Requirements	2535

A.6.5.7A	DL interruptions at switching between two uplink carriers with two transmit antenna connectors	2535

A.6.5.7A.1	DL interruptions at switching between two uplink carriers in FDD-TDD CA	2535

A.6.5.7A.1.1	Test Purpose and Environment	2535

A.6.5.7A.1.2	Test Requirements	2539

A.6.5.7A.2	DL interruptions at switching between two uplink carriers in TDD-TDD CA	2539

A.6.5.7A.2.1	Test Purpose and Environment	2539

A.6.5.7A.2.2	Test Requirements	2543

A.6.5.7B	DL interruptions at switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors	2543

A.6.5.7B.1	DL interruptions at switching between two uplink bands in FDD-TDD CA	2543

A.6.5.7B.1.1	Test Purpose and Environment	2543

A.6.5.7B.1.2	Test Requirements	2547

A.6.5.7B.2	DL interruptions at switching between two uplink bands in TDD-TDD CA	2547

A.6.5.7B.2.1	Test Purpose and Environment	2547

A.6.5.7B.2.2	Test Requirements	2551

A.6.5.7C	DL interruptions at switching between two uplink bands with two transmit antenna connectors	2551

A.6.5.7C.1	DL interruptions at switching between two uplink bands with two transmit antenna connectors in FDD-TDD CA	2551

A.6.5.7C.1.1	Test Purpose and Environment	2551

A.6.5.7C.1.2	Test Requirements	2555

A.6.5.7C.2	DL interruptions at switching between two uplink bands with two transmit antenna connectors in TDD-TDD CA	2556

A.6.5.7C.2.1	Test Purpose and Environment	2556

A.6.5.7C.2.2	Test Requirements	2560

A.6.5.8	UE specific CBW change	2561

A.6.5.8.1	UE specific CBW change on PCell in FR1 in non-DRX	2561

A.6.5.8.1.1	Test Purpose and Environment	2561

A.6.5.8.1.2	Test Requirements	2565

A.6.5.9	Pathloss reference signal switching delay	2565

A.6.5.9.1	MAC-CE based pathloss reference signal switch delay	2565

A.6.5.9.1.1	Test Purpose and Environment	2565

A.6.5.9.1.2	Test Requirements	2568

A.6.6	Measurement procedure	2569

A.6.6.1	Intra-frequency Measurements	2569

A.6.6.1.1	SA event triggered reporting tests without gap under non-DRX	2569

A.6.6.1.1.1	Test purpose and Environment	2569

A.6.6.1.1.2	Test parameters	2569

A.6.6.1.1.3	Test Requirements	2573

A.6.6.1.2	SA event triggered reporting tests without gap under DRX	2573

A.6.6.1.2.1	Test purpose and Environment	2573

A.6.6.1.2.2	Test parameters	2573

A.6.6.1.2.3	Test Requirements	2577

A.6.6.1.3	SA event triggered reporting tests with per-UE gaps under non-DRX	2577

A.6.6.1.3.1	Test purpose and Environment	2577

A.6.6.1.3.2	Test parameters	2577

A.6.6.1.3.3	Test Requirements	2581

A.6.6.1.4	SA event triggered reporting tests with per-UE gaps under DRX	2581

A.6.6.1.4.1	Test purpose and Environment	2581

A.6.6.1.4.2	Test parameters	2581

A.6.6.1.4.3	Test Requirements	2585

A.6.6.1.5	SA event triggered reporting tests without gap under non-DRX with SSB index reading	2585

A.6.6.1.5.1	Test purpose and Environment	2585

A.6.6.1.5.2	Test parameters	2585

A.6.6.1.5.3	Test Requirements	2587

A.6.6.1.6	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading	2588

A.6.6.1.6.1	Test purpose and Environment	2588

A.6.6.1.6.2	Test parameters	2588

A.6.6.1.6.3	Test Requirements	2589

A.6.6.1.7	SA event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	2590

A.6.6.1.7.1	Test purpose and Environment	2590

A.6.6.1.7.2	Test parameters	2590

A.6.6.1.7.3	Test Requirements	2594

A.6.6.1.8	SA event triggered reporting tests without gap under DRX for UE configured with highSpeedMeasCA-Scell-r17	2594

A.6.6.1.8.1	Test purpose and Environment	2594

A.6.6.1.8.2	Test parameters	2594

A.6.6.1.8.3	Test Requirements	2598

A.6.6.2	Inter-frequency Measurements	2598

A.6.6.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2598

A.6.6.2.1.1	Test Purpose and Environment	2598

A.6.6.2.1.2	Test Requirements	2602

A.6.6.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2602

A.6.6.2.2.1	Test Purpose and Environment	2602

A.6.6.2.2.2	Test Requirements	2606

A.6.6.2.3	Void	2607

A.6.6.2.4	Void	2607

A.6.6.2.5	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2607

A.6.6.2.5.1	Test Purpose and Environment	2607

A.6.6.2.5.2	Test Requirements	2611

A.6.6.2.6	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2611

A.6.6.2.6.1	Test Purpose and Environment	2611

A.6.6.2.6.2	Test Requirements	2615

A.6.6.2.7	Void	2615

A.6.6.2.8	Void	2615

A.6.6.2.9	SA event triggered reporting tests with additional mandatory gap pattern	2615

A.6.6.2.9.1	Test Purpose and Environment	2615

A.6.6.2.9.2	Test Requirements	2618

A.6.6.2.10	SA event triggered reporting tests for FR1 when DRX is used	2618

A.6.6.2.10.1	Test Purpose and Environment	2618

A.6.6.2.10.2	Test Requirements	2622

A.6.6.2.12	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for UE configured with highSpeedMeasInterFreq-r17	2626

A.6.6.2.12.1	Test Purpose and Environment	2626

A.6.6.2.12.2	Test Requirements	2630

A.6.6.3	Inter-RAT Measurements	2630

A.6.6.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1	2630

A.6.6.3.1.1	Test Purpose and Environment	2630

A.6.6.3.1.2	Test Requirements	2636

A.6.6.3.2	SA NR - E-UTRAN event-triggered reporting in DRX in FR1	2636

A.6.6.3.2.1	Test Purpose and Environment	2636

A.6.6.3.2.2	Test Requirements	2643

A.6.6.3.3	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for UE configured with highSpeedMeasFlag-r16	2643

A.6.6.3.3.1	Test Purpose and Environment	2643

A.6.6.3.3.2	Test Requirements	2648

A.6.6.4	L1-RSRP measurement for beam reporting	2649

A.6.6.4.1	SSB based L1-RSRP measurement when DRX is not used	2649

A.6.6.4.1.1	Test Purpose and Environment	2649

A.6.6.4.1.2	Test parameters	2649

A.6.6.4.1.3	Test Requirements	2652

A.6.6.4.2	SSB based L1-RSRP measurement when DRX is used	2652

A.6.6.4.2.1	Test Purpose and Environment	2652

A.6.6.4.2.2	Test parameters	2653

A.6.6.4.2.3	Test Requirements	2656

A.6.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	2656

A.6.6.4.3.1	Test Purpose and Environment	2656

A.6.6.4.3.2	Test parameters	2657

A.6.6.4.3.3	Test Requirements	2660

A.6.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	2660

A.6.6.4.4.1	Test Purpose and Environment	2660

A.6.6.4.4.2	Test parameters	2661

A.6.6.4.4.3	Test Requirements	2664

A.6.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	2664

A.6.6.4.5.1	Test Purpose and Environment	2664

A.6.6.4.5.2	Test parameters	2665

A.6.6.4.5.3	Test Requirements	2668

A.6.6.4.6	Inter-cell SSB based L1-RSRP measurements on FR1 PCell when DRX is used	2669

A.6.6.4.6.1	Test Purpose and Environment	2669

A.6.6.4.6.2	Test parameters	2669

A.6.6.4.6.3	Test Requirements	2672

A.6.6.5	Inter-RAT UTRAN FDD measurements	2673

A.6.6.5.1	SA NR - UTRAN FDD event-triggered reporting in non-DRX in FR1	2673

A.6.6.5.1.1	Test Purpose and Environment	2673

A.6.6.5.1.2	Test Requirements	2676

A.6.6.6	CLI measurements	2676

A.6.6.6.1	SRS-RSRP measurement with DRX	2676

A.6.6.6.1.1	Test Purpose and Environment	2676

A.6.6.6.1.2	Test Parameters	2677

A.6.6.6.1.3	Test Requirements	2680

A.6.6.6.2	CLI-RSSI measurement with DRX	2680

A.6.6.6.2.1	Test Purpose and Environment	2680

A.6.6.6.2.2	Test Parameters	2681

A.6.6.6.2.3	Test Requirements	2683

A.6.6.7	NR measurements with autonomous gaps	2683

A.6.6.7.1	SA intra-frequency CGI identification of NR neighbor cell in FR1	2683

A.6.6.7.1.1	Test Purpose and Environment	2683

A.6.6.7.1.2	Test Parameters	2683

A.6.6.7.1.3	Test Requirements	2686

A.6.6.7.2	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA	2686

A.6.6.7.2.1	Test Purpose and Environment	2686

A.6.6.7.2.2	Test Requirements	2689

A.6.6.8	L1-SINR measurement for beam reporting	2690

A.6.6.8.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	2692

A.6.6.8.2.1	Test Purpose and Environment	2692

A.6.6.8.2.2	Test parameters	2692

A.6.6.8.2.3	Test Requirements	2697

A.6.6.8.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	2697

A.6.6.8.3.1	Test Purpose and Environment	2697

A.6.6.8.3.2	Test parameters	2698

A.6.6.8.3.3	Test Requirements	2700

A.6.6.9	Idle Mode CA/DC Measurements	2700

A.6.6.9.1	SA Idle mode CA/DC measurement for FR1	2700

A.6.6.9.1.1	Test Purpose and Environment	2700

A.6.6.9.1.2	Test Requirements	2707

A.6.6.10	CSI-RS based intra-frequency Measurements	2707

A.6.6.10.1	SA event triggered reporting tests without gap under non-DRX	2707

A.6.6.10.1.1	Test purpose and Environment	2707

A.6.6.10.1.2	Test Requirements	2710

A.6.6.11	CSI-RS based inter-frequency Measurements	2710

A.6.6.11.1	 SA event triggered reporting tests with gap under DRX	2710

A.6.6.11.1.1	Test Purpose and Environment	2710

A.6.6.11.1.2	Test Requirements	2714

A.6.6.12	RSTD measurements	2714

A.6.6.12.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA	2714

A.6.6.12.1.1	Test Purpose and Environment	2714

A.6.6.12.1.2	Test Requirements	2721

A.6.6.12.2	NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR1 SA	2722

A.6.6.12.2.1	Test Purpose and Environment	2722

A.6.6.12.2.2	Test Requirements	2728

A.6.6.12.3	NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA	2728

A.6.6.12.3.1	Test Purpose and Environment	2728

A.6.6.12.3.2	Test Requirements	2734

A.6.6.12.4	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA without measurement gap	2734

A.6.6.12.4.1	Test Purpose and Environment	2734

A.6.6.12.4.2	Test Requirements	2738

A.6.6.12.5	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_CONNECTED state with Rx TEG	2739

A.6.6.12.5.1	Test Purpose and Environment	2739

A.6.6.12.5.2	Test Requirements	2743

A.6.6.13 PRS-RSRP measurements	2743

A.6.6.13.1	PRS-RSRP reporting delay test case for single positioning frequency layer	2743

A.6.6.13.1.1	Test purpose and Environment	2743

A.6.6.13.1.2	Test Requirements	2747

A.6.6.13.2	PRS-RSRP reporting delay test case for dual positioning frequency layer	2747

A.6.6.13.2.1	Test purpose and Environment	2747

A.6.6.13.2.2	Test Requirements	2751

A.6.6.13.3	PRS-RSRP reporting delay test case for reduced number of samples	2751

A.6.6.13.3.1	Test purpose and Environment	2751

A.6.6.13.3.2	Test Requirements	2755

A.6.6.13.4	PRS-RSRP reporting delay test case for single positioning frequency layer outside MG	2755

A.6.6.13.4.1	Test purpose and Environment	2755

A.6.6.14	UE Rx-Tx time difference measurements	2759

A.6.6.14.1	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA	2759

A.6.6.14.1.1	Test purpose and environment	2759

A.6.6.14.1.2	Test requirements	2763

A.6.6.14.2	UE Rx-Tx time difference measurement for dual positioning frequency layers in FR1 SA	2763

A.6.6.14.2.1	Test purpose and environment	2763

A.6.6.14.2.2	Test requirements	2767

A.6.6.14.3	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA with reduced sample number	2767

A.6.6.14.3.1	Test purpose and environment	2767

A.6.6.14.3.2	Test requirements	2771

A.6.6.14.4	UE Rx-Tx time difference measurement without gaps in FR1 SA	2771

A.6.6.14.4.1	Test purpose and environment	2771

A.6.6.14.4.2	Test requirements	2775

A.6.6.14.5	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA with multiple RxTx TEGs	2775

A.6.6.14.4.1	Test purpose and environment	2775

A.6.6.14.4.2	Test requirements	2779

A.6.6.15	Idle Mode measurements of inter-RAT DC candidate cells for early reporting	2779

A.6.6.15.1	Test Purpose and Environment	2779

A.6.6.15.2	Test Requirements	2786

A.6.6.16	PRS-RSRPP measurements	2787

A.6.6.16.1	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_CONNECTED state	2787

A.6.6.16.1.1	Test purpose and Environment	2787

A.6.6.16.1.2	Test Requirements	2789

A.6.6.16.2	PRS-RSRPP reporting delay test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_CONNECTED state	2789

A.6.6.16.2.1	Test purpose and Environment	2789

A.6.6.16.2.2	Test Requirements	2792

A.6.6.16.3	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_CONNECTED state without measurement gap	2792

A.6.6.16.3.1	Test purpose and Environment	2792

A.6.6.16.3.2	Test Requirements	2796

A.6.6.17	SA event triggered reporting tests with Pre-MG	2796

A.6.6.17.1	SA event triggered reporting tests with autonomous activation/deactivation Pre-MG	2796

A.6.6.17.1.1	Test purpose and Environment	2796

A.6.6.17.1.2	Test parameters	2796

A.6.6.17.1.3	Test Requirements	2800

A.6.6.17.2	SA event triggered reporting tests with pre-configured measurement gaps and network-controlled activation/deactivation	2800

A.6.6.17.2.1	Test purpose and Environment	2800

A.6.6.17.2.2	Test parameters	2800

A.6.6.17.2.3	Test Requirements	2805

A.6.6.17.3	Void	2805

A.6.6.17.3.1	Void	2805

A.6.6.17.3.2	Void	2805

A.6.6.17.3.3	Void	2805

A.6.6.18	SA event triggered reporting tests with concurrent gaps	2805

A.6.6.18.1	SA event triggered reporting tests for FR1 concurrent gaps with non-overalpping scenario for SSB-based measurements in both inter-frequency layers	2805

A.6.6.18.1.1	Test Purpose and Environment	2805

A.6.6.18.1.2	Test Requirements	2809

A.6.6.18.2	SA event triggered reporting tests for FR1 concurrent gap with partially partial overalpping scenario for SSB-based measurements in both inter-frequency layers	2809

A.6.6.18.2.1	Test Purpose and Environment	2809

A.6.6.18.2.2	Test Requirements	2813

A.6.6.18.3	SA NR - E-UTRAN and NR FR1 concurrent event-triggered reporting in non-DRX in FR1	2813

A.6.6.18.3.1	Test Purpose and Environment	2813

A.6.6.18.3.2	Test Requirements	2820

A.6.6.18.4	SA event triggered reporting tests for PRS and SSB measurement in FR1 without SSB time index detection when DRX is not used	2820

A.6.6.18.4.1	Test Purpose and Environment	2820

A.6.6.18.4.2	Test Requirements	2824

A.6.6.19	SA event triggered reporting tests with NCSG	2825

A.6.6.19.1	SA event triggered reporting tests with NCSG under non-DRX in FR1	2825

A.6.6.19.1.1	Test purpose and Environment	2825

A.6.6.19.1.2	Test parameters	2825

A.6.6.19.1.3	Test Requirements	2829

A.6.6.19.2	SA event triggered reporting tests for FR1 with NCSG for inter-frequency measurement	2829

A.6.6.19.2.1	Test Purpose and Environment	2829

A.6.6.19.2.2	Test parameters	2829

A.6.6.19.2.3	Test Requirements	2833

A.6.6.19.3	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 with NCSG	2833

A.6.6.19.3.1	Test Purpose and Environment	2833

A.6.6.19.3.2	Test parameters	2834

A.6.6.19.3.3	Test Requirements	2839

A.6.6.19.4	Event triggered reporting on SCC with deactivated SCell test with per-UE NCSG under non-DRX	2839

A.6.6.19.4.1	Test purpose and Environment	2839

A.6.6.19.4.2	Test parameters	2839

A.6.6.19.4.3	Test Requirements	2842

A.6.6.20	UE Rx-Tx time difference measurement for propagation delay compensation	2842

A.6.6.20.1	Test purpose and environment	2842

A.6.6.20.2	Test requirements	2846

A.6.6.21	UE Rx-Tx time difference measurement with TRS for RTT-based PDC in FR1 SA	2846

A.6.6.21.1	Test purpose and environment	2846

A.6.6.21.2	Test requirements	2848

A.6.7	Measurement Performance requirements	2849

A.6.7.1	SS-RSRP	2849

A.6.7.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	2849

A.6.7.1.1.1	Test Purpose and Environment	2849

A.6.7.1.1.2	Test parameters	2849

A.6.7.1.1.3	Test Requirements	2854

A.6.7.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	2854

A.6.7.1.2.1	Test Purpose and Environment	2854

A.6.7.1.2.2	Test parameters	2854

A.6.7.1.2.3	Test Requirements	2858

A.6.7.1.3	Void	2858

A.6.7.2	SS-RSRQ	2858

A.6.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2858

A.6.7.2.1.1	Test Purpose and Environment	2858

A.6.7.2.1.2	Test Parameters	2859

A.6.7.2.1.3	Test Requirements	2863

A.6.7.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2863

A.6.7.2.2.1	Test Purpose and Environment	2863

A.6.7.2.2.2	Test Parameters	2863

A.6.7.2.2.3	Test Requirements	2868

A.6.7.3	SS-SINR	2868

A.6.7.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2868

A.6.7.3.1.1	Test Purpose and Environment	2868

A.6.7.3.1.2	Test Parameters	2869

A.6.7.3.1.3	Test Requirements	2873

A.6.7.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2874

A.6.7.3.2.1	Test Purpose and Environment	2874

A.6.7.3.2.2	Test Parameters	2874

A.6.7.3.2.3	Test Requirements	2878

A.6.7.4	L1-RSRP measurement for beam reporting	2879

A.6.7.4.1	SSB based L1-RSRP measurement	2879

A.6.7.4.1.1	Test Purpose and Environment	2879

A.6.7.4.1.2	Test parameters	2879

A.6.7.4.1.3	Test Requirements	2883

A.6.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	2883

A.6.7.4.2.1	Test Purpose and Environment	2883

A.6.7.4.2.2	Test parameters	2884

A.6.7.4.2.3	Test Requirements	2888

A.6.7.5	E-UTRAN RSRP	2888

A.6.7.5.1	SA: inter-RAT measurement accuracy with FR1 serving cell	2888

A.6.7.5.1.1	Test Purpose and Environment	2888

A.6.7.5.1.2	Test parameters	2889

A.6.7.5.1.3	Test Requirements	2895

A.6.7.6	E-UTRAN RSRQ	2896

A.6.7.6.1	SA: inter-RAT measurement accuracy with FR1 serving cell	2896

A.6.7.6.1.1	Test Purpose and Environment	2896

A.6.7.6.1.2	Test parameters	2896

A.6.7.6.1.3	Test Requirements	2901

A.6.7.7	E-UTRAN RS-SINR	2902

A.6.7.7.1	SA: inter-RAT measurement accuracy with FR1 serving cell	2902

A.6.7.7.1.1	Test Purpose and Environment	2902

A.6.7.7.1.2	Test parameters	2902

A.6.7.7.1.3	Test Requirements	2908

A.6.7.8	CLI measurements	2909

A.6.7.8.1	SA SRS-RSRP measurement accuracy with FR1 serving cell	2909

A.6.7.8.1.1	Test Purpose and Environment	2909

A.6.7.8.1.2	Test parameters	2909

A.6.7.8.1.3	Test Requirements	2915

A.6.7.8.2	SA CLI-RSSI measurement accuracy with FR1 serving cell	2915

A.6.7.8.2.1	Test Purpose and Environment	2915

A.6.7.8.2.2	Test parameters	2916

A.6.7.8.2.3	Test Requirements	2919

A.6.7.9	L1-SINR measurement for beam reporting	2920

A.6.7.9.2	L1-SINR measurement with SSB based CMR and dedicated IMR	2924

A.6.7.9.2.1	Test Purpose and Environment	2924

A.6.7.9.2.2	Test parameters	2925

A.6.7.9.2.3	Test Requirements	2930

A.6.7.9.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	2930

A.6.7.9.3.1	Test Purpose and Environment	2930

A.6.7.9.3.2	Test parameters	2930

A.6.7.9.3.3	Test Requirements	2935

A.6.7.10	CSI-RSRP	2935

A.6.7.10.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	2935

A.6.7.9.10.1	Test Purpose and Environment	2935

A.6.7.9.10.2	Test parameters	2936

A.6.7.10.1.3	Test Requirements	2941

A.6.7.10.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	2941

A.6.7.9.10.1	Test Purpose and Environment	2941

A.6.7.10.2.2	Test parameters	2941

A.6.7.10.2.3	Test Requirements	2946

A.6.7.11	CSI-RSRQ	2946

A.6.7.11.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2946

A.6.7.11.1.1	Test Purpose and Environment	2946

A.6.7.11.1.2	Test Parameters	2946

A.6.7.11.1.3	Test Requirements	2951

A.6.7.11.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2951

A.6.7.11.2.1	Test Purpose and Environment	2951

A.6.7.11.2.2	Test Parameters	2951

A.6.7.11.2.3	Test Requirements	2957

A.6.7.12	CSI-SINR	2958

A.6.7.12.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2958

A.6.7.12.1.1	Test Purpose and Environment	2958

A.6.7.12.1.2	Test Parameters	2958

A.6.7.12.1.3	Test Requirements	2962

A.6.7.12.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2963

A.6.7.12.2.1	Test Purpose and Environment	2963

A.6.7.12.2.2	Test Parameters	2963

A.6.7.12.2.3	Test Requirements	2969

A.6.7.13	RSTD measurements	2969

A.6.7.13.1	RSTD measurement accuracy test case for single positioning frequency layer	2969

A.6.7.13.1.1	Test purpose and Environment	2969

A.6.7.13.1.2	Test Requirements	2972

A.6.7.13.2	RSTD measurement accuracy test case for dual positioning frequency layer	2972

A.6.7.13.2.1	Test purpose and Environment	2972

A.6.7.13.2.2	Test Requirements	2976

A.6.7.13.3	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_CONNECTED state	2976

A.6.7.13.3.1	Test purpose and Environment	2976

A.6.7.13.3.2	Test Requirements	2978

A.6.7.13.4	RSTD measurement accuracy test case with Rx TEG	2978

A.6.7.14	PRS-RSRP measurements	2982

A.6.7.14.1	SA: measurement accuracy with PRS in FR1	2982

A.6.7.14.1.1	Test Purpose and Environment	2982

A.6.7.14.1.2	Test parameters	2983

A.6.7.14.1.3	Test Requirements	2987

A.6.7.14.2	SA: measurement accuracy with PRS in FR1 with reduced sample number	2987

A.6.7.14.2.1	Test Purpose and Environment	2987

A.6.7.14.2.2	Test parameters	2987

A.6.7.14.2.3	Test Requirements	2990

A.6.7.14.3	Void	2991

A.6.7.14.3.1	Void	2991

A.6.7.14.3.2	Void	2991

A.6.7.14.3.3	Void	2991

A.6.7.15	UE Rx-Tx time difference measurements	2991

A.6.7.15.1	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR1 SA	2991

A.6.7.15.1.1	Test purpose and environment	2991

A.6.7.15.1.2	Test parameters	2991

A.6.7.15.1.3	Test requirements	2994

A.6.7.15.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR1 SA	2995

A.6.7.15.2.1	Test purpose and environment	2995

A.6.7.15.2.2	Test parameters	2995

A.6.7.15.2.3	Test requirements	2998

A.6.7.15.3	UE Rx-Tx time difference measurement accuracy with RxTx TEG	2998

A.6.7.15.3.1	Test purpose and environment	2998

A.6.7.15.3.2	Test parameters	2999

A.6.7.15.3.3	Test requirements	3002

A.6.7.16	PRS-RSRPP measurements	3002

A.6.7.16.1	SA: measurement accuracy with PRS in FR1	3002

A.6.7.16.1.1	Test Purpose and Environment	3002

A.6.7.16.1.2	Test parameters	3003

A.6.7.16.1.3	Test Requirements	3008

A.6.7.16.2	SA: measurement accuracy with reduced PRS samples in FR1	3008

A.6.7.16.2.1	Test Purpose and Environment	3008

A.6.7.16.2.2	Test parameters	3008

A.6.8	Measurement procedure in RRC_INACTIVE	3012

A.6.8.1	RSTD measurements	3012

A.6.8.1.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_INACTIVE state	3012

A.6.8.1.1.1	Test Purpose and Environment	3012

A.6.8.1.1.2	Test Requirements	3015

A.6.8.1.2	 NR RSTD measurement reporting delay test case with reduced number of samples in RRC_INACTIVE, FR1 SA	3015

A.6.8.1.2.1	Test Purpose and Environment	3015

A.6.8.1.2.2	Test Requirements	3020

A.6.8.2	PRS-RSRP measurements	3020

A.6.8.2.1	PRS-RSRP reporting delay test case for single positioning frequency layer in RRC_INACTIVE	3020

A.6.8.2.1.1	Test purpose and Environment	3020

A.6.8.2.1.2	Test Requirements	3022

A.6.8.2.2	PRS-RSRP reporting delay test case with reduced number of samples in RRC_INACTIVE	3023

A.6.8.2.2.1	Test purpose and Environment	3023

A.6.8.2.2.2	Test Requirements	3026

A.6.8.3	UE Rx-Tx time difference measurements	3026

A.6.8.3.1	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA	3026

A.6.8.3.1.1	Test purpose and environment	3026

A.6.8.3.1.2	Test requirements	3030

A.6.8.3.2	UE Rx-Tx time difference measurement with reduced number of samples in RRC_INACTIVE, FR1 SA	3030

A.6.8.3.2.1	Test purpose and environment	3030

A.6.8.3.2.2	Test requirements	3033

A.6.8.4	PRS-RSRPP measurements	3033

A.6.8.4.1	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_INACTIVE state	3033

A.6.8.4.1.1	Test purpose and Environment	3033

A.6.8.4.1.2	Test Requirements	3036

A.6.8.4.2	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_INACTIVE state for reduced number of samples	3036

A.6.8.4.2.1	Test purpose and Environment	3036

A.6.8.4.2.2	Test Requirements	3038

A.6.9	Measurement performance requirements in RRC_INACTIVE	3038

A.6.9.1	RSTD measurements	3038

A.6.9.1.1	RSTD measurement accuracy test case for single positioning frequency layer in FR1 in RRC_INACTIVE state	3038

A.6.9.1.1.1	Test purpose and Environment	3038

A.6.9.1.1.2	Test Requirements	3040

A.6.9.1.2	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_INACTIVE state	3040

A.6.9.1.2.1	Test purpose and Environment	3040

A.6.9.1.2.2	Test Requirements	3042

A.6.9.2	PRS-RSRP measurements	3042

A.6.9.2.1	SA: measurement accuracy with PRS in FR1 in RRC_INACTIVE	3042

A.6.9.2.1.1	Test Purpose and Environment	3042

A.6.9.2.1.2	Test parameters	3043

A.6.9.2.1.3	Test Requirements	3048

A.6.9.2.2	SA: measurement accuracy with PRS in FR1 with reduced number of samples in RRC_INACTIVE state	3048

A.6.9.2.2.1	Test Purpose and Environment	3048

A.6.9.2.2.2	Test parameters	3048

A.6.9.2.2.3	Test Requirements	3051

A.6.9.3	UE Rx-Tx time difference measurements	3052

A.6.9.3.1.1	UE Rx-Tx time difference measurement accuracy in FR1 SA	3052

A.6.9.3.1.1.1	Test purpose and environment	3052

A.6.9.3.1.1.2 Test parameters	3052

A.6.9.3.1.1.3	Test requirements	3055

A.6.9.3.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples	3055

A.6.9.3.2.1	Test purpose and environment	3055

A.6.9.3.2.2	Test parameters	3055

A.6.9.3.2.3	Test requirements	3058

A.6.9.4	PRS-RSRPP measurements	3058

A.6.9.4.1	SA: PRS-RSRPP measurement accuracy in FR1 in RRC INACTIVE	3058

A.6.9.4.1.1	Test Purpose and Environment	3058

A.6.9.4.1.2	Test parameters	3058

A.6.9.4.1.3	Test Requirements	3063

A.6.9.4.2	SA: measurement accuracy with reduced PRS samples in FR1 in RRC INACTIVE	3063

A.6.9.4.2.1	Test Purpose and Environment	3063

A.6.9.4.2.2	Test parameters	3063

A.6.9.4.2.3	Test Requirements	3066

A.7	NR standalone tests with one or more NR cells in FR2	3067

A.7.1	SA: RRC_IDLE state mobility	3067

A.7.1.1	Cell re-selection to NR	3067

A.7.1.1.1	Cell reselection to FR2 intra-frequency NR case	3067

A.7.1.1.1.1	Test Purpose and Environment	3067

A.7.1.1.1.2	Test Parameters	3067

A.7.1.1.1.3	Test Requirements	3070

A.7.1.1.2	Cell reselection to FR2 inter-frequency NR case	3070

A.7.1.1.2.1	Test Purpose and Environment	3070

A.7.1.1.2.2	Test Parameters	3071

A.7.1.1.2.3	Test Requirements	3073

A.7.1.1.3	Cell reselection to FR2 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	3073

A.7.1.1.3.1	Test Purpose and Environment	3073

A.7.1.1.3.2	Test Parameters	3073

A.7.1.1.3.3	Test Requirements	3076

A.7.1.1.4	Cell reselection to FR2 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	3076

A.7.1.1.4.1	Test Purpose and Environment	3076

A.7.1.1.4.2	Test Parameters	3076

A.7.1.1.4.3	Test Requirements	3079

A.7.1.1.5	Cell reselection to FR2 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	3079

A.7.1.1.5.1	Test Purpose and Environment	3079

A.7.1.1.5.2	Test Parameters	3079

A.7.1.1.5.3	Test Requirements	3082

A.7.1.1.6	Cell reselection to FR2 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	3082

A.7.1.1.6.1	Test Purpose and Environment	3082

A.7.1.1.6.2	Test Parameters	3082

A.7.1.1.6.3	Test Requirements	3085

A.7.1.1.7	Cell reselection to FR2 intra-frequency NR case for FR2 power class 6 UE configured with highSpeedMeasFlagFR2-r17	3085

A.7.1.1.7.1	Test Purpose and Environment	3085

A.7.1.1.7.2	Test Parameters	3085

A.7.1.1.7.3	Test Requirements	3089

A.7.2	SA: RRC_INACTIVE state mobility	3089

A.7.2.1	Small Data Transmission	3089

A.7.2.1.1	TA validation for CG-SDT in FR2	3089

A.7.2.1.1.1	Test Purpose and Environment	3089

A.7.2.1.1.2	Test Requirements	3093

A.7.3	RRC_CONNECTED state mobility	3093

A.7.3.1	Handover	3093

A.7.3.1.1	Inter-frequency handover from FR1 to FR2; unknown target cell	3093

A.7.3.1.1.1	Test Purpose and Environment	3093

A.7.3.1.1.2	Test Parameters	3093

A.7.3.1.1.3	Test Requirements	3097

A.7.3.1.2	Intra-frequency handover from FR2 to FR2; unknown target cell	3097

A.7.3.1.2.1	Test Purpose and Environment	3097

A.7.3.1.2.2	Test Parameters	3097

A.7.3.1.2.3	Test Requirements	3100

A.7.3.1.3	Inter-frequency handover from FR2 to FR2; unknown target cell	3100

A.7.3.1.3.1	Test Purpose and Environment	3100

A.7.3.1.3.2	Test Parameters	3100

A.7.3.1.3.3	Test Requirements	3101

A.7.3.1.4	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR2	3102

A.7.3.1.4.1	Test Purpose and Environment	3102

A.7.3.1.4.2	Test Parameters	3102

A.7.3.1.4.3 Test Requirements	3109

A.7.3.1.5	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR2	3109

A.7.3.1.5.1	Test Purpose and Environment	3109

A.7.3.1.5.2	Test Parameters	3109

A.7.3.1.5.3 Test Requirements	3116

A.7.3.1.6	Handover with PSCell from SA to EN-DC with; unknown FR2 target PScell	3116

A.7.3.1.6.1	Test Purpose and Environment	3116

A.7.3.1.6.2	Test Parameters	3116

A.7.3.1.6.3	Test Requirements	3124

A.7.3.1.7	HO with PSCell from FR1 NR-SA to EN-DC with known E-UTRA PCell and known FR2 PSCell	3125

A.7.3.1.7.1	Test purpose and environment	3125

A.7.3.1.7.2	Test Requirements	3130

A.7.3.1.8	NR PSCell change delay in HO with PSCell from NR-DC to NR-DC	3131

A.7.3.1.8.1	Test Purpose and Environment	3131

A.7.3.1.8.2	Test Requirements	3136

A.7.3.1.9	Intra-frequency handover from FR2-2 to FR2-2; unknown target cell	3136

A.7.3.1.9.1	Test Purpose and Environment	3136

A.7.3.1.9.2	Test Parameters	3136

A.7.3.1.9.3	Test Requirements	3139

A.7.3.1.10	Inter-frequency handover from FR2-2 to FR2-2; unknown target cell	3139

A.7.3.1.10.1	Test Purpose and Environment	3139

A.7.3.1.10.2	Test Parameters	3139

A.7.3.1.10.3	Test Requirements	3143

A.7.3.1.11	Inter-frequency handover from FR1 to FR2-2; unknown target cell	3143

A.7.3.1.11.1	Test Purpose and Environment	3143

A.7.3.1.11.2	Test Parameters	3143

A.7.3.1.11.3	Test Requirements	3147

A.7.3.2	RRC Connection Mobility Control	3147

A.7.3.2.1	SA: RRC Re-establishment	3147

A.7.3.2.1.1	Intra-frequency RRC Re-establishment in FR2	3147

A.7.3.2.1.2	Inter-frequency RRC Re-establishment in FR2	3150

A.7.3.2.1.3	Intra-frequency RRC Re-establishment in FR2 without serving cell timing	3153

A.7.3.2.1.3.1	Test Purpose and Environment	3153

A.7.3.2.1.3.2	Test Requirements	3155

A.7.3.2.1.4	Intra-frequency RRC Re-establishment in FR2-2	3156

A.7.3.2.1.4.1	Test Purpose and Environment	3156

A.7.3.2.1.4.2	Test Requirements	3158

A.7.3.2.1.5	Inter-frequency RRC Re-establishment in FR2-2	3159

A.7.3.2.1.5.1	Test Purpose and Environment	3159

A.7.3.2.1.5.2	Test Requirements	3162

A.7.3.2.1.6	Intra-frequency RRC Re-establishment in FR2-2 without serving cell timing	3162

A.7.3.2.1.6.1	Test Purpose and Environment	3162

A.7.3.2.1.6.2	Test Requirements	3164

A.7.3.2.2	Random Access	3165

A.7.3.2.2.1	4-step RA type c ontention based random access test in FR2 for NR Standalone	3165

A.7.3.2.2.2	4-step RA type n on-contention based random access test in FR2 for NR Standalone	3169

A.7.3.2.2.3	2-step RA type contention based random access test in FR2 for NR Standalone	3173

A.7.3.2.2.4	2-step RA type n on-contention based random access test in FR2 for NR Standalone	3176

A.7.3.2.3	SA: RRC Connection Release with Redirection	3179

A.7.3.2.3.1	Redirection from NR in FR2 to NR in FR2	3179

A.7.3.3 Conditional Handover	3183

A.7.3.3.1	Intra-frequency conditional handover from FR2 to FR2	3183

A.7.3.3.1.1	Test Purpose and Environment	3183

A.7.3.3.1.2	Test Parameters	3183

A.7.3.3.1.2.3	Test Requirements	3186

A.7.3.3.2	Inter-frequency conditional handover from FR2 to FR2; unknown target cell	3186

A.7.3.3.2.1	Test Purpose and Environment	3186

A.7.3.3.2.2	Test Parameters	3186

A.7.3.3.2.3 Test Requirements	3188

A.7.4	Timing	3188

A.7.4.1	UE transmit timing	3188

A.7.4.1.1	NR UE Transmit Timing Test for FR2	3188

A.7.4.1.1.1	Test Purpose and environment	3188

A.7.4.1.1.2	Test requirements	3191

A.7.4.1.2	NR UE Transmit Timing Test for FR2-2	3191

A.7.4.1.2.1	Test Purpose and environment	3191

A.7.4.1.2.2	Test requirements	3196

A.7.4.2	UE timer accuracy	3197

A.7.4.3	Timing advance	3197

A.7.4.3.1	SA FR2 timing advance adjustment accuracy	3197

A.7.4.3.1.1	Test Purpose and Environment	3197

A.7.4.3.1.2	Test Parameters	3197

A.7.4.3.1.3 Test Requirements	3200

A.7.4.3.2	SA FR2-2 timing advance adjustment accuracy	3201

A.7.4.3.2.1	Test Purpose and Environment	3201

A.7.4.3.2.2	Test Parameters	3201

A.7.4.3.2.3	Test Requirements	3205

A.7.5	Signaling characteristics	3205

A.7.5.1	Radio link Monitoring	3205

A.7.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	3205

A.7.5.1.1.1	Test Purpose and Environment	3205

A.7.5.1.1.2	Test Requirements	3208

A.7.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	3209

A.7.5.1.2.1	Test Purpose and Environment	3209

A.7.5.1.2.2	Test Requirements	3213

A.7.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	3214

A.7.5.1.3.1	Test Purpose and Environment	3214

A.7.5.1.3.2	Test Requirements	3218

A.7.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	3218

A.7.5.1.4.1	Test Purpose and Environment	3218

A.7.5.1.4.2	Test Requirements	3223

A.7.5.1.5	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	3223

A.7.5.1.5.1	Test Purpose and Environment	3223

A.7.5.1.5.2	Test Requirements	3228

A.7.5.1.6	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	3228

A.7.5.1.6.1	Test Purpose and Environment	3228

A.7.5.1.6.2	Test Requirements	3232

A.7.5.1.7	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	3232

A.7.5.1.7.1	Test Purpose and Environment	3232

A.7.5.1.7.2	Test Requirements	3236

A.7.5.1.8	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	3236

A.7.5.1.8.1	Test Purpose and Environment	3236

A.7.5.1.8.2	Test Requirements	3241

A.7.5.1.9	UE Radio Link Monitoring Scheduling Restrictions on FR2	3241

A.7.5.1.9.1	Test Purpose and Environment	3241

A.7.5.1.9.2	Test Requirements	3244

A.7.5.2	Interruption	3244

A.7.5.2.1	Interruptions during measurements on deactivated NR SCC in FR2	3244

A.7.5.2.1.1	Test Purpose and Environment	3244

A.7.5.2.1.2	Test Requirements	3247

A.7.5.2.2	SA interruptions at NR SRS carrier-based switching	3248

A.7.5.2.2.1	Test Purpose and Environment	3248

A.7.5.2.2.2	Test Parameters	3248

A.7.5.2.2.3	Test Requirements	3250

A.7.5.3	SCell Activation and Deactivation Delay	3250

A.7.5.3.1	SCell Activation and deactivation for SCell in FR2 intra-band in non-DRX	3250

A.7.5.3.1.1	Test Purpose and Environment	3250

A.7.5.3.1.2	Test Requirements	3252

A.7.5.3.2	SCell Activation and deactivation for FR1+FR2 inter-band with target SCell in FR2	3252

A.7.5.3.2.1	Test Purpose and Environment	3252

A.7.5.3.2.2	Test Requirements	3256

A.7.5.3.3	SCell Activation and deactivation for SCell in FR2 inter-band in non-DRX	3257

A.7.5.3.3.1	Test Purpose and Environment	3257

A.7.5.3.3.2	Test Requirements	3260

A.7.5.3.4	Direct SCell activation at SCell addition of known SCell in FR2	3261

A.7.5.3.4.1	Test Purpose and Environment	3261

A.7.5.3.4.2	Test Requirements	3264

A.7.5.3.5	Direct SCell activation at handover with known SCell in FR2	3265

A.7.5.3.5.1	Test Purpose and Environment	3265

A.7.5.3.5.2	Test Requirements	3268

A.7.5.3.6	PUCCH SCell activation and deactivation for FR1+FR2 inter-band with target SCell in FR2 and known	3269

A.7.5.3.6.1	Test Purpose and Environment	3269

A.7.5.3.6.2	Test Requirements	3273

A.7.5.3.7	PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with FR1 PCell	3274

A.7.5.3.7.1	Test Purpose and Environment	3274

A.7.5.3.7.2	Test Requirements	3278

A.7.5.3.8	SCell Activation and deactivation for known PUCCH SCell in FR2 inter-band in non-DRX	3279

A.7.5.3.8.1	Test Purpose and Environment	3279

A.7.5.3.8.2	Test Requirements	3283

A.7.5.3.9	PUCCH SCell Activation and deactivation of unknown SCell in FR2	3284

A.7.5.3.9.1	Test Purpose and Environment	3284

A.7.5.3.9.2	Test Requirements	3287

A.7.5.3.10	SCell Activation and deactivation of FR2 known PUCCH SCell and one FR2 unknown SCell with FR2 PCell	3288

A.7.5.3.10.1	Test Purpose and Environment	3288

A.7.5.3.10.2	Test Requirements	3292

A.7.5.3.11	PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with FR2 PCell	3293

A.7.5.3.11.1	PUCCH SCell activation with non-PUCCH SCell in a secondary PUCCH Group	3293

A.7.5.3.11.1.1	Test Purpose and Environment	3293

A.7.5.3.11.1.2	Test Requirements	3297

A.7.5.3.11.2	PUCCH SCell activation with non-PUCCH SCell in a primary PUCCH Group	3298

A.7.5.3.11.2.1	Test Purpose and Environment	3298

A.7.5.3.11.2.2	Test Requirements	3302

A.7.5.3.12	Void	3303

A.7.5.3.13	SCell Activation for SCell in FR2 intra-band in non-DRX	3303

A.7.5.3.13.1	Test Purpose and Environment	3303

A.7.5.3.13.2	Test Requirements	3305

A.7.5.3.14	SCell Activation for known SCell in FR2 inter-band	3306

A.7.5.3.14.1	Test Purpose and Environment	3306

A.7.5.3.14.2	Test Requirements	3309

A.7.5.15	Void	3310

A.7.5.4	Void	3310

A.7.5.5	Beam Failure Detection and Link recovery procedures	3310

A.7.5.5.1	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	3310

A.7.5.5.1.1	Test Purpose and Environment	3310

A.7.5.5.1.2	Test Requirements	3314

A.7.5.5.2	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode	3315

A.7.5.5.2.1	Test Purpose and Environment	3315

A.7.5.5.2.2	Test Requirements	3318

A.7.5.5.3	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	3319

A.7.5.5.3.1	Test Purpose and Environment	3319

A.7.5.5.3.2	Test Requirements	3323

A.7.5.5.4	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode	3324

A.7.5.5.4.1	Test Purpose and Environment	3324

A.7.5.5.4.2	Test Requirements	3328

A.7.5.5.5	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	3329

A.7.5.5.5.1	Test Purpose and Environment	3329

A.7.5.5.5.2	Test Requirements	3332

A.7.5.5.6	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	3333

A.7.5.5.6.1	Test Purpose and Environment	3333

A.7.5.5.6.2	Test Requirements	3337

A.7.5.5.7	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	3338

A.7.5.5.7.1	Test Purpose and Environment	3338

A.7.5.5.7.2	Test Requirements	3342

A.7.5.5.8	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode for UE fulfilling relaxed measurement criterion	3343

A.7.5.5.8.1	Test Purpose and Environment	3343

A.7.5.5.8.2	Test Requirements	3347

A.7.5.5.9	TRP specific Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	3347

A.7.5.5.9.1	Test Purpose and Environment	3347

A.7.5.5.9.2	Test Requirements	3353

A.7.5.5.10	TRP specific Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	3353

A.7.5.5.10.1	Test Purpose and Environment	3353

A.7.5.5.10.2	Test Requirements	3359

A.7.5.5.11	Beam Failure Detection and Link Recovery Test for FR2-2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	3359

A.7.5.5.11.1	Test Purpose and Environment	3359

A.7.5.5.11.2	Test Requirements	3364

A.7.5.5.12	Beam Failure Detection and Link Recovery Test for FR2-2 PCell configured with CSI-RS-based BFD and LR in DRX mode	3364

A.7.5.5.12.1	Test Purpose and Environment	3364

A.7.5.5.12.2	Test Requirements	3369

A.7.5.5.13	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2-2 PCell configured with SSB-based BFD and LR in non-DRX mode	3369

A.7.5.5.13.1	Test Purpose and Environment	3369

A.7.5.5.13.2	Test Requirements	3373

A.7.5.6	Active BWP switch	3373

A.7.5.6.1	DCI-based and Timer-based Active BWP Switch	3373

A.7.5.6.1.1	NR FR2- NR FR2 DL active BWP switch of SCell with non-DRX in SA	3373

A.7.5.6.1.2	NR FR1- NR FR2 DL active BWP switch of SCell with non-DRX in SA	3378

A.7.5.6.1.3	NR FR2 DL active BWP switch with non-DRX in SA	3383

A.7.5.6.1.3.1	Test Purpose and Environment	3383

A.7.5.6.1.3.2	Test Requirements	3386

A.7.5.6.1.4	NR FR2-2- NR FR2-2 DL active BWP switch of SCell with non-DRX in SA	3386

A.7.5.6.1.4.1	Test Purpose and Environment	3386

A.7.5.6.1.4.2	Test Requirements	3391

A.7.5.6.2	RRC-based Active BWP Switch	3392

A.7.5.6.2.1.1	Test Purpose and Environment	3392

A.7.5.6.2.1.2	Test Requirements	3395

A.7.5.6.2.2	NR FR2-2 DL active BWP switch of PCell with non-DRX in SA	3396

A.7.5.6.2.2.1	Test Purpose and Environment	3396

A.7.5.6.2.2.2	Test Requirements	3400

A.7.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	3401

A.7.5.6.3.1.1	Test Purpose and Environment	3401

A.7.5.6.3.1.2	Test Requirements	3404

A.7.5.6.4	SCell dormancy switch	3404

A.7.5.6.4.1	NR FR2 PCell SCell dormancy switch of single FR2 SCell inside active time	3404

A.7.5.6.4.1.1	Test Purpose and Environment	3404

A.7.5.6.4.1.2	Test Requirements	3409

A.7.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR2 SCells outside active time	3409

A.7.5.6.4.2.1	 Test Purpose and Environment	3409

A.7.5.6.4.2.2	 Test Requirements	3414

A.7.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	3414

A.7.5.6.5.1	Active BWP switch on multiple SCells with non-DRX in SA	3414

A.7.5.6.5.2	NR FR2-2 Active BWP switch on multiple SCells with non-DRX in SA	3417

A.7.5.6.5.2.1	Test Purpose and Environment	3417

A.7.5.6.5.2.2	Test Requirements	3421

A.7.5.7	PSCell addition and release delay	3422

A.7.5.7.1	Addition and Release Delay of known NR PSCell	3422

A.7.5.7.1.1	Test Purpose and Environment	3422

A.7.5.7.1.2	Test Requirements	3425

A.7.5.7.2	Addition and Release Delay of unknown NR PSCell in	3425

A.7.5.7.2.1	Test Purpose and Environment	3425

A.7.5.7.2.2	Test Requirements	3428

A.7.5.7.3	Addition and Release Delay of known NR PSCell in FR2-2	3428

A.7.5.7.3.1	Test Purpose and Environment	3428

A.7.5.7.3.2	Test Requirements	3431

A.7.5.7.4	Addition and Release Delay of unknown NR PSCell in FR2-2	3431

A.7.5.7.4.1	Test Purpose and Environment	3431

A.7.5.7.4.2	Test Requirements	3434

A.7.5.8	Active TCI state switch delay	3434

A.7.5.8.1	MAC-CE based active TCI state switch	3434

A.7.5.8.2	RRC based active TCI state switch	3438

A.7.5.8.3	MAC-CE based active TCI state switch for HST FR2 scenario	3442

A.7.5.8.3.1	NR PCell FR2 HST active TCI state switch for a known TCI state	3442

A.7.5.8.3.1.1	Test Purpose and Environment	3442

A.7.5.8.3.1.2	Test Requirements	3446

A.7.5.9	Uplink spatial relation switch delay	3447

A.7.5.9.1.1.1	Test Purpose and Environment	3447

A.7.5.9.1.1.2	Test Requirements	3450

A.7.5.9.2	RRC based spatial relation switch	3450

A.7.5.9.2.1	NR PCell FR2 spatial relation switch associated with a known DL-RS	3450

A.7.5.9.2.1.1	Test Purpose and Environment	3450

A.7.5.9.2.1.2	Test Requirements	3453

A.7.5.10	UE specific CBW change	3453

A.7.5.10.1	NR FR2 UE specific CBW change of PCell with non-DRX in SA	3453

A.7.5.10.1.1	Test Purpose and Environment	3453

A.7.5.10.1.2	Test Requirements	3456

A.7.5.11	UE UL carrier RRC reconfiguration Delay	3457

A.7.5.11.1	UE UL carrier RRC reconfiguration Delay	3457

A.7.5.11.1.1	Test Purpose and Environment	3457

A.7.5.11.1.2	Test Requirements	3460

A.7.5.12	Conditional PSCell addition and release delay (FR2 SA)	3460

A.7.5.12.1	Addition and Release Delay of PSCell	3460

A.7.5.12.1.1	Test purpose and environment	3460

A.7.5.12.1.2	Test Parameters	3460

A.7.5.12.1.3	Test Requirements	3463

A.7.5.13	 Unified TCI state switching delay	3463

A.7.5.13.1	MAC-CE based active joint TCI state switching	3463

A.7.5.13.1.1	NR PCell FR2 active joint TCI state switch for a known TCI state	3463

A.7.5.13.1.1.1	Test Purpose and Environment	3463

A.7.5.13.1.1.2	Test parameters	3464

A.7.5.13.1.1.3	Test Requirements	3466

A.7.5.13.2	 MAC-CE based active uplink TCI state switch	3466

A.7.5.13.2.1	 NR FR2 PCell uplink TCI state switch for a known TCI state	3466

A.7.5.13.2.1.1	Test Purpose and Environment	3466

A.7.5.13.2.1.2	Test parameters	3467

A.7.5.13.2.1.3	Test Requirements	3469

A.7.5.13.3	MAC-CE based active downlink TCI state switch	3469

A.7.5.13.3.1	NR PCell FR2 active downlink TCI state switch to cell with additional PCI for a known TCI state	3469

A.7.5.13.3.1.1	Test Purpose and Environment	3469

A.7.5.13.3.1.2	Test Parameters	3470

A.7.5.13.3.1.3	Test Requirements	3473

A.7.5.14	PSCell RACH-less based Activation and deactivation for FR1+FR2 inter-band with target PSCell in FR2	3473

A.7.5.14.1	Test Purpose and Environment	3473

A.7.5.14.2	Test Requirements	3476

A.7.6	Measurement procedure	3477

A.7.6.1	Intra-frequency Measurements	3477

A.7.6.1.1	SA event triggered reporting test without gap under non-DRX	3477

A.7.6.1.1.1	Test purpose and Environment	3477

A.7.6.1.1.2	Test Requirements	3480

A.7.6.1.2	SA event triggered reporting test without gap under DRX	3480

A.7.6.1.2.1	Test purpose and Environment	3480

A.7.6.1.2.2	Test Requirements	3483

A.7.6.1.3	SA event triggered reporting test with per-UE gaps under non-DRX	3484

A.7.6.1.3.1	Test purpose and Environment	3484

A.7.6.1.3.2	Test Requirements	3487

A.7.6.1.4	SA event triggered reporting test with per-UE gaps under DRX	3487

A.7.6.1.4.1	Test purpose and Environment	3487

A.7.6.1.4.2	Test Requirements	3490

A.7.6.1.5	SA event triggered reporting test without gap under non-DRX for UE configured with highSpeedMeasFlagFR2-r17	3491

A.7.6.1.5.1	Test purpose and Environment	3491

A.7.6.1.5.2	Test Requirements	3494

A.7.6.1.6	SA event triggered reporting test without gap under non-DRX for FR2-2	3494

A.7.6.1.6.1	Test purpose and Environment	3494

A.7.6.1.6.2	Test Requirements	3497

A.7.6.1.7	SA event triggered reporting test without gap under DRX for FR2-2	3498

A.7.6.1.7.1	Test purpose and Environment	3498

A.7.6.1.7.2	Test Requirements	3500

A.7.6.1.8	SA event triggered reporting test with per-UE gaps under non-DRX for FR2-2	3501

A.7.6.1.8.1	Test purpose and Environment	3501

A.7.6.1.8.2	Test Requirements	3504

A.7.6.1.9	SA event triggered reporting test with per-UE gaps under DRX for FR2-2	3505

A.7.6.1.9.1	Test purpose and Environment	3505

A.7.6.1.9.2	Test Requirements	3507

A.7.6.1.10	SA event triggered reporting test with SSB time index detection without gap under non-DRX for FR2-2	3508

A.7.6.1.10.1	Test purpose and Environment	3508

A.7.6.1.10.2	Test Requirements	3511

A.7.6.1.11	SA event triggered reporting test with SSB time index detection with per-UE gaps under non-DRX for FR2-2	3512

A.7.6.1.11.1	Test purpose and Environment	3512

A.7.6.1.11.2	Test Requirements	3514

A.7.6.2	Inter-frequency Measurements	3515

A.7.6.2.1	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR2)	3515

A.7.6.2.1.1	Test Purpose and Environment	3515

A.7.6.2.1.2	Test Requirements	3518

A.7.6.2.2	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (Pcell in FR2)	3518

A.7.6.2.2.1	Test Purpose and Environment	3518

A.7.6.2.2.2	Test Requirements	3522

A.7.6.2.3	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR2)	3522

A.7.6.2.3.1	Test Purpose and Environment	3522

A.7.6.2.3.2	Test Requirements	3526

A.7.6.2.4	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (Pcell in FR2)	3526

A.7.6.2.4.1	Test Purpose and Environment	3526

A.7.6.2.4.2	Test Requirements	3530

A.7.6.2.5	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR1)	3530

A.7.6.2.5.1	Test Purpose and Environment	3530

A.7.6.2.5.2	Test Requirements	3534

A.7.6.2.6	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (Pcell in FR1)	3535

A.7.6.2.6.1	Test Purpose and Environment	3535

A.7.6.2.6.2	Test Requirements	3539

A.7.6.2.7	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR1)	3540

A.7.6.2.7.1	Test Purpose and Environment	3540

A.7.6.2.7.2	Test Requirements	3544

A.7.6.2.8	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR1)	3545

A.7.6.2.8.1	Test Purpose and Environment	3545

A.7.6.2.8.2	Test Requirements	3549

A.7.6.2.9	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2) (rel16 additional mandatory gap pattern 17)	3550

A.7.6.2.9.1	Test Purpose and Environment	3550

A.7.6.2.9.2	Test Requirements	3554

A.7.6.2.10	SA event triggered reporting test without gap under non-DRX	3554

A.7.6.2.10.1	Test Purpose and Environment	3554

A.7.6.2.10.2	Test Requirements	3556

A.7.6.2.11	SA event triggered reporting test without gap under DRX	3556

A.7.6.2.11.1	Test Purpose and Environment	3556

A.7.6.2.11.2	Test Requirements	3559

A.7.6.2.12	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is not used (PCell in FR2-2)	3559

A.7.6.2.12.1	Test Purpose and Environment	3559

A.7.6.2.12.2	Test Requirements	3563

A.7.6.2.13	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is used (PCell in FR2-2)	3564

A.7.6.2.13.1	Test Purpose and Environment	3564

A.7.6.2.13.2	Test Requirements	3568

A.7.6.2.14	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is not used (PCell in FR2-2)	3569

A.7.6.2.14.1	Test Purpose and Environment	3569

A.7.6.2.14.2	Test Requirements	3573

A.7.6.2.15	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is used (PCell in FR2-2)	3574

A.7.6.2.15.1	Test Purpose and Environment	3574

A.7.6.2.15.2	Test Requirements	3578

A.7.6.2.16	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is not used (PCell in FR1)	3579

A.7.6.2.16.1	Test Purpose and Environment	3579

A.7.6.2.16.2	Test Requirements	3585

A.7.6.2.17	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is used (PCell in FR1)	3585

A.7.6.2.17.1	Test Purpose and Environment	3585

A.7.6.2.17.2	Test Requirements	3592

A.7.6.2.18	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is not used (PCell in FR1)	3593

A.7.6.2.18.1	Test Purpose and Environment	3593

A.7.6.2.18.2	Test Requirements	3598

A.7.6.2.19	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is used (PCell in FR1)	3599

A.7.6.2.19.1	Test Purpose and Environment	3599

A.7.6.2.19.2	Test Requirements	3607

A.7.6.3	L1-RSRP measurement for beam reporting	3608

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	3608

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	3608

A.7.6.3.1.1	Test Purpose and Environment	3608

A.7.6.3.1.2	Test parameters	3608

A.7.6.3.1.3	Test Requirements	3610

A.7.6.3.2	SSB based L1-RSRP measurement when DRX is used	3610

A.7.6.3.2.1	Test Purpose and Environment	3610

A.7.6.3.2.2	Test parameters	3611

A.7.6.3.2.3	Test Requirements	3613

A.7.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	3613

A.7.6.3.3.1	Test Purpose and Environment	3613

A.7.6.3.3.2	Test parameters	3614

A.7.6.3.3.3	Test Requirements	3616

A.7.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	3617

A.7.6.3.4.1	Test Purpose and Environment	3617

A.7.6.3.4.2	Test parameters	3617

A.7.6.3.3.3	Test Requirements	3619

A.7.6.3.5	SSB based L1-RSRP measurement when DRX is used for power class 6 UE configured with highSpeedMeasFlagFR2-r17	3620

A.7.6.3.5.1	Test Purpose and Environment	3620

A.7.6.3.5.2	Test parameters	3620

A.7.6.3.5.3	Test Requirements	3622

A.7.6.3.6	Inter-cell SSB based L1-RSRP measurements on FR2 SCell when DRX is not used	3622

A.7.6.3.6.1	Test Purpose and Environment	3622

A.7.6.3.6.2	Test parameters	3623

A.7.6.3.6.3	Test Requirements	3626

A.7.6.3.7	SSB based L1-RSRP measurement for FR2-2 when DRX is used	3626

A.7.6.3.7.1	Test Purpose and Environment	3626

A.7.6.3.7.2	Test parameters	3627

A.7.6.3.7.3	Test Requirements	3630

A.7.6.4	CLI measurements	3631

A.7.6.4.1	SRS-RSRP measurement with non-DRX	3631

A.7.6.4.1.1	Test Purpose and Environment	3631

A.7.6.4.1.2	Test Parameters	3631

A.7.6.4.1.3	Test Requirements	3633

A.7.6.4.2	CLI-RSSI measurement with non-DRX	3633

A.7.6.4.2.1	Test Purpose and Environment	3633

A.7.6.4.2.2	Test Parameters	3634

A.7.6.4.2.3	Test Requirements	3635

A.7.6.5.1   SA interfrequency CGI reporting in autonomous gaps test (PCell in FR2)	3636

A.7.6.5.1.1	Test Purpose and Environment	3636

A.7.6.5.1.2	Test Requirements	3639

A.7.6.6	L1-SINR measurement for beam reporting	3639

A.7.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	3641

A.7.6.6.2.1	Test Purpose and Environment	3641

A.7.6.6.2.2	Test parameters	3642

A.7.6.6.2.3	Test Requirements	3644

A.7.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	3644

A.7.6.6.3.1	Test Purpose and Environment	3644

A.7.6.6.3.2	Test parameters	3645

A.7.6.6.3.3	Test Requirements	3647

A.7.6.7	CSI-RS based intra-frequency Measurements	3647

A.7.6.7.1	SA event triggered reporting test without gap under DRX for CSI-RS based intra-frequency measurement	3647

A.7.6.7.1.1	Test purpose and Environment	3647

A.7.6.7.1.2	Test Requirements	3650

A.7.6.8	CSI-RS based inter-frequency Measurements	3650

A.7.6.8.1	SA event triggered reporting tests for FR2 CSI-RS based measurement when non-DRX is used (PCell in FR2)	3650

A.7.6.8.1.1	Test Purpose and Environment	3650

A.7.6.8.1.2	Test Requirements	3653

A.7.6.9	RSTD measurements	3653

A.7.6.9.1	 NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA	3653

A.7.6.9.1.1	Test Purpose and Environment	3653

A.7.6.9.1.2	Test Requirements	3661

A.7.6.9.2	 NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR2 SA	3661

A.7.6.9.2.1	Test Purpose and Environment	3661

A.7.6.9.2.2	Test Requirements	3669

A.7.6.9.3	NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA	3669

A.7.6.9.3.1	Test Purpose and Environment	3669

A.7.6.9.3.2	Test Requirements	3675

A.7.6.9.4	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA without measurement gap	3676

A.7.6.9.4.1	Test Purpose and Environment	3676

A.7.6.9.4.2	Test Requirements	3679

A.7.6.9.5	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_CONNECTED state with Rx TEG	3680

A.7.6.9.5.1	Test Purpose and Environment	3680

A.7.6.9.5.2	Test Requirements	3683

A.7.6.10 PRS-RSRP measurements	3683

A.7.6.10.1 PRS-RSRP reporting delay test case for single positioning frequency layer	3683

A.7.6.10.1.1	Test Purpose and Environment	3683

A.7.6.10.1.2	Test Requirements	3688

A.7.6.10.2	PRS-RSRP reporting delay test case for dual positioning frequency layer	3688

A.7.6.10.2.1	Test Purpose and Environment	3688

A.7.6.10.2.2	Test Requirements	3692

A.7.6.10.3	PRS-RSRP reporting delay test case for reduced number of samples	3692

A.7.6.10.3.1	Test Purpose and Environment	3692

A.7.6.10.3.2	Test Requirements	3696

A.7.6.10.4	PRS-RSRP reporting delay test case for single positioning frequency layer outside MG	3696

A.7.6.10.4.1	Test Purpose and Environment	3696

A.7.6.10.4.2	Test Requirements	3700

A.7.6.11	UE Rx-Tx time difference measurements	3700

A.7.6.11.1	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA	3700

A.7.6.11.1.1	Test purpose and environment	3700

A.7.6.11.1.2	Test requirements	3704

A.7.6.11.2	UE Rx-Tx time difference measurement period for dual positioning frequency layers in FR2 SA	3704

A.7.6.11.2.1	Test purpose and environment	3704

A.7.6.11.2.2	Test requirements	3708

A.7.6.11.3	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA with reduced sample number	3708

A.7.6.11.3.1	Test purpose and environment	3708

A.7.6.11.3.2	Test requirements	3710

A.7.6.11.4	UE Rx-Tx time difference measurements without gaps in FR2 SA	3711

A.7.6.11.4.1	Test purpose and environment	3711

A.7.6.11.4.2	Test requirements	3713

A.7.6.11.5	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA with RxTx TEG	3714

A.7.6.11.5.1	Test purpose and environment	3714

A.7.6.11.5.2	Test requirements	3716

A.7.6.12	PRS-RSRPP measurements	3717

A.7.6.12.1 PRS-RSRPP reporting delay test case for single positioning frequency layer in FR2 in RRC_CONNECTED state	3717

A.7.6.12.1.1	Test Purpose and Environment	3717

A.7.6.12.1.2	Test Requirements	3719

A.7.6.12.2	PRS-RSRPP reporting delay test case for reduced number of samples for single positioning frequency layer in FR2 in RRC_CONNECTED state	3720

A.7.6.12.2.1	Test Purpose and Environment	3720

A.7.6.12.2.2	Test Requirements	3722

A.7.6.12.3	PRS-RSRPP reporting delay test case for gapless measurement in FR2	3723

A.7.6.12.3.1	Test Purpose and Environment	3723

A.7.6.12.3.2	Test Requirements	3726

A.7.6.13	UE Rx-Tx time difference measurements for PDC	3726

A.7.6.13.1	UE Rx-Tx time difference measurement for propagation delay compensation using PRS in FR2	3726

A.7.6.13.1.1	Test purpose and environment	3726

A.7.6.13.1.2	Test requirements	3728

A.7.6.13.2	UE Rx-Tx time difference measurement for propagation delay compensation using TRS in FR2	3729

A.7.6.13.2.1	Test purpose and environment	3729

A.7.6.13.2.2	Test requirements	3730

A.7.6.14	SA event triggered reporting tests with Pre-MG	3731

A.7.6.14.1	Intra-frequency measurement test with SA event triggered reporting tests: with autonomous activation/deactivation of Pre-MG in FR2	3731

A.7.6.14.1.1	Test purpose and Environment	3731

A.7.6.14.1.2	Test parameters	3731

A.7.6.14.1.3	Test Requirements	3733

A.7.6.14.2	Intra-frequency measurement test with SA event triggered reporting tests: with network-controlled activation/deactivation of Pre-MG in FR2	3734

A.7.6.14.2.1	Test purpose and Environment	3734

A.7.6.14.2.2	Test parameters	3734

A.7.6.14.2.3	Test Requirements	3736

A.7.6.15	SA event triggered reporting tests with concurrent gaps	3737

A.7.6.15.1	SA event triggered reporting tests For FR2 with fully non-overlapping concurrent MGs for SSB-based inter-frequency measurements	3737

A.7.6.15.1.1	Test Purpose and Environment	3737

A.7.6.15.1.2	Test Requirements	3740

A.7.6.15.2	SA event triggered reporting tests For FR2 with concurrent measurement gaps without SSB time index detection when DRX is not used (PCell in FR2)	3740

A.7.6.15.2.1	Test Purpose and Environment	3740

A.7.6.15.2.2	Test Requirements	3743

A.7.6.15.3	SA event triggered reporting tests for FR2 concurrent gap with partially partial overalpping scenario for SSB-based measurements and PRS-based measurement	3743

A.7.6.15.3.1	Test Purpose and Environment	3743

A.7.6.15.3.2	Test Requirements	3747

A.7.6.16	SA event triggered reporting tests with NCSG	3747

A.7.6.16.1	SA event triggered reporting test with per-UE NCSG under non-DRX	3747

A.7.6.16.1.1	Test purpose and Environment	3747

A.7.6.16.1.2	Test Requirements	3751

A.7.6.16.2	SA event triggered reporting tests on inter-frequency measurement with NCSG for FR2 when DRX is not used (PCell in FR2)	3751

A.7.6.16.2.1	Test Purpose and Environment	3751

A.7.6.16.2.2	Test Requirements	3754

A.7.6.16.3	Event triggered reporting test on deactivated Scell measurement via NCSG in FR2 in non-DRX	3754

A.7.6.16.3.1	Test Purpose and Environment	3754

A.7.6.16.3.2	Test Requirements	3757

A.7.7	Measurement Performance requirements	3757

A.7.7.1	SS-RSRP	3758

A.7.7.1.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3758

A.7.7.1.1.1	Test Purpose and Environment	3758

A.7.7.1.1.2	Test parameters	3758

A.7.7.1.1.3	Test Requirements	3762

A.7.7.1.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3763

A.7.7.1.2.1	Test Purpose and Environment	3763

A.7.7.1.2.2	Test parameters	3763

A.7.7.1.2.3	Test Requirements	3767

A.7.7.1.3	SA inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	3768

A.7.7.1.3.1	Test Purpose and Environment	3768

A.7.7.1.3.2	Test parameters	3768

A.7.7.1.3.3	Test Requirements	3772

A.7.7.2	SS-RSRQ	3772

A.7.7.2.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	3772

A.7.7.2.1.1	Test Purpose and Environment	3772

A.7.7.2.1.2	Test Parameters	3772

A.7.7.2.1.3	Test Requirements	3774

A.7.7.2.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3774

A.7.7.2.2.1	Test Purpose and Environment	3774

A.7.7.2.2.2	Test Parameters	3774

A.7.7.2.2.3	Test Requirements	3776

A.7.7.3	SS-SINR	3776

A.7.7.3.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3776

A.7.7.3.1.1	Test Purpose and Environment	3776

A.7.7.3.1.2	Test Parameters	3776

A.7.7.3.1.3	Test Requirements	3778

A.7.7.3.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3778

A.7.7.3.2.1	Test Purpose and Environment	3778

A.7.7.3.2.2	Test Parameters	3778

A.7.7.3.2.3	Test Requirements	3780

A.7.7.4	L1-RSRP measurement for beam reporting	3780

A.7.7.4.1	SSB based L1-RSRP measurement	3780

A.7.7.4.1.1	Test Purpose and Environment	3780

A.7.7.4.1.2	Test parameters	3781

A.7.7.4.1.3	Test Requirements	3783

A.7.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	3784

A.7.7.4.2.1	Test Purpose and Environment	3784

A.7.7.4.2.2	Test parameters	3784

A.7.7.4.2.3	Test Requirements	3786

A.7.7.5	CLI measurements	3787

A.7.7.5.1	SA SRS-RSRP measurement accuracy with FR2 serving cell	3787

A.7.7.5.1.1	Test Purpose and Environment	3787

A.7.7.5.1.2	Test parameters	3787

A.7.7.5.1.3	Test Requirements	3790

A.7.7.5.2	SA CLI-RSSI measurement accuracy with FR2 serving cell	3791

A.7.7.5.2.1	Test Purpose and Environment	3791

A.7.7.5.2.2	Test parameters	3791

A.7.7.5.2.3	Test Requirements	3793

A.7.7.6	L1-SINR measurement for beam reporting	3794

A.7.7.6.1.1	Test Purpose and Environment	3794

A.7.7.6.1.2	Test parameters	3794

A.7.7.6.1.3	Test Requirements	3796

A.7.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	3797

A.7.7.6.2.1	Test Purpose and Environment	3797

A.7.7.6.2.2	Test parameters	3797

A.7.7.6.2.3	Test Requirements	3799

A.7.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	3800

A.7.7.6.3.1	Test Purpose and Environment	3800

A.7.7.6.3.2	Test parameters	3800

A.7.7.6.3.3	Test Requirements	3802

A.7.7.7	CSI-RSRP	3803

A.7.7.7.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3803

A.7.7.7.1.1	Test Purpose and Environment	3803

A.7.7.7.1.2	Test parameters	3803

A.7.7.7.1.3	Test Requirements	3806

A.7.7.7.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3807

A.7.7.7.2.1	Test Purpose and Environment	3807

A.7.7.7.2.2	Test parameters	3807

A.7.7.7.2.3	Test Requirements	3810

A.7.7.8	CSI-RSRQ	3811

A.7.7.8.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	3811

A.7.7.8.1.1	Test Purpose and Environment	3811

A.7.7.8.1.2	Test Parameters	3811

A.7.7.8.1.3	Test Requirements	3813

A.7.7.8.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3813

A.7.7.8.2.1	Test Purpose and Environment	3813

A.7.7.8.2.2	Test Parameters	3813

A.7.7.8.2.3	Test Requirements	3815

A.7.7.9	CSI-SINR	3815

A.7.7.9.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3815

A.7.7.9.1.1	Test Purpose and Environment	3815

A.7.7.9.1.2	Test Parameters	3815

A.7.7.9.1.3	Test Requirements	3817

A.7.7.9.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3817

A.7.7.9.2.1	Test Purpose and Environment	3817

A.7.7.9.2.2	Test Parameters	3818

A.7.7.9.2.3	Test Requirements	3820

A.7.7.10	RSTD measurements	3820

A.7.7.10.1	RSTD measurement accuracy test case for single positioning frequency layer	3820

A.7.7.10.1.1	Test purpose and Environment	3820

A.7.7.10.1.2	Test Requirements	3823

A.7.7.10.2	RSTD measurement accuracy test case for dual positioning frequency layer	3823

A.7.7.10.2.1	Test purpose and Environment	3823

A.7.7.10.2.2	Test Requirements	3827

A.7.7.10.3	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR2 in RRC_CONNECTED state	3828

A.7.7.10.3.1	Test purpose and Environment	3828

A.7.7.10.3.2	Test Requirements	3829

A.7.7.10.4	RSTD measurement accuracy test case with Rx TEG	3830

A.7.7.10.4.1	Test purpose and Environment	3830

A.7.7.10.4.2	Test Requirements	3832

A.7.7.11	PRS-RSRP measurements	3832

A.7.7.11.1	SA measurement accuracy with PRS in FR2	3832

A.7.7.11.1.1	Test Purpose and Environment	3832

A.7.7.11.1.2	Test parameters	3832

A.7.7.11.1.3	Test Requirements	3836

A.7.7.11.2	SA measurement accuracy with PRS in FR2 with reduced sample number	3837

A.7.7.11.2.1	Test Purpose and Environment	3837

A.7.7.11.2.2	Test parameters	3837

A.7.7.11.2.3	Test Requirements	3839

A.7.7.12	UE Rx-Tx time difference measurements	3840

A.7.7.12.1	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR2 SA	3840

A.7.7.12.1.1	Test purpose and environment	3840

A.7.7.12.1.2	Test parameters	3840

A.7.7.12.1.3	Test requirements	3843

A.7.7.12.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR2 SA	3844

A.7.7.12.2.1	Test purpose and environment	3844

A.7.7.12.2.2	Test parameters	3844

A.7.7.12.2.3	Test requirements	3847

A.7.7.12.3	UE Rx-Tx time difference measurement accuracy with RxTx TEG	3847

A.7.7.12.3.1	Test purpose and environment	3847

A.7.7.12.3.2	Test parameters	3847

A.7.7.12.3.3	Test requirements	3850

A.7.7.13	PRS-RSRPP measurements	3850

A.7.7.13.1	SA measurement accuracy with PRS in FR2	3850

A.7.7.13.1.1	Test Purpose and Environment	3850

A.7.7.13.1.2	Test parameters	3851

A.7.7.13.1.3	Test Requirements	3853

A.7.7.13.2	SA measurement accuracy with reduced PRS samples in FR2	3854

A.7.7.13.2.1	Test Purpose and Environment	3854

A.7.7.13.2.2	Test parameters	3854

A.7.7.13.2.3	Test Requirements	3856

A.7.8	Measurement procedure in RRC_INACTIVE	3857

A.7.8.1	RSTD measurements	3857

A.7.8.1.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_INACTIVE state	3857

A.7.8.1.1.1	Test Purpose and Environment	3857

A.7.8.1.1.2	Test Requirements	3860

A.7.8.1.2	NR RSTD measurement reporting delay test case with reduced number of samples in RRC_INACTIVE, FR1 SA	3860

A.7.8.1.2.1	Test Purpose and Environment	3860

A.7.8.1.2.2	Test Requirements	3866

A.7.8.2	PRS-RSRP measurements	3867

A.7.8.2.1	PRS-RSRP reporting delay test case for single positioning frequency layer in RRC_INACTIVE	3867

A.7.8.2.1.1	Test Purpose and Environment	3867

A.7.8.2.1.2	Test Requirements	3871

A.7.8.2.2	PRS-RSRP reporting delay test case with reduced number of samples in RRC_INACTIVE	3871

A.7.8.2.2.1	Test purpose and Environment	3871

A.7.8.2.2.2	Test Requirements	3875

A.7.8.3	UE Rx-Tx time difference measurements	3875

A.7.8.3.1	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA	3875

A.7.8.3.1.1	Test purpose and environment	3875

A.7.8.3.1.2	Test requirements	3879

A.7.8.3.2	UE Rx-Tx time difference measurement with reduced number of samples in RRC_INACTIVE, FR2 SA	3879

A.7.8.3.2.1	Test purpose and environment	3879

A.7.8.3.2.2	Test requirements	3881

A.7.8.4	PRS-RSRPP measurements	3882

A.7.8.4.1	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR2 in RRC_INACTIVE state	3882

A.7.8.4.1.1	Test Purpose and Environment	3882

A.7.8.4.1.2	Test Requirements	3884

A.7.8.4.2	PRS-RSRPP reporting delay test with reduced number of samples for single positioning frequency layer in FR2 in RRC_INACTIVE state	3884

A.7.8.4.2.1	Test Purpose and Environment	3884

A.7.8.4.2.2	Test Requirements	3887

A.7.9	Measurement performance requirements in RRC_INACTIVE	3887

A.7.9.1	RSTD measurements	3887

A.7.9.1.1	RSTD measurement accuracy test case for single positioning frequency layer in FR2 in RRC_INACTIVE state	3887

A.7.9.1.1.1	Test purpose and Environment	3887

A.7.9.1.1.2	Test Requirements	3889

A.7.9.1.2	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR2 in RRC_INACTIVE state	3889

A.7.9.1.2.1	Test purpose and Environment	3889

A.7.9.1.2.2	Test Requirements	3891

A.7.9.2	PRS-RSRP measurements	3891

A.7.9.2.1	SA measurement accuracy with PRS in FR2 in RRC_INACTIVE	3891

A.7.9.2.1.1	Test Purpose and Environment	3891

A.7.9.2.1.2	Test parameters	3891

A.7.9.2.1.3	Test Requirements	3893

A.7.9.2.2	PRS-RSRP measurements with reduced number of sample in RRC_INACTIVE	3894

A.7.9.2.2.1	Test Purpose and Environment	3894

A.7.9.2.2.2	Test parameters	3894

A.7.9.2.2.3	Test Requirements	3896

A.7.9.3	UE Rx-Tx time difference measurements	3897

A.7.9.3.1	UE Rx-Tx time difference measurements in RRC_INACTIVE	3897

A.7.9.3.1.1	Test purpose and environment	3897

A.7.9.3.1.2	Test parameters	3897

A.7.9.3.1.3	Test requirements	3900

A.7.9.3.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR2 SA	3900

A.7.9.3.2.1	Test purpose and environment	3900

A.7.9.3.2.2	Test parameters	3900

A.7.9.3.2.3	Test requirements	3903

A.7.9.4	PRS-RSRPP measurements	3903

A.7.9.4.1	SA measurement accuracy in FR2 in RRC INACTIVE	3903

A.7.9.4.1.1	Test Purpose and Environment	3903

A.7.9.4.1.2	Test parameters	3903

A.7.9.4.1.3	Test Requirements	3905

A.7.9.4.2	SA measurement accuracy with reduced PRS samples in FR2 in RRC INACTIVE	3906

A.7.9.4.2.1	Test Purpose and Environment	3906

A.7.9.4.2.2	Test parameters	3906

A.7.9.4.2.3	Test Requirements	3908

A.8	E-UTRA standalone tests for NR RRM	3909

A.8.1	Void	3909

A.8.2	RRC_IDLE state mobility	3909

A.8.2.1	Inter-RAT NR Cell re-selection	3909

A.8.2.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1	3909

A.8.2.1.1.1	Test Purpose and Environment	3909

A.8.2.1.1.2	Test Requirements	3914

A.8.2.1.2	E-UTRA Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16	3915

A.8.2.1.2.1	Test Purpose and Environment	3915

A.8.2.1.2.2	Test Requirements	3919

A.8.2.2	E-UTRA – NR Inter-RAT Early Measruement Reporting	3920

A.8.2.2.1	E-UTRA – NR Early Measurement Reporting for NR in FR1	3920

A.8.2.2.1.1	Test Purpose and Environment	3920

A.8.2.2.1.2	Test Requirements	3924

A.8.2.2.2	E-UTRA – NR Early Measurement Reporting for NR in FR2	3924

A.8.2.2.2.1	Test Purpose and Environment	3924

A.8.2.2.2.2	Test Requirements	3928

A.8.3	RRC_CONNECTED state mobility	3928

A.8.3.1	Handover	3928

A.8.3.1.1	E-UTRAN - NR handover in FR1	3928

A.8.3.1.1.1	Test Purpose and Environment	3928

A.8.3.1.1.2	Test Requirements	3933

A.8.4	Measurement procedure	3933

A.8.4.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay	3933

A.8.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay in non-DRX	3933

A.8.4.1.1.1	Test Purpose and Environment	3933

A.8.4.1.1.2	Test Requirements	3935

A.8.4.1.2	E-UTRA – NR Inter-RAT SFTD Measurement Delay in DRX	3936

A.8.4.1.2.1	Test Purpose and Environment	3936

A.8.4.1.2.2	Test Requirements	3937

A.8.4.2	E-UTRA – NR Inter-RAT Measurements	3937

A.8.4.2.1	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	3937

A.8.4.2.1.1	Test Purpose and Environment	3937

A.8.4.2.1.2	Test Requirements	3942

A.8.4.2.2	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	3942

A.8.4.2.2.1	Test Purpose and Environment	3942

A.8.4.2.2.2	Test Requirements	3946

A.8.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	3946

A.8.4.2.3.1	Test Purpose and Environment	3946

A.8.4.2.3.2	Test Requirements	3951

A.8.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	3951

A.8.4.2.4.1	Test Purpose and Environment	3951

A.8.4.2.4.2	Test Requirements	3955

A.8.4.2.5	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is not used	3955

A.8.4.2.5.1	Test Purpose and Environment	3955

A.8.4.2.5.2	Test Requirements	3957

A.8.4.2.6	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is used	3958

A.8.4.2.6.1	Test Purpose and Environment	3958

A.8.4.2.6.2	Test Requirements	3960

A.8.4.2.7	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is not used	3961

A.8.4.2.7.1	Test Purpose and Environment	3961

A.8.4.2.7.2	Test Requirements	3963

A.8.4.2.8	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is used	3964

A.8.4.2.8.1	Test Purpose and Environment	3964

A.8.4.2.8.2	Test Requirements	3966

A.8.4.2.9	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16	3967

A.8.4.2.9.1	Test Purpose and Environment	3967

A.8.4.2.9.2	Test Requirements	3972

A.8.5	Measurement performance	3972

A.8.5.1	SFTD accuracy	3972

A.8.5.1.1	SFTD accuracy	3972

A.8.5.1.1.1	Test Purpose	3972

A.8.5.1.1.2	Test Environment	3972

A.8.5.1.1.3	Test Requirements	3978

A.8.5.2	E-UTRA – NR Inter-RAT Measurement Performance requirements	3978

A.8.5.2.1.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	3978

A.8.5.2.1.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	3983

A.8.5.2.1.2.1	Test Purpose and Environment	3983

A.8.5.2.1.2.2	Test Parameters	3983

A.8.5.2.1.2.3	Test Requirements	3985

A.8.5.2.2	SS-RSRQ	3985

A.8.5.2.2.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	3985

A.8.5.2.2.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	3990

A.8.5.2.2.2.1	Test Purpose and Environment	3990

A.8.5.2.2.2.2	Test Parameters	3990

A.8.5.2.2.2.3	Test Requirements	3992

A.8.5.2.3	SS-SINR	3992

A.8.5.2.3.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	3992

A.8.5.2.3.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	3996

A.8.5.2.3.2.1	Test Purpose and Environment	3996

A.8.5.2.3.2.2	Test Parameters	3996

A.8.5.2.3.2.3	Test Requirements	3998

A.9	V2X Tests	3999

A.9.1	V2X Tests in FR1	3999

A.9.1.1	Test for V2X UE Transmit Timing	3999

A.9.1.1.1 Test for GNSS as Synchronization Reference Source	3999

A.9.1.1.1.1	Test Purpose and Environment	3999

A.9.1.1.1.2	Test requirements	3999

A.9.1.1.2	Test for SyncRef UE as Synchronization Reference Source	3999

A.9.1.1.2.1	Test Purpose and Environment	3999

A.9.1.1.2.2	Test requirements	4000

A.9.1.1.3	Test for FR1 NR Cell as Synchronization Reference Source	4000

A.9.1.1.3.1	Test Purpose and Environment	4000

A.9.1.1.3.2	Test requirements	4003

A.9.1.2	Test for Initiation/Cease of S-SSB Transmission with V2X Sidelink Communication	4003

A.9.1.2.1	Test for FR1 NR Cell as synchronization reference source without gap under non-DRX	4003

A.9.1.2.1.1	Test Purpose and Environment	4003

A.9.1.2.1.2	Test Requirements	4006

A.9.1.2.2	Test for SyncRef UE as synchronization reference source	4006

A.9.1.2.2.1	Test Purpose and Environment	4006

A.9.1.2.2.2	Test Requirements	4007

A.9.1.2.3	Test for SyncRef UE as synchronization reference source when SL-DRX is used	4008

A.9.1.2.3.1	Test Purpose and Environment	4008

A.9.1.2.3.2	Test Requirements	4010

A.9.1.3	 Test for V2X Synchronization Reference Selection/Reselection	4010

A.9.1.3.1	 Test for GNSS configured as the highest priority	4010

A.9.1.3.1.1	Test Purpose and Environment	4010

A.9.1.3.1.2	Test Requirements	4012

A.9.1.3.2	 Test for FR1 NR Cell configured as the highest priority	4013

A.9.1.3.2.1	Test Purpose and Environment	4013

A.9.1.3.2.2	Test Requirements	4015

A.9.1.3.3	Test for GNSS configured as the highest priority under SL-DRX	4016

A.9.1.3.3.1	Test Purpose and Environment	4016

A.9.1.3.3.2	Test Requirements	4018

A.9.1.3.4	Test for FR1 NR Cell configured as the highest priority under SL-DRX	4019

A.9.1.3.4.1	Test Purpose and Environment	4019

A.9.1.3.4.2	Test Requirements	4021

A.9.1.4	Test for L1 SL-RSRP Measurement	4022

A.9.1.4.1	Test for V2X UE Autonomous Resource Selection/Reselection	4022

A.9.1.4.1.1	Test Purpose and Environment	4022

A.9.1.4.1.2	Test Requirements	4025

A.9.1.4.2	Test for V2X UE Resource Pre-emption	4025

A.9.1.4.2.1	Test Purpose and Environment	4025

A.9.1.4.2.2	Test Requirements	4029

A.9.1.4.3	 Test for V2X UE Resource Re-evaluation	4029

A.9.1.4.3.1	Test Purpose and Environment	4029

A.9.1.4.3.2	Test Requirements	4036

A.9.1.4.4	Test for V2X UE Autonomous Resource Selection/Reselection with Periodic Sensing	4036

A.9.1.4.4.1	Test Purpose and Environment	4036

A.9.1.4.4.2	Test Requirements	4040

A.9.1.4.5	Test for V2X UE Autonomous Resource Selection/Reselection with Contiguous Sensing	4040

A.9.1.4.5.1	Test Purpose and Environment	4040

A.9.1.4.5.2	Test Requirements	4043

A.9.1.4.6	Test for V2X UE Autonomous Resource Selection/Reselection in SL-DRX	4043

A.9.1.4.6.1	Test Purpose and Environment	4043

A.9.1.4.6.2	Test Requirements	4047

A.9.1.5	Test for Congestion Control Measurement	4047

A.9.1.5.1	Test Purpose and Environment	4047

A.9.1.5.2	Test Requirements	4053

A.9.1.6	Test for Interruption	4053

A.9.1.6.1	Test for Interruption to WAN due to V2X Sidelink Communication	4053

A.9.1.6.1.1	Test Purpose and Environment	4053

A.9.1.6.1.2	Test Requirements	4057

A.9.1.6.2	Test for interruption to WAN at transitions between active and non-active during SL-DRX in asynchronous case	4057

A.9.1.6.2.1	Test Purpose and Environment	4057

A.9.1.6.2.2	Test Requirements	4061

A.9.1.6.3	Test for Interruption at NR Sidelink Diccovery Configuration	4061

A.9.1.6.3.1	Test Purpose and Environment	4061

A.9.1.6.3.2	Test Requirements	4064

A.9.1.7	Selection / Reselection of relay UE	4064

A.9.1.7.1	Test Purpose and Environment	4064

A.9.1.7.2	Test Requirements	4070

A.10	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	4071

A.10.1	RRC_CONNECTED state mobility	4071

A.10.1.1	RRC connection mobility control	4071

A.10.1.1.1	Random Access	4071

A.10.1.1.1.1	4-step RA type contention-based random access for NR PSCell with CCA	4071

A.10.1.1.1.1.1	Test Purpose and Environment	4071

A.10.1.1.1.1.2	Test Requirements	4074

A.10.1.1.1.1.2.1	Random Access Preamble Transmission	4074

A.10.1.1.1.1.2.2	Random Access Response Reception	4074

A.10.1.1.1.1.2.3	No Random Access Response Reception	4075

A.10.1.1.1.1.2.4	Receiving an UL grant for msg3 retransmission	4075

A.10.1.1.1.1.2.5	 Contention Resolution Timer expiry	4075

A.10.1.1.1.2	4-step RA type non-contention based random access for NR PSCell with CCA	4076

A.10.1.1.1.2.1	Test Purpose and Environment	4076

A.10.1.1.1.2.2	Test Requirements	4079

A.10.1.1.1.2.2.1	SSB-based Random Access Preamble Transmission	4079

A.10.1.1.1.2.2.2	Random Access Response Reception	4080

A.10.1.1.1.2.2.3	No Random Access Response Reception	4080

A.10.1.1.1.3	2-step RA type contention-based random access for NR PSCell with CCA	4080

A.10.1.1.1.3.1	Test Purpose and Environment	4080

A.10.1.1.1.3.2	Test Requirements	4084

A.10.1.1.1.3.2.1	MsgA Transmission	4084

A.10.1.1.1.3.2.2	MsgB Reception	4084

A.10.1.1.1.3.2.3	No MsgB Reception	4085

A.10.1.1.1.4	2-step RA type non-contention based random access for NR PSCell with CCA	4085

A.10.1.1.1.4.1	Test Purpose and Environment	4085

A.10.1.1.1.4.2	Test Requirements	4089

A.10.1.1.1.4.2.1	MsgA Transmission	4089

A.10.1.1.1.4.2.2	MsgB Reception	4090

A.10.1.1.1.4.2.3	No MsgB Reception	4090

A.10.1.2	Handover with PSCell from EN-DC to EN-DC with known target PSCell using CCA	4090

A.10.1.2.1	Test Purpose and Environment	4090

A.10.1.2.2	Test Requirements	4096

A.10.2	Timing	4097

A.10.2.1	UE transmit timing	4097

A.10.2.1.1	UE Transmit Timing Test with PSCell under DL CCA	4097

A.10.2.1.1.1	Test Purpose and environment	4097

A.10.2.1.1.2	Test requirements	4100

A.10.2.2	UE timing advance	4101

A.10.2.2.1	UE Timing Advance Adjustment Accuracy with PSCell under DL CCA	4101

A.10.2.2.1.1	Test Purpose and Environment	4101

A.10.2.2.1.2	Test Parameters	4101

A.10.2.2.1.3	Test Requirements	4105

A.10.3	Signalling characteristics	4105

A.10.3.1	Radio link monitoring	4105

A.10.3.1.1	Introduction	4105

A.10.3.1.2	Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	4106

A.10.3.1.2.1	Test purpose and environment	4106

A.10.3.1.2.2	Test requirements	4110

A.10.3.1.3	Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	4111

A.10.3.1.3.1	Test purpose and environment	4111

A.10.3.1.3.2	Test requirements	4116

A.10.3.1.4	Void	4116

A.10.3.1.4.1	Void	4116

A.10.3.1.4.2	Void	4116

A.10.3.1.5	Void	4116

A.10.3.1.5.1	Void	4116

A.10.3.1.5.2	Void	4116

A.10.3.2	Interruption	4116

A.10.3.2.1	E-UTRAN – NR interruptions during SCell operations with CCA	4116

A.10.3.2.1.1	Test Purpose and Environment	4116

A.10.3.2.1.2	Test Requirements	4121

A.10.3.3	SCell activation and deactivation delay	4121

A.10.3.3.1	SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 160 ms SCell measurement cycle	4121

A.10.3.3.1.1	Test Purpose and Environment	4121

A.10.3.3.1.2	Test Requirements	4126

A.10.3.3.2 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 640 ms SCell measurement cycle	4126

A.10.3.3.2.1	Test Purpose and Environment	4126

A.10.3.3.2.2	Test Requirements	4127

A.10.3.3.3	SCell Activation and Deactivation of unknown NR SCell with NR PSCell and NR SCell under CCA	4127

A.10.3.3.3.1	Test Purpose and Environment	4127

A.10.3.3.3.2	Test Requirements	4128

A.10.3.4	Beam failure detection and link recovery procedures	4128

A.10.3.4.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	4128

A.10.3.4.1.1	Test Purpose and Environment	4128

A.10.3.4.1.2	Test Requirements	4133

A.10.3.4.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	4133

A.10.3.4.2.1	Test Purpose and Environment	4133

A.10.3.4.2.2	Test Requirements	4139

A.10.3.5	Active BWP switching	4139

A.10.3.5.1	UL active BWP switch delay with consistent UL LBT failure on PSCell subject to UL CCA in EN-DC	4139

A.10.3.5.1.2	Test Requirements	4144

A.10.3.5.2	DCI-based and Timer-based Active BWP Switch	4145

A.10.3.5.2.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	4145

A.10.3.5.2.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	4148

A.10.3.5.3	RRC-based Active BWP Switch	4152

A.10.3.5.3.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	4152

A.10.3.6	PSCell addition and release delay	4156

A.10.3.6.1	Addition and Release Delay of known NR PSCell on the carrier under CCA	4156

A.10.3.6.1.1	Test purpose and environment	4156

A.10.3.6.1.2	Test Requirements	4161

A.10.3.7	Void	4162

A.10.4	Measurement procedure	4162

A.10.4.1	Intra-frequency measurements	4162

A.10.4.1.1	Event-triggered reporting tests on PSCC without gaps under non-DRX	4162

A.10.4.1.1.1	Test purpose and environment	4162

A.10.4.1.1.2	Test parameters	4162

A.10.4.1.1.3	Test Requirements	4165

A.10.4.1.2	Void A.10.4.1.3 Void A.10.4.1.4 Event-triggered reporting tests on PSCC with per-UE gaps under DRX	4165

A.10.4.1.4.1	Test purpose and environment	4165

A.10.4.1.4.2	Test parameters	4165

A.10.4.1.4.3	Test Requirements	4169

A.10.4.1.7	Void A.10.4.1.8 Void A.10.4.1.9 RSSI measurement reporting on PSCC	4169

A.10.4.1.9.1	Test purpose and environment	4169

A.10.4.1.9.2	Test parameters	4170

A.10.4.1.10	Channel occupancy measurement reporting on PSCC	4170

A.10.4.1.10.1	Test purpose and environment	4170

A.10.4.1.10.2	Test parameters	4170

A.10.4.1.11	RSSI measurement reporting on SCC	4170

A.10.4.1.11.1	Test purpose and environment	4170

A.10.4.1.11.2	Test parameters	4171

A.10.4.1.12	Channel occupancy measurement reporting on SCC	4171

A.10.4.1.12.1	Test purpose and environment	4171

A.10.4.1.12.2	Test parameters	4171

A.10.4.2	Inter-frequency measurements	4172

A.10.4.2.1	RSSI measurement reporting	4172

A.10.4.2.1.1	Test purpose and environment	4172

A.10.4.2.1.2	Test parameters	4172

A.10.4.2.2	Channel occupancy measurement reporting	4172

A.10.4.2.2.1	Test purpose and environment	4172

A.10.4.2.2.2	Test parameters	4172

A.10.4.2.3	EN-DC event triggered reporting tests for FR1 with CCA cell without SSB time index detection when DRX is not used	4173

A.10.4.2.3.1	Test Purpose and Environment	4173

A.10.4.2.3.2	Test Requirements	4177

A.10.4.2.4	EN-DC event triggered reporting tests for FR1 cell with CCA without SSB time index detection when DRX is used	4178

A.10.4.2.4.1	Test Purpose and Environment	4178

A.10.4.2.4.2	Test Requirements	4182

A.10.4.2.5	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is not used	4183

A.10.4.2.5.1	Test Purpose and Environment	4183

A.10.4.2.5.2	Test Requirements	4187

A.10.4.2.6	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is used	4188

A.10.4.2.6.1	Test Purpose and Environment	4188

A.10.4.2.6.2	Test Requirements	4192

A.10.4.2.7	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	4193

A.10.4.2.7.1	Test Purpose and Environment	4193

A.10.4.2.7.2	Test Requirements	4199

A.10.4.2.8	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	4199

A.10.4.2.8.1	Test Purpose and Environment	4199

A.10.4.2.8.2	Test Requirements	4205

A.10.4.2.9	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	4206

A.10.4.2.9.1	Test Purpose and Environment	4206

A.10.4.2.9.2	Test Requirements	4211

A.10.4.2.10	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	4211

A.10.4.2.10.1	Test Purpose and Environment	4211

A.10.4.2.10.2	Test Requirements	4217

A.10.4.3	L1-RSRP measurements for beam reporting	4218

A.10.4.3.1	SSB based L1-RSRP measurement on PSCC when DRX is not used	4218

A.10.4.3.1.1	Test Purpose and Environment	4218

A.10.4.3.1.2	Test parameters	4218

A.10.4.3.1.3	Test Requirements	4220

A.10.4.3.2	SSB based L1-RSRP measurement on PSCC when DRX is used	4221

A.10.4.3.2.1	Test Purpose and Environment	4221

A.10.4.3.2.2	Test parameters	4221

A.10.4.3.2.3	Test Requirements	4223

A.10.4.3.3	SSB based L1-RSRP measurement on SCC when DRX is not used	4224

A.10.4.3.3.1	Test Purpose and Environment	4224

A.10.4.3.3.2	Test parameters	4224

A.10.4.3.3.3	Test Requirements	4227

A.10.4.3.4	SSB based L1-RSRP measurement on SCC when DRX is used	4228

A.10.4.3.4.1	Test Purpose and Environment	4228

A.10.4.3.4.2	Test parameters	4228

A.10.4.3.4.3	Test Requirements	4231

A.10.4.4	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	4232

A.10.4.4.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4232

A.10.4.4.1.1	Test Purpose and Environment	4232

A.10.4.4.1.2	Test Requirements	4236

A.10.4.4.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4237

A.10.4.4.2.1	Test Purpose and Environment	4237

A.10.4.4.2.2	Test Requirements	4242

A.10.4.4.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4243

A.10.4.4.3.1	Test Purpose and Environment	4243

A.10.4.4.3.2	Test Requirements	4247

A.10.4.4.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4248

A.10.4.4.4.1	Test Purpose and Environment	4248

A.10.4.4.4.2	Test Requirements	4253

A.10.5	Measurement performance	4254

A.10.5.1	SS-RSRP	4254

A.10.5.1.1	Intra-frequency measurement accuracy on a CCA serving cell	4254

A.10.5.1.1.1	Test Purpose and Environment	4254

A.10.5.1.1.2	Test parameters	4254

A.10.5.1.1.3	Test Requirements	4257

A.10.5.1.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	4258

A.10.5.1.2.1	Test Purpose and Environment	4258

A.10.5.1.2.2	Test parameters	4258

A.10.5.1.2.3	Test Requirements	4261

A.10.5.2	SS-RSRQ	4261

A.10.5.2.1	Intra-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	4261

A.10.5.2.1.1	Test Purpose and Environment	4261

A.10.5.2.1.2	Test Parameters	4261

A.10.5.2.1.3	Test Requirements	4265

A.10.5.2.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	4265

A.10.5.2.2.1	Test Purpose and Environment	4265

A.10.5.2.2.2	Test Parameters	4265

A.10.5.2.2.3	Test Requirements	4268

A.10.5.3	SS-SINR	4268

A.10.5.3.1	Intra-frequency measurement accuracy on PSCC	4268

A.10.5.3.1.1	Test Purpose and Environment	4268

A.10.5.3.1.2	Test Parameters	4268

A.10.5.3.1.3	Test Requirements	4271

A.10.5.3.2	Inter-frequency measurement accuracy on PSCC	4271

A.10.5.3.2.1	Test Purpose and Environment	4271

A.10.5.3.2.2	Test Parameters	4271

A.10.5.3.2.3	Test Requirements	4274

A.10.5.3.3	Intra-frequency measurement accuracy on SCC	4274

A.10.5.3.3.1	Test Purpose and Environment	4274

A.10.5.3.3.2	Test Parameters	4274

A.10.5.3.3.3	Test Requirements	4277

A.10.5.4	L1-RSRP measurement for beam reporting with CCA serving cell	4277

A.10.5.4.1	SSB based L1-RSRP measurement	4277

A.10.5.4.1.1	Test Purpose and Environment	4277

A.10.5.4.1.2	Test parameters	4278

A.10.5.4.1.3	Test Requirements	4280

A.10.5.5	RSSI	4280

A.10.5.5.1 	RSSI measurement accuracy on PSCC with CCA	4280

A.10.5.5.1.1	Test Purpose and Environment	4280

A.10.5.5.1.2	Test parameters	4280

A.10.5.5.1.3	Test Requirements	4283

A.10.5.5.2 	RSSI measurement accuracy on SCC with CCA	4283

A.10.5.5.2.1	Test Purpose and Environment	4283

A.10.5.5.2.2	Test parameters	4283

A.10.5.5.2.3	Test Requirements	4286

A.10.5.5.3 	Inter-frequency RSSI measurement accuracy on a carrier with CCA	4286

A.10.5.5.3.1	Test Purpose and Environment	4286

A.10.5.5.3.2	Test parameters	4286

A.10.5.5.3.3	Test Requirements	4289

A.10.5.6	Channel occupancy	4289

A.10.5.6.1 	Channel occupancy measurement accuracy on PSCC with CCA	4289

A.10.5.6.1.1	Test Purpose and Environment	4289

A.10.5.6.1.2	Test parameters	4289

A.10.5.6.1.3	Test Requirements	4293

A.10.5.6.2 	Channel occupancy measurement accuracy on SCC with CCA	4293

A.10.5.6.2.1	Test Purpose and Environment	4293

A.10.5.6.2.2	Test parameters	4293

A.10.5.6.2.3	Test Requirements	4296

A.10.5.6.3 	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	4296

A.10.5.6.3.1	Test Purpose and Environment	4296

A.10.5.6.3.2	Test parameters	4296

A.10.5.6.3.3	Test Requirements	4300

A.11	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	4301

A.11.1	RRC_IDLE state mobility	4301

A.11.1.1	Cell re-selection with both source and target NR carrier frequencies under CCA	4301

A.11.1.1.1	Cell reselection to FR1 intra-frequency NR cells when subject to CCA on the serving and target cell	4301

A.11.1.1.1.1	Test Purpose and Environment	4301

A.11.1.1.1.2	Test Parameters	4301

A.11.1.1.1.3	Test Requirements	4304

A.11.1.1.2	Cell reselection to FR1 inter-frequency NR case when subject to CCA on the serving and target cell	4304

A.11.1.1.2.1	Test Purpose and Environment	4304

A.11.1.1.2.2	Test Parameters	4304

A.11.1.1.2.3	Test Requirements	4308

A.11.1.2	Cell re-selection to NR with source NR carrier frequency under CCA	4308

A.11.1.2.1	Cell reselection to FR1 inter-frequency NR case when serving cell is subject to CCA	4308

A.11.1.2.1.1	Test Purpose and Environment	4308

A.11.1.2.1.2	Test Parameters	4309

A.11.1.2.1.3	Test Requirements	4315

A.11.1.3	Cell re-selection from NR carrier with target NR carrier frequency under CCA	4316

A.11.1.3.1	Cell reselection to FR1 inter-frequency NR case when target cell is subject to CCA	4316

A.11.1.3.1.1	Test Purpose and Environment	4316

A.11.1.3.1.2	Test Parameters	4316

A.11.1.3.1.3	Test Requirements	4322

A.11.1.4	Inter-RAT cell re-selection to E-UTRAN with source NR carrier frequency under CCA	4323

A.11.1.4.1	Cell reselection to higher priority E-UTRAN when serving cell is subject to CCA	4323

A.11.1.4.1.1	Test Purpose and Environment	4323

A.11.1.4.1.2	Test Parameters	4323

A.11.1.4.1.3	Test Requirements	4326

A.11.1.4.2	Cell reselection to lower priority E-UTRAN when serving cell is subject to CCA	4327

A.11.1.4.2.1	Test Purpose and Environment	4327

A.11.1.4.2.2	Test Requirements	4330

A.11.2	RRC_CONNECTED state mobility	4331

A.11.2.1	Handover	4331

A.11.2.1.1	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; known target cell	4331

A.11.2.1.1.1	Test Purpose and Environment	4331

A.11.2.1.1.2	Test Parameters	4331

A.11.2.1.1.3 Test Requirements	4334

A.11.2.1.2	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	4334

A.11.2.1.2.1	Test Purpose and Environment	4334

A.11.2.1.2.2	Test Parameters	4334

A.11.2.1.2.3	Test Requirements	4337

A.11.2.1.3	Inter-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	4337

A.11.2.1.3.1	Test Purpose and Environment	4337

A.11.2.1.3.2	Test Parameters	4337

A.11.2.1.3.3 Test Requirements	4340

A.11.2.1.4	Inter-frequency handover from FR1 carrier under CCA to FR1; known target cell	4340

A.11.2.1.4.1	Test Purpose and Environment	4340

A.11.2.1.4.2	Test Parameters	4340

A.11.2.1.4.3 Test Requirements	4344

A.11.2.1.5	Inter-frequency handover from FR1 carrier under CCA to FR1; unknown target cell	4345

A.11.2.1.5.1	Test Purpose and Environment	4345

A.11.2.1.5.2	Test Parameters	4345

A.11.2.1.5.3 Test Requirements	4348

A.11.2.1.6	Inter-frequency handover from FR1 to FR1 carrier under CCA; unknown target cell	4349

A.11.2.1.6.1	Test Purpose and Environment	4349

A.11.2.1.6.2	Test Parameters	4349

A.11.2.1.6.3 Test Requirements	4352

A.11.2.1.7	 SA NR FR1 carrier under CCA - E-UTRAN handover with known target cell	4353

A.11.2.1.7.1	Test Purpose and Environment	4353

A.11.2.1.7.2	Test Requirements	4359

A.11.2.1.8	SA NR FR1 carrier under CCA - E-UTRAN handover with unknown target cell	4359

A.11.2.1.8.1	Test Purpose and Environment	4359

A.11.2.1.8.2	Test Requirements	4364

A.11.2.1.9	Handover with PSCell from NR SA to EN-DC with known target PSCell using CCA	4364

A.11.2.1.9.1	Test Purpose and Environment	4364

A.11.2.1.9.2	Test Requirements	4373

A.11.2.2	RRC connection mobility control	4374

A.11.2.2.1	RRC re-establishment	4374

A.11.2.2.1.1	Intra-frequency RRC Re-establishment with CCA in FR1	4374

A.11.2.2.1.2	Inter-frequency RRC Re-establishment with CCA in FR1	4377

A.11.2.2.1.4	Inter-frequency RRC Re-establishment from NR FR1 carrier without CCA to NR FR1 carrier under CCA	4384

A.11.2.2.2	Random Access	4389

A.11.2.2.2.1	4-step RA type contention-based random access for NR PCell with CCA	4389

A.11.2.2.2.1.1	Test Purpose and Environment	4389

A.11.2.2.2.1.2	Test Requirements	4392

A.11.2.2.2.1.2.1	Random Access Preamble Transmission	4392

A.11.2.2.2.1.2.2	Random Access Response Reception	4392

A.11.2.2.2.1.2.3	No Random Access Response Reception	4393

A.11.2.2.2.1.2.4	Receiving an UL grant for msg3 retransmission	4393

A.11.2.2.2.1.2.5	Reception of an Incorrect Message over Temporary C-RNTI	4393

A.11.2.2.2.1.2.6	Reception of a Correct Message over Temporary C-RNTI	4394

A.11.2.2.2.1.2.7	Contention Resolution Timer expiry	4394

A.11.2.2.2.2	4-step RA type non-contention based random access for NR PSCell with CCA	4394

A.11.2.2.2.2.1	Test Purpose and Environment	4394

A.11.2.2.2.2.2	Test Requirements	4397

A.11.2.2.2.2.2.1	SSB-based Random Access Preamble Transmission	4397

A.11.2.2.2.2.2.2	Random Access Response Reception	4398

A.11.2.2.2.2.2.3	No Random Access Response Reception	4398

A.11.2.2.2.3	2-step RA type contention-based random access for NR PCell with CCA	4398

A.11.2.2.2.3.1	Test Purpose and Environment	4398

A.11.2.2.2.3.2	Test Requirements	4402

A.11.2.2.2.3.2.1	MsgA Transmission	4402

A.11.2.2.2.3.2.2	MsgB Reception	4402

A.11.2.2.2.3.2.3	No MsgB Reception	4403

A.11.2.2.2.4	2-step RA type non-contention-based random access for NR PCell with CCA	4403

A.11.2.2.2.4.1	Test Purpose and Environment	4403

A.11.2.2.2.4.2	Test Requirements	4407

A.11.2.2.2.4.2.1	MsgA Transmission	4407

A.11.2.2.2.4.2.2	MsgB Reception	4408

A.11.2.2.2.4.2.3	No MsgB Reception	4408

A.11.2.2.3	RRC connection release with redirection	4409

A.11.2.2.3.1	Redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA	4409

A.11.2.2.3.2	Redirection from NR FR1 carrier without CCA to NR FR1 carrier with CCA	4413

A.11.3	Timing	4418

A.11.3.1	UE transmit timing	4418

A.11.3.1.1	UE Transmit Timing Test with PCell under DL CCA	4418

A.11.3.1.1.1	Test Purpose and environment	4418

A.11.3.1.1.2	Test requirements	4421

A.11.3.2	UE timing advance	4422

A.11.3.2.1	UE Timing Advance Adjustment Accuracy with PCell under DL CCA	4422

A.11.3.2.1.1	Test Purpose and Environment	4422

A.11.3.2.1.2	Test Parameters	4422

A.11.3.2.1.3	Test Requirements	4426

A.11.4	Signalling characteristics	4426

A.11.4.1	Radio link monitoring	4426

A.11.4.1.1	Introduction	4426

A.11.4.1.2	Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	4427

A.11.4.1.2.1	Test purpose and environment	4427

A.11.4.1.2.2	Test requirements	4431

A.11.4.1.3	Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	4431

A.11.4.1.3.1	Test purpose and environment	4431

A.11.4.1.3.2	Test requirements	4437

A.11.4.1.4	Void	4437

A.11.4.1.4.1	Void	4437

A.11.4.1.4.2	Void	4437

A.11.4.1.5	Void	4437

A.11.4.1.5.1	Void	4437

A.11.4.1.5.2	Void	4437

A.11.4.2	Interruption	4438

A.11.4.2.1	NR interruptions during Scell operations with CCA on PCell and SCell	4438

A.11.4.2.1.1 Test Purpose and Environment	4438

A.11.4.2.1.2	Test Requirements	4442

A.11.4.3	SCell activation and deactivation delay	4442

A.11.4.3.1	SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 160 ms SCell measurement cycle	4442

A.11.4.3.1.1	Test Purpose and Environment	4442

A.11.4.3.1.2	Test Requirements	4447

A.11.4.3.2	SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 640 ms SCell measurement cycle	4447

A.11.4.3.2.1	Test Purpose and Environment	4447

A.11.4.3.2.2	Test Requirements	4448

A.11.4.3.3	SCell Activation and Deactivation of unknown SCell with PCell and SCell under CCA	4448

A.11.4.3.3.1	Test Purpose and Environment	4448

A.11.4.3.3.2	Test Requirements	4449

A.11.4.4	Beam failure detection and link recovery procedures	4449

A.11.4.4.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	4449

A.11.4.4.1.1	Test Purpose and Environment	4449

A.11.4.4.1.2	Test Requirements	4454

A.11.4.4.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	4454

A.11.4.4.2.1	Test Purpose and Environment	4454

A.11.4.4.2.2	Test Requirements	4460

A.11.4.5	Active BWP switching	4460

A.11.4.5.1	UL active BWP switch delay with consistent UL LBT failure on PCell subject to UL CCA	4460

A.11.4.5.1.1	Test Purpose and Environment	4460

A.11.4.5.1.2	Test Requirements	4464

A.11.4.5.2	DCI-based and Timer-based Active BWP Switch	4465

A.11.4.5.2.1	NR FR1- NR FR1 DL active BWP switch of PCell with non-DRX in SA	4465

A.11.4.5.2.2	NR FR1 DL active BWP switch with non-DRX in SA	4468

A.11.4.5.3	RRC-based Active BWP Switch	4471

A.11.4.5.3.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	4471

A.11.4.6	Void	4474

A.11.5	Measurement procedure	4474

A.11.5.1	Intra-frequency measurements	4474

A.11.5.1.1	Event-triggered reporting tests on PCC without gaps under non-DRX	4474

A.11.5.1.1.1	Test purpose and environment	4474

A.11.5.1.1.2	Test parameters	4474

A.11.5.1.1.3	Test Requirements	4477

A.11.5.1.2	Event-triggered reporting tests on PCC without gaps under DRX	4477

A.11.5.1.2.1	Test purpose and environment	4477

A.11.5.1.2.2	Test parameters	4477

A.11.5.1.2.3	Test Requirements	4480

A.11.5.1.3	Event-triggered reporting tests on PCC with per-UE gaps under non-DRX	4480

A.11.5.1.3.1	Test purpose and environment	4480

A.11.5.1.3.2	Test parameters	4480

A.11.5.1.3.3	Test Requirements	4480

A.11.5.1.4	Event-triggered reporting tests on PCC with per-UE gaps under DRX	4481

A.11.5.1.4.1	Test purpose and environment	4481

A.11.5.1.4.2	Test parameters	4481

A.11.5.1.4.3	Test Requirements	4481

A.11.5.1.5	Event-triggered reporting tests on SCC without gaps under non-DRX	4481

A.11.5.1.5.1	Test purpose and environment	4481

A.11.5.1.5.2	Test parameters	4481

A.11.5.1.6	Event-triggered reporting tests on SCC without gaps under DRX	4482

A.11.5.1.6.1	Test purpose and environment	4482

A.11.5.1.6.2	Test parameters	4482

A.11.5.1.6.3	Test Requirements	4482

A.11.5.1.7	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	4482

A.11.5.1.7.1	Test purpose and environment	4482

A.11.5.1.7.2	Test parameters	4482

A.11.5.1.7.3	Test Requirements	4482

A.11.5.1.8	Event-triggered reporting tests on SCC with per-UE gaps under DRX	4483

A.11.5.1.8.1	Test purpose and environment	4483

A.11.5.1.8.2	Test parameters	4483

A.11.5.1.8.3	Test Requirements	4483

A.11.5.1.9	RSSI measurement reporting on PCC	4483

A.11.5.1.9.1	Test purpose and environment	4483

A.11.5.1.9.2	Test parameters	4483

A.11.5.1.10	Channel occupancy measurement reporting on PCC	4484

A.11.5.1.10.1	Test purpose and environment	4484

A.11.5.1.10.2	Test parameters	4484

A.11.5.1.11	RSSI measurement reporting on SCC	4484

A.11.5.1.11.1	Test purpose and environment	4484

A.11.5.1.11.2	Test parameters	4484

A.11.5.1.12	Channel occupancy measurement reporting on SCC	4485

A.11.5.1.12.1	Test purpose and environment	4485

A.11.5.1.12.2	Test parameters	4485

A.11.5.2	Inter-frequency measurements	4485

A.11.5.2.1	RSSI measurement reporting	4485

A.11.5.2.1.1	Test purpose and environment	4485

A.11.5.2.1.2	Test parameters	4485

A.11.5.2.2	Channel occupancy measurement reporting	4486

A.11.5.2.2.1	Test purpose and environment	4486

A.11.5.2.2.2	Test parameters	4486

A.11.5.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	4486

A.11.5.2.3.1	Test Purpose and Environment	4486

A.11.5.2.3.2	Test Requirements	4490

A.11.5.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	4491

A.11.5.2.4.1	Test Purpose and Environment	4491

A.11.5.2.4.2	Test Requirements	4496

A.11.5.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	4496

A.11.5.2.5.1	Test Purpose and Environment	4496

A.11.5.2.5.2	Test Requirements	4500

A.11.5.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	4501

A.11.5.2.6.1	Test Purpose and Environment	4501

A.11.5.2.6.2	Test Requirements	4506

A.11.5.2.7	Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4506

A.11.5.2.7.1	Test Purpose and Environment	4506

A.11.5.2.7.2	Test Requirements	4510

A.11.5.2.8	Event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4511

A.11.5.2.8.1	Test Purpose and Environment	4511

A.11.5.2.8.2	Test Requirements	4516

A.11.5.2.9	Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4516

A.11.5.2.9.1	Test Purpose and Environment	4516

A.11.5.2.9.2	Test Requirements	4521

A.11.5.2.10	Event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4522

A.11.5.2.10.1	Test Purpose and Environment	4522

A.11.5.2.10.2	Test Requirements	4527

A.11.5.3	Inter-RAT E-UTRAN measurements	4528

A.11.5.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1	4528

A.11.5.3.1.1	Test Purpose and Environment	4528

A.11.5.3.1.2	Test Requirements	4532

A.11.5.3.2	SA NR - E-UTRAN event-triggered reporting in DRX in FR1	4532

A.11.5.3.2.1	Test Purpose and Environment	4532

A.11.5.3.2.2	Test Requirements	4537

A.11.5.4	L1-RSRP measurements for beam reporting	4537

A.11.5.4.1	SSB based L1-RSRP measurement when DRX is not used	4537

A.11.5.4.1.1	Test Purpose and Environment	4537

A.11.5.4.1.2	Test parameters	4537

A.11.5.4.1.3	Test Requirements	4539

A.11.5.4.2	SSB based L1-RSRP measurement when DRX is used	4539

A.11.5.4.2.1	Test Purpose and Environment	4539

A.11.5.4.2.2	Test parameters	4540

A.11.5.4.2.3	Test Requirements	4542

A.11.5.4.3	SSB based L1-RSRP measurement on SCC when DRX is not used	4542

A.11.5.4.3.1	Test Purpose and Environment	4542

A.11.5.4.3.2	Test parameters	4543

A.11.5.4.3.3	Test Requirements	4546

A.11.5.4.4	SSB based L1-RSRP measurement on SCC when DRX is used	4547

A.11.5.4.4.1	Test Purpose and Environment	4547

A.11.5.4.4.2	Test parameters	4547

A.11.5.4.4.3	Test Requirements	4550

A.11.6	Measurement performance	4551

A.11.6.1	SS-RSRP	4551

A.11.6.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	4551

A.11.6.1.1.1	Test Purpose and Environment	4551

A.11.6.1.1.2	Test parameters	4551

A.11.6.1.1.3	Test Requirements	4553

A.11.6.1.2	Intra-frequency measurement accuracy on SCC on a carrier frequency with CCA	4553

A.11.6.1.2.1	Test Purpose and Environment	4553

A.11.6.1.2.2	Test parameters	4553

A.11.6.1.2.3	Test Requirements	4555

A.11.6.2	SS-RSRQ	4555

A.11.6.2.1	Intra-frequency measurement accuracy	4555

A.11.6.2.1.1	Test Purpose and Environment	4555

A.11.6.2.1.2	Test Parameters	4555

A.11.6.2.1.3	Test Requirements	4558

A.11.6.2.2	Inter-frequency measurement accuracy	4558

A.11.6.2.2.1	Test Purpose and Environment	4558

A.11.6.2.2.2	Test Parameters	4558

A.11.6.2.2.3	Test Requirements	4561

A.11.6.2.3	Intra-frequency measurement accuracy on SCC	4561

A.11.6.2.3.1	Test Purpose and Environment	4561

A.11.6.2.3.2	Test Parameters	4561

A.11.6.2.3.3	Test Requirements	4564

A.11.6.2.4	Inter-frequency measurement accuracy	4564

A.11.6.2.4.1	Test Purpose and Environment	4564

A.11.6.2.4.2	Test Parameters	4564

A.11.6.2.4.3	Test Requirements	4573

A.11.6.3	SS-SINR	4573

A.11.6.3.1	Intra-frequency measurement accuracy	4573

A.11.6.3.1.1	Test Purpose and Environment	4573

A.11.6.3.1.2	Test Parameters	4573

A.11.6.3.1.3	Test Requirements	4576

A.11.6.3.2	Inter-frequency measurement accuracy	4576

A.11.6.3.2.1	Test Purpose and Environment	4576

A.11.6.3.2.2	Test Parameters	4576

A.11.6.3.2.3	Test Requirements	4579

A.11.6.3.3	Intra-frequency measurement accuracy on SCC	4579

A.11.6.3.3.1	Test Purpose and Environment	4579

A.11.6.3.3.2	Test Parameters	4579

A.11.6.3.3.3	Test Requirements	4582

A.11.6.3.4	Inter-frequency measurement accuracy	4582

A.11.6.3.4.1	Test Purpose and Environment	4582

A.11.6.3.4.2	Test Parameters	4582

A.11.6.3.4.3	Test Requirements	4590

A.11.6.4	L1-RSRP measurement for beam reporting with CCA serving cell	4590

A.11.6.4.1	SSB based L1-RSRP measurement	4590

A.11.6.4.1.1	Test Purpose and Environment	4590

A.11.6.4.1.2	Test parameters	4591

A.11.6.4.1.3	Test Requirements	4594

A.11.6.5	RSSI	4594

A.11.6.5.1 	Intra-frequency RSSI measurement accuracy on PCC with CCA	4594

A.11.6.5.1.1	Test Purpose and Environment	4594

A.11.6.5.1.2	Test parameters	4594

A.11.6.5.1.3	Test Requirements	4597

A.11.6.5.2 	Intra-frequency RSSI measurement accuracy on SCC with CCA	4597

A.11.6.5.2.1	Test Purpose and Environment	4597

A.11.6.5.2.2	Test parameters	4597

A.11.6.5.2.3	Test Requirements	4600

A.11.6.5.3	Inter-frequency RSSI measurement accuracy on a carrier with CCA	4600

A.11.6.5.3.1	Test Purpose and Environment	4600

A.11.6.5.3.2	Test parameters	4600

A.11.6.5.3.3	Test Requirements	4603

A.11.6.6	Channel occupancy	4603

A.11.6.6.1	Intra-frequency channel occupancy measurement accuracy on PCC with CCA	4603

A.11.6.6.1.1	Test Purpose and Environment	4603

A.11.6.6.1.2	Test parameters	4603

A.11.6.6.1.3	Test Requirements	4606

A.11.6.6.2	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	4606

A.11.6.6.2.1	Test Purpose and Environment	4606

A.11.6.6.2.2	Test parameters	4606

A.11.6.6.2.3	Test Requirements	4609

A.11.6.6.3	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	4609

A.11.6.6.3.1	Test Purpose and Environment	4609

A.11.6.6.3.2	Test parameters	4609

A.11.6.6.3.3	Test Requirements	4612

A.11.6.7	E-UTRAN RSRP	4612

A.11.6.8	E-UTRAN RSRQ	4612

A.11.6.9	E-UTRAN SINR	4612

A.12	E-UTRA Standalone Tests with at Least One NR Cell under CCA	4612

A.12.1	RRC_IDLE state mobility	4612

A.12.1.1	Inter-RAT cell re-selection to NR on a carrier frequency with CCA	4612

A.12.1.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1 when target cell is subject to CCA	4612

A.12.1.1.1.1	Test Purpose and Environment	4612

A.12.1.1.1.2	Test Requirements	4617

A.12.2	RRC_CONNECTED state mobility	4618

A.12.2.1	Handover	4618

A.12.2.1.1	E-UTRAN - NR with CCA handover	4618

A.12.2.1.1.1	Test Purpose and Environment	4618

A.12.2.1.1.2	Test Requirements	4623

A.12.3	Signalling characteristics	4624

A.12.3.1	Interruptions	4624

A.12.4	Measurement procedure	4624

A.12.4.1	E-UTRANNR inter-RAT SFTD measurements	4624

A.12.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay with NR under CCA in non-DRX	4624

A.12.4.1.1.1	Test Purpose and Environment	4624

A.12.4.1.1.2	Test Requirements	4628

A.12.4.2	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	4628

A.12.4.2.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4628

A.12.4.2.1.1	Test Purpose and Environment	4628

A.12.4.2.1.2	Test Requirements	4632

A.12.4.2.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4633

A.12.4.2.2.1	Test Purpose and Environment	4633

A.12.4.2.2.2	Test Requirements	4637

A.12.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4638

A.12.4.2.3.1	Test Purpose and Environment	4638

A.12.4.2.3.2	Test Requirements	4642

A.12.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4643

A.12.4.2.4.1	Test Purpose and Environment	4643

A.12.4.2.4.2	Test Requirements	4647

A.12.4.2.5	RSSI measurement reporting	4648

A.12.4.2.5.1	Test purpose and environment	4648

A.12.4.2.5.2	Test parameters	4648

A.12.4.2.5.3	Test Requirements	4651

A.12.4.2.6	Channel occupancy measurement reporting	4651

A.12.4.2.6.1	Test purpose and environment	4651

A.12.4.2.6.2	Test parameters	4651

A.12.5	Measurement performance	4654

A.12.5.1	E-UTRANNR SFTD	4654

A.12.5.1.1	Inter-RAT SFTD accuracy with NR target cell under CCA	4654

A.12.5.1.1.1	Test Purpose	4654

A.12.5.1.1.2	Test Environment	4654

A.12.5.1.1.3	Test Requirements	4659

A.12.5.2	E-UTRANNR SS-RSRP	4659

A.12.5.3	E-UTRANNR SS-RSRQ	4659

A.12.5.4	E-UTRANNR SS-SINR	4659

A.12.5.5	E-UTRANNR RSSI	4659

A.12.5.6	E-UTRANNR channel occupancy	4659

A.13	NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	4660

A.13.1	Timing	4660

A.13.1.1	UE transmit timing	4660

A.13.1.2	Timing advance	4660

A.13.2	Signalling characteristics	4660

A.13.2.1	Interruption	4660

A.13.2.1.1	NR interruptions during SCell operations with CCA on SCell	4660

A.13.2.1.1.1	Test Purpose and Environment	4660

A.13.2.1.1.2	Test Requirements	4664

A.13.2.2	SCell activation and deactivation delay	4664

A.13.2.2.1	SCell Activation and Deactivation of known SCell under CCA, 160 ms SCell measurement cycle	4664

A.13.2.2.1.1	Test Purpose and Environment	4664

A.13.2.2.1.2	Test Requirements	4669

A.13.2.2.2 SCell Activation and Deactivation of known SCell under CCA, 640 ms SCell measurement cycle	4669

A.13.2.2.2.1	Test Purpose and Environment	4669

A.13.2.2.2.2	Test Requirements	4670

A.13.2.2.3	SCell Activation and Deactivation of unknown SCell under CCA	4670

A.13.2.2.3.1	Test Purpose and Environment	4670

A.13.2.2.3.2	Test Requirements	4671

A.13.2.3	Void	4671

A.13.3	Measurement procedure	4671

A.13.3.1	Intra-frequency measurements	4671

A.13.3.1.1	Event-triggered reporting tests on SCC without gaps under non-DRX	4671

A.13.3.1.1.1	Test purpose and environment	4671

A.13.3.1.1.2	Test parameters	4671

A.13.3.1.1.3	Test Requirements	4676

A.13.3.1.2	Event-triggered reporting tests on SCC without gaps under DRX	4676

A.13.3.1.2.1	Test purpose and environment	4676

A.13.3.1.2.2	Test parameters	4676

A.13.3.1.2.3	Test Requirements	4681

A.13.3.1.3	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	4681

A.13.3.1.3.1	Test purpose and environment	4681

A.13.3.1.3.2	Test parameters	4681

A.13.3.1.3.3	Test Requirements	4686

A.13.3.1.4	Event-triggered reporting tests on SCC with per-UE gaps under DRX	4686

A.13.3.1.4.1	Test purpose and environment	4686

A.13.3.1.4.2	Test parameters	4686

A.13.3.1.4.3	Test Requirements	4691

A.13.3.1.5	RSSI measurement reporting on SCC	4691

A.13.3.1.5.1	Test purpose and environment	4691

A.13.3.1.5.2	Test parameters	4691

A.13.3.1.6	Channel occupancy measurement reporting on SCC	4692

A.13.3.1.6.1	Test purpose and environment	4692

A.13.3.1.6.2	Test parameters	4693

A.13.3.2	Inter-frequency measurements	4693

A.13.3.2.1	RSSI measurement reporting	4693

A.13.3.2.1.1	Test purpose and environment	4693

A.13.3.2.1.2	Test parameters	4693

A.13.3.2.2	Channel occupancy measurement reporting	4694

A.13.3.2.2.1	Test purpose and environment	4694

A.13.3.2.2.2	Test parameters	4694

A.13.3.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	4694

A.13.3.2.3.1	Test Purpose and Environment	4694

A.13.3.2.3.2	Test Requirements	4699

A.13.3.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	4699

A.13.3.2.4.1	Test Purpose and Environment	4699

A.13.3.2.4.2	Test Requirements	4704

A.13.3.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	4705

A.13.3.2.5.1	Test Purpose and Environment	4705

A.13.3.2.5.2	Test Requirements	4710

A.13.3.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	4710

A.13.3.2.6.1	Test Purpose and Environment	4710

A.13.3.2.6.2	Test Requirements	4716

A.13.3.3	L1-RSRP measurements for beam reporting	4717

A.13.3.3.1	SSB based L1-RSRP measurement when DRX is not used	4717

A.13.3.3.1.1	Test Purpose and Environment	4717

A.13.3.3.1.2	Test parameters	4717

A.13.3.3.1.3	Test Requirements	4721

A.13.3.3.2	SSB based L1-RSRP measurement when DRX is used	4721

A.13.3.3.2.1	Test Purpose and Environment	4721

A.13.3.3.2.2	Test parameters	4721

A.13.3.3.2.3	Test Requirements	4725

A.13.4	Measurement performance	4725

A.13.4.1	SS-RSRP	4725

A.13.4.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	4725

A.13.4.1.1.1	Test Purpose and Environment	4725

A.13.4.1.1.2	Test parameters	4725

A.13.4.1.1.3	Test Requirements	4727

A.13.4.2	SS-RSRQ	4727

A.13.4.2.1	Intra-frequency measurement accuracy on SCC	4727

A.13.4.2.1.1	Test Purpose and Environment	4727

A.13.4.2.1.2	Test Parameters	4727

A.13.4.2.1.3	Test Requirements	4735

A.13.4.3	SS-SINR	4735

A.13.4.3.1	Intra-frequency measurement accuracy on SCC	4735

A.13.4.3.1.1	Test Purpose and Environment	4735

A.13.4.3.1.2	Test Parameters	4735

A.13.4.3.1.3	Test Requirements	4742

A.13.4.4	L1-RSRP measurement for beam reporting with CCA serving cell	4742

A.13.4.4.1	SSB based L1-RSRP measurement	4742

A.13.4.4.1.1	Test Purpose and Environment	4742

A.13.4.4.1.2	Test parameters	4742

A.13.4.4.1.3	Test Requirements	4746

A.13.4.5	RSSI	4746

A.13.4.5.1 	Intra-frequency RSSI measurement accuracy on a carrier with CCA	4746

A.13.4.5.1.1	Test Purpose and Environment	4746

A.13.4.5.1.2	Test parameters	4746

A.13.4.5.1.3	Test Requirements	4749

A.13.4.5.2	Inter-frequency RSSI measurement accuracy on a carrier with CCA	4749

A.13.4.5.2.1	Test Purpose and Environment	4749

A.13.4.5.2.2	Test parameters	4749

A.13.4.5.2.3	Test Requirements	4753

A.13.4.6	Channel occupancy	4753

A.13.4.6.1	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	4753

A.13.4.6.1.1	Test Purpose and Environment	4753

A.13.4.6.1.2	Test parameters	4753

A.13.4.6.1.3	Test Requirements	4757

A.13.4.6.2	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	4757

A.13.4.6.2.1	Test Purpose and Environment	4757

A.13.4.6.2.2	Test parameters	4757

A.13.4.6.2.3	Test Requirements	4761

A.14	NR standalone tests for Satellite access	4761

A.14.1	RRC_IDLE state mobility	4761

A.14.1.1	Cell reselection to FR1 intra-frequency NR case	4761

A.14.1.1.1	Test Purpose and Environment	4761

A.14.1.1.2	Test Parameters	4761

A.14.1.1.3	Test Requirements	4763

A.14.1.2	Cell reselection to FR1 intra-frequency NR cell for UE configured with the feature for enhanced requirements	4764

A.14.1.2.1	Test Purpose and Environment	4764

A.14.1.2.2	Test Parameters	4764

A.14.1.2.3	Test Requirements	4768

A.14.1.3	Time-based measurement initiation to FR1 intra-frequency NR cell reselection	4769

A.14.1.3.1	Test Purpose and Environment	4769

A.14.1.3.2	Test Parameters	4769

A.14.1.3.3	Test Requirements	4770

A.14.1.4	Location-based cell measurement initiation to FR1 intra-frequency NR cell reselection	4771

A.14.1.4.1	Test Purpose and Environment	4771

A.14.1.4.2	Test Parameters	4771

A.14.1.4.3	Test Requirements	4773

A.14.1.5	Cell reselection to FR1 inter-frequency NR case	4774

A.14.1.5.1	Test Purpose and Environment	4774

A.14.1.5.2	Test Parameters	4774

A.14.1.5.3	Test Requirements	4776

A.14.1.6	Cell reselection to FR1 inter-frequency NR cell for UE configured with [capability for enhanced requirements]	4777

A.14.1.6.1	Test Purpose and Environment	4777

A.14.1.6.2	Test Parameters	4777

A.14.1.6.3	Test Requirements	4779

A.14.1.7	Time-based Cell measurement initiation to FR1 inter-frequency cell reselection	4780

A.14.1.7.1	Test Purpose and Environment	4780

A.14.1.7.2	Test Parameters	4780

A.14.1.7.3	Test Requirements	4781

A.14.1.8	Location-based measurement initiation to FR1 inter-frequency NR cell reselection	4782

A.14.1.8.1	Test Purpose and Environment	4782

A.14.1.8.2	Test Parameters	4782

A.14.1.8.3	Test Requirements	4784

A.14.1.9	Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	4785

A.14.1.9.1	Test Purpose and Environment	4785

A.14.1.9.2	Test Parameters	4785

A.14.1.9.3	Test Requirements	4788

A.14.1.10	Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	4788

A.14.1.10.1	Test Purpose and Environment	4788

A.14.1.10.2	Test Parameters	4788

A.14.1.10.3	Test Requirements	4790

A.14.2	RRC_CONNECTED state mobility	4791

A.14.2.1	Handover	4791

A.14.2.1.1	Intra-frequency SAN Handover from FR1 to FR1	4791

A.14.2.1.1.1	Test Purpose and Environment	4791

A.14.2.1.1.2	Test Parameters	4791

A.14.2.1.1.3	Test Requirements	4793

A.14.2.1.2	Inter-frequency SAN Handover from FR1 to FR1	4794

A.14.2.1.2.1	Test Purpose and Environment	4794

A.14.2.1.2.2	Test Parameters	4794

A.14.2.1.2.3	Test Requirements	4796

A.14.2.1.3	Intra-frequency SAN time-based conditional Handover from FR1 to FR1	4796

A.14.2.1.3.1	Test Purpose and Environment	4796

A.14.2.1.3.2	Test Parameters	4796

A.14.2.1.3.3	Test Requirements	4799

A.14.2.1.4	Inter-frequency SAN time-based conditional Handover from FR1 to FR1	4799

A.14.2.1.4.1	Test Purpose and Environment	4799

A.14.2.1.4.2	Test Parameters	4799

A.14.2.1.4.3	Test Requirements	4801

A.14.2.1.5	Intra-frequency SAN distance-based conditional Handover from FR1 to FR1	4801

A.14.2.1.5.1	Test Purpose and Environment	4801

A.14.2.1.5.2	Test Parameters	4802

A.14.2.1.5.3	Test Requirements	4805

A.14.2.1.6	Inter-frequency SAN distance-based conditional Handover from FR1 to FR1	4805

A.14.2.1.6.1	Test Purpose and Environment	4805

A.14.2.1.6.2	Test Parameters	4805

A.14.2.1.6.3	Test Requirements	4809

A.14.2.2	RRC Connection Mobility Control	4809

A.14.2.2.1	SA: RRC Re-establishment for SAN	4809

A.14.2.2.1.1	Intra-frequency RRC Re-establishment in FR1	4809

A.14.2.2.1.2	Inter-frequency RRC Re-establishment in FR1	4812

A.14.2.2.2	Random Access	4815

A.14.2.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone	4815

A.14.2.2.2.1.1	Test Purpose and Environment	4815

A.14.2.2.2.1.2	Test Requirements	4818

A.14.2.2.2.2	4-step RA type non-contention based random access test in FR1 for NR standalone	4819

A.14.2.2.2.2.1	Test Purpose and Environment	4819

A.14.2.2.2.2.2	Test Requirements	4823

A.14.2.2.3	RRC Connection Release with Redirection	4824

A.14.2.2.3.1	Redirection from NR in FR1 to NR in FR1	4824

A.14.2.2.3.1.1	Test Purpose and Environment	4824

A.14.2.2.3.1.2	Test Parameters	4824

A.14.2.2.3.1.3	Test Requirements	4827

A.14.3	Timing for Satellite Access	4827

A.14.3.1	UE transmit timing for Satellite Access	4827

A.14.3.1.1	NR UE Transmit Timing Test for FR1	4827

A.14.3.1.1.1	Test Purpose and environment	4827

A.14.3.1.1.2	Test requirements	4829

A.14.3.2	Timing advance for satellite access	4830

A.14.3.2.1	SA FR1 timing advance adjustment accuracy	4830

A.14.3.2.1.1	Test Purpose and Environment	4830

A.14.3.2.1.2	Test Parameters	4830

A.14.3.2.1.3	Test Requirements	4833

A.14.4	Signalling characteristics	4833

A.14.4.1	Radio link Monitoring	4833

A.14.4.1.1	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in non-DRX mode	4833

A.14.4.1.1.1	Test Purpose and Environment	4833

A.14.4.1.1.2	Test Requirements	4838

A.14.4.1.2	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in non-DRX mode	4838

A.14.4.1.2.1	Test Purpose and Environment	4838

A.14.4.1.2.2	Test Requirements	4844

A.14.4.1.3	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in DRX mode	4844

A.14.4.1.3.1	Test Purpose and Environment	4844

A.14.4.1.3.2	Test Requirements	4848

A.14.4.1.4	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in DRX mode	4848

A.14.4.1.4.1	Test Purpose and Environment	4848

A.14.4.1.4.2	Test Requirements	4853

A.14.4.1.5	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in non-DRX mode	4853

A.14.4.1.5.1	Test Purpose and Environment	4853

A.14.4.1.5.2	Test Requirements	4858

A.14.4.1.6	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in non-DRX mode	4858

A.14.4.1.6.1	Test Purpose and Environment	4858

A.14.4.1.6.2	Test Requirements	4862

A.14.4.1.7	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in DRX mode	4862

A.14.4.1.7.1	Test Purpose and Environment	4862

A.14.4.1.7.2	Test Requirements	4867

A.14.4.1.8	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in DRX mode	4867

A.14.4.1.8.1	Test Purpose and Environment	4867

A.14.4.1.8.2	Test Requirements	4872

A.14.4.2	Beam Failure Detection and Link recovery procedures for satellite access	4872

A.14.4.2.1	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with SSB-based BFD and LR in non-DRX mode	4872

A.14.4.2.1.1	Test Purpose and Environment	4872

A.14.4.2.1.2	Test Requirements	4878

A.14.4.2.2	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with SSB-based BFD and LR in DRX mode	4878

A.14.4.2.2.1	Test Purpose and Environment	4878

A.14.4.2.2.2	Test Requirements	4883

A.14.4.2.3	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with CSI-RS-based BFD and LR in non-DRX mode	4883

A.14.4.2.3.1	Test Purpose and Environment	4883

A.14.4.2.3.2	Test Requirements	4889

A.14.4.2.4	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with CSI-RS-based BFD and LR in DRX mode	4889

A.14.4.2.4.1	Test Purpose and Environment	4889

A.14.4.2.4.2	Test Requirements	4894

A.14.4.2.5	Beam Failure Detection and Link Recovery Test for FR1 SCell for satellite access configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	4894

A.14.4.2.5.1	Test Purpose and Environment	4894

A.14.4.2.5.2	Test Requirements	4899

A.14.4.2.6	Beam Failure Detection and Link Recovery Test for FR1 SCell for satellite access configured with CSI-RS-based BFD and SSB-based LR in DRX mode	4899

A.14.4.2.6.1	Test Purpose and Environment	4899

A.14.4.2.6.2	Test Requirements	4904

A.14.4.3	Active BWP switch for satellite access	4904

A.14.4.3.1	DCI-based and Timer-based Active BWP Switch	4904

A.14.4.3.1.1	NR FR1 DL active BWP switch with non-DRX in SA	4904

A.14.4.3.1.1.1	Test Purpose and Environment	4904

A.14.4.3.1.1.2	Test Requirements	4909

A.14.4.3.2	RRC-based Active BWP Switch	4909

A.14.4.3.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	4909

A.14.4.3.2.1.1	Test Purpose and Environment	4909

A.14.4.3.2.1.2	Test Requirements	4913

A.14.4.4	UE specific CBW change for satellite access	4913

A.14.4.4.1	UE specific CBW change on PCell in FR1 in non-DRX	4913

A.14.4.4.1.1	Test Purpose and Environment	4913

A.14.4.4.1.2	Test Requirements	4917

A.14.4.5	Pathloss reference signal switching delay	4917

A.14.4.5.1	MAC-CE based pathloss reference signal switch delay	4917

A.14.4.5.1.1	Test Purpose and Environment	4917

A.14.4.5.1.2	Test Requirements	4919

A.14.5	Measurement procedure	4920

A.14.5.1	Intra-frequency Measurements	4920

A.14.5.1.1	SA event triggered reporting tests without gap under non-DRX	4920

A.14.5.1.1.1	Test purpose and Environment	4920

A.14.5.1.1.2	Test parameters	4920

A.14.5.1.1.3	Test Requirements	4922

A.14.5.1.2	SA event triggered reporting tests without gap under DRX	4923

A.14.5.1.2.1	Test purpose and Environment	4923

A.14.5.1.2.2	Test parameters	4923

A.14.5.1.2.3	Test Requirements	4924

A.14.5.1.3	SA event triggered reporting tests without gap under non-DRX with SSB index reading	4925

A.14.5.1.3.1	Test purpose and Environment	4925

A.14.5.1.3.2	Test parameters	4925

A.14.5.1.3.3	Test Requirements	4927

A.14.5.1.4	SA event triggered reporting tests with single measurement gap under non-DRX for satellite access	4928

A.14.5.1.4.1	Test purpose and Environment	4928

A.14.5.1.4.2	Test parameters	4928

A.14.5.1.4.3	Test Requirements	4930

A.14.5.1.5	SA event triggered reporting tests with FNO concurrent gaps under DRX for satellite access	4931

A.14.5.1.5.1	Test purpose and Environment	4931

A.14.5.1.5.2	Test parameters	4931

A.15.5.1.5.3	Test Requirements	4933

A.14.5.1.6	SA event triggered reporting tests with PPO concurrent gaps under non-DRX with SSB index reading for satellite access	4934

A.14.5.1.6.1	Test purpose and Environment	4934

A.14.5.1.6.2	Test parameters	4934

A.14.5.1.6.3	Test Requirements	4936

A.14.5.2	Inter-frequency Measurements	4937

A.14.5.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with single gap for satellite access	4937

A.14.5.2.1.1	Test Purpose and Environment	4937

A.14.5.2.1.2	Test Requirements	4941

A.14.5.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used with single gap for satellite access	4941

A.14.5.2.2.1	Test Purpose and Environment	4941

A.14.5.2.2.2	Test Requirements	4945

A.14.5.2.3	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used with single gap for satellite access	4945

A.14.5.2.3.1	Test Purpose and Environment	4945

A.14.5.2.3.2	Test Requirements	4949

A.14.5.2.4	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with two gaps in fully non-overlapped for satellite access	4949

A.14.5.2.4.1	Test Purpose and Environment	4949

A.14.5.2.4.2	Test Requirements	4953

A.14.5.2.5	void	4953

A.14.5.2.5.1	void	4953

A.14.5.2.5.2	void	4953

A.14.5.2.6	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with two gaps in partially partial overalpping for satellite access	4953

A.14.5.2.6.1	Test Purpose and Environment	4953

A.14.5.2.6.2	Test Requirements	4957

A.14.5.2.7	Event triggered reporting test without gap under non-DRX	4957

A.14.5.2.7.1	Test purpose and Environment	4957

A.14.5.2.7.2	Test parameters	4957

A.14.5.2.7.3	Test Requirements	4959

A.14.5.2.8	Event triggered reporting tests without gap under DRX	4960

A.14.5.2.8.1	Test purpose and Environment	4960

A.14.5.2.8.2	Test parameters	4960

A.14.5.2.8.3	Test Requirements	4961

A.14.5.3	L1-RSRP measurement for beam reporting for satellite access	4962

A.14.5.3.1	SSB based L1-RSRP measurement for satellite access when DRX is not used	4962

A.14.5.3.1.1	Test Purpose and Environment	4962

A.14.5.3.1.2	Test parameters	4962

A.14.5.3.1.3	Test Requirements	4965

A.14.5.3.2	SSB based L1-RSRP measurement for satellite access when DRX is used	4965

A.14.5.3.2.1	Test Purpose and Environment	4965

A.14.5.3.2.2	Test parameters	4965

A.14.5.3.2.3	Test Requirements	4969

A.14.5.3.3	CSI-RS based L1-RSRP measurement for satellite access when DRX is not used	4969

A.14.5.3.3.1	Test Purpose and Environment	4969

A.14.5.3.3.2	Test parameters	4969

A.14.5.3.3.3	Test Requirements	4973

A.14.5.3.4	CSI-RS based L1-RSRP measurement for satellite access when DRX is used	4973

A.14.5.3.4.1	Test Purpose and Environment	4973

A.14.5.3.4.2	Test parameters	4974

A.14.5.3.4.3	Test Requirements	4977

A.14.6	Measurement Performance requirements	4977

A.14.6.1	 SS-RSRP for SAN	4977

A.14.6.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	4977

A.14.6.1.1.1	Test Purpose and Environment	4977

A.14.6.1.1.2	Test parameters	4977

A.14.6.1.1.3	Test Requirements	4981

A.14.6.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	4981

A.14.6.1.2.1	Test Purpose and Environment	4981

A.14.6.1.2.2	Test parameters	4981

A.14.6.1.2.3	Test Requirements	4984

A.14.6.2	SS-RSRQ	4984

A.14.6.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for satellite access	4984

A.14.6.2.1.1	Test Purpose and Environment	4984

A.14.6.2.1.2	Test Parameters	4984

A.14.6.2.1.3	Test Requirements	4987

A.14.6.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for satellite access	4987

A.14.6.2.2.1	Test Purpose and Environment	4987

A.14.6.2.2.2	Test Parameters	4987

A.14.6.2.2.3	Test Requirements	4990

A.14.6.3	SS-SINR	4990

A.14.6.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	4990

A.14.6.3.1.1	Test Purpose and Environment	4990

A.14.6.3.1.2	Test Parameters	4990

A.14.6.3.1.3	Test Requirements	4994

A.14.6.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	4994

A.14.6.3.2.1	Test Purpose and Environment	4994

A.14.6.3.2.2	Test Parameters	4994

A.14.6.3.2.3	Test Requirements	4998

A.14.6.4	L1-RSRP measurement for beam reporting	4998

A.14.6.4.1	SSB based L1-RSRP measurement	4998

A.14.6.4.1.1	Test Purpose and Environment	4998

A.14.6.4.1.2	Test parameters	4998

A.14.6.4.1.3	Test Requirements	5001

A.14.6.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	5002

A.14.6.4.2.1	Test Purpose and Environment	5002

A.14.6.4.2.2	Test parameters	5002

A.14.6.4.2.3	Test Requirements	5005

A.15	NR standalone tests with one or more NR cells in FR2-2	5006

A.15.1	SA: RRC_IDLE state mobility	5006

A.15.1.1	Cell re-selection to NR	5006

A.15.1.1.1	Cell reselection to FR2-2 intra-frequency NR case	5006

A.15.1.1.1.1	Test Purpose and Environment	5006

A.15.1.1.1.2	Test Parameters	5006

A.15.1.1.1.3	Test Requirements	5010

A.15.1.2	Cell reselection to FR2-2 inter-frequency NR case	5010

A.15.1.2.1	Test Purpose and Environment	5010

A.15.1.2.2	Test Parameters	5010

A.15.1.2.3	Test Requirements	5014

A.15.1.3	Cell reselection to FR2-2 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	5014

A.15.1.3.1	Test Purpose and Environment	5014

A.15.1.3.2	Test Parameters	5014

A.15.1.3.3	Test Requirements	5017

A.15.1.4	Cell reselection to FR2-2 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	5017

A.15.1.4.1	Test Purpose and Environment	5017

A.15.1.4.2	Test Parameters	5017

A.15.1.4.3	Test Requirements	5020

A.15.1.5	Cell reselection to FR2-2 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	5020

A.15.1.5.1	Test Purpose and Environment	5020

A.15.1.5.2	Test Parameters	5020

A.15.1.5.3	Test Requirements	5024

A.15.1.6	Cell reselection to FR2-2 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	5024

A.15.1.6.1	Test Purpose and Environment	5024

A.15.1.6.2	Test Parameters	5024

A.15.1.6.3	Test Requirements	5027

A.15.2	Signaling characteristics	5028

A.15.2.1	SCell Activation and Deactivation Delay	5028

A.15.2.1.1	SCell Activation and deactivation for SCell in FR2-2 intra-band in non-DRX	5028

A.15.2.1.1.1	Test Purpose and Environment	5028

A.15.2.1.1.2	Test Requirements	5030

A.15.2.1.2	SCell Activation and deactivation for FR1+FR2-2 inter-band with target SCell in FR2-2	5030

A.15.2.1.2.1	Test Purpose and Environment	5030

A.15.2.1.2.2	Test Requirements	5035

A.15.2.1.3	SCell Activation and deactivation for SCell in FR2-2 inter-band in non-DRX	5036

A.15.2.1.3.1	Test Purpose and Environment	5036

A.15.2.1.3.2	Test Requirements	5039

A.15.2.1.4	Direct SCell activation at SCell addition of known SCell in FR2-2	5040

A.15.2.1.4.1	Test Purpose and Environment	5040

A.15.2.1.4.2	Test Requirements	5043

A.15.2.1.5	Direct SCell activation at handover with known SCell in FR2-2	5044

A.15.2.1.5.1	Test Purpose and Environment	5044

A.15.2.1.5.2	Test Requirements	5048

A.15.3	RRC_CONNECTED state mobility	5049

A.15.3.1	Handover	5049

A.15.3.1.1	Intra-frequency handover from FR2-2 carrier with CCA to FR2-2 carrier with CCA; unknown target cell	5049

A.15.3.1.1.1	Test Purpose and Environment	5049

A.15.3.1.1.2	Test Parameters	5049

A.15.3.1.1.3	Test Requirements	5053

A.15.3.1.2	Inter-frequency handover from FR1 to FR2-2 carrier with CCA; unknown target cell	5053

A.15.3.1.2.1	Test Purpose and Environment	5053

A.15.3.1.2.2	Test Parameters	5053

A.15.3.1.2.3	Test Requirements	5057

A.15.4	Measurement procedure	5058

A.15.4.1	Intra-frequency Measurements	5058

A.15.4.1.1	SA event triggered reporting test without gap under non-DRX for FR2-2 with CCA	5058

A.15.4.1.1.1	Test purpose and Environment	5058

A.15.4.1.1.2	Test Requirements	5060

A.15.4.2	Inter-frequency Measurements	5061

A.15.4.2.1	SA event triggered reporting tests for FR2-2 with CCA without SSB time index detection when DRX is not used (PCell in FR2-2)	5061

A.15.4.2.1.1	Test Purpose and Environment	5061

A.15.4.2.1.2	Test Requirements	5064

A.16	NR standalone tests with all NR cells in FR1 for RedCap	5065

A.16.1	SA: RRC_IDLE state mobility for RedCap	5065

A.16.1.1	Cell re-selection to NR	5065

A.16.1.1.1	Cell reselection to FR1 intra-frequency NR case for 1 Rx UE	5065

A.16.1.1.1.1	Test Purpose and Environment	5065

A.16.1.1.1.2	Test Parameters	5065

A.16.1.1.1.3	Test Requirements	5069

A.16.1.1.2	Cell reselection to FR1 intra-frequency NR case for 2 Rx UE	5069

A.16.1.1.2.1	Test Purpose and Environment	5069

A.16.1.1.2.2	Test Parameters	5069

A.16.1.1.2.3	Test Requirements	5073

A.16.1.1.3	Cell reselection to FR1 inter-frequency NR case for 1 Rx UE	5073

A.16.1.1.3.1	Test Purpose and Environment	5073

A.16.1.1.3.2	Test Parameters	5073

A.16.1.1.3.3	Test Requirements	5078

A.16.1.1.4	Cell reselection to FR1 inter-frequency NR case for 2 Rx UE	5078

A.16.1.1.4.1	Test Purpose and Environment	5078

A.16.1.1.4.2	Test Parameters	5078

A.16.1.1.4.3	Test Requirements	5083

A.16.1.1.5	Cell reselection to FR1 intra-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	5083

A.16.1.1.5.1	Test Purpose and Environment	5083

A.16.1.1.5.2	Test Parameters	5083

A.16.1.1.5.3	Test Requirements	5088

A.16.1.1.6	Cell reselection to FR1 intra-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5088

A.16.1.1.6.1	Test Purpose and Environment	5088

A.16.1.1.6.2	Test Parameters	5088

A.16.1.1.6.3	Test Requirements	5093

A.16.1.1.7	Cell reselection to FR1 inter-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	5093

A.16.1.1.7.1	Test Purpose and Environment	5093

A.16.1.1.7.2	Test Parameters	5093

A.16.1.1.7.3	Test Requirements	5098

A.16.1.1.8	Cell reselection to FR1 inter-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5098

A.16.1.1.8.1	Test Purpose and Environment	5098

A.16.1.1.8.2	Test Parameters	5099

A.16.1.1.8.3	Test Requirements	5103

A.16.1.2	Inter-RAT E-UTRAN cell re-selection for RedCap	5104

A.16.1.2.1	Cell reselection to higher priority E-UTRAN for 1RX	5104

A.16.1.2.1.1	Test Purpose and Environment	5104

A.16.1.2.1.2	Test Parameters	5104

A.16.1.2.1.3	Test Requirements	5107

A.16.1.2.2	Cell reselection to higher priority E-UTRAN for 2RX	5108

A.16.1.2.2.1	Test Purpose and Environment	5108

A.16.1.2.2.2	Test Parameters	5108

A.16.1.2.2.3	Test Requirements	5111

A.16.1.2.3.1	Test Purpose and Environment	5112

A. 16.1.2.3.2	Test Parameters	5112

A.16.1.2.3.3	Test Requirements	5115

A.16.1.2.4.1	Test Purpose and Environment	5116

A.16.1.2.4.2	Test Parameters	5116

A.16.1.3.1.3	Test Requirements	5119

A.16.1.2.5	Cell reselection to lower priority E-UTRAN for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	5120

A.16.1.2.5.1	Test Purpose and Environment	5120

A.16.1.2.5.2	Test Parameters	5120

A.16.1.2.5.3	Test Requirements	5123

A.16.1.2.6	Cell reselection to lower priority E-UTRAN for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5124

A.16.1.2.6.1	Test Purpose and Environment	5124

A.16.1.2.6.2	Test Parameters	5124

A.16.1.2.6.3	Test Requirements	5127

A.16.2	SA: RRC_INACTIVE state mobility for RedCap	5128

A.16.2.1	Configured Grant based Small Data Transmissions (CG-SDT) for RedCap	5128

A.16.2.1.1	NR UE CG-SDT Test in FR1 for 1Rx RedCap UE	5128

A.16.2.1.1.1	Test purpose and Environment	5128

A.16.2.1.1.2	Test Parameters	5130

A.16.2.1.1.3	Test requirements	5133

A.16.2.1.2	NR UE CG-SDT Test in FR1 for 2Rx RedCap UE	5133

A.16.2.1.2.1	Test purpose and Environment	5133

A.16.2.1.2.2	Test Parameters	5135

A.16.2.1.2.3	Test requirements	5138

A.16.3	RRC_CONNECTED state mobility for RedCap	5139

A.16.3.1	Handover	5139

A.16.3.1.1	Intra-frequency handover from FR1 to FR1; known target cell for 1 Rx UE	5139

A.16.3.1.1.1	Test Purpose and Environment	5139

A.16.3.1.1.2	Test Parameters	5139

A.16.3.1.1.3	Test Requirements	5142

A.16.3.1.2	Intra-frequency handover from FR1 to FR1; known target cell for 2 Rx UE	5142

A.16.3.1.2.1	Test Purpose and Environment	5142

A.16.3.1.2.2	Test Parameters	5142

A.16.3.1.2.3	Test Requirements	5146

A.16.3.1.3	Intra-frequency handover from FR1 to FR1; unknown target cell for 1 Rx UE	5146

A.16.3.1.3.1	Test Purpose and Environment	5146

A.16.3.1.3.2	Test Parameters	5146

A.16.3.1.3.3	Test Requirements	5150

A.16.3.1.4	Intra-frequency handover from FR1 to FR1; unknown target cell for 2 Rx UE	5150

A.16.3.1.4.1	Test Purpose and Environment	5150

A.16.3.1.4.2	Test Parameters	5150

A.16.3.1.5	Inter-frequency handover from FR1 to FR1; unknown target cell for 1 Rx UE	5154

A.16.3.1.5.1	Test Purpose and Environment	5154

A.16.3.1.5.2	Test Parameters	5154

A.16.3.1.5.3	Test Requirements	5158

A.16.3.1.6	Inter-frequency handover from FR1 to FR1; unknown target cell for 2 Rx UE	5158

A.16.3.1.6.1	Test Purpose and Environment	5158

A.16.3.1.6.2	Test Parameters	5158

A.16.3.1.6.3	Test Requirements	5162

A.16.3.1.7	SA NR - E-UTRAN handover for 1Rx UE	5162

A.16.3.1.7.1	Test Purpose and Environment	5162

A.16.3.1.7.2	Test Requirements	5168

A.16.3.1.8	 SA NR - E-UTRAN handover for 2Rx UE	5168

A.16.3.1.8.1	Test Purpose and Environment	5168

A.16.3.1.8.2	Test Requirements	5174

A.16.3.1.9	SA NR - E-UTRAN handover with unknown target cell for 1 Rx UE	5174

A.16.3.1.9.1	Test Purpose and Environment	5174

A.16.3.1.9.2	Test Requirements	5181

A.16.3.1.10	SA NR - E-UTRAN handover with unknown target cell for 2 Rx UE	5181

A.16.3.1.10.1	Test Purpose and Environment	5181

A.16.3.1.10.2	Test Requirements	5187

A.16.3.2	RRC Connection Mobility Control	5187

A.16.3.2.1	SA: RRC Re-establishment	5187

A.16.3.2.1.1	Intra-frequency RRC Re-establishment in FR1 for 1 Rx UE	5187

A.16.3.2.1.2	Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE	5191

A.16.3.2.1.3	Inter-frequency RRC Re-establishment in FR1 for 1 Rx UE	5195

A.16.3.2.1.4	Inter-frequency RRC Re-establishment in FR1 for 2 Rx UE	5200

A.16.3.2.1.5	Intra-frequency RRC Re-establishment in FR1 for 1 Rx UE without serving cell timing	5205

A.16.3.2.1.6	Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE without serving cell timing	5209

A.16.3.2.2	Random Access	5213

A.16.3.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE	5213

A.16.3.2.2.2	4-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE	5219

A.16.3.2.2.3	4-step RA type non-contention based random access test in FR1 for NR standalone for 1 Rx UE	5224

A.16.3.2.2.5	2-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE	5234

A.16.3.2.2.6	2-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE	5238

A.16.3.2.2.7	2-step RA type non-contention based test in FR1 for NR standalone for 1 RX UE	5242

A.16.3.2.2.8	2-step RA type non-contention based test in FR1 for NR standalone for 2 RX UE	5246

A.16.3.2.3	SA: RRC Connection Release with Redirection	5250

A.16.3.2.3.1	Redirection from NR in FR1 to NR in FR1 for 1 Rx UE	5250

A.16.3.2.3.2	Redirection from NR in FR1 to NR in FR1 for 2 Rx UE	5254

A.16.3.2.3.3	Redirection from NR in FR1 to E-UTRAN for 1 Rx UE	5258

A.16.3.2.3.4	Redirection from NR in FR1 to E-UTRAN for 2 Rx UE	5265

A.16.4	Timing for RedCap	5272

A.16.4.1	UE transmit timing	5272

A.16.4.1.1	NR UE Transmit Timing Test for FR1 for 1Rx RedCap UE	5272

A.16.4.1.1.1	Test Purpose and environment	5272

A.16.4.1.1.2	Test requirements	5275

A.16.4.1.2	NR UE Transmit Timing Test for FR1 for 2Rx RedCap UE	5276

A.16.4.1.2.1	Test Purpose and environment	5276

A.16.4.1.2.2	Test requirements	5278

A.16.4.2	UE timer accuracy	5278

A.16.4.3	Timing advance	5278

A.16.4.3.1	SA FR1 timing advance adjustment accuracy for 1 Rx UE	5278

A.16.4.3.1.1	Test Purpose and Environment	5278

A.16.4.3.1.2	Test Parameters	5278

A.16.4.3.1.3	Test Requirements	5282

A.16.4.3.2	SA FR1 timing advance adjustment accuracy for 2 Rx UE	5282

A.16.4.3.2.1	Test Purpose and Environment	5282

A.16.4.3.2.2	Test Parameters	5283

A.16.4.3.2.3	Test Requirements	5286

A.16.5	Signalling characteristics for RedCap	5287

A.16.5.1	Radio link Monitoring	5287

A.16.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE	5287

A.16.5.1.1.1	Test Purpose and Environment	5287

A.16.5.1.1.2	Test Requirements	5292

A.16.5.1.2	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE	5292

A.16.5.1.2.1	Test Purpose and Environment	5292

A.16.5.1.2.2	Test Requirements	5298

A.16.5.1.3	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE	5298

A.16.5.1.3.1	Test Purpose and Environment	5298

A.16.5.1.3.2	Test Requirements	5304

A.16.5.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE	5304

A.16.5.1.4.1	Test Purpose and Environment	5304

A.16.5.1.4.2	Test Requirements	5310

A.16.5.1.5	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE	5310

A.16.5.1.5.1	Test Purpose and Environment	5310

A.16.5.1.5.2	Test Requirements	5316

A.16.5.1.6	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE	5316

A.16.5.1.6.1	Test Purpose and Environment	5316

A.16.5.1.6.2	Test Requirements	5321

A.16.5.1.7	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE	5321

A.16.5.1.7.1	Test Purpose and Environment	5321

A.16.5.1.7.2	Test Requirements	5326

A.16.5.1.8	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE	5326

A.16.5.1.8.1	Test Purpose and Environment	5326

A.16.5.1.8.2	Test Requirements	5332

A.16.5.1.9	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE	5332

A.16.5.1.9.1	Test Purpose and Environment	5332

A.16.5.1.9.2	Test Requirements	5338

A.16.5.1.10	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE	5338

A.16.5.1.10.1	Test Purpose and Environment	5338

A.16.5.1.10.2	Test Requirements	5344

A.16.5.1.11	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE	5344

A.16.5.1.11.1	Test Purpose and Environment	5344

A.16.5.1.11.2	Test Requirements	5350

A.16.5.1.12	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE	5350

A.16.5.1.12.1	Test Purpose and Environment	5350

A.16.5.1.12.2	Test Requirements	5356

A.16.5.1.13	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE	5356

A.16.5.1.13.1	Test Purpose and Environment	5356

A.16.5.1.13.2	Test Requirements	5362

A.16.5.1.14	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE	5362

A.16.5.1.14.1	Test Purpose and Environment	5362

A.16.5.1.14.2	Test Requirements	5368

A.16.5.1.15	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE	5368

A.16.5.1.15.1	Test Purpose and Environment	5368

A.16.5.1.15.2	Test Requirements	5374

A.16.5.1.16	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE	5374

A.16.5.1.16.1	Test Purpose and Environment	5374

A.16.5.1.16.2	Test Requirements	5380

A.16.5.2	Beam Failure Detection and Link recovery procedures	5380

A.16.5.2.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 1 Rx UE	5380

A.16.5.2.1.1	Test Purpose and Environment	5380

A.16.5.2.1.2	Test Requirements	5386

A.16.5.2.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE	5386

A.16.5.2.2.1	Test Purpose and Environment	5386

A.16.5.2.2.2	Test Requirements	5392

A.16.5.2.3	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode for 1 Rx UE	5392

A.16.5.2.3.1	Test Purpose and Environment	5392

A.16.5.2.3.2	Test Requirements	5399

A.16.5.2.4	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode for 2 Rx UE	5399

A.16.5.2.4.1	Test Purpose and Environment	5399

A.16.5.2.4.2	Test Requirements	5405

A.16.5.2.5	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 1 Rx UE	5405

A.16.5.2.5.1	Test Purpose and Environment	5405

A.16.5.2.5.2	Test Requirements	5411

A.16.5.2.6	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 2 Rx UE	5411

A.16.5.2.6.1	Test Purpose and Environment	5411

A.16.5.2.6.2	Test Requirements	5417

A.16.5.2.7	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 1 Rx UE	5417

A.16.5.2.7.1	Test Purpose and Environment	5417

A.16.5.2.7.2	Test Requirements	5423

A.16.5.2.8	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 2 Rx UE	5423

A.16.5.2.8.1	Test Purpose and Environment	5423

A.16.5.2.8.2	Test Requirements	5429

A.16.5.3	Active BWP switch	5429

A.16.5.3.1	DCI-based and Timer-based Active BWP Switch	5429

A.16.5.3.1.1	NR FR1 DL active BWP switch with non-DRX in SA for 1 Rx UE	5429

A.16.5.3.1.1.1	Test Purpose and Environment	5429

A.16.5.3.1.2.1	Test Requirements	5434

A.16.5.3.1.2	NR FR1 DL active BWP switch with non-DRX in SA for 2 Rx UE	5434

A.16.5.3.1.2.1	Test Purpose and Environment	5434

A.16.5.3.1.2.2	Test Requirements	5439

A.16.5.3.2	RRC-based Active BWP Switch	5439

A.16.5.3.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA for 1 Rx UE	5439

A.16.5.3.2.1.1	Test Purpose and Environment	5439

A.16.5.3.2.1.2	Test Requirements	5443

A.16.5.3.2.2	NR FR1 DL active BWP switch of Cell with non-DRX in SA for 2 Rx UE	5443

A.16.5.3.2.2.1	Test Purpose and Environment	5443

A.16.5.3.2.2.2	Test Requirements	5447

A.16.5.4	UE specific CBW change	5447

A.16.5.4.1	UE specific CBW change on PCell in FR1 in non-DRX for 1 Rx UE	5447

A.16.5.4.1.1	Test Purpose and Environment	5447

A.16.5.4.1.2	Test Requirements	5451

A.16.5.4.2	UE specific CBW change on PCell in FR1 in non-DRX for 2 Rx UE	5451

A.16.5.4.2.1	Test Purpose and Environment	5451

A.16.5.4.2.2	Test Requirements	5455

A.16.6	Measurement procedure for RedCap	5455

A.16.6.1	Intra-frequency Measurements	5455

A.16.6.1.1	SA event triggered reporting tests without gap under non-DRX for 1 Rx UE	5455

A.16.6.1.1.1	Test purpose and Environment	5455

A.16.6.1.1.2	Test parameters	5455

A.16.6.1.1.3	Test Requirements	5459

A.16.6.1.2	SA event triggered reporting tests without gap under non-DRX for 2 Rx UE	5459

A.16.6.1.2.1	Test purpose and Environment	5459

A.16.6.1.2.2	Test parameters	5459

A.16.6.1.2.3	Test Requirements	5463

A.16.6.1.3	SA event triggered reporting tests without gap under DRX for 1 Rx UE	5463

A.16.6.1.3.1	Test purpose and Environment	5463

A.16.6.1.3.2	Test parameters	5463

A.16.6.1.3.3	Test Requirements	5467

A.16.6.1.4	SA event triggered reporting tests without gap under DRX for 2 Rx UE	5467

A.16.6.1.4.1	Test purpose and Environment	5467

A.16.6.1.4.2	Test parameters	5467

A.16.6.1.4.3	Test Requirements	5471

A.16.6.1.5	SA event triggered reporting tests with per-UE gaps under non-DRX for 1 Rx UE	5471

A.16.6.1.5.1	Test purpose and Environment	5471

A.16.6.1.5.2	Test parameters	5471

A.16.6.1.5.3	Test Requirements	5475

A.16.6.1.6	SA event triggered reporting tests with per-UE gaps under non-DRX for 2 Rx UE	5475

A.16.6.1.6.1	Test purpose and Environment	5475

A.16.6.1.6.2	Test parameters	5475

A.16.6.1.6.3	Test Requirements	5479

A.16.6.1.7	SA event triggered reporting tests with per-UE gaps under DRX for 1 Rx UE	5479

A.16.6.1.7.1	Test purpose and Environment	5479

A.16.6.1.7.2	Test parameters	5479

A.16.6.1.7.3	Test Requirements	5483

A.16.6.1.8	SA event triggered reporting tests with per-UE gaps under DRX for 2 Rx UE	5483

A.16.6.1.8.1	Test purpose and Environment	5483

A.16.6.1.8.2	Test parameters	5483

A.16.6.1.8.3	Test Requirements	5487

A.16.6.1.9	SA event triggered reporting tests without gap under non-DRX with SSB index reading for 1 Rx UE	5487

A.16.6.1.9.1	Test purpose and Environment	5487

A.16.6.1.9.2	Test parameters	5487

A.16.6.1.9.3	Test Requirements	5489

A.16.6.1.10	SA event triggered reporting tests without gap under non-DRX with SSB index reading for 2 Rx UE	5490

A.16.6.1.10.1	Test purpose and Environment	5490

A.16.6.1.10.2	Test parameters	5490

A.16.6.1.10.3	Test Requirements	5491

A.16.6.1.11	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 1 Rx UE	5492

A.16.6.1.11.1	Test purpose and Environment	5492

A.16.6.1.11.2	Test parameters	5492

A.16.6.1.11.3	Test Requirements	5494

A.16.6.1.12	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 2 Rx UE	5495

A.16.6.1.12.1	Test purpose and Environment	5495

A.16.6.1.12.2	Test parameters	5495

A.16.6.1.12.3	Test Requirements	5497

A.16.6.2	Inter-frequency Measurements	5498

A.16.6.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 1 Rx UE	5498

A.16.6.2.1.1	Test Purpose and Environment	5498

A.16.6.2.1.2	Test Requirements	5503

A.16.6.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 2 Rx UE	5503

A.16.6.2.2.1	Test Purpose and Environment	5503

A.16.6.2.2.2	Test Requirements	5508

A.16.6.2.3	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used for 1 Rx UE	5508

A.16.6.2.3.1	Test Purpose and Environment	5508

A.16.6.2.3.2	Test Requirements	5511

A.16.6.2.4	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used for 2 Rx UE	5512

A.16.6.2.4.1	Test Purpose and Environment	5512

A.16.6.2.4.2	Test Requirements	5515

A.16.6.2.5	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 1 Rx UE	5516

A.16.6.2.5.1	Test Purpose and Environment	5516

A.16.6.2.5.2	Test Requirements	5519

A.16.6.2.6	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 2 Rx UE	5520

A.16.6.2.6.1	Test Purpose and Environment	5520

A.16.6.2.6.2	Test Requirements	5523

A.16.6.2.7	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 1 Rx UE	5524

A.16.6.2.7.1	Test Purpose and Environment	5524

A.16.6.2.7.2	Test Requirements	5528

A.16.6.2.8	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 2 Rx UE	5529

A.16.6.2.8.1	Test Purpose and Environment	5529

A.16.6.2.8.2	Test Requirements	5533

A.16.6.2.9	SA event triggered reporting tests with additional mandatory gap pattern for 1 Rx UE	5534

A.16.6.2.9.1	Test Purpose and Environment	5534

A.16.6.2.9.2	Test Requirements	5536

A.16.6.2.10	SA event triggered reporting tests with additional mandatory gap pattern for 2 Rx UE	5536

A.16.6.2.10.1	Test Purpose and Environment	5536

A.16.6.2.10.2	Test Requirements	5539

A.16.6.2.11	SA event triggered reporting tests for FR1 when DRX is used for 1 Rx UE	5539

A.16.6.2.11.1	Test Purpose and Environment	5539

A.16.6.2.11.2	Test Requirements	5542

A.16.6.2.12	SA event triggered reporting tests for FR1 when DRX is used for 2 Rx UE	5543

A.16.6.2.12.1	Test Purpose and Environment	5543

A.16.6.2.12.2	Test Requirements	5547

A.16.6.3	Inter-RAT Measurements	5548

A.16.6.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 for 1 Rx UE	5548

A.16.6.3.1.1	Test purpose and Environment	5548

A.16.6.3.1.2	Test Requirements	5553

A.16.6.3.2	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 for 2 Rx UE	5553

A.16.6.3.2.1	Test purpose and Environment	5553

A.16.6.3.2.2	Test Requirements	5558

A.16.6.3.3	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for 1 Rx UE	5558

A.16.6.3.3.1	Test purpose and Environment	5558

A.16.6.3.3.2	Test Requirements	5563

A.16.6.3.4	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for 2 Rx UE	5563

A.16.6.3.4.1	Test purpose and Environment	5563

A.16.6.3.4.2	Test Requirements	5568

A.16.6.4	L1-RSRP measurement for beam reporting	5568

A.16.6.4.1	SSB based L1-RSRP measurement when DRX is not used for 1 Rx UE	5568

A.16.6.4.1.1	Test Purpose and Environment	5568

A.16.6.4.1.2	Test parameters	5568

A.16.6.4.1.3	Test Requirements	5571

A.16.6.4.2	SSB based L1-RSRP measurement when DRX is not used for 2 Rx UE	5571

A.16.6.4.2.1	Test Purpose and Environment	5571

A.16.6.4.2.2	Test parameters	5572

A.16.6.4.2.3	Test Requirements	5575

A.16.6.4.3	SSB based L1-RSRP measurement when DRX is used for 1 Rx UE	5575

A.16.6.4.3.1	Test Purpose and Environment	5575

A.16.6.4.3.2	Test parameters	5576

A.16.6.4.3.3	Test Requirements	5579

A.16.6.4.4	SSB based L1-RSRP measurement when DRX is used for 2 Rx UE	5579

A.16.6.4.4.1	Test Purpose and Environment	5579

A.16.6.4.4.2	Test parameters	5580

A.16.6.4.4.3	Test Requirements	5583

A.16.6.4.5	CSI-RS based L1-RSRP measurement when DRX is not used for 1 Rx UE	5583

A.16.6.4.5.1	Test Purpose and Environment	5583

A.16.6.4.5.2	Test parameters	5584

A.16.6.4.5.3	Test Requirements	5587

A.16.6.4.6	CSI-RS based L1-RSRP measurement when DRX is not used for 2 Rx UE	5587

A.16.6.4.6.1	Test Purpose and Environment	5587

A.16.6.4.6.2	Test parameters	5588

A.16.6.4.6.3	Test Requirements	5591

A.16.6.4.7	CSI-RS based L1-RSRP measurement when DRX is used for 1 Rx UE	5591

A.16.6.4.7.1	Test Purpose and Environment	5591

A.16.6.4.7.2	Test parameters	5592

A.16.6.4.7.3	Test Requirements	5595

A.16.6.4.8	CSI-RS based L1-RSRP measurement when DRX is used for 2 Rx UE	5595

A.16.6.4.8.1	Test Purpose and Environment	5595

A.16.6.4.8.2	Test parameters	5596

A.16.6.4.8.3	Test Requirements	5599

A.16.6.5	NR measurements with autonomous gaps	5599

A.16.6.5.1	SA intra-frequency CGI identification of NR neighbor cell in FR1 for 1 Rx UE	5599

A.16.6.5.1.1	Test Purpose and Environment	5599

A.16.6.5.1.2	Test Parameters	5599

A.16.6.5.1.3	Test Requirements	5602

A.16.6.5.2	SA intra-frequency CGI identification of NR neighbor cell in FR1 for 2 Rx UE	5602

A.16.6.5.2.1	Test Purpose and Environment	5602

A.16.6.5.2.2	Test Parameters	5602

A.16.6.5.2.3	Test Requirements	5605

A.16.6.5.3	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA for 1 Rx UE	5605

A.16.6.5.3.1	Test Purpose and Environment	5605

A.16.6.5.3.2	Test Requirements	5612

A.16.6.5.4	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA for 2 Rx UE	5612

A.16.6.5.4.1	Test Purpose and Environment	5612

A.16.6.5.4.2	Test Requirements	5619

A.16.7	Measurement Performance requirements for RedCap	5619

A.16.7.1	SS-RSRP	5619

A.16.7.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5619

A.16.7.1.1.1	Test Purpose and Environment	5619

A.16.7.1.1.2	Test parameters	5619

A.16.7.1.1.3	Test Requirements	5624

A.16.7.1.2	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 2Rx UE	5624

A.16.7.1.2.1	Test Purpose and Environment	5624

A.16.7.1.2.2	Test parameters	5624

A.16.7.1.2.3	Test Requirements	5629

A.16.7.1.3	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5629

A.16.7.1.3.1	Test Purpose and Environment	5629

A.16.7.1.3.2	Test parameters	5629

A.16.7.1.3.3	Test Requirements	5633

A.16.7.1.4	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5633

A.16.7.1.4.1	Test Purpose and Environment	5633

A.16.7.1.4.2	Test parameters	5634

A.16.7.1.4.3	Test Requirements	5638

A.16.7.2	SS-RSRQ	5638

A.16.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5638

A.16.7.2.1.1	Test Purpose and Environment	5638

A.16.7.2.1.2	Test Parameters	5639

A.16.7.2.1.3	Test Requirements	5644

A.16.7.2.2	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5644

A.16.7.2.2.1	Test Purpose and Environment	5644

A.16.7.2.2.2	Test Parameters	5644

A.16.7.2.2.3	Test Requirements	5649

A.16.7.2.3	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5649

A.16.7.2.3.1	Test Purpose and Environment	5649

A.16.7.2.3.2	Test parameters	5649

A.16.7.2.3.3	Test Requirements	5654

A.16.7.2.4	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5654

A.16.7.2.4.1	Test Purpose and Environment	5654

A.16.7.2.4.2	Test parameters	5654

A.16.7.2.4.3	Test Requirements	5660

A.16.7.3	SS-SINR	5660

A.16.7.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5660

A.16.7.3.1.1	Test Purpose and Environment	5660

A.16.7.3.1.2	Test parameters	5661

A.16.7.3.1.3	Test Requirements	5665

A.16.7.3.2	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5665

A.16.7.3.2.1	Test Purpose and Environment	5665

A.16.7.3.2.2	Test parameters	5666

A.16.7.3.3	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5670

A.16.7.3.3.1	Test Purpose and Environment	5670

A.16.7.3.3.2	Test parameters	5670

A.16.7.3.3.3	Test Requirements	5675

A.16.7.3.4	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5676

A.16.7.3.4.1	Test Purpose and Environment	5676

A.16.7.3.4.2	Test parameters	5676

A.16.7.3.4.3	Test Requirements	5680

A.16.7.4	L1-RSRP measurement for beam reporting	5681

A.16.7.4.1	SSB based L1-RSRP measurement for 1 Rx UE	5681

A.16.7.4.1.1	Test Purpose and Environment	5681

A.16.7.4.1.2	Test parameters	5681

A.16.7.4.1.3	Test Requirements	5685

A.16.7.4.2	SSB based L1-RSRP measurement for 2 Rx UE	5685

A.16.7.4.2.1	Test Purpose and Environment	5685

A.16.7.4.2.2	Test parameters	5685

A.16.7.4.2.3	Test Requirements	5686

A.16.7.4.3	CSI-RS based L1-RSRP measurement on resource set with repetition off for 1 Rx UE	5686

A.16.7.4.3.1	Test Purpose and Environment	5686

A.16.7.4.3.2	Test parameters	5686

A.16.7.4.3.3	Test Requirements	5690

A.16.7.4.4	CSI-RS based L1-RSRP measurement on resource set with repetition off for 2 Rx UE	5690

A.16.7.4.4.1	Test Purpose and Environment	5690

A.16.7.4.4.2	Test parameters	5691

A.16.7.4.4.3	Test Requirements	5691

A.16.7.5	E-UTRAN RSRP	5691

A.16.7.5.1	SA: inter-RAT measurement accuracy with FR1 serving cell for 1 Rx UE	5691

A.16.7.5.1.1	Test Purpose and Environment	5691

A.16.7.5.1.2	Test parameters	5691

A.16.7.5.1.3	Test Requirements	5697

A.16.7.5.2	SA: inter-RAT measurement accuracy with FR1 serving cell for 2 Rx UE	5698

A.16.7.5.2.1	Test Purpose and Environment	5698

A.16.7.5.2.2	Test parameters	5698

A.16.7.5.2.3	Test Requirements	5704

A.16.7.6	E-UTRAN RSRQ	5705

A.16.7.6.1	SA: inter-RAT measurement accuracy with FR1 serving cell for 1 Rx UE	5705

A.16.7.6.1.1	Test Purpose and Environment	5705

A.16.7.6.1.2	Test parameters	5705

A.16.7.6.1.3	Test Requirements	5711

A.16.7.6.2	SA: inter-RAT measurement accuracy with FR1 serving cell for 2 Rx UE	5712

A.16.7.6.2.1	Test Purpose and Environment	5712

A.16.7.6.2.2	Test parameters	5712

A.16.7.6.2.3	Test Requirements	5718

A.16.7.7	E-UTRAN RS-SINR	5719

A.16.7.7.1	SA: inter-RAT measurement accuracy with FR1 serving cell for 1 Rx UE	5719

A.16.7.7.2	SA: inter-RAT measurement accuracy with FR1 serving cell for 2 Rx UE	5719

A.17	NR standalone tests with one or more NR cells in FR2 for RedCap	5720

A.17.1	SA: RRC_IDLE state mobility for RedCap	5720

A.17.1.1	Cell re-selection to NR	5720

A.17.1.1.1	Cell reselection to FR2 intra-frequency NR case for 2 Rx	5720

A.17.1.1.1.1	Test Purpose and Environment	5720

A.17.1.1.1.2	Test Parameters	5720

A.17.1.1.1.3	Test Requirements	5724

A.17.1.1.2	Cell reselection to FR2 inter-frequency NR case	5724

A.17.1.1.2.1	Test Purpose and Environment	5724

A.17.1.1.2.2	Test Parameters	5724

A.17.1.1.2.3	Test Requirements	5728

A.17.1.1.3	Cell reselection to FR2 intra-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5728

A.17.1.1.3.1	Test Purpose and Environment	5728

A.17.1.1.3.2	Test Parameters	5728

A.17.1.1.3.3	Test Requirements	5731

A.17.1.1.4	Cell reselection to FR2 inter-frequency NR case for UE fulfilling stationary mobility relaxed measurement criterion for 2 Rx UE	5731

A.17.1.1.4.1	Test Purpose and Environment	5731

A.17.1.1.4.2	Test Parameters	5731

A.17.1.1.4.3	Test Requirements	5735

A.17.2	SA: RRC_INACTIVE state mobility for RedCap	5735

A.17.2.1	Configured Grant based Small Data Transmissions (CG-SDT) for RedCap	5735

A.17.2.1.1	TA validation for CG-SDT in FR2 for RedCap	5735

A.17.2.1.1.1	Test Purpose and Environment	5735

A.17.2.1.1.2	Test Requirements	5739

A.17.3	RRC_CONNECTED state mobility for RedCap	5740

A.17.3.1	Handover for RedCap	5740

A.17.3.1.1	Intra-frequency handover from FR2 to FR2; unknown target cell for 2 Rx	5740

A.17.3.1.1.1	Test Purpose and Environment	5740

A.17.3.1.1.2	Test Parameters	5740

A.17.3.1.1.3	Test Requirements	5742

A.17.3.1.2	Inter-frequency handover from FR2 to FR2; unknown target cell for 2 Rx	5742

A.17.3.1.2.1	Test Purpose and Environment	5742

A.17.3.1.2.2	Test Parameters	5742

A.17.3.1.2.3 Test Requirements	5744

A.17.3.2	RRC Connection Mobility Control for RedCap	5745

A.17.3.2.1	SA: RRC Re-establishment	5745

A.17.3.2.1.1	Intra-frequency RRC Re-establishment in FR2	5745

A.17.3.2.1.1.1	Test Purpose and Environment	5745

A.17.3.2.1.2	Inter-frequency RRC Re-establishment in FR2	5748

A.17.3.2.1.2.1	Test Purpose and Environment	5748

A.17.3.2.1.3	Intra-frequency RRC Re-establishment in FR2 without serving cell timing	5751

A.17.3.2.1.3.1	Test Purpose and Environment	5751

A.17.3.2.1.3.2	Test Requirements	5753

A.17.3.2.2	Random Access	5754

A.17.3.2.2.1	4-step RA type contention based random access test in FR2 for NR Standalone	5754

A.17.3.2.2.1.1	Test Purpose and Environment	5754

A.17.3.2.2.1.2	Test Requirements	5756

A.17.3.2.2.2	4-step RA type non-contention based random access test in FR2 for NR Standalone	5758

A.17.3.2.2.2.1	Test Purpose and Environment	5758

A.17.3.2.2.2.2	Test Requirements	5760

A.17.3.2.2.3	2-step RA type contention based random access test in FR2 for NR Standalone	5762

A.17.3.2.2.3.1	Test Purpose and Environment	5762

A.17.3.2.2.3.2	Test Requirements	5764

A.17.3.2.2.4	2-step RA type non-contention based random access test in FR2 for NR Standalone	5765

A.17.3.2.2.4.1	Test Purpose and Environment	5765

A.17.3.2.2.4.2	Test Requirements	5767

A.17.3.2.3	SA: RRC Connection Release with Redirection	5768

A.17.3.2.3.1	Redirection from NR in FR2 to NR in FR2	5768

A.17.3.2.3.1.1	Test Purpose and Environment	5768

A.17.3.2.3.1.2	Test Parameters	5768

A.17.3.2.3.1.3	Test Requirements	5772

A.17.4	Timing	5772

A.17.4.1	UE transmit timing	5772

A.17.4.1.1	NR UE Transmit Timing Test for FR2	5772

A.17.4.1.1.1	Test Purpose and environment	5772

A.17.4.1.1.2	Test requirements	5775

A.17.4.2	UE timer accuracy	5776

A.17.4.3	Timing advance	5776

A.17.4.3.1	SA FR2 timing advance adjustment accuracy	5776

A.17.4.3.1.1	Test Purpose and Environment	5776

A.17.4.3.1.2	Test Parameters	5776

A.17.4.3.1.3 Test Requirements	5779

A.17.4.2	UE timer accuracy	5779

A.17.4.3	Timing advance	5779

A.17.4.3.1	SA FR2 timing advance adjustment accuracy	5779

A.17.5	Signaling characteristics for RedCap	5779

A.17.5.1	Radio link Monitoring for RedCap	5779

A.17.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	5779

A.17.5.1.1.1	Test Purpose and Environment	5779

A.17.5.1.1.2	Test Requirements	5782

A.17.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	5783

A.17.5.1.2.1	Test Purpose and Environment	5783

A.17.5.1.2.2	Test Requirements	5787

A.17.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	5788

A.17.5.1.3.1	Test Purpose and Environment	5788

A.17.5.1.3.2	Test Requirements	5792

A.17.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	5792

A.17.5.1.4.1	Test Purpose and Environment	5792

A.17.5.1.4.2	Test Requirements	5797

A.17.5.1.5	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	5797

A.17.5.1.5.1	Test Purpose and Environment	5797

A.17.5.1.5.2	Test Requirements	5802

A.17.5.1.6	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	5802

A.17.5.1.6.1	Test Purpose and Environment	5802

A.17.5.1.6.2	Test Requirements	5806

A.17.5.1.7	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	5806

A.17.5.1.7.1	Test Purpose and Environment	5806

A.17.5.1.7.2	Test Requirements	5810

A.17.5.1.8	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	5810

A.17.5.1.8.1	Test Purpose and Environment	5810

A.17.5.1.8.2	Test Requirements	5815

A.17.5.1.9	UE Radio Link Monitoring Scheduling Restrictions on FR2	5815

A.17.5.1.9.1	Test Purpose and Environment	5815

A.17.5.1.9.2	Test Requirements	5818

A.17.5.2	Beam Failure Detection and Link recovery procedures	5818

A.17.5.2.1	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	5818

A.17.5.2.1.1	Test Purpose and Environment	5818

A.17.5.2.1.2	Test Requirements	5822

A.17.5.2.2	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode	5822

A.17.5.2.2.1	Test Purpose and Environment	5822

A.17.5.2.2.2	Test Requirements	5826

A.17.5.2.3	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	5826

A.17.5.2.3.1	Test Purpose and Environment	5826

A.17.5.2.3.2	Test Requirements	5830

A.17.5.2.4	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode	5830

A.17.5.2.4.1	Test Purpose and Environment	5830

A.17.5.2.4.2	Test Requirements	5834

A.17.5.2.5	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE	5834

A.17.5.2.5.1	Test Purpose and Environment	5834

A.17.5.2.5.2	Test Requirements	5838

A.17.5.3	Active BWP switch for RedCap	5838

A.17.5.3.1	DCI-based and Timer-based Active BWP Switch	5838

A.17.5.3.1.1	NR FR2 DL active BWP switch with non-DRX in SA	5838

A.17.5.3.1.1.1	Test Purpose and Environment	5838

A.17.5.3.1.1.2	Test Requirements	5841

A.17.5.3.2	RRC-based Active BWP Switch	5841

A.17.5.3.2.1	NR FR2 DL active BWP switch of PCell with non-DRX in SA	5841

A.17.5.3.2.1.1	Test Purpose and Environment	5841

A.17.5.3.2.1.2	Test Requirements	5844

A.17.5.4	Active TCI state switch delay	5845

A.17.5.4.1	MAC-CE based active TCI state switch	5845

A.17.5.4.1.1	NR PCell FR2 active TCI state switch for a known TCI state	5845

A.17.5.4.1.1.1	Test Purpose and Environment	5845

A.17.5.4.1.1.2	Test Requirements	5848

A.17.5.4.2	RRC based active TCI state switch	5848

A.17.5.4.2.1	NR PCell FR2 active TCI state switch for a known TCI state	5848

A.17.5.4.2.1.1	Test Purpose and Environment	5848

A.17.5.4.2.1.2	Test Requirements	5852

A.17.5.5	Uplink spatial relation switch delay	5852

A.17.5.5.1	MAC-CE based Spatial Relation switch	5852

A.17.5.5.1.1	 NR PCell FR2 spatial relation associated with known DL-RS	5852

A.17.5.5.1.1.1	Test Purpose and Environment	5852

A.17.5.5.1.1.2	Test Requirements	5855

A.17.5.5.2	RRC based spatial relation switch	5855

A.17.5.5.2.1	NR PCell FR2 spatial relation switch associated with a known DL-RS	5855

A.17.5.5.2.1.2	Test Requirements	5858

A.17.5.6	UE specific CBW change	5858

A.17.5.6.1	NR FR2 UE specific CBW change of PCell with non-DRX in SA	5858

A.17.5.6.1.1	Test Purpose and Environment	5858

A.17.5.6.1.2	Test Requirements	5861

A.17.6	Measurement procedure for RedCap	5862

A.17.6.1	Intra-frequency Measurements	5862

A.17.6.1.1	SA event triggered reporting test without gap under non-DRX	5862

A.17.6.1.1.1	Test purpose and Environment	5862

A.17.6.1.1.2	Test Requirements	5865

A.17.6.1.2	SA event triggered reporting test without gap under DRX	5865

A.17.6.1.2.1	Test purpose and Environment	5865

A.7.6.1.2.2	Test Requirements	5865

A.17.6.1.3	SA event triggered reporting test with per-UE gaps under non-DRX	5866

A.17.6.1.3.1	Test purpose and Environment	5866

A.17.6.1.3.2	Test Requirements	5870

A.17.6.1.4	SA event triggered reporting test with per-UE gaps under DRX	5870

A.17.6.1.4.1	Test purpose and Environment	5870

A.17.6.1.4.2	Test Requirements	5873

A.17.6.2	Inter-frequency Measurements	5874

A.17.6.2.1	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2)	5874

A.17.6.2.1.1	Test Purpose and Environment	5874

A.17.6.2.1.2	Test Requirements	5877

A.17.6.2.2	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is used (PCell in FR2)	5877

A.17.6.2.2.1	Test Purpose and Environment	5877

A.17.6.2.2.2	Test Requirements	5881

A.17.6.2.3	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is not used (PCell in FR2)	5881

A.17.6.2.3.1	Test Purpose and Environment	5881

A.17.6.2.3.2	Test Requirements	5885

A.17.6.2.4	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is used (PCell in FR2) for 2 RX UE	5885

A.17.6.2.4.1	Test Purpose and Environment	5885

A.17.6.2.4.2	Test Requirements	5889

A.17.6.3	L1-RSRP measurement for beam reporting	5889

A.17.6.3.1	SSB based L1-RSRP measurement when DRX is not used	5889

A.17.6.3.1.1	Test Purpose and Environment	5889

A.17.6.3.1.2	Test parameters	5890

A.17.6.3.1.3	Test Requirements	5890

A.17.6.3.2	SSB based L1-RSRP measurement when DRX is used	5890

A.17.6.3.2.1	Test Purpose and Environment	5890

A.17.6.3.2.2	Test parameters	5890

A.17.6.3.2.3	Test Requirements	5892

A.17.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	5892

A.17.6.3.3.1	Test Purpose and Environment	5892

A.17.6.3.3.2	Test parameters	5893

A.17.6.3.3.3	Test Requirements	5895

A.17.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	5896

A.17.6.3.4.1	Test Purpose and Environment	5896

A.17.6.3.4.2	Test parameters	5896

A.7.6.3.3.3	Test Requirements	5898

A.17.6.4.1	SA interfrequency CGI reporting in autonomous gaps test (PCell in FR2) for 2 RX UE	5899

A.17.6.4.1.1	Test Purpose and Environment	5899

A.17.6.4.1.2	Test Requirements	5902

A.17.7	Measurement Performance requirements	5902

A.17.7.1	SS-RSRP	5902

A.17.7.1.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	5902

A.17.7.1.1.1	Test Purpose and Environment	5902

A.17.7.1.1.2	Test parameters	5902

A.17.7.1.1.3	Test Requirements	5906

A.17.7.1.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	5907

A.17.7.1.2.1	Test Purpose and Environment	5907

A.17.7.1.2.2	Test parameters	5907

A.17.7.1.2.3	Test Requirements	5911

A.17.7.2	SS-RSRQ	5912

A.17.7.2.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	5912

A.17.7.2.1.1	Test Purpose and Environment	5912

A.17.7.2.1.2	Test Parameters	5912

A.17.7.2.1.3	Test Requirements	5914

A.17.7.2.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell for 2 Rx UE	5914

A.17.7.2.2.1	Test Purpose and Environment	5914

A.17.7.2.2.2	Test parameters	5914

A.17.7.2.2.3	Test Requirements	5916

A.17.7.2.3	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	5916

A.17.7.3	L1-RSRP measurement for beam reporting	5916

A.17.7.3.1	SSB based L1-RSRP measurement	5916

A.17.7.3.1.1	Test Purpose and Environment	5916

A.17.7.3.1.2	Test parameters	5917

A.17.7.3.1.3	Test Requirements	5917

A.17.7.3.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	5917

A.17.7.3.2.1	Test Purpose and Environment	5917

A.17.7.3.2.2	Test parameters	5917

A.17.7.3.2.3	Test Requirements	5918

A.17.7.4	SS-SINR	5918

A.17.7.4	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell for 2Rx UE	5918

A.17.7.4.1.1	Test Purpose and Environment	5918

A.17.7.4.1.2	Test parameters	5918

A.17.7.4.1.3	Test Requirements	5920

A.18	E-UTRA standalone tests for NR RRM for RedCap	5920

A.18.1	RRC_IDLE state mobility	5920

A.18.1.1	Inter-RAT NR Cell re-selection	5920

A.18.1.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1	5920

A.18.1.1.1.1	Test Purpose and Environment	5920

A.18.2.1.1.2	Test Requirements	5925

A.18.2	RRC_CONNECTED state mobility	5925

A.18.2.1	Handover	5925

A.18.2.1.1	E-UTRAN - NR handover in FR1	5925

A.18.2.1.1.1	Test Purpose and Environment	5925

A.18.2.1.1.2	Test Requirements	5930

A.18.2.2	RRC connection release with redirection	5930

A.18.2.2.1	Redirection from E-UTRA to NR FR1 for redcap UE	5930

A.18.2.2.1.1	Test Purpose and Environment	5930

A.18.2.2.1.2	Test Parameters	5930

A.18.2.2.1.3	Test Requirements	5937

A.18.3	Measurement procedure	5937

A.18.3.1	E-UTRA – NR Inter-RAT Measurements	5937

A.18.3.1.1	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	5937

A.18.3.1.1.1	Test Purpose and Environment	5937

A.18.3.1.1.2	Test Requirements	5943

A.18.3.1.2	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	5943

A.18.3.1.2.1	Test Purpose and Environment	5943

A.18.3.1.2.2	Test Requirements	5949

A.18.3.1.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	5949

A.18.3.1.3.1	Test Purpose and Environment	5949

A.18.3.1.3.2	Test Requirements	5955

A.18.3.1.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	5955

A.18.3.1.4.1	Test Purpose and Environment	5955

A.18.3.1.4.2	Test Requirements	5961

A.18.3.1.5	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is not used	5961

A.18.3.1.5.1	Test Purpose and Environment	5961

A.18.3.1.5.2	Test Requirements	5963

A.18.3.1.6	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is used	5964

A.18.3.1.6.1	Test Purpose and Environment	5964

A.18.3.1.6.2	Test Requirements	5966

A.18.3.1.7	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is not used	5967

A.18.3.1.7.1	Test Purpose and Environment	5967

A.18.3.1.7.2	Test Requirements	5969

A.18.3.1.8	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is used	5970

A.18.3.1.8.1	Test Purpose and Environment	5970

A.18.3.1.8.2	Test Requirements	5972

Annex B (normative): Conditions for RRM requirements applicability for operating bands	5973

B.1	Conditions for NR RRC_IDLE state mobility	5973

B.1.1	Introduction	5973

B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection	5973

B.1.3	Conditions for measurements on NR inter-frequency cells for cell re-selection	5974

B.1.4	Conditions for measurements on NR intra-frequency cells for cell re-selection for RedCap	5974

B.1.5	Conditions for measurements on NR inter-frequency cells for cell re-selection for RedCap	5976

B.1.6	Conditions for measurements on NR intra-frequency cells for cell re-selection for satellite access	5976

B.1.7	Conditions for measurements on NR inter-frequency cells for cell re-selection for satellite access	5977

B.2	Conditions for UE measurements procedures and performance requirements in RRC_CONNECTED state	5977

B.2.1	Introduction	5977

B.2.1.1	General	5977

B.2.1.2	Derivation of Minimum SSB_RP values for FR1	5977

B.2.1.3	Derivation of Minimum SSB_RP values for FR2	5977

B.2.1.3.1	Minimum SSB_RP values for Rx Beam Peak angle of arrival	5977

B.2.1.3.2	Minimum SSB_RP values for angle of arrival within Spherical coverage	5978

B.2.1.4	Gain to SS-RSRP and CSI-RSRP measurement point for FR1	5979

B.2.1.5	Gain to SS-RSRP and CSI-RSRP measurement point for FR2	5979

B.2.1.5.1	Gain to SS-RSRP and CSI-RSRP measurement point for Rx Beam Peak angle of arrival	5979

B.2.1.5.2	Gain to SS-RSRP measurement point for different frequency	5980

B.2.1.5.3	Alignment of Rough beam to Rx beam Peak	5980

B.2.1.6	Gain to PRS-RSRP measurement point for FR2	5980

B.2.1.6.1	Gain to PRS-RSRP measurement point for Rx Beam Peak angle of arrival	5980

B.2.2	Conditions for NR intra-frequency measurements	5981

B.2.3	Conditions for NR inter-frequency measurements	5982

B.2.4	Conditions for NR L1-RSRP reporting	5983

B.2.4.1	Conditions for SSB based L1-RSRP reporting	5983

B.2.4.2	Conditions for CSI-RS based L1-RSRP reporting	5984

B.2.5	Conditions for RRC connection release with redirection to NR	5986

B.2.6	Void	5987

B.2.6.1	Void	5987

B.2.6.2	Void	5987

B.2.7	Conditions for SRS-RSRP measurements	5987

B.2.8	Conditions for NR L1-SINR reporting	5988

B.2.8.1	Conditions for L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	5988

B.2.8.2	Conditions for L1-SINR reporting with SSB based CMR and dedicated IMR configured	5989

B.2.8.2.1	L1-SINR reporting with SSB based CMR and dedicated ZP-IMR configured	5989

B.2.8.2.2	L1-SINR reporting with SSB based CMR and dedicated NZP-IMR configured	5989

B.2.8.3	Conditions for L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	5991

B.2.8.3.1	L1-SINR reporting with CSI-RS based CMR and dedicated ZP-IMR configured	5991

B.2.8.3.2	L1-SINR reporting with CSI-RS based CMR and dedicated NZP-IMR configured	5992

B.2.9	Conditions for NR intra-frequency measurements under CCA	5993

B.2.10	Conditions for NR inter-frequency measurements under CCA	5993

B.2.11	Conditions for NR L1-RSRP reporting under CCA	5993

B.2.11.1	Conditions for SSB based L1-RSRP reporting	5993

B.2.12	Conditions for NR CSI-RS based intra-frequency measurements	5994

B.2.13	Conditions for NR CSI-RS based inter-frequency measurements	5995

B.2.14	Conditions for NR PRS-based measurements	5997

B.2.15	Conditions for NR intra-frequency measurements	5999

B.2.16	Conditions for NR inter-frequency measurements	6000

B.2.17	Conditions for NR intra-frequency measurements for satellite access	6002

B.2.18	Conditions for NR inter-frequency measurements for satellite access	6003

B.2.19	Conditions for NR L1-RSRP reporting for satellite access	6003

B.2.19.1	Conditions for SSB based L1-RSRP reporting for satellite access	6003

B.2.19.2	Conditions for CSI-RS based L1-RSRP reporting for satellite access	6003

B.2.20	Conditions for RRC connection release with redirection to NR for satellite access	6004

B.3	RRM Requirements Exceptions	6004

B.3.1	Introduction	6004

B.3.2	Receiver sensitivity relaxation for CA	6004

B.3.2.1	Receiver sensitivity relaxation for UE supporting CA in FR1	6004

B.3.2.2	Receiver sensitivity relaxation for UE configured with CA in FR1	6004

B.3.2.2.1	Inter-band carrier aggregation	6004

B.3.2.2.2	Reference sensitivity exceptions due to UL harmonic interference for CA	6004

B.3.2.2.3	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA	6005

B.3.2.3	Receiver sensitivity relaxation for UE supporting CA in FR2	6005

B.3.2.4	Receiver sensitivity relaxation for UE configured with CA in FR2	6005

B.3.2.4.1	Intra-band contiguous carrier aggregation	6005

B.3.2.4.2	Intra-band non-contiguous carrier aggregation	6005

B.3.3	Receiver sensitivity relaxation for DC	6005

B.3.3.1	Receiver sensitivity relaxation for EN-DC	6005

B.3.3.2	Receiver sensitivity relaxation for NE-DC	6005

B.3.4	Receiver sensitivity relaxation for SUL	6005

B.3.4.1	Receiver sensitivity relaxation for UE supporting SUL in FR1	6005

B.3.4.2	Receiver sensitivity relaxation for UE configured with SUL in FR1	6006

B.3.4.2.1	Reference sensitivity exceptions due to UL harmonic interference for SUL	6006

B.4	Conditions for V2X	6006

B.4.1	Test parameters for GNSS signals	6006

B.4.2	Conditions for PSBCH-RSRP Accuracy Requirements	6006

B.4.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	6007

B.4.4	Conditions for L1 SL-RSRP Accuracy Requirements	6007

B.5	High level test procedure for SAN RRM tests	6007

Annex C (informative): Change history	6009