| 3GPP TS 38.133 V16.19.0 (2024-03) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Requirements for support of radio resource management (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	42

1	Scope	44

2	References	44

3	Definitions, symbols and abbreviations	45

3.1	Definitions	45

3.2	Symbols	46

3.3	Abbreviations	47

3.4	Test tolerances	50

3.5	Frequency bands grouping	50

3.5.1	Introduction	50

3.5.2	NR operating bands in FR1	50

3.5.3	NR operating bands in FR2	51

3.6	Applicability of requirements in this specification version	51

3.6.1	RRC connected state requirements in DRX	52

3.6.2	Number of serving carriers	52

3.6.2.1	Number of serving carriers for SA	52

3.6.2.2	Number of serving carriers for EN-DC	52

3.6.2.3	Number of serving carriers for NE-DC	52

3.6.2.4	Number of serving carriers for NR-DC	53

3.6.3	Applicability for intra-band FR2	53

3.6.4	Applicability for FR2 UE power classes	53

3.6.5	Applicability for SDL bands	53

3.6.6	Applicability of requirements for NGEN-DC operation	53

3.6.7	Applicability of QCL	53

3.6.9	Applicability of requirements for scheduling availability	54

3.6.10	Applicability of requirements for measurement restrictions	54

4	SA: RRC_IDLE state mobility	54

4.1	Cell Selection	54

4.2	Cell Re-selection	54

4.2.1	Introduction	54

4.2.2	Requirements	55

4.2.2.1	UE measurement capability	55

4.2.2.2	Measurement and evaluation of serving cell	55

4.2.2.3	Measurements of intra-frequency NR cells	56

4.2.2.4	Measurements of inter-frequency NR cells	57

4.2.2.5	Measurements of inter-RAT E-UTRAN cells	59

4.2.2.6	Maximum interruption in paging reception	61

4.2.2.7	General requirements	61

4.2.2.8	Minimum requirement at transitions	61

4.2.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	62

4.2.2.9.1	Introduction	62

4.2.2.9.2	Measurements for UE fulfilling low mobility criterion	62

4.2.2.9.3	Measurements for UE fulfilling not-at-cell edge criterion	63

4.2.2.9.4	Measurements for UE fulfilling low mobility and not-at-cell edge criteria	64

4.2.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	64

4.2.2.10.1	Introduction	64

4.2.2.10.2	Measurements for UE fulfilling low mobility criterion	64

4.2.2.10.3	Measurements for UE fulfilling not-at-cell edge criterion	65

4.2.2.10.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	66

4.2.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	67

4.2.2.11.1	Introduction	67

4.2.2.11.2	Measurements for UE fulfilling low mobility criterion	67

4.2.2.11.3	Measurements for UE fulfilling with not-at-cell edge criterion	68

4.2.2.11.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	69

4.2A	Cell Re-selection when subject to CCA	69

4.2A.1	Introduction	69

4.2A.2	Requirements	70

4.2A.2.1	UE measurement capability	70

4.2A.2.2	Measurement and evaluation when subject to CCA on the serving cell	70

4.2A.2.3	Measurements of intra-frequency NR cells when subject to CCA on the serving cell and target cell	71

4.2A.2.4	Measurements of inter-frequency NR cells when subject to CCA on the target cell	72

4.2A.2.5	Measurements of inter-RAT E-UTRAN cells when subject to CCA on the serving cell	74

4.2A.2.6	Maximum interruption in paging reception when subject to CCA on the target cell	74

4.2A.2.7	General requirements	74

4.3	Minimization of Drive Tests (MDT)	75

4.3.1	Introduction	75

4.3.2	Measurement Requirements	75

4.3.3	Requirements for Relative Time Stamp Accuracy	75

4.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	76

4.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	76

4.4	Idle Mode CA/DC Measurements	76

4.4.1	Introduction	76

4.4.2	Measurement Requirements	76

4.4.2.1	Detected cell requirement during state transition and Idle mode	76

4.4.2.2	Measurements of inter-frequency CA/DC candidate cells	77

4.4.2.3	Measurements on serving cell	78

4.4.2.4	Measurements of E-UTRAN inter-RAT DC candidate cells	78

5	SA: RRC_INACTIVE state mobility	80

5.1	Cell Re-selection	80

5.1.1	Introduction	80

5.1.2	Requirements	80

5.1.2.1	UE measurement capability	80

5.1.2.2	Measurement and evaluation of serving cell	80

5.1.2.3	Measurements of intra-frequency NR cells	80

5.1.2.4	Measurements of inter-frequency NR cells	80

5.1.2.5	Measurements of inter-RAT E-UTRAN cells	80

5.1.2.6	Maximum interruption in paging reception	80

5.1.2.7	General requirements	80

5.1A	Cell Re-selection with CCA	80

5.1A.1	Introduction	80

5.1A.2	Requirements	81

5.1A.2.1	UE measurement capability	81

5.1A.2.2	Measurement and evaluation when CCA is used on the serving cell	81

5.1A.2.3	Measurements of intra-frequency NR cells when CCA is used on the serving cell and target cell	81

5.1A.2.4	Measurements of inter-frequency NR cells when CCA is used on the target cell	81

5.1A.2.5	Measurements of inter-RAT E-UTRAN cells when CCA is used on the serving cell	81

5.1A.2.6	Maximum interruption in paging reception when CCA is used on the target cell	81

5.1A.2.7	General requirements	81

5.2	Void	81

5.3	Minimization of Drive Tests (MDT)	81

5.3.1	Introduction	81

5.3.2	Measurement Requirements	82

5.3.3	Requirements for Relative Time Stamp Accuracy	82

5.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	82

5.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	82

5.3.6	Requirements for Relative Time Stamp Accuracy for RRC Resume Failure Log Reporting	82

5.4	Idle Mode CA/DC Measurements	82

5.4.1	Introduction	82

5.4.2	Measurement Requirements	82

5.4.2.1	Detected cell requirement during state transition and Idle mode	83

5.4.2.2	Measurements of inter-frequency CA/DC candidate cells	83

5.4.2.3	Measurements on serving cell	83

5.4.2.4	Measurements on E-UTRAN inter-RAT DC candidate cells	83

6	RRC_CONNECTED state mobility	83

6.1	Handover	83

6.1.1	NR Handover	83

6.1.1.1	Introduction	83

6.1.1.2	NR FR1 - NR FR1 Handover	83

6.1.1.2.1	Handover delay	83

6.1.1.2.2	Interruption time	83

6.1.1.3	NR FR2- NR FR1 Handover	84

6.1.1.3.1	Handover delay	84

6.1.1.3.2	Interruption time	84

6.1.1.4	NR FR2- NR FR2 Handover	85

6.1.1.4.1	Handover delay	85

6.1.1.4.2	Interruption time	85

6.1.1.5	NR FR1- NR FR2 Handover	86

6.1.1.5.1	Handover delay	86

6.1.1.5.2	Interruption time	86

6.1.2	NR Handover to other RATs	87

6.1.2.1	NR – E-UTRAN Handover	87

6.1.2.1.1	Introduction	87

6.1.2.1.2	Handover delay	87

6.1.2.1.3	Interruption time	87

6.1.2.2	NR – UTRAN Handover	88

6.1.2.2.1	Introduction	88

6.1.2.2.2	Handover delay	88

6.1.2.2.3	Interruption time	88

6.1.3	NR DAPS Handover	89

6.1.3.1	Introduction	89

6.1.3.2	NR FR1 - NR FR1 DAPS Handover	89

6.1.3.2.1	DAPS handover delay	89

6.1.3.2.2	Interruption time	90

6.1.3.3	NR FR2- NR FR1 DAPS Handover	92

6.1.3.3.1	DAPS handover delay	92

6.1.3.3.2	Interruption time	92

6.1.3.4	NR FR1- NR FR2 DAPS Handover	93

6.1.3.4.1	DAPS handover delay	93

6.1.3.4.2	Interruption time	93

6.1.4	NR Conditional Handover	94

6.1.4.1	Introduction	94

6.1.4.2	NR FR1 – NR FR1 conditional handover	94

6.1.4.3	NR FR2 – NR FR1 conditional handover	95

6.1.4.4	NR FR2 – NR FR2 conditional handover	96

6.1.4.4.1	Handover delay	96

6.1.4.4.2	Measurement time	96

6.1.4.4.3	Preparation time	96

6.1.4.4.4	Interruption time	96

6.1.4.5	NR FR1 – NR FR2 conditional handover	97

6.1A	Void	97

6.1A.1	Void	97

6.1A.1.1	Void	97

6.1A.1.2	Void	97

6.1A.1.2.1	Void	97

6.1A.1.2.2	Void	97

6.1B	Handover to target cell using CCA	97

6.1B.1	NR Handover	97

6.1B.1.1	Introduction	97

6.1B.1.2	NR FR1 - NR FR1 Handover	98

6.1B.1.2.1	Handover delay	98

6.1B.1.2.2	Interruption time	98

6.2	RRC Connection Mobility Control	99

6.2.1	SA: RRC Re-establishment	99

6.2.1.1	Introduction	99

6.2.1.2	Requirements	99

6.2.1.2.1	UE Re-establishment delay requirement	99

6.2.1A	RRC Re-establishment with CCA	100

6.2.1A.1	Introduction	100

6.2.1A.2	Requirements	101

6.2.1A.2.1	UE Re-establishment with CCA delay requirement	101

6.2.2	Random access	103

6.2.2.1	Introduction	103

6.2.2.2	Requirements for 4-step RA type	103

6.2.2.2.1	Contention based random access	103

6.2.2.2.2	Non-Contention based random access	104

6.2.2.2.3	UE behaviour when configured with supplementary UL	105

6.2.2.3	Requirements for 2-step RA type	105

6.2.2.3.1	Contention based random access	105

6.2.2.3.2	Non-Contention based random access	106

6.2.2.3.3	UE behaviour when configured with supplementary UL	107

6.2.2A	Random access when CCA is used on target frequency	107

6.2.2A.1	Introduction	107

6.2.2A.2	Requirements for 4-step RA type	107

6.2.2A.2.1	Contention based random access	107

6.2.2A.2.2	Non-Contention based random access	108

6.2.2A.3	Requirements for 2-step RA type	109

6.2.2A.3.1	Contention based random access	110

6.2.2A.3.2	Non-Contention based random access	111

6.2.3	SA: RRC Connection Release with Redirection	112

6.2.3.1	Introduction	112

6.2.3.2	Requirements	112

6.2.3.2.1	RRC connection release with redirection to NR	112

6.2.3.2.2	RRC connection release with redirection to E-UTRAN	113

6.2.3.2.3	RRC connection release with redirection to NR carrier subject to CCA	113

7	Timing	114

7.1	UE transmit timing	114

7.1.1	Introduction	114

7.1.2	Requirements	115

7.1.2.1	Gradual timing adjustment	116

7.1.2.2	Void	116

7.2	UE timer accuracy	116

7.2.1	Introduction	116

7.2.2	Requirements	117

7.3	Timing advance	117

7.3.1	Introduction	117

7.3.2	Requirements	117

7.3.2.1	Timing Advance adjustment delay	117

7.3.2.2	Timing Advance adjustment accuracy	117

7.4	Cell phase synchronization accuracy	117

7.4.1	Definition	117

7.4.2	Minimum requirements	117

7.5	Maximum Transmission Timing Difference	118

7.5.1	Introduction	118

7.5.2	Minimum Requirements for inter-band EN-DC	118

7.5.2.1	Minimum Requirements for inter-band synchronous EN-DC	118

7.5.3	Minimum Requirements for intra-band EN-DC	119

7.5.4	Minimum Requirements for NR Carrier Aggregation	119

7.5.5	Minimum Requirements for inter-band NE-DC	120

7.5.5.1	Minimum Requirements for inter-band synchronous NE-DC	120

7.5.6	Minimum Requirements for inter-band NR DC	121

7.6	Maximum Receive Timing Difference	121

7.6.1	Introduction	121

7.6.2	Minimum Requirements for inter-band EN-DC	122

7.6.2.1	Minimum Requirements for inter-band synchronous EN-DC	122

7.6.3	Minimum Requirements for intra-band EN-DC	122

7.6.4	Minimum Requirements for NR Carrier Aggregation	123

7.6.5	Minimum Requirements for inter-band NE-DC	124

7.6.5.1	Minimum Requirements for inter-band synchronous NE-DC	124

7.6.6	Minimum Requirements for inter-band NR DC	124

7.7	deriveSSB-IndexFromCell tolerance	125

7.7.1	Minimum requirements	125

7.8	Void	125

8	Signalling characteristics	125

8.1	Radio Link Monitoring	125

8.1.1	Introduction	125

8.1.2	Requirements for SSB based radio link monitoring	126

8.1.2.1	Introduction	126

8.1.2.2	Minimum requirement	127

8.1.2.3	Measurement restrictions for SSB based RLM	129

8.1.3	Requirements for CSI-RS based radio link monitoring	129

8.1.3.1	Introduction	129

8.1.3.2	Minimum requirement	130

8.1.3.3	Measurement restrictions for CSI-RS based RLM	132

8.1.4	Minimum requirement at transitions	133

8.1.5	Minimum requirement for UE turning off the transmitter	133

8.1.6	Minimum requirement for L1 indication	133

8.1.7	Scheduling availability of UE during radio link monitoring	134

8.1.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	134

8.1.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	134

8.1.7.3	Scheduling availability of UE performing radio link monitoring on FR2	134

8.1.7.4	Scheduling availability of UE performing radio link monitoring on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	135

8.1A	Radio Link Monitoring with CCA on Target Frequency	135

8.1A.1	Introduction	135

8.1A.2	Requirements for SSB Based Radio Link Monitoring	136

8.1A.2.1	Introduction	136

8.1A.2.2	Minimum Requirement	137

8.1A.3	Minimum requirement at transitions	138

8.1A.4	Minimum requirement for UE turning off the transmitter	139

8.1A.5	Minimum requirement for L1 indication	139

8.1A.6	Scheduling availability of UE during radio link monitoring	139

8.1A.6.1	Scheduling availability of UE performing radio link monitoring with the same subcarrier spacing as PDSCH/PDCCH	139

8.1A.6.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH	139

8.2	Interruption	140

8.2.1	EN-DC Interruption	140

8.2.1.1	Introduction	140

8.2.1.2	Requirements	140

8.2.1.2.1	Interruptions at transitions between active and non-active during DRX	140

8.2.1.2.2	Interruptions at transitions from non-DRX to DRX	141

8.2.1.2.3	Interruptions at SCell addition/release	141

8.2.1.2.4	Interruptions at SCell activation/deactivation	142

8.2.1.2.5	Interruptions during measurements on SCC	143

8.2.1.2.6	Interruptions at UL carrier RRC reconfiguration	145

8.2.1.2.7	Interruptions due to Active BWP switching Requirement	145

8.2.1.2.8	Interruptions at direct SCell activation and hibernation	146

8.2.1.2.9	Interruptions at SCell hibernation	147

8.2.1.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	147

8.2.1.2.11	Interruptions due to UE-specific CBW change	147

8.2.1.2.12	 Interruptions at NR SRS carrier based switching	148

8.2.1.2.13	 Interruptions at E-UTRA SRS carrier based switching	149

8.2.1.2.14	DL Interruptions at switching between two uplink carriers	150

8.2.1.2.15	Interruptions due to SCell dormancy	150

8.2.1.2.16	Interruptions when identifying CGI of an NR cell with autonomous gaps	151

8.2.1.2.17	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	151

8.2.2	SA: Interruptions with Standalone NR Carrier Aggregation	152

8.2.2.1	Introduction	152

8.2.2.2	Requirements	153

8.2.2.2.1	Interruptions at SCell addition/release	153

8.2.2.2.2	Interruptions at SCell activation/deactivation	154

8.2.2.2.3	Interruptions during measurements on deactivated SCC	155

8.2.2.2.4	Interruptions at UL carrier RRC reconfiguration	155

8.2.2.2.5	Interruptions due to Active BWP switching Requirement	156

8.2.2.2.6	Interruptions at inter-frequency SFTD measurement	157

8.2.2.2.7	Interruptions at SCell activation/deactivation with multiple downlink SCells	158

8.2.2.2.8	Interruptions due to UE-specific CBW change	158

8.2.2.2.9	 Interruptions at NR SRS carrier based switching	158

8.2.2.2.10	DL Interruptions at UE switching between two uplink carriers	160

8.2.2.2.11	Interruptions at direct SCell activation	160

8.2.2.2.12	Interruptions due to SCell dormancy	161

8.2.2.2.13	Interruptions at transitions between active and non-active during DRX	161

8.2.2.2.14	Interruptions when identifying CGI of an NR cell with autonomous gaps	161

8.2.2.2.15	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	162

8.2.3	NE-DC Interruptions	162

8.2.3.1	Introduction	162

8.2.3.2	Requirements	163

8.2.3.2.1	Interruptions at transitions between active and non-active during DRX	163

8.2.3.2.2	Interruptions at transitions from non-DRX to DRX	163

8.2.3.2.3	Interruptions at PSCell/SCell addition/release	163

8.2.3.2.4	Interruptions at SCell activation/deactivation	165

8.2.3.2.5	Interruptions during measurements on SCC	166

8.2.3.2.6	Interruptions at UL carrier RRC reconfiguration	167

8.2.3.2.7	Interruptions due to Active BWP switching Requirement	167

8.2.3.2.8	Interruptions at direct SCell activation and hibernation	168

8.2.3.2.9	Interruptions at SCell hibernation	168

8.2.3.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	168

8.2.3.2.11	 Interruptions at NR SRS carrier based switching	168

8.2.3.2.12	 Interruptions at E-UTRA SRS carrier based switching	170

8.2.3.2.13	Interruptions due to SCell dormancy	171

8.2.3.2.14	Interruptions when identifying CGI of an NR cell with autonomous gaps	171

8.2.3.2.15	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	172

8.2.3.2.16	Interruptions due to UE-specific CBW change	172

8.2.4	NR-DC: Interruptions	173

8.2.4.1	Introduction	173

8.2.4.2	Requirements	173

8.2.4.2.1	Interruptions at PSCell/SCell addition/release	173

8.2.4.2.3	Interruptions during measurements on SCC	175

8.2.4.2.4	Interruptions at UL carrier RRC reconfiguration	175

8.2.4.2.5	Interruptions due to Active BWP switching Requirement	176

8.2.4.2.6	Interruptions at transitions between active and non-active during DRX	176

8.2.4.2.7	Interruptions at transitions from non-DRX to DRX	176

8.2.4.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	176

8.2.4.2.9	Interruptions at NR SRS carrier based switching	177

8.2.4.2.10	Interruptions at direct SCell activation	178

8.2.4.2.11	Interruptions when identifying CGI of an NR cell with autonomous gaps	178

8.2.4.2.12	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	179

8.2.4.2.13	 Interruptions due to SCell dormancy	180

8.2.4.2.14	Interruptions due to UE-specific CBW change	180

8.2.4.2A	Void	180

8.2.4.2A.1	Void	180

8.2.4.2A.2	Void	180

8.2.4.2A.3	Void	180

8.3	SCell Activation and Deactivation Delay	180

8.3.1	Introduction	180

8.3.2	SCell Activation Delay Requirement for Deactivated SCell	181

8.3.3	SCell Deactivation Delay Requirement for Activated SCell	185

8.3.4	Direct SCell Activation at SCell addition	185

8.3.5	Direct SCell Activation at Handover	187

8.3.7	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	189

8.3.8	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	192

8.3.9	Direct SCell Activation of Multiple Downlink SCells at SCell addition	193

8.3.10	Direct SCell Activation of Multiple Downlink SCells at Handover	194

8.3A	SCell Activation and Deactivation Delay in Carriers with CCA	195

8.3A.1	Introduction	195

8.3A.2	SCell Activation Delay Requirement for Deactivated SCell	196

8.3A.3	SCell Deactivation Delay Requirement for Activated SCell	198

8.4	UE UL carrier RRC reconfiguration delay	199

8.4.1	Introduction	199

8.4.2	UE UL carrier configuration delay requirement	199

8.4.3	UE UL carrier deconfiguration delay requirement	199

8.5	Link Recovery Procedures	199

8.5.1	Introduction	199

8.5.2	Requirements for SSB based beam failure detection	200

8.5.2.1	Introduction	200

8.5.2.2	Minimum requirement	201

8.5.2.3	Measurement restriction for SSB based beam failure detection	202

8.5.3	Requirements for CSI-RS based beam failure detection	203

8.5.3.1	Introduction	203

8.5.3.2	Minimum requirement	203

8.5.3.3	Measurement restrictions for CSI-RS beam failure detection	205

8.5.4	Minimum requirement for L1 indication	206

8.5.5	Requirements for SSB based candidate beam detection	207

8.5.5.1	Introduction	207

8.5.5.2	Minimum requirement	207

8.5.5.3	Measurement restriction for SSB based candidate beam detection	209

8.5.6	Requirements for CSI-RS based candidate beam detection	209

8.5.6.1	Introduction	209

8.5.6.2	Minimum requirement	209

8.5.6.3	Measurement restriction for CSI-RS based candidate beam detection	212

8.5.7	Scheduling availability of UE during beam failure detection	212

8.5.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	212

8.5.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	212

8.5.7.3	Scheduling availability of UE performing beam failure detection on FR2	213

8.5.7.4	Scheduling availability of UE performing beam failure detection on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR DC	213

8.5.8	Scheduling availability of UE during candidate beam detection	213

8.5.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	213

8.5.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	214

8.5.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	214

8.5.8.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	214

8.5.9	Requirements for Beam Failure Recovery in SCell	215

8.5.9.1	Introduction	215

8.5.9.2	Requirement	215

8.5.10	Minimum requirement at transitions for beam failure detection	215

8.5A	Link Recovery Procedures when CCA is used on target frequency	216

8.5A.1	Introduction	216

8.5A.2	Requirements for SSB based beam failure detection	216

8.5A.2.1	Introduction	216

8.5A.2.2	Minimum requirement	217

8.5A.2.3	Measurement restriction for SSB based beam failure detection	217

8.5A.4	Minimum requirement for L1 indication	218

8.5A.5	Requirements for SSB based candidate beam detection	218

8.5A.5.1	Introduction	218

8.5A.5.2	Minimum requirement	218

8.5A.5.3	Measurement restriction for SSB based candidate beam detection	219

8.5A.7	Scheduling availability of UE during beam failure detection	219

8.5A.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH	219

8.5A.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH	219

8.5A.8	Scheduling availability of UE during candidate beam detection	219

8.5A.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH	220

8.5A.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH	220

8.6	Active BWP switch delay	220

8.6.1	Introduction	220

8.6.2	DCI and timer based BWP switch delay on a single CC	220

8.6.2A	DCI based BWP switch delay on multiple CCs	221

8.6.2A.1	Simultaneous DCI based BWP switch delay on multiple CCs	221

8.6.2A.2	Non-simultaneous DCI based BWP switch delay on multiple CCs	223

8.6.2B	Timer based BWP switch delay on multiple CCs	223

8.6.2B.1	Simultaneous timer based BWP switch delay on multiple CCs	223

8.6.2B.2	Non-simultaneous timer based BWP switch delay on multiple CCs	224

8.6.3	RRC based BWP switch delay on a single CC	224

8.6.3A	RRC based BWP switch delay on multiple CCs	225

8.6.3A.1	Simultaneous RRC based BWP switch delay on multiple CCs	225

8.6.3A.2	Non-simultaneous RRC based BWP switch delay on multiple CCs	225

8.6.4	BWP switch delay on Consistent UL CCA recovery	226

8.7	Void	226

8.8	NE-DC: E-UTRAN PSCell Addition and Release Delay	226

8.8.1	Introduction	226

8.8.2	E-UTRAN PSCell Addition Delay Requirement	226

8.8.3	E-UTRAN PSCell Release Delay Requirement	227

8.9	NR-DC: PSCell Addition and Release Delay	227

8.9.1	Introduction	227

8.9.2	PSCell Addition Delay Requirement	227

8.9.3	PSCell Release Delay Requirement	228

8.10	Active TCI state switching delay	228

8.10.4	DCI based TCI state switch delay	230

8.10.5	RRC based TCI state switch delay	230

8.10.6	Active TCI state list update delay	230

8.10A	Active TCI state switching delay with CCA	231

8.10A.1	Introduction	231

8.10A.2	Known conditions for TCI state	231

8.10A.3	MAC-CE based TCI state switch delay	231

8.10A.4	DCI based TCI state switch delay	232

8.10A.5	RRC based TCI state switch delay	232

8.10A.6	Active TCI state list update delay	233

8.11	PSCell Change	233

8.11A	void	234

8.11B	Conditional PSCell Change	234

8.11B.1	Introduction	234

8.11B.2	Conditoinal PSCell Change delay	234

8.11B.2.1	Measurement time	234

8.12	Uplink spatial relation switch delay	235

8.12.1	Introduction	235

8.12.2	Known conditions for spatial relation when associated with DL-RS	235

8.12.3	MAC-CE based spatial relation switch delay	235

8.12.4	DCI based spatial relation switch delay	236

8.12.5	RRC based spatial relation switch delay	236

8.13	UE-specific CBW change	237

8.13.1	Introduction	237

8.13.2	UE-specific CBW change delay	237

8.14	Pathloss reference signal switching delay	237

8.14.1	Introduction	237

8.14.2	Known conditions for pathloss reference signal	237

8.14.3	MAC-CE based pathloss reference signal switch delay	238

9	Measurement Procedure	239

9.1	General measurement requirement	239

9.1.1	Introduction	239

9.1.2	Measurement gap	239

9.1.2.1	EN-DC: Measurement Gap Sharing	250

9.1.2.1a	SA: Measurement Gap Sharing	250

9.1.2.1b	NE-DC: Measurement Gap Sharing	251

9.1.2.1c	NR-DC: Measurement Gap Sharing	252

9.1.3	UE Measurement capability	253

9.1.3.1	EN-DC: Monitoring of multiple layers using gaps	253

9.1.3.1a	SA: Monitoring of multiple layers using gaps	253

9.1.3.1b	NE-DC: Monitoring of multiple layers using gaps	254

9.1.3.1c	NR-DC: Monitoring of multiple layers using gaps	254

9.1.3.2	EN-DC: Maximum allowed layers for multiple monitoring	255

9.1.3.2a	SA: Maximum allowed layers for multiple monitoring	256

9.1.3.2b	NE-DC: Maximum allowed layers for multiple monitoring	256

9.1.3.2c	NR-DC: Maximum allowed layers for multiple monitoring	257

9.1A.3.2	Void	258

9.1.3A	UE Measurement capability under operation mode with CCA	258

9.1.3A.1	EN-DC: Monitoring of multiple layers using gaps under CCA	258

9.1.3A.1A	SA: Monitoring of multiple layers using gaps under CCA	258

9.1.3A.2	EN-DC: Maximum allowed layers for multiple monitoring under CCA	258

9.1A.3.2a	Void	259

9.1.3A.2A	SA: Maximum allowed layers for multiple monitoring under CCA	259

9.1.4	Capabilities for Support of Event Triggering and Reporting Criteria	259

9.1.4.1	Introduction	259

9.1.4.2	Requirements	259

9.1.5	Carrier-specific scaling factor	263

9.1.5.1	Monitoring of multiple layers outside gaps	263

9.1.5.1.1	EN-DC mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	265

9.1.5.1.2	SA mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	266

9.1.5.1.3	NR-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed outside gaps	267

9.1.5.1.4	NE-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based measurements performed outside gaps	268

9.1.5.2	Monitoring of multiple layers within gaps	269

9.1.5.2.1	EN-DC mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	270

9.1.5.2.2	SA mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	272

9.1.5.2.3	NE-DC: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed within gaps	274

9.1.5.2.4	NR-DC: carrier-specific scaling factor for SSB-based and CSI-RS-based L3 measurements performed within gaps	276

9.1.5.2.5	SA mode: carrier-specific scaling factor for PRS-based measurements performed within gaps	277

9.1.5.2.6	NE-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	278

9.1.5.2.7	NR-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	278

9.1.6	Minimum requirement at transitions	278

9.2	NR intra-frequency measurements	278

9.2.1	Introduction	278

9.2.2	Requirements applicability	279

9.2.3	Number of cells and number of SSB	279

9.2.3.1	Requirements for FR1	279

9.2.3.2	Requirements for FR2	279

9.2.4	Measurement Reporting Requirements	280

9.2.4.1	Periodic Reporting	280

9.2.4.2	Event-triggered Periodic Reporting	280

9.2.4.3	Event Triggered Reporting	280

9.2.5	Intrafrequency measurements without measurement gaps	281

9.2.5.1	Intrafrequency cell identification	281

9.2.5.2	Measurement period	283

9.2.5.3	Scheduling availability of UE during intra-frequency measurements	285

9.2.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	285

9.2.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	285

9.2.5.3.3	Scheduling availability of UE performing measurements on FR2	286

9.2.5.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	287

9.2.5.4	SFTD Measurements between PCell and PSCell	287

9.2.5.4.1	Introduction	287

9.2.5.4.2	SFTD Measurement delay	287

9.2.5.4.3	SFTD Measurement Reporting Delay	288

9.2.6	Intra-frequency measurements with measurement gaps	288

9.2.6.1	Void	288

9.2.6.2	Intra-frequency cell identification	288

9.2.6.3	Intrafrequency Measurement Period	290

9.2A	NR intra-frequency measurements with CCA	291

9.2A.1	Introduction	291

9.2A.2	Requirements applicability	291

9.2A.3	Number of cells and number of SSB	292

9.2A.4	Measurement Reporting Requirements	292

9.2A.5	Intra-frequency measurements without measurement gaps	292

9.2A.5.2	Measurement period	295

9.2A.5.3	Scheduling availability of UE during intra-frequency measurements	296

9.2A.5.3.1	Scheduling availability of UE performing measurements in TDD bands	296

9.2A.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH	297

9.2A.6	Intra-frequency measurements with measurement gaps	297

9.2A.6.1	Intra-frequency cell identification	297

9.2A.6.2	Intra-frequency Measurement Period	298

9.2A.7	Intra-frequency RSSI and Channel occupancy measurements	299

9.2A.7.1	Intra-frequency RSSI measurements	299

9.2A.7.2	Intra-frequency Channel occupancy measurements	300

9.2A.7.3	Scheduling restriction during RSSI and Channel Occupancy measurements	301

9.3	NR inter-frequency measurements	301

9.3.1	Introduction	301

9.3.2	Requirements applicability	302

9.3.2.1	Void	302

9.3.2.2	Void	302

9.3.3	Number of cells and number of SSB	302

9.3.3.1	Requirements for FR1	302

9.3.3.2	Requirements for FR2	302

9.3.4	Inter-frequency measurement with measurement gaps	302

9.3.4.1	Void	304

9.3.4.2	Void	304

9.3.5	Inter-frequency measurements	304

9.3.5.1	Void	304

9.3.5.2	Void	304

9.3.5.3	Void	304

9.3.6	Inter-frequency measurements reporting requirements	304

9.3.6.1	Periodic Reporting	304

9.3.6.2	Event-triggered Periodic Reporting	305

9.3.6.3	Event-triggered Reporting	305

9.3.7	Void	305

9.3.8	Inter-frequency SFTD measurement requirements	305

9.3.8.1	Introduction	305

9.3.8.2	SFTD Measurement delay	305

9.3.8.3	SFTD Measurement reporting delay	306

9.3.9	Inter frequency measurements without measurement gaps	307

9.3.9.1	Inter frequency Cell identification	307

9.3.9.2	Measurement period	308

9.3.9.3	Scheduling availability of UE during inter-frequency measurements	309

9.3.9.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	309

9.3.9.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	310

9.3.9.3.3	Scheduling availability of UE performing measurements on FR2	310

9.3.9.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	310

9.3A	NR inter-frequency measurements in carrier frequencies with CCA	311

9.3A.1	Introduction	311

9.3A.2	Requirements applicability	311

9.3A.3	Number of cells and number of SSB	311

9.3A.3.1	Requirements	311

9.3A.4	Inter-frequency cell identification	312

9.3A.5	Inter-frequency measurements	313

9.3A.6	NR Inter-frequency measurements reporting requirements	314

9.3A.6.1	Periodic Reporting	314

9.3A.6.2	Event-triggered Periodic Reporting	314

9.3A.6.3	Event-triggered Reporting	314

9.3A.8	Inter-frequency RSSI measurements	314

9.3A.9	Inter-frequency channel occupancy measurements	315

9.4	Inter-RAT measurements	315

9.4.1	Introduction	315

9.4.2	NR − E-UTRAN FDD measurements	317

9.4.2.1	Introduction	317

9.4.2.2	Requirements when no DRX is used	317

9.4.2.3	Requirements when DRX is used	318

9.4.2.4	Measurement reporting requirements	319

9.4.2.4.1	Periodic Reporting	319

9.4.2.4.2	Event-Triggered Periodic Reporting	319

9.4.2.4.3	Event-Triggered Reporting	319

9.4.3	NR − E-UTRAN TDD measurements	319

9.4.3.1	Introduction	319

9.4.3.2	Requirements when no DRX is used	319

9.4.3.3	Requirements when DRX is used	320

9.4.3.4	Measurement reporting requirements	322

9.4.3.4.1	Periodic Reporting	322

9.4.3.4.2	Event-Triggered Periodic Reporting	322

9.4.3.4.3	Event-Triggered Reporting	322

9.4.4	Inter-RAT RSTD measurements	322

9.4.4.1	NR − E-UTRAN FDD RSTD measurements	322

9.4.4.1.1	Introduction	322

9.4.4.1.2	Requirements	323

9.4.4.2	NR − E-UTRAN TDD RSTD measurements	326

9.4.4.2.1	Introduction	326

9.4.4.2.2	Requirements	327

9.4.5	Inter-RAT E-CID measurements	330

9.4.5.1	NR−E-UTRAN FDD E-CID RSRP and RSRQ measurements	330

9.4.5.1.1	Introduction	330

9.4.5.1.2	Requirements	330

9.4.5.1.3	Measurement Reporting Delay	330

9.4.5.2	NR−E-UTRAN TDD E-CID RSRP and RSRQ measurements	330

9.4.5.2.1	Introduction	330

9.4.5.2.2	Requirements	330

9.4.5.2.3	Measurement Reporting Delay	331

9.4.6	NR − UTRAN FDD measurements	331

9.4.6.1	Introduction	331

9.4.6.2	Requirements when no DRX is used	331

9.4.6.3	Requirements when DRX is used	332

9.4.7	NR – E-UTRAN measurements with autonomous gaps	334

9.4.7.1	CGI identification of an E-UTRA cell with autonomous gaps	334

9.4.7.2	CGI reporting delay	334

9.5	L1-RSRP measurements for Reporting	335

9.5.1	Introduction	335

9.5.2	Requirements applicability	335

9.5.3	Measurement Reporting Requirements	335

9.5.3.1	Periodic Reporting	336

9.5.3.2	Semi-Persistent Reporting	336

9.5.3.3	Aperiodic Reporting	336

9.5.4	L1-RSRP measurement requirements	336

9.5.4.1	SSB based L1-RSRP Reporting	336

9.5.4.2	CSI-RS based L1-RSRP Reporting	338

9.5.4A	Void	341

9.5.4A.1	Void	341

9.5.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	341

9.5.5.1	Measurement restriction for SSB based L1-RSRP	341

9.5.5.2	Measurement restriction for CSI-RS based L1-RSRP	341

9.5.6	Scheduling availability of UE during L1-RSRP measurement	342

9.5.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	342

9.5.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	342

9.5.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	343

9.5.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	343

9.5A	L1-RSRP measurements for Reporting under CCA	344

9.5A.1	Introduction	344

9.5A.2	Requirements applicability	344

9.5A.3	Measurement Reporting Requirements	344

9.5A.3.1	Periodic Reporting	344

9.5A.3.2	Semi-Persistent Reporting	345

9.5A.3.3	Aperiodic Reporting	345

9.5A.4	L1-RSRP measurement requirements	345

9.5A.4.1	SSB based L1-RSRP Reporting	345

9.5A.5	Measurement restriction for L1-RSRP measurement	346

9.5A.5.1	Measurement restriction for SSB based L1-RSRP	346

9.5A.6	Scheduling availability of UE during L1-RSRP measurement	346

9.5A.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH	346

9.5A.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH	347

9.5A.6.3	Scheduling availability of UE performing L1-RSRP measurement in case of FR1-FR2 inter-band CA	347

9.6	NE-DC: Measurements	347

9.6.1	Introduction	347

9.6.2	SFTD Measurements	347

9.6.2.1	Introduction	347

9.6.2.2	SFTD Measurement requirements	347

9.7	Cross Link Interference measurements	348

9.7.1	Introduction	348

9.7.2	SRS-RSRP measurements	348

9.7.2.1	Introduction	348

9.7.2.2	Requirements applicability	348

9.7.2.3	Measurement Reporting Requirements	349

9.7.2.3.1	Periodic Reporting	349

9.7.2.3.2	Event-triggered Periodic Reporting	349

9.7.2.3.3	Event Triggered Reporting	349

9.7.2.4	Measurement capability	349

9.7.2.5	SRS-RSRP measurement period	349

9.7.3	CLI-RSSI measurements	350

9.7.3.1	Introduction	350

9.7.3.2	Requirements applicability	350

9.7.3.3	Measurement Reporting Requirements	350

9.7.3.3.1	Periodic Reporting	350

9.7.3.3.2	Event-triggered Periodic Reporting	350

9.7.3.3.3	Event Triggered Reporting	350

9.7.3.4	Measurement capability	351

9.7.3.5	CLI-RSSI measurement period	351

9.7.4	Scheduling availability of UE during CLI measurements	351

9.7.4.1	Scheduling availability of UE performing measurement on FR1	351

9.7.4.2	Scheduling availability of UE performing measurement on FR2	351

9.8	L1-SINR measurements for Reporting	352

9.8.1	Introduction	352

9.8.2	Requirements applicability	352

9.8.3	Measurement Reporting Requirements	353

9.8.3.1	Periodic Reporting	353

9.8.3.2	Semi-Persistent Reporting	353

9.8.3.3	Aperiodic Reporting	354

9.8.4	L1-SINR measurement requirements	354

9.8.4.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	354

9.8.4.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	357

9.8.5	Measurement restriction for L1-SINR measurement	359

9.8.5.1	Measurement restriction if SSB configured for L1-SINR Measurement	359

9.8.5.2	Measurement restriction if CSI-RS configured for L1-SINR measurement	360

9.8.5.3	Measurement restriction if CSI-IM configured for L1-SINR measurement	360

9.8.6	Scheduling availability of UE during L1-SINR measurement	361

9.8.6.1	Scheduling availability of UE performing L1-SINR measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	361

9.8.6.2	Scheduling availability of UE performing L1-SINR measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	361

9.8.6.4	Scheduling availability of UE performing L1-SINR measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	362

9.9	 NR measurements for positioning	362

9.9.1	Introduction	362

9.9.2	RSTD measurements	363

9.9.2.1	Introduction	363

9.9.2.2	Requirements Applicability	363

9.9.2.3	Measurement Capability	363

9.9.2.4	Measurement Reporting Requirements	363

9.9.2.4.1	Void	363

9.9.2.4.2	Void	363

9.9.2.4.3	Void	363

9.9.2.5	Measurements Period Requirements	363

9.9.2.6	Void	365

9.9.3	PRS-RSRP measurements	365

9.9.3.1	Introduction	365

9.9.3.2	Requirements applicability	365

9.9.3.3	Measurement Capability	366

9.9.3.4	Measurement Reporting Requirements	366

9.9.3.5	Measurement Period Requirements	366

9.9.4	UE Rx-Tx time difference measurements	368

9.9.4.1 Introduction	368

9.9.4.2 Requirements Applicability	368

9.9.4.3 Measurement Capability	368

9.9.4.4 Measurement Reporting Requirements	368

9.9.4.5	Measurement Period Requirements	368

9.9.5	NR E-CID measurements	371

9.9.5.1	Introduction	371

9.9.5.2	Measurement Requirements	371

9.9.5.2.1	Intra-frequency Measurement Requirements	371

9.9.5.2.2	Inter-frequency Measurement Requirements	371

9.9.5.2.3	Measurement Reporting Delay	372

9.10	CSI-RS based L3 measurements	372

9.10.1	Introduction	372

9.10.2	CSI-RS based intra-frequency measurements	372

9.10.2.1	Introduction	372

9.10.2.2	Requirements applicability	373

9.10.2.3	Number of cells and number of CSI-RS	374

9.10.2.3.1	Requirements for FR1	374

9.10.2.3.2	Requirements for FR2	374

9.10.2.4	Measurement Reporting Requirements	374

9.10.2.4.1	Periodic Reporting	375

9.10.2.4.2	Event-triggered Periodic Reporting	375

9.10.2.4.3	Event Triggered Reporting	375

9.10.2.5	Intra-frequency measurements without measurement gaps	375

9.10.2.6	Scheduling availability of UE during CSI-RS based intra-frequency measurements	376

9.10.2.6.1	Scheduling availability of UE performing CSI-RS based measurements in TDD bands	376

9.10.2.6.2	Scheduling availability of UE performing CSI-RS based measurements in FR2	377

9.10.3	CSI-RS based Inter-frequency measurements	377

9.10.3.1	Introduction	377

9.10.3.2	Requirements applicability	377

9.10.3.3	Number of cells and number of CSI-RS resources	378

9.10.3.3.1	Requirements for FR1	378

9.10.3.3.2	Requirements for FR2	378

9.10.3.4	Measurements reporting requirements	378

9.10.3.4.1	Periodic Reporting	378

9.10.3.4.2	Event-triggered Periodic Reporting	378

9.10.3.4.3	Event-triggered Reporting	379

9.10.3.5	Inter frequency measurements with measurement gaps	379

9.11	NR measurements with autonomous gaps	380

9.11.1	Introduction	380

9.11.2	CGI identification of an NR cell with autonomous gaps	380

9.11.3	CGI reporting delay	381

10	Measurement Performance requirements	381

10.1	NR measurements	381

10.1.1	Introduction	381

10.1.2	Intra-frequency RSRP accuracy requirements for FR1	382

10.1.2.1	Intra-frequency SS-RSRP accuracy requirements	382

10.1.2.1.1	Absolute SS-RSRP Accuracy	382

10.1.2.1.2	Relative SS-RSRP Accuracy	383

10.1.2.2	Void	384

10.1.2.3	Intra-frequency CSI-RSRP accuracy requirements	384

10.1.2.3.1	Absolute CSI-RSRP Accuracy	384

10.1.2.3.2	Relative CSI-RSRP Accuracy	385

10.1.2B	Intra-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	386

10.1.2B.1	Intra-frequency SS-RSRP accuracy requirements	386

10.1.2B.1.1	Absolute SS-RSRP Accuracy	386

10.1.3	Intra-frequency RSRP accuracy requirements for FR2	387

10.1.3.1	Intra-frequency SS-RSRP accuracy requirements	387

10.1.3.1.1	Absolute SS-RSRP Accuracy	387

10.1.3.1.2	Relative SS-RSRP Accuracy	388

10.1.3.2	Void	389

10.1.3.3	Intra-frequency CSI-RSRP accuracy requirements	389

10.1.3.3.1	Absolute CSI-RSRP Accuracy	389

10.1.3.3.2	Relative CSI-RSRP Accuracy	389

10.1.3B	Intra-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	390

10.1.3B.1	Intra-frequency SS-RSRP accuracy requirements	390

10.1.3B.1.1	Absolute SS-RSRP Accuracy	391

10.1.4	Inter-frequency RSRP accuracy requirements for FR1	391

10.1.4.1	Inter-frequency SS-RSRP accuracy requirements	391

10.1.4.1.1	Absolute Accuracy of SS-RSRP in FR1	391

10.1.4.1.2	Relative Accuracy of SS-RSRP in FR1	392

10.1.4.2	Void	393

10.1.4.3	Inter-frequency CSI-RSRP accuracy requirements	393

10.1.4.3.1	Absolute Accuracy of CSI-RSRP in FR1	393

10.1.4.3.2	Relative Accuracy of CS-RSRP in FR1	394

10.1.4B	Inter-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	395

10.1.4B.1	Inter-frequency SS-RSRP accuracy requirements	395

10.1.4B.1.1	Absolute Accuracy of SS-RSRP in FR1	395

10.1.5	Inter-frequency RSRP accuracy requirements for FR2	396

10.1.5.1	Inter-frequency SS-RSRP accuracy requirements	396

10.1.5.1.1	Absolute SS-RSRP Accuracy	396

10.1.5.1.2	Relative SS-RSRP Accuracy	397

10.1.5.2	Void	398

10.1.5.3	Inter-frequency CSI-RSRP accuracy requirements	398

10.1.5.3.1	Absolute CSI-RSRP Accuracy	398

10.1.5.3.2	Relative CSI-RSRP Accuracy	398

10.1.5B	Inter-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	399

10.1.6 RSRP Measurement Report Mapping	400

10.1.7	Intra-frequency RSRQ accuracy requirements for FR1	402

10.1.7.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	402

10.1.7.1.1	Absolute SS-RSRQ Accuracy in FR1	402

10.1.7.2	Intra-frequency CSI-RSRQ accuracy requirements	403

10.1.7.2.1	Absolute CSI-RSRQ Accuracy	403

10.1.7B	Intra-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	404

10.1.7B.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	404

10.1.7B.1.1	Absolute SS-RSRQ Accuracy in FR1	404

10.1.8	Intra-frequency RSRQ accuracy requirements for FR2	405

10.1.8.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	405

10.1.8.1.1	Absolute SS-RSRQ Accuracy in FR2	405

10.1.8.2	Intra-frequency CSI-RSRQ accuracy requirements	406

10.1.8.2.1	Absolute CSI-RSRQ Accuracy	406

10.1.8B	Intra-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	407

10.1.8B.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	407

10.1.8B.1.1	Absolute SS-RSRQ Accuracy in FR2	407

10.1.9	Inter-frequency RSRQ accuracy requirements for FR1	408

10.1.9.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	408

10.1.9.1.1	Absolute Accuracy of SS-RSRQ in FR1	408

10.1.9.1.2	Relative Accuracy of SS-RSRQ in FR1	409

10.1.9.2	Inter-frequency CSI-RSRQ accuracy requirements	410

10.1.9.2.1	Absolute CSI-RSRQ Accuracy	410

10.1.9.2.2	Relative CSI-RSRQ Accuracy	411

10.1.9B	Inter-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	412

10.1.9B.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	412

10.1.9B.1.1	Absolute Accuracy of SS-RSRQ in FR1	412

10.1.10	Inter-frequency RSRQ accuracy requirements for FR2	413

10.1.10.2	Inter-frequency CSI-RSRQ accuracy requirements	415

10.1.10.2.1	Absolute CSI-RSRQ Accuracy	415

10.1.10.2.2	Relative CSI-RSRQ Accuracy	415

10.1.10B	Inter-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	416

10.1.10B.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	416

10.1.10B.1.1	Absolute Accuracy of SS-RSRQ in FR2	416

10.1.11	RSRQ report mapping	417

10.1.12	Intra-frequency SINR accuracy requirements for FR1	418

10.1.12.2	Intra-frequency CSI-SINR accuracy requirements in FR1	418

10.1.12.2.1	Absolute CSI-SINR Accuracy in FR1	418

10.1.13	Intra-frequency SINR accuracy requirements for FR2	419

10.1.13.2	Intra-frequency CSI-SINR accuracy requirements in FR2	420

10.1.13.2.1	Absolute CSI-SINR Accuracy in FR2	420

10.1.14	Inter-frequency SINR accuracy requirements for FR1	421

10.1.14.2	Inter-frequency CSI-SINR accuracy requirements in FR1	423

10.1.14.2.1	Aboslute Accuracy of CSI-SINR in FR1	423

10.1.15	Inter-frequency SINR accuracy requirements for FR2	425

10.1.15.2	Inter-frequency CSI-SINR accuracy requirements in FR2	427

10.1.15.2.1	Aboslute Accuracy of CSI-SINR in FR2	427

10.1.15.2.2	Relative Accuracy of CSI-SINR in FR2	427

10.1.16	SINR report mapping	428

10.1.17	Power Headroom	429

10.1.18	PCMAX,c,f	430

10.1.19	L1-RSRP accuracy requirements for FR1	430

10.1.20	L1-RSRP accuracy requirements for FR2	434

10.1.21	SFTD accuracy requirements	437

10.1.22	CLI measurement accuracy requirements	441

10.1.24.3.1	Absolute PRS-RSRP Measurement Report Mapping	445

10.1.24.3.2	Differential Report Mapping for PRS-RSRP Measurement	445

10.1.23	RSTD Measurements	447

10.1.23.1	Introduction	447

10.1.23.2	Measurement Accuracy Requirements	448

10.1.23.3	Report mapping	454

10.1.23.3.1	 Absolute DL RSTD Measurement Reporting	454

10.1.23.3.2	 Differential Reporting for DL RSTD Measurement	456

10.1.23.3.3	Additional Path Report Mapping for DL RSTD	458

10.1.24	PRS-RSRP Measurements	460

10.1.24.1	Introduction	460

10.1.24.2	Measurement Accuracy Requirements	460

10.1.24.2.1 Absolute PRS RSRP accuracy	460

10.1.24.3	Report mapping	464

10.1.24.3.1	Absolute PRS-RSRP Measurement Report Mapping	464

10.1.24.3.2	Differential Report Mapping for PRS-RSRP Measurement	465

10.1.25	UE Rx-Tx Time Difference Measurements	467

10.1.25.1	Introduction	467

10.1.25.2	Measurement Accuracy Requirements	468

10.1.25.3	Report mapping	474

10.1.25.3.1	 Absolute UE Rx-Tx Measurement Report Mapping	474

10.1.25.3.2	 Differential UE Rx-Tx Measurement Report Mapping	476

10.1.25.3.3	 Additional Path Report Mapping for UE Rx-Tx Time Difference	477

10.1.26	FR2 P-MPR report	479

10.1.26.1	Report mapping	479

10.1.27	L1-SINR accuracy requirements for FR1	479

10.1.27.1	L1-SINR accuracy requirements with CSI-RS based CMR and no dedicated IMR configured	479

10.1.27.1.1	Absolute Accuracy	479

10.1.27.1.2	Relative Accuracy	480

10.1.27.2	L1-SINR accuracy requirements with SSB based CMR and dedicated IMR configured	481

10.1.27.2.1	Absolute Accuracy	481

10.1.27.2.2	Relative Accuracy	482

10.1.27.3	L1-SINR accuracy requirements with CSI-RS based CMR and dedicated IMR configured	484

10.1.27.3.1	Absolute Accuracy	484

10.1.27.3.2	Relative Accuracy	486

10.1.28	L1-SINR accuracy requirements for FR2	487

10.1.28.1	L1-SINR accuracy requirements with CSI-RS based CMR and no dedicated IMR configured	487

10.1.28.1.1	Absolute Accuracy	487

10.1.28.1.2	Relative Accuracy	488

10.1.28.2	L1-SINR accuracy requirements with SSB based CMR and dedicated IMR configured	489

10.1.28.2.1	Absolute Accuracy	489

10.1.28.2.2	Relative Accuracy	490

10.1.28.3	L1-SINR accuracy requirements with CSI-RS based CMR and dedicated IMR configured	491

10.1.28.3.1	Absolute Accuracy	491

10.1.28.3.2	Relative Accuracy	492

10.1.29	Intra-frequency RSRQ accuracy requirements under CCA	494

10.1.29.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	494

10.1.29.1.1	Absolute SS-RSRQ Accuracy	494

10.1.30	Inter-frequency RSRQ accuracy requirements under CCA	495

10.1.30.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	495

10.1.30.1.1	Aboslute Accuracy of SS-RSRQ	495

10.1.30.1.2	Relative Accuracy of SS-RSRQ	495

10.1.31	Intra-frequency SINR accuracy requirements under CCA	496

10.1.32	Inter-frequency SINR accuracy requirements under CCA	497

10.1.33	L1-RSRP accuracy requirements under CCA	498

10.1.34	RSSI measurements under CCA	499

10.1.34.1	Intra-frequency absolute RSSI measurement accuracy requirements in FR1	499

10.1.34.2	Inter-frequency absolute RSSI measurement accuracy requirements in FR1	500

10.1.34.3	RSSI measurement report mapping	500

10.1.35	Channel occupancy measurements under CCA	500

10.1.35.1	Intra-frequency channel occupancy measurement accuracy requirements in FR1	500

10.1.35.2	Inter-frequency channel occupancy measurement accuracy requirements in FR1	501

10.1.36	Intra-frequency RSRP accuracy requirements under CCA	501

10.1.36.1	Intra-frequency SS-RSRP accuracy requirements in FR1	501

10.1.36.1.1	Absolute SS-RSRP Accuracy	501

10.1.36.1.2	Relative SS-RSRP Accuracy	502

10.1.37	Inter-frequency RSRP accuracy requirements under CCA	502

10.1.37.1	Inter-frequency SS-RSRP accuracy requirements in FR1	502

10.1.37.1.1	Absolute Accuracy of SS-RSRP	502

10.1.37.1.2	Relative Accuracy of SS-RSRP	503

10.2	E-UTRAN measurements	503

10.2.1	Introduction	503

10.2.2	E-UTRAN RSRP measurements	504

10.2.3	E-UTRAN RSRQ measurements	504

10.2.4	E-UTRAN RSTD measurements	504

10.2.5	E-UTRAN RS-SINR measurements	505

10.2.6	E-UTRAN RSRP measurements for CA/DC Idle Mode Measurements	505

10.2.7	E-UTRAN RSRQ measurements for CA/DC Idle Mode Measurements	505

10.3	UTRAN FDD Measurements	505

10.3.1	UTRAN FDD CPICH RSCP	506

10.3.2	UTRAN FDD CPICH Ec/No	506

10.4	V2X measurements	507

10.4.1	Introduction	507

10.4.2	Intra-frequency PSBCH-RSRP accuracy requirements for FR1	507

10.4.2.1	PSBCH-RSRP Absolute Accuracy	507

10.4.2.2	PSBCH-RSRP Relative Accuracy	507

10.4.3	Intra-Frequency SL-RSSI Measurement Accuracy Requirements for FR1	508

10.4.3.1	Absolute SL-RSSI Accuracy	508

10.4.4	Intra-Frequency L1 SL-RSRP Measurement Accuracy Requirements for FR1	509

10.4.4.1	Absolute L1 SL-RSRP Accuracy	509

11	Void	510

12	V2X Requirements	510

12.1	Introduction	510

12.2	UE Transmit Timing	510

12.2.1	Introduction	510

12.2.2	GNSS as synchronization reference source	510

12.2.3	NR Cell as synchronization reference source	511

12.2.4	E-URTAN Cell as synchronization reference source	511

12.2.5	SyncRef UE as synchronization reference source	511

12.3	Initiation/Cease of SLSS Transmissions	512

12.3.1	Introduction	512

12.3.1.1	Initiation/Cease of SLSS transmissions with NR cell as synchronization reference source	512

12.3.1.2	Initiation/Cease of SLSS transmissions with EUTRAN cell as synchronization reference source	513

12.3.1.3	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	514

12.3.1.4	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	514

12.4	Selection / Reselection of V2X Synchronization Reference Source	514

12.5	L1 SL-RSRP measurements	515

12.5.1	Introduction	515

12.5.2	SL-RSRP measurements	515

12.6	Congestion Control measurements	516

12.7	Interruption	516

12.7.1	Interruptions to WAN due to V2X Sidelink Communication	516

12.7.2	V2X Sidelink Communication Dropping due to synchronization source change	516

12.7.3	Interruptions to WAN due to switching between E-UTRA V2X Sidelink and NR V2X Sidelink	518

12.8	Reliability of GNSS signal	518

12.9	Scheduling availability	519

12.9.1	Scheduling availability of UE switching between E-UTRA sidelink and NR sidelink	519

13	Measurement Performance Requirements for NR gNB	519

13.1	UL-RTOA	519

13.1.1	Report mapping	519

13.2	gNB Rx-Tx time difference	521

13.2.1	Report mapping	521

13.2.2	Measurement Accuracy Requirements	523

13.2.2.1	Introduction	523

13.2.2.2	Requirements	524

13.3	UL SRS RSRP measurement	525

13.3.1	Report mapping	525

13.3.2	Measurement accuracy requirements	525

13.3.2.1	Introduction	525

13.3.2.2	Requirements	526

13.4	AoA/ZoA	527

13.4.1	Report mapping	527

Annex A (normative): Test Cases	423

A.1	Purpose of annex	423

A.2	Requirement classification for statistical testing	423

A.2.1	Types of requirements in TS 38.133	423

A.2.1.1	Time and delay requirements on UE higher layer actions	423

A.2.1.2	Measurements of power levels, relative powers and time	423

A.2.1.3	Implementation requirements	424

A.2.1.4	Physical layer timing requirements	424

A.2.1.5	Requirements under CCA	424

A.3	RRM test configurations	425

A.3.1	Reference measurement channels	425

A.3.1.1	PDSCH	425

A.3.1.1.1	FDD	425

A.3.1.1.2	TDD	426

A.3.1A	Reference measurement channels under CCA	429

A.3.1A.1	PDSCH	429

A.3.1A.1.1	TDD	429

A.3.1A.2	CORESET for RMSI scheduling	430

A.3.1A.2.1	TDD	430

A.3.1A.3	CORESET for RMC scheduling	431

A.3.1A.3.1	TDD	431

A.3.1A.4	TDD UL/DL configuration	432

A.3.1A.5	RMC burst transmission model	432

A.3.1.2	CORESET for RMSI scheduling	433

A.3.1.2.1	FDD	433

A.3.1.2.2	TDD	434

A.3.1.3	CORESET for RMC scheduling	437

A.3.1.3.1	FDD	437

A.3.1.3.2	TDD	438

A.3.1.4	TDD UL/DL configuration	441

A.3.2	OFDMA channel noise generator (OCNG)	442

A.3.2.1	Generic OFDMA Channel Noise Generator (OCNG)	442

A.3.2.1.1	OCNG pattern 1: Generic OCNG pattern for all unused REs	442

A.3.2.1.2	OCNG pattern 2: Generic OCNG pattern for all unused REs for 2AoA setup	443

A.3.2.1.3	OCNG pattern 3: Generic OCNG pattern for unused REs in the same bandwidth as CORESET	443

A.3.2.1.4	OCNG pattern 4: Generic OCNG pattern for all unused REs outside SSB slot(s)	444

A.3.2.2	Void	445

A.3.3	Reference DRX configurations	445

A.3.3.1	DRX Configuration 1: DRX cycle = 40 ms and TAT = 500 ms	445

A.3.3.2	DRX Configuration 2: DRX cycle = 640 ms and TAT = 500 ms	445

A.3.3.3	DRX Configuration 3: DRX cycle = 40 ms and TAT = Infinity	445

A.3.3.4	DRX Configuration 4: DRX cycle = 160 ms and TAT = Infinity	446

A.3.3.5	DRX Configuration 5: DRX cycle = 320 ms and TAT = Infinity	446

A.3.3.6	DRX Configuration 6: DRX cycle = 320 ms and TAT = 500 ms	446

A.3.3.7	DRX Configuration 7: DRX cycle = 640 ms and TAT = Infinity	447

A.3.3.8	DRX Configuration 8: DRX cycle = 320 ms and TAT = Infinity	447

A.3.3.9	DRX Configuration 9: DRX cycle = 40 ms and TAT = 500 ms	447

A.3.3.10	DRX Configuration 10: DRX cycle = 640 ms and TAT = 500 ms	448

A.3.3.11	DRX Configuration 11: DRX cycle = 20 ms and TAT = Infinity	448

A.3.3.12	DRX Configuration 12: DRX cycle = 640 ms and TAT = Infinity	448

A.3.4	Test Cases with Different Channel Bandwidths	448

A.3.4.1	Test Cases with Different E-UTRA Channel Bandwidths	448

A.3.4.1.1	Introduction	448

A.3.4.1.2	Principle of testing	449

A.3.5	Test Cases for Synchronous and Asynchronous DC Operations	449

A.3.5.1	EN-DC Test Cases for Synchronous and Asynchronous EN-DC Operations	449

A.3.5.1.1	Introduction	449

A.3.5.1.2	Principle of Testing	449

A.3.6	Antenna configurations	449

A.3.6.1	Antenna configurations for FR1	449

A.3.6.1.1	Antenna connection for 4 Rx capable UEs	449

A.3.6.1.1.1	Introduction	449

A.3.6.1.1.2	Principle of testing	449

A.3.6.2	Antenna configurations for FR2	452

A.3.6A	Antenna configurations with unlicensed bands	452

A.3.6A.1	Antenna configurations for FR1	452

A.3.6A.1.1	Antenna connection for 4 Rx capable UEs	452

A.3.6A.1.1.1	Introduction	452

A.3.6A.1.1.2	Principle of testing	452

A.3.7	EN-DC test setup	454

A.3.7.1	Introduction	454

A.3.7.2	E-UTRAN Serving Cell Parameters	454

A.3.7.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	454

A.3.7.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	455

A.3.7A	NR FR1-FR2 test setup	456

A.3.7B	EN-DC test setup with unlicensed bands	457

A.3.7B.1	Introduction	457

A.3.7B.2	E-UTRAN Serving Cell Parameters	457

A.3.7B.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) under CCA in FR1	457

A.3.7C	LTE-FR1/FR2 test setup	458

A.3.7D	NE-DC test setup	458

A.3.7D.1	Introduction	458

A.3.7D.2	E-UTRAN Serving Cell Parameters	458

A.3.7D.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	458

A.3.7D.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	458

A.3.8	PRACH configurations	458

A.3.8.1	Introduction	458

A.3.8.2	PRACH configurations in FR1	459

A.3.8.2.1	FR1 PRACH configuration 1	459

A.3.8.2.2	FR1 PRACH configuration 2	459

A.3.8.2.3	FR1 PRACH configuration 3	460

A.3.8.2.4	FR1 PRACH configuration 4	461

A.3.8.3	PRACH configurations in FR2	462

A.3.8.3.1	FR2 PRACH configuration 1	462

A.3.8.3.2	FR2 PRACH configuration 2	463

A.3.8.3.3	FR2 PRACH configuration 3	464

A.3.8.3.4	FR2 PRACH configuration 4	465

A.3.8A	PRACH configurations under CCA	466

A.3.8A.1	Introduction	466

A.3.8A.2	PRACH configurations in FR1	466

A.3.8A.2.1	FR1 PRACH configuration 1 under CCA	466

A.3.8A.2.2	FR1 PRACH configuration 2 under CCA	467

A.3.9	BWP configurations	468

A.3.9.1	Introduction	468

A.3.9.2	Downlink BWP configurations	469

A.3.9.2.1	Initial BWP	469

A.3.9.2.2	Dedicated BWP	469

A.3.9.3	Uplink BWP configurations	469

A.3.9.3.1	Initial BWP	469

A.3.9.3.2	Dedicated BWP	470

A.3.10	SSB Configurations	470

A.3.10.1	SSB Configurations for FR1	470

A.3.10.1.1	SSB pattern 1 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	470

A.3.10.1.5	SSB pattern 5 in FR1: SSB allocation for SSB SCS=15 kHz starting from odd SFN in 10 MHz	472

A.3.10A	SSB Configurations under CCA	477

A.3.10A.1	SSB Configurations under CCA for FR1	477

A.3.10A.1.1	SSB pattern 1 under CCA for semi-static channel access: SSB allocation for SSB SCS=30kHz in 40MHz	477

A.3.10A.1.2	SSB pattern 2 under CCA for dynamic channel access: SSB allocation for SSB SCS=30kHz in 40MHz	478

A.3.10A.1.3	SSB pattern 3 under CCA for semi-static channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	478

A.3.10A.1.4	SSB pattern 4 under CCA for dynamic channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	479

A.3.11	SMTC Configurations	479

A.3.11.1	SMTC pattern 1: SMTC period = 20 ms with SMTC duration = 1 ms	479

A.3.11.2	SMTC pattern 2: SMTC period = 20 ms with SMTC duration = 5 ms	479

A.3.11.3	SMTC pattern 3: SMTC period = 160 ms with SMTC duration = 1 ms	480

A.3.11.4	SMTC pattern 4: SMTC period = 20 ms with SMTC duration = 1 ms	480

A.3.11.5	SMTC pattern 5: SMTC period = 20 ms with SMTC duration = 5 ms	480

A.3.11.6	SMTC pattern 6: SMTC period = 20 ms with SMTC duration = 5 ms	480

A.3.11.7	Void	481

A.3.11.8	Void	481

A.3.11.9	SMTC pattern 9: SMTC period = 20 ms with SMTC duration = 1 ms	481

A.3.12	Test Cases with Different CC Configurations	481

A.3.12.1 EN-DC Test Cases with Different EN-DC Configurations	481

A.3.12.1.1	Introduction	481

A.3.12.1.2	Principle of testing	481

A.3.12.2	Carrier Aggregation Test Cases with Different CA Configurations	481

A.3.12.2.1	Introduction	481

A.3.12.2.2	Principle of testing	481

A.3.13	Test Cases in SA and EN-DC Operations	482

A.3.13.1	Introduction	482

A.3.13.2	Principle of Testing	482

A.3.13A Test Cases involving E-UTRA/FR1 and FR2 carriers	482

A.3.13A.1	Introduction	482

A.3.13A.2	Principle of Testing in EN-DC	482

A.3.13A.3	Principle of Testing in SA	483

A.3.13B	Test Cases for EN-DC and NE-DC Operations	484

A.3.13B.1	Active BWP switch Test Cases for EN-DC and NE-DC Operations	484

A.3.13B.1.1	Introduction	484

A.3.13B.1.2	Principle of Testing	484

A.3.13B.2	SFTD accuracy Test Cases for EN-DC and NE-DC Operations	484

A.3.13B.2.1	Introduction	484

A.3.13B.2.2	Principle of Testing	484

A.3.14	CSI-RS configurations	485

A.3.14.1	FDD	485

A.3.14.2	TDD	487

A.3.15	Angle of Arrival (AoA) for FR2 RRM test cases	492

A.3.15.1	Setup 1: Single AoA in Rx beam peak direction	492

A.3.15.2	Setup 2: Single AoA in non Rx beam peak direction	492

A.3.15.2.1	Setup 2a: Single AoA in non Rx beam peak direction without change in direction	492

A.3.15.2.2	Setup 2b: Single AoA in non Rx beam peak direction with change in direction	493

A.3.15.3	Setup 3: 2 AoAs	493

A.3.15.4	Setup 4: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak	493

A.3.15.4.1	Setup 4a: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak without change in direction	493

A.3.15.4.2	Setup 4b: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak with change in direction	493

A.3.16	TCI State Configuration	494

A.3.16.1	Introduction	494

A.3.16.2	TCI states	494

A.3.17	Configurations of CSI-RS for tracking	494

A.3.17.1	Configuration of CSI-RS for tracking for FR1	494

A.3.17.1.1	FDD	494

A.3.17.1.2	TDD	495

A.3.17.2	Configuration of CSI-RS for tracking for FR2	496

A.3.17.2.1	TDD	496

A.3.18	Additional definitions related to OTA testing for FR2 RRM test cases	497

A.3.18.1	Introduction	497

A.3.18.2	PRACH Power Measurement	497

A.3.19	Test applicability for DAPS handover	497

A.3.19.1	Introduction	497

A.3.19.2	Principle of testing	497

A.3.20	MsgA configurations	498

A.3.20.1	Introduction	498

A.3.20.2	MsgA configurations in FR1	498

A.3.20.2.1	FR1 MsgA configuration 1	498

A.3.20.2.2	FR1 MsgA configuration 2	499

A.3.20.3	MsgA configurations in FR2	501

A.3.20.3.1	FR2 MsgA configuration 1	501

A.3.20.3.2	FR2 MsgA configuration 2	502

A.3.20A	MsgA configurations under CCA	504

A.3.20A.1	Introduction	504

A.3.20A.2	MsgA configurations in FR1	504

A.3.20A.2.1	FR1 MsgA configuration 1 under CCA	504

A.3.20A.2.2	FR1 MsgA configuration 2 under CCA	505

A.3.21	V2X sidelink communication	508

A.3.21.1	Introduction	508

A.3.21.2	Reference resource pool configurations for V2X Sidelink Communication	508

A.3.21.3	Reference measurement channels for V2X Sidelink Communication	512

A.3.22	CSI-IM configurations	512

A.3.22.1	FDD	512

A.3.22.2	TDD	513

A.3.23	Spatial Relation Configuration	515

A.3.23.1	Introduction	515

A.3.23.2	Spatial Relation	515

A.3.24	SRS configuration	516

A.3.25	Channel bandwidth (CBW) configurations	518

A.3.25.1	DL UE specific CBW	519

A.3.26	CCA model	519

A.3.26.1	Introduction	519

A.3.26.2	CCA model for operation on a carrier frequency with CCA in FR1	519

A.3.26.2.1	DL CCA model	519

A.3.26.2.2	UL CCA model	521

A.3.27	Test Cases with at Least One Cell on a Carrier Frequency with CCA	521

A.3.27.1	Introduction	522

A.3.27.2	 NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	522

A.3.27.3	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	522

A.3.27.4	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	522

A.3.27.5	E-UTRA Standalone Tests with at Least One NR Cell under CCA	522

A.3.28	Discovery Burst Transmission Window configuration under CCA	522

A.3.28.1	DBT Window pattern 1: DBT Window period = 20 ms with DBT Window duration = 1 ms	522

A.3.29	Testing principles for UE capable of only NR bands with shared spectrum access	522

A.3.29.1	Introduction	522

A.3.29.2	Principle of testing for UE capable of EN-DC with only NR bands with shared spectrum access	523

A.3.29.3	Principle of testing for UE capable of SA operation with only NR bands with shared spectrum access	523

A.3.30	CSI-RS configurations for RRM	524

A.3.30.1	FDD	524

A.3.30.2	TDD	525

A.3.31	PRS Configurations	527

A.3.31.1.	PRS Configurations for FR1	527

A.3.31.1.1.	PRS pattern 1 in FR1: SCS=15 KHz	527

A.3.31.1.2.	PRS pattern 2 in FR1: SCS=30 KHz	528

A.3.31.2.	PRS Configurations for FR2	528

A.3.31.2.1.	PRS pattern 1 in FR2: SCS=120 KHz	528

A.4	EN-DC tests with all NR cells in FR1	529

A.4.1	Void	529

A.4.2	Void	529

A.4.3	RRC_CONNECTED state mobility	529

A.4.3.1	Void	529

A.4.3.2	RRC Connection Mobility Control	529

A.4.3.2.1	Void	529

A.4.3.2.2	Random Access	529

A.4.3.2.2.1	4-step RA type contention based random access test in FR1 for PSCell in EN-DC	529

A.4.3.2.2.2	4-step RA type non-contention based random access test in FR1 for PSCell in EN-DC	533

A.4.3.2.2.3	2-step RA type contention based random access test in FR1 for PSCell in EN-DC	537

A.4.3.2.2.4	2-step RA type non-contention based random access test in FR1 for PSCell in EN-DC	541

A.4.3.2.3	Void	545

A.4.4	Timing	545

A.4.4.1	UE transmit timing	545

A.4.4.1.1	NR UE Transmit Timing Test for FR1	545

A.4.4.1.1.1	Test Purpose and environment	545

A.4.4.1.1.2	Test requirements	549

A.4.4.2	UE timer accuracy	549

A.4.4.3	Timing advance	549

A.4.4.3.1	EN-DC FR1 timing advance adjustment accuracy	549

A.4.4.3.1.1	Test Purpose and Environment	549

A.4.4.3.1.2	Test Parameters	549

A.4.4.3.1.3	Test Requirements	553

A.4.5	Signaling characteristics	553

A.4.5.1	Radio link Monitoring	553

A.4.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	554

A.4.5.1.1.1	Test Purpose and Environment	554

A.4.5.1.1.2	Test Requirements	558

A.4.5.1.2	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	558

A.4.5.1.2.1	Test Purpose and Environment	558

A.4.5.1.2.2	Test Requirements	564

A.4.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	564

A.4.5.1.3.1	Test Purpose and Environment	564

A.4.5.1.3.2	Test Requirements	570

A.4.5.1.4	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	570

A.4.5.1.4.1	Test Purpose and Environment	570

A.4.5.1.4.2	Test Requirements	575

A.4.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	575

A.4.5.1.5.1	Test Purpose and Environment	575

A.4.5.1.5.2	Test Requirements	581

A.4.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	581

A.4.5.1.6.1	Test Purpose and Environment	581

A.4.5.1.6.2	Test Requirements	587

A.4.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	587

A.4.5.1.7.1	Test Purpose and Environment	587

A.4.5.1.7.2	Test Requirements	592

A.4.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	593

A.4.5.1.8.1	Test Purpose and Environment	593

A.4.5.1.8.2	Test Requirements	598

A.4.5.2	Interruption	598

A.4.5.2.1	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	598

A.4.5.2.1.1	Test Purpose and Environment	598

A.4.5.2.1.2	Test Requirements	602

A.4.5.2.2	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	602

A.4.5.2.2.1	Test Purpose and Environment	602

A.4.5.2.2.2	Test Requirements	606

A.4.5.2.3	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	606

A.4.5.2.3.1	Test Purpose and Environment	606

A.4.5.2.3.2	Test Requirements	611

A.4.5.2.4	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	612

A.4.5.2.4.1	Test Purpose and Environment	612

A.4.5.2.4.2	Test Requirements	616

A.4.5.2.5	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	617

A.4.5.2.5.1	Test Purpose and Environment	617

A.4.5.2.5.2	Test Requirements	621

A.4.5.2.6	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	621

A.4.5.2.6.1	Test Purpose and Environment	621

A.4.5.2.6.2	Test Requirements	625

A.4.5.2.7	Void	625

A.4.5.2.8	E-UTRAN - NR FR1 interruptions at NR SRS carrier based switching in asynchronous EN-DC	625

A.4.5.2.8.1	Test Purpose and Environment	625

A.4.5.2.8.2	Test Requirements	628

A.4.5.2.9	E-UTRAN – NR interruptions at E-UTRA SRS carrier based switching	629

A.4.5.2.9.1	Test Purpose and Environment	629

A.4.5.2.9.2	Test Requirements	633

A.4.5.3	SCell Activation and Deactivation Delay	634

A.4.5.3.1	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	634

A.4.5.3.1.1	Test Purpose and Environment	634

A.4.5.3.1.2	Test Requirements	640

A.4.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 640ms SCell measurement cycle	641

A.4.5.3.2.1	Test Purpose and Environment	641

A.4.5.3.2.2	Test Requirements	641

A.4.5.3.3	SCell Activation and deactivation of unknown SCell in FR1	641

A.4.5.3.3.1	Test Purpose and Environment	641

A.4.5.3.3.2	Test Requirements	642

A.4.5.3.4	SCell Activation and deactivation of multiple unknown SCells in FR1 with single activation/deactivation command	643

A.4.5.3.4.1	Test Purpose and Environment	643

A.4.5.3.4.2	Test Requirements	647

A.4.5.3.5	Direct SCell activation at SCell addition of known SCell in FR1	647

A.4.5.3.5.1	Test Purpose and Environment	647

A.4.5.3.5.2	Test Requirements	655

A.4.5.4	UE UL carrier RRC reconfiguration Delay	656

A.4.5.4.1	UE UL carrier RRC reconfiguration Delay	656

A.4.5.4.1.1 Test Purpose and Environment	656

A.4.5.4.1.2	Test Requirements	664

A.4.5.5	Beam Failure Detection and Link recovery procedures	665

A.4.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	665

A.4.5.5.1.1	Test Purpose and Environment	665

A.4.5.5.1.2	Test Requirements	670

A.4.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	671

A.4.5.5.2.1	Test Purpose and Environment	671

A.4.5.5.2.2	Test Requirements	676

A.4.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	677

A.4.5.5.3.1	Test Purpose and Environment	677

A.4.5.5.3.2	Test Requirements	682

A.4.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in DRX mode	683

A.4.5.5.4.1	Test Purpose and Environment	683

A.4.5.5.4.2	Test Requirements	688

A.4.5.5.5	EN-DC Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	689

A.4.5.5.5.1	Test Purpose and Environment	689

A.4.5.5.5.2	Test Requirements	694

A.4.5.5.6	EN-DC Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode	695

A.4.5.5.6.1	Test Purpose and Environment	695

A.4.5.5.6.2	Test Requirements	701

A.4.5.6	Active BWP switch	702

A.4.5.6.1	DCI-based and Timer-based Active BWP Switch	702

A.4.5.6.1.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	702

A.4.5.6.1.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	707

A.4.5.6.2	RRC-based Active BWP Switch	713

A.4.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	717

A.4.5.6.3.1	Simultaneous E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in EN-DC on multiple CCs	717

A.4.5.6.3.1.2	Test Requirements	723

A.4.5.6.4	SCell dormancy switch	723

A.4.5.6.4.1	E-UTRAN – NR FR1 PSCell SCell dormancy switch of single FR1 SCell outside active time	723

A.4.5.6.4.1.1	Test Purpose and Environment	723

A.4.5.6.4.1.2	Test Requirements	731

A.4.5.6.4.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR1 SCells inside active time	731

A.4.5.6.4.2.1	Test Purpose and Environment	731

A.4.5.6.4.2.2	Test Requirements	743

A.4.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	744

A.4.5.6.5.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC on multiple CCs	744

A.4.5.7	PSCell addition and release delay	748

A.4.5.7.1	Addition and Release Delay of known NR PSCell	748

A.4.5.7.1.1	Test purpose and environment	748

A.4.5.7.1.2	Test Requirements	752

A.4.5.8	DL Interruptions at switching between two uplink carriers	753

A.4.5.8.1	Test Purpose and Environment	753

A.4.5.8.2	Test Requirements	756

A.4.5.9	UE specific CBW change	757

A.4.5.9.1	UE specific CBW change on FR1 NR PSCell with non-DRX in synchronous EN- DC	757

A.4.5.9.1.1	Test Purpose and Environment	757

A.4.5.9.1.2	Test Requirements	761

A.4.6	Measurement procedure	761

A.4.6.1	Intra-frequency Measurements	761

A.4.6.1.1	EN-DC event triggered reporting tests without gap under non-DRX	761

A.4.6.1.1.1	Test purpose and Environment	761

A.4.6.1.1.2	Test parameters	761

A.4.6.1.1.3	Test Requirements	765

A.4.6.1.2	EN-DC event triggered reporting tests without gap under DRX	765

A.4.6.1.2.1	Test purpose and Environment	765

A.4.6.1.2.2	Test parameters	765

A.4.6.1.2.2	Test Requirements	769

A.4.6.1.3	EN-DC event triggered reporting tests with per-UE gaps under non-DRX	769

A.4.6.1.3.1	Test purpose and Environment	769

A.4.6.1.3.2	Test parameters	769

A.4.6.1.3.3	Test Requirements	773

A.4.6.1.4	EN-DC event triggered reporting tests with per-UE gaps under DRX	773

A.4.6.1.4.1	Test purpose and Environment	773

A.4.6.1.4.2	Test parameters	773

A.4.6.1.4.3	Test Requirements	777

A.4.6.1.5	EN-DC event triggered reporting tests without gap under non-DRX with SSB index reading	777

A.4.6.1.5.1	Test purpose and Environment	777

A.4.6.1.5.2	Test parameters	777

A.4.6.1.5.3	Test Requirements	779

A.4.6.1.6	EN-DC event triggered reporting tests with SSB index reading with per-UE gaps	780

A.4.6.1.6.1	Test purpose and Environment	780

A.4.6.1.6.2	Test parameters	780

A.4.6.1.6.3	Test Requirements	782

A.4.6.1.7	EN-DC event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	783

A.4.6.1.7.1	Test purpose and Environment	783

A.4.6.1.7.2	Test parameters	783

A.4.6.1.7.3	Test Requirements	787

A.4.6.2	Inter-frequency Measurements	787

A.4.6.2.1	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	787

A.4.6.2.1.1	Test Purpose and Environment	787

A.4.6.2.1.2	Test Requirements	791

A.4.6.2.2	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	791

A.4.6.2.2.1	Test Purpose and Environment	791

A.4.6.2.2.2	Test Requirements	795

A.4.6.2.3	Void	795

A.4.6.2.4	Void	795

A.4.6.2.5	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	795

A.4.6.2.5.1	Test Purpose and Environment	795

A.4.6.2.5.2	Test Requirements	800

A.4.6.2.6	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	800

A.4.6.2.6.1	Test Purpose and Environment	800

A.4.6.2.6.2	Test Requirements	805

A.4.6.2.7	Void	806

A.4.6.2.8	Void	806

A.4.6.3	Void	806

A.4.6.4	L1-RSRP measurement for beam reporting	806

A.4.6.4.1	SSB based L1-RSRP measurement when DRX is not used	806

A.4.6.4.1.1	Test Purpose and Environment	806

A.4.6.4.1.2	Test parameters	806

A.4.6.4.1.3	Test Requirements	809

A.4.6.4.2	SSB based L1-RSRP measurement when DRX is used	809

A.4.6.4.2.1	Test Purpose and Environment	809

A.4.6.4.2.2	Test parameters	810

A.4.6.4.2.3	Test Requirements	813

A.4.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	813

A.4.6.4.3.1	Test Purpose and Environment	813

A.4.6.4.3.2	Test parameters	814

A.4.6.4.3.3	Test Requirements	817

A.4.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	817

A.4.6.4.4.1	Test Purpose and Environment	817

A.4.6.4.4.2	Test parameters	818

A.4.6.4.4.3	Test Requirements	820

A.4.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	820

A.4.6.4.5.1	Test Purpose and Environment	820

A.4.6.4.5.2	Test parameters	821

A.4.6.4.5.3	Test Requirements	824

A.4.6.5	CLI measurements	824

A.4.6.5.1	SRS-RSRP measurement with non-DRX	824

A.4.6.5.1.1	Test Purpose and Environment	824

A.4.6.5.1.2	Test Parameters	825

A.4.6.5.1.3	Test Requirements	828

A.4.6.5.2	CLI-RSSI measurement with non-DRX	828

A.4.6.5.2.1	Test Purpose and Environment	828

A.4.6.5.2.2	Test Parameters	829

A.4.6.5.2.3	Test Requirements	830

A.4.6.6	Measurements with autonomous gaps	831

A.4.6.6.1	EN-DC intra-frequency CGI identification of NR FR1 cell with autonomous gaps in synchronous EN-DC	831

A.4.6.6.1.1	Test Purpose and Environment	831

A.4.6.6.1.2	Test Requirements	835

A.4.6.7	L1-SINR measurement for beam reporting	835

A.4.6.7.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR when DRX is not used	835

A.4.6.7.1.1	Test Purpose and Environment	835

A.4.6.7.1.2	Test parameters	836

A.4.6.7.1.3	Test Requirements	839

A.4.6.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	839

A.4.6.7.2.1	Test Purpose and Environment	839

A.4.6.7.2.2	Test parameters	840

A.4.6.7.2.3	Test Requirements	843

A.4.6.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	843

A.4.6.7.3.1	Test Purpose and Environment	843

A.4.6.7.3.2	Test parameters	844

A.4.6.7.3.3	Test Requirements	847

A.4.6.8	CSI-RS based intra-frequency Measurement	847

A.4.6.8.1	EN-DC event triggered reporting tests without gap under DRX	847

A.4.6.8.1.1	Test purpose and Environment	847

A.4.6.8.1.2	Test Requirements	850

A.4.6.9	CSI-RS based inter-frequency Measurement	851

A.4.6.9.1	EN-DC event triggered reporting tests for FR1 cell when non-DRX is used	851

A.4.6.9.1.1	Test Purpose and Environment	851

A.4.6.9.1.2	Test Requirements	854

A.4.7	Measurement Performance requirements	855

A.4.7.1	SS-RSRP	855

A.4.7.1.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	855

A.4.7.1.1.1	Test Purpose and Environment	855

A.4.7.1.1.2	Test parameters	855

A.4.7.1.1.3	Test Requirements	859

A.4.7.1.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	859

A.4.7.1.2.1	Test Purpose and Environment	859

A.4.7.1.2.2	Test parameters	860

A.4.7.1.2.3	Test Requirements	864

A.4.7.1.3	Void	864

A.4.7.2	SS-RSRQ	864

A.4.7.2.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	864

A.4.7.2.1.1	Test Purpose and Environment	864

A.4.7.2.1.2	Test Parameters	865

A.4.7.2.1.3	Test Requirements	870

A.4.7.2.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	870

A.4.7.2.2.1	Test Purpose and Environment	870

A.4.7.2.2.2	Test Parameters	870

A.4.7.2.2.3	Test Requirements	875

A.4.7.3	SS-SINR	875

A.4.7.3.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	875

A.4.7.3.1.1	Test Purpose and Environment	875

A.4.7.3.1.2	Test Parameters	875

A.4.7.3.1.3	Test Requirements	879

A.4.7.3.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	879

A.4.7.3.2.1	Test Purpose and Environment	879

A.4.7.3.2.2	Test Parameters	879

A.4.7.3.2.3	Test Requirements	884

A.4.7.4	L1-RSRP measurement for beam reporting	884

A.4.7.4.1	SSB based L1-RSRP measurement	884

A.4.7.4.1.1	Test Purpose and Environment	884

A.4.7.4.1.2	Test parameters	885

A.4.7.4.1.3	Test Requirements	888

A.4.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	888

A.4.7.4.2.1	Test Purpose and Environment	888

A.4.7.4.2.2	Test parameters	889

A.4.7.4.2.3	Test Requirements	892

A.4.7.5	SFTD accuracy	892

A.4.7.5.1	SFTD accuracy	892

A.4.7.5.1.1	Test Purpose and Environment	892

A.4.7.5.1.2	Test Parameters	892

A.4.7.5.1.3	Test Requirements	897

A.4.7.5.2	Void	897

A.4.7.5.3	Void	897

A.4.7.6	CLI measurements	897

A.4.7.6.1	EN-DC SRS-RSRP measurement accuracy with FR1 serving cell	897

A.4.7.6.1.1	Test Purpose and Environment	897

A.4.7.6.1.2	Test parameters	898

A.4.7.6.1.3	Test Requirements	903

A.4.7.6.2	EN-DC CLI-RSSI measurement accuracy with FR1 serving cell	903

A.4.7.6.2.1	Test Purpose and Environment	903

A.4.7.6.2.2	Test parameters	904

A.4.7.6.2.3	Test Requirements	907

A.4.7.7	L1-SINR measurement for beam reporting	907

A.4.7.7.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	907

A.4.7.7.1.1	Test Purpose and Environment	907

A.4.7.7.1.2	Test parameters	908

A.4.7.7.1.3	Test Requirements	912

A.4.7.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR	912

A.4.7.7.2.1	Test Purpose and Environment	912

A.4.7.7.2.2	Test parameters	913

A.4.7.7.2.3	Test Requirements	917

A.4.7.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	917

A.4.7.7.3.1	Test Purpose and Environment	917

A.4.7.7.3.2	Test parameters	918

A.4.7.7.3.3	Test Requirements	921

A.4.7.8	CSI-RSRP	921

A.4.7.8.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	921

A.4.7.8.1.1	Test Purpose and Environment	921

A.4.7.8.1.2	Test parameters	922

A.4.7.8.1.3	Test Requirements	925

A.4.7.8.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	925

A.4.7.8.2.1	Test Purpose and Environment	925

A.4.7.8.2.2	Test parameters	926

A.4.7.8.2.3	Test Requirements	929

A.4.7.9	CSI-RSRQ	930

A.4.7.9.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	930

A.4.7.9.1.1	Test Purpose and Environment	930

A.4.7.9.1.2	Test Parameters	930

A.4.7.9.1.3	Test Requirements	935

A.4.7.9.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	935

A.4.7.9.2.1	Test Purpose and Environment	935

A.4.7.9.2.2	Test Parameters	935

A.4.7.2.2.3	Test Requirements	940

A.4.7.10	CSI-SINR	940

A.4.7.10.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	940

A.4.7.10.1.1	Test Purpose and Environment	940

A.4.7.10.1.2	Test Parameters	941

A.4.7.10.1.3	Test Requirements	944

A.4.7.10.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	944

A.4.7.10.2.1	Test Purpose and Environment	944

A.4.7.10.2.2	Test Parameters	944

A.4.7.10.2.3	Test Requirements	947

A.4.8	Void	948

A.4A	NE-DC test with all NR cells in FR1	948

A.4A.1	Signaling characteristics	948

A.4A.1.1	E-UTRAN PSCell addition	948

A.4A.1.1.1	Test purpose and environment	948

A.4A.1.1.2	Test Requirements	952

A.4A.1.2	Active BWP switch	953

A.4A.1.2.1	E-UTRAN PSCell – NR PCell FR1 DCI-based and Timer-based DL active BWP switch in non-DRX in synchronous NE-DC	953

A.4A.1.2.1.1	Test Purpose and Environment	953

A.4A.1.2.1.2	Test Requirements	957

A.4A.2	Measurement performance	958

A.4A.2.1	SFTD accuracy	958

A.4A.2.1.1	SFTD accuracy	958

A.4A.2.1.1.1	Test Purpose	958

A.4A.2.1.1.2	Test Environment	958

A.4A.2.1.1.3	Test Requirements	961

A.5	EN-DC tests with one or more NR cells in FR2	962

A.5.1	Void	962

A.5.2	Void	962

A.5.3	RRC_CONNECTED state mobility	962

A.5.3.1	Void	962

A.5.3.2	RRC Connection Mobility Control	962

A.5.3.2.1	Void	962

A.5.3.2.2	Random Access	962

A.5.3.2.2.1	4-step RA type contention based random access test in FR2 for PSCell/SCell in EN-DC	962

A.5.3.2.2.2	4-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	965

A.5.3.2.2.3	2-step RA type contention based random access test in FR2 for PSCell/SCell in EN-DC	971

A.5.3.2.2.4	2-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	974

A.5.3.2.3	Void	978

A.5.4	Timing	978

A.5.4.1	UE transmit timing	978

A.5.4.1.1	NR UE Transmit Timing Test for FR2	978

A.5.4.1.1.1	Test Purpose and environment	978

A.5.4.1.1.2	Test requirements	981

A.5.4.2	UE timer accuracy	982

A.5.4.3	Timing advance	982

A.5.4.3.1 EN-DC FR2 timing advance adjustment accuracy	982

A.5.4.3.1.1 Test Purpose and Environment	982

A.5.4.3.1.2 Test Parameters	982

A.5.4.3.1.3	Test Requirements	986

A.5.5	Signaling characteristics	986

A.5.5.1	Radio link Monitoring	986

A.5.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	986

A.5.5.1.1.1	Test Purpose and Environment	986

A.5.5.1.1.2	Test Requirements	990

A.5.5.1.2	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	991

A.5.5.1.2.1	Test Purpose and Environment	991

A.5.5.1.2.2	Test Requirements	994

A.5.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	995

A.5.5.1.3.1	Test Purpose and Environment	995

A.5.5.1.3.2	Test Requirements	999

A.5.5.1.4	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	999

A.5.5.1.4.1	Test Purpose and Environment	999

A.5.5.1.4.2	Test Requirements	1003

A.5.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	1003

A.5.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	1007

A.5.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	1011

A.5.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	1016

A.5.5.1.8.2	Test Requirements	1020

A.5.5.1.9	EN-DC Radio Link Monitoring UE Scheduling Restrictions on FR2	1021

A.5.5.1.9.1	Test Purpose and Environment	1021

A.5.5.1.9.2	Test Requirements	1023

A.5.5.2	Interruption	1023

A.5.5.2.1	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	1023

A.5.5.2.1.1	Test Purpose and Environment	1023

A.5.5.2.1.2	Test Requirements	1026

A.5.5.2.2	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	1026

A.5.5.2.2.1	Test Purpose and Environment	1026

A.5.5.2.2.2	Test Requirements	1029

A.5.5.2.3	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	1029

A.5.5.2.3.1	Test Purpose and Environment	1029

A.5.5.2.3.2	Test Requirements	1033

A.5.5.2.4	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	1033

A.5.5.2.4.1	Test Purpose and Environment	1033

A.5.5.2.4.2	Test Requirements	1036

A.5.5.2.5	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	1037

A.5.5.2.5.1	Test Purpose and Environment	1037

A.5.5.2.5.2	Test Requirements	1040

A.5.5.2.6	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	1041

A.5.5.2.6.1	Test Purpose and Environment	1041

A.5.5.2.6.2	Test Requirements	1043

A.5.5.2.7	E-UTRAN – NR FR2 interruptions at E-UTRA SRS carrier based switching	1044

A.5.5.2.7.1	Test Purpose and Environment	1044

A.5.5.2.7.2	Test Requirements	1047

A.5.5.2.8 E-UTRAN – NR FR2 interruptions at NR SRS carrier based switching	1047

A.5.5.2.8.1 Test Purpose and Environment	1047

A.5.5.2.8.3	Test Requirements	1050

A.5.5.3	SCell Activation and Deactivation Delay	1050

A.5.5.3.1	SCell Activation and deactivation of SCell in FR2 intra-band	1050

A.5.5.3.1.1	Test Purpose and Environment	1050

A.5.5.3.1.2	Test Requirements	1053

A.5.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	1053

A.5.5.3.2.1	Test Purpose and Environment	1053

A.5.5.3.2.2	Test Requirements	1057

A.5.5.3.3	Void	1057

A.5.5.3.4	Void	1057

A.5.5.3.5	SCell Activation and deactivation of SCell in FR2	1057

A.5.5.3.5.1	Test Purpose and Environment	1057

A.5.5.3.5.2	Test Requirements	1061

A.5.5.3.6	Multiple SCell Activation and deactivation of one unknown SCell and one known SCell in FR2	1062

A.5.5.3.6.1	Test Purpose and Environment	1062

A.5.5.3.6.2	Test Requirements	1065

A.5.5.3.7	Direct SCell activation at SCell addition of known SCell in FR2	1066

A.5.5.3.7.1	Test Purpose and Environment	1066

A.5.5.3.7.2	Test Requirements	1069

A.5.5.4	Void	1069

A.5.5.5	Beam Failure Detection and Link recovery procedures	1069

A.5.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	1069

A.5.5.5.1.1	Test Purpose and Environment	1069

A.5.5.5.1.2	Test Requirements	1073

A.5.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in DRX mode	1074

A.5.5.5.2.1	Test Purpose and Environment	1074

A.5.5.5.2.2	Test Requirements	1078

A.5.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	1079

A.5.5.5.3.1	Test Purpose and Environment	1079

A.5.5.5.3.2	Test Requirements	1083

A.5.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX mode	1084

A.5.5.5.4.1	Test Purpose and Environment	1084

A.5.5.5.4.2	Test Requirements	1088

A.5.5.5.5	EN-DC scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	1089

A.5.5.5.5.1	Test Purpose and Environment	1089

A.5.5.5.5.2	Test Requirements	1093

A.5.5.5.6	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	1094

A.5.5.5.6.1	Test Purpose and Environment	1094

A.5.5.5.6.2	Test Requirements	1098

A.5.5.5.7	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	1099

A.5.5.5.7.1	Test Purpose and Environment	1099

A.5.5.5.7.2	Test Requirements	1103

A.5.5.6	Active BWP switch	1104

A.5.5.6.1	DCI-based and Timer-based Active BWP Switch	1104

A.5.5.6.1.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	1104

A.5.5.6.1.1.1	Test Purpose and Environment	1104

A.5.5.6.1.1.2	Test Requirements	1107

A.5.5.6.1.2	E-UTRAN – NR PSCell FR2 with FR2 SCell DL active BWP switch in non-DRX in synchronous EN-DC	1107

A.5.5.6.2	RRC-based Active BWP Switch	1112

A.5.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1116

A.5.5.6.3.1	E-UTRAN – NR PSCell FR2 and NR SCell FR2 DL active BWP switch on multiple CCs in synchronous EN-DC	1116

A.5.5.6.3.1.1	Test Purpose and Environment	1116

A.5.5.6.4	SCell dormancy switch	1120

A.5.5.6.4.1	E-UTRAN – NR FR2 PSCell SCell dormancy switch of single FR2 SCell inside active time	1120

A.5.5.6.4.1.1	Test Purpose and Environment	1120

A.5.5.6.4.1.2	Test Requirements	1124

A.5.5.6.4.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR2 SCells outside active time	1124

A.5.5.6.4.2.1	Test Purpose and Environment	1124

A.5.5.6.4.2.2	Test Requirements	1131

A.5.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	1131

A.5.5.7	PSCell addition and release delay	1134

A.5.5.7.1	Addition and Release Delay of NR PSCell	1134

A.5.5.7.1.1	Test purpose and environment	1134

A.5.5.7.1.2	Test Requirements	1138

A.5.5.8	Active TCI state switch delay	1139

A.5.5.8.1	MAC-CE based active TCI state switch	1139

A.5.5.8.1.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	1139

A.5.5.8.1.1.1	Test Purpose and Environment	1139

A.5.5.8.1.1.2	Test Requirements	1142

A.5.5.8.2	RRC based active TCI state switch	1142

A.5.5.8.2.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	1142

A.5.5.8.2.1.1	Test Purpose and Environment	1142

A.5.5.8.2.1.2	Test Requirements	1146

A.5.5.9	Uplink spatial relation switch delay	1146

A.5.5.9.1	MAC-CE based uplink spatial relation switch	1146

A.5.5.9.1.1	E-UTRAN – NR PSCell FR2 uplink spatial relation switch for a known spatial relation	1146

A.5.5.9.1.1.1	Test Purpose and Environment	1146

A.5.5.9.1.1.2	Test Requirements	1149

A.5.5.9.2	RRC based spatial relation switch	1149

A.5.5.9.2.1	E-UTRAN – NR PSCell FR2 spatial relation switch associated with a known DL-RS	1149

A.5.5.9.2.1.1	Test Purpose and Environment	1149

A.5.5.9.2.1.2	Test Requirements	1152

A.5.5.10	UE specific CBW change	1152

A.5.5.10.1	UE specific CBW change on FR2 NR PSCell	1152

A.5.5.10.1.1	Test Purpose and Environment	1152

A.5.5.10.1.2	Test Requirements	1155

A.5.6	Measurement procedure	1156

A.5.6.1	Intra-frequency Measurements	1156

A.5.6.1.1	EN-DC event triggered reporting test without gap under non-DRX	1156

A.5.6.1.1.1	Test purpose and Environment	1156

A.5.6.1.1.2	Test Requirements	1159

A.5.6.1.2	EN-DC event triggered reporting test without gap under DRX	1159

A.5.6.1.2.1	Test purpose and Environment	1159

A.5.6.1.2.2	Test Requirements	1161

A.5.6.1.3	EN-DC event triggered reporting test with per-UE gaps under non-DRX	1162

A.5.6.1.3.1	Test purpose and Environment	1162

A.5.6.1.3.2	Test Requirements	1166

A.5.6.1.4	EN-DC event triggered reporting test with per-UE gaps under DRX	1166

A.5.6.1.4.1	Test purpose and Environment	1166

A.5.6.1.4.2	Test Requirements	1169

A.5.6.2	Inter-frequency Measurements	1170

A.5.6.2.1 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	1170

A.5.6.2.1.1	Test Purpose and Environment	1170

A.5.6.2.1.2	Test Requirements	1173

A.5.6.2.2 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	1173

A.5.6.2.2.1	Test Purpose and Environment	1173

A.5.6.2.2.2	Test Requirements	1177

A.5.6.2.3 	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	1177

A.5.6.2.3.1	Test Purpose and Environment	1177

A.5.6.2.3.2	Test Requirements	1181

A.5.6.2.4	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	1181

A.5.6.2.4.1	Test Purpose and Environment	1181

A.5.6.2.4.2	Test Requirements	1185

A.5.6.2.5	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	1185

A.5.6.2.5.1	Test Purpose and Environment	1185

A.5.6.2.5.2	Test Requirements	1190

A.5.6.2.6	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	1190

A.5.6.2.6.1	Test Purpose and Environment	1190

A.5.6.2.6.2	Test Requirements	1194

A.5.6.2.7	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	1194

A.5.6.2.7.1	Test Purpose and Environment	1194

A.5.6.2.7.2	Test Requirements	1199

A.5.6.2.8	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	1199

A.5.6.2.8.1	Test Purpose and Environment	1199

A.5.6.2.8.2	Test Requirements	1204

A.5.6.3	L1-RSRP measurement for beam reporting	1204

A.5.6.3.1	SSB based L1-RSRP measurement when DRX is not used	1204

A.5.6.3.1.1	Test Purpose and Environment	1204

A.5.6.3.1.2	Test parameters	1204

A.5.6.3.1.3	Test Requirements	1206

A.5.6.3.2	SSB based L1-RSRP measurement when DRX is used	1206

A.5.6.3.2.1	Test Purpose and Environment	1206

A.5.6.3.2.2	Test parameters	1207

A.5.6.3.2.3	Test Requirements	1209

A.5.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	1209

A.5.6.3.3.1	Test Purpose and Environment	1209

A.5.6.3.3.2	Test parameters	1210

A.5.6.3.3.3	Test Requirements	1212

A.5.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	1213

A.5.6.3.4.1	Test Purpose and Environment	1213

A.5.6.3.4.2	Test parameters	1213

A.5.6.3.4.3	Test Requirements	1215

A.5.6.4	CLI measurements	1216

A.5.6.4.1	SRS-RSRP measurement with DRX	1216

A.5.6.4.1.1	Test Purpose and Environment	1216

A.5.6.4.1.2	Test Parameters	1216

A.5.6.4.1.3	Test Requirements	1218

A.5.6.4.2	CLI-RSSI measurement with DRX	1218

A.5.6.4.2.1	Test Purpose and Environment	1218

A.5.6.4.2.2	Test Parameters	1219

A.5.6.4.2.3	Test Requirements	1221

A.5.6.5	Measurements with autonomous gaps	1221

A.5.6.5.1 	EN-DC inter-frequency CGI identification of NR neighbor cell in FR2	1221

A.5.6.5.1.1	Test Purpose and Environment	1221

A.5.6.5.1.2	Test Requirements	1224

A.5.6.6	L1-SINR measurement for beam reporting	1225

A.5.6.6.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured when DRX is used	1225

A.5.6.6.1.1	Test Purpose and Environment	1225

A.5.6.6.1.2	Test parameters	1225

A.5.6.6.1.3	Test Requirements	1227

A.5.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	1227

A.5.6.6.2.1	Test Purpose and Environment	1227

A.5.6.6.2.2	Test parameters	1228

A.5.6.6.2.3	Test Requirements	1231

A.5.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	1231

A.5.6.6.3.1	Test Purpose and Environment	1231

A.5.6.6.3.2	Test parameters	1232

A.5.6.6.3.3	Test Requirements	1234

A.5.6.7	CSI-RS based Intra-frequency Measurements	1234

A.5.6.7.1	EN-DC event triggered reporting test without gap under non-DRX	1234

A.5.6.7.1.1	Test purpose and Environment	1234

A.5.6.7.1.2	Test Requirements	1237

A.5.6.8	CSI-RS based Inter-frequency Measurements	1237

A.5.6.8.1 	EN-DC event triggered reporting tests for NR FR2 cell when DRX is used	1237

A.5.6.8.1.1	Test Purpose and Environment	1237

A.5.6.8.1.2	Test Requirements	1241

A.5.7	Measurement Performance requirements	1241

A.5.7.1	SS-RSRP	1242

A.5.7.1.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1242

A.5.7.1.1.1	Test Purpose and Environment	1242

A.5.7.1.1.2	Test parameters	1242

A.5.7.1.1.3	Test Requirements	1244

A.5.7.1.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1245

A.5.7.1.2.1	Test Purpose and Environment	1245

A.5.7.1.2.2	Test parameters	1245

A.5.7.1.2.3	Test Requirements	1250

A.5.7.1.3	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	1251

A.5.7.1.3.1	Test Purpose and Environment	1251

A.5.7.1.3.2	Test parameters	1251

A.5.7.1.3.3	Test Requirements	1254

A.5.7.2	SS-RSRQ	1255

A.5.7.2.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1255

A.5.7.2.1.1	Test Purpose and Environment	1255

A.5.7.2.1.2	Test Parameters	1255

A.5.7.2.1.3	Test Requirements	1257

A.5.7.2.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1258

A.5.7.2.2.1	Test Purpose and Environment	1258

A.5.7.2.2.2	Test Parameters	1258

A.5.7.2.2.3	Test Requirements	1260

A.5.7.3	SS-SINR	1260

A.5.7.3.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1260

A.5.7.3.1.1	Test Purpose and Environment	1260

A.5.7.3.1.2	Test Parameters	1260

A.5.7.3.1.3	Test Requirements	1262

A.5.7.3.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1262

A.5.7.3.2.1	Test Purpose and Environment	1262

A.5.7.3.2.2	Test Parameters	1262

A.5.7.3.2.3	Test Requirements	1264

A.5.7.4	L1-RSRP measurement for beam reporting	1264

A.5.7.4.1	SSB based L1-RSRP measurement	1264

A.5.7.4.1.1	Test Purpose and Environment	1264

A.5.7.4.1.2	Test parameters	1265

A.5.7.4.1.3	Test Requirements	1267

A.5.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	1268

A.5.7.4.2.1	Test Purpose and Environment	1268

A.5.7.4.2.2	Test parameters	1268

A.5.7.4.2.3	Test Requirements	1270

A.5.7.5	CLI measurements	1271

A.5.7.5.1	EN-DC SRS-RSRP measurement accuracy with FR2 serving cell	1271

A.5.7.5.1.1	Test Purpose and Environment	1271

A.5.7.5.1.2	Test parameters	1271

A.5.7.5.1.3	Test Requirements	1274

A.5.7.5.2	EN-DC CLI-RSSI measurement accuracy with FR2 serving cell	1275

A.5.7.5.2.1	Test Purpose and Environment	1275

A.5.7.5.2.2	Test parameters	1275

A.5.7.5.2.3	Test Requirements	1277

A.5.7.6	L1-SINR measurement for beam reporting	1278

A.5.7.6.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	1278

A.5.7.6.1.1	Test Purpose and Environment	1278

A.5.7.6.1.2	Test parameters	1278

A.5.7.6.1.3	Test Requirements	1280

A.5.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	1281

A.5.7.6.2.1	Test Purpose and Environment	1281

A.5.7.6.2.2	Test parameters	1281

A.5.7.6.2.3	Test Requirements	1283

A.5.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	1284

A.5.7.6.3.1	Test Purpose and Environment	1284

A.5.7.6.3.2	Test parameters	1284

A.5.7.6.3.3	Test Requirements	1286

A.5.7.7	CSI-RSRP	1287

A.5.7.7.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1287

A.5.7.7.1.2	Test parameters	1287

A.5.7.7.1.3	Test Requirements	1289

A.5.7.7.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1290

A.5.7.7.2.1	Test Purpose and Environment	1290

A.5.7.7.2.2	Test parameters	1290

A.5.7.7.2.3	Test Requirements	1294

A.5.7.8	CSI-RSRQ	1295

A.5.7.8.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	1295

A.5.7.8.1.1	Test Purpose and Environment	1295

A.5.7.8.1.2	Test Parameters	1295

A.5.7.8.1.3	Test Requirements	1297

A.5.7.8.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1297

A.5.7.8.2.1	Test Purpose and Environment	1297

A.5.7.8.2.2	Test Parameters	1297

A.5.7.9	CSI-SINR	1299

A.5.7.9.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1299

A.5.7.9.1.1	Test Purpose and Environment	1299

A.5.7.9.1.2	Test Parameters	1299

Table A.5.7.9.1.2-1: CSI-SINR Intra frequency CSI-SINR supported test configurations	1300

A.5.7.9.1.3	Test Requirements	1302

A.5.7.9.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1302

A.5.7.9.2.1	Test Purpose and Environment	1302

A.5.7.9.2.2	Test Parameters	1302

A.5.7.9.2.3	Test Requirements	1305

A.5.8	Void	1305

A.6	NR standalone tests with all NR cells in FR1	1011

A.6.1	SA: RRC_IDLE state mobility	1011

A.6.1.1	Cell re-selection to NR	1011

A.6.1.1.1	Cell reselection to FR1 intra-frequency NR case	1011

A.6.1.1.1.1	Test Purpose and Environment	1011

A.6.1.1.1.2	Test Parameters	1011

A.6.1.1.1.3	Test Requirements	1015

A.6.1.1.2	Cell reselection to FR1 inter-frequency NR case	1015

A.6.1.1.2.1	Test Purpose and Environment	1015

A.6.1.1.2.2	Test Parameters	1015

A.6.1.1.2.3	Test Requirements	1019

A.6.1.1.3	Cell reselection to FR1 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1019

A.6.1.1.3.1	Test Purpose and Environment	1019

A.6.1.1.3.2	Test Parameters	1019

A.6.1.1.3.3	Test Requirements	1024

A.6.1.1.4	Cell reselection to FR1 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	1024

A.6.1.1.4.1	Test Purpose and Environment	1024

A.6.1.1.4.2	Test Parameters	1024

A.6.1.1.4.3	Test Requirements	1028

A.6.1.1.5	Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1028

A.6.1.1.5.1	Test Purpose and Environment	1028

A.6.1.1.5.2	Test Parameters	1028

A.6.1.1.5.3	Test Requirements	1032

A.6.1.1.6	Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	1033

A.6.1.1.6.1	Test Purpose and Environment	1033

A.6.1.1.6.2	Test Parameters	1033

A.6.1.1.6.3	Test Requirements	1037

A.6.1.1.7	Cell reselection to FR1 intra-frequency NR case for UE configured with highSpeedMeasFlag-r16	1038

A.6.1.1.7.1	Test Purpose and Environment	1038

A.6.1.1.7.2	Test Parameters	1038

A.6.1.1.7.3	Test Requirements	1042

A.6.1.2	Inter-RAT E-UTRAN cell re-selection	1042

A.6.1.2.1	Cell reselection to higher priority E-UTRAN	1042

A.6.1.2.1.1	Test Purpose and Environment	1042

A.6.1.2.1.2	Test Parameters	1042

A.6.1.2.1.3	Test Requirements	1045

A.6.1.2.2	Cell reselection to lower priority E-UTRAN	1046

A.6.1.2.2.1	Test Purpose and Environment	1046

A.6.1.2.2.2	Test Parameters	1046

A.6.1.2.2.3	Test Requirements	1049

A.6.1.2.3	Cell reselection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion	1050

A.6.1.2.3.1	Test Purpose and Environment	1050

A.6.1.2.3.2	Test Parameters	1050

A.6.1.2.3.3	Test Requirements	1053

A.6.1.2.4	Cell reselection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion	1054

A.6.1.2.4.1	Test Purpose and Environment	1054

A.6.1.2.4.2	Test Parameters	1054

A.6.1.2.4.3	Test Requirements	1057

A.6.1.2.5.3	Test Requirements	1061

A.6.2	SA: RRC_INACTIVE state mobility	1062

A.6.3	RRC_CONNECTED state mobility	1062

A.6.3.1	Handover	1062

A.6.3.1.1	Intra-frequency handover from FR1 to FR1; known target cell	1062

A.6.3.1.1.1	Test Purpose and Environment	1062

A.6.3.1.1.2	Test Parameters	1062

A.6.3.1.1.3 Test Requirements	1066

A.6.3.1.2	Intra-frequency handover from FR1 to FR1; unknown target cell	1066

A.6.3.1.2.1	Test Purpose and Environment	1066

A.6.3.1.2.2	Test Parameters	1066

A.6.3.1.2.3	Test Requirements	1070

A.6.3.1.3	Inter-frequency handover from FR1 to FR1; unknown target cell	1070

A.6.3.1.3.1	Test Purpose and Environment	1070

A.6.3.1.3.2	Test Parameters	1070

A.6.3.1.3.3 Test Requirements	1074

A.6.3.1.4	 SA NR - E-UTRAN handover	1074

A.6.3.1.4.1	Test Purpose and Environment	1074

A.6.3.1.4.2	Test Requirements	1080

A.6.3.1.5	SA NR - E-UTRAN handover with unknown target cell	1080

A.6.3.1.5.1	Test Purpose and Environment	1080

A.6.3.1.5.2	Test Requirements	1086

A.6.3.1.6	 SA NR - UTRAN FDD handover	1086

A.6.3.1.6.1	Test Purpose and Environment	1086

A.6.3.1.6.2	Test Requirements	1090

A.6.3.1.7	Intra-frequency synchronous DAPS handover in FR1	1090

A.6.3.1.7.1	Test Purpose and Environment	1090

A.6.3.1.7.2	Test Parameters	1090

A.6.3.1.7.3 Test Requirements	1094

A.6.3.1.8	Intra-frequency asynchronous DAPS handover in FR1	1095

A.6.3.1.8.1	Test Purpose and Environment	1095

A.6.3.1.8.2	Test Parameters	1095

A.6.3.1.8.3 Test Requirements	1099

A.6.3.1.9	Intra-band inter-frequency synchronous DAPS handover test in SA for FR1	1100

A.6.3.1.9.1	Test Purpose and Environment	1100

A.6.3.1.9.2	Test Parameters	1100

A.6.3.1.9.3 Test Requirements	1104

A.6.3.1.10	Intra-band inter-frequency asynchronous DAPS handover test in SA for FR1	1104

A.6.3.1.10.1	Test Purpose and Environment	1104

A.6.3.1.10.2	Test Parameters	1104

A.6.3.1.10.3 Test Requirements	1107

A.6.3.1.11	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR1	1107

A.6.3.1.11.1	Test Purpose and Environment	1107

A.6.3.1.11.2	Test Parameters	1107

A.6.3.1.11.3 Test Requirements	1114

A.6.3.1.12	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR1	1114

A.6.3.1.12.1	Test Purpose and Environment	1114

A.6.3.1.12.2	Test Parameters	1114

A.6.3.1.12.3 Test Requirements	1122

A.6.3.2	RRC Connection Mobility Control	1122

A.6.3.2.1	SA: RRC Re-establishment	1122

A.6.3.2.1.1	Intra-frequency RRC Re-establishment in FR1	1122

A.6.3.2.1.2	Inter-frequency RRC Re-establishment in FR1	1126

A.6.3.2.1.3	Intra-frequency RRC Re-establishment in FR1 without serving cell timing	1130

A.6.3.2.2	Random Access	1134

A.6.3.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone	1134

A.6.3.2.2.2	4-step RA type non-contention based random access test in FR1 for NR standalone	1139

A.6.3.2.2.3	2-step RA type contention based random access test in FR1 for NR standalone	1143

A.6.3.2.2.4	2-step RA type non-contention based test in FR1 for NR standalone	1148

A.6.3.2.3	SA: RRC Connection Release with Redirection	1152

A.6.3.2.3.1	Redirection from NR in FR1 to NR in FR1	1152

A.6.3.2.3.2	Redirection from NR in FR1 to E-UTRAN	1156

A.6.3.3	Conditional handover	1163

A.6.3.3.1	Intra-frequency conditional handover from FR1 to FR1	1163

A.6.3.3.1.1	Test Purpose and Environment	1163

A.6.3.3.1.2	Test Parameters	1163

A.6.3.3.1.3 Test Requirements	1167

A.6.3.3.2	Inter-frequency conditional handover from FR1 to FR1	1167

A.6.3.3.2.1	Test Purpose and Environment	1167

A.6.3.3.2.2	Test Parameters	1167

A.6.3.3.2.3 Test Requirements	1171

A.6.4	Timing	1171

A.6.4.1	UE transmit timing	1171

A.6.4.1.1	NR UE Transmit Timing Test for FR1	1171

A.6.4.1.1.1	Test Purpose and environment	1171

A.6.4.1.1.2	Test requirements	1175

A.6.4.2	UE timer accuracy	1175

A.6.4.3	Timing advance	1175

A.6.4.3.1	SA FR1 timing advance adjustment accuracy	1175

A.6.4.3.1.1	Test Purpose and Environment	1175

A.6.4.3.1.2	Test Parameters	1175

A.6.4.3.1.3	Test Requirements	1179

A.6.5	Signalling characteristics	1179

A.6.5.1	Radio link Monitoring	1179

A.6.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	1180

A.6.5.1.1.1	Test Purpose and Environment	1180

A.6.5.1.1.2	Test Requirements	1185

A.6.5.1.2	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	1185

A.6.5.1.2.1	Test Purpose and Environment	1185

A.6.5.1.2.2	Test Requirements	1191

A.6.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	1191

A.6.5.1.3.1	Test Purpose and Environment	1191

A.6.5.1.3.2	Test Requirements	1197

A.6.5.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	1197

A.6.5.1.4.1	Test Purpose and Environment	1197

A.6.5.1.4.2	Test Requirements	1203

A.6.5.1.5	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	1203

A.6.5.1.5.1	Test Purpose and Environment	1203

A.6.5.1.5.2	Test Requirements	1209

A.6.5.1.6	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	1209

A.6.5.1.6.1	Test Purpose and Environment	1209

A.6.5.1.6.2	Test Requirements	1214

A.6.5.1.7	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	1214

A.6.5.1.7.1	Test Purpose and Environment	1214

A.6.5.1.7.2	Test Requirements	1218

A.6.5.1.8	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	1218

A.6.5.1.8.1	Test Purpose and Environment	1218

A.6.5.1.8.2	Test Requirements	1224

A.6.5.2	Interruption	1224

A.6.5.2.1	Interruptions during measurements on deactivated NR SCC in FR1	1224

A.6.5.2.1.2	Test Requirements	1228

A.6.5.2.2	SA interruptions at NR SRS carrier based switching	1229

A.6.5.2.2.1	Test Purpose and Environment	1229

A.6.5.2.2.2	Test Parameters	1229

A.6.5.2.2.3	Test Requirements	1231

A.6.5.3	SCell Activation and Deactivation Delay	1232

A.6.5.3.1	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160ms SCell measurement cycle	1232

A.6.5.3.1.1	Test Purpose and Environment	1232

A.6.5.3.1.2	Test Requirements	1237

A.6.5.3.2	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle	1238

A.6.5.3.2.1	Test Purpose and Environment	1238

A.6.5.3.2.2	Test Requirements	1238

A.6.5.3.3	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX	1238

A.6.5.3.3.1	Test Purpose and Environment	1238

A.6.5.3.3.2	Test Requirements	1239

A.6.5.3.4	Direct SCell activation at SCell addition of known SCell in FR1	1239

A.6.5.3.4.1	 Test Purpose and Environment	1239

A.6.5.3.4.2	Test Requirements	1244

A.6.5.3.5	Direct SCell activation at handover with known SCell in FR1	1244

A.6.5.3.5.1	Test Purpose and Environment	1244

A.6.5.3.5.2	Test Requirements	1249

A.6.5.4	UE UL carrier RRC reconfiguration Delay	1250

A.6.5.4.1	UE UL carrier RRC reconfiguration Delay	1250

A.6.5.4.1.1	Test Purpose and Environment	1250

A.6.5.4.1.2	Test Requirements	1260

A.6.5.4.2	Void	1261

A.6.5.5	Beam Failure Detection and Link recovery procedures	1261

A.6.5.5.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	1261

A.6.5.5.1.1	Test Purpose and Environment	1261

A.6.5.5.1.2	Test Requirements	1266

A.6.5.5.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	1267

A.6.5.5.2.1	Test Purpose and Environment	1267

A.6.5.5.2.2	Test Requirements	1273

A.6.5.5.3	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	1274

A.6.5.5.3.1	Test Purpose and Environment	1274

A.6.5.5.3.2	Test Requirements	1279

A.6.5.5.4	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode	1280

A.6.5.5.4.1	Test Purpose and Environment	1280

A.6.5.5.4.2	Test Requirements	1285

A.6.5.5.5	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	1286

A.6.5.5.5.1	Test Purpose and Environment	1286

A.6.5.5.5.2	Test Requirements	1290

A.6.5.5.6	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode	1290

A.6.5.5.6.1	Test Purpose and Environment	1290

A.6.5.5.6.2	Test Requirements	1294

A.6.5.6	Active BWP switch	1295

A.6.5.6.1	DCI-based and Timer-based Active BWP Switch	1295

A.6.5.6.1.1	NR FR1- NR FR1 DL active BWP switch of SCell with non-DRX in SA	1295

A.6.5.6.1.2	NR FR1 DL active BWP switch with non-DRX in SA	1302

A.6.5.6.2	RRC-based Active BWP Switch	1306

A.6.5.6.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	1306

A.6.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1310

A.6.5.6.3.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	1310

A.6.5.6.4	SCell dormancy switch	1317

A.6.5.6.4.1	NR FR1 PCell SCell dormancy switch of single FR1 SCell outside active time	1317

A.6.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR1 SCells inside active time	1323

A.6.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	1330

A.6.5.6.5.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	1330

A.6.5.7	DL interruptions at switching between two uplink carriers	1335

A.6.5.7.1	DL interruptions at switching between two uplink carriers in FDD-TDD CA	1335

A.6.5.7.1.1	Test Purpose and Environment	1335

A.6.5.7.1.2	Test Requirements	1339

A.6.5.7.2	DL interruptions at switching between two uplink carriers in TDD-TDD CA	1339

A.6.5.7.2.1	Test Purpose and Environment	1339

A.6.5.7.2.2	Test Requirements	1343

A.6.5.8	UE specific CBW change	1343

A.6.5.8.1	UE specific CBW change on PCell in FR1 in non-DRX	1343

A.6.5.8.1.1	Test Purpose and Environment	1343

A.6.5.8.1.2	Test Requirements	1347

A.6.5.9	Pathloss reference signal switching delay	1347

A.6.5.9.1	MAC-CE based pathloss reference signal switch delay	1347

A.6.5.9.1.1	Test Purpose and Environment	1347

A.6.5.9.1.2	Test Requirements	1350

A.6.6	Measurement procedure	1351

A.6.6.1	Intra-frequency Measurements	1351

A.6.6.1.1	SA event triggered reporting tests without gap under non-DRX	1351

A.6.6.1.1.1	Test purpose and Environment	1351

A.6.6.1.1.2	Test parameters	1351

A.6.6.1.1.3	Test Requirements	1355

A.6.6.1.2	SA event triggered reporting tests without gap under DRX	1355

A.6.6.1.2.1	Test purpose and Environment	1355

A.6.6.1.2.2	Test parameters	1355

A.6.6.1.2.3	Test Requirements	1359

A.6.6.1.3	SA event triggered reporting tests with per-UE gaps under non-DRX	1359

A.6.6.1.3.1	Test purpose and Environment	1359

A.6.6.1.3.2	Test parameters	1359

A.6.6.1.3.3	Test Requirements	1363

A.6.6.1.4	SA event triggered reporting tests with per-UE gaps under DRX	1363

A.6.6.1.4.1	Test purpose and Environment	1363

A.6.6.1.4.2	Test parameters	1363

A.6.6.1.4.3	Test Requirements	1367

A.6.6.1.5	SA event triggered reporting tests without gap under non-DRX with SSB index reading	1367

A.6.6.1.5.1	Test purpose and Environment	1367

A.6.6.1.5.2	Test parameters	1367

A.6.6.1.5.3	Test Requirements	1369

A.6.6.1.6	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading	1370

A.6.6.1.6.1	Test purpose and Environment	1370

A.6.6.1.6.2	Test parameters	1370

A.6.6.1.6.3	Test Requirements	1371

A.6.6.1.7	SA event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	1372

A.6.6.1.7.1	Test purpose and Environment	1372

A.6.6.1.7.2	Test parameters	1372

A.6.6.1.7.3	Test Requirements	1376

A.6.6.2	Inter-frequency Measurements	1376

A.6.6.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	1376

A.6.6.2.1.1	Test Purpose and Environment	1376

A.6.6.2.1.2	Test Requirements	1380

A.6.6.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used	1380

A.6.6.2.2.1	Test Purpose and Environment	1380

A.6.6.2.2.2	Test Requirements	1384

A.6.6.2.3	Void	1385

A.6.6.2.4	Void	1385

A.6.6.2.5	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	1385

A.6.6.2.5.1	Test Purpose and Environment	1385

A.6.6.2.5.2	Test Requirements	1389

A.6.6.2.6	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used	1389

A.6.6.2.6.1	Test Purpose and Environment	1389

A.6.6.2.6.2	Test Requirements	1393

A.6.6.2.7	Void	1394

A.6.6.2.8	Void	1394

A.6.6.2.9	SA event triggered reporting tests with additional mandatory gap pattern	1394

A.6.6.2.9.1	Test Purpose and Environment	1394

A.6.6.2.9.2	Test Requirements	1397

A.6.6.2.10	SA event triggered reporting tests for FR1 when DRX is used	1397

A.6.6.2.10.1	Test Purpose and Environment	1397

A.6.6.2.10.2	Test Requirements	1401

A.6.6.2.11	SA event triggered reporting tests for FR1 without gap when DRX is not used	1402

A.6.6.2.11.1	Test Purpose and Environment	1402

A.6.6.2.11.2	Test Requirements	1405

A.6.6.3	Inter-RAT Measurements	1405

A.6.6.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1	1405

A.6.6.3.1.1	Test Purpose and Environment	1405

A.6.6.3.1.2	Test Requirements	1411

A.6.6.3.2	SA NR - E-UTRAN event-triggered reporting in DRX in FR1	1411

A.6.6.3.2.1	Test Purpose and Environment	1411

A.6.6.3.2.2	Test Requirements	1418

A.6.6.3.3	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for UE configured with highSpeedMeasFlag-r16	1418

A.6.6.3.3.1	Test Purpose and Environment	1418

A.6.6.3.3.2	Test Requirements	1425

A.6.6.4	L1-RSRP measurement for beam reporting	1425

A.6.6.4.1	SSB based L1-RSRP measurement when DRX is not used	1425

A.6.6.4.1.1	Test Purpose and Environment	1425

A.6.6.4.1.2	Test parameters	1425

A.6.6.4.1.3	Test Requirements	1428

A.6.6.4.2	SSB based L1-RSRP measurement when DRX is used	1428

A.6.6.4.2.1	Test Purpose and Environment	1428

A.6.6.4.2.2	Test parameters	1429

A.6.6.4.2.3	Test Requirements	1432

A.6.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	1432

A.6.6.4.3.1	Test Purpose and Environment	1432

A.6.6.4.3.2	Test parameters	1433

A.6.6.4.3.3	Test Requirements	1436

A.6.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	1436

A.6.6.4.4.1	Test Purpose and Environment	1436

A.6.6.4.4.2	Test parameters	1437

A.6.6.4.4.3	Test Requirements	1440

A.6.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	1440

A.6.6.4.5.1	Test Purpose and Environment	1440

A.6.6.4.5.2	Test parameters	1441

A.6.6.4.5.3	Test Requirements	1444

A.6.6.5	1444

A.6.6.5.1	SA NR - UTRAN FDD event-triggered reporting in non-DRX in FR1	1444

A.6.6.5.1.1	Test Purpose and Environment	1444

A.6.6.5.1.2	Test Requirements	1448

A.6.6.6	CLI measurements	1448

A.6.6.6.1	SRS-RSRP measurement with DRX	1448

A.6.6.6.1.1	Test Purpose and Environment	1448

A.6.6.6.1.2	Test Parameters	1449

A.6.6.6.1.3	Test Requirements	1452

A.6.6.6.2	CLI-RSSI measurement with DRX	1452

A.6.6.6.2.1	Test Purpose and Environment	1452

A.6.6.6.2.2	Test Parameters	1453

A.6.6.6.2.3	Test Requirements	1455

A.6.6.7	NR measurements with autonomous gaps	1455

A.6.6.7.1	SA intra-frequency CGI identification of NR neighbor cell in FR1	1455

A.6.6.7.1.1	Test Purpose and Environment	1455

A.6.6.7.1.2	Test Parameters	1455

A.6.6.7.1.3	Test Requirements	1458

A.6.6.7.2	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA	1458

A.6.6.7.2.1	Test Purpose and Environment	1458

A.6.6.7.2.2	Test Requirements	1461

A.6.6.8	L1-SINR measurement for beam reporting	1462

A.6.6.8.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured when DRX is used	1462

A.6.6.8.1.1	Test Purpose and Environment	1462

A.6.6.8.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	1465

A.6.6.8.2.1	Test Purpose and Environment	1465

A.6.6.8.2.2	Test parameters	1466

A.6.6.8.2.3	Test Requirements	1470

A.6.6.8.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	1470

A.6.6.8.3.1	Test Purpose and Environment	1470

A.6.6.8.3.2	Test parameters	1471

A.6.6.8.3.3	Test Requirements	1474

A.6.6.9	Idle Mode CA/DC Measurements	1474

A.6.6.9.1	SA Idle mode CA/DC measurement for FR1	1474

A.6.6.9.1.1	Test Purpose and Environment	1474

A.6.6.9.1.2	Test Requirements	1481

A.6.6.10	CSI-RS based intra-frequency Measurements	1481

A.6.6.10.1	SA event triggered reporting tests without gap under non-DRX	1481

A.6.6.10.1.1	Test purpose and Environment	1481

A.6.6.10.1.2	Test Requirements	1484

A.6.6.11	CSI-RS based inter-frequency Measurements	1484

A.6.6.11.1	 SA event triggered reporting tests with gap under DRX	1484

A.6.6.11.1.1	Test Purpose and Environment	1484

A.6.6.11.1.2	Test Requirements	1488

A.6.6.12	RSTD measurements	1488

A. 6.6.12.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA	1488

A. 6.6.12.1.1	Test Purpose and Environment	1488

A.6.6.12.1.2	Test Requirements	1496

A. 6.6.12.2	NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR1 SA	1496

A. 6.6.12.2.1	Test Purpose and Environment	1496

A.6.6.12.2.2	Test Requirements	1503

A.6.6.13 PRS-RSRP measurements	1503

A.6.6.13.1	PRS-RSRP reporting delay test case for single positioning frequency layer	1503

A.6.6.13.1.1	Test purpose and Environment	1503

A.6.6.13.1.2	Test Requirements	1507

A.6.6.13.2	PRS-RSRP reporting delay test case for dual positioning frequency layer	1507

A.6.6.13.2.1	Test purpose and Environment	1507

A.6.6.13.2.2	Test Requirements	1511

A.6.6.14	UE Rx-Tx time difference measurements	1511

A.6.6.14.1 UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA	1511

A.6.6.14.1.1	Test purpose and environment	1511

A.6.6.14.1.2	Test requirements	1515

A.6.6.14.2 UE Rx-Tx time difference measurement for dual positioning frequency layers in FR1 SA	1515

A.6.6.14.2.1	Test purpose and environment	1515

A.6.6.14.2.2	Test requirements	1519

A.6.6.15	Idle Mode measurements of inter-RAT DC candidate cells for early reporting	1519

A.6.6.15.1	Test Purpose and Environment	1519

A.6.6.15.2	Test Requirements	1526

A.6.7	Measurement Performance requirements	1527

A.6.7.1	SS-RSRP	1527

A.6.7.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	1527

A.6.7.1.1.1	Test Purpose and Environment	1527

A.6.7.1.1.2	Test parameters	1527

A.6.7.1.1.3	Test Requirements	1532

A.6.7.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	1532

A.6.7.1.2.1	Test Purpose and Environment	1532

A.6.7.1.2.2	Test parameters	1532

A.6.7.1.2.3	Test Requirements	1536

A.6.7.1.3	Void	1536

A.6.7.2	SS-RSRQ	1536

A.6.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1536

A.6.7.2.1.1	Test Purpose and Environment	1536

A.6.7.2.1.2	Test Parameters	1537

A.6.7.2.1.3	Test Requirements	1542

A.6.7.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1542

A.6.7.2.2.1	Test Purpose and Environment	1542

A.6.7.2.2.2	Test Parameters	1542

A.6.7.2.2.3	Test Requirements	1547

A.6.7.3	SS-SINR	1547

A.6.7.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1547

A.6.7.3.1.1	Test Purpose and Environment	1547

A.6.7.3.1.2	Test Parameters	1547

A.6.7.3.1.3	Test Requirements	1551

A.6.7.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1551

A.6.7.3.2.1	Test Purpose and Environment	1551

A.6.7.3.2.2	Test Parameters	1552

A.6.7.3.2.3	Test Requirements	1556

A.6.7.4	L1-RSRP measurement for beam reporting	1557

A.6.7.4.1	SSB based L1-RSRP measurement	1557

A.6.7.4.1.1	Test Purpose and Environment	1557

A.6.7.4.1.2	Test parameters	1557

A.6.7.4.1.3	Test Requirements	1561

A.6.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	1561

A.6.7.4.2.1	Test Purpose and Environment	1561

A.6.7.4.2.2	Test parameters	1562

A.6.7.4.2.3	Test Requirements	1566

A.6.7.5	E-UTRAN RSRP	1566

A.6.7.5.1	SA: inter-RAT measurement accuracy with FR1 serving cell	1566

A.6.7.5.1.1	Test Purpose and Environment	1566

A.6.7.5.1.2	Test parameters	1567

A.6.7.5.1.3	Test Requirements	1573

A.6.7.6	E-UTRAN RSRQ	1574

A.6.7.6.1	SA: inter-RAT measurement accuracy with FR1 serving cell	1574

A.6.7.6.1.1	Test Purpose and Environment	1574

A.6.7.6.1.2	Test parameters	1574

A.6.7.6.1.3	Test Requirements	1579

A.6.7.7	E-UTRAN RS-SINR	1580

A.6.7.7.1	SA: inter-RAT measurement accuracy with FR1 serving cell	1580

A.6.7.7.1.1	Test Purpose and Environment	1580

A.6.7.7.1.2	Test parameters	1580

A.6.7.7.1.3	Test Requirements	1586

A.6.7.8	CLI measurements	1587

A.6.7.8.1	SA SRS-RSRP measurement accuracy with FR1 serving cell	1587

A.6.7.8.1.1	Test Purpose and Environment	1587

A.6.7.8.1.2	Test parameters	1587

A.6.7.8.1.3	Test Requirements	1593

A.6.7.8.2	SA CLI-RSSI measurement accuracy with FR1 serving cell	1593

A.6.7.8.2.1	Test Purpose and Environment	1593

A.6.7.8.2.2	Test parameters	1594

A.6.7.8.2.3	Test Requirements	1597

A.6.7.9	L1-SINR measurement for beam reporting	1598

A.6.7.9.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	1598

A.6.7.9.1.1	Test Purpose and Environment	1598

A.6.7.9.1.2	Test parameters	1598

A.6.7.9.1.3	Test Requirements	1602

A.6.7.9.2	L1-SINR measurement with SSB based CMR and dedicated IMR	1602

A.6.7.9.2.1	Test Purpose and Environment	1602

A.6.7.9.2.2	Test parameters	1603

A.6.7.9.2.3	Test Requirements	1608

A.6.7.9.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	1608

A.6.7.9.3.1	Test Purpose and Environment	1608

A.6.7.9.3.2	Test parameters	1608

A.6.7.9.3.3	Test Requirements	1613

A.6.7.10	CSI-RSRP	1613

A.6.7.10.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	1613

A.6.7.10.1.1	Test Purpose and Environment	1613

A.6.7.10.1.2	Test parameters	1614

A.6.7.10.1.3	Test Requirements	1619

A.6.7.10.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	1619

A.6.7.10.2.1	Test Purpose and Environment	1619

A.6.7.10.2.2	Test parameters	1619

A.6.7.10.2.3	Test Requirements	1624

A.6.7.11	CSI-RSRQ	1624

A.6.7.11.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1624

A.6.7.11.1.1	Test Purpose and Environment	1624

A.6.7.11.1.2	Test Parameters	1624

A.6.7.11.1.3	Test Requirements	1629

A.6.7.11.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1629

A.6.7.11.2.1	Test Purpose and Environment	1629

A.6.7.11.2.2	Test Parameters	1629

A.6.7.11.2.3	Test Requirements	1634

A.6.7.12	CSI-SINR	1634

A.6.7.12.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1634

A.6.7.12.1.1	Test Purpose and Environment	1634

A.6.7.12.1.2	Test Parameters	1635

A.6.7.12.1.3	Test Requirements	1639

A.6.7.12.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1640

A.6.7.12.2.1	Test Purpose and Environment	1640

A.6.7.12.2.2	Test Parameters	1640

A.6.7.12.2.3	Test Requirements	1646

A.6.7.13	RSTD measurements	1646

A.6.7.13.1	RSTD measurement accuracy test case for single positioning frequency layer	1646

A.6.7.13.1.1	Test purpose and Environment	1646

A.6.7.13.1.2	Test Requirements	1649

A.6.7.13.2	RSTD measurement accuracy test case for dual positioning frequency layer	1649

A.6.7.13.2.1	Test purpose and Environment	1649

A.6.7.13.2.2	Test Requirements	1653

A.6.7.14	PRS-RSRP measurements	1653

A.6.7.14.1	SA: measurement accuracy with PRS in FR1	1653

A.6.7.14.1.1	Test Purpose and Environment	1653

A.6.7.14.1.2	Test parameters	1654

A.6.7.14.1.3	Test Requirements	1658

A.6.7.15	UE Rx-Tx time difference measurements	1658

A.6.7.15.1 UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR1 SA	1658

A.6.7.15.1.1	Test purpose and environment	1658

A.6.7.15.1.2	Test parameters	1659

A.6.7.15.1.3	Test requirements	1662

A.7.1	SA: RRC_IDLE state mobility	1663

A.7.1.1	Cell re-selection to NR	1663

A.7.1.1.1	Cell reselection to FR2 intra-frequency NR case	1663

A.7.1.1.1.1	Test Purpose and Environment	1663

A.7.1.1.1.2	Test Parameters	1663

A.7.1.1.1.3	Test Requirements	1666

A.7.1.1.2	Cell reselection to FR2 inter-frequency NR case	1666

A.7.1.1.2.1	Test Purpose and Environment	1666

A.7.1.1.2.2	Test Parameters	1667

A.7.1.1.2.3	Test Requirements	1669

A.7.1.1.3	Cell reselection to FR2 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1669

A.7.1.1.3.1	Test Purpose and Environment	1669

A.7.1.1.3.2	Test Parameters	1669

A.7.1.1.3.3	Test Requirements	1672

A.7.1.1.4	Cell reselection to FR2 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	1672

A.7.1.1.4.1	Test Purpose and Environment	1672

A.7.1.1.4.2	Test Parameters	1672

A.7.1.1.4.3	Test Requirements	1675

A.7.1.1.5	Cell reselection to FR2 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1675

A.7.1.1.5.1	Test Purpose and Environment	1675

A.7.1.1.5.2	Test Parameters	1675

A.7.1.1.5.3	Test Requirements	1678

A.7.1.1.6	Cell reselection to FR2 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	1678

A.7.1.1.6.1	Test Purpose and Environment	1678

A.7.1.1.6.2	Test Parameters	1678

A.7.1.1.6.3	Test Requirements	1681

A.7.2	SA: RRC_INACTIVE state mobility	1681

A.7.3	RRC_CONNECTED state mobility	1681

A.7.3.1	Handover	1681

A.7.3.1.1	Inter-frequency handover from FR1 to FR2; unknown target cell	1681

A.7.3.1.1.1	Test Purpose and Environment	1681

A.7.3.1.1.2	Test Parameters	1681

A.7.3.1.1.3	Test Requirements	1685

A.7.3.1.2	Intra-frequency handover from FR2 to FR2; unknown target cell	1685

A.7.3.1.2.1	Test Purpose and Environment	1685

A.7.3.1.2.2	Test Parameters	1685

A.7.3.1.2.3	Test Requirements	1688

A.7.3.1.3	Inter-frequency handover from FR2 to FR2; unknown target cell	1688

A.7.3.1.3.1	Test Purpose and Environment	1688

A.7.3.1.3.2	Test Parameters	1688

A.7.3.1.3.3 Test Requirements	1689

A.7.3.1.4	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR2	1690

A.7.3.1.4.1	Test Purpose and Environment	1690

A.7.3.1.4.2	Test Parameters	1690

A.7.3.1.4.3 Test Requirements	1697

A.7.3.1.5	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR2	1697

A.7.3.1.5.1	Test Purpose and Environment	1697

A.7.3.1.5.2	Test Parameters	1697

A.7.3.1.5.3 Test Requirements	1704

A.7.3.2	RRC Connection Mobility Control	1704

A.7.3.2.1	SA: RRC Re-establishment	1704

A.7.3.2.1.1	Intra-frequency RRC Re-establishment in FR2	1704

A.7.3.2.1.2	Inter-frequency RRC Re-establishment in FR2	1707

A.7.3.2.1.3	Intra-frequency RRC Re-establishment in FR2 without serving cell timing	1710

A.7.3.2.1.3.1	Test Purpose and Environment	1710

A.7.3.2.1.3.2	Test Requirements	1712

A.7.3.2.2	Random Access	1713

A.7.3.2.2.1	4-step RA type contention based random access test in FR2 for NR Standalone	1713

A.7.3.2.2.2	4-step RA type non-contention based random access test in FR2 for NR Standalone	1717

A.7.3.2.2.3	2-step RA type contention based random access test in FR2 for NR Standalone	1722

A.7.3.2.2.4	2-step RA type non-contention based random access test in FR2 for NR Standalone	1725

A.7.3.2.3	SA: RRC Connection Release with Redirection	1728

A.7.3.2.3.1	Redirection from NR in FR2 to NR in FR2	1728

A.7.3.3	Conditional Handover	1732

A.7.3.3.1	Intra-frequency conditional handover from FR2 to FR2	1732

A.7.3.3.1.1	Test Purpose and Environment	1732

A.7.3.3.1.2	Test Parameters	1732

A.7.3.3.1.2.3	Test Requirements	1735

A.7.3.3.2	Inter-frequency conditional handover from FR2 to FR2; unknown target cell	1735

A.7.3.3.2.1	Test Purpose and Environment	1735

A.7.3.3.2.2	Test Parameters	1735

A.7.3.3.2.3 Test Requirements	1738

A.7.4	Timing	1738

A.7.4.1	UE transmit timing	1738

A.7.4.1.1	NR UE Transmit Timing Test for FR2	1738

A.7.4.1.1.1	Test Purpose and environment	1738

A.7.4.1.1.2	Test requirements	1741

A.7.4.2	UE timer accuracy	1742

A.7.4.3	Timing advance	1742

A.7.4.3.1	SA FR2 timing advance adjustment accuracy	1742

A.7.4.3.1.1	Test Purpose and Environment	1742

A.7.4.3.1.2	Test Parameters	1742

A.7.4.3.1.3 Test Requirements	1745

A.7.5	Signaling characteristics	1746

A.7.5.1	Radio link Monitoring	1746

A.7.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	1746

A.7.5.1.1.1	Test Purpose and Environment	1746

A.7.5.1.1.2	Test Requirements	1749

A.7.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	1750

A.7.5.1.2.1	Test Purpose and Environment	1750

A.7.5.1.2.2	Test Requirements	1754

A.7.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	1755

A.7.5.1.3.1	Test Purpose and Environment	1755

A.7.5.1.3.2	Test Requirements	1759

A.7.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	1759

A.7.5.1.4.1	Test Purpose and Environment	1759

A.7.5.1.4.2	Test Requirements	1764

A.7.5.1.5	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	1764

A.7.5.1.5.1	Test Purpose and Environment	1764

A.7.5.1.5.2	Test Requirements	1769

A.7.5.1.6	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	1769

A.7.5.1.6.1	Test Purpose and Environment	1769

A.7.5.1.6.2	Test Requirements	1773

A.7.5.1.7	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	1773

A.7.5.1.7.1	Test Purpose and Environment	1773

A.7.5.1.7.2	Test Requirements	1777

A.7.5.1.8	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	1777

A.7.5.1.8.1	Test Purpose and Environment	1777

A.7.5.1.8.2	Test Requirements	1782

A.7.5.1.9	UE Radio Link Monitoring Scheduling Restrictions on FR2	1782

A.7.5.1.9.1	Test Purpose and Environment	1782

A.7.5.1.9.2	Test Requirements	1785

A.7.5.2	Interruption	1785

A.7.5.2.1	Interruptions during measurements on deactivated NR SCC in FR2	1785

A.7.5.2.2	SA interruptions at NR SRS carrier-based switching	1789

A.7.5.2.2.1	Test Purpose and Environment	1789

A.7.5.2.2.2	Test Parameters	1789

A.7.5.2.2.3	Test Requirements	1791

A.7.5.3	SCell Activation and Deactivation Delay	1791

A.7.5.3.1	SCell Activation and deactivation for SCell in FR2 intra-band in non-DRX	1791

A.7.5.3.1.1	Test Purpose and Environment	1791

A.7.5.3.1.2	Test Requirements	1793

A.7.5.3.2	SCell Activation and deactivation for FR1+FR2 inter-band with target SCell in FR2	1793

A.7.5.3.2.1	Test Purpose and Environment	1793

A.7.5.3.2.2	Test Requirements	1797

A.7.5.3.3	SCell Activation and deactivation for SCell in FR2 inter-band in non-DRX	1798

A.7.5.3.3.1	Test Purpose and Environment	1798

A.7.5.3.3.2	Test Requirements	1801

A.7.5.3.4	Direct SCell activation at SCell addition of known SCell in FR2	1802

A.7.5.3.4.1	Test Purpose and Environment	1802

A.7.5.3.4.2	Test Requirements	1805

A.7.5.3.5	Direct SCell activation at handover with known SCell in FR2	1806

A.7.5.3.5.1	Test Purpose and Environment	1806

A.7.5.3.5.2	Test Requirements	1809

A.7.5.4	Void	1810

A.7.5.5	Beam Failure Detection and Link recovery procedures	1810

A.7.5.5.1	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	1810

A.7.5.5.1.1	Test Purpose and Environment	1810

A.7.5.5.1.2	Test Requirements	1814

A.7.5.5.2	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode	1815

A.7.5.5.2.1	Test Purpose and Environment	1815

A.7.5.5.2.2	Test Requirements	1818

A.7.5.5.3	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	1819

A.7.5.5.3.1	Test Purpose and Environment	1819

A.7.5.5.3.2	Test Requirements	1823

A.7.5.5.4	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode	1824

A.7.5.5.4.1	Test Purpose and Environment	1824

A.7.5.5.4.2	Test Requirements	1828

A.7.5.5.5	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	1829

A.7.5.5.5.1	Test Purpose and Environment	1829

A.7.5.5.5.2	Test Requirements	1832

A.7.5.5.6	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	1833

A.7.5.5.6.1	Test Purpose and Environment	1833

A.7.5.5.6.2	Test Requirements	1837

A.7.5.5.7	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	1838

A.7.5.5.7.1	Test Purpose and Environment	1838

A.7.5.5.7.2	Test Requirements	1842

A.7.5.6	Active BWP switch	1843

A.7.5.6.1	DCI-based and Timer-based Active BWP Switch	1843

A.7.5.6.1.1	NR FR2- NR FR2 DL active BWP switch of SCell with non-DRX in SA	1843

A.7.5.6.1.2	NR FR1- NR FR2 DL active BWP switch of SCell with non-DRX in SA A.7.5.6.1.2.1 Test Purpose and Environment	1847

A.7.5.6.1.3	NR FR2 DL active BWP switch with non-DRX in SA	1852

A.7.5.6.1.3.1	Test Purpose and Environment	1852

A.7.5.6.1.3.2	Test Requirements	1855

A.7.5.6.2	RRC-based Active BWP Switch	1855

A.7.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1859

A.7.5.6.3.1	Active BWP switch on multiple SCells with non-DRX in SA	1859

A.7.5.6.4	SCell dormancy switch	1862

A.7.5.6.4.1	NR FR2 PCell SCell dormancy switch of single FR2 SCell inside active time	1862

A.7.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR2 SCells outside active time	1867

A.7.5.6.4.2.2	 Test Requirements	1872

A.7.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	1872

A.7.5.6.5.1	Active BWP switch on multiple SCells with non-DRX in SA	1872

A.7.5.7	PSCell addition and release delay	1875

A.7.5.7.1	Addition and Release Delay of known NR PSCell	1875

A.7.5.7.1.1	Test Purpose and Environment	1875

A.7.5.7.2	Addition and Release Delay of unknown NR PSCell	1879

A.7.5.7.2.1	Test Purpose and Environment	1879

A.7.5.7.2.2	Test Requirements	1882

A.7.5.8	Active TCI state switch delay	1882

A.7.5.8.1	MAC-CE based active TCI state switch	1882

A.7.5.8.2	RRC based active TCI state switch	1886

A.7.5.9	Uplink spatial relation switch delay	1890

A.7.5.9.1	MAC-CE based Spatial Relation switch	1890

A.7.5.9.1.1	 NR PCell FR2 spatial relation associated with known DL-RS	1890

A.7.5.9.1.1.2	Test Requirements	1893

A.7.5.9.2	RRC based spatial relation switch	1893

A.7.5.9.2.1	NR PCell FR2 spatial relation switch associated with a known DL-RS	1893

A.7.5.9.2.1.1	Test Purpose and Environment	1893

A.7.5.9.2.1.2	Test Requirements	1896

A.7.5.10	UE specific CBW change	1896

A.7.5.10.1	NR FR2 UE specific CBW change of PCell with non-DRX in SA	1896

A.7.5.10.1.1	Test Purpose and Environment	1896

A.7.5.10.1.2	Test Requirements	1899

A.7.6	Measurement procedure	1900

A.7.6.1	Intra-frequency Measurements	1900

A.7.6.1.1	SA event triggered reporting test without gap under non-DRX	1900

A.7.6.1.1.1	Test purpose and Environment	1900

A.7.6.1.1.2	Test Requirements	1902

A.7.6.1.2	SA event triggered reporting test without gap under DRX	1903

A.7.6.1.2.1	Test purpose and Environment	1903

A.7.6.1.2.2	Test Requirements	1905

A.7.6.1.3	SA event triggered reporting test with per-UE gaps under non-DRX	1906

A.7.6.1.3.1	Test purpose and Environment	1906

A.7.6.1.3.2	Test Requirements	1908

A.7.6.1.4	SA event triggered reporting test with per-UE gaps under DRX	1909

A.7.6.1.4.1	Test purpose and Environment	1909

A.7.6.1.4.2	Test Requirements	1912

A.7.6.2	Inter-frequency Measurements	1913

A.7.6.2.1	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2)	1913

A.7.6.2.1.1	Test Purpose and Environment	1913

A.7.6.2.1.2	Test Requirements	1916

A.7.6.2.2	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is used (PCell in FR2)	1916

A.7.6.2.2.1	Test Purpose and Environment	1916

A.7.6.2.2.2	Test Requirements	1920

A.7.6.2.3	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is not used (PCell in FR2)	1920

A.7.6.2.3.1	Test Purpose and Environment	1920

A.7.6.2.3.2	Test Requirements	1924

A.7.6.2.4	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is used (PCell in FR2)	1924

A.7.6.2.4.1	Test Purpose and Environment	1924

A.7.6.2.4.2	Test Requirements	1928

A.7.6.2.5	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR1)	1928

A.7.6.2.5.1	Test Purpose and Environment	1928

A.7.6.2.5.2	Test Requirements	1932

A.7.6.2.6	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (PCell in FR1)	1933

A.7.6.2.6.1	Test Purpose and Environment	1933

A.7.6.2.6.2	Test Requirements	1937

A.7.6.2.7	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR1)	1938

A.7.6.2.7.1	Test Purpose and Environment	1938

A.7.6.2.7.2	Test Requirements	1942

A.7.6.2.8	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR1)	1943

A.7.6.2.8.1	Test Purpose and Environment	1943

A.7.6.2.8.2	Test Requirements	1947

A.7.6.2.9	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2) (rel16 additional mandatory gap pattern 17)	1948

A.7.6.2.9.1	Test Purpose and Environment	1948

A.7.6.2.9.2	Test Requirements	1951

A.7.6.2.10	SA event triggered reporting test without gap under non-DRX	1951

A.7.6.2.10.1	Test Purpose and Environment	1951

A.7.6.2.10.2	Test Requirements	1953

A.7.6.2.11	SA event triggered reporting test without gap under DRX	1953

A.7.6.2.11.1	Test Purpose and Environment	1953

A.7.6.2.11.2	Test Requirements	1956

A.7.6.3	L1-RSRP measurement for beam reporting	1957

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	1957

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	1957

A.7.6.3.1.1	Test Purpose and Environment	1957

A.7.6.3.1.2	Test parameters	1957

A.7.6.3.1.3	Test Requirements	1959

A.7.6.3.2	SSB based L1-RSRP measurement when DRX is used	1959

A.7.6.3.2.1	Test Purpose and Environment	1959

A.7.6.3.2.2	Test parameters	1960

A.7.6.3.2.3	Test Requirements	1962

A.7.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	1962

A.7.6.3.3.1	Test Purpose and Environment	1962

A.7.6.3.3.2	Test parameters	1963

A.7.6.3.3.3	Test Requirements	1965

A.7.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	1966

A.7.6.3.4.1	Test Purpose and Environment	1966

A.7.6.3.4.2	Test parameters	1966

A.7.6.3.3.3	Test Requirements	1968

A.7.6.4	CLI measurements	1969

A.7.6.4.1	SRS-RSRP measurement with non-DRX	1969

A.7.6.4.1.1	Test Purpose and Environment	1969

A.7.6.4.1.2	Test Parameters	1969

A.7.6.4.1.3	Test Requirements	1971

A.7.6.4.2	CLI-RSSI measurement with non-DRX	1971

A.7.6.4.2.1	Test Purpose and Environment	1971

A.7.6.4.2.2	Test Parameters	1972

A.7.6.4.2.3	Test Requirements	1973

A.7.6.5	NR Measurements with autonomous gaps	1974

A.7.6.5.1	SA interfrequency CGI reporting in autonomous gaps test (PCell in FR2)	1974

A.7.6.5.1.1	Test Purpose and Environment	1974

A.7.6.5.1.2	Test Requirements	1977

A.7.6.6	L1-SINR measurement for beam reporting	1977

A.7.6.6.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured when DRX is not used	1977

A.7.6.6.1.1	Test Purpose and Environment	1977

A.7.6.6.1.2	Test parameters	1977

A.7.6.6.1.3	Test Requirements	1979

A.7.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	1979

A.7.6.6.2.1	Test Purpose and Environment	1979

A.7.6.6.2.2	Test parameters	1980

A.7.6.6.2.3	Test Requirements	1982

A.7.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	1982

A.7.6.6.3.1	Test Purpose and Environment	1982

A.7.6.6.3.2	Test parameters	1983

A.7.6.6.3.3	Test Requirements	1985

A.7.6.7	CSI-RS based intra-frequency Measurements	1985

A.7.6.7.1	SA event triggered reporting test without gap under DRX for CSI-RS based intra-frequency measurement	1985

A.7.6.7.1.1	Test purpose and Environment	1985

A.7.6.7.1.2	Test Requirements	1988

A.7.6.8	CSI-RS based inter-frequency Measurements	1988

A.7.6.8.1	SA event triggered reporting tests for FR2 CSI-RS based measurement when non-DRX is used (PCell in FR2)	1988

A.7.6.8.1.1	Test Purpose and Environment	1988

A.7.6.2.2.2	Test Requirements	1992

A.7.6.9	RSTD measurements	1992

A.7.6.9.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA	1992

A.7.6.9.1.1	Test Purpose and Environment	1992

A.7.6.9.1.2	Test Requirements	1998

A.7.6.9.2	 NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR2 SA	1998

A.7.6.9.2.1	Test Purpose and Environment	1998

A.7.6.9.2.2	Test Requirements	2005

A.7.6.10 PRS-RSRP measurements	2005

A.7.6.10.1 PRS-RSRP reporting delay test case for single positioning frequency layer	2005

A.7.6.10.1.1	Test Purpose and Environment	2005

A.7.6.10.1.2	Test Requirements	2009

A.7.6.10.2 PRS-RSRP reporting delay test case for dual positioning frequency layer	2009

A.7.6.10.2.1	Test Purpose and Environment	2009

A.7.6.10.2.2	Test Requirements	2012

A.7.6.11	UE Rx-Tx time difference measurements	2012

A.7.6.11.1 UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA	2012

A.7.6.11.1.1	Test purpose and environment	2012

A.7.6.11.1.2	Test requirements	2016

A.7.6.11.2 UE Rx-Tx time difference measurement period for dual positioning frequency layers in FR2 SA	2016

A.7.6.11.2.1	Test purpose and environment	2016

A.7.6.11.2.2	Test requirements	2020

A.7.7	Measurement Performance requirements	2020

A.7.7.1	SS-RSRP	2020

A.7.7.1.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2020

A.7.7.1.1.1	Test Purpose and Environment	2020

A.7.7.1.1.2	Test parameters	2021

A.7.7.1.1.3	Test Requirements	2025

A.7.7.1.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2025

A.7.7.1.2.1	Test Purpose and Environment	2025

A.7.7.1.2.2	Test parameters	2026

A.7.7.1.2.3	Test Requirements	2030

A.7.7.1.3	SA inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	2031

A.7.7.1.3.1	Test Purpose and Environment	2031

A.7.7.1.3.2	Test parameters	2031

A.7.7.1.3.3	Test Requirements	2035

A.7.7.2	SS-RSRQ	2035

A.7.7.2.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	2035

A.7.7.2.1.1	Test Purpose and Environment	2035

A.7.7.2.1.2	Test Parameters	2035

A.7.7.2.1.3	Test Requirements	2037

A.7.7.2.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2037

A.7.7.2.2.1	Test Purpose and Environment	2037

A.7.7.2.2.2	Test Parameters	2037

A.7.7.2.2.3	Test Requirements	2039

A.7.7.3	SS-SINR	2039

A.7.7.3.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2039

A.7.7.3.1.1	Test Purpose and Environment	2039

A.7.7.3.1.2	Test Parameters	2039

A.7.7.3.1.3	Test Requirements	2042

A.7.7.3.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2042

A.7.7.3.2.1	Test Purpose and Environment	2042

A.7.7.3.2.2	Test Parameters	2042

A.7.7.3.2.3	Test Requirements	2044

A.7.7.4	L1-RSRP measurement for beam reporting	2044

A.7.7.4.1	SSB based L1-RSRP measurement	2044

A.7.7.4.1.1	Test Purpose and Environment	2044

A.7.7.4.1.2	Test parameters	2045

A.7.7.4.1.3	Test Requirements	2047

A.7.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	2048

A.7.7.4.2.1	Test Purpose and Environment	2048

A.7.7.4.2.2	Test parameters	2048

A.7.7.4.2.3	Test Requirements	2050

A.7.7.5	CLI measurements	2051

A.7.7.5.1	SA SRS-RSRP measurement accuracy with FR2 serving cell	2051

A.7.7.5.1.1	Test Purpose and Environment	2051

A.7.7.5.1.2	Test parameters	2051

A.7.7.5.1.3	Test Requirements	2054

A.7.7.5.2	SA CLI-RSSI measurement accuracy with FR2 serving cell	2055

A.7.7.5.2.1	Test Purpose and Environment	2055

A.7.7.5.2.2	Test parameters	2055

A.7.7.5.2.3	Test Requirements	2057

A.7.7.6	L1-SINR measurement for beam reporting	2058

A.7.7.6.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	2058

A.7.7.6.1.1	Test Purpose and Environment	2058

A.7.7.6.1.2	Test parameters	2058

A.7.7.6.1.3	Test Requirements	2060

A.7.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	2061

A.7.7.6.2.1	Test Purpose and Environment	2061

A.7.7.6.2.2	Test parameters	2061

A.7.7.6.2.3	Test Requirements	2063

A.7.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	2064

A.7.7.6.3.1	Test Purpose and Environment	2064

A.7.7.6.3.2	Test parameters	2064

A.7.7.6.3.3	Test Requirements	2066

A.7.7.7	CSI-RSRP	2067

A.7.7.7.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2067

A.7.7.7.1.1	Test Purpose and Environment	2067

A.7.7.7.1.2	Test parameters	2067

A.7.7.7.1.3	Test Requirements	2070

A.7.7.7.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2071

A.7.7.7.2.1	Test Purpose and Environment	2071

A.7.7.7.2.2	Test parameters	2071

A.7.7.7.2.3	Test Requirements	2074

A.7.7.8	CSI-RSRQ	2075

A.7.7.8.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	2075

A.7.7.8.1.1	Test Purpose and Environment	2075

A.7.7.8.1.2	Test Parameters	2075

A.7.7.8.1.3	Test Requirements	2077

A.7.7.8.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2077

A.7.7.8.2.1	Test Purpose and Environment	2077

A.7.7.8.2.2	Test Parameters	2077

A.7.7.8.2.3	Test Requirements	2079

A.7.7.9	CSI-SINR	2079

A.7.7.9.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2079

A.7.7.9.1.1	Test Purpose and Environment	2079

A.7.7.9.1.2	Test Parameters	2079

A.7.7.9.1.3	Test Requirements	2082

A.7.7.9.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2082

A.7.7.9.2.2	Test Parameters	2082

A.7.7.9.2.3	Test Requirements	2084

A.7.7.10	RSTD measurements	2084

A.7.7.10.1	RSTD measurement accuracy test case for single positioning frequency layer	2084

A.7.7.10.1.1	Test purpose and Environment	2084

A.7.7.10.1.2	Test Requirements	2087

A.7.7.10.2	RSTD measurement accuracy test case for dual positioning frequency layer	2087

A.7.7.10.2.1	Test purpose and Environment	2087

A.7.7.10.2.2	Test Requirements	2090

A.7.7.11	PRS-RSRP measurements	2090

A.7.7.11.1	SA measurement accuracy with PRS in FR2	2090

A.7.7.11.1.1	Test Purpose and Environment	2090

A.7.7.11.1.2	Test parameters	2090

A.7.7.11.1.3	Test Requirements	2094

A.7.7.12	UE Rx-Tx time difference measurements	2095

A.7.7.12.1 UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR2 SA	2095

A.7.7.12.1.1	Test purpose and environment	2095

A.7.7.12.1.2	Test parameters	2095

A.7.7.12.1.3	Test requirements	2098

A.8	E-UTRA standalone tests for NR RRM	2098

A.8.1	Void	2099

A.8.2	RRC_IDLE state mobility	2099

A.8.2.1	Inter-RAT NR Cell re-selection	2099

A.8.2.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1	2099

A.8.2.1.1.1	Test Purpose and Environment	2099

A.8.2.1.1.2	Test Requirements	2104

A.8.2.1.2	E-UTRA Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16	2105

A.8.2.1.2.1	Test Purpose and Environment	2105

A.8.2.1.2.2	Test Requirements	2110

A.8.2.2	E-UTRA – NR Inter-RAT Early Measruement Reporting	2111

A.8.2.2.1	E-UTRA – NR Early Measurement Reporting for NR in FR1	2111

A.8.2.2.1.1	Test Purpose and Environment	2111

A.8.2.2.1.2	Test Requirements	2116

A.8.2.2.2	E-UTRA – NR Early Measurement Reporting for NR in FR2	2116

A.8.2.2.2.1	Test Purpose and Environment	2116

A.8.2.2.2.2	Test Requirements	2119

A.8.3	RRC_CONNECTED state mobility	2120

A.8.3.1	Handover	2120

A.8.3.1.1	E-UTRAN - NR handover in FR1	2120

A.8.3.1.1.1	Test Purpose and Environment	2120

A.8.3.1.1.2	Test Requirements	2125

A.8.4	Measurement procedure	2125

A.8.4.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay	2125

A.8.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay in non-DRX	2125

A.8.4.1.1.1	Test Purpose and Environment	2125

A.8.4.1.1.2	Test Requirements	2127

A.8.4.1.2	E-UTRA – NR Inter-RAT SFTD Measurement Delay in DRX	2128

A.8.4.1.2.1	Test Purpose and Environment	2128

A.8.4.1.2.2	Test Requirements	2129

A.8.4.2	E-UTRA – NR Inter-RAT Measurements	2129

A.8.4.2.1	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2129

A.8.4.2.1.1	Test Purpose and Environment	2129

A.8.4.2.1.2	Test Requirements	2134

A.8.4.2.2	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2134

A.8.4.2.2.1	Test Purpose and Environment	2134

A.8.4.2.2.2	Test Requirements	2138

A.8.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2138

A.8.4.2.3.1	Test Purpose and Environment	2138

A.8.4.2.3.2	Test Requirements	2143

A.8.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2143

A.8.4.2.4.1	Test Purpose and Environment	2143

A.8.4.2.4.2	Test Requirements	2147

A.8.4.2.5	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is not used	2147

A.8.4.2.5.1	Test Purpose and Environment	2147

A.8.4.2.5.2	Test Requirements	2149

A.8.4.2.6	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is used	2150

A.8.4.2.6.1	Test Purpose and Environment	2150

A.8.4.2.6.2	Test Requirements	2152

A.8.4.2.7	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is not used	2153

A.8.4.2.7.1	Test Purpose and Environment	2153

A.8.4.2.7.2	Test Requirements	2155

A.8.4.2.8	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is used	2156

A.8.4.2.8.1	Test Purpose and Environment	2156

A.8.4.2.8.2	Test Requirements	2158

A.8.4.2.9	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16	2159

A.8.4.2.9.1	Test Purpose and Environment	2159

A.8.4.2.9.2	Test Requirements	2165

A.8.5	Measurement performance	2165

A.8.5.1	SFTD accuracy	2165

A.8.5.1.1	SFTD accuracy	2165

A.8.5.1.1.1	Test Purpose	2165

A.8.5.1.1.2	Test Environment	2165

A.8.5.1.1.3	Test Requirements	2171

A.8.5.2	E-UTRA – NR Inter-RAT Measurement Performance requirements	2171

A.8.5.2.1.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	2171

A.8.5.2.1.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	2176

A.8.5.2.1.2.1	Test Purpose and Environment	2176

A.8.5.2.1.2.2	Test Parameters	2176

A.8.5.2.1.2.3	Test Requirements	2178

A.8.5.2.2	SS-RSRQ	2178

A.8.5.2.2.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	2178

A.8.5.2.2.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	2183

A.8.5.2.2.2.1	Test Purpose and Environment	2183

A.8.5.2.2.2.2	Test Parameters	2183

A.8.5.2.2.2.3	Test Requirements	2185

A.8.5.2.3	SS-SINR	2185

A.8.5.2.3.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	2185

A.8.5.2.3.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	2189

A.8.5.2.3.2.1	Test Purpose and Environment	2189

A.8.5.2.3.2.2	Test Parameters	2189

A.8.5.2.3.2.3	Test Requirements	2191

A.9	V2X Tests	2192

A.9.1	V2X Tests in FR1	2192

A.9.1.1	Test for V2X UE Transmit Timing	2192

A.9.1.1.1	Test for GNSS as Synchronization Reference Source	2192

A.9.1.1.1.1	Test Purpose and Environment	2192

A.9.1.1.1.2	Test requirements	2192

A.9.1.1.2	Test for SyncRef UE as Synchronization Reference Source	2192

A.9.1.1.2.1	Test Purpose and Environment	2192

A.9.1.1.2.2	Test requirements	2193

A.9.1.1.3	Test for FR1 NR Cell as Synchronization Reference Source	2193

A.9.1.1.3.1	Test Purpose and Environment	2193

A.9.1.1.3.2	Test requirements	2196

A.9.1.2	Test for Initiation/Cease of S-SSB Transmission with V2X Sidelink Communication	2196

A.9.1.2.1	Test for FR1 NR Cell as synchronization reference source without gap under non-DRX	2196

A.9.1.2.1.1	Test Purpose and Environment	2196

A.9.1.2.1.2	Test Requirements	2200

A.9.1.2.2	Test for SyncRef UE as synchronization reference source	2200

A.9.1.2.2.1	Test Purpose and Environment	2200

A.9.1.2.2.2	Test Requirements	2201

A.9.1.3	Test for V2X Synchronization Reference Selection/Reselection	2202

A.9.1.3.1	Test for GNSS configured as the highest priority	2202

A.9.1.3.1.1	Test Purpose and Environment	2202

A.9.1.3.1.2	Test Requirements	2204

A.9.1.3.2	 Test for FR1 NR Cell configured as the highest priority	2206

A.9.1.3.2.1	Test Purpose and Environment	2206

A.9.1.3.2.2	Test Requirements	2208

A.9.1.4	Test for L1 SL-RSRP Measurement	2209

A.9.1.4.1	Test for V2X UE Autonomous Resource Selection/Reselection	2209

A.9.1.4.1.1	Test Purpose and Environment	2209

A.9.1.4.1.2	Test Requirements	2212

A.9.1.4.2	Test for V2X UE Resource Pre-emption	2213

A.9.1.4.2.1	Test Purpose and Environment	2213

A.9.1.4.2.2	Test Requirements	2216

A.9.1.4.3	Test for V2X UE Resource Re-evaluation	2216

A.9.1.4.3.1	Test Purpose and Environment	2216

A.9.1.4.3.2	Test Requirements	2223

A.9.1.5	Test for Congestion Control Measurement	2223

A.9.1.5.1	Test Purpose and Environment	2223

A.9.1.5.2	Test Requirements	2229

A.9.1.6	Test for Interruption	2229

A.9.1.6.1	Test for Interruption to WAN due to V2X Sidelink Communication	2229

A.9.1.6.1.1	Test Purpose and Environment	2229

A.9.1.6.1.2	Test Requirements	2232

A.10	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	2232

A.10.1	RRC_CONNECTED state mobility	2232

A.10.1.1	RRC connection mobility control	2232

A.10.1.1.1	Random Access	2232

A.10.1.1.1.1	4-step RA type contention-based random access for NR PSCell with CCA	2232

A.10.1.1.1.1.1	Test Purpose and Environment	2232

A.10.1.1.1.1.2	Test Requirements	2235

A.10.1.1.1.1.2.1	Random Access Preamble Transmission	2235

A.10.1.1.1.1.2.2	Random Access Response Reception	2235

A.10.1.1.1.1.2.3	No Random Access Response Reception	2236

A.10.1.1.1.1.2.4	Receiving an UL grant for msg3 retransmission	2236

A.10.1.1.1.1.2.5	 Contention Resolution Timer expiry	2236

A.10.1.1.1.2	4-step RA type non-contention based random access for NR PSCell with CCA	2237

A.10.1.1.1.2.1	Test Purpose and Environment	2237

A.10.1.1.1.2.2	Test Requirements	2240

A.10.1.1.1.2.2.1	SSB-based Random Access Preamble Transmission	2240

A.10.1.1.1.2.2.2	Random Access Response Reception	2241

A.10.1.1.1.2.2.3	No Random Access Response Reception	2241

A.10.1.1.1.3	2-step RA type contention-based random access for NR PSCell with CCA	2241

A.10.1.1.1.3.1	Test Purpose and Environment	2241

A.10.1.1.1.3.2	Test Requirements	2245

A.10.1.1.1.3.2.1	MsgA Transmission	2245

A.10.1.1.1.3.2.2	MsgB Reception	2245

A.10.1.1.1.3.2.3	No MsgB Reception	2246

A.10.1.1.1.4	2-step RA type non-contention based random access for NR PSCell with CCA	2246

A.10.1.1.1.4.1	Test Purpose and Environment	2246

A.10.1.1.1.4.2	Test Requirements	2250

A.10.1.1.1.4.2.1	MsgA Transmission	2250

A.10.1.1.1.4.2.2	MsgB Reception	2251

A.10.1.1.1.4.2.3	No MsgB Reception	2251

A.10.2	Timing	2252

A.10.2.1	UE transmit timing	2252

A.10.2.2	UE timing advance	2256

A.10.3	Signalling characteristics	2259

A.10.3.1	Radio link monitoring	2259

A.10.3.1.1	Introduction	2259

A.10.3.1.2	Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	2260

A.10.3.1.2.1	Test purpose and environment	2260

A.10.3.1.2.2	Test requirements	2265

A.10.3.1.3	Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	2265

A.10.3.1.3.1	Test purpose and environment	2265

A.10.3.1.3.2	Test requirements	2271

A.10.3.1.4	Void	2271

A.10.3.1.4.1	Void	2271

A.10.3.1.4.2	Void	2271

A.10.3.1.5	Void	2271

A.10.3.1.5.1	Void	2271

A.10.3.1.5.2	Void	2271

A.10.3.2	Interruption	2271

A.10.3.2.1	E-UTRAN – NR interruptions during SCell operations with CCA	2271

A.10.3.2.1.1	Test Purpose and Environment	2271

A.10.3.2.1.2	Test Requirements	2276

A.10.3.3	SCell activation and deactivation delay	2276

A.10.3.3.2 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 640 ms SCell measurement cycle	2281

A.10.3.3.2.1	Test Purpose and Environment	2281

A.10.3.3.2.2	Test Requirements	2282

A.10.3.3.3 SCell Activation and Deactivation of unknown NR SCell with NR PSCell and NR SCell under CCA	2282

A.10.3.3.3.1	Test Purpose and Environment	2282

A.10.3.3.3.2	Test Requirements	2283

A.10.3.4	Beam failure detection and link recovery procedures	2283

A.10.3.4.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	2283

A.10.3.4.1.1	Test Purpose and Environment	2283

A.10.3.4.1.2	Test Requirements	2288

A.10.3.4.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	2288

A.10.3.4.2.1	Test Purpose and Environment	2288

A.10.3.4.2.2	Test Requirements	2294

A.10.3.5	Active BWP switching	2294

A.10.3.5.1	UL active BWP switch delay with consistent UL LBT failure on PSCell subject to UL CCA in EN-DC	2294

A.10.3.5.1.1	Test Purpose and Environment	2294

A.10.3.5.1.2	Test Requirements	2299

A.10.3.5.2	DCI-based and Timer-based Active BWP Switch	2300

A.10.3.5.2.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	2300

A.10.3.5.2.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	2303

A.10.3.5.3	RRC-based Active BWP Switch	2307

A.10.3.6	PSCell addition and release delay	2311

A.10.3.6.1	Addition and Release Delay of known NR PSCell on the carrier under CCA	2311

A.10.3.6.1.1	Test purpose and environment	2311

A.10.3.6.1.2	Test Requirements	2316

A.10.3.7	Void	2317

A.10.4	Measurement procedure	2317

A.10.4.1	Intra-frequency measurements	2317

A.10.4.1.1	Event-triggered reporting tests on PSCC without gaps under non-DRX	2317

A.10.4.1.1.1	Test purpose and environment	2317

A.10.4.1.1.2	Test parameters	2317

A.10.4.1.1.3	Test Requirements	2321

A.10.4.1.2	Void	2321

A.10.4.1.3	Void	2321

A.10.4.1.4	Event-triggered reporting tests on PSCC with per-UE gaps under DRX	2321

A.10.4.1.4.1	Test purpose and environment	2321

A.10.4.1.4.2	Test parameters	2322

A.10.4.1.4.3	Test Requirements	2326

A.10.4.1.5	Void	2327

A.10.4.1.6	Void	2327

A.10.4.1.7	Void	2327

A.10.4.1.8	Void	2327

A.10.4.1.9	RSSI measurement reporting on PSCC	2327

A.10.4.1.9.1	Test purpose and environment	2327

A.10.4.1.9.2	Test parameters	2327

A.10.4.1.10	Channel occupancy measurement reporting on PSCC	2327

A.10.4.1.10.1	Test purpose and environment	2327

A.10.4.1.10.2	Test parameters	2327

A.10.4.1.11	RSSI measurement reporting on SCC	2328

A.10.4.1.11.1	Test purpose and environment	2328

A.10.4.1.11.2	Test parameters	2328

A.10.4.1.12	Channel occupancy measurement reporting on SCC	2328

A.10.4.1.12.1	Test purpose and environment	2328

A.10.4.1.12.2	Test parameters	2328

A.10.4.2	Inter-frequency measurements	2329

A.10.4.2.1	RSSI measurement reporting	2329

A.10.4.2.1.1	Test purpose and environment	2329

A.10.4.2.1.2	Test parameters	2329

A.10.4.2.2	Channel occupancy measurement reporting	2329

A.10.4.2.2.1	Test purpose and environment	2329

A.10.4.2.2.2	Test parameters	2329

A.10.4.2.3	EN-DC event triggered reporting tests for FR1 with CCA cell without SSB time index detection when DRX is not used	2330

A.10.4.2.3.1	Test Purpose and Environment	2330

A.10.4.2.3.2	Test Requirements	2334

A.10.4.2.4	EN-DC event triggered reporting tests for FR1 cell with CCA without SSB time index detection when DRX is used	2335

A.10.4.2.4.1	Test Purpose and Environment	2335

A.10.4.2.4.2	Test Requirements	2339

A.10.4.2.5	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is not used	2340

A.10.4.2.5.1	Test Purpose and Environment	2340

A.10.4.2.5.2	Test Requirements	2344

A.10.4.2.6	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is used	2345

A.10.4.2.6.1	Test Purpose and Environment	2345

A.10.4.2.6.2	Test Requirements	2349

A.10.4.2.7	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	2350

A.10.4.2.7.1	Test Purpose and Environment	2350

A.10.4.2.7.2	Test Requirements	2356

A.10.4.2.8	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	2356

A.10.4.2.8.1	Test Purpose and Environment	2356

A.10.4.2.8.2	Test Requirements	2362

A.10.4.2.9	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	2363

A.10.4.2.9.1	Test Purpose and Environment	2363

A.10.4.2.9.2	Test Requirements	2368

A.10.4.2.10	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	2368

A.10.4.2.10.1	Test Purpose and Environment	2368

A.10.4.2.10.2	Test Requirements	2374

A.10.4.3	L1-RSRP measurements for beam reporting	2375

A.10.4.3.1	SSB based L1-RSRP measurement on PSCC when DRX is not used	2375

A.10.4.3.1.1	Test Purpose and Environment	2375

A.10.4.3.1.2	Test parameters	2375

A.10.4.3.1.3	Test Requirements	2377

A.10.4.3.2	SSB based L1-RSRP measurement on PSCC when DRX is used	2378

A.10.4.3.2.1	Test Purpose and Environment	2378

A.10.4.3.2.2	Test parameters	2378

A.10.4.3.2.3	Test Requirements	2380

A.10.4.3.3	SSB based L1-RSRP measurement on SCC when DRX is not used	2381

A.10.4.3.3.1	Test Purpose and Environment	2381

A.10.4.3.3.2	Test parameters	2381

A.10.4.3.3.3	Test Requirements	2384

A.10.4.3.4	SSB based L1-RSRP measurement on SCC when DRX is used	2385

A.10.4.3.4.1	Test Purpose and Environment	2385

A.10.4.3.4.2	Test parameters	2385

A.10.4.3.4.3	Test Requirements	2388

A.10.4.4	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	2389

A.10.4.4.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2389

A.10.4.4.1.1	Test Purpose and Environment	2389

A.10.4.4.1.2	Test Requirements	2393

A.10.4.4.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2394

A.10.4.4.2.1	Test Purpose and Environment	2394

A.10.4.4.2.2	Test Requirements	2399

A.10.4.4.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2400

A.10.4.4.3.1	Test Purpose and Environment	2400

A.10.4.4.3.2	Test Requirements	2404

A.10.4.4.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2405

A.10.4.4.4.1	Test Purpose and Environment	2405

A.10.4.4.4.2	Test Requirements	2410

A.10.5	Measurement performance	2411

A.10.5.1	SS-RSRP	2411

A.10.5.1.1	Intra-frequency measurement accuracy on a CCA serving cell	2411

A.10.5.1.1.1	Test Purpose and Environment	2411

A.10.5.1.1.2	Test parameters	2411

A.10.5.1.1.3	Test Requirements	2414

A.10.5.1.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	2414

A.10.5.1.2.1	Test Purpose and Environment	2414

A.10.5.1.2.2	Test parameters	2414

A.10.5.1.2.3	Test Requirements	2418

A.10.5.2	SS-RSRQ	2418

A.10.5.2.1	Intra-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	2418

A.10.5.2.1.1	Test Purpose and Environment	2418

A.10.5.2.1.2	Test Parameters	2418

A.10.5.2.1.3	Test Requirements	2421

A.10.5.2.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	2421

A.10.5.2.2.1	Test Purpose and Environment	2421

A.10.5.2.2.2	Test Parameters	2421

A.10.5.2.2.3	Test Requirements	2424

A.10.5.3	SS-SINR	2424

A.10.5.3.1	Intra-frequency measurement accuracy on PSCC	2424

A.10.5.3.1.1	Test Purpose and Environment	2424

A.10.5.3.1.2	Test Parameters	2424

A.10.5.3.1.3	Test Requirements	2427

A.10.5.3.2	Inter-frequency measurement accuracy on PSCC	2427

A.10.5.3.2.1	Test Purpose and Environment	2427

A.10.5.3.2.2	Test Parameters	2427

A.10.5.3.2.3	Test Requirements	2431

A.10.5.3.3	Intra-frequency measurement accuracy on SCC	2431

A.10.5.3.3.1	Test Purpose and Environment	2431

A.10.5.3.3.2	Test Parameters	2431

A.10.5.3.3.3	Test Requirements	2434

A.10.5.4	L1-RSRP measurement for beam reporting with CCA serving cell	2434

A.10.5.4.1	SSB based L1-RSRP measurement	2434

A.10.5.4.1.1	Test Purpose and Environment	2434

A.10.5.4.1.2	Test parameters	2435

A.10.5.4.1.3	Test Requirements	2437

A.10.5.5	RSSI	2437

A.10.5.5.1 	RSSI measurement accuracy on PSCC with CCA	2437

A.10.5.5.1.1	Test Purpose and Environment	2437

A.10.5.5.1.2	Test parameters	2437

A.10.5.5.1.3	Test Requirements	2440

A.10.5.5.2 	RSSI measurement accuracy on SCC with CCA	2440

A.10.5.5.2.1	Test Purpose and Environment	2440

A.10.5.5.2.2	Test parameters	2440

A.10.5.5.2.3	Test Requirements	2443

A.10.5.5.3 	Inter-frequency RSSI measurement accuracy on a carrier with CCA	2443

A.10.5.5.3.1	Test Purpose and Environment	2443

A.10.5.5.3.2	Test parameters	2443

A.10.5.5.3.3	Test Requirements	2447

A.10.5.6	Channel occupancy	2447

A.10.5.6.1 	Channel occupancy measurement accuracy on PSCC with CCA	2447

A.10.5.6.1.1	Test Purpose and Environment	2447

A.10.5.6.1.2	Test parameters	2447

A.10.5.6.1.3	Test Requirements	2451

A.10.5.6.2 	Channel occupancy measurement accuracy on SCC with CCA	2451

A.10.5.6.2.1	Test Purpose and Environment	2451

A.10.5.6.2.2	Test parameters	2451

A.10.5.6.2.3	Test Requirements	2454

A.10.5.6.3 	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	2454

A.10.5.6.3.1	Test Purpose and Environment	2454

A.10.5.6.3.2	Test parameters	2454

A.10.5.6.3.3	Test Requirements	2458

A.11	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	2458

A.11.1	RRC_IDLE state mobility	2458

A.11.1.1	Cell re-selection with both source and target NR carrier frequencies under CCA	2458

A.11.1.1.1	Cell reselection to FR1 intra-frequency NR cells when subject to CCA on the serving and target cell	2458

A.11.1.1.1.1	Test Purpose and Environment	2458

A.11.1.1.1.2	Test Parameters	2459

A.11.1.1.1.3	Test Requirements	2462

A.11.1.1.2	Cell reselection to FR1 inter-frequency NR case when subject to CCA on the serving and target cell	2462

A.11.1.1.2.1	Test Purpose and Environment	2462

A.11.1.1.2.2	Test Parameters	2462

A.11.1.1.2.3	Test Requirements	2466

A.11.1.2	Cell re-selection to NR with source NR carrier frequency under CCA	2466

A.11.1.2.1	Cell reselection to FR1 inter-frequency NR case when serving cell is subject to CCA	2466

A.11.1.2.1.1	Test Purpose and Environment	2466

A.11.1.2.1.2	Test Parameters	2467

A.11.1.2.1.3	Test Requirements	2473

A.11.1.3	Cell re-selection from NR carrier with target NR carrier frequency under CCA	2474

A.11.1.3.1	Cell reselection to FR1 inter-frequency NR case when target cell is subject to CCA	2474

A.11.1.3.1.1	Test Purpose and Environment	2474

A.11.1.3.1.2	Test Parameters	2474

A.11.1.3.1.3	Test Requirements	2479

A.11.1.4	Inter-RAT cell re-selection to E-UTRAN with source NR carrier frequency under CCA	2480

A.11.1.4.1	Cell reselection to higher priority E-UTRAN when serving cell is subject to CCA	2480

A.11.1.4.1.1	Test Purpose and Environment	2480

A.11.1.4.1.2	Test Parameters	2480

A.11.1.4.1.3	Test Requirements	2483

A.11.1.4.2	Cell reselection to lower priority E-UTRAN when serving cell is subject to CCA	2484

A.11.1.4.2.1	Test Purpose and Environment	2484

A.11.1.4.2.2	Test Requirements	2486

A.11.2	RRC_CONNECTED state mobility	2487

A.11.2.1	Handover	2487

A.11.2.1.1	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; known target cell	2487

A.11.2.1.1.1	Test Purpose and Environment	2487

A.11.2.1.1.2	Test Parameters	2487

A.11.2.1.1.3 Test Requirements	2490

A.11.2.1.2	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	2490

A.11.2.1.2.1	Test Purpose and Environment	2490

A.11.2.1.2.2	Test Parameters	2490

A.11.2.1.2.3	Test Requirements	2493

A.11.2.1.3	Inter-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	2493

A.11.2.1.3.1	Test Purpose and Environment	2493

A.11.2.1.3.2	Test Parameters	2493

A.11.2.1.3.3 Test Requirements	2496

A.11.2.1.4	Inter-frequency handover from FR1 carrier under CCA to FR1; known target cell	2496

A.11.2.1.4.1	Test Purpose and Environment	2496

A.11.2.1.4.2	Test Parameters	2496

A.11.2.1.4.3 Test Requirements	2500

A.11.2.1.5	Inter-frequency handover from FR1 carrier under CCA to FR1; unknown target cell	2501

A.11.2.1.5.1	Test Purpose and Environment	2501

A.11.2.1.5.2	Test Parameters	2501

A.11.2.1.5.3 Test Requirements	2504

A.11.2.1.6	Inter-frequency handover from FR1 to FR1 carrier under CCA; unknown target cell	2505

A.11.2.1.6.1	Test Purpose and Environment	2505

A.11.2.1.6.2	Test Parameters	2505

A.11.2.1.6.3 Test Requirements	2508

A.11.2.1.7	 SA NR FR1 carrier under CCA - E-UTRAN handover with known target cell	2509

A.11.2.1.7.1	Test Purpose and Environment	2509

A.11.2.1.7.2	Test Requirements	2515

A.11.2.1.8	SA NR FR1 carrier under CCA - E-UTRAN handover with unknown target cell	2515

A.11.2.1.8.1	Test Purpose and Environment	2515

A.11.2.1.8.2	Test Requirements	2520

A.11.2.2	RRC connection mobility control	2520

A.11.2.2.1	RRC re-establishment	2520

A.11.2.2.1.1	Intra-frequency RRC Re-establishment with CCA in FR1	2520

A.11.2.2.1.2	Inter-frequency RRC Re-establishment with CCA in FR1	2523

A.11.2.2.1.3	Intra-frequency RRC Re-establishment with CCA in FR1 without serving cell timing	2527

A.11.2.2.1.4	Inter-frequency RRC Re-establishment from NR FR1 carrier without CCA to NR FR1 carrier under CCA	2530

A.11.2.2.2	Random Access	2535

A.11.2.2.2.1	4-step RA type contention-based random access for NR PCell with CCA	2535

A.11.2.2.2.1.1	Test Purpose and Environment	2535

A.11.2.2.2.1.2	Test Requirements	2538

A.11.2.2.2.1.2.1	Random Access Preamble Transmission	2538

A.11.2.2.2.1.2.2	Random Access Response Reception	2538

A.11.2.2.2.1.2.3	No Random Access Response Reception	2539

A.11.2.2.2.1.2.4	Receiving an UL grant for msg3 retransmission	2539

A.11.2.2.2.1.2.5	Reception of an Incorrect Message over Temporary C-RNTI	2539

A.11.2.2.2.1.2.6	Reception of a Correct Message over Temporary C-RNTI	2540

A.11.2.2.2.1.2.7	Contention Resolution Timer expiry	2540

A.11.2.2.2.2	4-step RA type non-contention based random access for NR PSCell with CCA	2540

A.11.2.2.2.2.1	Test Purpose and Environment	2540

A.11.2.2.2.2.2	Test Requirements	2543

A.11.2.2.2.2.2.1	SSB-based Random Access Preamble Transmission	2543

A.11.2.2.2.2.2.2	Random Access Response Reception	2544

A.11.2.2.2.2.2.3	No Random Access Response Reception	2544

A.11.2.2.2.3	2-step RA type contention-based random access for NR PCell with CCA	2544

A.11.2.2.2.3.1	Test Purpose and Environment	2544

A.11.2.2.2.3.2	Test Requirements	2548

A.11.2.2.2.3.2.1	MsgA Transmission	2548

A.11.2.2.2.3.2.2	MsgB Reception	2548

A.11.2.2.2.3.2.3	No MsgB Reception	2549

A.11.2.2.2.4	2-step RA type non-contention-based random access for NR PCell with CCA	2549

A.11.2.2.2.4.1	Test Purpose and Environment	2549

A.11.2.2.2.4.2	Test Requirements	2553

A.11.2.2.2.4.2.1	MsgA Transmission	2553

A.11.2.2.2.4.2.2	MsgB Reception	2554

A.11.2.2.2.4.2.3	No MsgB Reception	2554

A.11.2.2.3	RRC connection release with redirection	2555

A.11.2.2.3.1	Redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA	2555

A.11.3	Timing	2563

A.11.3.1	UE transmit timing	2563

A.11.3.2	UE timing advance	2567

A.11.3.2.1	UE Timing Advance Adjustment Accuracy with PCell under DL CCA	2567

A.11.3.2.1.1	Test Purpose and Environment	2567

A.11.3.2.1.2	Test Parameters	2567

A.11.3.2.1.3	Test Requirements	2571

A.11.4	Signalling characteristics	2571

A.11.4.1	Radio link monitoring	2571

A.11.4.1.1	Introduction	2571

A.11.4.1.2	Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	2572

A.11.4.1.2.1	Test purpose and environment	2572

A.11.4.1.2.2	Test requirements	2576

A.11.4.1.3	Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	2576

A.11.4.1.3.1	Test purpose and environment	2576

A.11.4.1.3.2	Test requirements	2582

A.11.4.1.4	Void	2582

A.11.4.1.4.1	Void	2582

A.11.4.1.4.2	Void	2582

A.11.4.1.5	Void	2582

A.11.4.1.5.1	Void	2582

A.11.4.1.5.2	Void	2582

A.11.4.2	Interruption	2582

A.11.4.2.1	NR interruptions during Scell operations with CCA on PCell and SCell	2582

A.11.4.2.1.1 Test Purpose and Environment	2582

A.11.4.2.1.2	Test Requirements	2586

A.11.4.3	SCell activation and deactivation delay	2586

A.11.4.3.2	SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 640 ms SCell measurement cycle	2591

A.11.4.3.2.1	Test Purpose and Environment	2591

A.11.4.3.2.2	Test Requirements	2592

A.11.4.4	Beam failure detection and link recovery procedures	2593

A.11.4.4.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	2593

A.11.4.4.1.1	Test Purpose and Environment	2593

A.11.4.4.1.2	Test Requirements	2599

A.11.4.4.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	2599

A.11.4.4.2.1	Test Purpose and Environment	2599

A.11.4.4.2.2	Test Requirements	2605

A.11.4.5	Active BWP switching	2605

A.11.4.5.1	UL active BWP switch delay with consistent UL LBT failure on PCell subject to UL CCA	2605

A.11.4.5.1.1	Test Purpose and Environment	2605

A.11.4.5.1.2	Test Requirements	2609

A.11.4.6	Void	2619

A.11.5	Measurement procedure	2619

A.11.5.1	Intra-frequency measurements	2619

A.11.5.1.1	Event-triggered reporting tests on PCC without gaps under non-DRX	2619

A.11.5.1.1.1	Test purpose and environment	2619

A.11.5.1.1.2	Test parameters	2619

A.11.5.1.1.3	Test Requirements	2623

A.11.5.1.2	Event-triggered reporting tests on PCC without gaps under DRX	2623

A.11.5.1.2.1	Test purpose and environment	2623

A.11.5.1.2.2	Test parameters	2623

A.11.5.1.2.3	Test Requirements	2627

A.11.5.1.3	Event-triggered reporting tests on PCC with per-UE gaps under non-DRX	2627

A.11.5.1.3.1	Test purpose and environment	2627

A.11.5.1.3.2	Test parameters	2627

A.11.5.1.3.3	Test Requirements	2627

A.11.5.1.4	Event-triggered reporting tests on PCC with per-UE gaps under DRX	2627

A.11.5.1.4.1	Test purpose and environment	2627

A.11.5.1.4.2	Test parameters	2628

A.11.5.1.4.3	Test Requirements	2628

A.11.5.1.5	Event-triggered reporting tests on SCC without gaps under non-DRX	2628

A.11.5.1.5.1	Test purpose and environment	2628

A.11.5.1.5.2	Test parameters	2628

A.11.5.1.6	Event-triggered reporting tests on SCC without gaps under DRX	2628

A.11.5.1.6.1	Test purpose and environment	2628

A.11.5.1.6.2	Test parameters	2628

A.11.5.1.6.3	Test Requirements	2628

A.11.5.1.7	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	2629

A.11.5.1.7.1	Test purpose and environment	2629

A.11.5.1.7.2	Test parameters	2629

A.11.5.1.7.3	Test Requirements	2629

A.11.5.1.8	Event-triggered reporting tests on SCC with per-UE gaps under DRX	2629

A.11.5.1.8.1	Test purpose and environment	2629

A.11.5.1.8.2	Test parameters	2629

A.11.5.1.8.3	Test Requirements	2629

A.11.5.1.9	RSSI measurement reporting on PCC	2630

A.11.5.1.9.1	Test purpose and environment	2630

A.11.5.1.9.2	Test parameters	2630

A.11.5.1.10	Channel occupancy measurement reporting on PCC	2630

A.11.5.1.10.1	Test purpose and environment	2630

A.11.5.1.10.2	Test parameters	2630

A.11.5.1.11	RSSI measurement reporting on SCC	2631

A.11.5.1.11.1	Test purpose and environment	2631

A.11.5.1.11.2	Test parameters	2631

A.11.5.1.12	Channel occupancy measurement reporting on SCC	2631

A.11.5.1.12.1	Test purpose and environment	2631

A.11.5.1.12.2	Test parameters	2631

A.11.5.2	Inter-frequency measurements	2632

A.11.5.2.1	RSSI measurement reporting	2632

A.11.5.2.1.1	Test purpose and environment	2632

A.11.5.2.1.2	Test parameters	2632

A.11.5.2.2	Channel occupancy measurement reporting	2632

A.11.5.2.2.1	Test purpose and environment	2632

A.11.5.2.2.2	Test parameters	2632

A.11.5.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	2633

A.11.5.2.3.1	Test Purpose and Environment	2633

A.11.5.2.3.2	Test Requirements	2637

A.11.5.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	2638

A.11.5.2.4.1	Test Purpose and Environment	2638

A.11.5.2.4.2	Test Requirements	2643

A.11.5.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	2643

A.11.5.2.5.1	Test Purpose and Environment	2643

A.11.5.2.5.2	Test Requirements	2647

A.11.5.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	2648

A.11.5.2.6.1	Test Purpose and Environment	2648

A.11.5.2.6.2	Test Requirements	2653

A.11.5.2.7	Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2653

A.11.5.2.7.1	Test Purpose and Environment	2653

A.11.5.2.7.2	Test Requirements	2657

A.11.5.2.8	Event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2658

A.11.5.2.8.1	Test Purpose and Environment	2658

A.11.5.2.8.2	Test Requirements	2663

A.11.5.2.9	Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2663

A.11.5.2.9.1	Test Purpose and Environment	2663

A.11.5.2.9.2	Test Requirements	2668

A.11.5.2.10	Event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2669

A.11.5.2.10.1	Test Purpose and Environment	2669

A.11.5.2.10.2	Test Requirements	2674

A.11.5.3	Inter-RAT E-UTRAN measurements	2675

A.11.5.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1	2675

A.11.5.3.1.1	Test Purpose and Environment	2675

A.11.5.3.1.2	Test Requirements	2679

A.11.5.3.2	SA NR - E-UTRAN event-triggered reporting in DRX in FR1	2679

A.11.5.3.2.1	Test Purpose and Environment	2679

A.11.5.3.2.2	Test Requirements	2684

A.11.5.4	L1-RSRP measurements for beam reporting	2684

A.11.5.4.1	SSB based L1-RSRP measurement when DRX is not used	2684

A.11.5.4.1.1	Test Purpose and Environment	2684

A.11.5.4.1.2	Test parameters	2684

A.11.5.4.1.3	Test Requirements	2686

A.11.5.4.2	SSB based L1-RSRP measurement when DRX is used	2686

A.11.5.4.2.1	Test Purpose and Environment	2686

A.11.5.4.2.2	Test parameters	2687

A.11.5.4.2.3	Test Requirements	2689

A.11.5.4.3	SSB based L1-RSRP measurement on SCC when DRX is not used	2689

A.11.5.4.3.1	Test Purpose and Environment	2689

A.11.5.4.3.2	Test parameters	2690

A.11.5.4.3.3	Test Requirements	2693

A.11.5.4.4	SSB based L1-RSRP measurement on SCC when DRX is used	2694

A.11.5.4.4.1	Test Purpose and Environment	2694

A.11.5.4.4.2	Test parameters	2694

A.11.5.4.4.3	Test Requirements	2697

A.11.6	Measurement performance	2698

A.11.6.1	SS-RSRP	2698

A.11.6.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	2698

A.11.6.1.1.1	Test Purpose and Environment	2698

A.11.6.1.1.2	Test parameters	2698

A.11.6.1.1.3	Test Requirements	2700

A.11.6.1.2	Intra-frequency measurement accuracy on SCC on a carrier frequency with CCA	2700

A.11.6.1.2.1	Test Purpose and Environment	2700

A.11.6.1.2.2	Test parameters	2700

A.11.6.1.2.3	Test Requirements	2702

A.11.6.2	SS-RSRQ	2702

A.11.6.2.1	Intra-frequency measurement accuracy	2702

A.11.6.2.1.1	Test Purpose and Environment	2702

A.11.6.2.1.2	Test Parameters	2702

A.11.6.2.1.3	Test Requirements	2705

A.11.6.2.2	Inter-frequency measurement accuracy	2705

A.11.6.2.2.1	Test Purpose and Environment	2705

A.11.6.2.2.2	Test Parameters	2705

A.11.6.2.2.3	Test Requirements	2708

A.11.6.2.3	Intra-frequency measurement accuracy on SCC	2708

A.11.6.2.3.1	Test Purpose and Environment	2708

A.11.6.2.3.2	Test Parameters	2708

A.11.6.2.3.3	Test Requirements	2711

A.11.6.2.4	Inter-frequency measurement accuracy	2711

A.11.6.2.4.1	Test Purpose and Environment	2711

A.11.6.2.4.2	Test Parameters	2711

A.11.6.2.4.3	Test Requirements	2720

A.11.6.3	SS-SINR	2720

A.11.6.3.1	Intra-frequency measurement accuracy	2720

A.11.6.3.1.1	Test Purpose and Environment	2720

A.11.6.3.1.2	Test Parameters	2720

A.11.6.3.1.3	Test Requirements	2723

A.11.6.3.2	Inter-frequency measurement accuracy	2723

A.11.6.3.2.1	Test Purpose and Environment	2723

A.11.6.3.2.2	Test Parameters	2723

A.11.6.3.2.3	Test Requirements	2726

A.11.6.3.3	Intra-frequency measurement accuracy on SCC	2726

A.11.6.3.3.1	Test Purpose and Environment	2726

A.11.6.3.3.2	Test Parameters	2726

A.11.6.3.3.3	Test Requirements	2729

A.11.6.3.4	Inter-frequency measurement accuracy	2729

A.11.6.3.4.1	Test Purpose and Environment	2729

A.11.6.3.4.2	Test Parameters	2729

A.11.6.3.4.3	Test Requirements	2737

A.11.6.4	L1-RSRP measurement for beam reporting with CCA serving cell	2737

A.11.6.4.1	SSB based L1-RSRP measurement	2737

A.11.6.4.1.1	Test Purpose and Environment	2737

A.11.6.4.1.2	Test parameters	2738

A.11.6.4.1.3	Test Requirements	2741

A.11.6.5	RSSI	2741

A.11.6.5.1 	Intra-frequency RSSI measurement accuracy on PCC with CCA	2741

A.11.6.5.1.1	Test Purpose and Environment	2741

A.11.6.5.1.2	Test parameters	2741

A.11.6.5.1.3	Test Requirements	2744

A.11.6.5.2 	Intra-frequency RSSI measurement accuracy on SCC with CCA	2744

A.11.6.5.2.1	Test Purpose and Environment	2744

A.11.6.5.2.2	Test parameters	2744

A.11.6.5.2.3	Test Requirements	2747

A.11.6.5.3 	Inter-frequency RSSI measurement accuracy on a carrier with CCA	2747

A.11.6.5.3.1	Test Purpose and Environment	2747

A.11.6.5.3.2	Test parameters	2747

A.11.6.5.3.3	Test Requirements	2750

A.11.6.6	Channel occupancy	2750

A.11.6.6.1 	Intra-frequency channel occupancy measurement accuracy on PCC with CCA	2750

A.11.6.6.1.1	Test Purpose and Environment	2750

A.11.6.6.1.2	Test parameters	2750

A.11.6.6.1.3	Test Requirements	2754

A.11.6.6.2 	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	2754

A.11.6.6.2.1	Test Purpose and Environment	2754

A.11.6.6.2.2	Test parameters	2754

A.11.6.6.2.3	Test Requirements	2757

A.11.6.6.3 	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	2757

A.11.6.6.3.1	Test Purpose and Environment	2757

A.11.6.6.3.2	Test parameters	2757

A.11.6.6.3.3	Test Requirements	2760

A.11.6.7	E-UTRAN RSRP	2760

A.11.6.8	E-UTRAN RSRQ	2760

A.11.6.9	E-UTRAN SINR	2760

A.12	E-UTRA Standalone Tests with at Least One NR Cell under CCA	2760

A.12.1	RRC_IDLE state mobility	2760

A.12.1.1	Inter-RAT cell re-selection to NR on a carrier frequency with CCA	2760

A.12.1.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1 when target cell is subject to CCA	2760

A.12.1.1.1.1	Test Purpose and Environment	2760

A.12.1.1.1.2	Test Requirements	2766

A.12.2	RRC_CONNECTED state mobility	2767

A.12.2.1	Handover	2767

A.12.2.1.1	E-UTRAN - NR with CCA handover	2767

A.12.2.1.1.1	Test Purpose and Environment	2767

A.12.2.1.1.2	Test Requirements	2772

A.12.3	Signalling characteristics	2773

A.12.3.1	Interruptions	2773

A.12.4	Measurement procedure	2773

A.12.4.1	E-UTRANNR inter-RAT SFTD measurements	2773

A.12.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay with NR under CCA in non-DRX	2773

A.12.4.1.1.1	Test Purpose and Environment	2773

A.12.4.1.1.2	Test Requirements	2777

A.12.4.2	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	2777

A.12.4.2.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2777

A.12.4.2.1.1	Test Purpose and Environment	2777

A.12.4.2.1.2	Test Requirements	2781

A.12.4.2.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2782

A.12.4.2.2.1	Test Purpose and Environment	2782

A.12.4.2.2.2	Test Requirements	2786

A.12.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2787

A.12.4.2.3.1	Test Purpose and Environment	2787

A.12.4.2.3.2	Test Requirements	2791

A.12.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2792

A.12.4.2.4.1	Test Purpose and Environment	2792

A.12.4.2.4.2	Test Requirements	2796

A.12.4.2.5	RSSI measurement reporting	2797

A.12.4.2.5.1	Test purpose and environment	2797

A.12.4.2.5.2	Test parameters	2797

A.12.4.2.5.3	Test Requirements	2800

A.12.4.2.6	Channel occupancy measurement reporting	2800

A.12.4.2.6.1	Test purpose and environment	2800

A.12.4.2.6.2	Test parameters	2800

A.12.4.2.6.3	Test Requirements	2804

A.12.5	Measurement performance	2804

A.12.5.1	E-UTRANNR SFTD	2804

A.12.5.2	E-UTRANNR SS-RSRP	2810

A.12.5.3	E-UTRANNR SS-RSRQ	2810

A.12.5.4	E-UTRANNR SS-SINR	2810

A.12.5.5	E-UTRANNR RSSI	2810

A.12.5.6	E-UTRANNR channel occupancy	2810

A.13	 NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	2810

A.13.1	Timing	2810

A.13.1.1	UE transmit timing	2810

A.13.1.2	Timing advance	2810

A.13.2	Signalling characteristics	2810

A.13.2.1	Interruption	2810

A.13.2.1.1	NR interruptions during SCell operations with CCA on SCell	2810

A.13.2.1.1.1	Test Purpose and Environment	2810

A.13.2.1.1.2	Test Requirements	2814

A.13.2.2	SCell activation and deactivation delay	2814

A.13.2.2.2	SCell Activation and Deactivation of known SCell under CCA, 640 ms SCell measurement cycle	2819

A.13.2.2.2.1	Test Purpose and Environment	2819

A.13.2.2.2.2	Test Requirements	2820

A.13.2.2.3	SCell Activation and Deactivation of unknown SCell under CCA	2820

A.13.2.2.3.1	Test Purpose and Environment	2820

A.13.2.2.3.2	Test Requirements	2821

A.13.2.3	Void	2821

A.13.3	Measurement procedure	2821

A.13.3.1	Intra-frequency measurements	2821

A.13.3.1.1	Event-triggered reporting tests on SCC without gaps under non-DRX	2821

A.13.3.1.1.1	Test purpose and environment	2821

A.13.3.1.1.2	Test parameters	2821

A.13.3.1.1.3	Test Requirements	2826

A.13.3.1.2	Event-triggered reporting tests on SCC without gaps under DRX	2826

A.13.3.1.2.1	Test purpose and environment	2826

A.13.3.1.2.2	Test parameters	2826

A.13.3.1.2.3	Test Requirements	2831

A.13.3.1.3	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	2831

A.13.3.1.3.1	Test purpose and environment	2831

A.13.3.1.3.2	Test parameters	2831

A.13.3.1.3.3	Test Requirements	2836

A.13.3.1.4	Event-triggered reporting tests on SCC with per-UE gaps under DRX	2836

A.13.3.1.4.1	Test purpose and environment	2836

A.13.3.1.4.2	Test parameters	2836

A.13.3.1.4.3	Test Requirements	2841

A.13.3.1.5	RSSI measurement reporting on SCC	2841

A.13.3.1.5.1	Test purpose and environment	2841

A.13.3.1.5.2	Test parameters	2841

A.13.3.1.6	Channel occupancy measurement reporting on SCC	2843

A.13.3.1.6.1	Test purpose and environment	2843

A.13.3.1.6.2	Test parameters	2844

A.13.3.2	Inter-frequency measurements	2844

A.13.3.2.1	RSSI measurement reporting	2844

A.13.3.2.1.1	Test purpose and environment	2844

A.13.3.2.1.2	Test parameters	2844

A.13.3.2.2	Channel occupancy measurement reporting	2845

A.13.3.2.2.1	Test purpose and environment	2845

A.13.3.2.2.2	Test parameters	2845

A.13.3.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	2845

A.13.3.2.3.1	Test Purpose and Environment	2845

A.13.3.2.3.2	Test Requirements	2851

A.13.3.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	2851

A.13.3.2.4.1	Test Purpose and Environment	2851

A.13.3.2.4.2	Test Requirements	2856

A.13.3.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	2857

A.13.3.2.5.1	Test Purpose and Environment	2857

A.13.3.2.5.2	Test Requirements	2862

A.13.3.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	2862

A.13.3.2.6.1	Test Purpose and Environment	2862

A.13.3.2.6.2	Test Requirements	2868

A.13.3.3	L1-RSRP measurements for beam reporting	2869

A.13.3.3.1	SSB based L1-RSRP measurement when DRX is not used	2869

A.13.3.3.1.1	Test Purpose and Environment	2869

A.13.3.3.1.2	Test parameters	2869

A.13.3.3.1.3	Test Requirements	2874

A.13.3.3.2	SSB based L1-RSRP measurement when DRX is used	2874

A.13.3.3.2.1	Test Purpose and Environment	2874

A.13.3.3.2.2	Test parameters	2874

A.13.3.3.2.3	Test Requirements	2878

A.13.4	Measurement performance	2878

A.13.4.1	SS-RSRP	2878

A.13.4.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	2878

A.13.4.1.1.1	Test Purpose and Environment	2878

A.13.4.1.1.2	Test parameters	2878

A.13.4.1.1.3	Test Requirements	2880

A.13.4.2	SS-RSRQ	2880

A.13.4.2.1	Intra-frequency measurement accuracy on SCC	2880

A.13.4.2.1.1	Test Purpose and Environment	2880

A.13.4.2.1.2	Test Parameters	2880

A.13.4.2.1.3	Test Requirements	2889

A.13.4.3	SS-SINR	2889

A.13.4.3.1	Intra-frequency measurement accuracy on SCC	2889

A.13.4.3.1.1	Test Purpose and Environment	2889

A.13.4.3.1.2	Test Parameters	2889

A.13.4.3.1.3	Test Requirements	2895

A.13.4.4	L1-RSRP measurement for beam reporting with CCA serving cell	2896

A.13.4.4.1	SSB based L1-RSRP measurement	2896

A.13.4.4.1.1	Test Purpose and Environment	2896

A.13.4.4.1.2	Test parameters	2896

A.13.4.4.1.3	Test Requirements	2900

A.13.4.5	RSSI	2900

A.13.4.5.1 	Intra-frequency RSSI measurement accuracy on a carrier with CCA	2900

A.13.4.5.1.1	Test Purpose and Environment	2900

A.13.4.5.1.2	Test parameters	2900

A.13.4.5.1.3	Test Requirements	2904

A.13.4.5.2	  Inter-frequency RSSI measurement accuracy on a carrier with CCA	2904

A.13.4.5.2.1	Test Purpose and Environment	2904

A.13.4.5.2.2	Test parameters	2904

A.13.4.5.2.3	Test Requirements	2908

A.13.4.6	Channel occupancy	2908

A.13.4.6.1 	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	2908

A.13.4.6.1.1	Test Purpose and Environment	2908

A.13.4.6.1.2	Test parameters	2908

A.13.4.6.1.3	Test Requirements	2912

A.13.4.6.2	  Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	2912

A.13.4.6.2.1	Test Purpose and Environment	2912

A.13.4.6.2.2	Test parameters	2912

A.13.4.6.2.3	Test Requirements	2916

Annex B (normative): Conditions for RRM requirements applicability for operating bands	2914

B.1	Conditions for NR RRC_IDLE state mobility	2914

B.1.1	Introduction	2914

B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection	2914

B.1.3	Conditions for measurements on NR inter-frequency cells for cell re-selection	2915

B.2	Conditions for UE measurements procedures and performance requirements in RRC_CONNECTED state	2916

B.2.1	Introduction	2916

B.2.1.1	General	2916

B.2.1.2	Derivation of Minimum SSB_RP values for FR1	2916

B.2.1.3	Derivation of Minimum SSB_RP values for FR2	2916

B.2.1.3.1	Minimum SSB_RP values for Rx Beam Peak angle of arrival	2916

B.2.1.3.2	Minimum SSB_RP values for angle of arrival within Spherical coverage	2917

B.2.1.4	Gain to SS-RSRP and CSI-RSRP measurement point for FR1	2917

B.2.1.5	Gain to SS-RSRP and CSI-RSRP measurement point for FR2	2918

B.2.1.5.1	Gain to SS-RSRP and CSI-RSRP measurement point for Rx Beam Peak angle of arrival	2918

B.2.1.5.2	Gain to SS-RSRP measurement point for different frequency	2918

B.2.1.5.3	Alignment of Rough beam to Rx beam Peak	2918

B.2.1.6	Gain to PRS-RSRP measurement point for FR2	2919

B.2.1.6.1	Gain to PRS-RSRP measurement point for Rx Beam Peak angle of arrival	2919

B.2.2	Conditions for NR intra-frequency measurements	2920

B.2.3	Conditions for NR inter-frequency measurements	2921

B.2.4	Conditions for NR L1-RSRP reporting	2922

B.2.4.1	Conditions for SSB based L1-RSRP reporting	2922

B.2.4.2	Conditions for CSI-RS based L1-RSRP reporting	2923

B.2.5	Conditions for RRC connection release with redirection to NR	2925

B.2.6	Void	2926

B.2.6.1	Void	2926

B.2.6.2	Void	2926

B.2.7	Conditions for SRS-RSRP measurements	2926

B.2.8	Conditions for NR L1-SINR reporting	2927

B.2.8.1	Conditions for L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	2927

B.2.8.2	Conditions for L1-SINR reporting with SSB based CMR and dedicated IMR configured	2928

B.2.8.2.1	L1-SINR reporting with SSB based CMR and dedicated ZP-IMR configured	2928

B.2.8.2.2	L1-SINR reporting with SSB based CMR and dedicated NZP-IMR configured	2928

B.2.8.3	Conditions for L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	2930

B.2.8.3.1	L1-SINR reporting with CSI-RS based CMR and dedicated ZP-IMR configured	2930

B.2.8.3.2	L1-SINR reporting with CSI-RS based CMR and dedicated NZP-IMR configured	2931

B.2.9	Conditions for NR intra-frequency measurements under CCA	2932

B.2.10	Conditions for NR inter-frequency measurements under CCA	2932

B.2.11	Conditions for NR L1-RSRP reporting under CCA	2932

B.2.11.1	Conditions for SSB based L1-RSRP reporting	2932

B.2.12	Conditions for NR CSI-RS based intra-frequency measurements	2933

B.2.13	Conditions for NR CSI-RS based inter-frequency measurements	2934

B.2.14	Conditions for NR PRS-based measurements	2936

B.3	RRM Requirements Exceptions	2937

B.3.1	Introduction	2937

B.3.2	Receiver sensitivity relaxation for CA	2937

B.3.2.1	Receiver sensitivity relaxation for UE supporting CA in FR1	2937

B.3.2.2	Receiver sensitivity relaxation for UE configured with CA in FR1	2938

B.3.2.2.1	Inter-band carrier aggregation	2938

B.3.2.2.2	Reference sensitivity exceptions due to UL harmonic interference for CA	2938

B.3.2.2.3	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA	2938

B.3.2.3	Receiver sensitivity relaxation for UE supporting CA in FR2	2938

B.3.2.4	Receiver sensitivity relaxation for UE configured with CA in FR2	2938

B.3.2.4.1	Intra-band contiguous carrier aggregation	2938

B.3.2.4.2	Intra-band non-contiguous carrier aggregation	2939

B.3.3	Receiver sensitivity relaxation for DC	2939

B.3.3.1	Receiver sensitivity relaxation for EN-DC	2939

B.3.3.2	Receiver sensitivity relaxation for NE-DC	2939

B.3.4	Receiver sensitivity relaxation for SUL	2939

B.3.4.1	Receiver sensitivity relaxation for UE supporting SUL in FR1	2939

B.3.4.2	Receiver sensitivity relaxation for UE configured with SUL in FR1	2939

B.3.4.2.1	Reference sensitivity exceptions due to UL harmonic interference for SUL	2939

B.4	Conditions for V2X	2939

B.4.1	Test parameters for GNSS signals	2939

B.4.2	Conditions for PSBCH-RSRP Accuracy Requirements	2940

B.4.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	2940

B.4.4	Conditions for L1 SL-RSRP Accuracy Requirements	2940

Annex C (informative): Change history	2942