| 3GPP TS 38.133 V17.13.0 (2024-03) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Requirements for support of radio resource management (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	119

1	Scope	121

2	References	121

3	Definitions, symbols and abbreviations	122

3.1	Definitions	122

3.2	Symbols	123

3.3	Abbreviations	124

3.4	Test tolerances	127

3.5	Frequency bands grouping	127

3.5.1	Introduction	127

3.5.2	NR operating bands in FR1	127

3.5.2A	NR operating bands for satellite access in FR1	128

3.5.3	NR operating bands in FR2	128

3.6	Applicability of requirements in this specification version	129

3.6.1	RRC connected state requirements in DRX	130

3.6.2	Number of serving carriers	130

3.6.2.1	Number of serving carriers for SA	130

3.6.2.2	Number of serving carriers for EN-DC	130

3.6.2.3	Number of serving carriers for NE-DC	130

3.6.2.4	Number of serving carriers for NR-DC	131

3.6.3	Applicability for intra-band FR2	131

3.6.4	Applicability for FR2 UE power classes	131

3.6.5	Applicability for SDL bands	131

3.6.6	Applicability of requirements for NGEN-DC operation	131

3.6.7	Applicability of QCL	131

3.6.9	Applicability of requirements for scheduling availability	132

3.6.10	Applicability of requirements for measurement restrictions	132

3.6.11	Applicability of requirements for Redcap UEs	132

3.6.11.1	RRC connected state requirements in DRX	132

3.6.11.2	Applicability for FR2 Redcap UE power classes	132

3.6.11.3	Applicability of QCL	132

3.6.12	Applicability of requirements for Satellite Access	133

3.6.13	Applicability of requirements for FR2	133

3.6.14	Applicability of requirements for FR2 Power Class 6	133

3.6.15	Applicability of requirements for per-FR gap	133

4	SA: RRC_IDLE state mobility	133

4.1	Cell Selection	133

4.2	Cell Re-selection	134

4.2.1	Introduction	134

4.2.2	Requirements	134

4.2.2.1	UE measurement capability	134

4.2.2.2	Measurement and evaluation of serving cell	134

4.2.2.3	Measurements of intra-frequency NR cells	137

4.2.2.4	Measurements of inter-frequency NR cells	140

4.2.2.5	Measurements of inter-RAT E-UTRAN cells	145

4.2.2.6	Maximum interruption in paging reception	148

4.2.2.7	General requirements	149

4.2.2.8	Minimum requirement at transitions	149

4.2.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	149

4.2.2.9.1	Introduction	149

4.2.2.9.2	Measurements for UE fulfilling low mobility criterion	150

4.2.2.9.3	Measurements for UE fulfilling not-at-cell edge criterion	152

4.2.2.9.4	Measurements for UE fulfilling low mobility and not-at-cell edge criteria	154

4.2.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	155

4.2.2.10.1	Introduction	155

4.2.2.10.2	Measurements for UE fulfilling low mobility criterion	155

4.2.2.10.3	Measurements for UE fulfilling not-at-cell edge criterion	158

4.2.2.10.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	161

4.2.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	161

4.2.2.11.1	Introduction	161

4.2.2.11.2	Measurements for UE fulfilling low mobility criterion	162

4.2.2.11.3	Measurements for UE fulfilling with not-at-cell edge criterion	163

4.2.2.11.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	165

4.2A	Cell Re-selection when subject to CCA	166

4.2A.1	Introduction	166

4.2A.2	Requirements	166

4.2A.2.1	UE measurement capability	166

4.2A.2.2	Measurement and evaluation when subject to CCA on the serving cell	167

4.2A.2.3	Measurements of intra-frequency NR cells when subject to CCA on the serving cell and target cell	167

4.2A.2.4	Measurements of inter-frequency NR cells when subject to CCA on the target cell	169

4.2A.2.5	Measurements of inter-RAT E-UTRAN cells when subject to CCA on the serving cell	171

4.2A.2.6	Maximum interruption in paging reception when subject to CCA on the target cell	171

4.2A.2.7	General requirements	171

4.2B	Cell Re-selection for RedCap	172

4.2B.1	Introduction	172

4.2B.2	Requirements	172

4.2B.2.1	UE measurement capability for RedCap	172

4.2B.2.1.1	UE measurement capability for 1 Rx RedCap	172

4.2B.2.1.2	UE measurement capability for 2 Rx RedCap	172

4.2B.2.2	Measurement and evaluation of serving cell for RedCap UE	172

4.2B.2.3	Measurements of intra-frequency NR cells for RedCap UE	174

4.2B.2.4	Measurements of inter-frequency NR cells for RedCap UE	177

4.2B.2.5	Measurements of inter-RAT E-UTRAN cells for RedCap UE	180

4.2B.2.6	Maximum interruption in paging reception for RedCap	182

4.2B.2.7	General requirements for RedCap	182

4.2B.2.8	Minimum requirement at transitions	182

4.2B.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion for RedCap	183

4.2B.2.9.1	Introduction	183

4.2B.2.9.2	Measurements for UE fulfilling stationary criterion	184

4.2B.2.9.3	Measurements for a UE fulfilling not-at-cell edge while stationary criterion	187

4.2B.2.9.3A	Measurements for a UE fulfilling stationary and not-at-cell-edge criteria	187

4.2B.2.9.4	Measurements for a UE fulfilling low mobility and stationary criteria	187

4.2B.2.9.5	Measurements for a UE fulfilling low mobility and not-at-cell-edge while stationary criteria	188

4.2B.2.9.6	Measurements for a UE fulfilling not-at-cell edge and not-at-cell edge while stationary criteria	188

4.2B.2.9.7	Measurements for a UE fulfilling low mobility and not-at-cell edge criteria and not-at-cell-edge while stationary criteria	188

4.2B.2.9.8	Measurements for a UE fulfilling low mobility, not-at-cell edge and stationary criterion	188

4.2B.2.9.9	Measurements for UE fulfilling low mobility criterion	189

4.2B.2.9.10	Measurements for UE fulfilling not-at-cell edge criterion	192

4.2B.2.9.11	Measurements for UE fulfilling low mobility and not-at-cell edge criteria	194

4.2B.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	194

4.2B.2.10.1	Introduction	194

4.2B.2.10.2	Measurements for UE fulfilling stationary criterion	195

4.2B.2.10.3	Measurements for a UE fulfilling not-at-cell edge while stationary criterion	197

4.2B.2.10.3A	Measurements for a UE fulfilling stationary and not-at-cell-edge-criterion	198

4.2B.2.10.4	Measurements for a UE fulfilling low mobility and stationary criteria	198

4.2B.2.10.5	Measurements for a UE fulfilling low mobility and not-at-cell-edge while stationary criteria	199

4.2B.2.10.6	Measurements for a UE fulfilling not-at-cell edge and not-at-cell edge while stationary criteria	199

4.2B.2.10.7	Measurements for a UE fulfilling low mobility and not-at-cell edge criteria and not-at-cell-edge while stationary criteria	199

4.2B.2.10.8	Measurements for a UE fulfilling low mobility, not-at-cell edge and stationary criteria	199

4.2B.2.10.9	Measurements for UE fulfilling low mobility criterion	200

4.2B.2.10.10	Measurements for UE fulfilling not-at-cell edge criterion	202

4.2B.2.10.11	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	205

4.2B.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	205

4.2B.2.11.1	Introduction	205

4.2B.2.11.2	Measurements for UE fulfilling stationary criterion	206

4.2B.2.11.3	Measurements for a UE not-at-cell edge while stationary criterion	207

4.2B.2.11.3A	Measurements for a UE fulfilling stationary and not-at-cell-edge criterion	207

4.2B.2.11.4	Measurements for a UE fulfilling low mobility and stationary criteria	208

4.2B.2.11.5	Measurements for a UE fulfilling low mobility and stationary not-at-cell-edge while stationary criteria	208

4.2B.2.11.6	Measurements for a UE fulfilling not-at-cell edge and not-at-cell edge while stationary criteria	208

4.2B.2.11.7	Measurements for a UE fulfilling low mobility and not-at-cell edge criteria and not-at-cell-edge while stationary criteria	209

4.2B.2.11.8	Measurements for a UE fulfilling low mobility, not-at-cell edge and stationary criteria	209

4.2B.2.11.9	Measurements for UE fulfilling low mobility criterion	209

4.2B.2.11.10	Measurements for UE fulfilling with not-at-cell edge criterion	211

4.2B.2.11.11	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	212

4.2C	Cell Re-selection for NR UE for Satellite Access	212

4.2C.1	Introduction	212

4.2C.2	Requirements	212

4.2C.2.1	UE measurement capability	212

4.2C.2.2	Measurement and evaluation of serving cell	213

4.2C.2.3	Measurements of intra-frequency NR cells	213

4.2C.2.4	Measurements of inter-frequency NR cells	216

4.2C.2.5	Maximum interruption in paging reception	219

4.2C.2.6	Minimum requirement at transitions	219

4.2C.2.7	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	219

4.2C.2.8	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	220

4.2C.2.9	General requirements	220

4.3	Minimization of Drive Tests (MDT)	220

4.3.1	Introduction	220

4.3.2	Measurement Requirements	220

4.3.3	Requirements for Relative Time Stamp Accuracy	221

4.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	221

4.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	221

4.3C	Minimization of Drive Tests (MDT) for Satellite Access	221

4.3C.1	Introduction	221

4.3C.2	Measurement Requirements	222

4.3C.3	Requirements for Relative Time Stamp Accuracy	222

4.3C.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	222

4.3C.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	223

4.4	Idle Mode CA/DC Measurements	223

4.4.1	Introduction	223

4.4.2	Measurement Requirements	223

4.4.2.1	Detected cell requirement during state transition and Idle mode	223

4.4.2.2	Measurements of inter-frequency CA/DC candidate cells	224

4.4.2.3	Measurements on serving cell	225

4.4.2.4	Measurements of E-UTRAN inter-RAT DC candidate cells	225

5	SA: RRC_INACTIVE state mobility	227

5.1	Cell Re-selection	227

5.1.1	Introduction	227

5.1.2	Requirements	227

5.1.2.1	UE measurement capability	227

5.1.2.2	Measurement and evaluation of serving cell	227

5.1.2.3	Measurements of intra-frequency NR cells	228

5.1.2.4	Measurements of inter-frequency NR cells	229

5.1.2.5	Measurements of inter-RAT E-UTRAN cells	230

5.1.2.6	Maximum interruption in paging reception	231

5.1.2.7	General requirements	231

5.1A	Cell Re-selection with CCA	231

5.1A.1	Introduction	231

5.1A.2	Requirements	231

5.1A.2.1	UE measurement capability	231

5.1A.2.2	Measurement and evaluation when CCA is used on the serving cell	231

5.1A.2.3	Measurements of intra-frequency NR cells when CCA is used on the serving cell and target cell	231

5.1A.2.4	Measurements of inter-frequency NR cells when CCA is used on the target cell	231

5.1A.2.5	Measurements of inter-RAT E-UTRAN cells when CCA is used on the serving cell	231

5.1A.2.6	Maximum interruption in paging reception when CCA is used on the target cell	231

5.1A.2.7	General requirements	232

5.1B	Cell Re-selection for RedCap	232

5.1B.1	Introduction	232

5.1B.2	Requirements	232

5.1B.2.1	UE measurement capability	232

5.1B.2.2	Measurement and evaluation of serving cell	232

5.1B.2.3	Measurements of intra-frequency NR cells	233

5.1B.2.4	Measurements of inter-frequency NR cells	234

5.1B.2.5	Measurements of inter-RAT E-UTRAN cells	235

5.1B.2.6	Maximum interruption in paging reception	235

5.1B.2.7	General requirements	235

5.1B.2.8	Minimum requirement at transitions	235

5.1B.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	236

5.1B.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	237

5.1B.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	237

5.1C	Cell Re-selection	238

5.1C.1	Introduction	238

5.1C.2	Requirements	238

5.1C.2.1	UE measurement capability	238

5.1C.2.2	Measurement and evaluation of serving cell	238

5.1C.2.3	Measurements of intra-frequency NR cells	239

5.1C.2.4	Measurements of inter-frequency NR cells	239

5.1C.2.5	Maximum interruption in paging reception	239

5.1C.2.6	General requirements	239

5.2	Void	239

5.2B	Configured Grant based Small Data Transmissions (CG-SDT) for RedCap	239

5.2B.1	Introduction	239

5.2B.2	Requirements on UE synchronization for small data transmissions for RedCap	239

5.2B.2.1	Void	239

5.2B.3	TA validation requirements for RedCap	239

5.2B.2.2	Void	240

5.2B.2.3	Void	241

5.2B.4	Scheduling restriction	241

5.2B.5	Applicability conditions for CG-SDT for RedCap	241

5.3	Minimization of Drive Tests (MDT)	241

5.3.1	Introduction	241

5.3.2	Measurement Requirements	241

5.3.3	Requirements for Relative Time Stamp Accuracy	241

5.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	242

5.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	242

5.3.6	Requirements for Relative Time Stamp Accuracy for RRC Resume Failure Log Reporting	242

5.3C	Minimization of Drive Tests (MDT) for Satellite Access	242

5.3C.1	Introduction	242

5.3C.2	Measurement Requirements	242

5.3C.3	Requirements for Relative Time Stamp Accuracy	242

5.3C.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	243

5.3C.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	243

5.3C.6	Requirements for Relative Time Stamp Accuracy for RRC Resume Failure Log Reporting	243

5.4	Inactive Mode CA/DC Measurements	243

5.4.1	Introduction	243

5.4.2	Measurement Requirements	243

5.4.2.1	Detected cell requirement during state transition and inactive mode	243

5.4.2.2	Measurements of inter-frequency CA/DC candidate cells	243

5.4.2.3	Measurements on serving cell	243

5.4.2.4	Measurements on E-UTRAN inter-RAT DC candidate cells	243

5.5	Configured Grant based Small Data Transmissions (CG-SDT)	244

5.5.1	Introduction	244

5.5.2	Requirements on UE synchronization for small data transmissions	244

5.5.3	TA validation requirements	244

5.5.4	Scheduling restriction	245

5.5.4.1	Scheduling availability of UE performing measurements in TDD bands on FR1	245

5.5.4.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	245

5.5.4.3	Scheduling availability of UE performing measurements on FR2	245

5.5.5	Applicability conditions for SDT	246

5.6	NR measurements for positioning	246

5.6.1	Introduction	246

5.6.2	RSTD measurements	247

5.6.2.1	Introduction	247

5.6.2.2	Requirements Applicability	247

5.6.2.3	Measurement Capability	247

5.6.2.5	Measurements Period Requirements	248

5.6.3	PRS-RSRP measurements	250

5.6.3.1	Introduction	250

5.6.3.2	Requirements applicability	250

5.6.3.3	Measurement Capability	250

5.6.3.4	Measurement Reporting Requirements	250

5.6.3.5	Measurement Period Requirements	251

5.6.4	UE Rx-Tx time difference measurements	253

5.6.4.1	Introduction	253

5.6.4.2	Requirements Applicability	253

5.6.4.3	Measurement Capability	253

5.6.4.4	Measurement Reporting Requirements	254

5.6.4.5	Measurement Period Requirements	254

5.6.5	PRS-RSRPP measurements	257

5.6.5.1	Introduction	257

5.6.5.2	Requirements Applicability	257

5.6.5.3	Measurement Capability	257

5.6.5.4	Measurement Reporting Requirements	257

5.6.5.5	Measurement Period Requirements	257

5.7	Random access based Small Data Transmissions (RA-SDT)	259

5.7.1	Introduction	259

5.7.2	Requirements for small data transmissions based on 2-step RA	259

5.7.3	Requirements for small data transmissions based on 4-step RA	259

5.7.4	Applicability conditions for SDT	259

5.7B	Random access based Small Data Transmissions (RA-SDT) for RedCap	259

5.7B.1	Introduction	259

5.7B.2	Requirements for small data transmissions based on 2-step RA	259

5.7B.3	Requirements for small data transmissions based on 4-step RA	259

5.7B.4	Applicability conditions for RA-SDT for RedCap	259

6	RRC_CONNECTED state mobility	260

6.1	Handover	260

6.1.1	NR Handover	260

6.1.1.1	Introduction	260

6.1.1.2	NR FR1 - NR FR1 Handover	260

6.1.1.2.1	Handover delay	260

6.1.1.2.2	Interruption time	260

6.1.1.3	NR FR2- NR FR1 Handover	261

6.1.1.3.1	Handover delay	261

6.1.1.3.2	Interruption time	261

6.1.1.4	NR FR2- NR FR2 Handover	262

6.1.1.4.1	Handover delay	262

6.1.1.4.2	Interruption time	262

6.1.1.5	NR FR1- NR FR2 Handover	263

6.1.1.5.1	Handover delay	263

6.1.1.5.2	Interruption time	263

6.1.2	NR Handover to other RATs	264

6.1.2.1	NR – E-UTRAN Handover	264

6.1.2.1.1	Introduction	264

6.1.2.1.2	Handover delay	264

6.1.2.1.3	Interruption time	264

6.1.2.2	NR – UTRAN Handover	265

6.1.2.2.1	Introduction	265

6.1.2.2.2	Handover delay	265

6.1.2.2.3	Interruption time	265

6.1.3	NR DAPS Handover	266

6.1.3.1	Introduction	266

6.1.3.2	NR FR1 - NR FR1 DAPS Handover	266

6.1.3.2.1	DAPS handover delay	266

6.1.3.2.2	Interruption time	267

6.1.3.3	NR FR2- NR FR1 DAPS Handover	269

6.1.3.3.1	DAPS handover delay	269

6.1.3.3.2	Interruption time	269

6.1.3.4	NR FR1- NR FR2 DAPS Handover	270

6.1.3.4.1	DAPS handover delay	270

6.1.3.4.2	Interruption time	270

6.1.4	NR Conditional Handover	271

6.1.4.1	Introduction	271

6.1.4.2	NR FR1 – NR FR1 conditional handover	271

6.1.4.3	NR FR2 – NR FR1 conditional handover	272

6.1.4.4	NR FR2 – NR FR2 conditional handover	273

6.1.4.4.1	Handover delay	273

6.1.4.4.2	Measurement time	273

6.1.4.4.3	Preparation time	273

6.1.4.4.4	Interruption time	273

6.1.4.5	NR FR1 – NR FR2 conditional handover	274

6.1.5	NR Handover with PSCell	274

6.1.5.1	Introduction	274

6.1.5.2	Handover with PSCell from NR SA to EN-DC	274

6.1.5.2.1	Interruption time for inter-RAT HO from NR to E-UTRAN	275

6.1.5.2.2	PSCell addition in HO with PSCell for NR SA to EN-DC	275

6.1.5.3	HO with PSCell from NE-DC to NE-DC	275

6.1.5.3.1	Handover delay	276

6.1.5.3.2	HO with PSCell - PCell Interruption time	276

6.1.5.3.3	PSCell addition/change in NE-DC to NE-DC HO with PSCell	276

6.1.5.4	HO with PSCell from NR-DC to NR-DC	276

6.1.5.5	Handover with PSCell from NR SA to EN-DC with PSCell using CCA	278

6.1.5.5.1	Introduction	278

6.1.5.5.2	NR SA to EN-DC HO with PSCell- NR to E-UTRA HO Interruption time	278

6.1.5.5.3	NR SA to EN-DC HO with PSCell - NR PSCell Addition Delay requirements	278

6.1A	Void	279

6.1A.1	Void	279

6.1A.1.1	Void	279

6.1A.1.2	Void	279

6.1A.1.2.1	Void	279

6.1A.1.2.2	Void	279

6.1B	Handover to target cell using CCA	280

6.1B.1	NR Handover	280

6.1B.1.1	Introduction	280

6.1B.1.2	NR FR1 - NR FR1 Handover	280

6.1B.1.2.1	Handover delay	280

6.1B.1.2.2	Interruption time	280

6.1B.1.3	NR FR2-2 NR FR2-2 Handover	281

6.1B.1.3.1	Handover delay	281

6.1B.1.3.2	Interruption time	281

6.1B.1.4	NR FR1- NR FR2-2 Handover	282

6.1B.1.4.1	Handover delay	282

6.1B.1.4.2	Interruption time	282

6.1C	Handover for SAN	283

6.1C.1	NR SAN Handover	283

6.1C.1.1	Introduction	283

6.1C.1.2	NR SAN FR1 – NR SAN FR1 Handover	283

6.1C.1.2.1	Handover delay	284

6.1C.1.2.2	Interruption time	284

6.1C.2	NR SAN Conditional Handover	285

6.1C.2.1	Introduction	285

6.1C.2.2	NR SAN FR1 – NR SAN FR1 conditional handover	285

6.1C.2.2.1	Handover delay	285

6.1C.2.2.2	Measurement time	285

6.1C.2.2.3	Preparation time	286

6.1C.2.2.4	Interruption time	286

6.1D	Handover for RedCap	287

6.1D.1	NR Handover	287

6.1D.1.1	Introduction	287

6.1D.1.2	NR FR1 - NR FR1 Handover	287

6.1D.1.2.1	Handover delay	287

6.1D.1.2.2	Interruption time	288

6.1D.1.3	NR FR2- NR FR2 Handover	289

6.1D.1.3.1	Handover delay	289

6.1D.1.3.2	Interruption time	289

6.1D.2	NR Handover to other RATs	291

6.1D.2.1	NR – E-UTRAN Handover	291

6.2	RRC Connection Mobility Control	291

6.2.1	SA: RRC Re-establishment	291

6.2.1.1	Introduction	291

6.2.1.2	Requirements	291

6.2.1.2.1	UE Re-establishment delay requirement	291

6.2.1A	RRC Re-establishment with CCA	293

6.2.1A.1	Introduction	293

6.2.1A.2	Requirements	293

6.2.1A.2.1	UE Re-establishment with CCA delay requirement	293

6.2.1B	SA: RRC Re-establishment for RedCap	295

6.2.1B.1	Introduction	295

6.2.1B.2	Requirements	295

6.2.2	Random access	296

6.2.2.1	Introduction	296

6.2.2.2	Requirements for 4-step RA type	296

6.2.2.2.1	Contention based random access	296

6.2.2.2.1.1	Correct behaviour when transmitting Random Access Preamble	296

6.2.2.2.1.2	Correct behaviour when receiving Random Access Response	297

6.2.2.2.1.3	Correct behaviour when not receiving Random Access Response	297

6.2.2.2.1.4	Correct behaviour when receiving an UL grant for msg3 retransmission	297

6.2.2.2.1.5	SA: Correct behaviour when receiving a message over Temporary C-RNTI	297

6.2.2.2.1.6	Correct behaviour when contention Resolution timer expires	297

6.2.2.2.2	Non-Contention based random access	297

6.2.2.2.2.1	Correct behaviour when transmitting Random Access Preamble	297

6.2.2.2.2.2	Correct behaviour when receiving Random Access Response	298

6.2.2.2.2.3	Correct behaviour when not receiving Random Access Response	298

6.2.2.2.3	UE behaviour when configured with supplementary UL	298

6.2.2.3	Requirements for 2-step RA type	298

6.2.2.3.1	Contention based random access	299

6.2.2.3.1.1	Correct behaviour when transmitting MsgA	299

6.2.2.3.1.2	Correct behaviour when receiving MsgB	299

6.2.2.3.1.3	Correct behaviour when not receiving MsgB	299

6.2.2.3.2	Non-Contention based random access	299

6.2.2.3.2.1	Correct behaviour when transmitting MsgA	299

6.2.2.3.2.2	Correct behaviour when receiving MsgB	300

6.2.2.3.2.3	Correct behaviour when not receiving MsgB	300

6.2.2.3.3	UE behaviour when configured with supplementary UL	300

6.2.2A	Random access when CCA is used on target frequency	300

6.2.2A.1	Introduction	300

6.2.2A.2	Requirements for 4-step RA type	300

6.2.2A.2.1	Contention based random access	301

6.2.2A.2.1.1	Correct behaviour when transmitting Random Access Preamble	301

6.2.2A.2.1.2	Correct behaviour when receiving Random Access Response	301

6.2.2A.2.1.3	Correct behaviour when not receiving Random Access Response	301

6.2.2A.2.1.4	Correct behaviour when receiving an UL grant for msg3 retransmission	301

6.2.2A.2.1.6	Correct behaviour when contention Resolution timer expires	302

6.2.2A.2.2	Non-Contention based random access	302

6.2.2A.2.2.1	Correct behaviour when transmitting Random Access Preamble	302

6.2.2A.2.2.2	Correct behaviour when receiving Random Access Response	302

6.2.2A.2.2.3	Correct behaviour when not receiving Random Access Response	303

6.2.2A.3	Requirements for 2-step RA type	303

6.2.2A.3.1	Contention based random access	303

6.2.2A.3.1.1	Correct behaviour when transmitting MsgA	303

6.2.2A.3.1.2	Correct behaviour when receiving MsgB	304

6.2.2A.3.1.3	Correct behaviour when not receiving MsgB	304

6.2.2A.3.2	Non-Contention based random access	304

6.2.2A.3.2.1	Correct behaviour when transmitting MsgA	304

6.2.2A.3.2.2	Correct behaviour when receiving MsgB	305

6.2.2A.3.2.3	Correct behaviour when not receiving MsgB	305

6.2.2B	Random access for RedCap	305

6.2.2B.1	Introduction	305

6.2.2B.2	Requirements	305

6.2.3	SA: RRC Connection Release with Redirection	306

6.2.3.1	Introduction	306

6.2.3.2	Requirements	306

6.2.3.2.1	RRC connection release with redirection to NR	306

6.2.3.2.2	RRC connection release with redirection to E-UTRAN	307

6.2.3.2.3	RRC connection release with redirection to NR carrier subject to CCA	307

6.2.3A	SA: RRC Connection Release with Redirection for RedCap	309

6.2.3A.1	Introduction	309

6.2.3A.2	Requirements	309

6.2.3A.2.1	RRC connection release with redirection to NR	309

6.2.3A.2.2	RRC connection release with redirection to E-UTRAN	309

6.2C	RRC Connection Mobility Control for Satellite Access	309

6.2C.1	SA: RRC Re-establishment for Satellite Access	309

6.2C.1.1	Introduction	309

6.2C.1.2	Requirements	310

6.2C.1.2.1	UE Re-establishment delay requirement	310

6.2C.2	Random access for satellite access	311

6.2C.2.1	Introduction	311

6.2C.2.2	Requirements for 4-step RA type	311

6.2C.2.2.1	Contention based random access	312

6.2C.2.2.1.1	Correct behaviour when transmitting Random Access Preamble	312

6.2C.2.2.1.2	Correct behaviour when receiving Random Access Response	312

6.2C.2.2.1.3	Correct behaviour when not receiving Random Access Response	312

6.2C.2.2.1.4	Correct behaviour when receiving an UL grant for msg3 retransmission	312

6.2C.2.2.1.5	SA: Correct behaviour when receiving a message over Temporary C-RNTI	312

6.2C.2.2.1.6	Correct behaviour when contention Resolution timer expires	312

6.2C.2.2.2	Non-Contention based random access	312

6.2C.2.2.2.1	Correct behaviour when transmitting Random Access Preamble	312

6.2C.2.2.2.2	Correct behaviour when receiving Random Access Response	313

6.2C.2.2.2.3	Correct behaviour when not receiving Random Access Response	313

6.2C.2.3	Requirements for 2-step RA type	313

6.2C.2.3.1	Contention based random access	314

6.2C.2.3.1.1	Correct behaviour when transmitting MsgA	314

6.2C.2.3.1.2	Correct behaviour when receiving MsgB	314

6.2C.2.3.1.3	Correct behaviour when not receiving MsgB	314

6.2C.2.3.2	Non-Contention based random access	314

6.2C.2.3.2.1	Correct behaviour when transmitting MsgA	314

6.2C.2.3.2.2	Correct behaviour when receiving MsgB	315

6.2C.2.3.2.3	Correct behaviour when not receiving MsgB	315

6.2C.3	SA: RRC Connection Release with Redirection for Satellite Access	315

6.2C.3.1	Introduction	315

6.2C.3.2	Requirements	315

6.2C.3.2.1	RRC connection release with redirection to NR	315

7	Timing	316

7.1	UE transmit timing	316

7.1.1	Introduction	316

7.1.2	Requirements	316

7.1.2.1	Gradual timing adjustment	318

7.1.2.2	Void	319

7.1.2.3	One shot large UL timing adjustment for FR2 Power Class 6 UE	319

7.1A	UE transmit timing for RedCap	319

7.1A.1	Introduction	319

7.1A.2	Requirements	320

7.1A.2.1	Gradual timing adjustment	321

7.1C	UE transmit timing for Satellite Access	321

7.1C.1	Introduction	321

7.1C.2	Requirements	321

7.1C.2.1	Gradual timing adjustment	322

7.2	UE timer accuracy	323

7.2.1	Introduction	323

7.2.2	Requirements	323

7.2A	UE timer accuracy for RedCap	323

7.2A.1	Introduction	323

7.2A.2	Requirements	323

7.2C	UE timer accuracy for satellite access	323

7.2C.1	Introduction	323

7.2C.2	Requirements	324

7.3	Timing advance	324

7.3.1	Introduction	324

7.3.2	Requirements	324

7.3.2.1	Timing Advance adjustment delay	324

7.3.2.2	Timing Advance adjustment accuracy	324

7.3A	Timing Advance for RedCap	324

7.3A.1	Introduction	324

7.3A.2	Requirements	325

7.3A.2.1	Timing Advance adjustment delay	325

7.3A.2.2	Timing Advance adjustment accuracy	325

7.3C	Timing advance for satellite access	325

7.3C.1	Introduction	325

7.3C.2	Requirements	325

7.3C.2.1	Timing Advance adjustment delay	325

7.3C.2.2	Timing Advance adjustment accuracy	325

7.4	Cell phase synchronization accuracy	326

7.4.1	Definition	326

7.4.2	Minimum requirements	326

7.5	Maximum Transmission Timing Difference	326

7.5.1	Introduction	326

7.5.2	Minimum Requirements for inter-band EN-DC	326

7.5.2.1	Minimum Requirements for inter-band synchronous EN-DC	327

7.5.3	Minimum Requirements for intra-band EN-DC	327

7.5.4	Minimum Requirements for NR Carrier Aggregation	328

7.5.5	Minimum Requirements for inter-band NE-DC	328

7.5.5.1	Minimum Requirements for inter-band synchronous NE-DC	329

7.5.6	Minimum Requirements for inter-band NR DC	329

7.6	Maximum Receive Timing Difference	330

7.6.1	Introduction	330

7.6.2	Minimum Requirements for inter-band EN-DC	330

7.6.2.1	Minimum Requirements for inter-band synchronous EN-DC	331

7.6.3	Minimum Requirements for intra-band EN-DC	331

7.6.4	Minimum Requirements for NR Carrier Aggregation	332

7.6.5	Minimum Requirements for inter-band NE-DC	333

7.6.5.1	Minimum Requirements for inter-band synchronous NE-DC	333

7.6.6	Minimum Requirements for inter-band NR DC	333

7.7	deriveSSB-IndexFromCell tolerance	334

7.7.1	Minimum requirements	334

7.7A	deriveSSB-IndexFromCell tolerance for RedCap	335

7.7A.1	Minimum requirements	335

7.8	Void	335

7.9	deriveSSB-IndexFromCellInter-r17 tolerance	335

7.9.1	Minimum requirements	335

8	Signalling characteristics	332

8.1	Radio Link Monitoring	332

8.1.1	Introduction	332

8.1.1.1	Introduction of Requirement on Radio Link Monitoring for UE Configured with Relaxed Measurement Criteria	333

8.1.2	Requirements for SSB based radio link monitoring	333

8.1.2.1	Introduction	334

8.1.2.2	Minimum requirement	334

8.1.2.3	Measurement restrictions for SSB based RLM	338

8.1.2.4	Minimum requirement of SSB based radio link monitoring for UE fulfilling relaxed measurement criteria	338

8.1.3	Requirements for CSI-RS based radio link monitoring	339

8.1.3.1	Introduction	339

8.1.3.2	Minimum requirement	340

8.1.3.3	Measurement restrictions for CSI-RS based RLM	343

8.1.3.4	Minimum requirement of CSI-RS based radio link monitoring for UE fulfilling relaxed measurement criteria	344

8.1.4	Minimum requirement at transitions	345

8.1.5	Minimum requirement for UE turning off the transmitter	345

8.1.6	Minimum requirement for L1 indication	345

8.1.7	Scheduling availability of UE during radio link monitoring	346

8.1.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	346

8.1.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	346

8.1.7.3	Scheduling availability of UE performing radio link monitoring on FR2	346

8.1.7.4	Scheduling availability of UE performing radio link monitoring on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	347

8.1A	Radio Link Monitoring with CCA on Target Frequency	347

8.1A.1	Introduction	347

8.1A.2	Requirements for SSB Based Radio Link Monitoring	348

8.1A.2.1	Introduction	348

8.1A.2.2	Minimum Requirement	349

8.1A.3	Minimum requirement at transitions	352

8.1A.4	Minimum requirement for UE turning off the transmitter	353

8.1A.5	Minimum requirement for L1 indication	353

8.1A.6	Scheduling availability of UE during radio link monitoring	353

8.1A.6.3	Scheduling availability of UE performing radio link monitoring on FR2-2	353

8.1A.6.4	Scheduling availability of UE performing radio link monitoring on FR1 or FR2-2 in case of FR1-FR2-2 inter-band CA and NR-DC	354

8.1B	Radio Link Monitoring for RedCap	354

8.1B.1	Introduction	354

8.1B.2	Requirements for SSB based radio link monitoring	355

8.1B.2.1	Introduction	355

8.1B.2.2	Minimum requirement	356

8.1B.2.3	Measurement restrictions for SSB based RLM	358

8.1B.3	Requirements for CSI-RS based radio link monitoring	359

8.1B.3.1	Introduction	359

8.1B.3.2	Minimum requirement	359

8.1B.3.3	Measurement restrictions for CSI-RS based RLM	362

8.1B.4	Minimum requirement at transitions	363

8.1B.5	Minimum requirement for UE turning off the transmitter	363

8.1B.6	Minimum requirement for L1 indication	363

8.1B.7	Scheduling availability of UE during radio link monitoring	363

8.1B.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	364

8.1B.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	364

8.1B.7.3	Scheduling availability of UE performing radio link monitoring on FR2	364

8.1C	Radio Link Monitoring for Satellite Access	364

8.1C.1	Introduction	364

8.1C.2	Requirements for SSB based radio link monitoring	365

8.1C.2.1	Introduction	365

8.1C.2.2	Minimum requirement	366

8.1C.2.3	Measurement restrictions for SSB based RLM	367

8.1C.3	Requirements for CSI-RS based radio link monitoring	367

8.1C.3.1	Introduction	367

8.1C.3.2	Minimum requirement	368

8.1C.3.3	Measurement restrictions for CSI-RS based RLM	369

8.1C.4	Minimum requirement at transitions	370

8.1C.5	Minimum requirement for UE turning off the transmitter	370

8.1C.6	Minimum requirement for L1 indication	370

8.1C.7	Scheduling availability of UE during radio link monitoring	370

8.1C.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	371

8.1C.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	371

8.2	Interruption	371

8.2.1	EN-DC Interruption	371

8.2.1.1	Introduction	371

8.2.1.2	Requirements	372

8.2.1.2.1	Interruptions at transitions between active and non-active during DRX	372

8.2.1.2.2	Interruptions at transitions from non-DRX to DRX	372

8.2.1.2.3	Interruptions at SCell addition/release	372

8.2.1.2.4	Interruptions at SCell activation/deactivation	373

8.2.1.2.5	Interruptions during measurements on SCC	375

8.2.1.2.5.1	Interruptions during measurements on deactivated NR SCC	375

8.2.1.2.5.2	Interruptions during measurements on deactivated E-UTRAN SCC	375

8.2.1.2.5.3	Interruptions during CQI measurements on dormant E-UTRAN SCell	375

8.2.1.2.5.4	Interruptions during RRM measurements on dormant E-UTRAN SCC	376

8.2.1.2.6	Interruptions at UL carrier RRC reconfiguration	376

8.2.1.2.7	Interruptions due to Active BWP switching Requirement	377

8.2.1.2.8	Interruptions at direct SCell activation and hibernation	378

8.2.1.2.8.1	Interruptions during direct SCell activation and hibernation of E-UTRA SCell	378

8.2.1.2.8.2	Interruptions during direct SCell activation	378

8.2.1.2.9	Interruptions at SCell hibernation	378

8.2.1.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	379

8.2.1.2.11	Interruptions due to UE-specific CBW change	379

8.2.1.2.12	Interruptions at NR SRS carrier based switching	379

8.2.1.2.13	Interruptions at E-UTRA SRS carrier based switching	381

8.2.1.2.14	DL Interruptions at switching between two uplink carriers	382

8.2.1.2.15	Interruptions due to SCell dormancy	382

8.2.1.2.15.1	Interruptions due to SCell dormancy switch	382

8.2.1.2.15.2	Interruptions due to CQI measurements during SCell dormancy	382

8.2.1.2.15.3	Interruptions due to RRM measurements during SCell dormancy	383

8.2.1.2.16	Interruptions when identifying CGI of an NR cell with autonomous gaps	383

8.2.1.2.17	Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	383

8.2.1.2.18	Interruptions at NR SRS antenna port switching	384

8.2.1.2.19	Interruptions at fast SCell activation	385

8.2.1.2.20	Interruptions due to PUCCH SCell activation/deactivation	386

8.2.2	SA: Interruptions with Standalone NR Carrier Aggregation	386

8.2.2.1	Introduction	386

8.2.2.2	Requirements	387

8.2.2.2.1	Interruptions at SCell addition/release	387

8.2.2.2.2	Interruptions at SCell activation/deactivation	388

8.2.2.2.3	Interruptions during measurements on deactivated SCC	389

8.2.2.2.4	Interruptions at UL carrier RRC reconfiguration	390

8.2.2.2.5	Interruptions due to Active BWP switching Requirement	390

8.2.2.2.6	Interruptions at inter-frequency SFTD measurement	391

8.2.2.2.7	Interruptions at SCell activation/deactivation with multiple downlink SCells	393

8.2.2.2.8	Interruptions due to UE-specific CBW change	393

8.2.2.2.9	Interruptions at NR SRS carrier based switching	393

8.2.2.2.10	DL Interruptions at UE switching between two uplink carriers	395

8.2.2.2.10A	DL Interruptions at UE switching between two uplink carriers with two transmit antenna connectors	395

8.2.2.2.10B	DL Interruptions at UE switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors	396

8.2.2.2.10C	DL Interruptions at UE switching between two uplink bands with two transmit antenna connectors	396

8.2.2.2.11	Interruptions at direct SCell activation	396

8.2.2.2.12	Interruptions due to SCell dormancy	397

8.2.2.2.12.1	Interruptions due to SCell dormancy switch	397

8.2.2.2.12.2	Interruptions due to CQI measurements during SCell dormancy	397

8.2.2.2.12.3	Interruptions due to RRM measurements during SCell dormancy	397

8.2.2.2.13	Interruptions at transitions between active and non-active during DRX	397

8.2.2.2.14	Interruptions when identifying CGI of an NR cell with autonomous gaps	397

8.2.2.2.15	Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	398

8.2.2.2.16	Interruptions at NR SRS antenna port switching	398

8.2.2.2.17	Interruptions at fast SCell activation	399

8.2.2.2.18	Interruptions due to PUCCH SCell activation/deactivation	400

8.2.3	NE-DC Interruptions	400

8.2.3.1	Introduction	400

8.2.3.2	Requirements	401

8.2.3.2.1	Interruptions at transitions between active and non-active during DRX	401

8.2.3.2.2	Interruptions at transitions from non-DRX to DRX	401

8.2.3.2.3	Interruptions at PSCell/SCell addition/release	401

8.2.3.2.4	Interruptions at SCell activation/deactivation	403

8.2.3.2.5	Interruptions during measurements on SCC	404

8.2.3.2.5.1	Interruptions during measurements on deactivated NR SCC	404

8.2.3.2.5.2	Interruptions during measurements on deactivated E-UTRAN SCC	404

8.2.3.2.5.3	Interruptions during CQI measurements on dormant E-UTRAN SCC	405

8.2.3.2.5.4	Interruptions during RRM measurements on dormant E-UTRAN SCC	405

8.2.3.2.6	Interruptions at UL carrier RRC reconfiguration	405

8.2.3.2.7	Interruptions due to Active BWP switching Requirement	406

8.2.3.2.8	Interruptions at direct SCell activation and hibernation	406

8.2.3.2.9	Interruptions at SCell hibernation	406

8.2.3.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	407

8.2.3.2.11	 Interruptions at NR SRS carrier based switching	407

8.2.3.2.12	 Interruptions at E-UTRA SRS carrier based switching	408

8.2.3.2.13	Interruptions due to SCell dormancy	409

8.2.3.2.14	Interruptions when identifying CGI of an NR cell with autonomous gaps	410

8.2.3.2.15	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	410

8.2.3.2.17	Interruptions at fast SCell activation	412

8.2.3.2.18	Interruptions due to UE-specific CBW change	412

8.2.3.2.19	Interruptions due to PUCCH SCell activation/deactivation	413

8.2.4	NR-DC: Interruptions	413

8.2.4.1	Introduction	413

8.2.4.2	Requirements	414

8.2.4.2.1	Interruptions at PSCell/SCell addition/release	414

8.2.4.2.2	Interruptions at SCell activation/deactivation	415

8.2.4.2.3	Interruptions during measurements on SCC	416

8.2.4.2.4	Interruptions at UL carrier RRC reconfiguration	416

8.2.4.2.5	Interruptions due to Active BWP switching Requirement	417

8.2.4.2.6	Interruptions at transitions between active and non-active during DRX	417

8.2.4.2.7	Interruptions at transitions from non-DRX to DRX	417

8.2.4.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	418

8.2.4.2.9	Interruptions at NR SRS carrier based switching	418

8.2.4.2.10	Interruptions at direct SCell activation	419

8.2.4.2.11	Interruptions when identifying CGI of an NR cell with autonomous gaps	420

8.2.4.2.12	Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	420

8.2.4.2.13	 Interruptions due to SCell dormancy	421

8.2.4.2.14	Interruptions at NR SRS antenna port switching	421

8.2.4.2.15	Interruptions at fast SCell activation	422

8.2.4.2.16	Interruptions at SCG activation/deactivation	423

8.2.4.2.17	Interruptions due to RRM measurements on deactivated SCG	423

8.2.4.2.18 Interruptions during RLM/BFD measurements on deactivated PSCell	424

8.2.4.2.19	Interruptions due to UE-specific CBW change	424

8.2.4.2A	Void	424

8.2.4.2A.1	Void	424

8.2.4.2A.2	Void	424

8.2.4.2A.3	Void	424

8.3	SCell Activation and Deactivation Delay	424

8.3.1	Introduction	424

8.3.2	SCell Activation Delay Requirement for Deactivated SCell	424

8.3.3	SCell Deactivation Delay Requirement for Activated SCell	429

8.3.4	Direct SCell Activation at SCell addition	429

8.3.5	Direct SCell Activation at Handover	431

8.3.7	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	432

8.3.8	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	436

8.3.9	Direct SCell Activation of Multiple Downlink SCells at SCell addition	437

8.3.10	Direct SCell Activation of Multiple Downlink SCells at Handover	438

8.3.12	SCell Activation Delay Requirement for Deactivated PUCCH SCell	439

8.3.13	SCell activation delay Requirement for Deactivated PUCCH SCell with Multiple SCells	442

8.3.13.1	Introduction	442

8.3.14	SCell Deactivation Delay Requirement for Activated PUCCH SCell	444

8.3.15	SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple Downlink SCells	444

8.3.16	Fast SCell Activation Delay Requirement for Deactivated SCell	445

8.3A	SCell Activation and Deactivation Delay in Carriers with CCA	447

8.3A.1	Introduction	447

8.3A.2	SCell Activation Delay Requirement for Deactivated SCell	448

8.3A.3	SCell Deactivation Delay Requirement for Activated SCell	452

8.4	UE UL carrier RRC reconfiguration delay	452

8.4.1	Introduction	452

8.4.2	UE UL carrier configuration delay requirement	453

8.4.3	UE UL carrier deconfiguration delay requirement	453

8.5	Link Recovery Procedures	453

8.5.1	Introduction	453

8.5.1.1	Introduction of Requirement on Link Recovery Procedures for UE configured with relaxed measurement criteria	454

8.5.2	Requirements for SSB based beam failure detection	455

8.5.2.1	Introduction	455

8.5.2.2	Minimum requirement	455

8.5.2.3	Measurement restriction for SSB based beam failure detection	458

8.5.2.4	Minimum requirement of SSB based beam failure detection for UE fulfilling relaxed measurement criteria	459

8.5.3	Requirements for CSI-RS based beam failure detection	460

8.5.3.1	Introduction	460

8.5.3.2	Minimum requirement	460

8.5.3.3	Measurement restrictions for CSI-RS beam failure detection	464

8.5.3.4	Minimum requirement of CSI-RS based beam failure detection for UE fulfilling relaxed measurement criteria	465

8.5.4	Minimum requirement for L1 indication	465

8.5.5	Requirements for SSB based candidate beam detection	466

8.5.5.1	Introduction	466

8.5.5.2	Minimum requirement	466

8.5.5.3	Measurement restriction for SSB based candidate beam detection	469

8.5.6	Requirements for CSI-RS based candidate beam detection	469

8.5.6.1	Introduction	469

8.5.6.2	Minimum requirement	470

8.5.6.3	Measurement restriction for CSI-RS based candidate beam detection	473

8.5.7	Scheduling availability of UE during beam failure detection	473

8.5.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	473

8.5.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	474

8.5.7.3	Scheduling availability of UE performing beam failure detection on FR2	474

8.5.7.4	Scheduling availability of UE performing beam failure detection on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR DC	475

8.5.8	Scheduling availability of UE during candidate beam detection	475

8.5.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	475

8.5.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	475

8.5.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	475

8.5.8.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	476

8.5.9	Requirements for Beam Failure Recovery in SCell	476

8.5.9.1	Introduction	476

8.5.9.2	Requirement	476

8.5.10	Minimum requirement at transitions for beam failure detection	476

8.5A	Link Recovery Procedures when CCA is used on target frequency	477

8.5A.1	Introduction	477

8.5A.2	Requirements for SSB based beam failure detection	478

8.5A.2.1	Introduction	478

8.5A.2.2	Minimum requirement	478

8.5A.2.3	Measurement restriction for SSB based beam failure detection	480

8.5A.3	Void	480

8.5A.4	Minimum requirement for L1 indication	480

8.5A.5	Requirements for SSB based candidate beam detection	481

8.5A.5.1	Introduction	481

8.5A.5.2	Minimum requirement	481

8.5A.5.3	Measurement restriction for SSB based candidate beam detection	483

8.5A.6	Void	484

8.5A.7	Scheduling availability of UE during beam failure detection	484

8.5A.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH	484

8.5A.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH	484

8.5A.7.3	Scheduling availability of UE performing beam failure detection on FR2-2	484

8.5A.7.4	Scheduling availability of UE performing beam failure detection on FR1 or FR2-2 in case of FR1-FR2-2 inter-band CA and NR DC	484

8.5A.8	Scheduling availability of UE during candidate beam detection	484

8.5A.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2-2	484

8.5A.8.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2-2 in case of FR1-FR2-2 inter-band CA and NR-DC	485

8.5B	Link Recovery Procedures for Redcap	485

8.5B.1	Introduction	485

8.5B.2	Requirements for SSB based beam failure detection for Redcap	485

8.5B.2.1	Introduction	485

8.5B.2.2	Minimum requirement	486

8.5B.2.3	Measurement restriction for SSB based beam failure detection	487

8.5B.3	Requirements for CSI-RS based beam failure detection for Redcap	488

8.5B.3.1	Introduction	488

8.5B.3.2	Minimum requirement	488

8.5B.3.3	Measurement restrictions for CSI-RS beam failure detection	490

8.5B.4	Minimum requirement for L1 indication for Redcap	491

8.5B.5	Requirements for SSB based candidate beam detection for Redcap	492

8.5B.5.1	Introduction	492

8.5B.5.2	Minimum requirement	492

8.5B.5.3	Measurement restriction for SSB based candidate beam detection	494

8.5B.6	Requirements for CSI-RS based candidate beam detection for Redcap	494

8.5B.6.1	Introduction	494

8.5B.6.2	Minimum requirement	494

8.5B.6.3	Measurement restriction for CSI-RS based candidate beam detection	496

8.5B.7	Scheduling availability of UE during beam failure detection for Redcap	497

8.5B.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	497

8.5B.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	497

8.5B.7.3	Scheduling availability of UE performing beam failure detection on FR2	497

8.5B.8	Scheduling availability of UE during candidate beam detection for Redcap	498

8.5B.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	498

8.5B.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	498

8.5B.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	498

8.5B.9	Minimum requirement at transitions for beam failure detection for Redcap	498

8.5C	Link Recovery Procedures for Satellite Access	499

8.5C.1	Introduction	499

8.5C.2	Requirements for SSB based beam failure detection	499

8.5C.2.1	Introduction	499

8.5C.2.2	Minimum requirement	500

8.5C.2.3	Measurement restriction for SSB based beam failure detection	500

8.5C.3	Requirements for CSI-RS based beam failure detection	501

8.5C.3.1	Introduction	501

8.5C.3.2	Minimum requirement	501

8.5C.3.3	Measurement restrictions for CSI-RS beam failure detection	502

8.5C.4	Minimum requirement for L1 indication	503

8.5C.5	Requirements for SSB based candidate beam detection	503

8.5C.5.1	Introduction	503

8.5C.5.2	Minimum requirement	503

8.5C.5.3	Measurement restriction for SSB based candidate beam detection	504

8.5C.6	Requirements for CSI-RS based candidate beam detection	505

8.5C.6.1	Introduction	505

8.5C.6.2	Minimum requirement	505

8.5C.6.3	Measurement restriction for CSI-RS based candidate beam detection	506

8.5C.7	Scheduling availability of UE during beam failure detection	506

8.5C.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	506

8.5C.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	506

8.5C.8	Scheduling availability of UE during candidate beam detection	507

8.5C.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	507

8.5C.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	507

8.5C.9	Minimum requirement at transitions for beam failure detection	507

8.6	Active BWP switch delay	507

8.6.1	Introduction	507

8.6.2	DCI and timer based BWP switch delay on a single CC	508

8.6.2A	DCI based BWP switch delay on multiple CCs	509

8.6.2A.1	Simultaneous DCI based BWP switch delay on multiple CCs	509

8.6.2A.2	Non-simultaneous DCI based BWP switch delay on multiple CCs	511

8.6.2B	Timer based BWP switch delay on multiple CCs	511

8.6.2B.1	Simultaneous timer based BWP switch delay on multiple CCs	511

8.6.2B.2	Non-simultaneous timer based BWP switch delay on multiple CCs	511

8.6.3	RRC based BWP switch delay on a single CC	512

8.6.3A	RRC based BWP switch delay on multiple CCs	513

8.6.3A.1	Simultaneous RRC based BWP switch delay on multiple CCs	513

8.6.3A.2	Non-simultaneous RRC based BWP switch delay on multiple CCs	513

8.6.4	BWP switch delay on Consistent UL CCA recovery	514

8.6A	Active BWP switch delay for RedCap	514

8.6A.1	Introduction	514

8.6A.2	DCI and timer based BWP switch delay on a single CC	514

8.6A.3	RRC based BWP switch delay on a single CC	516

8.6C	Active BWP switch delay for satellite access	516

8.6C.1	Introduction	516

8.6C.2	DCI and timer based BWP switch delay on a single CC	516

8.6C.3	RRC based BWP switch delay on a single CC	518

8.7	Void	518

8.8	NE-DC: E-UTRAN PSCell Addition and Release Delay	518

8.8.1	Introduction	518

8.8.2	E-UTRAN PSCell Addition Delay Requirement	518

8.8.3	E-UTRAN PSCell Release Delay Requirement	519

8.9	NR-DC: PSCell Addition and Release Delay	519

8.9.1	Introduction	519

8.9.2	PSCell Addition Delay Requirement	519

8.9.3	PSCell Release Delay Requirement	520

8.9A	Conditional PSCell Addition Delay	520

8.9A.1	Introduction	520

8.9A.2	Conditional PSCell Addition Delay Requirement	520

8.9A.2.1	Measurement time	521

8.9B	NR-DC: PSCell Addition and Release Delay in Carriers with CCA	521

8.9B.1	Introduction	521

8.9B.2	PSCell Addition Delay Requirement	522

8.9B.3	PSCell Release Delay Requirement	522

8.10	Active TCI state switching delay	523

8.10.3A	MAC-CE based TCI state switch delay in HST FR2 scenarios	524

8.10.4	DCI based TCI state switch delay	524

8.10.5	RRC based TCI state switch delay	525

8.10.6	Active TCI state list update delay	525

8.10A	Active TCI state switching delay with CCA	525

8.10A.1	Introduction	525

8.10A.2	Known conditions for TCI state	526

8.10A.3	MAC-CE based TCI state switch delay	526

8.10A.4	DCI based TCI state switch delay	527

8.10A.5	RRC based TCI state switch delay	527

8.10A.6	Active TCI state list update delay	528

8.10B	Active TCI state switching delay for RedCap	528

8.10B.1	Introduction	528

8.10B.2	Known conditions for TCI state	528

8.10B.3	MAC-CE based TCI state switch delay	529

8.10B.4	DCI based TCI state switch delay	529

8.10B.5	RRC based TCI state switch delay	530

8.10B.6	Active TCI state list update delay	530

8.10C	Active TCI state switching delay for satellite access	531

8.10C.1	Introduction	531

8.10C.2	MAC-CE based TCI state switch delay	531

8.10C.4	DCI based TCI state switch delay	531

8.10C.5	RRC based TCI state switch delay	531

8.10C.6	Active TCI state list update delay	531

8.11	PSCell Change	532

8.11A	PSCell Change in Carriers with CCA	532

8.11B	Conditional PSCell Change	532

8.11B.1	Introduction	532

8.11B.2	Conditoinal PSCell Change delay	533

8.11B.2.1	Measurement time	533

8.11D	Conditional PSCell Change in Carriers with CCA	534

8.11D.1	Introduction	534

8.11D.2	Conditional PSCell Change delay	534

8.11D.2.1	Measurement time	535

8.12	Uplink spatial relation switch delay	535

8.12.1	Introduction	535

8.12.2	Known conditions for spatial relation when associated with DL-RS	535

8.12.3	MAC-CE based spatial relation switch delay	536

8.12.4	DCI based spatial relation switch delay	536

8.12.5	RRC based spatial relation switch delay	537

8.12A	Uplink spatial relation switch delay for RedCap	537

8.12A.1	Introduction	537

8.12A.2	Known conditions for spatial relation when associated with DL-RS	537

8.12A.3	MAC-CE based spatial relation switch delay	538

8.12A.4	DCI based spatial relation switch delay	538

8.12A.5	RRC based spatial relation switch delay	539

8.12C	Uplink spatial relation switch delay for satellite access	539

8.12C.1	Void	540

8.12C.2	Void	540

8.12C.3	Void	540

8.12C.4	Void	540

8.12C.5	Void	540

8.13	UE-specific CBW change	540

8.13.1	Introduction	540

8.13.2	UE-specific CBW change delay	540

8.13A	UE-specific CBW change for RedCap	540

8.13A.1	Introduction	540

8.13A.2	UE-specific CBW change delay	540

8.13C	UE-specific CBW change for satellite access	541

8.13C.1	Introduction	541

8.13C.2	UE-specific CBW change delay	541

8.14	Pathloss reference signal switching delay	541

8.14.1	Introduction	541

8.14.2	Known conditions for pathloss reference signal	542

8.14.3	MAC-CE based pathloss reference signal switch delay	542

8.14C	Pathloss reference signal switching delay for satellite access	543

8.14C.1	Introduction	543

8.14C.2	Known conditions for pathloss reference signal	543

8.14C.3	MAC-CE based pathloss reference signal switch delay	543

8.15	Active downlink TCI state switching delay for unified TCI	544

8.15.1	Introduction	544

8.15.4	DCI based downlink TCI state switch delay	546

8.15.5	Active Downlink TCI state list update delay	546

8.16	Active uplink TCI state switching delay for unified TCI	547

8.16.1	Introduction	547

8.16.4	DCI based uplink TCI state switch delay	549

8.16.5	Active Uplink TCI state list update delay	550

8.17	SCG Activation and Deactivation Delay	551

8.17.1	Introduction	551

8.17.2	SCG Activation Delay Requirement	551

8.17.3	SCG Deactivation Delay Requirement	552

8.18	TRP specific Link Recovery Procedures	553

8.18.1	Introduction	553

8.18.2	Requirements for TRP specific SSB based beam failure detection	553

8.18.2.1	Introduction	553

8.18.2.2	Minimum requirement	554

8.18.2.3	Measurement restriction for SSB based beam failure detection	556

8.18.3	Requirements for CSI-RS based beam failure detection	556

8.18.3.1	Introduction	556

8.18.3.2	Minimum requirement	557

8.18.3.3	Measurement restrictions for CSI-RS beam failure detection	559

8.18.4	Minimum requirement for L1 indication	560

8.18.5	Requirements for SSB based candidate beam detection	560

8.18.5.1	Introduction	560

8.18.5.2	Minimum requirement	560

8.18.5.3	Measurement restriction for SSB based candidate beam detection	562

8.18.6	Requirements for CSI-RS based candidate beam detection	563

8.18.6.1	Introduction	563

8.18.6.2	Minimum requirement	563

8.18.6.3	Measurement restriction for CSI-RS based candidate beam detection	565

8.18.7	Requirements for TRP specific Beam Failure Recovery	566

8.18.7.1	Introduction	566

8.18.7.2	Requirement	566

8.18.8	Scheduling availability of UE during TRP specific beam failure detection	566

8.18.8.1	Scheduling availability of UE performing TRP specific beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	566

8.18.8.2	Scheduling availability of UE performing TRP specific beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	567

8.18.8.3	Scheduling availability of UE performing TRP specific beam failure detection on FR2	567

8.18.8.4	Scheduling availability of UE performing TRP specific beam failure detection on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR DC	568

8.18.9	Scheduling availability of UE during TRP specific candidate beam detection	568

8.18.9.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	568

8.18.9.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	568

8.18.9.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	568

8.18.9.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	569

8.19	Pre-configured measurement gap activation/deactivation delay	569

8.19.1	Introduction	569

8.19.2	Pre-configured measurement gap activation/deactivation upon DCI/timer-based BWP switch	569

8.19.2.1	Activation/deactivation upon DCI/timer-based BWP switch delay on a single CC	569

8.19.3	Pre-configured measurement gap activation/deactivation upon SCell activation/deactivation	569

8.19.4	Pre-configured measurement gap activation/deactivation upon RRC reconfiguration	570

9	Measurement Procedure	573

9.1	General measurement requirement	573

9.1.1	Introduction	573

9.1.2	Measurement gap	573

9.1.2.1	EN-DC: Measurement Gap Sharing	585

9.1.2.1a	SA: Measurement Gap Sharing	586

9.1.2.1b	NE-DC: Measurement Gap Sharing	587

9.1.2.1c	NR-DC: Measurement Gap Sharing	588

9.1.3	UE Measurement capability	589

9.1.3.1	EN-DC: Monitoring of multiple layers using gaps	589

9.1.3.1a	SA: Monitoring of multiple layers using gaps	589

9.1.3.1b	NE-DC: Monitoring of multiple layers using gaps	590

9.1.3.1c	NR-DC: Monitoring of multiple layers using gaps	590

9.1.3.2	EN-DC: Maximum allowed layers for multiple monitoring	591

9.1.3.2a	SA: Maximum allowed layers for multiple monitoring	592

9.1.3.2b	NE-DC: Maximum allowed layers for multiple monitoring	592

9.1.3.2c	NR-DC: Maximum allowed layers for multiple monitoring	593

9.1A.3.2	Void	594

9.1.3A	UE Measurement capability under operation mode with CCA	594

9.1.3A.1	EN-DC: Monitoring of multiple layers using gaps under CCA	594

9.1.3A.1a	SA: Monitoring of multiple layers using gaps under CCA	594

9.1.3A.2	EN-DC: Maximum allowed layers for multiple monitoring under CCA	594

9.1.3A.2a	SA: Maximum allowed layers for multiple monitoring under CCA	595

9.1.3C	UE Measurement capability under operation mode with satellite access	595

9.1.3C.1a	SA: Monitoring of multiple layers using gaps under satellite access	595

9.1.3C.2a	SA: Maximum allowed layers for multiple monitoring for SAN	596

9.1.4	Capabilities for Support of Event Triggering and Reporting Criteria	596

9.1.4.1	Introduction	596

9.1.4.2	Requirements	596

9.1.5	Carrier-specific scaling factor	600

9.1.5.1	Monitoring of multiple layers outside gaps	601

9.1.5.1.1	EN-DC mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	603

9.1.5.1.2	SA mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	604

9.1.5.1.3	NR-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed outside gaps	605

9.1.5.1.4	NE-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based measurements performed outside gaps	605

9.1.5.2	Monitoring of multiple layers within gaps	606

9.1.5.2.1	EN-DC mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	608

9.1.5.2.2	SA mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	610

9.1.5.2.3	NE-DC: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed within gaps	612

9.1.5.2.4	NR-DC: carrier-specific scaling factor for SSB-based and CSI-RS-based L3 measurements performed within gaps	614

9.1.5.2.5	SA mode: carrier-specific scaling factor for PRS-based measurements performed within gaps	615

9.1.5.2.6	NE-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	616

9.1.5.2.7	NR-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	616

9.1.5.3	Monitoring of multiple layers within NCSG	616

9.1.5.3.1	SA mode: carrier-specific scaling factor for measurements performed within NCSG	617

9.1.6	Minimum requirement at transitions	617

9.1.7	Pre-configured measurement gap	618

9.1.7.1	Introduction	618

9.1.7.2	Requirements applicability	618

9.1.7.3	Requirements	618

9.1.7.3.1	Requirements for autonomous activation/deactivation mechanism	619

9.1.7.3.2	Requirements for network-controlled activation/deactivation mechanism	619

9.1.7.3.3	Requirements for reception/transmission during activation/deactivation	620

9.1.8	Concurrent measurement gaps	620

9.1.8.1	Introduction	620

9.1.8.2	Requirements	620

9.1.8.3	Collision between concurrent measurement gaps	622

9.1.8.4	Measurement gap related requirements of concurrent measurement gaps	622

9.1.9	Network controlled small gap	622

9.1.9.1	Introduction	622

9.1.9.2	Requirements applicability	623

9.1.10	MUSIM gaps	625

9.1.11	UL gap for Tx power management	626

9.1A	General measurement requirement for RedCap	627

9.1A.1	Introduction	627

9.1A.2	Measurement gap	627

9.1A.2.1	SA: Measurement Gap Sharing	631

9.1A.3	UE Measurement capability	632

9.1A.3.1	SA: Monitoring of multiple layers using gaps	632

9.1A.3.2	SA: Maximum allowed layers for multiple monitoring	632

9.1A.4	Capabilities for Support of Event Triggering and Reporting Criteria	632

9.1A.4.1	Introduction	632

9.1A.4.2	Requirements	633

9.1A.5	Carrier-specific scaling factor	633

9.1A.5.1	Monitoring of multiple layers outside gaps	633

9.1A.5.1.1	SA mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps	634

9.1A.5.2	Monitoring of multiple layers within gaps	634

9.1A.5.2.1	SA mode: carrier-specific scaling factor for SSB measurements performed within gaps	634

9.1A.6	Minimum requirement at transitions	635

9.1C	General measurement requirement for SAN	636

9.1C.1	Introduction	636

9.1C.2	Measurement gap	636

9.1C.8	Concurrent measurement gaps for SAN	638

9.1C.8.1	Introduction	638

9.1C.8.2	Requirements	638

9.1C.8.3	Collision between concurrent measurement gaps	638

9.1C.8.4	Measurement gap related requirements of concurrent measurement gaps	639

9.1C.9	Collision between SMTC and measurement gap for SAN	639

9.1C.9.1	Introduction	639

9.1C.9.2	Collision between SMTCs and measurement gap	639

9.1C.9.3	Collision between multiple SMTCs on a SAN carrier	639

9.2	NR intra-frequency measurements	640

9.2.1	Introduction	640

9.2.2	Requirements applicability	642

9.2.3	Number of cells and number of SSB	642

9.2.3.1	Requirements for FR1	642

9.2.3.2	Requirements for FR2	642

9.2.4	Measurement Reporting Requirements	643

9.2.4.1	Periodic Reporting	643

9.2.4.2	Event-triggered Periodic Reporting	643

9.2.4.3	Event Triggered Reporting	643

9.2.5	Intrafrequency measurements without measurement gaps	643

9.2.5.1	Intrafrequency cell identification	643

9.2.5.2	Measurement period	648

9.2.5.3	Scheduling availability of UE during intra-frequency measurements	651

9.2.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	651

9.2.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	652

9.2.5.3.3	Scheduling availability of UE performing measurements on FR2	652

9.2.5.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	654

9.2.5.4	SFTD Measurements between PCell and PSCell	654

9.2.5.4.1	Introduction	654

9.2.5.4.2	SFTD Measurement delay	654

9.2.5.4.3	SFTD Measurement Reporting Delay	655

9.2.6	Intra-frequency measurements with measurement gaps	655

9.2.6.1	Void	655

9.2.6.2	Intra-frequency cell identification	655

9.2.6.3	Intrafrequency Measurement Period	658

9.2.7	Intra-frequency measurements with NCSG	659

9.2.7.1	Intra-frequency cell identification	659

9.2.7.2	Measurement period	661

9.2.7.3	Scheduling availability during intra-frequency measurement with NCSG	662

9.2A	NR intra-frequency measurements with CCA	662

9.2A.1	Introduction	662

9.2A.2	Requirements applicability	663

9.2A.3	Number of cells and number of SSB	664

9.2A.3.1	Requirements for FR1	664

9.2A.3.2	Requirements for FR2-2	664

9.2A.4	Measurement Reporting Requirements	664

9.2A.5	Intra-frequency measurements without measurement gaps	665

9.2A.5.2	Measurement period	669

9.2A.5.3	Scheduling availability of UE during intra-frequency measurements	671

9.2A.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	672

9.2A.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	672

9.2A.5.3.3	Scheduling availability of UE performing measurements in TDD bands on FR2-2	672

9.2A.6	Intra-frequency measurements with measurement gaps	672

9.2A.6.1	Intra-frequency cell identification	672

9.2A.6.2	Intra-frequency Measurement Period	675

9.2A.7	Intra-frequency RSSI and Channel occupancy measurements	676

9.2A.7.1	Intra-frequency RSSI measurements	676

9.2A.7.2	Intra-frequency Channel occupancy measurements	678

9.2A.7.3	Scheduling restriction during RSSI and Channel Occupancy measurements in FR1	679

9.2A.7.4	Scheduling restriction during RSSI measurements in FR2-2	679

9.2B	NR intra-frequency measurements for RedCap	680

9.2B.1	Introduction	680

9.2B.2	Requirements applicability	680

9.2B.3	Number of cells and number of SSB	681

9.2B.3.1	Requirements for FR1	681

9.2B.3.2	Requirements for FR2	681

9.2B.4	Measurement Reporting Requirements	681

9.2B.4.1	Periodic Reporting	681

9.2B.4.2	Event-triggered Periodic Reporting	681

9.2B.4.3	Event Triggered Reporting	681

9.2B.5	Intra-frequency measurements without measurement gaps	682

9.2B.5.1	Intra-frequency cell identification	682

9.2B.5.2	Measurement period	684

9.2B.5.3	Scheduling availability of UE during intra-frequency measurements	685

9.2B.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	685

9.2B.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	685

9.2B.5.3.3	Scheduling availability of UE performing measurements on FR2	686

9.2B.5.3.4	Scheduling availability of UE performing measurements in HD-FDD bands on FR1	686

9.2B.6	Intra-frequency measurements with measurement gaps	687

9.2B.6.1	Intra-frequency cell identification	687

9.2B.6.2	Intra-frequency Measurement Period	688

9.2C	NR intra-frequency measurements for SAN	689

9.2C.1	Introduction	689

9.2C.2	Requirements applicability	690

9.2C.3	Number of cells and number of SSB	690

9.2C.3.1	Requirements for FR1	690

9.2C.4	Measurement Reporting Requirements	690

9.2C.4.1	Periodic Reporting	690

9.2C.4.2	Event-triggered Periodic Reporting	691

9.2C.4.3	Event Triggered Reporting	691

9.2C.5	Intra frequency measurements without measurement gaps	691

9.2C.5.1	Intra frequency cell identification	691

9.2C.5.2	Measurement period	693

9.2C.5.3	Scheduling availability of UE during intra-frequency measurements	694

9.2C.5.3.1	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	694

9.2C.5.3.2	Scheduling availability of UE performing measurements on a neighbor cell served by a different satellite in LEO	694

9.2C.6	Intra-frequency measurements with measurement gaps	695

9.2C.6.1	Intra-frequency cell identification	695

9.2C.6.3	Intrafrequency Measurement Period	696

9.3	NR inter-frequency measurements	696

9.3.1	Introduction	696

9.3.2	Requirements applicability	698

9.3.2.1	Void	699

9.3.2.2	Void	699

9.3.3	Number of cells and number of SSB	699

9.3.3.1	Requirements for FR1	699

9.3.3.2	Requirements for FR2	699

9.3.4	Inter-frequency measurement with measurement gaps	699

9.3.4.1	Void	702

9.3.4.2	Void	702

9.3.5	Inter-frequency measurements	702

9.3.5.1	Void	704

9.3.5.2	Void	704

9.3.5.3	Void	704

9.3.6	Inter-frequency measurements reporting requirements	704

9.3.6.1	Periodic Reporting	704

9.3.6.2	Event-triggered Periodic Reporting	704

9.3.6.3	Event-triggered Reporting	704

9.3.7	Void	705

9.3.8	Inter-frequency SFTD measurement requirements	705

9.3.8.1	Introduction	705

9.3.8.2	SFTD Measurement delay	705

9.3.8.3	SFTD Measurement reporting delay	706

9.3.9	Inter frequency measurements without measurement gaps	706

9.3.9.1	Inter frequency Cell identification	706

9.3.9.2	Measurement period	709

9.3.9.3	Scheduling availability of UE during inter-frequency measurements when the SSB is completely contained in the active BWP of the UE	710

9.3.9.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	710

9.3.9.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	711

9.3.9.3.3	Scheduling availability of UE performing measurements on FR2	711

9.3.9.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	712

9.3.9.4	Scheduling availability of UE during inter-frequency measurements when the SSB is not completely contained in the active BWP of the UE	712

9.3.9.4.1	Scheduling availability of UE performing measurements in TDD bands on FR1	712

9.3.9.4.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	713

9.3.9.4.3	Scheduling availability of UE performing measurements on FR2	713

9.3.9.4.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	715

9.3.10	Inter-frequency measurement with NCSG	715

9.3.10.1	Inter-frequency cell identification	715

9.3.10.2	Measurement period	717

9.3.10.3	Scheduling availability during inter-frequency measurement with NCSG	717

9.3.10.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	717

9.3.10.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	718

9.3.10.3.3	Scheduling availability of UE performing measurements on FR2	719

9.3.10.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	721

9.3A	NR inter-frequency measurements in carrier frequencies with CCA	721

9.3A.1	Introduction	721

9.3A.2	Requirements applicability	721

9.3A.3	Number of cells and number of SSB	722

9.3A.3.1	Requirements for FR1	722

9.3A.3.2	Requirements for FR2-2	722

9.3A.4	Inter-frequency cell identification	722

9.3A.5	Inter-frequency measurements	724

9.3A.6	NR Inter-frequency measurements reporting requirements	726

9.3A.6.1	Periodic Reporting	726

9.3A.6.2	Event-triggered Periodic Reporting	726

9.3A.6.3	Event-triggered Reporting	726

9.3A.8	Inter-frequency RSSI measurements	726

9.3A.9	Inter-frequency channel occupancy measurements	727

9.3B	NR inter-frequency measurements for RedCap	728

9.3B.1	Introduction	728

9.3B.2	Requirements applicability	728

9.3B.3	Number of cells and number of SSB	729

9.3B.3.1	Requirements for FR1	729

9.3B.3.2	Requirements for FR2	729

9.3B.4	Inter-frequency measurement with measurement gaps	729

9.3B.5	Inter-frequency measurements	730

9.3B.6	Inter-frequency measurements reporting requirements	731

9.3B.6.1	Periodic Reporting	731

9.3B.6.2	Event-triggered Periodic Reporting	731

9.3B.6.3	Event-triggered Reporting	731

9.3B.7	Inter frequency measurements without measurement gaps	732

9.3B.7.1	Inter frequency Cell identification	732

9.3B.7.2	Measurement period	734

9.3B.7.3	Scheduling availability of UE during inter-frequency measurements	735

9.3B.7.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	735

9.3B.7.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	736

9.3B.7.3.3	Scheduling availability of UE performing measurements on FR2	736

9.3B.7.3.4	Scheduling availability of UE performing measurements in HD-FDD bands on FR1	736

9.3C	NR inter-frequency measurements for SAN	737

9.3C.1	Introduction	737

9.3C.2	Requirements applicability	737

9.3C.3	Number of cells and number of SSB	737

9.3C.3.1	Requirements for FR1	737

9.3C.4	Inter-frequency measurement with measurement gaps	738

9.3C.5	Inter-frequency measurements	739

9.3C.6	Inter-frequency measurements reporting requirements	739

9.3C.6.1	Periodic Reporting	739

9.3C.6.2	Event-triggered Periodic Reporting	739

9.3C.6.3	Event-triggered Reporting	740

9.3C.7	Inter frequency measurements without measurement gaps	740

9.3C.7.1	Inter frequency Cell identification	740

9.3C.7.2	Measurement period	742

9.3C.7.3	Scheduling availability of UE during inter-frequency measurements	742

9.3C.7.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	742

9.3C.7.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	743

9.4	Inter-RAT measurements	743

9.4.1	Introduction	743

9.4.2	NR − E-UTRAN FDD measurements	745

9.4.2.1	Introduction	745

9.4.2.2	Requirements when no DRX is used	745

9.4.2.3	Requirements when DRX is used	746

9.4.2.4	Measurement reporting requirements	748

9.4.2.4.1	Periodic Reporting	748

9.4.2.4.2	Event-Triggered Periodic Reporting	748

9.4.2.4.3	Event-Triggered Reporting	749

9.4.2.5	Scheduling Availability During NR − E-UTRAN FDD measurements with NCSG	749

9.4.3	NR − E-UTRAN TDD measurements	749

9.4.3.1	Introduction	749

9.4.3.2	Requirements when no DRX is used	749

9.4.3.3	Requirements when DRX is used	751

9.4.3.4	Measurement reporting requirements	752

9.4.3.4.1	Periodic Reporting	752

9.4.3.4.2	Event-Triggered Periodic Reporting	753

9.4.3.4.3	Event-Triggered Reporting	753

9.4.3.5	Scheduling Availability During NR − E-UTRAN TDD measurements with NCSG	753

9.4.4	Inter-RAT RSTD measurements	753

9.4.4.1	NR − E-UTRAN FDD RSTD measurements	753

9.4.4.1.1	Introduction	753

9.4.4.1.2	Requirements	754

9.4.4.2	NR − E-UTRAN TDD RSTD measurements	757

9.4.4.2.1	Introduction	757

9.4.4.2.2	Requirements	758

9.4.5	Inter-RAT E-CID measurements	761

9.4.5.1	NR−E-UTRAN FDD E-CID RSRP and RSRQ measurements	761

9.4.5.1.1	Introduction	761

9.4.5.1.2	Requirements	761

9.4.5.1.3	Measurement Reporting Delay	761

9.4.5.2	NR−E-UTRAN TDD E-CID RSRP and RSRQ measurements	762

9.4.5.2.1	Introduction	762

9.4.5.2.2	Requirements	762

9.4.5.2.3	Measurement Reporting Delay	762

9.4.6	NR − UTRAN FDD measurements	762

9.4.6.1	Introduction	762

9.4.6.2	Requirements when no DRX is used	762

9.4.6.3	Requirements when DRX is used	763

9.4.7	NR – E-UTRAN measurements with autonomous gaps	765

9.4.7.1	CGI identification of an E-UTRA cell with autonomous gaps	765

9.4.7.2	CGI reporting delay	765

9.4A	Inter-RAT measurements for RedCap	766

9.4A.1	Introduction	766

9.4A.2	NR − E-UTRAN FDD measurements	767

9.4A.2.1	Introduction	767

9.4A.2.2	Requirements when no DRX is used	767

9.4A.2.3	Requirements when DRX is used	768

9.4A.2.4	Measurement reporting requirements	769

9.4A.2.4.1	Periodic Reporting	769

9.4A.2.4.2	Event-Triggered Periodic Reporting	769

9.4A.2.4.3	Event-Triggered Reporting	769

9.4A.3	NR − E-UTRAN TDD measurements	770

9.4A.3.1	Introduction	770

9.4A.3.2	Requirements when no DRX is used	770

9.4A.3.3	Requirements when DRX is used	772

9.4A.3.4	Measurement reporting requirements	773

9.4A.3.4.1	Periodic Reporting	773

9.4A.3.4.2	Event-Triggered Periodic Reporting	773

9.4A.3.4.3	Event-Triggered Reporting	773

9.4A.4	NR – E-UTRAN measurements with autonomous gaps	774

9.4A.4.1	CGI identification of an E-UTRA cell with autonomous gaps	774

9.4A.4.2	CGI reporting delay	774

9.4A.4.3	CGI reporting scheduling restriction	774

9.5	L1-RSRP measurements for Reporting	775

9.5.1	Introduction	775

9.5.2	Requirements applicability	775

9.5.3	Measurement Reporting Requirements	776

9.5.3.1	Periodic Reporting	776

9.5.3.2	Semi-Persistent Reporting	776

9.5.3.3	Aperiodic Reporting	776

9.5.4	L1-RSRP measurement requirements	777

9.5.4.1	SSB based L1-RSRP Reporting	777

9.5.4.2	CSI-RS based L1-RSRP Reporting	780

9.5.4A	Void	783

9.5.4A.1	Void	783

9.5.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	783

9.5.5.1	Measurement restriction for SSB based L1-RSRP	783

9.5.5.2	Measurement restriction for CSI-RS based L1-RSRP	784

9.5.6	Scheduling availability of UE during L1-RSRP measurement	785

9.5.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	785

9.5.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	785

9.5.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	785

9.5.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	787

9.5A	L1-RSRP measurements for Reporting under CCA	787

9.5A.1	Introduction	787

9.5A.2	Requirements applicability	787

9.5A.3	Measurement Reporting Requirements	787

9.5A.3.1	Periodic Reporting	788

9.5A.3.2	Semi-Persistent Reporting	788

9.5A.3.3	Aperiodic Reporting	788

9.5A.4	L1-RSRP measurement requirements	788

9.5A.4.1	SSB based L1-RSRP Reporting	788

9.5A.5	Measurement restriction for L1-RSRP measurement	791

9.5A.5.1	Measurement restriction for SSB based L1-RSRP	791

9.5A.6	Scheduling availability of UE during L1-RSRP measurement	791

9.5A.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	791

9.5A.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	792

9.5A.6.3	Scheduling availability of UE performing L1-RSRP measurement in case of FR1-FR2 inter-band CA	792

9.5A.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2-2	792

9.5A.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	793

9.5B	L1-RSRP measurements for Reporting for RedCap	793

9.5B.1	Introduction	793

9.5B.2	Requirements applicability	793

9.5B.3	Measurement Reporting Requirements	794

9.5B.3.1	Periodic Reporting	794

9.5B.3.2	Semi-Persistent Reporting	794

9.5B.3.3	Aperiodic Reporting	794

9.5B.4	L1-RSRP measurement requirements	795

9.5B.4.1	SSB based L1-RSRP Reporting	795

9.5B.4.2	CSI-RS based L1-RSRP Reporting	796

9.5B.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	799

9.5B.5.1	Measurement restriction for SSB based L1-RSRP	799

9.5B.5.2	Measurement restriction for CSI-RS based L1-RSRP	800

9.5B.6	Scheduling availability of UE during L1-RSRP measurement	800

9.5B.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	800

9.5B.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	800

9.5B.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	801

9.5C	L1-RSRP measurements for Reporting for satellite access	801

9.5C.1	Introduction	801

9.5C.3	Measurement Reporting Requirements	802

9.5C.3.1	Periodic Reporting	802

9.5C.3.2	Semi-Persistent Reporting	802

9.5C.3.3	Aperiodic Reporting	803

9.5C.4	L1-RSRP measurement requirements	803

9.5C.4.1	SSB based L1-RSRP Reporting	803

9.5C.5	Measurement restriction for L1-RSRP measurement	805

9.5C.5.1	Measurement restriction for SSB based L1-RSRP	805

9.5C.5.2	Measurement restriction for CSI-RS based L1-RSRP	805

9.5C.6	Scheduling availability of UE during L1-RSRP measurement	806

9.5C.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	806

9.5C.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	806

9.6	NE-DC: Measurements	806

9.6.1	Introduction	806

9.6.2	SFTD Measurements	806

9.6.2.1	Introduction	806

9.6.2.2	SFTD Measurement requirements	806

9.7	Cross Link Interference measurements	807

9.7.1	Introduction	807

9.7.2	SRS-RSRP measurements	807

9.7.2.1	Introduction	807

9.7.2.2	Requirements applicability	808

9.7.2.3	Measurement Reporting Requirements	808

9.7.2.3.1	Periodic Reporting	808

9.7.2.3.2	Event-triggered Periodic Reporting	808

9.7.2.3.3	Event Triggered Reporting	808

9.7.2.4	Measurement capability	808

9.7.2.5	SRS-RSRP measurement period	809

9.7.3	CLI-RSSI measurements	809

9.7.3.1	Introduction	809

9.7.3.2	Requirements applicability	809

9.7.3.3	Measurement Reporting Requirements	809

9.7.3.3.1	Periodic Reporting	809

9.7.3.3.2	Event-triggered Periodic Reporting	809

9.7.3.3.3	Event Triggered Reporting	810

9.7.3.4	Measurement capability	810

9.7.3.5	CLI-RSSI measurement period	810

9.7.4	Scheduling availability of UE during CLI measurements	810

9.7.4.1	Scheduling availability of UE performing measurement on FR1	810

9.7.4.2	Scheduling availability of UE performing measurement on FR2	811

9.8	L1-SINR measurements for Reporting	811

9.8.1	Introduction	811

9.8.2	Requirements applicability	812

9.8.3	Measurement Reporting Requirements	812

9.8.3.1	Periodic Reporting	812

9.8.3.2	Semi-Persistent Reporting	813

9.8.4	L1-SINR measurement requirements	813

9.8.4.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	813

9.8.4.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	818

9.8.5	Measurement restriction for L1-SINR measurement	819

9.8.5.1	Measurement restriction if SSB configured for L1-SINR Measurement	820

9.8.5.2	Measurement restriction if CSI-RS configured for L1-SINR measurement	820

9.8.5.3	Measurement restriction if CSI-IM configured for L1-SINR measurement	821

9.8.6	Scheduling availability of UE during L1-SINR measurement	821

9.8.6.1	Scheduling availability of UE performing L1-SINR measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	821

9.8.6.2	Scheduling availability of UE performing L1-SINR measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	821

9.8.6.4	Scheduling availability of UE performing L1-SINR measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	822

9.9	 NR measurements for positioning	823

9.9.1	Introduction	823

9.9.1.1	General Aspects of Gap-based Measurement	823

9.9.1.2	General Aspects of Gapless Measurement	823

9.9.1.3	Scheduling Availability of UE during PRS Measurement without Measurement Gaps	824

9.9.2	RSTD measurements	825

9.9.2.1	Introduction	825

9.9.2.2	Requirements Applicability	825

9.9.2.3	Measurement Capability	825

9.9.2.4	Measurement Reporting Requirements	825

9.9.2.4.1	Void	826

9.9.2.4.2	Void	826

9.9.2.4.3	Void	826

9.9.2.5	Measurements Period Requirements	826

9.9.2.6	Void	829

9.9.2.7	Measurements Period Requirements without Measurement Gaps	829

9.9.2.8	Void	832

9.9.2.9	Measurements Period Requirements with both MG and PPW	832

9.9.3	PRS-RSRP measurements	832

9.9.3.1	Introduction	832

9.9.3.2	Requirements applicability	832

9.9.3.3	Measurement Capability	833

9.9.3.4	Measurement Reporting Requirements	833

9.9.3.5	Measurement Period Requirements	833

9.9.3.6	Measurement Period Requirements without Measurement Gaps	835

9.9.3.7	Void	838

9.9.3.8	Measurements Period Requirements with both MG and PPW	838

9.9.4	UE Rx-Tx time difference measurements	839

9.9.4.1 Introduction	839

9.9.4.2 Requirements Applicability	839

9.9.4.3 Measurement Capability	839

9.9.4.4 Measurement Reporting Requirements	839

9.9.4.5	Measurement Period Requirements	839

9.9.4.6	Measurement Period Requirements without Measurement Gaps	843

9.9.4.7	Void	846

9.9.4.8	Measurements Period Requirements with both MG and PPW	846

9.9.5	E-CID measurements	847

9.9.5.1	Introduction	847

9.9.5.2	Measurement Requirements	847

9.9.5.2.1	Intra-frequency Measurement Requirements	847

9.9.5.2.2	Inter-frequency Measurement Requirements	847

9.9.5.2.3	Measurement Reporting Delay	848

9.9.6	PRS-RSRPP measurements	848

9.9.6.1	Introduction	848

9.9.6.2	Requirements applicability	848

9.9.6.3	Measurement capability	848

9.9.6.4	Measurement reporting requirements	848

9.9.6.5	Measurement period requirements	848

9.9.6.6	Measurement Period Requirements without Measurement Gaps	849

9.9.6.7	Void	849

9.9.6.8	Measurements Period Requirements with both MG and PPW	849

9.10	CSI-RS based L3 measurements	849

9.10.1	Introduction	849

9.10.2	CSI-RS based intra-frequency measurements	849

9.10.2.1	Introduction	849

9.10.2.2	Requirements applicability	850

9.10.2.3	Number of cells and number of CSI-RS	851

9.10.2.3.1	Requirements for FR1	851

9.10.2.3.2	Requirements for FR2	851

9.10.2.4	Measurement Reporting Requirements	851

9.10.2.4.1	Periodic Reporting	851

9.10.2.4.2	Event-triggered Periodic Reporting	852

9.10.2.4.3	Event Triggered Reporting	852

9.10.2.5	Intra-frequency measurements without measurement gaps	852

9.10.2.6	Scheduling availability of UE during CSI-RS based intra-frequency measurements	853

9.10.2.6.1	Scheduling availability of UE performing CSI-RS based measurements in TDD bands	854

9.10.2.6.2	Scheduling availability of UE performing CSI-RS based measurements in FR2	854

9.10.3	CSI-RS based Inter-frequency measurements	854

9.10.3.1	Introduction	854

9.10.3.2	Requirements applicability	854

9.10.3.3	Number of cells and number of CSI-RS resources	855

9.10.3.3.1	Requirements for FR1	855

9.10.3.3.2	Requirements for FR2	855

9.10.3.4	Measurements reporting requirements	855

9.10.3.4.1	Periodic Reporting	856

9.10.3.4.2	Event-triggered Periodic Reporting	856

9.10.3.4.3	Event-triggered Reporting	856

9.10.3.5	Inter frequency measurements with measurement gaps	856

9.11	NR measurements with autonomous gaps	858

9.11.1	Introduction	858

9.11.2	CGI identification of an NR cell with autonomous gaps	858

9.11.3	CGI reporting delay	859

9.11A	NR measurements with autonomous gaps for RedCap	859

9.11A.1	Introduction	859

9.11A.2	CGI identification of an NR cell with autonomous gaps	859

9.11A.3	CGI reporting delay	860

9.11A.4	CGI reporting scheduling restriction	861

9.12	Measurement for Propagation Delay Compensation	861

9.12.1	Introduction	861

9.12.2	Requirements Applicability	861

9.12.3	Measurement Capability	862

9.12.4	Measurement period requirements	862

9.12.4.1	PRS Measurement Period	862

9.12.4.2	TRS Measurement Period	863

9.12.5	Measurement Reporting Requirements	864

9.12.6	Scheduling availability during measurement for Propagation Delay Compensation	864

9.12.7	Measurement restriction for measurement for Propagation Delay Compensation	864

9.13	L1-RSRP measurements for a cell with different PCI from serving cell	864

9.13.1	Introduction	864

9.13.2	Requirements Applicability	865

9.13.3	Measurement Reporting Requirements	865

9.13.3.1	Periodic Reporting	865

9.13.3.2	Semi-Persistent Reporting	866

9.13.3.3	Aperiodic Reporting	866

9.13.4	L1-RSRP measurement requirements	866

9.13.4.1	Inter-cell SSB based L1-RSRP Reporting	866

9.13.5	Measurement restriction for L1-RSRP measurement	868

9.13.5.1	Measurement restriction for SSB based L1-RSRP	868

9.13.6	Scheduling availability of UE during L1-RSRP measurement	869

9.13.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	869

9.13.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	869

9.13.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	870

9.13.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	870

9.13.6.5	Scheduling availability of UE performing L1-RSRP measurement in TDD bands on FR1	870

10	Measurement Performance requirements	872

10.1	NR measurements	872

10.1.1	Introduction	872

10.1.2	Intra-frequency RSRP accuracy requirements for FR1	872

10.1.2.1	Intra-frequency SS-RSRP accuracy requirements	872

10.1.2.1.1	Absolute SS-RSRP Accuracy	872

10.1.2.1.2	Relative SS-RSRP Accuracy	873

10.1.2.2	Void	874

10.1.2.3	Intra-frequency CSI-RSRP accuracy requirements	874

10.1.2.3.1	Absolute CSI-RSRP Accuracy	874

10.1.2.3.2	Relative CSI-RSRP Accuracy	875

10.1.2B	Intra-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	876

10.1.2B.1	Intra-frequency SS-RSRP accuracy requirements	876

10.1.2B.1.1	Absolute SS-RSRP Accuracy	876

10.1.2C	Intra-frequency RSRP accuracy requirements for FR1 SAN	877

10.1.2C.1	Intra-frequency SS-RSRP accuracy requirements	877

10.1.2C.1.1	Absolute SS-RSRP Accuracy	877

10.1.2C.1.2	Relative SS-RSRP Accuracy	878

10.1.3	Intra-frequency RSRP accuracy requirements for FR2	878

10.1.3.1	Intra-frequency SS-RSRP accuracy requirements	878

10.1.3.1.1	Absolute SS-RSRP Accuracy	878

10.1.3.1.2	Relative SS-RSRP Accuracy	879

10.1.3.2	Void	880

10.1.3.3	Intra-frequency CSI-RSRP accuracy requirements	880

10.1.3.3.1	Absolute CSI-RSRP Accuracy	880

10.1.3.3.2	Relative CSI-RSRP Accuracy	881

10.1.3B	Intra-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	882

10.1.3B.1	Intra-frequency SS-RSRP accuracy requirements	882

10.1.3B.1.1	Absolute SS-RSRP Accuracy	882

10.1.4	Inter-frequency RSRP accuracy requirements for FR1	883

10.1.4.1	Inter-frequency SS-RSRP accuracy requirements	883

10.1.4.1.1	Absolute Accuracy of SS-RSRP in FR1	883

10.1.4.1.2	Relative Accuracy of SS-RSRP in FR1	884

10.1.4.2	Void	885

10.1.4.3	Inter-frequency CSI-RSRP accuracy requirements	885

10.1.4.3.1	Absolute Accuracy of CSI-RSRP in FR1	885

10.1.4.3.2	Relative Accuracy of CS-RSRP in FR1	886

10.1.4B	Inter-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	887

10.1.4B.1	Inter-frequency SS-RSRP accuracy requirements	887

10.1.4B.1.1	Absolute Accuracy of SS-RSRP in FR1	887

10.1.4C	Inter-frequency RSRP accuracy requirements for FR1 SAN	888

10.1.4C.1	Inter-frequency SS-RSRP accuracy requirements	888

10.1.4C.1.1	Absolute Accuracy of SS-RSRP in FR1	888

10.1.4C.1.2	Relative Accuracy of SS-RSRP in FR1	889

10.1.5	Inter-frequency RSRP accuracy requirements for FR2	889

10.1.5.1	Inter-frequency SS-RSRP accuracy requirements	889

10.1.5.1.1	Absolute SS-RSRP Accuracy	889

10.1.5.1.2	Relative SS-RSRP Accuracy	890

10.1.5.2	Void	891

10.1.5.3	Inter-frequency CSI-RSRP accuracy requirements	891

10.1.5.3.1	Absolute CSI-RSRP Accuracy	891

10.1.5.3.2	Relative CSI-RSRP Accuracy	892

10.1.5B	Inter-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	893

10.1.5B.1	Inter-frequency SS-RSRP accuracy requirements	893

10.1.5B.1.1	Absolute SS-RSRP Accuracy	893

10.1.6	RSRP Measurement Report Mapping	894

10.1.7	Intra-frequency RSRQ accuracy requirements for FR1	896

10.1.7.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	896

10.1.7.1.1	Absolute SS-RSRQ Accuracy in FR1	896

10.1.7.2	Intra-frequency CSI-RSRQ accuracy requirements	897

10.1.7.2.1	Absolute CSI-RSRQ Accuracy	897

10.1.7B	Intra-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	898

10.1.7B.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	898

10.1.7B.1.1	Absolute SS-RSRQ Accuracy in FR1	898

10.1.7C	Intra-frequency RSRQ accuracy requirements for FR1 SAN	899

10.1.7C.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	899

10.1.7C.1.1	Absolute SS-RSRQ Accuracy in FR1	899

10.1.8	Intra-frequency RSRQ accuracy requirements for FR2	900

10.1.8.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	900

10.1.8.1.1	Absolute SS-RSRQ Accuracy in FR2	900

10.1.8.2	Intra-frequency CSI-RSRQ accuracy requirements	901

10.1.8.2.1	Absolute CSI-RSRQ Accuracy	901

10.1.8B	Intra-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	901

10.1.8B.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	901

10.1.8B.1.1	Absolute SS-RSRQ Accuracy in FR2	902

10.1.9	Inter-frequency RSRQ accuracy requirements for FR1	902

10.1.9.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	902

10.1.9.1.1	Absolute Accuracy of SS-RSRQ in FR1	902

10.1.9.1.2	Relative Accuracy of SS-RSRQ in FR1	903

10.1.9.2	Inter-frequency CSI-RSRQ accuracy requirements	904

10.1.9.2.1	Absolute CSI-RSRQ Accuracy	904

10.1.9.2.2	Relative CSI-RSRQ Accuracy	905

10.1.9B	Inter-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	906

10.1.9B.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	906

10.1.9B.1.1	Absolute Accuracy of SS-RSRQ in FR1	906

10.1.9C	Inter-frequency RSRQ accuracy requirements for FR1 SAN	907

10.1.9C.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	907

10.1.9C.1.1	Absolute Accuracy of SS-RSRQ in FR1	907

10.1.9C.1.2	Relative Accuracy of SS-RSRQ in FR1	908

10.1.10	Inter-frequency RSRQ accuracy requirements for FR2	909

10.1.10.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	909

10.1.10.1.1	Absolute Accuracy of SS-RSRQ in FR2	909

10.1.10.1.2	Relative Accuracy of SS-RSRQ in FR2	909

10.1.10.2	Inter-frequency CSI-RSRQ accuracy requirements	910

10.1.10.2.1	Absolute CSI-RSRQ Accuracy	910

10.1.10.2.2	Relative CSI-RSRQ Accuracy	911

10.1.10B	 Inter-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	912

10.1.10B.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	912

10.1.10B.1.1	Absolute Accuracy of SS-RSRQ in FR2	912

10.1.11	RSRQ report mapping	913

10.1.12	Intra-frequency SINR accuracy requirements for FR1	913

10.1.12.1	Intra-frequency SS-SINR accuracy requirements in FR1	913

10.1.12.1.1	Absolute SS-SINR Accuracy in FR1	913

10.1.12.2	Intra-frequency CSI-SINR accuracy requirements in FR1	914

10.1.12.2.1	Absolute CSI-SINR Accuracy in FR1	914

10.1.12C	Intra-frequency SINR accuracy requirements for FR1 SAN	915

10.1.12C.1	Intra-frequency SS-SINR accuracy requirements in FR1	915

10.1.12C.1.1	Absolute SS-SINR Accuracy in FR1	915

10.1.13	Intra-frequency SINR accuracy requirements for FR2	916

10.1.13.1	Intra-frequency SS-SINR accuracy requirements in FR2	916

10.1.13.1.1	Absolute SS-SINR Accuracy in FR2	916

10.1.13.2	Intra-frequency CSI-SINR accuracy requirements in FR2	917

10.1.13.2.1	Absolute CSI-SINR Accuracy in FR2	917

10.1.14	Inter-frequency SINR accuracy requirements for FR1	917

10.1.14.1	Inter-frequency SS-SINR accuracy requirements in FR1	917

10.1.14.1.1	Aboslute Accuracy of SS-SINR in FR1	917

10.1.14.1.2	Relative Accuracy of SS-SINR in FR1	918

10.1.14.2	Inter-frequency CSI-SINR accuracy requirements in FR1	919

10.1.14.2.1	Aboslute Accuracy of CSI-SINR in FR1	919

10.1.14.2.2	Relative Accuracy of CSI-SINR in FR1	920

10.1.14C	Inter-frequency SINR accuracy requirements for FR1 SAN	921

10.1.14C.1	Inter-frequency SS-SINR accuracy requirements in FR1	921

10.1.14C.1.1	Aboslute Accuracy of SS-SINR in FR1	921

10.1.14C.1.2	Relative Accuracy of SS-SINR in FR1	922

10.1.15	Inter-frequency SINR accuracy requirements for FR2	923

10.1.15.1	Inter-frequency SS-SINR accuracy requirements in FR2	923

10.1.15.1.1	Aboslute Accuracy of SS-SINR in FR2	923

10.1.15.1.2	Relative Accuracy of SS-SINR in FR2	923

10.1.15.2	Inter-frequency CSI-SINR accuracy requirements in FR2	924

10.1.15.2.1	Aboslute Accuracy of CSI-SINR in FR2	924

10.1.15.2.2	Relative Accuracy of CSI-SINR in FR2	925

10.1.16	SINR report mapping	926

10.1.16.1	SS-SINR and CSI-SINR measurement report mapping	926

10.1.17	Power Headroom	927

10.1.17.1	Power Headroom Report	927

10.1.17.1.1	Power Headroom Report Mapping	927

10.1.18	PCMAX,c,f	928

10.1.18.1	Report Mapping	928

10.1.19	L1-RSRP accuracy requirements for FR1	928

10.1.19.1	SSB based L1-RSRP accuracy requirements	928

10.1.19.1.1	Absolute Accuracy	928

10.1.19.1.2	Relative Accuracy	929

10.1.19.2	CSI-RS based L1-RSRP accuracy requirements	930

10.1.19.2.1	Absolute Accuracy	930

10.1.19.2.2	Relative Accuracy	931

10.1.19C	L1-RSRP accuracy requirements for FR1 SAN	932

10.1.19C.1	SSB based L1-RSRP accuracy requirements	932

10.1.19C.1.1	Absolute Accuracy	932

10.1.19C.1.2	Relative Accuracy	933

10.1.20	L1-RSRP accuracy requirements for FR2	933

10.1.20.1	SSB based L1-RSRP accuracy requirements	933

10.1.20.1.1	Absolute Accuracy	933

10.1.20.1.2	Relative Accuracy	934

10.1.20.2	CSI-RS based L1-RSRP accuracy requirements	935

10.1.20.2.1	Absolute Accuracy	935

10.1.20.2.2	Relative Accuracy	936

10.1.21	SFTD accuracy requirements	937

10.1.21.1	SFTD acuracy requirements for NE-DC	937

10.1.21.2	SFTD acuracy requirements for NR-DC	939

10.1.21.3	Inter frequency SFTD acuracy requirements	940

10.1.22	CLI measurement accuracy requirements	941

10.1.22.1	SRS-RSRP	941

10.1.22.1.1	SRS-RSRP Accuracy	941

10.1.22.1.2	SRS-RSRP report mapping	943

10.1.22.2	CLI-RSSI	943

10.1.22.2.1	CLI-RSSI Accuracy	943

10.1.22.2.2	CLI-RSSI report mapping	944

10.1.23	RSTD Measurements	945

10.1.23.1	Introduction	945

10.1.23.2	Measurement Accuracy Requirements	945

10.1.23.3	Report mapping	955

10.1.23.3.1	Absolute DL RSTD Measurement Reporting	955

10.1.23.3.2	Differential Reporting for DL RSTD Measurement	957

10.1.23.3.3	Additional Path Report Mapping for DL RSTD	959

10.1.24	PRS-RSRP Measurements	961

10.1.24.1	Introduction	961

10.1.24.2	Measurement Accuracy Requirements	961

10.1.24.2.1	Absolute PRS RSRP accuracy	961

10.1.24.2.2	Relative PRS RSRP accuracy	965

10.1.24.3	Report mapping	969

10.1.24.3.1	Absolute PRS-RSRP Measurement Report Mapping	969

10.1.24.3.2	Differential Report Mapping for PRS-RSRP Measurement	970

10.1.25	UE Rx-Tx Time Difference Measurements	972

10.1.25.1	Introduction	972

10.1.25.2	Measurement Accuracy Requirements	972

10.1.25.3	Report mapping	985

10.1.25.3.1	Absolute UE Rx-Tx Measurement Report Mapping	985

10.1.25.3.2	Differential UE Rx-Tx Measurement Report Mapping	987

10.1.25.3.3	Additional Path Report Mapping for UE Rx-Tx Time Difference	988

10.1.26	FR2 P-MPR report	990

10.1.26.1	Report mapping	990

10.1.27	L1-SINR accuracy requirements for FR1	990

10.1.27.1	L1-SINR accuracy requirements with CSI-RS based CMR and no dedicated IMR configured	990

10.1.27.1.1	Absolute Accuracy	990

10.1.27.1.2	Relative Accuracy	991

10.1.27.2	L1-SINR accuracy requirements with SSB based CMR and dedicated IMR configured	992

10.1.27.2.1	Absolute Accuracy	992

10.1.27.2.2	Relative Accuracy	993

10.1.27.3	L1-SINR accuracy requirements with CSI-RS based CMR and dedicated IMR configured	995

10.1.27.3.1	Absolute Accuracy	995

10.1.27.3.2	Relative Accuracy	997

10.1.28	L1-SINR accuracy requirements for FR2	998

10.1.29	Intra-frequency RSRQ accuracy requirements under CCA	1005

10.1.29.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	1005

10.1.29.1.1	Absolute SS-RSRQ Accuracy	1005

10.1.30	Inter-frequency RSRQ accuracy requirements under CCA	1005

10.1.30.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	1005

10.1.30.1.1	Aboslute Accuracy of SS-RSRQ	1005

10.1.30.1.2	Relative Accuracy of SS-RSRQ	1006

10.1.31	Intra-frequency SINR accuracy requirements under CCA	1007

10.1.31.1	Intra-frequency SS-SINR accuracy requirements in FR1	1007

10.1.31.1.1	Absolute SS-SINR Accuracy	1007

10.1.32	Inter-frequency SINR accuracy requirements under CCA	1007

10.1.32.1	Inter-frequency SS-SINR accuracy requirements in FR1	1007

10.1.32.1.1	Aboslute Accuracy of SS-SINR	1007

10.1.32.1.2	Relative Accuracy of SS-SINR	1008

10.1.33	L1-RSRP accuracy requirements under CCA	1009

10.1.33.1	SSB based L1-RSRP accuracy requirements in FR1	1009

10.1.33.1.1	Absolute Accuracy	1009

10.1.33.1.2	Relative Accuracy	1009

10.1.34	RSSI measurements under CCA	1010

10.1.34.1	Intra-frequency absolute RSSI measurement accuracy requirements in FR1	1010

10.1.34.2	Inter-frequency absolute RSSI measurement accuracy requirements in FR1	1010

10.1.34.3	RSSI measurement report mapping	1010

10.1.35	Channel occupancy measurements under CCA	1011

10.1.35.1	Intra-frequency channel occupancy measurement accuracy requirements in FR1	1011

10.1.35.2	Inter-frequency channel occupancy measurement accuracy requirements in FR1	1011

10.1.36	Intra-frequency RSRP accuracy requirements under CCA	1011

10.1.36.1	Intra-frequency SS-RSRP accuracy requirements in FR1	1011

10.1.36.1.1	Absolute SS-RSRP Accuracy	1011

10.1.36.1.2	Relative SS-RSRP Accuracy	1012

10.1.37	Inter-frequency RSRP accuracy requirements under CCA	1013

10.1.37.1	Inter-frequency SS-RSRP accuracy requirements in FR1	1013

10.1.37.1.1	Absolute Accuracy of SS-RSRP	1013

10.1.37.1.2	Relative Accuracy of SS-RSRP	1013

10.1.38	PRS-RSRPP Measurements	1014

10.1.38.1	Introduction	1014

10.1.38.2	Measurement Accuracy Requirements	1014

10.1.38.2.1	Absolute PRS RSRPP accuracy	1014

10.1.38.3	Report mapping	1018

10.1.38.3.1	Absolute PRS-RSRPP Measurement Report Mapping	1018

10.1.38.3.2	Differential Report Mapping for PRS-RSRPP Measurement	1019

10.1.39	UE Rx-Tx time difference measurements for RTT-based PDC	1020

10.1.39.1	 Void	1020

10.1.39.2	 Measurement Accuracy Requirements for PRS	1020

10.1.39.3	 Measurement Accuracy Requirements for TRS	1024

10.1.40	Void	1027

10.1A	NR measurements for RedCap	1027

10.1A.1	Introduction	1027

10.1A.2	Intra-frequency RSRP accuracy requirements for FR1	1027

10.1A.2.1	Intra-frequency SS-RSRP accuracy requirements	1027

10.1A.2.1.1	Absolute SS-RSRP Accuracy	1027

10.1A.2.1.2	Relative SS-RSRP Accuracy	1028

10.1A.3	Intra-frequency RSRP accuracy requirements for FR2	1029

10.1A.3.1	Intra-frequency SS-RSRP accuracy requirements	1029

10.1A.3.1.1	Absolute SS-RSRP Accuracy	1029

10.1A.3.1.2	Relative SS-RSRP Accuracy	1029

10.1A.4	Inter-frequency RSRP accuracy requirements for FR1	1029

10.1A.4.1	Inter-frequency SS-RSRP accuracy requirements	1029

10.1A.4.1.1	Absolute Accuracy of SS-RSRP in FR1	1029

10.1A.4.1.2	Relative Accuracy of SS-RSRP in FR1	1030

10.1A.5	Inter-frequency RSRP accuracy requirements for FR2	1031

10.1A.5.1	Inter-frequency SS-RSRP accuracy requirements	1031

10.1A.5.1.1	Absolute SS-RSRP Accuracy	1031

10.1A.5.1.2	Relative SS-RSRP Accuracy	1031

10.1A.6	Intra-frequency RSRQ accuracy requirements for FR1	1031

10.1A.6.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	1031

10.1A.6.1.1	Absolute SS-RSRQ Accuracy in FR1	1031

10.1A.7	Intra-frequency RSRQ accuracy requirements for FR2	1032

10.1A.7.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	1032

10.1A.7.1.1	Absolute SS-RSRQ Accuracy in FR2	1032

10.1A.8	Inter-frequency RSRQ accuracy requirements for FR1	1032

10.1A.8.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	1032

10.1A.8.1.1	Absolute Accuracy of SS-RSRQ in FR1	1032

10.1A.8.1.2	Relative Accuracy of SS-RSRQ in FR1	1033

10.1A.9	Inter-frequency RSRQ accuracy requirements for FR2	1034

10.1A.9.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	1034

10.1A.9.1.1	Absolute Accuracy of SS-RSRQ in FR2	1034

10.1A.9.1.2	Relative Accuracy of SS-RSRQ in FR2	1034

10.1A.10	 Intra-frequency SINR accuracy requirements for FR1	1034

10.1A.10.1	Intra-frequency SS-SINR accuracy requirements in FR1	1034

10.1A.10.1.1	Absolute SS-SINR Accuracy in FR1	1034

10.1A.11	Intra-frequency SINR accuracy requirements for FR2	1035

10.1A.11.1	Intra-frequency SS-SINR accuracy requirements in FR2	1035

10.1A.11.1.1	Absolute SS-SINR Accuracy in FR2	1035

10.1A.12 	Inter-frequency SINR accuracy requirements for FR1	1035

10.1A.12.1	Inter-frequency SS-SINR accuracy requirements in FR1	1035

10.1A.12.1.1	Aboslute Accuracy of SS-SINR in FR1	1035

10.1A.12.1.2	Relative Accuracy of SS-SINR in FR1	1036

10.1A.13	 Inter-frequency SINR accuracy requirements for FR2	1037

10.1A.13.1	Inter-frequency SS-SINR accuracy requirements in FR2	1037

10.1A.13.1.1	Aboslute Accuracy of SS-SINR in FR2	1037

10.1A.13.1.2	Relative Accuracy of SS-SINR in FR2	1037

10.1A.14	 L1-RSRP accuracy requirements for FR1	1037

10.1A.14.1	SSB based L1-RSRP accuracy requirements	1037

10.1A.14.1.1	Absolute Accuracy	1037

10.1A.14.1.2	Relative Accuracy	1038

10.1A.14.2	CSI-RS based L1-RSRP accuracy requirements	1039

10.1A.14.2.1	Absolute Accuracy	1039

10.1A.14.2.2	Relative Accuracy	1040

10.1A.15	 L1-RSRP accuracy requirements for FR2	1041

10.1A.15.1	SSB based L1-RSRP accuracy requirements	1041

10.1A.15.1.1	Absolute Accuracy	1041

10.1A.15.1.2	Relative Accuracy	1041

10.1A.15.2	CSI-RS based L1-RSRP accuracy requirements	1041

10.1A.15.2.1	Absolute Accuracy	1041

10.1A.15.2.2	Relative Accuracy	1041

10.2	E-UTRAN measurements	1042

10.2.1	Introduction	1042

10.2.2	E-UTRAN RSRP measurements	1042

10.2.3	E-UTRAN RSRQ measurements	1042

10.2.4	E-UTRAN RSTD measurements	1042

10.2.5	E-UTRAN RS-SINR measurements	1043

10.2.6	E-UTRAN RSRP measurements for CA/DC Idle Mode Measurements	1043

10.2.7	E-UTRAN RSRQ measurements for CA/DC Idle Mode Measurements	1043

10.2A	E-UTRAN measurements for RedCap	1044

10.2A.1	Introduction	1044

10.2A.2	E-UTRAN RSRP measurements	1044

10.2A.3	E-UTRAN RSRQ measurements	1044

10.2A.4	E-UTRAN RS-SINR measurements	1045

10.3	UTRAN FDD Measurements	1045

10.3.1	UTRAN FDD CPICH RSCP	1045

10.3.2	UTRAN FDD CPICH Ec/No	1046

10.4	V2X measurements	1046

10.4.1	Introduction	1046

10.4.2	Intra-frequency PSBCH-RSRP accuracy requirements for FR1	1046

10.4.2.1	PSBCH-RSRP Absolute Accuracy	1046

10.4.2.2	PSBCH-RSRP Relative Accuracy	1047

10.4.3	Intra-Frequency SL-RSSI Measurement Accuracy Requirements for FR1	1048

10.4.3.1	Absolute SL-RSSI Accuracy	1048

10.4.4	Intra-Frequency L1 SL-RSRP Measurement Accuracy Requirements for FR1	1048

10.4.4.1	Absolute L1 SL-RSRP Accuracy	1048

10.4.5	Intra-Frequency Discovery Signal Measurement Accuracy Requirements	1049

10.4.5.1	Absolute Discovery Signal Measurement Accuracy	1049

11	Void	1050

12	V2X Requirements	1051

12.1	Introduction	1051

12.2	UE Transmit Timing	1051

12.2.1	Introduction	1051

12.2.2	GNSS as synchronization reference source	1051

12.2.3	NR Cell as synchronization reference source	1052

12.2.4	E-URTAN Cell as synchronization reference source	1052

12.2.5	SyncRef UE as synchronization reference source	1052

12.3	Initiation/Cease of SLSS Transmissions	1053

12.3.1	Introduction	1053

12.3.1.1	Initiation/Cease of SLSS transmissions with NR cell as synchronization reference source	1053

12.3.1.2	Initiation/Cease of SLSS transmissions with EUTRAN cell as synchronization reference source	1054

12.3.1.3	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	1055

12.3.1.4	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	1055

12.4	Selection / Reselection of V2X Synchronization Reference Source	1056

12.5	L1 SL-RSRP measurements	1057

12.5.1	Introduction	1057

12.5.2	SL-RSRP measurements	1057

12.6	Congestion Control measurements	1058

12.7	Interruption	1058

12.7.1	Interruptions to WAN due to V2X Sidelink Communication	1058

12.7.2	V2X Sidelink Communication Dropping due to synchronization source change	1059

12.7.3	Interruptions to WAN due to switching between E-UTRA V2X Sidelink and NR V2X Sidelink	1060

12.7.4	Interruptions to WAN at transitions between active and non-active during SL-DRX	1061

12.7.5	Interruptions to V2X sidelink at transitions between active and non-active during DRX	1061

12.7.6	Interruptions to V2X sidelink due to Active BWP switching Requirement	1062

12.7.7	Interruptions to WAN due to SyncRef UE detection and/or Sensing during SL DRX off duration	1062

12.7.8	Interruptions at NR sidelink discovery configuration	1062

12.8	Reliability of GNSS signal	1063

12.9	Scheduling availability	1063

12.9.1	Scheduling availability of UE switching between E-UTRA sidelink and NR sidelink	1063

12.9.2	Scheduling availability of UE switching between Uu uplink  and V2X sidelink	1063

12.10	Selection / Reselection of relay UE	1064

12.10.1	Introduction	1064

12.10.2	Selection / Reselection of relay UE	1064

13	Measurement Performance Requirements for NR gNB	1064

13.1	UL-RTOA	1064

13.1.1	Report mapping	1064

13.1.1A	Additional Path Report Mapping for UL-RTOA	1066

13.2	gNB Rx-Tx time difference	1068

13.2.1	Report mapping	1068

13.2.1A	 Additional Path Report Mapping for gNB Rx-Tx	1070

13.2.2	Measurement Accuracy Requirements	1072

13.2.2.1	Introduction	1072

13.3	UL SRS RSRP measurement	1073

13.3.1	Report mapping	1073

13.3.2	Measurement accuracy requirements	1074

13.3.2.1	Introduction	1074

13.3.2.2	Requirements	1074

13.4	AoA/ZoA	1075

13.4.1	Report mapping	1075

13.5	Timing advance (TADV)	1076

13.5.1	Report mapping	1076

13.6	UL SRS RSRPP measurement	1077

13.6.1	Report mapping	1077

13.7	gNB Rx-Tx time difference measurements for RTT-based PDC	1079

13.7.1	Report mapping	1079

13.7.2	Measurement Accuracy Requirements	1079

13.7.2.1	Introduction	1079

13.7.2.2	Requirements	1079

Annex A (normative): Test Cases	1082

A.1	Purpose of annex	1082

A.2	Requirement classification for statistical testing	1082

A.2.1	Types of requirements in TS 38.133	1082

A.2.1.1	Time and delay requirements on UE higher layer actions	1082

A.2.1.2	Measurements of power levels, relative powers and time	1082

A.2.1.3	Implementation requirements	1083

A.2.1.4	Physical layer timing requirements	1083

A.2.1.5	Requirements under CCA	1083

A.3	RRM test configurations	1084

A.3.1	Reference measurement channels	1084

A.3.1.1	PDSCH	1084

A.3.1.1.1	FDD	1084

A.3.1.1.2	TDD	1085

A.3.1.2	CORESET for RMSI scheduling	1088

A.3.1.2.1	FDD	1088

A.3.1.2.2	TDD	1089

A.3.1.3	CORESET for RMC scheduling	1092

A.3.1.3.1	FDD	1092

A.3.1.3.2	TDD	1093

A.3.1.4	TDD UL/DL configuration	1096

A.3.1A	Reference measurement channels under CCA	1098

A.3.1A.1	PDSCH	1098

A.3.1A.1.1	TDD	1098

A.3.1A.2	CORESET for RMSI scheduling	1099

A.3.1A.2.1	TDD	1099

A.3.1A.3	CORESET for RMC scheduling	1100

A.3.1A.3.1	TDD	1100

A.3.1A.4	TDD UL/DL configuration	1101

A.3.1A.5	RMC burst transmission model	1101

A.3.2.1	Generic OFDMA Channel Noise Generator (OCNG)	1101

A.3.2.1.1	OCNG pattern 1: Generic OCNG pattern for all unused REs	1102

A.3.2.1.2	OCNG pattern 2: Generic OCNG pattern for all unused REs for 2AoA setup	1102

A.3.2.1.3	OCNG pattern 3: Generic OCNG pattern for unused REs in the same bandwidth as CORESET	1103

A.3.2.1.4	OCNG pattern 4: Generic OCNG pattern for all unused REs outside SSB slot(s)	1103

A.3.2.2	Void	1104

A.3.3	Reference DRX configurations	1104

A.3.3.1	DRX Configuration 1: DRX cycle = 40 ms and TAT = 500 ms	1104

A.3.3.2	DRX Configuration 2: DRX cycle = 640 ms and TAT = 500 ms	1105

A.3.3.3	DRX Configuration 3: DRX cycle = 40 ms and TAT = Infinity	1105

A.3.3.4	DRX Configuration 4: DRX cycle = 160 ms and TAT = Infinity	1105

A.3.3.5	DRX Configuration 5: DRX cycle = 320 ms and TAT = Infinity	1106

A.3.3.6	DRX Configuration 6: DRX cycle = 320 ms and TAT = 500 ms	1106

A.3.3.7	DRX Configuration 7: DRX cycle = 640 ms and TAT = Infinity	1106

A.3.3.8	DRX Configuration 8: DRX cycle = 320 ms and TAT = Infinity	1107

A.3.3.9	DRX Configuration 9: DRX cycle = 40 ms and TAT = 500 ms	1107

A.3.3.10	DRX Configuration 10: DRX cycle = 640 ms and TAT = 500 ms	1107

A.3.3.11	DRX Configuration 11: DRX cycle = 20 ms and TAT = Infinity	1108

A.3.3.12	DRX Configuration 12: DRX cycle = 640 ms and TAT = Infinity	1108

A.3.3.13	DRX Configuration X1: DRX cycle = 80 ms and TAT = Infinity	1108

A.3.4	Test Cases with Different Channel Bandwidths	1109

A.3.4.1	Test Cases with Different E-UTRA Channel Bandwidths	1109

A.3.4.1.1	Introduction	1109

A.3.4.1.2	Principle of testing	1109

A.3.5	Test Cases for Synchronous and Asynchronous DC Operations	1109

A.3.5.1	EN-DC Test Cases for Synchronous and Asynchronous EN-DC Operations	1109

A.3.5.1.1	Introduction	1109

A.3.5.1.2	Principle of Testing	1109

A.3.6	Antenna configurations	1109

A.3.6.1	Antenna configurations for FR1	1109

A.3.6.1.1	Antenna connection for 4 Rx capable UEs	1109

A.3.6.1.1.1	Introduction	1109

A.3.6.1.1.2	Principle of testing	1110

A.3.6.2	Antenna configurations for FR2	1113

A.3.6A	Antenna configurations with unlicensed bands	1113

A.3.6A.1	Antenna configurations for FR1	1113

A.3.6A.1.1	Antenna connection for 4 Rx capable UEs	1113

A.3.6A.1.1.1	Introduction	1113

A.3.6A.1.1.2	Principle of testing	1113

A.3.7	EN-DC test setup	1115

A.3.7.1	Introduction	1115

A.3.7.2	E-UTRAN Serving Cell Parameters	1115

A.3.7.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	1115

A.3.7.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	1116

A.3.7A	NR FR1-FR2 test setup	1117

A.3.7B	EN-DC test setup with unlicensed bands	1118

A.3.7B.1	Introduction	1118

A.3.7B.2	E-UTRAN Serving Cell Parameters	1118

A.3.7B.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) under CCA in FR1	1118

A.3.7C	LTE-FR1/FR2 test setup	1119

A.3.7D	NE-DC test setup	1119

A.3.7D.1	Introduction	1119

A.3.7D.2	E-UTRAN Serving Cell Parameters	1119

A.3.7D.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	1119

A.3.7D.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	1119

A.3.8	PRACH configurations	1120

A.3.8.1	Introduction	1120

A.3.8.2	PRACH configurations in FR1	1120

A.3.8.2.1	FR1 PRACH configuration 1	1120

A.3.8.2.2	FR1 PRACH configuration 2	1120

A.3.8.2.3	FR1 PRACH configuration 3	1121

A.3.8.2.4	FR1 PRACH configuration 4	1122

A.3.8.3	PRACH configurations in FR2	1123

A.3.8.3.1	FR2 PRACH configuration 1	1123

A.3.8.3.2	FR2 PRACH configuration 2	1124

A.3.8.3.3	FR2 PRACH configuration 3	1125

A.3.8.3.4	FR2 PRACH configuration 4	1126

A.3.8A	PRACH configurations under CCA	1127

A.3.8A.1	Introduction	1127

A.3.8A.2	PRACH configurations in FR1	1127

A.3.8A.2.1	FR1 PRACH configuration 1 under CCA	1127

A.3.8A.2.2	FR1 PRACH configuration 2 under CCA	1128

A.3.9	BWP configurations	1129

A.3.9.1	Introduction	1129

A.3.9.2	Downlink BWP configurations	1130

A.3.9.2.1	Initial BWP	1130

A.3.9.2.2	Dedicated BWP	1130

A.3.9.3	Uplink BWP configurations	1131

A.3.9.3.1	Initial BWP	1131

A.3.9.3.2	Dedicated BWP	1131

A.3.9A	BWP configurations for RedCap	1131

A.3.9A.1	Introduction	1131

A.3.9A.2	Downlink BWP configurations	1132

A.3.9A.2.1	Dedicated BWP	1132

A.3.9A.3	Uplink BWP configurations	1132

A.3.9A.3.1	Dedicated BWP	1132

A.3.10	SSB Configurations	1133

A.3.10.1	SSB Configurations for FR1	1133

A.3.10.1.1	SSB pattern 1 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1133

A.3.10.1.5	SSB pattern 5 in FR1: SSB allocation for SSB SCS=15 kHz starting from odd SFN in 10 MHz	1135

A.3.10.1.6	SSB pattern 6 in FR1: SSB allocation for SSB SCS=30 kHz starting from odd SFN in 40 MHz	1135

A.3.10.1.7	SSB pattern 7 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1136

A.3.10.1.8	SSB pattern 8 in FR1: SSB allocation for SSB SCS=30 kHz in 40 MHz	1136

A.3.10.2	SSB Configurations for FR2	1137

A.3.10.2.1	SSB pattern 1 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1137

A.3.10.2.2	SSB pattern 2 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1137

A.3.10.2.3	SSB pattern 3 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1138

A.3.10.2.4	SSB pattern 4 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1138

A.3.10.2.5	SSB pattern 5 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1139

A.3.10.2.6	SSB pattern 6 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1139

A.3.10.2.7	SSB pattern 7 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1140

A.3.10.2.8	SSB pattern 8 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1140

A.3.10.2.9	SSB pattern 9 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1141

A.3.10.2.10	SSB pattern 10 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1141

A.3.10A	SSB Configurations under CCA	1146

A.3.10A.1	SSB Configurations under CCA for FR1	1146

A.3.10A.1.1	SSB pattern 1 under CCA for semi-static channel access: SSB allocation for SSB SCS=30kHz in 40MHz	1146

A.3.10A.1.2	SSB pattern 2 under CCA for dynamic channel access: SSB allocation for SSB SCS=30kHz in 40MHz	1146

A.3.10A.1.3	SSB pattern 3 under CCA for semi-static channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	1147

A.3.10A.1.4	SSB pattern 4 under CCA for dynamic channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	1147

A.3.10B	SSB Configurations for RedCap	1148

A.3.10B.1	SSB Configurations for FR1	1148

A.3.10B.1.1	SSB pattern 1 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1148

A.3.10B.1.2	SSB pattern 2 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1148

A.3.10B.1.3	SSB pattern 3 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz starting from odd SFN in 20 MHz	1149

A.3.10B.1.4	SSB pattern 4 for RedCap in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1149

A.3.10B.1.5	SSB pattern 5 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1150

A.3.10B.1.6	SSB pattern 6 for RedCap in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1150

A.3.10B.1.7	SSB pattern 7 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1151

A.3.10B.2	SSB Configurations for FR2	1151

A.3.10B.2.1	SSB pattern 1 for RedCap in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1151

A.3.10B.2.2	SSB pattern 2 for RedCap in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1152

A.3.10B.2.3	SSB pattern 3 for RedCap in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1152

A.3.10B.2.4	SSB pattern 4 for RedCap in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1153

A.3.10B.2.5	SSB pattern 5 for RedCap in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1153

A.3.11	SMTC Configurations	1154

A.3.11.1	SMTC pattern 1: SMTC period = 20 ms with SMTC duration = 1 ms	1154

A.3.11.2	SMTC pattern 2: SMTC period = 20 ms with SMTC duration = 5 ms	1154

A.3.11.3	SMTC pattern 3: SMTC period = 160 ms with SMTC duration = 1 ms	1154

A.3.11.4	SMTC pattern 4: SMTC period = 20 ms with SMTC duration = 1 ms	1154

A.3.11.5	SMTC pattern 5: SMTC period = 20 ms with SMTC duration = 5 ms	1155

A.3.11.6	SMTC pattern 6: SMTC period = 20 ms with SMTC duration = 5 ms	1155

A.3.11.7	SMTC pattern 7: SMTC period = 20 ms with SMTC duration = 5 ms	1155

A.3.11.8	SMTC pattern 8: SMTC period = 10 ms with SMTC duration = 1 ms	1155

A.3.11.9	SMTC pattern 9: SMTC period = 20 ms with SMTC duration = 1 ms	1155

A.3.11A	SMTC Configurations for RedCap	1156

A.3.11A.1	SMTC pattern 1 for RedCap: SMTC period = 40 ms with SMTC duration = 1 ms	1156

A.3.11A.2	SMTC pattern 2 for RedCap: SMTC period = 80 ms with SMTC duration = 1 ms	1156

A.3.11A.3	SMTC pattern 3 for RedCap: SMTC period = 40 ms with SMTC duration = 1 ms	1156

A.3.11A.4	SMTC pattern 4 for RedCap: SMTC period = 80 ms with SMTC duration = 5 ms	1156

A.3.12	Test Cases with Different CC Configurations	1156

A.3.12.1 EN-DC Test Cases with Different EN-DC Configurations	1156

A.3.12.1.1	Introduction	1156

A.3.12.1.2	Principle of testing	1157

A.3.12.2	Carrier Aggregation Test Cases with Different CA Configurations	1157

A.3.12.2.1	Introduction	1157

A.3.12.2.2	Principle of testing	1157

A.3.13	Test Cases in SA and EN-DC Operations	1157

A.3.13.1	Introduction	1157

A.3.13.2	Principle of Testing	1157

A.3.13A	Test Cases involving E-UTRA/FR1 and FR2 carriers	1158

A.3.13A.1	Introduction	1158

A.3.13A.2	Principle of Testing in EN-DC	1158

A.3.13A.3	Principle of Testing in SA	1158

A.3.13A.4	Principle of Testing in E-UTRA	1159

A.3.13B	Test Cases for EN-DC and NE-DC Operations	1160

A.3.13B.1	Active BWP switch Test Cases for EN-DC and NE-DC Operations	1160

A.3.13B.1.1	Introduction	1160

A.3.13B.1.2	Principle of Testing	1160

A.3.13B.2	SFTD accuracy Test Cases for EN-DC and NE-DC Operations	1160

A.3.13B.2.1	Introduction	1160

A.3.13B.2.2	Principle of Testing	1160

A.3.14	CSI-RS configurations	1161

A.3.14.1	FDD	1161

A.3.14.2	TDD	1163

A.3.15	Angle of Arrival (AoA) for FR2 RRM test cases	1169

A.3.15.1	Setup 1: Single AoA in Rx beam peak direction	1169

A.3.15.2	Setup 2: Single AoA in non Rx beam peak direction	1170

A.3.15.2.1	Setup 2a: Single AoA in non Rx beam peak direction without change in direction	1170

A.3.15.2.2	Setup 2b: Single AoA in non Rx beam peak direction with change in direction	1170

A.3.15.3	Setup 3: 2 AoAs	1170

A.3.15.4	Setup 4: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak	1170

A.3.15.4.1	Setup 4a: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak without change in direction	1170

A.3.15.4.2	Setup 4b: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak with change in direction	1171

A.3.16	TCI State Configuration	1171

A.3.16.1	Introduction	1171

A.3.16.2	TCI states	1171

A.3.16A	Unified TCI State Configuration	1171

A.3.16A.1	Introduction	1171

A.3.16A.2	DLorJoint TCI states	1172

A.3.16A.3	UL TCI states	1172

A.3.17	Configurations of CSI-RS for tracking	1173

A.3.17.1	Configuration of CSI-RS for tracking for FR1	1173

A.3.17.1.1	FDD	1173

A.3.17.1.2	TDD	1175

A.3.17.2	Configuration of CSI-RS for tracking for FR2	1177

A.3.17.2.1	TDD	1177

A.3.18	Additional definitions related to OTA testing for FR2 RRM test cases	1178

A.3.18.1	Introduction	1178

A.3.18.2	PRACH Power Measurement	1178

A.3.19	Test applicability for DAPS handover	1178

A.3.19.1	Introduction	1178

A.3.19.2	Principle of testing	1178

A.3.20	MsgA configurations	1179

A.3.20.1	Introduction	1179

A.3.20.2	MsgA configurations in FR1	1179

A.3.20.2.1	FR1 MsgA configuration 1	1179

A.3.20.2.2	FR1 MsgA configuration 2	1180

A.3.20.3	MsgA configurations in FR2	1182

A.3.20.3.1	FR2 MsgA configuration 1	1182

A.3.20.3.2	FR2 MsgA configuration 2	1183

A.3.20A	MsgA configurations under CCA	1185

A.3.20A.1	Introduction	1185

A.3.20A.2	MsgA configurations in FR1	1185

A.3.20A.2.1	FR1 MsgA configuration 1 under CCA	1185

A.3.20A.2.2	FR1 MsgA configuration 2 under CCA	1186

A.3.21	V2X sidelink communication	1189

A.3.21.1	Introduction	1189

A.3.21.2	Reference resource pool configurations for V2X Sidelink Communication	1189

A.3.21.3	Reference measurement channels for V2X Sidelink Communication	1193

A.3.21.4	Reference SL-DRX configurations	1194

A.3.21.4.1	SL-DRX Configuration 1: SL-DRX cycle = 40 ms	1194

A.3.21.4.2	SL-DRX Configuration 2: SL-DRX cycle = 320 ms	1194

A.3.21.4.3	SL-DRX Configuration 3: SL-DRX cycle = 640 ms	1194

A.3.22	CSI-IM configurations	1194

A.3.22.1	FDD	1194

A.3.22.2	TDD	1195

A.3.23	Spatial Relation Configuration	1196

A.3.23.1	Introduction	1196

A.3.23.2	Spatial Relation	1197

A.3.24	SRS configuration	1198

A.3.25	Channel bandwidth (CBW) configurations	1200

A.3.25.1	DL UE specific CBW	1200

A.3.25.2	UL UE specific CBW	1201

A.3.26	CCA model	1201

A.3.26.1	Introduction	1201

A.3.26.2	CCA model for operation on a carrier frequency with CCA in FR1	1201

A.3.26.2.1	DL CCA model	1201

A.3.26.2.2	UL CCA model	1203

A.3.26.3	CCA model for operation on a carrier frequency with CCA in FR2-2	1203

A.3.26.3.1	DL CCA model	1203

A.3.26.3.2	UL CCA model	1204

A.3.27	Test Cases with at Least One Cell on a Carrier Frequency with CCA	1204

A.3.27.1	Introduction	1204

A.3.27.2	 NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	1204

A.3.27.3	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	1205

A.3.27.4	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	1205

A.3.27.5	E-UTRA Standalone Tests with at Least One NR Cell under CCA	1205

A.3.28	Discovery Burst Transmission Window configuration under CCA	1205

A.3.28.1	DBT Window pattern 1: DBT Window period = 20 ms with DBT Window duration = 1 ms	1205

A.3.29	Testing principles for UE capable of only NR bands with shared spectrum access	1205

A.3.29.1	Introduction	1205

A.3.29.2	Principle of testing for UE capable of EN-DC with only NR bands with shared spectrum access	1205

A.3.29.3	Principle of testing for UE capable of SA operation with only NR bands with shared spectrum access	1206

A.3.30	CSI-RS configurations for RRM	1207

A.3.30.1	FDD	1207

A.3.30.2	TDD	1208

A.3.31	PRS Configurations	1210

A.3.31.1.	PRS Configurations for FR1	1210

A.3.31.1.1.	PRS pattern 1 in FR1: SCS=15 KHz	1210

A.3.31.1.2.	PRS pattern 2 in FR1: SCS=30 KHz	1211

A.3.31.2.	PRS Configurations for FR2	1211

A.3.31.2.1.	PRS pattern 1 in FR2: SCS=120 KHz	1211

A.3.32	NR sidelink discovery	1211

A.3.32.1	Introduction	1211

A.3.32.2	Reference resource pool configurations for NR Sidelink Discovery	1211

A.3.32.3	Principle of Testing	1212

A.3.33	PRS Processing Window (PPW) configurations	1212

A.3.34	Testing principles for test cases related to PRS measurements	1212

A.3.34.1	Introduction	1212

A.3.34.2	Test cases in RRC_INACTIVE state	1213

A.3.34.3	Test cases for PRS measurements with gaps in RRC_CONNECTED state	1213

A.3.34.4	Test cases for PRS measurements without gaps in RRC_CONNECTED state	1213

A.3.35	Testing principle for RedCap UE	1214

A.3.35.1	Introduction	1214

A.3.35.2	Principle of testing for FR1	1214

A.3.35.3	Principle of testing for FR2	1214

A.3.36	Testing related to Satellite access	1214

A.3.36.1	Introduction	1214

A.3.36.2	Principle of testing GSO and NGSO scenarios	1214

A.3.36.2	Principle of testing different RRM requirements	1215

A.3.36.3	Principle of testing different ephemeris formats	1215

A.3.36.4	General setup for SIB19	1217

A.3.36.5	Satellite specific parameters configuration	1217

A.3.36.5.1	Satellite specific configuration for serving cell	1217

A.3.36.5.2	Satellite specific configuration for neighbour cell	1218

A.4	EN-DC tests with all NR cells in FR1	1219

A.4.1	Void	1219

A.4.2	Void	1219

A.4.3	RRC_CONNECTED state mobility	1219

A.4.3.1	Void	1219

A.4.3.2	RRC Connection Mobility Control	1219

A.4.3.2.1	Void	1219

A.4.3.2.2	Random Access	1219

A.4.3.2.2.1	4-step RA type contention based random access test in FR1 for PSCell in EN-DC	1219

A.4.3.2.2.2	4-step RA type n on-contention based random access test in FR1 for PSCell in EN-DC	1223

A.4.3.2.2.3	2-step RA type contention based random access test in FR1 for PSCell in EN-DC	1227

A.4.3.2.2.4	2-step RA type n on-contention based random access test in FR1 for PSCell in EN-DC	1231

A.4.3.2.3	Void	1235

A.4.3.3	Handover with PSCell from EN-DC to EN-DC with known target PSCell in FR1	1235

A.4.3.3.1	Test Purpose and Environment	1235

A.4.3.3.2	Test Requirements	1241

A.4.4	Timing	1242

A.4.4.1	UE transmit timing	1242

A.4.4.1.1	NR UE Transmit Timing Test for FR1	1242

A.4.4.1.1.1	Test Purpose and environment	1242

A.4.4.1.1.2	Test requirements	1246

A.4.4.2	UE timer accuracy	1246

A.4.4.3	Timing advance	1246

A.4.4.3.1	EN-DC FR1 timing advance adjustment accuracy	1246

A.4.4.3.1.1	Test Purpose and Environment	1246

A.4.4.3.1.2	Test Parameters	1246

A.4.4.3.1.3	Test Requirements	1250

A.4.5	Signaling characteristics	1250

A.4.5.1	Radio link Monitoring	1250

A.4.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	1251

A.4.5.1.1.1	Test Purpose and Environment	1251

A.4.5.1.1.2	Test Requirements	1255

A.4.5.1.2	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	1255

A.4.5.1.2.1	Test Purpose and Environment	1255

A.4.5.1.2.2	Test Requirements	1261

A.4.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	1261

A.4.5.1.3.1	Test Purpose and Environment	1261

A.4.5.1.3.2	Test Requirements	1266

A.4.5.1.4	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	1266

A.4.5.1.4.1	Test Purpose and Environment	1266

A.4.5.1.4.2	Test Requirements	1272

A.4.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	1272

A.4.5.1.5.1	Test Purpose and Environment	1272

A.4.5.1.5.2	Test Requirements	1277

A.4.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	1277

A.4.5.1.6.1	Test Purpose and Environment	1277

A.4.5.1.6.2	Test Requirements	1283

A.4.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	1283

A.4.5.1.7.1	Test Purpose and Environment	1283

A.4.5.1.7.2	Test Requirements	1288

A.4.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	1289

A.4.5.1.8.1	Test Purpose and Environment	1289

A.4.5.1.8.2	Test Requirements	1294

A.4.5.1.9	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion	1294

A.4.5.1.9.1	Test Purpose and Environment	1294

A.4.5.2	Interruption	1300

A.4.5.2.1	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	1300

A.4.5.2.1.1	Test Purpose and Environment	1300

A.4.5.2.1.2	Test Requirements	1304

A.4.5.2.2	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	1304

A.4.5.2.2.1	Test Purpose and Environment	1304

A.4.5.2.2.2	Test Requirements	1308

A.4.5.2.3	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	1308

A.4.5.2.3.1	Test Purpose and Environment	1308

A.4.5.2.3.2	Test Requirements	1313

A.4.5.2.4	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	1314

A.4.5.2.4.1	Test Purpose and Environment	1314

A.4.5.2.4.2	Test Requirements	1318

A.4.5.2.5	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	1319

A.4.5.2.5.1	Test Purpose and Environment	1319

A.4.5.2.5.2	Test Requirements	1323

A.4.5.2.6	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	1323

A.4.5.2.6.1	Test Purpose and Environment	1323

A.4.5.2.6.2	Test Requirements	1327

A.4.5.2.7	Void	1327

A.4.5.2.8	E-UTRAN - NR FR1 interruptions at NR SRS carrier based switching in asynchronous EN-DC	1327

A.4.5.2.8.1	Test Purpose and Environment	1327

A.4.5.2.8.2	Test Requirements	1330

A.4.5.2.9	E-UTRAN – NR interruptions at E-UTRA SRS carrier based switching	1331

A.4.5.2.9.1	Test Purpose and Environment	1331

A.4.5.2.9.2	Test Requirements	1334

A.4.5.2.10	E-UTRAN – NR FR1 interruptions due to RRM and RLM/BFD measurements on deactivated NR PSCell	1335

A.4.5.2.10.1	Test Purpose and Environment	1335

A.4.5.2.10.2	Test Requirements	1338

A.4.5.2.11	E-UTRAN - NR FR1 interruptions at NR SRS antenna port switching with 1 SRS symbol in a slot in synchronous EN-DC	1338

A.4.5.2.11.1	Test Purpose and Environment	1338

A.4.5.2.11.2	Test Requirements	1343

A.4.5.2.12	E-UTRAN - NR FR1 interruptions at NR SRS antenna port switching in asynchronous EN-DC	1344

A.4.5.2.12.1	Test Purpose and Environment	1344

A.4.5.3	SCell Activation and Deactivation Delay	1351

A.4.5.3.1	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	1351

A.4.5.3.1.1	Test Purpose and Environment	1351

A.4.5.3.1.2	Test Requirements	1357

A.4.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 640ms SCell measurement cycle	1358

A.4.5.3.2.1	Test Purpose and Environment	1358

A.4.5.3.2.2	Test Requirements	1358

A.4.5.3.3	SCell Activation and deactivation of unknown SCell in FR1	1358

A.4.5.3.3.1	Test Purpose and Environment	1358

A.4.5.3.3.2	Test Requirements	1359

A.4.5.3.4	SCell Activation and deactivation of multiple unknown SCells in FR1 with single activation/deactivation command	1360

A.4.5.3.4.1	Test Purpose and Environment	1360

A.4.5.3.3.2	Test Requirements	1362

A.4.5.3.5	Direct SCell activation at SCell addition of known SCell in FR1	1362

A.4.5.3.5.1	Test Purpose and Environment	1362

A.4.5.3.5.2	Test Requirements	1370

A.4.5.3.6	Fast SCell Activation of known SCell in FR1 for 160ms SCell measurement cycle	1371

A.4.5.3.6.1	Test Purpose and Environment	1371

A.4.5.3.6.2	Test Requirements	1375

A.4.5.3.7	Fast SCell Activation of known SCell in FR1 for 640 ms SCell measurement cycle	1376

A.4.5.3.7.1	Test Purpose and Environment	1376

A.4.5.3.7.2	Test Requirements	1376

A.4.5.4	UE UL carrier RRC reconfiguration Delay	1377

A.4.5.4.1	UE UL carrier RRC reconfiguration Delay	1377

A.4.5.4.1.1 Test Purpose and Environment	1377

A.4.5.4.1.2	Test Requirements	1387

A.4.5.5	Beam Failure Detection and Link recovery procedures	1387

A.4.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	1387

A.4.5.5.1.1	Test Purpose and Environment	1387

A.4.5.5.1.2	Test Requirements	1393

A.4.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	1394

A.4.5.5.2.1	Test Purpose and Environment	1394

A.4.5.5.2.2	Test Requirements	1399

A.4.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	1400

A.4.5.5.3.1	Test Purpose and Environment	1400

A.4.5.5.3.2	Test Requirements	1405

A.4.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in DRX mode	1406

A.4.5.5.4.1	Test Purpose and Environment	1406

A.4.5.5.4.2	Test Requirements	1411

A.4.5.5.5	EN-DC Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	1412

A.4.5.5.5.1	Test Purpose and Environment	1412

A.4.5.5.5.2	Test Requirements	1417

A.4.5.5.6	EN-DC Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode	1418

A.4.5.5.6.1	Test Purpose and Environment	1418

A.4.5.5.6.2	Test Requirements	1424

A.4.5.5.7	EN-DC TRP specific Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	1425

A.4.5.5.7.1	Test Purpose and Environment	1425

A.4.5.5.7.2	Test Requirements	1430

A.4.5.5.8	EN-DC TRP specific Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	1431

A.4.5.5.8.1	Test Purpose and Environment	1431

A.4.5.5.8.2	Test Requirements	1437

A.4.5.6	Active BWP switch	1438

A.4.5.6.1	DCI-based and Timer-based Active BWP Switch	1438

A.4.5.6.1.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	1438

A.4.5.6.1.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	1443

A.4.5.6.2	RRC-based Active BWP Switch	1449

A.4.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1453

A.4.5.6.3.1	Simultaneous E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in EN-DC on multiple CCs	1453

A.4.5.6.3.1.1	Test Purpose and Environment	1453

A.4.5.6.4	Simultaneous RRC-based Active BWP Switch on multiple CCs	1459

A.4.5.6.4.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC on multiple CCs	1459

A.4.5.6.4.1.1	Test Purpose and Environment	1459

A.4.5.6.4.1.2	Test Requirements	1466

A.4.5.6.4.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR1 SCells inside active time	1466

A.4.5.6.4.2.1	Test Purpose and Environment	1466

A.4.5.6.4.2.2	Test Requirements	1477

A.4.5.6.5	SCell dormancy switch	1478

A.4.5.6.5.1	E-UTRAN – NR FR1 PSCell SCell dormancy switch of single FR1 SCell outside active time	1478

A.4.5.6.5.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR1 SCells inside active time	1483

A.4.5.6.5.2.1	Test Purpose and Environment	1483

A.4.5.6.5.2.2	Test Requirements	1489

A.4.5.7	PSCell addition and release delay	1489

A.4.5.7.1	Addition and Release Delay of known NR PSCell	1489

A.4.5.7.1.1	Test purpose and environment	1489

A.4.5.7.1.2	Test Requirements	1494

A.4.5.8	DL Interruptions at switching between two uplink carriers	1495

A.4.5.8.1	Test Purpose and Environment	1495

A.4.5.8.2	Test Requirements	1498

A.4.5.9	UE specific CBW change	1499

A.4.5.9.1	UE specific CBW change on FR1 NR PSCell with non-DRX in synchronous EN- DC	1499

A.4.5.9.1.1	Test Purpose and Environment	1499

A.4.5.9.1.2	Test Requirements	1503

A.4.5.10	PSCell activation and deactivation delay	1503

A.4.5.10.1	PSCell activation and deactivation delay	1503

A.4.5.10.1.1	Test purpose and environment	1503

A.4.5.10.1.2	Test Requirements	1507

A.4.5.11	Conditional PSCell addition and release delay (FR1 EN-DC)	1508

A.4.5.11.1	Conditional PSCell Addition and Release Delay	1508

A.4.5.11.1.1	Test purpose and environment	1508

A.4.5.11.1.2	Test Parameters	1508

A.4.5.11.1.3	Test Requirements	1512

A.4.6	Measurement procedure	1512

A.4.6.1	Intra-frequency Measurements	1512

A.4.6.1.1	EN-DC event triggered reporting tests without gap under non-DRX	1512

A.4.6.1.1.1	Test purpose and Environment	1512

A.4.6.1.1.2	Test parameters	1512

A.4.6.1.1.3	Test Requirements	1516

A.4.6.1.2	EN-DC event triggered reporting tests without gap under DRX	1516

A.4.6.1.2.1	Test purpose and Environment	1516

A.4.6.1.2.2	Test parameters	1516

A.4.6.1.2.2	Test Requirements	1520

A.4.6.1.3	EN-DC event triggered reporting tests with per-UE gaps under non-DRX	1520

A.4.6.1.3.1	Test purpose and Environment	1520

A.4.6.1.3.2	Test parameters	1520

A.4.6.1.3.3	Test Requirements	1524

A.4.6.1.4	EN-DC event triggered reporting tests with per-UE gaps under DRX	1524

A.4.6.1.4.1	Test purpose and Environment	1524

A.4.6.1.4.2	Test parameters	1524

A.4.6.1.4.3	Test Requirements	1528

A.4.6.1.5	EN-DC event triggered reporting tests without gap under non-DRX with SSB index reading	1528

A.4.6.1.5.1	Test purpose and Environment	1528

A.4.6.1.5.2	Test parameters	1528

A.4.6.1.5.3	Test Requirements	1530

A.4.6.1.6	EN-DC event triggered reporting tests with SSB index reading with per-UE gaps	1531

A.4.6.1.6.1	Test purpose and Environment	1531

A.4.6.1.6.2	Test parameters	1531

A.4.6.1.6.3	Test Requirements	1533

A.4.6.1.7	EN-DC event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	1534

A.4.6.1.7.1	Test purpose and Environment	1534

A.4.6.1.7.2	Test parameters	1534

A.4.6.1.7.3	Test Requirements	1538

A.4.6.1.8	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used for UE configured with highSpeedMeasCA-Scell-r17	1538

A.4.6.1.8.1	Test Purpose and Environment	1538

A.4.6.1.8.2	Test Requirements	1543

A.4.6.2	Inter-frequency Measurements	1544

A.4.6.2.1	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	1544

A.4.6.2.1.1	Test Purpose and Environment	1544

A.4.6.2.1.2	Test Requirements	1548

A.4.6.2.2	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	1548

A.4.6.2.2.1	Test Purpose and Environment	1548

A.4.6.2.2.2	Test Requirements	1552

A.4.6.2.3	Void	1552

A.4.6.2.4	Void	1552

A.4.6.2.5	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	1552

A.4.6.2.5.1	Test Purpose and Environment	1552

A.4.6.2.5.2	Test Requirements	1557

A.4.6.2.6	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	1557

A.4.6.2.6.1	Test Purpose and Environment	1557

A.4.6.2.6.2	Test Requirements	1562

A.4.6.2.7	Void	1562

A.4.6.2.8	Void	1562

A.4.6.2.9	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used for UE configured with highSpeedMeasInterFreq-r17	1563

A.4.6.2.9.1	Test Purpose and Environment	1563

A.4.6.2.9.2	Test Requirements	1566

A.4.6.3	Void	1566

A.4.6.4	L1-RSRP measurement for beam reporting	1566

A.4.6.4.1	SSB based L1-RSRP measurement when DRX is not used	1566

A.4.6.4.1.1	Test Purpose and Environment	1566

A.4.6.4.1.2	Test parameters	1567

A.4.6.4.1.3	Test Requirements	1570

A.4.6.4.2	SSB based L1-RSRP measurement when DRX is used	1570

A.4.6.4.2.1	Test Purpose and Environment	1570

A.4.6.4.2.2	Test parameters	1571

A.4.6.4.2.3	Test Requirements	1574

A.4.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	1574

A.4.6.4.3.1	Test Purpose and Environment	1574

A.4.6.4.3.2	Test parameters	1575

A.4.6.4.3.3	Test Requirements	1578

A.4.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	1578

A.4.6.4.4.1	Test Purpose and Environment	1578

A.4.6.4.4.2	Test parameters	1579

A.4.6.4.4.3	Test Requirements	1581

A.4.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	1581

A.4.6.4.5.1	Test Purpose and Environment	1581

A.4.6.4.5.2	Test parameters	1582

A.4.6.4.5.3	Test Requirements	1585

A.4.6.5	CLI measurements	1585

A.4.6.5.1	SRS-RSRP measurement with non-DRX	1585

A.4.6.5.1.1	Test Purpose and Environment	1585

A.4.6.5.1.2	Test Parameters	1586

A.4.6.5.1.3	Test Requirements	1589

A.4.6.5.2	CLI-RSSI measurement with non-DRX	1589

A.4.6.5.2.1	Test Purpose and Environment	1589

A.4.6.5.2.2	Test Parameters	1590

A.4.6.5.2.3	Test Requirements	1591

A.4.6.6.1.2	Test Requirements	1596

A.4.6.7	L1-SINR measurement for beam reporting	1596

A.4.6.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	1599

A.4.6.7.2.1	Test Purpose and Environment	1599

A.4.6.7.2.2	Test parameters	1599

A.4.6.7.2.3	Test Requirements	1602

A.4.6.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	1602

A.4.6.7.3.1	Test Purpose and Environment	1602

A.4.6.7.3.2	Test parameters	1602

A.4.6.7.3.3	Test Requirements	1604

A.4.6.8	CSI-RS based intra-frequency Measurement	1605

A.4.6.8.1	EN-DC event triggered reporting tests without gap under DRX	1605

A.4.6.8.1.1	Test purpose and Environment	1605

A.4.6.8.1.2	Test Requirements	1609

A.4.6.9	CSI-RS based inter-frequency Measurement	1609

A.4.6.9.1	EN-DC event triggered reporting tests for FR1 cell when non-DRX is used	1609

A.4.6.9.1.1	Test Purpose and Environment	1609

A.4.6.9.1.2	Test Requirements	1613

A.4.7	Measurement Performance requirements	1614

A.4.7.1	SS-RSRP	1614

A.4.7.1.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1614

A.4.7.1.1.1	Test Purpose and Environment	1614

A.4.7.1.1.2	Test parameters	1614

A.4.7.1.1.3	Test Requirements	1618

A.4.7.1.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1618

A.4.7.1.2.1	Test Purpose and Environment	1618

A.4.7.1.2.2	Test parameters	1619

A.4.7.1.2.3	Test Requirements	1623

A.4.7.1.3	Void	1623

A.4.7.2	SS-RSRQ	1623

A.4.7.2.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1623

A.4.7.2.1.1	Test Purpose and Environment	1623

A.4.7.2.1.2	Test Parameters	1624

A.4.7.2.1.3	Test Requirements	1628

A.4.7.2.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1628

A.4.7.2.2.1	Test Purpose and Environment	1628

A.4.7.2.2.2	Test Parameters	1628

A.4.7.2.2.3	Test Requirements	1633

A.4.7.3	SS-SINR	1633

A.4.7.3.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1633

A.4.7.3.1.1	Test Purpose and Environment	1633

A.4.7.3.1.2	Test Parameters	1633

A.4.7.3.1.3	Test Requirements	1636

A.4.7.3.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1636

A.4.7.3.2.1	Test Purpose and Environment	1636

A.4.7.3.2.2	Test Parameters	1636

A.4.7.3.2.3	Test Requirements	1641

A.4.7.4	L1-RSRP measurement for beam reporting	1641

A.4.7.4.1	SSB based L1-RSRP measurement	1641

A.4.7.4.1.1	Test Purpose and Environment	1641

A.4.7.4.1.2	Test parameters	1642

A.4.7.4.1.3	Test Requirements	1645

A.4.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	1645

A.4.7.4.2.1	Test Purpose and Environment	1645

A.4.7.4.2.2	Test parameters	1646

A.4.7.4.2.3	Test Requirements	1649

A.4.7.5	SFTD accuracy	1649

A.4.7.5.1	SFTD accuracy	1649

A.4.7.5.1.1	Test Purpose and Environment	1649

A.4.7.5.1.2	Test Parameters	1649

A.4.7.5.1.3	Test Requirements	1654

A.4.7.5.2	Void	1654

A.4.7.5.3	Void	1654

A.4.7.6	CLI measurements	1654

A.4.7.6.1	EN-DC SRS-RSRP measurement accuracy with FR1 serving cell	1654

A.4.7.6.1.1	Test Purpose and Environment	1654

A.4.7.6.1.2	Test parameters	1655

A.4.7.6.1.3	Test Requirements	1660

A.4.7.6.2	EN-DC CLI-RSSI measurement accuracy with FR1 serving cell	1660

A.4.7.6.2.1	Test Purpose and Environment	1660

A.4.7.6.2.2	Test parameters	1661

A.4.7.6.2.3	Test Requirements	1664

A.4.7.7	L1-SINR measurement for beam reporting	1664

A.4.7.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR	1668

A.4.7.7.2.1	Test Purpose and Environment	1668

A.4.7.7.2.2	Test parameters	1668

A.4.7.7.2.3	Test Requirements	1672

A.4.7.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	1672

A.4.7.7.3.1	Test Purpose and Environment	1672

A.4.7.7.3.2	Test parameters	1672

A.4.7.7.3.3	Test Requirements	1675

A.4.7.8	CSI-RSRP	1675

A.4.7.8.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1675

A.4.7.8.1.1	Test Purpose and Environment	1675

A.4.7.8.1.2	Test parameters	1676

A.4.7.8.1.3	Test Requirements	1681

A.4.7.8.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1681

A.4.7.8.2.1	Test Purpose and Environment	1681

A.4.7.8.2.2	Test parameters	1681

A.4.7.8.2.3	Test Requirements	1686

A.4.7.9	CSI-RSRQ	1686

A.4.7.9.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1686

A.4.7.9.1.1	Test Purpose and Environment	1686

A.4.7.9.1.2	Test Parameters	1686

A.4.7.9.1.3	Test Requirements	1691

A.4.7.9.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1691

A.4.7.9.2.1	Test Purpose and Environment	1691

A.4.7.9.2.2	Test Parameters	1691

A.4.7.9.2.3	Test Requirements	1696

A.4.7.10	CSI-SINR	1696

A.4.7.10.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1696

A.4.7.10.1.1	Test Purpose and Environment	1696

A.4.7.10.1.2	Test Parameters	1697

A.4.7.10.1.3	Test Requirements	1701

A.4.7.10.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1701

A.4.7.10.2.1	Test Purpose and Environment	1701

A.4.7.10.2.2	Test Parameters	1702

A.4.7.10.2.3	Test Requirements	1706

A.4.8	Void	1707

A.4A	NE-DC test with all NR cells in FR1	1707

A.4A.1	Signaling characteristics	1707

A.4A.1.1	E-UTRAN PSCell addition	1707

A.4A.1.1.1	Test purpose and environment	1707

A.4A.1.1.2	Test Requirements	1711

A.4A.1.2	Active BWP switch	1712

A.4A.1.2.1	E-UTRAN PSCell – NR PCell FR1 DCI-based and Timer-based DL active BWP switch in non-DRX in synchronous NE-DC	1712

A.4A.1.2.1.1	Test Purpose and Environment	1712

A.4A.1.2.1.2	Test Requirements	1716

A.4A.1.3	Intra-frequency handover with E-UTRAN PSCell	1717

A.4A.1.3.1	Test purpose and environment	1717

A.4A.1.3.2	Test Requirements	1721

A.4A.1.4	Handover with PSCell from NE-DC to NE-DC with unknown target PSCell	1722

A.4A.1.4.1	Test Purpose and Environment	1722

A.4A.1.4.2	Test Parameters	1722

A.4A.1.4.3	Test Requirements	1727

A.4A.1.4.3.1	Test Requirements for NR HO	1727

A.4A.1.4.3.2	Test Requirements for LTE PSCell Change	1727

A.4A.2	Measurement performance	1728

A.4A.2.1	SFTD accuracy	1728

A.4A.2.1.1	SFTD accuracy	1728

A.4A.2.1.1.1	Test Purpose	1728

A.4A.2.1.1.2	Test Environment	1728

A.4A.2.1.1.3	Test Requirements	1731

A.5	EN-DC tests with one or more NR cells in FR2	1732

A.5.1	Void	1732

A.5.2	Void	1732

A.5.3	RRC_CONNECTED state mobility	1732

A.5.3.1	Void	1732

A.5.3.2	RRC Connection Mobility Control	1732

A.5.3.2.1	Void	1732

A.5.3.2.2	Random Access	1732

A.5.3.2.2.1	4-step RA type c ontention based random access test in FR2 for PSCell/SCell in EN-DC	1732

A.5.3.2.2.2	4-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	1735

A.5.3.2.2.3	2-step RA type contention based random access test in FR2 for PSCell/SCell in EN-DC	1741

A.5.3.2.2.4	2-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	1744

A.5.3.2.3	Void	1748

A.5.3.3	Handover with PSCell with known FR2 target PSCell	1748

A.5.3.3.1	Test purpose and environment	1748

A.5.3.3.2	Test Requirements	1752

A.5.5.3.3	void	1753

A.5.5.3.4	void	1753

A.5.5.3.5	void	1753

A.5.5.3.6	void	1753

A.5.4	Timing	1753

A.5.4.1	UE transmit timing	1753

A.5.4.1.1	NR UE Transmit Timing Test for FR2	1753

A.5.4.1.1.1	Test Purpose and environment	1753

A.5.4.1.1.2	Test requirements	1756

A.5.4.2	UE timer accuracy	1757

A.5.4.3	Timing advance	1757

A.5.4.3.1 EN-DC FR2 timing advance adjustment accuracy	1757

A.5.4.3.1.1 Test Purpose and Environment	1757

A.5.4.3.1.2 Test Parameters	1757

A.5.4.3.1.3	Test Requirements	1761

A.5.5	Signaling characteristics	1761

A.5.5.1	Radio link Monitoring	1761

A.5.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	1761

A.5.5.1.1.1	Test Purpose and Environment	1761

A.5.5.1.1.2	Test Requirements	1765

A.5.5.1.2	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	1766

A.5.5.1.2.1	Test Purpose and Environment	1766

A.5.5.1.2.2	Test Requirements	1769

A.5.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	1770

A.5.5.1.3.1	Test Purpose and Environment	1770

A.5.5.1.3.2	Test Requirements	1774

A.5.5.1.4	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	1774

A.5.5.1.4.1	Test Purpose and Environment	1774

A.5.5.1.4.2	Test Requirements	1778

A.5.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	1778

A.5.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	1782

A.5.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	1786

A.5.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	1791

A.5.5.1.8.2	Test Requirements	1795

A.5.5.1.9	EN-DC Radio Link Monitoring UE Scheduling Restrictions on FR2	1796

A.5.5.1.9.1	Test Purpose and Environment	1796

A.5.5.1.9.2	Test Requirements	1798

A.5.5.1.10	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion	1798

A.5.5.1.10.1	Test Purpose and Environment	1798

A.5.5.1.10.2	Test Requirements	1801

A.5.5.2	Interruption	1801

A.5.5.2.1	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	1801

A.5.5.2.1.1	Test Purpose and Environment	1801

A.5.5.2.1.2	Test Requirements	1804

A.5.5.2.2	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	1804

A.5.5.2.2.1	Test Purpose and Environment	1804

A.5.5.2.2.2	Test Requirements	1807

A.5.5.2.3	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	1807

A.5.5.2.3.1	Test Purpose and Environment	1807

A.5.5.2.3.2	Test Requirements	1811

A.5.5.2.4	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	1811

A.5.5.2.4.1	Test Purpose and Environment	1811

A.5.5.2.4.2	Test Requirements	1814

A.5.5.2.5	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	1815

A.5.5.2.5.1	Test Purpose and Environment	1815

A.5.5.2.5.2	Test Requirements	1818

A.5.5.2.6	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	1818

A.5.5.2.6.1	Test Purpose and Environment	1818

A.5.5.2.6.2	Test Requirements	1821

A.5.5.2.7	E-UTRAN – NR FR2 interruptions at E-UTRA SRS carrier based switching	1821

A.5.5.2.7.1	Test Purpose and Environment	1821

A.5.5.2.7.2	Test Requirements	1825

A.5.5.2.8 E-UTRAN – NR FR2 interruptions at NR SRS carrier based switching	1825

A.5.5.2.8.1 Test Purpose and Environment	1825

A.5.5.2.8.3	Test Requirements	1827

A.5.5.3	SCell Activation and Deactivation Delay	1827

A.5.5.3.1	SCell Activation and deactivation of SCell in FR2 intra-band	1827

A.5.5.3.1.1	Test Purpose and Environment	1827

A.5.5.3.1.2	Test Requirements	1829

A.5.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	1829

A.5.5.3.2.1	Test Purpose and Environment	1829

A.5.5.3.2.2	Test Requirements	1833

A.5.5.3.3	Void	1833

A.5.5.3.4	Void	1833

A.5.5.3.5	SCell Activation and deactivation of SCell in FR2	1833

A.5.5.3.5.1	Test Purpose and Environment	1833

A.5.5.3.5.2	Test Requirements	1836

A.5.5.3.6	Multiple SCell Activation and deactivation of one unknown SCell and one known SCell in FR2	1837

A.5.5.3.6.1	Test Purpose and Environment	1837

A.5.5.3.6.2	Test Requirements	1841

A.5.5.3.7	Direct SCell activation at SCell addition of known SCell in FR2	1842

A.5.5.3.7.1	Test Purpose and Environment	1842

A.5.5.3.7.2	Test Requirements	1845

A.5.5.3.8	Fast SCell Activation of SCell in FR2 intra-band	1845

A.5.5.3.8.1	Test Purpose and Environment	1845

A.5.5.3.8.2	Test Requirements	1849

A.5.5.3.9	PUCCH SCell Activation and deactivation of known SCell in FR2	1849

A.5.5.3.9.1	Test Purpose and Environment	1849

A.5.5.3.9.2	Test Requirements	1853

A.5.5.3.10	PUCCH SCell Activation and deactivation of unknown SCell in FR2	1853

A.5.5.3.10.1	Test Purpose and Environment	1853

A.5.5.3.10.2	Test Requirements	1856

A.5.5.3.11	Multiple SCell activation and deactivation of one known PUCCH SCell and one unknown SCell in FR2	1857

A.5.5.3.11.1	Test Purpose and Environment	1857

A.5.5.3.11.2	Test Requirements	1860

A.5.5.3.12	SCell Activation and deactivation of unknown PUCCH SCell and unknown DL SCell in FR2 in non-DRX	1861

A.5.5.3.12.1	Test Purpose and Environment	1861

A.5.5.3.12.2	Test Requirements	1864

A.5.5.4	Void	1865

A.5.5.5	Beam Failure Detection and Link recovery procedures	1865

A.5.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	1865

A.5.5.5.1.1	Test Purpose and Environment	1865

A.5.5.5.1.2	Test Requirements	1869

A.5.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in DRX mode	1870

A.5.5.5.2.1	Test Purpose and Environment	1870

A.5.5.5.2.2	Test Requirements	1874

A.5.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	1875

A.5.5.5.3.1	Test Purpose and Environment	1875

A.5.5.5.3.2	Test Requirements	1879

A.5.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX mode	1880

A.5.5.5.4.1	Test Purpose and Environment	1880

A.5.5.5.4.2	Test Requirements	1884

A.5.5.5.5	EN-DC scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	1885

A.5.5.5.5.1	Test Purpose and Environment	1885

A.5.5.5.5.2	Test Requirements	1889

A.5.5.5.6	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	1890

A.5.5.5.6.1	Test Purpose and Environment	1890

A.5.5.5.6.2	Test Requirements	1894

A.5.5.5.7	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	1895

A.5.5.5.7.1	Test Purpose and Environment	1895

A.5.5.5.7.2	Test Requirements	1899

A.5.5.5.8	EN-DC TRP specific Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX mode	1900

A.5.5.5.8.1	Test Purpose and Environment	1900

A.5.5.5.8.2	Test Requirements	1905

A.5.5.5.9	Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in DRX mode for UE fulfilling relaxed measurement criterion	1905

A.5.5.5.9.1	Test Purpose and Environment	1905

A.5.5.5.9.2	Test Requirements	1909

A.5.5.6	Active BWP switch	1909

A.5.5.6.1	DCI-based and Timer-based Active BWP Switch	1909

A.5.5.6.1.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	1909

A.5.5.6.1.1.1	Test Purpose and Environment	1909

A.5.5.6.1.1.2	Test Requirements	1913

A.5.5.6.1.2	E-UTRAN – NR PSCell FR2 with FR2 SCell DL active BWP switch in non-DRX in synchronous EN-DC	1913

A.5.5.6.2	RRC-based Active BWP Switch	1918

A.5.5.6.2.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	1918

A.5.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1922

A.5.5.6.3.1	E-UTRAN – NR PSCell FR2 and NR SCell FR2 DL active BWP switch on multiple CCs in synchronous EN-DC	1922

A.5.5.6.4	SCell dormancy switch	1926

A.5.5.6.4.1	E-UTRAN – NR FR2 PSCell SCell dormancy switch of single FR2 SCell inside active time	1926

A.5.5.6.4.1.1	Test Purpose and Environment	1926

A.5.5.6.4.1.2	Test Requirements	1930

A.5.5.6.4.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR2 SCells outside active time	1930

A.5.5.6.4.2.1	Test Purpose and Environment	1930

A.5.5.6.4.2.2	Test Requirements	1937

A.5.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	1937

A.5.5.6.5.1	E-UTRAN – NR PSCell FR2  and NR SCell FR2 DL active BWP switch on multiple CCs with non-DRX in synchronous EN-DC	1937

A.5.5.7	PSCell addition and release delay	1941

A.5.5.7.1	Addition and Release Delay of NR PSCell	1941

A.5.5.7.1.1	Test purpose and environment	1941

A.5.5.7.1.2	Test Requirements	1944

A.5.5.8	Active TCI state switch delay	1945

A.5.5.8.1	MAC-CE based active TCI state switch	1945

A.5.5.8.1.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	1945

A.5.5.8.1.1.1	Test Purpose and Environment	1945

A.5.5.8.1.1.2	Test Requirements	1948

A.5.5.8.2	RRC based active TCI state switch	1948

A.5.5.8.2.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	1948

A.5.5.8.2.1.1	Test Purpose and Environment	1948

A.5.5.8.2.1.2	Test Requirements	1952

A.5.5.9	Uplink spatial relation switch delay	1952

A.5.5.9.1	MAC-CE based uplink spatial relation switch	1952

A.5.5.9.1.1	E-UTRAN – NR PSCell FR2 uplink spatial relation switch for a known spatial relation	1952

A.5.5.9.1.1.1	Test Purpose and Environment	1952

A.5.5.9.1.1.2	Test Requirements	1955

A.5.5.9.2	RRC based spatial relation switch	1955

A.5.5.9.2.1	E-UTRAN – NR PSCell FR2 spatial relation switch associated with a known DL-RS	1955

A.5.5.9.2.1.1	Test Purpose and Environment	1955

A.5.5.9.2.1.2	Test Requirements	1958

A.5.5.10	UE specific CBW change	1958

A.5.5.10.1	UE specific CBW change on FR2 NR PSCell	1958

A.5.5.10.1.1	Test Purpose and Environment	1958

A.5.5.10.1.2	Test Requirements	1961

A.5.5.11	Unified TCI state switch delay	1962

A.5.5.11.1	MAC-CE based active joint TCI state switch	1962

A.5.5.11.1.1	E-UTRAN – NR PSCell FR2 active joint TCI state switch for a known TCI state	1962

A.5.5.11.1.1.1	Test Purpose and Environment	1962

A.5.5.11.1.1.2	Test parameters	1962

A.5.5.11.1.1.3	Test Requirements	1965

A.5.5.11.2	MAC-CE based active uplink TCI state switch	1965

A.5.5.11.2.1	E-UTRAN – NR PSCell FR2 active uplink TCI state switch for a known TCI state	1965

A.5.5.11.2.1.1	Test Purpose and Environment	1965

A.5.5.11.2.1.2	Test parameters	1966

A.5.5.11.2.1.3	Test Requirements	1968

A.5.5.11.3	MAC-CE based active downlink TCI state switch	1968

A.5.5.11.3.1	E-UTRAN – NR PSCell FR2 downlink TCI state switch to cell with additional PCI for a known TCI state	1968

A.5.5.11.3.1.1	Test Purpose and Environment	1968

A.5.5.11.3.1.2	Test Parameters	1969

A.5.5.11.3.1.3	Test Requirements	1973

A.5.5.12	PSCell activation and deactivation delay	1973

A.5.5.12.1	PSCell activation and deactivation delay	1973

A.5.5.12.1.1	Test purpose and environment	1973

A.5.5.12.1.2	Test Requirements	1976

A.5.5.13	Conditional PSCell addition and release delay	1977

A.5.5.13.1	Addition and Release Delay of NR PSCell	1977

A.5.5.13.1.1	Test purpose and environment	1977

A.5.5.13.1.2	Test Requirements	1980

A.5.5.13.2	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR PSCell	1981

A.5.5.13.2.1	Test Purpose and Environment	1981

A.5.5.13.2.2	Test Requirements	1984

A.5.6	Measurement procedure	1984

A.5.6.1	Intra-frequency Measurements	1984

A.5.6.1.1	EN-DC event triggered reporting test without gap under non-DRX	1984

A.5.6.1.1.1	Test purpose and Environment	1984

A.5.6.1.1.2	Test Requirements	1988

A.5.6.1.2	EN-DC event triggered reporting test without gap under DRX	1988

A.5.6.1.2.1	Test purpose and Environment	1988

A.5.6.1.2.2	Test Requirements	1990

A.5.6.1.3	EN-DC event triggered reporting test with per-UE gaps under non-DRX	1991

A.5.6.1.3.1	Test purpose and Environment	1991

A.5.6.1.3.2	Test Requirements	1995

A.5.6.1.4	EN-DC event triggered reporting test with per-UE gaps under DRX	1995

A.5.6.1.4.1	Test purpose and Environment	1995

A.5.6.1.4.2	Test Requirements	1998

A.5.6.2	Inter-frequency Measurements	1999

A.5.6.2.1 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	1999

A.5.6.2.1.1	Test Purpose and Environment	1999

A.5.6.2.1.2	Test Requirements	2002

A.5.6.2.2 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	2002

A.5.6.2.2.1	Test Purpose and Environment	2002

A.5.6.2.2.2	Test Requirements	2006

A.5.6.2.3 	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	2006

A.5.6.2.3.1	Test Purpose and Environment	2006

A.5.6.2.3.2	Test Requirements	2010

A.5.6.2.4	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	2010

A.5.6.2.4.1	Test Purpose and Environment	2010

A.5.6.2.4.2	Test Requirements	2014

A.5.6.2.5	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	2014

A.5.6.2.5.1	Test Purpose and Environment	2014

A.5.6.2.5.2	Test Requirements	2019

A.5.6.2.6	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	2019

A.5.6.2.6.1	Test Purpose and Environment	2019

A.5.6.2.6.2	Test Requirements	2023

A.5.6.2.7	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	2023

A.5.6.2.7.1	Test Purpose and Environment	2023

A.5.6.2.7.2	Test Requirements	2028

A.5.6.2.8	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	2028

A.5.6.2.8.1	Test Purpose and Environment	2028

A.5.6.2.8.2	Test Requirements	2033

A.5.6.3	L1-RSRP measurement for beam reporting	2034

A.5.6.3.1	SSB based L1-RSRP measurement when DRX is not used	2034

A.5.6.3.1.1	Test Purpose and Environment	2034

A.5.6.3.1.2	Test parameters	2034

A.5.6.3.1.3	Test Requirements	2036

A.5.6.3.2	SSB based L1-RSRP measurement when DRX is used	2036

A.5.6.3.2.1	Test Purpose and Environment	2036

A.5.6.3.2.2	Test parameters	2037

A.5.6.3.2.3	Test Requirements	2039

A.5.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	2039

A.5.6.3.3.1	Test Purpose and Environment	2039

A.5.6.3.3.2	Test parameters	2040

A.5.6.3.3.3	Test Requirements	2042

A.5.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	2042

A.5.6.3.4.1	Test Purpose and Environment	2042

A.5.6.3.4.2	Test parameters	2043

A.5.6.3.4.3	Test Requirements	2045

A.5.6.4	CLI measurements	2046

A.5.6.4.1	SRS-RSRP measurement with DRX	2046

A.5.6.4.1.1	Test Purpose and Environment	2046

A.5.6.4.1.2	Test Parameters	2046

A.5.6.4.1.3	Test Requirements	2048

A.5.6.4.2	CLI-RSSI measurement with DRX	2048

A.5.6.4.2.1	Test Purpose and Environment	2048

A.5.6.4.2.2	Test Parameters	2049

A.5.6.4.2.3	Test Requirements	2051

A.5.6.5	Measurements with autonomous gaps	2051

A.5.6.5.1 	EN-DC inter-frequency CGI identification of NR neighbor cell in FR2	2051

A.5.6.5.1.1	Test Purpose and Environment	2051

A.5.6.5.1.2	Test Requirements	2054

A.5.6.6	L1-SINR measurement for beam reporting	2055

A.5.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	2057

A.5.6.6.2.1	Test Purpose and Environment	2057

A.5.6.6.2.2	Test parameters	2058

A.5.6.6.2.3	Test Requirements	2061

A.5.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	2061

A.5.6.6.3.1	Test Purpose and Environment	2061

A.5.6.6.3.2	Test parameters	2062

A.5.6.6.3.3	Test Requirements	2064

A.5.6.7	CSI-RS based Intra-frequency Measurements	2064

A.5.6.7.1	EN-DC event triggered reporting test without gap under non-DRX	2064

A.5.6.7.1.1	Test purpose and Environment	2064

A.5.6.7.1.2	Test Requirements	2067

A.5.6.8	CSI-RS based Inter-frequency Measurements	2067

A.5.6.8.1 	EN-DC event triggered reporting tests for NR FR2 cell when DRX is used	2067

A.5.6.8.1.1	Test Purpose and Environment	2067

A.5.6.8.1.2	Test Requirements	2072

A.5.7	Measurement Performance requirements	2072

A.5.7.1	SS-RSRP	2072

A.5.7.1.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2072

A.5.7.1.1.1	Test Purpose and Environment	2072

A.5.7.1.1.2	Test parameters	2073

A.5.7.1.1.3	Test Requirements	2075

A.5.7.1.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2076

A.5.7.1.2.1	Test Purpose and Environment	2076

A.5.7.1.2.2	Test parameters	2076

A.5.7.1.2.3	Test Requirements	2080

A.5.7.1.3	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	2081

A.5.7.1.3.1	Test Purpose and Environment	2081

A.5.7.1.3.2	Test parameters	2081

A.5.7.1.3.3	Test Requirements	2084

A.5.7.2	SS-RSRQ	2085

A.5.7.2.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2085

A.5.7.2.1.1	Test Purpose and Environment	2085

A.5.7.2.1.2	Test Parameters	2085

A.5.7.2.1.3	Test Requirements	2088

A.5.7.2.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2088

A.5.7.2.2.1	Test Purpose and Environment	2088

A.5.7.2.2.2	Test Parameters	2088

A.5.7.2.2.3	Test Requirements	2090

A.5.7.3	SS-SINR	2091

A.5.7.3.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2091

A.5.7.3.1.1	Test Purpose and Environment	2091

A.5.7.3.1.2	Test Parameters	2091

A.5.7.3.1.3	Test Requirements	2094

A.5.7.3.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2094

A.5.7.3.2.1	Test Purpose and Environment	2094

A.5.7.3.2.2	Test Parameters	2094

A.5.7.3.2.3	Test Requirements	2096

A.5.7.4	L1-RSRP measurement for beam reporting	2096

A.5.7.4.1	SSB based L1-RSRP measurement	2096

A.5.7.4.1.1	Test Purpose and Environment	2096

A.5.7.4.1.2	Test parameters	2097

A.5.7.4.1.3	Test Requirements	2099

A.5.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	2100

A.5.7.4.2.1	Test Purpose and Environment	2100

A.5.7.4.2.2	Test parameters	2100

A.5.7.4.2.3	Test Requirements	2102

A.5.7.5	CLI measurements	2103

A.5.7.5.1	EN-DC SRS-RSRP measurement accuracy with FR2 serving cell	2103

A.5.7.5.1.1	Test Purpose and Environment	2103

A.5.7.5.1.2	Test parameters	2103

A.5.7.5.1.3	Test Requirements	2106

A.5.7.5.2	EN-DC CLI-RSSI measurement accuracy with FR2 serving cell	2107

A.5.7.5.2.1	Test Purpose and Environment	2107

A.5.7.5.2.2	Test parameters	2107

A.5.7.5.2.3	Test Requirements	2109

A.5.7.6	L1-SINR measurement for beam reporting	2110

A.5.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	2113

A.5.7.6.2.1	Test Purpose and Environment	2113

A.5.7.6.2.2	Test parameters	2113

A.5.7.6.2.3	Test Requirements	2115

A.5.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	2116

A.5.7.6.3.1	Test Purpose and Environment	2116

A.5.7.6.3.2	Test parameters	2116

A.5.7.6.3.3	Test Requirements	2118

A.5.7.7	CSI-RSRP	2119

A.5.7.7.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2119

A.5.7.7.1.1	Test Purpose and Environment	2119

A.5.7.7.1.2	Test parameters	2119

A.5.7.7.1.3	Test Requirements	2121

A.5.7.7.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2122

A.5.7.7.2.1	Test Purpose and Environment	2122

A.5.7.7.2.2	Test parameters	2122

A.5.7.7.2.3	Test Requirements	2126

A.5.7.8	CSI-RSRQ	2127

A.5.7.8.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	2127

A.5.7.8.1.1	Test Purpose and Environment	2127

A.5.7.8.1.2	Test Parameters	2127

A.5.7.8.1.3	Test Requirements	2129

A.5.7.8.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2129

A.5.7.8.2.1	Test Purpose and Environment	2129

A.5.7.8.2.2	Test Parameters	2129

A.5.7.8.2.3	Test Requirements	2131

A.5.7.9	CSI-SINR	2131

A.5.7.9.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2131

A.5.7.9.1.1	Test Purpose and Environment	2131

A.5.7.9.1.2	Test Parameters	2131

A.5.7.9.1.3	Test Requirements	2134

A.5.7.9.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2134

A.5.7.9.2.1	Test Purpose and Environment	2134

A.5.7.9.2.2	Test Parameters	2134

A.5.7.9.2.3	Test Requirements	2137

A.5.8	Void	2137

A.6	NR standalone tests with all NR cells in FR1	2135

A.6.1	SA: RRC_IDLE state mobility	2135

A.6.1.1	Cell re-selection to NR	2135

A.6.1.1.1	Cell reselection to FR1 intra-frequency NR case	2135

A.6.1.1.1.1	Test Purpose and Environment	2135

A.6.1.1.1.2	Test Parameters	2135

A.6.1.1.1.3	Test Requirements	2139

A.6.1.1.2	Cell reselection to FR1 inter-frequency NR case	2139

A.6.1.1.2.1	Test Purpose and Environment	2139

A.6.1.1.2.2	Test Parameters	2139

A.6.1.1.2.3	Test Requirements	2143

A.6.1.1.3	Cell reselection to FR1 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	2143

A.6.1.1.3.1	Test Purpose and Environment	2143

A.6.1.1.3.2	Test Parameters	2143

A.6.1.1.3.3	Test Requirements	2148

A.6.1.1.4	Cell reselection to FR1 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	2148

A.6.1.1.4.1	Test Purpose and Environment	2148

A.6.1.1.4.2	Test Parameters	2148

A.6.1.1.4.3	Test Requirements	2151

A.6.1.1.5	Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	2151

A.6.1.1.5.1	Test Purpose and Environment	2151

A.6.1.1.5.2	Test Parameters	2151

A.6.1.1.5.3	Test Requirements	2156

A.6.1.1.6	Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	2157

A.6.1.1.6.1	Test Purpose and Environment	2157

A.6.1.1.6.2	Test Parameters	2157

A.6.1.1.6.3	Test Requirements	2161

A.6.1.1.7	Cell reselection to FR1 intra-frequency NR case for UE configured with highSpeedMeasFlag-r16	2162

A.6.1.1.7.1	Test Purpose and Environment	2162

A.6.1.1.7.2	Test Parameters	2162

A.6.1.1.7.3	Test Requirements	2165

A.6.1.1.8	Cell reselection to FR1 inter-frequency NR case for UE configured with highSpeedMeasInterFreq-r17	2165

A.6.1.1.8.1	Test Purpose and Environment	2165

A.6.1.1.8.2	Test Parameters	2165

A.6.1.1.8.3	Test Requirements	2169

A.6.1.2	Inter-RAT E-UTRAN cell re-selection	2169

A.6.1.2.1	Cell reselection to higher priority E-UTRAN	2169

A.6.1.2.1.1	Test Purpose and Environment	2169

A.6.1.2.1.2	Test Parameters	2169

A.6.1.2.1.3	Test Requirements	2172

A.6.1.2.2	Cell reselection to lower priority E-UTRAN	2173

A.6.1.2.2.1	Test Purpose and Environment	2173

A.6.1.2.2.2	Test Parameters	2173

A.6.1.2.2.3	Test Requirements	2176

A.6.1.2.3	Cell reselection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion	2177

A.6.1.2.3.1	Test Purpose and Environment	2177

A.6.1.2.3.2	Test Parameters	2177

A.6.1.2.3.3	Test Requirements	2180

A.6.1.2.4	Cell reselection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion	2181

A.6.1.2.4.1	Test Purpose and Environment	2181

A.6.1.2.4.2	Test Parameters	2181

A.6.1.2.4.3	Test Requirements	2184

A.6.1.2.5	Cell reselection to lower priority E-UTRAN cell for UE configured with highSpeedMeasFlag-r16	2185

A.6.1.2.5.1	Test Purpose and Environment	2185

A.6.1.2.5.2	Test Parameters	2185

A.6.1.2.5.3	Test Requirements	2189

A.6.1.1.7	Void	2190

A.6.2	SA: RRC_INACTIVE state mobility	2190

A.6.2.1	Configured Grant based Small Data Transmissions (CG-SDT)	2190

A.6.2.1.1	Test purpose and Environment	2190

A.6.2.1.2	Test Parameters	2192

A.6.2.1.3	Test requirements	2194

A.6.3	RRC_CONNECTED state mobility	2194

A.6.3.1	Handover	2194

A.6.3.1.1	Intra-frequency handover from FR1 to FR1; known target cell	2194

A.6.3.1.1.1	Test Purpose and Environment	2194

A.6.3.1.1.2	Test Parameters	2194

A.6.3.1.1.3 Test Requirements	2198

A.6.3.1.2	Intra-frequency handover from FR1 to FR1; unknown target cell	2198

A.6.3.1.2.1	Test Purpose and Environment	2198

A.6.3.1.2.2	Test Parameters	2198

A.6.3.1.2.3	Test Requirements	2202

A.6.3.1.3	Inter-frequency handover from FR1 to FR1; unknown target cell	2202

A.6.3.1.3.1	Test Purpose and Environment	2202

A.6.3.1.3.2	Test Parameters	2202

A.6.3.1.3.3	Test Requirements	2206

A.6.3.1.4	SA NR - E-UTRAN handover	2206

A.6.3.1.4.1	Test Purpose and Environment	2206

A.6.3.1.4.2	Test Requirements	2212

A.6.3.1.5	SA NR - E-UTRAN handover with unknown target cell	2212

A.6.3.1.5.1	Test Purpose and Environment	2212

A.6.3.1.5.2	Test Requirements	2218

A.6.3.1.6	 SA NR - UTRAN FDD handover	2218

A.6.3.1.6.1	Test Purpose and Environment	2218

A.6.3.1.6.2	Test Requirements	2222

A.6.3.1.7	Intra-frequency synchronous DAPS handover in FR1	2222

A.6.3.1.7.1	Test Purpose and Environment	2222

A.6.3.1.7.2	Test Parameters	2222

A.6.3.1.7.3	Test Requirements	2226

A.6.3.1.8	Intra-frequency asynchronous DAPS handover in FR1	2227

A.6.3.1.8.1	Test Purpose and Environment	2227

A.6.3.1.8.2	Test Parameters	2227

A.6.3.1.8.3	Test Requirements	2231

A.6.3.1.9	Intra-band inter-frequency synchronous DAPS handover test in SA for FR1	2232

A.6.3.1.9.1	Test Purpose and Environment	2232

A.6.3.1.9.2	Test Parameters	2232

A.6.3.1.9.3	Test Requirements	2236

A.6.3.1.10	Intra-band inter-frequency asynchronous DAPS handover test in SA for FR1	2236

A.6.3.1.10.1	Test Purpose and Environment	2236

A.6.3.1.10.2	Test Parameters	2236

A.6.3.1.10.3	Test Requirements	2239

A.6.3.1.11	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR1	2239

A.6.3.1.11.1	Test Purpose and Environment	2239

A.6.3.1.11.2	Test Parameters	2239

A.6.3.1.11.3 Test Requirements	2246

A.6.3.1.12	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR1	2246

A.6.3.1.12.1	Test Purpose and Environment	2246

A.6.3.1.12.2	Test Parameters	2246

A.6.3.1.12.3 Test Requirements	2254

A.6.3.1.13	SA NR - E-UTRAN with NR PSCell addition in FR1	2254

A.6.3.1.13.1	Test Purpose and Environment	2254

A.6.3.1.13.2	Test Requirements	2263

A.6.3.1.14	SA NR - E-UTRAN handover with NR FR1 PScell addition	2263

A.6.3.1.14.1	Test Purpose and Environment	2263

A.6.3.1.14.2	Test Requirements	2272

A.6.3.2	RRC Connection Mobility Control	2273

A.6.3.2.1	SA: RRC Re-establishment	2273

A.6.3.2.1.1	Intra-frequency RRC Re-establishment in FR1	2273

A.6.3.2.1.2	Inter-frequency RRC Re-establishment in FR1	2277

A.6.3.2.1.3	Intra-frequency RRC Re-establishment in FR1 without serving cell timing	2281

A.6.3.2.2	Random Access	2284

A.6.3.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone	2284

A.6.3.2.2.2	4-step RA type non-contention based random access test in FR1 for NR standalone	2288

A.6.3.2.2.3	2-step RA type contention based random access test in FR1 for NR standalone	2293

A.6.3.2.2.4	2-step RA type non-contention based test in FR1 for NR standalone	2298

A.6.3.2.3	SA: RRC Connection Release with Redirection	2302

A.6.3.2.3.1	Redirection from NR in FR1 to NR in FR1	2302

A.6.3.2.3.2	Redirection from NR in FR1 to E-UTRAN	2306

A.6.3.3 Conditional handover	2313

A.6.3.3.1	Intra-frequency conditional handover from FR1 to FR1	2313

A.6.3.3.1.1	Test Purpose and Environment	2313

A.6.3.3.1.2	Test Parameters	2313

A.6.3.3.1.3 Test Requirements	2317

A.6.3.3.2	Inter-frequency conditional handover from FR1 to FR1	2317

A.6.3.3.2.1	Test Purpose and Environment	2317

A.6.3.3.2.2	Test Parameters	2317

A.6.3.3.2.3 Test Requirements	2321

A.6.4	Timing	2321

A.6.4.1	UE transmit timing	2321

A.6.4.1.1	NR UE Transmit Timing Test for FR1	2321

A.6.4.1.1.1	Test Purpose and environment	2321

A.6.4.1.1.2	Test requirements	2325

A.6.4.2	UE timer accuracy	2325

A.6.4.3	Timing advance	2325

A.6.4.3.1	SA FR1 timing advance adjustment accuracy	2325

A.6.4.3.1.1	Test Purpose and Environment	2325

A.6.4.3.1.2	Test Parameters	2325

A.6.4.3.1.3	Test Requirements	2329

A.6.5	Signalling characteristics	2329

A.6.5.1	Radio link Monitoring	2329

A.6.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	2330

A.6.5.1.1.1	Test Purpose and Environment	2330

A.6.5.1.1.2	Test Requirements	2335

A.6.5.1.2	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	2335

A.6.5.1.2.1	Test Purpose and Environment	2335

A.6.5.1.2.2	Test Requirements	2341

A.6.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	2341

A.6.5.1.3.1	Test Purpose and Environment	2341

A.6.5.1.3.2	Test Requirements	2347

A.6.5.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	2347

A.6.5.1.4.1	Test Purpose and Environment	2347

A.6.5.1.4.2	Test Requirements	2353

A.6.5.1.5	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	2353

A.6.5.1.5.1	Test Purpose and Environment	2353

A.6.5.1.5.2	Test Requirements	2359

A.6.5.1.6	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	2359

A.6.5.1.6.1	Test Purpose and Environment	2359

A.6.5.1.6.2	Test Requirements	2364

A.6.5.1.7	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	2364

A.6.5.1.7.1	Test Purpose and Environment	2364

A.6.5.1.7.2	Test Requirements	2369

A.6.5.1.8	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	2369

A.6.5.1.8.1	Test Purpose and Environment	2369

A.6.5.1.8.2	Test Requirements	2374

A.6.5.1.9	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM for UE fulfilling relaxed measurement criterion	2374

A.6.5.1.9.1	Test Purpose and Environment	2374

A.6.5.1.9.2	Test Requirements	2380

A.6.5.2	Interruption	2380

A.6.5.2.1	Interruptions during measurements on deactivated NR SCC in FR1	2380

A.6.5.2.1.2	Test Requirements	2384

A.6.5.2.2	SA interruptions at NR SRS carrier based switching	2385

A.6.5.2.2.1	Test Purpose and Environment	2385

A.6.5.2.2.2	Test Parameters	2385

A.6.5.2.2.3	Test Requirements	2387

A.6.5.2.3	SA interruptions at NR SRS antenna port switching with 1 SRS symbol in a slot in NR-CA	2388

A.6.5.2.3.1	Test Purpose and Environment	2388

A.6.5.2.3.2	Test Parameters	2388

A.6.5.2.3.3	Test Requirements	2390

A.6.5.2.4	SA interruptions at NR SRS antenna port switching	2391

A.6.5.2.4.1	Test Purpose and Environment	2391

A.6.5.2.4.2	Test Parameters	2391

A.6.5.2.4.3	Test Requirements	2393

A.6.5.3	SCell Activation and Deactivation Delay	2394

A.6.5.3.1	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160ms SCell measurement cycle	2394

A.6.5.3.1.1	Test Purpose and Environment	2394

A.6.5.3.1.2	Test Requirements	2399

A.6.5.3.2	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle	2400

A.6.5.3.2.1	Test Purpose and Environment	2400

A.6.5.3.2.2	Test Requirements	2400

A.6.5.3.3	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX	2400

A.6.5.3.3.1	Test Purpose and Environment	2400

A.6.5.3.3.2	Test Requirements	2401

A.6.5.3.4	Direct SCell activation at SCell addition of known SCell in FR1	2401

A.6.5.3.4.1	Test Purpose and Environment	2401

A.6.5.3.4.2	Test Requirements	2408

A.6.5.3.5	Direct SCell activation at handover with known SCell in FR1	2408

A.6.5.3.5.1	Test Purpose and Environment	2408

A.6.5.3.5.2	Test Requirements	2413

A.6.5.3.6	PUCCH SCell Activation and deactivation of known SCell in FR1	2413

A.6.5.3.6.1	Test Purpose and Environment	2413

A.6.5.3.6.2	Test Requirements	2417

A.6.5.3.7	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX	2417

A.6.5.3.7.1	Test Purpose and Environment	2417

A.6.5.3.7.2	Test Requirements	2421

A.6.5.3.8	SCell Activation and Deactivation of one FR1 known PUCCH SCell and one FR1 unknown SCell with single activation/deactivation command	2422

A.6.5.3.8.1	Test Purpose and Environment	2422

A.6.5.3.8.2	Test Requirements	2426

A.6.5.3.9	SCell Activation and deactivation of unknown PUCCH SCell and unknown DL SCell in FR1 in non-DRX	2427

A.6.5.3.9.1	Test Purpose and Environment	2427

A.6.5.3.9.2	Test Requirements	2431

A.6.5.3.10	Fast SCell Activation of known SCell in FR1 in non-DRX for 160ms SCell measurement cycle	2432

A.6.5.3.10.1	Test Purpose and Environment	2432

A.6.5.3.10.2	Test Requirements	2435

A.6.5.3.11	SCell Activation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle	2436

A.6.5.3.11.1	Test Purpose and Environment	2436

A.6.5.3.11.2	Test Requirements	2436

A.6.5.4	UE UL carrier RRC reconfiguration Delay	2436

A.6.5.4.1	UE UL carrier RRC reconfiguration Delay	2436

A.6.5.4.1.1	Test Purpose and Environment	2436

A.6.5.4.1.2	Test Requirements	2446

A.6.5.4.2	Void	2447

A.6.5.5	Beam Failure Detection and Link recovery procedures	2447

A.6.5.5.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	2447

A.6.5.5.1.1	Test Purpose and Environment	2447

A.6.5.5.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	2453

A.6.5.5.2.1	Test Purpose and Environment	2453

A.6.5.5.2.2	Test Requirements	2459

A.6.5.5.3	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	2460

A.6.5.5.3.1	Test Purpose and Environment	2460

A.6.5.5.3.2	Test Requirements	2465

A.6.5.5.4	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode	2466

A.6.5.5.4.1	Test Purpose and Environment	2466

A.6.5.5.4.2	Test Requirements	2471

A.6.5.5.5	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	2472

A.6.5.5.5.1	Test Purpose and Environment	2472

A.6.5.5.5.2	Test Requirements	2476

A.6.5.5.6	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode	2476

A.6.5.5.6.1	Test Purpose and Environment	2476

A.6.5.5.6.2	Test Requirements	2481

A.6.5.5.7	TRP Specific Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode	2481

A.6.5.5.7.1	Test Purpose and Environment	2481

A.6.5.5.7.2	Test Requirements	2487

A.6.5.6	Active BWP switch	2488

A.6.5.6.1	DCI-based and Timer-based Active BWP Switch	2488

A.6.5.6.1.1	NR FR1- NR FR1 DL active BWP switch of SCell with non-DRX in SA	2488

A.6.5.6.1.2	NR FR1 DL active BWP switch with non-DRX in SA	2494

A.6.5.6.2	RRC-based Active BWP Switch	2499

A.6.5.6.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	2499

A.6.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	2503

A.6.5.6.3.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	2503

A.6.5.6.4	SCell dormancy switch	2510

A.6.5.6.4.1	NR FR1 PCell SCell dormancy switch of single FR1 SCell outside active time	2510

A.6.5.6.4.1.1	Test Purpose and Environment	2510

A.6.5.6.4.1.2	Test Requirements	2516

A.6.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR1 SCells inside active time	2516

A.6.5.6.4.2.1	 Test Purpose and Environment	2516

A.6.5.6.4.2.2	Test Requirements	2523

A.6.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	2523

A.6.5.6.5.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	2523

A.6.5.7	DL interruptions at switching between two uplink carriers	2528

A.6.5.7.1	DL interruptions at switching between two uplink carriers in FDD-TDD CA	2528

A.6.5.7.1.1	Test Purpose and Environment	2528

A.6.5.7.1.2	Test Requirements	2532

A.6.5.7.2	DL interruptions at switching between two uplink carriers in TDD-TDD CA	2532

A.6.5.7.2.1	Test Purpose and Environment	2532

A.6.5.7.2.2	Test Requirements	2536

A.6.5.7A	DL interruptions at switching between two uplink carriers with two transmit antenna connectors	2536

A.6.5.7A.1	DL interruptions at switching between two uplink carriers in FDD-TDD CA	2536

A.6.5.7A.1.1	Test Purpose and Environment	2536

A.6.5.7A.1.2	Test Requirements	2540

A.6.5.7A.2	DL interruptions at switching between two uplink carriers in TDD-TDD CA	2540

A.6.5.7A.2.1	Test Purpose and Environment	2540

A.6.5.7A.2.2	Test Requirements	2544

A.6.5.7B	DL interruptions at switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors	2544

A.6.5.7B.1	DL interruptions at switching between two uplink bands in FDD-TDD CA	2544

A.6.5.7B.1.1	Test Purpose and Environment	2544

A.6.5.7B.1.2	Test Requirements	2548

A.6.5.7B.2	DL interruptions at switching between two uplink bands in TDD-TDD CA	2548

A.6.5.7B.2.1	Test Purpose and Environment	2548

A.6.5.7B.2.2	Test Requirements	2552

A.6.5.7C	DL interruptions at switching between two uplink bands with two transmit antenna connectors	2552

A.6.5.7C.1	DL interruptions at switching between two uplink bands with two transmit antenna connectors in FDD-TDD CA	2552

A.6.5.7C.1.1	Test Purpose and Environment	2552

A.6.5.7C.1.2	Test Requirements	2556

A.6.5.7C.2	DL interruptions at switching between two uplink bands with two transmit antenna connectors in TDD-TDD CA	2557

A.6.5.7C.2.1	Test Purpose and Environment	2557

A.6.5.7C.2.2	Test Requirements	2561

A.6.5.8	UE specific CBW change	2562

A.6.5.8.1	UE specific CBW change on PCell in FR1 in non-DRX	2562

A.6.5.8.1.1	Test Purpose and Environment	2562

A.6.5.8.1.2	Test Requirements	2566

A.6.5.9	Pathloss reference signal switching delay	2566

A.6.5.9.1	MAC-CE based pathloss reference signal switch delay	2566

A.6.5.9.1.1	Test Purpose and Environment	2566

A.6.5.9.1.2	Test Requirements	2569

A.6.6	Measurement procedure	2570

A.6.6.1	Intra-frequency Measurements	2570

A.6.6.1.1	SA event triggered reporting tests without gap under non-DRX	2570

A.6.6.1.1.1	Test purpose and Environment	2570

A.6.6.1.1.2	Test parameters	2570

A.6.6.1.1.3	Test Requirements	2574

A.6.6.1.2	SA event triggered reporting tests without gap under DRX	2574

A.6.6.1.2.1	Test purpose and Environment	2574

A.6.6.1.2.2	Test parameters	2574

A.6.6.1.2.3	Test Requirements	2578

A.6.6.1.3	SA event triggered reporting tests with per-UE gaps under non-DRX	2578

A.6.6.1.3.1	Test purpose and Environment	2578

A.6.6.1.3.2	Test parameters	2578

A.6.6.1.3.3	Test Requirements	2582

A.6.6.1.4	SA event triggered reporting tests with per-UE gaps under DRX	2582

A.6.6.1.4.1	Test purpose and Environment	2582

A.6.6.1.4.2	Test parameters	2582

A.6.6.1.4.3	Test Requirements	2586

A.6.6.1.5	SA event triggered reporting tests without gap under non-DRX with SSB index reading	2586

A.6.6.1.5.1	Test purpose and Environment	2586

A.6.6.1.5.2	Test parameters	2586

A.6.6.1.5.3	Test Requirements	2588

A.6.6.1.6	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading	2589

A.6.6.1.6.1	Test purpose and Environment	2589

A.6.6.1.6.2	Test parameters	2589

A.6.6.1.6.3	Test Requirements	2590

A.6.6.1.7	SA event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	2591

A.6.6.1.7.1	Test purpose and Environment	2591

A.6.6.1.7.2	Test parameters	2591

A.6.6.1.7.3	Test Requirements	2595

A.6.6.1.8	SA event triggered reporting tests without gap under DRX for UE configured with highSpeedMeasCA-Scell-r17	2595

A.6.6.1.8.1	Test purpose and Environment	2595

A.6.6.1.8.2	Test parameters	2595

A.6.6.1.8.3	Test Requirements	2599

A.6.6.2	Inter-frequency Measurements	2599

A.6.6.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2599

A.6.6.2.1.1	Test Purpose and Environment	2599

A.6.6.2.1.2	Test Requirements	2603

A.6.6.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2603

A.6.6.2.2.1	Test Purpose and Environment	2603

A.6.6.2.2.2	Test Requirements	2607

A.6.6.2.3	Void	2608

A.6.6.2.4	Void	2608

A.6.6.2.5	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2608

A.6.6.2.5.1	Test Purpose and Environment	2608

A.6.6.2.5.2	Test Requirements	2612

A.6.6.2.6	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2612

A.6.6.2.6.1	Test Purpose and Environment	2612

A.6.6.2.6.2	Test Requirements	2616

A.6.6.2.7	Void	2616

A.6.6.2.8	Void	2616

A.6.6.2.9	SA event triggered reporting tests with additional mandatory gap pattern	2616

A.6.6.2.9.1	Test Purpose and Environment	2616

A.6.6.2.9.2	Test Requirements	2620

A.6.6.2.10	SA event triggered reporting tests for FR1 when DRX is used	2620

A.6.6.2.10.1	Test Purpose and Environment	2620

A.6.6.2.10.2	Test Requirements	2624

A.6.6.2.12	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for UE configured with highSpeedMeasInterFreq-r17	2628

A.6.6.2.12.1	Test Purpose and Environment	2628

A.6.6.2.12.2	Test Requirements	2632

A.6.6.3	Inter-RAT Measurements	2632

A.6.6.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1	2632

A.6.6.3.1.1	Test Purpose and Environment	2632

A.6.6.3.1.2	Test Requirements	2638

A.6.6.3.2	SA NR - E-UTRAN event-triggered reporting in DRX in FR1	2638

A.6.6.3.2.1	Test Purpose and Environment	2638

A.6.6.3.2.2	Test Requirements	2645

A.6.6.3.3	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for UE configured with highSpeedMeasFlag-r16	2645

A.6.6.3.3.1	Test Purpose and Environment	2645

A.6.6.3.3.2	Test Requirements	2650

A.6.6.4	L1-RSRP measurement for beam reporting	2651

A.6.6.4.1	SSB based L1-RSRP measurement when DRX is not used	2651

A.6.6.4.1.1	Test Purpose and Environment	2651

A.6.6.4.1.2	Test parameters	2651

A.6.6.4.1.3	Test Requirements	2654

A.6.6.4.2	SSB based L1-RSRP measurement when DRX is used	2654

A.6.6.4.2.1	Test Purpose and Environment	2654

A.6.6.4.2.2	Test parameters	2655

A.6.6.4.2.3	Test Requirements	2658

A.6.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	2658

A.6.6.4.3.1	Test Purpose and Environment	2658

A.6.6.4.3.2	Test parameters	2659

A.6.6.4.3.3	Test Requirements	2662

A.6.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	2662

A.6.6.4.4.1	Test Purpose and Environment	2662

A.6.6.4.4.2	Test parameters	2663

A.6.6.4.4.3	Test Requirements	2666

A.6.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	2666

A.6.6.4.5.1	Test Purpose and Environment	2666

A.6.6.4.5.2	Test parameters	2667

A.6.6.4.5.3	Test Requirements	2670

A.6.6.4.6	Inter-cell SSB based L1-RSRP measurements on FR1 PCell when DRX is used	2671

A.6.6.4.6.1	Test Purpose and Environment	2671

A.6.6.4.6.2	Test parameters	2671

A.6.6.4.6.3	Test Requirements	2674

A.6.6.5	Inter-RAT UTRAN FDD measurements	2675

A.6.6.5.1	SA NR - UTRAN FDD event-triggered reporting in non-DRX in FR1	2675

A.6.6.5.1.1	Test Purpose and Environment	2675

A.6.6.5.1.2	Test Requirements	2678

A.6.6.6	CLI measurements	2678

A.6.6.6.1	SRS-RSRP measurement with DRX	2678

A.6.6.6.1.1	Test Purpose and Environment	2678

A.6.6.6.1.2	Test Parameters	2679

A.6.6.6.1.3	Test Requirements	2682

A.6.6.6.2	CLI-RSSI measurement with DRX	2682

A.6.6.6.2.1	Test Purpose and Environment	2682

A.6.6.6.2.2	Test Parameters	2683

A.6.6.6.2.3	Test Requirements	2685

A.6.6.7	NR measurements with autonomous gaps	2685

A.6.6.7.1	SA intra-frequency CGI identification of NR neighbor cell in FR1	2685

A.6.6.7.1.1	Test Purpose and Environment	2685

A.6.6.7.1.2	Test Parameters	2685

A.6.6.7.1.3	Test Requirements	2688

A.6.6.7.2	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA	2688

A.6.6.7.2.1	Test Purpose and Environment	2688

A.6.6.7.2.2	Test Requirements	2691

A.6.6.8	L1-SINR measurement for beam reporting	2692

A.6.6.8.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	2694

A.6.6.8.2.1	Test Purpose and Environment	2694

A.6.6.8.2.2	Test parameters	2694

A.6.6.8.2.3	Test Requirements	2699

A.6.6.8.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	2699

A.6.6.8.3.1	Test Purpose and Environment	2699

A.6.6.8.3.2	Test parameters	2700

A.6.6.8.3.3	Test Requirements	2702

A.6.6.9	Idle Mode CA/DC Measurements	2702

A.6.6.9.1	SA Idle mode CA/DC measurement for FR1	2702

A.6.6.9.1.1	Test Purpose and Environment	2702

A.6.6.9.1.2	Test Requirements	2709

A.6.6.10	CSI-RS based intra-frequency Measurements	2709

A.6.6.10.1	SA event triggered reporting tests without gap under non-DRX	2709

A.6.6.10.1.1	Test purpose and Environment	2709

A.6.6.10.1.2	Test Requirements	2712

A.6.6.11	CSI-RS based inter-frequency Measurements	2712

A.6.6.11.1	 SA event triggered reporting tests with gap under DRX	2712

A.6.6.11.1.1	Test Purpose and Environment	2712

A.6.6.11.1.2	Test Requirements	2716

A.6.6.12	RSTD measurements	2716

A.6.6.12.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA	2716

A.6.6.12.1.1	Test Purpose and Environment	2716

A.6.6.12.1.2	Test Requirements	2723

A.6.6.12.2	NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR1 SA	2724

A.6.6.12.2.1	Test Purpose and Environment	2724

A.6.6.12.2.2	Test Requirements	2730

A.6.6.12.3	NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA	2730

A.6.6.12.3.1	Test Purpose and Environment	2730

A.6.6.12.3.2	Test Requirements	2736

A.6.6.12.4	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA without measurement gap	2737

A.6.6.12.4.1	Test Purpose and Environment	2737

A.6.6.12.4.2	Test Requirements	2741

A.6.6.12.5	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_CONNECTED state with Rx TEG	2741

A.6.6.12.5.1	Test Purpose and Environment	2741

A.6.6.12.5.2	Test Requirements	2745

A.6.6.13 PRS-RSRP measurements	2746

A.6.6.13.1	PRS-RSRP reporting delay test case for single positioning frequency layer	2746

A.6.6.13.1.1	Test purpose and Environment	2746

A.6.6.13.1.2	Test Requirements	2750

The rate of correct events observed during repeated tests shall be at least 90%A.6.6.13.2	PRS-RSRP reporting delay test case for dual positioning frequency layer	2750

A.6.6.13.2.1	Test purpose and Environment	2750

A.6.6.13.2.2	Test Requirements	2754

A.6.6.13.3	PRS-RSRP reporting delay test case for reduced number of samples	2754

A.6.6.13.3.1	Test purpose and Environment	2754

A.6.6.13.3.2	Test Requirements	2757

A.6.6.13.4	PRS-RSRP reporting delay test case for single positioning frequency layer outside MG	2757

A.6.6.13.4.1	Test purpose and Environment	2757

A.6.6.14	UE Rx-Tx time difference measurements	2762

A.6.6.14.1	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA	2762

A.6.6.14.1.1	Test purpose and environment	2762

A.6.6.14.1.2	Test requirements	2766

A.6.6.14.2	UE Rx-Tx time difference measurement for dual positioning frequency layers in FR1 SA	2766

A.6.6.14.2.1	Test purpose and environment	2766

A.6.6.14.2.2	Test requirements	2770

A.6.6.14.3	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA with reduced sample number	2770

A.6.6.14.3.1	Test purpose and environment	2770

A.6.6.14.3.2	Test requirements	2773

A.6.6.14.4	UE Rx-Tx time difference measurement without gaps in FR1 SA	2773

A.6.6.14.4.1	Test purpose and environment	2773

A.6.6.14.4.2	Test requirements	2776

A.6.6.14.5	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA with multiple RxTx TEGs	2776

A.6.6.14.4.1	Test purpose and environment	2776

A.6.6.14.4.2	Test requirements	2780

A.6.6.15	Idle Mode measurements of inter-RAT DC candidate cells for early reporting	2780

A.6.6.15.1	Test Purpose and Environment	2780

A.6.6.15.2	Test Requirements	2787

A.6.6.16	PRS-RSRPP measurements	2788

A.6.6.16.1	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_CONNECTED state	2788

A.6.6.16.1.1	Test purpose and Environment	2788

A.6.6.16.1.2	Test Requirements	2790

A.6.6.16.2	PRS-RSRPP reporting delay test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_CONNECTED state	2790

A.6.6.16.2.1	Test purpose and Environment	2790

A.6.6.16.2.2	Test Requirements	2793

A.6.6.16.3	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_CONNECTED state without measurement gap	2793

A.6.6.16.3.1	Test purpose and Environment	2793

A.6.6.16.3.2	Test Requirements	2796

A.6.6.17	SA event triggered reporting tests with Pre-MG	2796

A.6.6.17.1	SA event triggered reporting tests with autonomous activation/deactivation Pre-MG	2796

A.6.6.17.1.1	Test purpose and Environment	2796

A.6.6.17.1.2	Test parameters	2796

A.6.6.17.1.3	Test Requirements	2801

A.6.6.17.2	SA event triggered reporting tests with pre-configured measurement gaps and network-controlled activation/deactivation	2801

A.6.6.17.2.1	Test purpose and Environment	2801

A.6.6.17.2.2	Test parameters	2801

A.6.6.17.2.3	Test Requirements	2806

A.6.6.17.3	Void	2806

A.6.6.17.3.1	Void	2806

A.6.6.17.3.2	Void	2806

A.6.6.17.3.3	Void	2806

A.6.6.18	SA event triggered reporting tests with concurrent gaps	2806

A.6.6.18.1	SA event triggered reporting tests for FR1 concurrent gaps with non-overalpping scenario for SSB-based measurements in both inter-frequency layers	2806

A.6.6.18.1.1	Test Purpose and Environment	2806

A.6.6.18.1.2	Test Requirements	2810

A.6.6.18.2	SA event triggered reporting tests for FR1 concurrent gap with partially partial overalpping scenario for SSB-based measurements in both inter-frequency layers	2810

A.6.6.18.2.1	Test Purpose and Environment	2810

A.6.6.18.2.2	Test Requirements	2814

A.6.6.18.3	SA NR - E-UTRAN and NR FR1 concurrent event-triggered reporting in non-DRX in FR1	2814

A.6.6.18.3.1	Test Purpose and Environment	2814

A.6.6.18.3.2	Test Requirements	2821

A.6.6.18.4	SA event triggered reporting tests for PRS and SSB measurement in FR1 without SSB time index detection when DRX is not used	2821

A.6.6.18.4.1	Test Purpose and Environment	2821

A.6.6.18.4.2	Test Requirements	2825

A.6.6.19	SA event triggered reporting tests with NCSG	2826

A.6.6.19.1	SA event triggered reporting tests with NCSG under non-DRX in FR1	2826

A.6.6.19.1.1	Test purpose and Environment	2826

A.6.6.19.1.2	Test parameters	2826

A.6.6.19.1.3	Test Requirements	2830

A.6.6.19.2	SA event triggered reporting tests for FR1 with NCSG for inter-frequency measurement	2830

A.6.6.19.2.1	Test Purpose and Environment	2830

A.6.6.19.2.2	Test parameters	2830

A.6.6.19.2.3	Test Requirements	2834

A.6.6.19.3	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 with NCSG	2834

A.6.6.19.3.1	Test Purpose and Environment	2834

A.6.6.19.3.2	Test parameters	2835

A.6.6.19.3.3	Test Requirements	2840

A.6.6.19.4	Event triggered reporting on SCC with deactivated SCell test with per-UE NCSG under non-DRX	2840

A.6.6.19.4.1	Test purpose and Environment	2840

A.6.6.19.4.2	Test parameters	2840

A.6.6.19.4.3	Test Requirements	2843

A.6.6.20	UE Rx-Tx time difference measurement for propagation delay compensation	2843

A.6.6.20.1	Test purpose and environment	2843

A.6.6.20.2	Test requirements	2847

A.6.6.21	UE Rx-Tx time difference measurement with TRS for RTT-based PDC in FR1 SA	2847

A.6.6.21.1	Test purpose and environment	2847

A.6.6.21.2	Test requirements	2849

A.6.7	Measurement Performance requirements	2850

A.6.7.1	SS-RSRP	2850

A.6.7.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	2850

A.6.7.1.1.1	Test Purpose and Environment	2850

A.6.7.1.1.2	Test parameters	2850

A.6.7.1.1.3	Test Requirements	2855

A.6.7.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	2855

A.6.7.1.2.1	Test Purpose and Environment	2855

A.6.7.1.2.2	Test parameters	2855

A.6.7.1.2.3	Test Requirements	2859

A.6.7.1.3	Void	2859

A.6.7.2	SS-RSRQ	2859

A.6.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2859

A.6.7.2.1.1	Test Purpose and Environment	2859

A.6.7.2.1.2	Test Parameters	2860

A.6.7.2.1.3	Test Requirements	2864

A.6.7.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2864

A.6.7.2.2.1	Test Purpose and Environment	2864

A.6.7.2.2.2	Test Parameters	2864

A.6.7.2.2.3	Test Requirements	2869

A.6.7.3	SS-SINR	2869

A.6.7.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2869

A.6.7.3.1.1	Test Purpose and Environment	2869

A.6.7.3.1.2	Test Parameters	2870

A.6.7.3.1.3	Test Requirements	2874

A.6.7.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2875

A.6.7.3.2.1	Test Purpose and Environment	2875

A.6.7.3.2.2	Test Parameters	2875

A.6.7.3.2.3	Test Requirements	2879

A.6.7.4	L1-RSRP measurement for beam reporting	2880

A.6.7.4.1	SSB based L1-RSRP measurement	2880

A.6.7.4.1.1	Test Purpose and Environment	2880

A.6.7.4.1.2	Test parameters	2880

A.6.7.4.1.3	Test Requirements	2884

A.6.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	2884

A.6.7.4.2.1	Test Purpose and Environment	2884

A.6.7.4.2.2	Test parameters	2885

A.6.7.4.2.3	Test Requirements	2889

A.6.7.5	E-UTRAN RSRP	2889

A.6.7.5.1	SA: inter-RAT measurement accuracy with FR1 serving cell	2889

A.6.7.5.1.1	Test Purpose and Environment	2889

A.6.7.5.1.2	Test parameters	2890

A.6.7.5.1.3	Test Requirements	2896

A.6.7.6	E-UTRAN RSRQ	2897

A.6.7.6.1	SA: inter-RAT measurement accuracy with FR1 serving cell	2897

A.6.7.6.1.1	Test Purpose and Environment	2897

A.6.7.6.1.2	Test parameters	2897

A.6.7.6.1.3	Test Requirements	2902

A.6.7.7	E-UTRAN RS-SINR	2903

A.6.7.7.1	SA: inter-RAT measurement accuracy with FR1 serving cell	2903

A.6.7.7.1.1	Test Purpose and Environment	2903

A.6.7.7.1.2	Test parameters	2903

A.6.7.7.1.3	Test Requirements	2909

A.6.7.8	CLI measurements	2910

A.6.7.8.1	SA SRS-RSRP measurement accuracy with FR1 serving cell	2910

A.6.7.8.1.1	Test Purpose and Environment	2910

A.6.7.8.1.2	Test parameters	2910

A.6.7.8.1.3	Test Requirements	2916

A.6.7.8.2	SA CLI-RSSI measurement accuracy with FR1 serving cell	2916

A.6.7.8.2.1	Test Purpose and Environment	2916

A.6.7.8.2.2	Test parameters	2917

A.6.7.8.2.3	Test Requirements	2920

A.6.7.9	L1-SINR measurement for beam reporting	2921

A.6.7.9.2	L1-SINR measurement with SSB based CMR and dedicated IMR	2925

A.6.7.9.2.1	Test Purpose and Environment	2925

A.6.7.9.2.2	Test parameters	2926

A.6.7.9.2.3	Test Requirements	2931

A.6.7.9.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	2931

A.6.7.9.3.1	Test Purpose and Environment	2931

A.6.7.9.3.2	Test parameters	2931

A.6.7.9.3.3	Test Requirements	2936

A.6.7.10	CSI-RSRP	2936

A.6.7.10.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	2936

A.6.7.9.10.1	Test Purpose and Environment	2936

A.6.7.9.10.2	Test parameters	2937

A.6.7.10.1.3	Test Requirements	2942

A.6.7.10.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	2942

A.6.7.9.10.1	Test Purpose and Environment	2942

A.6.7.10.2.2	Test parameters	2942

A.6.7.10.2.3	Test Requirements	2947

A.6.7.11	CSI-RSRQ	2947

A.6.7.11.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2947

A.6.7.11.1.1	Test Purpose and Environment	2947

A.6.7.11.1.2	Test Parameters	2947

A.6.7.11.1.3	Test Requirements	2952

A.6.7.11.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2952

A.6.7.11.2.1	Test Purpose and Environment	2952

A.6.7.11.2.2	Test Parameters	2952

A.6.7.11.2.3	Test Requirements	2958

A.6.7.12	CSI-SINR	2959

A.6.7.12.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2959

A.6.7.12.1.1	Test Purpose and Environment	2959

A.6.7.12.1.2	Test Parameters	2959

A.6.7.12.1.3	Test Requirements	2963

A.6.7.12.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2964

A.6.7.12.2.1	Test Purpose and Environment	2964

A.6.7.12.2.2	Test Parameters	2964

A.6.7.12.2.3	Test Requirements	2970

A.6.7.13	RSTD measurements	2970

A.6.7.13.1	RSTD measurement accuracy test case for single positioning frequency layer	2970

A.6.7.13.1.1	Test purpose and Environment	2970

A.6.7.13.1.2	Test Requirements	2973

A.6.7.13.2	RSTD measurement accuracy test case for dual positioning frequency layer	2973

A.6.7.13.2.1	Test purpose and Environment	2973

A.6.7.13.2.2	Test Requirements	2977

A.6.7.13.3	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_CONNECTED state	2977

A.6.7.13.3.1	Test purpose and Environment	2977

A.6.7.13.3.2	Test Requirements	2979

A.6.7.13.4	RSTD measurement accuracy test case with Rx TEG	2980

A.6.7.14	PRS-RSRP measurements	2982

A.6.7.14.1	SA: measurement accuracy with PRS in FR1	2982

A.6.7.14.1.1	Test Purpose and Environment	2982

A.6.7.14.1.2	Test parameters	2982

A.6.7.14.1.3	Test Requirements	2986

A.6.7.14.2	SA: measurement accuracy with PRS in FR1 with reduced sample number	2986

A.6.7.14.2.1	Test Purpose and Environment	2986

A.6.7.14.2.2	Test parameters	2986

A.6.7.14.2.3	Test Requirements	2989

A.6.7.14.3	Void	2989

A.6.7.14.3.1	Void	2989

A.6.7.14.3.2	Void	2989

A.6.7.14.3.3	Void	2989

A.6.7.15	UE Rx-Tx time difference measurements	2989

A.6.7.15.1	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR1 SA	2989

A.6.7.15.1.1	Test purpose and environment	2989

A.6.7.15.1.2	Test parameters	2990

A.6.7.15.1.3	Test requirements	2993

A.6.7.15.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR1 SA	2994

A.6.7.15.2.1	Test purpose and environment	2994

A.6.7.15.2.2	Test parameters	2994

A.6.7.15.2.3	Test requirements	2996

A.6.7.15.3	UE Rx-Tx time difference measurement accuracy with RxTx TEG	2996

A.6.7.15.3.1	Test purpose and environment	2996

A.6.7.15.3.2	Test parameters	2997

A.6.7.15.3.3	Test requirements	3000

A.6.7.16	PRS-RSRPP measurements	3000

A.6.7.16.1	SA: measurement accuracy with PRS in FR1	3000

A.6.7.16.1.1	Test Purpose and Environment	3000

A.6.7.16.1.2	Test parameters	3001

A.6.7.16.1.3	Test Requirements	3005

A.6.7.16.2	SA: measurement accuracy with reduced PRS samples in FR1	3005

A.6.7.16.2.1	Test Purpose and Environment	3005

A.6.7.16.2.2	Test parameters	3005

A.6.8	Measurement procedure in RRC_INACTIVE	3009

A.6.8.1	RSTD measurements	3009

A.6.8.1.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_INACTIVE state	3009

A.6.8.1.1.1	Test Purpose and Environment	3009

A.6.8.1.1.2	Test Requirements	3012

A.6.8.1.2	 NR RSTD measurement reporting delay test case with reduced number of samples in RRC_INACTIVE, FR1 SA	3012

A.6.8.1.2.1	Test Purpose and Environment	3012

A.6.8.1.2.2	Test Requirements	3017

A.6.8.2	PRS-RSRP measurements	3017

A.6.8.2.1	PRS-RSRP reporting delay test case for single positioning frequency layer in RRC_INACTIVE	3017

A.6.8.2.1.1	Test purpose and Environment	3017

A.6.8.2.1.2	Test Requirements	3019

A.6.8.2.2	PRS-RSRP reporting delay test case with reduced number of samples in RRC_INACTIVE	3020

A.6.8.2.2.1	Test purpose and Environment	3020

A.6.8.2.2.2	Test Requirements	3024

A.6.8.3	UE Rx-Tx time difference measurements	3024

A.6.8.3.1	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA	3024

A.6.8.3.1.1	Test purpose and environment	3024

A.6.8.3.1.2	Test requirements	3028

A.6.8.3.2	UE Rx-Tx time difference measurement with reduced number of samples in RRC_INACTIVE, FR1 SA	3028

A.6.8.3.2.1	Test purpose and environment	3028

A.6.8.3.2.2	Test requirements	3031

A.6.8.4	PRS-RSRPP measurements	3031

A.6.8.4.1	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_INACTIVE state	3031

A.6.8.4.1.1	Test purpose and Environment	3031

A.6.8.4.1.2	Test Requirements	3034

A.6.8.4.2	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_INACTIVE state for reduced number of samples	3034

A.6.8.4.2.1	Test purpose and Environment	3034

A.6.8.4.2.2	Test Requirements	3036

A.6.9	Measurement performance requirements in RRC_INACTIVE	3037

A.6.9.1	RSTD measurements	3037

A.6.9.1.1	RSTD measurement accuracy test case for single positioning frequency layer in FR1 in RRC_INACTIVE state	3037

A.6.9.1.1.1	Test purpose and Environment	3037

A.6.9.1.1.2	Test Requirements	3039

A.6.9.1.2	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_INACTIVE state	3039

A.6.9.1.2.1	Test purpose and Environment	3039

A.6.9.1.2.2	Test Requirements	3041

A.6.9.2	PRS-RSRP measurements	3041

A.6.9.2.1	SA: measurement accuracy with PRS in FR1 in RRC_INACTIVE	3041

A.6.9.2.1.1	Test Purpose and Environment	3041

A.6.9.2.1.2	Test parameters	3042

A.6.9.2.1.3	Test Requirements	3045

A.6.9.2.2	SA: measurement accuracy with PRS in FR1 with reduced number of samples in RRC_INACTIVE state	3045

A.6.9.2.2.1	Test Purpose and Environment	3045

A.6.9.2.2.2	Test parameters	3045

A.6.9.2.2.3	Test Requirements	3047

A.6.9.3	UE Rx-Tx time difference measurements	3048

A.6.9.3.1.1	UE Rx-Tx time difference measurement accuracy in FR1 SA	3048

A.6.9.3.1.1.1	Test purpose and environment	3048

A.6.9.3.1.1.2 Test parameters	3048

A.6.9.3.1.1.3	Test requirements	3050

A.6.9.3.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples	3050

A.6.9.3.2.1	Test purpose and environment	3050

A.6.9.3.2.2	Test parameters	3050

A.6.9.3.2.3	Test requirements	3053

A.6.9.4	PRS-RSRPP measurements	3053

A.6.9.4.1	SA: PRS-RSRPP measurement accuracy in FR1 in RRC INACTIVE	3053

A.6.9.4.1.1	Test Purpose and Environment	3053

A.6.9.4.1.2	Test parameters	3053

A.6.9.4.1.3	Test Requirements	3057

A.6.9.4.2	SA: measurement accuracy with reduced PRS samples in FR1 in RRC INACTIVE	3057

A.6.9.4.2.1	Test Purpose and Environment	3057

A.6.9.4.2.2	Test parameters	3057

A.6.9.4.2.3	Test Requirements	3059

A.7	NR standalone tests with one or more NR cells in FR2	3067

A.7.1	SA: RRC_IDLE state mobility	3067

A.7.1.1	Cell re-selection to NR	3067

A.7.1.1.1	Cell reselection to FR2 intra-frequency NR case	3067

A.7.1.1.1.1	Test Purpose and Environment	3067

A.7.1.1.1.2	Test Parameters	3067

A.7.1.1.1.3	Test Requirements	3070

A.7.1.1.2	Cell reselection to FR2 inter-frequency NR case	3070

A.7.1.1.2.1	Test Purpose and Environment	3070

A.7.1.1.2.2	Test Parameters	3071

A.7.1.1.2.3	Test Requirements	3073

A.7.1.1.3	Cell reselection to FR2 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	3073

A.7.1.1.3.1	Test Purpose and Environment	3073

A.7.1.1.3.2	Test Parameters	3073

A.7.1.1.3.3	Test Requirements	3076

A.7.1.1.4	Cell reselection to FR2 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	3076

A.7.1.1.4.1	Test Purpose and Environment	3076

A.7.1.1.4.2	Test Parameters	3076

A.7.1.1.4.3	Test Requirements	3079

A.7.1.1.5	Cell reselection to FR2 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	3079

A.7.1.1.5.1	Test Purpose and Environment	3079

A.7.1.1.5.2	Test Parameters	3079

A.7.1.1.5.3	Test Requirements	3082

A.7.1.1.6	Cell reselection to FR2 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	3082

A.7.1.1.6.1	Test Purpose and Environment	3082

A.7.1.1.6.2	Test Parameters	3082

A.7.1.1.6.3	Test Requirements	3085

A.7.1.1.7	Cell reselection to FR2 intra-frequency NR case for FR2 power class 6 UE configured with highSpeedMeasFlagFR2-r17	3085

A.7.1.1.7.1	Test Purpose and Environment	3085

A.7.1.1.7.2	Test Parameters	3085

A.7.1.1.7.3	Test Requirements	3089

A.7.2	SA: RRC_INACTIVE state mobility	3089

A.7.2.1	Small Data Transmission	3089

A.7.2.1.1	TA validation for CG-SDT in FR2	3089

A.7.2.1.1.1	Test Purpose and Environment	3089

A.7.2.1.1.2	Test Requirements	3093

A.7.3	RRC_CONNECTED state mobility	3093

A.7.3.1	Handover	3093

A.7.3.1.1	Inter-frequency handover from FR1 to FR2; unknown target cell	3093

A.7.3.1.1.1	Test Purpose and Environment	3093

A.7.3.1.1.2	Test Parameters	3093

A.7.3.1.1.3	Test Requirements	3097

A.7.3.1.2	Intra-frequency handover from FR2 to FR2; unknown target cell	3097

A.7.3.1.2.1	Test Purpose and Environment	3097

A.7.3.1.2.2	Test Parameters	3097

A.7.3.1.2.3	Test Requirements	3100

A.7.3.1.3	Inter-frequency handover from FR2 to FR2; unknown target cell	3100

A.7.3.1.3.1	Test Purpose and Environment	3100

A.7.3.1.3.2	Test Parameters	3100

A.7.3.1.3.3	Test Requirements	3101

A.7.3.1.4	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR2	3102

A.7.3.1.4.1	Test Purpose and Environment	3102

A.7.3.1.4.2	Test Parameters	3102

A.7.3.1.4.3 Test Requirements	3109

A.7.3.1.5	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR2	3109

A.7.3.1.5.1	Test Purpose and Environment	3109

A.7.3.1.5.2	Test Parameters	3109

A.7.3.1.5.3 Test Requirements	3116

A.7.3.1.6	Handover with PSCell from SA to EN-DC with; unknown FR2 target PScell	3116

A.7.3.1.6.1	Test Purpose and Environment	3116

A.7.3.1.6.2	Test Parameters	3116

A.7.3.1.6.3	Test Requirements	3124

A.7.3.1.7	HO with PSCell from FR1 NR-SA to EN-DC with known E-UTRA PCell and known FR2 PSCell	3125

A.7.3.1.7.1	Test purpose and environment	3125

A.7.3.1.7.2	Test Requirements	3130

A.7.3.1.8	NR PSCell change delay in HO with PSCell from NR-DC to NR-DC	3131

A.7.3.1.8.1	Test Purpose and Environment	3131

A.7.3.1.8.2	Test Requirements	3136

A.7.3.1.9	Intra-frequency handover from FR2-2 to FR2-2; unknown target cell	3136

A.7.3.1.9.1	Test Purpose and Environment	3136

A.7.3.1.9.2	Test Parameters	3136

A.7.3.1.9.3	Test Requirements	3139

A.7.3.1.10	Inter-frequency handover from FR2-2 to FR2-2; unknown target cell	3139

A.7.3.1.10.1	Test Purpose and Environment	3139

A.7.3.1.10.2	Test Parameters	3139

A.7.3.1.10.3	Test Requirements	3143

A.7.3.1.11	Inter-frequency handover from FR1 to FR2-2; unknown target cell	3143

A.7.3.1.11.1	Test Purpose and Environment	3143

A.7.3.1.11.2	Test Parameters	3143

A.7.3.1.11.3	Test Requirements	3147

A.7.3.2	RRC Connection Mobility Control	3147

A.7.3.2.1	SA: RRC Re-establishment	3147

A.7.3.2.1.1	Intra-frequency RRC Re-establishment in FR2	3147

A.7.3.2.1.2	Inter-frequency RRC Re-establishment in FR2	3150

A.7.3.2.1.3	Intra-frequency RRC Re-establishment in FR2 without serving cell timing	3153

A.7.3.2.1.3.1	Test Purpose and Environment	3153

A.7.3.2.1.3.2	Test Requirements	3155

A.7.3.2.1.4	Intra-frequency RRC Re-establishment in FR2-2	3156

A.7.3.2.1.4.1	Test Purpose and Environment	3156

A.7.3.2.1.4.2	Test Requirements	3158

A.7.3.2.1.5	Inter-frequency RRC Re-establishment in FR2-2	3159

A.7.3.2.1.5.1	Test Purpose and Environment	3159

A.7.3.2.1.5.2	Test Requirements	3162

A.7.3.2.1.6	Intra-frequency RRC Re-establishment in FR2-2 without serving cell timing	3162

A.7.3.2.1.6.1	Test Purpose and Environment	3162

A.7.3.2.1.6.2	Test Requirements	3164

A.7.3.2.2	Random Access	3165

A.7.3.2.2.1	4-step RA type c ontention based random access test in FR2 for NR Standalone	3165

A.7.3.2.2.2	4-step RA type n on-contention based random access test in FR2 for NR Standalone	3169

A.7.3.2.2.3	2-step RA type contention based random access test in FR2 for NR Standalone	3173

A.7.3.2.2.4	2-step RA type n on-contention based random access test in FR2 for NR Standalone	3176

A.7.3.2.3	SA: RRC Connection Release with Redirection	3179

A.7.3.2.3.1	Redirection from NR in FR2 to NR in FR2	3179

A.7.3.3 Conditional Handover	3183

A.7.3.3.1	Intra-frequency conditional handover from FR2 to FR2	3183

A.7.3.3.1.1	Test Purpose and Environment	3183

A.7.3.3.1.2	Test Parameters	3183

A.7.3.3.1.2.3	Test Requirements	3186

A.7.3.3.2	Inter-frequency conditional handover from FR2 to FR2; unknown target cell	3186

A.7.3.3.2.1	Test Purpose and Environment	3186

A.7.3.3.2.2	Test Parameters	3186

A.7.3.3.2.3 Test Requirements	3188

A.7.4	Timing	3188

A.7.4.1	UE transmit timing	3188

A.7.4.1.1	NR UE Transmit Timing Test for FR2	3188

A.7.4.1.1.1	Test Purpose and environment	3188

A.7.4.1.1.2	Test requirements	3191

A.7.4.1.2	NR UE Transmit Timing Test for FR2-2	3191

A.7.4.1.2.1	Test Purpose and environment	3191

A.7.4.1.2.2	Test requirements	3196

A.7.4.2	UE timer accuracy	3197

A.7.4.3	Timing advance	3197

A.7.4.3.1	SA FR2 timing advance adjustment accuracy	3197

A.7.4.3.1.1	Test Purpose and Environment	3197

A.7.4.3.1.2	Test Parameters	3197

A.7.4.3.1.3 Test Requirements	3200

A.7.4.3.2	SA FR2-2 timing advance adjustment accuracy	3201

A.7.4.3.2.1	Test Purpose and Environment	3201

A.7.4.3.2.2	Test Parameters	3201

A.7.4.3.2.3	Test Requirements	3205

A.7.5	Signaling characteristics	3205

A.7.5.1	Radio link Monitoring	3205

A.7.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	3205

A.7.5.1.1.1	Test Purpose and Environment	3205

A.7.5.1.1.2	Test Requirements	3208

A.7.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	3209

A.7.5.1.2.1	Test Purpose and Environment	3209

A.7.5.1.2.2	Test Requirements	3213

A.7.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	3214

A.7.5.1.3.1	Test Purpose and Environment	3214

A.7.5.1.3.2	Test Requirements	3218

A.7.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	3218

A.7.5.1.4.1	Test Purpose and Environment	3218

A.7.5.1.4.2	Test Requirements	3223

A.7.5.1.5	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	3223

A.7.5.1.5.1	Test Purpose and Environment	3223

A.7.5.1.5.2	Test Requirements	3228

A.7.5.1.6	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	3228

A.7.5.1.6.1	Test Purpose and Environment	3228

A.7.5.1.6.2	Test Requirements	3232

A.7.5.1.7	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	3232

A.7.5.1.7.1	Test Purpose and Environment	3232

A.7.5.1.7.2	Test Requirements	3236

A.7.5.1.8	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	3236

A.7.5.1.8.1	Test Purpose and Environment	3236

A.7.5.1.8.2	Test Requirements	3241

A.7.5.1.9	UE Radio Link Monitoring Scheduling Restrictions on FR2	3241

A.7.5.1.9.1	Test Purpose and Environment	3241

A.7.5.1.9.2	Test Requirements	3244

A.7.5.2	Interruption	3244

A.7.5.2.1	Interruptions during measurements on deactivated NR SCC in FR2	3244

A.7.5.2.1.1	Test Purpose and Environment	3244

A.7.5.2.1.2	Test Requirements	3247

A.7.5.2.2	SA interruptions at NR SRS carrier-based switching	3248

A.7.5.2.2.1	Test Purpose and Environment	3248

A.7.5.2.2.2	Test Parameters	3248

A.7.5.2.2.3	Test Requirements	3250

A.7.5.3	SCell Activation and Deactivation Delay	3250

A.7.5.3.1	SCell Activation and deactivation for SCell in FR2 intra-band in non-DRX	3250

A.7.5.3.1.1	Test Purpose and Environment	3250

A.7.5.3.1.2	Test Requirements	3252

A.7.5.3.2	SCell Activation and deactivation for FR1+FR2 inter-band with target SCell in FR2	3252

A.7.5.3.2.1	Test Purpose and Environment	3252

A.7.5.3.2.2	Test Requirements	3256

A.7.5.3.3	SCell Activation and deactivation for SCell in FR2 inter-band in non-DRX	3257

A.7.5.3.3.1	Test Purpose and Environment	3257

A.7.5.3.3.2	Test Requirements	3260

A.7.5.3.4	Direct SCell activation at SCell addition of known SCell in FR2	3261

A.7.5.3.4.1	Test Purpose and Environment	3261

A.7.5.3.4.2	Test Requirements	3264

A.7.5.3.5	Direct SCell activation at handover with known SCell in FR2	3265

A.7.5.3.5.1	Test Purpose and Environment	3265

A.7.5.3.5.2	Test Requirements	3268

A.7.5.3.6	PUCCH SCell activation and deactivation for FR1+FR2 inter-band with target SCell in FR2 and known	3269

A.7.5.3.6.1	Test Purpose and Environment	3269

A.7.5.3.6.2	Test Requirements	3273

A.7.5.3.7	PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with FR1 PCell	3274

A.7.5.3.7.1	Test Purpose and Environment	3274

A.7.5.3.7.2	Test Requirements	3278

A.7.5.3.8	SCell Activation and deactivation for known PUCCH SCell in FR2 inter-band in non-DRX	3279

A.7.5.3.8.1	Test Purpose and Environment	3279

A.7.5.3.8.2	Test Requirements	3283

A.7.5.3.9	PUCCH SCell Activation and deactivation of unknown SCell in FR2	3284

A.7.5.3.9.1	Test Purpose and Environment	3284

A.7.5.3.9.2	Test Requirements	3287

A.7.5.3.10	SCell Activation and deactivation of FR2 known PUCCH SCell and one FR2 unknown SCell with FR2 PCell	3288

A.7.5.3.10.1	Test Purpose and Environment	3288

A.7.5.3.10.2	Test Requirements	3292

A.7.5.3.11	PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with FR2 PCell	3293

A.7.5.3.11.1	PUCCH SCell activation with non-PUCCH SCell in a secondary PUCCH Group	3293

A.7.5.3.11.1.1	Test Purpose and Environment	3293

A.7.5.3.11.1.2	Test Requirements	3297

A.7.5.3.11.2	PUCCH SCell activation with non-PUCCH SCell in a primary PUCCH Group	3298

A.7.5.3.11.2.1	Test Purpose and Environment	3298

A.7.5.3.11.2.2	Test Requirements	3302

A.7.5.3.12	Void	3303

A.7.5.3.13	SCell Activation for SCell in FR2 intra-band in non-DRX	3303

A.7.5.3.13.1	Test Purpose and Environment	3303

A.7.5.3.13.2	Test Requirements	3305

A.7.5.3.14	SCell Activation for known SCell in FR2 inter-band	3306

A.7.5.3.14.1	Test Purpose and Environment	3306

A.7.5.3.14.2	Test Requirements	3309

A.7.5.15	Void	3310

A.7.5.4	Void	3310

A.7.5.5	Beam Failure Detection and Link recovery procedures	3310

A.7.5.5.1	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	3310

A.7.5.5.1.1	Test Purpose and Environment	3310

A.7.5.5.1.2	Test Requirements	3314

A.7.5.5.2	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode	3315

A.7.5.5.2.1	Test Purpose and Environment	3315

A.7.5.5.2.2	Test Requirements	3318

A.7.5.5.3	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	3319

A.7.5.5.3.1	Test Purpose and Environment	3319

A.7.5.5.3.2	Test Requirements	3323

A.7.5.5.4	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode	3324

A.7.5.5.4.1	Test Purpose and Environment	3324

A.7.5.5.4.2	Test Requirements	3328

A.7.5.5.5	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	3329

A.7.5.5.5.1	Test Purpose and Environment	3329

A.7.5.5.5.2	Test Requirements	3332

A.7.5.5.6	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	3333

A.7.5.5.6.1	Test Purpose and Environment	3333

A.7.5.5.6.2	Test Requirements	3337

A.7.5.5.7	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	3338

A.7.5.5.7.1	Test Purpose and Environment	3338

A.7.5.5.7.2	Test Requirements	3342

A.7.5.5.8	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode for UE fulfilling relaxed measurement criterion	3343

A.7.5.5.8.1	Test Purpose and Environment	3343

A.7.5.5.8.2	Test Requirements	3347

A.7.5.5.9	TRP specific Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	3347

A.7.5.5.9.1	Test Purpose and Environment	3347

A.7.5.5.9.2	Test Requirements	3353

A.7.5.5.10	TRP specific Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	3353

A.7.5.5.10.1	Test Purpose and Environment	3353

A.7.5.5.10.2	Test Requirements	3359

A.7.5.5.11	Beam Failure Detection and Link Recovery Test for FR2-2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	3359

A.7.5.5.11.1	Test Purpose and Environment	3359

A.7.5.5.11.2	Test Requirements	3364

A.7.5.5.12	Beam Failure Detection and Link Recovery Test for FR2-2 PCell configured with CSI-RS-based BFD and LR in DRX mode	3364

A.7.5.5.12.1	Test Purpose and Environment	3364

A.7.5.5.12.2	Test Requirements	3369

A.7.5.5.13	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2-2 PCell configured with SSB-based BFD and LR in non-DRX mode	3369

A.7.5.5.13.1	Test Purpose and Environment	3369

A.7.5.5.13.2	Test Requirements	3373

A.7.5.6	Active BWP switch	3373

A.7.5.6.1	DCI-based and Timer-based Active BWP Switch	3373

A.7.5.6.1.1	NR FR2- NR FR2 DL active BWP switch of SCell with non-DRX in SA	3373

A.7.5.6.1.2	NR FR1- NR FR2 DL active BWP switch of SCell with non-DRX in SA	3378

A.7.5.6.1.3	NR FR2 DL active BWP switch with non-DRX in SA	3383

A.7.5.6.1.3.1	Test Purpose and Environment	3383

A.7.5.6.1.3.2	Test Requirements	3386

A.7.5.6.1.4	NR FR2-2- NR FR2-2 DL active BWP switch of SCell with non-DRX in SA	3386

A.7.5.6.1.4.1	Test Purpose and Environment	3386

A.7.5.6.1.4.2	Test Requirements	3391

A.7.5.6.2	RRC-based Active BWP Switch	3392

A.7.5.6.2.1.1	Test Purpose and Environment	3392

A.7.5.6.2.1.2	Test Requirements	3395

A.7.5.6.2.2	NR FR2-2 DL active BWP switch of PCell with non-DRX in SA	3396

A.7.5.6.2.2.1	Test Purpose and Environment	3396

A.7.5.6.2.2.2	Test Requirements	3400

A.7.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	3401

A.7.5.6.3.1.1	Test Purpose and Environment	3401

A.7.5.6.3.1.2	Test Requirements	3404

A.7.5.6.4	SCell dormancy switch	3404

A.7.5.6.4.1	NR FR2 PCell SCell dormancy switch of single FR2 SCell inside active time	3404

A.7.5.6.4.1.1	Test Purpose and Environment	3404

A.7.5.6.4.1.2	Test Requirements	3409

A.7.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR2 SCells outside active time	3409

A.7.5.6.4.2.1	 Test Purpose and Environment	3409

A.7.5.6.4.2.2	 Test Requirements	3414

A.7.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	3414

A.7.5.6.5.1	Active BWP switch on multiple SCells with non-DRX in SA	3414

A.7.5.6.5.2	NR FR2-2 Active BWP switch on multiple SCells with non-DRX in SA	3417

A.7.5.6.5.2.1	Test Purpose and Environment	3417

A.7.5.6.5.2.2	Test Requirements	3421

A.7.5.7	PSCell addition and release delay	3422

A.7.5.7.1	Addition and Release Delay of known NR PSCell	3422

A.7.5.7.1.1	Test Purpose and Environment	3422

A.7.5.7.1.2	Test Requirements	3425

A.7.5.7.2	Addition and Release Delay of unknown NR PSCell in	3425

A.7.5.7.2.1	Test Purpose and Environment	3425

A.7.5.7.2.2	Test Requirements	3428

A.7.5.7.3	Addition and Release Delay of known NR PSCell in FR2-2	3428

A.7.5.7.3.1	Test Purpose and Environment	3428

A.7.5.7.3.2	Test Requirements	3431

A.7.5.7.4	Addition and Release Delay of unknown NR PSCell in FR2-2	3431

A.7.5.7.4.1	Test Purpose and Environment	3431

A.7.5.7.4.2	Test Requirements	3434

A.7.5.8	Active TCI state switch delay	3434

A.7.5.8.1	MAC-CE based active TCI state switch	3434

A.7.5.8.2	RRC based active TCI state switch	3438

A.7.5.8.3	MAC-CE based active TCI state switch for HST FR2 scenario	3442

A.7.5.8.3.1	NR PCell FR2 HST active TCI state switch for a known TCI state	3442

A.7.5.8.3.1.1	Test Purpose and Environment	3442

A.7.5.8.3.1.2	Test Requirements	3446

A.7.5.9	Uplink spatial relation switch delay	3447

A.7.5.9.1.1.1	Test Purpose and Environment	3447

A.7.5.9.1.1.2	Test Requirements	3450

A.7.5.9.2	RRC based spatial relation switch	3450

A.7.5.9.2.1	NR PCell FR2 spatial relation switch associated with a known DL-RS	3450

A.7.5.9.2.1.1	Test Purpose and Environment	3450

A.7.5.9.2.1.2	Test Requirements	3453

A.7.5.10	UE specific CBW change	3453

A.7.5.10.1	NR FR2 UE specific CBW change of PCell with non-DRX in SA	3453

A.7.5.10.1.1	Test Purpose and Environment	3453

A.7.5.10.1.2	Test Requirements	3456

A.7.5.11	UE UL carrier RRC reconfiguration Delay	3457

A.7.5.11.1	UE UL carrier RRC reconfiguration Delay	3457

A.7.5.11.1.1	Test Purpose and Environment	3457

A.7.5.11.1.2	Test Requirements	3460

A.7.5.12	Conditional PSCell addition and release delay (FR2 SA)	3460

A.7.5.12.1	Addition and Release Delay of PSCell	3460

A.7.5.12.1.1	Test purpose and environment	3460

A.7.5.12.1.2	Test Parameters	3460

A.7.5.12.1.3	Test Requirements	3463

A.7.5.13	 Unified TCI state switching delay	3463

A.7.5.13.1	MAC-CE based active joint TCI state switching	3463

A.7.5.13.1.1	NR PCell FR2 active joint TCI state switch for a known TCI state	3463

A.7.5.13.1.1.1	Test Purpose and Environment	3463

A.7.5.13.1.1.2	Test parameters	3464

A.7.5.13.1.1.3	Test Requirements	3466

A.7.5.13.2	 MAC-CE based active uplink TCI state switch	3466

A.7.5.13.2.1	 NR FR2 PCell uplink TCI state switch for a known TCI state	3466

A.7.5.13.2.1.1	Test Purpose and Environment	3466

A.7.5.13.2.1.2	Test parameters	3467

A.7.5.13.2.1.3	Test Requirements	3469

A.7.5.13.3	MAC-CE based active downlink TCI state switch	3469

A.7.5.13.3.1	NR PCell FR2 active downlink TCI state switch to cell with additional PCI for a known TCI state	3469

A.7.5.13.3.1.1	Test Purpose and Environment	3469

A.7.5.13.3.1.2	Test Parameters	3470

A.7.5.13.3.1.3	Test Requirements	3473

A.7.5.14	PSCell RACH-less based Activation and deactivation for FR1+FR2 inter-band with target PSCell in FR2	3473

A.7.5.14.1	Test Purpose and Environment	3473

A.7.5.14.2	Test Requirements	3476

A.7.6	Measurement procedure	3477

A.7.6.1	Intra-frequency Measurements	3477

A.7.6.1.1	SA event triggered reporting test without gap under non-DRX	3477

A.7.6.1.1.1	Test purpose and Environment	3477

A.7.6.1.1.2	Test Requirements	3480

A.7.6.1.2	SA event triggered reporting test without gap under DRX	3480

A.7.6.1.2.1	Test purpose and Environment	3480

A.7.6.1.2.2	Test Requirements	3483

A.7.6.1.3	SA event triggered reporting test with per-UE gaps under non-DRX	3484

A.7.6.1.3.1	Test purpose and Environment	3484

A.7.6.1.3.2	Test Requirements	3487

A.7.6.1.4	SA event triggered reporting test with per-UE gaps under DRX	3487

A.7.6.1.4.1	Test purpose and Environment	3487

A.7.6.1.4.2	Test Requirements	3490

A.7.6.1.5	SA event triggered reporting test without gap under non-DRX for UE configured with highSpeedMeasFlagFR2-r17	3491

A.7.6.1.5.1	Test purpose and Environment	3491

A.7.6.1.5.2	Test Requirements	3494

A.7.6.1.6	SA event triggered reporting test without gap under non-DRX for FR2-2	3494

A.7.6.1.6.1	Test purpose and Environment	3494

A.7.6.1.6.2	Test Requirements	3497

A.7.6.1.7	SA event triggered reporting test without gap under DRX for FR2-2	3498

A.7.6.1.7.1	Test purpose and Environment	3498

A.7.6.1.7.2	Test Requirements	3500

A.7.6.1.8	SA event triggered reporting test with per-UE gaps under non-DRX for FR2-2	3501

A.7.6.1.8.1	Test purpose and Environment	3501

A.7.6.1.8.2	Test Requirements	3504

A.7.6.1.9	SA event triggered reporting test with per-UE gaps under DRX for FR2-2	3505

A.7.6.1.9.1	Test purpose and Environment	3505

A.7.6.1.9.2	Test Requirements	3507

A.7.6.1.10	SA event triggered reporting test with SSB time index detection without gap under non-DRX for FR2-2	3508

A.7.6.1.10.1	Test purpose and Environment	3508

A.7.6.1.10.2	Test Requirements	3511

A.7.6.1.11	SA event triggered reporting test with SSB time index detection with per-UE gaps under non-DRX for FR2-2	3512

A.7.6.1.11.1	Test purpose and Environment	3512

A.7.6.1.11.2	Test Requirements	3514

A.7.6.2	Inter-frequency Measurements	3515

A.7.6.2.1	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR2)	3515

A.7.6.2.1.1	Test Purpose and Environment	3515

A.7.6.2.1.2	Test Requirements	3518

A.7.6.2.2	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (Pcell in FR2)	3518

A.7.6.2.2.1	Test Purpose and Environment	3518

A.7.6.2.2.2	Test Requirements	3522

A.7.6.2.3	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR2)	3522

A.7.6.2.3.1	Test Purpose and Environment	3522

A.7.6.2.3.2	Test Requirements	3526

A.7.6.2.4	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (Pcell in FR2)	3526

A.7.6.2.4.1	Test Purpose and Environment	3526

A.7.6.2.4.2	Test Requirements	3530

A.7.6.2.5	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR1)	3530

A.7.6.2.5.1	Test Purpose and Environment	3530

A.7.6.2.5.2	Test Requirements	3534

A.7.6.2.6	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (Pcell in FR1)	3535

A.7.6.2.6.1	Test Purpose and Environment	3535

A.7.6.2.6.2	Test Requirements	3539

A.7.6.2.7	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR1)	3540

A.7.6.2.7.1	Test Purpose and Environment	3540

A.7.6.2.7.2	Test Requirements	3544

A.7.6.2.8	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR1)	3545

A.7.6.2.8.1	Test Purpose and Environment	3545

A.7.6.2.8.2	Test Requirements	3549

A.7.6.2.9	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2) (rel16 additional mandatory gap pattern 17)	3550

A.7.6.2.9.1	Test Purpose and Environment	3550

A.7.6.2.9.2	Test Requirements	3554

A.7.6.2.10	SA event triggered reporting test without gap under non-DRX	3554

A.7.6.2.10.1	Test Purpose and Environment	3554

A.7.6.2.10.2	Test Requirements	3556

A.7.6.2.11	SA event triggered reporting test without gap under DRX	3556

A.7.6.2.11.1	Test Purpose and Environment	3556

A.7.6.2.11.2	Test Requirements	3559

A.7.6.2.12	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is not used (PCell in FR2-2)	3559

A.7.6.2.12.1	Test Purpose and Environment	3559

A.7.6.2.12.2	Test Requirements	3563

A.7.6.2.13	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is used (PCell in FR2-2)	3564

A.7.6.2.13.1	Test Purpose and Environment	3564

A.7.6.2.13.2	Test Requirements	3568

A.7.6.2.14	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is not used (PCell in FR2-2)	3569

A.7.6.2.14.1	Test Purpose and Environment	3569

A.7.6.2.14.2	Test Requirements	3573

A.7.6.2.15	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is used (PCell in FR2-2)	3574

A.7.6.2.15.1	Test Purpose and Environment	3574

A.7.6.2.15.2	Test Requirements	3578

A.7.6.2.16	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is not used (PCell in FR1)	3579

A.7.6.2.16.1	Test Purpose and Environment	3579

A.7.6.2.16.2	Test Requirements	3585

A.7.6.2.17	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is used (PCell in FR1)	3585

A.7.6.2.17.1	Test Purpose and Environment	3585

A.7.6.2.17.2	Test Requirements	3592

A.7.6.2.18	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is not used (PCell in FR1)	3593

A.7.6.2.18.1	Test Purpose and Environment	3593

A.7.6.2.18.2	Test Requirements	3598

A.7.6.2.19	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is used (PCell in FR1)	3599

A.7.6.2.19.1	Test Purpose and Environment	3599

A.7.6.2.19.2	Test Requirements	3607

A.7.6.3	L1-RSRP measurement for beam reporting	3608

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	3608

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	3608

A.7.6.3.1.1	Test Purpose and Environment	3608

A.7.6.3.1.2	Test parameters	3608

A.7.6.3.1.3	Test Requirements	3610

A.7.6.3.2	SSB based L1-RSRP measurement when DRX is used	3610

A.7.6.3.2.1	Test Purpose and Environment	3610

A.7.6.3.2.2	Test parameters	3611

A.7.6.3.2.3	Test Requirements	3613

A.7.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	3613

A.7.6.3.3.1	Test Purpose and Environment	3613

A.7.6.3.3.2	Test parameters	3614

A.7.6.3.3.3	Test Requirements	3616

A.7.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	3617

A.7.6.3.4.1	Test Purpose and Environment	3617

A.7.6.3.4.2	Test parameters	3617

A.7.6.3.3.3	Test Requirements	3619

A.7.6.3.5	SSB based L1-RSRP measurement when DRX is used for power class 6 UE configured with highSpeedMeasFlagFR2-r17	3620

A.7.6.3.5.1	Test Purpose and Environment	3620

A.7.6.3.5.2	Test parameters	3620

A.7.6.3.5.3	Test Requirements	3622

A.7.6.3.6	Inter-cell SSB based L1-RSRP measurements on FR2 SCell when DRX is not used	3622

A.7.6.3.6.1	Test Purpose and Environment	3622

A.7.6.3.6.2	Test parameters	3623

A.7.6.3.6.3	Test Requirements	3626

A.7.6.3.7	SSB based L1-RSRP measurement for FR2-2 when DRX is used	3626

A.7.6.3.7.1	Test Purpose and Environment	3626

A.7.6.3.7.2	Test parameters	3627

A.7.6.3.7.3	Test Requirements	3630

A.7.6.4	CLI measurements	3631

A.7.6.4.1	SRS-RSRP measurement with non-DRX	3631

A.7.6.4.1.1	Test Purpose and Environment	3631

A.7.6.4.1.2	Test Parameters	3631

A.7.6.4.1.3	Test Requirements	3633

A.7.6.4.2	CLI-RSSI measurement with non-DRX	3633

A.7.6.4.2.1	Test Purpose and Environment	3633

A.7.6.4.2.2	Test Parameters	3634

A.7.6.4.2.3	Test Requirements	3635

A.7.6.5.1   SA interfrequency CGI reporting in autonomous gaps test (PCell in FR2)	3636

A.7.6.5.1.1	Test Purpose and Environment	3636

A.7.6.5.1.2	Test Requirements	3639

A.7.6.6	L1-SINR measurement for beam reporting	3639

A.7.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	3641

A.7.6.6.2.1	Test Purpose and Environment	3641

A.7.6.6.2.2	Test parameters	3642

A.7.6.6.2.3	Test Requirements	3644

A.7.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	3644

A.7.6.6.3.1	Test Purpose and Environment	3644

A.7.6.6.3.2	Test parameters	3645

A.7.6.6.3.3	Test Requirements	3647

A.7.6.7	CSI-RS based intra-frequency Measurements	3647

A.7.6.7.1	SA event triggered reporting test without gap under DRX for CSI-RS based intra-frequency measurement	3647

A.7.6.7.1.1	Test purpose and Environment	3647

A.7.6.7.1.2	Test Requirements	3650

A.7.6.8	CSI-RS based inter-frequency Measurements	3650

A.7.6.8.1	SA event triggered reporting tests for FR2 CSI-RS based measurement when non-DRX is used (PCell in FR2)	3650

A.7.6.8.1.1	Test Purpose and Environment	3650

A.7.6.8.1.2	Test Requirements	3653

A.7.6.9	RSTD measurements	3653

A.7.6.9.1	 NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA	3653

A.7.6.9.1.1	Test Purpose and Environment	3653

A.7.6.9.1.2	Test Requirements	3661

A.7.6.9.2	 NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR2 SA	3661

A.7.6.9.2.1	Test Purpose and Environment	3661

A.7.6.9.2.2	Test Requirements	3669

A.7.6.9.3	NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA	3669

A.7.6.9.3.1	Test Purpose and Environment	3669

A.7.6.9.3.2	Test Requirements	3675

A.7.6.9.4	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA without measurement gap	3676

A.7.6.9.4.1	Test Purpose and Environment	3676

A.7.6.9.4.2	Test Requirements	3679

A.7.6.9.5	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_CONNECTED state with Rx TEG	3680

A.7.6.9.5.1	Test Purpose and Environment	3680

A.7.6.9.5.2	Test Requirements	3683

A.7.6.10 PRS-RSRP measurements	3684

A.7.6.10.1 PRS-RSRP reporting delay test case for single positioning frequency layer	3684

A.7.6.10.1.1	Test Purpose and Environment	3684

A.7.6.10.1.2	Test Requirements	3688

A.7.6.10.2	PRS-RSRP reporting delay test case for dual positioning frequency layer	3688

A.7.6.10.2.1	Test Purpose and Environment	3688

A.7.6.10.2.2	Test Requirements	3692

A.7.6.10.3	PRS-RSRP reporting delay test case for reduced number of samples	3692

A.7.6.10.3.1	Test Purpose and Environment	3692

A.7.6.10.3.2	Test Requirements	3696

A.7.6.10.4	PRS-RSRP reporting delay test case for single positioning frequency layer outside MG	3696

A.7.6.10.4.1	Test Purpose and Environment	3696

A.7.6.10.4.2	Test Requirements	3700

A.7.6.11	UE Rx-Tx time difference measurements	3700

A.7.6.11.1	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA	3700

A.7.6.11.1.1	Test purpose and environment	3700

A.7.6.11.1.2	Test requirements	3704

A.7.6.11.2	UE Rx-Tx time difference measurement period for dual positioning frequency layers in FR2 SA	3704

A.7.6.11.2.1	Test purpose and environment	3704

A.7.6.11.2.2	Test requirements	3708

A.7.6.11.3	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA with reduced sample number	3708

A.7.6.11.3.1	Test purpose and environment	3708

A.7.6.11.3.2	Test requirements	3710

A.7.6.11.4	UE Rx-Tx time difference measurements without gaps in FR2 SA	3711

A.7.6.11.4.1	Test purpose and environment	3711

A.7.6.11.4.2	Test requirements	3713

A.7.6.11.5	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA with RxTx TEG	3714

A.7.6.11.5.1	Test purpose and environment	3714

A.7.6.11.5.2	Test requirements	3716

A.7.6.12	PRS-RSRPP measurements	3717

A.7.6.12.1 PRS-RSRPP reporting delay test case for single positioning frequency layer in FR2 in RRC_CONNECTED state	3717

A.7.6.12.1.1	Test Purpose and Environment	3717

A.7.6.12.1.2	Test Requirements	3720

A.7.6.12.2	PRS-RSRPP reporting delay test case for reduced number of samples for single positioning frequency layer in FR2 in RRC_CONNECTED state	3720

A.7.6.12.2.1	Test Purpose and Environment	3720

A.7.6.12.2.2	Test Requirements	3723

A.7.6.12.3	PRS-RSRPP reporting delay test case for gapless measurement in FR2	3723

A.7.6.12.3.1	Test Purpose and Environment	3723

A.7.6.12.3.2	Test Requirements	3726

A.7.6.13	UE Rx-Tx time difference measurements for PDC	3727

A.7.6.13.1	UE Rx-Tx time difference measurement for propagation delay compensation using PRS in FR2	3727

A.7.6.13.1.1	Test purpose and environment	3727

A.7.6.13.1.2	Test requirements	3728

A.7.6.13.2	UE Rx-Tx time difference measurement for propagation delay compensation using TRS in FR2	3729

A.7.6.13.2.1	Test purpose and environment	3729

A.7.6.13.2.2	Test requirements	3730

A.7.6.14	SA event triggered reporting tests with Pre-MG	3731

A.7.6.14.1	Intra-frequency measurement test with SA event triggered reporting tests: with autonomous activation/deactivation of Pre-MG in FR2	3731

A.7.6.14.1.1	Test purpose and Environment	3731

A.7.6.14.1.2	Test parameters	3731

A.7.6.14.1.3	Test Requirements	3733

A.7.6.14.2	Intra-frequency measurement test with SA event triggered reporting tests: with network-controlled activation/deactivation of Pre-MG in FR2	3734

A.7.6.14.2.1	Test purpose and Environment	3734

A.7.6.14.2.2	Test parameters	3734

A.7.6.14.2.3	Test Requirements	3736

A.7.6.15	SA event triggered reporting tests with concurrent gaps	3737

A.7.6.15.1	SA event triggered reporting tests For FR2 with fully non-overlapping concurrent MGs for SSB-based inter-frequency measurements	3737

A.7.6.15.1.1	Test Purpose and Environment	3737

A.7.6.15.1.2	Test Requirements	3740

A.7.6.15.2	SA event triggered reporting tests For FR2 with concurrent measurement gaps without SSB time index detection when DRX is not used (PCell in FR2)	3740

A.7.6.15.2.1	Test Purpose and Environment	3740

A.7.6.15.2.2	Test Requirements	3743

A.7.6.15.3	SA event triggered reporting tests for FR2 concurrent gap with partially partial overlapping scenario for SSB-based measurements and PRS-based measurement	3743

A.7.6.15.3.1	Test Purpose and Environment	3743

A.7.6.15.3.2	Test Requirements	3747

A.7.6.16	SA event triggered reporting tests with NCSG	3747

A.7.6.16.1	SA event triggered reporting test with per-UE NCSG under non-DRX	3747

A.7.6.16.1.1	Test purpose and Environment	3747

A.7.6.16.1.2	Test Requirements	3751

A.7.6.16.2	SA event triggered reporting tests on inter-frequency measurement with NCSG for FR2 when DRX is not used (PCell in FR2)	3751

A.7.6.16.2.1	Test Purpose and Environment	3751

A.7.6.16.2.2	Test Requirements	3754

A.7.6.16.3	Event triggered reporting test on deactivated Scell measurement via NCSG in FR2 in non-DRX	3754

A.7.6.16.3.1	Test Purpose and Environment	3754

A.7.6.16.3.2	Test Requirements	3757

A.7.7	Measurement Performance requirements	3757

A.7.7.1	SS-RSRP	3758

A.7.7.1.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3758

A.7.7.1.1.1	Test Purpose and Environment	3758

A.7.7.1.1.2	Test parameters	3758

A.7.7.1.1.3	Test Requirements	3762

A.7.7.1.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3763

A.7.7.1.2.1	Test Purpose and Environment	3763

A.7.7.1.2.2	Test parameters	3763

A.7.7.1.2.3	Test Requirements	3767

A.7.7.1.3	SA inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	3768

A.7.7.1.3.1	Test Purpose and Environment	3768

A.7.7.1.3.2	Test parameters	3768

A.7.7.1.3.3	Test Requirements	3772

A.7.7.2	SS-RSRQ	3772

A.7.7.2.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	3772

A.7.7.2.1.1	Test Purpose and Environment	3772

A.7.7.2.1.2	Test Parameters	3772

A.7.7.2.1.3	Test Requirements	3774

A.7.7.2.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3774

A.7.7.2.2.1	Test Purpose and Environment	3774

A.7.7.2.2.2	Test Parameters	3774

A.7.7.2.2.3	Test Requirements	3776

A.7.7.3	SS-SINR	3776

A.7.7.3.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3776

A.7.7.3.1.1	Test Purpose and Environment	3776

A.7.7.3.1.2	Test Parameters	3776

A.7.7.3.1.3	Test Requirements	3778

A.7.7.3.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3778

A.7.7.3.2.1	Test Purpose and Environment	3778

A.7.7.3.2.2	Test Parameters	3778

A.7.7.3.2.3	Test Requirements	3780

A.7.7.4	L1-RSRP measurement for beam reporting	3780

A.7.7.4.1	SSB based L1-RSRP measurement	3780

A.7.7.4.1.1	Test Purpose and Environment	3780

A.7.7.4.1.2	Test parameters	3781

A.7.7.4.1.3	Test Requirements	3783

A.7.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	3784

A.7.7.4.2.1	Test Purpose and Environment	3784

A.7.7.4.2.2	Test parameters	3784

A.7.7.4.2.3	Test Requirements	3786

A.7.7.5	CLI measurements	3787

A.7.7.5.1	SA SRS-RSRP measurement accuracy with FR2 serving cell	3787

A.7.7.5.1.1	Test Purpose and Environment	3787

A.7.7.5.1.2	Test parameters	3787

A.7.7.5.1.3	Test Requirements	3790

A.7.7.5.2	SA CLI-RSSI measurement accuracy with FR2 serving cell	3791

A.7.7.5.2.1	Test Purpose and Environment	3791

A.7.7.5.2.2	Test parameters	3791

A.7.7.5.2.3	Test Requirements	3793

A.7.7.6	L1-SINR measurement for beam reporting	3794

A.7.7.6.1.1	Test Purpose and Environment	3794

A.7.7.6.1.2	Test parameters	3794

A.7.7.6.1.3	Test Requirements	3796

A.7.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	3797

A.7.7.6.2.1	Test Purpose and Environment	3797

A.7.7.6.2.2	Test parameters	3797

A.7.7.6.2.3	Test Requirements	3799

A.7.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	3800

A.7.7.6.3.1	Test Purpose and Environment	3800

A.7.7.6.3.2	Test parameters	3800

A.7.7.6.3.3	Test Requirements	3802

A.7.7.7	CSI-RSRP	3803

A.7.7.7.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3803

A.7.7.7.1.1	Test Purpose and Environment	3803

A.7.7.7.1.2	Test parameters	3803

A.7.7.7.1.3	Test Requirements	3806

A.7.7.7.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3807

A.7.7.7.2.1	Test Purpose and Environment	3807

A.7.7.7.2.2	Test parameters	3807

A.7.7.7.2.3	Test Requirements	3810

A.7.7.8	CSI-RSRQ	3811

A.7.7.8.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	3811

A.7.7.8.1.1	Test Purpose and Environment	3811

A.7.7.8.1.2	Test Parameters	3811

A.7.7.8.1.3	Test Requirements	3813

A.7.7.8.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3813

A.7.7.8.2.1	Test Purpose and Environment	3813

A.7.7.8.2.2	Test Parameters	3813

A.7.7.8.2.3	Test Requirements	3815

A.7.7.9	CSI-SINR	3815

A.7.7.9.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3815

A.7.7.9.1.1	Test Purpose and Environment	3815

A.7.7.9.1.2	Test Parameters	3815

A.7.7.9.1.3	Test Requirements	3817

A.7.7.9.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3817

A.7.7.9.2.1	Test Purpose and Environment	3817

A.7.7.9.2.2	Test Parameters	3818

A.7.7.9.2.3	Test Requirements	3820

A.7.7.10	RSTD measurements	3820

A.7.7.10.1	RSTD measurement accuracy test case for single positioning frequency layer	3820

A.7.7.10.1.1	Test purpose and Environment	3820

A.7.7.10.1.2	Test Requirements	3823

A.7.7.10.2	RSTD measurement accuracy test case for dual positioning frequency layer	3823

A.7.7.10.2.1	Test purpose and Environment	3823

A.7.7.10.2.2	Test Requirements	3827

A.7.7.10.3	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR2 in RRC_CONNECTED state	3828

A.7.7.10.3.1	Test purpose and Environment	3828

A.7.7.10.3.2	Test Requirements	3830

A.7.7.10.4	RSTD measurement accuracy test case with Rx TEG	3830

A.7.7.10.4.1	Test purpose and Environment	3830

A.7.7.10.4.2	Test Requirements	3832

A.7.7.11	PRS-RSRP measurements	3832

A.7.7.11.1	SA measurement accuracy with PRS in FR2	3832

A.7.7.11.1.1	Test Purpose and Environment	3832

A.7.7.11.1.2	Test parameters	3832

A.7.7.11.1.3	Test Requirements	3835

A.7.7.11.2	SA measurement accuracy with PRS in FR2 with reduced sample number	3836

A.7.7.11.2.1	Test Purpose and Environment	3836

A.7.7.11.2.2	Test parameters	3836

A.7.7.11.2.3	Test Requirements	3838

A.7.7.12	UE Rx-Tx time difference measurements	3839

A.7.7.12.1	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR2 SA	3839

A.7.7.12.1.1	Test purpose and environment	3839

A.7.7.12.1.2	Test parameters	3839

A.7.7.12.1.3	Test requirements	3842

A.7.7.12.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR2 SA	3843

A.7.7.12.2.1	Test purpose and environment	3843

A.7.7.12.2.2	Test parameters	3843

A.7.7.12.2.3	Test requirements	3846

A.7.7.12.3	UE Rx-Tx time difference measurement accuracy with RxTx TEG	3846

A.7.7.12.3.1	Test purpose and environment	3846

A.7.7.12.3.2	Test parameters	3846

A.7.7.12.3.3	Test requirements	3849

A.7.7.13	PRS-RSRPP measurements	3849

A.7.7.13.1	SA measurement accuracy with PRS in FR2	3849

A.7.7.13.1.1	Test Purpose and Environment	3849

A.7.7.13.1.2	Test parameters	3850

A.7.7.13.1.3	Test Requirements	3852

A.7.7.13.2	SA measurement accuracy with reduced PRS samples in FR2	3853

A.7.7.13.2.1	Test Purpose and Environment	3853

A.7.7.13.2.2	Test parameters	3853

A.7.7.13.2.3	Test Requirements	3855

A.7.8	Measurement procedure in RRC_INACTIVE	3856

A.7.8.1	RSTD measurements	3856

A.7.8.1.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_INACTIVE state	3856

A.7.8.1.1.1	Test Purpose and Environment	3856

A.7.8.1.1.2	Test Requirements	3859

A.7.8.1.2	NR RSTD measurement reporting delay test case with reduced number of samples in RRC_INACTIVE, FR1 SA	3859

A.7.8.1.2.1	Test Purpose and Environment	3859

A.7.8.1.2.2	Test Requirements	3865

A.7.8.2	PRS-RSRP measurements	3866

A.7.8.2.1	PRS-RSRP reporting delay test case for single positioning frequency layer in RRC_INACTIVE	3866

A.7.8.2.1.1	Test Purpose and Environment	3866

A.7.8.2.1.2	Test Requirements	3870

A.7.8.2.2	PRS-RSRP reporting delay test case with reduced number of samples in RRC_INACTIVE	3870

A.7.8.2.2.1	Test purpose and Environment	3870

A.7.8.2.2.2	Test Requirements	3874

A.7.8.3	UE Rx-Tx time difference measurements	3874

A.7.8.3.1	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA	3874

A.7.8.3.1.1	Test purpose and environment	3874

A.7.8.3.1.2	Test requirements	3878

A.7.8.3.2	UE Rx-Tx time difference measurement with reduced number of samples in RRC_INACTIVE, FR2 SA	3878

A.7.8.3.2.1	Test purpose and environment	3878

A.7.8.3.2.2	Test requirements	3880

A.7.8.4	PRS-RSRPP measurements	3881

A.7.8.4.1	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR2 in RRC_INACTIVE state	3881

A.7.8.4.1.1	Test Purpose and Environment	3881

A.7.8.4.1.2	Test Requirements	3883

A.7.8.4.2	PRS-RSRPP reporting delay test with reduced number of samples for single positioning frequency layer in FR2 in RRC_INACTIVE state	3884

A.7.8.4.2.1	Test Purpose and Environment	3884

A.7.8.4.2.2	Test Requirements	3886

A.7.9	Measurement performance requirements in RRC_INACTIVE	3887

A.7.9.1	RSTD measurements	3887

A.7.9.1.1	RSTD measurement accuracy test case for single positioning frequency layer in FR2 in RRC_INACTIVE state	3887

A.7.9.1.1.1	Test purpose and Environment	3887

A.7.9.1.1.2	Test Requirements	3889

A.7.9.1.2	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR2 in RRC_INACTIVE state	3889

A.7.9.1.2.1	Test purpose and Environment	3889

A.7.9.1.2.2	Test Requirements	3891

A.7.9.2	PRS-RSRP measurements	3891

A.7.9.2.1	SA measurement accuracy with PRS in FR2 in RRC_INACTIVE	3891

A.7.9.2.1.1	Test Purpose and Environment	3891

A.7.9.2.1.2	Test parameters	3891

A.7.9.2.1.3	Test Requirements	3893

A.7.9.2.2	PRS-RSRP measurements with reduced number of sample in RRC_INACTIVE	3894

A.7.9.2.2.1	Test Purpose and Environment	3894

A.7.9.2.2.2	Test parameters	3894

A.7.9.2.2.3	Test Requirements	3896

A.7.9.3	UE Rx-Tx time difference measurements	3897

A.7.9.3.1	UE Rx-Tx time difference measurements in RRC_INACTIVE	3897

A.7.9.3.1.1	Test purpose and environment	3897

A.7.9.3.1.2	Test parameters	3897

A.7.9.3.1.3	Test requirements	3900

A.7.9.3.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR2 SA	3900

A.7.9.3.2.1	Test purpose and environment	3900

A.7.9.3.2.2	Test parameters	3900

A.7.9.3.2.3	Test requirements	3903

A.7.9.4	PRS-RSRPP measurements	3903

A.7.9.4.1	SA measurement accuracy in FR2 in RRC INACTIVE	3903

A.7.9.4.1.1	Test Purpose and Environment	3903

A.7.9.4.1.2	Test parameters	3903

A.7.9.4.1.3	Test Requirements	3905

A.7.9.4.2	SA measurement accuracy with reduced PRS samples in FR2 in RRC INACTIVE	3906

A.7.9.4.2.1	Test Purpose and Environment	3906

A.7.9.4.2.2	Test parameters	3906

A.7.9.4.2.3	Test Requirements	3908

A.8	E-UTRA standalone tests for NR RRM	3909

A.8.1	Void	3909

A.8.2	RRC_IDLE state mobility	3909

A.8.2.1	Inter-RAT NR Cell re-selection	3909

A.8.2.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1	3909

A.8.2.1.1.1	Test Purpose and Environment	3909

A.8.2.1.1.2	Test Requirements	3914

A.8.2.1.2	E-UTRA Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16	3915

A.8.2.1.2.1	Test Purpose and Environment	3915

A.8.2.1.2.2	Test Requirements	3919

A.8.2.2	E-UTRA – NR Inter-RAT Early Measruement Reporting	3920

A.8.2.2.1	E-UTRA – NR Early Measurement Reporting for NR in FR1	3920

A.8.2.2.1.1	Test Purpose and Environment	3920

A.8.2.2.1.2	Test Requirements	3924

A.8.2.2.2	E-UTRA – NR Early Measurement Reporting for NR in FR2	3924

A.8.2.2.2.1	Test Purpose and Environment	3924

A.8.2.2.2.2	Test Requirements	3928

A.8.3	RRC_CONNECTED state mobility	3928

A.8.3.1	Handover	3928

A.8.3.1.1	E-UTRAN - NR handover in FR1	3928

A.8.3.1.1.1	Test Purpose and Environment	3928

A.8.3.1.1.2	Test Requirements	3933

A.8.4	Measurement procedure	3933

A.8.4.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay	3933

A.8.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay in non-DRX	3933

A.8.4.1.1.1	Test Purpose and Environment	3933

A.8.4.1.1.2	Test Requirements	3935

A.8.4.1.2	E-UTRA – NR Inter-RAT SFTD Measurement Delay in DRX	3936

A.8.4.1.2.1	Test Purpose and Environment	3936

A.8.4.1.2.2	Test Requirements	3937

A.8.4.2	E-UTRA – NR Inter-RAT Measurements	3937

A.8.4.2.1	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	3937

A.8.4.2.1.1	Test Purpose and Environment	3937

A.8.4.2.1.2	Test Requirements	3942

A.8.4.2.2	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	3942

A.8.4.2.2.1	Test Purpose and Environment	3942

A.8.4.2.2.2	Test Requirements	3946

A.8.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	3946

A.8.4.2.3.1	Test Purpose and Environment	3946

A.8.4.2.3.2	Test Requirements	3951

A.8.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	3951

A.8.4.2.4.1	Test Purpose and Environment	3951

A.8.4.2.4.2	Test Requirements	3955

A.8.4.2.5	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is not used	3955

A.8.4.2.5.1	Test Purpose and Environment	3955

A.8.4.2.5.2	Test Requirements	3957

A.8.4.2.6	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is used	3958

A.8.4.2.6.1	Test Purpose and Environment	3958

A.8.4.2.6.2	Test Requirements	3960

A.8.4.2.7	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is not used	3961

A.8.4.2.7.1	Test Purpose and Environment	3961

A.8.4.2.7.2	Test Requirements	3963

A.8.4.2.8	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is used	3964

A.8.4.2.8.1	Test Purpose and Environment	3964

A.8.4.2.8.2	Test Requirements	3966

A.8.4.2.9	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16	3967

A.8.4.2.9.1	Test Purpose and Environment	3967

A.8.4.2.9.2	Test Requirements	3972

A.8.5	Measurement performance	3972

A.8.5.1	SFTD accuracy	3972

A.8.5.1.1	SFTD accuracy	3972

A.8.5.1.1.1	Test Purpose	3972

A.8.5.1.1.2	Test Environment	3972

A.8.5.1.1.3	Test Requirements	3978

A.8.5.2	E-UTRA – NR Inter-RAT Measurement Performance requirements	3978

A.8.5.2.1.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	3978

A.8.5.2.1.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	3983

A.8.5.2.1.2.1	Test Purpose and Environment	3983

A.8.5.2.1.2.2	Test Parameters	3983

A.8.5.2.1.2.3	Test Requirements	3985

A.8.5.2.2	SS-RSRQ	3985

A.8.5.2.2.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	3985

A.8.5.2.2.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	3990

A.8.5.2.2.2.1	Test Purpose and Environment	3990

A.8.5.2.2.2.2	Test Parameters	3990

A.8.5.2.2.2.3	Test Requirements	3992

A.8.5.2.3	SS-SINR	3992

A.8.5.2.3.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	3992

A.8.5.2.3.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	3996

A.8.5.2.3.2.1	Test Purpose and Environment	3996

A.8.5.2.3.2.2	Test Parameters	3996

A.8.5.2.3.2.3	Test Requirements	3998

A.9	V2X Tests	3999

A.9.1	V2X Tests in FR1	3999

A.9.1.1	Test for V2X UE Transmit Timing	3999

A.9.1.1.1 Test for GNSS as Synchronization Reference Source	3999

A.9.1.1.1.1	Test Purpose and Environment	3999

A.9.1.1.1.2	Test requirements	3999

A.9.1.1.2	Test for SyncRef UE as Synchronization Reference Source	3999

A.9.1.1.2.1	Test Purpose and Environment	3999

A.9.1.1.2.2	Test requirements	4000

A.9.1.1.3	Test for FR1 NR Cell as Synchronization Reference Source	4000

A.9.1.1.3.1	Test Purpose and Environment	4000

A.9.1.1.3.2	Test requirements	4003

A.9.1.2	Test for Initiation/Cease of S-SSB Transmission with V2X Sidelink Communication	4003

A.9.1.2.1	Test for FR1 NR Cell as synchronization reference source without gap under non-DRX	4003

A.9.1.2.1.1	Test Purpose and Environment	4003

A.9.1.2.1.2	Test Requirements	4006

A.9.1.2.2	Test for SyncRef UE as synchronization reference source	4006

A.9.1.2.2.1	Test Purpose and Environment	4006

A.9.1.2.2.2	Test Requirements	4007

A.9.1.2.3	Test for SyncRef UE as synchronization reference source when SL-DRX is used	4008

A.9.1.2.3.1	Test Purpose and Environment	4008

A.9.1.2.3.2	Test Requirements	4010

A.9.1.3	 Test for V2X Synchronization Reference Selection/Reselection	4010

A.9.1.3.1	 Test for GNSS configured as the highest priority	4010

A.9.1.3.1.1	Test Purpose and Environment	4010

A.9.1.3.1.2	Test Requirements	4012

A.9.1.3.2	 Test for FR1 NR Cell configured as the highest priority	4013

A.9.1.3.2.1	Test Purpose and Environment	4013

A.9.1.3.2.2	Test Requirements	4015

A.9.1.3.3	Test for GNSS configured as the highest priority under SL-DRX	4016

A.9.1.3.3.1	Test Purpose and Environment	4016

A.9.1.3.3.2	Test Requirements	4018

A.9.1.3.4	Test for FR1 NR Cell configured as the highest priority under SL-DRX	4019

A.9.1.3.4.1	Test Purpose and Environment	4019

A.9.1.3.4.2	Test Requirements	4021

A.9.1.4	Test for L1 SL-RSRP Measurement	4022

A.9.1.4.1	Test for V2X UE Autonomous Resource Selection/Reselection	4022

A.9.1.4.1.1	Test Purpose and Environment	4022

A.9.1.4.1.2	Test Requirements	4025

A.9.1.4.2	Test for V2X UE Resource Pre-emption	4025

A.9.1.4.2.1	Test Purpose and Environment	4025

A.9.1.4.2.2	Test Requirements	4029

A.9.1.4.3	 Test for V2X UE Resource Re-evaluation	4029

A.9.1.4.3.1	Test Purpose and Environment	4029

A.9.1.4.3.2	Test Requirements	4036

A.9.1.4.4	Test for V2X UE Autonomous Resource Selection/Reselection with Periodic Sensing	4036

A.9.1.4.4.1	Test Purpose and Environment	4036

A.9.1.4.4.2	Test Requirements	4040

A.9.1.4.5	Test for V2X UE Autonomous Resource Selection/Reselection with Contiguous Sensing	4040

A.9.1.4.5.1	Test Purpose and Environment	4040

A.9.1.4.5.2	Test Requirements	4043

A.9.1.4.6	Test for V2X UE Autonomous Resource Selection/Reselection in SL-DRX	4043

A.9.1.4.6.1	Test Purpose and Environment	4043

A.9.1.4.6.2	Test Requirements	4047

A.9.1.5	Test for Congestion Control Measurement	4047

A.9.1.5.1	Test Purpose and Environment	4047

A.9.1.5.2	Test Requirements	4053

A.9.1.6	Test for Interruption	4053

A.9.1.6.1	Test for Interruption to WAN due to V2X Sidelink Communication	4053

A.9.1.6.1.1	Test Purpose and Environment	4053

A.9.1.6.1.2	Test Requirements	4057

A.9.1.6.2	Test for interruption to WAN at transitions between active and non-active during SL-DRX in asynchronous case	4057

A.9.1.6.2.1	Test Purpose and Environment	4057

A.9.1.6.2.2	Test Requirements	4061

A.9.1.6.3	Test for Interruption at NR Sidelink Diccovery Configuration	4061

A.9.1.6.3.1	Test Purpose and Environment	4061

A.9.1.6.3.2	Test Requirements	4064

A.9.1.7	Selection / Reselection of relay UE	4064

A.9.1.7.1	Test Purpose and Environment	4064

A.9.1.7.2	Test Requirements	4070

A.10	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	4071

A.10.1	RRC_CONNECTED state mobility	4071

A.10.1.1	RRC connection mobility control	4071

A.10.1.1.1	Random Access	4071

A.10.1.1.1.1	4-step RA type contention-based random access for NR PSCell with CCA	4071

A.10.1.1.1.1.1	Test Purpose and Environment	4071

A.10.1.1.1.1.2	Test Requirements	4074

A.10.1.1.1.1.2.1	Random Access Preamble Transmission	4074

A.10.1.1.1.1.2.2	Random Access Response Reception	4074

A.10.1.1.1.1.2.3	No Random Access Response Reception	4075

A.10.1.1.1.1.2.4	Receiving an UL grant for msg3 retransmission	4075

A.10.1.1.1.1.2.5	 Contention Resolution Timer expiry	4075

A.10.1.1.1.2	4-step RA type non-contention based random access for NR PSCell with CCA	4076

A.10.1.1.1.2.1	Test Purpose and Environment	4076

A.10.1.1.1.2.2	Test Requirements	4079

A.10.1.1.1.2.2.1	SSB-based Random Access Preamble Transmission	4079

A.10.1.1.1.2.2.2	Random Access Response Reception	4080

A.10.1.1.1.2.2.3	No Random Access Response Reception	4080

A.10.1.1.1.3	2-step RA type contention-based random access for NR PSCell with CCA	4080

A.10.1.1.1.3.1	Test Purpose and Environment	4080

A.10.1.1.1.3.2	Test Requirements	4084

A.10.1.1.1.3.2.1	MsgA Transmission	4084

A.10.1.1.1.3.2.2	MsgB Reception	4084

A.10.1.1.1.3.2.3	No MsgB Reception	4085

A.10.1.1.1.4	2-step RA type non-contention based random access for NR PSCell with CCA	4085

A.10.1.1.1.4.1	Test Purpose and Environment	4085

A.10.1.1.1.4.2	Test Requirements	4089

A.10.1.1.1.4.2.1	MsgA Transmission	4089

A.10.1.1.1.4.2.2	MsgB Reception	4090

A.10.1.1.1.4.2.3	No MsgB Reception	4090

A.10.1.2	Handover with PSCell from EN-DC to EN-DC with known target PSCell using CCA	4090

A.10.1.2.1	Test Purpose and Environment	4090

A.10.1.2.2	Test Requirements	4096

A.10.2	Timing	4097

A.10.2.1	UE transmit timing	4097

A.10.2.1.1	UE Transmit Timing Test with PSCell under DL CCA	4097

A.10.2.1.1.1	Test Purpose and environment	4097

A.10.2.1.1.2	Test requirements	4100

A.10.2.2	UE timing advance	4101

A.10.2.2.1	UE Timing Advance Adjustment Accuracy with PSCell under DL CCA	4101

A.10.2.2.1.1	Test Purpose and Environment	4101

A.10.2.2.1.2	Test Parameters	4101

A.10.2.2.1.3	Test Requirements	4105

A.10.3	Signalling characteristics	4105

A.10.3.1	Radio link monitoring	4105

A.10.3.1.1	Introduction	4105

A.10.3.1.2	Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	4106

A.10.3.1.2.1	Test purpose and environment	4106

A.10.3.1.2.2	Test requirements	4110

A.10.3.1.3	Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	4111

A.10.3.1.3.1	Test purpose and environment	4111

A.10.3.1.3.2	Test requirements	4116

A.10.3.1.4	Void	4116

A.10.3.1.4.1	Void	4116

A.10.3.1.4.2	Void	4116

A.10.3.1.5	Void	4116

A.10.3.1.5.1	Void	4116

A.10.3.1.5.2	Void	4116

A.10.3.2	Interruption	4116

A.10.3.2.1	E-UTRAN – NR interruptions during SCell operations with CCA	4116

A.10.3.2.1.1	Test Purpose and Environment	4116

A.10.3.2.1.2	Test Requirements	4121

A.10.3.3	SCell activation and deactivation delay	4121

A.10.3.3.1	SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 160 ms SCell measurement cycle	4121

A.10.3.3.1.1	Test Purpose and Environment	4121

A.10.3.3.1.2	Test Requirements	4126

A.10.3.3.2 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 640 ms SCell measurement cycle	4126

A.10.3.3.2.1	Test Purpose and Environment	4126

A.10.3.3.2.2	Test Requirements	4127

A.10.3.3.3	SCell Activation and Deactivation of unknown NR SCell with NR PSCell and NR SCell under CCA	4127

A.10.3.3.3.1	Test Purpose and Environment	4127

A.10.3.3.3.2	Test Requirements	4128

A.10.3.4	Beam failure detection and link recovery procedures	4128

A.10.3.4.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	4128

A.10.3.4.1.1	Test Purpose and Environment	4128

A.10.3.4.1.2	Test Requirements	4133

A.10.3.4.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	4133

A.10.3.4.2.1	Test Purpose and Environment	4133

A.10.3.4.2.2	Test Requirements	4139

A.10.3.5	Active BWP switching	4139

A.10.3.5.1	UL active BWP switch delay with consistent UL LBT failure on PSCell subject to UL CCA in EN-DC	4139

A.10.3.5.1.2	Test Requirements	4144

A.10.3.5.2	DCI-based and Timer-based Active BWP Switch	4145

A.10.3.5.2.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	4145

A.10.3.5.2.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	4148

A.10.3.5.3	RRC-based Active BWP Switch	4152

A.10.3.5.3.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	4152

A.10.3.6	PSCell addition and release delay	4156

A.10.3.6.1	Addition and Release Delay of known NR PSCell on the carrier under CCA	4156

A.10.3.6.1.1	Test purpose and environment	4156

A.10.3.6.1.2	Test Requirements	4161

A.10.3.7	Void	4162

A.10.4	Measurement procedure	4162

A.10.4.1	Intra-frequency measurements	4162

A.10.4.1.1	Event-triggered reporting tests on PSCC without gaps under non-DRX	4162

A.10.4.1.1.1	Test purpose and environment	4162

A.10.4.1.1.2	Test parameters	4162

A.10.4.1.1.3	Test Requirements	4165

A.10.4.1.2	Void A.10.4.1.3 Void A.10.4.1.4 Event-triggered reporting tests on PSCC with per-UE gaps under DRX	4165

A.10.4.1.4.1	Test purpose and environment	4165

A.10.4.1.4.2	Test parameters	4165

A.10.4.1.4.3	Test Requirements	4169

A.10.4.1.7	Void A.10.4.1.8 Void A.10.4.1.9 RSSI measurement reporting on PSCC	4169

A.10.4.1.9.1	Test purpose and environment	4169

A.10.4.1.9.2	Test parameters	4170

A.10.4.1.10	Channel occupancy measurement reporting on PSCC	4170

A.10.4.1.10.1	Test purpose and environment	4170

A.10.4.1.10.2	Test parameters	4170

A.10.4.1.11	RSSI measurement reporting on SCC	4170

A.10.4.1.11.1	Test purpose and environment	4170

A.10.4.1.11.2	Test parameters	4171

A.10.4.1.12	Channel occupancy measurement reporting on SCC	4171

A.10.4.1.12.1	Test purpose and environment	4171

A.10.4.1.12.2	Test parameters	4171

A.10.4.2	Inter-frequency measurements	4172

A.10.4.2.1	RSSI measurement reporting	4172

A.10.4.2.1.1	Test purpose and environment	4172

A.10.4.2.1.2	Test parameters	4172

A.10.4.2.2	Channel occupancy measurement reporting	4172

A.10.4.2.2.1	Test purpose and environment	4172

A.10.4.2.2.2	Test parameters	4172

A.10.4.2.3	EN-DC event triggered reporting tests for FR1 with CCA cell without SSB time index detection when DRX is not used	4173

A.10.4.2.3.1	Test Purpose and Environment	4173

A.10.4.2.3.2	Test Requirements	4177

A.10.4.2.4	EN-DC event triggered reporting tests for FR1 cell with CCA without SSB time index detection when DRX is used	4178

A.10.4.2.4.1	Test Purpose and Environment	4178

A.10.4.2.4.2	Test Requirements	4182

A.10.4.2.5	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is not used	4183

A.10.4.2.5.1	Test Purpose and Environment	4183

A.10.4.2.5.2	Test Requirements	4187

A.10.4.2.6	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is used	4188

A.10.4.2.6.1	Test Purpose and Environment	4188

A.10.4.2.6.2	Test Requirements	4192

A.10.4.2.7	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	4193

A.10.4.2.7.1	Test Purpose and Environment	4193

A.10.4.2.7.2	Test Requirements	4199

A.10.4.2.8	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	4199

A.10.4.2.8.1	Test Purpose and Environment	4199

A.10.4.2.8.2	Test Requirements	4205

A.10.4.2.9	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	4206

A.10.4.2.9.1	Test Purpose and Environment	4206

A.10.4.2.9.2	Test Requirements	4211

A.10.4.2.10	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	4211

A.10.4.2.10.1	Test Purpose and Environment	4211

A.10.4.2.10.2	Test Requirements	4217

A.10.4.3	L1-RSRP measurements for beam reporting	4218

A.10.4.3.1	SSB based L1-RSRP measurement on PSCC when DRX is not used	4218

A.10.4.3.1.1	Test Purpose and Environment	4218

A.10.4.3.1.2	Test parameters	4218

A.10.4.3.1.3	Test Requirements	4220

A.10.4.3.2	SSB based L1-RSRP measurement on PSCC when DRX is used	4221

A.10.4.3.2.1	Test Purpose and Environment	4221

A.10.4.3.2.2	Test parameters	4221

A.10.4.3.2.3	Test Requirements	4223

A.10.4.3.3	SSB based L1-RSRP measurement on SCC when DRX is not used	4224

A.10.4.3.3.1	Test Purpose and Environment	4224

A.10.4.3.3.2	Test parameters	4224

A.10.4.3.3.3	Test Requirements	4227

A.10.4.3.4	SSB based L1-RSRP measurement on SCC when DRX is used	4228

A.10.4.3.4.1	Test Purpose and Environment	4228

A.10.4.3.4.2	Test parameters	4228

A.10.4.3.4.3	Test Requirements	4231

A.10.4.4	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	4232

A.10.4.4.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4232

A.10.4.4.1.1	Test Purpose and Environment	4232

A.10.4.4.1.2	Test Requirements	4236

A.10.4.4.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4237

A.10.4.4.2.1	Test Purpose and Environment	4237

A.10.4.4.2.2	Test Requirements	4242

A.10.4.4.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4243

A.10.4.4.3.1	Test Purpose and Environment	4243

A.10.4.4.3.2	Test Requirements	4247

A.10.4.4.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4248

A.10.4.4.4.1	Test Purpose and Environment	4248

A.10.4.4.4.2	Test Requirements	4253

A.10.5	Measurement performance	4254

A.10.5.1	SS-RSRP	4254

A.10.5.1.1	Intra-frequency measurement accuracy on a CCA serving cell	4254

A.10.5.1.1.1	Test Purpose and Environment	4254

A.10.5.1.1.2	Test parameters	4254

A.10.5.1.1.3	Test Requirements	4257

A.10.5.1.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	4258

A.10.5.1.2.1	Test Purpose and Environment	4258

A.10.5.1.2.2	Test parameters	4258

A.10.5.1.2.3	Test Requirements	4261

A.10.5.2	SS-RSRQ	4261

A.10.5.2.1	Intra-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	4261

A.10.5.2.1.1	Test Purpose and Environment	4261

A.10.5.2.1.2	Test Parameters	4261

A.10.5.2.1.3	Test Requirements	4265

A.10.5.2.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	4265

A.10.5.2.2.1	Test Purpose and Environment	4265

A.10.5.2.2.2	Test Parameters	4265

A.10.5.2.2.3	Test Requirements	4268

A.10.5.3	SS-SINR	4268

A.10.5.3.1	Intra-frequency measurement accuracy on PSCC	4268

A.10.5.3.1.1	Test Purpose and Environment	4268

A.10.5.3.1.2	Test Parameters	4268

A.10.5.3.1.3	Test Requirements	4271

A.10.5.3.2	Inter-frequency measurement accuracy on PSCC	4271

A.10.5.3.2.1	Test Purpose and Environment	4271

A.10.5.3.2.2	Test Parameters	4271

A.10.5.3.2.3	Test Requirements	4274

A.10.5.3.3	Intra-frequency measurement accuracy on SCC	4274

A.10.5.3.3.1	Test Purpose and Environment	4274

A.10.5.3.3.2	Test Parameters	4274

A.10.5.3.3.3	Test Requirements	4277

A.10.5.4	L1-RSRP measurement for beam reporting with CCA serving cell	4277

A.10.5.4.1	SSB based L1-RSRP measurement	4277

A.10.5.4.1.1	Test Purpose and Environment	4277

A.10.5.4.1.2	Test parameters	4278

A.10.5.4.1.3	Test Requirements	4280

A.10.5.5	RSSI	4280

A.10.5.5.1 	RSSI measurement accuracy on PSCC with CCA	4280

A.10.5.5.1.1	Test Purpose and Environment	4280

A.10.5.5.1.2	Test parameters	4280

A.10.5.5.1.3	Test Requirements	4283

A.10.5.5.2 	RSSI measurement accuracy on SCC with CCA	4283

A.10.5.5.2.1	Test Purpose and Environment	4283

A.10.5.5.2.2	Test parameters	4283

A.10.5.5.2.3	Test Requirements	4286

A.10.5.5.3 	Inter-frequency RSSI measurement accuracy on a carrier with CCA	4286

A.10.5.5.3.1	Test Purpose and Environment	4286

A.10.5.5.3.2	Test parameters	4286

A.10.5.5.3.3	Test Requirements	4290

A.10.5.6	Channel occupancy	4290

A.10.5.6.1 	Channel occupancy measurement accuracy on PSCC with CCA	4290

A.10.5.6.1.1	Test Purpose and Environment	4290

A.10.5.6.1.2	Test parameters	4290

A.10.5.6.1.3	Test Requirements	4294

A.10.5.6.2 	Channel occupancy measurement accuracy on SCC with CCA	4294

A.10.5.6.2.1	Test Purpose and Environment	4294

A.10.5.6.2.2	Test parameters	4294

A.10.5.6.2.3	Test Requirements	4297

A.10.5.6.3 	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	4297

A.10.5.6.3.1	Test Purpose and Environment	4297

A.10.5.6.3.2	Test parameters	4297

A.10.5.6.3.3	Test Requirements	4301

A.11	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	4301

A.11.1	RRC_IDLE state mobility	4301

A.11.1.1	Cell re-selection with both source and target NR carrier frequencies under CCA	4301

A.11.1.1.1	Cell reselection to FR1 intra-frequency NR cells when subject to CCA on the serving and target cell	4301

A.11.1.1.1.1	Test Purpose and Environment	4301

A.11.1.1.1.2	Test Parameters	4301

A.11.1.1.1.3	Test Requirements	4304

A.11.1.1.2	Cell reselection to FR1 inter-frequency NR case when subject to CCA on the serving and target cell	4304

A.11.1.1.2.1	Test Purpose and Environment	4304

A.11.1.1.2.2	Test Parameters	4304

A.11.1.1.2.3	Test Requirements	4308

A.11.1.2	Cell re-selection to NR with source NR carrier frequency under CCA	4308

A.11.1.2.1	Cell reselection to FR1 inter-frequency NR case when serving cell is subject to CCA	4308

A.11.1.2.1.1	Test Purpose and Environment	4308

A.11.1.2.1.2	Test Parameters	4309

A.11.1.2.1.3	Test Requirements	4315

A.11.1.3	Cell re-selection from NR carrier with target NR carrier frequency under CCA	4316

A.11.1.3.1	Cell reselection to FR1 inter-frequency NR case when target cell is subject to CCA	4316

A.11.1.3.1.1	Test Purpose and Environment	4316

A.11.1.3.1.2	Test Parameters	4316

A.11.1.3.1.3	Test Requirements	4322

A.11.1.4	Inter-RAT cell re-selection to E-UTRAN with source NR carrier frequency under CCA	4323

A.11.1.4.1	Cell reselection to higher priority E-UTRAN when serving cell is subject to CCA	4323

A.11.1.4.1.1	Test Purpose and Environment	4323

A.11.1.4.1.2	Test Parameters	4323

A.11.1.4.1.3	Test Requirements	4326

A.11.1.4.2	Cell reselection to lower priority E-UTRAN when serving cell is subject to CCA	4327

A.11.1.4.2.1	Test Purpose and Environment	4327

A.11.1.4.2.2	Test Requirements	4330

A.11.2	RRC_CONNECTED state mobility	4331

A.11.2.1	Handover	4331

A.11.2.1.1	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; known target cell	4331

A.11.2.1.1.1	Test Purpose and Environment	4331

A.11.2.1.1.2	Test Parameters	4331

A.11.2.1.1.3 Test Requirements	4334

A.11.2.1.2	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	4334

A.11.2.1.2.1	Test Purpose and Environment	4334

A.11.2.1.2.2	Test Parameters	4334

A.11.2.1.2.3	Test Requirements	4337

A.11.2.1.3	Inter-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	4337

A.11.2.1.3.1	Test Purpose and Environment	4337

A.11.2.1.3.2	Test Parameters	4337

A.11.2.1.3.3 Test Requirements	4340

A.11.2.1.4	Inter-frequency handover from FR1 carrier under CCA to FR1; known target cell	4340

A.11.2.1.4.1	Test Purpose and Environment	4340

A.11.2.1.4.2	Test Parameters	4340

A.11.2.1.4.3 Test Requirements	4344

A.11.2.1.5	Inter-frequency handover from FR1 carrier under CCA to FR1; unknown target cell	4345

A.11.2.1.5.1	Test Purpose and Environment	4345

A.11.2.1.5.2	Test Parameters	4345

A.11.2.1.5.3 Test Requirements	4348

A.11.2.1.6	Inter-frequency handover from FR1 to FR1 carrier under CCA; unknown target cell	4349

A.11.2.1.6.1	Test Purpose and Environment	4349

A.11.2.1.6.2	Test Parameters	4349

A.11.2.1.6.3 Test Requirements	4352

A.11.2.1.7	 SA NR FR1 carrier under CCA - E-UTRAN handover with known target cell	4353

A.11.2.1.7.1	Test Purpose and Environment	4353

A.11.2.1.7.2	Test Requirements	4359

A.11.2.1.8	SA NR FR1 carrier under CCA - E-UTRAN handover with unknown target cell	4359

A.11.2.1.8.1	Test Purpose and Environment	4359

A.11.2.1.8.2	Test Requirements	4364

A.11.2.1.9	Handover with PSCell from NR SA to EN-DC with known target PSCell using CCA	4364

A.11.2.1.9.1	Test Purpose and Environment	4364

A.11.2.1.9.2	Test Requirements	4373

A.11.2.2	RRC connection mobility control	4374

A.11.2.2.1	RRC re-establishment	4374

A.11.2.2.1.1	Intra-frequency RRC Re-establishment with CCA in FR1	4374

A.11.2.2.1.2	Inter-frequency RRC Re-establishment with CCA in FR1	4377

A.11.2.2.1.4	Inter-frequency RRC Re-establishment from NR FR1 carrier without CCA to NR FR1 carrier under CCA	4384

A.11.2.2.2	Random Access	4389

A.11.2.2.2.1	4-step RA type contention-based random access for NR PCell with CCA	4389

A.11.2.2.2.1.1	Test Purpose and Environment	4389

A.11.2.2.2.1.2	Test Requirements	4392

A.11.2.2.2.1.2.1	Random Access Preamble Transmission	4392

A.11.2.2.2.1.2.2	Random Access Response Reception	4392

A.11.2.2.2.1.2.3	No Random Access Response Reception	4393

A.11.2.2.2.1.2.4	Receiving an UL grant for msg3 retransmission	4393

A.11.2.2.2.1.2.5	Reception of an Incorrect Message over Temporary C-RNTI	4393

A.11.2.2.2.1.2.6	Reception of a Correct Message over Temporary C-RNTI	4394

A.11.2.2.2.1.2.7	Contention Resolution Timer expiry	4394

A.11.2.2.2.2	4-step RA type non-contention based random access for NR PSCell with CCA	4394

A.11.2.2.2.2.1	Test Purpose and Environment	4394

A.11.2.2.2.2.2	Test Requirements	4397

A.11.2.2.2.2.2.1	SSB-based Random Access Preamble Transmission	4397

A.11.2.2.2.2.2.2	Random Access Response Reception	4398

A.11.2.2.2.2.2.3	No Random Access Response Reception	4398

A.11.2.2.2.3	2-step RA type contention-based random access for NR PCell with CCA	4398

A.11.2.2.2.3.1	Test Purpose and Environment	4398

A.11.2.2.2.3.2	Test Requirements	4402

A.11.2.2.2.3.2.1	MsgA Transmission	4402

A.11.2.2.2.3.2.2	MsgB Reception	4402

A.11.2.2.2.3.2.3	No MsgB Reception	4403

A.11.2.2.2.4	2-step RA type non-contention-based random access for NR PCell with CCA	4403

A.11.2.2.2.4.1	Test Purpose and Environment	4403

A.11.2.2.2.4.2	Test Requirements	4407

A.11.2.2.2.4.2.1	MsgA Transmission	4407

A.11.2.2.2.4.2.2	MsgB Reception	4408

A.11.2.2.2.4.2.3	No MsgB Reception	4408

A.11.2.2.3	RRC connection release with redirection	4409

A.11.2.2.3.1	Redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA	4409

A.11.2.2.3.2	Redirection from NR FR1 carrier without CCA to NR FR1 carrier with CCA	4413

A.11.3	Timing	4418

A.11.3.1	UE transmit timing	4418

A.11.3.1.1	UE Transmit Timing Test with PCell under DL CCA	4418

A.11.3.1.1.1	Test Purpose and environment	4418

A.11.3.1.1.2	Test requirements	4421

A.11.3.2	UE timing advance	4422

A.11.3.2.1	UE Timing Advance Adjustment Accuracy with PCell under DL CCA	4422

A.11.3.2.1.1	Test Purpose and Environment	4422

A.11.3.2.1.2	Test Parameters	4422

A.11.3.2.1.3	Test Requirements	4426

A.11.4	Signalling characteristics	4426

A.11.4.1	Radio link monitoring	4426

A.11.4.1.1	Introduction	4426

A.11.4.1.2	Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	4427

A.11.4.1.2.1	Test purpose and environment	4427

A.11.4.1.2.2	Test requirements	4431

A.11.4.1.3	Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	4431

A.11.4.1.3.1	Test purpose and environment	4431

A.11.4.1.3.2	Test requirements	4437

A.11.4.1.4	Void	4437

A.11.4.1.4.1	Void	4437

A.11.4.1.4.2	Void	4437

A.11.4.1.5	Void	4437

A.11.4.1.5.1	Void	4437

A.11.4.1.5.2	Void	4437

A.11.4.2	Interruption	4438

A.11.4.2.1	NR interruptions during Scell operations with CCA on PCell and SCell	4438

A.11.4.2.1.1 Test Purpose and Environment	4438

A.11.4.2.1.2	Test Requirements	4442

A.11.4.3	SCell activation and deactivation delay	4442

A.11.4.3.1	SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 160 ms SCell measurement cycle	4442

A.11.4.3.1.1	Test Purpose and Environment	4442

A.11.4.3.1.2	Test Requirements	4447

A.11.4.3.2	SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 640 ms SCell measurement cycle	4447

A.11.4.3.2.1	Test Purpose and Environment	4447

A.11.4.3.2.2	Test Requirements	4448

A.11.4.3.3	SCell Activation and Deactivation of unknown SCell with PCell and SCell under CCA	4448

A.11.4.3.3.1	Test Purpose and Environment	4448

A.11.4.3.3.2	Test Requirements	4449

A.11.4.4	Beam failure detection and link recovery procedures	4449

A.11.4.4.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	4449

A.11.4.4.1.1	Test Purpose and Environment	4449

A.11.4.4.1.2	Test Requirements	4454

A.11.4.4.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	4454

A.11.4.4.2.1	Test Purpose and Environment	4454

A.11.4.4.2.2	Test Requirements	4460

A.11.4.5	Active BWP switching	4460

A.11.4.5.1	UL active BWP switch delay with consistent UL LBT failure on PCell subject to UL CCA	4460

A.11.4.5.1.1	Test Purpose and Environment	4460

A.11.4.5.1.2	Test Requirements	4464

A.11.4.5.2	DCI-based and Timer-based Active BWP Switch	4465

A.11.4.5.2.1	NR FR1- NR FR1 DL active BWP switch of PCell with non-DRX in SA	4465

A.11.4.5.2.2	NR FR1 DL active BWP switch with non-DRX in SA	4468

A.11.4.5.3	RRC-based Active BWP Switch	4471

A.11.4.5.3.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	4471

A.11.4.6	Void	4474

A.11.5	Measurement procedure	4474

A.11.5.1	Intra-frequency measurements	4474

A.11.5.1.1	Event-triggered reporting tests on PCC without gaps under non-DRX	4474

A.11.5.1.1.1	Test purpose and environment	4474

A.11.5.1.1.2	Test parameters	4474

A.11.5.1.1.3	Test Requirements	4477

A.11.5.1.2	Event-triggered reporting tests on PCC without gaps under DRX	4477

A.11.5.1.2.1	Test purpose and environment	4477

A.11.5.1.2.2	Test parameters	4477

A.11.5.1.2.3	Test Requirements	4480

A.11.5.1.3	Event-triggered reporting tests on PCC with per-UE gaps under non-DRX	4480

A.11.5.1.3.1	Test purpose and environment	4480

A.11.5.1.3.2	Test parameters	4480

A.11.5.1.3.3	Test Requirements	4480

A.11.5.1.4	Event-triggered reporting tests on PCC with per-UE gaps under DRX	4481

A.11.5.1.4.1	Test purpose and environment	4481

A.11.5.1.4.2	Test parameters	4481

A.11.5.1.4.3	Test Requirements	4481

A.11.5.1.5	Event-triggered reporting tests on SCC without gaps under non-DRX	4481

A.11.5.1.5.1	Test purpose and environment	4481

A.11.5.1.5.2	Test parameters	4481

A.11.5.1.6	Event-triggered reporting tests on SCC without gaps under DRX	4482

A.11.5.1.6.1	Test purpose and environment	4482

A.11.5.1.6.2	Test parameters	4482

A.11.5.1.6.3	Test Requirements	4482

A.11.5.1.7	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	4482

A.11.5.1.7.1	Test purpose and environment	4482

A.11.5.1.7.2	Test parameters	4482

A.11.5.1.7.3	Test Requirements	4482

A.11.5.1.8	Event-triggered reporting tests on SCC with per-UE gaps under DRX	4483

A.11.5.1.8.1	Test purpose and environment	4483

A.11.5.1.8.2	Test parameters	4483

A.11.5.1.8.3	Test Requirements	4483

A.11.5.1.9	RSSI measurement reporting on PCC	4483

A.11.5.1.9.1	Test purpose and environment	4483

A.11.5.1.9.2	Test parameters	4483

A.11.5.1.10	Channel occupancy measurement reporting on PCC	4484

A.11.5.1.10.1	Test purpose and environment	4484

A.11.5.1.10.2	Test parameters	4484

A.11.5.1.11	RSSI measurement reporting on SCC	4484

A.11.5.1.11.1	Test purpose and environment	4484

A.11.5.1.11.2	Test parameters	4484

A.11.5.1.12	Channel occupancy measurement reporting on SCC	4485

A.11.5.1.12.1	Test purpose and environment	4485

A.11.5.1.12.2	Test parameters	4485

A.11.5.2	Inter-frequency measurements	4485

A.11.5.2.1	RSSI measurement reporting	4485

A.11.5.2.1.1	Test purpose and environment	4485

A.11.5.2.1.2	Test parameters	4485

A.11.5.2.2	Channel occupancy measurement reporting	4486

A.11.5.2.2.1	Test purpose and environment	4486

A.11.5.2.2.2	Test parameters	4486

A.11.5.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	4486

A.11.5.2.3.1	Test Purpose and Environment	4486

A.11.5.2.3.2	Test Requirements	4490

A.11.5.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	4491

A.11.5.2.4.1	Test Purpose and Environment	4491

A.11.5.2.4.2	Test Requirements	4496

A.11.5.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	4496

A.11.5.2.5.1	Test Purpose and Environment	4496

A.11.5.2.5.2	Test Requirements	4500

A.11.5.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	4501

A.11.5.2.6.1	Test Purpose and Environment	4501

A.11.5.2.6.2	Test Requirements	4506

A.11.5.2.7	Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4506

A.11.5.2.7.1	Test Purpose and Environment	4506

A.11.5.2.7.2	Test Requirements	4510

A.11.5.2.8	Event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4511

A.11.5.2.8.1	Test Purpose and Environment	4511

A.11.5.2.8.2	Test Requirements	4516

A.11.5.2.9	Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4516

A.11.5.2.9.1	Test Purpose and Environment	4516

A.11.5.2.9.2	Test Requirements	4521

A.11.5.2.10	Event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4522

A.11.5.2.10.1	Test Purpose and Environment	4522

A.11.5.2.10.2	Test Requirements	4527

A.11.5.3	Inter-RAT E-UTRAN measurements	4528

A.11.5.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1	4528

A.11.5.3.1.1	Test Purpose and Environment	4528

A.11.5.3.1.2	Test Requirements	4532

A.11.5.3.2	SA NR - E-UTRAN event-triggered reporting in DRX in FR1	4532

A.11.5.3.2.1	Test Purpose and Environment	4532

A.11.5.3.2.2	Test Requirements	4537

A.11.5.4	L1-RSRP measurements for beam reporting	4537

A.11.5.4.1	SSB based L1-RSRP measurement when DRX is not used	4537

A.11.5.4.1.1	Test Purpose and Environment	4537

A.11.5.4.1.2	Test parameters	4537

A.11.5.4.1.3	Test Requirements	4539

A.11.5.4.2	SSB based L1-RSRP measurement when DRX is used	4539

A.11.5.4.2.1	Test Purpose and Environment	4539

A.11.5.4.2.2	Test parameters	4540

A.11.5.4.2.3	Test Requirements	4542

A.11.5.4.3	SSB based L1-RSRP measurement on SCC when DRX is not used	4542

A.11.5.4.3.1	Test Purpose and Environment	4542

A.11.5.4.3.2	Test parameters	4543

A.11.5.4.3.3	Test Requirements	4546

A.11.5.4.4	SSB based L1-RSRP measurement on SCC when DRX is used	4547

A.11.5.4.4.1	Test Purpose and Environment	4547

A.11.5.4.4.2	Test parameters	4547

A.11.5.4.4.3	Test Requirements	4550

A.11.6	Measurement performance	4551

A.11.6.1	SS-RSRP	4551

A.11.6.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	4551

A.11.6.1.1.1	Test Purpose and Environment	4551

A.11.6.1.1.2	Test parameters	4551

A.11.6.1.1.3	Test Requirements	4553

A.11.6.1.2	Intra-frequency measurement accuracy on SCC on a carrier frequency with CCA	4553

A.11.6.1.2.1	Test Purpose and Environment	4553

A.11.6.1.2.2	Test parameters	4553

A.11.6.1.2.3	Test Requirements	4556

A.11.6.2	SS-RSRQ	4556

A.11.6.2.1	Intra-frequency measurement accuracy	4556

A.11.6.2.1.1	Test Purpose and Environment	4556

A.11.6.2.1.2	Test Parameters	4556

A.11.6.2.1.3	Test Requirements	4559

A.11.6.2.2	Inter-frequency measurement accuracy	4559

A.11.6.2.2.1	Test Purpose and Environment	4559

A.11.6.2.2.2	Test Parameters	4559

A.11.6.2.2.3	Test Requirements	4562

A.11.6.2.3	Intra-frequency measurement accuracy on SCC	4562

A.11.6.2.3.1	Test Purpose and Environment	4562

A.11.6.2.3.2	Test Parameters	4562

A.11.6.2.3.3	Test Requirements	4565

A.11.6.2.4	Inter-frequency measurement accuracy	4565

A.11.6.2.4.1	Test Purpose and Environment	4565

A.11.6.2.4.2	Test Parameters	4565

A.11.6.2.4.3	Test Requirements	4574

A.11.6.3	SS-SINR	4574

A.11.6.3.1	Intra-frequency measurement accuracy	4574

A.11.6.3.1.1	Test Purpose and Environment	4574

A.11.6.3.1.2	Test Parameters	4574

A.11.6.3.1.3	Test Requirements	4577

A.11.6.3.2	Inter-frequency measurement accuracy	4577

A.11.6.3.2.1	Test Purpose and Environment	4577

A.11.6.3.2.2	Test Parameters	4577

A.11.6.3.2.3	Test Requirements	4580

A.11.6.3.3	Intra-frequency measurement accuracy on SCC	4580

A.11.6.3.3.1	Test Purpose and Environment	4580

A.11.6.3.3.2	Test Parameters	4580

A.11.6.3.3.3	Test Requirements	4583

A.11.6.3.4	Inter-frequency measurement accuracy	4583

A.11.6.3.4.1	Test Purpose and Environment	4583

A.11.6.3.4.2	Test Parameters	4583

A.11.6.3.4.3	Test Requirements	4591

A.11.6.4	L1-RSRP measurement for beam reporting with CCA serving cell	4591

A.11.6.4.1	SSB based L1-RSRP measurement	4591

A.11.6.4.1.1	Test Purpose and Environment	4591

A.11.6.4.1.2	Test parameters	4592

A.11.6.4.1.3	Test Requirements	4595

A.11.6.5	RSSI	4595

A.11.6.5.1 	Intra-frequency RSSI measurement accuracy on PCC with CCA	4595

A.11.6.5.1.1	Test Purpose and Environment	4595

A.11.6.5.1.2	Test parameters	4595

A.11.6.5.1.3	Test Requirements	4598

A.11.6.5.2 	Intra-frequency RSSI measurement accuracy on SCC with CCA	4598

A.11.6.5.2.1	Test Purpose and Environment	4598

A.11.6.5.2.2	Test parameters	4598

A.11.6.5.2.3	Test Requirements	4601

A.11.6.5.3	Inter-frequency RSSI measurement accuracy on a carrier with CCA	4601

A.11.6.5.3.1	Test Purpose and Environment	4601

A.11.6.5.3.2	Test parameters	4601

A.11.6.5.3.3	Test Requirements	4604

A.11.6.6	Channel occupancy	4604

A.11.6.6.1	Intra-frequency channel occupancy measurement accuracy on PCC with CCA	4604

A.11.6.6.1.1	Test Purpose and Environment	4604

A.11.6.6.1.2	Test parameters	4604

A.11.6.6.1.3	Test Requirements	4607

A.11.6.6.2	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	4607

A.11.6.6.2.1	Test Purpose and Environment	4607

A.11.6.6.2.2	Test parameters	4607

A.11.6.6.2.3	Test Requirements	4610

A.11.6.6.3	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	4610

A.11.6.6.3.1	Test Purpose and Environment	4610

A.11.6.6.3.2	Test parameters	4610

A.11.6.6.3.3	Test Requirements	4613

A.11.6.7	E-UTRAN RSRP	4613

A.11.6.8	E-UTRAN RSRQ	4613

A.11.6.9	E-UTRAN SINR	4613

A.12	E-UTRA Standalone Tests with at Least One NR Cell under CCA	4613

A.12.1	RRC_IDLE state mobility	4613

A.12.1.1	Inter-RAT cell re-selection to NR on a carrier frequency with CCA	4613

A.12.1.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1 when target cell is subject to CCA	4613

A.12.1.1.1.1	Test Purpose and Environment	4613

A.12.1.1.1.2	Test Requirements	4618

A.12.2	RRC_CONNECTED state mobility	4619

A.12.2.1	Handover	4619

A.12.2.1.1	E-UTRAN - NR with CCA handover	4619

A.12.2.1.1.1	Test Purpose and Environment	4619

A.12.2.1.1.2	Test Requirements	4624

A.12.3	Signalling characteristics	4625

A.12.3.1	Interruptions	4625

A.12.4	Measurement procedure	4625

A.12.4.1	E-UTRANNR inter-RAT SFTD measurements	4625

A.12.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay with NR under CCA in non-DRX	4625

A.12.4.1.1.1	Test Purpose and Environment	4625

A.12.4.1.1.2	Test Requirements	4629

A.12.4.2	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	4629

A.12.4.2.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4629

A.12.4.2.1.1	Test Purpose and Environment	4629

A.12.4.2.1.2	Test Requirements	4633

A.12.4.2.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4634

A.12.4.2.2.1	Test Purpose and Environment	4634

A.12.4.2.2.2	Test Requirements	4638

A.12.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4639

A.12.4.2.3.1	Test Purpose and Environment	4639

A.12.4.2.3.2	Test Requirements	4643

A.12.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4644

A.12.4.2.4.1	Test Purpose and Environment	4644

A.12.4.2.4.2	Test Requirements	4648

A.12.4.2.5	RSSI measurement reporting	4649

A.12.4.2.5.1	Test purpose and environment	4649

A.12.4.2.5.2	Test parameters	4649

A.12.4.2.5.3	Test Requirements	4652

A.12.4.2.6	Channel occupancy measurement reporting	4652

A.12.4.2.6.1	Test purpose and environment	4652

A.12.4.2.6.2	Test parameters	4652

A.12.5	Measurement performance	4655

A.12.5.1	E-UTRANNR SFTD	4655

A.12.5.1.1	Inter-RAT SFTD accuracy with NR target cell under CCA	4655

A.12.5.1.1.1	Test Purpose	4655

A.12.5.1.1.2	Test Environment	4655

A.12.5.1.1.3	Test Requirements	4660

A.12.5.2	E-UTRANNR SS-RSRP	4660

A.12.5.3	E-UTRANNR SS-RSRQ	4660

A.12.5.4	E-UTRANNR SS-SINR	4660

A.12.5.5	E-UTRANNR RSSI	4660

A.12.5.6	E-UTRANNR channel occupancy	4660

A.13	NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	4660

A.13.1	Timing	4660

A.13.1.1	UE transmit timing	4660

A.13.1.2	Timing advance	4660

A.13.2	Signalling characteristics	4660

A.13.2.1	Interruption	4660

A.13.2.1.1	NR interruptions during SCell operations with CCA on SCell	4660

A.13.2.1.1.1	Test Purpose and Environment	4660

A.13.2.1.1.2	Test Requirements	4664

A.13.2.2	SCell activation and deactivation delay	4664

A.13.2.2.1	SCell Activation and Deactivation of known SCell under CCA, 160 ms SCell measurement cycle	4664

A.13.2.2.1.1	Test Purpose and Environment	4664

A.13.2.2.1.2	Test Requirements	4669

A.13.2.2.2 SCell Activation and Deactivation of known SCell under CCA, 640 ms SCell measurement cycle	4669

A.13.2.2.2.1	Test Purpose and Environment	4669

A.13.2.2.2.2	Test Requirements	4670

A.13.2.2.3	SCell Activation and Deactivation of unknown SCell under CCA	4670

A.13.2.2.3.1	Test Purpose and Environment	4670

A.13.2.2.3.2	Test Requirements	4671

A.13.2.3	Void	4671

A.13.3	Measurement procedure	4671

A.13.3.1	Intra-frequency measurements	4671

A.13.3.1.1	Event-triggered reporting tests on SCC without gaps under non-DRX	4671

A.13.3.1.1.1	Test purpose and environment	4671

A.13.3.1.1.2	Test parameters	4671

A.13.3.1.1.3	Test Requirements	4676

A.13.3.1.2	Event-triggered reporting tests on SCC without gaps under DRX	4676

A.13.3.1.2.1	Test purpose and environment	4676

A.13.3.1.2.2	Test parameters	4676

A.13.3.1.2.3	Test Requirements	4681

A.13.3.1.3	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	4681

A.13.3.1.3.1	Test purpose and environment	4681

A.13.3.1.3.2	Test parameters	4681

A.13.3.1.3.3	Test Requirements	4686

A.13.3.1.4	Event-triggered reporting tests on SCC with per-UE gaps under DRX	4686

A.13.3.1.4.1	Test purpose and environment	4686

A.13.3.1.4.2	Test parameters	4686

A.13.3.1.4.3	Test Requirements	4691

A.13.3.1.5	RSSI measurement reporting on SCC	4691

A.13.3.1.5.1	Test purpose and environment	4691

A.13.3.1.5.2	Test parameters	4691

A.13.3.1.6	Channel occupancy measurement reporting on SCC	4692

A.13.3.1.6.1	Test purpose and environment	4692

A.13.3.1.6.2	Test parameters	4693

A.13.3.2	Inter-frequency measurements	4693

A.13.3.2.1	RSSI measurement reporting	4693

A.13.3.2.1.1	Test purpose and environment	4693

A.13.3.2.1.2	Test parameters	4693

A.13.3.2.2	Channel occupancy measurement reporting	4694

A.13.3.2.2.1	Test purpose and environment	4694

A.13.3.2.2.2	Test parameters	4694

A.13.3.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	4694

A.13.3.2.3.1	Test Purpose and Environment	4694

A.13.3.2.3.2	Test Requirements	4699

A.13.3.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	4699

A.13.3.2.4.1	Test Purpose and Environment	4699

A.13.3.2.4.2	Test Requirements	4704

A.13.3.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	4705

A.13.3.2.5.1	Test Purpose and Environment	4705

A.13.3.2.5.2	Test Requirements	4710

A.13.3.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	4710

A.13.3.2.6.1	Test Purpose and Environment	4710

A.13.3.2.6.2	Test Requirements	4716

A.13.3.3	L1-RSRP measurements for beam reporting	4717

A.13.3.3.1	SSB based L1-RSRP measurement when DRX is not used	4717

A.13.3.3.1.1	Test Purpose and Environment	4717

A.13.3.3.1.2	Test parameters	4717

A.13.3.3.1.3	Test Requirements	4721

A.13.3.3.2	SSB based L1-RSRP measurement when DRX is used	4721

A.13.3.3.2.1	Test Purpose and Environment	4721

A.13.3.3.2.2	Test parameters	4721

A.13.3.3.2.3	Test Requirements	4725

A.13.4	Measurement performance	4725

A.13.4.1	SS-RSRP	4725

A.13.4.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	4725

A.13.4.1.1.1	Test Purpose and Environment	4725

A.13.4.1.1.2	Test parameters	4725

A.13.4.1.1.3	Test Requirements	4727

A.13.4.2	SS-RSRQ	4727

A.13.4.2.1	Intra-frequency measurement accuracy on SCC	4727

A.13.4.2.1.1	Test Purpose and Environment	4727

A.13.4.2.1.2	Test Parameters	4727

A.13.4.2.1.3	Test Requirements	4735

A.13.4.3	SS-SINR	4735

A.13.4.3.1	Intra-frequency measurement accuracy on SCC	4735

A.13.4.3.1.1	Test Purpose and Environment	4735

A.13.4.3.1.2	Test Parameters	4735

A.13.4.3.1.3	Test Requirements	4742

A.13.4.4	L1-RSRP measurement for beam reporting with CCA serving cell	4742

A.13.4.4.1	SSB based L1-RSRP measurement	4742

A.13.4.4.1.1	Test Purpose and Environment	4742

A.13.4.4.1.2	Test parameters	4742

A.13.4.4.1.3	Test Requirements	4746

A.13.4.5	RSSI	4746

A.13.4.5.1 	Intra-frequency RSSI measurement accuracy on a carrier with CCA	4746

A.13.4.5.1.1	Test Purpose and Environment	4746

A.13.4.5.1.2	Test parameters	4746

A.13.4.5.1.3	Test Requirements	4749

A.13.4.5.2	Inter-frequency RSSI measurement accuracy on a carrier with CCA	4749

A.13.4.5.2.1	Test Purpose and Environment	4749

A.13.4.5.2.2	Test parameters	4749

A.13.4.5.2.3	Test Requirements	4753

A.13.4.6	Channel occupancy	4753

A.13.4.6.1	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	4753

A.13.4.6.1.1	Test Purpose and Environment	4753

A.13.4.6.1.2	Test parameters	4753

A.13.4.6.1.3	Test Requirements	4757

A.13.4.6.2	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	4757

A.13.4.6.2.1	Test Purpose and Environment	4757

A.13.4.6.2.2	Test parameters	4757

A.13.4.6.2.3	Test Requirements	4761

A.14	NR standalone tests for Satellite access	4761

A.14.1	RRC_IDLE state mobility	4761

A.14.1.1	Cell reselection to FR1 intra-frequency NR case	4761

A.14.1.1.1	Test Purpose and Environment	4761

A.14.1.1.2	Test Parameters	4761

A.14.1.1.3	Test Requirements	4763

A.14.1.2	Cell reselection to FR1 intra-frequency NR cell for UE configured with the feature for enhanced requirements	4764

A.14.1.2.1	Test Purpose and Environment	4764

A.14.1.2.2	Test Parameters	4764

A.14.1.2.3	Test Requirements	4768

A.14.1.3	Time-based measurement initiation to FR1 intra-frequency NR cell reselection	4769

A.14.1.3.1	Test Purpose and Environment	4769

A.14.1.3.2	Test Parameters	4769

A.14.1.3.3	Test Requirements	4770

A.14.1.4	Location-based cell measurement initiation to FR1 intra-frequency NR cell reselection	4771

A.14.1.4.1	Test Purpose and Environment	4771

A.14.1.4.2	Test Parameters	4771

A.14.1.4.3	Test Requirements	4773

A.14.1.5	Cell reselection to FR1 inter-frequency NR case	4774

A.14.1.5.1	Test Purpose and Environment	4774

A.14.1.5.2	Test Parameters	4774

A.14.1.5.3	Test Requirements	4776

A.14.1.6	Cell reselection to FR1 inter-frequency NR cell for UE configured with [capability for enhanced requirements]	4777

A.14.1.6.1	Test Purpose and Environment	4777

A.14.1.6.2	Test Parameters	4777

A.14.1.6.3	Test Requirements	4779

A.14.1.7	Time-based Cell measurement initiation to FR1 inter-frequency cell reselection	4780

A.14.1.7.1	Test Purpose and Environment	4780

A.14.1.7.2	Test Parameters	4780

A.14.1.7.3	Test Requirements	4781

A.14.1.8	Location-based measurement initiation to FR1 inter-frequency NR cell reselection	4782

A.14.1.8.1	Test Purpose and Environment	4782

A.14.1.8.2	Test Parameters	4782

A.14.1.8.3	Test Requirements	4784

A.14.1.9	Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	4785

A.14.1.9.1	Test Purpose and Environment	4785

A.14.1.9.2	Test Parameters	4785

A.14.1.9.3	Test Requirements	4788

A.14.1.10	Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	4788

A.14.1.10.1	Test Purpose and Environment	4788

A.14.1.10.2	Test Parameters	4788

A.14.1.10.3	Test Requirements	4790

A.14.2	RRC_CONNECTED state mobility	4791

A.14.2.1	Handover	4791

A.14.2.1.1	Intra-frequency SAN Handover from FR1 to FR1	4791

A.14.2.1.1.1	Test Purpose and Environment	4791

A.14.2.1.1.2	Test Parameters	4791

A.14.2.1.1.3	Test Requirements	4793

A.14.2.1.2	Inter-frequency SAN Handover from FR1 to FR1	4794

A.14.2.1.2.1	Test Purpose and Environment	4794

A.14.2.1.2.2	Test Parameters	4794

A.14.2.1.2.3	Test Requirements	4796

A.14.2.1.3	Intra-frequency SAN time-based conditional Handover from FR1 to FR1	4796

A.14.2.1.3.1	Test Purpose and Environment	4796

A.14.2.1.3.2	Test Parameters	4796

A.14.2.1.3.3	Test Requirements	4799

A.14.2.1.4	Inter-frequency SAN time-based conditional Handover from FR1 to FR1	4799

A.14.2.1.4.1	Test Purpose and Environment	4799

A.14.2.1.4.2	Test Parameters	4799

A.14.2.1.4.3	Test Requirements	4801

A.14.2.1.5	Intra-frequency SAN distance-based conditional Handover from FR1 to FR1	4801

A.14.2.1.5.1	Test Purpose and Environment	4801

A.14.2.1.5.2	Test Parameters	4802

A.14.2.1.5.3	Test Requirements	4805

A.14.2.1.6	Inter-frequency SAN distance-based conditional Handover from FR1 to FR1	4805

A.14.2.1.6.1	Test Purpose and Environment	4805

A.14.2.1.6.2	Test Parameters	4805

A.14.2.1.6.3	Test Requirements	4809

A.14.2.2	RRC Connection Mobility Control	4809

A.14.2.2.1	SA: RRC Re-establishment for SAN	4809

A.14.2.2.1.1	Intra-frequency RRC Re-establishment in FR1	4809

A.14.2.2.1.2	Inter-frequency RRC Re-establishment in FR1	4812

A.14.2.2.2	Random Access	4815

A.14.2.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone	4815

A.14.2.2.2.1.1	Test Purpose and Environment	4815

A.14.2.2.2.1.2	Test Requirements	4818

A.14.2.2.2.2	4-step RA type non-contention based random access test in FR1 for NR standalone	4819

A.14.2.2.2.2.1	Test Purpose and Environment	4819

A.14.2.2.2.2.2	Test Requirements	4823

A.14.2.2.3	RRC Connection Release with Redirection	4824

A.14.2.2.3.1	Redirection from NR in FR1 to NR in FR1	4824

A.14.2.2.3.1.1	Test Purpose and Environment	4824

A.14.2.2.3.1.2	Test Parameters	4824

A.14.2.2.3.1.3	Test Requirements	4827

A.14.3	Timing for Satellite Access	4827

A.14.3.1	UE transmit timing for Satellite Access	4827

A.14.3.1.1	NR UE Transmit Timing Test for FR1	4827

A.14.3.1.1.1	Test Purpose and environment	4827

A.14.3.1.1.2	Test requirements	4829

A.14.3.2	Timing advance for satellite access	4830

A.14.3.2.1	SA FR1 timing advance adjustment accuracy	4830

A.14.3.2.1.1	Test Purpose and Environment	4830

A.14.3.2.1.2	Test Parameters	4830

A.14.3.2.1.3	Test Requirements	4833

A.14.4	Signalling characteristics	4833

A.14.4.1	Radio link Monitoring	4833

A.14.4.1.1	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in non-DRX mode	4833

A.14.4.1.1.1	Test Purpose and Environment	4833

A.14.4.1.1.2	Test Requirements	4838

A.14.4.1.2	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in non-DRX mode	4838

A.14.4.1.2.1	Test Purpose and Environment	4838

A.14.4.1.2.2	Test Requirements	4844

A.14.4.1.3	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in DRX mode	4844

A.14.4.1.3.1	Test Purpose and Environment	4844

A.14.4.1.3.2	Test Requirements	4848

A.14.4.1.4	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in DRX mode	4848

A.14.4.1.4.1	Test Purpose and Environment	4848

A.14.4.1.4.2	Test Requirements	4853

A.14.4.1.5	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in non-DRX mode	4853

A.14.4.1.5.1	Test Purpose and Environment	4853

A.14.4.1.5.2	Test Requirements	4858

A.14.4.1.6	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in non-DRX mode	4858

A.14.4.1.6.1	Test Purpose and Environment	4858

A.14.4.1.6.2	Test Requirements	4862

A.14.4.1.7	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in DRX mode	4862

A.14.4.1.7.1	Test Purpose and Environment	4862

A.14.4.1.7.2	Test Requirements	4867

A.14.4.1.8	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in DRX mode	4867

A.14.4.1.8.1	Test Purpose and Environment	4867

A.14.4.1.8.2	Test Requirements	4872

A.14.4.2	Beam Failure Detection and Link recovery procedures for satellite access	4872

A.14.4.2.1	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with SSB-based BFD and LR in non-DRX mode	4872

A.14.4.2.1.1	Test Purpose and Environment	4872

A.14.4.2.1.2	Test Requirements	4878

A.14.4.2.2	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with SSB-based BFD and LR in DRX mode	4878

A.14.4.2.2.1	Test Purpose and Environment	4878

A.14.4.2.2.2	Test Requirements	4883

A.14.4.2.3	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with CSI-RS-based BFD and LR in non-DRX mode	4883

A.14.4.2.3.1	Test Purpose and Environment	4883

A.14.4.2.3.2	Test Requirements	4889

A.14.4.2.4	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with CSI-RS-based BFD and LR in DRX mode	4889

A.14.4.2.4.1	Test Purpose and Environment	4889

A.14.4.2.4.2	Test Requirements	4894

A.14.4.2.5	Beam Failure Detection and Link Recovery Test for FR1 SCell for satellite access configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	4894

A.14.4.2.5.1	Test Purpose and Environment	4894

A.14.4.2.5.2	Test Requirements	4899

A.14.4.2.6	Beam Failure Detection and Link Recovery Test for FR1 SCell for satellite access configured with CSI-RS-based BFD and SSB-based LR in DRX mode	4899

A.14.4.2.6.1	Test Purpose and Environment	4899

A.14.4.2.6.2	Test Requirements	4904

A.14.4.3	Active BWP switch for satellite access	4904

A.14.4.3.1	DCI-based and Timer-based Active BWP Switch	4904

A.14.4.3.1.1	NR FR1 DL active BWP switch with non-DRX in SA	4904

A.14.4.3.1.1.1	Test Purpose and Environment	4904

A.14.4.3.1.1.2	Test Requirements	4909

A.14.4.3.2	RRC-based Active BWP Switch	4909

A.14.4.3.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	4909

A.14.4.3.2.1.1	Test Purpose and Environment	4909

A.14.4.3.2.1.2	Test Requirements	4913

A.14.4.4	UE specific CBW change for satellite access	4913

A.14.4.4.1	UE specific CBW change on PCell in FR1 in non-DRX	4913

A.14.4.4.1.1	Test Purpose and Environment	4913

A.14.4.4.1.2	Test Requirements	4917

A.14.4.5	Pathloss reference signal switching delay	4917

A.14.4.5.1	MAC-CE based pathloss reference signal switch delay	4917

A.14.4.5.1.1	Test Purpose and Environment	4917

A.14.4.5.1.2	Test Requirements	4919

A.14.5	Measurement procedure	4920

A.14.5.1	Intra-frequency Measurements	4920

A.14.5.1.1	SA event triggered reporting tests without gap under non-DRX	4920

A.14.5.1.1.1	Test purpose and Environment	4920

A.14.5.1.1.2	Test parameters	4920

A.14.5.1.1.3	Test Requirements	4922

A.14.5.1.2	SA event triggered reporting tests without gap under DRX	4923

A.14.5.1.2.1	Test purpose and Environment	4923

A.14.5.1.2.2	Test parameters	4923

A.14.5.1.2.3	Test Requirements	4924

A.14.5.1.3	SA event triggered reporting tests without gap under non-DRX with SSB index reading	4925

A.14.5.1.3.1	Test purpose and Environment	4925

A.14.5.1.3.2	Test parameters	4925

A.14.5.1.3.3	Test Requirements	4927

A.14.5.1.4	SA event triggered reporting tests with single measurement gap under non-DRX for satellite access	4928

A.14.5.1.4.1	Test purpose and Environment	4928

A.14.5.1.4.2	Test parameters	4928

A.14.5.1.4.3	Test Requirements	4930

A.14.5.1.5	SA event triggered reporting tests with FNO concurrent gaps under DRX for satellite access	4931

A.14.5.1.5.1	Test purpose and Environment	4931

A.14.5.1.5.2	Test parameters	4931

A.15.5.1.5.3	Test Requirements	4933

A.14.5.1.6	SA event triggered reporting tests with PPO concurrent gaps under non-DRX with SSB index reading for satellite access	4934

A.14.5.1.6.1	Test purpose and Environment	4934

A.14.5.1.6.2	Test parameters	4934

A.14.5.1.6.3	Test Requirements	4936

A.14.5.2	Inter-frequency Measurements	4937

A.14.5.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with single gap for satellite access	4937

A.14.5.2.1.1	Test Purpose and Environment	4937

A.14.5.2.1.2	Test Requirements	4941

A.14.5.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used with single gap for satellite access	4941

A.14.5.2.2.1	Test Purpose and Environment	4941

A.14.5.2.2.2	Test Requirements	4945

A.14.5.2.3	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used with single gap for satellite access	4945

A.14.5.2.3.1	Test Purpose and Environment	4945

A.14.5.2.3.2	Test Requirements	4949

A.14.5.2.4	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with two gaps in fully non-overlapped for satellite access	4949

A.14.5.2.4.1	Test Purpose and Environment	4949

A.14.5.2.4.2	Test Requirements	4953

A.14.5.2.5	void	4953

A.14.5.2.5.1	void	4953

A.14.5.2.5.2	void	4953

A.14.5.2.6	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with two gaps in partially partial overalpping for satellite access	4953

A.14.5.2.6.1	Test Purpose and Environment	4953

A.14.5.2.6.2	Test Requirements	4957

A.14.5.2.7	Event triggered reporting test without gap under non-DRX	4957

A.14.5.2.7.1	Test purpose and Environment	4957

A.14.5.2.7.2	Test parameters	4957

A.14.5.2.7.3	Test Requirements	4959

A.14.5.2.8	Event triggered reporting tests without gap under DRX	4960

A.14.5.2.8.1	Test purpose and Environment	4960

A.14.5.2.8.2	Test parameters	4960

A.14.5.2.8.3	Test Requirements	4961

A.14.5.3	L1-RSRP measurement for beam reporting for satellite access	4962

A.14.5.3.1	SSB based L1-RSRP measurement for satellite access when DRX is not used	4962

A.14.5.3.1.1	Test Purpose and Environment	4962

A.14.5.3.1.2	Test parameters	4962

A.14.5.3.1.3	Test Requirements	4965

A.14.5.3.2	SSB based L1-RSRP measurement for satellite access when DRX is used	4965

A.14.5.3.2.1	Test Purpose and Environment	4965

A.14.5.3.2.2	Test parameters	4965

A.14.5.3.2.3	Test Requirements	4969

A.14.5.3.3	CSI-RS based L1-RSRP measurement for satellite access when DRX is not used	4969

A.14.5.3.3.1	Test Purpose and Environment	4969

A.14.5.3.3.2	Test parameters	4969

A.14.5.3.3.3	Test Requirements	4973

A.14.5.3.4	CSI-RS based L1-RSRP measurement for satellite access when DRX is used	4973

A.14.5.3.4.1	Test Purpose and Environment	4973

A.14.5.3.4.2	Test parameters	4974

A.14.5.3.4.3	Test Requirements	4977

A.14.6	Measurement Performance requirements	4977

A.14.6.1	 SS-RSRP for SAN	4977

A.14.6.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	4977

A.14.6.1.1.1	Test Purpose and Environment	4977

A.14.6.1.1.2	Test parameters	4977

A.14.6.1.1.3	Test Requirements	4981

A.14.6.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	4981

A.14.6.1.2.1	Test Purpose and Environment	4981

A.14.6.1.2.2	Test parameters	4981

A.14.6.1.2.3	Test Requirements	4984

A.14.6.2	SS-RSRQ	4984

A.14.6.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for satellite access	4984

A.14.6.2.1.1	Test Purpose and Environment	4984

A.14.6.2.1.2	Test Parameters	4984

A.14.6.2.1.3	Test Requirements	4987

A.14.6.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for satellite access	4987

A.14.6.2.2.1	Test Purpose and Environment	4987

A.14.6.2.2.2	Test Parameters	4987

A.14.6.2.2.3	Test Requirements	4990

A.14.6.3	SS-SINR	4990

A.14.6.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	4990

A.14.6.3.1.1	Test Purpose and Environment	4990

A.14.6.3.1.2	Test Parameters	4990

A.14.6.3.1.3	Test Requirements	4994

A.14.6.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	4994

A.14.6.3.2.1	Test Purpose and Environment	4994

A.14.6.3.2.2	Test Parameters	4994

A.14.6.3.2.3	Test Requirements	4998

A.14.6.4	L1-RSRP measurement for beam reporting	4998

A.14.6.4.1	SSB based L1-RSRP measurement	4998

A.14.6.4.1.1	Test Purpose and Environment	4998

A.14.6.4.1.2	Test parameters	4998

A.14.6.4.1.3	Test Requirements	5001

A.14.6.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	5002

A.14.6.4.2.1	Test Purpose and Environment	5002

A.14.6.4.2.2	Test parameters	5002

A.14.6.4.2.3	Test Requirements	5005

A.15	NR standalone tests with one or more NR cells in FR2-2	5006

A.15.1	SA: RRC_IDLE state mobility	5006

A.15.1.1	Cell re-selection to NR	5006

A.15.1.1.1	Cell reselection to FR2-2 intra-frequency NR case	5006

A.15.1.1.1.1	Test Purpose and Environment	5006

A.15.1.1.1.2	Test Parameters	5006

A.15.1.1.1.3	Test Requirements	5010

A.15.1.2	Cell reselection to FR2-2 inter-frequency NR case	5010

A.15.1.2.1	Test Purpose and Environment	5010

A.15.1.2.2	Test Parameters	5010

A.15.1.2.3	Test Requirements	5014

A.15.1.3	Cell reselection to FR2-2 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	5014

A.15.1.3.1	Test Purpose and Environment	5014

A.15.1.3.2	Test Parameters	5014

A.15.1.3.3	Test Requirements	5017

A.15.1.4	Cell reselection to FR2-2 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	5017

A.15.1.4.1	Test Purpose and Environment	5017

A.15.1.4.2	Test Parameters	5017

A.15.1.4.3	Test Requirements	5020

A.15.1.5	Cell reselection to FR2-2 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	5020

A.15.1.5.1	Test Purpose and Environment	5020

A.15.1.5.2	Test Parameters	5020

A.15.1.5.3	Test Requirements	5024

A.15.1.6	Cell reselection to FR2-2 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	5024

A.15.1.6.1	Test Purpose and Environment	5024

A.15.1.6.2	Test Parameters	5024

A.15.1.6.3	Test Requirements	5027

A.15.2	Signaling characteristics	5027

A.15.2.1	SCell Activation and Deactivation Delay	5027

A.15.2.1.1	SCell Activation and deactivation for SCell in FR2-2 intra-band in non-DRX	5027

A.15.2.1.1.1	Test Purpose and Environment	5027

A.15.2.1.1.2	Test Requirements	5030

A.15.2.1.2	SCell Activation and deactivation for FR1+FR2-2 inter-band with target SCell in FR2-2	5030

A.15.2.1.2.1	Test Purpose and Environment	5030

A.15.2.1.2.2	Test Requirements	5035

A.15.2.1.3	SCell Activation and deactivation for SCell in FR2-2 inter-band in non-DRX	5036

A.15.2.1.3.1	Test Purpose and Environment	5036

A.15.2.1.3.2	Test Requirements	5039

A.15.2.1.4	Direct SCell activation at SCell addition of known SCell in FR2-2	5040

A.15.2.1.4.1	Test Purpose and Environment	5040

A.15.2.1.4.2	Test Requirements	5043

A.15.2.1.5	Direct SCell activation at handover with known SCell in FR2-2	5044

A.15.2.1.5.1	Test Purpose and Environment	5044

A.15.2.1.5.2	Test Requirements	5048

A.15.3	RRC_CONNECTED state mobility	5049

A.15.3.1	Handover	5049

A.15.3.1.1	Intra-frequency handover from FR2-2 carrier with CCA to FR2-2 carrier with CCA; unknown target cell	5049

A.15.3.1.1.1	Test Purpose and Environment	5049

A.15.3.1.1.2	Test Parameters	5049

A.15.3.1.1.3	Test Requirements	5053

A.15.3.1.2	Inter-frequency handover from FR1 to FR2-2 carrier with CCA; unknown target cell	5053

A.15.3.1.2.1	Test Purpose and Environment	5053

A.15.3.1.2.2	Test Parameters	5053

A.15.3.1.2.3	Test Requirements	5057

A.15.4	Measurement procedure	5058

A.15.4.1	Intra-frequency Measurements	5058

A.15.4.1.1	SA event triggered reporting test without gap under non-DRX for FR2-2 with CCA	5058

A.15.4.1.1.1	Test purpose and Environment	5058

A.15.4.1.1.2	Test Requirements	5060

A.15.4.2	Inter-frequency Measurements	5061

A.15.4.2.1	SA event triggered reporting tests for FR2-2 with CCA without SSB time index detection when DRX is not used (PCell in FR2-2)	5061

A.15.4.2.1.1	Test Purpose and Environment	5061

A.15.4.2.1.2	Test Requirements	5064

A.16	NR standalone tests with all NR cells in FR1 for RedCap	5065

A.16.1	SA: RRC_IDLE state mobility for RedCap	5065

A.16.1.1	Cell re-selection to NR	5065

A.16.1.1.1	Cell reselection to FR1 intra-frequency NR case for 1 Rx UE	5065

A.16.1.1.1.1	Test Purpose and Environment	5065

A.16.1.1.1.2	Test Parameters	5065

A.16.1.1.1.3	Test Requirements	5069

A.16.1.1.2	Cell reselection to FR1 intra-frequency NR case for 2 Rx UE	5069

A.16.1.1.2.1	Test Purpose and Environment	5069

A.16.1.1.2.2	Test Parameters	5069

A.16.1.1.2.3	Test Requirements	5073

A.16.1.1.3	Cell reselection to FR1 inter-frequency NR case for 1 Rx UE	5073

A.16.1.1.3.1	Test Purpose and Environment	5073

A.16.1.1.3.2	Test Parameters	5073

A.16.1.1.3.3	Test Requirements	5078

A.16.1.1.4	Cell reselection to FR1 inter-frequency NR case for 2 Rx UE	5078

A.16.1.1.4.1	Test Purpose and Environment	5078

A.16.1.1.4.2	Test Parameters	5078

A.16.1.1.4.3	Test Requirements	5083

A.16.1.1.5	Cell reselection to FR1 intra-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	5083

A.16.1.1.5.1	Test Purpose and Environment	5083

A.16.1.1.5.2	Test Parameters	5083

A.16.1.1.5.3	Test Requirements	5088

A.16.1.1.6	Cell reselection to FR1 intra-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5088

A.16.1.1.6.1	Test Purpose and Environment	5088

A.16.1.1.6.2	Test Parameters	5088

A.16.1.1.6.3	Test Requirements	5093

A.16.1.1.7	Cell reselection to FR1 inter-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	5093

A.16.1.1.7.1	Test Purpose and Environment	5093

A.16.1.1.7.2	Test Parameters	5093

A.16.1.1.7.3	Test Requirements	5098

A.16.1.1.8	Cell reselection to FR1 inter-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5098

A.16.1.1.8.1	Test Purpose and Environment	5098

A.16.1.1.8.2	Test Parameters	5099

A.16.1.1.8.3	Test Requirements	5103

A.16.1.2	Inter-RAT E-UTRAN cell re-selection for RedCap	5104

A.16.1.2.1	Cell reselection to higher priority E-UTRAN for 1RX	5104

A.16.1.2.1.1	Test Purpose and Environment	5104

A.16.1.2.1.2	Test Parameters	5104

A.16.1.2.1.3	Test Requirements	5107

A.16.1.2.2	Cell reselection to higher priority E-UTRAN for 2RX	5108

A.16.1.2.2.1	Test Purpose and Environment	5108

A.16.1.2.2.2	Test Parameters	5108

A.16.1.2.2.3	Test Requirements	5111

A.16.1.2.3.1	Test Purpose and Environment	5112

A. 16.1.2.3.2	Test Parameters	5112

A.16.1.2.3.3	Test Requirements	5115

A.16.1.2.4.1	Test Purpose and Environment	5116

A.16.1.2.4.2	Test Parameters	5116

A.16.1.3.1.3	Test Requirements	5119

A.16.1.2.5	Cell reselection to lower priority E-UTRAN for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	5120

A.16.1.2.5.1	Test Purpose and Environment	5120

A.16.1.2.5.2	Test Parameters	5120

A.16.1.2.5.3	Test Requirements	5123

A.16.1.2.6	Cell reselection to lower priority E-UTRAN for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5124

A.16.1.2.6.1	Test Purpose and Environment	5124

A.16.1.2.6.2	Test Parameters	5124

A.16.1.2.6.3	Test Requirements	5127

A.16.2	SA: RRC_INACTIVE state mobility for RedCap	5128

A.16.2.1	Configured Grant based Small Data Transmissions (CG-SDT) for RedCap	5128

A.16.2.1.1	NR UE CG-SDT Test in FR1 for 1Rx RedCap UE	5128

A.16.2.1.1.1	Test purpose and Environment	5128

A.16.2.1.1.2	Test Parameters	5130

A.16.2.1.1.3	Test requirements	5133

A.16.2.1.2	NR UE CG-SDT Test in FR1 for 2Rx RedCap UE	5133

A.16.2.1.2.1	Test purpose and Environment	5133

A.16.2.1.2.2	Test Parameters	5135

A.16.2.1.2.3	Test requirements	5138

A.16.3	RRC_CONNECTED state mobility for RedCap	5138

A.16.3.1	Handover	5138

A.16.3.1.1	Intra-frequency handover from FR1 to FR1; known target cell for 1 Rx UE	5138

A.16.3.1.1.1	Test Purpose and Environment	5138

A.16.3.1.1.2	Test Parameters	5139

A.16.3.1.1.3	Test Requirements	5142

A.16.3.1.2	Intra-frequency handover from FR1 to FR1; known target cell for 2 Rx UE	5142

A.16.3.1.2.1	Test Purpose and Environment	5142

A.16.3.1.2.2	Test Parameters	5142

A.16.3.1.2.3	Test Requirements	5146

A.16.3.1.3	Intra-frequency handover from FR1 to FR1; unknown target cell for 1 Rx UE	5146

A.16.3.1.3.1	Test Purpose and Environment	5146

A.16.3.1.3.2	Test Parameters	5146

A.16.3.1.3.3	Test Requirements	5150

A.16.3.1.4	Intra-frequency handover from FR1 to FR1; unknown target cell for 2 Rx UE	5150

A.16.3.1.4.1	Test Purpose and Environment	5150

A.16.3.1.4.2	Test Parameters	5150

A.16.3.1.5	Inter-frequency handover from FR1 to FR1; unknown target cell for 1 Rx UE	5154

A.16.3.1.5.1	Test Purpose and Environment	5154

A.16.3.1.5.2	Test Parameters	5154

A.16.3.1.5.3	Test Requirements	5158

A.16.3.1.6	Inter-frequency handover from FR1 to FR1; unknown target cell for 2 Rx UE	5158

A.16.3.1.6.1	Test Purpose and Environment	5158

A.16.3.1.6.2	Test Parameters	5158

A.16.3.1.6.3	Test Requirements	5162

A.16.3.1.7	SA NR - E-UTRAN handover for 1Rx UE	5162

A.16.3.1.7.1	Test Purpose and Environment	5162

A.16.3.1.7.2	Test Requirements	5168

A.16.3.1.8	 SA NR - E-UTRAN handover for 2Rx UE	5168

A.16.3.1.8.1	Test Purpose and Environment	5168

A.16.3.1.8.2	Test Requirements	5174

A.16.3.1.9	SA NR - E-UTRAN handover with unknown target cell for 1 Rx UE	5174

A.16.3.1.9.1	Test Purpose and Environment	5174

A.16.3.1.9.2	Test Requirements	5181

A.16.3.1.10	SA NR - E-UTRAN handover with unknown target cell for 2 Rx UE	5181

A.16.3.1.10.1	Test Purpose and Environment	5181

A.16.3.1.10.2	Test Requirements	5187

A.16.3.2	RRC Connection Mobility Control	5187

A.16.3.2.1	SA: RRC Re-establishment	5187

A.16.3.2.1.1	Intra-frequency RRC Re-establishment in FR1 for 1 Rx UE	5187

A.16.3.2.1.2	Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE	5191

A.16.3.2.1.3	Inter-frequency RRC Re-establishment in FR1 for 1 Rx UE	5195

A.16.3.2.1.4	Inter-frequency RRC Re-establishment in FR1 for 2 Rx UE	5200

A.16.3.2.1.5	Intra-frequency RRC Re-establishment in FR1 for 1 Rx UE without serving cell timing	5205

A.16.3.2.1.6	Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE without serving cell timing	5209

A.16.3.2.2	Random Access	5213

A.16.3.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE	5213

A.16.3.2.2.2	4-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE	5219

A.16.3.2.2.3	4-step RA type non-contention based random access test in FR1 for NR standalone for 1 Rx UE	5224

A.16.3.2.2.5	2-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE	5234

A.16.3.2.2.6	2-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE	5238

A.16.3.2.2.7	2-step RA type non-contention based test in FR1 for NR standalone for 1 RX UE	5242

A.16.3.2.2.8	2-step RA type non-contention based test in FR1 for NR standalone for 2 RX UE	5246

A.16.3.2.3	SA: RRC Connection Release with Redirection	5250

A.16.3.2.3.1	Redirection from NR in FR1 to NR in FR1 for 1 Rx UE	5250

A.16.3.2.3.2	Redirection from NR in FR1 to NR in FR1 for 2 Rx UE	5254

A.16.3.2.3.3	Redirection from NR in FR1 to E-UTRAN for 1 Rx UE	5258

A.16.3.2.3.4	Redirection from NR in FR1 to E-UTRAN for 2 Rx UE	5265

A.16.4	Timing for RedCap	5272

A.16.4.1	UE transmit timing	5272

A.16.4.1.1	NR UE Transmit Timing Test for FR1 for 1Rx RedCap UE	5272

A.16.4.1.1.1	Test Purpose and environment	5272

A.16.4.1.1.2	Test requirements	5275

A.16.4.1.2	NR UE Transmit Timing Test for FR1 for 2Rx RedCap UE	5276

A.16.4.1.2.1	Test Purpose and environment	5276

A.16.4.1.2.2	Test requirements	5278

A.16.4.2	UE timer accuracy	5278

A.16.4.3	Timing advance	5278

A.16.4.3.1	SA FR1 timing advance adjustment accuracy for 1 Rx UE	5278

A.16.4.3.1.1	Test Purpose and Environment	5278

A.16.4.3.1.2	Test Parameters	5278

A.16.4.3.1.3	Test Requirements	5282

A.16.4.3.2	SA FR1 timing advance adjustment accuracy for 2 Rx UE	5282

A.16.4.3.2.1	Test Purpose and Environment	5282

A.16.4.3.2.2	Test Parameters	5283

A.16.4.3.2.3	Test Requirements	5286

A.16.5	Signalling characteristics for RedCap	5287

A.16.5.1	Radio link Monitoring	5287

A.16.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE	5287

A.16.5.1.1.1	Test Purpose and Environment	5287

A.16.5.1.1.2	Test Requirements	5292

A.16.5.1.2	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE	5292

A.16.5.1.2.1	Test Purpose and Environment	5292

A.16.5.1.2.2	Test Requirements	5298

A.16.5.1.3	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE	5298

A.16.5.1.3.1	Test Purpose and Environment	5298

A.16.5.1.3.2	Test Requirements	5304

A.16.5.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE	5304

A.16.5.1.4.1	Test Purpose and Environment	5304

A.16.5.1.4.2	Test Requirements	5310

A.16.5.1.5	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE	5310

A.16.5.1.5.1	Test Purpose and Environment	5310

A.16.5.1.5.2	Test Requirements	5316

A.16.5.1.6	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE	5316

A.16.5.1.6.1	Test Purpose and Environment	5316

A.16.5.1.6.2	Test Requirements	5321

A.16.5.1.7	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE	5321

A.16.5.1.7.1	Test Purpose and Environment	5321

A.16.5.1.7.2	Test Requirements	5326

A.16.5.1.8	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE	5326

A.16.5.1.8.1	Test Purpose and Environment	5326

A.16.5.1.8.2	Test Requirements	5332

A.16.5.1.9	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE	5332

A.16.5.1.9.1	Test Purpose and Environment	5332

A.16.5.1.9.2	Test Requirements	5338

A.16.5.1.10	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE	5338

A.16.5.1.10.1	Test Purpose and Environment	5338

A.16.5.1.10.2	Test Requirements	5344

A.16.5.1.11	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE	5344

A.16.5.1.11.1	Test Purpose and Environment	5344

A.16.5.1.11.2	Test Requirements	5350

A.16.5.1.12	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE	5350

A.16.5.1.12.1	Test Purpose and Environment	5350

A.16.5.1.12.2	Test Requirements	5356

A.16.5.1.13	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE	5356

A.16.5.1.13.1	Test Purpose and Environment	5356

A.16.5.1.13.2	Test Requirements	5361

A.16.5.1.14	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE	5361

A.16.5.1.14.1	Test Purpose and Environment	5361

A.16.5.1.14.2	Test Requirements	5367

A.16.5.1.15	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE	5367

A.16.5.1.15.1	Test Purpose and Environment	5367

A.16.5.1.15.2	Test Requirements	5373

A.16.5.1.16	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE	5373

A.16.5.1.16.1	Test Purpose and Environment	5373

A.16.5.1.16.2	Test Requirements	5379

A.16.5.2	Beam Failure Detection and Link recovery procedures	5379

A.16.5.2.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 1 Rx UE	5379

A.16.5.2.1.1	Test Purpose and Environment	5379

A.16.5.2.1.2	Test Requirements	5385

A.16.5.2.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE	5385

A.16.5.2.2.1	Test Purpose and Environment	5385

A.16.5.2.2.2	Test Requirements	5391

A.16.5.2.3	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode for 1 Rx UE	5391

A.16.5.2.3.1	Test Purpose and Environment	5391

A.16.5.2.3.2	Test Requirements	5398

A.16.5.2.4	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode for 2 Rx UE	5398

A.16.5.2.4.1	Test Purpose and Environment	5398

A.16.5.2.4.2	Test Requirements	5404

A.16.5.2.5	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 1 Rx UE	5404

A.16.5.2.5.1	Test Purpose and Environment	5404

A.16.5.2.5.2	Test Requirements	5410

A.16.5.2.6	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 2 Rx UE	5410

A.16.5.2.6.1	Test Purpose and Environment	5410

A.16.5.2.6.2	Test Requirements	5416

A.16.5.2.7	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 1 Rx UE	5416

A.16.5.2.7.1	Test Purpose and Environment	5416

A.16.5.2.7.2	Test Requirements	5422

A.16.5.2.8	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 2 Rx UE	5422

A.16.5.2.8.1	Test Purpose and Environment	5422

A.16.5.2.8.2	Test Requirements	5428

A.16.5.3	Active BWP switch	5428

A.16.5.3.1	DCI-based and Timer-based Active BWP Switch	5428

A.16.5.3.1.1	NR FR1 DL active BWP switch with non-DRX in SA for 1 Rx UE	5428

A.16.5.3.1.1.1	Test Purpose and Environment	5428

A.16.5.3.1.2.1	Test Requirements	5433

A.16.5.3.1.2	NR FR1 DL active BWP switch with non-DRX in SA for 2 Rx UE	5433

A.16.5.3.1.2.1	Test Purpose and Environment	5433

A.16.5.3.1.2.2	Test Requirements	5438

A.16.5.3.2	RRC-based Active BWP Switch	5438

A.16.5.3.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA for 1 Rx UE	5438

A.16.5.3.2.1.1	Test Purpose and Environment	5438

A.16.5.3.2.1.2	Test Requirements	5442

A.16.5.3.2.2	NR FR1 DL active BWP switch of Cell with non-DRX in SA for 2 Rx UE	5442

A.16.5.3.2.2.1	Test Purpose and Environment	5442

A.16.5.3.2.2.2	Test Requirements	5446

A.16.5.4	UE specific CBW change	5446

A.16.5.4.1	UE specific CBW change on PCell in FR1 in non-DRX for 1 Rx UE	5446

A.16.5.4.1.1	Test Purpose and Environment	5446

A.16.5.4.1.2	Test Requirements	5450

A.16.5.4.2	UE specific CBW change on PCell in FR1 in non-DRX for 2 Rx UE	5450

A.16.5.4.2.1	Test Purpose and Environment	5450

A.16.5.4.2.2	Test Requirements	5454

A.16.6	Measurement procedure for RedCap	5454

A.16.6.1	Intra-frequency Measurements	5454

A.16.6.1.1	SA event triggered reporting tests without gap under non-DRX for 1 Rx UE	5454

A.16.6.1.1.1	Test purpose and Environment	5454

A.16.6.1.1.2	Test parameters	5454

A.16.6.1.1.3	Test Requirements	5458

A.16.6.1.2	SA event triggered reporting tests without gap under non-DRX for 2 Rx UE	5458

A.16.6.1.2.1	Test purpose and Environment	5458

A.16.6.1.2.2	Test parameters	5458

A.16.6.1.2.3	Test Requirements	5462

A.16.6.1.3	SA event triggered reporting tests without gap under DRX for 1 Rx UE	5462

A.16.6.1.3.1	Test purpose and Environment	5462

A.16.6.1.3.2	Test parameters	5462

A.16.6.1.3.3	Test Requirements	5466

A.16.6.1.4	SA event triggered reporting tests without gap under DRX for 2 Rx UE	5466

A.16.6.1.4.1	Test purpose and Environment	5466

A.16.6.1.4.2	Test parameters	5466

A.16.6.1.4.3	Test Requirements	5470

A.16.6.1.5	SA event triggered reporting tests with per-UE gaps under non-DRX for 1 Rx UE	5470

A.16.6.1.5.1	Test purpose and Environment	5470

A.16.6.1.5.2	Test parameters	5470

A.16.6.1.5.3	Test Requirements	5474

A.16.6.1.6	SA event triggered reporting tests with per-UE gaps under non-DRX for 2 Rx UE	5474

A.16.6.1.6.1	Test purpose and Environment	5474

A.16.6.1.6.2	Test parameters	5474

A.16.6.1.6.3	Test Requirements	5478

A.16.6.1.7	SA event triggered reporting tests with per-UE gaps under DRX for 1 Rx UE	5478

A.16.6.1.7.1	Test purpose and Environment	5478

A.16.6.1.7.2	Test parameters	5478

A.16.6.1.7.3	Test Requirements	5482

A.16.6.1.8	SA event triggered reporting tests with per-UE gaps under DRX for 2 Rx UE	5482

A.16.6.1.8.1	Test purpose and Environment	5482

A.16.6.1.8.2	Test parameters	5482

A.16.6.1.8.3	Test Requirements	5486

A.16.6.1.9	SA event triggered reporting tests without gap under non-DRX with SSB index reading for 1 Rx UE	5486

A.16.6.1.9.1	Test purpose and Environment	5486

A.16.6.1.9.2	Test parameters	5486

A.16.6.1.9.3	Test Requirements	5488

A.16.6.1.10	SA event triggered reporting tests without gap under non-DRX with SSB index reading for 2 Rx UE	5489

A.16.6.1.10.1	Test purpose and Environment	5489

A.16.6.1.10.2	Test parameters	5489

A.16.6.1.10.3	Test Requirements	5490

A.16.6.1.11	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 1 Rx UE	5491

A.16.6.1.11.1	Test purpose and Environment	5491

A.16.6.1.11.2	Test parameters	5491

A.16.6.1.11.3	Test Requirements	5493

A.16.6.1.12	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 2 Rx UE	5494

A.16.6.1.12.1	Test purpose and Environment	5494

A.16.6.1.12.2	Test parameters	5494

A.16.6.1.12.3	Test Requirements	5496

A.16.6.2	Inter-frequency Measurements	5497

A.16.6.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 1 Rx UE	5497

A.16.6.2.1.1	Test Purpose and Environment	5497

A.16.6.2.1.2	Test Requirements	5502

A.16.6.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 2 Rx UE	5502

A.16.6.2.2.1	Test Purpose and Environment	5502

A.16.6.2.2.2	Test Requirements	5507

A.16.6.2.3	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used for 1 Rx UE	5507

A.16.6.2.3.1	Test Purpose and Environment	5507

A.16.6.2.3.2	Test Requirements	5510

A.16.6.2.4	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used for 2 Rx UE	5511

A.16.6.2.4.1	Test Purpose and Environment	5511

A.16.6.2.4.2	Test Requirements	5514

A.16.6.2.5	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 1 Rx UE	5515

A.16.6.2.5.1	Test Purpose and Environment	5515

A.16.6.2.5.2	Test Requirements	5518

A.16.6.2.6	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 2 Rx UE	5519

A.16.6.2.6.1	Test Purpose and Environment	5519

A.16.6.2.6.2	Test Requirements	5522

A.16.6.2.7	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 1 Rx UE	5523

A.16.6.2.7.1	Test Purpose and Environment	5523

A.16.6.2.7.2	Test Requirements	5527

A.16.6.2.8	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 2 Rx UE	5528

A.16.6.2.8.1	Test Purpose and Environment	5528

A.16.6.2.8.2	Test Requirements	5531

A.16.6.2.9	SA event triggered reporting tests with additional mandatory gap pattern for 1 Rx UE	5532

A.16.6.2.9.1	Test Purpose and Environment	5532

A.16.6.2.9.2	Test Requirements	5535

A.16.6.2.10	SA event triggered reporting tests with additional mandatory gap pattern for 2 Rx UE	5535

A.16.6.2.10.1	Test Purpose and Environment	5535

A.16.6.2.10.2	Test Requirements	5538

A.16.6.2.11	SA event triggered reporting tests for FR1 when DRX is used for 1 Rx UE	5538

A.16.6.2.11.1	Test Purpose and Environment	5538

A.16.6.2.11.2	Test Requirements	5541

A.16.6.2.12	SA event triggered reporting tests for FR1 when DRX is used for 2 Rx UE	5541

A.16.6.2.12.1	Test Purpose and Environment	5541

A.16.6.2.12.2	Test Requirements	5545

A.16.6.3	Inter-RAT Measurements	5546

A.16.6.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 for 1 Rx UE	5546

A.16.6.3.1.1	Test purpose and Environment	5546

A.16.6.3.1.2	Test Requirements	5551

A.16.6.3.2	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 for 2 Rx UE	5551

A.16.6.3.2.1	Test purpose and Environment	5551

A.16.6.3.2.2	Test Requirements	5556

A.16.6.3.3	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for 1 Rx UE	5556

A.16.6.3.3.1	Test purpose and Environment	5556

A.16.6.3.3.2	Test Requirements	5561

A.16.6.3.4	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for 2 Rx UE	5561

A.16.6.3.4.1	Test purpose and Environment	5561

A.16.6.3.4.2	Test Requirements	5566

A.16.6.4	L1-RSRP measurement for beam reporting	5566

A.16.6.4.1	SSB based L1-RSRP measurement when DRX is not used for 1 Rx UE	5566

A.16.6.4.1.1	Test Purpose and Environment	5566

A.16.6.4.1.2	Test parameters	5566

A.16.6.4.1.3	Test Requirements	5569

A.16.6.4.2	SSB based L1-RSRP measurement when DRX is not used for 2 Rx UE	5569

A.16.6.4.2.1	Test Purpose and Environment	5569

A.16.6.4.2.2	Test parameters	5570

A.16.6.4.2.3	Test Requirements	5573

A.16.6.4.3	SSB based L1-RSRP measurement when DRX is used for 1 Rx UE	5573

A.16.6.4.3.1	Test Purpose and Environment	5573

A.16.6.4.3.2	Test parameters	5574

A.16.6.4.3.3	Test Requirements	5577

A.16.6.4.4	SSB based L1-RSRP measurement when DRX is used for 2 Rx UE	5577

A.16.6.4.4.1	Test Purpose and Environment	5577

A.16.6.4.4.2	Test parameters	5578

A.16.6.4.4.3	Test Requirements	5581

A.16.6.4.5	CSI-RS based L1-RSRP measurement when DRX is not used for 1 Rx UE	5581

A.16.6.4.5.1	Test Purpose and Environment	5581

A.16.6.4.5.2	Test parameters	5582

A.16.6.4.5.3	Test Requirements	5585

A.16.6.4.6	CSI-RS based L1-RSRP measurement when DRX is not used for 2 Rx UE	5585

A.16.6.4.6.1	Test Purpose and Environment	5585

A.16.6.4.6.2	Test parameters	5586

A.16.6.4.6.3	Test Requirements	5589

A.16.6.4.7	CSI-RS based L1-RSRP measurement when DRX is used for 1 Rx UE	5589

A.16.6.4.7.1	Test Purpose and Environment	5589

A.16.6.4.7.2	Test parameters	5590

A.16.6.4.7.3	Test Requirements	5593

A.16.6.4.8	CSI-RS based L1-RSRP measurement when DRX is used for 2 Rx UE	5593

A.16.6.4.8.1	Test Purpose and Environment	5593

A.16.6.4.8.2	Test parameters	5594

A.16.6.4.8.3	Test Requirements	5597

A.16.6.5	NR measurements with autonomous gaps	5597

A.16.6.5.1	SA intra-frequency CGI identification of NR neighbor cell in FR1 for 1 Rx UE	5597

A.16.6.5.1.1	Test Purpose and Environment	5597

A.16.6.5.1.2	Test Parameters	5597

A.16.6.5.1.3	Test Requirements	5600

A.16.6.5.2	SA intra-frequency CGI identification of NR neighbor cell in FR1 for 2 Rx UE	5600

A.16.6.5.2.1	Test Purpose and Environment	5600

A.16.6.5.2.2	Test Parameters	5600

A.16.6.5.2.3	Test Requirements	5603

A.16.6.5.3	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA for 1 Rx UE	5603

A.16.6.5.3.1	Test Purpose and Environment	5603

A.16.6.5.3.2	Test Requirements	5608

A.16.6.5.4	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA for 2 Rx UE	5609

A.16.6.5.4.1	Test Purpose and Environment	5609

A.16.6.5.4.2	Test Requirements	5614

A.16.7	Measurement Performance requirements for RedCap	5615

A.16.7.1	SS-RSRP	5615

A.16.7.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5615

A.16.7.1.1.1	Test Purpose and Environment	5615

A.16.7.1.1.2	Test parameters	5615

A.16.7.1.1.3	Test Requirements	5620

A.16.7.1.2	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 2Rx UE	5620

A.16.7.1.2.1	Test Purpose and Environment	5620

A.16.7.1.2.2	Test parameters	5620

A.16.7.1.2.3	Test Requirements	5625

A.16.7.1.3	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5625

A.16.7.1.3.1	Test Purpose and Environment	5625

A.16.7.1.3.2	Test parameters	5625

A.16.7.1.3.3	Test Requirements	5629

A.16.7.1.4	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5629

A.16.7.1.4.1	Test Purpose and Environment	5629

A.16.7.1.4.2	Test parameters	5630

A.16.7.1.4.3	Test Requirements	5634

A.16.7.2	SS-RSRQ	5634

A.16.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5634

A.16.7.2.1.1	Test Purpose and Environment	5634

A.16.7.2.1.2	Test Parameters	5635

A.16.7.2.1.3	Test Requirements	5640

A.16.7.2.2	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5640

A.16.7.2.2.1	Test Purpose and Environment	5640

A.16.7.2.2.2	Test Parameters	5640

A.16.7.2.2.3	Test Requirements	5645

A.16.7.2.3	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5645

A.16.7.2.3.1	Test Purpose and Environment	5645

A.16.7.2.3.2	Test parameters	5645

A.16.7.2.3.3	Test Requirements	5650

A.16.7.2.4	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5650

A.16.7.2.4.1	Test Purpose and Environment	5650

A.16.7.2.4.2	Test parameters	5650

A.16.7.2.4.3	Test Requirements	5656

A.16.7.3	SS-SINR	5656

A.16.7.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5656

A.16.7.3.1.1	Test Purpose and Environment	5656

A.16.7.3.1.2	Test parameters	5657

A.16.7.3.1.3	Test Requirements	5661

A.16.7.3.2	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5661

A.16.7.3.2.1	Test Purpose and Environment	5661

A.16.7.3.2.2	Test parameters	5662

A.16.7.3.3	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5666

A.16.7.3.3.1	Test Purpose and Environment	5666

A.16.7.3.3.2	Test parameters	5666

A.16.7.3.3.3	Test Requirements	5671

A.16.7.3.4	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5672

A.16.7.3.4.1	Test Purpose and Environment	5672

A.16.7.3.4.2	Test parameters	5672

A.16.7.3.4.3	Test Requirements	5676

A.16.7.4	L1-RSRP measurement for beam reporting	5677

A.16.7.4.1	SSB based L1-RSRP measurement for 1 Rx UE	5677

A.16.7.4.1.1	Test Purpose and Environment	5677

A.16.7.4.1.2	Test parameters	5677

A.16.7.4.1.3	Test Requirements	5681

A.16.7.4.2	SSB based L1-RSRP measurement for 2 Rx UE	5681

A.16.7.4.2.1	Test Purpose and Environment	5681

A.16.7.4.2.2	Test parameters	5681

A.16.7.4.2.3	Test Requirements	5682

A.16.7.4.3	CSI-RS based L1-RSRP measurement on resource set with repetition off for 1 Rx UE	5682

A.16.7.4.3.1	Test Purpose and Environment	5682

A.16.7.4.3.2	Test parameters	5682

A.16.7.4.3.3	Test Requirements	5686

A.16.7.4.4	CSI-RS based L1-RSRP measurement on resource set with repetition off for 2 Rx UE	5686

A.16.7.4.4.1	Test Purpose and Environment	5686

A.16.7.4.4.2	Test parameters	5687

A.16.7.4.4.3	Test Requirements	5687

A.16.7.5	E-UTRAN RSRP	5687

A.16.7.5.1	SA: inter-RAT measurement accuracy with FR1 serving cell for 1 Rx UE	5687

A.16.7.5.1.1	Test Purpose and Environment	5687

A.16.7.5.1.2	Test parameters	5687

A.16.7.5.1.3	Test Requirements	5693

A.16.7.5.2	SA: inter-RAT measurement accuracy with FR1 serving cell for 2 Rx UE	5694

A.16.7.5.2.1	Test Purpose and Environment	5694

A.16.7.5.2.2	Test parameters	5694

A.16.7.5.2.3	Test Requirements	5700

A.16.7.6	E-UTRAN RSRQ	5701

A.16.7.6.1	SA: inter-RAT measurement accuracy with FR1 serving cell for 1 Rx UE	5701

A.16.7.6.1.1	Test Purpose and Environment	5701

A.16.7.6.1.2	Test parameters	5701

A.16.7.6.1.3	Test Requirements	5707

A.16.7.6.2	SA: inter-RAT measurement accuracy with FR1 serving cell for 2 Rx UE	5708

A.16.7.6.2.1	Test Purpose and Environment	5708

A.16.7.6.2.2	Test parameters	5708

A.16.7.6.2.3	Test Requirements	5714

A.17	NR standalone tests with one or more NR cells in FR2 for RedCap	5720

A.17.1	SA: RRC_IDLE state mobility for RedCap	5720

A.17.1.1	Cell re-selection to NR	5720

A.17.1.1.1	Cell reselection to FR2 intra-frequency NR case for 2 Rx	5720

A.17.1.1.1.1	Test Purpose and Environment	5720

A.17.1.1.1.2	Test Parameters	5720

A.17.1.1.1.3	Test Requirements	5724

A.17.1.1.2	Cell reselection to FR2 inter-frequency NR case	5724

A.17.1.1.2.1	Test Purpose and Environment	5724

A.17.1.1.2.2	Test Parameters	5724

A.17.1.1.2.3	Test Requirements	5728

A.17.1.1.3	Cell reselection to FR2 intra-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5728

A.17.1.1.3.1	Test Purpose and Environment	5728

A.17.1.1.3.2	Test Parameters	5728

A.17.1.1.3.3	Test Requirements	5731

A.17.1.1.4	Cell reselection to FR2 inter-frequency NR case for UE fulfilling stationary mobility relaxed measurement criterion for 2 Rx UE	5731

A.17.1.1.4.1	Test Purpose and Environment	5731

A.17.1.1.4.2	Test Parameters	5731

A.17.1.1.4.3	Test Requirements	5735

A.17.2	SA: RRC_INACTIVE state mobility for RedCap	5735

A.17.2.1	Configured Grant based Small Data Transmissions (CG-SDT) for RedCap	5735

A.17.2.1.1	TA validation for CG-SDT in FR2 for RedCap	5735

A.17.2.1.1.1	Test Purpose and Environment	5735

A.17.2.1.1.2	Test Requirements	5739

A.17.3	RRC_CONNECTED state mobility for RedCap	5740

A.17.3.1	Handover for RedCap	5740

A.17.3.1.1	Intra-frequency handover from FR2 to FR2; unknown target cell for 2 Rx	5740

A.17.3.1.1.1	Test Purpose and Environment	5740

A.17.3.1.1.2	Test Parameters	5740

A.17.3.1.1.3	Test Requirements	5742

A.17.3.1.2	Inter-frequency handover from FR2 to FR2; unknown target cell for 2 Rx	5742

A.17.3.1.2.1	Test Purpose and Environment	5742

A.17.3.1.2.2	Test Parameters	5742

A.17.3.1.2.3 Test Requirements	5745

A.17.3.2	RRC Connection Mobility Control for RedCap	5745

A.17.3.2.1	SA: RRC Re-establishment	5745

A.17.3.2.1.1	Intra-frequency RRC Re-establishment in FR2	5745

A.17.3.2.1.1.1	Test Purpose and Environment	5745

A.17.3.2.1.2	Inter-frequency RRC Re-establishment in FR2	5748

A.17.3.2.1.2.1	Test Purpose and Environment	5748

A.17.3.2.1.3	Intra-frequency RRC Re-establishment in FR2 without serving cell timing	5751

A.17.3.2.1.3.1	Test Purpose and Environment	5751

A.17.3.2.1.3.2	Test Requirements	5753

A.17.3.2.2	Random Access	5754

A.17.3.2.2.1	4-step RA type contention based random access test in FR2 for NR Standalone	5754

A.17.3.2.2.1.1	Test Purpose and Environment	5754

A.17.3.2.2.1.2	Test Requirements	5756

A.17.3.2.2.2	4-step RA type non-contention based random access test in FR2 for NR Standalone	5758

A.17.3.2.2.2.1	Test Purpose and Environment	5758

A.17.3.2.2.2.2	Test Requirements	5760

A.17.3.2.2.3	2-step RA type contention based random access test in FR2 for NR Standalone	5762

A.17.3.2.2.3.1	Test Purpose and Environment	5762

A.17.3.2.2.3.2	Test Requirements	5764

A.17.3.2.2.4	2-step RA type non-contention based random access test in FR2 for NR Standalone	5765

A.17.3.2.2.4.1	Test Purpose and Environment	5765

A.17.3.2.2.4.2	Test Requirements	5767

A.17.3.2.3	SA: RRC Connection Release with Redirection	5768

A.17.3.2.3.1	Redirection from NR in FR2 to NR in FR2	5768

A.17.3.2.3.1.1	Test Purpose and Environment	5768

A.17.3.2.3.1.2	Test Parameters	5768

A.17.3.2.3.1.3	Test Requirements	5771

A.17.4	Timing	5771

A.17.4.1	UE transmit timing	5771

A.17.4.1.1	NR UE Transmit Timing Test for FR2	5771

A.17.4.1.1.1	Test Purpose and environment	5771

A.17.4.1.1.2	Test requirements	5774

A.17.4.2	UE timer accuracy	5775

A.17.4.3	Timing advance	5775

A.17.4.3.1	SA FR2 timing advance adjustment accuracy	5775

A.17.4.3.1.1	Test Purpose and Environment	5775

A.17.4.3.1.2	Test Parameters	5775

A.17.4.3.1.3 Test Requirements	5778

A.17.5	Signaling characteristics for RedCap	5778

A.17.5.1	Radio link Monitoring for RedCap	5778

A.17.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	5778

A.17.5.1.1.1	Test Purpose and Environment	5778

A.17.5.1.1.2	Test Requirements	5781

A.17.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	5782

A.17.5.1.2.1	Test Purpose and Environment	5782

A.17.5.1.2.2	Test Requirements	5786

A.17.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	5787

A.17.5.1.3.1	Test Purpose and Environment	5787

A.17.5.1.3.2	Test Requirements	5791

A.17.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	5791

A.17.5.1.4.1	Test Purpose and Environment	5791

A.17.5.1.4.2	Test Requirements	5796

A.17.5.1.5	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	5796

A.17.5.1.5.1	Test Purpose and Environment	5796

A.17.5.1.5.2	Test Requirements	5801

A.17.5.1.6	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	5801

A.17.5.1.6.1	Test Purpose and Environment	5801

A.17.5.1.6.2	Test Requirements	5805

A.17.5.1.7	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	5805

A.17.5.1.7.1	Test Purpose and Environment	5805

A.17.5.1.7.2	Test Requirements	5809

A.17.5.1.8	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	5809

A.17.5.1.8.1	Test Purpose and Environment	5809

A.17.5.1.8.2	Test Requirements	5814

A.17.5.1.9	UE Radio Link Monitoring Scheduling Restrictions on FR2	5814

A.17.5.1.9.1	Test Purpose and Environment	5814

A.17.5.1.9.2	Test Requirements	5817

A.17.5.2	Beam Failure Detection and Link recovery procedures	5817

A.17.5.2.1	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	5817

A.17.5.2.1.1	Test Purpose and Environment	5817

A.17.5.2.1.2	Test Requirements	5821

A.17.5.2.2	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode	5821

A.17.5.2.2.1	Test Purpose and Environment	5821

A.17.5.2.2.2	Test Requirements	5825

A.17.5.2.3	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	5825

A.17.5.2.3.1	Test Purpose and Environment	5825

A.17.5.2.3.2	Test Requirements	5829

A.17.5.2.4	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode	5829

A.17.5.2.4.1	Test Purpose and Environment	5829

A.17.5.2.4.2	Test Requirements	5833

A.17.5.2.5	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE	5833

A.17.5.2.5.1	Test Purpose and Environment	5833

A.17.5.2.5.2	Test Requirements	5837

A.17.5.3	Active BWP switch for RedCap	5837

A.17.5.3.1	DCI-based and Timer-based Active BWP Switch	5837

A.17.5.3.1.1	NR FR2 DL active BWP switch with non-DRX in SA	5837

A.17.5.3.1.1.1	Test Purpose and Environment	5837

A.17.5.3.1.1.2	Test Requirements	5840

A.17.5.3.2	RRC-based Active BWP Switch	5840

A.17.5.3.2.1	NR FR2 DL active BWP switch of PCell with non-DRX in SA	5840

A.17.5.3.2.1.1	Test Purpose and Environment	5840

A.17.5.3.2.1.2	Test Requirements	5843

A.17.5.4	Active TCI state switch delay	5844

A.17.5.4.1	MAC-CE based active TCI state switch	5844

A.17.5.4.1.1	NR PCell FR2 active TCI state switch for a known TCI state	5844

A.17.5.4.1.1.1	Test Purpose and Environment	5844

A.17.5.4.1.1.2	Test Requirements	5847

A.17.5.4.2	RRC based active TCI state switch	5847

A.17.5.4.2.1	NR PCell FR2 active TCI state switch for a known TCI state	5847

A.17.5.4.2.1.1	Test Purpose and Environment	5847

A.17.5.4.2.1.2	Test Requirements	5851

A.17.5.5	Uplink spatial relation switch delay	5851

A.17.5.5.1	MAC-CE based Spatial Relation switch	5851

A.17.5.5.1.1	 NR PCell FR2 spatial relation associated with known DL-RS	5851

A.17.5.5.1.1.1	Test Purpose and Environment	5851

A.17.5.5.1.1.2	Test Requirements	5854

A.17.5.5.2	RRC based spatial relation switch	5854

A.17.5.5.2.1	NR PCell FR2 spatial relation switch associated with a known DL-RS	5854

A.17.5.5.2.1.2	Test Requirements	5857

A.17.5.6	UE specific CBW change	5857

A.17.5.6.1	NR FR2 UE specific CBW change of PCell with non-DRX in SA	5857

A.17.5.6.1.1	Test Purpose and Environment	5857

A.17.5.6.1.2	Test Requirements	5860

A.17.6	Measurement procedure for RedCap	5861

A.17.6.1	Intra-frequency Measurements	5861

A.17.6.1.1	SA event triggered reporting test without gap under non-DRX	5861

A.17.6.1.1.1	Test purpose and Environment	5861

A.17.6.1.1.2	Test Requirements	5864

A.17.6.1.2	SA event triggered reporting test without gap under DRX	5864

A.17.6.1.2.1	Test purpose and Environment	5864

A.7.6.1.2.2	Test Requirements	5864

A.17.6.1.3	SA event triggered reporting test with per-UE gaps under non-DRX	5865

A.17.6.1.3.1	Test purpose and Environment	5865

A.17.6.1.3.2	Test Requirements	5869

A.17.6.1.4	SA event triggered reporting test with per-UE gaps under DRX	5869

A.17.6.1.4.1	Test purpose and Environment	5869

A.17.6.1.4.2	Test Requirements	5872

A.17.6.2	Inter-frequency Measurements	5873

A.17.6.2.1	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2)	5873

A.17.6.2.1.1	Test Purpose and Environment	5873

A.17.6.2.1.2	Test Requirements	5876

A.17.6.2.2	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is used (PCell in FR2)	5876

A.17.6.2.2.1	Test Purpose and Environment	5876

A.17.6.2.2.2	Test Requirements	5880

A.17.6.2.3	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is not used (PCell in FR2)	5880

A.17.6.2.3.1	Test Purpose and Environment	5880

A.17.6.2.3.2	Test Requirements	5884

A.17.6.2.4	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is used (PCell in FR2) for 2 RX UE	5884

A.17.6.2.4.1	Test Purpose and Environment	5884

A.17.6.2.4.2	Test Requirements	5888

A.17.6.3	L1-RSRP measurement for beam reporting	5889

A.17.6.3.1	SSB based L1-RSRP measurement when DRX is not used	5889

A.17.6.3.1.1	Test Purpose and Environment	5889

A.17.6.3.1.2	Test parameters	5889

A.17.6.3.1.3	Test Requirements	5889

A.17.6.3.2	SSB based L1-RSRP measurement when DRX is used	5889

A.17.6.3.2.1	Test Purpose and Environment	5889

A.17.6.3.2.2	Test parameters	5889

A.17.6.3.2.3	Test Requirements	5891

A.17.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	5891

A.17.6.3.3.1	Test Purpose and Environment	5891

A.17.6.3.3.2	Test parameters	5892

A.17.6.3.3.3	Test Requirements	5894

A.17.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	5895

A.17.6.3.4.1	Test Purpose and Environment	5895

A.17.6.3.4.2	Test parameters	5895

A.7.6.3.3.3	Test Requirements	5897

A.17.6.4.1	SA interfrequency CGI reporting in autonomous gaps test (PCell in FR2) for 2 RX UE	5898

A.17.6.4.1.1	Test Purpose and Environment	5898

A.17.6.4.1.2	Test Requirements	5901

A.17.7	Measurement Performance requirements	5901

A.17.7.1	SS-RSRP	5901

A.17.7.1.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	5901

A.17.7.1.1.1	Test Purpose and Environment	5901

A.17.7.1.1.2	Test parameters	5901

A.17.7.1.1.3	Test Requirements	5905

A.17.7.1.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	5906

A.17.7.1.2.1	Test Purpose and Environment	5906

A.17.7.1.2.2	Test parameters	5906

A.17.7.1.2.3	Test Requirements	5910

A.17.7.2	SS-RSRQ	5911

A.17.7.2.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	5911

A.17.7.2.1.1	Test Purpose and Environment	5911

A.17.7.2.1.2	Test Parameters	5911

A.17.7.2.1.3	Test Requirements	5913

A.17.7.2.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell for 2 Rx UE	5913

A.17.7.2.2.1	Test Purpose and Environment	5913

A.17.7.2.2.2	Test parameters	5913

A.17.7.2.2.3	Test Requirements	5915

A.17.7.3	L1-RSRP measurement for beam reporting	5915

A.17.7.3.1	SSB based L1-RSRP measurement	5915

A.17.7.3.1.1	Test Purpose and Environment	5915

A.17.7.3.1.2	Test parameters	5916

A.17.7.3.1.3	Test Requirements	5916

A.17.7.3.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	5916

A.17.7.3.2.1	Test Purpose and Environment	5916

A.17.7.3.2.2	Test parameters	5916

A.17.7.3.2.3	Test Requirements	5916

A.17.7.4	SS-SINR	5917

A.17.7.4.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell for 2Rx UE	5917

A.17.7.4.1.1	Test Purpose and Environment	5917

A.17.7.4.1.2	Test parameters	5917

A.17.7.4.1.3	Test Requirements	5919

The SS-SINR absolute measurement accuracy in test 1 shall be within the range Nominal SS-SINR+3B to Nominal SS-SINR -3dB and the SS-SINR measurement accuracy in test 2 shall be within the range Nominal SS-SINR +3.5dB to Nominal SS-SINR -3.5dB according to the requirements in clause 10.1A.10.13.1. A.17.7.4.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell for 2Rx UE	5919

A.17.7.4.2.1	Test Purpose and Environment	5919

A.17.7.4.2.2	Test Parameters	5919

A.17.7.4.2.3	Test Requirements	5921

A.18	E-UTRA standalone tests for NR RRM for RedCap	5921

A.18.1	RRC_IDLE state mobility	5921

A.18.1.1	Inter-RAT NR Cell re-selection	5921

A.18.1.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1	5921

A.18.1.1.1.1	Test Purpose and Environment	5921

A.18.2.1.1.2	Test Requirements	5926

A.18.2	RRC_CONNECTED state mobility	5926

A.18.2.1	Handover	5926

A.18.2.1.1	E-UTRAN - NR handover in FR1	5926

A.18.2.1.1.1	Test Purpose and Environment	5926

A.18.2.1.1.2	Test Requirements	5931

A.18.2.2	RRC connection release with redirection	5931

A.18.2.2.1	Redirection from E-UTRA to NR FR1 for redcap UE	5931

A.18.2.2.1.1	Test Purpose and Environment	5931

A.18.2.2.1.2	Test Parameters	5931

A.18.2.2.1.3	Test Requirements	5938

A.18.3	Measurement procedure	5938

A.18.3.1	E-UTRA – NR Inter-RAT Measurements	5938

A.18.3.1.1	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	5938

A.18.3.1.1.1	Test Purpose and Environment	5938

A.18.3.1.1.2	Test Requirements	5944

A.18.3.1.2	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	5944

A.18.3.1.2.1	Test Purpose and Environment	5944

A.18.3.1.2.2	Test Requirements	5950

A.18.3.1.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	5950

A.18.3.1.3.1	Test Purpose and Environment	5950

A.18.3.1.3.2	Test Requirements	5956

A.18.3.1.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	5956

A.18.3.1.4.1	Test Purpose and Environment	5956

A.18.3.1.4.2	Test Requirements	5962

A.18.3.1.5	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is not used	5962

A.18.3.1.5.1	Test Purpose and Environment	5962

A.18.3.1.5.2	Test Requirements	5964

A.18.3.1.6	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is used	5965

A.18.3.1.6.1	Test Purpose and Environment	5965

A.18.3.1.6.2	Test Requirements	5967

A.18.3.1.7	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is not used	5968

A.18.3.1.7.1	Test Purpose and Environment	5968

A.18.3.1.7.2	Test Requirements	5970

A.18.3.1.8	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is used	5971

A.18.3.1.8.1	Test Purpose and Environment	5971

A.18.3.1.8.2	Test Requirements	5973

Annex B (normative): Conditions for RRM requirements applicability for operating bands	5973

B.1	Conditions for NR RRC_IDLE state mobility	5973

B.1.1	Introduction	5973

B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection	5973

B.1.3	Conditions for measurements on NR inter-frequency cells for cell re-selection	5974

B.1.4	Conditions for measurements on NR intra-frequency cells for cell re-selection for RedCap	5974

B.1.5	Conditions for measurements on NR inter-frequency cells for cell re-selection for RedCap	5976

B.1.6	Conditions for measurements on NR intra-frequency cells for cell re-selection for satellite access	5976

B.1.7	Conditions for measurements on NR inter-frequency cells for cell re-selection for satellite access	5977

B.2	Conditions for UE measurements procedures and performance requirements in RRC_CONNECTED state	5977

B.2.1	Introduction	5977

B.2.1.1	General	5977

B.2.1.2	Derivation of Minimum SSB_RP values for FR1	5977

B.2.1.3	Derivation of Minimum SSB_RP values for FR2	5977

B.2.1.3.1	Minimum SSB_RP values for Rx Beam Peak angle of arrival	5977

B.2.1.3.2	Minimum SSB_RP values for angle of arrival within Spherical coverage	5978

B.2.1.4	Gain to SS-RSRP and CSI-RSRP measurement point for FR1	5979

B.2.1.5	Gain to SS-RSRP and CSI-RSRP measurement point for FR2	5979

B.2.1.5.1	Gain to SS-RSRP and CSI-RSRP measurement point for Rx Beam Peak angle of arrival	5979

B.2.1.5.2	Gain to SS-RSRP measurement point for different frequency	5980

B.2.1.5.3	Alignment of Rough beam to Rx beam Peak	5980

B.2.1.6	Gain to PRS-RSRP measurement point for FR2	5980

B.2.1.6.1	Gain to PRS-RSRP measurement point for Rx Beam Peak angle of arrival	5980

B.2.2	Conditions for NR intra-frequency measurements	5981

B.2.3	Conditions for NR inter-frequency measurements	5982

B.2.4	Conditions for NR L1-RSRP reporting	5983

B.2.4.1	Conditions for SSB based L1-RSRP reporting	5983

B.2.4.2	Conditions for CSI-RS based L1-RSRP reporting	5985

B.2.5	Conditions for RRC connection release with redirection to NR	5986

B.2.6	Void	5987

B.2.6.1	Void	5987

B.2.6.2	Void	5987

B.2.7	Conditions for SRS-RSRP measurements	5987

B.2.8	Conditions for NR L1-SINR reporting	5988

B.2.8.1	Conditions for L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	5988

B.2.8.2	Conditions for L1-SINR reporting with SSB based CMR and dedicated IMR configured	5989

B.2.8.2.1	L1-SINR reporting with SSB based CMR and dedicated ZP-IMR configured	5989

B.2.8.2.2	L1-SINR reporting with SSB based CMR and dedicated NZP-IMR configured	5990

B.2.8.3	Conditions for L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	5991

B.2.8.3.1	L1-SINR reporting with CSI-RS based CMR and dedicated ZP-IMR configured	5991

B.2.8.3.2	L1-SINR reporting with CSI-RS based CMR and dedicated NZP-IMR configured	5992

B.2.9	Conditions for NR intra-frequency measurements under CCA	5994

B.2.10	Conditions for NR inter-frequency measurements under CCA	5994

B.2.11	Conditions for NR L1-RSRP reporting under CCA	5994

B.2.11.1	Conditions for SSB based L1-RSRP reporting	5994

B.2.12	Conditions for NR CSI-RS based intra-frequency measurements	5995

B.2.13	Conditions for NR CSI-RS based inter-frequency measurements	5996

B.2.14	Conditions for NR PRS-based measurements	5998

B.2.15	Conditions for NR intra-frequency measurements for RedCap	6000

B.2.16	Conditions for NR inter-frequency measurements for RedCap	6001

B.2.17	Conditions for NR intra-frequency measurements for satellite access	6003

B.2.18	Conditions for NR inter-frequency measurements for satellite access	6004

B.2.19	Conditions for NR L1-RSRP reporting for satellite access	6004

B.2.19.1	Conditions for SSB based L1-RSRP reporting for satellite access	6004

B.2.19.2	Conditions for CSI-RS based L1-RSRP reporting for satellite access	6004

B.2.20	Conditions for RRC connection release with redirection to NR for satellite access	6005

B.3	RRM Requirements Exceptions	6005

B.3.1	Introduction	6005

B.3.2	Receiver sensitivity relaxation for CA	6005

B.3.2.1	Receiver sensitivity relaxation for UE supporting CA in FR1	6005

B.3.2.2	Receiver sensitivity relaxation for UE configured with CA in FR1	6005

B.3.2.2.1	Inter-band carrier aggregation	6005

B.3.2.2.2	Reference sensitivity exceptions due to UL harmonic interference for CA	6005

B.3.2.2.3	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA	6006

B.3.2.3	Receiver sensitivity relaxation for UE supporting CA in FR2	6006

B.3.2.4	Receiver sensitivity relaxation for UE configured with CA in FR2	6006

B.3.2.4.1	Intra-band contiguous carrier aggregation	6006

B.3.2.4.2	Intra-band non-contiguous carrier aggregation	6006

B.3.3	Receiver sensitivity relaxation for DC	6006

B.3.3.1	Receiver sensitivity relaxation for EN-DC	6006

B.3.3.2	Receiver sensitivity relaxation for NE-DC	6006

B.3.4	Receiver sensitivity relaxation for SUL	6006

B.3.4.1	Receiver sensitivity relaxation for UE supporting SUL in FR1	6006

B.3.4.2	Receiver sensitivity relaxation for UE configured with SUL in FR1	6007

B.3.4.2.1	Reference sensitivity exceptions due to UL harmonic interference for SUL	6007

B.4	Conditions for V2X	6007

B.4.1	Test parameters for GNSS signals	6007

B.4.2	Conditions for PSBCH-RSRP Accuracy Requirements	6007

B.4.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	6008

B.4.4	Conditions for L1 SL-RSRP Accuracy Requirements	6008

B.5	High level test procedure for SAN RRM tests	6008

Annex C (informative): Change history	6010