| 3GPP TS 38.133 V16.20.0 (2024-06) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Requirements for support of radio resource management (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	69

1	Scope	71

2	References	71

3	Definitions, symbols and abbreviations	72

3.1	Definitions	72

3.2	Symbols	73

3.3	Abbreviations	74

3.4	Test tolerances	77

3.5	Frequency bands grouping	77

3.5.1	Introduction	77

3.5.2	NR operating bands in FR1	77

3.5.3	NR operating bands in FR2	78

3.6	Applicability of requirements in this specification version	78

3.6.1	RRC connected state requirements in DRX	79

3.6.2	Number of serving carriers	79

3.6.2.1	Number of serving carriers for SA	79

3.6.2.2	Number of serving carriers for EN-DC	79

3.6.2.3	Number of serving carriers for NE-DC	79

3.6.2.4	Number of serving carriers for NR-DC	80

3.6.3	Applicability for intra-band FR2	80

3.6.4	Applicability for FR2 UE power classes	80

3.6.5	Applicability for SDL bands	80

3.6.6	Applicability of requirements for NGEN-DC operation	80

3.6.7	Applicability of QCL	80

3.6.9	Applicability of requirements for scheduling availability	81

3.6.10	Applicability of requirements for measurement restrictions	81

4	SA: RRC_IDLE state mobility	81

4.1	Cell Selection	81

4.2	Cell Re-selection	81

4.2.1	Introduction	81

4.2.2	Requirements	82

4.2.2.1	UE measurement capability	82

4.2.2.2	Measurement and evaluation of serving cell	82

4.2.2.3	Measurements of intra-frequency NR cells	83

4.2.2.4	Measurements of inter-frequency NR cells	84

4.2.2.5	Measurements of inter-RAT E-UTRAN cells	86

4.2.2.6	Maximum interruption in paging reception	88

4.2.2.7	General requirements	88

4.2.2.8	Minimum requirement at transitions	88

4.2.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	89

4.2.2.9.1	Introduction	89

4.2.2.9.2	Measurements for UE fulfilling low mobility criterion	89

4.2.2.9.3	Measurements for UE fulfilling not-at-cell edge criterion	90

4.2.2.9.4	Measurements for UE fulfilling low mobility and not-at-cell edge criteria	91

4.2.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	91

4.2.2.10.1	Introduction	91

4.2.2.10.2	Measurements for UE fulfilling low mobility criterion	91

4.2.2.10.3	Measurements for UE fulfilling not-at-cell edge criterion	92

4.2.2.10.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	93

4.2.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	94

4.2.2.11.1	Introduction	94

4.2.2.11.2	Measurements for UE fulfilling low mobility criterion	94

4.2.2.11.3	Measurements for UE fulfilling with not-at-cell edge criterion	95

4.2.2.11.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	96

4.2A	Cell Re-selection when subject to CCA	96

4.2A.1	Introduction	96

4.2A.2	Requirements	97

4.2A.2.1	UE measurement capability	97

4.2A.2.2	Measurement and evaluation when subject to CCA on the serving cell	97

4.2A.2.3	Measurements of intra-frequency NR cells when subject to CCA on the serving cell and target cell	98

4.2A.2.4	Measurements of inter-frequency NR cells when subject to CCA on the target cell	99

4.2A.2.5	Measurements of inter-RAT E-UTRAN cells when subject to CCA on the serving cell	101

4.2A.2.6	Maximum interruption in paging reception when subject to CCA on the target cell	101

4.2A.2.7	General requirements	101

4.3	Minimization of Drive Tests (MDT)	102

4.3.1	Introduction	102

4.3.2	Measurement Requirements	102

4.3.3	Requirements for Relative Time Stamp Accuracy	102

4.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	103

4.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	103

4.4	Idle Mode CA/DC Measurements	103

4.4.1	Introduction	103

4.4.2	Measurement Requirements	103

4.4.2.1	Detected cell requirement during state transition and Idle mode	103

4.4.2.2	Measurements of inter-frequency CA/DC candidate cells	104

4.4.2.3	Measurements on serving cell	105

4.4.2.4	Measurements of E-UTRAN inter-RAT DC candidate cells	105

5	SA: RRC_INACTIVE state mobility	107

5.1	Cell Re-selection	107

5.1.1	Introduction	107

5.1.2	Requirements	107

5.1.2.1	UE measurement capability	107

5.1.2.2	Measurement and evaluation of serving cell	107

5.1.2.3	Measurements of intra-frequency NR cells	107

5.1.2.4	Measurements of inter-frequency NR cells	107

5.1.2.5	Measurements of inter-RAT E-UTRAN cells	107

5.1.2.6	Maximum interruption in paging reception	107

5.1.2.7	General requirements	107

5.1A	Cell Re-selection with CCA	107

5.1A.1	Introduction	107

5.1A.2	Requirements	108

5.1A.2.1	UE measurement capability	108

5.1A.2.2	Measurement and evaluation when CCA is used on the serving cell	108

5.1A.2.3	Measurements of intra-frequency NR cells when CCA is used on the serving cell and target cell	108

5.1A.2.4	Measurements of inter-frequency NR cells when CCA is used on the target cell	108

5.1A.2.5	Measurements of inter-RAT E-UTRAN cells when CCA is used on the serving cell	108

5.1A.2.6	Maximum interruption in paging reception when CCA is used on the target cell	108

5.1A.2.7	General requirements	108

5.2	Void	108

5.3	Minimization of Drive Tests (MDT)	108

5.3.1	Introduction	108

5.3.2	Measurement Requirements	109

5.3.3	Requirements for Relative Time Stamp Accuracy	109

5.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	109

5.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	109

5.3.6	Requirements for Relative Time Stamp Accuracy for RRC Resume Failure Log Reporting	109

5.4	Inactive Mode CA/DC Measurements	109

5.4.1	Introduction	109

5.4.2	Measurement Requirements	109

5.4.2.1	Detected cell requirement during state transition and inactive mode	110

5.4.2.2	Measurements of inter-frequency CA/DC candidate cells	110

5.4.2.3	Measurements on serving cell	110

5.4.2.4	Measurements on E-UTRAN inter-RAT DC candidate cells	110

6	RRC_CONNECTED state mobility	110

6.1	Handover	110

6.1.1	NR Handover	110

6.1.1.1	Introduction	110

6.1.1.2	NR FR1 - NR FR1 Handover	110

6.1.1.2.1	Handover delay	110

6.1.1.2.2	Interruption time	110

6.1.1.3	NR FR2- NR FR1 Handover	111

6.1.1.3.1	Handover delay	111

6.1.1.3.2	Interruption time	111

6.1.1.4	NR FR2- NR FR2 Handover	112

6.1.1.4.1	Handover delay	112

6.1.1.4.2	Interruption time	112

6.1.1.5	NR FR1- NR FR2 Handover	113

6.1.1.5.1	Handover delay	113

6.1.1.5.2	Interruption time	113

6.1.2	NR Handover to other RATs	114

6.1.2.1	NR – E-UTRAN Handover	114

6.1.2.1.1	Introduction	114

6.1.2.1.2	Handover delay	114

6.1.2.1.3	Interruption time	114

6.1.2.2	NR – UTRAN Handover	115

6.1.2.2.1	Introduction	115

6.1.2.2.2	Handover delay	115

6.1.2.2.3	Interruption time	115

6.1.3	NR DAPS Handover	116

6.1.3.1	Introduction	116

6.1.3.2	NR FR1 - NR FR1 DAPS Handover	116

6.1.3.2.1	DAPS handover delay	116

6.1.3.2.2	Interruption time	117

6.1.3.3	NR FR2- NR FR1 DAPS Handover	119

6.1.3.3.1	DAPS handover delay	119

6.1.3.3.2	Interruption time	119

6.1.3.4	NR FR1- NR FR2 DAPS Handover	120

6.1.3.4.1	DAPS handover delay	120

6.1.3.4.2	Interruption time	120

6.1.4	NR Conditional Handover	121

6.1.4.1	Introduction	121

6.1.4.2	NR FR1 – NR FR1 conditional handover	121

6.1.4.3	NR FR2 – NR FR1 conditional handover	122

6.1.4.4	NR FR2 – NR FR2 conditional handover	123

6.1.4.4.1	Handover delay	123

6.1.4.4.2	Measurement time	123

6.1.4.4.3	Preparation time	123

6.1.4.4.4	Interruption time	123

6.1.4.5	NR FR1 – NR FR2 conditional handover	124

6.1A	Void	124

6.1A.1	Void	124

6.1A.1.1	Void	124

6.1A.1.2	Void	124

6.1A.1.2.1	Void	124

6.1A.1.2.2	Void	124

6.1B	Handover to target cell using CCA	124

6.1B.1	NR Handover	124

6.1B.1.1	Introduction	124

6.1B.1.2	NR FR1 - NR FR1 Handover	125

6.1B.1.2.1	Handover delay	125

6.1B.1.2.2	Interruption time	125

6.2	RRC Connection Mobility Control	126

6.2.1	SA: RRC Re-establishment	126

6.2.1.1	Introduction	126

6.2.1.2	Requirements	126

6.2.1.2.1	UE Re-establishment delay requirement	126

6.2.1A	RRC Re-establishment with CCA	127

6.2.1A.1	Introduction	127

6.2.1A.2	Requirements	128

6.2.1A.2.1	UE Re-establishment with CCA delay requirement	128

6.2.2	Random access	130

6.2.2.1	Introduction	130

6.2.2.2	Requirements for 4-step RA type	130

6.2.2.2.1	Contention based random access	130

6.2.2.2.2	Non-Contention based random access	131

6.2.2.2.3	UE behaviour when configured with supplementary UL	132

6.2.2.3	Requirements for 2-step RA type	132

6.2.2.3.1	Contention based random access	132

6.2.2.3.2	Non-Contention based random access	133

6.2.2.3.3	UE behaviour when configured with supplementary UL	134

6.2.2A	Random access when CCA is used on target frequency	134

6.2.2A.1	Introduction	134

6.2.2A.2	Requirements for 4-step RA type	134

6.2.2A.2.1	Contention based random access	134

6.2.2A.2.2	Non-Contention based random access	135

6.2.2A.3	Requirements for 2-step RA type	136

6.2.2A.3.1	Contention based random access	137

6.2.2A.3.2	Non-Contention based random access	138

6.2.3	SA: RRC Connection Release with Redirection	139

6.2.3.1	Introduction	139

6.2.3.2	Requirements	139

6.2.3.2.1	RRC connection release with redirection to NR	139

6.2.3.2.2	RRC connection release with redirection to E-UTRAN	140

6.2.3.2.3	RRC connection release with redirection to NR carrier subject to CCA	140

7	Timing	141

7.1	UE transmit timing	141

7.1.1	Introduction	141

7.1.2	Requirements	142

7.1.2.1	Gradual timing adjustment	143

7.1.2.2	Void	143

7.2	UE timer accuracy	143

7.2.1	Introduction	143

7.2.2	Requirements	144

7.3	Timing advance	144

7.3.1	Introduction	144

7.3.2	Requirements	144

7.3.2.1	Timing Advance adjustment delay	144

7.3.2.2	Timing Advance adjustment accuracy	144

7.4	Cell phase synchronization accuracy	144

7.4.1	Definition	144

7.4.2	Minimum requirements	144

7.5	Maximum Transmission Timing Difference	145

7.5.1	Introduction	145

7.5.2	Minimum Requirements for inter-band EN-DC	145

7.5.2.1	Minimum Requirements for inter-band synchronous EN-DC	145

7.5.3	Minimum Requirements for intra-band EN-DC	146

7.5.4	Minimum Requirements for NR Carrier Aggregation	146

7.5.5	Minimum Requirements for inter-band NE-DC	147

7.5.5.1	Minimum Requirements for inter-band synchronous NE-DC	147

7.5.6	Minimum Requirements for inter-band NR DC	148

7.6	Maximum Receive Timing Difference	148

7.6.1	Introduction	148

7.6.2	Minimum Requirements for inter-band EN-DC	149

7.6.2.1	Minimum Requirements for inter-band synchronous EN-DC	149

7.6.3	Minimum Requirements for intra-band EN-DC	149

7.6.4	Minimum Requirements for NR Carrier Aggregation	150

7.6.5	Minimum Requirements for inter-band NE-DC	151

7.6.5.1	Minimum Requirements for inter-band synchronous NE-DC	151

7.6.6	Minimum Requirements for inter-band NR DC	151

7.7	deriveSSB-IndexFromCell tolerance	152

7.7.1	Minimum requirements	152

7.8	Void	152

8	Signalling characteristics	152

8.1	Radio Link Monitoring	152

8.1.1	Introduction	152

8.1.2	Requirements for SSB based radio link monitoring	153

8.1.2.1	Introduction	153

8.1.2.2	Minimum requirement	154

8.1.2.3	Measurement restrictions for SSB based RLM	156

8.1.3	Requirements for CSI-RS based radio link monitoring	156

8.1.3.1	Introduction	156

8.1.3.2	Minimum requirement	157

8.1.3.3	Measurement restrictions for CSI-RS based RLM	159

8.1.4	Minimum requirement at transitions	160

8.1.5	Minimum requirement for UE turning off the transmitter	160

8.1.6	Minimum requirement for L1 indication	160

8.1.7	Scheduling availability of UE during radio link monitoring	161

8.1.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	161

8.1.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	161

8.1.7.3	Scheduling availability of UE performing radio link monitoring on FR2	161

8.1.7.4	Scheduling availability of UE performing radio link monitoring on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	162

8.1A	Radio Link Monitoring with CCA on Target Frequency	162

8.1A.1	Introduction	162

8.1A.2	Requirements for SSB Based Radio Link Monitoring	163

8.1A.2.1	Introduction	163

8.1A.2.2	Minimum Requirement	164

8.1A.3	Minimum requirement at transitions	165

8.1A.4	Minimum requirement for UE turning off the transmitter	166

8.1A.5	Minimum requirement for L1 indication	166

8.1A.6	Scheduling availability of UE during radio link monitoring	166

8.1A.6.1	Scheduling availability of UE performing radio link monitoring with the same subcarrier spacing as PDSCH/PDCCH	166

8.1A.6.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH	166

8.2	Interruption	167

8.2.1	EN-DC Interruption	167

8.2.1.1	Introduction	167

8.2.1.2	Requirements	167

8.2.1.2.1	Interruptions at transitions between active and non-active during DRX	167

8.2.1.2.2	Interruptions at transitions from non-DRX to DRX	168

8.2.1.2.3	Interruptions at SCell addition/release	168

8.2.1.2.4	Interruptions at SCell activation/deactivation	169

8.2.1.2.5	Interruptions during measurements on SCC	170

8.2.1.2.6	Interruptions at UL carrier RRC reconfiguration	172

8.2.1.2.7	Interruptions due to Active BWP switching Requirement	172

8.2.1.2.8	Interruptions at direct SCell activation and hibernation	173

8.2.1.2.9	Interruptions at SCell hibernation	174

8.2.1.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	174

8.2.1.2.11	Interruptions due to UE-specific CBW change	174

8.2.1.2.12	 Interruptions at NR SRS carrier based switching	175

8.2.1.2.13	 Interruptions at E-UTRA SRS carrier based switching	176

8.2.1.2.14	DL Interruptions at switching between two uplink carriers	177

8.2.1.2.15	Interruptions due to SCell dormancy	177

8.2.1.2.16	Interruptions when identifying CGI of an NR cell with autonomous gaps	178

8.2.1.2.17	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	178

8.2.2	SA: Interruptions with Standalone NR Carrier Aggregation	179

8.2.2.1	Introduction	179

8.2.2.2	Requirements	180

8.2.2.2.1	Interruptions at SCell addition/release	180

8.2.2.2.2	Interruptions at SCell activation/deactivation	181

8.2.2.2.3	Interruptions during measurements on deactivated SCC	182

8.2.2.2.4	Interruptions at UL carrier RRC reconfiguration	182

8.2.2.2.5	Interruptions due to Active BWP switching Requirement	183

8.2.2.2.6	Interruptions at inter-frequency SFTD measurement	184

8.2.2.2.7	Interruptions at SCell activation/deactivation with multiple downlink SCells	185

8.2.2.2.8	Interruptions due to UE-specific CBW change	185

8.2.2.2.9	 Interruptions at NR SRS carrier based switching	185

8.2.2.2.10	DL Interruptions at UE switching between two uplink carriers	187

8.2.2.2.11	Interruptions at direct SCell activation	187

8.2.2.2.12	Interruptions due to SCell dormancy	188

8.2.2.2.13	Interruptions at transitions between active and non-active during DRX	188

8.2.2.2.14	Interruptions when identifying CGI of an NR cell with autonomous gaps	188

8.2.2.2.15	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	189

8.2.3	NE-DC Interruptions	189

8.2.3.1	Introduction	189

8.2.3.2	Requirements	190

8.2.3.2.1	Interruptions at transitions between active and non-active during DRX	190

8.2.3.2.2	Interruptions at transitions from non-DRX to DRX	190

8.2.3.2.3	Interruptions at PSCell/SCell addition/release	190

8.2.3.2.4	Interruptions at SCell activation/deactivation	192

8.2.3.2.5	Interruptions during measurements on SCC	193

8.2.3.2.6	Interruptions at UL carrier RRC reconfiguration	194

8.2.3.2.7	Interruptions due to Active BWP switching Requirement	194

8.2.3.2.8	Interruptions at direct SCell activation and hibernation	195

8.2.3.2.9	Interruptions at SCell hibernation	195

8.2.3.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	195

8.2.3.2.11	 Interruptions at NR SRS carrier based switching	195

8.2.3.2.12	 Interruptions at E-UTRA SRS carrier based switching	197

8.2.3.2.13	Interruptions due to SCell dormancy	198

8.2.3.2.14	Interruptions when identifying CGI of an NR cell with autonomous gaps	198

8.2.3.2.15	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	199

8.2.3.2.16	Interruptions due to UE-specific CBW change	199

8.2.4	NR-DC: Interruptions	200

8.2.4.1	Introduction	200

8.2.4.2	Requirements	200

8.2.4.2.1	Interruptions at PSCell/SCell addition/release	200

8.2.4.2.3	Interruptions during measurements on SCC	202

8.2.4.2.4	Interruptions at UL carrier RRC reconfiguration	202

8.2.4.2.5	Interruptions due to Active BWP switching Requirement	203

8.2.4.2.6	Interruptions at transitions between active and non-active during DRX	203

8.2.4.2.7	Interruptions at transitions from non-DRX to DRX	203

8.2.4.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	203

8.2.4.2.9	Interruptions at NR SRS carrier based switching	204

8.2.4.2.10	Interruptions at direct SCell activation	205

8.2.4.2.11	Interruptions when identifying CGI of an NR cell with autonomous gaps	205

8.2.4.2.12	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	206

8.2.4.2.13	 Interruptions due to SCell dormancy	207

8.2.4.2.14	Interruptions due to UE-specific CBW change	207

8.2.4.2A	Void	207

8.2.4.2A.1	Void	207

8.2.4.2A.2	Void	207

8.2.4.2A.3	Void	207

8.3	SCell Activation and Deactivation Delay	207

8.3.1	Introduction	207

8.3.2	SCell Activation Delay Requirement for Deactivated SCell	208

8.3.3	SCell Deactivation Delay Requirement for Activated SCell	212

8.3.4	Direct SCell Activation at SCell addition	212

8.3.5	Direct SCell Activation at Handover	214

8.3.7	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	216

8.3.8	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	219

8.3.9	Direct SCell Activation of Multiple Downlink SCells at SCell addition	220

8.3.10	Direct SCell Activation of Multiple Downlink SCells at Handover	221

8.3A	SCell Activation and Deactivation Delay in Carriers with CCA	222

8.3A.1	Introduction	222

8.3A.2	SCell Activation Delay Requirement for Deactivated SCell	223

8.3A.3	SCell Deactivation Delay Requirement for Activated SCell	225

8.4	UE UL carrier RRC reconfiguration delay	226

8.4.1	Introduction	226

8.4.2	UE UL carrier configuration delay requirement	226

8.4.3	UE UL carrier deconfiguration delay requirement	226

8.5	Link Recovery Procedures	226

8.5.1	Introduction	226

8.5.2	Requirements for SSB based beam failure detection	227

8.5.2.1	Introduction	227

8.5.2.2	Minimum requirement	228

8.5.2.3	Measurement restriction for SSB based beam failure detection	229

8.5.3	Requirements for CSI-RS based beam failure detection	230

8.5.3.1	Introduction	230

8.5.3.2	Minimum requirement	230

8.5.3.3	Measurement restrictions for CSI-RS beam failure detection	232

8.5.4	Minimum requirement for L1 indication	233

8.5.5	Requirements for SSB based candidate beam detection	234

8.5.5.1	Introduction	234

8.5.5.2	Minimum requirement	234

8.5.5.3	Measurement restriction for SSB based candidate beam detection	236

8.5.6	Requirements for CSI-RS based candidate beam detection	236

8.5.6.1	Introduction	236

8.5.6.2	Minimum requirement	236

8.5.6.3	Measurement restriction for CSI-RS based candidate beam detection	239

8.5.7	Scheduling availability of UE during beam failure detection	239

8.5.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	239

8.5.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	239

8.5.7.3	Scheduling availability of UE performing beam failure detection on FR2	240

8.5.7.4	Scheduling availability of UE performing beam failure detection on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR DC	240

8.5.8	Scheduling availability of UE during candidate beam detection	240

8.5.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	240

8.5.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	241

8.5.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	241

8.5.8.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	241

8.5.9	Requirements for Beam Failure Recovery in SCell	242

8.5.9.1	Introduction	242

8.5.9.2	Requirement	242

8.5.10	Minimum requirement at transitions for beam failure detection	242

8.5A	Link Recovery Procedures when CCA is used on target frequency	243

8.5A.1	Introduction	243

8.5A.2	Requirements for SSB based beam failure detection	243

8.5A.2.1	Introduction	243

8.5A.2.2	Minimum requirement	244

8.5A.2.3	Measurement restriction for SSB based beam failure detection	244

8.5A.4	Minimum requirement for L1 indication	245

8.5A.5	Requirements for SSB based candidate beam detection	245

8.5A.5.1	Introduction	245

8.5A.5.2	Minimum requirement	245

8.5A.5.3	Measurement restriction for SSB based candidate beam detection	246

8.5A.7	Scheduling availability of UE during beam failure detection	246

8.5A.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH	246

8.5A.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH	246

8.5A.8	Scheduling availability of UE during candidate beam detection	246

8.5A.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH	247

8.5A.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH	247

8.6	Active BWP switch delay	247

8.6.1	Introduction	247

8.6.2	DCI and timer based BWP switch delay on a single CC	247

8.6.2A	DCI based BWP switch delay on multiple CCs	248

8.6.2A.1	Simultaneous DCI based BWP switch delay on multiple CCs	248

8.6.2A.2	Non-simultaneous DCI based BWP switch delay on multiple CCs	250

8.6.2B	Timer based BWP switch delay on multiple CCs	250

8.6.2B.1	Simultaneous timer based BWP switch delay on multiple CCs	250

8.6.2B.2	Non-simultaneous timer based BWP switch delay on multiple CCs	251

8.6.3	RRC based BWP switch delay on a single CC	251

8.6.3A	RRC based BWP switch delay on multiple CCs	252

8.6.3A.1	Simultaneous RRC based BWP switch delay on multiple CCs	252

8.6.3A.2	Non-simultaneous RRC based BWP switch delay on multiple CCs	252

8.6.4	BWP switch delay on Consistent UL CCA recovery	253

8.7	Void	253

8.8	NE-DC: E-UTRAN PSCell Addition and Release Delay	253

8.8.1	Introduction	253

8.8.2	E-UTRAN PSCell Addition Delay Requirement	253

8.8.3	E-UTRAN PSCell Release Delay Requirement	254

8.9	NR-DC: PSCell Addition and Release Delay	254

8.9.1	Introduction	254

8.9.2	PSCell Addition Delay Requirement	254

8.9.3	PSCell Release Delay Requirement	255

8.10	Active TCI state switching delay	255

8.10.4	DCI based TCI state switch delay	257

8.10.5	RRC based TCI state switch delay	257

8.10.6	Active TCI state list update delay	257

8.10A	Active TCI state switching delay with CCA	258

8.10A.1	Introduction	258

8.10A.2	Known conditions for TCI state	258

8.10A.3	MAC-CE based TCI state switch delay	258

8.10A.4	DCI based TCI state switch delay	259

8.10A.5	RRC based TCI state switch delay	259

8.10A.6	Active TCI state list update delay	260

8.11	PSCell Change	260

8.11A	void	261

8.11B	Conditional PSCell Change	261

8.11B.1	Introduction	261

8.11B.2	Conditoinal PSCell Change delay	261

8.11B.2.1	Measurement time	261

8.12	Uplink spatial relation switch delay	262

8.12.1	Introduction	262

8.12.2	Known conditions for spatial relation when associated with DL-RS	262

8.12.3	MAC-CE based spatial relation switch delay	262

8.12.4	DCI based spatial relation switch delay	263

8.12.5	RRC based spatial relation switch delay	263

8.13	UE-specific CBW change	264

8.13.1	Introduction	264

8.13.2	UE-specific CBW change delay	264

8.14	Pathloss reference signal switching delay	264

8.14.1	Introduction	264

8.14.2	Known conditions for pathloss reference signal	264

8.14.3	MAC-CE based pathloss reference signal switch delay	265

9	Measurement Procedure	266

9.1	General measurement requirement	266

9.1.1	Introduction	266

9.1.2	Measurement gap	266

9.1.2.1	EN-DC: Measurement Gap Sharing	276

9.1.2.1a	SA: Measurement Gap Sharing	277

9.1.2.1b	NE-DC: Measurement Gap Sharing	278

9.1.2.1c	NR-DC: Measurement Gap Sharing	279

9.1.3	UE Measurement capability	280

9.1.3.1	EN-DC: Monitoring of multiple layers using gaps	280

9.1.3.1a	SA: Monitoring of multiple layers using gaps	280

9.1.3.1b	NE-DC: Monitoring of multiple layers using gaps	281

9.1.3.1c	NR-DC: Monitoring of multiple layers using gaps	281

9.1.3.2	EN-DC: Maximum allowed layers for multiple monitoring	282

9.1.3.2a	SA: Maximum allowed layers for multiple monitoring	283

9.1.3.2b	NE-DC: Maximum allowed layers for multiple monitoring	283

9.1.3.2c	NR-DC: Maximum allowed layers for multiple monitoring	284

9.1A.3.2	Void	285

9.1.3A	UE Measurement capability under operation mode with CCA	285

9.1.3A.1	EN-DC: Monitoring of multiple layers using gaps under CCA	285

9.1.3A.1A	SA: Monitoring of multiple layers using gaps under CCA	285

9.1.3A.2	EN-DC: Maximum allowed layers for multiple monitoring under CCA	285

9.1A.3.2a	Void	286

9.1.3A.2A	SA: Maximum allowed layers for multiple monitoring under CCA	286

9.1.4	Capabilities for Support of Event Triggering and Reporting Criteria	286

9.1.4.1	Introduction	286

9.1.4.2	Requirements	286

9.1.5	Carrier-specific scaling factor	290

9.1.5.1	Monitoring of multiple layers outside gaps	290

9.1.5.1.1	EN-DC mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	292

9.1.5.1.2	SA mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	293

9.1.5.1.3	NR-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed outside gaps	294

9.1.5.1.4	NE-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based measurements performed outside gaps	295

9.1.5.2	Monitoring of multiple layers within gaps	296

9.1.5.2.1	EN-DC mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	297

9.1.5.2.2	SA mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	299

9.1.5.2.3	NE-DC: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed within gaps	301

9.1.5.2.4	NR-DC: carrier-specific scaling factor for SSB-based and CSI-RS-based L3 measurements performed within gaps	303

9.1.5.2.5	SA mode: carrier-specific scaling factor for PRS-based measurements performed within gaps	304

9.1.5.2.6	NE-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	305

9.1.5.2.7	NR-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	305

9.1.6	Minimum requirement at transitions	305

9.2	NR intra-frequency measurements	305

9.2.1	Introduction	305

9.2.2	Requirements applicability	306

9.2.3	Number of cells and number of SSB	306

9.2.3.1	Requirements for FR1	306

9.2.3.2	Requirements for FR2	306

9.2.4	Measurement Reporting Requirements	307

9.2.4.1	Periodic Reporting	307

9.2.4.2	Event-triggered Periodic Reporting	307

9.2.4.3	Event Triggered Reporting	307

9.2.5	Intrafrequency measurements without measurement gaps	308

9.2.5.1	Intrafrequency cell identification	308

9.2.5.2	Measurement period	310

9.2.5.3	Scheduling availability of UE during intra-frequency measurements	312

9.2.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	312

9.2.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	312

9.2.5.3.3	Scheduling availability of UE performing measurements on FR2	313

9.2.5.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	314

9.2.5.4	SFTD Measurements between PCell and PSCell	314

9.2.5.4.1	Introduction	314

9.2.5.4.2	SFTD Measurement delay	314

9.2.5.4.3	SFTD Measurement Reporting Delay	315

9.2.6	Intra-frequency measurements with measurement gaps	315

9.2.6.1	Void	315

9.2.6.2	Intra-frequency cell identification	315

9.2.6.3	Intrafrequency Measurement Period	317

9.2A	NR intra-frequency measurements with CCA	318

9.2A.1	Introduction	318

9.2A.2	Requirements applicability	318

9.2A.3	Number of cells and number of SSB	319

9.2A.4	Measurement Reporting Requirements	319

9.2A.5	Intra-frequency measurements without measurement gaps	319

9.2A.5.2	Measurement period	322

9.2A.5.3	Scheduling availability of UE during intra-frequency measurements	323

9.2A.5.3.1	Scheduling availability of UE performing measurements in TDD bands	323

9.2A.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH	323

9.2A.6	Intra-frequency measurements with measurement gaps	324

9.2A.6.1	Intra-frequency cell identification	324

9.2A.6.2	Intra-frequency Measurement Period	325

9.2A.7	Intra-frequency RSSI and Channel occupancy measurements	326

9.2A.7.1	Intra-frequency RSSI measurements	326

9.2A.7.2	Intra-frequency Channel occupancy measurements	327

9.2A.7.3	Scheduling restriction during RSSI and Channel Occupancy measurements	327

9.3	NR inter-frequency measurements	328

9.3.1	Introduction	328

9.3.2	Requirements applicability	328

9.3.2.1	Void	329

9.3.2.2	Void	329

9.3.3	Number of cells and number of SSB	329

9.3.3.1	Requirements for FR1	329

9.3.3.2	Requirements for FR2	329

9.3.4	Inter-frequency measurement with measurement gaps	329

9.3.4.1	Void	330

9.3.4.2	Void	330

9.3.5	Inter-frequency measurements	330

9.3.5.1	Void	331

9.3.5.2	Void	331

9.3.5.3	Void	331

9.3.6	Inter-frequency measurements reporting requirements	331

9.3.6.1	Periodic Reporting	331

9.3.6.2	Event-triggered Periodic Reporting	331

9.3.6.3	Event-triggered Reporting	331

9.3.7	Void	332

9.3.8	Inter-frequency SFTD measurement requirements	332

9.3.8.1	Introduction	332

9.3.8.2	SFTD Measurement delay	332

9.3.8.3	SFTD Measurement reporting delay	333

9.3.9	Inter frequency measurements without measurement gaps	333

9.3.9.1	Inter frequency Cell identification	333

9.3.9.2	Measurement period	335

9.3.9.3	Scheduling availability of UE during inter-frequency measurements	336

9.3.9.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	336

9.3.9.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	336

9.3.9.3.3	Scheduling availability of UE performing measurements on FR2	337

9.3.9.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	337

9.3A	NR inter-frequency measurements in carrier frequencies with CCA	337

9.3A.1	Introduction	337

9.3A.2	Requirements applicability	338

9.3A.3	Number of cells and number of SSB	338

9.3A.3.1	Requirements	338

9.3A.4	Inter-frequency cell identification	338

9.3A.5	Inter-frequency measurements	340

9.3A.6	NR Inter-frequency measurements reporting requirements	340

9.3A.6.1	Periodic Reporting	340

9.3A.6.2	Event-triggered Periodic Reporting	340

9.3A.6.3	Event-triggered Reporting	340

9.3A.8	Inter-frequency RSSI measurements	341

9.3A.9	Inter-frequency channel occupancy measurements	341

9.4	Inter-RAT measurements	342

9.4.1	Introduction	342

9.4.2	NR − E-UTRAN FDD measurements	343

9.4.2.1	Introduction	343

9.4.2.2	Requirements when no DRX is used	344

9.4.2.3	Requirements when DRX is used	344

9.4.2.4	Measurement reporting requirements	345

9.4.2.4.1	Periodic Reporting	345

9.4.2.4.2	Event-Triggered Periodic Reporting	346

9.4.2.4.3	Event-Triggered Reporting	346

9.4.3	NR − E-UTRAN TDD measurements	346

9.4.3.1	Introduction	346

9.4.3.2	Requirements when no DRX is used	346

9.4.3.3	Requirements when DRX is used	347

9.4.3.4	Measurement reporting requirements	349

9.4.3.4.1	Periodic Reporting	349

9.4.3.4.2	Event-Triggered Periodic Reporting	349

9.4.3.4.3	Event-Triggered Reporting	349

9.4.4	Inter-RAT RSTD measurements	349

9.4.4.1	NR − E-UTRAN FDD RSTD measurements	349

9.4.4.1.1	Introduction	349

9.4.4.1.2	Requirements	350

9.4.4.2	NR − E-UTRAN TDD RSTD measurements	353

9.4.4.2.1	Introduction	353

9.4.4.2.2	Requirements	354

9.4.5	Inter-RAT E-CID measurements	357

9.4.5.1	NR−E-UTRAN FDD E-CID RSRP and RSRQ measurements	357

9.4.5.1.1	Introduction	357

9.4.5.1.2	Requirements	357

9.4.5.1.3	Measurement Reporting Delay	357

9.4.5.2	NR−E-UTRAN TDD E-CID RSRP and RSRQ measurements	357

9.4.5.2.1	Introduction	357

9.4.5.2.2	Requirements	357

9.4.5.2.3	Measurement Reporting Delay	358

9.4.6	NR − UTRAN FDD measurements	358

9.4.6.1	Introduction	358

9.4.6.2	Requirements when no DRX is used	358

9.4.6.3	Requirements when DRX is used	359

9.4.7	NR – E-UTRAN measurements with autonomous gaps	361

9.4.7.1	CGI identification of an E-UTRA cell with autonomous gaps	361

9.4.7.2	CGI reporting delay	361

9.5	L1-RSRP measurements for Reporting	362

9.5.1	Introduction	362

9.5.2	Requirements applicability	362

9.5.3	Measurement Reporting Requirements	362

9.5.3.1	Periodic Reporting	363

9.5.3.2	Semi-Persistent Reporting	363

9.5.3.3	Aperiodic Reporting	363

9.5.4	L1-RSRP measurement requirements	363

9.5.4.1	SSB based L1-RSRP Reporting	363

9.5.4.2	CSI-RS based L1-RSRP Reporting	365

9.5.4A	Void	368

9.5.4A.1	Void	368

9.5.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	368

9.5.5.1	Measurement restriction for SSB based L1-RSRP	368

9.5.5.2	Measurement restriction for CSI-RS based L1-RSRP	368

9.5.6	Scheduling availability of UE during L1-RSRP measurement	369

9.5.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	369

9.5.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	369

9.5.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	370

9.5.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	370

9.5A	L1-RSRP measurements for Reporting under CCA	371

9.5A.1	Introduction	371

9.5A.2	Requirements applicability	371

9.5A.3	Measurement Reporting Requirements	371

9.5A.3.1	Periodic Reporting	371

9.5A.3.2	Semi-Persistent Reporting	372

9.5A.3.3	Aperiodic Reporting	372

9.5A.4	L1-RSRP measurement requirements	372

9.5A.4.1	SSB based L1-RSRP Reporting	372

9.5A.5	Measurement restriction for L1-RSRP measurement	373

9.5A.5.1	Measurement restriction for SSB based L1-RSRP	373

9.5A.6	Scheduling availability of UE during L1-RSRP measurement	373

9.5A.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH	373

9.5A.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH	374

9.5A.6.3	Scheduling availability of UE performing L1-RSRP measurement in case of FR1-FR2 inter-band CA	374

9.6	NE-DC: Measurements	374

9.6.1	Introduction	374

9.6.2	SFTD Measurements	374

9.6.2.1	Introduction	374

9.6.2.2	SFTD Measurement requirements	374

9.7	Cross Link Interference measurements	375

9.7.1	Introduction	375

9.7.2	SRS-RSRP measurements	375

9.7.2.1	Introduction	375

9.7.2.2	Requirements applicability	375

9.7.2.3	Measurement Reporting Requirements	376

9.7.2.3.1	Periodic Reporting	376

9.7.2.3.2	Event-triggered Periodic Reporting	376

9.7.2.3.3	Event Triggered Reporting	376

9.7.2.4	Measurement capability	376

9.7.2.5	SRS-RSRP measurement period	376

9.7.3	CLI-RSSI measurements	377

9.7.3.1	Introduction	377

9.7.3.2	Requirements applicability	377

9.7.3.3	Measurement Reporting Requirements	377

9.7.3.3.1	Periodic Reporting	377

9.7.3.3.2	Event-triggered Periodic Reporting	377

9.7.3.3.3	Event Triggered Reporting	377

9.7.3.4	Measurement capability	378

9.7.3.5	CLI-RSSI measurement period	378

9.7.4	Scheduling availability of UE during CLI measurements	378

9.7.4.1	Scheduling availability of UE performing measurement on FR1	378

9.7.4.2	Scheduling availability of UE performing measurement on FR2	378

9.8	L1-SINR measurements for Reporting	379

9.8.1	Introduction	379

9.8.2	Requirements applicability	379

9.8.3	Measurement Reporting Requirements	380

9.8.3.1	Periodic Reporting	380

9.8.3.2	Semi-Persistent Reporting	380

9.8.3.3	Aperiodic Reporting	381

9.8.4	L1-SINR measurement requirements	381

9.8.4.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	381

9.8.4.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	384

9.8.5	Measurement restriction for L1-SINR measurement	386

9.8.5.1	Measurement restriction if SSB configured for L1-SINR Measurement	386

9.8.5.2	Measurement restriction if CSI-RS configured for L1-SINR measurement	387

9.8.5.3	Measurement restriction if CSI-IM configured for L1-SINR measurement	387

9.8.6	Scheduling availability of UE during L1-SINR measurement	388

9.8.6.1	Scheduling availability of UE performing L1-SINR measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	388

9.8.6.2	Scheduling availability of UE performing L1-SINR measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	388

9.8.6.4	Scheduling availability of UE performing L1-SINR measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	389

9.9	 NR measurements for positioning	389

9.9.1	Introduction	389

9.9.2	RSTD measurements	390

9.9.2.1	Introduction	390

9.9.2.2	Requirements Applicability	390

9.9.2.3	Measurement Capability	390

9.9.2.4	Measurement Reporting Requirements	390

9.9.2.4.1	Void	390

9.9.2.4.2	Void	390

9.9.2.4.3	Void	390

9.9.2.5	Measurements Period Requirements	390

9.9.2.6	Void	392

9.9.3	PRS-RSRP measurements	392

9.9.3.1	Introduction	392

9.9.3.2	Requirements applicability	392

9.9.3.3	Measurement Capability	393

9.9.3.4	Measurement Reporting Requirements	393

9.9.3.5	Measurement Period Requirements	393

9.9.4	UE Rx-Tx time difference measurements	395

9.9.4.1 Introduction	395

9.9.4.2 Requirements Applicability	395

9.9.4.3 Measurement Capability	395

9.9.4.4 Measurement Reporting Requirements	395

9.9.4.5	Measurement Period Requirements	395

9.9.5	NR E-CID measurements	398

9.9.5.1	Introduction	398

9.9.5.2	Measurement Requirements	398

9.9.5.2.1	Intra-frequency Measurement Requirements	398

9.9.5.2.2	Inter-frequency Measurement Requirements	398

9.9.5.2.3	Measurement Reporting Delay	399

9.10	CSI-RS based L3 measurements	399

9.10.1	Introduction	399

9.10.2	CSI-RS based intra-frequency measurements	399

9.10.2.1	Introduction	399

9.10.2.2	Requirements applicability	400

9.10.2.3	Number of cells and number of CSI-RS	401

9.10.2.3.1	Requirements for FR1	401

9.10.2.3.2	Requirements for FR2	401

9.10.2.4	Measurement Reporting Requirements	401

9.10.2.4.1	Periodic Reporting	402

9.10.2.4.2	Event-triggered Periodic Reporting	402

9.10.2.4.3	Event Triggered Reporting	402

9.10.2.5	Intra-frequency measurements without measurement gaps	402

9.10.2.6	Scheduling availability of UE during CSI-RS based intra-frequency measurements	403

9.10.2.6.1	Scheduling availability of UE performing CSI-RS based measurements in TDD bands	403

9.10.2.6.2	Scheduling availability of UE performing CSI-RS based measurements in FR2	404

9.10.3	CSI-RS based Inter-frequency measurements	404

9.10.3.1	Introduction	404

9.10.3.2	Requirements applicability	404

9.10.3.3	Number of cells and number of CSI-RS resources	405

9.10.3.3.1	Requirements for FR1	405

9.10.3.3.2	Requirements for FR2	405

9.10.3.4	Measurements reporting requirements	405

9.10.3.4.1	Periodic Reporting	405

9.10.3.4.2	Event-triggered Periodic Reporting	405

9.10.3.4.3	Event-triggered Reporting	406

9.10.3.5	Inter frequency measurements with measurement gaps	406

9.11	NR measurements with autonomous gaps	407

9.11.1	Introduction	407

9.11.2	CGI identification of an NR cell with autonomous gaps	407

9.11.3	CGI reporting delay	408

10	Measurement Performance requirements	408

10.1	NR measurements	408

10.1.1	Introduction	408

10.1.2	Intra-frequency RSRP accuracy requirements for FR1	409

10.1.2.1	Intra-frequency SS-RSRP accuracy requirements	409

10.1.2.1.1	Absolute SS-RSRP Accuracy	409

10.1.2.1.2	Relative SS-RSRP Accuracy	410

10.1.2.2	Void	411

10.1.2.3	Intra-frequency CSI-RSRP accuracy requirements	411

10.1.2.3.1	Absolute CSI-RSRP Accuracy	411

10.1.2.3.2	Relative CSI-RSRP Accuracy	412

10.1.2B	Intra-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	413

10.1.2B.1	Intra-frequency SS-RSRP accuracy requirements	413

10.1.2B.1.1	Absolute SS-RSRP Accuracy	413

10.1.3	Intra-frequency RSRP accuracy requirements for FR2	414

10.1.3.1	Intra-frequency SS-RSRP accuracy requirements	414

10.1.3.1.1	Absolute SS-RSRP Accuracy	414

10.1.3.1.2	Relative SS-RSRP Accuracy	415

10.1.3.2	Void	416

10.1.3.3	Intra-frequency CSI-RSRP accuracy requirements	416

10.1.3.3.1	Absolute CSI-RSRP Accuracy	416

10.1.3.3.2	Relative CSI-RSRP Accuracy	416

10.1.3B	Intra-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	417

10.1.3B.1	Intra-frequency SS-RSRP accuracy requirements	417

10.1.3B.1.1	Absolute SS-RSRP Accuracy	418

10.1.4	Inter-frequency RSRP accuracy requirements for FR1	418

10.1.4.1	Inter-frequency SS-RSRP accuracy requirements	418

10.1.4.1.1	Absolute Accuracy of SS-RSRP in FR1	418

10.1.4.1.2	Relative Accuracy of SS-RSRP in FR1	419

10.1.4.2	Void	420

10.1.4.3	Inter-frequency CSI-RSRP accuracy requirements	420

10.1.4.3.1	Absolute Accuracy of CSI-RSRP in FR1	420

10.1.4.3.2	Relative Accuracy of CS-RSRP in FR1	421

10.1.4B	Inter-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	422

10.1.4B.1	Inter-frequency SS-RSRP accuracy requirements	422

10.1.4B.1.1	Absolute Accuracy of SS-RSRP in FR1	422

10.1.5	Inter-frequency RSRP accuracy requirements for FR2	423

10.1.5.1	Inter-frequency SS-RSRP accuracy requirements	423

10.1.5.1.1	Absolute SS-RSRP Accuracy	423

10.1.5.1.2	Relative SS-RSRP Accuracy	424

10.1.5.2	Void	425

10.1.5.3	Inter-frequency CSI-RSRP accuracy requirements	425

10.1.5.3.1	Absolute CSI-RSRP Accuracy	425

10.1.5.3.2	Relative CSI-RSRP Accuracy	425

10.1.5B	Inter-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	426

10.1.6 RSRP Measurement Report Mapping	427

10.1.7	Intra-frequency RSRQ accuracy requirements for FR1	429

10.1.7.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	429

10.1.7.1.1	Absolute SS-RSRQ Accuracy in FR1	429

10.1.7.2	Intra-frequency CSI-RSRQ accuracy requirements	430

10.1.7.2.1	Absolute CSI-RSRQ Accuracy	430

10.1.7B	Intra-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	431

10.1.7B.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	431

10.1.7B.1.1	Absolute SS-RSRQ Accuracy in FR1	431

10.1.8	Intra-frequency RSRQ accuracy requirements for FR2	432

10.1.8.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	432

10.1.8.1.1	Absolute SS-RSRQ Accuracy in FR2	432

10.1.8.2	Intra-frequency CSI-RSRQ accuracy requirements	433

10.1.8.2.1	Absolute CSI-RSRQ Accuracy	433

10.1.8B	Intra-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	434

10.1.8B.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	434

10.1.8B.1.1	Absolute SS-RSRQ Accuracy in FR2	434

10.1.9	Inter-frequency RSRQ accuracy requirements for FR1	435

10.1.9.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	435

10.1.9.1.1	Absolute Accuracy of SS-RSRQ in FR1	435

10.1.9.1.2	Relative Accuracy of SS-RSRQ in FR1	436

10.1.9.2	Inter-frequency CSI-RSRQ accuracy requirements	437

10.1.9.2.1	Absolute CSI-RSRQ Accuracy	437

10.1.9.2.2	Relative CSI-RSRQ Accuracy	438

10.1.9B	Inter-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	439

10.1.9B.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	439

10.1.9B.1.1	Absolute Accuracy of SS-RSRQ in FR1	439

10.1.10	Inter-frequency RSRQ accuracy requirements for FR2	440

10.1.10.2	Inter-frequency CSI-RSRQ accuracy requirements	442

10.1.10.2.1	Absolute CSI-RSRQ Accuracy	442

10.1.10.2.2	Relative CSI-RSRQ Accuracy	442

10.1.10B	Inter-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	443

10.1.10B.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	443

10.1.10B.1.1	Absolute Accuracy of SS-RSRQ in FR2	443

10.1.11	RSRQ report mapping	444

10.1.12	Intra-frequency SINR accuracy requirements for FR1	445

10.1.12.2	Intra-frequency CSI-SINR accuracy requirements in FR1	445

10.1.12.2.1	Absolute CSI-SINR Accuracy in FR1	445

10.1.13	Intra-frequency SINR accuracy requirements for FR2	446

10.1.13.2	Intra-frequency CSI-SINR accuracy requirements in FR2	447

10.1.13.2.1	Absolute CSI-SINR Accuracy in FR2	447

10.1.14	Inter-frequency SINR accuracy requirements for FR1	448

10.1.14.2	Inter-frequency CSI-SINR accuracy requirements in FR1	450

10.1.14.2.1	Aboslute Accuracy of CSI-SINR in FR1	450

10.1.15	Inter-frequency SINR accuracy requirements for FR2	452

10.1.15.2	Inter-frequency CSI-SINR accuracy requirements in FR2	454

10.1.15.2.1	Aboslute Accuracy of CSI-SINR in FR2	454

10.1.15.2.2	Relative Accuracy of CSI-SINR in FR2	454

10.1.16	SINR report mapping	455

10.1.17	Power Headroom	456

10.1.18	PCMAX,c,f	457

10.1.19	L1-RSRP accuracy requirements for FR1	457

10.1.20	L1-RSRP accuracy requirements for FR2	461

10.1.21	SFTD accuracy requirements	464

10.1.22	CLI measurement accuracy requirements	468

10.1.24.3.1	Absolute PRS-RSRP Measurement Report Mapping	472

10.1.24.3.2	Differential Report Mapping for PRS-RSRP Measurement	472

10.1.23	RSTD Measurements	474

10.1.23.1	Introduction	474

10.1.23.2	Measurement Accuracy Requirements	475

10.1.23.3	Report mapping	481

10.1.23.3.1	 Absolute DL RSTD Measurement Reporting	481

10.1.23.3.2	 Differential Reporting for DL RSTD Measurement	483

10.1.23.3.3	Additional Path Report Mapping for DL RSTD	485

10.1.24	PRS-RSRP Measurements	487

10.1.24.1	Introduction	487

10.1.24.2	Measurement Accuracy Requirements	487

10.1.24.2.1 Absolute PRS RSRP accuracy	487

10.1.24.3	Report mapping	491

10.1.24.3.1	Absolute PRS-RSRP Measurement Report Mapping	491

10.1.24.3.2	Differential Report Mapping for PRS-RSRP Measurement	492

10.1.25	UE Rx-Tx Time Difference Measurements	494

10.1.25.1	Introduction	494

10.1.25.2	Measurement Accuracy Requirements	495

10.1.25.3	Report mapping	501

10.1.25.3.1	 Absolute UE Rx-Tx Measurement Report Mapping	501

10.1.25.3.2	 Differential UE Rx-Tx Measurement Report Mapping	503

10.1.25.3.3	 Additional Path Report Mapping for UE Rx-Tx Time Difference	504

10.1.26	FR2 P-MPR report	506

10.1.26.1	Report mapping	506

10.1.27	L1-SINR accuracy requirements for FR1	506

10.1.27.1	L1-SINR accuracy requirements with CSI-RS based CMR and no dedicated IMR configured	506

10.1.27.1.1	Absolute Accuracy	506

10.1.27.1.2	Relative Accuracy	507

10.1.27.2	L1-SINR accuracy requirements with SSB based CMR and dedicated IMR configured	508

10.1.27.2.1	Absolute Accuracy	508

10.1.27.2.2	Relative Accuracy	509

10.1.27.3	L1-SINR accuracy requirements with CSI-RS based CMR and dedicated IMR configured	511

10.1.27.3.1	Absolute Accuracy	511

10.1.27.3.2	Relative Accuracy	513

10.1.28	L1-SINR accuracy requirements for FR2	514

10.1.28.1	L1-SINR accuracy requirements with CSI-RS based CMR and no dedicated IMR configured	514

10.1.28.1.1	Absolute Accuracy	514

10.1.28.1.2	Relative Accuracy	515

10.1.28.2	L1-SINR accuracy requirements with SSB based CMR and dedicated IMR configured	516

10.1.28.2.1	Absolute Accuracy	516

10.1.28.2.2	Relative Accuracy	517

10.1.28.3	L1-SINR accuracy requirements with CSI-RS based CMR and dedicated IMR configured	518

10.1.28.3.1	Absolute Accuracy	518

10.1.28.3.2	Relative Accuracy	519

10.1.29	Intra-frequency RSRQ accuracy requirements under CCA	521

10.1.29.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	521

10.1.29.1.1	Absolute SS-RSRQ Accuracy	521

10.1.30	Inter-frequency RSRQ accuracy requirements under CCA	522

10.1.30.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	522

10.1.30.1.1	Aboslute Accuracy of SS-RSRQ	522

10.1.30.1.2	Relative Accuracy of SS-RSRQ	522

10.1.31	Intra-frequency SINR accuracy requirements under CCA	523

10.1.32	Inter-frequency SINR accuracy requirements under CCA	524

10.1.33	L1-RSRP accuracy requirements under CCA	525

10.1.34	RSSI measurements under CCA	526

10.1.34.1	Intra-frequency absolute RSSI measurement accuracy requirements in FR1	526

10.1.34.2	Inter-frequency absolute RSSI measurement accuracy requirements in FR1	527

10.1.34.3	RSSI measurement report mapping	527

10.1.35	Channel occupancy measurements under CCA	527

10.1.35.1	Intra-frequency channel occupancy measurement accuracy requirements in FR1	527

10.1.35.2	Inter-frequency channel occupancy measurement accuracy requirements in FR1	528

10.1.36	Intra-frequency RSRP accuracy requirements under CCA	528

10.1.36.1	Intra-frequency SS-RSRP accuracy requirements in FR1	528

10.1.36.1.1	Absolute SS-RSRP Accuracy	528

10.1.36.1.2	Relative SS-RSRP Accuracy	529

10.1.37	Inter-frequency RSRP accuracy requirements under CCA	529

10.1.37.1	Inter-frequency SS-RSRP accuracy requirements in FR1	529

10.1.37.1.1	Absolute Accuracy of SS-RSRP	529

10.1.37.1.2	Relative Accuracy of SS-RSRP	530

10.2	E-UTRAN measurements	530

10.2.1	Introduction	530

10.2.2	E-UTRAN RSRP measurements	531

10.2.3	E-UTRAN RSRQ measurements	531

10.2.4	E-UTRAN RSTD measurements	531

10.2.5	E-UTRAN RS-SINR measurements	532

10.2.6	E-UTRAN RSRP measurements for CA/DC Idle Mode Measurements	532

10.2.7	E-UTRAN RSRQ measurements for CA/DC Idle Mode Measurements	532

10.3	UTRAN FDD Measurements	532

10.3.1	UTRAN FDD CPICH RSCP	533

10.3.2	UTRAN FDD CPICH Ec/No	533

10.4	V2X measurements	534

10.4.1	Introduction	534

10.4.2	Intra-frequency PSBCH-RSRP accuracy requirements for FR1	534

10.4.2.1	PSBCH-RSRP Absolute Accuracy	534

10.4.2.2	PSBCH-RSRP Relative Accuracy	534

10.4.3	Intra-Frequency SL-RSSI Measurement Accuracy Requirements for FR1	535

10.4.3.1	Absolute SL-RSSI Accuracy	535

10.4.4	Intra-Frequency L1 SL-RSRP Measurement Accuracy Requirements for FR1	536

10.4.4.1	Absolute L1 SL-RSRP Accuracy	536

11	Void	537

12	V2X Requirements	537

12.1	Introduction	537

12.2	UE Transmit Timing	537

12.2.1	Introduction	537

12.2.2	GNSS as synchronization reference source	537

12.2.3	NR Cell as synchronization reference source	538

12.2.4	E-URTAN Cell as synchronization reference source	538

12.2.5	SyncRef UE as synchronization reference source	538

12.3	Initiation/Cease of SLSS Transmissions	539

12.3.1	Introduction	539

12.3.1.1	Initiation/Cease of SLSS transmissions with NR cell as synchronization reference source	539

12.3.1.2	Initiation/Cease of SLSS transmissions with EUTRAN cell as synchronization reference source	540

12.3.1.3	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	541

12.3.1.4	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	541

12.4	Selection / Reselection of V2X Synchronization Reference Source	541

12.5	L1 SL-RSRP measurements	542

12.5.1	Introduction	542

12.5.2	SL-RSRP measurements	542

12.6	Congestion Control measurements	543

12.7	Interruption	543

12.7.1	Interruptions to WAN due to V2X Sidelink Communication	543

12.7.2	V2X Sidelink Communication Dropping due to synchronization source change	543

12.7.3	Interruptions to WAN due to switching between E-UTRA V2X Sidelink and NR V2X Sidelink	545

12.8	Reliability of GNSS signal	545

12.9	Scheduling availability	546

12.9.1	Scheduling availability of UE switching between E-UTRA sidelink and NR sidelink	546

13	Measurement Performance Requirements for NR gNB	546

13.1	UL-RTOA	546

13.1.1	Report mapping	546

13.2	gNB Rx-Tx time difference	548

13.2.1	Report mapping	548

13.2.2	Measurement Accuracy Requirements	550

13.2.2.1	Introduction	550

13.2.2.2	Requirements	551

13.3	UL SRS RSRP measurement	552

13.3.1	Report mapping	552

13.3.2	Measurement accuracy requirements	552

13.3.2.1	Introduction	552

13.3.2.2	Requirements	553

13.4	AoA/ZoA	554

13.4.1	Report mapping	554

Annex A	(normative): Test Cases	555

A.1	Purpose of annex	555

A.2	Requirement classification for statistical testing	555

A.2.1	Types of requirements in TS 38.133	555

A.2.1.1	Time and delay requirements on UE higher layer actions	555

A.2.1.2	Measurements of power levels, relative powers and time	555

A.2.1.3	Implementation requirements	556

A.2.1.4	Physical layer timing requirements	556

A.2.1.5	Requirements under CCA	556

A.3	RRM test configurations	557

A.3.1	Reference measurement channels	557

A.3.1.1	PDSCH	557

A.3.1.1.1	FDD	557

A.3.1.1.2	TDD	558

A.3.1A	Reference measurement channels under CCA	561

A.3.1A.1	PDSCH	561

A.3.1A.1.1	TDD	561

A.3.1A.2	CORESET for RMSI scheduling	562

A.3.1A.2.1	TDD	562

A.3.1A.3	CORESET for RMC scheduling	563

A.3.1A.3.1	TDD	563

A.3.1A.4	TDD UL/DL configuration	564

A.3.1A.5	RMC burst transmission model	564

A.3.1.2	CORESET for RMSI scheduling	565

A.3.1.2.1	FDD	565

A.3.1.2.2	TDD	566

A.3.1.3	CORESET for RMC scheduling	569

A.3.1.3.1	FDD	569

A.3.1.3.2	TDD	570

A.3.1.4	TDD UL/DL configuration	573

A.3.2	OFDMA channel noise generator (OCNG)	574

A.3.2.1	Generic OFDMA Channel Noise Generator (OCNG)	574

A.3.2.1.1	OCNG pattern 1: Generic OCNG pattern for all unused REs	574

A.3.2.1.2	OCNG pattern 2: Generic OCNG pattern for all unused REs for 2AoA setup	575

A.3.2.1.3	OCNG pattern 3: Generic OCNG pattern for unused REs in the same bandwidth as CORESET	575

A.3.2.1.4	OCNG pattern 4: Generic OCNG pattern for all unused REs outside SSB slot(s)	576

A.3.2.2	Void	577

A.3.3	Reference DRX configurations	577

A.3.3.1	DRX Configuration 1: DRX cycle = 40 ms and TAT = 500 ms	577

A.3.3.2	DRX Configuration 2: DRX cycle = 640 ms and TAT = 500 ms	577

A.3.3.3	DRX Configuration 3: DRX cycle = 40 ms and TAT = Infinity	577

A.3.3.4	DRX Configuration 4: DRX cycle = 160 ms and TAT = Infinity	578

A.3.3.5	DRX Configuration 5: DRX cycle = 320 ms and TAT = Infinity	578

A.3.3.6	DRX Configuration 6: DRX cycle = 320 ms and TAT = 500 ms	578

A.3.3.7	DRX Configuration 7: DRX cycle = 640 ms and TAT = Infinity	579

A.3.3.8	DRX Configuration 8: DRX cycle = 320 ms and TAT = Infinity	579

A.3.3.9	DRX Configuration 9: DRX cycle = 40 ms and TAT = 500 ms	579

A.3.3.10	DRX Configuration 10: DRX cycle = 640 ms and TAT = 500 ms	580

A.3.3.11	DRX Configuration 11: DRX cycle = 20 ms and TAT = Infinity	580

A.3.3.12	DRX Configuration 12: DRX cycle = 640 ms and TAT = Infinity	580

A.3.4	Test Cases with Different Channel Bandwidths	580

A.3.4.1	Test Cases with Different E-UTRA Channel Bandwidths	580

A.3.4.1.1	Introduction	580

A.3.4.1.2	Principle of testing	581

A.3.5	Test Cases for Synchronous and Asynchronous DC Operations	581

A.3.5.1	EN-DC Test Cases for Synchronous and Asynchronous EN-DC Operations	581

A.3.5.1.1	Introduction	581

A.3.5.1.2	Principle of Testing	581

A.3.6	Antenna configurations	581

A.3.6.1	Antenna configurations for FR1	581

A.3.6.1.1	Antenna connection for 4 Rx capable UEs	581

A.3.6.1.1.1	Introduction	581

A.3.6.1.1.2	Principle of testing	581

A.3.6.2	Antenna configurations for FR2	584

A.3.6A	Antenna configurations with unlicensed bands	584

A.3.6A.1	Antenna configurations for FR1	584

A.3.6A.1.1	Antenna connection for 4 Rx capable UEs	584

A.3.6A.1.1.1	Introduction	584

A.3.6A.1.1.2	Principle of testing	584

A.3.7	EN-DC test setup	586

A.3.7.1	Introduction	586

A.3.7.2	E-UTRAN Serving Cell Parameters	586

A.3.7.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	586

A.3.7.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	587

A.3.7A	NR FR1-FR2 test setup	588

A.3.7B	EN-DC test setup with unlicensed bands	589

A.3.7B.1	Introduction	589

A.3.7B.2	E-UTRAN Serving Cell Parameters	589

A.3.7B.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) under CCA in FR1	589

A.3.7C	LTE-FR1/FR2 test setup	590

A.3.7D	NE-DC test setup	590

A.3.7D.1	Introduction	590

A.3.7D.2	E-UTRAN Serving Cell Parameters	590

A.3.7D.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	590

A.3.7D.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	590

A.3.8	PRACH configurations	590

A.3.8.1	Introduction	590

A.3.8.2	PRACH configurations in FR1	591

A.3.8.2.1	FR1 PRACH configuration 1	591

A.3.8.2.2	FR1 PRACH configuration 2	591

A.3.8.2.3	FR1 PRACH configuration 3	592

A.3.8.2.4	FR1 PRACH configuration 4	593

A.3.8.3	PRACH configurations in FR2	594

A.3.8.3.1	FR2 PRACH configuration 1	594

A.3.8.3.2	FR2 PRACH configuration 2	595

A.3.8.3.3	FR2 PRACH configuration 3	596

A.3.8.3.4	FR2 PRACH configuration 4	597

A.3.8A	PRACH configurations under CCA	598

A.3.8A.1	Introduction	598

A.3.8A.2	PRACH configurations in FR1	598

A.3.8A.2.1	FR1 PRACH configuration 1 under CCA	598

A.3.8A.2.2	FR1 PRACH configuration 2 under CCA	599

A.3.9	BWP configurations	600

A.3.9.1	Introduction	600

A.3.9.2	Downlink BWP configurations	601

A.3.9.2.1	Initial BWP	601

A.3.9.2.2	Dedicated BWP	601

A.3.9.3	Uplink BWP configurations	602

A.3.9.3.1	Initial BWP	602

A.3.9.3.2	Dedicated BWP	602

A.3.10	SSB Configurations	603

A.3.10.1	SSB Configurations for FR1	603

A.3.10.1.1	SSB pattern 1 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	603

A.3.10.1.5	SSB pattern 5 in FR1: SSB allocation for SSB SCS=15 kHz starting from odd SFN in 10 MHz	605

A.3.10A	SSB Configurations under CCA	610

A.3.10A.1	SSB Configurations under CCA for FR1	610

A.3.10A.1.1	SSB pattern 1 under CCA for semi-static channel access: SSB allocation for SSB SCS=30kHz in 40MHz	610

A.3.10A.1.2	SSB pattern 2 under CCA for dynamic channel access: SSB allocation for SSB SCS=30kHz in 40MHz	610

A.3.10A.1.3	SSB pattern 3 under CCA for semi-static channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	611

A.3.10A.1.4	SSB pattern 4 under CCA for dynamic channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	611

A.3.11	SMTC Configurations	612

A.3.11.1	SMTC pattern 1: SMTC period = 20 ms with SMTC duration = 1 ms	612

A.3.11.2	SMTC pattern 2: SMTC period = 20 ms with SMTC duration = 5 ms	612

A.3.11.3	SMTC pattern 3: SMTC period = 160 ms with SMTC duration = 1 ms	612

A.3.11.4	SMTC pattern 4: SMTC period = 20 ms with SMTC duration = 1 ms	612

A.3.11.5	SMTC pattern 5: SMTC period = 20 ms with SMTC duration = 5 ms	613

A.3.11.6	SMTC pattern 6: SMTC period = 20 ms with SMTC duration = 5 ms	613

A.3.11.7	Void	613

A.3.11.8	Void	613

A.3.11.9	SMTC pattern 9: SMTC period = 20 ms with SMTC duration = 1 ms	613

A.3.12	Test Cases with Different CC Configurations	613

A.3.12.1 EN-DC Test Cases with Different EN-DC Configurations	613

A.3.12.1.1	Introduction	613

A.3.12.1.2	Principle of testing	613

A.3.12.2	Carrier Aggregation Test Cases with Different CA Configurations	614

A.3.12.2.1	Introduction	614

A.3.12.2.2	Principle of testing	614

A.3.13	Test Cases in SA and EN-DC Operations	614

A.3.13.1	Introduction	614

A.3.13.2	Principle of Testing	614

A.3.13A Test Cases involving E-UTRA/FR1 and FR2 carriers	614

A.3.13A.1	Introduction	614

A.3.13A.2	Principle of Testing in EN-DC	615

A.3.13A.3	Principle of Testing in SA	615

A.3.13B	Test Cases for EN-DC and NE-DC Operations	616

A.3.13B.1	Active BWP switch Test Cases for EN-DC and NE-DC Operations	616

A.3.13B.1.1	Introduction	616

A.3.13B.1.2	Principle of Testing	617

A.3.13B.2	SFTD accuracy Test Cases for EN-DC and NE-DC Operations	617

A.3.13B.2.1	Introduction	617

A.3.13B.2.2	Principle of Testing	617

A.3.14	CSI-RS configurations	617

A.3.14.1	FDD	617

A.3.14.2	TDD	619

A.3.15	Angle of Arrival (AoA) for FR2 RRM test cases	624

A.3.15.1	Setup 1: Single AoA in Rx beam peak direction	624

A.3.15.2	Setup 2: Single AoA in non Rx beam peak direction	624

A.3.15.2.1	Setup 2a: Single AoA in non Rx beam peak direction without change in direction	624

A.3.15.2.2	Setup 2b: Single AoA in non Rx beam peak direction with change in direction	625

A.3.15.3	Setup 3: 2 AoAs	625

A.3.15.4	Setup 4: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak	625

A.3.15.4.1	Setup 4a: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak without change in direction	625

A.3.15.4.2	Setup 4b: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak with change in direction	625

A.3.16	TCI State Configuration	626

A.3.16.1	Introduction	626

A.3.16.2	TCI states	626

A.3.17	Configurations of CSI-RS for tracking	626

A.3.17.1	Configuration of CSI-RS for tracking for FR1	626

A.3.17.1.1	FDD	626

A.3.17.1.2	TDD	627

A.3.17.2	Configuration of CSI-RS for tracking for FR2	628

A.3.17.2.1	TDD	628

A.3.18	Additional definitions related to OTA testing for FR2 RRM test cases	629

A.3.18.1	Introduction	629

A.3.18.2	PRACH Power Measurement	629

A.3.19	Test applicability for DAPS handover	629

A.3.19.1	Introduction	629

A.3.19.2	Principle of testing	629

A.3.20	MsgA configurations	630

A.3.20.1	Introduction	630

A.3.20.2	MsgA configurations in FR1	630

A.3.20.2.1	FR1 MsgA configuration 1	630

A.3.20.2.2	FR1 MsgA configuration 2	631

A.3.20.3	MsgA configurations in FR2	633

A.3.20.3.1	FR2 MsgA configuration 1	633

A.3.20.3.2	FR2 MsgA configuration 2	634

A.3.20A	MsgA configurations under CCA	636

A.3.20A.1	Introduction	636

A.3.20A.2	MsgA configurations in FR1	636

A.3.20A.2.1	FR1 MsgA configuration 1 under CCA	636

A.3.20A.2.2	FR1 MsgA configuration 2 under CCA	637

A.3.21	V2X sidelink communication	640

A.3.21.1	Introduction	640

A.3.21.2	Reference resource pool configurations for V2X Sidelink Communication	640

A.3.21.3	Reference measurement channels for V2X Sidelink Communication	644

A.3.22	CSI-IM configurations	644

A.3.22.1	FDD	644

A.3.22.2	TDD	645

A.3.23	Spatial Relation Configuration	647

A.3.23.1	Introduction	647

A.3.23.2	Spatial Relation	647

A.3.24	SRS configuration	648

A.3.25	Channel bandwidth (CBW) configurations	650

A.3.25.1	DL UE specific CBW	651

A.3.26	CCA model	651

A.3.26.1	Introduction	651

A.3.26.2	CCA model for operation on a carrier frequency with CCA in FR1	651

A.3.26.2.1	DL CCA model	651

A.3.26.2.2	UL CCA model	653

A.3.27	Test Cases with at Least One Cell on a Carrier Frequency with CCA	653

A.3.27.1	Introduction	654

A.3.27.2	 NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	654

A.3.27.3	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	654

A.3.27.4	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	654

A.3.27.5	E-UTRA Standalone Tests with at Least One NR Cell under CCA	654

A.3.28	Discovery Burst Transmission Window configuration under CCA	654

A.3.28.1	DBT Window pattern 1: DBT Window period = 20 ms with DBT Window duration = 1 ms	654

A.3.29	Testing principles for UE capable of only NR bands with shared spectrum access	654

A.3.29.1	Introduction	654

A.3.29.2	Principle of testing for UE capable of EN-DC with only NR bands with shared spectrum access	655

A.3.29.3	Principle of testing for UE capable of SA operation with only NR bands with shared spectrum access	655

A.3.30	CSI-RS configurations for RRM	656

A.3.30.1	FDD	656

A.3.30.2	TDD	657

A.3.31	PRS Configurations	659

A.3.31.1.	PRS Configurations for FR1	659

A.3.31.1.1.	PRS pattern 1 in FR1: SCS=15 KHz	659

A.3.31.1.2.	PRS pattern 2 in FR1: SCS=30 KHz	660

A.3.31.2.	PRS Configurations for FR2	660

A.3.31.2.1.	PRS pattern 1 in FR2: SCS=120 KHz	660

A.4	EN-DC tests with all NR cells in FR1	661

A.4.1	Void	661

A.4.2	Void	661

A.4.3	RRC_CONNECTED state mobility	661

A.4.3.1	Void	661

A.4.3.2	RRC Connection Mobility Control	661

A.4.3.2.1	Void	661

A.4.3.2.2	Random Access	661

A.4.3.2.2.1	4-step RA type contention based random access test in FR1 for PSCell in EN-DC	661

A.4.3.2.2.2	4-step RA type non-contention based random access test in FR1 for PSCell in EN-DC	665

A.4.3.2.2.3	2-step RA type contention based random access test in FR1 for PSCell in EN-DC	669

A.4.3.2.2.4	2-step RA type non-contention based random access test in FR1 for PSCell in EN-DC	673

A.4.3.2.3	Void	677

A.4.4	Timing	677

A.4.4.1	UE transmit timing	677

A.4.4.1.1	NR UE Transmit Timing Test for FR1	677

A.4.4.1.1.1	Test Purpose and environment	677

A.4.4.1.1.2	Test requirements	681

A.4.4.2	UE timer accuracy	681

A.4.4.3	Timing advance	681

A.4.4.3.1	EN-DC FR1 timing advance adjustment accuracy	681

A.4.4.3.1.1	Test Purpose and Environment	681

A.4.4.3.1.2	Test Parameters	682

A.4.4.3.1.3	Test Requirements	685

A.4.5	Signaling characteristics	685

A.4.5.1	Radio link Monitoring	685

A.4.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	686

A.4.5.1.1.1	Test Purpose and Environment	686

A.4.5.1.1.2	Test Requirements	690

A.4.5.1.2	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	690

A.4.5.1.2.1	Test Purpose and Environment	690

A.4.5.1.2.2	Test Requirements	696

A.4.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	696

A.4.5.1.3.1	Test Purpose and Environment	696

A.4.5.1.3.2	Test Requirements	702

A.4.5.1.4	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	702

A.4.5.1.4.1	Test Purpose and Environment	702

A.4.5.1.4.2	Test Requirements	708

A.4.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	708

A.4.5.1.5.1	Test Purpose and Environment	708

A.4.5.1.5.2	Test Requirements	714

A.4.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	714

A.4.5.1.6.1	Test Purpose and Environment	714

A.4.5.1.6.2	Test Requirements	720

A.4.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	720

A.4.5.1.7.1	Test Purpose and Environment	720

A.4.5.1.7.2	Test Requirements	725

A.4.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	726

A.4.5.1.8.1	Test Purpose and Environment	726

A.4.5.1.8.2	Test Requirements	731

A.4.5.2	Interruption	731

A.4.5.2.1	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	731

A.4.5.2.1.1	Test Purpose and Environment	731

A.4.5.2.1.2	Test Requirements	735

A.4.5.2.2	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	735

A.4.5.2.2.1	Test Purpose and Environment	735

A.4.5.2.2.2	Test Requirements	739

A.4.5.2.3	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	739

A.4.5.2.3.1	Test Purpose and Environment	739

A.4.5.2.3.2	Test Requirements	744

A.4.5.2.4	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	745

A.4.5.2.4.1	Test Purpose and Environment	745

A.4.5.2.4.2	Test Requirements	749

A.4.5.2.5	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	750

A.4.5.2.5.1	Test Purpose and Environment	750

A.4.5.2.5.2	Test Requirements	754

A.4.5.2.6	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	754

A.4.5.2.6.1	Test Purpose and Environment	754

A.4.5.2.6.2	Test Requirements	758

A.4.5.2.7	Void	758

A.4.5.2.8	E-UTRAN - NR FR1 interruptions at NR SRS carrier based switching in asynchronous EN-DC	758

A.4.5.2.8.1	Test Purpose and Environment	758

A.4.5.2.8.2	Test Requirements	761

A.4.5.2.9	E-UTRAN – NR interruptions at E-UTRA SRS carrier based switching	762

A.4.5.2.9.1	Test Purpose and Environment	762

A.4.5.2.9.2	Test Requirements	766

A.4.5.3	SCell Activation and Deactivation Delay	767

A.4.5.3.1	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	767

A.4.5.3.1.1	Test Purpose and Environment	767

A.4.5.3.1.2	Test Requirements	773

A.4.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 640ms SCell measurement cycle	774

A.4.5.3.2.1	Test Purpose and Environment	774

A.4.5.3.2.2	Test Requirements	774

A.4.5.3.3	SCell Activation and deactivation of unknown SCell in FR1	774

A.4.5.3.3.1	Test Purpose and Environment	774

A.4.5.3.3.2	Test Requirements	775

A.4.5.3.4	SCell Activation and deactivation of multiple unknown SCells in FR1 with single activation/deactivation command	776

A.4.5.3.4.1	Test Purpose and Environment	776

A.4.5.3.4.2	Test Requirements	780

A.4.5.3.5	Direct SCell activation at SCell addition of known SCell in FR1	780

A.4.5.3.5.1	Test Purpose and Environment	780

A.4.5.3.5.2	Test Requirements	788

A.4.5.4	UE UL carrier RRC reconfiguration Delay	789

A.4.5.4.1	UE UL carrier RRC reconfiguration Delay	789

A.4.5.4.1.1	Test Purpose and Environment	789

A.4.5.4.1.2	Test Requirements	796

A.4.5.5	Beam Failure Detection and Link recovery procedures	796

A.4.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	796

A.4.5.5.1.1	Test Purpose and Environment	796

A.4.5.5.1.2	Test Requirements	802

A.4.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	803

A.4.5.5.2.1	Test Purpose and Environment	803

A.4.5.5.2.2	Test Requirements	808

A.4.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	809

A.4.5.5.3.1	Test Purpose and Environment	809

A.4.5.5.3.2	Test Requirements	814

A.4.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in DRX mode	815

A.4.5.5.4.1	Test Purpose and Environment	815

A.4.5.5.4.2	Test Requirements	820

A.4.5.5.5	EN-DC Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	821

A.4.5.5.5.1	Test Purpose and Environment	821

A.4.5.5.5.2	Test Requirements	826

A.4.5.5.6	EN-DC Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode	827

A.4.5.5.6.1	Test Purpose and Environment	827

A.4.5.5.6.2	Test Requirements	833

A.4.5.6	Active BWP switch	834

A.4.5.6.1	DCI-based and Timer-based Active BWP Switch	834

A.4.5.6.1.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	834

A.4.5.6.1.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	839

A.4.5.6.2	RRC-based Active BWP Switch	845

A.4.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	849

A.4.5.6.3.1	Simultaneous E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in EN-DC on multiple CCs	849

A.4.5.6.3.1.2	Test Requirements	855

A.4.5.6.4	SCell dormancy switch	855

A.4.5.6.4.1	E-UTRAN – NR FR1 PSCell SCell dormancy switch of single FR1 SCell outside active time	855

A.4.5.6.4.1.1	Test Purpose and Environment	855

A.4.5.6.4.1.2	Test Requirements	863

A.4.5.6.4.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR1 SCells inside active time	863

A.4.5.6.4.2.1	Test Purpose and Environment	863

A.4.5.6.4.2.2	Test Requirements	875

A.4.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	876

A.4.5.6.5.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC on multiple CCs	876

A.4.5.7	PSCell addition and release delay	880

A.4.5.7.1	Addition and Release Delay of known NR PSCell	880

A.4.5.7.1.1	Test purpose and environment	880

A.4.5.7.1.2	Test Requirements	884

A.4.5.8	DL Interruptions at switching between two uplink carriers	885

A.4.5.8.1	Test Purpose and Environment	885

A.4.5.8.2	Test Requirements	888

A.4.5.9	UE specific CBW change	889

A.4.5.9.1	UE specific CBW change on FR1 NR PSCell with non-DRX in synchronous EN- DC	889

A.4.5.9.1.1	Test Purpose and Environment	889

A.4.5.9.1.2	Test Requirements	893

A.4.6	Measurement procedure	893

A.4.6.1	Intra-frequency Measurements	893

A.4.6.1.1	EN-DC event triggered reporting tests without gap under non-DRX	893

A.4.6.1.1.1	Test purpose and Environment	893

A.4.6.1.1.2	Test parameters	893

A.4.6.1.1.3	Test Requirements	897

A.4.6.1.2	EN-DC event triggered reporting tests without gap under DRX	897

A.4.6.1.2.1	Test purpose and Environment	897

A.4.6.1.2.2	Test parameters	897

A.4.6.1.2.2	Test Requirements	901

A.4.6.1.3	EN-DC event triggered reporting tests with per-UE gaps under non-DRX	901

A.4.6.1.3.1	Test purpose and Environment	901

A.4.6.1.3.2	Test parameters	901

A.4.6.1.3.3	Test Requirements	905

A.4.6.1.4	EN-DC event triggered reporting tests with per-UE gaps under DRX	905

A.4.6.1.4.1	Test purpose and Environment	905

A.4.6.1.4.2	Test parameters	905

A.4.6.1.4.3	Test Requirements	909

A.4.6.1.5	EN-DC event triggered reporting tests without gap under non-DRX with SSB index reading	909

A.4.6.1.5.1	Test purpose and Environment	909

A.4.6.1.5.2	Test parameters	909

A.4.6.1.5.3	Test Requirements	911

A.4.6.1.6	EN-DC event triggered reporting tests with SSB index reading with per-UE gaps	912

A.4.6.1.6.1	Test purpose and Environment	912

A.4.6.1.6.2	Test parameters	912

A.4.6.1.6.3	Test Requirements	914

A.4.6.1.7	EN-DC event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	915

A.4.6.1.7.1	Test purpose and Environment	915

A.4.6.1.7.2	Test parameters	915

A.4.6.1.7.3	Test Requirements	919

A.4.6.2	Inter-frequency Measurements	919

A.4.6.2.1	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	919

A.4.6.2.1.1	Test Purpose and Environment	919

A.4.6.2.1.2	Test Requirements	923

A.4.6.2.2	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	923

A.4.6.2.2.1	Test Purpose and Environment	923

A.4.6.2.2.2	Test Requirements	927

A.4.6.2.3	Void	927

A.4.6.2.4	Void	927

A.4.6.2.5	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	927

A.4.6.2.5.1	Test Purpose and Environment	927

A.4.6.2.5.2	Test Requirements	932

A.4.6.2.6	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	932

A.4.6.2.6.1	Test Purpose and Environment	932

A.4.6.2.6.2	Test Requirements	937

A.4.6.2.7	Void	938

A.4.6.2.8	Void	938

A.4.6.3	Void	938

A.4.6.4	L1-RSRP measurement for beam reporting	938

A.4.6.4.1	SSB based L1-RSRP measurement when DRX is not used	938

A.4.6.4.1.1	Test Purpose and Environment	938

A.4.6.4.1.2	Test parameters	938

A.4.6.4.1.3	Test Requirements	941

A.4.6.4.2	SSB based L1-RSRP measurement when DRX is used	941

A.4.6.4.2.1	Test Purpose and Environment	941

A.4.6.4.2.2	Test parameters	942

A.4.6.4.2.3	Test Requirements	945

A.4.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	945

A.4.6.4.3.1	Test Purpose and Environment	945

A.4.6.4.3.2	Test parameters	946

A.4.6.4.3.3	Test Requirements	949

A.4.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	949

A.4.6.4.4.1	Test Purpose and Environment	949

A.4.6.4.4.2	Test parameters	950

A.4.6.4.4.3	Test Requirements	952

A.4.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	952

A.4.6.4.5.1	Test Purpose and Environment	952

A.4.6.4.5.2	Test parameters	953

A.4.6.4.5.3	Test Requirements	956

A.4.6.5	CLI measurements	956

A.4.6.5.1	SRS-RSRP measurement with non-DRX	956

A.4.6.5.1.1	Test Purpose and Environment	956

A.4.6.5.1.2	Test Parameters	957

A.4.6.5.1.3	Test Requirements	960

A.4.6.5.2	CLI-RSSI measurement with non-DRX	960

A.4.6.5.2.1	Test Purpose and Environment	960

A.4.6.5.2.2	Test Parameters	961

A.4.6.5.2.3	Test Requirements	962

A.4.6.6	Measurements with autonomous gaps	963

A.4.6.6.1	EN-DC intra-frequency CGI identification of NR FR1 cell with autonomous gaps in synchronous EN-DC	963

A.4.6.6.1.1	Test Purpose and Environment	963

A.4.6.6.1.2	Test Requirements	967

A.4.6.7	L1-SINR measurement for beam reporting	967

A.4.6.7.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR when DRX is not used	967

A.4.6.7.1.1	Test Purpose and Environment	967

A.4.6.7.1.2	Test parameters	968

A.4.6.7.1.3	Test Requirements	971

A.4.6.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	971

A.4.6.7.2.1	Test Purpose and Environment	971

A.4.6.7.2.2	Test parameters	972

A.4.6.7.2.3	Test Requirements	975

A.4.6.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	975

A.4.6.7.3.1	Test Purpose and Environment	975

A.4.6.7.3.2	Test parameters	976

A.4.6.7.3.3	Test Requirements	979

A.4.6.8	CSI-RS based intra-frequency Measurement	979

A.4.6.8.1	EN-DC event triggered reporting tests without gap under DRX	979

A.4.6.8.1.1	Test purpose and Environment	979

A.4.6.8.1.2	Test Requirements	982

A.4.6.9	CSI-RS based inter-frequency Measurement	983

A.4.6.9.1	EN-DC event triggered reporting tests for FR1 cell when non-DRX is used	983

A.4.6.9.1.1	Test Purpose and Environment	983

A.4.6.9.1.2	Test Requirements	986

A.4.7	Measurement Performance requirements	987

A.4.7.1	SS-RSRP	987

A.4.7.1.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	987

A.4.7.1.1.1	Test Purpose and Environment	987

A.4.7.1.1.2	Test parameters	987

A.4.7.1.1.3	Test Requirements	991

A.4.7.1.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	991

A.4.7.1.2.1	Test Purpose and Environment	991

A.4.7.1.2.2	Test parameters	992

A.4.7.1.2.3	Test Requirements	996

A.4.7.1.3	Void	996

A.4.7.2	SS-RSRQ	996

A.4.7.2.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	996

A.4.7.2.1.1	Test Purpose and Environment	996

A.4.7.2.1.2	Test Parameters	997

A.4.7.2.1.3	Test Requirements	1002

A.4.7.2.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1002

A.4.7.2.2.1	Test Purpose and Environment	1002

A.4.7.2.2.2	Test Parameters	1002

A.4.7.2.2.3	Test Requirements	1007

A.4.7.3	SS-SINR	1007

A.4.7.3.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1007

A.4.7.3.1.1	Test Purpose and Environment	1007

A.4.7.3.1.2	Test Parameters	1007

A.4.7.3.1.3	Test Requirements	1011

A.4.7.3.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1011

A.4.7.3.2.1	Test Purpose and Environment	1011

A.4.7.3.2.2	Test Parameters	1011

A.4.7.3.2.3	Test Requirements	1016

A.4.7.4	L1-RSRP measurement for beam reporting	1016

A.4.7.4.1	SSB based L1-RSRP measurement	1016

A.4.7.4.1.1	Test Purpose and Environment	1016

A.4.7.4.1.2	Test parameters	1017

A.4.7.4.1.3	Test Requirements	1020

A.4.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	1020

A.4.7.4.2.1	Test Purpose and Environment	1020

A.4.7.4.2.2	Test parameters	1021

A.4.7.4.2.3	Test Requirements	1024

A.4.7.5	SFTD accuracy	1024

A.4.7.5.1	SFTD accuracy	1024

A.4.7.5.1.1	Test Purpose and Environment	1024

A.4.7.5.1.2	Test Parameters	1024

A.4.7.5.1.3	Test Requirements	1029

A.4.7.5.2	Void	1029

A.4.7.5.3	Void	1029

A.4.7.6	CLI measurements	1029

A.4.7.6.1	EN-DC SRS-RSRP measurement accuracy with FR1 serving cell	1029

A.4.7.6.1.1	Test Purpose and Environment	1029

A.4.7.6.1.2	Test parameters	1030

A.4.7.6.1.3	Test Requirements	1035

A.4.7.6.2	EN-DC CLI-RSSI measurement accuracy with FR1 serving cell	1035

A.4.7.6.2.1	Test Purpose and Environment	1035

A.4.7.6.2.2	Test parameters	1036

A.4.7.6.2.3	Test Requirements	1039

A.4.7.7	L1-SINR measurement for beam reporting	1039

A.4.7.7.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	1039

A.4.7.7.1.1	Test Purpose and Environment	1039

A.4.7.7.1.2	Test parameters	1040

A.4.7.7.1.3	Test Requirements	1044

A.4.7.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR	1044

A.4.7.7.2.1	Test Purpose and Environment	1044

A.4.7.7.2.2	Test parameters	1045

A.4.7.7.2.3	Test Requirements	1049

A.4.7.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	1049

A.4.7.7.3.1	Test Purpose and Environment	1049

A.4.7.7.3.2	Test parameters	1050

A.4.7.7.3.3	Test Requirements	1053

A.4.7.8	CSI-RSRP	1053

A.4.7.8.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1053

A.4.7.8.1.1	Test Purpose and Environment	1053

A.4.7.8.1.2	Test parameters	1054

A.4.7.8.1.3	Test Requirements	1057

A.4.7.8.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1057

A.4.7.8.2.1	Test Purpose and Environment	1057

A.4.7.8.2.2	Test parameters	1058

A.4.7.8.2.3	Test Requirements	1061

A.4.7.9	CSI-RSRQ	1062

A.4.7.9.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1062

A.4.7.9.1.1	Test Purpose and Environment	1062

A.4.7.9.1.2	Test Parameters	1062

A.4.7.9.1.3	Test Requirements	1067

A.4.7.9.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1067

A.4.7.9.2.1	Test Purpose and Environment	1067

A.4.7.9.2.2	Test Parameters	1067

A.4.7.2.2.3	Test Requirements	1072

A.4.7.10	CSI-SINR	1072

A.4.7.10.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1072

A.4.7.10.1.1	Test Purpose and Environment	1072

A.4.7.10.1.2	Test Parameters	1073

A.4.7.10.1.3	Test Requirements	1077

A.4.7.10.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1077

A.4.7.10.2.1	Test Purpose and Environment	1077

A.4.7.10.2.2	Test Parameters	1077

A.4.7.10.2.3	Test Requirements	1082

A.4.8	Void	1083

A.4A	NE-DC test with all NR cells in FR1	1083

A.4A.1	Signaling characteristics	1083

A.4A.1.1	E-UTRAN PSCell addition	1083

A.4A.1.1.1	Test purpose and environment	1083

A.4A.1.1.2	Test Requirements	1087

A.4A.1.2	Active BWP switch	1088

A.4A.1.2.1	E-UTRAN PSCell – NR PCell FR1 DCI-based and Timer-based DL active BWP switch in non-DRX in synchronous NE-DC	1088

A.4A.1.2.1.1	Test Purpose and Environment	1088

A.4A.1.2.1.2	Test Requirements	1092

A.4A.2	Measurement performance	1093

A.4A.2.1	SFTD accuracy	1093

A.4A.2.1.1	SFTD accuracy	1093

A.4A.2.1.1.1	Test Purpose	1093

A.4A.2.1.1.2	Test Environment	1093

A.4A.2.1.1.3	Test Requirements	1097

A.5	EN-DC tests with one or more NR cells in FR2	1098

A.5.1	Void	1098

A.5.2	Void	1098

A.5.3	RRC_CONNECTED state mobility	1098

A.5.3.1	Void	1098

A.5.3.2	RRC Connection Mobility Control	1098

A.5.3.2.1	Void	1098

A.5.3.2.2	Random Access	1098

A.5.3.2.2.1	4-step RA type contention based random access test in FR2 for PSCell/SCell in EN-DC	1098

A.5.3.2.2.2	4-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	1101

A.5.3.2.2.3	2-step RA type contention based random access test in FR2 for PSCell/SCell in EN-DC	1107

A.5.3.2.2.4	2-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	1110

A.5.3.2.3	Void	1114

A.5.4	Timing	1114

A.5.4.1	UE transmit timing	1114

A.5.4.1.1	NR UE Transmit Timing Test for FR2	1114

A.5.4.1.1.1	Test Purpose and environment	1114

A.5.4.1.1.2	Test requirements	1117

A.5.4.2	UE timer accuracy	1118

A.5.4.3	Timing advance	1118

A.5.4.3.1 EN-DC FR2 timing advance adjustment accuracy	1118

A.5.4.3.1.1 Test Purpose and Environment	1118

A.5.4.3.1.2 Test Parameters	1118

A.5.4.3.1.3	Test Requirements	1122

A.5.5	Signaling characteristics	1122

A.5.5.1	Radio link Monitoring	1122

A.5.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	1122

A.5.5.1.1.1	Test Purpose and Environment	1122

A.5.5.1.1.2	Test Requirements	1126

A.5.5.1.2	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	1127

A.5.5.1.2.1	Test Purpose and Environment	1127

A.5.5.1.2.2	Test Requirements	1130

A.5.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	1131

A.5.5.1.3.1	Test Purpose and Environment	1131

A.5.5.1.3.2	Test Requirements	1135

A.5.5.1.4	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	1135

A.5.5.1.4.1	Test Purpose and Environment	1135

A.5.5.1.4.2	Test Requirements	1139

A.5.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	1139

A.5.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	1143

A.5.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	1147

A.5.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	1152

A.5.5.1.8.2	Test Requirements	1156

A.5.5.1.9	EN-DC Radio Link Monitoring UE Scheduling Restrictions on FR2	1157

A.5.5.1.9.1	Test Purpose and Environment	1157

A.5.5.1.9.2	Test Requirements	1159

A.5.5.2	Interruption	1159

A.5.5.2.1	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	1159

A.5.5.2.1.1	Test Purpose and Environment	1159

A.5.5.2.1.2	Test Requirements	1162

A.5.5.2.2	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	1162

A.5.5.2.2.1	Test Purpose and Environment	1162

A.5.5.2.2.2	Test Requirements	1165

A.5.5.2.3	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	1165

A.5.5.2.3.1	Test Purpose and Environment	1165

A.5.5.2.3.2	Test Requirements	1169

A.5.5.2.4	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	1169

A.5.5.2.4.1	Test Purpose and Environment	1169

A.5.5.2.4.2	Test Requirements	1172

A.5.5.2.5	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	1173

A.5.5.2.5.1	Test Purpose and Environment	1173

A.5.5.2.5.2	Test Requirements	1176

A.5.5.2.6	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	1177

A.5.5.2.6.1	Test Purpose and Environment	1177

A.5.5.2.6.2	Test Requirements	1179

A.5.5.2.7	E-UTRAN – NR FR2 interruptions at E-UTRA SRS carrier based switching	1180

A.5.5.2.7.1	Test Purpose and Environment	1180

A.5.5.2.7.2	Test Requirements	1183

A.5.5.2.8 E-UTRAN – NR FR2 interruptions at NR SRS carrier based switching	1183

A.5.5.2.8.1 Test Purpose and Environment	1183

A.5.5.2.8.3	Test Requirements	1186

A.5.5.3	SCell Activation and Deactivation Delay	1186

A.5.5.3.1	SCell Activation and deactivation of SCell in FR2 intra-band	1186

A.5.5.3.1.1	Test Purpose and Environment	1186

A.5.5.3.1.2	Test Requirements	1189

A.5.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	1189

A.5.5.3.2.1	Test Purpose and Environment	1189

A.5.5.3.2.2	Test Requirements	1193

A.5.5.3.3	Void	1193

A.5.5.3.4	Void	1193

A.5.5.3.5	SCell Activation and deactivation of SCell in FR2	1193

A.5.5.3.5.1	Test Purpose and Environment	1193

A.5.5.3.5.2	Test Requirements	1197

A.5.5.3.6	Multiple SCell Activation and deactivation of one unknown SCell and one known SCell in FR2	1198

A.5.5.3.6.1	Test Purpose and Environment	1198

A.5.5.3.6.2	Test Requirements	1201

A.5.5.3.7	Direct SCell activation at SCell addition of known SCell in FR2	1202

A.5.5.3.7.1	Test Purpose and Environment	1202

A.5.5.3.7.2	Test Requirements	1205

A.5.5.4	Void	1205

A.5.5.5	Beam Failure Detection and Link recovery procedures	1205

A.5.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	1205

A.5.5.5.1.1	Test Purpose and Environment	1205

A.5.5.5.1.2	Test Requirements	1209

A.5.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in DRX mode	1210

A.5.5.5.2.1	Test Purpose and Environment	1210

A.5.5.5.2.2	Test Requirements	1214

A.5.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	1215

A.5.5.5.3.1	Test Purpose and Environment	1215

A.5.5.5.3.2	Test Requirements	1219

A.5.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX mode	1220

A.5.5.5.4.1	Test Purpose and Environment	1220

A.5.5.5.4.2	Test Requirements	1224

A.5.5.5.5	EN-DC scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	1225

A.5.5.5.5.1	Test Purpose and Environment	1225

A.5.5.5.5.2	Test Requirements	1229

A.5.5.5.6	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	1229

A.5.5.5.6.1	Test Purpose and Environment	1229

A.5.5.5.6.2	Test Requirements	1234

A.5.5.5.7	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	1235

A.5.5.5.7.1	Test Purpose and Environment	1235

A.5.5.5.7.2	Test Requirements	1239

A.5.5.6	Active BWP switch	1240

A.5.5.6.1	DCI-based and Timer-based Active BWP Switch	1240

A.5.5.6.1.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	1240

A.5.5.6.1.1.1	Test Purpose and Environment	1240

A.5.5.6.1.1.2	Test Requirements	1243

A.5.5.6.1.2	E-UTRAN – NR PSCell FR2 with FR2 SCell DL active BWP switch in non-DRX in synchronous EN-DC	1243

A.5.5.6.2	RRC-based Active BWP Switch	1248

A.5.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1252

A.5.5.6.3.1	E-UTRAN – NR PSCell FR2 and NR SCell FR2 DL active BWP switch on multiple CCs in synchronous EN-DC	1252

A.5.5.6.3.1.1	Test Purpose and Environment	1252

A.5.5.6.4	SCell dormancy switch	1256

A.5.5.6.4.1	E-UTRAN – NR FR2 PSCell SCell dormancy switch of single FR2 SCell inside active time	1256

A.5.5.6.4.1.1	Test Purpose and Environment	1256

A.5.5.6.4.1.2	Test Requirements	1260

A.5.5.6.4.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR2 SCells outside active time	1260

A.5.5.6.4.2.1	Test Purpose and Environment	1260

A.5.5.6.4.2.2	Test Requirements	1267

A.5.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	1267

A.5.5.7	PSCell addition and release delay	1270

A.5.5.7.1	Addition and Release Delay of NR PSCell	1270

A.5.5.7.1.1	Test purpose and environment	1270

A.5.5.7.1.2	Test Requirements	1274

A.5.5.8	Active TCI state switch delay	1275

A.5.5.8.1	MAC-CE based active TCI state switch	1275

A.5.5.8.1.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	1275

A.5.5.8.1.1.1	Test Purpose and Environment	1275

A.5.5.8.1.1.2	Test Requirements	1279

A.5.5.8.2	RRC based active TCI state switch	1279

A.5.5.8.2.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	1279

A.5.5.8.2.1.1	Test Purpose and Environment	1279

A.5.5.8.2.1.2	Test Requirements	1283

A.5.5.9	Uplink spatial relation switch delay	1283

A.5.5.9.1	MAC-CE based uplink spatial relation switch	1283

A.5.5.9.1.1	E-UTRAN – NR PSCell FR2 uplink spatial relation switch for a known spatial relation	1283

A.5.5.9.1.1.1	Test Purpose and Environment	1283

A.5.5.9.1.1.2	Test Requirements	1286

A.5.5.9.2	RRC based spatial relation switch	1286

A.5.5.9.2.1	E-UTRAN – NR PSCell FR2 spatial relation switch associated with a known DL-RS	1286

A.5.5.9.2.1.1	Test Purpose and Environment	1286

A.5.5.9.2.1.2	Test Requirements	1289

A.5.5.10	UE specific CBW change	1289

A.5.5.10.1	UE specific CBW change on FR2 NR PSCell	1289

A.5.5.10.1.1	Test Purpose and Environment	1289

A.5.5.10.1.2	Test Requirements	1292

A.5.6	Measurement procedure	1293

A.5.6.1	Intra-frequency Measurements	1293

A.5.6.1.1	EN-DC event triggered reporting test without gap under non-DRX	1293

A.5.6.1.1.1	Test purpose and Environment	1293

A.5.6.1.1.2	Test Requirements	1296

A.5.6.1.2	EN-DC event triggered reporting test without gap under DRX	1296

A.5.6.1.2.1	Test purpose and Environment	1296

A.5.6.1.2.2	Test Requirements	1298

A.5.6.1.3	EN-DC event triggered reporting test with per-UE gaps under non-DRX	1299

A.5.6.1.3.1	Test purpose and Environment	1299

A.5.6.1.3.2	Test Requirements	1303

A.5.6.1.4	EN-DC event triggered reporting test with per-UE gaps under DRX	1303

A.5.6.1.4.1	Test purpose and Environment	1303

A.5.6.1.4.2	Test Requirements	1306

A.5.6.2	Inter-frequency Measurements	1307

A.5.6.2.1 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	1307

A.5.6.2.1.1	Test Purpose and Environment	1307

A.5.6.2.1.2	Test Requirements	1310

A.5.6.2.2 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	1310

A.5.6.2.2.1	Test Purpose and Environment	1310

A.5.6.2.2.2	Test Requirements	1314

A.5.6.2.3 	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	1314

A.5.6.2.3.1	Test Purpose and Environment	1314

A.5.6.2.3.2	Test Requirements	1318

A.5.6.2.4	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	1318

A.5.6.2.4.1	Test Purpose and Environment	1318

A.5.6.2.4.2	Test Requirements	1322

A.5.6.2.5	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	1322

A.5.6.2.5.1	Test Purpose and Environment	1322

A.5.6.2.5.2	Test Requirements	1327

A.5.6.2.6	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	1328

A.5.6.2.6.1	Test Purpose and Environment	1328

A.5.6.2.6.2	Test Requirements	1331

A.5.6.2.7	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	1332

A.5.6.2.7.1	Test Purpose and Environment	1332

A.5.6.2.7.2	Test Requirements	1336

A.5.6.2.8	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	1336

A.5.6.2.8.1	Test Purpose and Environment	1336

A.5.6.2.8.2	Test Requirements	1341

A.5.6.3	L1-RSRP measurement for beam reporting	1341

A.5.6.3.1	SSB based L1-RSRP measurement when DRX is not used	1341

A.5.6.3.1.1	Test Purpose and Environment	1341

A.5.6.3.1.2	Test parameters	1341

A.5.6.3.1.3	Test Requirements	1343

A.5.6.3.2	SSB based L1-RSRP measurement when DRX is used	1343

A.5.6.3.2.1	Test Purpose and Environment	1343

A.5.6.3.2.2	Test parameters	1344

A.5.6.3.2.3	Test Requirements	1346

A.5.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	1346

A.5.6.3.3.1	Test Purpose and Environment	1346

A.5.6.3.3.2	Test parameters	1347

A.5.6.3.3.3	Test Requirements	1349

A.5.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	1350

A.5.6.3.4.1	Test Purpose and Environment	1350

A.5.6.3.4.2	Test parameters	1350

A.5.6.3.4.3	Test Requirements	1352

A.5.6.4	CLI measurements	1353

A.5.6.4.1	SRS-RSRP measurement with DRX	1353

A.5.6.4.1.1	Test Purpose and Environment	1353

A.5.6.4.1.2	Test Parameters	1353

A.5.6.4.1.3	Test Requirements	1355

A.5.6.4.2	CLI-RSSI measurement with DRX	1355

A.5.6.4.2.1	Test Purpose and Environment	1355

A.5.6.4.2.2	Test Parameters	1356

A.5.6.4.2.3	Test Requirements	1358

A.5.6.5	Measurements with autonomous gaps	1358

A.5.6.5.1 	EN-DC inter-frequency CGI identification of NR neighbor cell in FR2	1358

A.5.6.5.1.1	Test Purpose and Environment	1358

A.5.6.5.1.2	Test Requirements	1361

A.5.6.6	L1-SINR measurement for beam reporting	1362

A.5.6.6.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured when DRX is used	1362

A.5.6.6.1.1	Test Purpose and Environment	1362

A.5.6.6.1.2	Test parameters	1362

A.5.6.6.1.3	Test Requirements	1364

A.5.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	1364

A.5.6.6.2.1	Test Purpose and Environment	1364

A.5.6.6.2.2	Test parameters	1365

A.5.6.6.2.3	Test Requirements	1368

A.5.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	1368

A.5.6.6.3.1	Test Purpose and Environment	1368

A.5.6.6.3.2	Test parameters	1369

A.5.6.6.3.3	Test Requirements	1371

A.5.6.7	CSI-RS based Intra-frequency Measurements	1371

A.5.6.7.1	EN-DC event triggered reporting test without gap under non-DRX	1371

A.5.6.7.1.1	Test purpose and Environment	1371

A.5.6.7.1.2	Test Requirements	1374

A.5.6.8	CSI-RS based Inter-frequency Measurements	1374

A.5.6.8.1 	EN-DC event triggered reporting tests for NR FR2 cell when DRX is used	1374

A.5.6.8.1.1	Test Purpose and Environment	1374

A.5.6.8.1.2	Test Requirements	1378

A.5.7	Measurement Performance requirements	1378

A.5.7.1	SS-RSRP	1379

A.5.7.1.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1379

A.5.7.1.1.1	Test Purpose and Environment	1379

A.5.7.1.1.2	Test parameters	1379

A.5.7.1.1.3	Test Requirements	1381

A.5.7.1.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1382

A.5.7.1.2.1	Test Purpose and Environment	1382

A.5.7.1.2.2	Test parameters	1382

A.5.7.1.2.3	Test Requirements	1387

A.5.7.1.3	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	1388

A.5.7.1.3.1	Test Purpose and Environment	1388

A.5.7.1.3.2	Test parameters	1388

A.5.7.1.3.3	Test Requirements	1391

A.5.7.2	SS-RSRQ	1392

A.5.7.2.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1392

A.5.7.2.1.1	Test Purpose and Environment	1392

A.5.7.2.1.2	Test Parameters	1392

A.5.7.2.1.3	Test Requirements	1394

A.5.7.2.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1395

A.5.7.2.2.1	Test Purpose and Environment	1395

A.5.7.2.2.2	Test Parameters	1395

A.5.7.2.2.3	Test Requirements	1397

A.5.7.3	SS-SINR	1397

A.5.7.3.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1397

A.5.7.3.1.1	Test Purpose and Environment	1397

A.5.7.3.1.2	Test Parameters	1397

A.5.7.3.1.3	Test Requirements	1399

A.5.7.3.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1399

A.5.7.3.2.1	Test Purpose and Environment	1399

A.5.7.3.2.2	Test Parameters	1399

A.5.7.3.2.3	Test Requirements	1401

A.5.7.4	L1-RSRP measurement for beam reporting	1401

A.5.7.4.1	SSB based L1-RSRP measurement	1401

A.5.7.4.1.1	Test Purpose and Environment	1401

A.5.7.4.1.2	Test parameters	1402

A.5.7.4.1.3	Test Requirements	1404

A.5.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	1405

A.5.7.4.2.1	Test Purpose and Environment	1405

A.5.7.4.2.2	Test parameters	1405

A.5.7.4.2.3	Test Requirements	1407

A.5.7.5	CLI measurements	1408

A.5.7.5.1	EN-DC SRS-RSRP measurement accuracy with FR2 serving cell	1408

A.5.7.5.1.1	Test Purpose and Environment	1408

A.5.7.5.1.2	Test parameters	1408

A.5.7.5.1.3	Test Requirements	1411

A.5.7.5.2	EN-DC CLI-RSSI measurement accuracy with FR2 serving cell	1412

A.5.7.5.2.1	Test Purpose and Environment	1412

A.5.7.5.2.2	Test parameters	1412

A.5.7.5.2.3	Test Requirements	1414

A.5.7.6	L1-SINR measurement for beam reporting	1415

A.5.7.6.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	1415

A.5.7.6.1.1	Test Purpose and Environment	1415

A.5.7.6.1.2	Test parameters	1415

A.5.7.6.1.3	Test Requirements	1417

A.5.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	1418

A.5.7.6.2.1	Test Purpose and Environment	1418

A.5.7.6.2.2	Test parameters	1418

A.5.7.6.2.3	Test Requirements	1420

A.5.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	1421

A.5.7.6.3.1	Test Purpose and Environment	1421

A.5.7.6.3.2	Test parameters	1421

A.5.7.6.3.3	Test Requirements	1423

A.5.7.7	CSI-RSRP	1424

A.5.7.7.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1424

A.5.7.7.1.2	Test parameters	1424

A.5.7.7.1.3	Test Requirements	1426

A.5.7.7.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1427

A.5.7.7.2.1	Test Purpose and Environment	1427

A.5.7.7.2.2	Test parameters	1427

A.5.7.7.2.3	Test Requirements	1431

A.5.7.8	CSI-RSRQ	1432

A.5.7.8.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	1432

A.5.7.8.1.1	Test Purpose and Environment	1432

A.5.7.8.1.2	Test Parameters	1432

A.5.7.8.1.3	Test Requirements	1434

A.5.7.8.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1434

A.5.7.8.2.1	Test Purpose and Environment	1434

A.5.7.8.2.2	Test Parameters	1434

A.5.7.9	CSI-SINR	1436

A.5.7.9.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1436

A.5.7.9.1.1	Test Purpose and Environment	1436

A.5.7.9.1.2	Test Parameters	1436

Table A.5.7.9.1.2-1: CSI-SINR Intra frequency CSI-SINR supported test configurations	1437

A.5.7.9.1.3	Test Requirements	1439

A.5.7.9.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1439

A.5.7.9.2.1	Test Purpose and Environment	1439

A.5.7.9.2.2	Test Parameters	1439

A.5.7.9.2.3	Test Requirements	1442

A.5.8	Void	1442

A.6	NR standalone tests with all NR cells in FR1	1446

A.6.1	SA: RRC_IDLE state mobility	1446

A.6.1.1	Cell re-selection to NR	1446

A.6.1.1.1	Cell reselection to FR1 intra-frequency NR case	1446

A.6.1.1.1.1	Test Purpose and Environment	1446

A.6.1.1.1.2	Test Parameters	1446

A.6.1.1.1.3	Test Requirements	1450

A.6.1.1.2	Cell reselection to FR1 inter-frequency NR case	1450

A.6.1.1.2.1	Test Purpose and Environment	1450

A.6.1.1.2.2	Test Parameters	1450

A.6.1.1.2.3	Test Requirements	1454

A.6.1.1.3	Cell reselection to FR1 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1454

A.6.1.1.3.1	Test Purpose and Environment	1454

A.6.1.1.3.2	Test Parameters	1454

A.6.1.1.3.3	Test Requirements	1459

A.6.1.1.4	Cell reselection to FR1 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	1459

A.6.1.1.4.1	Test Purpose and Environment	1459

A.6.1.1.4.2	Test Parameters	1459

A.6.1.1.4.3	Test Requirements	1463

A.6.1.1.5	Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1463

A.6.1.1.5.1	Test Purpose and Environment	1463

A.6.1.1.5.2	Test Parameters	1463

A.6.1.1.5.3	Test Requirements	1467

A.6.1.1.6	Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	1468

A.6.1.1.6.1	Test Purpose and Environment	1468

A.6.1.1.6.2	Test Parameters	1468

A.6.1.1.6.3	Test Requirements	1472

A.6.1.1.7	Cell reselection to FR1 intra-frequency NR case for UE configured with highSpeedMeasFlag-r16	1473

A.6.1.1.7.1	Test Purpose and Environment	1473

A.6.1.1.7.2	Test Parameters	1473

A.6.1.1.7.3	Test Requirements	1477

A.6.1.2	Inter-RAT E-UTRAN cell re-selection	1477

A.6.1.2.1	Cell reselection to higher priority E-UTRAN	1477

A.6.1.2.1.1	Test Purpose and Environment	1477

A.6.1.2.1.2	Test Parameters	1477

A.6.1.2.1.3	Test Requirements	1480

A.6.1.2.2	Cell reselection to lower priority E-UTRAN	1481

A.6.1.2.2.1	Test Purpose and Environment	1481

A.6.1.2.2.2	Test Parameters	1481

A.6.1.2.2.3	Test Requirements	1484

A.6.1.2.3	Cell reselection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion	1485

A.6.1.2.3.1	Test Purpose and Environment	1485

A.6.1.2.3.2	Test Parameters	1485

A.6.1.2.3.3	Test Requirements	1488

A.6.1.2.4	Cell reselection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion	1489

A.6.1.2.4.1	Test Purpose and Environment	1489

A.6.1.2.4.2	Test Parameters	1489

A.6.1.2.4.3	Test Requirements	1492

A.6.1.2.5.3	Test Requirements	1496

A.6.2	SA: RRC_INACTIVE state mobility	1497

A.6.3	RRC_CONNECTED state mobility	1497

A.6.3.1	Handover	1497

A.6.3.1.1	Intra-frequency handover from FR1 to FR1; known target cell	1497

A.6.3.1.1.1	Test Purpose and Environment	1497

A.6.3.1.1.2	Test Parameters	1497

A.6.3.1.1.3 Test Requirements	1501

A.6.3.1.2	Intra-frequency handover from FR1 to FR1; unknown target cell	1501

A.6.3.1.2.1	Test Purpose and Environment	1501

A.6.3.1.2.2	Test Parameters	1501

A.6.3.1.2.3	Test Requirements	1505

A.6.3.1.3	Inter-frequency handover from FR1 to FR1; unknown target cell	1505

A.6.3.1.3.1	Test Purpose and Environment	1505

A.6.3.1.3.2	Test Parameters	1505

A.6.3.1.3.3 Test Requirements	1509

A.6.3.1.4	 SA NR - E-UTRAN handover	1509

A.6.3.1.4.1	Test Purpose and Environment	1509

A.6.3.1.4.2	Test Requirements	1515

A.6.3.1.5	SA NR - E-UTRAN handover with unknown target cell	1515

A.6.3.1.5.1	Test Purpose and Environment	1515

A.6.3.1.5.2	Test Requirements	1521

A.6.3.1.6	 SA NR - UTRAN FDD handover	1521

A.6.3.1.6.1	Test Purpose and Environment	1521

A.6.3.1.6.2	Test Requirements	1525

A.6.3.1.7	Intra-frequency synchronous DAPS handover in FR1	1525

A.6.3.1.7.1	Test Purpose and Environment	1525

A.6.3.1.7.2	Test Parameters	1525

A.6.3.1.7.3 Test Requirements	1529

A.6.3.1.8	Intra-frequency asynchronous DAPS handover in FR1	1530

A.6.3.1.8.1	Test Purpose and Environment	1530

A.6.3.1.8.2	Test Parameters	1530

A.6.3.1.8.3 Test Requirements	1534

A.6.3.1.9	Intra-band inter-frequency synchronous DAPS handover test in SA for FR1	1535

A.6.3.1.9.1	Test Purpose and Environment	1535

A.6.3.1.9.2	Test Parameters	1535

A.6.3.1.9.3 Test Requirements	1539

A.6.3.1.10	Intra-band inter-frequency asynchronous DAPS handover test in SA for FR1	1539

A.6.3.1.10.1	Test Purpose and Environment	1539

A.6.3.1.10.2	Test Parameters	1539

A.6.3.1.10.3 Test Requirements	1542

A.6.3.1.11	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR1	1542

A.6.3.1.11.1	Test Purpose and Environment	1542

A.6.3.1.11.2	Test Parameters	1542

A.6.3.1.11.3 Test Requirements	1549

A.6.3.1.12	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR1	1549

A.6.3.1.12.1	Test Purpose and Environment	1549

A.6.3.1.12.2	Test Parameters	1549

A.6.3.1.12.3 Test Requirements	1557

A.6.3.2	RRC Connection Mobility Control	1557

A.6.3.2.1	SA: RRC Re-establishment	1557

A.6.3.2.1.1	Intra-frequency RRC Re-establishment in FR1	1557

A.6.3.2.1.2	Inter-frequency RRC Re-establishment in FR1	1561

A.6.3.2.1.3	Intra-frequency RRC Re-establishment in FR1 without serving cell timing	1565

A.6.3.2.2	Random Access	1569

A.6.3.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone	1569

A.6.3.2.2.2	4-step RA type non-contention based random access test in FR1 for NR standalone	1574

A.6.3.2.2.3	2-step RA type contention based random access test in FR1 for NR standalone	1578

A.6.3.2.2.4	2-step RA type non-contention based test in FR1 for NR standalone	1583

A.6.3.2.3	SA: RRC Connection Release with Redirection	1587

A.6.3.2.3.1	Redirection from NR in FR1 to NR in FR1	1587

A.6.3.2.3.2	Redirection from NR in FR1 to E-UTRAN	1591

A.6.3.3	Conditional handover	1598

A.6.3.3.1	Intra-frequency conditional handover from FR1 to FR1	1598

A.6.3.3.1.1	Test Purpose and Environment	1598

A.6.3.3.1.2	Test Parameters	1598

A.6.3.3.1.3 Test Requirements	1602

A.6.3.3.2	Inter-frequency conditional handover from FR1 to FR1	1602

A.6.3.3.2.1	Test Purpose and Environment	1602

A.6.3.3.2.2	Test Parameters	1602

A.6.3.3.2.3 Test Requirements	1606

A.6.4	Timing	1606

A.6.4.1	UE transmit timing	1606

A.6.4.1.1	NR UE Transmit Timing Test for FR1	1606

A.6.4.1.1.1	Test Purpose and environment	1606

A.6.4.1.1.2	Test requirements	1610

A.6.4.2	UE timer accuracy	1610

A.6.4.3	Timing advance	1610

A.6.4.3.1	SA FR1 timing advance adjustment accuracy	1610

A.6.4.3.1.1	Test Purpose and Environment	1610

A.6.4.3.1.2	Test Parameters	1610

A.6.4.3.1.3	Test Requirements	1614

A.6.5	Signalling characteristics	1614

A.6.5.1	Radio link Monitoring	1614

A.6.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	1615

A.6.5.1.1.1	Test Purpose and Environment	1615

A.6.5.1.1.2	Test Requirements	1620

A.6.5.1.2	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	1620

A.6.5.1.2.1	Test Purpose and Environment	1620

A.6.5.1.2.2	Test Requirements	1626

A.6.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	1626

A.6.5.1.3.1	Test Purpose and Environment	1626

A.6.5.1.3.2	Test Requirements	1632

A.6.5.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	1632

A.6.5.1.4.1	Test Purpose and Environment	1632

A.6.5.1.4.2	Test Requirements	1638

A.6.5.1.5	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	1638

A.6.5.1.5.1	Test Purpose and Environment	1638

A.6.5.1.5.2	Test Requirements	1644

A.6.5.1.6	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	1644

A.6.5.1.6.1	Test Purpose and Environment	1644

A.6.5.1.6.2	Test Requirements	1649

A.6.5.1.7	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	1649

A.6.5.1.7.1	Test Purpose and Environment	1649

A.6.5.1.7.2	Test Requirements	1653

A.6.5.1.8	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	1653

A.6.5.1.8.1	Test Purpose and Environment	1653

A.6.5.1.8.2	Test Requirements	1659

A.6.5.2	Interruption	1659

A.6.5.2.1	Interruptions during measurements on deactivated NR SCC in FR1	1659

A.6.5.2.1.2	Test Requirements	1663

A.6.5.2.2	SA interruptions at NR SRS carrier based switching	1664

A.6.5.2.2.1	Test Purpose and Environment	1664

A.6.5.2.2.2	Test Parameters	1664

A.6.5.2.2.3	Test Requirements	1666

A.6.5.3	SCell Activation and Deactivation Delay	1667

A.6.5.3.1	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160ms SCell measurement cycle	1667

A.6.5.3.1.1	Test Purpose and Environment	1667

A.6.5.3.1.2	Test Requirements	1672

A.6.5.3.2	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle	1673

A.6.5.3.2.1	Test Purpose and Environment	1673

A.6.5.3.2.2	Test Requirements	1673

A.6.5.3.3	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX	1673

A.6.5.3.3.1	Test Purpose and Environment	1673

A.6.5.3.3.2	Test Requirements	1674

A.6.5.3.4	Direct SCell activation at SCell addition of known SCell in FR1	1674

A.6.5.3.4.1	 Test Purpose and Environment	1674

A.6.5.3.4.2	Test Requirements	1679

A.6.5.3.5	Direct SCell activation at handover with known SCell in FR1	1679

A.6.5.3.5.1	Test Purpose and Environment	1679

A.6.5.3.5.2	Test Requirements	1684

A.6.5.4	UE UL carrier RRC reconfiguration Delay	1685

A.6.5.4.1	UE UL carrier RRC reconfiguration Delay	1685

A.6.5.4.1.1	Test Purpose and Environment	1685

A.6.5.4.1.2	Test Requirements	1693

A.6.5.4.2	Void	1694

A.6.5.5	Beam Failure Detection and Link recovery procedures	1694

A.6.5.5.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	1694

A.6.5.5.1.1	Test Purpose and Environment	1694

A.6.5.5.1.2	Test Requirements	1699

A.6.5.5.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	1700

A.6.5.5.2.1	Test Purpose and Environment	1700

A.6.5.5.2.2	Test Requirements	1706

A.6.5.5.3	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	1707

A.6.5.5.3.1	Test Purpose and Environment	1707

A.6.5.5.3.2	Test Requirements	1712

A.6.5.5.4	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode	1713

A.6.5.5.4.1	Test Purpose and Environment	1713

A.6.5.5.4.2	Test Requirements	1718

A.6.5.5.5	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	1719

A.6.5.5.5.1	Test Purpose and Environment	1719

A.6.5.5.5.2	Test Requirements	1723

A.6.5.5.6	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode	1723

A.6.5.5.6.1	Test Purpose and Environment	1723

A.6.5.5.6.2	Test Requirements	1727

A.6.5.6	Active BWP switch	1728

A.6.5.6.1	DCI-based and Timer-based Active BWP Switch	1728

A.6.5.6.1.1	NR FR1- NR FR1 DL active BWP switch of SCell with non-DRX in SA	1728

A.6.5.6.1.2	NR FR1 DL active BWP switch with non-DRX in SA	1735

A.6.5.6.2	RRC-based Active BWP Switch	1739

A.6.5.6.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	1739

A.6.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1743

A.6.5.6.3.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	1743

A.6.5.6.4	SCell dormancy switch	1750

A.6.5.6.4.1	NR FR1 PCell SCell dormancy switch of single FR1 SCell outside active time	1750

A.6.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR1 SCells inside active time	1756

A.6.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	1763

A.6.5.6.5.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	1763

A.6.5.7	DL interruptions at switching between two uplink carriers	1768

A.6.5.7.1	DL interruptions at switching between two uplink carriers in FDD-TDD CA	1768

A.6.5.7.1.1	Test Purpose and Environment	1768

A.6.5.7.1.2	Test Requirements	1772

A.6.5.7.2	DL interruptions at switching between two uplink carriers in TDD-TDD CA	1772

A.6.5.7.2.1	Test Purpose and Environment	1772

A.6.5.7.2.2	Test Requirements	1776

A.6.5.8	UE specific CBW change	1776

A.6.5.8.1	UE specific CBW change on PCell in FR1 in non-DRX	1776

A.6.5.8.1.1	Test Purpose and Environment	1776

A.6.5.8.1.2	Test Requirements	1780

A.6.5.9	Pathloss reference signal switching delay	1780

A.6.5.9.1	MAC-CE based pathloss reference signal switch delay	1780

A.6.5.9.1.1	Test Purpose and Environment	1780

A.6.5.9.1.2	Test Requirements	1783

A.6.6	Measurement procedure	1784

A.6.6.1	Intra-frequency Measurements	1784

A.6.6.1.1	SA event triggered reporting tests without gap under non-DRX	1784

A.6.6.1.1.1	Test purpose and Environment	1784

A.6.6.1.1.2	Test parameters	1784

A.6.6.1.1.3	Test Requirements	1788

A.6.6.1.2	SA event triggered reporting tests without gap under DRX	1788

A.6.6.1.2.1	Test purpose and Environment	1788

A.6.6.1.2.2	Test parameters	1788

A.6.6.1.2.3	Test Requirements	1792

A.6.6.1.3	SA event triggered reporting tests with per-UE gaps under non-DRX	1792

A.6.6.1.3.1	Test purpose and Environment	1792

A.6.6.1.3.2	Test parameters	1792

A.6.6.1.3.3	Test Requirements	1796

A.6.6.1.4	SA event triggered reporting tests with per-UE gaps under DRX	1796

A.6.6.1.4.1	Test purpose and Environment	1796

A.6.6.1.4.2	Test parameters	1796

A.6.6.1.4.3	Test Requirements	1800

A.6.6.1.5	SA event triggered reporting tests without gap under non-DRX with SSB index reading	1800

A.6.6.1.5.1	Test purpose and Environment	1800

A.6.6.1.5.2	Test parameters	1800

A.6.6.1.5.3	Test Requirements	1802

A.6.6.1.6	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading	1803

A.6.6.1.6.1	Test purpose and Environment	1803

A.6.6.1.6.2	Test parameters	1803

A.6.6.1.6.3	Test Requirements	1804

A.6.6.1.7	SA event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	1805

A.6.6.1.7.1	Test purpose and Environment	1805

A.6.6.1.7.2	Test parameters	1805

A.6.6.1.7.3	Test Requirements	1809

A.6.6.2	Inter-frequency Measurements	1809

A.6.6.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	1809

A.6.6.2.1.1	Test Purpose and Environment	1809

A.6.6.2.1.2	Test Requirements	1813

A.6.6.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used	1813

A.6.6.2.2.1	Test Purpose and Environment	1813

A.6.6.2.2.2	Test Requirements	1817

A.6.6.2.3	Void	1818

A.6.6.2.4	Void	1818

A.6.6.2.5	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	1818

A.6.6.2.5.1	Test Purpose and Environment	1818

A.6.6.2.5.2	Test Requirements	1822

A.6.6.2.6	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used	1822

A.6.6.2.6.1	Test Purpose and Environment	1822

A.6.6.2.6.2	Test Requirements	1826

A.6.6.2.7	Void	1827

A.6.6.2.8	Void	1827

A.6.6.2.9	SA event triggered reporting tests with additional mandatory gap pattern	1827

A.6.6.2.9.1	Test Purpose and Environment	1827

A.6.6.2.9.2	Test Requirements	1830

A.6.6.2.10	SA event triggered reporting tests for FR1 when DRX is used	1830

A.6.6.2.10.1	Test Purpose and Environment	1830

A.6.6.2.10.2	Test Requirements	1834

A.6.6.2.11	SA event triggered reporting tests for FR1 without gap when DRX is not used	1835

A.6.6.2.11.1	Test Purpose and Environment	1835

A.6.6.2.11.2	Test Requirements	1838

A.6.6.3	Inter-RAT Measurements	1838

A.6.6.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1	1838

A.6.6.3.1.1	Test Purpose and Environment	1838

A.6.6.3.1.2	Test Requirements	1844

A.6.6.3.2	SA NR - E-UTRAN event-triggered reporting in DRX in FR1	1844

A.6.6.3.2.1	Test Purpose and Environment	1844

A.6.6.3.2.2	Test Requirements	1851

A.6.6.3.3	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for UE configured with highSpeedMeasFlag-r16	1851

A.6.6.3.3.1	Test Purpose and Environment	1851

A.6.6.3.3.2	Test Requirements	1858

A.6.6.4	L1-RSRP measurement for beam reporting	1858

A.6.6.4.1	SSB based L1-RSRP measurement when DRX is not used	1858

A.6.6.4.1.1	Test Purpose and Environment	1858

A.6.6.4.1.2	Test parameters	1858

A.6.6.4.1.3	Test Requirements	1861

A.6.6.4.2	SSB based L1-RSRP measurement when DRX is used	1861

A.6.6.4.2.1	Test Purpose and Environment	1861

A.6.6.4.2.2	Test parameters	1862

A.6.6.4.2.3	Test Requirements	1865

A.6.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	1865

A.6.6.4.3.1	Test Purpose and Environment	1865

A.6.6.4.3.2	Test parameters	1866

A.6.6.4.3.3	Test Requirements	1869

A.6.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	1869

A.6.6.4.4.1	Test Purpose and Environment	1869

A.6.6.4.4.2	Test parameters	1870

A.6.6.4.4.3	Test Requirements	1873

A.6.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	1873

A.6.6.4.5.1	Test Purpose and Environment	1873

A.6.6.4.5.2	Test parameters	1874

A.6.6.4.5.3	Test Requirements	1877

A.6.6.5	1877

A.6.6.5.1	SA NR - UTRAN FDD event-triggered reporting in non-DRX in FR1	1877

A.6.6.5.1.1	Test Purpose and Environment	1877

A.6.6.5.1.2	Test Requirements	1881

A.6.6.6	CLI measurements	1881

A.6.6.6.1	SRS-RSRP measurement with DRX	1881

A.6.6.6.1.1	Test Purpose and Environment	1881

A.6.6.6.1.2	Test Parameters	1882

A.6.6.6.1.3	Test Requirements	1885

A.6.6.6.2	CLI-RSSI measurement with DRX	1885

A.6.6.6.2.1	Test Purpose and Environment	1885

A.6.6.6.2.2	Test Parameters	1886

A.6.6.6.2.3	Test Requirements	1888

A.6.6.7	NR measurements with autonomous gaps	1888

A.6.6.7.1	SA intra-frequency CGI identification of NR neighbor cell in FR1	1888

A.6.6.7.1.1	Test Purpose and Environment	1888

A.6.6.7.1.2	Test Parameters	1888

A.6.6.7.1.3	Test Requirements	1891

A.6.6.7.2	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA	1891

A.6.6.7.2.1	Test Purpose and Environment	1891

A.6.6.7.2.2	Test Requirements	1894

A.6.6.8	L1-SINR measurement for beam reporting	1895

A.6.6.8.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured when DRX is used	1895

A.6.6.8.1.1	Test Purpose and Environment	1895

A.6.6.8.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	1898

A.6.6.8.2.1	Test Purpose and Environment	1898

A.6.6.8.2.2	Test parameters	1899

A.6.6.8.2.3	Test Requirements	1903

A.6.6.8.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	1903

A.6.6.8.3.1	Test Purpose and Environment	1903

A.6.6.8.3.2	Test parameters	1904

A.6.6.8.3.3	Test Requirements	1907

A.6.6.9	Idle Mode CA/DC Measurements	1907

A.6.6.9.1	SA Idle mode CA/DC measurement for FR1	1907

A.6.6.9.1.1	Test Purpose and Environment	1907

A.6.6.9.1.2	Test Requirements	1914

A.6.6.10	CSI-RS based intra-frequency Measurements	1914

A.6.6.10.1	SA event triggered reporting tests without gap under non-DRX	1914

A.6.6.10.1.1	Test purpose and Environment	1914

A.6.6.10.1.2	Test Requirements	1917

A.6.6.11	CSI-RS based inter-frequency Measurements	1917

A.6.6.11.1	 SA event triggered reporting tests with gap under DRX	1917

A.6.6.11.1.1	Test Purpose and Environment	1917

A.6.6.11.1.2	Test Requirements	1921

A.6.6.12	RSTD measurements	1921

A. 6.6.12.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA	1921

A. 6.6.12.1.1	Test Purpose and Environment	1921

A.6.6.12.1.2	Test Requirements	1929

A. 6.6.12.2	NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR1 SA	1929

A. 6.6.12.2.1	Test Purpose and Environment	1929

A.6.6.12.2.2	Test Requirements	1936

A.6.6.13 PRS-RSRP measurements	1936

A.6.6.13.1	PRS-RSRP reporting delay test case for single positioning frequency layer	1936

A.6.6.13.1.1	Test purpose and Environment	1936

A.6.6.13.1.2	Test Requirements	1940

A.6.6.13.2	PRS-RSRP reporting delay test case for dual positioning frequency layer	1940

A.6.6.13.2.1	Test purpose and Environment	1940

A.6.6.13.2.2	Test Requirements	1944

A.6.6.14	UE Rx-Tx time difference measurements	1944

A.6.6.14.1 UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA	1944

A.6.6.14.1.1	Test purpose and environment	1944

A.6.6.14.1.2	Test requirements	1948

A.6.6.14.2 UE Rx-Tx time difference measurement for dual positioning frequency layers in FR1 SA	1948

A.6.6.14.2.1	Test purpose and environment	1948

A.6.6.14.2.2	Test requirements	1952

A.6.6.15	Idle Mode measurements of inter-RAT DC candidate cells for early reporting	1952

A.6.6.15.1	Test Purpose and Environment	1952

A.6.6.15.2	Test Requirements	1960

A.6.7	Measurement Performance requirements	1961

A.6.7.1	SS-RSRP	1961

A.6.7.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	1961

A.6.7.1.1.1	Test Purpose and Environment	1961

A.6.7.1.1.2	Test parameters	1961

A.6.7.1.1.3	Test Requirements	1966

A.6.7.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	1966

A.6.7.1.2.1	Test Purpose and Environment	1966

A.6.7.1.2.2	Test parameters	1966

A.6.7.1.2.3	Test Requirements	1970

A.6.7.1.3	Void	1970

A.6.7.2	SS-RSRQ	1970

A.6.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1970

A.6.7.2.1.1	Test Purpose and Environment	1970

A.6.7.2.1.2	Test Parameters	1971

A.6.7.2.1.3	Test Requirements	1976

A.6.7.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1976

A.6.7.2.2.1	Test Purpose and Environment	1976

A.6.7.2.2.2	Test Parameters	1976

A.6.7.2.2.3	Test Requirements	1981

A.6.7.3	SS-SINR	1981

A.6.7.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1981

A.6.7.3.1.1	Test Purpose and Environment	1981

A.6.7.3.1.2	Test Parameters	1981

A.6.7.3.1.3	Test Requirements	1985

A.6.7.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1985

A.6.7.3.2.1	Test Purpose and Environment	1985

A.6.7.3.2.2	Test Parameters	1986

A.6.7.3.2.3	Test Requirements	1990

A.6.7.4	L1-RSRP measurement for beam reporting	1991

A.6.7.4.1	SSB based L1-RSRP measurement	1991

A.6.7.4.1.1	Test Purpose and Environment	1991

A.6.7.4.1.2	Test parameters	1991

A.6.7.4.1.3	Test Requirements	1995

A.6.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	1995

A.6.7.4.2.1	Test Purpose and Environment	1995

A.6.7.4.2.2	Test parameters	1996

A.6.7.4.2.3	Test Requirements	2000

A.6.7.5	E-UTRAN RSRP	2000

A.6.7.5.1	SA: inter-RAT measurement accuracy with FR1 serving cell	2000

A.6.7.5.1.1	Test Purpose and Environment	2000

A.6.7.5.1.2	Test parameters	2001

A.6.7.5.1.3	Test Requirements	2007

A.6.7.6	E-UTRAN RSRQ	2008

A.6.7.6.1	SA: inter-RAT measurement accuracy with FR1 serving cell	2008

A.6.7.6.1.1	Test Purpose and Environment	2008

A.6.7.6.1.2	Test parameters	2008

A.6.7.6.1.3	Test Requirements	2013

A.6.7.7	E-UTRAN RS-SINR	2014

A.6.7.7.1	SA: inter-RAT measurement accuracy with FR1 serving cell	2014

A.6.7.7.1.1	Test Purpose and Environment	2014

A.6.7.7.1.2	Test parameters	2014

A.6.7.7.1.3	Test Requirements	2020

A.6.7.8	CLI measurements	2021

A.6.7.8.1	SA SRS-RSRP measurement accuracy with FR1 serving cell	2021

A.6.7.8.1.1	Test Purpose and Environment	2021

A.6.7.8.1.2	Test parameters	2021

A.6.7.8.1.3	Test Requirements	2027

A.6.7.8.2	SA CLI-RSSI measurement accuracy with FR1 serving cell	2027

A.6.7.8.2.1	Test Purpose and Environment	2027

A.6.7.8.2.2	Test parameters	2028

A.6.7.8.2.3	Test Requirements	2031

A.6.7.9	L1-SINR measurement for beam reporting	2032

A.6.7.9.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	2032

A.6.7.9.1.1	Test Purpose and Environment	2032

A.6.7.9.1.2	Test parameters	2032

A.6.7.9.1.3	Test Requirements	2036

A.6.7.9.2	L1-SINR measurement with SSB based CMR and dedicated IMR	2036

A.6.7.9.2.1	Test Purpose and Environment	2036

A.6.7.9.2.2	Test parameters	2037

A.6.7.9.2.3	Test Requirements	2042

A.6.7.9.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	2042

A.6.7.9.3.1	Test Purpose and Environment	2042

A.6.7.9.3.2	Test parameters	2042

A.6.7.9.3.3	Test Requirements	2047

A.6.7.10	CSI-RSRP	2047

A.6.7.10.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	2047

A.6.7.10.1.1	Test Purpose and Environment	2047

A.6.7.10.1.2	Test parameters	2048

A.6.7.10.1.3	Test Requirements	2053

A.6.7.10.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	2053

A.6.7.10.2.1	Test Purpose and Environment	2053

A.6.7.10.2.2	Test parameters	2053

A.6.7.10.2.3	Test Requirements	2058

A.6.7.11	CSI-RSRQ	2058

A.6.7.11.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2058

A.6.7.11.1.1	Test Purpose and Environment	2058

A.6.7.11.1.2	Test Parameters	2058

A.6.7.11.1.3	Test Requirements	2063

A.6.7.11.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2063

A.6.7.11.2.1	Test Purpose and Environment	2063

A.6.7.11.2.2	Test Parameters	2063

A.6.7.11.2.3	Test Requirements	2068

A.6.7.12	CSI-SINR	2068

A.6.7.12.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2068

A.6.7.12.1.1	Test Purpose and Environment	2068

A.6.7.12.1.2	Test Parameters	2069

A.6.7.12.1.3	Test Requirements	2073

A.6.7.12.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2074

A.6.7.12.2.1	Test Purpose and Environment	2074

A.6.7.12.2.2	Test Parameters	2074

A.6.7.12.2.3	Test Requirements	2080

A.6.7.13	RSTD measurements	2080

A.6.7.13.1	RSTD measurement accuracy test case for single positioning frequency layer	2080

A.6.7.13.1.1	Test purpose and Environment	2080

A.6.7.13.1.2	Test Requirements	2083

A.6.7.13.2	RSTD measurement accuracy test case for dual positioning frequency layer	2083

A.6.7.13.2.1	Test purpose and Environment	2083

A.6.7.13.2.2	Test Requirements	2087

A.6.7.14	PRS-RSRP measurements	2087

A.6.7.14.1	SA: measurement accuracy with PRS in FR1	2087

A.6.7.14.1.1	Test Purpose and Environment	2087

A.6.7.14.1.2	Test parameters	2088

A.6.7.14.1.3	Test Requirements	2092

A.6.7.15	UE Rx-Tx time difference measurements	2092

A.6.7.15.1 UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR1 SA	2092

A.6.7.15.1.1	Test purpose and environment	2092

A.6.7.15.1.2	Test parameters	2093

A.6.7.15.1.3	Test requirements	2096

A.7.1	SA: RRC_IDLE state mobility	2097

A.7.1.1	Cell re-selection to NR	2097

A.7.1.1.1	Cell reselection to FR2 intra-frequency NR case	2097

A.7.1.1.1.1	Test Purpose and Environment	2097

A.7.1.1.1.2	Test Parameters	2097

A.7.1.1.1.3	Test Requirements	2100

A.7.1.1.2	Cell reselection to FR2 inter-frequency NR case	2100

A.7.1.1.2.1	Test Purpose and Environment	2100

A.7.1.1.2.2	Test Parameters	2101

A.7.1.1.2.3	Test Requirements	2103

A.7.1.1.3	Cell reselection to FR2 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	2103

A.7.1.1.3.1	Test Purpose and Environment	2103

A.7.1.1.3.2	Test Parameters	2103

A.7.1.1.3.3	Test Requirements	2106

A.7.1.1.4	Cell reselection to FR2 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	2106

A.7.1.1.4.1	Test Purpose and Environment	2106

A.7.1.1.4.2	Test Parameters	2106

A.7.1.1.4.3	Test Requirements	2109

A.7.1.1.5	Cell reselection to FR2 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	2109

A.7.1.1.5.1	Test Purpose and Environment	2109

A.7.1.1.5.2	Test Parameters	2109

A.7.1.1.5.3	Test Requirements	2112

A.7.1.1.6	Cell reselection to FR2 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	2112

A.7.1.1.6.1	Test Purpose and Environment	2112

A.7.1.1.6.2	Test Parameters	2112

A.7.1.1.6.3	Test Requirements	2115

A.7.2	SA: RRC_INACTIVE state mobility	2115

A.7.3	RRC_CONNECTED state mobility	2115

A.7.3.1	Handover	2115

A.7.3.1.1	Inter-frequency handover from FR1 to FR2; unknown target cell	2115

A.7.3.1.1.1	Test Purpose and Environment	2115

A.7.3.1.1.2	Test Parameters	2115

A.7.3.1.1.3	Test Requirements	2119

A.7.3.1.2	Intra-frequency handover from FR2 to FR2; unknown target cell	2119

A.7.3.1.2.1	Test Purpose and Environment	2119

A.7.3.1.2.2	Test Parameters	2119

A.7.3.1.2.3	Test Requirements	2122

A.7.3.1.3	Inter-frequency handover from FR2 to FR2; unknown target cell	2122

A.7.3.1.3.1	Test Purpose and Environment	2122

A.7.3.1.3.2	Test Parameters	2122

A.7.3.1.3.3 Test Requirements	2123

A.7.3.1.4	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR2	2124

A.7.3.1.4.1	Test Purpose and Environment	2124

A.7.3.1.4.2	Test Parameters	2124

A.7.3.1.4.3 Test Requirements	2131

A.7.3.1.5	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR2	2131

A.7.3.1.5.1	Test Purpose and Environment	2131

A.7.3.1.5.2	Test Parameters	2131

A.7.3.1.5.3 Test Requirements	2138

A.7.3.2	RRC Connection Mobility Control	2138

A.7.3.2.1	SA: RRC Re-establishment	2138

A.7.3.2.1.1	Intra-frequency RRC Re-establishment in FR2	2138

A.7.3.2.1.2	Inter-frequency RRC Re-establishment in FR2	2141

A.7.3.2.1.3	Intra-frequency RRC Re-establishment in FR2 without serving cell timing	2144

A.7.3.2.1.3.1	Test Purpose and Environment	2144

A.7.3.2.1.3.2	Test Requirements	2146

A.7.3.2.2	Random Access	2147

A.7.3.2.2.1	4-step RA type contention based random access test in FR2 for NR Standalone	2147

A.7.3.2.2.2	4-step RA type non-contention based random access test in FR2 for NR Standalone	2151

A.7.3.2.2.3	2-step RA type contention based random access test in FR2 for NR Standalone	2156

A.7.3.2.2.4	2-step RA type non-contention based random access test in FR2 for NR Standalone	2159

A.7.3.2.3	SA: RRC Connection Release with Redirection	2162

A.7.3.2.3.1	Redirection from NR in FR2 to NR in FR2	2162

A.7.3.3	Conditional Handover	2166

A.7.3.3.1	Intra-frequency conditional handover from FR2 to FR2	2166

A.7.3.3.1.1	Test Purpose and Environment	2166

A.7.3.3.1.2	Test Parameters	2166

A.7.3.3.1.2.3	Test Requirements	2169

A.7.3.3.2	Inter-frequency conditional handover from FR2 to FR2; unknown target cell	2169

A.7.3.3.2.1	Test Purpose and Environment	2169

A.7.3.3.2.2	Test Parameters	2169

A.7.3.3.2.3 Test Requirements	2172

A.7.4	Timing	2172

A.7.4.1	UE transmit timing	2172

A.7.4.1.1	NR UE Transmit Timing Test for FR2	2172

A.7.4.1.1.1	Test Purpose and environment	2172

A.7.4.1.1.2	Test requirements	2175

A.7.4.2	UE timer accuracy	2176

A.7.4.3	Timing advance	2176

A.7.4.3.1	SA FR2 timing advance adjustment accuracy	2176

A.7.4.3.1.1	Test Purpose and Environment	2176

A.7.4.3.1.2	Test Parameters	2176

A.7.4.3.1.3 Test Requirements	2179

A.7.5	Signaling characteristics	2180

A.7.5.1	Radio link Monitoring	2180

A.7.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	2180

A.7.5.1.1.1	Test Purpose and Environment	2180

A.7.5.1.1.2	Test Requirements	2183

A.7.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	2184

A.7.5.1.2.1	Test Purpose and Environment	2184

A.7.5.1.2.2	Test Requirements	2188

A.7.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	2189

A.7.5.1.3.1	Test Purpose and Environment	2189

A.7.5.1.3.2	Test Requirements	2193

A.7.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	2193

A.7.5.1.4.1	Test Purpose and Environment	2193

A.7.5.1.4.2	Test Requirements	2198

A.7.5.1.5	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	2198

A.7.5.1.5.1	Test Purpose and Environment	2198

A.7.5.1.5.2	Test Requirements	2203

A.7.5.1.6	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	2203

A.7.5.1.6.1	Test Purpose and Environment	2203

A.7.5.1.6.2	Test Requirements	2207

A.7.5.1.7	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	2207

A.7.5.1.7.1	Test Purpose and Environment	2207

A.7.5.1.7.2	Test Requirements	2211

A.7.5.1.8	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	2211

A.7.5.1.8.1	Test Purpose and Environment	2211

A.7.5.1.8.2	Test Requirements	2216

A.7.5.1.9	UE Radio Link Monitoring Scheduling Restrictions on FR2	2216

A.7.5.1.9.1	Test Purpose and Environment	2216

A.7.5.1.9.2	Test Requirements	2219

A.7.5.2	Interruption	2219

A.7.5.2.1	Interruptions during measurements on deactivated NR SCC in FR2	2219

A.7.5.2.2	SA interruptions at NR SRS carrier-based switching	2223

A.7.5.2.2.1	Test Purpose and Environment	2223

A.7.5.2.2.2	Test Parameters	2223

A.7.5.2.2.3	Test Requirements	2225

A.7.5.3	SCell Activation and Deactivation Delay	2225

A.7.5.3.1	SCell Activation and deactivation for SCell in FR2 intra-band in non-DRX	2225

A.7.5.3.1.1	Test Purpose and Environment	2225

A.7.5.3.1.2	Test Requirements	2227

A.7.5.3.2	SCell Activation and deactivation for FR1+FR2 inter-band with target SCell in FR2	2227

A.7.5.3.2.1	Test Purpose and Environment	2227

A.7.5.3.2.2	Test Requirements	2231

A.7.5.3.3	SCell Activation and deactivation for SCell in FR2 inter-band in non-DRX	2232

A.7.5.3.3.1	Test Purpose and Environment	2232

A.7.5.3.3.2	Test Requirements	2235

A.7.5.3.4	Direct SCell activation at SCell addition of known SCell in FR2	2236

A.7.5.3.4.1	Test Purpose and Environment	2236

A.7.5.3.4.2	Test Requirements	2239

A.7.5.3.5	Direct SCell activation at handover with known SCell in FR2	2240

A.7.5.3.5.1	Test Purpose and Environment	2240

A.7.5.3.5.2	Test Requirements	2243

A.7.5.4	Void	2244

A.7.5.5	Beam Failure Detection and Link recovery procedures	2244

A.7.5.5.1	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	2244

A.7.5.5.1.1	Test Purpose and Environment	2244

A.7.5.5.1.2	Test Requirements	2248

A.7.5.5.2	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode	2249

A.7.5.5.2.1	Test Purpose and Environment	2249

A.7.5.5.2.2	Test Requirements	2252

A.7.5.5.3	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	2253

A.7.5.5.3.1	Test Purpose and Environment	2253

A.7.5.5.3.2	Test Requirements	2257

A.7.5.5.4	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode	2258

A.7.5.5.4.1	Test Purpose and Environment	2258

A.7.5.5.4.2	Test Requirements	2262

A.7.5.5.5	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	2263

A.7.5.5.5.1	Test Purpose and Environment	2263

A.7.5.5.5.2	Test Requirements	2266

A.7.5.5.6	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	2267

A.7.5.5.6.1	Test Purpose and Environment	2267

A.7.5.5.6.2	Test Requirements	2271

A.7.5.5.7	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	2272

A.7.5.5.7.1	Test Purpose and Environment	2272

A.7.5.5.7.2	Test Requirements	2276

A.7.5.6	Active BWP switch	2277

A.7.5.6.1	DCI-based and Timer-based Active BWP Switch	2277

A.7.5.6.1.1	NR FR2- NR FR2 DL active BWP switch of SCell with non-DRX in SA	2277

A.7.5.6.1.2	NR FR1- NR FR2 DL active BWP switch of SCell with non-DRX in SA A.7.5.6.1.2.1 Test Purpose and Environment	2281

A.7.5.6.1.3	NR FR2 DL active BWP switch with non-DRX in SA	2286

A.7.5.6.1.3.1	Test Purpose and Environment	2286

A.7.5.6.1.3.2	Test Requirements	2289

A.7.5.6.2	RRC-based Active BWP Switch	2289

A.7.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	2293

A.7.5.6.3.1	Active BWP switch on multiple SCells with non-DRX in SA	2293

A.7.5.6.4	SCell dormancy switch	2296

A.7.5.6.4.1	NR FR2 PCell SCell dormancy switch of single FR2 SCell inside active time	2296

A.7.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR2 SCells outside active time	2301

A.7.5.6.4.2.2	 Test Requirements	2306

A.7.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	2306

A.7.5.6.5.1	Active BWP switch on multiple SCells with non-DRX in SA	2306

A.7.5.7	PSCell addition and release delay	2309

A.7.5.7.1	Addition and Release Delay of known NR PSCell	2309

A.7.5.7.1.1	Test Purpose and Environment	2309

A.7.5.7.2	Addition and Release Delay of unknown NR PSCell	2313

A.7.5.7.2.1	Test Purpose and Environment	2313

A.7.5.7.2.2	Test Requirements	2316

A.7.5.8	Active TCI state switch delay	2316

A.7.5.8.1	MAC-CE based active TCI state switch	2316

A.7.5.8.2	RRC based active TCI state switch	2320

A.7.5.9	Uplink spatial relation switch delay	2324

A.7.5.9.1	MAC-CE based Spatial Relation switch	2324

A.7.5.9.1.1	 NR PCell FR2 spatial relation associated with known DL-RS	2324

A.7.5.9.1.1.2	Test Requirements	2327

A.7.5.9.2	RRC based spatial relation switch	2327

A.7.5.9.2.1	NR PCell FR2 spatial relation switch associated with a known DL-RS	2327

A.7.5.9.2.1.1	Test Purpose and Environment	2327

A.7.5.9.2.1.2	Test Requirements	2330

A.7.5.10	UE specific CBW change	2330

A.7.5.10.1	NR FR2 UE specific CBW change of PCell with non-DRX in SA	2330

A.7.5.10.1.1	Test Purpose and Environment	2330

A.7.5.10.1.2	Test Requirements	2333

A.7.6	Measurement procedure	2334

A.7.6.1	Intra-frequency Measurements	2334

A.7.6.1.1	SA event triggered reporting test without gap under non-DRX	2334

A.7.6.1.1.1	Test purpose and Environment	2334

A.7.6.1.1.2	Test Requirements	2336

A.7.6.1.2	SA event triggered reporting test without gap under DRX	2337

A.7.6.1.2.1	Test purpose and Environment	2337

A.7.6.1.2.2	Test Requirements	2339

A.7.6.1.3	SA event triggered reporting test with per-UE gaps under non-DRX	2340

A.7.6.1.3.1	Test purpose and Environment	2340

A.7.6.1.3.2	Test Requirements	2342

A.7.6.1.4	SA event triggered reporting test with per-UE gaps under DRX	2343

A.7.6.1.4.1	Test purpose and Environment	2343

A.7.6.1.4.2	Test Requirements	2346

A.7.6.2	Inter-frequency Measurements	2347

A.7.6.2.1	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2)	2347

A.7.6.2.1.1	Test Purpose and Environment	2347

A.7.6.2.1.2	Test Requirements	2350

A.7.6.2.2	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is used (PCell in FR2)	2350

A.7.6.2.2.1	Test Purpose and Environment	2350

A.7.6.2.2.2	Test Requirements	2354

A.7.6.2.3	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is not used (PCell in FR2)	2354

A.7.6.2.3.1	Test Purpose and Environment	2354

A.7.6.2.3.2	Test Requirements	2358

A.7.6.2.4	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is used (PCell in FR2)	2358

A.7.6.2.4.1	Test Purpose and Environment	2358

A.7.6.2.4.2	Test Requirements	2362

A.7.6.2.5	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR1)	2362

A.7.6.2.5.1	Test Purpose and Environment	2362

A.7.6.2.5.2	Test Requirements	2366

A.7.6.2.6	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (PCell in FR1)	2367

A.7.6.2.6.1	Test Purpose and Environment	2367

A.7.6.2.6.2	Test Requirements	2371

A.7.6.2.7	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR1)	2372

A.7.6.2.7.1	Test Purpose and Environment	2372

A.7.6.2.7.2	Test Requirements	2376

A.7.6.2.8	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR1)	2377

A.7.6.2.8.1	Test Purpose and Environment	2377

A.7.6.2.8.2	Test Requirements	2381

A.7.6.2.9	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2) (rel16 additional mandatory gap pattern 17)	2382

A.7.6.2.9.1	Test Purpose and Environment	2382

A.7.6.2.9.2	Test Requirements	2385

A.7.6.2.10	SA event triggered reporting test without gap under non-DRX	2385

A.7.6.2.10.1	Test Purpose and Environment	2385

A.7.6.2.10.2	Test Requirements	2387

A.7.6.2.11	SA event triggered reporting test without gap under DRX	2387

A.7.6.2.11.1	Test Purpose and Environment	2387

A.7.6.2.11.2	Test Requirements	2390

A.7.6.3	L1-RSRP measurement for beam reporting	2391

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	2391

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	2391

A.7.6.3.1.1	Test Purpose and Environment	2391

A.7.6.3.1.2	Test parameters	2391

A.7.6.3.1.3	Test Requirements	2393

A.7.6.3.2	SSB based L1-RSRP measurement when DRX is used	2393

A.7.6.3.2.1	Test Purpose and Environment	2393

A.7.6.3.2.2	Test parameters	2394

A.7.6.3.2.3	Test Requirements	2396

A.7.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	2396

A.7.6.3.3.1	Test Purpose and Environment	2396

A.7.6.3.3.2	Test parameters	2397

A.7.6.3.3.3	Test Requirements	2399

A.7.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	2400

A.7.6.3.4.1	Test Purpose and Environment	2400

A.7.6.3.4.2	Test parameters	2400

A.7.6.3.3.3	Test Requirements	2402

A.7.6.4	CLI measurements	2403

A.7.6.4.1	SRS-RSRP measurement with non-DRX	2403

A.7.6.4.1.1	Test Purpose and Environment	2403

A.7.6.4.1.2	Test Parameters	2403

A.7.6.4.1.3	Test Requirements	2405

A.7.6.4.2	CLI-RSSI measurement with non-DRX	2405

A.7.6.4.2.1	Test Purpose and Environment	2405

A.7.6.4.2.2	Test Parameters	2406

A.7.6.4.2.3	Test Requirements	2407

A.7.6.5	NR Measurements with autonomous gaps	2408

A.7.6.5.1	SA interfrequency CGI reporting in autonomous gaps test (PCell in FR2)	2408

A.7.6.5.1.1	Test Purpose and Environment	2408

A.7.6.5.1.2	Test Requirements	2411

A.7.6.6	L1-SINR measurement for beam reporting	2411

A.7.6.6.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured when DRX is not used	2411

A.7.6.6.1.1	Test Purpose and Environment	2411

A.7.6.6.1.2	Test parameters	2411

A.7.6.6.1.3	Test Requirements	2413

A.7.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	2413

A.7.6.6.2.1	Test Purpose and Environment	2413

A.7.6.6.2.2	Test parameters	2414

A.7.6.6.2.3	Test Requirements	2416

A.7.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	2416

A.7.6.6.3.1	Test Purpose and Environment	2416

A.7.6.6.3.2	Test parameters	2417

A.7.6.6.3.3	Test Requirements	2419

A.7.6.7	CSI-RS based intra-frequency Measurements	2419

A.7.6.7.1	SA event triggered reporting test without gap under DRX for CSI-RS based intra-frequency measurement	2419

A.7.6.7.1.1	Test purpose and Environment	2419

A.7.6.7.1.2	Test Requirements	2422

A.7.6.8	CSI-RS based inter-frequency Measurements	2422

A.7.6.8.1	SA event triggered reporting tests for FR2 CSI-RS based measurement when non-DRX is used (PCell in FR2)	2422

A.7.6.8.1.1	Test Purpose and Environment	2422

A.7.6.2.2.2	Test Requirements	2426

A.7.6.9	RSTD measurements	2426

A.7.6.9.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA	2426

A.7.6.9.1.1	Test Purpose and Environment	2426

A.7.6.9.1.2	Test Requirements	2432

A.7.6.9.2	 NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR2 SA	2432

A.7.6.9.2.1	Test Purpose and Environment	2432

A.7.6.9.2.2	Test Requirements	2439

A.7.6.10 PRS-RSRP measurements	2439

A.7.6.10.1 PRS-RSRP reporting delay test case for single positioning frequency layer	2439

A.7.6.10.1.1	Test Purpose and Environment	2439

A.7.6.10.1.2	Test Requirements	2443

A.7.6.10.2 PRS-RSRP reporting delay test case for dual positioning frequency layer	2443

A.7.6.10.2.1	Test Purpose and Environment	2443

A.7.6.10.2.2	Test Requirements	2446

A.7.6.11	UE Rx-Tx time difference measurements	2446

A.7.6.11.1 UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA	2446

A.7.6.11.1.1	Test purpose and environment	2446

A.7.6.11.1.2	Test requirements	2450

A.7.6.11.2 UE Rx-Tx time difference measurement period for dual positioning frequency layers in FR2 SA	2450

A.7.6.11.2.1	Test purpose and environment	2450

A.7.6.11.2.2	Test requirements	2454

A.7.7	Measurement Performance requirements	2454

A.7.7.1	SS-RSRP	2455

A.7.7.1.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2455

A.7.7.1.1.1	Test Purpose and Environment	2455

A.7.7.1.1.2	Test parameters	2455

A.7.7.1.1.3	Test Requirements	2459

A.7.7.1.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2459

A.7.7.1.2.1	Test Purpose and Environment	2459

A.7.7.1.2.2	Test parameters	2460

A.7.7.1.2.3	Test Requirements	2464

A.7.7.1.3	SA inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	2465

A.7.7.1.3.1	Test Purpose and Environment	2465

A.7.7.1.3.2	Test parameters	2465

A.7.7.1.3.3	Test Requirements	2469

A.7.7.2	SS-RSRQ	2469

A.7.7.2.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	2469

A.7.7.2.1.1	Test Purpose and Environment	2469

A.7.7.2.1.2	Test Parameters	2469

A.7.7.2.1.3	Test Requirements	2471

A.7.7.2.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2471

A.7.7.2.2.1	Test Purpose and Environment	2471

A.7.7.2.2.2	Test Parameters	2471

A.7.7.2.2.3	Test Requirements	2473

A.7.7.3	SS-SINR	2473

A.7.7.3.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2473

A.7.7.3.1.1	Test Purpose and Environment	2473

A.7.7.3.1.2	Test Parameters	2473

A.7.7.3.1.3	Test Requirements	2476

A.7.7.3.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2476

A.7.7.3.2.1	Test Purpose and Environment	2476

A.7.7.3.2.2	Test Parameters	2476

A.7.7.3.2.3	Test Requirements	2478

A.7.7.4	L1-RSRP measurement for beam reporting	2478

A.7.7.4.1	SSB based L1-RSRP measurement	2478

A.7.7.4.1.1	Test Purpose and Environment	2478

A.7.7.4.1.2	Test parameters	2479

A.7.7.4.1.3	Test Requirements	2481

A.7.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	2482

A.7.7.4.2.1	Test Purpose and Environment	2482

A.7.7.4.2.2	Test parameters	2482

A.7.7.4.2.3	Test Requirements	2484

A.7.7.5	CLI measurements	2485

A.7.7.5.1	SA SRS-RSRP measurement accuracy with FR2 serving cell	2485

A.7.7.5.1.1	Test Purpose and Environment	2485

A.7.7.5.1.2	Test parameters	2485

A.7.7.5.1.3	Test Requirements	2488

A.7.7.5.2	SA CLI-RSSI measurement accuracy with FR2 serving cell	2489

A.7.7.5.2.1	Test Purpose and Environment	2489

A.7.7.5.2.2	Test parameters	2489

A.7.7.5.2.3	Test Requirements	2491

A.7.7.6	L1-SINR measurement for beam reporting	2492

A.7.7.6.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	2492

A.7.7.6.1.1	Test Purpose and Environment	2492

A.7.7.6.1.2	Test parameters	2492

A.7.7.6.1.3	Test Requirements	2494

A.7.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	2495

A.7.7.6.2.1	Test Purpose and Environment	2495

A.7.7.6.2.2	Test parameters	2495

A.7.7.6.2.3	Test Requirements	2497

A.7.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	2498

A.7.7.6.3.1	Test Purpose and Environment	2498

A.7.7.6.3.2	Test parameters	2498

A.7.7.6.3.3	Test Requirements	2500

A.7.7.7	CSI-RSRP	2501

A.7.7.7.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2501

A.7.7.7.1.1	Test Purpose and Environment	2501

A.7.7.7.1.2	Test parameters	2501

A.7.7.7.1.3	Test Requirements	2504

A.7.7.7.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2505

A.7.7.7.2.1	Test Purpose and Environment	2505

A.7.7.7.2.2	Test parameters	2505

A.7.7.7.2.3	Test Requirements	2508

A.7.7.8	CSI-RSRQ	2509

A.7.7.8.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	2509

A.7.7.8.1.1	Test Purpose and Environment	2509

A.7.7.8.1.2	Test Parameters	2509

A.7.7.8.1.3	Test Requirements	2511

A.7.7.8.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2511

A.7.7.8.2.1	Test Purpose and Environment	2511

A.7.7.8.2.2	Test Parameters	2511

A.7.7.8.2.3	Test Requirements	2513

A.7.7.9	CSI-SINR	2513

A.7.7.9.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2513

A.7.7.9.1.1	Test Purpose and Environment	2513

A.7.7.9.1.2	Test Parameters	2513

A.7.7.9.1.3	Test Requirements	2516

A.7.7.9.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2516

A.7.7.9.2.2	Test Parameters	2516

A.7.7.9.2.3	Test Requirements	2518

A.7.7.10	RSTD measurements	2518

A.7.7.10.1	RSTD measurement accuracy test case for single positioning frequency layer	2518

A.7.7.10.1.1	Test purpose and Environment	2518

A.7.7.10.1.2	Test Requirements	2521

A.7.7.10.2	RSTD measurement accuracy test case for dual positioning frequency layer	2521

A.7.7.10.2.1	Test purpose and Environment	2521

A.7.7.10.2.2	Test Requirements	2524

A.7.7.11	PRS-RSRP measurements	2524

A.7.7.11.1	SA measurement accuracy with PRS in FR2	2524

A.7.7.11.1.1	Test Purpose and Environment	2524

A.7.7.11.1.2	Test parameters	2524

A.7.7.11.1.3	Test Requirements	2528

A.7.7.12	UE Rx-Tx time difference measurements	2529

A.7.7.12.1 UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR2 SA	2529

A.7.7.12.1.1	Test purpose and environment	2529

A.7.7.12.1.2	Test parameters	2529

A.7.7.12.1.3	Test requirements	2532

A.8	E-UTRA standalone tests for NR RRM	2532

A.8.1	Void	2533

A.8.2	RRC_IDLE state mobility	2533

A.8.2.1	Inter-RAT NR Cell re-selection	2533

A.8.2.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1	2533

A.8.2.1.1.1	Test Purpose and Environment	2533

A.8.2.1.1.2	Test Requirements	2538

A.8.2.1.2	E-UTRA Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16	2539

A.8.2.1.2.1	Test Purpose and Environment	2539

A.8.2.1.2.2	Test Requirements	2544

A.8.2.2	E-UTRA – NR Inter-RAT Early Measruement Reporting	2545

A.8.2.2.1	E-UTRA – NR Early Measurement Reporting for NR in FR1	2545

A.8.2.2.1.1	Test Purpose and Environment	2545

A.8.2.2.1.2	Test Requirements	2550

A.8.2.2.2	E-UTRA – NR Early Measurement Reporting for NR in FR2	2550

A.8.2.2.2.1	Test Purpose and Environment	2550

A.8.2.2.2.2	Test Requirements	2553

A.8.3	RRC_CONNECTED state mobility	2554

A.8.3.1	Handover	2554

A.8.3.1.1	E-UTRAN - NR handover in FR1	2554

A.8.3.1.1.1	Test Purpose and Environment	2554

A.8.3.1.1.2	Test Requirements	2559

A.8.4	Measurement procedure	2559

A.8.4.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay	2559

A.8.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay in non-DRX	2559

A.8.4.1.1.1	Test Purpose and Environment	2559

A.8.4.1.1.2	Test Requirements	2561

A.8.4.1.2	E-UTRA – NR Inter-RAT SFTD Measurement Delay in DRX	2562

A.8.4.1.2.1	Test Purpose and Environment	2562

A.8.4.1.2.2	Test Requirements	2563

A.8.4.2	E-UTRA – NR Inter-RAT Measurements	2563

A.8.4.2.1	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2563

A.8.4.2.1.1	Test Purpose and Environment	2563

A.8.4.2.1.2	Test Requirements	2568

A.8.4.2.2	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2568

A.8.4.2.2.1	Test Purpose and Environment	2568

A.8.4.2.2.2	Test Requirements	2572

A.8.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2572

A.8.4.2.3.1	Test Purpose and Environment	2572

A.8.4.2.3.2	Test Requirements	2576

A.8.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2576

A.8.4.2.4.1	Test Purpose and Environment	2576

A.8.4.2.4.2	Test Requirements	2580

A.8.4.2.5	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is not used	2580

A.8.4.2.5.1	Test Purpose and Environment	2580

A.8.4.2.5.2	Test Requirements	2582

A.8.4.2.6	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is used	2583

A.8.4.2.6.1	Test Purpose and Environment	2583

A.8.4.2.6.2	Test Requirements	2585

A.8.4.2.7	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is not used	2586

A.8.4.2.7.1	Test Purpose and Environment	2586

A.8.4.2.7.2	Test Requirements	2588

A.8.4.2.8	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is used	2589

A.8.4.2.8.1	Test Purpose and Environment	2589

A.8.4.2.8.2	Test Requirements	2591

A.8.4.2.9	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16	2592

A.8.4.2.9.1	Test Purpose and Environment	2592

A.8.4.2.9.2	Test Requirements	2597

A.8.5	Measurement performance	2597

A.8.5.1	SFTD accuracy	2597

A.8.5.1.1	SFTD accuracy	2597

A.8.5.1.1.1	Test Purpose	2597

A.8.5.1.1.2	Test Environment	2597

A.8.5.1.1.3	Test Requirements	2603

A.8.5.2	E-UTRA – NR Inter-RAT Measurement Performance requirements	2603

A.8.5.2.1.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	2603

A.8.5.2.1.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	2608

A.8.5.2.1.2.1	Test Purpose and Environment	2608

A.8.5.2.1.2.2	Test Parameters	2608

A.8.5.2.1.2.3	Test Requirements	2610

A.8.5.2.2	SS-RSRQ	2610

A.8.5.2.2.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	2610

A.8.5.2.2.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	2615

A.8.5.2.2.2.1	Test Purpose and Environment	2615

A.8.5.2.2.2.2	Test Parameters	2615

A.8.5.2.2.2.3	Test Requirements	2617

A.8.5.2.3	SS-SINR	2617

A.8.5.2.3.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	2617

A.8.5.2.3.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	2621

A.8.5.2.3.2.1	Test Purpose and Environment	2621

A.8.5.2.3.2.2	Test Parameters	2621

A.8.5.2.3.2.3	Test Requirements	2623

A.9	V2X Tests	2624

A.9.1	V2X Tests in FR1	2624

A.9.1.1	Test for V2X UE Transmit Timing	2624

A.9.1.1.1	Test for GNSS as Synchronization Reference Source	2624

A.9.1.1.1.1	Test Purpose and Environment	2624

A.9.1.1.1.2	Test requirements	2624

A.9.1.1.2	Test for SyncRef UE as Synchronization Reference Source	2624

A.9.1.1.2.1	Test Purpose and Environment	2624

A.9.1.1.2.2	Test requirements	2625

A.9.1.1.3	Test for FR1 NR Cell as Synchronization Reference Source	2625

A.9.1.1.3.1	Test Purpose and Environment	2625

A.9.1.1.3.2	Test requirements	2628

A.9.1.2	Test for Initiation/Cease of S-SSB Transmission with V2X Sidelink Communication	2628

A.9.1.2.1	Test for FR1 NR Cell as synchronization reference source without gap under non-DRX	2628

A.9.1.2.1.1	Test Purpose and Environment	2628

A.9.1.2.1.2	Test Requirements	2632

A.9.1.2.2	Test for SyncRef UE as synchronization reference source	2632

A.9.1.2.2.1	Test Purpose and Environment	2632

A.9.1.2.2.2	Test Requirements	2633

A.9.1.3	Test for V2X Synchronization Reference Selection/Reselection	2634

A.9.1.3.1	Test for GNSS configured as the highest priority	2634

A.9.1.3.1.1	Test Purpose and Environment	2634

A.9.1.3.1.2	Test Requirements	2636

A.9.1.3.2	 Test for FR1 NR Cell configured as the highest priority	2638

A.9.1.3.2.1	Test Purpose and Environment	2638

A.9.1.3.2.2	Test Requirements	2640

A.9.1.4	Test for L1 SL-RSRP Measurement	2641

A.9.1.4.1	Test for V2X UE Autonomous Resource Selection/Reselection	2641

A.9.1.4.1.1	Test Purpose and Environment	2641

A.9.1.4.1.2	Test Requirements	2644

A.9.1.4.2	Test for V2X UE Resource Pre-emption	2645

A.9.1.4.2.1	Test Purpose and Environment	2645

A.9.1.4.2.2	Test Requirements	2648

A.9.1.4.3	Test for V2X UE Resource Re-evaluation	2648

A.9.1.4.3.1	Test Purpose and Environment	2648

A.9.1.4.3.2	Test Requirements	2655

A.9.1.5	Test for Congestion Control Measurement	2655

A.9.1.5.1	Test Purpose and Environment	2655

A.9.1.5.2	Test Requirements	2661

A.9.1.6	Test for Interruption	2661

A.9.1.6.1	Test for Interruption to WAN due to V2X Sidelink Communication	2661

A.9.1.6.1.1	Test Purpose and Environment	2661

A.9.1.6.1.2	Test Requirements	2664

A.10	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	2664

A.10.1	RRC_CONNECTED state mobility	2664

A.10.1.1	RRC connection mobility control	2664

A.10.1.1.1	Random Access	2664

A.10.1.1.1.1	4-step RA type contention-based random access for NR PSCell with CCA	2664

A.10.1.1.1.1.1	Test Purpose and Environment	2664

A.10.1.1.1.1.2	Test Requirements	2667

A.10.1.1.1.1.2.1	Random Access Preamble Transmission	2667

A.10.1.1.1.1.2.2	Random Access Response Reception	2667

A.10.1.1.1.1.2.3	No Random Access Response Reception	2668

A.10.1.1.1.1.2.4	Receiving an UL grant for msg3 retransmission	2668

A.10.1.1.1.1.2.5	 Contention Resolution Timer expiry	2668

A.10.1.1.1.2	4-step RA type non-contention based random access for NR PSCell with CCA	2669

A.10.1.1.1.2.1	Test Purpose and Environment	2669

A.10.1.1.1.2.2	Test Requirements	2672

A.10.1.1.1.2.2.1	SSB-based Random Access Preamble Transmission	2672

A.10.1.1.1.2.2.2	Random Access Response Reception	2673

A.10.1.1.1.2.2.3	No Random Access Response Reception	2673

A.10.1.1.1.3	2-step RA type contention-based random access for NR PSCell with CCA	2673

A.10.1.1.1.3.1	Test Purpose and Environment	2673

A.10.1.1.1.3.2	Test Requirements	2677

A.10.1.1.1.3.2.1	MsgA Transmission	2677

A.10.1.1.1.3.2.2	MsgB Reception	2677

A.10.1.1.1.3.2.3	No MsgB Reception	2678

A.10.1.1.1.4	2-step RA type non-contention based random access for NR PSCell with CCA	2678

A.10.1.1.1.4.1	Test Purpose and Environment	2678

A.10.1.1.1.4.2	Test Requirements	2682

A.10.1.1.1.4.2.1	MsgA Transmission	2682

A.10.1.1.1.4.2.2	MsgB Reception	2683

A.10.1.1.1.4.2.3	No MsgB Reception	2683

A.10.2	Timing	2684

A.10.2.1	UE transmit timing	2684

A.10.2.2	UE timing advance	2688

A.10.3	Signalling characteristics	2691

A.10.3.1	Radio link monitoring	2691

A.10.3.1.1	Introduction	2691

A.10.3.1.2	Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	2692

A.10.3.1.2.1	Test purpose and environment	2692

A.10.3.1.2.2	Test requirements	2697

A.10.3.1.3	Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	2697

A.10.3.1.3.1	Test purpose and environment	2697

A.10.3.1.3.2	Test requirements	2703

A.10.3.1.4	Void	2703

A.10.3.1.4.1	Void	2703

A.10.3.1.4.2	Void	2703

A.10.3.1.5	Void	2703

A.10.3.1.5.1	Void	2703

A.10.3.1.5.2	Void	2703

A.10.3.2	Void	2704

A.10.3.3	SCell activation and deactivation delay	2704

A.10.3.3.2 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 640 ms SCell measurement cycle	2708

A.10.3.3.2.1	Test Purpose and Environment	2708

A.10.3.3.2.2	Test Requirements	2709

A.10.3.3.3 SCell Activation and Deactivation of unknown NR SCell with NR PSCell and NR SCell under CCA	2709

A.10.3.3.3.1	Test Purpose and Environment	2709

A.10.3.3.3.2	Test Requirements	2710

A.10.3.4	Beam failure detection and link recovery procedures	2710

A.10.3.4.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	2710

A.10.3.4.1.1	Test Purpose and Environment	2710

A.10.3.4.1.2	Test Requirements	2715

A.10.3.4.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	2715

A.10.3.4.2.1	Test Purpose and Environment	2715

A.10.3.4.2.2	Test Requirements	2721

A.10.3.5	Active BWP switching	2721

A.10.3.5.1	UL active BWP switch delay with consistent UL LBT failure on PSCell subject to UL CCA in EN-DC	2721

A.10.3.5.1.1	Test Purpose and Environment	2721

A.10.3.5.1.2	Test Requirements	2726

A.10.3.5.2	DCI-based and Timer-based Active BWP Switch	2727

A.10.3.5.2.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	2727

A.10.3.5.2.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	2730

A.10.3.5.3	RRC-based Active BWP Switch	2734

A.10.3.6	PSCell addition and release delay	2738

A.10.3.6.1	Addition and Release Delay of known NR PSCell on the carrier under CCA	2738

A.10.3.6.1.1	Test purpose and environment	2738

A.10.3.6.1.2	Test Requirements	2743

A.10.3.7	Void	2744

A.10.4	Measurement procedure	2744

A.10.4.1	Intra-frequency measurements	2744

A.10.4.1.1	Event-triggered reporting tests on PSCC without gaps under non-DRX	2744

A.10.4.1.1.1	Test purpose and environment	2744

A.10.4.1.1.2	Test parameters	2744

A.10.4.1.1.3	Test Requirements	2748

A.10.4.1.2	Void	2748

A.10.4.1.3	Void	2748

A.10.4.1.4	Event-triggered reporting tests on PSCC with per-UE gaps under DRX	2748

A.10.4.1.4.1	Test purpose and environment	2748

A.10.4.1.4.2	Test parameters	2749

A.10.4.1.4.3	Test Requirements	2753

A.10.4.1.5	Void	2754

A.10.4.1.6	Void	2754

A.10.4.1.7	Void	2754

A.10.4.1.8	Void	2754

A.10.4.1.9	Void	2754

A.10.4.1.10	Void	2754

A.10.4.1.11	Void	2754

A.10.4.1.12	Void	2754

A.10.4.2	Inter-frequency measurements	2754

A.10.4.2.1	Void	2754

A.10.4.2.2	Void	2754

A.10.4.2.3	EN-DC event triggered reporting tests for FR1 with CCA cell without SSB time index detection when DRX is not used	2754

A.10.4.2.3.1	Test Purpose and Environment	2754

A.10.4.2.3.2	Test Requirements	2758

A.10.4.2.4	EN-DC event triggered reporting tests for FR1 cell with CCA without SSB time index detection when DRX is used	2759

A.10.4.2.4.1	Test Purpose and Environment	2759

A.10.4.2.4.2	Test Requirements	2763

A.10.4.2.5	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is not used	2764

A.10.4.2.5.1	Test Purpose and Environment	2764

A.10.4.2.5.2	Test Requirements	2768

A.10.4.2.6	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is used	2769

A.10.4.2.6.1	Test Purpose and Environment	2769

A.10.4.2.6.2	Test Requirements	2773

A.10.4.2.7	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	2774

A.10.4.2.7.1	Test Purpose and Environment	2774

A.10.4.2.7.2	Test Requirements	2780

A.10.4.2.8	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	2780

A.10.4.2.8.1	Test Purpose and Environment	2780

A.10.4.2.8.2	Test Requirements	2786

A.10.4.2.9	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	2787

A.10.4.2.9.1	Test Purpose and Environment	2787

A.10.4.2.9.2	Test Requirements	2792

A.10.4.2.10	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	2792

A.10.4.2.10.1	Test Purpose and Environment	2792

A.10.4.2.10.2	Test Requirements	2798

A.10.4.3	L1-RSRP measurements for beam reporting	2799

A.10.4.3.1	SSB based L1-RSRP measurement on PSCC when DRX is not used	2799

A.10.4.3.1.1	Test Purpose and Environment	2799

A.10.4.3.1.2	Test parameters	2799

A.10.4.3.1.3	Test Requirements	2801

A.10.4.3.2	SSB based L1-RSRP measurement on PSCC when DRX is used	2802

A.10.4.3.2.1	Test Purpose and Environment	2802

A.10.4.3.2.2	Test parameters	2802

A.10.4.3.2.3	Test Requirements	2804

A.10.4.3.3	SSB based L1-RSRP measurement on SCC when DRX is not used	2805

A.10.4.3.3.1	Test Purpose and Environment	2805

A.10.4.3.3.2	Test parameters	2805

A.10.4.3.3.3	Test Requirements	2808

A.10.4.3.4	SSB based L1-RSRP measurement on SCC when DRX is used	2809

A.10.4.3.4.1	Test Purpose and Environment	2809

A.10.4.3.4.2	Test parameters	2809

A.10.4.3.4.3	Test Requirements	2812

A.10.4.4	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	2813

A.10.4.4.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2813

A.10.4.4.1.1	Test Purpose and Environment	2813

A.10.4.4.1.2	Test Requirements	2817

A.10.4.4.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2818

A.10.4.4.2.1	Test Purpose and Environment	2818

A.10.4.4.2.2	Test Requirements	2823

A.10.4.4.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2824

A.10.4.4.3.1	Test Purpose and Environment	2824

A.10.4.4.3.2	Test Requirements	2828

A.10.4.4.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2829

A.10.4.4.4.1	Test Purpose and Environment	2829

A.10.4.4.4.2	Test Requirements	2834

A.10.5	Measurement performance	2835

A.10.5.1	SS-RSRP	2835

A.10.5.1.1	Intra-frequency measurement accuracy on a CCA serving cell	2835

A.10.5.1.1.1	Test Purpose and Environment	2835

A.10.5.1.1.2	Test parameters	2835

A.10.5.1.1.3	Test Requirements	2838

A.10.5.1.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	2838

A.10.5.1.2.1	Test Purpose and Environment	2838

A.10.5.1.2.2	Test parameters	2838

A.10.5.1.2.3	Test Requirements	2842

A.10.5.2	SS-RSRQ	2842

A.10.5.2.1	Intra-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	2842

A.10.5.2.1.1	Test Purpose and Environment	2842

A.10.5.2.1.2	Test Parameters	2842

A.10.5.2.1.3	Test Requirements	2845

A.10.5.2.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	2845

A.10.5.2.2.1	Test Purpose and Environment	2845

A.10.5.2.2.2	Test Parameters	2845

A.10.5.2.2.3	Test Requirements	2848

A.10.5.3	SS-SINR	2848

A.10.5.3.1	Intra-frequency measurement accuracy on PSCC	2848

A.10.5.3.1.1	Test Purpose and Environment	2848

A.10.5.3.1.2	Test Parameters	2848

A.10.5.3.1.3	Test Requirements	2851

A.10.5.3.2	Inter-frequency measurement accuracy on PSCC	2851

A.10.5.3.2.1	Test Purpose and Environment	2851

A.10.5.3.2.2	Test Parameters	2851

A.10.5.3.2.3	Test Requirements	2855

A.10.5.3.3	Intra-frequency measurement accuracy on SCC	2855

A.10.5.3.3.1	Test Purpose and Environment	2855

A.10.5.3.3.2	Test Parameters	2855

A.10.5.3.3.3	Test Requirements	2858

A.10.5.4	L1-RSRP measurement for beam reporting with CCA serving cell	2858

A.10.5.4.1	SSB based L1-RSRP measurement	2858

A.10.5.4.1.1	Test Purpose and Environment	2858

A.10.5.4.1.2	Test parameters	2859

A.10.5.4.1.3	Test Requirements	2861

A.10.5.5	RSSI	2861

A.10.5.5.1 	RSSI measurement accuracy on PSCC with CCA	2861

A.10.5.5.1.1	Test Purpose and Environment	2861

A.10.5.5.1.2	Test parameters	2861

A.10.5.5.1.3	Test Requirements	2864

A.10.5.5.2 	RSSI measurement accuracy on SCC with CCA	2864

A.10.5.5.2.1	Test Purpose and Environment	2864

A.10.5.5.2.2	Test parameters	2864

A.10.5.5.2.3	Test Requirements	2867

A.10.5.5.3 	Inter-frequency RSSI measurement accuracy on a carrier with CCA	2867

A.10.5.5.3.1	Test Purpose and Environment	2867

A.10.5.5.3.2	Test parameters	2867

A.10.5.5.3.3	Test Requirements	2871

A.10.5.6	Channel occupancy	2871

A.10.5.6.1 	Channel occupancy measurement accuracy on PSCC with CCA	2871

A.10.5.6.1.1	Test Purpose and Environment	2871

A.10.5.6.1.2	Test parameters	2871

A.10.5.6.1.3	Test Requirements	2875

A.10.5.6.2 	Channel occupancy measurement accuracy on SCC with CCA	2875

A.10.5.6.2.1	Test Purpose and Environment	2875

A.10.5.6.2.2	Test parameters	2875

A.10.5.6.2.3	Test Requirements	2878

A.10.5.6.3 	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	2878

A.10.5.6.3.1	Test Purpose and Environment	2878

A.10.5.6.3.2	Test parameters	2878

A.10.5.6.3.3	Test Requirements	2882

A.11	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	2882

A.11.1	RRC_IDLE state mobility	2883

A.11.1.1	Cell re-selection with both source and target NR carrier frequencies under CCA	2883

A.11.1.1.1	Cell reselection to FR1 intra-frequency NR cells when subject to CCA on the serving and target cell	2883

A.11.1.1.1.1	Test Purpose and Environment	2883

A.11.1.1.1.2	Test Parameters	2883

A.11.1.1.1.3	Test Requirements	2886

A.11.1.1.2	Cell reselection to FR1 inter-frequency NR case when subject to CCA on the serving and target cell	2886

A.11.1.1.2.1	Test Purpose and Environment	2886

A.11.1.1.2.2	Test Parameters	2886

A.11.1.1.2.3	Test Requirements	2890

A.11.1.2	Cell re-selection to NR with source NR carrier frequency under CCA	2890

A.11.1.2.1	Cell reselection to FR1 inter-frequency NR case when serving cell is subject to CCA	2890

A.11.1.2.1.1	Test Purpose and Environment	2890

A.11.1.2.1.2	Test Parameters	2891

A.11.1.2.1.3	Test Requirements	2897

A.11.1.3	Cell re-selection from NR carrier with target NR carrier frequency under CCA	2898

A.11.1.3.1	Cell reselection to FR1 inter-frequency NR case when target cell is subject to CCA	2898

A.11.1.3.1.1	Test Purpose and Environment	2898

A.11.1.3.1.2	Test Parameters	2898

A.11.1.3.1.3	Test Requirements	2903

A.11.1.4	Inter-RAT cell re-selection to E-UTRAN with source NR carrier frequency under CCA	2904

A.11.1.4.1	Cell reselection to higher priority E-UTRAN when serving cell is subject to CCA	2904

A.11.1.4.1.1	Test Purpose and Environment	2904

A.11.1.4.1.2	Test Parameters	2904

A.11.1.4.1.3	Test Requirements	2907

A.11.1.4.2	Cell reselection to lower priority E-UTRAN when serving cell is subject to CCA	2908

A.11.1.4.2.1	Test Purpose and Environment	2908

A.11.1.4.2.2	Test Requirements	2910

A.11.2	RRC_CONNECTED state mobility	2911

A.11.2.1	Handover	2911

A.11.2.1.1	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; known target cell	2911

A.11.2.1.1.1	Test Purpose and Environment	2911

A.11.2.1.1.2	Test Parameters	2911

A.11.2.1.1.3 Test Requirements	2914

A.11.2.1.2	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	2914

A.11.2.1.2.1	Test Purpose and Environment	2914

A.11.2.1.2.2	Test Parameters	2914

A.11.2.1.2.3	Test Requirements	2917

A.11.2.1.3	Inter-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	2917

A.11.2.1.3.1	Test Purpose and Environment	2917

A.11.2.1.3.2	Test Parameters	2917

A.11.2.1.3.3 Test Requirements	2920

A.11.2.1.4	Inter-frequency handover from FR1 carrier under CCA to FR1; known target cell	2920

A.11.2.1.4.1	Test Purpose and Environment	2920

A.11.2.1.4.2	Test Parameters	2920

A.11.2.1.4.3 Test Requirements	2924

A.11.2.1.5	Inter-frequency handover from FR1 carrier under CCA to FR1; unknown target cell	2925

A.11.2.1.5.1	Test Purpose and Environment	2925

A.11.2.1.5.2	Test Parameters	2925

A.11.2.1.5.3 Test Requirements	2928

A.11.2.1.6	Inter-frequency handover from FR1 to FR1 carrier under CCA; unknown target cell	2929

A.11.2.1.6.1	Test Purpose and Environment	2929

A.11.2.1.6.2	Test Parameters	2929

A.11.2.1.6.3 Test Requirements	2932

A.11.2.1.7	 SA NR FR1 carrier under CCA - E-UTRAN handover with known target cell	2933

A.11.2.1.7.1	Test Purpose and Environment	2933

A.11.2.1.7.2	Test Requirements	2939

A.11.2.1.8	SA NR FR1 carrier under CCA - E-UTRAN handover with unknown target cell	2939

A.11.2.1.8.1	Test Purpose and Environment	2939

A.11.2.1.8.2	Test Requirements	2944

A.11.2.2	RRC connection mobility control	2944

A.11.2.2.1	RRC re-establishment	2944

A.11.2.2.1.1	Intra-frequency RRC Re-establishment with CCA in FR1	2944

A.11.2.2.1.2	Inter-frequency RRC Re-establishment with CCA in FR1	2947

A.11.2.2.1.3	Intra-frequency RRC Re-establishment with CCA in FR1 without serving cell timing	2951

A.11.2.2.1.4	Inter-frequency RRC Re-establishment from NR FR1 carrier without CCA to NR FR1 carrier under CCA	2954

A.11.2.2.2	Random Access	2959

A.11.2.2.2.1	4-step RA type contention-based random access for NR PCell with CCA	2959

A.11.2.2.2.1.1	Test Purpose and Environment	2959

A.11.2.2.2.1.2	Test Requirements	2962

A.11.2.2.2.1.2.1	Random Access Preamble Transmission	2962

A.11.2.2.2.1.2.2	Random Access Response Reception	2962

A.11.2.2.2.1.2.3	No Random Access Response Reception	2963

A.11.2.2.2.1.2.4	Receiving an UL grant for msg3 retransmission	2963

A.11.2.2.2.1.2.5	Reception of an Incorrect Message over Temporary C-RNTI	2963

A.11.2.2.2.1.2.6	Reception of a Correct Message over Temporary C-RNTI	2964

A.11.2.2.2.1.2.7	Contention Resolution Timer expiry	2964

A.11.2.2.2.2	4-step RA type non-contention based random access for NR PSCell with CCA	2964

A.11.2.2.2.2.1	Test Purpose and Environment	2964

A.11.2.2.2.2.2	Test Requirements	2967

A.11.2.2.2.2.2.1	SSB-based Random Access Preamble Transmission	2967

A.11.2.2.2.2.2.2	Random Access Response Reception	2968

A.11.2.2.2.2.2.3	No Random Access Response Reception	2968

A.11.2.2.2.3	2-step RA type contention-based random access for NR PCell with CCA	2968

A.11.2.2.2.3.1	Test Purpose and Environment	2968

A.11.2.2.2.3.2	Test Requirements	2972

A.11.2.2.2.3.2.1	MsgA Transmission	2972

A.11.2.2.2.3.2.2	MsgB Reception	2972

A.11.2.2.2.3.2.3	No MsgB Reception	2973

A.11.2.2.2.4	2-step RA type non-contention-based random access for NR PCell with CCA	2973

A.11.2.2.2.4.1	Test Purpose and Environment	2973

A.11.2.2.2.4.2	Test Requirements	2977

A.11.2.2.2.4.2.1	MsgA Transmission	2977

A.11.2.2.2.4.2.2	MsgB Reception	2978

A.11.2.2.2.4.2.3	No MsgB Reception	2978

A.11.2.2.3	RRC connection release with redirection	2979

A.11.2.2.3.1	Redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA	2979

A.11.3	Timing	2987

A.11.3.1	UE transmit timing	2987

A.11.3.2	UE timing advance	2991

A.11.3.2.1	UE Timing Advance Adjustment Accuracy with PCell under DL CCA	2991

A.11.3.2.1.1	Test Purpose and Environment	2991

A.11.3.2.1.2	Test Parameters	2991

A.11.3.2.1.3	Test Requirements	2995

A.11.4	Signalling characteristics	2995

A.11.4.1	Radio link monitoring	2995

A.11.4.1.1	Introduction	2995

A.11.4.1.2	Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	2996

A.11.4.1.2.1	Test purpose and environment	2996

A.11.4.1.2.2	Test requirements	3000

A.11.4.1.3	Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	3000

A.11.4.1.3.1	Test purpose and environment	3000

A.11.4.1.3.2	Test requirements	3006

A.11.4.1.4	Void	3006

A.11.4.1.4.1	Void	3006

A.11.4.1.4.2	Void	3006

A.11.4.1.5	Void	3006

A.11.4.1.5.1	Void	3006

A.11.4.1.5.2	Void	3006

A.11.4.2	Void	3007

A.11.4.3	SCell activation and deactivation delay	3007

A.11.4.3.2	SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 640 ms SCell measurement cycle	3011

A.11.4.3.2.1	Test Purpose and Environment	3011

A.11.4.3.2.2	Test Requirements	3012

A.11.4.4	Beam failure detection and link recovery procedures	3013

A.11.4.4.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	3013

A.11.4.4.1.1	Test Purpose and Environment	3013

A.11.4.4.1.2	Test Requirements	3019

A.11.4.4.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	3019

A.11.4.4.2.1	Test Purpose and Environment	3019

A.11.4.4.2.2	Test Requirements	3025

A.11.4.5	Active BWP switching	3025

A.11.4.5.1	UL active BWP switch delay with consistent UL LBT failure on PCell subject to UL CCA	3025

A.11.4.5.1.1	Test Purpose and Environment	3025

A.11.4.5.1.2	Test Requirements	3029

A.11.4.6	Void	3039

A.11.5	Measurement procedure	3039

A.11.5.1	Intra-frequency measurements	3039

A.11.5.1.1	Event-triggered reporting tests on PCC without gaps under non-DRX	3039

A.11.5.1.1.1	Test purpose and environment	3039

A.11.5.1.1.2	Test parameters	3039

A.11.5.1.1.3	Test Requirements	3043

A.11.5.1.2	Event-triggered reporting tests on PCC without gaps under DRX	3043

A.11.5.1.2.1	Test purpose and environment	3043

A.11.5.1.2.2	Test parameters	3043

A.11.5.1.2.3	Test Requirements	3047

A.11.5.1.3	Event-triggered reporting tests on PCC with per-UE gaps under non-DRX	3047

A.11.5.1.3.1	Test purpose and environment	3047

A.11.5.1.3.2	Test parameters	3047

A.11.5.1.3.3	Test Requirements	3047

A.11.5.1.4	Event-triggered reporting tests on PCC with per-UE gaps under DRX	3047

A.11.5.1.4.1	Test purpose and environment	3047

A.11.5.1.4.2	Test parameters	3048

A.11.5.1.4.3	Test Requirements	3048

A.11.5.1.5	Event-triggered reporting tests on SCC without gaps under non-DRX	3048

A.11.5.1.5.1	Test purpose and environment	3048

A.11.5.1.5.2	Test parameters	3048

A.11.5.1.6	Event-triggered reporting tests on SCC without gaps under DRX	3048

A.11.5.1.6.1	Test purpose and environment	3048

A.11.5.1.6.2	Test parameters	3048

A.11.5.1.6.3	Test Requirements	3048

A.11.5.1.7	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	3049

A.11.5.1.7.1	Test purpose and environment	3049

A.11.5.1.7.2	Test parameters	3049

A.11.5.1.7.3	Test Requirements	3049

A.11.5.1.8	Event-triggered reporting tests on SCC with per-UE gaps under DRX	3049

A.11.5.1.8.1	Test purpose and environment	3049

A.11.5.1.8.2	Test parameters	3049

A.11.5.1.8.3	Test Requirements	3049

A.11.5.1.9	Void	3050

A.11.5.1.10	Void	3050

A.11.5.1.11	Void	3050

A.11.5.1.12	Void	3050

A.11.5.2	Inter-frequency measurements	3050

A.11.5.2.1	Void	3050

A.11.5.2.2	Void	3050

A.11.5.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	3050

A.11.5.2.3.1	Test Purpose and Environment	3050

A.11.5.2.3.2	Test Requirements	3054

A.11.5.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	3055

A.11.5.2.4.1	Test Purpose and Environment	3055

A.11.5.2.4.2	Test Requirements	3060

A.11.5.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	3060

A.11.5.2.5.1	Test Purpose and Environment	3060

A.11.5.2.5.2	Test Requirements	3064

A.11.5.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	3065

A.11.5.2.6.1	Test Purpose and Environment	3065

A.11.5.2.6.2	Test Requirements	3070

A.11.5.2.7	Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	3070

A.11.5.2.7.1	Test Purpose and Environment	3070

A.11.5.2.7.2	Test Requirements	3074

A.11.5.2.8	Event triggered reporting tests for FR1 without SSB time index detection when DRX is used	3075

A.11.5.2.8.1	Test Purpose and Environment	3075

A.11.5.2.8.2	Test Requirements	3080

A.11.5.2.9	Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	3080

A.11.5.2.9.1	Test Purpose and Environment	3080

A.11.5.2.9.2	Test Requirements	3085

A.11.5.2.10	Event triggered reporting tests for FR1 with SSB time index detection when DRX is used	3086

A.11.5.2.10.1	Test Purpose and Environment	3086

A.11.5.2.10.2	Test Requirements	3091

A.11.5.3	Inter-RAT E-UTRAN measurements	3092

A.11.5.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1	3092

A.11.5.3.1.1	Test Purpose and Environment	3092

A.11.5.3.1.2	Test Requirements	3096

A.11.5.3.2	SA NR - E-UTRAN event-triggered reporting in DRX in FR1	3096

A.11.5.3.2.1	Test Purpose and Environment	3096

A.11.5.3.2.2	Test Requirements	3101

A.11.5.4	L1-RSRP measurements for beam reporting	3101

A.11.5.4.1	SSB based L1-RSRP measurement when DRX is not used	3101

A.11.5.4.1.1	Test Purpose and Environment	3101

A.11.5.4.1.2	Test parameters	3101

A.11.5.4.1.3	Test Requirements	3103

A.11.5.4.2	SSB based L1-RSRP measurement when DRX is used	3103

A.11.5.4.2.1	Test Purpose and Environment	3103

A.11.5.4.2.2	Test parameters	3104

A.11.5.4.2.3	Test Requirements	3106

A.11.5.4.3	SSB based L1-RSRP measurement on SCC when DRX is not used	3106

A.11.5.4.3.1	Test Purpose and Environment	3106

A.11.5.4.3.2	Test parameters	3107

A.11.5.4.3.3	Test Requirements	3110

A.11.5.4.4	SSB based L1-RSRP measurement on SCC when DRX is used	3111

A.11.5.4.4.1	Test Purpose and Environment	3111

A.11.5.4.4.2	Test parameters	3111

A.11.5.4.4.3	Test Requirements	3114

A.11.6	Measurement performance	3115

A.11.6.1	SS-RSRP	3115

A.11.6.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	3115

A.11.6.1.1.1	Test Purpose and Environment	3115

A.11.6.1.1.2	Test parameters	3115

A.11.6.1.1.3	Test Requirements	3117

A.11.6.1.2	Intra-frequency measurement accuracy on SCC on a carrier frequency with CCA	3117

A.11.6.1.2.1	Test Purpose and Environment	3117

A.11.6.1.2.2	Test parameters	3117

A.11.6.1.2.3	Test Requirements	3119

A.11.6.2	SS-RSRQ	3119

A.11.6.2.1	Intra-frequency measurement accuracy	3119

A.11.6.2.1.1	Test Purpose and Environment	3119

A.11.6.2.1.2	Test Parameters	3119

A.11.6.2.1.3	Test Requirements	3122

A.11.6.2.2	Inter-frequency measurement accuracy	3122

A.11.6.2.2.1	Test Purpose and Environment	3122

A.11.6.2.2.2	Test Parameters	3122

A.11.6.2.2.3	Test Requirements	3125

A.11.6.2.3	Intra-frequency measurement accuracy on SCC	3125

A.11.6.2.3.1	Test Purpose and Environment	3125

A.11.6.2.3.2	Test Parameters	3125

A.11.6.2.3.3	Test Requirements	3128

A.11.6.2.4	Inter-frequency measurement accuracy	3128

A.11.6.2.4.1	Test Purpose and Environment	3128

A.11.6.2.4.2	Test Parameters	3128

A.11.6.2.4.3	Test Requirements	3136

A.11.6.3	SS-SINR	3136

A.11.6.3.1	Intra-frequency measurement accuracy	3136

A.11.6.3.1.1	Test Purpose and Environment	3136

A.11.6.3.1.2	Test Parameters	3136

A.11.6.3.1.3	Test Requirements	3139

A.11.6.3.2	Inter-frequency measurement accuracy	3139

A.11.6.3.2.1	Test Purpose and Environment	3139

A.11.6.3.2.2	Test Parameters	3139

A.11.6.3.2.3	Test Requirements	3142

A.11.6.3.3	Intra-frequency measurement accuracy on SCC	3142

A.11.6.3.3.1	Test Purpose and Environment	3142

A.11.6.3.3.2	Test Parameters	3142

A.11.6.3.3.3	Test Requirements	3145

A.11.6.3.4	Inter-frequency measurement accuracy	3145

A.11.6.3.4.1	Test Purpose and Environment	3145

A.11.6.3.4.2	Test Parameters	3145

A.11.6.3.4.3	Test Requirements	3153

A.11.6.4	L1-RSRP measurement for beam reporting with CCA serving cell	3153

A.11.6.4.1	SSB based L1-RSRP measurement	3153

A.11.6.4.1.1	Test Purpose and Environment	3153

A.11.6.4.1.2	Test parameters	3154

A.11.6.4.1.3	Test Requirements	3157

A.11.6.5	RSSI	3157

A.11.6.5.1	Intra-frequency RSSI measurement accuracy on PCC with CCA	3157

A.11.6.5.1.1	Test Purpose and Environment	3157

A.11.6.5.1.2	Test parameters	3157

A.11.6.5.1.3	Test Requirements	3160

A.11.6.5.2 	Intra-frequency RSSI measurement accuracy on SCC with CCA	3160

A.11.6.5.2.1	Test Purpose and Environment	3160

A.11.6.5.2.2	Test parameters	3160

A.11.6.5.2.3	Test Requirements	3163

A.11.6.5.3 	Inter-frequency RSSI measurement accuracy on a carrier with CCA	3163

A.11.6.5.3.1	Test Purpose and Environment	3163

A.11.6.5.3.2	Test parameters	3163

A.11.6.5.3.3	Test Requirements	3166

A.11.6.6	Channel occupancy	3166

A.11.6.6.1 	Intra-frequency channel occupancy measurement accuracy on PCC with CCA	3166

A.11.6.6.1.1	Test Purpose and Environment	3166

A.11.6.6.1.2	Test parameters	3166

A.11.6.6.1.3	Test Requirements	3170

A.11.6.6.2 	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	3170

A.11.6.6.2.1	Test Purpose and Environment	3170

A.11.6.6.2.2	Test parameters	3170

A.11.6.6.2.3	Test Requirements	3173

A.11.6.6.3 	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	3173

A.11.6.6.3.1	Test Purpose and Environment	3173

A.11.6.6.3.2	Test parameters	3173

A.11.6.6.3.3	Test Requirements	3176

A.11.6.7	E-UTRAN RSRP	3176

A.11.6.8	E-UTRAN RSRQ	3176

A.11.6.9	E-UTRAN SINR	3176

A.12	E-UTRA Standalone Tests with at Least One NR Cell under CCA	3176

A.12.1	RRC_IDLE state mobility	3176

A.12.1.1	Inter-RAT cell re-selection to NR on a carrier frequency with CCA	3176

A.12.1.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1 when target cell is subject to CCA	3176

A.12.1.1.1.1	Test Purpose and Environment	3176

A.12.1.1.1.2	Test Requirements	3182

A.12.2	RRC_CONNECTED state mobility	3183

A.12.2.1	Handover	3183

A.12.2.1.1	E-UTRAN - NR with CCA handover	3183

A.12.2.1.1.1	Test Purpose and Environment	3183

A.12.2.1.1.2	Test Requirements	3188

A.12.3	Void	3189

A.12.3.1	Void	3189

A.12.4	Measurement procedure	3189

A.12.4.1	E-UTRANNR inter-RAT SFTD measurements	3189

A.12.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay with NR under CCA in non-DRX	3189

A.12.4.1.1.1	Test Purpose and Environment	3189

A.12.4.1.1.2	Test Requirements	3193

A.12.4.2	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	3193

A.12.4.2.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	3193

A.12.4.2.1.1	Test Purpose and Environment	3193

A.12.4.2.1.2	Test Requirements	3197

A.12.4.2.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	3198

A.12.4.2.2.1	Test Purpose and Environment	3198

A.12.4.2.2.2	Test Requirements	3202

A.12.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	3203

A.12.4.2.3.1	Test Purpose and Environment	3203

A.12.4.2.3.2	Test Requirements	3207

A.12.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	3208

A.12.4.2.4.1	Test Purpose and Environment	3208

A.12.4.2.4.2	Test Requirements	3212

A.12.4.2.5	RSSI measurement reporting	3213

A.12.4.2.5.1	Test purpose and environment	3213

A.12.4.2.5.2	Test parameters	3213

A.12.4.2.5.3	Test Requirements	3216

A.12.4.2.6	Channel occupancy measurement reporting	3216

A.12.4.2.6.1	Test purpose and environment	3216

A.12.4.2.6.2	Test parameters	3216

A.12.4.2.6.3	Test Requirements	3220

A.12.5	Measurement performance	3220

A.12.5.1	E-UTRANNR SFTD	3220

A.12.5.2	E-UTRANNR SS-RSRP	3227

A.12.5.3	E-UTRANNR SS-RSRQ	3227

A.12.5.4	E-UTRANNR SS-SINR	3227

A.12.5.5	E-UTRANNR RSSI	3227

A.12.5.6	E-UTRANNR channel occupancy	3227

A.13	 NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	3227

A.13.1	Void	3227

A.13.1.1	Void	3227

A.13.1.2	Void	3227

A.13.2	Signalling characteristics	3227

A.13.2.1	Void	3227

A.13.2.2	SCell activation and deactivation delay	3227

A.13.2.2.2	SCell Activation and Deactivation of known SCell under CCA, 640 ms SCell measurement cycle	3232

A.13.2.2.2.1	Test Purpose and Environment	3232

A.13.2.2.2.2	Test Requirements	3233

A.13.2.2.3	SCell Activation and Deactivation of unknown SCell under CCA	3233

A.13.2.2.3.1	Test Purpose and Environment	3233

A.13.2.2.3.2	Test Requirements	3234

A.13.2.3	Void	3234

A.13.3	Measurement procedure	3234

A.13.3.1	Intra-frequency measurements	3234

A.13.3.1.1	Event-triggered reporting tests on SCC without gaps under non-DRX	3234

A.13.3.1.1.1	Test purpose and environment	3234

A.13.3.1.1.2	Test parameters	3234

A.13.3.1.1.3	Test Requirements	3239

A.13.3.1.2	Event-triggered reporting tests on SCC without gaps under DRX	3239

A.13.3.1.2.1	Test purpose and environment	3239

A.13.3.1.2.2	Test parameters	3239

A.13.3.1.2.3	Test Requirements	3244

A.13.3.1.3	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	3244

A.13.3.1.3.1	Test purpose and environment	3244

A.13.3.1.3.2	Test parameters	3244

A.13.3.1.3.3	Test Requirements	3249

A.13.3.1.4	Event-triggered reporting tests on SCC with per-UE gaps under DRX	3249

A.13.3.1.4.1	Test purpose and environment	3249

A.13.3.1.4.2	Test parameters	3249

A.13.3.1.4.3	Test Requirements	3254

A.13.3.1.5	Void	3254

A.13.3.1.6	Void	3254

A.13.3.2	Inter-frequency measurements	3254

A.13.3.2.1	Void	3254

A.13.3.2.2	Void	3254

A.13.3.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	3254

A.13.3.2.3.1	Test Purpose and Environment	3254

A.13.3.2.3.2	Test Requirements	3259

A.13.3.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	3259

A.13.3.2.4.1	Test Purpose and Environment	3259

A.13.3.2.4.2	Test Requirements	3265

A.13.3.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	3266

A.13.3.2.5.1	Test Purpose and Environment	3266

A.13.3.2.5.2	Test Requirements	3271

A.13.3.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	3271

A.13.3.2.6.1	Test Purpose and Environment	3271

A.13.3.2.6.2	Test Requirements	3277

A.13.3.3	L1-RSRP measurements for beam reporting	3278

A.13.3.3.1	SSB based L1-RSRP measurement when DRX is not used	3278

A.13.3.3.1.1	Test Purpose and Environment	3278

A.13.3.3.1.2	Test parameters	3278

A.13.3.3.1.3	Test Requirements	3283

A.13.3.3.2	SSB based L1-RSRP measurement when DRX is used	3283

A.13.3.3.2.1	Test Purpose and Environment	3283

A.13.3.3.2.2	Test parameters	3283

A.13.3.3.2.3	Test Requirements	3287

A.13.4	Measurement performance	3287

A.13.4.1	SS-RSRP	3287

A.13.4.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	3287

A.13.4.1.1.1	Test Purpose and Environment	3287

A.13.4.1.1.2	Test parameters	3287

A.13.4.1.1.3	Test Requirements	3289

A.13.4.2	SS-RSRQ	3289

A.13.4.2.1	Intra-frequency measurement accuracy on SCC	3289

A.13.4.2.1.1	Test Purpose and Environment	3289

A.13.4.2.1.2	Test Parameters	3289

A.13.4.2.1.3	Test Requirements	3297

A.13.4.3	SS-SINR	3297

A.13.4.3.1	Intra-frequency measurement accuracy on SCC	3297

A.13.4.3.1.1	Test Purpose and Environment	3297

A.13.4.3.1.2	Test Parameters	3297

A.13.4.3.1.3	Test Requirements	3304

A.13.4.4	L1-RSRP measurement for beam reporting with CCA serving cell	3304

A.13.4.4.1	SSB based L1-RSRP measurement	3304

A.13.4.4.1.1	Test Purpose and Environment	3304

A.13.4.4.1.2	Test parameters	3305

A.13.4.4.1.3	Test Requirements	3309

A.13.4.5	RSSI	3309

A.13.4.5.1 	Intra-frequency RSSI measurement accuracy on a carrier with CCA	3309

A.13.4.5.1.1	Test Purpose and Environment	3309

A.13.4.5.1.2	Test parameters	3309

A.13.4.5.1.3	Test Requirements	3313

A.13.4.5.2	  Inter-frequency RSSI measurement accuracy on a carrier with CCA	3313

A.13.4.5.2.1	Test Purpose and Environment	3313

A.13.4.5.2.2	Test parameters	3313

A.13.4.5.2.3	Test Requirements	3317

A.13.4.6	Channel occupancy	3317

A.13.4.6.1 	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	3317

A.13.4.6.1.1	Test Purpose and Environment	3317

A.13.4.6.1.2	Test parameters	3317

A.13.4.6.1.3	Test Requirements	3321

A.13.4.6.2	  Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	3321

A.13.4.6.2.1	Test Purpose and Environment	3321

A.13.4.6.2.2	Test parameters	3321

A.13.4.6.2.3	Test Requirements	3325

Annex B (normative): Conditions for RRM requirements applicability for operating bands	3328

B.1	Conditions for NR RRC_IDLE state mobility	3328

B.1.1	Introduction	3328

B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection	3328

B.1.3	Conditions for measurements on NR inter-frequency cells for cell re-selection	3329

B.2	Conditions for UE measurements procedures and performance requirements in RRC_CONNECTED state	3330

B.2.1	Introduction	3330

B.2.1.1	General	3330

B.2.1.2	Derivation of Minimum SSB_RP values for FR1	3330

B.2.1.3	Derivation of Minimum SSB_RP values for FR2	3330

B.2.1.3.1	Minimum SSB_RP values for Rx Beam Peak angle of arrival	3330

B.2.1.3.2	Minimum SSB_RP values for angle of arrival within Spherical coverage	3331

B.2.1.4	Gain to SS-RSRP and CSI-RSRP measurement point for FR1	3331

B.2.1.5	Gain to SS-RSRP and CSI-RSRP measurement point for FR2	3332

B.2.1.5.1	Gain to SS-RSRP and CSI-RSRP measurement point for Rx Beam Peak angle of arrival	3332

B.2.1.5.2	Gain to SS-RSRP measurement point for different frequency	3332

B.2.1.5.3	Alignment of Rough beam to Rx beam Peak	3332

B.2.1.6	Gain to PRS-RSRP measurement point for FR2	3333

B.2.1.6.1	Gain to PRS-RSRP measurement point for Rx Beam Peak angle of arrival	3333

B.2.2	Conditions for NR intra-frequency measurements	3333

B.2.3	Conditions for NR inter-frequency measurements	3335

B.2.4	Conditions for NR L1-RSRP reporting	3336

B.2.4.1	Conditions for SSB based L1-RSRP reporting	3336

B.2.4.2	Conditions for CSI-RS based L1-RSRP reporting	3337

B.2.5	Conditions for RRC connection release with redirection to NR	3339

B.2.6	Void	3340

B.2.6.1	Void	3340

B.2.6.2	Void	3340

B.2.7	Conditions for SRS-RSRP measurements	3340

B.2.8	Conditions for NR L1-SINR reporting	3341

B.2.8.1	Conditions for L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	3341

B.2.8.2	Conditions for L1-SINR reporting with SSB based CMR and dedicated IMR configured	3342

B.2.8.2.1	L1-SINR reporting with SSB based CMR and dedicated ZP-IMR configured	3342

B.2.8.2.2	L1-SINR reporting with SSB based CMR and dedicated NZP-IMR configured	3342

B.2.8.3	Conditions for L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	3344

B.2.8.3.1	L1-SINR reporting with CSI-RS based CMR and dedicated ZP-IMR configured	3344

B.2.8.3.2	L1-SINR reporting with CSI-RS based CMR and dedicated NZP-IMR configured	3345

B.2.9	Conditions for NR intra-frequency measurements under CCA	3346

B.2.10	Conditions for NR inter-frequency measurements under CCA	3346

B.2.11	Conditions for NR L1-RSRP reporting under CCA	3346

B.2.11.1	Conditions for SSB based L1-RSRP reporting	3346

B.2.12	Conditions for NR CSI-RS based intra-frequency measurements	3347

B.2.13	Conditions for NR CSI-RS based inter-frequency measurements	3348

B.2.14	Conditions for NR PRS-based measurements	3350

B.3	RRM Requirements Exceptions	3351

B.3.1	Introduction	3351

B.3.2	Receiver sensitivity relaxation for CA	3351

B.3.2.1	Receiver sensitivity relaxation for UE supporting CA in FR1	3351

B.3.2.2	Receiver sensitivity relaxation for UE configured with CA in FR1	3352

B.3.2.2.1	Inter-band carrier aggregation	3352

B.3.2.2.2	Reference sensitivity exceptions due to UL harmonic interference for CA	3352

B.3.2.2.3	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA	3352

B.3.2.3	Receiver sensitivity relaxation for UE supporting CA in FR2	3352

B.3.2.4	Receiver sensitivity relaxation for UE configured with CA in FR2	3352

B.3.2.4.1	Intra-band contiguous carrier aggregation	3352

B.3.2.4.2	Intra-band non-contiguous carrier aggregation	3353

B.3.3	Receiver sensitivity relaxation for DC	3353

B.3.3.1	Receiver sensitivity relaxation for EN-DC	3353

B.3.3.2	Receiver sensitivity relaxation for NE-DC	3353

B.3.4	Receiver sensitivity relaxation for SUL	3353

B.3.4.1	Receiver sensitivity relaxation for UE supporting SUL in FR1	3353

B.3.4.2	Receiver sensitivity relaxation for UE configured with SUL in FR1	3353

B.3.4.2.1	Reference sensitivity exceptions due to UL harmonic interference for SUL	3353

B.4	Conditions for V2X	3353

B.4.1	Test parameters for GNSS signals	3353

B.4.2	Conditions for PSBCH-RSRP Accuracy Requirements	3354

B.4.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	3354

B.4.4	Conditions for L1 SL-RSRP Accuracy Requirements	3354

Annex C (informative): Change history	3356