ETSI/STC/SMG1 - Working Group	Tdoc SMG1-WG-RSS- TDoc ____/95

Roaming Services Support

Bonn, Germany

13 - 14 March 1995

Title:		Support of Optimal Routing

Source:	SMG1 Rapporteur

Support of Optimal Routing (SOR)


Service description, Stage 1

Proposed Version 0.43.0

146 MarchDecember, 1994

INTRODUCTION

At the SMG1 meeting in Cambridge the work for a stage 1 description of Optimal Routing, as requested by SMG and GSM MoU, has commenced. This draft reflects the policy decisions taken at the SMG1 meeting in Madrid and the conclusions of the SMG1 Working Party Roaming Services in Düsseldorf & Bonn. Especially it should be mentioned that all references to data privacy services have been erased (as decided by SMG1). This is a clean version, without revision marks.The revision marks reflect the changes since version 0.3.0 (16 December 1994).

There was no progress at the recent SMG1 meeting in Wien.

Editorial Note:

This draft standard forms the current achievements in SMG1 concerning the specification of all necessary means for the support of Optimal Routing.

A Footnote is inserted, in Italics, upon the following occasions:

-	Something is not covered by this first stage of SOR, but might be addressed at a later stage.

-	Other STC's are asked for guidance.

Table of contents

<to be inserted>

1	Scope

This Stage 1 description

·	compiles the basic service requirements for SOR, based on the principles elaborated by GSM MoU;

·	describes the interactions for Supplementary Services (SS) in order to cater for SOR;

·	refers to modifications to network features required by SOR.

This ETS does not address the following:

·	There is no need for optimisation of the routing of calls originally directed to a fixed network subscriber, because the physical address of a fixed network terminating line cannot differ from its logical address.

·	SOR in non-GSM mobile networks is not a subject of this ETS, but might be possible by bilateral arrangement between GSM network operators and those non-GSM mobile network operators.

[Editor’s Note: The Scope needs to be extended to cover the phased introduction of SOR.]

2	References

<to be inserted>

3	Definitions and abbreviations

·	Support of Optimal Routing (SOR):

It is a network feature which allows the routing of a call according to the rules specified in this standard. All PLMNs involved must support this feature otherwise reduced functionality, as described, will be experienced.

·	Direct route:

A call uses the direct route if it is routed from the serving network of the original calling party to the serving network of the ultimate called party without any intermediate exchange being involved in the computation of the charge for the callthe destination is defined by the ultimate called number (MSRN or forwarded-to number).

·	HPLMN route:

A call uses the HPLMN route if the destination is defined by the MSISDN of the called party; this forces the call to be routed via the HPLMN of the called party. If the called party is registered in the HPLMN, the direct route and the HPLMN route are the same.

·	HPLMN leg:

The HPLMN leg is that part of the HPLMN route from the originating switch to the HPLMN of the called party.

·	IPLMN:

The IPLMN is the PLMN which interrogates the HPLMN of a called or forwarded-to party to determine the whereabouts of that party.

4	Description of Optimal Routing

Support of Optimal Routing (SOR) is a network feature, by which calls directed to a mobile subscriber, who has roamed away from his HPLMN, are routed directly (instead of via the HPLMN) to the mobile subscriber's actual location, or the forwarded-to destination.

NOTE:	The principles for SOR may also apply within the HPLMN, but this is outside the scope of this ETS.

This ETS is elaborated upon the following charging principles:

SOR shall should only be used only if financial benefits are obtained and provided acceptable quality of service can be maintained. The application of SOR shall however not lead to any one user being charged more than would have been the case without SOR. The financial benefits of SOR shall be applied in the following order of priority:

1.	forwardingcalled party or parties; if there are two or more forwarding parties then a party nearer (in routeing terms) to the calling party takes higher priority than one further from the calling party.

2.	ultimate destination party (i.e. neither the calling party nor a forwarding party).

32.	calling party.;

43.	network operator or operators.

NOTE:	Charging principles are not subject of this ETS, but they are a underlying precondition for the elaboration of this ETS. The charging principles agreed by GSM MoU have been applied and reprinted as appropriate.

NOTE:	Reduced costs due to the optimised routing of a given call may reveal that the subscriber is not at the location expected.

In the first phase of SOR, the possibility of dividing the charge payable for a given leg of a route is not covered.

This ETS does not address the following:

·	The portion of fixed network originated calls until they enter the PLMN are excluded from the call scenarios for the reason that any solutions which aim at the optimisation of call routing within the fixed network, would need an active support from the fixed network by appropriate signalling means. However, GSM operators may be in the position to introduce optimisation schemes on a mutual base with the fixed network operator, including the fixed network portion of the call. Such additional means are outside the scope of this ETS.

·	At a later stage the total amount of costs, including all legs of the call, shall be taken into account for the calculation of the optimal route of the call. This may imply that the HPLMN of the called party has to provide cost estimates for the roaming leg towards the serving network of the called party.

·	All call scenarios leading to multiple call forwardings are outside the scope of this ETS. Generic solutions for optimal routing described within this standard may be supplemented with specific amendments for multiple call forwardings at a later stage.

·	There is no need for optimisation of the routing of calls originally directed to a fixed network subscriber, because the physical address of a fixed network terminating line cannot differ from its logical address. Optimisation of calls to Universal Personal Telecommunication (UPT) subscribers, residing in fixed networks are outside the scope of this standard .

·	SOR in non-GSM mobile networks is not subject of this ETS, but might be possible on mutual base among GSM and those non-GSM mobile network operators.

In some cases supplementary services may have to be modified to interact correctly with SOR, e.g. Call Barring (CB).

This Stage 1 description

·	compiles the basic service requirements for SOR, based on the principles elaborated by GSM MoU;

·	collects the necessary amendments for Supplementary Services (SS) in order to cater for SOR;

·	collects additional specifying text for call set-up procedures and MS features;

5	Functional requirements

5.1	General

The serving network shall provide means for an optimisation of the route of a given call from the calling party to the ultimate destination, taking account of call forwarding,called party and/or the forwarded-to party before the call is actually completed, independent of the location of the ultimate destination or any forwarding partyno matter whether the called party is roaming, the called party has activated Call Forwarding (CF), etc.

If Tthe serving network supports SOR, it shall must also support the consequential modifications to existing SS as described below.

If the serving network of the ultimate destination supports SOR, it shall be able to generate a call record for the ultimate destination indicating that the call has been optimally routed.

If any party involved in a call is a fixed network party, the equivalent of the VPLMN and the HPLMN is the fixed network serving that party.

5.2	Normal procedures

5.2.1	Normal operation with successful outcome

SOR shall beis provided for all circuit switched Telecommunication Services, except emergency calls, Dedicated PAD and Dedicated Packet Access (GPRS is for further study). The network feature is offered to all home and visiting subscribers served by the network. It ishall be possible as a network operator option to deny optimal routing of calls directed to a specified subscriber.

SOR shall beis invoked automatically by the network at every call set-up. The route to the ultimate destination shall be is optimised. This means, that if a call is forwarded the route is the optimal one from the original calling party to the forwarded-to party.

Optimal routeing applies in principle to an indefinite number of forwardings. However multiple forwarding, especially on no reply or on no response to paging, will lead to unacceptable delay in call setup.

5.2.2	Normal operation with successful outcome - first stage of implementation

In the first phasestage of implementation of optimal routing the following limitation shall applyoccur:

Call routing shall be optimised only if the total charge for the proposed optimal route is equal to or less than the charge for the call leg from the calling subscriber to the HPLMN of the called subscriber, i.e. according to the dialled number. If not, i.e. if the total charge for the proposed optimal route is higher, optimal routing shall not be applied at all, i.e. the call shall be set up via the HPLMN of the called subscriber. For this purpose the PLMN of the calling party when informed of the routing information (e.g. roaming number or forwarded-to number) shall compare the charging amount for the route to the HPLMN with that of the destination.The charge for the leg of the call from the A party’s serving network to the selected destination shall not exceed the charge for the leg from the A party’s serving network to the B party’s HPLMN. If the selected destination is not the ultimate destination, the call shall be routed optimally from the selected destination to the ultimate destination. The upper limit of the charge for the subsequent leg of the call is the charge for the leg from the selected destination to the HPLMN of the party to which the selected destination has forwarded the call.

See also figures 1, 2 and 3.

The figures below aim to clarify the routing principles and to clarify the criteria for deciding whether or not a call shall be optimised, as seen from the originating network.

[Editor’s note: need to cover the principles for choosing which route to take (ultimate destination’s VPLMN is favourite, then ultimate destination’s HPLMN, then step back towards B HPLMN), also requirement that we can drop back to a forwarding pary’s VPLMN only if that’s also the HPLMN.

The effect of having to ‘nail up’ a call leg because a later leg is too expensive also needs review.]

Fig.1:	B is roamed mobile subscriber: 	A call is set up from a calling subscriber A to a mobile subscriber B. The originating network interrogates the HPLMN of subscriber B and gets back the MSRN. The call shall be routed directly from A to the VPLMN of subscriber B if the charge for the direct route (OR1) is equal to or less than the charge for leg (a). If not, the call shall be routed via the HPLMN of subscriber B.

Fig.2a:	B is roamed mobile subscriber, with CF active to a fixed subscriber, the call to B has not been optimised: If a non-optimised call to a mobile subscriber B is conditionally to be forwarded to a number C, the connection on the HPLMN leg is released and the call shall be routed directly from A to C if the charge for the new optimised route (OR2) is equal to or less than the charge for leg (a).

Fig.2b:	B is roamed subscriber, with CF active to a fixed subscriber, the call to B has been optimised: A  call directly routed (OR1) to a mobile subscriber B is to be conditionally forwarded to a forwarded-to number C. The leg (OR1) of the call is released and the call shall be routed directly from A to C if the charge for the new route (OR2) is equal to or less than the charge for leg (a). If not, the call shall be routed via the HPLMN of subscriber B.

Fig.3:	B is roamed subscriber with CF active to a roamed mobile subscriber: A call to a mobile subscriber C. The originating network interrogates the HPLMN of subscriber C. The originating network analyses the routing towards subscriber C on the same basis as if the original call was directed towards subscriber C, i.e. if the charge for the direct route to VPLMN C is less than or equal to the charge for the leg to HPLMN C then the direct route to VPLMN C is preferred; otherwise the route via HPLMN C is preferred. If the charge payable by subscriber A for the preferred route to subscriber C is less than or equal to the charge for the leg to HPLMN B, then the call shall use the preferred route to subscriber C; otherwise the call shall use the route via HPLMN B. If the connection set up to route the call to subscriber B is not required to route the call to subscriber C, it shall be released.

Where  SOR is invoked and the called party has subscribed to Advice of Charge (AoC), AoC parameters for the called party shall be set to zero.

5.2.3	Normal operation with successful outcome - full functionality

For further study.

5.3	Exceptional procedures or unsuccessful outcome

5.3.1	Roaming into networks not supporting SOR

If a subscriber whose HPLMN supports SOR registers in a VPLMN which does not support SOR, the VPLMN shall inform the HPLMN that the VPLMN does not support SOR.

If the VPLMN of the called party does not support SOR and the called party has subscribed to AoC (C), the serving network of the calling party shall not invoke SOR shall not be invoked.

5.3.2	Roaming into networks not supporting SOR - minimum requirementsPartial Support of SOR

As a minimum requirement for the VPLMN of the called party, the VPLMN shall be able to generate call records for the terminating leg indicating OR, otherwise OR cannot take place.A network which does not support optimal routeing for conditionally forwarded calls shall not support optimal routeing for a call which may terminate in that network.

5.3.3	Non-support of SOR in the HPLMN

If the HPLMN of the called party does not support SOR, SOR cannot be invoked and the route of the call as far as the called party’s VPLMN cannot be optimised (the call would normally be routed via the HPLMN of the called party). If the call is to be forwarded from the called party’s VPLMN, optimisation from there towards the forwarded-to party may be possible.

6	Adaptations for Support of Optimal Routing

6.1	Operator Determined Barring (GSM 02.41)Requirements for call set-up procedures (GSM 02.01, ...)

No additional requirementsThe principles for the interaction between operator determined barring and SOR are the same as those for the interaction between Supplementary Service Call Barring and SOR.

6.2	Mobile Station Features (GSM 02.07, ...)

No additional requirements.

6.23	Supplementary Services

Supplementary services shall be invoked based on the outcome of an optimal routing attempt unless it is stated differently belowunderneath.

6.23.1	Advice of Charge (GSM 02.86)Operator Determined Barring (GSM 02.41)

A subscriber who has subscribed to Advice of Charge (Charging) may forward a call if the charge payable by the forwarding subscriber for the call is zero as a result of optimal routeing. This requires the check for the interaction between AoC(C) and Call Forwarding to be carried out when the call forwarding is to be invoked, rather than when registration or activation of call forwarding is requested or when AoC(C) is provisioned.

6.3.2	Call Barring (GSM 02.88)Closed User Group (GSM 02.85)

6.3.3	Call Barring (GSM 02.88)

The basis of the interactions between SOR and Supplementary Service Call Barring is that barring of outgoing calls and barring of incoming calls when roaming outside the HPLMN country (BIC-Roam) are used for cost control.It is assumed that Barring of outgoing and incoming international calls is used by the subscriber for cost control reasons.

If Call Barring does not prevent a call which is not optimally routed than it shall not prevent the use of optimal routeing for that call.A call should be permitted also in case the invocation of CB prior to SOR had permitted the call.

Note:	Ifn case a subscriber has subscribed to Barring of Outgoing International Calls (BOIC) or Barring of Outgoing International Calls except to the Home PLMN Country (BOIC-exHC), calls which would be prevented without the invocation of SOR might be permitted. But calls to a subscriber of the HPLMN of the calling party who has roamed abroad might be prevented.

It is in general not possible to predict at the time when registration or activation of call forwarding or activation of call barring is requested whether a call forwarding would be prevented by a call barring programme. The check for the interactions between Call Forwarding and Call Barring therefore has to be carried out when the call forwarding is to be invoked, rather than when registration or activation of call forwarding or activation of call barring is requested. The only exception to this principle is the interaction between BIC-Roam and Call Forwarding on No Reply can be checked at registration/activation.

6.3.4	Call Transfer (GSM 02.91)

The two transferred calls shall follow the general rules for the optimal routing as defined for the basic call set-up.

Note:	Optimal routing can apply after the connection of the remote parties. The location of the connection can be re-routed to another MSC. But this needs further investigation.

6.9	CAMEL (Customised Application for Mobile Network Enhanced Logic, 02.78, ...)

If CAMEL support is invoked at call set-up, SOR shall be applied to the outcome of the invocation of CAMEL.

It might be worthwhile to address this matter as well, especially because of the fact that the support of UPT Phase 2 is subject of the GSM Phase 2+ work programme. Undoubtedly in this respect a close co-operation among NA7 (UPT), NA6 (IN) and probably some of the SPS groups is necessary.