3GPP TR 21.916 V0.4.0 (2020-03)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Release 16 Description;

Summary of Rel-16 Work Items

(Release 16)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

Description, Summary, Release 16

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	4

1	Scope	5

2	References	5

3	Definitions of terms, symbols and abbreviations	5

3.1	Terms	5

3.2	Symbols	5

3.3	 Abbreviations	5

4	Rel-16 Executive Summary	6

5	Enhancement of Ultra-Reliable (UR) Low Latency Communications (URLLC)	6

6	5GS Enhanced support of Vertical and LAN Services	6

7	Cellular IoT support and evolution	6

8	Advanced V2X support	6

9	5G Location and Positionning Services	8

10	UE radio capability signalling optimization	9

11	Satellite Access in 5G	9

12	Enablers for Network Automation Architecture for 5G	9

13	Wireless and Wireline Convergence Enhancement	9

14	Mission Critical, Public Warning, Railways and Maritime	9

15	Streaming and TV	9

16	User Identities, Authentication, multi-device	13

17	Slicing	14

18	Other cross-TSG Release 16 Features	18

19	Radio Features	24

19.1	NR-related Release 16 Features	24

19.2	Release 16 Features impacting both LTE and NR	29

19.3	LTE-related Release 16 Features	33

20	All other Release 16 Features	34

Annex A1:	Template for contributing	49

Annex B:	Process to get further information	51

B.1	General	51

B.2	Direct method	52

B.3	Step by step method	52

Annex C:	Change history	55

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The present document provides a summary of each and every 3GPP Release 15 Feature, and more generally of all Work Items for which a summary has been agreed to be provided.

These summaries are based on the inputs issued by the Work Item Rapporteurs, slightly rewritten by the TR Rapporteur to ensure overall consistency. The original Work Item Rapporteur inputs can be retrieved as temporary document (tdoc), as stated in the first sentence of each clause.

Editor’s note: INPUTS ARE EXPECTED from rapporteurs. The list of expected contributors is provided at the beginning of each section. 
It also appears in “Annex B: List of contributors”, in the cell highlighted in yellow. Annex A provides the expected rapporteur in term of Working Group. E.g. if “SA2” is indicated, it means: “The SA2 WID rapporteur is expected to provide the summary for the full feature”.
The name of the rapporteur is provided in “Annex C: Full list of Rel-16 Work Items”, in the column “WI_rapporteur_e_mail”.

Editor’s note: to the contributors: 
The template to be used for the contribution is provided in “Annex A: Template for contributing”. The input type in 3GU is “WI Summary”.
The target audience of this document are readers inside and outside the 3GPP community. Contributions should be written as to be understandable by non-specialist. 
The summary should reflect the actual outcome of the Feature - and not the initial intentions given in the WID.
An exhaustive list of all the CRs and TS/TRs is not to be provided in the summary (instead, the procedure on how to retrieve this information is given in section 4). Only the essential CRs and TS/TRs can be mentioned in the Reference. 
When the usual Stage 1/Stage 2/Stage 3 process is not followed -and in particular if a full stage is missing- explain why.

# 1	Scope

The present document provides a summary of each Release 15 Feature or, whenever needed, of each significant Work Item.

The information provided in the present document is limited to an overview of each Feature, explaining briefly its purpose and the main lines of the system's behaviour to execute the Feature.

More information is available by consulting the 3GPP Ultimate web site, as explained in "Annex C: Process to get further information".

The present document presents the "initial state" of the Features introduced in Release 15, i.e. as they are by the time of publication of the present document. Each Feature is subject to be later modified or enhanced, over several years, by the means of Change Requests (CRs). It is therefore recommended to retrieve all the CRs which relate to the given Feature, as explained in Annex C, to further outline a feature at a given time.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	TR 21.905: "Vocabulary for 3GPP Specifications".

NOTE:	Due to the specificity of the present document, consisting in a collection of independent summaries, the references are given at the end of each clause rather than in this clause.

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

## 3.2	Symbols

Void.

## 3.3		Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1]. Abbreviations specific to a given clause are provided in the clause they appear.

KPI	Key Performance Indicator

Rel	3GPP Release

# 4	Rel-16 Executive Summary

Editor's note: To be provided later.

# 5	Enhancement of Ultra-Reliable (UR) Low Latency Communications (URLLC)

# 6	5GS Enhanced support of Vertical and LAN Services

# 7	Cellular IoT support and evolution

# 8	Advanced V2X support

| 820024 | Improvement of V2X service Handling | V2XIMP | S1 | SP-181013 | SungDuck Chun; LG Electronics |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by LG Electronics France in SP-200126.

This WI introduced requirements related to vehicle quality of service support, which enables a V2X application to be timely notified of expected or estimated change of quality of service (QoS). For example, when the communication packet error is expected to increase or decrease, the V2X application such as platooning application can increase or decrease inter-vehicle distance.

Followings are key requirements out of this WI:

- A standardized interface to external V2X application servers are provided.

- Using the standardized interface, the V2X application servers can request a specific QoS requirements and, optionally, related alternative QoS requirements.

- Using the standardized interface, the V2X application servers can request to be notified when the requested QoS requirements and/or the alternative QoS requirements can or cannot be met.

- The information that delivered over the interface can be past statistics or future prediction.

With this information, the V2X applications can adapt its behaviour according to the QoS that can be provided by 3GPP System.

Stage-2/3 works related to this WI were progressed within eV2XARC WI.

Reference

[1]	TS 22.186: "Enhancement of 3GPP support for V2X scenarios; Stage 1".

| 840078 | Architecture enhancements for 3GPP support of advanced V2X services | eV2XARC | S2 | SP-181121 | LaeYoung Kim, LG Electronics |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by LG Electronics in SP-200058.

Architecture enhancements to the 5G System are specified in TS 23.287 [1] in order to facilitate vehicular communications for Vehicle-to-Everything (V2X) services, over the following reference points, based on service requirements defined in TS 22.185 [2] and TS 22.186 [3]:

-	PC5 reference point: NR PC5 RAT, LTE PC5 RAT.

-	Uu reference point: NR, E-UTRA.

Interworking between EPS V2X and 5GS V2X is also specified.

The following architectural reference models are specified:

-	5G System architectures for V2X communication over PC5 and Uu reference points

-	5G System architecture for AF-based service parameter provisioning for V2X communications

-	Architecture reference model for interworking with EPS V2X

The various parameters for V2X communications over PC5 and Uu reference points are specified and these parameters may be made available to the UE in following ways:

-	pre-configured in the ME; or

-	configured in the UICC; or

-	preconfigured in the ME and configured in the UICC; or

-	provided/updated by the V2X Application Server via PCF and/or V1 reference point; or

-	provided/updated by the PCF to the UE.

In addition to PCF initiated Policy Provisioning procedure, the UE may perform UE triggered Policy Provisioning procedure to the PCF when the UE determines the V2X Policy/Parameter is invalid (e.g. Policy/Parameter is outdated, missing or invalid).

Regarding V2X communication over PC5 reference point, two types of PC5 reference points exist: the LTE based PC5 reference point as defined in TS 23.285 [4], and the NR based PC5 reference point as defined in TS 23.287 [1]. A UE may use either type of PC5 or both for V2X communication depending on the services the UE supports. The V2X communication over PC5 reference point supports roaming and inter-PLMN operations. V2X communication over PC5 reference point is supported when UE is "served by NR or E-UTRA" or when the UE is "not served by NR or E-UTRA".

V2X communication over NR based PC5 reference point supports broadcast mode, groupcast mode and unicast mode. For unicast mode, Layer-2 link establishment, Link identifier update, Layer-2 link release, Layer-2 link modification and Layer-2 link maintenance procedures are specified. Per-Flow PC5 QoS Model is introduced for V2X communication over NR based PC5 reference point.

Architecture enhancements for EPS to support V2X communication over NR PC5 reference point are specified in TS 23.285 [4].

For V2X communication over Uu reference point, only unicast is supported. Latency reduction for V2X message transfer via unicast may be achieved by using various mechanisms, including via e.g., edge computing defined in TS 23.501 [5].

Notification on QoS Sustainability Analytics to the V2X Application Server is specified so that the V2X Application Server may request notifications on QoS Sustainability Analytics for an indicated geographic area and time interval in order to adjust the application behaviour in advance with potential QoS change.

To support V2X applications that can operate with different configurations (e.g. different bitrates or delay requirements), the V2X Application Server, acting as the Application Function, can provide, in addition to the requested level of service requirements, Alternative Service Requirements to the 5G System. This enables the 5G System to act on the Alternative Service Requirements and apply them for the extended NG-RAN notification (i.e. Alternative QoS Profiles are provided from SMF to NG-RAN), as described in TS 23.501 [5] and TS 23.503 [6].

In order to facilitate deployment of dedicated network slice for use of, for example, automotive industry and to facilitate roaming support, a new standardized Slice/Service Type (SST) value dedicated for V2X services, i.e. 4 is defined in TS 23.501 [5].

Security aspects of 3GPP support for advanced V2X services are specified in TS 33.536 [7].

Stage 3 normative works made by CT working groups are specified in TS 24.587 [8] and TS 24.588 [9] that are new specifications for V2X as well as many specifications listed in the WID on CT aspects of architecture enhancements for 3GPP support of advanced V2X services [10].

References

[1] 3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[2] 3GPP TS 22.185: "Service requirements for V2X services; Stage 1".

[3] 3GPP TS 22.186: "Enhancement of 3GPP support for V2X scenarios; Stage 1".

[4] 3GPP TS 23.285: "Architecture enhancements for V2X services".

[5] 3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[6] 3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[7] 3GPP TS 33.536: "Security aspects of 3GPP support for advanced Vehicle-to-Everything (V2X) services".

[8] 3GPP TS 24.587: "Vehicle-to-Everything (V2X) services in 5G System (5GS); Stage 3".

[9] 3GPP TS 24.588: "Vehicle-to-Everything (V2X) services in 5G System (5GS); User Equipment (UE) policies; Stage 3".

[10] 3GPP CP-192078: "WID: CT aspects of architecture enhancements for 3GPP support of advanced V2X services".

# 9	5G Location and Positionning Services

| 830102 | Enhancement to the 5GC LoCation Services | 5G_eLCS | S2 | SP-181119 | Ming Ai, CATT |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by CATT in SP-191223.

The WI Enhancement to the 5GC LoCation Services (5G_eLCS) [1]  defines the stage 2 of the service-based architecture used for location services in the 5G system, and corresponding Network Functions (NFs), NF services and procedures, to meet the service requirements defined in TS 22.261 [2] and TS 22.071 [3].

Location Services specified in the document [4] include regulatory location services and commercial location services.

The architecture and signalling procedures in NG-RAN are defined in TS 38.305 [5].

Following aspects has been specified for 5G Location Services:

-	Service based 5G location architecture, including roaming and non-roaming, Function description of per Network Functions, and etc.

-	General Concepts, e.g. Type of Location Requests, LCS Quality of services;

-	High Level Features, e.g. LMF selection, UE LCS privacy handling;

-	Location Service Procedures, which includes

-  5GC-MT-LR Procedure

-  5GC-MO-LR Procedure

-  Deferred 5GC-MT-LR Procedure for Periodic, Triggered and UE Available Location Events

-  LMF Change Procedure

-  Unified Location Service Exposure Procedure

-  Low Power Periodic and Triggered 5GC-MT-LR Procedure

-  Bulk Operation of LCS Service Request Targeting to Multiple UEs

-  Procedures to Support Non-3GPP Access

-  Procedures dedicated to Support Regulatory services

-  UE Assisted and UE Based Positioning Procedure

-  Network Assisted Positioning Procedure

-  Obtaining Non-UE Associated Network Assistance Data

-  UE Location Privacy Setting Procedure

-  Procedures with interaction between 5GC and EPC

- Support of Concurrent Location Request;

-	Network Function Services, e.g. LMF services, GMLC services.

References

[1]	SP-181119: " New WID: Enhancement to the 5GC LoCation Services"

[2] 	TS 22.261: "5G system; Services, operations and procedures of charging using Service Based Interface (SBI)"

[3]	TS 22.071: "Service requirements for next generation new services and markets; Stage 1"

[4] 	TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2"

[5]      TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

# 10	UE radio capability signalling optimization

# 11	Satellite Access in 5G

# 12	Enablers for Network Automation Architecture for 5G

# 13	Wireless and Wireline Convergence Enhancement

# 14	Mission Critical, Public Warning, Railways and Maritime

# 15	Streaming and TV

| 810003 | Coverage and Handoff Enhancements for Multimedia | CHEM | S4 | SP-180664 | Nikolai Leung |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Qualcomm Incorporated in SP-190865.

The Coverage and Handoff Enhancements for Multimedia (CHEM) feature enables the network to delay or reduce handoffs of a Multimedia Telephony Service for IMS ( MTSI) terminal by providing the eNB/gNB additional information about the robustness to packet losses of the negotiated media configurations.  The eNB/gNB can set handoff thresholds that allow the MTSI terminal to remain on the current sector, cell, access technology, or domain (packet-switched vs. circuit-switched) and avoid a handoff even when the MTSI terminal experiences higher packet loss on the traffic channel as the media configuration robustness will help mitigate the effects of the additional packet losses.

The feature is structured with a baseline core functionality and some optional enhancements as illustrated in Figure 1.  The optional enhancements can be independently used with the core CHEM feature and each other.  3GPP TR 26.959 [5] has also been updated to explain the operation and interaction of these options and the core feature.

Figure 1: Core function and optional enhancements of the CHEM feature.  References are to clauses in 3GPP TS 26.114 [6].

The core CHEM functionality specified in clauses W.1 and W.2 of TS 26.114 [6] introduces the ‘PLR_adapt’ SDP attribute that is used to negotiate the CHEM feature between the MTSI clients.  MNOs choosing not to enable the feature can remove the attribute in the SDP negotiation.  The attribute is also used by the PCRF/PCF to determine whether it can expect the MTSI clients to adapt to the most robust configurations that have been negotiated without relying in application layer redundancy.  The PCRF/PCF uses this information to determine the appropriate PLR thresholds, i.e., choosing the PLR of the most robust mode when ‘PLR_adapt’ is negotiated, and communicate the PLR thresholds to the eNB/gNB.

An optional enhancement specified in clause W.4 of TS 26.114 [6] enables the MTSI clients to indicate the maximum tolerable end-to-end PLRs for particular codec configurations and then negotiate how this end-to-end PLR can be budgeted across their respective uplinks and downlinks.  Indicating maximum tolerable end-to-end PLR thresholds can be useful to account for differing client implementations of features, e.g., such as the de-jitter buffer and Packet-Loss-Concealment (PLC) in the media receiver, which can affect the maximum tolerable PLR for achieving a certain Quality of Experience (QoE).  This enhancement uses the defined ‘MAX-e2e-PLR’ attribute and associated parameters to perform the necessary indication and negotiation.  The PCRF/PCF uses the ‘MAX-e2e-PLR’ attribute information in the SDP answer to determine and communicate the appropriate PLR thresholds to the eNB/gNB.

Another optional enhancement for speech media specified in clause W.3 of TS 26.114 [6] is to support use of application layer redundancy to improve media robustness that, when used with the CHEM feature, can delay or avoid handing off an MTSI terminal.  The optional ‘ALR’ parameter for the ‘PLR_adapt’ attribute has been specified to enable this option and allows the MTSI clients to use newly specified in-band RTP CMR codepoints to request application layer redundancy without having to use RTCP-APP messages.  The PCRF/PCF uses the presence of the ‘PLR_adapt: ALR’ attribute and parameter in the SDP answer to determine the appropriate PLR thresholds based on the maximum tolerable PLRs of the codec configurations (among those that both use, and do not use, application layer redundancy), then communicates this to the eNB/gNB.

Annex X of TS 26.114 [6] provides example PLR values that can be used for different speech codecs and their configurations.  The annex also provides SDP examples for using the specified SDP attributes and parameters.

References

[1] Tdoc S4-191046, Draft Revised WID on “Coverage and Handoff Enhancements for Multimedia” (CHEM)

[2] Tdoc S4-190778, CR 26.114-0446, “Addition of Coverage and Handoff Enhancements for Multimedia”

[3] Tdoc S4-191037, CR 26.114-0485, “Management Objects for CHEM feature”

[4] Tdoc S4-191038, CR 26.114-0486, “SDP Examples for CHEM feature”

[5] 3GPP TR 26.959 “Study on enhanced Voice over LTE (VoLTE) performance”

[6] 3GPP TS 26.114 “IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction”

| 820001 | EVS Floating-point Conformance for Non Bit-Exact | EVS_FCNBE | S4 | SP-180983 | Fabrice, Plante, Intel |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Intel in SP-191328.

This summary reports on the normative specification work accomplished during the course of the EVS_FCNBE work item [1]. The related agreed CR can be found in Tdoc S4-191326 (CR 26.444-0027) [2].

The 3GPP EVS codec is standardized in three code versions such that it can be implemented on processors with either fixed-point (TS 26.442, TS 26.452) or floating-point arithmetic (TS 26.443). Implementations can be conformance-tested using “bit-exact” methods that have been specified already when the EVS codec was first standardized. These methods are suitable for fixed-point implementations but their applicability is limited for floating-point implementations. The EVS_FCNBE work item provides methods which enable conformance testing also for “non bit-exact” floating-point implementations.

During Rel-15, the FS_EVS_FCNBE study item investigated tools, conformance process, and criteria for EVS floating-point non bit-exact conformance, and documented in TR 26.843 [3] several recommendations. Based on the agreed conclusions and recommendations of TR 26.843, the EVS_FCNBE work item conducted normative work on TS 26.444 to introduce the non bit-exact conformance process and criteria in the CR 26.444-0027 [2].

The conformance process uses three specific tests:

-	Decoder test comparing the implementation decoder with TS 26.443 decoder.

-	Encoder test comparing the implementation encoder with TS 26.443 encoder.

-	MOS-LQO verification comparing the coder implementation with TS 26.442 coder.

These three tests allow to compare the implementation with the reference floating point code in TS 26.443, and interoperability with the fixed-point code in TS 26.442. All three tests shall pass for the implementation to be declared conformant.

The decoder test relies on a set of signal-based metrics (RMS, SNR, and Spectral Distortion). The encoder test uses a loudness metric.  The MOS-LQO test uses a set of metrics using objective quality measurements.

To define a range of acceptable variation for the various metrics, a set of 10 reference implementations (based on mainstream compilers and platforms) has been used to define reference values.

Annex B of  CR 26.444-0027 [2] provides details on the tests as well as the conformance process and criteria.

The results obtained during the course of the normative work indicate that this conformance process correctly discriminates implementations with functional code changes and aggressive compiler optimization.

References

[1] Tdoc SP-180983, New WID on “EVS Floating-point Conformance for Non Bit-exact” (EVS_FCNBE)

[2] Tdoc S4-191326, CR 26.444-0027, “EVS Non Bit-Exact Float Conformance”

[3] 3GPP TR 26.843 “Study on non-bit-exact conformance criteria and tools for floating-point EVS codec”

| 810040 | Media Handling Extensions for 5G Conversational Services | 5G_MEDIA_MTSI_ext | S4 | SP-180663 | Oyman, Ozgur, Company: Intel |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Intel in SP-200036.

This summary reports on the normative specification progress accomplished during the course of the 5G_MEDIA_MTSI_ext work item [1]. The related agreed CRs are in CR 26.114-0436 [2], CR 26.114-0438 [3], CR 26.114-0461 [4], CR 26.114-0480 [5], CR 26.114-0496 [6] and CR 26.223-0013 [7].

During Rel-15, the FS_5G_MEDIA_MTSI study item investigated media handling aspects of conversational services in 5G, and documented in TR 26.919 several recommendations on Multimedia Telephony Service over IMS (MTSI) in TS 26.114 around media rate adaptation, NR access and profiles for 5G deployments. Based on the agreed conclusions and recommendations of TR 26.919, this work item conducted normative work on TS 26.114 to introduce the following functionality:

-	For media rate adaptation, the CR 26.114-0436 [2] in SP-180975 recommended additional speech and video adaptation capabilities based on access network bitrate recommendation (ANBR) (a.k.a. RAN-assisted codec adaptation) support. In particular, this CR makes ANBR-triggered speech adaptation a recommended feature in MTSI clients for Rel-16 and beyond. Moreover, it provides additional recommendations for MTSI receivers to generate CMR/RTCP-APP (for speech) and TMMBR (for video) and for MTSI senders to generate TMMBN (for video) messages in response to receiving ANBR information.

-    On profiles for 5G deployments, the CR 26.114-0438 [3] in SP-180975 defined the constrained MTSI client terminal to address codec requirements for IMS/VoLTE/ViLTE/VoNR conversational services in the low end 5G verticals such as wearables and IoT. In particular, this CR relaxed the Rel-15 speech and video codec requirements for these constrained terminals, e.g., resulting in optional support for super-wideband for speech and HEVC for video, exceptionally only for this special category of constrained terminals (support for SWB for speech and HEVC for video is otherwise mandatory for Rel-15 and beyond).

-	To realize further RAN-assisted codec adaptation enhancements, the CR 26.114-0461 [4] in S4-190489 and CR 26.114-0480 in S4-190603 [5] introduced ANBR capability signalling into MTSI. In particular, the CR standardized a new SDP attribute ‘anbr’ to enable end-to-end signaling and coordination of ANBR capabilities (both at the radio level and application level) across the UEs, Access Networks, and PCRF/PCF. According to the adopted solution, the MTSI client in terminal supporting "anbr" signals this attribute in the SDP only if: (i) MTSI client in terminal supports ANBR at the application layer, including the use of ANBR with dynamic bitrate adaptation. (ii) The UE of the MTSI client in terminal is capable of RAN-assisted codec adaptation at the radio layer, including the ability to query and receive ANBR information (both downlink and uplink ANBR) from its eNB/gNB. (iii) The P-CSCF has indicated to the UE of the MTSI client in terminal its ability to handle this SDP attribute through the respective SIP registration procedures specified in TS 24.229 via the corresponding feature capability indicator g.3gpp.anbr.

-	The low and ultra-low latency of 5G will enable new types of use cases with real-time communication and interaction needs where it will be desirable to not only have voice, text and video calling connectivity but also delivery of any kind of data stream and real-time interaction within the same IMS multimedia session. This is already enabled as part of the IMS-based Tele-Presence service in TS 26.223 toward the exchange of CLUE messages and has been introduced to MTSI in TS 26.114 as part of this work item to provide a generic and flexible data channel support for IMS-based multimedia telephony. In particular, CR 26.114-0496 in Tdoc S4-200266 [6] adds necessary UNI procedures to support MTSI data channel based on WebRTC data channel and web scripting techniques as a new MTSI media. This provides a generic and flexible data channel for MTSI in TS 26.114 that allows both UE-to-UE and UE-to-network generic data transfers, in addition to and in parallel with the existing MTSI media such as audio, video, and text. Accordingly, TS 26.223 was also updated via CR 26.223-0013 in Tdoc S4-200270 [7] to align the data channel support of a Tele-Presence (TP) UE with that specified in MTSI.

References

[1] Tdoc SP-180663, New WID on “Media Handling Extensions for 5G Conversational Services” (5G_MEDIA_MTSI_ext)

[2] Tdoc SP-180975, CR 26.114-0436, “Recommendations on Media Rate Adaptation”

[3] Tdoc SP-180975, CR 26.114-0438, “MTSI Client Profiles”

[4] Tdoc S4-190489, CR 26.114-0461, “Signaling of ANBR Capabilities”

[5] Tdoc S4-190603, CR 26.114-0480, “Updates on ANBR Capability Signaling”

[6] Tdoc S4-200266, CR 26.114-0496, “Addition of MTSI Data Channel Media”

[7] Tdoc S4-200269, CR 26.223-0013, “Alignment with MTSI on IMS Data Channel Support”

# 16	User Identities, Authentication, multi-device

| 800012 | User Identities and Authentication | UIA | S1 | SP-180328 | Bischinger, Kurt; Deutsche Telekom |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Deutsche Telekom AG in SP-200131.

The feature allows the use of a User Identifier [1], which is independent of existing identifiers relating to a 3GPP subscription or UE, for Network Slice-Specific Authentication and Authorization. [1][2][3]

The actual identity provisioning service with creation, managing and authentication of identities is not specified by 3GPP, but the interworking of the 3GPP system with some external entity to authenticate the User Identity and to authorize the user for access to a specific network slice, in addition to the 3GPP network slice selection and authorization.

Rel. 15 Network slice selection (i.e. the procedure to select an AMF that supports the required Network Slices and establishing PDU Session(s) to the required Data network via the Network Slice instance(s) with its Control Plane and User Plane Network Functions) is based on the NSSAI, which consists of one or more S-NSSAIs.

Now with the Rel. 16 feature, Subscription information allows the indication which S-NSSAIs are subject to Network Slice-Specific Authentication and Authorization (NSSAA). For those S-NSSAIs the AMF invokes an EAP-based Network Slice-Specific authentication procedure in which the AUSF exchanges AAA protocol messages with a potentially external AAA Server (AAA-S) via an optional AAA Proxy (AAA-P) to authenticate and authorize a UE for the network slice. Depending on the result of the procedure a UE is either authorized for a network slice, re-allocated to a different one or deregistered. A re-authentication and re-authorization procedure may be triggered by the AAA Server at any time. [2][3]

Security aspects are specified in detail in [4], UE and network capabilities to support and perform NSSAA in [5], message flows for interworking to the AAA-S in [6], flags to indicate that a slice is subject to NSSAA are defined in [7] as well as [8] and the impact on the AUSF is captured in [9].

References

[1] TS 22.101, Service aspects; Service principles

[2] TS 23.501, System architecture for the 5G System (5GS)

[3] TS 23.502, Procedures for the 5G System (5GS)

[4] TS 33.501, Security architecture and procedures for 5G System

[5] TS 24.501, Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3

[6] TS 29.561, 5G System; Interworking between 5G Network and external Data Networks; Stage 3

[7] TS 29.503, 5G System; Unified Data Management Services; Stage 3

[8] TS 29.505, 5G System; Usage of the Unified Data Repository services for Subscription Data; Stage 3

[9] TS 29.509, 5G System; Authentication Server Services; Stage 3

# 17	Slicing

| 830103 | Enhancement of Network Slicing | eNS |  | SP-181232 | ZTE, Tricci So |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by ZTE Wistron Telecom AB in SP-200125.

The objective of this work item is to address the following aspects:

-	Enhance the network slicing interworking support from EPC to 5GC, when UE moves from EPC to 5GC by selecting the proper target serving AMF as well as V-SMF.  Note that, the 5GC may not be able to serve all the PDU sessions that the UE intends to move from EPC.

-	Provide the network slice based authentication and authorization that uses User Identities and Credentials different from the 3GPP SUPI and that takes place after the primary authentication.

Release 16 Network Slicing addresses two major limitations of Release 15 in 5GC:

(1)	Enhancement of interworking between EPC and 5GC when UE moves from EPC to 5GC, the target serving AMF may not be able to serve all the PDU sessions that the UE intends to move to the 5GC.  More specifically, the following aspects needs to be addressed:

-	Selecting an AMF based on the slices associated to the active PDU connections that serve the UE in the EPC in Connected mode and during the Idle mode mobility

-	Selecting an appropriate serving V-SMF based on the slices associated to the active PDN connections that serve the UE in the EPC in Connected Mode

(2) Support for Network Slice Specific Authentication and Authorization (NSSAA)

-	Enable the support for separate authentication and authorization per Network Slice.  The trigger of NSSAA in the 5GC is based on UE subscription information from UDM and also operator’s policy.  However, the UE shall indicate its support for NSSAA to its serving 5GC.

-	The AMF performs the role of the EAP Authenticator and communicates with the AAA-S via the AUSF. The AUSF undertakes any AAA protocol interworking with the AAA protocol supported by the AAA-S.

Enhancement of Interworking Between EPC and 5GC

During the mobility from EPS to 5GS, in case of CM-IDLE state, the PGW-C+SMF sends PDU Session IDs and related S-NSSAIs to AMF in Registration procedure. The AMF derives S-NSSAI values for the Serving PLMN and determines whether the current AMF is appropriate to serve the UE. If not, the AMF reallocation may need to be triggered. For each PDU Session the AMF determines whether the V-SMF need to be reselected based on the associated S-NSSAI value for the Serving PLMN. If the V-SMF need be reallocated, the AMF trigger the V-SMF reallocation.

In case of CM-CONNECTED state, during handover preparation phase the PGW-C+SMF sends PDU Session IDs and related S-NSSAIs to AMF. Based on the received S-NSSAIs values, the target AMF derives the S-NSSAI values for the Serving PLMN, the target AMF reselects a final target AMF if necessary and forwards the handover request to the final target AMF. When the Handover procedure completes successfully, the UE proceeds with the Registration procedure. For each PDU Session based on the associated derived S-NSSAI values, if the V-SMF need be reallocated, the final target AMF triggers the V-SMF reallocation. The final target AMF sends the S-NSSAI value for the Serving PLMN to V-SMF to update the SM context. The V-SMF updates NG RAN with the S-NSSAI value for the Serving PLMN via N2 SM message.

Network Slice-Specific Authentication and Authorization (NSSAA)

In Release-16, based on UE’s 5GMM Core Network Capability and subscription information, the serving AMF will trigger Network Slice-Specific Authentication and Authorization for the S-NSSAIs of the HPLMN. If a UE does not support this feature but requests these S-NSSAIs that are subject to Network Slice-Specific Authentication and Authorization, these S-NSSAIs will be rejected by the PLMN.

If a UE supports this feature and requests these S-NSSAIs, which are subject to Network Slice-Specific Authentication and Authorization, the UE shall leverage the corresponding credentials for these S-NSSAIs for the Network Slice-Specific Authentication and Authorization. As for how to these credentials in the UE are not specified.

To perform the Network Slice-Specific Authentication and Authorization for an S-NSSAI, the AMF invokes an EAP- based Network Slice-Specific authorization procedure for the S-NSSAI.

This procedure can be invoked for a supporting UE by an AMF at any time, e.g. when:

a.	The UE registers with the AMF and one of the S-NSSAIs of the HPLMN which maps to an S-NSSAI in the Requested NSSAI is requiring Network Slice-Specific Authentication and Authorization; or

b.	The Network Slice-Specific AAA Server triggers a UE re-authentication and re-authorization for an S-NSSAI; or

c.	The AMF, based on operator policy or a subscription change, decides to initiate the Network Slice-Specific Authentication and Authorization procedure for a certain S-NSSAI which was previously authorized.

Based on the outcome of the Network Slice-Specific Authentication and Authorization, the Allowed NSSAI for each Access Type will be updated accordingly.   It is network policies to decide for which Access Type to be used if both Access Types are subject for the Network Slice-Specific Authentication and Authorization. However, if the Network Slice-Specific Authentication and Authorization fails for all S-NSSAIs in the Allowed NSSAI, the AMF shall execute the Network-initiated Deregistration procedure with the appropriate rejection cause value for each Rejected S-NSSAI.

After a successful or unsuccessful UE Network Slice-Specific Authentication and Authorization, the UE context in the AMF shall retain the authentication and authorization status for the UE for the related specific S-NSSAI of the HPLMN while the UE remains RM-REGISTERED in the PLMN, so that the AMF is not required to execute a Network Slice-Specific Authentication and Authorization for a UE at every Periodic Registration Update or Mobility Registration procedure with the PLMN.

A Network Slice-Specific AAA server may revoke the authorization or challenge the authentication and authorization of a UE at any time. When authorization is revoked for an S-NSSAI that is in the current Allowed NSSAI for an Access Type, the AMF shall provide a new Allowed NSSAI to the UE and trigger the release of all PDU sessions associated with the S-NSSAI, for this Access Type.

The AMF provides the GPSI of the UE related to the S-NSSAI to the AAA Server to allow the AAA server to initiate the Network Slice-Specific Authentication and Authorization, or the Authorization revocation procedure, where the UE current AMF needs to be identified by the system, so the UE authorization status can be challenged or revoked.

The Network Slice-Specific Authentication and Authorization requires that the UE Primary Authentication and Authorization of the SUPI has successfully completed. If the SUPI authorization is revoked, then also the Network Slice-Specific authorization is revoked.

References

[1]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

| 810051 | Business Role Models for Network Slicing | BRMNS | S1 | SP-180773 | Covell, Betsy, Nokia |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Nokia in SP-200217.

This WI adds requirements to enable a 3GPP system to adequately support the variety of business role models for network slicing that are possible in a 5G system.  The requirements address service exposure giving additional control to 3rd parties, impact of SLAs on network slice management, and non-public network access to MNO spectrum. Also addressed are security relationships between

•	a UE and a private slice,

•	a private slice and a network, and

•	a private slice and other slices of the same network.

The requirements are captured in 3GPP TS 22.261 [1].

The requirements from the BRMNS WI provide the means to support new business role models introduced for supporting vertical markets.  These role models extend the possible interactions between Mobile Network Operators (MNOs) and 3rd parties who use the network and/or own/manage part of the network infrastructure.  In particular, the introduction of network slicing allows new opportunities for 3rd parties to exercise additional control over the portion of the network, i.e., the network slice, supporting their network needs.  Four key business relationship models were considered.

­	Model 3a: MNO provides the virtual/physical infrastructure and V/NFs; a 3rd party uses the functionality provided by the MNO,

-	Model 3b: MNO provides the virtual/physical infrastructure and V/NFs; a 3rd party manages some V/NFs via APIs exposed by the MNO,

-	Model 3c: MNO provides virtual/physical infrastructure; a 3rd party provides some of the V/NFs,

-	Model 3d: a 3rd party provides and manages some of the virtual/physical infrastructure and V/NFs.

Models 3a and 3b are essentially supported by the Rel-15 network slicing requirements, with some enhancements in the Rel-16 APIs and management functions to extend 3rd party access and control of capabilities provided by the MNO, and to do so in a secure manner. Within these two models, the 3rd party has increasing control over the network capabilities that support its service. However, this control is limited to what is allowed by the MNO through the provided APIs. The Rel-16 enhancements allow the 3rd party to monitor the activities of their UEs when using the network slice and to monitor resource usage within the network slice. These capabilities are added to support SLAs between the MNO and 3rd  party.

As an example of the additional monitoring functionality, a network slice supporting a local smart grid needs additional APIs to monitor activities of many sensors throughout the grid and receive network status information that can be used to detect and resolve any communications problems that arise within the grid.  Figure 2-1 from TR 22.830 [2] illustrates the smart grid role models.

Figure 1: role model scenario between MNO and power grid company

Models 3c and 3d resulted in the addition of new functionality to support three new network management models.

1)	MNO manages all virtual/physical infrastructure and all V/NFs including 3rd party’s ones,

2)	3rd party manages its own virtual/physical infrastructure and/or its own V/NFs; MNO manages the others.

3)	3rd party manages virtual/physical infrastructure and/or V/NFs including its own virtual/physical infrastructure and/or V/NFs and some MNO’s virtual/physical infrastructure and/or V/NFs; MNO manages the others.

From the 3rd party perspective, the management role models 2 and 3 support the 3rd party management function and provide extended management for the MNO to coordinate with the 3rd party management. The 3rd party may use suitable APIs provided by the MNO to directly manage the V/NFs as well as the infrastructure resources so that it can properly handle when their business requirements are changed.

For models 3c and 3d, additional consideration is needed on the mechanisms to provide the isolation and interfaces that give the 3rd party the appropriate level of control while securing the PLMN. Specifically, where the 3rd party provides V/NFs or provides and manages some of the virtual/physical infrastructure and V/NFs, SLAs may be used to address some trust issues, such as what each party will provide and manage. However, the principles of trust and verification also come into play. The 3rd party must be able to verify, though the management interface, that all terms of the SLA are being met by the MNO. Similarly, the MNO must be able to verify that the 3rd party is managing resources appropriately so that there is no adverse impact to the rest of the network. Appropriate management interfaces are needed to support both the network operator and 3rd party.

Models 3c and 3d allow for additional interactions between non-public networks, private slices, and PLMNs as shown in the following figures from [2].

Figure 2: Example of enterprise communications using a combination of 5G PLMNs, a private slice, and a non-public network

Figure 3: Example of enterprise communications using a geographically constrained private slice

Figure 4: Example of enterprise communications using a private slice throughout the PLMN

Support for enterprise communications may be provided by a variety of network configurations, including multiple PLMNs, private slices, and non-public networks.  New requirements allow for managing the enterprise communications in a coordinated manner across these various implementation options. New requirements also provide for isolation of private network resources which may be constrained for access within a geographic area such as a factory site or restricted for use only by UEs belonging to the enterprise. As a counterpart, additional requirements provide for restricting some enterprise UEs from accessing a PLMN, this may be useful for example to keep factory equipment connected to the private slice or non-public network when a PLMN covers the same area.

References

[1] TS 22.261 Service requirements for the 5G system

[2] TR 22.830 Feasibility Study on Business Role Models for Network Slicing

# 18	Other cross-TSG Release 16 Features

| 810041 | Single radio voice continuity from 5GS to 3G | 5G_SRVCC |  | SP-180737 | Chi Ren, China Unicom |
| --- | --- | --- | --- | --- | --- |

Summary based on the inputs provided by China Unicom in SP-200129 for global aspects and in RP-200152 for the radio aspects [merging needed!].

The work item of 5G_SRVCC introduced a mechanism to support single radio voice call continuity (SRVCC) from 5GS to UTRAN for the following scenarios:

-	Operators with both 5G Voice over IMS and LTE enabled, but no VoLTE.

-	Operators with no LTE (nor VoLTE).

-	Operators with both 5G Voice over IMS and VoLTE enabled, but the voice service continuity may not be guaranteed if the VoLTE coverage provided by the operators is not (ideal) complete, i.e. there are some “holes” of VoLTE coverage.

This work item implemented the conclusion of the study for single radio voice continuity from 5GS to 3G (FS_5G-SRVCC) which is specified in TR 23.756[4]. The main enhancements introduced by this work item are specified in TS 23.216[3].

The architecture of 5G-SRVCC is illustrated in Figure 2-1. A simplified MME function (MME_SRVCC) was introduced to facilitate session transfer (SRVCC) of the voice component to the CS domain. The support of N26 interface and Sv interface is required for the MME_SRVCC.

Figure 1: 5G-SRVCC architecture

The 5G-SRVCC mechanism defined by this work item is an indirect SRVCC from NG-RAN to UTRAN. When an UE moves out of the coverage of NG-RAN while a 5G voice session over IMS (either a normal voice session or an emergency session) is ongoing, a PS to CS handover procedure between 5GS and 3G CS is triggered by the NG-RAN. The 5G-SRVCC procedure including two main parts:

-	5GS to EPS handover using N26 procedure triggered by the NG-RAN.

-	PS-CS handover procedure initiated by the selected MME_SRVCC.

No E-UTRAN involved during the whole process. All the PDU sessions is released by the request of AMF at the end of the 5G-SRVCC procedure.

References

[1] SP-180897, Single radio voice continuity from 5GS to 3G.

[2] CP-193014, CT aspect of 5G_SRVCC.

[3] TS 23.216, Single Radio Voice Call Continuity (SRVCC); Stage 2.

[4] TR 23.756, Study for single radio voice continuity from 5G to 3G.

Radio part

In Rel-16, the WI “Single Radio Voice Call Continuity from 5G to 3G” was approved (latest WID in RP-200151). This work item aims to specify the support of SRVCC from 5GS to UTRAN in RAN. More specifically, the main objectives are as follows:

-	Inter-RAT measurement to support voice service continuity from 5GS to UTRAN [RAN2, RAN1].

-	Specify that the NG-RAN and UE in NR RRC connected mode support the UTRAN cell measurement procedure, e.g. measurement configuration of target UTRAN cells, and the measurement performing and reporting by UE.

-	Indirect Inter-RAT handover procedure to support voice service continuity from 5GS to UTRAN [RAN2, RAN3].

-	Specify the procedures of SRVCC (including emergency call) in RAN, which includes handover preparation between gNB and AMF, gNB and UE, and Handover execution between UE to RNS etc.

-	Signalling of source RAT to target RAT at incoming SRVCC [RAN3].

-	UE capability reporting for supporting SRVCC [RAN2, RAN3].

-	Specify the transfer of UE capability information between UE, NG-RAN and AMF.

-	RRM requirements to support voice service continuity from 5GS to UTRAN [RAN4].

-	Requirements of NR handover to UMTS.

-	Requirements of NR- UMTS inter-RAT measurements.

This is the summary of outcomes of WI “Single Radio Voice Call Continuity from 5G to 3G.” Documents related to this WI can be found in the reference section.

This section provides a summary of the key functionalities whose motivation was introduced above.

-	Support of the UTRAN cell measurement procedure for NR node and UE in NR RRC connected mode. A new measurement object for UTRA is defined to include only UTRA-FDD. -	The UE is only required to measurement and report the listed cells. Periodic and event triggered (event B1/B2) reporting for UTRA measurement are both supported. The same measurement gap configuration for UTRA is reused in measurement in NR as in LTE.

-	Support of inter-RAT handover procedure of SRVCC. MobilityFromNRCommand message is reused for SRVCC from NR to UTRAN. SRVCC to UTRAN is also supported for UEs configured with NR-DC and NE-DC, network can trigger SRVCC to UTRAN procedure directly irrespective of the whether the bearer for the voice call is anchored in SN or MN. When and how to perform the return procedure from UTRAN to NR after the UE completes the voice service depends on UE implementation.

-	Support of Signalling of source RAT to target RAT at incoming SRVCC. Add 5G-SRVCC possible into Initial Context Setup Request, HO Request, Path Switch Request Ack. Clarify the source to target transparent container is source RNC to target RNC Transparent Container in 5G-SRVCC. Clarify the target to source transparent container is Target RNC to Source RNC Transparent Container in 5G-SRVCC.

-	Support of UE capability reporting for supporting SRVCC, including SRVCC handover capability with FR1/FR2 and TDD/FDD split, and UTRA-FDD band list is also reported.

-	RRM Requirements have also been specified, including measurement gap applicability requirement for SRVCC, Measurement capability, CSSF, UMTS inter-RAT measurement requirements and handover requirements for SRVCC.

References

Documents related to this WI are listed as follows:

-	Last approved work item description: RP-200151

[1]	RP-200151	Revised WID: Single Radio Voice Call Continuity from 5G to 3G	China Unicom, Telefonica, Huawei, Hisilicon

-	Last status report: RP-200116

[1]	RP-200116	Status report for WI Single Radio Voice Call Continuity from 5G to 3G; rapporteur: China Unicom	China Unicom

-	CRs:

[1]	R2-2000325	Introduction of SRVCC from 5G to 3G	Ericsson, ZTE

[2]	R2-2000335	Introduction of SRVCC from 5G to 3G	Ericsson

[3]	R2-2000542	Introduction of SRVCC from 5G to 3G	Huawei, HiSilicon, China Unicom

[4]	R2-2000651	Introduction of SRVCC from 5G to 3G	China Unicom, Huawei, HiSilicon

[5]	R3-194169	Support SRVCC from 5G to 3G	Ericsson

[6]	R3-194171	Support SRVCC from 5G to 3G	Ericsson

[7]	R3-200071	Support SRVCC from 5G to 3G	Ericsson

[8]	R3-200073	Support SRVCC from 5G to 3G	Huawei, China Unicom, Ericsson, Nokia, Nokia Shanghai Bell, ZTE, Qualcomm Incorporated

| 820044 | Access Traffic Steering, Switch and Splitting support in the 5G system architecture | ATSSS |  | SP-181124 | So, Tricci, ZTE |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by ZTE Wistron Telecom AB in SP-200124.

The ATSSS feature enables a multi-access PDU Connectivity Service, which can exchange PDUs between the UE and a data network by simultaneously using one 3GPP access network and one non-3GPP access network and two independent N3/N9 tunnels between the PSA and RAN/AN. The multi-access PDU Connectivity Service is realized by establishing a Multi-Access PDU (MA PDU) Session, i.e. a PDU Session that may have user-plane resources on two access networks.

Access Traffic Steering, Switch and Splitting support in the 5G system architecture (ATSSS) are defined as:

-	Access Traffic Steering: The procedure that selects an access network for a new data flow and transfers the traffic of this data flow over the selected access network. Access traffic steering is applicable between one 3GPP access and one non-3GPP access.

-	Access Traffic Switching: The procedure that moves all traffic of an ongoing data flow from one access network to another access network in a way that maintains the continuity of the data flow. Access traffic switching is applicable between one 3GPP access and one non-3GPP access.

-	Access Traffic Splitting: The procedure that splits the traffic of a data flow across multiple access networks. When traffic splitting is applied to a data flow, some traffic of the data flow is transferred via one access and some other traffic of the same data flow is transferred via another access. Access traffic splitting is applicable between one 3GPP access and one non-3GPP access.

Key concepts of ATSSS supported in Release 16 include the following:

-	Multi-access PDU Session is a PDU Session that provides a PDU connectivity service, which can use one access network at a time, or simultaneously one 3GPP access network and one non-3GPP access network and two independent N3/N9 tunnels between the PSA and RAN/AN.

-	After the establishment of a MA PDU Session:

o	When there are user-plane resources on both access networks:

	The UE applies network-provided policy (i.e. ATSSS rules derived by UE’s serving SMF based on ATSSS policy from serving PCF) and considers local conditions (such as network interface availability, signal loss conditions, user preferences, etc.) for deciding how to distribute the uplink traffic across the two access networks.

	Similarly, the UPF anchor of the MA PDU Session applies network-provided policy (i.e. N4 rules derived by UE’s serving SMF based on ATSSS policy from serving PCF) and the feedback information received from the UE via the user-plane (such as access network Unavailability or Availability), the UPF then decides how to distribute the downlink traffic across the two N3/N9 tunnels and two access networks.

o	When there are user-plane resources on only one access network, the UE applies the ATSSS rules and considers local conditions for triggering the establishment or activation of the user plane resources over another access.

-	The type of a MA PDU Session may be one of the following types: i.e. IPv4, IPv6, IPv4v6, and Ethernet. The Unstructured type is not supported in Release 16.

-	The ATSSS feature can be supported over 3GPP and non-3GPP accesses, including untrusted and trusted non-3GPP access networks, wireline 5G access networks, etc., as long as a MA PDU Session can be established over the given type of access network

-	Two ATSSS steering functionalities are supported:

o	MPTCP functionality, for TCP traffic, with MPTCP proxy in UPF, by using the MPTCP protocol over the 3GPP and/or the non-3GPP user plane; and

o	ATSSS-LL functionality for all types of traffic, including TCP traffic, UDP traffic, Ethernet traffic, etc. ATSSS-LL functionality is mandatory for MA PDU Session of type Ethernet.

The following presents the example of the ATSSS traffic steering functionality within the UE.

Figure 1: Steering functionalities in an example UE model

-	The Performance Measurement Function (PMF) is supported by UPF and is specific for ATSSS-LL functionality, if enabled.  In Release 16, PMF supports two types of measurements between UE and UPF to assist access selection and they are:

o	UE and UPF make RTT measurements per access when the "Smallest Delay" steering mode is used; and

o	UE reports access availability/unavailability to UPF

The following presents the protocol stacks of the PMF for the user plane measurements over 3GPP and non-3GPP accesses respectively.

Figure 2: UE/UPF measurements related protocol stack for 3GPP access and for an MA PDU Session with type IP

In the case of an MA PDU Session with type Ethernet, the protocol stack over 3GPP access is that same as the one in the above figure, but the PMF protocol operates on top of Ethernet, instead of UDP/IP.

Figure 3: UE/UPF measurements related protocol stack for non-3GPP access and for an MA PDU Session with type IP

In the case of an MA PDU Session with type Ethernet, the protocol stack over non-3GPP access is that same as the one in the above figure, but the PMF protocol operates on top of Ethernet, instead of UDP/IP.

-	An ATSSS-capable UE may decide to request a MA PDU Session based on the provisioned URSP rules. In particular, the UE should request a MA PDU Session when the UE applies a URSP rule, which triggers the UE to establish a new PDU Session and the Access Type Preference component of the URSP rule indicates "Multi-Access".

-	The 5G QoS model for the Single-Access PDU Session is also applied to the MA PDU Session, i.e. the QoS Flow is the finest granularity of QoS differentiation in the MA PDU Session. One difference compared to the Single-Access PDU Session is that in a MA PDU Session there can be separate user-plane tunnels between the AN and the PSA, each one associated with a different access. The SMF shall provide the same QFI in 3GPP and non-3GPP accesses so that the same QoS is supported in both accesses. Non GBR QoS Flow can be distributed over 3GPP access and non 3GPP access, but GBR QoS Flow is transferred over single access.

-	ATSSS is currently not supported when moving to EPC from 5GC, except for the specific case with wireline access integrated to EPC/5GC with 5G-RG; ATSSS with one User Plane leg in E-UTRA/EPC and one User Plane leg in wireline/5GC is supported.

References

[1]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[2]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[3]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

# 19	Radio Features

## 19.1	NR-related Release 16 Features

| 820068 | 2-step RACH for NR | NR_2step_RACH | R1 | RP-190711 | ZTE |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by ZTE Corporation in RP-200087.

The Rel-16 Work Item 2-step RACH for NR achieves the following objectives:

-	A simplified random access procedure was developed. This reduces the number of interactions between the UE and network during the connection setup and connection resume, thereby enabling a lower control plane latency for IDLE and INACTIVE UEs. In case of connected mode, a small amount of data can be sent over the 2-step RACH channel thus enabling a lower latency for UL UP data for connected mode UEs.

-	Channel structure of transmitting PRACH and PUSCH in one step (i.e. without an intermediate message from the network) was developed

-	The above enhancements are applicable to both licensed spectrum and the shared spectrum (i.e. NR-U).

The general procedure of 4-step RACH and 2-step RACH are depicted in Figure 1. The first step of 2-step RACH comprises an UL MSGA transmission which includes the equivalent contents of msg1 and msg3 of 4-step RACH. The second step of 2-step RACH is a DL MSGB transmission which includes the equivalent content of msg2 and msg4 of 4-step RACH.

(a) 4-step RACH	               		                                 (b) 2-step RACH

Figure 19.1-1: General procedure of 4-step RACH and 2-step RACH

RA type selection

All the triggers for Rel-15 NR 4-step RACH are also applicable to 2-step RACH. However, contention free random access (CFRA) procedure with 2-step RACH is only supported for handover.

The UE selects the type of random access at initiation of the random access procedure based on network configuration:

-	when CFRA resources are not configured, an RSRP threshold is used by the UE to select between 2-step RA type and 4-step RA type;

-	when CFRA resources for 4-step RA type are configured, UE performs random access with 4-step RA type;

-	when CFRA resources for 2-step RA type are configured, UE performs random access with 2-step RA type.

MSGA structure: PRACH

The MSGA in 2-step RACH comprise a PRACH and a PUSCH. The PRACH resources for 2-step RACH in time/frequency domain can be either shared with 4-step RACH or can be configured to be separate. All the preamble formats and the PRACH configuration indexes defined in Rel-15 can be used. In case of shared time domain PRACH resources between 4-step RACH and 2-step RACH, different preambles are allocated to differentiate the RA types.  The mapping between SSB and PRACH occasion reuses that for 4-step RACH.

MSGA structure: PUSCH

2-step RACH uses a specified mapping rule to determine the PUSCH resource of MSGA that is associated with the selected PRACH resource. Each PRACH slot is mapped to a number of PUSCH occasions, once the UE selects a preamble in a PRACH occasion, the corresponding PUSCH occasion and DMRS resource can be determined by a predefined mapping order.

MSGB

After MSGA transmission, the UE monitors the downlink for a response from the network within a configured window. This response from the network is called the MSGB. The contents of MSGB depend on whether or not the gNB is able to successfully detect both the PRACH and the PUSCH parts.

-	If the PRACH is detected but the decoding of PUSCH fails, network will include a fallback indication in MSGB and the subsequent UE procedure will be similar to the 4 step RACH.

-	If both preamble and PUSCH are decoded, network will include a successRAR and this completes the contention resolution. HARQ feedback is enabled for the successful reception of the successRAR.

References

[1]	RP-192330, Revised work item proposal 2 step RACH for NR

[2]	RP-192414, Status Report for WI NR 2-step RACH

| 841000 | Dual Connectivity (EN-DC) with 3 bands DL and 3 bands UL | DC_R16_LTE_NR_3DL3UL | R4 | RP-191598 | ZTE |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by ZTE Corporation in RP-200112.

In Release 15, the operating bands are specified for these eligible operations with EN-DC configured. The EN-DC band combinations include at least one E-UTRA operating band and a NR operation band. In moving forward to Release 16, the configuration of EN-DC operation needs to be expanded for more simultaneous uplink and downlink configurations, including band combination of LTE, FR1 and FR2 bands.

New configurations still emerge from existing bands and whenever new band is specified, it will create a potential for several new EN-DC configurations consisting of 3 different bands DL with 3 different bands UL (1 LTE band UL and 2 NR different band UL(i.e. 1CC LTE band +  1CC NR FR1 band +  NR FR2 band), or 2 LTE different band UL and 1 NR FR2 band UL).

The EN-DC configurations including NR CA will be introduced in a release independent manner based on TS38.307, which will be updated depending on newly introduced EN-DC configurations including NR CA.

All new ENDC configurations including NR with 3 bands  will be defined under this WI, where :

-	For only 1 NR band included, only NR FR2 band is applied

-	For only 2 NR bands included, 1 NR FR1 band  and 1 NR FR2 band are included and operated as inter-band CA

The companies can request their interesting ENDC configurations before each 3GPP RAN4 meeting. The WID was started from RAN #84 meeting, and will completed at RAN #88 meeting.

Usually these configurations are the potential NR NSA deployment scenarios for the operators.  Each of the requested ENDC configurations includes at least applicable frequencies if necessary, applicable bandwidths and bandwidth sets if necessary, and Downlink ENDC configurations and Uplink ENDC configuration, respectively. More importantly, each ENDC configuration shall be requested with at least three other supporting companies. In addition,  these combinations will be introduced in a Rel-independent way starting from Rel-15.

All of the ENDC configurations requested by the companies are all captured in two tables in the WID, shown as follow.

1. EN-DC of 3 different bands DL(LTE 1 band + NR 2 bands) with 3 different bands UL

2. EN-DC of 3 different bands DL(LTE 2 bands + NR 1 band) with 3 different bands UL

For the ENDC configuration in the WID, the RF requirements shall be specified, including:

Analyse combinations that have self-desensitization due to following reasons:

-	TX Harmonic and/or intermodulation overlap of receive band

-	TX signal overlap of receiver harmonic frequency

-	TX frequency being in close proximity of one of the receive bands

-	Any other identified reasons such that insufficient cross band isolation, harmonic mixing

For the combination where self-desensitization exists, specify at least needed

-	∆TIB, c and ∆RIB, c

-	Reference sensitivity exceptions including MSD test cases

All of the study results are captured into TR37.716-33, and for the ENDC configurations will be included in TS38.101-3 specification in the case of these configurations are approved in RAN4 meeting.

References

[1]	RP-192537,  Revised WID for Dual Connectivity (EN-DC) with 3 bands DL and 3 bands , ZTE Corporation

[2]	R4-1913627, CR to reflect the completed ENDC combinations for 3 bands DL with 3 bands UL into Rel16 TS 38.101-3, ZTE Corporation

[3]	R4-2000501, CR to reflect the completed ENDC combinations for 3 bands DL with 3 bands UL into Rel16 TS 38.101-3, ZTE Corporation

[4]	R4-1913631, TR 37.716-33 v0.1.0 Dual Connectivity (EN-DC) with 3 bands DL  and 3 bands UL, ZTE Corporation

[5]	RP-200110, Revised WID for Dual Connectivity (EN-DC) with 3 bands DL and 3 bands , ZTE Corporation

| 800074 | Rel-16 NR inter-band CA/Dual Connectivity for 2 bands DL with x bands UL (x=1, 2) | NR_CADC_R16_2BDL_xBUL | R4 | RP-191565 | ZTE |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by ZTE Corporation in RP-200170.

This WI is to introduce the band combinations for inter-band carrier aggregation and dual connectivity for 2 bands DL with up to 2 bands UL into Rel-16 specifications and specify the corresponding configurations and RF requirements.

The study results for each band combination are captured in TR 38.716-02-00, and the configurations and RF requirements are added into the corresponding technical specifications.

This WI includes all target band combinations for both NR CA and DC including the following configurations:

•	New NR CA configurations including inter band CA for 2 different bands DL with up to 2 different bands UL

o	new configurations from exiting bands

o	new configurations from new band

o	The NR CA configurations will be introduced in a release independent manner based on TS38.307

•	New NR DC configurations including inter band CA for 2 different bands DL with up to 2 different bands UL

o	Accommodated only until the outcome of Rel-15 late drop handling becomes clear.

The preconditions to propose NR CA/DC configurations including Inter band CA for 2 different bands DL with up to 2 different bands UL in rel-16 are as follows.

o	Constituent NR band and NR Intra band CA shall be completed and specified in advance.

o	NR Band n1 and n2 requirements shall be completed and specified in advanced.

Specification for each band combination includes:

o	Applicable frequencies if necessary

o	Applicable bandwidth and bandwidth sets if necessary

o	Analysis on potential self-desensitization due to following reasons:

o	TX Harmonic and/or intermodulation overlap of receive band

o	TX signal overlap of receiver harmonic frequency

o	TX frequency being in close proximity of one of the receive bands

o	Any other identified reasons

o	At least the following for the combination where self-desensitization exists

o	∆TIB, c

o	∆RIB, c

o	Reference sensitivity exceptions including MSD test cases

o	Add conformance testing in RAN5 specifications (to follow at a later stage)

The target band combinations are captured in four tables in this WI respectively:

o	CA configurations for 2 different bands DL with 1 band UL

o	CA configurations for 2 different bands DL with 2 band UL

o	CA configurations for 2 different bands DL with up to 2 band UL within FR2

o	DC configurations for 2 different bands DL with 2 band UL within FR1, or within FR2 or including FR2

References

[1]	R4-2000502 Revised WID on Rel-16 NR Inter-band Carrier Aggregation/Dual Connectivity  for 2 bands DL with x bands UL (x=1,2), ZTE Corporation

[2]	R4-2000497 CR to reflect the completed NR inter band CA DC combinations for 2 bands DL with up to 2 bands UL into Rel16 TS 38.101-1, ZTE Corporation

[3]	R4-2000498 CR to reflect the completed NR inter band CA DC combinations for 2 bands DL with up to 2 bands UL into Rel16 TS 38.101-3, ZTE Corporation

[4]	R4-2000803 TR 38.716-02-00 v0.9.0, ZTE Corporation

[5]	RP-200168, Revised WID for,  ZTE Corporation

| 830095 | Rel16 NR inter-band Carrier Aggregation for 3 bands DL with 1 band UL | NR_CA_R16_3BDL_1BUL | R4 | RP-191153 | CATT |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by ZTE Corporation in RP-200173.

This WI is to introduce the band combinations for inter-band carrier aggregation and dual connectivity for 3 bands DL with 2 bands UL into Rel-16 specifications and specify the corresponding configurations and RF requirements.

The study results for each band combination are captured in TR 38.716-03-02, and the configurations and RF requirements are added into the corresponding technical specifications.

This WI includes all target band combinations for both NR CA and DC including the following configurations:

•	New NR CA configurations including inter band CA for 3 different bands DL with 2 different bands UL

o	new configurations from exiting bands

o	new configurations from new band

o	The NR CA configurations will be introduced in a release independent manner based on TS38.307

•	New NR DC configurations including inter band CA for 3 different bands DL with 2 different bands UL

The preconditions to propose NR CA/DC configurations including Inter band CA for 3 different bands DL with 2 different bands UL in rel-16 are as follows.

o	Requirements for all concerning bands shall be completed and specified in advanced

o	Constituent NR band and NR intra band CA shall be completed and specified in advance

Specification for each band combination includes:

o	Applicable frequencies if necessary

o	Applicable bandwidth and bandwidth sets if necessary

o	Analysis on potential self-desensitization due to following reasons:

o	TX Harmonic and/or intermodulation overlap of receive band

o	TX signal overlap of receiver harmonic frequency

o	TX frequency being in close proximity of one of the receive bands

o	Any other identified reasons

o	At least the following for the combination where self-desensitization exists

o	∆TIB, c

o	∆RIB, c

o	Reference sensitivity exceptions including MSD test cases

o	Add conformance testing in RAN5 specifications (to follow at a later stage)

The target band combinations are captured in two tables in this WI respectively:

o	CA configurations for 3 different bands DL with 2 band UL

o	DC configurations for 3 different bands DL with 2 band UL

References

[1]	R4-2000503 Revised WID on Rel-16 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with 2 bands UL, ZTE Corporation

[2]	R4-2000499 CR to reflect the completed NR inter band CA DC combinations for 3 bands DL with 2 bands UL into Rel16 TS 38.101-1, ZTE Corporation

[3]	R4-2000500 CR to reflect the completed NR inter band CA DC combinations for 3 bands DL with 2 bands UL into Rel16 TS 38.101-3, ZTE Corporation

[4]	R4-2000804 TR 38.716-03-02 v0.4.0, ZTE Corporation

[5]	RP-200171, Revised WID for Rel-16 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with 2 bands UL,  ZTE Corporation

## 19.2	Release 16 Features impacting both LTE and NR

| 820071 | Transfer of Iuant interface specifications from 25-series to 37-series | Iuant_transfer | R3 | RP-190160 | Huawei |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Huawei in RP-190144.

This feature transfers the contents of the TSs 25.46x into TSs 37.46x (x=0, 1, 2, 6) for releases 8/9/10/11/12/13/14/15, considering the fact theses specifications apply to different RAT i.e. UTRA, E-UTRA and NR.

Indeed, 25 series of specifications are dedicated to UMTS only, when the material which was previous there referred to multiple RATs, i.e. UTRA, E-UTRA and NR.

Rel-6/7: UMTS only (no changes needed, also no changes possible as these releases are no longer maintained)

Rel-8/9/10/11/12/13/14: UMTS and E-UTRA/LTE

Rel-15: UMTS, E-UTRA/LTE and NR

Note: The TSs 25.4.6x are turned into empty specifications referring to the new created TSs 37.46x.

The TSs 25.46x are not withdrawn so references to these specs will lead to the correct specs.

Additional remarks:

1. This transfer of data over 8 releases is an exceptional case: Instead of creating Rel-8 specs and step-by-step upgrade to higher releases via CRs (which would be the normal approach), it was decided with the specs manager to bring v1.N.0 of TS 37.46x for 1-step approval to RAN #83 and instead of approving them to vN.0.0 it was decided to approve them the same version as the last 25.46x version that is created via the CRs.

2. The text shown in TS 37.46x is taken 1:1 from the corresponding TS 25.46x showing changes of the contents via revision marks (note: spec cover and history adaptations are done without revision marks). The changes are mainly to indicate the considered RATs in the scope, to replace 25.46x references by 37.46x references and to correct obvious problems. Further technical changes of the contents of the specifications may rather be considered in a future cat.F CR (and probably just focusing on the latest releases).

3. There is also a TS 25.463 which exists for Rel-6 and Rel-7 only. TS 25.466 replaced TS 25.463 in Rel-7 and future releases. This means for Rel-6 TS 25.463 is still valid and cannot be withdrawn. Rel-7 is no longer maintained i.e. no CRs are possible i.e. 3GPP internal references to TS 25.463 could not be corrected anymore, so also 25.463 Rel-7 will not be withdrawn but it will be clarified on the 3GPP webpage that for Rel-7 TS 25.466 should be considered instead of TS 25.463.

4. References of Rel-16 and higher should refer to TS 37.46x instead of TS 25.46x. Changes of existing references of Rel-8 to Rel-15 specifications of TS 25.46x may be corrected to references to TS 37.46x but such a change is not mandatory as 25.46x Rel-8 to Rel-15 will remain. A further upgrade of 25.46x Iuant specs to Rel-16 and higher will not be considered. Rel-16 and higher specifications shall refer to TS 37.46x.

Following this transfer, RAN3 agreed to transfer the ownership of the clauses currently owned by RAN4 in TS 37.461 back to RAN3. RAN3 informed RAN4 by LS (R3-191145).

References

[1]	R3-190104	Presentation of Specifications to TSG: TS 37.460 v1.N.0, TS 37.461 v1.N.0, TS 37.462 v1.N.0, TS 37.466 v1.N.0 (N=8/9/a/b/c/d/e/f)	ETSI MCC, Huawei, Ericsson

| 830086 | Introduction of GSM, UTRA, E-UTRA and NR capability set(s) (CS(s)) to the multi-standard radio (MSR) specifications | MSR_GSM_UTRA_LTE_NR | R4 | RP-190642 | Ericsson |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Ericsson in RP-191816.

The WI added support for MSR BS including GSM, UTRA, E-UTRA and NR combinations by means of creating two new CS, CS 18 (GSM+E-UTRA+NR) and CS19 (UTRA+E-UTRA+NR).

The WI created two new capability sets, CS 18 (GSM+E-UTRA+NR) and CS19 (UTRA+E-UTRA+NR). A BS with multiple connectors can support all 4 RATs by transmitting CS18 on at least one connector and CS19 on at least one connector. To add these CS, the following was needed:

- Updating of the MSR core specification to include correct OBUE emissions tables for GSM/UTRA when operating together with MSR as well as some fixes to the blocking and RX IM requirements.

- Introduction in the conformance specification of:

  - The new capability sets

  - Test configurations relating to the new capability sets

  - Test applicability rules relating to the new capability sets and test configurations

  - Updated test requirements corresponding to the core requirement updates.

All of the new functionality was introduced in the two approved CRs in [1] and [2].

References

[1] 	R4-1908049	Introduction of requirements for NR + UTRA/GSM combinations, Ericsson, Nokia, Nokia Shanghai Bell, Vodafone, ZTE

[2]	R4-1910476	Introduction of requirements for NR + UTRA/GSM combinations, Ericsson, Nokia, Nokia Shanghai Bell, Vodafone, Huawei

| 820072 | Direct data forwarding between NG-RAN and E-UTRAN nodes for inter-system mobility | Direct_data_fw_NR | R3 | RP-182886 | Ericsson |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Ericsson in RP-192162.

This WI specified direct data forwarding per bearer during inter-system mobility between NG-RAN and E-UTRAN nodes in addition to the solution for indirect data forwarding specified in Rel-15 during the NR WI.

This WI continued discussions on direct data forwarding between NG-RAN and E-UTRAN nodes during the Rel-15 discussions on the NR WI. Direct data forwarding encompasses the following two main features:

-	While data forwarding between NG-RAN and E-UTRAN nodes in Rel-15 foresees establishing the data forwarding path via the EPC and the 5GC, direct data forwarding allows establishing the data forwarding path directly between the involved RAN nodes, without the need to interact with user plane functions of the EPC and the 5GC.

-	Indirect data forwarding for inter-system handover in Rel-15 foresees transporting to be forwarded user plane data between the NG-RAN node and the UPF via PDU Session tunnels. As forwarding paths on the E-UTRAN side are established on a per-E-RAB basis, the Rel-15 solution requires interworking functions at the UPF and the NG-RAN node in order to map per-DRB / per-E-RAB packets to the per-PDU Session tunnel and vice versa. Such interworking is not necessary in case of direct data forwarding.

In the course of the WI RAN3 realised that direct data forwarding tunnel related information may be communicated between the NG-RAN node and the 5GC in a way that the inter-system handover signalling performance benefits from such approach, which was realised in the agreed CRs in [3] and [4].

Further, SA2 and CT4 where contacted to perform necessary work in their areas of responsibility [5].

References

[1]    RP-182886	"New WID: Direct data forwarding for inter-system mobility", approved at RAN#82

[2]    RP-191369	"Status report of WI: Core part: Direct data forwarding between NG-RAN and E-UTRAN nodes for inter-system mobility"

[3]    R3-192626	"Support of Direct Data forwarding for handover between 4G and 5G", agreed CR0041r4 38.413

[4]    R3-193272	"Introduction of direct and indirect data forwarding for inter-system HO between EPS and 5GS", endorsed CR 38.300

[5]    R3-193284	"LS on Finalization of Direct Data Forwarding feature", agreed LS out to SA2 and CT4

| Unique_ID | Name | Acronym | WG | WID | WI Rapporteur |
| --- | --- | --- | --- | --- | --- |
| 790054 | eNB(s) Architecture Evolution for E-UTRAN and NG-RAN | LTE_NR_arch_evo | R3 | RP-180531 | China Unicom |

Summary based on the input provided by China Unicom in RP-192964.

In Rel-15, the F1 interface was standardized in the gNB (LTE) to support gNB-CU and gNB-DU split, i.e. to split the Control Unit from the Data Unit (supporting the user data).

Similarly, in Rel-16, the W1 interface is standardized in the ng-eNB (5G) to support ng-eNB-CU and ng-eNB-DU split.

These two interfaces adopt the same higher layer split architecture for between PDCP and RLC, with similar functionalities, such as control plan functionalities.

The ng-eNB-CU and gNB-CU could be physically collocated, deployed in a same physical "box". This unification of the , as to unify the ng-eNB and gNB architectures would simplify the network deployment complexity and facilitate hardware upgrades.

The W1 interface specifications includes the general aspects and principles (TS 37.470), W1 layer 1 (TS 37.471), signalling transport (TS 37.472) and W1AP aspects (TS 37.473). The general aspects and principles and W1 layer1 are aligned with F1 interface. For signalling transport, the W1-C signalling bearer aligned with F1 interface, function and protocol stack in figure 1 are defined.

Figure 1: W1-C signalling bearer protocol stack

The functionalities of W1 interface are supported over W1-C and W1-U (in a similar way as for the F1 interface).

The W1-C functionalities are the following:

- Interface management: W1 setup, eNB-CU/eNB-DU configuration update, error indication function, reset function coordination function.

- system information management: system information is critical for high layer split architecture, eNB-DU is responsible for transmitting MIB, SIB1, SIB2, SIB3, SIB8 and SIB 16. Other system information is responsible by ng-eNB-DU.

- Paging function: The eNB-DU is responsible for transmitting the paging information. While, the eNB-CU provides paging information to enable the eNB-DU to calculate the exact PO and PF.

- UE context management function: The W1 UE context management function supports the establishment and modification of the necessary overall UE context.

- RRC message transfer function:  RRC message transfer function supports the establishment and modification of the necessary overall UE context. This function allows to transfer RRC messages between eNB-CU and eNB-DU.

- Warning message information transfer function: This function allows to cooperate with the warning message transmission procedures over NG interface.

The W1-U functionalities are specified as below:

- Transfer of user data: The functionalities are used to transfer of user data between ng-eNB-CU and ng-eNB-DU.

- flow control function: It is designed for control the downlink user data flow to the ng-eNB-DU.

More detail procedures and IEs for W1 can be founded in TS 37.473 on W1AP specification.

References

[1]	 The WID

[2]	RP-191972  Status report for WI Core part: eNB(s) Architecture Evolution for E-UTRAN and NG-RAN WI (China Unicom)

[3]	R3-187270	pCR to 37.470 on W1AP functions and procedures (Huawei, China Unicom, TIM)

[4]	R3-191163	pCR to 37.470 on W1AP functions and procedures(Huawei, TIM, China Unicom)

[5]	R3-193174	pCR to 37.470 on W1AP functions and procedures (Huawei, China Unicom, Orange, TIM)

[6]	R3-194676	pCR to 37.470 on W1 general aspects and principles (China Unicom)

[7]	R3-194678	pCR to 37.472 on W1 signalling transport (China Unicom)

[8]	R3-194425	pCR to 37.470 on W1AP functions (Huawei, China Unicom, TIM)

[9]	R3-194426	pCR for 37.473 on UE context management procedures (Huawei, China Unicom, TIM)

[10]	R3-194427	pCR for 37.473 on Interface Management procedures (Huawei, China Unicom, TIM)

[11]	R3-194565	Structure pCR for TS 37.473 (Huawei, China Unicom, TIM)

[12]	R3-194562	pCR to 37.470 on W1AP functions (Huawei, China Unicom, TIM, Orange)

[13]	R3-194563	pCR for 37.473 on UE context management procedures (Huawei, China Unicom, TIM, Orange)

[14]	R3-194564	pCR for 37.473 on Interface Management procedures (Huawei, China Unicom, TIM, Orange)

[15]	R3-197544	pCR to 37.473 on ASN.1 completition (Huawei, China Unicom, TIM)

[16]	R3-197636	pCR to 37.473 on miscellaneous correction to contexts (Huawei, China Unicom, Orange, TIM)

[17]	R3-197544	CR to 38.401 on corrections to ng-eNB deployment (Huawei, China Unicom, Orange, TIM)

| 820079 | High power UE (power class 2) for EN-DC (1 LTE TDD band + 1 NR TDD band) | ENDC_UE_PC2_TDD_TDD | R4 | RP-190315 | CMCC |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by CMCC in RP-200214.

This WI contains a general part and band specific combination part for high power UE (power class 2) for EN-DC (1 LTE TDD band + 1 NR TDD band). The purpose is to introduce high power UE (power class2) for EN-DC (1 LTE TDD band +1 NR TDD band). The actual requirements are added to the corresponding technical specifications.

The purpose of this WI on High power UE (power class 2) for EN-DC (1 LTE TDD band + 1 NR TDD band) is to improve the UP coverage of 5G UE. In order to reduce the big imbalance between 5G NR uplink and downlink coverage, we can improve uplink coverage by defining high power UE for SA and NSA, and it is a feasible way to improve the uplink coverage. This PC2 EN-DC (TDD+TDD) feature will meet the operator's deployment request for uplink coverage enhancement.

In this work item, we focus on the PC2 EN-DC RF requirements including UE maximum output power, Tx power tolerance, MPR, A-MPR, configured output power, ACLR, SAR and n41-B40 UE-UE co-existence for Power Class 2 EN-DC.

There are two important issues that have been addressed in this topic, and in the end we have resolved them through appropriate solutions

For SAR issue, RAN4 specified UE Tx duty cycle requirements sufficient to prevent exceeding local regulatory limits such as SAR for inter-band PC2 EN-DC UE.

For PC2 EN-DC (TDD+TDD) release independent issue, RAN4 reached a conclusion that PC2 inter-band EN-DC (LTE TDD PC3 + NR TDD PC3) can be supported from Rel-15 in release independence manner.

This WI was completed in the RAN#86 meeting and the complete version of NR Technical Report 37.825 has been approved.

References

[1]R4-1913208	    TR 37.825 v0.4.0 Finalization, CMCC, RAN4#93

[2]R4-1915992     CR for adding solutions for addressing SAR issue for EN-DC PC2 UE, CATT，CMCC, RAN4#93

[3]R4-1913209    CR for REL-16 TS 38.307 for PC2 EN-DC TDD+TDD, CMCC, RAN4#93

[4]R4-1913210    CR for REL-15 TS 38.307 for PC2 EN-DC TDD+TDD, CMCC, RAN4#93

[5]RP-192583      Presentation of TR 37.825 V1.0.0，CMCC，RAN#86

## 19.3	LTE-related Release 16 Features

| Unique_ID | Name | Acronym | WG | WID | WI Rapporteur |
| --- | --- | --- | --- | --- | --- |
| 800079 | Further performance enhancement for LTE in high speed scenario | LTE_high_speed_enh2 | R4 | RP-181482 | NTT DOCOMO |

Summary based on the input provided by NTT DOCOMO, INC. in RP-192721.

In Rel-13 and 14, the mobility and throughput performance were enhanced to cover high speeds (up to 350 km/h) by specifying the requirements for UE RRM, UE demodulation and base station demodulation, considering the two types of operator’s practical deployments shown in Figures 1 and 2. Figure 1 shows the case where no specific installation is deployed to handle high-speed trains, i.e. UEs in the train use the "standard" LTE eNBs. Alternatively, figure 2 shows the case where Single Frequency Network (SFN) are deployed. SFNs use so-called "Remote Radio Heads" (RRH), which are dedicated antennas deployed along the train track. In this case, the baseband unit (BBU) is connected to the RRH, e.g. using fiber.

Figure 1. Non-Single Frequency Network (SFN) high speed scenario

Figure 2. SFN high speed scenario

These Rel-13 and 14 enhancements were conducted both for non-SFN and for SFN, but only for LTE single carrier, i.e. not covering Carrier Aggregation (CA).

This Rel-16 work item further improves the mobility and throughput performance, now considering CA and speeds up to 500 km/h. To this aim, it enhances RRM, UE demodulation and base station demodulation: it specifies enhanced RRM core requirements and corresponding RRC signals in respectively TS 36.133 [2] and TS 36.331 [3].

Part 1: RRM requirements enhancements:

In Release 14 cases (limited to 350 km/h and single carrier), the latency requirements under DRX configuration up to 1.28s DRX cycle were enhanced by reducing the cell identification delay in connected mode and cell reselection delay in idle mode [1].

In Rel-16, considering Carrier Aggregation and speeds up to 500km/h, the following enhanced requirements were introduced to achieve good mobility performance and less paging outage:

- Enhanced RRM requirements for active SCells (for 350km/h velocity):

The same requirements specified in Rel-14 high speed WI are applied to active SCells.

- Enhanced RRM requirements for deactivated SCells (for 350km/h velocity):

The cell identification delay and measurement period are reduced.

- Enhanced RRM requirements in DRX in connected mode (for 500km/h velocity):

The cell identification delay and measurement period on 1.28s DRX cycle are further reduced from those in Rel-14 high speed WI.

- Enhanced RRM requirements in idle mode (for 500km/h velocity):

The cell detection delay is further reduced from those in Rel-14 high speed WI.

- Enhanced UL timing adjustment requirements in connected mode (for 500km/h velocity):

The larger maximum autonomous time adjustment step is applied when the downlink bandwidth is wider than 10MHz.

Part 2: Demodulation enhancements

- For UE and base station demodulation enhancements

In Release 14, UE and base station demodulation requirements were enhanced [1], for both cases of operator’s practical deployments shown in figures 1 and 2.

In Release 16, regarding the CA case in SFN (figure 2), the requirements specified in Rel-14 are expanded to Dual Connectivity's Secondary Cells (SCells) as defined in TS 36.331 [3]. Regarding further high speed up to 500 km/h, additional requirements are introduced to ensure the PDSCH/PUSCH/PRACH demodulation performance with larger Doppler shift.

The impacts on RRC signals to meet the new requirements above are also specified.

References

[1]		RP-160172, Rel-14 WID on "Performance enhancements for high speed scenario in LTE"

[2]       TS 36.133, “Requirements for support of radio resource management.”

[3]       TS 36.331, “Radio Resource Control.”

# 20	All other Release 16 Features

| Unique_ID | Name | Acronym | WG | WID | WI_rapporteur_name |
| --- | --- | --- | --- | --- | --- |
| 810023 | Energy Efficiency of 5G | EE_5G | S5 | SP-191193 | Jean-Michel Cornily, Orange |

Summary based on the input provided by Orange in [tdoc numbed].

The term ‘Energy Efficiency’ (EE) is to be understood as covering two areas:

1. Definition of EE KPI, required performance measurements and method(s) to collect them;

2. Definition of use cases, requirements and solutions to optimize Energy Efficiency – this is also sometimes referred to as Energy Saving.

Both aspects have been addressed in SA5 for pre-5G radio access technologies, focusing on how OA&M can provide support to them. This work item (EE_5G) addresses NG-RAN.

TS 28.310 [1] captures use cases, requirements and procedures for collecting performance measurements (namely data volume measurements and PEE (Power, Energy and Environment) parameters from 5G base stations. The EE KPI for 5G base stations is obtained thanks to these measurements. Those measurements are collected by the OSS thanks to the usage of generic management services defined in TS 28.532. Data Volume (DV) measurements, at PDCP layer, have been introduced in TS 28.552 [2], as well as PEE parameters definition (amongst which the Energy Consumption (EC) of the base station). A new KPI, named EE KPI, calculated by dividing Data Volume (DV) by Energy Consumption (EC) has been defined in TS 28.554 [5]. All this is fully aligned with baseline ETSI TC EE (Environmental Engineering) specifications ES 203 228 [7] and ES 202 336-12 [8].

Some intra- and inter-RAT energy saving use cases and requirements have been specified in TS 28.310 [1]. The 5G Network Resource Model (TS 28.541 [9]) has been augmented with an Energy Saving NRM fragment enabling the management of both centralized and distributed energy saving modes.

References

[1] TS 28.310: Management and orchestration; Energy efficiency of 5G

[2] TS 28.552: Management and orchestration; 5G performance measurements

[3] SP-190755: Add Power, Energy and Environmental (PEE) measurements and related use case description

[4] SP-190119: Add PDCP data volume measurements for EE

[5] TS 28.554: Management and orchestration; 5G end to end Key Performance Indicators (KPI)

[6] SP-191165: Add 5G Energy Efficiency KPI

[7] ETSI ES 203 228: Environmental Engineering (EE); Assessment of mobile network energy efficiency

[8] ETSI ES 202 336-12: Environmental Engineering (EE); Monitoring and control interface for infrastructure equipment (power, cooling and building environment systems used in telecommunication networks); Part 12: ICT equipment power, energy and environmental parameters monitoring information model

[9] TS 28.541: Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3

| Unique_ID | Name | Acronym | WG | WID | WI Rapporteur |
| --- | --- | --- | --- | --- | --- |
| 790019 | Usage of CAPIF for xMB API | CAPIF4xMB | S4 | SP-180031 | Thorsten Lohmar, Ericsson LM |

Summary based on the input provided by Ericsson in SP-181187.

The CAPIF4xMB work item resulted in the creation of the xMB specification in TS.26.348 [1]. This TS contains the reference point specification for the external interface towards content providers or other 3GPP defined API invokers.

3GPP SA4 aligned the xMB reference point to the Common API Framework (CAPIF) within the CAPIF4xMB work item targeting Release 16. The xMB reference point exists between Content Providers (as API invoker) and the BM-SC. An API invoker can be a 3rd party provider or other 3GPP defined services.

xMB offers a simple interface to control the MBMS delivery procedure and to ingest the data to be delivered via MBMS. The MBMS usage complexity, e.g. handling of packet losses, etc is handled by the BM-SC.

xMB offers control procedures for different session types, namely generic file delivery, application (timed file streaming like DASH over MBMS), streaming and transport-only (access to a plain IP multicast transport). The xMB reference point offers an extended set of control features for MC Services.

As part of the CAPIF4xMB work item, the xMB Stage 2 architecture and procedures are moved from TS 26.346 to a new TS 26.348, containing the xMB Stage 2 and necessary extensions for CAPIF. The new TS also contains more elaborative guidelines around the usage of the different xMB API features.

References

[1]	TS 26.348, "Northbound Application Programming Interface (API) for Multimedia Broadcast/Multicast Service (MBMS) at the xMB reference point"

| 810024 | OAM aspects of LTE and WLAN integration | OAM_LTE_WLAN | S5 | SP-180820 | Yizhi Yao, Intel Corporation |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Intel Corporation in SP-191252.

The OAM_LTE_WLAN work item defined the requirements and solutions for management aspects of LWA (LTE-WLAN aggregation) and LWIP (LTE WLAN integration with IPsec tunnel) features defined by 3GPP RAN WGs. The management aspects of LWA and LWIP addressed by this work item include:

-   Configurations to eNB to support LWA and LWIP;

-   Management model for WT for co-located and non-co-located LWA deployment scenarios; and

-   Performance measurements to monitor the LWA (including co-located and non-co-located scenarios) and LWIP.

The work item resulted in a number of CRs on TS 32.425, 28.657, 28.658, and 28.659 based on the preceding TR 32.868.

| 840066 | Volume Based Charging Aspects for VoLTE | VBCLTE | S5 | SP-180813 | Chen, Ai, China Mobile |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by China Mobile in SP-191277.

Based on the Study Item FS_VBCLTE (3GPP TR 32.848) which has investigated potential charging requirements and possible mechanisms to support volume based charging for VoLTE, the WI VBCLTE is the enhancement of online and offline charging aspects of volume based charging for VoLTE.

Stage 2 work on WI VBCLTE for TS 32.251 [1] and TS 32.260 [4]:

-	Clarify that IMS charging only supports duration based charging for VoLTE.

-	Introduce volume based charging for VoLTE in PS offline and online charging.

-	Introduce VoLTE Information parameter for volume based charging for VoLTE in PGW-CDR.

-	Add Bindings of CDR parameter, Information Element and AVP for VoLTE Information parameter.

Stage 3 work on WI VBCLTE for 32.298 [3] and 32.299 [4]:

-	Introduce two new values for Change-Condition AVP to indicate the status of VoLTE service delivery.

-	Introduce two new reasons of Cause for Record Closing field in the CDR to indicate the status of VoLTE service delivery.

-	Introduce VoLTE Information parameter including Caller Information and Callee Information for volume based charging for VoLTE in PS domain CDRs.

-	Introduce VoLTE-Information AVP including Calling-Party-Address AVP and Callee-Information AVP for volume based charging for VoLTE in PS domain online charging messages.

References

[1]	TS 32.251: "Packet Switched (PS) domain charging"

[2] 	TS 32.260: "IP Multimedia Subsystem (IMS) charging"

[3] 	TS 32.298: "Charging Data Record (CDR) parameter description"

[4]	TS 32.299: "Diameter charging application"

| 820031 | Charging Enhancement of 5GC interworking with EPC | 5GIEPC_CH | S5 | SP-181067 | Chen Shan, Huawei |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Huawei in SP-190855.

The WI 5GIEPC_CH is the enhancement of the WI 5GS_Ph1-SBI_CH and 5GS_Ph1-DCH (handover with N26 scenario) on charging aspect of 5GC interworking with EPC, to specify procedure, parameter and Open API impact for the handover cancel and handover without N26 scenarios.

Stage 2 work on WI 5GIEPC_CH for TS 32.255 [1] and TS 32.290 [2]:

-	New message flow for EPS to 5GS handover for roaming in Home routed scenario, reflecting the "Visited created Charging Id, Home provided Charging ID" conveyed between VPLMN and HPLMN, and their use within each PLMN.

-	Definition of charging information for interworking with EPC: use of 5G attributes and CDR fields extended with EPC specific ones in the new Interworking Annex.

-	Handover triggers for handover cancel and handover without N26 scenarios.

Stage 3 work on WI 5GIEPC_CH for 32.291 [3] and 32.298 [4]:

-	TS 32.291 Nchf_ConvergedCharging API

-	Extensions of Data model

-	Update OpenAPI

-	TS 32.298 ASN.1 CHF CDR with "charging Ids" and extended EPC parameters

References

[1]	TS 32.255: "5G data connectivity domain charging"

[2] 	TS 32.290: "5G system; Services, operations and procedures of charging using Service Based Interface (SBI)"

[3]	TS 32.291: "5G system; Charging service, stage 3"

[4] 	TS 32.298: "Charging Data Record (CDR) parameter description"

| 770020 | Service Interactivity | SerInter | S4 | SP-170796 | Lo, Charles, Qualcomm |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Qualcomm Incorporated in SP-191247.

This summarizes the progress of the normative specifications accomplished during the course of the SerInter work item [1]. The related agreed CRs can be found in the Tdocs [2], [3], [4], [5] and [6].

The service interactivity feature enables dynamic user engagement and resulting auxiliary content presentation during the consumption of a streaming service or content item, received over unicast or broadcast.  Interactive service capabilities can be further personalized to the end user consuming the service. Examples of service interactivity functionality include voting, rating, purchasing, online chats, and reception of targeted advertisements and other content, in real-time during the viewing of a streaming program.

Technical functionality to enable and support dynamic and personalized service interactivity are added to 3GPP TS 26.247 [7], TS 26.346 [8] and TS 26.347 [9] and comprise the following components:

- Signaling of upcoming interactivity events to native or Web app based interactivity applications. Such interactivity event signaling, which could be sent at specific and potentially arbitrary times during the consumption of an associated 3GPP User Service, contain metadata of upcoming interactivity events and which enable an interactivity application to perform its intended task. The interactivity event signaling is defined by the DASH Industry Forum upon request from 3GPP, and is implemented as either  a DASH Event stream, or as samples of a ISOBMFF timed metadata track.

- Processing model and rules for the 3GP-DASH Client to extract metadata contained in interactivity event signaling   .

- WebIDL APIs exposed by the 3GP-DASH Client to an interactivity application, for the subscription by and delivery to, the application, of interactivity event metadata.

(The above functionalities are defined by the DASH Industry Forum, based on request from 3GPP to support the Service Interactivity work item, in the specification “DASH Player’s Application Events and Timed Metadata Processing Model and APIs” [10].)

- For DASH-formatted streaming services, signaling via the DASH MPD of the intended measurement and reporting of interactivity-related usage by the user/device. This signaling enables the service provider to: a) specify the parameters and criteria regarding interactivity consumption reporting; b) specify the type of interactivity usage report to be submitted by the DASH client, and c) employ either random or selective control of the user/device population to perform the reporting. In addition, the XML-based interactivity usage report format and HTTP POST based reporting protocol are defined. The above functionality are specified in TS 26.247 [7].

- Clarifications to the MBMS APIs specification TS 26.347 [9] that an MBMS-Aware Application providing an auxiliary service interactivity feature is expected to acquire, from the MBMS client, a multiplicity of Application User Services. These Application User Services provide the media content of the DASH-over-MBMS application service as well as media content and/or metadata of adjunct application service(s) that provide the service interactivity functionality.

An architecture illustrating the high level interactions between the relevant entities, in the delivery of DASH-formatted content and interactivity event signaling from the network to the 3GP-DASH Client, forwarding of event signaling data from the 3GP-DASH Client to the interactivity application, and subsequent execution by the application of its interactivity task, is shown in the diagram below, which is copied from TR 26.953 [11].

References

[1] 	Tdoc SP-170796, “New WID on Service Interactivity” (SerInter).

[2]	Tdoc S4-180854, CR 26.247-0145, “Signaling and Reporting of Interactivity Usage in 3GP-DASH”.

[3] 	Tdoc S4-190182, CR 26.346-0625, “TS 26.346 Changes Pertaining to Service Interactivity Feature”.

[4] 	Tdoc S4-190196, CR 26.347-0006, “Changes Pertaining to Service Interactivity”.

[5] 	Tdoc S4-191314, CR 26.247-0163, “Support for Service Interactivity via Event Signaling and DASH APIs”.

[6]	Tdoc S4-191318, CR 26.346-0629, “Service Interactivity Support in DASH-over-MBMS Service”.

[7] 	3GPP TS 26.247 “Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)”.

[8] 	3GPP TS 26.346 “Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs”.

[9]	3GPP TS 26.347 “Multimedia Broadcast/Multicast Service (MBMS); Application Programming Interface and URL”.

[10]	DASH Industry Forum specification “DASH Player’s Application Events and Timed Metadata Processing Model and APIs”.

[11]	3GPP TR 26.953 “(MBMS and PSS) Interactivity Support for 3GPP-Based Streaming and Download Services”.

| 850003 | RTCP Verification for Real-Time Services | RTCPVer | S4 | SP-190639 | Burman, Bo, Ericsson LM |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Ericsson LM in SP-200034.

This summarizes the progress of the new, normative specification accomplished during the course of the RTCPVer work item [1].

The RTCP verification for real-time services feature enables verification of the most important parts of the RTP/RTCP and SRTP/SRTCP protocols used by current 3GPP conversational and real-time services.

Technical functionality to enable and support this verification are added to a new 3GPP TS 26.139 [2] and comprise the following aspects:

-	Explicitly allowing for and describing examples of different, possible test architectures.

-	Test cases needed to ensure an adequate level of RTP operation and RTP stream monitoring.

-	Test methods capable to verify that information contained in the RTP header and in RTCP is correct and consistent with the observed characteristics of the related RTP streams:

o	Between RTP/RTCP within the scope of a single RTP stream (e.g. between an RTP stream and the corresponding RTCP reporting from the remote party, or between an RTP stream and the corresponding RTCP metadata, e.g. for sampling clock accuracy compensation between RTP sender and RTP receiver).

o	Between RTP/RTCP across RTP streams in the same RTP session (e.g. between sent and received RTP streams, or between audio RTP streams and video RTP streams).

-	Requirements on what constitutes acceptable RTP/RTCP protocol field values, including RTP payload header and RTP payload length, based on the observed characteristics of the related RTP streams.

-	A method for an RTP/RTCP implementation to announce that it has passed the necessary tests and conforms to the new specification at call setup and during the call.

References

[1] 	Tdoc SP-190639, "New WID on ‘RTP/RTCP Verification for Real-Time Services’" (RTCPVer).

[2] 	3GPP TS 26.139 "Real-time Transport Protocol (RTP) / RTP Control Protocol (RTCP) verification procedures".

| 820042 | Stage-3 SAE Protocol Development for Rel16 | SAES16 | C1 | CP-183088 | Aghili, Behrouz, InterDigital Communications |
| --- | --- | --- | --- | --- | --- |
| 820038 | IMS Stage-3 IETF Protocol Alignment | IMSProtoc16 | C1 | CP-183084 | Leis, Peter, Nokia |

The description of these WIDs has not changed at all from its predecessors, e.g. SAES6, SAES5, etc.

The contents of the WID are exact copies of the ones before as it is an “ongoing” WID at CT1, meaning that it keeps existing for every new release until CT1 makes a decision that it is no longer needed. The only thing that happened this time, i.e. for the Rel-16 version, was that we decided to align the “x” in SAESx with the corresponding release so, as of Rel-16, the WID is called SAES16 and it will be SAES17 for Rel-17.

Same with IMSProtocolx.

| Unique_ID | Name | Acronym | WG | WID | WI_rapporteur_name |
| --- | --- | --- | --- | --- | --- |
| 840017 | Reliable Data Service Serialization Indication | RDSSI | S2 | SP-190446 | Michael Starsinic, Convida Wireless LLC |

Summary based on the input provided by Convida Wireless LLC in SP-200057.

This work item enhances the Reliable Data Service by adding support to the Reliable Data Service for indicating the serialization format of the data that will be sent in a NIDD session

The Reliable Data Service was added to EPS in Rel-14 as part of the “Extended architecture support for Cellular Internet of Things” work item and added to 5GS in Rel-16 as part of the “Cellular IoT support and evolution for the 5G System” work item.

The work was motivated by an LS from oneM2M and resulted in TS 23.682, TS 23.501, and TS 23.502 CRs which added support to the Reliable Data Service for indicating the serialization format of the data that will be sent in a NIDD session. The CRs also updated the NIDD Configuration procedure so that the SCS/AS can indicate to the SCEF or NEF the serialization format(s) that are supported by the SCS/AS.

References

[1] S2-1903031, NIDD RDS API enhancement, LS from oneM2M Technical Plenary to CT3

[2] S2-1903039, LS Reply on NIDD RDS API enhancement, LS from CT3 to SA2 and oneM2M Technical Plenary

[3] S2-1906025, Adding Support for Indicating Serialization Format in RDS, TS 23.682 CR, Convida Wireless LLC, Intel, Deutsche Telekom, AT&T

[4] S2-1906027, Adding Support for Indicating Serialization Format in RDS, TS 23.502 CR, Convida Wireless LLC, Intel, Deutsche Telekom, AT&T

[5] S2-1906162, Adding Support for Indicating Serialization Format in RDS, TS 23.501 CR, Convida Wireless LLC, Intel, Deutsche Telekom, AT&T

| 800046 | Shared Data Handling on Nudm and Nudr | Shared_Data | C4 | CP-181136 | Wiehe, Ulrich, Nokia |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Nokia, Nokia Shanghai Bell in CP-200143.

The problem of potential signalling floods on Nudm and Nudr service based interfaces that are caused by (identical) subscription data changes for a huge set of subscriptions (e.g. MTC devices) is solved by the concept of shared data.

Subscription data that are retrieved by serving nodes (AMF, SMF, SMSF) from the UDM (and by the UDM from the UDR) may contain a subset that is shared by a huge number of subscriptions (e.g. 1000 specific MTC devices). A simple example for such a subset is e.g. a subscribed periodic registration time with a value of 300 seconds. When the operator decides to modify the value for all these MTC devices to e.g. 400 seconds, a single notification per serving node is sent to update the shared subset; there is no need to send 1000 notifications addressing each single subscription.

The solution is a pure stage 3 signalling optimization issue and is not based on specific stage1 or stage 2 requirements.

CT4#84 decided to implement the solution already in Rel-15.

References

Impacted TSs:

-	3GPP TS 29.503: "Unified Data Management Services".

-	3GPP TS 29.504: "5G System; Unified Data Repository Services; Stage 3".

-	3GPP TS 29.505: "5G System; Usage of the Unified Data Repository Services for Subscription Data; Stage 3".

Main CRs

29.503 CR 0008r4 C4-186497

29.504 CR 0010r1 C4-187507

29.505 CR 0019r1 C4-187506

| 810049 | Remote Identification of Unmanned Aerial Systems | ID_UAS |  | SP-180771 | Eddy HALL, Qualcomm |
| --- | --- | --- | --- | --- | --- |

This item was moved to Rel-17.
Annex A:
List of expected contributions

Summary needed for highlighted items. Summary to be produced by the Rapporteur as highlighted in the table below.

Green: summary provided.

No need for summary for items shown with strike through font.

| UID | Name | Acronym | WG | WID | Rapporteur |
| --- | --- | --- | --- | --- | --- |
| 0 | Release 16 Features and related studies | - |  | - | - |
| 0 | Enhancement of Ultra-Reliable (UR) Low Latency Communications (URLLC) | - |  | - | - |
| 830098 | Enhancement of URLLC support in the 5G Core network | 5G_URLLC | S2 | SP-181122 | Hui Ni, Huawei |
| 790008 | Study on enhancement of URLLC supporting in 5GC | FS_5G_URLLC | S2 | SP-180118 | Hui Ni, Huawei |
| 810036 | Study on the security of URLLC for 5GS | FS_5G_URLLC_SEC | S3 | SP-180910 | Rong Wu, Huawei Technologies |
| 820019 | Stage 2 of 5G_URLLC | 5G_URLLC | S2 | SP-181122 | Hui Ni, Huawei |
| 840026 | Security of URLLC for 5GS | 5G_URLLC_SEC | S3 | SP-190351 | Rong Wu, Huawei Technologies |
| 840005 | CT Aspects of 5G_URLLC | 5G_URLLC | ct | CP-191063 | Qi Caixia, Huawei |
| 840060 | CT3 Aspects of 5G_URLLC | 5G_URLLC | C3 | CP-191063 | Qi Caixia, Huawei |
| 840061 | CT4 Aspects of 5G_URLLC | 5G_URLLC | C4 | CP-191063 | Qi Caixia, Huawei |
| 780030 | Stopped - Study on EPC support for Mobility with Low Latency Communication | FS_LLC_Mob | S2 | SP-171069 | Chris PUDNEY |
| 790009 | Stopped - Study on enht of systems using EPS for UR and Availability using commodity equipment | FS_EPS_URACE | S2 | SP-180119 | Chris Pudney |
| 830074 | Physical Layer Enhancements for NR Ultra-Reliable and Low Latency Communication (URLLC) | NR_L1enh_URLLC | R1 | RP-191584 | Huawei |
| 800095 | Study on physical layer enhancements for NR UR Low Latency Cases | FS_NR_L1enh_URLLC | R1 | RP-181477 | Huawei |
| 830174 | Core part: Physical Layer Enhancements for NR Ultra-Reliable and Low Latency Communication (URLLC) | NR_L1enh_URLLC-Core | R1 | RP-190726 | Huawei |
| 830274 | Perf. part: Physical Layer Enhancements for NR Ultra-Reliable and Low Latency Communication (URLLC) | NR_L1enh_URLLC-Perf | R4 | RP-190726 | Huawei |
| 830080 | Support of NR Industrial Internet of Things (IoT) | NR_IIOT | R2 | RP-191561 | Nokia |
| 810060 | Study on channel modeling for indoor industrial scenarios | FS_IIIOT_CM | R1 | RP-182138 | Ericsson |
| 800098 | Study on NR Industrial Internet of Things (IoT) | FS_NR_IIOT | R2 | RP-181479 | Nokia |
| 830180 | Core part: Support of NR Industrial Internet of Things (IoT) | NR_IIOT-Core | R2 | RP-190728 | Nokia |
| 820999 | 5GS Enhanced support of Vertical and LAN Services | - |  | - | - |
| 750004 | Study on Communication for Automation in Vertical Domains | FS_CAV | S1 | SP-170169 | Walewski, Joachim, Siemens AG |
| 800007 | Service requirements for cyber-physical control applications in vertical domains | cyberCAV | S1 | SP-180321 | Joachim W. Walewski (Siemens) |
| 830042 | 5GS Enhanced support of Vertical and LAN Services | Vertical_LAN | S2 | SP-181120 | Devaki Chandramouli, Nokia |
| 800028 | Study on 5GS Enhanced support of Vertical and LAN Services | FS_Vertical_LAN | S2 | SP-180507 | Devaki Chandramouli, Nokia |
| 820017 | Stage 2 of Vertical_LAN | Vertical_LAN | S2 | SP-181120 | Devaki Chandramouli, Nokia |
| 810038 | Study on Security for 5GS Enhanced support of Vertical and LAN Services | FS_Vertical_LAN_SEC | S3 | SP-180697 | Jerichow, Anja (Nokia) |
| 840027 | Security for Vertical_LAN | Vertical_LAN_SEC | S3 | SP-190352 | Jerichow, Anja, Nokia |
| 830010 | CT aspects of Vertical_LAN | Vertical_LAN | ct | CP-191157 | WON, Sung Hwan, Nokia |
| 830035 | CT1 aspects of Vertical_LAN | Vertical_LAN | C1 | CP-191157 | WON, Sung Hwan, Nokia |
| 830036 | CT3 aspects of Vertical_LAN | Vertical_LAN | C3 | CP-191157 | WON, Sung Hwan, Nokia |
| 830037 | CT4 aspects of Vertical_LAN | Vertical_LAN | C4 | CP-191157 | WON, Sung Hwan, Nokia |
| 820027 | Service Enabler Architecture Layer for Verticals | SEAL | S6 | SP-181141 | Basavaraj (Basu) Pattan, Samsung |
| 820067 | NR-based access to unlicensed spectrum | NR_unlic | R1 | RP-191575 | Qualcomm |
| 750045 | Study on NR-based access to unlicensed spectrum | FS_NR_unlic | R1 | RP-181339 | Qualcomm |
| 820167 | Core part: NR-based access to unlicensed spectrum | NR_unlic-Core | R1 | RP-190706 | Qualcomm |
| 820267 | Perf. part: NR-based access to unlicensed spectrum | NR_unlic-Perf | R4 | RP-190706 | Qualcomm |
| 800006 | LAN support in 5G | 5GLAN | S1 | SP-180593 | Jose Almodovar, KPN |
| 760007 | Study on LAN Support in 5G | FS_5GLAN | S1 | SP-170456 | Nasielski, Jack, Qualcomm |
| 800047 | Stage 1 of 5GLAN | 5GLAN | S1 | SP-180593 | Jose Almodovar, KPN |
| 830024 | Study on non-public networks management | FS_OAM_NPN | S5 | SP-190137 | ZHANG, Kai, Huawei |
| 0 | Cellular IoT support and evolution | - |  | - | - |
| 830043 | Cellular IoT support and evolution for the 5G System | 5G_CIoT | S2 | SP-181118 | Sebastian Speicher |
| 770038 | Study on 5G_CIoT | FS_CIoT_5G | S2 | SP-180614 | Sebastian Speicher |
| 820015 | Stage 2 of 5G_CIoT | 5G_CIoT | S2 | SP-181118 | Sebastian Speicher |
| 830013 | CT aspects of 5G_CIoT | 5G_CIoT | ct | CP-111237 | Mahmoud Watfa; Qualcomm |
| 830038 | CT1 aspects of 5G_CIoT | 5G_CIoT | C1 | CP-111237 | Mahmoud Watfa; Qualcomm |
| 830039 | CT3 aspects of 5G_CIoT | 5G_CIoT | C3 | CP-111237 | Mahmoud Watfa; Qualcomm |
| 830040 | CT4 aspects of 5G_CIoT | 5G_CIoT | C4 | CP-111237 | Mahmoud Watfa; Qualcomm |
| 830041 | CT6 aspects of 5G_CIoT | 5G_CIoT | C6 | CP-111237 | Mahmoud Watfa; Qualcomm |
| 800033 | Study on evolution of Cellular IoT security for the 5G System | FS_CIoT_sec_5G | S3 | SP-180440 | Noamen Ben Henda |
| 760040 | Study on MBMS User Services for IoT | FS_MBMS_IoT | S4 | SP-170592 | Cedric thienot, Expway |
| 800083 | Additional MTC enhancements for LTE | LTE_eMTC5 | R1 | RP-191356 | Ericsson |
| 800183 | Core part: Additional MTC enhancements for LTE | LTE_eMTC5-Core | R1 | RP-190770 | Ericsson |
| 800283 | Perf. part: Additional MTC enhancements for LTE | LTE_eMTC5-Perf | R4 | RP-190770 | Ericsson |
| 800084 | Additional enhancements for NB-IoT | NB_IOTenh3 | R1 | RP-191576 | Huawei |
| 800184 | Core part: Additional enhancements for NB-IoT | NB_IOTenh3-Core | R1 | RP-190757 | Huawei |
| 800284 | Perf. part: Additional enhancements for NB-IoT | NB_IOTenh3-Perf | R4 | RP-190757 | Huawei |
| 810050 | 5G message service | 5GMSG | S1 | SP-180772 | Zheng, Jianping, China Mobile |
| 770004 | Study on 5G message service for MIoT | FS_5GMSG | S1 | SP-170704 | Zheng, Jianping, China Mobile |
| 810014 | Message Service Within the 5G System requirements | MSGin5G | S1 | SP-180920 | Zheng, Jianping, China Mobile |
| 0 | Advanced V2X support | - |  | - | - |
| 790002 | Study on Improvement of V2X Service Handling | FS_V2XIMP | S1 | SP-180247 | SungDuck Chun; LG Electronics |
| 820024 | Improvement of V2X service Handling | V2XIMP | S1 | SP-181013 | SungDuck Chun; LG Electronics |
| 840074 | Application layer support for V2X services | V2XAPP | S6 | SP-180898 | Niranth Amogh, Huawei Tel.India |
| 780025 | Study on V2XAPP | FS_V2XAPP | S6 | SP-171071 | Niranth Amogh, Huawei Tel.India |
| 810020 | Stage 2 of V2XAPP | V2XAPP | S6 | SP-180898 | Niranth Amogh, Huawei Tel.India |
| 840010 | CT aspects of V2XAPP | V2XAPP | ct | CP-191153 | Herrero Veron, Christian (Huawei) |
| 840075 | CT1 aspects of V2XAPP | V2XAPP | C1 | CP-191153 | Herrero Veron, Christian (Huawei) |
| 840076 | CT3 aspects of V2XAPP | V2XAPP | C3 | CP-191153 | Herrero Veron, Christian (Huawei) |
| 840077 | CT4 aspects of V2XAPP | V2XAPP | C4 | CP-191153 | Herrero Veron, Christian (Huawei) |
| 840078 | Architecture enhancements for 3GPP support of advanced V2X services | eV2XARC | S2 | SP-181121 | LaeYoung Kim, LG Electronics |
| 760043 | Study on eV2XARC | FS_eV2XARC | S2 | SP-180733 | LaeYoung Kim, LG Electronics |
| 820018 | Stage 2 of eV2XARC | eV2XARC | S2 | SP-181121 | LaeYoung Kim, LG Electronics |
| 840011 | CT aspects of eV2XARC | eV2XARC | ct | CP-191154 | Herrero Veron, Christian (Huawei) |
| 840079 | CT1 aspects of eV2XARC | eV2XARC | C1 | CP-191154 | Herrero Veron, Christian (Huawei) |
| 840080 | CT3 aspects of eV2XARC | eV2XARC | C3 | CP-191154 | Herrero Veron, Christian (Huawei) |
| 840081 | CT4 aspects of eV2XARC | eV2XARC | C4 | CP-191154 | Herrero Veron, Christian (Huawei) |
| 840082 | CT6 aspects of eV2XARC | eV2XARC | C6 | CP-191154 | Herrero Veron, Christian (Huawei) |
| 800096 | Study on NR Vehicle-to-Everything (V2X) | FS_NR_V2X | R1 | RP-190224 | LG Electronics |
| 770029 | Study on V2X Media Handling and Interaction | FS_mV2X | S4 | SP-170799 | Jung, Kyunghun, Samsung Electronics Co., Ltd |
| 830023 | Study on Security Aspects of 3GPP support for Advanced V2X Services | FS_eV2X_Sec | S3 | SP-190108 | Joonwoong Kim, LG Electronics |
| 830078 | 5G V2X with NR sidelink | 5G_V2X_NRSL | R1 | RP-190984 | LG Electronics |
| 830178 | Core part: 5G V2X with NR sidelink | 5G_V2X_NRSL-Core | R1 | RP-190766 | LG Electronics |
| 830278 | Perf. part: 5G V2X with NR sidelink | 5G_V2X_NRSL-Perf | R4 | RP-190766 | LG Electronics |
| 0 | 5G Location and Positionning Services | - |  | - | - |
| 800013 | 5G positioning services | 5G_HYPOS | S1 | SP-180329 | Ries, Lionel, ESA |
| 760006 | Study on positioning use cases | FS_5G_HYPOS | S1 | SP-170589 | Ries, Lionel, ESA |
| 800049 | Stage 1 of 5G_HYPOS | 5G_HYPOS | S1 | SP-180329 | Ries, Lionel, ESA |
| 830102 | Enhancement to the 5GC LoCation Services | 5G_eLCS | S2 | SP-181119 | Ming Ai, CATT |
| 780028 | Study on Enhancement to the 5GC Location Services | FS_eLCS | S2 | SP-180734 | Ming Ai, CATT |
| 810035 | Study on Security of the enhancement to the 5GC location services | FS_eLCS_Sec | S3 | SP-180694 | Wei Zhou, CATT |
| 820016 | Stage 2 of 5G_eLCS | 5G_eLCS | S2 | SP-181119 | Ming Ai, CATT |
| 830003 | CT aspects of 5G_eLCS | 5G_eLCS | ct | CP-190014 | Tingfang Tang, CATT |
| 830044 | CT1 aspects of 5G_eLCS | 5G_eLCS | C1 | CP-191064 | Tingfang Tang, CATT |
| 830045 | CT3 aspects of 5G_eLCS | 5G_eLCS | C3 | CP-191064 | Tingfang Tang, CATT |
| 830046 | CT4 aspects of 5G_eLCS | 5G_eLCS | C4 | CP-191064 | Tingfang Tang, CATT |
| 830077 | NR positioning support | NR_pos | R1 | RP-191156 | Intel |
| 800092 | Study on NR positioning support | FS_NR_pos | R1 | RP-181399 | Intel |
| 830073 | Study on local NR positioning in NG-RAN | FS_NR_local_pos | R3 | RP-190754 | CMCC |
| 830177 | Core part: NR positioning support | NR_pos-Core | R1 | RP-190752 | Intel |
| 830277 | Perf. part: NR positioning support | NR_pos-Perf | R4 | RP-190752 | Intel |
| 0 | UE radio capability signalling optimization | - |  | - | - |
| 840054 | Optimisations on UE radio capability signalling | RACS | S2 | RP-191460 | Haris Zisimopoulos |
| 800025 | Study on RACS | FS_RACS | S2 | SP-180599 | Haris Zisimopoulos, Qualcomm |
| 830027 | Stage 2 of RACS | RACS | S2 | SP-190180 | Haris Zisimopoulos |
| 840003 | CT aspects of RACS | RACS | ct | CP-191061 | Chaponniere, Lena, Qualcomm |
| 840055 | CT1 aspects of RACS | RACS | C1 | CP-191061 | Chaponniere, Lena, Qualcomm |
| 840056 | CT3 aspects of RACS | RACS | C3 | CP-191061 | Chaponniere, Lena, Qualcomm |
| 840057 | CT4 aspects of RACS | RACS | C4 | CP-191061 | Chaponniere, Lena, Qualcomm |
| 830079 | Optimisations on UE radio capability signalling – NR/E-UTRA Aspects | RACS-RAN | R2 | RP-190657 | MediaTek |
| 800097 | Study on optimisations on UE radio capability signalling – NR/E-UTRA Aspects | FS_RACS_RAN | R2 | RP-181459 | MediaTek |
| 830179 | Core part: Optimisations on UE radio capability signalling – NR/E-UTRA Aspects | RACS-RAN-Core | R2 | RP-191088 | MediaTek |
| 0 | Satellite Access in 5G | - |  | - | - |
| 800010 | Integration of Satellite Access in 5G | 5GSAT | S1 | SP-180326 | MICHEL, Cyril, Thales |
| 770002 | Study on using Satellite Access in 5G | FS_5GSAT | S1 | SP-170702 | MICHEL, Cyril |
| 800048 | Stage 1 of 5GSAT | 5GSAT | S1 | SP-180326 | MICHEL, Cyril, Thales |
| 800026 | Study on architecture aspects for using satellite access in 5G | FS_5GSAT_ARCH | S2 | SP-181253 | MICHEL, Cyril |
| 800099 | Study on solutions for NR to support non-terrestrial networks (NTN) | FS_NR_NTN_solutions | R3 | RP-190710 | Thales |
| 830025 | Study on management and orchestration aspects with integrated satellite components in a 5G network | FS_5GSAT_MO | S5 | SP-190138 | MICHEL, Cyril, THALES |
| 0 | Enablers for Network Automation Architecture for 5G | - |  | - | - |
| 830047 | Enablers for Network Automation for 5G | eNA | S2 | SP-181123 | Xiaobo Wu, Huawei Technologies |
| 760047 | Study of enablers for Network Automation for 5G | FS_eNA | S2 | SP-180792 | Xiaobo Wu, Huawei Technologies |
| 820020 | Stage 2 of eNA | eNA | S2 | SP-181123 | Xiaobo Wu, Huawei Technologies |
| 830009 | CT aspects of eNA | eNA | ct | CP-191111 | Yali Yan, Huawei |
| 830048 | CT3 aspects of eNA | eNA | C3 | CP-191111 | Yali Yan, Huawei |
| 830049 | CT4 aspects of eNA | eNA | C4 | CP-191111 | Yali Yan, Huawei |
| 0 | Wireless and Wireline Convergence Enhancement | - |  | - | - |
| 760044 | Study on the Wireless and Wireline Convergence for the 5G system architecture | FS_5WWC | S2 | SP-170380 | Marco Spini, Huawei Technologies |
| 800031 | Study on the security of the Wireless and Wireline Convergence for the 5G system architecture | FS_5WWC_SEC | S3 | SP-180438 | He Li, Huawei Technologies |
| 830050 | Wireless and Wireline Convergence for the 5G system architecture | 5WWC |  | SP-181117 | Marco Spini, Huawei Technologies |
| 820014 | Stage 2 of 5WWC | 5WWC | S2 | SP-181117 | Marco Spini, Huawei Technologies |
| 829910 | CT aspects of 5WWC | 5WWC | ct | CP-191156 | Herrero Veron, Christian (Huawei) |
| 829911 | CT1 aspects of 5WWC | 5WWC | C1 | CP-191156 | Herrero Veron, Christian (Huawei) |
| 829912 | CT3 aspects of 5WWC | 5WWC | C3 | CP-191156 | Herrero Veron, Christian (Huawei) |
| 829914 | CT4 aspects of 5WWC | 5WWC | C4 | CP-191156 | Herrero Veron, Christian (Huawei) |
| 830082 | NG interface usage for WWC | 5WWC-NG_interface | R3 | RP-190772 | Huawei |
| 830182 | Core part: NG interface usage for WWC | 5WWC-NG_interface-Core | R3 | RP-190999 | Huawei |
| 0 | Mission Critical, Public Warning, Railways and Maritime | - |  | - | - |
| 790024 | Study on MC services access aspects | FS_MCSAA | S6 | SP-181135 | Joakim Åkesson, Ericsson |
| 800024 | Study into discreet listening and logging for mission critical services | FS_MCLOG | S6 | SP-181138 | Chater-Lea, David, Motorola Solutions |
| 810055 | Study on location enhancements for mission critical services | FS_enhMCLoc | S6 | SP-180921 | Dom Lazara, Motorola Solutions |
| 800022 | Enhanced Mission Critical Push-to-talk architecture phase 2 | enh2MCPTT | S6 | SP-190068 | Dom Lazara; Motorola Solutions |
| 830051 | Enhancements to Functional architecture and information flows for Mission Critical Data | eMCData2 | S6 | SP-180378 | Shih, Jerry, AT&T |
| 800018 | Stage 2 of eMCData2 | eMCData2 | S6 | SP-180378 | Shih, Jerry, AT&T |
| 830014 | CT aspects of eMCData2 | eMCData2 | C1 | CP-190199 | Val Oprescu (AT&T) |
| 800023 | Study on Mission Critical services support over 5G System | FS_MCOver5GS | S6 | SP-181136 | Verweij, Kees, The Police of the Netherlands |
| 810004 | MCData File Distribution support over xMB | MC_XMB | S4 | SP-180665 | Thiénot Cédric, Expway |
| 810011 | Protocol enhancements for Mission Critical Services | MCProtoc16 | C1 | CP-182110 | AXELL, Jörgen |
| 800053 | MBMS APIs for Mission Critical Services | MBMSAPI_MCS | S6 | SP-180380 | Ling Zhang, TD Tech Ltd. |
| 760051 | Study on MBMS APIs for MC Services | FS_MBMSAPI_MC | S6 | SP-180237 | Ling Zhang, TD Tech Ltd |
| 800020 | Stage 2 of MBMS APIs for MC Services | MBMSAPI_MCS | S6 | SP-180380 | Ling Zhang, TD Tech Ltd. |
| 840013 | Enhancement of 3GPP Northbound APIs | eNAPIs | C3 | CP-191233 | Yali Yan, Huawei |
| 800019 | Enhanced Mission Critical System Migration and Interconnection | eMCSMI | S6 | SP-180379 | Chater-Lea, David; Motorola Solutions |
| 810054 | Enhanced Mission Critical Communication Interworking with Land Mobile Radio Systems | eMCCI |  |  | Monnes, Peter, Harris Corporation |
| 800021 | Enhanced Mission Critical Communication Interworking with Land Mobile Radio Systems | eMCCI | S6 | SP-180680 | Monnes, Peter, Harris Corporation |
| 800016 | Stage 3 for MC Communication Interworking with Land Mobile Radio Systems | MCCI_CT | C1 | CP-190203 | Monnes, Peter, Harris Corporation |
| 820040 | Mission Critical system migration and interconnection | MCSMI_CT | C1 | CP-190143 | Dom Lazara, Motorola Solutions |
| 800032 | Mission Critical Services Security Enhancements | MCXSec | S3 | SP-180596 | Woodward, Tim, Motorola Solutions, Inc |
| 780003 | Enhancements of Public Warning System | ePWS |  | SP-170998 | Hyounhee Koo, SyncTechno Inc. |
| 730005 | Study on enhancements of Public Warning System | FS_ePWS | S1 | SP-160733 | SyncTechno Inc., Hyounhee Koo |
| 800052 | Stage 1 of ePWS | ePWS | S1 | SP-170998 | Hyounhee Koo, SyncTechno Inc. |
| 810012 | CT aspects of enhancements of Public Warning System | ePWS | ct | CP-191155 | TaeHyeon Kim, SyncTechno Inc. |
| 810053 | Study on stages 2 and 3 of enhancements of ePWS | ePWS | C1 | CP-191155 | TaeHyeon Kim, SyncTechno Inc. |
| 810047 | CT1 aspects of ePWS | ePWS | C1 | CP-191155 | TaeHyeon Kim, SyncTechno Inc. |
| 810048 | CT4 aspects of ePWS (Possible impacts) | ePWS | C4 | CP-191155 | TaeHyeon Kim, SyncTechno Inc. |
| 760054 | Mobile Communication System for Railways 2 | MONASTERY2 |  | SP-170451 | Merkel, Juergen, Nokia |
| 760005 | Study on MONASTERY2 | FS_FRMCS2 | S1 | SP-170588 | Merkel, Juergen, Nokia |
| 790023 | Study on application architecture for the Future Railway Mobile Communication System (FRMCS) Phase 2 | FS_FRMCS2_SA6 | S6 | SP-181133 | Oettl, Martin, Nokia |
| 760004 | Stage 1 of MONASTERY2 | MONASTERY2 | S1 | SP-170451 | Merkel, Juergen, Nokia |
| 830007 | Application Architecture for MONASTERY2 | MONASTERY2 | S6 | SP-190064 | Oettl, Martin, Nokia |
| 840009 | CT aspects of MONASTERY2 | MONASTERY2 | ct | CP-191152 | Gkatzikis, Lazaros (Nokia) |
| 840072 | CT1 aspects of MONASTERY2 | MONASTERY2 | C1 | CP-191152 | Gkatzikis, Lazaros (Nokia) |
| 840073 | CT3 aspects of MONASTERY2 | MONASTERY2 | C3 | CP-191152 | Gkatzikis, Lazaros (Nokia) |
| 840092 | NR support for high speed train scenario | NR_HST | R4 | RP-191512 | CMCC |
| 840192 | Core part: NR support for high speed train scenario | NR_HST-Core | R4 | RP-191512 | CMCC |
| 840292 | Perf. part: NR support for high speed train scenario | NR_HST-Perf | R4 | RP-191512 | CMCC |
| 800011 | Maritime Communication Services over 3GPP System | MARCOM | S1 | SP-180594 | Hyounhee Koo, SyncTechno Inc |
| 730007 | Study on Maritime Communication Services over 3GPP system | FS_MARCOM | S1 | SP-170453 | SyncTechno Inc., Hyounhee Koo |
| 800051 | Stage 1 of MARCOM | MARCOM | S1 | SP-180594 | Hyounhee Koo, SyncTechno Inc |
| 0 | Streaming and TV | - |  | - | - |
| 800008 | Enhancement of LTE for Efficient delivery of Streaming Service | eLSTR | S1 | SP-180322 | Xia, Xu, China Telecom |
| 740001 | Study on eLSTR | FS_eLESTR | S1 | SP-160960 | China Telecommunications, XIA Xu |
| 800054 | Stage 1 of eLSTR | eLSTR | S1 | SP-180322 | Xia, Xu, China Telecom |
| 800001 | Enhancements to Framework for Live Uplink Streaming | E_FLUS | S4 | SP-180285 | Lo, Charles, Qualcomm |
| 810003 | Coverage and Handoff Enhancements for Multimedia | CHEM | S4 | SP-180664 | Nikolai Leung |
| 770027 | Study on QoE metrics for VR | FS_QoE_VR | S4 | SP-170614 | Ericsson (Gunnar Heikkilä |
| 760039 | Study on 5G enhanced Mobile Broadband Media Distribution | FS_5GMedia_Distribution | S4 | SP-170334 | Bouazizi, Imed, Samsung Electronics Co., Ltd., |
| 820002 | Media streaming architecture | 5GMSA | S4 | SP-180984 | Frédéric GABIN |
| 810040 | Media Handling Extensions for 5G Conversational Services | 5G_MEDIA_MTSI_ext | S4 | SP-180663 | Oyman, Ozgur, Company: Intel |
| 760041 | Study on Media Handling Aspects of Conversational Services in 5G Systems | FS_5G_MEDIA_MTSI | S4 | SP-170336 | Oyman, Ozgur, Company: Intel |
| 810002 | Media Handling Extensions for 5G Conversational Services | 5G_MEDIA_MTSI_ext | S4 | SP-180663 | Oyman, Ozgur, Company: Intel |
| 770031 | Study on EVS Float Conformance Non Bit-Exact | FS_EVS_FCNBE | S4 | SP-170618 | Fabrice Plante, Intel |
| 820001 | EVS Floating-point Conformance for Non Bit-Exact | EVS_FCNBE | S4 | SP-180983 | Fabrice, Plante, Intel |
| 800003 | Addition of HLG HDR to TV Video Profiles | HLG_HDR | S4 | SP-180287 | Paul Szucs, Sony Europe Limited |
| 0 | User Identities, Authentication, multi-device | - |  | - | - |
| 800012 | User Identities and Authentication | UIA | S1 | SP-180328 | Bischinger, Kurt; Deutsche Telekom |
| 780004 | Study on a Layer for User Centric Identifiers and Authentication | FS_LUCIA | S1 | SP-170995 | Bischinger, Kurt; Deutsche Telekom |
| 800050 | Stage 1 of User Identities and Authentication | UIA | S1 | SP-180328 | Bischinger, Kurt; Deutsche Telekom |
| 810044 | Multi-device and multi-identity | MuD | S1 | SP-180315 | Peter Bleckert, Ericsson |
| 800004 | Stage 1 of Multi-device and multi-identity | MuD | S1 | SP-180315 | Peter Bleckert, Ericsson |
| 810010 | CT aspects of Multi-device and multi-identity | MuD | ct | CP-182227 | Axell, Jörgen, Ericsson |
| 810045 | CT1 aspects of Multi-device and multi-identity | MuD | C1 | CP-182227 | Axell, Jörgen, Ericsson |
| 810046 | CT3 aspects of Multi-device and multi-identity | MuD | C3 | CP-182227 | Axell, Jörgen, Ericsson |
| 0 | Slicing | - |  | - | - |
| 830103 | Enhancement of Network Slicing | eNS |  | SP-181232 | ZTE, Tricci So |
| 790011 | Study on Enhancement of Network Slicing | FS_eNS | S2 | SP-180503 | ZTE, Tricci So |
| 820023 | Stage 2 of eNS | eNS | S2 | SP-181232 | ZTE, Tricci So |
| 830011 | CT aspects of eNS | eNS | ct | CP-190196 | Fei Lu, ZTE |
| 830052 | CT1 aspects of eNS | eNS | C1 | CP-190196 | Fei Lu, ZTE |
| 830053 | CT3 aspects of eNS | eNS | C3 | CP-190196 | Fei Lu, ZTE |
| 830054 | CT4 aspects of eNS | eNS | C4 | CP-190196 | Fei Lu, ZTE |
| 810051 | Business Role Models for Network Slicing | BRMNS | S1 | SP-180773 | Covell, Betsy, Nokia |
| 770005 | Study on Business Role Models for Network Slicing | FS_BMNS | S1 | SP-180169 | Covell, Betsy, Nokia |
| 810015 | Business Role Models for Network Slicing | BRMNS | S1 | SP-180773 | Covell, Betsy, Nokia |
| 810034 | Study on Security aspects of Enhancement of Network Slicing | FS_eNS_SEC | S3 | SP-180692 |  |
| 810022 | Study on tenancy concept in 5G networks and network slicing management | FS_TENANCYC | S5 | SP-180815 | ZHU, Lei (Huawei |
| 810031 | Enhancement of performance assurance for 5G networks including network slicing | 5G_SLICE_ePA | S5 | SP-190247 | Xiaowei Sun (China Mobile) |
| 820029 | Study on Charging Aspects of Network Slicing | FS_NETSLICE_CH | S5 | SP-181065 | Chen Shan, Huawei |
| 0 | Other cross-TSG Release 16 Features | - |  | - | - |
| 760003 | Provision of Access to Restricted Local Operator Services by Unauthenticated UEs | PARLOS |  | SP-170449 | Covell, Betsy Nokia |
| 740002 | Study on Stage 1 of PARLOS | FS_PARLOS | S1 | SP-160904 | Covell, Betsy |
| 760046 | Study on Stage 2 for PARLOS | FS_PARLOS_SA2 | S2 | SP-180501 | Nokia (Nicolas Drevon |
| 800035 | Study on Security Aspects of PARLOS | FS_PARLOS_Sec | S3 | SP-180442 | Greg Schumacher, Sprint |
| 760071 | Stage 1 of PARLOS | PARLOS | S1 | SP-170449 | Covell, Betsy Nokia |
| 810008 | Stage 2 of PARLOS | PARLOS | S2 | SP-180738 | Nokia (Nicolas Drevon |
| 830012 | CT aspects of PARLOS | PARLOS | ct | CP-190197 | Liu, Jennifer; Nokia |
| 830062 | CT1 aspects of PARLOS | PARLOS | C1 | CP-190197 | Liu, Jennifer; Nokia |
| 830063 | CT3 aspects of PARLOS | PARLOS | C3 | CP-190197 | Liu, Jennifer; Nokia |
| 830064 | CT4 aspects of PARLOS | PARLOS | C4 | CP-190197 | Liu, Jennifer; Nokia |
| 830065 | CT6 aspects of PARLOS | PARLOS | C6 | CP-190197 | Liu, Jennifer; Nokia |
| 820043 | Enhancing Topology of SMF and UPF in 5G Networks | ETSUN |  | SP-181116 | Laurent Thiebaut (Nokia) |
| 770039 | Study on ETSUN | FS_ETSUN | S2 | SP-180731 | Laurent Thiebaut (Nokia) |
| 820013 | Stage 2 of ETSUN | ETSUN | S2 | SP-181116 |  |
| 830002 | CT aspects of ETSUN | ETSUN | ct | CP-190192 | Landais, Bruno, Nokia |
| 830055 | CT3 aspects of ETSUN | ETSUN | C3 | CP-190192 | Landais, Bruno, Nokia |
| 830056 | CT4 aspects of ETSUN | ETSUN | C4 | CP-190192 | Landais, Bruno, Nokia |
| 820044 | Access Traffic Steering, Switch and Splitting support in the 5G system architecture | ATSSS |  | SP-181124 | So, Tricci, ZTE |
| 760052 | Study on ATSSS | FS_ATSSS | S2 | SP-180732 | So, Tricci, ZTE |
| 820021 | Stage 2 of ATSSS | ATSSS | S2 | SP-181124 | So, Tricci, ZTE |
| 830016 | CT aspects of ATSSS | ATSSS | ct | CP-190201 | ZHOU, Xingyue (Joy), ZTE |
| 830057 | CT1 aspects of ATSSS | ATSSS | C1 | CP-190201 | ZHOU, Xingyue (Joy), ZTE |
| 830058 | CT3 aspects of ATSSS | ATSSS | C3 | CP-190201 | ZHOU, Xingyue (Joy), ZTE |
| 830059 | CT4 aspects of ATSSS | ATSSS | C4 | CP-190201 | ZHOU, Xingyue (Joy), ZTE |
| 840999 | (IETF) TCP Extensions for Multipath Operation with Multiple Addresses (draft-ietf-mptcp-rfc6824bis) | ATSSS | S2-IETF |  |  |
| 840998 | (IETF) 0-RTT TCP Convert Protocol (draft-ietf-tcpm-converters) | ATSSS | S2-IETF |  |  |
| 810041 | Single radio voice continuity from 5GS to 3G | 5G_SRVCC |  | SP-180737 | Chi Ren, China Unicom |
| 790010 | Study for single radio voice continuity from 5GS to 3G | FS_5G-SRVCC | S2 | SP-180239 | Chi Ren, China Unicom |
| 810007 | Single radio voice continuity from 5GS to 3G | 5G_SRVCC | S2 | SP-180897 | Chi Ren, China Unicom |
| 820008 | Security aspects of single radio voice continuity from 5GS to UTRAN | 5GS_UTRAN_SEC | S3 | SP-181037 | Feng Gao, China Unicom |
| 820069 | RAN aspects of 5G_SRVCC | SRVCC_NR_to_UMTS | R2 | RP-190713 | China Unicom |
| 820169 | Core part: Single Radio Voice Call Continuity from 5G to 3G | SRVCC_NR_to_UMTS-Core | R2 | RP-190713 | China Unicom |
| 830270 | Perf. part: Single Radio Voice Call Continuity from 5G to 3G | SRVCC_NR_to_UMTS-Perf | R4 | RP-190713 | China Unicom |
| 840004 | CT aspect of 5G_SRVCC | 5G_SRVCC | ct | CP-191062 | Chi Ren, China Unicom |
| 840058 | CT1 aspect of 5G_SRVCC | 5G_SRVCC | C1 | CP-191062 | Chi Ren, China Unicom |
| 840059 | CT4 aspect of 5G_SRVCC | 5G_SRVCC | C4 | CP-191062 | Chi Ren, China Unicom |
| 810049 | Remote Identification of Unmanned Aerial Systems | ID_UAS |  | SP-180771 | Eddy HALL, Qualcomm |
| 790005 | Study on Remote Identification of Unmanned Aerial Systems | FS_ID_UAS | S1 | SP-180172 | Eddy Hall; Qualcomm |
| 810013 | Stage 1 of Remote Identification of Unmanned Aerial Systems | ID_UAS | S1 | SP-180771 | Eddy HALL, Qualcomm |
| 800034 | Study on Security Aspects of the 5G Service Based Architecture | FS_SBA_Sec | S3 | SP-180598 | Hans Christian Rudolph, Deutsche Telekom AG |
| 820045 | Enhancements to the Service-Based 5G System Architecture | 5G_eSBA | S2 | SP-181125 |  |
| 790007 | Study on Enhancements to the Service-Based 5G System Architecture | FS_eSBA | S2 | SP-180231 | Tao Sun (China Mobile) |
| 820022 | Stage 2 of 5G_eSBA | 5G_eSBA | S2 | SP-181125 | Tao Sun (China Mobile) |
| 830001 | CT aspects of 5G_eSBA | 5G_eSBA | ct | CP-190191 | Song Yue (China Mobile) |
| 830060 | CT3 aspects of 5G_eSBA | 5G_eSBA | C3 | CP-190191 | Song Yue (China Mobile) |
| 830061 | CT4 aspects of 5G_eSBA | 5G_eSBA | C4 | CP-190191 | Song Yue (China Mobile) |
| 780029 | Study on Enhanced IMS to 5GC Integration | FS_eIMS5G | S2 | SP-180736 | Joul, Chris, T-Mobile USA |
| 840062 | SBA aspects of enhanced IMS to 5GC integration | eIMS5G_SBA | S2 | SP-190181 | T-Mobile USA, Christopher Joul |
| 830028 | Stage 2 of eIMS5G_SBA | eIMS5G_SBA | S2 | SP-190181 | T-Mobile USA, Christopher Joul |
| 840006 | CT aspects of eIMS5G_SBA | eIMS5G_SBA | ct | CP-191065 | de Gregorio, Jesus. Ericsson |
| 840063 | CT3 aspects of eIMS5G_SBA | eIMS5G_SBA | C3 | CP-191065 | de Gregorio, Jesus. Ericsson |
| 840064 | CT4 aspects of eIMS5G_SBA | eIMS5G_SBA | C4 | CP-191065 | de Gregorio, Jesus. Ericsson |
| 830081 | Private Network Support for NG-RAN | NG_RAN_PRN | R3 | RP-190729 | China Telecom |
| 830181 | Core part: Private Network Support for NG-RAN | NG_RAN_PRN-Core | R3 | RP-191563 | China Telecom |
| 810039 | Media Handling Aspects of RAN Delay Budget Reporting in MTSI | E2E_DELAY | S4 | SP-180662 | Oyman, Ozgur, Company: Intel |
| 780022 | Study on E2E_DELAY | FS_E2E_DELAY | S4 | SP-170837 | Oyman, Ozgur, Company: Intel, |
| 810001 | Stage 2 of E2E_DELAY | E2E_DELAY | S4 | SP-180662 | Oyman, Ozgur, Company: Intel |
| 830004 | CT Aspects of E2E_DELAY | E2E_DELAY | ct | CP-190193 | Luetzenkirchen, Thomas, Intel |
| 830066 | CT1 Aspects of E2E_DELAY | E2E_DELAY | C1 | CP-190193 | Luetzenkirchen, Thomas, Intel |
| 830067 | CT3 Aspects of E2E_DELAY | E2E_DELAY | C3 | CP-190193 | Luetzenkirchen, Thomas, Intel |
| 830068 | CT4 Aspects of E2E_DELAY | E2E_DELAY | C4 | CP-190193 | Luetzenkirchen, Thomas, Intel |
| 830069 | Enhancements for Common API Framework for 3GPP Northbound APIs | eCAPIF |  | SP-181137 | Basavaraj Pattan, Samsung |
| 790022 | Stage 2 for eCAPIF | eCAPIF | S6 | SP-181137 | Basavaraj Pattan, Samsung |
| 830022 | Security aspects of eCAPIF | eCAPIF | S3 | SP-1901240 | Rajavelsamy Rajadurai, Samsung |
| 790019 | Usage of CAPIF for xMB API | CAPIF4xMB | S4 | SP-180031 | Thorsten Lohmar, Ericsson LM |
| 840065 | User data interworking, Coexistence and Migration | UDICOM | S2 | SP-190182 | Alessio Casati, Nokia |
| 800055 | Study on UDICOM | FS_UDICoM | S2 | SP-190148 | Susana Sabater (Vodafone |
| 830029 | Stage 2 of UDICOM | UDICOM | S2 | SP-190182 | Alessio Casati, Nokia |
| 840007 | CT aspects of UDICOM | UDICOM | C4 | CP-191066 | Wiehe, Ulrich, Nokia |
| 0 | NR-related Release 16 Features | - |  | - | - |
| 830075 | UE Power Saving in NR | NR_UE_pow_sav | R1 | RP-191607 | CATT |
| 800094 | Study on UE power saving in NR | FS_NR_UE_pow_sav | R1 | RP-181463 | CATT |
| 830175 | Core part: UE Power Saving in NR | NR_UE_pow_sav-Core | R1 | RP-190727 | CATT |
| 830275 | Perf. part: UE Power Saving in NR | NR_UE_pow_sav-Perf | R4 | RP-190727 | CATT |
| 820070 | Integrated access and backhaul for NR | NR_IAB |  | RP-191558 | Qualcomm |
| 750047 | Study onNR_IAB | FS_NR_IAB | R2 | RP-181349 | Qualcomm |
| 820170 | Core part: NR_IAB | NR_IAB-Core | R2 | RP-190712 | Qualcomm |
| 820270 | Perf. Part: NR_IAB | NR_IAB-Perf | R4 | RP-190712 | Qualcomm |
| 830021 | Study on Security for NR_IAB | FS_NR_IAB_Sec | S3 | SP-190106 | Rajavelsamy Rajadurai, Samsung, |
| 800072 | Band combinations for SA NR supplementary uplink (SUL), NSA NR SUL, NSA NR SUL with UL sharing from the UE perspective (ULSUP) | NR_SUL_combos_R16 | R4 | RP-191126 | Huawei |
| 800172 | Core part: Band combinations for SA NR supplementary uplink (SUL), NSA NR SUL, NSA NR SUL with UL sharing from the UE perspective (ULSUP) | NR_SUL_combos_R16-Core | R4 | RP-190714 | Huawei |
| 800272 | Perf. part: Band combinations for SA NR supplementary uplink (SUL), NSA NR SUL, NSA NR SUL with UL sharing from the UE perspective (ULSUP) | NR_SUL_combos_R16-Perf | R4 | RP-190714 | Huawei |
| 800057 | NR intra-band Carrier Aggregation |  |  |  |  |
| 800073 | Rel-16 NR intra band CA for xCC DL/yCC UL including contiguous and non-contiguous spectrum (x>=y) | NR_CA_R16_intra | R4 | RP-191194 | Ericsson |
| 800173 | Core part: Rel-16 NR intra band CA for xCC DL/yCC UL including contiguous and non-contiguous spectrum (x>=y) | NR_CA_R16_intra-Core | R4 | RP-190689 | Ericsson |
| 800273 | Perf. part: Rel-16 NR intra band CA for xCC DL/yCC UL including contiguous and non-contiguous spectrum (x>=y) | NR_CA_R16_intra-Perf | R4 | RP-190689 | Ericsson |
| 800074 | Rel-16 NR inter-band CA/Dual Connectivity for 2 bands DL with x bands UL (x=1, 2) | NR_CADC_R16_2BDL_xBUL | R4 | RP-191565 | ZTE |
| 800174 | Core part: Rel-16 NR inter-band CA/Dual Connectivity for 2 bands DL with x bands UL (x=1, 2) | NR_CADC_R16_2BDL_xBUL-Core | R4 | RP-190494 | ZTE |
| 800274 | Perf. part: Rel-16 NR inter-band CA/Dual Connectivity for 2 bands DL with x bands UL (x=1, 2) | NR_CADC_R16_2BDL_xBUL-Perf | R4 | RP-190494 | ZTE |
| 830099 | NR inter-band Carrier Aggregation |  |  |  |  |
| 830083 | UE Conformance Test Aspects - Rel-16 NR CA and DC; and NR and LTE DC Configurations | NR_CADC_NR_LTE_DC_R16-UEConTest | R5 | RP-190321 | CMCC |
| 830095 | Rel16 NR inter-band Carrier Aggregation for 3 bands DL with 1 band UL | NR_CA_R16_3BDL_1BUL | R4 | RP-191153 | CATT |
| 830195 | Core part: Rel16 NR inter-band Carrier Aggregation for 3 bands DL with 1 band UL | NR_CA_R16_3BDL_1BUL-Core | R4 | RP-190692 | CATT |
| 830295 | Perf. part: Rel16 NR inter-band Carrier Aggregation for 3 bands DL with 1 band UL | NR_CA_R16_3BDL_1BUL-Perf | R4 | RP-190692 | CATT |
| 830096 | Rel-16 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with 2 bands UL | NR_CADC_R16_3BDL_2BUL | R4 | RP-191566 | ZTE |
| 830196 | Core part: Rel-16 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with 2 bands UL | NR_CADC_R16_3BDL_2BUL-Core | R4 | RP-190693 | ZTE |
| 830296 | Perf. part: Rel-16 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with 2 bands UL | NR_CADC_R16_3BDL_2BUL-Perf | R4 | RP-190693 | ZTE |
| 831000 | New bands for New Radio |  | R4 |  | Dish Network |
| 800077 | Band 65 for New Radio | NR_band_n65 | R4 | RP-190360 | Dish Network |
| 800177 | Core part: Band 65 for New Radio | NR_band_n65-Core | R4 | RP-190360 | Dish Network |
| 800277 | Perf. part: Band 65 for New Radio | NR_band_n65-Perf | R4 | RP-190360 | Dish Network |
| 820073 | Introduction of NR band n18 | NR_n18 | R4 | RP-191556 | KDDI |
| 820173 | Core part: Introduction of NR band n18 | NR_n18-Core | R4 | RP-190173 | KDDI |
| 820273 | Perf. part: Introduction of NR band n18 | NR_n18-Perf | R4 | RP-190173 | KDDI |
| 820077 | Introduction of NR band n48 | NR_n48 | R4 | RP-190908 | US Cellular |
| 820177 | Core part: Introduction of NR band n48 | NR_n48-Core | R4 | RP-182865 | US Cellular |
| 830271 | Perf. part: Introduction of NR band n48 | NR_n48-Perf | R4 | RP-190140 | Nokia |
| 830090 | Introduction of NR band n14 | NR_n14 | R4 | RP-190965 | AT&T |
| 830190 | Core part: Introduction of NR band n14 | NR_n14-Core | R4 | RP-190165 | AT&T |
| 830290 | Perf. part: Introduction of NR band n14 | NR_n14-Perf | R4 | RP-190165 | AT&T |
| 830091 | Introduction of NR band n30 | NR_n30 | R4 | RP-190967 | AT&T |
| 830191 | Core part: Introduction of NR band n30 | NR_n30-Core | R4 | RP-190166 | AT&T |
| 830291 | Perf. part: Introduction of NR band n30 | NR_n30-Perf | R4 | RP-190166 | AT&T |
| 830097 | Introduction of NR band n259 | NR_n259 | R4 | RP-191416 | Ericsson |
| 830197 | Core part: Introduction of NR band n259 | NR_n259-Core | R4 | RP-190765 | Ericsson |
| 830297 | Perf. part: Introduction of NR band n259 | NR_n259-Perf | R4 | RP-190765 | Ericsson |
| 840097 | Introduction of NR supplementary DL band n29 | NR_n29 | R4 | RP-191474 | Dish Network |
| 840197 | Core part: Introduction of NR supplementary DL band n29 | NR_n29-Core | R4 | RP-191474 | Dish Network |
| 840297 | Perf. part: Introduction of NR supplementary DL band n29 | NR_n29-Perf | R4 | RP-191474 | Dish Network |
| 831001 | New Radio Band-related topics |  | R4 |  | Nokia |
| 830087 | RF requirements for NR frequency range 1 (FR1) | NR_RF_FR1 | R4 | RP-191114 | Huawei |
| 830187 | Core part: RF requirements for NR frequency range 1 (FR1) | NR_RF_FR1-Core | R4 | RP-191114 | Huawei |
| 830287 | Perf. part: RF requirements for NR frequency range 1 (FR1) | NR_RF_FR1-Perf | R4 | RP-191114 | Huawei |
| 830088 | Add support of NR DL 256QAM for frequency range 2 (FR2) | NR_DL256QAM_FR2 | R4 | RP-190925 | China Telecom |
| 830188 | Core part: Add support of NR DL 256QAM for frequency range 2 (FR2) | NR_DL256QAM_FR2-Core | R4 | RP-190760 | China Telecom |
| 830288 | Perf. part: Add support of NR DL 256QAM for frequency range 2 (FR2) | NR_DL256QAM_FR2-Perf | R4 | RP-190760 | China Telecom |
| 830089 | NR RF requirement enhancements for frequency range 2 (FR2) | NR_RF_FR2_req_enh | R4 | RP-191290 | Nokia |
| 830189 | Core part: NR RF requirement enhancements for frequency range 2 (FR2) | NR_RF_FR2_req_enh-Core | R4 | RP-190761 | Nokia |
| 830092 | Introduction of an NR SUL (supplemental uplink) band with same uplink frequency range as NR band n5 | NR_SUL_UL_n5 | R4 | RP-191116 | Huawei |
| 830192 | Core part: Introduction of an NR SUL (supplemental uplink) band with same uplink frequency range as NR band n5 | NR_SUL_UL_n5-Core | R4 | RP-191116 | Huawei |
| 830292 | Perf. part: Introduction of an NR SUL (supplemental uplink) band with same uplink frequency range as NR band n5 | NR_SUL_UL_n5-Perf | R4 | RP-191116 | Huawei |
| 820074 | Adding wider channel bandwidths in NR band n7 | NR_n7_BW | R4 | RP-190137 | BT |
| 820174 | Core part: Adding wider channel bandwidths in NR band n7 | NR_n7_BW-Core | R4 | RP-190137 | BT |
| 830093 | Addition of wider channel bandwidth in NR band n38 | NR_n38_BW | R4 | RP-191472 | Huawei |
| 830193 | Core part: Addition of wider channel bandwidth in NR band n38 | NR_n38_BW-Core | R4 | RP-190392 | Huawei |
| 830293 | Perf. part: Addition of wider channel bandwidth in NR band n38 | NR_n38_BW-Perf | R4 | RP-190392 | Huawei |
| 820075 | Addition of 30MHz channel bandwidth for NR band n41 | NR_n41_BW | R4 | RP-191569 | KDDI |
| 820175 | Core part: Addition of 30MHz channel bandwidth for NR band n41 | NR_n41_BW-Core | R4 | RP-190174 | KDDI |
| 820275 | Perf. part: Addition of 30MHz channel bandwidth for NR band n41 | NR_n41_BW-Perf | R4 | RP-190174 | KDDI |
| 820076 | Adding wider channel bandwidths in NR band n50 | NR_n50_BW | R4 | RP-182829 | Huawei |
| 820176 | Core part: Adding wider channel bandwidths in NR band n50 | NR_n50_BW-Core | R4 | RP-182829 | Huawei |
| 800085 | Enhancements on MIMO for NR | NR_eMIMO | R1 | RP-182863 | Samsung |
| 800185 | Core part: Enhancements on MIMO for NR | NR_eMIMO-Core | R1 | RP-182863 | Samsung |
| 800285 | Perf. part: Enhancements on MIMO for NR | NR_eMIMO-Perf | R4 | RP-182863 | Samsung |
| 800087 | NR mobility enhancements | NR_Mob_enh | R2 | RP-190489 | Intel |
| 800187 | Core part: NR mobility enhancements | NR_Mob_enh-Core | R2 | RP-190489 | Intel |
| 800287 | Perf. part: NR mobility enhancements | NR_Mob_enh-Perf | R4 | RP-190489 | Intel |
| 820068 | 2-step RACH for NR | NR_2step_RACH | R1 | RP-190711 | ZTE |
| 820168 | Core part: 2-step RACH for NR | NR_2step_RACH-Core | R1 | RP-190711 | ZTE |
| 820268 | Perf. part: 2-step RACH for NR | NR_2step_RACH-Perf | R4 | RP-190711 | ZTE |
| 840091 | SON (Self-Organising Networks) and MDT (Minimization of Drive Tests) support for NR | NR_SON_MDT | R3 | RP-191594 | CMCC |
| 840191 | Core part: SON (Self-Organising Networks) and MDT (Minimization of Drive Tests) support for NR | NR_SON_MDT-Core | R3 | RP-191594 | CMCC |
| 840093 | RRM requirement for CSI-RS based L3 measurement in NR | NR_CSIRS_L3meas | R4 | RP-191580 | CATT |
| 840193 | Core part: RRM requirement for CSI-RS based L3 measurement in NR | NR_CSIRS_L3meas-Core | R4 | RP-191580 | CATT |
| 840293 | Perf. part: RRM requirement for CSI-RS based L3 measurement in NR | NR_CSIRS_L3meas-Perf | R4 | RP-191580 | CATT |
| 840094 | NR performance requirement enhancement | NR_perf_enh | R4 | RP-191587 | China Telecom |
| 840294 | Perf. part: NR performance requirement enhancement | NR_perf_enh-Perf | R4 | RP-191587 | China Telecom |
| 840095 | NR RRM enhancement | NR_RRM_enh | R4 | RP-191601 | Intel |
| 840195 | Core part: NR RRM enhancement | NR_RRM_enh-Core | R4 | RP-191601 | Intel |
| 840295 | Perf. part: NR RRM enhancement | NR_RRM_enh-Perf | R4 | RP-191601 | Intel |
| 840096 | Rel-16 NR inter-band Carrier Aggregation for 4 bands DL with 1 band UL | NR_CA_R16_4BDL_1BUL | R4 | RP-191196 | Ericsson |
| 840196 | Core part: Rel-16 NR inter-band Carrier Aggregation for 4 bands DL with 1 band UL | NR_CA_R16_4BDL_1BUL-Core | R4 | RP-191196 | Ericsson |
| 840296 | Perf. part: Rel-16 NR inter-band Carrier Aggregation for 4 bands DL with 1 band UL | NR_CA_R16_4BDL_1BUL-Perf | R4 | RP-191196 | Ericsson |
| 840098 | Addition of wider channel bandwidth in NR band n1 | NR_n1_BW | R4 | RP-191595 | Ericsson |
| 840198 | Core part: Addition of wider channel bandwidth in NR band n1 | NR_n1_BW-Core | R4 | RP-191595 | Ericsson |
| 840298 | Perf. part: Addition of wider channel bandwidth in NR band n1 | NR_n1_BW-Perf | R4 | RP-191595 | Ericsson |
| 840099 | Addition of channel bandwidths in Band n77 and n78 | NR_n77_n78_BW | R4 | RP-191606 | Huawei |
| 840199 | Core part: Addition of channel bandwidths in Band n77 and n78 | NR_n77_n78_BW-Core | R4 | RP-191606 | Huawei |
| 841000 | Dual Connectivity (EN-DC) with 3 bands DL and 3 bands UL | DC_R16_LTE_NR_3DL3UL | R4 | RP-191598 | ZTE |
| 841100 | Core part: Dual Connectivity (EN-DC) with 3 bands DL and 3 bands UL | DC_R16_LTE_NR_3DL3UL-Core | R4 | RP-191598 | ZTE |
| 841200 | Perf. part: Dual Connectivity (EN-DC) with 3 bands DL and 3 bands UL | DC_R16_LTE_NR_3DL3UL-Perf | R4 | RP-191598 | ZTE |
| 841001 | Introduction of NR FDD bands with variable duplex and corresponding framework | NR_FDD_bands_varduplex | R4 | RP-191567 | Huawei |
| 841101 | Core part: Introduction of NR FDD bands with variable duplex and corresponding framework | NR_FDD_bands_varduplex-Core | R4 | RP-191567 | Huawei |
| 841002 | Common RF requirement configured output power for EN-DC with 3 uplink CC and 2 different bands (2CC LTE, 1CC NR FR1) | DC_Pcmax_3UL_CC | R4 | RP-191604 | Huawei |
| 841102 | Core part: Common RF requirement configured output power for EN-DC with 3 uplink CC and 2 different bands (2CC LTE, 1CC NR FR1) | DC_Pcmax_3UL_CC-Core | R4 | RP-191604 | Huawei |
| 0 | Release 16 Features impacting both LTE and NR | - |  | - | - |
| 800088 | LTE-NR & NR-NR Dual Connectivity and NR Carrier Aggregation enhancements | LTE_NR_DC_CA_enh | R2 | RP-191600 | Ericsson |
| 800188 | Core part: LTE-NR & NR-NR Dual Connectivity and NR CA enhancements | LTE_NR_DC_CA_enh-Core | R2 | RP-190452 | Ericsson |
| 800288 | Perf. part: LTE-NR & NR-NR Dual Connectivity and NR CA enhancements | LTE_NR_DC_CA_enh-Perf | R4 | RP-190452 | Ericsson |
| 820079 | High power UE (power class 2) for EN-DC (1 LTE TDD band + 1 NR TDD band) | ENDC_UE_PC2_TDD_TDD | R4 | RP-190315 | CMCC |
| 820061 | Study on high power UE (power class 2) for EN-DC (1 LTE FDD band + 1 NR TDD band) | FS_ENDC_UE_PC2_FDD_TDD | R4 | RP-190167 | China Unicom |
| 820179 | Core part: High power UE (power class 2) for EN-DC (1 LTE TDD band + 1 NR TDD band) | ENDC_UE_PC2_TDD_TDD-Core | R4 | RP-190315 | CMCC |
| 820279 | Perf. part: Power class 2 UE for EN-DC (1 LTE TDD band + 1 NR TDD band) | ENDC_UE_PC2_TDD_TDD-Perf | R4 | RP-190315 | CMCC |
| 790054 | eNB(s) Architecture Evolution for E-UTRAN and NG-RAN | LTE_NR_arch_evo | R3 | RP-180531 | China Unicom |
| 790154 | Core part: eNB(s) Architecture Evolution for E-UTRAN and NG-RAN | LTE_NR_arch_evo-Core | R3 | RP-181375 | China Unicom |
| 820080 | LTE/NR spectrum sharing in Band 41/n41 frequency range | NR_n41_LTE_41_coex | R4 | RP-191588 | KDDI |
| 820180 | Core part: LTE/NR spectrum sharing in Band 41/n41 frequency range | NR_n41_LTE_41_coex-Core | R4 | RP-190175 | KDDI |
| 800078 | 29 dBm UE Power Class for LTE band 41 and NR Band n41 | LTE_NR_B41_Bn41_PC29dBm | R4 | RP-190502 | Sprint |
| 800178 | Core part: 29 dBm UE Power Class for LTE band 41 and NR Band n41 | LTE_NR_B41_Bn41_PC29dBm-Core | R4 | RP-190502 | Sprint |
| 800058 | Dual Connectivity (EN-DC) |  | R4 |  | Ericsson |
| 800066 | Dual Connectivity (EN-DC) of 1 LTE band (1UL/1DL) and 1 NR band (1DL/1UL) | DC_R16_1BLTE_1BNR_2DL2UL | R4 | RP-191568 | NTT DOCOMO |
| 800166 | Core part: Dual Connectivity (EN-DC) of 1 LTE band (1UL/1DL) and 1 NR band (1DL/1UL) | DC_R16_1BLTE_1BNR_2DL2UL-Core | R4 | RP-190705 | NTT DOCOMO |
| 800266 | Perf. part: Dual Connectivity (EN-DC) of 1 LTE band (1UL/1DL) and 1 NR band (1DL/1UL) | DC_R16_1BLTE_1BNR_2DL2UL-Perf | R4 | RP-190705 | NTT DOCOMO |
| 800067 | Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL) | DC_R16_2BLTE_1BNR_3DL2UL | R4 | RP-191124 | Huawei |
| 800167 | Core part: Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL) | DC_R16_2BLTE_1BNR_3DL2UL-Core | R4 | RP-190379 | Huawei |
| 800267 | Perf. part: Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL) | DC_R16_2BLTE_1BNR_3DL2UL-Perf | R4 | RP-190379 | Huawei |
| 800068 | Dual Connectivity (EN-DC) of 3 bands LTE inter-band CA (3DL/1UL) and 1 NR band (1DL/1UL) | DC_R16_3BLTE_1BNR_4DL2UL | R4 | RP-191195 | Ericsson |
| 800168 | Core part: Dual Connectivity (EN-DC) of 3 bands LTE inter-band CA (3DL/1UL) and 1 NR band (1DL/1UL) | DC_R16_3BLTE_1BNR_4DL2UL-Core | R4 | RP-190159 | Ericsson |
| 800268 | Perf. part: Dual Connectivity (EN-DC) of 3 bands LTE inter-band CA (3DL/1UL) and 1 NR band (1DL/1UL) | DC_R16_3BLTE_1BNR_4DL2UL-Perf | R4 | RP-190159 | Ericsson |
| 800069 | Dual Connectivity (EN-DC) of 4 bands LTE inter-band CA (4DL/1UL) and 1 NR band (1DL/1UL) | DC_R16_4BLTE_1BNR_5DL2UL | R4 | RP-191205 | Nokia |
| 800169 | Core part: Dual Connectivity (EN-DC) of 4 bands LTE inter-band CA (4DL/1UL) and 1 NR band (1DL/1UL) | DC_R16_4BLTE_1BNR_5DL2UL-Core | R4 | RP-190294 | Nokia |
| 800269 | Perf. part: Dual Connectivity (EN-DC) of 4 bands LTE inter-band CA (4DL/1UL) and 1 NR band (1DL/1UL) | DC_R16_4BLTE_1BNR_5DL2UL-Perf | R4 | RP-190294 | Nokia |
| 800070 | Dual Connectivity (EN-DC) of 5 bands LTE inter-band CA (5DL/1UL) and 1 NR band (1DL/1UL) | DC_R16_5BLTE_1BNR_6DL2UL | R4 | RP-180997 | Samsung |
| 800170 | Core part: Dual Connectivity (EN-DC) of 5 bands LTE inter-band CA (5DL/1UL) and 1 NR band (1DL/1UL) | DC_R16_5BLTE_1BNR_6DL2UL-Core | R4 | RP-180997 | Samsung |
| 800270 | Perf. part: Dual Connectivity (EN-DC) of 5 bands LTE inter-band CA (5DL/1UL) and 1 NR band (1DL/1UL) | DC_R16_5BLTE_1BNR_6DL2UL-Perf | R4 | RP-180997 | Samsung |
| 800071 | Dual Connectivity (EN-DC) of x bands (x=1, 2, 3, 4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL) | DC_R16_xBLTE_2BNR_yDL2UL | R4 | RP-190949 | LG Electronics |
| 800171 | Core part: Dual Connectivity (EN-DC) of x bands (x=2, 3, 4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL) | DC_R16_xBLTE_2BNR_yDL2UL-Core | R4 | RP-190151 | LG Electronics |
| 800271 | Perf. part: Dual Connectivity (EN-DC) of x bands (x=2, 3, 4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL) | DC_R16_xBLTE_2BNR_yDL2UL-Perf | R4 | RP-190151 | LG Electronics |
| 0 | LTE-related Release 16 Features | - |  | - | - |
| 790057 | E-UTRA 2.4 GHz TDD Band for US | LTE_TDD_2400_US | R4 | RP-182405 | Globalstar |
| 790157 | Core part: E-UTRA 2.4 GHz TDD Band for US | LTE_TDD_2400_US-Core | R4 | RP-182405 | Globalstar |
| 790257 | Perf. part: E-UTRA 2.4 GHz TDD Band for US | LTE_TDD_2400_US-Perf | R4 | RP-182405 | Globalstar |
| 820063 | UE Conformance Test Aspects - E-UTRA 2.4 GHz TDD Band for US (Band 53) | LTE_TDD_2400_US-UEConTest | R5 | RP-182406 | Globalstar |
| 800059 | LTE intra-band Carrier Aggregation (CA) |  | R4 |  | Qualcomm |
| 800060 | Rel-16 LTE intra-band CA for x CC DL/y CC UL including contiguous and non-contiguous spectrum (x>=y) | LTE_CA_R16_intra | R4 | RP-191193 | Ericsson |
| 800160 | Core part: Rel-16 LTE intra-band CA for x CC DL/y CC UL including contiguous and non-contiguous spectrum (x>=y) | LTE_CA_R16_intra-Core | R4 | RP-190157 | Ericsson |
| 800260 | Perf. part: Rel-16 LTE intra-band CA for x CC DL/y CC UL including contiguous and non-contiguous spectrum (x>=y) | LTE_CA_R16_intra-Perf | R4 | RP-190157 | Ericsson |
| 800061 | Rel-16 LTE inter-band CA for 2 bands DL with 1 band UL | LTE_CA_R16_2BDL_1BUL | R4 | RP-191085 | Qualcomm |
| 800161 | Core part: Rel-16 LTE inter-band CA for 2 bands DL with 1 band UL | LTE_CA_R16_2BDL_1BUL-Core | R4 | RP-191085 | Qualcomm |
| 800261 | Perf. part: Rel-16 LTE inter-band CA for 2 bands DL with 1 band UL | LTE_CA_R16_2BDL_1BUL-Perf | R4 | RP-191085 | Qualcomm |
| 800062 | Rel-16 LTE inter-band CA for 3 bands DL with 1 band UL | LTE_CA_R16_3BDL_1BUL | R4 | RP-191120 | Huawei |
| 800162 | Core part: Rel-16 LTE inter-band CA for 3 bands DL with 1 band UL | LTE_CA_R16_3BDL_1BUL-Core | R4 | RP-191120 | Huawei |
| 800262 | Perf. part: Rel-16 LTE inter-band CA for 3 bands DL with 1 band UL | LTE_CA_R16_3BDL_1BUL-Perf | R4 | RP-191120 | Huawei |
| 800063 | Rel-16 LTE inter-band CA for x bands DL (x=4, 5) with 1 band UL | LTE_CA_R16_xBDL_1BUL | R4 | RP-190906 | Nokia |
| 800163 | Core part: Rel-16 LTE inter-band CA for x bands DL (x=4, 5) with 1 band UL | LTE_CA_R16_xBDL_1BUL-Core | R4 | RP-190138 | Nokia |
| 800263 | Perf. part: Rel-16 LTE inter-band CA for x bands DL (x=4, 5) with 1 band UL | LTE_CA_R16_xBDL_1BUL-Perf | R4 | RP-190138 | Nokia |
| 800064 | Rel-16 LTE inter-band CA for 2 bands DL with 2 band UL | LTE_CA_R16_2BDL_2BUL | R4 | RP-190377 | Huawei |
| 800164 | Core part: Rel-16 LTE inter-band CA for 2 bands DL with 2 band UL | LTE_CA_R16_2BDL_2BUL-Core | R4 | RP-190377 | Huawei |
| 800264 | Perf. part: Rel-16 LTE inter-band CA for 2 bands DL with 2 band UL | LTE_CA_R16_2BDL_2BUL-Perf | R4 | RP-190377 | Huawei |
| 800065 | Rel-16 LTE inter-band CA for x bands DL (x= 3, 4, 5) with 2 band UL | LTE_CA_R16_xBDL_2BUL | R4 | RP-190946 | LG Electronics |
| 800165 | Core part: Rel-16 LTE inter-band CA for x bands DL (x= 3, 4, 5) with 2 band UL | LTE_CA_R16_xBDL_2BUL-Core | R4 | RP-190150 | LG Electronics |
| 800265 | Perf. part: Rel-16 LTE inter-band CA for x bands DL (x= 3, 4, 5) with 2 band UL | LTE_CA_R16_xBDL_2BUL-Perf | R4 | RP-190150 | LG Electronics |
| 810061 | UE Conformance Test Aspects – Rel-16 LTE CA configurations | LTE_CA_R16-UEConTest | R5 | RP-182648 | Ericsson |
| 830076 | LTE-based 5G terrestrial broadcast | LTE_terr_bcast | R1 | RP-190732 | Qualcomm |
| 800091 | Study on LTE-based 5G terrestrial broadcast | FS_LTE_terr_bcast | R1 | RP-181342 | Qualcomm |
| 830176 | Core part: LTE-based 5G terrestrial broadcast | LTE_terr_bcast-Core | R1 | RP-190732 | Qualcomm |
| 830276 | Perf. part: LTE-based 5G terrestrial broadcast | LTE_terr_bcast-Perf | R4 | RP-190732 | Qualcomm |
| 830094 | Power class 2 UE for LTE bands 31 and 72 | LTE_PC2_B31_B72 | R4 | RP-191285 | Nokia |
| 830194 | Core part: Power class 2 UE for LTE bands 31 and 72 | LTE_PC2_B31_B72-Core | R4 | RP-190665 | Nokia |
| 830294 | Perf. part: Power class 2 UE for LTE bands 31 and 72 | LTE_PC2_B31_B72-Perf | R4 | RP-190665 | Nokia |
| 800056 | Additional LTE bands |  | R4 |  | Ericsson |
| 800075 | Additional LTE bands for UE category M1 and/or NB1 in Rel-16 | LTE_bands_R16_M1_NB1 | R4 | RP-190332 | Ericsson |
| 800175 | Core part: Additional LTE bands for UE category M1 and/or NB1 in Rel-16 | LTE_bands_R16_M1_NB1-Core | R4 | RP-190332 | Ericsson |
| 800275 | Perf. part: Additional LTE bands for UE category M1 and/or NB1 in Rel-16 | LTE_bands_R16_M1_NB1-Perf | R4 | RP-190332 | Ericsson |
| 800076 | Additional LTE bands for UE category M2 and/or NB2 in Rel-16 | LTE_bands_R16_M2_NB2 | R4 | RP-190333 | Ericsson |
| 800176 | Core part: Additional LTE bands for UE category M2 and/or NB2 in Rel-16 | LTE_bands_R16_M2_NB2-Core | R4 | RP-190333 | Ericsson |
| 800079 | Further performance enhancement for LTE in high speed scenario | LTE_high_speed_enh2 | R4 | RP-181482 | NTT DOCOMO |
| 800179 | Core part: Further performance enhancement for LTE in high speed scenario | LTE_high_speed_enh2-Core | R4 | RP-181482 | NTT DOCOMO |
| 800279 | Perf. part: Further performance enhancement for LTE in high speed scenario | LTE_high_speed_enh2-Perf | R4 | RP-181482 | NTT DOCOMO |
| 800082 | Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR | NR_CLI_RIM | R1 | RP-191546 | LG Electronics |
| 800182 | Core part: Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR | NR_CLI_RIM-Core | R1 | RP-190700 | LG Electronics |
| 800282 | Perf. part: Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR | NR_CLI_RIM-Perf | R4 | RP-190700 | LG Electronics |
| 800086 | DL MIMO efficiency enhancements for LTE | LTE_DL_MIMO_EE | R1 | RP-182901 | Huawei |
| 800186 | Core part: DL MIMO efficiency enhancements for LTE | LTE_DL_MIMO_EE-Core | R1 | RP-182901 | Huawei |
| 800286 | Perf. part: DL MIMO efficiency enhancements for LTE | LTE_DL_MIMO_EE-Perf | R4 | RP-182901 | Huawei |
| 820071 | Transfer of Iuant interface specifications from 25-series to 37-series | Iuant_transfer | R3 | RP-190160 | Huawei |
| 820171 | Core part: Transfer of Iuant interface specifications from 25-series to 37-series | Iuant_transfer-Core | R3 | RP-190160 | Huawei |
| 820072 | Direct data forwarding between NG-RAN and E-UTRAN nodes for inter-system mobility | Direct_data_fw_NR | R3 | RP-182886 | Ericsson |
| 820172 | Core part: Direct data forwarding between NG-RAN and E-UTRAN nodes for inter-system mobility | Direct_data_fw_NR-Core | R3 | RP-182886 | Ericsson |
| 800089 | Even further mobility enhancement in E-UTRAN | LTE_feMob | R2 | RP-190921 | China Telecom |
| 800189 | Core part: Even further mobility enhancement in E-UTRAN | LTE_feMob-Core | R2 | RP-190272 | China Telecom |
| 800289 | Perf. part: Even further mobility enhancement in E-UTRAN | LTE_feMob-Perf | R4 | RP-190272 | China Telecom |
| 820078 | 410 - 430 MHz E-UTRA FDD Band(s) for LTE PPDR and PMR/PAMR in Europe | LTE410_Europe_PPDR | R4 | RP-191287 | Nokia |
| 820178 | Core part: 410 - 430 MHz E-UTRA FDD Band(s) for LTE PPDR and PMR/PAMR in Europe | LTE410_Europe_PPDR-Core | R4 | RP-182898 | Nokia |
| 820278 | Perf. part: 410 - 430 MHz E-UTRA FDD Band(s) for LTE PPDR and PMR/PAMR in Europe | LTE410_Europe_PPDR-Perf | R4 | RP-182898 | Nokia |
| 0 | All other Release 16 Features | - |  | - | - |
| 830086 | Introduction of GSM, UTRA, E-UTRA and NR capability set(s) (CS(s)) to the multi-standard radio (MSR) specifications | MSR_GSM_UTRA_LTE_NR | R4 | RP-190642 | Ericsson |
| 830186 | Core part: Introduction of GSM, UTRA, E-UTRA and NR capability set(s) (CS(s)) to the multi-standard radio (MSR) specifications | MSR_GSM_UTRA_LTE_NR-Core | R4 | RP-190642 | Ericsson |
| 830286 | Perf. part: Introduction of GSM, UTRA, E-UTRA and NR capability set(s) (CS(s)) to the multi-standard radio (MSR) specifications | MSR_GSM_UTRA_LTE_NR-Perf | R4 | RP-190642 | Ericsson |
| 840066 | Volume Based Charging Aspects for VoLTE | VBCLTE | S5 | SP-180813 | Chen, Ai, China Mobile |
| 810021 | Stage 2 of VBCLTE | VBCLTE | S5 | SP-180813 | Chen, Ai, China Mobile |
| 840012 | CT aspects of VBCLTE | VBCLTE | ct | CP-191110 | Huang Zhenning (China Mobile) |
| 840067 | CT1 aspects of VBCLTE | VBCLTE | C1 | CP-191110 | Huang Zhenning (China Mobile) |
| 840068 | CT3 aspects of VBCLTE | VBCLTE | C3 | CP-191110 | Huang Zhenning (China Mobile) |
| 790003 | Enhancements to IMS for new real time communication services | enIMS | S1 | SP-180137 | China Mobile, Hui Cai |
| 820028 | Enhancement of 5G PCC related services | en5GPccSer | C3 | CP-183246 | Xiaoyun Zhou, Huawei |
| 810023 | Energy efficiency of 5G | EE_5G | S5 | SP-180819 | CORNILY, Jean-Michel, ORANGE |
| 820039 | Signalling Improvements for Network Efficiency in 5GS | SINE_5G | C1 | CP-190142 | Shu, Lin |
| 790001 | New Services and Markets Technology Enablers – Phase 2 | SMARTER_Ph2 | S1 | SP-180589 | Li, Alice, Vodafone |
| 780055 | 5G Voice Service Continuity | 5GVSC | S1 | SP-171077 | Qun Wei, China Unicom |
| 790015 | Security Assurance Specification for 5G | SCAS_5G | S3 | SP-180689 | Marcus Wong, Huawei Technologies |
| 840069 | 5GS Transfer of Policies for Background Data Transfer | xBDT | S2 | SP-180509 | Takeshi Usui KDDI |
| 800030 | Stage 2 of xBDT | xBDT | S2 | SP-180509 | Takeshi Usui KDDI |
| 840014 | CT aspects of xBDT | xBDT | ct | CP-191113 | Yan Xiaojian, ZTE |
| 840070 | CT1 aspects of xBDT | xBDT | C1 | CP-191113 | Yan Xiaojian, ZTE |
| 840071 | CT3 aspects of xBDT | xBDT | C3 | CP-191113 | Yan Xiaojian, ZTE |
| 770020 | Service Interactivity | SerInter | S4 | SP-170796 | Lo, Charles, Qualcomm |
| 770024 | EVS Codec Extension for Immersive Voice and Audio Services | IVAS_Codec | S4 | SP-170611 | Gibbs, Jon, Huawei Technologies Co Ltd |
| 800002 | Alternative EVS implementation using updated fixed-point basic operators | Alt_FX_EVS | S4 | SP-180286 | Pawate, Raj, Cadence Design Systems Inc |
| 800046 | Shared Data Handling on Nudm and Nudr | Shared_Data | C4 | CP-181136 | Wiehe, Ulrich, Nokia |
| 800009 | Policy delivery to UE for background data transfer | PDBDT | S1 | SP-180324 | Takatsugu Kito, KDDI |
| 790004 | QoS Monitoring | QoS_MON | S1 | SP-180141 | Amar Deol |
| 800005 | Inter-RAT Mobility requirements for real time service | MOBRT | S1 | SP-180592 | Yang Xu, Oppo |
| 760058 | Management of QoE measurement collection | QOED | S5 | SP-181069 | Petersen, Robert, Ericsson |
| 760059 | Study on network policy management for mobile networks based on NFV scenarios | FS_NETPOL | S5 | SP-170593 | China Mobile Jinglei Liu |
| 810025 | Network policy management for mobile networks based on NFV scenarios | NETPOL | S5 | SP-180821 | China Mobile Hao Zhang |
| 810027 | Intent driven management service for mobile network | IDMS_MN | S5 | SP-180899 | Zou Lan, Huawei |
| 810024 | OAM aspects of LTE and WLAN integration | OAM_LTE_WLAN | S5 | SP-180820 | Yizhi Yao, Intel Corporation |
| 810026 | Methodology for 5G management specifications | METHOGY | S5 | SP-180822 | Groenendijk, Jan, Ericsson |
| 820030 | Nchf Online and Offline charging services | OFSBI_CH | S5 | SP-181066 | Zhu Lei, Huawei |
| 820031 | Charging Enhancement of 5GC interworking with EPC | 5GIEPC_CH | S5 | SP-181067 | Chen Shan, Huawei |
| 820032 | NRM enhancements | eNRM | S5 | SP-190140 | Jing Ping, Nokia |
| 820033 | Network Exposure Charging in 5G System Architecture | 5GS_Ph1_NEFCH | S5 | SP-181070 | Tornkvist, Robert, Ericsson |
| 820034 | Charging AMF in 5G System Architecture Phase 1 | 5GS_Ph1_AMFCH | S5 | SP-181071 | Gardella, Maryse, Nokia |
| 820035 | Discovery of management services in 5G | 5GDMS | S5 | SP-181072 | Attila Horvat, Huawei Technologies Sweden AB |
| 820036 | Trace Management in the context of Services Based Management Architecture | TM_SBMA | S5 | SP-181073 | Andrianov, Anatoly, Nokia |
| 820005 | Lawful Interception Rel-16 | LI16 | S3LI | SP-181210 | Alex Leadbeater, BT |
| 820038 | IMS Stage-3 IETF Protocol Alignment | IMSProtoc16 | C1 | CP-183084 | Leis, Peter, Nokia |
| 820041 | Stage-3 5GS NAS protocol development | 5GProtoc16 | C1 | CP-183087 | Sedlacek, Ivo, Ericsson |
| 820049 | Stage-3 5GS NAS protocol development general aspects | 5GProtoc16 | C1 | CP-183087 | Sedlacek, Ivo, Ericsson |
| 820050 | Stage-3 5GS NAS protocol development for support for non-3GPP accesses | 5GProtoc16-non3GPP | C1 | CP-183087 | Sedlacek, Ivo, Ericsson |
| 820042 | Stage-3 SAE Protocol Development for Rel16 | SAES16 | C1 | CP-183088 | Aghili, Behrouz, InterDigital Communications |
| 820046 | Stage-3 SAE Protocol Development for Rel16 general aspects | SAES16 | C1 | CP-183088 | Aghili, Behrouz, InterDigital Communications |
| 820047 | Stage-3 SAE Protocol Development for Rel16 for CS Fallback in EPS | SAES16-CSFB | C1 | CP-183088 | Aghili, Behrouz, InterDigital Communications |
| 820048 | Stage-3 SAE Protocol Development Rel16 for support for non-3GPP accesses | SAES16-non3GPP | C1 | CP-183088 | Aghili, Behrouz, InterDigital Communications |
| 840017 | Reliable Data Service Serialization Indication | RDSSI | S2 | SP-190446 | Starsinic, Michael, Convida Wireless LLC |
| 840018 | S6b Optional for ePDG connected to 5GS | 5GS_S6b_Optional | S2 | SP-190447 | Irfan Ali, Cisco |
| 840002 | Service Based Interface Protocol Improvements | SBIProtoc16 | ct | CP-191060 | Yue Song, China Mobile |
| 840052 | CT3 aspects of SBIProtoc16 | SBIProtoc16 | C3 | CP-191060 | Yue Song, China Mobile |
| 840053 | CT4 aspects of SBIProtoc16 | SBIProtoc16 | C4 | CP-191060 | Yue Song, China Mobile |
| 840008 | Withdrawal of TS 24.323 from Rel-11, Rel-12, Rel-13 | ISAT-MO-WITHDRAW | C1 | CP-191151 | Dawes, Peter, Vodafone |
| 840051 | Lawful Interception Report Rel-16 | LIR16 | S3LI | SP-190342 | Nag Rao, Nokia |
| 770050 | (Small) Technical Enhancements and Improvements for Rel-16 | TEI16 |  |  |  |
| 800100 | (IETF) The Transport Layer Security (TLS) Protocol Version 1.3 (RFC 8446) | TEI16 | S3-IETF |  |  |

Annex A1:
Template for contributing

Editor's note: To contribute to this document, please copy-paste the following template into a document to be submitted to the TSG plenary as "Work Item Summary". This will be further endorsed by the plenary.
Each summary is expected to be one page long maximum. 
The target audience of this document are readers inside and outside the 3GPP community. Contributions should be written as to be understandable by non-specialist. 
The summary should reflect the actual outcome of the Feature - and not the initial intentions.

---

Summary based on the input provided by [company name] in [tdoc numbed].

| Unique_ID | Name | Acronym | WG | WID | WI_rapporteur_name |
| --- | --- | --- | --- | --- | --- |
| AABBCC | Title here | XXX | e.g. S4 | xP-19XXXX | Firstname Familyname, Company |

1	Introduction

<guidance:
- summarize in one to a few sentences what was achieved by this WI and how: what problems it solves, what new functionality it adds, etc.;

- remember that only what ended up in the specs is of interest;

- clarify what is the relation of this WI to other WIs (e.g. is this an enhancement of a former WI? Is there a related WI?) and what was the status before this WI

2	Description

<guidance:

- Explain here in a couple of paragraphs the main impacts on the system, mostly: the new and/or impacted entities, and the big lines of the system's behaviour;

- remember that only what ended up in the specs shall be covered here;

- this it not intended to be a copy and paste from the specifications, the WID or status report;

- the reader shall understand how the key functionalities are working without running too far into details;

- use figures, tables, references to illlustrate and  facilitate the understanding;

- remember that you do not write the WI summary for people who contributed to this WI so make sure that you use terminology and abbreviations that can be understood by any engineer or make sure you explain them;

- When the usual Stage 1/Stage 2/Stage 3 process is not followed -and in particular if a full stage is missing- explain why.

- make the following test: give this summary to a colleague who was not at all involved in this topic and let this colleague explain in her/his own words how your WI works after reading the summary>

3	References

<guidance: It is recommended to add some references where the interested reader can find further documentation, e.g.

- new/impacted TRs and TSs (if any)

- last status report (if any)

- main CRs

NOTE: Please provide references that can be found easily, e.g. indicating Tdoc numbers.>

[X] TS XX.XXX, Spec title

Annex B:
Process to get further information

# B.1	General

Since the present document is limited to provide an overview of each Feature, this chapter explains how to get additional information, in particular how to retrieve all the Specifications (TSs) and Reports (TRs) as well as all the CRs which relate to a given Work Item.

The Unique Identifier (UID) is the key to get additional information on a given Work Item. It can be found in the table located just below the clause's header. The table has the following format:

Table B-1: table format

| Unique Identifier (UID) | Name | Acronym | Outline Level (1=Feature, 2=Building Block, 3=Work Task) | Responsible Working Group | Work Item Description |
| --- | --- | --- | --- | --- | --- |

For readability reasons, the table headers are omitted in the continuation of the present document.

For instance, for the "Mission Critical Push to Talk over LTE Realignment", the table has to be understood as:

Table B-2: Example of table at the introduction of each Feature

| Unique Identifier (UID) | Name | Acronym | Outline Level | Responsible Working Group | Work Item Description |
| --- | --- | --- | --- | --- | --- |
| 700029 | Mission Critical Push to Talk over LTE Realignment | MCImp-MCPTTR | 2 | S1 | SP-150821 |

Thus, the UID for this Work Item is 700029.

Two methods are now possible to retrieve more information on a given feature: the "Step by step method" and the "Direct method". The "direct method" is faster but implies to know the hierarchical structure of the Work Items. The "step by step method" is slower but is easier to use, in particular when the hierarchical structure is unknown.

For instance, for retrieving all the CRs that relate to "Enhancements for Mission Critical Push To Talk", the search has to be done on UID 740022 but also potentially on its children Work Items (UID 720056, 740023 and 740024).

Table B-3: Example of a hierarchical structure and its consequences on the search procedure

| 740022 | Enhancements for Mission Critical Push To Talk | MCImp-eMCPTT |  | SP-160490 |
| --- | --- | --- | --- | --- |
| 720056 | Stage 2 of Enhancements for Mission Critical Push To Talk | MCImp-eMCPTT | S6 | SP-160490 |
| 740023 | Stage 3 of Enhancements for Mission Critical Push To Talk | MCImp-eMCPTT-CT | CT | CP-160824 |
| 740024 | CT1 aspects of Enhancements for Mission Critical Push To Talk | MCImp-eMCPTT-CT | C1 | CP-160824 |

These two methods are described in the following clauses.

# B.2	Direct method

The links below lead to the pages containing respectively all the Specifications and all the Change Requests (CRs) linked to a given Feature:

https://portal.3gpp.org/Specifications.aspx?q=1&WiUid=[UID]

https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=[UID]

where "[UID]" has to be preplaced by the UID value.

Using the example provided in the table 4.1-3, the specification linked to "Stage 2 of Enhancements for Mission Critical Push To Talk" can be found in:

https://portal.3gpp.org/Specifications.aspx?q=1&WiUid=720056

And all the related Change Requests are listed in:

https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=720056

# B.3	Step by step method

This method is to be used when the hierarchical structure is not known or when the "direct method" above does not show the expected results.

In this case, the 3GPP Ultimate web site has to be used:

https://portal.3gpp.org

As a preliminary step, it is essential that the "Customized Selection" is set to "All TSGs" (otherwise, a filter would be applied).

Figure B.3-1: Selecting "All TSGs" in "Customized Selection" as to remove any potential filter on the Search

Then select the "Work Plan" tab (upper red arrow in the figure below).

Figure B.3-2: Using the 3GU Portal to retrieve more information on a given Feature

Then the search might be performed by either typing the Acronym (as shown by the left red arrow on the figure above, using the example "MCPTT"), or by the name or UID (right box) then by clicking on the "Search" button. Watch the "Granularity (Level)" field, which is a filter to return only the Items which level is specified here.

In the results, the icon depicting some binoculars has to be hit (lower right red arrow on the figure above).

This will lead to the page shown in the figure below:

Figure B.3-3: Window resulting from a "Search"

On this window, the "Related" tab has to be clicked, as pointed by the red arrow in the figure above. This will lead to the window depicted in the figure below.

Figure B.3-4: "Related" tab in a Work Item search, with links to all related Specifications and Change Requests

The two links pointed by the red arrows in the figure above lead to the pages containing respectively all the Specifications and all the Change Requests (CRs) linked to this Feature.

Annex C:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-09 | TSG#85 |  |  |  |  | Initital draft, call for contributions | 0.1.0 |
| 2019-12 | TSG#86 |  |  |  |  | Inputs received: RP-190144 : Transfer of Iuant interface specifications from 25-series to 37-series RP-191816 Introduction of GSM, UTRA, E-UTRA and NR capability set(s) (CS(s)) to the multi-standard radio (MSR) specifications RP-192162 Direct data forwarding between NG-RAN and E-UTRAN nodes for inter-system mobility SP-190855 Charging Enhancement of 5GC interworking with EPC SP-190865 Coverage and Handoff Enhancements for Multimedia SP-191223 WI summary for 5G_eLCS CATT SP-191229 Work Item Summary for RDSSI Convida Wireless LLC SP-191247 Summary for work item 'Service Interactivity' Qualcomm Incorporated SP-191252 Summary for work item 'OAM aspects of LTE and WLAN integration' Intel SP-191277 Summary for WI Volume Based Charging Aspects for VoLTE China Mobile SP-191328 Summary for work item 'EVS Float Conformance for Non Bit-exact EVS_FCNBE Rapporteur (Intel) | 0.2.0 |
| 2020-01 | SA1#89 |  |  |  |  | Inputs received from RAN#86: RP-192721 Summary for WI: Further performance enhancement for LTE in high speed scenario DOCOMO Communications Lab. RP-192964 Summary for WI:eNB(s) Architecture Evolution for E-UTRAN and NG-RAN China Unicom | 0.3.0 |
| 2020-xx | SA1#90 |  |  |  |  | Inputs received from SA#86: SP-191223 WI summary for 5G_eLCS CATT (SP-191229 Work Item Summary for RDSSI Convida Wireless LLC-> replaced by SP-200057) SP-191247 Summary for work item 'Service Interactivity' Qualcomm Incorporated SP-191252 Summary for work item 'OAM aspects of LTE and WLAN integration' Intel SP-191277 Summary for WI Volume Based Charging Aspects for VoLTE China Mobile SP-191328 Summary for work item 'EVS Float Conformance for Non Bit-exact EVS_FCNBE Rapporteur (Intel) Inputs received from TSG#87e: RP-200087 Work Item Summary for 2-step RACH ZTE Corporation RP-200112 Work Item Summary for Dual Connectivity (EN-DC) with 3 bands DL and 3 bands UL ZTE Corporation RP-200152 Summary for WI: Single Radio Voice Call Continuity from 5G to 3G China Unicom RP-200170 Work Item Summary for inter-band CA/DC for 2 bands DL with up to 2 bands UL ZTE Corporation RP-200173 Work Item Summary for inter-band CA/DC for 3 bands DL with 2 bands UL ZTE Corporation RP-200214 Work Item Summary for High power UE (power class 2) for EN-DC (1 LTE TDD band + 1 NR TDD band) CMCC SP-200034 Summary for work item 'RTCP Verification for Real-Time Services' Ericsson LM SP-200036 Summary for work item "Media Handling Extensions for 5G Conversational Services” (5G_MEDIA_MTSI_ext) Intel (Rapporteur) SP-200057 Work Item Summary for RDSSI Convida Wireless LLC SP-200058 Work Item Summary for Rel-16 eV2XARC LG Electronics SP-200124 Work Item Summary for ATSSS ZTE Wistron Telecom AB SP-200125 Work Item Summary for eNS ZTE Wistron Telecom AB SP-200126 Work Item Summary for Rel-16 V2XIMP LG Electronics France SP-200128 Summary for work item "VR QoE Metrics" Ericsson LM=> NOT INCLUDED, SAID TO BE REL-17 in the Work Plan SP-200129 Work Item Summary for 5G_SRVCC China Unicom SP-200131 Work Item Summary for User Identities and Authentication Deutsche Telekom AG SP-200217 Work Item Summary for Business Role Models for Network Slicing Nokia CP-200143 Work Item Summary for Shared Data Handling on Nudm and Nudr Nokia, Nokia Shanghai Bell | 0.4.0 |

List of missing inputs:

| UID | Name | Acronym | WG | WID | Rapporteur |
| --- | --- | --- | --- | --- | --- |
| 830098 | Enhancement of URLLC support in the 5G Core network | 5G-URLLC | S2 | SP-181122 | Hui Ni, Huawei |
| 830074 | Physical Layer Enhancements for NR Ultra-Reliable and Low Latency Communication (URLLC) | NR-L1enh-URLLC | R1 | RP-191584 | Huawei |
| 830080 | Support of NR Industrial Internet of Things (IoT) | NR-IIOT | R2 | RP-191561 | Nokia |
| 830042 | 5GS Enhanced support of Vertical and LAN Services | Vertical-LAN | S2 | SP-181120 | Devaki Chandramouli, Nokia |
| 820027 | Service Enabler Architecture Layer for Verticals | SEAL | S6 | SP-181141 | Basavaraj (Basu) Pattan, Samsung |
| 820067 | NR-based access to unlicensed spectrum | NR-unlic | R1 | RP-191575 | Qualcomm |
| 800006 | LAN support in 5G | 5GLAN | S1 | SP-180593 | Jose Almodovar, KPN |
| 830043 | Cellular IoT support and evolution for the 5G System | 5G-CIoT | S2 | SP-181118 | Sebastian Speicher |
| 800083 | Additional MTC enhancements for LTE | LTE-eMTC5 | R1 | RP-191356 | Ericsson |
| 800084 | Additional enhancements for NB-IoT | NB-IOTenh3 | R1 | RP-191576 | Huawei |
| 810050 | 5G message service | 5GMSG | S1 | SP-180772 | Zheng, Jianping, China Mobile |
| 840074 | Application layer support for V2X services | V2XAPP | S6 | SP-180898 | Niranth Amogh, Huawei Tel.India |
| 830078 | 5G V2X with NR sidelink | 5G-V2X-NRSL | R1 | RP-190984 | LG Electronics |
| 800049 | Stage 1 of 5G_HYPOS | 5G-HYPOS | S1 | SP-180329 | Ries, Lionel, ESA |
| 830077 | NR positioning support | NR-pos | R1 | RP-191156 | Intel |
| 840054 | Optimisations on UE radio capability signalling | RACS | S2 | RP-191460 | Haris Zisimopoulos |
| 830079 | Optimisations on UE radio capability signalling – NR/E-UTRA Aspects | RACS-RAN | R2 | RP-190657 | MediaTek |
| 800048 | Stage 1 of 5GSAT | 5GSAT | S1 | SP-180326 | MICHEL, Cyril, Thales |
| 830047 | Enablers for Network Automation for 5G | eNA | S2 | SP-181123 | Xiaobo Wu, Huawei Technologies |
| 830050 | Wireless and Wireline Convergence for the 5G system architecture | 5WWC |  | SP-181117 | Marco Spini, Huawei Technologies |
| 810004 | MCData File Distribution support over xMB | MC-XMB | S4 | SP-180665 | Thiénot Cédric, Expway |
| 800053 | MBMS APIs for Mission Critical Services | MBMSAPI-MCS | S6 | SP-180380 | Ling Zhang, TD Tech Ltd. |
| 840013 | Enhancement of 3GPP Northbound APIs | eNAPIs | C3 | CP-191233 | Yali Yan, Huawei |
| 800019 | Enhanced Mission Critical System Migration and Interconnection | eMCSMI | S6 | SP-180379 | Chater-Lea, David; Motorola Solutions |
| 800021 | Enhanced Mission Critical Communication Interworking with Land Mobile Radio Systems | eMCCI | S6 | SP-180680 | Monnes, Peter, Harris Corporation |
| 800016 | Stage 3 for MC Communication Interworking with Land Mobile Radio Systems | MCCI-CT | C1 | CP-190203 | Monnes, Peter, Harris Corporation |
| 820040 | Mission Critical system migration and interconnection | MCSMI-CT | C1 | CP-190143 | Dom Lazara, Motorola Solutions |
| 800032 | Mission Critical Services Security Enhancements | MCXSec | S3 | SP-180596 | Woodward, Tim, Motorola Solutions, Inc |
| 780003 | Enhancements of Public Warning System | ePWS |  | SP-170998 | Hyounhee Koo, SyncTechno Inc. |
| 760054 | Mobile Communication System for Railways 2 | MONASTERY2 |  | SP-170451 | Merkel, Juergen, Nokia |
| 840092 | NR support for high speed train scenario | NR-HST | R4 | RP-191512 | CMCC |
| 800011 | Maritime Communication Services over 3GPP System | MARCOM | S1 | SP-180594 | Hyounhee Koo, SyncTechno Inc |
| 800008 | Enhancement of LTE for Efficient delivery of Streaming Service | eLSTR | S1 | SP-180322 | Xia, Xu, China Telecom |
| 800001 | Enhancements to Framework for Live Uplink Streaming | E-FLUS | S4 | SP-180285 | Lo, Charles, Qualcomm |
| 810003 | Coverage and Handoff Enhancements for Multimedia | CHEM | S4 | SP-180664 | Nikolai Leung |
| 820002 | Media streaming architecture | 5GMSA | S4 | SP-180984 | Frédéric GABIN |
| 800003 | Addition of HLG HDR to TV Video Profiles | HLG-HDR | S4 | SP-180287 | Paul Szucs, Sony Europe Limited |
| 810044 | Multi-device and multi-identity | MuD | S1 | SP-180315 | Peter Bleckert, Ericsson |
| 810031 | Enhancement of performance assurance for 5G networks including network slicing | 5G-SLICE-ePA | S5 | SP-190247 | Xiaowei Sun (China Mobile) |
| 760003 | Provision of Access to Restricted Local Operator Services by Unauthenticated UEs | PARLOS |  | SP-170449 | Covell, Betsy Nokia |
| 820043 | Enhancing Topology of SMF and UPF in 5G Networks | ETSUN |  | SP-181116 | Laurent Thiebaut (Nokia) |
| 820045 | Enhancements to the Service-Based 5G System Architecture | 5G-eSBA | S2 | SP-181125 |  |
| 840062 | SBA aspects of enhanced IMS to 5GC integration | eIMS5G-SBA | S2 | SP-190181 | T-Mobile USA, Christopher Joul |
| 830081 | Private Network Support for NG-RAN | NG-RAN-PRN | R3 | RP-190729 | China Telecom |
| 810039 | Media Handling Aspects of RAN Delay Budget Reporting in MTSI | E2E-DELAY | S4 | SP-180662 | Oyman, Ozgur, Company: Intel |
| 830069 | Enhancements for Common API Framework for 3GPP Northbound APIs | eCAPIF |  | SP-181137 | Basavaraj Pattan, Samsung |
| 790019 | Usage of CAPIF for xMB API | CAPIF4xMB | S4 | SP-180031 | Thorsten Lohmar, Ericsson LM |
| 840065 | User data interworking, Coexistence and Migration | UDICOM | S2 | SP-190182 | Alessio Casati, Nokia |
| 830075 | UE Power Saving in NR | NR-UE-pow-sav | R1 | RP-191607 | CATT |
| 820070 | Integrated access and backhaul for NR | NR-IAB |  | RP-191558 | Qualcomm |
| 800072 | Band combinations for SA NR supplementary uplink (SUL), NSA NR SUL, NSA NR SUL with UL sharing from the UE perspective (ULSUP) | NR-SUL-combos-R16 | R4 | RP-191126 | Huawei |
| 831000 | New bands for New Radio |  | R4 |  | Dish Network |
| 831001 | New Radio Band-related topics |  | R4 |  | Nokia |
| 800085 | Enhancements on MIMO for NR | NR-eMIMO | R1 | RP-182863 | Samsung |
| 800087 | NR mobility enhancements | NR-Mob-enh | R2 | RP-190489 | Intel |
| 840091 | SON (Self-Organising Networks) and MDT (Minimization of Drive Tests) support for NR | NR-SON-MDT | R3 | RP-191594 | CMCC |
| 840093 | RRM requirement for CSI-RS based L3 measurement in NR | NR-CSIRS-L3meas | R4 | RP-191580 | CATT |
| 840094 | NR performance requirement enhancement | NR-perf-enh | R4 | RP-191587 | China Telecom |
| 840095 | NR RRM enhancement | NR-RRM-enh | R4 | RP-191601 | Intel |
| 840096 | Rel-16 NR inter-band Carrier Aggregation for 4 bands DL with 1 band UL | NR-CA-R16-4BDL-1BUL | R4 | RP-191196 | Ericsson |
| 840098 | Addition of wider channel bandwidth in NR band n1 | NR-n1-BW | R4 | RP-191595 | Ericsson |
| 840099 | Addition of channel bandwidths in Band n77 and n78 | NR-n77-n78-BW | R4 | RP-191606 | Huawei |
| 841001 | Introduction of NR FDD bands with variable duplex and corresponding framework | NR-FDD-bands-varduplex | R4 | RP-191567 | Huawei |
| 841002 | Common RF requirement configured output power for EN-DC with 3 uplink CC and 2 different bands (2CC LTE, 1CC NR FR1) | DC-Pcmax-3UL-CC | R4 | RP-191604 | Huawei |
| 800088 | LTE-NR & NR-NR Dual Connectivity and NR Carrier Aggregation enhancements | LTE-NR-DC-CA-enh | R2 | RP-191600 | Ericsson |
| 790054 | eNB(s) Architecture Evolution for E-UTRAN and NG-RAN | LTE-NR-arch-evo | R3 | RP-180531 | China Unicom |
| 820080 | LTE/NR spectrum sharing in Band 41/n41 frequency range | NR-n41-LTE-41-coex | R4 | RP-191588 | KDDI |
| 800078 | 29 dBm UE Power Class for LTE band 41 and NR Band n41 | LTE-NR-B41-Bn41-PC29dBm | R4 | RP-190502 | Sprint |
| 800058 | Dual Connectivity (EN-DC) |  | R4 |  | Ericsson |
| 800059 | LTE intra-band Carrier Aggregation (CA) |  | R4 |  | Qualcomm |
| 830076 | LTE-based 5G terrestrial broadcast | LTE-terr-bcast | R1 | RP-190732 | Qualcomm |
| 830094 | Power class 2 UE for LTE bands 31 and 72 | LTE-PC2-B31-B72 | R4 | RP-191285 | Nokia |
| 800056 | Additional LTE bands |  | R4 |  | Ericsson |
| 800079 | Further performance enhancement for LTE in high speed scenario | LTE-high-speed-enh2 | R4 | RP-181482 | NTT DOCOMO |
| 800082 | Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR | NR-CLI-RIM | R1 | RP-191546 | LG Electronics |
| 800086 | DL MIMO efficiency enhancements for LTE | LTE-DL-MIMO-EE | R1 | RP-182901 | Huawei |
| 820071 | Transfer of Iuant interface specifications from 25-series to 37-series | Iuant-transfer | R3 | RP-190160 | Huawei |
| 820072 | Direct data forwarding between NG-RAN and E-UTRAN nodes for inter-system mobility | Direct-data-fw-NR | R3 | RP-182886 | Ericsson |
| 800089 | Even further mobility enhancement in E-UTRAN | LTE-feMob | R2 | RP-190921 | China Telecom |
| 830086 | Introduction of GSM, UTRA, E-UTRA and NR capability set(s) (CS(s)) to the multi-standard radio (MSR) specifications | MSR-GSM-UTRA-LTE-NR | R4 | RP-190642 | Ericsson |
| 790003 | Enhancements to IMS for new real time communication services | enIMS | S1 | SP-180137 | China Mobile, Hui Cai |
| 820028 | Enhancement of 5G PCC related services | en5GPccSer | C3 | CP-183246 | Xiaoyun Zhou, Huawei |
| 820039 | Signalling Improvements for Network Efficiency in 5GS | SINE-5G | C1 | CP-190142 | Shu, Lin |
| 790001 | New Services and Markets Technology Enablers – Phase 2 | SMARTER-Ph2 | S1 | SP-180589 | Li, Alice, Vodafone |
| 780055 | 5G Voice Service Continuity | 5GVSC | S1 | SP-171077 | Qun Wei, China Unicom |
| 790015 | Security Assurance Specification for 5G | SCAS-5G | S3 | SP-180689 | Marcus Wong, Huawei Technologies |
| 840069 | 5GS Transfer of Policies for Background Data Transfer | xBDT | S2 | SP-180509 | Takeshi Usui KDDI |
| 770024 | EVS Codec Extension for Immersive Voice and Audio Services | IVAS-Codec | S4 | SP-170611 | Gibbs, Jon, Huawei Technologies Co Ltd |
| 800002 | Alternative EVS implementation using updated fixed-point basic operators | Alt-FX-EVS | S4 | SP-180286 | Pawate, Raj, Cadence Design Systems Inc |
| 800009 | Policy delivery to UE for background data transfer | PDBDT | S1 | SP-180324 | Takatsugu Kito, KDDI |
| 790004 | QoS Monitoring | QoS-MON | S1 | SP-180141 | Amar Deol |
| 800005 | Inter-RAT Mobility requirements for real time service | MOBRT | S1 | SP-180592 | Yang Xu, Oppo |
| 760058 | Management of QoE measurement collection | QOED | S5 | SP-181069 | Petersen, Robert, Ericsson |
| 810025 | Network policy management for mobile networks based on NFV scenarios | NETPOL | S5 | SP-180821 | China Mobile Hao Zhang |
| 810027 | Intent driven management service for mobile network | IDMS-MN | S5 | SP-180899 | Zou Lan, Huawei |
| 810026 | Methodology for 5G management specifications | METHOGY | S5 | SP-180822 | Groenendijk, Jan, Ericsson |
| 820030 | Nchf Online and Offline charging services | OFSBI-CH | S5 | SP-181066 | Zhu Lei, Huawei |
| 820031 | Charging Enhancement of 5GC interworking with EPC | 5GIEPC-CH | S5 | SP-181067 | Chen Shan, Huawei |
| 820032 | NRM enhancements | eNRM | S5 | SP-190140 | Jing Ping, Nokia |
| 820033 | Network Exposure Charging in 5G System Architecture | 5GS-Ph1-NEFCH | S5 | SP-181070 | Tornkvist, Robert, Ericsson |
| 820034 | Charging AMF in 5G System Architecture Phase 1 | 5GS-Ph1-AMFCH | S5 | SP-181071 | Gardella, Maryse, Nokia |
| 820035 | Discovery of management services in 5G | 5GDMS | S5 | SP-181072 | Attila Horvat, Huawei Technologies Sweden AB |
| 820036 | Trace Management in the context of Services Based Management Architecture | TM-SBMA | S5 | SP-181073 | Andrianov, Anatoly, Nokia |
| 820041 | Stage-3 5GS NAS protocol development | 5GProtoc16 | C1 | CP-183087 | Sedlacek, Ivo, Ericsson |
| 840018 | S6b Optional for ePDG connected to 5GS | 5GS-S6b-Optional | S2 | SP-190447 | Irfan Ali, Cisco |
| 840002 | Service Based Interface Protocol Improvements | SBIProtoc16 | ct | CP-191060 | Yue Song, China Mobile |

I notice two items from SA6 are missing in your list 1) enh2MCPTT 2) eMCData2.

For MBMSAPI_MCS, Ling Zhang has changed affiliation to CATT.

Suresh