| 3GPP TS 23.482 V0.2.0 (2024-08) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Functional architecture and information flows for AIML Enablement Service; (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

Introduction	7

1	Scope	8

2	References	8

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	9

3.3	Abbreviations	9

4	Architectural requirements	9

4.1	General requirements	9

4.2	AIML capability related requirements	9

5	Application architecture for enabling AI/ML services	10

5.1	General	10

5.2	Application enablement architecture	10

5.2.1	On-Network AIML Enablement (AIMLE) Functional Architecture	10

5.2.1.1	Service-based AIMLE architecture representation	11

5.2.2	Off-Network AIMLE Functional Architecture	12

5.2.3	Functional Entities Description	12

5.2.3.1	General	12

5.2.3.2	AIMLE client	12

5.2.3.3	AIMLE server	12

5.2.3.4	ML repository	12

5.2.4	Reference Points Description	13

5.2.4.1	General	13

5.2.4.2	AIML-UU	13

5.2.4.3	AIML-S	13

5.2.4.4	AIML-C	13

5.2.4.5	AIML-R	13

5.2.4.6	AIML-E	13

5.2.4.7	AIML-PC5	13

6	AIMLE Functional Description	13

6.1	Support for FL member registration	13

6.2	Support for FL events subscription and notification	13

6.x	<Functionality>	14

7	Identities and commonly used values	14

7.1	General	14

7.2	<name>	14

8	Procedures and information flows	14

8.1	General	14

8.2	ML model retrieval	14

8.2.1	General	14

8.2.2	Procedure	14

8.2.2.1	General	14

8.2.2.2	ML model retrieval	15

8.2.2.3	ML model retrieval subscription	15

8.2.2.3.1	General	15

8.2.2.3.2	Subscribe	15

8.2.2.3.3	Notify	16

8.2.2.3.4	Subscription update	17

8.2.2.3.5	Unsubscribe	17

8.2.3	Information flows	18

8.2.3.1	ML model retrieval request	18

8.2.3.2	ML model retrieval response	18

8.2.3.3	ML model retrieval subscribe request	18

8.2.3.4	ML model retrieval subscription response	19

8.2.3.5	ML model retrieval notification	19

8.2.3.6	ML model retrieval subscription update request	19

8.2.3.7	ML model retrieval subscription update response	20

8.2.3.8	ML model retrieval unsubscribe request	20

8.2.3.9	ML model retrieval unsubscribe response	20

8.3	ML model training	21

8.3.1	General	21

8.3.2	Procedure for ML model training	21

8.3.3	Information flows	21

8.3.3.1	ML model training request	21

8.3.3.2	ML model training response	22

8.4	Support for FL member registration	22

8.4.1	General	22

8.4.2	Procedure on FL member registration	23

8.4.3	Procedure on FL member registration update	23

8.4.4	Information flows	24

8.4.4.1	General	24

8.4.4.2	FL member registration request	24

8.4.4.3	FL member registration response	24

8.4.4.4	FL member registration update request	24

8.4.4.5	FL member registration update response	25

8.5	Support for FL events subscription and notification	25

8.5.1	General	25

8.5.2	Procedure on subscription for FL related events	25

8.5.3	Procedure on FL related event notification	26

8.5.4	Definition of FL-related Events	27

8.5.5	Information flows	27

8.5.5.1	General	27

8.5.5.2	FL-related event subscription request	27

8.5.5.3	FL-related event subscription response	28

8.5.5.4	FL related event notification	28

8.5.5.5	FL-related event notification acknowledgement	29

8.6	Support AI/ML Task Transfer	29

8.6.1	General	29

8.6.2	Procedures for AI/ML task transfer	29

8.6.2.1	Request AIMLE Server to assist AI/ML task transfer	29

8.6.2.2	Request target AI/ML member for AI/ML task transfer	30

8.6.2.3	Direct AI/ML task transfer	31

8.6.3	Information flows	32

8.6.3.1	General	32

8.6.3.2	AI/ML task transfer assist request	32

8.6.3.3	AI/ML task transfer assist response	32

8.6.3.4	AI/ML task transfer request	32

8.6.3.5	AI/ML task transfer response	33

8.6.3.6	Direct AI/ML task transfer request	33

8.6.3.7	Direct AI/ML task transfer Response	33

8.7	AIMLE client registration	34

8.7.1	General	34

8.7.2	Procedure	34

8.7.3	Information flows	34

8.7.3.1	AIMLE client registration request	34

8.7.3.2	AIMLE client registration response	35

8.8	AIMLE client discovery	35

8.8.1	General	35

8.8.2	Procedure	35

8.8.2.1	AIMLE client discovery	35

8.8.3	Information flows	36

8.8.3.1	AIMLE client discovery request	36

8.8.3.2	AIMLE client discovery response	37

8.9	AIMLE client selection	37

8.9.1	General	37

8.9.2	Procedure	37

8.9.2.1	AIMLE client selection	37

8.9.3	Information flows	39

8.9.3.1	AIMLE client selection request	39

8.9.3.2	AIMLE client selection response	39

8.10	AIMLE client participation	39

8.10.1	General	39

8.10.2	Procedure	40

8.10.2.1	AIMLE client participation	40

8.10.3	Information flows	40

8.10.3.1	AIMLE client participation request	40

8.10.3.2	AIMLE client participation response	40

8.11	ML model management	41

8.11.1	General	41

8.11.2	ML model information storage	41

8.11.3	ML model information discovery	41

8.11.4	Information flows	42

8.11.4.1	ML model information storage request	42

8.11.4.2	ML model information storage response	43

8.11.4.3	ML model information discovery request	44

8.11.4.4	ML model information discovery response	44

8.12	HFL training	45

8.12.1	General	45

8.12.2	Procedure	45

8.12.3	Information flows	46

8.12.3.1	HFL training subscription request	46

8.12.3.2	HFL training subscription response	47

8.12.3.3	HFL training subscription notification	47

8.13	AIMLE client selection subscription and notification	47

8.13.1	General	47

8.13.2	Procedure	48

8.13.3	Information flows	48

8.x	<procedure name>	49

8.x.1	General	49

8.x.2	Procedure	49

8.x.3	Information flows	49

9	AIMLE APIs	49

9.1	AIMLE AI/ML Task Transfer API	49

Annex A (informative): Deployment scenarios	50

A.1	General	50

A.2	Deployment model #1: Cloud-deployed AIMLE server	50

A.3	Deployment model #2: Edge-deployed AIMLE server	50

A.4	Deployment model #3: Hierarchical AIMLE server deployment	51

Annex B (informative): Business Scenarios	52

Annex C (informative): Role of AIMLE in ML Model Lifecycle	52

C.1	 General	52

C.2	 Role#1 of AIMLE in ML Model Lifecycle	53

C.3	 Role#2 of AIMLE in ML Model Lifecycle	53

Annex D: Change history	54

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

Data analytics is a useful tool for the operator to help optimizing the service offering by predicting events related to the network or slice or UE conditions. 3GPP introduced data analytics function (NWDAF) [2] to support network data analytics services in 5G Core network, management data analytics service (MDAS) [3] to provide data analytics at the OAM, and application data analytics service (ADAES) [4].

In this direction, the support for AI/ML services in 3GPP core network has been studied for providing AI/ML enabled analytics in NWDAF, as well as for assisting the ASP/3rd party AI/ML application service provider for the AI/ML model distribution, transfer, training for various applications (e.g., video/speech recognition, robot control, automotive).

Considering vertical-specific applications and edge applications as the major consumers of 3GPP-provided data analytics services, the AIML enablement (AIMLE) service plays role on the exposure of AI/ML services from different 3GPP domains to the vertical/ASP in a unified manner on top of 3GPP core network and OAM; and on defining, at a SEAL layer, value-add support services for assisting AI/ML services provided by the VAL layer, while being complementary to AI/ML support solutions provided in other 3GPP domains.

This technical specification provides architecture and procedures for enabling AIMLE service over 3GPP networks.

# 1	Scope

The present document specifies the procedures and information flows necessary for AIML Enablement SEAL Service.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[3]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics".

[4]	3GPP TS 23.436: "Functional architecture and information flows for Application Data Analytics Enablement Service".

[5]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[6]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[7]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[8]	3GPP TS 28.105: "Management and orchestration; Artificial Intelligence/ Machine Learning (AI/ML) management".

[9]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

# 3	Definitions of terms, symbols and abbreviations

This clause and its three subclauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

ML model: According to 3GPP TS 28.105 [8], mathematical algorithm that can be "trained" by data and human expert input as examples to replicate a decision an expert would make when provided that same information.

ML model lifecycle: The lifecycle of an ML model aka ML model operational workflow consists of a sequence of ML operations for a given ML task / job (such job can be an analytics task or a VAL automation task). This definition is aligned with the 3GPP definition on ML model lifecycle according to 3GPP TS 28.105 [8].

ML model training: According to 3GPP TS 28.105 [8], ML model training includes capabilities of an ML training function or service to take data, run it through an ML model, derive the associated loss and adjust the parameterization of that ML model based on the computed loss.

ML model inference: According to 3GPP TS 28.105 [8], ML model inference includes capabilities of an ML model inference function that employs an ML model and/or AI decision entity to conduct inference.

AIMLE service: An AIMLE service is an AIMLE capability which aims assisting in performing or enabling one or more AIML operations.

AI/ML client: an application layer entity (also referred as ML client) which is an AI/ML endpoint, and performs client-side operations (e.g. related to the ML model lifecycle). Such AI/ML client can be a VAL client or AIML enabler client and may be configured e.g., to provide ML model training and inference locally e.g., at the VAL UE side.

AIMLE client set identifier: an identifier of the set of selected AIMLE clients.

AI/ML server: an application layer entity which is an AI/ML endpoint, and performs server-side operations (e.g. related to the ML model lifecycle). Such AI/ML server can be a VAL server or AIML enablement server.

FL member: An FL member or participant is an entity which has a role in the FL process. An FL member can be an FL client performing ML model training, or an FL server performing aggregation/collaboration for the FL process.

FL client: An FL member which locally trains the ML model as requested by the FL server. Such FL client functionality can be at the network (e.g. AIMLE server with FL client capability) or at the device side (e.g. AIMLE client with FL client capability).

FL server: An FL member which generates global ML model by aggregating local model information from FL clients.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	Architectural requirements

## 4.1	General requirements

[AR-4.1-a]	The AI/ML enablement layer shall be able to support one or more VAL applications.

[AR-4.1-b]	Supported AI/ML enablement capabilities shall be offered as APIs to the VAL applications.

[AR-4.1-c]	The AIML enablement layer shall support interaction with 3GPP network system to consume network and AI/ML support services.

[AR-4.1-d]	The AIMLE client shall be capable to communicate with one or more AIMLE servers of the same AIMLE service provider.

## 4.2	AIML capability related requirements

[AR-4.2-a]	The AIMLE server shall be capable of provisioning and exposing ML client information.

[AR-4.2-b]	The AIMLE server shall be capable of supporting the registration, discovery, and selection of AIMLE clients which participate as ML members in AIML service lifecycle.

[AR-4.2-c]	The AIMLE layer shall be capable of supporting ML service lifecycle operations (e.g., ML model training).

[AR-4.2-d]	The AIMLE server shall be capable of supporting discovery and provisioning of AIML models.

# 5	Application architecture for enabling AI/ML services

## 5.1	General

The functional architecture enhancements for the AIML enablement service are based on the generic functional model specified in clause 6.2 of 3GPP TS 23.434 [5]. The architecture enhancements are organized into functional entities to describe a functional architecture enhancement which addresses the support for AIML enablement aspects for vertical applications.

## 5.2	Application enablement architecture

### 5.2.1	On-Network AIML Enablement (AIMLE) Functional Architecture

Figure 5.2.1-1: On-network AIMLE functional model

Figure 5.2.1-1 illustrates the on-network functional model of AIMLE. In the vertical application layer, the VAL client communicates with the VAL server over VAL-UU reference point. VAL-UU supports both unicast and multicast delivery modes. The AIMLE functional entities on the UE and the server are grouped into AIMLE client(s) and AIMLE server(s) respectively.

The AIMLE includes of a common set of services for comprehensive enablement of AIML functionality, including federated and distributed learning (e.g., FL client registration management, FL client discovery and selection), and reference points. The AIMLE services are offered to the vertical application layer (VAL).

The AIMLE client communicates with the AIMLE server(s) over the AIML-UU reference points. The AIMLE client provides functionality to the VAL client(s) over AIML-C reference point. The VAL server(s) communicate with the AIMLE server(s) over AIML-S reference points. The AIMLE servers communicate with the underlying 3GPP network systems using the respective 3GPP interfaces specified by the 3GPP network system. AIML-E reference point enables interactions between two AIMLE servers (e.g. central and edge AIMLE servers).

The AIMLE server interacts with the ML repository which serves as repository for ML model and ML participants over AIML-R.

#### 5.2.1.1	Service-based AIMLE architecture representation

Figure 5.2.1.1-1 exhibits the service-based interfaces for providing and consuming AIMLE services. The AIMLE server could provide service to VAL server and AIMLE client through interface SAiml.

Figure 5.2.1.1-1: Architecture for AIML enablement – Service based representation.

Figure 5.2.1.1-2 illustrates the service-based representation for utilization of the 5GS network services based on the 5GS SBA specified in 3GPP TS 23.501 [6].

Figure 5.2.1.1-2: Architecture for AIMLE utilizing the 5GS network services based on the 5GS SBA – Service based representation,

The AIMLE server as well as ADAES is deployed as a SEAL server; hence enhancements to SEAL architecture (as specified in 3GPP TS 23.434 [5]) are needed to incorporate the AIMLE service. Figure 5.2.1.1-3 illustrates the service-based representation including AIMLE server as part of the SEAL framework.

Figure 5.2.1.1-3: SEAL functional model representation using service-based interfaces and including AIMLE function.

### 5.2.2	Off-Network AIMLE Functional Architecture

Figure 5.2.2-1: Off-network AIMLE functional model

Figure 5.2.2-1 illustrates the off-network (UE-to-UE) functional model of AIML enablement. In the vertical application layer, the VAL client communicates with a further VAL client over VAL-PC5 reference point. VAL-PC5 supports both unicast and multicast delivery modes. The UE1, if connected to the network via Uu reference point, can also act as a UE-to-network relay, to enable UE2 to access the VAL server(s) over the VAL-UU reference point.

The AIMLE client communicates with a further AIMLE client(s) over the AIML-PC5 reference points. The AIMLE client provides functionality to the VAL client(s) over AIML-C reference point. Such communication is performed for supporting local ML operations (training, distribution, inference) in a coordinated manner.

The off network functional architecture is similar to SEAL off-network architecture (as specified in 3GPP TS 23.434 [5]).

### 5.2.3	Functional Entities Description

#### 5.2.3.1	General

Each subclause is a description of a functional entity and does not imply a physical entity.

#### 5.2.3.2	AIMLE client

The AIMLE client functional entity acts as the application client supporting AIMLE services. It interacts with the AIMLE server.

#### 5.2.3.3	AIMLE server

The AIMLE server functional entity provides AIMLE services supported within the vertical application layer. It interacts with the AIMLE client, the 3GPP network, other SEAL services, and VAL server.

#### 5.2.3.4	ML repository

The ML repository is a logical entity that serves both as a registry for ML/FL members and as a repository for application layer ML model related information. It can be accessed by the AIMLE server.

### 5.2.4	Reference Points Description

#### 5.2.4.1	General

The reference points for the functional model related to AIMLE are described in the following subclauses.

#### 5.2.4.2	AIML-UU

The interactions related to AIML enablement functions between the AIMLE client and AIMLE server are supported by AIML-UU reference point. This reference point utilizes Uu reference point as described in 3GPP TS 23.401 [7] and 3GPP TS 23.501 [6].

#### 5.2.4.3	AIML-S

The interactions related to AIML enablement functions between the VAL server(s) and the AIMLE server are supported by AIML-S reference point.

#### 5.2.4.4	AIML-C

The interactions related to AIML enablement functions between the VAL client(s) and the AIMLE client within a VAL UE are supported by AIML-C reference point.

#### 5.2.4.5	AIML-R

The interactions related to AIML enablement functions between the AIMLE server, and the ML repository are supported by AIML-R reference point.

#### 5.2.4.6	AIML-E

The interactions related to AIML enablement functions between AIMLE servers (e.g., central and edge AIMLE servers).

#### 5.2.4.7	AIML-PC5

The interactions related to AIML enablement functions between AIMLE clients in off-network deployments.

# 6	AIMLE Functional Description

Clauses under this clause shall document the functionalities of the AIMLE layer.

## 6.1	Support for FL member registration

This functionality covers the registration and registration update of the candidate FL member to the ML repository which is keeping the FL member registrations. Such candidate member can be a VAL server functionality or an enabler layer functionality (e.g. AIMLE server) which is registering to the ML repository/registry to act as FL member for a given application event (analytics event or event triggered by a VAL layer application server).

## 6.2	Support for FL events subscription and notification

This functionality enables a consumer (who can be the AIMLE server or a VAL server e.g. acting as FL server) to subscribe for FL related events and getting notified on changes on the availability of the FL members which are to be used for the FL-related task (e.g., training). This capability at the ML repository acting as an AIML service registry supports the subscription for events related to FL members and the notification to the consumer in case of changes. This feature assumes that such FL members (AIMLE or VAL server or AIMLE clients) have previously registered to this registry their availability and capabilities.

## 6.x	<Functionality>

Add a copy of this clause for each functionality, adding a title and description.

# 7	Identities and commonly used values

Describe the required identities and commonly used values in clauses under this clause. If need be, before approval of the TS, this clause can be merged with the 'Application architecture' clause.

## 7.1	General

## 7.2	<name>

Add a copy of this clause for each identity or a commonly used value, adding a title and description.

# 8	Procedures and information flows

Subclause of this clause shall describe the procedures for the application architecture for enabling AIML services.

## 8.1	General

## 8.2	ML model retrieval

### 8.2.1	General

ML model retrieval enable applications consuming services from the AI/ML application enablement layer to retrieve ML models available with the enablement layer.

ML model retrieval enables an AIMLE client or VAL server to obtain ML models of interest. The retrieval of ML models is based on matching ML model retrieval filters provided in the request.

ML model retrieval subscription enables an AIMLE client or VAL server to receive notifications about ML models of interest. The subscription is based on matching ML model retrieval filters provided in the subscription request.

### 8.2.2	Procedure

#### 8.2.2.1	General

Following procedures are supported for ML model retrieval:

-	ML model retrieval request-response procedure;

-	ML model subscribe-notify procedures for retrieval of ML models, comprising:

-	Subscription procedure;

-	Subscription update procedure; and

-	Unsubscribe procedure;

#### 8.2.2.2	ML model retrieval

Figure 8.2.2.2-1 illustrates the procedure for an AIMLE client or VAL server to discover and obtain ML models from the AIMLE server.

Pre-conditions:

- The AIMLE client or VAL server has received information (e.g., URI, IP address) related to the AIMLE Server.

- The AIMLE client or VAL server has received security credentials authorizing it to communicate with the AIMLE server.

Figure 8.2.2.2-1: ML model retrieval

1.	The requestor (e.g., AIMLE client or VAL server) sends a ML model retrieval request to the AIMLE server. The request includes the requestor identifier along with security credentials and may include ML model retrieval filters.

2.	Upon receiving the request from the requestor, the AIMLE server validates if the requestor is authorized for the request.

If the requestor is authorized, the AIMLE server may determine if the requested ML model is available with the AIMLE server based on ML model retrieval filters.

Editor’s Note: Details of AIMLE server interactions with the ML model repository are FFS.

3.	The AIMLE server sends a ML model retrieval response to the requestor. If the AIMLE server has determined ML models (e.g., locally, or from the ML repository), the response includes an indication of success and may include the ML models. Otherwise, the response includes an indication of failure and may include a reason for failure.

Upon receiving a response including ML models, the requestor can store the ML models and may provide the ML model to the VAL client(s) if the requestor is the AIMLE client.

#### 8.2.2.3	ML model retrieval subscription

##### 8.2.2.3.1	General

Clause 8.2.2.3.2 and clause 8.2.2.3.3 together illustrate the ML model retrieval subscribe-notify ML model.

Clause 8.2.2.3.4 illustrates the ML model retrieval subscription update procedure.

Clause 8.2.2.3.5 illustrates the ML model retrieval unsubscribe procedure.

##### 8.2.2.3.2	Subscribe

Figure 8.2.2.3.2-1 illustrates the procedure for an AIMLE client or VAL server to subscribe with the AIMLE server to be notified of retrieval of ML models.

Pre-conditions:

- The AIMLE client or VAL server has received information (e.g. URI, IP address) related to the AIMLE Server.

- The AIMLE client or VAL server has received security credentials authorizing it to communicate with the AIMLE server.

Figure 8.2.2.3.2-1: ML model retrieval subscription

1.	The requestor (e.g., AIMLE client or VAL server) sends a ML model retrieval subscribe request to the AIMLE server. The request includes the requestor identifier, security credentials and may include ML model retrieval filters and expiration time.

2.	Upon receiving the request from the requestor, the AIMLE server validates if the requestor is authorized for the request. If the requestor is authorized, the AIMLE server creates the subscription and stores the subscription information.

3.	The AIMLE server sends a ML model retrieval subscribe response to the requestor. If the AIMLE server has created the subscription, the response includes an indication of success, the subscription identity and may include an expiration time; to maintain the subscription, the requestor shall send a subscription update request before the expiration time, otherwise the ML model retrieval subscription expires. If the AIMLE server has not created the subscription, the response includes an indication of failure and may include a reason for failure.

##### 8.2.2.3.3	Notify

Figure 8.2.2.3.3-1 illustrates the ML model retrieval  notify operation between the AIMLE server and an AIMLE client or VAL server.

Pre-conditions:

- The AIMLE client or VAL server has subscribed for ML model retrieval with the AIMLE Server.

Figure 8.2.2.3.3-1: ML model retrieval notification

1.	An event occurs at the AIMLE server that satisfies trigger conditions for notifying a subscriber (e.g., AIMLE client or VAL server). The event may be the detection of availability of new ML model(s) (e.g., new ML model or new revision of a ML model) that satisfies the ML model retrieval filters of the subscription.

Editor’s Note: Details of AIMLE server interactions with the ML model repository to detect trigger conditions are FFS.

2.	The AIMLE server sends a ML model retrieval notification to the requestor indicating newly available ML models. The notification includes a subscription identity and newly available ML model(s).

##### 8.2.2.3.4	Subscription update

Figure 8.2.2.3.4-1 illustrates the procedure for an AIMLE client or VAL server to update a subscription with the AIMLE server.

Pre-conditions:

- The AIMLE client or VAL server has subscribed for ML model retrieval with the AIMLE Server.

Figure 8.2.2.3.4-1: ML model retrieval subscription update

1.	The requestor (e.g., AIMLE client or VAL server) sends a ML model retrieval update request to the AIMLE server. The request includes the requestor identifier, security credentials and the subscription identifier and may include ML model retrieval filters and expiration time.

2.	Upon receiving the request from the requestor, the AIMLE server validates if the requestor is authorized for the request. If the requestor is authorized, the AIMLE server updates the subscription information.

3.	The AIMLE server sends a ML model retrieval subscription update response to the requestor. If the AIMLE server has updated the subscription, the response includes an indication of success and may include an expiration time. To maintain the subscription, the requestor shall send a subscription update request before the expiration time, otherwise the ML model retrieval subscription expires. If the AIMLE server has not updated the subscription, the response includes an indication of failure and may include a reason for failure.

##### 8.2.2.3.5	Unsubscribe

Figure 8.2.2.3.5-1 illustrates the procedure for an AIMLE client or VAL server to unsubscribe with the AIMLE server.

Pre-conditions:

- The AIMLE client or VAL server has subscribed for ML model retrieval with the AIMLE Server.

Figure 8.2.2.3.5-1: ML model retrieval unsubscribe

1.	The requestor (e.g., AIMLE client or VAL server) sends a ML model retrieval unsubscribe request to the AIMLE server. The request includes the requestor identifier, security credentials and the subscription identifier.

2.	Upon receiving the request from the requestor, the AIMLE server validates if the requestor is authorized for the request. If the requestor is authorized, the AIMLE server cancels the subscription.

3.	The AIMLE server sends a ML model retrieval unsubscribe response to the requestor. If the AIMLE server has canceled the subscription, the response includes an indication of success. If the AIMLE server has not canceled the subscription, the response includes an indication of failure and may include a reason for failure.

### 8.2.3	Information flows

##### 8.2.3.1	ML model retrieval request

Table 8.2.3.1-1 describes information elements for the ML model retrieval request from the AIMLE client or VAL server to the AIMLE server.

Table 8.2.3.1-1: ML model retrieval request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., AIMLE client or VAL server). |
| Security credentials | M | The security credentials of the requestor. |
| Filtering criteria | O | Represents the filtering criteria, which can be any of the ML model information. |
| > ML model identifier | O | The identifier of the ML model. |
| > Analytics identifier | O | The identifier of the analytics. |

Editor’s Note: The alignment between ML model retrieval request and ML model information discovery request is FFS

##### 8.2.3.2	ML model retrieval response

Table 8.2.3.2-1 describes information elements for the ML model retrieval response from the AIMLE server.

Table 8.2.3.2-1: ML model retrieval response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the ML model retrieval request was successful. |
| List of ML models | O | List of retrieved ML models. |
| > ML model identifier | O | The ML model identifier. |
| > ML model retrieval endpoint (NOTE) | O | The endpoint (e.g., URL, URI, IP address) where the ML model file can be retrieved. |
| > ML model (NOTE) | O | The ML model containing a retrieved ML model. |
| Failure response | O | Indicates that the request has failed. |
| > Cause | O | The cause for the request failure. |
| NOTE: At least one of these elements is needed. |  |  |

Editor’s Note: The alignment between ML model retrieval response and ML model information discovery response is FFS

##### 8.2.3.3	ML model retrieval subscribe request

Table 8.2.3.3-1 describes information elements for the ML model retrieval subscription request from the AIMLE client or VAL server to the AIMLE server.

Table 8.2.3.3-1: ML model retrieval subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., AIMLE client or VAL server). |
| Security credentials | M | The security credentials of the requestor. |
| Notification Target Address | M | Notification target address (e.g. URL) where the notifications should be sent. |
| Proposed expiration time | O | Proposed expiration time for the subscription. |
| Filtering criteria | O | Represents the filtering criteria, which can be any of the ML model information. |
| > ML model identifier | O | The identifier of the ML model. |
| > Analytics identifier | O | The analytics identifier of the analytics. |

Editor’s Note: The alignment between ML model retrieval request and ML model information discovery request is FFS.

##### 8.2.3.4	ML model retrieval subscription response

Table 8.2.3.4-1 describes information elements for the ML model retrieval subscription response from the AIMLE server.

Table 8.2.3.4-1: ML model retrieval subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the ML model retrieval request was successful. |
| > Subscription ID | M | Subscription identifier corresponding to the subscription. |
| > Expiration time | O | Indicates the expiration time of the subscription. To maintain an active subscription, a subscription update is required before the expiration time. |
| Failure response | O | Indicates that the request has failed. |
| > Cause | O | The cause for the request failure. |

##### 8.2.3.5	ML model retrieval notification

Table 8.2.3.5-1 describes information elements for the ML model retrieval subscription response from the AIMLE server.

Table 8.2.3.5-1: ML model retrieval notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription. |
| List of ML models | O | List of retrieved ML models. |
| > ML model identifier | O | The ML model identifier. |
| > ML model retrieval endpoint (NOTE) | O | The endpoint (e.g., URL, URI, IP address) where the ML model file can be retrieved. |
| > ML model (NOTE) | O | The ML model containing a retrieved ML model. |
| NOTE: At least one of these elements is needed. |  |  |

Editor’s Note: The alignment between ML model retrieval notification and ML model information discovery response is FFS

##### 8.2.3.6	ML model retrieval subscription update request

Table 8.2.3.6-1 describes information elements for the ML model retrieval subscription update request from the AIMLE client or VAL server to the AIMLE server.

Table 8.2.3.6-1: ML model retrieval subscription update request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., AIMLE client or VAL server). |
| Security credentials | M | The security credentials of the requestor. |
| Subscription ID | M | Subscription identifier corresponding to the subscription to be updated. |
| Notification Target Address | O | Notification target address (e.g. URL) where the notifications should be sent. |
| Proposed expiration time | O | Proposed expiration time for the subscription. |
| Filtering criteria | O | Represents the filtering criteria, which can be any of the ML model information. |
| > ML model identifier | O | The identifier of the ML model. |
| > Analytics identifier | O | The analytics identifier of the analytics. |

Editor’s Note: The alignment between ML model retrieval subscription update request and ML model information discovery request is FFS

##### 8.2.3.7	ML model retrieval subscription update response

Table 8.2.3.7-1 describes information elements for the ML model retrieval subscription update response from the AIMLE server.

Table 8.2.3.7-1: ML model retrieval subscription update response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the ML model retrieval subscription update request was successful. |
| > Expiration time | O | Indicates the expiration time of the subscription. To maintain an active subscription, a subscription update is required before the expiration time. |
| Failure response | O | Indicates that the request has failed. |
| > Cause | O | The cause for the request failure. |

##### 8.2.3.8	ML model retrieval unsubscribe request

Table 8.2.3.8-1 describes information elements for the ML model retrieval unsubscribe request from the AIMLE client or VAL server to the AIMLE server.

Table 8.2.3.8-1: ML model retrieval unsubscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., AIMLE client or VAL server). |
| Security credentials | M | The security credentials of the requestor. |
| Subscription ID | M | Subscription identifier corresponding to the subscription to be unsubscribed. |

##### 8.2.3.9	ML model retrieval unsubscribe response

Table 8.2.3.9-1 describes information elements for the ML model retrieval unsubscribe response from the AIMLE server.

Table 8.2.3.9-1: ML model retrieval unsubscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the ML model retrieval unsubscribe request was successful. |
| Failure response | O | Indicates that the request has failed. |
| > Cause | O | The cause for the request failure. |

## 8.3	ML model training

### 8.3.1	General

This clause describes the procedure for supporting the ML model training by the AIMLE server. The VAL server requests the AIMLE server for the ML model training. VAL server requests the AIMLE server to train a ML model by specifying the type of learning, details about the ML model to be trained or requirements to select a ML model for training and criteria for selecting the members who can participate in the ML model training.

### 8.3.2	Procedure for ML model training

Figure 8.3.2-1 illustrates the procedure for AIMLE server to support ML model training based on the request from the VAL server.

Figure 8.3.2-1: ML model training

1.	The VAL server sends an ML model training request to AIMLE server, requesting to assist in its ML model training. This request consists of ML model information or ML model requirement information, etc.

2.	The AIMLE server checks whether the VAL server is authorized to perform the ML model training request. If no model information is provided but only the model requirement information is provided in step 1, the AIMLE server identifies and selects the appropriate ML model for training based on the ML model requirement information.

Editor’s Note: Adding a reference to the procedure for ML model discovery and selection is FFS.

3.	If the VAL server is authorized, AIME server returns the success response, otherwise a failure response indication the reason for failure.

### 8.3.3	Information flows

#### 8.3.3.1	ML model training request

Table 8.3.3.1-1 describes the information flow from the VAL server to the AIMLE server for requesting the ML model training.

Table 8.3.3.1-1: ML model training request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | Identity of the VAL server performing the request. |
| Training type | M | Identifies whether the VFL or HFL training to be performed. |
| List of member clients | O (NOTE 1) | List of member clients to be utilized for training the ML model. |
| Member selection criteria | O (NOTE 1) | Identifies the criteria that needs to be continuously monitored for selecting the member clients (e.g., AIMLE clients in a particular location, member availability duration, new member clients registering with the required capabilities). |
| ML model information | O (NOTE 2) | Identifies the ML model that has to be distributed to the selected member clients for training. This information consists of the model identifier, address (e.g., a URL or an FQDN) of the ML model file or address of the model repository where the ML model resides |
| ML model requirement information | O (NOTE 2) | Identifies the requirement for selecting a model to be trained |
| Members update notification | O | Indicates whether requestor needs to be notified whenever there is update related to new member clients selected or de-selected. |
| NOTE 1: At least one of these IE shall be present. NOTE 2: At least one of these IE shall be present. |  |  |

Editor's Note: Additional information elements which are generic and that are required for HFL and VFL training type are FFS.

#### 8.3.3.2	ML model training response

Table 8.3.3.2-1 describes the information flow of ML model training response from the AIMLE server to the VAL server.

Table 8.3.3.2-1: ML model training response

| Information element | Status | Description |
| --- | --- | --- |
| Success response | O (NOTE) | Indicates that the ML model training request was successful. |
| > ML model identifier | O | Identifies the ML model selected by AIMLE server for training. |
| Failure response | O (NOTE) | Indicates that the ML model training request was failure. |
| > Cause | M | Reason for the failure. |
| NOTE: At least one of these information elements shall be present |  |  |

## 8.4	Support for FL member registration

### 8.4.1	General

This clause provides the procedures for the registration and registration update of candidate FL members in the ML repository which serves as service registry for the FL members undertaking a task related to the ML model lifecycle, such as ML model local training. Candidate FL members can be application layer entities at the server side (e.g., VAL server, AIMLE server), which can potentially be selected as FL clients or FL server for a particular ASP /vertical requirement.

Editor’s note: It is FFS whether the VAL server is registering directly or indirectly via AIMLE server for FL-related events.

### 8.4.2	Procedure on FL member registration

Figure 8.4.2-1 illustrates the procedure where the registration of a candidate FL member happens via the ML repository, serving as AIML service registry.

Figure 8.4.2-1 Procedure for registration on FL member registry

1.	The candidate FL member (e.g., VAL server, AIMLE server) sends an FL member registration request to the ML repository for registering to the ML repository which acts as the AIML service registry.

2.	The ML repository validates the received request and generates the identity and other security related information for all the FL members listed in the registration request.

3.	The ML repository sends the generated information in the FL member registration response message to the candidate FL member.

### 8.4.3	Procedure on FL member registration update

Figure 8.4.3-1 illustrates the procedure where the registration update of a candidate FL member happens via the ML repository.

Figure 8.4.3-1: Procedure for registration update

1.	The candidate FL member (VAL server, AIMLE Server) sends an FL member registration update request to the ML repository acting as AIML service registry for updating the registration of the FL member.

2.	The ML repository validates the received update request and generates the identity and other security related information for the FL member.

3.	The ML repository sends the generated information in the FL member registration update response message to the candidate FL member.

### 8.4.4	Information flows

#### 8.4.4.1	General

The following information flows are specified for supporting the FL member registration and registration update based on clause 8.4.2 and clause 8.4.3.

#### 8.4.4.2	FL member registration request

Table 8.4.4.2-1 describes information elements for the FL member registration request from the candidate FL member (VAL server, AIMLE server) to the ML repository/registry.

Table 8.4.4.2-1: FL member registration request

| Information element | Status | Description |
| --- | --- | --- |
| Security information | M | Information for ML repository to validate the registration request. |
| FL member ID | M | The identifier of the candidate FL member. This can be the VAL server ID, the EAS ID, the AIMLE server ID based on which entity is the candidate FL member. |
| FL member supported AI/ML role | M | The supported AI/ML role of FL member which can be used as FL server or FL client. |
| FL member capabilities | O (NOTE) | FL member capability information (e.g. ML application type, allowed resource usage level). |
| FL member Availability schedule | O (NOTE) | Availability schedule of the FL member for a certain FL member. |
| Supported ML model ID list | O | The list of ML model IDs for which the FL member can be used. |
| Area of Interest | O | The area of interest for which the registration applies. |
| Time validity | O | The time validity for the registration of the FL member. |
| NOTE: At least one of these shall be present. |  |  |

#### 8.4.4.3	FL member registration response

Table 8.4.4.3-1 describes information elements for the FL member registration response to the candidate FL member (VAL server, AIMLE server) from the ML repository/registry.

Table 8.4.4.3-1: FL member registration response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the registration request (positive or negative acknowledgement). |
| Registration ID | M | The generated ID for the registration. |

#### 8.4.4.4	FL member registration update request

Table 8.4.4.4-1 describes information elements for the FL member registration update request from the candidate FL member (VAL server, AIMLE server) to the ML repository/registry.

Table 8.4.4.4-1: FL member registration update request

| Information element | Status | Description |
| --- | --- | --- |
| Security information | M | Information for ML repository to validate the registration update request |
| FL member ID | M | The identifier of the candidate FL member for which the update applies. This can be the VAL server ID, the EAS ID, the AIMLE server ID based on which entity is the candidate FL member. |
| Updated registration parameters | M | The updates of the registration parameters. |
| > FL member supported AI/ML role | O | The updated supported AI/ML role of FL member which can be FL server, FL client. |
| > FL member Availability schedule | O | Updated Availability schedule of the FL member. |
| > supported ML model ID list | O | The updated list of ML model IDs for which the FL member can be used |
| > area of interest | O | The updated area of interest for the registration update. |
| > time validity | O | The update time validity for the FL member registration update. |

#### 8.4.4.5	FL member registration update response

Table 8.4.4.5-1 describes information elements for the FL member registration update response to the candidate FL member (VAL server, AIMLE server) from the ML repository/registry.

Table 8.4.4.5-1: FL member registration update response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the registration update request (positive or negative acknowledgement). |
| Registration ID | O | The generated ID for the new registration based on the update (if needed). |

## 8.5	Support for FL events subscription and notification

### 8.5.1	General

This clause consists of the following procedures 1) the subscription for the FL related events in clause 8.5.2 and 2) the event notification procedure in clause 8.5.3.

This capability targets the enhancements to a global ML repository acting as AIML service registry which monitors the status and changes to the availability and capabilities of the FL members. The subscriber can be the AIMLE server or VAL server which requires to get notified on the FL member availability, since this information is vital for enabling the subscriber to perform the FL-related AIML operations (e.g. AIMLE server selecting FL members for a certain FL task).

Editor’s note: The procedure on supporting the subscription and events notification for a VAL server being the FL member is FFS.

### 8.5.2	Procedure on subscription for FL related events

This procedure, as illustrated in Figure 8.5.2-1, describes the subscription for events related to FL member availability.

Pre-conditions:

- The AIMLE server has the authorization to subscribe for the FL-related events (events described in clause 8.5.4).

Figure 8.5.2-1: Procedure for FL-related event subscription

1.	The AIMLE server sends an FL-related event subscription request to the ML repository to receive notification of FL related events and in particular the availability of FL member for a target area and time.

2.	Upon receiving the event subscription request from the AIMLE server, the ML repository checks for the relevant authorization for the event subscription.

3.	If the authorization is successful, the ML repository stores the subscription information.

4.	The ML repository sends an FL-related event subscription response to AIMLE server indicating successful operation.

### 8.5.3	Procedure on FL related event notification

This procedure describes the event notification procedure for the FL member availability. This can be triggered based on a change at the availability or capabilities of the candidate or selected FL members.

Pre-conditions:

- The subscription procedure in clause 8.5.2 was performed by the AIMLE server.

- The FL member has registered its capabilities or availability to the ML repository based on the support for FL registration capability.

Figure 8.5.3-1: Procedure for FL-related event notifications

1.	The ML repository detects a change related to the availability of an FL member and generates events to be consumed by the AIMLE server, based on the notification, or based on a received updated registration from FL member, who has already registered at the ML repository.

2.	For the generated event, the ML repository retrieves the list of corresponding subscriptions.

3.	The ML repository sends FL-related event notification to all entities (AIMLE servers) that have subscribed for the FL-related event matching the criteria. If a notification reception information is available as part of the FL-related event subscription, then the notification reception information is used by the ML repository to send event notifications to the corresponding AIMLE servers.

4.	The notified AIMLE server (s) send an event notification acknowledgement to the ML repository.

### 8.5.4	Definition of FL-related Events

The definition of the FL related events is provided in Table 8.5.4-1.

Table 8.5.4-1: Definition of FL related events

| Events | Events Description |
| --- | --- |
| Availability changes of FL member | The event type relates to the availability change of a FL member. Such availability change can be an indication of availability or unavailability for a given service area, or for a given ML model ID/profile. |
| FL model information | Information such as accuracy, time schedule and latency for the FL training. Latency is considering the target latency, i.e., when the FL model training shall be completed. |

Editor’s note: The definition of further FL-related events is FFS.

### 8.5.5	Information flows

#### 8.5.5.1	General

The following information flows are specified for supporting the FL events subscription and events notification based on clause 8.5.2 and clause 8.5.3.

#### 8.5.5.2	FL-related event subscription request

Table 8.5.5.2-1 describes information elements for FL-related event subscription request from the subscriber (AIMLE server) to the ML repository.

Table 8.5.5.2-1: FL-related event subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor Identifier | M | The identifier to determine the identity of the requestor entity. This can be the VAL server ID. |
| Security Credentials | M | The security credentials of the requestor. |
| Time of validity | M | The time validity for the subscription. |
| ML model information | M | Information of the ML model for which the subscription is needed. |
| >ML model ID | M | The identity of the ML model for which the subscription is needed. |
| FL member information | M | Information on the FL member (candidate or selected). |
| >FL member type | O (NOTE) | The type of FL member which can be an FL server or FL client. |
| >FL member ID | O (NOTE) | The ID of the entity serving as FL member. This can be used for events where the FL member is known and possibly selected, and the availability needs to be checked. |
| Event Information | M | Information on the FL-related event. |
| > Event ID | O | The identity of the FL-related event (if known by the requestor). |
| > Event type | M | The event type requested, based on the Definition of FL related events (shown in Table 8.5.4-1). |
| Notification endpoint | M | The information of the endpoint for receiving the notifications for the event. |
| NOTE: At least one of these shall be present. |  |  |

#### 8.5.5.3	FL-related event subscription response

Table 8.5.5.3-1 describes information elements for the FL-related event subscription response to the subscriber (e.g. AIMLE server) from the ML repository.

Table 8.5.5.3-1: FL-related event subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of the event subscription operation. |
| Subscription Identifier | M | The unique identifier for the event subscription. |

#### 8.5.5.4	FL related event notification

Table 8.5.5.4-1 describes information elements for the FL-related event notification from the ML repository to the requestor (e.g., AIMLE server).

Table 8.5.5.4-1: FL-related event notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription identifier | M | The unique identifier of the event subscription. |
| Event identifier | M | The unique identifier for the event. For the definition of events, refer to the Table 8.5.4-1 with the list of FL related events. |
| Event related information | M | The event related information (e.g., time at which the event originated or is planned to be executed, location of event). |
| Event content | M | The content of the event information. |
| > List of FL member ID /address | O | The list of the FL members and their addresses. |
| >> FL member enter / leave | O | The indication of an FL member entering or leaving the available list. |
| >> FL member updated capability | O | The update of capabilities for the FL members. |
| >> Time availability of FL member | O | The time period for which the FL member is available. |
| >> Area of interest | O | The area of interest or exact or predicted location for which the event applies. |

#### 8.5.5.5	FL-related event notification acknowledgement

Table 8.5.5.5-1 describes information elements for the FL-related event notification acknowledgement from the AIMLE server to the ML repository.

Table 8.5.5.5-1: FL-related event notification acknowledgement

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The positive or negative acknowledgement of the notification reception. |

## 8.6	Support AI/ML Task Transfer

This clause describes procedure for supporting AI/ML task transfer.

### 8.6.1	General

The following clauses specify procedures to support AI/ML task transfer.

### 8.6.2	Procedures for AI/ML task transfer

Due to various reasons (e.g., changes of available resource, changes of available time), an AI/ML member (e.g., AIMLE Client, VAL Client) finds itself cannot finish the assigned AI/ML task during the performing process. Here AI/ML member means the participant of an AI/ML task. To save resource and time, the AI/ML member (source AI/ML member) decides to transfer the intermediate AI/ML information (e.g., the intermediate AI/ML operation status and results) to another AI/ML member (target AI/ML member) for further operations to complete the AI/ML task.

#### 8.6.2.1	Request AIMLE Server to assist AI/ML task transfer

Pre-conditions:

-	The information of target AI/ML member (e.g., another AIMLE Client or VAL Client different from the source AI/ML member) is unknown at the source AI/ML member. The source AI/ML member decides that assistance from the AIMLE Server is needed.

Figure 8.6.2.1-1: Procedure for requesting AIMLE Server assist AI/ML task transfer

Figure 8.6.2.1-1 illustrates the procedure for requesting AIMLE Server to assist the AI/ML task transfer. The detailed corresponding procedure is detailed as follows:

- The source AI/ML member (e.g., an AIMLE Client) sends an AI/ML task transfer assist request to AIMLE Server for assisting the AI/ML task transfer. The request message contains the information as specified in Table 8.6.3.2-1.

Editor’s Note:	The procedure can also be used for transfer AI/ML services (e.g., ML model performance evaluation, ML model update) between AI/ML members. The details are FFS.

- The AIMLE Server discovers other AI/ML members (e.g., other AIMLE Clients) directly or via ML repository, and selects one or more target AI/ML member(s) based on the request from the source AI/ML member. The AIMLE Server sends request to the selected target AI/ML member(s) for intermediate AI/ML operation information transfer, as defined in clause 8.6.2.2. Then, the AIMLE Server determines the transfer mode, i.e., transfer with server-controlled or transfer directly from the source AI/ML member to the target AI/ML member.

If target AI/ML member(s) is discovered, the AIMLE Server generates assistance information for the AI/ML task transfer from the source AI/ML member to the target AI/ML member(s) (e.g., time window for the transfer).

- The AIMLE Server sends an AI/ML task transfer assist response to the source AI/ML member with the information of the target AI/ML member(s), and assistance information generated in step 5. The response message contains the information as specified in Table 8.6.3.3-1.

#### 8.6.2.2	Request target AI/ML member for AI/ML task transfer

Figure 8.6.2.2-1: Procedure for requesting target AI/ML member for AI/ML task transfer

Figure 8.6.2.2-1 illustrates the procedure for requesting target AI/ML member for AI/ML task transfer. The corresponding procedure in detail is as follows:

- The AIMLE Server sends an AI/ML task transfer request to the selected target AI/ML member(s) for AI/ML task transfer. The request message contains the information as specified in Table 8.6.3.4-1.

- The target AI/ML member(s) authenticates and authorizes the request from the AIMLE Server. If the request is authorized, the target AI/ML member(s) performs step 3.

- The target AI/ML member(s) sends an AI/ML task transfer response to the AIMLE Server with information as specified in Table 8.6.3.5-1.

#### 8.6.2.3	Direct AI/ML task transfer

Pre-conditions:

-	The information of target AI/ML member (e.g., another AIMLE Client or VAL Client different from the source AI/ML member) is assumed to be known at the source AI/ML member, or the source AI/ML member may get the target AI/ML member information via ML repository. The source AI/ML member decides to transfer the AI/ML task to the target AI/ML member.

Figure 8.6.2.3-1: Procedure for direct AI/ML task transfer

Figure 8.6.2.3-1 illustrates the procedure for direct AI/ML task transfer. The corresponding procedure in detail is as follows:

- The source AI/ML member (e.g. an AIMLE Client) sends a direct AI/ML task transfer request to the target AI/ML member(s) for direct AI/ML task transfer. The request message contains the information as specified in Table 8.6.3.6-1.

- The target AI/ML member(s) authenticates and authorizes the request from the AIMLE Server. If the request is authorized, the target AI/ML member(s) checks its availability for continue the AI/ML operations (e.g., available resource and time).

- The target AI/ML member(s) sends a direct AI/ML task transfer response to the source AI/ML member with information as specified in Table 8.6.3.7-1.

- The source AI/ML member interacts with the target AI/ML member(s) to performs AI/ML task transfer to the target AI/ML member(s).

Editor’s Note:	The intermediate AI/ML operation information may be transferred directly between the source and target AI/ML members, or via the AIMLE Server (e.g. AIMLE Server controlled information transfer between two AI/ML members). How the AI/ML task transfer via the AIMLE Server is FFS.

### 8.6.3	Information flows

#### 8.6.3.1	General

The following information flows are specified for supporting AI/ML task transfer. In the following clauses 8.6.3.2 to 8.6.3.7, the source AI/ML member and the target AI/ML member could be e.g., VAL UE, AIMLE Client.

#### 8.6.3.2	AI/ML task transfer assist request

Table 8.6.3.2-1 describes information elements for the AI/ML task transfer assist request from the source AI/ML member to the AIMLE Server.

Table 8.6.3.2-1: AI/ML task transfer assist request

| Information element | Status | Description |
| --- | --- | --- |
| Request ID | M | The identifier of the VAL UE ID (the source AI/ML member). |
| VAL service ID | O | The identifier of the VAL service for which the assistance information is requested. |
| AI/ML Task Type | M | The type of the AI/ML operation (e.g. ML model training). |
| AI/ML Information Type | M | The type of the AI/ML information in the AI/ML task need be transferred (e.g. intermediate AI/ML operation status, intermediate AI/ML operation results). |
| Time validity | O | The time validity of the request. |

Editor’s Note:	Whether there is need of additional IEs or reduction of IEs is FFS.

#### 8.6.3.3	AI/ML task transfer assist response

Table 8.6.3.3-1 describes information elements for the AI/ML task transfer assist response from the AIMLE Server to the source AI/ML member.

Table 8.6.3.3-1: AI/ML task transfer assist response

| Information element | Status | Description |
| --- | --- | --- |
| Transfer Mode | O | Indication the transfer mode (e.g. direct transfer). |

Editor’s Note:	Whether need additional IEs or reduction of IEs is FFS.

#### 8.6.3.4	AI/ML task transfer request

Table 8.6.3.4-1 describes information elements for the AI/ML task transfer request from the AIMLE Server to the target AI/ML member.

Table 8.6.3.4-1: AI/ML task transfer request

| Information element | Status | Description |
| --- | --- | --- |
| Request ID | M | The identifier of the VAL Server. |
| VAL UE ID | M | The identifier of the VAL UE (the source AI/ML member). |
| AI/ML Task Type | M | The type of the AI/ML operation (e.g. ML model training). |
| AI/ML Information Type | M | The type of the AI/ML information in the AI/ML task need be transferred (e.g. intermediate AI/ML operation status, intermediate AI/ML operation results). |
| Time validity | O | The time validity of the request. |

Editor’s Note:	Whether need additional IEs or reduction of IEs is FFS.

#### 8.6.3.5	AI/ML task transfer response

Table 8.6.3.5-1 describes information elements for the AI/ML task transfer response from the target AI/ML member to the AIMLE Server.

Table 8.6.3.5-1: AI/ML task transfer response

| Information element | Status | Description |
| --- | --- | --- |

Editor’s Note:	The IE for AI/ML task transfer response is FFS.

#### 8.6.3.6	Direct AI/ML task transfer request

Table 8.6.3.6-1 describes information elements for the direct AI/ML task transfer request from the source AI/ML member to the target AI/ML member.

Table 8.6.3.6-1: Direct AI/ML task transfer request

| Information element | Status | Description |
| --- | --- | --- |
| Request ID | M | The identifier of the VAL UE (the source AI/ML member). |
| AI/ML Task Type | M | The type of the AI/ML operation (e.g. ML model training). |
| AI/ML Information Type | M | The type of the AI/ML information in the AI/ML task need be transferred (e.g. intermediate AI/ML operation status, intermediate AI/ML operation results). |
| Time validity | O | The time validity of the request. |

Editor’s Note:	Whether there is need of additional IEs or reduction of IEs is FFS.

#### 8.6.3.7	Direct AI/ML task transfer Response

Table 8.6.3.7-1 describes information elements for the direct AI/ML task transfer response from the target AI/ML member to the source AI/ML member.

Table 8.6.3.7-1: Direct AI/ML task transfer response

| Information element | Status | Description |
| --- | --- | --- |

Editor’s Note:	The IE for direct AI/ML task transfer response is FFS.

## 8.7	AIMLE client registration

### 8.7.1	General

Prior to participation in AI/ML operations, AI/ML capable UEs register to an AIMLE server and provide an AIMLE client profile and optionally a list of supported services. The AIMLE server uses information provided by the AIMLE client to discover and select suitable AIMLE clients for requested AI/ML operations.

The following clauses specify procedures, information flows, and APIs for AIMLE client registration.

### 8.7.2	Procedure

Pre-conditions:

1.	The AIMLE client has been pre-configured or has discovered the address (e.g., URI) of the AIMLE server.

2.	The AIMLE client has been pre-configured with an AIMLE client profile.

Figure 8.7.2-1: AIMLE client registration

1.	The AIMLE client sends an AIMLE client registration request to the AIMLE server, the registration request includes information as described in Table 8.7.3.1-1. The AIMLE client indicates in the registration request its AI/ML capabilities such as supported ML model types and supported AI/ML operations to assist with performing AIMLE client discovery and AIMLE client selection.

2.	The AIMLE server validates the registration request and performs an authentication and authorization check to determine if the AIMLE client is permitted to register to the AIMLE server and participate in AI/ML operations. The AIMLE server saves the context of the AIMLE client registration in the ML repository.

3.	The AIMLE server returns an AIMLE client registration response to the AIMLE client with the status of the request.

### 8.7.3	Information flows

#### 8.7.3.1	AIMLE client registration request

Table 8.7.3.1-1 shows the request sent by an AIMLE client to an AIMLE server for the AIMLE client registration procedure.

Table 8.7.3.1-1: AIMLE client registration request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| AIMLE client profile | M | Information about the capability of the AIMLE client to support AI/ML operations as detailed in Table 8.7.3.1-2. |
| List of supported services | O | Supported service information. |
| > List of service ID(s) | O | List of service ID(s) that the AIMLE client supports |
| > Corresponding service permission level(s) | O | Service permission level(s) (e.g., premium resource usage, standard resource usage, limited resource usage). |

Editor’s Note: The addition of AIMLE client profile ID is FFS.

Table 8.7.3.1-2: AIMLE client profile

| Information element | Status | Description |
| --- | --- | --- |
| Supported AI/ML model types | O | AI/ML model types supported by the AIMLE client (e.g., decision trees, linear regression, neural networks). |
| Supported AI/ML operations | M | AI/ML operations supported by the AIMLE client such as training. |
| Availability schedule | O | Availability schedule of the AIMLE client. |
| Application layer AIMLE client capabilities | M | Application layer AIMLE client capability information (e.g. ML application type, allowed resource usage level). |
| Dataset availability | O | Dataset availability such as dataset size, age, list of dataset features, and dataset identifiers. |
| Data capabilities | O | A list of data capabilities such as the type of data that can be collected (e.g. raw data), supported data processing capabilities (e.g. processed data), and supported exploratory data analysis functions. |

#### 8.7.3.2	AIMLE client registration response

Table 8.7.3.2-1 shows the response sent by the AIMLE server to the AIMLE client for the AIMLE client registration procedure.

Table 8.7.3.2-1: AIMLE client registration response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request: success or fail. |

## 8.8	AIMLE client discovery

### 8.8.1	General

Discovery of AIMLE clients is an important step in the AI/ML process for distributed, federated, split AI/ML, and transfer learning. Due to the nature of such learning, VAL servers need to discover suitable AIMLE clients to fulfil the requirements for the AI/ML application. The VAL server can then use the discovered AIMLE clients to select a set of AIMLE clients to perform AI/ML operations.

The following clauses specify procedures, information flows, and APIs for AIMLE client discovery.

### 8.8.2	Procedure

#### 8.8.2.1	AIMLE client discovery

Pre-conditions:

1.	AIMLE clients that support AI/ML operations have registered with the AIMLE server and included their AIMLE client profiles and optionally a list of supported services.

2.	The AIMLE server can access a ML repository to obtain AIMLE client profiles and supported services associated with AIMLE clients.

Figure 8.8.2.1-1: AIMLE client discovery

1.	A VAL server sends an AIMLE client discovery request to an AIMLE server to discover a list of AIMLE clients that are available to participate in AI/ML operations (e.g., is available and have required data to train an ML model). The AIMLE client discovery request includes information as described in Table 8.8.3.1-1.

2.	The AIMLE server performs authentication and authorization checks to determine if the requestor is able to discover AIMLE clients. If the requestor is authorized, the AIMLE server performs the following to discover AIMLE clients fulfilling the provided AIMLE client discovery criteria.

The AIMLE server also obtains candidate UEs from the ML repository.

Editor’s Note: Whether Member UE filtering criteria is included in the request is FFS.

From the list of candidate UEs, the AIMLE server discovers a list of UEs (i.e., AIMLE clients) that fulfils the discovery criteria based on AIMLE client profiles and AIMLE client discovery criteria.

Editor’s Note: Whether AI/ML Policy IDs is included in the request is FFS.

3.	The AIMLE server sends an AIMLE client discovery response that includes information in Table 8.8.3.2-1.

### 8.8.3	Information flows

#### 8.8.3.1	AIMLE client discovery request

Table 8.8.3.1-1 shows the request sent by a VAL server to an AIMLE server for the AIMLE client discovery procedure.

Table 8.8.3.1-1: AIMLE client discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor (e.g., VAL server). |
| Service identifier | O | An identifier for the service associated with the requestor. |
| AIMLE client discovery criteria | M | Discovery criteria for finding suitable AIMLE clients for AI/ML operations as detailed in Table 8.8.3.1-2. |

Table 8.8.3.1-2: AIMLE client discovery criteria

| Information element | Status | Description |
| --- | --- | --- |
| Requested ML model types | O | Requested ML model types (decision trees, linear regression, neural networks). |
| Requested AIML operations | M | Requested role for AI/ML operations such as training. |
| Application layer AIMLE client capabilities | M | Application layer AIMLE client capability information (e.g., ML application type, client availability to support AIML operations at the UE). |
| Dataset requirements | O | Information about dataset. |
| > Dataset availability | O | Dataset availability, including dataset identifiers, dataset size, age, list of dataset features. |
| > Data capabilities | O | A list of data capabilities such as the type of data that can be collected (e.g., raw data), supported data processing capabilities (e.g., processed data) and supported exploratory data analysis functions. |
| Required service | O | Information about the service. |
| > Required service ID | O | Required service ID that the client supports. |
| > Required corresponding service permission level | O | Required corresponding service permission level (e.g., premium resource usage standard resource usage, limited resource usage). |

Editor’s Note: Whether target area for data collection is included in the AIMLE client discovery criteria is FFS.

#### 8.8.3.2	AIMLE client discovery response

Table 8.8.3.2-1 shows the response sent by the AIMLE server to the VAL server for the AIMLE client discovery procedure.

Table 8.8.3.2-1: AIMLE client discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request: success or fail. |
| List of AIMLE client IDs | M | A list of AIMLE client IDs that matches the AIML discovery criteria. |

## 8.9	AIMLE client selection

### 8.9.1	General

The AIMLE client selection procedure has two modes of operation: VAL server selection and AIMLE server selection. In VAL server selection, a List of AIMLE client IDs is provided to form an AIMLE client set, which can be used for AI/ML operations (e.g., training). In AIMLE server selection, a VAL server provides AIMLE client selection criteria and number of the required AIMLE clients for the AIMLE server to select the AIMLE client set.

The following clauses specify procedures, information flows, and APIs for AIMLE client selection.

### 8.9.2	Procedure

#### 8.9.2.1	AIMLE client selection

Pre-conditions:

1.	AIMLE clients that support AI/ML operations have registered with the AIMLE server and included their AIMLE client profiles and optionally a list of supported services.

2.	The AIMLE server can access a ML repository to obtain AIMLE client profiles and supported services associated with the AIMLE clients.

Figure 8.9.2.1-1: AIMLE client selection

1.	A VAL server sends an AIMLE client selection request to an AIMLE server to select AIMLE clients available for participation in AI/ML operations (e.g., is available and have required data to train an ML model). The AIMLE client selection request includes information as described in Table 8.9.3.1-1.

2.	The AIMLE server performs authentication and authorization checks to determine if the requestor is able to select AIMLE clients.

3.	If the requestor is authorized, the AIMLE server performs AIMLE client selection based on the provided inputs as described below.

For VAL server selection, the AIMLE server receives a list of AIMLE client IDs in the request and selects the AIMLE clients in the list as candidate AIMLE clients.

For AIMLE server selection, the AIMLE server retrieves a list of clients with AI/ML capabilities from the ML repository. From the list of clients, the AIMLE server selects a list of candidate AIMLE clients, whose client profiles fulfil the AIMLE client selection criteria.

The AIMLE server may then determine the QoS parameters for the AIML traffic session between the requestor and the candidate AIMLE client(s) and configure the AI/ML traffic session(s) via SEALDD (Sdd_RegularTransmission API) or NEF services (AfSessionWithQoS API).

NOTE 1:	If the available AIMLE clients (as determined by the AIMLE server) that fulfil discovery criteria are less than the required minimum number of AIMLE clients for AI/ML operation (e.g., split AI/ML), then the AIMLE server can discover the remaining required AIMLE clients from other AIMLE servers over AIML-E reference point and include them in the response message.

NOTE 2: The AIMLE server can reuse SEAL group management for any necessary group management.

Editor’s Note: Whether and how the AIMLE server determines the QoS parameters is FFS.

4.	The AIMLE server performs AIMLE client participation procedure with each candidate AIMLE client as described in clause 8.10. For all candidate AIMLE clients that agreed to participate in AI/ML operations, the AIMLE server selects the AIMLE clients and assigns an AIMLE client set identifier for the selected clients. The AIMLE client set identifier may then be used for training a ML model.

5.	The AIMLE server sends an AIMLE client selection response that includes information in Table 8.9.3.2-1.

### 8.9.3	Information flows

#### 8.9.3.1	AIMLE client selection request

Table 8.9.3.1-1 shows the request sent by a VAL server to an AIMLE server for the AIMLE client selection procedure.

Table 8.9.3.1-1: AIMLE client selection request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor (e.g., VAL server). |
| Service identifier | O | An identifier for the service associated with the requestor. |
| List of AIMLE client IDs | O (NOTE 1) (NOTE 3) | A list of AIMLE client IDs that was previously discovered for inclusion into an AIMLE client set. |
| AIMLE client selection criteria | O (NOTE 2) | Selection criteria for finding suitable AIMLE clients for AI/ML operations as detailed in Table 8.8.3.1-2. |
| Number of the required AIMLE clients | O (NOTE 2) (NOTE 3) | Indicates the requested number of AIMLE clients to be selected based on selection policies. |
| NOTE 1: This information element is only present for VAL server selection. NOTE 2: These information elements are only present for AIMLE server selection. NOTE 3: At least one of the information elements are present. |  |  |

Editor’s Note: The inclusion of List of AI/ML policy IDs and Consumer Endpoint are FFS.

#### 8.9.3.2	AIMLE client selection response

Table 8.9.3.2-1 shows the response sent by the AIMLE server to the VAL server for the AIMLE client selection procedure.

Table 8.9.3.2-1: AIMLE client selection response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request: success or fail. |
| AIMLE client set identifier | M | An identifier to associate with the set of AIMLE clients that the VAL server has provided in the request. The AIMLE client set can be updated by using this identifier. |
| List of AIMLE client IDs | M (NOTE) | A list of AIMLE client IDs that matches the AIML discovery criteria. |
| NOTE: This information element is present for dynamic selection. |  |  |

## 8.10	AIMLE client participation

### 8.10.1	General

The following clauses specify procedures, information flows, and APIs for AIMLE client participation.

### 8.10.2	Procedure

#### 8.10.2.1	AIMLE client participation

Figure 8.10.2.1-1: AIMLE client participation

1.	An AIMLE server sends an AIMLE client participation request to an AIMLE client to verify participation in AI/ML operation(s). The request includes information as described in Table 8.10.3.1-1.

2.	The AIMLE client acknowledge its willingness to be part of (or removed from) the AIMLE client set in an AIMLE client participation response sent to the AIMLE server. The response includes information as described in Table 8.10.3.1-2.

### 8.10.3	Information flows

#### 8.10.3.1	AIMLE client participation request

Table 8.10.3.1-1 shows the request sent by the AIMLE server to each AIMLE client selected for AIMLE client participation procedure.

Table 8.10.3.1-1: AIMLE client participation request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor. |
| AIMLE client set identifier | M | An identifier for the AIMLE client set. |
| Add/remove indicator | M | Indicator for adding/removing the AIMLE client to/from the AIMLE client set. |
| AI/ML model ID | M | The AI/ML model ID to use for the AI/ML operation. |
| AI/ML operations | M | A list of AI/ML operations (e.g., training) required to be performed. |
| Operational schedule | O | Schedule for the AI/ML operations. |
| Dataset requirement | M | Dataset requirements for the AI/ML operations. Requirements includes dataset identifier, dataset size and age, and/or dataset features. |
| Service requirement | M | Information about the required Service ID and its service permission level (e.g., premium resource usage standard resource usage, limited resource usage) |

#### 8.10.3.2	AIMLE client participation response

Table 8.10.3.2-1 shows the response sent by the AIMLE clients to the AIMLE server for the AIMLE client participation procedure.

Table 8.10.3.2-1: AIMLE client participation response

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor. |
| Acknowledge | M | An indicator to acknowledge whether the AIMLE client accepts being added to the AIMLE client set. |

## 8.11	ML model management

### 8.11.1	General

In this functionality, two prodedures are described in more detail in clause 8.11.2 and clause 8.11.3 accordingly:

-	Clause 8.11.2 for ML model information storage;

-	Clause 8.11.3 for ML model information discovery.

### 8.11.2	ML model information storage

Figure 8.11.2-1 illustrates the procedure of ML model information storage.

Figure 8.11.2-1: ML model information storage

1.	The AIMLE server sends a ML model information storage request to the ML repository to store an ML model. The ML model included in the request can be one trained by the ML repository consumer or its information can be provisioned to the ML repository consumer. The request contains information elements as described in Table 8.11.4.1-1.

NOTE 1:	The security requirements within the ML model storage requirements are out of scope.

2.	Upon receiving the ML model information storage request, the ML repository verifies if the AIMLE server is authorized to store the ML model identified by Analytics ID and/or list of the allowed vendors provided within the ML model profile attribute. If the AIMLE server is authorized, the ML repository processes the request and records information of the ML model (e.g., by creating a ML model profile). The ML repository sends an ML model information storage response to the AIMLE server with an identifier of the created ML model profile.

### 8.11.3	ML model information discovery

Figure 8.11.3-1 illustrates the procedure of ML model information discovery.

Figure 8.11.3-1: ML model information discovery

1.	The AIMLE server sends an ML model information discovery request to the ML repository. The request contains information elements as described in Table 8.11.4.3-1.

2.	Upon receiving the ML model information discovery request, the ML repository verifies if the AIMLE server is authorized to discover the ML model(s) identified by the ML model ID, Analytics ID, list of allowed vendors, Base Model ID, and/or ML model interoperability information. If the ML repository consumer is authorized, the ML repository processes the request.

If the discovery request is for transfer of learning, the ML repository identifies all candidate models matching the domain and required accuracy level.

The ML repository sends the response message to the ML repository consumer. The response includes information of the discovered ML model. Optionally, the ML repository can initiate a model transfer process to the consumer.

NOTE 1:	When the ML model is within training or re-training phase, the ML model is not available for discovery. The details is indicated within the Result IE in Table 8.11.4.4-1. The training and re-training phase is used during federated learning so that AIML enabler consumer can rely on model repository to get the model details.

NOTE 2:	The model transfer process mentioned in step 2 can use the AI/ML data transfer policies.

### 8.11.4	Information flows

#### 8.11.4.1	ML model information storage request

Table 8.11.4.1-1 describes the information flow from the AIMLE server to the ML repository as a request for the ML model information storage.

Table 8.11.4.1-1: ML model information storage request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the ML repository consumer performing the request. |
| ML model | O (NOTE 1) | Provides the trained ML model. |
| ML model information | O (NOTE 1) | Provides information of the ML model, as described in Table 8.11.4.1-2. |
| NOTE 1: At least one of these information elements shall be provided. |  |  |

Table 8.11.4.1-2: ML model information

| Information element | Status | Description |
| --- | --- | --- |
| ML model identifier | M | An identifier for the ML model |
| ML model address | O | Represents the ML model address that can be used by Model repository to download the ML model. |
| Analytics ID | O | Represents analytics ID for which the model can be used. |
| Domain | O | Specifies domain for which the model can be used (e.g., for speech recognition, image recognition, video processing, location predication, etc.). |
| List of allowed vendors | O (NOTE 1) | Indicates which vendors that are allowed to retrieve the ML model and thereby also are interoperable to the model. |
| ML model interoperability information | O (NOTE 1) | Represents the vendor-specific information that conveys, e.g., requested model file format, model execution environment, etc. The encoding, format, and value of ML Model Interoperable Information is not specified since it is vendor specific information, and is agreed between vendors, if necessary for sharing purposes. |
| ML Model phase | O (NOTE 1) | Represents the ML model phase, e.g., in training, trained, re-training, deployed. |
| > Observed accuracy | O (NOTE 2) | Provides accuracy of the model (if ML model is in trained phase). |
| ML model storage and discovery requirements | O (NOTE 1) | Represents the requirements for the ML repository for the ML model storage and discovery. |
| > Storage duration | O | Represents the ML model storage duration time. When the storage duration time is expired, the stored ML model and the related information shall be deleted. |
| > Security and access requirements | O | Represents the information on security requirements for storing the ML model information and the ML model access requirements (e.g., publicly available, private use only, or available for the list of VAL server IDs). |
| NOTE 1: At least one of these information elements shall be provided. NOTE 2: This IE is included only if trained ML model is available. |  |  |

#### 8.11.4.2	ML model information storage response

Table 8.11.4.2-1 describes the information flow from the ML repository to the AIMLE server as a response for the ML model information storage request.

Table 8.11.4.2-1: ML model information storage response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates success or failure of the request. |

The ML model profile includes information about the ML model stored in the ML repository, as defined in Table 8.11.4.2-2.

Table 8.11.4.2-2: ML model profile

| Information element | Status | Description |
| --- | --- | --- |
| ML model information | M | Provides the ML model information, as described in Table 8.11.4.1-2. |

#### 8.11.4.3	ML model information discovery request

Table 8.11.4.3-1 describes the information flow from the AIMLE server to the ML repository as a request for the ML model information discovery.

Table 8.11.4.3-1: ML model information discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the ML repository consumer performing the request. |
| Filtering criteria | M | Represents the filtering criteria, which can be any of the ML model information as in Table 8.11.4.1-1. |
| > Domain | O | Domain of the service in which the model will be used to transfer learning (e.g., image processing, location predication, etc). |
| > Required accuracy performance | O | Required accuracy for the ML model (e.g. for transfer learning). |
| > Analytics ID | O (NOTE 1) | Represents analytics ID for which the model can be used. |
| > List of allowed vendors | O (NOTE 1) | Indicates which vendors that are allowed to retrieve the ML model and thereby also are interoperable to the model. |
| > Required ML model interoperability | O (NOTE 1) | Represents the vendor-specific information that conveys, e.g., requested model file format, model execution environment, etc. The encoding, format, and value of ML Model Interoperable Information is not specified since it is vendor specific information, and is agreed between vendors, if necessary for sharing purposes. |
| > ML Model phase | O (NOTE 1) | Required ML model phase, e.g., in training, trained, re-training, deployed. |
| NOTE 1: At least one of these information elements shall be provided. |  |  |

Editor’s Note: The inclusion of ML model ID and Base Model ID are FFS.

#### 8.11.4.4	ML model information discovery response

Table 8.11.4.4-1 describes the information flow from the model repository to the AIMLE server as a response for the ML model information discovery request.

Table 8.11.4.4-1: ML model information discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates success or failure of the request. |
| ML model ID list | O | Represents the identifiers of the discovered list of ML models. This information element shall be provided if the result is success. |
| > ML model | O (NOTE 1) (NOTE 2) | Represents the ML model(s). This information element may be provided if the result is success. |
| > ML model address(es) | O (NOTE 1) (NOTE 2) | Represents the ML model address(es) at Model repository that can be used to download the ML model. This information element may be provided if the result is success. |
| NOTE 1: Only one of these information elements shall be provided. NOTE 2: For transfer learning, the list of all possible models matching the filtering criteria can be included along with model address. |  |  |

## 8.12	HFL training

### 8.12.1	General

AI/ML training is a highly iterative process and is performed over many training rounds. In the case of federated and distributed learning, the training is performed with many AIMLE clients. Due to the repetitive nature of AI/ML training, the AIMLE server can be configured to manage the training process over multiple training rounds for VAL servers. Note that the HFL training procedure supports horizontal federated learning, distributed learning, transfer learning, and split AI/ML.

The following clauses specify procedures, information flows, and APIs to support HFL training.

### 8.12.2	Procedure

Pre-conditions:

- AIMLE client discovery and selection have been performed.

- Datasets are available and prepared for AI/ML training at the AIMLE clients.

Figure 8.12.2-1: HFL training

  - An AIMLE server receives an ML model training request from a VAL server as described in clause 8.3.2. If the AIMLE server determines to use HFL training, the procedure continues to step 2.

  - The AIMLE server retrieves the indicated ML model using the ML model retrieval procedure as described in clause 8.1.

  - At the beginning of a training round, the AIMLE server performs AIMLE client selection using the AIMLE client selection criteria or a list of AIMLE clients provided in step 1. If AIMLE client selection criteria are provided in step 1, the AIMLE server continuously monitors and selects AIMLE clients for the HFL training. If a list of AIMLE clients is provided in step 1, the AIMLE server selects the provided AIMLE clients for the HFL training.

  - The AIMLE server configures training schedule for each of the AIMLE client and sends a HFL training subscription request with information as described in Table 8.12.3.1-1.

  - Each AIMLE client sends a HFL training subscription response with information as described in Table 8.12.3.2-2. If the AIMLE client is not able to grant the subscription (e.g., is not able to perform the training), the AIMLE client sends a response with a fail status and the procedure skips to step 7.

  - Each AIMLE client performs local training using the configured AI/ML model, model parameters, and the prepared local data associated with the dataset identifier for the specified number of samples according to the operational schedule.

  - Upon completion or due to errors in the training, each AIMLE client sends a HFL training notification to the AIMLE server. The notification includes information as described in Table 8.12.3.1-3. The AIMLE client provides an AIMLE service ID for the HFL training operation.

  - If errors were encountered, the procedure skips to step 8.

If training was successful, the AIMLE server aggregates (e.g. averages) the model parameters received from the AIMLE clients.

If the training schedule is not complete (e.g., there are remaining training rounds), the AIMLE server configures the next set of training schedules and steps 3 to 7 are repeated for the next training round.

When the training schedule has been exhausted (e.g. there are no remaining training round) and if configured, the AIMLE server makes a ML model information storage request as described in clause 8.11.2 to store the AI/ML model in the ML repository.

  - The AIMLE server sends a ML model training notification to the VAL server as described in clause 8.3.2.

Editor’s Note: The description of the procedure will be further discussed and possibly reformatted.

### 8.12.3	Information flows

#### 8.12.3.1	HFL training subscription request

Table 8.12.3.1-1 shows the request sent by the AIMLE server to AIMLE clients for the HFL training subscription procedure.

Table 8.12.3.1-1: HFL training subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| AI/ML model and model parameters | M | Information about the AI/ML model and model parameters for use in HFL training. |
| Dataset identifier | M | The dataset identifier associated with the dataset used for HFL training. |
| Number of data samples | M | The number of data samples required for a round of HFL training. |
| Operational schedule | O | A schedule for when training is to occur. |
| Notification settings | O | Settings for how often to send notifications providing status of HFL training. For example, periodic, event-based (e.g. based on percentage completion), upon completion of each training round. |

Editor’s Note: The presence of Dataset identifier in the HFL training subscription request is FFS.

#### 8.12.3.2	HFL training subscription response

Table 8.12.3.2-1 shows the response sent by AIMLE clients to the AIMLE server for the HFL training subscription procedure.

Table 8.12.3.2-1: HFL training subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the subscription request: success, fail. |
| Subscription ID | O | An identifier for the subscription only if the status is success. |
| AIMLE service ID | M | The service identifier for the AIMLE HFL training operation. |

#### 8.12.3.3	HFL training subscription notification

Table 8.12.3.3-1 shows the notification sent by AIMLE clients to the AIMLE server for the HFL training subscription procedure.

Table 8.12.3.3-1: HFL training subscription notification

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | Status for the request: success, fail. |
| AIMLE service ID | M | The service identifier for the AIMLE HFL training operation. |
| HFL training output | M | ML model parameters from HFL training. |
| Errors list | O | A list of errors encountered during a HFL training round. |
| Timestamp | O | A timestamp for the notification. |

## 8.13	AIMLE client selection subscription and notification

### 8.13.1	General

AIMLE client selection subscription request and notification enable VAL Servers to subscribe for monitoring and selection of AIMLE clients and receive notification when there is an update on the selected and re-selected AIML client’s status when re-selection is performed according to AIML member selection criteria.

The AIMLE server interacts with the NEF and/or SEAL services to monitor AIML members who meet the selection criteria and obtain their identifiers and configuring the AIML traffic session(s) between the VAL Server and the selected AIMLE Client(s) who meet the criteria. When the AIMLE clients no more meet the criteria, the QoS adjustment is reversed.

### 8.13.2	Procedure

Figure 8.13.2-1: AIMLE client selection subscription and notification

1.	A VAL server sends an AIMLE client selection subscription request to the AIMLE Server. The AIMLE client selection subscription request includes information as described in Table 8.13.3-1 which includes selection criteria.

2.	The AIMLE server validates the AIMLE client selection subscription request. The AIMLE server further performs authentication and authorization checks to determine if the requestor is able to subscribe to the selected AIMLE client selection subscription request.

3.	The AIMLE server sends the AIMLE client selection subscription response to the VAL server.

4.	The AIME server monitors AIMLE clients whether they fulfil the selection criteria as provided in step 1. The AIMLE server interacts with the NEF and/or SEAL services (including SEALDD) to establish monitoring.

5.	The AIMLE Server obtains the identifiers of the AIMLE clients from the monitoring and selects the clients that fulfil the selection criteria and remove the AIMLE clients which do not fulfil the selection criteria. The AIMLE Server may determine the QoS parameters for the AIML traffic session between the VAL server and the selected AIMLE client and configure the AIML traffic session(s) via SEALDD (Sdd_RegularTransmission API) or NEF services (AfSessionWithQoS API). When the AIMLE clients no more meet the criteria, the QoS adjustment is reversed.

6.	The AIMLE Server notifies the VAL server about the selected and re-selected AIMLE clients e.g., the AIMLE Client A is re-selected and replaced by AIMLE Client B.

Editor’s Note: Further details in how AIML polices can be used for selection is FFS.

Editor’s Note: How the AIMLE Server determine the QoS parameters is FFS.

Editor’s Note: Service operation definition in the API clause is FFS.

### 8.13.3	Information flows

Table 8.13.3-1 shows the request sent by a VAL server to an AIMLE server for the Selected AIMLE Client selection subscription.

Table 8.13.3-1: AILME client selection subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor. |
| VAL service ID | O | An identifier for the VAL service. |
| Number of the required AIML clients | O | Indicates the requested number of AIML clients to be selected based on member selection policies. |
| Notification endpoint for the selected AIMLE Client’s | M | Represents the endpoint at the VAL server for receiving the notifications on the selected AIMLE Client’s status update. |

Editors Note: Further information elements and criterias are FFS.

Table 8.13.3-2 shows the response sent by the AIMLE server to the VAL server for the selected AIMLE client selection subscription response.

Table 8.13.3-2: AIMLE client selection subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request: success or fail. |

Table 8.13.3-3 shows the notification sent by the AIMLE Server to the VAL server for the AIMLE client selection update notification.

Table 8.13.3-3: AIMLE client selection update notification

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | The identifier of the requestor. |
| List of the selected AIMLE Client status update events | M | Represents the list of selected AIMLE Client status update events, e.g., the AIMLE Client A is re-selected and replaced by AIMLE Client B. |

## 8.x	<procedure name>

Add a copy of this clause for each procedure, adding a title, a general description, information flows and the procedure. Include an APIs clause if APIs are needed for the procedure.

### 8.x.1	General

### 8.x.2	Procedure

### 8.x.3	Information flows

# 9	AIMLE APIs

This clause may be skipped if the procedure under 8.x does not require specification of APIs.

### 9.1	AIMLE AI/ML Task Transfer API

Editor’s Note:	This clause provides the corresponding AIMLE AI/ML task transfer API.

Annex A (informative):
Deployment scenarios

## A.1	General

This Annex provides the different deployment models for AIMLE services. There could be three deployment options:

-	AIMLE server can be deployed at a centralized cloud platform and collects data from multiple EDNs.

-	AIMLE server can be deployed at the edge platform.

-	Hierarchical AIMLE server deployment, where multiple AIML enablement services are deployed in edge or central clouds (e.g., in hierarchical architecture). Such deployment allows for local-global analytics for system wide optimization.

## A.2	Deployment model #1: Cloud-deployed AIMLE server

In this deployment, , the AIMLE server is centrally located and can provide support for AIML operations to the application and edge services (EAS/EES, VAL server). An example deployment option for AIMLE server at the cloud is shown in Figure A.2-1

Figure A.2-1 Example deployment for AIMLE at the cloud

## A.3	Deployment model #2: Edge-deployed AIMLE server

In this deployment, the AIMLE server is located at the EDN as EAS and provides AIML enablement services to the other EAS(s) or other edge native applications at the edge platform. AIMLE services can be deployed by the ECSP or the MNO to provide value-add services related to AI/ML operations.

The ML support operations, that the edge deployed AIMLE Server provides, are applicable to the AIMLE service areas (as shown in the example deployment scenario in Figure A.3-1), which are equivalent to the EDN service areas.

NOTE:	AIMLE server deployed as EAS can provide application enablement service to EES.

Figure A.3-1 Example deployment for AIMLE at the cloud

## A.4	Deployment model #3: Hierarchical AIMLE server deployment

In this deployment, multiple AIMLE servers can be located at different EDNs/DNs and can be deployed by the same provider.  Such hierarchical deployments allow the local – global ML operations (e.g., federated learning across domains).

The ML support services that the edge deployed AIMLE server correspond to the AIMLE service areas (as shown in the example in Figure A.4-1), which is equivalent to the EDN service areas. The central AIMLE server covers all PLMN area and is used to coordinate the ML related operations (e.g., FL server / aggregator) with the distributed AIMLE servers.

Figure A.4-1 Example hierarchical deployment of AIMLE

Annex B (informative):
Business Scenarios

Figure B-1 shows the business relationships that exist for the AIMLE functionality and that are needed to support a single VAL user.

Figure B-1: Business relationships for VAL services

The VAL user belongs to a VAL service provider based on a VAL service agreement between the VAL user and the VAL service provider. The VAL service provider can have VAL service agreements with several VAL users. The VAL user can have VAL service agreements with several VAL service providers.

The VAL service provider can have AIMLE provider arrangements with multiple AIMLE providers and the AIMLE provider can have PLMN operator service arrangements with multiple PLMN operators.

The AIMLE server is part of the AIMLE provider. The AIMLE provider can have SEAL service agreements with other SEAL service providers and in particular ADAE provider if needed for utilizing AIMLE services for ADAE analytics. Such arrangements allow the ADAE provider to utilize the AIMLE provider services.

NOTE: The ADAE provider can have further arrangements with PLMN operator and VAL service provider; however, this is not shown in the figure.

Annex C (informative):
Role of AIMLE in ML Model Lifecycle

## C.1		General

ML model lifecycle (aka ML model operation workflow) contains several parts including AI/ML member discovery & selection, Task/model split, Task/model delivery, (Local) model training, intermediate model aggregation, Trained AI/ML model delivery, and so on. The consumer can delegate all or part of ML model lifecycle to the AI/ML enabler layer. In doing so the AI/ML enabler layer can take over some AI/Ml related work for the consumer and reduce the complexity of implementation for consumers. In the below sub-clauses, some options for the role of AIMLE in the ML model lifecycle are provided.

## C.2		Role#1 of AIMLE in ML Model Lifecycle

In this scenario, the consumer completely rely on the AI/ML enabler layer to for ML operational workflow. That is, the consumer send the requirements of the AI/ML application, then the AI/ML enabler layer can perform ML operational workflows and determine the required AI/ML model to send to the consumer.

## C.3		Role#2 of AIMLE in ML Model Lifecycle

In this scenario, consumer would like to perform part of the ML operational workflow, then consumer partially rely on the AI/ML enabler layer to assist with ML operational workflowss. Due to different ML operational workflow division, the AI/ML enablement layer can provide different levels of assistance on ML workflows for consumers.

Editor’s Note: Further details and illustration of the ML model lifecycle operations and the exact role of AIMLE are FFS.

Annex D:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-07 | SA6#62 ad hoc-e |  |  |  |  | TS skeleton | 0.0.0 |
| 2024-07 | SA6#62 ad hoc-e |  |  |  |  | Implementation of the following pCRs approved by SA6: S6a240288, S6a240289. | 0.1.0 |
| 2024-08 | SA6#62 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-243211, S6-243580, S6-243699, S6-243589, S6-243714, S6-243591, S6-243734, S6-243727, S6-243728, S6-243729, S6-243751, S6-243752, S6-243702, S6-243659, S6-243660, S6-243752, S6-243642, S6-243721, S6-243644, S6-243645. | 0.2.0 |