| 3GPP TR 23.700-35 V0.1.0 (2024-10) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on Service Enabler Architecture Layer (SEAL) Phase4; (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	8

3.3	Abbreviations	8

4	SEAL services technical gap analysis	8

4. 1	Existing SEAL service#1: NRM service	8

4.1.1	Analysis	8

4.1.2	gaps	9

4.2	Existing SEAL service #2:	9

4.2.1	Analysis	9

4.2.2	Identified gaps	9

4.x.	<Existing SEAL service #X>	9

4.x.1	Analysis	9

4.x.2	Identified gaps	10

5	SEAL services adoption gap analysis	10

5.1	Generic gap #1: Representention of SEAL services within 3GPP system	10

5.1.1	Analysis	10

5.1.2	Identified gaps	10

5.2	Generic gap #2: API design guidelines for SEAL services	10

5.2.1	Analysis	10

5.2.2	Identified gaps	11

5.3	Generic gap #3: 	11

5.3.1	Analysis	11

5.3.2	Identified gaps	11

5.4	Generic gap #4: Linking SEAL services to target application usecases	11

5.x.1	Analysis	11

5.x.2	Identified gaps	11

5.x	Generic gap #X: <>	12

5.x.1	Analysis	12

5.x.2	Identified gaps	12

5.y	<SEAL service x>	12

5.y.1	Analysis	12

5.x.2	Identified gaps	12

6	Use cases mapping to SEAL services	12

6.x	<Use case x>	12

6.x.1	Mapped SEAL services and APIs	12

7	Architecture requirements	12

7.x	< API design guidance> requirements	12

8	Solutions	13

8.1	Solution#1: Service based Architecture of 3GPP system supporting SEAL services	13

8.1.1	Solution description	13

8.1.2	Functional Elements	14

8.1.3	Reference Points	14

8.2	Solution#2: Layered representation of 3GPP network exposure system	14

8.2.1	Solution description	14

8.2.2	Functional Elements	14

8.x.3	Relationship between different layer	15

8.3	Solution#3	15

8.3.1	Solution description	15

8.4	Solution #4: < API design guidelines>	18

8.4.1	Solution description	18

8.4.2	Solution evaluation	18

9	Overall evaluation	18

9.1	Solution evaluations	18

9.2	Gap evaluations	18

10	Conclusions	19

10.1	General conclusions	19

10.2	Conclusions of SEAL services technical gap #x	19

10.2	Conclusions of SEAL services technical gap #x	19

Annex A (informative): Change history	20

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In drafting the TS/TR, pay particular attention to the use of modal auxiliary verbs! TRs shall not contain any normative provisions.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The scope of Technical Report is to study on potential enhancements to Service Enabler Architecture Layer (SEAL). In particular, the study reviews the existing SEAL specifications and architecture to identify any gaps or potential enhancements necessary to enable usage of SEAL for 3rd party applications, including aspects such as API abstraction, minimizing transformation of SEAL APIs, representation of SEAL architecture in relationship with the overall 3GPP system, and new deployment option.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.433: "Service Enabler Architecture Layer for Verticals (SEAL); Data Delivery enabler for vertical applications; Stage2".

[3]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Stage 2".

[4]	3GPP TS 23.435: "Procedures for Network Slice Capability Exposure for Application Layer Enablement Service; Stage 2".

[5]	3GPP TS 23.436: "Functional architecture and information flows for Application Data Analytics Enablement Service; Stage 2".

[6]	3GPP TS 23.437: "Service Enabler Architecture Layer for Verticals (SEAL); Spatial mapping and Spatial anchors management Stage 2".

[7]	3GPP TS 23.438: "Service Enabler Architecture Layer for Verticals (SEAL); Digital asset".

[8]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[9]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs; Stage 2"

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	SEAL services technical gap analysis

## 4. 1	Existing SEAL service#1: NRM service

### 4.1.1	Analysis

XR is an emerging mobile media service expected to contribute more and more traffics to 5G network. 3GPP specifies several new capabilities to support this feature, including the enhancement of PCF/N33 APIs towards the AF/AS. There is some gaps from the easy-to-use perspective.

(1)	Unnecessary parameters for XR, which is against minimum parameters principle

Currently the PCF/N33 APIs are common to all the verticals and its input parameters for the PCF/N33 APIs are increasing more and more and become excessive complicated. From the XR service point of view, some parameters introduced by URLLC is not required. Those non-XR related parameters, may cause confusion and mis-use risks for XR application developers.

(2) API parameters is not application developers understandable. XR specific input parameters covers several in the PCF/N33 API e.g., AFSessionWithQoS are present for various purposes (or features as defined in 3GPP TS 29.522).

The following table show the 5G added features to AsSessionWithQoS.  All the yellow highlighted features can be used by eXtended Reality (XR) and interactive media services, but not been described. It is difficult for XR application developers to link them with certain XR related service.

| AsSessionWithQoS | - The following 5G-only features defined in clause 5.14.4 of 3GPP TS 29.122 [4] may be supported only by the NEF: "EthAsSessionQoS_5G", "QoSMonitoring_5G", "PacketDelayFailureReport", "MacAddressRange_5G", "AlternativeQoS_5G", "TSC_5G", "DisableUENotification_5G", "ExposureToEAS", "AltQosWithIndParams_5G", "EnEthAsSessionQoS_5G", "enNB_5G", "AltQoSProfilesSupportReport", "ExtQoS_5G", "EnTSCAC", "L4S", "MultiMedia", "PowerSaving", "EnQoSMon", "PDUSetHandling", "RTLatency", "ToSTC_5G", "QoSTiming_5G", "ListUE_5G" and "GMEC". and "QoSMonCapRepo". - The "LOSS_OF_BEARER", "RECOVERY_OF_BEARER" and "RELEASE_OF_BEARER" events do not apply for 5G. |
| --- | --- |

NRM are defined to ease the usage of 5G network resource services, but there are some issues to well support the XR service:

-	The existing input parameters of the SS_NetworkResourceAdaptation API are not suffcient to support the latest network XR capabilities to be used by the XR service, e.g., lack of the support of PDU set handling.

-	The issue of unnecessary parameters for XR also exists, i.e., SS_NetworkResourceAdaptation API cover all the verticals XR, TSN, TSC and so on, which make it is not friendly for the XR service to identify which parameters to be used.

The above issues are caused due to the lack of application - oriented and friendly API design principle including:

-	The parameters and granularity of service shall be defined in application developer’s understandable way.

-	Minimum parameters for an API to support certain service.

So how to design XR specific NRM APIs complied with above principle needs to be studied.

### 4.1.2	gaps

- How to enhance NRM services API for XR services to ease the usage of 5G services by XR application developer.

## 4.2	Existing SEAL service #2:

### 4.2.1	Analysis

Currently, the API defined in SEALDD can be improved to expose a better service to the 3rd party. The improvement can contains following aspect:

The functions and values supported by APIs defined in SEALDD need to be further improved to make it easier for third parties to understand the functions and values of APIs. In this case, third parties can easily understand the functions and values of each API and select appropriate APIs.

The functions supported by APIs need to be further simplified and clarified. That is, it is recommended that an API support only one function. In this way, an API does not corresponding too many optional unnecessary parameters. This makes it easier for 3rd parties to use the API.

### 4.2.2	Identified gaps

Currently, the API defined in SEALDD is not friendly to vertical application:

The functions supported by APIs are should be further improved for 3rd party to understand.

The APIs need to be further simplified and clarified to make it easier for 3rd party to use.

## 4.x.	<Existing SEAL service #X>

### 4.x.1	Analysis

Editor's Note:	This clause will provide technical GAP analysis which may affect the adoption of exisisted  SEAL service X by the ecosystem partners.

Editor's Note: The Gaps can be identified from the following aspects: <<interaction with Core Network, Deployment options, adoption of APIs by other SDOs, Reason for low adoption>>

### 4.x.2	Identified gaps

Editor's Note: The Gaps identified may be also applied to other SEAL services

# 5	SEAL services adoption gap analysis

## 5.1	Generic gap #1: Representention of SEAL services within 3GPP system

### 5.1.1	Analysis

In the existing SEAL service specification 3GPP TS 23.434, the architecture mainly focuses on SEAL services layer. The holistic picture providing the role and responsibility of SEAL service layer within the complete 3GPP system perspective is missing.

As the SEAL service server is a new type of 3GPP network function, the lack of holistic picture including 3GPP system may cause issues for different interpretation of SEAL services layer.

### 5.1.2	Identified gaps

GAP# 5-1-2-1:The representation of SEAL services layer within 3GPP system needs to be added.

## 5.2	Generic gap #2: API design guidelines for SEAL services

### 5.2.1	Analysis

The supporting of application ecosystem is very important for the commercialization of the SEAL technologies. So make APIs easy-to- use is already a widely used API design principle.

There is no global unified standard/guidance about how to design easy-to-use API. However many organizations or API providers has setup their own API guidance for the easy-to use from different aspects and also announce their APIs are user friendly (e.g. https://learn.microsoft.com/en-us/azure/architecture/best-practices/api-design; https://cloud.google.com/apis/design/re.ources)

Defining easy-to-use should be and has already been one of key motivation of all SEAL services APIs. Clause 9 of 3GPP TS 23.222 [9] also defines several API guidelines. However, existing API design guidelines for easy-to-use needs to be further enhanced.

### 5.2.2	Identified gaps

-	How to design easy-to-use APIs o guide the API definition work and provide solution about how to enhance 3GPP TS 23.222 [9].

-	Revisit the existed SEAL services/APIs and make improvement based on the API design guidelines.

## 5.3	Generic gap #3:

### 5.3.1	Analysis

One of the important role of SEAL services is to re-expose the 3GPP system capabilities by combining multiple capabilities of 3GPP system, across different domain including core network, OAM and terminal client.

In the existed SEAL service specification, the architecture mainly focus on SEAL services layer. The information to tell the role and responsibility of SEAL service layer from the whole 3GPP system perspective are missing.

Using a layered representation of whole 3GPP network exposure system in the network exposure language will be helpful for customers to understand SEAL services better.

### 5.3.2	Identified gaps

-How to represent the SEAL service layer in the whole 3GPP network exposure system needs to be added

## 5.4	Generic gap #4: Linking SEAL services to target application usecases

### 5.x.1	Analysis

Many third-party application providers lack a comprehensive understanding of the mechanisms provided by 3GPP. A more detailed knowledge of SEAL could be highly beneficial for application layer developers, enabling them to make more effective use of its capabilities. Additionally, identifying and analyzing relevant use cases could lead to significant improvements in SEAL functionality. As an example; for location services, developers are often unaware of the distinctions and benefits with usage of SEAL compared to 3GPP Core Network location services or alternative OTT location services.

### 5.x.2	Identified gaps

Gap#5-x-2-1: It is essential to study how SEAL services benefit application developers (e.g., streamlined development time, enhanced robustness and reliability, abstraction of complicated APIs, etc.) and explore what new use cases can be enabled by these advantages.

Editor Note: Solutions for this key issue should be per SEAL services.

## 5.x	Generic gap #X: <>

### 5.x.1	Analysis

Editor's Note:	This clause will provide generic gap analysis which may affect the adoption of SEAL by the ecosystem partners.

Editor's Note: The gaps can be identified from the following aspects: <<interaction with Core Network, Deployment options, adoption of APIs by other SDOs, Reason for low adoption>>

### 5.x.2	Identified gaps

Editor's Note:	This clause will provide generic gap analysis which may affect the adoption of SEAL by the ecosystem partners.

## 5.y	<SEAL service x>

### 5.y.1	Analysis

Editor's Note:	This clause will provide generic gap analysis which may affect the adoption of SEAL service X by the ecosystem partners.

Editor's Note: The GAPs can be identified from the following aspects: <<interaction with Core Network, Deployment options, adoption of APIs by other SDOs, Reason for low adoption>>

### 5.x.2	Identified gaps

Editor's Note: The GAPs identified may be also applied to other SEAL services

# 6	Use cases mapping to SEAL services

## 6.x	<Use case x>

### 6.x.1	Mapped SEAL services and APIs

# 7	Architecture requirements

Editor's Note: requirements can be based on the idetified gaps

## 7.x	< API design guidance> requirements

Editor's Note:	This clause will describe the architectural requirements for the studied application layer capabilities.

[Requirement 5.X.1] All the SEAL services and APIs should be defined in the principle of simple, consistent and easy-to-use.

# 8	Solutions

<<for the idetified gaps and architecture requirements>>

## 8.1	Solution#1: Service based Architecture of 3GPP system supporting SEAL services

### 8.1.1	Solution description

This solution is to solve GAP# 5-1-2-1.

Figure 8.1.1-1 Service based Architecture of 3GPP system supporting SEAL services

In the figure 8.1.1-1, SEAL services layer represents the SEAL server(s) supporting SEAL services defined in the SEAL specification 3GPP TS 23.434.

SEAL services which are part of the 3GPP system can be deployed either by the MNO or third party.

SEAL server(s) can invoke service APIs exposed by 3GPP core network functions directly over the Core network SBA when deployed by MNO’s trusted domain. SEAL server(s) can invoke service APIs exposed by 3GPP core network functions via NEF as a non-trusted AF when deployed by the third party.

SEAL server(s) can invoke service APIs exposed by the OAM system over the OAM SBA when deployed by MNO. SEAL server(s) can invoke services APIs exposed by 3GPP core network functions via EGMF as non-trusted AF, when deployed by the third party.

SEAL services API can be exposed by MNOs or third party, to the Application server or the Application client(s) or the SEAL client(s).

NOTE 1:	In some serverless application software implementations, application server (VAL server) may not be implemented and deployed. In such deployments, Application client(s) can support all the application requirements by invoking SEAL service APIs.

NOTE 2:	3GPP TS 23.222 [9] supports where the API invokers can either be on the network side (Application server) or on the UE side (Application clients), to use services API.

Editor’s note: It is FFS whether and how OAM interacts with SEAL in a service based manner.

Editor’s note: It is FFS whether and how to add the interface between UE and Radio network.

Network Services

(e.g. NEF, NWDAF)

O&M Services

(e.g. FCAPS, slicing)

SEAL services servers

(e.g. NRM/SEAL DD for connection over 5G/4G, NSCE for 5G Slice, Group, location)

Southbound Interface (SBI)

(Network APIs)

Northbound Interface (NBI)

(Service APIs)

APIs publish

APIs discovery

VAL server

(V2X)

VAL server

(UAS)

VAL server

(IoT)

VALClient

(V2X, UAS, IoT, …)

SEAL Clients

Application Enablement layer services over 5G/EPS

UE APIs

UE/Device

5G Network services

Application Layer

UE Modem

APIs

APIs

APIs

APIs

APIs

APIs

CCF

AEF(Authorization checking， message routing )

APIs

APIs

Network connection layer

Abstraction /value added layer

Exposure framework

Application layer

APIs publish

8.1.2	Functional Elements

NOTE:	No new function elements are added to the existing 3GPP system.

### 8.1.3	Reference Points

NOTE:	No new reference points added to the existing 3GPP system.

## 8.2	Solution#2: Layered representation of 3GPP network exposure system

### 8.2.1	Solution description

This solution is to solve key issue X.

Figure 8.2.1-1: Layered representation of SEAL service layer in 3GPP network exposure system

### 8.2.2	Functional Elements

#### 8.2.2.1	3GPP Exposure framework layer

3GPP Expose layer supports functions and APIs to API invoker of 3GPP system, including Applications/or a third party’s re-exposure system. The supports the following functions as defined as CAPIF:

- Providing Common functions for a system to expose different APIs: including operate related (API discovery, Invoker account creation) for API customers [e.g. Application owner] making API service subscription.

- Providing API invoking control (validating the API invokers and authorization checking) and messages routing to the API associated service supporting entity when receiving API calling message from a running application.

#### 8.2.2.2 3GPP application enabler layer

Services created by the combination use of different network connection layer capabilities from one or multiple Network Functions deployed in telecom system, may potential combining the resources/information of client on the UE.

Services exposed at this layer normally is larger granularity than the API service exposed by single network function at network connection layer and provide an application layer meaningful functions. It could simply the application layer implementation compared to directly invoke network connection layer APIs one by one.

NOTE:	In some organizations, this layer is also called added value service layer or service abstraction layer.

#### 8.2.2.3 3GPP network connection layer

The smallest granularity services that a 3GPP network layer function entity or OAM function entity, or a 3GPP UE is able to support e.g. 3GPP network connectivity services, QoS control, 3GPP position, IMS voice call, SMS, slice creation/updation/deletion.

Network functions and capabilities defined at this layer are designed generally to serve multiple applications/scenarios.

NOTE:	The principles of network layer design normally consider how to use one network capability to support multiple scenarios. So normally the capabilities defined at this layer is in a small granularity. These capabilities can be used as element capabilities and been further aggregated to a larger granularity added value service.

### 8.x.3	Relationship between different layer

APIs from 3GPP network connection layer and APIs from application enabler layer can be provided by MNO and offered certain services towards application layer via CAPIF.

3GPP application enabler layer consumes the API services from 3GPP network connection layer and also terminal side APIs services.

## 8.3	Solution#3

### 8.3.1	Solution description

The 3GPP TS 23.433 [2] should capture following enhancement as this solution proposed.

#### 8.3.1.1 SEALDD regular connection management

## 8.3.1.1.1	General

The following clauses specify procedures, information flow and APIs for establishing an SEALDD enabled end-to-end connection between VAL client and VAL server. The end-to-end connection is uniquely identified in the SEALDD layer by the SEALDD-UU flow ID. The specific procedures detailed in the subsequent clauses are for cases in which the SEALDD regular connection is used respectively for application signalling, application data delivery initiated by VAL server, and application data delivery initiated based on DD policy.

As for the signalling transmission connection establishment, the SEALDD enabler layer can determine to establish different signalling transmission connection (i.e. best effort signalling transmission connection, reliable signalling transmission connection) for the VAL server based on the signalling transmission requirement.

##### 8.3.1.1.2	Procedure

###### 8.3.1.1.2.1 SEALDD enabled best effort signalling transmission connection establishment procedure

Figure 8.3.1.1.2.1-1 illustrates the procedure for best effort signalling transmission connection establishment. In this clause, considering VAL server’s requirement on signalling transmission is not strict, then the VAL server can invoke best effort signalling transmission connection establishment API to enable the connection establishment.

Pre-condition:

-	The VAL server can discover and select the SEALDD server by CAPIF functions.

Figure 8.3.1.1.2.1-1: SEALDD best effort signalling transmission connection establishment procedure

1.	The VAL server decides to use SEALDD service for application signalling transfer and allocates address as SEALDD-S connection information for receiving the application signalling packets from SEALDD server. The VAL server sends SEALDD enabled best effort signalling transmission connection establishment request to the SEALDD server. The SEALDD enabled best effort signalling transmission connection establishment request includes the VAL server ID, VAL service ID to identify the VAL application traffic, the SEALDD-S Data transmission connection information of the VAL server side.

2.	Upon receiving the request, the SEALDD server performs an authorization check. If authorization is successful, the SEALDD server allocates a specific address used for SEALDD traffic transfer with the incoming SEALDD client(s) for the VAL server and responds with a SEALDD enabled best effort signalling transmission connection establishment response.

Step3-step8 are same as 3GPP TS 23.433 [2] clause 9.2.2.1 step 2-step8.

##### 8.3.1.1.3	Signalling flow

##### 8.3.1.1.3.1	SEALDD enabled best effort signalling transmission connection establishment request

Table 8.3.1.1.3.1-1 describes the information flow from the VAL server to the SEALDD server for requesting the best effort application signalling traffic transmission service.

Table 8.3.1.1.3.1-1: SEALDD enabled Best effort transmission connection establishment request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | Identity of the VAL server |
| VAL service ID | O | Identity of the VAL service |
| Identity | O | Identifier of specific UE or VAL user |
| SEALDD-S connection information | M | Address information (e.g., IP address and/or port, URL) of the VAL server to receive the signalling traffic from the SEALDD server |
| Requested 5GS Delay | O | The Requested 5GS Delay provided by the VAL server |

#### 8.3.1.1.3.2	SEALDD enabled best effort transmission connection establishment response

Table 8.3.1.1.3.2-1 describes the information flow from the SEALDD server to the VAL server for responding to the best effort application signalling traffic transmission.

Table 8.3.1.1.3.2-1: SEALDD enabled best effort transmission connection establishment response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure. |
| SEALDD-S connection information | O | Address information (e.g., IP address and/or port, URL) of the SEALDD server to receive the signalling traffic from the VAL server |
| Cause | O See NOTE | Indicates the reason for the failure, e.g. SEALDD policy mismatch. |
| NOTE: The IE is only present if the Result is failure. |  |  |

###### 8.3.1.1.3.3	SEALDD signalling transmission connection establishment initiated by SEALDD client request

Table 8.3.1.1.3.3-1 describes the information flow from the SEALDD client to the SEALDD server for requesting the  SEALDD signalling transmission connection establishment.

Table 8.3.1.1.3.1-1: SEALDD signalling transmission connection establishment initiated by SEALDD client request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | Identity of the requestor (SEALDD client or SEALDD server). |
| SEALDD-UU flow ID | M (See NOTE 1) | Identity of the SEALDD-UU flow. |
| VAL server ID | O | Identity of the VAL server, applicable for SEALDD client side initiated request. |
| VAL service ID | O | Identity of the VAL service |
| Selected VAL server endpoint | M | Endpoint of the selected VAL server |
| SEALDD traffic descriptor | O | SEALDD traffic descriptor (e.g. address, port, URL, transport layer protocol) of the SEALDD client side (for client side initiated request) |
| Identity | O | The VAL user ID of the VAL user or VAL UE ID. |
| SEALDD communication lifetime | O | Identifies the DD communication lifetime, applicable for SEALDD server side initiated request. |
| NOTE 1: The SEALDD-UU flow ID is used by the SEALDD client and SEALDD server to identify different application traffic, and it is mapped to the identifiers of the application traffic and data transmission session. |  |  |

#### 8.3.1.1.3.4	SEALDD regular signalling transmission connection establishment initiated by SEALDD client response

Table 8.3.1.1.3.4-1 describes the information flow from the SEALDD server to the SEALDD client for responding to the SEALDD signalling transmission connection establishment initiated by SEALDD client.

Table 8.3.1.1.3.4-1: SEALDD signalling transmission connection establishment initiated by SEALDD client response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of establishing the SEALDD connection. |
| SEALDD traffic descriptor | O | SEALDD traffic descriptor (e.g. address, port, URL, transport layer protocol) of the SEALDD server side |
| Pending timer | O (See NOTE 1) | The pending timer to trigger the re-connection from SEALDD client when bandwidth limit check is failed. |
| Cause | O (See NOTE 2) | Indicates the reason for the failure |
| NOTE 1: These IEs are used for the SEALDD enabled bandwidth control for different VAL users, applicable for client side initiated request. NOTE 2: This IE is only present if the Result is failure |  |  |

## 8.4	Solution #4: < API design guidelines>

### 8.4.1	Solution description

This solution maps to KI# x in clause 5.x. This solution is about how to establishing common evaluation criteria to support easy-to-use API.

APP developer is one of important customer of SEAL services APIs. The APIs defined according to the following API guidelines which benefits the application implementation should be met and added to clause 9 of 3GPP TS 23.222 [9]:

- Each of the SEAL services offered by a SEAL Function shall be self-contained, reusable and use management schemes independently of other SEAL services offered by the same SEAL Function.

- The expected impact to implementation of Application shall be minimized.

- Complies with (or adapts to) the API paradigm used by application.

### 8.4.2	Solution evaluation

Editor's note:	This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

# 9	Overall evaluation

## 9.1	Solution evaluations

Editor's Note:	This clause will provide evaluation of different solution.

## 9.2	Gap evaluations

Editor's Note:	This clause will provide evaluation of different solutions.

# 10	Conclusions

## 10.1	General conclusions

Editor's note:	This clause will provide general conclusions for the study.

## 10.2	Conclusions of SEAL services technical gap #x

Editor's Note:	This clause will provide conclusions for the specific key issue in clause 4

## 10.2	Conclusions of SEAL services technical gap #x

Editor's Note:	This clause will provide conclusions for the specific key issue in clause 5

# Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-10 | SA6#63 | S6-244531 |  |  |  | TS skeleton | 0.0.0 |
| 2024-10 | SA6#63 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-244217, S6-244535, S6-244541, S6-244653, S6-244657, S6-244658, S6-244659, S6-244660, S6-244675, S6-244702. | 0.1.0 |