| 3GPP TS 24.186 V0.4.0 (2023-10) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; IMS Data Channel applications; Protocol specification; (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Abbreviations	8

4	General	9

5	Functional entities	9

5.1	General	9

5.2	UE	9

5.3	MRF	9

5.4	IMS AS	9

6	Service provision	10

7	Basic communication	10

7.1	IMS Session Control	10

8	IMS Data Channel applications	10

8.1	Procedures at UE	10

8.2	Procedures at IMS AS	10

8.3	Procedures at MRF	10

9	Signalling Procedures	11

9.1	General	11

9.2	IMS Data Channel Capability Negotiation	11

9.2.1	IMS Data Channel Capability negotiation during IMS Initial Registration	11

9.2.1.1	Procedure at the UE	11

9.2.2	IMS Data Channel Capability negotiation during IMS re-Registration	11

9.2.2.1	Procedure at the UE	11

9.2.3	IMS Data Channel Capability indication during IMS session establishment and modification	12

9.2.3.1	Procedure at the UE	12

9.3	 MMTel session procedures	12

9.3.1	General	12

9.3.2	Originating side	12

9.3.2.1	Procedures at the UE	12

9.3.2.1.1	General	12

9.3.2.1.2	IMS data channel setup in conjunction with MMTel session setup	12

9.3.2.1.3	IMS data channel setup in conjunction with MMTel session modification	13

9.3.2.1.4	Closing IMS data channel in conjunction with MMTel session modification	13

9.3.2.1.5	Closing IMS data channel in conjunction with MMTel session release	13

9.3.2.2	Procedure at the IMS AS	13

9.3.2.3	Procedure at the MRF	14

9.3.3	Terminating side	14

9.3.3.1	Procedures at the UE	14

9.3.3.1.1	General	14

9.3.3.1.2	IMS data channel setup in conjunction with MMTel session Setup	14

9.3.3.1.3	IMS data channel setup in conjunction with MMTel session modify	14

9.3.3.1.4	IMS data channel termination in conjunction with MMTel session modify	14

9.3.3.2	Procedures at the serving IMS AS for the terminating UE	15

9.3.3.2.1	IMS data channel session setup	15

9.3.3.2.2	IMS data channel session modify	15

9.3.3.3	Procedures at the MRF/MF	15

10	Interaction with supplementary services	15

10.1	Originating Identification Presentation (OIP)	15

10.2	Terminating Identification Presentation (TIP)	15

10.3	Originating Identification Restriction (OIR)	15

10.4	Terminating Identification Restriction (TIR)	15

10.5 Message Waiting Indication (MWI)	15

10.6	Conferencing (CONF)	15

10.6.1	Procedure at UE	15

10.7	Communication Diversion (CDIV)	16

10.7.1	Communication Forwarding Unconditional (CFU)	16

10.7.1.1	Actions at the AS of the diverting User	16

10.7.2	Communication Forwarding on Busy user (CFB)	16

10.7.2.1	Actions at the AS of the diverting User	16

10.7.3	Communication Deflection (CD)	17

10.7.3.1	Actions at the AS of the diverting User	17

10.7.4	Communication Forwarding on No Reply (CFNR)	17

10.7.4.1	Actions at the AS of the diverting User	17

10.7.5	Communication Forwarding on Not Reachable (CFNRc)	17

10.7.5.1	Actions at the AS of the diverting User	17

10.7.6	Communication Forwarding on Not Logged-in (CFNL)	17

10.7.6.1	Actions at the AS of the diverting User	17

10.8	Communication Waiting (CW)	18

10.8.1	Actions at AS of user B	18

10.8.2	Actions at UE of user B	18

Annex A (informative): Signalling flows	18

A.1 Interaction with supplementary services	18

A.1.1	Communication Diversion	18

A.1.1.1	Communication Forwarding unconditional	18

A.1.1.2	Communication Forwarding on Busy	19

A.1.2	Communication Waiting (CW)	20

Annex B (normative): Extensions within the present document	25

B.1	Feature-capability indicators defined in the present document	25

B.1.1	Definition of feature-capability indicator g.3gpp.datachannel	25

Annex C(normative):  Applications based on IMS data channel	25

C.1 General	25

C.2 AR communication	25

C.2.1	AR Remote Cooperation	25

C.2.1.1	General Description	25

C.2.2	Procedures	26

C.2.2.1	Session Setup Procedure	26

C.2.2.1.1	Procedure at the UE	26

C.2.2.1.2	Procedure at the IMS AS	27

C.2.2.1.3 	Procedure at the MRF	27

C.2.2.2	Session Release Procedure	27

Annex <X> (informative): Change history	28

Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document provides the protocol details for enhancements to IMS multimedia telephony communication services enabled by supporting the IMS data channel and for AR communication which is one of the applications based on IMS data channel capability, based on stage 1 requirements in 3GPP TS 22.261 [2] and stage 2 requirements in 3GPP TS 23.228 [3].

The present document is applicable to User Equipment (UE), Application Servers  (AS)and IP Multimedia (IM) Core Network (CN) subsystem which are intended to support IMS multimedia telephony communication services supporting the IMS data channel and AR communication which is one of the applications based on IMS data channel capability.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[3]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[4]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[5]	IETF RFC 5688: "A Session Initiation Protocol (SIP) Media Feature Tag for MIME Application Subtype".

[6]	IETF RFC 6809: "Mechanism to Indicate Support of Features and Capabilities in the Session Initiation Protocol (SIP)".

[7]	IETF RFC 3264: "An Offer/Answer Model with the Session Description Protocol (SDP) ".

[8]	3GPP TS 22.173: "IP Multimedia Core Network Subsystem (IMS) Multimedia Telephony Service and supplementary services; Stage 1".

[9]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[10]	3GPP TS 24.173: "IMS Multimedia telephony communication service and supplementary services; Stage 3".

[11]	3GPP TS 24.275: "Management Object (MO) for Basic Communication Part (BCP) of IMS Multimedia Telephony (MMTEL) communication service".

[12]	3GPP TS 22.261: " Service requirements for the 5G System; Stage 1".

[13]	3GPP TR 22.873: "Study on evolution of the IP Multimedia Subsystem (IMS) multimedia telephony service".

[14]	IETF RFC 8864: "Negotiation Data Channels Using the Session Description Protocol (SDP)".

[15]	3GPP TS 24.147: "Conferencing using the IP Multimedia (IM) Core Network (CN) subsystem".

[16] 	3GPP TS 24.604: "Communication Diversion (CDIV) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".

[17]	3GPP TS 24.615: "Communication Waiting (CW) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".

[18]	3GPP TR 29.175: "IP Multimedia Subsystem; IP Multimedia Subsystem (IMS) Application Server (AS) Services; Stage 3".

[19]	3GPP TR 29.176: "IP Multimedia Subsystems (IMS); Media Function (MF) Services; Stage 3".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

IMS data channel application: An IMS data channel application is an application using IMS data channel capability to provide IMS services.

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AR	Augmented Reality

AS	Application Server

CD	Communication Deflection

CDIV	Communication DIVersion

CFB	Communication Forwarding Busy

CFNL	Communication Forwarding on Not Logged-in

CFNR	Communication Forwarding No Reply

CFNRc	Communication Forwarding on subscriber Not Reachable

CFU	Communication Forwarding Unconditional

CN	Core Network

CONF	Conference

CW	Communication Waiting

DC	Data Channel

IM	IP Multimedia

IMS	IP Multimedia Core Network Subsystem

MF	Media Function

MRF	Multimedia Resource Function

MWI	Message Waiting Indication

OIP	Originating Identification Presentation

OIR	Originating Identification Restriction

TIP	Terminating Identification Presentation

TIR	Terminating Identification Restriction

UE	User Equipment

4	General

According to 3GPP TS 23.228 [3], IMS multimedia telephony service  supporting IMS data channel includes IMS data channel capability  negotiation and IMS data channel session establishment. AR communication which is application  based on IMS data channel capability, provisioned to the UE as an IMS data channel application, includes respective application domain specific media capability negotiation and media processing (e,g. AR communication).

5	Functional entities

5.1	General

This clause specifies the functionalities of the functional entities for IMS data channel.

5.2	UE

An UE supporting IMS data channel has the following functionalities:

-	support IMS data channel capability negotiation; and

-	support bootstrap data channel and application data channel establishment and management.

Additionally, theUE supporting the IMS data channel capability and provisioned with AR communication, which is an application having IMS data channel capability, supports the following functionalities:

-	support application's domain specific media capability exchange; and

-	support application's domain specific media processing.

5.3	MRF

The MRF provides the data channel media resource management and data channel media traffic forwarding.

The MRF may support application's domain specific media capability and respective media processing for the AR communication which is an application having IMS data channel capability.

For functionalities of the MRF supporting IMS data channel refer to 3GPP TS 23.228 [3] clause AC.2.2.3 and AC.9.2.

NOTE: The functionalities of the MRF supporting IMS data channel can also be provided by an MF.

NOTE:	Above description is also applicable for MF.

5.4	IMS AS

The IMS AS interacts with the DCSF and the MF or MRF.

For functionalities of the IMS AS supporting IMS data channel refer to 3GPP TS 23.228 [3] clause AC.2.2.4.

For the IMS AS interaction with the Media Function (MF) refer to 3GPP TS 29.176 [19].

For the IMS AS interaction with the Data Channel Signalling Function (DCSF) refer to 3GPP TS 29.175 [18].

6	Service provision

Editor's note: This clause will provide the details on service provision from operational view, including IMS data channel subscription, IMS data channel application subscription and provision.

Editor's note: Whether a generic subscription and provision applying to all IMS data channel applications need to be specified is FFS.

7	Basic communication

Editor's note: This clause will provide the details on the enhancement to the basic communication of IMS Multimedia Telephony communication services. So far, IMS communication service identifier, IMS data channel capability indication/negotiation and session control aspects are considered.

7.1	IMS Session Control

The IMS multimedia telephony communication enhanced to support data channel applications can support different types of media, including IMS data channel media specified in 3GPP TS 24.116 [4] in addition to MMTel media types listed in 3GPP TS 22.173 [8]. The session control procedures for the different media types shall be in accordance with 3GPP TS 24.229 [9], 3GPP TS 24.173 [10] and clause 9.

To identify how the IMS data channel media can be used in an IMS session, the SDP offer / answer negotiation is applied. When an originating SDP offer with a data channel media description in the SDP media, is included in the IMS session towards a target entity without data channel capability, to reject the offered data channel media stream, the target entity shall set the port number in the data channel media stream of the SDP answer to zero, as specified in section 6 of IETF RFC 3264 [7]. In this case the IMS session does not support IMS data channel media.

8	IMS Data Channel applications

8.1	Procedures at UE

Once the bootstrap data channels have been established, if the IMS data channel applications are available, based on the IMS data channel applications list received via the established bootstrap data channel, the UE shall follow the procedures in clause 9.3.2.1.3 to set up an application data channel and include the updated SDP offer in the reINVITE message with negotiated bootstrap data channel media description, the requested application data channel media description as well as the associated data channel application binding information, according to 3GPP TS 23.228 [3].

8.2	Procedures at IMS AS

Editor’s Note:	Role of IMS AS is FFS.

8.3	Procedures at MRF

MRF shall allocate or update application data channel media resources based on the request from IMS AS, and response to IMS AS with the allocated or updated data channel media resources information.

9	Signalling Procedures

Editor's note: This clause will provide the signalling procedures for IMS data channel subscription, IMS data channel capability indication/negotiation, IMS data channel setup and so on, involving UE, AS, IMS CN nodes, including normal/abnormal scenarios, different use cases of IMS data channel, activation or modification of QoS flow, new requirements for basic IMS procedures (e.g. IMS registration, IMS session establishment/modification, media control.)

9.1	General

This clause provides the following signalling procedures for IMS Data Channel:

-	IMS Data Channel capability negotiation or indication during IMS initial registration, re-registration and session establishment;

-	IMS Data Channel establishment which includes both bootstrap data channel and application data channel establishment during session establishment and modification;

-	IMS Data Channel shutdown which includes both bootstrap data channel and application data channel; and

-	abnormal cases.

9.2	IMS Data Channel Capability Negotiation

9.2.1	IMS Data Channel Capability negotiation during IMS Initial Registration

9.2.1.1	Procedure at the UE

On sending an unprotected REGISTER request, the UE shall include the media feature tag defined in RFC 5688 [5] for supported streaming media type. For the data channel capability indication, the UE shall use +sip.app-subtype="webrtc-datachannel" as specified in 3GPP TS 26.114 [4].

Editor's Note:	The policy related to the IMS data channel allowed at the UE, can be provided by the network to the UE using e.g., OMA-DM with the management objects specified in 3GPP TS 24.275 [11]. When the UE is configured as specified in 3GPP TS 24.275 [11] to allow to use the IMS data channel, then the UE shall include media feature tag as specified above.

On receiving the 200 (OK) response to the REGISTER request, if the 200 (OK) response includes a Feature-Caps header field containing feature-capability indicator "g.3gpp.datachannel", the UE shall determine that the network supports the data channel capability as specified in 3GPP TS 26.114 [4].

If the network doesn't support the data channel capability, the UE shall not include data channel capability indication in SIP headers and data channel related media description in SDP offers of SIP messages.

9.2.2	IMS Data Channel Capability negotiation during IMS re-Registration

9.2.2.1	Procedure at the UE

For user-initiated reregistration, the UE supporting data channel capability shall include the media feature tag defined in IETF RFC 5688 [5] for supported streaming media type. For the data channel capability indication, the UE shall use +sip.app-subtype="webrtc-datachannel" as specified in 3GPP TS 26.114 [4].

On receiving the 200 (OK) response to the REGISTER request, if the 200 (OK) response includes a Feature-Caps header field containing feature-capability indicator "g.3gpp.datachannel", the UE shall determine that the network supports the data channel capability as specified in 3GPP TS 26.114 [4].

The UE shall continue to indicate its IMS data channel capability as specified in the above procedure when the UE has successfully done the IMS data channel capability negotiation during IMS initial registration or re-registartion.

On receiving the 200 (OK) response to the REGISTER request, if the 200 (OK) response does not include a Feature-Caps header field containing feature-capability indicator "g.3gpp.datachannel",

- the UE shall not include data channel capability indication in SIP headers and data channel media description SDP bodies of SIP messages in further initial session setup request; and

- shall keep established data channel media of the UE's existing IMS session.

9.2.3	IMS Data Channel Capability indication during IMS session establishment and modification

9.2.3.1	Procedure at the UE

Upon generating an initial INVITE request or a reINVITE request, the UE supporting data channel capability and if the UE determined that the network supports the data channel capability, the UE shall include the media feature tag defined in IETF RFC 5688 [5] for supported streaming media type in the Contact header field to the remote UE and use +sip.app-subtype="webrtc-datachannel" as specified in 3GPP TS 26.114 [4]. The UE may include an Accept-Contact header field containing the "sip.app-subtype" media feature tag defined in IETF RFC 5688 [5] with a value of "webrtc-datachannel" as specified in 3GPP TS 24.173 [10].

9.3		MMTel session procedures

9.3.1	General

The support of the IMS data channel is optional.

The session control procedures for IMS multimedia telephony communication service with IMS data channel shall be in accordance with 3GPP TS 24.173 [10] with the additions defined in the present document.

9.3.2	Originating side

9.3.2.1	Procedures at the UE

9.3.2.1.1	General

The UE shall only initiate an MMTel session with an IMS data channel if the UE has determined that the UE and the network supports the IMS data channel capability.

The policy related to the UE supporting the IMS data channel can be provided by the network to the UE using e.g. OMA-DM with the management objects specified in 3GPP TS 24.275 [11]. When the UE is configured as specified in 3GPP TS 24.275 [11] with configuration for IMS data channel capability support, then the UE may setup the IMS data channel.

Editor's Note: The procedure as to how to take the policy into account will be updated.

Editor's Note: It is FFS whether this policy can also be provided on the UICC.

9.3.2.1.2	IMS data channel setup in conjunction with MMTel session setup

If the UE wants to initiate an MMTel session with IMS data channel, the UE:

1)	shall generate an initial INVITE request in accordance with 3GPP TS 24.229 [9] and 3GPP TS 24.173 [10];

2)	shall include the media feature tag defined in IETF RFC 5688 [5] for supported streaming media type with +sip.app-subtype="webrtc-datachannel" as specified in 3GPP TS 26.114 [4] in the Contact header field;

3)	may include an Accept-Contact header field containing the "sip.app-subtype" media feature tag defined in IETF RFC 5688 [5] with a value of "webrtc-datachannel" as specified in 3GPP TS 26.114 [4]; and

4)	shall include an SDP offer containing the media descriptions for the MMTel media according 3GPP TS 24.173 [10] and a data channel media description for the bootstrap data channel in accordance with 3GPP TS 26.114 [4].

9.3.2.1.3	IMS data channel setup in conjunction with MMTel session modification

If a UE wants to establish a bootstrap data channel within an existing MMTel session, the UE:

1)	shall generate a reINVITE request in accordance with 3GPP TS 24.229 [9] and 3GPP TS 24.173 [10];

2)	shall include the media feature tag defined in RFC 5688 [5] for supported streaming media type with +sip.app-subtype="webrtc-datachannel" as specified in 3GPP TS 26.114 [4] in the Contact header field;

3)	may include an Accept-Contact header field containing the "sip.app-subtype" media feature tag defined in IETF RFC 5688 [5] with a value of "webrtc-datachannel" as specified in 3GPP TS 26.114 [4]; and

4)	shall include an updated SDP offer that contains a data channel media description for the bootstrap data channel information according to 3GPP TS 26.114 [4].

If a UE wants to establish an application data channel within an existing MMTel session and when the UE has an established bootstrap data channel associated with the MMTel session available, the UE:

1)	shall generate a reINVITE request in accordance with 3GPP TS 24.229 [9] and 3GPP TS 24.173 [10];

2)	shall include the media feature tag defined in RFC 5688 [5] for supported streaming media type with +sip.app-subtype="webrtc-datachannel" as specified in 3GPP TS 26.114 [4] in the Contact header field;

3)	may include an Accept-Contact header field containing the "sip.app-subtype" media feature tag defined in IETF RFC 5688 [5] with a value of "webrtc-datachannel" as specified in 3GPP TS 26.114 [4]; and

4)	shall include an updated SDP offer that contains a data channel media description for the bootstrap data channel, as well as the requested application data channel and the associated DC application binding information, according to 3GPP TS 26.114 [4].

9.3.2.1.4	Closing IMS data channel in conjunction with MMTel session modification

If the UE wants to close the established application data channel, it shall initiate the SDP re-negotiation as defined in IETF RFC 8864 [14] subclause 6.6.1.

9.3.2.1.5	Closing IMS data channel in conjunction with MMTel session release

The IMS data channel shall be available during the IMS call session, and be released or terminated along with the call release, the UE triggered IMS call release procedure shall apply the procedures described in 3GPP TS 24.229 [9] subclause 5.1.5.

9.3.2.2	Procedure at the IMS AS

Based on served user service specific data which is enhanced with IMS data channel specific service details, if the served user is not authorized to use IMS data channel, the IMS AS may delete the received data channel media information from INVITE message, e.g. m=application line with webrtc-datachannel, and send the INVITE message to the S-CSCF.

Based on served user service specific data which is enhanced with IMS data channel specific service details, if the served user is authorized to use IMS data channel and the required data channel media resources is reserved, The IMS AS will trigger the DC media resource reservation according to 3GPP TS 23.228 [3], , the IMS AS shall send the INVITE request  with audio, video and modified data channel SDP offer to the S-CSCF.

Upon receipt of a 2xx response for a BYE request matching an existing IMS session of application data channel, the IMS AS shall follow the call release procedure as per 3GPP TS 24.229 [9].

Additionally, IMS AS will notify session release event to DCSF and as per media instruction request from DCSF, IMS AS will send media resource management request to MRF to release the allocated data channel media resources for this IMS Session. IMS AS will notifies DCSF about the DC media release as part of media instruction response.

9.3.2.3	Procedure at the MRF

MRF shall allocate bootstrap data channel media resources based on the request from the IMS AS:

a)	create one bootstrap data channel media termination to be terminated locally; or

b)	create one bootstrap data channel media termination to be offered to remote network

and response to the IMS AS with the allocated data channel media resources information.

NOTE:	This is also applicable for the procedure at the MF.

Upon receipt the data channel media resources release message from IMS AS, MRF shall release the corresponding allocated data channel media resources for this IMS session.

9.3.3	Terminating side

9.3.3.1	Procedures at the UE

9.3.3.1.1	General

The terminating UE can also setup or terminate data channels during the session modification.

9.3.3.1.2	IMS data channel setup in conjunction with MMTel session Setup

If the terminating UE determines that the UE and the network supports the data channel and it is configured to setup data channel as part of the initial session setup, on the reception of SIP INVITE, the terminating UE shall include the media feature tags defined in RFC 5688 [5] for supported streaming media type with +sip.app-subtype="webrtc-datachannel" as specified in 3GPP TS 26.114 [4] in the SIP response to the SIP INVITE request.

If the terminating UE receives an SDP offer which includes the data channel media descriptions, i.e. the "m=application" section, and the terminating UE accepts the data channel, it shall return a 18x response with the generated the SDP answer based on the 3GPP TS 26.114 [4] and IETF RFC 8864 [14].

9.3.3.1.3	IMS data channel setup in conjunction with MMTel session modify

the terminating UE determines that the UE and the network supports the data channel and it is configured to setup data channel during session modify, and the terminating UE receives an SDP offer in the re-INVITE message, which includes the data channel media descriptions, i.e. the "m=application" section, it shall return a 200 OK response to the reINVITE with the generated the SDP answer based on the 3GPP TS 26.114 [4] and IETF RFC 8864 [14].

If the UE determines that the UE and the network supports data channel and it is is configured to setup data channel as part of the initial session setup, the above paragraph also applies.

If the terminating UE wants to setup a data channel during the session modification by sending SIP re-INVITE request, the procedure defined in clause 9.3.2.1.3 applies.

9.3.3.1.4	IMS data channel termination in conjunction with MMTel session modify

If the terminating UE receives a reINVITE message including an SDP offer in which the UDP port number of the data channel media description was set to zero or the "a=dcmap" line of the data channel media description was removed, and the terminating UE accepts the data channel termination, it shall return a 200 OK response on the re-INVITE with the generated the SDP answer based on the IETF RFC 8864 [14].

If the terminating UE wants to terminate a separate data channel during the session modification by sending re-INVITE request, the procedure defined in clause 9.3.2.1.4 applies.

NOTE:	The IMS data channel termination during the session modification does not impact the ongoing audio, video or other data channels within the IMS session.

9.3.3.2	Procedures at the serving IMS AS for the terminating UE

9.3.3.2.1	IMS data channel session setup

Upon receipt of a SIP INVITE or re-INVITE message with the SDP offer including data channel media descriptions from the originating network, the IMS AS will trigger the DC media resources according to 3GPP TS 23.228 [3] and send the INVITE or re-INVITE message to the S-CSCF and then to the terminating UE.

9.3.3.2.2	IMS data channel session modify

Upon receipt of a re-INVITE message with the SDP offer including data channel media descriptions from the originating network, the IMS AS will trigger the DC media resources according to 3GPP TS 23.228 [3] and send the re-INVITE message to the S-CSCF and then to the terminating UE.

9.3.3.3	Procedures at the MRF/MF

The procedure defined in clause 9.3.2.3 applies.

10	Interaction with supplementary services

10.1	Originating Identification Presentation (OIP)

No interaction with IMS data channel.

Editor’s note:	It is subject to be changed.

10.2	Terminating Identification Presentation (TIP)

No interaction with IMS data channel.

Editor’s note:	It is subject to be changed.

10.3	Originating Identification Restriction (OIR)

No interaction with IMS data channel.

Editor’s note:	It is subject to be changed.

10.4	Terminating Identification Restriction (TIR)

No interaction with IMS data channel.

Editor’s note:	It is subject to be changed.

10.5 Message Waiting Indication (MWI)

No interaction with IMS data channel.

10.6	Conferencing (CONF)

10.6.1	Procedure at UE

When a user is participating in two or more SIP sessions, established SIP session's data channel media streams are specific to each SIP session. The user is handling multiple SIP sessions, only one SIP session shall be active at a time.

The user (conference creator) creates SIP session with the conference focus by sending an INVITE request as described in clause 5.3.1.3 3GPP TS 24.147 [15], there is a new SIP session established between the user (conference creator) and the conference focus.

The user (conference creator) decides and perform the procedures as described in clause 5.3.1.4 3GPP TS 24.147 [15] for inviting a user (conference participant) to a conference by sending an REFER request for each of the active sessions that are requested to be joined to the three-way session. There are new SIP sessions established between the users (conference participants) and the conference focus.

At the establishment of the SIP session with the conference focus, the conference creator and conference participants, shall terminate the existing call session including DC media stream as per 3GPP TS 24.147 [15] and clause 9.3.

Editor's note:	Data channel establishment with the conference focus, is FFS.

10.7	Communication Diversion (CDIV)

10.7.1	Communication Forwarding Unconditional (CFU)

10.7.1.1	Actions at the AS of the diverting User

On reception of incoming session setup INVITE request in the IMS AS of the diverting user with the media feature tag +sip.app-subtype="webrtc-datachannel" as specified in 3GPP TS 26.114 [4] in the Contact header field and SDP offer containing the media descriptions for the MMTel media according 3GPP TS 24.173 [10] and a data channel media description for the bootstrap data channel in accordance with 3GPP TS 26.114 [4], if a CFU service condition is satisfied based on the diverting user subscription data, the IMS AS of the diverting user shall not send session event notification to the DCSF. The IMS AS of the diverting user shall route the incoming session setup INVITE request towards a diverted-to user as defined in 3GPP TS 24.604 [16].

There will be no media negotiation between the originating user and the diverting user, including data channel media negotiation. The data channel media session setup shall be performed between originating user and the diverted-to user together with audio, video media negotiation as per procedures defined in clause 9.3.

10.7.2	Communication Forwarding on Busy user (CFB)

10.7.2.1	Actions at the AS of the diverting User

On reception of incoming session setup INVITE request in the IMS AS of the diverting user with the media feature tag +sip.app-subtype="webrtc-datachannel" as specified in 3GPP TS 26.114 [4] in the Contact header field and SDP offer containing the media descriptions for the MMTel media according 3GPP TS 24.173 [10] and a data channel media description for the bootstrap data channel in accordance with 3GPP TS 26.114 [4], diverting user's network functions shall reserve the data channel media resources before routing the session setup request to the diverting user.

On reception of SIP response 486 (User Busy) from the diverting user, if CFB has been triggered as defined in 3GPP TS 24.604 [16], the diverting user’s network functions shall release the reserved data channel media as per procedures defined in clause 9.3 and route the incoming session setup INVITE request towards a diverted-to user as defined in 3GPP TS 24.604 [16]. The data channel media session setup shall be performed between originating user and the diverted-to user together with audio, video media negotiation as per procedures defined in clause 9.3.

In case of failure of data channel media resources reservations at serving network functions of diverting user, the IMS AS of diverting user shall proceed with setup of the MMTel session without performing data channel bootstrapping, by deleting data channel media description(m lines) from SDP offer of incoming INVITE request and route the updated INVITE request to the diverted-to user.

For the CFB under Network Determined User Busy as defined in 3GPP TS 24.604 [16], the CFB behavior will be same with CFU as specified in clause 10.7.1.

10.7.3	Communication Deflection (CD)

10.7.3.1	Actions at the AS of the diverting User

The CD service can only be triggered before the 200OK SIP response reception from the diverting user as defined in 3GPP TS 24.604 [16].

If 302(Moved Temporarily) SIP response,

-	Before alerting 180 (Ringing) SIP response reception from the diverting user without DC media session setup between the originating user and the diverting user, the data channel media negotiation shall be performed between the originating user and the diverted-to user together with audio or/and video media as per procedures defined in clause 9.3, which is same as CFU behavior in clause 10.7.1.

-	after early SIP response 183 (Session Progress) or after alerting 180 (Ringing) SIP response reception from the diverting user with DC media session setup between the originating user and the diverting user, the established data channel between the originating and the diverting user’s network shall be released as per procedures defined in clause 9.3. IMS AS shall route the incoming session setup INVITE request towards a diverted-to user as defined in 3GPP TS 24.604 [16]. The data channel media session set up shall be performed between originating user and the diverted-to user together with audio, video media negotiation as per procedures defined in clause 9.3.

Editor's note:	SIP signalling behaviour to release established data channel between originating and diverting user’s network during pre-alerting and alerting is FFS.

10.7.4	Communication Forwarding on No Reply (CFNR)

10.7.4.1	Actions at the AS of the diverting User

The CFNR service no-reply timer at IMS AS shall be started at the reception of 180 (Ringing) SIP response reception. On no-reply timer expiry, the established data channel between the originating and the diverting user’s network shall be released as per procedures defined in clause 9.3. IMS AS shall route the incoming session setup INVITE request towards a diverted-to user as defined in 3GPP TS 24.604 [16]. The data channel media session set up shall be performed between originating user and the diverted-to user together with audio, video media negotiation as per procedures defined in clause 9.3.

Editor's note: SIP signalling behaviour to release established data channel between originating and diverting user’s network during pre-alerting or altering phase is FFS.

10.7.5	Communication Forwarding on Not Reachable (CFNRc)

10.7.5.1	Actions at the AS of the diverting User

There’s no data channel media session setup between the originating and the diverting user’s network, hence the CFNRc behavior shall be same as CFU service in clause 10.7.1.

10.7.6	Communication Forwarding on Not Logged-in (CFNL)

10.7.6.1	Actions at the AS of the diverting User

When the AS of the diverting user receives an incoming session setup INVITE request for an unregistered served user with the media feature tag +sip.app-subtype="webrtc-datachannel" as specified in 3GPP TS 26.114 [4] in the Contact header field and SDP offer containing the media descriptions for the MMTel media according 3GPP TS 24.173 [10] and a data channel media description for the bootstrap data channel in accordance with 3GPP TS 26.114 [4], and a CFNL condition is determined at the IMS AS, there’s no data channel media negotiation between the originating user and the diverting user, hence the CFNL service behavior shall be same as CFU service in clause 10.7.1.

In case of a late CFNL detection by the network (e.g., the terminating S-CSCF of a diverting user), the network shall send 480 (Temporarily Unavailable) response to the IMS AS, for the incoming INVITE request to the diverting user. On reception of a 480 (Temporarily Unavailable) response, the IMS AS of the diverting user shall trigger the release of reserved DC media resources and route the incoming INVITE request towards the diverted-to user. Hence for this use case, CFNL service behavior shall be same as CFB service in clause 10.7.2

10.8	Communication Waiting (CW)

10.8.1	Actions at AS of user B

If a network-based CW ("approaching NDUB") or terminal based CW condition is determined, after a CW service execution, the serving IMS AS will interact with the serving DCSF and the MF or MRF of the user B, to reserve the DC media resources for waiting communication, based on the served user B subscription data. The serving IMS AS shall forward or send the INVITE request to the user B, as per 3GPP TS 24.615 [17].

NOTE:	Bandwidth usage by active session DC media and the requested bandwidth usage in a waiting communication, can be one of the conditions to evaluate "approaching NDUB".

The user B may proceed with below actions when a communication waiting indication is to be given to the user B:

- the user B may accept the waiting communication and holds the active communication or releases the active communication (per procedures in 3GPP TS 24.615 [17]):

- on reception of a Re-INVITE request, which is meant for holding the active communication, the IMS AS interaction with DCSF and MF or MRF for DC media handling is not required as per clause 10.X; or

- on reception of a BYE request for the active communication, the serving IMS AS of the user B, will trigger the release of reserved DC Media resources of active communication by interacting with the serving DCSF and the MF or MRF of the user B. The serving IMS AS of the user B, shall follow the session release procedure as specified in 3GPP TS 24.229 [9].

- the user B may reject the waiting communication:

- on reception of an unsuccessful response for waiting communication from the user B, the serving IMS AS of the user B will trigger the release the reserved DC Media resources of waiting communication by interacting with the DCSF and the MF or MRF of the user B and shall reject the communication by sending unsuccessful response to the user C.

Upon expiry of the TAS-CW timer, the serving IMS AS of the user B will trigger the release of the reserved DC Media resources of waiting communication by interacting with the DCSF and the MF or MRF of the user B before sending a CANCEL request for waiting communication towards the user B.

10.8.2	Actions at UE of user B

If the user B accepts the waiting communication and holds the active communication (as per procedures in 3GPP TS 24.615 [17]), the hold invoking UE of the user B shall perform the hold procedure.

Annex A (informative):
Signalling flows

Editor's note: This annex will provide the signalling flows related to clause 9.

A.1 Interaction with supplementary services

Below sections depict signalling flow diagram for IMS data channel service interaction with existing MMTel supplementary services.

A.1.1	Communication Diversion

A.1.1.1	Communication Forwarding unconditional

Figure A.1.1.1-1 shows an example signalling flow for a successful communication forwarding unconditional based on an AS providing the forwarding and initial communication setup request consist of DC media session setup request along with other MMTel media session setup request.

Figure A.1.1.1-1: Call Forwarding Unconditional

Editor's note: Message description is FFS.

A.1.1.2	Communication Forwarding on Busy

Figures A.1.1.2-1 shows an example signalling flow for a successful communication forwarding on busy based on an AS providing the forwarding and initial communication setup request consist of DC media session setup request along with other MMTel media session setup request.

Figure A.1.1.2-1: Call Forwarding on Busy

Editor's note: Message description is FFS.

A.1.2	Communication Waiting (CW)

Figure A.1.2-1 shows an example of network-based communication waiting signalling flow at the terminating side and successful communication establishment. Waiting communication request contains DC media session along with other MMTel media sessions.

Figure A.1.2-1: Network based CW flow: Successful communication establishment.

Editor's note: Message description is FSS.

Figure A.1.2-2 shows an example of terminal-based communication waiting signalling flow at the terminating side and successful communication establishment. Waiting communication request contains DC media session along with other MMTel media sessions.

Figure A.1.2-2 Terminal based CW: Successful communication establishment.

Editor's note: Message description is FSS.

Figure A.1.2-3 shows an example of terminal-based communication waiting signalling flow at the terminating side and CW timer expires at IMS AS. Waiting communication request contains DC media session along with other MMTel media sessions.

Figure A.1.2-3 Terminal based CW: CW timer expires at AS.

Editor's note: Message description is FSS.

Figure A.1.2-4 shows an example of terminal-based communication waiting signalling flow at the terminating side and CW timer expires at UE-B. Waiting communication request contains DC media session along with other MMTel media sessions.

Figure A.1.2-4 Terminal based CW: CW timer expires at UE-B.

Editor's note: Message description is FSS.

Annex B (normative):
Extensions within the present document

B.1	Feature-capability indicators defined in the present document

This subclause describes the feature-capability indicators definitions, according to RFC 6809 [6], that are applicable for the 3GPP IM CN subsystem.

B.1.1	Definition of feature-capability indicator g.3gpp.datachannel

Editor's note: this feature-capability indicator is to be registered with IANA when release 18 is completed.

Feature-capability indicator name: g.3gpp.datachannel

Summary of the feature indicated by this feature-capability indicator:

This feature-capability indicator indicates the support of data channel capability in the network, and can be included in a Feature-Caps header field as specified in RFC 6809 [6] in a 200 (OK) response to the REGISTER request.

Feature-capability indicator specification reference:

3GPP TS 24.186, http://www.3gpp.org/ftp/Specs/archive/24_series/24.186/

Values appropriate for use with this feature-capability indicator: Not applicable

Examples of typical use: Indicating the support of data channel capability in the network.

Security Considerations: Security considerations for this feature-capability indicator are discussed in clause 9 of RFC 6809[6].

Annex C(normative): 
Applications based on IMS data channel

C.1 General

C.2 AR communication

C.2.1	AR Remote Cooperation

C.2.1.1	General Description

According to clause 6.39.2 of TS 22.261 [12], the IMS network should support AR media processing. AR Remote Cooperation is a typical AR call service and the detailed user experience of AR Remote Cooperation is described in clause 5.3 of TR 22.873 [13].

It’s assumed that local UE shares the camera to its peer UE for assistance, and remote UE displays the shared camera and provides assistance. A voice call is established between local UE and remote UE, and then AR Remote Assistance application is triggered by local UE.

The overall solution is based on IMS DC architecture specified in Annex AC of TS 23.228 [3] and shown as follows:

- The local UE triggers the media renegotiation for AR Remote Cooperation based on user actions to establish a new video stream to transmit local video content (see A002 in Figure C.2.1.1-1) and an application data channel to transmit AR marks (see A001 in Figure C.2.1.1-1).

- After the DCSF recognizes the AR Remote Cooperation service, it anchors the video stream and application data channel to MF. Then DCSF initiates media renegotiation with remote UE to establish a new video stream to transmit local video content (see B002 in Figure C.2.1.1-1) and an application data channel to transmit AR marks (see B001 in Figure C.2.1.1-1).

- The local UE or remote UE extract the original AR marks input from the user and transmits the marks to the MF through the application data channel.

- The MF tracks the specified object in the video content based on the original AR marks, and sends the updated AR marks to the local UE and remote UE.

- The local UE and remote UE display the updated AR marks on the video stream.

Figure C.2.1.1-1 illustrates the media connection model of the AR Remote Cooperation, and Table C.2.1.1-1 lists the media streams for the AR Remote Cooperation.

Figure C.2.1.1-1: Media Connection model of AR Remote Cooperation

Table C.2.1.1 -1 Media stream list for the AR Remote Cooperation

| Media ID (Example) | Media Resource Type | Direction | Description |
| --- | --- | --- | --- |
| A001 | DC | bi-directional | Transmit upstream and downstream AR marks between MF and local UE |
| A002 | Video | unidirectional | Transmit video content from local UE to MF |
| B001 | DC | bi-directional | Transmit upstream and downstream AR marks between MF and remote UE |
| B002 | Video | unidirectional | Transmit local UE's video content to remote UE |

C.2.2	Procedures

C.2.2.1	Session Setup Procedure

C.2.2.1.1	Procedure at the UE

Local UE sends a SIP re-INVITE request to establish a video stream (see B001 in Figure C.2.1.1-1) which is sendonly and an application data channel (see A001 in Figure C.2.1.1-1) as specified in 3GPP TS 24.229 [9] and 3GPP TS 24.173 [10]. The application  binding information indicates that the data channel is used for AR Remote Cooperation application.

After application data channel and video stream established, local UE or remote UE extracts the original AR mark input by the user and transmits it to MF through the application data channel.

When receiving the updated AR marks transmitted via application data channel, the UE displays the updated AR marks based on video stream.

C.2.2.1.2	Procedure at the IMS AS

When receiving the SIP re-INVITE request from local UE, IMS AS sends  a request to notify DCSF of session events related to local UE requesting to setup application data channel.

When receiving media reservation instruction from DCSF, the IMS AS converts the media instructions to the corresponding media resource operations and sends request to create or update media resources on MF, and reserve media processing resources for AR Remote Cooperation.

When receiving  response from MF, IMS AS sends  media reservation response to DCSF, indicating the URL addresses of each stream involved AR Remote Cooperation service control, which is specified in 3GPP TS 29.175 [18].

C.2.2.1.3 	Procedure at the MRF

When receiving request for creating or updating media resources from IMS AS, the MRF shall allocate media resources for each media stream and respond to IMS AS.

When receiving a media instruction from the listening port of the URL address of a specific media stream, the MRF initiate AR Remote Cooperation media processing.

When receiving AR marks from local UE or remote UE, the MRF tracks the specified object in the video content based on the original AR marks, and sends the updated AR marks to local UE and remote UE via the corresponding application data channels.

NOTE：	Procedure also applies to MF.

C.2.2.2	Session Release Procedure

The procedure of AR Remote Cooperation session release is consistent with the normal application data channel release procedure.

3GPP

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |  |
| 2023-04 | CT1#141 | C1-232099 |  |  |  | Draft skeleton provided by the rapporteur. | 0.0.0 |  |
| 2023-04 | CT1#141 | C1-232932 C1-232933 C1-232934 |  |  |  | Implementing the agreed pCR: C1-232932 C1-232933 C1-232934 | 0.1.0 |  |
| 2023-05 | CT1#142 | C1-234121 C1-234122 C1-234123 C1-234124 |  |  |  | Implementing the agreed pCR: C1-234121 C1-234122 C1-234123 C1-234124 | 0.2.0 |  |
| 2023-08 | CT1#143 | C1-236169 C1-236178 C1-236184 C1-236188 C1-236189 C1-236191 C1-236544 |  |  |  | Implementing the agreed pCR: C1-236169 C1-236178 C1-236184 C1-236188 C1-236189 C1-236191 C1-236544 | 0.3.0 |  |  |
| 2023-10 | CT1#144 | C1-238296 C1-238298 C1-238299 C1-238303 C1-238306 C1-238310 C1-238313 C1-238320 C1-238321 C1-238322 C1-238323 |  |  |  | Implementing the agreed pCR: C1-238296 C1-238298 C1-238299 C1-238303 C1-238306 C1-238310 C1-238313 C1-238320 C1-238321 C1-238322 C1-238323 | 0.4.0 |