3GPP TS 24.501 V0.3.2 (2018-02)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Core Network and Terminals;

Non-Access-Stratum (NAS) protocol
 for 5G System (5GS);

Stage 3

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

5G, 5GS, EPS, stage 3, layer 3, user equipment, network

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2017, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	14

1	Scope	15

2	References	15

3	Definitions and abbreviations	16

3.1	Definitions	16

3.2	Abbreviations	18

4	General	20

4.1	Overview	20

4.2	Coordination between the protocols for 5GS mobility management and 5GS session management	20

4.3	UE domain selection	20

4.3.1	UE’s usage setting	20

4.3.2	Domain selection for UE originating sessions / calls	20

4.3.3	Change of UE’s usage setting	21

4.3.4	Change or determination of IMS voice availability	22

4.4	NAS security	22

4.4.1	General	22

4.4.2	Handling of 5G NAS security contexts	22

4.5	Unified access control	23

4.5.1	General	23

4.5.2	Determination of the access identities and access category associated with a request for access	24

4.5.3	Operator-defined access categories	25

4.5.4	Access control and checking	26

4.5.4.1	Access control and checking in 5GMM-IDLE mode	26

4.5.4.2	Access control and checking in 5GMM-CONNECTED mode and in 5GMM-CONNECTED mode with RRC inactive indication	26

4.5.5	Exception handling and avoiding double barring	27

4.6	Network slicing	29

4.6.1	General	29

4.6.2	Mobility management aspects	29

4.6.2.1	General	29

4.6.2.2	NSSAI storage	29

4.6.3	Session management aspects	30

4.6.3.1	General	30

4.7	NAS over non-3GPP access	30

4.7.1	General	30

4.7.2	5GS mobility management aspects	31

4.7.3	5GS session management aspects	31

4.8	Interworking with E-UTRAN connected to EPC	31

4.8.1	General	31

4.8.2	Coordination between 5GMM and EMM with N26 interface	31

4.8.2.1	Handling of the 5G security context	31

4.8.3	Single-registration mode without N26 interface	32

4.8.4	Core Network selection	32

4.9	Disabling and re-enabling of UE's N1 mode radio capability	32

5	Elementary procedures for 5GS mobility management	33

5.1	Overview	33

5.1.1	General	33

5.1.2	Types of 5GMM procedures	33

5.1.3	5GMM sublayer states	35

5.1.3.1	General	35

5.1.3.2	5GMM sublayer states	35

5.1.3.2.1	5GMM sublayer states in the UE	35

5.1.3.2.1.1	General	35

5.1.3.2.1.2	Main states	35

5.1.3.2.1.2.1	5GMM-NULL	35

5.1.3.2.1.2.2	5GMM-DEREGISTERED	35

5.1.3.2.1.2.3	5GMM-REGISTERED-INITIATED	35

5.1.3.2.1.2.4	5GMM-REGISTERED	36

5.1.3.2.1.2.5	5GMM-DEREGISTERED-INITIATED	36

5.1.3.2.1.2.6	5GMM-SERVICE-REQUEST-INITIATED	36

5.1.3.2.1.3	Substates of state 5GMM-DEREGISTERED	36

5.1.3.2.1.3.1	General	36

5.1.3.2.1.3.2	5GMM-DEREGISTERED.NORMAL-SERVICE	36

5.1.3.2.1.3.3	5GMM-DEREGISTERED.LIMITED-SERVICE	36

5.1.3.2.1.3.4	5GMM-DEREGISTERED.ATTEMPTING-REGISTRATION	36

5.1.3.2.1.3.5	5GMM-DEREGISTERED.PLMN-SEARCH	36

5.1.3.2.1.3.6	5GMM-DEREGISTERED.NO-SUPI	37

5.1.3.2.1.3.7	5GMM-DEREGISTERED.NO-CELL-AVAILABLE	37

5.1.3.2.1.3.8	5GMM-DEREGISTERED.eCALL-INACTIVE	37

5.1.3.2.1.4	Substates of state 5GMM-REGISTERED	37

5.1.3.2.1.4.1	General	37

5.1.3.2.1.4.2	5GMM-REGISTERED.NORMAL-SERVICE	37

5.1.3.2.1.4.3	5GMM-REGISTERED.NON-ALLOWED-SERVICE	37

5.1.3.2.1.4.4	5GMM-REGISTERED.ATTEMPTING-REGISTRATION-UPDATE	37

5.1.3.2.1.4.5	5GMM-REGISTERED.LIMITED-SERVICE	38

5.1.3.2.1.4.6	5GMM-REGISTERED.PLMN-SEARCH	38

5.1.3.2.1.4.7	5GMM-REGISTERED.NO-CELL-AVAILABLE	38

5.1.3.2.2	5GS update status in the UE	38

5.1.3.2.3	5GMM sublayer states in the network side	38

5.1.3.2.3.1	General	38

5.1.3.2.3.2	5GMM-DEREGISTERED	39

5.1.3.2.3.3	5GMM-COMMON-PROCEDURE-INITIATED	39

5.1.3.2.3.4	5GMM-REGISTERED	39

5.1.3.2.3.5	5GMM-DEREGISTERED-INITIATED	39

5.1.4	Coordination between 5GMM and EMM	39

5.1.4.1	General	39

5.1.4.2	Coordination between 5GMM and EMM with N26 interface	39

5.1.4.3	Coordination between 5GMM and EMM without N26 interface	40

5.1.5	Dual-registration mode	40

5.2	Behaviour of the UE in state 5GMM-DEREGISTERED and state 5GMM-REGISTERED	41

5.2.1	General	41

5.2.2	UE behaviour in state 5GMM-DEREGISTERED	41

5.2.2.1	General	41

5.2.2.2	Detailed description of UE behaviour in state 5GMM-DEREGISTERED	41

5.2.2.2.1	NORMAL-SERVICE	41

5.2.2.2.2	LIMITED-SERVICE	41

5.2.5.2.3	ATTEMPTING-REGISTRATION	41

5.2.5.2.4	PLMN-SEARCH	42

5.2.5.2.5	NO-SUPI	42

5.2.5.2.6	NO-CELL-AVAILABLE	42

5.2.5.2.7	eCALL-INACTIVE	42

5.2.3	UE behaviour in state 5GMM-REGISTERED	42

5.2.3.1	General	42

5.2.3.2	Detailed description of UE behaviour in state 5GMM-REGISTERED	42

5.2.3.2.1	NORMAL-SERVICE	42

5.2.3.2.2	NON-ALLOWED-SERVICE	43

5.2.3.2.3	ATTEMPTING-REGISTRATION-UPDATE	43

5.2.3.2.4	LIMITED-SERVICE	43

5.2.3.2.5	PLMN-SEARCH	43

5.2.3.2.6	NO-CELL-AVAILABLE	43

5.3	General on elementary 5GMM procedures	43

5.3.1	5GMM modes and N1 NAS signalling connection	43

5.3.1.1	Establishment of the N1 NAS signalling connection	43

5.3.1.2	Release of the N1 NAS signalling connection	44

5.3.1.3	5GMM-CONNECTED mode with RRC inactive indication	45

5.3.2	Permanent identifiers	46

5.3.3	Temporary identities	47

5.3.4	Registration areas	47

5.3.5	Service area restrictions	48

5.3.6	Mobile initiated connection only mode	49

5.3.7	Handling of the periodic registration update timer and mobile reachable timer	50

5.3.8	Handling of NAS level mobility management congestion control	51

5.4	5GMM common procedures	51

5.4.1	Primary authentication and key agreement procedure	51

5.4.1.1	General	51

5.4.1.2	EAP based primary authentication and key agreement procedure	52

5.4.1.2.1	General	52

5.4.1.2.2	EAP-AKA' related procedures	53

5.4.1.2.2.1	General	53

5.4.1.2.2.2	Initiation	54

5.4.1.2.2.3	UE successfully authenticates network	54

5.4.1.2.2.4	Errors when handling EAP-request/AKA'-challenge message	54

5.4.1.2.2.5	Network successfully authenticates UE	54

5.4.1.2.2.6	UE handling EAP-AKA' notification	54

5.4.1.2.2.7	Network sending EAP-success message	54

5.4.1.2.2.8	UE handling EAP-success message	55

5.4.1.2.2.9	Network not successfully authenticates UE	55

5.4.1.2.2.10	Network sending EAP-failure message	55

5.4.1.2.2.11	UE handling EAP-success	55

5.4.1.2.3	EAP message reliable transport procedure	55

5.4.1.2.3.1	General	55

5.4.1.2.3.2	EAP message reliable transport procedure initiation by the network	55

5.4.1.2.3.3	EAP message reliable transport procedure accepted by the UE	56

5.4.1.2.3.4	Abnormal cases on the network side	56

5.4.1.2.3.5	Abnormal cases in the UE	56

5.4.1.3	5G AKA based primary authentication and key agreement procedure	56

5.4.1.3.1	General	56

5.4.1.3.2	Authentication initiation by the network	56

5.4.1.3.3	Authentication response by the UE	57

5.4.1.3.4	Authentication completion by the network	58

5.4.1.3.5	Authentication not accepted by the network	58

5.4.1.3.6	Authentication not accepted by the UE	58

5.4.1.3.7	Abnormal cases	58

5.4.2	Security mode control procedure	59

5.4.2.1	General	59

5.4.2.2	NAS security mode control initiation by the network	59

5.4.2.3	NAS security mode command accepted by the UE	60

5.4.2.4	NAS security mode control completion by the network	62

5.4.2.5	NAS security mode command not accepted by the UE	62

5.4.2.6	Abnormal cases in the UE	62

5.4.2.7	Abnormal cases on the network side	62

5.4.3	Identification procedure	63

5.4.3.1	General	63

5.4.3.2	Identification initiation by the network	63

5.4.3.3	Identification response by the UE	64

5.4.3.4	Identification completion by the network	64

5.4.3.5	Abnormal cases in the UE	64

5.4.3.6	Abnormal cases on the network side	64

5.4.4	Generic UE configuration update procedure	64

5.4.4.1	General	64

5.4.4.2	Generic UE configuration update procedure initiated by the network	65

5.4.4.3	Generic UE configuration update accepted by the UE	66

5.4.4.4	Generic UE configuration update completion by the network	67

5.4.4.5	Abnormal cases in the UE	67

5.4.4.6	Abnormal cases on the network side	67

5.4.5	NAS transport procedure(s)	68

5.4.5.1	General	68

5.4.5.2	UE-initiated NAS transport procedure	68

5.4.5.2.1	General	68

5.4.5.2.2	UE-initiated NAS transport procedure initiation	68

5.4.5.2.3	UE-initiated NAS transport of messages	69

5.4.5.2.4	UE-initiated NAS transport of messages failure	71

5.4.5.2.5	Abnormal cases on the network side	71

5.4.5.3	Network-initiated NAS transport procedure	73

5.4.5.3.1	General	73

5.4.5.3.2	Network-initiated NAS transport procedure initiation	73

5.4.5.3.3	Network-initiated NAS transport of messages	74

5.4.6	5GMM status procedure	74

5.4.6.1	General	74

5.4.6.2	5GMM status received in the UE	74

5.4.6.3	5GMM status received in the network	74

5.5	5GMM specific procedures	75

5.5.1	Registration procedure	75

5.5.1.1	General	75

5.5.1.2	Registration procedure for initial registration	75

5.5.1.2.1	General	75

5.5.1.2.2	Initial registration initiation	75

5.5.1.2.3	5GMM common procedure initiation	77

5.5.1.2.4	Initial registration accepted by the network	77

5.5.1.2.5	Initial registration not accepted by the network	80

5.5.1.2.6	Initial registration for emergency services not accepted by the network	80

5.5.1.2.7	Abnormal cases in the UE	81

5.5.1.2.8	Abnormal cases on the network side	81

5.5.1.3	Registration procedure for mobility and periodic registration update	81

5.5.1.3.1	General	81

5.5.1.3.2	Mobility and periodic registration update initiation	81

5.5.1.3.3	5GMM common procedure initiation	84

5.5.1.3.4	Mobility and periodic registration update accepted by the network	84

5.5.1.3.5	Mobility and periodic registration update not accepted by the network	87

5.5.1.3.6	Abnormal cases in the UE	87

5.5.1.3.7	Abnormal cases on the network side	88

5.5.2	De-registration procedure	88

5.5.2.1	General	88

5.5.2.2	UE-initiated de-registration procedure	89

5.5.2.2.1	UE-initiated de-registration procedure initiation	89

5.5.2.2.2	UE-initiated de-registration procedure completion for 5GS services over 3GPP access	89

5.5.2.2.3	UE-initiated de-registration procedure completion for 5GS services over both 3GPP access and non-3GPP access	90

5.5.2.2.4	Abnormal cases in the UE	90

5.5.2.2.5	Abnormal cases in the network side	90

5.5.2.3	Network-initiated de-registration procedure	90

5.5.2.3.1	Network-initiated de-registration procedure initiation	90

5.5.2.3.2	Network-initiated de-registration procedure completion by the UE	91

5.5.2.3.3	Network-initiated de-registration procedure completion by the network	92

5.5.2.3.4	Abnormal cases in the UE	92

5.5.2.3.5	Abnormal cases in the network side	92

5.6	5GMM connection management procedures	92

5.6.1	Service request procedure	92

5.6.1.1	General	92

5.6.1.2	Service request procedure initiation	93

5.6.1.3	Common procedure initiation	94

5.6.1.4	Service request procedure accepted by the network	94

5.6.1.5	Service request procedure not accepted by the network	95

5.6.1.6	Abnormal cases in the UE	96

5.6.1.7	Abnormal cases on the network side	96

5.6.2	Paging procedure	96

5.6.2.1	General	96

5.6.2.2	Paging for 5GS services	96

5.6.2.2.1	General	96

5.6.2.2.2	Abnormal cases on the network side	97

5.6.2.2.3	Abnormal cases in the UE	97

5.6.3	Notification procedure	97

5.6.3.1	General	97

5.6.3.2	Notification procedure initiation	98

5.6.3.3	Notification procedure completion	98

5.6.3.4	Abnormal cases on the network side	99

6	Elementary procedures for 5GS session management	99

6.1	Overview	99

6.1.1	General	99

6.1.2	Types of 5GSM procedures	99

6.1.3	5GSM sublayer states	99

6.1.3.1	General	99

6.1.3.2	5GSM sublayer states in the UE	99

6.1.3.2.1	Overview	99

6.1.3.2.2	PDU SESSION INACTIVE	100

6.1.3.2.3	PDU SESSION ACTIVE PENDING	100

6.1.3.2.4	PDU SESSION ACTIVE	100

6.1.3.2.5	PDU SESSION INACTIVE PENDING	100

6.1.3.2.6	PDU SESSION MODIFICATION PENDING	100

6.1.3.2.7	PDU SESSION ACTIVE PENDING	100

6.1.3.2.8	PDU SESSION ACTIVE	101

6.1.3.2.9	PDU SESSION INACTIVE PENDING	101

6.1.3.2.10	PDU SESSION MODIFICATION PENDING	101

6.1.3.2.11	PROCEDURE TRANSACTION INACTIVE	101

6.1.3.2.12	PROCEDURE TRANSACTION PENDING	101

6.1.3.3	5GSM sublayer states in the network side	101

6.1.3.3.1	Overview	101

6.1.3.3.2	PDU SESSION INACTIVE	102

6.1.3.3.3	PDU SESSION ACTIVE	102

6.1.3.3.4	PDU SESSION INACTIVE PENDING	102

6.1.3.3.5	PDU SESSION MODIFICATION PENDING	102

6.1.3.3.6	PROCEDURE TRANSACTION INACTIVE	102

6.1.3.3.7	PROCEDURE TRANSACTION PENDING	102

6.1.4	Coordination between 5GSM and ESM	103

6.1.4.1	General	103

6.4.1.2	Coordination between 5GSM and ESM without N26 interface in single-registration mode	104

6.2	General on elementary 5GSM procedures	105

6.2.1	Principles of PTI handling for 5GSM procedures	105

6.2.2	PDU session types	106

6.2.3	PDU session management	106

6.2.4	IP address allocation	107

6.2.4.1	General	107

6.2.4.2	IP address allocation via NAS signalling	107

6.2.5	Quality of service	108

6.2.5.1	General	108

6.2.5.1.1	QoS rules	108

6.2.5.1.1.1	General	108

6.2.5.1.1.2	Signalled QoS rules	108

6.2.5.1.1.3	Derived QoS rules	109

6.2.5.1.2	Session-AMBR	110

6.2.5.1.3	UL user data packet matching	110

6.2.5.1.4	Reflective QoS	110

6.2.5.1.4.1	General	110

6.2.5.1.4.2	Derivation of packet filter for UL direction from DL user data packet	110

6.2.5.1.4.3	Creating a derived QoS rule by reflective QoS in the UE	112

6.2.5.1.4.4	Updating a derived QoS rule by reflective QoS in the UE	112

6.2.5.1.4.5	Deleting a derived QoS rule by reflective QoS in the UE	112

6.2.6	Local area data network	113

NOTE:	In this release, LADNs apply only to 3GPP access.6.2.7 Handling of DNN based congestion control	113

6.2.8	Handling of S-NSSAI based congestion control	113

6.2.9	URSP	114

6.3	Network-requested 5GSM procedures	115

6.3.1	PDU session establishment authentication and authorization procedure	115

6.3.1.1	General	115

6.3.1.2	PDU session establishment authentication and authorization procedure initiation	116

6.3.1.3	PDU session establishment authentication and authorization procedure accepted by the UE	116

6.3.1.4	Abnormal cases on the network side	117

6.3.1.5	Abnormal cases in the UE	117

6.3.2	Network-requested PDU session modification procedure	117

6.3.2.1	General	117

6.3.2.2	Network-requested PDU session modification procedure initiation	117

6.3.2.3	Network-requested PDU session modification procedure accepted by the UE	118

6.3.2.4	Network-requested PDU session modification procedure not accepted by the UE	119

6.3.2.5	Abnormal cases on the network side	119

6.3.2.6	Abnormal cases in the UE	119

6.3.3	Network-requested PDU session release procedure	119

6.3.3.1	General	119

6.3.3.2	Network-requested PDU session release procedure initiation	120

6.3.3.3	Network-requested PDU session release procedure accepted by the UE	121

6.3.3.4	N1 SM delivery skipped	123

6.3.3.5	Abnormal cases on the network side	123

6.3.3.6	Abnormal cases in the UE	123

6.4	UE-requested 5GSM procedures	123

6.4.1	UE-requested PDU session establishment procedure	123

6.4.1.1	General	123

6.4.1.2	UE-requested PDU session establishment procedure initiation	123

6.4.1.3	UE-requested PDU session establishment procedure accepted by the network	125

6.4.1.4	UE requested PDU session establishment procedure not accepted by the network	127

6.4.1.4.1	General	127

6.4.1.4.2	Handling of network rejection due to 5GSM cause #26	130

6.4.1.4.3	Handling of network rejection due to 5GSM cause other than 5GSM cause #26	130

6.4.1.5	Abnormal cases in the UE	130

6.4.1.6	Abnormal cases on the network side	130

6.4.2	UE-requested PDU session modification procedure	131

6.4.2.1	General	131

6.4.2.2	UE-requested PDU session modification procedure initiation	131

6.4.2.3	UE-requested PDU session modification procedure accepted by the network	132

6.4.2.4	UE-requested PDU session modification procedure not accepted by the network	132

6.4.2.5	Abnormal cases in the UE	134

6.4.2.6	Abnormal cases on the network side	134

6.4.3	UE-requested PDU session release procedure	134

6.4.3.1	General	134

6.4.3.2	UE-requested PDU session release procedure initiation	135

6.4.3.3	UE-requested PDU session release procedure accepted by the network	135

6.4.3.4	UE-requested PDU session release procedure not accepted by the network	135

6.4.3.5	Abnormal cases in the UE	135

6.4.3.6	Abnormal cases on the network side	136

6.5	5GSM status procedure	136

6.5.1	General	136

6.5.2	5GSM status received in the UE	136

6.5.3	5GSM status received in the SMF	136

6.6	Miscellaneous procedures	137

6.6.1	Exchange of extended protocol configuration options	137

7	Handling of unknown, unforeseen, and erroneous protocol data	137

7.1	General	137

7.2	Message too short	137

7.3	Unknown or unforeseen procedure transaction identity or PDU Session identity	137

7.3.1	Procedure transaction identity	137

7.3.2	PDU Session identity	137

7.4	Unknown or unforeseen message type	138

7.5	Non-semantical mandatory information element errors	138

7.5.1	Common procedures	138

7.5.2	5GS mobility management	138

7.5.3	5GS session management	139

7.6	Unknown and unforeseen IEs in the non-imperative message part	139

7.6.1	IEIs unknown in the message	139

7.6.2	Out of sequence IEs	139

7.6.3	Repeated IEs	139

7.7	Non-imperative message part errors	139

7.7.1	Syntactically incorrect optional IEs	139

7.7.2	Conditional IE errors	139

7.8	Messages with semantically incorrect contents	140

8	Message functional definitions and contents	140

8.1	Overview	140

8.2	5GS mobility management messages	140

8.2.1	Authentication request	140

8.2.1.1	Message definition	140

8.2.1.2	EAP message	141

8.2.2	Authentication response	141

8.2.2.1	Message definition	141

8.2.2.2	EAP message	141

8.2.3	Authentication failure	141

8.2.3.1	Message definition	141

8.2.4	Authentication reject	142

8.2.4.1	Message definition	142

8.2.5	Registration request	142

8.2.5.1	Message definition	142

8.2.5.2	NonceUE	143

8.2.5.3	5GMM capability	143

8.2.5.4	UE security capability	143

8.2.5.5	Requested NSSAI	143

8.2.5.6	Last visited registered TAI	144

8.2.5.7	S1 UE network capability	144

8.2.5.8	Uplink data status	144

8.2.5.9	PDU session status	144

8.2.5.11	UE status	144

8.2.5.12	Additional GUTI	144

8.2.5.13	NSSAI info for PDU sessions	144

8.2.5.14	SMS requested	144

8.2.5.15	EPS NAS message container	144

8.2.6	Registration accept	144

8.2.6.1	Message definition	144

8.2.6.2	5G-GUTI	145

8.2.6.3	Equivalent PLMNs	145

8.2.6.4	TAI list	145

8.2.6.5	Allowed NSSAI	145

8.2.6.6	Rejected NSSAI	145

8.2.6.8	5GS network feature support	145

8.2.6.9	PDU session status	146

8.2.6.10	PDU session reactivation result	146

8.2.6.11	LADN information	146

8.2.6.13	SMS allowed	146

8.2.7	Registration complete	146

8.2.7.1	Message definition	146

8.2.8	Registration reject	146

8.2.8.1	Message definition	146

8.2.8.2	T3346 value	147

8.2.9	UL NAS transport	147

8.2.9.1	Message definition	147

8.2.9.2	PDU session ID	148

8.2.9.3	Old PDU session ID	148

8.2.9.4	Request type	148

8.2.9.5	S-NSSAI	148

8.2.9.6	DNN	148

8.2.9.7	Additional information	148

8.2.10	DL NAS transport	148

8.2.10.1	Message definition	148

8.2.10.2	PDU session ID	149

8.2.10.3	Additional information	149

8.2.11	De-registration request (UE originating de-registration)	149

8.2.11.1	Message definition	149

8.2.12	De-registration accept (UE originating de-registration)	149

8.2.12.1	Message definition	149

8.2.13	De-registration request (UE terminated de-registration)	150

8.2.13.1	Message definition	150

8.2.13.2	5GMM cause	150

8.2.13.3	T3346 value	150

8.2.14	De-registration accept (UE terminated de-registration)	150

8.2.14.1	Message definition	150

8.2.15	Service request	151

8.2.15.1	Message definition	151

8.2.15.2	Uplink data status	151

8.2.15.3	PDU session status	151

8.2.15.4	Allowed PDU session status	152

8.2.16	Service accept	152

8.2.16.1	Message definition	152

8.2.16.2	PDU session status	152

8.2.16.3	PDU session reactivation result	152

8.2.17	Service reject	152

8.2.17.1	Message definition	152

8.2.17.2	PDU session status	153

8.2.17.3	T3346 value	153

8.2.18	Configuration update command	153

8.2.18.1	Message definition	153

8.2.18.2	5G-GUTI	154

8.2.18.3	TAI list	154

8.2.18.4	Allowed NSSAI	154

8.2.18.5	Service area list	154

8.2.18.6	Full name for network	154

8.2.18.7	Short name for network	155

8.2.18.8	Local time zone	155

8.2.18.9	Universal time and local time zone	155

8.2.18.10	Network daylight saving time	155

8.2.18.11	LADN information	155

8.2.18.12	MICO indication	155

8.2.18.13	Configured NSSAI	155

8.2.19	Configuration update complete	155

8.2.19.1	Message definition	155

8.2.20	Identity request	156

8.2.20.1	Message definition	156

8.2.21	Identity response	156

8.2.21.1	Message definition	156

8.2.22	Notification	157

8.2.22.1	Message definition	157

8.2.24	Security mode command	157

8.2.24.1	Message definition	157

8.2.24.2	Allowed NSSAI	158

8.2.24.3	IMEISV request	158

8.2.24.4	Replayed nonceUE	158

8.2.24.5	NonceAMF	158

8.2.24.6	HashAMF	158

8.2.25	Security mode complete	159

8.2.25.1	Message definition	159

8.2.25.2	IMEISV	159

8.2.25.3	NAS message container	159

8.2.26	Security mode reject	159

8.6.26.1	Message definition	159

8.2.27	5GMM status	160

8.2.27.1	Message definition	160

8.3	5GS session management messages	160

8.3.1	PDU session establishment request	160

8.3.1.1	Message definition	160

8.3.1.2	PDU session type	161

8.3.1.3	SSC mode	161

8.3.1.4	5GSM capability	161

8.3.1.5	SM PDU DN request container	161

8.3.1.6	Extended protocol configuration options	161

8.3.2	PDU session establishment accept	161

8.3.2.1	Message definition	161

8.3.2.2	5GSM cause	162

8.3.2.3	PDU address	162

8.3.2.4	EAP message	162

8.3.2.5	RQ timer value	162

8.3.2.6	Extended protocol configuration options	162

8.3.2.x	S-NSSAI	163

8.3.3	PDU session establishment reject	163

8.3.3.1	Message definition	163

8.3.3.2	Extended protocol configuration options	163

8.3.3.3	EAP message	163

8.3.4	PDU session authentication command	163

8.3.4.1	Message definition	163

8.3.4.2	EAP message	164

8.3.4.3	Extended protocol configuration options	164

8.3.5	PDU session authentication complete	164

8.3.5.1	Message definition	164

8.3.5.2	EAP message	165

8.3.5.3	Extended protocol configuration options	165

8.3.7	PDU session modification request	165

8.3.7.1	Message definition	165

8.3.7.2	Extended protocol configuration options	165

8.3.7.3	Requested QoS rules	165

8.3.8	PDU session modification reject	165

8.3.8.1	Message definition	165

8.3.8.2	Extended protocol configuration options	166

8.3.9	PDU session modification command	166

8.3.9.1	Message definition	166

8.3.9.2	5GSM cause	166

8.3.9.3	Authorized QoS rules	167

8.3.9.4	Session-AMBR	167

8.3.9.5	PDU session release time	167

8.3.9.6	Extended protocol configuration options	167

8.3.10	PDU session modification complete	167

8.3.10.1	Message definition	167

8.3.10.2	Extended protocol configuration options	167

8.3.11	PDU session modification command reject	167

8.3.11.1	Message definition	167

8.3.11.2	Extended protocol configuration options	168

8.3.12	PDU session release request	168

8.3.12.1	Message definition	168

8.3.12.2	Extended protocol configuration options	168

8.3.13	PDU session release reject	169

8.3.13.1	Message definition	169

8.3.13.2	Extended protocol configuration options	169

8.3.14	PDU session release command	169

8.3.14.1	Message definition	169

8.3.1.2	Extended protocol configuration options	170

8.3.15	PDU session release complete	170

8.3.15.1	Message definition	170

8.3.15.2	Extended protocol configuration options	170

8.3.16	5GSM status	171

8.3.16.1	Message definition	171

9	General message format and information elements coding	171

9.1	Overview	171

9.2	Extended protocol discriminator	172

9.3	Security header type	172

9.4	PDU session identity	173

9.5	Spare half octet	173

9.6	Procedure transaction identity	173

9.7	Message type	173

9.8	Other information elements	174

9.8.1	General	174

9.8.2	Common information elements	174

9.8.2.1	Additional information	174

9.8.2.2	PDU session status	175

9.8.2.3	Uplink data status	175

9.9.3	5GS mobility management (5GMM) information elements	176

9.9.3.1	5GMM capability	176

9.8.3.2	5GMM cause	176

9.8.3.3	5GS mobile identity	177

9.8.3.4	5GS network feature support	178

9.8.3.5	5GS registration result	178

9.8.3.6	5GS registration type	178

9.8.3.7	Allowed PDU session status	179

9.8.3.8	Authentication parameter AUTN	179

9.8.3.9	Authentication parameter RAND	180

9.8.3.10	Configuration update indication	180

9.8.3.11	Daylight saving time	180

9.8.3.12	De-registration type	180

9.8.3.13	DNN	181

9.8.3.14	EAP message	181

9.8.3.15	EPS NAS message container	182

9.8.3.16	GPRS timer 2	182

9.8.3.17	HashAMF	182

9.8.3.18	IMEISV request	183

9.8.3.19	LADN information	183

9.8.3.20	Message authentication code	184

9.8.3.21	MICO indication	184

9.8.3.22	NAS key set identifier	184

9.8.3.23	NAS message container	185

9.8.3.24	NAS security algorithms	185

9.8.3.25	NAS security parameters to NG-RAN	186

9.8.3.26	Network name	187

9.8.3.27	Nonce	187

9.8.3.28	NSSAI	187

9.8.3.29	NSSAI info for PDU sessions	188

9.8.3.30	Payload container	189

9.8.3.31	Payload container type	189

9.8.3.32	PDU session reactivation result	190

9.8.3.33	PLMN list	190

9.8.3.34	Old PDU session identity	190

9.8.3.36	S1 UE network capability	192

9.8.3.37	S-NSSAI	192

9.8.3.38	Sequence number	193

9.8.3.39	Service area list	193

9.8.3.40	SMS allowed	197

9.8.3.41	SMS requested	197

9.8.3.42	Request type	198

9.8.3.43	Service type	198

9.8.3.44	Tracking area identity	199

9.8.3.45	Tracking area identity list	199

9.8.3.46	Time zone	199

9.8.3.47	Time zone and time	199

9.8.3.48	UE security capability	199

9.8.3.49	UE status	202

9.8.4	5GS session management (5GSM) information elements	202

9.8.4.1	5GSM cause	202

9.8.4.2	Extended protocol configuration options	203

9.8.4.3	GPRS timer	203

9.8.4.4	PDU address	203

9.8.4.5	PDU session type	204

9.8.4.6	QoS rules	204

9.8.4.7	Session-AMBR	213

9.8.4.8	SM PDU DN request container	214

9.8.4.9	SSC mode	214

9.8.4.10	5GSM capability	215

9.9	3GPP specific coding information defined within present document	215

9.9.1	Serving network name (SNN)	215

10	List of system parameters	216

10.1	General	216

10.2	Timers of 5GS mobility management	216

10.3	Timers of 5GS session management	219

Annex A (informative): Cause values for 5GS mobility management	221

A.1	Causes related to UE identification	221

A.2	Cause related to subscription options	221

A.3	Causes related to PLMN specific network failures and congestion/authentication failures	221

A.4	Causes related to invalid messages	222

Annex B (informative): Cause values for 5GS session management	223

B.1	Causes related to nature of request	223

B.2	Protocol errors (e.g., unknown message)	223

Annex C (informative): Change history	225

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document specifies the non-access stratum (NAS) procedures in the 5G system (5GS) used by the protocols for:

-	mobility management between the user equipment (UE) and the access and mobility management function (AMF) for both 3GPP access and non-3GPP access; and

-	session management between the user equipment (UE) and the session management function (SMF) for both 3GPP access and non-3GPP access.

The 5GS mobility management (5GMM) protocol defined in the present document provides procedures for the control of mobility when the user equipment (UE) is using the NG radio access network (NG-RAN) and/or non-3GPP access network. The 5GMM protocol also provides control of security for the NAS protocols.

The 5GS session management (5GSM) protocol defined in the present document provides procedures for the handling of 5GS PDU sessions. Together with the bearer control provided by the access stratum, this protocol is used for the control of user plane bearers.

For both NAS protocols the present document specifies procedures for the support of inter-system mobility between the NG-RAN and the evolved universal terrestrial radio access (E-UTRAN) connected to the evolved packet core (EPC) and between the NG-RAN and the non-3GPP access network.

The present document is applicable to the UE, the access and mobility management function (AMF) and to the session management function (SMF) in the 5GS.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.003: "Numbering, addressing and identification".

[3]	3GPP TS 23.122: "Non-Access-Stratum functions related to Mobile Station (MS) in idle mode".

[4]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[5]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[6]	3GPP TS 23.503: " Policy and Charging Control Framework for the 5G System; Stage 2".

[7]	3GPP TS 24.007: "Mobile radio interface signalling layer 3; General aspects".

[8]	3GPP TS 24.008: "Mobile Radio Interface Layer 3 specification; Core Network Protocols; Stage 3".

[9]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[10]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[11]	3GPP TS 24.302: "Access to the 3GPP Evolved Packet Core (EPC) via non-3GPP access networks; Stage 3"

[12]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[13]	3GPP TS 36.355: "Evolved Universal Terrestrial Radio Access (E-UTRA); LTE Positioning Protocol (LPP)".

[14]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[15]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol Specification".

[16]	3GPP TS 38.413: "NG Radio Access Network (NG-RAN); NG Application Protocol (NGAP)".

[17]	IETF RFC 793: "Transmission Control Protocol."

[18]	IETF RFC 768: "User Datagram Protocol".

[19]	IETF RFC 3748: "Extensible Authentication Protocol (EAP)".

[20]	IETF RFC 4191: "Default Router Preferences and More-Specific Routes".

[21]	IETF RFC 4282: "The Network Access Identifier".

[22]	IETF RFC 4303: "IP Encapsulating Security Payload (ESP)".

[23]	IETF RFC 4862: "IPv6 Stateless Address Autoconfiguration".

[24]	IETF RFC 5448: "Improved Extensible Authentication Protocol Method for 3rd Generation Authentication and Key Agreement (EAP-AKA')".

[25]	IETF RFC 7296: "Internet Key Exchange Protocol Version 2 (IKEv2)".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

5GMM-IDLE mode: In this specification, if the term is used standalone, a UE in 5GMM-IDLE mode means the UE can be either in 5GMM-IDLE mode over 3GPP access or in 5GMM-IDLE mode over non-3GPP access.

5GMM-CONNECTED mode: In this specification, if the term is used standalone, a UE in 5GMM-CONNECTED mode means the UE can be either in 5GMM-CONNECTED mode over 3GPP access or in 5GMM-CONNECTED mode over non-3GPP access.

5GMM-IDLE mode over 3GPP access: A UE is in 5GMM-IDLE mode over 3GPP access when no N1 NAS signalling connection between the UE and network over 3GPP access exists. The term 5GMM-IDLE mode over 3GPP access used in the present document corresponds to the term CM-IDLE state for 3GPP access used in 3GPP TS 23.501 [4].

5GMM-CONNECTED mode over 3GPP access: A UE is in 5GMM-CONNECTED mode over 3GPP access when a N1 NAS signalling connection between the UE and network over 3GPP access exists. The term 5GMM-CONNECTED mode over 3GPP access used in the present document corresponds to the term CM-CONNECTED state for 3GPP access used in 3GPP TS 23.501 [4].

5GMM-IDLE mode over non-3GPP access: A UE is in 5GMM-IDLE mode over non-3GPP access no N1 NAS signalling connection between the UE and network over non-3GPP access exists. The term 5GMM-IDLE mode over non-3GPP access used in the present document corresponds to the term CM-IDLE state for non-3GPP access used in 3GPP TS 23.501 [4].

5GMM-CONNECTED mode over non-3GPP access: A UE is in 5GMM-CONNECTED mode over non-3GPP access when it has N1 NAS signalling connection between the UE and network over non-3GPP access exists. The term 5GMM-CONNECTED mode over non-3GPP access used in the present document corresponds to the term CM-CONNECTED state for non-3GPP access used in 3GPP TS 23.501 [4].

Access stratum connection: A peer to peer access stratum connection between either the UE and the NG-RAN for 3GPP access or the UE and the N3IWF for non-3GPP access. The access stratum connection for 3GPP access corresponds to an RRC connection via the Uu reference point. The creation of the access stratum connection for non-3GPP access corresponds to the completion of the IKE_SA_INIT exchange (see IETF RFC 7296 [25]) via the NWu reference point.

DNN based congestion control: Type of congestion control at session management level that is applied to reject session management requests from UEs or release PDU sessions when the associated DNN is congested.

General NAS level congestion control: Type of congestion control at mobility management level that is applied at a general overload or congestion situation in the network, e.g. lack of processing resources.

Last visited registered TAI: A TAI which is contained in the registration area that the UE registered to the network and which identifies the tracking area last visited by the UE.

N1 mode: A mode of a UE allowing access to the 5G core network via the 5G access network.

N1 NAS signalling connection: A peer to peer N1 mode connection between UE and AMF. An N1 NAS signalling connection is either the concatenation of an RRC connection via the Uu reference point and an NG connection via the N2 reference point for 3GPP access, or the concatenation of an IPsec tunnel via the NWu reference point and an NG connection via the N2 reference point for non-3GPP access.

PDU session for LADN: A PDU session with a DNN associated with a LADN.

Registered for emergency services: A UE is registered for emergency services if it has successfully completed initial registration for emergency services or if it has only one PDU session established which is for emergency services.

Selected core network type information: A type of core network (EPC or 5GCN) selected by the UE NAS layer in case of an E-UTRA cell connected to both EPC and 5GCN.

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.501 [4] apply:

5G access network

5G core network

5G QoS flow

5G QoS identifier

5G-GUTI

5G System

5G-S-TMSI

Allowed area

Allowed NSSAI

AMF region

AMF set

Configured NSSAI

Local area data network

Network slice

NG-RAN

Non-allowed area

PDU session

PDU session type

PEI

Requested NSSAI

SUPI

SUCI

For the purposes of the present document, the following terms and definitions given in 3GPP TS 24.301 [10] apply:

UE configured to use AC11 – 15 in selected PLMN

For the purposes of the present document, the following terms and definitions given in 3GPP TS 33.501 [12] apply:

5G security context

5G NAS security context

Current 5G security context

Full native 5G security context

Native 5G security context

For the purposes of the present document, the following terms and definitions given in 3GPP TS 38.413 [16] apply:

NG connection

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

4G-GUTI	4G-Globally Unique Temporary Identifier

5GCN	5G Core Network

5G-GUTI	5G-Globally Unique Temporary Identifier

5GMM	5GS Mobility Management

5GS	5G System

5GSM	5GS Session Management

5G-S-TMSI	5G S-Temporary Mobile Subscription Identifier

5G-TMSI	5G Temporary Mobile Subscription Identifier

5QI	5G QoS Identifier

AKA	Authentication and Key Agreement

AMF	Access and Mobility Management Function

DL	Downlink

DN	Data Network

DNN	Data Network Name

E-UTRAN	Evolved Universal Terrestrial Radio Access Network

EAP-AKA'	Improved Extensible Authentication Protocol method for 3rd generation Authentication and Key Agreement

EPD	Extended Protocol Discriminator

EMM	EPS Mobility Management

EPC	Evolved Packet Core Network

EPS	Evolved Packet System

ESM	EPS Session Management

GFBR	Guaranteed Flow Bit Rate

GUAMI	Globally Unique AMF Identifier

LADN	Local Area Data Network

LMF	Location Management Function

LPP	LTE Positioning Protocol

MFBR	Maximum Flow Bit Rate

MICO	Mobile Initiated Connection Only

N3IWF	Non-3GPP Inter-Working Function

NAI	Network Access Identifier

NR	New Radio

ngKSI	Key Set Identifier for Next Generation Radio Access Network

NSSAI	Network Slice Selection Assistance Information

QFI	QoS Flow Identifier

QoS	Quality of Service

QRI	QoS Rule Identifier

RQA	Reflective QoS Attribute

RQI	Reflective QoS Indication

(R)AN	(Radio) Access Network

S-NSSAI	Single NSSAI

SA	Security Association

SNN	Serving Network Name

SMF	Session Management Function

TA	Tracking Area

TAC	Tracking Area Code

TAI	Tracking Area Identity

UL	Uplink

URSP	UE Route Selection Policy

# 4	General

## 4.1	Overview

## 4.2	Coordination between the protocols for 5GS mobility management and 5GS session management

A 5GS session management (5GSM) message is piggybacked in specific 5GS mobility management (5GMM) transport messages. To this purpose, the 5GSM messages can be transmitted in an information element in the 5GMM transport messages. In this case, the UE, the AMF and the SMF execute the 5GMM procedure and the 5GSM procedure in parallel. The success of the 5GMM procedure is not dependent on the success of the piggybacked 5GSM procedure.

The UE can only initiate the 5GSM procedure when there is a 5GMM context established at the UE and the AMF can only forward the 5GSM message for the 5GSM procedure initiated by the SMF when there is a 5GMM context established at the AMF.

Except for the 5GMM procedures piggybacking 5GSM messages, during 5GMM procedures the UE and the AMF shall suspend the transmission of 5GSM messages.

## 4.3	UE domain selection

### 4.3.1	UE’s usage setting

The UE’s usage setting defined in 3GPP TS 24.301 [10] applies to voice capable UEs in 5GS and indicates whether the UE has preference for voice services over data services or vice-versa, where:

a)	voice services include IMS voice; and

b)	data services include any kind of user data transfer without a voice media component.

The UE’s usage setting can be set to:

a)	"voice centric"; or

b)	"data centric".

If the UE is capable of S1 mode, there is a single UE’s usage setting at the UE which applies to both 5GS and EPS.

Editor's note:	The definition of the UE’s usage setting in TS 24.301 needs to be updated to be applicable to 5GS.

### 4.3.2	Domain selection for UE originating sessions / calls

The behaviour of the UE for domain selection is determined by:

a)	the UE usage setting;

b)	the availability of IMS voice; and

c)	whether the UE operates in single-registration mode or dual-registration mode (see 3GPP TS 23.501 [4]).

In the present document, "IMS voice not available" refers to one of the following conditions:

a)	the UE does not support IMS voice;

b)	the UE supports IMS voice, but the network indicates in the REGISTRATION ACCEPT message that IMS voice over PS sessions are not supported; or

c)	the UE supports IMS voice, the network indicates in the REGISTRATION ACCEPT message that IMS voice over PS sessions are supported, but the upper layers:

1)	provide no indication that the UE is available for voice call in the IMS within a manufacturer determined period of time; or

2)	indicate that the UE is not available for voice calls in the IMS.

NOTE:	If conditions a and b evaluate to false, the upper layers need time to attempt IMS registration. In the event an indication from the upper layers that the UE is available for voice calls in the IMS takes longer than the manufacturer determined period of time (e.g. due to delay when attempting IMS registration or due to delay in obtaining a QoS flow for SIP signalling), the NAS layer assumes the UE is not available for voice calls in the IMS.

Other conditions may exist but these are implementation specific.

In the present document, "IMS voice available" applies when "IMS voice not available" does not apply.

When IMS voice is not available, if the UE’s usage setting is "voice centric" and the UE operates in single-registration mode, the UE shall disable the N1 mode radio capabilities (see subclause 4.9), and attempt to select an E-UTRA cell connected to EPC. If such a cell is found, the UE shall then perform voice domain selection procedures as defined in 3GPP TS 24.301 [10].

When IMS voice is not available, if the UE’s usage setting is "voice centric" for 5GS and the UE operates in dual-registration mode, the UE may disable the N1 mode radio capabilities (see subclause 4.9), and attempt to select an E-UTRA cell connected to EPC. If such a cell is found, the UE shall then perform voice domain selection procedures as defined in 3GPP TS 24.301 [10].

### 4.3.3	Change of UE’s usage setting

Whenever the UE's usage setting changes, the UE shall execute procedures according to table 4.3.3.1 and table 4.3.3.2:

a)	The UE is in single-registration mode

Table 4.3.3.1: Change of UE's usage setting for a UE in single-registration mode

| UE's usage setting change | Procedure to execute |
| --- | --- |
| From "data centric" to "voice centric" and "IMS voice not available" | Disable the N1 mode radio capabilities (see subclause 4.9) |
| From "voice centric" to "data centric" and the N1 mode radio capabilities are disabled at the UE | Re-enable the N1 mode radio capabilities (see subclause 4.9) |

b)	The UE is in dual-registration mode

Table 4.3.3.2: Change of UE's usage setting for a UE in dual-registration mode

| UE's usage setting change | Procedure to execute |
| --- | --- |
| From "data centric" to "voice centric" and "IMS voice not available" | Disable the N1 mode radio capabilities (see subclause 4.9) |
| From "voice centric" to "data centric" and the N1 mode radio capabilities are disabled at the UE | Re-enable the N1 mode radio capabilities (see subclause 4.9) |

### 4.3.4	Change or determination of IMS voice availability

Whenever the IMS voice availability is determined or changes, the UE shall execute procedures according to table 4.3.4.1 and table 4.3.4.2:

a)	The UE is in single-registration mode

Table 4.3.4.1: Change of IMS voice availability for a UE in single-registration mode

| Change of IMS voice available condition | Procedure to execute |
| --- | --- |
| IMS voice not available and the UE’s usage setting is "voice centric" | Disable the N1 mode radio capabilities (see subclause 4.9) |
| IMS voice available and the N1 mode radio capabilities are disabled at the UE | Re-enable the N1 mode radio capabilities (see subclause 4.9) |

b)	The UE is in dual-registration mode

Table 4.3.4.2: Change of IMS voice availability for a UE in dual mode

| Change of IMS voice available condition | Procedure to execute |
| --- | --- |
| IMS voice not available and the UE’s usage setting is "voice centric" | Disable the N1 mode radio capabilities (see subclause 4.9) |
| IMS voice available and the N1 mode radio capabilities are disabled at the UE | Re-enable the N1 mode radio capabilities (see subclause 4.9) |

## 4.4	NAS security

### 4.4.1	General

This clause describes the principles for the handling of 5G NAS security contexts in the UE and in the AMF and the procedures used for the security protection of NAS messages between the UE and the AMF. Security protection involves integrity protection and ciphering of the 5GMM and 5GSM NAS messages.

The signalling procedures for the control of NAS security are part of the 5GMM protocol and are described in detail in clause 5.

NOTE:	The use of ciphering in a network is an operator option. In this subclause, for the ease of description, it is assumed that ciphering is used, unless explicitly indicated otherwise. Operation of a network without ciphering is achieved by configuring the AMF so that it always selects the "null ciphering algorithm", NEA0.

### 4.4.2	Handling of 5G NAS security contexts

The security parameters for authentication, integrity protection and ciphering are tied together in a 5G NAS security context and identified by a key set identifier (ngKSI). The relationship between the security parameters is defined in 3GPP TS 33.501 [12].

Before security can be activated, the AMF and the UE need to establish a 5G NAS security context. Usually, the 5G NAS security context is created as the result of a primary authentication and key agreement procedure between the AMF and the UE. Alternatively, during inter-system handover from S1 mode to N1 mode, the AMF and the UE derive a mapped 5G security context from an EPS security context that has been established while the UE was in S1 mode.

The 5G NAS security context is taken into use by the UE and the AMF, when the AMF initiates a NAS security mode control procedure or during the inter-system handover procedure from S1 mode to N1 mode. The 5G NAS security context which has been taken into use by the network most recently is called current 5G NAS security context. This current 5G security context can be of type native or mapped, i.e. originating from a native 5G NAS security context or mapped 5G NAS security context.

The key set identifier ngKSI is assigned by the AMF either during the primary authentication and key agreement procedure procedure or, for the mapped 5G NAS security context, during the inter-system handover procedure. The ngKSI consists of a value and a type of security context parameter indicating whether a 5G NAS security context is a native 5G NAS security context or a mapped 5G NAS security context. When the 5G NAS security context is a native 5G NAS security context, the ngKSI has the value of KSIAMF, and when the current 5G NAS security context is of type mapped, the ngKSI has the value of KSIASME.

## 4.5	Unified access control

### 4.5.1	General

When the UE wants to access the 5GS, the UE must first perform access control checks to determine if the access is allowed. Access control checks shall be perfomed for the access attempts defined by the following list of events:

a)	the UE is in 5GMM-IDLE mode over 3GPP access and an event that requires a transition to 5GMM-CONNECTED mode occurs;

b)	the UE is in 5GMM-CONNECTED mode over 3GPP access or 5GMM-CONNECTED mode with RRC inactive indication and one of the following events occurs:

1)	5GMM receives an MO-MMTEL-voice-call-started indication, an MO-MMTEL-video-call-started indication or an MO-SMSoIP-attempt-started indication from upper layers;

2)	5GMM receives a request from upper layers to send a mobile originated SMS over NAS;

3)	5GMM receives a request from upper layers to send an UL NAS TRANSPORT message for the purpose of PDU session establishment;

4)	5GMM receives a request from upper layers to send an UL NAS TRANSPORT message for the purpose of PDU session modification; and

5)	5GMM receives a request to re-establish the user plane for an existing PDU session.

NOTE 1:	5GMM specific procedures initiated by NAS in 5GMM-CONNECTED mode are not subject to access control. E.g. a registration procedure after PS handover will not be prevented by access control.

Editor's note: For the case that the UE is in 5GMM-IDLE  mode when 5GMM receives a request from an upper layer to send an UL NAS TRANSPORT message, so that the procedure for the initial NAS message is followed by a NAS transport procedure, resolution of double barring is FFS.

Editor's note:	Whether other events need to be considered in 5GMM-CONNECTED mode or 5GMM-CONNECTED mode with RRC inactive indication, is FFS.

When the NAS detects one of the above events, the NAS needs to perform the mapping of the kind of request to one or more access identities and one access category and lower layers will perform access barring checks for that request based on the determined access identities and access category.

NOTE 2:	The NAS is aware of the above events through indications provided by upper layers or when determing the need to start 5GMM procedures through normal NAS behaviour, or both.

To determine the access identities and the access category for a request, the NAS checks the reason for access, types of service requested and profile of the UE including UE configurations, against a set of access identities and access categories defined in 3GPP TS 22.261 [3], namely:

a)	a set of standardized access identities;

b)	a set of standardized access categories; and

c)	a set of operator-defined access categories, if available.

For the purpose of determining the applicable access identities from the set of standardized access identities defined in 3GPP TS 22.261 [3], the NAS shall follow the requirements set out in subclause 4.5.2 and the rules and actions defined in table 4.5.2.2.2.

For the purpose of determining the applicable access category from the set of standardized access categories and operator-defined access categories defined in 3GPP TS 22.261 [3], the NAS shall follow the requirements set out in subclause 4.5.2 and the rules and actions defined in table 4.5.2.2.

### 4.5.2	Determination of the access identities and access category associated with a request for access

When the UE needs to initiate an access attempt in one of the events listed in subclause 4.5.1, the UE shall determine one or more access identities from the set of standardized access identities, and one access category from the set of standardized access categories and operator-defined access categories, to be associated with that access attempt.

The set of the access identities applicable for the request is determined by the UE in the following way:

a)	for each of the access identities 1, 2, 11, 12, 13, 14 and 15 in table 4.5.2.2.1, the UE shall check whether the access identity is applicable in the selected PLMN, if a new PLMN is selected, or otherwise if it is applicable in the RPLMN or equivalent PLMN; and

b)	if none of the above access identities is applicable, then access identity 0 is applicable.

Table 4.5.2.2.1: Access Identities (according to 3GPP TS 22.261 [3], table 6.22.2.2-1)

| Access Identity number | UE configuration |
| --- | --- |
| 0 | UE is not configured with any parameters from this table |
| 1 (NOTE 1) | UE is configured for Multimedia Priority Service (MPS). |
| 2 (NOTE 2) | UE is configured for Mission Critical Service (MCS). |
| 3-10 | Reserved for future use |
| 11 (NOTE 3) | Access Class 11 is configured in the UE. |
| 12 (NOTE 3) | Access Class 12 is configured in the UE. |
| 13 (NOTE 3) | Access Class 13 is configured in the UE. |
| 14 (NOTE 3) | Access Class 14 is configured in the UE. |
| 15 (NOTE 3) | Access Class 15 is configured in the UE. |
| NOTE 1: Access Identity 1 is used to provide overrides according to the subscription information in UEs configured for MPS. The subscription information defines whether an overide applies to UEs within one of the following categories: a) UEs that are configured for MPS; b) UEs that are configured for MPS and are in the PLMN listed as most preferred PLMN of the country where the UE is roaming in the operator-defined PLMN selector list or in their HPLMN or in a PLMN that is equivalent to their HPLMN; c) UEs that are configured for MPS and are in their HPLMN or in a PLMN that is equivalent to it. NOTE 2: Access Identity 2 is used to provide overrides according to the subscription information in UEs configured for MCS. The subscription information defines whether an overide applies to UEs within one of the following categories: a) UEs that are configured for MCS; b) UEs that are configured for MCS and are in the PLMN listed as most preferred PLMN of the country where the UE is roaming in the operator-defined PLMN selector list or in their HPLMN or in a PLMN that is equivalent to their HPLMN; c) UEs that are configured for MCS and are in their HPLMN or in a PLMN that is equivalent to it. NOTE 3: Access Identities 11 and 15 are valid in Home PLMN only if the EHPLMN list is not present or in any EHPLMN. Access Identities 12, 13 and 14 are valid in Home PLMN and visited PLMNs of home country only. For this purpose the home country is defined as the country of the MCC part of the IMSI. |  |

In order to determine the access category applicable for the access attempt, the NAS shall check the rules in table 4.5.2.2, and use the access category for which there is a match for barring check. If the access attempt matches more than one rule, the access category of the lowest rule number shall be selected.

Table 4.5.2.2: Mapping table for access categories

| Rule # | Type of access attempt | Requirements to be met | Access Category |
| --- | --- | --- | --- |
| 1 | Response to paging | Access attempt is for MT access | 0 (= MT_acc) |
| 2 | Emergency | UE is attempting access for an emergency session (NOTE 1, NOTE 2) | 2 (= emergency) |
| 3 | Access attempt for operator-defined access category | UE was provided with operator-defined access categories for the current PLMN, and access attempt is matching criteria of an operator-defined access category | 32-63 (= based on operator classification) |
| 4 | Access attempt for delay tolerant service | UE is configured for delay tolerant service, the PLMN is broadcasting one of the categories a, b or c, and the UE is a member of the broadcasted category in the selected PLMN or RPLMN/equivalent PLMN (NOTE 3) | 1 (= delay tolerant) |
| 5 | MO MMTel voice call | Access attempt is for MO MMTel voice call or for NAS signalling connection recovery during ongoing MO MMTel voice call (NOTE 2) | 4 (= MO MMTel voice) |
| 6 | MO MMTel video call | Access attempt is for MO MMTel video call or for NAS signalling connection recovery during ongoing MO MMTel video call (NOTE 2) | 5 (= MO MMTel video) |
| 7 | MO SMS over NAS or MO SMSoIP | Access attempt is for MO SMS or SMSoIP transfer or for NAS signalling connection recovery during ongoing MO SMS or SMSoIP transfer (NOTE 2) | 6 (= MO SMS and SMSoIP) |
| 8 | UE NAS initiated 5GMM specific procedures | Access attempt is for MO signalling | 3 (= MO_sig) |
| 9 | UE NAS initiated 5GMM connection management procedures or 5GMM NAS transport procedure | Access attempt is for MO data | 7 (= MO_data) |
| NOTE 1: This includes 5GMM specific procedures while the service is ongoing and 5GMM connection management procedures required to establish a PDU session with request type = "emergency" or to re-establish radio bearers for such a PDU session. NOTE 2: Access for the purpose of NAS signalling connection recovery during an ongoing service is mapped to the access category of the ongoing service in order to derive an RRC establishment cause, but barring checks will be skipped for this access attempt. NOTE 3: If the UE selects a new PLMN, then the selected PLMN is used to check the membership; otherwise the UE uses the RLPMN or a PLMN equivalent to the RPLMN. |  |  |  |

Editor's note:	The need for and the derivation of the RRC establishment cause (or its equivalence in 5GS) requires further work by RAN2.

### 4.5.3	Operator-defined access categories

Operator-defined access categories can be signalled to the UE using NAS signalling. Each operator-defined category consists of the following parameters:

Editor's note:	The encoding of the operator-defined access categories is FFS.

Editor's note:	Whether the operator-defined access categories are sent to the UE in a CONFIGURATION UPDATE COMMAND message, a DL NAS TRANSPORT message or another NAS message, is FFS.

a)	a precedence value which indicates in which order the UE shall evaluate the operator-defined categories for a match;

b)	an access category number in the 32-63 range that uniquely identifies the access category in the PLMN in which the access categories are being sent to the UE; and

c)	one or more access category criteria type and associated access category criteria type values. The access category criteria type can be set to one of the following:

1)	DNN name;

2)	5QI;

Editor's note:	Whether the 5QI is a suitable access category criteria type is FFS.

3)	OS Id + OS App Id of application triggering the access attempt; or

4)	S-NSSAI.

Editor's note:	Other access category criteria types, in particular whether QFI is a suitable parameter, are FFS.

NOTE:	An access category criteria type can be associated with more than one access category criteria values. In this case, the access attempt matches the access category if the access criteria for the access attempt matches any of the associated access criteria type values.

If the UE is configured with operator-defined access categories for a PLMN, then access control in 5GMM-CONNECTED mode and in 5GMM-CONNECTED mode with RRC inactive indication will only be performed for the events 1) to 5) defined in subclause 4.5.1.

Editor's note:	Whether access control in 5GMM-CONNECTED mode and in 5GMM-CONNECTED mode with RRC inactive indication also needs to be performed when 5GMM receives an indication that an application is started including the OS Id + OS App Id from the upper layers is FFS.

Upon receiving a NAS signalling message with operator-defined access category definitions, the UE shall store the operator-defined access category definitions for the registered PLMN.

### 4.5.4	Access control and checking

Editor's note:	It is FFS whether the alleviation of barring is or is not associated with the access category for which barring is alleviated.

#### 4.5.4.1	Access control and checking in 5GMM-IDLE mode

When the UE is in 5GMM-IDLE mode, upon receiving a request from the upper layers for an access attempt, the NAS shall categorize the access attempt into access identities and an access category following subclause 4.5.2, table 4.5.2.1 and table 4.5.2.2, and subclause 4.5.3, and provide the applicable access identities and the access category to the lower layers for the purpose of access control checking.

NOTE:	The access barring check is performed by the lower layers.

Editor's note:	Whether the applicable access identities also need to be provided to the lower layers based on other triggers is FFS.

If the lower layers indicate that the access attempt is allowed, the NAS shall initiate the procedure to send the initial NAS message for the access attempt.

If the lower layers indicate that the access attempt is barred, the NAS shall not initiate the procedure to send the initial NAS message for the access attempt. Additionally, if the event which triggered the access attempt was an MO-MMTEL-voice-call-started indication, an MO-MMTEL-video-call-started indication or an MO-SMSoIP-attempt-started indication, the NAS shall notify the upper layers that the access attempt is barred. Upon receiving an indication from the lower layers that access is granted, the NAS may initiate the procedure to send the initial NAS message, if still needed.

#### 4.5.4.2	Access control and checking in 5GMM-CONNECTED mode and in 5GMM-CONNECTED mode with RRC inactive indication

When the UE is in 5GMM-CONNECTED mode or 5GMM-CONNECTED mode with RRC inactive indication, upon detecting one of events 1) through 5) listed in subclause 4.5.1, the NAS shall categorize the corresponding access attempt into access identities and an access category following subclause 4.5.2, table 4.5.2.1 and table 4.5.2.2, and subclause 4.5.2.3, and provide the access identities and the access category to the lower layers for the purpose of access control checking.

Editor's note:	Whether one or more applicable access identities also need to be provided to the lower layers based on other triggers is FFS.

If the lower layers indicate that the access attempt is allowed, the NAS shall take the following action depending on the event which triggered the access attempt:

a)	if the event which triggered the access attempt was an MO-MMTEL-voice-call-started indication, an MO-MMTEL-video-call-started indication or an MO-SMSoIP-attempt-started indication, the NAS shall notify the upper layers that the access attempt is allowed;

b)	if the event which triggered the access attempt was a request from upper layers to send a mobile originated SMS over NAS, 5GMM shall initiate the NAS transport procedure as specified in subclause 5.4.5 to send the SMS in an UL NAS TRANSPORT message;

c)	if the event which triggered the access attempt was a request from upper layers to establish a new PDU session, 5GMM shall initiate the NAS transport procedure as specified in subclause 5.4.5 to send the PDU SESSION ESTABLISHMENT REQUEST message;

d)	if the event which triggered the access attempt was a request from upper layers to modify an existing PDU session, 5GMM shall initiate the NAS transport procedure as specified in subclause 5.4.5 to send the PDU SESSION MODIFICATION REQUEST message; and

e)	if the event which triggered the access attempt was a request to re-establish the user plane for an existing PDU session, 5GMM shall initiate the service request procedure as specified in subclause 5.6.1.

If the lower layers indicate that the access attempt is barred, the NAS shall take the following action depending on the event which triggered the access attempt:

-	if the event which triggered the access attempt was an MO-MMTEL-voice-call-started indication, an MO-MMTEL-video-call-started indication or an MO-SMSoIP-attempt-started indication, the NAS shall notify the upper layers that the access attempt is barred. Upon receiving an indication from the lower layers that access is granted, the NAS shall notify the upper layers that access is granted;

NOTE:	In this case prohibiting the initiation of the MMTEL voice session, MMTEL video session or prohibiting sending of the SMS over IP is performed by the upper layers.

-	if the event which triggered the access attempt was a request from upper layers to send a mobile originated SMS over NAS, 5GMM shall not initiate the NAS transport procedure as specified in subclause 5.4.5 to send the SMS in an UL NAS TRANSPORT message. Upon receiving an indication from the lower layers that access is granted, 5GMM may initiate the NAS transport procedure as specified in subclause 5.4.5 to send the SMS in an UL NAS TRANSPORT message, if still needed;

-	if the event which triggered the access attempt was a request from upper layers to establish a new PDU session, 5GMM shall not initiate the NAS transport procedure to send the PDU SESSION ESTABLISHMENT REQUEST message. Upon receiving an indication from the lower layers that access is granted, the NAS may initiate the NAS transport procedure as specified in subclause 5.4.5, if still needed;

-	if the event which triggered the access attempt was a request from upper layers to modify an existing PDU session modification, 5GMM shall not initiate the NAS transport procedure to send the PDU SESSION MODIFICATION REQUEST message. Upon receiving an indication from the lower layers that access is granted, the NAS may initiate the NAS transport procedure as specified in subclause 5.4.5, if still needed; and

-	if the event which triggered the access attempt was a request to re-establish the user plane for an existing PDU session, the NAS shall not initiate the service request procedure as specified in subclause 5.6.1. Upon receiving an indication from the lower layers that access is granted, the NAS may initiate the service request procedure as specified in subclause 5.6.1, if still needed.

### 4.5.5	Exception handling and avoiding double barring

There are several services for which the NAS needs to be informed when the service starts and stops,

a)	because, while the service is ongoing, the mapping of other access attempts to a specific access category can be affected; and

b)	in order to avoid double barring at the start of these services.

These services are:

a)	emergency service;

b)	MMTEL voice;

c)	MMTEL video;

d)	SMSoIP; and

e)	SMS over NAS.

The UE considers an emergency service i) as started when 5GMM receives a request from upper layers to attach for emergency services or to establish a PDU session with request type = "initial emergency request". It considers the emergency service as stopped when this PDU session is released.

While an emergency service i) is ongoing, any access attempt triggered by the initiation of a registration, deregistration or service request procedure is mapped to access category 2 = emergency.

Once the emergency service service has successfully passed access control, then as long as the service is ongoing, the following access attempts are allowed to proceed without further access control checking in order to avoid double barring:

a)	any service request procedure related to the PDU session associated with request type = "initial emergency request"; and

b)	any service request or registration procedure initiated in 5GMM-IDLE mode for the purpose of NAS signalling connection recovery.

NOTE 1:	Although the access control checking is skipped, the mapping is performed in order to derive an RRC establishment cause.

Editor's note:	The need for and the derivation of the RRC establishment cause (or its equivalence in 5GS) requires further work by RAN2.

Editor's note:	If MT LCS are used during an emergency service to locate the UE, the 5GMM will transfer LCS messages to the network. It is FFS based on which criteria the UE will determine to skip access control for these messages.

For services b) to e) the 5GMM receives explicit start and stop indications from the upper layers.

Editor's note:	Whether explicit start and stop indications for SMS over NAS need to be defined is FFS.

Once the service has successfully passed access control, then as long as the service is ongoing, the following access attempts are allowed to proceed without further access control checking in order to avoid double barring:

a)	for services b), c) and d), any service request procedure related to the PDU session established for DNN = "IMS"; and

Editor's note:	It has been observed that some operators use an APN different from "IMS" for SMSoIP. How to handle that case when a DNN different from "IMS" is used, is FFS.

b)	for services b), c), d) and e), any service request or registration procedure initiated in 5GMM-IDLE mode for the purpose of NAS signalling connection recovery.

While an MMTEL voice or video call, or SMS over IMS is ongoing, any of the above access attempts is mapped to access category 5, 6 or 7 respectively.

NOTE 2:	Although the access control checking is skipped, the mapping is performed in order to derive an RRC establishment cause.

Editor's note:	The need for and the derivation of the RRC establishment cause (or its equivalence in 5GS) requires further work by RAN2.

## 4.6	Network slicing

### 4.6.1	General

The 5GS supports network slicing as described in 3GPP TS 23.501 [4]. Within a PLMN, a network slice is identified by an S-NSSAI, which is comprised of a slice/service type (SST) and a slice differentiator (SD). Inclusion of an SD in an S-NSSAI is optional. A set of one or more S-NSSAIs is called the NSSAI. The following NSSAIs are defined in the current specification (see 3GPP TS 23.501 [4]):

a)	configured NSSAI;

b)	requested NSSAI;

c)	allowed NSSAI;

d)	subscribed S-NSSAIs;

e)	rejected NSSAI for the current PLMN;

f)	rejected NSSAI for the current PLMN and registration area combination.

The HPLMN may configure a UE with the configured NSSAI per PLMN. In addition, the HPLMN may configure a UE with a configured NSSAI not associated with a PLMN. The UE shall have a single configured NSSAI not associated with a PLMN and consider the configured NSSAI as valid in a PLMN for which the UE has neither configured NSSAI nor allowed NSSAI.

### 4.6.2	Mobility management aspects

#### 4.6.2.1	General

Upon registration to a PLMN, the UE shall send to the AMF the requested NSSAI which includes one or more S-NSSAIs of the configured NSSAI or the allowed NSSAI for the PLMN and corresponds to the network slice(s) to which the UE wishes to register if the UE has a configured NSSAI or an allowed NSSAI for the PLMN defined in 3GPP TS 23.501 [4]. The UE NAS shall also provide the lower layers with the requested NSSAI, if available. If the UE has neither a configured NSSAI nor an allowed NSSAI for a valid PLMN, the UE does not send a requested NSSAI when requesting registration towards the PLMN. In roaming scenarios the UE may also provide the mapping of each S-NSSAI of the requested NSSAI to the S-NSSAIs of the Configured NSSAI for the HPLMN, if available. The AMF verifies if the requested NSSAI is permitted based on the subscribed S-NSSAIs in the UE subscription and optionally the mapping information provided by the UE, and if so then the AMF shall provide the UE with the allowed NSSAI for the PLMN, and may also provide the UE with the mapping of the allowed NSSAI for the PLMN to the configured NSSAI for the HPLMN if available. The AMF may also query the NSSF to determine the allowed NSSAI for a given registration area as defined in 3GPP TS 23.501 [4].

The set of network slice(s) for a UE can be changed at any time while the UE is registered to a PLMN, and the change may be initiated by the network, or the UE. In this case, the allowed NSSAI and associated registration area may be changed during the registration procedure. The network may notify the UE of the change of the supported network slice(s) in order to trigger the registration procedure. Change in the allowed NSSAI may lead to AMF relocation subject to operator policy. See subclause 5.4.4 describing the generic UE configuration update procedure for further details.

#### 4.6.2.2	NSSAI storage

If available, the configured NSSAI(s), the allowed NSSAI(s), the temporarily rejected NSSAI(s), and the permanently rejected NSSAI(s) shall be stored in a non-volatile memory in the ME together with the SUPI from the USIM per PLMN. Those NSSAIs can only be used if the SUPI from the USIM matches the SUPI stored in the non-volatile memory of the ME; else the UE shall delete them.

Each of the configured NSSAI and the allowed NSSAI stored in the UE is a set composed of at most 8 S-NSSAIs. Each of the configured NSSAI, the allowed NSSAI, and the rejected NSSAI is associated with a PLMN identity. The S-NSSAI(s) in the rejected NSSAI for the current PLMN and registration area combination are further associated with a registration area where the rejected S-NSSAI(s) is not available. There shall be no duplicated PLMN identities in each of the list of configured NSSAI(s), allowed NSSAI(s), rejected NSSAI(s) for the current PLMN, and rejected NSSAI(s) for the current PLMN and registration area combination.

Editor's note:	 The maximum number of S-NSSAI in the configured NSSAI provisioned by the network needs to be re-evaluated.

The UE stores NSSAIs as follows:

a)	The configured NSSAI shall be stored until a new configured NSSAI is received for a given PLMN. When provisioned with a new configured NSSAI for a PLMN, the UE shall replace any stored configured NSSAI for this PLMN with the new configured NSSAI, and delete any stored allowed NSSAI, rejected NSSAI for the current PLMN, and rejected NSSAI for the current PLMN and registration area combination. If the UE receives one or more S-NSSAIs from the network during the PDN connection establishment procedure in EPS as specified in 3GPP TS 24.301 [10], the UE may store the received S-NSSAI(s) in the configured NSSAI, if not already in the configured NSSAI. If the configured NSSAI for the registered PLMN is updated, the UE shall perform the mobility registration updating procedure and the UE shall store the allowed NSSAI or rejected NSSAI in case any NSSAI is received from the AMF.

b)	The allowed NSSAI shall be stored until a new allowed NSSAI is received for a given PLMN. When a new allowed NSSAI for a PLMN is received, the UE shall replace any stored allowed NSSAI for the current PLMN with this new allowed NSSAI. The UE shall remove, if any, the stored rejected S-NSSAI(s) which is/are included in the new allowed NSSAI for the current PLMN.

c)	When the UE receives the S-NSSAI(s) included in rejected NSSAI in the REGISTRATION ACCEPT message, the UE shall store the S-NSSAI(s) into the rejected NSSAI based on the associated rejection cause(s). Once the UE is deregistered, the  rejected NSSAI for the current PLMN and the rejected NSSAI for the current PLMN and registration combination shall be deleted. The UE shall delete, if any, the stored rejected NSSAI for the current PLMN and registration area combination if the UE moves out of the registration area.

NOTE:	Whether the UE stores the allowed NSSAI also when the UE is switched off is implementation specific.

The network may provide to the UE the mapping information as to how the S-NSSAI(s) in the configured NSSAI maps to S-NSSAI(s) in the configured NSSAI for the HPLMN in the UE. The UE shall store this mapping information and use this information for registration.

The network may provide to the UEthe  mapping information as to how the S-NSSAI(s) in the allowed NSSAI maps to S-NSSAI(s) in the configured NSSAI for the HPLMN in the UE (see subclauses 5.5.1.2 and 5.5.1.3). The UE shall store this mapping information in the ME and use this information for e.g., the PDU session establishment procedure (see subclause 4.6.3).

### 4.6.3	Session management aspects

#### 4.6.3.1	General

In order to enable PDU transmission in a network slice, the UE may request establishment of a PDU session in a network slice towards a data network (DN) which is associated with an S-NSSAI and a data network name (DNN) if there is no established PDU session adequate for the PDU transmission. The S-NSSAI included is part of allowed NSSAI of the serving PLMN, which is an S-NSSAI value valid in the serving PLMN, and in roaming scenarios the mapped configured S-NSSAI for the HPLMN is also included for the PDU session if available. See subclause 6.4.1 for further details. The UE determines whether to establish a new PDU session or use one of the established PDU session(s) based on  the URSP rules which include S-NSSAIs, if any (see subclause 6.2.x).

## 4.7	NAS over non-3GPP access

### 4.7.1	General

From the UE's NAS perspective, in general the procedures and messages defined for 5GMM and 5GSM are used over non-3GPP access as over 3GPP access. However, a number of aspects are different

### 4.7.2	5GS mobility management aspects

The mobility management procedures defined over 3GPP access are re-used over non-3GPP access with the following exceptions:

a)	the registration status of the UE's 3GPP access and non-3GPP access 5GMM state machine instances are independent and can be different;

b)	single-registration mode and dual-registration mode do not apply for 5GMM over non-3GPP access;

c)	the RPLMN over non-3GPP access can be different from the RPLMN over 3GPP access. The MCC of the RPLMN over 3GPP access and the MCC of the RPLMN over the non-3GPP access can also be different;

d)	the registration for 3GPP access and for non-3GPP access are performed separately. Like for 3GPP access, an access stratum connection exists before the UE can perform the registration procedure for non-3GPP access. As over non-3GPP access the 5GS operates one single common registration area for an entire PLMN, which is associated with one fixed well-known N3GPP TAI, list management of registration areas is not required, and registration updating due to registration area change with the registered PLMN is not performed. Furthermore, the periodic registration update procedure is also not performed. New registration at change of PLMN is required;

e)	the 5GMM over non-3GPP access considers that the N1 NAS signalling connection is established when the lower layers indicate that the access stratum connection is established succcessfully;

f)	the UE-initiated service request procedure via non-3GPP access is supported. Upon indication from the lower layers of non-3GPP access, that the access stratum conncetion is established between the UE and the network, the UE in 5GMM-REGISTERED state and in 5GMM-IDLE mode over non-3GPP access shall initiate the service request procedure via non-3GPP access to re-activate user-plane connections for all PDU sessions associated with non-3GPP access, but not PDU sessions associated with 3GPP access;

g)	network-initiated service request procedure is not performed via non-3GPP access; and

h)	paging procedure is not performed via non-3GPP access.

### 4.7.3	5GS session management aspects

The session management procedures defined over 3GPP access are re-used over non-3GPP access.

Editor's note:	Further exceptions are possible and are FFS.

## 4.8	Interworking with E-UTRAN connected to EPC

### 4.8.1	General

In order to interwork with E-UTRAN connected to EPC, the UE supporting both S1 mode and N1 mode can operate in single-registration mode or dual-registration mode (see 3GPP TS 23.501 [4]). Support of single-registration mode is mandatory for UEs supporting both S1 mode and N1 mode.

During the attach procedure or initial registration procedure, the mode for intersystem change is selected if the UE supports both S1 mode and N1 mode, and the network supports intersystem change.

### 4.8.2	Coordination between 5GMM and EMM with N26 interface

##### 4.8.2.1	Handling of the 5G security context

To enable re-use of a previously established 5G security context when returning to 5GC, the UE shall store the 5G-GUTI and the native 5G security context upon intersystem change from N1 mode to S1 mode in 5GMM-IDLE and. Upon intersystem change from S1 mode to N1 mode in the same PLMN, the UE includes the stored 5G-GUTI in the Additional GUTI IE in the REGISTRATION REQUEST message.

### 4.8.3	Single-registration mode without N26 interface

At intersystem change from N1 mode to S1 mode in EMM-IDLE mode when at least one PDU session context supporting interworking to EPS is active, the UE shall proceed as follows:

a)	if the UE supports sending an ATTACH REQUEST message containing a PDN CONNECTIVITY REQUEST message with request type set to "handover" to transfer a PDU session from N1 mode to S1 mode and the UE has received an "interworking without N26 supported" indication from the network, the UE shall:

1)	enter substates EMM-DEREGISTERED.NORMAL-SERVICE and 5GMM-REGISTERED.NO-CELL-AVAILABLE;

2)	map each PDU session context supporting interworking to EPS which is in state PDU SESSION ACTIVE or PDU SESSION MODIFICATION PENDING to a default EPS bearer context; and

Editor's note:	Handling of EPS bearer context states and PDU session states is FFS.

3)	initiate an EPS attach procedure and include a PDN CONNECTIVITY REQUEST message with request type set to "handover" in the ATTACH REQUEST message to activate a default EPS bearer context for one of the PDU sessions for which interworking to EPS is supported.

After successful completion of the attach procedure, the UE shall attempt to activate each of the other default EPS bearer contexts, if any, by initiating a stand-alone PDN connectivity procedure with request type set to "handover" in the PDN CONNECTIVITY REQUEST message; and

b)	otherwise, enter substates EMM-REGISTERED.NORMAL-SERVICE and 5GMM-REGISTERED.NO-CELL-AVAILABLE and initiate a tracking area update procedure.

Editor's note:	Handling of EPS bearer context states and PDU session states is FFS.

At intersystem change from N1 mode to S1 mode in EMM-IDLE mode when no PDU session context supporting interworking to EPS is active, the UE shall enter substates EMM-DEREGISTERED.NORMAL-SERVICE and 5GMM-DEREGISTERED.NO-CELL-AVAILABLE, and initiate an attach procedure.

### 4.8.4	Core Network selection

If the UE is capable of both N1 mode and S1 mode, when the UE needs to use one or more functionalities not supported in 5GS but supported in EPS and the UE is in 5GMM-IDLE mode, the UE may disable the N1 mode radio capabilities (see subclause 4.9).

If the UE is capable of both N1 mode and S1 mode and lower layers provide an indication that the current E-UTRA cell is connected to both EPC and 5GCN, the UE shall select a core network type (EPC or 5GCN) based on the PLMN selection procedures as specified in 3GPP TS 23.122 [3] and provide the selected core network type information to the lower layer during the initial registration procedure.

NOTE:	If the PLMN selection information provisioned in the USIM does not contain any prioritization between E-UTRAN and NG-RAN for a PLMN, which core network type to select for that PLMN is up to UE implementation.

Editor's note:	The indication of the current E-UTRA cell is connected to both 5GCN and EPC provided by lower layers need to be defined by RAN WGs.

## 4.9	Disabling and re-enabling of UE's N1 mode radio capability

When the UE supporting both N1 mode and S1 mode is disabling the N1 mode radio capability, it should proceed as follows:

a)	select an E-UTRA cell connected to EPC of the registered PLMN or a PLMN from the list of equivalent PLMNs;

b)	if an E-UTRA cell connected to EPC of the registered PLMN or a PLMN from the list of equivalent PLMNs cannot be found, the UE may select another RAT of the registered PLMN or a PLMN from the list of equivalent PLMNs that the UE supports;

c)	if another RAT of the registered PLMN or a PLMN from the list of equivalent PLMNs cannot be found, or the UE does not have a registered PLMN, then perform PLMN selection as specified in 3GPP TS 23.122 [3]; or

d)	if no other allowed PLMN and RAT combinations are available, then the UE may re-enable the N1 mode capability and remain camped in NG-RAN of the registered PLMN, and may periodically scan for another PLMN and RAT combination which can provide non-5GS services.

When the UE supporting both N1 mode and S1 mode needs to stay in E-UTRA connected to EPC (e.g. due to the domain selection for UE originating sessions as specified in subclause 4.3.2), in order to prevent unwanted handover or cell reselection from E-UTRA connected to EPC to NG-RAN connected to 5GC, the UE operating in single-registration mode shall disable the N1 mode radio capability and:

a)	shall not indicate the support of N1 mode radio capability to the network; and

b)	the UE NAS layer shall indicate the access stratum layer(s) of disabling of the N1 mode radio capability.

NOTE:	The UE can only disable the N1 mode radio capability when in 5GMM-IDLE mode.

Editor's note:	How the UE indicates not supporting the N1 mode radio capability to the network is FFS.

The UE shall re-enable the N1 mode radio capability when:

a)	the UE performs PLMN selection; or

b)	the UE powers off and powers on again or the USIM is removed.

If the disabling of N1 mode radio capability was due to IMS voice is not available and the UE’s usage setting is "voice centric", the UE shall re-enable the N1 mode radio capability when the UE’s usage setting is changed from "voice centric" to "data centric" or when the IMS voice becomes available in 5GS, as specified in subclauses 4.3.3 and 4.3.4.

As an implementation option, the UE may start a timer for re-enabling N1 mode radio capability, after the N1 mode radio capability is disabled. On the expiry of this timer, the UE should enable the N1 mode radio capability.

The UE should memorize the identity of the PLMN where N1 mode radio capability was disabled and should not consider this PLMN in subsequent PLMN selections as specified in 3GPP TS 23.122 [3].

Editor’s note:	How the UE uses the identity of the PLMN where N1 mode radio capability was disabled in subsequent PLMN selections is to be specified in 3GPP TS 23.122 [3].

# 5	Elementary procedures for 5GS mobility management

## 5.1	Overview

### 5.1.1	General

The main function of the 5GS mobility management (5GMM) sublayer is to support the identification, security, mobility of a UE as well as generic message transport.

A further function of the 5GMM sublayer is to provide connection management services to the other sublayer(s).

Editor's note:	Sublayer design can be revisited after the protocol framework is identified.

### 5.1.2	Types of 5GMM procedures

Depending on how they can be initiated, three types of 5GMM procedures can be distinguished:

a)	5GMM common procedures

5GMM common procedure can always be initiated when the UE is in 5GMM-CONNECTED mode. The procedures belonging to this type are:

1)	Initiated by the network:

i)	network-initiated NAS transport.

ii)	primary authentication and key agreement procedure

iii)	security mode control;

iv)	generic UE configuration update

v)	identification

2)	Initiated by the UE:

UE-initiated NAS transport.

3)	Initiated by the UE or the network and used to report certain error conditions detected upon receipt of 5GMM protocol data:

5GMM status.

b)	5GMM specific procedures:

At any time only one UE initiated 5GMM specific procedure can be running for each of the access network(s) that the UE is camping in. The procedures belonging to this type are:

1)	Initiated by the UE and used e.g. to register to the network for 5GS services and establish a 5GMM context, to update the location/parameter(s) of the UE:

registration.

Initiated by the UE or the network and used to deregister from the network for 5GS services and to release a 5GMM context:

de-registration.

c)	5GMM connection management procedures:

1)	Initiated by the UE and used to establish a secure connection to the network or to request the resource reservation for sending data, or both:

service request.

The service request procedure can only be initiated if no UE initiated 5GMM specific procedure is ongoing for each of the access network(s) that the UE is camping in.

2)	Initiated by the network and used to request the establishment of an N1 NAS signalling connection or to prompt the UE to perform re-registration if necessary as a result of a network failure; not applicable for the non-3GPP access network:

paging.

3)	Initiated by the network and used to request re-activation the PDU session(s) associated with non-3GPP access over 3GPP access when  the UE is in 5GMM-CONNECTED mode over either 3GPP access or non 3GPP access:

notification.

NOTE:	The network can page the UE over 3GPP access for the re-activation of the PDU session(s) associated with non-3GPP access over 3GPP access.

### 5.1.3	5GMM sublayer states

#### 5.1.3.1	General

In the following subclauses, the 5GS mobility management (5GMM) sublayer of the UE and the network is described by means of different state machines. The 5GMM sublayer states is managed per access type independently, i.e. 3GPP access or non-3GPP access. In subclause 5.1.3.2, the states of the 5GMM sublayer are introduced.

#### 5.1.3.2	5GMM sublayer states

##### 5.1.3.2.1	5GMM sublayer states in the UE

###### 5.1.3.2.1.1	General

In the following subclauses, the possible 5GMM sublayer states of the UE are described and shown in Figure 5.1.3.2.1.1.1.

NOTE:	Not all possible transitions are shown in this figure.

Editor's note:	The fatal causes and non-fatal causes used in the 5GMM procedures are FFS.

Figure 5.1.3.2.1.1.1: 5GMM main states in the UE

###### 5.1.3.2.1.2	Main states

5.1.3.2.1.2.1	5GMM-NULL

5GS services are disabled in the UE. No 5GS mobility management function shall be performed in this state.

5.1.3.2.1.2.2	5GMM-DEREGISTERED

In the state 5GMM-DEREGISTERED, no 5GMM context has been established and the UE location is unknown to the network and hence it is unreachable by a network. In order to establish a 5GMM context, the UE shall start the initial registration procedure.

5.1.3.2.1.2.3	5GMM-REGISTERED-INITIATED

A UE enters the state 5GMM-REGISTERED-INITIATED after it has started the initial registration procedure or the non-initial registration procedure, excluding the periodic regstriation update over non-3GPP access, and is waiting for a response from the network.

5.1.3.2.1.2.4	5GMM-REGISTERED

In the state 5GMM-REGISTERED, a 5GMM context has been established. Additionally, one or more PDU session context(s) may be activated at the UE. The UE may initiate the non-initial registration procedure (including the normal regstriation update and periodic registration update) and the service request procedure. The UE in the state 5GMM-REGISTERED over non-3GPP access shall not initiate the periodic registration update procedure.

5.1.3.2.1.2.5	5GMM-DEREGISTERED-INITIATED

A UE enters the state 5GMM-DEREGISTERED-INITIATED after it has requested release of the 5GMM context by starting the de-registration procedure and is waiting for a response from the network.

5.1.3.2.1.2.6	5GMM-SERVICE-REQUEST-INITIATED

A UE enters the state 5GMM-SERVICE-REQUEST-INITIATED after it has started the service request procedure and is waiting for a response from the network.

###### 5.1.3.2.1.3	Substates of state 5GMM-DEREGISTERED

5.1.3.2.1.3.1	General

The state 5GMM-DEREGISTERED is subdivided into a number of substates as described in this subclause. The following substates are not applicable to non-3GPP access:

a)	5GMM-DEREGISTERED.LIMITED-SERVICE;

b)	5GMM-DEREGISTERED.PLMN-SEARCH:

c)	5GMM-DEREGISTERED.NO-SUPI;

d)	5GMM-DEREGISTERED.NO-CELL-AVAILABLE; and

e)	5GMM-DEREGISTERED.eCALL-INACTIVE.

Valid subscriber data are available for the UE before it enters the substates, except for the substate 5GMM-DEREGISTERED.NO-SUPI.

5.1.3.2.1.3.2	5GMM-DEREGISTERED.NORMAL-SERVICE

The substate 5GMM-DEREGISTERED.NORMAL-SERVICE is chosen in the UE when a suitable cell has been found and the PLMN or tracking area is not in the forbidden list.

5.1.3.2.1.3.3	5GMM-DEREGISTERED.LIMITED-SERVICE

The substate 5GMM-DEREGISTERED.LIMITED-SERVICE is chosen in the UE, when it is known that a selected cell is unable to provide normal service (e.g. the selected cell is in a forbidden PLMN or is in a forbidden tracking area).

This substate is not applicable to non-3GPP access.

5.1.3.2.1.3.4	5GMM-DEREGISTERED.ATTEMPTING-REGISTRATION

The substate 5GMM-DEREGISTERED.ATTEMPTING-REGISTRATION is chosen in the UE if the initial registration procedure failed due to a missing response from the network.

Editor's note:	Other cases in which this substate is chosen are FFS.

5.1.3.2.1.3.5	5GMM-DEREGISTERED.PLMN-SEARCH

The substate 5GMM-DEREGISTERED.PLMN-SEARCH is chosen in the UE, if the UE is searching for PLMNs. This substate is left either when a cell has been selected (the new substate is NORMAL-SERVICE or LIMITED-SERVICE) or when it has been concluded that no cell is available at the moment (the new substate is NO-CELL-AVAILABLE).

This substate is not applicable to non-3GPP access.

5.1.3.2.1.3.6	5GMM-DEREGISTERED.NO-SUPI

The substate 5GMM-DEREGISTERED.NO-SUPI is chosen in the UE, if the UE has no valid subscriber data available (SIM/USIM not available, the SIM/USIM is considered invalid by the UE) and a cell has been selected.

This substate is not applicable to non-3GPP access.

5.1.3.2.1.3.7	5GMM-DEREGISTERED.NO-CELL-AVAILABLE

No 5G cell can be selected. This substate is entered after a first intensive search failed when in substate 5GMM-DEREGISTERED.PLMN-SEARCH. Cells are searched for at a low rhythm. No 5GS services are offered.

This substate is not applicable to non-3GPP access.

5.1.3.2.1.3.8	5GMM-DEREGISTERED.eCALL-INACTIVE

Editor's note:	How the UE enters 5GMM-DEREGISTERED.eCALL-INACTIVE sutstate is FFS.

Editor's note:	Other substates of state 5GMM-DEREGISTERED are FFS, e.g. 5GMM-DEREGISTERED.REGISTRATION-NEEDED.

This substate is not applicable to non-3GPP access.

###### 5.1.3.2.1.4	Substates of state 5GMM-REGISTERED

5.1.3.2.1.4.1	General

The state 5GMM-REGISTERED is subdivided into a number of substates as described in this subclause. The following substates are not applicable to non-3GPP access:

a)	5GMM-REGISTERED.LIMITED-SERVICE;

b)	5GMM-REGISTERED.PLMN-SEARCH:

c)	5GMM-DEREGISTERED.NON-ALLOWED-SERVICE; and

d)	5GMM-REGISTERED.NO-CELL-AVAILABLE.

5.1.3.2.1.4.2	5GMM-REGISTERED.NORMAL-SERVICE

The substate 5GMM-REGISTERED.NORMAL-SERVICE is chosen by the UE as the primary substate when the UE enters the state 5GMM-REGISTERED and the cell the UE selected is known to be in an allowed area.

5.1.3.2.1.4.3	5GMM-REGISTERED.NON-ALLOWED-SERVICE

The substate 5GMM-REGISTERED.NON-ALLOWED-SERVICE is chosen in the UE, if the cell the UE selected is known to be in a non-allowed area.

This substate is not applicable to non-3GPP access.

5.1.3.2.1.4.4	5GMM-REGISTERED.ATTEMPTING-REGISTRATION-UPDATE

The substate 5GMM-REGISTERED.ATTEMPTING-REGISTRATION-UPDATE is chosen by the UE if the mobility and periodic registration update procedure failed due to a missing response from the network. No 5GMM procedure except:

a)	mobility and periodic registration update procedure over 3GPP access; and

b)	mobility registration procedure over non-3GPP access

shall be initiated by the UE in this substate. No data shall be sent or received.

NOTE:	Mobility registration procedure over non-3GPP access can be trigger by e.g. the change of UE network capability or renegotiating some parameters.

Editor's note:	Other cases in which this substate is chosed are FFS.

5.1.3.2.1.4.5	5GMM-REGISTERED.LIMITED-SERVICE

The substate 5GMM-REGISTERED.LIMITED-SERVICE is chosen in the UE, if the cell the UE selected is known not to be able to provide normal service.

This substate is not applicable to non-3GPP access.

5.1.3.2.1.4.6	5GMM-REGISTERED.PLMN-SEARCH

The substate 5GMM-REGISTERED.PLMN-SEARCH is chosen in the UE, while the UE is searching for PLMNs.

This substate is not applicable to non-3GPP access.

5.1.3.2.1.4.7	5GMM-REGISTERED.NO-CELL-AVAILABLE

5G coverage has been lost or MICO mode is active in the UE. If MICO mode is active, the UE can deactivate MICO mode at any time by activating the AS layer when the UE needs to send mobile originated signalling or user data. Otherwise, the UE shall not initiate any 5GMM procedure except for cell and PLMN reselection.

Editor's note:	Other substates of state 5GMM-REGISTERED are FFS, e.g. 5GMM-REGISTERED.UPDATE-NEEDED.

This substate is not applicable to non-3GPP access.

##### 5.1.3.2.2	5GS update status in the UE

In order to describe the detailed UE behaviour, the 5GS update (5U) status pertaining to a specific subscriber is defined.

The 5GS update status is stored in a non-volatile memory in the USIM if the corresponding file is present in the USIM, else in the non-volatile memory in the ME.

The 5GS update status value is changed only after the execution of a registration, network-initiated de-registration, 5GS based primary authentication and key agreement, service request or paging procedure.

5U1: UPDATED

The last registration attempt was successful.

5U2: NOT UPDATED

The last registration attempt failed procedurally, e.g. no response or reject message was received from the AMF.

5U3: ROAMING NOT ALLOWED

The last registration, service request, or registration for mobility or periodic registration update attempt was correctly performed, but the answer from the AMF was negative (because of roaming or subscription restrictions).

##### 5.1.3.2.3	5GMM sublayer states in the network side

###### 5.1.3.2.3.1	General

In the following subclauses, the possible 5GMM sublayer states of the network are described and shown in Figure 5.1.3.2.3.1.1.

NOTE:	Not all possible transitions are shown in this figure.

Editor's note:	The fatal causes and non-fatal causes used in the 5GMM procedures are FFS.

Figure 5.1.3.2.3.1.1: 5GMM main states in the network

###### 5.1.3.2.3.2	5GMM-DEREGISTERED

In the state 5GMM-DEREGISTERED, no 5GMM context has been established or the 5GMM context is marked as deregistered. The UE is deregistered. The network may answer to an initial registration procedure initiated by the UE. The network may also answer to a de-registration procedure initiated by the UE.

###### 5.1.3.2.3.3	5GMM-COMMON-PROCEDURE-INITIATED

The network enters the state 5GMM-COMMON-PROCEDURE-INITIATED, after it has started a common 5GMM procedure and is waiting for a response from the UE.

###### 5.1.3.2.3.4	5GMM-REGISTERED

In the state 5GMM-REGISTERED, a 5GMM context has been established. Additionally, one or more PDU session context(s) may be activated at the network.

###### 5.1.3.2.3.5	5GMM-DEREGISTERED-INITIATED

The network enters the state 5GMM-DEREGISTERED-INITIATED after it has started a de-registration procedure and is waiting for a response from the UE.

### 5.1.4	Coordination between 5GMM and EMM

#### 5.1.4.1	General

If both 5GMM and EMM are enabled, a UE, which is capable of N1 mode and S1 mode and operates in single-registration mode, shall maintain one common registration for 5GMM and EMM.

#### 5.1.4.2	Coordination between 5GMM and EMM with N26 interface

A UE that is not registered shall be in state EMM-DEREGISTERED and state 5GMM-DEREGISTERED.

In N1 mode, upon successful completion of a registration procedure, the UE operating in single-registration mode shall enter substates 5GMM-REGISTERED.NORMAL-SERVICE and EMM-REGISTERED.NO-CELL-AVAILABLE.

At intersystem change from S1 mode to N1 mode, the UE shall enter substates 5GMM-REGISTERED.NORMAL-SERVICE and EMM-REGISTERED.NO-CELL-AVAILABLE and initiate a mobility registration update procedure.

In S1 mode, upon successful completion of an attach or tracking area updating procedure, the UE operating in single-registration mode shall enter substates 5GMM-REGISTERED.NO-CELL-AVAILABLE and EMM-REGISTERED.NORMAL-SERVICE.

At intersystem change from N1 mode to S1 mode when there is no active PDU session context for which interworking to EPS is supported as specified in subclause 6.1.4, and EMM-REGISTERED without PDN connection is not supported by the UE or the MME, the UE shall enter state 5GMM-DEREGISTERED and state EMM-DEREGISTERED and then initiate the attach procedure. If EMM-REGISTERED without PDN connection is supported by the UE and the MME, the UE shall enter substates EMM-REGISTERED.NORMAL-SERVICE and 5GMM-REGISTERED.NO-CELL-AVAILABLE and initiate a tracking area updating procedure.

At intersystem change from N1 mode to S1 mode when there is at least one active PDU session context for which interworking to EPS is supported as specified in subclause 6.1.4, the UE shall enter substates EMM-REGISTERED.NORMAL-SERVICE and 5GMM-REGISTERED.NO-CELL-AVAILABLE and initiate a tracking area updating procedure.

#### 5.1.4.3	Coordination between 5GMM and EMM without N26 interface

A UE operating in the single-registration mode that is not registered shall be in state EMM-DEREGISTERED and in state 5GMM-DEREGISTERED.

In N1 mode, upon successful completion of a registration procedure, the UE operating in the single-registration mode shall enter substates 5GMM-REGISTERED.NORMAL-SERVICE and EMM-REGISTERED.NO-CELL-AVAILABLE.

In S1 mode, upon successful completion of an attach or tracking area updating procedure, the UE operating in the single-registration mode shall enter substates 5GMM-REGISTERED.NO-CELL-AVAILABLE and EMM-REGISTERED.NORMAL-SERVICE.

At intersystem change from S1 mode to N1 mode in 5GMM-IDLE mode, the UE operating in the single-registration mode shall enter substates EMM-REGISTERED.NO-CELL-AVAILABLE and 5GMM- REGISTERED.NORMAL-SERVICE and then initiate the mobility registration update procedure

### 5.1.5	Dual-registration mode

If both 5GMM and EMM are enabled, a UE, which is capable of N1 mode and S1 mode in the dual-registration mode, shall maintain independent registrations for 5GMM and EMM independently and coordination between 5GMM and EMM is not needed. A UE operating in the dual-registration mode may register to N1 mode only, S1 mode only, or to both N1 mode and S1 mode.

When no PDU session context is active and the UE has not registered to S1 mode yet, the UE may initiate the attach procedure if EMM-REGISTERED without PDN connection is not supported by the MME. If EMM-REGISTERED without PDN connection is supported by the MME, the UE may initiate either the attach procedure without PDN connection establishment or the attach procedure with PDN connection establishment.

When at least one PDU session context is active and the UE has not registered to S1 mode yet, the UE may initiate the attach procedure. If necessary, the UE may transfer an active PDU session from N1 mode to S1 mode by initiating the attach procedure with request type set to "handover" in the PDN CONNECTIVITY REQUEST message. After succesfully attached in S1 mode, if necessary, the UE may transfer other active PDU sessions from N1 mode to S1 mode by initiating the PDN connectivity procedure with request type set to "handover" in the PDN CONNECTIVITY REQUEST message.

NOTE 1:	It is up to UE implementation to determine which active PDU session is transferred from N1 mode to S1 mode.

When the UE has not registered to N1 mode, the UE may initiate the initial registration procedure. After succesfully registered in N1 mode, if necessary, the UE may transfer one or more active PDN connections from S1 mode to N1 mode by initiating the PDU session establishment procedure with request type set to "existing PDU session".

NOTE 2:	It is up to UE implementation to determine which active PDN connection is transferred from S1 mode to N1 mode.

If both the UE and the MME support EMM-REGISTERED without PDN connection, the UE that transferred all PDN connections to the 5GS, may stay in state EMM-REGISTERED. Otherwise, the UE shall enter state EMM-DEREGISTERED upon transferring all PDN connection to the 5GS.

## 5.2	Behaviour of the UE in state 5GMM-DEREGISTERED and state 5GMM-REGISTERED

### 5.2.1	General

In this subclause, the detailed behaviour of the UE in the states 5GMM-DEREGISTERED and 5GMM-REGISTERED is described.

### 5.2.2	UE behaviour in state 5GMM-DEREGISTERED

#### 5.2.2.1	General

The state 5GMM-DEREGISTERED is entered in the UE, when:

a)	the de-registration is performed either by the UE or by the network (see subclause 5.5.2);

b)	the registration request is rejected by the AMF (see subclause 5.5.1.2.5 and 5.5.1.3.5);

c)	the service request request is rejected by the MME (see subclause 5.6.1); or

d)	the UE is switched on.

In state 5GMM-DEREGISTERED, the UE shall behave according to the substate as explained in subclause 5.2.2.1.2.

Editor's note:	Other cases in which the UE enters the state 5GMM-DEREGISTERED are FFS.

#### 5.2.2.2	Detailed description of UE behaviour in state 5GMM-DEREGISTERED

##### 5.2.2.2.1	NORMAL-SERVICE

The UE shall initiate an initial registration procedure.

##### 5.2.2.2.2	LIMITED-SERVICE

The UE shall initiate an initial registration procedure when entering a cell which provides normal service.

The UE may initiate initial registration for emergency services.

##### 5.2.5.2.3	ATTEMPTING-REGISTRATION

The UE in 3GPP access:

a)	shall initiate an initial registration procedure on the expiry of timers T3502 or T3511 or T3346;

b)	may initiate an initial registration procedure for emergency services even if timer T3346 is running;

c)	shall initiate an initial registration procedure when entering a new PLMN, if timer T3346 is running and the new PLMN is not equivalent to the PLMN where the UE started timer T3346, the PLMN identity of the new cell is not in the forbidden PLMN lists and the tracking area is not in one of the lists of forbidden tracking areas; and

d)	shall initiate an initial registration procedure when the tracking area of the serving cell has changed, if timer T3346 is not running, the PLMN identity of the new cell is not in one of the forbidden PLMN lists and the tracking area of the new cell is not in one of the lists of forbidden tracking areas.

The UE in non-3GPP access:

-	shall initiate an initial registration procedure on the expiry of timers T3502 or T3511.

##### 5.2.5.2.4	PLMN-SEARCH

The UE shall perform PLMN selection. If a new PLMN is selected, the UE shall reset the registration attempt counter and initiate the initial registration procedure (see subclause 5.5.1.2.2).

If the selected cell is known not to be able to provide normal service, the UE may initiate registration for emergency services.

##### 5.2.5.2.5	NO-SUPI

The UE shall perform cell selection.

The UE may initiate registration for emergency services.

##### 5.2.5.2.6	NO-CELL-AVAILABLE

The UE shall perform cell selection and choose an appropriate substate when a cell is found.

##### 5.2.5.2.7	eCALL-INACTIVE

Editor's note:	The UE behaviour in this substate is FFS.

Editor's note:	The other required UE behaviour in state 5GMM-DEREGISTERED is FFS, e.g. selection of the substate after power on, substate when back to state 5GMM-DEREGISTERED from another 5GMM state.

### 5.2.3	UE behaviour in state 5GMM-REGISTERED

#### 5.2.3.1	General

The state 5GMM-REGISTERED is entered at the UE, when:

-	the initial registration procedure is performed by the UE (see subclause 5.5.1.2.2).

In state 5GMM-REGISTERED, the UE shall behave according to the substate as explained in subclause 5.2.3.2.

#### 5.2.3.2	Detailed description of UE behaviour in state 5GMM-REGISTERED

##### 5.2.3.2.1	NORMAL-SERVICE

The UE:

a)	shall initiate the mobility or the periodic registration update procedure (according to conditions given in subclause 5.5.1.3.2) , except that the periodic registration update procedure shall not be initiated over non-3GPP access;

b)	shall initiate the service request procedure (according to conditions given in subclause 5.6.1); and

c)	shall respond to paging.

NOTE:	Paging is not supported over non-3GPP access.

##### 5.2.3.2.2	NON-ALLOWED-SERVICE

The UE shall behave as specified in subclause 5.2.x.

##### 5.2.3.2.3	ATTEMPTING-REGISTRATION-UPDATE

The UE in 3GPP access:

a)	shall not send any user data;

b)	shall initiate mobility registration update procedure on the expiry of timers T3502 or T3511 or T3346;

c)	shall initiate mobility registration update procedure when entering a new PLMN, if timer T3346 is running and the new PLMN is not equivalent to the PLMN where the UE started timer T3346, the PLMN identity of the new cell is not in the forbidden PLMN lists, and the tracking area is not in one of the lists of forbidden tracking areas;

d)	shall initiate mobility registration update procedure when the tracking area of the serving cell has changed, if timer T3346 is not running, the PLMN identity of the new cell is not in one of the forbidden PLMN lists and the tracking area is not in one of the lists of forbidden tracking areas;

e)	may initiate a mobility registration update procedure upon request of the upper layers to establish a PDU session for emergency services; and

f)	may perform de-registration locally and initiate an initial registration procedure for emergency services even if timer T3346 is running.

The UE in non-3GPP access:

a)	shall not send any user data; and

b)	shall initiate mobility registration update procedure on the expiry of timers T3502 or T3511.

##### 5.2.3.2.4	LIMITED-SERVICE

The UE:

a)	shall perform cell selection/reselection; and

b)	may initiate registration for emergency services.

##### 5.2.3.2.5	PLMN-SEARCH

The UE shall perform PLMN selection. If a new PLMN is selected, the UE shall reset the registration attempt counter and initiate the initial registration procedure (see subclause 5.5.1.2).

If the selected cell is known not to be able to provide normal service, the UE may initiate registration for emergency services.

##### 5.2.3.2.6	NO-CELL-AVAILABLE

The UE shall perform cell selection and choose an appropriate substate when a cell is found.

## 5.3	General on elementary 5GMM procedures

### 5.3.1	5GMM modes and N1 NAS signalling connection

#### 5.3.1.1	Establishment of the N1 NAS signalling connection

When the UE is in 5GMM-IDLE mode over 3GPP access and needs to transmit an initial NAS message, the UE shall request the lower layer to establish an RRC connection. Upon indication from the lower layers that the RRC connection has been established, the UE shall consider that the N1 NAS signalling connection over 3GPP access is established and enter 5GMM-CONNECTED mode over 3GPP access.

When the UE is in 5GMM-IDLE mode over non-3GPP access, and the UE receives an indication from the lower layers of access stratum connection establishment, the UE shall consider the N1 NAS signalling connection established enter 5GMM-CONNECTED mode over non-3GPP access and send an initial NAS message.

Initial NAS messages are:

a)	REGISTRATION REQUEST message;

b)	DEREGISTRATION REQUEST message; and

c)	SERVICE REQUEST message.

If the UE is capable of both N1 mode and S1 mode and lower layers provide an indication that the current E-UTRA cell is connected to both EPC and 5GCN, for the routing of the REGISTRATION REQUEST message during the initial registration procedure to the appropriate core network (EPC or 5GCN), the UE NAS provides the lower layers with the selected core network type information.

For the routing of the initial NAS message to the appropriate AMF, if the UE holds a valid 5G-GUTI, the UE NAS provides the lower layers with either the 5G-S-TMSI or the registered GUAMI that consists of the PLMN ID, the AMF region ID, the AMF set ID and the AMF pointer according to the following rules:

a)	When the tracking area of the current cell is in the registration area, the UE NAS shall provide the lower layers with the 5G-S-TMSI, but shall not provide the registered GUAMI to the lower layers;

b)	When the tracking area of the current cell is not in the registration area, the UE NAS shall provide the lower layers with the GUAMI (AMF identifier part of the valid 5G-GUTI);

c)	When the REGISTRATION REQUEST message with the registration type set to "initial registration" is sent over 3GPP access in the same PLMN or equivalent PLMN to establish the NAS signalling connection, the UE NAS shall provide the lower layers with the GUAMI (AMF identifier part of the valid 5G-GUTI which is assigned over the non-3GPP access); or

d)	When the REGISTRATION REQUEST message with the registration type set to "initial registration" is sent over non-3GPP access in the same PLMN or equivalent PLMN to establish the NAS signalling connection, the UE NAS shall provide the lower layers with the GUAMI (AMF identifier part of the valid 5G-GUTI which is assigned over the 3GPP access).

#### 5.3.1.2	Release of the N1 NAS signalling connection

The signalling procedure for the release of the N1 NAS signalling connection is initiated by the network.

In N1 mode, upon indication from lower layers that the access stratum connection has been released, the UE shall enter 5GMM-IDLE mode and consider the N1 NAS signalling connection released.

To allow the network to release the N1 NAS signalling connection, the UE:

a)	shall start the timer T3540 if the UE receives any of the 5GMM cause values #11, #12, #13 (not applicable to the service request procedure);

b)	shall start the timer T3540 if:

1)	the UE receives a REGISTRATION ACCEPT message;

2)	the UE has not set the "follow-on request pending" indication in the REGISTRATION REQUEST message;

3)	the registration for mobility and periodic registration update procedure has been initiated in 5GMM-IDLE mode; and

4)	the user-plane radio resources for PDU sessions have not been set up;

c)	shall start the timer T3540 if the UE receives a REGISTRATION REJECT message indicating:

-	the 5GMM cause value #10.

d)	shall start the timer T3540 if the UE receives a SERVICE REJECT message indicating:

-	the 5GMM cause value #10.

e)	shall start the timer T3540 if:

1)	the UE receives a CONFIGURATION UPDATE COMMAND message indicating registration requested with new allowed NSSAI information included; and

2)	the user-plane resources for PDU sessions have not been set up.

Upon expiry of T3540,

-	in cases a, and b, the UE shall locally release the established NAS signalling connection; or

-	in cases c, and d the UE shall locally release the established N1 NAS signalling connection and the UE shall initiate the registration procedure as described in subclause 5.5.1.2.2 or 5.5.1.3.2.

-	in case e, the UE shall locally release the established N1 NAS signalling connection and perform a new registration procedure as specified in subclause 5.5.1.3.2 using SUPI.

In case b,

-	upon an indication from the lower layers that the user-plane radio resources for PDU sessions are set up, the UE shall stop timer T3540 and may send uplink signalling via the existing N1 NAS signalling connection or user data via user plane. If the uplink signalling is associated with emergency services fallback or establishing a PDU session for emergency services, the UE shall stop timer T3540 and send the uplink signalling via the existing NAS signalling connection; or

-	upon receipt of a DEREGISTRATION REQUEST message, the UE shall stop timer T3540 and respond to the network-initiated de-registration request as specified in subclause 5.5.2.3.

In case c,

-	upon an indication from the lower layers that the access stratum connection has been released, the UE shall stop timer T3540 and perform a new registration procedure as specified in subclause 5.5.1.2.2 or 5.5.1.3.2.

-	upon receiving a request from upper layers to send NAS signalling associated with emergency services fallback or establishing a PDU session for emergency services, the UE shall stop timer T3540 and shall locally release the NAS signalling connection, before proceeding as specified in subclause 5.5.1.

In cases d,

-	upon an indication from the lower layers that the RRC connection has been released, the UE shall stop timer T3540 and perform a new registration procedure as specified in subclause 5.5.1.2.2 or 5.5.1.3.2.

-	upon receiving a request from upper layers to send NAS signalling associated with emergency services fallback or establishing a PDU session for emergency services, the UE shall stop timer T3540 and shall locally release the NAS signalling connection, before proceeding as specified in subclause 5.5.1.

In case e,

- upon an indication from the lower layers that the RRC connection has been released, the UE shall stop timer T3540 and perform a new registration procedure as specified in subclause 5.5.1.3.2 using SUPI.

#### 5.3.1.3	5GMM-CONNECTED mode with RRC inactive indication

This subclause is only applicable for UE's 5GMM mode over 3GPP access.

The UE is in 5GMM-CONNECTED mode with RRC inactive indication when the UE is in:

a)	5GMM-CONNECTED mode over 3GPP access at the NAS layer; and

b)	RRC_INACTIVE state at the AS layer (see 3GPP TS 38.300 [14]).

Unless stated otherwise, the UE behavior in 5GMM-CONNECTED mode with RRC inactive indication follows the UE behavior in 5GMM-CONNECTED over 3GPP access, except that:

a)	the UE shall apply the mobility restrictions; and

b)	the UE shall perform the PLMN selection procedures

as in 5GMM-IDLE mode over 3GPP access.

The UE shall transition from 5GMM-CONNECTED mode over 3GPP access to 5GMM-CONNECTED mode with RRC inactive indication upon indication from the lower layers that the UE has transitioned to RRC_INACTIVE state.

The UE shall trigger a transition from 5GMM-CONNECTED mode with RRC inactive indication to 5GMM-CONNECTED mode over 3GPP access upon:

a)	uplink data transfer; or

b)	mobile originaiting signaling.

The UE shall transition from 5GMM-CONNECTED mode with RRC inactive indication to 5GMM-CONNECTED mode over 3GPP access upon receving from the lower layers indication that the UE has transitioned to RRC_CONNECTED state (see 3GPP TS 38.300 [14]).

NOTE:	The AMF can be aware of the transition between 5GMM-CONNECTED mode and 5GMM-CONNECTED mode with RRC inactive indication for a UE (see 3GPP TS 23.502 [5]).

The UE shall trigger a transition from 5GMM-CONNECTED mode with RRC inactive indication to 5GMM-IDLE mode upon selection of a PLMN that is not an equivalent PLMN to the registered PLMN.

The UE shall transition from 5GMM-CONNECTED mode with RRC inactive indication to 5GMM-IDLE mode over 3GPP access upon receving from the lower layers:

a)	indication that the transition from RRC_INACTIVE state to RRC_CONNECTED state has failed;

b)	indication of transition from RRC_INACTIVE to RRC_IDLE;

c)	AMF paging notification; or

d)	indication of  cell selection to E-UTRAN.

The UE shall not trigger a transition from 5GMM-CONNECTED mode with RRC inactive indication to 5GMM-IDLE mode upon entering a new PLMN which is in the list of equivalent PLMNs.

### 5.3.2	Permanent identifiers

A globally unique permanent identity, the 5G subscription permanent identifier (SUPI), is allocated to each subscriber for 5GS-based services. In the current release, the IMSI and the network access identifier (NAI) are valid SUPI types. Though non-IMSI based SUPIs are possible by using NAI, the IMSI can be contained within the NAI for the SUPI. The structure of the SUPI and its derivatives will be specified in 3GPP TS 23.003 [2].

A UE supporting NG-RAN includes a SUCI when a valid 5G-GUTI is not available from the PLMN or equivalent PLMN to which the UE is attempting to register or if requested by the network, during the registration procedure. The SUCI is a privacy preserving identifier containing the concealed SUPI.

NOTE:	If the home network has not provisioned the public key needed to generate a SUCI, the UE uses "null-scheme" as specified in 3GPP TS 33.501 [12] to generate a SUCI which equals to the SUPI.

Each UE supporting NG-RAN contains a permanent equipment identifier (PEI) for accessing 5GS-based services. In the current release, the IMEI and the IMEISV are the only PEI format supported by 5GS.The structure of the PEI and its formats will be specified in 3GPP TS 23.003 [2].

A UE supporting NG-RAN includes a PEI:

a)	when neither SUPI nor valid 5G-GUTI is available to use for emergency services in the REGISTRATION REQUEST message with 5GS registration type IE set to "emergency registration"; and

b)	when the network requests the PEI by using the identification procedure, in the IDENTIFICATION RESPONSE message.

The AMF can request the PEI at any time by using the identification procedure.

### 5.3.3	Temporary identities

A globally unique temporary user identity for 5GS-based services, the 5G globally unique temporary identity (5G-GUTI), is used for identification within the signalling procedures. The 5G-GUTI is common to both 3GPP and non-3GPP access. In the paging and service request procedures, a shortened form of the 5G-GUTI, the 5G S-temporary mobile subscriber identity (5G-S-TMSI), is used to enable more efficient radio signalling. The purpose of the 5G-GUTI and 5G-S-TMSI is to provide identity confidentiality, i.e., to protect a user from being identified and located by an intruder. The structure of the 5G-GUTI and its derivatives is specified in 3GPP TS 23.003 [2]. The 5G-GUTI has two main components:

a)	the globally unique AMF id (GUAMI) that uniquely identifies the AMF that allocated the 5G-GUTI, and

b)	the 5G-TMSI that provides for an unambiguous identity of the UE within this AMF.

The 5G-S-TMSI has three main components:

a)	the AMF set ID that uniquely identifies the AMF set within the AMF region;

b)	the AMF pointer that uniquely identifies the AMF within the AMF set; and

c)	the 5G-TMSI.

A UE supporting N1 mode includes a valid 5G-GUTI, if any is available, in the  REGISTRATION REQUEST and DEREGISTRATION REQUEST messages. In the SERVICE REQUEST message, the UE includes a valid 5G-S-TMSI as user identity. The AMF may assign a new 5G-GUTI for a particular UE at successful registration and generic UE configuration update procedures.

If a new 5G-GUTI is assigned by the AMF, the UE and the AMF handle the 5G-GUTI as follows:

a)	Upon receipt of a 5GMM message containing a new 5G-GUTI the UE considers the new 5G-GUTI as valid and the old 5G-GUTI as invalid.

b)	The AMF considers the old 5G-GUTI as invalid as soon as an acknowledgement for a registration or generic UE configuration update procedure is received.

### 5.3.4	Registration areas

Within the 5GS, the registration area is managed independently per access type, i.e., 3GPP access or non-3GPP access. The AMF assigns a registration area to the UE during the registration procedure. A registration area is defined as a set of tracking areas and each of these tracking areas consists of one or more cells that cover a geographical area. Tracking areas cannot overlap each other. Within the 5GS, the concept of "registration to multiple tracking areas" applies:

a)	A tracking area is identified by a TAI which is broadcast in the cells of the tracking area. The TAI is constructed from a TAC and a PLMN identifier. In case of a shared network, one or more TAC and multiple PLMN identifiers are broadcast.

b)	In order to reduce the tracking area update signalling within the 5GS, the AMF can assign several tracking areas to the UE. These tracking areas construct a list of tracking areas which is identified by a TAI list. When generating the TAI list, the AMF shall include only TAIs that are applicable on the access where the TAI list is sent. The AMF shall be able to allocate a TAI List over different NG-RAN access technologies.

c)	The UE considers itself registered to a list of tracking areas and does not need to trigger mobility registration update procedure (i.e. registration procedure with registration type set to "mobility registration update") as long as the UE stays in one of the tracking areas of the list of tracking areas received from the AMF.

Editor's note:	The maximum number of tracking areas which can be allocated per UE needs to be defined which can be as in EPS or different.

d)	The UE will consider the TAI list as valid, until it receives a new TAI list in the next mobility registration update, periodic registration update, or generic UE configuration update procedure, or the UE is commanded by the network to delete the TAI list by a reject message or it is deregistered from the 5GS. If the registration request is accepted or the TAI list is reallocated by the AMF, the AMF shall provide at least one entry in the TAI list. If the new and the old TAI list are identical, the AMF does not need to provide the new TAI list to the UE during mobility registration update or periodic registration update.

e)	The TAI list can be reallocated by the AMF.

f)-	When the UE is deregistered from the 5GS, the TAI list in the UE is invalid.

g)	The AMF allocates one 5G-GUTI, which is common between 3GPP and non3GPP, to the UE.

h)	The UE includes the last visited registered TAI, if available, to the AMF.

### 5.3.5	Service area restrictions

The service area restrictions consist of either an allowed area, or a non-allowed area. The allowed area can be limited by a maximum allowed number of tracking areas or include all tracking areas in a PLMN. The non-allowed area can contain up to 16 tracking areas. The network conveys the service area restrictions to the UE by including either an allowed area, or a non-allowed area, but not both, in the Service area list IE of a REGISTRATION ACCEPT message or a CONFIGURATION UPDATE COMMAND message,

When the UE receives a Service area list IE with an allowed area indication during a registration procedure or a generic UE configuration update procedure:

a)	if the "Type of list" included in the Service area list IE does not indicate all TAIs belonging to the PLMN are allowed area, the UE shall delete the old list of "allowed tracking areas" and store the tracking areas in the allowed area as the list of "allowed tracking areas". If the UE has a stored list of "non-allowed tracking areas" for the registered PLMN, the UE shall delete that list; or

b)	if the "Type of list" included in the Service area list IE indicates all TAIs belonging to the PLMN are allowed area, the UE shall treat all tracking areas in the registered PLMN are allowed area and delete the stored list of "allowed tracking areas" or the stored list of "non-allowed tracking areas" for the registered PLMN.

When the UE receives a Service area list IE with a non-allowed area indication during a registration procedure or a generic UE configuration update procedure, the UE shall delete the old list of "non-allowed tracking areas" and store the tracking areas in the non-allowed area as the list of "non-allowed tracking areas". If the UE has a stored list of "allowed tracking areas" for the registered PLMN, the UE shall delete that list.

If the UE has a stored list of "allowed tracking areas":

a)	while camped on a cell whose TAI is in the list of "allowed tracking areas", the UE is allowed to initiate any 5GMM and 5GSM procedures; and

b)	while camped on a cell whose TAI is not in the list of "allowed tracking areas":

1)	if the UE is in 5GMM-IDLE mode over 3GPP access, the UE:

i)	shall not perform the mobility and periodic registration update procedure with Uplink data status IE except for emergency services; and

ii)	shall not initiate a service request procedure except for emergency services or for responding to core network paging; and

2)	if the UE is in 5GMM-CONNECTED mode or 5GMM-CONNECTED mode with RRC inactive indication over 3GPP access, the UE:

i)	shall not perform the mobility registration update procedure with Uplink data status IE except for emergency services;

ii)	shall not initiate a service request procedure except for emergency services; and

iii)	shall not initiate a 5GSM procedure except for emergency services.

If the UE has a stored list of "non-allowed tracking areas":

a)	while camped on a cell whose TAI is not in the list of "non-allowed tracking areas", the UE is allowed to initiate any 5GMM and 5GSM procedures; and

b)	while camped on a cell whose TAI is in the list of "non-allowed tracking areas":

1)	if the UE is in 5GMM-IDLE mode over 3GPP access, the UE:

i)	shall not perform the mobility and periodic registration update procedure with Uplink data status IE except for emergency services; and

ii)	shall not initiate a service request procedure except for emergency services or for responding to core network paging; and

2)	if the UE is in 5GMM-CONNECTED mode or 5GMM-CONNECTED mode with RRC inactive indication over 3GPP access, the UE:

i)	shall not perform the mobility registration update procedure with Uplink data status IE except for emergency services;

ii)	shall not initiate a service request procedure except for emergency services; and

iii)	shall not initiate a 5GSM procedure except for emergency services.

The list of "allowed tracking areas", as well as the list of "non-allowed tracking areas" shall be erased when:

a)	the UE is switched off;

b)	the UICC containing the USIM is removed; and

c)	periodically (with a period in the range 12 to 24 hours).

When the list of "allowed tracking areas" or the list of "non-allowed tracking areas" is erased, the UE performs cell selection procedure.

The UE shall evaluate the information in the list of "5GS forbidden tracking areas for roaming" and in the list of "5GS forbidden tracking areas for regional provision of service" before evaluating the information in the list of "allowed tracking areas" or the list of "non-allowed tracking areas".

### 5.3.6	Mobile initiated connection only mode

The UE can request the use of mobile initiated connection only (MICO) mode during the registration procedure (see 3GPP TS 23.501 [4] and 3GPP TS 23.502[5]). The UE shall not request the use of MICO mode during:

-	an initial registration for emergency services procedure; or

-	a mobility and periodic registration update procedure when the UE has a PDU session for emergency services established.

If the UE requests the use of MICO mode, the network can accept the use of MICO mode by providing a MICO indication when accepting the registration procedure. The UE may use MICO mode only if the network has provided the MICO indication IE during the last registration procedure.

If the network accepts the use of MICO mode, the AMF may include an "all PLMN registration area allocated" indication in the MICO indication IE to the UE.

If the network accepts the use of MICO mode, the UE may deactivate the AS layer and activate MICO mode by entering the state 5GMM-REGISTERED.NO-CELL-AVAILABLE if:

a)	the UE is in 5GMM-IDLE mode; and

b)	in the 5GMM-REGISTERED.NORMAL-SERVICE state.

When MICO mode is activated all NAS timers are stopped and associated procedures aborted except for periodic registration update timer, any backoff timers, and the timer T controlling the periodic search for HPLMN or EHPLMN or higher prioritized PLMNs (see 3GPP TS 23.122 [3]).

The UE may deactivate MICO mode and activate the AS layer at any time. Upon deactivating MICO mode, the UE may initiate 5GMM procedures (e.g. for the transfer of mobile originated signalling or user data).

When a PDU session for emergency services is successfully established after the MICO mode was activated, the UE shall request the deactivation of MICO mode to the network by initiating a mobility registration update procedure without including the MICO indication IE in the REGISTRATION REQUEST message.

If the AMF accepts the use of MICO mode, the AMF starts the implicit de-registration timer when entering 5GMM-IDLE mode.

### 5.3.7	Handling of the periodic registration update timer and mobile reachable timer

The periodic registration update procedure is used to periodically notify the availability of the UE to the network. The procedure is controlled in the UE by the periodic registration update timer, T3512.

The AMF maintains an implicit de-registration timer to control when the UE is considered implicitly de-registered.

When the N1 NAS signalling connection is released, the AMF shall start a non-3GPP implicit de-registration timer. When the timer expires, the AMF shall perform de-registration procedure for non-3GPP access and enter the state 5GMM-DEREGISTERED over non-3GPP access.

The non-3GPP implicit de-registration timer shall be longer than the non-3GPP de-registration timer.

Editor’s note:	The non-3GPP de-registration timer and the non-3GPP implicit de-registration timer are FFS.

The value of timer T3512 is sent by the network to the UE in the REGISTRATION ACCEPT message. The UE shall apply this value in all tracking areas of the list of tracking areas assigned to the UE until a new value is received. The periodic registration update timer only applies to the UE registered to the 5GS services over 3GPP access.

If timer T3512 received by the UE in a REGISTRATION ACCEPT message contains an indication that the timer is deactivated or the timer value is zero, then timer T3512 is deactivated and the UE shall not perform the periodic registration update procedure.

Timer T3512 is reset and started with its initial value, when the UE changes from 5GMM-CONNECTED to 5GMM-IDLE mode. Timer T3512 is stopped when the UE enters 5GMM-CONNECTED mode or the 5GMM-DEREGISTERED state.

If the UE is registered for emergency services, and timer T3512 expires, the UE shall not initiate a periodic registration update procedure, but shall locally de-register from the network. When the UE is camping on a suitable cell, it may re-register to regain normal service.

When a UE is not registered for emergency services, and timer T3512 expires, the periodic registration update procedure shall be started.

If the UE is not registered for emergency services, and is in a state other than 5GMM-REGISTERED.NORMAL-SERVICE when timer T3512 expires, the periodic registration update procedure is delayed until the UE returns to 5GMM-REGISTERED.NORMAL-SERVICE.

The network supervises the periodic registration update procedure of the UE by means of the mobile reachable timer.

If the UE is not registered for emergency services, the mobile reachable timer shall be longer than T3512. In this case, by default, the mobile reachable timer is 4 minutes greater than timer T3512.

The network behaviour upon expiry of the mobile reachable timer is network dependent, but typically the network stops sending paging messages to the UE on the first expiry, and may take other appropriate actions.

If the UE is registered for emergency services, the AMF shall set the mobile reachable timer with a value equal to timer T3512. When the mobile reachable timer expires, the AMF shall locally de-register the UE.

The mobile reachable timer shall be reset and started with the value as indicated above, when the AMF releases the NAS signalling connection for the UE. The mobile reachable timer shall be stopped when a NAS signalling connection is established for the UE.

Upon expiry of the mobile reachable timer the network shall start the implicit de-registration timer. The value of the implicit de-registration timer is network dependent. If MICO is activated, the default value of the implicit de-registration timer is 4 minutes greater than timer T3512.

If the implicit de-registration timer expires before the UE contacts the network, the network shall implicitly detached-register the UE. The implicit de-registration timer shall be stopped when a NAS signalling connection is established for the UE.

If the AMF provides T3346 value IE in the mobility management messages and T3346 value is greater than timer T3512, the AMF sets the mobile reachable timer and the implicit de-registration timer such that the sum of the timer values is greater than timer T3346 value.

### 5.3.8	Handling of NAS level mobility management congestion control

The AMF may detect 5GMM signalling congestion and perform general NAS level congestion control. Under the 5GMM signalling congestion conditions the AMF may reject 5GMM signalling requests from UEs as specified in 3GPP TS 23.501 [4]. The AMF should not reject the following request:

-	requests for emergency services; and

-	requests from UEs configured to use AC11 – 15 in selected PLMN.

When general NAS level congestion control is active, the AMF may include a value for the mobility management back-off timer T3346 in the reject messages. The UE starts the timer T3346 with the value received in the 5GMM reject messages. To avoid that large numbers of UEs simultaneously initiate deferred requests, the AMF should select the value for the timer T3346 for the rejected UEs so that timeouts are not synchronised.

If the timer T3346 is running when the UE enters state 5GMM-DEREGISTERED, the UE remains switched on, and the USIM in the UE remains the same, then the timer T3346 is kept running until it expires or it is stopped.

If the UE is switched off when the timer T3346 is running, the UE shall behave as follows when the UE is switched on and the USIM in the UE remains the same:

-	let t1 be the time remaining for T3346 timeout at switch off and let t be the time elapsed between switch off and switch on. If t1 is greater than t, then the timer shall be restarted with the value t1 – t. If t1 is equal to or less than t, then the timer need not be restarted. If the UE is not capable of determining t, then the UE shall restart the timer with the value t1.

Editor's note:	Whether dual priority is supported in 5GS and how it is handled in the NAS level 5GMM congestion control are FFS.

If the UE enters a new PLMN while timer T3346 is running, and the new PLMN is not equivalent to the PLMN where the UE started timer T3346, the UE shall stop timer T3346 when initiating 5GMM procedures in the new PLMN.

## 5.4	5GMM common procedures

### 5.4.1	Primary authentication and key agreement procedure

#### 5.4.1.1	General

The purpose of the primary authentication and key agreement procedure is to enable mutual authentication between the UE and the network and to provide keying material that can be used between the UE and network in subsequent security procedures, as specified in 3GPP TS 33.501 [12].

Two methods are defined:

a)	EAP based primary authentication and key agreement procedure.

b)	5G AKA based primary authentication and key agreement procedure.

The UE and the AMF shall support the EAP based primary authentication and key agreement procedure and the 5G AKA based primary authentication and key agreement procedure.

#### 5.4.1.2	EAP based primary authentication and key agreement procedure

##### 5.4.1.2.1	General

Extensible authentication protocol (EAP) as specified in IETF RFC 3748 [19] enables authentication using various EAP methods.

EAP defines four types of EAP messages:

a)	an EAP-request message;

b)	an EAP-response message;

c)	an EAP-success message; and

d)	an EAP-failure message.

Several rounds of exchanges of an EAP-request message and a related EAP-response message can be required to achieve the authentication (see example in figure 5.4.1.2.1.1).

The EAP-request message is transported from the network to the UE using the AUTHENTICATION REQUEST message of the EAP message reliable transport procedure.

The EAP-response message is transported from the UE to the network using the AUTHENTICATION RESPONSE AUTHENTICATION RESPONSE message of the EAP message reliable transport procedure.

If the authentication of the UE completes successfully, the EAP-success message is transported from the network to the UE using the SECURITY MODE COMMAND message of the security mode command procedure.

Editor's note:	The security mode command procedure is not yet included in the TR, and EAP authentication procedure may need to be aligned when included.

If the authentication of the UE completes unsuccessfully, the EAP-failure message is transported from the network to the UE in a response of the initial 5GMM procedure as part of which the EAP based primary authentication and key agreement procedure is performed.

The AMF shall set the authenticator retransmission timer specified in IETF RFC 3748 [19] subclause 4.3 to infinite value.

NOTE:	The EAP message reliable transport procedure provides a reliable transport of EAP messages and therefore retransmissions at the EAP layer do not occur.

The AUSF and the AMF support exchange of EAP messages using N12.

Editor's note:	The ngKSI handling at EAP authentication is FFS.

Figure 5.4.1.2.1.1: EAP based primary authentication and key agreement procedure

##### 5.4.1.2.2	EAP-AKA' related procedures

###### 5.4.1.2.2.1	General

The UE shall support acting as EAP-AKA' peer as specified in IETF RFC 5448 [24]. The AUSF may support acting as EAP-AKA' server as specified in IETF RFC 5448 [19].

The EAP-AKA' enables mutual authentication of the UE and the network.

###### 5.4.1.2.2.2	Initiation

In order to initiate the EAP based primary authentication and key agreement procedure using EAP-AKA', the AUSF shall send an EAP-request/AKA'-challenge message as specified in IETF RFC 5448 [24]. The AUSF shall set the AT_KDF_INPUT attribute of the EAP-request/AKA'-challenge message to the SNN. The SNN is in format described in subclause 9.9.1. The AUSF may include AT_RESULT_IND attribute in the EAP-request/AKA'-challenge message.

Upon receiving an EAP-request/AKA'-challenge message, the UE checks whether the network name field of the AT_KDF_INPUT attribute is the SNN constructed according to subclause 9.9.1.

###### 5.4.1.2.2.3	UE successfully authenticates network

If the SNN check is successful and procedures for handling of the EAP-request/AKA'-challenge message specified in IETF RFC 5448 [19] are successful, the UE shall generate KAUSF, KSEAF, and KAMF as described in 3GPP TS 33.501 [12] and shall send an EAP-response/AKA'-challenge message as specified in IETF RFC 5448 [24].

If the EAP-request/AKA'-challenge message contains AT_RESULT_IND attribute, the UE may include AT_RESULT_IND attribute in the EAP-response/AKA'-challenge message.

###### 5.4.1.2.2.4	Errors when handling EAP-request/AKA'-challenge message

If the SNN check fails or the UE does not accept AUTN during handling of the EAP-request/AKA'-challenge message as specified in IETF RFC 5448 [24], the UE shall send an EAP-response/AKA'-authentication-reject message as specified in IETF RFC 5448 [24].

If the SNN check is successful but the UE detects that the sequence number in AUTN is not correct during handling of the EAP-request/AKA'-challenge message as specified in IETF RFC 5448 [24], the UE shall send an EAP-response/AKA'-synchronization-failure message as specified in IETF RFC 5448 [24].

If the SNN check is successful, the sequence number in AUTN is correct and the UE detects another error during handling of the EAP-request/AKA'-challenge message as specified in IETF RFC 5448 [24], the UE shall send an EAP-response/AKA'-client-error message as specified in IETF RFC 5448 [24].

###### 5.4.1.2.2.5	Network successfully authenticates UE

Upon reception of the EAP-response/AKA'-challenge message, if procedures for handling an EAP-response/AKA'-challenge message as specified in IETF RFC 5448 [24] are successful, the AUSF may generate KAUSF and shall generate KSEAF as described in 3GPP TS 33.501 [12] and checks whether the AT_RESULT_IND attribute is included in the EAP-response/AKA'-challenge message and:

a)	if the AT_RESULT_IND attribute is included in the EAP-response/AKA'-challenge message, the AUSF shall send an EAP-request/AKA'-notification message as specified in IETF RFC 5448 [24]; and

b)	if the AT_RESULT_IND attribute is not included in the EAP-response/AKA'-challenge message, the AUSF shall send an EAP-success message as specified in IETF RFC 5448 [24] along with the KSEAF and shall consider the procedure complete.

NOTE:	SEAF generates KAMF based on the received KSEAF immediately following the primary authentication and key agreement procedure and provides KAMF to AMF.

###### 5.4.1.2.2.6	UE handling EAP-AKA' notification

Upon receiving an EAP-request/AKA'-notification message, the UE shall send an EAP-response/AKA'-notification message as specified in IETF RFC 5448 [24].

###### 5.4.1.2.2.7	Network sending EAP-success message

Upon reception of the EAP-response/AKA'-notification message, if earlier procedures for handling an EAP-request/AKA'-challenge message as specified in IETF RFC 5448 [24] were successful, the AUSF shall send an EAP-success message as specified in IETF RFC 5448 [24] along with the KSEA and shall consider the procedure complete.

NOTE:	SEAF generates KAMF based on the received KSEAF immediately following the primary authentication and key agreement procedure and provides KAMF to AMF.

###### 5.4.1.2.2.8	UE handling EAP-success message

Upon receiving an EAP-success message, the UE shall consider the procedure complete.

###### 5.4.1.2.2.9	Network not successfully authenticates UE

Upon reception of the EAP-response/AKA'-challenge message, if procedures for handling an EAP-response/AKA'-challenge message as specified in IETF RFC 5448 [24] are not successful, the AUSF shall send an EAP-request/AKA'-notification message that implies failure as specified in IETF RFC 5448 [24].

###### 5.4.1.2.2.10	Network sending EAP-failure message

Upon reception of the EAP-response/AKA'-notification message, if earlier procedures for handling an EAP-request/AKA'-challenge message as specified in IETF RFC 5448 [24] were not successful, the AUSF shall send an EAP-failure message as specified in IETF RFC 5448 [24] and shall consider the procedure complete.

###### 5.4.1.2.2.11	UE handling EAP-success

Upon receiving an EAP-failure message, the UE shall consider the procedure complete.

##### 5.4.1.2.3	EAP message reliable transport procedure

###### 5.4.1.2.3.1	General

The purpose of the EAP message reliable transport procedure is to provide a reliable transport of an EAP-request message from the network to the UE and of an EAP-response message from the UE to the network.

The EAP message reliable transport procedure is initiated by an AUTHENTICATION REQUEST message with the EAP message IE.

###### 5.4.1.2.3.2	EAP message reliable transport procedure initiation by the network

In order to initiate the EAP message reliable transport procedure, the AMF shall create an AUTHENTICATION REQUEST message.

The AMF shall set the EAP message IE of the AUTHENTICATION REQUEST message to the EAP-request message to be sent to the UE.

The AMF shall send the AUTHENTICATION REQUEST message to the UE, and the AMF shall start timer T3560 (see example in figure 5.4.1.2.3.2.1).

Figure 5.4.1.2.3.2.1: EAP message reliable transport procedure

Upon receipt of a AUTHENTICATION REQUEST message with the EAP message IE, the UE handles the EAP message received in the EAP message IE of the AUTHENTICATION REQUEST message.

###### 5.4.1.2.3.3	EAP message reliable transport procedure accepted by the UE

The UE shall create a AUTHENTICATION RESPONSE message.

If the received EAP message is an EAP-request message, the UE shall set the EAP message IE of the AUTHENTICATION RESPONSE message to the EAP-response message responding to the received EAP-request message.

The UE shall send the AUTHENTICATION RESPONSE message to the AMF.

Upon receipt of a AUTHENTICATION RESPONSE message, the AMF shall stop timer T3560. If the EAP message IE is included in the AUTHENTICATION RESPONSE message, the AMF handles the EAP message received in the EAP message IE of the AUTHENTICATION RESPONSE message.

###### 5.4.1.2.3.4	Abnormal cases on the network side

The following abnormal cases can be identified:

a)	Expiry of timer T3560

Editor's note:	Further abnormal cases are FFS

###### 5.4.1.2.3.5	Abnormal cases in the UE

Editor's note:	Abnormal cases are FFS

#### 5.4.1.3	5G AKA based primary authentication and key agreement procedure

##### 5.4.1.3.1	General

The purpose of the 5G AKA based primary authentication and key agreement procedure is to provide mutual authentication between the UE and the network and to agree on a key KAMF (see 3GPP TS 33.501 [12]). The cases when the 5G AKA based primary authentication and key agreement procedure is used are defined in 3GPP TS 33.501 [11].

The 5G AKA based primary authentication and key agreement procedure is always initiated and controlled by the network. However, the UE can reject the 5G authentication challenge sent by the network.

The UE shall proceed with an 5G authentication challenge only if a USIM is present.

A partial native 5GS security context is established in the UE and the network when an 5G authentication is successfully performed. During a successful 5G AKA based primary authentication and key agreement procedure, the CK and IK are computed by the USIM. CK and IK are then used by the ME as key material to compute a new key, KAMF. KAMF is stored in the 5GS security contexts (see 3GPP TS 33.501 [12]) of both the network and in the volatile memory of the ME while attached to the network, and is the root for the 5GS integrity protection and ciphering key hierarchy.

The 5G AKA based primary authentication and key agreement procedure is initiated by an AUTHENTICATION REQUEST message without the EAP message IE.

##### 5.4.1.3.2	Authentication initiation by the network

The network may initiate a 5G AKA based primary authentication and key agreement procedure for a UE in 5GMM-CONNECTED mode at any time. For restrictions applicable after handover or inter-system handover to N1 mode see subclause 5.4.1.2.3.

The network initiates the 5G AKA based primary authentication and key agreement procedure by sending an AUTHENTICATION REQUEST message to the UE and starting the timer T3560 (see example in figure 5.4.1.3.2.1). The AUTHENTICATION REQUEST message contains the parameters necessary to calculate the authentication response (see 3GPP TS 33.501 [12]).

If an ngKSI is contained in an initial NAS message during a 5GMM procedure, the network shall include a different ngKSI value in the AUTHENTICATION REQUEST message when it initiates a 5G AKA based primary authentication and key agreement procedure.

Figure 5.4.1.3.2.1: 5G AKA based primary authentication and key agreement procedure

##### 5.4.1.3.3	Authentication response by the UE

The UE shall respond to an AUTHENTICATION REQUEST message. With the exception of the cases described in subclause 5.4.1.3.5, the UE shall process the authentication challenge data and respond with an AUTHENTICATION RESPONSE message to the network.

Upon a successful 5G authentication challenge, the UE shall determine the PLMN identity to be used for the calculation of the new KAMF from the authentication challenge data according to the following rules:

a)	When the UE moves from 5GMM-IDLE mode to 5GMM-CONNECTED mode, until the first handover, the UE shall use the PLMN identity of the selected PLMN; and

b)	After handover or inter-system handover to N1 mode,

1)	if the target cell is not a shared network cell, the UE shall use the PLMN identity received as part of the broadcast system information;

2)	if the target cell is a shared network cell and the UE has a valid 5G-GUTI, the UE shall use the PLMN identity that is part of the 5G-GUTI; and

3)	if the target cell is a shared network cell and the UE has a valid 4G-GUTI and TAI, but not a valid 5G-GUTI, the UE shall use the PLMN identity that is part of the TAI.

Editor's note:	Security context coordination between EPS and 5GS is FFS.

Upon a successful 5G authentication challenge, the new KAMF calculated from the authentication challenge data shall be stored in a new 5GS security context in the volatile memory of the ME.

The USIM will compute the authentication response (RES) using the authentication challenge data received from the ME, and pass RES to the ME.

In order to avoid a synchronisation failure, when the UE receives an AUTHENTICATION REQUEST message, the UE shall store the received RAND together with the RES returned from the USIM in the volatile memory of the ME. When the UE receives a subsequent AUTHENTICATION REQUEST message, if the stored RAND value is equal to the new received value in the AUTHENTICATION REQUEST message, then the ME shall not pass the RAND to the USIM, but shall send the AUTHENTICATION RESPONSE message with the stored RES. If there is no valid stored RAND in the ME or the stored RAND is different from the new received value in the AUTHENTICATION REQUEST message, the ME shall pass the RAND to the USIM, shall override any previously stored RAND and RES with the new ones and start, or reset and restart timer T3516.

The RAND and RES values stored in the ME shall be deleted and timer T3516, if running, shall be stopped:

a)	upon receipt of a

1)	SECURITY MODE COMMAND message,

2)	SERVICE REJECT message,

3)	REGISTRATION REJECT message,

4)	REGISTRATION ACCEPT message, or

5)	AUTHENTICATION REJECT message;

b)	upon expiry of timer T3516;

1)	if the UE enters the 5GMM state 5GMM-DEREGISTERED or 5GMM-NULL; or

2)	if the UE enters 5GMM-IDLE mode.

##### 5.4.1.3.4	Authentication completion by the network

Upon receipt of an AUTHENTICATION RESPONSE message, the network stops the timer T3560 and checks the correctness of RES (see 3GPP TS 33.501 [12]).

If the 5G AKA based primary authentication and key agreement procedure has been completed successfully and the related ngKSI is stored in the 5GS security context of the network, the network shall include a different ngKSI value in the AUTHENTICATION REQUEST message when it initiates a new 5G AKA based primary authentication and key agreement procedure.

Upon receipt of an AUTHENTICATION FAILURE message, the network stops the timer T3560. In the case where the 5GMM cause #21 "synch failure" is received, the core network may renegotiate with the UDM/AUSF and provide the UE with new authentication parameters.

##### 5.4.1.3.5	Authentication not accepted by the network

Editor's note:	Authentication not accepted for normal services by the network is FFS.

Depending on local requirements or operator preference for emergency services, if the UE initiates a registration procedure with 5GS registration type IE set to "emergency registration" and the AMF is configured to allow emergency registration without user identity, the AMF needs not follow the procedures specified for the authentication failure in the present subclause. The AMF may continue a current 5GMM specific procedure.

##### 5.4.1.3.6	Authentication not accepted by the UE

Editor's note:	Authentication not accepted by the UE is FFS.

##### 5.4.1.3.7	Abnormal cases

Editor's note:	Abnormal cases are FFS. However the below message flow is expected to apply, .i.e. an authentication failure message is needed.

Figure 5.4.1.3.7.1: Authentication failure during 5G AKA based primary authentication and key agreement procedure

### 5.4.2	Security mode control procedure

#### 5.4.2.1	General

The purpose of the NAS security mode control procedure is to take an 5GS security context into use, and initialise and start NAS signalling security between the UE and the AMF with the corresponding 5GS NAS keys and 5GS security algorithms.

Furthermore, the network may also initiate the security mode control procedure in the following cases:

-	in order to change the NAS security algorithms for a current 5GS security context already in use; and

-	in order to change the value of uplink NAS COUNT used in the latest SECURITY MODE COMPLETE message as described in 3GPP TS 33.501 [12], subclause 8.3.1.4.2.

For restrictions concerning the concurrent running of a security mode control procedure with other security related procedures in the AS or inside the core network see 3GPP TS 33.501 [12], subclause 6.5.3.6.

#### 5.4.2.2	NAS security mode control initiation by the network

The AMF initiates the NAS security mode control procedure by sending a SECURITY MODE COMMAND message to the UE and starting timer T3560 (see example in figure 5.4.2.2).

The AMF shall reset the downlink NAS COUNT counter and use it to integrity protect the initial SECURITY MODE COMMAND message if the security mode control procedure is initiated:

-	to take into use the S security context created after a successful execution of the 5G AKA based primary authentication and key agreement procedure;

-	upon receipt of REGISTRATION REQUEST message including an eKSI, if the AMF wishes to create a mapped 5GS security context (i.e. the type of security context flag is set to "mapped security context" in the NAS key set identifier IE included in the SECURITY MODE COMMAND message).

Editor's note:	The name of NAS key set identifier used in 5GS is FFS.

The AMF shall send the SECURITY MODE COMMAND message unciphered, but shall integrity protect the message with the 5GS integrity key based on KAMF or mapped K'AMF indicated by the ngKSI included in the message. The AMF shall set the security header type of the message to "integrity protected with new 5GS security context".

Editor's note:	Handling at emergency registration and emergency PDU sessions is FFS.

Upon receipt of a REGISTRATION REQUEST message including an eKSI, if the AMF does not have the valid current 5GS security context indicated by the UE, the AMF shall indicate the use of the new mapped 5GS security context to the UE by setting the type of security context flag in the NAS key set identifier IE to "mapped security context" and the KSI value related to the security context of the source system.

Editor's note:	Handling at non-existing 5GS security context indicated by the UE when an emergency PDU session exists is FFS.

While having a current mapped 5GS security context with the UE, if the AMF wants to take the native 5GS security context into use, the AMF shall include the ngKSI that indicates the native 5GS security context in the SECURITY MODE COMMAND message.

The AMF shall include the replayed security capabilities of the UE (including the security capabilities with regard to NAS, RRC and UP (user plane) ciphering as well as NAS and RRC integrity, and other possible target network security capabilities, i.e. E-UTRAN if the UE included them in the message to network), the replayed nonceUE when creating a mapped 5GS security context and if the UE included it in the message to the network, the selected 5GS ciphering and integrity algorithms and the Key Set Identifier (ngKSI).

The AMF shall include both the nonceAMF and the nonceUE when creating a mapped 5GS security context during inter-system change from S1 mode to N1 mode in 5GMM-IDLE mode.

The AMF may initiate a SECURITY MODE COMMAND in order to change the 5GS security algorithms for a current 5GS security context already in use. The AMF re-derives the 5GS NAS keys from KAMF with the new 5GS algorithm identities as input and provides the new 5GS algorithm identities within the SECURITY MODE COMMAND message. The AMF shall set the security header type of the message to "integrity protected with new 5GS security context".

If, during an ongoing registration procedure, the AMF is initiating a SECURITY MODE COMMAND (i.e. after receiving the REGISTRATION REQUEST message, but before sending a response to that message) and the REGISTRATION REQUEST message does not successfully pass the integrity check at the AMF, the AMF shall calculate the HASHAMF of the entire plain REGISTRATION REQUEST message as described in 3GPP TS 33.501 [12] and shall include the HASHAMF in the SECURITY MODE COMMAND message.

Additionally, the AMF may request the UE to include its IMEISV in the SECURITY MODE COMPLETE message.

NOTE 2:	The AS and NAS security capabilities will be the same, i.e. if the UE supports one algorithm for NAS, the same algorithm is also supported for AS.

Figure 5.4.2.2: Security mode control procedure

#### 5.4.2.3	NAS security mode command accepted by the UE

Upon receipt of the SECURITY MODE COMMAND message, the UE shall check whether the security mode command can be accepted or not. This is done by performing the integrity check of the message and by checking that the received replayed UE security capabilities and the received nonceUE have not been altered compared to the latest values that the UE sent to the network. However, the UE is not required to perform the checking of the received nonceUE if the UE does not want to re-generate the K'AMF (i.e. the SECURITY MODE COMMAND message is to derive and take into use a mapped 5GS security context and the ngKSI matches the current 5GS security context, if it is a mapped 5GS security context).

Editor's note:	Handling at emergency PDU sessions is FFS.

Editor's note:	Handling of "null algorithms" is FFS.

If the type of security context flag included in the SECURITY MODE COMMAND message is set to "native security context" and if the KSI matches a valid non-current native 5GS security context held in the UE while the UE has a mapped 5GS security context as the current 5GS security context, the UE shall take the non-current native 5GS security context into use which then becomes the current native 5GS security context and delete the mapped 5GS security context.

If the SECURITY MODE COMMAND message can be accepted, the UE shall take the 5GS security context indicated in the message into use. The UE shall in addition reset the uplink NAS COUNT counter if:

-	the SECURITY MODE COMMAND message is received in order to take a 5GS security context into use created after a successful execution of the 5G AKA based primary authentication and key agreement procedure;

-	the SECURITY MODE COMMAND message received includes the type of security context flag set to "mapped security context" in the NAS key set identifier IE the ngKSI does not match the current 5GS security context, if it is a mapped 5GS security context.

If the SECURITY MODE COMMAND message can be accepted and a new 5GS security context is taken into use and SECURITY MODE COMMAND message does not indicate the "null integrity protection algorithm"5G-IA0 as the selected NAS integrity algorithm, the UE shall:

-	if the SECURITY MODE COMMAND message has been successfully integrity checked using an estimated downlink NAS COUNT equal to 0, then the UE shall set the downlink NAS COUNT of this new 5GS security context to 0;

-	otherwise the UE shall set the downlink NAS COUNT of this new 5GS security context to the downlink NAS COUNT that has been used for the successful integrity checking of the SECURITY MODE COMMAND message.

If the SECURITY MODE COMMAND message can be accepted, the UE shall send a SECURITY MODE COMPLETE message integrity protected with the selected 5GS integrity algorithm and the 5GS NAS integrity key based on the KAMF or mapped K'AMF if the type of security context flag is set to "mapped security context" indicated by the ngKSI. When the SECURITY MODE COMMAND message includes the type of security context flag set to "mapped security context" in the NAS key set identifier IE, the nonceAMF and the nonceUE, then the UE shall either:

-	generate K'AMF from both the nonceAMF and the nonceUE as indicated in 3GPP TS 33.501 [12];or

-	check whether the SECURITY MODE COMMAND message indicates the ngKSI of the current 5GS security context, if it is a mapped 5GS security context, in order not to re-generate the K'AMF.

Furthermore, if the SECURITY MODE COMMAND message can be accepted, the UE shall cipher the SECURITY MODE COMPLETE message with the selected 5GS ciphering algorithm and the 5GS NAS ciphering key based on the KAMF or mapped K'AMF indicated by the ngKSI. The UE shall set the security header type of the message to "integrity protected and ciphered with new 5GS security context".

From this time onward the UE shall cipher and integrity protect all NAS signalling messages with the selected 5GS integrity and ciphering algorithms.

If the AMF indicated in the SECURITY MODE COMMAND message that the IMEISV is requested, the UE shall include its IMEISV in the SECURITY MODE COMPLETE message.

If, during an ongoing registration procedure, the SECURITY MODE COMMAND message includes a HASHAMF, the UE shall compare HASHAMF with a hash value locally calculated as described in 3GPP TS 33.501 [12] from the entire plain REGISTRATION REQUEST message that the UE had sent to initiate the procedure. If HASHAMF and the locally calculated hash value are different, the UE shall include the complete REGISTRATION REQUEST message which the UE had previously sent in the Replayed NAS message container IE of the SECURITY MODE COMPLETE message.

If, prior to receiving the SECURITY MODE COMMAND message, the UE had sent an initial NAS message containing a limited set of IEs needed to establish security context, the UE shall include the complete initial message in the NAS SECURITY MODE COMPLETE message.

#### 5.4.2.4	NAS security mode control completion by the network

The AMF shall, upon receipt of the SECURITY MODE COMPLETE message, stop timer T3560. From this time onward the AMF shall integrity protect and encipher all signalling messages with the selected 5GS integrity and ciphering algorithms.

If the SECURITY MODE COMPLETE message contains a Replayed NAS container message IE with an REGISTRATION REQUEST message, the AMF shall complete the ongoing registration procedure by considering the REGISTRATION REQUEST message contained in the Replayed NAS message container IE as the message that triggered the procedure.

#### 5.4.2.5	NAS security mode command not accepted by the UE

If the security mode command cannot be accepted, the UE shall send a SECURITY MODE REJECT message. The SECURITY MODE REJECT message contains an 5GMM cause that typically indicates one of the following cause values:

#23:	UE security capabilities mismatch;

#24:	security mode rejected, unspecified.

Upon receipt of the SECURITY MODE REJECT message, the AMF shall stop timer T3560. The AMF shall also abort the ongoing procedure that triggered the initiation of the NAS security mode control procedure.

Both the UE and the AMF shall apply the 5GS security context in use before the initiation of the security mode control procedure, if any, to protect the SECURITY MODE REJECT message and any other subsequent messages according to the rules in subclause TBD.

Editor´s note:	The NAS security subclause in the present document is FFS.

#### 5.4.2.6	Abnormal cases in the UE

The following abnormal cases can be identified:

a)	Transmission failure of SECURITY MODE COMPLETE message or SECURITY MODE REJECT message indication from lower layers (if the security mode control procedure is triggered by a registration procedure)

The UE shall abort the security mode control procedure and re-initiate the registration procedure.

b)	Transmission failure of SECURITY MODE COMPLETE message or SECURITY MODE REJECT message indication with TAI change from lower layers (if the security mode control procedure is triggered by a service request procedure)

If the current TAI is not in the TAI list, the security mode control procedure shall be aborted and a registration procedure shall be initiated.

If the current TAI is still part of the TAI list, the security mode control procedure shall be aborted and it is up to the UE implementation how to re-run the ongoing procedure that triggered the security mode control procedure.

c)	Transmission failure of SECURITY MODE COMPLETE message or SECURITY MODE REJECT message indication without TAI change from lower layers (if the security mode control procedure is triggered by a service request procedure)

The security mode control procedure shall be aborted and it is up to the UE implementation how to re-run the ongoing procedure that triggered the security mode control procedure.

#### 5.4.2.7	Abnormal cases on the network side

The following abnormal cases can be identified:

# a)	Lower layer failure before the SECURITY MODE COMPLETE or SECURITY MODE REJECT message is received

The network shall abort the security mode control procedure.

# b)	Expiry of timer T3560

The network shall, on the first expiry of the timer T3560, retransmit the SECURITY MODE COMMAND message and shall reset and start timer T3560. This retransmission is repeated four times, i.e. on the fifth expiry of timer T3560, the procedure shall be aborted.

NOTE:	If the SECURITY MODE COMMAND message was sent to create a mapped 5GS security context during inter-system change from S1 mode to N1 mode, then the network does not generate new values for the nonceAMF and the nonceUE, but includes the same values in the SECURITY MODE COMMAND message (see the subclause 6.7.2 in 3GPP TS 33.501 [12]).

# c)	Collision between security mode control procedure and registration, service request or detach procedure not indicating switch off

# The network shall abort the security mode control procedure and proceed with the UE initiated procedure.

# d)	Collision between security mode control procedure and other 5GMM procedures than in item c

The network shall progress both procedures.

# e)	Lower layers indication of non-delivered NAS PDU due to handover

If the SECURITY MODE COMMAND message could not be delivered due to an intra AMF handover and the target TA is included in the TAI list, then upon successful completion of the intra AMF handover the AMF shall retransmit the SECURITY MODE COMMAND message. If a failure of the handover procedure is reported by the lower layer and the N1 signalling connection exists, the AMF shall retransmit the SECURITY MODE COMMAND message.

### 5.4.3	Identification procedure

#### 5.4.3.1	General

The purpose of this procedure is to to request a particular UE to provide specific identification parameters, e.g. the SUPI or the PEI. SUPI and PEI definition and structure are specified in 3GPP TS 23.003 [2].

#### 5.4.3.2	Identification initiation by the network

The AMF initiates the identification procedure by sending an IDENTITY REQUEST message to the UE and starting timer T3570 (see example in figure 5.4.3.2.1). The IDENTITY REQUEST message specifies the requested identification parameters in the Identity type information element.

Figure 5.4.3.2.1: Identification procedure

#### 5.4.3.3	Identification response by the UE

A UE shall be ready to respond to an IDENTITY REQUEST message at any time whilst in 5GMM-CONNECTED mode.

Upon receipt of the IDENTITY REQUEST message the UE shall send an IDENTITY RESPONSE message to the network. The IDENTITY RESPONSE message shall contain the identification parameters as requested by the network.

#### 5.4.3.4	Identification completion by the network

Upon receipt of the IDENTITY RESPONSE the network shall stop the timer Tf.

#### 5.4.3.5	Abnormal cases in the UE

The following abnormal cases can be identified:

a)	Transmission failure of the IDENTIFY RESPONSE message (if the identification procedure is triggered by a registration procedure)

Editor's note:	Further abnormal cases in the UE are FFS.

#### 5.4.3.6	Abnormal cases on the network side

The following abnormal cases can be identified:

a)	Expiry of timer T3570

The network shall, on the first expiry of the timer T3570, retransmit the IDENTITY REQUEST message and reset and restart the timer T3570. This retransmission is repeated four times, i.e. on the fifth expiry of timer T3570, the network shall abort the identification procedure and any ongoing 5GMM procedure.

Editor's note:	Further abnormal cases in the network are FFS.

### 5.4.4	Generic UE configuration update procedure

#### 5.4.4.1	General

The purpose of this procedure is to allow the AMF to update UE configuration by providing new parameter information within the command or requesting the UE to perform a mobility and periodic registration update procedure with the network to update the parameters.

The procedure may be initiated by the network and can only be used when the UE has an established 5GMM context, and the UE is in 5GMM-CONNECTED mode. The AMF may require a confirmation response in order to ensure that the parameter has been updated by the UE.

The following parameters are supported by the generic UE configuration update procedure without the need for triggering the UE to perform the mobility and periodic registration update procedure:

a)	5G-GUTI;

b)	TAI list;

c)	Service area list;

d)	Network identity and Time zone information (Full name for network, Short name for network, Local time zone, Universal time and local time zone, Network daylight saving time); and

e)	LADN information;

Editor's note:	It is FFS which configuration parameters are updated by this procedure (e.g. policy information).

Editor's note:	It is FFS whether the case when configuration provided by a different NF than the AMF is covered by this or a different NAS procedure (e.g. UE route selection policies (URSP) provided by PCF).

The following parameters may trigger the UE to perform Registration Update procedure:

a)	Allowed NSSAI.

The following parameters require triggering the UE to perform the mobility and periodic registration update procedure:

a)	MICO; and

b)	Configured NSSAI.

Editor's note:	Other parameters requiring negotiation are FFS.

Figure 5.4.4.1.1: Generic UE configuration update procedure

#### 5.4.4.2	Generic UE configuration update procedure initiated by the network

The AMF shall initiate the generic UE configuration procedure by sending the CONFIGURATION UPDATE COMMAND message to the UE.

The AMF shall in the CONFIGURATION UPDATE COMMAND message either:

a)	include one or more of 5G-GUTI, TAI list, allowed NSSAI, LADN information, service area list, MICO indication NITZ information, or configured NSSAI;

b)	indicate registration requested; or

c)	a combination of both.

If an acknowledgement from the UE is requested, the AMF shall indicate acknowledgement requested in the Configuration update indication IE in the CONFIGURATION UPDATE COMMAND message and shall start timer T3555. Acknowledgement shall be requested for all parameters except when only NITZ is included.

To initiate parameter re-negotiation between the UE and network, the AMF shall indicate "registration requested" in the Configuration update indication IE in the CONFIGURATION UPDATE COMMAND message. In this case, the acknowledgement shall be requested.

If a new allowed NSSAI information or AMF re-configuration of supported S-NSSAIs requires an AMF relocation, the AMF shall indicate "registration requested" in the Configuration update indication IE and include the Allowed NSSAI IE in the CONFIGURATION UPDATE COMMAND message. In this case, the acknowledgement shall be requested.

If the AMF includes the Configured NSSAI in the CONFIGURATION UPDATE COMMAND message, the AMF shall indicate "registration requested" in the Configuration update indication IE in the message.

The CONFIGURATION UPDATE COMMAND message shall not include both a new allowed NSAI information and a new configured NSSAI information.

During an established 5GMM context, the network may send none, one, or more CONFIGURATION UPDATE COMMAND messages to the UE. If more than one CONFIGURATION UPDATE COMMAND message is sent, the messages need not have the same content.

#### 5.4.4.3	Generic UE configuration update accepted by the UE

Upon receiving the CONFIGURATION UPDATE COMMAND message, the UE shall use the contents to update appropriate information stored within the UE.

If acknowledgement requested is indicated in the Configuration update indication IE in the CONFIGURATION UPDATE COMMAND message and:

a)	if all information elements included are successfully accepted by the UE; or

b)	if "registration requested" in the Configuration update indication IE is indicated;

the UE shall send a CONFIGURATION UPDATE COMPLETE message.

If the UE receives a new 5G-GUTI in the CONFIGURATION UPDATE COMMAND message, the UE considers the new 5G-GUTI as valid and the old 5G-GUTI as invalid; otherwise, the UE shall consider the old 5G-GUTI as valid.

If the UE receives a new TAI list in the CONFIGURATION UPDATE COMMAND message, the UE shall consider the new TAI list as valid and the old TAI list as invalid; otherwise, the UE shall consider the old TAI list as valid.

If the UE receives a new service area list in the CONFIGURATION UPDATE COMMAND message, the UE shall consider the new service area list as valid and the old service area list as invalid; otherwise, the UE shall consider the old service area list, if any, as valid.

If the UE receives new NITZ information in the CONFIGURATION UPDATE COMMAND message, the UE considers the new NITZ information as valid and the old NITZ information as invalid; otherwise, the UE shall consider the old NITZ information as valid.

If the UE receives a new LADN information in the CONFIGURATION UPDATE COMMAND message, the UE shall consider the new LADN information as valid and the old LADN information as invalid; otherwise, the UE shall consider the old LADN information as valid.

If the UE receives a MICO indication and an indication for "registration requested" in the Configuration update indication IE in the CONFIGURATION UPDATE COMMAND message, the UE shall send a REGISTRATION REQUEST message as specified in subclause 5.5.1.3 to re-negotiate MICO mode with the network. If there is also a new allowed NSSAI in the CONFIGURATION UPDATE COMMAND message, the UE shall send a REGISTRATION REQUEST message after releasing the existing NAS signalling connection as described below for updating the allowed NSSAI, and re-negotiate MICO mode with the network.

If the UE receives a new allowed NSSAI in the CONFIGURATION UPDATE COMMAND message, the UE shall consider the new allowed NSSAI as valid, store the allowed NSSAI as specified in subclause 4.6.2.2 and consider the old allowed NSSAI as invalid; otherwise, the UE shall consider the old Allowed NSSAI as valid.

If the UE receives an allowed NSSAI in the CONFIGURATION UPDATE COMMAND message and the UE has one or more PDU session contexts associated with S-NSSAI(s) not included in the received allowed NSSAI, the UE shall locally release all such PDU session context(s).

If the UE receives a new configured NSSAI in the CONFIGURATION UPDATE COMMAND message, the UE shall consider the new configured NSSAI for the registered PLMN as valid and the old configured NSSAI for the registered PLMN as invalid; otherwise, the UE shall consider the old configured NSSAI for the registered PLMN as valid. See subclause 4.6.2.2. In this case, the UE shall delete the stored allowed NSSAI and perform a mobility registration updating procedure as specified in subclause 5.5.1.3 to obtain a new allowed NSSAI.

If the CONFIGURATION UPDATE COMMAND indicates "registration requested" in the Configuration update indication IE and a new allowed NSSAI is included, the UE shall set the 5G-GUTI as invalid and wait for the network to release the N1 NAS signalling connection prior to a registration update procedure using SUPI is performed as specified in subclause 5.3.1.2. When the NAS signalling connection is released the UE shall deactivate the PDU session(s) context locally, if any. At a successful registration update, the UE should re-establish any previously active PDU sessions and:

a)	deactivate the PDU session(s) context locally, if any; and

b)	set the 5G-GUTI as invalid and initiate a mobility registration update procedure with a REGISTRATION REQUEST message using SUPI and including the new allowed NSSAI in the requested NSSAI as specified in subclause 5.5.1.3. If the UE was registered over both 3GPP access and non-3GPP access with the same PLMN, the UE should register again over both 3GPP access and non-3GPP access on the same PLMN. In this case, the UE should first register over the 3GPP access; and

c)	should re-establish any previous active PDU sessions(s).

Editor's note:	It is FFS if the UE must wait for AMF to release the N1 NAS signalling connection prior to the UE attempts to initiate a mobility update procedure with SUPI.

Editor's note:	Further details on handling of specific IEs are FFS.

Editor's note:	it is FFS whether the AMF indicates which access technology the parameters in the message are associated to.

#### 5.4.4.4	Generic UE configuration update completion by the network

Upon receipt of the CONFIGURATION UPDATE COMPLETE message, the AMF shall stop the timer T3555.

If a new 5G-GUTI was included in the CONFIGURATION UPDATE COMMAND message, the AMF shall consider the new 5G-GUTI as valid and the old 5G-GUTI as invalid.

If a new TAI list was included in the CONFIGURATION UPDATE COMMAND message, the AMF shall consider the new TAI list as valid and the old TAI list as invalid.

If a new service area list was included in the CONFIGURATION UPDATE COMMAND message, the AMF shall consider the new service area list as valid and the old service area list as invalid.

If new allowed NSSAI information was included in the CONFIGURATION UPDATE COMMAND message, the AMF shall consider the new Allowed NSSAI information as valid and the old allowed NSSAI information as invalid.

If a new LADN information was included in the CONFIGURATION UPDATE COMMAND message, the AMF shall consider the new LADN information as valid and the old LADN information as invalid. In addition, if registration requested was indicated in the CONFIGURATION UPDATE COMMAND message, the AMF shall initiate the release of the N1 NAS signalling connection.

Editor's note:	Further details on handling of specific IEs are FFS.

#### 5.4.4.5	Abnormal cases in the UE

The following abnormal cases can be identified:

a)	Transmission failure of the CONFIGURATION UPDATE COMPLETE message

Editor's note:	Further abnormal cases in the UE are FFS.

#### 5.4.4.6	Abnormal cases on the network side

The following abnormal cases can be identified:

a)	Expiry of timer T3555

The network shall, on the first expiry of the timer T3555, retransmit the CONFIGURATION UPDATE COMMAND message and shall reset and start timer T3555. This retransmission is repeated four times, i.e. on the fifth expiry of timer T3555, the procedure shall be aborted.

b)	Lower layer failure

If a lower layer failure is detected before the CONFIGURATION UPDATE COMPLETE message is received, the old and the new 5G-GUTI shall be considered as valid until the old 5G-GUTI can be considered as invalid by the AMF. If a new TAI list was provided in the CONFIGURATION UPDATE COMMAND message, the old and new TAI list shall also be considered as valid until the old TAI list can be considered as invalid by the AMF.

During this period the AMF:

1)	may first use the old 5G-S-TMSI from the old 5G-GUTI for paging within the area defined by the old TAI list for an implementation dependent number of paging attempts for network originated transactions. If a new TAI list was provided with old 5G-GUTI in the CONFIGURATION UPDATE COMMAND message, the new TAI list should also be used for paging. Upon response from the UE, the AMF may re-initiate the CONFIGURATION UPDATE COMMAND. If the response is received from a tracking area within the old and new TAI list, the network shall re-initiate the CONFIGURATION UPDATE COMMAND message. If no response is received to the paging attempts, the network may use the new 5G-S-TMSI from the new 5G-GUTI for paging for an implementation dependent number of paging attempts. In this case, if a new TAI list was provided with new 5G-GUTI in the CONFIGURATION UPDATE COMMAND message, the new TAI list shall be used instead of the old TAI list. Upon response from the UE the AMF shall consider the new 5G-GUTI as valid and the old 5G-GUTI as invalid. If no response is received to the paging attempts, the AMF may use the IMSI for paging for an implementation dependent number of paging attempts;

2)	shall consider the new 5G-GUTI as valid if it is used by the UE and, additionally, the new TAI list as valid if it was provided with this 5G-GUTI in the CONFIGURATION UPDATE COMMAND message; and

3)	may use the identification procedure followed by a new generic UE configuration update procedure if the UE uses the old 5G-GUTI.

Editor's note:	Further abnormal cases in the network are FFS.

### 5.4.5	NAS transport procedure(s)

#### 5.4.5.1	General

The purpose of the NAS transport procedures is to provide a transport of payload between the UE to the AMF. The type of the payload is identified by the Payload container type IE and includes one of the following:

a-	a single 5GSM message;

b)	SMS; or

c)	an LPP message (see 3GPP TS 36.355 [13]).

Editor's note:	Other types of payload, such as generic application payload, are FFS.

Along with the payload, the NAS transport procedure may transport the associated information (e.g. PDU session information for 5GSM message payload).

#### 5.4.5.2	UE-initiated NAS transport procedure

##### 5.4.5.2.1	General

The purpose of the UE-initiated NAS transport procedure is to provide a transport of:

a)	a single 5GSM message;

b)	SMS; or

c)	an LPP message

and optional associated payload routing information from the UE to the AMF in a 5GMM message.

##### 5.4.5.2.2	UE-initiated NAS transport procedure initiation

In the connected mode, the UE initiates the NAS transport procedure by sending the UL NAS TRANSPORT message, as shown in figure 5.4.5.2.2.1.

In case a) in subclause 5.4.5.2.1, the UE shall:

-	include the PDU session information (PDU session ID, old PDU session ID, S-NSSAI, DNN, request type), if available:

-	set the Payload container type IE to "N1 SM information"; and

-	set the Payload container IE to the 5GSM message.

The UE shall set the PDU session ID IE to the PDU session ID. If an old PDU session ID is to be included, the UE shall set the Old PDU session ID IE to the old PDU session ID.

If an S-NSSAI is to be included, the UE shall set the S-NSSAI IE to the S-NSSAI selected for the PDU session from the allowed NSSAI for the serving PLMN, associated with the mapped configured NSSAI for the HPLMN if available in roaming scenarios.

If a DNN is to be included, the UE shall set the DNN IE to the DNN. 5GSM procedures specified in clause 9 describe conditions for inclusion of the S-NSSAI and the DNN.

If a request type is to be included, the UE shall set the Request type IE to the request type. The request type is not provided along 5GSM messages other than the PDU SESSION ESTABLISHMENT REQUEST message.

The UE shall send the UL NAS TRANSPORT message to the AMF (see example in figure 5.4.5.2.2.1).

In case b) in subclause 5.4.5.2.1, the UE shall:

-	set the Payload container type IE to "SMS"; and

-	set the Payload container IE to the SMS payload.

Editor's note:	How the UE selects the access type to deliver the UL NAS TRANSPORT message with the Payload container type IE set to "SMS" is FFS.

In case c) in subclause 5.4.5.2.1, the UE shall:

-	set the Payload container type IE to "LTE Positioning Protocol (LPP) message container";

-	set the Payload container IE to the LPP message payload; and

-	set the Additional information IE to the routing information provided by the upper layer location services application.

Figure 5.4.5.2.2.1: UE-initiated NAS transport procedure

##### 5.4.5.2.3	UE-initiated NAS transport of messages

Upon reception of a UL NAS TRANSPORT message, if the Payload container type IE is set to:

a)	"N1 SM information", the AMF looks up a PDU session routing context for:

1)	the UE and the PDU session ID IE in case the Old PDU session ID IE is not included, and:

NOTE 1:	If the Old PDU session ID IE is not included in the UL NAS TRANSPORT message and the AMF has received a reallocation requested indication from the SMF, the AMF needs to ignore the reallocation requested indication.

i)	if the AMF has a PDU session routing context for the PDU session ID and the UE, and the Request type IE is not included, the AMF shall forward the 5GSM message, and the PDU session ID IE towards the SMF identified by the SMF ID of the PDU session routing context;

ii)	if the AMF has a PDU session routing context for the PDU session ID and the UE, the PDU session routing context indicates that the PDU session is not an emergency PDU session, and the Request type IE is included and is set to "existing PDU session", the AMF shall forward the 5GSM message, the PDU session ID, the S-NSSAI (if received), the DNN (if received) and the request type towards the SMF identified by the SMF ID of the PDU session routing context;

iii)	if the AMF does not have a PDU session routing context for the PDU session ID and the UE, and the Request type IE is included and is set to "initial request":

A)	the AMF shall select an SMF. If the DNN IE is not included, the AMF shall use the default DNN as the DNN. If the S-NSSAI IE is not included, the AMF may determine a default S-NSSAI according to the user’s subscription context obtained from UDM and, if determined, the AMF may use the default S-NSSAI as the S-NSSAI; and

NOTE 2:	SMF selection is out of scope of CT1.

B)	if the SMF selection is successful:

-	the AMF shall store a PDU session routing context for the PDU session ID and the UE, shall set the SMF ID in the stored PDU session routing context to the SMF ID corresponding to the DNN in the user’s subscription context obtained from the UDM; and

-	the AMF shall forward the 5GSM message, the PDU session ID, the S-NSSAI (if received), the DNN (if received) and the request type towards the SMF identified by the SMF ID of the PDU session routing context;

iv)	if the AMF does not have a PDU session routing context for the PDU session ID and the UE, the Request type IE is included and is set to "existing PDU session", and the user’s subscription context obtained from the UDM contains an SMF ID corresponding to:

A)	the DNN in the DNN IE, if the DNN IE is included; or

B)	the default DNN, if the DNN IE is not included;

Editor's note:	FFS how to obtain the correct SMF ID when the UE has several PDU sessions with a given DNN, each PDU session is provided by a different SMF, and the UE attempts to handover one of those PDU sessions.

then:

A)	the AMF shall store a PDU session routing context for the PDU session ID and the UE, shall set the SMF ID in the stored PDU session routing context to the SMF ID corresponding to the DNN in the user’s subscription context obtained from the UDM; and

B)	the AMF shall forward the 5GSM message, the PDU session ID, the S-NSSAI (if received), the DNN (if received) and the request type towards the SMF identified by the SMF ID of the PDU session routing context;

v)	if the AMF does not have a PDU session routing context for the PDU session ID and the UE, the Request type IE is included and is set to "initial emergency request", and the AMF does not have a PDU session routing context for another PDU session ID of the UE indicating that the PDU session is an emergency PDU session:

A)	the AMF shall select an SMF. The AMF shall use the emergency DNN from the AMF emergency configuration data as the DNN, if configured. The AMF shall derive the SMF from the emergency DNN or use the statically configured SMF from the AMF emergency configuration data, if configured; and

B)	if the SMF selection is successful:

-	the AMF shall store a PDU session routing context for the PDU session ID and the UE, shall set the SMF ID in the stored PDU session routing context to the SMF ID of the selected SMF, and shall store an indication that the PDU session is an emergency PDU session in the stored PDU session routing context; and

-	the AMF shall forward the 5GSM message, the PDU session ID, the S-NSSAI (if configured), the DNN (if configured), and the request type towards the SMF identified by the SMF ID of the PDU session routing context; and

vi)	if the AMF does not have a PDU session routing context for the PDU session ID and the UE, the Request type IE is included and is set to "initial emergency request", and the AMF has a PDU session routing context indicating that the PDU session is an emergency PDU session for another PDU session ID of the UE:

A)	the AMF shall store a PDU session routing context for the PDU session ID and the UE and shall set the SMF ID in the stored PDU session routing context to the SMF ID of the PDU session routing context for the other PDU session ID of the UE; and

B)	the AMF shall forward the 5GSM message, the PDU session ID, the S-NSSAI (if configured), the DNN (if configured) and the request type towards the SMF identified by the SMF ID of the PDU session routing context; or

Editor's note:	Handover between 3GPP access and non-3GPP access of emergency PDU session is FFS.

2)	the UE and the Old PDU session ID IE in case the Old PDU session ID IE is included, and:

i)	the AMF has a PDU session routing context for the old PDU session ID and the UE and does not have a PDU session routing context for the PDU session ID and the UE, the Request type IE is included and is set to "initial request", and the AMF received a reallocation requested indication from the SMF indicating that the SMF is to be reused, the AMF shall store a PDU session routing context for the PDU session ID and the UE, set the SMF ID in the stored PDU session routing context to the SMF ID of the PDU session routing context for the old PDU session ID and the UE, and forward the 5GSM message, the PDU session ID, the old PDU session ID, the S-NSSAI (if received), the DNN (if received) and the request type towards the SMF identified by the SMF ID of the PDU session routing context;

ii)	the AMF has a PDU session routing context for the old PDU session ID and the UE and does not have a PDU session routing context for the PDU session ID and the UE, the Request type IE is included and is set to "initial request", the AMF received a reallocation requested indication from the SMF indicating that the SMF is to be reallocated:

A)	the AMF shall select an SMF. If the DNN IE is not included, the AMF shall use the default DNN as the DNN. If the S-NSSAI IE is not included, the AMF may determine a default S-NSSAI according to the user's subscription context obtained from UDM and, if determined, the AMF may use the default S-NSSAI as the S-NSSAI; and

B)	if the SMF selection is successful:

-	the AMF shall store a PDU session routing context for the PDU session ID and the UE and set the SMF ID of the PDU session routing context to the SMF ID of the selected SMF; and

-	the AMF shall forward the 5GSM message, the PDU session ID, the old PDU session ID, the S-NSSAI (if received), the DNN (if received) and the request type towards the SMF identified by the SMF ID of the PDU session routing context for the PDU session ID and the UE; and

b)	"SMS", the AMF shall forward the content of the Payload container IE to the SMSF associated with the UE; and

c)	"LTE Positioning Protocol (LPP) message container", the AMF shall forward the content of the Payload container IE to the LMF associated with the routing information included in the Additional information IE of the UL NAS TRANSPORT message.

Editor's note:	What to do if the AMF cannot locate the LMF is FFS.

##### 5.4.5.2.4	UE-initiated NAS transport of messages failure

Upon reception of an UL NAS TRANSPORT UNROUTABLE INDICATION message:

a)	if the Payload container type is set to "N1 SM information", the UE passes to the 5GSM sublayer an indication that the 5GSM message cannot be routed along with the 5GSM message from the Payload container IE of the UL NAS TRANSPORT UNROUTABLE INDICATION message.

##### 5.4.5.2.5	Abnormal cases on the network side

The following abnormal cases in AMF are identified:

a)	if the Old PDU session ID IE is not included in the UL NAS TRANSPORT message, the AMF does not have a PDU session routing context for the PDU session ID and the UE, the Request type IE is set to "initial request", and the SMF selection fails then the AMF may send to the UE an indication that the 5GSM message cannot be routed.

b)	if the Old PDU session ID IE is included in the UL NAS TRANSPORT message, the AMF has a PDU session routing context for the old PDU session ID and the UE and does not have a PDU session routing context for the PDU session ID and the UE, the Request type IE is set to "initial request", the AMF received a reallocation requested indication from the SMF indicating that the SMF is to be reallocated, and the SMF selection fails then the AMF may send to the UE an indication that the 5GSM message cannot be routed.

c)	if the AMF does not have a PDU session routing context for the PDU session ID and the UE, the Request type IE is set to "existing PDU session", and the user’s subscription context obtained from the UDM does not contain an SMF ID corresponding to:

1)	the DNN of the DNN IE, if the DNN IE is included; or

2)	the default DNN, if the DNN IE is not included;

then the AMF may send to the UE an indication that the 5GSM message cannot be routed.

d)	if the Old PDU session ID IE is included in the UL NAS TRANSPORT message, and the AMF has a PDU session routing context for the old PDU session ID and the UE and does not have a PDU session routing context for the PDU session ID and the UE, the Request type IE is set to "initial request" and the AMF has not received a reallocation requested indication, the AMF should select an SMF. If the SMF selection is successful, the AMF should store a PDU session routing context for the PDU session ID and the UE, set the SMF ID in the stored PDU session routing context to the selected SMF ID, and forward the 5GSM message, the PDU session ID, the old PDU session ID, the S-NSSAI (if received), the DNN (if received) and the request type towards the SMF ID of the PDU session routing context

e)	if the AMF has a PDU session routing context for the PDU session ID and the UE, the PDU session routing context indicates that the PDU session is an emergency PDU session, the Request type IE is set to "initial emergency request", the AMF should forward the 5GSM message, the PDU session ID, the S-NSSAI (if configured), the DNN (if configured) and the request type towards the SMF ID of the PDU session routing context

f)	if the Request type IE is set to "initial emergency request" and the S-NSSAI or the DNN is received, the AMF ignores the received S-NSSAI or the DNN and uses the emergency DNN from the AMF emergency configuration data, if any

g)	if the AMF does not have a PDU session routing context for the PDU session ID and the UE, and the Request type IE of the UL NAS TRANSPORT message is not provided, then the AMF may send to the UE an indication that the 5GSM message cannot be routed.

h)	if the AMF unsuccessfully attempted to forward the 5GSM message, the PDU session ID, the S-NSSAI (if received), the DNN (if received) and the request type (if received) towards a SMF ID, then the AMF may send to the UE an indication that the 5GSM message cannot be routed.

i)	the Old PDU session ID IE is included in the UL NAS TRANSPORT message, the AMF does not have a PDU session routing context for the old PDU session ID and the UE, the AMF does not have a PDU session routing context for the PDU session ID and the UE, the Request type IE is set to "initial request", the AMF should select an SMF. If the SMF selection is successful, the AMF should store a PDU session routing context for the PDU session ID and the UE, set the SMF ID in the stored PDU session routing context to the selected SMF ID, and forward the 5GSM message, the PDU session ID, the old PDU session ID, the S-NSSAI (if received), the DNN (if received) and the request type towards the SMF ID of the PDU session routing context.

In order to send to the UE an indication that the 5GSM message cannot be routed, the AMF shall create a UL NAS TRANSPORT UNROUTABLE INDICATION message. The AMF shall set the Payload container type IE of the UL NAS TRANSPORT UNROUTABLE INDICATION message to "N1 SM information", shall set the Payload container IE of the UL NAS TRANSPORT UNROUTABLE INDICATION message to the 5GSM message received in the Payload container IE of the UL NAS TRANSPORT message.

Editor's note: FFS whether DL NAS TRANSPORT message with some indication or a new payload type can be used instead of UL NAS TRANSPORT UNROUTABLE INDICATION message.

The AMF shall send the UL NAS TRANSPORT UNROUTABLE INDICATION message to the UE.

#### 5.4.5.3	Network-initiated NAS transport procedure

##### 5.4.5.3.1	General

The purpose of the network-initiated NAS transport procedure is to provide a transport of:

a)	a single 5GSM message;

b)	SMS; or

c)	an LPP message

from the AMF to the UE in a 5GMM message.

##### 5.4.5.3.2	Network-initiated NAS transport procedure initiation

In connected mode, the AMF initiaties the NAS transport procedure by sending the DL NAS TRANSPORT message, as shown in figure 5.4.5.3.2.1.

In case a) in subclause 5.4.5.3.1, i.e. upon reception from an SMF of a 5GSM message without an N1 SM delivery skip allowed indication for a UE or a 5GSM message with an N1 SM delivery skip allowed indication for a UE in the 5GMM-CONNECTED mode, the AMF shall:

a)	include the PDU session information (PDU session ID) in the PDU session ID IE;

b)	set the Payload container type IE to "N1 SM information"; and

c)	set the Payload container IE to the 5GSM message.

In case b) in subclause 5.4.5.3.1, i.e. upon reception from an SMSF of an SMS payload, the AMF shall:

a)	set the Payload container type IE to "SMS";

b)	set the Payload container IE to the SMS payload ; and

c)	select the access type to deliver the DL NAS TRANSPORT message as follows in case the access type selection is required:

1)	if the UE to receive the DL NAS TRANSPORT message is registered to the network via both 3GPP access and non-3GPP access, and the SMS allowed IE in the 5GMM context of the UE is set to "both 3GPP access and non-3GPP access", then the AMF selects either 3GPP access or non-3GPP access; and

2)	otherwise, the AMF selects 3GPP access.

NOTE:	The AMF selects an access type between 3GPP access and non-3GPP access based on operator policy.

In case c) in subclause 5.4.5.3.1 i.e. upon reception from an LMF of an LPP message payload,, the AMF shall:

a)	set the Payload container type IE to "LTE Positioning Protocol (LPP) message container";

b)	set the Payload container IE to the LPP message payload received from the LMF; and

c)	set the Additional information IE to the routing information associated with the LMF from which the LPP message was received.

Figure 5.4.5.3.2.1: Network-initiated NAS transport procedure

##### 5.4.5.3.3	Network-initiated NAS transport of messages

Upon reception of a DL NAS TRANSPORT message, if the Payload container type IE is set to:

a)	 "N1 SM information".the 5GSM message in the Payload container IE and the PDU session ID are handled in the 5GSM procedures specified in clause 9;

b)	"SMS", the UE shall forward the content of the Payload container IE to the SMS stack entity ; and

c)	"LTE Positioning Protocol (LPP) message container", the UE shall forward the content of the Payload container IE and the routing information included in the Additional information IE to the upper layer location services application.

### 5.4.6	5GMM status procedure

#### 5.4.6.1	General

The purpose of the 5GMM STATUS procedure is to report at any time in the 5GMM STATUS message certain error conditions detected upon receipt of 5GMM protocol data in the UE. The 5GMM STATUS message can be sent by both the AMF and the UE (see example in figure 5.4.6.1).

Figure 5.4.6.1: 5GMM status procedure

#### 5.4.6.2	5GMM status received in the UE

On receipt of a 5GMM STATUS message, no state transition and no specific action shall be taken as seen from the radio interface, i.e. local actions are possible. The local actions to be taken by UE on receipt of a 5GMM STATUS message are implementation dependent.

#### 5.4.6.3	5GMM status received in the network

On receipt of a 5GMM STATUS message in the AMF, no state transition and no specific action shall be taken as seen from the radio interface, i.e. local actions are possible. The local actions to be taken by the AMF on receipt of a 5GMM STATUS message are implementation dependent.

## 5.5	5GMM specific procedures

### 5.5.1	Registration procedure

#### 5.5.1.1	General

The registration procedure is always initiated by the UE and can be used for the following purposes:

a)	initial registration for 5GS services;

b)	mobility registration updating to update the registration of the actual tracking area of a UE in the network;

c)	mobility registration updating for 5GS services upon intersystem change from S1 mode to N1 mode;

d)	periodic registration updating to periodically notify the availability of the UE to the network;

e)	initial registration for emergency services;

f)	initial registration for SMS over NAS service, change in the requirements to use SMS over NAS service, and de-registration from SMS over NAS service.

Editor's note:	Further purposes are FFS.

#### 5.5.1.2	Registration procedure for initial registration

##### 5.5.1.2.1	General

This procedure can be used by a UE for initial registration for 5GS services.

When the UE initiates the registration procedure for initial registration, the UE shall indicate "initial registration" in the 5GS registration type IE. When the UE initiates the registration procedure for emergency services, the UE shall indicate "emergency registration" in the 5GS registration type IE.

##### 5.5.1.2.2	Initial registration initiation

The UE initiates the registration procedure for initial registration by sending a REGISTRATION REQUEST message to the AMF, starting timer T3510. If timer T3502 is currently running, the UE shall stop timer T3502. If timer T3511 is currently running, the UE shall stop timer T3511.

If the UE holds a valid 5G-GUTI, the UE shall indicate the 5G-GUTI in the 5GS mobile identity IE. If the UE does not hold a valid 5G-GUTI, the UE shall include the SUCI in the 5GS mobile identity IE. If the UE is initiating the registration procedure for emergency services and does not hold a valid 5G-GUTI, 4G-GUTI or SUCI, the PEI shall be included in the 5GS mobile identity IE.

If the UE operating in the single-registration mode holds a valid 4G-GUTI only, the UE shall include the 5G-GUTI IE mapped from the 4G-GUTI as specified in 3GPP TS 23.003 [2] in the 5GS mobile identity IE of the REGISTRATION REQUEST message. If the UE operating in the single-registration mode holds a valid 4G-GUTI and a valid 5G-GUTI, it shall include in the 5GS mobile identity IE of the REGISTRATION REQUEST message either 5G-GUTI mapped from 4G-GUTI or 5G-GUTI according to the system (i.e. either EPS or 5GS), where the last successful EPS attach, tracking area updating or registration procedure was performed.

If the UE is operating in the dual-registration mode, the UE shall include the UE status IE with the EMM registration status set to "UE is in EMM-REGISTERED state". If the UE has a valid 5G-GUTI, the UE shall include the 5G-GUTI in the REGISTRATION REQUEST message. The UE operating in the dual-registration mode shall not use 4G-GUTI even if the UE has a valid 4G-GUTI.

If the last visited registered TAI is available, the UE shall include the last visited registered TAI in the REGISTRATION REQUEST message.

If the UE requests the use of SMS over NAS, the UE shall include the SMS requested IE in the REGISTRATION REQUEST message and set:

a)	the "supported accesses" bits of the SMS requested IE to:

1)	"SMS over NAS supported via 3GPP access only" if the UE supports SMS delivery over NAS via 3GPP access only; or

2)	"SMS over NAS supported via both 3GPP access and non-3GPP access" if the UE supports SMS delivery over NAS via both 3GPP access and non-3GPP access; and

Editor's note:	A decision still needs to be made on whether "one step" functionality needs to be added.

If the UE supports MICO mode and requests the use of MICO mode, then the UE shall include the MICO indication IE in the REGISTRATION REQUEST message.

If the UE supports network slicing, the UE shall include the requested NSSAI containing the S-NSSAI(s) corresponding to the slice(s) to which the UE wishes to register and the mapping of requested NSSAI which is the mapping of each S-NSSAI of the requested NSSAI to the S-NSSAI of the configured NSSAI for the HPLMN in the REGISTRATION REQUEST. If the UE has allowed NSSAI or configured NSSAI for the current PLMN, the requested NSSAI shall be either:

a)	the configured NSSAI for the current PLMN, or a subset thereof as described below, if the UE has no allowed NSSAI for the current PLMN; or

b)	the allowed NSSAI for the current PLMN, or a subset thereof as described below, if the UE has an allowed NSSAI for the current PLMN; or

c)	the allowed NSSAI for the current PLMN, or a subset thereof as described below, plus one or more S-NSSAIs from the configured NSSAI for which no corresponding S-NSSAI is present in the allowed NSSAI and those are neither in the rejected NSSAI for the current PLMN nor in the rejected NSSAI for the current PLMN and registration area combination.

If the UE has neither allowed NSSAI for the current PLMN nor configured NSSAI for the current PLMN and has a configured NSSAI not associated with a PLMN, the UE shall include the S-NSSAI(s) in the Requested NSSAI IE of the REGISTRATION REQUEST message using the configured NSSAI not associated with a PLMN. If the UE has no allowed NSSAI for the current PLMN, no configured NSSAI for the current PLMN, and no configured NSSAI not associated with a PLMN, the UE shall not include a requested NSSAI in the REGISTRATION message.

The subset of configured NSSAI provided in the requested NSSAI consists of one or more S-NSSAIs in the configured NSSAI applicable to the current PLMN, if the S-NSSAI is neither in the rejected NSSAI for the current PLMN nor in the rejected NSSAI for the current PLMN and registration area combination.

The subset of allowed NSSAI provided in the requested NSSAI consists of one or more S-NSSAIs in the allowed NSSAI for the current PLMN.

NOTE 1:	How the UE selects the subset of configured NSSAI or allowed NSSAI to be provided in the requested NSSAI is implementation.

NOTE 2:	The number of S-NSSAI(s) included in the requested NSSAI cannot exceed eight.

If the UE wishes to prolong the established NAS signalling connection after the completion of the initial registration procedure, it shall set the "follow-on request pending" indication to 1.

If the UE supports S1 mode, the UE shall set the S1 mode bit to "S1 mode supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

Figure 5.5.1.2.2.1: Registration procedure for initial registration

##### 5.5.1.2.3	5GMM common procedure initiation

The network may initiate 5GMM common procedures, e.g. the identification, authentication and security procedures during the registration procedure, depending on the information received in the REGISTRATION REQUEST message.

During a registration procedure with 5GS registration type IE set to "emergency registration", if the AMF is configured to support emergency registration for unauthenticated SUCIs, the AMF may choose to skip the authentication procedure even if no 5GS security context is available and proceed directly to the execution of the security mode control procedure.

##### 5.5.1.2.4	Initial registration accepted by the network

During a registration procedure with 5GS registration type IE set to "emergency registration", the AMF shall not check for mobility and access restrictions, regional restrictions or subscription restrictions, when processing the REGISTRATION REQUEST message.

If the initial registration request is accepted by the network, the AMF shall send a REGISTRATION ACCEPT message to the UE and start timer T3550.

The AMF shall assign and include a TAI list as a registration area the UE is registered to in the REGISTRATION ACCEPT message. The UE, upon receiving a REGISTRATION ACCEPT message, shall delete its old TAI list and store the received TAI list.

The AMF shall include the LADN information in the LADN information IE of the REGISTRATION ACCEPT message, if there are valid LADN service area(s) for the subscribed DNN(s) of the UE in the current registration area. The UE, upon receiving the REGISTRATION ACCEPT message with the LADN information, shall delete its old LADN information (if any) and store the received LADN information.

The 5G-GUTI reallocation may be part of the initial registration procedure. When the REGISTRATION REQUEST message includes the SUCI or PEI, or the AMF considers the 5G-GUTI provided by the UE is invalid, or the 5G-GUTI provided by the UE was assigned by another AMF, the AMF shall allocate a new 5G-GUTI to the UE. The AMF shall include in the REGISTRATION ACCEPT message the new assigned 5G-GUTI together with the assigned TAI list. In this case the AMF shall shall start timer T3550 and enter state 5GMM-COMMON-PROCEDURE-INITIATED as described in subclause 5.1.3.2.3.3.

The AMF shall include the MICO indication IE in the REGISTRATION ACCEPT only if the MICO indication IE was included in the REGISTRATION REQUEST message, the AMF supports and accepts the use of MICO mode. If the AMF supports and accepts the use of MICO mode, the AMF may indicate "all PLMN registration area allocated" in the MICO indication IE in the REGISTRATION ACCEPT message. If "all PLMN registration area allocated" is indciated in the MICO indication IE, the AMF shall not assign and include the TAI list in the REGISTRATION ACCEPT message.

If the REGISTRATION ACCEPT message includeded an MICO indication IE indicating "all PLMN registration area allocated", the UE shall treat all TAIs in the current PLMN as a registration area.

If the REGISTRATION ACCEPT message contained a 5G-GUTI, the UE shall return a REGISTRATION COMPLETE message to the AMF to acknowledge the received 5G-GUTI.

Upon receiving a REGISTRATION COMPLETE message, the AMF shall stop timer T3550 and change to state 5GMM-REGISTERED. The 5G-GUTI, if sent in the REGISTRATION ACCEPT message, shall be considered as valid.

If the REGISTRATION REQUEST message contained the SMS requested IE with the supported accesses bits indicating that the UE supports SMS delivery over NAS via 3GPP access only, or via both 3GPP access and non-3GPP access, and SMSF selection is successful, then the AMF shall send the REGISTRATION ACCEPT message after the SMSF has confirmed that the activation of the SMS service was successful. When sending the REGISTRATION ACCEPT message, the AMF shall:

a)	store the SMSF address and the contents of the SMS requested IE in the UE 5GMM context, and consider the UE available for SMS; and

b)	include the SMS allowed IE in the REGISTRATION ACCEPT message with:

1)	the "allowed accesses" bits of the SMS allowed IE to:

A)	"SMS over NAS supported via 3GPP access only" if:

i)	the UE has set the "supported accesses" bits of the SMS requested IE to "SMS over NAS supported via 3GPP access only" and the network allows the SMS delivery over NAS via 3GPP access; or

ii)	the UE has set the "supported accesses" bits of the SMS requested IE to "SMS over NAS supported via both 3GPP access and non-3GPP access" and the network allows the SMS delivery over NAS via 3GPP access only; or

B)	"SMS over NAS supported via both 3GPP access and non-3GPP access" if the UE has set the "supported accesses" bits of the SMS requested IE to "SMS over NAS supported via both 3GPP access and non-3GPP access" and the network allows the SMS delivery over NAS via both 3GPP access and non-3GPP access;

Editor's note:	A decision still needs to be made on whether "one step" functionality needs to be added.

If SMSF selection in the AMF or SMS activation via the SMSF is not successful, then the AMF shall not include the SMS allowed IE in the REGISTRATION ACCEPT message. If the AMF does not allow the use of SMS over NAS, then the AMF shall not include the SMS allowed IE in the REGISTRATION ACCEPT message.

The AMF shall include the allowed NSSAI for the current PLMN and may include the mapping of the allowed NSSAI to the configured NSSAI for the HPLMN contained in the requested NSSAI from the UE if available, in the REGISTRATION ACCEPT if the UE included the requested NSSAI in the REGISTRATION REQUEST message and the AMF allows one or more S-NSSAIs in the requested NSSAI. The AMF may also include rejected NSSAI in the REGISTRATION ACCEPT message. Rejected NSSAI contains S-NSSAI(s) which was included in the Request NSSAI but rejected by the network associated with rejection cause(s).

The UE receiving the rejected NSSAI in the REGISTRATION ACCEPT message takes the following actions based on the rejection cause in the rejected NSSAI:

"S-NSSAI not available in the current PLMN"

The UE shall add the rejected S-NSSAI(s) in the rejected NSSAI for the current PLMN as specified in subclause 4.6.2.2 and not attempt to use this S-NSSAI in the current PLMN until switching off the UE or the UICC containing the USIM is removed.

"S-NSSAI not available in the current registration area"

The UE shall add the rejected S-NSSAI(s) in the rejected NSSAI for the current PLMN and registration area combination as specified in subclause 4.6.2.2 and not attempt to use this S-NSSAI in the present registration area until switching off the UE, the UE moving out of the current registration area or the UICC containing the USIM is removed.

"S-NSSAI temporarily not available in the current registration area"

The UE shall add the rejected S-NSSAI(s) in the tempararily rejected NSSAI for the current PLMN and registration area combination as specified in subclause 4.6.2.2 and consider the S-NSSAI is temporarily not available in this current registration area.

Editor's note:	Further UE action is FFS when the rejection cause is "S-NSSAI temporarily not available in the registration area".

If the UE did not include the requested NSSAI in the REGISTRATION REQUEST message and one or more subscribed S-NSSAIs (containing one or more S-NSSAIs each of which may be associated with a new S-NSSAI) marked as default are available, the AMF shall put the subscribed S-NSSAIs marked as default in the allowed NSSAI of the REGISTRAION ACCEPT message.

If the REGISTRATION ACCEPT contains the allowed NSSAI, then the UE shall store the included allowed NSSAI together with the PLMN identity of the registered PLMN and the registration area as specified in subclause 4.6.2.2.

If the UE included S1 mode supported indication in the REGISTRATION REQUEST message, the AMF supporting intersystem change with EPS shall set the IWK N26 bit to either:

a)	"interworking without N26 not supported" if the AMF does not support interworking procedures with N26 interface; or

b)	"interworking without N26 not supported" if the AMF supports interworking procedures without N26 interface.

in the 5GS network feature support IE in the REGISTRATION ACCEPT message.

The UE shall operate in the mode for intersystem change with EPS as follows:

a)	if the IWK N26 bit in the 5GS network feature support IE is set to "interworking without N26 not supported", the UE shall operate in single-registration mode;

b)	if the IWK N26 bit  in the 5GS network feature support IE is set to "interworking without N26 supported" and the UE supports dual-registration mode,, the UE shall operate in dual-registration mode;

c)	if the IWK N26 bit  in the 5GS network feature support IE is set to "interworking without N26 supported" and the UE only supports single-registration mode, the UE shall operate in single-registration mode or;

d)	if the IWK N26 bit is not received or not supported by the UE, the UE shall operate in single-registration mode.

The UE shall treat the received intworking without N26 supported indication for intersystem change with EPS as valid in the entire PLMN.

The network informs the UE about the support of specific features, such as IMS voice over PS session or emergency services, in the 5GS network feature support information element. In a UE with IMS voice over PS session capability, the IMS voice over PS session indicator, the emergency service support indicator, and the emergency service fallback indicator shall be provided to the upper layers. The upper layers take the IMS voice over PS session indicator into account when selecting the access domain for voice sessions or calls. When initiating an emergency call, the upper layers also take the IMS voice over PS session indicator, the emergency service support indicator, and the emergency service fallback indicator into account for the access domain selection.

Editor's note:	Further details on initial registration accepted by the network are FFS.

Editor's note:	The information element coding for 5GS network feature support information element is FFS.

If the UE has indicated "follow-on request pending" in REGISTRATION REQUEST message, or the network has downlink signalling pending, the AMF shall not immediately release the NAS signalling connection after the completion of the registration procedure.

##### 5.5.1.2.5	Initial registration not accepted by the network

If the initial registration request cannot be accepted by the network, the AMF shall send a REGISTRATION REJECT message to the UE including an appropriate 5GMM cause value.

If the initial registration request request is rejected due to general NAS level mobility management congestion control, the network shall set the 5GMM cause value to #22 "congestion" and assign a back-off timer T3346.

The UE shall take the following actions depending on the 5GMM cause value received in the REGISTRATION REJECT message.

#3		(Illegal UE);

#6		(Illegal ME);

#8		(5GS services not allowed); or

#22	(Congestion);

If the T3346 value IE is present in the REGISTRATION REJECT message and the value indicates that this timer is neither zero nor deactivated, the UE shall proceed as described below; otherwise it shall be considered as an abnormal case and the behaviour of the UE for this case is specified in subclause 5.5.1.2.7.

The UE shall abort the initial registration procedure, set the 5GS update status to 5U2 NOT UPDATED and enter state 5GMM-DEREGISTERED.ATTEMPTING-REGISTRATION.

The UE shall stop timer T3346 if it is running.

If the REGISTRATION REJECT message is integrity protected, the UE shall start timer T3346 with the value provided in the T3346 value IE.

If the REGISTRATION REJECT message is not integrity protected, the UE shall start timer T3346 with a random value from the default range specified in 3GPP TS 24.008 [8].

The UE stays in the current serving cell and applies the normal cell reselection process. The initial registration procedure is started if still needed when timer T3346 expires or is stopped.

#27	(N1 mode not allowed);

The UE capable of S1 mode shall disable the N1 mode radio capability (see subclause .3.1.x).

Editor's note:	Further details on initial registration not accepted by the network are FFS.

##### 5.5.1.2.6	Initial registration for emergency services not accepted by the network

Upon receiving the REGISTRATION REJECT message including 5GMM cause #5 "PEI not accepted", the UE shall enter the state 5GMM-DEREGISTERED.NO-SUPI.

Upon receiving the REGISTRATION REJECT message including 5GMM cause value which is not #5 "PEI not accepted", the UE shall perform the actions as described in subclause 5.5.1.2.5 with the following addition: the UE shall inform the upper layers of the failure of the procedure.

NOTE 1:	This can result in the upper layers requesting implementation specific mechanisms, e.g. procedures specified in 3GPP TS 24.229 can result in the emergency call being attempted to another IP-CAN.

If the initial registration request for emergency services fails due to abnormal cases, the UE shall perform the actions as described in subclause 5.5.1.2.7 and inform the upper layers of the failure to access the network or the failure of the procedure.

NOTE 2:	This can result in the upper layers requesting other implementation specific mechanisms, e.g. procedures specified in 3GPP TS 24.229 can result in the emergency call being attempted to another IP-CAN.

In a shared network, upon receiving the REGISTRATION REJECT message, the UE shall perform the actions as described in subclause 5.5.1.2.5, and shall:

a)	inform the upper layers of the failure of the procedure; or

NOTE 1:	The upper layers may request implementation specific mechanisms, e.g. procedures specified in 3GPP TS 24.229 [9] that can result in the emergency call being attempted to another IP-CAN.

b)	attempt to perform a PLMN selection in the shared network and initiate an initial registration for emergency services to the selected PLMN.

In a shared network, if the initial registration request for emergency services fails due to abnormal cases, the UE shall perform the actions as described in subclause 5.5.1.2.7 and shall:

a)	inform the upper layers of the failure of the procedure; or

NOTE 2:	The upper layers may request implementation specific mechanisms, e.g. procedures specified in 3GPP TS 24.229 [9] that can result in the emergency call being attempted to another IP-CAN.

b)	attempt the initial registration for emergency services to another PLMN in the shared network.

##### 5.5.1.2.7	Abnormal cases in the UE

The following abnormal cases can be identified:

a)	Timer T3346 is running

The UE shall not start the initial registration procedure unless:

1)	the UE is a UE configured to use AC11 – 15 in selected PLMN; or

2)	the UE needs to initial registration for emergency services

The UE stays in the current serving cell and applies the normal cell reselection process.

NOTE 1:	It is considered an abnormal case if the UE needs to initiate an initial registration procedure while timer T3346 is running independent on whether timer T3346 was started due to an abnormal case or a non successful case.

##### 5.5.1.2.8	Abnormal cases on the network side

Editor's note:	Abnormal cases in the network are FFS.

#### 5.5.1.3	Registration procedure for mobility and periodic registration update

##### 5.5.1.3.1	General

This procedure can be used by a UE for mobility and periodic registration update for 5GS services.

When the UE initiates the registration procedure for mobility and periodic registration update, the UE shall indicate "mobility registration update" or "periodic registration update" respectively in the 5GS registration type IE.

The periodic registration updating procedure is controlled in the UE by timer T3512. When timer T3512 expires, the periodic registration area updating procedure is started. Start and reset of timer T3512 is described in subclause 10.2.

##### 5.5.1.3.2	Mobility and periodic registration update initiation

The UE in state 5GMM-REGISTERED shall initiate the mobility and periodic registration updating procedure by sending a REGISTRATION REQUEST message to the AMF,

a)	when the UE detects entering a tracking area that is not in the list of tracking areas that the UE previously registered in the AMF; or

b)	when the periodic registration updating timer Te expires;

c)	when requested by the CONFIGURATION UPDATE COMMAND message;

d)	when the UE in state 5GMM-REGISTERED.ATTEMPTING-REGISTRATION-UPDATE either receives a paging with access type indicating non-3GPP or the UE receives a NOTIFICATION message over the non-3GPP access for PDU sessions associated with 3GPP; or

d) 	upon intersystem change from S1 mode to N1 mode.

Editor's note:	Further cases for initiation of mobility and periodic registration update are FFS.

The UE in state 5GMM-REGISTERED shall initiate the mobility registration updating procedure by sending a REGISTRATION REQUEST message to the AMF when the UE needs to request the use of SMS over NAS transport or the current requirements to use SMS over NAS transport change in the UE. The UE shall include the SMS requested IE in the REGISTRATION REQUEST message as specified in subclause 5.5.1.2.2.

When initiating a periodic registration update and the requirements to use SMS over NAS transport have not changed in the UE, the UE shall not include the SMS requested IE in the REGISTRATION REQUEST message.

If the UE no longer requires the use of SMS over NAS transport, then the UE shall include the SMS requested IE in the REGISTRATION REQUEST message with the "supported accesses" bits set to "SMS over NAS not supported".

After sending the REGISTRATION REQUEST message to the AMF the UE shall start timer T3510. If timer T3502 is currently running, the UE shall stop timer T3502. If timer T3511 is currently running, the UE shall stop timer T3511.

If the last visited registered TAI is available, the UE shall include the last visited registered TAI in the REGISTRATION REQUEST message.

If the UE holds a valid 5G-GUTI, the UE shall indicate the 5G-GUTI in the 5GS mobile identity IE.

If the UE supports MICO mode and requests the use of MICO mode, then the UE shall include the MICO indication IE in the REGISTRATION REQUEST message.

If the UE is initiating the mobility registration update procedure, the UE may include the Uplink data status IE to indicate which PDU session(s) have pending user data to be sent. If the UE is located outside the LADN service area, the UE shall not include the PDU session for LADN in the Uplink data status IE.

When the registration procedure is initiated in 5GMM-IDLE mode, the UE may include a PDU session status IE in the REGISTRATION REQUEST message, indicating which PDU session contexts are active in the UE.

If the UE received a paging message with the access type indicating non-3GPP, or if the UE received a NOTIFICATION message over non-3GPP access for PDU sessions associated with 3GPP, the UE shall include the Allowed PDU session status IE in the REGISTRATION REQUEST message indicating the PDU session(s) that the UE allows to re-activate over 3GPP access.

If the UE operating in the single-registration mode performs mobility from S1 mode to N1 mode, the UE:

a)	shall indicate "mobility registration update" in the 5GS registration type IE and shall include the UE status IE with the EMM registration status set to "UE is in EMM-REGISTERED state" in the REGISTRATION REQUEST message. In this case, if the UE holds a valid 4G-GUTI, the UE shall include the 5G-GUTI IE mapped from the 4G-GUTI as specified in 3GPP TS 23.003 [2] in the 5GS mobility identity IE of the REGISTRATION REQUEST message If network slicing is supported and the UE has at least one PDN connection with active EPS bearer context(s) for which interworking to 5GS is supported as specified in subclause 6.1.4, the UE shall:

a)	include the S-NSSAI(s) associated with the established PDN connection(s) for which interworking to 5GS is supported in the Requested NSSAI IE of the REGISTRATION REQUEST message; and

b)	include the PDU session ID(s) and the S-NSSAI(s), both associated with the established PDN connection(s) for which interworking to 5GS is supported, in the NSSAI info for PDU sessions IE of the REGISTRATION REQUEST message.

b)	may include the PDU session status IE in the REGISTRATION REQUEST message indicating the status of the PDU session(s) mapped during the intersystem change from S1 mode to N1 mode from the PDN connection(s) for which the EPS indicated that interworking to 5GS is supported, if any (see subclause 6.1.4) ; and

c)	shall include a TRACKING AREA UPDATE REQUEST message as specified in 3GPP TS 24.301 [10] in the EPS NAS message container IE in the REGISTRATION REQUEST message.

If the UE is operating in the dual-registration mode and the UE has a valid 5G-GUTI, the UE shall include the 5G-GUTI in the REGISTRATION REQUEST message. The UE operating in the dual-registration mode shall not use 4G-GUTI even if the UE has a valid 4G-GUTI.

If the UE supports network slicing, the UE shall include the requested NSSAI containing the S-NSSAIs corresponding to the slices to which the UE wishes to register in the REGISTRATION REQUEST message. If the UE has allowed NSSAI or configured NSSAI for the current PLMN, the requested NSSAI shall be either:

a)	the configured NSSAI for the current PLMN, or a subset thereof as described below, if the UE has no allowed NSSAI for the current PLMN; or

b)	the allowed NSSAI for the current PLMN, or a subset thereof as described below, if the UE has an allowed NSSAI for the current PLMN; or

c)	the allowed NSSAI for the current PLMN, or a subset thereof as described below, plus one or more S-NSSAIs from the configured NSSAI for which no corresponding S-NSSAI is present in the allowed NSSAI and those are neither in the rejected NSSAI for the current PLMN nor in the rejected NSSAI for the current PLMN and registration area combination.

If the UE has neither allowed NSSAI for the current PLMN nor configured NSSAI for the current PLMN and has a configured NSSAI not associated with a PLMN, the UE shall include the S-NSSAI(s) in the Requested NSSAI IE of the REGISTRATION REQUEST message using the configured NSSAI not associated with a PLMN. If the UE has no allowed NSSAI for the current PLMN, no configured NSSAI for the current PLMN, and no configured NSSAI not associated with a PLMN, the UE shall not include a requested NSSAI in the REGISTRATION message.

The subset of configured NSSAI provided in the requested NSSAI consists of one or more S-NSSAIs in the configured NSSAI applicable to this PLMN, if the S-NSSAI is neither in the rejected NSSAIs for the current PLMN nor in the rejected NSSAI for the current PLMN and registration area combination.

The subset of allowed NSSAI provided in the requested NSSAI consists of one or more S-NSSAIs in the allowed NSSAI for this PLMN.

NOTE 1:	How the UE selects the subset of configured NSSAI or allowed NSSAI to be provided in the requested NSSAI is implementation.

NOTE 2:	The number of S-NSSAI(s) included in the requested NSSAI cannot exceed eight.

In roaming scenarios, the UE may also include in the REGISTRATION REQUEST the mapping of each S-NSSAI of the requested NSSAI for the serving PLMN to the S-NSSAIs of the Configured NSSAI for the HPLMN, if available.

If the UE wishes to prolong the established NAS signalling connection after the completion of the mobility and periodic registration updating registration procedure, it shall set the "follow-on request pending" indication to 1.

Figure 5.5.1.3.2.1: Registration procedure for mobility and periodic registration update

##### 5.5.1.3.3	5GMM common procedure initiation

The network may initiate 5GMM common procedures, e.g. the identification, authentication and security procedures during the registration procedure, depending on the information received in the REGISTRATION REQUEST message.

##### 5.5.1.3.4	Mobility and periodic registration update accepted by the network

If the registration update request has been accepted by the network, the AMF shall send a REGISTRATION ACCEPT message to the UE. If the AMF assigns a new 5G-GUTI for the UE, a 5G-GUTI shall be included in the REGISTRATION ACCEPT message. In this case, the AMF shall start timer T3550.

If timer T3513 is running in the AMF, the AMF shall stop timer T3513 if a paging request was sent with the access type indicating non-3GPP and the REGISTRATION REQUEST message includes the Allowed PDU session status IE.

If timer T3565 is running in the AMF, the AMF shall stop timer T3565 when a REGISTRATION REQUEST message is received.

The AMF may include a new TAI list for the UE in the REGISTRATION ACCEPT message. The UE, upon receiving a REGISTRATION ACCEPT message, shall delete its old TAI list and store the received TAI list. If there is no TAI list received, the UE shall consider the old TAI list as valid.

If the AMF assigns a new 5G-GUTI for the UE, a 5G-GUTI shall be included in the REGISTRATION ACCEPT message. In this case the AMF shall start timer T3550 and enter state 5GMM-COMMON-PROCEDURE-INITIATED as described in subclause 5.1.3.2.3.3.

The AMF shall include the MICO indication IE in the REGISTRATION ACCEPT only if the MICO indication IE was included in the REGISTRATION REQUEST message, the AMF supports and accepts the use of MICO mode. If the AMF supports and accepts the use of MICO mode, the AMF may indicate "all PLMN registration area allocated" in the MICO indication IE in the REGISTRATION ACCEPT message. If "all PLMN registration area allocated" is indciated in the MICO indication IE, the AMF shall not assign and include the TAI list in the REGISTRATION ACCEPT message.

If the REGISTRATION ACCEPT message includeded an MICO indication IE indicating "all PLMN registration area allocated", the UE shall treat all TAIs in the current PLMN as a registration area.

If the REGISTRATION ACCEPT message contained a 5G-GUTI, the UE shall return a REGISTRATION COMPLETE message to the AMF to acknowledge the received 5G-GUTI.

Upon receiving a REGISTRATION COMPLETE message, the AMF shall stop timer T3550 and change to state 5GMM-REGISTERED. The 5G-GUTI, if sent in the REGISTRATION ACCEPT message, shall be considered as valid.

If the REGISTRATION REQUEST message contained the SMS requested IE with the supported accesses bits indicating that the UE supports SMS delivery over NAS via 3GPP access only, or via both 3GPP access and non-3GPP access and an SMSF address is not stored in the 5GMM context, then the AMF shall send the REGISTRATION ACCEPT message after the selected SMSF has confirmed that the activation of the SMS service was successful.

If the REGISTRATION REQUEST message contained the SMS requested IE with the supported accesses bits indicating that the UE supports SMS delivery over NAS via 3GPP access only, or via both 3GPP access and non-3GPP access then the AMF shall:

a)	store the SMSF address in the UE 5GMM context if not stored already;

b)	store the contents of the SMS requested IE in the UE 5GMM context;

c)	consider the UE available for SMS; and

d)	include the SMS allowed IE in the REGISTRATION ACCEPT message as specified in subclause 5.5.1.2.4.

If SMSF selection in the AMF or SMS activation via the SMSF is not successful, then the AMF shall not include the SMS allowed IE in the REGISTRATION ACCEPT message. If the AMF does not allow the use of SMS over NAS, then the AMF shall not include the SMS allowed IE in the REGISTRATION ACCEPT message.

If the REGISTRATION REQUEST message contained the SMS requested IE with the "supported accesses" bits set to "SMS over NAS not supported", then the AMF shall:

a)	mark the 5GMM context to indicate that SMS over NAS is not required by the UE; and

NOTE:	The AMF can notify the SMSF that the UE is deregistered from SMS over NAS based on local configuration.

b)	not include the SMS allowed IE in the REGISTRATION ACCEPT message.

The AMF shall include the allowed NSSAI for the current PLMN and may include the mapping of the allowed NSSAI to the configured NSSAI for the HPLMN contained in the requested NSSAI from the UE if available, in the REGISTRATION ACCEPT if the UE included the requested NSSAI in the REGISTRATION REQUEST message and the AMF allows one or more S-NSSAIs in the requested NSSAI. The AMF may also include rejected NSSAI in the REGISTRATION ACCEPT message. Rejected NSSAI contains S-NSSAI(s) which was included in the Request NSSAI but rejected by the network associated with rejection cause(s).

The UE receiving the rejected NSSAI in the REGISTRATION ACCEPT message takes the following actions based on the rejection cause in the rejected NSSAI:

"S-NSSAI not available in the current PLMN"

The UE shall add the rejected S-NSSAI(s) in the rejected NSSAI for the current PLMN as specified in subclause 4.6.2.2 and not attempt to use this S-NSSAI in the current PLMN until switching off the UE or the UICC containing the USIM is removed.

"S-NSSAI not available in the current registration area"

The UE shall add the rejected S-NSSAI(s) in the rejected NSSAI for the current PLMN and registration area combination as specified in subclause 4.6.2.2 and not attempt to use this S-NSSAI in the current registration area until switching off the UE, the UE moving out of the current registration area or the UICC containing the USIM is removed.

If the UE did not include the requested NSSAI in the REGISTRATION REQUEST message and one or more subscribed S-NSSAIs marked as default are available, the AMF shall put the subscribed S-NSSAIs marked as default in the allowed NSSAI of the REGISTRATION ACCEPT message.

If the REGISTRATION ACCEPT contains the allowed NSSAI, then the UE shall store the included allowed NSSAI together with the PLMN identity of the registered PLMN and the registration area as specified in subclause 4.6.2.2. If the UE has one or more PDU session contexts associated with S-NSSAI(s) not included in the received allowed NSSAI, the UE shall locally release all such PDU session context(s).

If the Uplink data status IE is included in the REGISTRATION REQUEST message, the AMF shall:

a)	indicate the SMF to re-activate the user-plane resources for the corresponding PDU session; and

b)	include PDU session reactivation result IE in the REGISTRATION ACCEPT message to indicate the user-plane resources reactivation result of the PDU sessions the UE requested to re-activate.

If the Uplink data status IE is not included in the REGISTRATION REQUEST message, the AMF may indicate the SMF to re-activate the user-plane resources for the PDU sessions.

If a PDU session status IE is included in the REGISTRATION REQUEST message, the AMF shall:

a)	release all those PDU session locally (without peer-to-peer signalling between the network and the UE) which are in 5GSM state PDU SESSION ACTIVE on the AMF side, but are indicated by the UE as being in 5GSM state PDU SESSION INACTIVE; and

b)	include a PDU session status IE in the REGISTRATION ACCEPT message to indicate which PDU sessions are active in the AMF.

If the Allowed PDU session status IE is included in the REGISTRATION REQUEST message, the AMF shall:

a)	indicate the SMF to re-activate the user plane for the corresponding PDU session contexts allowed to be re-activated over 3GPP access and have indicated pending downlink data; and

b)	notify the SMF that have indicated pending downlink data, that reactivation of the corresponding PDU Session contexts user plane cannot be performed if not allowed to be re-activated over 3GPP access; and

c)	include the PDU session reactivation result IE in the REGISTRATION ACCEPT message to indicate the successfully reactivated PDU session contexts.

If the AMF needs to initiate PDU session status synchronization the AMF shall include a PDU session status IE in the REGISTRATION ACCEPT message to indicate the UE which PDU sessions are active in the AMF.

The AMF may include the LADN information in the REGISTRATION ACCEPT message as described in subclause 5.5.1.2.4. The UE, upon receiving the REGISTRATION ACCEPT message with the LADN information, shall delete its old LADN information (if any) and store the received new LADN information.

If the AMF does not include the LADN information in the REGISTATION ACCEPT message during mobility registration update procedure, the UE shall delete its old LADN information.

If the PDU session status IE is included in the REGISTRATION ACCEPT message, the UE shall release all those PDU sessions locally (without peer-to-peer signalling between the network and the UE) which are in 5GSM state PDU SESSION ACTIVE on the UE side, but are indicated by the AMF as being in 5GSM state PDU SESSION INACTIVE.

The network informs the UE about the support of specific features, such as IMS voice over PS session, in the 5GS network feature support information element. In a UE with IMS voice over PS session capability, the IMS voice over PS session indicator shall be provided to the upper layers. The upper layers take the IMS voice over PS session indicator into account when selecting the access domain for voice sessions or calls.

Editor's note:	Further details on mobility and periodic registration update accepted by the network are FFS.

If the UE has indicated "follow-on request pending" in REGISTRATION REQUEST message, or the network has downlink signalling pending, the AMF shall not immediately release the NAS signalling connection after the completion of the registration procedure.

If the UE included in the REGISTRATION REQUEST message the UE status information IE with the EMM registration status set to "UE in EMM-REGISTERED state" and the AMF does not support N26 interface, the AMF shall operate as described in subclause 5.5.1.2.4.

If due to regional subscription restrictions or access restrictions the UE is not allowed to access the TA, but the UE has a PDU session for emergency services established, the AMF may accept the REGISTRATION REQUEST message and indicate to the SMF to release all non-emergency PDU sessions when the registration procedure is initiated in 5GMM-CONNECTED mode. When the registration procedure is initiated in 5GMM-IDLE mode, the AMF indicates to the SMF to release all non-emergency PDU sessions and informs the UE via the PDU session status IE in the REGISTRATION ACCEPT message. The AMF shall not indicate to the SMF to release the emergency PDU session. The network shall consider the UE to be registered for emergency service.

##### 5.5.1.3.5	Mobility and periodic registration update not accepted by the network

If the mobility and periodic registration update request cannot be accepted by the network, the AMF shall send a REGISTRATION REJECT message to the UE including an appropriate 5GMM cause value.

The UE shall take the following actions depending on the 5GMM cause value received in the REGISTRATION REJECT message.

#xx	(N1 mode not allowed);

The UE capable of S1 mode shall disable the N1 mode radio capability (see subclause 4.9).

#3		(Illegal UE);

#6		(Illegal ME); or

#8		(5GS services not allowed) or

#22	(Congestion);

If the T3346 value IE is present in the REGISTRATION REJECT message and the value indicates that this timer is neither zero nor deactivated, the UE shall proceed as described below, otherwise it shall be considered as an abnormal case and the behaviour of the UE for this case is specified in subclause 5.5.1.3.6.

The UE shall abort the mobility and periodic registration update procedure. If the rejected request was not for initiating a PDU session for emergency services, the UE shall set the 5GS update status to 5U2 NOT UPDATED and change to state 5GMM-REGISTERED.ATTEMPTING-REGISTRATION-UPDATE.

The UE shall stop timer T3346 if it is running.

If the REGISTRATION REJECT message is integrity protected, the UE shall start timer T3346 with the value provided in the T3346 value IE.

If the REGISTRATION REJECT message is not integrity protected, the UE shall start timer T3346 with a random value from the default range specified in 3GPP TS 24.008 [8].

The UE stays in the current serving cell and applies the normal cell reselection process. The mobility and periodic registration update procedure is started, if still necessary, when timer T3346 expires or is stopped.

Editor's note:	Further details on mobility and periodic registration update not accepted by the network are FFS.

##### 5.5.1.3.6	Abnormal cases in the UE

The following abnormal cases can be identified:

a)	Timer T3346 is running

The UE shall not start the mobility and periodic registration update procedure unless:

1)	the UE is in 5GMM-CONNECTED mode;

2)	the UE received a paging;

3)	the UE receives a NOTIFICATION message over non-3GPP access when the UE is in 5GMM-CONNECTED mode over non-3GPP access and in 5GMM-IDLE mode over 3GPP access;

4)	the UE is a UE configured to use AC11 – 15 in selected PLMN; or

5)	the UE has a PDU session for emergency services established or is establishing a PDU session for emergency services

The UE stays in the current serving cell and applies the normal cell reselection process.

NOTE 1:	It is considered an abnormal case if the UE needs to initiate a mobility and periodic registration update procedure while timer T3346 is running independent on whether timer T3346 was started due to an abnormal case or a non successful case.

##### 5.5.1.3.7	Abnormal cases on the network side

Editor's note:	Abnormal cases in the network are FFS.

### 5.5.2	De-registration procedure

#### 5.5.2.1	General

The de-registration procedure is used:

a)	by the UE to de-register for 5GS services over 3GPP access when the UE is registered over 3GPP access;;

b)	by the UE to de-register for 5GS services over 3GPP access, non-3GPP access, or both when the UE is registered in the same PLMN over both accesses;

c)	by the network to inform the UE that it is deregistered for 5GS services over 3GPP access when the UE is registered over 3GPP access;

d)	by the network to inform the UE that it is deregistered for 5GS services over 3GPP access, non-3GPP access, or both when the UE is registered in the same PLMN over both accesses; and

e)	by the network to inform the UE to re-register to the network.

The de-registration procedure with appropriate de-registration type shall be invoked by the UE:

-	if the UE is switched off.

The de-registration procedure with appropriate de-registration type shall be invoked by the network:

-	if the network informs whether the UE should re-register to the network.

The de-registration procedure with appropriate access type shall be invoked by the UE:

a)	if the UE wishes to de-register for 5GS services over 3GPP access when the UE is registered over 3GPP access; or

b)	the UE wishes to de-register for 5GS services over 3GPP access, non-3GPP access, or both when the UE is registered in the same PLMN over both accesses.

If the de-registration procedure is triggered due to USIM removal, the UE shall indicate "switch off" in the de-registration type IE.

If the de-registration procedure is requested by the UDM for a UE that has PDU sessions for emergency services, the AMF shall not send a DEREGISTRATION REQUEST message to the UE.

If the de-registration procedure for 5GS services is performed, the PDU sessions, if any, for this particular UE are released locally without peer-to-peer signalling between the UE and the network.

The UE is allowed to initiate the de-registration procedure even if the timer T3346 is running.

NOTE:	When the UE has no PDU sessions over non-3GPP access, or the UE moves all the PDU sessions over a non-3GPP access to a 3GPP access, the UE and the AMF need not initiate de-registration over the non-3GPP access.

The AMF shall provide the UE with a non-3GPP de-registration timer.

Once the UE enters the 5GMM-IDLE mode over non-3GPP access, the UE starts a non-3GPP de-registraion timer. When the timer expires, the UE shall perform de-registration procedure for non-3GPP access and enter the state 5GMM-DEREGISTERED over non-3GPP access.

#### 5.5.2.2	UE-initiated de-registration procedure

##### 5.5.2.2.1	UE-initiated de-registration procedure initiation

The de-registration procedure is initiated by the UE by sending a DEREGISTRATION REQUEST message (see example in figure 5.5.2.2.1). The De-registration type IE included in the message indicates whether the de-registration procedure is due to a "switch off" or not. The access type included in the message indicates whether the de-registration procedure is:

a)	for 5GS services over 3GPP access when the UE is registered over 3GPP access, or

b)	for 5GS services over 3GPP access, non-3GPP access or both 3GPP access and non-3GPP access when the UE is registered in the same PLMN over both accesses.

If the UE has a valid 5G-GUTI, the UE shall populate the Mobile identity IE with the valid 5G-GUTI. If the UE does not have a valid 5G-GUTI, the UE shall populate the Mobile identity IE with its SUPI.

If the UE does not have a valid 5G-GUTI and it does not have a valid SUCI, then the UE shall populate the Mobile identity IE with its PEI.

If the de-registration request is not due to switch off and the UE is in the state 5GMM-REGISTERED or 5GMM-REGISTERED-INITIATED, timer Txa shall be started in the UE after the DEREGISTRATION REQUEST message has been sent. The UE shall enter the state 5GMM-DEREGISTERED-INITIATED.

If the UE is to be switched off, the UE shall try for a period of 5 seconds to send the DEREGISTRATION REQUEST message. During this period, the UE may be switched off as soon as the DEREGISTRATION REQUEST message has been sent.

Figure 5.5.2.2.1.1: UE-initiated de-registration procedure

##### 5.5.2.2.2	UE-initiated de-registration procedure completion for 5GS services over 3GPP access

When the DEREGISTRATION REQUEST message is received by the AMF, the AMF shall send a DEREGISTRATION ACCEPT message to the UE, if the De-registration type IE does not indicate "switch off". Otherwise, the procedure is completed when the AMF receives the DEREGISTRATION REQUEST message.

If the access type in the DEREGISTRATION REQUEST message indicates that the de-registration procedure is for 3GPP access, the AMF shall trigger the SMF to release the PDU session context(s) established over 3GPP access, if any, locally for this UE without peer-to-peer signalling between the UE and the SMF. The UE shall release the PDU session context(s) established over 3GPP access, if any, locally without peer-to-peer signalling between the UE and the SMF. The UE is marked as inactive in the AMF for 5GS services for 3GPP access. The AMF shall enter the state 5GMM-DEREGISTERED for 3GPP access.

The UE, when receiving the DEREGISTRATION ACCEPT message, shall stop timer Txa. Furthermore, if the UE supports N1 mode only then:

a)	if the de-registration procedure was performed due to disabling of 5GS services, then the UE shall enter the 5GMM-NULL state for 3GPP access;

b)	otherwise, the UE shall enter the 5GMM-DEREGISTERED state for 3GPP access.

##### 5.5.2.2.3	UE-initiated de-registration procedure completion for 5GS services over both 3GPP access and non-3GPP access

The description for UE-initiated de-registration procedure completion for 5GS services over 3GPP access as specified in subclause 5.5.2.2.2 shall be followed. In addition, the following description for de-registration over non-3GPP access applies.

If the access type in the DEREGISTRATION REQUEST indicates that the de-registration procedure is for both 3GPP access and non-3GPP access when the UE is registered in the same PLMN over both accesses, the AMF shall trigger SMF to release the PDU session context(s) established over non-3GPP access, if any, locally for this UE without peer-to-peer signalling between the UE and the SMF. The UE shall release the PDU session context(s) established over non-3GPP access, if any, locally without peer-to-peer signalling between the UE and the SMF. The UE is marked as inactive in the AMF for 5GS services for non-3GPP access. The AMF shall enter the state 5GMM-DEREGISTERED over non-3GPP access.

If the UE supports N1 mode only then:

a)	if the de-registration procedure was performed due to disabling of 5GS services, then the UE shall enter the 5GMM-NULL state for non-3GPP access;

b)	otherwise, the UE shall enter the 5GMM-DEREGISTERED state for non-3GPP access.

##### 5.5.2.2.4	Abnormal cases in the UE

Editor's note:	Abnormal cases in the UE are FFS.

##### 5.5.2.2.5	Abnormal cases in the network side

Editor's note:	Abnormal cases in the network are FFS.

#### 5.5.2.3	Network-initiated de-registration procedure

##### 5.5.2.3.1	Network-initiated de-registration procedure initiation

The network initiates the de-registration procedure by sending a DEREGISTRATION REQUEST message to the UE (see example in figure 5.5.2.3.1.1).

NOTE:	If the AMF performs a local de-registration, it will inform the UE with a 5GMM messages (e.g. SERVICE REJECT message or REGISTRATION REJECT message) with 5GMM cause #10 "implicitly de-registered" only when the UE initiates a 5GMM procedure.

The network may include a 5GMM cause IE to specify the reason for the DEREGISTRATION REQUEST message. The network shall start timer T3522. The network shall indicate whether re-registration is needed or not in the De-registration type IE. The network shall also indicate via the access type whether the de-registration procedure is for 3GPP access, or for both 3GPP access and non-3GPP access when the UE is registered in the same PLMN for both accesses. The network shall release the PDU session context(s) over the indicated access(es), if any, for the UE locally and enter state 5GMM-DEREGISTERED-INITIATED.

Figure 5.5.2.3.1.1: Network-initiated de-registration procedure

##### 5.5.2.3.2	Network-initiated de-registration procedure completion by the UE

Upon receiving the DEREGISTRATION REQUEST message, and the DEREGISTRATION REQUEST message indicates "re-registration required" and the de-registration request is for 3GPP access, the UE shall release the PDU sessions over 3GPP access, if any, locally without peer-to-peer signalling between the UE and the network. The UE shall stop the timer(s) T3346, if it is running, T35ab, if it is running, T35cd, if it is running.The UE shall ignore the 5GMM cause IE if received. The UE shall send a DEREGISTRATION ACCEPT message to the network and enter the state 5GMM-DEREGISTERED for 3GPP access. The UE shall delete the trejected NSSAI for the current PLMN and the rejected NSSAI for the current PLMN and registration area combination. Furthermore, the UE shall, after the completion of the de-registration procedure, and the release of the existing NAS signalling connection, initiate an initial registration. The UE should also re-establish any previously established PDU sessions.

Editor's note:	It is FFS to use which timer for T35ab.

Editor's note:	It is FFS to use which timer for T35cd.

Upon receiving the DEREGISTRATION REQUEST message and the DEREGISTRATION REQUEST message indicates "re-registration required" and the de-registration request is for both 3GPP access and non-3GPP access when the UE is registered in the same PLMN for both accesses, the UE shall release the PDU sessions over both 3GPP access and non-3GPP access, if any, locally without peer-to-peer signalling between the UE and the network. The UE shall stop the timer(s) T3346, if it is running, T35ab, if it is running, T35cd, if it is running.The UE shall ignore the 5GMM cause IE if received. The UE shall send a DEREGISTRATION ACCEPT message to the network and enter the state 5GMM-DEREGISTERED for both 3GPP access and non-3GPP access. Furthermore, the UE shall, after the completion of the de-registration procedure, and the release of the existing NAS signalling connection, initiate an initial registration. The UE should also re-establish any previously established PDU sessions.

NOTE 1:	When the de-registration type indicates "re-registration required", user interaction is necessary in some cases when the UE cannot re-establish the PDU session (s), if any, automatically.

Upon receiving the DEREGISTRATION REQUEST message and the DEREGISTRATION REQUEST message indicates "re-registration not required" and the de-registration request is for 3GPP access, the UE shall release the PDU sessions over 3GPP access, if any, locally without peer-to-peer signalling between the UE and the network. The UE shall send a DEREGISTRATION ACCEPT message to the network and enter the state 5GMM-DEREGISTERED for 3GPP access.

Upon receiving the DEREGISTRATION REQUEST message and the DEREGISTRATION REQUEST message indicates "re-registration not required" and the de-registration request is for both 3GPP access and non-3GPP access when the UE is registered in the same PLMN for both accesses, the UE shall release the PDU sessions over both 3GPP access and non-3GPP access, if any, locally without peer-to-peer signalling between the UE and the network. The UE shall send a DEREGISTRATION ACCEPT message to the network and enter the state 5GMM-DEREGISTERED for both 3GPP access and non-3GPP access.

If the de-registration type indicates "re-registration required", then the UE shall ignore the 5GMM cause IE if received.

If the de-registration type indicates "re-registration not required", the UE shall take the actions depending on the received 5GMM cause value:

#xx		(N1 mode not allowed);

The UE capable of S1 mode shall disable the N1 mode radio capability (see subclause 4.9).

##### 5.5.2.3.3	Network-initiated de-registration procedure completion by the network

The network shall stop timer T3522 upon receipt of the DEREGISTRATION ACCEPT message. The network shall enter state 5GMM-DEREGISTERED for 3GPP access if the de-registration request is for 3GPP access. The network shall enter state 5GMM-DEREGISTERED for both 3GPP access and non-3GPP access if the de-registration request is for both 3GPP access and non-3GPP access.

##### 5.5.2.3.4	Abnormal cases in the UE

Editor's note:	Abnormal cases in the UE are FFS.

##### 5.5.2.3.5	Abnormal cases in the network side

Editor's note:	Abnormal cases in the network are FFS.

## 5.6	5GMM connection management procedures

### 5.6.1	Service request procedure

#### 5.6.1.1	General

The purpose of the service request procedure is to change the 5GMM mode from 5GMM-IDLE to 5GMM-CONNECTED mode, and/or to request establishment of user-plane radio resources for PDU sessions which are activated without radio resources. In latter case, the 5GMM mode can be the 5GMM-IDLE mode or the 5GMM-CONNECTED mode over 3GPP access if the UE requires radio resources. This procedure is used when:

a)	the network has downlink signalling pending over 3GPP access and the UE is in 5GMM-IDLE mode over 3GPP access;

b)	the network has downlink signalling pending over non-3GPP access, the UE is in 5GMM-IDLE  mode over non-3GPP access and in 5GMM-IDLE or 5GMM-CONNECTED mode over 3GPP access;

c)	the UE has uplink signalling pending over 3GPP access and the UE is in 5GMM-IDLE mode over 3GPP access;

d)	the network has downlink user data pending over 3GPP access and the UE is in 5GMM-IDLE mode over 3GPP access;

e)	the network has downlink user data pending over non-3GPP access, the UE is in 5GMM-IDLE mode over non-3GPP access and in 5GMM-IDLE or 5GMM-CONNECTED mode over 3GPP access;

f)	the UE has user data pending over 3GPP access and the UE is in 5GMM-IDLE or 5GMM-CONNECTED mode over 3GPP access; or

g)	the UE in 5GMM-IDLE mode over non-3GPP access, receives an indication from the lower layers of non-3GPP access, that the access stratum connection is established between UE and network; or

h)	the UE in 5GMM-IDLE or 5GMM-CONNECTED mode over 3GPP access receives a request for emergency service from the upper layer and performs emergency services fallback as specified in subclause 4.13.4.2 of 3GPP TS 23.502 [5].

This procedure shall not be used for initiating user data transfer or PDU session related signalling related to a PDU session for LADN when the UE is located outside the LADN service area.

The service request procedure is initiated by the UE, however, it can be triggered by the network by means of:

a)	the paging procedure (see subclause 5.6.2) for the transfer of downlink signalling or user data pending over 3GPP access to a UE in 5GMM-IDLE mode over 3GPP access;

b)	the paging procedure (see subclause 5.6.2) for the transfer of downlink signalling or user data pending over non-3GPP access to a UE in 5GMM-IDLE mode over 3GPP access and in 5GMM-IDLE mode over non-3GPP access;

c)	the notification procedure (see subclause 5.6.3) for the transfer of downlink signalling or user data pending over non-3GPP access to a UE in 5GMM-CONNECTED mode over3GPP access and in 5GMM-IDLE mode over non-3GPP access; or

d)	the notification procedure (see subclause 5.6.3) for the transfer of downlink signalling or user data pending over 3GPP access to a UE in 5GMM-IDLE mode over 3GPP access and in 5GMM-CONNECTED mode over non-3GPP access.

NOTE:	In case the UE is in 5GMM-IDLE mode over 3GPP access and in 5GMM-CONNECTED mode over non-3GPP access and downlink signalling or user data pending over 3GPP access needs to be transferred, the AMF can trigger either the notification procedure or the paging procedure based on implementation.

The UE shall invoke the service request procedure when:

a)	the UE, in 5GMM-IDLE mode over 3GPP access, receives a paging request from the network;

b)	the UE, in 5GMM-CONNECTED mode over 3GPP access, receives a notification from the network;

c)	the UE, in 5GMM-IDLE mode over 3GPP access, has uplink signalling pending;

d)	the UE, in 5GMM-IDLE mode over 3GPP access, has uplink user data pending;

e)	the UE, in 5GMM-CONNECTED mode over 3GPP access, has user data pending due to no user-plane radio resources established for PDU session(s) used for user data transport;

f)	the UE in 5GMM-IDLE mode over non-3GPP access, receives an indication from the lower layers of non-3GPP access, that the access stratum connection is established between UE and network;

g)	the UE, in 5GMM-IDLE mode over 3GPP access, receives a notification from the network when the UE is in 5GMM-CONNECTED mode over non-3GPP access; or

h)	the UE in 5GMM-IDLE or 5GMM-CONNECTED mode over 3GPP access receives a request for emergency service from the upper layer and performs emergency services fallback as specified in subclause 4.13.4.2 of 3GPP TS 23.502 [5].

Figure 5.6.1.1.1: Service Request procedure

#### 5.6.1.2	Service request procedure initiation

The UE initiates the service request procedure by sending a SERVICE REQUEST message to the AMF and starts timer T3517.

For case a in subclause 5.6.1.1:

-	if the paging request includes an indication for non-3GPP access type, the Allowed PDU session status IE shall be included in the SERVICE REQUEST message to indicate the user-plane resources of PDU session(s) that the UE allows to be re-activate over 3GPP access;

-	if the UE has uplink user data pending to be sent, the Uplink data status IE shall be included in the SERVICE REQUEST message to indicate the PDU session(s) the UE has pending user data to be sent; or

-	otherwise, the Uplink data status IE shall not be included in the SERVICE REQUEST message.

For case b in subclause 5.6.1.1, the Allowed PDU session status IE shall be included in the SERVICE REQUEST message to indicate the user-plane resources of PDU session(s) that the UE allows to re-activate over 3GPP access.

For case c and f in subclause 5.6.1.1, the Uplink data status IE shall not be included in the SERVICE REQUEST message.

For cases d and e in subclause 5.6.1.1, the Uplink data status IE shall be included in the SERVICE REQUEST message to indicate the PDU session(s) the UE has pending user data to be sent.

For case g in subclause 5.6.1.1, if the UE has uplink user data pending to be sent, the Uplink data status IE shall be included in the SERVICE REQUEST message to indicate the PDU session(s) the UE has pending user data to be sent.

For case h in subclause 5.6.1.1, if the UE receives a request for emergency service from the upper layer and performs emergency services fallback as specified in subclause 4.13.4.2 of 3GPP TS 23.502 [5], the UE shall send a SERVICE REQUEST message with service type set to "emergency services fallback".

The PDU session status information element may be included in the SERVICE REQUEST message to indicate the PDU session(s) available in the UE. If the PDU session status information element is included in the SERVICE REQUEST message, then the AMF shall release all those PDU session contexts locally (without peer-to-peer signalling between the UE and the network) which are in active on the AMF side, but are indicated by the UE as being inactive.

#### 5.6.1.3	Common procedure initiation

Upon receipt of the SERVICE REQUEST message, the AMF may initiate the common procedures e.g. the 5G AKA based primary authentication and key agreement procedure or the EAP based primary authentication and key agreement procedure.

#### 5.6.1.4	Service request procedure accepted by the network

For cases a, b, c, d, e and g in subclause 5.6.1.1, the UE shall treat the reception of the SERVICE ACCEPT message as successful completion of the procedure and stop timer T3517 and enter the state 5GMM-REGISTERED.

Editor's note:	For case h in subclause 5.6.1.1, whether the indication from the lower layers when the UE has been handed over or redirected to E-UTRAN needs to be considered as a successful completion of the procedure is FFS.

If the AMF needs to initiate PDU session status synchronization or a PDU session status IE was included in the SERVICE REQUEST message, the AMF shall include a PDU session status IE in the SERVICE ACCEPT message to indicate which PDU sessions are active in the AMF. If the PDU session status information element is included in the SERVICE ACCEPT message, then the UE shall release all those PDU sessions locally (without peer-to-peer signalling between the network and the UE) which are in active on the UE side, but are indicated by the AMF as being inactive.

The AMF may include the rejection cause in the SERVICE ACCEPT message if the UE initiated the service request procedure for a PDU Session corresponding to a LADN when the UE is located outside the LADN service area. The rejection cause indicates the PDU session is not activated due to LADN not available in the PDU session reactivation result IE.

If the Uplink data status IE is included in the SERVICE REQUEST message, the AMF shall:

a)	indicate the SMF to re-activate the user-plane resources for the corresponding PDU session contexts; and

b)	include the PDU session reactivation result IE in the SERVICE ACCEPT message to indicate the user plane reactivation result of the PDU sessions the UE requested to re-activate.

If the Allowed PDU session status IE is included in the SERVICE REQUEST message, the AMF shall:

a)	indicate the SMF to re-activate the user-plane resources for the corresponding PDU session contexts allowed to be re-activated over 3GPP access and have indicated pending downlink data; and

b)	notify the SMF that have indicated pending downlink data, that reactivation of the user-plane resources for the corresponding PDU Session contexts cannot be performed if not allowed to be re-activated over 3GPP access; and

c)	include the PDU session reactivation result IE in the SERVICE ACCEPT message to indicate the successfully reactivated PDU session contexts.

If the SERVICE REQUEST message is for emergency services fallback, the AMF triggers the emergency services fallback procedure as specified in subclause 4.13.4.2 of 3GPP TS 23.502 [5].

#### 5.6.1.5	Service request procedure not accepted by the network

If the service request cannot be accepted, the network shall return a SERVICE REJECT message to the UE including an appropriate 5GMM cause value and stop timer T3517.

If the AMF needs to initiate PDU session status synchronisation or a PDU session status IE was included in the SERVICE REQUEST message, the AMF shall include a PDU session status IE in the SERVICE REJECT message to indicate which PDU sessions are active in the AMF. If the PDU session status IE is included in the SERVICE REJECT message, then the UE shall deactivate all those PDU sessions locally (without peer-to-peer signalling between the network and the UE) which are active on the UE side, but are indicated by the AMF as being inactive.

On receipt of the SERVICE REJECT message, if the message is integrity protected, the UE shall stop timer Tx if running. If the SERVICE REJECT message was received without integrity protection, then the UE shall discard the message.

If the service request for mobile originated services is rejected due to general NAS level mobility management congestion control, the network shall set the 5GMM cause value to #22 "congestion" and assign a value for back-off timer T3346.

The UE shall take the following actions depending on the 5GMM cause value received in the SERVICE REJECT message.

#xx	(N1 mode not allowed);

The UE capable of S1 mode shall disable the N1 mode radio capability (see subclause 4.9).

#xy	(Restricted service area):

The UE shall perform the mobility registration update procedure.

Editor's note:	Whether the existing cause value(s) can be used for this purpose is FFS.

#3		(Illegal UE);

#6		(Illegal ME); or

#8		(5GS services not allowed).

Editor's note:	Further details on UE actions upon receipt of the 5GMM cause value are FFS.

#10		(Implicitly de-registered).

Editor's note:	Further details on UE actions upon receipt of the 5GMM cause value are FFS.

#22	(Congestion);

If the T3346 value IE is present in the SERVICE REJECT message and the value indicates that this timer is neither zero nor deactivated, the UE shall proceed as described below, otherwise it shall be considered as an abnormal case and the behaviour of the UE for this case is specified in subclause 5.6.1.6.

If the rejected request was not for initiating a PDU session for emergency services, the UE shall abort the service request procedure and enter state 5GMM-REGISTERED, and stop timer T3517 if still running.

The UE shall stop timer T3346 if it is running.

If the SERVICE REJECT message is integrity protected, the UE shall start timer T3346 with the value provided in the T3346 value IE.

If the SERVICE REJECT message is not integrity protected, the UE shall start timer T3346 with a random value from the default range specified in 3GPP TS 24.008 [8].

For all other cases the UE stays in the current serving cell and applies normal cell reselection process. The service request procedure is started, if still necessary, when timer T3346 expires or is stopped.

#### 5.6.1.6	Abnormal cases in the UE

The following abnormal cases can be identified:

a)	Timer T3346 is running

The UE shall not start the service request procedure unless:

1)	the UE receives a paging;

2)	the UE receives a NOTIFICATION message over non-3GPP access when the UE is in 5GMM-CONNECTED mode over non-3GPP access and in 5GMM-IDLE mode over 3GPP access;

3)	the UE is a UE configured to use AC11 – 15 in selected PLMN; or

4)	the UE has a PDU session for emergency services established or is establishing a PDU session for emergency services;

If the UE is in 5GMM-IDLE mode, the UE stays in the current serving cell and applies normal cell reselection process. The service request procedure is started, if still necessary, when timer T3346 expires or is stopped.

#### 5.6.1.7	Abnormal cases on the network side

Editor's note:	Abnormal cases on the network side are FFS.

### 5.6.2	Paging procedure

#### 5.6.2.1	General

The paging procedure is used by the network to request the establishment of an NAS signalling connection to the UE. The paging procedure is also used by the network to request the UE to re-activate the user plane radio resources of PDU sessions for downlink user data transport. Another purpose of the paging procedure is to request the UE to re-activate the user-plane resources of PDU session(s) associated with non-3GPP access over 3GPP access.

Additionally, the network can use the paging procedure to initiate the mobile terminating SMS.

#### 5.6.2.2	Paging for 5GS services

##### 5.6.2.2.1	General

The network shall initiate the paging procedure for 5GS services when NAS signalling messages or user data is pending to be sent to the UE in 5GMM-IDLE mode (see example in figure 5.6.2.2.1).

Figure 5.6.2.2.1.1: Paging procedure

To initiate the procedure the 5GMM entity in the AMF requests the lower layer to start paging and shall start timer T3513. Upon reception of a paging indication, the UE shall initiate a service request procedure to respond to the paging (see 3GPP TS 23.502 [5]).

Editor's note:	It is FFS how the paging procedure is performed depending on different UE Identity of the paging message.

If downlink signalling or user data is pending to be sent over non-3GPP access, the 5GMM entity in the AMF shall indicate the lower layer that the paging is due to signalling or user data associated to non-3GPP access.

Upon reception of a paging indication, the UE shall stop the timer T3346, if running, and shall initiate the service request procedure over 3GPP access as specified in subclauses 5.6.1.

The network shall stop timer T3513 for the paging procedure when an integrity-protected response is received from the UE and successfully integrity checked by the network. If the response received is not integrity protected, or the integrity check is unsuccessful, timer T3513 for the paging procedure shall be kept running.

Upon expiry of timer T3513, the network may reinitiate paging.

If the network, while waiting for a response to the paging sent without paging priority, receives downlink signalling or downlink data associated with priority user-plane radio resources for PDU sessions, the network shall stop the timer for the paging procedure (i.e. either timer T3413), and then initiate the paging procedure with paging priority.

##### 5.6.2.2.2	Abnormal cases on the network side

Editor's note:	Abnormal cases on the network side are FFS.

##### 5.6.2.2.3	Abnormal cases in the UE

The following abnormal cases can be identified:

a)	Paging message received with access type set to non-3GPP access while the UE is in 5GMM-CONNECTED mode overnon-3GPP access

The UE shall not respond to paging message.

Editor's note:	Abnormal cases on the UE side are FFS.

### 5.6.3	Notification procedure

#### 5.6.3.1	General

The notification procedure is used by the network:

a)	to request the UE, by sending the NOTIFICATION message over 3GPP access,  to re-activate the user-plane resources of PDU session(s) associated with non-3GPP access over 3GPP access when the UE is in 5GMM-IDLE mode over non-3GPP access and in 5GMM-CONNECTED mode over 3GPP access; or

b)	to request the UE, by sending the NOTIFICATION message over non-3GPP access, to re-activate user-plane resources of the PDU session(s) associated with 3GPP access over 3GPP access when the UE is in 5GMM-CONNECTED mode over non-3GPP access and in 5GMM-IDLE mode over 3GPP access.

#### 5.6.3.2	Notification procedure initiation

The network shall initiate the notification procedure by sending the NOTIFICATION message to the UE and start timer T3565 (see example in figure 5.6.3.2.1).

For a) in subclause 5.6.3.1, the NOTIFICATION message is sent from the network to the UE via 3GPP access.

For b) in subclause 5.6.3.1, the NOTIFICATION message is sent from the network to the UE via non-3GPP access.

Figure 5.6.3.2.1: Notification procedure

Upon reception of NOTIFICATION message, the UE may initiate a service request procedure over 3GPP access as specified in subclauses 5.6.1.

Upon reception of NOTIFICATION message:

-	for case a) in subclause 5.6.3.1, if the UE decides not to initiate a service request procedure over 3GPP access based on UE policy; or

-	for case b) in subclause 5.6.3.1, if the UE is in 5GMM-REGISTERED.NO-CELL-AVAILABLE state or 5GMM-REGISTERED.PLMN-SEARCH state.

then the UE shall respond with NOTIFICATION RESPONSE message indicating failure to re-activate the user plane radio resources of PDU sessions.

For case b) in subclause 5.6.3.1, upon reception of NOTIFICATION message, the UE shall stop the timer T3346, if running.

#### 5.6.3.3	Notification procedure completion

Upon reception of SERVICE REQUEST message, the network shall stop T3565 and trigger service request procedure as specified in subclauses 5.6.3.1.

Upon reception of NOTIFICATION RESPONSE message, the network shall stop T3565 and:

-	for case a) in subclause 5.6.3.1, the AMF should notify the SMF that the UE was reachable but did not accept to re-activate the user plane radio resources of PDU sessions.

-	for case b) in subclause 5.6.3.1, the AMF should notify the SMF that the UE is unreachable.

#### 5.6.3.4	Abnormal cases on the network side

The following abnormal cases can be identified:

a)	Expiry of timer T3565

The network shall, on the first expiry of the timer T3565, retransmit the NOTIFICATION message and shall reset and start timer T3565. This retransmission is repeated four times, i.e. on the fifth expiry of timer T3565, the procedure shall be aborted. In addition, upon the fifth expiry of timer T3565:

For case a) in subclause 5.6.3.1, the AMF should notify the SMF that the UE is unreachable. The AMF may enter 5GMM-IDLE mode over 3GPP access.

For case b) in subclause 5.6.3.1, the AMF may either:

1)	perform the paging procedure over the 3GPP access; or

2)	notify the SMF that the UE is unreachable.

NOTE:	Whether the AMF performs the paging procedure or notifies the SMF is up to operator’s policies.

Editor's note:	Further abnormal cases on the network side are FFS.

# 6	Elementary procedures for 5GS session management

## 6.1	Overview

### 6.1.1	General

### 6.1.2	Types of 5GSM procedures

### 6.1.3	5GSM sublayer states

#### 6.1.3.1	General

In the following subclauses, the possible states of a 5G PDU session context in the UE and the network side are described.

#### 6.1.3.2	5GSM sublayer states in the UE

##### 6.1.3.2.1	Overview

In the following subclauses, the possible 5GSM sublayer states of the UE are described and shown in figure 6.1.3.2.1.1.

NOTE:	Not all possible transitions are shown in this figure.

Editor's note:	The fatal causes and non-fatal causes used in the 5GSM procedure are FFS.

Editor's note:	Whether the PDU session establishment authentication and authorization procedure impacts the state transition is FFS.

Figure 6.1.3.2.1.1: The 5GSM sublayer states for PDU session handling in the UE (overview)

##### 6.1.3.2.2	PDU SESSION INACTIVE

No PDU session context exists.

##### 6.1.3.2.3	PDU SESSION ACTIVE PENDING

The UE has initiated a PDU session establishment procedure towards the network and is waiting for a response from the network.

##### 6.1.3.2.4	PDU SESSION ACTIVE

The PDU session context is active in the UE.

##### 6.1.3.2.5	PDU SESSION INACTIVE PENDING

The UE has initiated a PDU session release procedure towards the network and is waiting for a response from the network.

##### 6.1.3.2.6	PDU SESSION MODIFICATION PENDING

The UE has initiated a PDU session modification procedure towards the network and is waiting for a response from the network.

##### 6.1.3.2.7	PDU SESSION ACTIVE PENDING

The UE has initiated a PDU session establishment procedure towards the network and is waiting for a response from the network.

##### 6.1.3.2.8	PDU SESSION ACTIVE

The PDU session context is active in the UE.

##### 6.1.3.2.9	PDU SESSION INACTIVE PENDING

The UE has initiated a PDU session release procedure towards the network and is waiting for a response from the network.

##### 6.1.3.2.10	PDU SESSION MODIFICATION PENDING

The UE has initiated a PDU session modification procedure towards the network and is waiting for a response from the network.

##### 6.1.3.2.11	PROCEDURE TRANSACTION INACTIVE

No procedure transaction exists.

##### 6.1.3.2.12	PROCEDURE TRANSACTION PENDING

The UE has initiated a procedure transaction towards the network.

Figure 6.1.3.2.12.1: The procedure transaction states in the UE (overview)

#### 6.1.3.3	5GSM sublayer states in the network side

##### 6.1.3.3.1	Overview

In the following subclauses, the possible 5GSM sublayer states of the network are described and shown in Figure 6.1.3.3.1.1.

NOTE:	Not all possible transitions are shown in this figure.

Editor's note:	The fatal causes and non-fatal causes used in the 5GSM procedure are FFS.

Editor's note:	Whether the PDU session establishment authentication and authorization procedure impacts the state transition is FFS.

Figure 6.1.3.3.1.1: The 5GSM sublayer states for PDU session handling in the network (overview)

##### 6.1.3.3.2	PDU SESSION INACTIVE

No PDU session context exists.

##### 6.1.3.3.3	PDU SESSION ACTIVE

The PDU session context is active in the network.

##### 6.1.3.3.4	PDU SESSION INACTIVE PENDING

The network has initiated a PDU session release procedure towards the UE and is waiting for a response from the UE.

##### 6.1.3.3.5	PDU SESSION MODIFICATION PENDING

The network has initiated a PDU session modification procedure towards the UE and is waiting for a response from the UE.

##### 6.1.3.3.6	PROCEDURE TRANSACTION INACTIVE

No procedure transaction exists.

##### 6.1.3.3.7	PROCEDURE TRANSACTION PENDING

The network has initiated a procedure transaction towards the UE.

Figure 6.1.3.3.7.1: The procedure transaction states in the network (overview)

### 6.1.4	Coordination between 5GSM and ESM

#### 6.1.4.1	General

Interworking to EPS is supported for a PDU session context, if the context includes one or more QoS rules each of which contains associated EPS bearer identity, mapped EPS QoS parameters, mapped extended EPS QoS parameters, and optionally a mapped traffic flow template. The QoS rule for the default QoS flow does not need to contain a mapped traffic flow template. See coding of the QoS rules in subclause 9.8.4.6.

Upon inter-system change from N1 mode to S1 mode in EMM-IDLE mode, the UE shall create the default EPS bearer context from the QoS flow of the default QoS rule and create the dedicated EPS bearer context(s) from the other QoS flow(s) in the PDU session context, if available. The UE uses the parameters from each PDU session context for which interworking to EPS is supported to create corresponding default EPS bearer context and optinally dedicated EPS bearer context(s) as follows:

a)	the PDU session type of the PDU session context shall be mapped to the PDN type of the default EPS bearer context as follows:

1)	the PDN type shall be set to "non-IP" if the PDU session type is "Ethernet" or "Unstructured";

2)	the PDN type shall be set to "IPv4" if the PDU session type is "IPv4"; and

3)	the PDN type shall be set to "IPv6" if the PDU session type is "IPv6";

b)	the PDU address of the PDU session context shall be mapped to the PDN address of the default EPS bearer context;

c)	the DNN of the PDU session shall be mapped to the APN of the default EPS bearer context;

d)	for each PDU session context of a PDU session in state PDU SESSION ACTIVE or PDU SESSION MODIFICATION PENDING the UE shall set the state of the mapped EPS bearer context(s) to BEARER CONTEXT ACTIVE; and

e)	for any other PDU session context of a PDU session the UE shall set the state of the mapped EPS bearer context(s) to BEARER CONTEXT INACTIVE.

Additionally, the UE shall set for each QoS flow in the PDU session context:

a)	the EPS bearer identity to the EPS bearer identity received in the parameters of a QoS rule for the QoS flow;

b)	the EPS QoS parameters to the mapped EPS QoS parameters received in the parameters of a QoS rule for the QoS flow;

c)	the extended EPS QoS parameters to the mapped extended EPS QoS parameters received in the parameters of a QoS rule for the QoS flow; and

d)	the traffic flow template to the mapped traffic flow template received in the parameters of a QoS rule for the QoS flow, if available.

Interworking to 5GS is supported for a PDN connection, if the corresponding default EPS bearer context includes a PDU session identity, session AMBR and one or more QoS rules (see 3GPP TS 24.301 [10]).

Upon inter-system change from S1 mode to N1 mode in 5GMM-IDLE mode, the UE uses the parameters from the default EPS bearer context of each PDN connection for which interworking to 5GS is supported to create a corresponding PDU session context as follows:

a)	the PDN type of the default EPS bearer context optionally associated with locally available information associated with the PDN connection (either "Ethernet" or "Unstructured") shall be mapped to the PDU session type of the PDU session context as follows:

1)	if the PDN type is "non-IP", the PDU session type is set to the locally available information associated with the PDN connection (either "Ethernet" or "Unstructured"), if available;

2)	if the PDN type is "IPv4" the PDU session type is set to "IPv4"; and

3)	if the PDN type is "IPv6", the PDU session type is set to "IPv6";

b)	the PDN address of the default EPS bearer context shall be mapped to PDU address of the PDU session context;

c)	the APN of the default EPS bearer context shall be mapped to the DNN of the PDU session context;

d)	for each default EPS bearer context in state BEARER CONTEXT ACTIVE or BEARER CONTEXT MODIFY PENDING the UE shall set the state of the mapped PDU session context to PDU SESSION ACTIVE; and

e)	for any other default EPS bearer context the UE shall set the state of the mapped PDU session context to PDU SESSION INACTIVE.

Additionally, the UE shall set:

a)	the PDU session identity of the PDU session context to the PDU session identity included by the UE in the protocol configuration options IE or extended protocol configuration options IE in the PDN CONNECTIVITY REQUEST message; and

b)	the S-NSSAI of the PDU session context to the S-NSSAI included by the network in the protocol configuration options IE or extended protocol configuration options IE in the ACTIVATE DEFAULT EPS BEARER REQUEST message; and

c)	the session-AMBR of the PDU session context to the session-AMBR included by the network in the protocol configuration options IE or extended protrocol configuration options IE in the ACTIVATE DEFAULT EPS BEARER REQUEST message.

For each EPS bearer context of the PDN connection, the UE shall create QoS flow(s) each of which is associated with the QoS rule(s) received in the protocol configuration options IE or extended protrocol configuration options IE in the ACTIVATE DEFAULT EPS BEARER REQUEST message, ACTIVATE DEDICATED EPS BEARER REQUEST message, and/or MODIFY EPS BEARER REQUEST message (see 3GPP TS 24.301 [10]).

#### 6.4.1.2	Coordination between 5GSM and ESM without N26 interface in single-registration mode

At intersystem change from S1 mode to N1 mode, after having successfully registered in N1 mode the UE operating in the single-registration mode shall:

a)	if the UE supports the PDU session establishment procedure with request type set to "existing PDU session" to transfer a PDN connection from S1 mode to N1 mode and the UE has received an "interworking without N26 supported" indication from the network, attempt to transfer all active PDN connections, if any, for which interworking to 5GS is supported from S1 mode to N1 mode by initiating the PDU session establishment procedure with request type set to "existing PDU session"; and

Editor's note:	Handling of EPS bearer context states and PDU session states are FFS.

b)	otherwise, establish PDU sessions corresponding to all active PDN connections, if any, by initiating the PDU session establishment procedure with request type set to "initial request".

Editor's note:	Handling of EPS bearer context states and PDU session states are FFS.

## 6.2	General on elementary 5GSM procedures

### 6.2.1	Principles of PTI handling for 5GSM procedures

When the UE or the network initiates a transaction related procedure (i.e. a procedure consisting of more than one message and the messages are related), it shall include a valid PTI value in the message header of the request message or of the command message.

If a response message is sent as result of a received request message or a received command message, the sending entity shall include in the response message the PTI value received within the request message or within the command message (see examples in figure 6.2.1.1, figure 6.2.1.2, figure 6.2.1.3, and figure 6.2.1.4).

If a command message is sent as result of a received request message, the sending entity shall include in the command message the PTI value received with the request message (see examples in figure 6.2.1.3).

If a command message is not sent as result of a received request message, the sending entity shall include in the command message the PTI value set to "no procedure transaction identity assigned" (see examples in figure 6.2.1.3).

Figure 6.2.1.1: UE-requested transaction related procedure accepted by the network

Figure 6.2.1.2: UE-requested transaction related procedure rejected by the network

Figure 6.2.1.3: UE-requested transaction related procedure triggering a network-requested transaction related procedure

Figure 6.2.1.4: network-requested transaction related procedure not triggered by a UE-requested transaction related procedure

### 6.2.2	PDU session types

The following PDU Session types are supported:

a)	IPv4;

b)	IPv6;

c)	Ethernet (EtherType as defined in IEEE 802.3); and

d)	Unstructured.

IP address allocation for IPv4 and IPv6 PDU session types is described in subclause 6.2.4.

Neither a MAC nor an IP address is allocated by the 5GCN to the UE for Ethernet PDU session type.

### 6.2.3	PDU session management

The SMF is responsible for the session management functions to provide the PDU connectivity service to the UE via the SM signalling between UE and SMF.  The session management procedures includes:

a)	the UE-requested PDU session establishment procedure;

b)	the PDU session establishment authentication and authorization procedure;

c)	the UE-requested PDU session modification procedure;

d)	the network-requested PDU session modification procedure; and

e)	the UE-requested PDU session release procedure; and

f)	the network-requested PDU session release procedure.

A UE may establish multiple PDU sessions, to the same data network or to different data networks, via 3GPP and via and Non-3GPP access networks at the same time.

The session management messages between UE and SMF are transferred via AMF as specified in subclause 8.3.

### 6.2.4	IP address allocation

#### 6.2.4.1	General

This clause specifies IP address allocation for the PDU session.

In this release of specification, PDU sessions initiated with the PDU session type IP shall support only one IP version, i.e. IPv4 PDU session type or IPv6 PDU session type.

IP address allocation to the UE shall be performed by SMF based on the selected IP version and operator policies. If IPv4 PDU session type is selected, an IPv4 address is allocated to the UE. If IPv6 PDU session type is selected, an IPv6 prefix and an interface identifier for the IPv6 link local address are allocated to the UE.

For IPv4 PDU session type, the UE:

a)	shall obtain an IPv4 address via:

1)	NAS signalling as specified in subclause 6.2.4.2; or

2)	DHCPv4; and

b)	may obtain IPv4 configuration parameters (e.g. DNS server address) via DHCPv4.

For IPv6 PDU session type, the UE:

a)	shall build an IPv6 link local address based on the allocated interface identifier for the IPv6 link local address;

b)	shall obtain /64 IPv6 prefix via IPv6 stateless address autoconfiguration as specified in 3GPP TS 23.501 [4] and IETF RFC 4862 [23]; and

c)	may obtain IPv6 configuration parameters via stateless DHCPv6 as specified in IETF RFC 3736 [29].

The UE capable of IPv6 shall support acting as a type C host as specified in IETF RFC 4191 [20].

#### 6.2.4.2	IP address allocation via NAS signalling

The UE shall set the PDU session type IE in the PDU SESSION ESTABLISHMENT REQUEST message, based on its IP stack capabilities if the UE requests IP connectivity as follows:

a)	A UE:

1)	which is IPv6 and IPv4 capable, shall set the PDU session type IE to IP.

2)	which is only IPv6 capable, shall set the PDU session type IE to IPv6.

3)	which is only IPv4 capable, shall set the PDN type IE to IPv4.

b)	When the IP version capability of the UE is unknown in the UE (as in the case when the MT and TE are separated and the capability of the TE is not known in the MT), the UE shall set the PDU session type IE to IP.

If the UE wants to use DHCPv4 for IPv4 address assignment, it shall indicate that to the network within the Extended protocol configuration options IE in the PDU SESSION ESTABLISHMENT REQUEST.

On receipt of the PDU SESSION ESTABLISHMENT REQUEST message sent by the UE, the network when allocating an IP address shall take into account the PDU session type IE, the operator's policies of the network, and the user's subscription data and:

a)	if the UE requests the PDU session type IP, but the subscription or SMF configuration is limited to IPv4 only or IPv6 only for the requested DNN, the network shall set the PDU session type IE to either "IPv4" or "IPv6" and the 5GSM cause value to #50 "PDU session type IPv4 only allowed", or #51 "PDU session type IPv6 only allowed" in the PDU SESSION ESTABLISHMENT ACCEPT message, respectively. The UE shall not subsequently initiate another UE requested PDU session establishment procedure to the same DNN to obtain a PDU session type different from the one allowed by the network;

b)	if the network sets the PDU session type IE to IPv4, the network shall include an IPv4 address in the PDU address IE; and

c)	if the network sets the PDU session type IE to IPv6, the network shall include an interface identifier for the IPv6 link local address in the PDU address IE.

### 6.2.5	Quality of service

#### 6.2.5.1	General

##### 6.2.5.1.1	QoS rules

###### 6.2.5.1.1.1	General

In a PDU session of IPv4, IPv6, and Ethernet PDU session type, the NAS protocol enables different forwarding treatments of UL user data packets in one or more QoS flows based on signalled QoS rules, derived QoS rules or any combination of them.

In a PDU session of Unstructured PDU session type, all UL user data packets are associated with the same QoS flow.

###### 6.2.5.1.1.2	Signalled QoS rules

The NAS protocol enables the network to provide the UE with signalled QoS rules associated with a PDU session.

The network can provide the UE with one or more signalled QoS rules associated with a PDU session at the PDU session establishment or at QoS flow establishment.

Each signalled QoS rule contains:

a)	an indication of whether the QoS rule is the default QoS rule;

b)	a QoS rule identifier (QRI);

c)	a QoS flow identifier (QFI);

d)	optionally a set of packet filters. If the QoS rule is not the default QoS rule, the set of packet filters is included and contains at least one packet filter for UL direction or at least one packet filter for both UL and DL directions;

e)	a precedence value;

f)	if the flow is a GBR QoS flow:

1)	Guaranteed flow bit rate (GFBR) for UL;

2)	Guaranteed flow bit rate (GFBR) for DL;

3) Maximum flow bit rate (MFBR) for UL;

4)	Maximum flow bit rate (MFBR) for DL;

5)	optionally averaging window for UL; and

6)	optionally averaging window for DL; and

g)	5QI, if the QFI is not the same as the 5QI of the QoS flow identified by the QFI; and

h)	optionally a EPS bearer identity (EBI), mapped EPS QoS parameters, mapped extended EPS QoS parameters and mapped traffic flow template if the QoS flow can be mapped to an EPS bearer as specified in subclause 4.11.2 of 3GPP TS 23.501 [4].

If the UE requests a new QoS flow, it shall assign a precedence value for the signalled QoS rule which is not in the range from 70 to 99 (decimal).

If the averaging window for UL is not included in a QoS rule for a GBR QoS flow with a 5QI indicated in 3GPP TS 23.501 [4] table 5.7.4-1, the averaging window associated with the 5QI in 3GPP TS 23.501 [4] table 5.7.4-1 applies for the averaging window for UL. If the averaging window for DL is not included in a QoS rule for a GBR QoS flow with a 5QI indicated in 3GPP TS 23.501 [4] table 5.7.4-1, the averaging window associated with the 5QI in 3GPP TS 23.501 [4] table 5.7.4-1 applies for the averaging window for DL.

If the averaging window for UL is not included in a QoS rule for a GBR QoS flow with a 5QI not indicated in 3GPP TS 23.501 [4] table 5.7.4-1, the standardized value of two seconds is used as the averaging window for UL. If the averaging window for DL is not included in a QoS rule for a GBR QoS flow with a 5QI not indicated in 3GPP TS 23.501 [4] table 5.7.4-1, the standardized value of two seconds is used as the averaging window for DL.

Editor's note:	The need of the averaging window for DL is FFS.

Within a PDU session:

a)	each signalled QoS rule has a unique QRI;

b)	there is at least one signalled QoS rule;

c)	one signalled QoS rule is the default QoS rule;

d)	there can be zero, one or more signalled QoS rules associated with a given QFI; and

e)	there can be up to one signalled QoS rule without a set of packet filters or with a set of packet filters not containing a packet filter for UL direction and not containing a packet filter for both UL and DL directions.

###### 6.2.5.1.1.3	Derived QoS rules

The reflective QoS in the UE creates derived QoS rules associated with a PDU session based on DL user data packets received via the PDU session.

Each derived QoS rule contains:

a)	a QoS flow identifier (QFI);

b)	a packet filter for UL direction; and

c)	a precedence value of 80 (decimal).

NOTE:	On the network side, the corresponding QoS rule can be associated with a different precedence value in the range from 70 to 99 (decimal).

Within a PDU session:

a)	there can be zero, one or more derived QoS rules associated with a given QFI; and

b)	there can be up to one derived QoS rule associated with a given packet filter for UL direction.

In the UE, a timer X runs for each derived QoS rule.

Editor's note:	Creation and maintaining of derived QoS rules generates additional requirements for the UE processing capacity. The UE can support only a limited number of concurrent derived QoS rules. Whether the maximum number of concurrent derived QoS rules should be standardized or implementation specific is FFS.

Editor's note:	The UE actions when the maximum number of concurrent derived QoS rules is exceeded (e.g. whether the UE deletes the least used derived QoS rule or stop deriving new QoS rules etc.) is FFS.

##### 6.2.5.1.2	Session-AMBR

The NAS protocol enables the network to provide the UE with the session-AMBR associated with a PDU session.

The standardized value of two seconds is used as the averaging window for the UE's enforcement of the UL rate limitation indicated by the session-AMBR.

##### 6.2.5.1.3	UL user data packet matching

Upon receiving an UL user data packet from the upper layers for transmission via a PDU session, the UE shall attempt to associate the UL user data packet with:

a)	the QFI of a signalled QoS rule associated with the PDU session which has a set of packet filters containing a packet filter for UL direction matching the UL user data packet or containing a packet filter for both UL and DL directions matching the UL user data packet;

b)	the QFI of a derived QoS rule associated with the PDU session which has the packet filter for UL direction matching the UL user data packet; or

c)	the QFI of a signalled QoS rule associated with the PDU session which does not have a set of packet filters or has a set of packet filters not containing a packet filter for UL direction and not containing a packet filter for both UL and DL directions;

by evaluating the QoS rules in increasing order of their precedence values until the UL user data packet is associated with a QFI or all QoS rules are evaluated.

If the UL user data packet is associated with a QFI, the UE shall pass the QFI along the UL user data packet to the lower layers for transmission.

NOTE:	Marking of the UL user data packet with the QFI is performed by the lower layers.

If all QoS rules are evaluated and the UL user data packet is not associated with a QFI, the UE shall discard the UL user data packet.

##### 6.2.5.1.4	Reflective QoS

###### 6.2.5.1.4.1	General

The UE may support reflective QoS.

If the UE supports the reflective QoS, the UE shall support the procedures in the following subclauses.

The reflective QoS is applicable in a PDU session of IPv4, IPv6 and Ethernet PDU session type. The reflective QoS is not applicable in a PDU session of Unstructured PDU session type.

###### 6.2.5.1.4.2	Derivation of packet filter for UL direction from DL user data packet

The UE shall derive a packet filter for UL direction from a DL user data packet as follows:

a)	if the received DL user data packet belongs to a PDU session IPv4 PDU session type and:

1)	the protocol field of the received DL user data packet indicates TCP as specified in IETF RFC 793 [17];

2)	the protocol field of the received DL user data packet indicates UDP as specified in IETF RFC 768 [18]; or

3)	the protocol field of the received DL user data packet indicates ESP as specified in IETF RFC 4303 [22] and an uplink IPSec SA corresponding to a downlink IPSec SA indicated in the security parameters index field of the received DL user data packet exists;

then the packet filter for UL direction contains the following packet filter components:

1)	an IPv4 remote address component set to the value of the source address field of the received DL user data packet;

2)	an IPv4 local address component set to the value of the destination address field of the received DL user data packet;

3)	a protocol identifier/next header type component set to the value of the protocol field of the received DL user data packet;

4)	if the protocol field of the received DL user data packet indicates TCP as specified in IETF RFC 793 [17] or UDP as specified in IETF RFC 768 [18]:

i)	a single local port type component set to the value of the destination port field of the received DL user data packet; and

ii)	a single remote port type component set to the value of the source port field of the received DL user data packet; and

5)	if the protocol field of the received DL user data packet indicates ESP as specified in IETF RFC 4303 [22]:

i)	a security parameter index type component set to the security parameters index of the uplink IPSec SA corresponding to the downlink IPSec SA indicated in the security parameters index field of the received DL user data packet;

b)	if the received DL user data packet belongs to a PDU session of IPv6 PDU session type and:

1)	the last next header field of the received DL user data packet indicates TCP as specified in IETF RFC 793 [17];

2)	the last next header field of the received DL user data packet indicates UDP as specified in IETF RFC 768 [18]; or

3)	the last next header field of the received DL user data packet indicates ESP as specified in IETF RFC 4303 [22] and an uplink IPSec SA corresponding to a downlink IPSec SA indicated in the security parameters index field of the received DL user data packet exists;

then the packet filter for UL direction contains the following packet filter components:

1)	an IPv6 remote address/prefix length component set to the value of the source address field of the received DL user data packet;

2)	an IPv6 local address/prefix length component set to the value of the destination address field of the received DL user data packet;

3)	a protocol identifier/next header type component set to the value of the last next header field of the received DL user data packet;

4)	if the last next header field of the received DL user data packet indicates TCP as specified in IETF RFC 793 [17] or UDP as specified in IETF RFC 768 [18]:

i)	a single local port type component set to the value of the destination port field of the received DL user data packet; and

ii)	a single remote port type component set to the value of the source port field of the received DL user data packet; and

5)	if the last next header field of the received DL user data packet indicates ESP as specified in IETF RFC 4303 [22]:

i)	a security parameter index type component set to the security parameters index of the uplink IPSec SA corresponding to the downlink IPSec SA indicated in the security parameters index field of the received DL user data packet; and

c)	if the received DL user data packet belongs to a PDU session of Ethernet PDU session type, the packet filter for UL direction contains the following packet filter components:

1)	a destination MAC address component set to the source MAC address of the received DL user data packet;

2)	a source MAC address component set to the destination MAC address of the received DL user data packet;

3)	if an 802.1Q C-TAG is included in the received DL user data packet, an 802.1Q C-TAG VID component set to the 802.1Q C-TAG VID of the received DL user data packet and an 802.1Q C-TAG PCP/DEI component set to the 802.1Q C-TAG PCP/DEI of the received DL user data packet;

4)	if an 802.1Q S-TAG is included in the received DL user data packet, an 802.1Q S-TAG VID component set to the 802.1Q S-TAG VID of the received DL user data packet and an 802.1Q S-TAG PCP/DEI component set to the 802.1Q S-TAG PCP/DEI of the received DL user data packet;

5)	If the Ethertype field of the received DL user data packet is set to a value of 1536 or above, an Ethertype component set to the Ethertype of the received DL user data packet;

6)	if the Ethertype field of the Ethernet frame header indicates that the data carried in the Ethernet frame is IPv4 data, the UE shall also add to the packet filter for UL direction the IP-specific components based on the contents of the IP header of the received DL user data packet as described in bullet a) above; and

7)	if the Ethertype field of the Ethernet frame header indicates that the data carried in the Ethernet frame is IPv6 data, the UE shall also add to the packet filter for UL direction the IP-specific components based on the contents of the IP header of the received DL user data packet as described in bullet b) above.

###### 6.2.5.1.4.3	Creating a derived QoS rule by reflective QoS in the UE

If the UE receives a DL user data packet marked with a QFI and an RQI, the DL user data packet belongs to a PDU session of IPv4, IPv6, or Ethernet PDU session type, and the UE does not have a derived QoS rule with the same packet filter for UL direction as the packet filter for UL direction derived from the DL user data packet as specified in subclause 6.2.5.1.4.2, then the UE shall create a new derived QoS rule as follows:

a)	the QFI of the derived QoS rule is set to the received QFI;

b)	the precedence value of the derived QoS rule is set to80 (decimal); and

c)	the packet filter for UL direction of the derived QoS rule is set to the derived packet filter for UL direction;

and the UE shall start the timer X associated with the derived QoS rule with the RQ timer value received during the UE-requested PDU session establishment procedure as specified in subclause 6.4.1. If the RQ timer value was not received in the UE-requested PDU session establishment procedure, the default standardized RQ timer value is used.

Editor's note: FFS what is the default standardized RQ timer value.

###### 6.2.5.1.4.4	Updating a derived QoS rule by reflective QoS in the UE

If the UE receives a DL user data packet associated with a QFI and an RQI, the DL user data packet belongs to a PDU session of IPv4, IPv6, or Ethernet PDU session type, and the UE has a derived QoS rule with the same packet filter for UL direction as the packet filter for UL direction derived from the DL user data packet as specified in subclause 6.2.5.1.4.2:

a)	if the QFI value associated with the DL user data packet is different from the QFI value stored for the derived QoS rule, the UE shall replace the QFI value stored for the derived QoS rule with the new QFI value for the derived QoS rule; and

b)	the UE shall re-start the timer X associated with the derived QoS rule with the RQ timer value received during the UE-requested PDU session establishment procedure as specified in subclause 6.4.1. If the RQ timer value was not received in the UE-requested PDU session establishment procedure, the default standardized RQ timer value is used.

Editor's note:	FFS what is the default standardized RQ timer value.

###### 6.2.5.1.4.5	Deleting a derived QoS rule by reflective QoS in the UE

Upon expiration of timer X associated with a derived QoS rule, the UE shall remove the derived QoS rule.

Upon release of the PDU session, the UE shall remove the derived QoS rule(s) associated with the PDU session.

### 6.2.6	Local area data network

The UE can receive the local area data network (LADN) information consisting of LADN DNNs and LADN service area (a set of tracking areas that belong to the current registration area) information based on the UE location during the registration procedure or the generic UE configuration update procedure (see subclause 5.5.1 and subclause 5.4.4).

The UE may request a PDU session establishment and modification or initiate the service request to re-active the PDU session for an available LADN when the UE is located in the LADN service area. If the UE has moved out of the LADN service area, the PDU session for LADN may be released by the SMF, or the user-plane resources for the PDU session may be deactivated by the SMF. If the UE has returned to the LADN service area, and the network has downlink user data pending, the network re-activates the user-plane resources for the PDU session for LADN.

### NOTE:	In this release, LADNs apply only to 3GPP access.6.2.7	Handling of DNN based congestion control

The network may detect and start performing DNN based congestion control when one or more DNN congestion criteria as specified in 3GPP TS 23.501 [4] are met. The network may store a DNN congestion back-off timer on a per UE and congested DNN basis. If the UE does not provide a DNN for a non-emergency PDU session, then the SMF uses the selected DNN.

In the UE, 5GS session management timers T35ab for DNN based congestion control are started and stopped on a per DNN basis.

In case the timer T35ab is provided during the PDU session establishment procedure, the DNN associated with T35ab is the DNN provided by the UE when the PDU session is established. If no DNN is provided by the UE along the PDU SESSION ESTABLISHMENT REQUEST, then T35ab is associated with no DNN. For this purpose the UE shall memorize the DNN provided to the network during the PDU session establishment. The timer T35ab associated with no DNN will never be started due to any 5GSM procedure related to an emergency PDU session. If the timer T35ab associated with no DNN is running, it does not affect the ability of the UE to request an emergency PDU session.

In case the timer T35ab is provided during the UE-requested PDU session modification procedure or the network-requested PDU session release procedure, the DNN associated with T35ab is the DNN associated with the PDU session corresponding to the 5GSM procedure.

If T35ab is running or is deactivated, and the UE is a UE configured to use AC11 – 15 in selected PLMN, then the UE is allowed to initiate 5GSM session management procedure for the respective DNN.

### 6.2.8	Handling of S-NSSAI based congestion control

The network may detect and start performing S-NSSAI based congestion control when one or more S-NSSAI congestion criteria as specified in 3GPP TS 23.501 [4] are met. The network may store a S-NSSAI congestion back-off timer on a per UE, S-NSSAI, and optionally DNN basis. If the UE does not provide a DNN for a non-emergency PDU session, then the SMF uses the selected DNN.

In the UE, 5GS session management timers T35cd for the S-NSSAI based congestion control are started and stopped on a per S-NSSAI and optionally DNN basis.

In case the timer T35cd is provided during the PDU session establishment procedure, the S-NSSAI associated with T35cd is the S-NSSAI provided by the UE when the PDU session is established. The DNN associated with T35cd is the DNN provided by the UE when the PDU session is established. If no DNN is provided by the UE along the PDU SESSION ESTABLISHMENT REQUEST, then T35cd is associated with no DNN and S-NSSAI provided to the network during the PDU session establishment. For this purpose the UE shall memorize the DNN and S-NSSAI provided to the network during the PDU session establishment. The timer T35cd associated with no DNN and an S-NSSAI will never be started due to any 5GSM procedure related to an emergency PDU session. If the timer T35cd associated with no DNN and a S-NSSAI is running, it does not affect the ability of the UE to request an emergency PDU session.

In case the timer T35cd is provided during the UE-requested PDU session modification procedure or the network-requested PDU session release procedure, the S-NSSAI and DNN associated with T35cd are the S-NSSAI and DNN associated with the PDU session corresponding to the 5GSM procedure.

If T35cd is running or is deactivated, and the UE is a UE configured to use AC11 – 15 in selected PLMN, then the UE is allowed to initiate 5GSM session management procedure for the respective DNN and an S-NSSAI.

### 6.2.9	URSP

Editor's note:	Which entity in the UE handles URSP is FFS

URSP is a set of one or more URSP rules, where a URSP rule is composed of:

a)	a precedence value of the URSP rule;

b)	a route selection descriptor, including non-seamless offload indication and at least one of the followings:

1)	SSC mode;

2)	S-NSSAI;

3)	DNN; and

4)	preferred access type; and

c)	optionally, a traffic descriptor including:

1)	one or more packet filters for UL direction; or

2)	one or more application identifiers.

Only one URSP rule in URSP can be a default URSP rule and except for the default URSP rule, a URSP rule shall contain a traffic descriptor. If a default URSP rule and one or more non-default URSP rules are included in URSP, any non-default URSP rule shall have lower precedence value than (i.e. shall be prioritised over) the default URSP rule.

When a PDU needs to be transmitted, the UE shall evaluate the URSP rules in increasing order of their precedence values. If the UE finds a URSP rule matching the PDU (including a default URSP rule):

a)	if there exists a PDU session matching the route selection descriptor of the URSP rule, the UE shall transmit the PDU via the PDU session; or

b)	otherwise if the UE is registered via the preferred access type included in the route selection descriptor of the matching URSP rule, the non-seamless offload indication included in the route descriptor of the matching URSP rule indicates that "non-seamless offload should not be applied", and other conditions do not prohibit the UE from establishing a new PDU session, the UE establishes a PDU session using the SSC mode, S-NSSAI, and/or DNN via the preferred access type indicated by the route selection descriptor of the URSP rule (see subclause 6.4.1).

Editor's note:	It is FFS how the UE operates in case the UE is not registered via the preferred access type or the non-seamless offload indication indicates that "Non-seamless offload should be applied".

Editor's note:	Non-seamless offload should be defined.

The HPLMN may provision the UE with URSP. The URSP shall be stored in a non-volatile memory in the ME together with the SUPI from the USIM. The URSP shall be stored until new URSP are provisioned by HPLMN or the USIM is removed. The URSP can only be used if the SUPI from the USIM matches the SUPI stored in the non-volatile memory of the ME; else the UE shall delete them. The URSP may be updated via the generic UE configuration update procedure (see 3GPP TS 23.503 [6] and subclause 5.4.4).

Editor's note:	Coding of the URSP is FFS.

## 6.3	Network-requested 5GSM procedures

### 6.3.1	PDU session establishment authentication and authorization procedure

#### 6.3.1.1	General

The purpose of the PDU session establishment authentication and authorization procedure is to enable the DN:

1)	to authenticate the upper layers of the UE, when establishing the PDU session;

2)	to authorize the upper layers of the UE, when establishing the PDU session; or

3)	both of the above.

The PDU session establishment authentication and authorization procedure can be performed only during the UE-requested PDU session establishment procedure establishing a non-emergency PDU session. The PDU session establishment authentication and authorization procedure shall not be performed during the UE-requested PDU session establishment procedure establishing an emergency PDU session.

The network authenticates the UE using the Extensible Authentication Protocol (EAP) as specified in IETF RFC 3748 [19].

EAP has defined four types of EAP messages:

a)	an EAP-request message;

b)	an EAP-response message;

c)	an EAP-success message; and

d)	an EAP-failure message.

The EAP-request message is transported from the network to the UE using the PDU SESSION AUTHENTICATION COMMAND message.

The EAP-response message to the EAP-request message is transported from the UE to the network using the PDU SESSION AUTHENTICATION COMPLETE message.

If the DN authentication of the UE completes successfully, the EAP-success message is transported from the network to the UE as part of the UE-requested PDU session establishment procedure in the PDU SESSION ESTABLISHMENT ACCEPT message.

If the DN authentication of the UE completes unsuccessfully, the EAP-failure message is transported from the network to the UE as part of the UE-requested PDU session establishment procedure in the PDU SESSION ESTABLISHMENT REJECT message.

There can be several rounds of exchange of an EAP-request message and a related EAP-response message for the DN to complete the authentication and authorization of the request for a PDU session (see example in figure 6.3.1.1).

The SMF shall set the authenticator retransmission timer specified in IETF RFC 3748 [19] subclause 4.3 to infinite value.

NOTE 2:	The PDU session establishment authentication and authorization procedure provides a reliable transport of EAP messages and therefore retransmissions at the EAP layer of the SMF do not occur.

Figure 6.3.1.1: PDU session establishment authentication and authorization procedure

#### 6.3.1.2	PDU session establishment authentication and authorization procedure initiation

In order to initiate the PDU session establishment authentication and authorization procedure, the SMF shall create a PDU SESSION AUTHENTICATION COMMAND message.

The SMF shall set the PTI IE of the PDU SESSION AUTHENTICATION COMMAND message to "No procedure transaction identity assigned".

The SMF shall set the EAP message IE of the PDU SESSION AUTHENTICATION COMMAND message to the EAP-request message provided by the DN or generated locally.

The SMF shall send the PDU SESSION AUTHENTICATION COMMAND message, and the SMF shall start timer T3590 (see example in figure 6.3.1.1).

Upon receipt of a PDU SESSION AUTHENTICATION COMMAND message and a PDU session ID, using the NAS transport procedure as specified in subclause 5.4.5, the UE passes to the upper layers the EAP message received in the EAP message IE of the PDU SESSION AUTHENTICATION COMMAND message. Apart from this action, the authentication and authorization procedure initiated by the DN is transparent to the 5GSM layer of the UE.

#### 6.3.1.3	PDU session establishment authentication and authorization procedure accepted by the UE

When the upper layers provide an EAP-response message responding to the received EAP-request message, the UE shall create a PDU SESSION AUTHENTICATION COMPLETE message.

The UE shall set the EAP message IE of the PDU SESSION AUTHENTICATION COMPLETE message to the EAP-response message.

The UE shall transport the PDU SESSION AUTHENTICATION COMPLETE message and the PDU session ID, using the NAS transport procedure as specified in subclause 5.4.5. Apart from this action, the authentication and authorization procedure initiated by the DN is transparent to the 5GSM layer of the UE.

Upon receipt of a PDU SESSION AUTHENTICATION COMPLETE message, the SMF shall stop timer T3590 and provides the EAP message received in the EAP message IE of the PDU SESSION AUTHENTICATION COMPLETE message to the DN or handles it locally.

#### 6.3.1.4	Abnormal cases on the network side

The following abnormal cases can be identified:

a)	T3590 expired

The SMF shall, on the first expiry of the timer T3590, retransmit the PDU SESSION AUTHENTICATION COMMAND message and shall reset and start timer T3590. This retransmission is repeated four times, i.e. on the fifth expiry of timer T3590, the SMF shall abort the procedure.

Editor's note:	Further abnormal cases are FFS

#### 6.3.1.5	Abnormal cases in the UE

Editor's note:	Abnormal cases are FFS

### 6.3.2	Network-requested PDU session modification procedure

#### 6.3.2.1	General

The purpose of the network-requested PDU session modification procedure is to enable the network to modify a PDU session.

Editor's note:	The solution for situation when the UE is in 5GMM-IDLE mode and the AMF decides to invoke asynchronous type communication is FFS.

#### 6.3.2.2	Network-requested PDU session modification procedure initiation

In order to initiate the network-requested PDU session modification procedure, the SMF shall create a PDU SESSION MODIFICATION COMMAND message.

If the authorized QoS rules of the PDU session is modified, the SMF shall set the authorized QoS rules IE of the PDU SESSION MODIFICATION COMMAND message to the authorized QoS rules of the PDU session.

If the session-AMBR of the PDU session is modified, the SMF shall set the selected Session-AMBR IE of the PDU SESSION MODIFICATION COMMAND message to the session-AMBR of the PDU session.

If the network-requested PDU session modification procedure is triggered by a UE-requested PDU session modification procedure, the SMF shall set the PTI IE of the PDU SESSION MODIFICATION COMMAND message to the PTI of the PDU SESSION MODIFICATION REQUEST message received as part of the UE-requested PDU session modification procedure.

If the network-requested PDU session modification procedure is not triggered by a UE-requested PDU session modification procedure, the SMF shall set the PTI IE of the PDU SESSION MODIFICATION COMMAND message to "No procedure transaction identity assigned".

If the selected SSC mode of the PDU session is "SSC mode 3" and the SMF requests the relocation of SSC mode 3 PDU session anchor with multiple PDU sessions as specified in 3GPP TS 23.502 [5], the SMF shall include 5GSM cause #39 "reactivation requested" , in the PDU SESSION MODIFICATION COMMAND message.

The SMF shall send the PDU SESSION MODIFICATION COMMAND message, and the SMF shall start timer T3591 (see example in figure 6.3.2.2.1).

NOTE:	If the SMF requests the relocation of SSC mode 3 PDU session anchor with multiple PDU sessions as specified in 3GPP TS 23.502 [5], the reallocation requested indication indicating whether the SMF is to be reallocated or the SMF is to be reused is provided to the AMF.

Figure 6.3.2.2.1: Network-requested PDU session modification procedure

#### 6.3.2.3	Network-requested PDU session modification procedure accepted by the UE

Upon receipt of the PDU SESSION MODIFICATION COMMAND message, if the UE provided a DNN for the establishment of the PDU session, the UE shall stop timer T35ab, if it is running for the DNN provided by the UE. If the UE did not provide a DNN for the establishment of the PDU session and the request type was different from "initial emergency request", the UE shall stop the timer T35ab associated with no DNN if it is running. If the MODIFY EPS BEARER CONTEXT REQUEST message was received for an emergency PDU session, the UE shall not stop the timer T35ab associated with no DNN if it is running.

Upon receipt of the PDU SESSION MODIFICATION COMMAND message, the UE shall stop timer T35cd associated with the same [S-NSSAI, DNN] combination as that the UE provided when the PDU session is established, if it is running.

Upon receipt of a PDU SESSION MODIFICATION COMMAND message and a PDU session ID, using the NAS transport procedure as specified in subclause 5.4.5, if the UE accepts the PDU SESSION MODIFICATION COMMAND message, the UE considers the PDU session as modified and the UE shall create a PDU SESSION MODIFICATION COMPLETE message.

If the PDU SESSION MODIFICATION COMMAND message contains the PTI value allocated in the UE-requested PDU session modification procedure, the UE shall release the PTI value indicated by the PTI IE and shall stop the timer T3581.

If the selected SSC mode of the PDU session is "SSC mode 3" and the PDU SESSION MODIFICATION COMMAND message includes 5GSM cause #39 "reactivation requested", the UE may re-initiate the UE-requested PDU session establishment procedure as specified in subclause 6.4.1 for the PDU session type, the SSC mode, the DNN, and the S-NSSAI if provided in the UE-requested PDU session establishment procedure of the present PDU session, after the completion of the network-requested PDU session modification procedure.

The UE shall transport the PDU SESSION MODIFICATION COMPLETE message and the PDU session ID, using the NAS transport procedure as specified in subclause 5.4.5.

Upon receipt of a PDU SESSION MODIFICATION COMPLETE message, the SMF shall stop timer T3591 and shall consider the PDU session as modified.

#### 6.3.2.4	Network-requested PDU session modification procedure not accepted by the UE

Upon receipt of a PDU SESSION MODIFICATION COMMAND message and a PDU session ID, using the NAS transport procedure as specified in subclause 5.4.5, if the UE rejects the PDU SESSION MODIFICATION COMMAND message, the UE considers the PDU session as not modified and the UE shall create a PDU SESSION MODIFICATION COMMAND REJECT message.

If the PDU SESSION MODIFICATION COMMAND message contains the PTI value allocated in the UE-requested PDU session modification procedure, the UE shall release the PTI indicated by the PTI IE and shall stop the timer T3581.

The UE shall set the 5GSM cause IE of the PDU SESSION MODIFICATION COMMAND REJECT message to indicate the reason for rejecting the PDU session modification.

If the selected SSC mode of the PDU session is "SSC mode 3" and the PDU SESSION MODIFICATION COMMAND messages includes 5GSM cause #39 "reactivation requested", while the UE does not have sufficient resources for initiating the PDU session establishment procedure as specified in subclause 6.4.1 then the UE shall set cause IE to #26 "insufficient resources".

The 5GSM cause IE typically indicates one of the following 5GSM cause values:

#26: "insufficient resources".

Editor's note:	Other 5GSM causes are FFS.

The UE shall transport the PDU SESSION MODIFICATION COMMAND REJECT message and the PDU session ID, using the NAS transport procedure as specified in subclause 5.4.5.

Upon receipt of a PDU SESSION MODIFICATION COMMAND REJECT message, the SMF shall stop timer T3591 and shall consider the PDU session as not modified.

#### 6.3.2.5	Abnormal cases on the network side

The following abnormal cases can be identified:

a)	Expiry of timer T3591

On the first expiry of the timer T3591.

the SMF shall resend the PDU SESSION MODIFICATION COMMAND message and shall reset and restart timer T3591. This retransmission is repeated four times, i.e. on the fifth expiry of timer T3591, the SMF shall abort the procedure and enter the state PDU SESSION ACTIVE.

The SMF may continue to use the previous configuration of the PDU session or initiate the network-requested PDU session release procedure.

Editor's note:	Further abnormal cases are FFS.

#### 6.3.2.6	Abnormal cases in the UE

Editor's note:	Abnormal cases are FFS.

### 6.3.3	Network-requested PDU session release procedure

#### 6.3.3.1	General

The purpose of the network-requested PDU session release procedure is to enable the network to release a PDU session.

#### 6.3.3.2	Network-requested PDU session release procedure initiation

In order to initiate the network-requested PDU session release procedure, the SMF shall create a PDU SESSION RELEASE COMMAND message.

The SMF shall set the SM cause IE of the PDU SESSION RELEASE COMMAND message to indicate the reason for releasing the PDU session.

The SM cause IE typically indicates one of the following SM cause values:

#26:	"insufficient resources";

#36:	regular deactivation; or

#39:	reactivation requested.

Editor's note:	Further 5GSM causes are FFS.

If the selected SSC mode of the PDU session is "SSC mode 2" and the SMF requests the relocation of SSC mode 2 PDU session anchor with different PDU sessions as specified in 3GPP TS 23.502 [5], the SMF shall include 5GSM cause #39 "reactivation requested".

If the network-requested PDU session release procedure is triggered by a UE-requested PDU session release procedure, the SMF shall set the PTI IE of the PDU SESSION RELEASE COMMAND message to the PTI of the PDU SESSION RELEASE REQUEST message received as part of the UE-requested PDU session release procedure.

If the network-requested PDU session release procedure is not triggered by a UE-requested PDU session release procedure, the SMF shall set the PTI IE of the PDU SESSION RELEASE COMMAND message to "No procedure transaction identity assigned".

The SMF may include a Back-off timer value IE in the PDU SESSION RELEASE COMMAND message when the 5GSM cause value #26"insufficient resources" is included in the PDU SESSION RELEASE COMMAND message. If the 5GSM cause value is #26 "insufficient resources" and the PDU SESSION RELEASE COMMAND message is sent to a UE configured to use AC11 – 15 in selected PLMN or the request type was set to "initial emergency request" for the establishment of the PDU session, the network shall not include a Back-off timer value IE.

Editor's note:	It is FFS whether both 5GSM cause value #26 and 5GSM cause values #ss are used or whether 5GSM cause value #26 together with an indication should be used instead.

The SMF shall send:

a)	the PDU SESSION RELEASE COMMAND message; and

b)	if the SMF allows the AMF to skip sending of the N1 SM container to the UE and the 5GSM cause IE is not set to #39 "reactivation requested", the N1 SM delivery skip allowed indication;

towards the AMF, and the SMF shall start timer T3592 (see example in figure 6.3.3.2.1).

Figure 6.3.3.2.1: Network-requested PDU session release procedure

#### 6.3.3.3	Network-requested PDU session release procedure accepted by the UE

Upon receipt of a PDU SESSION RELEASE COMMAND message and a PDU session ID, using the NAS transport procedure as specified in subclause 5.4.5, the UE considers the PDU session as released and the UE shall create an PDU SESSION RELEASE COMPLETE message.

If the PDU SESSION RELEASE COMMAND message contains the PTI value allocated in the UE-requested PDU session release procedure, the UE shall release the PTI value indicated by the PTI IE and shall stop the timer T3582.

If the PDU SESSION RELEASE COMMAND message includes 5GSM cause #39 "reactivation requested", the UE should re-initiate the UE-requested PDU session establishment procedure as specified in subclause 6.4.1 for the PDU session type, the SSC mode, the DNN, and the S-NSSAI as provided in the UE-requested PDU session establishment procedure of the released PDU session, after completion of the network-requested PDU session release procedure.

If the PDU SESSION RELEASE COMMAND message includes 5GSM cause #39 "reactivation requested" and the UE provided a DNN for the establishment of the PDU session, the UE shall stop timer T35ab if it is running for the DNN provided by the UE. The UE should then re-initiate the UE requested PDU session establishment procedure for the same DNN. If the UE did not provide a DNN for the establishment of the PDU session and the request type was different from "initial emergency request", the UE shall stop the timer T35ab associated with no DNN if it is running, and should re-initiate the UE requested PDU session establishment procedure without including a DNN. If the PDU SESSION RELEASE COMMAND message was received for an emergency PDU session, the UE shall not stop the timer T35ab associated with no DNN if it is running.

If the PDU SESSION RELEASE COMMAND message includes 5GSM cause #39 "reactivation requested", the UE shall stop timer T35cd if it is running for the same [S-NSSAI, DNN] combination provided by the UE. The UE should then re-initiate the UE requested PDU session establishment procedure for the same [S-NSSAI, DNN] combination.

NOTE:	User interaction is necessary in some cases when the UE cannot re-initiate the UE-requested PDU session establishment procedure automatically.

If the PDU SESSION RELEASE COMMAND message includes 5GSM cause #26 "insufficient resources" and the Back-off timer value IE, the UE shall take different actions depending on the timer value received for timer T35ab in the Back-off timer value:

a)	If the timer value indicates neither zero nor deactivated and a DNN was included in the PDU SESSION ESTABLISHMENT REQUEST message, the UE shall stop timer T35ab associated with the corresponding DNN, if it is running. If the timer value indicates neither zero nor deactivated and no DNN was included in the PDU SESSION ESTABLISHMENT REQUEST message and the request type was different from "initial emergency request", the UE shall stop timer T35ab associated with no DNN if it is running. The UE shall then start timer T35ab with the value provided in the Back-off timer value IE and:

1)	shall not send a PDU SESSION ESTABLISHMENT REQUEST or PDU SESSION MODIFICATION REQUEST message for the same DNN that was sent by the UE, until timer T35ab expires or timer T35ab is stopped; and

2)	shall not send a PDU SESSION ESTABLISHMENT REQUEST message without an DNN and with request type different from "initial emergency request", or a PDU SESSION MODIFICATION REQUEST message for a non-emergency PDU session established without an DNN provided by the UE, if no DNN was included in the PDU SESSION ESTABLISHMENT REQUEST message and the request type was different from "initial emergency request", until timer T35ab expires or timer T35ab is stopped.

The UE shall not stop timer T35ab upon a PLMN change or inter-system change;

b)	if the timer value indicates that this timer is deactivated and a DNN was included in the PDU SESSION ESTABLISHMENT REQUEST message, the UE shall stop timer T35ab associated with the corresponding DNN, if it is running. If the timer value indicates that this timer is deactivated and no DNN was included in the PDU SESSION ESTABLISHMENT REQUEST message and the request type was different from "initial emergency request", the UE shall stop timer T35ab associated with no DNN if it is running. The UE:

1)	shall not send a PDU SESSION ESTABLISHMENT REQUEST, or PDU SESSION MODIFICATION REQUEST for the same DNN until the UE is switched off or the USIM is removed, or the UE receives an PDU SESSION MODIFICATION REQUEST message for the same DNN from the network or a PDU SESSION RELEASE COMMAND message including 5GSM cause #39 "reactivation requested" for the same DNN from the network; and

2)	shall not send a PDU SESSION ESTABLISHMENT REQUEST message without an DNN and with request type different from "initial emergency request", or a PDU SESSION MODIFICATION REQUEST message for a non-emergency PDU session established without an DNN provided by the UE, if no DNN was included in the PDU SESSION ESTABLISHMENT REQUEST message and the request type was different from "initial emergency request", until the UE is switched off or the USIM is removed, or the UE receives an PDU SESSION MODIFICATION REQUEST message for a non-emergency PDU session established without an DNN provided by the UE, or a PDU SESSION RELEASE COMMAND messages including 5GSM cause IE set to 5GSM cause #39 "reactivation requested" for a non-emergency PDU session established without an DNN provided by the UE.

The timer T35ab remains deactivated upon a PLMN change or inter-system change; and

c)	if the timer value indicates zero, the UE:

1)	shall stop timer T35ab associated with the corresponding DNN, if running, and may send a PDU SESSION ESTABLISHMENT REQUEST, or PDU SESSION MODIFICATION REQUEST message for the same DNN; and

2)	if no DNN was included in the PDU SESSION ESTABLISHMENT REQUEST message and the request type was different from "initial emergency request ", the UE shall stop timer T35ab associated with no DNN, if running, and may send a PDU SESSION ESTABLISHMENT REQUEST message without a DNN, or a PDU SESSION MODIFICATION REQUEST message without an DNN provided by the UE.

If the PDU SESSION RELEASE COMMAND message includes 5GSM cause #26 "insufficient resources" and the Back-off timer value IE is not included, then the UE may send a PDU SESSION ESTABLISHMENT REQUEST or PDU SESSION MODIFICATION REQUEST message for the same DNN or without a DNN.

When the timer T35ab is running or the timer is deactivated, the UE is allowed to initiate a PDU session establishment procedure for emergency services.

If the timer T35ab is running when the UE enters state 5GMM-DEREGISTERED, the UE remains switched on, and the USIM in the UE remains the same, then timer T35ab is kept running until it expires or it is stopped.

If the UE is switched off when the timer T35ab is running, and if the USIM in the UE remains the same when the UE is switched on, the UE shall behave as follows:

-	let t1 be the time remaining for T35ab timeout at switch off and let t be the time elapsed between switch off and switch on. If t1 is greater than t, then the timer shall be restarted with the value t1 – t. If t1 is equal to or less than t, then the timer need not be restarted. If the UE is not capable of determining t, then the UE shall restart the timer with the value t1.

The UE shall transport the PDU SESSION RELEASE COMPLETE message and the PDU session ID, using the NAS transport procedure as specified in subclause 5.4.5.

Upon receipt of a PDU SESSION RELEASE COMPLETE message, the SMF shall stop timer T3592 and shall consider the PDU session as released.

#### 6.3.3.4	N1 SM delivery skipped

If the PDU SESSION RELEASE COMMAND message was sent along the N1 SM delivery skip allowed indication towards AMF, then upon receipt of an indication that N1 SM delivery was skipped, the SMF shall stop timer T3592 and shall consider the PDU session as released.

#### 6.3.3.5	Abnormal cases on the network side

The following abnormal cases can be identified:

a)	Expiry of timer T3592

The SMF shall, on the first expiry of the timer T3592, retransmit the PDU SESSION RELEASE COMMAND message and shall reset and start timer T3592. This retransmission is repeated four times, i.e. on the fifth expiry of timer T3592, the SMF shall abort the procedure.

Editors' note:	Further abnormal cases are FFS.

#### 6.3.3.6	Abnormal cases in the UE

Editors' note:	Abnormal cases are FFS.

## 6.4	UE-requested 5GSM procedures

### 6.4.1	UE-requested PDU session establishment procedure

#### 6.4.1.1	General

The purpose of the UE-requested PDU session establishment procedure is to establish a new PDU session with a DN or to perform handover of an existing PDU session between 3GPP access and non-3GPP access or to transfer an existing PDN connection in the EPS to the 5GS. If accepted by the network, the PDU session enables exchange of PDUs between the UE and the DN. The UE shall not request a PDU session establishment for an LADN when the UE is located outside the LADN service area.

#### 6.4.1.2	UE-requested PDU session establishment procedure initiation

In order to initiate the UE-requested PDU session establishment procedure, the UE shall create a PDU SESSION ESTABLISHMENT REQUEST message.

The UE shall allocate a PTI value currently not used and shall set the PTI IE of the PDU SESSION ESTABLISHMENT REQUEST message to the allocated PTI value.

If the UE requests to establish a new non-emergency PDU session with a DN and requests a PDU session type, the UE shall set the PDU session type IE of the PDU SESSION ESTABLISHMENT REQUEST message to one of the following values: the IP version capability as specified in subclause 6.2.4.2, "Ethernet" or "Unstructured".

NOTE:	When the UE initiates the UE-requested PDU session establishment procedure to transfer an existing non-IP PDN connection in the EPS to the 5GS, the UE can use locally available information associated with the PDN connection to select the PDU session type between "Ethernet" and "Unstructured".

If the UE requests to establish a new non-emergency PDU session with a DN and the UE requests an SSC mode, the UE shall set the SSC mode IE of the PDU SESSION ESTABLISHMENT REQUEST message to the SSC mode.

If the UE requests to establish a new PDU session with a DN, the UE may include the SM PDU DN request container with a DN-specific identity of the UE complying with network access identifier (NAI) format as specified in IETF RFC 4282 [21].

If the UE requests to establish a new PDU session of "IP", "IPv4", "IPv6" or "Ethernet" PDU session type and the UE supports reflective QoS, the UE shall set the RQoS bit to "Reflective QoS supported" in the 5GSM capability IE of the PDU SESSION ESTABLISHMENT REQUEST message.

If the UE has an emergency PDU session, the UE shall not perform the UE-requested PDU session establishment procedure to establish another emergency PDU session. The UE may perform the UE-requested PDU session establishment procedure to transfer an existing emergency PDU session or an existing PDN connection for emergency bearer services.

Upon intersystem change from S1 mode to N1 mode, when the interworking with N26 interface is not supported, if the UE wants to establish a PDU session corresponding to an existing PDN connection to EPS, the UE shall:

-	set the PDU session ID in the PDU SESSION ESTABLISHMENT REQUEST message and in the UL NAS TRANSPORT message to the stored PDU session ID corresponding to the PDN connection; and

-	set the S-NSSAI in the UL NAS TRANSPORT message to the stored S-NSSAI associated with the PDU session ID.

The UE shall transport:

a)	the PDU SESSION ESTABLISHMENT REQUEST message;

b)	the PDU session ID of the PDU session being established;

c)	if the request type is set to:

1)	"initial request" and the UE determined to establish a new PDU session based on a URSP rule including an S-NSSAI in the URSP (see subclause 6.2.x), an S-NSSAI and, if available in roaming scenarios, a mapped configured S-NSSAI from the configured NSSAI for the HPLMN;

2)	"existing PDU session", an S-NSSAI, which is an S-NSSAI associated with the PDU session and, if available in roaming scenarios, a mapped configured S-NSSAI from the configured NSSAI for the HPLMN;

d)	the requested DNN, if the request type is not set to "initial emergency request" and the UE requests a connectivity to a DNN other than the default DNN;

e)	the request type which is set to:

1)	"initial request", if the UE requests to establish a new non-emergency PDU session;

2)	"existing PDU session", if the UE requests:

i)	handover of an existing non-emergency PDU session between 3GPP access and non-3GPP access; or

ii)	transfer of an existing PDN connection for non-emergency bearer services in the EPS to the 5GS;

3)	"initial emergency request", if the UE requests to establish a new emergency PDU session; and

4)	"existing emergency PDU session", if the UE requests:

i)	handover of an existing emergency PDU session between 3GPP access and non-3GPP access; or

ii)	transfer of an existing PDN connection for emergency bearer services in the EPS to the 5GS; and

f)	the old PDU session ID which is the PDU session ID of the existing PDU session, if the UE initiates the UE-requested PDU session establishment procedure upon receiving the PDU SESSION MODIFICATION COMMAND messages with the 5GSM cause IE set to #39 "reactivation requested";

using the NAS transport procedure as specified in subclause 5.4.5, and the UE shall start timer T3580 (see example in figure 6.4.1.2.1).

Figure 6.4.1.2.1: UE-requested PDU session establishment procedure

Upon receipt of a PDU SESSION ESTABLISHMENT REQUEST message, a PDU session ID, optionally a S-NSSAI, optionally a DNN, the request type, and optionally an old PDU session ID, the SMF checks whether connectivity with the requested DN can be established. If the requested DNN is not included, the SMF shall use the default DNN.

If the PDU session being established is a non-emergency PDU session, the request type is not set to "existing PDU session" and the PDU session authentication and authorization by the external DN is required due to local policy, the SMF shall check whether the PDU SESSION ESTABLISHMENT REQUEST message includes the PDU DN request container.

If the PDU session being established is a non-emergency PDU session, the request type is not set to "existing PDU session", the PDU DN request container is included in the PDU SESSION ESTABLISHMENT REQUEST message and the PDU session authentication and authorization by the external DN is required due to local policy and user's subscription data, the SMF shall:

a)	if the information for the PDU session authentication and authorization by the external DN in PDU DN request container is compliant with the local policy and user's subscription data, proceed with the EAP Authentication procedure specified in 3GPP TS 33.501 [12] and refrain from accepting or rejecting the PDU SESSION ESTABLISHMENT REQUEST message until the EAP Authentication procedure finalizes; and

b)	if the information for the PDU session authentication and authorization by the external DN in PDU DN request container is not compliant with the local policy, reject the PDU session establishment request including the 5GSM cause #29 "user authentication failed", in the PDU SESSION ESTABLISHMENT REJECT message.

If the PDU session being established is a non-emergency PDU session, the request type is not set to "existing PDU session", the PDU DN request container is not included in the PDU SESSION ESTABLISHMENT REQUEST message and the PDU session authentication and authorization by the external DN is required due to local policy and user's subscription data, the SMF shall proceed with the EAP Authentication procedure specified in 3GPP TS 33.501 [12] and refrain from accepting or rejecting the PDU SESSION ESTABLISHMENT REQUEST message until the EAP Authentication procedure finalizes.

If the SMF receives the old PDU session ID from the AMF and a PDU session exists for the old PDU session ID, the SMF shall consider that the request for the relocation of SSC mode 3 PDU session anchor with multiple PDU sessions as specified in 3GPP TS 23.502 [5] is accepted by the UE.

#### 6.4.1.3	UE-requested PDU session establishment procedure accepted by the network

If the connectivity with the requested DN is accepted by the network, the SMF shall create a PDU SESSION ESTABLISHMENT ACCEPT message.

The SMF shall set the authorized QoS rules IE of the PDU SESSION ESTABLISHMENT ACCEPT message to the authorized QoS rules of the PDU session. If the received request type is "initial emergency request", the SMF shall set the authorized QoS rules IE according to the initial QoS parameters used for establishing emergency services configured in the SMF Emergency Configuration Data.

The SMF shall set the selected SSC mode IE of the PDU SESSION ESTABLISHMENT ACCEPT message to:

-	either the received SSC mode or the SSC mode modified from the received SSC mode based on the subscription, the SMF configuration, or both if the request type IE is set to "initial request", and the SSC mode IE is included in the PDU SESSION ESTABLISHMENT REQUEST message;

-	otherwise, either the default SSC mode for the data network listed in the subscription or the SSC mode associated with the SMF configuration.

If the PDU session is an emergency PDU session, the SMF shall set the selected SSC mode IE of the PDU SESSION ESTABLISHMENT ACCEPT message to "SSC mode 1".

The SMF shall set the S-NSSAI IE of the PDU SESSION ESTABLISHMENT ACCEPT message to the selected S-NSSAI of the PDU session.

The SMF shall set the selected PDU session type IE of the PDU SESSION ESTABLISHMENT ACCEPT message to the PDU session type of the PDU session.

If the PDU SESSION ESTABLISHMENT REQUEST message includes a PDU session type IE set to "IP", the SMF shall select either "IPv4" or "IPv6" as the selected PDU session type IE of the PDU session. If the subscription, the SMF configuration, or both, are limited to IPv4 only or IPv6 only for the requested DNN, the SMF shall include the 5GSM cause value #50 "PDU session type IPv4 only allowed", or #51 "PDU session type IPv6 only allowed", respectively, in the 5GSM cause IE of the PDU SESSION ESTABLISHMENT ACCEPT message.

If the selected PDU session type is "IPv4", the SMF shall include the PDU address IE in the PDU SESSION ESTABLISHMENT ACCEPT message and shall set the PDU address IE to an IPv4 address is allocated to the UE in the PDU session.

If the selected PDU session type is "IPv6", the SMF shall include the PDU address IE in the PDU SESSION ESTABLISHMENT ACCEPT message and shall set the PDU address IE to an interface identifier for the IPv6 link local address.

The SMF shall set the DNN IE of the PDU SESSION ESTABLISHMENT ACCEPT message to the DNN of the PDU session.

The SMF shall set the Session-AMBR IE of the PDU SESSION ESTABLISHMENT ACCEPT message to the Session-AMBR of the PDU session.

If the selected PDU session type is "IPv4", "IPv6" or "Ethernet" and if the PDU SESSION ESTABLISHMENT REQUEST message includes a 5GSM capability IE with the RQoS bit set to "Reflective QoS supported", the SMF shall consider that reflective QoS is supported for QoS flows belonging to this PDU session and include the RQ timer IE set to an RQ timer value.

NOTE:	If the 5G core network determines that reflective QoS is to be used for a QoS flow, the SMF sends reflective QoS indication (RQI) to UPF to activate reflective QoS. If the QoS flow is established over 3GPP access, the SMF also includes reflective QoS Attribute (RQA) in QoS profile of the QoS flow during QoS flow establishment.

If the DN authentication of the UE was performed and completed successfully, the SMF shall set the EAP message IE of the PDU SESSION ESTABLISHMENT ACCEPT message to an EAP-success message as specified in IETF RFC 3748 [19], provided by the DN.

The SMF shall send the PDU SESSION ESTABLISHMENT ACCEPT message.

Upon receipt of a PDU SESSION ESTABLISHMENT ACCEPT message and a PDU session ID, using the NAS transport procedure as specified in subclause 5.4.5, the UE shall stop timer T3580, shall release the allocated PTI value and shall consider that the PDU session was established.

If the UE requests the PDU session type "IP" and:

a)	the UE receives the selected PDU session type set to "IPv4" and does not receive the 5GSM cause value #50 "PDU session type IPv4 only allowed"; or

b)	 the UE receives the selected PDU session type set to "IPv6" and does not receive the 5GSM cause value #51 "PDU session type IPv6 only allowed";

the UE may subsequently request another PDU session for the other IP version using the UE-requested PDU session establishment procedure to the same DNN with a single address PDN type (IPv4 or IPv6) other than the one already activated.

If the UE requests the PDU session type "IP", receives the selected PDU session type set to "IPv4" and the 5GSM cause value #50 "PDU session type IPv4 only allowed", the UE shall not subsequently request another PDU session for "IPv6" using the UE-requested PDU session establishment procedure to the same DNN and the PDU session type "IPv6" until the PDU session is released.

If the UE requests the PDU session type "IP", receives the selected PDU session type set to "IPv6" and the 5GSM cause value #51 "PDU session type IPv6 only allowed", the UE shall not subsequently request another PDU session for "IPv4" using the UE-requested PDU session establishment procedure to the same DNN and the PDU session type "IPv4" until the PDU session is released.

#### 6.4.1.4	UE requested PDU session establishment procedure not accepted by the network

##### 6.4.1.4.1	General

If the connectivity with the requested DN is rejected by the network, the SMF shall create a SM PDU SESSION ESTABLISHMENT REJECT message.

If the UE requests a PDU session establishment for an LADN when the UE is located outside the LADN service area, the SMF shall reject the request.

The SMF shall set the 5GSM cause IE of the PDU SESSION ESTABLISHMENT REJECT message to indicate the reason for rejecting the PDU session establishment.

The 5GSM cause IE typically indicates one of the following SM cause values:

#26:	insufficient resources;

#27:	missing or unknown DNN;

#29:	user authentication failed;

#31:	request rejected, unspecified;

#50	PDU session type IPv4 only allowed;

#51	PDU session type IPv6 only allowed;

#ss		insufficient resources for specific slice and DNN; or

#zz:	not supported SSC mode.

Editor's note:	Further 5GSM causes are FFS.

Editor's note:	Further contents of the PDU SESSION ESTABLISHMENT REJECT are FFS.

If the PDU SESSION ESTABLISHMENT REQUEST message includes a PDU session type IE set to "IPv6", and the subscription, the SMF configuration, or both, are limited to IPv4 only for the requested DNN, the SMF shall include the 5GSM cause value #50 "PDU session type IPv4 only allowed" in the 5GSM cause IE of the PDU SESSION ESTABLISHMENT REJECT message.

If the PDU SESSION ESTABLISHMENT REQUEST message includes a PDU session type IE set to "IPv4", and the subscription, the SMF configuration, or both, are limited to IPv6 only for the requested DNN, the SMF shall include the 5GSM cause value #51 "PDU session type IPv6 only allowed" in the 5GSM cause IE of the PDU SESSION ESTABLISHMENT REJECT message.

If the PDU SESSION ESTABLISHMENT REQUEST message contains the SSC mode IE indicating an SSC mode not supported by the subscription, the SMF configuration, or both of them, and the SMF decides to rejects the PDU session establishment, the SMF shall include the 5GSM cause value #zz "not supported SSC mode" in the 5GSM cause IE of the PDU SESSION ESTABLISHMENT REJECT message.

The SMF may include a Back-off timer value IE in the PDU SESSION ESTABLISHMENT REJECT message when the 5GSM cause value #26"insufficient resources" is included in the PDU SESSION ESTABLISHMENT REJECT message. If the 5GSM cause value is  #26"insufficient resources" and the PDU SESSION ESTABLISHMENT REQUEST message was received from a UE configured to use AC11 – 15 in selected PLMN or the request type in the PDU SESSION ESTABLISHMENT REQUEST message was set to "initial emergency request", the network shall not include a Back-off timer value IE.

Editor's note	It is FFS whether both 5GSM cause value #26 and 5GSM cause values #ss are used or whether 5GSM cause value #26 together with an indication should be used instead.

The SMF may include a Back-off timer value IE in the PDU SESSION ESTABLISHMENT REJECT message when the 5GSM cause cause #ss "insufficient resources for specific slice and DNN" is included in the PDU SESSION ESTABLISHMENT REJECT message. If the 5GSM cause value is #ss "insufficient resources for specific slice and DNN" and the PDU SESSION ESTABLISHMENT REQUEST message was received from a UE configured to use AC11 – 15 in selected PLMN or the request type is "initial emergency request" in the PDU SESSION ESTABLISHMENT REQUEST message, the network shall not include a Back-off timer value IE.

Editor's note:	It is FFS whether both 5GSM cause value #26 and 5GSM cause values #ss are used or whether 5GSM cause value #26 together with an indication should be used instead.

The SMF shall send the SM PDU SESSION ESTABLISHMENT REJECT message.

Upon receipt of a PDU SESSION ESTABLISHMENT REJECT message and a PDU session ID, using the NAS transport procedure as specified in subclause 5.4.5, the UE shall stop timer T3580 shall release the allocated PTI value and shall consider that the PDU session was not established.

If the DN authentication of the UE was performed and completed unsuccessfully, the SMF shall include the 5GSM cause value #xx "DN authentication failed" in the 5GSM cause IE of the PDU SESSION ESTABLISHMENT REJECT message and shall set the EAP message IE of the PDU SESSION ESTABLISHMENT REJECT message to an EAP-failure message as specified in IETF RFC 3748 [19], provided by the DN.

If the 5GSM cause value is #26 "insufficient resources" and the Back-off timer value IE is included, the UE shall take different actions depending on the timer value received for timer T35ab in the Back-off timer value:

a)	If the timer value indicates neither zero nor deactivated and a DNN was included in the PDU SESSION ESTABLISHMENT REQUEST message, the UE shall stop timer T35ab associated with the corresponding DNN, if it is running. If the timer value indicates neither zero nor deactivated and no DNN was included in the PDU SESSION ESTABLISHMENT REQUEST message and the request type was different from "initial emergency request", the UE shall stop timer T35ab associated with no DNN if it is running. The UE shall then start timer T35ab with the value provided in the Back-off timer value IE and:

1)	shall not send another PDU SESSION ESTABLISHMENT REQUEST message,or PDU SESSION MODIFICATION REQUEST message for the same DNN that was sent by the UE, until timer T35ab expires or timer T35ab is stopped; and

2)	shall not send another PDU SESSION ESTABLISHMENT REQUEST message without a DNN and with request type different from "initial emergency request", or another PDU SESSION MODIFICATION REQUEST message for a non-emergency PDU session established without a DNN provided by the UE, if no DNN was included in the PDU SESSION ESTABLISHMENT REQUEST message and the request type was different from "initial emergency request", until timer T35ab expires or timer T35ab is stopped.

The UE shall not stop timer T35ab upon a PLMN change or inter-system change;

b)	if the timer value indicates that this timer is deactivated and a DNN was included in the PDU SESSION ESTABLISHMENT REQUEST message, the UE shall stop timer T35ab associated with the corresponding DNN, if it is running. If the timer value indicates that this timer is deactivated and no DNN was included in the PDU SESSION ESTABLISHMENT REQUEST message and the request type was different from "initial emergency request", the UE shall stop timer T35ab associated with no DNN if it is running. The UE:

1)	shall not send another PDU SESSION ESTABLISHMENT REQUEST, or PDU SESSION MODIFICATION REQUEST for the same DNN until the UE is switched off or the USIM is removed, or the UE receives a PDU SESSION MODIFICATION REQUEST message for the same DNN from the network or a PDU SESSION RELEASE COMMAND message including 5GSM cause #39 "reactivation requested" for the same DNN from the network; and

2)	shall not send another PDU SESSION ESTABLISHMENT REQUEST message without a DNN and with request type different from "initial emergency request", or another PDU SESSION MODIFICATION REQUEST message for a non-emergency PDU session established without a DNN provided by the UE, if no DNN was included in the PDU SESSION ESTABLISHMENT REQUEST message and the request type was different from "initial emergency request", until the UE is switched off or the USIM is removed, or the UE receives a PDU SESSION MODIFICATION REQUEST message for a non-emergency PDU session established without a DNN provided by the UE, or a PDU SESSION RELEASE COMMAND messages including 5GSM cause IE set to 5GSM cause #39 "reactivation requested" for a non-emergency PDU session established without a DNN provided by the UE.

The timer T35ab remains deactivated upon a PLMN change or inter-system change; and

c)	if the timer value indicates zero, the UE:

1)	shall stop timer T35ab associated with the corresponding DNN, if running, and may send another PDU SESSION ESTABLISHMENT REQUEST, or PDU SESSION MODIFICATION REQUEST message for the same DNN; and

2)	if no DNN was included in the PDU SESSION ESTABLISHMENT REQUEST message and the request type was different from "initial emergency request ", the UE shall stop timer T35ab associated with no DNN, if running, and may send another PDU SESSION ESTABLISHMENT REQUEST message without a DNN, or another PDU SESSION MODIFICATION REQUEST message without a DNN provided by the UE.

If the Back-off timer value IE is not included, then the UE may send another PDU SESSION ESTABLISHMENT REQUEST or PDU SESSION MODIFICATION REQUEST message for the same DNN or without a DNN.

When the timer T35ab is running or the timer is deactivated, the UE is allowed to initiate a PDU session establishment procedure for emergency services.

If the timer T35ab is running when the UE enters state5GMM-DEREGISTERED, the UE remains switched on, and the USIM in the UE remains the same, then timer T35ab is kept running until it expires or it is stopped.

If the UE is switched off when the timer T35ab is running, and if the USIM in the UE remains the same when the UE is switched on, the UE shall behave as follows:

-	let t1 be the time remaining for T35ab timeout at switch off and let t be the time elapsed between switch off and switch on. If t1 is greater than t, then the timer shall be restarted with the value t1 – t. If t1 is equal to or less than t, then the timer need not be restarted. If the UE is not capable of determining t, then the UE shall restart the timer with the value t1.

If the 5GSM cause value is #ss "insufficient resources for specific slice and DNN" and the Back-off timer value IE is included, the UE shall take different actions depending on the timer value received for timer T35cd in the Back-off timer value:

a)	If the timer value indicates neither zero nor deactivated, the UE shall stop timer T35cd associated with the same [S-NSSAI, DNN] combination as that the UE provided when the PDU session is established, if it is running. The UE shall then start timer T35cd with the value provided in the Back-off timer value IE. The UE shall not send another PDU SESSION ESTABLISHMENT REQUEST message with request type different from "initial emergency request", or PDU SESSION MODIFICATION REQUEST message for the same [S-NSSAI, DNN] combination that was sent by the UE, until timer T35cd expires or timer T35cd is stopped;

The UE shall not stop timer T35cd upon a PLMN change or inter-system change;

b)	if the timer value indicates that this timer is deactivated, the UE shall stop timer T35cd associated with the same [S-NSSAI, DNN] combination as that the UE provided when the PDU session is established, if it is running. The UE shall not send another PDU SESSION ESTABLISHMENT REQUEST message with request type different from "initial emergency request", or PDU SESSION MODIFICATION REQUEST message for the same [S-NSSAI, DNN] combination that was sent by the UE, until the UE is switched off or the USIM is removed, or the UE receives a PDU SESSION MODIFICATION REQUEST message for the same [S-NSSAI, DNN] combination from the network or a PDU SESSION RELEASE COMMAND message including 5GSM cause #39 "reactivation requested" for the same [S-NSSAI, DNN] combination from the network; and

The timer T35cd remains deactivated upon a PLMN change or inter-system change; and

c)	if the timer value indicates zero, the UE shall stop timer T35cd associated with the same [S-NSSAI, DNN] combination that was sent by the UE, if running, and may send another PDU SESSION ESTABLISHMENT REQUEST message, or PDU SESSION MODIFICATION REQUEST message for the same [S-NSSAI, DNN] combination.

If the Back-off timer value IE is not included, then the UE may send another PDU SESSION ESTABLISHMENT REQUEST message or PDU SESSION MODIFICATION REQUEST message for the same [S-NSSAI, DNN] combination.

When the timer T35cd is running or the timer is deactivated, the UE is allowed to initiate a PDU session establishment procedure for emergency services.

If the timer T35cd is running when the UE enters state5GMM-DEREGISTERED, the UE remains switched on, and the USIM in the UE remains the same, then timer T35cd is kept running until it expires or it is stopped.

If the UE is switched off when the timer T35cd is running, and if the USIM in the UE remains the same when the UE is switched on, the UE shall behave as follows:

-	let t1 be the time remaining for T35cd timeout at switch off and let t be the time elapsed between switch off and switch on. If t1 is greater than t, then the timer shall be restarted with the value t1 – t. If t1 is equal to or less than t, then the timer need not be restarted. If the UE is not capable of determining t, then the UE shall restart the timer with the value t1.

##### 6.4.1.4.2	Handling of network rejection due to 5GSM cause #26

Editor's note:	Handling of network rejection due to 5GSM cause #26 is FFS.

##### 6.4.1.4.3	Handling of network rejection due to 5GSM cause other than 5GSM cause #26

If the 5GSM cause value is #27 "missing or unknown DNN", the UE shall not send another PDU SESSION ESTABLISHMENT REQUEST message in the PLMN for the same DNN that was sent by the UE, until the UE is switched off, the USIM is removed, or the network updates the LADN information during the registration procedure or the generic UE configuration update procedure.

Editor's note:	Handling of network rejection due to 5GSM cause other than 5GSM cause #27 "missing or unknown DNN" is FFS.

#### 6.4.1.5	Abnormal cases in the UE

The following abnormal cases can be identified:

a)	Expiry of timer T3580

The UE shall, on the first expiry of the timer T3580, retransmit the PDU SESSION ESTABLISHMENT REQUEST message and shall reset and start timer T3580. This retransmission is repeated four times, i.e. on the fifth expiry of timer T3580, the UE shall abort the procedure.

b)	Upon receiving an indication that the 5GSM message cannot be routed along with a PDU SESSION ESTABLISHMENT REQUEST message with the PDU session ID IE set to the PDU session ID of the PDU session, the UE shall stop timer T3580 and shall abort the procedure.

Editor's note:	Further abnormal cases in the UE are FFS.

#### 6.4.1.6	Abnormal cases on the network side

Editor's note:	Further abnormal cases in the network side are FFS.

The following abnormal cases can be identified:

a)	If the received request type is "initial emergency request" and there is already another emergency PDU session for the UE, the SMF shall reject the PDU SESSION ESTABLISHMENT REQUEST message with ESM cause #31 "request rejected, unspecified" or release the existing emergency PDU session locally without notification to the UE and proceed the new PDU SESSION ESTABLISHMENT REQUEST message

b)	The information for the PDU session authentication and authorization by the external DN in PDU DN request container is not compliant with local policy and user's subscription data

If the PDU session being established is a non-emergency PDU session, the PDU session authentication and authorization by the external DN is required due to local policy and user's subscription data and the information for the PDU session authentication and authorization by the external DN in PDU DN request container is not compliant with local policy and user's subscription data, the SMF shall reject the PDU session establishment request including the 5GSM cause #29 "user authentication failed", in the PDU SESSION ESTABLISHMENT REJECT message.

### 6.4.2	UE-requested PDU session modification procedure

#### 6.4.2.1	General

The purpose of the UE-requested PDU session modification procedure is to enable the UE to request modification of a PDU session. The UE shall not request a PDU session modification for an LADN when the UE is located outside the LADN service area. If the UE supports Reflective QoS, the UE shall initiate this procedure to indicate the support of Reflective QoS after an inter-system change from S1 mode to N1 mode.

#### 6.4.2.2	UE-requested PDU session modification procedure initiation

In order to initiate the UE-requested PDU session modification procedure, the UE shall create a PDU SESSION MODIFICATION REQUEST message.

The UE shall allocate a PTI value currently not used and shall set the PTI IE of the PDU SESSION MODIFICATION REQUEST message to the allocated PTI value.

The UE shall not perform the UE-requested PDU session modification procedure for an emergency PDU session.

The UE shall not perform the UE-requested PDU session modification procedure for a PDU session for LADN when the UE is located outside the LADN service area.

If the UE requests a specific QoS handling, the UE shall include the requested QoS rules IE indicating requested QoS rules for the specific QoS handling.

The UE shall transport the PDU SESSION MODIFICATION REQUEST message and the PDU session ID, using the NAS transport procedure as specified in subclause 5.4.5, and the UE shall start timer T3581 (see example in figure 6.4.2.2.1).

Figure 6.4.2.2.1: UE-requested PDU session modification procedure

#### 6.4.2.3	UE-requested PDU session modification procedure accepted by the network

Upon receipt of a PDU SESSION MODIFICATION REQUEST message, if the SMF accepts the request to modify the PDU session, the SMF shall perform the network-requested PDU session modification procedure as specified in subclause 6.3.2.

#### 6.4.2.4	UE-requested PDU session modification procedure not accepted by the network

Upon receipt of a PDU SESSION MODIFICATION REQUEST message, if the SMF does not accepts the request to modify the PDU session, the SMF shall create a PDU SESSION MODIFICATION REJECT message.

If the UE requests a PDU session modification for an LADN when the UE is located outside the LADN service area, the SMF shall reject the request.

The SMF shall set the 5GSM cause IE of the PDU SESSION MODIFICATION REJECT message to indicate the reason for rejecting the PDU session modification.

The 5GSM cause IE typically indicates one of the following SM cause values:

#26:	insufficient resources;

#31:	request rejected, unspecified; or

#ss:	insufficient resources for specific slice and DNN;

Editor's note:	Further 5GSM causes are FFS.

The SMF may include a Back-off timer value IE in the PDU SESSION MODIFICATION REJECT message. If the 5GSM cause value is #26 "insufficient resources" and the PDU SESSION MODIFICATION REQUEST message was received from a UE configured to use AC11 – 15 in selected PLMN or the request type was set to "initial emergency request" for the establishment of the PDU session, the network shall not include a Back-off timer value IE.

Editor's note:	It is FFS whether both 5GSM cause value #26 and 5GSM cause values #ss are used or whether 5GSM cause value #26 together with an indication should be used instead.

The SMF may include a Back-off timer value IE in the PDU SESSION MODIFICATION REJECT message when the 5GSM cause cause #ss "insufficient resources for specific slice and DNN" is included in the PDU SESSION MODIFICATION REJECT message. If the 5GSM cause value is #ss "insufficient resources for specific slice and DNN" and the PDU SESSION MODIFICATION REQUEST message was received from a UE configured to use AC11 – 15 in selected PLMN or the request type is set to "initial emergency request" for the establishment of the PDU session, the network shall not include a Back-off timer value IE.

Editor's note:	It is FFS whether both 5GSM cause value #26 and 5GSM cause values #ss are used or whether 5GSM cause value #26 together with an indication should be used instead.

The SMF shall send the PDU SESSION MODIFICATION REJECT message.

Upon receipt of a PDU SESSION MODIFICATION REJECT message and a PDU session ID, using the NAS transport procedure as specified in subclause 5.4.5, the UE shall stop timer T3581, shall release the allocated PTI value and shall consider that the PDU session is not modified.

If the 5GSM cause value is #26 "insufficient resources" and the Back-off timer value IE is included, the UE shall take different actions depending on the timer value received for timer T35ab in the Back-off timer value:

a)	If the timer value indicates neither zero nor deactivated, the UE shall stop timer T35ab associated with the corresponding DNN, if it is running. The UE shall then start timer T35ab with the value provided in the Back-off timer value IE and shall not send another PDU SESSION ESTABLISHMENT REQUEST message, or PDU SESSION MODIFICATION REQUEST message for the same DNN that was sent by the UE, until timer T35ab expires or timer T35ab is stopped.

If the UE did not provide a DNN for the establishment of the PDU session and the request type was different from "initial emergency request", the UE shall stop the timer T35ab associated with no DNN if it is running. The UE shall start timer T35ab with the received value and not send another PDU SESSION ESTABLISHMENT REQUEST message without a DNN and with request type different from "initial emergency request", or another PDU SESSION MODIFICATION REQUEST message for a non-emergency PDU session established without a DNN provided by the UE, until timer T35ab expires or timer T35ab is stopped.

The UE shall not stop timer T35ab upon a PLMN change or inter-system change.

b)	if the timer value indicates that this timer is deactivated, the UE shall not send another PDU SESSION ESTABLISHMENT REQUEST, or PDU SESSION MODIFICATION REQUEST message for the same DNN until the UE is switched off or the USIM is removed, or the UE receives a PDU SESSION MODIFICATION REQUEST message for the same DNN from the network or a PDU SESSION RELEASE COMMAND message including 5GSM cause #39 "reactivation requested" for the same DNN from the network.

If the UE did not provide a DNN for the establishment of the PDU session and the request type was different from "initial emergency request", the UE shall not send another PDU SESSION ESTABLISHMENT REQUEST message without a DNN and with request type different from "initial emergency request", or another PDU SESSION MODIFICATION REQUEST for a non-emergency PDU session established without DNN provided by the UE, until the UE is switched off or the USIM is removed, or the UE receives a PDU SESSION MODIFICATION REQUEST message for a non-emergency PDU session established without a DNN provided by the UE, or the UE receives a PDU SESSION RELEASE COMMAND messages including 5GSM cause IE set to 5GSM cause #39 "reactivation requested" for a non-emergency PDU session established without a DNN provided by the UE.

The timer T35ab remains deactivated upon a PLMN change or inter-system change.

c)	if the timer value indicates zero, the UE shall stop timer T35ab associated with the corresponding DNN, if running, and may send another PDU SESSION ESTABLISHMENT REQUEST, or PDU SESSION MODIFICATION REQUEST message for the same DNN.

if the UE did not provide a DNN for the establishment of the PDU session and the request type was different from " initial emergency request ", the UE shall stop timer T35ab associated with no DNN, if running, and may send another PDU SESSION ESTABLISHMENT REQUEST message without a DNN, or another PDU SESSION MODIFICATION REQUEST message established without a DNN provided by the UE.

If the Back-off timer value IE is not included, then the UE may send another PDU SESSION ESTABLISHMENT REQUEST or PDU SESSION MODIFICATION REQUEST message for the same DNN or without DNN.

If the timer T35ab is running when the UE enters state5GMM-DEREGISTERED, the UE remains switched on, and the USIM in the UE remains the same, then timer T35ab is kept running until it expires or it is stopped.

If the UE is switched off when the timer T35ab is running, and if the USIM in the UE remains the same when the UE is switched on, the UE shall behave as follows:

-	let t1 be the time remaining for T35ab timeout at switch off and let t be the time elapsed between switch off and switch on. If t1 is greater than t, then the timer shall be restarted with the value t1 – t. If t1 is equal to or less than t, then the timer need not be restarted. If the UE is not capable of determining t, then the UE shall restart the timer with the value t1.

If the 5GSM cause value is #ss "insufficient resources for specific slice and DNN" and the Back-off timer value IE is included, the UE shall take different actions depending on the timer value received for timer T35cd in the Back-off timer value:

a)	If the timer value indicates neither zero nor deactivated, the UE shall stop timer T35cd associated with the same [S-NSSAI, DNN] combination as that the UE provided when the PDU session is established, if it is running. The UE shall then start timer T35cd with the value provided in the Back-off timer value IE and shall not send another PDU SESSION ESTABLISHMENT REQUEST message, or PDU SESSION MODIFICATION REQUEST message for the same [S-NSSAI, DNN] combination that was sent by the UE, until timer T35cd expires or timer Tsm1 is stopped.

The UE shall not stop timer Tsm1 upon a PLMN change or inter-system change.

b)	if the timer value indicates that this timer is deactivated, the UE shall stop timer T35cd associated with the same [S-NSSAI, DNN] combination as that the UE provided when the PDU session is established, if it is running. the UE shall not send another PDU SESSION ESTABLISHMENT REQUEST message, or PDU SESSION MODIFICATION REQUEST for the same [S-NSSAI, DNN] combination until the UE is switched off or the USIM is removed, or the UE receives a PDU SESSION MODIFICATION REQUEST message for the same [S-NSSAI, DNN] combination from the network or a PDU SESSION RELEASE COMMAND message including 5GSM cause #39 "reactivation requested" for the same [S-NSSAI, DNN] combination from the network

The timer T35cd remains deactivated upon a PLMN change or inter-system change.

c)	if the timer value indicates zero, the UE shall stop timer T35cd associated with the same [S-NSSAI, DNN] combination, if running, and may send another PDU SESSION ESTABLISHMENT REQUEST message, or PDU SESSION MODIFICATION REQUEST message for the same [S-NSSAI, DNN] combination.

If the Back-off timer value IE is not included, then the UE may send another PDU SESSION ESTABLISHMENT REQUEST message, or PDU SESSION MODIFICATION REQUEST message for the same [S-NSSAI, DNN] combination.

When the timer T35cd is running or the timer is deactivated, the UE is allowed to initiate a PDU session establishment procedure for emergency services.

If the timer T35cd is running when the UE enters state5GMM-DEREGISTERED, the UE remains switched on, and the USIM in the UE remains the same, then timer T35cd is kept running until it expires or it is stopped.

If the UE is switched off when the timer T35cd is running, and if the USIM in the UE remains the same when the UE is switched on, the UE shall behave as follows:

-	let t1 be the time remaining for T35cd timeout at switch off and let t be the time elapsed between switch off and switch on. If t1 is greater than t, then the timer shall be restarted with the value t1 – t. If t1 is equal to or less than t, then the timer need not be restarted. If the UE is not capable of determining t, then the UE shall restart the timer with the value t1.

#### 6.4.2.5	Abnormal cases in the UE

The following abnormal cases can be identified:

a)	Expiry of timer T3581

The UE shall, on the first expiry of the timer T3581, retransmit the PDU SESSION MODIFICATION REQUEST message and shall reset and start timer T3581. This retransmission is repeated four times, i.e. on the fifth expiry of timer T3581, the UE shall abort the procedure and shall release the allocated PTI.

Editor's note:	Further abnormal cases are FFS.

#### 6.4.2.6	Abnormal cases on the network side

Editor's note:	Further abnormal cases are FFS.

The following abnormal cases can be identified:

a)	If the PDU session is an emergency PDU session, the SMF shall reject the PDU SESSION MODIFICATION REQUEST message with ESM cause #31 "request rejected, unspecified"

### 6.4.3	UE-requested PDU session release procedure

#### 6.4.3.1	General

The purpose of the UE-requested PDU session release procedure is to enable by the UE to request a release of a PDU session.

The UE is allowed to initiate the PDU session release procedure even if the timer T35ab is running.

The UE is allowed to initiate the PDU session release procedure even if the timer T35cd is running.

#### 6.4.3.2	UE-requested PDU session release procedure initiation

In order to initiate the UE-requested PDU session release procedure, the UE shall create an PDU SESSION RELEASE REQUEST message.

The UE shall allocate a PTI value currently not used and shall set the PTI IE of the PDU SESSION RELEASE REQUEST message to the allocated PTI value.

The UE shall transport the PDU SESSION RELEASE REQUEST message and the PDU session ID, using the NAS transport procedure as specified in subclause 5.4.5, and the UE shall start timer T3582 (see example in figure 6.4.3.2.1).

Figure 6.4.3.2.1: UE-requested PDU session release procedure

#### 6.4.3.3	UE-requested PDU session release procedure accepted by the network

Upon receipt of a PDU SESSION RELEASE REQUEST message and a PDU session ID, if the SMF accepts the request to release the PDU session, and shall perform the network-requested PDU session release procedure as specified in subclause 6.3.3.

#### 6.4.3.4	UE-requested PDU session release procedure not accepted by the network

Upon receipt of a PDU SESSION RELEASE REQUEST message, if the SMF does not accept the request to release the PDU session, the SMF shall create an PDU SESSION RELEASE REJECT message.

The SMF shall set the 5GSM cause IE of the PDU SESSION RELEASE REJECT message to indicate the reason for rejecting the PDU session release.

The 5GSM cause IE typically indicates one of the following SM cause values:

Editor's note:	5GSM causes are FFS.

The SMF shall send the PDU SESSION RELEASE REJECT message.

Upon receipt of a PDU SESSION RELEASE REJECT message and a PDU session ID, using the NAS transport procedure as specified in subclause 5.4.5, the UE shall stop timer T3582, shall release the allocated PTI value, and shall consider that the PDU session is not released.

#### 6.4.3.5	Abnormal cases in the UE

The following abnormal cases can be identified:

a)	Expiry of timer T3582

The UE shall, on the first expiry of the timer T3582, retransmit the PDU SESSION RELEASE REQUEST message and shall reset and start timer T3582. This retransmission is repeated four times, i.e. on the fifth expiry of timer T3582, the UE shall abort the procedure, shall release the allocated PTI, and shall perform the registration procedure for mobility and periodic registration update with a REGISTRATION REQUEST message including the PDU session status IE.

Editors' note:	Further abnormal cases are FFS.

#### 6.4.3.6	Abnormal cases on the network side

Editors' note:	Abnormal cases are FFS.

## 6.5	5GSM status procedure

### 6.5.1	General

The purpose of the sending of the 5GSM STATUS message is to report at any time certain error conditions detected upon receipt of 5GSM protocol data. The 5GSM STATUS message can be sent by both the network and the UE (see example in figure 6.5.1.1).

Figure 6.5.1.1: 5GSM status procedure

### 6.5.2	5GSM status received in the UE

If the 5GSM entity of the UE receives a 5GSM STATUS message the UE shall take different actions depending on the received 5GSM cause value:

#97	(Message type non-existent or not implemented);

The UE shall abort any ongoing 5GSM procedure related to the PTI or PDU session Id and stop any related timer.

Editor's note:	Additional cause values and actions is FFS.

On receipt of a 5GSM STATUS message with any other 5GSM cause value no state transition and no specific action shall be taken as seen from the radio interface, i.e. local actions are possible.

### 6.5.3	5GSM status received in the SMF

If the SMF receives a 5GSM STATUS message the SMF shall take different actions depending on the received 5GSM cause value:

#97	(Message type non-existent or not implemented);

The SMF shall abort any ongoing 5GSM procedure related to the PTI or PDU session Id and stop any related timer.

Editor's note:	Additional cause values and actions is FFS.

The local actions to be taken by the SMF on receipt of a 5GSM STATUS message with any other 5GSM cause value are implementation dependent.

## 6.6	Miscellaneous procedures

### 6.6.1	Exchange of extended protocol configuration options

# 7	Handling of unknown, unforeseen, and erroneous protocol data

## 7.1	General

The procedures specified in the present document apply to those messages which pass the checks described in this subclause.

This subclause also specifies procedures for the handling of unknown, unforeseen, and erroneous protocol data by the receiving entity. These procedures are called "error handling procedures", but in addition to providing recovery mechanisms for error situations they define a compatibility mechanism for future extensions of the protocols.

Subclauses 7.1 to 7.8 shall be applied in order of precedence.

Detailed error handling procedures in the network are implementation dependent and may vary from PLMN to PLMN. However, when extensions of this protocol are developed, networks will be assumed to have the error handling that is indicated in this subclause as mandatory ("shall") and that is indicated as strongly recommended ("should").

Also, the error handling of the network is only considered as mandatory or strongly recommended when certain thresholds for errors are not reached during a dedicated connection.

For definition of semantical and syntactical errors see 3GPP TS 24.007 [7], subclause 11.4.2.

Editor's note:	It is FFS to add more details and further alignment to align with 3GPP TS 24.301.

## 7.2	Message too short

When a message is received that is too short to contain a complete message type information element, that message shall be ignored, cf. 3GPP TS 24.007 [7].

## 7.3	Unknown or unforeseen procedure transaction identity or PDU Session identity

### 7.3.1	Procedure transaction identity

Editor's note:	It is FFS to add the content.

### 7.3.2	PDU Session identity

Editor's note:	It is FFS to add the content.

## 7.4	Unknown or unforeseen message type

If UE receives a 5GMM message or 5GSM message with message type not defined for the extended protocol discriminator (EPD) or not implemented by the receiver, it shall return a status message (5GMM STATUS or 5GSM STATUS depending on the EPD) with cause #97 "message type non-existent or not implemented".

If the network receives a 5GMM or 5GSM message with message type not defined for the EPD or not implemented by the receiver in a protocol state where reception of an unsolicited message with the given EPD from the UE is not foreseen in the protocol, the network actions are implementation dependent. Otherwise, if the network receives a message with message type not defined for the EPD or not implemented by the receiver, it shall ignore the message except that it should return a status message (5GMM STATUS or 5GSM STATUS depending on the EPD) with cause #97 "message type non-existent or not implemented".

NOTE:	A message type not defined for the EPD in the given direction is regarded by the receiver as a message type not defined for the EPD, see 3GPP TS 24.007 [7].

If the UE receives a message not compatible with the protocol state, the UE shall return a status message (5GMM STATUS or 5GSM STATUS depending on the EPD) with cause #98 "message type not compatible with protocol state".

If the network receives a message not compatible with the protocol state, the network actions are implementation dependent.

## 7.5	Non-semantical mandatory information element errors

### 7.5.1	Common procedures

When on receipt of a message,

a)	an "imperative message part" error; or

b)	a "missing mandatory IE" error

is diagnosed or when a message containing:

a)	a syntactically incorrect mandatory IE;

b)	an IE unknown in the message, but encoded as "comprehension required" (see 3GPP TS 24.007 [7]); or

c)	an out of sequence IE encoded as "comprehension required" (see 3GPP TS 24.007 [7]) is received,

the UE shall proceed as follows:

If the message is not one of the messages listed in subclause 7.5.3, item TBD, the UE shall return a status message (5GMM STATUS or 5GSM STATUS depending on the EPD) with cause #96 "invalid mandatory information"; and

Editor's note:	Specific cases when the above is not applicable are FFS.

the network shall proceed as follows:

a)	If the message is not one of the messages listed in subclause 7.5.3, item TBD, the network shall either:

1)	try to treat the message (the exact further actions are implementation dependent); or

2)	ignore the message except that it should return a status message (5GMM STATUS or 5GSM STATUS depending on the EPD) with cause #96 "invalid mandatory information".

Editor's note:	Specific cases when the above is not applicable are FFS.

### 7.5.2	5GS mobility management

Editor's note:	It is FFS to add the content.

### 7.5.3	5GS session management

Editor's note:	It is FFS to add the content.

## 7.6	Unknown and unforeseen IEs in the non-imperative message part

### 7.6.1	IEIs unknown in the message

The UE shall ignore all IEs unknown in a message which are not encoded as "comprehension required" (see 3GPP TS 24.007 [7]).

The network shall take the same approach.

### 7.6.2	Out of sequence IEs

The UE shall ignore all out of sequence IEs in a message which are not encoded as "comprehension required" (see 3GPP TS 24.007 [7]).

The network should take the same approach.

### 7.6.3	Repeated IEs

If an information element with format T, TV, TLV, or TLV-E is repeated in a message in which repetition of the information element is not specified in clause 8 and clause 9 of the present document, the UE shall handle only the contents of the information element appearing first and shall ignore all subsequent repetitions of the information element. When repetition of information elements is specified, the UE shall handle only the contents of specified repeated information elements. If the limit on repetition of information elements is exceeded, the UE shall handle the contents of information elements appearing first up to the limit of repetitions and shall ignore all subsequent repetitions of the information element.

The network should follow the same procedures.

## 7.7	Non-imperative message part errors

This category includes:

a)	syntactically incorrect optional IEs; and

b)	conditional IE errors.

### 7.7.1	Syntactically incorrect optional IEs

The UE shall treat all optional IEs that are syntactically incorrect in a message as not present in the message.

The network shall take the same approach.

### 7.7.2	Conditional IE errors

When upon receipt of a 5GMM or 5GSM message the UE diagnoses a "missing conditional IE" error or an "unexpected conditional IE" error, or when it receives a 5GMM or 5GSM message containing at least one syntactically incorrect conditional IE, the UE shall ignore the message and shall return a status message (5GMM STATUS or 5GSM STATUS depending on the EPD) with cause #100 "conditional IE error".

When the network receives a message and diagnoses a "missing conditional IE" error or an "unexpected conditional IE" error or when it receives a message containing at least one syntactically incorrect conditional IE, the network shall either:

a)	try to treat the message (the exact further actions are implementation dependent); or

b)	ignore the message except that it should return a status message (5GMM STATUS or 5GSM STATUS depending on the EPD) with cause #100 "conditional IE error".

## 7.8	Messages with semantically incorrect contents

When a message with semantically incorrect contents is received, the UE shall perform the foreseen reactions of the procedural part of the present document (i.e. of clauses 8, 9). If, however no such reactions are specified, the UE shall ignore the message except that it shall return a status message (5GMM STATUS or 5GSM STATUS depending on the EPD) with cause #95 "semantically incorrect message".

The network should follow the same procedure except that a status message is not normally transmitted.

# 8	Message functional definitions and contents

## 8.1	Overview

## 8.2	5GS mobility management messages

### 8.2.1	Authentication request

#### 8.2.1.1	Message definition

The AUTHENTICATION REQUEST message is sent by the AMF to the UE to initiate authentication of the UE identity. See table 8.2.1.1.1.

Message type:	AUTHENTICATION REQUEST

Significance:		dual

Direction:			network to UE

Table 8.2.1.1.1: AUTHENTICATION REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Authentication request message identity | Message type 9.7 | M | V | 1 |
|  | ngKSI | NAS key set identifier 9.8.3.22 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
| 21 | Authentication parameter RAND (5G authentication challenge) | Authentication parameter RAND 9.8.3.9 | O | TV | 17 |
| 20 | Authentication parameter AUTN (5G authentication challenge) | Authentication parameter AUTN 9.8.3.8 | O | TLV | 18 |
| 78 | EAP message | EAP message 9.8.3.14 | O | TLV-E | 7-1503 |

Editor's note:	The content of the Authentication request message is FFS.

#### 8.2.1.2	EAP message

EAP message IE is included if the AUTHENTICATION REQUEST message is used in an EAP authentication procedure.

### 8.2.2	Authentication response

#### 8.2.2.1	Message definition

The AUTHENTICATION RESPONSE message is sent by the UE to the AMF to deliver a calculated authentication response to the network. See table 8.2.2.1.1.

Message type:	AUTHENTICATION RESPONSE

Significance:		dual

Direction:			UE to network

Table 8.2.2.1.1: AUTHENTICATION RESPONSE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Authentication response message identity | Message type 9.7 | M | V | 1 |
| 78 | EAP message | EAP message 9.8.3.14 | O | TLV-E | 7-1503 |

Editor's note:	The content of the Authentication response message is FFS. At least the following is needed: authentication response parameter (mandatory).

#### 8.2.2.2	EAP message

EAP message IE is included if the EAP message received in a related AUTHENTICATION REQUEST message was an EAP-request.

### 8.2.3	Authentication failure

#### 8.2.3.1	Message definition

The AUTHENTICATION FAILURE message is sent by the UE to the AMF to indicate that authentication of the network has failed. See table 8.2.3.1.1.

Message type:	AUTHENTICATION FAILURE

Significance:		dual

Direction:			UE to network

Table 8.2.3.1.1: AUTHENTICATION FAILURE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Authentication failure message identity | Message type 9.7 | M | V | 1 |
|  | 5GMM cause | 5GMM cause 9.8.3.2 | M | V | 1 |

Editor's note:	The further contents of the Authentication failure message are FFS. At least the following is needed: authentication failure parameter (optional).

### 8.2.4	Authentication reject

#### 8.2.4.1	Message definition

The AUTHENTICATION REJECT message is sent by the AMF to the UE to indicate that the authentication procedure has failed and that the UE shall abort all activities. See table 8.2.4.1.1.

Message type:	AUTHENTICATION REJECT

Significance:		dual

Direction:			network to UE

Table 8.2.4.1.1: AUTHENTICATION REJECT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Authentication reject message identity | Message type 9.7 | M | V | 1 |

Editor's note:	The content of the Authentication reject message is FFS.

### 8.2.5	Registration request

#### 8.2.5.1	Message definition

The REGISTRATION REQUEST message is sent by the UE to the AMF. See table 8.2.5.1.1.

Message type:	REGISTRATION REQUEST

Significance:		dual

Direction:			UE to network

Table 8.2.5.1.1: REGISTRATION REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended Protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Registration request message identity | Message type 9.7 | M | V | 1 |
|  | 5GS registration type | 5GS registration type 9.8.3.6 | M | V | 1 |
|  | ngKSI | NAS key set identifier 9.8.3.22 | M | V | 1 |
|  | Mobile identity | 5GS mobile identity 9.8.3.3 | M | LV | TBD |
| 55 | NonceUE | Nonce 9.8.3.27 | O | TV | 5 |
| 10 | 5GMM capability | 5GMM capability 9.8.3.1 | O | TLV | 4-15 |
| 2E | UE security capability | UE security capability 9.8.3.48 | O | TLV | 4-6 |
| 2F | Requested NSSAI | NSSAI 9.8.3.28 | O | TLV | 4-74 |
| 52 | Last visited registered TAI | Tracking area identity 9.8.3.44 | O | TV | 6 |
| 30 | S1 UE network capability | S1 UE network capability 9.8.3.36 | O | TV | 6-13 |
| 40 | Uplink data status | Uplink data status 9.8.2.3 | O | TLV | 4 |
| 50 | PDU session status | PDU session status 9.8.2.2 | O | TLV | 4 |
| B- | MICO indication | MICO indication 9.8.3.21 | O | TV | 1 |
| 2B | UE status | UE status 9.8.3.49 | O | TLV | 3 |
| 2C | Additional GUTI | 5GS mobile identity 9.8.3.3 | O | TLV | TBD |
| 2D | NSSAI info for PDU sessions | NSSAI info for PDU sessions 9.8.3.29 | O | TLV | 6-90 |
| C- | SMS requested | SMS requested 9.8.3.41 | O | TV | 1 |
| 7C | EPS NAS message container | EPS NAS message container 9.8.3.15 | O | TLV-E | TBD |

Editor's note:	 The content of the REGISTRATION REQUEST message when a limited set of IEs including those needed to establish security in the initial message when it has no NAS security context is FFS.

#### 8.2.5.2	NonceUE

Editor's note:	Inclusion criteria for NonceUE are FFS.

#### 8.2.5.3	5GMM capability

The UE shall include this IE, unless the UE performs a periodic registration updating procedure.

#### 8.2.5.4	UE security capability

The UE shall include this IE, unless the UE performs a periodic registration updating procedure.

#### 8.2.5.5	Requested NSSAI

A UE supporting slicing shall include this IE, unless the UE performs a periodic registration updating procedure.

#### 8.2.5.6	Last visited registered TAI

This IE shall be included if the UE holds a valid last visited registered TAI.

#### 8.2.5.7	S1 UE network capability

A UE supporting S1 mode shall include this IE, unless the UE performs a periodic registration updating procedure.

#### 8.2.5.8	Uplink data status

This IE shall be included if the UE has uplink user data pending to be sent.

#### 8.2.5.9	PDU session status

This IE shall be included if the UE wants to indicate the PDU session that are active within the UE.

#### 8.2.5.10	MICO indication

The UE may include this IE to request the use of MICO mode.

#### 8.2.5.11	UE status

This IE shall be included if the UE wants to provide the network with information concerning aspects of the current UE registration status which is used for interworking with EPS.

#### 8.2.5.12	Additional GUTI

This IE shall be included if the UE performs the registration procedure due to intersystem change from S1 mode to N1 mode, the UE operates in single-registration mode and the UE has a 5G-GUTI.

#### 8.2.5.13	NSSAI info for PDU sessions

This IE shall be included if the UE performs the registration procedure due to intersystem change from S1 mode to N1 mode and has PDU session identities corresponding to PDN connection for which interworking to 5GS is supported and has S-NSSAI values associated with such PDU session identities.

#### 8.2.5.14	SMS requested

This IE shall be included if the UE wants to indicate to the network that it requires the use of SMS over NAS transport or its requirements to use SMS over NAS transport have changed.

#### 8.2.5.15	EPS NAS message container

The UE operating in the single-registration mode shall include this information elememnt if the UE performs mobility from S1 mode to N1 mode.  The content of this message container is the complete integrity protected TRACKING AREA UPATE REQUEST message, using EPS NAS security context.

### 8.2.6	Registration accept

#### 8.2.6.1	Message definition

The REGISTRATION ACCEPT message is sent by the AMF to the UE. See table 8.2.6.1.1.

Message type:	REGISTRATION ACCEPT

Significance:		dual

Direction:			network to UE

Table 8.2.6.1.1: REGISTRATION ACCEPT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Registration accept message identity | Message type 9.7 | M | V | 1 |
|  | 5GS registration result | 5GS registration result 9.8.3.5 | M | V | TBD |
| 2C | 5G-GUTI | 5GS mobile identity 9.8.3.3 | O | TLV | TBD |
| 4A | Equivalent PLMNs | PLMN list 9.8.3.33 | O | TLV | 5-47 |
| 54 | TAI list | Tracking area identity list 9.8.3.45 | O | TLV | 8-98 |
| 70 | Allowed NSSAI | NSSAI 9.8.3.28 | O | TLV | 4-74 |
| 11 | Rejected NSSAI | Rejected NSSAI 9.8.3.35 | O | TLV | 4-42 |
| wx | 5GS network feature support | 5GS network feature support 9.8.3.4 | O | TBD | TBD |
| 50 | PDU session status | PDU session status 9.8.2.2 | O | TLV | 4 |
| 26 | PDU session reactivation result | PDU session reactivation result 9.8.3.32 | O | TLV | 4-32 |
| 79 | LADN information | LADN information 9.8.3.19 | O | TLV-E | 11-1579 |
| B- | MICO indication | MICO indication 9.8.3.21 | O | TV | 1 |
| D- | SMS allowed | SMS allowed 9.8.3.40 | O | TV | 1 |

#### 8.2.6.2	5G-GUTI

This IE may be included to assign a 5G-GUTI to a UE.

#### 8.2.6.3	Equivalent PLMNs

This IE may be included in order to assign a new equivalent PLMNs list to a UE.

#### 8.2.6.4	TAI list

This IE may be included to assign a TAI list to a UE.

#### 8.2.6.5	Allowed NSSAI

Editor's note:	Inclusion criteria for Allowed NSSAI are FFS.

#### 8.2.6.6	Rejected NSSAI

Editor's note:	Inclusion criteria for Rejected NSSAI are FFS.

#### 8.2.6.8	5GS network feature support

The network may include this IE to inform the UE of the support of certain features. If this IE is not included then the UE shall interpret this as a receipt of an information element with all bits of the value part coded as zero.

#### 8.2.6.9	PDU session status

This IE shall be included if the network wants to indicate the PDU session contexts that are active in the network.

#### 8.2.6.10	PDU session reactivation result

This IE shall be included if the Uplink data status IE is included in the REGISTRATION REQUEST message.

#### 8.2.6.11	LADN information

Editor's note:	Inclusion criteria for LADN information are FFS.

#### 8.2.6.12	MICO indication

The network shall include the MICO indication IE if:

-	the UE included the MICO indication IE in the REGISTRATION REQUEST message; and

-	the network supports and accepts the use of MICO mode.

#### 8.2.6.13	SMS allowed

This IE shall be included if the network wants to indicate to the UE that the use of SMS over NAS transport is allowed.

### 8.2.7	Registration complete

#### 8.2.7.1	Message definition

The REGISTRATION COMPLETE message is sent by the UE to the AMF. See table 8.2.7.1.1.

Message type:	REGISTRATION COMPLETE

Significance:		dual

Direction:			UE to network

Table 8.2.7.1.1: REGISTRATION COMPLETE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Registration complete message identity | Message type 9.6 | M | V | 1 |

### 8.2.8	Registration reject

#### 8.2.8.1	Message definition

The REGISTRATION REJECT message is sent by the AMF to the UE. See table 8.2.8.1.1.

Message type:	REGISTRATION REJECT

Significance:		dual

Direction:			network to UE

Table 8.2.8.1.1: REGISTRATION REJECT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Registration reject message identity | Message type 9.6 | M | V | 1 |
|  | 5GMM cause | 5GMM cause 9.8.3.2 | M | V | 1 |
| 5F | T3346 value | GPRS timer 2 9.8.3.16 | O | TLV | 3 |

#### 8.2.8.2	T3346 value

The AMF may include this IE when the general NAS level mobility management congestion control is active

### 8.2.9	UL NAS transport

#### 8.2.9.1	Message definition

The UL NAS TRANSPORT message transports message payload and associated information to the AMF. See table 8.2.9.1.1.

Message type:	UL NAS TRANSPORT

Significance:		dual

Direction:			UE to network

Table 8.2.9.1.1: UL NAS TRANSPORT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | UL NAS TRANSPORT message identity | Message type 9.7 | M | V | 1 |
|  | Payload container type | Payload container type 9.8.3.31 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Payload container | Payload container 9.8.3.30 | M | LV-E | 3-65537 |
|  | PDU session ID | PDU session identity 9.4 | C | V | 1 |
| 70 | Old PDU session ID | Old PDU session identity 9.8.3.34 | O | TV | 2 |
| 8- | Request type | Request type 9.8.3.42 | O | TV | 1 |
| 22 | S-NSSAI | S-NSSAI 9.8.3.37 | O | TLV | 3-10 |
| 25 | DNN | DNN 9.8.3.13 | O | TLV | 3-102 |
| 24 | Additional information | Additional information 9.8.2.1 | O | TLV | 3-n |

#### 8.2.9.2	PDU session ID

The UE shall include this IE when the Payload container type IE is set to "N1 SM information".

#### 8.2.9.3	Old PDU session ID

The UE shall include this IE if the UL NAS TRANSPORT message transports a PDU SESSION ESTABLISHMENT REQUEST message upon receiving the PDU SESSION MODIFICATION COMMAND message with the 5GSM cause IE set to #39 "reactivation requested".

#### 8.2.9.4	Request type

The UE may include this IE when the PDU session ID IE is included.

#### 8.2.9.5	S-NSSAI

The UE may include this IE when the Request type IE is set to "initial request".

#### 8.2.9.6	DNN

The UE may include this IE when the Request type IE is set to "initial request".

#### 8.2.9.7	Additional information

The UE may include this IE when the Payload container type IE is set to "LTE Positioning Protocol (LPP) message container".

### 8.2.10	DL NAS transport

#### 8.2.10.1	Message definition

The DL NAS TRANSPORT message transports message payload and associated information to the UE. See table 8.2.10.1.1.

Message type:	DL NAS TRANSPORT

Significance:		dual

Direction:			network to UE

Table 8.2.10.1.1: DL NAS TRANSPORT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | DL NAS TRANSPORT message identity | Message type 9.7 | M | V | 1 |
|  | Payload container type | Payload container type 9.8.3.31 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Payload container | Payload container 9.8.3.30 | M | LV-E | 3-65537 |
|  | PDU session ID | PDU session idenitty 9.4 | C | V | 1 |
| 24 | Additional information | Additional information 9.8.2.1 | O | TLV | 3-n |

#### 8.2.10.2	PDU session ID

The AMF shall include this IE when the Payload container type IE is set to "N1 SM information".

#### 8.2.10.3	Additional information

The AMF may include this IE when the Payload container type IE is set to "LTE Positioning Protocol (LPP) message container".

### 8.2.11	De-registration request (UE originating de-registration)

#### 8.2.11.1	Message definition

The DEREGISTRATION REQUEST message is sent by the UE to the AMF. See table 8.2.11.1.1.

Message type:	DEREGISTRATION REQUEST

Significance:		dual

Direction:			UE to network

Table 8.2.11.1.1: DEREGISTRATION REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | De-registration request message identity | Message type 9.7 | M | V | 1 |
|  | De-registration type | De-registration type 9.8.3.12 | M | V | 1 |
|  | 5GS mobile identity | 5GS mobile identity 9.8.3.3 | M | TLV | TBD |

### 8.2.12	De-registration accept (UE originating de-registration)

#### 8.2.12.1	Message definition

The DEREGISTRATION ACCEPT message is sent by the AMF to the UE. See table 8.2.12.1.1.

Message type:	DEREGISTRATION ACCEPT

Significance:		dual

Direction:			network to UE

Table 8.2.12.1.1: DEREGISTRATION ACCEPT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | De-registration accept message identity | Message type 9.7 | M | V | 1 |

### 8.2.13	De-registration request (UE terminated de-registration)

#### 8.2.13.1	Message definition

The DEREGISTRATION REQUEST message is sent by the AMF to the UE. See table 8.2.13.1.1.

Message type:	DEREGISTRATION REQUEST

Significance:		dual

Direction:			network to UE

Table 8.2.13.1.1: DEREGISTRATION REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | De-registration request message identity | Message type 9.7 | M | V | 1 |
|  | De-registration type | De-registration type 9.8.3.12 | M | V | 1 |
| 72 | 5GMM cause | 5GMM cause 9.8.3.2 | O | TV | 2 |
| 5F | T3346 value | GPRS timer 2 9.8.3.16 | O | TLV | 3 |

#### 8.2.13.2	5GMM cause

This information element is included if a 5GMM cause is provided.

#### 8.2.13.3	T3346 value

The AMF may include this IE when the general NAS level mobility management congestion control is active.

### 8.2.14	De-registration accept (UE terminated de-registration)

#### 8.2.14.1	Message definition

The DEREGISTRATION ACCEPT message is sent by the UE to the AMF. See table 8.2.14.1.1.

Message type:	DEREGISTRATION ACCEPT

Significance:		dual

Direction:			UE to network

Table 8.2.14.1.1.1: DEREGISTRATION ACCEPT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | De-registration accept message identity | Message type 9.7 | M | V | 1 |

### 8.2.15	Service request

#### 8.2.15.1	Message definition

The SERVICE REQUEST message is sent by the UE to the AMF in order to request the establishment of an N1 NAS signalling connection and/or to request establishment of user-plane radio resources for PDU sessions which are activated without radio resources. See table 8.2.15.1.1.

Message type:	SERVICE REQUEST

Significance:		dual

Direction:			UE to network

Table 8.2.15.1.1: SERVICE REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Service type | Service type9.8.3.43 | M | V | 1/2 |
|  | 5G-TMSI | 5GS mobile identity 9.8.3.3 | M | LV | 6 |
|  | Service request message identity | Message type 9.7 | M | V | 1 |
|  | ngKSI | NAS key set identifier 9.8.3.22 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.4 | M | V | 1/2 |
| 40 | Uplink data status | Uplink data status 9.8.2.3 | O | TLV | 4 |
| 50 | PDU session status | PDU session status 9.8.2.2 | O | TLV | 4 |
| 25 | Allowed PDU session status | Allowed PDU session status 9.8.3.7 | O | TLV | 4-34 |

#### 8.2.15.2	Uplink data status

This IE shall be included if the UE has uplink user data pending to be sent.

#### 8.2.15.3	PDU session status

This IE shall be included if the UE wants to indicate the PDU sessions that are active within the UE.

#### 8.2.15.4	Allowed PDU session status

This IE shall be included if the SERVICE REQUEST message is sent as a response to paging or notification via 3GPP access for PDU session(s) associated with non-3GPP access and the UE wants to indicate the user-plane resources of PDU session(s) associated with non-3GPP access allowed to be re-activated over 3GPP access.

### 8.2.16	Service accept

#### 8.2.16.1	Message definition

The SERVICE ACCEPT message is sent by the AMF to the UE in order to accept the service request procedure. See table 8.2.16.1.1.

Message type:	SERVICE ACCEPT

Significance:		dual

Direction:			network to UE

Table 8.2.16.1.1: SERVICE ACCEPT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Service accept message identity | Message type 9.7 | M | V | 1 |
| 50 | PDU session status | PDU session status 9.8.2.2 | O | TLV | 4 |
| 26 | PDU session reactivation result | PDU session reactivation result 9.8.3.32 | O | TLV | 4-32 |

#### 8.2.16.2	PDU session status

This IE shall be included if the network wants to indicate the PDU sessions that are active within the network.

#### 8.2.16.3	PDU session reactivation result

This IE shall be included if the Uplink data status IE or Allowed PDU session status IE is included in the SERVICE REQUEST message.

### 8.2.17	Service reject

#### 8.2.17.1	Message definition

The SERVICE REJECT message is sent by the AMF to the UE in order to reject the service request procedure. See table 8.2.17.1.1.

Message type:	SERVICE REJECT

Significance:		dual

Direction:			network to UE

Table 8.2.17.1.1: SERVICE REJECT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Service reject message identity | Message type 9.7 | M | V | 1 |
|  | 5GMM cause | 5GMM cause 9.8.3.2 | M | V | 1 |
| 50 | PDU session status | PDU session status 9.8.2.2 | O | TLV | 4 |
| 5F | T3346 value | GPRS timer 2 9.8.3.16 | O | TLV | 3 |

#### 8.2.17.2	PDU session status

This IE shall be included if the network wants to indicate the PDU sessions that are active within the network.

#### 8.2.17.3	T3346 value

The AMF may include this IE when the general NAS level mobility management congestion control is active.

### 8.2.18	Configuration update command

#### 8.2.18.1	Message definition

The CONFIGURATION UPDATE COMMAND message is sent by the AMF to the UE. See table 8.2.18.1.1.

Message type:	CONFIGURATION UPDATE COMMAND

Significance:		dual

Direction:			network to UE

Table 8.2.18.1.1: CONFIGURATION UPDATE COMMAND message content

| IEI |  | Information Element |  | Type/Reference |  | Presence |  | Format |  | Length |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Extended protocol discriminator |  | Extended protocol discriminator 9.2 |  | M |  | V |  | 1 |  |
|  |  | Security header type |  | Security header type 9.3 |  | M |  | V |  | 1/2 |  |
|  |  | Spare half octet |  | Spare half octet 9.5 |  | M |  | V |  | 1/2 |  |
|  |  | Configuration update command message identity |  | Message type 9.7 |  | M |  | V |  | 1 |  |
|  |  | Configuration update indication |  | Configuration update indication 9.8.3.10 |  | M |  | V |  | 1/2 |  |
|  |  | Spare half octet |  | Spare half octet 9.5 |  | M |  | V |  | 1/2 |  |
| 2C |  | 5G-GUTI |  | 5GS mobile identity 9.8.3.3 |  | O |  | TLV |  | TBD |  |
| 54 |  | TAI list |  | Tracking area identity list 9.8.3.45 |  | O |  | TLV |  | 8-98 |  |
| 70 |  | Allowed NSSAI |  | NSSAI 9.8.3.28 |  | O |  | TLV |  | 4-74 |  |
| 27 |  | Service area list |  | Service area list 9.8.3.39 |  | O |  | TLV |  | 6-194 |  |
| 43 |  | Full name for network |  | Network name 9.8.3.26 |  | O |  | TLV |  | 3-n |  |
| 45 |  | Short name for network |  | Network name 9.8.3.26 |  | O |  | TLV |  | 3-n |  |
| 46 |  | Local time zone |  | Time zone 9.8.3.46 |  | O |  | TV |  | 2 |  |
| 47 |  | Universal time and local time zone |  | Time zone and time 9.8.3.47 |  | O |  | TV |  | 8 |  |
| 49 |  | Network daylight saving time |  | Daylight saving time 9.8.3.11 |  | O |  | TLV |  | 3 |  |
| 79 |  | LADN information |  | LADN information 9.8.3.19 |  | O |  | TLV-E |  | 11-1579 |  |
| B- |  | MICO indication |  | MICO indication 9.8.3.21 |  | O |  | TV |  | 1 |  |
| 31 |  | Configured NSSAI |  | NSSAI 9.8.3.28 |  | O |  | TLV |  | 4-74 |  |

Editor's note:	 The further contents of the CONFIGURATION UPDATE COMMAND message are FFS.

#### 8.2.18.2	5G-GUTI

This IE may be included to assign a new 5G GUTI to the UE.

#### 8.2.18.3	TAI list

This IE may be included to assign a new TAI list to the UE.

#### 8.2.18.4	Allowed NSSAI

This IE may be included to assign a new allowed NSSAI to the UE.

#### 8.2.18.5	Service area list

This IE may be included to assign a new service area list to the UE.

#### 8.2.18.6	Full name for network

This IE may be included to assign a new full name for network to the UE.

#### 8.2.18.7	Short name for network

This IE may be included to assign a new short name for network to the UE.

#### 8.2.18.8	Local time zone

This IE may be included to assign a new local time zone to the UE.

#### 8.2.18.9	Universal time and local time zone

This IE may be included to assign new universal time and local time zone to the UE.

#### 8.2.18.10	Network daylight saving time

This IE may be included to assign new network daylight saving time to the UE.

#### 8.2.18.11	LADN information

This IE may be included to assign new LADN information to the UE.

#### 8.2.18.12	MICO indication

This IE may be included to assign a new MICO indication to the UE.

#### 8.2.18.13	Configured NSSAI

The AMF shall include this IE if the AMF wants to provide the UE with a new configured NSSAI for the current PLMN.

### 8.2.19	Configuration update complete

#### 8.2.19.1	Message definition

The CONFIGURATION UPDATE COMPLETE message is sent by the UE to the AMF. See table 8.2.19.1.1.

Message type:	CONFIGURATION UPDATE COMPLETE

Significance:		dual

Direction:			UE to network

Table 8.2.19.1.1: CONFIGURATION UPDATE COMPLETE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Configuration update complete message identity | Message type 9.7 | M | V | 1 |

Editor's note:	 The further contents of the CONFIGURATION UPDATE COMPLETE message are FFS.

### 8.2.20	Identity request

#### 8.2.20.1	Message definition

The IDENTITY REQUEST message is sent by the AMF to the UE to request the UE to provide specified identity. See table 8.2.20.1.1

Message type:	IDENTITY REQUEST

Significance:		dual

Direction:			AMF to UE

Table 8.2.20.1.1: IDENTITY REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Identity request message identity | Message type 9.7 | M | V | 1 |
|  | Identity type | Identity type FFS | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |

Editor’s note:	The reference to "identity type" information element is FFS as certain identities will need to be defined.

### 8.2.21	Identity response

#### 8.2.21.1	Message definition

The IDENTITY RESPONSE message is sent by the UE to the AMF to provide the requested identity. See table 8.2.21.1.

Message type:	IDENTITY RESPONSE

Significance:		dual

Direction:			UE to AMF

Table 8.2.21.1.1: IDENTITY RESPONSE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Identity response message identity | Message type 9.7 | M | V | 1 |
|  | Mobile identity | 5GS mobile identity 9.8.3.3 | M | LV | TBD |

### 8.2.22	Notification

#### 8.2.22.1	Message definition

The NOTIFICATION message is sent by the AMF to the UE to notify the UE to initiate a service request procedure. See table 8.2.22.1.1.

Message type:	NOTIFICATION

Significance:		dual

Direction:			network to UE

Table 8.2.22.1.1: NOTIFICATION message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Notification message identity | Message type 9.7 | M | V | 1 |

Editor's note:	 The further contents of the NOTIFICATION message are FFS.

### 8.2.23	Notification response

#### 8.2.23.1	Message definition

The NOTIFICATION RESPONSE message is sent by the UE to the AMF to notify the failure to initiate the service request procedure as a response of notification. See table 8.2.23.1.1.

Message type:	NOTIFICATION RESPONSE

Significance:		dual

Direction:			UE to network

Table 8.2.23.1.1: NOTIFICATION RESPONSE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Notification response message identity | Message type 9.7 | M | V | 1 |

### 8.2.24	Security mode command

#### 8.2.24.1	Message definition

The SECURITY MODE COMMAND message is sent by the AMF to the UE to establish NAS signalling security. See table 8.2.24.1.1.

Message type:	SECURITY MODE COMMAND

Significance:		dual

Direction:			network to UE

# Table 8.6.24.1.1: SECURITY MODE COMMAND message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Security mode command message identity | Message type 9.7 | M | V | 1 |
|  | Selected NAS security algorithms | NAS security algorithms 9.8.3.24 | M | V | 1 |
|  | NAS key set identifier | NAS key set identifier 9.8.3.22 | M | V | 1 |
|  | Replayed UE security capabilities | UE security capability 9.8.3.48 | M | LV | 3-5 |
| Xa | Allowed NSSAI | NSSAI 9.8.3.28 | O | TBD | TBD |
| E- | IMEISV request | IMEISV request 9.8.3.18 | O | TV | 1 |
| 55 | Replayed nonceUE | Nonce 9.8.3.27 | O | TV | 5 |
| 56 | NonceAMF | Nonce 9.8.3.27 | O | TV | 5 |
| 4F | HashAMF | HashAMF 9.8.3.17 | O | TV | 9 |
| 78 | EAP message | EAP message 9.8.3.14 | O | TLV-E | 7 |

#### 8.2.24.2	Allowed NSSAI

The AMF may include this information element to indicate Allowed NSSAI.

Editor’s note:	In 3GPP TS 33.501 v0.6.0 inclusion of the Allowed NSSAI is indicated to be mandatory in the SECURITY MODE COMMAND message. Whether this is correct or if inclusion is optional is FFS.

#### 8.2.24.3	IMEISV request

The AMF may include this information element to request the UE to send its IMEISV with the corresponding SECURITY MODE COMPLETE message.

#### 8.2.24.4	Replayed nonceUE

The AMF may include this information element to indicate to the UE to use the replayed nonceUE.

#### 8.2.24.5	NonceAMF

The AMF may include this information element to indicate to the UE to use the nonceAMF.

#### 8.2.24.6	HashAMF

The AMF shall include this information element when the AMF is initiating a SECURITY MODE COMMAND during a registration procedure and the REGISTRATION REQUEST message did not successfully pass the integrity check at the AMF.

#### 8.2.24.7	EAP message

This IE is included when the EAP Success message is sent as part of the EAP-based primary authentication and key agreement procedure, as specified in subclause 5.4.1.2.

### 8.2.25	Security mode complete

#### 8.2.25.1	Message definition

The SECURITY MODE COMPLETE message is sent by the UE to the AMF in response to a SECURITY MODE COMMAND message. See table 8.2.25.1.1.

Message type:	SECURITY MODE COMPLETE

Significance:		dual

Direction:			UE to network

# Table 8.2.25.1.1: SECURITY MODE COMPLETE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Security mode complete message identity | Message type 9.6 | M | V | 1 |
|  | Allowed NSSAI | NSSAI 9.8.3.28 | M | TBD | TBD |
| 2C | IMEISV | 5G mobile identity 9.8.3.3 | O | TLV | TBD |
| 7D | NAS message container | NAS message container 9.8.3.23 | O | TLV-E | 3-n |

#### 8.2.25.2	IMEISV

The UE shall include this information element, if the IMEISV was requested within the corresponding SECURITY MODE COMMAND message.

#### 8.2.25.3	NAS message container

The UE shall include this information element:

a)	if during an ongoing registration procedure, the AMF included HASHAMF in the SECURITY MODE COMMAND message and HASHAMF has a different value from the hash value locally calculated at the UE as described in 3GPP TS 33.501 [12]; and

b)	when the UE has sent an initial NAS message with a limited set of IEs before the SECURITY MODE COMMAND message was received and the UE needs to include the complete initial NAS message in the SECURITY MODE COMPLETE message, as described in 3GPP TS 33.501 [12].

### 8.2.26	Security mode reject

#### 8.6.26.1	Message definition

The SECURITY MODE REJECT message is sent by the UE to the AMF to indicate that the corresponding security mode command has been rejected. See table 8.2.26.1.1.

Message type:	SECURITY MODE REJECT

Significance:		dual

Direction:			UE to network

# Table 8.2.26.1.1: SECURITY MODE REJECT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Security mode reject message identity | Message type 9.6 | M | V | 1 |
|  | EMM cause | 5GMM cause 9.8.3.2 | M | V | 1 |

### 8.2.27	5GMM status

#### 8.2.27.1	Message definition

The 5GMM STATUS message is sent by the UE or by the AMF at any time to report certain error conditions. See table 8.2.27.1.1.

Message type:	5GMM STATUS

Significance:		local

Direction:			both

Table 8.2.27.1.1: 5GMM STATUS message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | 5GMM STATUS message identity | Message type 9.7 | M | V | 1 |
|  | 5GMM cause | 5GMM cause 9.8.3.2 | M | V | 2 |

## 8.3	5GS session management messages

### 8.3.1	PDU session establishment request

#### 8.3.1.1	Message definition

The PDU SESSION ESTABLISHMENT REQUEST message is sent by the UE to the SMF to initiate establishment of a PDU session. See table 8.3.1.1.1.

Message type:	PDU SESSION ESTABLISHMENT REQUEST

Significance:		dual

Direction:			UE to network

Table 8.3.1.1.1: PDU SESSION ESTABLISHMENT REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | PDU session ID | PDU session identity 9.4 | M | V | 1 |
|  | PTI | Procedure transaction identity 9.6 | M | V | 1 |
|  | PDU SESSION ESTABLISHMENT REQUEST message identity | Message type 9.7 | M | V | 1 |
| 9- | PDU session type | PDU session type 9.8.4.5 | O | TV | 1 |
| A- | SSC mode | SSC mode 9.8.4.9 | O | TV | 1 |
| 28 | 5GSM capability | 5GSM capability 9.8.4.10 | O | TLV | 3-15 |
| yz | SM PDU DN request container | SM PDU DN request container 9.8.4.8 | O | TBD | TBD |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.8.4.2 | O | TLV-E | 4-65538 |

#### 8.3.1.2	PDU session type

This IE is included in the message when the UE requests to establish a new PDU session with a DN and requests a PDU session type.

#### 8.3.1.3	SSC mode

This IE is included in the message when the UE requests to establish a new PDU session with a DN and requests an SSC mode.

#### 8.3.1.4	5GSM capability

This IE is included in the message when the UE requests to establish a new PDU session of "IP", "IPv4", "IPv6" or "Ethernet" PDU session type and the UE supports Reflective QoS.

#### 8.3.1.5	SM PDU DN request container

This IE is included in the message when the UE requests to establish a new PDU session with a DN and needs to provide information for the PDU session authentication and authorization by the external DN.

#### 8.3.1.6	Extended protocol configuration options

This IE is included in the message when the UE wishes to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the network.

### 8.3.2	PDU session establishment accept

#### 8.3.2.1	Message definition

The PDU SESSION ESTABLISHMENT ACCEPT message is sent by the SMF to the UE in response to PDU SESSION ESTABLISHMENT REQUEST message and indicates successful establishment of a PDU session. See table 8.3.2.1.1.

Message type:	PDU SESSION ESTABLISHMENT ACCEPT

Significance:		dual

Direction:			network to UE

Table 8.3.2.1.1: PDU SESSION ESTABLISHMENT ACCEPT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | PDU session ID | PDU session identity 9.4 | M | V | 1 |
|  | PTI | Procedure transaction identity 9.6 | M | V | 1 |
|  | PDU SESSION ESTABLISHMENT ACCEPT message identity | Message type 9.7 | M | V | 1 |
|  | Selected PDU session type | PDU session type 9.8.4.5 | M | V | 1/2 |
|  | Selected SSC mode | SSC mode 9.8.4.9 | M | V | 1/2 |
|  | DNN | DNN 9.8.3.13 | M | LV | 2-TBD |
|  | Authorized QoS rules | QoS rules 9.8.4.6 | M | LV-E | 2-65537 |
|  | Session AMBR | Session-AMBR 9.8.4.7 | M | LV | TBD |
| 73 | 5GSM cause | 5GSM cause 9.8.4.1 | O | TV | 2 |
| 29 | PDU address | PDU address 9.8.4.4 | O | TLV | 7 |
| 78 | EAP message | EAP message 9.8.3.14 | O | TLV-E | 7-1503 |
| 74 | RQ timer value | GPRS timer 9.8.4.3 | O | TV | 2 |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.8.4.2 | O | TLV-E | 4-65538 |
| 22 | S-NSSAI | S-NSSAI 9.8.3.37 | O | TLV | 3-6 |

#### 8.3.2.2	5GSM cause

This IE is included when the selected PDU session type which is different from the PDU session type requested by the UE.

#### 8.3.2.3	PDU address

This IE is included when the selected PDU session type is “Ipv4” or “Ipv6”.

#### 8.3.2.4	EAP message

This IE is included when the external DN successfully performed authentication and authorization of the UE using EAP.

#### 8.3.2.5	RQ timer value

This IE is included when the network wants to provide the RQ timer value.

#### 8.3.2.6	Extended protocol configuration options

This IE is included in the message when the network wishes to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the UE.

#### 8.3.2.x	S-NSSAI

This IE shall be included in the message when the SMF received from the AMF an S-NSSAI together with the PDU SESSION ESTABLISHMENT REQUEST message from the AMF.

### 8.3.3	PDU session establishment reject

#### 8.3.3.1	Message definition

The PDU SESSION ESTABLISHMENT REJECT message is sent by the SMF to the UE in response to PDU SESSION ESTABLISHMENT REQUEST message and indicates unsuccessful establishment of a PDU session. See table 8.3.3.1.1.

Message type:	PDU SESSION ESTABLISHMENT REJECT

Significance:		dual

Direction:			network to UE

Table 8.3.3.1.1: PDU SESSION ESTABLISHMENT REJECT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | PDU session ID | PDU session identity 9.4 | M | V | 1 |
|  | PTI | Procedure transaction identity 9.6 | M | V | 1 |
|  | PDU SESSION ESTABLISHMENT REJECT message identity | Message type 9.7 | M | V | 1 |
|  | 5GSM cause | 5GSM cause 9.8.4.1 | M | V | 1 |
| 78 | EAP message | EAP message 9.8.3.14 | O | TLV-E | 7-1503 |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.8.4.2 | O | TLV-E | 4-65538 |

#### 8.3.3.2	Extended protocol configuration options

This IE is included in the message when the network wishes to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the UE.

#### 8.3.3.3	EAP message

This IE is included when the external DN unsuccessfully performed authentication and authorization of the UE using EAP.

### 8.3.4	PDU session authentication command

#### 8.3.4.1	Message definition

The PDU SESSION AUTHENTICATION COMMAND message is sent by the SMF to the UE for authentication of the UE establishing the PDU session. See table 8.3.4.1.1.

Message type:	PDU SESSION AUTHENTICATION COMMAND

Significance:		dual

Direction:			network to UE

Table 8.3.4.1.1: PDU SESSION AUTHENTICATION COMMAND message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | PDU session ID | PDU session identity 9.4 | M | V | 1 |
|  | PTI | Procedure transaction identity 9.6 | M | V | 1 |
|  | PDU SESSION AUTHENTICATION COMMAND message identity | Message type 9.7 | M | V | 1 |
| 78 | EAP message | EAP message 9.8.3.14 | O | TLV-E | 7-1503 |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.8.4.2 | O | TLV-E | 4-65538 |

#### 8.3.4.2	EAP message

This IE is included when the external DN performs authentication and authorization of the UE using EAP.

#### 8.3.4.3	Extended protocol configuration options

This IE is included in the message when the network wishes to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the UE.

### 8.3.5	PDU session authentication complete

#### 8.3.5.1	Message definition

The PDU SESSION AUTHENTICATION COMPLETE message is sent by the UE to the SMF in response to the PDU SESSION AUTHENTICATION COMMAND message and indicates acceptance of the PDU SESSION AUTHENTICATION COMMAND message. See table 8.3.5.1.1.

Message type:	PDU SESSION AUTHENTICATION COMPLETE

Significance:		dual

Direction:			UE to network

Table 8.3.5.1.1: PDU SESSION AUTHENTICATION ACCEPT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | PDU session ID | PDU session identity 9.4 | M | V | 1 |
|  | PTI | Procedure transaction identity 9.6 | M | V | 1 |
|  | PDU SESSION AUTHENTICATION COMPLETE message identity | Message type 9.7 | M | V | 1 |
| 78 | EAP message | EAP message 9.8.3.14 | O | TLV-E | 7-1503 |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.8.4.2 | O | TLV-E | 4-65538 |

#### 8.3.5.2	EAP message

This IE is included when the external DN performs authentication and authorization of the UE using EAP.

#### 8.3.5.3	Extended protocol configuration options

This IE is included in the message when the UE wishes to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the network.

### 8.3.7	PDU session modification request

#### 8.3.7.1	Message definition

The PDU SESSION MODIFICATION REQUEST message is sent by the UE to the SMF to request a modification of a PDU session. See table 8.3.7.1.1.

Message type:	PDU SESSION MODIFICATION REQUEST

Significance:		dual

Direction:			UE to network

Table 8.3.7.1.1: PDU SESSION MODIFICATION REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | PDU session ID | PDU session identity 9.4 | M | V | 1 |
|  | PTI | Procedure transaction identity 9.6 | M | V | 1 |
|  | PDU SESSION MODIFICATION REQUEST message identity | Message type 9.7 | M | V | 1 |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.8.4.2 | O | TLV-E | 4-65538 |
| 7A | Requested QoS rules | QoS rules 9.8.4.6 | O | TLV-E | 3-65538 |

#### 8.3.7.2	Extended protocol configuration options

This IE is included in the message when the UE wishes to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the network.

#### 8.3.7.3	Requested QoS rules

This IE is included in the message when the UE requests a specific QoS handling.

### 8.3.8	PDU session modification reject

#### 8.3.8.1	Message definition

The PDU SESSION MODIFICATION REJECT message is sent by the SMF to the UE to indicate rejection of the PDU SESSION MODIFICATION REQUEST. See table 8.3.8.1.1.

Message type:	PDU SESSION MODIFICATION REJECT

Significance:		dual

Direction:			network to UE

Table 8.3.8.1.1: PDU SESSION MODIFICATION REJECT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | PDU session ID | PDU session identity 9.4 | M | V | 1 |
|  | PTI | Procedure transaction identity 9.6 | M | V | 1 |
|  | PDU SESSION MODIFICATION REJECT message identity | Message type 9.7 | M | V | 1 |
|  | 5GSM cause | 5GSM cause 9.8.4.1 | M | V | 1 |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.8.4.2 | O | TLV-E | 4-65538 |

#### 8.3.8.2	Extended protocol configuration options

This IE is included in the message when the network wishes to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the UE.

### 8.3.9	PDU session modification command

#### 8.3.9.1	Message definition

The PDU SESSION MODIFICATION COMMAND message is sent by the SMF to the UE to indicate a modification of a PDU session. See table 8.3.9.1.1

Message type:	PDU SESSION MODIFICATION COMMAND

Significance:		dual

Direction:			network to UE

Table 8.3.9.1.1: PDU SESSION MODIFICATION COMMAND message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | PDU session ID | PDU session identity 9.4 | M | V | 1 |
|  | PTI | Procedure transaction identity 9.6 | M | V | 1 |
|  | PDU SESSION MODIFICATION COMMAND message identity | Message type 9.7 | M | V | 1 |
| 73 | 5GSM cause | 5GSM cause 9.8.4.1 | O | TV | 2 |
| 7B | Authorized QoS rules | QoS rules 9.8.4.6 | O | TLV-E | 3-65538 |
| 2A | Session AMBR | Session-AMBR 9.8.4.7 | O | TLV | 8 |
| 75 | PDU session release time | GPRS timer 9.8.4.3 | O | TV | 2 |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.8.4.2 | O | TLV-E | 4-65538 |

#### 8.3.9.2	5GSM cause

This IE is included when the network performs the PDU session anchor relocation for SSC mode 3.

#### 8.3.9.3	Authorized QoS rules

This IE is included when the authorized QoS rules of the PDU session are modified.

#### 8.3.9.4	Session-AMBR

This IE is included when the session-AMBR of the PDU session is modified.

#### 8.3.9.5	PDU session release time

This IE is included when the network performs the PDU session anchor relocation for SSC mode 3.

#### 8.3.9.6	Extended protocol configuration options

This IE is included in the message when the network wishes to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the UE.

### 8.3.10	PDU session modification complete

#### 8.3.10.1	Message definition

The PDU SESSION MODIFICATION COMPLETE message is sent by the UE to the SMF in response to the PDU SESSION MODIFICATION COMMAND message and indicates an acceptance of the PDU SESSION MODIFICATION COMMAND message. See table 8.3.10.1.1.

Message type:	PDU SESSION MODIFICATION COMPLETE

Significance:		dual

Direction:			UE to network

Table 8.3.10.1.1: PDU SESSION MODIFICATION COMPLETE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | PDU session ID | PDU session identity 9.4 | M | V | 1 |
|  | PTI | Procedure transaction identity 9.6 | M | V | 1 |
|  | PDU SESSION MODIFICATION COMPLETE message identity | Message type 9.7 | M | V | 1 |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.8.4.2 | O | TLV-E | 4-65538 |

#### 8.3.10.2	Extended protocol configuration options

This IE is included in the message when the UE wishes to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the network.

### 8.3.11	PDU session modification command reject

#### 8.3.11.1	Message definition

The PDU SESSION MODIFICATION COMMAND REJECT message is sent by the UE to the SMF to indicate rejection of the PDU SESSION MODIFICATION COMMAND message. See table 8.3.11.1.1.

Message type:	PDU SESSION MODIFICATION COMMAND REJECT

Significance:		dual

Direction:			UE to network

Table 8.3.11.1.1: PDU SESSION MODIFICATION COMMAND REJECT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | PDU session ID | PDU session identity 9.4 | M | V | 1 |
|  | PTI | Procedure transaction identity 9.6 | M | V | 1 |
|  | PDU SESSION MODIFICATION COMMAND REJECT message identity | Message type 9.7 | M | V | 1 |
|  | 5GSM cause | 5GSM cause 9.8.4.1 | M | V | 1 |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.8.4.2 | O | TLV-E | 4-65538 |

#### 8.3.11.2	Extended protocol configuration options

This IE is included in the message when the UE wishes to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the network.

### 8.3.12	PDU session release request

#### 8.3.12.1	Message definition

The PDU SESSION RELEASE REQUEST message is sent by the UE to the SMF to request a release of a PDU session. See table 8.3.12.1.1.

Message type:	PDU SESSION RELEASE REQUEST

Significance:		dual

Direction:			UE to network

Table 8.3.12.1.1: PDU SESSION RELEASE REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | PDU session ID | PDU session identity 9.4 | M | V | 1 |
|  | PTI | Procedure transaction identity 9.6 | M | V | 1 |
|  | PDU SESSION RELEASE REQUEST message identity | Message type 9.7 | M | V | 1 |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.8.4.2 | O | TLV-E | 4-65538 |

#### 8.3.12.2	Extended protocol configuration options

This IE is included in the message when the UE wishes to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the network.

### 8.3.13	PDU session release reject

#### 8.3.13.1	Message definition

The PDU SESSION RELEASE REJECT message is sent by the SMF to the UE to indicate rejection of request a release of a PDU session. See table 8.3.13.1.1.

Message type:	PDU SESSION RELEASE REJECT

Significance:		dual

Direction:			network to UE

Table 8.3.13.1.1: PDU SESSION RELEASE REJECT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | PDU session ID | PDU session identity 9.4 | M | V | 1 |
|  | PTI | Procedure transaction identity 9.6 | M | V | 1 |
|  | PDU RELEASE RELEASE REJECT message identity | Message type 9.7 | M | V | 1 |
|  | 5GSM cause | 5GSM cause 9.8.4.1 | M | V | 1 |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.8.4.2 | O | TLV-E | 4-65538 |

#### 8.3.13.2	Extended protocol configuration options

This IE is included in the message when the network wishes to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the UE.

### 8.3.14	PDU session release command

#### 8.3.14.1	Message definition

The PDU SESSION RELEASE COMMAND message is sent by the SMF to the UE to indicate a release of a PDU session. See table 8.3.14.1.1.

Message type:	PDU SESSION RELEASE COMMAND

Significance:		dual

Direction:			network to UE

Table 8.3.14.1.1: PDU SESSION RELEASE COMMAND message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | PDU session ID | PDU session identity 9.4 | M | V | 1 |
|  | PTI | Procedure transaction identity 9.6 | M | V | 1 |
|  | PDU SESSION RELEASE COMMAND message identity | Message type 9.7 | M | V | 1 |
|  | 5GSM cause | 5GSM cause 9.8.4.1 | M | V | 1 |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.8.4.2 | O | TLV-E | 4-65538 |

#### 8.3.1.2	Extended protocol configuration options

This IE is included in the message when the network wishes to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the UE.

### 8.3.15	PDU session release complete

#### 8.3.15.1	Message definition

The PDU SESSION RELEASE COMPLETE message is sent by the UE to the SMF in response to the PDU SESSION RELEASE COMMAND message and indicates an acceptance of a release of the PDU session. See table 8.3.15.1.1.

Message type:	PDU SESSION RELEASE COMPLETE

Significance:		dual

Direction:			UE to network

Table 8.3.15.1.1: PDU SESSION RELEASE COMPLETE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | PDU session ID | PDU session identity 9.4 | M | V | 1 |
|  | PTI | Procedure transaction identity 9.6 | M | V | 1 |
|  | PDU SESSION RELEASE COMPLETE message identity | Message type 9.7 | M | V | 1 |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.8.4.2 | O | TLV-E | 4-65538 |

#### 8.3.15.2	Extended protocol configuration options

This IE is included in the message when the UE wishes to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the network.

### 8.3.16	5GSM status

#### 8.3.16.1	Message definition

The 5GSM STATUS message is sent by the SMF or the UE to pass information on the status of the indicated PDU session and report certain error conditions. See table 8.3.16.1.

Message type:	5GSM STATUS

Significance:		dual

Direction:			both

# Table 8.3.16.1: 5GSM STATUS message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | PDU session ID | PDU session identity 9.4 | M | V | 1 |
|  | PTI | Procedure transaction identity 9.6 | M | V | 1 |
|  | 5GSM STATUS message identity | Message type 9.7 | M | V | 1 |
|  | 5GSM cause | 5GSM cause 9.8.4.1 | M | V | 1 |

# 9	General message format and information elements coding

## 9.1	Overview

Within the protocols defined in the present document, every 5GS NAS message is a standard L3 message as defined in 3GPP TS 24.007 [7]. This means that the message consists of the following parts:

1)	if the message is a plain 5GS NAS message:

a)	extended protocol discriminator;

b)	security header type or PDU session identity;

c)	procedure transaction identity;

d)	message type;

e)	other information elements, as required.

2)	if the message is a security protected 5GS NAS message:

Editor's note:	The message parts of a security protected 5GS NAS message are FFS.

The organization of a plain 5GS NAS message is illustrated in the example shown in figure 6.6.1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Extended protocol discriminator |  |  |  |  |  |  |  | octet 1 |
| Security header type associated with a spare half octet; or PDU session identity |  |  |  |  |  |  |  | octet 2 |
| Procedure transaction identity |  |  |  |  |  |  |  | octet 3 |
| Message type |  |  |  |  |  |  |  | octet 4 |
|  |  |  |  |  |  |  |  | octet 5 |
| Other information elements as required |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | octet n |

Figure 6.6.1.1: General message organization example for a plain 5GS NAS message

Editor's note:	The organization of a security protected 5GS NAS message is FFS.

Unless specified otherwise in the message descriptions of clause 8 and clause 9, a particular information element shall not be present more than once in a given message.

When a field extends over more than one octet, the order of bit values progressively decreases as the octet number increases. The least significant bit of the field is represented by the lowest numbered bit of the highest numbered octet of the field.

## 9.2	Extended protocol discriminator

Bits 1 to 8 of the first octet of every 5GS NAS message contain the Extended protocol discriminator (EPD) IE. The EPD and its use are defined in 3GPP TS 24.007 [7].

## 9.3	Security header type

Bits 1 to 4 of the second octet of every 5GMM message contain the Security header type IE. This IE includes control information related to the security protection of a 5GMM message. The total size of the Security header type IE is 4 bits.

The Security header type IE can take the values shown in table 9.3.1.

Table 9.3.1: Security header type

| Security header type (octet 1) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bits |  |  |  |  |
| 4 | 3 | 2 | 1 |  |
| 0 | 0 | 0 | 0 | Plain NAS message, not security protected |
|  |  |  |  | Security protected NAS message: |
| 0 | 0 | 0 | 1 | Integrity protected |
| 0 | 0 | 1 | 0 | Integrity protected and ciphered |
| 0 | 0 | 1 | 1 | Integrity protected with new 5GS security context (NOTE 1) |
| 0 | 1 | 0 | 0 | Integrity protected and ciphered with new 5GS security context (NOTE 2) |
| All other values are reserved. |  |  |  |  |
| NOTE 1: This codepoint may be used only for a SECURITY MODE COMMAND message. NOTE 2: This codepoint may be used only for a SECURITY MODE COMPLETE message. |  |  |  |  |

An 5GMM message received with the security header type encoded as 0000 shall be treated as not security protected, plain NAS message. A protocol entity sending a not security protected 5GMM message shall send the message as plain NAS message and encode the security header type as 0000.

## 9.4	PDU session identity

Bits 1 to 8 of the second octet of every 5GSM message contain the PDU session identity IE. The PDU session identity and its use to identify a message flow are defined in 3GPP TS 24.007 [7]. See Annex E.

## 9.5	Spare half octet

Bits 5 to 8 of the second octet of every 5GMM message contains the spare half octet which is filled with spare bits set to zero.

## 9.6	Procedure transaction identity

Bits 1 to 8 of the third octet of every 5GSM message contain the procedure transaction identity. The procedure transaction identity and its use are defined in 3GPP TS 24.007 [7].

## 9.7	Message type

The Message type IE and its use are defined in 3GPP TS 24.007 [7]. Tables 9.7.1 and 9.7.2 define the value part of the message type IE used in the 5GS mobility management protocol and 5GS session management protocol.

Table 9.7.1: Message types for 5GS mobility management

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 1 | - | - | - | - | - | - |  | 5GS mobility management messages |
| 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 |  | Registration request |
| 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 |  | Registration accept |
| 0 | 1 | 0 | 0 | 0 | 0 | 1 | 1 |  | Registration complete |
| 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 |  | Registration reject |
| 0 | 1 | 0 | 0 | 0 | 1 | 0 | 1 |  | Deregistration request (UE originating) |
| 0 | 1 | 0 | 0 | 0 | 1 | 1 | 0 |  | Deregistration accept (UE originating) |
| 0 | 1 | 0 | 0 | 0 | 1 | 1 | 1 |  | Deregistration request (UE terminated) |
| 0 | 1 | 0 | 0 | 1 | 0 | 0 | 0 |  | Deregistration accept (UE terminated) |
| 0 | 1 | 0 | 0 | 1 | 1 | 0 | 0 |  | Service request |
| 0 | 1 | 0 | 0 | 1 | 1 | 0 | 1 |  | Service reject |
| 0 | 1 | 0 | 0 | 1 | 1 | 1 | 0 |  | Service accept |
| 0 | 1 | 0 | 1 | 0 | 1 | 0 | 0 |  | Configuration update command |
| 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |  | Configuration update complete |
| 0 | 1 | 0 | 1 | 0 | 1 | 1 | 0 |  | Authentication request |
| 0 | 1 | 0 | 1 | 0 | 1 | 1 | 1 |  | Authentication response |
| 0 | 1 | 0 | 1 | 1 | 0 | 0 | 0 |  | Authentication reject |
| 0 | 1 | 0 | 1 | 1 | 0 | 0 | 1 |  | Authentication failure |
| 0 | 1 | 0 | 1 | 1 | 0 | 1 | 0 |  | Identity request |
| 0 | 1 | 0 | 1 | 1 | 0 | 1 | 1 |  | Identity response |
| 0 | 1 | 0 | 1 | 1 | 1 | 0 | 0 |  | Security mode command |
| 0 | 1 | 0 | 1 | 1 | 1 | 0 | 1 |  | Security mode complete |
| 0 | 1 | 0 | 1 | 1 | 1 | 1 | 0 |  | Security mode reject |
| 0 | 1 | 1 | 0 | 0 | 1 | 0 | 0 |  | 5GMM status |
| 0 | 1 | 1 | 0 | 0 | 1 | 0 | 1 |  | Notification |
| 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 |  | DL NAS transport |
| 0 | 1 | 1 | 0 | 0 | 1 | 1 | 1 |  | UL NAS transport |

Table 9.7.2: Message types for 5GS session management

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 1 | 1 | - | - | - | - | - | - |  | 5GS session management messages |
| 1 | 1 | 0 | 0 | 0 | 0 | 0 | 1 |  | PDU session establishment request |
| 1 | 1 | 0 | 0 | 0 | 0 | 1 | 0 |  | PDU session establishment accept |
| 1 | 1 | 0 | 0 | 0 | 0 | 1 | 1 |  | PDU session establishment reject |
| 1 | 1 | 0 | 0 | 0 | 1 | 0 | 1 |  | PDU session authentication request |
| 1 | 1 | 0 | 0 | 0 | 1 | 1 | 0 |  | PDU session authentication accept |
| 1 | 1 | 0 | 0 | 1 | 0 | 0 | 1 |  | PDU session modification request |
| 1 | 1 | 0 | 0 | 1 | 0 | 1 | 0 |  | PDU session modification reject |
| 1 | 1 | 0 | 0 | 1 | 0 | 1 | 1 |  | PDU session modification command |
| 1 | 1 | 0 | 0 | 1 | 1 | 0 | 1 |  | PDU session modification complete |
| 1 | 1 | 0 | 0 | 1 | 1 | 1 | 0 |  | PDU session modification command reject |
| 1 | 1 | 0 | 1 | 0 | 0 | 0 | 1 |  | PDU session release request |
| 1 | 1 | 0 | 1 | 0 | 0 | 1 | 0 |  | PDU session release reject |
| 1 | 1 | 0 | 1 | 0 | 0 | 1 | 0 |  | PDU session release command |
| 1 | 1 | 0 | 1 | 0 | 0 | 1 | 1 |  | PDU session release complete |
| 1 | 1 | 0 | 1 | 0 | 1 | 1 | 0 |  | 5GSM status |

## 9.8	Other information elements

### 9.8.1	General

### 9.8.2	Common information elements

#### 9.8.2.1	Additional information

The purpose of the Additional information information element is to provide additional information to upper layers in relation to the NAS transport mechanism.

The Additional information information element is coded as shown in figure 9.8.2.1.1 and table 9.8.2.1.1.

The Additional information is a type 4 information element with a minimum length of 3 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Additional information IEI |  |  |  |  |  |  |  | octet 1 |
| Additional information length |  |  |  |  |  |  |  | octet 2 |
| Additional information value |  |  |  |  |  |  |  | octets 3-n |

Figure 9.8.2.1.1: Additional information information element

Table 9.8.2.1.1 : Additional information information element

Additional information value (octet 3 to octet n)

The coding of the additional information value is dependent on the LCS application.

#### 9.8.2.2	PDU session status

The purpose of the PDU session status information element is to indicate the state of each PDU session that can be identified by a PDU session identity.

The PDU session status information element is coded as shown in figure 9.8.2.2.1 and table 9.8.2.2.1.

The PDU session status information element is a type 4 information element with minimum length of 4 octets and a maximum length of 34 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PDU session status IEI |  |  |  |  |  |  |  | octet 1 |
| Length of PDU session status contents |  |  |  |  |  |  |  | octet 2 |
| PSI (7) | PSI (6) | PSI (5) | PSI (4) | PSI (3) | PSI (2) | PSI (1) | PSI (0) | octet 3 |
| PSI (15) | PSI (14) | PSI (13) | PSI (12) | PSI (11) | PSI (10) | PSI (9) | PSI (8) | octet 4 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | octet 5*- |
| spare |  |  |  |  |  |  |  | 34* |

Figure 9.8.2.2.1: PDU session status information element

Table 9.8.2.2.1: PDU session status information element

PSI(x) shall be coded as follows: PSI(0) – PSI(4): Bits 1 to 5 of octet 3 are spare and shall be coded as zero. PSI(5) – PSI(15): 0 indicates that the 5GSM state of the corresponding PDU session is PDU SESSION INACTIVE. 1 indicates that the 5GSM state of the corresponding PDU session is not PDU SESSION INACTIVE All bits in octet 5 to 34 are spare and shall be coded as zero, if the respective octet is included in the information element.

#### 9.8.2.3	Uplink data status

The purpose of the Uplink data status information element is to indicate to the network which preserved PDU session contexts have uplink data pending.

The Uplink data status information element is coded as shown in figure 9.8.2.3.1 and table 9.8.2.3.1.

The Uplink data status information element is a type 4 information element with minimum length of 4 octets a maximum length of 34 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Uplink data status IEI |  |  |  |  |  |  |  | octet 1 |
| Length of uplink data status contents |  |  |  |  |  |  |  | octet 2 |
| PSI (7) | PSI (6) | PSI (5) | PSI (4) | PSI (3) | PSI (2) | PSI (1) | PSI (0) | octet 3 |
| PSI (15) | PSI (14) | PSI (13) | PSI (12) | PSI (11) | PSI (10) | PSI (9) | PSI (8) | octet 4 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |
| spare |  |  |  |  |  |  |  | octet 5* -34* |

Figure 9.8.2.3.1: Upllink data status information element

Table 9.8.2.3.1: Uplink data status information element

PSI(x) shall be coded as follows: PSI(0) – PSI(4): Bits 1 to 5 of octet 3 are spare and shall be coded as zero. PSI(5) – PSI(15): 0 indicates that no uplink data are pending for the corresponding PDU session identity. 1 indicates that uplink data are pending for the corresponding PDU session identity. All bits in octet 5 to 34 are spare and shall be coded as zero, if the respective octet is included in the information element.

### 9.9.3	5GS mobility management (5GMM) information elements

#### 9.9.3.1	5GMM capability

The purpose of the 5GMM capability information element is to provide the network with information concerning aspects of the UE related to the 5GCN or interworking with the EPS. The contents might affect the manner in which the network handles the operation of the UE.

The 5GMM capability information element is coded as shown in figure 9.8.3.1.1 and table 9.8.3.1.1.

The 5GMM capability is a type 4 information element with a minimum length of 3 octets and a maximum length of 15 octets.

| 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5GMM capability IEI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 1 |  |
| Length of 5GMM capability contents |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 2 |  |
| 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | S1 mode |  | octet 3 |  |
| 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | octet 4* -15* |  |
| Spare |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Figure 9.8.3.1.1: 5GMM capability information element

Table 9.8.3.1.1: 5GMM capability information element

| EPC NAS supported (S1 mode) (octet 3, bit 1) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 0 |  |  |  |  | S1 mode not supported |
| 1 |  |  |  |  | S1 mode supported |
| All other bits in octet 3 to 15 are spare and shall be coded as zero, if the respective octet is included in the information element. |  |  |  |  |  |

#### 9.8.3.2	5GMM cause

The purpose of the 5GMM cause information element is to indicate the reason why a 5GMM request from the UE is rejected by the network.

The 5GMM cause information element is coded as shown in figure 9.8.3.1.2 and table 9.8.3.1.2.

The 5GMM cause is a type 3 information element with 2 octets length.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5GMM cause IEI |  |  |  |  |  |  |  | octet 1 |
| Cause value |  |  |  |  |  |  |  | octet 2 |

Figure 9.8.3.1.2: 5GMM cause information element

Table 9.8.3.1.2 : 5GMM cause information element

| Cause value (octet 2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bits |  |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |  | Illegal UE |
| 0 | 0 | 0 | 0 | 0 | 1 | 0 | 1 |  | PEI not accepted |
| 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 |  | Illegal ME |
| 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 |  | 5GS services not allowed |
| 0 | 0 | 0 | 0 | 1 | 0 | 1 | 0 |  | Implicitly deregistered |
| 0 | 0 | 0 | 0 | 1 | 0 | 1 | 1 |  | PLMN not allowed |
| 0 | 0 | 0 | 0 | 1 | 1 | 0 | 0 |  | Tracking area not allowed |
| 0 | 0 | 0 | 0 | 1 | 1 | 0 | 1 |  | Roaming not allowed in this tracking area |
| 0 | 0 | 0 | 1 | 0 | 1 | 0 | 1 |  | Synch failure |
| 0 | 0 | 0 | 1 | 1 | 0 | 1 | 1 |  | N1 mode not allowed |
| 0 | 1 | 0 | 1 | 1 | 1 | 1 | 1 |  | Semantically incorrect message |
| 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 |  | Invalid mandatory information |
| 0 | 1 | 1 | 0 | 0 | 0 | 0 | 1 |  | Message type non-existent or not implemented |
| 0 | 1 | 1 | 0 | 0 | 0 | 1 | 0 |  | Message type not compatible with the protocol state |
| 0 | 1 | 1 | 0 | 0 | 0 | 1 | 1 |  | Information element non-existent or not implemented |
| 0 | 1 | 1 | 0 | 0 | 1 | 0 | 0 |  | Conditional IE error |
| 0 | 1 | 1 | 0 | 0 | 1 | 0 | 1 |  | Message not compatible with the protocol state |
| 0 | 1 | 1 | 0 | 1 | 1 | 1 | 1 |  | Protocol error, unspecified |
| Any other value received by the mobile station shall be treated as 0110 1111, "protocol error, unspecified". Any other value received by the network shall be treated as 0110 1111, "protocol error, unspecified". |  |  |  |  |  |  |  |  |  |

#### 9.8.3.3	5GS mobile identity

The purpose of the 5GS mobile identity information element is to provide either the SUCI, the 5G-GUTI or the IMEI.

The 5GS mobile identity information element is coded as shown in figures 9.8.3.3.1 and 9.8.3.3.2 and table 9.8.3.3.1.

The 5GS mobile identity is a type 4 information element with a minimum length of 3 octets and a maximum length of xx2 octets.

Editor's note:	 The definition of 5G-GUTI is FFS, and will be added when defined by CT4.

Figure 9.8.3.3.1: 5GS mobile identity information element for type of identity "5G-GUTI"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5GS mobile identity IEI |  |  |  |  |  |  |  | octet 1 |
| Length of 5GS mobile identity contents |  |  |  |  |  |  |  | octet 2 |
| Identity digit 1 |  |  |  | odd/ even indic | Type of identity |  |  | octet 3 |
| Identity digit p+1 |  |  |  | Identity digit p |  |  |  | octet 4* |

Figure 9.8.3.3.2: 5GS mobile identity information element for type of identity "IMSI" or "IMEI"

Table 9.8.3.3.1: 5GS mobile identity information element

| Type of identity (octet 3) Bits |  |  |  |
| --- | --- | --- | --- |
| 3 | 2 | 1 |  |
| 0 | 0 | 1 | SUCI |
| 1 | 1 | 0 | 5G-GUTI |
| 0 | 1 | 1 | IMEI |
| 1 | 0 | 0 | 5G-TMSI |
| All other values are reserved. |  |  |  |
| Odd/even indication (octet 3) Bit |  |  |  |
| 4 |  |  |  |
| 0 |  |  | even number of identity digits |
| 1 |  |  | odd number of identity digits |
| Identity digits (octet 4 etc) For the SUCI, this field is coded using BCD coding. If the number of identity digits is even then bits 5 to 8 of the last octet shall be filled with an end mark coded as "1111". |  |  |  |
| For the IMEI, this field is coded using BCD coding. The format of the IMEI is described in 3GPP TS 23.003 [2]. |  |  |  |
| If the mobile identity is the 5G-TMSI then bits 5 to 8 of octet 3 are coded as "1111" and bit 8 of octet4 is the most significant bit and bit 1 of the last octet the least significant bit. The coding of the 5G-TMSI is left open for each administration. |  |  |  |

#### 9.8.3.4	5GS network feature support

Editor's note:	The definition of 5GS network feature support is FFS, but should include a dual-registration supported indication.

#### 9.8.3.5	5GS registration result

Editor's note:	The definition of 5GS registration result is FFS.

#### 9.8.3.6	5GS registration type

The purpose of the 5GS registration type information element is to indicate the type of the requested registration.

The 5GS registration type information element is coded as shown in figure 9.8.3.6.1 and table 9.8.3.6.1.

The 5GS registration type is a type 1 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5GS registration type IEI |  |  |  | FOR | 5GS registration type value |  |  | octet 1 |

Figure 9.8.3.6.1: 5GS registration type information element

Table 9.8.3.6.1: 5GS registration type information element

| 5GS registration type value (octet 1) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bits |  |  |  |  |
| 3 | 2 | 1 |  |  |
| 0 | 0 | 1 |  | initial registration |
| 0 | 1 | 0 |  | mobility registration updating |
| 0 | 1 | 1 |  | periodic registration updating |
| 1 | 1 | 0 |  | 5GS emergency registration |
| 1 | 1 | 1 |  | reserved |
| All other values are unused and shall be interpreted as "initial registration", if received by the network. |  |  |  |  |
| Follow-On Request bit (FOR) (octet 1) |  |  |  |  |
| Bit |  |  |  |  |
| 4 |  |  |  |  |
| 0 |  |  |  | No follow-on request pending |
| 1 |  |  |  | Follow-on request pending |

#### 9.8.3.7	Allowed PDU session status

The purpose of the Allowed PDU session status information element is to indicate to the network user-plane resources of PDU sessions associated with non-3GPP access that are allowed to be re-activated over 3GPP access.

The Allowed PDU session status information element is coded as shown in figure 9.8.3.6.1 and table 9.8.3.6.1.

The Allowed PDU session status is a type 4 information element with minimum length of 4 octets and maximum length of 34 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Allowed PDU session status IEI |  |  |  |  |  |  |  | octet 1 |
| Length of Allowed PDU session status contents |  |  |  |  |  |  |  | octet 2 |
| PSI (7) | PSI (6) | PSI (5) | PSI (4) | PSI (3) | PSI (2) | PSI (1) | PSI (0) | octet 3 |
| PSI (15) | PSI (14) | PSI (13) | PSI (12) | PSI (11) | PSI (10) | PSI (9) | PSI (8) | octet 4 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |
| Spare |  |  |  |  |  |  |  | octet 5* -34* |

Figure 9.8.3.6.1: Allowed PDU session status information element

Table 9.8.3.6.1: Allowed PDU session status information element

PSI(x) shall be coded as follows: PSI(0) – PSI(4): Bits 1 to 5 of octet 3 are spare and shall be coded as zero. PSI(5) – PSI(15): 0 indicates that the user-plane resources of corresponding PDU session is not allowed to be re-activated over 3GPP access. 1 indicates that the user-plane resources of corresponding PDU session can be re-activated over 3GPP access. All bits in octet 5 to 34 are spare and shall be coded as zero, if the respective octet is included in the information element.

#### 9.8.3.8	Authentication parameter AUTN

See subclause 10.5.3.1.1 in 3GPP TS 24.008 [8].

Editor's note:	The format of the Authentication parameter AUTN IE in 3GPP TS 24.008 is TLV with a length of 18 octets.

#### 9.8.3.9	Authentication parameter RAND

See subclause 10.5.3.1 in 3GPP TS 24.008 [8].

Editor's note:	The format of the Authentication parameter RAND IE in 3GPP TS 24.008 is TV with a length of 17 octets.

#### 9.8.3.10	Configuration update indication

The purpose of the Configuration update indication information element is to indicate the additional information associated with the generic UE configuration update procedure.

The Configuration update indication information element is coded as shown in figure 9.8.3.9.1 and table 9.8.3.9.1.

The Configuration update indication is a type 1 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Configuration update indication IEI |  |  |  | 0 Spare | 0 Spare | RED | ACK | octet 1 |

Figure 9.8.3.9.1: Configuration update indication

Table 9.9.3.9.1: Configuration update indication

| Acknowledgement (ACK) value (octet 1, bit 1) |  |
| --- | --- |
| Bits |  |
| 1 |  |
| 0 | acknowledgement not requested |
| 1 | acknowledgement requested |
| Registration requested (RED) value (octet 1, bit 2) |  |
| Bits |  |
| 1 |  |
| 0 | registration not requested |
| 1 | registration requested |
| Bits 3 and 4 are spare and shall be coded as zero, |  |

#### 9.8.3.11	Daylight saving time

See subclause 10.5.3.12 in 3GPP TS 24.008 [8].

#### 9.8.3.12	De-registration type

The purpose of the De-registration type information element is to indicate the type of de-registration.

The De-registration type information element is coded as shown in figure 9.8.3.11.1 and table 9.8.3.11.1.

The De-registration type is a type 1 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| De-registration type IEI |  |  |  | Switch off | Re-registration required | Access type |  | octet 1 |

Figure 9.8.3.11.1: Deregistration type information element

Table 9.8.3.11.1: Deregistration type information element

| Switch off (octet 1, bit 4) |  |  |  |  |
| --- | --- | --- | --- | --- |
| In the UE to network direction: |  |  |  |  |
| Bit |  |  |  |  |
| 4 |  |  |  |  |
| 0 |  |  |  | Normal detach |
| 1 |  |  |  | Switch off |
| In the network to UE direction bit 4 is spare. The network shall set this bit to zero. |  |  |  |  |
| Re-registration required (octet 1, bit 3) |  |  |  |  |
| In the network to UE direction: |  |  |  |  |
| Bit |  |  |  |  |
| 3 |  |  |  |  |
| 0 |  |  |  | re-registration not required |
| 1 |  |  |  | re-registration required |
| In the UE to network direction bit 3 is spare. The UE shall set this bit to zero. |  |  |  |  |
| Access type (octet 1,bit 2, bit 1) |  |  |  |  |
| Bit |  |  |  |  |
| 2 | 1 |  |  |  |
| 0 | 1 |  |  | 3GPP access |
| 1 | 0 |  |  | Non-3GPP access |
| 1 | 1 |  |  | 3GPP access and non-3GPP access |
| All other values are reserved. |  |  |  |  |

#### 9.8.3.13	DNN

The purpose of the DNN information element is to identify the data network.

The DNN information element is coded as shown in figure 9.8.3.12.1.

The DNN is a type 4 information element with a minimum length of 3 octets and a maximum length of 102 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DNN IEI |  |  |  |  |  |  |  | octet 1 |
| Length of DNN contents |  |  |  |  |  |  |  | octet 2 |
| DNN value |  |  |  |  |  |  |  | octet 3 octet n |

Figure 9.8.3.12.1: DNN information element

A DNN value field contains an APN as defined in 3GPP TS 23.003 [2].

#### 9.8.3.14	EAP message

The purpose of the EAP message information element is to transport an EAP message as specified in IETF RFC 3748 [19].

The EAP message information element is coded as shown in figure 9.8.3.14.1 and table 9.8.3.14.1.

The EAP message is a type 6 information element with minimum length of 7 octets and maximum length of 1503 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EAP message IEI |  |  |  |  |  |  |  | octet 1 |
| Length of EAP message contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| EAP message |  |  |  |  |  |  |  | octet 4 octet x |

Figure 9.8.3.14.1: EAP message information element

Table 9.8.3.14.1: EAP message information element

EAP message (octet 4 to x)

An EAP message as specified in IETF RFC 3748 [19].

#### 9.8.3.15	EPS NAS message container

The purpose of the EPS NAS message container information element is to transport an EPS NAS message as specified in 3GPP TS 24.301 [10].

The EPS NAS message containerinformation element is coded as shown in figure 9.8.3.15.1 and table 9.8.3.15.1.

The EPS NAS message container is a type 6 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EPS NAS message container IEI |  |  |  |  |  |  |  | octet 1 |
| Length of EPS NAS message container contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| EPS NAS message container |  |  |  |  |  |  |  | octet 4 octet x |

Figure 9.8.3.15.1: EPS NAS message container information element

Table 9.8.3.15.1: EPS NAS message container information element

EPS NAS message container (octet 4 to x)

An EPS NAS message as specified in 3GPP TS 24.301 [10].

#### 9.8.3.16	GPRS timer 2

See subclause 10.5.7.4 in 3GPP TS 24.008 [8].

#### 9.8.3.17	HashAMF

The purpose of the HashAMF information element is to transfer a 64-bit hash value to the UE so the UE can check the AMF calculated value against the value locally calculated by the UE to determine whether the REGISTRATION REQUEST message sent by the UE has been modified.

The HashAMF information element is coded as shown in figure 9.8.3.17.1 and table 9.8.3.17.1.

The HashAMF is a type 3 information element with a length of 9 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HashAMF IEI |  |  |  |  |  |  |  | octet 1 |
| HashAMF value |  |  |  |  |  |  |  | octet 2 octet 9 |

Figure 9.8.3.17.1: HashAMF information element

Table 9.8.3.17.1: HashAMF information element

HashAMF value (octet 2 to 9)

This field contains the binary representation of the HashAMF. Bit 8 of octet 2 represents the most significant bit of the HashAMF and bit 1 of octet 9 the least significant bit.

#### 9.8.3.18	IMEISV request

See subclause 10.5.5.10 in 3GPP TS 24.008 [13].

#### 9.8.3.19	LADN information

The purpose of the LADN information information element is to provide the UE the LADN service area for each available LADN in the current registration area.

The LADN information information element is coded as shown in figure 9.8.3.19.1, figure 9.8.3.19.2 and table 9.8.3.19.1.

The LADN information is a type 6 information element with a minimum length of 11 octets and a maximum length of 1579 octets.

The LADN information information element can contain a maximum of 8 different LADNs each including a DNN and a tracking area identity lists.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LADN information IEI |  |  |  |  |  |  |  | octet 1 |
| Length of LADN information contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| LADN 1 |  |  |  |  |  |  |  | octet 4 octet a |
| LADN 2 |  |  |  |  |  |  |  | octet a+1* octet b* |
| … |  |  |  |  |  |  |  | octet b+1* octet g* |
| LADN n |  |  |  |  |  |  |  | octet g+1* octet h* |

Figure 9.8.3.19.1: LADN information information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of DNN value |  |  |  |  |  |  |  | octet 1 |
| DNN value |  |  |  |  |  |  |  | octet 2 octet a |
| Tracking area identity list contents |  |  |  |  |  |  |  | octet a+1 octet b |

Figure 9.8.3.19.2: LADN

Table 9.8.3.19.1 : LADN information information element

Value part of the LADN information information element (octet 3 to h) The value part of the LADN information information element consists of one or several LADNs. Each LADN consists one DNN value and one Tracking area identity list contents. The length of each LADN is determined by the length of DNN value and the length of Tracking area identity list contents. The UE shall store the complete list received. If more than 8 LADNs are included in this information element, the UE shall store the first 8 LADNs and ignore the remaining octets of the information element. LADN (octet 4 to octet a): DNN value is coded as DNN value part of DNN information element as specified in subclause 9.8.3.12 starting with the third octet.

Tracking area identity list contents (octet a+1 to octet b):

Tracking area identity list contents is coded as the value part of the Tracking area identity list information element as specified in subclause 9.8.3.45 starting with the third octet.

#### 9.8.3.20	Message authentication code

The message authentication code (MAC) information element contains the integrity protection information for the message. The MAC IE shall be included in the security protected NAS message if a valid 5GS security context exists and security functions are started.

Editor's note:	The usage of MAC is FFS.

#### 9.8.3.21	MICO indication

The purpose of the MICO indication information element is to indicate the use of MICO mode or the re-negotiation of MICO mode.

The MICO indication information element is coded as shown in figure 9.8.3.21.1 and table 9.8.3.21.1.

The MICO indication is a type 1 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MICO indication IEI |  |  |  | 0 Spare | 0 Spare | 0 Spare | RAAI | octet 1 |

Figure 9.8.3.21.1: MICO indication

Table 9.8.3.21.1: MICO indication

| Registration Area Allocation Indication (RAAI) (octet 1) |  |
| --- | --- |
| In the network to UE direction: Bits |  |
| 1 |  |
| 0 | all PLMN registration area not allocated |
| 1 | all PLMN registration area allocated |
| In the UE to network direction bit 1 is spare. The UE shall set this bit to zero. |  |
| Bits 2, 3 and 4 are spare and shall be coded as zero, |  |

#### 9.8.3.22	NAS key set identifier

The NAS key set identifier is allocated by the network.

The NAS key set identifier information element is coded as shown in figure 9.8.3.22.1 and table 9.8.3.22.1.

The NAS key set identifier is a type 1 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NAS key set identifier IEI |  |  |  | TSC | NAS key set identifier |  |  | octet 1 |

# Figure 9.8.3.22.1: NAS key set identifier information element

Table 9.8.3.22.1: NAS key set identifier information element

| Type of security context flag (TSC) (octet 1) |  |  |  |
| --- | --- | --- | --- |
| Bit |  |  |  |
| 4 |  |  |  |
| 0 |  |  | native security context (for KSIAMF) |
| 1 |  |  | mapped security context (for KSIASME) |
| TSC does not apply for NAS key set identifier value "111". |  |  |  |
| NAS key set identifier (octet 1) |  |  |  |
| Bits |  |  |  |
| 3 | 2 | 1 |  |
| 0 | 0 | 0 |  |
| through |  |  | possible values for the NAS key set identifier |
| 1 | 1 | 0 |  |
| 1 | 1 | 1 | no key is available (UE to network); |
|  |  |  | reserved (network to UE) |

#### 9.8.3.23	NAS message container

The purpose of the NAS message container IE is to encapsulate a NAS message without NAS security header.

The NAS message container information element is coded as shown in figure 9.8.3.23.1 and table 9.8.3.23.1.

The NAS message container is a type 6 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NAS message container IEI |  |  |  |  |  |  |  | octet 1 |
| Length of NAS message container contents |  |  |  |  |  |  |  | octet 2 |
|  |  |  |  |  |  |  |  | octet 3 |
|  |  |  |  |  |  |  |  | octet 4 |
| NAS message container contents |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | octet n |

# Figure 9.8.3.23.1: NAS message container information element

Table 9.8.3.23.1: NAS message container information element

NAS message container contents (octet 4 to octet n); Max value of 65535 octets

This IE can contain a REGISTRATION REQUEST message as defined in subclause 5.5.1, or a SERVICE REQUEST message as defined in subclause 5.6.1.

#### 9.8.3.24	NAS security algorithms

The purpose of the NAS security algorithms information element is to indicate the algorithms to be used for ciphering and integrity protection.

The NAS security algorithms information element is coded as shown in figure 9.8.3.24.1 and table 9.8.3.24.1.

The NAS security algorithms is a type 3 information element with a length of 2 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NAS security algorithms IEI |  |  |  |  |  |  |  | octet 1 |
| Type of ciphering algorithm |  |  |  | Type of integrity protection algorithm |  |  |  | octet 2 |

# Figure 9.8.3.24.1: NAS security algorithms information element

Table 9.8.3.24.1: NAS security algorithms information element

| Type of integrity protection algorithm (octet 2, bit 1 to 3) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bits |  |  |  |  |
| 4 | 3 | 2 | 1 |  |
| 0 | 0 | 0 | 0 | 5GS integrity algorithm 5G-IA0 (null integrity protection algorithm) |
| 0 | 0 | 0 | 1 | 5GS integrity algorithm 128-5G-IA1 |
| 0 | 0 | 1 | 0 | 5GS integrity algorithm 128-5G-IA2 |
| 0 | 0 | 1 | 1 | 5GS integrity algorithm 128-5G-IA3 |
| 0 | 1 | 0 | 0 | 5GS integrity algorithm 5G-IA4 |
| 0 | 1 | 0 | 1 | 5GS integrity algorithm 5G-IA5 |
| 0 | 1 | 1 | 0 | 5GS integrity algorithm 5G-IA6 |
| 0 | 1 | 1 | 1 | 5GS integrity algorithm 5G-IA7 |
| All other values are reserved. |  |  |  |  |
| Type of ciphering algorithm (octet 2, bit 5 to 7) |  |  |  |  |
| Bits |  |  |  |  |
| 8 | 7 | 6 | 5 |  |
| 0 | 0 | 0 | 0 | 5GS encryption algorithm 5G-EA0 (null ciphering algorithm) |
| 0 | 0 | 0 | 1 | 5GS encryption algorithm 128-5G-EA1 |
| 0 | 0 | 1 | 0 | 5GS encryption algorithm 128-5G-EA2 |
| 0 | 0 | 1 | 1 | 5GS encryption algorithm 128-5G-EA3 |
| 0 | 1 | 0 | 0 | 5GS encryption algorithm 5G-EA4 |
| 0 | 1 | 0 | 1 | 5GS encryption algorithm 5G-EA5 |
| 0 | 1 | 1 | 0 | 5GS encryption algorithm 5G-EA6 |
| 0 | 1 | 1 | 1 | 5GS encryption algorithm 5G-EA7 |
| All other values are reserved. |  |  |  |  |

#### 9.8.3.25	NAS security parameters to NG-RAN

The purpose of the NAS security parameters to NG-RAN information element is to provide the UE with parameters that enable the UE to create a mapped 5GS security context and take this context into use after inter-system handover to N1 mode.

The NAS security parameters to NG-RAN information element is coded as shown in figure 9.8.3.25.1 and table 9.8.3.25.1.

The NAS security parameters to NG-RAN is a type 3 information element with a length of 7 octets.

The value part of the NAS security parameters to NG-RAN information element is included in specific information elements within some RRC messages sent to the UE.

NOTE:	For these cases the coding of the information element identifier and length information of RRC is defined in 3GPP TS 38.331 [15].

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NAS security parameters to NG-RAN IEI |  |  |  |  |  |  |  | octet 1 |
| NonceAMF value |  |  |  |  |  |  |  | octet 2 octet 5 |
| Type of ciphering algorithm |  |  |  | Type of integrity protection algorithm |  |  |  | octet 6 |
| 0 | 0 | 0 | 0 | TSC | NAS key set identifier |  |  | octet 7 |
| spare |  |  |  |  |  |  |  |  |

Figure 9.8.3.25.1: NAS security parameters to NG-RAN information element

Table 9.8.3.25.1: NAS security parameters to NG-RAN information element

NonceAMF value (octet 2 to 5)

This field is coded as the nonce value in the Nonce information element (see subclause 9.8.3.27).

Type of integrity protection algorithm (octet 6, bit 1 to 4) and type of ciphering algorithm (octet 6, bit 5 to 8)

These fields are coded as the type of integrity protection algorithm and type of ciphering algorithm in the NAS security algorithms information element (see subclause 9.8.3.24).

NAS key set identifier (octet 7, bit 1 to 3) and type of security context flag (TSC) (octet 7, bit 4)

These fields are coded as the NAS key set identifier and type of security context flag in the NAS key set identifier information element (see subclause 9.8.3.19).

Bit 5 to 8 of octet 7 are spare and shall be coded as zero.

#### 9.8.3.26	Network name

See subclause 10.5.3.5a in 3GPP TS 24.008 [8].

#### 9.8.3.27	Nonce

See subclause 9.9.3.25 in 3GPP TS 24.301 [10].

#### 9.8.3.28	NSSAI

The purpose of the NSSAI information element is to identify a collection of S-NSSAIs

The NSSAI information element is coded as shown in figure 9.8.3.28.1 and table 9.8.3.28.1.

The S-NSSAI is a type 4 information element with a minimum length of 4 octets and a maximum length of 74 octets.

NOTE:	The number of S-NSSAI(s) not including mapped configured S-NSSAI(s) cannot exceed eight.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NSSAI IEI |  |  |  |  |  |  |  | octet 1 |
| Length of NSSAI contents |  |  |  |  |  |  |  | octet 2 |
| S-NSSAI 1 |  |  |  |  |  |  |  | octet 3 octet m |
| S-NSSAI 2 |  |  |  |  |  |  |  | octet m+1* octet n* |
| … |  |  |  |  |  |  |  | octet n+1* octet u* |
| S-NSSAI n |  |  |  |  |  |  |  | octet u+1* octet v* |

Figure 9.8.3.28.1: NSSAI information element

Table 9.8.3.28.1: NSSAI information element

Value part of the NSSAI information element (octet 3 to v) The value part of the NSSAI information element consists of one or more S-NSSAIs. Each S-NSSAI consists of one S-NSSAI and optionally one mapped configured S-NSSAI from the configured NSSAI for the HPLMN. The UE shall store the complete list received. If more than 8 S-NSSAIs are included in this information element, the UE shall store the first 8 S-NSSAIs and ignore the remaining octets of the information element. S-NSSAI: S-NSSAI value is coded as the length and value part of S-NSSAI information element as specified in subclause 9.8.3.37 starting with the second octet.

#### 9.8.3.29	NSSAI info for PDU sessions

The purpose of the NSSAI info for PDU sessions information element is to provide the list of PDU session identities and the associated S-NSSAI that have been established via EPS.

The NSSAI info for PDU sessions information element is coded as shown in figure 9.8.3.29.1 and table 9.8.3.29.1.

The NSSAI info for PDU sessions information element is a type 4 information element with a minimum length of 6 octets and a maximum length of 90 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NSSAI info for PDU session IEI |  |  |  |  |  |  |  | octet 1 |
| Length of NSSAI info for PDU sessions contents |  |  |  |  |  |  |  | octet 2 |
| PDU session identity 1 |  |  |  |  |  |  |  | octet 3 |
| S-NSSAI 1 |  |  |  |  |  |  |  | octet 4 octet m |
| PDU session identity 2 |  |  |  |  |  |  |  | octet m+1* |
| S-NSSAI 2 |  |  |  |  |  |  |  | octet m+2* octet n* |
| … |  |  |  |  |  |  |  | octet n+1* |
| … |  |  |  |  |  |  |  | octet n+2* octet u* |
| PDU session identity n |  |  |  |  |  |  |  | octet u+1* |
| S-NSSAI n |  |  |  |  |  |  |  | octet u+2* octet v* |

Figure 9.8.3.29.1: NSSAI info for PDU sessions information element

Table 9.8.3.29.1: NSSAI info for PDU sessions information element

NSSAI info for PDU session IEI (octet 1) Length of NSSAI info for PDU sessions contents (octet 2) PDU session identity 1 (octet 3) The encoding of the PDU session identity field is described in 3GPP TS 24.007 [7].

S-NSSAI (octet 4 to m) The encoding of the S-NSSAI field is shown in figure 9.8.3.37.2 and table 9.8.3.37.1. NOTE 1: If the octet m+1 is included, then octet m+1 through octet n shall be included.

#### 9.8.3.30	Payload container

The purpose of the Payload container information element is to transport a payload.

The Payload container information element is coded as shown in figure 9.8.3.30.1 and table 9.8.3.30.1.

The Payload container is a type 6 information element with a minimum length of 3 octets and a maximum length of 65537 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of payload container contents |  |  |  |  |  |  |  | octet 1 |
|  |  |  |  |  |  |  |  | octet 2 |
|  |  |  |  |  |  |  |  | octet 3 |
| Payload container contents |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | octet n |

Figure 9.8.3.30.1: Payload container information element

Table 9.8.3.30.1: Payload container information element

Payload container contents (octet 3 to octet n); max value of 65535 octets

#### 9.8.3.31	Payload container type

The purpose of the Payload container type information element indicates type of payload included in the payload container information element.

The Payload container information element is coded as shown in figure 9.8.3.31.1 and table 9.8.3.31.1.

The Payload container is a type 1 information element with a length of half octet.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - | - | - | - | Payload container type value |  |  |  | octet 1 |

Figure 9.8.3.31.1: Payload container information element

Table 9.8.3.31.1: Payload container information element

| Payload container type value (octet 1, bit 1 to bit 4) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bits |  |  |  |  |
| 4 | 3 | 2 | 1 |  |
| 0 | 0 | 0 | 1 | N1 SM information |
| 0 | 0 | 1 | 0 | SMS |
| 0 | 0 | 1 | 1 | LTE Positioning Protocol (LPP) message container |
| All other values are reserved. |  |  |  |  |

#### 9.8.3.32	PDU session reactivation result

The purpose of the PDU session reactivation result information element is to indicate the result of PDU sessions activation.

The PDU session reactivation result information element is coded as shown in figure 9.8.3.32.1 and table 9.8.3.32.1.

The PDU session reactivation result is a type 4 information element with minimum length of 4 octets and maximum length of 34 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PDU session reactivation result IEI |  |  |  |  |  |  |  | octet 1 |
| Length of PDU session reactivation result |  |  |  |  |  |  |  | octet 2 |
| PSI (7) | PSI (6) | PSI (5) | PSI (4) | PSI (3) | PSI (2) | PSI (1) | PSI (0) | octet 3 |
| PSI (15) | PSI (14) | PSI (13) | PSI (12) | PSI (11) | PSI (10) | PSI (9) | PSI (8) | octet 4 |
| 0; 0; 0; 0; 0; 0; 0; 0. Spare. | octet 5* -34* |

Figure 9.8.3.32.1: PDU session reactivation result information element

Table 9.8.3.32.1: PDU session reactivation result information element

PSI(x) shall be coded as follows: PSI(0) – PSI(4): Bits 0 to 4 of octet 3 are spare and shall be coded as zero. PSI(5) – PSI(15): 0 indicates PDU session reactivation was not requested in Uplink data status IE or User plane reactivation is successful. 1 indicates PDU session reactivation was requested in Uplink data status IE but User plane reactivation is not successful. All bits in octet 5 to 34 are spare and shall be coded as zero, if the respective octet is included in the information element.

Editor's note:	Coding of the reject cause value for the case of user plane reactivation failure of corresponding PDU session ID is FFS.

#### 9.8.3.33	PLMN list

See subclause 10.5.1.13 in 3GPP TS 24.008 [8].

#### 9.8.3.34	Old PDU session identity

The purpose of the Old PDU session identity information element is to indicate the PDU session identity associated with the PDU SESSION MODIFICATION COMMAND message with the 5GSM cause IE set to #39 "reactivation requested".

The Old PDU session identity information element is coded as shown in figure 9.8.3.34.1 and table 9.8.3.34.1.

The Old PDU session identity is a type 3 information element with 2 octet length.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Old PDU session identity IEI |  |  |  |  |  |  |  | octet 1 |
| Old PDU session identity value |  |  |  |  |  |  |  | octet 2 |

Figure 9.8.3.34.1: Old PDU session identity information element

Table 9.8.3.34.1: Old PDU session identity information element

Old PDU session identity value (octet 2) The coding of the old PDU session identity value is identical to the coding of the PDU session identity value (see annex E.1).

#### 9.8.3.35	Rejected NSSAI

The purpose of the Rejected NSSAI information element is to identify a collection of rejected S-NSSAIs

The Rejected NSSAI information element is coded as shown in figure 9.8.3.35.1, figure 9.8.3.35.2 and table 9.8.3.35.1.

The Rejected NSSAI is a type 4 information element with a minimum length of 4 octets and a maximum length of 42 octets.

NOTE:	The number of rejected S-NSSAI(s) cannot exceed eight.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rejected NSSAI IEI |  |  |  |  |  |  |  | octet 1 |
| Length of Rejected NSSAI contents |  |  |  |  |  |  |  | octet 2 |
| Rejected S-NSSAI 1 |  |  |  |  |  |  |  | octet 3 octet m |
| Rejected S-NSSAI 2 |  |  |  |  |  |  |  | octet m+1* octet n* |
| … |  |  |  |  |  |  |  | octet n+1* octet u* |
| Rejected S-NSSAI n |  |  |  |  |  |  |  | octet u+1* octet v* |

Figure 9.8.3.35.1: Rejected NSSAI information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of rejected S-NSSAI |  |  |  | Cause value |  |  |  | octet 1 |
| SST |  |  |  |  |  |  |  | octet 2 |
| SD |  |  |  |  |  |  |  | octet 3* octet 5* |

Figure 9.8.3.35.2: Rejected S-NSSAI

Table 9.8.3.35.1: Rejected NSSAI information element

| Value part of the Rejected NSSAI information element (octet 3 to v) The value part of the Rejected NSSAI information element consists of one or more rejected S-NSSAIs. Each rejected S-NSSAI consists of one S-NSSAI and an associated cause value. The length of each rejected S-NSSAI can be determined by the 'length of rejected S-NSSAI' field in the first octet of the rejected S-NSSAI. The UE shall store the complete list received. If more than 8 rejected S-NSSAIs are included in this information element, the UE shall store the first 8 rejected S-NSSAIs and ignore the remaining octets of the information element. Rejected S-NSSAI: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Cause value (octet 1) |  |  |  |  |  |  |  |
| Bits |  |  |  |  |  |  |  |
| 4 | 3 | 2 | 1 |  |  |  |  |
| 0 | 0 | 0 | 0 | S-NSSAI not available in the current PLMN |  |  |  |
| 0 | 0 | 0 | 1 | S-NSSAI not available in the current registration area |  |  |  |
| All other values are reserved. |  |  |  |  |  |  |  |
| Slice/service type (SST) (octet 2) This field contains the 8 bit SST value. The coding of the SST value part is defined in 3GPP TS 23.003 [2]. Slice differentiator (SD) (octet 3 to octet 5) This field contains the 24 bit SD value. The coding of the SD value part is defined in 3GPP TS 23.003 [2]. |  |  |  |  |  |  |  |
| NOTE: If octet 3 is included, then octet 4 and octet 5 shall be included. |  |  |  |  |  |  |  |

#### 9.8.3.36	S1 UE network capability

See subclause 9.9.3.34 in 3GPP TS 24.301 [10].

#### 9.8.3.37	S-NSSAI

The purpose of the S-NSSAI information element is to identify a network slice.

The S-NSSAI information element is coded as shown in figure 9.8.3.37.1 and table 9.8.3.37.1.

The S-NSSAI is a type 4 information element with a minimum length of 3 octets and a maximum length of 10 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-NSSAI IEI |  |  |  |  |  |  |  | octet 1 |
| Length of S-NSSAI contents |  |  |  |  |  |  |  | octet 2 |
| SST |  |  |  |  |  |  |  | octet 3 |
| SD |  |  |  |  |  |  |  | octet 4* octet 6* |
| Mapped configured SST |  |  |  |  |  |  |  | octet 7* |
| Mapped configured SD |  |  |  |  |  |  |  | octet 8* octet 10* |

Figure 9.8.3.37.1: S-NSSAI information element

Table 9.8.3.37.1: S-NSSAI information element

Slice/service type (SST) (octet 3)

This field contains the 8 bit SST value. The coding of the SST value part is defined in 3GPP TS 23.003 [2].

Slice differentiator (SD) (octet 4 to octet 6) This field contains the 24 bit SD value. The coding of the SD value part is defined in 3GPP TS 23.003 [2].

mapped configured Slice/service type (SST) (octet 7)

This field contains the 8 bit SST value of an S-NSSAI in the configured NSSAI for the HPLMN to which the SST value is mapped. The coding of the SST value part is defined in 3GPP TS 23.003 [2].

mapped configured Slice differentiator (SD) (octet 8 to octet 10) This field contains the 24 bit SD value of an S-NSSAI in the configured NSSAI for the HPLMN to which the SD value is mapped. The coding of the SD value part is defined in 3GPP TS 23.003 [2].

NOTE 1: If the octet 4 is included, then octet 5 and octet 6 shall be included. NOTE 2: If the octet 8 is included, then octet 9 and octet 10 shall be included.

#### 9.8.3.38	Sequence number

This IE includes the NAS message sequence number (SN).

Editor's note:	Further description of the SN would be aligned with 3GPP TS 33.501 [12]. The usage of SN is FFS.

#### 9.8.3.39	Service area list

The purpose of the Service area list information element is to transfer a list of allowed tracking areas for an allowed area or a list of non-allowed tracking areas for a non-allowed area from the network to the UE.

The coding of the information element allows combining different types of lists. The lists of type "00" and "01" allow a more compact encoding, when the different TAIs are sharing the PLMN identity. The lists of type "11" indicate all TAIs in the PLMN are allowed area.

The Service area list information element is coded as shown in figure 9.8.3.39.1, figure 9.8.3.39.2, figure 9.8.3.39.3, figure 9.8.3.39.4, figure 9.8.3.39.5 and table 9.8.3.39.1.

The Service area list is a type 4 information element with a minimum length of 6 octets and a maximum length of 98 octets. The list can contain a maximum of 16 different tracking area identities.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Service area list IEI |  |  |  |  |  |  |  | octet 1 |
| Length of service area list contents |  |  |  |  |  |  |  | octet 2 |
| Partial service area list 1 |  |  |  |  |  |  |  | octet 3 octet i |
| Partial service area list 2 |  |  |  |  |  |  |  | octet i+1* octet l* |
| … |  |  |  |  |  |  |  | octet l+1* octet m* |
| Partial service area list p |  |  |  |  |  |  |  | octet m+1* octet n* |

Figure 9.8.3.39.1: Service area list information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Allowed type | Type of list |  | Number of elements |  |  |  |  | octet 1 |
| MCC digit 2 |  |  |  | MCC digit 1 |  |  |  | octet 2 |
| MNC digit 3 |  |  |  | MCC digit 3 |  |  |  | octet 3 |
| MNC digit 2 |  |  |  | MNC digit 1 |  |  |  | octet 4 |
| TAC 1 |  |  |  |  |  |  |  | octet 5 |
| TAC 1 (continued) |  |  |  |  |  |  |  | octet 6 |
| … |  |  |  |  |  |  |  | … |
| … |  |  |  |  |  |  |  | … |
| TAC k |  |  |  |  |  |  |  | octet 2k+3* |
| TAC k (continued) |  |  |  |  |  |  |  | octet 2k+4* |

Figure 9.8.3.39.2: Partial service area list – type of list = "00"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Allowed type | Type of list |  | Number of elements |  |  |  |  | octet 1 |
| MCC digit 2 |  |  |  | MCC digit 1 |  |  |  | octet 2 |
| MNC digit 3 |  |  |  | MCC digit 3 |  |  |  | octet 3 |
| MNC digit 2 |  |  |  | MNC digit 1 |  |  |  | octet 4 |
| TAC 1 |  |  |  |  |  |  |  | octet 5 |
| TAC 1 (continued) |  |  |  |  |  |  |  | octet 6 |

Figure 9.8.3.39.3: Partial service area list – type of list = "01"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Allowed type | Type of list |  | Number of elements |  |  |  |  | octet 1 |
| MCC digit 2 |  |  |  | MCC digit 1 |  |  |  | octet 2 |
| MNC digit 3 |  |  |  | MCC digit 3 |  |  |  | octet 3 |
| MNC digit 2 |  |  |  | MNC digit 1 |  |  |  | octet 4 |
| TAC 1 |  |  |  |  |  |  |  | octet 5 |
| TAC 1 (continued) |  |  |  |  |  |  |  | octet 6 |
| MCC digit 2 |  |  |  | MCC digit 1 |  |  |  | octet 7* |
| MNC digit 3 |  |  |  | MCC digit 3 |  |  |  | octet 8* |
| MNC digit 2 |  |  |  | MNC digit 1 |  |  |  | octet 9* |
| TAC 2 |  |  |  |  |  |  |  | octet 10* |
| TAC 2 (continued) |  |  |  |  |  |  |  | octet 11* |
| … |  |  |  |  |  |  |  |  |
| … |  |  |  |  |  |  |  |  |
| MCC digit 2 |  |  |  | MCC digit 1 |  |  |  | octet 5k-3* |
| MNC digit 3 |  |  |  | MCC digit 3 |  |  |  | octet 5k-2* |
| MNC digit 2 |  |  |  | MNC digit 1 |  |  |  | octet 5k-1* |
| TAC k |  |  |  |  |  |  |  | octet 5k* |
| TAC k (continued) |  |  |  |  |  |  |  | octet 5k+1* |

Figure 9.8.3.39.4: Partial service area list – type of list = "10"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Allowed type | Type of list |  | Number of elements |  |  |  |  | octet 1 |
| MCC digit 2 |  |  |  | MCC digit 1 |  |  |  | octet 2 |
| MNC digit 3 |  |  |  | MCC digit 3 |  |  |  | octet 3 |
| MNC digit 2 |  |  |  | MNC digit 1 |  |  |  | octet 4 |

Figure 9.8.3.39.5: Partial service area list – type of list = "11"

Table 9.8.3.39.1: Service area list information element

| Value part of the Service area list information element (octet 3 to n) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The value part of the Service area list information element consists of one or several partial service area lists. The length of each partial service area list can be determined from the 'type of list' field and the 'number of elements' field in the first octet of the partial service area list. |  |  |  |  |  |
| For allowed type "0", TAIs contained in all partial service area lists are in the allowed area. For allowed type "1", TAIs contained in all partial service area lists are in the non-allowed area. The UE shall store the complete list received. If more than 16 TAIs are included in this information element, the UE shall store the first 16 TAIs and ignore the remaining octets of the information element. |  |  |  |  |  |
| Partial service area list: |  |  |  |  |  |
| Allowed type (octet 1) |  |  |  |  |  |
| Bit |  |  |  |  |  |
| 8 |  |  |  |  |  |
| 0 |  | TAIs in the list are in the allowed area |  |  |  |
| 1 |  | TAIs in the list are in the non-allowed area |  |  |  |
| Type of list (octet 1) |  |  |  |  |  |
| Bits |  |  |  |  |  |
| 7 | 6 |  |  |  |  |
| 0 | 0 | list of TACs belonging to one PLMN, with non-consecutive TAC values |  |  |  |
| 0 | 1 | list of TACs belonging to one PLMN, with consecutive TAC values |  |  |  |
| 1 | 0 | list of TAIs belonging to different PLMNs (see NOTE) |  |  |  |
| 1 | 1 | All TAIs belonging to the PLMN are in the allowed area |  |  |  |
| Number of elements (octet 1) |  |  |  |  |  |
| Bits |  |  |  |  |  |
| 5 | 4 | 3 | 2 | 1 |  |
| 0 | 0 | 0 | 0 | 0 | 1 element |
| 0 | 0 | 0 | 0 | 1 | 2 elements |
| 0 | 0 | 0 | 1 | 0 | 3 elements |
| … |  |  |  |  |  |
| 0 | 1 | 1 | 0 | 1 | 14 elements |
| 0 | 1 | 1 | 1 | 0 | 15 elements |
| 0 | 1 | 1 | 1 | 1 | 16 elements |
| All other values are unused and shall be interpreted as 16, if received by the UE. |  |  |  |  |  |
| For type of list = "00" and number of elements = k: |  |  |  |  |  |
| octet 2 to 4 contain the MCC+MNC, and |  |  |  |  |  |
| for j = 1, k: |  |  |  |  |  |
| octet 2j+3 and 2j+4 contain the TAC of the j-th TAI belonging to the partial list, |  |  |  |  |  |
| For type of list = "01" and number of elements = k: |  |  |  |  |  |
| octet 2 to 4 contain the MCC+MNC, and |  |  |  |  |  |
| octet 5 and 6 contain the TAC of the first TAI belonging to the partial list. |  |  |  |  |  |
| The TAC values of the other k-1 TAIs are TAC+1, TAC+2, …, TAC+k-1. |  |  |  |  |  |
| For type of list = "10" and number of elements = k: |  |  |  |  |  |
| for j = 1, k. |  |  |  |  |  |
| octet 5j-3 to 5j-1 contain the MCC+MNC, and |  |  |  |  |  |
| octet 5j and 5j+1 contain the TAC of the j-th TAI belonging to the partial list. |  |  |  |  |  |
| For type of list = "11": |  |  |  |  |  |
| Allowed type shall be coded as "0" and number of elements shall be ignored, and |  |  |  |  |  |
| octet 2 to 4 contain the MCC+MNC. If allowed type is coded as "1", it shall be interpreted as "0". |  |  |  |  |  |
| MNC, Mobile network code |  |  |  |  |  |
| The coding of this field is the responsibility of each administration but BCD coding shall be used. The MNC shall consist of 2 or 3 digits. If a network operator decides to use only two digits in the MNC, MNC digit 3 shall be coded as "1111". |  |  |  |  |  |
| TAC, Tracking area code |  |  |  |  |  |
| In the TAC field bit 8 of the first octet is the most significant bit and bit 1 of second octet the least significant bit. |  |  |  |  |  |
| The coding of the tracking area code is the responsibility of each administration. Coding using full hexadecimal representation may be used. The tracking area code consists of 2 octets. |  |  |  |  |  |
| NOTE: If the "list of TAIs belonging to different PLMNs" is used, the PLMNs included in the list need to be present in the list of "equivalent PLMNs". |  |  |  |  |  |

#### 9.8.3.40	SMS allowed

The purpose of the SMS allowed information element is for the network to indicate to the UE that the use of SMS over NAS transport is allowed.

The SMS allowed information element is coded as shown in figure 9.8.3.40.1 and table 9.8.3.40.1.

The SMS allowed type is a type 1 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 |  | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SMS allowed IEI |  |  |  | 0 Spare | 0 Spare |  | allowed accesses |  |

Figure 9.8.3.40.1: SMS allowed information element

Table 9.8.3.40.1: SMS allowed information element

| Accesses allowed for SMS over NAS transport (allowed accesses) (octet 1) |  |  |
| --- | --- | --- |
| Bit |  |  |
| 2 | 1 |  |
| 0 | 0 | SMS over NAS not allowed |
| 0 | 1 | SMS over NAS allowed via 3GPP access only |
| 1 | 0 | SMS over NAS allowed via both 3GPP access and non-3GPP access |
| 1 | 1 | reserved |
| Bits 3 and 4 are spare and shall be coded as zero, |  |  |

#### 9.8.3.41	SMS requested

The purpose of the SMS requested information element is for the UE to indicate to the network that it requires the use of SMS over NAS transport or its requirements to use SMS over NAS transport have changed.

The SMS requested information element is coded as shown in figure 9.8.3.41.1 and table 9.8.3.41.1.

The SMS requested type is a type 1 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 |  | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SMS requested IEI |  |  |  | 0 Spare | 0 Spare |  | supported accesses |  |

Figure 9.8.3.41.1: SMS requested information element

Table 9.8.3.41.1: SMS requested information element

| Accesses supported for SMS over NAS transport (supported accesses) (octet 1) |  |  |
| --- | --- | --- |
| Bit |  |  |
| 2 | 1 |  |
| 0 | 0 | SMS over NAS not supported |
| 0 | 1 | SMS over NAS supported via 3GPP access only |
| 1 | 0 | SMS over NAS supported via both 3GPP access and non-3GPP access |
| 1 | 1 | reserved |
| Bits 3 and 4 are spare and shall be coded as zero, |  |  |

#### 9.8.3.42	Request type

The purpose of the Request type information element is to indicate type of the PDU session establishment.

The Request type information element is coded as shown in figure 9.8.3.42.1 and table 9.8.3.42.1.

The Request type is a type 1 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request type IEI |  |  |  | Request type value |  |  |  | octet 1 |

Figure 9.8.3.42.1: Request type information element

Table 9.8.3.42.1: Request type information element

| Request type value (octet 1, bit 1 to bit 4) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bits |  |  |  |  |
| 4 | 3 | 2 | 1 |  |
| 0 | 0 | 0 | 1 | initial request |
| 0 | 0 | 1 | 0 | existing PDU session |
| 0 | 0 | 1 | 1 | initial emergency request |
| 0 | 1 | 0 | 0 | existing emergency PDU session |
| All other values are reserved. |  |  |  |  |

#### 9.8.3.43	Service type

The purpose of the service type information element is to specify the purpose of the service request procedure.

The service type is a type 1 information element.

The service type information element is coded as shown in figure 9.8.3.43.1 and table 9.8.3.43.1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Service type IEI |  |  |  | 0 spare | Service type |  |  | octet 1 |

Figure 9.8.3.43.1: Service type information element

Table 9.8.3.43.1: Service type information element

| Service type value (octet 1) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bits |  |  |  |  |
| 3 | 2 | 1 |  |  |
| 0 | 0 | 0 |  | Signalling |
| 0 | 0 | 1 |  | Data |
| 0 | 1 | 0 |  | Paging response |
| 1 | 0 | 0 |  | Emergency services fallback |
| All other values are reserved. |  |  |  |  |

#### 9.8.3.44	Tracking area identity

See subclause 9.9.3.32 in 3GPP TS 24.301 [10].

#### 9.8.3.45	Tracking area identity list

See subclause 9.9.3.33 in 3GPP TS 24.301 [10].

#### 9.8.3.46	Time zone

See subclause 10.5.3.8 in 3GPP TS 24.008 [8].

#### 9.8.3.47	Time zone and time

See subclause 10.5.3.9 in 3GPP TS 24.008 [8].

#### 9.8.3.48	UE security capability

The UE security capability information element is used by the network to indicate which security algorithms are supported by the UE in N1 mode and S1 mode. Security algorithms supported in N1 mode are supported both for NAS and for AS security.

The UE security capability information element is coded as shown in figure 9.8.3.48.1 and table 9.8.3.48.1.

The UE security capability is a type 4 information element with a minimum length of 4 octets and a maximum length of 6 octets.

Octets 5 and 6 are optional. If octet 5 is included, then also octet 6 shall be included.

If the UE did not indicate support of any security algorithm for S1 mode octets 5 and 6 shall not be included.

| 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UE security capability IEI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 1 |  |
| Length of UE security capability contents |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 2 |  |
| 5G-EA0 |  | 128- 5G-EA1 |  | 128- 5G-EA2 |  | 128- 5G-EA3 |  | 5G-EA4 |  | 5G-EA5 |  | 5G-EA6 |  | 5G-EA7 |  | octet 3 |  |
| 5G-IA0 |  | 128- 5G-IA1 |  | 128- 5G-IA2 |  | 128- 5G-IA3 |  | 5G-IA4 |  | 5G-IA5 |  | 5G-IA6 |  | 5G-IA7 |  | octet 4 |  |
| EEA0 |  | 128- EEA1 |  | 128- EEA2 |  | 128- EEA3 |  | EEA4 |  | EEA5 |  | EEA6 |  | EEA7 |  | octet 5* |  |
| EIA0 |  | 128- EIA1 |  | 128- EIA2 |  | 128- EIA3 |  | EIA4 |  | EIA5 |  | EIA6 |  | EIA7 |  | octet 6* |  |

# Figure 9.8.3.48.1: UE security capability information element

Table 9.8.3.48.1: UE security capability information element

| 5GS encryption algorithms supported (octet 3) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 5GS encryption algorithm 5G-EA0 supported (octet 3, bit 8) |  |  |  |  |  |
| 0 |  |  |  |  | 5GS encryption algorithm 5G-EA0 not supported |
| 1 |  |  |  |  | 5GS encryption algorithm 5G-EA0 supported |
| 5GS encryption algorithm 128-5G-EA1 supported (octet 3, bit 7) |  |  |  |  |  |
| 0 |  |  |  |  | 5GS encryption algorithm 128-5G-EA1 not supported |
| 1 |  |  |  |  | 5GS encryption algorithm 128-5G-EA1 supported |
| 5GS encryption algorithm 128-5G-EA2 supported (octet 3, bit 6) |  |  |  |  |  |
| 0 |  |  |  |  | 5GS encryption algorithm 128-5G-EA2 not supported |
| 1 |  |  |  |  | 5GS encryption algorithm 128-5G-EA2 supported |
| 5GS encryption algorithm 128-5G-EA3 supported (octet 3, bit 5) |  |  |  |  |  |
| 0 |  |  |  |  | 5GS encryption algorithm 128-5G-EA31 not supported |
| 1 |  |  |  |  | 5GS encryption algorithm 128-5G-EA3 supported |
| 5GS encryption algorithm 5G-EA4 supported (octet 3, bit 4) |  |  |  |  |  |
| 0 |  |  |  |  | 5GS encryption algorithm 5G-EA4 not supported |
| 1 |  |  |  |  | 5GS encryption algorithm 5G-EA4 supported |
| 5GS encryption algorithm 5G-EA5 supported (octet 3, bit 3) |  |  |  |  |  |
| 0 |  |  |  |  | 5GS encryption algorithm 5G-EA5 not supported |
| 1 |  |  |  |  | 5GS encryption algorithm 5G-EA5 supported |
| 5GS encryption algorithm 5G-EA6 supported (octet 3, bit 2) |  |  |  |  |  |
| 0 |  |  |  |  | 5GS encryption algorithm 5G-EA6 not supported |
| 1 |  |  |  |  | 5GS encryption algorithm 5G-EA6 supported |
| 5GS encryption algorithm 5G-EA7 supported (octet 3, bit 1) |  |  |  |  |  |
| 0 |  |  |  |  | 5GS encryption algorithm 5G-EA7 not supported |
| 1 |  |  |  |  | 5GS encryption algorithm 5G-EA7 supported |
| EPS integrity algorithms supported (octet 4) |  |  |  |  |  |
| 5GS integrity algorithm 5G-IA0 supported (octet 4, bit 8) |  |  |  |  |  |
| 0 |  |  |  |  | 5GS integrity algorithm 5G-IA0 not supported |
| 1 |  |  |  |  | 5GS integrity algorithm 5G-IA0 supported |
| 5GS integrity algorithm 128-5G-IA1 supported (octet 4, bit 7) |  |  |  |  |  |
| 0 |  |  |  |  | 5GS integrity algorithm 128-5G-IA1 not supported |
| 1 |  |  |  |  | 5GS integrity algorithm 128-5G-IA1 supported |
| 5GS integrity algorithm 128-5G-IA2 supported (octet 4, bit 6) |  |  |  |  |  |
| 0 |  |  |  |  | 5GS integrity algorithm 128-5G-IA2 not supported |
| 1 |  |  |  |  | 5GS integrity algorithm 128-5G-IA2 supported |
| 5GS integrity algorithm 128-5G-IA3 supported (octet 4, bit 5) |  |  |  |  |  |
| 0 |  |  |  |  | 5GS integrity algorithm 128-5G-IA3 not supported |
| 1 |  |  |  |  | 5GS integrity algorithm 128-5G-IA3 supported |
| 5GS integrity algorithm 5G-IA4 supported (octet 4, bit 4) |  |  |  |  |  |
| 0 |  |  |  |  | 5GS integrity algorithm 5G-IA4 not supported |
| 1 |  |  |  |  | 5GS integrity algorithm 5G-IA4 supported |
| 5GS integrity algorithm 5G-IA5 supported (octet 4, bit 3) |  |  |  |  |  |
| 0 |  |  |  |  | 5GS integrity algorithm 5G-IA5 not supported |
| 1 |  |  |  |  | 5GS integrity algorithm 5G-IA5 supported |
| 5GS integrity algorithm 5G-IA6supported (octet 4, bit 2) |  |  |  |  |  |
| 0 |  |  |  |  | 5GS integrity algorithm 5G-IA6 not supported |
| 1 |  |  |  |  | 5GS integrity algorithm 5G-IA6 supported |
| 5GS integrity algorithm 5G-IA7 supported (octet 4, bit 1) |  |  |  |  |  |
| 0 |  |  |  |  | 5GS integrity algorithm 5G-IA7 not supported |
| 1 |  |  |  |  | 5GS integrity algorithm 5G-IA7 supported |
| EPS encryption algorithms supported (octet 5) |  |  |  |  |  |
| EPS encryption algorithm EEA0 supported (octet 5, bit 8) |  |  |  |  |  |
| 0 |  |  |  |  | EPS encryption algorithm EEA0 not supported |
| 1 |  |  |  |  | EPS encryption algorithm EEA0 supported |
| EPS encryption algorithm 128-EEA1 supported (octet 5, bit 7) |  |  |  |  |  |
| 0 |  |  |  |  | EPS encryption algorithm 128-EEA1 not supported |
| 1 |  |  |  |  | EPS encryption algorithm 128-EEA1 supported |
| EPS encryption algorithm 128-EEA2 supported (octet 5, bit 6) |  |  |  |  |  |
| 0 |  |  |  |  | EPS encryption algorithm 128-EEA2 not supported |
| 1 |  |  |  |  | EPS encryption algorithm 128-EEA2 supported |
| EPS encryption algorithm 128-EEA3 supported (octet 5, bit 5) |  |  |  |  |  |
| 0 |  |  |  |  | EPS encryption algorithm 128-EEA3 not supported |
| 1 |  |  |  |  | EPS encryption algorithm 128-EEA3 supported |
| EPS encryption algorithm EEA4 supported (octet 5, bit 4) |  |  |  |  |  |
| 0 |  |  |  |  | EPS encryption algorithm EEA4 not supported |
| 1 |  |  |  |  | EPS encryption algorithm EEA4 supported |
| EPS encryption algorithm EEA5 supported (octet 5, bit 3) |  |  |  |  |  |
| 0 |  |  |  |  | EPS encryption algorithm EEA5 not supported |
| 1 |  |  |  |  | EPS encryption algorithm EEA5 supported |
| EPS encryption algorithm EEA6 supported (octet 5, bit 2) |  |  |  |  |  |
| 0 |  |  |  |  | EPS encryption algorithm EEA6 not supported |
| 1 |  |  |  |  | EPS encryption algorithm EEA6 supported |
| EPS encryption algorithm EEA7 supported (octet 5, bit 1) |  |  |  |  |  |
| 0 |  |  |  |  | EPS encryption algorithm EEA7 not supported |
| 1 |  |  |  |  | EPS encryption algorithm EEA7 supported |
| EPS integrity algorithms supported (octet 6) |  |  |  |  |  |
| EPS integrity algorithm EIA0 supported (octet 6, bit 8) |  |  |  |  |  |
| 0 |  |  |  |  | EPS integrity algorithm EIA0 not supported |
| 1 |  |  |  |  | EPS integrity algorithm EIA0 supported |
| EPS integrity algorithm 128-EIA1 supported (octet 6, bit 7) |  |  |  |  |  |
| 0 |  |  |  |  | EPS integrity algorithm 128-EIA1 not supported |
| 1 |  |  |  |  | EPS integrity algorithm 128-EIA1 supported |
| EPS integrity algorithm 128-EIA2 supported (octet 6, bit 6) |  |  |  |  |  |
| 0 |  |  |  |  | EPS integrity algorithm 128-EIA2 not supported |
| 1 |  |  |  |  | EPS integrity algorithm 128-EIA2 supported |
| EPS integrity algorithm 128-EIA3 supported (octet 6, bit 5) |  |  |  |  |  |
| 0 |  |  |  |  | EPS integrity algorithm 128-EIA3 not supported |
| 1 |  |  |  |  | EPS integrity algorithm 128-EIA3 supported |
| EPS integrity algorithm EIA4 supported (octet 6, bit 4) |  |  |  |  |  |
| 0 |  |  |  |  | EPS integrity algorithm EIA4 not supported |
| 1 |  |  |  |  | EPS integrity algorithm EIA4 supported |
| EPS integrity algorithm EIA5 supported (octet 6, bit 3) |  |  |  |  |  |
| 0 |  |  |  |  | EPS integrity algorithm EIA5 not supported |
| 1 |  |  |  |  | EPS integrity algorithm EIA5 supported |
| EPS integrity algorithm EIA6 supported (octet 6, bit 2) |  |  |  |  |  |
| 0 |  |  |  |  | EPS integrity algorithm EIA6 not supported |
| 1 |  |  |  |  | EPS integrity algorithm EIA6 supported |
| EPS integrity algorithm EIA7 supported (octet 6, bit 1) |  |  |  |  |  |
| 0 |  |  |  |  | EPS integrity algorithm EIA7 not supported |
| 1 |  |  |  |  | EPS integrity algorithm EIA7 supported |

#### 9.8.3.49	UE status

The purpose of the UE status information element is to provide the network with information concerning aspects of the current UE registration status which is used for interworking with EPS.

The UE status information element is coded as shown in figure 9.8.3.49.1 and table 9.8.3.49.1.

The UE status is a type 4 information element with a length of 3 octets.

| 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UE status IEI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 1 |  |
| Length of UE status contents |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 2 |  |
| 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | S1 mode reg |  | octet 3 |  |

Figure 9.8.3.49.1: UE status information element

Table 9.8.3.49.1: UE status information element

| EMM registration status (S1 mode reg) (octet 3, bit 1) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 0 |  |  |  |  | UE is not in EMM-REGISTERED state |
| 1 |  |  |  |  | UE is in EMM-REGISTERED state |
| All other bits in the octet 3 are spare and shall be coded as zero, |  |  |  |  |  |

### 9.8.4	5GS session management (5GSM) information elements

#### 9.8.4.1	5GSM cause

The purpose of the 5GSM cause information element is to indicate the reason why a 5GSM request is rejected.

The 5GSM cause information element is coded as shown in figure 9.8.4.1.1 and table 9.8.4.1.1.

The 5GSM cause is a type 3 information element with 2 octets length.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5GSM cause IEI |  |  |  |  |  |  |  | octet 1 |
| Cause value |  |  |  |  |  |  |  | octet 2 |

Figure 9.8.4.1.1: 5GSM cause information element

Table 9.8.4.1.1: 5GSM cause information element

| Cause value (octet 2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bits |  |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 0 | 0 | 1 | 1 | 1 | 0 | 1 |  | User authentication failed |
| 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 |  | Request rejected, unspecified |
| 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 |  | Service option temporarily out of order |
| 0 | 0 | 1 | 0 | 0 | 1 | 0 | 0 |  | Regular deactivation |
| 0 | 0 | 1 | 0 | 0 | 1 | 1 | 1 |  | Reactivation requested |
| 0 | 0 | 1 | 1 | 0 | 0 | 1 | 0 |  | PDU session type Ipv4 only allowed |
| 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |  | PDU session type Ipv6 only allowed |
| 0 | 1 | 0 | 1 | 1 | 1 | 1 | 1 |  | Semantically incorrect message |
| 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 |  | Invalid mandatory information |
| 0 | 1 | 1 | 0 | 0 | 0 | 0 | 1 |  | Message type non-existent or not implemented |
| 0 | 1 | 1 | 0 | 0 | 0 | 1 | 0 |  | Message type not compatible with the protocol state |
| 0 | 1 | 1 | 0 | 0 | 0 | 1 | 1 |  | Information element non-existent or not implemented |
| 0 | 1 | 1 | 0 | 0 | 1 | 0 | 0 |  | Conditional IE error |
| 0 | 1 | 1 | 0 | 0 | 1 | 0 | 1 |  | Message not compatible with the protocol state |
| 0 | 1 | 1 | 0 | 1 | 1 | 1 | 1 |  | Protocol error, unspecified |
| Any other value received by the UE shall be treated as 0010 0010, "service option temporarily out of order". Any other value received by the network shall be treated as 0110 1111, "protocol error, unspecified". |  |  |  |  |  |  |  |  |  |

#### 9.8.4.2	Extended protocol configuration options

See subclause 10.5.6.3A in 3GPP TS 24.008 [8].

#### 9.8.4.3	GPRS timer

See subclause 10.5.7.3 in 3GPP TS 24.008 [8].

#### 9.8.4.4	PDU address

The purpose of the PDU address information element is to assign to the UE an Ipv4 address associated with a PDU session and can provide the UE with an interface identifier for the Ipv6 link local address associated with a PDU session.

The PDU address information element is coded as shown in figure 9.8.4.4.1 and table 9.8.4.4.1.

The PDN address is a type 4 information element with length of 7 or 11 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PDU address IEI |  |  |  |  |  |  |  | octet 1 |
| Length of PDU address contents |  |  |  |  |  |  |  | octet 2 |
| 0 Spare | 0 Spare | 0 Spare | 0 Spare | PDU session type value |  |  |  | octet 3 |
| PDU address information |  |  |  |  |  |  |  | octet 4 octet x |

Figure 9.8.4.4.1: PDU address information element

Table 9.8.4.4.1: PDU address information element

| PDU session type value (octet 3) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bits |  |  |  |  |
| 4 | 3 | 2 | 1 |  |
| 0 | 0 | 1 | 0 | IPv4 |
| 0 | 0 | 1 | 1 | IPv6 |
| All other values are reserved. |  |  |  |  |
| PDU address information (octet 4 to x) |  |  |  |  |
| If the PDU session type value indicates Ipv4, the PDU address information in octet 4 to octet 7 contains an Ipv4 address. |  |  |  |  |
| If the PDU session type value indicates Ipv6, the PDU address information in octet 4 to octet 11 contains an interface identifier for the IPv6 link local address. |  |  |  |  |

#### 9.8.4.5	PDU session type

The purpose of the PDU session type information element is to indicate type of the PDU session.

The PDU session type information element is coded as shown in figure 9.8.4.1.5 and table 9.8.4.1.5.

The PDU session type is a type 1 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PDU session type IEI |  |  |  | PDU session type value |  |  |  | octet 1 |

Figure 9.8.4.1.5: PDU session type information element

Table 9.8.4.1.5: PDU session type information element

| PDU session type value (octet 1, bit 1 to bit 4) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bits |  |  |  |  |
| 4 | 3 | 2 | 1 |  |
| 0 | 0 | 0 | 1 | IP |
| 0 | 0 | 1 | 0 | Ipv4 |
| 0 | 0 | 1 | 1 | Ipv6 |
| 0 | 1 | 0 | 0 | Unstructured |
| 0 | 1 | 0 | 1 | Ethernet |
| All other values are reserved. |  |  |  |  |

#### 9.8.4.6	QoS rules

The purpose of the QoS rules information element is to indicate a set of QoS rules where each QoS rule is a set of parameters as described in subclause 6.2.5.1.1.2, that are used by the UE:

a)	for classification and marking of uplink user traffic; and

b)	for identification of a QoS flow which the network is to use for a particular downlink user traffic.

NOTE:	The UE needs to be aware of a QoS flow which the network is to use for a particular downlink user traffic e.g. to determine whether a resource is available for downlink media of a media stream of an SDP media description provided by the UE in an IMS session.

The QoS rules may contain a set of packet filters consisting of zero or more packet filters for UL direction, zero or more packet filters for DL direction, zero or more packet filters for both UL and DL directions or any combinations of these. The set of packet filters determine the traffic mapping to QoS flows. The QoS rules may contain a EPS bearer identity (EBI), mapped EPS QoS parameters, mapped extended EPS QoS parameters and mapped traffic flow template if the QoS flow can be mapped to an EPS bearer as specified in subclause 4.11.2 of 3GPP TS 23.501 [4].

The QoS rules information element is a type 6 information element with a minimum length of 7 octets. The maximum length for the information element is 65538 octets.

The QoS rules information element is coded as shown in figure 9.8.4.6.1, figure 9.8.4.6.2, figure 9.8.4.6.3, figure 9.8.4.6.4, figure 9.8.4.6.5 and table 9.8.4.6.1.

|  |  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | QoS rules IEI |  |  |  |  |  |  |  |  | octet 1 |  |
|  |  | Length of QoS rules IE |  |  |  |  |  |  |  |  | octet 2 |  |
|  |  |  |  |  |  |  |  |  |  |  | octet 3 |  |
|  |  | QoS rule 1 |  |  |  |  |  |  |  |  | octet 4 octet u |  |
|  |  | QoS rule 2 |  |  |  |  |  |  |  |  | octet u+1 octet v |  |
|  |  | … |  |  |  |  |  |  |  |  | octet v+1 octet w |  |
|  |  | QoS rule n |  |  |  |  |  |  |  |  | octet w+1 octet x |  |

Figure 9.8.4.6.1: QoS rules information element

|  |  | 8 | 7 | 6 | 5 |  | 4 |  | 3 | 2 | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | QoS rule identifier |  |  |  |  |  |  |  |  |  |  | octet 4 |  |
|  |  | Length of QoS rule |  |  |  |  |  |  |  |  |  |  | octet 5 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | octet 6 |  |
|  |  | Rule operation code |  |  |  | DQR bit |  | Number of packet filters |  |  |  |  | octet 7 |  |
|  |  | Packet filter list |  |  |  |  |  |  |  |  |  |  | octet 8* octet z |  |
|  |  | 0 Spare | E | Number of parameters |  |  |  |  |  |  |  |  | octet z+1 |  |
|  |  | Parameters list |  |  |  |  |  |  |  |  |  |  | octet z+1 octet u |  |
|  |  | QoS rule precedence |  |  |  |  |  |  |  |  |  |  | octet u+1* |  |
|  |  | QoS flow identifier |  |  |  |  |  |  |  |  |  |  | octet u+2* |  |

Figure 9.8.4.6.2: QoS rule

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 0 | 0 | 0 | Packet filter identifier 1 |  |  |  | octet 8 |
|  | Spare |  |  |  |  |  |  |  |  |
|  | 0 | 0 | 0 | 0 | Packet filter identifier 2 |  |  |  | octet 9 |
|  | Spare |  |  |  |  |  |  |  |  |
|  | … |  |  |  |  |  |  |  |  |
|  | 0 | 0 | 0 | 0 | Packet filter identifier N |  |  |  | octet n+3 |
|  | Spare |  |  |  |  |  |  |  |  |

Figure 9.8.4.6.3: Packet filter list when the rule operation is "modify existing QoS rule and delete packet filters" (z=N+3)

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 0 | Packet filter direction 1 |  | Packet filter identifier 1 |  |  |  | octet 8 |
|  | Spare |  |  |  |  |  |  |  |  |
|  | Length of packet filter contents 1 |  |  |  |  |  |  |  | octet 9 |
|  | Packet filter contents 1 |  |  |  |  |  |  |  | octet 10 octet m |
|  | 0 | 0 | Packet filter direction 2 |  | Packet filter identifier 2 |  |  |  | octet m+1 |
|  | Spare |  |  |  |  |  |  |  |  |
|  | Length of packet filter contents 2 |  |  |  |  |  |  |  | octet m+2 |
|  | Packet filter contents 2 |  |  |  |  |  |  |  | octet m+3 octet n |
|  | … |  |  |  |  |  |  |  | octet n+1 octet y |
|  | 0 | 0 | Packet filter direction N |  | Packet filter identifier N |  |  |  | octet y+1 |
|  | Spare |  |  |  |  |  |  |  |  |
|  | Length of packet filter contents N |  |  |  |  |  |  |  | octet y+2 |
|  | Packet filter contents N |  |  |  |  |  |  |  | octet y+3 octet z |

Figure 9.8.4.6.4: Packet filter list when the rule operation is "create new QoS rule", or "modify existing QoS rule and add packet filters" or "modify existing QoS rule and replace packet filters"

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Parameter identifier 1 |  |  |  |  |  |  |  | octet z+2 |
|  | Length of Parameter contents 1 |  |  |  |  |  |  |  | octet z+3 |
|  | Parameter contents 1 |  |  |  |  |  |  |  | octet z+4 octet k |
|  | Parameter identifier 2 |  |  |  |  |  |  |  | octet k+1 |
|  | Length of Parameter contents 2 |  |  |  |  |  |  |  | octet k+2 |
|  | Parameter contents 2 |  |  |  |  |  |  |  | octet k+3 octet p |
|  | … |  |  |  |  |  |  |  | octet p+1 octet q |
|  | Parameter identifier N |  |  |  |  |  |  |  | octet q+1 |
|  | Length of Parameter contents N |  |  |  |  |  |  |  | octet q+2 |
|  | Parameter contents N |  |  |  |  |  |  |  | octet q+3 octet v |

Figure 9.8.4.6.5: Parameters list

Table 9.8.4.6.1: QoS rules information element

QoS rule identifier (octet 4) The QoS rule identifier field is used to identify the QoS rule. QoS rule precedence (octet u+1) The QoS rule precedence field is used to specify the precedence of the QoS rule among all QoS rules (both the signalled QoS rules as described in subclause 6.2.5.1.1.2 and the derived QoS rules as described in subclause 6.2.5.1.1.3) associated with the PDU session of the QoS flow. This field includes the binary coded value of the QoS rule precedence in the range from 0 to 255 (decimal). The higher the value of the QoS rule precedence field, the lower the precedence of that QoS rule is. The value 80 (decimal) is reserved. QoS flow identifier (octet u+2) The QoS flow identifier field is used to identify the QoS flow. DQR bit (bit 5 of octet 7) The DQR bit indicates whether the QoS rule is the default QoS rule and it is encoded as follows: Bit 5 0 the QoS rule is not the default QoS rule. 1 the QoS rule is the default QoS rule. Rule operation code (bits 8 to 6 of octet 7) Bits 8 7 6 0 0 0 Reserved 0 0 1 Create new QoS rule 0 1 0 Delete existing QoS rule 0 1 1 Modify existing QoS rule and add packet filters 1 0 0 Modify existing QoS rule and replace packet filters 1 0 1 Modify existing QoS rule and delete packet filters 1 1 0 Modify existing QoS rule without modifying packet filters 1 1 1 Reserved E bit (bit 7 of octet z+1) For the "create new QoS rule" operation, the E bit is encoded as follows: Bit 7 0 parameters list is not included 1 parameters list is included For the "modify existing QoS rule and add packet filters" operation, the "modify existing QoS rule and replace packet filters", the "modify existing QoS rule and delete packet filters" operation and the "modify existing QoS rule without modifying packet filters" operation, the E bit is encoded as follows: Bit 7 0 previously provided parameters list extension 1 previously provided parameters list replacement If the E bit is set to "parameters list is not included", the number of parameters field has zero value. If the E bit is set to "parameters list is included", the number of parameters field has non-zero value. If the E bit is set to "previously provided parameters list extension" or "previously provided parameters list replacement", the number of parameters field can have zero or non-zero value. Number of packet filters (bits 4 to 1 of octet 7) The number of packet filters contains the binary coding for the number of packet filters in the packet filter list. The number of packet filters field is encoded in bits 4 through 1 of octet 7 where bit 4 is the most significant and bit 1 is the least significant bit. For the "delete existing QoS rule" operation and for the "modify existing QoS rule without modifying packet filters" operation, the number of packet filters shall be coded as 0. For the "create new QoS rule" operation and the "modify existing QoS rule and replace packet filters" operation, the number of packet filters shall be greater than or equal to 0 and less than or equal to 15. For all other operations, the number of packet filters shall be greater than 0 and less than or equal to 15. Packet filter list (octets 8 to z) The packet filter list contains a variable number of packet filters. For the "delete existing QoS rule" operation, the length of QoS rule field is set to one. For the "delete existing QoS rule" operation and the "modify existing QoS rule without modifying packet filters" operation, the packet filter list shall be empty. For the "modify existing QoS rule and delete packet filters" operation, the packet filter list shall contain a variable number of packet filter identifiers. This number shall be derived from the coding of the number of packet filters field in octet 7. For the "create new QoS rule" operation and for the "modify existing QoS rule and replace packet filters" operation, the packet filter list shall contain 0 or a variable number of packet filters. This number shall be derived from the coding of the number of packet filters field in octet 7. For the "modify existing QoS rule and add packet filters" operation, the packet filter list shall contain a variable number of packet filters. This number shall be derived from the coding of the number of packet filters field in octet 7. Each packet filter is of variable length and consists of a packet filter direction (2 bits); - a packet filter identifier (4 bits); - the length of the packet filter contents (1 octet); and - the packet filter contents itself (variable amount of octets). The packet filter direction field is used to indicate for what traffic direction the filter applies. Bits 6 5 0 0 reserved 0 1 downlink only 1 0 uplink only 1 1 bidirectional (see NOTE 1) The packet filter identifier field is used to identify each packet filter in a QoS rule. The least significant 4 bits are used. The length of the packet filter contents field contains the binary coded representation of the length of the packet filter contents field of a packet filter. The first bit in transmission order is the most significant bit. The packet filter contents field is of variable size and contains a variable number (at least one) of packet filter components. Each packet filter component shall be encoded as a sequence of a one octet packet filter component type identifier and a fixed length packet filter component value field. The packet filter component type identifier shall be transmitted first. In each packet filter, there shall not be more than one occurrence of each packet filter component type. Among the "Ipv4 remote address type" and "IPv6 remote address/prefix length type" packet filter components, only one shall be present in one packet filter. Among the "IPv4 local address type" and "IPv6 local address/prefix length type" packet filter components, only one shall be present in one packet filter. Among the "single local port type" and "local port range type" packet filter components, only one shall be present in one packet filter. Among the "single remote port type" and "remote port range type" packet filter components, only one shall be present in one packet filter. The term local refers to the UE and the term remote refers to an external network entity. Packet filter component type identifier Bits 8 7 6 5 4 3 2 1 0 0 0 1 0 0 0 0 IPv4 remote address type 0 0 0 1 0 0 0 1 IPv4 local address type 0 0 1 0 0 0 0 1 IPv6 remote address/prefix length type 0 0 1 0 0 0 1 1 IPv6 local address/prefix length type 0 0 1 1 0 0 0 0 Protocol identifier/Next header type 0 1 0 0 0 0 0 0 Single local port type 0 1 0 0 0 0 0 1 Local port range type 0 1 0 1 0 0 0 0 Single remote port type 0 1 0 1 0 0 0 1 Remote port range type 0 1 1 0 0 0 0 0 Security parameter index type 0 1 1 1 0 0 0 0 Type of service/Traffic class type 1 0 0 0 0 0 0 0 Flow label type 1 0 0 0 0 0 0 1 Destination MAC address type 1 0 0 0 0 0 1 0 Source MAC address type 1 0 0 0 0 0 1 1 802.1Q C-TAG VID type 1 0 0 0 0 1 0 0 802.1Q S-TAG VID type 1 0 0 0 0 1 0 1 802.1Q C-TAG PCP/DEI type 1 0 0 0 0 1 1 0 802.1Q S-TAG PCP/DEI type 1 0 0 0 0 1 1 1 Ethertype type All other values are reserved. The description and valid combinations of packet filter component type identifiers in a packet filter are defined in 3GPP TS 23.501 [4]. For "IPv4 remote address type", the packet filter component value field shall be encoded as a sequence of a four octet IPv4 address field and a four octet IPv4 address mask field. The IPv4 address field shall be transmitted first. For "IPv4 local address type", the packet filter component value field shall be encoded as defined for "IPv4 remote address type". For "IPv6 remote address/prefix length type", the packet filter component value field shall be encoded as a sequence of a sixteen octet IPv6 address field and one octet prefix length field. The IPv6 address field shall be transmitted first. For "IPv6 local address/prefix length type", the packet filter component value field shall be encoded as defined for "IPv6 remote address /prefix length". For "protocol identifier/Next header type", the packet filter component value field shall be encoded as one octet which specifies the IPv4 protocol identifier or Ipv6 next header. For "single local port type" and "single remote port type", the packet filter component value field shall be encoded as two octets which specify a port number. For "local port range type" and "remote port range type", the packet filter component value field shall be encoded as a sequence of a two octet port range low limit field and a two octet port range high limit field. The port range low limit field shall be transmitted first. For "security parameter index", the packet filter component value field shall be encoded as four octets which specify the IPSec security parameter index. For "type of service/traffic class type", the packet filter component value field shall be encoded as a sequence of a one octet type-of-service/traffic class field and a one octet type-of-service/traffic class mask field. The type-of-service/traffic class field shall be transmitted first. For "flow label type", the packet filter component value field shall be encoded as three octets which specify the IPv6 flow label. The bits 8 through 5 of the first octet shall be spare whereas the remaining 20 bits shall contain the IPv6 flow label. For "destination MAC address type" and "source MAC address type", the packet filter component value field shall be encoded as 6 octets which specify a MAC address. For "802.1Q C-TAG VID type", the packet filter component value field shall be encoded as two octets which specify the VID of the customer-VLAN tag (C-TAG). The bits 8 through 5 of the first octet shall be spare whereas the remaining 12 bits shall contain the VID. For "802.1Q S-TAG VID type", the packet filter component value field shall be encoded as two octets which specify the VID of the service-VLAN tag (S-TAG). The bits 8 through 5 of the first octet shall be spare whereas the remaining 12 bits shall contain the VID. For "802.1Q C-TAG PCP/DEI type", the packet filter component value field shall be encoded as one octet which specifies the 802.1Q C-TAG PCP and DEI. The bits 8 through 5 of the octet shall be spare, the bits 4 through 2 contain the PCP and bit 1 contains the DEI. For "802.1Q S-TAG PCP/DEI type", the packet filter component value field shall be encoded as one octet which specifies the 802.1Q S-TAG PCP. The bits 8 through 5 of the octet shall be spare, the bits 4 through 2 contain the PCP and bit 1 contains the DEI. For "ethertype type", the packet filter component value field shall be encoded as two octets which specify an ethertype. Number of parameters (bits 6 to 1 of octet z+1) The number of parameters field contains the binary coding for the number of parameters in the parameters list field. The number of parameters field is encoded in bits 6 through 1 of octet z+1 where bit 6 is the most significant and bit 1 is the least significant bit. Parameters list (octets z+2 to v) The parameters list contains a variable number of parameters. Each parameter included in the parameters list is of variable length and consists of: - a parameter identifier (1 octet); - the length of the parameter contents (1 octet); and - the parameter contents itself (variable amount of octets). The parameter identifier field is used to identify each parameter included in the parameters list and it contains the hexadecimal coding of the parameter identifier. Bit 8 of the parameter identifier field contains the most significant bit and bit 1 contains the least significant bit. In this version of the protocol, the following parameter identifiers are specified: - 01H (5QI); - 02H (GFBR uplink); - 03H (GFBR downlink); - 04H (MFBR uplink); - 05H (MFBR downlink); - 06H (Averaging window for uplink); - 07H (Averaging window for downlink); 08H (EPS bearer identity); - 09H (Mapped EPS QoS parameters); - 10H (Mapped extended EPS QoS parameters); and - 11H (Traffic flow template). If the parameters list contains a parameter identifier that is not supported by the receiving entity the corresponding parameter shall be discarded. The length of parameter contents field contains the binary coded representation of the length of the parameter contents field. The first bit in transmission order is the most significant bit. When the parameter identifier indicates 5QI, the parameter contents field contains the binary representation of 5G QoS identifier (5QI) that is one octet in length. 5QI: Bits 8 7 6 5 4 3 2 1 In network to UE direction: 0 0 0 0 0 0 0 0 Reserved 0 0 0 0 0 0 0 1 5QI 1 0 0 0 0 0 0 1 0 5QI 2 0 0 0 0 0 0 1 1 5QI 3 0 0 0 0 0 1 0 0 5QI 4 0 0 0 0 0 1 0 1 5QI 5 0 0 0 0 0 1 1 0 5QI 6 0 0 0 0 0 1 1 1 5QI 7 0 0 0 0 1 0 0 0 5QI 8 0 0 0 0 1 0 0 1 5QI 9 0 0 0 0 1 0 1 0 to Spare 0 1 0 0 0 0 0 0 0 1 0 0 0 0 0 1 5QI 65 0 1 0 0 0 0 1 0 5QI 66 0 1 0 0 0 0 1 1 to Spare 0 1 0 0 0 1 0 0 0 1 0 0 0 1 0 1 5QI 69 0 1 0 0 0 1 1 0 5QI 70 0 1 0 0 0 1 1 1 to Spare 0 1 0 0 1 0 1 0 0 1 0 0 1 0 1 1 5QI 75 0 1 0 0 1 1 0 0 to Spare 0 1 0 0 1 1 1 0 0 1 0 0 1 1 1 1 5QI 79 0 1 0 1 0 0 0 0 to Spare 0 1 1 1 1 1 1 1 1 0 0 0 0 0 0 0 to Operator-specific 5Qis 1 1 1 1 1 1 1 0 1 1 1 1 1 1 1 1 Reserved The network shall consider all other values not explicitly defined in this version of the protocol as unsupported. If the UE receives a 5QI value (excluding the reserved 5QI values) that it does not understand, the UE shall choose a 5QI value from the set of 5QI values defined in this version of the protocol (see 3GPP TS 23.501 [4]) and associated with: - GBR QoS flows, if the QoS flow includes a GFBR uplink parameter and a GFBR dowlinkink parameter; and - non-GBR QoS flows, if the QoS flow does not include a GFBR uplink parameter or does not include a GFBR downlink parameter. The UE shall use this chosen 5QI value for internal operations only. The UE shall use the received 5QI value in subsequent NAS signalling procedures. When the parameter identifier indicates "GFBR uplink", the parameter contents field contains one octet indicating the unit of the guaranteed flow bit rate for uplink followed by two octets containing the value of the guaranteed flow bit rate for uplink. Unit of the guaranteed flow bit rate for uplink (octet 1) Bits 8 7 6 5 4 3 2 1 0 0 0 0 0 0 0 0 0 value is not used 0 0 0 0 0 0 0 0 1 value is incremented in multiples of 1 Kbps 0 0 0 0 0 0 0 1 0 value is incremented in multiples of 4 Kbps 0 0 0 0 0 0 0 1 1 value is incremented in multiples of 16 Kbps 0 0 0 0 0 0 1 0 0 value is incremented in multiples of 64 Kbps 0 0 0 0 0 0 1 0 1 value is incremented in multiples of 256 Kbps 0 0 0 0 0 0 1 1 0 value is incremented in multiples of 1 Mbps 0 0 0 0 0 0 1 1 1 value is incremented in multiples of 4 Mbps 0 0 0 0 0 1 0 0 0 value is incremented in multiples of 16 Mbps 0 0 0 0 0 1 0 0 1 value is incremented in multiples of 64 Mbps 0 0 0 0 0 1 0 1 0 value is incremented in multiples of 256 Mbps 0 0 0 0 0 1 0 1 1 value is incremented in multiples of 1 Gbps 0 0 0 0 0 1 1 0 0 value is incremented in multiples of 4 Gbps 0 0 0 0 0 1 1 0 1 value is incremented in multiples of 16 Gbps 0 0 0 0 0 1 1 1 0 value is incremented in multiples of 64 Gbps 0 0 0 0 0 1 1 1 1 value is incremented in multiples of 256 Gbps 0 0 0 0 1 0 0 0 0 value is incremented in multiples of 1 Tbps 0 0 0 0 1 0 0 0 1 value is incremented in multiples of 4 Tbps 0 0 0 0 1 0 0 1 0 value is incremented in multiples of 16 Tbps 0 0 0 0 1 0 0 1 1 value is incremented in multiples of 64 Tbps 0 0 0 0 1 0 1 0 0 value is incremented in multiples of 256 Tbps 0 0 0 0 1 0 1 0 1 value is incremented in multiples of 1 Pbps 0 0 0 0 1 0 1 1 0 value is incremented in multiples of 4 Pbps 0 0 0 0 1 0 1 1 1 value is incremented in multiples of 16 Pbps 0 0 0 0 1 1 0 0 0 value is incremented in multiples of 64 Pbps 0 0 0 0 1 1 0 0 1 value is incremented in multiples of 256 Pbps Other values shall be interpreted as multiples of 256 Pbps in this version of the protocol. Value of the guaranteed flow bit rate for uplink (octets 2 and 3) Octets 2 and 3 represent the binary coded value of the guaranteed flow bit rate for uplink in units defined by the unit of the guaranteed flow bit rate for uplink. When the parameter identifier indicates "GFBR downlink", the parameter contents field contains one octet indicating the unit of the guaranteed flow bit rate for downlink followed by two octets containing the value of the guaranteed flow bit rate for downlink. Unit of the guaranteed flow bit rate for downlink (octet 1) The coding is identical to that of the unit of the guaranteed flow bit rate for uplink. Value of the guaranteed flow bit rate for downlink (octets 2 and 3) Octets 2 and 3 represent the binary coded value of the guaranteed flow bit rate for downlink in units defined by the unit of the guaranteed flow bit rate for downlink. When the parameter identifier indicates "MFBR uplink", the parameter contents field contains the one octet indicating the unit of the maximum flow bit rate for uplink followed by two octets containing the value of maximum flow bit rate for uplink. Unit of the maximum flow bit rate for uplink (octet 1) The coding is identical to that of the unit of the guaranteed flow bit rate for uplink. Value of the maximum flow bit rate for uplink (octets 2 and 3) Octets 2 and 3 represent the binary coded value of the maximum flow bit rate for uplink in units defined by the unit of the maximum flow bit rate for uplink. When the parameter identifier indicates "MFBR downlink", the parameter contents field contains one octet indicating the unit of the maximum flow bit rate for downlink followed by two octets containing the value of the maximum flow bit rate for downlink. Unit of the maximum flow bit rate for downlink (octet 1) The coding is identical to that of the unit of the guaranteed flow bit rate for uplink. Value of the maximum flow bit rate for downlink (octets 2 and 3) Octets 2 and 3 represent the binary coded value of the maximum flow bit rate for downlink in units defined by the unit of the maximum flow bit rate for downlink. When the parameter identifier indicates "averaging window for uplink", the parameter contents field contains the binary representation of the averaging window for uplink in milliseconds and the parameter contents field is two octets in length. When the parameter identifier indicates "averaging window for downlink", the parameter contents field contains the binary representation of the averaging window for downlink in milliseconds and the parameter contents field is two octets in length. When the parameter identifier indicates EPS bearer identity, the length of EPS bearer identity is one octet and parameter contents field is coded as specified in subclause 9.3.2 of 3GPP TS 24.301 [10] (see NOTE 2). When the parameter identifier indicates mapped EPS QoS parameters, the length and parameter contents field are coded as specified in subclause 9.9.4.3 of 3GPP TS 24.301 [10]. When the parameter identifier indicates mapped extended EPS QoS parameters, the length and parameter contents field are coded as specified in subclause 9.9.4.30 of 3GPP TS 24.301 [10]. When the parameter identifier indicates traffic flow template, the length and parameter contents field are coded from octet 2 as shown figure 10.5.144 and table 10.5.162 of 3GPP TS 24.008 [8].

NOTE 1: A packet filter with the "bidirectional" packet filter direction is used both as a packet filter with the "downlink only" packet filter direction and a packet filter with the "uplink only" packet filter direction. NOTE 2: The total number of EPS bearer identities included in all QoS rules of a UE cannot exceed eleven.

#### 9.8.4.7	Session-AMBR

The purpose of the Session-AMBR information element is to indicate the initial subscribed PDU session aggregate maximum bit rate when the UE establishes a PDU session or to indicate the new subscribed PDU session aggregate maximum bit rate if it is changed by the network.

The Session-AMBR information element is coded as shown in figure 9.8.4.7.1 and table 9.8.4.7.1.

The Session-AMBR is a type 4 information element with a length of 8 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Session-AMBR IEI |  |  |  |  |  |  |  | octet 1 |
| Length of Session-AMBR contents |  |  |  |  |  |  |  | octet 2 |
| Unit for Session-AMBR for downlink |  |  |  |  |  |  |  | octet 3 |
| Session-AMBR for downlink |  |  |  |  |  |  |  | octet 4-5 |
| Unit for Session-AMBR for uplink |  |  |  |  |  |  |  | octet 6 |
| Session-AMBR for uplink |  |  |  |  |  |  |  | octet 7-8 |

Figure 9.8.4.7.1: Session-AMBR information element

Table 9.8.4.7.1: Session-AMBR information element

Unit for Session-AMBR for downlink (octet 3) 0 0 0 0 0 0 0 0 value is not used 0 0 0 0 0 0 0 1 value is incremented in multiples of 1 Kbps 0 0 0 0 0 0 1 0 value is incremented in multiples of 4 Kbps 0 0 0 0 0 0 1 1 value is incremented in multiples of 16 Kbps 0 0 0 0 0 1 0 0 value is incremented in multiples of 64 Kbps 0 0 0 0 0 1 0 1 value is incremented in multiples of 256 kbps 0 0 0 0 0 1 1 0 value is incremented in multiples of 1 Mbps 0 0 0 0 0 1 1 1 value is incremented in multiples of 4 Mbps 0 0 0 0 1 0 0 0 value is incremented in multiples of 16 Mbps 0 0 0 0 1 0 0 1 value is incremented in multiples of 64 Mbps 0 0 0 0 1 0 1 0 value is incremented in multiples of 256 Mbps 0 0 0 0 1 0 1 1 value is incremented in multiples of 1 Gbps 0 0 0 0 1 1 0 0 value is incremented in multiples of 4 Gbps 0 0 0 0 1 1 0 1 value is incremented in multiples of 16 Gbps 0 0 0 0 1 1 1 0 value is incremented in multiples of 64 Gbps 0 0 0 0 1 1 1 1 value is incremented in multiples of 256 Gbps 0 0 0 1 0 0 0 0 value is incremented in multiples of 1 Tbps 0 0 0 1 0 0 0 1 value is incremented in multiples of 4 Tbps 0 0 0 1 0 0 1 0 value is incremented in multiples of 16 Tbps 0 0 0 1 0 0 1 1 value is incremented in multiples of 64 Tbps 0 0 0 1 0 1 0 0 value is incremented in multiples of 256 Tbps 0 0 0 1 0 1 0 1 value is incremented in multiples of 1 Pbps 0 0 0 1 0 1 1 0 value is incremented in multiples of 4 Pbps 0 0 0 1 0 1 1 1 value is incremented in multiples of 16 Pbps 0 0 0 1 1 0 0 0 value is incremented in multiples of 64 Pbps 0 0 0 1 1 0 0 1 value is incremented in multiples of 256 Pbps Other values shall be interpreted as multiples of 256 Pbps in this version of the protocol. Session-AMBR for downlink (octets 4 and 5) Octets 4 and 5 represent the binary coded value of PDU session aggregated maximum bit rate for downlink in units defined by octet 3. Unit for Session-AMBR for uplink (octet 6) The coding is identical to the unit coding defined for Session-AMBR for downlink (octet 3) Session-AMBR for uplink (octets 7 and 8) Octets 7 and 8 represent the binary coded value of PDU session aggregated maximum bit rate for uplink in units defined by octet 6.

#### 9.8.4.8	SM PDU DN request container

The SM PDU DN request container contains a DN-specific identity of the UE in the network access identifier (NAI) format.

#### 9.8.4.9	SSC mode

The purpose of the SSC mode information element is to indicate SSC mode.

The SSC mode information element is coded as shown in figure 9.8.4.9.1 and table 9.8.4.9.1.

The SSC mode is a type 1 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SSC mode IEI |  |  |  | SSC mode value |  |  |  | octet 1 |

Figure 9.8.4.9.1: SSC mode information element

Table 9.8.4.9.1: SSC mode information element

| SSC mode value (octet 1, bit 1 to bit 4) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bits |  |  |  |  |
| 4 | 3 | 2 | 1 |  |
| 0 | 0 | 0 | 1 | SSC mode 1 |
| 0 | 0 | 1 | 0 | SSC mode 2 |
| 0 | 0 | 1 | 1 | SSC mode 3 |
| All other values are reserved. |  |  |  |  |

#### 9.8.4.10	5GSM capability

The purpose of the 5GSM capability information element is to indicate UE capability related to the PDU session management.

The 5GSM capability information element is coded as shown in figure 9.8.4.10.1 and table 9.8.4.10.1.

The 5GSM capability is a type 4 information element with a minimum length of 3 octets and a maximum length of 15 octets.

| 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5GSM capability IEI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 1 |  |
| Length of 5GSM capability contents |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 2 |  |
| 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | RqoS |  | octet 3 |  |
| 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | octet 4* -15* |  |
| Spare |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Figure 9.8.4.10.1: 5GSM capability information element

Table 9.8.4.10.1: 5GSM capability information element

| 5GSM capability value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| RqoS(octet 3, bit 1) This bit indicates the 5GSM capability to support reflective QoS. |  |  |  |  |  |
| 0 |  |  |  |  | Reflective QoS not supported |
| 1 |  |  |  |  | Reflective QoS supported |
| All other bits in octet 3 to 15 are spare and shall be coded as zero, if the respective octet is included in the information element. |  |  |  |  |  |

## 9.9	3GPP specific coding information defined within present document

### 9.9.1	Serving network name (SNN)

The serving network name (SNN) is used in the network name field of the AT_KDF_INPUT attribute defined in IETF RFC 5448 [24].

SNN shall take the generic format of an octet string without terminating null characters.

SNN is of maximum length of 1020 octets.

SNN consists of SNN-prefix and SNN-value, delimited by a colon.

SNN-value identifies the serving PLMN.

MCC and MNC in the SNN-PLMN-ID are MCC and MNC of the serving PLMN. If the MNC of the serving PLMN has two digits, then a zero is added at the beginning.

ABNF syntax of SNN is specified in table 9.9.1.1

Table 9.9.1.1: ABNF syntax of SNN

SNN = SNN-prefix ":" SNN-value

SNN-prefix = %x35.47 ; "5G"

SNN-value = SNN-PLMN-ID

SNN-PLMN-ID = SNN-mnc-string SNN-mnc-digits "." SNN-mcc-string SNN-mcc-digits "." SNN-3gppnetwork-string "." SNN-org-string

SNN-mnc-digits = DIGIT DIGIT DIGIT ; MNC of the PLMN ID

SNN-mcc-digits = DIGIT DIGIT DIGIT ; MCC of the PLMN ID

SNN-mnc-string = %x6d.6e.63 ; "mnc" in lower case

SNN-mcc-string = %x6d.63.63 ; "mcc" in lower case

SNN-3gppnetwork-string = %x33.67.70.70.6e.65.74.77.6f.72.6b ; "3gppnetwork" in lower case

SNN-org-string = %x6f.72.67 ; "org" in lower case

NOTE:	SNN-prefix allows for distinguishing of ANID specified in 3GPP TS 24.302 [11] and SNN as either of SNN or ANID can be carried in the AT_KDF_INPUT attribute.

EXAMPLE:	If PLMN ID contains MCC = 234 and MNC = 15, SNN is 5G:mnc015.mcc234.3gppnetwork.org.

# 10	List of system parameters

## 10.1	General

## 10.2	Timers of 5GS mobility management

Timers of 5GS mobility management are shown in table 10.2.1 and table 10.2.2

Table 10.2.1: Timers of 5GS mobility management – UE side

| TIMER NUM. | TIMER VALUE | STATE | CAUSE OF START | NORMAL STOP | ON EXPIRY |
| --- | --- | --- | --- | --- | --- |
| T3510 | TBD | 5GMM-REGISTERED-INITIATED | Transmission of REGISTRATION REQUEST message | REGISTRATION ACCEPT message received or REGISTRATION REJECT message received | Retransmission of REGISTRATION REQUEST message |
| T3502 | TBD | 5GMM-REGISTERED | At registration failure and the attempt counter is equal to 5 | Transmission of REGISTRATION REQUEST message | Initiation of the registration procedure, if still required |
| T3511 | TBD | 5GMM-DEREGISTERED.ATTEMPTING-REGISTRATION 5GMM-REGISTERED.ATTEMPTING-REGISTRATION-UPDATE | At registration failure due to lower layer failure, T3510 timeout or registration rejected with other 5GMM cause values than those treated in subclause 5.5.1.2.5 for initial registration or subclause 5.5.1.3.5 for mobility and periodic registration | Transmission of REGISTRATION REQUEST message | Retransmission of the REGISTRATION REQUEST, if still required |
| T3512 | TBD | 5GMM-REGISTERED | In 5MM-REGISTERED, when 5MM-CONNECTED mode is left | When entering state 5MM-DEREGISTERED or when entering 5MM-CONNECTED mode | Initiation of the periodic registration procedure if the UE is not registered for emergency services. Locally deregister if the UE is registered for emergency services |
| T3517 | TBD | 5GMM-SERVICE-REQUEST-INITIATED | Transmission of SERVICE REQUEST message | SERVICE ACCEPT message received, or SERVICE REJECT message received | Abort the procedure |
| T3521 | TBD | 5GMM-DEREGISTERED-INITIATED | Transmission of DEREGISTRATION REQUEST message when de-registration procedure is not due to a "switch off" | DEREGISTRATION ACCEPT message received | Retransmission of DEREGISTRATION REQUEST message |
| T3540 | TBD | 5GMM-REGISTERED-INITIATED 5GMM-DEREGISTERED-INITIATED 5GMM-SERVICE-REQUEST-INITIATED | REGISTRATION REJECT, DEREGISTRATION REQUEST, with any of the 5GMM cause #8, #11, #12 or #13 SERVICE REJECT received with any of the 5GMM cause #8, #11, #12 or #13 | N1 NAS signalling connection released Bearers have been set up | Release the NAS signalling connection for the cases a) and b) as described in subclause 5.3.1.2 |
|  |  | 5GMM-DEREGISTERED 5GMM-DEREGISTERED.NORMAL-SERVICE | REGISTRATION REJECT, SERVICE REJECT with the 5GMM cause #10 | N1 NAS signalling connection released | Release the NAS signalling connection for the cases c) as described in subclause 5.3.1.2 and initiation of the registration procedure as specified in subclause 5.5.1.2.2 or 5.5.1.3.2 |

Table 10.2.2: Timers of 5GS mobility management – AMF side

| TIMER NUM. | TIMER VALUE | STATE | CAUSE OF START | NORMAL STOP | ON EXPIRY |
| --- | --- | --- | --- | --- | --- |
| T3550 | TBD | 5GMM-COMMON-PROCEDURE-INITIATED | Transmission of REGISTRATION ACCEPT message at initial registration. Transmission of REGISTRATION ACCEPT message with 5G-GUTI at mobility or periodic registration | REGISTRATION COMPLETE message received | Retransmission of REGISTRATION ACCEPT message |
| T3560 | TBD | 5GMM-COMMON-PROCEDURE-INITIATED | AUTHENTICATION REQUEST message sent SECURITY MODE COMMAND message sent | AUTHENTICATION RESPONSE message received AUTHENTICATION FAILURE message received SECURITY MODE COMPLETE received SECURITY MODE REJECT received | Retransmission of AUTHENTICATION REQUEST message or SECURITY MODE COMMAND |
| T3570 | TBD | 5GMM-COMMON-PROCEDURE-INITIATED | Transmission of IDENTITY REQUEST message | IDENTITY RESPONSE message received | Retransmission of IDENTITY REQUEST message |
| T3513 | TBD | 5GMM-REGISTERED | Paging procedure initiated | Paging procedure completed as specified in subclause 5.6.2.2.1 | Network dependent |
| T3522 | TBD | 5GMM-DEREGISTERED-INITIATED | Transmission of DEREGISTRATION REQUEST message | DEREGISTRATION ACCEPT message received | Retransmission of DEREGISTRATION REQUEST message |
| T3555 | TBD | 5GMM-REGISTERED | Transmission of CONFIGURATION UPDATE COMMAND message with Acknowledgement requested flag IE | CONFIGURATION UPDATE COMPLETE message received | Retransmission of CONFIGURATION UPDATE COMMAND message |
| T3565 | TBD | 5GMM-REGISTERED | Transmission of NOTIFICATION message | SERVICE REQUEST message received | Trigger of service request procedure |
| Mobile reachable timer | NOTE 1 | All except 5GMM-DEREGISTERED | Entering 5GMM-IDLE mode | N1 NAS signalling connection established | Network dependent, but typically paging is halted on 1st expiry, and start implicit de-registration timer, if the UE is not registered for emergency services. Implicitly detached-register the UE which is registered for emergency services |
| Implicit de-registration timer | NOTE 2 | All except 5GMM-DEREGISTERED | The mobile reachable timer expires while the network is in 5GMM-IDLE mode | N1 NAS signalling connection established | Implicitly detach the UE on 1st expiry |
| NOTE 1: The default value of this timer is 4 minutes greater than T3512. If the UE is registered for emergency services, the value of this timer is set equal to T3512. NOTE 2: The value of this timer is network dependent. If MICO is activated, the default value of this timer is 4 minutes greater than T3512. |  |  |  |  |  |

## 10.3	Timers of 5GS session management

Timers of 5GS session management are shown in table 10.3.1 and table 10.3.2.

Table 10.3.1: Timers of 5GS session management – UE side

| TIMER NUM. | TIMER VALUE | STATE | CAUSE OF START | NORMAL STOP | ON THE 1st, 2nd, 3rd, 4th EXPIRY (NOTE 1) |
| --- | --- | --- | --- | --- | --- |
| T3580 | TBD | TBD | Transmission of PDU SESSION ESTABLISHMENT REQUEST message | PDU SESSION ESTABLISHMENT ACCEPT message received or PDU SESSION ESTABLISHMENT REJECT message received | Retransmission of PDU SESSION ESTABLISHMENT REQUEST message |
| T3581 | TBD | TBD | Transmission of PDU SESSION MODIFICATION REQUEST message | PDU SESSION MODIFICATION COMMAND message with the same PTI is received or PDU SESSION MODIFICATION REJECT message received | Retransmission of PDU SESSION MODIFICATION REQUEST message |
| T3582 | TBD | TBD | Transmission of PDU SESSION RELEASE REQUEST message | PDU SESSION RELEASE COMMAND message with the same PTI is received or PDU SESSION RELEASE REJECT message received | Retransmission of PDU SESSION RELEASE REQUEST message |
| NOTE 1: Typically, the procedures are aborted on the fifth expiry of the relevant timer. Exceptions are described in the corresponding procedure description. |  |  |  |  |  |

Table 10.3.2: Timers of 5GS session management – SMF side

| TIMER NUM. | TIMER VALUE | STATE | CAUSE OF START | NORMAL STOP | ON THE 1st, 2nd, 3rd, 4th EXPIRY (NOTE 1) |
| --- | --- | --- | --- | --- | --- |
| T3590 | TBD | TBD | Transmission of PDU SESSION AUTHENTICATION COMMAND message | PDU SESSION AUTHENTICATION COMPLETE message received | Retransmission of PDU SESSION AUTHENTICATION COMMAND message |
| T3591 | TBD | TBD | Transmission of PDU SESSION MODIFICATION COMMAND message with any 5GSM cause but #39 | PDU SESSION MODIFICATION COMPLETE message received or PDU SESSION MODIFICATION COMMAND REJECT message received | Retransmission of PDU SESSION MODIFICATION COMMAND message |
|  |  |  | Transmission of PDU SESSION MODIFICATION COMMAND message with 5GSM cause #39. | PDU SESSION RELEASE REQUEST message is received | Retransmission of PDU SESSION MODIFICATION COMMAND message |
| T3591 | TBD | TBD | Transmission of PDU SESSION RELEASE COMMAND message | PDU SESSION RELEASE COMPLETE message received or N1 SM delivery skipped indication received | Retransmission of PDU SESSION RELEASE COMMAND message |
| NOTE 1: Typically, the procedures are aborted on the fifth expiry of the relevant timer. Exceptions are described in the corresponding procedure description. |  |  |  |  |  |

Annex A (informative):
Cause values for 5GS mobility management

## A.1	Causes related to UE identification

Cause #3 – Illegal UE

This 5GMM cause is sent to the UE when the network refuses service to the UE either because an identity of the UE is not acceptable to the network or because the UE does not pass the authentication check..

Cause #6 – Illegal ME

This 5GMM cause is sent to the UE if the ME used is not acceptable to the network, e.g. blacklisted.

Cause #10 – Implicitly de-registered

This 5GMM cause is sent to the UE either if the network has implicitly de-registered the UE, e.g. after the implicit de-registration timer has expired, or if the 5GMM context data related to the subscription does not exist in the AMF e.g. because of a AMF restart, or because of a registration request for mobility or registration update is routed to a new AMF.

## A.2	Cause related to subscription options

Cause #5 – PEI not accepted

This cause is sent to the UE if the network does not accept an initial registration procedure for emergency services using a PEI.

Cause #8 – 5GS services not allowed

This 5GMM cause is sent to the UE when it is not allowed to operate 5GS services.

Cause #11 – PLMN not allowed

This 5GMM cause is sent to the UE if it requests service, or if the network initiates a de-registration request, in a PLMN where the UE, by subscription or due to operator determined barring, is not allowed to operate.

Cause #12 – Tracking area not allowed

This 5GMM cause is sent to the UE if it requests service, or if the network initiates a de-registration request, in a tracking area where the HPLMN determines that the UE, by subscription, is not allowed to operate.

NOTE 1:	If 5GMM cause #12 is sent to a roaming subscriber the subscriber is denied service even if other PLMNs are available on which registration was possible.

Cause #13 – Roaming not allowed in this tracking area

This 5GMM cause is sent to a UE which requests service, or if the network initiates a de-registration request, in a tracking area of a PLMN which by subscription offers roaming to that UE but not in that tracking area.

Cause #xx – N1 mode not allowed

This 5GMM cause is sent to the UE if it requests service, or if the network initiates a de-registration request, in a PLMN where the UE by subscription, is not allowed to operate in N1 mode.

## A.3	Causes related to PLMN specific network failures and congestion/authentication failures

Cause #21 – Synch failure

This 5GMM cause is sent to the network if the USIM detects that the SQN in the AUTHENTICATION REQUEST message is out of range.

Cause #22 – Congestion

This 5GMM cause is sent to the UE because of congestion in the network (e.g. no channel, facility busy/congested etc.).

Cause #23 – UE security capabilities mismatch

This 5GMM cause is sent to the network if the UE detects that the UE security capability does not match the one sent back by the network.

Cause #24 – Security mode rejected, unspecified

This 5GMM cause is sent to the network if the security mode command is rejected by the UE if the UE detects that the nonceUE does not match the one sent back by the network or for unspecified reasons.

Cause #xy – Restricted service area

This 5GMM cause is sent to the UE if it requests service in a tracking area which is a part of the UE’s non-allowed area or is not a part of the UE’s allowed area.

## A.4	Causes related to invalid messages

Cause #95 – Semantically incorrect message.

This 5GMM cause is used to report receipt of a message with semantically incorrect contents.

Cause #96 – Invalid mandatory information.

This cause 5GMM indicates that the equipment sending this 5GMM cause has received a message with a non-semantical mandatory IE error.

Cause #97 – Message type non-existent or not implemented.

This 5GMM cause indicates that the equipment sending this 5GMM cause has received a message with a message type it does not recognize either because this is a message not defined, or defined but not implemented by the equipment sending this 5GMM cause.

Cause #98 – Message type not compatible with protocol state.

This 5GMM cause indicates that the equipment sending this 5GMM cause has received a message not compatible with the protocol state.

Cause #99 – Information element non-existent or not implemented.

This 5GMM cause indicates that the equipment sending this 5GMM cause has received a message which includes information elements not recognized because the information element identifier is not defined or it is defined but not implemented by the equipment sending the 5GMM cause. However, the information element is not required to be present in the message in order for the equipment sending the 5GMM cause to process the message.

Cause #100 – Conditional IE error.

This 5GMM cause indicates that the equipment sending this cause has received a message with conditional IE errors.

Cause #101 – Message not compatible with protocol state.

This 5GMM cause indicates that a message has been received which is incompatible with the protocol state.

Cause #111 – Protocol error, unspecified.

This 5GMM cause is used to report a protocol error event only when no other 5GMM cause in the protocol error class applies.

Annex B (informative):
Cause values for 5GS session management

## B.1	Causes related to nature of request

Cause #26 – Insufficient resources

This 5GSM cause is used by the UE or by the network to indicate that the requested service cannot be provided due to insufficient resources.

Editor's note:	Semantic of 5GSM cause value #26 includes semantic of 5GSM cause value #ss. If both 5GSM cause values are needed, semantics of 5GSM cause values have to be different.

Cause #27 – Missing or unknown DNN

This 5GSM cause is used by the network to indicate that the requested service was rejected by the external DN because the DNN was not included although required or if the DNN could not be resolved.

Cause #29 – User authentication failed

This 5GSM cause is used by the network to indicate that the requested service was rejected by the external DN due to a failed user authentication.

Cause #31 – Request rejected, unspecified

This 5GSM cause is used by the network or by the UE to indicate that the requested service or operation or the request for a resource was rejected due to unspecified reasons.

Cause #36 – Regular deactivation

This 5GSM cause is used to indicate a regular UE or network initiated release of PDU session resources.

Cause #39 – Reactivation requested

This 5GSM cause is used by the network to request a PDU session reactivation.

Cause #50 – PDU session type Ipv4 only allowed

This 5GSM cause is used by the network to indicate that only PDU session type Ipv4 is allowed for the requested IP connectivity.

Cause #51 – PDU session type Ipv6 only allowed

This 5GSM cause is used by the network to indicate that only PDU session type Ipv6 is allowed for the requested IP connectivity.

Cause #ss – Insufficient resources for specific slice and DNN;

This 5GSM cause is by the network to indicate that the requested service cannot be provided due to insufficient resources for specific slice and DNN.

Cause #ss – Not supported SSC mode;

This 5GSM cause is used by the network to indicate that the requested SSC mode is not supported.

Editor's note:	Semantic of 5GSM cause value #26 includes semantic of 5GSM cause value #ss. If both 5GSM cause values are needed, semantics of 5GSM cause values have to be different.

## B.2	Protocol errors (e.g., unknown message)

Cause #95 – Semantically incorrect message

This 5GSM cause is used to report receipt of a message with semantically incorrect contents.

Cause #96 – Invalid mandatory information

This 5GSM cause indicates that the equipment sending this 5GSM cause has received a message with a non-semantical mandatory IE error.

Cause #97 – Message type non-existent or not implemented

This 5GSM cause indicates that the equipment sending this 5GSM cause has received a message with a message type it does not recognize either because this is a message not defined, or defined but not implemented by the equipment sending this 5GSM cause.

Cause #98 – Message type not compatible with protocol state

This 5GSM cause indicates that the equipment sending this 5GSM cause has received a message not compatible with the protocol state.

Cause #99 – Information element non-existent or not implemented

This 5GSM cause indicates that the equipment sending this 5GSM cause has received a message which includes information elements not recognized because the information element identifier is not defined or it is defined but not implemented by the equipment sending the 5GSM cause. However, the information element is not required to be present in the message in order for the equipment sending the 5GSM cause to process the message.

Cause #100 – Conditional IE error

This 5GSM cause indicates that the equipment sending this cause has received a message with conditional IE errors.

Cause #101 – Message not compatible with protocol state

This 5GSM cause indicates that a message has been received which is incompatible with the protocol state.

Cause #111 – Protocol error, unspecified

This 5GSM cause is used to report a protocol error event only when no other 5GSM cause in the protocol error class applies.

Annex C (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | Tdoc | CR | Rev | Cat | Subject/Comment | New version |
| 2017-10 | CT1#106 | C1-174182 |  |  |  | Draft skeleton provided by the rapporteur. | 0.0.0 |
| 2017-11 | CT1#106 |  |  |  |  | Implementing the following p-CRs agreed by CT1: C1-174183, C1-174184, C1-174185. | 0.1.0 |
| 2017-12 | CT1#107 |  |  |  |  | Implementing the following p-CRs agreed by CT1: C1-175098, C1-175313. Corrections done by the rapporteur. | 0.2.0 |
| 2017-12 | CT1 e-mail review |  |  |  |  | Editorial corrections. | 0.2.1 |
| 2017-12 | CT1 e-mail review |  |  |  |  | Re-introduction of table in subclause 8.2.23.1 | 0.2.2 |
| 2018-02 | CT1#108 |  |  |  |  | Implementing the following p-CRs agreed by CT1: C1-180663, C1-180224, C1-180046, C1-180437, C1-180438, C1-180448, C1-180307, C1-180211, C1-180316, C1-180221, C1-180281, C1-180339, C1-180361, C1-180148, C1-180415, C1-180451, C1-180453, C1-180455, C1-180459, C1-180482, C1-180483, C1-180484, C1-180619, C1-180620, C1-180623, C1-180624, C1-180627, C1-180628, C1-180664, C1-180665, C1-180668, C1-180672, C1-180673, C1-180679, C1-180680, C1-180684, C1-180707, C1-180721, C1-180725, C1-180736, C1-180737, C1-180738, C1-180739, C1-180740, C1-180741, C1-180750, C1-180751, C1-180013, C1-180311, C1-180312, C1-180197, C1-180313, C1-180283, C1-180037, C1-180041, C1-180464, C1-180465, C1-180466, C1-180469, C1-180645, C1-180646, C1-180648, C1-180688, C1-180689, C1-180690, C1-180473, C1-180720, C1-180226, C1-180632, C1-180633, C1-180635, C1-180640, C1-180669, C1-180731, C1-180732, C1-180734, C1-180735, C1-180746, C1-180209, C1-180040, C1-180015, C1-180035, C1-180198, C1-180421, C1-180487, C1-180488, C1-180490, C1-180621, C1-180622, C1-180701, C1-180162, C1-180190, C1-180604, C1-180605, C1-180606, C1-180611, C1-180614, C1-180616, C1-180704, C1-180719, C1-180722, C1-180747, C1-180755, C1-180756 Corrections done by the rapporteur. | 0.3.0 |