| 3GPP TS 24.514 V0.1.0 (2023-04) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Ranging based services and sidelink positioning in 5G system(5GS); Stage 3; (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Abbreviations	7

4	General description	7

5	Provisioning of configuration information for ranging and sidelink positioning service	7

5.1	General	7

5.2	Configuration and precedence of ranging and sidelink positioning configuration parameters	7

5.3	Procedures	7

5.3.1	General	8

5.3.2	UE-requested RSLPP provisioning procedure	8

5.3.2.1	General	8

5.3.2.2	UE-requested RSLPP provisioning procedure initiation	8

5.3.2.3	UE-requested RSLPP provisioning procedure accepted by the network	9

5.3.2.4	UE-requested RSLPP provisioning procedure not accepted by the network	9

5.3.2.5	Abnormal cases on the network side	9

5.3.2.6	Abnormal cases on the UE	10

6	Ranging and sidelink positioning UE discovery	10

6.1	Overview	10

6.2	Ranging and sidelink positioning direct discovery	10

6.2.1	General	10

6.2.2	5G ProSe capable UEs discovery	10

6.2.3	V2X capable UEs discovery	10

6.3	Located UE discovery	10

7	Ranging and sidelink positioning communication	10

7.1	Overview	10

7.2	Ranging and sidelink positioning direct communication over PC5	10

7.2.1	General	10

7.2.2	Unicast mode ranging and sidelink positioning direct communication over PC5	11

7.2.3	Groupcast mode ranging and sidelink positioning direct communication over PC5	11

7.2.4	Broadcast mode ranging and sidelink positioning direct communication over PC5	11

7.3	Ranging and sidelink positioning communication on LCS aspect	11

8.	Handling of unknown, unforeseen, and erroneous PC5 signalling protocol data	12

8.1	General	12

9.	Message functional definition and contents	12

9.1	Overview	12

10.	Information elements coding	12

10.1	Overview	12

10.2	General	12

Annex <X> (informative):  Change history	13

Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document specifies the protocols for ranging based service and sidelink positioning in 5G system as specified in 3GPP TS 23.586 [2] for the following aspects:

a)	provisioning of configuration information for ranging and sidelink positioning service;

b)	ranging and sidelink positioning UE discovery; and

c)	ranging and sidelink positioning communication, including the communication among UEs and the communication between UE and LMF. The support of ranging and sidelink positioning protocol (RSPP) transport is defined as part of the ranging and sidelink positioning communication.

The present document defines the associated procedures for the aspects listed above, and also defines the message format, message contents, error handling and system parameters applied by the protocols for ranging based service and sidelink positioning in 5GS.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 23.586: " Ranging based services and Sidelink Positioning ".

[3]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[4]	3GPP TS 24.587: "Vehicle-to-Everything (V2X) services in 5G System (5GS); Protocol aspects; Stage 3".

[5]	3GPP TS 33.533: "Security aspects of ranging based services and sidelink positioning".

[6]	3GPP TS 24.554: "Proximity-services (ProSe) in 5G System (5GS) protocol aspects"

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

RSLPP	Ranging and sidelink provisioning policy

4	General description

Any UE supporting ranging and sidelink positioning, e.g. target UE, reference UE, sidelink positioning server UE, supports a ranging and sidelink positioning (RSP) layer. The RSP layer handles service requests received from application layer to control the ranging and sidelink positioning operation.

The RSP layer supports the following functionalities:

a)	Provisioning of configuration information and authorization for ranging and sidelink positioning;

b)	UE discovery and selection; and

c)	Ranging and sidelink positioning communication between UEs or between the UE and the LMF.

Transport of ranging and sidelink positioning protocol (RSPP) payload over PC5-U is supported by the ranging and sidelink positioning communication between UEs over PC5.

NOTE:	RSPP is defined in 3GPP TS 38.xxx [ts38xxx]

Editor’s note:	The specification number for RSPP (SLPP for RAN) will be added based on RAN WG outcome.

Editor’s note:	The protocol used between UE and LMF will be decided by RAN.

The above functions are applicable for both public safety UE and commercial UEs.

The security aspects for 5G ranging and sidelink positioning features are specified in 3GPP TS 33.533 [5].

5	Provisioning of configuration information for ranging and sidelink positioning service

5.1	General

Editor’s Note:	This clause will provide description of provisioning of Raning_SL configuration.

5.2	Configuration and precedence of ranging and sidelink positioning configuration parameters

Editor’s Note:	This clause will provide description of Ranging_SL configuration precedence and parameters.

5.3	Procedures

Editor’s Note:	This clause will provide procedures to provision Ranging_SL configuration parameter

5.3.1	General

The procedure for provisioning of parameters for ranging and sidelink positioning over PC5 allows the UE to obtain the ranging and sidelink positioning over PC5 policy(RSLPP).

5.3.2	UE-requested RSLPP provisioning procedure

5.3.2.1	General

The UE-requested RSLPP provisioning procedure enables the UE to request RSLPP from the PCF in the following cases:

a)	if the T5aaa for a UE policies for ranging and sidelink positioning over PC5 expires;

b)	if the T5bbb for a UE policies for ranging and sidelink positioning located UE expires;

c)	if the T5ccc for a UE policies for ranging and sidelink positioning target UE expires;

d)	if the T5ddd for UE policies for ranging and sidelink positioning client UE expires;

e)	if the T5eee for UE policies for ranging and sidelink positioning server UE expires; and

f)	if there are no valid configuration parameters, e.g., for the current area, or due to abnormal situation.

The UE shall follow the principles of PTI handling for UE policy delivery service procedures defined in 3GPP TS 24.501 [3] clause D.1.2.

5.3.2.2	UE-requested RSLPP provisioning procedure initiation

In order to initiate the UE-requested RSLPP provisioning procedure, the UE shall create a UE POLICY PROVISIONING REQUEST message (see example in figure 5.3.2.2.1). The UE:

a)	shall allocate a PTI value currently not used and set the PTI IE to the allocated PTI value;

b)	shall include the Requested UE policies IE indicating whether the UE policies for ranging and sidelink positioning over PC5, the UE policies for ranging and sidelink positioning located UE, the UE policies for ranging and sidelink positioning target UE, the UE policies for ranging and sidelink positioning client UE, the UE policies for ranging and sidelink positioning server UE, or any combination of them are requested;

c)	shall transport the UE POLICY PROVISIONING REQUEST message using the NAS transport procedure as specified in 3GPP TS 24.501 [3] clause 5.4.5; and

d)	shall start timer T5040.

Figure 5.3.2.2.1: UE-requested RSLPP provisioning procedure

5.3.2.3	UE-requested RSLPP provisioning procedure accepted by the network

Handling in 3GPP TS 24.587 [4] clause 5.3.2.3 shall apply.

If new UE policies for ranging and sidelink positioning over PC5 are included in the MANAGE UE POLICY COMMAND message, the UE shall stop timer T5aaa if it is running and start timer T5aaa with the value included in the UE policies for ranging and sidelink positioning over PC5 and start using the new UE policies for ranging and sidelink positioning over PC5 included in the MANAGE UE POLICY COMMAND message.

If new UE policies for ranging and sidelink positioning located UE are included in the MANAGE UE POLICY COMMAND message, the UE shall stop timer T5bbb if it is running and start timer T5bbb with the value included in the UE policies for ranging and sidelink positioning located UE and start using the new UE policies for ranging and sidelink positioning located UE included in the MANAGE UE POLICY COMMAND message.

If new UE policies for ranging and sidelink positioning target UE are included in the MANAGE UE POLICY COMMAND message, the UE shall stop timer T5ccc if it is running and start timer T5ccc with the value included in the UE policies for ranging and sidelink positioning target UE and start using the new UE policies for ranging and sidelink positioning target UE included in the MANAGE UE POLICY COMMAND message.

If new UE policies for ranging and sidelink positioning client UE are included in the MANAGE UE POLICY COMMAND message, the UE shall stop timer T5ddd if it is running and start timer T5ddd with the value included in the UE policies for ranging and sidelink positioning client UE and start using the new UE policies for ranging and sidelink positioning client UE included in the MANAGE UE POLICY COMMAND message.

If new UE policies for ranging and sidelink positioning server UE are included in the MANAGE UE POLICY COMMAND message, the UE shall stop timer T5eee if it is running and start timer T5eee with the value included in the UE policies for ranging and sidelink positioning server UE, and start using the UE policies for ranging and sidelink positioning server UE included in the MANAGE UE POLICY COMMAND message.

5.3.2.4	UE-requested RSLPP provisioning procedure not accepted by the network

Handling in 3GPP TS 24.587 [4] clause 5.3.2.4 shall apply.

5.3.2.5	Abnormal cases on the network side

Handling in 3GPP TS 24.587 [4] clause 5.3.2.5 shall apply.

5.3.2.6	Abnormal cases on the UE

Handling in 3GPP TS 24.587 [4] clause 5.3.2.6 shall apply.

6	Ranging and sidelink positioning UE discovery

6.1	Overview

Editor’s Note:	This clause will provide description of Ranging_SL UE discovery.

6.2	Ranging and sidelink positioning direct discovery

6.2.1	General

Editor’s Note:	This clause will provide description at the UE, and between UEs, for the general direct discovery used for ranging and sidelink positioning.

6.2.2	5G ProSe capable UEs discovery

Editor’s Note:	This clause will provide description and the procedures at the UE, and between UEs, for the 5G ProSe capable UEs used for ranging and sidelink positioning. The description may just be brief, and the details of direct discovery procedures may refer to that defined in 3GPP TS 24.554.

6.2.3	V2X capable UEs discovery

Editor’s Note:	This clause will provide description and the procedures at the UE, and between UEs, for the 5G V2X capable UEs used for ranging and sidelink positioning. The description may just be brief, and the details of direct discovery procedures may refer to that defined in 3GPP TS 24.287.

6.3	Located UE discovery

Editor’s Note:	This clause will provide description of the procedures at the UE, between UEs, and between the UE and the LMF for the located UE discovery.

7	Ranging and sidelink positioning communication

7.1	Overview

Editor’s Note:	This clause will provide description of Ranging_SL communication.

7.2	Ranging and sidelink positioning direct communication over PC5

Editor’s Note:	This clause will provide description and the procedures for Ranging_SL direct communication over PC5 .

7.2.1	General

Depending on type of the UE (e.g. V2X capable or 5G ProSe capable), V2X communication procedures as defined in TS 24.587 [4] or 5G ProSe direct communication procedures as defined in 3GPP TS 24.554 [6] are used for ranging and sidelink positioning communication over PC5.

For the RSPP transport between UEs over PC5, the following modification is applied:

a)	For V2X capable UEs, V2XP is used to determine the corresponding transport configurations for the RSPP signalling. The V2X service identifier shall take the value(s) defined for "Ranging/SL Positioning Protocol".

b)	For 5G ProSe capable UEs, ProSeP is used to determine the corresponding transport configuration for the RSPP signalling. The ProSe identifier shall take the value(s) defined for "Ranging/SL Positioning Protocol".

NOTE:	For a UE with both V2X capability and 5G ProSe capability, separate RSPP transport links per the capability are used, i.e., the RSPP transport link over V2X communication is independent with the RSPP transport link over 5G ProSe direct communication.

The RSP layer supports broadcast mode, groupcast mode, and unicast mode PC5 communication depending on the policy and parameter configuration in the UE.

"Non-IP" layer-3 protocol data unit type for V2X capable UEs and "Unstructured" layer-3 protocol data unit type for 5G ProSe capable UEs are used for the transport of RSPP payload.

7.2.2	Unicast mode ranging and sidelink positioning direct communication over PC5

Editor’s Note:	This clause will provide description of the procedures for unicast mode Ranging_SL direct communication over PC5.

7.2.3	Groupcast mode ranging and sidelink positioning direct communication over PC5

Editor’s Note:	This clause will provide description of the procedures for groupcast Ranging_SL direct communication over PC5.

7.2.4	Broadcast mode ranging and sidelink positioning direct communication over PC5

Editor’s Note:	This clause will provide description of the procedures for broadcast mode Ranging_SL direct communication over PC5.

7.3	Ranging and sidelink positioning communication on LCS aspect

Editor’s Note:	This clause will provide description and the procedures for Ranging_SL communication on LCS aspect, e.g. to support Network assisted Sidelink Positioning and Ranging/SL Positioning service exposure. The description may just be brief and help to give a whole picture of the solution for Ranging_SL, and the details of LCS signalling may refer to that defined in 3GPP TS 24.571.

8.	Handling of unknown, unforeseen, and erroneous PC5 signalling protocol data

8.1	General

Editor’s Note:	This clause will provide description for handling of unknown, unforeseen, and erroneous signalling protocol data

9.	Message functional definition and contents

9.1	Overview

Editor’s Note:	This clause will provide description of the message functional definition and contents for this specification.

10.	Information elements coding

10.1	Overview

Editor’s Note:	This clause will provide information elements coding for RANGING_SL.

10.2	General

3GPP

Annex <X> (informative): 
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2023-04 | CT1#141e | C1-232663 |  |  |  | TS skeleton proposed by the rapporteur | 0.0.0 |
| 2023-04 | CT1#141e | C1-232639 C1-232665 C1-232783 |  |  |  | Implementing the following p-CR agreed by CT1: C1-232639, C1-232665, C1-232783; and Editorial change from the rapporteur. | 0.1.0 |