| 3GPP TS 26.258 V0.0.1 (2023-08) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Codec for immersive voice and audio services; C code (floating-point) (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	8

4	Examples for styles	8

4.1	Heading styles	8

4.2	Other common styles	8

"TSG <Name>" on the front page	9

Page setup parameters	9

Proforma copyright release text block	11

X.1	The right to copy	11

Abstract Test Suite (ATS) text block	12

Y	Abstract Test Suite (ATS)	12

Y.1	Introduction	12

Y.2	The TTCN Graphical form (TTCN.GR)	12

Y.3	The TTCN Machine Processable form (TTCN.MP)	12

Annex <A> (normative): <Normative annex for a Technical Specification>	13

Annex <B> (informative): <Informative annex for a Technical Specification>	14

B.1	Heading levels in an annex	14

Annex <C>: <Informative annex title for a Technical Report>	15

Annex <D> (informative): Bibliography	16

Annex <E> (informative): Index	17

Annex <X> (informative): Change history	18

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document contains an electronic copy of the floating-point C code for the Immersive Voice and Audio Services (IVAS) Codec. This C code is the unique alternative reference specification besides the fixed-point C code for the IVAS Codec (3GPP TS 26.251) for a standard compliant implementation of the IVAS Codec (3GPP TS 26.253), Rendering (3GPP TS 26.254), Error Concealment of Lost Packets (3GPP TS 26.255) and Jitter Buffer Management (JBM) (3GPP TS 26.256).

[The bit-exact fixed-point C code in 3GPP TS 26.251 is the preferred implementation for all applications, but the floating-point codec may be used instead of the fixed-point codec when the implementation platform is better suited for a floating-point implementation. ]

Requirements for any implementation of the IVAS codec to be standard compliant are specified in 3GPP TS 26.252 (Test sequences).

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 26.253: "Codec for immersive voice and audio services - Detailed Algorithmic Description incl. RTP payload format and SDP parameter definitions".

[3]	3GPP TS 26.254: "Codec for immersive voice and audio services - Rendering".

[4]	3GPP TS 26.255: "Codec for immersive voice and audio services - Error concealment of lost packets".

[5]	3GPP TS 26.256: "Codec for immersive voice and audio services - Jitter Buffer Management".

[6]	IETF RFC 3550: "RTP: A Transport Protocol for Real-Time Applications".

[7]	Recommendation ITU-T G.191 (03/23): "Software tools for speech and audio coding standardization".

[8]	Recommendation ITU-T G.192: "A common digital parallel interface for speech standardization activities".

[9]	ISO/IEC 23008-3:2015: “High efficiency coding and media delivery in heterogeneous environments — Part 3: 3D audio”

[10]	ISO/IEC 23091-3:2018: “Coding-independent code points — Part 3: Audio“

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

Void.

## 3.2	Symbols

Void.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

EVS	Enhanced Voice Services

FB	Fullband

FEC	Frame Erasure Concealment

IVAS	Immersive Voice and Audio Services

JBM	Jitter Buffer Management

LFE	Low Frequency Enhancement

NB	Narrowband

SID	Silence Insertion Descriptor

SWB	Super Wideband

WB	Wideband

WMOPS	Weighted Millions of Operations Per Second

# 4	C code structure

This clause gives an overview of the structure of the floating-point C code and provides an overview of the contents and organization of the C code attached to the present document.

The C code has been verified on the following platforms:

-	IBM PC compatible computers with Windows 10 operating systems and Microsoft Visual C++ 2017 compiler, 32-bit.

C was selected as the programming language because portability was desirable.

## 4.1	Contents of the C source code

The C code is organized as listed in Table 1:

Table 1: Source code directory structure

| Directory | Description |
| --- | --- |
| readme.txt | information on how to compile and use |
| Makefile | UNIX style encoder Makefile |
| Workspace_msvc/ | Directory for the MSVC 2017 (or newer) project files |
| apps/ | Source code files used solely for the encoder/decoder/renderer applications; these applications make use of the libraries built from lib_com, lib_dec, lib_enc, lib_rend, lib_lc3plus, and lib_util |
| lib_com/ | Source code files used both in encoder and decoder |
| lib_dec/ | Source code files used solely in the decoder |
| lib_enc/ | Source code files used solely in the encoder |
| lib_lc3plus/ | Source code files used solely in the LC3plus part of the binaural split rendering scheme |
| lib_rend/ | Source code files used solely in the renderer |
| lib_util/ | Source code files solely for utility functions used by the applications |

The distributed files with suffix "c" contain the source code and the files with suffix "h" are the header files. The ROM data is contained in files named "rom_*" and “ivas_rom_*” with suffix "c".

Makefiles are provided for the platforms in which the C code has been verified (listed above). Once the software is installed, this directory will have a compiled version of the encoder (named IVAS_cod), the decoder (named IVAS_dec) and the renderer (named IVAS_rend).

## 4.2	Program execution

The codec for Immersive Voice and Audio Services is implemented in three programs:

-	IVAS_cod: encoder;

-	IVAS_dec: decoder;

- 	IVAS_rend: renderer.

The programs should be called like:

-	IVAS_cod [encoder options] <input file> <bitstream file>;

-	IVAS_dec [decoder options]<bitstream file> <output file>;

- 	IVAS_rend [renderer options] -i <input file> -if <input format> -o <output file> -of <output format>.

The input and output files contain 16-bit linear encoded PCM samples and the bitstream file contains encoded data.

The encoder, decoder, and renderer options will be explained by running the programs without any input arguments. See the file readme.txt for more information on how to run the IVAS_enc, IVAS_dec and IVAS_rend programs.

# 5	File Formats

This clause describes the file formats used by the encoder and decoder programs. The test sequences defined in [1] also use the file formats described here.

# 5.1	Audio Input/output file format

For the input files read by the encoder/renderer and output files written by the decoder/renderer the following formats are supported:

- Headerless format: 16-bit integer words per each data sample. The byte order in each word depends on the host architecture (e.g. LSB first on PCs, etc.).

- WAVE format: 16-bit little-endian integer words per each data sample.

Both the encoder and the decoder program process complete frames corresponding to multiples of 20 ms.

The encoder will pad the last frame to integer multiples of 20ms frames, i.e. n speech frames will be produced from an input file with a length between [(n-1)*20ms+1 sample; n*20ms]. The files produced by the decoder will always have a length of n*20ms.

Input/output audio shall follow configurations as specified in Table 2. Ambisonics components follow the ACN ordering where ACNindex = n2+n+m for real-valued spherical harmonics components of order n and degree m=0, 1, … , where m=[-n,…,n].

Table 3: Audio track configurations

| Audio format (designator) | Number of tracks | Index | Configuration (incl. ordering) | Azimuth Range | Elevation Range |
| --- | --- | --- | --- | --- | --- |
| Mono (M) | 1 | 1 | M | - | - |
| Stereo (ST) | 2 | 1,2 | L, R | - | - |
| Binaural (BIN) | 2 | 1,2 | L, R | - | - |
| Multi-channel 5.1 (MC51) | 6 | 1 | CH_A+030_E+00 | +30 | 0 |
|  |  | 2 | CH_A-030_E+00 | -30 | 0 |
|  |  | 3 | CH_A+000_E+00 | 0 | 0 |
|  |  | 4 | LFE | - | - |
|  |  | 5 | CH_A+110_E+00 | +100 … +120 | 0 … +15 |
|  |  | 6 | CH_A-110_E+00 | -100 … -120 | 0 ... +15 |
| Multi-channel 7.1 (MC71) | 8 | 1 | CH_A+030_E+00 | +30 ... +45 | 0 |
|  |  | 2 | CH_A-030_E+00 | -30 … -45 | 0 |
|  |  | 3 | CH_A+000_E+00 | 0 | 0 |
|  |  | 4 | LFE | - | - |
|  |  | 5 | CH_A+110_E+00 | +85 … +110 | 0 |
|  |  | 6 | CH_A-110_E+00 | -85 … -110 | 0 |
|  |  | 7 | CH_A+135_E+00 | +120 … +150 | 0 |
|  |  | 8 | CH_A-135_E+00 | -120 … -150 | 0 |
| Multi-channel 5.1+4 (MC514) | 10 | 1 | CH_A+030_E+00 | +30 | 0 |
|  |  | 2 | CH_A-030_E+00 | -30 | 0 |
|  |  | 3 | CH_A+000_E+00 | 0 | 0 |
|  |  | 4 | LFE | - | - |
|  |  | 5 | CH_A+110_E+00 | +100 … +120 | 0 … +15 |
|  |  | 6 | CH_A-110_E+00 | -100 … -120 | 0 … +15 |
|  |  | 7 | CH_A+030_E+35 | +30 … +45 | +30 … +55 |
|  |  | 8 | CH_A-030_E+35 | -30 … -45 | +30 … +55 |
|  |  | 9 | CH_A+110_E+35 | +100 … +135 | +30 … +55 |
|  |  | 10 | CH_A-110_E+35 | -100 … -135 | +30 … +55 |
| Multi-channel 7.1+4 (MC714) | 12 | 1 | CH_A+030_E+00 | +30 … +45 | 0 |
|  |  | 2 | CH_A-030_E+00 | -30 … -45 | 0 |
|  |  | 3 | CH_A+000_E+00 | 0 | 0 |
|  |  | 4 | LFE | - | - |
|  |  | 5 | CH_A+135_E+00 | +120 … +150 | 0 |
|  |  | 6 | CH_A-135_E+00 | -120 … -150 | 0 |
|  |  | 7 | CH_A+090_E+00 | +85 … +110 | 0 |
|  |  | 8 | CH_A-090_E+00 | -85 … -110 | 0 |
|  |  | 9 | CH_A+030_E+35 | +30 … +45 | +30 … +55 |
|  |  | 10 | CH_A-030_E+35 | -30 … -45 | +30 … +55 |
|  |  | 11 | CH_A+135_E+35 | +100 … +150 | +30 … +55 |
|  |  | 12 | CH_A-135_E+35 | -100 … -150 | +30 … +55 |
| FOA (SBA1) | 4 | 1…4 | Ambisonics components with ACNindex 0,1,2,3 | - | - |
| HOAO* (SBAO) | O+12 | 1…O+12 | Ambisonics components with ACNindex 0,1, 2,… O+12-1 | - | - |
| Mono objects (OBA) | 1…4 | 1…4 | Object(s) with ID 1…4 | - | - |
| Metadata-assisted spatial audio, mono (MASA1) | 1 | 1 | M | - | - |
| Metadata-assisted spatial audio, stereo (MASA2) | 2 | 1,2 | L, R | - | - |
| Combined mono MASA and OBA | 2...5 | 1..4 2...5 | Object(s) with ID 1…4 M MASA | - - | - - |
| Combined stereo MASA and OBA | 3...6 | 1..4 5,6 | Object(s) with ID 1…4 L, R MASA | - - | - - |
| Combined HOAO* (SBAO) and OBA | 1…4+ O+12 | 1…O+12+1…4 | Object(s) with ID 1…4 Ambisonics components with ACNindex 0,1, 2,… O+12-1 | - | - |

*O = Ambisonics order

For Ambisonics, SN3D normalization is assumed.

# 5.2	Rate switching profile (encoder input)

The encoder program can optionally read in a rate switching profile file which specifies the encoding bitrate for each frame of the input data. The rate switching profile is a binary file, generated by 'gen-rate-profile' tool, which is part of STL 2023, as contained in ITU-T G.191 [7]. The rate switching profile contains 32-bit integer words where each word represents the encoding bitrate for each particular frame. The rate switching profile is recycled if it contains less entries than the total number of frames in the input file.

# 5.3	Bandwidth switching profile (encoder input)

The encoder program can optionally read in a bandwidth switching profile, which specifies the encoding bandwidth for each frame of speech processed. The file is a text file where each line contains "nb_frames B". B specifies the signal bandwidth that is one of the supported bandwidths. For IVAS operation modes, WB, SWB or FB are supported. For EVS operation modes, NB, WB, SWB and FB are supported. "nb_frames" is an integer number of frames and specifies the duration of activation of the accompanied signal bandwidth B.

# 5.4	Channel-aware configuration file (encoder input and decoder output)

For the EVS operation modes, the encoder program can optionally read in a configuration file which specifies the values of FEC indicator p and FEC offset o, where FEC indicator, p: LO or HI, and FEC offset, o: 2, 3, 5, or 7 in number of frames. Each line of the configuration file contains the values of p and o separated by a space.

The channel-aware configuration file is meant to simulate channel feedback from a receiver to a sender, i.e. the decoder would generate FEC indication and FEC offset values for receiver feedback that correspond to the current transmission channel characteristics, thereby allowing optimization of the transmission by the encoder which applies the FEC offset and FEC indication when in the channel-aware mode.

# 5.5	Object based audio metadata file (encoder/renderer input and decoder output)

For object based audio input, the encoder/renderer can optionally read corresponding metadata files describing the object characteristics. The metadata files for object based audio (per audio object) are files consisting of comma-separated values (CSV). Each line corresponds to 20ms audio at the renderer and consists of:

- Azimuth (floating-point, range [-180°;180°[; mandatory)

- Elevation (floating-point, range [-90°;90°]; mandatory)

- Radius (floating-point, range [0; 15.75]; optional; default: 1.0)

- Spread (floating-point, range [0; 360]; optional; default: 0.0)

- Gain (floating-point, range [0;1]; optional; default: 1.0)

- Yaw (floating-point, range [-180; 180], positive indicates left; optional; default: 0.0)

- Pitch (floating-point, range [-90; 90], positive indicates up; optional; default: 0.0)

- Non-diegetic (floating-point, range [0; 1]; optional; default: 0; if Flag is set to 1, panning gain is specified by azimuth Value between [-90,90], 90 left, -90 right, 0 center)

The columns are in the following order:

Azimuth,Elevation,Radius,Spread,Gain,Yaw,Pitch,Non-diegetic

The metadata reader accepts 1-8 values specified per line. If a value is not specified, the default value is assumed.

# 5.6	Metadata-assisted spatial audio (MASA) metadata file (encoder/renderer input and decoder output)

For MASA audio input, the encoder/renderer reads MASA metadata files. For bitstreams containing MASA audio, the decoder can optionally write MASA metadata files. The Syntax of the MASA metadata files is specified in Annex A.

# 5.7	Parameter bitstream file (encoder output / decoder input)

The files produced by the speech/audio encoder/expected by the speech decoder contain an arbitrary number of frames in the following available formats.

### 5.7.1	ITU-T G.192 compliant format

| SYNC_WORD | DATA_LENGTH | B1 | B2 | … | Bnn |
| --- | --- | --- | --- | --- | --- |

The encoder/decoder support parameter bitstream files according to ITU-T G.192 [8]: Each box corresponds to one Word16 value in the bitstream file, for a total of 2+nn words or 4+2nn bytes per frame, where nn is the number of encoded bits in the frame. Each encoded bit is represented as follows: Bit 0 = 0x007f, Bit 1 = 0x0081. The fields have the following meaning:

- SYNC_WORD: Word to ensure correct frame synchronization between the encoder and the decoder. It is also used to indicate the occurrences of bad frames.

In the encoder output:	(0x6b21)

In the decoder input:	Good frames	(0x6b21)

Bad frames	(0x6b20)

- DATA_LENGTH: Length of the speech data. Codec mode and frame type is extracted in the decoder using this parameter

# 5.8	VoIP parameter bitstream file (decoder input)

| Packet size | Arrival time | RTP header | G.192 format (see 5.7.1) |
| --- | --- | --- | --- |

The fields have the following size and meaning:

- Packet size: 32 bit unsigned integer (= 12 + 2 + DATA_LENGTH).

- Arrival time: 32 bit unsigned integer in ms.

- RTP header: 96 bits (see RFC 3550 [6]), including RTP timestamp and SSRC.

# 5.9	JBM trace file (decoder output)

The decoder can generate a JBM trace file with the –Tracefile switch as a by-product of the decoder operation in case of JBM operation (which is triggered with the –VOIP switch on the decoder side).

The trace file is a CSV file with semi-colon as separator. The trace file starts with one header line that contains the column names in the following order:

rtpSeqNo;rtpTs;rcvTime;playtime;active

For each played out speech frame one entry is written to the trace file. The interval of the playtime values is usually 20ms, but may differ, depending on the JBM operation. Each entry is a line in the trace file that contains values as specified in Table 4.

Table 5: JBM trace file entry format

| Name | Unit | Description |
| --- | --- | --- |
| rtpSeqNo | 1 | RTP sequence number of played out speech frame. -1 if no corresponding RTP packet for the speech frame exists. |
| rtpTs | ms | RTP time stamp of played out speech frame. -1 if no corresponding RTP packet for the speech frame exists |
| rcvTime | ms | Absolute reception time of the RTP packet that corresponds to the speech frame. -1 if no corresponding RTP packet for the speech frame exists. |
| playtime | ms | Absolute play time (i.e. the time at which the PCM data is made available by the decoder). Can be floating-point value. |
| active | 0 or 1 | Binary entry, which is set to 1 for active speech frames (i.e. frames that are neither SID nor NO_DATA) |

# 5.10	HRTF filter file (decoder/renderer input)

# 5.11	Head rotation trajectory file (decoder/renderer input)

# 5.12	Reference rotation/vector file (decoder/renderer input)

# 5.13	External orientation file (decoder/renderer input)

# 5.14	Renderer config file (decoder/renderer input)

# 5.15	Split Rendering bitstream file (decoder/renderer output)

# 5.16	Split Rendering Metadata file (decoder/renderer output)

# 5.17	Scene description file (renderer input)

The renderer can render scenes consisting of one or multiple sources. The scenes can be described using a scene description file (textfile) which is defined according to Table 6:

Table 6: Scene Description File Syntax

| Line no. | Type | Description |
| --- | --- | --- |
| 1 | string | Path to a "multitrack" audio file. This should be a single multichannel wav/pcm file that contains all input audio. For example channels 1-4 can be an FOA scene, channel 5 - an object and channels 6-11 - a 5.1 channel bed. The path given shall be relative to the location of the config file. This path has lower priority than the one given on the command line: it is ignored if the --inputAudio argument to the renderer executable is specified. |
| 2 | integer | Contains number of inputs. An input can either be an Ambisonics scene, an object or a channel bed. This does not correspond the total number of channels in the input audio file. The renderer simultaneously supports: - 1 Ambisonics input - 1 Channel-based input - 1 MASA input - Up to 4 audio objects (ISM) inputs |
| Following lines |  | Definition of each of the inputs. Inputs can be listed in any order. They are not required to be listed in the same order as in the audio file. Ambisonics: Line no. (relative to each input); Type; Description. 1; string; SBA. 2; integer; Index of the first channel of this input in the multitrack file (1-indexed) Ambisonics order. Channel-based: Line no. (relative to each input); Type; Description. 1; string; MC. 2; integer; Index of the first channel of this input in the multitrack file (1-indexed). 3; string; Name of speaker layout (X_Y_Z or CICPx format). MASA: Line no. (relative to each input); Type; Description. 1; string; MASA. 2; integer; Index of the first channel of this input in the multitrack file (1-indexed). 3; integer; Number of transport channels. 4; string; Path to MASA metadata file, see 5.6. The path must be relative to config file location.. Audio Objects, Option 1: Line no. (relative to each input); Type; Description. 1; string; ISM. 2; integer; Index of this input's audio in the multitrack file (1-indexed). 3; string; Path to Object based audio metadata file, see 5.5. The path must be relative to config file location.. or Audio Objects, Option 2: Line no. (relative to each input); Type; Description. 1; string; ISM. 2; integer; Number N of positions defined, followed by N lines in form:. Following N lines:; integer, float, (…); Number of frames for which the defined position is kept, object position according to values defined in Object based audio metadata file, see 5.5.. |

Each input definition may be followed by a list of optional properties in the following format:

<property_key>:<property_value>

Each key-value pair shall be placed on a separate line.

The following key-value pairs are supported:

| Key | Type | Description |
| --- | --- | --- |
| gain_dB | float | Applied gain to input in dB |
| lfe_matrix | string | CSV file containing a LFE panning matrix. File containing a containing a matrix of dimensions [ num_input_lfe x num_output_channels ] with elements specifying linear routing gain. If specified, overrides the output LFE position option and the default behavior which attempts to map input to output LFE channel(s) |
| lfe_gain_dB | float | Applied gain to input LFE in dB. Applicable only to formats containing an LFE. |
| lfe_azi | float | Azimuth defining LFE position on sphere; alternative to lfe_matrics. |
| lfe_ele | float | Elevation defining LFE position on sphere; alternative to lfe_matrics. |

Example configuration:

The following example defines a scene with 4 inputs:

- ISM with trajectory defined in a separate file. Channel 12 in the input file. Apply a gain of 0.5 dB.

- Ambisonics, order 1. Channels 1-4 in the input audio file. Apply -6 dB of gain.

- CICP6 channel bed. Channels 5-10 in the input audio file.

- ISM with 2 defined positions (-90,0) and (90,0). Channel 11 in the input file. The object will start at position (-90,0) and stay there for 5 frames, then move to (90,0) and stay there for 5 frames. This trajectory is looped ver the duration of the input audio file.

./input_audio.wav 4 ISM 12 path/to/IVAS_ISM_metadata.csv gain_dB:0.5 SBA 1 1 gain_dB:-6 MC 5 5_1 ISM 11 2 5,-90,0 5,90,0

Annex A (normative):
Metadata-assisted spatial audio (MASA) format

# A.1	General

This Annex describes the Metadata-assisted spatial audio (MASA) format. The MASA format consists of audio signals and metadata. The audio signals for MASA can be mono or stereo. The metadata is provided according to a structure defined here, and it comprises descriptive metadata and spatial metadata, as defined in the following clauses.

# A.2	MASA format metadata structure

MASA format input to IVAS encoder follows the 20-ms frame size. For each 20-ms audio frame, one corresponding metadata frame is provided. Each metadata frame is structured as illustrated in Figure A.1. The descriptive metadata common for the whole frame is written first. This is followed by the spatial metadata, which consists of four spatial metadata subframes, each corresponding to 5 ms of audio. The structure of the spatial metadata subframes depends on the number of direction parameters in the frame. There are two options for the structure, illustrated in Figure A.2 and Figure A.3 for one direction and two directions, respectively.

Descriptive common metadata

(Table A.1)

Subframe 1

Spatial metadata

(Figure A.2 & A.3)

Subframe 2

Spatial metadata

(Figure A.2 & A.3)

Subframe 3

Spatial metadata

(Figure A.2 & A.3)

Subframe 4

Spatial metadata

(Figure A.2 & A.3)

Spatial metadata

MASA metadata frame

Descriptive common metadata

(Table A.1)

Subframe 1

Spatial metadata

(Figure A.2 & A.3)

Subframe 2

Spatial metadata

(Figure A.2 & A.3)

Subframe 3

Spatial metadata

(Figure A.2 & A.3)

Subframe 4

Spatial metadata

(Figure A.2 & A.3)

Spatial metadata

MASA metadata frame

Figure A.1: Metadata structure for one MASA input signal frame

Direction 1

Spatial metadata

(Table A.2a)

Common

Spatial metadata

(Table A.2b)

Direction 1

Spatial metadata

(Table A.2a)

Common

Spatial metadata

(Table A.2b)

Figure A.2: MASA spatial metadata structure for one subframe with one direction

Direction 1

Spatial metadata

(Table A.2a)

Direction 2

Spatial metadata

(Table A.2a)

Common

Spatial metadata

(Table A.2b)

Direction 1

Spatial metadata

(Table A.2a)

Direction 2

Spatial metadata

(Table A.2a)

Common

Spatial metadata

(Table A.2b)

Figure A.3: MASA spatial metadata structure for one subframe with two directions

Table A.1 presents the MASA descriptive common metadata parameters in order of writing. The definitions and use of the descriptive metadata parameters are described in clause A.4.

Table A.2a and Table A.2b present the MASA spatial metadata parameters dependent and independent of the number of directions, respectively. The definitions and use of the spatial metadata parameters are described in clause A.5.

Table A.1: MASA format descriptive common metadata parameters

| Field | Bits | Description |
| --- | --- | --- |
| Format descriptor | 64 | Defines the MASA format for IVAS. Eight 8-bit ASCII characters: 01001001, 01010110, 01000001, 01010011, 01001101, 01000001, 01010011, 01000001 Values stored as 8 consecutive 8-bit unsigned integers. |
| Channel audio format | 16 | Combined following fields stored in two bytes. Value stored as a single 16-bit unsigned integer. |
| Number of directions | (1) | Number of directions described by the spatial metadata. Each direction is associated with a set of direction dependent spatial metadata. Range of values: [1, 2] |
| Number of channels | (1) | Number of transport channels in the format. Range of values: [1, 2] |
| Source format | (2) | Describes the original format from which MASA was created. |
| (Variable description) | (12) | Further description fields based on the values of ‘Number of channels’ and ‘Source format’ fields. When all bits are not used, zero padding is applied. |

Table A.2a: MASA format spatial metadata parameters (dependent of number of directions)

| Field | Bits | Description |
| --- | --- | --- |
| Direction index | 16 | Direction of arrival of the sound at a time-frequency parameter interval. Spherical representation at about 1-degree accuracy. Range of values: “covers all directions at about 1° accuracy” Values stored as 16-bit unsigned integers. |
| Direct-to-total energy ratio | 8 | Energy ratio for the direction index (i.e., time-frequency subframe). Calculated as energy in direction / total energy. Range of values: [0.0, 1.0] Values stored as 8-bit unsigned integers with uniform spacing of mapped values. |
| Spread coherence | 8 | Spread of energy for the direction index (i.e., time-frequency subframe). Defines the direction to be reproduced as a point source or coherently around the direction. Range of values: [0.0, 1.0] Values stored as 8-bit unsigned integers with uniform spacing of mapped values. |

Table A.2b: MASA format spatial metadata parameters (independent of number of directions)

| Field | Bits | Description |
| --- | --- | --- |
| Diffuse-to-total energy ratio | 8 | Energy ratio of non-directional sound over surrounding directions. Calculated as energy of non-directional sound / total energy. Range of values: [0.0, 1.0] (Parameter is independent of number of directions provided.) Values stored as 8-bit unsigned integers with uniform spacing of mapped values. |
| Surround coherence | 8 | Coherence of the non-directional sound over the surrounding directions. Range of values: [0.0, 1.0] (Parameter is independent of number of directions provided.) Values stored as 8-bit unsigned integers with uniform spacing of mapped values. |
| Remainder-to-total energy ratio | 8 | Energy ratio of the remainder (such as microphone noise) sound energy to fulfil requirement that sum of energy ratios is 1. Calculated as energy of remainder sound / total energy. Range of values: [0.0, 1.0] (Parameter is independent of number of directions provided.) Values stored as 8-bit unsigned integers with uniform spacing of mapped values. |

# A.3	MASA format time-frequency resolution

The MASA spatial metadata parameters describe the spatial characteristics of the captured spatial sound scene. This parametric representation is based on frequency bands. A certain spatial characteristic thus relates to a frequency band, and a neighbouring frequency band can exhibit a different characteristic. For MASA format, 24 frequency bands are used. Table A.3 presents these frequency bands.

The metadata frame corresponding to 20-ms frame of audio is divided into four subframes of 5 ms each, which allows for higher temporal resolution of the spatial characteristics than offered by the frame size. The parametric representation in each frame therefore consists of 24 frequency bands in 4 time slots giving a total of 96 time-frequency tiles.

When a frame describes the scene using one spatial direction, there are 96 instances of each of the spatial metadata parameters corresponding with the 96 time-frequency tiles. When a frame describes the scene using two spatial directions, there are two values per time-frequency tile for some of the spatial metadata parameters. In this case, there are 192 instances of those spatial metadata parameters in one metadata frame.

Table A.3. MASA spatial metadata frequency bands

| Band | LF (Hz) | HF (Hz) | BW (Hz) |
| --- | --- | --- | --- |
| 1 | 0 | 400 | 400 |
| 2 | 400 | 800 | 400 |
| 3 | 800 | 1200 | 400 |
| 4 | 1200 | 1600 | 400 |
| 5 | 1600 | 2000 | 400 |
| 6 | 2000 | 2400 | 400 |
| 7 | 2400 | 2800 | 400 |
| 8 | 2800 | 3200 | 400 |
| 9 | 3200 | 3600 | 400 |
| 10 | 3600 | 4000 | 400 |
| 11 | 4000 | 4400 | 400 |
| 12 | 4400 | 4800 | 400 |
| 13 | 4800 | 5200 | 400 |
| 14 | 5200 | 5600 | 400 |
| 15 | 5600 | 6000 | 400 |
| 16 | 6000 | 6400 | 400 |
| 17 | 6400 | 6800 | 400 |
| 18 | 6800 | 7200 | 400 |
| 19 | 7200 | 7600 | 400 |
| 20 | 7600 | 8000 | 400 |
| 21 | 8000 | 10000 | 2000 |
| 22 | 10000 | 12000 | 2000 |
| 23 | 12000 | 16000 | 4000 |
| 24 | 16000 | 24000 | 8000 |

# A.4	MASA descriptive metadata parameters

The MASA descriptive metadata is provided once per frame. It includes information for correctly reading the metadata frame and information relating to creation of the current MASA format signal and its transport audio signals that can be used to assist encoding or rendering of the spatial audio.

The parameter fields of Table A.1 as defined as follows:

Format descriptor (64 bits)

The unique format descriptor code is provided at the beginning of every MASA format metadata frame. It specifies MASA format for the IVAS codec.

| Required bit value | Decoded value | Additional description |
| --- | --- | --- |
| 01001001, 01010110, 01000001, 01010011, 01001101, 01000001, 01010011, 01000001 | “IVASMASA” | Unique format descriptor |

Channel audio format (16 bits as specified below)

Two bytes providing the following individual fields:

- Number of directions

- Number of channels

- Source format

and a variable 12-bit description configured based on ‘Number of channels’ and ‘Source format’.

Number of directions (1 bit)

This parameter field indicates how many directions are described in current MASA format frame. Size of the metadata associated with the current frame depends on the number of directions.

| Bit value | Decoded value | Additional description |
| --- | --- | --- |
| 0 | 1 direction | - |
| 1 | 2 directions | - |

Number of channels (1 bit)

This parameter field indicates how many transport channels are used for the MASA format. This parameter is required by the codec or renderer in some form to read the correct number of channels. Some additional channel format descriptors further depend on the number of channels.

| Bit value | Decoded value | Additional description |
| --- | --- | --- |
| 0 | 1 channel | - |
| 1 | 2 channels | - |

Source format (2 bits)

This parameter field describes the format of source signals that were used to form the MASA format input file/stream. This parameter provides additional information that can benefit encoding, decoding, and/or rendering. First bit value (00) is the default value.

| Bit value | Decoded value | Additional description |
| --- | --- | --- |
| 00 | Default/Other | Audio originates from unknown format(s) including mixed sources |
| 01 | Microphone grid | Audio originates from various (irregular) microphone grids (e.g., smartphones or other UEs) |
| 10 | Channel-based | Audio originates from premixed channel-based audio (e.g., 5.1) |
| 11 | Ambisonics | Audio originates from Ambisonics format |

Variable description (12 bits including zero padding)

Based on the values of the ‘Number of channels’ bit and ‘Source format’ bits, the variable description is configured to provide up to three additional fields to further describe the source format or transport channels. This information can guide, e.g., metadata encoding and rendering. The following presents the possible field combinations and their definitions.

Source format == 00 (Default/Other)

If number of channels is 1 (bit value 0), no additional metadata is specified. Instead, 12-bit zero padding is applied.

If number of channels is 2 (bit value 1), following additional fields are configured in order:

- Transport definition field (3 bits). This field describes the configuration of the two transport channels. The possible bit values and corresponding configurations are provided in Table A.4.

- Channel angle field (3 bits). This field describes symmetric angle positions for transport signals with directivity patterns. In this notation, 0° corresponds to the front. The bit values and corresponding configuration are defined in Table A.5.

- Channel distance field (6 bits). The bit values and corresponding configuration are defined in Table A.6.

Table A.4: Transport definition field for Source formats: Default/Other and Microphone grid

| Bit value | Decoded value | Additional description |
| --- | --- | --- |
| 000 | Unknown/Other | Default |
| 001 | Omni | - |
| 010 | Subcardioid | - |
| 011 | Cardioid | - |
| 100 | Supercardioid | - |
| 101 | Hypercardioid | - |
| 110 | Dipole | - |
| 111 | Binaural | - |

Table A.5: Channel angles for directive patterns for Source formats: Default/Other and Microphone grid

| Bit value | Decoded value | Additional description |
| --- | --- | --- |
| 000 | Unspecified | Default |
| 001 | ±90 deg. | - |
| 010 | ±70 deg. | XY stereo |
| 011 | ±55 deg. | XY stereo, ORTF stereo |
| 100 | ±45 deg. | NOS stereo, XY stereo, Blumlein pair |
| 101 | ±30 deg. | - |
| 110 | ±0 deg. | AB stereo. Needs spacing for stereo image. |
| 111 | Reserved | - |

Note: If Transport definition value is “Unknown”, “Omni”, or “Binaural”, value 000 is used.

The channel distance parameter is defined with a few predefined values and the distance values between 0.01 m and 1 m are calculated as an equal multiplicative interval such that there are 60 values from 0.01 m to 1 m. The equation for this is given as:

ddec=59100B-3100

where ddec is the decoded distance value and B is the bit value as an integer value, i.e., B=3, …, 62. The result is in meters.

Table A.6: Channel distance for Source formats: Default/Other and Microphone grid

| Bit value | Decoded value | Additional description |
| --- | --- | --- |
| 000000 | Unspecified | Distance is not specified, or it is unknown |
| 000001 | 0 m / coincident | No distance between microphones, i.e., they are coincident |
| 000010 | < 0.01 m | Distances smaller than 0.01 m |
| 000011 | 0.01 m | (Distances formed with equation above) |
| … | … | (Distances formed with equation above) |
| 111110 | 1 m | (Distances formed with equation above) |
| 111111 | > 1 m | Distances larger than 1 m |

Source format == 01 (Microphone grid)

If number of channels is 1 (bit value 0), no additional metadata is specified. Instead, 12-bit zero padding is applied.

If number of channels is 2 (bit value 1), following additional fields are configured in order:

- Transport definition field (3 bits). This field describes the configuration of the two transport channels. The possible bit values and corresponding configurations are provided in Table A.4.

- Channel angle field (3 bits). This field describes symmetric angle positions for transport signals with directivity patterns. In this notation, 0° corresponds to the front. The bit values and corresponding configuration are defined in Table A.5.

- Channel distance field (6 bits). The bit values and corresponding configuration are defined in Table A.6.

The field definitions used for Microphone grid source format and Default/Other source format are the same. Differentiation is based on Source format parameter itself.

Source format == 10 (Channel-based)

For premixed content, the original layout can be provided. In addition to common CICP layouts relevant for IVAS, two generic options (3D and 2D) are available. The description of the bit values is provided in Table A.7. The transport signals with this source format are assumed to be a mono (1 channels) or left-right stereo (2 channels) downmix of the multi-channel signals, and thus the number of channels can be 1 or 2 (bit values 0 or 1).

In addition to the 3-bit Channel layout field, 9 bits of zero padding is applied to complete the 12-bit variable description.

Table A.7: Channel layout field for the channel-based source format

| Bit value | Decoded value | Additional description |
| --- | --- | --- |
| 000 | Unknown/Other | Unknown layout or other (3D) layout. Default option. |
| 001 | Other planar | Other 2D layout |
| 010 | 2.0 | CICP2 positions, ITU order |
| 011 | 5.1 | CICP6 positions, ITU order |
| 100 | 5.1+2 | CICP14 positions azimuth, 35° elevation, ITU order |
| 101 | 5.1+4 | CICP16 positions azimuth, 35° elevation, ITU order |
| 110 | 7.1 | CICP12 positions, ITU order |
| 111 | 7.1+4 | CICP19 positions azimuth, 35° elevation, ITU order |

Note 1: ITU channel order is given in ISO/IEC 23008-3:2015 [9], Table 95.
Note 2: Azimuth positions are given in ISO/IEC 23091-3:2018 [10], Table 3.

Source format == 11 (Ambisonics)

If number of channels is 1 (bit value 0), no additional metadata is specified. Instead, 12-bit zero padding is applied.

If number of channels is 2 (bit value 1), following two additional fields are configured in order:

- Transport definition field (3 bits). This describes the configuration of the two transport channels. The possible bit values and corresponding configurations are provided in Table A.4. However, bit values 001 (omni) and 111 (binaural) are not allowed and are interpreted as bit value 000.

- Channel angle field (3 bits). Describes symmetric angle positions for transports signals with directive patterns. In this notation, 0° corresponds to the front. This is defined in Table A.5.

- In addition, 6 bits of zero padding is applied to complete the 12-bit variable description.

For Ambisonics-based transport signals, transport channels are considered coincident, and there is therefore no ‘Channel distance’ field specified.

# A.5	MASA spatial metadata parameters

The MASA spatial metadata describes the spatial audio characteristics corresponding to the one or two transport audio signals. Thus, the spatial audio scene can be rendered for listening based on the combination of the transport audio signals and the spatial metadata.

The MASA spatial metadata is provided once per subframe in each frame following the time-frequency resolution presented in clause A.3. Spatial metadata for each subframe contains one or two first sets of parameters depending on the number of directions (as defined by the corresponding metadata field in descriptive metadata, clause A.4) and one second set of parameters that does not depend on the number of directions. As shown in Figure A.2 and Figure A.3, the parameters corresponding to Table A.2a are written first in the stream, followed by the parameters corresponding to Table A.2b.

The definitions and use of the MASA spatial metadata parameters are described in order in the following.

Direction index: Spatial direction(s)

Spatial directions represent the directional energy flows in the sound scene. Each spatial direction together with corresponding direct-to-total energy ratio describes how much of the total energy for each time-frequency tile is coming from that specific direction. In general, this parameter can also be thought of as the direction of arrival (DOA).

There can be one or two spatial directions for each time-frequency tile in the input metadata. Each spatial direction is represented using a 16-bit direction index. This is an efficient representation of directions as points of a spherical grid with an accuracy of about 1 degree in any arbitrary direction.

The direction indexing corresponds to the function for transforming the audio direction angular values (azimuth ϕ and elevation θ) into an index, and the inverse function for transforming the index into the audio direction angular values.

Each pair of values containing the elevation and the azimuth is first quantized on a spatial spherical grid of points and the index of the corresponding point is constructed. The structure of the spherical grid is defined first, followed by the quantization function and lastly the index formation followed by the corresponding de-indexing function.

The spherical grid is defined as a succession of horizontal circles of points. The circles are distributed on the sphere, and they correspond to several elevation values. The indexing functions make the connection between the angles (elevation and azimuth) corresponding to each of these points on the grid and a 16-bit index.

The spherical grid is on a sphere of unitary radius that is defined by the following elements:

- The elevation values are equidistant between -90 and +90 degrees; the value 0 is represented and corresponds to the circle situated on the equator. The values are symmetrical with respect to the origin. The number of positive elevation values is Nθ=122.

- For each elevation value there are several equally spaced azimuth values. One point on the grid is given by the elevation and the azimuth value. The number n(i) of azimuth values is calculated as follows:

  - on the equator of the spherical grid (θ=0) is it set to

n1=430

- there is one point at each of the poles (θ=±90 degrees)

nNθ=1

  - the function calculating the number of points ni on the grid for other elevation indices, i=2,…,Nθ-1, uses the following definition: 
ni=cumNi-cumNi-12

with cumN1=0 and

cumNi=2  roundi216-4322sin(i-12)δ-sinδ2sin(Nθ-32)δ-sinδ2

where δ is the uniform quantization step for i=1,…,Nθ-1,  2 roundix/2 is a rounding function to the nearest even integer (above x for  i=2, closest for i>2). The term cumNi gives the cumulative cardinality (i.e., cumulative number of points in the spherical grid) in a spherical zone going from the first non-zero elevation value to the i-th elevation value. This cumulative cardinality is derived from the relative area on the spherical surface, assuming a (near) uniform point distribution of the remaining number of points 216-432 (let alone the equator and poles).

- The azimuth values start from the front direction and are in trigonometrical order from 0 to 2π.

- The quantized azimuth values for odd values of i are equally spaced and start at 0.

- The quantized azimuth values for even values of i are equally spaced and start at πn(i).

- There is a same number of quantized azimuth values for same absolute value elevation codewords.

The quantization in the spherical grid is done as follows:

- The elevation value is quantized in the uniform scalar quantizer to the two closest values θ1, θ2

- The azimuth value is quantized in the azimuth scalar quantizers corresponding to the elevation values θ1, θ2

- The distance on the sphere is calculated between the input elevation azimuth pair and each of the quantized pairs θ1,ϕ1, (θ2,ϕ2)

di=-(sinθsinθi+ cosθicos(ϕ-ϕi)), i=1:2

- The pair with lower distance is chosen as the quantized direction.

The resulting quantized direction index is obtained by enumerating the points on the spherical grid by starting with the points for null elevation first, then the points corresponding to the smallest positive elevation codeword, the points corresponding to the first negative elevation codeword, followed by the points on the following positive elevation codeword and so on.

Direct-to-total energy ratio(s)

Direct-to-total energy ratios work together with spatial directions as described above. Each direct-to-total energy ratio corresponds to a specific spatial direction and describes how much of the energy comes from that specific spatial direction compared to the total energy.

Spread coherence

Spread coherence is a parameter that describes the directional energy flow further. It represents situations where coherent directional sound energy is coming from multiple directions at the same time. This is represented with a single spread coherence parameter that describes how the sound should be synthesized.

In synthesis, this parameter should be used such that value 0 means that the sound is synthesized to single direction as directed by the spatial direction, value 0.5 means that the sound is synthesized to the spatial direction and two surrounding directions as coherent, and 1 means that the sound is synthesized to two surrounding directions around the spatial direction.

Diffuse-to-total energy

Diffuse-to-total energy ratio represents non-directional energy flow in the sound scene. This is a complement to the direct-to-total energy ratios and in an ideal capture with no undesired signal (or synthesized sound scene), the diffuse-to-total ratio value is always

rdiff= 1-∑rdir.

Surround coherence

Surround coherence is a parameter that describes the non-directional energy flow. It represents how much of the non-directional energy should be presented as coherent reproduction instead of decorrelated reproduction.

Remainder-to-total energy ratio

Remainder-to-total represents all the energy that does not “belong” to the captured sound scene based on the used model. This includes possible microphone noise and other capture artefacts that have not been removed from the signal in pre-processing. This means that by considering the direct-to-total energy ratio, the diffuse-to-total energy ratio, and the remainder-to-total energy we end up with a complete energy ratio model of

mMrdirm+rdiff+rrem=1.

when there is any remainder energy present. Otherwise, the energy ratio equation defined for diffuse-to-total energy ratio can be followed.

Annex <B> (informative):
<Informative annex for a Technical Specification>

Informative annexes may appear in both Technical Specifications and Technical Reports. Use style "Heading 8" for use in TSs.

Informative annexes shall not contain requirements for the implementation of the Technical Specification.

# B.1	Heading levels in an annex

Heading levels within an annex are used as in the main document, but for Heading level selection, the "A.", "B.", etc. are ignored. e.g. B.1.2 is formatted using Heading 2 style.

Annex <C>:
<Informative annex title for a Technical Report>

Informative annexes in Technical Reports do not use "(informative") in the title, since all annexes in TRs are informative. Use style "Heading 9" in TRs.

Annex <D> (informative):
Bibliography

Use style "Heading 8" in TSs and "Heading 9" in TRs. Do not use "informative" in the title in TRs.

The Bibliography is optional. If it exists, it shall follow the last technical annex in the document.

The following material, though not specifically referenced in the body of the present document (or not publicly available), gives supporting information.

Bibliography format

<Publication>: "<Title>".

Annex <E> (informative):
Index

Use style "Heading 8" in TSs and "Heading 9" in TRs. Do not use "informative" in the title in TRs.

The Index is optional. If it exists, it shall immediately precede the Changes history annex.

Generate the index using MS Word's index field feature.

Annex <F> (informative):
Change history

Use style "Heading 8" in TSs and "Heading 9" in TRs. Do not use "informative" in the title in TRs.

This is the last annex for TS/TSs which details the change history using the following table.
This table is to be used for recording progress during the WG drafting process till TSG approval of this TS/TR.
For TRs under change control, use one line per approved Change Request
Date: use format YYYY-MM
CR: four digits, leading zeros as necessary
Rev: blank, or number (max two digits)
Cat: use one of the letters A, B, C, D, F
Subject/Comment: for TSs under change control, include full text of the subject field of the Change Request cover
New vers: use format [n]n.[n]n.[n]n

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |

Change history of this template:

| 2001-07 | Copyright date changed to 2001; space character added before TTC in copyright notification; space character before first reference deleted. | 1.3.3 |
| --- | --- | --- |
| 2002-01 | Copyright date changed to 2002. | 1.3.4 |
| 2002-07 | Extra Releases added to title area. | 1.3.5 |
| 2002-12 | "TM" added to 3GPP logo. | 1.3.6 |
| 2003-02 | Copyright date changed to 2003. | 1.3.7 |
| 2003-12 | Copyright date changed to 2004. Chinese OP changed from CWTS to CCSA | 14.0 |
| 2004-04 | North American OP changed from T1 to ATIS | 1.5.0 |
| 2005-11 | Stock text of clause 3 includes reference to 21.905. | 1.6.0 |
| 2005-11 | Caters for new TSG structure. Minor corrections. | 1.6.1 |
| 2006-01 | Revision marks removed. | 1.6.2 |
| 2008-11 | LTE logo line added, © date changed to 2008, guidance on keywords modified; acknowledgement of trade marks; sundry editorial corrections and cosmetic improvements | 1.7.0 |
| 2010-02 | 3GPP logo changed for cleaner version, with tag line; LTE-Advanced logo line added; © date changed to 2010; editorial change to cover page footnote text; trade marks acknowledgement text modified; additional Releases added on cover page; proforma copyright release text block modified | 1.8.0 |
| 2010-02 | Smaller 3GPP logo file used. | 1.8.1 |
| 2010-07 | Guidance note concerning use of LTE-Advanced logo added. | 1.8.2 |
| 2011-04-01 | Guidance of use of logos on cover page modified; copyright year modified. | 1.8.3 |
| 2013-05-15 | Changed File Properties to MCC macro default. Removed R99, added Rel-12/13. Modified Copyright year. Guidance on annex X Change history. | 1.8.4 |
| 2014-10-27 | Updated Release selection on cover. In clause 3, added "3GPP" to TR 21.905. | 1.8.5 |
| 2015-01-06 | New Organizational Partner TSDSI added to copyright block. Old Releases removed. | 1.9.0 |
| 2015-12-03 | Provision for LTE Advanced Pro logo Update copyright year to 2016 | 1.10.0 |
| 2016-03-08 | Standarization of the layout of the Change History table in the last annex.(Unreleased) | 1.11.0 |
| 2016-06-15 | Minor adjustment to Change History table heading | 1.11.1 |
| 2017-03-13 | Adds option for 5G logo on cover | 1.12.0 |
| 2017-05-03 | Smaller 5G logo to reduce file size | 1.12.1 |
| 2019-02-25 | Replacement of frames on cover pages by in-line text. Clarification of help text on when to use 5G logo. Removal of defunct keywords frame on page 2. Add Rel-16, Rel-17 options, eliminated earlier, frozen, Releases (cover page, below title) Corrections to some guidance text, addition of guidance text concerning automatic page headers under Word 2016 ff. Use of modal auxiliary verbs added to Foreword. More explicit guidance on Bibliography and Index annexes. Converted to .docx format. | 1.13.0 |
| 2019-09-12 | Cover page table outline shown dotted for ease of logo selection. (Author to hide outline after logo selection.) User now needs to delete whole table rows instead of individual cells, which proved to be tricky. Change of style for "notes" in the Foreword to normal paragraphs. Insertion of new bookmarks, correction of location of existing bookmarks. (To improve navigation.) Improvements to guidance text. | 1.13.1 |
| 2021-06-18 | Provision for 5G Advanced logo Update copyright year to 2021 Additional guidance on the use of Heading 8/9 in annexes C, D and X. | 1.14.0 |
| 2022-04-01 | Correction of table formatting Update copyright year to 2022 | 1.15.0 |
| 2023-03-14 | Updated copyright year to 2023 Updated URLs from HTTP to HTTPS Updated FTP link to HTTP counterpart Fixed numbering of annexes | 1.16.0 |