| 3GPP TS 26.506 V1.0.0 (2022-12) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; 5G Real-time Media Communication Architecture (Stage 2) (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	7

4	Real-time Media Communication Architecture	7

4.1	Overall architecture for Real-Time Media Communication (RTC)	7

4.2	Functions and entities	8

4.2.1	General	8

4.2.2	Provisioning function	8

4.2.3	Configuration function	8

4.2.4	Media Session Handler (MSH)	8

4.2.5	Network support function	9

4.2.6	Trusted ICE functions	9

4.2.7	Trusted WebRTC signalling function	9

4.2.8	Trusted inter-working function	9

4.2.9	Trusted transport gateway function	9

4.2.10	Trusted media function	9

4.2.11	Trusted application supporting web function	10

4.3	Interfaces	10

4.3.1	RTC-1: Provisioning interface	10

4.3.2	RTC-3: AS to AF interface	10

4.3.3	RTC-4: Media-centric transport interface	10

4.3.4	RTC-5: Control transport interface	11

4.3.5	RTC-6: Client API	11

4.3.6	RTC-7: Client interface	12

4.3.7	RTC-8: Application interface	12

4.4	5G-RTC Architecture extension	12

4.4.1	Introduction	12

4.4.2	Extended 5G-RTC architecture for Edge Computing	12

4.4.2.1	Edge Application Server (EAS)	13

4.4.2.2	Edge Interfaces	13

5	Procedures for Real-time Media Communication	13

5.1	Procedures for Edge Processing	13

5.2	xxx	14

Annex A (informative): Architecture variants for collaboration scenarios	15

A.1	General	15

A.2	Collaboration scenario 1:	15

A.3	Collaboration scenario 2:	17

A.4	Collaboration scenario 3:	18

A.5	Collaboration scenario 4:	18

Annex B (informative): Change history	20

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

This clause is optional. If it exists, it shall be the second unnumbered clause.

# 1	Scope

The present document specifies the architecture for real-time media communication. To support MNO and third-party services for real-time media, it is specified the essential functionalities and interfaces. The primary scope of this Technical Specification is the documentation of the following aspects:

-	A real-time media communication architecture mapped to the 5GS architecture and any SA2 stage 2 architecture additions, with relevant core building blocks, reference point, and interfaces to support modern operator and third-party media services, based on the 5GMS architecture

-	Provide all relevant reference points and interfaces to support different collaboration scenarios between 5G System operator and third-party media communication service provider, including but not limited to an AR media communication service provider.

-	Call flows and procedures for different real-time communication service types,

-	Specify support for AR relevant functionalities such as split-rendering or spatial computing on top of a 5G System based on this architecture

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

Abbreviation format (EW)

<ABBREVIATION>	<Expansion>

# 4	Real-time Media Communication Architecture

## 4.1	Overall architecture for Real-Time Media Communication (RTC)

Real-Time media Communication over 5G system (5G-RTC) in the context of this specification is defined as the delivery of delay-sensitive media from one peer to another with support of 5G network. AR conversational service described in TR 26.998 is a typical use cases for 5G-RTC, which enables end-users to directly communicate real-time media including AR/MR media contents as specified in TS 26.119 [x].As identified in clause 8.4 of TR 26.998, there may be different options to enable such AR conversational service, for example re-use of parts of MTSI such as the IMS data channel or 5G Media Streaming for managed services. In this specification, 5G-RTC architecture provides the core functions and entities to support WebRTC framework over 5G system. The WebRTC framework is a subset of WebRTC and is limited to a protocol stack and its implementation, excluding media codecs and other media processing functions defined in W3C and IETF.

Editor’s NOTE:	Text above may need to be further elaborated.

The overall 5G-RTC architecture is shown in Figure 4.1-1 as below.

Figure 4.1-1: 5G-RTC General Architecture

Editor’s NOTE:	Client architecture as well as the associated text in clause 4.2 should be updated based on the discussion during SA4#121

## 4.2	Functions and entities

### 4.2.1	General

This clause defines minimal and essential functions and extra functions and entities may appear in some cases. The definitions of extra functions and entities are specified in TS 26.113 [y] and TR 26.930 [z].

### 4.2.2	Provisioning function

The provisioning function may enable an application provider to perform provisioning of the following functionalities:

-	QoS support provisioning for WebRTC sessions

-	Charging provisioning for WebRTC sessions

-	Collection of consumption and QoE metrics data provisioning related to WebRTC sessions

-	Offering ICE functionality provisioning such as STUN and TURN servers

-	Offering WebRTC signalling servers provisioning, potentially with interoperability to other signalling servers.

The provisioning function may not be relevant to all collaboration scenarios and some of the 5G support functionality may be offered without application provider provisioning.

NOTE:	The integration/collocation of this 5G-RTC AF and WebRTC signalling server is possible. Co-located WebRTC signalling server is able to act as a 5G-RTC AF which is accessible to 5GC, and replace some of this 5G-RTC AF’s interfaces and APIs with WebRTC signalling. For example, interfaces and APIs between this 5G-RTC AF and UE will be replaced to avoid concurrent/redundant requests from UE.

### 4.2.3	Configuration function

The configuration function stores WebRTC-related configuration information and makes them accessible to the UE. It stores information and recommendations to operate network-assisted WebRTC sessions over 5G system.

The configuration information may consist of static information such as the following:

-	Recommendations for media configurations

-	Configurations of STUN and TURN server locations

-	Configuration about consumption and QoE reporting

-	Discovery information for WebRTC signalling and data channel servers and their capabilities in static and/or dynamic way.

NOTE:	The integration/collocation of this 5G-RTC AF and WebRTC signalling server is possible. Co-located WebRTC signalling server is able to act as a 5G-RTC AF which is accessible to 5GC, and replace some of this 5G-RTC AF’s interfaces and APIs with WebRTC signalling. For example, interfaces and APIs between this 5G-RTC AF and UE will be replaced to avoid concurrent/redundant requests from UE.

### 4.2.4	Media Session Handler (MSH)

The MSH is an entity running on the UE, which assists with the 5G integration of the WebRTC application. It exchanges, on behalf of the application, information about the WebRTC sessions with the network.

The MSH receives information about a new WebRTC session from the application. It relays the information to the Network Support Function. It also receives events and other network information about the WebRTC session from the Network Support Function, which it may relay to the application.

In addition, one of subfunction in MSH is the metric collection and reporting. It executes the collection of QoS and QoE metrics measurements from the WebRTC Framework and the WebRTC application and sends metrics reports to the 5G-RTC AF for the purpose of metrics analysis or to enable potential transport optimizations by the network.

### 4.2.5	Network support function

The support functionality includes the following:

-	Network Support Function receives information from the UE and/or other ASs about a WebRTC session and its state

-	Network Support Function requests the network that QoS should be allocated (or satisfied) for a starting or modified session

-	Network Support Function receives notification from the network about changes to the QoS allocation for the ongoing WebRTC session

-	Network Support Function exchanges information about the WebRTC session with the trusted STUN/TURN/Signalling Server, e.g. to identify a WebRTC session and associate it with a QoS template.

NOTE:	The integration/collocation of this 5G-RTC AF and WebRTC signalling server is possible. Co-located WebRTC signalling server is able to act as a 5G-RTC AF which is accessible to 5GC, and replace some of this 5G-RTC AF’s interfaces and APIs with WebRTC signalling. For example, interfaces and APIs between this 5G-RTC AF and UE will be replaced to avoid concurrent/redundant requests from UE.

### 4.2.6	Trusted ICE functions

The MNO may offer trusted ICE functions to the WebRTC application to be used during the WebRTC ICE gathering phase. These functions may be STUN and TURN servers that facilitate NAT and firewall traversal.

The MNO-operated trusted ICE functions may assist with the 5G integration of the WebRTC application. This could be done by triggering network assistance to starting or ongoing WebRTC sessions.

### 4.2.7	Trusted WebRTC signalling function

The trusted WebRTC signalling function is used to setup and manage MNO-operated WebRTC applications. They offer a standardized signalling protocol for the session setup to both parties of the WebRTC session. The WebRTC signalling function will handle the offer/answer exchange and will have access to the SDP in both directions.

The WebRTC signalling function may use that knowledge to offer network assistance and other 5G features to the endpoints of the WebRTC session.

The WebRTC signalling function manages media flow sessions in both uplink and downlink directions.

### 4.2.8	Trusted inter-working function

This function provides inter-working functionality to enable MNO-facilitated WebRTC sessions that involve endpoints across different MNOs. They may for example provide cross-network signalling functionality to allow WebRTC signalling server that are hosted in different networks to communicate, in order to establish and manage the WebRTC sessions.

### 4.2.9	Trusted transport gateway function

A transport gateway function may be offered by the MNO to support cross-operator WebRTC sessions. It may offer the border control function for user plane (e.g., topology hiding, IPv4-IPv6 translation) as a gateway, which is located at the network boundary where different operators or third-party network connects. It works under the control of the trusted inter-working function.

Note:	Detailed functionality is specified in TR 26.930 [z].

### 4.2.10	Trusted media function

A media server may be offered by the MNO to support WebRTC sessions. It may offer a wide range of functionality such as:

-	a content server that serves content to the WebRTC application, e.g. through a data channel

-	media processing functionality: may be used by the WebRTC application as a relay that performs some media processing function such as transcoding, recording, 3D reconstruction, etc.

-	scene composition functionality: the server may compose a 3D scene and distribute it to several point-to-point WebRTC sessions

-	Multi-point Control Unit (MCU) functionality: the server may offer multi-party conferencing functionality to merge a number of point-to-point WebRTC sessions

-	Selective Forwarding Unit (SFU) functionality: the server may offer the selection, copy, and forwarding functionality of IP steams produced by multiple WebRTC endpoints (i.e., participants).

-	Maintain uplink and downlink flow context (QoS, remote control and etc.) by interacting with the WebRTC signalling function.

### 4.2.11	Trusted application supporting web function

A web server may be offered by the MNO to support applications by providing web service entry point, authorization/authentication, sharing files, or scheduling conferencing sessions.

## 4.3	Interfaces

Editor’s NOTE:	All context here needs to be updated based on the future inputs/discussions

### 4.3.1	RTC-1: Provisioning interface

The RTC-1 interface allows the Application Provider to provision support for RTC sessions that are offered by it. The provisioning may cover the following aspects:

-	QoS support for WebRTC sessions

-	Charging provisioning for WebRTC sessions

-	Collection of consumption and QoE metrics data related to WebRTC sessions

-	Offering ICE functionality such as STUN and TURN servers

-	Offering WebRTC signalling servers, potentially with interoperability to other signalling servers

The provisioning interface is not relevant to all collaboration scenarios and some of the 5G support functionality may be offered without application provider provisioning.

### 4.3.2	RTC-3: AS to AF interface

The 5G-RTC AS may exchange information regarding the RTC session with the 5G-RTC AF. This information may cover QoS flow information and QoS allocation as well as QoE and consumption reports. The 5G-RTC AF may subscribe to information about the status of the QoS flow, which it may share with the 5G-RTC AS, e.g. in form of bitrate recommendations.

### 4.3.3	RTC-4: Media-centric transport interface

This interface is used to exchange the WebRTC traffic with the other endpoint as well as to exchange signaling information related to the WebRTC session with the trusted application servers.

The traffic includes:

-	Media streams sent over RTP

-	Application data sent over data channel

-	WebRTC Signaling data along with STUN and TURN servers

-	Other application data

RTC-4 may further be grouped into two sub-interfaces as follows.

RTC-4s:

The RTC-4s interface is an interface between the WebRTC framework and the 5G-RTC AS such as WebRTC Signaling server. This interface is used for the exchange of signaling information related to the WebRTC session between two or more WebRTC endpoints using trusted application servers. In some cases where the signaling is not handled by WebRTC framework, the RTC-4s interface is an interface between the native WebRTC applications and the WebRTC Signaling server.

RTC-4m:

This interface is used for transmission of media and other related data between two or more WebRTC endpoints.

The traffic includes

- Media data transmitted over RTP

- Application data transmitted using Data channel

- Media related meta-data transmitted using Data channel

NOTE 1:	The Media Server should maintain the status for both uplink and downlink traffic and a separate interface for supporting downlink and uplink is expected to be defined in this specification.

NOTE 2:	WebRTC-enabled UE should support streaming functions for uplink and downlink traffic. Therefore a new entity in UE may be defined.

### 4.3.4	RTC-5: Control transport interface

The RTC-5 interface is an interface between the Media Session Handler and the 5G-RTC AF. It is used to convey configuration information from the 5G-RTC AF to the MSH and to request support for a starting/ongoing WebRTC session. The configuration information may consist of static information such as the following:

-	Recommendations for media configurations

-	Configurations of STUN and TURN server locations

-	Configuration about consumption and QoE reporting

-	Discovery information for WebRTC signaling and data channel servers and their capabilities

The support functionality includes the following:

-	MSH receives the configuration information

-	MSH informs the 5G-RTC AF about a WebRTC session and its state

-	MSH requests QoS allocation for a starting or modified session

-	MSH receives notification about changes to the QoS allocation for the ongoing WebRTC session

-	MSH receives the updated information about the WebRTC session with the 5G-RTC STUN/TURN/Signaling Server, e.g. to identify a WebRTC session and associate it with a QoS template

The 5G-RTC functionality that offer application functions to the WebRTC application may equally be provided by Application Servers (5G-RTC AS) instead of 5G-RTC AFs. These then use a dedicated interface RTC-3 to request configurations and network support for the ongoing WebRTC sessions from the 5G-RTC AF.

### 4.3.5	RTC-6: Client API

The MSH is a function in the UE that provides access to 5G-RTC support functions to the native WebRTC applications. These functions may be offered on request, i.e., through the RTC-6 interface, or transparently without direct involvement of the application. The MSH may assist indirectly in the ICE negotiation by providing a list of STUN and TURN server candidates that offer 5G-RTC functionality. The MSH also collects QoE metric reports and submits consumption reports. It may also offer media configuration recommendations to the application through RTC-6.

### 4.3.6	RTC-7: Client interface

This interface is similar in functionality to RTC-6. The difference lies in the face that this interface may not be exposed as an API to application developers but may be in form of a direct communication between the MSH and the WebRTC framework. The WebRTC framework hides away all details of the QoS allocation and network support from the application. It autonomously and transparently invokes the functions offered by the MSH to provide support for the RTC session.

### 4.3.7	RTC-8: Application interface

This is a proprietary interface between the application and the application provider, which may be used to exchange configuration information related to the RTC session or the application.

## 4.4	5G-RTC Architecture extension

### 4.4.1	Introduction

This clause defines an architecture that enables a 5G-RTC Application Provider to provision resources in the Edge Data Network (EDN) for an application through the M1 interface.

Media processing in the edge may be achieved in one of two different ways at the application layer:

1.	Client-driven management. 5G-RTC Applications that are aware of the edge processing can directly request an edge resource and discover the Edge Application Server (EAS) that is best suited to serve the application.

2.	Application Provider-driven management. The 5G-RTC AF automatically allocates edge resources for new streaming sessions on behalf of the application using information in the 5G-RTC provisioning session.

-An Edge-enabled 5G=RTC Client leverages the Edge Computing capabilities as defined in TS 23.558.

### 4.4.2	Extended 5G-RTC architecture for Edge Computing

The 5G-RTC architecture can be extended to add support for media processing in the edge. The extended architecture is an integration of the 5G-RTC architecture defined in TS 26.506 with the architecture for enabling Edge Applications defined in TS 23.558. The extended architecture is as shown in Figure 4.4.2-1.

Figure 4.4.2-1: Edge-enabled 5G-RTC architecture

#### 4.4.2.1	Edge Application Server (EAS)

EAS is the application server resident in the EDN, performing edge-based processing for AR functionalities such as split rendering and spatial computing. The Application Client (AC) connects to the EAS in order to avail the services of the application with the benefits of Edge Computing.

It is possible that the server functions of an application are available only as an EAS.

However, it is also possible that certain server functions are available both at the edge and in the cloud as an EAS and an Application Server resident in the cloud.

The EAS can use the 3GPP Core Network capabilities in the following ways, all of which are optional to support:

a)	invoking 3GPP Core Network capabilities via the edge enabler layer through the Edge Enabler Server (EES)

b)	invoking 3GPP Core Network function (e.g., PCF) APIs directly, if it is an entity trusted by the 3GPP Core Network; and

c)	invoking the 3GPP Core Network capabilities through the capability exposure functions, i.e., SCEF/NEF/SCEF+NEF.

The functions of Edge enabler Client (EEC), Edge Enabler Server (EES), Edge Configuration Server (ECS) are as defined in TS 23.558.

#### 4.4.2.2	Edge Interfaces

Based on the extended architecture, the following interfaces are defined for performing edge-based processing for AR functionalities such as split rendering and spatial computing:

1.	A 5G-RTC AF that is edge-enabled shall support EES functionality including:

-	EDGE-1 API for supporting registration and provisioning of EEC functions, and discovery by them of EAS instances.

-	EDGE-3 API towards the EAS function of 5G-RTC AS instances.

-	EDGE-6 API for registering with an ECS function.

-	EDGE-9 API for media session relocation.

2.	A 5G-RTC AS that is edge-enabled shall support EAS functionality including the EDGE-3 API for registration with the EES.

3.	A Media Session Handler that is edge-enabled should support EEC functionality including:

-	Invoking the EES function using the EDGE-1 API.

-	Invoking the ECS function using the EDGE-4 API.

-	EDGE-5 API exposed to the Application Client.

4.	A WebRTC Application that is edge-enabled shall support Application Client functionality and should invoke the ECS function using the EDGE-5 API.

# 5	Procedures for Real-time Media Communication

## 5.1	Procedures for Edge Processing

[Editor’s Note: This clause is corresponding to clause 4.2]

## 5.2	xxx

[Editor’s Note: Detailed procedures will be address per each topic in clause 4.]

Annex A (informative):
Architecture variants for collaboration scenarios

## A.1	General

This clause addresses the derivative architecture for each of the collaboration scenarios. The four collaboration scenarios are summarized below and further details is specified in Annex A.

It specifies the four collaboration scenarios as summarized below based on the location of required functional entities in trusted domain as defined as follows.

-	5G support for OTT WebRTC: in this scenario the WebRTC session runs completely over the top. However, the MNO may offer support in form of QoS allocation, bitrate recommendations, and QoE report collection based on request by the UE.

-	MNO-provided trusted WebRTC functions: in this scenario the MNO offers trusted support functions such as ICE servers to the WebRTC application on the UE.

-	MNO-facilitated WebRTC services: the MNO may host and facilitate WebRTC sessions by providing a trusted WebRTC signaling server, which may also offer 5G network assistance.

-	Inter-operable WebRTC services: collaboration scenario 3 is extended with functions to support MNO to MNO inter-operability.

The list of key functional entities in trusted domain differs from collaboration scenarios as described in Table A.1-1.

Table A.1-1: Mapping of key functions to each collaboration scenarios

| Functions/CS | Collaboration scenario 1 | Collaboration scenario 2 | Collaboration scenario 3 | Collaboration scenario 4 |
| --- | --- | --- | --- | --- |
| Provisioning function | Optional | Optional | Optional | Optional |
| Configuration function | Optional | Required | Optional (maybe fulfilled by WebRTC signalling server) | Optional (maybe fulfilled by WebRTC signalling server) |
| MSH | Required | Optional | Optional | Optional |
| Network support function | Required | Required | Optional (maybe fulfilled by WebRTC signalling server) | Optional (maybe fulfilled by WebRTC signalling server) |
| Trusted ICE function | N/A | Required | Optional | Optional |
| Trusted WebRTC signalling server | N/A | N/A | Required | Required |
| Trusted media server | N/A | Optional | Optional | Optional |

NOTE:	The collaboration scenario 3 may further split depending on the role of MNO, as addressed in TR 26.930.

## A.2	Collaboration scenario 1:

Editor’s NOTE:	Detailed text to be added

Figure A.2-1: Derivative 5G-RTC architecture for collaboration scenario 1

Call flow for collaboration scenario 1 is as follows. Session setup part and QoS request part are separately performed.

Figure A.2-2: Session setup flow in Collaboration Scenario #1

Figure A.2-3: QoS request flow in Collaboration Scenario #1

Requests and responses in the sequence are as follows:

-	1 and 2. The session information is exchanged between the client application and WebRTC signalling server. It can be proprietary when the interoperability between operators is not necessary.

-	3 and 10. The application requests MSH the network assistance with iRTC session information. The response includes the result of the request and may include the bitrate recommendation.

-	4 and 5. MSH requests Network Support AF the service access information for following procedures. The response is the result of the request and may include the service access information.

-	6 and 9. MSH requests Network Support AF the network assistance. The response may include the bitrate recommendation.

-	7 and 8 Network Support AF requests PCF the QoS control through N5. The response is the result of the request success or failure. The QoS flow event subscription may be conducted.

## A.3	Collaboration scenario 2:

Editor’s NOTE:	Detailed text to be added

Figure A.3-1: Derivative 5G-RTC architecture for collaboration scenario 2

Editor’s NOTE: RTC-4m interface is present only when the ICE function contains the TURN server in CS-2.

## A.4	Collaboration scenario 3:

Editor’s NOTE:	Diagram and discussion are under the developments

Figure A.4-1: Derivative 5G-RTC architecture for collaboration scenario 3x

## A.5	Collaboration scenario 4:

Editor’s NOTE:	Diagram and discussion are under the developments

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-08 | SA4#120 |  |  |  |  | Initial draft | 0.1.0 |
| 2022-11 | SA4#121 | S4-221543 |  |  |  | SA4#121 Agreements: S4-221344, S4-221542, S4-221544, S4-221545, S4-221510, S4-221509, S4-221371, S4-221508 | 0.2.0 |
| 2022-11 | SA4#121 | S4-221610 |  |  |  | Minor update in Scope: word “generic” removed | 0.2.1 |
| 2022-12 |  |  |  |  |  | Created by MCC to be presented to TSG for information | 1.0.0 |