Source:	Dolby Sweden AB

Title:	ISAR Track-a: Evaluations of ISAR at operating bitrates 384kbps and 512 kbps

Agenda item:	7.7

Document for:	    Discussion and Agreement

# Summary

This document contains the results from the subjective audio tests performed by Dolby to show the performance of the selected ISAR split rendering feature of IVAS [3] at operating bitrates 384 kbps and 512 kbps (specified in TR 26.865 [2] clauses 6 (Physical Design Constraints)). Presented are the results of tests for input formats SBA-HOA3, MC 7.1.4, ISM 4 and MASA.

Key Acronyms

CuT		Condition under Test

HOA	            Higher Order Ambisonics

IVAS	            Immersive Voice and Audio Services

ISM	            Independent Streams with Metadata

MASA	            Metadata-Assisted Spatial Audio

MC	            Multi Channel

ISAR               Immersive Audio for Split Rendering

LL		Listening Laboratory

MUSHRA	Multi Stimulus test with Hidden Reference and Anchor

BT		Better than

NWT               Not worse than

DoF                Degree of Freedom

# 1. Introduction

Dolby performed headphone listening experiments similar to the BS.1534-1, BS.1534-2, BS.1534-3 and BS.1534-4 defined in ISAR Pdoc on Testing Aspects for Phase/Track 2/a [4]. In these new evaluations the default ISAR configuration was used at bitrates of 512 kbps and 384 kbps. The experiments and corresponding input formats are listed in Table 1.

Table 1: Overview of experiments and associated input formats

| Exp. | Input format |
| --- | --- |
| BS1534-1-LR | SBA-HOA3 |
| BS1534-2-LR | MC 7.1.4 |
| BS1534-3-LR | ISM 4 |
| BS1534-4-LR | MASA |

# 2. Motivation

To test ISAR performance at 384 and 512 kbps. These bitrates are specified in TR 26.865 [2] clauses 6 (Physical Design Constraints) as recommended bitrates for ISAR operation.

# 3. Test material

The test material for the BS.1534 tests was the same as specified for the ISAR track-A solution selection tests, i.e., it was obtained after cross-check of the processing as specified in the ISAR Pdoc on Testing Aspects for Phase/Track 2/a [4]. IVAS and ISAR processing was done based on IVAS floating point code (details given below).

GIT repo: https://forge.3gpp.org/rep/ivas-codec-pc/ivas-codec

commit ID: e11198a7

# 4. Test procedure

The tests were carried out in a blind fashion. The identity of the conditions under test was not shown to the test subjects.

# 5. Listening Environment

The tests were performed in acoustically treated listening rooms at Ittiam Systems that conform to the requirements of ITU-R BS.1534 [5]. High-quality stereo headphones (Sennheiser HD650 or similar) were used for all experiments.

A test user interface in accordance to ITU-R BS.1534 [5] was used. To ensure unbiased testing, the test interface presented the test items and test conditions in pseudo-random order, with a different pseudo-random presentation order used for each test subject.

# 6. Test subjects

Eight expert listeners were used for each test.

The source used experienced, trained listeners who are frequently involved in BS.1534 tests and who are already familiar with the user interface that was used in the tests. It was therefore unnecessary to provide any specific test instructions. In accordance with [4], the test subjects were not involved in developing the split rendering features or the involved codecs in any of the systems under test. The test subjects had no knowledge of the systems under test.

# 7. Post Screening

The source applied post-screening rules to the test results according to ITU-R BS.1534 [5]. It was not necessary to exclude any listener.

# 8. Test conditions

A description of test conditions is given in Table 2.

Table 2:  Description of test conditions

| Condition | Description |
| --- | --- |
| c01 (REF) | # Hidden reference: Native coding system (IVAS@512kbps rendered to post renderer pose) |
| c02 (LP7) | LP7 anchor: Hidden reference, 7Khz LP filtered |
| c03 (0DOF) | # 0-DOF native transcoding reference (IVAS@512kbps binaurally rendered to pre-renderer pose, IVAS stereo coded@256kbps) |
| c04 (CuT1) | 3-DOF system 1 under test (IVAS@512kbps split-rendered with ISAR operating at 512kbps) |
| c05 (CuT2) | 3-DOF system 2 under test (IVAS@512kbps split-rendered with ISAR operating at 384kbps) |

# 9. Results

A summary of the test results is shown below.

- Experiment BS1534-1-LR

The per-condition mean scores are shown in Table 3. Table 4 shows p-values of a single-sided Student’s Dependent Groups t-tests comparing c03 against c04 and c03 against c05 (DF = 95). The p-values were adjusted for multiple comparisons using the Bonferroni method. The conclusion of the test is as follows:

- The 512kbps ISAR condition (c04) is better than (BT) the 0-DOF (c03) condition.

- The 384kbps ISAR condition (c05) is better than (BT) the 0-DOF (c03) condition.

Table 3:  Per-condition mean MUSHRA scores

| Condition | Mean score |
| --- | --- |
| c01 (REF) | 98.34 |
| c02 (LP7) | 40.323 |
| c03 (0DOF) | 76.4375 |
| c04 (CuT1) | 94.74 |
| c05 (CuT2) | 95.073 |

Table 4:  T-test results

| Hypothesis | p-value | Result | Conclusion |
| --- | --- | --- | --- |
| c04 <= c03 | 6.13E-22 | Hypothesis rejected | c04 BT c03 |
| c05 <= c03 | 3.02E-22 | Hypothesis rejected | c05 BT c03 |

Figure 1:  Plot of test results

- Experiment BS1534-2-LR

The per-condition mean scores are shown in Table 5. Table 6 shows p-values of a single-sided Student’s Dependent Groups t-tests comparing c03 against c04 and c03 against c05 (DF = 95). The p-values were adjusted for multiple comparisons using the Bonferroni method. The conclusion of the test is as follows:

- The 512kbps ISAR condition (c04) is better than (BT) the 0-DOF (c03) condition.

- The 384kbps ISAR condition (c05) is better than (BT) the 0-DOF (c03) condition.

Table 5:  Per-condition mean MUSHRA scores

| Condition | Mean score |
| --- | --- |
| c01 (REF) | 98.73 |
| c02 (LP7) | 39.24 |
| c03 (0DOF) | 74.7396 |
| c04 (CuT1) | 93.1875 |
| c05 (CuT2) | 92.09375 |

Table 6:  T-test results

| Hypothesis | p-value | Result | Conclusion |
| --- | --- | --- | --- |
| c04 <= c03 | 7.72E-62 | Hypothesis rejected | c04 BT c03 |
| c05 <= c03 | 9.42E-55 | Hypothesis rejected | c05 BT c03 |

Figure 2:  Plot of test results

- Experiment BS1534-3-LR

The per-condition mean scores are shown in Table 7. Table 8 shows p-values of a single-sided Student’s Dependent Groups t-tests comparing c03 against c04 and c03 against c05 (DF = 95). The p-values were adjusted for multiple comparisons using the Bonferroni method. The conclusion of the test is as follows:

- The 512kbps ISAR condition (c04) is better than (BT) the 0-DOF (c03) condition.

- The 384kbps ISAR condition (c05) is better than (BT) the 0-DOF (c03) condition.

Table 7:  Per-condition mean MUSHRA scores

| Condition | Mean score |
| --- | --- |
| c01 (REF) | 99.04 |
| c02 (LP7) | 40.531 |
| c03 (0DOF) | 76.9375 |
| c04 (CuT1) | 92.03125 |
| c05 (CuT2) | 90.61458333 |

Table 8:  T-test results

| Hypothesis | p-value | Result | Conclusion |
| --- | --- | --- | --- |
| c04 <= c03 | 3.79E-25 | Hypothesis rejected | c04 BT c03 |
| c05 <= c03 | 7.74E-24 | Hypothesis rejected | c05 BT c03 |

Figure 3:  Plot of test results

- Experiment BS1534-4-LR

The per-condition mean scores are shown in Table 9. Table 10 shows p-values of a single-sided Student’s Dependent Groups t-tests comparing c03 against c04 and c03 against c05 (DF = 95). The p-values were adjusted for multiple comparisons using the Bonferroni method. The conclusion of the test is as follows:

- The 512kbps ISAR condition (c04) is better than (BT) the 0-DOF (c03) condition.

- The 384kbps ISAR condition (c05) is better than (BT) the 0-DOF (c03) condition.

Table 9:  Per-condition mean MUSHRA scores

| Condition | Mean score |
| --- | --- |
| c01 (REF) | 98.51 |
| c02 (LP7) | 41.031 |
| c03 (0DOF) | 82.1771 |
| c04 (CuT1) | 95.1875 |
| c05 (CuT2) | 94.07291667 |

Table 10:  T-test results

| Hypothesis | p-value | Result | Conclusion |
| --- | --- | --- | --- |
| c04 <= c03 | 3.07E-39 | Hypothesis rejected | c04 BT c03 |
| c05 <= c03 | 1.13E-36 | Hypothesis rejected | c05 BT c03 |

Figure 4:  Plot of test results

# 10. Conclusion

The ISAR listening tests BS.1534-1-LR, BS.1534-2-LR, BS.1534-3-LR and BS.1534-4-LR were performed in accordance with [4] and [5]. The systems under test obtained scores in the “Excellent” range in all experiments. In all experiments, the systems under test’s mean scores were statistically better than the 0DOF reference at a confidence level above 95%.

It is proposed to document this result as part of the IVAS technical report [6] and the ISAR technical report [7].

References

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]   3GPP TR 26.865: Immersive Audio for Split Rendering Scenarios; Requirements

[3]	S4-240709: "TS26.253CR0003 on Adding ISAR track-a split rendering feature".

[4]	S4-240024: “ISAR Pdoc on Testing Aspects for Phase/Track 2/a, v0.1.4”

[5]	ITU-R BS.1534, “Method for the subjective assessment of intermediate quality level of audio systems, 2015

[6]	3GPP TR 26.997: "IVAS codec performance characterization".

[7]	3GPP TR 26.996: "Immersive Audio for Split Rendering Scenarios; Performance characterization".