| 3GPP TS 28.abc V0.1.0 (2024-10) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Management and orchestration; Signalling traffic monitoring management (Stage 1, stage 2, and stage 3) (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	7

4	Signalling traffic monitoring management capabilities (stage 1)	8

4.1	 Signalling Monitoring lifecycle	8

4.1.1	 Description	8

4.1.2	 Requirements	8

5	Signalling traffic monitoring management operations (stage 2)	8

5.1	Overview	8

5.2	Operations	8

5.3	Parameters	9

6	Signalling traffic monitoring management Information Mode (stage 2)	9

6.1 	Imported and associated information entities	9

6.1.1	Imported information entities and local labels	9

6.1.2	Class diagram	9

6.2	Class definitions	9

6.2.1	<<ClassName>>	9

6.3	Attribute definitions	9

6.3.1	Attribute properties	9

6.4	Common notifications	10

7	Management services for Signalling traffic monitoring management (stage 3)	10

7.1	Reporting format	10

7.2	Reporting content	10

Annex A (informative): Use Cases	10

A.1 	Signalling Monitoring Activation	10

A.2 	Signalling Monitoring Termination	10

A.3 	Signalling Traffic Monitoring Streaming	10

Annex B (informative): Change history	11

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

The present document covers the 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Telecommunication management, signalling traffic monitoring management including stage 1, stage 2 and stage 3, as identified below:

TS 28.abc:	Management and orchestration; Signalling traffic monitoring management (Stage 1, Stage 2, Stage 3).

# 1	Scope

The present document specifies the definitions of signalling traffic monitoring management. This monitoring is performed at network signalling level on 5GC control plane. When the monitoring is enabled, the 5GC network function collects the required decrypted signalling message and forwards it to the configured collection entity.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Editor's note: TBC

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Editor's note: TBC

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

Editor's note: TBC

# 4	Signalling traffic monitoring management capabilities (stage 1)

## 4.1		Signalling Monitoring lifecycle

### 4.1.1		Description

The lifecycle management of the signalling monitoring is to be enabled by the signalling monitor consumer in charge of the signalling traffic monitoring management. The lifecycle management includes activation, streaming of a copy of the signalling traffic and termination of the signalling traffic monitoring.

### 4.1.2		Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-SM-FUN-1 | The signalling monitor producer shall have the capability to allow its authorized consumer to request activating the monitoring of signalling traffic of the selected interface(s) of the selected NF(s). | Signalling Monitoring activation |
| REQ-SM-FUN-2 | The signalling monitor producer shall have the capability to stream the NF signalling traffic to its authorized consumer. | Signalling Traffic Monitoring Streaming |
| REQ-SM-FUN-3 | The signalling monitor producer shall have the capability to allow its authorized consumer to request terminating the monitoring of signalling traffic | Signalling Monitoring termination |
| REQ-SM-FUN-4 | The signalling monitor producer shall have the capability to stream the signalling traffic in a reliable manner | Signalling Traffic Monitoring Streaming |

# 5	Signalling traffic monitoring management operations (stage 2)

## 5.1	Overview

Editor's note: TBC

## 5.2	Operations

Editor's note: OperationName is the name of the operation followed by a qualifier indicating whether the operation is Mandatory (M), Optional (O), Conditional-Mandatory (CM), Conditional-Optional (CO), or SS-Conditional (C).

## 5.3	Parameters

Editor's note: List of parameters to support the operations. Each element contains theParameter Name, Support Qualifier, Documentation and Allowed Values and Properties. Legal Values for the Support Qualifier are: Mandatory (M), Optional (O), Conditional-Mandatory (CM), Conditional-Optional (CO), or SS-Conditional (C).

# 6	Signalling traffic monitoring management Information Mode (stage 2)

## 6.1 	Imported and associated information entities

### 6.1.1	Imported information entities and local labels

Editor Note: TBC

### 6.1.2	Class diagram

Editor Note: TBC

## 6.2	Class definitions

### 6.2.1	<<ClassName>>

Editor Note: TBC

#### 6.2.1.1	Definition

Editor Note: TBC

#### 6.2.1.2	Attributes

Editor Note: TBC

#### 6.2.1.3	Attribute constraints

Editor Note: TBC

#### 6.2.1.4	Notifications

Editor Note: TBC

#### 6.2.1.5	State diagram

Editor Note: TBC

## 6.3	Attribute definitions

### 6.3.1	Attribute properties

Editor Note: TBC

## 6.4	Common notifications

Editor Note: TBC

# 7	Management services for Signalling traffic monitoring management (stage 3)

## 7.1	Reporting format

Editor's note: TBC

## 7.2	Reporting content

Editor's note: TBC

# Annex A (informative):
Use Cases

## A.1 	Signalling Monitoring Activation

External monitoring systems are often used by mobile network operators (MNOs) to track network activity for analysis and troubleshooting purposes, and subsequently to perform diagnosis and analysis of their system. Such monitoring system(s) are fully under the control of the MNOs, and the monitoring is performed at signalling level.

The goal of this use case is to enable the authorized consumer (signalling monitor consumer) to activate the streaming of packets from a specific NF or entity acting on behalf of the  NF (signalling monitor producer). In this activation it will be requested the desired reliability for the streaming of the signalling traffic, along with the targeted interface(s)

## A.2 	Signalling Monitoring Termination

Once the signalling traffic monitoring is not needed anymore it needs to be possible to stop the streaming of signalling traffic. The goal of this use case is to enable the authorized consumer (signalling monitoring consumer) to request a specified NF to terminate the streaming of signalling traffic.

## A.3 	Signalling Traffic Monitoring Streaming

For the signalling traffic to be sent to a signalling monitor consumer it needs to be streamed by the signalling monitor producer. The signalling traffic streams will be transported in a reliable or unreliable way depending on the request at the time of activation.

# Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-10 | SA5#157 | S5-245983 | - | - | - | Initial skeleton (v0.0.0) | V0.0.0 |
| 2024-10 | SA5#157 | S5-245986 |  |  |  | pCR of draft TS28.abc Stage 1 skeleton | V0.1.0 |
| 2024-10 | SA5#157 | S5-245987 |  |  |  | pCR of draft TS28.abc Stage 2 skeleton | V0.1.0 |
| 2024-10 | SA5#157 | S5-245988 |  |  |  | pCR of draft TS28.abc Stage 3 skeleton | V0.1.0 |
| 2024-10 | SA5#157 | S5-245984 |  |  |  | pCR of draft TS28.abc Introduction | V0.1.0 |
| 2024-10 | SA5#157 | S5-245985 |  |  |  | pCR of draft TS28.abc Scope | V0.1.0 |
| 2024-10 | SA5#157 | S5-245989 |  |  |  | pCR of draft TS28.abc Stage 1 requirements | V0.1.0 |