| 3GPP TS 28.572 V0.1.0 (2024-10) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Management and orchestration; Management of planned configurations (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	3

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	6

4	Concept and overview	6

5	Requirements	6

6	Solution description	8

6.1	Planned configurations	8

6.1.1	Definition	8

6.1.1	Format of planned configurations	8

6.1.2	Meta data for planned configurations	9

6.1.3	Control parameters for planned configurations	9

6.1.4	Planned configuration descriptor	9

6.2	Planned configuration groups	9

6.2.1	Definition	9

6.2.2	Planned configuration group members	9

6.2.3	Meta data for planned configuration groups	9

6.2.4	Control data for planned configuration groups	9

6.2.5	Planned configuration group descriptor	10

6.3	Validation of planned configurations	10

6.3.1	Definition	10

6.3.2	Potential problems of planned configurations	10

6.3.3	Validation process	10

6.3.4	Validation of multiple planned configurations	10

6.4	Activation of planned configurations	11

6.4.1	Definition	11

6.5	Conditional activation of planned configurations	11

6.6	Asynchronous updates	11

6.7	Fallback	11

6.8	Planned configuration lifecycle	11

6.9	Managing planned configurations	11

6.10	Managing planned configuration groups	11

6.11	Managing validations	13

6.12	Managing activations	13

6.13	Notifications	14

7	Model	14

Annex A (normative):  Solution sets	14

A.1	RESTful HTTP-based solution set	14

Annex B (informative): Change history	15

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

# 1	Scope

The present document …

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

# 4	Concept and overview

# 5	Requirements

Requirements related to planned configurations

Req-PCG-1: The 3GPP management system shall support creating, reading and deleting planned configurations for managed systems.

Req-PCG-2: The 3GPP management system should support updating planned configurations for managed systems.

Req-PCG-3: The 3GPP management system shall support creating, reading, updating, and deleting meta data for planned configurations.

Requirements related to control parameters for validation and activation of planned configurations

Req-PCC-1: The 3GPP management system shall support a capability allowing a MnS consumer to indicate to a MnS producer if validation and activation shall be processed in best effort or atomic mode.

Requirements related to planned configuration groups

Req-PGG-1: The 3GPP management system should allow MnS consumers to group planned configurations into planned configuration groups.

Req-PGG-2: The 3GPP management system should allow MnS consumers to group planned configuration groups into planned configuration groups.

Req-PGG-3: The 3GPP management system shall support creating, reading and deleting planned configuration groups.

Req-PGG-4: The 3GPP management system should support updating planned configuration groups.

Req-PGG-5: The 3GPP management system shall support creating, reading, updating, and deleting meta data for planned configuration groups.

Requirements related to control parameters for validation and activation of planned configuration groups

Req-PGC-1: The 3GPP management system shall support creating, reading, updating, and deleting control parameters for planned configuration groups.

General plan validation requirements

Req-PVG-1: The 3GPP management system shall support a capability allowing a MnS consumer to request a MnS producer to validate a planned configuration.

Req-PVG-2: The 3GPP management system should support a capability allowing a MnS consumer to retrieve the status of a plan validation.

Req-PVG-3: The 3GPP management system should support a capability allowing a MnS consumer to request a MnS producer to cancel an ongoing plan validation.

Req-PVG-4: The 3GPP management system shall support a capability allowing a MnS consumer to retrieve the result of a plan validation.

Req-PVG-5: The 3GPP management system should allow for multiple sequential validation of the same plan.

Req-PAG-6: The 3GPP management system shall support a capability allowing a MnS consumer to find out if a plan has been previously validated.

Requirements related to the validation of multiple plans

Req-PVM-1: The 3GPP management system should support a capability allowing a MnS consumer to request a MnS producer to validate multiple planned configurations with a single request.

Req-PVM-2: The 3GPP management system shall support a capability allowing a MnS consumer to indicate to a MnS producer the type of relationship between the plans, for the case where multiple plans are requested to be validated with a single request.

Req-PVM-3: The 3GPP management system shall support a capability allowing a MnS consumer to indicate to a MnS producer the behavior if plan conflicts are detected, for the case where multiple plans are requested to be validated with a single request.

General plan activation requirements

Req-PAG-1: The 3GPP management system shall support a capability allowing a MnS consumer to request a MnS producer to activate a planned configuration.

Req-PAG-2: The 3GPP management system should support a capability allowing a MnS consumer to retrieve the status of a plan activation.

Req-PAG-3: The 3GPP management system should support a capability allowing a MnS consumer to request a MnS producer to cancel an ongoing plan activation.

Req-PAG-4: The 3GPP management system shall support a capability allowing a MnS consumer to retrieve the result of a plan activation.

Req-PAG-5: The 3GPP management system should allow for multiple sequential activations of the same plan.

Req-PAG-6: The 3GPP management system shall support a capability allowing a MnS consumer to find out if a plan has been previously activated.

Req-PAG-7: The 3GPP management system shall validate a planned configuration automatically when activation of the planned configuration is requested without prior validation.

Requirements related to control parameters for activation

Req-PAC-1: The 3GPP management system should support a capability allowing a MnS consumer to indicate to a MnS producer if activation should be done with the least possible service impact or within the shortest possible time.

Req-PAC-2: The 3GPP management system should support a capability allowing a MnS consumer to indicate to a MnS producer if fallback shall be enabled.

Requirements related to the activation of multiple plans

Req-PAM-1: The 3GPP management system should support a capability allowing a MnS consumer to request a MnS producer to activate multiple planned configurations with a single request.

Req-PAM-2: The 3GPP management system shall support a capability allowing a MnS consumer to indicate to a MnS producer the type of relationship between the plans, for the case where multiple plans are requested to be activated with a single request.

Req-PAM-3: The 3GPP management system shall support a capability allowing a MnS consumer to indicate to a MnS producer the behavior if plan conflicts are detected, for the case where multiple plans are requested to be activated with a single request.

# 6	Solution description

## 6.1	Planned configurations

### 6.1.1	Definition

The current configuration is the configuration currently used by the managed system. It is represented by (a tree of) configuration data nodes. State data nodes are not included. The format of the configuration data node tree is defined by (stage 2) NRM definitions and (stage 3) schema definitions.

Editor's note: The name of the current configuration is tbc. May also be actual configuration or operational configuration.

Note that the data node tree representing a manged system typically comprises besides configuration data nodes also (read-only) state data nodes, that represent the current state of the managed system. Configuration data nodes and state data nodes can be separated with dedicated subtrees, but they can be also contained in one tree.

A planned configuration is a configuration that can be manipulated without impacting the current configuration of the system. It relates to configuration data nodes and not to state data nodes. It is maintained side-by-side with the current configuration.

### 6.1.1	Format of planned configurations

A planned configuration is represented by a set of operations to be applied to the current configuration. The operations are described by the input parameters of the "changeMOIs" operation specified in TS 28.532, with the addition that each operation in the operation set has a key for identifying the operation.

The following guidelines may ease the processing of operation sets during validation and activation:

- Parent objects should be created before child objects.

- Objects that are referenced should be created before the reference.

### 6.1.2	Meta data for planned configurations

Each planned configuration is described by meta data. This includes the name, the version, and a human readable textual description. Furthermore, it includes the content type of the planned configuration and a pointer to the schema describing the current configuration. The date and time at which information in the planned configuration was modified the last time is provided as well.

### 6.1.3	Control parameters for planned configurations

Control parameters determine how a planned configuration is processed when validated or activated. They can be specified together with the planned configuration, or when validation or activation of the planned configuration is requested.

Together with the planned configuration is specified the control parameter determining whether the operations in the operations set are processed during validation or activation in atomic or best effort mode. In atomic mode, either all operations shall be successfully processed or no operation at all. In the best effort mode, the updates that can be successfully processed are processed, and those that for whatever reason cannot be successfully processed are not processed.

### 6.1.4	Planned configuration descriptor

The planned configuration, its meta data and control parameters are specified in a planned configuration descriptor. Each descriptor has a unique identifier.

## 6.2	Planned configuration groups

### 6.2.1	Definition

A planned configuration group is a set of planned configurations that shall be processed (validated and activated) together.

For example, one plan may create a new cell, and another plan may start the collection of measurements for the new cell. Or one plan that configures one base station and another plan configures another base station, when both base stations need to be consistently configured to enable handovers between them.

### 6.2.2	Planned configuration group members

A planned configuration belonging to a planned configuration group is also called a member of that planned configuration group. A planned configuration can be a member of more than one planned configuration group.

### 6.2.3	Meta data for planned configuration groups

Each planned configuration group is described by meta data. This includes the name, the version, and a human readable textual description. The date and time at which information in the planned configuration group was modified the last time is provided as well.

### 6.2.4	Control data for planned configuration groups

Control data needs to be specified for the validation and activation of planned configuration groups. This can be done in the definition of the planned configuration group or when validating or activating the planned configuration group.

For a validation, the conflict mode is specified. For activation, the conflict mode, the desired service impact and if fallback shall be enabled are specified.

### 6.2.5	Planned configuration group descriptor

The planned configuration group members, its meta data and control parameters are specified in a planned configuration group descriptor. Each descriptor has a unique identifier.

## 6.3	Validation of planned configurations

### 6.3.1	Definition

Planned configurations may have problems and it may be impossible to activate them. Validation is the process to unveil potential problems with planned configurations before activation of the planned configuration.

The validation function has two input arguments: the operations set representing the planned configuration and a copy of the current configuration. The planned configuration is applied to the copy of the current configuration.

### 6.3.2	Potential problems of planned configurations

Validation discovers problems of planned configuration. These problems may be classified as follows:

- Information model problems

  - Problems of the planned configuration itself

  - Problems of the planned configuration in relationship to the current configuration

- Application layer problems

Examples for information model problems of the planned configuration itself:

- Bad data type of an attribute value

- Bad object representation

Examples for problems of the planned configuration in relationship to the current configuration:

- Creation of objects under parents that do not exist

- Deletion of leaf objects that do not exist

- Deletion of objects that are no leaves

- Violation of object cardinality constraints

- Violation of the multiplicity property of an attribute

In general, after applying the planned configuration to the current configuration, the resulting data node tree must comply to all constraints defined in the information model.

Examples for application layer problems:

- An attribute value in the planned configuration is in the value range allowed by the attribute properties but the value is not allowed in the current state of the application.

Problems may result in validation failures.

### 6.3.3	Validation process

### 6.3.4	Validation of multiple planned configurations

## 6.4	Activation of planned configurations

### 6.4.1	Definition

Activation is the process of applying a planned configuration to the current configuration (instead of to a copy of the current configuration as for validation). This process can take a bit of time.

## 6.5	Conditional activation of planned configurations

## 6.6	Asynchronous updates

## 6.7	Fallback

## 6.8	Planned configuration lifecycle

## 6.9	Managing planned configurations

Planned configuration descriptors are created, updated and deleted by MnS consumers on MnS producers.

Multiple planned configuration descriptors can be maintained on a MnS producer. Each planned configuration is identified by an identifier. The identifier is assigned by the MnS producer.

The following table specifies the information elements of the information model for planned configurations.

| Information element name | Description |
| --- | --- |
| planned-conf-id | The identifier of the planned configuration. |
| name | The name of the planned configuration. |
| version | The version of the planned configuration. |
| user-label | The user label of the planned configuration. |
| description | The textual human-readable description of the planned configuration. |
| content-type | The format of the planned configuration. |
| current-conf-schema | The pointer to the schema of the current configuration. |
| last-modified | The date and time at which the planned configuration descriptor was modified the last time. |
| validation-mode | The validation mode for the planned configuration, either "ATOMIC" or "BEST_EFFORT". |
| activation-mode | The activation mode for the planned configuration, either "ATOMIC" or "BEST_EFFORT". |
| planned-conf | The planned configuration itself. |

## 6.10	Managing planned configuration groups

Planned configuration group descriptors are created, updated and deleted by MnS consumers on MnS producers. Planned configurations referred to in a planned configuration group must exist.

Multiple planned configuration group descriptors can be maintained on a MnS producer. Each planned configuration group is identified by an identifier. The identifier is assigned by the MnS producer.

When control parameters are specified in the planned configuration group descriptor and in the validation or activation request, the latter takes precedence.

The following table specifies the information elements of the information model for planned configuration groups.

| Information element name | S | Documentation and Allowed Values | Properties |
| --- | --- | --- | --- |
| plan-group-id | M | Identifier of the planned configuration group. | Type: String multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None |
| name | M | The name of the planned configuration group. | Type: String multiplicity: 0..1 isOrdered: NA isUnique: NA defaultValue: None |
| version | M | The version of the planned configuration group. Format is implementation specific. | Type: String multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None |
| description | M | The textual human-readable description of the planned configuration group. | Type: String multiplicity: 0..1 isOrdered: NA isUnique: NA defaultValue: None |
| plan-ids | M | The identifiers of the planned configurations that are members of the planned configuration group. | Type: String multiplicity: 1..* isOrdered: True isUnique: True defaultValue: None |
| relationship-between-plans | M | The relationship between the planned configurations. allowedValues: - SEQUENTIAL_ORDERED - COMBINED Editor's note: The definition of the values is FFS | Type: ENUM multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None |
| last-modified | M | The date and time at which the planned configuration group descriptor was modified the last time. | Type: DateTime multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None isWritable: False |
| conflict-mode | M | This attribute allows to specify, if conflicting changes in different plans shall result in a failure, or if conflicting changes shall be applied in order. allowedValues: - STOP_ON_CONFLICT - CONTINUE_ON_CONFLICT - ROLLBACK_ON_CONFLICT | Type: ENUM multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None |
| enable-fallback | M | Specifies if the MnS producer shall create a fallback-plan o allow for a later fallback to undo the changes in the curent plan-group. If the element is missing it has the same effect as if it was False. The plan-id of the fallback-plan shall be reurned to the consumer. | Type: Boolean multiplicity: 0..1 isOrdered: NA isUnique: NA defaultValue: False |
| activation-service-impact | M | The service impact mode allowedValues: - LEAST_SERVICE_IMPACT - SHORTEST_TIME | Type: ENUM multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None |

## 6.11	Managing validations

Validation of a planned configuration is requested by MnS consumers and performed by MnS producers. The planned configuration to be validated is identified by its plan identifier. Alternatively, if no planned configuration descriptor has been created for the planned configuration to be validated, the planned configuration may also be specified in the validation request.

Optionally, a MnS consumer can request validation of multiple plans by specifying multiple plan identifiers or a planned configuration group identifier in the validation request.

When only one plan is requested to be validated no control parameters need to be specified. When more than one plan is requested to be validated, the MnS consumer needs to specify the plan dependencies and conflict mode control parameters.

The state of the validation is represented by a specific information element.

The MnS producer makes the validation result available to MnS consumers.

Editor's note: Result details incl. error details and error overview are ffs

The following table specifies the information elements of the information model for validation of planned configurations.

| Information element name | Description |
| --- | --- |
| validation-job-id | The identifier of the validation job. |
| name | The name of the validation job. |
| description. | The textual human-readable description of the validation job. |
| plan-ids | The ordered list of the identifiers of the plans to be validated. |
| planned-configuration-group-id | or, alternatively, the planned configuration group id for the planned configuration group to be validated. |
| planned-conf | or, alternatively, the planned configuration to be activated, if no planned configuration was created earlier. |
| relationship-between-plans | When more than one plan shall be validated, this attribute allows to specify the relationship between the plans validation-to be validated. Allowed values are "INDEPENDENT", "SEQUENTIAL_ORDERED and "COMBINED". Editor's note: It is ffs if an atomic and best effort mode is needed on the level of planned configurations, when multiple planned configurations are validated, or if this is implicitely covered by the three possible values of this attribute.. |
| conflict-mode | When more than one plan shall be validated, this attribute allows to specify for the "ORDERED-DEPENDENT" mode, if conflicting changes in different plans shall result in a failure, or if conflicting changes shall be applied in order. Allowed values are "STOP_ON_CONFLICT" or "CONTINUE_ON_CONFLICT". |
| validation-state | The validation state, either "NOT_STARTED", "RUNNING", "CANCELLED" or "COMPLETED". Editor's note: It is ffs if more detailed state and progress info needs to be added |
| copy-time | The date and time at which a copy of the current validation was taken. |
| validation-start-time | The date and time at which the validation process started. |
| validation-stop-time | The date and time at which the validation process stopped. |
| validation-result | The validation result, either "VALIDATED" or "VALIDATION_FAILED". When "validation-mode" is "BEST_EFFORT" also "VALIDATION_PARTIALLY_FAILED". |
| error-details | In case validation failed or partially failed, error reasons are specified here. |

## 6.12	Managing activations

Activation of a planned configuration is requested by MnS consumers and performed by MnS producers. The planned configuration to be activated is identified in the request by its plan identifier. Alternatively, if no planned configuration descriptor has been created for the planned configuration to be activated, the planned configuration may also be specified in the activation request.

Optionally, a MnS consumer can request activation of multiple plans by specifying multiple plan identifiers or a planned configuration group identifier in the activation request.

Editor's note: Result details incl. error details and error overview are ffs

Editor's note: The precedence between activation parameters specified for the plan and the activation job is FFS.

The following table specifies the information elements of the information model for plan activation.

| Information element name | Description |
| --- | --- |
| activation-job-id | The identifier of the activation job. |
| name | The name of the activation job. |
| description. | The textual human-readable description of the activation job. |
| plan-ids | The ordered list of the identifiers of the plans to be activated. |
| planned-configuration-group-id | or, alternatively, the planned configuration group id for the planned configuration group to be activated. |
| planned-conf | or, alternatively, the planned configuration to be activated, if no planned configuration was created earlier. |
| activation-mode | The activation mode for the planned configuration, either "ATOMIC" or "BEST_EFFORT". Only when "planned-conf" is present. |
| relationship-between plans | When more than one plan shall be activated, this attribute allows to specify the relationship between the plans to be activated. Allowed values are "INDEPENDENT", "SEQUENTIAL_ORDERED and "COMBINED". Editor's note: It is ffs if an atomic and best effort mode is needed on the level of planned configurations, when multiple planned configurations are validated, or if this is implicitely covered by the three possible values of this attribute. |
| conflict-mode | When more than one plan shall be validated, this attribute allows to specify for the "INCREMENTAL_ORDERED" and "COMBINED" modes, if conflicting changes in different plans shall result in a failure, or if conflicting changes shall be applied in order. Allowed values are "STOP_ON_CONFLICT" or "CONTINUE_ON_CONFLICT". |
| enable-fallback | This boolean attribute allows to specify if the MnS producer shall create a plan with the current configuration to allow for a later fallback. The plan-id of the fallback plan shall be returned to the MnS consumer. |
| activation-service-impact | The service impact mode, either "LEAST_SERVICE_IMPACT" or "SHORTEST_TIME". |
| cancel-request | This boolean attribute allows to request to cancel the activation process. |
| activation-state | The activation state, either "NOT_STARTED", "RUNNING", "CANCELLED" or "COMPLETED". Editor's note: It is ffs if more detailed state and progress info needs to be added |
| activation-start-time | The date and time at which the activation process started. |
| activation-stop-time | The date and time at which the activation process stopped. |
| activation-result | The activation result, either "ACTIVATED" or "ACTIVATION_FAILED". When "activation-mode" is "BEST_EFFORT" also "ACTIVATION_PARTIALLY_FAILED". |
| error-details | In case validation failed or partially failed, error reasons are specified here. |

## 6.13	Notifications

# 7	Model

Annex A (normative): 
Solution sets

## A.1	RESTful HTTP-based solution set

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-10 | SA5#157 | S5-245426 |  |  |  | Input Draft TS 28.572 Management of planned configurations | 0.0.0 |
| 2024-10 | SA5#157 | S5-246080 |  |  |  | pCR 28.572 Add skeleton | 0.1.0 |
| 2024-10 | SA5#157 | S5-246081 |  |  |  | pCR 28.572 Add requirements for plans | 0.1.0 |
| 2024-10 | SA5#157 | S5-246082 |  |  |  | pCR 28.572 Add requirements for transaction | 0.1.0 |
| 2024-10 | SA5#157 | S5-246083 |  |  |  | pCR 28.572 Add requirements for validation | 0.1.0 |
| 2024-10 | SA5#157 | S5-246084 |  |  |  | pCR 28.572 Add requirements for activation | 0.1.0 |
| 2024-10 | SA5#157 | S5-246085 |  |  |  | pCR 28.572 Add content for plans | 0.1.0 |
| 2024-10 | SA5#157 | S5-246086 |  |  |  | pCR 28.572 Add content for transactions | 0.1.0 |
| 2024-10 | SA5#157 | S5-246087 |  |  |  | pCR 28.572 Add content for validation | 0.1.0 |
| 2024-10 | SA5#157 | S5-246088 |  |  |  | pCR 28.572 Add content for activation | 0.1.0 |