3GPP TS 29.500 V16.2.0 (2019-12)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Core Network and Terminals;

5G System;

Technical Realization of Service Based Architecture;

Stage 3

(Release 16)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

3GPP, 5GS, SBA

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

1	Scope	7

2	References	7

3	Definitions and abbreviations	8

3.1	Definitions	8

3.2	Abbreviations	8

4	Service Based Architecture Overview	9

4.1	NF Services	9

4.2	Service Based Interfaces	9

4.3	NF Service Framework	9

4.3.1	General	9

4.3.2	NF Service Advertisement URI	9

5	Protocols Over Service Based Interfaces	10

5.1	Protocol Stack Overview	10

5.2	HTTP/2 Protocol	10

5.2.1	General	10

5.2.2	HTTP standard headers	10

5.2.2.1	General	10

5.2.2.2	Mandatory to support HTTP standard headers	11

5.2.3	HTTP custom headers	12

5.2.3.1	General	12

5.2.3.2	Mandatory to support custom headers	12

5.2.3.2.1	General	12

5.2.3.2.2	3gpp-Sbi-Message-Priority	13

5.2.3.2.3	3gpp-Sbi-Callback	13

5.2.3.2.4	3gpp-Sbi-Target-apiRoot	14

5.2.3.2.5	3gpp-Sbi-Server-Binding	14

5.2.3.2.6	3gpp-Sbi-Client-Binding	15

5.2.3.2.7	3gpp-Sbi-Discovery-*	15

5.2.3.2.8	3gpp-Sbi-Producer-Id	15

5.2.3.3	Optional to support custom headers	16

5.2.3.3.1	General	16

5.2.3.3.2	3gpp-Sbi-Sender-Timestamp	16

5.2.3.3.2	3gpp-Sbi-Max-Rsp-Time	16

5.2.4	HTTP error handling	16

5.2.5	HTTP/2 server push	16

5.2.6	HTTP/2 connection management	17

5.2.7	HTTP status codes	17

5.2.7.1	General	17

5.2.7.2	NF as HTTP Server	18

5.2.7.3	NF as HTTP Client	22

5.2.8	HTTP/2 request retries	23

5.2.9	Handling of unsupported query parameters	23

5.3	Transport Protocol	23

5.4	Serialization Protocol	23

5.5	Interface Definition Language	24

6	General Functionalities in Service Based Architecture	24

6.1	Routing Mechanisms	24

6.1.1	General	24

6.1.2	Identifying a target resource	24

6.1.3	Connecting inbound	24

6.1.4	Pseudo-header setting	24

6.1.4.1	General	24

6.1.4.2	Routing within a PLMN	25

6.1.4.3	Routing across PLMN	25

6.1.4.3.1	General	25

6.1.4.3.2	Use of telescopic FQDN	26

6.1.4.3.3	Use of 3gpp-Sbi-Target-apiRoot	26

6.1.5	Host header	27

6.1.6	Message forwarding	27

6.2	Server-Initiated Communication	27

6.3	Load Control	27

6.4	Overload Control	27

6.4.1	General	27

6.4.2	HTTP Status Code "503 Service Unavailable"	28

6.4.3	HTTP Status Code "429 Too Many Requests"	28

6.4.4	HTTP Status Code "307 Temporary Redirect"	29

6.5	Support of Stateless NFs	29

6.5.1	General	29

6.5.2	Stateless AMFs	29

6.5.2.1	General	29

6.5.2.2	AMF as service consumer	29

6.5.2.3	AMF as service producer	30

6.5.3	Stateless NFs (for any 5GC NF type)	31

6.5.3.1	General	31

6.5.3.2	Stateless NF as service consumer	31

6.5.3.3	Stateless NF as service producer	31

6.6	Extensibility Mechanisms	32

6.6.1	General	32

6.6.2	Feature negotiation	32

6.6.3	Vendor-specific extensions	33

6.6.4	Extensibility for Query parameters	34

6.7	Security Mechanisms	34

6.7.1	General	34

6.7.2	Transport layer security protection of messages	34

6.7.3	Authorization of NF service access	35

6.7.4	Application layer security across PLMN	35

6.7.4.1	General	35

6.7.4.2	N32 Procedures	35

6.8	SBI Message Priority Mechanism	36

6.8.1	General	36

6.8.2	Message level priority	36

6.8.3	Stream priority	36

6.8.4	Recommendations when defining SBI Message Priorities	37

6.8.5	HTTP/2 client behaviour	38

6.8.6	HTTP/2 server behaviour	38

6.8.7	HTTP/2 proxy behaviour	38

6.8.8	DSCP marking of messages	38

6.9	Discovering the communication options supported by a target resource	39

6.9.1	General	39

6.9.2	Discoverable communication options	39

6.9.2.2	Content-encodings supported in HTTP requests	39

6.10	Support of Indirect Communication	39

6.10.1	General	39

6.10.2	Routing Mechanism with SCP with TLS between NFs and SCP	40

6.10.2.1	General	40

6.10.2.2	HTTP/2 connection management	40

6.10.2.3	Connecting inbound	40

6.10.2.4	Pseudo-header setting	40

6.10.2.5	3gpp-Sbi-Target-apiRoot header setting	41

6.10.2.5	3gpp-Sbi-Target-apiRoot header setting	41

6.10.2.6	Cache key (ck) query parameter	41

6.10.2A	Routing Mechanism with SCP without TLS between NFs and SCP	41

6.10.2A.1	Connecting inbound	41

6.10.2A.2	HTTP/2 connection management	41

6.10.2A.3	Pseudo-header setting	42

6.10.3	NF Discovery and Selection for indirect communication with Delegated Discovery	42

6.10.3.1	General	42

6.10.3.2	Conveyance of NF Discovery Factors	42

6.10.3.3	Notifications corresponding to default notification subscriptions	42

6.10.3.4	Returning the NF Service Producer ID to the NF Service Consumer	43

6.10.4	Authority and/or deployment-specific string in apiRoot of resource URI	43

6.10.5	NF / NF service instance selection for Indirect Communication without Delegated Discovery	43

6.10.5.1	General	43

6.10.6	Feature negotiation for Indirect Communication with Delegated Discovery	44

6.10.7	Routing Mechanism with SCP (when not using TLS between NFs and SCP)	44

6.10.7.1	Connecting inbound	44

6.10.7.2	HTTP/2 connection management	44

6.10.7.3	Pseudo-header setting	44

6.11	Detection and handling of late arriving requests	45

6.11.1	General	45

6.11.2	Detection and handling of requests which have timed out at the HTTP client	45

6.11.2.1	General	45

6.11.2.2	Principles	45

6.12	Binding between an NF Service Consumer and an NF Service Resource	45

6.12.1	General	45

6.12.2	Binding created as part of a service response	46

6.12.3	Binding created as part of a service request	46

6.12.4	Binding for explicit or implicit subscription requests	47

Annex A (informative): Client-side Adaptive Throttling for Overload Control	47

Annex B (normative): 3gpp-Sbi-Callback Types	48

Annex C (informative): Internal NF Routing of HTTP Requests	48

Annex D (informative): Change history	49

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document specifies the technical realization of the 5GC Service Based Architecture, protocols supported over the Service Based Interfaces, and the functionalities supported in the Service Based Architecture.

The service requirements for the 5G system are defined in 3GPP TS 22.261 [2]. The system architecture requirements are defined in 3GPP TS 23.501 [3] and the procedures and flows in 3GPP TS 23.502 [4].

The design principles and documentation guidelines for 5GC SBI APIs are specified in 3GPP TS 29.501 [5].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[3]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[4]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[5]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[6]	IETF RFC 793: "Transmission Control Protocol".

[7]	IETF RFC 7540: "Hypertext Transfer Protocol Version 2 (HTTP/2)".

[8]	3GPP TS 29.510: "5G System; Network Function Repository Services; Stage 3".

[9]	OpenAPI: "OpenAPI 3.0.0 Specification", https://github.com/OAI/OpenAPI-Specification/blob/master/versions/3.0.0.md.

[10]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[11]	IETF RFC 7231: "Hypertext Transfer Protocol (HTTP/1.1): Semantics and Content".

[12]	IETF RFC 7230: "Hypertext Transfer Protocol (HTTP/1.1): Message Syntax and Routing".

[13]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces Stage 3".

[14]	IETF RFC 3986: "Uniform Resource Identifier (URI): Generic Syntax".

[15]	3GPP TS 23.003: "Numbering, addressing and identification".

[16]	IETF RFC 5681: "TCP Congestion Control".

[17]	3GPP TS 33.501: "Security Architecture and Procedures for 5G System".

[18]	IANA: "SMI Network Management Private Enterprise Codes", http://www.iana.org/assignments/enterprise-numbers.

[19]	IETF RFC 7944: "Diameter Routing Message Priority".

[20]	IETF RFC 7234: "Hypertext Transfer Protocol (HTTP/1.1): Caching".

[21]	IETF RFC 7235: " Hypertext Transfer Protocol (HTTP/1.1): Authentication".

[22]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[23]	IETF RFC 6750: "The OAuth 2.0 Authorization Framework: Bearer Token Usage".

[24]	IETF RFC 7232: "Hypertext Transfer Protocol (HTTP/1.1): Conditional Requests".

[25]	IETF RFC 7516: "JSON Web Encryption (JWE)".

[26]	IETF RFC 7515: "JSON Web Signature (JWS)".

[27]	3GPP TS 29.573: "5G System: Public Land Mobile Network (PLMN) Interconnection; Stage 3".

[28]	3GPP TS 29.502: "5G System; Session Management Services; Stage 3".

[29]	3GPP TS 29.503: "5G System; Unified Data Management Services; Stage 3".

[30]	Void.

[31]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[32]	3GPP TS 29.531: "5G System; Network Slice Selection Services; Stage 3".

[33]	IETF RFC 7694: "Hypertext Transfer Protocol (HTTP) Client-Initiated Content-Encoding".

[34]	IETF RFC 1952: "GZIP file format specification version 4.3".

[35]	3GPP TS 29.525: "5G System; UE Policy Control Service; Stage 3".

[36]	IETF RFC 3040: "Internet Web Replication and Caching Taxonomy".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

HTTP	Hypertext Transfer Protocol

SCP	Service Communication Proxy

SMP	SBI Message Priority

TCP	Transmission Control Protocol

# 4	Service Based Architecture Overview

## 4.1	NF Services

3GPP TS 23.501 [3] defines the 5G System Architecture as a Service Based Architecture, i.e. a system architecture in which the system functionality is achieved by a set of NFs providing services to other authorized NFs to access their services.

Control Plane (CP) Network Functions in the 5G System architecture shall be based on the service based architecture.

A NF service is one type of capability exposed by a NF (NF Service Producer) to other authorized NF (NF Service Consumer) through a service based interface. A NF service may support one or more NF service operation(s). See clause 7 of 3GPP TS 23.501 [3].

Network Functions may offer different functionalities and thus different NF services. Each of the NF services offered by a Network Function shall be self-contained, acted upon and managed independently from other NF services offered by the same Network Function (e.g. for scaling, healing).

## 4.2	Service Based Interfaces

A service based interface represents how the set of services is provided or exposed by a given NF. This is the interface where the NF service operations are invoked.

The service based Control Plane interfaces within the 5G Core Network are specified in 3GPP TS 23.501 [3].

## 4.3	NF Service Framework

### 4.3.1	General

The Service Based Architecture shall support the NF Service Framework that enable the use of NF services as specified in clause 7.1 of 3GPP TS 23.501 [3].

The NF Service Framework includes the following mechanisms:

-	NF service registration and de-registration: to make the NRF aware of the available NF instances and supported services (see clause 7.1.5 of 3GPP TS 23.501 [3]);

-	NF service discovery: to enable a NF Service Consumer to discover NF Service Producer instance(s) which provide the expected NF service(s) (see clause 7.1.3 of 3GPP TS 23.501 [3]);

-	NF service authorization: to ensure the NF Service Consumer is authorized to access the NF service provided by the NF Service Producer (see clause 7.1.4 of 3GPP TS 23.501 [3]);

-	Inter service communication: NF Service Consumers and NF Service Producers may communicate directly or indirectly via a Service Communication Proxy (SCP). Whether a NF uses Direct Communication or Indirect Communication via an SCP is based on configuration of the NF.

The stage 3 procedures for NF service registration and de-registration, NF service discovery and NF service authorization are defined in 3GPP TS 29.510 [8].

### 4.3.2	NF Service Advertisement URI

When invoking a service operation of a NF Service Producer that use HTTP methods with a message body (i.e PUT, POST and PATCH), the NF Service Consumer may provide NF Service Advertisement URL(s) in the service operation request, based on operator policy, if it expects that the NF Service Producer may subsequently consume NF service(s) which the NF Service Consumer can provide (as a NF Service Producer).

When receiving NF Service Advertisement URI(s) in a service operation request, the NF Service Producer may store and use the Service Advertisement URL(s) to discover NF services produced by the NF Service Consumer in subsequent procedures, based on operator policy.

The NF Service Advertisement URI identifies the nfInstance resource(s) in the NRF which are registered by NF Service Producer(s).

An example of NF Service Advertisement URI could be represented as:

"{apiRoot}/nnrf-disc/nf-instances?nfInstanceId={nfInstanceId}".

NOTE:	The NF Service Advertisement URI can be used e.g. when different NRFs are deployed in the PLMN.

When applicable, the NF Service Advertisement URI(s) shall be carried in HTTP message body.

# 5	Protocols Over Service Based Interfaces

## 5.1	Protocol Stack Overview

The protocol stack for the service based interfaces is shown on Figure 5.1-1.

# Figure 5.1-1: SBI Protocol Stack

The service based interfaces use HTTP/2 protocol (see clause 5.2) with JSON (see clause 5.4) as the application layer serialization protocol. For the security protection at the transport layer, all 3GPP NFs shall support TLS and TLS shall be used within a PLMN if network security is not provided by other means, as specified in 3GPP TS 33.501 [17].

## 5.2	HTTP/2 Protocol

### 5.2.1	General

HTTP/2 as described in IETF RFC 7540 [7] shall be used in Service based interface.

### 5.2.2	HTTP standard headers

#### 5.2.2.1	General

This clause lists the HTTP standard headers that shall be supported on SBI, other HTTP standard headers defined in IETF RFCs may be supported by NF.

#### 5.2.2.2	Mandatory to support HTTP standard headers

The HTTP request standard headers and the HTTP response standard headers that shall be supported on SBI are defined in Table 5.2.2.2-1 and in Table 5.2.2.2-2 respectively. Mandatory to support HTTP standard headers does not mean all the HTTP requests and responses carry the identified request and response headers respectively. It only means it is mandatory to support the processing of the identified headers in request and response message.

Table 5.2.2.2-1: Mandatory to support HTTP request standard headers

| Name | Reference | Description |
| --- | --- | --- |
| Accept | IETF RFC 7231 [11] | This header is used to specify response media types that are acceptable. |
| Accept-Encoding | IETF RFC 7231 [11] | This header may be used to indicate what response content-encodings (e.g gzip) are acceptable in the response. |
| Content-Length | IETF RFC 7230 [12] | This header is used to provide the anticipated size, as a decimal number of octets, for a potential payload body. |
| Content-Type | IETF RFC 7231 [11] | This header is used to indicate the media type of the associated representation. |
| Content-Encoding | IETF RFC 7231 [11] | This header may be used in some requests to indicate the content encodings (e.g gzip) applied to the resource representation beyond those inherent in the media type. |
| User-Agent | IETF RFC 7231 [11] | This header shall be mainly used to identify the NF type of the HTTP/2 client. The pattern of the content should start with the value of NF type (e.g. udm, see NOTE 1) and followed by a "-" and any other specific information if needed afterwards. |
| Cache-Control | IETF RFC 7234 [20] | This header may be used in some HTTP/2 requests to provide the HTTP cache-control directives that the client is willing to accept from the server. |
| If-Modified-Since | IETF RFC 7232 [24] | This header may be used in a conditional GET request, for server revalidation. This is used in conjunction with the Last-Modified server response header, to fetch content only if the content has been modified from the cached version. |
| If-None-Match | IETF RFC 7232 [24] | This header may be used in a conditional GET request. This is used in conjunction with the ETag server response header, to fetch content only if the tag value of the resource on the server differs from the tag value in the If-None-Match header. |
| If-Match | IETF RFC 7232 [24] | This header may be used in a conditional POST or PUT or DELETE or PATCH request. This is used in conjunction with the ETag server response header, to update / delete content only if the tag value of the resource on the server matches the tag value in the If-Match header. |
| Via | IETF RFC 7230 [12] | This header shall be inserted by HTTP proxies. |
| Authorization | IETF RFC 7235 [21] | This header shall be used if OAuth 2.0 based access authorization with "Client Credentials" grant type is used as specified in clause 13.4.1 of 3GPP TS 33.501 [17], clause 7 of IETF RFC 6749 [22] and IETF RFC 6750 [23]. |
| NOTE 1: The value of NF type in the User-Agent header shall comply with the enumeration value of Table 6.1.6.3.3-1 in 3GPP TS 29.510 [8]. |  |  |

Table 5.2.2.2-2: Mandatory to support HTTP response standard headers

| Name | Reference | Description |
| --- | --- | --- |
| Content-Length | IETF RFC 7230 [12] | This header may be used to provide the anticipated size, as a decimal number of octets, for a potential payload body. |
| Content-Type | IETF RFC 7231 [11] | This header shall be used to indicate the media type of the associated representation. |
| Location | IETF RFC 7231 [11] | This header may be used in some responses to refer to a specific resource in relation to the response. |
| Retry-After | IETF RFC 7231 [11] | This header may be used in some responses to indicate how long the user agent ought to wait before making a follow-up request. |
| Content-Encoding | IETF RFC 7231 [11] | This header may be used in some responses to indicate to the HTTP/2 client the content encodings (e.g gzip) applied to the resource representation beyond those inherent in the media type. |
| Cache-Control | IETF RFC 7234 [20] | This header may be used in some responses (e.g. NRF responses to queries) to provide HTTP response cache control directives. The cache directives "no-cache", "no-store", "max-age" and "must-revalidate" values shall be supported. |
| Age | IETF RFC 7234 [20] | This header may be inserted by HTTP proxies when returning a cached response. The "Age" header field conveys the sender's estimate of the amount of time since the response was generated or successfully validated at the origin server. The presence of an Age header field implies that the response was not generated or validated by the origin server for this request. |
| Last-Modified | IETF RFC 7232 [24] | This header may be sent to allow for conditional GET with the If-Modified-Since header. |
| ETag | IETF RFC 7232 [24] | This header may be sent to allow for conditional GET with the If-If-None-Match header or a conditional POST / PUT / PATCH / DELETE with the If-Match header. |
| Via | IETF RFC 7230 [12] | This header shall be inserted by HTTP proxies. |
| Allow | IETF RFC 7231 [11] | This header field shall be used to indicate methods supported by the target resource. |
| WWW-Authenticate | IETF RFC 7235 [21] | This header field shall be included when an NF service producer rejects a request with a "401 Unauthorized" status code (e.g when a request is sent without an OAuth 2.0 access token or with an invalid OAuth 2.0 access token). |
| Accept-Encoding | IETF RFC 7694 [33] | See clause 6.9 for the use of this header. |

### 5.2.3	HTTP custom headers

#### 5.2.3.1	General

The list of custom HTTP headers applicable to 3GPP Service Based NFs are specified below.

#### 5.2.3.2	Mandatory to support custom headers

#### 5.2.3.2.1	General

The 3GPP NF Services shall support the HTTP custom headers specified in Table 5.2.3.2-1 below. A description of each custom header and the normative requirements on when to include them are also provided in Table 5.2.3.2-1.

Table 5.2.3.2-1: Mandatory HTTP custom headers

| Name | Reference | Description |
| --- | --- | --- |
| 3gpp-Sbi-Message-Priority | Clause 5.2.3.2.2 | This header is used to specify the HTTP/2 message priority for 3GPP service based interfaces. This header shall be included in HTTP/2 messages when a priority for the message needs to be conveyed (e.g HTTP/2 messages related to Multimedia Priority Sessions). |
| 3gpp-Sbi-Callback | Clause 5.2.3.2.3 | This header is used to indicate if a HTTP/2 message is a callback (e.g notification). This header shall be included in HTTP POST messages for callbacks towards NF service consumer(s) in another PLMN via the SEPP (See 3GPP TS 29.573 [27]). |
| 3gpp-Sbi-Target-apiRoot | Clause 5.2.3.2.4 | This header is used by an HTTP client to indicate the apiRoot of the target URI when communicating indirectly with the HTTP server via an SCP using HTTPS. |
| 3gpp-Sbi-Server-Binding | Clause 5.2.3.2.5 | This header is used in a service request to signal binding information to direct the service request to an HTTP server which has the targeted NF Service Resource context (see clause 6.12). |
| 3gpp-Sbi-Client-Binding | Clause 5.2.3.2.6 | This header is used to signal binding information related to an NF Service Resource to a future consumer (HTTP client) of that resource (see clause 6.12). |
| 3gpp-Sbi-Discovery-* | Clause 5.2.3.2.7 | Headers beginning with the prefix 3gpp-Sbi-Discovery- are used in indirect communication mode for discovery and selection of a suitable producer by the SCP. Such headers may be included in any SBI message and include information allowing an SCP to find a suitable producer as per the consumer's included delegated discovery parameters. |
| 3gpp-Sbi-Producer-Id | Clause 5.2.3.2.8 | This header is used in a service response from the SCP to the NF Service Consumer, when using indirect communication with delegated discovery and the NF Service Producer does not return a binding indication in a service response creating a resource. See clause 6.10.3.4. |

#### 5.2.3.2.2	3gpp-Sbi-Message-Priority

The header contains the HTTP/2 message priority value.

The encoding of the header follows the ABNF as defined in IETF RFC 7230 [12].

3gpp-Sbi-Message-Priority = "3gpp-Sbi-Message-Priority" ":" (DIGIT / %x31-32 DIGIT / "3" %x30-31)

A message with 3gpp-Sbi-Message-Priority "0" has the highest priority.

An example is: 3gpp-Sbi-Message-Priority: 10.

#### 5.2.3.2.3	3gpp-Sbi-Callback

The header contains the type of notification. The value for the notificaition type is a string used identifing a particular type of callback (e.g a notification, typically the name of the notify service operation).

The encoding of the header follows the ABNF as defined in IETF RFC 7230 [12].

3gpp-Sbi-Notification header field  = "3gpp-Sbi-Callback" ":" OWS cbtype

cbtype = 1*cbchar

cbchar = "-" / "_" / DIGIT / ALPHA

An example is: 3gpp-Sbi-Callback: Nnrf_NFManagement_NFStatusNotify.

The list of valid values for this header is specified in Annex B.

#### 5.2.3.2.4	3gpp-Sbi-Target-apiRoot

The header contains the apiRoot of the target URI (see clause 4.4 of 3GPP TS 29.501 [5]) in a request sent to an SCP using HTTPS, when using Indirect Communication. It may also be used in a request sent to a SEPP using HTTPS (see clause 6.1.4.3.2).

The encoding of the header follows the ABNF as defined in IETF RFC 7230 [12].

3gpp-Sbi-Target-apiRoot header field  = "3gpp-Sbi-Target-apiRoot" ":" OWS scheme "://" authority ["/" prefix ]

scheme = "http" / "https"

authority = host [ ":" port ]

port = *DIGIT

prefix = token

An example is: 3gpp-Sbi-Target-apiRoot:

#### 5.2.3.2.5	3gpp-Sbi-Server-Binding

This header contains a binding indication associated to an HTTP server (see clause 6.12).

The encoding of the header follows the ABNF as defined in IETF RFC 7230 [12].

3gpp-Sbi-Server-Binding = "3gpp-Sbi-Server-Binding" ":" OWS "bl=" blvalue 1*( OWS ";" parameter)

blvalue			= "nfinstance" / "nfset" / "nfserviceinstance" / "nfserviceset"

parameter			=  parametername "=" token

parametername	= "nfinst" / "nfset" / "nfservinst" / "nfserviceset"

The following parameters are defined:

- bl (binding level): indicates a binding to either a NF Instance, a NF set, a NF Service Instance or a NF Service Set.

- nfinst (NF instance): indicates a NF Instance ID, as defined in 3GPP TS 29.510 [8].

- nfset (NF set): indicates a NF Set ID, as defined in 3GPP TS 23.003 [15].

- nfservinst (NF service instance): indicates a NF Service Instance ID

- nfserviceset (NF service set): indicates a NF Service Set ID, as defined in 3GPP TS 23.003 [15].

See clause 3.2.6 of IETF RFC 7230 [12] for the "token" definition.

EXAMPLE 1:	Binding to SMF set 1 of MCC 345 and MNC 012: 

3gpp-Sbi-Server-Binding: bl=nfset; nfset=set1.smfset.5gc.mnc012.mcc345

EXAMPLE 2:	Binding to an SMF instance within SMF set of Example 1: 

3gpp-Sbi-Server-Binding: bl=nfinstance; nfinst=54804518-4191-46b3-955c-ac631f953ed8; nfset= set1.smfset.5gc.mnc012.mcc345

EXAMPLE 3:	Binding to a SMF Service Set "xyz" within an SMF instance within SMF set of Example 1:

3gpp-Sbi-Server-Binding: bl=nfserviceset; nfservset= setxyz.snnsmf-pdusession.nfi54804518-4191-46b3-955c-ac631f953ed8.5gc.mnc012.mcc345; nfset=set1.smfset.5gc.mnc012.mcc345

EXAMPLE 4:	Binding to AMF set 1 within AMF region 48 (hexadecimal):
3gpp-Sbi-Server-Binding: bl=nfset; nfset= set1.region48.amfset.5gc.mnc012.mcc345

#### 5.2.3.2.6	3gpp-Sbi-Client-Binding

This header contains a binding indication from an HTTP server for storage and subsequent use by an HTTP client (see clause 6.12).

The encoding of the header follows the ABNF as defined in IETF RFC 7230 [12].

3gpp-Sbi-Client-Binding = "3gpp-Sbi-Client-Binding" ":" OWS "bl=" blvalue 1*( OWS ";" parameter)

The encoding of the blvalue and parameters shall be as defined for the 3gpp-Sbi-Server-Binding in clause 5.2.3.2.5.

#### 5.2.3.2.7	3gpp-Sbi-Discovery-*

These headers shall be used to convey NF service discovery factors to the SCP in indirect communication models. They contain discovery parameters to be conveyed by the NF consumer to the SCP and shall be used for finding a suitable NF producer instance, e.g. by performing the NF service discovery procedure with the NRF on behalf of the NF consumer in case of indirect communication with delegated discovery model.

The name of each NF service discovery factors header shall be constructed by concatenating the string "3gpp-Sbi-Discovery-" with the name of the conveyed discovery parameter, i.e. "3gpp-Sbi-Discovery-<discovery parameter>".

The discovery headers shall be used to include any of the discovery query parameters listed in 3GPP TS 29.510 [8], Table 6.2.3.2.3.1-1. The value of each NF service discovery header shall be encoded in the same way as the corresponding discovery parameter. Thus, the values of these headers may be validated with the same data model as that of the corresponding discovery parameters. Table 5.2.3.2.7-1 lists examples of NF service discovery headers.

Table 5.2.3.2.7-1: NF service discovery factors headers examples

| Header in request | Discovery parameter | Header value | Data Model |
| --- | --- | --- | --- |
| 3gpp-Sbi-Discovery-target-nf-type: AMF | target-nf-type (TS 29.510 [8], Table 6.2.3.2.3.1-1) | AMF | NFType: Enumeration as of TS 29.510 [8], Table 6.1.6.3.3-1. |
| 3gpp-Sbi-Discovery-snssais: [{"sst": 1, "sd": "A08923"}, {"sst": 1, "sd": "0023F1"}] | snssais (TS 29.510 [8], Table 6.2.3.2.3.1-1) | [{"sst": 1, "sd": "A08923"}, {"sst": 1, "sd": "0023F1"}] | array(Snssai), where Snssai is a structured data type as of TS 29.571 [13], Table 5.4.4.2-1 |
| 3gpp-Sbi-Discovery-target-nf-instance-id: e553cf50-f32b-4638-8a7e-0d416cc60952 | target-nf-instance-id (TS 29.510 [8], Table 6.2.3.2.3.1-1) | e553cf50-f32b-4638-8a7e-0d416cc60952 | NfInstanceId: simple data type as of TS 29.571 [13], Table 5.3.2-1 |

Editor's Note: The handling of non-supported discovery headers is FFS

Editor's Note: How the discovery header shall be described in OpenAPI is FFS

#### 5.2.3.2.8	3gpp-Sbi-Producer-Id

This header contains the NF Service Producer Instance ID (see clause 6.10.3.4).

The encoding of the header follows the ABNF as defined in IETF RFC 7230 [12].

3gpp-Sbi-Producer-Id = "3gpp-Sbi-Producer-Id" ":" OWS "nfinst=" nfInstanceIdvalue

The following parameter is defined:

- nfinst (NF instance): indicates a NF Instance ID, as defined in 3GPP TS 29.510 [8].

EXAMPLE:	3gpp-Sbi-Producer-Id: nfinst=54804518-4191-46b3-955c-ac631f953ed8

#### 5.2.3.3	Optional to support custom headers

#### 5.2.3.3.1	General

The 3GPP NF Services may support the HTTP custom headers specified in Table 5.2.3.3-1 below. A description of each custom header and the normative requirements on when to include them are also provided in Table 5.2.3.3-1.

Table 5.2.3.3-1: Optional HTTP custom headers

| Name | Reference | Description |
| --- | --- | --- |
| 3gpp-Sbi-Sender-Timestamp | Clause 5.2.3.3.2 | This header may be used to indicate the date and time (with a millisecond granularity) at which an HTTP request or response is originated. This may be used e.g. for measuring signalling delays between different NF service instances. |
| 3gpp-Sbi-Max-Rsp-Time | Clause 5.2.3.2.3 | This header may be used in a HTTP request to indicate the duration during which the HTTP client waits for a response. See clause 6.11.2. |

#### 5.2.3.3.2	3gpp-Sbi-Sender-Timestamp

The header contains the date and time (with a millisecond granularity) at which an HTTP request or response is originated.

The encoding of the header follows the ABNF as defined in IETF RFC 7230 [12].

3gpp-Sbi-Sender-Timestamp = "3gpp-Sbi-Sender-Timestamp" ":" day-name "," SP date1 SP time-of-day "." milliseconds SP GMT

milliseconds = 3DIGIT

day-name, date1, time-of-day shall comply with the definition in clause 7.1.1.1 of IETF RFC 7231 [11].

When a 3gpp-Sbi-Sender-Timestamp header field is generated, the sender should generate its field value as the best available approximation of the date and time of message generation.

NOTE:	This is the same format as the Date header of clause 7.1.1.2 of IETF RFC 7231 [11], but with the time expressed with a millisecond granularity.

EXAMPLE:	3gpp-Sbi-Sender-Timestamp: Sun, 04 Aug 2019 08:49:37.845 GMT

#### 5.2.3.3.2	3gpp-Sbi-Max-Rsp-Time

The header indicates the duration, expressed in milliseconds since the request was originated, during which the HTTP client waits for a response. See clause 6.x.2.

The encoding of the header follows the ABNF as defined in IETF RFC 7230 [12].

3gpp-Sbi-Max-Rsp-Time = "3gpp-Sbi-Max-Rsp-Time" ":" 1*5DIGIT

EXAMPLE:	3gpp-Sbi-Max-Rsp-Time: 10000

### 5.2.4	HTTP error handling

HTTP/2 connection error and stream error shall be supported as specified in clause 5.4 of IETF RFC 7540 [7].

Guidelines for error responses to the invocation of APIs of NF services are specified in clause 4.8 of 3GPP TS 29.501 [3]. API specific error responses are specified in the respective technical specifications.

### 5.2.5	HTTP/2 server push

HTTP/2 Server Push as specified in clause 8.2 of IETF RFC 7540 [7] may be supported and may be used by a NF Service Producer to proactively push resources to a NF Service Consumer, see clause 4.9.5 of 3GPP TS 29.501 [5].

A NF Service Consumer may choose to disable HTTP/2 Server Push by setting SETTINGS_ENABLE_PUSH to 0, as specified in clause 8.2 of IETF RFC 7540 [7].

### 5.2.6	HTTP/2 connection management

The HTTP request / response exchange mechanism as specified in clause 8.1 of IETF RFC 7540 [7] shall be supported between the 3GPP NFs. An HTTP/2 endpoint shall support establishing multiple HTTP/2 connections (at least two) towards a peer HTTP/2 endpoint. The peer HTTP/2 endpoint is identified by host and port pair where the host is derived from the target URI (see clause 6.1.1).

NOTE 1:	HTTP/2 connection redundancy allows transporting messages through diverse IP paths and improve 5GC resiliency.

As per clause 8.1 of IETF RFC 7540 [7] a HTTP request / response exchange fully consumes a single stream. When the HTTP/2 Stream IDs on a given HTTP/2connection is exhausted, an HTTP/2 endpoint, shall establish another HTTP/2connection towards that peer HTTP/2 endpoints.

NOTE 2: As per IETF RFC 7540 [7], a stream ID once closed cannot be reused on the same HTTP/2 connection.

The 3GPP NF shall take care to avoid simultaneous stream ID exhaustion on all the available HTTP/2 connections towards each peer.

An NF acting as an HTTP/2 client shall support testing whether a connection is still active by sending a PING frame as specified in clause 6.7 of IETF RFC 7540 [7]. When and how often a PING frame may be sent is implementation specific but shall be configurable by operator policy.

A PING frame shall not be sent more often than every 60 s on each path.

### 5.2.7	HTTP status codes

#### 5.2.7.1	General

This clause describes the HTTP status codes usage on SBI.

HTTP status codes are carried in ":status" pseudo header field in HTTP/2, as defined in clause 8.1.2.4 in IETF RFC 7540 [7].

Table 5.2.7.1-1 specifies HTTP status codes per HTTP method which shall be supported on SBI. Support of an HTTP status code shall be:

-	mandatory, which is marked in table as "M". This means that all 3GPP NFs shall support the processing of the specific HTTP status code for the specific HTTP method, when received in a HTTP response message. In such cases the 3GPP NF shall also support the handling of the "ProblemDetails" JSON object with the Content-Type header field set to the value "application/problem+json" for HTTP status codes 4xx and 5xx, if the corresponding API definition in the related technical specification does not specify another response body for the corresponding status code;

-	service specific, which is marked in table as "SS" and means that the requirement to process the HTTP status code depends on the definition of the specific API; or

-	not applicable, which is marked in table as "N/A". This means that the specific HTTP status code shall not be used for the specific HTTP method within the 3GPP NFs.

Table 5.2.7.1-1: HTTP status code supported on SBI

| HTTP status code | HTTP method |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | DELETE | GET | PATCH | POST | PUT | OPTIONS |
| 100 Continue | N/A | N/A | N/A | N/A | N/A | N/A |
| 200 OK (NOTE 1, NOTE 2) | SS | M | SS | SS | SS | M |
| 201 Created | N/A | N/A | N/A | SS | SS | N/A |
| 202 Accepted | SS | N/A | SS | SS | SS | N/A |
| 204 No Content (NOTE 2) | M | N/A | SS | SS | SS | SS |
| 300 Multiple Choices | N/A | N/A | N/A | N/A | N/A | N/A |
| 303 See Other | SS | SS | N/A | SS | SS | N/A |
| 307 Temporary Redirect | SS | SS | SS | SS | SS | SS |
| 308 Permanent Redirect | SS | SS | SS | SS | SS | SS |
| 400 Bad Request | M | M | M | M | M | M |
| 401 Unauthorized | M | M | M | M | M | M |
| 403 Forbidden | M | M | M | M | M | M |
| 404 Not Found | M | M | M | M | M | M |
| 405 Method Not Allowed | SS | SS | SS | SS | SS | SS |
| 406 Not Acceptable | N/A | M | N/A | N/A | N/A | SS |
| 408 Request Timeout | SS | SS | SS | SS | SS | SS |
| 409 Conflict | N/A | N/A | SS | SS | SS | N/A |
| 410 Gone | SS | SS | SS | SS | SS | SS |
| 411 Length Required | N/A | N/A | M | M | M | SS |
| 412 Precondition Failed | SS | SS | SS | SS | SS | N/A |
| 413 Payload Too Large | N/A | N/A | M | M | M | SS |
| 414 URI Too Long | N/A | SS (NOTE 3) | N/A | N/A | SS | N/A |
| 415 Unsupported Media Type | N/A | N/A | M | M | M | SS |
| 429 Too Many Requests | M | M | M | M | M | M |
| 500 Internal Server Error | M | M | M | M | M | M |
| 501 Not Implemented | SS | SS | SS | SS | SS | SS |
| 503 Service Unavailable | M | M | M | M | M | M |
| 504 Gateway Timeout | SS | SS | SS | SS | SS | SS |
| NOTE 1: "200 OK" response used on SBI shall contain body. NOTE 2: If the NF acting as an HTTP Client receives 2xx response code not appearing in table, the NF shall treat the received 2xx response: - as "204 No Content" if 2xx response does not contain body; and - as "200 OK" if 2xx response contains body. NOTE 3: If GET method includes any query parameter, the NF acting as an HTTP Client shall support "414 URI Too Long" status code. |  |  |  |  |  |  |

#### 5.2.7.2	NF as HTTP Server

A NF acting as an HTTP server shall be able to generate HTTP status codes specified in clause 5.2.7.1 per indicated HTTP method.

A request using an HTTP method which is not supported by any resource of a given 5GC SBI API shall be rejected with the HTTP status code "501 Not Implemented".

NOTE 1:	In this case, the NF does not need to include in the HTTP response the "cause" attribute indicating corresponding error since the HTTP status code "501 Not Implemented" itself provides enough information of the error, i.e. the NF does not recognize the HTTP method.

If the specified target resource does not exist, the NF shall reject the HTTP method with the HTTP status code "404 Not Found".

If the NF supports the HTTP method for several resources in the API, but not for the target resource of a given HTTP request, the NF shall reject the request with the HTTP status code "405 Method Not Allowed" and shall include in the response an Allow header field containing the supported method(s) for that resource.

NOTE 2:	In this case, the NF does not need to include in the HTTP response the "cause" attribute indicating corresponding error since the HTTP status code "405 Method Not Allowed" itself provides enough information of the error and hence the Allow header field lists HTTP method(s) supported by the target resource.

If a received HTTP request contains unknown IEs, i.e. Information Elements within the JSON body, the NF may discard such IEs and shall process the rest of the request message, unless the schema definition of the received message prohibits the presence of additional IEs or constrains their types. There are cases (e.g. Nnrf_NFManagement API) where the receiver of certain HTTP requests needs to process unknown IEs (e.g. to store in NRF an NF Profile containing vendor-specific attributes, and send them in NFDiscovery results).

If a received HTTP request contains IEs or query parameters not compliant with the schema defined in the corresponding OpenAPI specification, the NF should reject the request with the appropriate error code, e.g. "400 Bad Request (INVALID_MSG_FORMAT)", even when the failed IEs are defined as optional by the schema.

If a received HTTP PATCH request contains a body with modification instruction(s) for unknown attribute(s) in addition to modification instruction(s) for known attribute(s), the NF shall:

a)	implement all the modification(s) for known attribute(s) and unknown attribute(s) if explicitly specified in the corresponding specification of the API; or

b)	otherwise, implement the modification(s) for known attribute(s) and discard those modification instruction(s) for unknown attribute(s). The NF may additionally indicate in the response the result of the execution of the PATCH request, i.e. which modification(s) are implemented and/or which modification(s) are discarded, using the "PatchResult" JSON object as defined in 3GPP TS 29.571 [13].

If the NF supports the HTTP method by a target resource but the NF cannot successfully fulfil the received request, the following requirements apply.

A NF as HTTP Server should map application errors to the most similar 3xx/4xx/5xx HTTP status code specified in table 5.2.7.1-1. If no such code is applicable, it should use "400 Bad Request" status code for errors caused by client side or "500 Server Internal Error" status code for errors caused on server side.

If the received HTTP request contains unsupported payload format, the NF shall reject the HTTP request with the HTTP status code "415 Unsupported Media Type". If the HTTP PATCH method is rejected due to unsupported patch document, the NF shall include the Accept-Patch header field set to the value of supported patch document media types for a target resource i.e. to "application/merge-patch+json" if the NF supports "JSON Merge Patch" and to "application/json-patch+json" if the NF supports "JSON Patch". If the received HTTP PATCH request contains both "JSON Merge Patch" and "JSON Patch" documents and the NF supports only one of them, the NF shall ignore unsupported patch document.

NOTE 3:	The format problem might be due to the request's indicated Content-Type or Content-Encoding header fields, or as a result of inspecting the payload body directly.

If the received HTTP request contains payload body larger than the NF is able to process, the NF shall reject the HTTP request with the HTTP status code "413 Payload Too Large".

If the result of the received HTTP POST request used for a resource creation would be equivalent to the existing resource, the NF shall reject the HTTP request with the HTTP status code "303 See Other" and shall include in the HTTP response a Location header field set to the URI of the existing resource.

Protocol and application errors common to several 5GC SBI API specifications for which the NF shall include in the HTTP response a payload body ("ProblemDetails" data structure or application specific error data structure) with the "cause" attribute indicating corresponding error are listed in table 5.2.7.2-1.

Table 5.2.7.2-1: Protocol and application errors common to several 5GC SBI API specifications

| Protocol or application Error | HTTP status code | Description |
| --- | --- | --- |
| INVALID_API | 400 Bad Request | The HTTP request contains an unsupported API name or API version in the URI. |
| INVALID_MSG_FORMAT | 400 Bad Request | The HTTP request has an invalid format. |
| INVALID_QUERY_PARAM | 400 Bad Request | The HTTP request contains an unsupported query parameter in the URI. (NOTE 1) |
| MANDATORY_QUERY_PARAM_INCORRECT | 400 Bad Request | A mandatory query parameter, or a conditional query parameter but mandatory required, for an HTTP method was received in the URI with semantically incorrect value. (NOTE 1) |
| OPTIONAL_QUERY_PARAM_INCORRECT | 400 Bad Request | An optional query parameter for an HTTP method was received in the URI with a semantically incorrect value that prevents successful processing of the service request. (NOTE 1) |
| MANDATORY_QUERY_PARAM_MISSING | 400 Bad Request | Query parameter which is defined as mandatory, or as conditional but mandatory required, for an HTTP method is not included in the URI of the request. (NOTE 1) |
| MANDATORY_IE_INCORRECT | 400 Bad Request | A mandatory IE (within the JSON body or within a variable part of an "apiSpecificResourceUriPart" or within an HTTP header), or conditional IE but mandatory required, for an HTTP method was received with a semantically incorrect value. (NOTE 1) |
| OPTIONAL_IE_INCORRECT | 400 Bad Request | An optional IE (within the JSON body or within an HTTP header) for an HTTP method was received with a semantically incorrect value that prevents successful processing of the service request. (NOTE 1) |
| MANDATORY_IE_MISSING | 400 Bad Request | A mandatory IE (within the JSON body or within the variable part of an "apiSpecificResourceUriPart" or within an HTTP header), or conditional IE but mandatory required, for an HTTP method is not included in the request. (NOTE 1) |
| UNSPECIFIED_MSG_FAILURE | 400 Bad Request | The request is rejected due to unspecified client error. (NOTE 2) |
| NF_DISCOVERY_FAILURE | 400 Bad Request | The request is rejected by the SCP because no NF Service Producer can be found matching the NF service discovery factors. |
| MODIFICATION_NOT_ALLOWED | 403 Forbidden | The request is rejected because the contained modification instructions attempt to modify IE which is not allowed to be modified. |
| SUBSCRIPTION_NOT_FOUND | 404 Not Found | The request for modification or deletion of subscription is rejected because the subscription is not found in the NF. |
| RESOURCE_URI_STRUCTURE_NOT_FOUND | 404 Not Found | The request is rejected because a fixed part after the first variable part of an "apiSpecificResourceUriPart" (as defined in clause 4.4.1 of 3GPP TS 29.501 [5]) is not found in the NF. This fixed part of the URI may represent a sub-resource collection (e.g. contexts, subscriptions, policies) or a custom operation. (NOTE 5) |
| INCORRECT_LENGTH | 411 Length Required | The request is rejected due to incorrect value of a Content-length header field. |
| NF_CONGESTION_RISK | 429 Too Many Requests | The request is rejected due to excessive traffic which, if continued over time, may lead to (or may increase) an overload situation. |
| INSUFFICIENT_RESOURCES | 500 Internal Server Error | The request is rejected due to insufficient resources. |
| UNSPECIFIED_NF_FAILURE | 500 Internal Server Error | The request is rejected due to unspecified reason at the NF. (NOTE 3) |
| SYSTEM_FAILURE | 500 Internal Server Error | The request is rejected due to generic error condition in the NF. |
| NF_CONGESTION | 503 Service Unavailable | The NF experiences congestion and performs overload control, which does not allow the request to be processed. (NOTE 4) |
| TIMED_OUT_REQUEST | 504 Gateway Timeout | The request is rejected due a request that has timed out at the HTTP client (see clause 6.11.2). |
| NOTE 1: "invalidParams" attribute shall be included in the "ProblemDetails" data structure indicating unsupported, missing or incorrect IE(s) or query parameter(s). NOTE 2: This application error indicates error in the HTTP request and there is no other application error value that can be used instead. NOTE 3: This application error indicates error condition in the NF and there is no other application error value that can be used instead. NOTE 4: If the reason for rejection is a temporary overload, the NF may include in the response a Retry-After header field to indicate how long the service is expected to be unavailable. NOTE 5: If the request is rejected because of an error in an URI before the first variable part of an "apiSpecificResourceUriPart", the "404 Not Found" HTTP status code may be sent without "ProblemDetails" data structure indicating protocol or application error. |  |  |

#### 5.2.7.3	NF as HTTP Client

Besides the HTTP Status Codes defined in the API specification, a NF as HTTP client should support handling of 1xx, 3xx, 4xx and 5xx HTTP Status Codes specified in table 5.2.7.1-1, following the client behaviour in corresponding IETF RFC where the received HTTP Status Code is defined.

When receiving a not recommended or not recognized 1xx, 3xx, 4xx or 5xx HTTP Status Code, a NF as HTTP client should treat it as x00 status code of the class, as described in clause 6 of IETF RFC 7231 [11].

If 100, 200/204, 300, 400 or 500 response code is not defined by the API specification, the client may follow guidelines below:

a)	For 1xx (Informational):

1)	Discard the response and wait for final response.

b)	For 2xx (Successful):

1)	Consider the service operation is successful if no mandatory information is expected from the response payload in subsequent procedure.

2)	If mandatory information is expected from response payload in subsequent procedure, parse the payload following description in clause 6.2.1 of IETF RFC 7231 [11]. If parse is successful and mandatory information is extracted, continue with subsequent procedure.

3)	Otherwise, consider service operation has failure and start failure handling.

c)	For 3xx (Redirection):

1)	Retry the request towards the directed resource referred in the Location header, using same request method.

d)	For 4xx (Client Error):

1)	Validate the request message and make correction before resending. Otherwise, stop process and go to error handling procedure.

e)	For 5xx (Server Error):

1)	Stop process and go to error handling process.

The handling of unknown, unexpected or erroneous HTTP request message IEs shall provide for the forward compatibility of the HTTP APIs used for the service based interfaces. Therefore, the sending HTTP entity shall be able to safely include in a message a new optional IE. Such an IE may also have a new type. A receiving HTTP entity shall behave as specified in clause 5.2.7.2.

### 5.2.8	HTTP/2 request retries

All NF services expose APIs across the service based interfaces and the APIs operate on resources. Invocation of an API though a HTTP method may result in the change of state of a resource depending of the request type. When a HTTP/2 client sends a request and it does not receive a response or it experiences a delay, it does not guarantee that the HTTP/2 request has not been processed by the HTTP/2 server.

A HTTP/2 client may retry the same request that uses an idempotent method any time (see IETF RFC 7231 [11] clause 4.2.2).

Retrying a non-idempotent HTTP/2 request on the same resource before a response for the previous request is received may lead to state changes on the resource with unspecified behaviour. HTTP conditional requests, as specified in IETF RFC 7232 [24] may be used to avoid such situations.

An NF acting as an HTTP/2 client may decide to retry non-idempotent request if it matches one of the conditions set out in clause 8.1.4 of IETF RFC 7540 [7]. API specific mechanisms as specified in respective technical specifications may be used for reconciling the state of resources, if the retry is attempted through a new TCP connection after a TCP connection failure.

The number of retry shall be limited. A client should always prefer to retry requests to an alternative server if the initial server is overloaded. In case of general overload situation where all possible servers are overloaded retry mechanisms should be disabled automatically.

### 5.2.9	Handling of unsupported query parameters

Unless specified otherwise for an API, a NF Service Producer that receives an HTTP request with one or more unsupported (i.e. not comprehended) query parameters shall:

a)	for safe HTTP methods (e.g. HTTP GET request):

-	ignore the unsupported query parameters and respond to the request based on the rest of the request (e.g. other supported query parameters); or

-	reject the HTTP request as specified below for non-safe HTTP methods, e.g. based on other query parameters in the request or based on a response becoming very large;

b)	for non-safe HTTP methods:

-	reject the request with a 400 Bad Request including a ProblemDetails IE with:

-	the cause attribute set to INVALID_QUERY_PARAM;

-	the invalidParams attribute indicating the unsupported query parameters;

-	the supportedFeatures attribute listing the features supported by the NF Service Producer, if any, set as specified for HTTP responses in clause 6.6.2.

## 5.3	Transport Protocol

The Transmission Control Protocol as described in IETF RFC 793 [6] shall be used as transport protocol as required by HTTP/2 (see IETF RFC 7540 [7]).

NOTE:	When using TCP as the transport protocol, an HTTP/2 connection is mapped to a TCP connection.

If a Network Function does not register any port number to the NRF then it shall be prepared to receive connections on default port numbers, i.e. TCP port 80 for "http" URIs and TCP port 443 for "https" URIs as specified in IETF RFC 7540 [7].

## 5.4	Serialization Protocol

The JavaScript Object Notation (JSON) format as described in IETF RFC 8259 [10] shall be used as serialization protocol.

For transmitting large parts of opaque binary data along with JSON format, multipart messages shall be supported using:

-	A multipart/related media type;

-	3gpp vendor specific content subtype; and

-	Cross-referencing from the JSON payload using the Content-ID field.

Use of multipart messages is documented in specific specifications.

## 5.5	Interface Definition Language

OpenAPI Specification [9] shall be used as Interface Definition Language (IDL) of SBI.

# 6	General Functionalities in Service Based Architecture

## 6.1	Routing Mechanisms

### 6.1.1	General

This clause specifies the generic routing mechanisms in the 5GC. Specific requirements to support Indirect Communication are further defined in clause 6.10.

For HTTP message routing between Network Functions, the message routing mechanism as specified in clause 5 of IETF RFC 7230 [12] is almost followed with some differences due to the adoption of HTTP/2 and to some 5G system specificities.

NOTE:	The term "inbound" are defined in clause 2.3 of IETF RFC 7230 [12]. It describes a directional requirement in relation to the request route: "inbound" means toward the origin server.

### 6.1.2	Identifying a target resource

The target resource is identified by a target URI (e.g. a Resource URI, a Custom operation URI or a Callback URI as defined in clause 4.4 of 3GPP TS 29.501 [5]).

### 6.1.3	Connecting inbound

If the request is not satisfied by a local cache, then the client shall connect to an authority server for the target resource or to a proxy.

If a proxy is applicable for the target URI, the client connects inbound by establishing (or reusing) a connection to that proxy as defined in clause 5.2 of IETF RFC 7230 [12]. For connecting inbound to an authority not in the same PLMN, the client connects to the Security Edge Protection Proxy.

If no proxy is applicable, then the client connects directly to an authority server for the target resource as defined in IETF RFC 7230 [12].

### 6.1.4	Pseudo-header setting

#### 6.1.4.1	General

Once an inbound connection is obtained, the client sends a request message over the wire. The message starts with a HEADERS frame containing the Pseudo-Header Fields identifying the request target. The ":method" pseudo-header is always present.

When sending a request directly to an origin server or to a proxy, other than a CONNECT or server-wide OPTIONS request, a client shall include the below pseudo-headers:

-	":scheme".

-	":authority".

-	"path" includes the path and query components of the target URI. The path includes the optional deployment-specific string of the Resource URI or Custom operation URI "apiRoot" part.

When sending a CONNECT request to a proxy, a client shall include the ":authority" pseudo-header. The ":scheme" and ":path" ones shall be absent.

When sending a server-wide OPTIONS request to an origin server or to a proxy, a client shall include the below pseudo-headers:

-	":scheme".

-	":authority".

-	"path" set with the value "*".

#### 6.1.4.2	Routing within a PLMN

For HTTP/2 request messages where the target URI authority component designates an origin server in the same PLMN as the client, the ":authority" HTTP/2 pseudo-header field shall be set to:

":authority" = uri-host [":" port] as specified in clause 8.1.2.3 of IETF RFC 7540 [7], excluding the [userinfo "@"] information as specified in clause 3.2 of IETF RFC 3986 [14].

Where the uri-host shall be:

-	FQDN of the target NF service; or

-	IP address of the target NF service

The FQDN of the target NF service need not contain the PLMN identifier.

#### 6.1.4.3	Routing across PLMN

##### 6.1.4.3.1	General

In order to reach the correct target NF service in the right PLMN and for HTTP/2 request messages where the target URI authority component designates an origin server not in the same PLMN as the client, the ":authority" HTTP/2 pseudo-header shall contain the FQDN including the PLMN ID.

The ":authority" pseudo-header field in the HTTP/2 request message shall be set to:

":authority" = uri-host [":" port] as specified in clause 8.1.2.3 of IETF RFC 7540 [7], excluding the [userinfo "@"] information as specified in clause 3.2 of IETF RFC 3986 [14].

Where the uri-host shall be:

-	FQDN of the target NF service or the FQDN (authority) part of a callback URI or a specified link relation

The FQDN of the target NF service or the FQDN (authority) part of a callback URI or a specified link relation shall contain the PLMN identifier.

The format of the FQDN of target NF service is specified in clause 28.5 of 3GPP TS 23.003 [15].

To allow for TLS protection between the SEPP and Network Functions within a PLMN, the SEPP shall support:

-	TLS wildcard certificate for its domain name and generation of telescopic FQDN, as specified in clause 13.1 of 3GPP TS 33.501 [17] and in clause 6.1.4.3.2; and

-	forwarding HTTP requests originated by NFs within the SEPP's PLMN towards the remote PLMN using the 3gpp-Sbi-Target-apiRoot header as specified in clause 6.1.4.3.3.

NOTE:	Whether the SEPP and NFs within the SEPP's PLMN use telescopic FQDN or the 3gpp-Sbi-Target-apiRoot header is based on PLMN operator's policy and is independent from the method supported and used in the remote PLMN.

Both solutions for TLS protection between the SEPP and Network Functions within a PLMN may be used concurrently in a PLMN, e.g. in the transient phase where not all NFs of the PLMN have been upgraded to support the 3gpp-Sbi-Target-apiRoot header but when the PLMN operator would like to use the solution based on the 3gpp-Sbi-Target-apiRoot header with upgraded NFs. In this case, the SEPP should skip converting URIs into telescopic FQDNs (and use the solution based on 3gpp-Sbi-Target-apiRoot header) in:

-	HTTP responses received from the remote PLMN (e.g. including the FQDN of the target NF service) when the corresponding HTTP request contains a 3gpp-Sbi-Target-apiRoot header;

-	HTTP requests received from the remote PLMN (e.g. including callback URIs) using SEPP policies based on the target URI (i.e. target FQDN).

##### 6.1.4.3.2	Use of telescopic FQDN

When using TLS wildcard certificate and telescopic FQDN between the SEPP and NFs within the SEPP's PLMN, the SEPP on the HTTP/2 client side shall form the telescopic FQDN, as specified in 3GPP TS 23.003 [15], for the following cases:

-	FQDN of the target NF service in HPLMN is modified into a telescopic FQDN by the SEPP in the VPLMN;

-	FQDN of the target NF service in VPLMN is modified into a telescopic FQDN by the SEPP in the HPLMN;

-	FQDN (authority) part of callback URI of NF service resources in VPLMN is modified into a telescopic FQDN by the SEPP in the HPLMN;

-	FQDN (authority) part of callback URI of NF service resources in HPLMN is modified into a telescopic FQDN by the SEPP in the VPLMN;

-	FQDN (authority) part of link relation URI of NF service resources in VPLMN is modified into a telescopic FQDN by the SEPP in the HPLMN;

-	FQDN (authority) part of link relation URI of NF service resources in HPLMN is modified into a telescopic FQDN by the SEPP in the VPLMN.

##### 6.1.4.3.3	Use of 3gpp-Sbi-Target-apiRoot

When using the 3gpp-Sbi-Target-apiRoot header between the SEPP and NFs within the SEPP's PLMN, HTTP requests between the NFs and the SEPP shall be routed as specified in clause 6.10.2 for indirect communications, with the SEPP taking the role of the SCP.

When sending an HTTP request targeting a URI with an authority of a remote PLMN, NFs shall include the 3gpp-Sbi-Target-apiRoot header in the HTTP request, containing the apiRoot of the target URI in the remote PLMN, and shall set the authority in the request URI to the FQDN of the SEPP (or to the FQDN of the SCP if the communication between the NF and SEPP goes through an SCP).

An SCP that receives an HTTP request targeting a URI with an authority of a remote PLMN shall route the HTTP request towards the SEPP as specified in clause 6.10.2 for indirect communications, i.e. the SCP shall forward the 3gpp-Sbi-Target-apiRoot header in the HTTP request it forwards to the SEPP, containing the apiRoot of the target URI in the remote PLMN, and it shall set the the authority in the request URI to the FQDN of the SEPP.

The SEPP shall remove the 3gpp-Sbi-Target-apiRoot header and set the apiRoot of the request URI it forwards on the N32-f interface to the apiRoot received in the 3gpp-Sbi-Target-apiRoot header from the HTTP client.

If the SEPP receives an HTTP request from a NF with a request URI containing a telescopic FQDN and with a 3gpp-Sbi-Target-apiRoot header, the SEPP shall ignore the 3gpp-Sbi-Target-apiRoot header and route the request using the telescopic FQDN.

NOTE 1:	This is to address the case of a potentially malicious or misbehaving NF that would include the 3gpp-Sbi-Target-apiRoot header and a request URI containing a telescopic FQDN when communicating with the SEPP.

NOTE 2:	This solution does not require the SEPP to support TLS wildcard certificate for its domain name, nor the SEPP to modify URI attributes in HTTP request and response payloads with telescopic FQDNs.

NOTE 3:	The communication between the NF and SEPP can be direct or go through an SCP.

### 6.1.5	Host header

Clients that generate HTTP/2 requests shall use the ":authority" pseudo-header field instead of the Host header field.

### 6.1.6	Message forwarding

An HTTP/2 proxy shall use the ":authority" pseudo-header field to connect inbound to the origin server or another proxy if the request cannot be satisfied by the proxy cache.

An HTTP/2 proxy may also use other headers and/or payload content to connect inbound to the origin server or another proxy if the request cannot be satisfied by the proxy cache.

## 6.2	Server-Initiated Communication

The Subscribe-Notify service operations shall be supported between NFs as specified in clause 7.1.2 of 3GPP TS 23.501 [3].

Subscribe-Notify service operations require bidirectional communication between the NFs when the server needs to initiate communication with the client.

Subscribe-Notify service operations shall be supported with two TCP connections, one per direction, as follows:

-	NF service consumer acts as an HTTP client and NF service producer acts as an HTTP server when NF service consumer subscribes to NF service producer's notifications;

-	NF service producer acts as an HTTP client and NF service consumer acts as an HTTP server when NF service producer delivers notifications to NF service consumer.

## 6.3	Load Control

Load control for the service based interfaces, based on the current dynamic load on an NF and/or NF service instance, is not specified in this release of this specification.

During NF discovery procedures (see clause 4.17.4 and 4.17.5 of 3GPP TS 23.502 [4]), the NRF may provide the NF instance and/or the NF service instance information with the NF capacity information advertised during NF Service Registration and/or NF Service Update procedures (see clause 4.17.1 and 4.17.2 of 3GPP TS 23.502 [4] and clause 6.2.6 of 3GPP TS 23.501 [3]). The NRF may also provide load information of the NF instance and/or the NF service instance in NF discovery response.

The NF service consumer that is discovering the NF service producer, may use the available information (e.g. NF capacity information, load information) to select the appropriate NF instance as specified in 3GPP TS 29.510 [8].

## 6.4	Overload Control

### 6.4.1	General

Service Based Interfaces use HTTP/2 over TCP for communication between the NF Services. TCP provides transport level congestion control mechanisms as specified in IETF RFC 5681 [16], which may be used for congestion control between two TCP endpoints (i.e., hop by hop). HTTP/2 also provides flow control mechanisms and limitation of stream concurrency, as specified in IETF RFC 7540 [7], that may be configured for connection level congestion control.

In addition to TCP and HTTP/2 congestion control mechanisms, end to end overload control shall be supported per NF service / API according to the below defined principles.

An NF Service Producer may mitigate a potential overload status by sending the NF Service Consumer the following HTTP status codes as a response to requests received during, or close to reaching, an overload situation:

-	503 Service Unavailable;

-	429 Too Many Requests; or

-	307 Temporary Redirect

The first 2 status codes (503 and 429) are intended to inform the NF Service Consumer that the server cannot handle the current received traffic rate, so it shall abate the traffic sent to the NF Service Producer by throttling part of this traffic locally at the NF Service Consumer, or diverting it to an alternative destination (another NF Service Producer where an alternative resource exists) that is not overloaded. If possible, traffic diversion shall always be preferred to throttling; the result of the throttling is a permanent rejection of the transaction.

If the client needs to abate a certain part of the available traffic, it shall do it based on the determined priority of each message.

Depending on regional/national requirements and network operator policy, requests related to priority traffic and emergency shall be the last to be throttled by the client, and shall be exempted from throttling due to overload control up to the point where the required traffic reduction cannot be achieved without throttling the priority requests.

The last status code (307) is intended to inform the NF Service Consumer about the availability of other endpoints where the service offered by the NF Service Producer is available, so the NF Service Consumer does not need to discard traffic locally.

### 6.4.2	HTTP Status Code "503 Service Unavailable"

This status code should be sent when the NF Service Producer undergoes an overload situation, and it needs to reject HTTP requests. The NF Service Producer may include detailed information about its status in the ProblemDetails JSON element, in the HTTP response body. Also, the HTTP header field "Retry-After" may be added in the response to convey an estimated time (in number of seconds) for the recovery of the service.

As for all 5xx status codes, this indicates a server-related issue (not limited to a specific client, or HTTP method), and it indicates that the server is incapable of performing the request.

Upon receipt of a "503 Service Unavailable" status code, the NF Service Consumer shall monitor the amount of rejected and timed-out traffic, in comparison to the accepted traffic by the NF Service Producer, and it shall abate (by divertion or throttling) the traffic sent to the NF Service Producer in such a way that the rate between accepted and rejected traffic improves with time, and eventually reaches a situation where the server accepts all requests once the overload status ceases at the server. The mechanism to achieve this is implementation-specific; Annex A contains a description of an example algorithm based on "adaptive throttling" of the traffic sent by the NF Service Consumer towards an NF Service Producer.

### 6.4.3	HTTP Status Code "429 Too Many Requests"

This status code may be sent, if supported by the server, when the NF Service Producer detects that a given NF Service Consumer is sending excessive traffic which, if continued over time, may lead to (or may increase) an overload situation in the NF Service Producer.

How the NF Service Producer detects that the incoming traffic comes from a same NF Service Consumer, and therefore subject to a given traffic rate limit, is out of the scope of this specification. The HTTP header field "Retry-After" may be added in the response to indicate how long the NF Service Consumer has to wait before making a new request.

As for all 4xx status codes, this indicates a client-related issue (not limited to a specific HTTP method), and it indicates that the client seems to be misbehaving.

### 6.4.4	HTTP Status Code "307 Temporary Redirect"

This status code should be sent when the NF Service Producer decides to redirect HTTP requests to another less loaded server, or HTTP/2 end point, to offload some part of the incoming traffic, with the goal to avoid entering (or to mitigate) an overload situation. The NF Service Producer shall not use it if it does not know the load status of the alternative server.

How the NF Service Producer becomes aware of the load levels of other servers or HTTP/2 end points is deployment-specific, and out of the scope of this specification. The URI for the temporary redirection shall be given by the Location header field of the response.

As for all 3xx status codes (redirection), this indicates a client-related action; the client shall be responsible of the detection of infinite redirection loops.

## 6.5	Support of Stateless NFs

### 6.5.1	General

A NF may become stateless by decoupling the "compute" resource and "storage" resource as specified in clause 4.1 of 3GPP TS 23.501 [3].

### 6.5.2	Stateless AMFs

#### 6.5.2.1	General

AMF may become stateless by storing the UE related information in the UDSF. Procedures for AMF planned removal or the AMF auto-recovery are specified in clauses 5.21.2.2 and 5.21.2.3 of 3GPP TS 23.501 [3].

#### 6.5.2.2	AMF as service consumer

1.	When the AMF subscribes to notifications from another NF Service Producer, the AMF shall provide its GUAMI to the NF Service Producer to enable the latter to discover AMFs within the AMF set, or information about a backup AMF, in addition to Callback URI in the subscription resource.

The AMF may also provide the name of the AMF service to which these notifications are to be sent (this service shall be one of the service produced by the AMF and registered in the NRF or a custom service registered in the NRF for the purpose of receiving these notifications);

NOTE 1:	Providing an AMF service name allows the NF Service Producer to find the endpoint address to deliver the notifications (see bullet 2). The provided AMF service might not use itself the information received in these notifications.

2.	A NF service producer may also use the Nnrf_NFDiscovery service to discover AMFs within an AMF set or backup AMF.

If the AMF provided the name of its service (see bullet 1), the NF Service Producer shall look up for the same AMF service from the AMFs within the AMF set or from backup AMF, and use endpoint addresses (i.e. IP addresses, transport and port information, or FQDN) of that service to send notifications (see bullet 6). Otherwise, the notifications shall be sent to an endpoint address registered in the NF Profile of the AMF.

NOTE 2:	The AMF can register different endpoint addresses in the NRF for different services.

3.	An NF service producer may subscribe to GUAMI changes using the AMFStatusChange service operation of the Namf_Communication service.

4.	An NF service producer may become aware of AMF changes (at the time of the AMF change or subsequently when sending signalling to the AMF) via Namf_Communication service AMFStatusChange Notifications, via HTTP Error response from the old or a wrongly selected new AMF, via link level failures (e.g. no response from the AMF), or via a notification from the NRF that the AMF has deregistered. The HTTP error response may be a 3xx redirect response pointing to a new AMF.

NOTE 3:.	AMFs are identified by GUAMIs. A GUAMI can point to an individual AMF or to some or all AMFs within an AMF set. If a GUAMI points to several AMFs, and the UE is served by one of those, all those AMFs can immediately handle communication for that service, and the NF service producer does not need to be aware which of those AMFs is serving a UE.

5.	When becoming aware of an AMF change, and the new AMF is not known, the NF service producer shall select an AMF within the AMF set or the possibly earlier received backup AMF.

6.	When becoming aware of an AMF change, the NF service producer shall exchange the authority part of the Notification URI with new AMF information and shall use that URI in subsequent communication.

7.	Each AMF within the AMF set shall be prepared to receive notifications from the NF service producer, by either handling the notification to the Notification URI constructed according to bullet 6 with the own address as authority part, or by replying with an HTTP 3xx redirect pointing to a new AMF, or by replying with another HTTP error.

8.	The NF service producer shall be prepared to receive updates to resources of the related service from any AMF within the set.

9.	If the UE moves to an AMF from a different AMF Set, or to an AMF from the same AMF set that does not support handling the notification as specified in bullet 7, the new AMF should update peer NFs with the new callback URI for the notification.

#### 6.5.2.3	AMF as service producer

1.	When AMF receives request to establish a service, it may provide information about a backup AMF in a suitable resource.

2.	NF service consumer may also use the Nnrf_NFDiscovery service to discover AMFs within an AMF set.

3.	An NF service consumer may subscribe to GUAMI changes using the AMFStatusChange service operation of the Namf_Communication service.

4.	An NF service consumer may become aware of AMF changes (at the time of the AMF change or subsequently when sending signalling to the AMF) via Namf_Communication service AMFStatusChange Notifications, via Error response from the old or a wrongly selected new AMF, via link level failures (e.g. no response from the AMF), or via a notification from the NRF that the AMF has deregistered. The HTTP error response may be a 3xx redirect response pointing to a new AMF.

NOTE.	AMFs are identified by GUAMIs. A GUAMI can point to an individual AMF or to some or all AMFs within an AMF set. If a GUAMI points to several AMFs, and the UE is served by one of those, all those AMFs can immediately handle communication for that service, and the NF service consumer does not need to be aware which of those AMFs is serving a UE.

5.	When becoming aware of an AMF change, and the new AMF is not known, the NF service consumer shall select an AMF within the AMF set or the possibly earlier received backup AMF.

6.	When becoming aware of an AMF change, the NF service consumer shall exchange the authority part of resource URIs with new AMF information and shall use that URI in subsequent communication.

7.	Each AMF within the AMF set shall be prepared to receive updates for resources from the NF service consumer, by either handling the updates to the resource URIs constructed according to step 6 with the own address as authority part, or by replying with an HTTP 3xx redirect pointing to a new AMF, or by replying with another HTTP error.

8.	For a service that includes notifications from the AMF, the NF service consumer shall be prepared to receive for the that service notifications from any AMF within the set.

NOTE:	If the UE moves to an AMF from a different AMF Set, or to an AMF from the same AMF set that does not support handling the updates as specified in bullet 7, but mechanisms exist to transfer information related to the resource to the AMF, service specific mechanism can exist.to notify the NF service consumer about the resource at the AMF. For instance, for the Namf_EventExposure service, information and an event subscription is transferred to the new AMF in such a manner and the new AMF will then report an event-change event.

### 6.5.3	Stateless NFs (for any 5GC NF type)

#### 6.5.3.1	General

A NF may become stateless by storing its contexts in the UDSF, or in the UDR for UDM, PCF, NEF.

A NF may also be deployed such that several stateless network function instances are present within a set of NF instances. Additionally, within a NF a NF service may have multiple instances grouped into a NF Service Set if they are interchangeable with each other because they share the same context data. See clause 5.21 of 3GPP TS 23.501 [3].

A UDM / AUSF / UDR / PCF group may consist of one or multiple UDM / AUSF / UDR / PCF sets.

#### 6.5.3.2	Stateless NF as service consumer

1.	When the NF service consumer subscribes to notifications from another NF service producer, the NF service consumer may provide a binding indication to the NF service producer as specified in clause 6.3.1.0 of 3GPP TS 23.501 [3] and clause 4.17.12.4 of 3GPP TS 23.502 [4], to enable the related notifications to be sent to an alternative NF within the NF set, in addition to providing the Callback URI in the subscription resource.

2.	A NF service producer or SCP may use the Nnrf_NFDiscovery service to discover NFs within an NF set.

3.	An NF service producer may become aware of a NF service consumer change, via receiving an updated binding information, or via an Error response to a notification, via link level failures (e.g. no response from the NF), or via a notification from the NRF that the NF has deregistered. The HTTP error response may be a 3xx redirect response pointing to a new NF.

4.	When becoming aware of an NF service consumer change, and the new NF service consumer is not known, the NF service producer shall select an NF according to the binding indication as specified in clause 6.3.1.0 of 3GPP TS 23.501 [3] and clause 4.17.12.4 of 3GPP TS 23.502 [4].

5.	When becoming aware of an NF service consumer change, the NF service producer or SCP shall exchange the authority part of the Notification URI with new NF service consumer information and shall use that URI in subsequent communication.

6.	Each NF within the NF set shall be prepared to receive notifications from the NF service producer, by either handling the notification to the Notification URI constructed according to bullet 5 with the own address as authority part, or by replying with an HTTP 3xx redirect pointing to a new NF, or by replying with another HTTP error.

Editor's Note: the behaviour when NFs of an NF Set are not able to receive Notification URI constructed according to bullet 5 is FFS, e.g. regarding how to update the NF producer with a new notification URI and how to handle and reply to an incoming notification request with an unknown URI.

7.	The NF service producer shall be prepared to receive updates to resources of the related service from any NF within the set.

#### 6.5.3.3	Stateless NF as service producer

1.	When the NF service producer receives a request to establish a service, it may provide binding information as specified in clause 6.3.1.0 of 3GPP TS 23.501 [3] and clause 4.17.12 of 3GPP TS 23.502 [4] to establish a binding between the NF service consumer and the NF service producer for subsequent related requests.

2.	The NF service consumer or SCP may use the Nnrf_NFDiscovery service to discover NFs within an NF set or NF services within a NF service set.

3.	An NF service consumer may become aware of a NF service producer change, by receiving an updated binding information, or via an Error response from the old or a selected new NF, via link level failures (e.g. no response from the NF), or via a notification from the NRF that the NF has deregistered. The HTTP error response may be a 3xx redirect response pointing to a new NF.

4.	When becoming aware of a NF service producer change, and the new NF service producer is not known, the NF service consumer or SCP shall select an NF or NF service according to the binding information as specified in clause 6.3.1.0 of 3GPP TS 23.501 [3].

5.	When becoming aware of a NF service producer change, the NF service consumer or SCP shall exchange the authority part of resource URIs with new NF service producer information and shall use that URI in subsequent communication.

6.	Each NF service producer within the NF set shall be prepared to receive updates for resources from the NF service consumer, by either handling the updates to the resource URIs constructed according to step 5 with the own address as authority part, or by replying with an HTTP 3xx redirect pointing to a new NF, or by replying with another HTTP error.

7.	For a service that includes notifications from the NF service producer, the NF service consumer shall be prepared to receive for that service notifications from any NF service producer within the set.

## 6.6	Extensibility Mechanisms

### 6.6.1	General

This clause describes the extensibility mechanisms supported in the Service-Based Architecture in 3GPP 5GC, such as feature negotiation, vendor-specific extensions, etc.

### 6.6.2	Feature negotiation

A versioning of services in the request URI shall be supported by 3GPP 5G APIs, but version upgrades shall only be applied for non-backward compatible changes or the introduction of new mandatory features.

The following mechanism to negotiate applicable optional features shall be used by 5G APIs. This supported feature mechanism shall be applied separately for each API.

For any API that defines resources, suitable resources associated to or representing the NF Service Consumer (e.g. a top-level resource or a sub-resource representing the NF Service Consumer) shall be identified in each API to support the negotiation of the applicable optional features between the NF Service Consumer and NF Service Producer for this resource. Each such resource for a 5G API shall contain an attribute (e.g. "supportedFeatures") of the SupportedFeatures data type defined in 3GPP TS 29.571 [13] containing a bitmask to indicate supported features. The features and their positions in that bitmask are defined separately for each API.

The HTTP client acting as NF service consumer shall include the attribute of the SupportedFeatures data type defined in 3GPP TS 29.571 [13] in the HTTP PUT or POST requests to create the resource associated to or representing the NF Service Consumer of 5G API. This attribute indicates which of the optional features defined for the corresponding service are supported by the HTTP client. The HTTP server shall determine the supported features for the corresponding resource by comparing the supported features indicated by the client with the supported features the HTTP server supports. Features that are supported both by the client and the server are supported for that resource. The HTTP server shall include the attribute of the SupportedFeatures data type defined in 3GPP TS 29.571 [13] indicating those features in the representation of the resource it returns to the HTTP client in the HTTP response confirming the creation of the resource.

The HTTP client acting as NF service consumer may include a query parameter (e.g. "supported-features") of the SupportedFeatures data type defined in 3GPP TS 29.571 [13] in HTTP GET requests to fetch resource(s) associated to the NF Service Consumer of 5G API. This query parameter indicates which of the optional features defined for the corresponding service are supported by the HTTP client. The HTTP server shall determine the supported features for the corresponding resource(s) by comparing the supported features indicated by the client with the supported features the HTTP server supports. Features that are supported both by the client and the server are supported for the resource(s); attributes or enumerated values that are only of relevance to a feature unsupported by the requested resource(s) should be ommitted from the representation sent in the response. The HTTP Server shall include the attribute of the SupportedFeatures data type defined in 3GPP TS 29.571 [13] indicating those features in the HTTP GET response, if supported by the API definition.

The supported features for a resource associated to or representing the NF Service Consumer shall also be applicable to all subordinate resources of that resource, and for all custom operations related to any of those resources. If any of those resources is used for the subscription to notifications (see clause 4.6.2 of 3GPP TS 29.501 [5]), the supported features shall also apply to those notifications.

Attributes used for the representation of a resource, particular values in enumerated data types, and/or procedural description can be marked to relate to a particular supported feature. Such attributes shall not be mandated in data structures. Such attributes or enumerated values shall only be sent and such procedures shall only be applied if the corresponding feature is supported.

Unknown attributes and values shall be ignored by the receiving entity. Unsupported query parameters shall be handled as specified in clause 5.2.9.

NOTE:	The sender may send such information before the supported features for a resource have been determined.

For an API that does not define any resources, only custom operations without associated resources or notifications without subscription will be used. For such APIs, if a feature negotiation is desired, the request and response bodies of a suitable custom operation or notification need to be defined in such a manner that an attribute of the SupportedFeatures data type defined in 3GPP TS 29.571 [13] is included. The client invoking that custom operation or notification shall indicate its supported features for that API within the corresponding HTTP request. The data structures to be included in the HTTP request as defined for that API, shall include the attribute of the SupportedFeatures data type defined in 3GPP TS 29.571 [13], preferably in the outermost data structure for cases where the body contains a complex structure with several layers of JSON objects. The server shall determine the supported features by comparing the supported features indicated by the client with its own supported features. Features that are supported both by the client and the server are supported for subsequent custom operations and notifications of that API. The server shall include the attribute of the SupportedFeatures data type defined in 3GPP TS 29.571 [13] indicating those features in the successful response to the custom operation or notification. The data structures to be included in the HTTP response as defined for that API, shall include the attribute of the SupportedFeatures data type defined in 3GPP TS 29.571 [13], preferably in the outermost data structure for cases where the body contains a complex structure with several layers of JSON objects. The client and server shall only use those supported features in subsequent communication of that API between each other until the supported feature negotiation performed as part of that communication yields a new result.

Additionally, a NF instance should register all the features it supports to the NRF, to enable NF Service Consumers to discover NF Service Producers supporting specific features.

Specific requirements for support of Indirect Communication with Delegated Discovery are specified in clause 6.10.6.

### 6.6.3	Vendor-specific extensions

Information elements sent on the 3GPP 5GC APIs should be extensible with vendor-specific data. The definition of JSON data structures using OpenAPI as Interface Definition Language (see OpenAPI Specification [9]) allows to extend by default any JSON object with additional member elements, as long as no specific directives are included in the schema definition preventing such extension (e.g., by setting "additionalProperties" to false).

NOTE 1:	The only JSON data types that can be extended, by defining additional members, are JSON objects; simple data types (and arrays of items of simple data types) cannot be extended in this way.

However, in order to avoid duplication of member names inside a same object (see 3GPP TS 29.501 [5], clause 5.2.4.2, for the requirement of uniqueness of member names in JSON objects), it is necessary to comply with a naming scheme for vendor-specific data elements, to avoid clashing names between vendors.

Vendor-specific member names in JSON objects shall be named in the following manner:

"vendorSpecific-nnnnnn": {

...

}

where the value "nnnnnn" is a fixed 6-digit string, using the IANA-assigned "SMI Network Management Private Enterprise Codes" [18] value associated to a given vendor, and padding with leading digits "0" to complete a 6-digit value.

NOTE 2:	The content (value) of those vendor-specific member elements, and their usage, is not to be defined by any of the 3GPP Technical Specifications. Also, the type of value assigned to these members is not defined by 3GPP, and therefore, they can be any of the types allowed in the JSON specification: objects, arrays, or simple types (string, number, Boolean, etc.). However, to allow future extensibility of these values, it is recommended that they are defined as objects.

EXAMPLE:	The vendor-specific member name for vendor "3GPP" would be:

"vendorSpecific-010415": {

...

}

### 6.6.4	Extensibility for Query parameters

New query parameters may be defined after the OpenAPI freeze of the first 3GPP release that contains an API.

A new feature should be defined in the API for any query parameter added in a new version of the API or for any new functionality resulting in defining new query parameter(s). A single feature may be defined, if appropriate, when adding multiple query parameters in a new version of the API.

Prior to using such a query parameter in an HTTP request, the NF Service Consumer should determine, if possible, whether the query parameter is supported by the NF Service Producer, using the feature negotiation mechanism specified in clause 6.6.2.

NOTE 1:	Not doing so could result in the NF Service Producer rejecting the request if it does not support the query parameters, see clause 5.2.9.

NOTE 2:	A NF Service Consumer can discover the features (and therefore the query parameters) supported by a NF Service Producer using the NRF, if the latter has registered the features it supports to the NRF.

If the NF Service Consumer includes the query parameter (e.g. "supported-features") of the SupportedFeatures data type defined in 3GPP TS 29.571 [13] in an HTTP GET request (see clause 6.6.2), the NF Service Producer shall include the attribute (e.g. "supportedFeatures") of the SupportedFeatures data type defined in 3GPP TS 29.571 [13] in the HTTP GET response, set as defined for HTTP responses in clause 6.6.2, if supported by the API definition.

NOTE 3:	This allows a NF Service Consumer to discover the features (and therefore the query parameters) supported by the NF Service Producer when the first interaction with the NF Service Producer is an HTTP GET request and the service was not discovered via the NRF, e.g. for a NF Discovery Request sent to the NRF.

NOTE 4:	Some APIs are designed to allow returning the attribute of the SupportedFeatures data type defined in 3GPP TS 29.571 [13] in the HTTP GET response, regardless of whether the query parameter of the SupportedFeatures data type defined in 3GPP TS 29.571 [13] is present in the request.

If a NF Service Consumer uses such a query parameter in an HTTP GET request without prior knowledge of whether it is supported by the NF Service Producer, the NF Service Consumer shall be prepared to receive a successful response that may not match all the query parameters sent in the request, and to act accordingly. The NF Service Consumer may use the attribute of the SupportedFeatures data type defined in 3GPP TS 29.571 [13] returned by the NF Service Producer in the HTTP GET response, if available, to determine the features (and thus query parameters) not supported by the NF Service Producer.

When defining new query parameters in a new version of an API, it needs to be checked that the addition of the query parameter does not cause backward compatibility problems with NF Service Producers complying with an earlier version of the API, e.g. if the query parameter is ignored in a HTTP GET request. Otherwise, it needs to be ascertained that the NF Service Consumer does not use such a query parameter without prior knowledge that it is supported by the NF Service Producer.

## 6.7	Security Mechanisms

### 6.7.1	General

The security mechanisms for service based interfaces are specified in clause 13 of 3GPP TS 33.501 [17].

Security Protection Edge Proxy (SEPP), as specified in 3GPP TS 33.501 [17], shall be used between service based interfaces across PLMNs. The NFs in a PLMN shall use the SEPP as a HTTP/2 proxy for the HTTP/2 messages that carry ":authority" pseudo header with a uri-host formatted as specified in clause 6.1.4.3.

### 6.7.2	Transport layer security protection of messages

As specified in clause 13.1 of 3GPP TS 33.501 [17], TLS shall be used for the security protection of messages at the transport layer for the service based interfaces if network security is not provided by other means.

### 6.7.3	Authorization of NF service access

As specified in clause 13.4.1 of 3GPP TS 33.501 [17] OAuth 2.0 (see IETF RFC 6749 [22]) may be used for authorization of NF service access. All NFs and the NRF shall support the OAuth 2.0 authorization framework with "Client Credentials" grant type as specified in clause 4.4 of IETF RFC 6749 [22] , except that there is no "Authorization" HTTP request header in the access token request.

The NRF shall act as the Authorization Server providing "Bearer" access tokens (IETF RFC 6750 [23]) to the NF service consumers to access the services provided by the NF service providers.

If an NF service (i.e API) receives an OAuth 2.0 access token in the "Authorization" HTTP request header field, the NF service shall validate the access token, its expiry and its access scope before allowing access to the requested resource, as specified in clause 7 of IETF RFC 6749 [22].

An NF service consumer shall support OAuth 2.0.

For request/response semantics service operations and for the subscribe and unsubscribe operations of subscribe/notify semantics service operations, an NF service consumer may use OAuth 2.0 for the authorization of the API access, based on local configuration.

When Oauth2 authorization is used, the NF service consumer shall use the token received from NRF as a "Bearer" token and include it in the Authorization header of the HTTP service requests, as described in IETF RFC 6750 [23] clause 2.1.

An NF service producer shall decide to accept or reject an API request without the OAuth2.0 access token in the "Authorization" HTTP request header field, based on local configuration.

If an NF service producer rejects an API request without the OAuth2.0 access token or an API request with an invalid OAuth2.0 access token, it shall return the HTTP "401 Unauthorized" status code together with the "WWW-Authenticate" header as specified in IETF RFC 7235 [21].

The scheme for challenge in the "WWW-Authenticate" header shall be set to "Bearer" in this case (IETF RFC 6750 [23]). The realm shall be set to:

-	the URI of the service (i.e API URI) for which the access without an OAuth2.0 access token failed, in the case of request / response service operations.

For the notify operation of subscribe/notify semantics service operations, in this release of this specification OAuth 2.0 access token is not used.

When an NF service consumer receives a "401 Unauthorized" status code with a "WWW-Authenticate" header containing "Bearer" as the scheme for challenge it shall not repeat the same request without an OAuth2.0 access token or with an access token that has been already used. The NF service consumer may repeat the same request with a new OAuth 2.0 access token.

NOTE:	If a NF service producer accepts a request without the OAuth 2.0 access token, based on local policy, it is assumed that such accesses are allowed based on trust relationships and hence full access to the resource as it would have been otherwise allowed, is provided.

### 6.7.4	Application layer security across PLMN

#### 6.7.4.1	General

HTTP/2 messages sent across the PLMN between the SEPPs shall follow the application layer security procedures specified in clause 13.2 of 3GPP TS 33.501 [17].

#### 6.7.4.2	N32 Procedures

As specified in clause 13.2 of 3GPP TS 33.501 [17], the following procedures shall be supported across N32

-	Capability Negotiation Request and Response;

-	Parameter Exchange Request and Response;

-	forwarding of the JOSE (see IETF RFC 7516 [25] and IETF RFC 7515 [26]) protected messages over N32.

Based on the capability negotiation and parameters exchanged between the SEPPs, the service based interface messages sent across N32 interface shall be subjected to JOSE based protection (see IETF RFC 7516 [25] and IETF RFC 7515 [26]) as specified in clause 13.2.4 of 3GPP TS 33.501 [17].

3GPP TS 29.573 [27] specifies protocol for the exchange of the messages described above over N32, the format of the JOSE (see IETF RFC 7516 [25] and IETF RFC 7515 [26]) protected messages and the procedure for forwarding of the JOSE protected messages over N32.

## 6.8	SBI Message Priority Mechanism

### 6.8.1	General

The primary usage of SBI Message Priority (SMP) is to provide guidance to 5GC NF acting as HTTP/2 clients or servers and HTTP/2 proxies when making throttling decisions related to overload control. The priority information may also be used for routing in proxies. Eventually a server may use the priority information to process higher-priority requests before lower-priority requests.

The SMP mechanism defined in this clause uses the "3gpp-Sbi-Message-Priority" custom HTTP header defined in clause 5.2.3.2.1 to set and carry the message priority between the client and the server.

NOTE 1:	The custom HTTP header enforces the message priority end to end between the client and the server through one or more proxies.

The SMP mechanism should also use the stream priority mechanism specified in IETF RFC 7540 [7] clause 5.3.

NOTE 2:	The stream priority enforces the message priority at the HTTP/2 connection level not end to end.

HTTP/2 clients, servers and proxies implementing SBIs shall support the custom HTTP header and should support the stream priority.

### 6.8.2	Message level priority

A client, proxy and server shall use the "3gpp-Sbi-Message-Priority" value (see clause 5.2.3.2.1) when setting or evaluating the priority of a message.

The client shall assign the request priority by adding the "3gpp-Sbi-Message-Priority" custom HTTP header (see clause 5.2.3.2.1) to the message and setting its value.

If the "3gpp-Sbi-Message-Priority" custom HTTP header is not present in a response message then the HTTP nodes shall use the priority indicated in the "3gpp-Sbi-Message-Priority" of the associated request message.

If the server wants to assign a different priority to the response message than the request one then the server shall assign the response priority by adding the "3gpp-Sbi-Message-Priority" custom HTTP header to the message and setting its value.

### 6.8.3	Stream priority

The purpose of HTTP/2 stream priority is to allow an endpoint to prioritize streams for transmitting frames when there is limited capacity for sending and to express how it would prefer its peer to allocate resources when managing concurrent streams. Setting the stream priority ensures a priority treatment to a message between the two endpoints of an HTTP/2 connection.

The stream priority applies to all frames in both directions. If it is not changed until the stream is closed then all frames of the request and response messages will have the same priority. A client assigns a priority for a request and the correspondent response by including dependency and Weight information in the HEADERS frame that opens the stream carrying the message.

The stream dependency shall be set to 0.

If the stream priority is used then the stream priority Weight is mapped from the custom HTTP header. The mapping algorithm shall respect the ordering of the priority. If message 1 has a priority of "x" and message 2 has a priority of "y" where "x" is lower than "y" then the Weight of the stream carrying the message 1 shall be higher than the Weight of the stream carrying the message 2.

If the server wants to change the priority of the response, it shall send a PRIORITY frame after the stream state became "half-closed (remote)" or shall send the priority information in the HEADERS frame.

### 6.8.4	Recommendations when defining SBI Message Priorities

The recommendations provided in this clause are compliant with clause 10 of IETF RFC 7944 [19]. They have been adapted to 5G services and Service Based Architecture.

The priorities defined for all messages across all SBIs used in an HTTP/2 administrative domain must be defined in a consistent and coordinated fashion, taking the default priority (see below for default priority values) into account.

The following are some guidelines to be considered when defining the SMPs to be used in SBA networks that support HTTP/2 nodes handling multiple services.

-	As with any prioritization scheme, it is possible for higher-priority messages to block lower-priority messages from ever being handled. In 5GC, this will often result in the messages being retried. This may result in more traffic than the network would have handled without use of the SMP mechanism.

One potential guideline to prevent unwanted starving of lower-priority messages is to have higher-priority messages represent a relatively small portion of messages handled by the 5GC under normal scenarios. Multimedia Priority Service (see 3GPP TS 23.501 [5] clause 5.16.5) and Mission Critical Service (see 3GPP TS 23.501 [5] clause 5.16.6) typically generate little traffic compared to the total traffic of a 5GC.

Multimedia Priority Service (MPS) or Mission Critical Service (MCX) requires the blocking of lower-priority services.

-	When setting priorities for Multimedia Priority Services, Mission Critical Services or Emergency calls, it is important to use the same priority values across all APIs and services exposed by the 5GC. For instance, if it is defined that the MPS priority level of [1; n] shall be assigned the priority of [k; k+n-1] in the same order then it shall be the same on all SBIs.

-	Messages related to MPS, MCX and Emergency calls may be ranked according to their priority (e.g., based on ARP priority level, 5QI priority level, MPS Priority) based on regional/national regulatory and operator policies when it is known by the application sending the message. Otherwise MPS and MCX should have higher priorities than Emergency calls. Emergency call related messages should have higher priority than the rest of the messages.

NOTE:	In some situations (e.g. REGISTRATION or SERVICE REQUEST procedure); it is possible to identify that the message belongs to a procedure of a high priority user without knowing the identity of the priority service. In such a case, all the messages sent over an SBI of these high priority procedures should be given the same SBI message priority.

-	Requests without the "3gpp-Sbi-Message-Priority" header shall be assigned the default priority value of "24".

-	Streams without priority shall be assigned a Stream Dependency of 0x0 and the default Weight of 16.

-	When defining priorities of the messages of a service it is needed to follow the same rules independently of the application, the SBI and the service.

-	When there is a series of request/response required to complete a procedure, it is appropriate to mark request/response occurrences that occur later in the series at a higher priority than those that occur early in the series.

-	The requests that establish new sessions should have a lower priority than the ones that update or end a session.

-	The requests that will result on deregistering users if they failed (authentication vector retrieval, update location…) shall have a higher priority than the ones of a non-registered user.

-	Request/response of optional procedure and delay tolerant services should have lower priority than those of mandatory procedures.

### 6.8.5	HTTP/2 client behaviour

The client sending a request shall determine its required priority according to 6.8.4. It shall include a "3gpp-Sbi-Message-Priority" header (see clause 5.2.3.2.1) indicating the required priority level in the request and shall prioritise the requests according to the required priority level. If the client also uses the stream priority at the HTTP/2 connection level then it shall map the header value into a Weight and include it in the HEADERS of the request message.

When the client receives a response with the "3gpp-Sbi-Message-Priority" header, it shall prioritise the received response according to the priority level received, otherwise according to the priority level of the corresponding request. This includes determining the order in which responses are handled and resources that are applied to the handling of the responses. The client may use the stream priority to determine how to prioritize the response at the HTTP/2 connection level.

### 6.8.6	HTTP/2 server behaviour

The server should use the "3gpp-Sbi-Message-Priority" header (see clause 5.2.3.2.1) and may use the stream priority information to determine how to handle the request. This includes determining the order in which requests are handled and resources that are applied to the handling of the request.

Servers should use "3gpp-Sbi-Message-Priority" value when making overload throttling decisions.

Servers should use stream priority information when making overload throttling decisions at the connection level.

When the priority of the response message needs to have a different value than the request, a server shall include a "3gpp-Sbi-Message-Priority" header in the response message which value is set to the response required priority level.

If a server has included "3gpp-Sbi-Message-Priority" header in the response message it may also set the stream priority as described in IETF RFC 7540 [7], via priority information in the HEADERS frame or in a PRIORITY frame.  In both cases the priority Weight value shall be mapped from the 3gpp-Sbi-Message-Priority" header value. When sending the priority information with a PRIORITY frame the server shall send it before sending the HEADERS frame of the response message. A server shall not send a PRIORITY frame after the HEADER one.

### 6.8.7	HTTP/2 proxy behaviour

A proxy should forward request and response without removing the "3gpp-Sbi-Message-Priority" header or changing its value.

While done only in exceptional circumstances, a proxy may modify priority information when relaying request and response by changing the "3gpp-Sbi-Message-Priority" value. For example, a SEPP may modify the priority set by a roaming partner.

Proxies should use the request priority information (respectively response priority information) according to the "3gpp-Sbi-Message-Priority" value  and may use the stream priority Weight value when making overload throttling decisions to a request (respectively a response).

Proxies may use the priority information according to the "3gpp-Sbi-Message-Priority" value and may use the stream priority Weight value when relaying a request or a response messages. This includes the selection of routes (only for the requests) and the ordering of messages relayed.

### 6.8.8	DSCP marking of messages

A client, proxy or server may prioritize traffic at IP level by placing messages into different traffic classes and marking them with an appropriate Differentiated Services Code Point (DSCP).

Multiple HTTP/2 connections between two HTTP/2 end points are necessary: one per DSCP value. All messages sent over a connection are assigned the same traffic class and receive the same DSCP marking. The "3gpp-Sbi-Message-Priority" value shall be considered in the selection of the appropriate connection to send the message.

## 6.9	Discovering the communication options supported by a target resource

### 6.9.1	General

The OPTIONS method, as described in clause 4.3.7 of IETF RFC 7231 [11], may be used by a NF Service Consumer to determine the communication options supported by a NF Service Producer for a target resource.

Clause 6.9.2 describes example communication options that may be discovered using the OPTIONS method.

### 6.9.2	Discoverable communication options

#### 6.9.2.2	Content-encodings supported in HTTP requests

Certain service operations may result in large HTTP request payloads, e.g. to register NF profiles in the NRF (see 3GPP TS 29.510 [8]) or to update the NSSF with the available S-NSSAIs supported by Tracking Areas (see 3GPP TS 29.531 [32]). Gzip coding (see IETF RFC 1952 [34]) may be supported to optimally reduce the payload size of HTTP requests in this case.

A NF Service Consumer may determine the content-encodings supported by the NF Service Producer in HTTP requests targeting a particular resource by:

-	sending an HTTP OPTIONS request targeting the resource of the NF Service Producer; and/or

-	receiving an "Accept-Encoding" header in HTTP responses from the NF Service Producer for requests targeting the resource.

A NF Service Producer that receives an HTTP OPTIONS request for a target resource shall include an "Accept-Encoding" header in the HTTP 200 OK response (see IETF RFC 7694 [33]), if specific content-encodings, e.g. Gzip coding (e.g. see IETF RFC 1952 [34]) are supported in HTTP requests targeting the resource.

A NF Service Producer that receives an HTTP request with a content-encoding that it does not support shall reject the request with the status code "415 Unsupported Media Type" and include an "Accept-Encoding" header in the response indicating the supported encodings in HTTP requests, as described in clause 3 of IETF RFC 7694 [33].

A NF Service Producer may include an "Accept-Encoding" header in the HTTP 2xx response for requests other than HTTP OPTIONS if specific content-encodings, e.g. Gzip coding (e.g. see IETF RFC 1952 [34]), are supported in HTTP requests targeting the resource, to optimize future interactions, e.g. when the request payload was big enough to justify use of a compression coding but the client did not do so.

## 6.10	Support of Indirect Communication

### 6.10.1	General

NF Service Consumers and NF Service Producers may support or be configured to use an Indirect Communication via SCP as specified in clauses 6.3 and 7.1 of 3GPP TS 23.501 [3]. This clause defines specific requirements to support Indirect Communication.

An SCP may be known to the NF (e.g. SCP based on independent deployment units) or not (e.g. SCP based on service mesh, with co-located NF and SCP within a same deployment unit).

NOTE:	See Annex G of 3GPP TS 23.501 [3] for deployment examples for the SCP.

Indirect Communication shall support the same level of security as Direct Communication. Security requirements for Indirect Communications are specified in 3GPP TS 33.501 [17]. TLS shall be used between the SCP and NFs, if network security is not provided by other means. When co-located within a same deployment unit, a NF and SCP may interact using HTTP.

Editor's Note: Security requirements for Indirect Communication are still being studied by 3GPP SA3 in 3GPP TR 33.855, see potential security requirements in clauses 4.1.10 and 4.1.11 of 3GPP TR 33.855 in particular. Clause 4.1.11.3 of 3GPP TR 33.855 also indicates that it is possible to assume physical security within a deployment unit. The above paragraph will be aligned with the final SA3 outcomes.

### 6.10.2	Routing Mechanism with SCP with TLS between NFs and SCP

#### 6.10.2.1	General

The routing mechanisms specified in clause 6.1 shall apply for Indirect Communication with the additions or modifications specified in this clause.

#### 6.10.2.2	HTTP/2 connection management

When TLS is used between NFs and the SCP, an end-to-end TLS association is not possible between the NF Service Consumer and NF Service Producer, since the SCP shall be able to read and modify the contents of the HTTP messages, for both indirect communication with or without delegated discovery. Therefore, separate HTTPS connections have to be set up between the HTTP client and the SCP, and between the SCP and the HTTP server. HTTP CONNECT requests shall not be used for Indirect Communication.

The NF and SCP shall manage the HTTP/2 connections as defined in clause 5.2.6.

#### 6.10.2.3	Connecting inbound

If the request is not satisfied by a local cache, the NF (acting as HTTP/2 client) shall connect inbound by establishing (or reusing) a connection to an available SCP as defined in clause 5.2 of IETF RFC 7230 [12] when sending HTTP/2 request.

When the SCP forwards the request to the HTTP server, the SCP (acting as an HTTP/2 client) shall connect inbound to an authority server for the target resource. For connecting inbound to an authority not in the same PLMN, the SCP shall connects to the Security Edge Protection Proxy.

#### 6.10.2.4	Pseudo-header setting

For Indirect Communication with or without delegated discovery, when sending a request to the SCP, the HTTP client shall set the pseudo-headers as follows:

-	":scheme"set to "https";

-	":authority" set to the FQDN or IP address of the SCP;

-	":path" including the path and query components of the target URI as specified in clause 6.1.4.

Additionally, for HTTP requests for which an HTTP client may cache responses (e.g. GET request), the HTTP client should include the cache key (ck) query parameter set to an implementation specific value that is bound to the target NF. See clause 6.10.2.6.

The HTTP client shall include the apiRoot of an authority server for the target resource, if available, in the 3gpp-Sbi-Target-apiRoot header (see clause 6.10.2.6).

When forwarding a request to the HTTP server, the SCP shall set the pseudo-headers as specified in clause 6.1, with the following additions:

-	the SCP shall modify the ":authority" HTTP/2 pseudo-header field to the FQDN or IP address of the target NF service instance. If the 3gpp-Sbi-Target-apiRoot header was received in the request, the SCP shall set the apiRoot of the request URI to the apiRoot received in the 3gpp-Sbi-Target-apiRoot header from the client, if the SCP does not (re)select a different HTTP server;

-	the SCP shall remove the cache key query parameter, if this parameter was received in the request.

#### 6.10.2.5	3gpp-Sbi-Target-apiRoot header setting

#### 6.10.2.5	3gpp-Sbi-Target-apiRoot header setting

For Indirect Communication with or without delegated discovery, the HTTP client shall include a 3gpp-Sbi-Target-apiRoot header set to the apiRoot of an authority server for the target resource, if available, in requests it sends to the SCP. In particular:

-	for Indirect Communication without Delegated Discovery, a service request sent to the SCP to create a resource shall include a 3gpp-Sbi-Target-apiRoot header set to the apiRoot of the selected NF Service Producer, if the NF Service Consumer has selected a specific NF service instance;

-	after a resource has been created, subsequent service requests sent to the SCP and targeting the resource shall include a 3gpp-Sbi-Target-apiRoot header set to the apiRoot received earlier from the NF Service Producer;

-	notifications or callbacks sent via the SCP shall include the apiRoot of the notification or callback URI (i.e. "https" scheme, the fixed string "://" and authority (host and optional port) as defined in IETF RFC 3986 [14]).

When forwarding the request to the HTTP server, the SCP shall:

-	remove the 3gpp-Sbi-Target-apiRoot header; and

-	set the apiRoot of the request URI to the apiRoot received in the 3gpp-Sbi-Target-apiRoot header from the client, if the SCP does not (re)select a different HTTP server.

#### 6.10.2.6	Cache key (ck) query parameter

The cache key (ck) query parameter may be used by cache systems in the NF service consumer and/or in the SCP in order to distinguish cache objects.

It shall be set to a string value that is linked to the apiRoot of the target NF, i.e. the same apiRoot shall always produce the same value for the content of the ck parameter. The ck parameter may contain e.g. a short compact hash value of the whole API root of the target NF.

The ck parameter shall only be used in HTTP requests between the NF service consumer and the SCP, it shall not be sent to the actual NF service producer.

The ck parameter is not part of the actual service definition and therefore it is not documented in OpenAPI specification files. It shall comply with the following OpenAPI definition:

paths:

/<resource>:

<method>:

parameters:

- name: ck

in: query

description: cache key parameter

schema:

type: string

### 6.10.2A	Routing Mechanism with SCP without TLS between NFs and SCP

#### 6.10.2A.1	Connecting inbound

When indirect communication is used and the NF sends an HTTP/2 request, the NF (acting as HTTP/2 client) shall connect directly to an authority for the target resource via an available SCP, which then acts as an "interception proxy" as defined in clause 2.5 of IETF RFC 3040 [36] and also referred to in clause 2.3 of IETF RFC 7230 [12].

#### 6.10.2A.2	HTTP/2 connection management

The NF shall manage the HTTP/2 connections as defined in clause 5.2.6.

#### 6.10.2A.3	Pseudo-header setting

The NF service consumer shall set the following pseudo-headers:

-	":scheme" pseudo-header shall be set to "http";

NOTE:	When the SCP is implemented using service mesh technology e.g. as described in Annex G.2 in 3GPP TS 23.501 [3], the SCP needs to be able to read the start line and the header fields of HTTP/2 messages, and https cannot be used by NF. In this case, mutual authentication and TLS between NF and SCP can be implicit by physical security; mutual authentication and TLS is between SCP interfaces.

-	":authority" pseudo-header shall be set as follows:

-	if delegated discovery is used and has not been performed yet by the SCP: set based on local configuration, depending on how the SCP is configured to use this pseudo-header for matching for traffic routing selection in addition to use other factors;

-	if delegated discovery is not used or has already been performed by the SCP: set as specified in clause 6.1.4.

-	":path" pseudo-header shall include the path and query components of the target URI as specified in clause 6.1.4.

### 6.10.3	NF Discovery and Selection for indirect communication with Delegated Discovery

#### 6.10.3.1	General

This clause specifies the requirements that shall apply when the discovery and associated selection of NF instances or NF service instances is delegated to an SCP (see clause 6.3 and Model D in Annex E of 3GPP TS 23.501 [3]).

#### 6.10.3.2	Conveyance of NF Discovery Factors

When the NF service consumer is configured to use delegated service discovery, it shall include in the HTTP/2 request message the necessary NF service discovery factors to be used by the SCP to perform NF service discovery procedures on behalf of the NF service consumer. The latter shall convey these NF service discovery factors using the"3gpp-Sbi-Discovery-*" request headers. How to set the values of these "3gpp-Sbi-Discovery-*" request headers is detailed in clause 5.2.3.2.7.

When receiving from the NF service consumer a service request containing "3gpp-Sbi-Discovery-*" request headers, and the SCP is to invoke NF service discovery towards the NRF to fulfil this task, then it shall take into account all the NF service discovery factors contained in the "3gpp-Sbi-Discovery-*" request headers. It is also possible for the SCP to be internally configured to fulfil these service discovery tasks without interacting with the NRF.

#### 6.10.3.3	Notifications corresponding to default notification subscriptions

An NF may register default notification subscriptions in its NF profile or NF services in the NRF for notifications the NF is prepared to consume, including for each type of notification the corresponding notification endpoint (i.e. callback URI).

NOTE:	This can be used e.g. by an AMF to discover the notification endpoint of other AMFs to forward N1 or N2 messages, or by an AMF to notify location information to a GMLC, or by an UDR to nofity data change or removal to an UDM.

The following procedures may be used to support notifications corresponding to default notification subscriptions:

-	an NF producer may perform a discovery request towards the NRF (possibly through an SCP) to discover default notification suscriptions of an NF consumer, and if so, send notifications to the corresponding notification endpoints, using routing mechanisms specified in clause 6.1; or

-	an NF producer may be configured with the types of notifications corresponding to default notification subscriptions it needs to generate, and send such notifications using delegated discovery, i.e with an SCP discovering and selecting an NF service consumer with a corresponding default notification subscription. To enable the latter, the NF producer shall include in the notification request:

-	the 3gpp-Sbi-Discovery-notification-type header set to the type of notification being set;

-	the 3gpp-Sbi-Discovery-target-nf-type header indicating the type of the consumer NF;

-	optionally, additional NF service discovery factors header to be used by the SCP to discover and select the consumer NF.

Editor's Note: It is FFS whether a new custom header, distinct from the custom discovery headers, should be defined to convey the notification type, e.g. to enable the SCP to also distinguish that the notification request corresponds to a default notification.

#### 6.10.3.4	Returning the NF Service Producer ID to the NF Service Consumer

When using delegated discovery, if and only if the NF Service Producer does not return a client binding indication in a service response creating a resource, the SCP shall include the 3gpp-Sbi-Producer-Id header in the HTTP response it forwards to the NF Service Consumer, containing the NF Instance ID of the NF Service Producer selected by the SCP (see clause 6.10.3.2).

NOTE 1:	This allows to support deployments where not all NF Service Producers have been upgraded to support the binding procedures.

NOTE 2:	In scenarios where the same NF Service Producer needs to be selected when creating new resources, e.g. when the AMF needs to establish a new PDU session towards the same SMF as the one selected for a previous PDU session, the NF Service Consumer can include the 3gpp-Sbi-Discovery-target-nf-instance-id header set to the NF Instance ID of the NF Service Producer in the request creating the new resource.

### 6.10.4	Authority and/or deployment-specific string in apiRoot of resource URI

For Indirect Communication with or without delegated discovery, the SCP may select or reselect the specific NF service instance towards which to send a request.

NOTE 1:	For Indirect Communication without delegated discovery, the SCP selects for instance a specific service instance within a NF set or NF Service Set selected by the NF Service Consumer.

Consequently, NF Service Consumers shall be capable to receive and process an authority and/or deployment-specific string in the apiRoot of the created resource URI that differs from the authority and/or deployment-specific string of the apiRoot of the Request URI.

If the NF Service Producer includes a relative URI (see IETF RFC 3986 [14]) in the "Location" header of an HTTP response creating a resource, the SCP shall resolve the URI reference using the target URI included in the HTTP POST request sent to the NF Service Producer as base URI, and return an absolute URI in the "Location" header in the HTTP response to the NF Service consumer, unless the SCP did not change the target URI when forwarding the HTTP POST request from the NF Service Consumer to the NF Service Producer.

NOTE 2:	The target URI can remain unchanged when forwarding an HTTP POST request from the NF Service Consumer to the NF Service Producer if indirect communication is used without delegated discovery and without TLS between the NF Service Consumer and the SCP, and the SCP uses the NF Service Producer instance selected by the NF Service Consumer.

### 6.10.5	NF / NF service instance selection for Indirect Communication without Delegated Discovery

#### 6.10.5.1	General

For Indirect Communication without Delegated Discovery, the NF Service Consumer performs the discovery by querying the NRF and does the selection of a NF Set or a specific NF instance in the NF Set. The selection of the NF service instance may be done by the NF Service Consumer or SCP.

The NF Service Consumer shall send its request to the SCP containing:

-	the 3gpp-Sbi-Target-apiRoot header set to the apiRoot of the selected NF service instance, if TLS is used between the NF and the SCP and if the NF Service Consumer has selected a specific NF service instance;

-	the identify of the selected NF Set ID or NF Service Set ID in the associated "3gpp-Sbi-Discovery-*" request headers (see clause 6.10.2.2), if the NF Service Consumer has selected a NF Set ID or a NF Service Set ID.

If the request does not include the apiRoot of a selected NF service instance, or if the SCP needs to reselect a different NF service instance, the SCP shall select an NF Service instance using the NF Set ID or NF Service Set ID received in the corresponding "3gpp-Sbi-Discovery-*" request headers.

The SCP shall then route the request to the selected NF service producer instance.

### 6.10.6	Feature negotiation for Indirect Communication with Delegated Discovery

The requirements specified in clause 6.6.2 for feature negotiation shall apply with the following additions.

With Indirect Communication with Delegated Discovery, the NF Service Consumer cannot discover the features supported by the NF Service Producer via the NRF.

The NF Service Consumer shall include in HTTP PUT or POST requests to create a resource that requires specific features to be supported by the NF Service Producer, the 3gpp-Sbi-Discovery-required-features header  set to the required features to be supported.

The SCP shall reject the request with the HTTP status code "400 Bad Request" and the protocol error "NF_DISCOVERY_FAILURE", if no NF Service Producer supporting the required features can be discovered.

### 6.10.7	Routing Mechanism with SCP (when not using TLS between NFs and SCP)

#### 6.10.7.1	Connecting inbound

When indirect communication is used and the NF sends an HTTP/2 request, the NF (acting as HTTP/2 client) shall connect directly to an authority for the target resource via an available SCP, which then acts as an "interception proxy" as defined in clause 2.5 of IETF RFC 3040 [36] and also referred to in clause 2.3 of IETF RFC 7230 [12].

#### 6.10.7.2	HTTP/2 connection management

The NF shall manage the HTTP/2 connections as defined in clause 5.2.6.

Editor's Note:	It is FFS whether additional requirement is needed regarding connection management for indirect communication mode.

#### 6.10.7.3	Pseudo-header setting

Editor's Note:	It is FFS how to set the ":scheme" and ":authority" pseudo-headers for indirect communication without delegated discovery.

The NF service consumer shall set the pseudo-headers as such:

-	":scheme" pseudo-header is set to "http";

NOTE:	When the SCP is implemented using service mesh technology e.g. as described in Annex G.2 in 3GPP TS 23.501 [3], the SCP needs to be able to read the start line and the header fields of HTTP/2 messages, and https cannot be used by NF. In this case, mutual authentication and TLS between NF and SCP can be implicit by physical security; mutual authentication and TLS is between SCP interfaces.

-	":authority" pseudo-header is set as follows:

-	if delegated discovery is used and has not been performed yet by the SCP: set based on local configuration, depending on how the SCP is configured to use this pseudo-header for matching for traffic routing selection in addition to use other factors;

-	if delegated discovery is not used or has already been performed by the SCP: set as specified in clause 6.1.4.

## 6.11	Detection and handling of late arriving requests

### 6.11.1	General

The procedures specified in this clause aim at handling more efficiently requests which may arrive late at upstreams entities, e.g. in networks experiencing processing or transport delays.

These procedures are optional to support. When supported, the use of these procedures is dependent on operator policy.

### 6.11.2	Detection and handling of requests which have timed out at the HTTP client

#### 6.11.2.1	General

This procedure enables an HTTP server which receives a request to know when the request times out at the HTTP client, and to stop further processing, at the receiver and further upstream NFs, a request which has timed out at the HTTP client.

The HTTP client and HTTP server shall be NTP synchronized. This procedure may be used between NFs pertaining to the same PLMN, and if allowed by operator policy, between NFs pertaining to different PLMNs.

#### 6.11.2.2	Principles

An HTTP client originating a request may include in the request the 3gpp-Sbi-Sender-Timestamp and the 3gpp-Sbi-Max-Rsp-Time headers indicating respectively the absolute time at which the request is originated and the maximum time period to complete the processing of the request; both headers together indicate the absolute time at which the request times out at the HTTP client.

Upon receipt of a request which contains the 3gpp-Sbi-Sender-Timestamp and the 3gpp-Sbi-Max-Rsp-Time headers, the HTTP server should check that the request has not already timed out at the originating HTTP client. The HTTP server may perform additional similar checks during the processing of the request, e.g. upon receipt of a response from the next upstream NF service.

Based on local configuration, the HTTP server may reject a request that is known to have timed out with the HTTP status code "504 Gateway Timeout" and the protocol error "TIMED_OUT_REQUEST"; it may alternatively drop the request. If so, the HTTP server should initiate the release of any resource it may have successfully created towards an upstream entity, to avoid hanging resources in the network.

## 6.12	Binding between an NF Service Consumer and an NF Service Resource

### 6.12.1	General

A binding indication for an NF Service Resource may be provided to an NF Service Consumer of the resource as part of the Direct or Indirection Communication procedures, to be used in subsequent related service requests. This allows the NF Service Resource owner to indicate that the NF Service Consumer, for a particular resource, should be bound to an NF service instance, NF instance, NF service set or NF set. See clause 6.3.1.0 of 3GPP TS 23.501 [3] and clause 4.17.12 of 3GPP TS 23.502 [4].

A binding may be established or updated as part of a:

1)	service response creating or modifying a resource, to be used for subsequent requests targeting this resource (see clause 4.17.12.2 of 3GPP TS 23.502 [4]), for any API that defines resources;

2)	service request, if the NF Service Consumer can also act as an NF Service Producer for later communication from the contacted NF Service Producer, to be used for subsequent service requests initiated by the contacted NF Service Producer (see clause 4.17.12.3 of 3GPP TS 23.502 [4]);

3)	service request creating an explicit or an implicit subscription, to be used for subsequent notification requests initiated by the NF Service Producer;

4)	service response creating an implicit subscription, to be used for subsequent operations on the subscription (see clause 4.17.12.4 of 3GPP TS 23.502 [4]).

Two types of binding information are defined to manage the binding between an NF Service Consumer and an NF Service Resource:

1)  A client binding conveys binding information for a resource which must be stored by the consumer (client) of that resource and used by the client to direct future requests to the resource.  When contained in a service request, the client binding information is associated with a resource owned by the NF Service Consumer for the current transaction.  When contained in a service response, the client binding information is associated with a resource owned by the NF Service Producer for the current transaction.

2)  A server binding conveys binding information to direct a request from a client to a server which has the resource context. Server binding information shall only be contained in a service request.

A service request may convey binding information for a resource using a 3gpp-Sbi-Client-Binding header and/or include binding information to influence routing of the request e.g. to an appropriate set of NF Service Producers or to an appropriate service set of the NF Service Producer using a 3gpp-Sbi-Server-Binding header.  A service response may convey binding information for a resource using a 3gpp-Sbi-Client-Binding header.

### 6.12.2	Binding created as part of a service response

An NF Service Producer may provide a client binding indication in a service response by including a 3gpp-Sbi-Client-Binding header (see clause 5.2.3.2.5) in the HTTP response with:

-	the binding level (bl) parameter indicating a binding to either a NF Service Instance, a NF Service Set, a NF Instance or a NF set;

-	at least one of the NF Service Instance (nfservinst), NF Service Set (nfservset), NF instance (nfinst) and NF Set (nfset) parameters, set to a NF Service Instance ID, NF Service Set ID, NF Instance ID and NF Set ID respectively, as described in Table 6.3.1.0-1 of 3GPP TS 23.502 [4].

The NF Service Consumer shall store the client binding indication received from the NF Service Producer and include it in a 3gpp-Sbi-Server-Binding header in subsequent related service requests. The NF Service Consumer or the SCP shall use this information for selecting or reselecting an NF Service Producer which has access to the NF Service Resource context, for direct or indirect communication respectively, as specified in clause 6.3.1.0 of 3GPP TS 23.501 [3].

NOTE:	The client binding indication can be part of an HTTP response with or without payload body, e.g. in a 204 No Content. The server binding indication can be part of an HTTP request with or without payload body, e.g. in a DELETE request.

### 6.12.3	Binding created as part of a service request

An NF Service Consumer may provide a client binding indication in a service request by including a 3gpp-Sbi-Client-Binding header (see clause 5.2.3.2.6) in an HTTP request with:

-	the binding level (bl) parameter indicating a binding to either a NF Service Instance, a NF Service Set, a NF Instance or a NF set;

-	at least one of the NF Service Instance (nfservinst), NF Service Set (nfservset), NF instance (nfinst) and NF Set (nfset) parameters, set to a NF Service Instance ID, NF Service Set ID, NF Instance ID and NF Set ID respectively, as described in Table 6.3.1.0-1 of 3GPP TS 23.502 [4].

The NF Service Producer shall store the client binding indication received from the NF Service Consumer and include it in a 3gpp-Sbi-Server-Binding header in subsequent service requests it sends, where the NF Service Consumer acts as an NF Service Producer. The NF Service Producer (when acting as a NF service consumer) or the SCP shall use this information for selecting or reselecting an NF Service Producer which has access to the original consumer's NF Service Resource context, for direct or indirect communication respectively, as specified in clause 6.3.1.0 of 3GPP TS 23.501 [3].

### 6.12.4	Binding for explicit or implicit subscription requests

A NF Service Consumer may provide a client binding indication in a service request creating an explicit or an implicit subscription by including a 3gpp-Sbi-Client-Binding header (see clause 5.2.3.2.6) in an HTTP request as specified in clause 6.12.3.

The NF Service Producer shall store the client binding indication received from the NF Service Consumer and include it in a 3gpp-Sbi-Server-Binding header in subsequent notification requests it sends to the NF Service Consumer (that acts as an HTTP server) related to this subscription. The NF Service Producer or the SCP shall use this information for selecting or reselecting an NF Service Consumer (HTTP server) which has access to the original consumer's NF Service Resource context, for direct or indirect communication respectively, as specified in clause 6.3.1.0 of 3GPP TS 23.501 [3].

The NF Service Producer may also provide a client binding indication in a service response creating an explicit or an implicit subscription by including a 3gpp-Sbi-Client-Binding header (see clause 5.2.3.2.5) in the HTTP response as specified in clause 6.12.2. If the service request creates a resource and a subscription, the client binding returned in the HTTP response shall apply to both the NF Service Resource and the subscription, i.e. the created resource and subscription shall be bound to the same (service) set of producers or producer instance. The NF Service Consumer shall store the client binding indication received from the NF Service Producer and include it in a 3gpp-Sbi-Server-Binding header in subsequent related service requests as specified in clause 6.12.2.

Annex A (informative):
Client-side Adaptive Throttling for Overload Control

This clause contains an example algorithm to make an NF Service Consumer adjust the traffic rate sent to an NF Service Producer based on the number of received "rejects" of HTTP requests with a status code "503 Service Unavailable", or requests that have timed-out and the response was never received. This algorithm is described in the book "Betsy Beyer, et al; Google: Site Reliability Engineering" (https://landing.google.com/sre/book.html), chapter 21, "Handling Overload".

NOTE:	The reference link provided to the book can change and hence the name of the book is expected to be used for referring to the latest edition.

Each client (NF Service Consumer) keeps track of the following counters during a certain time window:

-	Requests: The number of requests that the client (NF Service Consumer) needs to handle. Under normal operation (no overload), all these requests are sent to the server (NF Service Producer). Under an overload situation, part of these requests are locally rejected by the client (and not sent to the server), and the rest of the requests are sent to the server.

-	Accepts: The number of requests accepted by the server (i.e., requests for which a response has been effectively received at the client, with a status code other than "503 Service Unavailable").

When there is no server overload, these values are equal.

When there is an overload status in the server, the rate between "Accepts" and "Requests" decreases progressively. When this rate falls below a certain point (given by an algorithm parameter named "K"), the client shall start dropping some requests locally and not send them to the server.

The local rejection of requests can be done by calculating a "Client request rejection probability", as:

So, for example, assuming that the K parameter is set at 1.5:

-	if the server accepts >67% of the traffic, and rejects <33% of the traffic, the client does not take any throttling action, and keeps sending to the server all the traffic it has available for processing

-	if, during a first time-window, the server accepts, e.g., only 60% of the requests, and rejects 40% due to overload, the application of this algorithm implies that the client must drop locally 10% of the requests (probabilistically), and only send to the server the remainder 90% of its traffic.

-	if, during a second time-window, the client keeps the same amount of available traffic to handle, but the server continues rejecting requests with same rate as before (40%) of the received requests, the application of the algorithm again, results in increasing the drop rate to 14.5%, and sending to the server only 85.5% of the available traffic.

The value of the parameter K, along with the size of the time window during which the total number of "requests" and "accepts" is accounted for, has a fundamental role on how the algorithm behaves. If K is higher, the algorithm is more "permissive", and the client does not start dropping requests locally until the rejection rate is higher (e.g., >50%, for K = 2); if K is lower, the algorithm is more "aggressive", and the client starts dropping requests sooner (e.g., K = 1.1 implies to start dropping requests as soon as the server rejects >10% of the requests).

Annex B (normative):
3gpp-Sbi-Callback Types

This annex specifies the list of allowed 3GPP SBI callback type values for the 3gpp-Sbi-Callback HTTP custom header specified in clause 5.2.3.2.3. Only the callbacks that are invoked across PLMN are specified.

Table B-1: Values for 3gpp-Sbi-Callback Custom HTTP Header

| Value for 3gpp-Sbi-Callback Custom HTTP Header | Reference |
| --- | --- |
| "Nsmf_PDUSession_Update" | 3GPP TS 29.502 [28], Clause 5.2.2.8.3.2, 5.2.2.8.3.3, 5.2.2.8.3.4 and 5.2.2.8.3.5. |
| "Nsmf_PDUSession_StatusNotify" | 3GPP TS 29.502 [28], Clause 5.2.2.10. |
| "Nudm_SDM_Notification" | 3GPP TS 29.503 [29], Clause 6.1.5.2 |
| "Nudm_UECM_DeregistrationNotification" | 3GPP TS 29.503 [29], Clause 6.2.5.2 |
| "Nudm_UECM_PCSCFRestorationNotification" | 3GPP TS 29.503 [29], Clause 6.2.5.3 |
| "Nnrf_NFManagement_NFStatusNotify" | 3GPP TS 29.510 [8], Clause 6.1.5.2. |
| "Namf_EventExposure_Notify" | 3GPP TS 29.518 [31], Clause 6.2.5.2. |
| "Npcf_UEPolicyControl_UpdateNotify" | 3GPP TS 29.525 [35], Clauses 4.2.4, 5.5.2 and 5.5.3. |
| "Nnssf_NSSAIAvailability_Notification" | 3GPP TS 29.531 [32], Clause 6.2.5.2 |

Annex C (informative):
Internal NF Routing of HTTP Requests

The internal details of the architecture of a Network Function instance is out of the scope of 3GPP and are entirely implementation-specific. This annex describes how an instance of an NF Service Producer can route internally HTTP requests received on a given Service-Based Interface.

Figure C-1 illustrates an example component architecture where incoming HTTP requests are received and processed in a component named as "Ingress Proxy" module and route them to the appropriate computing resource in the NF.

Figure C-1: Internal message routing inside NF Service Producer

The Ingress Proxy may parse any of the different components in the HTTP request, but typically it may parse the path of the URI (i.e. the :path pseudo-header in the HTTP/2 request). Parsing of other component in the request message, such as the HTTP body, is also possible but it is not desirable as it requires the parsing of the entire body (i.e. a JSON document) which is a much more computing-intensive task.

The path component of the URI contains the service name of the requested SBA service, so frequently the routing is done based on this component.

It is also frequent to inspect other components of the path (i.e. path segments), to do a more fine-grained routing and direct requests done on a specific HTTP resource(s) towards a given computing resource(s).

It can be noted that the path components used to determine the target computing resource typically do not need to be statically defined but are frequently defined in terms of "variables", or placeholders, similarly to how they are defined in the OpenAPI specification language (a mechanism usually known as "path templating"). See: https://github.com/OAI/OpenAPI-Specification/blob/master/versions/3.0.0.md#path-templating

Annex D (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2017-10 | CT4#80 | C4-175246 |  |  |  | TR skeleton | 0.1.0 |
| 2017-10 | CT4#80 | C4-175390 |  |  |  | Implementation of pCRs agreed at CT4#80. | 0.2.0 |
| 2017-12 | CT4#81 | C4-176433 |  |  |  | Implementation of pCRs agreed at CT4#81. | 0.3.0 |
| 2018-01 | CT4#82 | C4-181387 |  |  |  | Implementation of pCRs agreed at CT4#82. | 0.4.0 |
| 2018-03 | CT4#83 | C4-182430 |  |  |  | Implementation of pCRs agreed at CT4#83. | 0.5.0 |
| 2018-03 | CT#79 | CP-180028 |  |  |  | Presented for information | 1.0.0 |
| 2018-04 | CT4#84 | C4-183512 |  |  |  | Implementation of pCRs agreed at CT4#84. | 1.1.0 |
| 2018-05 | CT4#85 | C4-184617 |  |  |  | Implementation of pCRs agreed at CT4#85. The following pCRs are implemented. C4-184589, C4-184580, C4-184347, C4-184590, C4-184338, C4-184591, C4-184349, C4-184490, C4-184350, C4-184579, C4-184577 and C4-184498. | 1.2.0 |
| 2018-06 | CT#80 | CP-181098 |  |  |  | Presented for approval | 2.0.0 |
| 2018-06 | CT#80 |  |  |  |  | Apprvoed in CT#80 | 15.0.0 |
| 2018-09 | CT#81 | CP-182053 | 0001 | 4 | F | OAuth2.0 Clarifications | 15.1.0 |
| 2018-09 | CT#81 | CP-182053 | 0002 | 2 | B | Specifying N32 Aspects | 15.1.0 |
| 2018-09 | CT#81 | CP-182053 | 0003 | 1 | F | Determination of SBI message priorities | 15.1.0 |
| 2018-09 | CT#81 | CP-182053 | 0004 | 5 | F | Stateless AMF support | 15.1.0 |
| 2018-09 | CT#81 | CP-182053 | 0005 |  | F | Support of status code "501 Not implemented" | 15.1.0 |
| 2018-09 | CT#81 | CP-182053 | 0006 | 2 | B | Default port number | 15.1.0 |
| 2018-12 | CT#82 | CP-183011 | 0009 | 3 | F | Keep-alive on idle HTTP connections | 15.2.0 |
| 2018-12 | CT#82 | CP-183011 | 0010 | 1 | F | Stream Concurrency for overload control | 15.2.0 |
| 2018-12 | CT#82 | CP-183011 | 0011 | 1 | F | Update of missing status code 429 | 15.2.0 |
| 2018-12 | CT#82 | CP-183011 | 0012 | 1 | F | Correction of the entity upon which content encoding is performed | 15.2.0 |
| 2018-12 | CT#82 | CP-183011 | 0013 | 2 | F | Custom header for notifications | 15.2.0 |
| 2018-12 | CT#82 | CP-183011 | 0014 | 3 | F | Routing across PLMN | 15.2.0 |
| 2018-12 | CT#82 | CP-183011 | 0015 |  | F | HTTP status code "406 Not Acceptable" | 15.2.0 |
| 2018-12 | CT#82 | CP-183011 | 0016 | 1 | F | Support of HTTP status code "414 URI Too Long" | 15.2.0 |
| 2018-12 | CT#82 | CP-183011 | 0018 |  | F | HTTP status code "414 URI Too Long" on PUT method | 15.2.0 |
| 2018-12 | CT#82 | CP-183011 | 0020 | 1 | F | Correction of Stream Priority in HTTP/2 Server Behaviour | 15.2.0 |
| 2018-12 | CT#82 | CP-183194 | 0022 | 2 | F | Change 403 to mandatory and clarify conditional headers | 15.2.0 |
| 2018-12 |  |  |  |  |  | Change history annex number corected | 15.2.1 |
| 2019-03 | CT#83 | CP-190016 | 0023 | 1 | F | Extensibility mechanism for Query parameters | 15.3.0 |
| 2019-03 | CT#83 | CP-190016 | 0024 | 1 | F | Bearer Tokens | 15.3.0 |
| 2019-03 | CT#83 | CP-190016 | 0025 | 1 | F | Handling of Incorrect IEs | 15.3.0 |
| 2019-03 | CT#83 | CP-190016 | 0026 | 2 | F | Clarification on Handling of Incorrect Optional IEs | 15.3.0 |
| 2019-03 | CT#83 | CP-190016 | 0027 |  | F | Status Codes | 15.3.0 |
| 2019-06 | CT#84 | CP-191027 | 0030 | 1 | F | Content-encodings supported in HTTP requests | 15.4.0 |
| 2019-06 | CT#84 | CP-191027 | 0031 | 3 | F | Missing Application Error Codes | 15.4.0 |
| 2019-06 | CT#84 | CP-191027 | 0032 | 2 | F | Correction on Feature Negotiation | 15.4.0 |
| 2019-06 | CT#84 | CP-191027 | 0037 | 1 | F | Allowed values of 3gpp-Sbi-Callback header field | 15.4.0 |
| 2019-06 | CT#84 | CP-191027 | 0038 | 1 | F | Adding the Control Plane interfaces that support service based interface | 15.4.0 |
| 2019-06 | CT#84 | CP-191055 | 0033 | 1 | B | Support of Indirect Communication (General) | 16.0.0 |
| 2019-06 | CT#84 | CP-191055 | 0034 | 2 | B | Support of Indirect Communication (Routing to SCP) | 16.0.0 |
| 2019-06 | CT#84 | CP-191055 | 0035 | 1 | B | Support of Indirect Communication (NF discovery and selection) | 16.0.0 |
| 2019-06 | CT#84 | CP-191057 | 0036 | 2 | B | Partially implemented PATCH | 16.0.0 |
| 2019-09 | CT#85 | CP-192194 | 0040 | 2 | B | Support of stateless NFs | 16.1.0 |
| 2019-09 | CT#85 | CP-192194 | 0041 | 1 | B | Routing mechanisms for Indirect Communication | 16.1.0 |
| 2019-09 | CT#85 | CP-192194 | 0042 | 1 | B | Routing extensions for Indirect Communication | 16.1.0 |
| 2019-09 | CT#85 | CP-192194 | 0043 | - | B | Authority and/or deployment-specific string in apiRoot of resource URI for Indirect Communication | 16.1.0 |
| 2019-09 | CT#85 | CP-192194 | 0044 | 1 | B | NF / NF service instance selection for Indirect Communication without Delegated Discovery | 16.1.0 |
| 2019-09 | CT#85 | CP-192194 | 0045 | - | B | Feature negotiation for Indirect Communication with Delegated Discovery | 16.1.0 |
| 2019-09 | CT#85 | CP-192194 | 0053 | 2 | B | Routing for indirect Communication with HTTP between NFs and SCP | 16.1.0 |
| 2019-09 | CT#85 | CP-192123 | 0046 | - | B | Timestamp in HTTP messages | 16.1.0 |
| 2019-09 | CT#85 | CP-19212 | 0047 | 1 | B | Handling of timed out requests | 16.1.0 |
| 2019-09 | CT#85 | CP-19212 | 0049 | 1 | B | Indicating partially implemented PATCH | 16.1.0 |
| 2019-09 | CT#85 | CP-19212 | 0052 | 2 | F | Adding the Control Plane interfaces that support service based interface | 16.1.0 |
| 2019-12 | CT#86 | CP-193036 | 0059 | 1 | F | Load Info used for Load Control | 16.2.0 |
| 2019-12 | CT#86 | CP-193036 | 0062 | - | F | Informative description of internal NF routing of HTTP messages | 16.2.0 |
| 2019-12 | CT#86 | CP-193057 | 0039 | 5 | B | Binding between NF Service Consumer and NF Service Producer | 16.2.0 |
| 2019-12 | CT#86 | CP-193057 | 0056 | 2 | B | Routing of Indirect Communication with TLS between NFs and SCP | 16.2.0 |
| 2019-12 | CT#86 | CP-193057 | 0057 | 2 | B | Routing of Indirect Communication without TLS between NFs and SCP | 16.2.0 |
| 2019-12 | CT#86 | CP-193057 | 0060 | 3 | B | Conveyance of Delegated Discovery Parameters in HTTP/2 Headers | 16.2.0 |
| 2019-12 | CT#86 | CP-193057 | 0064 | 1 | B | Binding indication for subscribe/notify | 16.2.0 |
| 2019-12 | CT#86 | CP-193057 | 0065 | - | B | General Introduction for Delegated Discovery | 16.2.0 |
| 2019-12 | CT#86 | CP-193057 | 0067 | 1 | B | Handling of relative URIs with indirect communication | 16.2.0 |
| 2019-12 | CT#86 | CP-193057 | 0068 | 2 | B | Use of 3gpp-Sbi-Target-apiRoot header in HTTP requests from NFs to SEPP | 16.2.0 |
| 2019-12 | CT#86 | CP-193057 | 0069 | 2 | B | Returning NF Producer ID to NF Consumer when using Delegated Discovery | 16.2.0 |
| 2019-12 | CT#86 | CP-193057 | 0066 | 1 | B | Handling of default notification subscriptions with Delegated Discovery | 16.2.0 |
| 2019-12 | CT#86 | CP-193063 | 0063 | 1 | F | Clarification of Cause MANDATORY_IE_INCORRECT | 16.2.0 |