| 3GPP TS 29.502 V17.2.0 (2021-09) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; 5G System; Session Management Services; Stage 3 (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	9

1	Scope	10

2	References	10

3	Definitions and abbreviations	12

3.1	Definitions	12

3.2	Abbreviations	12

4	Overview	12

4.1	Introduction	12

5	Services offered by the SMF	13

5.1	Introduction	13

5.2	Nsmf_PDUSession Service	14

5.2.1	Service Description	14

5.2.2	Service Operations	15

5.2.2.1	Introduction	15

5.2.2.2	Create SM Context service operation	16

5.2.2.2.1	General	16

5.2.2.2.2	EPS to 5GS Idle mode mobility using N26 interface (with or without data forwarding)	19

5.2.2.2.3	EPS to 5GS Handover Preparation using N26 interface	21

5.2.2.2.4	I-SMF Insertion, Change or Removal during Xn based Handover	22

5.2.2.2.5	I-SMF Insertion, Change or Removal during N2 based Handover	22

5.2.2.2.6	Service Request with I-SMF insertion/change/removal or with V-SMF change	23

5.2.2.2.7	Registration procedure for a UE with a PDU session with I-SMF insertion, change and removal	23

5.2.2.2.8	SMF Context Transfer procedure, LBO or no Roaming, no I-SMF	24

5.2.2.2.9	I-SMF Context Transfer procedure	24

5.2.2.2.10	Handover between 3GPP and non-3GPP accesses with I-SMF insertion/removal or V-SMF change	25

5.2.2.2.11	I-SMF/V-SMF restoration procedure	25

5.2.2.2.12	I-SMF selection per DNAI	26

5.2.2.3	Update SM Context service operation	27

5.2.2.3.1	General	27

5.2.2.3.2	Activation and Deactivation of the User Plane connection of a PDU session	29

5.2.2.3.2.1	General	29

5.2.2.3.2.2	Activation of User Plane connectivity of a PDU session	29

5.2.2.3.2.3	Deactivation of User Plane connectivity of a PDU session	31

5.2.2.3.2.4	Changing the access type of a PDU session from non-3GPP access to 3GPP access during a Service Request procedure	32

5.2.2.3.3	Xn Handover	32

5.2.2.3.4	N2 Handover	34

5.2.2.3.4.1	General	34

5.2.2.3.4.2	N2 Handover Preparation	34

5.2.2.3.4.3	N2 Handover Execution	36

5.2.2.3.4.4	N2 Handover Cancellation	37

5.2.2.3.5	Handover between 3GPP and untrusted non-3GPP access procedures	38

5.2.2.3.5.1	General	38

5.2.2.3.5.2	Handover of a PDU session without AMF change or with target AMF in same PLMN	38

5.2.2.3.6	Inter-AMF change or mobility	39

5.2.2.3.7	RAN Initiated QoS Flow Mobility	39

5.2.2.3.8	EPS to 5GS Handover using N26 interface	40

5.2.2.3.8.1	General	40

5.2.2.3.8.2	EPS to 5GS Handover Preparation	40

5.2.2.3.8.3	EPS to 5GS Handover Execution	41

5.2.2.3.8.4	EPS to 5GS Handover Cancellation	41

5.2.2.3.8.5	EPS to 5GS Handover Failure	41

5.2.2.3.9	5GS to EPS Handover using N26 interface	41

5.2.2.3.9.1	General	41

5.2.2.3.9.2	Data forwarding tunnels setup during 5GS to EPS handover	41

5.2.2.3.9.3	Indirect data forwarding tunnels removal for 5GS to EPS handover cancellation or failure	42

5.2.2.3.10	P-CSCF Restoration Procedure via AMF	43

5.2.2.3.11	AMF requested PDU Session Release due to duplicated PDU Session Id	43

5.2.2.3.12	AMF requested PDU Session Release due to slice not available	43

5.2.2.3.13	Indirect Data Forwarding Tunnel establishment during N2 based Handover with I-SMF	43

5.2.2.3.13A	Indirect Data Forwarding Tunnel removal during N2 based Handover with I-SMF	44

5.2.2.3.14	Request to forward buffered downlink data packets at I-UPF	45

5.2.2.3.15	Connection Suspend procedure	46

5.2.2.3.16	Connection Resume in CM-IDLE with Suspend procedure	46

5.2.2.3.17	AMF requested PDU Session Release due to Network Slice-Specific Authentication and Authorization failure or revocation	47

5.2.2.3.18	5GS to EPS Idle mode mobility using N26 interface with data forwarding	47

5.2.2.3.19	AMF requested PDU Session Release due to Control Plane Only indication associated with PDU Session is not applicable any longer	48

5.2.2.4	Release SM Context service operation	48

5.2.2.4.1	General	48

5.2.2.5	Notify SM Context Status service operation	50

5.2.2.5.1	General	50

5.2.2.6	Retrieve SM Context service operation	53

5.2.2.6.1	General	53

5.2.2.7	Create service operation	55

5.2.2.7.1	General	55

5.2.2.7.2	EPS to 5GS Idle mode mobility	58

5.2.2.7.3	EPS to 5GS Handover Preparation	58

5.2.2.7.4	N2 Handover Preparation with I-SMF Insertion	59

5.2.2.7.5	Xn Handover with I-SMF Insertion	59

5.2.2.7.6	I-SMF/V-SMF restoration procedure	60

5.2.2.8	Update service operation	60

5.2.2.8.1	General	60

5.2.2.8.2	Update service operation towards H-SMF or SMF	61

5.2.2.8.2.1	General	61

5.2.2.8.2.2	UE or network (e.g. AMF, V-SMF, I-SMF) requested PDU session modification	62

5.2.2.8.2.3	UE requested PDU session release	63

5.2.2.8.2.4	EPS to 5GS Handover Execution	63

5.2.2.8.2.5	Handover between 3GPP access and untrusted or trusted non-3GPP access	63

5.2.2.8.2.6	P-CSCF Restoration Procedure via AMF	64

5.2.2.8.2.7	Addition of PSA and BP or UL CL controlled by I-SMF	64

5.2.2.8.2.8	Removal of PSA and BP or UL CL controlled by I-SMF	65

5.2.2.8.2.9	Change of PSA for IPv6 multi-homing or UL CL controlled by I-SMF	65

5.2.2.8.2.10	PDU Session modification with I-SMF or V-SMF change	66

5.2.2.8.2.11	Sending by I-SMF of N4 notifications related with traffic usage reporting	66

5.2.2.8.2.12	N2 Handover Execution with I-SMF Insertion	66

5.2.2.8.2.13	N2 Handover Cancellation with I-SMF Insertion	67

5.2.2.8.2.14	EPS to 5GS Handover Cancellation	67

5.2.2.8.2.15	5G-AN requested PDU session resource release	68

5.2.2.8.2.16	Xn Handover with or without I-SMF or V-SMF Change	68

5.2.2.8.2.17	EPS to 5GS Handover Failure	68

5.2.2.8.2.18	EPS Bearer ID revocation	69

5.2.2.8.2.19	Network requested PDU session release	69

5.2.2.8.2.20	N2 Handover Execution with or without I-SMF or V-SMF Change	69

5.2.2.8.3	Update service operation towards V-SMF or I-SMF	69

5.2.2.8.3.1	General	69

5.2.2.8.3.2	Network (e.g. H-SMF, SMF) requested PDU session modification	70

5.2.2.8.3.3	Network (e.g. H-SMF, SMF) requested PDU session release	71

5.2.2.8.3.4	Handover between 3GPP and untrusted non-3GPP access, from 5GC-N3IWF to EPS or from 5GS to EPC/ePDG	72

5.2.2.8.3.5	EPS Bearer ID assignment	72

5.2.2.8.3.6	Addition of PSA and BP or UL CL controlled by I-SMF	72

5.2.2.8.3.7	Removal of PSA and BP or UL CL controlled by I-SMF	73

5.2.2.8.3.8	Change of PSA for IPv6 multi-homing or UL CL controlled by I-SMF	73

5.2.2.8.3.9	Policy update procedures with an I-SMF	74

5.2.2.8.3.10	Simultaneous change of PSA and BP or UL CL controlled by I-SMF	74

5.2.2.9	Release service operation	75

5.2.2.9.1	General	75

5.2.2.10	Notify Status service operation	76

5.2.2.10.1	General	76

5.2.2.11	Send MO Data service operation	78

5.2.2.11.1	General	78

5.2.2.12	Transfer MO Data service operation	79

5.2.2.12.1	General	79

5.2.2.13	Transfer MT Data service operation	79

5.2.2.13.1	General	79

5.2.2.14	Retrieve service operation	80

5.2.2.14.1	General	80

5.2.3	General procedures	81

5.2.3.1	Transfer of NAS SM information between UE and H-SMF for Home Routed PDU sessions	81

5.2.3.1.1	General	81

5.2.3.1.2	V-SMF Behaviour	81

5.2.3.1.3	H-SMF Behaviour	82

5.2.3.2	Transfer of NAS SM information between UE and SMF for PDU sessions with an I-SMF	82

5.2.3.2.1	General	82

5.2.3.3	Detection and handling of late arriving requests	82

5.2.3.3.1	Handling of requests which collide with an existing SM context or PDU session context	82

5.2.3.3.1.1	General	82

5.2.3.3.1.2	Principles	82

5.2.3.3.2	Detection and handling of requests which have timed out at the HTTP client	83

5.2.3.3.2.1	General	83

5.2.3.4	UE Location Information	83

6	API Definitions	83

6.1	Nsmf_PDUSession Service API	83

6.1.1	API URI	83

6.1.2	Usage of HTTP	84

6.1.2.1	General	84

6.1.2.2	HTTP standard headers	84

6.1.2.2.1	General	84

6.1.2.2.2	Content type	84

6.1.2.3	HTTP custom headers	85

6.1.2.3.1	General	85

6.1.2.3.2	3gpp-Sbi-Origination-Timestamp	85

6.1.2.4	HTTP multipart messages	85

6.1.2.5	HTTP/2 request retries	86

6.1.3	Resources	86

6.1.3.1	Overview	86

6.1.3.2	Resource: SM contexts collection	88

6.1.3.2.1	Description	88

6.1.3.2.2	Resource Definition	88

6.1.3.2.3	Resource Standard Methods	89

6.1.3.2.3.1	POST	89

6.1.3.2.4	Resource Custom Operations	93

6.1.3.3	Resource: Individual SM context	93

6.1.3.3.1	Description	93

6.1.3.3.2	Resource Definition	93

6.1.3.3.3	Resource Standard Methods	93

6.1.3.3.4	Resource Custom Operations	93

6.1.3.3.4.1	Overview	93

6.1.3.3.4.2	Operation: modify	93

6.1.3.3.4.2.1	Description	93

6.1.3.3.4.2.2	Operation Definition	93

6.1.3.3.4.3	Operation: release	97

6.1.3.3.4.3.1	Description	97

6.1.3.3.4.3.2	Operation Definition	97

6.1.3.3.4.4	Operation: retrieve	98

6.1.3.3.4.4.1	Description	98

6.1.3.3.4.4.2	Operation Definition	98

6.1.3.3.4.5	Operation: send-mo-data	100

6.1.3.3.4.5.1	Description	100

6.1.3.3.4.5.2	Operation Definition	100

6.1.3.4	Void	101

6.1.3.5	Resource: PDU sessions collection (H-SMF or SMF)	101

6.1.3.5.1	Description	101

6.1.3.5.2	Resource Definition	101

6.1.3.5.3	Resource Standard Methods	101

6.1.3.5.3.1	POST	101

6.1.3.5.4	Resource Custom Operations	105

6.1.3.5.4.1	Overview	105

6.1.3.6	Resource: Individual PDU session (H-SMF or SMF)	106

6.1.3.6.1	Description	106

6.1.3.6.2	Resource Definition	106

6.1.3.6.3	Resource Standard Methods	106

6.1.3.6.4	Resource Custom Operations	106

6.1.3.6.4.1	Overview	106

6.1.3.6.4.2	Operation: modify	106

6.1.3.6.4.2.1	Description	106

6.1.3.6.4.2.2	Operation Definition	106

6.1.3.6.4.3	Operation: release	109

6.1.3.6.4.3.1	Description	109

6.1.3.6.4.3.2	Operation Definition	109

6.1.3.6.4.4	Operation: transfer-mo-data	110

6.1.3.6.4.4.1	Description	110

6.1.3.6.4.4.2	Operation Definition	110

6.1.3.6.4.5	Operation: retrieve	111

6.1.3.6.4.5.1	Description	111

6.1.3.6.4.5.2	Operation Definition	111

6.1.3.7	Resource: Individual PDU session (V-SMF or I-SMF)	113

6.1.3.7.1	Description	113

6.1.3.7.2	Resource Definition	113

6.1.3.7.3	Resource Standard Methods	113

6.1.3.7.3.1	POST	113

6.1.3.7.4	Resource Custom Operations	114

6.1.3.7.4.1	Overview	114

6.1.3.7.4.2	Operation: modify	115

6.1.3.7.4.2.1	Description	115

6.1.3.7.4.2.2	Operation Definition	115

6.1.3.7.4.3	Operation: transfer-mt-data	118

6.1.3.7.4.3.1	Description	118

6.1.3.7.4.3.2	Operation Definition	118

6.1.4	Custom Operations without associated resources	119

6.1.5	Notifications	119

6.1.5.1	General	119

6.1.5.2	SM Context Status Notification	120

6.1.5.2.1	Description	120

6.1.5.2.2	Notification Definition	120

6.1.6	Data Model	121

6.1.6.1	General	121

6.1.6.2	Structured data types	127

6.1.6.2.1	Introduction	127

6.1.6.2.2	Type: SmContextCreateData	128

6.1.6.2.3	Type: SmContextCreatedData	138

6.1.6.2.4	Type: SmContextUpdateData	141

6.1.6.2.5	Type: SmContextUpdatedData	149

6.1.6.2.6	Type: SmContextReleaseData	152

6.1.6.2.7	Type: SmContextRetrieveData	154

6.1.6.2.8	Type: SmContextStatusNotification	155

6.1.6.2.9	Type: PduSessionCreateData	158

6.1.6.2.10	Type: PduSessionCreatedData	166

6.1.6.2.11	Type: HsmfUpdateData	171

6.1.6.2.12	Type: HsmfUpdatedData	178

6.1.6.2.13	Type: ReleaseData	181

6.1.6.2.14	Type: HsmfUpdateError	182

6.1.6.2.15	Type: VsmfUpdateData	183

6.1.6.2.16	Type: VsmfUpdatedData	187

6.1.6.2.17	Type: StatusNotification	189

6.1.6.2.18	Type: QosFlowItem	189

6.1.6.2.19	Type: QosFlowSetupItem	190

6.1.6.2.20	Type: QosFlowAddModifyRequestItem	190

6.1.6.2.21	Type: QosFlowReleaseRequestItem	191

6.1.6.2.22	Type: QosFlowProfile	191

6.1.6.2.23	Type: GbrQosFlowInformation	192

6.1.6.2.24	Type: QosFlowNotifyItem	192

6.1.6.2.25	Type: Void	193

6.1.6.2.26	Type: Void	193

6.1.6.2.27	Type: SmContextRetrievedData	193

6.1.6.2.28	Type: TunnelInfo	194

6.1.6.2.29	Type: StatusInfo	194

6.1.6.2.30	Type: VsmfUpdateError	195

6.1.6.2.31	Type: EpsPdnCnxInfo	196

6.1.6.2.32	Type: EpsBearerInfo	196

6.1.6.2.33	Type: PduSessionNotifyItem	196

6.1.6.2.34	Type: EbiArpMapping	196

6.1.6.2.35	Type: SmContextCreateError	197

6.1.6.2.36	Type: SmContextUpdateError	197

6.1.6.2.37	Type: PduSessionCreateError	198

6.1.6.2.38	Type: MmeCapabilities	198

6.1.6.2.39	Type: SmContext	199

6.1.6.2.40	Type: ExemptionInd	204

6.1.6.2.41	Type: PsaInformation	205

6.1.6.2.42	Type: DnaiInformation	205

6.1.6.2.43	Type: N4Information	205

6.1.6.2.44	Type: IndirectDataForwardingTunnelInfo	206

6.1.6.2.45	Type: SmContextReleasedData	206

6.1.6.2.46	Type: ReleasedData	207

6.1.6.2.47	Type: SendMoDataReqData	207

6.1.6.2.48	Type: CnAssistedRanPara	208

6.1.6.2.49	Type: UlclBpInformation	208

6.1.6.2.50	Type: TransferMoDataReqData	208

6.1.6.2.51	Type: TransferMtDataReqData	209

6.1.6.2.52	Type: TransferMtDataError	209

6.1.6.2.53	Type: TransferMtDataAddInfo	209

6.1.6.2.54	Type: VplmnQos	210

6.1.6.2.55	Type: DdnFailureSubs	210

6.1.6.2.56	Type: RetrieveData	210

6.1.6.2.57	Type: RetrievedData	211

6.1.6.2.58	Type: SecurityResult	211

6.1.6.2.59	Type: UpSecurityInfo	211

6.1.6.2.60	Type: DdnFailureSubInfo	212

6.1.6.2.61	Type: AlternativeQosProfile	212

6.1.6.2.62	Type: ProblemDetailsAddInfo	212

6.1.6.2.63	Type: ExtProblemDetails	213

6.1.6.2.64	Type: QosMonitoringInfo	213

6.1.6.2.65	Type: IpAddress	213

6.1.6.2.66	Type: RedundantPduSessionInformation	213

6.1.6.2.67	Type: QosFlowTunnel	214

6.1.6.2.68	Type: TargetDnaiInfo	214

6.1.6.2.69	Type: AfCoordinationInfo	214

6.1.6.2.70	Type: NotificationInfo	214

6.1.6.3	Simple data types and enumerations	215

6.1.6.3.1	Introduction	215

6.1.6.3.2	Simple data types	215

6.1.6.3.3	Enumeration: UpCnxState	216

6.1.6.3.4	Enumeration: HoState	217

6.1.6.3.5	Enumeration: RequestType	217

6.1.6.3.6	Enumeration: RequestIndication	217

6.1.6.3.7	Enumeration: NotificationCause	218

6.1.6.3.8	Enumeration: Cause	218

6.1.6.3.9	Enumeration: ResourceStatus	219

6.1.6.3.10	Enumeration: DnnSelectionMode	220

6.1.6.3.11	Enumeration: EpsInterworkingIndication	220

6.1.6.3.12	Enumeration: N2SmInfoType	221

6.1.6.3.13	Enumeration: MaxIntegrityProtectedDataRate	222

6.1.6.3.14	Enumeration: MaReleaseIndication	222

6.1.6.3.15	Enumeration: SmContextType	222

6.1.6.3.16	Enumeration: PsaIndication	222

6.1.6.3.17	Enumeration: N4MessageType	222

6.1.6.3.18	Enumeration: QosFlowAccessType	223

6.1.6.3.19	Enumeration: UnavailableAccessIndication	223

6.1.6.3.20	Enumeration: ProtectionResult	223

6.1.6.3.21	Enumeration: QosMonitoringReq	223

6.1.6.3.22	Enumeration: Rsn	224

6.1.6.3.23	Enumeration: SmfSelectionType	224

6.1.6.4	Binary data	224

6.1.6.4.1	Introduction	224

6.1.6.4.2	N1 SM Message	224

6.1.6.4.3	N2 SM Information	225

6.1.6.4.4	n1SmInfoFromUe, n1SmInfoToUe, unknownN1SmInfo	226

6.1.6.4.5	N4 Message Payload	228

6.1.6.4.6	Mobile Originated Data	228

6.1.6.4.7	Mobile Terminated Data	229

6.1.7	Error Handling	229

6.1.7.1	General	229

6.1.7.2	Protocol Errors	229

6.1.7.3	Application Errors	229

6.1.8	Feature Negotiation	233

6.1.9	Security	236

6.1.10	HTTP redirection	237

Annex A (normative): OpenAPI specification	238

A.1	General	238

A.2	Nsmf_PDUSession API	238

Annex B (Informative): HTTP Multipart Messages	297

B.1	Example of HTTP multipart message	297

B.1.1	General	297

B.1.2	Example HTTP multipart message with N1 SM Message binary data	297

Annex C (informative): Change history	299

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies the stage 3 protocol and data model for the Nsmf Service Based Interface. It provides stage 3 protocol definitions and message flows, and specifies the API for each service offered by the SMF other than the Session Management Event Exposure service and Session Management services for Non-IP Data Delivery (NIDD).

The 5G System stage 2 architecture and procedures are specified in 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3].

The Technical Realization of the Service Based Architecture and the Principles and Guidelines for Services Definition are specified in 3GPP TS 29.500 [4] and 3GPP TS 29.501 [5].

The Session Management Event Exposure Service is specified in 3GPP TS 29.508 [6]. The Session Management services for Non-IP Data Delivery (NIDD) are specified in 3GPP TS 29.542 [37].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[6]	3GPP TS 29.508: "5G System; Session Management Event Exposure Service; Stage 3".

[7]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[8]	3GPP TS 24.007: "Mobile radio interface signalling layer 3; General aspects".

[9]	3GPP TS 38.413: "NG Radio Access Network (NG-RAN); NG Application Protocol (NGAP)".

[10]	IETF RFC 2387: "The MIME Multipart/Related Content-type".

[11]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[12]	IETF RFC 2045: "Multipurpose Internet Mail Extensions (MIME) Part One: Format of Internet Message Bodies".

[13]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[14]	IETF RFC 7540: "Hypertext Transfer Protocol Version 2 (HTTP/2)".

[15]	OpenAPI Initiative, "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[16]	3GPP TS 29.274: "3GPP Evolved Packet System (EPS); Evolved General Packet Radio Service (GPRS) Tunnelling Protocol for Control plane (GTPv2-C); Stage 3".

[17]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[18]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[19]	3GPP TS 29.510: "Network Function Repository Services; Stage 3".

[20]	3GPP TS 29.518: "5G System; Access and Mobility Management Service; Stage 3".

[21]	3GPP TS 23.380: "IMS Restoration Procedures".

[22]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[23]	IETF RFC 7807: "Problem Details for HTTP APIs".

[24]	3GPP TS 23.527: "5G System; Restoration Procedures".

[25]	3GPP TS 32.255: "Charging management; 5G data connectivity domain charging; stage 2".

[26]	3GPP TS 32.291: "Charging management; 5G system, charging service; Stage 3".

[27]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[28]	3GPP TR 21.900: "Technical Specification Group working methods".

[29]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane Nodes; stage 3".

[30]	3GPP TS 29.512: "5G System; Session Management Policy Control Service; Stage 3".

[31]	IETF RFC 7230: "Hypertext Transfer Protocol (HTTP/1.1): Message Syntax and Routing".

[32]	IETF RFC 7231: "Hypertext Transfer Protocol (HTTP/1.1): Semantics and Content".

[33]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access; Stage 2".

[34]	3GPP TS 29.524: "5G System; Cause codes mapping between 5GC interfaces; Stage 3".

[35]	3GPP TS 23.216: "Single Radio Voice Call Continuity (SRVCC); Stage 2".

[36]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[37]	3GPP TS 29.542: "5G System; Session Management Services for Non-IP Data Delivery (NIDD); Stage 3".

[38]	3GPP TS 29.519: "5G System; Usage of the Unified Data Repository service for Policy Control Data, Application Data and Structured Data for Exposure; Stage 3".

[39]	3GPP TS 23.548: "5G System Enhancements for Edge Computing; Stage 2".

[40]	3GPP TS 29.531: "5G System; Network Slice Selection Services; Stage 3".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

BP	Branching Point

DNAI	Data Network Access Identifier

DNN	Data Network Name

HR	Home Routed

H-SMF	Home SMF

I-SMF	Intermediate SMF

JSON	Javascript Object NotationNAS	Non-Access Stratum

LADN	Local Area Data Network

MA	Multi-Access

MO	Mobile Originated

MT	Mobile TerminatedPSA	PDU Session Anchor

RSN	Redundancy Sequence Number

SM	Session Management

SMF	Session Management Function

SNPN	Stand-alone Non-Public Network

TNGF	Trusted Non-3GPP Gateway Function

TWIF	Trusted WLAN Interworking Function

UL CL	Uplink Classifier

UPF	User Plane Function

V-SMF	Visited SMF

W-AGF	Wireline Access Gateway Function

# 4	Overview

## 4.1	Introduction

Within the 5GC, the SMF offers services to the AMF, other SMF (V-SMF, H-SMF or I-SMF), PCF and NEF via the Nsmf service based interface (see 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3]).

Figure 4.1-1 provides the reference model (in service based interface representation and in reference point representation), with focus on the SMF and the scope of the present specification.

Figure 4.1-1: Reference model – SMF

N16 is the reference point between the V-SMF and H-SMF in Home Routed (HR) roaming cases.

N16a is the reference point between SMF and I-SMF.

N38 is the reference point between I-SMFs or V-SMFs.

The functionalities supported by the SMF are listed in clause 6.2.2 of 3GPP TS 23.501 [2].

# 5	Services offered by the SMF

## 5.1	Introduction

The SMF supports the following services.

Table 5.1-1: NF Services provided by SMF

| Service Name | Description | Example Consumer |
| --- | --- | --- |
| Nsmf_PDUSession | This service manages the PDU sessions and uses the policy and charging rules received from the PCF. The service operations exposed by this NF service allows the consumer NFs to establish, modify and delete the PDU sessions, and to send mobile originated data. | V-SMF, H-SMF, I-SMF, SMF, AMF |
| Nsmf_EventExposure | This service exposes the events happening on the PDU sessions to the consumer NFs. | PCF, NEF, AMF, SMF |
| Nsmf_NIDD | This service enables the delivery of Non-IP data towards a UE. | NEF |

The Nsmf_EventExposure service is specified in 3GPP TS 29.508 [6]. The Session Management services for Non-IP Data Delivery (NIDD) are specified in 3GPP TS 29.542 [37].

Table 5.1-2 summarizes the corresponding APIs defined in this specification.

Table 5.1-2: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | apiName | Annex |
| --- | --- | --- | --- | --- | --- |
| Nsmf_PDUSession | 6.1 | SMF PDU Session Service | TS29502_Nsmf_PDUSession.yaml | nsmf-pdusession | A.2 |

## 5.2	Nsmf_PDUSession Service

### 5.2.1	Service Description

The Nsmf_PDUSession service operates on the PDU Sessions. The service operations exposed by this service allow other NFs to establish, modify and release the PDU Sessions. The following are the key functionalities of this NF service:

-	Creation, modification and deletion of SM contexts for PDU Sessions; an SM context represents an association between the NF Service Consumer (e.g. AMF) and the SMF for a PDU session;

-	Retrieval of SM contexts of PDU sessions (i.e. UE EPS PDN connection or complete SM context), e.g. to move PDU sessions towards the EPC using the N26 interface or to transfer SM contexts between I-SMFs or V-SMFs over the N38 interface;

-	Creation, modification and deletion of PDU sessions between the V-SMF and H-SMF in HR roaming scenarios, or between the I-SMF and SMF for PDU sessions involving an I-SMF;

-	Sending of mobile originated data (received over NAS) for a PDU session to the SMF, V-SMF in HR roaming scenarios, or I-SMF for PDU sessions involving an I-SMF;

-	Transferring of NEF anchored mobile originated data for a PDU session to the H-SMF in HR roaming scenarios, or SMF for PDU sessions involving an I-SMF;

-	Transferring of NEF anchored mobile terminated data for a PDU session to the V-SMF in HR roaming scenarios, or I-SMF for PDU sessions involving an I-SMF;

-	Association of policy and charging rules with PDU Sessions and binding the policy and charging rules to flows;

-	Interacting with the UPF over N4 for creating, modifying and releasing user plane sessions;

-	Process user plane events from the UPF and apply the corresponding policy and charging rules.

The Nsmf_PDUSession service supports the following service operations.

Table 5.2.1-1: Service operations supported by the Nsmf_PDUSession service

| Service Operations | Description | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Create SM Context | Create an SM context in SMF, or in V-SMF in HR roaming scenarios, or in I-SMF during the I-SMF insertion and change scenarios, for a PDU session. | Request/Response | AMF |
| Update SM Context | Update the SM context of a PDU session and/or provide the SMF with N1 or N2 SM information received from the UE or from the AN. | Request/Response | AMF, I-SMF |
| Release SM Context | Release the SM context of a PDU session when the PDU session has been released. | Request/Response | AMF |
| Notify SM Context Status (NOTE 1) | Notify the NF Service Consumer about the status of an SM Context of a PDU session (e.g. the SM Context is released within the SMF). | Subscribe/Notify | AMF |
| Retrieve SM Context (NOTE 2) | Retrieve an SM context of a PDU session: - from SMF, or from V-SMF in HR roaming scenarios, for 5GS to EPS mobility; - from SMF during I-SMF insertion or from I-SMF during I-SMF change/removal; - from V-SMF during change of V-SMF. | Request/Response | AMF, I-SMF, V-SMF, SMF |
| Create | Create a PDU session in the H-SMF in HR roaming scenarios, or in the SMF for PDU sessions involving an I-SMF. | Request/Response | V-SMF, I-SMF |
| Update | Update a PDU session in the H-SMF or V-SMF in HR roaming scenarios, or in the I-SMF or SMF for PDU sessions involving an I-SMF. | Request/Response | V-SMF, H-SMF, I-SMF, SMF |
| Release | Release a PDU session in the H-SMF in HR roaming scenarios, or in the SMF for PDU sessions involving an I-SMF. | Request/Response | V-SMF, I-SMF |
| Notify Status (NOTE 3) | Notify the NF Service Consumer about the status of a PDU session (e.g. the PDU session is released due to local reasons within the SMF). | Subscribe/Notify | V-SMF, I-SMF |
| Retrieve (NOTE 2) | Retrieve information from a PDU session context from the H-SMF for a HR PDU session, or from the SMF for a PDU session with an I-SMF. | Request/Response | V-SMF, I-SMF |
| Send MO Data | Send mobile originated data received over NAS for a PDU session | Request/Response | AMF |
| Transfer MO Data (NOTE 4) | Transfer NEF anchored mobile originated data received from AMF for a PDU session | Request/Response | V-SMF, I-SMF |
| Transfer MT Data (NOTE 5) | Transfer NEF anchored mobile terminated data received from NEF for a PDU session | Request/Response | H-SMF, SMF |
| NOTE 1: This corresponds to the SMContextStatusNotify service operation defined in 3GPP TS 23.502 [3]. NOTE 2: This corresponds to the ContextRequest service operation defined in 3GPP TS 23.502 [3]. NOTE 3: This corresponds to the StatusNotify service operation defined in 3GPP TS 23.502 [3]. NOTE 4: This corresponds to the MessageTransfer service operation in clause 4.25.4 of 3GPP TS 23.502 [3]. NOTE 5: This corresponds to the MessageTransfer service operation in clause 4.25.5 of 3GPP TS 23.502 [3]. |  |  |  |

### 5.2.2	Service Operations

#### 5.2.2.1	Introduction

See Table 5.2.1-1 for an overview of the service operations supported by the Nsmf_PDUSession service.

#### 5.2.2.2	Create SM Context service operation

##### 5.2.2.2.1	General

The Create SM Context service operation shall be used to create an individual SM context, for a given PDU session, in the SMF, in the V-SMF for HR roaming scenarios, or in the I-SMF for a PDU session with an I-SMF.

It is used in the following procedures:

-	UE requested PDU Session Establishment (see clauses 4.3.2 and 4.23.5.1 of 3GPP TS 23.502 [3]);

-	EPS to 5GS Idle mode mobility, EPS to 5GS Idle mode mobility with data forwarding or handover using N26 interface (see clauses 4.11.1, 4.23.12.3, 4.23.12.5 and 4.23.12.7 of 3GPP TS 23.502 [3]);

-	EPS to 5GS mobility without N26 interface (see clause 4.11.2.3 3GPP TS 23.502 [3]);

-	Handover of a PDU session between 3GPP access and non-3GPP access, when the target AMF does not know the SMF resource identifier of the SM context used by the source AMF, e.g. when the target AMF is not in the PLMN of the N3IWF (see clause 4.9.2.3.2 of 3GPP TS 23.502 [3]), or when the UE is roaming and the selected N3IWF is in the HPLMN (see clause 4.9.2.4.2 of 3GPP TS 23.502 [3]);

-	Handover from EPS to 5GC-N3IWF (see clause 4.11.3.1 of 3GPP TS 23.502 [3]);

-	Handover from EPC/ePDG to 5GS (see clause 4.11.4.1 of 3GPP TS 23.502 [3]);

-	Xn based or N2 based handover with I-SMF or V-SMF insertion and change (see clauses 4.23.7.3, 4.23.11 and 4.23.12 of 3GPP TS 23.502 [3]);

-	UE Triggered Service Request with I-SMF insertion/change/removal or V-SMF change (see clause 4.23.4.3 of 3GPP TS 23.502 [3]);

-	Registration procedure for a UE with a PDU session with I-SMF or V-SMF insertion, change and removal (see clause 4.23.3 of 3GPP TS 23.502 [3]);

-	Handover from EPC/ePDG to 5GS with I-SMF insertion (see clause 4.23 of 3GPP TS 23.502 [3]);

-	Handover from non-3GPP to 3GPP access with I-SMF insertion or V-SMF change, and Handover from 3GPP to non-3GPP access with I-SMF removal (see clause 4.23.16 of 3GPP TS 23.502 [3]);

-	SMF Context Transfer procedure, LBO or no Roaming, no I-SMF (see clause 4.26.5.3 of 3GPP TS 23.502 [3]);

-	I-SMF Context Transfer procedure (see clause 4.26.5.2 of 3GPP TS 23.502 [3]);

-	5G-RG requested PDU Session Establishment via W-5GAN (see clause 7.3.1 of 3GPP TS 23.316 [36]);

-	FN-RG related PDU Session Establishment via W-5GAN (see clause 7.3.4 of 3GPP TS 23.316 [36]);

-	Non-5G capable device behind 5G-CRG and FN-CRG requested PDU Session Establishment via W-5GAN (see clause 4.10a of 3GPP TS 23.316 [36]);

-	Handover from 3GPP access/EPS to W-5GAN/5GC (see clause 7.6.4.1 of 3GPP TS 23.316 [36]);

-	I-SMF selection per DNAI (see clause 4.23.5.4 of 3GPP TS 23.502 [3]).

There shall be only one individual SM context per PDU session.

The NF Service Consumer (e.g. AMF) shall create an SM context by using the HTTP POST method as shown in Figure 5.2.2.2.1-1.

Figure 5.2.2.2.1-1: SM context creation

1.	The NF Service Consumer shall send a POST request to the resource representing the SM contexts collection resource of the SMF. The payload body of the POST request shall contain:

-	a representation of the individual SM context resource to be created;

-	the requestType IE, if the Request type IE is received from the UE for a single access PDU session and if the request refers to an existing PDU session or an existing Emergency PDU session; the requestType IE shall not be included for a MA-PDU session establishment request; it may be included otherwise;

-	the Old PDU Session ID, if it is received from the UE (i.e. for a PDU session establishment for the SSC mode 3 operation);

-	the indication that the UE is inside or outside of the LADN (Local Area Data Network) service area, if the DNN corresponds to a LADN;

-	the indication that a MA-PDU session is requested if a MA-PDU session is requested to be established by the UE, or the indication that the PDU session is allowed to be upgraded to a MA-PDU session if so indicated by the UE;

-	the indication that the same PCF is required for the requested DNN and S-NSSAI, if it is received by the AMF from UE Subscription data in the UDM, together with the PCF ID selected by the AMF;

-	the anType;

-	the additionalAnType, if the UE is registered over both 3GPP and Non-3GPP accesses;

-	the cpCiotEnabled IE with the value "True", if the NF service consumer (e.g. the AMF) has verified that the CIOT feature is supported by the SMF (and for a home-routed session, that it is also supported by the H-SMF), and Control Plane CIoT 5GS Optimisation is enabled for this PDU session;

-	the cpOnlyInd IE with the value "True", if the PDU session shall only use Control Plane CIoT 5GS Optimisation;

-	the Invoke NEF indication with the value "True" for a home-routed PDU session, if the cpCiotEnabled IE is set to "True" and data delivery via NEF is selected for the PDU session;

-	a subscription for SM context status notification;

-	the servingNfId identifying the serving AMF;

-	trace control and configuration parameters, if trace is to be activated (see 3GPP TS 32.422 [22]);

-	identifiers (i.e. FQDN or IP address) of N3 terminations at the W-AGF, TNGF or TWIF, if available;

-	a subscription for DDN failure notification, if the Availability after DDN failure event is subscribed by the UDM.

For the UE requested PDU Session Establishment procedure in home routed roaming scenario (see clause 4.3.2.2.2 of 3GPP TS 23.502 [3]), the NF Service Consumer shall provide the URI of the Nsmf_PDUSession service of the H-SMF in the hSmfUri IE and optionally the corresponding SMF ID, and may provide the URI of the Nsmf_PDUSession service of additional H-SMF(s) with the corresponding SMF ID(s). The V-SMF shall try to create the PDU session using the hSmfUri IE. If due to communication failure on the N16 interface the V-SMF does not receive any response from the H-SMF, then:

-	depending on operator policy, the V-SMF may try reaching the hSmfUri via an alternate path; or

-	if additional H-SMF URI is provided, the V-SMF may try to create the PDU session on one of the additional H-SMF(s) provided.

For a PDU session establishment with an I-SMF (see clause 4.23.5.1 of of 3GPP TS 23.502 [3]), the NF Service Consumer shall provide the URI of the Nsmf_PDUSession service of the SMF in the smfUri IE and optionally the corresponding SMF ID, and may provide the URI of the Nsmf_PDUSession service of additional SMF(s) with the corresponding SMF ID(s). The I-SMF shall try to create the PDU session using the smfUri IE. If due to communication failure on the N16a interface the I-SMF does not receive any response from the SMF, then:

-	depending on operator policy, the I-SMF may try reaching the smfUri via an alternate path; or

-	if additional SMF URI is provided, the I-SMF may try to create the PDU session on one of the additional SMF(s) provided.

For the UE requested PDU Session Establishment procedure, if the AMF determines that the RAT type is NB-IoT and the UE has already 2 PDU Sessions with user plane resources activated, the AMF may continue with the PDU Session establishment and include the cpCiotEnabled IE or cpOnlyInd IE with the value "True" to the SMF as specified in clause 4.3.2.2.1 of 3GPP TS 23.502 [3].

The payload body of the POST request may further contain:

-	the name of the AMF service to which SM context status notification are to be sent (see clause 6.5.2.2 of 3GPP TS 29.500 [4]), encoded in the serviceName attribute.

2a.	On success, "201 Created" shall be returned, the payload body of the POST response shall contain the representation describing the status of the request and the "Location" header shall be present and shall contain the URI of the created resource. The authority and/or deployment-specific string of the apiRoot of the created resource URI may differ from the authority and/or deployment-specific string of the apiRoot of the request URI received in the POST request.

If the requestType IE was received in the request and set to EXISTING_PDU_SESSION or EXISTING_EMERGENCY_PDU_SESSION (i.e. indicating that this is a UE request for an existing PDU session or an existing emergency PDU session), the SMF shall identify the existing PDU session or emergency PDU session based on the PDU Session ID; in this case, the SMF shall not create a new SM context but instead update the existing SM context and provide the representation of the updated SM context in the "201 Created" response to the NF Service Consumer.

The POST request shall be considered as colliding with an existing SM context if:

-	it includes the same SUPI, or PEI for an emergency registered UE without a UICC or without an authenticated SUPI, and the same PDU Session ID as for an existing SM context; and

-	this is a request to establish a new PDU session, i.e.:

-	the RequestType IE is present in the request and set to INITIAL_REQUEST or INITIAL_EMERGENCY_REQUEST (e.g. single access PDU session establishment request);

-	the RequestType IE and the maRequestInd IE are both absent in the request (e.g. EPS to 5GS mobility); or

-	the maRequestInd IE is present in the request (i.e. MA-PDU session establishment request) and the access type indicated in the request corresponds to the access type of the existing SM context.

A POST request that collides with an existing SM context shall be treated as a request for a new SM context. Before creating the new SM context, the SMF should delete the existing SM context locally and any associated resources in the UPF and PCF. See also clause 5.2.3.3.1 for the handling of requests which collide with an existing SM context. If the smContextStatusUri of the existing SM context differs from the smContextStatusUri received in the POST request, the SMF shall also send an SM context status notification (see clause 5.2.2.5) targeting the smContextStatusUri of the existing SM context to notify the release of the existing SM context. For a HR PDU session, if the H-SMF URI in the request is different from the H-SMF URI of the existing PDU session, the V-SMF should also delete the existing PDU session in the H-SMF by invoking the Release service operation (see clause 5.2.2.9). For a PDU session with an I-SMF, if the SMF URI in the request is different from the SMF URI of the existing PDU session, the I-SMF should also delete the existing PDU session in the SMF by invoking the Release service operation (see clause 5.2.2.9).

If the requestType IE was received in the request and indicates this is a request for a new PDU session (i.e. INITIAL_REQUEST) and if the Old PDU Session ID was also included in the request, the SMF shall identify the existing PDU session to release and to which the new PDU session establishment relates, based on the Old PDU Session ID.

If no GPSI IE is provided in the request, e.g. for a PDU session moved from another access or another system, and the SMF knows that a GPSI is already associated with the PDU session (or a GPSI is received from h-SMF for a HR PDU session), the SMF shall include the GPSI in the response.

2b.	If the request does not include the "UE presence in LADN service area" indication and the SMF determines that the DNN corresponds to a LADN, then the SMF shall consider that the UE is outside of the LADN service area. The SMF shall reject the request if the UE is outside of the LADN service area.

On failure, or redirection during a UE requested PDU Session Establishment, one of the HTTP status code listed in Table 6.1.3.2.3.1-3 shall be returned. For a 4xx/5xx response, the message body shall contain an SmContextCreateError structure, including:

-	a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.3.2.3.1-3;

-	N1 SM information (PDU Session Reject), if the request included N1 SM information, except if the error prevents the SMF from generating a response to the UE (e.g. invalid request format).

##### 5.2.2.2.2	EPS to 5GS Idle mode mobility using N26 interface (with or without data forwarding)

The NF Service Consumer (e.g. AMF) shall request the SMF to move a UE EPS PDN connection to 5GS using N26 interface, as follows.

Figure 5.2.2.2.2-1: EPS to 5GS Idle mode mobility using N26 interface

1.	The NF Service Consumer shall send a POST request towards the SMF (+PGW-C) of each UE EPS PDN connection, as specified in clause 5.2.2.2.1, with the following additional information:

-	UE EPS PDN connection, including the EPS bearer contexts, received from the MME, representing the individual SM context resource to be created;

-	the pduSessionsActivateList attribute, including the PDU Session ID of all the PDU session(s) to be re-activated;

-	the epsBearerCtxStatus attribute, indicating the status of all the EPS bearer contexts in the UE, if corresponding information is received in the Registration Request from the UE;

-	the dlDataWaitingInd attribute, indicating that DL data buffered in EPS needs to be forwarded to the UE, if such indication is present in the Context Response received from the MME.

2a.	Upon receipt of such a request, if:

-	a corresponding PDU session is found based on the EPS bearer contexts (after invoking a Create service operation towards the H-SMF for a Home Routed PDU session, or towards the SMF for a PDU session with an I-SMF);

-	the default EPS bearer context of the corresponding PDU session is not reported as inactive by the UE in the epsBearerCtxStatus attribute, if received; and

-	it is possible to proceed with moving the PDN connection to 5GS,

then the SMF shall return a 201 Created response including the following information:

-	PDU Session ID corresponding to the default EPS bearer ID of the EPS PDN connection;

-	S-NSSAI assigned to the PDU session; in home routed roaming case, the S-NSSAI for home PLMN shall be returned;

-	the allocatedEbiList attribute, containing the EBI(s) allocated to the PDU session;

and, if the PDU session that is derived by the SMF based on the EPS bearer contexts was requested to be re-activated, i.e. if the PDU Session ID was present in the pduSessionsActivateList,or if DL data buffered in EPS needs to be forwarded to the UE:

-	the upCnxState attribute set to ACTIVATING;

-	N2 SM information to request the 5G-AN to assign resources to the PDU session (see PDU Session Resource Setup Request Transfer IE in clause 9.3.4.1 of 3GPP TS 38.413 [9]), including (among others) the transport layer address and tunnel endpoint of the uplink termination point for the user plane data for this PDU session (i.e. UPF's GTP-U F-TEID for uplink traffic).

The "Location" header shall be present in the POST response and shall contain the URI of the created SM context resource.

If the epsBearerCtxStatus attribute is received in the request, the SMF shall check whether some EPS bearer(s) of the corresponding PDU session have been deleted by the UE but not notified to the EPS, and if so, the SMF shall release these EPS bearers, corresponding QoS rules and QoS flow level parameters locally, as specified in clause 4.11.1.3.3 of 3GPP TS 23.502 [3].  

The NF Service Consumer (e.g. AMF) shall store the association of the PDU Session ID and the SMF ID, and store the allocated EBI(s) associated to the PDU Session ID.

NOTE:	The behaviour specified in this step also applies if the POST request collides with an existing SM context, i.e. if the POST request includes the same SUPI, or PEI for an emergency registered UE without a UICC or without an authenticated SUPI, and the default EPS bearer ID received in the UE EPS PDN connection is the same as in the existing SM context.

2b.	Same as step 2b of figure 5.2.2.2.1-1. Steps 3 to 4 are skipped in this case.

If the SMF determines that seamless session continuity from EPS to 5GS is not supported for the PDU session, the SMF shall set the "cause" attribute in the ProblemDetails structure to "NO_EPS_5GS_CONTINUITY".

If the default EPS bearer context of the PDU session is reported as inactive by the UE in the epsBearerCtxStatus attribute, the SMF shall set the "cause" attribute in the ProblemDetails structure to "DEFAULT_EPS_BEARER_INACTIVE".

3.	Same as step 3 of figure 5.2.2.3.2.2-1, if the SMF returned a 201 Created response with the upConnectionState set to ACTIVATING and N2 SM Information,

4.	Same as step 4 of figure 5.2.2.3.2.2-1. During an EPS to 5GS Idle mode mobility using N26 interface with data forwarding (see clause 4.11.1.3.3A of 3GPP TS 23.502 [3]), the 200 OK response shall additionally contain the CN tunnel information for data forwarding from EPS, i.e. the forwardingFTeid attribute or the forwarding bearer contexts to be sent to the MME in the Context Acknowledge, based on the association between the EPS bearer ID(s) and QFI(s) for the QoS flow(s).

##### 5.2.2.2.3	EPS to 5GS Handover Preparation using N26 interface

The NF Service Consumer (e.g. AMF) shall request the SMF to handover a UE EPS PDN connection to 5GS using N26 interface, as follows.

Figure 5.2.2.2.3-1: EPS to 5GS handover using N26 interface

1.	The NF Service Consumer shall send a POST request, as specified in clause 5.2.2.2.1, with the following additional information:

-	UE EPS PDN connection, including the EPS bearer contexts, representing the individual SM context resource to be created;

-	hoState attribute set to PREPARING (see clause 5.2.2.3.4.1);

-	the indication of whether direct or indirect DL data forwarding applies;

-	targetId identifying the target RAN Node ID and TAI based on the Target ID IE received in the Forward Relocation Request message from the source MME.

NOTE 1:	The Target ID IE can be set to the Target NG-RAN Node ID containing a Global RAN Node ID and selected TAI with 3-octets length, or the Target eNB ID containing a Global eNB ID and selected TAI with 2-octets length; for the latter case, the NF Service Consumer, i.e. the AMF needs determine a value for the Target NG-RAN Node ID and TAI with 3-octets length based on the local configuration to be provided to the SMF.

2a.	Upon receipt of such a request, if a corresponding PDU session is found based on the EPS bearer contexts (after invoking a Create service operation towards the H-SMF, for a Home Routed PDU session) and it is possible to proceed with handing over the PDN connection to 5GS, the SMF shall return a 201 Created response including the following information:

-	hoState attribute set to PREPARING and N2 SM information to request the target 5G-AN to assign resources to the PDU session, as specified in step 2 of Figure 5.2.2.3.4.2-1; if the SMF was indicated in step 1 that direct data forwarding is applicable, the SMF shall include an indication that a direct forwarding path is available in the N2 SM information;

-	PDU Session ID corresponding to the default EPS bearer ID of the EPS PDN connection;

-	S-NSSAI assigned to the PDU session; in home routed roaming case, the S-NSSAI for home PLMN shall be returned;

-	allocatedEbiList, containing the EBI(s) allocated to the PDU session.

The "Location" header shall be present in the POST response and shall contain the URI of the created SM context resource.

The NF Service Consumer (e.g. AMF) shall store the association of the PDU Session ID and the SMF ID, and store the allocated EBI(s) associated to the PDU Session ID.

NOTE 2:	The behaviour specified in this step also applies if the POST request collides with an existing SM context, i.e. if the POST request includes the same SUPI, or PEI for an emergency registered UE without a UICC or without an authenticated SUPI, and the default EPS bearer ID received in the UE EPS PDN connection is the same as in the existing SM context.

2b.	Same as step 2b of figure 5.2.2.2.1-1 with the following additions. Steps 3 and 4 of figure 5.2.2.3.8.2-1 are skipped in this case.

If the SMF determines that seamless session continuity from EPS to 5GS is not supported for the PDU session, the SMF shall set the "cause" attribute in the ProblemDetails structure to "NO_EPS_5GS_CONTINUITY".

When receiving a 4xx/5xx response from the SMF, the NF service consumer (e.g. the AMF) shall regard the hoState of the SM Context to be NONE.

##### 5.2.2.2.4	I-SMF Insertion, Change or Removal during Xn based Handover

The NF Service Consumer (e.g. AMF) shall request the I-SMF (for I-SMF insertion or change) or the SMF (for I-SMF removal) to create a SM context during Xn based handover, as follows.

1.	The NF Service Consumer shall send a POST request, with the following additional information:

-	N2 SM information received from the target 5G-AN (see Path Switch Request Transfer IE in clause 9.3.4.8 of 3GPP TS 38.413 [9]);

-	additional N2 SM information received from the source 5G-AN (see Secondary RAT Data Usage Report Transfer IE in clause 9.3.4.23 of 3GPP TS 38.413 [9]), if any;

-	the smContextRef attribute set to the identifier of the SM Context resource in the SMF during I-SMF insertion, or the SM Context resource in the source I-SMF during I-SMF change or removal, and optionally the NF instance identifier of the SMF hosting the SM Context resource.

2a.	On success, the SMF shall return a 201 Created response.

The "Location" header shall be present in the POST response and shall contain the URI of the created SM context resource.

The NF Service Consumer (e.g. AMF) shall store the association of the PDU Session ID and the SMF ID.

2b.	Same as step 2b of figure 5.2.2.2.1-1.

If the Path Swith Request Transfer IE is included within the N2 SM Information in the request message but the path switch failed, the message body shall contain an SmContextCreateError structure, including:

-	N2 SM information (Path Swith Request Unsuccessful Transfer).

##### 5.2.2.2.5	I-SMF Insertion, Change or Removal during N2 based Handover

The NF Service Consumer (e.g. AMF) shall request the I-SMF (for I-SMF insertion or change) or the SMF (for I-SMF removal) to create a SM context during N2 based handover, as follows.

1.	The NF Service Consumer shall send a POST request, with the following additional information:

-	N2 SM information received from the source NG-RAN (see Handover Required Transfer IE in clause 9.3.4.14 of 3GPP TS 38.413 [9]);

-	the hoState attribute set to PREPARING (see clause 5.2.2.3.4.1);

-	the smContextRef attribute set to the identifier of the SM Context resource in the SMF during I-SMF insertion,,or the SM Context resource in the source I-SMF during I-SMF change or removal, and optionally the NF instance identifier of the SMF hosting the SM Context resource.

2a.	On success, the SMF shall return a 201 Created response including the following information:

-	hoState attribute set to PREPARING and N2 SM information to request the target 5G-AN to assign resources to the PDU session, as specified in step 2 of Figure 5.2.2.3.4.2-1;

The "Location" header shall be present in the POST response and shall contain the URI of the created SM context resource.

The NF Service Consumer (e.g. AMF) shall store the association of the PDU Session ID and the SMF ID.

2b.	Same as step 2b of figure 5.2.2.2.1-1.

##### 5.2.2.2.6	Service Request with I-SMF insertion/change/removal or with V-SMF change

The NF Service Consumer (e.g. AMF) shall request the new I-SMF or new V-SMF to create a SM context during a Service Request with I-SMF insertion/change or with V-SMF change, or shall request the SMF to create a SM context during a Service Request with I-SMF removal, as follows.

Figure 5.2.2.2.6-1: Service Request with I-SMF insertion/change/removal or with V-SMF change

1.	The NF Service Consumer shall send a POST request as specified in clause 5.2.2.2.1, with the following additional information:

-	the smContextRef attribute set to the identifier of the SM Context resource in the SMF (for a Service Request with an I-SMF insertion) or in the old I-SMF (for a Service Request with an I-SMF change or removal) or in the old V-SMF (for a Service Request with a V-SMF change), and optionally the NF instance identifier of the SMF hosting the SM Context resource.

-	the upCnxState attribute set to ACTIVATING (see clause 5.2.2.3.2.1) to indicate the establishment of N3 tunnel User Plane resources for the PDU Session;

2a.	On success, the SMF shall return a 201 Created response as specified in clause 5.2.2.2.1 with the following additional information:

-	the upCnxState attribute set to ACTIVATING;

-	N2 SM information to request the 5G-AN to assign resources to the PDU session (see PDU Session Resource Setup Request Transfer IE in clause 9.3.4.1 of 3GPP TS 38.413 [9]), including the transport layer address and tunnel endpoint of the uplink termination point for the user plane data for this PDU session (i.e. UPF's GTP-U F-TEID for uplink traffic).

2b.	Same as step 2b of figure 5.2.2.2.1-1. Steps 3 to 4 of figure 5.2.2.3.2.2-1 are skipped in this case.

##### 5.2.2.2.7	Registration procedure for a UE with a PDU session with I-SMF insertion, change and removal

The NF Service Consumer (e.g. AMF) shall request the SMF to create a SM context during UE Registration procedure for a PDU session with I-SMF insertion, change and removal, as follows.

1.	Same as step 1 of 5.2.2.2.1-1, the NF Service Consumer shall send a POST request, with the following additional information:

-	the smContextRef attribute set to the identifier of the SM Context resource in the SMF during I-SMF insertion or the SM Context resource in the I-SMF during I-SMF removal or the SM Context resource in the old I-SMF during I-SMF change, and optionally the NF instance identifier of the SMF hosting the SM Context resource;

-	the upCnxState attribute set to ACTIVATING (see clause 5.2.2.3.2.1) to indicate the establishment of N3 tunnel User Plane resources for the PDU Session, if the UE requested to activate the PDU session;

-	if the UE is in CM-CONNECTED state during the registration procedure (see clause 4.11.1.3.3 of 3GPP TS 23.502 [3]), ranUnchangedInd attribute shall be set to indicate that NG-RAN is not changed for the PDU Session (i.e. for this case, the NG-RAN tunnel info shall be included in SM context retrieved from old I-SMF or SMF).

2a.	On success, the SMF shall return a 201 Created response.

If the SMF establishes N3 tunnel User Plane resources for the PDU Session, e.g. due to the NF Service Consumer requesting so or due to buffered DL data in the old I-SMF/I-UPF (see clause 4.23.3 of 3GPP TS 23.502 [3]), the 201 Created response shall contain the following additional information:

-	the upCnxState attribute set to ACTIVATING;

-	N2 SM information to request the 5G-AN to assign resources to the PDU session (see PDU Session Resource Setup Request Transfer IE in clause 9.3.4.1 of 3GPP TS 38.413 [9]), including the transport layer address and tunnel endpoint of the uplink termination point for the user plane data for this PDU session (i.e. UPF's GTP-U F-TEID for uplink traffic).

If the SMF receives the ranUnchangedInd attribute set to indicate that NG-RAN is not changed for the PDU Session, the SMF shall respond with a 201 Created with the following additional information:

-	N2 SM information to request the 5G-AN to update UPF tunnel info of the PDU session (see PDU Session Resource Modify Request Transfer IE in clause 9.3.4.3 of 3GPP TS 38.413 [9]), including the transport layer address and tunnel endpoint of the uplink termination point for the user plane data for this PDU session (i.e. UPF's GTP-U F-TEID for uplink traffic and NG-RAN's GTP-U F-TEID for downlink traffic).

The "Location" header shall be present in the POST response and shall contain the URI of the created SM context resource.

The NF Service Consumer (e.g. AMF) shall store the association of the PDU Session ID and the SMF ID.

2b.	Same as step 2b of figure 5.2.2.2.1-1.

##### 5.2.2.2.8	SMF Context Transfer procedure, LBO or no Roaming, no I-SMF

The NF Service Consumer (e.g. AMF) shall request the SMF to create a SM context during an SMF Context Transfer procedure, LBO or no Roaming, no I-SMF, as follows.

1.	Same as step 1 of 5.2.2.2.1-1, the NF Service Consumer shall send a POST request, with the following additional information:

-	SMF transfer indication, Old SMF ID, the identifier of the SM Context resource in old SMF.

2a.	On success, the SMF shall return a 201 Created response.

The "Location" header shall be present in the POST response and shall contain the URI of the created SM context resource.

The NF Service Consumer (e.g. AMF) shall store the association of the PDU Session ID and the SMF ID.

2b.	Same as step 2b of figure 5.2.2.2.1-1.

##### 5.2.2.2.9	I-SMF Context Transfer procedure

The NF Service Consumer (e.g. AMF) shall request the SMF to create a SM context during I-SMF Context Transfer procedure, as follows.

1.	Same as step 1 of 5.2.2.2.1-1, the NF Service Consumer shall send a POST request, with the following additional information:

-	SMF transfer indication, Old SMF ID, the identifier of the SM Context resource in old SMF.

2a.	On success, the SMF shall return a 201 Created response.

The "Location" header shall be present in the POST response and shall contain the URI of the created SM context resource.

The NF Service Consumer (e.g. AMF) shall store the association of the PDU Session ID and the SMF ID.

2b.	Same as step 2b of figure 5.2.2.2.1-1.

##### 5.2.2.2.10	Handover between 3GPP and non-3GPP accesses with I-SMF insertion/removal or V-SMF change

The NF Service Consumer (e.g. AMF) shall request the I-SMF (for I-SMF insertion during a handover from non-3GPP to 3GPP access), the V-SMF (for V-SMF change during a handover from non-3GPP to 3GPP access) or the SMF (for I-SMF removal during a handover from 3GPP to non-3GPP access) to create a SM context as follows.

1.	The NF Service Consumer shall send a POST request as specified in clause 5.2.2.2.1, with the following additional information:

-	the smContextRef attribute set to the identifier of the SM Context resource in the SMF (during I-SMF insertion), the SM Context resource in the source I-SMF during I-SMF removal, or the SM Context resource in the source V-SMF during V-SMF change, and optionally the NF instance identifier of the SMF hosting the SM Context resource.

2a.	Same as step 2a of figure 5.2.2.2.1-1.

2b.	Same as step 2b of figure 5.2.2.2.1-1.

##### 5.2.2.2.11	I-SMF/V-SMF restoration procedure

The NF Service Consumer (e.g. AMF) shall request the new I-SMF/V-SMF to create a SM context during the I-SMF/V-SMF restoration procedure, as specified in clause 6.7 of 3GPP TS 23.527 [24], as follows.

1.	Same as step 1 of 5.2.2.2.1-1, the NF Service Consumer shall send a POST request, with the following additional information:

-	an I-SMF/V-SMF restoration indication which indicates that I-SMF/V-SMF restoration procedure is triggered;

-	the PDU Session ID indicating the PDU session for which the I-SMF/V-SMF restoration procedure is invoked;

-	the hSmfId and hSmfUri attributes set to the NF Instance ID and API URI of the home SMF in the case of HR roaming scenario, or the smfId and smfUri attributes set to the NF Instance ID and API URI of the anchor SMF in the case of I-SMF scenario;

-	the upCnxState attribute set to ACTIVATING, if it is requested by the UE during a Service Request procedure;

In the CreateSMContext Request, the smContextRef attribute shall not be included, so as to avoid the new I-SMF/V-SMF to retrieve SM Context from the old I-SMF/V-SMF or from the anchor SMF.

On receiving the CreateSMContext Request, the new I-SMF/V-SMF shall detect this procedure is used for I-SMF/V-SMF restoration based on the I-SMF/V-SMF restoration indication, and thus send Create Request to the anchor SMF, as specified in clause 5.2.2.7.6.

2a.	On success, the new I-SMF/V-SMF shall return a 201 Created response.

If the new I-SMF/V-SMF establishes N3 tunnel User Plane resources for the PDU Session, as requested by the NF Service Consumer, the 201 Created response shall contain the following additional information:

-	the upCnxState attribute set to ACTIVATING;

-	N2 SM information to request the 5G-AN to assign resources to the PDU session (see PDU Session Resource Setup Request Transfer IE in clause 9.3.4.1 of 3GPP TS 38.413 [9]), including the N3 tunnel information (i.e. I-UPF/V-UPF's GTP-U F-TEID for uplink traffic).

The "Location" header shall be present in the POST response and shall contain the URI of the created SM context resource.

The NF Service Consumer (e.g. AMF) shall store the association of the PDU Session ID and the new I-SMF/V-SMF ID.

2b.	Same as step 2b of figure 5.2.2.2.1-1.

##### 5.2.2.2.12	I-SMF selection per DNAI

The NF Service Consumer (e.g. AMF) shall request the new I-SMF to create a SM context during I-SMF selection per DNAI, as follows.

Figure 5.2.2.2.12-1: I-SMF selection per DNAI

1.	The NF Service Consumer shall send a POST request as defined in step 1 of Figure 5.2.2.2.6-1, with the following additional information:

-	the smContextRef attribute set to the identifier of the SM Context resource in the SMF during I-SMF insertion, or the SM Context resource in the source I-SMF during I-SMF change, and optionally the NF instance identifier of the SMF hosting the SM Context resource;

-	the target DNAI;

-	if the UE is in CM-CONNECTED state, the ranUnchangedInd attribute shall be set to indicate that NG-RAN is not changed for the PDU Session (i.e. for this case, the NG-RAN tunnel info shall be included in SM context retrieved from old I-SMF or SMF) as specified in clause 4.23.5.4 of 3GPP TS 23.502 [3].

2a.	On success, the SMF shall return a 201 Created response, with the following additional information:

If the SMF receives the ranUnchangedInd attribute set to indicate that NG-RAN is not changed for the PDU Session, the SMF shall include the N2 SM information to request the 5G-AN to update UPF tunnel info of the PDU session (see PDU Session Resource Modify Request Transfer IE in clause 9.3.4.3 of 3GPP TS 38.413 [9]), including the transport layer address and tunnel endpoint of the uplink termination point for the user plane data for this PDU session (i.e. UPF's GTP-U F-TEID for uplink traffic and NG-RAN's GTP-U F-TEID for downlink traffic).

The "Location" header shall be present in the POST response and shall contain the URI of the created SM context resource.

The NF Service Consumer (e.g. AMF) shall store the association of the PDU Session ID and the new I-SMF ID.

2b.	Same as step 2b of figure 5.2.2.2.1-1.

#### 5.2.2.3	Update SM Context service operation

##### 5.2.2.3.1	General

The Update SM Context service operation shall be used to update an individual SM context and/or provide N1 or N2 SM information received from the UE or the AN, for a given PDU session, towards the SMF, or the V-SMF for HR roaming scenarios, or the I-SMF for a PDU session with an I-SMF.

It is used in the following procedures:

-	PDU Session modification (see clause 4.3.3 of 3GPP TS 23.502 [3]);

-	UE or network requested PDU session release (see clause 4.3.4.2 and clause 4.3.4.3 of 3GPP TS 23.502 [3]);

-	UE requested MA PDU session establishment over the other access (see clause 4.22.7 of 3GPP TS 23.502 [3]);

-	UE or network-initiated MA PDU session release over a single access (see clause 4.22 of 3GPP TS 23.502 [3]);

-	Activation or Deactivation of the User Plane connection of an existing PDU session, i.e. establishment or release of the N3 tunnel between the AN and serving CN (see clause 5.6.8 of 3GPP TS 23.501 [2],  clauses 4.2.2.2, 4.2.3, 4.2.6, 4.2.10 and 4.9.1.3.3 of 3GPP TS 23.502 [3] and clauses 7.2.2.1, 7.2.2.2, 7.2.5.2 and 7.2.5.3 of 3GPP TS 23.316 [36]);

-	Xn and N2 Handover procedures (see clauses 4.9.1, 4.23.7 and 4.23.11 of 3GPP TS 23.502 [3]);

-	Handover between 3GPP and untrusted non-3GPP access procedures (see clause 4.9.2 of 3GPP TS 23.502 [3]);

-	Inter-AMF change due to AMF planned maintenance or AMF failure (see clause 5.21.2 of 3GPP TS 23.501 [2]), or inter-AMF mobility in CM-IDLE mode (see clauses 4.2.2.2 and 4.23.3 of 3GPP TS 23.502 [3]);

-	RAN Initiated QoS Flow Mobility (see clause 4.14.1 of 3GPP TS 23.502 [3] and clause 8.2.5 of 3GPP TS 38.413 [9]);

-	All procedures requiring to provide N1 or N2 SM information to the SMF, e.g. UE requested PDU Session Establishment procedure (see clause 4.3.2.2 of 3GPP TS 23.502 [3]), session continuity procedure (see clause 4.3.5 of 3GPP TS 23.502 [3]);

-	EPS to 5GS Idle mode mobility, EPS to 5GS Idle mode mobility with data forwarding or handover using N26 interface (see clause 4.11 of 3GPP TS 23.502 [3]);

-	5GS to EPS Handover using N26 interface (see clause 4.11.1.2 of 3GPP TS 23.502 [3]);

-	5GS to EPS Idle mode mobility using N26 interface with data forwarding (see clause 4.11.1.3.2A of 3GPP TS 23.502 [3]);

-	PDU Session Reactivation during P-CSCF Restoration procedure via AMF (see clause 5.8.4.3 of 3GPP TS 23.380 [21]);

-	AMF requested PDU session release due to a change of the set of network slices for a UE where a network slice instance is no longer available (see clause 4.3.4.2 of 3GPP TS 23.502 [3]);

-	AMF receives an "initial request" with PDU Session Id which already exists in PDU session context of the UE (see clause 5.4.5.2.5 of 3GPP TS 24.501 [7]);

-	Secondary RAT Usage Data Reporting (see clause 4.21 of 3GPP TS 23.502 [3]);

-	Service Request Procedures with I-SMF change or I-SMF removal when downlink data packets are buffered at the I-UPF (See clause 4.23.4 of 3GPP TS 23.502 [3]);

-	Connection Suspend procedure (see clause 4.8.1.2 of 3GPP TS 23.502 [3]);

-	Connection Resume in CM-IDLE with Suspend procedure (see clause 4.8.2.3 of 3GPP TS 23.502 [3]);

-	5G-RG or Network requested PDU Session Modification via W-5GAN (see clause 7.3.2 of 3GPP TS 23.316 [36]);

-	5G-RG or Network requested PDU Session Release via W-5GAN (see clause 7.3.3 of 3GPP TS 23.316 [36]);

-	FN-RG or Network requested PDU Session Modification via W-5GAN (see clause 7.3.6 of 3GPP TS 23.316 [36]);

-	FN-RG or Network requested PDU Session Release via W-5GAN (see clause 7.3.7 of 3GPP TS 23.316 [36]);

-	Handover between 3GPP access/5GC and W-5GAN access (see clause 7.6.3 of 3GPP TS 23.316 [36]);

-	AMF requested PDU session release due to Network Slice-Specific (Re-)Authentication and (Re-)Authorization failure or revocation (see clauses 4.2.9.2, 4.2.9.3 and 4.2.9.4 of 3GPP TS 23.502 [3]);

-	5G-RG requested PDU Session Establishment via W-5GAN (see clause 7.3.1 of 3GPP TS 23.316 [36]);

-	FN-RG related PDU Session Establishment via W-5GAN (see clause 7.3.4 of 3GPP TS 23.316 [36]);

-	Non-5G capable device behind 5G-CRG and FN-CRG requested PDU Session Establishment via W-5GAN (see clause 4.10a of 3GPP TS 23.316 [36]);

-	Non-5G capable device behind 5G-CRG and FN-CRG or Network requested PDU Session Modification via W-5GAN (see clause 4.10a of 3GPP TS 23.316 [36]);

-	Non-5G capable device behind 5G-CRG and FN-CRG or Network requested PDU Session Release via W-5GAN (see clause 4.10a of 3GPP TS 23.316 [36]);

-	CN-initiated selective deactivation of UP connection of an existing PDU Session associated with W-5GAN Access (see clause 7.3.5 of 3GPP TS 23.316 [36]);

-	Handover between 3GPP access / EPS and W-5GAN/5GC access (see clause 7.6.4 of 3GPP TS 23.316 [36]);

-	AMF requested PDU session release due to Control Plane Only indication associated with PDU Session is not applicable any longer as described in 3GPP TS 23.501 [2] clause 5.31.4.1;

-	Subscribe to / unsubscribe from the DDN failure status notification (see clauses 4.15.3.2.7 and 4.15.3.2.9 of 3GPP TS 23.502 [3]).

The NF Service Consumer (e.g. AMF) shall update an individual SM context and/or provide N1 or N2 SM information to the SMF by using the HTTP POST method (modify custom operation) as shown in Figure 5.2.2.3.1-1.

Figure 5.2.2.3.1-1: SM context update

1.	The NF Service Consumer shall send a POST request to the resource representing the individual SM context resource in the SMF. The payload body of the POST request shall contain the modification instructions and/or the N1 or N2 SM information, or the indication that the PDU session is allowed to be upgraded to a MA PDU session if so indicated by the UE as specified in clause 6.4.2.2 of 3GPP TS 24.501 [7], or subscribe/unsubscribe of the DDN failure notification as specified in clause 4.15.3.2.7 of 3GPP TS 23.502 [3]. If the request contains EBI(s) to revoke, then the SMF shall disassociate the EBI(s) with the QFI(s) with which they are associated.

2a.	On success, "204 No Content" or "200 OK" shall be returned; in the latter case, the payload body of the POST response shall contain the representation describing the status of the request and/or N1 or N2 SM information.

If the ExemptionInd IE is included in the request message, indicating that the NAS SM message included in the request was exempted from NAS congestion control by the AMF, the SMF shall verify that the included 5G SM message can be exempted from a NAS SM congestion control activated in the AMF as specified in clause 5.19.7 of 3GPP TS 23.501 [2].

The SMF may indicate to the NF Service Consumer that it shall release EBI(s) that were assigned to the PDU session by including the releaseEbiList IE, e.g. when a QoS flow is released.

2b.	On failure or redirection, one of the HTTP status code listed in Table 6.1.3.3.3.2-3 shall be returned.  For a 4xx/5xx response, the message body shall contain an SmContextUpdateError structure, including:

-	a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.3.3.3.2-3;

-	N1 SM information, if the SMF needs and can return a response to the UE;

-	N2 SM information, if the SMF needs and can return a response to the NG-RAN.

The following clauses specify additional requirements applicable to specific scenarios.

##### 5.2.2.3.2	Activation and Deactivation of the User Plane connection of a PDU session

###### 5.2.2.3.2.1	General

The upCnxState attribute of an SM context represents the state of the User Plane connection of the PDU session. The upCnxState attribute may take the following values:

-	ACTIVATED: a N3 tunnel is established between the 5G-AN and UPF (F-TEIDs assigned for both uplink and downlink traffic);

-	DEACTIVATED: no N3 tunnel is established between the 5G-AN and UPF;

-	ACTIVATING: a N3 tunnel is being established (5G-AN's F-TEID for downlink traffic is not assigned yet).

Clauses 5.2.2.3.2.2 and 5.2.2.3.2.3 specify how the NF Service Consumer (e.g. AMF) request the SMF to activate or deactivate the User Plane connection of the PDU session, e.g. upon receiving a Service Request from the UE requesting to activate a PDU session or upon an AN release procedure respectively. Clause 5.2.2.3.2.3 also applies in case of 5G-AN requested PDU session resource release by sending the NGAP PDU SESSION RESOURCE NOTIFY to the AMF (see step 1d in clause 4.3.4.2 of 3GPP TS 23.502 [3]).

In scenarios where the SMF takes the initiative to activate or deactivate the User Plane connection of the PDU session, e.g. during a Network Triggered Service Request or CN-initiated selective deactivation of the User Plane connection of a PDU session respectively, the SMF invokes the Namf_N1N2MessageTransfer procedure with the inclusion of N2 SM Information (and optionally of a N1 SM Container) as specified in 3GPP TS 23.502 [3] to request the establishment or release of the PDU session's resources in the 5G-AN. The Update SM Context service operation is then used as specified in clause 5.2.2.3.1 to transfer the response to the SMF.

Clause 5.2.2.3.2.4 specifies how the NF Service Consumer (e.g. AMF) indicates to the SMF that the access type of a PDU session can be changed from non-3GPP access to 3GPP access, during a Network Triggered Service Request initiated for a PDU session associated to the non-3GPP access, if the PDU Session for which the UE was paged or notified is in the List Of Allowed PDU Sessions provided by the UE and if the AMF has received N2 SM Information only or N1 SM Container and N2 SM Information for that PDU session from the SMF in step 3a of clause 4.2.3.3 of 3GPP TS 23.502 [3].

###### 5.2.2.3.2.2	Activation of User Plane connectivity of a PDU session

The NF Service Consumer (e.g. AMF) shall request the SMF to activate the User Plane connection of an existing PDU session, i.e. establish the N3 tunnel between the 5G-AN and UPF, as follows.

Figure 5.2.2.3.2.2-1: Activation of the User Plane connection of a PDU session

1.	The NF Service Consumer shall request the SMF to activate the user plane connection of the PDU session by sending a POST request, as specified in clause 5.2.2.3.1, with the following information:

-	the upCnxState attribute set to ACTIVATING;

-	the user location and access type associated to the PDU session, if modified;

-	the indication that the UE is inside or outside of the LADN service area, if the DNN of the established PDU session corresponds to a LADN;

-	the access type for which the user plane connection needs to be re-activated, for a MA PDU session (i.e. the access type over which a Registration or Service Request was received);

-	the "MO Exception Data Counter" if the UE has accessed the network by using "MO exception data" RRC establishment cause;

-	other information, if necessary.

2a.	Upon receipt of such a request, if the SMF can proceed with activating the user plane connection of the PDU session (see clause 4.2.3 of 3GPP TS 23.501 [2]), the SMF shall set the upCnxState attribute to ACTIVATING and shall return a 200 OK response including the following information:

-	upCnxState attribute set to ACTIVATING;

-	N2 SM information to request the 5G-AN to assign resources to the PDU session (see PDU Session Resource Setup Request Transfer IE in clause 9.3.4.1 of 3GPP TS 38.413 [9]), including the transport layer address and tunnel endpoint of the uplink termination point for the user plane data for this PDU session (i.e. UPF's GTP-U F-TEID for uplink traffic).

If the SMF finds the PDU session already activated when receiving the request in step 1, the SMF shall delete the N3 tunnel information and update the UPF accordingly (see step 8a of clause 4.2.3.2 of 3GPP TS 23.502 [3]).

For a MA-PDU session, the SMF shall perform the above requirements for the access type for which the user plane connection is requested to be re-activated (i.e. the access type indicated in the anTypeToReactivate attribute). The SMF shall not modify the user plane connection status for the other access type, e.g. if the user plane connection is already established for the other access type, it shall remain established.

If the "MO Exception Data Counter" is included in the request and Small Data Rate Control is enabled for the PDU session, then the V-SMF/I-SMF shall forward the counter to the H-SMF/SMF.

2b.	If the request does not include the "UE presence in LADN service area" indication and the SMF determines that the DNN corresponds to a LADN, then the SMF shall consider that the UE is outside of the LADN service area. The SMF shall reject the request if the UE is outside of the LADN service area.

If the SMF cannot proceed with activating the user plane connection of the PDU session (e.g. if the PDU session corresponds to a PDU session of SSC mode 2 and the SMF decides to change the PDU Session Anchor), the SMF shall return an error response, as specified for step 2b of figure 5.2.2.3.1-1.  For a 4xx/5xx response, the SmContextUpdateError structure shall include the following additional information:

-	upCnxState attribute set to DEACTIVATED.

3.	If the SMF returned a 200 OK response, the NF Service Consumer (e.g. AMF) shall subsequently update the SM context in the SMF by sending POST request, as specified in clause 5.2.2.3.1, with the following information:

-	N2 SM information received from the 5G-AN (see PDU Session Resource Setup Response Transfer IE in clause 9.3.4.2 of 3GPP TS 38.413 [9]), including the transport layer address and tunnel endpoint of one or two downlink termination point(s) and the associated list of QoS flows for this PDU session (i.e. 5G-AN's GTP-U F-TEID(s) for downlink traffic), if the 5G-AN succeeded in establishing resources for the PDU sessions; or

-	N2 SM information received from the 5G-AN (see PDU Session Resource Setup Unsuccessful Transfer IE in clause 9.3.4.16 of 3GPP TS 38.413 [9]), including the Cause of the failure, if resources failed to be established for the PDU session.

Upon receipt of this request, the SMF shall:

-	update the UPF with the 5G-AN's F-TEID(s) and set the upCnxState attribute to ACTIVATED, if the 5G-AN succeeded in establishing resources for the PDU sessions; or

-	consider that the activation of the User Plane connection has failed and set the upCnxState attribute to DEACTIVATED" otherwise.

4.	The SMF shall then return a 200 OK response including the upCnxState attribute representing the final state of the user plane connection. If the activation of the User Plane connection failed due to insufficient resources, the cause IE shall be included in the response and set to "INSUFFICIENT_UP_RESOURCES".

###### 5.2.2.3.2.3	Deactivation of User Plane connectivity of a PDU session

The NF Service Consumer (e.g. AMF) shall request the SMF to deactivate the User Plane connectivity of an existing PDU session, i.e. release the N3 tunnel, as follows.

Figure 5.2.2.3.2.2-1: Deactivation of the User Plane connection of a PDU session

1.	The NF Service Consumer shall request the SMF to deactivate the user plane connection of the PDU session by sending a POST request, as specified in clause 5.2.2.3.1, with the following information:

-	upCnxState attribute set to DEACTIVATED;

-	user location and user location timestamp;

-	cause of the user plane deactivation; the cause may indicate a cause received from the 5G-AN or due to an AMF internal event;

-	N2 SM information received from the 5G-AN (see PDU Session Resource Notify Released Transfer IE in clause 9.3.4.13 of 3GPP TS 38.413 [9])), if the request is triggered due to 5G-AN requested PDU session resource release;

-	other information, if necessary.

2.	Upon receipt of such a request, the SMF shall deactivate release the N3 tunnel of the PDU session, set the upCnxState attribute to DEACTIVATED and return a 200 OK response including the upCnxState attribute set to DEACTIVATED.

If the request is triggered due to 5G-AN requested PDU session resource release, the SMF may decide to keep the PDU Session (with user plane connection deactivated) or release the PDU Session. If the SMF decides to keep the PDU Session, it shall return "200 OK" with the upCnxState attribute set to DEACTIVATED, but not including n1SmMsg and n2SmInfo. If the SMF decides to release the PDU Session, it shall return "200 OK" with the upCnxState attribute set to DEACTIVATED, including n1SmMsg IE but not-including n2SmInfo IE.

###### 5.2.2.3.2.4	Changing the access type of a PDU session from non-3GPP access to 3GPP access during a Service Request procedure

The NF Service Consumer (e.g. AMF) shall indicate to the SMF that the access type of a PDU session can be changed as follows:

Figure 5.2.2.3.2.4-1: Indicating that the access type of a PDU session can be changed

1.	The NF Service Consumer shall indicate that the access type of a PDU session can be changed by sending a POST request, as specified in clause 5.2.2.3.1, with the following information:

-	anTypeCanBeChanged attribute set to "true";

-	other information, if necessary.

2a.	Same as step 2a of figure 5.2.2.3.1-1. In HR roaming scenarios, the V-SMF shall invoke the Update service operation towards the H-SMF to notify that the access type of the PDU session can be changed (see clause 5.2.2.8.2.2).

2b.	Same as step 2b of figure 5.2.2.3.1-1.

NOTE:	This is used during a Service Request procedure (see clause 4.2.3.2 of 3GPP TS 23.502 [3]), in response to paging or NAS notification indicating non-3GPP access, if the PDU Session for which the UE was paged or notified is in the List Of Allowed PDU Sessions provided by the UE and if the AMF has received N2 SM Information only or N1 SM Container and N2 SM Information for that PDU session from the SMF in step 3a of clause 4.2.3.3 of 3GPP TS 23.502 [3].

If the PDU Session is moved from the non-3GPP access to 3GPP access (i.e. N3 tunnel for the PDU Session is established successfully), the SMF and NF Service Consumer (e.g. AMF) updates the associated access of the PDU Session.

##### 5.2.2.3.3	Xn Handover

The NF Service Consumer (e.g. AMF) shall request the SMF to switch the downlink N3 tunnel of the PDU session towards a new GTP tunnel endpoint as follows.

Figure 5.2.2.3.3-1: Xn handover

1.	The NF Service Consumer shall request the SMF to switch the downlink N3 tunnel of the PDU session towards a new GTP tunnel endpoint by sending a POST request, as specified in clause 5.2.2.3.1, with the following information:

-	the indication that the PDU session is to be switched;

-	N2 SM information received from the target 5G-AN (see Path Switch Request Transfer IE in clause 9.3.4.8 of 3GPP TS 38.413 [9]), including the new transport layer address and tunnel endpoint of the downlink termination point for the user data for this PDU session (i.e. 5G-AN's GTP-U F-TEID for downlink traffic);

-	additional N2 SM information received from the source 5G-AN (see Secondary RAT Data Usage Report Transfer IE in clause 9.3.4.23 of 3GPP TS 38.413 [9]), if any;

-	the user location associated to the PDU session;

-	the indication that the UE is inside or outside of the LADN service area, if the DNN of the established PDU session corresponds to a LADN;

-	other information, if necessary.

2a.	If the SMF can proceed with switching the user plane connection of the PDU session, the SMF shall return a 200 OK response including the following information:

-	N2 SM information (see Path Switch Request Acknowledge Transfer IE in clause 9.3.4.9 of 3GPP TS 38.413 [9]), including the transport layer address and tunnel endpoint of the uplink termination point for the user data for this PDU session (i.e. UPF's GTP-U F-TEID for uplink traffic).

If the request does not include the "UE presence in LADN service area" indication and the SMF determines that the DNN corresponds to a LADN, then the SMF shall consider that the UE is outside of the LADN service area. The SMF shall proceed as specified in clause 5.6.5 of 3GPP TS 23.501 [2].

2b.	If the SMF cannot proceed with switching the user plane connection of the PDU session, the SMF shall return an error response, as specified for step 2b of figure 5.2.2.3.1-1, including:

-	N2 SM information (see Path Switch Request Unsuccessul Transfer IE in clause 9.3.4.20 of 3GPP TS 38.413 [9]), including the cause of the failure.

For a PDU session that is rejected by the target RAN (i.e. a PDU session indicated as failed to setup in the PATH SWITCH REQUEST), the NF Service Consumer (e.g. AMF) shall indicate the failure to setup the PDU session in the target RAN as follows.

Figure 5.2.2.3.3-2: Xn handover – PDU session rejected by the target RAN

1.	The NF Service Consumer shall indicate to the SMF that the PDU session could not be setup in the target RAN by sending a POST request, as specified in clause 5.2.2.3.1, with the following information:

-	the indication that the PDU session failed to be switched;

-	N2 SM information received from the target 5G-AN (see Path Switch Request Setup Failed Transfer IE in clause 9.3.4.15 of 3GPP TS 38.413 [9]), including the cause why the session could not be setup;

-	additional N2 SM information received from the source 5G-AN (see Secondary RAT Data Usage Report Transfer IE in clause 9.3.4.23 of 3GPP TS 38.413 [9]), if any;

-	other information, if necessary.

2a.	Upon receipt of such a request, the SMF shall return a "204 No Content" response. The SMF shall decide whether to release the PDU session or deactivate the user plane connection of the PDU session, as specified in clause 4.9.1.2 of 3GPP TS 23.502 [3].

2b.	Same as step 2b of figure 5.2.2.3.1-1.

##### 5.2.2.3.4	N2 Handover

###### 5.2.2.3.4.1	General

The hoState attribute of an SM context represents the handover state of the PDU session. The hoState attribute may take the following values:

-	NONE: no handover is in progress for the PDU session;

-	PREPARING: a handover is in preparation for the PDU session; SMF is preparing the N3 tunnel between the target 5G-AN and UPF, i.e. the UPF's F-TEID is assigned for uplink traffic;

-	PREPARED: a handover is prepared for the PDU session; SMF is updated for the N3 tunnel between the target 5G-AN and UPF, with the target 5G-AN's F-TEID to be assigned for downlink traffic upon handover execution;

-	COMPLETED: the handover is completed (successfully);

-	CANCELLED: the handover is cancelled.

###### 5.2.2.3.4.2	N2 Handover Preparation

The NF Service Consumer (e.g. T-AMF) shall request the SMF to prepare the handover of an existing PDU session, i.e. prepare the N3 tunnel between the target 5G-AN and UPF, as follows.

Figure 5.2.2.3.4.2-1: N2 Handover Preparation

1.	The NF Service Consumer shall request the SMF to prepare the handover of the PDU session by sending a POST request, as specified in clause 5.2.2.3.1, with the following information:

-	updating the hoState attribute of the individual SM Context resource in the SMF to PREPARING;

-	targetId identifying the target RAN Node ID and TAI received in the Handover Required from the source NG-RAN;

-	targetServingNfId set to the target AMF Id, for a N2 handover with AMF change;

-	N2 SM information received from the source NG-RAN (see Handover Required Transfer IE in clause 9.3.4.14 of 3GPP TS 38.413 [9]), indicating whether a direct path is available;

-	the supportedFeatures IE indicating the optional features it supports, if at least one optional feature defined in clause 6.1.8 is supported;

-	other information, if necessary.

2a.	Upon receipt of such a request, if the SMF can proceed with preparing the handover of the PDU session (see clause 4.9.1.3 of 3GPP TS 23.501 [2]), the SMF shall set the hoState attribute to PREPARING and shall return a 200 OK response including the following information:

-	hoState attribute set to PREPARING;

-	N2 SM information to request the target 5G-AN to assign resources to the PDU session (see PDU Session Resource Setup Request Transfer IE in clause 9.3.4.1 of 3GPP TS 38.413 [9]), including (among others) the transport layer address and tunnel endpoint of the uplink termination point for the user plane data for this PDU session (i.e. UPF's GTP-U F-TEID for uplink traffic);

-	the supportedFeatures IE in the response, if the supportedFeatures IE was received in the request and at least one optional feature defined in clause 6.1.8 is supported by the updated SM context resource.

The SMF shall store the targetServingNfId, if received in the request, but the SMF shall still consider the AMF (previously) received in the servingNfId IE as the serving AMF for the UE.

2b.	If the SMF cannot proceed with preparing the handover of the PDU session (e.g. the UE moves into a non-allowed service area), the SMF shall return an error response, as specified in step 2b of figure 5.2.2.3.1-1. 

When receiving a 4xx/5xx response from the SMF, the NF service consumer (e.g. the AMF) shall regard the hoState of the SM Context to be NONE.

3.	If the SMF returned a 200 OK response in step 2a, the NF Service Consumer (e.g. AMF) shall subsequently update the SM context in the SMF by sending POST request, as specified in clause 5.2.2.3.1, with the following information:

-	hoState attribute set to PREPARED;

-	N2 SM information received from the target 5G-AN (see Handover Request Acknowledge Transfer IE in clause 9.3.4.11 of 3GPP TS 38.413 [9]), including (among others) the transport layer address and tunnel endpoint of the downlink termination point for the user data for this PDU session (i.e. target 5G-AN's GTP-U F-TEID for downlink traffic), if the target 5G-AN succeeded in establishing resources for the PDU session;

-	N2 SM information received from the target 5G-AN (see Handover Resource Allocation Unsuccessful Transfer IE in clause 9.3.4.19 of 3GPP TS 38.413 [9]), including the Cause of the failure, if resources failed to be established for the PDU sessions.

4a.	If the target 5G-AN succeeded in establishing resources for the PDU sessions, the SMF shall set the hoState attribute to PREPARED and return a 200 OK response including the following information:

-	hoState attribute to PREPARED;

-	N2 SM information (see Handover Command Transfer IE in clause 9.3.4.10 of 3GPP TS 38.413 [9]) containing DL forwarding tunnel information to be sent to the source 5G-AN by the AMF if direct or indirect data forwarding applies (see step 11f of clause 4.9.1.3.2 of 3GPP TS 23.502 [3]).

4b.	If the SMF cannot proceed with preparing the handover of the PDU session (e.g. the target 5G-AN failed to establish resources for the PDU session), the SMF shall set the hoState to NONE, release resources reserved for the handover to the target 5G-AN, and return an error response as specified in step 2b of figure 5.2.2.3.1-1. For a 4xx/5xx response, the SmContextUpdateError structure shall include the following additional information:

-	N2 SM information (see Handover Preparation Unsuccessful Transfer IE in clause 9.3.4.18 of 3GPP TS 38.413 [9]) indicating the cause of the failure;

-	the cause in the error attribute set to HANDOVER_RESOURCE_ALLOCATION_FAILURE, if the target 5G-AN failed to establish resources for the PDU session.

When receiving a 4xx/5xx response from the SMF, the NF service consumer (e.g. the AMF) shall regard the hoState of the SM Context to be NONE.

If the handover preparation fails completely on the target 5G-AN (i.e. target 5G-AN returns a NGAP HANDOVER_FAILURE), the (T-)AMF shall request the SMF to cancel the handover of the PDU session as described in clause 5.2.2.3.4.4.

###### 5.2.2.3.4.3	N2 Handover Execution

The NF Service Consumer (e.g. T-AMF) shall request the SMF to complete the execution the handover of an existing PDU session, upon being notified by the target 5G-AN that the handover to the target 5G-AN has been successful, as follows.

Figure 5.2.2.3.4.3-1: N2 Handover Execution

1.	The NF Service Consumer shall request the SMF to complete the execution of the handover of the PDU session by sending a POST request, as specified in clause 5.2.2.3.1, with the following information:

-	updating the hoState attribute of the individual SM Context resource in the SMF to COMPLETED;

-	servingNfId set to the new serving AMF Id, for a N2 handover with AMF change;

-	the indication that the UE is inside or outside of the LADN service area, if the DNN of the established PDU session corresponds to a LADN;

-	N2 SM information received from the source 5G-AN (see Secondary RAT Data Usage Report Transfer IE in clause 9.3.4.23 of 3GPP TS 38.413 [9]), if any;

-	other information, if necessary.

2.	Upon receipt of such a request, the SMF shall return a 200 OK response including the following information:

-	hoState attribute set to COMPLETED.

The SMF shall complete the execution of the handover, e.g. switch the PDU session towards the downlink termination point for the user data received from the target 5G-AN (i.e. target 5G-AN's GTP-U F-TEID for downlink traffic), set the hoState to NONE and delete any stored targetServingNfId. For PDU session with I-SMF insertion, the I-SMF shall complete the execution of the handover by initiating an Update service operation towards the anchor SMF in order to switch the PDU session towards the I-UPF controlled by I-SMF (see clause 5.2.2.8.2.12).

If the request does not include the "UE presence in LADN service area" indication and the SMF determines that the DNN corresponds to a LADN, then the SMF shall consider that the UE is outside of the LADN service area. The SMF shall proceed as specified in clause 5.6.5 of 3GPP TS 23.501 [2].

The (T-)AMF shall request the SMF to complete the execution of the handover of the PDU session only for those PDU sessions that successfully completed the handover procedure. If there are PDU sessions that failed to handover due to timeout of SMF responses in any step of the handover preparation phase (e.g. if the Update SM Context Response arrived too late or not at all during the handover preparation phase, see step 7 of clause 4.9.1.3.3 of 3GPP TS 23.502 [3]), then the (T-)AMF shall inform the SMF about this failure, by sending a POST request with the cause attribute set to "HO_FAILURE" for every such PDU session, upon receipt of the NGAP HANDOVER NOTIFY. The SMF shall then release the resources prepared for the handover and consider that the PDU session is deactivated and that the handover attempt is terminated for the PDU session.

If the handover fails completely on the target 5G-AN due to the execution phase not completed successfully (i.e. missing NGAP HANDOVER NOTIFY), the (T-)AMF shall request the SMF to cancel the handover of the PDU session as described in clause 5.2.2.3.4.4.

###### 5.2.2.3.4.4	N2 Handover Cancellation

The NF Service Consumer (e.g. T-AMF) shall request the SMF to cancel the handover of an existing PDU session, e.g. upon receipt of such a request from the source 5G-AN, as follows.

Figure 5.2.2.3.4.3-1: N2 Handover Cancellation

1.	The NF Service Consumer shall request the SMF to cancel the execution of the handover of the PDU session by sending a POST request, as specified in clause 5.2.2.3.1, with the following information:

-	updating the hoState attribute of the individual SM Context resource in the SMF to CANCELLED;

-	cause information;

-	other information, if necessary.

2.	Upon receipt of such a request, the SMF return a 200 OK response including the following information:

-	hoState attribute set to CANCELLED.

The SMF shall cancel the execution of the handover, e.g. release resources reserved for the handover to the target 5G-AN, set the hoState to NONE and delete any stored targetServingNfId. For PDU Session with I-SMF insertion, the I-SMF shall cancel the handover by initiating an Update service operation towards the anchor SMF in order to release resources at the SMF and PSA UPF reserved during handover preparation (see clause 5.2.2.8.2.13).

##### 5.2.2.3.5	Handover between 3GPP and untrusted non-3GPP access procedures

###### 5.2.2.3.5.1	General

The handover of a PDU session between 3GPP and untrusted non-3GPP access shall be supported as specified in clause 4.9.2 of 3GPP TS 23.502 [3]. Such a handover may involve:

-	the same AMF, or a target AMF in the same PLMN as the source AMF (see clauses 4.9.2.1, 4.9.2.2, 4.9.2.3.1 and 4.9.2.4.1 of 3GPP TS 23.502 [3]). The Update SM Context service operation is used in these cases; or

-	a target AMF in a different PLMN than the source AMF (see clauses 4.9.2.3.2 and 4.9.2.4.2 of 3GPP TS 23.502 [3]). The Create SM Context service operation is used in this case (see clause 5.2.2.2).

For a Home-Routed PDU session, the target AMF may be located in the VPLMN, or in the HPLMN when the N3IWF is in the HPLMN.

###### 5.2.2.3.5.2	Handover of a PDU session without AMF change or with target AMF in same PLMN

In these scenarios, the same V-SMF is used before and after the handover.

The NF Service Consumer (e.g. AMF) shall request the SMF to handover an existing PDU session from 3GPP access to untrusted non-3GPP access, or vice-versa, as follows.

Figure 5.2.2.3.5.2-1: Handover between 3GPP and untrusted non-3GPP access

1.	The NF Service Consumer shall request the SMF to handover an existing PDU session from 3GPP access to untrusted non-3GPP access, or vice-versa, by sending a POST request, as specified in clause 5.2.2.3.1, with the following information:

-	updating the anType attribute of the individual SM Context resource in the SMF to the target access type, i.e. to 3GPP_ACCESS or NON_3GPP_ACCESS;

-	other information, if necessary.

2a.	Same as step 2a of Figure 5.2.2.3.1-1.

2b.	If the SMF cannot proceed with handing over the PDU session to the target access type, the SMF shall return an error response, as specified for step 2b of figure 5.2.2.3.1-1. For a 4xx/5xx response, the SmContextUpdateError structure shall include the following additional information:

-	N1 SM Information to reject the UE request.

##### 5.2.2.3.6	Inter-AMF change or mobility

The NF Service Consumer (e.g. new AMF) shall inform the SMF that it has taken over the role of serving the UE (e.g. it has taken the responsibility of the signalling towards the UE), when so required by 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3], as follows.

Figure 5.2.2.3.6-1: Inter-AMF change or mobility

1.	The NF Service Consumer shall update the SMF with the new serving AMF, by sending a POST request, as specified in clause 5.2.2.3.1, with the following information:

-	servingNfId set to the new serving AMF Id;

-	the supportedFeatures IE indicating the optional features it supports, if at least one optional feature defined in clause 6.1.8 is supported;

-	other information, if necessary, e.g. to activate the user plane connection of the PDU session (see clause 5.2.2.3.2.2).

2a.	Same as step 2a of Figure 5.2.2.3.1-1. In addition, the SMF shall include the supportedFeatures IE in the response, if the supportedFeatures IE was received in the request and at least one optional feature defined in clause 6.1.8 is supported by the updated SM context resource.

2b.	Same as step 2b of figure 5.2.2.3.1-1.

##### 5.2.2.3.7	RAN Initiated QoS Flow Mobility

The NF Service Consumer (e.g. AMF) shall request the SMF to transfer QoS flows to and from Secondary RAN node, or more generally, handle a NG-RAN PDU Session Resource Modify Indication, as follows.

Figure 5.2.2.3.7-1: RAN Initiated QoS Flow Mobility

1.	The NF Service Consumer shall request the SMF to modify the PDU session, as requested by the NG-RAN, by sending a POST request, as specified in clause 5.2.2.3.1, with the following information:

-	N2 SM information received from the 5G-AN (see PDU Session Resource Modify Indication Transfer IE in clause 9.3.4.6 of 3GPP TS 38.413 [9]), including the transport layer information for the QoS flows of this PDU session (i.e. 5G-AN's GTP-U F-TEIDs for downlink traffic);

-	other information, if necessary.

2a.	Upon receipt of such a request, if the SMF can proceed with switching the QoS flows of the PDU session, the SMF shall return a 200 OK response including the following information:

-	N2 SM information (see PDU Session Resource Modify Confirm Transfer IE in clause 9.3.4.7 of 3GPP TS 38.413 [9]), including the list of QoS flows which were modified successfully and the list of QoS flows which failed to be modified if available.

2b.	If the SMF cannot proceed with switching the QoS flows of the PDU session, the SMF shall return an error response, as specified for step 2b of figure 5.2.2.3.1-1, including:

-	N2 SM information (see PDU Session Resource Modify Indication Unsuccessful Transfer IE in clause 9.3.4.22 of 3GPP TS 38.413 [9]).

##### 5.2.2.3.8	EPS to 5GS Handover using N26 interface

###### 5.2.2.3.8.1	General

The NF Service Consumer (e.g. AMF) shall request the SMF to handover a UE EPS PDN connection to 5GS using N26 interface, following the same requirements as specified for N2 handover in clause 5.2.2.3.4 with the modifications specified in this clause.

###### 5.2.2.3.8.2	EPS to 5GS Handover Preparation

The requirements specified in clause 5.2.2.3.4.2 shall apply with the following modifications.

Figure 5.2.2.3.8.2-1: EPS to 5GS Handover Preparation

1.	Same as step 1 of Figure 5.2.2.2.3-1.

2a.	Same as step 2 of Figure 5.2.2.2.3-1.

2b.	Same as step 2b of figure 5.2.2.3.1-1.

3.	Same as step 3 of Figure 5.2.2.3.4.2-1.

4a.	Same as step 4 of Figure 5.2.2.3.4.2-1, with the following modifications:

The 200 OK response shall not include N2 SM information for DL forwarding tunnel setup, but shall additionally contain:

-	the epsBearerSetup IE(s), containing the list of EPS bearer context(s) successfully handed over to the 5GS and DL data forwarding information, containing either:

-	CN tunnel information generated based on the list of accepted QFI(s) received from the 5G-RAN, if indirect data forwarding applies; or

-	NG-RAN F-TEID per E-RAB accepted for direct data forwarding, as received from the target NG-RAN, if direct data forwarding applies.

4b.	Same as step 2b of figure 5.2.2.3.4.2-1.

###### 5.2.2.3.8.3	EPS to 5GS Handover Execution

The requirements specified in clause 5.2.2.3.4.3 shall apply, with the following modifications.

In step 1 of Figure 5.2.2.3.4.3-1, the NF Service Consumer, i.e. the target AMF, shall include one or more SecondaryRatUsageDataReportContainer(s) in the SmContextUpdateData for the POST request if it received one or more Secondary RAT Usage Data Report(s) applicable for the PDU session from the source MME.

In step 2 of Figure 5.2.2.3.4.3-1, for a Home Routed PDU session, the SMF shall complete the execution of the handover by initiating an Update service operation towards the H-SMF in order to switch the PDU session towards the V-UPF (see clause 5.2.2.8.2.3).

###### 5.2.2.3.8.4	EPS to 5GS Handover Cancellation

The requirements specified in clause 5.2.2.3.4.4 shall apply, with the following modifications.

In step 2 of Figure 5.2.2.3.4.4-1, for a Home Routed PDU session, the V-SMF shall cancel the handover by initiating an Update service operation towards the H-SMF in order to release resources at H-SMF and H-UPF reserved for handover (see clause 5.2.2.8.2.14).

###### 5.2.2.3.8.5	EPS to 5GS Handover Failure

If the handover to 5GS failed, e.g. rejected by the target NG-RAN, the requirements specified in clause 5.2.2.3.4.4 shall apply, with the following modifications:

-	the hoState attribute set to "CANCELLED", to indicate the handover is cancelled;

-	the cause attribute set to "HO_FAILURE".

In step 2 of Figure 5.2.2.3.4.4-1, for a Home Routed PDU session, the V-SMF shall cancel the handover by initiating an Update service operation towards the H-SMF in order to release resources at H-SMF and H-UPF reserved for handover (see clause 5.2.2.8.2.17).

##### 5.2.2.3.9	5GS to EPS Handover using N26 interface

###### 5.2.2.3.9.1	General

The NF Service Consumer (e.g. AMF) shall request the SMF to setup data forwarding tunnels if direct or indirect data forwarding applies to the 5GS to EPS handover using N26 interface, and to remove the indirect data forwarding tunnels previously established when the handover is cancelled or failed.

The AMF should initiate this procedure only if data forwarding is enabled and the MME returns data forwarding F-TEIDs for the related PDN connection context in the Forward Relocation response.

###### 5.2.2.3.9.2	Data forwarding tunnels setup during 5GS to EPS handover

If direct or indirect data forwarding applies to the 5GS to EPS handover, the NF Service Consumer (e.g. AMF) shall provide the SMF with the data forwarding information received from the MME, as specified in clause 4.11.1.2.1 of 3GPP TS 23.502 [3]), as follows.

Figure 5.2.2.3.9-1: 5GS to EPS Handover using N26 interface (data forwarding tunnels setup)

1.	The NF Service Consumer shall send a POST request, as specified in clause 5.2.2.3.1, with the following information:

-	dataForwarding IE set to true;

-	EPS bearer contexts received from the MME in the Forward Relocation Response, including F-TEID(s) for DL data forwarding tunnel(s) towards the target eNB (for direct data forwarding) or towards the forwarding SGW (for indirect data forwarding).

2a.	If indirect data forwarding applies, the SMF shall map the EPS bearers for Data Forwarding to the 5G QoS flows based on the association between the EPS bearer ID(s) and QFI(s) for the QoS flow(s).

The SMF shall return a 200 OK response including the following information:

-	N2 SM information (see Handover Command Transfer IE in clause 9.3.4.10 of 3GPP TS 38.413 [9]) containing DL forwarding tunnel information to be sent to the source 5G-AN by the AMF if direct or indirect data forwarding applies (see step 11f of clause 4.9.1.3.2 of 3GPP TS 23.502 [3]).

If direct data forwarding applies, the DL forwarding tunnel information shall contain the E-UTRAN tunnel info for data forwarding per EPS bearer received from the MME. 

If indirect data forwarding applies, the DL forwarding tunnel information shall contain the CN transport layer address and tunnel endpoint (i.e. UPF's GTP-U F-TEID) for Data Forwarding and the QoS flows for Data Forwarding for this PDU session.

2b.	If the SMF cannot proceed with the request, the SMF shall return an error response, as specified for step 2b of figure 5.2.2.3.1-1.
If none of the EPS bearer contexts received in the POST request body includes an F-TEID for DL data forwarding, the SMF shall return a 403 Forbidden response including a ProblemDetails structure with the "cause" attribute set to "NO_DATA_FORWARDING". Upon receipt of this response, the AMF shall proceed with the handover procedure (as if data forwarding was disabled).

NOTE:	The above use case can occur if an AMF initiates this procedure without checking whether the MME returns data forwarding F-TEIDs for the related PDN connection context in the Forward Relocation response (e.g. pre-Rel-17 or Rel-17 AMF that does not support such checking).

###### 5.2.2.3.9.3	Indirect data forwarding tunnels removal for 5GS to EPS handover cancellation or failure

During 5GS to EPS handover, if indirect data forwarding tunnel(s) have been previously established during the preparation phase and the handover is cancelled, the AMF shall update the SMF of handover cancellation by sending a POST request with the cause attribute set to "HO_CANCEL" and dataForwarding IE set to false with an empty list of EPS bearer contexts. The SMF shall then release the resources prepared for the handover and proceed with the PDU session as if no handover procedure had taken place.

If no resources for EPS bearer(s) can be assigned for any PDU session attempted to be handed over, the AMF shall update the SMF with the information that the handover preparation failed by sending a POST request with the cause attribute set to "HO_FAILURE" and with an empty list of EPS bearer contexts (and without the dataForwarding IE). The SMF shall then release the resources prepared for the handover and proceed with the PDU session as if no handover procedure had taken place.

##### 5.2.2.3.10	P-CSCF Restoration Procedure via AMF

The requirements specified in clause 5.2.2.3.1 shall apply with the following modifications.

1.	Same as step 1 of Figure 5.2.2.3.1-1, with the following modifications.

The POST request shall contain:

-	the release IE set to true;

-	the cause IE set to REL_DUE_TO_REACTIVATION.

##### 5.2.2.3.11	AMF requested PDU Session Release due to duplicated PDU Session Id

When the AMF receives an "initial request" with PDU Session Id which already exists in PDU session context of the UE (see clause 5.4.5.2.5 of 3GPP TS 24.501 [7]), the AMF shall request the SMF to release the existing PDU Session; upon subsequent receipt of an SM context status notification indicating that the SM context has been deleted in the SMF, the AMF shall release the stored context for the PDU session and proceed with the "initial request" with the PDU Session Id.

The requirements for releasing the existing PDU Session specified in clause 5.2.2.3.1 shall apply with the following modifications.

1.	Same as step 1 of Figure 5.2.2.3.1-1, with the following modifications.

The POST request shall contain:

-	the release IE set to true;

-	the cause IE set to REL_DUE_TO_DUPLICATE_SESSION_ID.

NOTE:	The SMF does not send NAS signaling to UE for the PDU session release in this procedure.

##### 5.2.2.3.12	AMF requested PDU Session Release due to slice not available

The requirements specified in clause 5.2.2.3.1 shall apply with the following modifications.

1.	Same as step 1 of Figure 5.2.2.3.1-1, with the following modifications.

The POST request shall contain:

-	the release IE set to true;

-	the cause IE set to REL_DUE_TO_SLICE_NOT_AVAILABLE;

-	optionally the skipN2PduSessionResRelInd IE with the value "true" to skip RAN resources release for the PDU session, e.g. for a PDU session with active UP associated with a slice that is no longer available after a handover.

##### 5.2.2.3.13	Indirect Data Forwarding Tunnel establishment during N2 based Handover with I-SMF

During N2 based handover with I-SMF insertion/change/removal, the NF Service Consumer (e.g. target I-SMF) shall use this procedure to exchange N3/N9 forwarding tunnel information with the NF Service Producer (e.g. source I-SMF).

The NF Service Consumer (e.g. target I-SMF) shall request the SMF to establish one or more downlink and/or uplink indirect data forwarding tunnels, as follows.

Figure 5.2.2.3.13-1: Indirect Data Forwarding Tunnel establishment during N2 based Handover with I-SMF

1.	The NF Service Consumer shall send a POST request, as specified in clause 5.2.2.3.1, with the following information:

-	dataForwarding attribute set to true, for the N2 based handover with I-SMF insertion/change/removal;

-	n9DlForwardingTnlList attribute carrying the N9 downlink indirect data forwarding tunnel(s) info of target I-UPF;

-	n9UlForwardingTnlList attribute carrying the N9 uplink indirect data forwarding tunnel(s) info of target I-UPF;

-	other information, if necessary.

2a.	Same as step 2a of Figure 5.2.2.3.1-1, with the following information:

-	n3DlForwardingTnlList attribute carrying the N3 downlink indirect data forwarding tunnel(s) info of source I-UPF or source UPF;

-	n3UlForwardingTnlList attribute carrying the N3 uplink indirect data forwarding tunnel(s) info of source I-UPF or source UPF;

-	other information, if necessary.

2b.	If the source SMF cannot proceed with the request, the source I-SMF shall return an error response, as specified for step 2b of figure 5.2.2.3.1-1.

##### 5.2.2.3.13A	Indirect Data Forwarding Tunnel removal during N2 based Handover with I-SMF

During N2 based handover cancellation with I-SMF insertion/change/removal, the NF Service Consumer (e.g. target I-SMF) shall use this procedure to remove previously established Indirect Data Forwarding Tunnel(s) at NF Service Producer (e.g. source I-SMF).

The NF Service Consumer (e.g. target I-SMF) shall request the NF service producer to remove the established Indirect Data Forwarding Tunnel(s), as follows.

Figure 5.2.2.3.13A-1: Indirect Data Forwarding Tunnel Removal during N2 based Handover with I-SMF

1.	The NF Service Consumer shall send a POST request, as specified in clause 5.2.2.3.1, with the following information:

-	dataForwarding attribute set to false;

-	other information, if necessary.

2a.	If successful, the SMF shall return a 204 No Content response.

2b.	If the SMF cannot proceed with the request, the SMF shall return an error response, as specified for step 2b of figure 5.2.2.3.1-1.

##### 5.2.2.3.14	Request to forward buffered downlink data packets at I-UPF

For I-SMF change or I-SMF removal when downlink data packets are buffered at the I-UPF, the new I-SMF (for I-SMF change) or SMF (for I-SMF removal) shall request the (old) I-SMF to forward buffered downlink data packets as following:

Figure 5.2.2.3.14-1: Request to forward buffered downlink data packets at I-UPF

1.	The NF Service Consumer shall send a POST request, as specified in clause 5.2.2.3.1, with the following information:

-	n9ForwardingTunnel IE indicating the allocated tunnel endpoints information to receive the buffered downlink data packets.

2a.	On success, the SMF shall initiate N4 session modification to the I-UPF trigger the sending of buffered DL data towards received tunnel endpoints and shall return "204 No Content" response.

2b.	If the SMF cannot proceed with the request, the SMF shall return an error response, as specified for step 2b of figure 5.2.2.3.1-1.

##### 5.2.2.3.15	Connection Suspend procedure

The NF Service Consumer (e.g. AMF) shall request the SMF to suspend the User Plane connection of an existing PDU session, as follows.

Figure 5.2.2.3.15-1: Connection Suspend

1.	The NF Service Consumer shall request the SMF to suspend the user plane connection of the PDU session by sending a POST request, as specified in clause 5.2.2.3.1, with the following information:

-	upCnxState attribute set to SUSPENDED;

-	user location and user location timestamp;

-	other information, if necessary.

2.	Upon receipt of such a request, the SMF shall deactivate the N3 tunnel of the PDU session, set the upCnxState attribute to SUSPENDED and return a 200 OK response including the upCnxState attribute set to SUSPENDED.

##### 5.2.2.3.16	Connection Resume in CM-IDLE with Suspend procedure

The NF Service Consumer (e.g. AMF) shall request the SMF to resume the User Plane connection of an existing PDU session, i.e. establish the N3 tunnel between the 5G-AN and UPF, as follows.

Figure 5.2.2.3.16-1: Connection Resume in CM-IDLE with Suspend

1.	The NF Service Consumer shall request the SMF to resume the user plane connection of the PDU session by sending a POST request, as specified in clause 5.2.2.3.1, with the following information:

-	the upCnxState attribute set to ACTIVATING;

-	user location and user location timestamp;

-	cause attribute set to "PDU_SESSION_RESUMED";

-	N2 SM information received from the 5G-AN, including the new transport layer address and tunnel endpoint of the downlink termination point for the user data for this PDU session (i.e. 5G-AN's GTP-U F-TEID for downlink traffic);

-	additional N2 SM information received from the 5G-AN, if any;

-	the "MO Exception Data Counter" if the UE has accessed the network by using "MO exception data" RRC establishment cause;

-	other information, if necessary.

2a.	If the SMF can proceed with resuming the user plane connection of the PDU session, the SMF shall return a 200 OK response including the following information:

-	the upCnxState attribute set to ACTIVATED;

-	N2 SM information, including the transport layer address and tunnel endpoint of the uplink termination point for the user data for this PDU session (i.e. UPF's GTP-U F-TEID for uplink traffic).

If the "MO Exception Data Counter is included in the request and Small Data Rate Control is enabled for the PDU session, the V-SMF shall update the H-SMF (see clause 5.2.2.8.2.2) for HR PDU Session (or I-SMF shall update the SMF for PDU session with I-SMF).

2b.	If the SMF cannot proceed with resuming the user plane connection of the PDU session, the SMF shall return an error response, as specified for step 2b of figure 5.2.2.3.1-1, including:

-	the upCnxState attribute representing the final state of the user plane connection (e.g. SUSPENDED);

-	N2 SM information, including the cause of the failure.

##### 5.2.2.3.17	AMF requested PDU Session Release due to Network Slice-Specific Authentication and Authorization failure or revocation

The requirements specified in clause 5.2.2.3.1 shall apply with the following modifications.

1.	Same as step 1 of Figure 5.2.2.3.1-1, with the following modifications.

The POST request shall contain:

-	the release IE set to true;

-	the cause IE set to REL_DUE_TO_SLICE_NOT_AUTHORIZED.

##### 5.2.2.3.18	5GS to EPS Idle mode mobility using N26 interface with data forwarding

The NF Service Consumer (e.g. AMF) shall request the SMF to forward buffered DL data towards the EPS during a 5GS to EPS Idle mode mobility using N26 interface with data forwarding (see 4.11.1.3.2A of 3GPP TS 23.502 [3]), as follows.

Figure 5.2.2.3.18-1: 5GS to EPS Idle mode mobility using N26 interface with data forwarding

1.	The NF Service Consumer shall send a POST request, as specified in clause 5.2.2.3.1, with the following information:

-	forwardingFTeid received from the MME in the Context Acknowdge, if any; or

-	forwarding bearer contexts received from the MME in Context Acknowdge, if any.

2a.	Upon receipt of such a request, the SMF shall forward the buffered DL data on the forwarding tunnel(s).

2b.	If the SMF cannot proceed with the request, the SMF shall return an error response, as specified for step 2b of figure 5.2.2.3.1-1.

##### 5.2.2.3.19	AMF requested PDU Session Release due to Control Plane Only indication associated with PDU Session is not applicable any longer

The requirements specified in clause 5.2.2.3.1 shall apply with the following modifications.

1.	Same as step 1 of Figure 5.2.2.3.1-1, with the following modifications.

The POST request shall contain:

-	the release IE set to true;

-	the cause IE set to REL_DUE_TO_CP_ONLY_NOT_APPLICABLE.

#### 5.2.2.4	Release SM Context service operation

##### 5.2.2.4.1	General

The Release SM Context service operation shall be used to release the SM Context of a given PDU session, in the SMF, in the V-SMF for HR roaming scenarios, or in the I-SMF for a PDU session with an I-SMF, in the following procedures:

-	Registration procedure with I-SMF/V-SMF change and removal (see clause 4.23.3 of 3GPP TS 23.502 [3]);

-	UE Triggered Service Request with I-SMF change and removal or V-SMF change (see clause 4.23.4.3 of 3GPP TS 23.502 [3]);

-	UE initiated Deregistration (see clause 4.2.2.3.2 of 3GPP TS 23.502 [3]);

-	Network initiated Deregistration, e.g. AMF initiated deregistration (see clause 4.2.2.3.3 of 3GPP TS 23.502 [3]), UDM triggered deregistration by sending Deregistration notification with initial Registration indication (see clause 4.2.2.2.2 of 3GPP TS 23.502 [3]);

-	Network requested PDU session release (see clause 4.3.4.2 of 3GPP TS 23.502 [3]), e.g. AMF initiated release when:

-	there is a mismatch of the PDU session status between the UE and the; or

-	there is a change of the set of network slices for a UE where a network slice instance is no longer available (as described in 3GPP TS 23.501 [2], clauses 5.15.5.2.2 and 4.2.2.2) and the PDU session is not activated;

-	5GS to EPS Idle mode mobility or handover, to release the SM context in the V-SMF only for a Home Routed PDU session or in the I-SMF only for a PDU session with an I-SMF (see clauses 4.23.12.2 and 4.23.12.6 of 3GPP TS 23.502 [3]), for the PDU sessions that are transferred to EPC;

-	5GS to EPS handover using N26 interface and 5GS to EPS Idle mode mobility using N26, to release the PDU session not transferred to EPC (see clauses 4.11.1.2.1 and 4.11.1.3.2 of 3GPP TS 23.502 [3]);

-	Inter NG-RAN node Xn based handover and N2 based handover with I-SMF change and removal;

-	5G-SRVCC from NG-RAN to 3GPP UTRAN procedure (see clause 6.5.4 of 3GPP TS 23.216 [35]);

-	5G-RG Deregistration via W-5GAN (see clause 7.2.1.2 of 3GPP TS 23.316 [36]);

-	FN-RG Deregistration via W-5GAN (see clause 7.2.1.4 of 3GPP TS 23.316 [36]);

-	Non-5G capable device behind 5G-CRG and FN-CRG Deregistration via W-5GAN (see clause 4.10a of 3GPP TS 23.316 [36]);

-	5G-RG or Network requested PDU Session Release via W-5GAN (see clause 7.3.3 of 3GPP TS 23.316 [36]);

-	FN-RG or Network Requested PDU Session Release via W-5GAN (see clause 7.3.7 of 3GPP TS 23.316 [36]);

-	Non-5G capable device behind 5G-CRG and FN-CRG or Network Requested PDU Session Release via W-5GAN (see clause 4.10a of 3GPP TS 23.316 [36]);

-	Mobility procedures with AMF changes (e.g. Registration / N2 based handover with AMF changes), to release the MA-PDU session if target AMF does not support MA-PDU session (see clause 4.22.9 of 3GPP TS 23.502 [3]).

The SMF shall release the SM context without sending any signalling towards the 5G-AN and the UE.

The NF Service Consumer (e.g. AMF) shall release the SM Context of a given PDU session by using the HTTP "release" custom operation as shown in Figure 5.2.2.4.1-1.

Figure 5.2.2.4.1-1: SM context release

1.	The NF Service Consumer shall send a POST request to the resource representing the individual SM context to be deleted. The payload body of the POST request shall contain any data that needs to be passed to the SMF and/or N2 SM information (if Secondary RAT usage data needs to be reported).

For a 5GS to EPS Idle mode mobility or handover, for a Home Routed PDU session associated with 3GPP access and with assigned EBI(s), the POST request shall contain the vsmfReleaseOnly indication; for a PDU session with an I-SMF and assigned EBI(s), the POST request shall contain the ismfReleaseOnly indication.

For a 5GS to EPS Idle mode mobility or handover, for a Home Routed PDU session associated with 3GPP access and with no assigned EBI(s), the POST request shall not contain the vsmfReleaseOnly indication to release the PDU session in the V-SMF and H-SMF; for a PDU session with an I-SMF and with no assigned EBI(s), the POST request shall not contain the ismfReleaseOnly indication to release the PDU session in the I-SMF and SMF.

For Registration, UE Triggered Service Request, Inter NG-RAN node Xn based handover and N2 based handover procedures with I-SMF change or removal, the POST request shall contain the ismfReleaseOnly indication; if with V-SMF change or removal, the POST request shall contain the vsmfReleaseOnly indication.

For 5G-SRVCC from NG-RAN to 3GPP UTRAN, the POST request body shall contain the "cause" attribute with the value "REL_DUE_TO_PS_TO_CS_HO".

2a.	On success, the SMF shall return a "200 OK" with message body containing the representation of the SmContextReleasedData when information needs to be returned to the NF Service Consumer, or a "204 No Content" response with an empty payload body in the POST response.

If the POST request contains a vsmfReleaseOnly indication (i.e. for a 5GS to EPS Idle mode mobility or handover, for a Home Routed PDU session with assigned EBI(s)), the V-SMF shall release its SM context and corresponding PDU session resource locally, i.e. without signalling towards the H-SMF.

If the POST request contains an ismfReleaseOnly indication (i.e. for a 5GS to EPS Idle mode mobility or handover, for a PDU session with an I-SMF and assigned EBI(s)), the I-SMF shall release its SM context and corresponding PDU session resource locally, i.e. without signalling towards the SMF.

If the POST request body contains the "cause" attribute with the value "REL_DUE_TO_PS_TO_CS_HO", the SMF shall indicate to the PCF within SM Policy Association termination that the PDU session is released due to 5G-SRVCC, or the cause value shall be passed from the V-SMF to the H-SMF (for a HR PDU session) or from the I-SMF to the SMF (for a PDU session with an I-SMF) within the Release service operation.

2b.	On failure or redirection, one of the HTTP status code listed in Table 6.1.3.3.4.3.2-2 shall be returned. For a 4xx/5xx response, the message body shall include a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.3.3.4.3.2-2.

#### 5.2.2.5	Notify SM Context Status service operation

##### 5.2.2.5.1	General

The Notify SM Context Status service operation shall be used by the SMF to notify the NF Service Consumer about the status of an SM context related to a PDU session (e.g. when the SM context is released and the release is not triggered by a Release SM Context Request, or when the SM context is moved to another system, or when the control of the PDU session is taken over by another I-SMF/V-SMF/SMF in the same SMF set) in the SMF, or in the V-SMF for HR roaming scenarios, or in the I-SMF for a PDU session with an I-SMF.

The Notify SM Context Status service operation may also be used by the SMF to provide the SMF derived CN assisted RAN parameters tuning to the NF Service Consumer (e.g. AMF), if the NF Service Consumer has indicated support of the CARPT (CN Assisted RAN Parameters Tuning) feature.

The Notify SM Context Status service operation may also be used by the SMF to notify the DDN failure status.

The Notify SM Context Status service operation may also be used to inform the NF service consumer (e.g. AMF) that the V-SMF has created the PDU session towards an alternative H-SMF for a HR PDU session or the I-SMF has created the PDU session towards an alternative SMF for a PDU session with I-SMF, during the PDU session establishment procedure.

It is used in the following procedures:

-	UE requested PDU Session Establishment procedure, when the PDU session establishment fails after the Create SM Context response or to provide the SMF derived CN assisted RAN parameters tuning, or when an alternative H-SMF is used by the V-SMF for a HR PDU session (see clause 4.3.2.2 of 3GPP TS 23.502 [3]), or when an alternative SMF is used by the I-SMF for a PDU session with an I-SMF (see clause 4.23.5.1 of 3GPP TS 23.502 [3]);

-	UE or Network requested PDU session Modification (see clause 4.3.3.2 of 3GPP TS 23.502 [3]) to provide the SMF derived CN assisted RAN parameters tuning;

-	UE or Network requested PDU session release (see clause 4.3.4.2 of 3GPP TS 23.502 [3]), e.g. SMF initiated release;

-	Handover of a PDU Session procedure between untrusted non-3GPP to 3GPP access (see clauses 4.9.2.3.2 and 4.9.2.4.2 of 3GPP TS 23.502 [3]);

-	Interworking procedures without N26 interface, e.g. 5GS to EPS Mobility (see clause 4.11.2.2 of 3GPP TS 23.502 [3]);

-	Handover from 5GC-N3IWF to EPS (see clause 4.11.3.2 of 3GPP TS 23.502 [3]);

-	Handover from 5GS to EPC/ePDG (see clause 4.11.4.2 of 3GPP TS 23.502 [3]);

-	I-SMF Context Transfer (see clause 4.26.5.2 of 3GPP TS 23.502 [3]);

-	SMF Context Transfer procedure, LBO or no Roaming, no I-SMF (see clause 4.26.5.3 of 3GPP TS 23.502 [3]);

-	Handover from W-5GAN/5GC to 3GPP access/EPS (see clause 7.6.4.2 of 3GPP TS 23.316 [36]);

-	5G-RG requested PDU Session Establishment via W-5GAN (see clause 7.3.1 of 3GPP TS 23.316 [36]);

-	5G-RG or Network requested PDU Session Modification via W-5GAN (see clause 7.3.2 of 3GPP TS 23.316 [36]);

-	5G-RG or Network requested PDU Session Release via W-5GAN (see clause 7.3.3 of 3GPP TS 23.316 [36])

-	FN-RG related PDU Session Establishment via W-5GAN (see clause 7.3.4 of 3GPP TS 23.316 [36]);

-	FN-RG or Network Requested PDU Session Modification via W-5GAN (see clause 7.3.6 of 3GPP TS 23.316 [36]);

-	FN-RG or Network Requested PDU Session Release via W-5GAN (see clause 7.3.7 of 3GPP TS 23.316 [36]);

-	Non-5G capable device behind 5G-CRG and FN-CRG requested PDU Session Establishment via W-5GAN (see clause 4.10a of 3GPP TS 23.316 [36]);

-	Non-5G capable device behind 5G-CRG and FN-CRG or Network requested PDU Session Modification via W-5GAN (see clause 4.10a of 3GPP TS 23.316 [36]);

-	Non-5G capable device behind 5G-CRG and FN-CRG or Network requested PDU Session Release via W-5GAN (see clause 4.10a of 3GPP TS 23.316 [36]);

-	Handover between 3GPP access/5GC and W-5GAN access (see clause 7.6.3 of 3GPP TS 23.316 [36]);

-	Handover from 3GPP access/EPS to W-5GAN/5GC (see clause 7.6.4.1 of 3GPP TS 23.316 [36]);

-	Information flow for Availability after DDN Failure with SMF buffering (see clause 4.15.3.2.7 of 3GPP TS 23.502 [3]);

-	Information flow for Availability after DDN Failure with UPF buffering (see clause 4.15.3.2.9 of 3GPP TS 23.502 [3]);

-	The control of the PDU session is taken over by a new anchor SMF within the same SMF set (see clause 5.22 of 3GPP TS 29.244 [29]) or taken over by a new intermediate SMF (e.g. I-SMF or V-SMF) within the same SMF set, and the new SMF instance decides to send an updated binding indication to the NF service consumer (as per step 6 of clause 6.5.3.3 of 3GPP TS 29.500 [4]);

-	I-SMF selection per DNAI (see clause 4.23.5.4 of 3GPP TS 23.502 [3]);

-	Change of SSC mode 2 PDU Session Anchor with different PDU Sessions (see clause 4.3.5.1 of 3GPP TS 23.502 [3]);

-	Change of SSC mode 3 PDU Session Anchor with multiple PDU Sessions (see clause 4.3.5.2 of 3GPP TS 23.502 [3]).

The SMF shall notify the NF Service Consumer by using the HTTP POST method as shown in Figure 5.2.2.5.1-1.

Figure 5.2.2.5.1-1: SM context status notification

1.	The SMF shall send a POST request to the SM Context Status callback reference provided by the NF Service Consumer during the subscription to this notification. The payload body of the POST request shall contain the notification payload.

If the notification is triggered by PDU session handover to release resources of the PDU session in the source access, the notification payload shall contain the resourceStatus IE with the value "RELEASED" and the Cause IE with the value "PDU_SESSION_HANDED_OVER" as specified in clause 4.9.2.3.2 of 3GPP TS 23.501 [2].

If the notification is triggered by PDU session handover to release resources of the PDU session in the target access due to handover failure between 3GPP access and non-3GPP access, the notification payload shall contain the resourceStatus IE with the value "RELEASED" and the Cause IE with the value "PDU_SESSION_HAND_OVER_FAILURE".

If the NF Service Consumer indicated support of the HOFAIL feature (see clause 6.1.8) and if the notification is triggered by PDU session handover to update the access type of the PDU session due to a handover failure between 3GPP access and non-3GPP access, the notification payload shall contain the resourceStatus IE with the value "UPDATED", the anType IE with the value "3GPP" or "NON_3GPP" indicating the access type of the PDU session after the handover failure scenario and the Cause IE with the value "PDU_SESSION_HAND_OVER_FAILURE".

If the notification is triggered by the SMF derived CN assisted RAN parameters tuning, the notification payload shall contain the resourceStatus IE with the value "UNCHANGED" and the Cause IE with the value "CN_ASSISTED_RAN_PARAMETER_TUNING".

If the notification is triggered by SMF Context Transfer procedure, the notification payload shall contain the Cause IE with the value "ISMF_CONTEXT_TRANSFER" or "SMF_CONTEXT_TRANSFER".

If the notification is triggered by the report of the DDN failure, the notification payload shall contain the resourceStatus IE with the value "UNCHANGED" and the Cause IE with the value "DDN_FAILURE_STATUS".

If the notification is triggered to report that an alternative (H-)SMF has been used during a HR PDU session establishment or the establishment of a PDU session with an I-SMF, the notification payload shall contain the resourceStatus IE with the value "ALT_ANCHOR_SMF". The notification payload shall also include the altAnchorSmfUri IE containing the API URI of the alternative (H-)SMF used for the PDU session and if available the altAnchorSmfId IE containing the NF Instance Id of the alternative (H-)SMF. The Notification shall only be sent to the NF service consumer (e.g. AMF) supporting the AASN feature.

For a PDU session without an I-SMF or V-SMF, if upon a change of anchor SMF, the new anchor SMF instance decides to send an updated binding indication to the NF service consumer (as per step 6 of clause 6.5.3.3 of 3GPP TS 29.500 [4]), then it shall contain the resourceStatus IE with the value "UPDATED" and the Cause IE with the value "CHANGED_ANCHOR_SMF" in the notification payload, and carry an updated binding indication in the HTTP headers to indicate the change of anchor SMF.

For a PDU session with an I-SMF or V-SMF, if upon a change of intermediate SMF (e.g. I-SMF or V-SMF), the new intermediate SMF instance decides to send an updated binding indication to the NF service consumer (as per step 6 of clause 6.5.3.3 of 3GPP TS 29.500 [4]), then it shall contain the resourceStatus IE with the value "UPDATED" and the Cause IE with the value "CHANGED_INTERMEDIATE_SMF" in the notification payload, and carry an updated binding indication in the HTTP headers to indicate the change of intermediate SMF.

If the notification is triggered by SMF for I-SMF selection for the current PDU session, or SMF selection during PDU Session re-establishment for SSC mode 2/3, the notification payload shall contain the resourceStatus IE with the value "UNCHANGED", the Cause IE with the value "TARGET_DNAI_NOTIFICATION" and the targetDnaiInfo IE. If the notification is triggered for SMF selection during PDU Session re-establishment for SSC mode 3, the notification payload may also contain the oldSmContextRef IE as specified in clause 4.3.5.2 of 3GPP TS 23.502 [3].

2a.	On success, "204 No Content" shall be returned and the payload body of the POST response shall be empty.

If the SMF indicated in the request that the SM context resource is released, the NF Service Consumer shall release its association with the SMF for the PDU session and release the EBI(s) that were assigned to the PDU session.

If the SMF indicated in the request that the SM context resource is updated with the anType IE, the NF Service Consumer shall change the access type of the PDU session with the value of anType IE.

2b.	On failure or redirection, one of the HTTP status code listed in Table 6.1.3.7.3.1-2 shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.3.7.3.1-2.

If the NF Service Consumer (e.g. AMF) is not able to handle the notification but knows by implementation specific means that another NF Service Consumer (e.g. AMF) is able to handle the notification (e.g. AMF deployment with Backup AMF), it shall reply with an HTTP "307 temporary redirect" response pointing to the URI of the new NF Service Consumer. If the NF Service Consumer is not able to handle the notification but another unknown NF Service Consumer could possibly handle the notification (e.g. AMF deployment with UDSF), it shall reply with an HTTP "404 Not found" error response.

If the SMF receives a "307 temporary redirect" response, the SMF shall use this URI as Notification URI in subsequent communication and shall resend the notification to that URI.

If the SMF becomes aware that a new NF Service Consumer (e.g. AMF) is requiring notifications (e.g. via the "404 Not found" response or via Namf_Communication service AMFStatusChange Notifications, or via link level failures, see clause 6.5.2 of 3GPP TS 29.500 [4]), and the SMF knows alternate or backup Address(es) where to send Notifications (e.g. via the GUAMI and/or backupAmfInfo received when the SM context was established or via AMFStatusChange Notifications, or via the Nnrf_NFDiscovery service specified in 3GPP TS 29.510 [19] using the service name and GUAMI or backupAMFInfo obtained during the creation of the SM context, see clause 6.5.2.2 of 3GPP TS 29.500 [4]), the SMF shall exchange the authority part of the corresponding Notification URI with one of those addresses and shall use that URI in subsequent communication; the SMF shall resend the notification to that URI.

#### 5.2.2.6	Retrieve SM Context service operation

##### 5.2.2.6.1	General

The Retrieve SM Context service operation shall be used to retrieve an individual SM context, for a given PDU session, from the (H-)SMF, from the V-SMF during change or removal of V-SMF, or from the I-SMF during change or removal of I-SMF.

It is used in the following procedures:

-	5GS to EPS handover using N26 interface (see clause 4.11.1.2.1 of 3GPP TS 23.502 [3]), for PDU sessions associated with 3GPP access;

-	5GS to EPS Idle mode mobility using N26 interface (see clause 4.11.1.3.2 of 3GPP TS 23.502 [3]), for PDU sessions associated with 3GPP access;

-	UE Triggered Service Request with I-SMF insertion/change/removal or with V-SMF insertion/change/removal (see clause 4.23.4.3 of of 3GPP TS 23.502 [3]);

-	Xn based inter NG-RAN handover with insertion of intermediate SMF (see clause 4.23.11 of 3GPP TS 23.502 [3]), for PDU sessions associated with 3GPP access;

-	Inter NG-RAN node N2 based handover, preparation phase, with I-SMF or V-SMF insertion/change (see clause 4.23.7.3.2 of 3GPP TS 23.502 [3]), for PDU sessions associated with 3GPP access;

-	SMF Context Transfer procedure, LBO or no Roaming, no I-SMF (see clause 4.26.5.3 of 3GPP TS 23.502 [3]), for PDU sessions associated with 3GPP access;

-	I-SMF selection per DNAI (see clause 4.23.5.4 of 3GPP TS 23.502 [3]);

-	Change of SSC mode 3 PDU Session Anchor with multiple PDU Sessions (see clause 4.3.5.2 of 3GPP TS 23.502 [3]).

Editor' Note: How to retrieve the AF Coordination Information when the PDU session established with the old SMF has an I-SMF is FFS.

The NF Service Consumer (e.g. AMF or SMF) shall retrieve an SM context by using the HTTP POST method (retrieve custom operation) as shown in Figure 5.2.2.6.1-1.

Figure 5.2.2.6.1-1: SM context retrieval

1.	The NF Service Consumer shall send a POST request to the resource representing the individual SM context to be retrieved. The POST request may contain a payload body with the following parameters:

-	target MME capabilities, if available, to allow the SMF to determine whether to include EPS bearer contexts for Ethernet PDN Type, non-IP PDN type or not;

-	SM context type:

-	indicating that this is a request to retrieve the complete SM context (i.e. 5G SM context including EPS context information as defined in clause 6.1.6.2.39), during scenarios with an I-SMF or V-SMF insertion/change/removal or SMF Context Transfer procedure; or

-	indicating that this is a request to retrieve the AF Coordination Information as defined in clause 6.1.6.2.x, during the change of SSC mode 3 PDU Session Anchor with multiple PDU Sessions, if the runtime coordination between old SMF and AF is enabled.

-	serving core network operator PLMN ID of the new V-SMF, when the procedure is triggered by a new V-SMF, if the new V-SMF supports inter-PLMN V-SMF change;

-	notToTransferEbiList IE, if the SM context type IE is absent or indicate a request to retrieve the EPS PDN connection, to request the SMF to not transfer EPS bearer context(s) corresponding to EBIs in the list, during an 5GS to EPS mobility when the target MME does not support 15 EPS bearers;

-	ranUnchangedInd IE, if the NG-RAN Tunnel info is required in scenario of I-SMF/V-SMF change/insertion during registration procedure after EPS to 5GS handover or I-SMF selection per DNAI, when the UE is in CM-CONNECTED state as specified in clauses 5.2.2.2.7 and 5.2.2.2.12.

2a.	On success, "200 OK" shall be returned; the payload body of the POST response shall contain the mapped EPS bearer contexts if this is a request for the UE EPS PDN connection, or the complete SM context if this is a request for retrieving the complete SM context, or the AF Coordination Information if this is a request for retrieving the AF Coordination Information. 

If this is a request for the UE EPS PDN connection and the target MME capabilities were provided in the request parameters:

-	if the target MME supports the non-IP PDN type, the SMF shall return, for a PDU session with PDU session type "Unstructured", an EPS bearer context with the "non-IP" PDN type;

-	if the target MME supports the Ethernet PDN type, the SMF shall return, for a PDU session with PDU session type "Ethernet", an EPS bearer context with the "Ethernet" PDN type;

-	if the target MME does not support the Ethernet PDN type but supports the non-IP PDN type, the SMF shall return, for a PDU session with PDU session type "Ethernet", an EPS bearer context with the "non-IP" PDN type.

If the notToTransferEbiList IE was included in the request, the SMF shall not provide EPS bearer context(s) corresponding to EBIs in the list.

If this is a request for retrieving the complete SM context and there are downlink data packets buffered at I-UPF, the SMF shall include the "forwardingInd" attribute with value "true" in the response body to indicate downlink data packets are buffered at the I-UPF. The NF Service Consumer receiving the "forwardingInd" attribute with the value "true" shall setup a forwarding tunnel for receiving the buffered downlink data packets.

If this is a request for retrieving the complete SM context for an inter-PLMN V-SMF change, i.e. if the request contains the serving core network operator PLMN ID indicating a different PLMN than the PLMN of the SMF (acting as the old V-SMF), the latter shall not include the chargingInfo IE and the roamingChargingProfile IE in the SM context returned in the response.

2b.	On failure or redirection, one of the HTTP status code listed in Table 6.1.3.3.4.4.2-2 shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.3.3.4.4.2-2.

If the EBI value of the QoS Flow associated with the default QoS Rule is included in the notToTransferEbiList IE, the SMF shall set the "cause" attribute in the ProblemDetails structure to "DEFAULT_EBI_NOT_TRANSFERRED".

#### 5.2.2.7	Create service operation

##### 5.2.2.7.1	General

The Create service operation shall be used to create an individual PDU session in the H-SMF for HR roaming scenarios, or in the SMF for PDU sessions involving an I-SMF.

It is used in the following procedures:

-	UE requested PDU Session Establishment with or without an I-SMF insertion (see clauses 4.3.2.2.2 and 4.23.5.1 of 3GPP TS 23.502 [3]);

-	when an I-SMF is inserted during the Registration, Service Request, Inter NG-RAN node N2 based handover, Xn based handover, Handover from EPC/ePDG to 5GS and Handover from non-3GPP to 3GPP access procedures (see clauses 4.23.3, 4.23.4, 4.23.7.3, 4.23.11.2 and 4.23.16 of 3GPP TS 23.502 [3]);

-	EPS to 5GS Idle mode mobility or handover using N26 interface (see clauses 4.11, 4.23.12.3, 4.23.12.5 and 4.23.12.7 of 3GPP TS 23.502 [3]);

-	EPS to 5GS mobility without N26 interface (see clause 4.11.2.3 of 3GPP TS 23.502 [3]);

-	Handover of a PDU session between 3GPP access and non-3GPP access, when the target AMF does not know the SMF resource identifier of the SM context used by the source AMF, e.g. when the target AMF is not in the PLMN of the N3IWF (see clause 4.9.2.3.2 of 3GPP TS 23.502 [3]);

-	Handover from EPS to 5GC-N3IWF (see clause 4.11.3.1 of 3GPP TS 23.502 [3]);

-	Handover from EPC/ePDG to 5GS (see clause 4.11.4.1 of 3GPP TS 23.502 [3]).

The NF Service Consumer (e.g. V-SMF or I-SMF) shall create a PDU session in the SMF (i.e. H-SMF for a HR PDU session, or SMF for a PDU session involving an I-SMF) by using the HTTP POST method as shown in Figure 5.2.2.7.1-1.

Figure 5.2.2.7.1-1: PDU session creation

1.	The NF Service Consumer shall send a POST request to the resource representing the PDU sessions collection resource of the SMF. The payload body of the POST request shall contain:

-	a representation of the individual PDU session resource to be created;

-	the requestType IE, if the Request type IE is received from the UE for a single access PDU session and if the request refers to an existing PDU session or an existing Emergency PDU session; the requestType IE shall not be included for a MA-PDU session establishment request; it may be included otherwise;

-	the indication that a MA-PDU session is requested if a MA-PDU session is requested to be established by the UE, or the indication that the PDU session is allowed to be upgraded to a MA PDU session if the UE indicated so;

-	the vsmfId IE or ismfId IE identifying the V-SMF or I-SMF respectively;

-	the cpCiotEnabled IE with the value "True", if Control Plane CIoT 5GS Optimisation is enabled for this PDU session;

-	the cpOnlyInd IE with the value "True", if the PDU session shall only use Control Plane CIoT 5GS Optimisation;

-	the Invoke NEF indication with the value "True", if the cpCiotEnabled IE is set to "True" and data delivery via NEF is selected for the PDU session;

-	the vcnTunnelInfo IE or icnTunnelInfo IE with the N9 tunnel information of the UPF controlled by the V-SMF or I-SMF respectively, except for EPS to 5GS handover using N26 interface and when Control Plane CIoT 5GS Optimisation is enabled and data delivery via NEF is selected for this PDU session;

-	the additionalCnTunnelInfo IE with additional N9 tunnel information, if a MA PDU session is requested or if the PDU session is allowed to be upgraded to a MA PDU session, and if the UE is registered over both 3GPP and Non-3GPP accesses;

-	the anType IE, indicating the access network type (3GPP or non-3GPP access) associated to the PDU session;

-	the additionalAnType IE indicating an additional access network type associated to the PDU session, for a MA PDU session, if the UE is registered over both 3GPP and Non-3GPP accesses;

-	the n9ForwardingTunnelInfo IE indicating the allocated N9 tunnel endpoints information for receiving the buffered downlink data packets, when downlink data packets are buffered at I-UPF controlled by the SMF during I-SMF insertion;

-	a callback URI ({vsmfPduSessionUri} or {ismfPduSessionUri}) representing the PDU session resource in the V-SMF or I-SMF. The SMF shall construct the callback URIs based on the received {vsmfPduSessionUri} or {ismfPduSessionUri} as defined in clause 6.1, e.g. the callback URI "{vsmfPduSessionUri}/modify" to modify a PDU session in the V-SMF;

-	the list of DNAIs supported by the I-SMF, for a PDU session with an I-SMF;

-	the QoS constraints from the VPLMN for the QoS Flow associated with the default QoS rule and/or for the Session-AMBR if any, for the HR PDU session, if the VQOS feature is supported by the V-SMF.

As specified in clause 4.3.2.2.2 of 3GPP TS 23.502 [3], the NF Service Consumer shall be able to receive an Update request before receiving the Create Response, e.g. for EPS bearer ID allocation (see clause 4.11.1.4.1 of 3GPP TS 23.502 [3]) or Secondary authorization/authentication (see clause 4.3.2.3 of 3GPP TS 23.502 [3]).

NOTE:	If the H-SMF supports the VQOS feature, when QoS constraints are received from the VPLMN and PCF is deployed, the H-SMF provides the QoS constraints from the VPLMN to the PCF; otherwise, in case dynamic PCC is not deployed, the SMF takes them into account when generating the default QoS rule.

2a.	On success, "201 Created" shall be returned, the payload body of the POST response shall contain:

-	the representation describing the status of the request;

-	the QoS flow(s) to establish for the PDU session, except when Control Plane CIoT 5GS Optimisation is enabled for this PDU session;

-	the epsPdnCnxInfo IE and, for each EPS bearer, an epsBearerInfo IE, if the PDU session is associated to (or handed over to) the 3GPP access type and may be moved to EPS during its lifetime;

-	a MA PDU Session Accepted indication, if a MA PDU session is established;

-	the smallDataRateControlEnabled indication set to "true" if small data rate control is applicable on the PDU session;

-	the "Location" header containing the URI of the created resource.

The payload body of the POST response may also contain the upSecurity, maxIntegrityProtectedDataRateUl and maxIntegrityProtectedDataRateDl IEs, if the PDU session is associated to (or handed over to) the 3GPP access type.

The SMF may provide alternative QoS profiles for each GBR QoS flow with Notification control enabled, to allow the NG-RAN to accept the setup of the QoS flow if the requested QoS parameters or at least one of the alternative QoS parameters sets can be fulfilled at the time of setup.

The authority and/or deployment-specific string of the apiRoot of the created resource URI may differ from the authority and/or deployment-specific string of the apiRoot of the request URI received in the POST request.

If an Update Request was sent to the NF Service Consumer before the Create Response, the URI in the "Location" header and in the hsmfPduSessionUri IE (or smfPduSessionUri IE for a PDU session with an I-SMF) of the SMF initiated Update Request shall be the same. If the requestType IE was received in the request and set to EXISTING_PDU_SESSION or EXISTING_EMERGENCY_PDU_SESSION (i.e. indicating that this is a UE request for an existing PDU session or an existing emergency PDU session), the SMF shall identify the existing PDU session or emergency PDU session based on the PDU Session ID; in this case, the SMF shall not create a new PDU session or emergency PDU session but instead update the existing PDU session or emergency PDU session and provide the representation of the updated PDU session or emergency PDU session in the response to the NF Service Consumer.

The POST request shall be considered as colliding with an existing PDU session context if:

-	it includes the same SUPI, or PEI for an emergency registered UE without a UICC or without an authenticated SUPI, and the same PDU Session ID as for an existing PDU session context; and

-	this is a request to establish a new PDU session, i.e.:

-	the RequestType IE is present in the request and set to INITIAL_REQUEST or INITIAL_EMERGENCY_REQUEST (e.g. single access PDU session establishment request);

-	the RequestType IE and the maRequestInd IE are both absent in the request (e.g. EPS to 5GS mobility); or

-	the maRequestInd IE is present in the request (i.e. MA-PDU session establishment request) and the access type indicated in the request corresponds to the access type of the existing PDU session context.

A POST request that collides with an existing PDU session context shall be treated as a request for a new PDU session context. Before creating the new PDU session context, the SMF should delete the existing PDU session context locally and any associated resources in the UPF and PCF. See also clause 5.2.3.3.1 for the handling of requests which collide with an existing PDU session context. If the vsmfPduSessionUri or ismfPduSessionUri of the existing PDU session context differs from the vsmfPduSessionUri or ismfPduSessionUri received in the POST request, the SMF shall also send a status notification (see clause 5.2.2.10) targeting the vsmfPduSessionUri or ismfPduSessionUri of the existing PDU session context to notify the release of the existing PDU session context.

If the requestType IE was received in the request and indicates this is a request for a new PDU session (i.e. INITIAL_REQUEST) and if the Old PDU Session ID was also included in the request, the SMF shall identify the existing PDU session to be released and to which the new PDU session establishment relates, based on the Old PDU Session ID.

The NF Service Consumer shall store any epsPdnCnxInfo and EPS bearer information received from the SMF.

If the response received from the SMF contains the alwaysOnGranted attribute set to true, the NF Service Consumer shall check and determine whether the PDU session can be established as an always-on PDU session based on local policy.

If no GPSI IE is provided in the request, e.g. for a PDU session moved from another access or another system, and the SMF knows that a GPSI is already associated with the PDU session, the SMF shall include the GPSI in the response.

If one or more requested QoS flow(s) fail to be established, the V-SMF or I-SMF shall send an Update Request including the qosFlowsRelNotifyList attribute to report the failure to the H-SMF or SMF (see clause 5.2.2.8.2.2), or a Release Request to release the PDU session if no QoS flow can be established (see clause 5.2.2.9).

For UE mobility with I-SMF/V-SMF insertion procedure, if a requested functionality is not supported for a PDU session with an I-SMF/V-SMF, the SMF shall accept the POST request and release the PDU Session after the mobility procedure, as specified in clause 4.23.1 of 3GPP TS 23.502 [3].

2b.	On failure, or redirection during a UE requested PDU Session Establishment, one of the HTTP status code listed in Table 6.1.3.5.3.1-3 shall be returned. For a 4xx/5xx response, the message body shall contain a PduSessionCreateError structure, including:

-	a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.3.5.3.1-3. The application error shall be set to "NOT_SUPPORTED_WITH_ISMF" during a UE requested PDU Session Establishment, if a requested functionality is not supported for a PDU session with an I-SMF/V-SMF.

-	the n1SmCause IE with the 5GSM cause that the SMF proposes the NF Service Consumer to return to the UE, if the request included n1SmInfoFromUe;

-	n1SmInfoToUe with any information to be sent to the UE (in the PDU Session Establishment Reject).

##### 5.2.2.7.2	EPS to 5GS Idle mode mobility

The requirements specified in clause 5.2.2.7.1 shall apply with the following modifications.

1.	Same as step 1 of Figure 5.2.2.7.1-1, with the following additions.

The POST request shall contain:

-	the list of EPS Bearer Ids received from the MME;

-	the PGW S8-C F-TEID received from the MME;

-	the epsBearerCtxStatus attribute, indicating the status of all the EPS bearer contexts in the UE, if corresponding information has been received in the Create SM Context request (see clause 5.2.2.2.2).

2a.	Same as step 2 of Figure 5.2.2.7.1-1, with the following modifications. 

If:

-	the SMF finds a corresponding PDU session based on the EPS Bearer Ids and PGW S8-C F-TEID received in the request;

-	the default EPS bearer context of the corresponding PDU session is not reported as inactive by the UE in the epsBearerCtxStatus attribute, if received; and

-	the SMF can proceed with moving the PDN connection to 5GS,

then the SMF shall return a 201 Created response including the following additional information:

-	PDU Session ID corresponding to the EPS PDN connection;

-	other PDU session parameters, such as PDU Session Type, Session AMBR, QoS flows information.

If the epsBearerCtxStatus attribute is received in the request, the SMF shall check whether some EPS bearer(s) of the corresponding PDU session have been deleted by the UE but not notified to the EPS, and if so, the SMF shall release these EPS bearers, corresponding QoS rules and QoS flow level parameters locally, as specified in clause 4.11.1.3.3 of 3GPP TS 23.502 [3].

NOTE:	The behaviour specified in this step also applies if the POST request collides with an existing PDU session context, i.e. if the POST request includes the same SUPI, or PEI for an emergency registered UE without a UICC or without an authenticated SUPI, and the received EPS bearer ID is the same as in the existing PDU session context.

2b.	Same as step 2b of Figure 5.2.2.7.1-1, with the following additions.

If the SMF determines that seamless session continuity from EPS to 5GS is not supported for the PDU session, the SMF shall set the "cause" attribute in the ProblemDetails structure to "NO_EPS_5GS_CONTINUITY".

If the default EPS bearer context of the PDU session is reported as inactive by the UE in the epsBearerCtxStatus attribute, the SMF shall set the "cause" attribute in the ProblemDetails structure to "DEFAULT_EPS_BEARER_INACTIVE".

##### 5.2.2.7.3	EPS to 5GS Handover Preparation

The requirements specified in clause 5.2.2.7.1 shall apply with the following modifications.

1.	Same as step 1 of Figure 5.2.2.7.1-1, with the following modifications.

The POST request shall contain:

-	the list of EPS Bearer Ids received from the MME;

-	the PGW S8-C F-TEID received from the MME;

-	the hoPreparationIndication IE set to "true", to indicate that a handover preparation is in progress and the PGW-C/SMF shall not switch the DL user plane of the PDU session yet.

2a.	Same as step 2 of Figure 5.2.2.7.1-1, with the following modifications.

If the SMF finds a corresponding PDU session based on the EPS Bearer Ids and PGW S8-C F-TEID received in the request, and if it can proceed with the procedure, the SMF shall return a 201 Created response including the following information:

-	PDU Session ID corresponding to the EPS PDN connection;

-	other PDU session parameters, such as PDU Session Type, Session AMBR, QoS flows information.

The SMF shall not switch the DL user plane of the PDU session, if the hoPreparationIndication IE was set to "true" in the request.

NOTE:	The behaviour specified in this step also applies if the POST request collides with an existing PDU session context, i.e. if the POST request includes the same SUPI, or PEI for an emergency registered UE without a UICC or without an authenticated SUPI, and the received EPS bearer ID is the same as in the existing PDU session context.

2b.	Same as step 2b of Figure 5.2.2.7.1-1, with the following additions.

If the H-SMF determines that seamless session continuity from EPS to 5GS is not supported for the PDU session, the H-SMF shall set the "cause" attribute in the ProblemDetails structure to "NO_EPS_5GS_CONTINUITY".

##### 5.2.2.7.4	N2 Handover Preparation with I-SMF Insertion

The requirements specified in clause 5.2.2.7.1 shall apply with the following modifications.

1.	Same as step 1 of Figure 5.2.2.7.1-1, with the following modifications.

The POST request shall contain:

-	the hoPreparationIndication IE set to "true", to indicate that a handover preparation is in progress and the SMF shall not switch the DL user plane of the PDU session yet.

2a.	Same as step 2 of Figure 5.2.2.7.1-1, with the following modifications:

The SMF shall not switch the DL user plane of the PDU session, if the hoPreparationIndication IE was set to "true" in the request.

##### 5.2.2.7.5	Xn Handover with I-SMF Insertion

The requirements specified in clause 5.2.2.7.1 shall apply with the following modifications.

1.	Same as step 1 of Figure 5.2.2.7.1-1, with the following modifications.

The POST request shall contain:

-	the upSecurityInfo IE, if received from the AMF.

2a.	Same as step 2 of Figure 5.2.2.7.1-1, with the following modifications:

The SMF shall verify that the upSecurity IE included in the received upSecurityInfo IE is same as the security policy for integrity protection and encryption that the SMF has locally stored. If there is a mismatch, the SMF shall send its locally stored security policy for integrity protection and encryption in upSecurity IE to NG-RAN as specified in clause 6.6.1 of 3GPP TS 33.501 [17].

##### 5.2.2.7.6	I-SMF/V-SMF restoration procedure

During I-SMF/V-SMF restoration procedure, when the new I-SMF/V-SMF receives CreateSMContext Request from the AMF, it shall send Create Request to the anchor SMF, to update the I-SMF/V-SMF information in the anchor SMF and retrieve SM Context related information from the anchor SMF, as specified in clause 6.7 of 3GPP TS 23.527 [24].

The requirements specified in clause 5.2.2.7.1 shall apply with the following modifications.

1.	Same as step 1 of Figure 5.2.2.7.1-1, with the following additional information:

-	an I-SMF/V-SMF restoration indication which indicates that I-SMF/V-SMF restoration procedure is triggered;

-	the PDU Session ID indicating the PDU session for which the I-SMF/V-SMF restoration procedure is invoked;

2a.	Same as step 2 of Figure 5.2.2.7.1-1, with the following modifications:

The anchor SMF shall detect the procedure is used for I-SMF/V-SMF restoration based on the I-SMF/V-SMF restoration indication, and thus update the I-SMF/V-SMF information. The anchor SMF shall prepare the SM context related information, and include such information in the response message.

#### 5.2.2.8	Update service operation

##### 5.2.2.8.1	General

The Update service operation shall be used for HR PDU sessions or for PDU sessions involving an I-SMF to:

-	update an individual PDU session in the H-SMF or SMF and/or provide the H-SMF or SMF with information received by the V-SMF or I-SMF in N1 SM signalling from the UE;

-	update a MA PDU session to indicate an additional access type, if the UE requests establishment of MA PDU session via the other access after the UE is registered to both 3GPPP access and non-3GPP access and the MA PDU session was successfully established on the first access (see clause 4.22.2.2 of 3GPP TS 23.502 [3]);

-	release a MA PDU session over a single access in the H-SMF or SMF;

-	update an individual PDU session in the V-SMF or I-SMF and/or provide information necessary for the V-SMF or I-SMF to send N1 SM signalling to the UE.

It is invoked by the V-SMF or I-SMF in the following procedures:

-	UE or network (e.g. V-SMF, I-SMF) requested PDU session modification (see clauses 4.3.3.3 and 4.23.5.3 of 3GPP TS 23.502 [3]);

-	UE or network (e.g. AMF, V-SMF, I-SMF) requested PDU session release (see clause 4.3.4.3 of 3GPP TS 23.502 [3]);

-	UE or network (e.g. AMF, V-SMF, I-SMF) initiated MA PDU session release over a single access (see clause 4.22 of 3GPP TS 23.502 [3]);

-	EPS to 5GS handover execution using N26 interface (see clause 4.11 of 3GPP TS 23.502 [3]);

-	Handover between 3GPP and untrusted or trusted non-3GPP access procedures (see clauses 4.9.2 and 4.9.3 of 3GPP TS 23.502 [3]), without AMF change or with target AMF in same PLMN;

-	All procedures requiring to provide the H-SMF or SMF with information received by the V-SMF or I-SMF in N1 SM signalling from the UE to the H-SMF or SMF;

-	Secondary RAT Usage Data Reporting (see clause 4.21 of 3GPP TS 23.502 [3]);

-	UPF anchored Mobile Originated Data Transport in Control Plane CIoT 5GS Optimisation (see clause 4.24.1 of 3GPP TS 23.502 [3]);

-	Connection Resume in CM-IDLE with Suspend procedure (see clause 4.8.2.3 of 3GPP TS 23.502 [3]).

It is invoked by the I-SMF in the following procedures:

-	Addition of PDU Session Anchor and Branching Point or UL CL controlled by I-SMF (see clause 4.23.9.1 of 3GPP TS 23.502 [3]);

-	Removal of PDU Session Anchor and Branching Point or UL CL controlled by I-SMF (see clause 4.23.9.2 of 3GPP TS 23.502 [3]);

-	Change of PDU Session Anchor for IPv6 multi-homing or UL CL controlled by I-SMF (see clause 4.23.9.3 of 3GPP TS 23.502 [3]);

-	Sending by I-SMF of N4 notifications related with traffic usage reporting (see clause 5.34.6 of 3GPP TS 23.501 [2]).

It is invoked by the H-SMF or SMF in the following procedures:

-	Network (e.g. H-SMF, SMF) requested PDU session modification (see clauses 4.3.3.3 and 4.23.5.3 of 3GPP TS 23.502 [3]);

-	Network (e.g. H-SMF, SMF) requested PDU session release (see clause 4.3.4.3 of 3GPP TS 23.502 [3]);

-	Network (e.g. H-SMF, SMF) initiated MA PDU session release over a single access (see clause 4.22 of 3GPP TS 23.502 [3]);

-	All procedures requiring to provide information necessary for the V-SMF or I-SMF to send N1 SM signalling to the UE;

-	EPS Bearer ID allocation or revocation (see clauses 4.11.1.4.1 and 4.11.1.4.3 of 3GPP TS 23.502 [3]);

-	Secondary authorization/authentication by an DN-AAA server (see clause 4.3.2.3 of of 3GPP TS 23.502 [3]).

It is invoked by the SMF in the following procedures:

-	Addition of PDU Session Anchor and Branching Point or UL CL controlled by I-SMF (see clause 4.23.9.1 of 3GPP TS 23.502 [3]);

-	Removal of PDU Session Anchor and Branching Point or UL CL controlled by I-SMF (see clause 4.23.9.2 of 3GPP TS 23.502 [3]);

-	Change of PDU Session Anchor for IPv6 multi-homing or UL CL controlled by I-SMF (see clause 4.23.9.3 of 3GPP TS 23.502 [3]);

-	Policy update procedures with an I-SMF (see clause 4.23.6.2 of 3GPP TS 23.502 [3]).

##### 5.2.2.8.2	Update service operation towards H-SMF or SMF

###### 5.2.2.8.2.1	General

The NF Service Consumer (i.e. the V-SMF for a HR PDU session, or the I-SMF for a PDU session with an I-SMF) shall update a PDU session in the H-SMF or SMF and/or provide the H-SMF or SMF with information received by the  NF Service Consumer in N1 SM signalling from the UE, by using the HTTP POST method (modify custom operation) as shown in Figure 5.2.2.8.2-1.

Figure 5.2.2.8.2-1: PDU session update towards H-SMF or SMF

1.	The NF Service Consumer shall send a POST request to the resource representing the individual PDU session resource in the H-SMF or SMF. The payload body of the POST request shall contain:

-	the requestIndication IE indicating the request type. Unless specified otherwise in clause 5.2.2.8.2, the value of the requestIndication IE shall be set to NW_REQ_PDU_SES_MOD;

-	the modification instructions and/or the information received by the NF Service Consumer in N1 signalling from the UE.

2a.	On success, "204 No Content" or "200 OK" shall be returned; in the latter case, the payload body of the POST response shall contain the representation describing the status of the request and/or information necessary for the NF Service Consumer to send N1 SM signalling to the UE.

2b.	On failure or redirection, one of the HTTP status code listed in Table 6.1.3.3.3.2-3 shall be returned. For a 4xx/5xx response, the message body shall contain an HsmfUpdateError structure, including:

-	a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.3.3.3.2-3;

-	the n1SmCause IE with the 5GSM cause the H-SMF or SMF proposes the NF Service Consumer to return to the UE, if the request included n1SmInfoFromUe;

-	n1SmInfoToUe binary data, if the H-SMF or SMF needs to return NAS SM information which the NF Service Consumer does not need to interpret;

-	the procedure transaction id that was received in the request, if this is a response sent to a UE requested PDU session modification.

###### 5.2.2.8.2.2	UE or network (e.g. AMF, V-SMF, I-SMF) requested PDU session modification

The requirements specified in clause 5.2.2.8.2.1 shall apply with the following modifications.

1.	Same as step 1 of Figure 5.2.2.8.2-1, with the following modifications.

The POST request shall contain:

-	the requestIndication set to UE_REQ_PDU_SES_MOD, and the modifications requested by the UE, e.g. UE requested QoS rules or UE requested Qos flow descriptions, in an N1 SM container IE as specified in clause 5.2.3.1, or indication that the PDU session is allowed to be upgraded to a MA PDU session as specified in clause 6.4.2.2 of 3GPP TS 24.501 [7], for a UE requested PDU session modification; or

-	the requestIndication set to NW_REQ_PDU_SES_MOD, and the modifications requested by the visited network or the notifications initiated by the visited network, for a visited network requested PDU session modification, e.g. to:

-	report the release of QoS flow(s) or notify QoS flow(s) whose targets QoS are no longer fulfilled; in the latter case, the V-SMF/I-SMF may also report an alternative QoS profile which the NG-RAN can currently fulfil in the currentQosProfileIndex IE or report that the NG-RAN cannot even fulfil the lowest alternative QoS profile by setting the nullQoSProfileIndex IE to "true" for the corresponding Qos flow(s);

-	report that the user plane security enforcement with a value Preferred is not fulfilled or is fulfilled again, in the NotifyList IE and the securityResult IE, if the new security status is received from NG-RAN;

-	report that access type of the PDU session can be changed; in this case, the anTypeCanBeChanged attribute shall be set to "true";

-	report the "MO Exception Data Counter";

-	request for QoS modification initiated by VPLMN, if the H-SMF supports the VPLMN QoS (VQOS) feature.

###### 5.2.2.8.2.3	UE requested PDU session release

The requirements specified in clause 5.2.2.8.2.1 shall apply with the following modifications.

1.	Same as step 1 of Figure 5.2.2.8.2-1, with the following modifications.

The POST request shall contain:

-	the requestIndication set to UE_REQ_PDU_SES_REL.

###### 5.2.2.8.2.4	EPS to 5GS Handover Execution

The requirements specified in clause 5.2.2.8.2.1 shall apply with the following modifications.

1.	Same as step 1 of Figure 5.2.2.8.2-1, with the following modifications.

The POST request shall contain:

-	the requestIndication set to PDU_SES_MOB;

-	the list of EPS Bearer Ids successfully handed over to 5GS;

-	the hoPreparationIndication IE set to "false", to indicate that there is no handover preparation in progress anymore and that the PGW-C/SMF shall switch the DL user plane of the PDU session;

-	one or more SecondaryRatUsageDataReportContainer(s) if it received the same in the SmContextUpdateData from the AMF.

2.	Same as step 2 of Figure 5.2.2.8.2-1, with the following modifications.

The H-SMF or SMF shall return a 200 OK response. The H-SMF or SMF shall switch the DL user plane of the PDU session using the N9 tunnel information that has been received in the vcnTunnelInfo or icnTunnelInfo, if the hoPreparationIndication IE was set to "false" in the request.

If the handover preparation failed (e.g. the target 5G-AN failed to establish resources for the PDU session), the requirements specified in clause 5.2.2.8.2.1 shall apply with the following modifications.

1.	Same as step 1 of Figure 5.2.2.8.2-1, with the following modifications.

The POST request shall contain:

-	the requestIndication set to PDU_SES_MOB;-	the cause attribute set to "HO_FAILURE";

-	an empty list of EPS Bearer Ids;

-	the hoPreparationIndication IE set to "false", to indicate that there is no handover preparation in progress anymore.

2.	Same as step 2 of Figure 5.2.2.8.2-1, with the following modifications.

The H-SMF or SMF shall return a 200 OK response. The H-SMF or SMF shall release the resources prepared for the handover.

###### 5.2.2.8.2.5	Handover between 3GPP access and untrusted or trusted non-3GPP access

For Handover between 3GPP access and untrusted or trusted non-3GPP access procedures, without AMF change or with the target AMF in the same PLMN, the requirements specified in clause 5.2.2.8.2.1 shall apply with the following modifications.

1.	Same as step 1 of Figure 5.2.2.8.2-1, with the following modifications.

The POST request shall contain the anType set to the target access type, i.e. to 3GPP_ACCESS or NON_3GPP_ACCESS.

The requestIndication IE shall be set to PDU_SES_MOB.

For a handover from non-3GPP access to 3GPP access with a V-SMF change, the requirements specified in step 1 of clause 5.2.2.8.2.10, other than how to set the requestIndication, shall also apply.

2a.	Same as step 2a of Figure 5.2.2.8.2-1, with the following modifications.

The payload body of the POST response shall include:

-	all QoS information for the QoS Flow(s) applicable to the PDU Session for the target access type, so that when sending the PDU Session Establishment Accept, the V-SMF or I-SMF can include all QoS information (e.g. QoS Rule(s) in N1 SM container, QFI(s) and QoS Profile(s) in N2 SM information) for the QoS Flow(s) (acceptable according to VPLMN policies for a HR PDU session); and

-	the epsPdnCnxInfo IE and, for each EPS bearer, an epsBearerInfo IE, if the PDU session may be moved to EPS during its lifetime, for a handover from non-3GPP access to 3GPP access.

The payload body of the POST response may also contain the upSecurity, maxIntegrityProtectedDataRateUl and maxIntegrityProtectedDataRateDl IEs during a handover from non-3GPP access to 3GPP access.

For a handover from non-3GPP access to 3GPP access with a V-SMF change, the requirements specified in step 2 of clause 5.2.2.8.2.10 shall also apply.

Upon receipt of the 200 OK response, the V-SMF or I-SMF shall delete any above information received earlier for the source access type and use the new information received for the target access type (see clause 6.1.6.2.12).

NOTE:	As specified in clause 4.11.1.4.3 of 3GPP TS 23.502 [3], the AMF, the SMF and the UE release locally the EBI(s) allocated to a PDU Session handed over from 3GPP access to non-3GPP access.

For a handover from non-3GPP access to 3GPP access, if the PDU session may be moved to EPS during its lifetime, the H-SMF or SMF may send an Update Request towards the V-SMF or I-SMF to request the allocation of EBIs prior to step 2a.

If one or more requested QoS flow(s) fail to be established in the target access type, the V-SMF or I-SMF shall send an Update Request including the qosFlowsRelNotifyList attribute to report the failure to the H-SMF or SMF (see clause 5.2.2.8.2.2), or a Release Request to release the PDU session if no QoS flow can be established (see clause 5.2.2.9).

###### 5.2.2.8.2.6	P-CSCF Restoration Procedure via AMF

The requirements specified in clause 5.2.2.8.2.1 shall apply with the following modifications.

1.	Same as step 1 of Figure 5.2.2.8.2-1, with the following modifications:

The POST request shall contain:

-	the requestIndication IE set to NW_REQ_PDU_SES_REL;

-	the cause IE set to REL_DUE_TO_REACTIVATION.

###### 5.2.2.8.2.7	Addition of PSA and BP or UL CL controlled by I-SMF

This clause applies only in case of non-roaming or LBO roaming as control of an UL CL or BP in VPLMN is not supported in HR roaming case (see clause 5.34.4 of 3GPP TS 23.501 [2]).

An I-SMF and I-UPF have already been inserted for the PDU session.

The requirements specified in clause 5.2.2.8.2.1 shall apply with the following modifications.

1.	Same as step 1 of Figure 5.2.2.8.2-1, with the following modifications:

The POST request shall contain:

-	the requestIndication IE set to NW_REQ_PDU_SES_MOD;

-	the indication that an UL CL or BP has been inserted into the data path of the PDU session;

-	the list of DNAIs supported by the inserted PSA;

-	the new UE IPv6 prefix at the PSA, assigned by the I-SMF or by the UPF supporting the PSA, if IPv6 multi-homing applies to the PDU session;

-	the icnTunnelInfo with the N9 tunnel information of the UL CL or BP for the downlink traffic, if a UPF different from the earlier I-UPF is selected for the UL CL or BP.

2a.	Same as step 2a of Figure 5.2.2.8.2-1.

###### 5.2.2.8.2.8	Removal of PSA and BP or UL CL controlled by I-SMF

This clause applies only in case of non-roaming or LBO roaming as control of an UL CL or BP in VPLMN is not supported in HR roaming case (see clause 5.34.4 of 3GPP TS 23.501 [2]).

The requirements specified in clause 5.2.2.8.2.1 shall apply with the following modifications.

1.	Same as step 1 of Figure 5.2.2.8.2-1, with the following modifications:

The POST request shall contain:

-	the requestIndication IE set to NW_REQ_PDU_SES_MOD;

-	the indication that an UL CL or BP has been removed from the data path of the PDU session;

-	the removed UE IPv6 prefix at the PSA, if IPv6 multi-homing applies to the PDU session;

-	the list of DNAIs supported by the removed PSA;

-	the icnTunnelInfo with the N9 tunnel information of the I-UPF for the downlink traffic.

2a.	Same as step 2a of Figure 5.2.2.8.2-1.

###### 5.2.2.8.2.9	Change of PSA for IPv6 multi-homing or UL CL controlled by I-SMF

This clause applies only in case of non-roaming or LBO roaming as control of an UL CL or BP in VPLMN is not supported in HR roaming case (see clause 5.34.4 of 3GPP TS 23.501 [2]).

The requirements specified in clause 5.2.2.8.2.1 shall apply with the following modifications.

1.	Same as step 1 of Figure 5.2.2.8.2-1, with the following modifications:

The POST request shall contain:

-	the requestIndication IE set to NW_REQ_PDU_SES_MOD;

-	the indication that a PSA is removed and another PSA is inserted;

-	the list of DNAIs supported by the inserted PSA;

-	the new UE IPv6 prefix at the inserted PSA, assigned by the I-SMF or by the UPF supporting the PSA, if IPv6 multi-homing applies to the PDU session;

-	the removed UE IPv6 prefix at the removed PSA, if IPv6 multi-homing applies to the PDU session;

-	the list of DNAIs supported by the removed PSA.

2a.	Same as step 2a of Figure 5.2.2.8.2-1.

###### 5.2.2.8.2.10	PDU Session modification with I-SMF or V-SMF change

During PDU Session modification with I-SMF or V-SMF change, the NF Service Consumer (i.e. the new V-SMF for a HR PDU session, or the new I-SMF for a PDU session with an I-SMF) shall update the PDU session in the H-SMF or SMF and provide the information of the new I-SMF or V-SMF.

The requirements specified in clause 5.2.2.8.2.1 shall apply with the following modifications.

1.	Same as step 1 of Figure 5.2.2.8.2-1, with the following additions:

The POST request shall contain:

-	the requestIndication set to NW_REQ_PDU_SES_MOD or UE_REQ_PDU_SES_MOD for network requested or UE requested PDU session modification respectively;

-	the ismfPduSessionUri or vsmfPduSessionUri IE containing the callback URI ({vsmfPduSessionUri} or {ismfPduSessionUri}) representing the PDU session in the new I-SMF or new V-SMF. The H-SMF or SMF shall construct the callback URIs based on the received {vsmfPduSessionUri} or {ismfPduSessionUri} as defined in clause 6.1, e.g. the callback URI "{vsmfPduSessionUri}/modify" to modify a PDU session in the V-SMF;

-	the ismfId or vsmfId IE containing the identifier of the new I-SMF or new V-SMF;

-	optionally the iSmfServiceInstanceId or vSmfServiceInstanceId IE containing the serviceInstanceId of the new I-SMF or new V-SMF service instance serving the PDU session;

-	the supportedFeatures IE indicating the optional features the NF Service Consumer supports, if at least one optional feature defined in clause 6.1.8 is supported.

2.	Same as step 1 of Figure 5.2.2.8.2-1, the SMF shall replace the corresponding information for the old I-SMF or old V-SMF stored locally with the received information. In addition, the SMF shall include the supportedFeatures IE in the response, if the supportedFeatures IE was received in the request and at least one optional feature defined in clause 6.1.8 is supported by the updated PDU session resource.

###### 5.2.2.8.2.11	Sending by I-SMF of N4 notifications related with traffic usage reporting

This clause applies only in case of non-roaming or LBO roaming as control of an UL CL or BP in VPLMN is not supported in HR roaming case (see clause 5.34.4 of 3GPP TS 23.501 [2]).

The requirements specified in clause 5.2.2.8.2.1 shall apply with the following modifications.

1.	Same as step 1 of Figure 5.2.2.8.2-1, with the following modifications:

The POST request shall contain:

-	the requestIndication IE set to NW_REQ_PDU_SES_MOD;

-	N4 information related with traffic usage reporting (i.e. PFCP Session Report Request, see Annex D of 3GPP TS 29.244 [29]);

-	the DNAI related to the N4 information if the latter relates to a local PSA;

2a.	Same as step 2a of Figure 5.2.2.8.2-1, with the following modifications:

The payload body of the POST response shall contain:

-	N4 response information (i.e. PFCP Session Report Response);

-	the DNAI related to the N4 information if the latter relates to a local PSA.

###### 5.2.2.8.2.12	N2 Handover Execution with I-SMF Insertion

The requirements specified in clause 5.2.2.8.2.1 shall apply with the following modifications.

1.	Same as step 1 of Figure 5.2.2.8.2-1, with the following modifications.

The POST request shall contain:

-	the requestIndication set to NW_REQ_PDU_SES_MOD;

-	the hoPreparationIndication IE set to "false", to indicate that there is no handover preparation in progress anymore and that the SMF shall switch the DL user plane of the PDU session;

-	the qosFlowsRelNotifyList IE indicating the failed QoS flow(s), if one or more QoS flow(s) cannot be established at the target RAN.

2.	Same as step 2 of Figure 5.2.2.8.2-1, with the following modifications.

The SMF shall return a 200 OK response. The SMF shall switch the DL user plane of the PDU session using the N9 tunnel information that has been received in the icnTunnelInfo, if the hoPreparationIndication IE was set to "false" in the request.

If the handover preparation failed (e.g. the target 5G-AN failed to establish resources for the PDU session), the requirements specified in clause 5.2.2.8.2.1 shall apply with the following modifications.

1.	Same as step 1 of Figure 5.2.2.8.2-1, with the following modifications.

The POST request shall contain:

-	the requestIndication set to NW_REQ_PDU_SES_MOD;-	the cause attribute set to "HO_FAILURE";

-	the hoPreparationIndication IE set to "false", to indicate that there is no handover preparation in progress anymore.

2.	Same as step 2 of Figure 5.2.2.8.2-1, with the following modifications.

The SMF shall return a 200 OK response. The SMF shall release the resources prepared for the handover.

###### 5.2.2.8.2.13	N2 Handover Cancellation with I-SMF Insertion

For handover cancellation, the requirements specified in clause 5.2.2.8.2.1 shall apply with the following modifications.

1.	Same as step 1 of Figure 5.2.2.8.2-1, with the following modifications.

The POST request shall contain:

-	the requestIndication set to NW_REQ_PDU_SES_MOD;

-	the cause attribute set to "HO_CANCEL";

-	the hoPreparationIndication IE set to "false", to indicate that there is no handover preparation in progress anymore.

2.	Same as step 2 of Figure 5.2.2.8.2-1, with the following modifications.

The SMF shall return a 200 OK response. The SMF shall release the resources prepared for the handover.

###### 5.2.2.8.2.14	EPS to 5GS Handover Cancellation

If the handover cancellation, the requirements specified in clause 5.2.2.8.2.1 shall apply with the following modifications.

1.	Same as step 1 of Figure 5.2.2.8.2-1, with the following modifications.

The POST request shall contain:

-	the requestIndication set to PDU_SES_MOB;

-	the cause attribute set to "HO_CANCEL";

-	an empty list of EPS Bearer Ids;

-	the hoPreparationIndication IE set to "false", to indicate that there is no handover preparation in progress anymore.

2.	Same as step 2 of Figure 5.2.2.8.2-1, with the following modifications.

The H-SMF or SMF shall return a 200 OK response. The H-SMF or SMF shall release the resources prepared for the handover. The combined PGW-C+SMF shall not release the PDN connection that was attempted to be handed over.

###### 5.2.2.8.2.15	5G-AN requested PDU session resource release

This clause applies only in case of 5G-AN requested PDU session resource release by sending the NGAP PDU SESSION RESOURCE NOTIFY to the AMF case (see step 1d in clause 4.3.4.3 of 3GPP TS 23.502 [3]).

The requirements specified in clause 5.2.2.8.2.1 shall apply with the following modifications.

1.	Same as step 1 of Figure 5.2.2.8.2-1, with the following modifications.

The POST request shall contain:

-	the requestIndication set to REL_DUE_TO_5G_AN_REQUEST to indicate that the PDU session resource has been released by the 5G-AN.

After receving the request, the SMF may decide to keep the PDU Session (with user plane connection deactivated) or release the PDU Session. If the SMF decides to keep the PDU Session, it shall return "200 OK" not including n1SmInfoToUe. If the SMF decides to release the PDU Session, it shall return "200 OK" including n1SmInfoToUe binary data containing the Message Type "PDU Session Release Command" and possibly PCO and cause information.

###### 5.2.2.8.2.16	Xn Handover with or without I-SMF or V-SMF Change

The requirements specified in clause 5.2.2.8.2.1 shall apply with the following modifications.

1.	Same as step 1 of Figure 5.2.2.8.2-1, with the following modifications.

The POST request shall contain:

-	the upSecurityInfo IE, if received from the NG-RAN;

-	the qosFlowsRelNotifyList IE indicating the failed QoS flow(s), if one or more QoS flow(s) cannot be established at the target RAN.

For an Xn handover with an I-SMF or V-SMF change, the requirements specified in step 1 of clause 5.2.2.8.2.10, other than how to set the requestIndication, shall also apply.

2.	Same as step 2 of Figure 5.2.2.8.2-1, with the following modifications.

The SMF shall verify that the upSecurity IE included in the received upSecurityInfo IE is same as the security policy for integrity protection and encryption that the SMF has locally stored. If there is a mismatch, the SMF shall send its locally stored security policy for integrity protection and encryption in upSecurity IE to NG-RAN as specified in clause 6.6.1 of 3GPP TS 33.501 [17].

For an Xn handover with an I-SMF or V-SMF change, the requirements specified in step 2 of clause 5.2.2.8.2.10 shall also apply.

###### 5.2.2.8.2.17	EPS to 5GS Handover Failure

If the handover to 5GS failed, e.g. rejected by the target NG-RAN, the requirements specified in clause 5.2.2.8.2.1 shall apply with the following modifications.

1.	Same as step 1 of Figure 5.2.2.8.2-1, with the following modifications.

The POST request shall contain:

-	the requestIndication set to PDU_SES_MOB;

-	the hoState attribute set to "CANCELLED";

-	the cause attribute set to "HO_FAILURE";

-	an empty list of EPS Bearer Ids;

-	the hoPreparationIndication IE set to "false", to indicate that there is no handover preparation in progress anymore.

2.	Same as step 2 of Figure 5.2.2.8.2-1, with the following modifications.

The H-SMF or SMF shall return a 200 OK response. The H-SMF or SMF shall release the resources prepared for the handover. The combined PGW-C+SMF shall not release the PDN connection that was attempted to be handed over.

###### 5.2.2.8.2.18	EPS Bearer ID revocation

When the AMF decides to revoke some EBI(s) and the I-SMF or V-SMF receives the request from AMF, the requirements specified in clause 5.2.2.8.2.1 shall apply with the following modifications.

1.	Same as step 1 of Figure 5.2.2.8.2-1, with the following modifications.

The POST request shall contain:

The requestIndication shall be set to EBI_ASSIGNMENT_REQ.

The NF Service Consumer shall include the revokeEbiList IE to request the SMF to release some EBI(s). The SMF shall disassociate the EBI(s) with the QFI(s) with which they are associated.

###### 5.2.2.8.2.19	Network requested PDU session release

The requirements specified in clause 5.2.2.8.2.1 shall apply with the following modifications.

1.	Same as step 1 of Figure 5.2.2.8.2-1, with the following modifications.

The POST request shall contain:

-	the requestIndication set to NW_REQ_PDU_SES_REL.

###### 5.2.2.8.2.20	N2 Handover Execution with or without I-SMF or V-SMF Change

The requirements specified in clause 5.2.2.8.2.1 shall apply with the following modifications.

1.	Same as step 1 of Figure 5.2.2.8.2-1, with the following modifications.

The POST request shall contain:

-	the qosFlowsRelNotifyList IE indicating the failed QoS flow(s), if one or more QoS flow(s) cannot be established at the target RAN.

For an N2 handover with an I-SMF or V-SMF change, the requirements specified in step 1 of clause 5.2.2.8.2.10, other than how to set the requestIndication, shall also apply.

2.	Same as step 2 of Figure 5.2.2.8.2-1, with the following modifications.

For an N2 handover with an I-SMF or V-SMF change, the requirements specified in step 2 of clause 5.2.2.8.2.10 shall also apply.

##### 5.2.2.8.3	Update service operation towards V-SMF or I-SMF

###### 5.2.2.8.3.1	General

The NF Service Consumer (i.e. the H-SMF for a HR PDU session, or the SMF for a PDU session with an I-SMF) shall update a PDU session in the V-SMF or I-SMF and/or provide information necessary for the V-SMF or I-SMF to send N1 SM signalling to the UE, or request to allocate or revoke EPS Bearer ID(s) for the PDU session, by using the HTTP "modify" custom operation as shown in Figure 5.2.2.8.3.1-1.

Figure 5.2.2.8.3.1-1: PDU session update towards V-SMF or I-SMF

1.	The NF Service Consumer shall send a POST request to the resource representing the individual PDU session resource in the V-SMF or I-SMF. The payload body of the POST request shall contain:

-	the requestIndication IE indicating the request type, which is set to NW_REQ_PDU_SES_MOD;

-	the modification instructions and/or the information necessary for the V-SMF or I-SMF to send N1 SM signalling to the UE;

-	the hsmfPduSessionUri IE or smfPduSessionUri IE if the Update Request is sent to the V-SMF or I-SMF before the Create Response, and the H-SMF or SMF PDU session resource URI has not been previously provided to the V-SMF or I-SMF; in this case, the supportedFeatures IE shall also be included if at least one optional feature defined in clause 6.1.8 is supported.

2a.	On success, "204 No Content" or "200 OK" shall be returned; in the latter case, the payload body of the POST response shall contain the representation describing the status of the request and/or information received by the V-SMF or I-SMF in N1 signalling from the UE.

2b.	On failure or redirection, one of the HTTP status code listed in Table 6.1.3.7.4.2.2-1 shall be returned. For a 4xx/5xx response, the message body shall contain a VsmfUpdateError structure, including:

-	a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.3.7.4.2.2-1;

-	the n1SmCause IE with the 5GSM cause returned by the UE, if available;

-	n1SmInfoFromUe and/or unknownN1SmInfo binary data, if NAS SM information has been received from the UE that needs to be transferred to the H-SMF or SMF, or that the V-SMF or I-SMF does not comprehend;

-	the procedure transaction id received from the UE, if available.

###### 5.2.2.8.3.2	Network (e.g. H-SMF, SMF) requested PDU session modification

The requirements specified in clause 5.2.2.8.3.1 shall apply with the following modifications.

1.	Same as step 1 of Figure 5.2.2.8.3.1-1, with the following modifications.

The requestIndication shall be set to NW_REQ_PDU_SES_MOD.

As part of the modification instructions, the NF Service Consumer may request to modify QoS parameters applicable at the PDU session level (e.g. modify the authorized Session AMBR values) or at the QoS flow level (e.g. modify the MFBR of a particular QoS flow). 

The NF Service Consumer may request to establish, modify and/or release QoS flows by including the qosFlowsAddModifyRequestList IE and/or the qosFlowsReleaseRequestList IE in the payload body.

The H-SMF or SMF may provide alternative QoS profiles for each GBR QoS flow with Notification control enabled, to allow the NG-RAN to accept the setup of the QoS flow if the requested QoS parameters or at least one of the alternative QoS parameters sets can be fulfilled at the time of setup. If the H-SMF or SMF provides a new list of alternative QoS profile(s) for a given GBR Qos flow, the V-SMF or I-SMF shall replace any previously stored list for this Qos flow with it.

The NF Service Consumer may include epsBearerInfo IE(s), if the PDU session may be moved to EPS during its lifetime and the EPS Bearer(s) information has changed (e.g. a new EBI has been assigned or the mapped EPS bearer QoS for an existing EBI has changed).

The NF Service Consumer may include the modifiedEbiList IE if the PDU session modification procedure resulted in the change of ARP for a QoS flow that has already been allocated an EBI.

The NF Service Consumer may include the revokeEbiList IE to request the V-SMF or I-SMF to release some EBI(s) and delete any corresponding EPS bearer context stored in the V-SMF or I-SMF. The V-SMF or I-SMF shall disassociate the EBI(s) with the QFI(s) with which they are associated.

2.	Same as step 2 of Figure 5.2.2.8.3.1-1, with the following modifications.

The V-SMF or I-SMF may accept all or only a subset of the QoS flows requested to be created or modified within the request.

The list of QoS flows which have been successfully setup or modified, and those which failed to be so, if any, shall be included in the qosFlowsAddModList IE and/or the qosFlowsFailedtoAddModList IE respectively.

The V-SMF or I-SMF may report an alternative QoS profile which the NG-RAN currently fulfils in the currentQosProfileIndex IE of the corresponding Qos flow in the qosFlowsAddModList IE, or report that the NG-RAN cannot even fulfil the lowest alternative QoS profile by setting the nullQoSProfileIndex IE to "true" for the corresponding Qos flow in the qosFlowsAddModList IE.

If the NG-RAN rejects the establishment of a voice QoS flow due to EPS Fallback for IMS voice (see clause 4.13 of 3GPP TS 23.502 [3]), the V-SMF or I-SMF shall return the cause indicating that "mobility due to EPS fallback for IMS voice is on-going" for the corresponding flow in the qosFlowsFailedtoAddModifyList IE.

The list of QoS flows which have been successfully released, and those which failed to be so, if any, shall be included in the qosFlowsReleaseList and/or qosFlowsFailedtoReleaseList IE respectively.

For a QoS flow which failed to be modified, the V-SMF or I-SMF shall fall back to the configuration of the QoS flow as it was configured prior to the reception of the PDU session update request from the NF Service Consumer.

The V-SMF or I-SMF shall store any EPS bearer information received from the H-SMF or SMF. If the revokeEbiList IE is present in the request, the V-SMF or I-SMF shall request delete the corresponding EPS bearer contexts and request the AMF to release the EBIs listed in this IE. If the modifiedEbiList IE is present in the request, the V-SMF or I-SMF shall request the AMF to update the mapping of EBI and ARP.

If the request received from the H-SMF or SMF contains the alwaysOnGranted attribute set to true, the V-SMF or I-SMF shall check and determine whether the PDU session can be established as an always-on PDU session based on local policy.

###### 5.2.2.8.3.3	Network (e.g. H-SMF, SMF) requested PDU session release

The requirements specified in clause 5.2.2.8.3.1 shall apply with the following modifications.

1.	Same as step 1 of Figure 5.2.2.8.3.1-1, with the following modifications.

The requestIndication shall be set to NW_REQ_PDU_SES_REL.

2.	Same as step 2 of Figure 5.2.2.8.3.1-1, with the following modifications.

If the requestIndication in the request is set to NW_REQ_PDU_SES_REL, the V-SMF or I-SMF shall initiate the release of RAN resources allocated for the PDU session if any and shall send a PDU session release command to the UE.

The V-SMF or I-SMF shall not release the SM context for the PDU session.

NOTE:	The SM context will be released when receiving Status notification from the H-SMF or SMF indicating the PDU session is released in the H-SMF or SMF.

###### 5.2.2.8.3.4	Handover between 3GPP and untrusted non-3GPP access, from 5GC-N3IWF to EPS or from 5GS to EPC/ePDG

The requirements specified in clause 5.2.2.8.3.1 shall apply with the following modifications.

1.	Same as step 1 of Figure 5.2.2.8.3.1-1, with the following modifications.

The NF Service Consumer shall request the source V-SMF or I-SMF to release the resources in the VPLMN without sending a PDU session release command to the UE, by setting the requestIndication IE to NW_REQ_PDU_SES_REL and the Cause IE indicating "Release due to Handover", in the following scenarios:

-	Handover of a PDU session between 3GPP and untrusted non-3GPP access, when the UE is roaming and the selected N3IWF is in the HPLMN (see clause 4.9.2.4.2 of 3GPP TS 23.502 [3]);

-	Handover from 5GC-N3IWF to EPS (see clause 4.11.3.2 of 3GPP TS 23.502 [3]);

-	Handover from 5GS to EPC/ePDG (see clause 4.11.4.2 of 3GPP TS 23.502 [3]).

2.	Same as step 2 of Figure 5.2.2.8.3.1-1, with the following modifications.

If the requestIndication in the request is set to NW_REQ_PDU_SES_REL and if the Cause IE indicates "Release due to Handover", the V-SMF or I-SMF shall initiate the release of RAN resources reserved for the PDU session if any but shall not send a PDU session release command to the UE.

The V-SMF or I-SMF shall not release the SM context for the PDU session.

NOTE:	The SM context will be released when receiving Status notification from the H-SMF or SMF indicating the PDU session is released in the H-SMF or SMF.

###### 5.2.2.8.3.5	EPS Bearer ID assignment

The requirements specified in clause 5.2.2.8.3.1 shall apply with the following modifications.

1.	Same as step 1 of Figure 5.2.2.8.3.1-1, with the following modifications.

The requestIndication shall be set to EBI_ASSIGNMENT_REQ.

The NF Service Consumer may include the assignEbiList IE to request the allocation of EBI(s). The NF Service Consumer may include the revokeEbiList IE to request the V-SMF or I-SMF to release some EBI(s) and delete any corresponding EPS bearer context stored in the V-SMF or I-SMF. The V-SMF or I-SMF shall disassociate the EBI(s) with the QFI(s) with which they are associated.

NOTE:	The SMF does not request EBI allocation when MA PDU Session is established only over non-3GPP access. For MA PDU Session with both 3GPP and non-3GPP accesses, the SMF does not allocate EBI(s) for GBR QoS Flow(s) that are only allowed over non-3GPP access.

Upon receipt of this request, the V-SMF or I-SMF shall request the AMF to assign (and release if required) EBIs (see clause 5.2.2.6 of 3GPP TS 29.518 [20].

2a.	Same as step 2a of Figure 5.2.2.8.3.1-1, with the following modifications.

If the AMF has successfully assigned all or part of the requested EBIs, the V-SMF or I-SMF shall respond with the status code 200 OK, and include the list of EBIs successfully allocated, those which failed to be so if any, and the list of EBIs released for this PDU session at AMF if any, in the assignedEbiList IE, the failedtoAssignEbiList IE and/or the releasedEbiList IE respectively.

2b.	Same as step 2b of Figure 5.2.2.8.3.1-1, with the following modifications.

For a 4xx/5xx response, the message body shall contain a VsmfUpdateError structure, including the list of EBIs which failed to be allocated in the failedtoAssignEbiList IE.

###### 5.2.2.8.3.6	Addition of PSA and BP or UL CL controlled by I-SMF

The requirements specified in clause 5.2.2.8.3.1 shall apply with the following modifications.

1.	Same as step 1 of Figure 5.2.2.8.3.1-1, with the following modifications.

The requestIndication shall be set to NW_REQ_PDU_SES_MOD.

The payload body of the POST response shall contain:

-	N4 information for the handling of the local traffic that is offloaded at the PSA (see Annex D of 3GPP TS 29.244 [29]);

-	N4 information for the local offload rules at the UL CL/BP (see Annex D of 3GPP TS 29.244 [29]);

-	the DNAI related to N4 information targeting the local PSA;

-	the indication that the DNAI shall not change, if applicable;

-	the indication that the local PSA shall not change, if applicable.

2a.	Same as step 2a of Figure 5.2.2.8.3.1-1, with the following modifications.

The payload body of the POST response shall contain:

-	N4 response information;

-	the DNAI related to the N4 information if the latter relates to a local PSA.

2b.	Same as step 2b of Figure 5.2.2.8.3.1-1, with the following modifications.

For a 4xx/5xx response, the message body shall contain a VsmfUpdateError structure, including N4 response information if available (e.g. PFCP Session Establishment Response with a rejection cause).

###### 5.2.2.8.3.7	Removal of PSA and BP or UL CL controlled by I-SMF

The requirements specified in clause 5.2.2.8.3.1 shall apply with the following modifications.

1.	Same as step 1 of Figure 5.2.2.8.3.1-1, with the following modifications.

The requestIndication shall be set to NW_REQ_PDU_SES_MOD.

The payload body of the POST response shall contain:

-	N4 information for the removal of the local offload rules at the UL CL/BP and PSA (see Annex D of 3GPP TS 29.244 [29]);

-	the DNAI related to N4 information targeting the local PSA.

2a.	Same as step 2a of Figure 5.2.2.8.3.1-1, with the following modifications.

The payload body of the POST response shall contain:

-	N4 response information;

-	the DNAI related to the N4 information if the latter relates to a local PSA.

2b.	Same as step 2b of Figure 5.2.2.8.3.1-1, with the following modifications.

For a 4xx/5xx response, the message body shall contain a VsmfUpdateError structure, including N4 response information if available (e.g. PFCP Session Deletion Response with a rejection cause).

###### 5.2.2.8.3.8	Change of PSA for IPv6 multi-homing or UL CL controlled by I-SMF

The requirements specified in clause 5.2.2.8.3.1 shall apply with the following modifications.

1.	Same as step 1 of Figure 5.2.2.8.3.1-1, with the following modifications.

The requestIndication shall be set to NW_REQ_PDU_SES_MOD.

The payload body of the POST response shall contain:

-	N4 information for the removal of the local offload rules at the removed PSA (see Annex D of 3GPP TS 29.244 [29]);

-	N4 information for the handling of the local traffic that is offloaded at the inserted PSA (see Annex D of 3GPP TS 29.244 [29]);

-	N4 information for the updating of the local offload rules at the UL CL/BP (see Annex D of 3GPP TS 29.244 [29]);

-	the DNAIs related to N4 information targeting the removed or inserted PSA.

2a.	Same as step 2a of Figure 5.2.2.8.3.1-1, with the following modifications.

The payload body of the POST response shall contain:

-	N4 response information;

-	the DNAI related to the N4 information if the latter relates to a local PSA.

2b.	Same as step 2b of Figure 5.2.2.8.3.1-1, with the following modifications.

For a 4xx/5xx response, the message body shall contain a VsmfUpdateError structure, including N4 response information if available (e.g. PFCP Session Establishment Response with a rejection cause).

###### 5.2.2.8.3.9	Policy update procedures with an I-SMF

The requirements specified in clause 5.2.2.8.3.1 shall apply with the following modifications.

1.	Same as step 1 of Figure 5.2.2.8.3.1-1, with the following modifications.

The requestIndication shall be set to NW_REQ_PDU_SES_MOD.

The payload body of the POST response may contain:

-	N4 information updating local offload rules at the I-SMF (see Annex D of 3GPP TS 29.244 [29]);

-	the DNAI related to the N4 information if the latter relates to a local PSA;

-	an updated list of DNAI(s) of interest for the PDU Session.

2a.	Same as step 2a of Figure 5.2.2.8.3.1-1, with the following modifications.

The payload body of the POST response shall contain:

-	N4 response information, if N4 information was received in the request;

-	the DNAI related to the N4 information if the latter relates to a local PSA.

2b.	Same as step 2b of Figure 5.2.2.8.3.1-1, with the following modifications.

For a 4xx/5xx response, the message body shall contain a VsmfUpdateError structure, including N4 response information if available (e.g. PFCP Session Modification Response with a rejection cause).

###### 5.2.2.8.3.10	Simultaneous change of PSA and BP or UL CL controlled by I-SMF

The requirements specified in clause 5.2.2.8.3.1 shall apply with the following modifications.

1.	Same as step 1 of Figure 5.2.2.8.3.1-1, with the following modifications.

The requestIndication shall be set to NW_REQ_PDU_SES_MOD.

The payload body of the POST response shall contain:

-	N4 information for the removal of the local offload rules at the removed PSA (see Annex D of 3GPP TS 29.244 [29]);

-	N4 information for the handling of the local traffic that is offloaded at the inserted PSA (see Annex D of 3GPP TS 29.244 [29]);

-	N4 information for the removal of the local offload rules at the removed UL CL/BP (see Annex D of 3GPP TS 29.244 [29]);

-	N4 information for the creation of the local offload rules at the inserted UL CL/BP (see Annex D of 3GPP TS 29.244 [29]);

-	the DNAIs related to N4 information targeting the removed or inserted PSA.

2a.	Same as step 2a of Figure 5.2.2.8.3.1-1, with the following modifications.

The payload body of the POST response shall contain:

-	N4 response information;

-	the DNAI related to the N4 information if the latter relates to a local PSA.

2b.	Same as step 2b of Figure 5.2.2.8.3.1-1, with the following modifications.

For a 4xx/5xx response, the message body shall contain a VsmfUpdateError structure, including N4 response information if available (e.g. PFCP Session Establishment Response with a rejection cause).

#### 5.2.2.9	Release service operation

##### 5.2.2.9.1	General

The Release service operation shall be used to request an immediate and unconditional deletion of an invidual PDU session resource in the SMF (i.e. in the H-SMF for a HR PDU session, or in the SMF for a PDU session with an I-SMF).

It is invoked by the NF Service Consumer (i.e. V-SMF or I-SMF) in the following procedures:

-	UE initiated Deregistration (see clause 4.2.2.3.2 of 3GPP TS 23.502 [3]);

-	Network initiated Deregistration (see clause 4.2.2.3.2 of 3GPP TS 23.502 [3]), e.g. AMF initiated deregistration;

-	visited network requested PDU Session release (see clause 4.3.4.3 of 3GPP TS 23.502 [3]), e.g. AMF initiated release in the following cases:

-	when there is a mismatch of the PDU session status between the UE and the AMF; or

-	when a network slice is no longer available.

-	5GS to EPS handover using N26 interface and 5GS to EPS Idle mode mobility using N26, to release the PDU session not transferred to EPC (see clauses 4.11.1.2.1 and 4.11.1.3.2 of 3GPP TS 23.502 [3]);

-	PDU session release procedure, for a PDU session with an I-SMF (see clause 4.23.5.2 of 3GPP TS 23.502 [3]);

-	5G-SRVCC from NG-RAN to 3GPP UTRAN procedure (see clause 6.5.4 of 3GPP TS 23.216 [35]).

The SMF shall release the PDU session context without triggering any signalling towards the 5G-AN and the UE.

The NF Service Consumer shall release a PDU session in the SMF by using the HTTP "release" custom operation as shown in Figure 5.2.2.9.1-1.

Figure 5.2.2.9.1-1: Pdu session release

1.	The NF Service Consumer shall send a POST request to the resource representing the individual PDU session resource in the SMF. The payload body of the POST request shall contain any data that needs to be passed to the SMF.

If an UL CL/BP was inserted in the data path of the PDU session and traffic usage measurements need to be reported to the SMF, the POST request shall contain:

-	N4 information related with traffic usage reporting (i.e. PFCP Session Report Request, see Annex D of 3GPP TS 29.244 [29]);

-	the DNAI related to the N4 information if the latter relates to a local PSA.

2a.	On success, the SMF shall return a "200 OK" with message body containing the representation of the ReleasedData when information needs to be returned to the NF Service Consumer, or a "204 No Content" response with an empty payload body in the POST response.

If N4 information was received in the request, the POST response shall contain:

-	N4 response information (i.e. PFCP Session Report Response);

-	the DNAI related to the N4 information if the latter relates to a local PSA.

If the request body contains the "cause" attribute with the value "REL_DUE_TO_PS_TO_CS_HO", the (H-) SMF shall indicate to the PCF within SM Policy Association termination that the PDU session is released due to 5G-SRVCC.

2b.	On failure or redirection, one of the HTTP status code listed in Table 6.1.3.6.4.3.2-2 shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in Table 6.1.3.6.4.3.2-2.

#### 5.2.2.10	Notify Status service operation

##### 5.2.2.10.1	General

The Notify Status service operation shall be used to notify the NF Service Consumer about status changes of a PDU session (e.g. when the PDU session is released and the release is not triggered by a Release Request, or when the PDU session is moved to another system, or when the control of the PDU session is taken over by another anchor SMF), for a HR PDU session or a PDU session involving an I-SMF.

It is used in the following procedures:

-	Home network requested PDU Session release (see clause 4.3.4.3 of 3GPP TS 23.502 [3]), e.g. H-SMF initiated release;

-	SMF requested PDU session release, for a PDU session involving an I-SMF (see clause 4.23 of 3GPP TS 23.502 [3]);

-	Handover of a PDU Session procedure from 3GPP to untrusted non-3GPP access (see clause 4.9.2.4.2 of 3GPP TS 23.502 [3]);

-	Interworking procedures without N26 interface, e.g. 5GS to EPS Mobility (see clause 4.11.2.2 of 3GPP TS 23.502 [3]);

-	Handover from 5GC-N3IWF to EPS (see clause 4.11.3.2 of 3GPP TS 23.502 [3]);

-	Handover from 5GS to EPC/ePDG (see clause 4.11.4.2 of 3GPP TS 23.502 [3]);

-	The control of PDU session is taken over by a new anchor SMF within the same SMF set (see clause 5.22 of 3GPP TS 29.244 [29]), and the new SMF instance decides to send an updated binding indication to the NF service consumer (as per step 6 of clause 6.5.3.3 of 3GPP TS 29.500 [4]);

-	I-SMF selection per DNAI (see clause 4.23.5.4 of 3GPP TS 23.502 [3]);

-	Change of SSC mode 2 PDU Session Anchor with different PDU Sessions (see clause 4.3.5.1 of 3GPP TS 23.502 [3]);

-	Change of SSC mode 3 PDU Session Anchor with multiple PDU Sessions (see clause 4.3.5.2 of 3GPP TS 23.502 [3]).

The SMF (i.e. H-SMF for a HR PDU session, or SMF for a PDU session involving an I-SMF) shall notify the NF Service Consumer (i.e. V-SMF for a HR PDU session, or I-SMF for a PDU session involving an I-SMF) by using the HTTP POST method as shown in Figure 5.2.2.10-1.

Figure 5.2.2.10-1: PDU session status notification

1.	The SMF shall send a POST request to the resource representing the individual PDU session resource in the NF Service Consumer. The payload body of the POST request shall contain the notification payload, with the status information.

If the notification is triggered by PDU session handover to release resources of the PDU Session in the source access, the notification payload shall contain the resourceStatus IE with the value "RELEASED" and the Cause IE with value "PDU_SESSION_HANDED_OVER" as specified in clause 4.2.9.4.2 of 3GPP TS 23.501 [2].

If the notification is triggered by SMF for I-SMF selection for the current PDU session, or SMF selection during PDU Session re-establishment for SSC mode 2/3, the notification payload shall contain the resourceStatus IE with the value "UNCHANGED", the Cause IE with the value "TARGET_DNAI_NOTIFICATION" and the targetDnaiInfo IE.

If the notification is triggered by PDU session handover to release resources of the PDU Session in the target access due to handover failure between 3GPP access and non-3GPP access, the notification payload shall contain the resourceStatus IE with the value "RELEASED" and the Cause IE with the value "PDU_SESSION_HAND_OVER_FAILURE".

If the NF Service Consumer indicated support of the HOFAIL feature (see clause 6.1.8) and if the notification is triggered by PDU session handover to update access type of the PDU Session due to handover failure between 3GPP access and non-3GPP access, the notification payload shall contain the resourceStatus IE with the value "UPDATED", the anType IE with the value "3GPP" or "NON_3GPP" indicating the access type of the PDU session after the handover failure scenario and the Cause IE with the value "PDU_SESSION_HAND_OVER_FAILURE".

If upon a change of anchor SMF, the new anchor SMF instance decides to send an updated binding indication to the NF service consumer (as per step 6 of clause 6.5.3.3 of 3GPP TS 29.500 [4]), then it shall contain the resourceStatus IE with the value "UPDATED" and the Cause IE with the value "CHANGED_ANCHOR_SMF" in the notification payload, and carry an updated binding indication in the HTTP headers to indicate the change of anchor SMF.

2a.	On success, "204 No Content" shall be returned and the payload body of the POST response shall be empty.

If the SMF indicated in the request that the PDU session in the SMF is released, the NF Service Consumer shall release the SM context for the PDU session.

If the SMF indicated in the request that the SM context resource is updated with the anType IE, the NF Service Consumer shall change the access type of the PDU session with the value of anType IE.

2b.	On failure or redirection, one of the HTTP status code listed in Table 6.1.3.7.3.1-2 shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in Table 6.1.3.7.3.1-2.

#### 5.2.2.11	Send MO Data service operation

##### 5.2.2.11.1	General

The Send MO Data service operation shall be used to send mobile originated data received over NAS, for a given PDU session, towards the SMF, or the V-SMF for HR roaming scenarios, or the I-SMF for a PDU session with an I-SMF.

It is used in the following procedures:

-	UPF anchored Mobile Originated Data Transport in Control Plane CIoT 5GS Optimisation (see clause 4.24.1 of 3GPP TS 23.502 [3]);

-	NEF anchored Mobile Originated Data Transport (see clause 4.25.4 of 3GPP TS 23.502 [3]).

The NF Service Consumer (e.g. AMF) shall send mobile originated data to the SMF by using the HTTP POST method (send-mo-data custom operation) as shown in Figure 5.2.2.11.1-1.

Figure 5.2.2.11.1-1: Send MO Data

1.	The NF Service Consumer shall send a POST request to the resource representing the individual SM context resource in the SMF. The payload body of the POST request shall contain the mobile originated data to send.

The request body may include the "MO Exception Data Counter", which indicates that the UE has accessed the network by using "MO exception data" RRC establishment, if Small Data Rate Control is enabled for the PDU session and the UE is accessing via the NB-IoT RAT.

2a.	On success, "204 No Content" shall be returned.

For UPF anchored Mobile Originated Data Transport in Control Plane CIoT 5GS Optimisation, if the "MO Exception Data Counter" is included in the request then:

-	for HR PDU session, the V-SMF shall update the H-SMF (see clause 5.2.2.8.2.2;

-	for PDU session with I-SMF, the I-SMF shall update the SMF (see clause 5.2.2.8.2.2.

2b.	On failure or redirection, one of the HTTP status code listed in Table 6.1.3.3.3.2-3 shall be returned.  For a 4xx/5xx response, the message body shall contain a ProblemDetails, with the "cause" attribute indicating the cause of the failure.

#### 5.2.2.12	Transfer MO Data service operation

##### 5.2.2.12.1	General

The Transfer MO Data service operation shall be used to transfer NEF anchored mobile originated data received from AMF, for a given PDU session, towards the H-SMF for HR roaming scenarios, or the SMF for a PDU session with an I-SMF.

It is used in the following procedures:

-	NEF anchored Mobile Originated Data Transport (see clause 4.25.4 of 3GPP TS 23.502 [3]).

The NF Service Consumer (e.g. V-SMF or I-SMF) shall transfer NEF anchored mobile originated data to the SMF by using the HTTP POST method (transfer-mo-data custom operation) as shown in Figure 5.2.2.12.1-1.

Figure 5.2.2.12.1-1: Transfer MO Data

1.	The NF Service Consumer shall send a POST request to the URI of Transfer MO Data custom operation on an individual PDU Session resource in the SMF. The payload body of the POST request shall contain the mobile originated data to be transferred.

The payload body shall also contain the MO Exception Data Counter, if received from AMF.

2a.	On success, "204 No Content" shall be returned.

2b.	On failure or redirection, one of the HTTP status code listed in Table 6.1.3.6.4.4.2-2 shall be returned.  For a 4xx/5xx response, the message body may contain a ProblemDetails, with the "cause" attribute indicating the cause of the failure.

#### 5.2.2.13	Transfer MT Data service operation

##### 5.2.2.13.1	General

The Transfer MT Data service operation shall be used to transfer NEF anchored mobile terminated data received from NEF, for a given PDU session, towards the V-SMF for HR roaming scenarios, or the I-SMF for a PDU session with an I-SMF.

It is used in the following procedures:

-	NEF anchored Mobile Terminated Data Transport (see clause 4.25.5 of 3GPP TS 23.502 [3]).

The NF Service Consumer (e.g. H-SMF or SMF) shall transfer NEF anchored mobile terminated data to the SMF by using the HTTP POST method (transfer-mt-data custom operation) as shown in Figure 5.2.2.13.1-1.

Figure 5.2.2.13.1-1: Transfer MT Data

1.	The NF Service Consumer shall send a POST request to the URI of Transfer MT Data custom operation on an individual PDU Session resource in the SMF. The payload body of the POST request shall contain the mobile terminated data to be transferred.

The SMF shall forward the mobile terminated data to AMF. If SMF determines Extended Buffering is allowed by local policy and the NEF supports Extended Buffering, the SMF indicate the Extending Buffering support to AMF.

If AMF responds that it is attempting to reach the UE, the SMF shall wait for potential failure notification from AMF before responding to the NF consumer.

2a.	On success, "204 No Content" shall be returned.

2b.	On failure or redirection, one of the HTTP status code listed in Table 6.1.3.7.4.3.2-2 shall be returned.  For a 4xx/5xx response, the message body may contain a TransferMtDataError or ProblemDetails object, with the "cause" attribute indicating the cause of the failure. If Estimated Maximum Waiting Time is received from AMF, the SMF shall include it in the message body.

#### 5.2.2.14	Retrieve service operation

##### 5.2.2.14.1	General

The Retrieve service operation shall be used to retrieve information from a PDU session context from the H-SMF for a HR PDU session, or from the SMF for a PDU session with an I-SMF.

It is used in the following procedures:

-	5GS to EPS handover using N26 interface and 5GS to EPS Idle mode mobility using N26 interface (see clauses 4.11.1.2.1 and 4.11.1.2.3 of 3GPP TS 23.502 [3]), for PDU sessions associated with 3GPP access and for which small data rate control is applicable.

The NF Service Consumer (e.g. V-SMF or I-SMF) shall retrieve information from a PDU session context by using the HTTP POST method (retrieve custom operation) as shown in Figure 5.2.2.14.1-1.

Figure 5.2.2.14.1-1: Retrieval of information from a PDU session context

1.	The NF Service Consumer shall send a POST request to the resource representing the individual PDU session context for which information needs to be retrieved. The POST request may contain a payload body with the following parameters:

-	smallDataRateStatusReq set to "true" to indicate a request to retrieve the small data rate control status of the PDU session, if small data rate control is applicable on the PDU session.

2a.	On success, "200 OK" shall be returned and the payload body of the POST response shall contain the small data rate control status if this is a request for retrieving the small data rate control status.

2b.	On failure or redirection, one of the HTTP status code listed in Table 6.1.3.6.4.5.2-2 shall be returned. For a 4xx/5xx response, the message body may contain a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.3.6.4.5.2-2.

### 5.2.3	General procedures

#### 5.2.3.1	Transfer of NAS SM information between UE and H-SMF for Home Routed PDU sessions

##### 5.2.3.1.1	General

As specified in clause 4.3.1 of 3GPP TS 23.502 [3], for Home Routed PDU sessions, there is NAS SM information that the V-SMF and H-SMF need to interpret, and NAS SM information that the V-SMF only needs to transfer between the UE and H-SMF but which it does not need to interpret.

NAS SM information that only needs to be transferred between the UE and H-SMF by the V-SMF can be extended in later versions or releases of the NAS specification, e.g. defining new fields or values within existing IEs, and the extensions should not impact the V-SMF.

Besides, in HR roaming scenarios, the V-SMF and H-SMF can comply to different versions or releases of the NAS specification. It should be possible to support new SM features only requiring support from the H-SMF without impacting the V-SMF, when the H-SMF complies with a more recent release than the V-SMF, e.g. defining new NAS SM IEs in signalling from the UE to the H-SMF and/or signalling from the H-SMF to the UE.

##### 5.2.3.1.2	V-SMF Behaviour

The V-SMF shall transfer NAS SM information that it only needs to transfer to the H-SMF (i.e. known IEs, and IEs that have an unknown value not set to "reserved" according to the release to which the V-SMF complies, that only need to be forwarded by the V-SMF) in n1SmInfoFromUe binary data within the HTTP payload. This carries N1 SM IE(s), encoded as specified in 3GPP TS 24.501 [7], including the Type field and, for TLV or TLV-E IEs, the Length field.

NOTE 1:	N1 SM IEs defined without a Type field need to be defined over N16/N16a as specific IEs.

The V-SMF shall transfer NAS SM information that it does not comprehend (i.e. unknown IEs, or known IEs to be interpreted by the V-SMF that have an unknown value not set to "reserved" according to the release to which the V-SMF complies) in unknownN1SmInfo binary data within the HTTP payload. This carries N1 SM IE(s), encoded as specified in 3GPP TS 24.501 [7], including the Type field and, for TLV or TLV-E IEs, the Length field.

When receiving n1SmInfoToUe binary data in the HTTP payload from the H-SMF, the V-SMF shall parse all the N1 SM IEs received in the binary data and construct the NAS SM message to the UE according to 3GPP TS 24.501 [7]. The V-SMF shall append unknown NAS SM IEs received in the binary data at the end of the NAS SM message it sends to the UE.

NOTE 2:	The V-SMF can infer the length of an unknown IE based on the IEI value. See clause 11.2.4 of 3GPP TS 24.007 [8].

The V-SMF shall comprehend and be able to encode at their right place in a given NAS message, all the IEs of the version of the NAS specification it implements that do not need to be interpreted by the V-SMF and which precede the last interpreted IE that the V-SMF implements in a NAS message.

NOTE 3:	The V-SMF encodes comprehended IEs at their right place in the NAS SM message

##### 5.2.3.1.3	H-SMF Behaviour

When receiving unknownN1SmInfo binary data in the HTTP payload from the V-SMF, the H-SMF shall process any N1 SM IE received in this binary data that do not require to be interpreted by the V-SMF. Other N1 SM IEs shall be dropped, e.g. IEs that the H-SMF comprehends but which require to be interpreted by the V-SMF.

The H-SMF shall transfer NAS SM information which the V-SMF does not need to interpret (i.e. that it only needs to transfer to the UE) in n1SmInfoToUe binary data within the HTTP payload. This carries N1 SM IE(s), encoded as specified in 3GPP TS 24.501 [7], including the Type field and, for TLV or TLV-E IEs, the Length field.

NOTE 1:	N1 SM IEs defined without a Type field need to be defined over N16/N16a as specific IEs.

The NAS SM IEs in n1SmInfoToUe binary data shall be encoded in the same order as specified in 3GPP TS 24.501 [7].

N1 SM information which does not require to be interpreted by the V-SMF is information that is not defined as specific IEs over N16.

#### 5.2.3.2	Transfer of NAS SM information between UE and SMF for PDU sessions with an I-SMF

##### 5.2.3.2.1	General

The requirements specified in clause 5.2.3.1 shall also apply for the transfer of NAS SM information between the UE and the SMF for PDU sessions with an I-SMF, whereby the I-SMF and SMF shall take the role of the V-SMF and H-SMF respectively.

#### 5.2.3.3	Detection and handling of late arriving requests

##### 5.2.3.3.1	Handling of requests which collide with an existing SM context or PDU session context

###### 5.2.3.3.1.1	General

This procedure enables an SMF, which receives a request colliding with an existing SM context or PDU session context, to know the time at which the new request and the existing PDU session were originated, and to accept the new request only if it is more recent than the existing PDU session.

The originating entities within the PLMN (i.e. AMFs) shall be NTP synchronized.

###### 5.2.3.3.1.2	Principles

The following principles shall apply if this procedure is supported and enabled by operator policy.

An AMF originating a Create SM Context request shall include in the message the Origination Time Stamp indicating the absolute time at which the request is initiated.

The V-SMF or I-SMF shall forward this header to the H-SMF or SMF, if it is received from the AMF.

Upon receipt of a Create SM Context request or a Create request which collides with an existing SM context or PDU session context, the SMF shall accept the new PDU session establishment request only if it contains a more recent time stamp than the time stamp stored for the existing PDU session. An incoming PDU session establishment request shall be considered as more recent than an existing PDU session and be accepted if no Origination Time Stamp information was provided for at least one of the two PDU sessions. The SMF shall reject an incoming request whose time stamp is less recent than the time stamp of the existing PDU session with the HTTP status code "403 Forbidden" and the application error "LATE_OVERLAPPING_REQUEST".

3GPP TS 29.512 [30] further specify:

-	the SMF requirements regarding the forwarding of the Origination Time Stamp towards the PCF, when received from the AMF;

-	the handling of the Origination Time Stamp parameter by the PCF for an incoming request colliding with an existing Individual SM Policy Association.

##### 5.2.3.3.2	Detection and handling of requests which have timed out at the HTTP client

###### 5.2.3.3.2.1	General

The procedure specified in clause 6.11.2 of 3GPP TS 29.500 [4] shall apply with the following additions.

An HTTP request may include the 3gpp-Sbi-Origination-Timestamp and the 3gpp-Sbi-Max-Rsp-Time headers, with or without the 3gpp-Sbi-Sender-Timestamp header.

The 3gpp-Sbi-Max-Rsp-Time header shall indicate the duration expressed in milliseconds since the absolute time indicated in the 3gpp-Sbi-Sender-Timestamp header, if this header is present, or indicated in the 3gpp-Sbi-Origination-Timestamp header otherwise.

NOTE:	The AMF does not need to include the 3gpp-Sbi-Sender-Timestamp if it includes the 3gpp-Sbi-Origination-Timestamp. A V-SMF or I-SMF forwards the 3gpp-Sbi-Origination-Timestamp over N16 or N16a, if received, and can include a 3gpp-Sbi-Sender-Timestamp header set to the time when it sends the Create request, in which case the 3gpp-Sbi-Max-Rsp-Time header contains the response time with respect to the 3gpp-Sbi-Sender-Timestamp header.

#### 5.2.3.4	UE Location Information

When attributes with the UserLocation data type (as defined in clause 5.4.4.7 of 3GPP TS 29.571 [13]) are included in the messages (as specified in clause 6) to report the UE location information to the SMF, the following information shall be included in these attributes:

-	the TAI and NCGI for NR user location; or

-	the TAI and ECGI for E-UTRA user location; or

-	the TAI, UE local IP address, Port if NAT is detected, and optionally n3Iwid if available, for untrusted non-3GPP access; or

-	the TAI and TNAP Id/TWAP Id for trusted non-3GPP access; or

-	the TAI and GLI and optionally LineType if available, or the TAI and hfcNodeId, or the TAI and GCI, for wireline network access.

# 6	API Definitions

## 6.1	Nsmf_PDUSession Service API

### 6.1.1	API URI

The Nsmf_PDUSession service shall use the Nsmf_PDUSession API.

The API URI of the Nsmf_PDUSession API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests from the NF service consumer towards the NF service producer shall have the Resource URI structure defined in clause 4.4.1 of 3GPP TS 29.501 [5], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [5].

-	The <apiName> shall be "nsmf-pdusession".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 6.1.3.

### 6.1.2	Usage of HTTP

#### 6.1.2.1	General

HTTP/2, as defined in IETF RFC 7540 [14], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [4].

HTTP messages and bodies for the Nsmf_PDUSession service shall comply with the OpenAPI [15] specification contained in Annex A.

#### 6.1.2.2	HTTP standard headers

##### 6.1.2.2.1	General

The usage of HTTP standard headers shall be supported as specified in clause 5.2.2 of 3GPP TS 29.500 [4].

##### 6.1.2.2.2	Content type

The following content types shall be supported:

-	the JSON format (IETF RFC 8259 [11]). The use of the JSON format shall be signalled by the content type "application/json". See also clause 5.4 of 3GPP TS 29.500 [4].

-	the Problem Details JSON Object (IETF RFC 7807 [23]). The use of the Problem Details JSON object in a HTTP response body shall be signalled by the content type "application/problem+json".

NOTE:	"application/json" is used in a response that includes a payload body containing an application-specific data structure, see clause 4.8 of 3GPP TS 29.501 [5].

Multipart messages shall also be supported (see clause 6.1.2.4) using the content type "multipart/related", comprising:

-	one JSON body part with the "application/json" content type; and

-	one or two binary body parts with 3gpp vendor specific content subtypes.

The 3gpp vendor specific content subtypes defined in Table 6.1.2.2.2-1 shall be supported.

Table 6.1.2.2.2-1: 3GPP vendor specific content subtypes

| content subtype | Description |
| --- | --- |
| vnd.3gpp.ngap | Binary encoded payload, encoding NG Application Protocol (NGAP) IEs, as specified in clause 9.3 of 3GPP TS 38.413 [9] (ASN.1 encoded). |
| vnd.3gpp.5gnas | Binary encoded payload, encoding a 5GS NAS message or 5G NAS IEs, as specified in 3GPP TS 24.501 [7]. |
| vnd.3gpp.pfcp | Binary encoded payload, encoding a PFCP message, as specified in 3GPP TS 29.244 [29]. (NOTE 2) |
| NOTE 1: Using 3GPP vendor content subtypes allows to describe the nature of the opaque payload (e.g. NGAP or 5GS NAS information) without having to rely on metadata in the JSON payload. NOTE 2: Binary encoded payload in vnd.3gpp.pfcp content subtype shall include application layer headers for PFCP and shall not include transport layer headers, i.e. IP and UDP. |  |

See clause 6.1.2.4 for the binary payloads supported in the binary body part of multipart messages.

#### 6.1.2.3	HTTP custom headers

##### 6.1.2.3.1	General

In this release of the specification, no specific custom headers are defined for the Nsmf_PDUSession service.

For 3GPP specific HTTP custom headers used across all service based interfaces, see clause 5.2.3 of 3GPP TS 29.500 [4].

##### 6.1.2.3.2	3gpp-Sbi-Origination-Timestamp

The header contains the date and time (with a millisecond granularity) when the originating entity initiated the request.

The encoding of the header follows the ABNF as defined in IETF RFC 7230 [31].

3gpp-Sbi-Origination-Timestamp = "3gpp-Sbi-Origination-Timestamp" ":" day-name "," SP date1 SP time-of-day "." milliseconds SP GMT

milliseconds = 3DIGIT

day-name, date1, time-of-day shall comply with the definition in clause 7.1.1.1 of IETF RFC 7231 [32].

NOTE:	This is the same format as the Date header of clause 7.1.1.2 of IETF RFC 7231 [32], but with the time expressed with a millisecond granularity.

EXAMPLE:	3gpp-Sbi-Origination-Timestamp: Sun, 04 Aug 2019 08:49:37.845 GMT

#### 6.1.2.4	HTTP multipart messages

HTTP multipart messages shall be supported, to transfer opaque N1 and/or N2 SMpayloads or N4 information, in the following service operations (and HTTP messages):

-	Create SM Context Request and Response (POST);

-	Update SM Context Request and Response (POST);

-	Release SM Context Request (POST);

-	Create Request and Response (POST);

-	Update Request and Response (POST (modify)).

HTTP multipart messages shall include one JSON body part and one or two binary body parts comprising:

-	an N1 SM payload, an N2 SM payload or both, over N11 (see clause 6.1.6.4);

-	one or two N1 SM payloads, over N16 (see clause 6.1.6.4);

-	one or two N2 SM payloads over N11 (see clause 5.2.2.3.3);

-	one, two or three N4 payloads over N16a (see clause 6.1.6.4.5).

The JSON body part shall be the "root" body part of the multipart message. It shall be encoded as the first body part of the multipart message. The "Start" parameter does not need to be included.

The multipart message shall include a "type" parameter (see IETF RFC 2387 [10]) specifying the media type of the root body part, i.e. "application/json".

NOTE:	The "root" body part (or "root" object) is the first body part the application processes when receiving a multipart/related message, see IETF RFC 2387 [10]. The default root is the first body within the multipart/related message. The "Start" parameter indicates the root body part, e.g. when this is not the first body part in the message.

For each binary body part in a HTTP multipart message, the binary body part shall include a Content-ID header (see IETF RFC 2045 [12]), and the JSON body part shall include an attribute, defined with the RefToBinaryData type, that contains the value of the Content-ID header field of the referenced binary body part.

Examples of multipart/related messages can be found in Annex B.

#### 6.1.2.5	HTTP/2 request retries

The principles specified in clause 5.2.8 of 3GPP TS 29.500 [4] shall be applied with the following modifications.

The NF Service Consumer of Nsmf_PDUSession service, e.g. the AMF, shall retry the same HTTP request in the following scenarios through a new TCP connection towards an (alternative) service endpoint pertaining to the same NF (Service) instance or set if the corresponding procedure triggering the service request allows such retries, e.g. before the timeout of the corresponding N1 or N2 procedure:

-	If the stream for the service request has not been processed in the SMF as specified in clause 5.2.8 of 3GPP TS 29.500 [4];

-	if the request is rejected by a HTTP status code indicating a temporary failure in the SMF, e.g. the status code 429, 500 and 503, as specified in clause 5.2.7 of 3GPP TS 29.500 [4];

-	if the request is timeout (i.e. the NF Service consumer doesn't receive any response after an implementation specific timer expires).

The NF Service Consumer shall determine an alternative service instance as specified in clause 6.5 of 3GPP TS 23.527 [24], i.e. using Binding Indication if supported/available or the NF (service) set or service persistency info from NF profile. The NF Service Consumer should also consider the Load control Information and the Overload Control Information if available as specified in clauses 6.3 and 6.4 in 3GPP TS 29.500 [4] when reselecting an alternative NF service instance.

The SMF shall support handling repeated retries successfully.

NOTE:	See clauses 5.2.2.2 and 5.2.2.7 for the handling by the SMF of an HTTP POST request that would have already been processed by the SMF and that would be retried by the NF Service Consumer.

HTTP conditional requests are not supported by the Nsmf_PDUSession service in this version of the API.

### 6.1.3	Resources

#### 6.1.3.1	Overview

Figure 6.1.3.1-1 describes the resource URI structure of the Nsmf_PDUSession API.

Figure 6.1.3.1-1: Resource URI structure of the Nsmf_PDUSession API

NOTE:	The sm-contexts and pdu-sessions collection resources can be distributed on different processing instances or hosts. Thus, the authority and/or deployment-specific string of the apiRoot of the created individual sm context and pdu-session resources' URIs can differ from the authority and/or deployment-specific string of the apiRoot of the sm-contexts and pdu-sessions distributed collections' URIs.

Table 6.1.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.1.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| SM contexts collection | /sm-contexts | POST | Create SM Context |
| Individual SM context | /sm-contexts/{smContextRef}/retrieve | retrieve (POST) | Retrieve SM Context |
|  | /sm-contexts/{smContextRef}/modify | modify (POST) | Update SM Context |
|  | /sm-contexts/{smContextRef}/release | release (POST) | Release SM Context |
|  | /sm-contexts/{smContextRef}/send-mo-data | send-mo-data (POST) | Send MO Data |
| PDU sessions collection (H-SMF or SMF) | /pdu-sessions | POST | Create |
| Individual PDU session (H-SMF or SMF) | /pdu-sessions/{pduSessionRef}/modify | modify (POST) | Update (initiated by V-SMF or I-SMF) |
|  | /pdu-sessions/{pduSessionRef}/release | release (POST) | Release |
|  | /pdu-sessions/{pduSessionRef}/retrieve | retrieve (POST) | Retrieve |
|  | /pdu-sessions/{pduSessionRef}/transfer-mo-data | transfer-mo-data (POST) | Transfer MO Data |
| Individual PDU session (V-SMF or I-SMF) | {vsmfPduSessionUri}/modify or {ismfPduSessionUri}/modify | modify (POST) | Update (initiated by H-SMF or SMF) |
|  | {vsmfPduSessionUri} or {ismfPduSessionUri} | POST | Notify Status |
|  | {vsmfPduSessionUri}/transfer-mt-data or {ismfPduSessionUri}/ transfer-mt-data | transfer-mt-data (POST) | Transfer MT Data |

#### 6.1.3.2	Resource: SM contexts collection

##### 6.1.3.2.1	Description

This resource represents the collection of the individual SM contexts created in the SMF.

This resource is modelled with the Collection resource archetype (see clause C.2 of 3GPP TS 29.501 [5]).

##### 6.1.3.2.2	Resource Definition

Resource URI: {apiRoot}/nsmf-pdusession/<apiVersion>/sm-contexts

This resource shall support the resource URI variables defined in table 6.1.3.2.2-1.

Table 6.1.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1. |
| apiVersion | string | See clause 6.1.1. |

##### 6.1.3.2.3	Resource Standard Methods

###### 6.1.3.2.3.1	POST

This method creates an individual SM context resource in the SMF, or in V-SMF in HR roaming scenarios.

This method shall support the URI query parameters specified in table 6.1.3.2.3.1-1.

Table 6.1.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.2.3.1-2 and the response data structures and response codes specified in table 6.1.3.2.3.1-3.

Table 6.1.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SmContextCreateData | M | 1 | Representation of the SM context to be created in the SMF. |

Table 6.1.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SmContextCreatedData | M | 1 | 201 Created | Successful creation of an SM context. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing a different URI, or the same URI if a request is redirected to the same target resource via a different SCP. In the former case, the URI shall be an alternative URI of the resource located on an alternative service instance within the same SMF or SMF (service) set. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing a different URI, or the same URI if a request is redirected to the same target resource via a different SCP. In the former case, the URI shall be an alternative URI of the resource located on an alternative service instance within the same SMF or SMF (service) set. (NOTE 2) |
| SmContextCreateError | M | 1 | 400 Bad Request | The "cause" attribute shall be set to one of the errors defined in Table 5.2.7.2-1 of 3GPP TS 29.500 [4]. |
| ProblemDetails | O | 0..1 | 400 Bad Request | This error shall only be returned by an SCP for errors it originates. As an exception, this error may also be returned by an SMF, with an empty payload body, for a protocol error other than those specified for the SMF PDUSession service logic (e.g. protocol error found by the HTTP stack). |
| SmContextCreateError | M | 1 | 403 Forbidden | The "cause" attribute shall be set to one of the following application error: - N1_SM_ERROR - N2_SM_ERROR - SNSSAI_DENIED - DNN_DENIED - PDUTYPE_DENIED - SSC_DENIED - SUBSCRIPTION_DENIED - DNN_NOT_SUPPORTED - PDUTYPE_NOT_SUPPORTED - SSC_NOT_SUPPORTED - HOME_ROUTED_ROAMING_REQUIRED - OUT_OF_LADN_SERVICE_AREA - NO_EPS_5GS_CONTINUITY - INTEGRITY_PROTECTED_MDR_NOT_ACCEPTABLE - DEFAULT_EPS_BEARER_INACTIVE - NOT_SUPPORTED_WITH_ISMF - SERVICE_NOT_AUTHORIZED_BY_NEXT_HOP See table 6.1.7.3-1 for the description of these errors. |
| ProblemDetails | O | 0..1 | 403 Forbidden | This error shall only be returned by an SCP for errors it originates. As an exception, this error may also be returned by an SMF, with an empty payload body, for a protocol error other than those specified for the SMF PDUSession service logic (e.g. protocol error found by the HTTP stack). |
| SmContextCreateError | M | 1 | 404 Not Found | The "cause" attribute shall be set to one of the following application error: - CONTEXT_NOT_FOUND See table 6.1.7.3-1 for the description of these errors. |
| ExtProblemDetails | O | 0..1 | 413 Payload Too Large |  |
| ExtProblemDetails | O | 0..1 | 415 Unsupported Media Type |  |
| ExtProblemDetails | O | 0..1 | 429 Too Many Requests |  |
| SmContextCreateError | M | 1 | 500 Internal Server Error | The "cause" attribute shall be set to one of the errors defined in Table 5.2.7.2-1 of 3GPP TS 29.500 [4] or to one of the following application errors: - INSUFFICIENT_RESOURCES_SLICE - INSUFFICIENT_RESOURCES_SLICE_DNN See table 6.1.7.3-1 for the description of these errors. |
| ProblemDetails | O | 0..1 | 500 Internal Server Error | This error shall only be returned by an SCP for errors it originates. As an exception, this error may also be returned by an SMF, with an empty payload body, for a general server error other than those specified for the SMF PDUSession service logic. |
| SmContextCreateError | M | 1 | 503 Service Unavailable | The "cause" attribute shall be set to one of the errors defined in Table 5.2.7.2-1 of 3GPP TS 29.500 [4] or to one of the following application errors: - DNN_CONGESTION - S_NSSAI_CONGESTION See table 6.1.7.3-1 for the description of these errors. |
| ProblemDetails | O | 0..1 | 503 Service Unavailable | This error shall only be returned by an SCP for errors it originates. As an exception, this error may also be returned by an SMF, with an empty payload body, for a general server error other than those specified for the SMF PDUSession service logic. |
| SmContextCreateError | M | 1 | 504 Gateway Timeout | The "cause" attribute shall be set to one of the following application error: - PEER_NOT_RESPONDING - NETWORK_FAILURE See table 6.1.7.3-1 for the description of these errors. |
| ProblemDetails | O | 0..1 | 504 Gateway Timeout | This error shall only be returned by an SCP for errors it originates. |
| NOTE 1: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with a ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). NOTE 2: RedirectResponse may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.1.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/nsmf-pdusession/<apiVersion>/sm-contexts/{smContextRef} |

Table 6.1.3.2.3.1-5: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same SMF or SMF (service) set. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target SMF (service) instance ID towards which the request is redirected |

Table 6.1.3.2.3.1-6: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same SMF or SMF (service) set. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target SMF (service) instance ID towards which the request is redirected |

##### 6.1.3.2.4	Resource Custom Operations

None.

#### 6.1.3.3	Resource: Individual SM context

##### 6.1.3.3.1	Description

This resource represents an individual SM context created in the SMF.

This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [5]).

##### 6.1.3.3.2	Resource Definition

Resource URI: {apiRoot}/nsmf-pdusession/<apiVersion>/sm-contexts/{smContextRef}

This resource shall support the resource URI variables defined in table 6.1.3.3.2-1.

Table 6.1.3.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1. |
| apiVersion | string | See clause 6.1.1. |
| smContextRef | string | SM context reference assigned by the SMF during the Create SM Context service operation. |

##### 6.1.3.3.3	Resource Standard Methods

None.

##### 6.1.3.3.4	Resource Custom Operations

###### 6.1.3.3.4.1	Overview

Table 6.1.3.3.4.1-1: Custom operations

| Operation Name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| modify | {resourceUri}/modify | POST | Update SM Context service operation |
| release | {resourceUri}/release | POST | Release SM Context service operation |
| retrieve | {resourceUri}/retrieve | POST | Retrieve SM Context service operation |
| send-mo-data | {resourceUri}/send-mo-data | POST | Send MO Data service operation |

###### 6.1.3.3.4.2	Operation: modify

6.1.3.3.4.2.1	Description

6.1.3.3.4.2.2	Operation Definition

This custom operation updates an individual SM context resource and/or provide N1 or N2 SM information received from the UE or the AN, for a given PDU session, towards the SMF, or in V-SMF in HR roaming scenario.

This operation shall support the request data structures specified in table 6.1.3.3.4.2.2-1 and the response data structure and response codes specified in table 6.1.3.3.4.2.2-2.

Table 6.1.3.3.4.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SmContextUpdateData | M | 1 | Representation of the updates to apply to the SM context. |

Table 6.1.3.3.4.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SmContextUpdatedData | C | 0..1 | 200 OK | Successful update of the SM context, when the SMF needs to return information in the response. |
| n/a |  |  | 204 No Content | Successful update of the SM context, when the SMF does not need to return information in the response. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing a different URI, or the same URI if a request is redirected to the same target resource via a different SCP. In the former case, the URI shall be an alternative URI of the resource located on an alternative service instance within the same SMF or SMF (service) set. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing a different URI, or the same URI if a request is redirected to the same target resource via a different SCP. In the former case, the URI shall be an alternative URI of the resource located on an alternative service instance within the same SMF or SMF (service) set. (NOTE 2) |
| SmContextUpdateError | M | 1 | 400 Bad Request | The "cause" attribute shall be set to one of the errors defined in Table 5.2.7.2-1 of 3GPP TS 29.500 [4]. |
| ProblemDetails | O | 0..1 | 400 Bad Request | This error shall only be returned by an SCP for errors it originates. As an exception, this error may also be returned by an SMF, with an empty payload body, for a protocol error other than those specified for the SMF PDUSession service logic (e.g. protocol error found by the HTTP stack). |
| SmContextUpdateError | M | 1 | 403 Forbidden | The "cause" attribute shall be set to one of the following application error: - N1_SM_ERROR - N2_SM_ERROR - SUBSCRIPTION_DENIED - OUT_OF_LADN_SERVICE_AREA - PRIORITIZED_SERVICES_ONLY - PDU_SESSION_ANCHOR_CHANGE - NO_DATA_FORWARDING See table 6.1.7.3-1 for the description of these errors. |
| ProblemDetails | O | 0..1 | 403 Forbidden | This error shall only be returned by an SCP for errors it originates. As an exception, this error may also be returned by an SMF, with an empty payload body, for a protocol error other than those specified for the SMF PDUSession service logic (e.g. protocol error found by the HTTP stack). |
| SmContextUpdateError | M | 1 | 404 Not Found | The "cause" attribute shall be set to one of the following application error: - CONTEXT_NOT_FOUND See table 6.1.7.3-1 for the description of these errors. |
| ExtProblemDetails | O | 0..1 | 413 Payload Too Large |  |
| ExtProblemDetails | O | 0..1 | 415 Unsupported Media Type |  |
| ExtProblemDetails | O | 0..1 | 429 Too Many Requests |  |
| SmContextUpdateError | M | 1 | 500 Internal Server Error | The "cause" attribute shall be set to one of the errors defined in Table 5.2.7.2-1 of 3GPP TS 29.500 [4]. |
| ProblemDetails | O | 0..1 | 500 Internal Server Error | This error shall only be returned by an SCP for errors it originates. As an exception, this error may also be returned by an SMF, with an empty payload body, for a general server error other than those specified for the SMF PDUSession service logic. |
| SmContextUpdateError | M | 1 | 503 Service Unavailable | The "cause" attribute shall be set to one of the errors defined in Table 5.2.7.2-1 of 3GPP TS 29.500 [4] or to one of the following application errors: - DNN_CONGESTION - S-NSSAI_ CONGESTION See table 6.1.7.3-1 for the description of these errors. |
| ProblemDetails | O | 0..1 | 503 Service Unavailable | This error shall only be returned by an SCP for errors it originates. As an exception, this error may also be returned by an SMF, with an empty payload body, for a general server error other than those specified for the SMF PDUSession service logic. |
| NOTE 1: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with a ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). NOTE 2: RedirectResponse may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.1.3.3.4.2.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same SMF or SMF (service) set. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target SMF (service) instance ID towards which the request is redirected |

Table 6.1.3.3.4.2.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same SMF or SMF (service) set. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target SMF (service) instance ID towards which the request is redirected |

###### 6.1.3.3.4.3	Operation: release

6.1.3.3.4.3.1	Description

6.1.3.3.4.3.2	Operation Definition

This custom operation releases an individual SM context resource in the SMF, or in V-SMF in HR roaming scenario

This operation shall support the request data structures specified in table 6.1.3.3.4.3.2-1 and the response data structure and response codes specified in table 6.1.3.3.4.3.2-2.

Table 6.1.3.3.4.3.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SmContextReleaseData | C | 0..1 | Representation of the data to be sent to the SMF when releasing the SM context. |

Table 6.1.3.3.4.3.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SmContextReleasedData | M | 1 | 200 OK | Successful release of an SM context, when information needs to be returned to the NF Service Consumer (NOTE 2). |
| n/a |  |  | 204 No Content | Successful release of an SM context. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing a different URI, or the same URI if a request is redirected to the same target resource via a different SCP. In the former case, the URI shall be an alternative URI of the resource located on an alternative service instance within the same SMF or SMF (service) set. (NOTE 3) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing a different URI, or the same URI if a request is redirected to the same target resource via a different SCP. In the former case, the URI shall be an alternative URI of the resource located on an alternative service instance within the same SMF or SMF (service) set. (NOTE 3) |
| NOTE 1: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with a ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). NOTE 2: The support for 200 OK shall be dependent on the support of the indicated feature. NOTE 3: RedirectResponse may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.1.3.3.4.3.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same SMF or SMF (service) set. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target SMF (service) instance ID towards which the request is redirected |

Table 6.1.3.3.4.3.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same SMF or SMF (service) set. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target SMF (service) instance ID towards which the request is redirected |

###### 6.1.3.3.4.4	Operation: retrieve

6.1.3.3.4.4.1	Description

6.1.3.3.4.4.2	Operation Definition

This custom operation retrieves an individual SM context resource from the SMF, from the V-SMF in HR roaming scenario or from the I-SMF.

This operation shall support the request data structures specified in table 6.1.3.3.4.4.2-1 and the response data structure and response codes specified in table 6.1.3.3.4.4.2-2.

Table 6.1.3.3.4.4.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SmContextRetrieveData | O | 0..1 | Optional parameters used to retrieve the SM context, e.g. target MME capabilities, SM context type. |

Table 6.1.3.3.4.4.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SmContextRetrievedData | M | 1 | 200 OK | Successful retrieval of the SM context. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing a different URI, or the same URI if a request is redirected to the same target resource via a different SCP. In the former case, the URI shall be an alternative URI of the resource located on an alternative service instance within the same SMF or SMF (service) set. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing a different URI, or the same URI if a request is redirected to the same target resource via a different SCP. In the former case, the URI shall be an alternative URI of the resource located on an alternative service instance within the same SMF or SMF (service) set. (NOTE 2) |
| ProblemDetails | M | 1 | 403 Forbidden | The "cause" attribute shall be set to one of the following application error: - TARGET_MME_CAPABILITY - DEFAULT_EBI_NOT_TRANSFERRED See table 6.1.7.3-1 for the description of these errors. |
| ProblemDetails | O | 0..1 | 504 Gateway Timeout | The "cause" attribute may be set to one of the following application errors: - UPF_NOT_RESPONDING See table 6.1.7.3-1 for the description of these errors. |
| NOTE 1: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with a ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). NOTE 2: RedirectResponse may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.1.3.3.4.4.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same SMF or SMF (service) set. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target SMF (service) instance ID towards which the request is redirected |

Table 6.1.3.3.4.4.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same SMF or SMF (service) set. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target SMF (service) instance ID towards which the request is redirected |

###### 6.1.3.3.4.5	Operation: send-mo-data

6.1.3.3.4.5.1	Description

6.1.3.3.4.5.2	Operation Definition

This custom operation enables to send mobile originated data received over NAS, for a given PDU session, towards the SMF, or the V-SMF for HR roaming scenarios, or the I-SMF for a PDU session with an I-SMF.

This operation shall support the request data structures specified in table 6.1.3.3.4.5.2-1 and the response data structure and response codes specified in table 6.1.3.3.4.5.2-2.

Table 6.1.3.3.4.5.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SendMoDataReqData | M | 1 | Representation of the payload of a Send MO Data Request |

Table 6.1.3.3.4.5.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful MO data transfer |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing a different URI, or the same URI if a request is redirected to the same target resource via a different SCP. In the former case, the URI shall be an alternative URI of the resource located on an alternative service instance within the same SMF or SMF (service) set. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing a different URI, or the same URI if a request is redirected to the same target resource via a different SCP. In the former case, the URI shall be an alternative URI of the resource located on an alternative service instance within the same SMF or SMF (service) set. (NOTE 2) |
| ExtProblemDetails | O | 0..1 | 400 Bad Request |  |
| ExtProblemDetails | O | 0..1 | 401 Unauthorized |  |
| ExtProblemDetails | O | 0..1 | 403 Forbidden |  |
| ExtProblemDetails | O | 0..1 | 404 Not Found |  |
| ExtProblemDetails | O | 0..1 | 413 Payload Too Large |  |
| ExtProblemDetails | O | 0..1 | 415 Unsupported Media Type |  |
| ExtProblemDetails | O | 0..1 | 429 Too Many Requests |  |
| ExtProblemDetails | O | 0..1 | 500 Internal Server Error |  |
| ExtProblemDetails | O | 0..1 | 503 Service Unavailable |  |
| NOTE 1: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with a ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). NOTE 2: RedirectResponse may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.1.3.3.4.5.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same SMF or SMF (service) set. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target SMF (service) instance ID towards which the request is redirected |

Table 6.1.3.3.4.5.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same SMF or SMF (service) set. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target SMF (service) instance ID towards which the request is redirected |

#### 6.1.3.4	Void

#### 6.1.3.5	Resource: PDU sessions collection (H-SMF or SMF)

##### 6.1.3.5.1	Description

This resource represents the collection of the individual PDU sessions created in the H-SMF for HR PDU sessions or in the SMF for PDU sessions with an I-SMF.

This resource is modelled with the Collection resource archetype (see clause C.2 of 3GPP TS 29.501 [5]).

##### 6.1.3.5.2	Resource Definition

Resource URI: {apiRoot}/nsmf-pdusession/<apiVersion>/pdu-sessions

This resource shall support the resource URI variables defined in table 6.1.3.5.2-1.

Table 6.1.3.5.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1. |
| apiVersion | string | See clause 6.1.1. |

##### 6.1.3.5.3	Resource Standard Methods

###### 6.1.3.5.3.1	POST

This method creates an individual PDU session resource in the H-SMF or SMF.

This method shall support the URI query parameters specified in table 6.1.3.5.3.1-1.

Table 6.1.3.5.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.5.3.1-2 and the response data structures and response codes specified in table 6.1.3.5.3.1-3.

Table 6.1.3.5.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| PduSessionCreateData | M | 1 | Representation of the PDU session to be created in the H-SMF or SMF. |

Table 6.1.3.5.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| PduSessionCreatedData | M | 1 | 201 Created | Successful creation of a PDU session. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing a different URI, or the same URI if a request is redirected to the same target resource via a different SCP. In the former case, the URI shall be an alternative URI of the resource located on an alternative service instance within the same (H-)SMF or (H-)SMF (service) set. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing a different URI, or the same URI if a request is redirected to the same target resource via a different SCP. In the former case, the URI shall be an alternative URI of the resource located on an alternative service instance within the same (H-)SMF or (H-)SMF (service) set. (NOTE 2) |
| PduSessionCreateError | M | 1 | 400 Bad Request | The "cause" attribute shall be set to one of the errors defined in Table 5.2.7.2-1 of 3GPP TS 29.500 [4]. |
| ProblemDetails | O | 0..1 | 400 Bad Request | This error shall only be returned by an SCP or a SEPP for errors they originate. As an exception, this error may also be returned by an SMF, with an empty payload body, for a protocol error other than those specified for the SMF PDUSession service logic (e.g. protocol error found by the HTTP stack). |
| PduSessionCreateError | M | 1 | 403 Forbidden | The "cause" attribute shall be set to one of the following application error: - N1_SM_ERROR - SNSSAI_DENIED - DNN_DENIED - PDUTYPE_DENIED - SSC_DENIED - SUBSCRIPTION_DENIED - DNN_NOT_SUPPORTED - PDUTYPE_NOT_SUPPORTED - SSC_NOT_SUPPORTED - NO_EPS_5GS_CONTINUITY - INTEGRITY_PROTECTED_MDR_NOT_ACCEPTABLE - NOT_SUPPORTED_WITH_ISMF See table 6.1.7.3-1 for the description of these errors. |
| ProblemDetails | O | 0..1 | 403 Forbidden | This error shall only be returned by an SCP or a SEPP for errors they originate. As an exception, this error may also be returned by an SMF, with an empty payload body, for a protocol error other than those specified for the SMF PDUSession service logic (e.g. protocol error found by the HTTP stack). |
| PduSessionCreateError | M | 1 | 404 Not Found | The "cause" attribute shall be set to one of the following application error: - CONTEXT_NOT_FOUND See table 6.1.7.3-1 for the description of these errors. |
| PduSessionCreateError | M | 1 | 500 Internal Server Error | The "cause" attribute shall be set to one of the errors defined in Table 5.2.7.2-1 of 3GPP TS 29.500 [4] or to one of the following application errors: - INSUFFIC_RESOURCES_SLICE - INSUFFIC_RESOURCES_SLICE_DNN See table 6.1.7.3-1 for the description of these errors. |
| ProblemDetails | O | 0..1 | 500 Internal Server Error | This error shall only be returned by an SCP or a SEPP for errors they originate. As an exception, this error may also be returned by an SMF, with an empty payload body, for a general server error other than those specified for the SMF PDUSession service logic. |
| PduSessionCreateError | M | 1 | 503 Service Unavailable | The "cause" attribute shall be set to one of the errors defined in Table 5.2.7.2-1 of 3GPP TS 29.500 [4] or to one of the following application errors: - DNN_CONGESTION - S-NSSAI_ CONGESTION See table 6.1.7.3-1 for the description of these errors. |
| ProblemDetails | O | 0..1 | 503 Service Unavailable | This error shall only be returned by an SCP or a SEPP for errors they originate. As an exception, this error may also be returned by an SMF, with an empty payload body, for a general server error other than those specified for the SMF PDUSession service logic. |
| PduSessionCreateError | M | 1 | 504 Gateway Timeout | The "cause" attribute shall be set to one of the following application error: - PEER_NOT_RESPONDING - NETWORK_FAILURE See table 6.1.7.3-1 for the description of these errors. |
| ProblemDetails | O | 0..1 | 504 Gateway Timeout | This error shall only be returned by an SCP or a SEPP for errors they originate. |
| NOTE 1: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with a ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). NOTE 2: RedirectResponse may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.1.3.5.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/nsmf-pdusession/<apiVersion>/pdu-sessions/{pduSessionRef} |

Table 6.1.3.5.3.1-5: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same SMF or SMF (service) set. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target SMF (service) instance ID towards which the request is redirected |

Table 6.1.3.5.3.1-6: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same SMF or SMF (service) set. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target SMF (service) instance ID towards which the request is redirected |

##### 6.1.3.5.4	Resource Custom Operations

###### 6.1.3.5.4.1	Overview

Table 6.1.3.5.4.1-1: Custom operations

| Custom operaration URI | Mapped HTTP method | Description |
| --- | --- | --- |
| n/a |  |  |

#### 6.1.3.6	Resource: Individual PDU session (H-SMF or SMF)

##### 6.1.3.6.1	Description

This resource represents an individual PDU session created in the H-SMF for a HR PDU session or in the SMF for a PDU session with an I-SMF.

This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [5]).

##### 6.1.3.6.2	Resource Definition

Resource URI: {apiRoot}/nsmf-pdusession/<apiVersion>/pdu-sessions/{pduSessionRef}

This resource shall support the resource URI variables defined in table 6.1.3.6.2-1.

Table 6.1.3.6.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1. |
| apiVersion | string | See clause 6.1.1. |
| pduSessionRef | string | PDU session reference assigned by the H-SMF or SMF during the Create service operation. |

##### 6.1.3.6.3	Resource Standard Methods

None.

##### 6.1.3.6.4	Resource Custom Operations

###### 6.1.3.6.4.1	Overview

Table 6.1.3.6.4.1-1: Custom operations

| Operation Name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| modify | {resourceUri}/modify | POST | Update service operation |
| release | {resourceUri}/release | POST | Release service operation |
| transfer-mo-data | {resourceUri}/ transfer-mo-data | POST | Transfer MO Data service operation |
| retrieve | {resourceUri}/retrieve | POST | Retrieve service operation |

###### 6.1.3.6.4.2	Operation: modify

6.1.3.6.4.2.1	Description

6.1.3.6.4.2.2	Operation Definition

This custom operation updates an individual PDU session resource in the H-SMF or SMF and/or provide the H-SMF or SMF with information received by the V-SMF or I-SMF in N1 SM signalling from the UE.

This operation shall support the request data structures specified in table 6.1.3.6.4.2.2-1 and the response data structure and response codes specified in table 6.1.3.6.4.2.2-2.

Table 6.1.3.6.4.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| HsmfUpdateData | M | 1 | Representation of the updates to apply to the PDU session. |

Table 6.1.3.6.4.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| HsmfUpdatedData | C | 0..1 | 200 OK | This case represents a successful update of the PDU session, when the H-SMF or SMF needs to return information in the response. |
| n/a |  |  | 204 No Content | This case represents a successful update of the PDU session, when the H-SMF or SMF does not need to return information in the response. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing a different URI, or the same URI if a request is redirected to the same target resource via a different SCP. In the former case, the URI shall be an alternative URI of the resource located on an alternative service instance within the same (H-)SMF or (H-)SMF (service) set. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing a different URI, or the same URI if a request is redirected to the same target resource via a different SCP. In the former case, the URI shall be an alternative URI of the resource located on an alternative service instance within the same (H-)SMF or (H-)SMF (service) set. (NOTE 2) |
| HsmfUpdateError | M | 1 | 400 Bad Request | The "cause" attribute shall be set to one of the errors defined in Table 5.2.7.2-1 of 3GPP TS 29.500 [4]. |
| ProblemDetails | O | 0..1 | 400 Bad Request | This error shall only be returned by an SCP or a SEPP for errors they originate. As an exception, this error may also be returned by an SMF, with an empty payload body, for a protocol error other than those specified for the SMF PDUSession service logic (e.g. protocol error found by the HTTP stack). |
| HsmfUpdateError | M | 1 | 403 Forbidden | The "cause" attribute shall be set to one of the following application errors: - N1_SM_ERROR - SUBSCRIPTION_DENIED - PDU_SESSION_ANCHOR_CHANGE See table 6.1.7.3-1 for the description of these errors. |
| ProblemDetails | O | 0..1 | 403 Forbidden | This error shall only be returned by an SCP or a SEPP for errors they originate. As an exception, this error may also be returned by an SMF, with an empty payload body, for a protocol error other than those specified for the SMF PDUSession service logic (e.g. protocol error found by the HTTP stack). |
| HsmfUpdateError | M | 1 | 404 Not Found | The "cause" attribute shall be set to one of the following application error: - CONTEXT_NOT_FOUND See table 6.1.7.3-1 for the description of these errors. |
| HsmfUpdateError | M | 1 | 500 Internal Server Error | The "cause" attribute shall be set to one of the errors defined in Table 5.2.7.2-1 of 3GPP TS 29.500 [4]. |
| ProblemDetails | O | 0..1 | 500 Internal Server Error | This error shall only be returned by an SCP or a SEPP for errors they originate. As an exception, this error may also be returned by an SMF, with an empty payload body, for a general server error other than those specified for the SMF PDUSession service logic. |
| HsmfUpdateError | M | 1 | 503 Service Unavailable | The "cause" attribute shall be set to one of the errors defined in Table 5.2.7.2-1 of 3GPP TS 29.500 [4] or to one of the following application errors: - DNN_CONGESTION - S-NSSAI_ CONGESTION See table 6.1.7.3-1 for the description of these errors. |
| ProblemDetails | O | 0..1 | 503 Service Unavailable | This error shall only be returned by an SCP or a SEPP for errors they originate. As an exception, this error may also be returned by an SMF, with an empty payload body, for a general server error other than those specified for the SMF PDUSession service logic. |
| NOTE 1: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with a ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). NOTE 2: RedirectResponse may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table Table 6.1.3.6.4.2.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same SMF or SMF (service) set. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target SMF (service) instance ID towards which the request is redirected |

Table Table 6.1.3.6.4.2.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same SMF or SMF (service) set. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target SMF (service) instance ID towards which the request is redirected |

###### 6.1.3.6.4.3	Operation: release

6.1.3.6.4.3.1	Description

6.1.3.6.4.3.2	Operation Definition

This custom operation releases an individual PDU session resource in the H-SMF for a HR PDU session or in the SMF for a PDU session with an I-SMF.

This operation shall support the request data structures specified in table 6.1.3.6.4.3.2-1 and the response data structure and response codes specified in table 6.1.3.6.4.3.2-2.

Table 6.1.3.6.4.3.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ReleaseData | C | 0..1 | Representation of the data to be sent to the H-SMF or SMF when releasing the PDU session. |

Table 6.1.3.6.4.3.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ReleasedData | M | 1 | 200 OK | Successful release of a PDU session context, when information needs to be returned to the NF Service Consumer. (NOTE 2) |
| n/a |  |  | 204 No Content | Successful release of a PDU session. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing a different URI, or the same URI if a request is redirected to the same target resource via a different SCP. In the former case, the URI shall be an alternative URI of the resource located on an alternative service instance within the same (H-)SMF or (H-)SMF (service) set. (NOTE 3) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing a different URI, or the same URI if a request is redirected to the same target resource via a different SCP. In the former case, the URI shall be an alternative URI of the resource located on an alternative service instance within the same (H-)SMF or (H-)SMF (service) set. (NOTE 3) |
| NOTE 1: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with a ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). NOTE 2: The support for 200 OK shall be dependent on the support of the indicated feature. NOTE 3: RedirectResponse may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.1.3.6.4.3.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same SMF or SMF (service) set. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target SMF (service) instance ID towards which the request is redirected |

Table 6.1.3.6.4.3.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same SMF or SMF (service) set. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target SMF (service) instance ID towards which the request is redirected |

###### 6.1.3.6.4.4	Operation: transfer-mo-data

6.1.3.6.4.4.1	Description

6.1.3.6.4.4.2	Operation Definition

This custom operation enables to transfer mobile originated data received from AMF, for a given PDU session, towards the H-SMF for HR roaming scenarios, or the SMF for a PDU session with an I-SMF.

This operation shall support the request data structures specified in table 6.1.3.6.4.4.2-1 and the response data structure and response codes specified in table 6.1.3.6.4.4.2-2.

Table 6.1.3.6.4.4.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| TransferMoDataReqData | M | 1 | Representation of the payload of a Transfer MO Data Request |

Table 6.1.3.6.4.4.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful MO data transfer |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing a different URI, or the same URI if a request is redirected to the same target resource via a different SCP. In the former case, the URI shall be an alternative URI of the resource located on an alternative service instance within the same (H-)SMF or (H-)SMF (service) set. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing a different URI, or the same URI if a request is redirected to the same target resource via a different SCP. In the former case, the URI shall be an alternative URI of the resource located on an alternative service instance within the same (H-)SMF or (H-)SMF (service) set. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with a ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). NOTE 2: RedirectResponse may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.1.3.6.4.4.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same SMF or SMF (service) set. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target SMF (service) instance ID towards which the request is redirected |

Table 6.1.3.6.4.4.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same SMF or SMF (service) set. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target SMF (service) instance ID towards which the request is redirected |

###### 6.1.3.6.4.5	Operation: retrieve

6.1.3.6.4.5.1	Description

6.1.3.6.4.5.2	Operation Definition

This custom operation retrieves information from an individual PDU session context in the H-SMF for a HR PDU session or in the SMF for a PDU session with an I-SMF.

This operation shall support the request data structures specified in table 6.1.3.6.4.5.2-1 and the response data structure and response codes specified in table 6.1.3.6.4.5.2-2.

Table 6.1.3.6.4.5.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| RetrieveData | M | 1 | Representation of the payload of a Retrieve Request |

Table 6.1.3.6.4.5.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| RetrievedData | M | 1 | 200 OK | Successful retrieval of information from a PDU session context. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing a different URI, or the same URI if a request is redirected to the same target resource via a different SCP. In the former case, the URI shall be an alternative URI of the resource located on an alternative service instance within the same (H-)SMF or (H-)SMF (service) set. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing a different URI, or the same URI if a request is redirected to the same target resource via a different SCP. In the former case, the URI shall be an alternative URI of the resource located on an alternative service instance within the same (H-)SMF or (H-)SMF (service) set. (NOTE 2) |
| ProblemDetails | O | 0..1 | 504 Gateway Timeout | The "cause" attribute may be set to one of the following application errors: - UPF_NOT_RESPONDING See table 6.1.7.3-1 for the description of these errors. |
| NOTE 1: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with a ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). NOTE 2: RedirectResponse may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.1.3.6.4.5.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same SMF or SMF (service) set. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target SMF (service) instance ID towards which the request is redirected |

Table 6.1.3.6.4.5.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same SMF or SMF (service) set. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target SMF (service) instance ID towards which the request is redirected |

#### 6.1.3.7	Resource: Individual PDU session (V-SMF or I-SMF)

##### 6.1.3.7.1	Description

This resource represents an individual PDU session created in the V-SMF for a HR PDU session or in the I-SMF for a PDU session with an I-SMF.

This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [5]).

##### 6.1.3.7.2	Resource Definition

Callback URI: {vsmfPduSessionUri} or {ismfPduSessionUri}

This resource shall support the callback URI variables defined in table 6.1.3.7.2-1.

Table 6.1.3.7.2-1: Callback URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| vsmfPduSessionUri | Uri | PDU session reference assigned by the V-SMF during the Create service operation. |
| ismfPduSessionUri | Uri | PDU session reference assigned by the I-SMF during the Create service operation. |

##### 6.1.3.7.3	Resource Standard Methods

###### 6.1.3.7.3.1	POST

This method sends a status notification to the NF Service Consumer.

This method shall support the URI query parameters specified in table 6.1.3.7.3.1-1.

Table 6.1.3.7.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.7.3.1-2 and the response data structures and response codes specified in table 6.1.3.7.3.1-3.

Table 6.1.3.7.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| StatusNotification | M | 1 | Representation of the status notification. |

Table 6.1.3.7.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful notification of status change |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing a different URI, or the same URI if a request is redirected to the same target resource via a different SCP. In the former case, the URI shall be an alternative URI of the resource located on an alternative service instance within the same SMF or SMF (service) set. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing a different URI, or the same URI if a request is redirected to the same target resource via a different SCP. In the former case, the URI shall be an alternative URI of the resource located on an alternative service instance within the same SMF or SMF (service) set. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with a ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). NOTE 2: RedirectResponse may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.1.3.7.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same SMF or SMF (service) set. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target SMF (service) instance ID towards which the request is redirected |

Table 6.1.3.7.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same SMF or SMF (service) set. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target SMF (service) instance ID towards which the request is redirected |

##### 6.1.3.7.4	Resource Custom Operations

###### 6.1.3.7.4.1	Overview

Table 6.1.3.7.4.1-1: Custom operations

| Operation Name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| modify | {vsmfPduSessionUri}/modify or {ismfPduSessionUri}/modify | POST | Update service operation (initiated by H-SMF or SMF) |
| transfer-mt-data | {vsmfPduSessionUri}/transfer-mt-data or {ismfPduSessionUri}/transfer-mt-data | POST | Transfer MT Data service operation |

###### 6.1.3.7.4.2	Operation: modify

6.1.3.7.4.2.1	Description

6.1.3.7.4.2.2	Operation Definition

This custom operation modifies an individual PDU session resource in the V-SMF for a HR PDU session or in the I-SMF for a PDU session with an I-SMF.

This operation shall support the request data structures specified in table 6.1.3.7.4.2.2-1 and the response data structure and response codes specified in table 6.1.3.7.4.2.2-2.

Table 6.1.3.7.4.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| VsmfUpdateData | M | 1 | Representation of the updates to apply to the PDU session. |

Table 6.1.3.7.4.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| VsmfUpdatedData | M | 1 | 200 OK | This case represents a successful update of the PDU session, when the V-SMF or I-SMF needs to return information in the response. |
| n/a |  |  | 204 No Content | This case represents a successful update of the PDU session, when the V-SMF or I-SMF does not need to return information in the response. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing a different URI, or the same URI if a request is redirected to the same target resource via a different SCP. In the former case, the URI shall be an alternative URI of the resource located on an alternative service instance within the same SMF or SMF (service) set. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing a different URI, or the same URI if a request is redirected to the same target resource via a different SCP. In the former case, the URI shall be an alternative URI of the resource located on an alternative service instance within the same SMF or SMF (service) set. (NOTE 2) |
| VsmfUpdateError | M | 1 | 400 Bad Request | The "cause" attribute shall be set to one of the errors defined in Table 5.2.7.2-1 of 3GPP TS 29.500 [4]. |
| ProblemDetails | O | 0..1 | 400 Bad Request | This error shall only be returned by an SCP or a SEPP for errors they originate. As an exception, this error may also be returned by an SMF, with an empty payload body, for a protocol error other than those specified for the SMF PDUSession service logic (e.g. protocol error found by the HTTP stack). |
| VsmfUpdateError | M | 1 | 403 Forbidden | The "cause" attribute shall be set to one of the following application errors: - N1_SM_ERROR - UNABLE_TO_PAGE_UE - UE_NOT_RESPONDING - REJECTED_BY_UE - REJECTED_DUE_VPLMN_POLICY - HO_TAU_IN_PROGRESS - EBI_EXHAUSTED - EBI_REJECTED_LOCAL_POLICY, if the EBI allocation was rejected due to local policies at the AMF as specified in clause 4.11.1.4.1 of 3GPP TS 23.502 [3]. - EBI_REJECTED_NO_N26, if the EBI allocation was rejected when the AMF is in a serving PLMN that does not support 5GS-EPS interworking procedures with N26 interface as specified in clause 5.17.2.3.1 of 3GPP TS 23.501 [2]. See table 6.1.7.3-1 for the description of these errors. |
| ProblemDetails | O | 0..1 | 403 Forbidden | This error shall only be returned by an SCP or a SEPP for errors they originate. As an exception, this error may also be returned by an SMF, with an empty payload body, for a protocol error other than those specified for the SMF PDUSession service logic (e.g. protocol error found by the HTTP stack). |
| VsmfUpdateError | M | 1 | 404 Not Found | The "cause" attribute shall be set to one of the following application error: - CONTEXT_NOT_FOUND See table 6.1.7.3-1 for the description of these errors. |
| VsmfUpdateError | O | 0..1 | 409 Conflict | The "cause" attribute may be used to indicate one of the following application errors: - HIGHER_PRIORITY_REQUEST_ONGOING; - UE_IN_CM_IDLE_STATE See table 6.1.7.3-1 for the description of these errors. |
| VsmfUpdateError | M | 1 | 500 Internal Server Error | The "cause" attribute shall be set to one of the errors defined in Table 5.2.7.2-1 of 3GPP TS 29.500 [4]. |
| ProblemDetails | O | 0..1 | 500 Internal Server Error | This error shall only be returned by an SCP or a SEPP for errors they originate. As an exception, this error may also be returned by an SMF, with an empty payload body, for a general server error other than those specified for the SMF PDUSession service logic. |
| VsmfUpdateError | M | 1 | 503 Service Unavailable | The "cause" attribute shall be set to one of the errors defined in Table 5.2.7.2-1 of 3GPP TS 29.500 [4]. |
| ProblemDetails | O | 0..1 | 503 Service Unavailable | This error shall only be returned by an SCP or a SEPP for errors they originate. As an exception, this error may also be returned by an SMF, with an empty payload body, for a general server error other than those specified for the SMF PDUSession service logic. |
| VsmfUpdateError | M | 1 | 504 Gateway Timeout | The "cause" attribute shall be set to one of the following application errors: - PEER_NOT_RESPONDING - NETWORK_FAILURE See table 6.1.7.3-1 for the description of these errors. |
| ProblemDetails | O | 0..1 | 504 Gateway Timeout | This error shall only be returned by an SCP or a SEPP for errors they originate. |
| NOTE 1: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with a ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). NOTE 2: RedirectResponse may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.1.3.7.4.2.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same SMF or SMF (service) set. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target SMF (service) instance ID towards which the request is redirected |

Table 6.1.3.7.4.2.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same SMF or SMF (service) set. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target SMF (service) instance ID towards which the request is redirected |

###### 6.1.3.7.4.3	Operation: transfer-mt-data

6.1.3.7.4.3.1	Description

This custom operation enables to transfer mobile terminated data received from NEF, for a given PDU session, towards the V-SMF for HR roaming scenarios, or the I-SMF for a PDU session with an I-SMF.

6.1.3.7.4.3.2	Operation Definition

This operation shall support the request data structures specified in Table 6.1.3.7.4.3.2-1 and the response data structure and response codes specified in Table 6.1.3.7.4.3.2-2.

Table 6.1.3.7.4.3.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| TransferMtDataReqData | M | 1 | Representation of the payload of a Transfer MT Data Request |

Table 6.1.3.7.4.3.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful MT data transfer |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing a different URI, or the same URI if a request is redirected to the same target resource via a different SCP. In the former case, the URI shall be an alternative URI of the resource located on an alternative service instance within the same SMF or SMF (service) set. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing a different URI, or the same URI if a request is redirected to the same target resource via a different SCP. In the former case, the URI shall be an alternative URI of the resource located on an alternative service instance within the same SMF or SMF (service) set. (NOTE 2) |
| TransferMtDataError | M | 1 | 504 Gateway Timeout | The "cause" attribute may be used to indicate the following application errors: - UE_NOT_REACHABLE, if the UE is not reachable to deliver the mobile terminated data; Estimated Maximum Waiting Time shall be included if available; See table 6.1.7.3-1 for the description of these errors. |
| NOTE 1: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with a ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). NOTE 2: RedirectResponse may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.1.3.7.4.3.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same SMF or SMF (service) set. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target SMF (service) instance ID towards which the request is redirected |

Table 6.1.3.7.4.3.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same SMF or SMF (service) set. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target SMF (service) instance ID towards which the request is redirected |

### 6.1.4	Custom Operations without associated resources

None.

### 6.1.5	Notifications

#### 6.1.5.1	General

This clause specifies the notifications provided by the Nsmf_PDUSession service.

The delivery of notifications shall be supported as specified in clause 6.2 of 3GPP TS 29.500 [4] for Server-initiated communication.

Table 6.1.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| SM Context Status Notification | {smContextStatusUri} (NF Service Consumer provided callback reference) | POST | Notify SM Context Status |

#### 6.1.5.2	SM Context Status Notification

##### 6.1.5.2.1	Description

If the NF Service Consumer (e.g AMF) has provided the callback URI for getting notified about change of SM context status, the SMF shall notify the NF Service Consumer when the SM context status information is updated.

##### 6.1.5.2.2	Notification Definition

The POST method shall be used for SM context status notification and the URI shall be the callback reference provided by the NF Service Consumer during the subscription to this notification.

Callback URI: {smContextStatusUri}

Support of URI query parameters is specified in table 6.1.5.2.2-1.

Table 6.1.5.2.2-1: URI query parameters supported by the POST method

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Support of request data structures is specified in table 6.1.5.2.2-2, and support of response data structures and response codes is specified in table 6.1.5.2-3.

Table 6.1.5.2.2-2: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SmContextStatusNotification | M | 1 | Representation of the SM context status notification. |

Table 6.1.5.2.2-3: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful notification of the SM context status change |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The NF service consumer shall generate a Location header field containing a URI pointing to the endpoint of another NF service consumer to which the notification should be sent. If an SCP redirects the message to another SCP then the location header field shall contain the same URI or a different URI pointing to the endpoint of the NF service consumer to which the notification should be sent. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The NF service consumer shall generate a Location header field containing a URI pointing to the endpoint of another NF service consumer to which the notification should be sent. If an SCP redirects the message to another SCP then the location header field shall contain the same URI or a different URI pointing to the endpoint of the NF service consumer to which the notification should be sent. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with a ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). NOTE 2: RedirectResponse may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.1.5.2.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | A URI pointing to the endpoint of NF service consumer to which the notification should be sent |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the notification is redirected |

Table 6.1.5.2.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | A URI pointing to the endpoint of NF service consumer to which the notification should be sent |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the notification is redirected |

### 6.1.6	Data Model

#### 6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Nsmf service based interface protocol.

Table 6.1.6.1-1: Nsmf specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| SmContextCreateData | 6.1.6.2.2 | Data within Create SM Context Request |
| SmContextCreatedData | 6.1.6.2.3 | Data within Create SM Context Response |
| SmContextUpdateData | 6.1.6.2.4 | Data within Update SM Context Request |
| SmContextUpdatedData | 6.1.6.2.5 | Data within Update SM Context Response |
| SmContextReleaseData | 6.1.6.2.6 | Data within Release SM Context Request |
| SmContextRetrieveData | 6.1.6.2.7 | Data within Retrieve SM Context Request |
| SmContextStatusNotification | 6.1.6.2.8 | Data within Notify SM Context Status Request |
| PduSessionCreateData | 6.1.6.2.9 | Data within Create Request |
| PduSessionCreatedData | 6.1.6.2.10 | Data within Create Response |
| HsmfUpdateData | 6.1.6.2.11 | Data within Update Request towards H-SMF, or from I-SMF to SMF |
| HsmfUpdatedData | 6.1.6.2.12 | Data within Update Response from H-SMF, or from SMF to I-SMF |
| ReleaseData | 6.1.6.2.13 | Data within Release Request |
| HsmfUpdateError | 6.1.6.2.14 | Error within Update Response from H-SMF, or from SMF to I-SMF |
| VsmfUpdateData | 6.1.6.2.15 | Data within Update Request towards V-SMF, or from SMF to I-SMF |
| VsmfUpdatedData | 6.1.6.2.16 | Data within Update Response from V-SMF, or from I-SMF to SMF |
| StatusNotification | 6.1.6.2.17 | Data within Notify Status Request |
| QosFlowItem | 6.1.6.2.18 | Individual QoS flow |
| QosFlowSetupItem | 6.1.6.2.19 | Individual QoS flow to setup |
| QosFlowAddModifyRequestItem | 6.1.6.2.20 | Individual QoS flow requested to be created or modified |
| QosFlowReleaseRequestItem | 6.1.6.2.21 | Individual QoS flow requested to be released |
| QosFlowProfile | 6.1.6.2.22 | QoS flow profile |
| GbrQosFlowInformation | 6.1.6.2.23 | GBR QoS flow information |
| QosFlowNotifyItem | 6.1.6.2.24 | Notification related to a QoS flow |
| SmContextRetrievedData | 6.1.6.2.27 | Data within Retrieve SM Context Response |
| TunnelInfo | 6.1.6.2.28 | Tunnel Information |
| StatusInfo | 6.1.6.2.29 | Status of SM context or of PDU session |
| VsmfUpdateError | 6.1.6.2.30 | Error within Update Response from V-SMF, or from I-SMF to SMF |
| EpsPdnCnxInfo | 6.1.6.2.31 | EPS PDN Connection Information from H-SMF to V-SMF, or from SMF to I-SMF |
| EpsBearerInfo | 6.1.6.2.32 | EPS Bearer Information from H-SMF to V-SMF, or from SMF to I-SMF |
| PduSessionNotifyItem | 6.1.6.2.33 | Notification related to a PDU session |
| EbiArpMapping | 6.1.6.2.34 | EBI to ARP mapping |
| SmContextCreateError | 6.1.6.2.35 | Error within Create SM Context Response |
| SmContextUpdateError | 6.1.6.2.36 | Error within Update SM Context Response |
| PduSessionCreateError | 6.1.6.2.37 | Error within Create Response |
| MmeCapabilities | 6.1.6.2.38 | MME capabilities |
| SmContext | 6.1.6.2.39 | Complete SM Context |
| ExemptionInd | 6.1.6.2.40 | Exemption Indication |
| PsaInformation | 6.1.6.2.41 | PSA Information |
| DnaiInformation | 6.1.6.2.42 | DNAI Information |
| N4Information | 6.1.6.2.43 | N4 Information |
| IndirectDataForwardingTunnelInfo | 6.1.6.2.44 | Indirect Data Forwarding Tunnel Information |
| SmContextReleasedData | 6.1.6.2.45 | Data within Release SM Context Response |
| ReleasedData | 6.1.6.2.46 | Data within Release Response |
| SendMoDataReqData | 6.1.6.2.47 | Data within Send MO Data Request |
| CnAssistedRanPara | 6.1.6.2.48 | SMF derived CN assisted RAN parameters tuning |
| UlclBpInformation | 6.1.6.2.49 | UL CL or BP Information |
| TransferMoDataReqData | 6.1.6.2.50 | Data within Transfer MO Data Request |
| TransferMtDataReqData | 6.1.6.2.51 | Data within Transfer MT Data Request |
| TransferMtDataError | 6.1.6.2.52 | Transfer MT Data Error Response |
| TransferMtDataAddInfo | 6.1.6.2.53 | Transfer MT Data Error Response Additional Information |
| VplmnQos | 6.1.6.2.54 | VPLMN QoS |
| DdnFailureSubs | 6.1.6.2.55 | DDN Failure Subscription |
| RetrieveData | 6.1.6.2.56 | Data within Retrieve Request |
| RetrievedData | 6.1.6.2.57 | Data within Retrieve Response |
| SecurityResult | 6.1.6.2.58 | Security Result |
| UpSecurityInfo | 6.1.6.2.59 | User Plane Security Information |
| DdnFailureSubInfo | 6.1.6.2.60 | DDN Failure Subscription Information |
| AlternativeQosProfile | 6.1.6.2.61 | Alternative QoS Profile |
| ProblemDetailsAddInfo | 6.1.6.2.62 | Problem Details Additional Information |
| ExtProblemDetails | 6.1.6.2.63 | Extended Problem Details |
| QosMonitoringInfo | 6.1.6.2.64 | QoS Monitoring Information |
| IpAddress | 6.1.6.2.65 | IP Address |
| RedundantPduSessionInformation | 6.1.6.2.66 | Redundant PDU Session Information |
| QosFlowTunnel | 6.1.6.2.67 | Tunnel Information per QoS Flow |
| TargetDnaiInfo | 6.1.6.2.68 | Target DNAI Information |
| AfCoordinationInfo | 6.1.6.2.69 | AF Coordination Information |
| NotificationInfo | 6.1.6.2.70 | Notification Correlation ID and Notification URI provided by the NF service consumer |
| Teid | 6.1.6.3.2 | GTP Tunnel Endpoint Identifier |
| ProcedureTransactionId | 6.1.6.3.2 | Procedure Transaction Identifier |
| EpsPdnCnxContainer | 6.1.6.3.2 | UE EPS PDN Connection container from SMF to AMF |
| EpsBearerId | 6.1.6.3.2 | EPS Bearer Id |
| EpsBearerContainer | 6.1.6.3.2 | EPS Bearer container from SMF to AMF |
| EpsBearerContextStatus | 6.1.6.3.2 | EPS Bearer context status |
| DrbId | 6.1.6.3.2 | Data Radio Bearer Identifier |
| UpCnxState | 6.1.6.3.3 | User Plane Connection State |
| HoState | 6.1.6.3.4 | Handover State |
| RequestType | 6.1.6.3.5 | Request Type in Create (SM context) service operation. |
| RequestIndication | 6.1.6.3.6 | Request Indication in Update (SM context) service operation. |
| NotificationCause | 6.1.6.3.7 | Cause for generating a notification |
| Cause | 6.1.6.3.8 | Cause information |
| ResourceStatus | 6.1.6.3.9 | Status of SM context or PDU session resource |
| DnnSelectionMode | 6.1.6.3.10 | DNN Selection Mode |
| EpsInterworkingIndication | 6.1.6.3.11 | EPS Interworking Indication |
| N2SmInfoType | 6.1.6.3.12 | N2 SM Information Type |
| MaxIntegrityProtectedDataRate | 6.1.6.3.13 | Maximum Integrity Protected Data Rate |
| MaReleaseIndication | 6.1.6.3.14 | Multi-Access PDU session release Indication |
| SmContextType | 6.1.6.3.15 | Type of SM Context information |
| PsaIndication | 6.1.6.3.16 | Indication of whether a PSA is inserted or removed |
| N4MessageType | 6.1.6.3.17 | N4 Message Type |
| QosFlowAccessType | 6.1.6.3.18 | Access type associated with the QoS Flow |
| UnavailableAccessIndication | 6.1.6.3.19 | Indicates the access type of a MA PDU session that is unavailable |
| ProtectionResult | 6.1.6.3.20 | Protection Result of the security policy indicated as "preferred" |
| QosMonitoringReq | 6.1.6.3.21 | Indicates to measure UL, or DL, or both UL/DL delays, or to stop on-going measurements. |
| Rsn | 6.1.6.3.22 | Redundancy Sequence Number |
| SmfSelectionType | 6.1.6.3.23 | SMF Selection Type |

Table 6.1.6.1-2 specifies data types re-used by the Nsmf service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nsmf service based interface.

Table 6.1.6.1-2: Nsmf re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| Uint32 | 3GPP TS 29.571 [13] | Unsigned 32-bit integers |
| Ipv4Addr | 3GPP TS 29.571 [13] | IPv4 Address |
| Ipv6Prefix | 3GPP TS 29.571 [13] | IPv6 Prefix |
| Uri | 3GPP TS 29.571 [13] | Uniform Resource Identifier |
| Supi | 3GPP TS 29.571 [13] | Subscription Permanent Identifier |
| Pei | 3GPP TS 29.571 [13] | Permanent Equipment Identifier |
| Gpsi | 3GPP TS 29.571 [13] | General Public Subscription Identifier |
| AccessType | 3GPP TS 29.571 [13] | Access Type (3GPP or non-3GPP access) |
| SupportedFeatures | 3GPP TS 29.571 [13] | Supported features |
| Qfi | 3GPP TS 29.571 [13] | QoS Flow Identifier |
| PduSessionId | 3GPP TS 29.571 [13] | PDU Session Identifier |
| PduSessionType | 3GPP TS 29.571 [13] | PDU Session Type |
| Ambr | 3GPP TS 29.571 [13] | PDU Session Aggregate Maximum Bit Rate |
| 5Qi | 3GPP TS 29.571 [13] | 5G QoS Identifier |
| Arp | 3GPP TS 29.571 [13] | Allocation and Retention Priority |
| ReflectiveQoSAttribute | 3GPP TS 29.571 [13] | Reflective QoS Attribute |
| Dynamic5Qi | 3GPP TS 29.571 [13] | QoS characteristics for a 5QI that is neither standardized nor pre-configured. |
| NonDynamic5Qi | 3GPP TS 29.571 [13] | QoS characteristics that replace the default QoS characteristics for a standardized or pre-configured 5QI. |
| PacketLossRate | 3GPP TS 29.571 [13] | Packet Loss Rate |
| NotificationControl | 3GPP TS 29.571 [13] | Notification Control |
| Dnn | 3GPP TS 29.571 [13] | Data Network Name |
| Snssai | 3GPP TS 29.571 [13] | Single Network Slice Selection Assistance Information |
| NfInstanceId | 3GPP TS 29.571 [13] | NF Instance Identifier |
| UserLocation | 3GPP TS 29.571 [13] | User Location |
| TimeZone | 3GPP TS 29.571 [13] | Time Zone |
| ProblemDetails | 3GPP TS 29.571 [13] | Error description |
| UpSecurity | 3GPP TS 29.571 [13] | User Plane Security Policy Enforcement information |
| RefToBinaryData | 3GPP TS 29.571 [13] | Cross-Reference to binary data encoded within a binary body part in an HTTP multipart message. |
| Guami | 3GPP TS 29.571 [13] | Globally Unique AMF ID |
| BackupAmfInfo | 3GPP TS 29.571 [13] | Backup AMF Information |
| PresenceState | 3GPP TS 29.571 [13] | Indicates the UE presence in or out of a LADN service area |
| TraceData | 3GPP TS 29.571 [13] | Trace control and configuration parameters |
| PlmnId | 3GPP TS 29.571 [13] | PLMN Identity |
| RatType | 3GPP TS 29.571 [13] | RAT Type |
| NgApCause | 3GPP TS 29.571 [13] | NGAP Cause |
| 5GMmCause | 3GPP TS 29.571 [13] | 5G MM Cause |
| DurationSec | 3GPP TS 29.571 [13] | Duration in units of seconds |
| AdditionalQosFlowInfo | 3GPP TS 29.571 [13] | Additional QoS Flow Information |
| NfGroupId | 3GPP TS 29.571 [13] | Network Function Group Id |
| SecondaryRatUsageReport | 3GPP TS 29.571 [13] | Secondary RAT Usage Report |
| SecondaryRatUsageInfo | 3GPP TS 29.571 [13] | Secondary RAT Usage Information |
| Dnai | 3GPP TS 29.571 [13] | Data Network Access Identifier |
| PlmnIdNid | 3GPP TS 29.571 [13] | PLMN Identity and, for SNPN, Network Identity |
| SmallDataRateStatus | 3GPP TS 29.571 [13] | Small Data Rate Control Status |
| ApnRateStatus | 3GPP TS 29.571 [13] | APN Rate Control Status |
| StationaryIndication | 3GPP TS 29.571 [13] | Stationary Indication |
| ScheduledCommunicationTime | 3GPP TS 29.571 [13] | Scheduled Communication Time |
| ScheduledCommunicationType | 3GPP TS 29.571 [13] | Scheduled Communication Type |
| TrafficProfile | 3GPP TS 29.571 [13] | Traffic Profile |
| BatteryIndication | 3GPP TS 29.571 [13] | Battery Indication |
| NfSetId | 3GPP TS 29.571 [13] | NF Set Identifier |
| MoExpDataCounter | 3GPP TS 29.571 [13] | MO Exception Data Counter |
| DddTrafficDescriptor | 3GPP TS 29.571 [13] | Traffic Descriptor |
| NfServiceSetId | 3GPP TS 29.571 [13] | NF Service Set ID |
| RedirectResponse | 3GPP TS 29.571 [13] | Response body of the redirect response message |
| ServiceName | 3GPP TS 29.510 [19] | Service Name |
| WAgfInfo | 3GPP TS 29.510 [19] | Information about N3 terminations at the W-AGF |
| TngfInfo | 3GPP TS 29.510 [19] | Information about N3 terminations at the TNGF |
| TwifInfo | 3GPP TS 29.510 [19] | Information about N3 terminations at the TWIF |
| ChargingInformation | 3GPP TS 29.512 [30] | CHF addresses |
| NgRanTargetId | 3GPP TS 29.518 [20] | NG-RAN Target Id |
| SbiBindingLevel | 3GPP TS 29.518 [20] | SBI Binding Level |
| IpIndex | 3GPP TS 29.519 [38] | Information that identifies which IP pool or external server is used to allocate the IP address. |
| RoamingChargingProfile | 3GPP TS 32.291 [26] | Roaming Charging Profile |

#### 6.1.6.2	Structured data types

##### 6.1.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 6.1.6.2.2	Type: SmContextCreateData

Table 6.1.6.2.2-1: Definition of type SmContextCreateData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supi | Supi | C | 0..1 | This IE shall be present, except if the UE is emergency registered and UICCless. When present, it shall contain the subscriber permanent identify. |  |
| unauthenticatedSupi | boolean | C | 0..1 | This IE shall be present if the SUPI is present in the message but is not authenticated and is for an emergency registered UE. When present, it shall be set as follows: - true: unauthenticated SUPI; - false (default): authenticated SUPI. |  |
| pei | Pei | C | 0..1 | This IE shall be present if the UE is emergency registered and it is either UIClless or the SUPI is not authenticated. For all other cases, this IE shall be present if it is available. When present, it shall contain the permanent equipment identifier. |  |
| gpsi | Gpsi | C | 0..1 | This IE shall be present if it is available. When present, it shall contain the user's GPSI. |  |
| pduSessionId | PduSessionId | C | 0..1 | This IE shall be present, except during an EPS to 5GS Idle mode mobility or handover using the N26 interface. When present, it shall contain the PDU Session ID. |  |
| dnn | Dnn | C | 0..1 | This IE shall be present, except during an EPS to 5GS Idle mode mobility or handover using the N26 interface. When present, it shall contain the requested DNN; the DNN shall be the full DNN (i.e. with both the Network Identifier and Operator Identifier) for a HR PDU session, and it should be the full DNN in LBO and non-roaming scenarios. If the Operator Identifier is absent, the serving core network operator shall be assumed. |  |
| selectedDnn | Dnn | C | 0..1 | This IE shall be present, if another DNN other than the UE requested DNN is selected for this PDU session. When present, it shall contain the selected DNN. The DNN shall be the full DNN (i.e. with both the Network Identifier and Operator Identifier) for a HR PDU session, and it should be the full DNN in LBO and non-roaming scenarios. If the Operator Identifier is absent, the serving core network operator shall be assumed. |  |
| sNssai | Snssai | C | 0..1 | This IE shall be present during the PDU session establishment procedure. In this case, it shall contain the requested S-NSSAI for the serving PLMN. This corresponds to an S-NSSAI from the allowed NSSAI. This IE shall also be present during an EPS to 5GS idle mode mobility or handover with I-SMF/V-SMF involved using the N26 interface. In this case, it shall contain the S-NSSAI configured in the AMF for EPS interworking. |  |
| hplmnSnssai | Snssai | C | 0..1 | This IE shall be present for a roaming PDU session, except during an EPS to 5GS idle mode mobility or handover using the N26 interface. When present, it shall contain the requested S-NSSAI for the HPLMN. This corresponds to an S-NSSAI from the Mapping Of Allowed NSSAI corresponding to the SNSSAI value included in the sNssai IE. |  |
| servingNfId | NfInstanceId | M | 1 | This IE shall contain the identifier of the serving NF (e.g. serving AMF). |  |
| guami | Guami | C | 0..1 | This IE shall contain the serving AMF's GUAMI. It shall be included if the NF service consumer is an AMF. |  |
| serviceName | ServiceName | O | 0..1 | When present, this IE shall contain the name of the AMF service to which SM context status notifications are to be sent (see clause 6.5.2.2 of 3GPP TS 29.500 [4]). This IE may be included if the NF service consumer is an AMF. |  |
| servingNetwork | PlmnIdNid | M | 1 | This IE shall contain the serving core network operator PLMN ID and, for an SNPN, the NID that together with the PLMN ID identifies the SNPN. |  |
| requestType | RequestType | C | 0..1 | This IE shall be present if the Request type IE is received from the UE for a single access PDU session and if the request refers to an existing PDU session or an existing emergency PDU session. The requestType IE shall not be included for a MA-PDU session establishment request. It may be present otherwise. When present, it shall indicate whether the request refers to a new PDU session or emergency PDU session, or to an existing PDU session or emergency PDU session. For request sent from UE, this IE shall be set based on the Request type IE received (see clause 9.11.3.47 of 3GPP TS 24.501 [7]). |  |
| n1SmMsg | RefToBinaryData | C | 0..1 | This IE shall be present and reference the N1 SM Message binary data (see clause 6.1.6.4.2), except during an EPS to 5GS Idle mode mobility or handover using N26. |  |
| anType | AccessType | M | 1 | This IE shall indicate the Access Network Type to which the PDU session is to be associated. |  |
| additionalAnType | AccessType | C | 0..1 | This IE shall indicate the additional Access Network Type to which the PDU session is to be associated. This IE shall be present if a MA-PDU session is requested and the UE is registered over both 3GPP access and Non-3GPP access. | MAPDU |
| ratType | RatType | C | 0..1 | This IE shall be present and indicate the RAT Type used by the UE, if available. |  |
| presenceInLadn | PresenceState | C | 0..1 | This IE shall be present if the DNN corresponds to a LADN. When present, it shall be set to "IN" or "OUT" to indicate that the UE is in or out of the LADN service area. |  |
| ueLocation | UserLocation | C | 0..1 | This IE shall contain the UE location information (see clause 5.2.3.4), if it is available. (NOTE 1). |  |
| ueTimeZone | TimeZone | C | 0..1 | This IE shall contain the UE Time Zone, if it is available. |  |
| addUeLocation | UserLocation | O | 0..1 | Additional UE location. This IE may be present, if anType indicates a non-3GPP access and valid 3GPP access user location information is available. When present, it shall contain: - the last known 3GPP access user location (see clause 5.2.3.4); and - the timestamp, if available, indicating the UTC time when the addUeLocation information was acquired. (NOTE 1) |  |
| smContextStatusUri | Uri | M | 1 | This IE shall include the callback URI to receive notification of SM context status. |  |
| hSmfUri | Uri | C | 0..1 | This IE shall be present in HR roaming scenarios, including Indirect Communication with Delegated Discovery. When present, it shall contain the API URI of the Nsmf_PDUSession service of the selected H-SMF. The API URI shall be formatted as specified in clause 6.1.1. |  |
| hSmfId | NfInstanceId | O | 0..1 | This IE may be present when hSmfUri is present. If present, this IE shall carry the NF instance ID of the selected H-SMF. (NOTE 2) |  |
| smfUri | Uri | C | 0..1 | This IE shall be present for a PDU session with an I-SMF, including Indirect Communication with Delegated Discovery. When present, it shall contain the API URI of the Nsmf_PDUSession service of the selected SMF. The API URI shall be formatted as specified in clause 6.1.1. | DTSSA |
| smfId | NfInstanceId | O | 0..1 | This IE may be present when smfUri is present. If present, this IE shall carry the NF instance ID of the selected SMF. (NOTE 2) | DTSSA |
| oldPduSessionId | PduSessionId | C | 0..1 | This IE shall be present if this information is received from the UE. When present, it shall contain the old PDU Session ID received from the UE. See clauses 4.3.2.2.1 and 4.3.5.2 of 3GPP TS 23.502 [3]. |  |
| pduSessionsActivateList | array(PduSessionId) | C | 1..N | This IE shall be present, during an EPS to 5GS Idle mode mobility using the N26 interface, if the UE indicated PDU session(s) to be activated in the Registration Request. When present, it shall indicate all the PDU session(s) requested to be re-activated by the UE. |  |
| ueEpsPdnConnection | EpsPdnCnxContainer | C | 0..1 | This IE shall be present, during an EPS to 5GS Idle mode mobility or handover using the N26 interface. When present, it shall contain an MME/SGSN UE EPS PDN connection including the EPS bearer context(s). |  |
| hoState | HoState | C | 0..1 | This IE shall be present during an EPS to 5GS handover using N26 interface, to request the preparation of a handover of the PDU session. When present, it shall be set as specified in clause 5.2.2.2.3. |  |
| additionalHsmfUri | array(Uri) | O | 1..N | This IE may be present in HR roaming scenarios. When present, it shall contain an array of API URI of the Nsmf_PDUSession service of the additional H-SMFs discovered by the AMF for the given DNN, hplmnSnssai and for this PDU session. If provided, the V-SMF shall use these additional H-SMF(s) if the V-SMF is not able to receive any response from the H-SMF identified by hSmfUri. The API URI shall be formatted as specified in clause 6.1.1. |  |
| additionalHsmfId | array(NfInstanceId) | O | 1..N | This IE may be present when additionalHsmfUri is present. If present, this IE shall carry the NF instance ID(s) of H-SMF(s) as stated in additionalHsmfUri IE, in exactly the same order. (NOTE 2) |  |
| additionalSmfUri | array(Uri) | O | 1..N | This IE may be present for a PDU session with an I-SMF. When present, it shall contain an array of API URI of the Nsmf_PDUSession service of the additional SMFs discovered by the AMF for the given DNN, Snssai and for this PDU session. If provided, the I-SMF shall use these additional SMF(s) if the I-SMF is not able to receive any response from the SMF identified by smfUri. The API URI shall be formatted as specified in clause 6.1.1. | DTSSA |
| additionalSmfId | array(NfInstanceId) | O | 1..N | This IE may be present when additionalSmfUri is present. If present, this IE shall carry the NF instance ID(s) of SMF(s) as stated in additionalSmfUri IE, in exactly the same order. (NOTE 2) | DTSSA |
| pcfId | NfInstanceId | O | 0..1 | When present, this IE shall contain the identifier of: - the H-PCF selected by the AMF (for UE Policy), for a HR PDU session; or - the V-PCF selected by the AMF (for Access and Mobility Policy), for a PDU session in LBO roaming scenarios; or - the PCF selected by the AMF (for Access and Mobility Policy and/or UE Policy), for a PDU session in non-roaming scenarios. |  |
| pcfGroupId | NfGroupId | O | 0..1 | This IE may be present in non-roaming and HR roaming scenarios. When present, this IE shall contain the identity of the (home) PCF group serving the UE for Access and Mobility Policy and/or UE Policy. |  |
| pcfSetId | NfSetId | O | 0..1 | This IE may be present if pcfId IE is present. When present, this IE shall contain the NF Set ID of the PCF indicated by the pcfId IE. |  |
| nrfUri | Uri | O | 0..1 | This IE may be present to indicate the NRF to use for PCF selection within the same network slice instance. When present, the SMF shall use the NRF URI to select the PCF. |  |
| supportedFeatures | SupportedFeatures | C | 0..1 | This IE shall be present if at least one optional feature defined in clause 6.1.8 is supported. |  |
| selMode | DnnSelectionMode | C | 0..1 | This IE shall be present if it is available. When present, it shall be set to: - "VERIFIED", if the requested DNN provided by UE or the selected DNN provided by the network corresponds to an explicitly subscribed DNN; or - "UE_DNN_NOT_VERIFIED", if the requested DNN provided by UE corresponds to the usage of a wildcard subscription; or - "NW_DNN_NOT_VERIFIED", if the selected DNN provided by the network corresponds to the usage of a wildcard subscription. If both the requested DNN (i.e. dnn IE) and selected DNN (i.e. selected Dnn IE) are present, the selMode shall be related to the selected DNN. |  |
| backupAmfInfo | array(BackupAmfInfo) | C | 1..N | This IE shall be included if the NF service consumer is an AMF and the AMF supports the AMF management without UDSF for the following cases: - First interaction with SMF. - Modification of the BackupAmfInfo. |  |
| traceData | TraceData | C | 0..1 | This IE shall be included if trace is required to be activated (see 3GPP TS 32.422 [22]). |  |
| udmGroupId | NfGroupId | O | 0..1 | When present, it shall indicate the identity of the UDM group serving the UE. |  |
| routingIndicator | string | O | 0..1 | When present, it shall indicate the Routing Indicator of the UE. |  |
| hNwPubKeyId | integer | O | 0..1 | When present, it shall indicate the Home Network Public Key Identifier of the UE. (NOTE 3) |  |
| epsInterworkingInd | EpsInterworkingIndication | O | 0..1 | The AMF may provide the indication when a PGW-C+SMF is selected to serve the PDU Session. When present, this IE shall indicate whether the PDU session may possibly be moved to EPS and whether N26 interface to be used during EPS interworking procedures. The AMF may derive the value of the indication from different sources, like UE 5GMM capabilities (e.g. "S1 mode supported"), UE subscription data (e.g. "Core Network Type Restriction to EPC" and "Interworking with EPS Indication" for the DNN) and configurations. |  |
| indirectForwardingFlag | boolean | C | 0..1 | The AMF shall include this indication during N26 based Handover procedure from EPS to 5GS (see 3GPP TS 23.502 [3], clause 4.11.1.2.2), to inform the SMF of the applicability or non-applicability of indirect data forwarding. When present, it shall be set as follows: - True: indirect data forwarding is applicable - False: indirect data forwarding is not applicable |  |
| directForwardingFlag | boolean | C | 0..1 | The AMF shall include this indication during N26 based Handover procedure from EPS to 5GS (see 3GPP TS 23.502 [3], clause 4.11.1.2.2), to inform the SMF of the applicability or non-applicability of direct data forwarding. When present, it shall be set as follows: - True: direct data forwarding is applicable - False: direct data forwarding is not applicable |  |
| targetId | NgRanTargetId | C | 0..1 | This IE shall be present in the following cases: - during an EPS to 5GS handover preparation using the N26 interface, when the hoState IE is set to the value "PREPARING"; - during N2 based handover procedure with I-SMF or V-SMF insertion/change/removal, when hostate IE is set to the value "PREPARING". When present, it shall contain the Target ID identifying the target RAN Node ID and TAI. In case of EPS to 5GS handover, the TAI is received in the Forward Relocation Request from the Source MME. |  |
| epsBearerCtxStatus | EpsBearerContextStatus | C | 0..1 | This IE shall be present during an EPS to 5GS idle mode mobility using the N26 interface, if received in the Registration Request from the UE. When present, it shall be set to the value received from the UE. |  |
| cpCiotEnabled | boolean | C | 0..1 | This IE shall be present with the value "True", if - the NF service consumer (e.g. the AMF) has verified that the CIOT feature is supported by the SMF (and for a home-routed session, that it is also supported by the H-SMF); and - Control Plane CIoT 5GS Optimisation is enabled for the PDU session (see 3GPP TS 23.502 [3], clauses 4.3.2.2.1 and 4.3.2.2.2). When present, it shall be set as follows: - True: Control Plane CIoT 5GS Optimisation is enabled. - False (default): Control Plane CIoT 5GS Optimisation is not enabled. | CIOT |
| cpOnlyInd | boolean | C | 0..1 | This IE shall be present with the value "True", if the PDU session shall only use Control Plane CIoT 5GS Optimisation (see clause 5.31.4.1 of 3GPP TS 23.501 [2]). When present, it shall be set as follows: - True: the PDU session shall only use Control Plane CIoT 5GS Optimisation - False (default): the PDU session is not constrained to only use Control Plane CIoT 5GS Optimisation. | CIOT |
| invokeNef | boolean | C | 0..1 | This IE shall be present with the value "True", if Control Plane CIoT 5GS Optimisation is enabled and data delivery via NEF is selected for the PDU session (see 3GPP TS 23.502 [3], clause 4.3.2.2.2). When present, it shall be set as follows: - True: Data delivery via NEF is selected. - False (default): Data delivery via NEF is not selected. | CIOT |
| maRequestInd | boolean | C | 0..1 | This IE shall be present if a MA-PDU session is requested to be established. When present, it shall be set as follows: - True: a MA-PDU session is requested - False (default): a MA-PDU session is not requested | MAPDU |
| maNwUpgradeInd | boolean | C | 0..1 | This IE shall only be present if the PDU session is allowed to be upgraded to MA PDU session (see clause 4.22.3 of 3GPP TS 23.502 [3]). When present, it shall be set as follows: - True: the PDU session is allowed to be upgraded to MA PDU session - False (default): the PDU session is not allowed to be upgraded to MA PDU session When maRequestInd is present and set to "true", this IE shall not be present. | MAPDU |
| n2SmInfo | RefToBinaryData | C | 0..1 | This IE shall be present if N2 SM Information needs to be sent to the I-SMF. | DTSSA |
| n2SmInfoType | N2SmInfoType | C | 0..1 | This IE shall be present if "n2SmInfo" attribute is present. When present, this IE shall indicate the NG AP IE type for the NG AP SMF related IE container carried in "n2SmInfo" attribute. | DTSSA |
| n2SmInfoExt1 | RefToBinaryData | C | 0..1 | This IE shall be present if more than one N2 SM Information has been received from the AN. When present, this IE shall reference the N2 SM Information binary data (see clause 6.1.6.4.3). | DTSSA |
| n2SmInfoTypeExt1 | N2SmInfoType | C | 0..1 | This IE shall be present if "n2SmInfoExt1" attribute is present. When present, this IE shall indicate the NG AP IE type for the NG AP SMF related IE container carried in "n2SmInfoExt1" attribute. | DTSSA |
| smContextRef | Uri | C | 0..1 | This IE shall be present during an I-SMF or V-SMF insertion if available and during an I-SMF or V-SMF change or removal. When present, this IE shall contain the URI of the SM Context resource in the SMF or of the SM context resource in the source I-SMF or V-SMF during an I-SMF or V-SMF insertion or during an I-SMF or V-SMF change/removal respectively. The URI shall be an absolute URI, including apiRoot (see clause 6.1.3.3.2). | DTSSA |
| smContextSmfId | NfInstanceId | O | 0..1 | This IE may be present if smContextRef is present. When present, this IE shall carry the NF instance ID of the SMF which hosts the SM Context resource identified by smContextRef IE. (NOTE 2) | DTSSA |
| smContextSmfSetId | NfSetId | C | 0..1 | This IE shall be present, if available. When present, this IE shall contain the NF Set ID of the old V-SMF or the old I-SMF or the SMF as identified by the smContextSmfId. | DTSSA |
| smContextSmfServiceSetId | NfServiceSetId | C | 0..1 | This IE shall be present, if available. When present, this IE shall contain the NF Service Set ID of the PDUSession service instance (for this SmContext) in the old V-SMF or the old I-SMF or the SMF. | DTSSA |
| smContextSmfBinding | SbiBindingLevel | C | 0..1 | This IE shall be present, if available. When present, this IE shall contain the SBI binding level of the SM context resource. | DTSSA |
| upCnxState | UpCnxState | C | 0..1 | This IE shall be present to request the activation of the user plane connection of the PDU session, in the following cases: - during Service Request procedure with an I-SMF insertion / change / removal, or with a V-SMF change (see clause 5.2.2.2.6); - during Registration procedure with an I-SMF insertion / change / removal, or with a V-SMF insertion / change / removal (see clause 5.2.2.2.7), if this PDU session is requested to be activated by the UE. | DTSSA |
| smallDataRateStatus | SmallDataRateStatus | C | 0..1 | This IE shall be present if the small data rate control status is available in AMF, see clause 5.31.14.3 of 3GPP TS 23.501 [2] and clause 4.3.2.2.1 of 3GPP TS 23.502 [3]. | CIOT |
| apnRateStatus | ApnRateStatus | C | 0..1 | This IE shall be present if the APN rate control status is available in AMF, see clause 4.7.7.3 in 3GPP TS 23.401 [33] and clause 5.2.8.2.5 in 3GPP TS 23.502 [3]. | CIOT |
| extendedNasSmTimerInd | boolean | C | 0..1 | This IE shall be present with the value "True" if the UE supports CE mode B and use of CE mode B is not restricted according to the Enhanced Coverage Restriction information in the UE context in the AMF. When present, it shall indicate whether extended NAS SM timers shall be used for the UE as specified in 3GPP TS 24.501 [7], as follows: - True: extended NAS SM timers shall be used - False (default): normal NAS SM timers shall be used. | CIOT |
| dlDataWaitingInd | boolean | C | 0..1 | This IE shall be present during an EPS to 5GS Idle mode mobility using N26 interface with data forwarding (see clause 4.11.1.3.3A of 3GPP TS 23.502 [3]), if the same indication is received from the MME in the Context Response message. When present, it shall be set as follows: - true: DL data needs to be sent to the UE; - false (default): no DL data needs to be sent to the UE. | CIOT |
| ddnFailureSubs | DdnFailureSubs | C | 0..1 | This IE shall be present to subscribe the notification of the DDN Failure if the Availability after DDN failure event is subscribed by the UDM, see clause 4.15.3.2.7 of 3GPP TS 23.502 [3]. | CIOT |
| smfTransferInd | boolean | C | 0..1 | This IE shall be present during an SMF Context Transfer procedure, LBO or no Roaming, no I-SMF. When present, it shall be set as follows: - True: SMF Context Transfer - False (default): Not an SMF Context Transfer | CTXTR |
| oldSmfId | NfInstanceId | C | 0..1 | This IE shall be present if smfTransferInd is set to true. When present, it shall indicate old SMF instance identifier. | CTXTR |
| oldSmContextRef | Uri | C | 0..1 | This IE shall be present if smfTransferInd is set to true. When present, this IE shall contain the identifier of the SM Context resource in the old SMF. This IE shall also be present, without smfTransferInd set, if this information was received earlier in Notify SM Context Status, see clause 4.3.5.2 of 3GPP TS 23.502 [3]. | CTXTR EEDGE |
| wAgfInfo | WAgfInfo | C | 0..1 | This IE shall be present, if received from the W-AGF. When present, it shall contain information about the N3 terminations of the W-AGF. The SMF may use this information when selecting the UPF. |  |
| tngfInfo | tngfInfo | C | 0..1 | This IE shall be present, if received from the TNGF. When present, it shall contain information about the N3 terminations of the TNGF. The SMF may use this information when selecting the UPF. |  |
| twifInfo | twifInfo | C | 0..1 | This IE shall be present, if received from the TWIF. When present, it shall contain information about the N3 terminations of the TWIF. The SMF may use this information when selecting the UPF. |  |
| ranUnchangedInd | boolean | C | 0..1 | This IE shall be present if the NG-RAN is not changed in case of I-SMF/V-SMF change or insertion during CM-CONNECTED registration procedure after EPS to 5GS handover (see clause 5.2.2.2.7) or I-SMF selection per DNAI (see clause 5.2.2.2.12). When present, it shall be set as follows: - true: NG-RAN is not changed; - false: NG-RAN is changed. | DTSSA |
| samePcfSelectionInd | boolean | C | 0..1 | This IE shall be present, if the AMF received the PCF Selection Assistance info from the UDM indicating that the same PCF is required. (NOTE 4) When present, it shall be set as follows: - True: the SMF is indicated to select the same PCF instance for the PDU session. - False (default): the SMF is not indicated to select the same PCF instance for the PDU session. |  |
| targetDnai | Dnai | C | 0..1 | This IE shall be present, if this information was received earlier in Notify SM Context Status. I-SMF or SMF shall use this information for I-UPF / L-PSA / PSA selection, see clauses 4.3.5.1, 4.3.5.2 or 4.23.5.4 of 3GPP TS 23.502 [3]. | EEDGE |
| nrfManagementUri | Uri | C | 0..1 | If included, this IE shall contain the API URI of the NFManagement Service (see clause 6.1.1 of 3GPP TS 29.510 [19]) of the NRF or hNRF. It shall be present during the PDU session establishment procedure with an I-SMF/V-SMF or mobility procedure with I-SMF/V-SMF insertion/change, if it is returned from the NSSF or hNSSF (see clause 6.1.6.2.7 of 3GPP TS 29.531 [40]). |  |
| nrfDiscoveryUri | Uri | C | 0..1 | If included, this IE shall contain the API URI of the NFDiscovery Service (see clause 6.2.1 of 3GPP TS 29.510 [19]) of the NRF or hNRF. It shall be present during the PDU session establishment procedure with an I-SMF/V-SMF or mobility procedure with I-SMF/V-SMF insertion/change, if it is returned from the NSSF or hNSSF (see clause 6.1.6.2.7 of 3GPP TS 29.531 [40]). |  |
| nrfAccessTokenUri | Uri | C | 0..1 | If included, this IE shall contain the API URI of the Access Token Service (see clause 6.3.2 of 3GPP TS 29.510 [19]) of the NRF or hNRF. It shall be present during the PDU session establishment procedure with an I-SMF/V-SMF or mobility procedure with I-SMF/V-SMF insertion/change, if it is returned from the NSSF or hNSSF (see clause 6.1.6.2.7 of 3GPP TS 29.531 [40]). |  |
| ivSmfRestoration | boolean | C | 0..1 | This IE shall be present if the I-SMF/V-SMF restoration procedure is triggered, as specified in clause 6.7 of 3GPP TS 23.527 [24]. When present, it shall be set as follows: - true: I-SMF/V-SMF restoration procedure is required; - false (default): I-SMF/V-SMF restoration procedure is not required. | IVRES |
| smfBindingInfo | string | C | 0..1 | This IE shall be present, if available. When present, this IE shall contain the Binding indications of the SM context resource and shall be set to the value of the 3gpp-Sbi-Binding header defined in clause 5.2.3.2.6 of 3GPP TS 29.500 [4], without the header name. | DTSSA |
| NOTE 1: In shared networks, when the message is sent from the VPLMN to the HPLMN, the PLMN ID that is communicated in this IE shall be that of the selected Core Network Operator. In shared networks, when the AMF and SMF pertain to the same PLMN, the Primary PLMN ID shall be communicated in the ECGI or NCGI to the SMF. The Core Network Operator PLMN ID shall be communicated in the TAI and the Serving Network. NOTE 2: If the SMF is aware that Oauth is enabled for the indicated next hop SMF, e.g. received a "401 Unauthorized" response code from next hop SMF, the SMF shall use the NF instance Identifier to acquire the access token for the Nsmf_PduSession service on the indicated SMF. NOTE 3: If present, this attribute shall be used together with routingIndicator. This attribute is only used by the HPLMN in roaming scenarios. NOTE 4: If present, this attribute shall be used together with the PCF ID received from the AMF for selecting the same PCF instance for the PDU session. |  |  |  |  |  |

##### 6.1.6.2.3	Type: SmContextCreatedData

Table 6.1.6.2.3-1: Definition of type SmContextCreatedData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| hsmfUri | Uri | C | 0..1 | This IE shall be present in HR roaming scenarios if the additionalHsmfUri IE was received in the request and the V-SMF established the PDU session towards an alternative SMF listed in the additionalHsmfUri IE. When present, it shall contain the API URI of the H-SMF towards which the PDU session was established. The API URI shall be formatted as specified in clause 6.1.1. |  |
| smfUri | Uri | C | 0..1 | This IE shall be present for a PDU session with an I-SMF, if the additionalSmfUri IE was received in the request and the I-SMF established the PDU session towards an alternative SMF listed in the additionalSmfUri IE. When present, it shall contain the API URI of the SMF towards which the PDU session was established. The API URI shall be formatted as specified in clause 6.1.1. | DTSSA |
| pduSessionId | PduSessionId | C | 0..1 | This IE shall be present, during an EPS to 5GS Idle mode mobility or handover using the N26 interface. When present, it shall be set to the PDU Session ID. |  |
| sNssai | Snssai | C | 0..1 | This IE shall be present during an EPS to 5GS Idle mode mobility or handover using the N26 interface. When present, it shall contain the S-NSSAI assigned to the PDU session. In Home-Routed roaming case, this IE shall contain the S-NSSAI for home PLMN. |  |
| upCnxState | UpCnxState | C | 0..1 | This IE shall be present if the SMF was requested to activate the user plane connection of the PDU session in the corresponding request. When present, it shall be set as specified in clauses 5.2.2.2.2, 5.2.2.2.6 or 5.2.2.2.7. |  |
| n2SmInfo | RefToBinaryData | C | 0..1 | This IE shall be present if N2 SM Information needs to be sent to the AN. |  |
| n2SmInfoType | N2SmInfoType | C | 0..1 | This IE shall be present if "n2SmInfo" attribute is present. When present, this IE shall indicate the NG AP IE type for the NG AP SMF related IE container carried in "n2SmInfo" attribute. |  |
| allocatedEbiList | array(EbiArpMapping) | C | 1..N | This IE shall be present if the consumer NF is an AMF and Inter-system mobility happens. When present, it shall contain an array of EBI to ARP mappings currently allocated to the PDU session. |  |
| hoState | HoState | C | 0..1 | This IE shall be present if the SMF was requested to prepare an EPS to 5GS handover for the PDU session in the corresponding request. When present, it shall be set as specified in clause 5.2.2.2.3. |  |
| gpsi | Gpsi | C | 0..1 | This IE shall be present if no GPSI IE is provided in the request, e.g. for a PDU session moved from another access or another system, and the SMF knows that a GPSI is already associated with the PDU session (or a GPSI is received from h-SMF for a HR PDU session). When present, it shall contain the user's GPSI associated with the PDU session. |  |
| smfServiceInstanceId | string | O | 0..1 | When present, this IE shall contain the serviceInstanceId of the SMF PDUSession service instance serving the SM Context, i.e. of: - the I-SMF, for a PDU session with I-SMF; - the V-SMF, for a HR PDU session; or - the SMF, for a non-roaming or an LBO roaming PDU session without I-SMF. This IE may be used by the AMF to identify PDU session contexts affected by a failure or restart of the SMF service instance (see clause 6.2 of 3GPP TS 23.527 [24]). |  |
| recoveryTime | DateTime | O | 0..1 | Timestamp when the SMF service instance serving the PDU session was (re)started (see clause 6.3 of 3GPP TS 23.527 [24]). |  |
| supportedFeatures | SupportedFeatures | C | 0..1 | This IE shall be present if at least one optional feature defined in clause 6.1.8 is supported. |  |
| selectedSmfId | NfInstanceId | C | 0..1 | This IE shall be present if a new (h)SMF is selected e.g. by the new I/V-SMF, or a SCP between the new I/V-SMF and the (h)SMF. (NOTE) When present, it shall contain the selected SMF NF Instance Id. | DTSSA |
| selectedOldSmfId | NfInstanceId | C | 0..1 | This IE shall be present if another old I/V-SMF(as alternative to the old I/V-SMF) is selected, e.g. by the new I/V-SMF, anchor SMF or a SCP between the new I/V-SMF and the old I/V-SMF. (NOTE) When present, it shall contain the selected old I/V-SMF NF Instance Id. | DTSSA |
| NOTE: During an SmContext Creation procedure, e.g. for I-SMF insertion or I-SMF change procedure, when the new I/V-SMF attempts to contact the old I/V-SMF or (h)SMF by invoking Nsmf_PDUSession_Context Request, if a new (h)SMF and/or another old I/V-SMF has been re-selected (since the old I/V-SMF or the (h)SMF is not reachable) by the new I-/V-SMF or a SCP, the selected old I-/V-SMF and/or (h)SMF shall be returned to the AMF, in order to perform potential subsequent operations on the SMF hosting the resource, e.g. to release the SM Context on old I-/V-SMF, or to create SM Context on SMF when the I/V-SMF needs to be removed. |  |  |  |  |  |

##### 6.1.6.2.4	Type: SmContextUpdateData

Table 6.1.6.2.4-1: Definition of type SmContextUpdateData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| pei | Pei | C | 0..1 | This IE shall be present if it is available and has not been provided earlier to the SMF. When present, this IE shall contain the permanent equipment identifier. |  |
| servingNfId | NfInstanceId | C | 0..1 | This IE shall be present upon inter-AMF change or mobility, or upon a N2 handover execution with AMF change. When present, it shall contain the identifier of the serving NF (e.g. AMF). |  |
| smContextStatusUri | Uri | C | 0..1 | This IE shall be present if the servingNfId IE is present. It may be present otherwise. When present, this IE shall include the callback URI to receive notification of SM context status. |  |
| guami | Guami | C | 0..1 | This IE shall be present if the servingNfId of AMF is present. When present, it shall contain the serving AMF's GUAMI. |  |
| servingNetwork | PlmnIdNid | C | 0..1 | This IE shall be present if the servingNfId IE is present. When present, it shall contain the serving core network operator PLMN ID and, for an SNPN, the NID that together with the PLMN ID identifies the SNPN. |  |
| backupAmfInfo | array(BackupAmfInfo) | C | 1..N | This IE shall be included for the modification of the BackupAmfInfo if the NF service consumer is an AMF and the AMF supports the AMF management without UDSF. For deleting the backupAmfInfo, it shall contain the Null value. |  |
| anType | AccessType | C | 0..1 | This IE shall be present upon a change of the Access Network Type associated to the PDU session, e.g. during a handover of the PDU session between 3GPP access and untrusted non-3GPP access (see clause 5.2.2.3.5.2). When present, this IE shall indicate the Access Network Type to which the PDU session is to be associated. |  |
| additionalAnType | AccessType | C | 0..1 | This IE shall indicate the additional Access Network Type to which the PDU session is to be associated. This IE shall be present when the UE requests to establish resources for MA PDU session over the other access. | MAPDU |
| anTypeToReactivate | AccessType | C | 0..1 | This IE shall indicate the Access Network Type for which the UP connection is requested to be re-activated, for a MA PDU session. | MAPDU |
| ratType | RatType | C | 0..1 | This IE shall be present and indicate the RAT Type used by the UE, if available, upon a change of RAT Type. |  |
| presenceInLadn | PresenceState | C | 0..1 | This IE shall be present during a Service Request procedure (see clause 5.2.2.3.2.2) ), an Xn handover (see clause 5.2.2.3.3) or a N2 handover execution (see clause 5.2.2.3.4.3), if the DNN of the PDU session corresponds to a LADN. When present, it shall be set to "IN" or "OUT" to indicate that the UE is in or out of the LADN service area. |  |
| ueLocation | UserLocation | C | 0..1 | This IE shall be present if it is available and if it needs to be reported to the SMF (e.g. the user location has changed or the user plane of the PDU session is deactivated). When present, this IE shall contain: - the UE location information (see clause 5.2.3.4); and - the timestamp, if available, indicating the UTC time when the UeLocation information was acquired. (NOTE 1) |  |
| ueTimeZone | TimeZone | C | 0..1 | This IE shall be present if it is available, the UE Time Zone has changed and needs to be reported to the SMF. When present, this IE shall contain the UE Time Zone. |  |
| addUeLocation | UserLocation | O | 0..1 | Additional UE location. This IE may be present, if anType indicates a non-3GPP access and a valid 3GPP access user location information is available. When present, it shall contain: - the last known 3GPP access user location (see clause 5.2.3.4); and - the timestamp, if available, indicating the UTC time when the addUeLocation information was acquired. (NOTE 1) |  |
| upCnxState | UpCnxState | C | 0..1 | This IE shall be present to request the activation or the deactivation of the user plane connection of the PDU session. When present, it shall be set as specified in clauses 5.2.2.3.2, 5.2.2.3.15 and 5.2.2.3.16. |  |
| hoState | HoState | C | 0..1 | This IE shall be present to request the preparation, execution or cancellation of a handover of the PDU session. When present, it shall be set as specified in clause 5.2.2.3.4. |  |
| toBeSwitched | boolean | C | 0..1 | This IE shall be present during an Xn Handover (see clause 5.2.2.3.3) to request to switch the PDU session to a new downlink N3 tunnel endpoint. When present, it shall be set as follows: - true: request to switch to the PDU session. - false (default): no request to switch the PDU session. |  |
| failedToBeSwitched | boolean | C | 0..1 | This IE shall be present during an Xn Handover (see clause 5.2.2.3.3) if the PDU session failed to be setup in the target RAN. When present, it shall be to true to indicate that the PDU session failed to be setup in the target RAN. |  |
| n1SmMsg | RefToBinaryData | C | 0..1 | This IE shall be present if N1 SM Information has been received from the UE. When present, this IE shall reference the N1 SM Message binary data (see clause 6.1.6.4.2). |  |
| n2SmInfo | RefToBinaryData | C | 0..1 | This IE shall be present if N2 SM Information has been received from the AN. When present, this IE shall reference the N2 SM Information binary data (see clause 6.1.6.4.3). |  |
| n2SmInfoType | N2SmInfoType | C | 0..1 | This IE shall be present if "n2SmInfo" attribute is present. When present, this IE shall indicate the NG AP IE type for the NG AP SMF related IE container carried in "n2SmInfo" attribute. |  |
| targetId | NgRanTargetId | C | 0..1 | This IE shall be present during a N2 handover preparation, when the hoState IE is set to the value "PREPARING". When present, it shall contain the Target ID identifying the target RAN Node ID and TAI received in the Handover Required from the Source RAN. |  |
| targetServingNfId | NfInstanceId | C | 0..1 | This IE shall be present during a N2 handover preparation with AMF change, when the hoState IE is set to the value "PREPARING". When present, it shall contain the identifier of the target serving NF (e.g. AMF). |  |
| dataForwarding | boolean | C | 0..1 | This IE shall be present and set as specified in clause 5.2.2.3.9 during a 5GS to EPS handover, or as specified in 5.2.2.3.13 during a N2 based handover with I-SMF insertion/change/removal. When present, it shall be set as follows: - true: setup the direct or indirect data forwarding tunnels; - false (default): data forwarding tunnels are not required to be setup (see clause 5.2.2.3.9). |  |
| n9ForwardingTunnel | TunnelInfo | C | 0..1 | This IE shall be present in the following case: - UE triggered Service Request with I-SMF change/removal, if requesting to forward buffered downlink data packets at I-UPF (See clause 4.23.4 of 3GPP TS 23.502 [3]). When present, it shall carry the N9 forwarding tunnel info of I-UPF. | DTSSA |
| n9DlForwardingTnlList | array (IndirectDataForwardingTunnelInfo) | C | 1..N | This IE shall be present in the following case: - N2 based handover with I-SMF insertion/change/removal, if downlink indirect data forwarding tunnels are requested to be established between target I-UPF and source I-UPF / source UPF (see clause 4.23.7 and 4.23.11 of 3GPP TS 23.502 [3]). When present, it shall carry the list of N9 downlink indirect data forwarding tunnel(s) info of I-UPF. | DTSSA |
| n9UlForwardingTnlList | array (IndirectDataForwardingTunnelInfo) | C | 1..N | This IE shall be present in the following case: - N2 based handover with I-SMF insertion/change/removal, if uplink indirect data forwarding tunnels are requested to be established between target I-UPF and source I-UPF / source UPF (see clause 4.23.7 and 4.23.11 of 3GPP TS 23.502 [3]). When present, it shall carry the list of N9 uplink indirect data forwarding tunnel(s) info of I-UPF. | DTSSA |
| epsBearerSetup | array(EpsBearerContainer) | C | 0..N | This IE shall be present during a 5GS to EPS handover using the N26 interface. When present, it shall include the EPS bearer context(s) successfully setup in EPS. The array shall be empty if no resource was successfully allocated in EPS for any PDU session. |  |
| revokeEbiList | array(EpsBearerId) | C | 1..N | This IE shall be present to request the SMF to revoke some EBIs (see clause 4.11.1.4.1 of 3GPP TS 23.502 [3]). When present, it shall contain the EBIs to revoke. |  |
| release | boolean | C | 0..1 | This IE shall be used to indicate a network initiated PDU session release is requested. This IE shall be present and set as specified in clause 5.2.2.3.10 during P-CSCF restoration procedure, in clause 5.2.2.3.11 during AMF requested PDU Session Release due to duplicated PDU Session Id, in clause 5.2.2.3.12 during AMF requested PDU Session Release due to slice not available, and in clause 5.2.2.3.17 during AMF requested PDU Session Release due to Network Slice-Specific Authentication and Authorization failure or revocation. When present, it shall be set as follows: - true: PDU session release is required; - false (default): PDU session release is not required. |  |
| cause | Cause | O | 0..1 | When present, this IE shall indicate the cause for the requested modification, e.g. the NF Service Consumer cause for requesting to deactivate the user plane connection of the PDU session. |  |
| ngApCause | NgApCause | C | 0..1 | This IE shall be present, if the information is available. When present, this IE shall indicate the cause for the requested modification, e.g. the NGAP cause for requesting to deactivate the user plane connection of the PDU session. |  |
| 5gMmCauseValue | 5GMmCause | C | 0..1 | This IE shall be included if the AMF received a 5GMM cause code from the UE during any network initiated PDU session modification or release procedure. (e.g 5GMM Status message in response to a Downlink NAS Transport message carrying 5GSM payload). |  |
| sNssai | Snssai | C | 0..1 | This IE shall be present and sent to the V-SMF, during an EPS to 5GS mobility registration using the N26 interface, if the S-NSSAI for the serving PLMN derived from the S-NSSAI of the home PLMN differs from the S-NSSAI provided in the Create SM Context Request. When present, it shall contain the S-NSSAI for the serving PLMN. |  |
| traceData | TraceData | C | 0..1 | This IE shall be included if trace is required to be activated, modified or deactivated (see 3GPP TS 32.422 [22]). For trace modification, it shall contain a complete replacement of trace data. For trace deactivation, it shall contain the Null value. |  |
| epsInterworkingInd | EpsInterworkingIndication | O | 0..1 | This IE may be present if the indication has been provided during the PDU session creation, and its value has changed after session creation or last update. When present, this IE shall indicate whether the PDU session may possibly be moved to EPS and whether N26 interface to be used during EPS interworking procedures. |  |
| anTypeCanBeChanged | boolean | C | 0..1 | This IE shall be present and set to true to indicate that the Access Network Type associated to the PDU session can be changed (see clause 5.2.2.3.2.4), during a Service Request procedure (see clause 4.2.3.2 of 3GPP TS 23.502 [3])), in response to paging or NAS notification indicating non-3GPP access, when the PDU Session for which the UE was paged or notified is in the List Of Allowed PDU Sessions provided by the UE, and the AMF received N2 SM Information only or N1 SM Container and N2 SM Information from the SMF in step 3a of clause 4.2.3.3 of 3GPP TS 23.502 [3]. When present, it shall be set as follows: - true: the access type of the PDU session can be changed. - false (default): the access type of the PDU session cannot be changed. |  |
| n2SmInfoExt1 | RefToBinaryData | C | 0..1 | This IE shall be present if more than one N2 SM Information has been received from the AN. When present, this IE shall reference the N2 SM Information binary data (see clause 6.1.6.4.3). |  |
| n2SmInfoTypeExt1 | N2SmInfoType | C | 0..1 | This IE shall be present if "n2SmInfoExt1" attribute is present. When present, this IE shall indicate the NG AP IE type for the NG AP SMF related IE container carried in "n2SmInfoExt1" attribute. |  |
| maReleaseInd | MaReleaseIndication | C | 0..1 | This IE shall be present if one access of a MA PDU session is requested to be released, in the following cases: - when UE/AMF initiates MA PDU session release over one access; or - when UE deregisters from one access. When present, it indicates the access to be released. | MAPDU |
| maNwUpgradeInd | boolean | C | 0..1 | This IE shall be present if the PDU session is allowed to be upgraded to MA PDU session (see clause 6.4.2.2 of 3GPP TS 24.501 [7]). When present, it shall be set as follows: - true: the PDU session is allowed to be upgraded to MA PDU session - false (default): the PDU session is not allowed to be upgraded to MA PDU session | MAPDU |
| maRequestInd | boolean | C | 0..1 | This IE shall be present if a MA-PDU session is requested to be established (see clause 4.22.6.3 of 3GPP TS 23.502 [3]). When present, it shall be set as follows: - true: a MA-PDU session is requested - false (default): a MA-PDU session is not requested | MAPDU |
| exemptionInd | ExemptionInd | C | 0..1 | This IE shall be present if the AMF has exempted the NAS message from a NAS SM congestion control activated in the AMF. |  |
| supportedFeatures | SupportedFeatures | C | 0..1 | This IE shall be present if the servingNfId or the targetServingNfId is present (i.e. during a change of AMF) and at least one optional feature defined in clause 6.1.8 is supported by the new AMF. If this IE is absent when the servingNfId or the targetServingNfId is present, the new serving AMF or the target AMF respectively shall be considered as not supporting any optional feature. |  |
| moExpDataCounter | MoExpDataCounter | C | 0..1 | This IE shall be included if the UE has accessed the network by using "MO exception data" RRC establishment cause and when the AMF decides to send a non-zero value to the SMF. (NOTE 2) When present, this IE shall contain the MO Exception Data Counter. | CIOT |
| extendedNasSmTimerInd | boolean | C | 0..1 | This IE shall be present if the UE supports CE mode B and use of CE mode B changes from restricted to unrestricted or vice versa in the Enhanced Coverage Restriction information in the UE context in the AMF. When present, it shall indicate whether extended NAS SM timers shall be used for the UE as specified in 3GPP TS 24.501 [7], as follows: - True: extended NAS SM timers shall be used - False: normal NAS SM timers shall be used. | CIOT |
| forwardingFTeid | Bytes | C | 0..1 | This IE shall be present during a 5GS to EPS Idle mode mobility using N26 interface with data forwarding (see clause 4.11.1.3.2A of 3GPP TS 23.502 [3]), if the Forwarding F-TEID IE is present in the Context Acknowledge message received from the MME. When present, it shall contain Base64-encoded characters, encoding the Forwarding F-TEID in the Context Acknowledge message, as specified in Figure 8.22-1 of 3GPP TS 29.274 [16] (starting from octet 1). | CIOT |
| forwardingBearerContexts | array(ForwardingBearerContainer) | C | 1..N | This IE shall be present during a 5GS to EPS Idle mode mobility using N26 interface with data forwarding (see clause 4.11.1.3.2A of 3GPP TS 23.502 [3]), if the Bearer Contexts IE is present in the Context Acknowledge message received from the MME. When present, it shall contain the Bearer Contexts in the Context Acknowledge message. | CIOT |
| ddnFailureSubs | DdnFailureSubs | C | 0..1 | This IE shall be present to subscribe or unsubscribe to the notification of the DDN Failure if the Availability after DDN failure event is subscribed/unsubscribed by the UDM, see clause 4.15.3.2.7 of 3GPP TS 23.502 [3]. This IE shall also be present if it is required to add, modify or remove DDN failure subscriptions. If it is present and the included dnnFailureSubsInd indicates notification of DDN failure is subscribed, the content of the received ddnFailureSubs shall overwrite any ddnFailureSubs received earlier. | CIOT |
| skipN2PduSessionResRelInd | boolean | O | 0..1 | This IE may be present when the release IE is present with value "true". When present, this IE shall indicate whether N2 message shall be skipped for the PDU session RAN resources release, if the UP connection is active: - true: N2 message shall be skipped. - false (default): N2 message shall not be skipped. |  |
| secondaryRatUsageDataReportContainer | array(SecondaryRatUsageDataReportContainer) | C | 1..N | The IE shall be present during an EPS to 5GS handover procedure, if one or more instance of Secondary RAT Usage Data Report IE(s) are present and applicable to the PDU session. When present, it shall contain Base64-encoded characters, encoding the Secondary RAT Usage Data Report in the Forward Relocation Complete Acknowledge message, as specified in Figure 8.132-1 of 3GPP TS 29.274 [16] (starting from octet 1). |  |
| NOTE 1: In shared networks, when the message is sent from the VPLMN to the HPLMN, the PLMN ID that is communicated in this IE shall be that of the selected Core Network Operator. In shared networks, when the AMF and SMF pertain to the same PLMN, the Primary PLMN ID shall be communicated in the ECGI or NCGI to the SMF. The Core Network Operator PLMN ID shall be communicated in the TAI and the Serving Network. NOTE 2: The AMF increments the MO Exception Data Counter when the UE establishes/resumes RRC with "MO Exception Data" RRC cause. The AMF may defer sending the moExpDataCounter attribute to the SMF based on local configuration. The AMF resets the MO Exception Data Counter when receiving successful response from the SMF. The SMF however keeps incrementing the counter locally. |  |  |  |  |  |

##### 6.1.6.2.5	Type: SmContextUpdatedData

Table 6.1.6.2.5-1: Definition of type SmContextUpdatedData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| upCnxState | UpCnxState | C | 0..1 | This IE shall be present if the SMF was requested to activate or deactivate the user plane connection of the PDU session in the corresponding request. When present, it shall be set as specified in clauses 5.2.2.3.2, 5.2.2.3.15 and 5.2.2.3.16. |  |
| hoState | HoState | C | 0..1 | This IE shall be present if the SMF was requested to prepare, execute or cancel a handover for the PDU session in the corresponding request. When present, it shall be set as specified in clause 5.2.2.3.4. |  |
| releaseEbiList | array(EpsBearerId) | C | 1..N | This IE shall be present if the SMF determines that some EBIs are not needed. When present, it shall contain the EBIs to be released. |  |
| allocatedEbiList | array(EbiArpMapping) | C | 1..N | This IE shall be present if the consumer NF is an AMF and Inter-system mobility happens. When present, it shall contain an array of EBI to ARP mappings currently allocated to the PDU session. |  |
| modifiedEbiList | array(EbiArpMapping) | C | 1..N | This IE shall be present if a PDU session modification procedure resulted in the change of ARP for a QoS flow that was already allocated an EBI. |  |
| n1SmMsg | RefToBinaryData | C | 0..1 | This IE shall be present if N1 SM Information needs to be sent to the UE. When present, this IE shall reference the N1 SM Message binary data (see clause 6.1.6.4.2). |  |
| n2SmInfo | RefToBinaryData | C | 0..1 | This IE shall be present if N2 SM Information needs to be sent to the AN. When present, this IE shall reference the N2 SM Information binary data (see clause 6.1.6.4.3). |  |
| n2SmInfoType | N2SmInfoType | C | 0..1 | This IE shall be present if "n2SmInfo" attribute is present. When present, this IE shall indicate the NG AP IE type for the NG AP SMF related IE container carried in "n2SmInfo" attribute. |  |
| epsBearerSetup | array(EpsBearerContainer) | C | 1..N | This IE shall be present during an EPS to 5GS handover using the N26 interface. When present, it shall include the EPS bearer context(s) successfully handed over to 5GS. |  |
| dataForwarding | boolean | C | 0..1 | This IE shall be present if it was present in the corresponding request. When present, it shall be set as specified in clause 5.2.2.3.9. |  |
| n3DlForwardingTnlList | array (IndirectDataForwardingTunnelInfo) | C | 1..N | This IE shall be present if indirect data forwarding is requested and N9 downlink indirect data forwarding tunnels info is included in the corresponding request. When present, it shall carry the list of N3 downlink indirect data forwarding tunnels info of source I-UPF or source UPF. | DTSSA |
| n3UlForwardingTnlList | array (IndirectDataForwardingTunnelInfo) | C | 1..N | This IE shall be present if indirect data forwarding is requested and N9 uplink indirect data forwarding tunnels info is included in the corresponding request. When present, it shall carry the list of N3 uplink indirect data forwarding tunnels info of source I-UPF or source UPF. | DTSSA |
| cause | Cause | C | 0..1 | This IE shall be present if the activation of the User Plane connection failed due to insufficient resources (see clause 5.2.2.3.2.2). |  |
| maAcceptedInd | boolean | C | 0..1 | This IE shall be present if a request to modify a single access PDU session into a MA PDU session was accepted (see clause 4.22.6.3 of 3GPP TS 23.502 [3]). When present, it shall be set as follows: - true: MA PDU session - false (default): single access PDU session | MAPDU |
| supportedFeatures | SupportedFeatures | C | 0..1 | This IE shall be present if the supportedFeatures IE was received in the request and at least one optional feature defined in clause 6.1.8 is supported by the updated SM context resource. |  |
| forwardingFTeid | Bytes | C | 0..1 | This IE shall be present during an EPS to 5GS Idle mode mobility using N26 interface with data forwarding (see clause 4.11.1.3.3A of 3GPP TS 23.502 [3]), if the Forwarding F-TEID IE shall be sent to the MME in the Context Acknowledge message. When present, it shall contain Base64-encoded characters, encoding the Forwarding F-TEID to be sent in the Context Acknowledge message, as specified in Figure 8.22-1 of 3GPP TS 29.274 [16] (starting from octet 1). | CIOT |
| forwardingBearerContexts | array(ForwardingBearerContainer) | C | 1..N | This IE shall be present during an EOS to 5GS Idle mode mobility using N26 interface with data forwarding (see clause 4.11.1.3.3A of 3GPP TS 23.502 [3]), if the Bearer Contexts IE shall be sent to the MME in the Context Acknowledge message. When present, it shall contain the Bearer Contexts to be sent in the Context Acknowledge message. | CIOT |
| selectedSmfId | NfInstanceId | C | 0..1 | This IE shall be present if a new (h)SMF is selected e.g. by the new I/V-SMF, or a SCP between the new I/V-SMF and the (h)SMF. (NOTE) When present, it shall contain the selected SMF NF Instance Id. | DTSSA |
| selectedOldSmfId | NfInstanceId | C | 0..1 | This IE shall be present if if another old I/V-SMF(as alternative to the old I/V-SMF) is selected, e.g. by the new I/V-SMF or a SCP between the new I/V-SMF and the old I/V-SMF. (NOTE) When present, it shall contain the selected old I/V-SMF NF Instance Id. | DTSSA |
| NOTE: During an SmContext Update procedure, if a new (h)SMF and/or another old I/V-SMF has been re-selected (since the old I/V-SMF or the (h)SMF is not reachable) by the new I-/V-SMF or a SCP, the selected old I-/V-SMF and/or (h)SMF shall be returned to the AMF, in order to perform potential subsequent operations on the SMF hosting the resource, e.g. to release the SM Context on old I-/V-SMF, or to create SM Context on SMF when the I/V-SMF needs to be removed. |  |  |  |  |  |

##### 6.1.6.2.6	Type: SmContextReleaseData

Table 6.1.6.2.6-1: Definition of type SmContextReleaseData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| cause | Cause | C | 0..1 | This IE shall be present, if the information is available. When present, this IE shall indicate the NF Service Consumer cause for the requested SM context release. |  |
| ngApCause | NgApCause | C | 0..1 | This IE shall be present, if the information is available. When present, this IE shall indicate the NGAP cause for the requested SM context release. |  |
| 5gMmCauseValue | 5GMmCause | C | 0..1 | This IE shall be included if the PDU session is released by the AMF due to any 5GMM failure. When present, this IE shall contain the 5GMM cause code value received from the UE. |  |
| ueLocation | UserLocation | C | 0..1 | This IE shall be present, if available. When present, it shall contain the UE location information (see clause 5.2.3.4). See NOTE. |  |
| ueTimeZone | TimeZone | C | 0..1 | This IE shall be present, if available. When present, it shall contain the UE Time Zone information. |  |
| addUeLocation | UserLocation | O | 0..1 | Additional UE location. This IE may be present, if anType previously reported is a non-3GPP access and a valid 3GPP access user location information is available. When present, it shall contain: - the last known 3GPP access user location (see clause 5.2.3.4); and - the timestamp, if available, indicating the UTC time when the addUeLocation information was acquired. See NOTE. |  |
| vsmfReleaseOnly | boolean | C | 0..1 | This IE shall be present and set to "true" during a 5GS to EPS Idle mode mobility or handover, for a Home Routed PDU session associated with 3GPP access and with assigned EBI(s), or during Registration, UE Triggered Service Request, Inter NG-RAN node Xn based handover and N2 based handover procedures with V-SMF change or removal. When present, it shall be set as follows: - true: release the SM context and PDU session in the V-SMF only; - false (default): release the SM context and PDU session in V-SMF and H-SMF. |  |
| n2SmInfo | RefToBinaryData | C | 0..1 | This IE shall be present if N2 SM Information has been received from the AN. When present, this IE shall reference the N2 SM Information binary data (see clause 6.1.6.4.3). |  |
| n2SmInfoType | N2SmInfoType | C | 0..1 | This IE shall be present if "n2SmInfo" attribute is present. When present, this IE shall indicate the NG AP IE type for the NG AP SMF related IE container carried in "n2SmInfo" attribute. |  |
| ismfReleaseOnly | boolean | C | 0..1 | This IE shall be present and set to "true" during a 5GS to EPS Idle mode mobility or handover with I-SMF removal, or during Registration, UE Triggered Service Request, Inter NG-RAN node Xn based handover and N2 based handover with I-SMF change or removal. When present, it shall be set as follows: - true: only release the SM context of the PDU session in the I-SMF; - false (default): release the SM context and PDU session in I-SMF and SMF. | DTSSA |
| NOTE: In shared networks, when the message is sent from the VPLMN to the HPLMN, the PLMN ID that is communicated in this IE shall be that of the selected Core Network Operator. In shared networks, when the AMF and SMF pertain to the same PLMN, the Primary PLMN ID shall be communicated in the ECGI or NCGI to the SMF. The Core Network Operator PLMN ID shall be communicated in the TAI and the Serving Network. |  |  |  |  |  |

##### 6.1.6.2.7	Type: SmContextRetrieveData

Table 6.1.6.2.7-1: Definition of type SmContextRetrieveData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| targetMmeCap | MmeCapabilities | C | 0..1 | This IE shall be present if it is available. When present, it shall contain the target MME capabilities. |  |
| smContextType | SmContextType | C | 0..1 | This IE shall be present if this is a request to retrieve the complete SM context, during scenarios with an I-SMF or V-SMF insertion/change/removal, or during SMF Context Transfer procedure for LBO or non-roaming PDU session without I-SMF (see clause 4.26.5.3 of 3GPP TS 23.502 [3]). This IE shall also be present if this is a request to retrieve the AF Coordination Information during the change of SSC mode 3 PDU Session Anchor with multiple PDU Sessions, if the runtime coordination between old SMF and AF is enabled (see clause 4.3.5.2 of 3GPP TS 23.502 [3]). | DTSSA, CTXTR EEDGE |
| servingNetwork | PlmnId | C | 0..1 | This IE shall be present when the procedure is triggered by a new V-SMF, if the new V-SMF supports inter-PLMN V-SMF change. When present, this IE shall contain the serving core network operator PLMN ID of the NF Service Consumer (i.e. new V-SMF). | DTSSA |
| notToTransferEbiList | array(EpsBearerId) | C | 1..N | This IE shall be present, if the SM context type IE is absent or indicate a request to retrieve the EPS PDN connection, and the AMF determines that certain EPS bearers shall not to be transferred to EPS during a 5GS to EPS mobility procedure, as specified in clause 4.11.1 of 3GPP TS 23.502 [3]. When present, it shall contain the EBI list not to be transferred. |  |
| ranUnchangedInd | boolean | C | 0..1 | This IE shall be present if AN Tunnel is required, in scenario of I-SMF/V-SMF change/insertion during registration procedure after EPS to 5GS handover or I-SMF selection per DNAI, when UE is in CM-CONNECTED state (see clause 5.2.2.6.1). When present, it shall be set as follows: - true: NG-RAN is not changed and the tunnel information is required; - false (default):NG-RAN is changed and the tunnel information is not required. | DTSSA |

##### 6.1.6.2.8	Type: SmContextStatusNotification

Table 6.1.6.2.8-1: Definition of type SmContextStatusNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| statusInfo | StatusInfo | M | 1 | This IE shall contain status information about the SM context. |  |
| smallDataRateStatus | SmallDataRateStatus | C | 0..1 | This IE shall be present, if the NF Service Consumer has indicated support of CIoT and if the status is available. When present, it shall indicate the current small data rate control status for the PDU session. | CIOT |
| apnRateStatus | ApnRateStatus | C | 0..1 | This IE shall be present, if the NF Service Consumer has indicated support of CIoT and if the status is available. When present, it shall indicate the current APN rate control status for the PDN connection (APN rates are shared by all PDN connections of the UE to this APN). | CIOT |
| ddnFailureStatus | boolean | C | 0..1 | This IE shall be present if the DDN Failure shall be reported (see clause 5.2.8.2.8 of 3GPP TS 23.502 [3]). When present, it shall be set as follows: - true: DDN failure detected - false (default): DDN failure is not detected | CIOT |
| notifyCorrelationIdsForddnFailure | array(string) | C | 1..N | This IE shall be present if the DDN Failure shall be reported. When present, it shall contain the notification correlation Id(s) of the DDN failure subscriptions for which a DDN failure has been detected. This parameter can be useful if the NF service consumer has multiple subscriptions for the same PDU session. | CIOT |
| newSmfId | NfInstanceId | C | 0..1 | This IE may be present if resourceStatus in statusInfo is: - TRANSFERRED When present, it shall include: - the new I-SMF instance identifier if the cause in statusInfo is "ISMF_CONTEXT_TRANSFER"; - the new SMF instance identifier if the cause in statusInfo is "SMF_CONTEXT_TRANSFER". This IE may also be present if the resourceStatus attribute in statusInfo is set to "UPDATED". When present, it shall include the NF instance identifier of the new H-SMF or SMF (for a PDU session with an I-SMF) handling the PDU session, when it is changed within an SMF set. | CTXTR ES3XX |
| newSmfSetId | NfSetId | C | 0..1 | This IE may be present if resourceStatus in statusInfo is: - TRANSFERRED When present, it shall include: - The new I-SMF set identifier if cause in statusInfo is "ISMF_SERVICE_CONTEXT_TRANSFER"; - The new SMF set identifier if cause in statusInfo is "SMF_SERVICE_CONTEXT_TRANSFER". | CTXTR |
| oldSmfId | NfInstanceId | C | 0..1 | This IE shall be present if resourceStatus in statusInfo is: - TRANSFERRED When present, it shall include: - The old I-SMF instance identifier if cause in statusInfo is "ISMF_CONTEXT_TRANSFER"; - The old SMF instance identifier if cause in statusInfo is "SMF_CONTEXT_TRANSFER". | CTXTR |
| oldSmContextRef | Uri | C | 0..1 | This IE may be present if resourceStatus in statusInfo is: - TRANSFERRED When present, this IE shall include the identifier of the SM Context resource in the old I-SMF or SMF. This IE may also be present if resourceStatus in statusInfo is "UNCHANGED", the SMF selection during PDU Session re-establishment for SSC mode 3 is needed and the runtime coordination between old SMF and AF is enabled. When present, this IE shall contain the URI of the SM Context resource in the old SMF with the structure: {apiRoot}/nsmf-pdusession/v1/sm-contexts/{smContextRef}. | CTXTR EEDGE |
| altAnchorSmfUri | Uri | C | 0..1 | This IE shall be present if resourceStatus in statusInfo is: - ALT_ANCHOR_SMF When present, it shall contain the API URI of the alternative (H-)SMF towards which the PDU session is established. | AASN |
| altAnchorSmfId | NfInstanceId | C | 0..1 | This IE may be present if resourceStatus in statusInfo is: - ALT_ANCHOR_SMF When present, it shall contain the NF Instance Id of the alternative (H-)SMF towards which the PDU session is established. | AASN |
| targetDnaiInfo | TargetDnaiInfo | C | 0..1 | This IE shall be present if the I-SMF selection for the current PDU session, or SMF selection during PDU Session re-establishment for SSC mode 2/3 is needed, or if it is received from the SMF in Notify Status. When present, this IE shall include the target DNAI Information. | EEDGE |
| NOTE: If resourceStatus in statusInfo is "TRANSFERRED", at least one of newSmfId and newSmfSetId shall be included. |  |  |  |  |  |

##### 6.1.6.2.9	Type: PduSessionCreateData

Table 6.1.6.2.9-1: Definition of type PduSessionCreateData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supi | Supi | C | 0..1 | This IE shall be present, except if the UE is emergency registered and UICCless. When present, it shall contain the subscriber permanent identify. |  |
| unauthenticatedSupi | boolean | C | 0..1 | This IE shall be present if the SUPI is present in the message but is not authenticated and is for an emergency registered UE. When present, it shall be set as follows: - true: unauthenticated SUPI; - false (default): authenticated SUPI. |  |
| pei | Pei | C | 0..1 | This IE shall be present if the UE is emergency registered and it is either UIClless or the SUPI is not authenticated. For all other cases, this IE shall be present if it is available. When present, it shall contain the permanent equipment identifier. |  |
| pduSessionId | PduSessionId | C | 0..1 | This IE shall contain the PDU Session ID, except during an EPS to 5GS Idle mode mobility or handover using the N26 interface. |  |
| dnn | Dnn | M | 1 | This IE shall contain the requested DNN. The DNN shall be the full DNN (i.e. with both the Network Identifier and Operator Identifier) for a HR PDU session, and it should be the full DNN in LBO and non-roaming scenarios. If the Operator Identifier is absent, the serving core network operator shall be assumed. |  |
| selectedDnn | Dnn | C | 0..1 | This IE shall be present, if another DNN other than the UE requested DNN is selected for this PDU session. When present, it shall contain the selected DNN. The DNN shall be the full DNN (i.e. with both the Network Identifier and Operator Identifier) for a HR PDU session, and it should be the full DNN in LBO and non-roaming scenarios. If the Operator Identifier is absent, the serving core network operator shall be assumed. |  |
| sNssai | Snssai | C | 0..1 | This IE shall be present, except during an EPS to 5GS idle mode mobility or handover using the N26 interface. When present, it shall contain: - the HPLMN S-NSSAI for a HR PDU session, which is mapped from the requested S-NSSAI by the VPLMN; or - the requested S-NSSAI in the serving PLMN for a PDU session with an I-SMF. |  |
| vsmfId | NfInstanceId | C | 0..1 | This IE shall be present for a HR PDU session. When present, it shall contain the identifier of the V-SMF. |  |
| ismfId | NfInstanceId | C | 0..1 | This IE shall be present for a PDU session with an I-SMF. When present, it shall contain the identifier of the I-SMF. | DTSSA |
| servingNetwork | PlmnIdNid | M | 1 | This IE shall contain the serving core network operator PLMN ID and, for an SNPN, the NID that together with the PLMN ID identifies the SNPN. |  |
| requestType | RequestType | C | 0..1 | This IE shall be present if the Request type IE is received from the UE for a single access PDU session and if the request refers to an existing PDU session or an existing emergency PDU session. The requestType IE shall not be included for a MA-PDU session establishment request. It may be present otherwise. When present, it shall indicate whether the request refers to a new PDU session or emergency PDU session, or to an existing PDU session or emergency PDU session. For request sent from AMF, this IE shall be set based on the requestType received. |  |
| epsBearerId | array(EpsBearerId) | C | 1..N | This IE shall be present during an EPS to 5GS Idle mode mobility or handover preparation using the N26 interface. When present, it shall contain the list of EPS bearer Id(s) received from the MME. |  |
| pgwS8cFteid | Bytes | C | 0..1 | This IE shall be present during an EPS to 5GS Idle mode mobility or handover preparation using the N26 interface. When present, it shall contain Base64-encoded characters, encoding the PGW S8 F-TEID for Control Plane as specified in Figure 8.22-1 of 3GPP TS 29.274 [16] (starting from octet 1), received from the MME. |  |
| vsmfPduSessionUri | Uri | C | 0..1 | This IE shall be present for a HR PDU session. When present, it shall include the callback URI representing the PDU session in the V-SMF. |  |
| ismfPduSessionUri | Uri | C | 0..1 | This IE shall be present for a PDU session with an I-SMF. When present, it shall include the callback URI representing the PDU session in the I-SMF. | DTSSA |
| vcnTunnelInfo | TunnelInfo | C | 0..1 | This IE shall be present for a HR PDU session, except for EPS to 5GS handover using N26 interface and when Control Plane CIoT 5GS Optimisation is enabled and data delivery via NEF is selected for this PDU session. When present, this IE shall contain the N9 tunnel information of the visited CN side, i.e. V-UPF. |  |
| icnTunnelInfo | TunnelInfo | C | 0..1 | This IE shall be present for a PDU session involving an I-SMF, except when Control Plane CIoT 5GS Optimisation is enabled and data delivery via NEF is selected for this PDU session. When present, this IE shall contain the N9 tunnel information of the I-UPF controlled by the I-SMF. | DTSSA |
| n9ForwardingTunnelInfo | TunnelInfo | C | 0..1 | This IE shall be present during Service Request procedures with I-SMF insertion, if buffered DL data is available at the I-UPF that is controlled by the SMF (see clause 4.23.4 in 3GPP TS 23.502 [3]). When present, this IE shall contain the N9 tunnel information of the I-UPF controlled by the I-SMF. | DTSSA |
| additionalCnTunnelInfo | TunnelInfo | C | 0..1 | This IE shall be present if a MA PDU session is requested or if the PDU session is allowed to be upgraded to a MA PDU session, and the UE is registered over both 3GPP access and Non-3GPP access. When present, it shall contain additional N9 tunnel information of the UPF controlled by the V-SMF or I-SMF. | MAPDU |
| anType | AccessType | M | 1 | This IE shall indicate the Access Network Type to which the PDU session is to be associated. |  |
| additionalAnType | AccessType | C | 0..1 | This IE shall indicate the additional Access Network Type to which the PDU session is to be associated. This IE shall be present if a MA-PDU session is requested and the UE is registered over both 3GPP access and Non-3GPP access. | MAPDU |
| ratType | RatType | C | 0..1 | This IE shall be present and indicate the RAT Type used by the UE, if available. |  |
| ueLocation | UserLocation | C | 0..1 | This IE shall contain the UE location information (see clause 5.2.3.4), if it is available. (NOTE 1) |  |
| ueTimeZone | TimeZone | C | 0..1 | This IE shall contain the UE Time Zone, if it is available. |  |
| addUeLocation | UserLocation | O | 0..1 | Additional UE location. This IE may be present, if anType indicates a non-3GPP access and a valid 3GPP access user location information is available. When present, it shall contain: - the last known 3GPP access user location (see clause 5.2.3.4); and - the timestamp, if available, indicating the UTC time when the addUeLocation information was acquired. (NOTE 1) |  |
| gpsi | Gpsi | C | 0..1 | This IE shall be present if it is available. When present, it shall contain the user's GPSI. |  |
| n1SmInfoFromUe | RefToBinaryData | C | 0..1 | This IE shall be present if the V-SMF or I-SMF has received known N1 SM information from the UE that does not need to be interpreted by the V-SMF or I-SMF. When present, this IE shall reference the n1SmInfoFromUe binary data (see clause 6.1.6.4.4). |  |
| unknownN1SmInfo | RefToBinaryData | C | 0..1 | This IE shall be present if the V-SMF or I-SMF has received unknown N1 SM information from the UE. When present, this IE shall reference the unknownN1SmInfo binary data (see clause 6.1.6.4.4). |  |
| supportedFeatures | SupportedFeatures | C | 0..1 | This IE shall be present if at least one optional feature defined in clause 6.1.8 is supported. |  |
| hPcfId | NfInstanceId | O | 0..1 | This IE may be used by V-SMF to indicate the home PCF selected by the AMF for the UE to the H-SMF, for a HR PDU session. When present, this IE shall contain the identifier of the H-PCF selected by the AMF for the UE (for UE Policy Control). |  |
| pcfId | NfInstanceId | O | 0..1 | This IE may be used by I-SMF to indicate the (V-)PCF selected by the AMF for the UE to the SMF, for a PDU session with an I-SMF. When present, this IE shall contain the identifier of the PCF (for Access and Mobility Policy Control and/or UE Policy Control) in non-roaming scenarios, or the V-PCF (for Access and Mobility Policy Control) in LBO roaming scenarios. | DTSSA |
| pcfGroupId | NfGroupId | O | 0..1 | This IE may be present in non-roaming and HR roaming scenarios. When present, this IE shall contain the identity of the (home) PCF group serving the UE for Access and Mobility Policy and/or UE Policy. |  |
| pcfSetId | NfSetId | O | 0..1 | When present, it shall contain the NF Set ID of the H-PCF indicated by the hPcfId IE or the (V-)PCF indicated by the pcfId IE. |  |
| hoPreparationIndication | boolean | C | 0..1 | This IE shall be present during an EPS to 5GS handover preparation using the N26 interface or during N2 handover preparation with I-SMF insertion. When present, it shall be set as follows: - true: an EPS to 5GS handover preparation or N2 handover preparation with I-SMF is in progress; the PGW-C/SMF shall not switch the DL user plane of the PDU session yet. - false: there is no on-going EPS to 5GS handover preparation or N2 handover preparation with I-SMF in progress. If a handover preparation was in progress, the handover has been completed. The PGW-C/SMF shall switch the DL user plane of the PDU session using the N9 tunnel information that has been received in the vcnTunnelInfo or icnTunnelInfo. It shall be set to "true" during an EPS to 5GS handover preparation using the N26 interface or during N2 handover preparation with I-SMF insertion. |  |
| selMode | DnnSelectionMode | C | 0..1 | This IE shall be present if it is available. When present, it shall be set to: - "VERIFIED", if the requested DNN provided by UE or the selected DNN provided by the network corresponds to an explicitly subscribed DNN; or - "UE_DNN_NOT_VERIFIED", if the requested DNN provided by UE corresponds to the usage of a wildcard subscription; or - "NW_DNN_NOT_VERIFIED", if the selected DNN provided by the network corresponds to the usage of a wildcard subscription. If both the requested DNN (i.e. dnn IE) and selected DNN (i.e. selected Dnn IE) are present, the selMode shall be related to the selected DNN. |  |
| alwaysOnRequested | boolean | C | 0..1 | This IE shall be present and set to true if the UE requests to setup an always-on PDU session and this is allowed by local policy in the V-SMF or I-SMF. When present, it shall be set as follows: - true: request for an always-on PDU session - false (default): not a request for an always-on PDU session |  |
| udmGroupId | NfGroupId | O | 0..1 | When present, it shall indicate the identity of the UDM group serving the UE. |  |
| routingIndicator | string | O | 0..1 | When present, it shall indicate the Routing Indicator of the UE. |  |
| hNwPubKeyId | integer | O | 0..1 | When present, it shall indicate the Home Network Public Key Identifier of the UE. (NOTE 2) |  |
| epsInterworkingInd | EpsInterworkingIndication | O | 0..1 | This IE may be present if the indication has been received from AMF and is allowed to be forwarded to H-SMF by operator configuration. When present, this IE shall indicate whether the PDU session may possibly be moved to EPS and whether N26 interface to be used during EPS interworking procedures. |  |
| vSmfServiceInstanceId | string | O | 0..1 | When present, this IE shall contain the serviceInstanceId of the V-SMF service instance serving the PDU session. This IE may be used by the H-SMF to identify PDU sessions affected by a failure or restart of the V-SMF service (see clauses 6.2 and 6.3 of 3GPP TS 23.527 [24]). |  |
| iSmfServiceInstanceId | string | O | 0..1 | When present, this IE shall contain the serviceInstanceId of I-SMF service instance serving the PDU session. This IE may be used by the SMF to identify PDU sessions affected by a failure or restart of the I-SMF service (see clauses 6.2 and 6.3 of 3GPP TS 23.527 [24]). | DTSSA |
| recoveryTime | DateTime | O | 0..1 | Timestamp when the V-SMF or I-SMF service instance serving the PDU session was (re)started (see clause 6.3 of 3GPP TS 23.527 [24]). |  |
| roamingChargingProfile | RoamingChargingProfile | O | 0..1 | Roaming Charging Profile applicable in the VPLMN (see clauses 5.1.9.1, 5.2.1.7 and 5.2.2.12.2 of 3GPP TS 32.255 [25]). |  |
| chargingId | string | O | 0..1 | Charging ID (see clauses 5.1.9.1 of 3GPP TS 32.255 [25]). |  |
| oldPduSessionId | PduSessionId | C | 0..1 | This IE shall be present if this information is received from the UE and the same SMF is selected for SSC mode 3. When present, it shall contain the old PDU Session ID received from the UE. See clauses 4.3.2.2.1 and 4.3.5.2 of 3GPP TS 23.502 [3]. |  |
| epsBearerCtxStatus | EpsBearerContextStatus | C | 0..1 | This IE shall be present during an EPS to 5GS idle mode mobility using the N26 interface, if received in the Create SM Context request. When present, it shall be set to the value received in the Create SM Context request. |  |
| amfNfId | NfInstanceId | C | 0..1 | This IE shall be present if it is received in the Create SM Context request, unless the PDU session is related to regulatory prioritized service. When present, it shall contain the identifier of the serving AMF. |  |
| guami | Guami | C | 0..1 | This IE shall be present if the amfNfId is present. When present, it shall contain the serving AMF's GUAMI. |  |
| maxIntegrityProtectedDataRateUl | MaxIntegrityProtectedDataRate | C | 0..1 | This IE shall be present if it is available. When present, it shall indicate the maximum integrity protected data rate supported by the UE for uplink. |  |
| maxIntegrityProtectedDataRateDl | MaxIntegrityProtectedDataRate | C | 0..1 | This IE shall be present if it is available. When present, it shall indicate the maximum integrity protected data rate supported by the UE for downlink. |  |
| cpCiotEnabled | boolean | C | 0..1 | This IE shall be present with the value "True" if the "5gCiotCpEnabled" attribute is received with "True" value in SM Context Create request, indicating the Control Plane CIoT 5GS Optimisation is enabled for the PDU session (see 3GPP TS 23.502 [3], clause 4.3.2.2.2). When present, it shall be set as follows: - True: Control Plane CIoT 5GS Optimisation is enabled. - False (default): Control Plane CIoT 5GS Optimisation is not enabled. | CIOT |
| cpOnlyInd | boolean | C | 0..1 | This IE shall be present with the value "True", if the PDU session shall only use Control Plane CIoT 5GS Optimisation (see clause 5.31.4.1 of 3GPP TS 23.501 [2]). When present, it shall be set as follows: - True: the PDU session shall only use Control Plane CIoT 5GS Optimisation - False (default): the PDU session is not constrained to only use Control Plane CIoT 5GS Optimisation. | CIOT |
| invokeNef | boolean | C | 0..1 | This IE shall be present with value "True", if Control Plane CIoT 5GS Optimisation is enabled and data delivery via NEF is selected for the PDU session. When present, it shall be set as follows: - True: Data delivery via NEF is selected. - False (default): Data delivery via NEF is not selected. | CIOT |
| maRequestInd | boolean | C | 0..1 | This IE shall be present if a MA-PDU session is requested to be established by the UE. When present, it shall be set as follows: - True: a MA-PDU session is requested - False (default): a MA-PDU session is not requested | MAPDU |
| maNwUpgradeInd | boolean | C | 0..1 | This IE shall only be present if the PDU session is allowed to be upgraded to MA PDU session (see clause 4.22.3 of 3GPP TS 23.502 [3]). When present, it shall be set as follows: - True: the PDU session is allowed to be upgraded to MA PDU session - False (default): the PDU session is not allowed to be upgraded to MA PDU session When maRequestInd is present and set to "true", this IE shall not be present. | MAPDU |
| dnaiList | array(Dnai) | C | 1..N | This IE shall be present over N16a if an I-SMF is inserted into a PDU session during the following procedures: PDU session establishment, Registration, Service Request, Xn based handover, Inter NG-RAN node N2 based handover (see clause 4.23 of 3GPP TS 23.502 [3]). When present, it shall include the list of DNAIs supported by the I-SMF. | DTSSA |
| presenceInLadn | PresenceState | C | 0..1 | This IE shall be present during Xn based handover with I-SMF insertion, if the DNN corresponds to a LADN. When present, it shall be set to "IN" or "OUT" to indicate that the UE is in or out of the LADN service area. | DTSSA |
| secondaryRatUsageInfo | array(SecondaryRatUsageInfo) | O | 1..N | This IE may be present to report usage data for a secondary RAT for QoS flows and/or the whole PDU session. | DTSSA |
| smallDataRateStatus | SmallDataRateStatus | C | 0..1 | This IE shall be present if the small data rate control status is received from AMF, see clause 5.31.14.3 of 3GPP TS 23.501 [2] and clause 4.3.2.2.2 of 3GPP TS 23.502 [3]. | CIOT |
| apnRateStatus | ApnRateStatus | C | 0..1 | This IE shall be present, if the APN rate control status (APN rates are shared by all PDN connections of the UE to this APN) is received from the AMF, see clause 4.7.7.3 in 3GPP TS 23.401 [33] and clause 4.11.5.3 in 3GPP TS 23.502 [3]. | CIOT |
| dlServingPlmnRateCtl | integer | C | 0..1 | This IE shall be present if Serving PLMN Rate Control for downlink data packets is enabled in the PLMN and Control Plane CIoT 5GS Optimisation is enabled for the PDU session. When present, this IE shall contain the maximum allowed number of Downlink NAS Data PDUs per deci hour of the serving PLMN, as specified in clause 5.31.14.2 of 3GPP TS 23.501 [2]. Minimum: 10 | CIOT |
| upSecurityInfo | UpSecurityInfo | C | 0..1 | This IE shall be present if received from NG-RAN during Xn handover procedure with I-SMF Insertion (see clause 5.2.2.7.5). When present, this IE shall contain the User Plane Security Information associated to the PDU session. See clause 9.3.1.60 of 3GPP TS 38.413 [9]. | DTSSA |
| vplmnQos | VplmnQos | C | 0..1 | This IE shall be present for a HR PDU session, if the V-SMF supports the VQOS feature and if VPLMN QoS constraints are required for the PDU session. When present, this IE shall contain the QoS constraints from the VPLMN. | VQOS |
| ivSmfRestoration | boolean | C | 0..1 | This IE shall be present if the I-SMF/V-SMF restoration procedure is triggered, as specified in clause 6.7 of 3GPP TS 23.527 [24]. When present, it shall be set as follows: - true: I-SMF/V-SMF restoration procedure is required; - false (default): I-SMF/V-SMF restoration procedure is not required. | IVRES |
| oldSmContextRef | Uri | C | 0..1 | This IE shall be present, if it is received in the Create SM Context request. When present, this IE shall contain the identifier of the SM Context resource in the old SMF. | EEDGE |
| redundantPduSessionInfo | RedundantPduSessionInformation | C | 0..1 | This IE shall be present for a PDU session with an I-SMF, if an RSN and/or PDU Session Pair ID was received from the UE. | DCE2ER |
| NOTE 1: In shared networks, the PLMN ID that is communicated in this IE shall be that of the selected Core Network Operator. NOTE 2: If present, this attribute shall be used together with routingIndicator. This attribute is only used by the HPLMN in roaming scenarios. |  |  |  |  |  |

##### 6.1.6.2.10	Type: PduSessionCreatedData

Table 6.1.6.2.10-1: Definition of type PduSessionCreatedData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| pduSessionType | PduSessionType | M | 1 | This IE shall indicate the selected PDU type. |  |
| sscMode | string | M | 1 | This IE shall indicate the SSC mode applicable to the PDU session. When present, it shall be encoded as one character in hexadecimal representation, taking a value of "0" to "7", representing the 3 bits of the SSC mode value of the SSC mode IE specified in clause 9.11.4.16 of 3GPP TS 24.501 [7]. Pattern: "^[0-7]$" Example: SSC mode 3 shall be encoded as "3". (NOTE 1). |  |
| hcnTunnelInfo | TunnelInfo | C | 0..1 | This IE shall be present for a HR PDU session, except when Control Plane CIoT 5GS Optimisation is enabled and data delivery via NEF is selected for this PDU session. When present, this IE shall contain the N9 tunnel information of the home CN side, i.e. H-UPF. |  |
| cnTunnelInfo | TunnelInfo | C | 0..1 | This IE shall be present for a PDU session involving an I-SMF, except when Control Plane CIoT 5GS Optimisation is enabled and data delivery via NEF is selected for this PDU session. When present, this IE shall contain the N9 tunnel information of the SMF. | DTSSA |
| additionalCnTunnelInfo | TunnelInfo | C | 0..1 | This IE shall be present if a MA-PDU session is established for a UE registered over both 3GPP access and Non-3GPP access. When present, it shall contain additional N9 tunnel information of the UPF controlled by the H-SMF or SMF. | MAPDU |
| sessionAmbr | Ambr | C | 0..1 | This IE shall be present, except when Control Plane CIoT 5GS Optimisation is enabled for the PDU session. When present, this IE shall contain the Session AMBR granted to the PDU session. |  |
| qosFlowsSetupList | array(QosFlowSetupItem) | C | 1..N | This IE shall be present, except when Control Plane CIoT 5GS Optimisation is enabled for the PDU session. When present, this IE shall contain the full set of QoS flow(s) to establish for the PDU session. It shall contain at least the Qos flow associated to the default Qos rule. In V-SMF/I-SMF insertion scenarios where no QoS Rule(s) associated to a QoS flow can or need to be sent to the UE, the qosRules attribute of the QosFlowSetupItem may be set to an empty string or to the latest QoS Rule(s) associated to the QoS flow. (NOTE 3) |  |
| hSmfInstanceId | NfInstanceId | C | 0..1 | This IE shall be present for a HR PDU session. When present, it shall contain the identifier of the home SMF. |  |
| smfInstanceId | NfInstanceId | C | 0..1 | This IE shall be present for a PDU session with an I-SMF. When present, it shall contain the identifier of the SMF. | DTSSA |
| pduSessionId | PduSessionId | C | 0..1 | This IE shall be present during an EPS to 5GS Idle mode mobility or handover preparation using the N26 interface. When present, it shall be set to the PDU Session ID. |  |
| sNssai | Snssai | C | 0..1 | This IE shall be present during an EPS to 5GS Idle mode mobility or handover using the N26 interface. When present, it shall contain: - the S-NSSAI assigned to the PDU session in the Home PLMN, for a HR PDU session; - the S-NSSAI assigned to the PDU session in the serving PLMN, for a PDU session with an I-SMF. The Snssai shall overwrite the S-NSSAI earlier stored in I-SMF, if they are different. |  |
| enablePauseCharging | boolean | C | 0..1 | This IE shall be present, based on operator's policy, to enable the use of Pause of Charging for the PDU session (see clause 4.4.4 of 3GPP TS 23.502 [3]). When present, it shall be set as follows: - true: enable Pause of Charging; - false (default): disable Pause of Charging. |  |
| ueIpv4Address | Ipv4Addr | C | 0..1 | This IE shall be present if the SMF assigns a UE IPv4 address to the PDU session. |  |
| ueIpv6Prefix | Ipv6Prefix | C | 0..1 | This IE shall be present if the SMF assigns a UE IPv6 prefix to the PDU session. |  |
| n1SmInfoToUe | RefToBinaryData | C | 0..1 | This IE shall be present if the SMF needs to send N1 SM information to the UE that does not need to be interpreted by the V-SMF or I-SMF. When present, this IE shall reference the n1SmInfoToUe binary data (see clause 6.1.6.4.4). |  |
| epsPdnCnxInfo | EpsPdnCnxInfo | C | 0..1 | This IE shall be present if the PDU session may be moved to EPS during its lifetime. |  |
| epsBearerInfo | array(EpsBearerInfo) | C | 1..N | This IE shall be present if the PDU session may be moved to EPS during its lifetime. |  |
| supportedFeatures | SupportedFeatures | C | 0..1 | This IE shall be present if at least one optional feature defined in clause 6.1.8 is supported. |  |
| maxIntegrityProtectedDataRate | MaxIntegrityProtectedDataRate | C | 0..1 | This IE shall be present if the upSecurity IE is present and indicates that integrity protection is preferred or required. When present, it shall indicate the maximum integrity protected data rate for uplink. If the maxIntegrityProtectedDataRateDl IE is absent, this IE applies to both uplink and downlink. |  |
| maxIntegrityProtectedDataRateDl | MaxIntegrityProtectedDataRate | C | 0..1 | This IE may be present if the upSecurity IE is present and indicates that integrity protection is preferred or required. When present, it shall indicate the maximum integrity protected data rate for downlink. |  |
| alwaysOnGranted | boolean | C | 0..1 | This IE shall be present if the alwaysOnRequested IE was received in the request or if the SMF determines, based on local policy, that the PDU session needs to be established as an always-on PDU session. When present, it shall be set as follows: - true: always-on PDU session granted. - false (default): always-on PDU session not granted. |  |
| gpsi | Gpsi | C | 0..1 | This IE shall be present if no GPSI IE is provided in the request, e.g. for a PDU session moved from another access or another system, and the SMF knows that a GPSI is already associated with the PDU session. When present, it shall contain the user's GPSI associated with the PDU session. |  |
| upSecurity | UpSecurity | O | 0..1 | When present, this IE shall indicate the security policy for integrity protection and encryption for the user plane of the PDU session. |  |
| roamingChargingProfile | RoamingChargingProfile | O | 0..1 | Roaming Charging Profile selected by the HPLMN (see clauses 5.1.9.1, 5.2.1.7 and 5.2.2.12.2 of 3GPP TS 32.255 [25]). |  |
| hSmfServiceInstanceId | string | O | 0..1 | When present, this IE shall contain the serviceInstanceId of the H-SMF service instance serving the PDU session, for a HR PDU session. This IE may be used by the V-SMF to identify PDU sessions affected by a failure or restart of the H-SMF service (see clause 6.2 of 3GPP TS 23.527 [24]). |  |
| smfServiceInstanceId | string | O | 0..1 | When present, this IE shall contain the serviceInstanceId of the SMF service instance serving the PDU session, for a PDU session with an I-SMF. This IE may be used by the I-SMF to identify PDU sessions affected by a failure or restart of the SMF service (see clause 6.2 of 3GPP TS 23.527 [24]). | DTSSA |
| recoveryTime | DateTime | O | 0..1 | Timestamp when the SMF service instance serving the PDU session was (re)started (see clause 6.3 of 3GPP TS 23.527 [24]). |  |
| dnaiList | array(Dnai) | C | 1..N | This IE shall be present over N16a, if available and an I-SMF has been inserted into a PDU session, during the following procedures: PDU session establishment, Registration, Service Request, Xn based handover, Inter NG-RAN node N2 based handover (see clause 4.23 of 3GPP TS 23.502 [3]). When present, it shall include the list of DNAIs of interest for the PDU session for local traffic steering at the I-SMF. | DTSSA |
| ipv6MultiHomingInd | boolean | C | 0..1 | This IE shall be present over N16a, if available and an I-SMF has been inserted into the PDU session during the following procedures: PDU session establishment, Registration, Service Request, Xn based handover, Inter NG-RAN node N2 based handover (see clause 4.23 of 3GPP TS 23.502 [3]). When present, it shall be set as follows: - true: IPv6 multi-homing is permitted. - false (default): IPv6 multi-homing is not allowed. | DTSSA |
| maAcceptedInd | boolean | C | 0..1 | This IE shall be present if a request to establish a MA PDU session was accepted or if a single access PDU session was upgraded into a MA PDU session (see clauses 4.22.2 and 4.22.3 of 3GPP TS 23.502 [3]). When present, it shall be set as follows: - true: MA PDU session - false (default): single access PDU session | MAPDU |
| homeProvidedChargingId | string | O | 0..1 | When present, this IE shall contain the Home provided Charging ID (see 3GPP TS 32.255 [25]). This IE shall be present during an EPS to 5GS Idle mode mobility or Handover of a HR PDU session. (NOTE 5) |  |
| nefExtBufSupportInd | boolean | C | 0..1 | This IE shall be present with value "true", if NEF has indicated Extended Buffering Support for mobile terminated data in SMF-NEF connection establishment response. When present, this IE shall be set as following: - true: Extended Buffering supported by NEF - false (default): Extended Buffering not supported by NEF | CIOT |
| smallDataRateControlEnabled | boolean | C | 0..1 | This IE shall be present and set to "true" if small data rate control is applicable on the PDU session. When present, it shall be set as follows: - true: small data rate control is applicable. - false (default): small data rate control is not applicable. | CIOT |
| ueIpv6InterfaceId | string | C | 0..1 | This IE shall be present if the H-SMF/SMF has assigned IPv6 interface identifier to the UE during the PDU session establishment for the Home-routed Roaming scenario or for a PDU session with an I-SMF. When present, it shall encode the UE IPv6 Interface Identifier to be used by the UE for its link-local address configuration with 16 hexadecimal digits. Pattern: "^[A-Fa-f0-9]{16}$" |  |
| ipv6Index | IpIndex | C | 0..1 | This IE shall be present if IPv6 Index has been received from PCF during SM Policy Creation. (NOTE 4) | DTSSA |
| dnAaaAddress | IpAddress | O | 0..1 | When present, this IE shall contain the address of DN-AAA server for UE IP Address allocation that has been received from UDM. (NOTE 4). | DTSSA |
| redundantPduSessionInfo | RedundantPduSessionInformation | C | 0..1 | This IE shall be present for a PDU session with an I-SMF, if Dual Connectivity based end to end Redundant User Plane Paths shall apply as specified in clause 5.33.2.1 of 3GPP TS 23.501 [2], regardless of whether the redundantPduSessionInfo IE was received or not in the request. If an RSN and/or PDU Session Pair ID was received from the UE, the same RSN and/or PDU Session Pair ID shall be returned in the response; additionally, if either the RSN or PDU Session Pair ID was not received from the UE, the anchor SMF shall determine and also return an RSN or PDU Session Pair ID respectively in the response. | DCE2ER |
| nspuSupportInd | boolean | C | 0..1 | This IE shall be present and set to "true" if enablePauseCharging is set to "true" and if the (H-)SMF and PSA UPF support Notify Start Pause of Charging via user plane feature as specified in clause 5.30 of 3GPP TS 29.244 [29]. When present, it shall be set as follows: - true: Notify Start Pause of Charging via user plane feature is supported. |  |
| NOTE 1: This IE contains information that the V-SMF or I-SMF only needs to transfer to the UE (without interpretation). It is sent as a separate IE rather than within the n1SmInfoToUE binary data because the Selected SSC mode IE is defined as a "V" IE (i.e. without a Type field) in the NAS PDU Session Establishment Accept message. NOTE 2: In scenarios with a V-SMF/I-SMF insertion, the V-SMF/I-SMF may receive in the Create Response some IEs it has already received during the earlier SM context retrieval from the SMF (e.g. due to the condition of presence of IEs in the Create Response). In such a case, the V-SMF/I-SMF shall overwrite the IEs earlier received with the new IEs received in the Create Response. NOTE 3: The V-SMF/I-SMF shall ignore any QoS Rule(s) associated to a QoS flow received in PduSessionCreatedData during V-SMF/I-SMF insertion scenarios where no QoS Rule(s) can be sent to the UE, i.e. during Registration, Inter NG-RAN node N2 based handover, and EPS to 5GS Idle mode mobility/handover using N26 interface procedures with V-SMF/I-SMF insertion, or during Service Request and Xn based handover procedures with I-SMF insertion. In such scenarios, the (H-)SMF shall initiate a subsequent PDU session modification procedure if it needs to change the QoS Rules associated to the QoS flows. NOTE 4: The I-SMF may use IPv6 index to assist in selecting how the IPv6 prefix is to be allocated for local PSA when IPv6 multi-homing is applied for the PDU session. If the IPv6 index indicates UE IP address allocation should be performed towards DN-AAA server, the DN-AAA server address may be included from the SMF to the I-SMF. NOTE 5: The chargingId IE in SmContext (see clause 6.1.6.2.39) shall be set to the value received in the homeProvidedChargingId IE during an EPS to 5GS Idle mode mobility or Handover of a HR PDU session. |  |  |  |  |  |

##### 6.1.6.2.11	Type: HsmfUpdateData

Table 6.1.6.2.11-1: Definition of type HsmfUpdateData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| requestIndication | RequestIndication | M | 1 | This IE shall indicate the request type. |  |
| pei | Pei | C | 0..1 | This IE shall be present if it is available and has not been provided earlier to the H-SMF or SMF. When present, this IE shall contain the permanent equipment identifier. |  |
| vcnTunnelInfo | TunnelInfo | C | 1 | This IE shall be present if the N9 tunnel information on the visited CN side provided earlier to the H-SMF has changed. When present, this IE shall contain the new N9 tunnel information on the visited CN side. |  |
| icnTunnelInfo | TunnelInfo | C | 0..1 | This IE shall be present if the N9 tunnel information of the I-UPF for DL traffic provided earlier by the I-SMF to the SMF has changed. When present, this IE shall contain the new N9 tunnel information of the I-UPF. | DTSSA |
| additionalCnTunnelInfo | TunnelInfo | C | 0..1 | This IE shall be present if additional N9 tunnel information provided earlier has changed, or if the UE requests to establish resources for a MA PDU session over the other access. When present, it shall contain additional N9 tunnel information of the UPF controlled by the V-SMF or I-SMF. | MAPDU |
| servingNetwork | PlmnIdNid | C | 0..1 | This IE shall contain the serving core network operator PLMN ID, and, for an SNPN, the NID that together with the PLMN ID identifies the SNPN, if the serving network has changed. |  |
| anType | AccessType | C | 0..1 | This IE shall be present if the Access Network Type provided earlier to the H-SMF or SMF has changed, e.g. during a handover of the PDU session between 3GPP access and untrusted non-3GPP access (see clause 5.2.2.8.2.5). When present, this IE shall indicate the new Access Network Type to which the PDU session is to be associated. |  |
| additionalAnType | AccessType | C | 0..1 | This IE shall indicate the additional Access Network Type to which the PDU session is to be associated. This IE shall be present when the UE requests to establish resources for MA PDU session over the other access. | MAPDU |
| ratType | RatType | C | 0..1 | This IE shall be present and indicate the RAT Type used by the UE, if available, upon a change of RAT Type. |  |
| ueLocation | UserLocation | C | 0..1 | This IE shall be present if it is available, the UE Location has changed and needs to be reported to the H-SMF or SMF. When present, this IE shall contain: - the new UE location information (see clause 5.2.3.4); and - the timestamp, if available, indicating the UTC time when the UeLocation information was acquired. (NOTE 1) |  |
| ueTimeZone | TimeZone | C | 0..1 | This IE shall be present if it is available, the UE Time Zone has changed and needs to be reported to the H-SMF or SMF. When present, this IE shall contain the new UE Time Zone. |  |
| addUeLocation | UserLocation | O | 0..1 | Additional UE location. This IE may be present, if anType indicates a non-3GPP access and a valid 3GPP access user location information is available. When present, it shall contain: - the last known 3GPP access user location (see clause 5.2.3.4); and - the timestamp, if available, indicating the UTC time when the addUeLocation information was acquired. (NOTE 1) |  |
| pauseCharging | boolean | C | 0..1 | This IE shall be present if the H-SMF or SMF enabled the use of Pause Pause of Charging for the PDU session during the PDU session establishment and Pause of Charging needs to be started or stopped (see clause 4.4.4 of 3GPP TS 23.502 [3]). When present, it shall be set as follows: - true: to Start Pause of Charging; - false: to Stop Pause of Charging. |  |
| pti | ProcedureTransactionId | C | 0..1 | This IE shall be present if the requestIndication indicates a UE requested PDU session modification or release. When present, it shall contain the PTI value received from the UE. |  |
| n1SmInfoFromUe | RefToBinaryData | C | 0..1 | This IE shall be present if the V-SMF or I-SMF has received known N1 SM information from the UE that does not need to be interpreted by the V-SMF or I-SMF. When present, this IE shall reference the n1SmInfoFromUe binary data (see clause 6.1.6.4.4). |  |
| unknownN1SmInfo | RefToBinaryData | C | 0..1 | This IE shall be present if the V-SMF or I-SMF has received unknown N1 SM information from the UE. When present, this IE shall reference the unknownN1SmInfo binary data (see clause 6.1.6.4.4). |  |
| qosFlowsRelNotifyList | array(QosFlowItem) | C | 1..N | This IE shall be present if QoS flows have been released. |  |
| qosFlowsNotifyList | array(QosFlowNotifyItem) | C | 1..N | This IE shall be present if the QoS targets for GBR QoS flow(s) are not fulfilled anymore or when they are fulfilled again. For each GBR QoS flow indicated as not fulfilled anymore, the V-SMF/I-SMF may also indicate an alternative QoS profile which the NG-RAN currently fulfils in the currentQosProfileIndex IE or indicate that the NG-RAN cannot even fulfil the lowest alternative QoS profile. |  |
| NotifyList | array(PduSessionNotifyItem) | C | 1..N | Description of notifications related to the PDU session. This IE shall be present if the NG-RAN has established user plane resources for the PDU session that do not fulfil the User Plane Security Enforcement with a value Preferred, or when the user plane security enforcement is fulfilled again. When present, this IE shall include the notification cause "UP_SEC_NOT_FULFILLED" if at least one of the UP integrity protection or UP ciphering security enforcement is not fulfilled. If the securityResult IE is present in the message, it provides additional details on the security enforcement results. |  |
| epsBearerId | array(EpsBearerId) | C | 0..N | This IE shall be present during an EPS to 5GS handover execution using the N26 interface. When present, it shall contain the list of EPS bearer Id(s) successfully handed over to 5GS. The array shall be empty if no resource was successfully allocated in 5GS for any PDU session. |  |
| hoPreparationIndication | boolean | C | 0..1 | This IE shall be present during an EPS to 5GS handover preparation and handover execution using the N26 interface or during N2 handover execution with I-SMF insertion. When present, it shall be set as follows: - true: an EPS to 5GS handover preparation or N2 handover preparation with I-SMF is in progress; the PGW-C/SMF shall not switch the DL user plane of the PDU session yet. - false: there is no on-going EPS to 5GS handover preparation or N2 handover preparation with I-SMF in progress. If a handover preparation was in progress, the handover has been completed. The PGW-C/SMF shall switch the DL user plane of the PDU session using the N9 tunnel information that has been received in the vcnTunnelInfo or icnTunnelInfo. It shall be set to "true" during an EPS to 5GS handover preparation using the N26 interface. It shall be set to "false" during an EPS to 5GS handover execution using the N26 interface or during N2 handover execution with I-SMF insertion. |  |
| revokeEbiList | array(EpsBearerId) | C | 1..N | This IE shall be present to request the H-SMF or SMF to revoke some EBIs (see clause 4.11.1.4.1 of 3GPP TS 23.502 [3]). When present, it shall contain the EBIs to revoke. |  |
| cause | Cause | C | 0..1 | This IE shall be present and set as specified in clause 5.2.2.8.2.6 during P-CSCF restoration procedure and clause 5.2.2.8.2.3 during 5G-AN requested PDU session resource release procedure. When present, this IE shall indicate the NF Service Consumer cause of the requested modification. |  |
| ngApCause | NgApCause | C | 0..1 | The V-SMF or I-SMF shall include this IE if it received it from the 5G-AN and, for a HR PDU session, if this information is permitted to be sent to the H-SMF operator according to the V-SMF operator's policy. When present, this IE shall indicate the NGAP cause for the requested modification. |  |
| 5gMmCauseValue | 5GMmCause | C | 0..1 | The V-SMF or I-SMF shall include this IE if it received it from the AMF and, for a HR PDU session, if this information is permitted to be sent to the H-SMF operator according to the V-SMF operator's policy. |  |
| alwaysOnRequested | boolean | C | 0..1 | This IE shall be present and set to true if the UE requests to change the PDU session to an always-on PDU session and this is allowed by local policy in the V-SMF or I-SMF. When present, it shall be set as follows: - true: request for an always-on PDU session - false (default): not a request for an always-on PDU session |  |
| epsInterworkingInd | EpsInterworkingIndication | O | 0..1 | This IE may be present if the indication has been received from AMF and, for a HR PDU session, it is allowed to be forwarded to H-SMF by operator configuration. When present, this IE shall indicate whether the PDU session may possibly be moved to EPS and whether N26 interface to be used during EPS interworking procedures. |  |
| secondaryRatUsageReport | array(SecondaryRatUsageReport) | O | 1..N | This IE may be present to report usage data for a secondary RAT for QoS flows. (NOTE 2) |  |
| secondaryRatUsageInfo | array(SecondaryRatUsageInfo) | O | 1..N | This IE may be present to report usage data for a secondary RAT for QoS flows and/or the whole PDU session. |  |
| anTypeCanBeChanged | boolean | C | 0..1 | This IE shall be present and set to true to indicate that the Access Network Type associated to the PDU session can be changed (see clause 5.2.2.8.2.2), during a Service Request procedure (see clauses 4.2.3.2 and 4.3.3.3 of 3GPP TS 23.502 [3])), in response to paging or NAS notification indicating non-3GPP access, when the PDU Session for which the UE was paged or notified is in the List Of Allowed PDU Sessions provided by the UE, and the AMF received N2 SM Information only or N1 SM Container and N2 SM Information from the SMF in step 3a of clause 4.2.3.3 of 3GPP TS 23.502 [3]. When present, it shall be set as follows: - true: the access type of the PDU session can be changed. - false (default): the access type of the PDU session cannot be changed. |  |
| maReleaseInd | MaReleaseIndication | C | 0..1 | This IE shall be present if a MA PDU session is requested to be released over a single access, in the following cases: - when UE/AMF/V-SMF initiates MA PDU session release over one access; or - when UE deregisters from one access. When present, it shall indicate the access to be released. | MAPDU |
| maNwUpgradeInd | boolean | C | 0..1 | This IE shall be present if the PDU session is allowed to be upgraded to MA PDU session (see clause 6.4.2.2 of 3GPP TS 24.501 [7]). When present, it shall be set as follows: - true: the PDU session is allowed to be upgraded to MA PDU session - false (default): the PDU session is not allowed to be upgraded to MA PDU session | MAPDU |
| maRequestInd | boolean | C | 0..1 | This IE shall be present if a MA-PDU session is requested to be established (see clause 4.22.6.3 of 3GPP TS 23.502 [3]). When present, it shall be set as follows: - true: a MA-PDU session is requested - false (default): a MA-PDU session is not requested | MAPDU |
| unavailableAccesslnd | UnavailableAccessIndication | C | 0..1 | This IE shall be present if an access of a MA-PDU session is unavailable (see clause 4.22.7 of 3GPP TS 23.502 [3]). When present, it shall indicate the access that is unavailable. | MAPDU |
| psaInfo | array(PsaInformation) | C | 1..N | This IE shall be present, for a PDU session with an I-SMF, if one or more PSAs UPF are inserted and/or removed by the I-SMF. | DTSSA |
| ulclBpInfo | UlclBpInformation | C | 0..1 | This IE shall be present, for a PDU session with an I-SMF, if an UL CL or BP UPF separate from the local PSA is inserted. | DTSSA |
| n4Info | N4Information | O | 0..1 | This IE may be present if the I-SMF needs to send N4 information (e.g. traffic usage reporting) to the SMF for traffic offloaded at a PSA controlled by an I-SMF. | DTSSA |
| n4InfoExt1 | N4Information | O | 0..1 | This IE may be present if the I-SMF needs to send additional N4 information (e.g. traffic usage reporting) to the SMF for traffic offloaded at a PSA controlled by an I-SMF. | DTSSA |
| n4InfoExt2 | N4Information | O | 0..1 | This IE may be present if the I-SMF needs to send additional N4 information to the SMF (e.g. during a change of PSA). | DTSSA |
| presenceInLadn | PresenceState | C | 0..1 | This IE shall be present during Xn based handover with I-SMF change, if the DNN corresponds to a LADN. When present, it shall be set to "IN" or "OUT" to indicate that the UE is in or out of the LADN service area. | DTSSA |
| vsmfPduSessionUri | Uri | C | 0..1 | This IE shall be present during any procedure when the V-SMF has changed, as specified in clause 4.23.4.3 of 3GPP TS 23.502 [3]. When present, it shall include the callback URI representing the PDU session in the new V-SMF. | DTSSA |
| ismfPduSessionUri | Uri | C | 0..1 | This IE shall be present during any procedure when the I-SMF has changed, as specified in clause 4.23.4.3 of 3GPP TS 23.502 [3]. When present, it shall include the callback URI representing the PDU session in the new I-SMF. | DTSSA |
| vsmfId | NfInstanceId | C | 0..1 | This IE shall be present during any procedure when the V-SMF has changed, as specified in clause 4.23.4.3 of 3GPP TS 23.502 [3]. When present, it shall contain the identifier of the new V-SMF. | DTSSA |
| ismfId | NfInstanceId | C | 0..1 | This IE shall be present during any procedure when the I-SMF has changed, as specified in clause 4.23.4.3 of 3GPP TS 23.502 [3]. When present, it shall contain the identifier of the new I-SMF. | DTSSA |
| vSmfServiceInstanceId | string | O | 0..1 | This IE may be present during any procedure when the V-SMF has changed, as specified in clause 4.23.4.3 of 3GPP TS 23.502 [3]. When present, this IE shall contain the serviceInstanceId of the new V-SMF service instance serving the PDU session. This IE may be used by the H-SMF to identify PDU sessions affected by a failure or restart of the V-SMF service (see clauses 6.2 and 6.3 of 3GPP TS 23.527 [24]). | DTSSA |
| iSmfServiceInstanceId | string | O | 0..1 | This IE may be present during any procedure when the I-SMF has changed, as specified in clause 4.23.4.3 of 3GPP TS 23.502 [3]. When present, this IE shall contain the serviceInstanceId of the new I-SMF service instance serving the PDU session. This IE may be used by the SMF to identify PDU sessions affected by a failure or restart of the I-SMF service (see clauses 6.2 and 6.3 of 3GPP TS 23.527 [24]). | DTSSA |
| dlServingPlmnRateCtl | integer | C | 0..1 | The IE shall be present when the Serving PLMN Rate Control for Downlink data packets has changed since last update to the H-SMF (for HR PDU session) or SMF (for PDU sessions with an I-SMF). When present, this IE shall contain the maximum allowed number of Downlink NAS Data PDUs per deci hour of the serving PLMN, as specified in clause 5.31.14.2 of 3GPP TS 23.501 [2]. If Serving PLMN Rate Control is disabled, the IE shall be set to null value. Minimum: 10 | CIOT |
| dnaiList | array(Dnai) | C | 1..N | This IE shall be present over N16a during UE Triggered Service Request procedure with I-SMF change, Xn based handover and Inter NG-RAN node N2 based handover with I-SMF change (see clauses 4.23.4.3, 4.23.11.3 and 4.23.7.3.3 in 3GPP TS 23.502 [3]). When present, it shall include the list of DNAIs supported by the new I-SMF. | DTSSA |
| supportedFeatures | SupportedFeatures | C | 0..1 | This IE shall be present if the vsmfId or the ismfId is present (i.e. during a change of V-SMF or I-SMF) and at least one optional feature defined in clause 6.1.8 is supported by the new V-SMF or I-SMF. If this IE is absent when the vsmfId or the ismfId is present, the new V-SMF or I-SMF respectively shall be considered as not supporting any optional feature. |  |
| roamingChargingProfile | RoamingChargingProfile | O | 0..1 | This IE may be present during an inter-PLMN V-SMF change. When present, it shall contain the Roaming Charging Profile applicable in the VPLMN (see clauses 5.1.9.1, 5.2.1.7 and 5.2.2.12.2 of 3GPP TS 32.255 [25]). |  |
| moExpDataCounter | MoExpDataCounter | C | 0..1 | This IE shall be present if received from AMF. When present, this IE shall contain the MO Exception Data Counter. | CIOT |
| vplmnQos | VplmnQos | O | 0..1 | When present, this IE shall contain the VPLMN QoS to be applied to the QoS flow with default QoS of the PDU Session. | VQOS |
| securityResult | SecurityResult | C | 0..1 | This IE shall be present if received from NG-RAN. When present, this IE shall contain the Security Result associated to the PDU session. See clause 9.3.1.59 of 3GPP TS 38.413 [9]. |  |
| upSecurityInfo | UpSecurityInfo | C | 0..1 | This IE shall be present if received from NG-RAN during Xn handover procedure (see clause 5.2.2.8.2.16). When present, this IE shall contain the User Plane Security Information associated to the PDU session. See clause 9.3.1.60 of 3GPP TS 38.413 [9]. |  |
| amfNfId | NfInstanceId | C | 0..1 | This IE shall be present if it is received in the Update SM Context request. When present, it shall contain the identifier of the serving AMF. |  |
| guami | Guami | C | 0..1 | This IE shall be present if the amfNfId is present. When present, it shall contain the serving AMF's GUAMI. |  |
| secondaryRatUsageDataReportContainer | array(SecondaryRatUsageDataReportContainer) | C | 1..N | The IE shall be present during an EPS to 5GS handover procedure, if one or more instance of Secondary RAT Usage Data Report IE(s) are present and applicable to the PDU session. The V-/I-SMF may determine to pass or not the information to the (H-)SMF based on operator policy; if the V-/I-SMF determines to pass the information, the V-/I-SMF shall transparently forward this information to the (H-)SMF. When present, it shall contain Base64-encoded characters, encoding the Secondary RAT Usage Data Report in the Forward Relocation Complete Acknowledge message, as specified in Figure 8.132-1 of 3GPP TS 29.274 [16] (starting from octet 1). |  |
| NOTE 1: In shared networks, the PLMN ID that is communicated in this IE shall be that of the selected Core Network Operator. NOTE 2: An SMF complying with this version of the specification should report secondary RAT usage using the secondaryRatUsageInfo attribute that replaces the secondaryRatUsageReport attribute. |  |  |  |  |  |

##### 6.1.6.2.12	Type: HsmfUpdatedData

Table 6.1.6.2.12-1: Definition of type HsmfUpdatedData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n1SmInfoToUe | RefToBinaryData | C | 0..1 | This IE shall be present if the H-SMF/SMF needs to send N1 SM information to the UE that does not need to be interpreted by the V-SMF/I-SMF. When present, this IE shall reference the n1SmInfoToUe binary data (see clause 6.1.6.4.4). |  |
| n4Info | N4Information | O | 0..1 | This IE may be present if the SMF needs to send N4 response information to the I-SMF (e.g. related with traffic usage reporting). | DTSSA |
| n4InfoExt1 | N4Information | O | 0..1 | This IE may be present if the SMF needs to send additional N4 response information to the I-SMF(e.g. related with traffic usage reporting). | DTSSA |
| n4InfoExt2 | N4Information | O | 0..1 | This IE may be present if the SMF needs to send additional N4 response information to the I-SMF (e.g. related with traffic usage reporting). | DTSSA |
| dnaiList | array(Dnai) | C | 1..N | This IE shall be present over N16a during UE Triggered Service Request procedure with I-SMF change, Xn based handover and Inter NG-RAN node N2 based handover with I-SMF change (see clauses 4.23.4.3, 4.23.11.3 and 4.23.7.3.3 in 3GPP TS 23.502 [3]). When present, it shall include the DNAI(s) of interest for this PDU Session. | DTSSA |
| supportedFeatures | SupportedFeatures | C | 0..1 | This IE shall be present if the supportedFeatures IE was received in the request and at least one optional feature defined in clause 6.1.8 is supported by the updated PDU session resource. |  |
| roamingChargingProfile | RoamingChargingProfile | O | 0..1 | This IE may be present during an inter-PLMN V-SMF change. When present, it shall contain the Roaming Charging Profile selected by the HPLMN (see clauses 5.1.9.1, 5.2.1.7 and 5.2.2.12.2 of 3GPP TS 32.255 [25]). |  |
| ipv6MultiHomingInd | boolean | C | 0..1 | This IE shall be present over N16a, if available and an I-SMF has been changed during the following procedures: Registration, Service Request, Xn based handover, Inter NG-RAN node N2 based handover (see clause 4.23 of 3GPP TS 23.502 [3]). When present, it shall be set as follows: - true: IPv6 multi-homing is permitted. - false (default): IPv6 multi-homing is not allowed. | DTSSA |
| upSecurity | UpSecurity | C | 0..1 | This IE shall be present if the "upSecurityInfo" IE was received in the request (i.e. during an Xn handover), and there is a mismatch between security policy received and stored (see clause 5.2.2.8.2.16). When present, this IE shall indicate the security policy for integrity protection and encryption for the user plane of the PDU session. This IE may be present during a handover from non-3GPP access to 3GPP access, to indicate the security policy for integrity protection and encryption for the user plane of the PDU session in the target access type. (NOTE) |  |
| maxIntegrityProtectedDataRateUl | MaxIntegrityProtectedDataRate | C | 0..1 | This IE shall be present if the upSecurity IE is present and indicates that integrity protection is preferred or required. When present, it shall indicate the maximum integrity protected data rate supported by the UE for uplink. (NOTE) |  |
| maxIntegrityProtectedDataRateDl | MaxIntegrityProtectedDataRate | C | 0..1 | This IE shall be present if the upSecurity IE is present and indicates that integrity protection is preferred or required. When present, it shall indicate the maximum integrity protected data rate supported by the UE for downlink. (NOTE) |  |
| qosFlowsSetupList | array(QosFlowSetupItem) | C | 1..N | This IE shall be present during a handover between 3GPP and non-3GPP accesses. When present, it shall contain the set of QoS flow(s) to establish for the PDU session for the target access type. (NOTE) |  |
| sessionAmbr | Ambr | C | 0..1 | This IE shall be present during a handover between 3GPP and non-3GPP accesses. When present, this IE shall contain the Session AMBR authorized for the PDU session for the target access type. (NOTE) |  |
| epsPdnCnxInfo | EpsPdnCnxInfo | C | 0..1 | This IE shall be present during a handover from non-3GPP access to 3GPP access, if the PDU session may be moved to EPS during its lifetime. (NOTE) |  |
| epsBearerInfo | array(EpsBearerInfo) | C | 1..N | This IE shall be present during a handover from non-3GPP access to 3GPP access, if the PDU session may be moved to EPS during its lifetime. When present, it shall include the complete epsBearerInfo IE(s) for all EBIs. (NOTE) |  |
| pti | ProcedureTransactionId | C | 0..1 | This IE shall be present during a handover between 3GPP and non-3GPP accesses. When present, it shall contain the PTI value received in the corresponding request. |  |
| NOTE: During a handover between 3GPP and non-3GPP accesses, the V-SMF or I-SMF shall delete any corresponding information received earlier for the source access type and use the new information received for the target access type. |  |  |  |  |  |

##### 6.1.6.2.13	Type: ReleaseData

Table 6.1.6.2.13-1: Definition of type ReleaseData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| cause | Cause | C | 0..1 | This IE shall be present, if the information is available. When present, this IE shall indicate the NF Service Consumer cause for the requested PDU session release. |  |
| ngApCause | NgApCause | C | 0..1 | This IE shall be present, if the information is available and if this information is permitted to be sent to the H-SMF operator according to the V-SMF operator's policy. When present, this IE shall indicate the NGAP cause for the requested PDU session release. |  |
| 5gMmCauseValue | 5GMmCause | C | 0..1 | The V-SMF shall include this IE if it received it from the AMF and if this information is permitted to be sent to the H-SMF operator according to the V-SMF operator's policy. |  |
| ueLocation | UserLocation | C | 0..1 | This IE shall be present, if available. When present, it shall contain the UE location information (see clause 5.2.3.4). |  |
| ueTimeZone | TimeZone | C | 0..1 | This IE shall be present, if available. When present, it shall contain the UE Time Zone. |  |
| addUeLocation | UserLocation | O | 0..1 | Additional UE location. This IE may be present, if anType previously reported is a non-3GPP access and a valid 3GPP access user location information is available. When present, it shall contain: - the last known 3GPP access user location (see clause 5.2.3.4); and - the timestamp, if available, indicating the UTC time when the addUeLocation information was acquired. |  |
| secondaryRatUsageReport | array(SecondaryRatUsageReport) | O | 1..N | This IE may be present to report usage data for a secondary RAT for QoS flows. (NOTE) |  |
| secondaryRatUsageInfo | array(SecondaryRatUsageInfo) | O | 1..N | This IE may be present to report usage data for a secondary RAT for QoS flows and/or the whole PDU session. |  |
| n4Info | N4Information | O | 0..1 | This IE may be present if the I-SMF needs to send N4 information (e.g. traffic usage reporting) to the SMF for traffic offloaded at a PSA controlled by an I-SMF. | DTSSA |
| n4InfoExt1 | N4Information | O | 0..1 | This IE may be present if the I-SMF needs to send additional N4 information (e.g. traffic usage reporting) to the SMF for traffic offloaded at a PSA controlled by an I-SMF. | DTSSA |
| n4InfoExt2 | N4Information | O | 0..1 | This IE may be present if the I-SMF needs to send additional N4 information (e.g. traffic usage reporting) to the SMF for traffic offloaded at a PSA controlled by an I-SMF. | DTSSA |
| NOTE: An SMF complying with this version of the specification should report secondary RAT usage using the secondaryRatUsageInfo attribute that replaces the secondaryRatUsageReport attribute. |  |  |  |  |  |

##### 6.1.6.2.14	Type: HsmfUpdateError

Table 6.1.6.2.14-1: Definition of type HsmfUpdateError

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| error | ProblemDetails | M | 1 | More information on the error shall be provided in the "cause" attribute of the "ProblemDetails" structure. |
| pti | ProcedureTransactionId | C | 0..1 | This IE shall be present if this is a response sent to a UE requested PDU session modification. When present, it shall contain the PTI value received in the corresponding request. |
| n1smCause | string | C | 0..1 | This IE shall be present if the request included n1SmInfoFromUe. When present, it shall contain the 5GSM cause the H-SMF proposes the V-SMF to return to the UE. It shall be encoded as two characters in hexadecimal representation with each character taking a value of "0" to "9" or "A" to "F", and represent the cause value of the 5GSM cause IE specified in clause 9.11.4.2 of 3GPP TS 24.501 [7]. Pattern: "^[A-F0-9]{2}$" Example: the cause "Invalid mandatory information" shall be encoded as "60". See NOTE. |
| n1SmInfoToUe | RefToBinaryData | C | 0..1 | This IE shall be present if the H-SMF needs to send N1 SM information to the UE that does not need to be interpreted by the V-SMF. When present, this IE shall reference the n1SmInfoToUe binary data (see clause 6.1.6.4.4). |
| backOffTimer | DurationSec | O | 0..1 | When present, this IE shall indicate a Back-off timer value, in seconds, that the V-SMF may use when rejecting the NAS message towards the UE. |
| recoveryTime | DateTime | O | 0..1 | Timestamp when the H-SMF service instance was (re)started (see clause 6.3 of 3GPP TS 23.527 [24]). |
| NOTE: This IE contains information that the V-SMF shall transfer to the UE without interpretation. It is sent as a separate IE rather than within the n1SmInfoToUE binary data because the 5GSM cause IE is defined as a "V" IE (i.e. without a Type field) in the NAS PDU Session Modification Reject message. |  |  |  |  |

##### 6.1.6.2.15	Type: VsmfUpdateData

Table 6.1.6.2.15-1: Definition of type VsmfUpdateData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| requestIndication | RequestIndication | M | 1 | This IE shall indicate the request type. |  |
| sessionAmbr | Ambr | C | 1 | This IE shall be present if the Session AMBR authorized for the PDU session is modified. When present, it shall contain the new Session AMBR authorized for the PDU session. |  |
| qosFlowsAddModRequestList | array(QosFlowAddModifyRequestItem) | C | 1..N | This IE shall be present if QoS flows are requested to be established or modified. |  |
| qosFlowsRelRequestList | array(QosFlowReleaseRequestItem) | C | 1..N | This IE shall be present if QoS flows are requested to be released. |  |
| epsBearerInfo | array(EpsBearerInfo) | C | 1..N | This IE shall be present if the PDU session may be moved to EPS during its lifetime and the ePSBearerInfo has changed. When present, it shall only include epsBearerInfo IE(s) for new EBI or for EBIs for which the epsBearerInfo has changed. The complete epsBearerInfo shall be provided for an EBI that is included (i.e. the epsBearerInfo newly received for a given EBI replaces any epsBearerInfo previously received for this EBI). |  |
| assignEbiList | array(Arp) | C | 1..N | This IE shall be present if the H-SMF requests EBIs to be assigned. |  |
| revokeEbiList | array(EpsBearerId) | C | 1..N | This IE shall be present if the H-SMF/SMF requests the V-SMF/I-SMF to revoke some EBI(s). When present, it shall contain the EBIs to revoke. |  |
| modifiedEbiList | array(EbiArpMapping) | C | 1..N | This IE shall be present if a PDU session modification procedure resulted in the change of ARP for a QoS flow that was already allocated an EBI. |  |
| pti | ProcedureTransactionId | C | 0..1 | This IE shall be present if the request is sent in response to a UE requested PDU session modification or release. When present, it shall contain the PTI value received in the corresponding request. |  |
| n1SmInfoToUe | RefToBinaryData | C | 0..1 | This IE shall be present if the H-SMF/SMF needs to send N1 SM information to the UE that does not need to be interpreted by the V-SMF/I-SMF. When present, this IE shall reference the n1SmInfoToUe binary data (see clause 6.1.6.4.4). |  |
| alwaysOnGranted | boolean | C | 0..1 | This IE shall be present if: - an alwaysOnRequested IE was received in an earlier V-SMF/I-SMF initiated Update request to change the PDU session to an always-on PDU session; or - the H-SMF/SMF determines, based on local policy, that the PDU session needs to be established as an always-on PDU session. When present, it shall be set as follows: - true: always-on PDU session granted. - false (default): always-on PDU session not granted. |  |
| hsmfPduSessionUri | Uri | C | 0..1 | This IE shall be included if: - an Update Request is sent to the V-SMF/I-SMF before the Create Response (e.g. for EPS bearer ID allocation as specified in clause 4.11.1.4.1 of 3GPP TS 23.502 [3], or for Secondary authorization/authentication as specified in clause 4.3.2.3 of 3GPP TS 23.502 [3]), and - the H-SMF PDU Session Resource URI has not been previously provided to the V-SMF/I-SMF. This IE shall not be included otherwise. When present, this IE shall include the URI representing the PDU session resource in the H-SMF. |  |
| supportedFeatures | SupportedFeatures | C | 0..1 | This IE shall be present if "hsmfPduSessionUri" IE is present and at least one optional feature defined in clause 6.1.8 is supported. |  |
| cause | Cause | O | 0..1 | When present, this IE shall indicate the cause for the requested modification. |  |
| n1smCause | string | O | 0..1 | When present, this IE shall contain the 5GSM cause the H-SMF proposes the V-SMF/I-SMF to send to the UE. It shall be encoded as two characters in hexadecimal representation with each character taking a value of "0" to "9" or "A" to "F", and represent the cause value of the 5GSM cause IE specified in clause 9.11.4.2 of 3GPP TS 24.501 [7]. Example: the cause "Invalid mandatory information" shall be encoded as "60". See NOTE. |  |
| backOffTimer | DurationSec | O | 0..1 | When present, this IE shall indicate a Back-off timer value, in seconds, that the V-SMF/I-SMF may use when sending the NAS message (PDU Session Release Command) towards the UE. |  |
| maReleaseInd | MaReleaseIndication | C | 0..1 | This IE shall be present if one access of a MA PDU session is to be released, when H-SMF or SMF initiates MA PDU session release over one access. When present, it shall indicate the access requested to be released. | MAPDU |
| maAcceptedInd | boolean | C | 0..1 | This IE shall be present if a request to modify a single access PDU session into a MA PDU session was accepted (see clause 4.22.6.3 of 3GPP TS 23.502 [3]). When present, it shall be set as follows: - true: MA PDU session - false (default): single access PDU session | MAPDU |
| additionalCnTunnelInfo | TunnelInfo | C | 0..1 | This IE shall be present for a MA-PDU session if the UE requested to establish resources for a MA PDU session over the other access. When present, it shall contain additional N9 tunnel information of the UPF controlled by the H-SMF or SMF. | MAPDU |
| dnaiList | array(Dnai) | C | 0..N | This IE shall be present if received from PCF during I-SMF Related Procedures with PCF (see clause 4.23.6.2 in 3GPP TS 23.502 [3]). When present, the IE shall include a list of DNAI(s) the SMF deems relevant for the PDU Session. If this IE is not present, the I-SMF shall consider that the dnaiList has not changed. If there is no more DNAI of interest for the PDU session, the dnaiList attribute shall be present and be encoded as an empty array. | DTSSA |
| n4Info | N4Information | O | 0..1 | This IE may be present if the SMF needs to send N4 information to the I-SMF for the control of traffic offloaded at a PSA/BP/ULCL controlled by an I-SMF. | DTSSA |
| n4InfoExt1 | N4Information | O | 0..1 | This IE may be present if the SMF needs to send additional N4 information to the I-SMF for the control of traffic offloaded at a PSA/BP/ULCL controlled by an I-SMF. | DTSSA |
| n4InfoExt2 | N4Information | O | 0..1 | This IE may be present if the SMF needs to send additional N4 information to the I-SMF for the control of traffic offloaded at a PSA/BP/ULCL controlled by an I-SMF (e.g. during a change of PSA). | DTSSA |
| n4InfoExt3 | N4Information | O | 0..1 | This IE may be present if the SMF needs to send additional N4 information to the I-SMF for the control of traffic offloaded at a PSA/BP/ULCL controlled by an I-SMF (e.g. during simultaneous change of BP/ULCL and PSA). | SCPBU |
| smallDataRateControlEnabled | boolean | C | 0..1 | This IE shall be present if the applicability of small data rate control on the PDU session changes. When present, it shall be set as follows: - true: small data rate control is applicable. - false: small data rate control is not applicable. | CIOT |
| qosMonitoringInfo | QosMonitoringInfo | C | 0..1 | This IE may be present if QoS monitoring has been activated for at least one QoS flow of the PDU session (see the qosMonitoringReq attribute in clause 6.1.6.2.22). | DTSSA |
| epsPdnCnxInfo | EpsPdnCnxInfo | C | 0..1 | This IE shall be present if the PDU session may be moved to EPS during its lifetime and the EpsInterworkingIndication is changed to "WITH_N26". |  |
| NOTE: This IE contains information that the V-SMF shall transfer to the UE without interpretation. It is sent as a separate IE rather than within the n1SmInfoToUE binary data because the 5GSM cause IE is defined as a "V" IE (i.e. without a Type field) in the NAS PDU Session Release Command message. |  |  |  |  |  |

##### 6.1.6.2.16	Type: VsmfUpdatedData

Table 6.1.6.2.16-1: Definition of type VsmfUpdatedData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| qosFlowsAddModList | array(QosFlowItem) | C | 1..N | This IE shall be present if QoS flows have been successfully established or modified. For each GBR QoS flow, the V-SMF/I-SMF may also indicate an alternative QoS profile which the NG-RAN currently fulfils in the currentQosProfileIndex IE or indicate that the NG-RAN cannot even fulfil the lowest alternative QoS profile. |  |
| qosFlowsRelList | array(QosFlowItem) | C | 1..N | This IE shall be present if QoS flows have been successfully released. |  |
| qosFlowsFailedtoAddModList | array(QosFlowItem) | C | 1..N | This IE shall be present if QoS flows failed to be established or modified. |  |
| qosFlowsFailedtoRelList | array(QosFlowItem) | C | 1..N | This IE shall be present if QoS flows failed to be released. |  |
| n1SmInfoFromUe | RefToBinaryData | C | 0..1 | This IE shall be present if the V-SMF/I-SMF has received known N1 SM information from the UE that does not need to be interpreted by the V-SMF/I-SMF. When present, this IE shall reference the n1SmInfoFromUe binary data (see clause 6.1.6.4.4). |  |
| unknownN1SmInfo | RefToBinaryData | C | 0..1 | This IE shall be present if the V-SMF/I-SMF has received unknown N1 SM information from the UE. When present, this IE shall reference the unknownN1SmInfo binary data (see clause 6.1.6.4.4). |  |
| ueLocation | UserLocation | C | 0..1 | This IE shall be present if it is available and QoS flows have been successfully established, modified or released. When present, this IE shall contain the UE location information (see clause 5.2.3.4). |  |
| ueTimeZone | TimeZone | C | 0..1 | This IE shall be present if it is available and QoS flows have been successfully established, modified or released. When present, this IE shall contain the new UE Time Zone. |  |
| addUeLocation | UserLocation | O | 0..1 | Additional UE location. This IE may be present, if anType previously reported is a non-3GPP access and a valid 3GPP access user location information is available. When present, it shall contain: - the last known 3GPP access user location (see clause 5.2.3.4); and - the timestamp, if available, indicating the UTC time when the addUeLocation information was acquired. |  |
| assignedEbiList | array(EbiArpMapping) | C | 1..N | This IE shall be present if the AMF assigned the requested EBI(s). When present, it shall contain the EBIs that were successfully assigned. |  |
| failedToAssignEbiList | array(Arp) | C | 1..N | This IE shall be present if the AMF failed to assign EBIs for a set of ARPs. |  |
| releasedEbiList | array(EpsBearerId) | C | 1..N | This IE shall be present if the NF Service Consumer requested the revoke EBI(s) or if the AMF revoked already assigned EBI(s) for this PDU session towards the V-SMF. This IE shall contain the list of EBI(s) released for this PDU session at the AMF. |  |
| secondaryRatUsageReport | array(SecondaryRatUsageReport) | O | 1..N | This IE may be present to report usage data for a secondary RAT for QoS flows. (NOTE) |  |
| secondaryRatUsageInfo | array(SecondaryRatUsageInfo) | O | 1..N | This IE may be present to report usage data for a secondary RAT for QoS flows and/or the whole PDU session. |  |
| n4Info | N4Information | O | 0..1 | This IE may be present if the I-SMF needs to send N4 response information to the SMF for the control of traffic offloaded at a PSA/BP/ULCL controlled by an I-SMF. | DTSSA |
| n4InfoExt1 | N4Information | O | 0..1 | This IE may be present if the I-SMF needs to send additional N4 response information to the SMF for the control of traffic offloaded at a PSA/BP/ULCL controlled by an I-SMF. | DTSSA |
| n4InfoExt2 | N4Information | O | 0..1 | This IE may be present if the I-SMF needs to send additional N4 response information to the SMF for the control of traffic offloaded at a PSA/BP/ULCL controlled by an I-SMF (e.g. during a change of PSA). | DTSSA |
| n4InfoExt3 | N4Information | O | 0..1 | This IE may be present if the I-SMF needs to send additional N4 response information to the SMF for the control of traffic offloaded at a PSA/BP/ULCL controlled by an I-SMF (e.g. during simultaneous change of BP/ULCL and PSA). | SCPBU |
| NOTE: An SMF complying with this version of the specification should report secondary RAT usage using the secondaryRatUsageInfo attribute that replaces the secondaryRatUsageReport attribute. |  |  |  |  |  |

##### 6.1.6.2.17	Type: StatusNotification

Table 6.1.6.2.17-1: Definition of type StatusNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| statusInfo | StatusInfo | M | 1 | This IE shall contain status information about the PDU session. |  |
| smallDataRateStatus | SmallDataRateStatus | C | 0..1 | This IE shall be present, if the NF Service Consumer has indicated support of CIoT and if the status is available. When present, it shall indicate the current small data rate control status for the PDU session. | CIOT |
| apnRateStatus | ApnRateStatus | C | 0..1 | This IE shall be present, if the NF Service Consumer has indicated support of CIoT and if the status is available. When present, it shall indicate the current APN rate control status for the PDN connection (APN rates are shared by all PDN connections of the UE to this APN). | CIOT |
| targetDnaiInfo | TargetDnaiInfo | C | 0..1 | This IE shall be present if the I-SMF selection for the current PDU session, or SMF selection during PDU Session re-establishment for SSC mode 2/3 is needed. When present, this IE shall include the target DNAI Information. | EEDGE |

##### 6.1.6.2.18	Type: QosFlowItem

Table 6.1.6.2.18-1: Definition of type QosFlowItem

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| qfi | Qfi | M | 1 | This IE shall contain the QoS Flow Identifier. |
| cause | Cause | O | 0..1 | When present, this IE shall contain cause information. |
| currentQosProfileIndex | Integer | C | 0..1 | When present, this IE shall indicate the index of the currently fulfilled alternative QoS profile. This IE shall not be present if the nullQoSProfileIndex IE is present. |
| nullQoSProfileIndex | boolean | C | 0..1 | This IE shall be present and set to "true" if the NG-RAN cannot even fulfil the lowest alternative QoS profile. This IE shall not be present if the currentQosProfileIndex IE is present. |

##### 6.1.6.2.19	Type: QosFlowSetupItem

Table 6.1.6.2.19-1: Definition of type QosFlowSetupItem

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| qfi | Qfi | M | 1 | This IE shall contain the QoS Flow Identifier. |  |
| qosRules | Bytes | M | 1 | This IE shall contain the QoS Rule(s) associated to the QoS flow to be sent to the UE. It shall be encoded as the Qos rules IE specified in clause 9.11.4.13 of 3GPP TS 24.501 [7] (starting from octet 4). |  |
| ebi | EpsBearerId | C | 0..1 | This IE shall be included when an EPS Bearer ID is allocated for the QoS Flow for interworking with EPS. When present, this IE shall contain the allocated EPS Bearer ID. |  |
| qosFlowDescription | Bytes | O | 0..1 | When present, this IE shall contain the description of the QoS Flow level Qos parameters to be sent to the UE. It shall be encoded as the Qos flow descriptions IE specified in clause 9.11.4.12 of 3GPP TS 24.501 [7] (starting from octet 1), encoding one single Qos flow description for the QoS flow to be set up. |  |
| qosFlowProfile | QosFlowProfile | O | 0..1 | When present, this IE shall contain the description of the QoS Flow level Qos parameters. |  |
| associatedAnType | QosFlowAccessType | O | 0..1 | This IE may be present if the QoS Flow belongs to MA PDU session. When present, this IE shall contain the indicated access type associated with the QoS Flow. | MAPDU |

##### 6.1.6.2.20	Type: QosFlowAddModifyRequestItem

Table 6.1.6.2.20-1: Definition of type QosFlowAddModifyRequestItem

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| qfi | Qfi | M | 1 | This IE shall contain the QoS Flow Identifier. |  |
| ebi | EpsBearerId | C | 0..1 | This IE shall be included when the EPS Bearer ID associated with a QoS Flow is modified. When present, this IE shall contain the EPS Bearer ID. |  |
| qosRules | Bytes | O | 0..1 | When present, this IE shall contain the QoS Rule(s) to be sent to the UE. It shall be encoded as the Qos rules IE specified in clause 9.11.4.13 of 3GPP TS 24.501 [7] (starting from octet 4). |  |
| qosFlowDescription | Bytes | O | 0..1 | When present, this IE shall contain the description of the QoS Flow level Qos parameters to be sent to the UE. It shall be encoded as the Qos flow descriptions IE specified in clause 9.11.4.12 of 3GPP TS 24.501 [7] (starting from octet 1), encoding one single Qos flow description for the QoS flow to be added or modified. |  |
| qosFlowProfile | QosFlowProfile | O | 0..1 | When present, this IE shall contain the description of the QoS Flow level QoS parameters. When modifying a QoS flow, the IE shall only contain the QoS Flow profile's attributes which are modified. |  |
| associatedAnType | QosFlowAccessType | O | 0..1 | This IE may be present if the QoS Flow belongs to MA PDU session. When present, this IE shall contain the indicated access type associated with the QoS Flow. | MAPDU |

##### 6.1.6.2.21	Type: QosFlowReleaseRequestItem

Table 6.1.6.2.21-1: Definition of type QosFlowReleaseRequestItem

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| qfi | Qfi | M | 1 | This IE shall contain the QoS Flow Identifier. |
| qosRules | Bytes | O | 0..1 | When present, this IE shall contain the QoS Rule(s) to be sent to the UE. It shall be encoded as the Qos rules IE specified in clause 9.11.4.13 of 3GPP TS 24.501 [7] (starting from octet 4). |
| qosFlowDescription | Bytes | O | 0..1 | When present, this IE shall contain the description of the QoS Flow level Qos parameters to be sent to the UE. It shall be encoded as the Qos flow descriptions IE specified in clause 9.11.4.12 of 3GPP TS 24.501 [7] (starting from octet 1), encoding one single Qos flow description for the QoS flow to be released. |

##### 6.1.6.2.22	Type: QosFlowProfile

Table 6.1.6.2.22-1: Definition of type QosFlowProfile

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| 5qi | 5Qi | M | 1 | This IE shall contain the 5G QoS Identifier (5QI) of the QoS flow. |
| nonDynamic5Qi | NonDynamic5Qi | C | 0..1 | When present, this IE shall indicate the QoS Characteristics for a standardized or pre-configured 5QI for downlink and uplink. See NOTE 1. |
| dynamic5Qi | Dynamic5Qi | C | 0..1 | When present, this IE shall indicate the QoS Characteristics for a Non-standardised or not pre-configured 5QI for downlink and uplink. See NOTE 1. |
| arp | Arp | C | 0..1 | This IE shall be present when establishing a QoS flow; it may be present when modifying a QoS flow. When present, this IE shall contain the Allocation and Retention Priority (ARP) assigned to the QoS flow. |
| gbrQosFlowInfo | GbrQosFlowInformation | C | 0..1 | This IE shall be present when establishing a GBR QoS flow or if the GBR QoS flow information is modified. |
| rqa | ReflectiveQoSAttribute | O | 0..1 | This IE may be present for a non-GBR QoS flow and it shall be ignored otherwise. When present, it shall indicate whether certain traffic on this QoS flow may be subject to Reflective QoS. |
| additionalQosFlowInfo | AdditionalQosFlowInfo | O | 0..1 | This IE may be present for a non-GBR QoS flow. When present, this IE indicates that traffic for this QoS flow is likely to appear more often than traffic for other flows established for the PDU session. See clause 9.3.1.12 of 3GPP TS 38.413 [9]. |
| qosMonitoringReq | QosMonitoringReq | O | 0..1 | This IE may be present to indicate the measurement of UL, or DL, or both UL/DL delays for the associated QoS flow. This IE may also be used to indicate the stop of corresponding measurement, by setting the value to "NONE". See clause 9.3.1.12 of 3GPP TS 38.413 [9]. |
| qosRepPeriod | DurationSec | O | 0..1 | This IE should be present if QoS monitoring is required. When present, this IE shall indicate the reporting period. See clause 4.23.5.3 of 3GPP TS 23.502 [3]. |
| NOTE 1: Either the nonDynamic5Qi IE or the dynamic5Qi IE may be present when establishing a QoS flow. Either the nonDynamic5Qi IE or the dynamic5Qi IE may be present when modifying a QoS flow; when present, the received nonDynamic5Qi IE or dynamic5Qi IE shall replace any value received previously for this IE. |  |  |  |  |

##### 6.1.6.2.23	Type: GbrQosFlowInformation

Table 6.1.6.2.23-1: Definition of type GbrQosFlowInformation

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| maxFbrDl | BitRate | M | 1 | This IE shall contain the Maximum Bit Rate in Downlink. See 3GPP TS 23.501 [2]. |
| maxFbrUl | BitRate | M | 1 | This IE shall contain the Maximum Bit Rate in Uplink. See 3GPP TS 23.501 [2]. |
| guaFbrDl | BitRate | M | 1 | This IE shall contain the Guaranteed Bit Rate in Downlink. See 3GPP TS 23.501 [2]. |
| guaFbrUl | BitRate | M | 1 | This IE shall contain the Guaranteed Bit Rate in Uplink. See 3GPP TS 23.501 [2]. |
| notifControl | NotificationControl | O | 0..1 | When present, this IE shall indicate whether notifications are requested from the RAN when the GFBR can no longer (or again) be fulfilled for a QoS flow during the lifetime of the QoS flow. See 3GPP TS 23.501 [2]. |
| maxPacketLossRateDl | PacketLossRate | O | 0..1 | When present, this IE shall indicate the maximum rate for lost packets that can be tolerated in the downlink direction. See 3GPP TS 23.501 [2]. |
| maxPacketLossRateUl | PacketLossRate | O | 0..1 | When present, this IE shall indicate the maximum rate for lost packets that can be tolerated in the Uplink direction. See 3GPP TS 23.501 [2]. |
| alternativeQosProfileList | array(AlternativeQosProfile) | O | 0..N | When present, this IE shall indicate alternative QoS profiles for the QoS flow. An empty array shall be interpreted as a request to delete any list of alternative QoS profiles associated with this QoS flow. |

##### 6.1.6.2.24	Type: QosFlowNotifyItem

Table 6.1.6.2.24-1: Definition of type QosFlowNotifyItem

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| qfi | Qfi | M | 1 | This IE shall contain the QoS Flow Identifier. |
| notificationCause | NotificationCause | M | 1 |  |
| currentQosProfileIndex | Integer | C | 0..1 | When present, this IE shall indicate the index of the currently fulfilled alternative QoS profile. This IE shall not be present if the nullQoSProfileIndex IE is present. |
| nullQoSProfileIndex | boolean | C | 0..1 | This IE shall be present and set to "true" if the NG-RAN cannot even fulfil the lowest alternative QoS profile. This IE shall not be present if the currentQosProfileIndex IE is present. |

##### 6.1.6.2.25	Type: Void

##### 6.1.6.2.26	Type: Void

##### 6.1.6.2.27	Type: SmContextRetrievedData

Table 6.1.6.2.27-1: Definition of type SmContextRetrievedData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ueEpsPdnConnection | EpsPdnCnxContainer | M | 1 | This IE shall contain an MME/SGSN UE EPS PDN Connection including the mapped EPS bearer context(s), if the SM context type was not present in the request or if it was present and indicated a request to retrieve the UE EPS PDN Connection. This IE shall be set to an empty string if the SM context type was present in the request and indicated a request to retrieve the complete SM context or the AF Coordination Information. |  |
| smContext | SmContext | C | 0..1 | This IE shall be present if the SM context type was present in the request and indicated a request to retrieve the complete SM context. | DTSSA |
| smallDataRateStatus | SmallDataRateStatus | C | 0..1 | This IE shall be present during N26 based Interworking Procedures, if in the request the smContextType is set to "EPS_PDN_CONNECTION" and if the status is available (see clauses 4.11.1.1 and 4.11.1.3.2 in 3GPP TS 23.502 [3]). When present, it shall indicate the small data rate control status for the PDU session. | CIOT |
| apnRateStatus | ApnRateStatus | C | 0..1 | This IE shall be present during N26 based Interworking Procedures, if in the request the smContextType is set to "EPS_PDN_CONNECTION" and if the status is available (see clauses 4.11.1.1 and 4.11.1.3.2 in 3GPP TS 23.502 [3]). When present, it shall indicate the APN rate control status for the PDN connection (APN rates are shared by all PDN connections of the UE to this APN). | CIOT |
| dlDataWaitingInd | boolean | C | 0..1 | This IE shall be present, if the SM context type was not present in the request or if it was present and indicated a request to retrieve the UE EPS PDN Connection, and if downlink data buffered in the SMF/UPF needs to be forwarded to EPS (see clause 4.11.1.3.2A of 3GPP TS 23.502 [3]). When present, it shall be set as follows: - true: DL data needs to be sent to the UE; - false (default): no DL data needs to be sent to the UE. | CIOT |
| afCoordinationInfo | AfCoordinationInfo | C | 0..1 | This IE shall be present if the SM context type was present in the request and indicated a request to retrieve the AF Coordination Information. | EEDGE |

##### 6.1.6.2.28	Type: TunnelInfo

Table 6.1.6.2.28-1: Definition of type TunnelInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ipv4Addr | Ipv4Addr | C | 0..1 | When present, this IE shall contain the GTP tunnel IPv4 address. At least one of the ipv4Addr or ipv6Addr shall be present. Both may be present. |  |
| ipv6Addr | Ipv6Addr | C | 0..1 | When present, this IE shall contain the GTP tunnel IPv6 address. At least one of the ipv4Addr or ipv6Addr shall be present. Both may be present. |  |
| gtpTeid | Teid | M | 1 | This IE shall contain the 4-octet GTP tunnel endpoint identifier. If both ipv4Addr and ipv6Addr are present, the TEID shall be shared by both addresses. |  |
| anType | AccessType | C | 0..1 | This IE shall be present over N16a/N16 in MA PDU session scenarios, to indicate the access type associated to the N9 tunnel. | MAPDU |

##### 6.1.6.2.29	Type: StatusInfo

Table 6.1.6.2.29-1: Definition of type StatusInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| resourceStatus | ResourceStatus | M | 1 | This IE shall indicate the status of the SM context or PDU session resource. |  |
| cause | Cause | O | 0..1 | When present, this IE shall indicate the cause for the resource status change. |  |
| cnAssistedRanPara | CnAssistedRanPara | C | 0..1 | This attribute shall be present when the cause value is "CN_ASSISTED_RAN_PARAMETER_TUNING". When present, this IE shall include the SMF derived CN assisted RAN parameters tuning. | CARPT |
| anType | AccessType | C | 0..1 | This IE shall indicate the access type of PDU session. | HOFAIL |

##### 6.1.6.2.30	Type: VsmfUpdateError

Table 6.1.6.2.30-1: Definition of type VsmfUpdateError

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| error | ExtProblemDetails | M | 1 | More information on the error shall be provided in the "cause" attribute of the "ProblemDetails" structure. |  |
| pti | ProcedureTransactionId | C | 0..1 | This IE shall be present if available. When present, it shall contain the PTI value received from the UE. |  |
| n1smCause | string | C | 0..1 | This IE shall be present if available. When present, it shall contain the 5GSM cause received from the UE. It shall be encoded as two characters in hexadecimal representation with each character taking a value of "0" to "9" or "A" to "F", and represent the cause value of the 5GSM cause IE specified in clause 9.11.4.2 of 3GPP TS 24.501 [7]. Pattern: "^[A-F0-9]{2}$" Example: the cause "Invalid mandatory information" shall be encoded as "60". See NOTE. |  |
| n1SmInfoFromUe | RefToBinaryData | C | 0..1 | This IE shall be present if the V-SMF has received known N1 SM information from the UE that does not need to be interpreted by the V-SMF. When present, this IE shall reference the n1SmInfoFromUe binary data (see clause 6.1.6.4.4). |  |
| unknownN1SmInfo | RefToBinaryData | C | 0..1 | This IE shall be present if the V-SMF has received unknown N1 SM information from the UE. When present, this IE shall reference the unknownN1SmInfo binary data (see clause 6.1.6.4.4). |  |
| failedToAssignEbiList | array(Arp) | C | 1..N | This IE shall be present if the AMF failed to assign the requested EBIs. |  |
| ngApCause | NgApCause | C | 0..1 | This IE shall be present, if the information is available and if this information is permitted to be sent to the H-SMF operator according to the V-SMF operator's policy. |  |
| 5gMmCauseValue | 5GMmCause | C | 0..1 | The V-SMF shall include this IE if it received it from the AMF and if this information is permitted to be sent to the H-SMF operator according to the V-SMF operator's policy. |  |
| recoveryTime | DateTime | O | 0..1 | Timestamp when the V-SMF service instance was (re)started (see clause 6.3 of 3GPP TS 23.527 [24]). |  |
| n4Info | N4Information | O | 0..1 | This IE may be present if the I-SMF needs to send N4 response information to the SMF for the control of traffic offloaded at a PSA/BP/ULCL controlled by an I-SMF. | DTSSA |
| n4InfoExt1 | N4Information | O | 0..1 | This IE may be present if the I-SMF needs to send additional N4 response information to the SMF for the control of traffic offloaded at a PSA/BP/ULCL controlled by an I-SMF. | DTSSA |
| n4InfoExt2 | N4Information | O | 0..1 | This IE may be present if the I-SMF needs to send additional N4 response information to the SMF for the control of traffic offloaded at a PSA/BP/ULCL controlled by an I-SMF (e.g. during a change of PSA). | DTSSA |
| n4InfoExt3 | N4Information | O | 0..1 | This IE may be present if the I-SMF needs to send additional N4 response information to the SMF for the control of traffic offloaded at a PSA/BP/ULCL controlled by an I-SMF (e.g. during simultaneous change of BP/ULCL and PSA). | SCPBU |
| NOTE: This IE is sent as a separate IE rather than within the n1SmInfoFromUE binary data because the 5GSM cause IE is defined as a "V" IE (i.e. without a Type field) in the NAS PDU Session Modification Command Reject message. |  |  |  |  |  |

##### 6.1.6.2.31	Type: EpsPdnCnxInfo

Table 6.1.6.2.31-1: Definition of type EpsPdnCnxInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| pgwS8cFteid | Bytes | M | 1 | Base64-encoded characters, encoding the PGW S8 F-TEID for Control Plane as specified in Figure 8.22-1 of 3GPP TS 29.274 [16] (starting from octet 1). |
| pgwNodeName | Bytes | C | 0..1 | Base64-encoded characters, encoding the PGW FQDN IE as specified in Figure 8.66-1 of 3GPP TS 29.274 [16] (starting from octet 1). It shall be present, if it is available. |
| linkedBearerId | EpsBearerId | C | 0..1 | An implementation complying with this version of the specification shall include this attribute and set it to the default bearer ID associated with the PDU session moved to EPS. |

##### 6.1.6.2.32	Type: EpsBearerInfo

Table 6.1.6.2.32-1: Definition of type EpsBearerInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| ebi | EpsBearerId | M | 1 | EPS Bearer ID |
| pgwS8uFteid | Bytes | M | 1 | Base64-encoded characters, encoding the PGW S8 F-TEID for User Plane as specified in Figure 8.22-1 of 3GPP TS 29.274 [16] (starting from octet 1). |
| bearerLevelQoS | Bytes | M | 1 | Base64-encoded characters, encoding the Bearer QoS IE as specified in Figure 8.15-1 of 3GPP TS 29.274 [16] (starting from octet 1). |

##### 6.1.6.2.33	Type: PduSessionNotifyItem

Table 6.1.6.2.33-1: Definition of type PduSessionNotifyItem

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| notificationCause | NotificationCause | M | 1 |  |

##### 6.1.6.2.34	Type: EbiArpMapping

Table 6.1.6.2.34-1: Definition of type EbiArpMapping

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| epsBearerId | EpsBearerId | M | 1 | This IE shall contain the EPS bearer identities. |
| arp | Arp | M | 1 | This IE shall contain the ARP corresponding to the EBI. |

##### 6.1.6.2.35	Type: SmContextCreateError

Table 6.1.6.2.35-1: Definition of type SmContextCreateError

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| error | ExtProblemDetails | M | 1 | More information on the error shall be provided in the "cause" attribute of the "ProblemDetails" structure. |
| n1SmMsg | RefToBinaryData | C | 0..1 | This IE shall be present, if an N1 SM information is received in the request and the SMF is able to return N1 SM information to the UE. When present, it shall reference the N1 SM Message binary data (see clause 6.1.6.4.2). |
| n2SmInfo | RefToBinaryData | C | 0..1 | This IE shall be present, if N2 SM information needs to be returned to the NG-RAN during Xn based handover procedure with I-SMF/V-SMF insertion, change or removal. When present, it shall reference the N2 SM Message binary data (see clause 6.1.6.4.3). |
| n2SmInfoType | N2SmInfoType | C | 0..1 | This IE shall be present if "n2SmInfo" attribute is present. When present, this IE shall indicate the NG AP IE type for the NG AP SMF related IE container carried in "n2SmInfo" attribute. |
| recoveryTime | DateTime | O | 0..1 | Timestamp when the SMF service instance was (re)started (see clause 6.3 of 3GPP TS 23.527 [24]). |

##### 6.1.6.2.36	Type: SmContextUpdateError

Table 6.1.6.2.36-1: Definition of type SmContextUpdateError

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| error | ExtProblemDetails | M | 1 | More information on the error shall be provided in the "cause" attribute of the "ProblemDetails" structure. |
| n1SmMsg | RefToBinaryData | C | 0..1 | This IE shall be present, if N1 SM Information needs to be returned to the UE. When present, it shall reference the N1 SM Message binary data (see clause 6.1.6.4.2). |
| n2SmInfo | RefToBinaryData | C | 0..1 | This IE shall be present, if N2 SM information needs to be returned to the NG-RAN. When present, it shall reference the N2 SM Message binary data (see clause 6.1.6.4.3). |
| n2SmInfoType | N2SmInfoType | C | 0..1 | This IE shall be present if "n2SmInfo" attribute is present. When present, this IE shall indicate the NG AP IE type for the NG AP SMF related IE container carried in "n2SmInfo" attribute. |
| upCnxState | UpCnxState | C | 0..1 | This IE shall be present if the SMF was requested to activate or deactivate the user plane connection of the PDU session in the corresponding request. When present, it shall be set as specified in clauses 5.2.2.3.2 and 5.2.2.3.16. |
| recoveryTime | DateTime | O | 0..1 | Timestamp when the SMF service instance was (re)started (see clause 6.3 of 3GPP TS 23.527 [24]). |

##### 6.1.6.2.37	Type: PduSessionCreateError

Table 6.1.6.2.37-1: Definition of type PduSessionCreateError

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| error | ProblemDetails | M | 1 | More information on the error shall be provided in the "cause" attribute of the "ProblemDetails" structure. |
| n1smCause | string | C | 0..1 | This IE shall be present if the request included n1SmInfoFromUe. When present, it shall contain the 5GSM cause the H-SMF proposes the V-SMF to return to the UE. It shall be encoded as two characters in hexadecimal representation with each character taking a value of "0" to "9" or "A" to "F", and represent the cause value of the 5GSM cause IE specified in clause 9.11.4.2 of 3GPP TS 24.501 [7]. Pattern: "^[A-F0-9]{2}$" Example: the cause "Invalid mandatory information" shall be encoded as "60". (NOTE) |
| n1SmInfoToUe | RefToBinaryData | C | 0..1 | This IE shall be present if the H-SMF needs to send N1 SM information to the UE that does not need to be interpreted by the V-SMF. When present, this IE shall reference the n1SmInfoToUe binary data (see clause 6.1.6.4.4). |
| backOffTimer | DurationSec | O | 0..1 | When present, this IE shall indicate a Back-off timer value, in seconds, that the V-SMF may use when rejecting the NAS message towards the UE. |
| recoveryTime | DateTime | O | 0..1 | Timestamp when the H-SMF service instance was (re)started (see clause 6.3 of 3GPP TS 23.527 [24]). |
| NOTE: This IE contains information that the V-SMF may transfer to the UE without interpretation. It is sent as a separate IE rather than within the n1SmInfoToUE binary data because the 5GSM cause IE is defined as a "V" IE (i.e. without a Type field) in the NAS PDU Session Establishment Reject message. |  |  |  |  |

##### 6.1.6.2.38	Type: MmeCapabilities

Table 6.1.6.2.38-1: Definition of type MmeCapabilities

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| nonIpSupported | boolean | C | 0..1 | This IE shall be present if non-IP PDN type is supported. It may be present otherwise. When present, this IE shall be set as follows: - true: non-IP PDN type is supported; - false (default): non-IP PDN type is not supported. |
| ethernetSupported | boolean | C | 0..1 | This IE shall be present if Ethernet PDN type is supported. It may be present otherwise. When present, this IE shall be set as follows: - true: Ethernet PDN type is supported; - false (default): Ethernet PDN type is not supported. |

##### 6.1.6.2.39	Type: SmContext

Table 6.1.6.2.39-1: Definition of type SmContext

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| pduSessionId | PduSessionId | M | 1 | This IE shall contain the PDU Session ID. |
| dnn | Dnn | M | 1 | This IE shall contain the UE requested DNN of the PDU session. The DNN shall be the full DNN (i.e. with both the Network Identifier and Operator Identifier) for a HR PDU session, and it should be the full DNN in LBO and non-roaming scenarios. If the Operator Identifier is absent, the serving core network operator shall be assumed. |
| selectedDnn | Dnn | C | 0..1 | This IE shall be present, if another DNN other than the UE requested DNN is selected for this PDU session. When present, it shall contain the selected DNN. The DNN shall be the full DNN (i.e. with both the Network Identifier and Operator Identifier) for a HR PDU session, and it should be the full DNN in LBO and non-roaming scenarios. If the Operator Identifier is absent, the serving core network operator shall be assumed. |
| sNssai | Snssai | M | 1 | This IE shall contain the S-NSSAI for the serving PLMN. |
| hplmnSnssai | Snssai | C | 0..1 | This IE shall be present for a HR PDU session. When present, it shall contain the S-NSSAI for the HPLMN. |
| pduSessionType | PduSessionType | M | 1 | This IE shall indicate the PDU session type. |
| gpsi | Gpsi | C | 0..1 | This IE shall be present if it is available. When present, it shall contain the user's GPSI. |
| hSmfUri | Uri | C | 0..1 | This IE shall be present in HR roaming scenarios. When present, it shall contain the API URI of the Nsmf_PDUSession service of the H-SMF. The API URI shall be formatted as specified in clause 6.1.1. |
| smfUri | Uri | C | 0..1 | This IE shall be present for a PDU session with an I-SMF. When present, it shall contain the API URI of the Nsmf_PDUSession service of the SMF. The API URI shall be formatted as specified in clause 6.1.1. |
| pduSessionRef | Uri | C | 0..1 | This IE shall be present for a HR PDU session or a PDU session with an I-SMF. When present, this IE shall include the absolute URI of the PDU Session in H-SMF or SMF, including apiRoot (see clause 6.1.3.6.2) |
| pcfId | NfInstanceId | O | 0..1 | When present, this IE shall contain the identifier of: - the H-PCF selected by the AMF (for UE Policy), for a HR PDU session; or - the V-PCF selected by the AMF (for Access and Mobility Policy), for a PDU session in LBO roaming scenarios; or - the PCF selected by the AMF (for Access and Mobility Policy and/or UE Policy), for a PDU session in non-roaming scenarios. |
| pcfGroupId | NfGroupId | O | 0..1 | This IE may be present in non-roaming and HR roaming scenarios. When present, this IE shall contain the identity of the (home) PCF group serving the UE for Access and Mobility Policy and/or UE Policy. |
| pcfSetId | NfSetId | O | 0..1 | This IE may be present if the pcfId IE is present. When present, it shall contain the NF Set ID of the PCF indicated by the pcfId IE. |
| selMode | DnnSelectionMode | C | 0..1 | This IE shall be present if it is available. When present, it shall be set to: - "VERIFIED", if the requested DNN provided by UE or the selected DNN provided by the network corresponds to an explicitly subscribed DNN; or - "UE_DNN_NOT_VERIFIED", if the requested DNN provided by UE corresponds to the usage of a wildcard subscription; or - "NW_DNN_NOT_VERIFIED", if the selected DNN provided by network corresponds to the usage of a wildcard subscription. If both the requested DNN (i.e. dnn IE) and selected DNN (i.e. selected Dnn IE) are present, the selMode shall be related to the selected DNN. |
| udmGroupId | NfGroupId | O | 0..1 | When present, it shall indicate the identity of the UDM group serving the UE. |
| routingIndicator | string | O | 0..1 | When present, it shall indicate the Routing Indicator of the UE. |
| hNwPubKeyId | integer | O | 0..1 | When present, it shall indicate the Home Network Public Key Identifier of the UE. (NOTE) |
| sessionAmbr | Ambr | M | 1 | This IE shall contain the Session AMBR granted to the PDU session. |
| qosFlowsList | array(QosFlowSetupItem) | M | 1..N | This IE shall contain the set of QoS flow(s) established for the PDU session. It shall contain at least the Qos flow associated to the default Qos rule. The qosRules attribute of each QosFlowSetupItem shall be set to an empty string. |
| hSmfInstanceId | NfInstanceId | C | 0..1 | This IE shall be present for a HR PDU session. When present, it shall contain the identifier of the home SMF. |
| smfInstanceId | NfInstanceId | C | 0..1 | This IE shall be present for a PDU session with an I-SMF. When present, it shall contain the identifier of the SMF. |
| pduSessionSmfSetId | NfSetId | C | 0..1 | This IE shall be present, if available. When present, this IE shall contain the NF Set ID of the home SMF as identified by hSmfInstanceId, or the SMF as identified by the smfInstanceId. |
| pduSessionSmfServiceSetId | NfServiceSetId | C | 0..1 | This IE shall be present, if available. When present, this IE shall contain the NF Service Set ID of the PDUSession service instance (for this PDU session) in the home SMF or the SMF. |
| pduSessionSmfBinding | SbiBindingLevel | C | 0..1 | This IE shall be present, if available. When present, this IE shall contain the SBI binding level of the PDU session resource in the home SMF or the SMF. |
| enablePauseCharging | boolean | C | 0..1 | This IE shall be present for a HR PDU session, if available. When present, it shall indicate whether the use of Pause of Charging is enabled for the PDU session (see clause 4.4.4 of 3GPP TS 23.502 [3]). When present, it shall be set as follows: - true: enable Pause of Charging; - false (default): disable Pause of Charging. |
| ueIpv4Address | Ipv4Addr | C | 0..1 | This IE shall be present if a UE IPv4 address to the PDU session. |
| ueIpv6Prefix | Ipv6Prefix | C | 0..1 | This IE shall be present if a UE IPv6 prefix to the PDU session. |
| epsPdnCnxInfo | EpsPdnCnxInfo | C | 0..1 | This IE shall be present if the PDU session may be moved to EPS during its lifetime. |
| epsBearerInfo | array(EpsBearerInfo) | C | 1..N | This IE shall be present if the PDU session may be moved to EPS during its lifetime. |
| maxIntegrityProtectedDataRate | MaxIntegrityProtectedDataRate | C | 0..1 | This IE shall be present if the upSecurity IE is present and indicates that integrity protection is preferred or required. |
| alwaysOnGranted | boolean | C | 0..1 | This IE shall be present if available. When present, it shall indicate whether this is an always On PDU session and it shall be set as follows: - true: always-on PDU session granted. - false (default): always-on PDU session not granted. |
| upSecurity | UpSecurity | O | 0..1 | When present, this IE shall indicate the security policy for integrity protection and encryption for the user plane of the PDU session. |
| hSmfServiceInstanceId | string | O | 0..1 | This IE may be present for a HR PDU session. When present, this IE shall contain the serviceInstanceId of the H-SMF service instance serving the PDU session. This IE may be used by the V-SMF to identify PDU sessions affected by a failure or restart of the H-SMF service (see clause 6.2 of 3GPP TS 23.527 [24]). |
| smfServiceInstanceId | string | O | 0..1 | This IE may be present for a PDU session with an I-SMF. When present, this IE shall contain the serviceInstanceId of the SMF service instance serving the PDU session. This IE may be used by the I-SMF to identify PDU sessions affected by a failure or restart of the SMF service (see clause 6.2 of 3GPP TS 23.527 [24]). |
| recoveryTime | DateTime | O | 0..1 | This IE may be present if available. When present, this IE shall indicate the timestamp when the H-SMF or SMF service instance serving the PDU session was (re)started (see clause 6.3 of 3GPP TS 23.527 [24]). |
| forwardingInd | boolean | C | 0..1 | This IE shall be present, when downlink data packets are buffered at I-UPF. The SMF or I-SMF shall use this IE to inform the NF service consumer that a forwarding tunnel is needed for receiving the buffered downlink data packets, as specified in clause 4.23.4 of 3GPP TS 23.502 [3]. When present, this IE shall be set as follows: - true: a forwarding tunnel is needed for sending buffered downlink data packets; - false (default): forwarding tunnel is not needed |
| psaTunnelInfo | TunnelInfo | C | 0..1 | This IE shall be present if available. When present, this IE shall contain the N9 tunnel information of PDU Session Anchor UPF controlled by SMF or H-SMF. |
| chargingId | string | C | 0..1 | This IE shall be present for a HR PDU session, in scenarios with a V-SMF insertion/change/removal. When present, it shall contain the Charging ID of the PDU session (see 3GPP TS 32.255 [25]). |
| chargingInfo | ChargingInformation | C | 0..1 | This IE shall be present for a HR PDU session, if available and if the NF Service Consumer requesting the SM Context pertains to the same PLMN (i.e. if the Retrieve SM Context Request does not contain the servingNetwork attribute set to a different PLMN ID). When present, it shall contain the addresses of the V-CHF used for the PDU session. |
| roamingChargingProfile | RoamingChargingProfile | C | 0..1 | This IE shall be present for a HR PDU session, if available and if the NF Service Consumer requesting the SM Context pertains to the same PLMN (i.e. if the Retrieve SM Context Request does not contain the servingNetwork attribute set to a different PLMN ID). When present, it shall contain the Roaming Charging Profile selected by the HPLMN (see clauses 5.1.9.1, 5.2.1.7 and 5.2.2.12.2 of 3GPP TS 32.255 [25]). |
| nefExtBufSupportInd | boolean | C | 0..1 | This IE shall be present with value "true", if the anchor NEF has indicated support of Extended Buffering for mobile terminated data during SMF-NEF connection establishment. When present, this IE shall be set as following: - true: Extended Buffering supported by NEF - false (default): Extended Buffering not supported by NEF |
| ipv6Index | IpIndex | C | 0..1 | This IE shall be present during I-SMF change scenarios, if IPv6 Index has previously been received by old I-SMF. |
| dnAaaAddress | IpAddress | O | 0..1 | When present, this IE shall contain the address of DN-AAA server for UE IP Address allocation previously received by old I-SMF. |
| redundantPduSessionInfo | RedundantPduSessionInformation | C | 0..1 | This IE shall be present for a PDU session with an I-SMF, if this information has been received previously from the UE, the anchor SMF or the old I-SMF. |
| ranTunnelInfo | QosFlowTunnel | C | 0..1 | This IE shall be present if the ranUnchangedInd IE is set to "true" in the SM context retrieve request. When present, this IE shall contain the N2 tunnel information of NG-RAN with associated QoS flows (see "DL QoS Flow per TNL Information" in clause 9.3.4.2 of 3GPP 38.413 [9]). |
| addRanTunnelInfo | array(QosFlowTunnel) | C | 1..N | This IE shall be present if the ranUnchangedInd IE is set to "true" in the SM context retrieve request. When present, this IE shall contain the additional N2 tunnel information of NG-RAN together with associated QoS flows for split PDU session (see "Additional DL QoS Flow per TNL Information" in clause 9.3.4.2 of 3GPP 38.413 [9]). |
| redRanTunnelInfo | QosFlowTunnel | C | 0..1 | This IE shall be present if the ranUnchangedInd IE is set to "true" in the SM context retrieve request. When present, this IE shall contain the additional N2 tunnel information of NG-RAN together with associated QoS flows for Redundant QoS Flow(s) (see "Redundant DL QoS Flow per TNL Information" in clause 9.3.4.2 of 3GPP 38.413 [9]). |
| addRedRanTunnelInfo | array(QosFlowTunnel) | C | 1..N | This IE shall be present if the ranUnchangedInd IE is set to "true" in the SM context retrieve request. When present, this IE shall contain the additional N2 tunnel information of NG-RAN together with associated QoS flows for Redundant QoS Flow(s) with split PDU session (see "Additional Redundant DL QoS Flow per TNL Information" in clause 9.3.4.2 of 3GPP 38.413 [9]). |
| nspuSupportInd | boolean | C | 0..1 | This IE shall be present and set to "true" if the enablePauseCharging in the SmContext data type is set to "true" and if the (H-)SMF and PSA UPF support Notify Start Pause of Charging via user plane feature as specified in clause 5.30 of 3GPP TS 29.244 [29]. When present, it shall be set as follows: - true: Notify Start Pause of Charging via user plane feature is supported. |
| smfBindingInfo | string | C | 0..1 | This IE shall be present, if available. When present, this IE shall contain the Binding indications of the PDU session resource in the home SMF or the SMF and shall be set to the value of the 3gpp-Sbi-Binding header defined in clause 5.2.3.2.6 of 3GPP TS 29.500 [4], without the header name. |
| NOTE: If present, this attribute shall be used together with routingIndicator. This attribute is only used by the HPLMN in roaming scenarios. |  |  |  |  |

##### 6.1.6.2.40	Type: ExemptionInd

The ExemptionInd indicates that the included NAS SM message was exempted from one or more NAS SM congestion control, e.g. DNN, and/or S-NSSAI based congestion control, activated in the AMF.

Table 6.1.6.2.40-1: Definition of type ExemptionInd

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| dnnCongestion | boolean | C | 0..1 | This IE shall be present and set to Yes if the included NAS Session Management message was exempted from the DNN based congestion activated in the AMF. true: Yes false (default): No |
| snssaiCongestion | boolean | C | 0..1 | This IE shall be present and set to Yes if the included NAS Session Management message was exempted from the S-NSSAI only based congestion activated in the AMF. true: Yes false (default): No |
| snssaiDnnCongestion | boolean | C | 0..1 | This IE shall be present and set to Yes if the included NAS Session Management message was exempted from the S-NSSAI and DNN based congestion activated in the AMF. true: Yes false (default): No |

##### 6.1.6.2.41	Type: PsaInformation

Table 6.1.6.2.41-1: Definition of type PsaInformation

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| psaInd | PsaIndication | M | 1 | This IE shall indicate, for a PDU session with an I-SMF, if a PSA and UL CL or BP, or only a PSA is inserted or removed by the I-SMF. |
| dnaiList | array(Dnai) | M | 1..N | This IE shall indicate the DNAI(s) supported by the PSA that is inserted or removed. |
| ueIpv6Prefix | Ipv6Prefix | C | 0..1 | This IE shall be present if a PSA and UL CL or BP is inserted or removed, and IPv6 multi-homing applies to the PDU session. |
| psaUpfId | NfInstanceId | C | 0..1 | This IE shall be present if a PSA UPF is inserted by the I-SMF. When present, it shall contain the identifier of the PSA UPF. |

##### 6.1.6.2.42	Type: DnaiInformation

Table 6.1.6.2.42-1: Definition of type DnaiInformation

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| dnai | Dnai | M | 1 |  |
| noDnaiChangeInd | boolean | C | 0..1 | This IE shall be sent by the SMF to the I-SMF during the insertion of a PSA and BP/UL CL controlled by I-SMF. When present, it shall be set as follows: - true: DNAI shall not be changed; - false: DNAI may be changed. |
| noLocalPsaChangeInd | boolean | C | 0..1 | This IE shall be sent by the SMF to the I-SMF during the insertion of a PSA and BP/UL CL controlled by I-SMF. When present, it shall be set as follows: - true: local PSA shall not be changed; - false: local PSA may be changed. |

##### 6.1.6.2.43	Type: N4Information

Table 6.1.6.2.43-1: Definition of type N4Information

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n4MessageType | N4MessageType | M | 1 | This IE shall indicate the PFCP message signalled in the n4MessagePayload. |
| n4MessagePayload | RefToBinaryData | M | 1 | This IE shall reference the N4 Message Payload binary data (for the n4Info attribute) or the N4 Information extension binary data (for the n4InfoExt1, n4InfoExt2 and n4InfoExt3 attributes), see clause 6.1.6.4.5. |
| n4DnaiInfo | DnaiInformation | C | 0..1 | This IE shall be present if the N4 information relates to a PSA. When present, it shall indicate the DNAI related to the N4 Information. If this IE is not present, this indicates N4 information relates to an UL CL or BP. |
| psaUpfId | NfInstanceId | O | 0..1 | This IE may be sent by SMF to I-SMF if multiple local PSAs are inserted for the PDU session. When present, it shall contain the identifier of the PSA UPF for which the N4 information applies. |

##### 6.1.6.2.44	Type: IndirectDataForwardingTunnelInfo

Table 6.1.6.2.44-1: Definition of type IndirectDataForwardingTunnelInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| ipv4Addr | Ipv4Addr | C | 0..1 | When present, this IE shall contain the GTP tunnel IPv4 address. At least one of the ipv4Addr or ipv6Addr shall be present. Both may be present. |
| ipv6Addr | Ipv6Addr | C | 0..1 | When present, this IE shall contain the GTP tunnel IPv6 address. At least one of the ipv4Addr or ipv6Addr shall be present. Both may be present. |
| gtpTeid | Teid | M | 1 | This IE shall contain the 4-octet GTP tunnel endpoint identifier. If both ipv4Addr and ipv6Addr are present, the TEID shall be shared by both addresses. |
| drbId | DrbId | C | 0..1 | This IE shall be present if this is an indirect data forwarding tunnel for a specific Data Radio Bearer (see clause 9.3.1.77 of 3GPP TS 38.413 [9]). This IE shall not present if the additionalTnlNb IE is present. (NOTE) |
| additionalTnlNb | AdditionalTnlNb | C | 0..1 | This IE shall be present if this is an additional indirect data forwarding tunnel for multi-connectivity. When present, it shall be set to the value 1 to 3 to indicate whether this is the first, second or third additional indirect data forwarding tunnel for multi-connectivity. This IE shall not present if the drbId IE is present. (NOTE) |
| NOTE: If neither the drbId IE nor the additionalTnlNb IE is present, the tunnel information shall correspond to the PDU session level indirect data forwarding tunnel (i.e. DL Forwarding UP TNL Information IE or UL Forwarding UP TNL Information IE of the Handover Request Acknowledge Transfer IE of clause 9.3.4.11 of 3GPP TS 38.413 [9]). |  |  |  |  |

##### 6.1.6.2.45	Type: SmContextReleasedData

Table 6.1.6.2.45-1: Definition of type SmContextReleasedData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| smallDataRateStatus | SmallDataRateStatus | C | 0..1 | This IE shall be present, if the NF Service Consumer has indicated support of CIoT and if the status is available. When present, it shall indicate the current small data rate control status for the PDU session. | CIOT |
| apnRateStatus | ApnRateStatus | C | 0..1 | This IE shall be present, if the NF Service Consumer has indicated support of CIoT and if the status is available. When present, it shall indicate the current APN rate control status for the PDN connection (APN rates are shared by all PDN connections of the UE to this APN). | CIOT |

##### 6.1.6.2.46	Type: ReleasedData

Table 6.1.6.2.46-1: Definition of type ReleasedData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| smallDataRateStatus | SmallDataRateStatus | C | 0..1 | This IE shall be present, if the NF Service Consumer has indicated support of CIoT and if available. When present, it shall indicate the current small data rate control status for the PDU session. | CIOT |
| apnRateStatus | ApnRateStatus | C | 0..1 | This IE shall be present, if the NF Service Consumer has indicated support of CIoT and if the status is available. When present, it shall indicate the current APN rate control status for the PDN connection (APN rates are shared by all PDN connections of the UE to this APN). | CIOT |
| n4Info | N4Information | O | 0..1 | This IE may be present if the SMF needs to send N4 information (e.g. acknowledgement of traffic usage reporting) to the I-SMF for traffic offloaded at a PSA controlled by an I-SMF. | DTSSA |
| n4InfoExt1 | N4Information | O | 0..1 | This IE may be present if the SMF needs to send additional N4 information (e.g. acknowledgement of traffic usage reporting) to the I-SMF for traffic offloaded at a PSA controlled by an I-SMF. | DTSSA |
| n4InfoExt2 | N4Information | O | 0..1 | This IE may be present if the SMF needs to send additional N4 information (e.g. acknowledgement of traffic usage reporting) to the I-SMF for traffic offloaded at a PSA controlled by an I-SMF. | DTSSA |

##### 6.1.6.2.47	Type: SendMoDataReqData

Table 6.1.6.2.47-1: Definition of type SendMoDataReqData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| moData | RefToBinaryData | M | 1 | This IE shall reference the mobile originated data (see clause 6.1.6.4.6). | CIOT |
| moExpDataCounter | MoExpDataCounter | C | 0..1 | This IE shall be included if the UE has accessed the network by using "MO exception data" RRC establishment cause and when the AMF decides to send a non-zero value to the SMF. (NOTE) When present, this IE shall contain the MO Exception Data Counter. | CIOT |
| ueLocation | UserLocation | O | 0..1 | When present, this IE shall contain the user location. | CIOT |
| NOTE: The AMF increments the MO Exception Data Counter when the UE establishes/resumes RRC with "MO Exception Data" RRC cause. The AMF may defer sending the moExpDataCounter attribute to the SMF based on local configuration. The AMF resets the MO Exception Data Counter when receiving successful response from the SMF. The SMF however keeps incrementing the counter locally. |  |  |  |  |  |

##### 6.1.6.2.48	Type: CnAssistedRanPara

Table 6.1.6.2.48-1: Definition of type CnAssistedRanPara

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| stationaryIndication | StationaryIndication | O | 0..1 | Identifies whether the UE is stationary or mobile (see 3GPP TS 23.502 [3] clause 4.15.6.3). |
| communicationDurationTime | DurationSec | O | 0..1 | Indicates for how long the UE will normally stay in CM-Connected for data transmission (see 3GPP TS 23.502 [3] clause 4.15.6.3). |
| periodicTime | DurationSec | O | 0..1 | Identifies interval time of periodic communication (see 3GPP TS 23.502 [3] clause 4.15.6.3). |
| scheduledCommunicationTime | ScheduledCommunicationTime | O | 0..1 | Identifies time and day of the week when the UE is available for communication (see 3GPP TS 23.502 [3] clause 4.15.6.3). |
| scheduledCommunicationType | ScheduledCommunicationType | O | 0..1 | Indicates that the Scheduled Communication Type (see 3GPP TS 23.502 [3] clause 4.15.6.3). (NOTE 2) |
| trafficProfile | TrafficProfile | O | 0..1 | Identifies the type of data transmission: single packet transmission (UL or DL), dual packet transmission (UL with subsequent DL or DL with subsequent UL), and multiple packets transmission (see 3GPP TS 23.502 [3] clause 4.15.6.3). |
| batteryIndication | BatteryIndication | O | 0..1 | Indicates the power consumption type(s) of the UE (see 3GPP TS 23.502 [3] clause 4.15.6.3). |
| NOTE 1: At least one of optional parameters above shall be present. NOTE 2: The value of attribute "scheduledCommunicationType" shall be used together with the value of "scheduledCommunicationTime". |  |  |  |  |

##### 6.1.6.2.49	Type: UlclBpInformation

Table 6.1.6.2.49-1: Definition of type UlclBpInformation

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| ulclBpUpfId | NfInstanceId | C | 0..1 | This IE shall be present if an UL CL or BP UPF separate from the local PSA is inserted by the I-SMF. When present, it shall contain the identifier of the UL CL or BP UPF. |

##### 6.1.6.2.50	Type: TransferMoDataReqData

Table 6.1.6.2.50-1: Definition of type TransferMoDataReqData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| moData | RefToBinaryData | M | 1 | This IE shall reference the mobile originated data (see clause 6.1.6.4.6). | CIOT |
| moExpDataCounter | MoExpDataCounter | C | 0..1 | This IE shall be present if received from AMF. When present, this IE shall contain the MO Exception Data Counter. | CIOT |
| ueLocation | UserLocation | O | 0..1 | When present, this IE shall contain the user location. | CIOT |

##### 6.1.6.2.51	Type: TransferMtDataReqData

Table 6.1.6.2.51-1: Definition of type TransferMtDataReqData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mtData | RefToBinaryData | M | 1 | This IE shall reference the mobile terminated data (see clause 6.1.6.4.7). | CIOT |

##### 6.1.6.2.52	Type: TransferMtDataError

Table 6.1.6.2.52-1: Definition of type TransferMtDataError as a list of to be combined data types

| Data type | Cardinality | Description | Applicability |
| --- | --- | --- | --- |
| ExtProblemDetails | 1 | Detail information of the problem |  |
| TransferMtDataAddInfo | 1 | Additional information to be returned in error response. |  |

##### 6.1.6.2.53	Type: TransferMtDataAddInfo

Table 6.1.6.2.53-1: Definition of type TransferMtDataAddInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| maxWaitingTime | DurationSec | C | 0..1 | This IE shall be present if available. When present, this IE shall contain the estimated maximum wait time (see clause 4.25.5 of 3GPP 23.502 [3]). | CIOT |

##### 6.1.6.2.54	Type: VplmnQos

Table 6.1.6.2.54-1: Definition of type VplmnQos

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| 5qi | 5Qi | O | 0..1 | When present, this IE shall contain the 5G QoS Identifier (5QI) accepted or requested for the QoS Flow associated with the default QoS rule. |
| arp | Arp | O | 0..1 | When present, this IE shall contain the Allocation and Retention Priority (ARP) accepted by the VPLMN for the QoS Flow associated with the default QoS rule. |
| sessionAmbr | Ambr | O | 0..1 | When present, this IE shall contain the highest Session-AMBR accepted by the VPLMN for the PDU session. |
| maxFbrDl | BitRate | O | 0..1 | When present, this IE shall contain the Maximum Bit Rate in Downlink accepted by the VPLMN for the QoS Flow associated with the default QoS rule (if this is a GBR QoS Flow). See 3GPP TS 23.501 [2]. |
| maxFbrUl | BitRate | O | 0..1 | When present, this IE shall contain the Maximum Bit Rate in Uplink accepted by the VPLMN for the QoS Flow associated with the default QoS rule (if this is a GBR QoS Flow). See 3GPP TS 23.501 [2]. |
| guaFbrDl | BitRate | O | 0..1 | When present, this IE shall contain the Guaranteed Bit Rate in Downlink accepted by the VPLMN for the QoS Flow associated with the default QoS rule (if this is a GBR QoS Flow). See 3GPP TS 23.501 [2]. |
| guaFbrUl | BitRate | O | 0..1 | This IE shall contain the Guaranteed Bit Rate in Uplink accepted by the VPLMN for the QoS Flow associated with the default QoS rule (if this is a GBR QoS Flow). See 3GPP TS 23.501 [2]. |

##### 6.1.6.2.55	Type: DdnFailureSubs

Table 6.1.6.2.55-1: Definition of type DdnFailureSubs

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| ddnFailureSubsInd | boolean | M | 1 | When present, it shall be set as follows: - true: Notification of DDN failure is subscribed. - false (default): Notification of DDN failure is not subscribed. |
| ddnFailureSubsInfoList | array(DdnFailureSubInfo) | C | 1..N | This IE shall be present, if notification of DDN failure is subscribed, to provide DDN failure subscription details. |

##### 6.1.6.2.56	Type: RetrieveData

Table 6.1.6.2.56-1: Definition of type RetrieveData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| smallDataRateStatusReq | boolean | C | 0..1 | This IE shall be present and set to "true" if this is a request to retrieve the Small Data Rate Status of the PDU session. When present, it shall be set as follows: - true: small data rate control status is requested. - false (default): small data rate control status is not requested. | CIOT |

##### 6.1.6.2.57	Type: RetrievedData

Table 6.1.6.2.57-1: Definition of type RetrievedData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| smallDataRateStatus | SmallDataRateStatus | C | 0..1 | This IE shall be present if the information has been requested in the request and is available. When present, it shall indicate the current small data rate control status for the PDU session. | CIOT |

##### 6.1.6.2.58	Type: SecurityResult

Table 6.1.6.2.58-1: Definition of type SecurityResult

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| integrityProtectionResult | ProtectionResult | C | 0..1 | This IE shall be included if available. If present, this IE indicates whether UP integrity protection is performed or not for the concerned PDU session. |
| confidentialityProtectionResult | ProtectionResult | C | 0..1 | This IE shall be included if available. If present, this IE indicates whether UP ciphering is performed or not for the concerned PDU session. |

##### 6.1.6.2.59	Type: UpSecurityInfo

Table 6.1.6.2.59-1: Definition of type UpSecurityInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| upSecurity | UpSecurity | M | 1 | This IE shall indicate the security policy for integrity protection and encryption for the user plane of the PDU session. See clause 9.3.1.60 of 3GPP TS 38.413 [9]. |
| maxIntegrityProtectedDataRateUl | MaxIntegrityProtectedDataRate | C | 0..1 | This IE shall be present if the upSecurity IE is present and indicates that integrity protection is preferred or required. See clause 9.3.1.60 of 3GPP TS 38.413 [9]. When present, it shall indicate the maximum integrity protected data rate supported by the UE for uplink. |
| maxIntegrityProtectedDataRateDl | MaxIntegrityProtectedDataRate | C | 0..1 | This IE shall be present if the upSecurity IE is present and indicates that integrity protection is preferred or required. When present, it shall indicate the maximum integrity protected data rate supported by the UE for downlink. |
| securityResult | SecurityResult | C | 0..1 | This IE shall be included if available. If present, this IE shall contain the Security Result associated to the PDU session. See clause 9.3.1.60 of 3GPP TS 38.413 [9]. |

##### 6.1.6.2.60	Type: DdnFailureSubInfo

Table 6.1.6.2.60-1: Definition of type DdnFailureSubInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| notifyCorrelationId | string | M | 1 | This IE shall indicate the notification correlation Id provided by the NF service consumer (e.g. AMF) when subscribing to the notification of the DDN Failure, which shall be returned by the SMF when a DDN Failure is notified for this subscription. This parameter can be useful if the NF service consumer has multiple subscriptions for the same PDU session. |
| dddTrafficDescriptorList | array(DddTrafficDescriptor) | C | 1..N | This IE shall be present if it is received from the UDM. When present, it shall contain a list of Traffic Descriptors related to the event of DDN Failure for which the subscription applies. |

##### 6.1.6.2.61	Type: AlternativeQosProfile

Table 6.1.6.2.61-1: Definition of type AlternativeQosProfile

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| index | Integer | M | 1 | When present, this IE shall contain the index identifying the alternative QoS profile. Minimum = 1. Maximum = 8. |
| guaFbrDl | BitRate | O | 0..1 | When present, this IE shall contain the Guaranteed Bit Rate in Downlink. See 3GPP TS 23.501 [2]. |
| guaFbrUl | BitRate | O | 0..1 | When present, this IE shall contain the Guaranteed Bit Rate in Uplink. See 3GPP TS 23.501 [2]. |
| packetDelayBudget | PacketDelBudget | O | 0..1 | When present, this IE shall indicate the packet delay budget. |
| packetErrRate | PacketErrRate | O | 0..1 | When present, this IE shall indicate the packet error rate. |

##### 6.1.6.2.62	Type: ProblemDetailsAddInfo

Table 6.1.6.2.62-1: Definition of type ProblemDetailsAddInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| remoteError | boolean | O | 0..1 | When present, this IE shall indicate whether the error is originated by the remote entity or by the entity sending the response, as follows: - true: the error is originated by the remote entity (i.e. H-SMF/SMF or AMF). - false: the error is originated by the entity sending the response (i.e. V-SMF/I-SMF). This IE shall be present and set to "true" for a HR PDU session or for a PDU session with an I-SMF, when the V-SMF/I-SMF retuns an error response to the AMF that was originated by the H-SMF/SMF, or when the V-SMF/I-SMF returns an error response to the H-SMF/SMF that was originated by the AMF. This IE may be present if the error is originated by the V-SMF/I-SMF. |

##### 6.1.6.2.63	Type: ExtProblemDetails

Table 6.1.6.2.63-1: Definition of type ExtProblemDetails as a list of to be combined data types

| Data type | Cardinality | Description | Applicability |
| --- | --- | --- | --- |
| ProblemDetails | 1 | Detail information of the problem |  |
| ProblemDetailsAddInfo | 1 | Additional information to be returned in error response. |  |

##### 6.1.6.2.64	Type: QosMonitoringInfo

Table 6.1.6.2.64-1: Definition of type QosMonitoringInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| qosMonitoringInd | boolean | C | 0..1 | This IE shall be present and set to "true" if QoS monitoring activated for QoS flows of the PDU session is performed using end to end accumulated packet delay reporting in UL GTP-U packets (see clause 5.33.3.3 of 3GPP TS 23.501 [2]). - True: QoS monitoring is performed using end to end accumulated packet delay reporting in UL GTP-U packets (see clause 5.33.3.3 of 3GPP TS 23.501 [2]). - False (default): QoS monitoring is performed using UPF and RAN time information in GTP-U packets (see clause 5.33.3.2 of 3GPP TS 23.501 [2]). When this attribute is present and set to "true", the I-SMF shall provision the I-UPF to report an end to end accumulated packet delay in UL GTP-U packets as specified in 3GPP TS 29.244 [29]. |

##### 6.1.6.2.65	Type: IpAddress

Table 6.1.6.2.65-1: Definition of type IpAddress

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| ipv4Addr | Ipv4Addr | C | 0..1 | Indicate an IPv4 Address |
| ipv6Addr | Ipv6Addr | C | 0..1 | Indicate an IPv6 Address |
| ipv6Prefix | Ipv6Prefix | C | 0..1 | Indicate an IPv6 Prefix |
| NOTE: Either ipv4Addr, or ipv6Addr, or ipv6Prefix shall be present. |  |  |  |  |

##### 6.1.6.2.66	Type: RedundantPduSessionInformation

Table 6.1.6.2.66-1: Definition of type RedundantPduSessionInformation

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| rsn | Rsn | M | 1 | RSN |
| pduSessionPairId | integer | O | 0..1 | PDU Session Pair ID |

##### 6.1.6.2.67	Type: QosFlowTunnel

Table 6.1.6.2.67-1: Definition of type QosFlowTunnel

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| qfiList | array(Qfi) | M | 1..N | This IE shall contain the list of QoS Flow Identifiers. |
| tunnelInfo | TunnelInfo | M | 1 | This IE shall contain the RAN Tunnel Information. |

##### 6.1.6.2.68	Type: TargetDnaiInfo

Table 6.1.6.2.68-1: Definition of type TargetDnaiInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| targetDnai | Dnai | M | 1 | This IE shall contain the target DNAI. |
| smfSelectionType | SmfSelectionType | M | 1 | This IE shall indicate whether I-SMF or SMF shall be selected based on the target DNAI. |

##### 6.1.6.2.69	Type: AfCoordinationInfo

Table 6.1.6.2.69-1: Definition of type AfCoordinationInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| sourceDnai | Dnai | C | 0..1 | This IE shall be present if available. If present, this IE shall contain the source DNAI. |
| sourceUeIpv4Addr | Ipv4Addr | C | 0..1 | This IE shall be present if available. If present, this IE shall contain the IPv4 Address of the served UE for the source DNAI. |
| sourceUeIpv6Prefix | Ipv6Prefix | C | 0..1 | This IE shall be present if available. If present, this IE shall contain the Ipv6 Address Prefix of the served UE for the source DNAI. |
| notificationInfoList | array(NotificationInfo) | C | 1..N | This IE shall be present if available. If present, this IE shall contain the list of the Notification Correlation ID and Notification URI provided by the NF service consumers. |

##### 6.1.6.2.70	Type: NotificationInfo

Table 6.1.6.2.70-1: Definition of type NotificationInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| notifId | string | M | 1 | Notification Correlation ID provided by the NF service consumer. |
| notifUri | Uri | M | 1 | Identifies the recipient of Notifications sent by the SMF. |
| upBufferInd | boolean | C | 0..1 | This IE shall be present, if available. When present, it shall be set as follows: - true: uplink buffering is required; - false (default): uplink buffering is not required. |

#### 6.1.6.3	Simple data types and enumerations

##### 6.1.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 6.1.6.3.2	Simple data types

The simple data types defined in table 6.1.6.3.2-1 shall be supported.

Table 6.1.6.3.2-1: Simple data types

| Type Name | Type Definition | Description |
| --- | --- | --- |
| ProcedureTransactionId | integer | Unsigned integer representing a Procedure Transaction Identity, within the range 0 to 255, as specified in 3GPP TS 24.007 [8]. |
| EpsBearerId | integer | Integer identifying an EPS bearer, within the range 0 to 15, as specified in clause 11.2.3.1.5, bits 5 to 8, of 3GPP TS 24.007 [8]. |
| EpsPdnCnxContainer | string | String with format "byte" as defined in OpenAPI Specification [15], i.e. base64-encoded characters, encoding the UeEpsPdnConnection IE specified in Table 7.3.1-2 or Table 7.3.6-2 of 3GPP TS 29.274 [16] for the N26 interface. |
| EpsBearerContainer | string | String with format "byte" as defined in OpenAPI Specification [15], i.e. base64-encoded characters, encoding the Bearer Context IE specified in Table 7.3.2-2 of 3GPP TS 29.274 [16]. |
| Teid | string | 4-octet GTP tunnel endpoint identifier, as defined in 3GPP TS 29.274 [16], in hexadecimal representation. Each character in the string shall take a value of "0" to "9" or "a" to "f" or "A" to "F" and shall represent 4 bits. The most significant character representing the 4 most significant bits of the TEID shall appear first in the string, and the character representing the 4 least significant bit of the TEID shall appear last in the string. Pattern: "^[A-Fa-f0-9]{8}" Example: A GTP TEID 0x5BD60076 shall be encoded as "5BD60076". |
| EpsBearerContextStatus | string | EPS bearer context status, as defined in octets 3 and 4 of the EPS bearer context status IE in clause 9.9.2.1 of 3GPP TS 24.301 [27], in hexadecimal representation. Each character in the string shall take a value of "0" to "9" or "a" to "f" or "A" to "F" and shall represent 4 bits. The most significant character representing the 4 most significant bits of the EPS bearer context status shall appear first in the string, and the character representing the 4 least significant bit of the EPS bearer context status shall appear last in the string. Pattern: '^[A-Fa-f0-9]{4}$' Example: An EPS bearer context status IE where only the EBIs 2, 5, 6 and 9 are active shall be encoded as "6402". |
| DrbId | integer | Unsigned integer representing a Data Radio Bearer Identity, within the range 1 to 32, as specified in clause 9.3.1.53 of 3GPP TS 38.413 [9]. |
| AdditionalTnlNb | integer | Unsigned integer, within the range 1 to 3, indicating whether this is the first, second or third additional indirect data forwarding tunnel for multi-connectivity. |
| ForwardingBearerContainer | string | String with format "byte" as defined in OpenAPI Specification [15], i.e. base64-encoded characters, encoding the Bearer Context IE within Context Acknowledge specified in Table 7.3.7-2 of 3GPP TS 29.274 [16]. |
| SecondaryRatUsageDataReportContainer | string | String with format "byte" as defined in OpenAPI Specification [15], i.e. base64-encoded characters, encoding the Secondary RAT Usage Data Report IE within Forward Relocation Complete Acknowledge specified in Table 7.3.4-1 of 3GPP TS 29.274 [16] (starting from octet 1). |

##### 6.1.6.3.3	Enumeration: UpCnxState

The enumeration UpCnxState represents the state of the user plane connection of a PDU session. It shall comply with the provisions defined in table 6.1.6.3.3-1.

Table 6.1.6.3.3-1: Enumeration UpCnxState

| Enumeration value | Description |
| --- | --- |
| "ACTIVATED" | A N3 tunnel is established between the 5G-AN and UPF. |
| "DEACTIVATED" | No N3 tunnel is established between the 5G-AN and UPF. |
| "ACTIVATING" | A N3 tunnel is being established (the 5G-AN's F-TEID for downlink traffic is not assigned yet). |
| "SUSPENDED" | A N3 tunnel is suspended between the 5G-AN and UPF. |

##### 6.1.6.3.4	Enumeration: HoState

The enumeration HoState represents the handover state of a PDU session. It shall comply with the provisions defined in table 6.1.6.3.4-1.

Table 6.1.6.3.4-1: Enumeration HoState

| Enumeration value | Description |
| --- | --- |
| "NONE" | No handover is in progress for the PDU session. |
| "PREPARING" | A handover is in preparation for the PDU session; see clause 5.2.2.3.4.1. |
| "PREPARED" | A handover is prepared for the PDU session; see clause 5.2.2.3.4.1. |
| "COMPLETED" | The handover is completed. |
| "CANCELLED" | The handover is cancelled. |

##### 6.1.6.3.5	Enumeration: RequestType

The enumeration RequestType indicates the type of a PDU session creation request. It shall comply with the provisions defined in table 6.1.6.3.5-1.

Table 6.1.6.3.5-1: Enumeration RequestType

| Enumeration value | Description |
| --- | --- |
| "INITIAL_REQUEST" | Request to establish a new PDU session. |
| "EXISTING_PDU_SESSION" | Request referring to an existing PDU session. |
| "INITIAL_EMERGENCY_REQUEST" | Request to establish a new PDU session for Emergency Services. |
| "EXISTING_EMERGENCY_PDU_SESSION" | Request referring to an existing PDU session for Emergency Services. |
| NOTE: Clause 9.11.3.47 of 3GPP TS 24.501 [7] defines a specific Request type value in NAS PDUs for a MA PDU request. This shall be mapped to the maRequestInd attribute in the Create SM Context Request, Update SM Context Request, Create Request and Update Request. Accordingly, no corresponding value is defined in the RequestType enumeration. |  |

##### 6.1.6.3.6	Enumeration: RequestIndication

The enumeration RequestIndication indicates the request type. It shall comply with the provisions defined in table 6.1.6.3.6-1.

Table 6.1.6.3.6-1: Enumeration RequestIndication

| Enumeration value | Description |
| --- | --- |
| "UE_REQ_PDU_SES_MOD" | UE Requested PDU Session Modification |
| "UE_REQ_PDU_SES_REL" | UE Requested PDU Session Release |
| "PDU_SES_MOB" | PDU Session Mobility (e.g. between 3GPP and non-3GPP access, or from EPS to 5GS with N26 interface) |
| "NW_REQ_PDU_SES_AUTH" | Network Requested PDU Session Authentication. This value is used for the procedures with a secondary authorization/ authentication in an Update request initiated by an H-SMF/SMF during e.g. the PDU Session establishment procedure to inform the V-SMF/I-SMF that the H-SMF/SMF decided to initiate the optional PDU Session establishment authentication/authorization procedure (see clause 4.3.2.3 in 3GPP TS 23.502 [3]). |
| "NW_REQ_PDU_SES_MOD" | Network Requested PDU Session Modification |
| "NW_REQ_PDU_SES_REL" | Network Requested PDU Session Release |
| "EBI_ASSIGNMENT_REQ" | EPS Bearer ID Assignment Request or EPS Bearer ID Revocation Request |
| "REL_DUE_TO_5G_AN_REQUEST" | 5G-AN Requested PDU Session Resource Release |

##### 6.1.6.3.7	Enumeration: NotificationCause

The enumeration NotificationCause indicates the cause of a notification. It shall comply with the provisions defined in table 6.1.6.3.7-1.

Table 6.1.6.3.7-1: Enumeration NotificationCause

| Enumeration value | Description |
| --- | --- |
| "QOS_FULFILLED" | The QoS targets are fulfilled again for the GBR QoS flow. |
| "QOS_NOT_FULFILLED" | The QoS targets are no longer fulfilled for the GBR QoS flow. |
| "UP_SEC_FULFILLED" | The user plane security enforcement "Preferred" is fulfilled again for the PDU session. |
| "UP_SEC_NOT_FULFILLED" | The user plane security enforcement "Preferred" is not fulfilled for the PDU session. |

##### 6.1.6.3.8	Enumeration: Cause

The enumeration Cause indicates a cause information. It shall comply with the provisions defined in table 6.1.6.3.8-1.

Table 6.1.6.3.8-1: Enumeration Cause

| Enumeration value | Description |
| --- | --- |
| "REL_DUE_TO_HO" | Release due to Handover |
| "EPS_FALLBACK" | Mobility due to EPS fallback for IMS voice is on-going. |
| "REL_DUE_TO_UP_SEC" | Release due to user plane Security requirements that cannot be fulfilled. |
| "DNN_CONGESTION" | Release due to the DNN based congestion control. |
| "S_NSSAI_CONGESTION" | Release due to the S-NSSAI based congestion control. |
| "REL_DUE_TO_REACTIVATION" | Release due to PDU session reactivation. |
| "5G_AN_NOT_RESPONDING" | The 5G AN did not respond to the request initiated by the network. |
| "REL_DUE_TO_SLICE_NOT_AVAILABLE" | Release due to the associated S-NSSAI becomes no longer available. |
| "REL_DUE_TO_DUPLICATE_SESSION_ID" | Release due to a UE request to establish a new PDU session with an identical PDU session Id. |
| "PDU_SESSION_STATUS_MISMATCH" | Release due to mismatch of PDU Session status between UE and AMF. |
| "HO_FAILURE" | Handover preparation failure |
| "INSUFFICIENT_UP_RESOURCES" | Failure to activate the User Plane connection of a PDU session due to insufficient user plane resources. |
| "PDU_SESSION_HANDED_OVER" | The PDU session is handed over to another system or access. |
| "PDU_SESSION_RESUMED" | Resume the user plane connection of the PDU session. |
| "CN_ASSISTED_RAN_PARAMETER_TUNING" | SMF derived CN assisted RAN parameters tuning. |
| "ISMF_ CONTEXT_TRANSFER" | The PDU session shall be transferred from old I-SMF to new I-SMF. |
| "SMF_ CONTEXT_TRANSFER" | The PDU session shall be transferred from old SMF to new SMF. |
| "REL_DUE_TO_PS_TO_CS_HO" | Release due to 5G SRVCC from NG-RAN to 3GPP UTRAN, as specified in clause 6.5.4 of 3GPP TS 23.216 [35]. |
| "REL_DUE_TO_SUBSCRIPTION_CHANGE" | Release due to UE subscription changes, e.g. removal of subscribed DNNs, or ODB changes which causes PDU session release. |
| "HO_CANCEL" | Handover cancellation |
| "REL_DUE_TO_SLICE_NOT_AUTHORIZED" | Release due to Network Slice-Specific Authentication and Authorization failure or revocation. |
| "PDU_SESSION_HAND_OVER_FAILURE" | Failure to handover PDU session to another access |
| "DDN_FAILURE_STATUS" | DDN failure status reporting |
| "REL_DUE_TO_CP_ONLY_NOT_APPLICABLE" | Release due to Control Plane Only indication associated with PDU Session is not applicable any longer |
| "NOT_SUPPORTED_WITH_ISMF" | PDU session release due to a requested functionality that is not supported for a PDU session with an I-SMF/V-SMF. |
| "CHANGED_ANCHOR_SMF" | The anchor SMF of the PDU session is changed. |
| "CHANGED_INTERMEDIATE_SMF" | The intermediate SMF (e.g. I-SMF or V-SMF) is changed. |
| "TARGET_DNAI_NOTIFICATION" | Notify the target DNAI for I-SMF selection for the current PDU Session, or SMF selection during PDU Session re-establishment for SSC mode 2/3. |

##### 6.1.6.3.9	Enumeration: ResourceStatus

The enumeration ResourceStatus indicates the status of an SM context or PDU session resource. It shall comply with the provisions defined in table 6.1.6.3.9-1.

Table 6.1.6.3.9-1: Enumeration ResourceStatus

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "RELEASED" | The SM context or PDU session resource is released. |  |
| "UNCHANGED" | The status of SM context or PDU session resource is not changed. |  |
| "TRANSFERRED" | The SM context shall be transferred. |  |
| "UPDATED" | The SM context or PDU session resource context has changed: - The access type of PDU session is changed. - The anchor SMF (H-SMF or SMF) has changed within the SMF set. | HOFAIL ES3XX |
| "ALT_ANCHOR_SMF" | The V-SMF has established the PDU session towards an alternative H-SMF during PDU session establishment for HR PDU session, as specified in clause 4.3.2.2.2 of 3GPP TS 23.502 [3], or the I-SMF has established the PDU session towards an alternative SMF during PDU session establishment for a PDU session with I-SMF (see clause 4.23.5.1 of 3GPP TS 23.502 [3]). | AASN |

##### 6.1.6.3.10	Enumeration: DnnSelectionMode

The enumeration DnnSelectionMode indicates whether the DNN of a PDU session being established corresponds to an explicitly subscribed DNN or to the usage of a wildcard subscription. It shall comply with the provisions defined in table 6.1.6.3.10-1.

Table 6.1.6.3.10-1: Enumeration DnnSelectionMode

| Enumeration value | Description |
| --- | --- |
| "VERIFIED" | UE or network provided DNN is authorized based on the explicitly subscribed DNN, subscription verified |
| "UE_DNN_NOT_VERIFIED" | UE provided DNN is authorized based on the wildcard DNN, subscription not verified |
| "NW_DNN_NOT_VERIFIED" | Network provided DNN is authorized based on the wildcard DNN, subscription not verified |

##### 6.1.6.3.11	Enumeration: EpsInterworkingIndication

The enumeration EpsInterworkingIndication indicates whether and how the PDU session will possibly be moved to EPS.

Table 6.1.6.3.11-1: Enumeration EpsInterworkingIndication

| Enumeration value | Description |
| --- | --- |
| "NONE" | The PDU session cannot be moved EPS. (NOTE) |
| "WITH_N26" | The PDU session may possibly be moved to EPS, with N26 interface supported during EPS interworking procedures. This may correspond to: - a PDU session or an MA-PDU session with a 3GPP access; or - a MA PDU Session with a non-3GPP access for a UE registered to the same PLMN over both 3GPP and non-3GPP accesses, i.e. served by the same AMF for both accesses. |
| "WITHOUT_N26" | The PDU session may possibly be moved to EPS, without N26 interface supported during EPS interworking procedures. This may correspond to: - a PDU session or an MA-PDU session with a 3GPP access; or - a MA PDU Session with a non-3GPP access for a UE registered to the same PLMN over both 3GPP and non-3GPP accesses, i.e. served by the same AMF for both accesses. |
| "IWK_NON_3GPP" | The PDU session via non-3GPP access may possibly be moved to EPS. |
| NOTE: Handover from 5GS to EPC/ePDG (as specified in clause 4.11.4.2 of 3GPP TS 23.502 [3]) shall be considered as allowed if the value of EpsInterworkingIndication is not set to "NONE" and if such handover is allowed based on operator's policy. |  |

##### 6.1.6.3.12	Enumeration: N2SmInfoType

Table 6.1.6.3.12-1: Enumeration N2SmInfoType

| Enumeration value | Description |
| --- | --- |
| "PDU_RES_SETUP_REQ" | PDU Session Resource Setup Request Transfer |
| "PDU_RES_SETUP_RSP" | PDU Session Resource Setup Response Transfer |
| "PDU_RES_SETUP_FAIL" | PDU Session Resource Setup Unsuccessful Transfer |
| "PDU_RES_REL_CMD" | PDU Session Resource Release Command Transfer |
| "PDU_RES_REL_RSP" | PDU Session Resource Release Response Transfer |
| "PDU_RES_MOD_REQ" | PDU Session Resource Modify Request Transfer |
| "PDU_RES_MOD_RSP" | PDU Session Resource Modify Response Transfer |
| "PDU_RES_MOD_FAIL" | PDU Session Resource Modify Unsuccessful Transfer |
| "PDU_RES_NTY" | PDU Session Resource Notify Transfer |
| "PDU_RES_NTY_REL" | PDU Session Resource Notify Released Transfer |
| "PDU_RES_MOD_IND" | PDU Session Resource Modify Indication Transfer |
| "PDU_RES_MOD_CFM" | PDU Session Resource Modify Confirm Transfer |
| "PATH_SWITCH_REQ" | Path Switch Request Transfer |
| "PATH_SWITCH_SETUP_FAIL" | Path Switch Request Setup Failed Transfer |
| "PATH_SWITCH_REQ_ACK" | Path Switch Request Acknowledge Transfer |
| "PATH_SWITCH_REQ_FAIL" | Path Switch Request Unsuccessful Transfer |
| "HANDOVER_REQUIRED" | Handover Required Transfer |
| "HANDOVER_CMD" | Handover Command Transfer |
| "HANDOVER_PREP_FAIL" | Handover Preparation Unsuccessful Transfer |
| "HANDOVER_REQ_ACK" | Handover Request Acknowledge Transfer |
| "HANDOVER_RES_ALLOC_FAIL" | Handover Resource Allocation Unsuccessful Transfer |
| "SECONDARY_RAT_USAGE" | Secondary RAT Data Usage Report Transfer |
| "PDU_RES_MOD_IND_FAIL" | PDU Session Resource Modify Indication Unsuccessful Transfer |

##### 6.1.6.3.13	Enumeration: MaxIntegrityProtectedDataRate

Table 6.1.6.3.13-1: Enumeration MaxIntegrityProtectedDataRate

| Enumeration value | Description |
| --- | --- |
| "64_KBPS" | 64 kbps |
| "MAX_UE_RATE" | Full data rate |

##### 6.1.6.3.14	Enumeration: MaReleaseIndication

The enumeration MaReleaseIndication indicates the access type over which the MA PDU session is requested to be released.

Table 6.1.6.3.14-1: Enumeration MaReleaseIndication

| Enumeration value | Description |
| --- | --- |
| "REL_MAPDU_OVER_3GPP" | The MA PDU session over 3GPP access is to be released or has been released. |
| "REL_MAPDU_OVER_N3GPP" | The MA PDU session over Non-3GPP access is to be released or has been released. |

##### 6.1.6.3.15	Enumeration: SmContextType

The enumeration SmContextType represents the type of SM context information requested during a Retrieve SM Context service operation. It shall comply with the provisions defined in table 6.1.6.3.15-1.

Table 6.1.6.3.15-1: Enumeration SmContextType

| Enumeration value | Description |
| --- | --- |
| "EPS_PDN_CONNECTION" | UE EPS PDN Connection. |
| "SM_CONTEXT" | Complete SM Context (i.e. 5G SM context including EPS context information as defined in clause 6.1.6.2.39) |
| "AF_COORDINATION_INFO" | AF Coordination Information |

##### 6.1.6.3.16	Enumeration: PsaIndication

The enumeration PsaIndication indicates whether a PSA and an UL CL or BP, or only a PSA has been inserted or removed to/from the data path of a PDU session by an I-SMF. It shall comply with the provisions defined in table 6.1.6.3.16-1.

Table 6.1.6.3.16-1: Enumeration PsaIndication

| Enumeration value | Description |
| --- | --- |
| "PSA_INSERTED" | A PSA and UL CL or BP has been inserted into the data path of the PDU session. |
| "PSA_REMOVED" | A PSA and UL CL or BP has been removed from the data path of the PDU session. |
| "PSA_INSERTED_ONLY" | A PSA has been inserted into the data path of the PDU session and the UL CL or BP is not changed. |
| "PSA_REMOVED_ONLY" | A PSA has been removed from the data path of the PDU session and the UL CL or BP is not changed. |

##### 6.1.6.3.17	Enumeration: N4MessageType

The enumeration N4MessageType indicates the PFCP message type sent within a N4 Message Payload. It shall comply with the provisions defined in table 6.1.6.3.17-1.

Table 6.1.6.3.17-1: Enumeration N4MessageType

| Enumeration value | Description |
| --- | --- |
| "PFCP_SES_EST_REQ" | PFCP Session Establishment Request |
| "PFCP_SES_EST_RSP" | PFCP Session Establishment Response |
| "PFCP_SES_MOD_REQ" | PFCP Session Modification Request |
| "PFCP_SES_MOD_RSP" | PFCP Session Modification Response |
| "PFCP_SES_DEL_REQ" | PFCP Session Deletion Request |
| "PFCP_SES_DEL_RSP" | PFCP Session Deletion Response |
| "PFCP_SES_REP_REQ" | PFCP Session Report Request |
| "PFCP_SES_REP_RSP" | PFCP Session Report Response |

##### 6.1.6.3.18	Enumeration: QosFlowAccessType

The enumeration QoSFlowAccessType indicates the access type which the QoS Flow is associated with.

Table 6.1.6.3.18-1: Enumeration QosFlowAccessType

| Enumeration value | Description |
| --- | --- |
| "3GPP" | The QoS Flow is only associated with 3GPP access. |
| "NON_3GPP" | The QoS Flow is only associated with Non-3GPP access. |
| "3GPP_AND_NON_3GPP" | The QoS Flow is associated with both 3GPP access and Non-3GPP access. |

##### 6.1.6.3.19	Enumeration: UnavailableAccessIndication

The enumeration UnavailableAccessIndication indicates the access type of the MA PDU session that is unavailable.

Table 6.1.6.3.19-1: Enumeration UnavailableAccessIndication

| Enumeration value | Description |
| --- | --- |
| "3GA_UNAVAILABLE" | The 3GPP access of the MA PDU session is unavailable. |
| "N3GA_UNAVAILABLE" | The Non-3GPP access of the MA PDU session is unavailable. |

##### 6.1.6.3.20	Enumeration: ProtectionResult

The enumeration ProtectionResult indicates whether the security policy indicated as "preferred" is performed or not.

Table 6.1.6.3.20-1: Enumeration ProtectionResult

| Enumeration value | Description |
| --- | --- |
| "PERFORMED" | The security policy indicated as "preferred" is performed |
| "NOT_PERFORMED" | The security policy indicated as "preferred" is not performed |

##### 6.1.6.3.21	Enumeration: QosMonitoringReq

The enumeration QosMonitoringReq indicates the measurement of UL, or DL, or both UL/DL delays, or no measurements are required.

Table 6.1.6.3.21-1: Enumeration QosMonitoringReq

| Enumeration value | Description |
| --- | --- |
| "UL" | Measurement of UL delay. |
| "DL" | Measurement of DL delay. |
| "BOTH" | Measurement of both UL/DL delays. |
| "NONE" | No measurements are required. This value shall be used to stop on-going UL and/or DL measurements. |

##### 6.1.6.3.22	Enumeration: Rsn

The enumeration Rsn indicates the RSN value which differentiates the PDU sessions that are handled redundantly (see clause 5.33.2.1 of 3GPP TS 23.501 [2]).

Table 6.1.6.3.22-1: Enumeration Rsn

| Enumeration value | Description |
| --- | --- |
| "V1" | V1 |
| "V2" | V2 |
| "NONE" | This value indicates that no RSN value is available (NOTE) |
| NOTE: This value shall be used in a Create Request if a PDU Session Pair ID was received from the UE without an RSN value. |  |

##### 6.1.6.3.23	Enumeration: SmfSelectionType

The enumeration SmfSelectionType represents whether the target DNAI is used for I-SMF selection for the current PDU Session, or for SMF selection during PDU Session re-establishment for SSC mode 2/3. It shall comply with the provisions defined in table 6.1.6.3.23-1.

Table 6.1.6.3.23-1: Enumeration SmfSelectionType

| Enumeration value | Description |
| --- | --- |
| "CURRENT_PDU_SESSION" | The target DNAI is used for I-SMF selection for the current PDU Session. |
| "NEXT_PDU_SESSION" | The target DNAI is used for SMF selection for the next PDU Session, i.e. the re-established PDU session for SSC mode 2/3. |

#### 6.1.6.4	Binary data

##### 6.1.6.4.1	Introduction

This clause defines the binary data that shall be supported in a binary body part in an HTTP multipart message (see clauses 6.1.2.2.2 and 6.1.2.4).

Table 6.1.6.4.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |
| N1 SM Message | 6.1.6.4.2 | vnd.3gpp.5gnas |
| N2 SM Information | 6.1.6.4.3 | vnd.3gpp.ngap |
| n1SmInfoFromUe | 6.1.6.4.4 | vnd.3gpp.5gnas |
| n1SmInfoToUe | 6.1.6.4.4 | vnd.3gpp.5gnas |
| unknownN1SmInfo | 6.1.6.4.4 | vnd.3gpp.5gnas |
| N4 Message Payload | 6.1.6.4.5 | vnd.3gpp.pfcp |

##### 6.1.6.4.2	N1 SM Message

N1 SM Message shall encode a 5GS NAS SM message as specified in 3GPP TS 24.501 [7], using the vnd.3gpp.5gnas content-type.

N1 SM Message may encode any 5GS NAS SM message specified in 3GPP TS 24.501 [7], as summarized in Table 6.1.6.4.2-1.

Table 6.1.6.4.2-1: N1 SM Message content

| 5GS NAS message | Reference (3GPP TS 24.501 [7]) |
| --- | --- |
| PDU session establishment request | 8.3.1 |
| PDU session establishment accept | 8.3.2 |
| PDU session establishment reject | 8.3.3 |
| PDU session authentication command | 8.3.4 |
| PDU session authentication complete | 8.3.5 |
| PDU session authentication result | 8.3.6 |
| PDU session modification request | 8.3.7 |
| PDU session modification reject | 8.3.8 |
| PDU session modification command | 8.3.9 |
| PDU session modification complete | 8.3.10 |
| PDU session modification command reject | 8.3.11 |
| PDU session release request | 8.3.12 |
| PDU session release reject | 8.3.13 |
| PDU session release command | 8.3.14 |
| PDU session release complete | 8.3.15 |
| 5GSM status | 8.3.16 |

##### 6.1.6.4.3	N2 SM Information

N2 SM Information shall encode NG Application Protocol (NGAP) IEs, as specified in clause 9.3 of 3GPP TS 38.413 [9] (ASN.1 encoded), using the vnd.3gpp.ngap content-type.

N2 SM Information may encode any NGAP SMF related IE specified in 3GPP TS 38.413 [9], as summarized in Table 6.1.6.4.3-1.

Table 6.1.6.4.3-1: N2 SM Information content

| N2 SM IE | Reference (3GPP TS 38.413 [9]) | Related NGAP message |
| --- | --- | --- |
| PDU Session Resource Setup Request Transfer | 9.3.4.1 | PDU SESSION RESOURCE SETUP REQUEST INITIAL CONTEXT SETUP REQUEST HANDOVER REQUEST |
| PDU Session Resource Setup Response Transfer | 9.3.4.2 | PDU SESSION RESOURCE SETUP RESPONSE INITIAL CONTEXT SETUP RESPONSE |
| PDU Session Resource Setup Unsuccessful Transfer | 9.3.4.16 | PDU SESSION RESOURCE SETUP RESPONSE INITIAL CONTEXT SETUP RESPONSE |
| PDU Session Resource Release Command Transfer | 9.3.4.12 | PDU SESSION RESOURCE RELEASE COMMAND |
| PDU Session Resource Release Response Transfer | 9.3.4.21 | PDU SESSION RESOURCE RELEASE RESPONSE |
| PDU Session Resource Modify Request Transfer | 9.3.4.3 | PDU SESSION RESOURCE MODIFY REQUEST |
| PDU Session Resource Modify Response Transfer | 9.3.4.4 | PDU SESSION RESOURCE MODIFY RESPONSE |
| PDU Session Resource Modify Unsuccessful Transfer | 9.3.4.17 | PDU SESSION RESOURCE MODIFY RESPONSE |
| PDU Session Resource Notify Transfer | 9.3.4.5 | PDU SESSION RESOURCE NOTIFY |
| PDU Session Resource Notify Released Transfer | 9.3.4.13 | PDU SESSION RESOURCE NOTIFY |
| PDU Session Resource Modify Indication Transfer | 9.3.4.6 | PDU SESSION RESOURCE MODIFY INDICATION |
| PDU Session Resource Modify Confirm Transfer | 9.3.4.7 | PDU SESSION RESOURCE MODIFY CONFIRM |
| PDU Session Resource Modify Indication Unsuccessful Transfer | 9.3.4.22 | PDU SESSION RESOURCE MODIFY CONFIRM |
| Path Switch Request Transfer | 9.3.4.8 | PATH SWITCH REQUEST |
| Path Switch Request Setup Failed Transfer | 9.3.4.15 | PATH SWITCH REQUEST |
| Path Switch Request Acknowledge Transfer | 9.3.4.9 | PATH SWITCH REQUEST ACKNOWLEDGE |
| Path Switch Request Unsuccessful Transfer | 9.3.4.20 | PATH SWITCH REQUEST ACKNOWLEDGE PATH SWITCH REQUEST FAILURE |
| Handover Required Transfer | 9.3.4.14 | HANDOVER REQUIRED |
| Handover Request Acknowledge Transfer | 9.3.4.11 | HANDOVER REQUEST ACKNOWLEDGE |
| Handover Resource Allocation Unsuccessful Transfer | 9.3.4.19 | HANDOVER REQUEST ACKNOWLEDGE |
| Handover Command Transfer | 9.3.4.10 | HANDOVER COMMAND |
| Handover Preparation Unsuccessful Transfer | 9.3.4.18 | HANDOVER COMMAND |
| Secondary RAT Data Usage Report Transfer | 9.3.4.23 | SECONDARY RAT DATA USAGE REPORT |

##### 6.1.6.4.4	n1SmInfoFromUe, n1SmInfoToUe, unknownN1SmInfo

n1SmInfoFromUe, n1SmInfoToUe and unknownN1SmInfo shall encode one or more NAS SM IEs, including the Type and Length fields, as specified in 3GPP TS 24.501 [7], using the vnd.3gpp.5gnas content-type.

Clause 5.2.3.1 specifies the information that shall be included in these payloads.

n1SmInfoFromUe and n1SmInfoToUe may encode the 5GS NAS IEs listed in tables 6.1.6.4.4-1 and 6.1.6.4.4-2.

Table 6.1.6.4.4-1: n1SmInfoFromUE content

| 5GS NAS IE | Reference (3GPP TS 24.501 [7]) | Related NAS SM message |
| --- | --- | --- |
| Message type | 9.7 | All NAS SM messages |
| PDU session type | 9.11.4.11 | PDU Session Establishment Request |
| SSC mode | 9.11.4.16 | PDU Session Establishment Request |
| Maximum number of supported packet filters | 9.11.4.9 | PDU Session Establishment Request PDU Session Modification Request |
| Integrity protection maximum data rate | 9.11.4.7 | PDU Session Modification Request (NOTE 3) |
| SM PDU DN request container | 9.11.4.15 | PDU Session Establishment Request |
| Extended protocol configuration options | 9.11.4.6 | PDU Session Establishment Request PDU Session Authentication Complete PDU Session Modification Request PDU Session Modification Complete PDU Session Modification Command Reject PDU Session Release Request PDU Session Release Complete |
| EAP message | 9.11.2.2 | PDU Session Authentication Complete |
| Requested QoS rules | 9.11.4.13 | PDU Session Modification Request |
| Requested QoS flow descriptions | 9.11.4.12 | PDU Session Modification Request |
| 5GSM cause | 9.11.4.2 | PDU Session Modification Request PDU Session Release Request PDU Session Release Complete (NOTE 2) |
| 5GSM capability | 9.11.4.1 | PDU Session Establishment Request PDU Session Modification Request (NOTE 1) |
| Mapped EPS bearer contexts | 9.11.4.8 | PDU Session Modification Request |
| NOTE 1: The 5GSM capability IE shall be encoded as received from the UE. It may contain UE capabilities that the V-SMF (or I-SMF) only needs to transfer to the H-SMF (or SMF), e.g. support of reflective QoS, or support of multi-homed IPv6 PDU session, and/or capabilities to be interpreted and used by the V-SMF (or I-SMF). NOTE 2: The 5GSM cause IE shall be encoded as received from the UE. This information is defined as a "V" IE (i.e. without a Type field) in other NAS messages, e.g. PDU Session Modification Command Reject message, in which case it shall be sent as a separate n1SmCause IE over N16/N16a and not within the n1SmInfoToUE binary data. NOTE 3: This information is defined as a "V" IE (i.e. without a Type field) in other NAS messages, e.g. PDU Session Establishment Request, in which case it shall be sent as separate maximum integrity protected data rate IEs over N16/N16a and not within the n1SmInfoToUE binary data. |  |  |

Table 6.1.6.4.4-2: n1SmInfoToUE parameters

| 5GS NAS IE | Reference (3GPP TS 24.501 [7]) | Related NAS SM message |
| --- | --- | --- |
| Message type | 9.7 | All NAS SM messages |
| RQ timer value | 9.11.2.3 | PDU Session Establishment Accept PDU Session Modification Command |
| EAP message | 9.11.2.2 | PDU Session Establishment Accept PDU Session Establishment Reject PDU Session Authentication Command PDU Session Authentication Result PDU Session Release Command |
| Allowed SSC mode | 9.11.4.5 | PDU Session Establishment Reject |
| Extended protocol configuration options | 9.11.4.6 | PDU Session Establishment Accept PDU Session Establishment Reject PDU Session Authentication Command PDU Session Authentication Result PDU Session Modification Reject PDU Session Modification Command PDU Session Release Reject PDU Session Release Command |
| 5GSM cause | 9.11.4.2 | PDU Session Establishment Accept PDU Session Modification Command (NOTE) |
| Mapped EPS bearer contexts | 9.11.4.8 | PDU Session Establishment Accept PDU Session Modification Command |
| ATSSS container | 9.11.4.22 | PDU Session Establishment Accept PDU Session Modification Command |
| NOTE: This IE indicates the 5GSM cause the H-SMF (or SMF) requires the V-SMF (or I-SMF) to send to the UE. The V-SMF (or I-SMF) shall transfer the received value to the UE without interpretation. This information is defined as a "V" IE (i.e. without a Type field) in other NAS messages, e.g. PDU Session Establishment Reject message, in which case it shall be sent as a separate n1SmCause IE over N16/N16a and not within the n1SmInfoToUE binary data. |  |  |

The Message Type shall be present and encoded as the first 5GS NAS IE in any n1SmInfoFromUe, n1SmInfoToUe and unknownN1SmInfo binary data, to enable the receiver to decode the 5GS NAS IEs.

NOTE:	The Information Element Identifier (see clause 11.2.1.1.3 of 3GPP TS 24.007 [8]) of a 5GS NAS IE uniquely identifies an IE in a given message.

##### 6.1.6.4.5	N4 Message Payload

The N4 Message Payload shall encode a PFCP session related message as specified in 3GPP TS 29.244 [29], using the vnd.3gpp.pfcp content-type.

##### 6.1.6.4.6	Mobile Originated Data

Mobile Originated Data shall encode the Data Contents of the CIoT small data container or Payload Container specified in 3GPP TS 24.501 [7], using the vnd.3gpp.5gnas content-type, as summarized in Table 6.1.6.4.6-1.

Table 6.1.6.4.6-1: Mobile Originated Data

| Mobile Originated Data | Reference (3GPP TS 24.501 [7]) | Related NAS SM message |
| --- | --- | --- |
| Data contents in octet 4 to octet up to 257 of CIoT small data container contents. | 9.11.3.18B (Figure 9.11.3.18B.2) | Control Plane Service Request |
| Payload container contents in octets 4 to n | 9.11.3.39 (Figure 9.11.3.39.1) | Control Plane Service Request UL NAS Transport |

##### 6.1.6.4.7	Mobile Terminated Data

Mobile Terminated Data shall encode the Data Contents of the Payload Container specified in 3GPP TS 24.501 [7], using the vnd.3gpp.5gnas content-type, as summarized in Table 6.1.6.4.7-1.

Table 6.1.6.4.7-1: Mobile Terminated Data

| Mobile Terminated Data | Reference (3GPP TS 24.501 [7]) | Related NAS SM message |
| --- | --- | --- |
| Payload container contents in octets 4 to n | 9.11.3.39 (Figure 9.11.3.39.1) | DL NAS Transport |

### 6.1.7	Error Handling

#### 6.1.7.1	General

HTTP error handling shall be supported as specified in clause 5.2.4 of 3GPP TS 29.500 [4].

The Cause codes mapping performed by AMF between the following HTTP responses returned by the SMF services to the AMF and the 5GMM related values is specified in clause 4.3.2 of 3GPP TS 29.524 [34].

In order to enable the AMF to figure out whether a request (e.g. PDU session establishment request) fails at the V-SMF/I-SMF or at the H-SMF/SMF for a HR PDU session or a PDU session with an I-SMF:

-	a V-SMF/I-SMF that returns an HTTP error response to the AMF that was originated by the H-SMF/SMF shall include the remoteError attribute set to "true" in the ProblemDetails information in the HTTP error response;

-	a V-SMF/I-SMF that originates an error to the AMF may include the remoteError attribute set to "false" in the ProblemDetails information in the HTTP error response.

For a HR PDU session or a PDU session with an I-SMF,  if the V-SMF or I-SMF needs to reject the request from the AMF or the H-SMF/SMF because the H-SMF/SMF or the AMF is not reachable respectively (even after retrying alternative endpoint addresses e.g. according to the Binding Indication when available), the V-SMF or I-SMF shall send a 504 Gateway Timeout response including a problemDetails data structure with the cause attribute set to "PEER_NOT_RESPONDING" and with the remoteError attribute set to "false".

#### 6.1.7.2	Protocol Errors

Protocol errors handling shall be supported as specified in clause 5.2.7 of 3GPP TS 29.500 [4].

#### 6.1.7.3	Application Errors

The common application errors defined in Table 5.2.7.2-1 of 3GPP TS 29.500 [4] may be used for the Nsmf_PDUSession service.

The following application errors listed in Table 6.1.7.3-1 are specific to the Nsmf_PDUSession service.

Table 6.1.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| N1_SM_ERROR | 403 Forbidden | This indicates that an error, other than those listed in this table, was detected when processing the N1 SM information received in the request, e.g. N1 SM protocol error. |
| N2_SM_ERROR | 403 Forbidden | This indicates that an error, other than those listed in this table, was detected when processing the N2 SM information received in the request, e.g. N2 SM protocol error. |
| SNSSAI_DENIED | 403 Forbidden | The subscriber does not have the necessary subscription to access the SNSSAI. |
| DNN_DENIED | 403 Forbidden | The subscriber does not have the necessary subscription to access the DNN. |
| PDUTYPE_DENIED | 403 Forbidden | The subscriber does not have the necessary subscription for the requested PDU session type. |
| SSC_DENIED | 403 Forbidden | The subscriber does not have the necessary subscription for the requested SSC mode. |
| SUBSCRIPTION_DENIED | 403 Forbidden | This indicates an error, other than those listed in this table, due to lack of necessary subscription to serve the UE request. |
| DNN_NOT_SUPPORTED | 403 Forbidden | The DNN is not supported by the SMF. |
| PDUTYPE_NOT_SUPPORTED | 403 Forbidden | The requested PDU session type is not supported by the SMF for the PDN corresponding to the DNN. |
| SSC_NOT_SUPPORTED | 403 Forbidden | The requested SSC mode is not supported by the SMF for the PDN corresponding to the DNN. |
| HOME_ROUTED_ROAMING_REQUIRED | 403 Forbidden | It is used in LBO roaming, if the V-SMF is not able to process some part of the N1 SM information that requires Home Routed Roaming. |
| OUT_OF_LADN_SERVICE_AREA | 403 Forbidden | The PDU session corresponds to a LADN and the UE is outside of the LADN Service Area. |
| N2_SM_ERROR | 403 Forbidden | This indicates that an error, other than those listed in this table, was detected when processing the N2 SM information received in the request, e.g. N2 SM protocol error. |
| PRIORITIZED_SERVICES_ONLY | 403 Forbidden | The SMF was notified that the UE is reachable only for regulatory prioritized service and the PDU Session to be activated is not for a regulatory prioritized service. |
| PDU_SESSION_ANCHOR_CHANGE | 403 Forbidden | The SMF decided to change the PDU Session Anchor for the PDU Session. |
| TARGET_MME_CAPABILITY | 403 Forbidden | A request to retrieve an SM context is rejected due to the target MME not capable to support the PDU session. |
| NO_EPS_5GS_CONTINUITY | 403 Forbidden | It is used during an EPS to 5GS Idle mode mobility or handover, if the PDU session does not support seamless session continuity to 5GS. |
| UNABLE_TO_PAGE_UE | 403 Forbidden | The request is rejected due to a temporarily inability to page the UE. |
| UE_NOT_RESPONDING | 403 Forbidden | The UE did not respond to the request initiated by the network, e.g. paging. |
| REJECTED_BY_UE | 403 Forbidden | The request is rejected by the UE. |
| REJECTED_DUE_VPLMN_POLICY | 403 Forbidden | The request is rejected due to VPLMN operator policy. |
| HO_TAU_IN_PROGRESS | 403 Forbidden | The request is rejected temporarily due to a mobilty procedure in progress. |
| INTEGRITY_PROTECTED_MDR_NOT_ACCEPTABLE | 403 Forbidden | The integrity protected maximum data rate value provided by the UE is not acceptable for the PDU session based on local policy at the SMF. This error is applicable when the UP Security Policy for the PDU Session is determined to have Integrity Protection set to "Required". An NF service consumer that receives this error cause may use it for maintaining KPIs. |
| EBI_EXHAUSTED | 403 Forbidden | The allocation of EPS Bearer ID failed due to exhaustion of EBI as the maximum number of EBIs has already been allocated to the UE. |
| EBI_REJECTED_LOCAL_POLICY | 403 Forbidden | The allocation of EPS Bearer ID was rejected due to local policy in the Serving PLMN. |
| EBI_REJECTED_NO_N26 | 403 Forbidden | The allocation of EPS Bearer ID was rejected when the AMF is in a serving PLMN that does not support 5GS-EPS interworking procedures with N26 interface. |
| DEFAULT_EPS_BEARER_INACTIVE | 403 Forbidden | It is used during EPS to 5GS mobility if the default EPS bearer context of the PDU session is reported as inactive by the UE in the epsBearerCtxStatus attribute. |
| HANDOVER_RESOURCE_ALLOCATION_FAILURE | 403 Forbidden | It is used during a N2 handover preparation or an EPS to 5GS handover preparation, if no resource is allocated by the target NG-RAN for the PDU session. |
| LATE_OVERLAPPING_REQUEST | 403 Forbidden | The request is rejected because it collides with an existing SM context or PDU session context with a more recent origination timestamp (see clause 5.2.3.3). |
| DEFAULT_EBI_NOT_TRANSFERRED | 403 Forbidden | It is used during 5GS to EPS mobility if the EBI of the default EPS bearer is included in the notToTransferEbiList attribute. |
| NOT_SUPPORTED_WITH_ISMF | 403 Forbidden | The request is rejected due to a requested functionality that is not supported for a PDU session with an I-SMF/V-SMF. |
| SERVICE_NOT_AUTHORIZED_BY_NEXT_HOP | 403 Forbidden | The SMF is not authorized to access service provided by next hop NF producer, e.g. H-SMF or SMF or old I-SMF or old V-SMF. |
| NO_DATA_FORWARDING | 403 Forbidden | The request to setup data forwarding tunnels is rejected because none of the EPS bearer contexts received in the request body contains an F-TEID for DL data forwarding. |
| CONTEXT_NOT_FOUND | 404 Not Found | It is used when no context corresponding to the request exists in the SMF. |
| HIGHER_PRIORITY_REQUEST_ONGOING | 409 Conflict | The request is rejected temporarily due to procedure for higher priority session in progress. |
| UE_IN_CM_IDLE_STATE | 409 Conflict | The request is rejected due to the UE being in CM-IDLE state for the PDU session associated to non-3GPP access. |
| INSUFFICIENT_RESOURCES_SLICE | 500 Internal Server Error | The request cannot be provided due to insufficient resources for the specific slice. |
| INSUFFICIENT_RESOURCES_SLICE_DNN | 500 Internal Server Error | The request cannot be provided due to insufficient resources for the specific slice and DNN. |
| DNN_CONGESTION | 503 Service Unavailable | The SMF has detected congestion for the requested DNN and performs overload control for that DNN which does not allow the PDU session to be established. |
| S_NSSAI_CONGESTION | 503 Service Unavailable | The SMF has detected congestion for the requested S-NSSAI and performs overload control for that S-NSSAI which does not allow the PDU session to be established. |
| PEER_NOT_RESPONDING | 504 Gateway Timeout | No response is received from a remote peer, or the remote peer is known to be not reachable, e.g. to indicate that no response has been received from the H-SMF for a HR PDU session or the SMF for a PDU session with I-SMF. |
| NETWORK_FAILURE | 504 Gateway Timeout | The request is rejected due to a network problem. |
| UPF_NOT_RESPONDING | 504 Gateway Timeout | The request is rejected due to no response received from the UPF. |
| UE_NOT_REACHABLE | 504 Gateway Timeout | The UE is not reachable for service. |

### 6.1.8	Feature Negotiation

The feature negotiation mechanism specified in clause 6.6 of 3GPP TS 29.500 [4] shall be used to negotiate the optional features applicable between the SMF and the NF Service Consumer, for the Nsmf_PDUSession service, if any.

The NF Service Consumer shall indicate the optional features it supports for the Nsmf_PDUSession service, if any, by including the supportedFeatures attribute in the HTTP POST request when requesting the SMF to create an SM context or a PDU session resource. In scenarios with a change of NF Service Consumer (e.g. change of AMF, V-SMF or I-SMF change), the new NF Service Consumer shall indicate the optional features it supports for the Nsmf_PDUSession service, if any, by including the supportedFeatures attribute in the HTTP POST request when requesting the SMF to update an SM context or a PDU session resource to change the NF Service Consumer.

The SMF shall determine the supported features for the created SM context or PDU session resource, or for the updated SM context or PDU session resource in scenarios with a change of NF Service Consumer, as specified in clause 6.6 of 3GPP TS 29.500 [4] and shall indicate the supported features by including the supportedFeatures attribute in the representation of the SM context or PDU session resource it returns in the HTTP response confirming the creation or the modification of the resource.

The syntax of the supportedFeatures attribute is defined in clause 5.2.2 of 3GPP TS 29.571 [13].

The following features are defined for the Nsmf_PDUSession service.

Table 6.1.8-1: Features of supportedFeatures attribute used by Nsmf_PDUSession service

| Feature Number | Feature | M/O | Description |
| --- | --- | --- | --- |
| 1 | CIOT | O | Cellular IoT Support of this feature implies the support of all the CIoT features specified in clause 5.31 of 3GPP TS 23.501 [2], including in particular corresponding SMF PDUSession service's extensions to support: - NB-IoT and LTE-M RAT types; - Control Plane CIoT 5GS Optimisation; - Rate control of user data; - Idle mode mobility with data forwarding between EPS and 5GS using N26 interface. The SMF shall indicate its support of this feature in supportedFeatures attribute in its profile registered in NRF. A NF service consumer (e.g. AMF) shall only select SMF(s) that supports this feature for PDU sessions with Control Plane CIoT 5GS Optimisation enabled. |
| 2 | MAPDU | O | Multi-Access PDU Session An SMF that supports this feature shall support the procedures specified in3GPP TS 23.501 [2] and 3GPP TS 23.502 [3] related to Access Traffic Steering, Switching and Splitting. |
| 3 | DTSSA | O | Deployments Topologies with specific SMF Service Areas A NF Service Consumer and an SMF that support this feature shall support the procedures specified in clause 5.34 of 3GPP TS 23.501 [2] and in clause 4.23 of 3GPP TS 23.502 [3]. |
| 4 | CARPT | O | SMF derived CN Assisted RAN parameters Tuning. A NF Service Consumer (e.g. AMF) and an SMF that support this feature shall support exchanging SMF derived CN assisted RAN parameters in Notify SM Context Status service operation (see clause 5.2.2.5.1). |
| 5 | CTXTR | O | This feature bit indicates whether the NF Service Consumer (e.g. AMF) and SMF supports Network Function/NF Service Context Transfer Procedures specified in clause 4.26 of 3GPP TS 23.502 [3]. The SMF shall only trigger these context transfer procedures if the NF Service Consumer has indicated support of this feature. |
| 6 | VQOS | O | VPLMN QoS An SMF that supports this feature shall support: - the handling of QoS constraints from the VPLMN during a HR PDU session establishment as specified in clause 4.3.2.2.2 of 3GPP TS 23.502 [3]; and - QoS modification requests initiated by the VPLMN, as specified in clause 4.3.3.3 of 3GPP TS 23.502 [3]. |
| 7 | HOFAIL | M | This feature bit indicates whether the NF Service Consumer (e.g. AMF, V-SMF, I-SMF) and SMF supports the Notify (SM Context) Status procedure to indicate a handover failure with the ResourceStatus set to "UPDATED" between 3GPP access and non-3GPP access as specified in clauses 5.2.2.5.1 and 5.2.2.10.1. The SMF shall only trigger such a resource status notify procedure if the NF Service Consumer has indicated support of this feature. |
| 8 | ES3XX | M | Extended Support of HTTP 307/308 redirection An NF Service Consumer (e.g. AMF, V-SMF, I-SMF) that supports this feature shall support handling of HTTP 307/308 redirection for any service operation of the PDUSession service. An NF Service Consumer that does not support this feature does only support HTTP redirection as specified for 3GPP Release 15. |
| 9 | DCE2ER | O | Dual Connectivity based end to end Redundant User Plane Paths An NF service consumer (e.g. I-SMF) and SMF that supports this feature shall support the procedures specified in clause 5.33.2.1 of 3GPP TS 23.501 [2]. |
| 10 | AASN | M | This feature bit indicates whether the NF Service Consumer (e.g. AMF) and the SMF support the Notify SM Context Status procedure to indicate that the PDU session is established towards an alternative anchor SMF, as specified in clause 5.2.2.5.1. The SMF shall only trigger such a Notify SM Context Status procedure if the NF Service Consumer has indicated support of this feature. |
| 11 | EEDGE | O | Enhancement of Edge Computing in 5G Core network A NF Service Consumer and an SMF that support this feature shall support to signal the target DNAI in Notify (SM Context) Status and Create SM Context service operations, support to signal the URI of the SM Context resource in Notify SM Context Status, Create SM Context and create service operations to retrieve the AF Coordination Information, as specified in 3GPP TS 23.501 [2], 3GPP TS 23.502 [3] and 3GPP TS 23.548 [39]. |
| 12 | SCPBU | O | Simultaneous Change of PSA and BP or UL CL This feature bit indicates whether the NF Service Consumer (e.g. I-SMF) and the SMF support the n4InfoExt3 IE included in VsmfUpdateData, VsmfUpdatedData or VsmfUpdateError to support the simultaneous change of PSA and BP or UL CL controlled by I-SMF. The SMF shall only include the n4InfoExt3 IE in VsmfUpdateData if the NF Service Consumer has indicated support of this feature. |
| 13 | IVRES | O | This feature bit indicates whether the NF Service Consumer (e.g. AMF, V-SMF, I-SMF) and SMF supports the I-SMF/V-SMF restoration procedure, as specified in clauses 6.7 of 3GPP TS 23.527 [24]. The I-SMF/V-SMF restoration procedure shall only be triggered if the AMF and the anchor SMF have indicated support of this feature. |
| Feature number: The order number of the feature within the supportedFeatures attribute (starting with 1). Feature: A short name that can be used to refer to the bit and to the feature. M/O: Defines if the implementation of the feature is mandatory ("M") or optional ("O"). Description: A clear textual description of the feature. |  |  |  |

### 6.1.9	Security

As indicated in 3GPP TS 33.501 [17] and 3GPP TS 29.500 [4], the access to the Nsmf_PDUSession API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [18]), based on local configuration, using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [19]) plays the role of the authorization server.

If OAuth2 authorization is used, an NF Service Consumer, prior to consuming services offered by the Nsmf_PDUSession API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [19], clause 5.4.2.2.

NOTE 1:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Nsmf_PDUSession service.

NOTE 2:	The security credentials for accessing a child resource URI of an sm-contexts or pdu-sessions collection distributed on different processing instances or hosts are the same as for accessing the collection URI.

The Nsmf_PDUSession API defines a single scope "nsmf-pdusession" for the entire service, and it does not define any additional scopes at resource and operation level.

### 6.1.10	HTTP redirection

An HTTP request may be redirected to a different SMF service instance, within the same SMF or a different SMF of an SMF set, e.g. when an SMF service instance is part of an SMF (service) set or when using indirect communications (see 3GPP TS 29.500 [4]). See also the ES3XX feature in clause 6.1.8.

An SCP that reselects a different SMF producer instance will return the NF Instance ID of the new SMF producer instance in the 3gpp-Sbi-Producer-Id header, as specified in clause 6.10.3.4 of 3GPP TS 29.500 [4].

If an SMF within an SMF set redirects a service request to a different SMF of the set using an 307 Temporary Redirect or 308 Permanent Redirect status code, the identity of the new SMF towards which the service request is redirected shall be indicated in the 3gpp-Sbi-Target-Nf-Id header of the 307 Temporary Redirect or 308 Permanent Redirect response as specified in clause 6.10.9.1 of 3GPP TS 29.500 [4].

For a HR PDU session or a PDU session with an I-SMF, the V-SMF or I-SMF shall update the AMF upon detecting a change of the H-SMF or SMF within the SMF set, by sending an SM context status notification with the resourceStatus set to "UPDATED" and with the new H-SMF or SMF identity.

NOTE:	This allows the AMF to receive the NF Instance ID of the new SMF handling the PDU session and to send the NF Instance ID of the last known SMF handling the PDU session during inter-AMF mobility.

Annex A (normative):
OpenAPI specification

## A.1	General

This Annex specifies the formal definition of the Nsmf_PDUSession service. It consists of OpenAPI 3.0.0 specifications, in YAML format.

This Annex takes precedence when being discrepant to other parts of the specification with respect to the encoding of information elements and methods within the API(s).

NOTE:	The semantics and procedures, as well as conditions, e.g. for the applicability and allowed combinations of attributes or values, not expressed in the OpenAPI definitions but defined in other parts of the specification also apply.

Informative copies of the OpenAPI specification files contained in this 3GPP Technical Specification are available on a Git-based repository, that uses the GitLab software version control system (see 3GPP TS 29.501 [5] clause 5.3.1 and 3GPP TR 21.900 [7] clause 5B).

## A.2	Nsmf_PDUSession API

openapi: 3.0.0

info:

version: '1.2.0-alpha.3'

title: 'Nsmf_PDUSession'

description: |

SMF PDU Session Service.

© 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.502 V17.2.0; 5G System; Session Management Services; Stage 3

url: http://www.3gpp.org/ftp/Specs/archive/29_series/29.502/

servers:

- url: '{apiRoot}/nsmf-pdusession/v1'

variables:

apiRoot:

default: https://example.com

description:  apiRoot as defined in clause 4.4 of 3GPP TS 29.501. The sm-contexts and pdu-sessions resources can be distributed on different processing instances or hosts. Thus the authority and/or deployment-specific string of the apiRoot of the created individual sm context and pdu-session resources' URIs may differ from the authority and/or deployment-specific string of the apiRoot of the sm-contexts and pdu-sessions collections' URIs.

security:

- {}

- oAuth2ClientCredentials:

- nsmf-pdusession

paths:

/sm-contexts:

post:

summary:  Create SM Context

tags:

- SM contexts collection

operationId: PostSmContexts

requestBody:

description: representation of the SM context to be created in the SMF

required: true

content:

multipart/related:  # message with binary body part(s)

schema:

type: object

properties: # Request parts

jsonData:

$ref: '#/components/schemas/SmContextCreateData'

binaryDataN1SmMessage:

type: string

format: binary

binaryDataN2SmInformation:

type: string

format: binary

binaryDataN2SmInformationExt1:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataN1SmMessage:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

binaryDataN2SmInformation:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

binaryDataN2SmInformationExt1:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

callbacks:

smContextStatusNotification:

'{$request.body#/smContextStatusUri}':

post:

requestBody:  # contents of the callback message

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SmContextStatusNotification'

responses:

'204':

description: successful notification

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

responses:

'201':

description: successful creation of an SM context

content:

application/json: # message without binary body part

schema:

$ref: '#/components/schemas/SmContextCreatedData'

multipart/related:  # message with binary body part(s)

schema:

type: object

properties: # Request parts

jsonData:

$ref: '#/components/schemas/SmContextCreatedData'

binaryDataN2SmInformation:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataN2SmInformation:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/nsmf-pdusession/<apiVersion>/sm-contexts/{smContextRef}'

required: true

schema:

type: string

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

description: unsuccessful creation of an SM context - bad request

content:

application/json: # message without binary body part

schema:

$ref: '#/components/schemas/SmContextCreateError'

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

multipart/related:  # message with binary body part(s)

schema:

type: object

properties: # Request parts

jsonData:

$ref: '#/components/schemas/SmContextCreateError'

binaryDataN1SmMessage:

type: string

format: binary

binaryDataN2SmMessage:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataN1SmMessage:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

binaryDataN2SmMessage:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

'403':

description: unsuccessful creation of an SM context - forbidden

content:

application/json: # message without binary body part

schema:

$ref: '#/components/schemas/SmContextCreateError'

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

multipart/related:  # message with binary body part(s)

schema:

type: object

properties: # Request parts

jsonData:

$ref: '#/components/schemas/SmContextCreateError'

binaryDataN1SmMessage:

type: string

format: binary

binaryDataN2SmMessage:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataN1SmMessage:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

binaryDataN2SmMessage:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

'404':

description: unsuccessful creation of an SM context - not found

content:

application/json: # message without binary body part

schema:

$ref: '#/components/schemas/SmContextCreateError'

multipart/related:  # message with binary body part(s)

schema:

type: object

properties: # Request parts

jsonData:

$ref: '#/components/schemas/SmContextCreateError'

binaryDataN1SmMessage:

type: string

format: binary

binaryDataN2SmMessage:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataN1SmMessage:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

binaryDataN2SmMessage:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: '#/components/responses/413'

'415':

$ref: '#/components/responses/415'

'429':

$ref: '#/components/responses/429'

'500':

description: unsuccessful creation of an SM context - internal server error

content:

application/json: # message without binary body part

schema:

$ref: '#/components/schemas/SmContextCreateError'

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

multipart/related:  # message with binary body part(s)

schema:

type: object

properties: # Request parts

jsonData:

$ref: '#/components/schemas/SmContextCreateError'

binaryDataN1SmMessage:

type: string

format: binary

binaryDataN2SmMessage:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataN1SmMessage:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

binaryDataN2SmMessage:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

'503':

description: unsuccessful creation of an SM context - service unavailable

content:

application/json: # message without binary body part

schema:

$ref: '#/components/schemas/SmContextCreateError'

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

multipart/related:  # message with binary body part(s)

schema:

type: object

properties: # Request parts

jsonData:

$ref: '#/components/schemas/SmContextCreateError'

binaryDataN1SmMessage:

type: string

format: binary

binaryDataN2SmMessage:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataN1SmMessage:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

binaryDataN2SmMessage:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

'504':

description: unsuccessful creation of an SM context - gateway timeout

content:

application/json: # message without binary body part

schema:

$ref: '#/components/schemas/SmContextCreateError'

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

multipart/related:  # message with binary body part(s)

schema:

type: object

properties: # Request parts

jsonData:

$ref: '#/components/schemas/SmContextCreateError'

binaryDataN1SmMessage:

type: string

format: binary

binaryDataN2SmMessage:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataN1SmMessage:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

binaryDataN2SmMessage:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/sm-contexts/{smContextRef}/retrieve:

post:

summary:  Retrieve SM Context

tags:

- Individual SM context

operationId: RetrieveSmContext

parameters:

- name: smContextRef

in: path

description:  SM context reference

required: true

schema:

type: string

requestBody:

description: parameters used to retrieve the SM context

required: false

content:

application/json:

schema:

$ref: '#/components/schemas/SmContextRetrieveData'

responses:

'200':

description: successful retrieval of an SM context

content:

application/json:

schema:

$ref: '#/components/schemas/SmContextRetrievedData'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/sm-contexts/{smContextRef}/modify:

post:

summary:  Update SM Context

tags:

- Individual SM context

operationId: UpdateSmContext

parameters:

- name: smContextRef

in: path

description:  SM context reference

required: true

schema:

type: string

requestBody:

description: representation of the updates to apply to the SM context

required: true

content:

application/json: # message without binary body part

schema:

$ref: '#/components/schemas/SmContextUpdateData'

multipart/related:  # message with binary body part(s)

schema:

type: object

properties: # Request parts

jsonData:

$ref: '#/components/schemas/SmContextUpdateData'

binaryDataN1SmMessage:

type: string

format: binary

binaryDataN2SmInformation:

type: string

format: binary

binaryDataN2SmInformationExt1:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataN1SmMessage:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

binaryDataN2SmInformation:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

binaryDataN2SmInformationExt1:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

responses:

'200':

description: successful update of an SM context with content in the response

content:

application/json: # message without binary body part

schema:

$ref: '#/components/schemas/SmContextUpdatedData'

multipart/related:  # message with binary body part(s)

schema:

type: object

properties: # Request parts

jsonData:

$ref: '#/components/schemas/SmContextUpdatedData'

binaryDataN1SmMessage:

type: string

format: binary

binaryDataN2SmInformation:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataN1SmMessage:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

binaryDataN2SmInformation:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

'204':

description: successful update of an SM context without content in the response

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

description: unsuccessful update of an SM context - bad request

content:

application/json: # message without binary body part

schema:

$ref: '#/components/schemas/SmContextUpdateError'

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

multipart/related:  # message with binary body part(s)

schema:

type: object

properties: # Request parts

jsonData:

$ref: '#/components/schemas/SmContextUpdateError'

binaryDataN1SmMessage:

type: string

format: binary

binaryDataN2SmInformation:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataN1SmMessage:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

binaryDataN2SmInformation:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

'403':

description: unsuccessful update of an SM context - forbidden

content:

application/json: # message without binary body part

schema:

$ref: '#/components/schemas/SmContextUpdateError'

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

multipart/related:  # message with binary body part(s)

schema:

type: object

properties: # Request parts

jsonData:

$ref: '#/components/schemas/SmContextUpdateError'

binaryDataN1SmMessage:

type: string

format: binary

binaryDataN2SmInformation:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataN1SmMessage:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

binaryDataN2SmInformation:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

'404':

description: unsuccessful update of an SM context - not found

content:

application/json: # message without binary body part

schema:

$ref: '#/components/schemas/SmContextUpdateError'

multipart/related:  # message with binary body part(s)

schema:

type: object

properties: # Request parts

jsonData:

$ref: '#/components/schemas/SmContextUpdateError'

binaryDataN1SmMessage:

type: string

format: binary

binaryDataN2SmInformation:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataN1SmMessage:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

binaryDataN2SmInformation:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: '#/components/responses/413'

'415':

$ref: '#/components/responses/415'

'429':

$ref: '#/components/responses/429'

'500':

description: unsuccessful update of an SM context - Internal server error

content:

application/json: # message without binary body part

schema:

$ref: '#/components/schemas/SmContextUpdateError'

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

multipart/related:  # message with binary body part(s)

schema:

type: object

properties: # Request parts

jsonData:

$ref: '#/components/schemas/SmContextUpdateError'

binaryDataN1SmMessage:

type: string

format: binary

binaryDataN2SmInformation:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataN1SmMessage:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

binaryDataN2SmInformation:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

'503':

description: unsuccessful update of an SM context - Service Unavailable

content:

application/json: # message without binary body part

schema:

$ref: '#/components/schemas/SmContextUpdateError'

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

multipart/related:  # message with binary body part(s)

schema:

type: object

properties: # Request parts

jsonData:

$ref: '#/components/schemas/SmContextUpdateError'

binaryDataN1SmMessage:

type: string

format: binary

binaryDataN2SmInformation:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataN1SmMessage:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

binaryDataN2SmInformation:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/sm-contexts/{smContextRef}/release:

post:

summary:  Release SM Context

tags:

- Individual SM context

operationId: ReleaseSmContext

parameters:

- name: smContextRef

in: path

description:  SM context reference

required: true

schema:

type: string

requestBody:

description: representation of the data to be sent to the SMF when releasing the SM context

required: false

content:

application/json: # message without binary body part

schema:

$ref: '#/components/schemas/SmContextReleaseData'

multipart/related:  # message with binary body part(s)

schema:

type: object

properties: # Request parts

jsonData:

$ref: '#/components/schemas/SmContextReleaseData'

binaryDataN2SmInformation:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataN2SmInformation:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

responses:

'200':

description: successful release of a PDU session with content in the response

content:

application/json: # message without binary body part

schema:

$ref: '#/components/schemas/SmContextReleasedData'

'204':

description: successful release of an SM context without content in the response

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/sm-contexts/{smContextRef}/send-mo-data:

post:

summary:  Send MO Data

tags:

- Individual SM context

operationId: SendMoData

parameters:

- name: smContextRef

in: path

description:  SM context reference

required: true

schema:

type: string

requestBody:

description: representation of the payload of Send MO Data Request

required: true

content:

multipart/related:  # message with a binary body part

schema:

type: object

properties:

jsonData:

$ref: '#/components/schemas/SendMoDataReqData'

binaryMoData:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryMoData:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

responses:

'204':

description: successful sending of MO data

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: '#/components/responses/400'

'401':

$ref: '#/components/responses/401'

'403':

$ref: '#/components/responses/403'

'404':

$ref: '#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: '#/components/responses/413'

'415':

$ref: '#/components/responses/415'

'429':

$ref: '#/components/responses/429'

'500':

$ref: '#/components/responses/500'

'503':

$ref: '#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/pdu-sessions:

post:

summary:  Create

tags:

- PDU sessions collection

operationId: PostPduSessions

requestBody:

description: representation of the PDU session to be created in the H-SMF or SMF

required: true

content:

application/json: # message without binary body part

schema:

$ref: '#/components/schemas/PduSessionCreateData'

multipart/related:  # message with binary body part(s)

schema:

type: object

properties: # Request parts

jsonData:

$ref: '#/components/schemas/PduSessionCreateData'

binaryDataN1SmInfoFromUe:

type: string

format: binary

binaryDataUnknownN1SmInfo:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataN1SmInfoFromUe:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

binaryDataUnknownN1SmInfo:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

callbacks:

statusNotification:

'{$request.body#/vsmfPduSessionUri}':

post:

summary:  Notify Status

tags:

- Individual PDU session (V-SMF)

operationId: NotifyStatus

requestBody:

$ref: '#/components/requestBodies/NotifyStatusRequestBody'

responses:

'204':

description: successful notificationof the status change

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

statusNotification-ismf:

'{$request.body#/ismfPduSessionUri}':

post:

summary:  Notify Status

tags:

- Individual PDU session (I-SMF)

operationId: NotifyStatus-isfm

requestBody:

$ref: '#/components/requestBodies/NotifyStatusRequestBody'

responses:

'204':

description: successful notificationof the status change

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

update:

'{$request.body#/vsmfPduSessionUri}/modify':

post:

summary:  Update (initiated by H-SMF)

tags:

- Individual PDU session (V-SMF)

operationId: ModifyPduSession

requestBody:

$ref: '#/components/requestBodies/VsmfUpdateRequestBody'

responses:

'200':

$ref: '#/components/responses/VsmfUpdateResponse200'

'204':

description: successful update of a PDU session without content in the response

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: '#/components/responses/VsmfUpdateError'

'403':

$ref: '#/components/responses/VsmfUpdateError'

'404':

$ref: '#/components/responses/VsmfUpdateError'

'409':

$ref: '#/components/responses/VsmfUpdateError'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: '#/components/responses/VsmfUpdateError'

'503':

$ref: '#/components/responses/VsmfUpdateError'

'504':

$ref: '#/components/responses/VsmfUpdateError'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

update-ismf:

'{$request.body#/ismfPduSessionUri}/modify':

post:

summary:  Update (initiated by SMF)

tags:

- Individual PDU session (I-SMF)

operationId: ModifyPduSession-ismf

requestBody:

$ref: '#/components/requestBodies/VsmfUpdateRequestBody'

responses:

'200':

$ref: '#/components/responses/VsmfUpdateResponse200'

'204':

description: successful update of a PDU session without content in the response

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: '#/components/responses/VsmfUpdateError'

'403':

$ref: '#/components/responses/VsmfUpdateError'

'404':

$ref: '#/components/responses/VsmfUpdateError'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: '#/components/responses/VsmfUpdateError'

'503':

$ref: '#/components/responses/VsmfUpdateError'

'504':

$ref: '#/components/responses/VsmfUpdateError'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

transferMtData:

'{$request.body#/vsmfPduSessionUri}/transfer-mt-data':

post:

summary:  Transfer MT Data (by H-SMF)

tags:

- Individual PDU session (V-SMF)

operationId: TransferMtData

requestBody:

description: representation of the payload of Transfer MT Data Request

required: true

content:

multipart/related:  # message with a binary body part

schema:

type: object

properties:

jsonData:

$ref: '#/components/schemas/TransferMtDataReqData'

binaryMtData:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryMtData:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

responses:

'204':

description: successful transfering of MT data

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

description: unsuccessful delivery of mobile terminated data - gateway timeout

content:

application/json:

schema:

$ref: '#/components/schemas/TransferMtDataError'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

transferMtData-ismf:

'{$request.body#/ismfPduSessionUri}/transfer-mt-data':

post:

summary:  Transfer MT Data (by SMF)

tags:

- Individual PDU session (I-SMF)

operationId: TransferMtData-ismf

requestBody:

description: representation of the payload of Transfer MT Data Request

required: true

content:

multipart/related:  # message with a binary body part

schema:

type: object

properties:

jsonData:

$ref: '#/components/schemas/TransferMtDataReqData'

binaryMtData:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryMtData:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

responses:

'204':

description: successful transfering of MT data

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

description: unsuccessful delivery of mobile terminated data - gateway timeout

content:

application/json:

schema:

$ref: '#/components/schemas/TransferMtDataError'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

responses:

'201':

description: successful creation of a PDU session

content:

application/json: # message without binary body part

schema:

$ref: '#/components/schemas/PduSessionCreatedData'

multipart/related:  # message with binary body part(s)

schema:

type: object

properties: # Request parts

jsonData:

$ref: '#/components/schemas/PduSessionCreatedData'

binaryDataN1SmInfoToUe:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataN1SmInfoToUe:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/nsmf-pdusession/<apiVersion>/pdu-sessions/{pduSessionRef}'

required: true

schema:

type: string

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: '#/components/responses/PduSessionCreateError'

'403':

$ref: '#/components/responses/PduSessionCreateError'

'404':

$ref: '#/components/responses/PduSessionCreateError'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: '#/components/responses/PduSessionCreateError'

'503':

$ref: '#/components/responses/PduSessionCreateError'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/pdu-sessions/{pduSessionRef}/modify:

post:

summary:  Update (initiated by V-SMF or I-SMF)

tags:

- Individual PDU session (H-SMF or SMF)

operationId: UpdatePduSession

parameters:

- name: pduSessionRef

in: path

description:  PDU session reference

required: true

schema:

type: string

requestBody:

description: representation of the updates to apply to the PDU session

required: true

content:

application/json: # message without binary body part

schema:

$ref: '#/components/schemas/HsmfUpdateData'

multipart/related:  # message with binary body part(s)

schema:

type: object

properties: # Request parts

jsonData:

$ref: '#/components/schemas/HsmfUpdateData'

binaryDataN1SmInfoFromUe:

type: string

format: binary

binaryDataUnknownN1SmInfo:

type: string

format: binary

binaryDataN4Information:

type: string

format: binary

binaryDataN4InformationExt1:

type: string

format: binary

binaryDataN4InformationExt2:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataN1SmInfoFromUe:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

binaryDataUnknownN1SmInfo:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

binaryDataN4Information:

contentType:  application/vnd.3gpp.pfcp

headers:

Content-Id:

schema:

type: string

binaryDataN4InformationExt1:

contentType:  application/vnd.3gpp.pfcp

headers:

Content-Id:

schema:

type: string

binaryDataN4InformationExt2:

contentType:  application/vnd.3gpp.pfcp

headers:

Content-Id:

schema:

type: string

responses:

'200':

description: successful update of a PDU session with content in the response

content:

application/json: # message without binary body part

schema:

$ref: '#/components/schemas/HsmfUpdatedData'

multipart/related:  # message with binary body part(s)

schema:

type: object

properties: # Request parts

jsonData:

$ref: '#/components/schemas/HsmfUpdatedData'

binaryDataN1SmInfoToUe:

type: string

format: binary

binaryDataN4Information:

type: string

format: binary

binaryDataN4InformationExt1:

type: string

format: binary

binaryDataN4InformationExt2:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataN1SmInfoToUe:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

binaryDataN4Information:

contentType:  application/vnd.3gpp.pfcp

headers:

Content-Id:

schema:

type: string

binaryDataN4InformationExt1:

contentType:  application/vnd.3gpp.pfcp

headers:

Content-Id:

schema:

type: string

binaryDataN4InformationExt2:

contentType:  application/vnd.3gpp.pfcp

headers:

Content-Id:

schema:

type: string

'204':

description: successful update of a PDU session without content in the response

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: '#/components/responses/HsmfUpdateError'

'403':

$ref: '#/components/responses/HsmfUpdateError'

'404':

$ref: '#/components/responses/HsmfUpdateError'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: '#/components/responses/HsmfUpdateError'

'503':

$ref: '#/components/responses/HsmfUpdateError'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/pdu-sessions/{pduSessionRef}/release:

post:

summary:  Release

tags:

- Individual PDU session (H-SMF or SMF)

operationId: ReleasePduSession

parameters:

- name: pduSessionRef

in: path

description:  PDU session reference

required: true

schema:

type: string

requestBody:

description: data sent to H-SMF or SMF when releasing the PDU session

required: false

content:

application/json: # message without binary body part

schema:

$ref: '#/components/schemas/ReleaseData'

multipart/related:  # message with binary body part(s)

schema:

type: object

properties:

jsonData:

$ref: '#/components/schemas/ReleaseData'

binaryDataN4Information:

type: string

format: binary

binaryDataN4InformationExt1:

type: string

format: binary

binaryDataN4InformationExt2:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataN4Information:

contentType:  application/vnd.3gpp.pfcp

headers:

Content-Id:

schema:

type: string

binaryDataN4InformationExt1:

contentType:  application/vnd.3gpp.pfcp

headers:

Content-Id:

schema:

type: string

binaryDataN4InformationExt2:

contentType:  application/vnd.3gpp.pfcp

headers:

Content-Id:

schema:

type: string

responses:

'200':

description: successful release of a PDU session with content in the response

content:

application/json: # message without binary body part

schema:

$ref: '#/components/schemas/ReleasedData'

multipart/related:  # message with binary body part(s)

schema:

type: object

properties:

jsonData:

$ref: '#/components/schemas/ReleasedData'

binaryDataN4Information:

type: string

format: binary

binaryDataN4InformationExt1:

type: string

format: binary

binaryDataN4InformationExt2:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataN4Information:

contentType:  application/vnd.3gpp.pfcp

headers:

Content-Id:

schema:

type: string

binaryDataN4InformationExt1:

contentType:  application/vnd.3gpp.pfcp

headers:

Content-Id:

schema:

type: string

binaryDataN4InformationExt2:

contentType:  application/vnd.3gpp.pfcp

headers:

Content-Id:

schema:

type: string

'204':

description: successful release of a PDU session

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/pdu-sessions/{pduSessionRef}/retrieve:

post:

summary:  Retrieve

tags:

- Individual PDU session (H-SMF or SMF)

operationId: RetrievePduSession

parameters:

- name: pduSessionRef

in: path

description:  PDU session reference

required: true

schema:

type: string

requestBody:

description: representation of the payload of the Retrieve Request

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/RetrieveData'

responses:

'200':

description: successful information retrieval

content:

application/json: # message without binary body part

schema:

$ref: '#/components/schemas/RetrievedData'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/pdu-sessions/{pduSessionRef}/transfer-mo-data:

post:

summary:  Transfer MO Data

tags:

- Individual PDU session (H-SMF or SMF)

operationId: TransferMoData

parameters:

- name: pduSessionRef

in: path

description:  PDU session reference

required: true

schema:

type: string

requestBody:

description: representation of the payload of Transfer MO Data Request

required: true

content:

multipart/related:  # message with a binary body part

schema:

type: object

properties:

jsonData:

$ref: '#/components/schemas/TransferMoDataReqData'

binaryMoData:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryMoData:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

responses:

'204':

description: successful transfering of MO data

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nsmf-pdusession: Access to the nsmf-pdusession API

schemas:

#

# STRUCTURED DATA TYPES

#

SmContextCreateData:

description: Data within Create SM Context Request

type: object

properties:

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

unauthenticatedSupi:

type: boolean

default:  false

pei:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pei'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

pduSessionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionId'

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

selectedDnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

sNssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

hplmnSnssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

servingNfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

guami:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Guami'

serviceName:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/ServiceName'

servingNetwork:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

requestType:

$ref: '#/components/schemas/RequestType'

n1SmMsg:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

anType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

additionalAnType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

ratType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

presenceInLadn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceState'

ueLocation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

ueTimeZone:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TimeZone'

addUeLocation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

smContextStatusUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

hSmfUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

hSmfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

smfUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

smfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

additionalHsmfUri:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

minItems: 1

additionalHsmfId:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

minItems: 1

additionalSmfUri:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

minItems: 1

additionalSmfId:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

minItems: 1

oldPduSessionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionId'

pduSessionsActivateList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionId'

minItems: 1

ueEpsPdnConnection:

$ref: '#/components/schemas/EpsPdnCnxContainer'

hoState:

$ref: '#/components/schemas/HoState'

pcfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

pcfGroupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfGroupId'

pcfSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSetId'

nrfUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

selMode:

$ref: '#/components/schemas/DnnSelectionMode'

backupAmfInfo:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BackupAmfInfo'

minItems: 1

traceData:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TraceData'

udmGroupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfGroupId'

routingIndicator:

type: string

hNwPubKeyId:

type: integer

epsInterworkingInd:

$ref: '#/components/schemas/EpsInterworkingIndication'

indirectForwardingFlag:

type: boolean

directForwardingFlag:

type: boolean

targetId:

$ref: 'TS29518_Namf_Communication.yaml#/components/schemas/NgRanTargetId'

epsBearerCtxStatus:

$ref: '#/components/schemas/EpsBearerContextStatus'

cpCiotEnabled:

type: boolean

default: false

cpOnlyInd:

type: boolean

default: false

invokeNef:

type: boolean

default: false

maRequestInd:

type: boolean

default: false

maNwUpgradeInd:

type: boolean

default: false

n2SmInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

n2SmInfoType:

$ref: '#/components/schemas/N2SmInfoType'

n2SmInfoExt1:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

n2SmInfoTypeExt1:

$ref: '#/components/schemas/N2SmInfoType'

smContextRef:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

smContextSmfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

smContextSmfSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSetId'

smContextSmfServiceSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfServiceSetId'

smContextSmfBinding:

$ref: 'TS29518_Namf_Communication.yaml#/components/schemas/SbiBindingLevel'

upCnxState:

$ref: '#/components/schemas/UpCnxState'

smallDataRateStatus:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SmallDataRateStatus'

apnRateStatus:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApnRateStatus'

extendedNasSmTimerInd:

type: boolean

default: false

dlDataWaitingInd:

type: boolean

default: false

ddnFailureSubs:

$ref: '#/components/schemas/DdnFailureSubs'

smfTransferInd:

type: boolean

default: false

oldSmfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

oldSmContextRef:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

wAgfInfo:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/WAgfInfo'

tngfInfo:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/TngfInfo'

twifInfo:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/TwifInfo'

ranUnchangedInd:

type: boolean

samePcfSelectionInd:

type: boolean

default: false

targetDnai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

nrfManagementUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

nrfDiscoveryUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

nrfAccessTokenUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

ivSmfRestoration:

type: boolean

default:  false

smfBindingInfo:

type: string

required:

- servingNfId

- servingNetwork

- anType

- smContextStatusUri

SmContextCreatedData:

description: Data within Create SM Context Response

type: object

properties:

hSmfUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

smfUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

pduSessionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionId'

sNssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

upCnxState:

$ref: '#/components/schemas/UpCnxState'

n2SmInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

n2SmInfoType:

$ref: '#/components/schemas/N2SmInfoType'

allocatedEbiList:

type: array

items:

$ref: '#/components/schemas/EbiArpMapping'

minItems: 1

hoState:

$ref: '#/components/schemas/HoState'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

smfServiceInstanceId:

type: string

recoveryTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

selectedSmfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

selectedOldSmfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

SmContextUpdateData:

description: Data within Update SM Context Request

type: object

properties:

pei:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pei'

servingNfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

guami:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Guami'

servingNetwork:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

backupAmfInfo:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BackupAmfInfo'

minItems: 1

nullable: true

anType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

additionalAnType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

anTypeToReactivate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

ratType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

presenceInLadn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceState'

ueLocation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

ueTimeZone:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TimeZone'

addUeLocation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

upCnxState:

$ref: '#/components/schemas/UpCnxState'

hoState:

$ref: '#/components/schemas/HoState'

toBeSwitched:

type: boolean

default: false

failedToBeSwitched:

type: boolean

n1SmMsg:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

n2SmInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

n2SmInfoType:

$ref: '#/components/schemas/N2SmInfoType'

targetId:

$ref: 'TS29518_Namf_Communication.yaml#/components/schemas/NgRanTargetId'

targetServingNfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

smContextStatusUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

dataForwarding:

type: boolean

default: false

n9ForwardingTunnel:

$ref: '#/components/schemas/TunnelInfo'

n9DlForwardingTnlList:

type: array

items:

$ref: '#/components/schemas/IndirectDataForwardingTunnelInfo'

minItems: 1

n9UlForwardingTnlList:

type: array

items:

$ref: '#/components/schemas/IndirectDataForwardingTunnelInfo'

minItems: 1

epsBearerSetup:

type: array

items:

$ref: '#/components/schemas/EpsBearerContainer'

minItems: 0

revokeEbiList:

type: array

items:

$ref: '#/components/schemas/EpsBearerId'

minItems: 1

release:

type: boolean

default: false

cause:

$ref: '#/components/schemas/Cause'

ngApCause:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NgApCause'

5gMmCauseValue:

$ref: 'TS29571_CommonData.yaml#/components/schemas/5GMmCause'

sNssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

traceData:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TraceData'

epsInterworkingInd:

$ref: '#/components/schemas/EpsInterworkingIndication'

anTypeCanBeChanged:

type: boolean

default: false

n2SmInfoExt1:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

n2SmInfoTypeExt1:

$ref: '#/components/schemas/N2SmInfoType'

maReleaseInd:

$ref: '#/components/schemas/MaReleaseIndication'

maNwUpgradeInd:

type: boolean

default: false

maRequestInd:

type: boolean

default: false

exemptionInd:

$ref: '#/components/schemas/ExemptionInd'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

moExpDataCounter:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MoExpDataCounter'

extendedNasSmTimerInd:

type: boolean

forwardingFTeid:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

forwardingBearerContexts:

type: array

items:

$ref: '#/components/schemas/ForwardingBearerContainer'

minItems: 1

ddnFailureSubs:

$ref: '#/components/schemas/DdnFailureSubs'

skipN2PduSessionResRelInd:

type: boolean

default: false

secondaryRatUsageDataReportContainer:

type: array

items:

$ref: '#/components/schemas/SecondaryRatUsageDataReportContainer'

minItems: 1

SmContextUpdatedData:

description: Data within Update SM Context Response

type: object

properties:

upCnxState:

$ref: '#/components/schemas/UpCnxState'

hoState:

$ref: '#/components/schemas/HoState'

releaseEbiList:

type: array

items:

$ref: '#/components/schemas/EpsBearerId'

minItems: 1

allocatedEbiList:

type: array

items:

$ref: '#/components/schemas/EbiArpMapping'

minItems: 1

modifiedEbiList:

type: array

items:

$ref: '#/components/schemas/EbiArpMapping'

minItems: 1

n1SmMsg:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

n2SmInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

n2SmInfoType:

$ref: '#/components/schemas/N2SmInfoType'

epsBearerSetup:

type: array

items:

$ref: '#/components/schemas/EpsBearerContainer'

minItems: 1

dataForwarding:

type: boolean

n3DlForwardingTnlList:

type: array

items:

$ref: '#/components/schemas/IndirectDataForwardingTunnelInfo'

minItems: 1

n3UlForwardingTnlList:

type: array

items:

$ref: '#/components/schemas/IndirectDataForwardingTunnelInfo'

minItems: 1

cause:

$ref: '#/components/schemas/Cause'

maAcceptedInd:

type: boolean

default: false

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

forwardingFTeid:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

forwardingBearerContexts:

type: array

items:

$ref: '#/components/schemas/ForwardingBearerContainer'

minItems: 1

selectedSmfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

selectedOldSmfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

SmContextReleaseData:

description: Data within Release SM Context Request

type: object

properties:

cause:

$ref: '#/components/schemas/Cause'

ngApCause:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NgApCause'

5gMmCauseValue:

$ref: 'TS29571_CommonData.yaml#/components/schemas/5GMmCause'

ueLocation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

ueTimeZone:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TimeZone'

addUeLocation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

vsmfReleaseOnly:

type: boolean

default: false

n2SmInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

n2SmInfoType:

$ref: '#/components/schemas/N2SmInfoType'

ismfReleaseOnly:

type: boolean

default: false

SmContextReleasedData:

description: Data within Release SM Context Response

type: object

properties:

smallDataRateStatus:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SmallDataRateStatus'

apnRateStatus:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApnRateStatus'

SmContextStatusNotification:

description: Data within Notify SM Context Status Request

type: object

properties:

statusInfo :

$ref: '#/components/schemas/StatusInfo'

smallDataRateStatus:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SmallDataRateStatus'

apnRateStatus:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApnRateStatus'

ddnFailureStatus:

type: boolean

default: false

notifyCorrelationIdsForddnFailure:

type: array

items:

type: string

minItems: 1

newSmfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

newSmfSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSetId'

oldSmfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

oldSmContextRef:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

altAnchorSmfUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

altAnchorSmfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

targetDnaiInfo:

$ref: '#/components/schemas/TargetDnaiInfo'

required:

- statusInfo

PduSessionCreateData:

description: Data within Create Request

type: object

properties:

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

unauthenticatedSupi:

type: boolean

default: false

pei:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pei'

pduSessionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionId'

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

selectedDnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

sNssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

vsmfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

ismfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

servingNetwork:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

requestType:

$ref: '#/components/schemas/RequestType'

epsBearerId:

type: array

items:

$ref: '#/components/schemas/EpsBearerId'

minItems: 1

pgwS8cFteid:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

vsmfPduSessionUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

ismfPduSessionUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

vcnTunnelInfo:

$ref: '#/components/schemas/TunnelInfo'

icnTunnelInfo:

$ref: '#/components/schemas/TunnelInfo'

n9ForwardingTunnelInfo:

$ref: '#/components/schemas/TunnelInfo'

additionalCnTunnelInfo:

$ref: '#/components/schemas/TunnelInfo'

anType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

additionalAnType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

ratType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

ueLocation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

ueTimeZone:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TimeZone'

addUeLocation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

n1SmInfoFromUe:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

unknownN1SmInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

hPcfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

pcfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

pcfGroupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfGroupId'

pcfSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSetId'

hoPreparationIndication:

type: boolean

selMode:

$ref: '#/components/schemas/DnnSelectionMode'

alwaysOnRequested:

type: boolean

default: false

udmGroupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfGroupId'

routingIndicator:

type: string

hNwPubKeyId:

type: integer

epsInterworkingInd:

$ref: '#/components/schemas/EpsInterworkingIndication'

vSmfServiceInstanceId:

type: string

iSmfServiceInstanceId:

type: string

recoveryTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

roamingChargingProfile:

$ref: 'TS32291_Nchf_ConvergedCharging.yaml#/components/schemas/RoamingChargingProfile'

chargingId:

type: string

oldPduSessionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionId'

epsBearerCtxStatus:

$ref: '#/components/schemas/EpsBearerContextStatus'

amfNfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

guami:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Guami'

maxIntegrityProtectedDataRateUl:

$ref: '#/components/schemas/MaxIntegrityProtectedDataRate'

maxIntegrityProtectedDataRateDl:

$ref: '#/components/schemas/MaxIntegrityProtectedDataRate'

cpCiotEnabled:

type: boolean

default: false

cpOnlyInd:

type: boolean

default: false

invokeNef:

type: boolean

default: false

maRequestInd:

type: boolean

default: false

maNwUpgradeInd:

type: boolean

default: false

dnaiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

minItems: 1

presenceInLadn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceState'

secondaryRatUsageInfo:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SecondaryRatUsageInfo'

minItems: 1

smallDataRateStatus:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SmallDataRateStatus'

apnRateStatus:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApnRateStatus'

dlServingPlmnRateCtl:

type: integer

minimum: 10

upSecurityInfo:

$ref: '#/components/schemas/UpSecurityInfo'

vplmnQos:

$ref: '#/components/schemas/VplmnQos'

ivSmfRestoration:

type: boolean

default:  false

oldSmContextRef:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

redundantPduSessionInfo:

$ref: '#/components/schemas/RedundantPduSessionInformation'

required:

- dnn

- servingNetwork

- anType

oneOf:

- required: [ vsmfId, vsmfPduSessionUri ]

- required: [ ismfId, ismfPduSessionUri ]

PduSessionCreatedData:

description: Data within Create Response

type: object

properties:

pduSessionType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionType'

sscMode:

type: string

pattern: '^[0-7]$'

hcnTunnelInfo:

$ref: '#/components/schemas/TunnelInfo'

cnTunnelInfo:

$ref: '#/components/schemas/TunnelInfo'

additionalCnTunnelInfo:

$ref: '#/components/schemas/TunnelInfo'

sessionAmbr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ambr'

qosFlowsSetupList:

type: array

items:

$ref: '#/components/schemas/QosFlowSetupItem'

minItems: 1

hSmfInstanceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

smfInstanceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

pduSessionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionId'

sNssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

enablePauseCharging:

type: boolean

default: false

ueIpv4Address:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

ueIpv6Prefix:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

n1SmInfoToUe:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

epsPdnCnxInfo:

$ref: '#/components/schemas/EpsPdnCnxInfo'

epsBearerInfo:

type: array

items:

$ref: '#/components/schemas/EpsBearerInfo'

minItems: 1

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

maxIntegrityProtectedDataRate:

$ref: '#/components/schemas/MaxIntegrityProtectedDataRate'

maxIntegrityProtectedDataRateDl:

$ref: '#/components/schemas/MaxIntegrityProtectedDataRate'

alwaysOnGranted:

type: boolean

default: false

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

upSecurity:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UpSecurity'

roamingChargingProfile:

$ref: 'TS32291_Nchf_ConvergedCharging.yaml#/components/schemas/RoamingChargingProfile'

hSmfServiceInstanceId:

type: string

smfServiceInstanceId:

type: string

recoveryTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

dnaiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

minItems: 1

ipv6MultiHomingInd:

type: boolean

default: false

maAcceptedInd:

type: boolean

default: false

homeProvidedChargingId:

type: string

nefExtBufSupportInd:

type: boolean

default: false

smallDataRateControlEnabled:

type: boolean

default: false

ueIpv6InterfaceId:

type: string

pattern: '^[A-Fa-f0-9]{16}$'

ipv6Index:

$ref: 'TS29519_Policy_Data.yaml#/components/schemas/IpIndex'

dnAaaAddress:

$ref: '#/components/schemas/IpAddress'

redundantPduSessionInfo:

$ref: '#/components/schemas/RedundantPduSessionInformation'

nspuSupportInd:

type: boolean

required:

- pduSessionType

- sscMode

oneOf:

- required: [ hSmfInstanceId ]

- required: [ smfInstanceId ]

HsmfUpdateData:

description: Data within Update Request towards H-SMF, or from I-SMF to SMF

type: object

properties:

requestIndication:

$ref: '#/components/schemas/RequestIndication'

pei:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pei'

vcnTunnelInfo:

$ref: '#/components/schemas/TunnelInfo'

icnTunnelInfo:

$ref: '#/components/schemas/TunnelInfo'

additionalCnTunnelInfo:

$ref: '#/components/schemas/TunnelInfo'

servingNetwork:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

anType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

additionalAnType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

ratType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

ueLocation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

ueTimeZone:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TimeZone'

addUeLocation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

pauseCharging:

type: boolean

pti:

$ref: '#/components/schemas/ProcedureTransactionId'

n1SmInfoFromUe:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

unknownN1SmInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

qosFlowsRelNotifyList:

type: array

items:

$ref: '#/components/schemas/QosFlowItem'

minItems: 1

qosFlowsNotifyList:

type: array

items:

$ref: '#/components/schemas/QosFlowNotifyItem'

minItems: 1

NotifyList:

type: array

items:

$ref: '#/components/schemas/PduSessionNotifyItem'

minItems: 1

epsBearerId:

type: array

items:

$ref: '#/components/schemas/EpsBearerId'

minItems: 0

hoPreparationIndication:

type: boolean

revokeEbiList:

type: array

items:

$ref: '#/components/schemas/EpsBearerId'

minItems: 1

cause:

$ref: '#/components/schemas/Cause'

ngApCause:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NgApCause'

5gMmCauseValue:

$ref: 'TS29571_CommonData.yaml#/components/schemas/5GMmCause'

alwaysOnRequested:

type: boolean

default: false

epsInterworkingInd:

$ref: '#/components/schemas/EpsInterworkingIndication'

secondaryRatUsageReport:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SecondaryRatUsageReport'

minItems: 1

secondaryRatUsageInfo:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SecondaryRatUsageInfo'

minItems: 1

anTypeCanBeChanged:

type: boolean

default: false

maReleaseInd:

$ref: '#/components/schemas/MaReleaseIndication'

maNwUpgradeInd:

type: boolean

default: false

maRequestInd:

type: boolean

default: false

unavailableAccessInd:

$ref: '#/components/schemas/UnavailableAccessIndication'

psaInfo:

type: array

items:

$ref: '#/components/schemas/PsaInformation'

minItems: 1

ulclBpInfo:

$ref: '#/components/schemas/UlclBpInformation'

n4Info:

$ref: '#/components/schemas/N4Information'

n4InfoExt1:

$ref: '#/components/schemas/N4Information'

n4InfoExt2:

$ref: '#/components/schemas/N4Information'

presenceInLadn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceState'

vsmfPduSessionUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

vsmfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

vSmfServiceInstanceId:

type: string

ismfPduSessionUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

ismfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

iSmfServiceInstanceId:

type: string

dlServingPlmnRateCtl:

type: integer

minimum: 10

nullable: true

dnaiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

minItems: 1

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

roamingChargingProfile:

$ref: 'TS32291_Nchf_ConvergedCharging.yaml#/components/schemas/RoamingChargingProfile'

moExpDataCounter:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MoExpDataCounter'

vplmnQos:

$ref: '#/components/schemas/VplmnQos'

securityResult:

$ref: '#/components/schemas/SecurityResult'

upSecurityInfo:

$ref: '#/components/schemas/UpSecurityInfo'

amfNfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

guami:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Guami'

secondaryRatUsageDataReportContainer:

type: array

items:

$ref: '#/components/schemas/SecondaryRatUsageDataReportContainer'

minItems: 1

required:

- requestIndication

HsmfUpdatedData:

description: Data within Update Response from H-SMF, or from SMF to I-SMF

type: object

properties:

n1SmInfoToUe:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

n4Info:

$ref: '#/components/schemas/N4Information'

n4InfoExt1:

$ref: '#/components/schemas/N4Information'

n4InfoExt2:

$ref: '#/components/schemas/N4Information'

dnaiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

minItems: 1

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

roamingChargingProfile:

$ref: 'TS32291_Nchf_ConvergedCharging.yaml#/components/schemas/RoamingChargingProfile'

upSecurity:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UpSecurity'

maxIntegrityProtectedDataRateUl:

$ref: '#/components/schemas/MaxIntegrityProtectedDataRate'

maxIntegrityProtectedDataRateDl:

$ref: '#/components/schemas/MaxIntegrityProtectedDataRate'

ipv6MultiHomingInd:

type: boolean

default: false

qosFlowsSetupList:

type: array

items:

$ref: '#/components/schemas/QosFlowSetupItem'

minItems: 1

sessionAmbr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ambr'

epsPdnCnxInfo:

$ref: '#/components/schemas/EpsPdnCnxInfo'

epsBearerInfo:

type: array

items:

$ref: '#/components/schemas/EpsBearerInfo'

minItems: 1

pti:

$ref: '#/components/schemas/ProcedureTransactionId'

ReleaseData:

description: Data within Release Request

type: object

properties:

cause:

$ref: '#/components/schemas/Cause'

ngApCause:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NgApCause'

5gMmCauseValue:

$ref: 'TS29571_CommonData.yaml#/components/schemas/5GMmCause'

ueLocation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

ueTimeZone:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TimeZone'

addUeLocation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

secondaryRatUsageReport:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SecondaryRatUsageReport'

minItems: 1

secondaryRatUsageInfo:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SecondaryRatUsageInfo'

minItems: 1

n4Info:

$ref: '#/components/schemas/N4Information'

n4InfoExt1:

$ref: '#/components/schemas/N4Information'

n4InfoExt2:

$ref: '#/components/schemas/N4Information'

ReleasedData:

description: Data within Release Response

type: object

properties:

smallDataRateStatus:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SmallDataRateStatus'

apnRateStatus:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApnRateStatus'

n4Info:

$ref: '#/components/schemas/N4Information'

n4InfoExt1:

$ref: '#/components/schemas/N4Information'

n4InfoExt2:

$ref: '#/components/schemas/N4Information'

VsmfUpdateData:

description: Data within Update Request towards V-SMF, or from SMF to I-SMF

type: object

properties:

requestIndication:

$ref: '#/components/schemas/RequestIndication'

sessionAmbr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ambr'

qosFlowsAddModRequestList:

type: array

items:

$ref: '#/components/schemas/QosFlowAddModifyRequestItem'

minItems: 1

qosFlowsRelRequestList:

type: array

items:

$ref: '#/components/schemas/QosFlowReleaseRequestItem'

minItems: 1

epsBearerInfo:

type: array

items:

$ref: '#/components/schemas/EpsBearerInfo'

minItems: 1

assignEbiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Arp'

minItems: 1

revokeEbiList:

type: array

items:

$ref: '#/components/schemas/EpsBearerId'

minItems: 1

modifiedEbiList:

type: array

items:

$ref: '#/components/schemas/EbiArpMapping'

minItems: 1

pti:

$ref: '#/components/schemas/ProcedureTransactionId'

n1SmInfoToUe:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

alwaysOnGranted:

type: boolean

default: false

hsmfPduSessionUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

cause:

$ref: '#/components/schemas/Cause'

n1smCause:

type: string

backOffTimer:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

maReleaseInd:

$ref: '#/components/schemas/MaReleaseIndication'

maAcceptedInd:

type: boolean

default: false

additionalCnTunnelInfo:

$ref: '#/components/schemas/TunnelInfo'

dnaiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

n4Info:

$ref: '#/components/schemas/N4Information'

n4InfoExt1:

$ref: '#/components/schemas/N4Information'

n4InfoExt2:

$ref: '#/components/schemas/N4Information'

n4InfoExt3:

$ref: '#/components/schemas/N4Information'

smallDataRateControlEnabled:

type: boolean

qosMonitoringInfo:

$ref: '#/components/schemas/QosMonitoringInfo'

epsPdnCnxInfo:

$ref: '#/components/schemas/EpsPdnCnxInfo'

required:

- requestIndication

VsmfUpdatedData:

description: Data within Update Response from V-SMF, or from I-SMF to SMF

type: object

properties:

qosFlowsAddModList:

type: array

items:

$ref: '#/components/schemas/QosFlowItem'

minItems: 1

qosFlowsRelList:

type: array

items:

$ref: '#/components/schemas/QosFlowItem'

minItems: 1

qosFlowsFailedtoAddModList:

type: array

items:

$ref: '#/components/schemas/QosFlowItem'

minItems: 1

qosFlowsFailedtoRelList:

type: array

items:

$ref: '#/components/schemas/QosFlowItem'

minItems: 1

n1SmInfoFromUe:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

unknownN1SmInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

ueLocation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

ueTimeZone:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TimeZone'

addUeLocation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

assignedEbiList:

type: array

items:

$ref: '#/components/schemas/EbiArpMapping'

minItems: 1

failedToAssignEbiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Arp'

minItems: 1

releasedEbiList:

type: array

items:

$ref: '#/components/schemas/EpsBearerId'

minItems: 1

secondaryRatUsageReport:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SecondaryRatUsageReport'

minItems: 1

secondaryRatUsageInfo:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SecondaryRatUsageInfo'

minItems: 1

n4Info:

$ref: '#/components/schemas/N4Information'

n4InfoExt1:

$ref: '#/components/schemas/N4Information'

n4InfoExt2:

$ref: '#/components/schemas/N4Information'

n4InfoExt3:

$ref: '#/components/schemas/N4Information'

StatusNotification:

description: Data within Notify Status Request

type: object

properties:

statusInfo :

$ref: '#/components/schemas/StatusInfo'

smallDataRateStatus:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SmallDataRateStatus'

apnRateStatus:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApnRateStatus'

targetDnaiInfo:

$ref: '#/components/schemas/TargetDnaiInfo'

required:

- statusInfo

QosFlowItem:

description: Individual QoS flow

type: object

properties:

qfi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Qfi'

cause:

$ref: '#/components/schemas/Cause'

currentQosProfileIndex:

type: integer

minimum: 1

maximum: 8

nullQoSProfileIndex:

type: boolean

required:

- qfi

QosFlowSetupItem:

description: Individual QoS flow to setup

type: object

properties:

qfi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Qfi'

qosRules:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

ebi:

$ref: '#/components/schemas/EpsBearerId'

qosFlowDescription:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

qosFlowProfile:

$ref: '#/components/schemas/QosFlowProfile'

associatedAnType:

$ref: '#/components/schemas/QosFlowAccessType'

required:

- qfi

- qosRules

QosFlowAddModifyRequestItem:

description: Individual QoS flow requested to be created or modified

type: object

properties:

qfi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Qfi'

ebi:

$ref: '#/components/schemas/EpsBearerId'

qosRules:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

qosFlowDescription:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

qosFlowProfile:

$ref: '#/components/schemas/QosFlowProfile'

associatedAnType:

$ref: '#/components/schemas/QosFlowAccessType'

required:

- qfi

QosFlowReleaseRequestItem:

description: Individual QoS flow requested to be released

type: object

properties:

qfi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Qfi'

qosRules:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

qosFlowDescription:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

required:

- qfi

QosFlowProfile:

description: QoS flow profile

type: object

properties:

5qi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/5Qi'

nonDynamic5Qi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NonDynamic5Qi'

dynamic5Qi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dynamic5Qi'

arp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Arp'

gbrQosFlowInfo:

$ref: '#/components/schemas/GbrQosFlowInformation'

rqa:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ReflectiveQoSAttribute'

additionalQosFlowInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AdditionalQosFlowInfo'

qosMonitoringReq:

$ref: '#/components/schemas/QosMonitoringReq'

qosRepPeriod:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

required:

- 5qi

GbrQosFlowInformation:

description: GBR QoS flow information

type: object

properties:

maxFbrDl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

maxFbrUl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

guaFbrDl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

guaFbrUl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

notifControl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NotificationControl'

maxPacketLossRateDl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketLossRate'

maxPacketLossRateUl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketLossRate'

alternativeQosProfileList:

type: array

items:

$ref: '#/components/schemas/AlternativeQosProfile'

required:

- maxFbrDl

- maxFbrUl

- guaFbrDl

- guaFbrUl

QosFlowNotifyItem:

description: Notification related to a QoS flow

type: object

properties:

qfi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Qfi'

notificationCause:

$ref: '#/components/schemas/NotificationCause'

currentQosProfileIndex:

type: integer

minimum: 1

maximum: 8

nullQoSProfileIndex:

type: boolean

required:

- qfi

- notificationCause

SmContextRetrieveData:

description: Data within Retrieve SM Context Request

type: object

properties:

targetMmeCap:

$ref: '#/components/schemas/MmeCapabilities'

smContextType:

$ref: '#/components/schemas/SmContextType'

servingNetwork:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

notToTransferEbiList:

type: array

items:

$ref: '#/components/schemas/EpsBearerId'

minItems: 1

ranUnchangedInd:

type: boolean

default: false

SmContextRetrievedData:

description: Data within Retrieve SM Context Response

type: object

properties:

ueEpsPdnConnection:

$ref: '#/components/schemas/EpsPdnCnxContainer'

smContext:

$ref: '#/components/schemas/SmContext'

smallDataRateStatus:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SmallDataRateStatus'

apnRateStatus:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApnRateStatus'

dlDataWaitingInd:

type: boolean

default: false

afCoordinationInfo:

$ref: '#/components/schemas/AfCoordinationInfo'

required:

- ueEpsPdnConnection

MmeCapabilities:

description: MME capabilities

type: object

properties:

nonIpSupported:

type: boolean

default: false

ethernetSupported:

type: boolean

default: false

TunnelInfo:

description: Tunnel Information

type: object

properties:

ipv4Addr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

ipv6Addr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Addr'

gtpTeid:

$ref: '#/components/schemas/Teid'

anType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

required:

- gtpTeid

StatusInfo:

description: Status of SM context or of PDU session

type: object

properties:

resourceStatus:

$ref: '#/components/schemas/ResourceStatus'

cause:

$ref: '#/components/schemas/Cause'

cnAssistedRanPara:

$ref: '#/components/schemas/CnAssistedRanPara'

anType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

required:

- resourceStatus

EpsPdnCnxInfo:

description: EPS PDN Connection Information from H-SMF to V-SMF, or from SMF to I-SMF

type: object

properties:

pgwS8cFteid:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

pgwNodeName:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

linkedBearerId:

$ref: '#/components/schemas/EpsBearerId'

required:

- pgwS8cFteid

EpsBearerInfo:

description: EPS Bearer Information from H-SMF to V-SMF, or from SMF to I-SMF

type: object

properties:

ebi:

$ref: '#/components/schemas/EpsBearerId'

pgwS8uFteid:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

bearerLevelQoS:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

required:

- ebi

- pgwS8uFteid

- bearerLevelQoS

PduSessionNotifyItem:

description: Notification related to a PDU session

type: object

properties:

notificationCause:

$ref: '#/components/schemas/NotificationCause'

required:

- notificationCause

EbiArpMapping:

description: EBI to ARP mapping

type: object

properties:

epsBearerId:

$ref: '#/components/schemas/EpsBearerId'

arp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Arp'

required:

- epsBearerId

- arp

SmContextCreateError:

description: Error within Create SM Context Response

type: object

properties:

error:

$ref: '#/components/schemas/ExtProblemDetails'

n1SmMsg:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

n2SmInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

n2SmInfoType:

$ref: '#/components/schemas/N2SmInfoType'

recoveryTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

required:

- error

SmContextUpdateError:

description: Error within Update SM Context Response

type: object

properties:

error:

$ref: '#/components/schemas/ExtProblemDetails'

n1SmMsg:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

n2SmInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

n2SmInfoType:

$ref: '#/components/schemas/N2SmInfoType'

upCnxState:

$ref: '#/components/schemas/UpCnxState'

recoveryTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

required:

- error

PduSessionCreateError:

description: Error within Create Response

type: object

properties:

error:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

n1smCause:

type: string

pattern: '^[A-F0-9]{2}$'

n1SmInfoToUe:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

backOffTimer:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

recoveryTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

required:

- error

HsmfUpdateError:

description: Error within Update Response from H-SMF, or from SMF to I-SMF

type: object

properties:

error:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

pti:

$ref: '#/components/schemas/ProcedureTransactionId'

n1smCause:

type: string

pattern: '^[A-F0-9]{2}$'

n1SmInfoToUe:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

backOffTimer:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

recoveryTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

required:

- error

VsmfUpdateError:

description: Error within Update Response from V-SMF, or from I-SMF to SMF

type: object

properties:

error:

$ref: '#/components/schemas/ExtProblemDetails'

pti:

$ref: '#/components/schemas/ProcedureTransactionId'

n1smCause:

type: string

pattern: '^[A-F0-9]{2}$'

n1SmInfoFromUe:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

unknownN1SmInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

failedToAssignEbiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Arp'

minItems: 1

ngApCause:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NgApCause'

5gMmCauseValue:

$ref: 'TS29571_CommonData.yaml#/components/schemas/5GMmCause'

recoveryTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

n4Info:

$ref: '#/components/schemas/N4Information'

n4InfoExt1:

$ref: '#/components/schemas/N4Information'

n4InfoExt2:

$ref: '#/components/schemas/N4Information'

n4InfoExt3:

$ref: '#/components/schemas/N4Information'

required:

- error

SmContext:

description: Complete SM Context

type: object

properties:

pduSessionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionId'

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

selectedDnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

sNssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

hplmnSnssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

pduSessionType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionType'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

hSmfUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

smfUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

pduSessionRef:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

pcfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

pcfGroupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfGroupId'

pcfSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSetId'

selMode:

$ref: '#/components/schemas/DnnSelectionMode'

udmGroupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfGroupId'

routingIndicator:

type: string

hNwPubKeyId:

type: integer

sessionAmbr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ambr'

qosFlowsList:

type: array

items:

$ref: '#/components/schemas/QosFlowSetupItem'

minItems: 1

hSmfInstanceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

smfInstanceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

pduSessionSmfSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSetId'

pduSessionSmfServiceSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfServiceSetId'

pduSessionSmfBinding:

$ref: 'TS29518_Namf_Communication.yaml#/components/schemas/SbiBindingLevel'

enablePauseCharging:

type: boolean

default: false

ueIpv4Address:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

ueIpv6Prefix:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

epsPdnCnxInfo:

$ref: '#/components/schemas/EpsPdnCnxInfo'

epsBearerInfo:

type: array

items:

$ref: '#/components/schemas/EpsBearerInfo'

minItems: 1

maxIntegrityProtectedDataRate:

$ref: '#/components/schemas/MaxIntegrityProtectedDataRate'

alwaysOnGranted:

type: boolean

default: false

upSecurity:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UpSecurity'

hSmfServiceInstanceId:

type: string

smfServiceInstanceId:

type: string

recoveryTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

forwardingInd:

type: boolean

default: false

psaTunnelInfo:

$ref: '#/components/schemas/TunnelInfo'

chargingId:

type: string

chargingInfo:

$ref: 'TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/ChargingInformation'

roamingChargingProfile:

$ref: 'TS32291_Nchf_ConvergedCharging.yaml#/components/schemas/RoamingChargingProfile'

nefExtBufSupportInd:

type: boolean

default: false

ipv6Index:

$ref: 'TS29519_Policy_Data.yaml#/components/schemas/IpIndex'

dnAaaAddress:

$ref: '#/components/schemas/IpAddress'

redundantPduSessionInfo:

$ref: '#/components/schemas/RedundantPduSessionInformation'

ranTunnelInfo:

$ref: '#/components/schemas/QosFlowTunnel'

addRanTunnelInfo:

type: array

items:

$ref: '#/components/schemas/QosFlowTunnel'

minItems: 1

redRanTunnelInfo:

$ref: '#/components/schemas/QosFlowTunnel'

addRedRanTunnelInfo:

type: array

items:

$ref: '#/components/schemas/QosFlowTunnel'

minItems: 1

nspuSupportInd:

type: boolean

smfBindingInfo:

type: string

required:

- pduSessionId

- dnn

- sNssai

- pduSessionType

- sessionAmbr

- qosFlowsList

ExemptionInd:

description: Exemption Indication

type: object

properties:

dnnCongestion:

type: boolean

default: false

snssaiOnlyCongestion:

type: boolean

default: false

snssaiDnnCongestion:

type: boolean

default: false

PsaInformation:

description: PSA Information

type: object

properties:

psaInd:

$ref: '#/components/schemas/PsaIndication'

dnaiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

minItems: 1

ueIpv6Prefix:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

psaUpfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

DnaiInformation:

description: DNAI Information

type: object

properties:

dnai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

noDnaiChangeInd:

type: boolean

noLocalPsaChangeInd:

type: boolean

required:

- dnai

N4Information:

description: N4 Information

type: object

properties:

n4MessageType:

$ref: '#/components/schemas/N4MessageType'

n4MessagePayload:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

n4DnaiInfo:

$ref: '#/components/schemas/DnaiInformation'

psaUpfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

required:

- n4MessageType

- n4MessagePayload

IndirectDataForwardingTunnelInfo:

description: Indirect Data Forwarding Tunnel Information

type: object

properties:

ipv4Addr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

ipv6Addr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Addr'

gtpTeid:

$ref: '#/components/schemas/Teid'

drbId:

$ref: '#/components/schemas/DrbId'

additionalTnlNb:

$ref: '#/components/schemas/AdditionalTnlNb'

required:

- gtpTeid

not:

required: [ drbId, additionalTnlNb ]

SendMoDataReqData:

description: Data within Send MO Data Request

type: object

properties:

moData:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

moExpDataCounter:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MoExpDataCounter'

ueLocation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

required:

- moData

CnAssistedRanPara:

description: SMF derived CN assisted RAN parameters tuning

type: object

properties:

stationaryIndication:

$ref: 'TS29571_CommonData.yaml#/components/schemas/StationaryIndication'

communicationDurationTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

periodicTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

scheduledCommunicationTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ScheduledCommunicationTime'

scheduledCommunicationType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ScheduledCommunicationType'

trafficProfile:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TrafficProfile'

batteryIndication:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BatteryIndication'

UlclBpInformation:

description: UL CL or BP Information

type: object

properties:

ulclBpUpfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

TransferMoDataReqData:

description: Data within Transfer MO Data Request

type: object

properties:

moData:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

moExpDataCounter:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MoExpDataCounter'

ueLocation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

required:

- moData

TransferMtDataReqData:

description: Data within Transfer MT Data Request

type: object

properties:

mtData:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

required:

- mtData

TransferMtDataError:

description: Transfer MT Data Error Response

allOf:

- $ref: '#/components/schemas/ExtProblemDetails'

- $ref: '#/components/schemas/TransferMtDataAddInfo'

TransferMtDataAddInfo:

description: Transfer MT Data Error Response Additional Information

type: object

properties:

maxWaitingTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

VplmnQos:

description: VPLMN QoS

type: object

properties:

5qi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/5Qi'

arp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Arp'

sessionAmbr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ambr'

maxFbrDl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

maxFbrUl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

guaFbrDl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

guaFbrUl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

DdnFailureSubs:

description: DDN Failure Subscription

type: object

properties:

ddnFailureSubsInd:

type: boolean

default: false

ddnFailureSubsInfoList:

type: array

items:

$ref: '#/components/schemas/DdnFailureSubInfo'

minItems: 1

DdnFailureSubInfo:

description: DDN Failure Subscription Information

type: object

required:

- notifyCorrelationId

properties:

notifyCorrelationId:

type: string

dddTrafficDescriptorList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DddTrafficDescriptor'

minItems: 1

RetrieveData:

description: Data within Retrieve Request

type: object

properties:

smallDataRateStatusReq:

type: boolean

default: false

RetrievedData:

description: Data within Retrieve Response

type: object

properties:

smallDataRateStatus:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SmallDataRateStatus'

SecurityResult:

description: Security Result

type: object

properties:

integrityProtectionResult:

$ref: '#/components/schemas/ProtectionResult'

confidentialityProtectionResult:

$ref: '#/components/schemas/ProtectionResult'

UpSecurityInfo:

description: User Plane Security Information

type: object

properties:

upSecurity:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UpSecurity'

maxIntegrityProtectedDataRateUl:

$ref: '#/components/schemas/MaxIntegrityProtectedDataRate'

maxIntegrityProtectedDataRateDl:

$ref: '#/components/schemas/MaxIntegrityProtectedDataRate'

securityResult:

$ref: '#/components/schemas/SecurityResult'

required:

- upSecurity

AlternativeQosProfile:

description: Alternative QoS Profile

type: object

properties:

index:

type: integer

minimum: 1

maximum: 8

guaFbrDl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

guaFbrUl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

packetDelayBudget:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketDelBudget'

packetErrRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketErrRate'

required:

- index

ExtProblemDetails:

description: Extended Problem Details

allOf:

- $ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

- $ref: '#/components/schemas/ProblemDetailsAddInfo'

ProblemDetailsAddInfo:

description: Problem Details Additional Information

type: object

properties:

remoteError:

type: boolean

QosMonitoringInfo:

description: QoS Monitoring Information

type: object

properties:

qosMonitoringInd:

type: boolean

default: false

IpAddress:

description: IP Address

type: object

oneOf:

- required:

- ipv4Addr

- required:

- ipv6Addr

- required:

- ipv6Prefix

properties:

ipv4Addr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

ipv6Addr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Addr'

ipv6Prefix:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

RedundantPduSessionInformation:

description: Redundant PDU Session Information

type: object

properties:

rsn:

$ref: '#/components/schemas/Rsn'

pduSessionPairId:

type: integer

minimum: 0

maximum: 255

required:

- rsn

QosFlowTunnel:

description: Tunnel Information per QoS Flow

type: object

properties:

qfiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Qfi'

minItems: 1

tunnelInfo:

$ref: '#/components/schemas/TunnelInfo'

required:

- qfiList

- tunnelInfo

TargetDnaiInfo:

description: Target DNAI Information

type: object

properties:

targetDnai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

smfSelectionType:

$ref: '#/components/schemas/SmfSelectionType'

required:

- targetDnai

- smfSelectionType

AfCoordinationInfo:

description: AF Coordination Information

type: object

properties:

sourceDnai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

sourceUeIpv4Addr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

sourceUeIpv6Prefix:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

notificationInfoList:

type: array

items:

$ref: '#/components/schemas/NotificationInfo'

minItems: 1

NotificationInfo:

description: Notification Correlation ID and Notification URI provided by the NF service consumer

type: object

properties:

notifId:

type: string

notifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

upBufferInd:

type: boolean

default: false

required:

- notifId

- notifUri

#

# SIMPLE DATA TYPES

#

ProcedureTransactionId:

description: Procedure Transaction Identifier

type: integer

minimum: 0

maximum: 255

EpsBearerId:

description: EPS Bearer Identifier

type: integer

minimum: 0

maximum: 15

EpsPdnCnxContainer:

description: UE EPS PDN Connection container from SMF to AMF

type: string

EpsBearerContainer:

description: EPS Bearer container from SMF to AMF

type: string

Teid:

type: string

description: GTP Tunnel Endpoint Identifier

pattern: '^[A-Fa-f0-9]{8}$'

EpsBearerContextStatus:

description: EPS bearer context status

type: string

pattern: '^[A-Fa-f0-9]{4}$'

DrbId:

description: Data Radio Bearer Identity

type: integer

minimum: 1

maximum: 32

AdditionalTnlNb:

description: indicates first, second or third additional indirect data forwarding tunnel

type: integer

minimum: 1

maximum: 3

ForwardingBearerContainer:

description: Forwarding Bearer Container

type: string

SecondaryRatUsageDataReportContainer:

description: Secondary Rat Usage Data Report Container

type: string

format: byte

#

# ENUMERATIONS

#

UpCnxState:

anyOf:

- type: string

enum:

- ACTIVATED

- DEACTIVATED

- ACTIVATING

- SUSPENDED

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

User Plane Connection State. Possible values are

- ACTIVATED

- DEACTIVATED

- ACTIVATING

- SUSPENDED

HoState:

anyOf:

- type: string

enum:

- NONE

- PREPARING

- PREPARED

- COMPLETED

- CANCELLED

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Handover state. Possible values are

- NONE

- PREPARING

- PREPARED

- COMPLETED

- CANCELLED

RequestType:

anyOf:

- type: string

enum:

- INITIAL_REQUEST

- EXISTING_PDU_SESSION

- INITIAL_EMERGENCY_REQUEST

- EXISTING_EMERGENCY_PDU_SESSION

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Request Type in Create (SM context) service operation. Possible values are

- INITIAL_REQUEST

- EXISTING_PDU_SESSION

- INITIAL_EMERGENCY_REQUEST

- EXISTING_EMERGENCY_PDU_SESSION

RequestIndication:

anyOf:

- type: string

enum:

- UE_REQ_PDU_SES_MOD

- UE_REQ_PDU_SES_REL

- PDU_SES_MOB

- NW_REQ_PDU_SES_AUTH

- NW_REQ_PDU_SES_MOD

- NW_REQ_PDU_SES_REL

- EBI_ASSIGNMENT_REQ

- REL_DUE_TO_5G_AN_REQUEST

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Request Indication in Update (SM context) service operation. Possible values are

- UE_REQ_PDU_SES_MOD

- UE_REQ_PDU_SES_REL

- PDU_SES_MOB

- NW_REQ_PDU_SES_AUTH

- NW_REQ_PDU_SES_MOD

- NW_REQ_PDU_SES_REL

- EBI_ASSIGNMENT_REQ

- REL_DUE_TO_5G_AN_REQUEST

NotificationCause:

anyOf:

- type: string

enum:

- QOS_FULFILLED

- QOS_NOT_FULFILLED

- UP_SEC_FULFILLED

- UP_SEC_NOT_FULFILLED

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Cause for generating a notification. Possible values are

- QOS_FULFILLED

- QOS_NOT_FULFILLED

- UP_SEC_FULFILLED

- UP_SEC_NOT_FULFILLED

Cause:

anyOf:

- type: string

enum:

- REL_DUE_TO_HO

- EPS_FALLBACK

- REL_DUE_TO_UP_SEC

- DNN_CONGESTION

- S_NSSAI_CONGESTION

- REL_DUE_TO_REACTIVATION

- 5G_AN_NOT_RESPONDING

- REL_DUE_TO_SLICE_NOT_AVAILABLE

- REL_DUE_TO_DUPLICATE_SESSION_ID

- PDU_SESSION_STATUS_MISMATCH

- HO_FAILURE

- INSUFFICIENT_UP_RESOURCES

- PDU_SESSION_HANDED_OVER

- PDU_SESSION_RESUMED

- CN_ASSISTED_RAN_PARAMETER_TUNING

- ISMF_CONTEXT_TRANSFER

- SMF_CONTEXT_TRANSFER

- REL_DUE_TO_PS_TO_CS_HO

- REL_DUE_TO_SUBSCRIPTION_CHANGE

- HO_CANCEL

- REL_DUE_TO_SLICE_NOT_AUTHORIZED

- PDU_SESSION_HAND_OVER_FAILURE

- DDN_FAILURE_STATUS

- REL_DUE_TO_CP_ONLY_NOT_APPLICABLE

- NOT_SUPPORTED_WITH_ISMF

- CHANGED_ANCHOR_SMF

- CHANGED_INTERMEDIATE_SMF

- TARGET_DNAI_NOTIFICATION

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Cause information. Possible values are

- REL_DUE_TO_HO

- EPS_FALLBACK

- REL_DUE_TO_UP_SEC

- DNN_CONGESTION

- S_NSSAI_CONGESTION

- REL_DUE_TO_REACTIVATION

- 5G_AN_NOT_RESPONDING

- REL_DUE_TO_SLICE_NOT_AVAILABLE

- REL_DUE_TO_DUPLICATE_SESSION_ID

- PDU_SESSION_STATUS_MISMATCH

- HO_FAILURE

- INSUFFICIENT_UP_RESOURCES

- PDU_SESSION_HANDED_OVER

- PDU_SESSION_RESUMED

- CN_ASSISTED_RAN_PARAMETER_TUNING

- ISMF_CONTEXT_TRANSFER

- SMF_CONTEXT_TRANSFER

- REL_DUE_TO_PS_TO_CS_HO

- REL_DUE_TO_SUBSCRIPTION_CHANGE

- HO_CANCEL

- REL_DUE_TO_SLICE_NOT_AUTHORIZED

- PDU_SESSION_HAND_OVER_FAILURE

- DDN_FAILURE_STATUS

- REL_DUE_TO_CP_ONLY_NOT_APPLICABLE

- NOT_SUPPORTED_WITH_ISMF

- CHANGED_ANCHOR_SMF

- CHANGED_INTERMEDIATE_SMF

- TARGET_DNAI_NOTIFICATION

ResourceStatus:

anyOf:

- type: string

enum:

- RELEASED

- UNCHANGED

- TRANSFERRED

- UPDATED

- ALT_ANCHOR_SMF

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Status of SM context or PDU session resource. Possible values are

- RELEASED

- UNCHANGED

- TRANSFERRED

- UPDATED

- ALT_ANCHOR_SMF

DnnSelectionMode:

anyOf:

- type: string

enum:

- VERIFIED

- UE_DNN_NOT_VERIFIED

- NW_DNN_NOT_VERIFIED

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

DNN Selection Mode. Possible values are

- VERIFIED

- UE_DNN_NOT_VERIFIED

- NW_DNN_NOT_VERIFIED

EpsInterworkingIndication:

anyOf:

- type: string

enum:

- NONE

- WITH_N26

- WITHOUT_N26

- IWK_NON_3GPP

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

EPS Interworking Indication. Possible values are

- NONE

- WITH_N26

- WITHOUT_N26

- IWK_NON_3GPP

N2SmInfoType:

anyOf:

- type: string

enum:

- PDU_RES_SETUP_REQ

- PDU_RES_SETUP_RSP

- PDU_RES_SETUP_FAIL

- PDU_RES_REL_CMD

- PDU_RES_REL_RSP

- PDU_RES_MOD_REQ

- PDU_RES_MOD_RSP

- PDU_RES_MOD_FAIL

- PDU_RES_NTY

- PDU_RES_NTY_REL

- PDU_RES_MOD_IND

- PDU_RES_MOD_CFM

- PATH_SWITCH_REQ

- PATH_SWITCH_SETUP_FAIL

- PATH_SWITCH_REQ_ACK

- PATH_SWITCH_REQ_FAIL

- HANDOVER_REQUIRED

- HANDOVER_CMD

- HANDOVER_PREP_FAIL

- HANDOVER_REQ_ACK

- HANDOVER_RES_ALLOC_FAIL

- SECONDARY_RAT_USAGE

- PDU_RES_MOD_IND_FAIL

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

N2 SM Information Type. Possible values are

- PDU_RES_SETUP_REQ

- PDU_RES_SETUP_RSP

- PDU_RES_SETUP_FAIL

- PDU_RES_REL_CMD

- PDU_RES_REL_RSP

- PDU_RES_MOD_REQ

- PDU_RES_MOD_RSP

- PDU_RES_MOD_FAIL

- PDU_RES_NTY

- PDU_RES_NTY_REL

- PDU_RES_MOD_IND

- PDU_RES_MOD_CFM

- PATH_SWITCH_REQ

- PATH_SWITCH_SETUP_FAIL

- PATH_SWITCH_REQ_ACK

- PATH_SWITCH_REQ_FAIL

- HANDOVER_REQUIRED

- HANDOVER_CMD

- HANDOVER_PREP_FAIL

- HANDOVER_REQ_ACK

- HANDOVER_RES_ALLOC_FAIL

- SECONDARY_RAT_USAGE

- PDU_RES_MOD_IND_FAIL

MaxIntegrityProtectedDataRate:

anyOf:

- type: string

enum:

- 64_KBPS

- MAX_UE_RATE

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Maximum Integrity Protected Data Rate. Possible values are

- 64_KBPS

- MAX_UE_RATE

MaReleaseIndication:

anyOf:

- type: string

enum:

- REL_MAPDU_OVER_3GPP

- REL_MAPDU_OVER_N3GPP

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Multi-Access PDU session release Indication. Possible values are

- REL_MAPDU_OVER_3GPP

- REL_MAPDU_OVER_N3GPP

SmContextType:

anyOf:

- type: string

enum:

- EPS_PDN_CONNECTION

- SM_CONTEXT

- AF_COORDINATION_INFO

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Type of SM Context information. Possible values are

- EPS_PDN_CONNECTION

- SM_CONTEXT

- AF_COORDINATION_INFO

PsaIndication:

anyOf:

- type: string

enum:

- PSA_INSERTED

- PSA_REMOVED

- PSA_INSERTED_ONLY

- PSA_REMOVED_ONLY

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Indication of whether a PSA is inserted or removed. Possible values are

- PSA_INSERTED

- PSA_REMOVED

- PSA_INSERTED_ONLY

- PSA_REMOVED_ONLY

N4MessageType:

anyOf:

- type: string

enum:

- PFCP_SES_EST_REQ

- PFCP_SES_EST_RSP

- PFCP_SES_MOD_REQ

- PFCP_SES_MOD_RSP

- PFCP_SES_DEL_REQ

- PFCP_SES_DEL_RSP

- PFCP_SES_REP_REQ

- PFCP_SES_REP_RSP

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

N4 Message Type. Possible values are

- PFCP_SES_EST_REQ

- PFCP_SES_EST_RSP

- PFCP_SES_MOD_REQ

- PFCP_SES_MOD_RSP

- PFCP_SES_DEL_REQ

- PFCP_SES_DEL_RSP

- PFCP_SES_REP_REQ

- PFCP_SES_REP_RSP

QosFlowAccessType:

anyOf:

- type: string

enum:

- 3GPP

- NON_3GPP

- 3GPP_AND_NON_3GPP

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Access type associated with a QoS Flow. Possible values are

- 3GPP

- NON_3GPP

- 3GPP_AND_NON_3GPP

UnavailableAccessIndication:

anyOf:

- type: string

enum:

- 3GA_UNAVAILABLE

- N3GA_UNAVAILABLE

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Indicates the access type of a MA PDU session that is unavailable. Possible values are

- 3GA_UNAVAILABLE

- N3GA_UNAVAILABLE

ProtectionResult:

anyOf:

- type: string

enum:

- PERFORMED

- NOT_PERFORMED

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Protection Result of the security policy indicated as "preferred". Possible values are

- PERFORMED

- NOT_PERFORMED

QosMonitoringReq:

anyOf:

- type: string

enum:

- UL

- DL

- BOTH

- NONE

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

QoS monitoring request. Possible values are

- UL

- DL

- BOTH

- NONE

Rsn:

anyOf:

- type: string

enum:

- V1

- V2

- NONE

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Redundancy Sequence Number. Possible values are

- V1

- V2

- NONE

SmfSelectionType:

anyOf:

- type: string

enum:

- CURRENT_PDU_SESSION

- NEXT_PDU_SESSION

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Smf Selection Type. Possible values are

- CURRENT_PDU_SESSION

- NEXT_PDU_SESSION

#

# HTTP request bodies

#

requestBodies:

'VsmfUpdateRequestBody':

description: representation of updates to apply to the PDU session

required: true

content:

application/json:   # message without binary body part

schema:

$ref: '#/components/schemas/VsmfUpdateData'

multipart/related:  # message with binary body part(s)

schema:

type: object

properties:     # Request parts

jsonData:

$ref: '#/components/schemas/VsmfUpdateData'

binaryDataN1SmInfoToUe:

type: string

format: binary

binaryDataN4Information:

type: string

format: binary

binaryDataN4InformationExt1:

type: string

format: binary

binaryDataN4InformationExt2:

type: string

format: binary

binaryDataN4InformationExt3:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataN1SmInfoToUe:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

binaryDataN4Information:

contentType:  application/vnd.3gpp.pfcp

headers:

Content-Id:

schema:

type: string

binaryDataN4InformationExt1:

contentType:  application/vnd.3gpp.pfcp

headers:

Content-Id:

schema:

type: string

binaryDataN4InformationExt2:

contentType:  application/vnd.3gpp.pfcp

headers:

Content-Id:

schema:

type: string

binaryDataN4InformationExt3:

contentType:  application/vnd.3gpp.pfcp

headers:

Content-Id:

schema:

type: string

'NotifyStatusRequestBody':

description: representation of the status notification

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/StatusNotification'

#

# HTTP responses

#

responses:

'VsmfUpdateResponse200':

description: successful update of a PDU session with content in the response

content:

application/json: # message without binary body part

schema:

$ref: '#/components/schemas/VsmfUpdatedData'

multipart/related:  # message with binary body part(s)

schema:

type: object

properties: # Request parts

jsonData:

$ref: '#/components/schemas/VsmfUpdatedData'

binaryDataN1SmInfoFromUe:

type: string

format: binary

binaryDataUnknownN1SmInfo:

type: string

format: binary

binaryDataN4Information:

type: string

format: binary

binaryDataN4InformationExt1:

type: string

format: binary

binaryDataN4InformationExt2:

type: string

format: binary

binaryDataN4InformationExt3:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataN1SmInfoFromUe:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

binaryDataUnknownN1SmInfo:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

binaryDataN4Information:

contentType:  application/vnd.3gpp.pfcp

headers:

Content-Id:

schema:

type: string

binaryDataN4InformationExt1:

contentType:  application/vnd.3gpp.pfcp

headers:

Content-Id:

schema:

type: string

binaryDataN4InformationExt2:

contentType:  application/vnd.3gpp.pfcp

headers:

Content-Id:

schema:

type: string

binaryDataN4InformationExt3:

contentType:  application/vnd.3gpp.pfcp

headers:

Content-Id:

schema:

type: string

'PduSessionCreateError':

description: unsuccessful creation of a PDU session

content:

application/json: # message without binary body part

schema:

$ref: '#/components/schemas/PduSessionCreateError'

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

multipart/related:  # message with binary body part(s)

schema:

type: object

properties: # Request parts

jsonData:

$ref: '#/components/schemas/PduSessionCreateError'

binaryDataN1SmInfoToUe:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataN1SmInfoToUe:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

'HsmfUpdateError':

description: unsuccessful update of a PDU session

content:

application/json: # message without binary body part

schema:

$ref: '#/components/schemas/HsmfUpdateError'

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

multipart/related:  # message with binary body part(s)

schema:

type: object

properties: # Request parts

jsonData:

$ref: '#/components/schemas/HsmfUpdateError'

binaryDataN1SmInfoToUe:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataN1SmInfoToUe:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

'VsmfUpdateError':

description: unsuccessful update of a PDU session

content:

application/json: # message without binary body part

schema:

$ref: '#/components/schemas/VsmfUpdateError'

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

multipart/related:  # message with binary body part(s)

schema:

type: object

properties: # Request parts

jsonData:

$ref: '#/components/schemas/VsmfUpdateError'

binaryDataN1SmInfoFromUe:

type: string

format: binary

binaryDataUnknownN1SmInfo:

type: string

format: binary

binaryDataN4Information:

type: string

format: binary

binaryDataN4InformationExt1:

type: string

format: binary

binaryDataN4InformationExt2:

type: string

format: binary

binaryDataN4InformationExt3:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataN1SmInfoFromUe:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

binaryDataUnknownN1SmInfo:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

binaryDataN4Information:

contentType:  application/vnd.3gpp.pfcp

headers:

Content-Id:

schema:

type: string

binaryDataN4InformationExt1:

contentType:  application/vnd.3gpp.pfcp

headers:

Content-Id:

schema:

type: string

binaryDataN4InformationExt2:

contentType:  application/vnd.3gpp.pfcp

headers:

Content-Id:

schema:

type: string

binaryDataN4InformationExt3:

contentType:  application/vnd.3gpp.pfcp

headers:

Content-Id:

schema:

type: string

'400':

description: Bad request

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ExtProblemDetails'

'401':

description: Unauthorized

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ExtProblemDetails'

'403':

description: Forbidden

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ExtProblemDetails'

'404':

description: Not Found

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ExtProblemDetails'

'413':

description: Payload Too Large

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ExtProblemDetails'

'415':

description: Unsupported Media Type

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ExtProblemDetails'

'429':

description: Too Many Requests

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ExtProblemDetails'

'500':

description: Internal Server Error

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ExtProblemDetails'

'503':

description: Service Unavailable

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ExtProblemDetails'

Annex B (Informative):
HTTP Multipart Messages

## B.1	Example of HTTP multipart message

### B.1.1	General

This clause provides a (partial) example of HTTP multipart message. The example does not aim to be a complete representation of the HTTP message, e.g. additional information or headers can be included.

This Annex is informative and the normative descriptions in this specification prevail over the description in this Annex if there is any difference.

### B.1.2	Example HTTP multipart message with N1 SM Message binary data

POST /example.com/nsmf-pdusession/v1/sm-contexts HTTP/2

Content-Type: multipart/related; type="application/json"; boundary=----Boundary

Content-Length: xyz

------Boundary

Content-Type: application/json

{

"supi": "imsi-<IMSI>",

"pduSessionId": 235,

"dnn": "<DNN>",

"sNssai": {

"sst": 0

},

"servingNfId": "<AMF Identifier>",

"n1SmMsg": {

"contentId": "n1msg"

},

"anType": "3GPP_ACCESS",

"smContextStatusUri": "<URI>"

}

------Boundary

Content-Type: application/vnd.3gpp.5gnas

Content-Id: n1msg

{ … N1 SM Message binary data …}

------Boundary

Annex C (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2017-10 | CT4#80 | C4-175050 |  |  |  | Initial Draft. | 0.1.0 |
| 2017-10 | CT4#80 | C4-175392 |  |  |  | Inclusion of pCRs agreed during CT4#80. | 0.2.0 |
| 2017-12 | CT4#81 | C4-176435 |  |  |  | Inclusion of pCRs agreed during CT4#81. | 0.3.0 |
| 2018-01 | CT4#82 | C4-181389 |  |  |  | Inclusion of pCRs agreed during CT4#82. | 0.4.0 |
| 2018-03 | CT4#83 | C4-182432 |  |  |  | Inclusion of pCRs agreed during CT4#83. | 0.5.0 |
| 2018-03 | CT#79 | CP-180030 |  |  |  | Presented for information | 1.0.0 |
| 2018-04 | CT4#84 | C4-183514 |  |  |  | Inclusion of pCRs agreed during CT4#84. | 1.1.0 |
| 2018-05 | CT4#85 | C4-184619 |  |  |  | Inclusion of pCRs agreed during CT4#85. | 1.2.0 |
| 2018-06 | CT#80 | CP-181100 |  |  |  | Presented for approval | 2.0.0 |
| 2018-06 | CT#80 |  |  |  |  | Approved in CT#80. | 15.0.0 |
| 2018-09 | CT#81 | CP-182055 | 0002 |  | F | Corrections to missing application errors in API response body description | 15.1.0 |
| 2018-09 | CT#81 | CP-182068 | 0006 |  | B | Add support for 5G Trace | 15.1.0 |
| 2018-09 | CT#81 | CP-182055 | 0007 |  | F | Error Responses | 15.1.0 |
| 2018-09 | CT#81 | CP-182055 | 0015 |  | F | Network Sharing | 15.1.0 |
| 2018-09 | CT#81 | CP-182055 | 0016 |  | F | RAT Type in Create and Update (SM Context) service operations | 15.1.0 |
| 2018-09 | CT#81 | CP-182055 | 0001 | 1 | F | Application specific error cause for Not Acceptable Integrity Protection Max Data Rate | 15.1.0 |
| 2018-09 | CT#81 | CP-182055 | 0008 | 1 | F | EBI Assignment for Home Routed PDU sessions | 15.1.0 |
| 2018-09 | CT#81 | CP-182055 | 0010 | 1 | F | Returning the H-SMF URI to the AMF | 15.1.0 |
| 2018-09 | CT#81 | CP-182055 | 0014 | 1 | F | N2 SM signalling | 15.1.0 |
| 2018-09 | CT#81 | CP-182055 | 0017 | 1 | F | Supporting AMF changes | 15.1.0 |
| 2018-09 | CT#81 | CP-182055 | 0018 | 1 | F | VPLMN S-NSSAI during mobility from EPS to 5GC with N26 | 15.1.0 |
| 2018-09 | CT#81 | CP-182055 | 0009 | 2 | F | HTTP message retransmissions and requests colliding with existing contexts | 15.1.0 |
| 2018-09 | CT#81 | CP-182055 | 0029 |  | F | Rejected PDU session during Xn handover | 15.1.0 |
| 2018-09 | CT#81 | CP-182055 | 0033 |  | F | Description of Structured data types | 15.1.0 |
| 2018-09 | CT#81 | CP-182055 | 0034 |  | F | Handling of LADN service area during handovers | 15.1.0 |
| 2018-09 | CT#81 | CP-182055 | 0036 |  | F | Mapping to stage 2 service operation names | 15.1.0 |
| 2018-09 | CT#81 | CP-182055 | 0039 |  | F | Stateless AMF support updates | 15.1.0 |
| 2018-09 | CT#81 | CP-182055 | 0040 |  | F | QoS rule structure improvement | 15.1.0 |
| 2018-09 | CT#81 | CP-182055 | 0020 | 1 | F | NRF URI for PCF Selection | 15.1.0 |
| 2018-09 | CT#81 | CP-182055 | 0022 | 1 | F | BackUp AMF Info | 15.1.0 |
| 2018-09 | CT#81 | CP-182055 | 0031 | 1 | F | NGAP causes | 15.1.0 |
| 2018-09 | CT#81 | CP-182055 | 0024 | 1 | F | EPS Interworking Ind | 15.1.0 |
| 2018-09 | CT#81 | CP-182050 | 0021 | 2 | F | References to common NonDynamic5Qi and Dynamic5Qi data types | 15.1.0 |
| 2018-09 | CT#81 | CP-182055 | 0025 | 1 | F | Not Allowed Slice | 15.1.0 |
| 2018-09 | CT#81 | CP-182055 | 0005 | 3 | F | N2 SM Information Type Definition | 15.1.0 |
| 2018-09 | CT#81 | CP-182055 | 0028 | 1 | F | OpenAPI corrections | 15.1.0 |
| 2018-09 | CT#81 | CP-182055 | 0032 | 1 | F | Age of User Location | 15.1.0 |
| 2018-09 | CT#81 | CP-182055 | 0037 | 1 | F | Detecting SMF Failure and Restart | 15.1.0 |
| 2018-09 | CT#81 | CP-182055 | 0041 |  | F | PresenceState reference | 15.1.0 |
| 2018-09 | CT#81 | CP-182055 | 0030 | 2 | F | URIs of created SM context and PDU session resources | 15.1.0 |
| 2018-09 | CT#81 | CP-182055 | 0023 | 2 | F | 5G MM Cause | 15.1.0 |
| 2018-09 | CT#81 | CP-182055 | 0042 |  | F | API version number update | 15.1.0 |
| 2018-12 | CT#82 | CP-183013 | 0044 | 2 | F | IndDirect Forwarding Flag | 15.2.0 |
| 2018-12 | CT#82 | CP-183013 | 0045 | 1 | F | Data Forwarding IE | 15.2.0 |
| 2018-12 | CT#82 | CP-183013 | 0047 |  | F | Alignments with NAS 5GS Session Management | 15.2.0 |
| 2018-12 | CT#82 | CP-183013 | 0048 | 1 | F | Alignments with NGAP | 15.2.0 |
| 2018-12 | CT#82 | CP-183013 | 0049 |  | F | Corrections to N2 Handover and Inter-AMF change or mobility procedures | 15.2.0 |
| 2018-12 | CT#82 | CP-183013 | 0050 | 2 | F | Indication of Access Type can be changed | 15.2.0 |
| 2018-12 | CT#82 | CP-183013 | 0051 |  | F | Roaming Charging Profile negotiation for Home Routed PDU sessions | 15.2.0 |
| 2018-12 | CT#82 | CP-183013 | 0052 |  | F | Service restart detection by direct signalling between NFs | 15.2.0 |
| 2018-12 | CT#82 | CP-183013 | 0053 |  | F | Use of the serviceName attribute by the Notify SM Context Status service operation | 15.2.0 |
| 2018-12 | CT#82 | CP-183013 | 0054 |  | F | Cardinality of arrays | 15.2.0 |
| 2018-12 | CT#82 | CP-183013 | 0055 |  | F | Data type of serviceName attribute | 15.2.0 |
| 2018-12 | CT#82 | CP-183013 | 0056 |  | F | HTTP status code "501 Not Implemented" | 15.2.0 |
| 2018-12 | CT#82 | CP-183013 | 0057 |  | F | Case conventions | 15.2.0 |
| 2018-12 | CT#82 | CP-183013 | 0058 |  | F | Resource URI structure of Nsmf_PDUSession service | 15.2.0 |
| 2018-12 | CT#82 | CP-183013 | 0059 |  | F | EPS bearer identity and data type definitions | 15.2.0 |
| 2018-12 | CT#82 | CP-183013 | 0060 | 3 | F | EPC Interworking in Home Routed Roaming | 15.2.0 |
| 2018-12 | CT#82 | CP-183013 | 0061 | 1 | F | Correction to EPS Interworking Indication | 15.2.0 |
| 2018-12 | CT#82 | CP-183013 | 0063 |  | F | Presence condition of targetServingNfId | 15.2.0 |
| 2018-12 | CT#82 | CP-183013 | 0064 | 2 | F | PDU session removal during interworking with N26 | 15.2.0 |
| 2018-12 | CT#82 | CP-183013 | 0067 | 1 | F | Complete the SSC mode 3 in the home routed roaming scenario | 15.2.0 |
| 2018-12 | CT#82 | CP-183013 | 0068 | 2 | F | Clarification on EBI allocation | 15.2.0 |
| 2018-12 | CT#82 | CP-183013 | 0069 | 1 | F | Correct description of Request type | 15.2.0 |
| 2018-12 | CT#82 | CP-183190 | 0071 | 3 | F | PDU Session Id Duplication | 15.2.0 |
| 2018-12 | CT#82 | CP-183013 | 0072 | 1 | F | Service Instance | 15.2.0 |
| 2018-12 | CT#82 | CP-183013 | 0075 | 2 | F | A new cause value 'PDU_Session_Status_Mismatch' for Cause Data Type | 15.2.0 |
| 2018-12 | CT#82 | CP-183013 | 0076 | 2 | F | UDM group Id | 15.2.0 |
| 2018-12 | CT#82 | CP-183013 | 0077 | 2 | F | Always-on PDU sessions | 15.2.0 |
| 2018-12 | CT#82 | CP-183013 | 0078 | 2 | F | Handover Failure scenarios | 15.2.0 |
| 2018-12 | CT#82 | CP-183013 | 0079 |  | F | Release of PDU session during 5GS to EPS mobility | 15.2.0 |
| 2018-12 | CT#82 | CP-183013 | 0080 | 1 | F | Mandatory HTTP status codes | 15.2.0 |
| 2018-12 | CT#82 | CP-183013 | 0081 | 1 | F | Clarification of API URI definition | 15.2.0 |
| 2018-12 | CT#82 | CP-183013 | 0082 |  | F | API version | 15.2.0 |
| 2018-12 | CT#82 | CP-183013 | 0083 |  | F | externalDocs field in OpenAPI document | 15.2.0 |
| 2018-12 | CT#82 | CP-183013 | 0084 |  | F | Location Header | 15.2.0 |
| 2018-12 | CT#82 | CP-183169 | 0085 | 1 | F | Regular Expression Patterns | 15.2.0 |
| 2018-12 | CT#82 | CP-183013 | 0086 |  | F | Correct reference of EPS to 5GS handover | 15.2.0 |
| 2018-12 | CT#82 | CP-183013 | 0087 |  | F | Secondary RAT usage data reporting | 15.2.0 |
| 2018-12 | CT#82 | CP-183013 | 0088 | 1 | F | OAuth2 requirements | 15.2.0 |
| 2018-12 | CT#82 | CP-183081 | 0089 |  | F | SMF Network Function Instance | 15.2.0 |
| 2018-12 |  |  |  |  |  | MaxIntegrityProtectedDataRate spare quote fixed in yaml-file | 15.2.1 |
| 2019-03 | CT#83 | CP-190018 | 0090 |  | F | OpenAPI correction | 15.3.0 |
| 2019-03 | CT#83 | CP-190018 | 0091 | 1 | F | Secondary RAT usage reporting | 15.3.0 |
| 2019-03 | CT#83 | CP-190018 | 0092 | 1 | F | Target ID during inter NG-RAN node N2 based handover and EPS to 5GS Handover | 15.3.0 |
| 2019-03 | CT#83 | CP-190018 | 0093 |  | F | Indirect data forwarding timer | 15.3.0 |
| 2019-03 | CT#83 | CP-190018 | 0095 |  | F | API version update | 15.3.0 |
| 2019-03 | CT#83 | CP-190018 | 0096 | 2 | F | Clarification on the use of pduSessionsActivateList during EPS to 5GS Idle Mode Mobility Procedure. | 15.3.0 |
| 2019-03 | CT#83 | CP-190018 | 0097 |  | F | Cause information | 15.3.0 |
| 2019-03 | CT#83 | CP-190018 | 0098 | 2 | F | Update ReleaseSMContext Service Operation Description for PDU Session Release due to Change of Set of Network Slices | 15.3.0 |
| 2019-03 | CT#83 | CP-190018 | 0099 | 1 | F | Trigger Conditions for SMContext Update | 15.3.0 |
| 2019-03 | CT#83 | CP-190018 | 0100 | 1 | F | Trigger Conditions for SMContext Release | 15.3.0 |
| 2019-03 | CT#83 | CP-190018 | 0102 | 1 | F | Status Notify for HO | 15.3.0 |
| 2019-03 | CT#83 | CP-190018 | 0103 | 1 | F | Provide Resource URI before PDU Session Creation Response | 15.3.0 |
| 2019-03 | CT#83 | CP-190018 | 0104 | 1 | F | Allocated EBIs during EPS to 5GS Preparation with N26 | 15.3.0 |
| 2019-06 | CT#84 | CP-191029 | 0106 | 1 | F | Change of access type in home routed roaming scenario | 15.4.0 |
| 2019-06 | CT#84 | CP-191029 | 0107 | 1 | F | EPS bearer synchronization upon EPS to 5GS idle mode mobility using N26 | 15.4.0 |
| 2019-06 | CT#84 | CP-191029 | 0109 |  | F | Removing multiple redundant appearances of major version number | 15.4.0 |
| 2019-06 | CT#84 | CP-191029 | 0112 | 1 | F | Application Error "S-NSSAI_CONGESTION" | 15.4.0 |
| 2019-06 | CT#84 | CP-191029 | 0113 | 3 | F | Clarification on Sending SMContextStatusNotify | 15.4.0 |
| 2019-06 | CT#84 | CP-191029 | 0117 | 1 | F | Essential Correction of HPLMN SNSSAI during SM Context Creation | 15.4.0 |
| 2019-06 | CT#84 | CP-191029 | 0118 | 2 | F | Handover Preparation Failure | 15.4.0 |
| 2019-06 | CT#84 | CP-191029 | 0129 | 2 | F | Storage of OpenAPI specification files | 15.4.0 |
| 2019-06 | CT#84 | CP-191029 | 0133 | 2 | F | Location header in redirect response | 15.4.0 |
| 2019-06 | CT#84 | CP-191029 | 0135 | 2 | F | Correct Nsmf_PDUSession_Create to support Mobility Restriction | 15.4.0 |
| 2019-06 | CT#84 | CP-191029 | 0139 | 1 | F | Correction of Procedure of Handover using UpdateSmContext | 15.4.0 |
| 2019-06 | CT#84 | CP-191029 | 0141 | 1 | F | Secondary RAT Usage reporting at PDU session level | 15.4.0 |
| 2019-06 | CT#84 | CP-191029 | 0142 |  | F | Mapped EPS bearer contexts in n1SmInfoFromUe | 15.4.0 |
| 2019-06 | CT#84 | CP-191029 | 0143 | 1 | F | Target ID sent to the SMF | 15.4.0 |
| 2019-06 | CT#84 | CP-191029 | 0144 |  | F | Copyright Note in YAML file | 15.4.0 |
| 2019-06 | CT#84 | CP-191029 | 0145 | 1 | F | Correction on GPSI for PDU session | 15.4.0 |
| 2019-06 | CT#84 | CP-191029 | 0148 |  | F | 3GPP TS 29.502 API version update | 15.4.0 |
| 2019-06 | CT#84 | CP-191048 | 0147 |  | B | 3GPP TS 29.502 API version update | 16.0.0 |
| 2019-06 | CT#84 | CP-191049 | 0116 | 4 | F | Updates to CreateSMContext for eNS Support | 16.0.0 |
| 2019-06 | CT#84 | CP-191050 | 0130 | 4 | B | PDU Session Establishment support Control Plane CIoT Optimization | 16.0.0 |
| 2019-06 | CT#84 | CP-191051 | 0115 | 2 | B | ATSSS: MA-PDU Session Establishment | 16.0.0 |
| 2019-06 | CT#84 | CP-191051 | 0136 | 2 | B | MA PDU Session Update | 16.0.0 |
| 2019-06 | CT#84 | CP-191051 | 0137 | 5 | B | MA PDU Session Release over a Single Access | 16.0.0 |
| 2019-06 | CT#84 | CP-191054 | 0108 | 1 | B | Updates to reference model for ETSUN | 16.0.0 |
| 2019-06 | CT#84 | CP-191054 | 0124 | 1 | B | Update SM context service operation for additional PDU Session Anchor and Branching Point or UL CL controlled by I-SMF | 16.0.0 |
| 2019-06 | CT#84 | CP-191054 | 0123 | 2 | B | Update Retrieve SM Context service operation from Source I-SMF or SMF | 16.0.0 |
| 2019-06 | CT#84 | CP-191054 | 0110 | 2 | B | SM context transfer between I-SMFs or V-SMFs | 16.0.0 |
| 2019-06 | CT#84 | CP-191054 | 0122 | 2 | B | Update Release SM Context service operation for I-SMF change and removal | 16.0.0 |
| 2019-06 | CT#84 | CP-191054 | 0121 | 6 | B | Update Create SM context service operation for the I-SMF insertion and change | 16.0.0 |
| 2019-06 | CT#84 | CP-191054 | 0134 | 3 | B | Update Create SM context service operation for the I-SMF insertion and change | 16.0.0 |
| 2019-06 | CT#84 | CP-191059 | 0140 | 1 | F | EPS Interworking Indication for N3GPP | 16.0.0 |
| 2019-06 | CT#84 | CP-191059 | 0131 | 3 | F | Exemption Indication | 16.0.0 |
| 2019-09 | CT#85 | CP-192193 | 0149 | 3 | B | I-SMF insertion implications on Nsmf_PDUSession_Update across N16a | 16.1.0 |
| 2019-09 | CT#85 | CP-192193 | 0151 | 3 | B | Implications of Policy Update Procedures with I-SMF on Nsmf_PDUSession_Update across N16a | 16.1.0 |
| 2019-09 | CT#85 | CP-192193 | 0153 | 1 | B | Implications of I-SMF insertion on Create service operation | 16.1.0 |
| 2019-09 | CT#85 | CP-192193 | 0154 |  | B | PDU session establishment with I-SMF insertion - Create SM Context service operation | 16.1.0 |
| 2019-09 | CT#85 | CP-192193 | 0155 | 1 | B | Transfer of NAS SM Information between I-SMF and SMF | 16.1.0 |
| 2019-09 | CT#85 | CP-192193 | 0156 |  | B | Updates to resources model and operations for PDU sessions with an I-SMF | 16.1.0 |
| 2019-09 | CT#85 | CP-192193 | 0157 |  | B | Release of a PDU session with an I-SMF | 16.1.0 |
| 2019-09 | CT#85 | CP-192193 | 0158 |  | B | Notify status of a PDU session with an I-SMF | 16.1.0 |
| 2019-09 | CT#85 | CP-192193 | 0159 |  | B | Update of a PDU session with an I-SMF | 16.1.0 |
| 2019-09 | CT#85 | CP-192029 | 0160 | 1 | B | Insertion of a PSA and UL CL/BP into the data path of a PDU session with an I-SMF | 16.1.0 |
| 2019-09 | CT#85 | CP-192193 | 0161 | 1 | B | Removal of a PSA and UL CL/BP from the data path of a PDU session with an I-SMF | 16.1.0 |
| 2019-09 | CT#85 | CP-192193 | 0162 | 1 | B | Change of a PSA for IPv6 multi-homing or UL CL controlled by I-SMF | 16.1.0 |
| 2019-09 | CT#85 | CP-192193 | 0163 | 1 | B | N4 notifications for traffic usage reporting from I-SMF to SMF | 16.1.0 |
| 2019-09 | CT#85 | CP-192193 | 0164 |  | B | Service Request with I-SMF insertion/change/removal or with V-SMF change | 16.1.0 |
| 2019-09 | CT#85 | CP-192193 | 0165 |  | B | Retrieve SM Context Request to or from SMF | 16.1.0 |
| 2019-09 | CT#85 | CP-192193 | 0166 |  | B | N26 based interworking with I-SMF | 16.1.0 |
| 2019-09 | CT#85 | CP-192193 | 0168 | 3 | B | End Marker indication during Xn/N2 handover for HR roaming and for I-SMF insertion | 16.1.0 |
| 2019-09 | CT#85 | CP-192194 | 0169 | 1 | B | SMF selection with Delegated Discovery | 16.1.0 |
| 2019-09 | CT#85 | CP-192193 | 0171 | 1 | B | PDUSession_CreateSMContext – Parameters Updating | 16.1.0 |
| 2019-09 | CT#85 | CP-192193 | 0172 | 1 | B | ReleaseSMContext – I-SMF Only Indication | 16.1.0 |
| 2019-09 | CT#85 | CP-192193 | 0173 | 1 | B | PDUSession_UpdateSMContext - N3/N9 Forwarding Tunnel Info | 16.1.0 |
| 2019-09 | CT#85 | CP-192193 | 0174 | 1 | B | Missing "UE presence in LADN service area" attribute | 16.1.0 |
| 2019-09 | CT#85 | CP-192193 | 0175 | 2 | B | Missing "Secondary RAT usage data" attributes | 16.1.0 |
| 2019-09 | CT#85 | CP-192193 | 0177 | 1 | F | Correction to Retrieve SM Context service operation during N2 based HO | 16.1.0 |
| 2019-09 | CT#85 | CP-192102 | 0179 | 2 | A | N2 Handover Preparation Failure | 16.1.0 |
| 2019-09 | CT#85 | CP-192102 | 0181 |  | A | Release of Indirect Data Forwarding Tunnels during 5GS to EPS handover | 16.1.0 |
| 2019-09 | CT#85 | CP-192123 | 0182 |  | B | Handling of requests which collide with an existing SM context / PDU session context | 16.1.0 |
| 2019-09 | CT#85 | CP-192128 | 0183 |  | F | Deactivation of the User Plane connection during handover procedure | 16.1.0 |
| 2019-09 | CT#85 | CP-192134 | 0184 | 1 | B | Clarification on Additional Access Type | 16.1.0 |
| 2019-09 | CT#85 | CP-192134 | 0185 | 1 | B | Indications for MA PDU Session | 16.1.0 |
| 2019-09 | CT#85 | CP-192193 | 0186 | 1 | B | Item 14 - Update reference to stage 2 procedure for I-SMF insertion, change, removal | 16.1.0 |
| 2019-09 | CT#85 | CP-192133 | 0187 |  | B | Serving Network Identifier for Stand-alone Non-Public Networks | 16.1.0 |
| 2019-09 | CT#85 | CP-192193 | 0188 | 1 | B | ETSUN_Create service operation for buffered data forwarding | 16.1.0 |
| 2019-09 | CT#85 | CP-192193 | 0189 | 1 | B | ETSUN_SM Context Request with buffered data forwarding | 16.1.0 |
| 2019-09 | CT#85 | CP-192193 | 0190 | 1 | B | ETSUN_Update Service Operation with I-V-SMF change | 16.1.0 |
| 2019-09 | CT#85 | CP-192193 | 0191 | 1 | B | ETSUN_Update SM Context for buffered data forwarding | 16.1.0 |
| 2019-09 | CT#85 | CP-192193 | 0192 | 4 | B | Missing attributes in SM Context | 16.1.0 |
| 2019-09 | CT#85 | CP-192193 | 0194 | 2 | B | ETSUN alignments to ReleaseSMContext service operation | 16.1.0 |
| 2019-09 | CT#85 | CP-192193 | 0195 | 2 | B | ETSUN alignments to RetrieveSMContext service operation | 16.1.0 |
| 2019-09 | CT#85 | CP-192132 | 0198 | 1 | B | Small Data Rate Control Status during PDU session establishment | 16.1.0 |
| 2019-09 | CT#85 | CP-192123 | 0199 |  | F | CRLF between Header fields and Data | 16.1.0 |
| 2019-09 | CT#85 | CP-192120 | 0201 |  | F | 3GPP TS 29.502 API version update | 16.1.0 |
| 2019-10 |  |  |  |  |  | Corrupted references corrected | 16.1.1 |
| 2019-12 | CT#86 | CP-193057 | 0202 | 1 | F | Delegated discovery parameter mapping in stage 3 | 16.2.0 |
| 2019-12 | CT#86 | CP-193056 | 0203 |  | B | N4 information in Update Request / Response | 16.2.0 |
| 2019-12 | CT#86 | CP-193056 | 0204 | 1 | B | Indirect data forwarding tunnels for N2 handover with I-SMF | 16.2.0 |
| 2019-12 | CT#86 | CP-193056 | 0205 |  | B | Contents of Sm context in SmContextRetrievedData | 16.2.0 |
| 2019-12 | CT#86 | CP-193051 | 0206 | 1 | B | MA PDU session Accepted indication | 16.2.0 |
| 2019-12 | CT#86 | CP-193051 | 0207 | 1 | B | Establishment of N9 tunnel per access network for a MA PDU session | 16.2.0 |
| 2019-12 | CT#86 | CP-193051 | 0209 |  | B | ATSSS container: ATSSS rules, Measurement Assistance Info, Network Steering Info | 16.2.0 |
| 2019-12 | CT#86 | CP-193036 | 0210 | 1 | F | Forwarding of Origination Time Stamp to PCF | 16.2.0 |
| 2019-12 | CT#86 | CP-193049 | 0211 | 1 | B | Connection suspend | 16.2.0 |
| 2019-12 | CT#86 | CP-193049 | 0212 | 2 | B | Connection resume | 16.2.0 |
| 2019-12 | CT#86 | CP-193051 | 0213 | 1 | B | MA PDU Network Upgrade Allowed indication | 16.2.0 |
| 2019-12 | CT#86 | CP-193051 | 0215 | 2 | B | Access Type Associated with QoS Flow | 16.2.0 |
| 2019-12 | CT#86 | CP-193056 | 0217 | 1 | B | Update Description for I-SMF Scenario | 16.2.0 |
| 2019-12 | CT#86 | CP-193057 | 0218 | 1 | B | Delegated Discovery Parameters Conveyance in HTTP/2 Headers | 16.2.0 |
| 2019-12 | CT#86 | CP-193049 | 0220 | 2 | B | Serving PLMN Rate Control | 16.2.0 |
| 2019-12 | CT#86 | CP-193049 | 0222 | 1 | B | APN Rate Control attribute during PDU Session Establishment across N11 and N16 | 16.2.0 |
| 2019-12 | CT#86 | CP-193128 | 0223 | 4 | B | Adding Rate Control attributes to the Release operations | 16.2.0 |
| 2019-12 | CT#86 | CP-193049 | 0224 | 2 | B | Adding Rate Control attributes to the Update operations | 16.2.0 |
| 2019-12 | CT#86 | CP-193056 | 0226 |  | F | smContextRef attribute in SmContextCreateData | 16.2.0 |
| 2019-12 | CT#86 | CP-193049 | 0227 |  | B | Control Plane Only Indicator | 16.2.0 |
| 2019-12 | CT#86 | CP-193168 | 0228 | 2 | B | Mobile Originated Data Transfer for Control Plane CIoT 5GS Optimisation | 16.2.0 |
| 2019-12 | CT#86 | CP-193056 | 0230 | 1 | B | Secondary RAT Data Usage Report | 16.2.0 |
| 2019-12 | CT#86 | CP-193051 | 0231 | 1 | B | MA PDU request indication | 16.2.0 |
| 2019-12 | CT#86 | CP-193129 | 0232 | 4 | B | SMF derived CN assisted RAN parameters turning | 16.2.0 |
| 2019-12 | CT#86 | CP-193026 | 0234 | 1 | A | Secondary RAT Data Usage Report | 16.2.0 |
| 2019-12 | CT#86 | CP-193036 | 0235 | 1 | F | Add Reference to 3GPP TS 29.524 | 16.2.0 |
| 2019-12 | CT#86 | CP-193057 | 0236 | 3 | B | SM Transfer indication | 16.2.0 |
| 2019-12 | CT#86 | CP-193057 | 0237 | 2 | B | SM Context Transfer between SMFs | 16.2.0 |
| 2019-12 | CT#86 | CP-193056 | 0238 | 1 | F | Clarification to the buffered data handling in PduSessionCreateData | 16.2.0 |
| 2019-12 | CT#86 | CP-193056 | 0241 | 2 | B | Missing DNAI list across N16a | 16.2.0 |
| 2019-12 | CT#86 | CP-193061 | 0243 | 2 | B | PDU Session Release due to SRVCC | 16.2.0 |
| 2019-12 | CT#86 | CP-193036 | 0244 | 1 | F | Editorial Correction | 16.2.0 |
| 2019-12 | CT#86 | CP-193044 | 0246 |  | F | 3GPP TS 29.502 API version update | 16.2.0 |
| 2020-03 | CT#87 | CP-200039 | 0247 | 2 | F | Add Corresponding API descriptions in clause 5.1 | 16.3.0 |
| 2020-03 | CT#87 | CP-200016 | 0248 | 3 | F | PCF Set ID and PCF Group ID | 16.3.0 |
| 2020-03 | CT#87 | CP-200035 | 0249 | 3 | B | N3 terminations of W-AGF, TNGF and TWIF for UPF selection | 16.3.0 |
| 2020-03 | CT#87 | CP-200035 | 0250 | 2 | B | Adding references to stage 2 procedures for wireline access | 16.3.0 |
| 2020-03 | CT#87 | CP-200042 | 0251 | 4 | F | Clarification to 3GPP vendor specific content subtypes | 16.3.0 |
| 2020-03 | CT#87 | CP-200033 | 0252 | 2 | F | Clarification to apnRateStatus attribute description | 16.3.0 |
| 2020-03 | CT#87 | CP-200017 | 0253 | 2 | F | Editor's note related to change of PSA | 16.3.0 |
| 2020-03 | CT#87 | CP-200017 | 0254 | 2 | F | Handover between ePDG/EPS to 5GS with I-SMF insertion or removal | 16.3.0 |
| 2020-03 | CT#87 | CP-200017 | 0255 | 2 | F | Missing DTSSA applicability | 16.3.0 |
| 2020-03 | CT#87 | CP-200017 | 0256 | 3 | F | Feature negotiation extension to support change of AMF, V-SMF or I-SMF | 16.3.0 |
| 2020-03 | CT#87 | CP-200031 | 0257 | 2 | B | Reporting that an access of a MA PDU session is unavailable | 16.3.0 |
| 2020-03 | CT#87 | CP-200031 | 0258 | 3 | B | Request Type parameter of a MA-PDU session | 16.3.0 |
| 2020-03 | CT#87 | CP-200033 | 0259 | 2 | B | Scope and Services offered by SMF | 16.3.0 |
| 2020-03 | CT#87 | CP-200020 | 0260 | 3 | F | DNN encoding in SMF PDUSession API | 16.3.0 |
| 2020-03 | CT#87 | CP-200017 | 0261 | 3 | B | Home Provided Charging ID and Roaming Charging Profile | 16.3.0 |
| 2020-03 | CT#87 | CP-200039 | 0262 | 2 | F | Correction - formatting consistency | 16.3.0 |
| 2020-03 | CT#87 | CP-200017 | 0263 | 2 | B | Corrections on the descriptions for the data types related to I-SMF | 16.3.0 |
| 2020-03 | CT#87 | CP-200031 | 0264 |  | F | maNwUpgradeInd in PduSessionCreateData | 16.3.0 |
| 2020-03 | CT#87 | CP-200031 | 0265 |  | F | anType in TunnelInfo | 16.3.0 |
| 2020-03 | CT#87 | CP-200042 | 0266 |  | F | Void a non-existent clause | 16.3.0 |
| 2020-03 | CT#87 | CP-200042 | 0268 |  | B | PDU Session Release Due to UE Subscription Change | 16.3.0 |
| 2020-03 | CT#87 | CP-200017 | 0269 | 1 | B | DNAI list | 16.3.0 |
| 2020-03 | CT#87 | CP-200017 | 0270 | 1 | B | End Marker Indication | 16.3.0 |
| 2020-03 | CT#87 | CP-200017 | 0271 | 1 | B | hoCompleteIndication in 5GS to EPS handover | 16.3.0 |
| 2020-03 | CT#87 | CP-200017 | 0272 |  | B | Notify Ipv6MultiHomingInd during I-SMF change procedure | 16.3.0 |
| 2020-03 | CT#87 | CP-200055 | 0273 | 1 | A | Linked EPS Bearer ID | 16.3.0 |
| 2020-03 | CT#87 | CP-200017 | 0274 | 1 | B | qosRules in SM Context | 16.3.0 |
| 2020-03 | CT#87 | CP-200017 | 0275 | 1 | B | Definition of smContextRef and Target ID | 16.3.0 |
| 2020-03 | CT#87 | CP-200053 | 0276 | 1 | A | EPS bearer ID correction | 16.3.0 |
| 2020-03 | CT#87 | CP-200020 | 0277 | 1 | F | Support 504 error code in retrieve SM Context service operation | 16.3.0 |
| 2020-03 | CT#87 | CP-200020 | 0278 | 1 | B | Support PDN type Ethernet at 5GS to EPS mobility with N26 | 16.3.0 |
| 2020-03 | CT#87 | CP-200017 | 0290 | 2 | B | UPF Instance ID | 16.3.0 |
| 2020-03 | CT#87 | CP-200017 | 0292 | 1 | F | Handover Cancel | 16.3.0 |
| 2020-03 | CT#87 | CP-200033 | 0293 | 1 | B | MO Data Transfer N16 | 16.3.0 |
| 2020-03 | CT#87 | CP-200033 | 0294 | 1 | B | MO Exception Data Delivery CP CIOT | 16.3.0 |
| 2020-03 | CT#87 | CP-200033 | 0295 | 1 | B | MO Exception Data Delivery UP CIOT | 16.3.0 |
| 2020-03 | CT#87 | CP-200033 | 0296 | 1 | B | MT Data Transfer N16 | 16.3.0 |
| 2020-03 | CT#87 | CP-200042 | 0297 | 1 | F | VPLMN QoS | 16.3.0 |
| 2020-03 | CT#87 | CP-200042 | 0298 |  | F | Multi-part message example | 16.3.0 |
| 2020-03 | CT#87 | CP-200033 | 0299 | 1 | B | NEF Extended Buffering Supporting Indication | 16.3.0 |
| 2020-03 | CT#87 | CP-200033 | 0300 |  | F | Removal of Serving PLMN Rate Control | 16.3.0 |
| 2020-03 | CT#87 | CP-200034 | 0301 | 2 | C | New cause value for NSSAA failure and revocation | 16.3.0 |
| 2020-03 | CT#87 | CP-200138 | 0302 | 4 | F | Preventing PDU Session release when handover between 3GPP and non-3GPP fails | 16.3.0 |
| 2020-03 | CT#87 | CP-200020 | 0303 | 1 | B | EBI list not to be transferred | 16.3.0 |
| 2020-03 | CT#87 | CP-200017 | 0304 |  | F | V-SMF insertion or removal | 16.3.0 |
| 2020-03 | CT#87 | CP-200033 | 0305 | 1 | B | Extended NAS-SM timer indication for UEs using CE mode B | 16.3.0 |
| 2020-03 | CT#87 | CP-200033 | 0306 |  | F | Feature definition for support of CIoT features | 16.3.0 |
| 2020-03 | CT#87 | CP-200033 | 0307 |  | B | Idle mode mobility between EPS and 5GS with data forwarding | 16.3.0 |
| 2020-03 | CT#87 | CP-200033 | 0308 |  | F | Mobile Originated Data | 16.3.0 |
| 2020-03 | CT#87 | CP-200033 | 0309 | 2 | B | Availability after DDN Failure | 16.3.0 |
| 2020-03 | CT#87 | CP-200017 | 0310 |  | F | Clarification to N4Information type | 16.3.0 |
| 2020-03 | CT#87 | CP-200052 | 0314 |  | F | 3GPP TS 29.502 Rel16 API version and External doc update | 16.3.0 |
| 2020-07 | CT#88 | CP-201018 | 0316 | 1 | F | Integrity protection maximum data rate in Create Request/Response | 16.4.0 |
| 2020-07 | CT#88 | CP-201046 | 0317 |  | F | Correction to the Resource URI structure | 16.4.0 |
| 2020-07 | CT#88 | CP-201046 | 0319 | 2 | B | Retrieval of Rate Control Status over N16/N16a | 16.4.0 |
| 2020-07 | CT#88 | CP-201055 | 0320 |  | F | Storage of YAML files in ETSI Forge | 16.4.0 |
| 2020-07 | CT#88 | CP-201031 | 0321 | 1 | F | URI of the Nsmf_PDUSession service | 16.4.0 |
| 2020-07 | CT#88 | CP-201031 | 0322 |  | F | V-SMF and I-SMF service instance Id | 16.4.0 |
| 2020-07 | CT#88 | CP-201055 | 0323 | 1 | F | Supported Headers Tables for Response types 2xx and 3xx | 16.4.0 |
| 2020-07 | CT#88 | CP-201055 | 0324 | 1 | F | Binary Data Types Table | 16.4.0 |
| 2020-07 | CT#88 | CP-201046 | 0325 | 1 | F | Maximum UP resources activation of 2 PDU sessions | 16.4.0 |
| 2020-07 | CT#88 | CP-201055 | 0326 | 1 | F | RAN Initiated QoS Flow Mobility failure scenario | 16.4.0 |
| 2020-07 | CT#88 | CP-201031 | 0327 | 2 | F | PDU Session Resource Notify Released Transfer | 16.4.0 |
| 2020-07 | CT#88 | CP-201031 | 0328 | 1 | F | Security Result | 16.4.0 |
| 2020-07 | CT#88 | CP-201034 | 0329 | 1 | F | sNssai during EPS to 5GS interworking | 16.4.0 |
| 2020-07 | CT#88 | CP-201048 | 0332 | 1 | F | Stage 2 procedures for wireline access | 16.4.0 |
| 2020-07 | CT#88 | CP-201044 | 0333 | 1 | B | Inform SMF that UE deregisters from one access | 16.4.0 |
| 2020-07 | CT#88 | CP-201046 | 0334 | 2 | B | Notification Correlation ID related to event Availability after DDN Failure | 16.4.0 |
| 2020-07 | CT#88 | CP-201018 | 0335 | 2 | F | Binary IE Encoding | 16.4.0 |
| 2020-07 | CT#88 | CP-201044 | 0336 | 1 | F | MAPDU via Non-3GPP Access | 16.4.0 |
| 2020-07 | CT#88 | CP-201031 | 0337 | 1 | F | SM Context Transfer | 16.4.0 |
| 2020-07 | CT#88 | CP-201055 | 0338 | 1 | F | Data type column in Resource URI variables Table | 16.4.0 |
| 2020-07 | CT#88 | CP-201068 | 0339 | 1 | B | Inter-system handover with direct data forwarding | 16.4.0 |
| 2020-07 | CT#88 | CP-201031 | 0340 |  | F | Registration with I-SMF insertion/change/removal and UP connection establishment | 16.4.0 |
| 2020-07 | CT#88 | CP-201055 | 0341 | 1 | F | Add custom operation Name | 16.4.0 |
| 2020-07 | CT#88 | CP-201047 | 0342 | 2 | F | Implicit PDU Session Release | 16.4.0 |
| 2020-07 | CT#88 | CP-201046 | 0343 | 3 | F | Aligning "MO Exception data" handling with stage 2 - NEF | 16.4.0 |
| 2020-07 | CT#88 | CP-201044 | 0344 | 1 | F | MA-PDU Session Release | 16.4.0 |
| 2020-07 | CT#88 | CP-201202 | 0345 | 2 | F | N4 information exchanged over N16a during PDU session release | 16.4.0 |
| 2020-07 | CT#88 | CP-201044 | 0346 | 1 | F | MA PDU Session re-activation | 16.4.0 |
| 2020-07 | CT#88 | CP-201037 | 0347 | 1 | B | Alternative QoS Profiles | 16.4.0 |
| 2020-07 | CT#88 | CP-201031 | 0348 | 1 | F | Missing N2 SM Info in SmContextCreateError | 16.4.0 |
| 2020-07 | CT#88 | CP-201037 | 0349 | 1 | B | QoS Monitoring Request to NG-RAN | 16.4.0 |
| 2020-07 | CT#88 | CP-201031 | 0350 | 1 | F | Application errors from I-SMF to SMF | 16.4.0 |
| 2020-07 | CT#88 | CP-201031 | 0351 | 1 | F | PduSessionCreatedData from SMF to I-SMF | 16.4.0 |
| 2020-07 | CT#88 | CP-201031 | 0352 | 1 | F | RequestIndication | 16.4.0 |
| 2020-07 | CT#88 | CP-201031 | 0353 |  | F | URIs in SmContextCreatedData | 16.4.0 |
| 2020-07 | CT#88 | CP-201055 | 0355 | 1 | F | Selected DNN | 16.4.0 |
| 2020-07 | CT#88 | CP-201055 | 0356 | 1 | F | Error Handling in Nsmf_PDUSession Service | 16.4.0 |
| 2020-07 | CT#88 | CP-201046 | 0357 | 1 | F | Clarification on CP Only Indication related issue in Update SM Context service | 16.4.0 |
| 2020-07 | CT#88 | CP-201044 | 0360 |  | F | EPSInterworkingInd for MA PDU without N26 | 16.4.0 |
| 2020-07 | CT#88 | CP-201018 | 0361 | 1 | A | TEID encoding clarification | 16.4.0 |
| 2020-07 | CT#88 | CP-201073 | 0365 |  | F | 29.502 Rel16 API version and External doc update | 16.4.0 |
| 2020-09 | CT#89 | CP-202104 | 0366 | 1 | F | Release PDU Session if NSSAA Re-Authentication and Re-Authorization Fails | 16.5.0 |
| 2020-09 | CT#89 | CP-202114 | 0367 | 2 | F | Correct the Condition of upCnxState in SmContextCreateData | 16.5.0 |
| 2020-09 | CT#89 | CP-202114 | 0368 | 2 | F | PDU session deactivation | 16.5.0 |
| 2020-09 | CT#89 | CP-202092 | 0369 | 3 | F | Callback URI correction | 16.5.0 |
| 2020-09 | CT#89 | CP-202092 | 0370 | 2 | F | Request Type | 16.5.0 |
| 2020-09 | CT#89 | CP-202107 | 0371 | 1 | F | QoS Monitoring | 16.5.0 |
| 2020-09 | CT#89 | CP-202016 | 0373 | 2 | F | Error handling for PDU sessions with an I-SMF or V-SMF | 16.5.0 |
| 2020-09 | CT#89 | CP-202107 | 0374 |  | F | Indicating that NG-RAN cannot fulfil the least preferred Alternative QoS Profile | 16.5.0 |
| 2020-09 | CT#89 | CP-202092 | 0375 | 1 | F | Handover Cancel 5GS to EPS | 16.5.0 |
| 2020-09 | CT#89 | CP-202114 | 0376 | 2 | F | QosRules during I-SMF and V-SMF Insertion | 16.5.0 |
| 2020-09 | CT#89 | CP-202092 | 0377 | 1 | F | Service Access Authorization between SMFs | 16.5.0 |
| 2020-09 | CT#89 | CP-202206 | 0381 | 2 | F | H-SMF/SMF sending IPv6 Interface Identifier to V-SMF/I-SMF | 16.5.0 |
| 2020-09 | CT#89 | CP-202096 | 0383 |  | F | 29.502 Rel-16 API version and External doc update | 16.5.0 |
| 2020-12 | CT#90 | CP-203072 | 0385 | 2 | F | HTTP 3xx redirection | 16.6.0 |
| 2020-12 | CT#90 | CP-203048 | 0386 |  | F | Storage of YAML files in 3GPP Forge | 16.6.0 |
| 2020-12 | CT#90 | CP-203046 | 0387 | 1 | F | QoS monitoring of a PDU session based on GTP-U path monitoring | 16.6.0 |
| 2020-12 | CT#90 | CP-203082 | 0390 | 2 | F | Binding Information for the old V/I-SMF and SMF | 16.6.0 |
| 2020-12 | CT#90 | CP-203083 | 0391 | 3 | F | Remote NF is not reachable | 16.6.0 |
| 2020-12 | CT#90 | CP-203041 | 0392 | 2 | F | Service operations invoked in DDN Failure event | 16.6.0 |
| 2020-12 | CT#90 | CP-203029 | 0394 | 1 | F | EBI and ARP mapping update | 16.6.0 |
| 2020-12 | CT#90 | CP-203046 | 0395 | 1 | F | QoS Monitoring Reporting Frequency | 16.6.0 |
| 2020-12 | CT#90 | CP-203051 | 0396 | 1 | F | Correction on H-PCF | 16.6.0 |
| 2020-12 | CT#90 | CP-203051 | 0397 | 1 | F | IPv6 Index for I-SMF | 16.6.0 |
| 2020-12 | CT#90 | CP-203029 | 0398 |  | F | VPLMN QoS Constraints | 16.6.0 |
| 2020-12 | CT#90 | CP-203036 | 0401 |  | F | 29.502 Rel16 API version and External doc update | 16.6.0 |
| 2021-03 | CT#91 | CP-210049 | 0404 | 2 | F | Handover Failure during EPS to 5GS Handover with AMF Re-allocation | 16.7.0 |
| 2021-03 | CT#91 | CP-210045 | 0405 | 1 | F | Stop of QoS Monitoring | 16.7.0 |
| 2021-03 | CT#91 | CP-210037 | 0408 | 1 | F | Error Responses for Indirect Communication | 16.7.0 |
| 2021-03 | CT#91 | CP-210040 | 0409 |  | F | Charging ID of HR PDU session upon V-SMF change | 16.7.0 |
| 2021-03 | CT#91 | CP-210040 | 0410 |  | F | Handover between 3GPP and non-3GPP accesses with I-SMF insertion/removal or V-SMF change | 16.7.0 |
| 2021-03 | CT#91 | CP-210167 | 0411 | 2 | F | Redundancy Sequence Number for Dual Connectivity based end to end Redundant User Plane Paths | 16.7.0 |
| 2021-03 | CT#91 | CP-210166 | 0413 | 2 | F | QoS parameters handling during handover between 3GPP and non-3GPP accesses | 16.7.0 |
| 2021-03 | CT#91 | CP-210040 | 0415 | 1 | F | DNNs in I-SMF | 16.7.0 |
| 2021-03 | CT#91 | CP-210040 | 0416 | 1 | F | I-SMF&V-SMF Release Only | 16.7.0 |
| 2021-03 | CT#91 | CP-210060 | 0417 |  | F | Incomplete CR Implementation | 16.7.0 |
| 2021-03 | CT#91 | CP-210040 | 0422 |  | F | requestIndication for EBI revocation | 16.7.0 |
| 2021-03 | CT#91 | CP-210060 | 0423 | 2 | F | Alternative Anchor SMF Notification to AMF | 16.7.0 |
| 2021-03 | CT#91 | CP-210054 | 0425 |  | F | 29.502 Rel-16 API version and External doc update | 16.7.0 |
| 2021-03 | CT#91 | CP-210165 | 0407 | 1 | F | Data Types Descriptions | 17.0.0 |
| 2021-03 | CT#91 | CP-210021 | 0414 | 2 | B | Home Network Public Key identifier as additional input for SMF service | 17.0.0 |
| 2021-03 | CT#91 | CP-210034 | 0418 | 2 | B | UPF ID for N4 Information | 17.0.0 |
| 2021-03 | CT#91 | CP-210021 | 0419 |  | F | Correction to apiVersion placeholder | 17.0.0 |
| 2021-03 | CT#91 | CP-210034 | 0421 | 1 | F | EpsInterworkingIndication Update | 17.0.0 |
| 2021-03 | CT#91 | CP-210029 | 0424 |  | F | 29.502 Rel-17 API version and External doc update | 17.0.0 |
| 2021-06 | CT#92 | CP-211025 | 0427 |  | B | Support of Notify Start Pause of Charging via User Plane | 17.1.0 |
| 2021-06 | CT#92 | CP-211022 | 0428 | 1 | B | Adding samePcfSelectionIndication in Nsmf_PDUSession service to support SPSFAS | 17.1.0 |
| 2021-06 | CT#92 | CP-211059 | 0430 | 3 | A | Notify PDU session control is taken over by another SMF | 17.1.0 |
| 2021-06 | CT#92 | CP-211031 | 0431 | 1 | B | (I-)SMF discovery based on DNAI | 17.1.0 |
| 2021-06 | CT#92 | CP-211023 | 0432 | 1 | F | hNRF from NSSF in home PLMN | 17.1.0 |
| 2021-06 | CT#92 | CP-211120 | 0433 | 1 | F | OpenAPI Reference | 17.1.0 |
| 2021-06 | CT#92 | CP-211046 | 0435 | 1 | F | Simultaneous change of Branching Point or UL CL and additional PSA | 17.1.0 |
| 2021-06 | CT#92 | CP-211082 | 0437 | 1 | A | DNN Selection Mode | 17.1.0 |
| 2021-06 | CT#92 | CP-211062 | 0439 | 1 | A | AMF ID in HsmfUpdateData | 17.1.0 |
| 2021-06 | CT#92 | CP-211068 | 0441 | 3 | A | NG-RAN tunnel information during mobility registration with I-SMF/V-SMF change | 17.1.0 |
| 2021-06 | CT#92 | CP-211054 | 0442 | 1 | F | Correction on SmContextUpdateData | 17.1.0 |
| 2021-06 | CT#92 | CP-211028 | 0443 | 2 | F | Secondary authorization/authentication by an DN-AAA | 17.1.0 |
| 2021-06 | CT#92 | CP-211049 | 0444 | 2 | B | I-SMF/V-SMF Restoration procedure | 17.1.0 |
| 2021-06 | CT#92 | CP-211028 | 0445 | 1 | F | Data forwarding during 5GS to EPS handover | 17.1.0 |
| 2021-06 | CT#92 | CP-211059 | 0446 | 1 | A | Redirect Responses | 17.1.0 |
| 2021-06 | CT#92 | CP-211059 | 0448 |  | A | 3xx description correction for SCP | 17.1.0 |
| 2021-06 | CT#92 | CP-211082 | 0453 | 1 | A | User location included in session management messages | 17.1.0 |
| 2021-06 | CT#92 | CP-211082 | 0455 |  | A | Missing Procedure | 17.1.0 |
| 2021-06 | CT#92 | CP-211050 | 0456 |  | F | 29.502 Rel-17 API version and External doc update | 17.1.0 |
| 2021-09 | CT#93 | CP-212026 | 0472 |  | F | Correction of Content-Type | 17.2.0 |
| 2021-09 | CT#93 | CP-212026 | 0471 | 1 | B | SMF Response in case of AN-Requested PDU Session Resource Release | 17.2.0 |
| 2021-09 | CT#93 | CP-212026 | 0480 | 1 | F | Corrections to the API URI | 17.2.0 |
| 2021-09 | CT#93 | CP-212031 | 0462 |  | B | Downlink Tunnel Info of NG-RAN in I-SMF selection per DNAI | 17.2.0 |
| 2021-09 | CT#93 | CP-212031 | 0461 | 2 | B | AF Coordination Information | 17.2.0 |
| 2021-09 | CT#93 | CP-212032 | 0478 | 1 | F | Selecting the same PCF for AMF and SMF in pure 5GS | 17.2.0 |
| 2021-09 | CT#93 | CP-212042 | 0468 | 1 | B | PDUSession API extensions for Redundant PDU Sessions | 17.2.0 |
| 2021-09 | CT#93 | CP-212047 | 0463 |  | F | Binding Indication | 17.2.0 |
| 2021-09 | CT#93 | CP-212047 | 0469 |  | F | requestType IE in Create (SM Context) service request | 17.2.0 |
| 2021-09 | CT#93 | CP-212047 | 0470 | 1 | F | Secondary RAT Usage Report | 17.2.0 |
| 2021-09 | CT#93 | CP-212047 | 0473 | 1 | F | Correction on remoteError and Snssai | 17.2.0 |
| 2021-09 | CT#93 | CP-212047 | 0460 | 1 | F | DNAI removal | 17.2.0 |
| 2021-09 | CT#93 | CP-212059 | 0483 |  | F | 29.502 Rel-17 API version and External doc update | 17.2.0 |
| 2021-09 | CT#93 | CP-212061 | 0459 |  | A | Change of ULCL/BP and/or local PSA | 17.2.0 |
| 2021-09 | CT#93 | CP-212064 | 0466 | 1 | A | Handover from 5GS to ePDG | 17.2.0 |
| 2021-09 | CT#93 | CP-212079 | 0476 | 1 | A | making vcnTunnelInfo optional on N16 | 17.2.0 |
| 2021-09 | CT#93 | CP-212079 | 0475 | 1 | A | S-NSSAI Not Supported in Handover | 17.2.0 |