| 3GPP TR38.787 V0.0.1 (2024-09) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; User Equipment (UE) radio transmission and reception for NR Sidelink supporting intra-band CA in ITS band (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Definitions	7

3.2	Symbols	7

3.3	Abbreviations	7

4	Background	8

5	Operating bands and channel arrangement for SL evolution	9

5.1	Operating bands	9

5.1.1	Operating bands for single carrier operation in unlicensed band	9

5.1.2	Operating band combinations for inter-band con-current operation	9

5.2	Channel bandwidth	9

5.2.1	Channel bandwidth for single carrier operation in unlicensed band	9

5.2.2	Channel bandwidth for inter-band con-current operation	9

5.3	Channel arrangement	9

5.3.1	Channel raster	9

5.3.2	Synchronization raster	9

6	Transmitter characteristics  SL evolution	10

6.1	Tx requirements  SL single carrier operation in unlicensed band	10

6.1.1	Maximum output power  	10

6.1.2	UE maximum output power reduction  	10

6.1.3	UE additional maximum output power reduction  	10

6.1.4	Configured transmitted power  	10

6.1.5	Minimum output power  	10

6.1.6	Transmit OFF power  	10

6.1.7	ON/OFF time mask  	10

6.1.8	Power control  	10

6.1.9	Transmit signal quality  	10

6.1.10	Spectrum emission mask  	10

6.1.11	ACLR requirements  	10

6.1.12	Spurious emissions  	10

6.1.13	Spurious emission band UE co-existence  	10

6.1.14	Transmit intermodulation  	10

6.2	Tx requirements for inter-band con-current operation	11

6.2.1	Maximum output power for inter-band con-current operation	11

6.2.2	UE maximum output power reduction for inter-band con-current operation	11

6.2.3	UE additional maximum output power reduction for inter-band con-current operation	11

6.2.4	Configured transmitted power for inter-band con-current operation	11

6.2.5	Minimum output power for inter-band con-current operation	11

6.2.6	Transmit OFF power for inter-band con-current operation	11

6.2.7	ON/OFF time mask for inter-band con-current operation	11

6.2.8	Power control for inter-band con-current operation	11

6.2.9	Transmit signal quality for inter-band con-current operation	11

6.2.10	Spectrum emission mask for inter-band con-current operation	11

6.2.11	ACLR requirements for inter-band con-current operation	11

6.2.12	Spurious emissions for inter-band con-current operation	11

6.2.13	Spurious emission band UE co-existence for inter-band con-current operation	11

6.2.14	Transmit intermodulation for inter-band con-current operation	11

7	Receiver characteristics  SL evolution	11

7.1	Rx requirements  SL single carrier operation in unlicensed bands	11

7.1.1	Reference sensitivity power level  	11

7.1.2	Maximum input level  	11

7.1.3	Adjacent Channel Selectivity  	11

7.1.4	Blocking characteristics  	11

7.1.5	Spurious response  	11

7.1.6	Intermodulation characteristics  	11

7.2	Rx requirements for inter-band con-current operation	12

7.2.1	Reference sensitivity power level for inter-band con-current operation	12

7.2.2	Maximum input level for inter-band con-current operation	12

7.2.3	Adjacent Channel Selectivity for inter-band con-current operation	12

7.2.4	Blocking characteristics for inter-band con-current operation	12

7.2.5	Spurious response for inter-band con-current operation	12

7.2.6	Intermodulation characteristics for inter-band con-current operation	12

8	Co-channel coexistence between LTE Sidelink and NR Sidelink	12

Annex A: Change history	12

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document is a technical report on sidelink evolution services in Rel-19. The purpose is to specify radio solutions that are necessary to support sidelink power class 2/3 in non-contiguous and PC2 contiguous intra band SL CA in ITS spectrum.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 30.007: "Guideline on WI/SI for new Operating Bands".

[3]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[4]	RP-240023, “LS on extending Sidelink Carrier Aggregation support in Rel-19,” 5GAA WG4

[5]	RP-240840, “New WID on NR Sidelink: Intra-band Carrier Aggregation in ITS band,”

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

AGC	Automatic Gain Control

A-MPR	Additional Maximum Power Reduction

BLER	BLock Error Rate

BS	Base Station

CBW	Channel Bandwidth

CDF	Cumulative Distribution Function

CP-OFDM	Cyclic Prefix-OFDM

DMRS	Demodulation Reference Signal

EIRP	Equivalent Isotropically Radiated Power

EVM	Error Vector Magnitude

FDD	Frequency Division Duplex

FDM	Frequency Division Multiplexing

FR1	Frequency Range 1

ITS	Intelligent Transportation System

LTE	Long Term Evolution

MPR	Maximum Power Reduction

NF	Noise Figure

NR	New Radio

OLPC	Open Loop Power Control

PC	Power Control

PRB	Physical Resource Block

ProSe	Proximity-based Services

PSCCH	Physical Sidelink Control CHannel

PSSCH	Physical Sidelink Shared CHannel

REFSENS	Reference Sensitivity

RF	Radio Frequency

SCS	Sub-Carrier Spacing

SINR	Signal to Interference plus Noise Ratio

SL	Sidelink

Sidelink at unlicensed band

SNR	Signal-to-Noise Ratio

TDD	Time Division Duplex

TDM	Time Division Multiplexing

UE	User Equipment

UL	Uplink

V2V	Vehicle to Vehicle

V2X	Vehicle to Everything

# 4	Background

In Rel-18, RAN4 specified the following NR sidelink requirements:

(1)	NR sidelink CA (n47) with intra-band contiguous CA and power class 3

(2)	NR sidelink single carrier in unlicensed spectrum (n46/n96/n102) with power class 5 for 1 Tx and 5 NS values

(3)	NR sidelink inter-band con-current operation with power class 3 at NR Uu licensed band + power class 5 at  un-licensed band

For NR sidelink CA, automotive industry shows strong interest towards power class 2/3 in contiguous and non-contiguous intra band SL CA to account for the fragmented utilization of different channels of 10MHz and 20MHz in n47 and to achieve sufficient communication range [4].

The detailed objectives for NR sidelink supporting intra-band CA in ITS band are captured in [5].

## 4.1	SL CA UE RF architectures

In SL evolution WI in Rel-19, RAN4 consider the following UE RF architectures for intra-band contiguous/non-contiguous CA operation in ITS spectrum.

For the NR intra-band contiguous/non-contiguous SL CA UE, the candidate RF architectures in Table 4.1-1 be considered to evaluate MPR/A-MPR requirements.

Table 4.1-1 SL intra-band CA UE RF architectures for MPR/A-MPR evaluation

| Arch | description | Remarks |
| --- | --- | --- |
| #1-1 | 1x26dBm PA + 1LO | Single–Tx |
| #1-2 | 2x23dBm PA + 1LO | dual-Tx or txDiversity |
| #2-2 | 2x23dBm PA + 2LO | dualPA-Architecture |

For the NR intra-band contiguous SL CA UE, the UE architecture #1-1 and #1-2 with 1LO is prioritized in the agreed WF (R4-2410587). For the NR intra-band non-contiguous SL CA UE, both 1LO and 2LO architectures are considered for MPR/A-MPR evaluation.

# 5	Operating bands and channel arrangement for Sidelink CA

## 5.1	Operating bands

### 5.1.1	Operating band for intra-band contiguous CA

NR SL PC2 CA operation is designed to operate in the operating bands in FR1 defined in Table 5.1.1-1.

Table 5.1.1-1: Intra-band contiguous CA operating bands for SL CA in FR1

| NR SL CA Band | NR Band | Interface |
| --- | --- | --- |
| SL_n47 | n47 | PC5 |

### 5.1.2	Operating band for intra-band non-contiguous CA

NR SL intra-band non-contiguous CA operation is designed to operate in the operating bands in FR1 defined in Table 5.1.1-1.

Table 5.1.1-1: Intra-band non-contiguous CA operating bands for SL CA in FR1

| NR SL CA Band | NR Band | Interface |
| --- | --- | --- |
| SL_n47 | n47 | PC5 |

## 5.2	Channel bandwidth

### 5.2.1	Channel bandwidth for intra-band contiguous CA

For NR SL PC2 CA operation, the SL CA channel bandwidths for each operating band is specified in Table 5.2.1-1.

Table 5.2.1-1: Intra-band contiguous CA operating bands for SL PC2 CA in FR1

| Sidelink CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sidelink CA configuration | Sidelink CA configuration for TX | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
|  |  | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) |  |  |
| SL_n47B | SL_n47B | 10 | 10, 20,30 |  |  | 70 | 0 |
|  |  | [20] | [20],[30] |  |  |  |  |
|  |  | 30 | 30,40 |  |  |  |  |

### 5.2.2	Channel bandwidth for intra-band non-contiguous CA

For NR SL CA intra-band non-contiguous operation, the SL CA channel bandwidths for each operating band is specified in Table 5.2.2-1.

Table 5.2.2-1 channel bandwidth combinations for SL intra-band non-contiguous CA

| Sidelink CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sidelink CA configuration | Sidelink CA configuration for TX | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| SL_n47(2A) | SL_n47(2A) | 10 | 10, 20 |  |  | 30 | 0 |

# 6	Transmitter characteristics for NR Sidelink supporting intra-band CA in ITS band

## 6.1	Tx requirements for SL intra-band contiguous CA

### 6.1.1	Maximum output power

### 6.1.2	UE maximum output power reduction

### 6.1.3	UE additional maximum output power reduction

### 6.1.4	Configured transmitted power

## 6.2	Tx requirements for SL intra-band non-contiguous CA

### 6.2.1	Maximum output power

For the intra-band non-contiguous SL CA operation with two uplink carriers, both PC3 and PC2 NR SL CA UE power classes will be supported for the maximum output power for any transmission bandwidth within the channel bandwidth. The period of measurement shall be at least one sub frame (1ms).

Table 6.2.1-1: NR intra-band non-contiguous SL CA UE Power Class

| NR SL CA band Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SL_n47(2A) |  |  | 26 | +2/-33 | 23 | +2/-33 |  |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: For intra-band SL CA UE, the maximum power requirement apply to the total transmitted power over all component carriers (per UE). NOTE 3: 3 refers to the transmission bandwidths (Figure 5.6-1 in TS38.101-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 4: Power class 3 is the default power class unless otherwise stated. |  |  |  |  |  |  |  |  |

### 6.2.2	UE maximum output power reduction

For the basic parameters, RAN4 reuse the simulation assumptions in TR38.785 (Rel-17 enhanced NR sidelink) for NR intra-band non-contiguous CA UE. Other constraints of PSCCH/PSSCH/PSFCH/S-SSB for the intra-band non-contiguous SL CA operation can be assumed based on current RAN1’s agreement. Aslo, the following parameters are considered to evalute MPR/A-MPR requirements.

Table 6.2.2-1: Basic RF parameters for PC2/PC3 NR intra-band non-contiguous SL CA UE

| Center frequency | 5.9GHz |
| --- | --- |
| Bandwidth | per CC: 10/20MHz aggregated CBW: 10+10MHz/ 10+20MHz |
| Maximum output power for aggregated CBW | 23dBm/26dBm |
| Numerology | 15 kHz/30kHz/60kHz |
| Modulation per CC | QPSK/16QAM/64QAM/256QAM |
| Waveform | CP-OFDM |
| ACLR | 30dBc(PC3) / 31dBc(PC2) |
| Carrier leakage | 25dBc |
| IQ image | 34dBc |
| CIM3 | 60dBc |
| PA calibration | PA calibrated to deliver 30dB ACLR for a fully allocated RBs in 20MHz QPSK DFT- S-OFDM waveform at 1 dB MPR. This is based to share PA between LTE V2X and NR V2X at 5.9GHz as worst case. |
| Frequency Gap | Consider the worst case for BW gap size between sub-block #1 and #2 for the MPR requirement. To derive MPR requirements, consider all candidate gap size. |

### 6.2.3	UE additional maximum output power reduction

For the A-MPR analysis to comply the US regulation, RAN4 agreed to study and specipy the related PC2 A-MPR requirements with NS_52 after final FCC regulation for the additional emission limits is released in US.

For the A-MPR analysis to comply the EU reguration, RAN4 need to study and specify the PC3/PC2 A-MPR requirements with NS_33 based on the merged the interested companies results.

The required EU regulations for SL CA UE with 10MHz CBW are listed as below

- Additional Spectrum mask: When "NS_33" is indicated in the cell or pre-configured radio parameters, the power of any V2X UE emission shall not exceed the levels specified in Table 6.5E.2.3.1-1.

Table 6.2.3-1: Additional spectrum mask requirements for 10MHz channel bandwidth

| Spectrum emission limit (dBm EIRP)/ Channel bandwidth |  |  |
| --- | --- | --- |
| ΔfOOB (MHz) | 10 MHz | Measurement bandwidth |
| 0-0.5 | [-13-12∆fOOBMHz] | 100 kHz |
| 0.5-5 | [-19-169∆fOOBMHz-0.5] | 100 kHz |
| 5-10 | [-27-2∆fOOBMHz-5.0] | 100 kHz |

- Additional emission requirements:

Table 6.2.3-2: Additional requirements for "NS_33"

| Protected band |  | Frequency range (MHz) |  |  | Maximum Level (EIRP2) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency range | 5925 |  | - | 5950 | -30 | 1 | 1 |
| Frequency range | 5815 |  | - | 5855 | -30 | 1 | 3 |
| NOTE 1: In the frequency range x-5950MHz, SE requirement of -30dBm/MHz should be applied; where x = max (5925, fc + 15), where fc is the channel centre frequency. NOTE 2: The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I in [11]. NOTE 3: Resolution BW is 10% of the measurement BW and the result should be integrated to achieve the measurement bandwidth. The sweep time shall be set larger than (symbol length)*(number of points in sweep) to improve the measurement accuracy. |  |  |  |  |  |  |  |

- Additional requirements to protect CEN DSRC:

When "NS_33" is configured from pre-configured radio parameters or the cell, and the indication from upper layers has indicated that the UE is within the protection zone of CEN DSRC devices or HDR DSRC devices, the power of any NR V2X UE emission shall fulfil either one of the two sets of conditions.

Table 6.5E.3.4.2-2: Requirements for spurious emissions to protect CEN DSRC for V2X UE

|  | Maximum Transmission Power (dBm EIRP1) | Emission Limit in Frequency Range 5795-5815 (dBm/MHz EIRP1) |
| --- | --- | --- |
| Condition 1 | 10 | -65 |
| Condition 2 | 10 | -45 |
| NOTE 1: The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I in [11]. |  |  |

### 6.2.4	Configured transmitted power

<Editor Note> The detail requirements are FFS.

### 6.2.5 Other Tx requirements

<Editor Note> The detail requirements are FFS.

# 7	Receiver characteristics for NR Sidelink supporting intra-band CA in ITS band

## 7.1	Rx requirements for SL intra-band contiguous CA

### 7.1.1	Reference sensitivity

It is agreed to reuse current PC3 SL intra-band contiguous CA requirement defined in TS 38.101-1 clause 7.3E.2A for PC2 SL intra-band contiguous CA.

### 7.1.2	Maximum input level

It is agreed to reuse current PC3 SL intra-band contiguous CA requirement defined in TS 38.101-1 clause 7.4E.1A for PC2 SL intra-band contiguous CA.

### 7.1.3	Adjacent Channel Selectivity

It is agreed to reuse current PC3 SL intra-band contiguous CA requirement defined in TS 38.101-1 clause 7.5E.1A for PC2 SL intra-band contiguous CA.

### 7.1.4	Blocking characteristics

It is agreed to reuse current PC3 SL intra-band contiguous CA requirement defined in TS 38.101-1 clause 7.6E.2.1A for in-band blocking and clause 7.6E.3.1A for out-of-band blocking for PC2 SL intra-band contiguous CA.

### 7.1.5	Spurious response

It is agreed to reuse current PC3 SL intra-band contiguous CA requirement defined in TS 38.101-1 clause 7.7E.1A for PC2 SL intra-band contiguous CA.

### 7.1.6	Intermodulation characteristics

It is agreed to reuse current PC3 SL intra-band contiguous CA requirement defined in TS 38.101-1 clause 7.8E.2.2A for PC2 SL intra-band contiguous CA.

## 7.2	Rx requirements for SL intra-band non-contiguous CA

### 7.2.1	Reference sensitivity

For intra-band non-contiguous SL CA UE in n47, the reference sensitivity requirement specified in Table 7.3E.2-1 [3] shall apply for each sub-block with all carriers active. The requirement is applied for each sub-block reception when 2 sub-block transmissions are activated at the same time.

### 7.2.2	Maximum input level

For intra-band non-contiguous SL CA, the maximum input level requirements in Section 7.4E.1 [3] will be applied to each sub-block. The throughput (>= 95% T-put) of each DL CC shall meet or exceed the minimum requirements for the specified reference measurement channel in A.7.3 and A.7.4.

### 7.2.3	Adjacent Channel Selectivity

For intra-band non-contiguous SL CA, the UE shall fulfil the minimum requirement specified in Table 7.5E.1-1 to Table 7.5E.1-3 [3] per sub-block where the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.7.2 while all DL carriers are active.

For the ACS requirement, the UE shall meet the requirements for each sub-block as specified in clauses 7.5E in TS 38.101-1 for one component carrier per sub-block. The UE shall fulfil the minimum requirements all values of a single adjacent channel interferer in-gap and out-of-gap up to a –25 dBm interferer power while all sidelink carriers are active. For the lower range of test parameters (Case 1), the interferer power Pinterferer shall be set to the maximum of the levels given by the carriers of the respective sub-blocks as specified in Table 7.5E-2 in TS 38.101-1 for one component carrier per sub-block. The wanted signal power levels for the carriers of each sub-block shall then be adjusted relative to Pinterferer in accordance with the ACS requirement for each sub-block (Table 7.5E-1 in TS 38.101-1). For the upper range of test parameters (Case 2) for which the interferer power Pinterferer is -25 dBm (Table 7.5E-3 in TS 38.101-1) the wanted signal power levels for the carriers of each sub-block shall be adjusted relative to Pinterferer like for Case 1.

### 7.2.4	Blocking characteristics

For in-band blocking requirement, the UE shall meet the requirements for each sub -block as specified in clause 7.6E.2 in TS 38.101-1for one component carrier per sub-block. The requirements apply for in-gap and out-of-gap interferers while all sidelink carriers are active.

To meet the in-band blocking requirements of the intra-band non-contiguous SL CA, the UE throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.7.2 with the test parameters defined in clause 7.6E.2.1 [3] per sub-block while all downlink carriers are active.

For out-of-band blocking requirement, the UE shall meet the requirements for each sub-block as specified in clauses 7.6E.3 in TS 38.101-1 for one component carrier per sub-block, respectively. The requirements apply with all sidelink carriers active.

To meet the Out-of-band blocking requirements of the intra-band non-contiguous SL CA, the UE throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.7.2 with test parameters defined in clause 7.6E.3.1 [3] per sub-block while all downlink carriers are active.

### 7.2.5	Spurious response

For spurious response requirement, the UE shall meet the requirements for each sub-block as specified in clauses 7.7E in TS 38.101-1for one component carrier per sub-block, respectively. The requirements apply with all sidelink carriers active.

For intra-band non-contiguous SL CA, the UE throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.7.2 with test parameters defined in clause 7.7E.1 [3] per sub-block while all downlink carriers are active.

### 7.2.6	Intermodulation characteristics

For intermodulation characteristics, the UE shall meet the requirements for each sub-block as specified in clauses 7.8E.2 in TS 38.101-1 for one component carrier per sub-block, respectively. The requirements apply with all sidelink carriers active.

For intra-band non-contiguous SL CA, the UE throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.7.2 with test parameters defined in clause 7.8E.2.1 [3] per sub-block while all downlink carriers are active.

Annex A: Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-09 | RAN4#112 |  |  |  |  | Initial draft | 0.0.1 |