3GPP TR 38.813 V0.1.0 (2017-11)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

NR;

New frequency range for NR (3.3-4.2 GHz)

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

Radio, LTE

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2017, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	6

2	References	6

3	Definitions, symbols and abbreviations	6

3.1	Definitions	6

3.2	Symbols	6

3.3	Abbreviations	6

4	Background	7

5	NR Frequency band definition	7

6	Channel numbering and channel bandwidth	7

7	NR band 3.3GHz - 3.8GHz specific issues	7

7.1	UE specific	7

7.1.1	Transmitter characteristics	7

7.1.1.1	UE maximum output power	7

7.1.1.2	UE maximum output power for modulation / channel bandwidth	8

7.1.1.3	UE maximum output power with additional requirements	8

7.1.1.4	Adjacent Channel Leakage Ratio (ACLR)	8

7.1.1.5	Spectrum emission mask	8

7.1.1.6	Spurious emissions	8

7.1.2	Receiver characteristics	8

7.1.2.1	Reference sensitivity	8

7.1.2.2	Adjacent Channel Selectivity (ACS)	9

7.1.2.3	Blocking	9

7.2	BS specific	10

7.2.1	Operating band unwanted emissions	10

7.2.2	Additional spurious emissions requirements	10

7.2.3	Co-location with other base stations	10

7.2.4	General blocking requirement	10

7.2.5	Blocking requirement for co-location with other base stations	10

8	NR band 3.3GHz - 4.2GHz specific issues	10

8.1	UE specific	10

8.1.1	Transmitter characteristics	10

8.1.1.1	UE maximum output power	10

8.1.1.2	UE maximum output power for modulation / channel bandwidth	11

8.1.1.3	UE maximum output power with additional requirements	11

8.1.1.4	Adjacent Channel Leakage Ratio (ACLR)	11

8.1.1.5	Spectrum emission mask	11

8.1.1.6	Spurious emissions	11

8.1.2	Receiver characteristics	11

8.1.2.1	Reference sensitivity	11

8.1.2.2	Adjacent Channel Selectivity (ACS)	11

8.1.2.3	Blocking	11

8.2	BS specific	12

8.2.1	Operating band unwanted emissions	12

8.2.2	Additional spurious emissions requirements	12

8.2.3	Co-location with other base stations	12

8.2.4	General blocking requirement	12

8.2.5	Blocking requirement for co-location with other base stations	12

9	Required changes to NR, E-UTRA, UTRA and MSR specifications	12

Annex <A> (normative): <Normative annex title>	13

Annex B: Change history … …………………………………………………………………………  14

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

1	Scope

<Text to be added>

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

3	Definitions, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

Abbreviation format (EW)

<ACRONYM>	<Explanation>

4	Background

<Text to be added>

5	NR Frequency band definition

<Text to be added>

6	Channel numbering and channel bandwidth

<Text to be added>

7	NR band 3.3GHz - 3.8GHz specific issues

7.1	UE specific

<Text to be added>

7.1.1	Transmitter characteristics

7.1.1.1	UE maximum output power

The MOP requirements for Band 42 and 43 single-carrier were specified as 23 dBm +2/-3 dB for power class 3 based on simulation results shown in Table 7.1.1.1-1 in August 2011.

Table 7.1.1.1-1.  Simulation results for combined Band 42 and Band 43 filter [R4-114656]

|  | Bandwidth | Max IL (corner) | 2f0 rejection | 2.7 GHz rejection |
| --- | --- | --- | --- | --- |
| Design 7 | 400 MHz | 1.9 | 20 dB | 30 dB |
| Design 8 | 400 MHz | 2.3 | 15 dB | 10 dB |
| Design 9 | 400 MHz | 2.0 | 20 dB | 15 dB |

When specifying MOP requirements of Band n77 and n78, the delta to be checked is IL impact of BPF due to the extension of pass-bandwidth. It is shown in Table 7.1.1.1-2.

Table 7.1.1.1-2.  Simulation results for Band n77 and n78

| Parameter | Frequency range | Vendor 1 |  | Vendor 2 |  | Vendor 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Band n78 BPF | Band n77 BPF | Band n78 BPF | Band n77 BPF | Band n78 BPF | Band n77 BPF |
| Insertion loss (ETC) | 3300-3400 MHz | 1.05 dB | 1.05 dB | 2.0 dB | 2.0 dB | 1.9 dB | 1.9 dB |
|  | 3400-3800 MHz | 1.0 dB | 1.0 dB | 1.5 dB | 1.5 dB |  |  |
|  | 3800-4200 MHz | - | 1.25 dB | - | 2.0 dB | - |  |
| Attenuation (Typ) | 698-2690 MHz | 41.0 dB | 41.0 dB | 45 dB | 35 dB (ETC) | 46.3 dB | 47.8 dB |
|  | 5150-5925 MHz | 40.2 dB | 40.2 dB | 35 dB | 35 dB (ETC) | 40.8 dB | 35.6 dB |

With comparison between Table 7.1.1.1-1 and Table 7.1.1.1-2, it was concluded that there is no degradation of IL at ETC even with the extension and agreed to specify the same MOP as that of Band 42 and 43 (i.e., 23 dBm +2/-3 dB) for Band n77 and n78 of power class 3.

Agreement: MOP is to be specified as 23 dBm +2/-3 dB for Band n77 and n78 of power class 3

7.1.1.2	UE maximum output power for modulation / channel bandwidth

<Text to be added>

7.1.1.3	UE maximum output power with additional requirements

<Text to be added>

7.1.1.4	Adjacent Channel Leakage Ratio (ACLR)

<Text to be added>

7.1.1.5	Spectrum emission mask

<Text to be added>

7.1.1.6	Spurious emissions

Since Band n77 and/or n78 will be used at least in Japan, bands operated in Japan need to be protected. In addition, protected bands required in Europe, Korea and China regions (i.e., operating bands specified for Band n78 of 3.3-3.8 GHz) should also be added for potential use of these bands. As a consequence, required protected bands for Band n77 and n78 will be the same. Note that it was already agreed not to specify the co-existence requirement between Band n77 and Band n79, which means that the co-existence will be guaranteed by the NR general emission requirements.

Table 7.1.1.6-1: Spurious emission band UE co-existence for Band n77 and n78

| NR band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n77, n78 | E-UTRA Band 1, 3, 11, 18, 19, 21, 28, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | PHS |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band [n257] | 26500 | - | 29500 | TBD | TBD |  |
|  | (More bands to be added for other regions) |  |  |  |  |  |  |

Agreement: Protected bands required for Band n77 are to be the same as those for Band n78 (i.e., not only Japanese bands but also ones required for other regions such as Europe, Korea and China)

7.1.2	Receiver characteristics

7.1.2.1	Reference sensitivity

Based on Table 7.1.1.1-2 and more simulation results in Table 7.1.2.1-1, 7.1.2.1-2 and 7.1.2.1-3 whose attenuation is at least typ. 35 dB below 2690 MHz and above 5150 MHz,  the following was agreed.

Table 7.1.2.1-1. Filter simulation results reported by Vendor A in [R4-1711334]

| Filter | Nominal IL [dB] |  | Min Attenuation [dB] @ |  |
| --- | --- | --- | --- | --- |
|  | w/o 30 dB Rejection in Radio Altimeter Band | with 30 dB Rejection in Radio Altimeter Band | 693-2690 MHz | 5150-5925 MHz |
| n77 (3.3-4.2 GHz) | 1.8 | 2.7 | >35 | >35 |
| n78 (3.3-3.8 GHz) | 1.5 | 1.8 | >35 | >35 |

Table 7.1.2.1-2. Filter simulation results reported by Vendor B in [R4-1711334]

| Filter | IL [dB] |  | Min Attenuation [dB] @ |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Nominal | Worst Case | 693-2690 MHz | 5150-5925 MHz | Radio Altimeter |
| n77 (3.3-4.2 GHz) | 1.87 | TBC | 44 | 46 |  |
| n78 (3.3-3.8 GHz) | 1.56 | TBC | 40.5 | 51 |  |

Table 7.1.2.1-3. Filter simulation results reported in [R4-1711334]

| Filter | IL [dB] |  |  |  |  |  | Min Attenuation [dB] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Nominal |  |  | Worst case |  |  |  |  |  |  |  |  |
| Frequencies [GHz] | 3.3 | 3.8 | 4.2 | 3.3 | 3.8 | 4.2 | 2.3-2.69 | 0.617-2.2 | 4.4-5.0 | 4.8-5.0 | 5.15-5.925 | 5.250-5.925 |
| n77 (3.3-4.2 GHz) | 2.6 | 1.9 | 2.4 | 2.9 | 2.1 | 2.6 | 35 | 45 | 5 | 18 | 40 | 45 |
| n78 (3.3-3.8 GHz) | 2.1 | 1.8 | - | 2.3 | 2.1 | - | 35 | 40 | 22 | 25 | 40 | 45 |

Agreement: REFSENS for Band n77 at 3.3-3.8 GHz and n78 should be 1 dB larger than that of bands which have NR smallest sensitivity (less RF challenges) such as Band n1. REFSENS for Band n77 at 3.8-4.2 GHz is increased by 0.5 dB compared to that of Band n78.

7.1.2.2	Adjacent Channel Selectivity (ACS)

<Text to be added>

7.1.2.3	Blocking

#### 7.1.2.3.1	Out-of-band blocking

Based on filter performance shown in Table 7.1.1.1-2, out-of-band blocking requirement needs to be optimized as with LTE band 42 and 43 as shown in Table 7.1.2.3-1. Wanted signal level is the same as that of LTE refarming bands.

Agreement: Out-of-band blocking for Band n77 and n78 should be specified as below.

Table 7.1.2.3-1: Out-of-band blocking parameters for Band n77 and n78

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |  |  |  |
|  |  | 6 | 7 | 9 | 12 | 13 | 14 | 15 | 16 |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified TBD with PCMAX_L as defined in subclause 6.2.5. NOTE 2: Reference measurement channel is TBD NOTE 3: The REFSENS power level is TBD |  |  |  |  |  |  |  |  |  |

Table 7.1.2.3-2: Out of band blocking for Band n77 and n78

| NR band | Parameter | Units | Frequency |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Range 1 | Range 2 | Range 3 |
|  | PInterferer | dBm | [-44] | [-30] | [-15] |
| n77, n78 (NOTE X) | FInterferer (CW) | MHz | FDL_low -TBD to FDL_low - TBD | FDL_low - TBD to FDL_low - TBD | FDL_low - TBD to 1 MHz |
|  |  |  | FDL_high + TBD to FDL_high + TBD | FDL_high + TBD to FDL_high + TBD | FDL_high + TBD to +12750 MHz |
| NOTE X: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > FDL_low - 600 MHz and FInterferer < FDL_high + 600 MHz. |  |  |  |  |  |

7.2	BS specific

<Text to be added>

7.2.1	Operating band unwanted emissions

<Text to be added>

7.2.2	Additional spurious emissions requirements

<Text to be added>

7.2.3	Co-location with other base stations

<Text to be added>

7.2.4	General blocking requirement

<Text to be added>

7.2.5	Blocking requirement for co-location with other base stations

<Text to be added>

8	NR band 3.3GHz - 4.2GHz specific issues

8.1	UE specific

<Text to be added>

8.1.1	Transmitter characteristics

8.1.1.1	UE maximum output power

Based on the same discussion as UE maximum output power of Band n78 described in clause 7.1.1.1, the following was agreed.

Agreement: MOP is to be specified as 23 dBm +2/-3 dB for Band n77 and n78 of power class 3

8.1.1.2	UE maximum output power for modulation / channel bandwidth

<Text to be added>

8.1.1.3	UE maximum output power with additional requirements

<Text to be added>

8.1.1.4	Adjacent Channel Leakage Ratio (ACLR)

<Text to be added>

8.1.1.5	Spectrum emission mask

<Text to be added>

8.1.1.6	Spurious emissions

Based on the same discussion as Spurious emissions of Band n78 described in clause 7.1.1.6, the following was agreed.

Table 8.1.1.6-1: Spurious emission band UE co-existence for Band n77 and n78

| NR band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n77, n78 | E-UTRA Band 1, 3, 11, 18, 19, 21, 28, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | PHS |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band [n257] | 26500 | - | 29500 | TBD | TBD |  |
|  | (More bands to be added for other regions) |  |  |  |  |  |  |

Agreement: Protected bands required for Band n77 are to be the same as those for Band n78 (i.e., not only Japanese bands but also ones required for other regions such as Europe, Korea and China)

8.1.2	Receiver characteristics

8.1.2.1	Reference sensitivity

Based on the same discussion as reference sensitivity of Band n78 described in clause 7.1.2.1, the following was agreed.

Agreement: REFSENS for Band n77 at 3.3-3.8 GHz and n78 should be 1 dB larger than that of bands which have NR smallest sensitivity (less RF challenges) such as Band n1. REFSENS for Band n77 at 3.8-4.2 GHz is increased by 0.5 dB compared to that of Band n78.

8.1.2.2	Adjacent Channel Selectivity (ACS)

<Text to be added>

8.1.2.3	Blocking

#### 8.1.2.3.1	Out-of-band blocking

Based on the same discussion as out-of-band blocking of Band n78 described in clause 7.1.2.3.1, the following was agreed.

Agreement: Out-of-band blocking for Band n77 and n78 should be specified as below.

Table 8.1.2.3.1-1: Out-of-band blocking parameters for Band n77 and n78

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |  |  |  |
|  |  | 6 | 7 | 9 | 12 | 13 | 14 | 15 | 16 |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified TBD with PCMAX_L as defined in subclause 6.2.5. NOTE 2: Reference measurement channel is TBD NOTE 3: The REFSENS power level is TBD |  |  |  |  |  |  |  |  |  |

Table 8.1.2.3.1-2: Out of band blocking for Band n77 and n78

| NR band | Parameter | Units | Frequency |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Range 1 | Range 2 | Range 3 |
|  | PInterferer | dBm | [-44] | [-30] | [-15] |
| n77, n78 (NOTE X) | FInterferer (CW) | MHz | FDL_low - TBD to FDL_low - TBD | FDL_low - TBD to FDL_low - TBD | FDL_low - TBD to 1 MHz |
|  |  |  | FDL_high + TBD to FDL_high + TBD | FDL_high + TBD to FDL_high + TBD | FDL_high + TBD to +12750 MHz |
| NOTE X: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > FDL_low - 600 MHz and FInterferer < FDL_high + 600 MHz. |  |  |  |  |  |

8.2	BS specific

<Text to be added>

8.2.1	Operating band unwanted emissions

<Text to be added>

8.2.2	Additional spurious emissions requirements

<Text to be added>

8.2.3	Co-location with other base stations

<Text to be added>

8.2.4	General blocking requirement

<Text to be added>

8.2.5	Blocking requirement for co-location with other base stations

<Text to be added>

9	Required changes to NR, E-UTRA, UTRA and MSR specifications

The required changes to the 3GPP specifications for the NR band 3.3-3.8GHz are summarised in a Table 9-1.

Table 9-1: Overview of 3GPP specifications with required changes

| Affected existing specifications |  |  |  |
| --- | --- | --- | --- |
| Spec No. | Subject of the CR | Comments | CR/TP (Tdoc) |

The required changes to the 3GPP specifications for the NR band 3.3-4.2GHz are summarised in a Table 9-2.

Table 9-2: Overview of 3GPP specifications with required changes

| Affected existing specifications |  |  |  |
| --- | --- | --- | --- |
| Spec No. | Subject of the CR | Comments | CR/TP (Tdoc) |

Annex <A> (normative):
<Normative annex title>

Annexes are only to be used where appropriate:

Annex B:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 08/2017 | RAN4#84 | R4-1707861 |  |  | TR skeleton | N/A | 0.0.1 |
| 11/2017 | RAN4#85 | R4-1713002 |  |  | R4-1711816, TP for TR 38.813 UE RF requirements of Band n77 and n78 | 0.0.1 | 0.1.0 |