| 3GPP TR 38.893 V0.0.1 (2022-09) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on UE support of regionally-defined subsets of an NR band (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	7

3.3	Abbreviations	7

4	Root cause	7

5	Possible solutions	7

5.1	Solution 1	7

5.2	Solution 2	7

6	Conclusions and recommendations	7

Annex A (informative): Change history	8

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

3GPP defines specifications that are applicable worldwide including the definition of NR frequency bands across both FR-1 and FR-2 frequency ranges.  In general, 3GPP seeks to enable economies of scale and commonality of bands across multiple countries and regions around the globe.  This can lead to bands with very wide bandwidth to be able to capture the frequency ranges within this band that might be available within different countries.  Hence, the band definitions are not necessarily tailored to the frequency ranges available in any one country but may be broad enough to enable applicability across many countries.  In addition, mechanisms such as NS signaling have been introduced in RAN4 specifications to enable the network to indicate additional or unique country or deployment-specific requirements on top of the general requirements associated with the band.

In order for a UE to declare support of a band, it must support the entire band including all of the general requirements as well as all NS requirements specified for this band.  Consequently, the UE is able to operate on the band in any country where this band or portion thereof is deployed while also meeting any country specific requirements or regulations that may be applicable.  However, it has been observed recently that there are situations where the UE can only operate on one or more subsets of the band within a particular country.  This was reported for Band n77 first in the US and then again in Canada.  In both of these cases, Band n77 was defined as a very wide band with global applicability, but the regulators in the US and in Canada only made available portions of the band for mobile usage.  Moreover, different portions of the band were made available at different times, so there was the possibility that UE’s might support one or more-than-one subset of the band depending on when the UE was designed, manufactured, and/or certified with respect to the timing of spectrum release and regulations in the country.  To be clear, to meet 3GPP requirements, the UE must support the entire band.  However, the situation describe above pertains to regulatory compliance rather than compliance to 3GPP requirements.  If the UE is not certified by a country’s regulator to operate on a certain part of the band, irrespective of what is specified in 3GPP, the UE is not allowed to operate on that part of the band in the country.

Specific solutions were identified and specified for Band n77 to resolve the issues related to spectrum subsets in the US and Canada.  However, it is anticipated that issues such as this may become more commonplace and therefore a RAN level study item to consider a more generalized solution was agreed.  This technical report captures the contents of the study item.

# 1	Scope

The present document captures the study on UE support of regionally-defined subsets of an NR band.  In accordance with 3GPP specifications, a UE is required to support the entire frequency range of a band as defined by [2], [3], or [4].  However, it was found that for some bands, there may be regional regulatory restrictions to this frequency range in some countries.  The scope of this study is to investigate the root causes that may require 3GPP specification changes to accommodate as well as to consider possible general solutions if needed.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.101-1: User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone

[3]	3GPP TS 38.101-2: User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone

[4]	3GPP TS 38.101-5: User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	Root cause

# 5	Possible solutions

## 5.1	Solution 1

## 5.2	Solution 2

# 6	Conclusions and recommendations

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-09 | RP-97e | RP-222221 | - | - | - | Skeleton TR for 38.893 | 0.0.1 |