| 3GPP TR 28.871 V1.1.0 (2024-10) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on Service Based Management Architecture (SBMA) enhancement phase 3 (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	7

Introduction	8

1	Scope	9

2	References	9

3	Definitions of terms, symbols and abbreviations	10

3.1	Terms	10

3.2	Symbols	11

3.3	Abbreviations	11

4	Concepts and background	11

4.1	Service Based Management Architecture	11

4.2	Naming an MnS supporting a specific set of capabilities	11

5	Use cases and potential requirements	11

5.1	NRM investigation	11

5.1.1	Description	11

5.1.2	Potential requirement	12

5.1.3	Potential solutions	12

5.2	Connection between 3GPP TS 28.537 and 3GPP TS 28.622 for generic control NRM capabilities	13

5.2.1	Description	13

5.2.2	Potential requirements	13

5.2.3	Potential solutions	13

5.2.3.0	General	13

5.2.3.1	Potential solution#1 Add reference for solution description (NRM fragment) for each management capability in TS 28.537.	14

5.2.3.2	Potential solution#2 Add reference for management capability description for each NRM fragment in 3GPP TS 28.622	14

5.2.3.3	Potential solution#3 Add reference for management capability requirements for each NRM fragment in 3GPP TS 28.622	14

5.2.4	Evaluation of potential solutions	15

5.3	Discovery of management capabilities of MnS supported by a MnS Producer	15

5.3.1	Description	15

5.3.2	Potential requirements	16

5.3.3	Potential solutions	16

5.3.4	Evaluation of potential solutions	20

5.4	 <<Support IOCs>> instances in SBMA	20

5.4.1	Description	20

5.4.2	Potential requirements	21

5.4.3	Potential solutions	21

5.4.4	Evaluation of potential solutions	21

5.5	Discovery of management capabilities of MnS instances based on area of interest	21

5.5.1	Description	21

5.5.2	Potential requirements	21

5.5.3	Potential solutions	21

5.5.4	Evaluation of potential solutions	22

5.6	PM investigation	22

5.6.1	Description	22

5.6.2	Potential requirement	22

5.6.3	Potential solutions	22

5.7	Common Notification Header	26

5.7.1	Description	26

5.7.2	Potential requirements	27

5.7.3	Potential solutions	27

5.7.3.1	Potential solution#1 Add definition of the common notification header to 3GPP TS 28.532	27

5.7.4	Evaluation of potential solutions	27

5.8	Schema retrieval enhancements	28

5.8.1	Description	28

5.8.2	Potential requirements	28

5.8.3	Potential solutions	28

5.8.3.1	Potential solution#1, Schema list at known location	28

5.8.3.2	Potential solution#2 Schema lists as capabilities	28

5.8.3.3	Potential solution#3 Solution set specific solutions	29

5.8.4	Evaluation of potential solutions	29

5.9	Schema reference enhancements	29

5.9.1	Description	29

5.9.2	Potential requirements	29

5.9.3	Potential solutions	29

5.9.3.1	Potential solution#1, Update ProvMnS OpenAPI definition to be more generic	29

5.9.3.2	Potential solution#2, decouples the ProvMnS schema and supported feature schema	30

5.9.4	Evaluation of potential solutions	32

5.10	List and handle Alarming-Conditions	32

5.10.1	Description	32

5.10.2	Potential requirements	32

5.10.3	Potential solutions	33

5.10.3.1	Potential solution #part1: List of alarming-conditions	33

5.10.3.2	Potential solution #part2: Filtering on alarming condition	33

5.10.4	Evaluation of potential solutions	33

5.11	Overview of management capabilities and corresponding Solution Sets	33

5.11.1	Description	33

5.11.2	Potential solutions	33

5.11.3	Evaluation of potential solutions	35

5.12	MnS Version Number Handling	35

5.12.1	Description	35

5.12.2	Potential requirements	36

5.12.3	Potential solutions	36

5.12.3.1 Potential solution#1 MnS Version scheme based on semver.	36

5.12.4	Evaluation of potential solutions	37

5.13	Reliable notification transfer	37

5.13.1	Description	37

5.13.2	Potential requirement	37

5.13.3	Potential solutions	38

5.13.3.1	Part#1 Add sequence number (REQ-rel-notif-1)	38

5.13.3.2	Part#2 Retrieve stored notifications  (REQ-rel-notif-2 & 3)	38

5.13.3.3	Part#3 Notification could not be sent/prepared (REQ-rel-notif-4)	38

5.13.3.3.1 	Problem Scenario	38

5.13.3.3.2	 Part#3a notifyNotificationNotSent	39

5.13.3.3.3 	 Part#3b state variables on the NtfSubsciptionControl	39

5.13.3.3.4	Part#3c Alarm from NtfSubsciptionControl	39

5.13.4	Evaluation of potential solutions	39

5.14	Usage of MnS Registry NRM fragment for MnS Registry and Discovery for different deployment scenarios	40

5.14.1	Description	40

5.14.2	Potential solutions	40

5.14.3	Evaluation of potential solutions	41

5.15	Decouple the management and resource models	42

5.15.1	Description	42

6	Conclusions and recommendations	43

6.1	NRM investigation	43

6.2	Connection between 3GPP TS 28.537 and 3GPP TS 28.622 for generic control NRM capabilities	43

6.3	Discovery of management capabilities of MnS instances based on area of interest	43

6.4	PM investigation	43

6.5	<<Support IOCs>> instances in SBMA	43

6.6	Schema reference enhancements	43

6.7	Overview of management capabilities and corresponding Solution Sets	44

6.8	Discovery of management capabilities of MnS instance supported by a MnS Producer	44

6.9	Usage of MnS Registry NRM fragment for MnS Registry and Discovery for different deployment scenarios	44

Annex <X> (informative): Change history	45

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

With the progression in the use of service-based management services by service providers and operators, new and updated use cases and potential requirements are introduced for managing 5G network and services within the service-based management architecture (SBMA). The present document studies the corresponding updates for the SBMA.

# 1	Scope

The present document describes use cases, potential requirements, and solutions for the enhancement of the Service-Based Management Architecture (SBMA). The present document provides conclusions and recommendations on the next steps in the standardization.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[3]	3GPP TS 28.537: "Management and orchestration; Management capabilities".

[4]	3GPP TS 28.623: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions".

[5]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[6]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics (MDA)".

[7]	3GPP TS 28.105: "Management and orchestration; Artificial Intelligence/ Machine Learning (AI/ML) management".

[8]	3GPP TS 28.111: "Fault management".

[9]	3GPP TS 28.311: "Management and orchestration; Network policy management for mobile networks based on Network Function Virtualization (NFV) scenarios".

[10]	3GPP TS 28.312: "Management and orchestration; Intent driven management services for mobile networks".

[11]	3GPP TS 28.317: "Management and orchestration; Self-configuration of Radio Access Network Entities (RAN NEs)".

[12]	3GPP TS 28.318: "Management and Orchestration; Network and services operations for energy utilities".

[13]	3GPP TS 28.319: "Management and orchestration; Access Control for Management services".

[14]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[15]	3GPP TS 28.536: "Management and orchestration; Management services for communication service assurance; Stage 2 and stage 3".

[16]	3GPP TS 28.538: "Management and orchestration; Edge Computing Management".

[17]	3GPP TS 28.556: "Management and orchestration; Network policy management for 5G mobile networks; Stage 2 and stage 3".

[18]	3GPP TS 28.404 "Telecommunication management; Quality of Experience (QoE) measurement collection; Concepts, use cases and requirements)".

[19]	3GPP TS 28.405 "Telecommunication management; Quality of Experience (QoE) measurement collection; Control and configuration)".

[20]	3GPP TS 28.406 "Telecommunication management; Quality of Experience (QoE) measurement collection; Information definition and transport".

[21]	3GPP TS 28.520 "Telecommunication management; Performance Management (PM) for mobile networks that include virtualized network functions; Requirements".

[22]	3GPP TS 28.521 "Telecommunication management; Performance Management (PM) for mobile networks that include virtualized network functions; Procedures".

[23]	3GPP TS 28.522 Telecommunication management; Performance Management (PM) for mobile networks that include virtualized network functions; Stage 2".

[24]	3GPP TS 28.523 Telecommunication management; Performance Management (PM) for mobile networks that include virtualized network functions; Stage 3".

[25]	3GPP TS 28.550 "Management and orchestration; Performance assurance".

[26]	3GPP TS 28.552 "Management and orchestration; 5G performance measurements".

[27]	3GPP TS 28.554 "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[28]	3GPP TS 28.558 "Management and orchestration; UE level measurements for 5G system".

[29]	3GPP TS 32.401 "Telecommunication management; Performance Management (PM);Concept and requirements".

[30]	3GPP TS 32.404 "Telecommunication management; Performance Management (PM); Performance measurements; Definitions and template".

[31]	3GPP TS 32.421 "Telecommunication management; Subscriber and equipment trace; Trace concepts and requirements".

[32]	3GPP TS 32.422 "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[33]	3GPP TS 32.423 "Telecommunication management; Subscriber and equipment trace; Trace data definition and management".

[34]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[35]	3GPP TS 28.531: "Management and orchestration; Provisioning"

[36]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH) "

[37]	3GPP TS 28.312: "Management and orchestration; Intent driven management services for mobile networks"

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

Void.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

CRUD	Create, Read, Update and Delete

IOC	Information Object Class

IRP	Integration Reference Point

IS	Information Service

MnS	Management Service

NRM	Network Resource Model

SBMA	Service Based Management Architecture

# 4	Concepts and background

## 4.1	Service Based Management Architecture

Service Based Management Architecture (SBMA) is specified in 3GPP TS 28.533 [34].

The fundamental building block of the SBMA is the Management Service (MnS). An MnS is a set of offered capabilities for management and orchestration of network and services. The entity producing an MnS is called MnS producer. The entity consuming an MnS is called MnS consumer.

The architecture for the present study is based on the existing SBMA.

Possible solutions comply with the SBMA principles in 3GPP TS 28.533 [34].

## 4.2	Naming an MnS supporting a specific set of capabilities

An MnS is composed of so called MnS components. There are three components called MnS component type A, MnS component type B and MnS component type C, see clause 4.2 of 3GPP TS 28.533 [34].

Each MnS has component A and B, and may, in addition, also have the component C. For the model driven approach using an NRM and CRUD operations, MnS component type A are always the CRUD operations. Therefore, the real capabilities of an MnS are defined through the component B and C. Annex F of 3GPP TS 28.533 [34] describes the usage of CRUD operations and NRM fragments to support management capabilities in SBMA. However, the management capabilities which are implemented by non-CRUD operations are missing. It will be benefit to illustrate all the management capabilities which can be implemented by CRUD operation and/or non-CRUD operations.

# 5	Use cases and potential requirements

## 5.1	NRM investigation

### 5.1.1	Description

One important argument when introducing SBMA was to get away from the fragmented specification structure that IRP architecture has.

The following TSs contain stage 2 NRM fragments specified for 5G:

28.622 Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS) [2]

28.541 Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3 [5]

28.104 Management and orchestration; Management Data Analytics (MDA) [6]

28.105 Management and orchestration; Artificial Intelligence/ Machine Learning (AI/ML) management [7]

28.111 Fault management [8]

28.311 Management and orchestration; Network policy management for mobile networks based on Network Function Virtualization (NFV) scenarios [9]

28.312 Management and orchestration; Intent driven management services for mobile networks [10]

28.317 Management and orchestration; Self-configuration of Radio Access Network Entities (RAN NEs) [11]

28.318 Management and Orchestration; Network and services operations for energy utilities [12]

28.319 Management and orchestration; Access Control for Management services [13]

28.532 Management and orchestration; Generic management services [14]

28.536 Management and orchestration; Management services for communication service assurance; Stage 2 and stage 3 [15]

28.538 Management and orchestration; Edge Computing Management [16]

28.556 Management and orchestration; Network policy management for 5G mobile networks; Stage 2 and stage 3 [17]

### 5.1.2	Potential requirement

NA.

### 5.1.3	Potential solutions

Solution proposal 1

Do nothing.

Pro: No risk for inconsistencies. No work needs to be done.

Con: Non SA5 NRM experts continue to have the problem of understanding the 3GPP 5G network resource model.

Solution proposal 2

Describe the dependencies in a more understandable way in a 900-series TR.

Pro: Non SA5 NRM experts have an easier way of understanding how SA5 NRM specifications relate to each other. Since this would be a 900-series TR it would be visible to organizations outside 3GPP and kept up-to-date across releases.

Con: As the information is duplicated, it is a risk for not being consistent.

Solution proposal 3

Change the structure of the NRM TSs. E.g. One TS could be for RAN NFs, another for Core Network NFs, a third for management system MnFs.

Pro: The understanding per NRM fragment would be better.

Con: All dependencies might not be visible. It is a very large work.

Solution proposal 4

Augment the "5G specifications overview" [28.533, Annex E] to include the NRM components. For example, the column currently headed "Related specifications" could be split into one describing use cases and requirements, and another defining common/specific NRM. A separate column could also be added, including not only the TS but the specific NRM component(s) defined in it that are related to the management feature. To increase visibility, and promote maintenance, the Annex could be promoted to normative, or even moved into the main body of the TS.

Pro: The mapping between specifications, management features, and NRM would be captured in a single location.

Con: The amount of information in the table could be large and difficult to maintain.

Solution proposal 5

Augment the existing specifications containing NRM definitions to indicate a clear "entry point" or "root" NRM component for each management feature. E.g. the "ManagementDataCollection" IOC for MADCOL, "Intent" IOC for IDMS, etc. Each of these would then document the management feature(s) to which it applies and the other IOCs/DTs which comprise the complete solution. E.g. Intent IOC would indicate it is the root for IDMS and that the IntentReport and related DTs are also required.

NOTE:	This solution could also be combined with Proposal 4 to reduce the amount of information required in the table.

Pro: Existing information is retained and augmented with more detail. The documentation on dependencies could be kept to the minimal number of 'root' NRM components.

Con: Could be difficult for multi-release maintenance when some components (or parts thereof) only apply to specific release(s).

Solution proposal 6

Create a new type of document, such as a web/wiki page, to document the NRMs and dependencies.

Pro: Could be easier to maintain and have least impact on existing specs. Method to introduce different 'views' on NRM usage for potentially different audiences. E.g. Rel-17 vs. Rel-18 view, Slice vs. NF mgmt., ORAN centric implementation, etc.

Con: Separation of the information from the actual specs could lead to inconsistencies.

## 5.2	Connection between 3GPP TS 28.537 and 3GPP TS 28.622 for generic control NRM capabilities

### 5.2.1	Description

Several control NRM fragments (e.g. MnS Registry NRM fragment, File retrieval NRM fragment, File download NRM fragment and Management data collection NRM fragment) are defined in 3GPP TS 28.622 [2]. However, corresponding stage1 (including general description, use case and requirements) contents are defined in 3GPP TS 28.537 [3]. The connection between stage 2 (defined in 3GPP TS 28.622 [2]) and stage 1 (defined in 3GPP TS 28.537 [3]) for each control NRM fragment needs to be added to improve the readability and traceability otherwise it is not clear which control NRM fragments support which management capabilities defined in 3GPP TS 28.537 [3].

### 5.2.2	Potential requirements

NA

### 5.2.3	Potential solutions

#### 5.2.3.0	General

Following potential solutions use management capability of discovery of management services defined in 3GPP TS 28.537[3] and MnS Registry NRM fragment defined in 3GPP TS 28.622 [2] as example, same solution should also be applied for other management capabilities in 3GPP TS 28.537 [3], including Heartbeat (clause 4 of 3GPP TS 28.537 [3]), Managing management data (clause 6 of 3GPP TS 28.537 [3]) and File management (clause 7 of 3GPP TS 28.537 [3]).

#### 5.2.3.1	Potential solution#1 Add reference for solution description (NRM fragment) for each management capability in TS 28.537.

Following clauses 5.1 overview and 5.2 specification level requirements are the existing clauses of 3GPP TS 28.537 [3], clause 5.3 solution (between quotes) is proposed to be added in clause 5 in 3GPP TS 28.537 [3] to “add reference to solution description (NRM fragment) for management capability of discovery of Management Services in 3GPP TS 28.537 [3]”.

5	Discovery of Management Services

5.1	Overview

5.2	Specification level requirements

5.3	Solution

“The stage 2 for this management capability is described in 3GPP TS 28.622 [3], clause X (MnSRegistry NRM fragment).”

“The stage 3 for this management capability is described in 3GPP TS 28.623 [4], clause X (YAML) and clause Y (YANG).”

#### 5.2.3.2	Potential solution#2 Add reference for management capability description for each NRM fragment in 3GPP TS 28.622

Following clause 4.3.41 is the existing clauses of 3GPP TS 28.622 [2], the first sentence (in italic) is proposed to be added in clause 4.3.41.1 in 3GPP TS 28.622 [2].

4.3.41	MnsRegistry

4.3.41.1	Definition

The MnSRegistry IOC and contained MnsInfo IOC is used to support the management capability of Discovery of Management Services in clause 5 in 3GPP TS 28.537 [3].

This IOC is a container for MnsInfo IOC-s. It can be contained only by SubNetwork IOC. A SubNetwork IOC can contain only one instance of MnsRegistry.The IOC is instantiated by the system.

#### 5.2.3.3	Potential solution#3 Add reference for management capability requirements for each NRM fragment in 3GPP TS 28.622

Following clause 4.3.42 is the existing clause, the sentence (in italic) and table below are proposed to be added in clause 4.3.42.1 in 3GPP TS 28.622 [2].

4.3.42	MnsInfo

4.3.42.1	Definition

This IOC represents an available Management Service (MnS) and provides the data required to support its discovery. It is name-contained by MnsRegistry.

This information is used by the consumer to discover the producers of specific Management Services and to derive the addresses of the Management Service.

Attributes mnsLabel, mnsType, and mnsVersion are used to describe the Management Service.

Attribute mnsAddress is used to provide addressing information for the Management Service operations.

Attribute mnsScope is used to provide information about the management scope of the Management Service. The management scope is defined as the set of managed object instances that can be accessed using the Management Service.

Following requirements related to management capability of Discovery of Management Services supported by the MnsInfo IOC.

| Table 5.2.3.3-1: MnSInfo requirements referencesReferenced TS | Requirement label | Comment |
| --- | --- | --- |
| 3GPP TS 28.537 [3] | REQ-DMS-1 |  |
| 3GPP TS 28.537 [3] | REQ-DMS-2 |  |
| 3GPP TS 28.537 [3] | REQ-DMS-3 |  |
| 3GPP TS 28.537 [3] | REQ-DMS-4 |  |

### 5.2.4	Evaluation of potential solutions

All potential solutions described in clause 5.2.3 can be used to establish the connection between 3GPP TS 28.537 [3] and 3GPP TS 28.622[2] for generic control NRM capabilities to improve the readability for management capability.

- The potential solution#1, for each management capability identified/listed in 3GPP TS 28.537[3], ensure that the clause describing such capability contains overview information (stage 1, stage 2 and stage 3). 3GPP TS 28.537[3] becomes one-stop shop for generic NRM capabilities. For example, the reader who wants to know the file management capability defined in SA5 can look at TS 28.537[3] to obtain all standardized information.

- The potential solution#2, for each NRM fragment identified/listed in 3GPP TS 28.622[2], clarify which management capabilities can be supported. It can be used to improve the readability of 3GPP TS 28.622 [3].

- The potential solution#3 follows the template for NRM defined in TS 32.160 and provides the detailed mapping information between the requirements and corresponding IOCs for each NRM fragment in 3GPP TS 28.622 [2].

Based on above analyse, it is proposed to adopt:

- Potential solution#1 for improving the readability of 3GPP TS 28.537 [3] to illustrate the reference for corresponding stage2 and stage 3 solutions for each management capability.

- Potential solution#3 for improving the readability of 3GPP TS 28.622 [2] to illustrate the reference for the supported stage1 requirements for each NRM fragment.

## 5.3	Discovery of management capabilities of MnS supported by a MnS Producer

### 5.3.1	Description

The use cases and requirements for discovery of Management Services are described in 3GPP TS 28.537 [3], which includes:

-	MnS Consumer retrieves management service information from MnS registry

-	MnS Consumer retrieves detailed capabilities about management service

In TS 28.622[2], MnsInfo IOC is introduced to describe the management service information and detailed capabilities about management service, which includes:

Table 5.3.1-1: MnSInfo properties

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| mnsLabel | M | T | F | F | T |
| mnsType | M | T | F | F | T |
| mnsVersion | M | T | F | F | T |
| mnsAddress | M | T | F | F | T |
| mnsScope | M | T | F | F | T |

The model driven approach (i.e. usage of provisioning MnS operations specified in 3GPP TS 28.532 [14] and NRM fragments) can be used to support various types of management capabilities in SBMA. MnS consumer may need to get management capability information of the MnS in an convenient way. For example, MnS consumer may need to query which MnS can provide the threshold control capability. The current MnSType attribute in MnSInfo IOC cannot be used to represent the management capability information of MnS.

### 5.3.2	Potential requirements

REQ-DMS-1: The 3GPP management system should provide capabilities allowing MnS consumers to retrieve the management capability information for MnS.

### 5.3.3	Potential solutions

Following are the proposed enhancement for MnSInfo IOC defined in 3GPP TS 28.622 [2]:

Enhancement Aspect 1: Add attribute 'mnsCapability' to represent the types of management capabilities of the MnS instance.

Table 5.3.3-1: mnsCapability description

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| mnsCapability | It describes the types of management capabilities of the MnS instance provided by the MnS producer. Alternative 1 for the allowed values: - NR_PROVISIONING - 5GC_PROVISIONING - NETWORK_SLICING_PROVISIONING - EDGE_COMPUTING_PROVISIONING - PERFORMANCE_METRIC_COLLECTION_CONTROL - PERFORMANCE_METRIC_DATA_REPORT - PERFORMANCE_METRIC_THRESHOLD_MONITOR_CONTROL - PERFORMANCE_METRIC_THRESHOLD_NOTIFICATION - FAULT_CONTROL - FAULT_NOTIFICATION - TRACE_MDT_DATA_COLLECTION_CONTROL - TRACE_MDT_DATA_REPORT - QOE_DATA_COLLECTION_CONTROL - QOE_DATA_REPORT - FILE_RETRIEVAL - FILE_DOWNLOAD - SUBSCRIPTION_CONTROL - HEARTBEAT_CONTROL - HEARTBEAT_NOTIFICATION - ML_MODEL_MANAGEMENT - MANAGEMENT DATA ANALYTIC - SON_POLICY - RANSC_MANAGEMENT - INTENT_DRIVEN_MANAGEMENT - MNS_REGISTRY_AND_DISCOVERY - COMMUNICATION_SERVICE_ASSURANCE_CONTROL - DSO_RAPID_RECOVERY_AND_THRESHOLD MONITORING - MNS_ACCESS_CONTROL_MANAGEMENT Alternative #2 for the allowed values: - MANAGEMENT_DATA_CONTROL - FAULT_MANAGEMENT - FILE_MANAGEMENT - NR_PROVISIONING - 5GC_PROVISIONING - NETWORK_SLICING_PROVISIONING - EDGE_COMPUTING_PROVISIONING - AI/ML_MANAGEMENT - MDA - SON_POLICY - RANSC_MANAGEMENT - INTENT_DRIVEN_MANAGEMENT - MNS_REGISTRY_AND_DISCOVERY - COMMUNICATION_SERVICE_ASSURANCE - NSOEU - MSAC_MANAGEMENT NOTE: Using the alternative#1 or alternative#2 as the enum values needs to be decided during normative | Type: Enum multiplicity: 0..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

### 5.3.4	Evaluation of potential solutions

Only one potential solution is proposed, which is feasible. The name and definition of the attribute “mnsCapability” needs to be revisited during normative phase, e.g.  mnsProvisioningCategories, label.

## 5.4		<<Support IOCs>> instances in SBMA

### 5.4.1	Description

SBMA is based on data that is visible, readable, and depending on its nature, potentially writable using the CRUD operations defined in 3GPP TS 28.532 [3]. <<SupportIOC>> instances, see <<SupportIOC>> defined in 3GPP TS 32.156 [2], cannot be handled by the CRUD operations.

The only reason to model data is because that data is available in the MnS producer. While it is possible to provide this information via notifications, files or to set the data via dedicated operations SBMA follows the principles:

- if the data is interesting for the consumer, it should be readable via CRUD, because the consumer might not be available when the data is produced. The consumer should be able to fetch the data on demand when the consumer needs it and when the consumer is capable of receiving it.

- it is easier for a consumer to handle a few simple CRUD operations with configuration specific data models than to implement and handle a potentially big number of different dedicated operations and access methods.

Based on the above CRUD operations should be available for modelled data. CRUD operations are available for <<InformationObjectClass>> instances but not for <<SupportIOC>> instances, hence specifying new <<SupportIOC>> definitions should be avoided.

Removing existing <<SupportIOC>> definition (if any) is out of scope for the present document.

### 5.4.2	Potential requirements

NA.

### 5.4.3	Potential solutions

Clarify in TS 32.156[2] that <<SupportIOC>> instances are not used when an MnS is designed based on a model driven approach using a NRM and CRUD operations.

### 5.4.4	Evaluation of potential solutions

One potential solution proposed in clause 5.4.3 is feasible.

## 5.5	Discovery of management capabilities of MnS instances based on area of interest

### 5.5.1	Description

In 3GPP TS 28.622 [2], MnsInfo IOC is introduced to describe the management service information and detailed capabilities about management service, which includes attribute "mnsScope".

Attribute "mnsScope" is used to provide information about the management scope of the Management Service. Current definition describes the list containing the DNs of the managed object instances that can be accessed using the Management Service. However, in some scenarios, MnS consumer also needs to retrieve the information of MnS instance supporting management capabilities for the specified area of interest instead of list of DN of managed object instance. The area can be geographical area or TAI. For an example, the MnS consumer (e.g. NWDAF) request to retrieve the MnS information for the MnS instance which can provide the MDA capabilities for a specified area (e.g. TAC).

### 5.5.2	Potential requirements

REQ-DMS-1: The 3GPP management system should provide capabilities allowing MnS consumers to retrieve the MnS information for MnS instances based on specified areas (geographical area or TAI) or RATs.

### 5.5.3	Potential solutions

Following are the proposed enhancement for MnSInfo IOC defined in 3GPP TS 28.622 [2]. Enhancement Aspect 1: Extend existing attribute "mnsScope" to support the following choices:

-	A list of DNs to represent the managed object instances that can be accessed using the Management Service. This is the already supported by 3GPP TS 28.622 [2].

-	A list of TAIs to represent the managed object instances covered by the specified TACs that can be accessed using the Management Service.

-	A list of GeoAreas to represent the managed object instances covered by the specified GeoAreas that can be accessed using the Management Service.

### 5.5.4	Evaluation of potential solutions

Only one potential solution is proposed, which is feasible.

## 5.6	PM investigation

### 5.6.1	Description

One important argument when introducing SBMA was to get away from the fragmented specification structure that IRP architecture has.

The following TSs contain performance information specified for 5G:

28.404 Telecommunication management; Quality of Experience (QoE) measurement collection; Concepts, use cases and requirements) [18]

28.405 Telecommunication management; Quality of Experience (QoE) measurement collection; Control and configuration) [19]

28.406 Telecommunication management; Quality of Experience (QoE) measurement collection; Information definition and transport [20]

28.520 Telecommunication management; Performance Management (PM) for mobile networks that include virtualized network functions; Requirements [21]

28.521 Telecommunication management; Performance Management (PM) for mobile networks that include virtualized network functions; Procedures [22]

28.522 Telecommunication management; Performance Management (PM) for mobile networks that include virtualized network functions; Stage 2 [23]

28.523 Telecommunication management; Performance Management (PM) for mobile networks that include virtualized network functions; Stage 3 [24]

28.550 Management and orchestration; Performance assurance [25]

28.552 Management and orchestration; 5G performance measurements [26]

28.554 Management and orchestration; 5G end to end Key Performance Indicators (KPI) [27]

28.558 Management and orchestration; UE level measurements for 5G system [28]

32.401 Telecommunication management;Performance Management (PM);Concept and requirements [29]

32.404 Telecommunication management; Performance Management (PM); Performance measurements; Definitions and template [30]

32.421 Telecommunication management; Subscriber and equipment trace; Trace concepts and requirements [31]

32.422 Telecommunication management; Subscriber and equipment trace; Trace control and configuration management [32]

32.423 Telecommunication management; Subscriber and equipment trace; Trace data definition and management [33]

### 5.6.2	Potential requirement

NA.

### 5.6.3	Potential solutions

Solution proposal 1

Do nothing.

Pro: No risk for inconsistencies. No work needs to be done.

Con: Non SA5 PM experts continue to have the problem of understanding how the 3GPP 5G performance TSs relate to each other.

Solution proposal 2

Describe the dependencies in a more understandable way in a 900-series TR.

Pro: Non SA5 PM experts have an easier way of understanding how SA5 performance specifications relate to each other. Since this would be a 900-series TR it would be visible to organizations outside 3GPP and kept up-to-date across releases.

Con: As the information is duplicated, it is a risk for not being consistent.

Solution proposal 3a

Change the structure of the performance TSs. E.g. One TS could be for RAN NFs, another for Core Network NFs, a third for management system MnFs.

Pro: The understanding for which performance metrics are produced would be better. It would mitigate compliance work.

Con: All dependencies might not be visible. It is a very large work.

Solution proposal 3b

Change the structure of the Subscriber and Equipment Trace and the Quality of Experience (QoE) measurement collection. The other PM specifications are divided into mechanism and performance data, which is not the case for the Subscriber and Equipment Trace and the Quality of Experience (QoE) measurement collection, so also these specifications can be changed to the structure of separating mechanism and performance data.

Pro: The mechanism is common for different 3GPP systems, while the performance data may differ. It will be very clear on what data is valid for which 3GPP system.

Con: When performance data is used for several 3GPP systems (e.g. NSA), the description of these cases needs to refer to another TS.

Solution proposal 4

Augment the "5G specifications overview" (3GPP TS 28.533 [34], Annex E) to include the performance components. For example, the column currently headed "Related specifications" could be split into one describing use cases and requirements, and another defining performance data. A separate column could also be added, including not only the TS but the specific performance data definitions defined in it that are related to the management feature. To increase visibility, and promote maintenance, the Annex could be promoted to normative, or even moved into the main body of the TS. Different releases can have different clauses or annexes in the TS.

Pro: The mapping between specifications, management features, and performance data definitions would be captured in a single location.

Con: The amount of information in the table could be large and difficult to maintain.

For example, the performance components table can be summarized as follows:

Editor notes: where to put the following table in normative work needs FFS.

Table 5.6.3-1: QoE measurements collection

| Category | Performance Measurements | Related specifications |
| --- | --- | --- |
| QoE measurements | QoE metrics for 3GP-DASH QoE metrics for MTSI QoE metrics for VR | 3GPP TS 28.404 [18] for the requirements of QoE measurement collection 3GPP TS 28.405 [19] for the procedure of QoE measurement collection 3GPP TS 28.406 [20] for the definition of recording content of QoE measurement |

Table 5.6.3-2: PM/KPI for 5G networks

| Category | Performance Measurements | Related specifications |
| --- | --- | --- |
| Performance measurements for gNB | Packet related measurements | 3GPP TS 28.552 [26] for the definition of performance measurements 3GPP TS 28.550 [25] for the performance measurements management services |
|  | Radio resource utilization related measurements |  |
|  | UE throughput measurements |  |
|  | RRC related measurements |  |
|  | Mobility related measurements |  |
|  | TB related Measurements |  |
|  | DRB related measurements |  |
|  | QoS related measurements |  |
|  | Energy related measurements |  |
|  | Random access related measurements |  |
|  | Signal related measurements |  |
|  | MRO related measurements |  |
|  | Paging Measurement |  |
|  | MU-MIMO related measurements |  |
|  | GTP related measurements |  |
| Performance measurements for 5GC | Performance measurements for NSOEU |  |
|  | Performance measurements for AMF |  |
|  | Performance measurements for SMF |  |
|  | Performance measurements for UPF |  |
|  | Performance measurements for PCF |  |
|  | Performance measurements for UDM |  |
|  | Common performance measurements for NFs |  |
|  | Performance measurements for N3IWF |  |
|  | Performance measurements for NEF |  |
|  | Performance measurements for NRF |  |
|  | Performance measurements for NSSF |  |
|  | Performance measurements for SMSF |  |
|  | Performance measurements for UDR |  |
|  | Performance measurements for ECS |  |
|  | Performance measurements for EES |  |
|  | Performance measurements for LMF |  |
|  | Performance measurements for NWDAF |  |
| Performance measurements for network slicing | Measurements related to end-to-end 5G network and network slicing | 3GPP TS 28.552 [26] for the definition of performance measurements 3GPP TS 28.550 [25] for the performance measurements management services |
| Key Performance Indicators (KPIs) | Accessibility KPI | 3GPP TS 28.554 [27] for the definition of the Key Performance Indicators 3GPP TS 28.550 [25] for the performance measurements management services |
|  | Integrity KPI |  |
|  | Utilization KPI |  |
|  | Retainability KPI |  |
|  | Mobility KPI |  |
|  | Energy Efficiency (EE) KPI |  |
|  | Reliability KPI |  |
|  | Average air-interface efficiency KPI |  |
|  | Network and Service Operations for Energy Utilities (NSOEU) KPI |  |

Table 5.6.3-3: performance measurements for UE

| Category | Performance Measurements | Related specifications |
| --- | --- | --- |
| UE level measurements for UPF | Packet delay related UE level measurements | 3GPP TS 28.558 [28] for the definition of UE performance measurements 3GPP TS 32.421 [31], 3GPP TS 32.422 [32], TS 32.423 [33] for the definition of UE performance measurements reporting and recording content |
| UE level measurements for gNB | Packet delay related UE level measurements |  |
|  | Packet Loss related UE level measurements |  |
|  | UE throughput related UE level measurements |  |

Table 5.6.3-4: MDT/Trace measurements for 5G networks

| Category | Performance Measurements | Related specifications |
| --- | --- | --- |
| MDT/Trace measurements | AMF Trace Record SMF Trace Record PCF Trace Record AUSF Trace Record NEF Trace Record NRF Trace Record NSSF Trace Record UDM Trace Record UPF Trace Record SMSF Trace Record AF Trace Record gNB-CU-CP Trace Record gNB-CU-UP Trace Record gNB-DU Trace Record ng-eNB Trace Record NR MDT Trace Record Content 5GC UE level measurement Trace Record | 3GPP TS 32.421 [31] for the requirements of MDT/Trace measurements reporting 3GPP TS 32.422 [32] for the definition of MDT/Trace measurements reporting procedure 3GPP TS 32.423 [33] for the definition of recording content of MDT/Trace measurement |

Solution proposal 5

Augment the existing specifications containing performance information to indicate a clear "entry point" or "root" NRM component for each management feature. E.g. the "PerfMetricJob" IOC for PM measurements, "TraceJob" IOC for Subscriber and Equipment trace, etc. Each of these would then document the management feature(s) to which it applies and the other IOCs/DTs which comprise the complete solution.

NOTE:	 This solution could also be combined with Proposal 4 to reduce the amount of information required in the table.

Pro: Existing information is retained and augmented with more detail. The documentation on dependencies could be kept to the minimal number of 'root' NRM components.

Con: Could be difficult for multi-release maintenance when some components (or parts thereof) only apply to specific release(s).

Solution proposal 6

Create a new type of document, such as a web/wiki page, to document the performance data dependencies.

Pro: Could be easier to maintain and have least impact on existing specs. Method to introduce different 'views' on usage performance information for potentially different audiences. E.g. Rel-17 vs. Rel-18 view, Slice vs. NF mgmt., ORAN centric implementation, etc.

Con: Separation of the information from the actual specs could lead to inconsistencies.

Different solutions proposals can be combined. E.g. the proposals 4 and 3b can be combined, which would mean that the structure for Subscriber and Equipment Trace and the Quality of Experience (QoE) measurement collection is changed and the relations between the specifications are described in the annex in 28.533.

## 5.7	Common Notification Header

### 5.7.1	Description

All SA5 notifications have a common header containing 6 parameters present in all notifications (objectClass, objectInstance, notificationId, notificationType, eventTime, systemDN).

The common header is not defined in 5G documentation as a concept; although it is referenced in multiple 3GPP and external documents, which as of necessity often still refer to 3GPP TS 32.302 (a document not valid for 5G).

The definition of these parameters should but is not always the same in different notifications which is confusing.

### 5.7.2	Potential requirements

NA

### 5.7.3	Potential solutions

#### 5.7.3.1	Potential solution#1 Add definition of the common notification header to 3GPP TS 28.532

Common notification header parameters

All notifications emitted by any MnS or function support and contain the following parameters:

Table 5.7.3.1-1: Input parameters

| Parameter Name | S | Documentation and Allowed Values | Properties |
| --- | --- | --- | --- |
| objectClass | M | Name of an IOC identified by objectInstance. | Type: String multiplicity: 1 isOrdered: N/A isUnique: N/A |
| objectInstance | M | Distinguished name (DN) of an MOI. Together with objectClass it identifies an MOI the notification is related to. It can be e.g. an MOI that caused the notification to be emitted or an MOI about which the notification reports some information. | Type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A |
| notificationId | M | This is an identifier for the individual notification, which may be used to correlate notifications. The identifier of the notification shall be chosen to be unique across all notifications of a particular managed object instance throughout the time that correlation is significant, it uniquely identifies the notification from other notifications generated by the MOI specified by objectInstance. | Type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A |
| notificationType | M | String: It specifies the type of notification and hence the information carried by the notification. It should be a fixed, short, human readable string for each type of notification. | Type: String multiplicity: 1 isOrdered: N/A isUnique: N/A |
| eventTime | M | The date and time when the event that triggered the sending of the notification occurred. | Type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A |
| systemDN | M | Distinguished name of the MnS producer. If an MnSAgent MOI is present, systemDN should be the DN of an MnSAgent. | Type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A |

The parameters may be further specified by the individual notification types.

Individual notification types may define additional parameters.

### 5.7.4	Evaluation of potential solutions

TBD

## 5.8	Schema retrieval enhancements

### 5.8.1	Description

This enhancement proposes methods to support retrieval of schema files/modules.

### 5.8.2	Potential requirements

REQ-Schema-retrieval -1: The MnS Producer shall support advertising its supported schema files/modules.

REQ-Schema-retrieval -2: The schema advertisement shall support files/modules located at any location specified by the MnS Producer.

REQ-Schema-retrieval -3: The MnS Producer shall support retrieval of all files/modules.

REQ-Schema-retrieval -4: The MnS Producer shall support retrieval of specific schema files/modules.

### 5.8.3	Potential solutions

#### 5.8.3.1	Potential solution#1, Schema list at known location

This enhancement proposes methods to support advertisement and retrieval of schema files/modules using a model- based approach:

- Add objects to define each available schema including:

  - Schema id (identifier, e.g. file/module name).

  - Schema version (based on versioning information in the file/module).

  - Schema format (Enum: yang, yaml).

  - Schema namespace (URI).

  - Schema location (file URI).

  - Supported feature list (if applicable).

  - Indicator of "implemented" vs. "imported" (if applicable).

- Advertisement of supported schema:

  - Define a specific location from which the list of supported schema is available. Similar to the ../schemas/.. tree defined in [3], clause 2.1.3 this would be a predefined location from which any consumer can query the list.

  - Each schema would be an entry in a 'list' of schema available at this location.

- Schema retrieval:

  - Consumer uses the information in the schema list entries to retrieve the schema file(s) of interest.

  - The actual method to retrieve them (e.g. HTTP, sftp) is defined by the solution and indicated in the <protocol> portion of the schema location URI.

#### 5.8.3.2	Potential solution#2 Schema lists as capabilities

This enhancement is similar to #1, but instead of placing the schema at a known location the schema files would be added to the capabilities advertised by specific MnS and/or managed entities. I.e. the schema file details would be data in the capabilities, not individual MOIs per schema file.

#### 5.8.3.3	Potential solution#3 Solution set specific solutions

This enhancement proposes different methods based on solution set:

- OpenAPI: predefine a resource end point from which the schema list can be queried. Define as a tree allowing single/multiple entries to be queried.

- NETCONF: implement per RFC6022 or RFC8525

### 5.8.4	Evaluation of potential solutions

Solution #1 is recommended as it provides a standardized set of meta data for each supported schema along with standardized mechanisms to store and query such information.  This solution could fulfil the proposed requirements in a consistent manner for all solution sets.

NOTE: whether the solution has any impact on MNSInfo IOC needs to be determined during normative work.

Solution#2 is not recommended.  It introduces an unnecessary coupling between the schema query and the MnS discovery.  This solution could fulfil the proposed requirements in a consistent manner for all solution sets.

Solution #3 is not recommended.  It adds no benefit from a standardization perspective.  It is already possible in current solution.  Whether or not the proposed requirements could be met, and how, would be solution set specific.

## 5.9	Schema reference enhancements

### 5.9.1	Description

The current ProvMnS.yaml contains all possible schema entries when only a subset may actually be supported by a particular management system. Additionally, a management system may require adjustment of the list of supported schema at runtime, e.g. when new NFs with different schema and/or versions are introduced.

In both cases above the current hard-coded ProvMnS which contains all possible entries despite their applicability, and which cannot be adjusted at runtime is not sufficient.

### 5.9.2	Potential requirements

REQ-Schema-retrieval -1: The MnS Producer shall support flexible extension of the required schema files.

### 5.9.3	Potential solutions

#### 5.9.3.1	Potential solution#1, Update ProvMnS OpenAPI definition to be more generic

This enhancement proposes replacing the current predefined schema references in ProvMns.yaml with an extensible mechanism allowing the ProvMnS to dynamically adjust to the required schema list at runtime.

Currently the ProvMnS.yaml includes the following specific list of schema entries:

Resource:

oneOf:

- type: object

properties:

id:

type: string

objectClass:

type: string

objectInstance:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

attributes:

type: object

additionalProperties:

type: array

items:

type: object

required:

- id

- anyOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/resources-genericNrm'

- $ref: 'TS28541_NrNrm.yaml#/components/schemas/resources-nrNrm'

- $ref: 'TS28541_5GcNrm.yaml#/components/schemas/resources-5gcNrm'

- $ref: 'TS28541_SliceNrm.yaml#/components/schemas/resources-sliceNrm'

- $ref: 'TS28536_CoslaNrm.yaml#/components/schemas/resources-coslaNrm'

- $ref: 'TS28312_IntentNrm.yaml#/components/schemas/resources-intentNrm'

- $ref: 'TS28104_MdaNrm.yaml#/components/schemas/resources-mdaNrm'

- $ref: 'TS28105_AiMlNrm.yaml#/components/schemas/resources AiMlNrm'

- $ref: 'TS28538_EdgeNrm.yaml#/components/schemas/resources-edgeNrm'

- $ref: 'TS28317_RanScNrm.yaml#/components/schemas/resources-RanScNrm'

Proposal is to update this to remove the references, and allow the ProvMnS to provide its list of supported schema (i.e. all supported schema as advertised/available for the MnS):

Resource:   # for a resource to be compliant with application/vnd.3gpp.object-tree-flat+json or application/vnd.3gpp.object-tree-hierarchical+json as per NRM definitions (28.541), See 32.158 $4.3.1, $ 6.1.4 for more info

type: object

properties:

id:

type: string

objectClass:

type: string

objectInstance:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

attributes:

type: object

additionalProperties:

type: array

items:

type: object

#### 5.9.3.2	Potential solution#2, decouples the ProvMnS schema and supported feature schema

This enhancement proposes replacing the current predefined schema references in ProvMns.yaml with an extensible mechanism allowing the ProvMnS to dynamically adjust to the required schema list at runtime.

Currently the ProvMnS.yaml includes the following specific list of schema entries for Resource:

Resource:

oneOf:

- type: object

properties:

id:

type: string

objectClass:

type: string

objectInstance:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

attributes:

type: object

additionalProperties:

type: array

items:

type: object

required:

- id

- anyOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/resources-genericNrm'

- $ref: 'TS28541_NrNrm.yaml#/components/schemas/resources-nrNrm'

- $ref: 'TS28541_5GcNrm.yaml#/components/schemas/resources-5gcNrm'

- $ref: 'TS28541_SliceNrm.yaml#/components/schemas/resources-sliceNrm'

- $ref: 'TS28536_CoslaNrm.yaml#/components/schemas/resources-coslaNrm'

- $ref: 'TS28312_IntentNrm.yaml#/components/schemas/resources-intentNrm'

- $ref: 'TS28104_MdaNrm.yaml#/components/schemas/resources-mdaNrm'

- $ref: 'TS28105_AiMlNrm.yaml#/components/schemas/resources AiMlNrm'

- $ref: 'TS28538_EdgeNrm.yaml#/components/schemas/resources-edgeNrm'

- $ref: 'TS28317_RanScNrm.yaml#/components/schemas/resources-RanScNrm'

- $ref: 'TS28623_FileManagementNrm.yaml#/components/schemas/resources-fileMgmtNrm'

- $ref: 'TS28623_ManagementDataCollectionNrm.yaml#/components/schemas/resources-mgmtDataCollectionNrm'

- $ref: 'TS28623_MnSRegistryNrm.yaml#/components/schemas/resources-mnSRegistryNrm'

- $ref: 'TS28623_PmControlNrm.yaml#/components/schemas/resources-pmControlNrm'

- $ref: 'TS28111_FaultNrm.yaml#/components/schemas/resources-faultNrm'

- $ref: 'TS28623_QoEMeasurementCollectionNrm.yaml#/components/schemas/resources-qoEMeasuremetCollectionNrm'

- $ref: 'TS28623_SubscriptionControlNrm.yaml#/components/schemas/resources-subscriptionControlNrm'

- $ref: 'TS28623_ThresholdMonitorNrm.yaml#/components/schemas/resources-thresholdMonitorNrm'

- $ref: 'TS28623_TraceControlNrm.yaml#/components/schemas/resources-traceControlNrm'

- $ref: 'TS28319_MsacNrm.yaml#/components/schemas/resources-msacNrm'

- $ref: 'TS28318_DsoNrm.yaml#/components/schemas/resources-DSORecovery'

Proposal is to add a separate feature NRM schema and move all the schema references to the new separate feature NRM schema. A reference to the new separate feature NRM schema also needs to be added in the ProvMnS schema.

New feature NRM schema see below:

openapi: 3.0.1

info:

title: NRM Featurue

version: 18.1.0

description: >-

OAS 3.0.1 definition of the features of NRM

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.623; Generic NRM, NRM feature

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.623/

paths: {}

components:

schemas:

#----- Definitions in TS 28.623 for TS 28.532 --------------------------#

resources-feature:

anyOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/resources-genericNrm'

- $ref: 'TS28541_NrNrm.yaml#/components/schemas/resources-nrNrm'

- $ref: 'TS28541_5GcNrm.yaml#/components/schemas/resources-5gcNrm'

- $ref: 'TS28541_SliceNrm.yaml#/components/schemas/resources-sliceNrm'

- $ref: 'TS28536_CoslaNrm.yaml#/components/schemas/resources-coslaNrm'

- $ref: 'TS28312_IntentNrm.yaml#/components/schemas/resources-intentNrm'

- $ref: 'TS28104_MdaNrm.yaml#/components/schemas/resources-mdaNrm'

- $ref: 'TS28105_AiMlNrm.yaml#/components/schemas/resources-AiMlNrm'

- $ref: 'TS28538_EdgeNrm.yaml#/components/schemas/resources-edgeNrm'

- $ref: 'TS28317_RanScNrm.yaml#/components/schemas/resources-RanScNrm'

- $ref: 'TS28623_FileManagementNrm.yaml#/components/schemas/resources-fileMgmtNrm'

- $ref: 'TS28623_ManagementDataCollectionNrm.yaml#/components/schemas/resources-mgmtDataCollectionNrm'

- $ref: 'TS28623_MnSRegistryNrm.yaml#/components/schemas/resources-mnSRegistryNrm'

- $ref: 'TS28623_PmControlNrm.yaml#/components/schemas/resources-pmControlNrm'

- $ref: 'TS28111_FaultNrm.yaml#/components/schemas/resources-faultNrm'

- $ref: 'TS28623_QoEMeasurementCollectionNrm.yaml#/components/schemas/resources-qoEMeasuremetCollectionNrm'

- $ref: 'TS28623_SubscriptionControlNrm.yaml#/components/schemas/resources-subscriptionControlNrm'

- $ref: 'TS28623_ThresholdMonitorNrm.yaml#/components/schemas/resources-thresholdMonitorNrm'

- $ref: 'TS28623_TraceControlNrm.yaml#/components/schemas/resources-traceControlNrm'

- $ref: 'TS28319_MsacNrm.yaml#/components/schemas/resources-msacNrm'

- $ref: 'TS28318_DsoNrm.yaml#/components'

#----- Definitions in TS 28.623 for TS 28.532 --------------------------#

Updated Resource schema in ProvMnS schema see below:

Resource:

oneOf:

- type: object

properties:

id:

type: string

objectClass:

type: string

objectInstance:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

attributes:

type: object

additionalProperties:

type: array

items:

type: object

- $ref: 'TS28623_featureNrm.yaml#/components/schemas/resources-feature'

#### 5.9.4	Evaluation of potential solutions

Solution #1 is not recommended for the reason it introduces a missing connection between ProvMnS schema and supported feature schema.

Solution#2 is recommended which addresses above concern. This solution decouples the ProvMnS schema and supported feature schema without missing connection.

## 5.10	List and handle Alarming-Conditions

### 5.10.1	Description

Observation 1: Alarm capability

Currently, there is no standardized mechanism defined that can be used to retrieve the set of all alarming-conditions that a MnS producer could notify about.

Observation 2: Alarm related notification filtering is difficult

The most basic requirement on alarm related notification filtering is to filter based on the alarming-condition (defined by the alarmType, probableCause, specificProblem). As some of these parameters are not carried in some alarm related notifications this filtering task is not possible e.g. for notifyCorrelatedNotificationChanged.

### 5.10.2	Potential requirements

Table 5.10.2-1: Requirements

| Requirement label | Description | Related use case(s)/Motivation |
| --- | --- | --- |
| REQ-MS-FMAL-1 | A producer should be able to provide a list of all the alarming-conditions it might raise an alarm for. |  |
| REQ-MS- FMAL-2 | Every alarming-condition should have a set of associated data that describes the alarming-condition. The information set includes at least: alarmType, probableCause, specificProblem | The information helps the user correctly handle an alarm. Additional information may be included for each alarming-condition |
| REQ-MS- FMAL-3 | It should be possible to filter alarm related notifications based on the alarming-condition. | Motivation: Management systems may not want to handle all alarming-conditions. |

### 5.10.3	Potential solutions

#### 5.10.3.1	Potential solution #part1: List of alarming-conditions

FFS

#### 5.10.3.2	Potential solution #part2: Filtering on alarming condition

3GPP TS 28.111[3] defines that the attributes alarmType, probableCause and specificProblem (if present) identify the alarming condition. To allow filtering on alarming conditions all notifications related to individual alarms shall carry the parameters alarmType, probableCause and specificProblem.

SpecificProblem shall be added to the input parameters of notifyClearedAlarm, notifyChangedAlarm, notifyAckStateChanged, notifyComments.

All 3 parameters shall be added to notifyCorrelatedNotificationChanged.

### 5.10.4	Evaluation of potential solutions

The potential solution described in clause 5.10.3.2 fulfils the requirement REQ-MS- FMAL-3. It is recommended for normative work.

## 5.11	Overview of management capabilities and corresponding Solution Sets

### 5.11.1	Description

Annex F in 3GPP TS 28.533 [34] describes the usage of CRUD operations and NRM fragments to support management capabilities in SBMA. However, the management capabilities which are implemented by non-CRUD operations are missing. It will be of benefit to illustrate all the management capabilities which can be implemented by CRUD operations and/or non-CRUD operations.

In addition, almost 30 NRM fragments are defined in several TSs (see overview of MnS usage label for different types of management capabilities implemented by CRUD operations and an NRM fragment in 3GPP TS 28.533) for SBMA. For some NRM fragments, both YAML and YANG solution sets are defined, while, for others, only YAML solution sets. It is difficult for the reader to obtain the information on whether one or both two solution sets are defined for each NRM fragment.

### 5.11.2	Potential solutions

The Table 5.11.2-1: Overview of MnS usage label for different types of management capabilities implemented by CRUD operations and an NRM fragment in 3GPP TS 28.533 [34] can be updated to illustrate the Overview of management capabilities and corresponding solution sets.

Table 5.11.2-1: Overview of management capabilities and corresponding solution sets

| Management Feature | Management Capability | MnS definition | Solution Sets |
| --- | --- | --- | --- |
| Provisioning | NR Provisioning | CRUD operations/notifications (3GPP TS 28.532 [14]) + NR NRM fragment (3GPP TS 28.541 [5]) | RESTFUL |
|  |  |  | NETCONF/YANG |
|  | 5GC Provisioning | CRUD operations/notifications (TS 28.532 [14]) + 5GC NRM fragment (3GPP TS 28.541 [5]) | RESTFUL |
|  |  |  | NETCONF/YANG |
|  | Network Slicing Provisioning | CRUD operations/notifications (3GPP TS 28.532 [14]) + Network Slicing NRM fragment (3GPP TS 28.541 [5]) | RESTFUL |
|  |  |  | NETCONF/YANG |
|  |  | Network Slice Provisioning MnS (3GPP TS 28.531 [35]) + Network Slice Subnet Provisioning MnS (3GPP TS 28.531 [35]) | RESTFUL |
|  | Edge Computing Provisioning | CRUD operations/notifications (3GPP TS 28.532 [14]) + Edge NRM fragment (3GPP TS 28.538 [16]) | RESTFUL |
| Performance Assurance | Performance Metric Collection Control | CRUD operations/notifications (3GPP TS 28.532 [14]) + PM control NRM fragment (3GPP TS 28.622 [2]) | RESTFUL |
|  |  |  | NETCONF/YANG |
|  |  | CRUD operations/notifications (3GPP TS 28.532 [14]) + ManagementDataCollection control NRM fragment (3GPP TS 28.622 [2]) | RESTFUL |
|  |  |  | NETCONF/YANG |
|  |  | Performance measurement job control (3GPP TS 28.550 [25]) | RESTFUL |
|  | Performance Metric Data Report | Streaming data reporting service (3GPP TS 28.532 [14]) + Performance data stream units (3GPP TS 28.550 [25]) | RESTFUL+WebSocket+(GPB/ASN.1) (also used by the NETCONF_YANG solution set) |
|  |  | File data reporting service (3GPP TS 28.532 [14]) + Performance data file format (3GPP TS 28.532 [14]) | RESTFUL+( SFTP/FTPES/HTTPS)+XML |
|  | Performance Metric Threshold Monitor Control | CRUD operations/notifications (3GPP TS 28.532 [14]) + Threshold monitoring control NRM fragment (3GPP TS 28.622 [2]) | RESTFUL |
|  |  |  | NETCONF/YANG |
|  | Performance Metric Threshold Notification | notifyThresholdCrossing notification (3GPP TS 28.532 [14]) | RESTFUL |
| Fault Management | Fault control | CRUD operations/notifications (3GPP TS 28.532 [14]) + FM control NRM fragment (3GPP TS 28.111 [8]) | RESTFUL |
|  |  |  | NETCONF/YANG |
|  | Fault Notification | Fault Notifications (3GPP TS 28.111 [8]) | RESTFUL (also used by NETCONF/YANG) |
| Trace&MDT | Trace/MDT data collection control | CRUD operations/notifications (3GPP TS 28.532 [14]) + Trace control NRM fragment (3GPP TS 28.622 [2]) | RESTFUL |
|  |  |  | NETCONF/YANG |
|  |  | CRUD operations/notifications (3GPP TS 28.532 [14]) + ManagementDataCollection control NRM fragment (3GPP TS 28.622 [2]) | RESTFUL |
|  |  |  | NETCONF/YANG |
|  | Trace/MDT data report | Streaming data reporting service (3GPP TS 28.532 [14]) + Trace/MDT stream date schema definition (3GPP TS 32.423 [33]) | RESTFUL+WebSocket+(GPB/ASN.1) |
|  |  | File data reporting service (3GPP TS 28.532 [14]) + Trace/MDT file date format definition (TS 32.423 [33]) | RESTFUL+( SFTP/FTPES/HTTPS)+XML |
| QoE | QoE data collection control | CRUD operations/notifications (3GPP TS 28.532 [14]) + QoE Measurement Collection control NRM fragment (3GPP TS 28.622 [2]) | RESTFUL |
|  |  |  | NETCONF/YANG |
|  | QoE data report | File data reporting service (3GPP TS 28.532 [14]) + QoE data file format (3GPP TS 26.247 [36]) | RESTFUL+XML |
| File Management | File Retrieval | CRUD operations/notifications (3GPP TS 28.532 [14]) + File retrieval NRM fragment (3GPP TS 28.622 [2]) | RESTFUL |
|  |  |  | NETCONF/YANG |
|  | File Download | CRUD operations/notifications (3GPP TS 28.532 [14]) + File download NRM fragment (3GPP TS 28.622 [2]) | RESTFUL |
|  |  |  | NETCONF/YANG |
| Notification subscription and Heartbeat notification control | Subscription Control | CRUD operations/notifications (3GPP TS 28.532 [14]) + Notification subscription and heartbeat notification control NRM fragment (3GPP TS 28.622 [2]) | RESTFUL |
|  |  |  | NETCONF/YANG |
|  | Heartbeat Control | CRUD operations/notifications (3GPP TS 28.532 [14]) + Heartbeat notification control NRM fragment (3GPP TS 28.622[2]) | RESTFUL |
|  |  |  | NETCONF/YANG |
|  | Heartbeat Notification | notifyHeartbeat notification (3GPP TS 28.532 [14]) | RESTFUL |
| MDAS | Management Data Analytic | CRUD operations/notifications (3GPP TS 28.532 [14]) + NRM fragment for MDA request and MDA report (3GPP TS 28.104 [6]) | RESTFUL |
| SON | RANSC Management | CRUD operations/notifications (3GPP TS 28.532 [14]) + RANSC NRM Fragment 3GPP TS 28.317 [11]) | RESTFUL |
|  | SON policy | CRUD operations/notifications (3GPP TS 28.532 [14]) + NRM Fragment for DANR/DES/DRACH/DMRO/DPCI/CES/CPCI/DLMO/CCO Management (3GPP TS 28.541 [5]) | RESTFUL |
|  |  |  | NETCONF/YANG |
| Closed-loop SLS | Communication Service Assurance Control | CRUD operations/notifications (3GPP TS 28.532 [14]) + Assurance management NRM fragment (3GPP TS 28.536 [15]) | RESTFUL |
| Intent-driven | Intent Driven Management | CRUD operations/notifications (3GPP TS 28.532 [14]) + NRM fragment for intent driven management (3GPP TS 28.312 [35]) | RESTFUL |
| AI/ML | ML model Management | CRUD operations/notifications (3GPP TS 28.532 [14]) + NRM fragment for ML model training/testing/ inference emulation control/ ML model loading / inference (3GPP TS 28.105 [7]) | RESTFUL |
| MnS Registry and Discovery | MnS Registry and Discovery | CRUD operations/notifications (3GPP TS 28.532 [14]) + MnS Registry NRM fragment (TS 28.622 [2]) | RESTFUL |
|  |  |  | NETCONF/YANG |
| MSAC | MnS Access Control | CRUD operations/notifications (3GPP TS 28.532 [14]) + Information model for role based access control (3GPP TS 28.319 [13]) | RESTFUL |
| NSOEU | DSO Rapid Recovery and Threshold Monitoring | CRUD operations/notifications (3GPP TS 28.532 [14])+ DSO Rapid Recovery NRM fragment (3GPP TS 28.318 [12]) + DSO Rapid Recovery NRM fragment(3GPP TS 28.318 [12]) | RESTFUL |

### 5.11.3	Evaluation of potential solutions

Only one potential solution is proposed, which is feasible.

## 5.12	MnS Version Number Handling

### 5.12.1	Description

Each MnS Producer needs to advertise itself in the MnS Registry with a version number sufficient for the MnS Consumer to understand its basic operations (i.e. component A as specified in a given 3GPP release) to then allow its Consumers to connect and perform detailed discovery of specific capabilities of that MnS (i.e. component B and component C).

Whether a vendor deploys the MnS definition as-is (i.e. no modifications from 3GPP versions) or extends them to add vendor defined capabilities a versioning scheme should allow MnS Consumers to clearly identify a particular MnS Producer version.

A versioning scheme needs to be defined to allow an MnS Producer to differentiate its baseline capabilities across different versions.  This versioning will account for the basic operations the MnS Producer can perform (i.e. specific component A as defined in a particular 3GPP release), but does not imply other capabilities (i.e. component B and component C) which are provided during detailed discovery.

In other words, the MnS Consumer needs to know about the MnS Producer version to know how to perform basic operations but relies on capabilities discovery to get detailed management information.

### 5.12.2	Potential requirements

REQ-MNS-versioning-1: Information identifying the 3GPP TS(s) and version(s) for which the content is defined shall be provided in the yaml and yang files.

REQ-MNS-versioning-2: An MnS Producer may comprise multiple yaml files or yang modules, each containing version information, however the MnS version in the registry shall provide a single version number.

REQ-MNS-versioning-3: The MnS Producer shall advertise the MnS version in the MnS registry.

REQ-MNS-versioning-4: The MnS version number shall be vendor specified.

### 5.12.3	Potential solutions

#### 5.12.3.1 Potential solution#1 MnS Version scheme based on semver.

This proposal is aligned with how other 3GPP groups have defined versions for OpenAPI as defined in [3].

MnS version numbers shall consist of at least 3 fields, following a MAJOR.MINOR.PATCH pattern according to the Semantic Versioning Specification [4].  The 1st Field (MAJOR), the 2nd Field (MINOR), and the 3rd Field (PATCH) shall contain unsigned integer numbers, and they shall not contain leading zeroes.

Information in the YAML files and YANG modules indicates which TS versions the definitions are based on:

Examples:

- yaml indicates 3GPP TS, plus version info as follows:

 extermalDocs:  
   description: 3GPP TS 28.532; Generic management services
   url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.532/

info:
   title: Provisioning MnS
   version: 18.2.0

- yang indicates the 3GPP TS, plus version information as follows:

reference "3GPP TS 28.623

Generic Network Resource Model (NRM)

Integration Reference Point (IRP);

Solution Set (SS) definitions

3GPP TS 28.622

Generic Network Resource Model (NRM)

Integration Reference Point (IRP);

Information Service (IS)" ;

       revision 2023-11-06 { reference "CR-0290 CR-0294"; }

The values above are used to associate the yaml and yang to the 3GPP TS and version.  These files may contain a default MnS version value.  The default value is not used to publish the MnS version in the MnS Registry.

The MnS version is a vendor specified value for the MnS used to advertise the MnS version in the registry.  I.e. MNSInfo.mnsVersion.  Example:

- “1.0.0” indicates a vendor defined version defined in yang module(s) or yaml file(s) provided by the vendor which may contain vendor extensions.

### 5.12.4	Evaluation of potential solutions

TBD

## 5.13	Reliable notification transfer

### 5.13.1	Description

The handling of notifications has been identified as an area of improvement because there is no guarantee that notifications are always successfully sent to and received by consumers. Consumers might be unaware of the misalignment with the producer. There are multiple conditions that can cause the misalignment:

- Network issues: both temporary and permanent outages. These may be caused both by real connectivity issues and unavailability of the notification recipient.

- Producer internal issues while preparing/sending the notification, (e.g. overload situation, error) may result in notifications not being sent.

- Consumer exceptions reported to the producer e.g. with HTTP response codes (4xx, 5xx).

- Notifications may be received out of order e.g. if separate https connections are used to transfer subsequent notifications.

- Consumer issues e.g. abrupt shutdown while processing a notification.

We need a reliable mechanism, to detect abnormal conditions and to recover from them. While the heartbeat mechanism checks the network connection, it does not check that the individual notifications are successfully transferred.

To introduce a reliable notification transfer, the following responsibilities have been identified:

- Producer should be capable of storing all generated notifications per subscription (actual storage might be optimized to avoid duplicate storage for multiple subscriptions).

- Producer should be capable of providing stored notifications, when requested.

- Producer should be capable of providing a mechanism to detect missing notifications.

- Producer should be capable to detect that a notification is not possible to send out and notify it with a notifyNotificationNotSent message and related state variables

- Consumer is responsible for discovering lost or out of order notifications

- Consumer should be capable of requesting missing notifications from the producer

- Consumer should be capable of processing the notifyNotificationNotSent notification.

The feature should be optional to support, but if supported its use should be mandatory.

### 5.13.2	Potential requirement

REQ-rel-notif-1: Producer shall provide a mechanism for the consumer that allows it to discover missing or out-of-order notifications.

REQ-rel-notif-2: Producer shall store all generated notifications up to an implementation specific limit.

REQ-rel-notif-3: Producer shall provide a mechanism for the consumer that allows it to retrieve the stored notifications from the producer.

REQ-rel-notif-4: Producer shall inform the consumer if the producer did not succeed to properly send one or more notifications that should have been sent according to the subscription.

### 5.13.3	Potential solutions

#### 5.13.3.1	Part#1 Add sequence number (REQ-rel-notif-1)

Delivery of notifications can be prevented by many reasons (overload, bugs, firewalls, proxy or concentrator nodes, etc.) some of which cannot be foreseen at the moment. The smartest approach to ensure reliability has been identified as adding a sequence number to all notifications so that the consumer can identify ‘holes’ in the sequence of the notifications and out-of-order notifications. Sequence numbering is a robust mechanism that can detect problems both for the current HTTP 1.1 protocol (see 3GPP TS 28.622 [2]) or any future protocols (e.g. HTTP 3).

Notifications can arrive out-of-order. If the HTTP connection is closed after sending a notification and a new connection is opened for the subsequent notification, the order of delivery is not guaranteed. The sequence number can be used to re-order the notifications.

Every notification should carry a monotonically increasing sequence number, that will be separate for each notification subscription. The sequence number should be a large unsigned integer that is reset to zero at start or restart of the producer.

This will allow the consumer to detect missing or out-of-order notifications. Heartbeat notifications should also carry a new parameter lastEventTime that contains the eventTime parameter from the last (non-heartbeat) notification.

Impacted specifications are: 3GPP TS 28.532 [14] and 3GPP TS 28.111 [8].

#### 5.13.3.2	Part#2 Retrieve stored notifications  (REQ-rel-notif-2 & 3)

To ensure that notifications that failed to be delivered can be retrieved later, for each subscription the producer should store all notifications that were prepared for that specific subscription in a buffer, except notifyHeartbeat messages. The size of the buffer is implementation dependent.

The producer should expose the stored notifications in one or more IOCs per subscription in the NRM (that can be handled by the normal provisioning operations).

Beside exposing the notifications the IOCs may provide

- information about the timespan and/or the sequence number range for which notifications are stored

- a method to retrieve only a specific notification or specific set of notifications

- lastSequenceNumber used (equivalent to the parameter in notifyHeartbeat)

- lastEventTime used (equivalent to the parameter in notifyHeartbeat)

- numberOfStoredNotifications.

Any such attributes should be isNotifiable=False, otherwise an infinite number of value change notifications could be generated.

Other/alternative retrieval methods are for further study.

Impacted specifications are: 3GPP TS 28.622 [2] and 3GPP TS 28.623 [14].

#### 5.13.3.3	Part#3 Notification could not be sent/prepared (REQ-rel-notif-4)

##### 5.13.3.3.1 	Problem Scenario

If a producer knows or suspects that it failed or will fail to prepare or deliver some notifications that are part of a subscription, it should inform the consumer about this.

Situations where notification sending may fail include:

- notifyMOIChanges is not sent if a big set of configuration data is updated in one transaction and the producer doesn’t want to or cannot notify each item,

- when due to overload a set of notifications cannot be sent, this can be substituted by sending a single notifyNotificationNotSent,

- producer internal SW problems e.g. a SW component restarts in the producer. The notification sender may or may not loose information during this restart.

There can be many different situations where a producer is unable to prepare or send out notifications depending on the notification types subscribed but also depending on the implementation of the producer. Due to the many possible failure reasons it would be very difficult for the consumer to monitor all possible underlying reasons e.g. with state variables. To help the consumer (even simple consumers) to detect such producer failures, the subscription should include an overall indication of the problem.

In this problem scenario the original notifications were never prepared/sent. They are not stored, so the sequence number should not be affected.

This function is not an alternative to solutions described in part#1 and part#2. This solution protects against a different problem scenario; thus it should be used together with solutions part#1 and part#2.

##### 5.13.3.3.2		Part#3a notifyNotificationNotSent

In this problem scenario the consumer should send a new notification (type) notifyNotificationNotSent about this problem scenario.

The notification should carry the notificationType(s) of the notifications that were not sent (if it is known). This allows the consumer to limit the recovery action to certain notification types e.g. if only FM is down, there is no need to retrieve CM notifications.

The notifyNotificationNotSent notification should increment and carry the sequence number as all other notifications.

The notification type notifyNotificationNotSent should be implicitly included in all subscriptions, there is no need to include it in the NtfSubscriptionControl.notificationTypes attribute.

Impacted specifications are: 3GPP TS 28.622 [2] and 3GPP TS 28.623 [4].

##### 5.13.3.3.3 		Part#3b state variables on the NtfSubsciptionControl

In this problem scenario the subscription is in a degraded state. This state should be visible not just via the notifyNotificationNotSent notification, it should also be visible by reading NRM attributes. (As a general principle, any data that is available via a notification should also be readable via CRUD operations.)

The producer should provide per subscription

- operationalState attribute

- availabilityStatus attribute potentially with a “dependency/degraded” value

- a list of notification types that are not sent (may be unknown).

##### 5.13.3.3.4	Part#3c Alarm from NtfSubsciptionControl

Implementations may define an alarm to indicate the degraded state of a notification subscription, however a standard alarm is not proposed.

### 5.13.4	Evaluation of potential solutions

Solution part #1, #2 and #3 fulfil the proposed requirements.

Solutions #1, #2 and #3b are recommended for normative work.

It is for further study whether solution #3a should or should not be recommended for normative work.

Solution #3c is not recommended for normative work.

## 5.14	Usage of MnS Registry NRM fragment for MnS Registry and Discovery for different deployment scenarios

### 5.14.1	Description

The following use cases to discover Management Services which are described in clause 5.2.1 in 3GPP TS 28.537 [3]:

- Adding a new management service producer to MnS registry

- Removing a management service producer from MnS registry

- MnS Consumer retrieves management service information from MnS registry

- Providing detailed capabilities about management service

Based on above use cases, the following two mandatory requirements are identified in 5.2.2 in 3GPP TS 28.537 [3]:

REQ-DMS-1: The 3GPP management system shall provide capabilities allowing MnS producers to register their management capabilities (including the endpoint address) at MnS discovery service producer for use by MnS consumers wishing to interact with these MnS producers.

REQ-DMS-2: The 3GPP management system shall provide capabilities allowing MnS consumers to retrieve the management capabilities registered at MnS discovery service producer by MnS producers.

However, the following MnsInfo IOC defined in 3GPP TS 28.622 [2] has all attributes specified as read only. It is obvious that the solution in TS 28.622 [2] only support the MnS discovery capability represented by REQ-DMS-2. The solution to support MnS registration capability represented by REQ-DMS-1 is missing.

Table 5.14.1-1 MnsInfo attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| mnsLabel | M | T | F | F | T |
| mnsType | M | T | F | F | T |
| mnsVersion | M | T | F | F | T |
| mnsAddress | M | T | F | F | T |
| mnsScope | M | T | F | F | T |

In addition, the usage of MnS Registry NRM fragment to support different scenarios are missing.

### 5.14.2	Potential solutions

Following are two potential deployment scenarios for usage of MnS Registry NRM:

-  Scenario#1:  Separate MnSRegistry, the MnsRegistry and the MnS producer are separate entities with a published interface between them. The scenario can be handled with one central MnSRegistry or with multiple distributed MnSRegistries. In this scenario, MnS Registry support both registry and discovery capabilities. In this scenario, the MnS Registry isused to store the MnS Infos for MnS instance provided by different MnFs, especially for different MnFs from different vendors. Following is one example, Domain MnF_1 needs to register MnSInfo_1 in the MnSRegistry implemented in Cross Domain MnF, and domain MnF_2 needs to retrieve/discovery the MnSInfo_1 implemented in Cross Domain MnF.

Figure 5.14.2-1 Example of Separate MnSRegistry

Both REQ-DMS-1 and REQ-DMS-2 described in clause 5.2.1 in TS 28.537 [3] are valid for this scenario. However, the MnsInfo IOC defined in TS 28.622 [2] needs to be updated to change “isWritable” property from “False” to “True”, especially for the scenario #2.

-  Scenario#2: Embedded MnSRegistry, the MnsRegistry and the MnS producer are hosted by the same entity with an implementation specific interface between them. MnS Registry supports discovery capabilities only. In this scenario, the MnS Registry is implemented in a distributed MnF (e.g. RAN domain MnF) and used to store the MnS Infos for MnS instance provided by the MnF itself. Following is one example, Cross Domain MnF needs to retrieve/discovery MnSInfo_1 in the MnSRegistry implemented in Domain MnF_1.

Figure 5.14.2-1 Example of Embedded MnSRegistry

Only REQ-DMS-2 described in clause 5.2.1 in 3GPP TS 28.537 [3] are valid for this scenario and the MnsInfo IOC defined in 3GPP TS 28.622 [2] is correct for this scenario. However, the REQ-DMS-1 in 3GPP TS 28.537 [3] needs to be updated to optional requirement.

Based on above description, following are proposed for the enhancements on 3GPP TS 28.537 [3] and 3GPP TS 28.622 [2] for MnS registry and discovery capabilities:

- Add description on 3GPP TS 28.537 [3] to clarify above two deployment scenarios and corresponding usage of MnS Registry NRM.

- Update the REQ-DMS-1 in 3GPP TS 28.537 [3] as optional requirement as it is not valid for above scenario#2.

- Update the “isWritable” property for all attributes of MnSInfo IOC to “True”, at least “True” for above scenario#1.

### 5.14.3	Evaluation of potential solutions

Only one potential solution is proposed, which is feasible.

## 5.15	Decouple the management and resource models

### 5.15.1	Description

The SA5 MnS architecture includes generic services and generic management information models to support them.  These services are intended to manage RAN and CN entities in a consistent manner.

There is a problem however in how the information model for the management services, and those of the resources it manages, have been defined.  Specifically, generic management objects (e.g. PerfMetricJob) are defined within the same model as the objects that they manage (e.g. name contained under specific Subnetwork, ManagedElement and/or NF instances).

This sort of coupling can be particularly problematic for a management system intending to support multiple releases in parallel.  Since the management objects are themselves defined as part of the NRM resources they are subject to differences between releases and are versioned along with the resource models which they manage.  The contents of the management objects do not have to change at similar cadence as the resource models though.  E.g. new PMs in a release can be supported without change to the PerfMetricJob IOC, ProvMnS versioning does not have to follow cadence of the resource NRMs it operates upon.

Additionally, the NRM defines Subnetwork and ManagedElement as root objects, meaning any instances of the management objects are confined to the same, and single, namespace as the objects being managed.  In other words, a management system spanning multiple Subnetworks or containing multiple root ManagedElements cannot configure truly common management across multiple root elements.  At least not according to the standardized modelling.

A means of improving the decoupling of the management domain model from the NRM models would be beneficial.

NOTE: The term decoupling does not imply any solution choices such as the separation of the 3GPP management model and the resource model into dedicated subtrees.

To better support 3GPP management system implementations capable of consistently managing multiple resource model instances and versions, the objects required by the management system related to management should be able to be fully decoupled from the NRM models they are managing.

### 5.15.2	Potential requirements

REQ-MNS-MgmtModel-1: The 3GPP management model shall be decoupled from the resource models which it manages.

REQ-MNS-MgmtModel-2: The 3GPP management model shall be versioned independent of the resource model versions.

REQ-MNS-MgmtModel-3: The 3GPP management model shall be capable of managing multiple resource model instances.

REQ-MNS-MgmtModel-4: The 3GPP management model shall be capable of managing multiple versions of resource models.

### 5.15.3	Potential solutions

FFS

### 5.15.4	Evaluation of potential solutions

No potential solutions are defined in this document.  The requirements are recommended to be used as input for a future study.

# 6	Conclusions and recommendations

## 6.1	NRM investigation

Solution 4 combined with solution 5 is recommended to be specified in a normative work item.

Solution 4 will specify the entry point specifications and clause (but not list all specifications for 5G), and solution 5 will give the details and the links to the other specifications.

## 6.2	Connection between 3GPP TS 28.537 and 3GPP TS 28.622 for generic control NRM capabilities

The 3GPP TS 28.537 [3] and 3GPP TS 28.622 [2] need to be enhanced to address the issue of missing overview information for each generic management capability.

It is recommended to adopt:

- Potential solution#1 for improving the readability of 3GPP TS 28.537 [3] to illustrate the reference for corresponding stage2 and stage 3 solutions for each management capability.

- Potential solution#3 for improving the readability of 3GPP TS 28.622 [2] to illustrate the reference for the supported stage1 requirements for each NRM fragment.

## 6.3	Discovery of management capabilities of MnS instances based on area of interest

The 3GPP management system needs to provide capabilities allowing MnS consumers to retrieve the MnS information for MnS instances based on specified areas (geographical area or TAI). To support this capability, the attribute "mnsScope" in MnSInfo IOC in 3GPP TS 28.622 [2] needs to be extended to support a list of DNs, a list of TAIs or a list of GeoAreas.

It is recommended to extend existing attribute "mnsScope" in MnSInfo IOC in 3GPP TS 28.622 [2] to support the capabilities allowing MnS consumers to retrieve the MnS information for MnS instances based on specified areas (geographical area or TAI). The detailed solution in clause 5.5.3 is used for normative work.

## 6.4	PM investigation

Solution 4 combined with solution 5 is recommended to be specified in a normative work item.

Solution 4 will specify the entry point specifications and clause (but not list all specifications for 5G), and solution 5 will give the details and the links to the other specifications.

## 6.5	<<Support IOCs>> instances in SBMA

The following bullet is recommended for normative work for use case 5.4:

-	Clarify in 3GPP TS 32.156 [2] that <<SupportIOC>> instances are not used when an MnS is designed based on a model driven approach using a NRM and CRUD operations.

## 6.6	Schema reference enhancements

The requirement on decoupling the ProvMnS schema and supported feature schema (e.g. nrNrm schema, intentNrm schema, coslaNrm schema) is identified in clause 5.9.

Two potential solutions are investigated to satisfy the requirement and solution#2 is identified as feasible solution to better support flexible extension of the required schema files.

It is recommended to update the ProvMnS schema in 3GPP TS 28.532 [14] to decouple the ProvMnS schema and supported feature schema. The detailed solution described in clause 5.9.3.2 is used for normative work.

## 6.7	Overview of management capabilities and corresponding Solution Sets

Table 5.11.2-1 illustrates the overview of management capabilities (management capabilities which can be implemented by CRUD operations and/or non-CRUD operations) and corresponding solution sets.

It is recommended to update the Annex F “Usage of CRUD operations and NRM fragments to support management capabilities in SBMA” in 3GPP TS 28.533 [34] with the overview of management capabilities and corresponding Solution Sets. The detailed solution in clause 5.11.2 can be used as baseline for normative work.

## 6.8	Discovery of management capabilities of MnS instance supported by a MnS Producer

The 3GPP management system needs to provide capabilities allowing MnS consumers to retrieve the management capability information of MnS instance provided by MnS producer. To support this capability, the attribute "mnsCapability" needs to be introduced in MnSInfo IOC in 3GPP TS 28.622 [2] to describe the types of management capabilities of MnS instance provided by MnS producer.

It is recommended to add a new attribute "mnsCapability" in MnSInfo IOC in 3GPP TS 28.622 [2] to support the capabilities allowing MnS consumers to retrieve the management capability information of MnS instance provided by MnS producer. The detailed solution in clause 5.3.3 is used as baseline for normative work.

## 6.9	Usage of MnS Registry NRM fragment for MnS Registry and Discovery for different deployment scenarios

Following two deployment scenarios are described in clause 5.14.1 which requires the enhancement of requirements and solution for MnS registry and discovery capabilities in 3GPP TS 28.537 [3] and 3GPP TS 28.622 [2].

It is recommended to update the 3GPP TS 28.537 [3] and 3GPP TS 28.622 [2] to clearly describe the MnS registry and discovery capabilities and corresponding solutions for different deployment scenarios. The detailed solution described in clause 5.14.2 is used for normative work.

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-04 | SA5#154 |  | - | - | - | Initial skeleton | 0.0.0 |
| 2024-04 | SA5#154 | S5‑241379 | - | - | - | pCR TR 28.871 Add scope for SBMA study report | 0.1.0 |
| 2024-04 | SA5#154 | S5‑241380 | - | - | - | pCR TR 28.871 Add structure for SBMA study report | 0.1.0 |
| 2024-04 | SA5#154 | S5‑241381 | - | - | - | pCR TR 28.871 Add introduction for SBMA study report | 0.1.0 |
| 2024-04 | SA5#154 | S5‑242077 | - | - | - | pCR TR 28.871 Add use case, requirement and solution for connection between TS 28.537 and TS 28.622 for control NRM capabilities | 0.1.0 |
| 2024-04 | SA5#154 | S5‑242080 | - | - | - | Rel-19 pCR TR 28.871 Do not use support IOCs in SBMA | 0.1.0 |
| 2024-04 | SA5#154 | S5‑242081 | - | - | - | pCR 28.871-000 Inventory of 5G NRM specifications | 0.1.0 |
| 2024-04 | SA5#154 | S5-242083 | - | - | - | pCR TR 28.871 Add use case, requirement and solution for discovery of management capabilities of provisioning MnS supported by a MnS Producer | 0.1.0 |
| 2024-05 | SA5#155 | S5‑243203 | - | - | - | pCR TR 28.871 Add use case, requirement and solution for discovery of management capabilities based on area of interest | 0.2.0 |
| 2024-05 | SA5#155 | S5‑243206 | - | - | - | pCR TR 28.871 Add abbreviations for SBMA study report | 0.2.0 |
| 2024-05 | SA5#155 | S5‑243207 | - | - | - | pCR TR 28.871 Update UC for connection between TS 28.537 and TS 28.622 for generic control NRM capabilities | 0.2.0 |
| 2024-05 | SA5#155 | S5‑243205 | - | - | - | pCR TR 28.871 Update UC for discovery of management capabilities of provisioning MnS supported by a MnS Producer | 0.2.0 |
| 2024-05 | SA5#155 | S5‑243208 | - | - | - | pCR TR 28.871 Update Support IOCs in SBMA | 0.2.0 |
| 2024-05 | SA5#155 | S5‑243213 | - | - | - | pCR 28.871 Inventory of 5G performance specifications | 0.2.0 |
| 2024-05 | SA5#155 | S5‑242873 | - | - | - | pCR R19 28.871 Proposed solutions for NRM structure | 0.2.0 |
| 2024-05 | SA5#155 | S5‑242936 | - | - | - | Rel-19 pCR TR 28.871 Common Notification Header | 0.2.0 |
| 2024-05 | SA5#155 | S5‑243204 | - | - | - | pCR TR 28.871 Update solution for discovering of management capabilities of provisioning MnS supported by a MnS Producer | 0.2.0 |
| 2024-05 | SA5#155 | S5-243212 |  |  |  | pCR TR 28.871 Add use case reqs and potential solutions (schema retrieval support) | 0.3.0 |
| 2024-06 | SA5#156 | S5-243789 |  |  |  | pCR TR 28.871 Rapporteur proposal for NRM investigation | 0.3.0 |
| 2024-06 | SA5#156 | S5‑243790 |  |  |  | pCR TR 28.871 Rapporteur proposal for PM investigation | 0.3.0 |
| 2024-06 | SA5#156 | S5‑243791 |  |  |  | pCR TR 28.871 Update Support IOCs in SBMA | 0.3.0 |
| 2024-06 | SA5#156 | S5-243792 |  |  |  | pCR TR 28.871 Update UC for connection between TS 28.537 and TS 28.622 for generic control NRM capabilities | 0.3.0 |
| 2024-06 | SA5#156 | S5-244861 |  |  |  | pCR TR 28.871 Naming an MnS supporting a specific set of capabilities | 0.3.0 |
| 2024-06 | SA5#156 | S5‑244710 |  |  |  | pCR TR 28.871 Add potential solution for schema reference enhancements | 0.3.0 |
| 2024-06 | SA5#156 | S5-244711 |  |  |  | Rel-19 pCR TR 28.871 add potential solutions for PM investigation | 0.3.0 |
| 2024-06 | SA5#156 | S5-244457 |  |  |  | R19 pCR 28871-020 NRM investigation conclusion | 0.3.0 |
| 2024-06 | SA5#156 | S5-244459 |  |  |  | R19 pCR 28871-020 PM investigation conclusion | 0.3.0 |
| 2024-06 | SA5#156 | S5-244727 |  |  |  | pCR TR 28.871 Alarm definition | 0.3.0 |
| 2024-06 | SA5#156 | S5-244729 |  |  |  | pCR TR 28.871 Add evaluation and conclusion for Discovery of management capabilities of MnS instances based on area of interest | 0.3.0 |
| 2024-06 | SA5#156 | S5-244707 |  |  |  | pCR TR 28.871 Add evaluation and conclusion for Connection between TS 28.537 and TS 28.622 for generic control NRM capabilities | 0.3.0 |
| 2024-06 | SA5#156 | S5-244709 |  |  |  | pCR TR 28.871 Add Overview of MnS Capabilities and corresponding Solution Sets | 0.3.0 |
| 2024-09 | SA#105 | SP-241132 |  |  |  | Presented at SA#105 for Information | 1.0.0 |
| 2024-09 |  |  |  |  |  | After editHelp cleanup | 1.0.1 |
| 2024-05 | SA5#155 | S5-243210 | - | - | - | Rel-19 pCR TR 28.871 Mgmt Model and NRM Decoupling | 1.1.0 |
| 2024-10 | SA5#107 | S5-246061 | - | - | - | pCR TR 28.871 Rapporteur clean-up.docx | 1.1.0 |
| 2024-10 | SA5#107 | S5-245294 | - | - | - | pCR TR 28.871 Add SBMA concepts and background | 1.1.0 |
| 2024-10 | SA5#107 | S5-245295 | - | - | - | pCR TR 28.871 Add evaluation and conclusion for Support IOCs in SBMA | 1.1.0 |
| 2024-10 | SA5#107 | S5-245351 | - | - | - | pCR TR 28.871 Add conclusion for schema reference enhancements | 1.1.0 |
| 2024-10 | SA5#107 | S5-246060 | - | - | - | pCR TR 28.871 Add conclusion for Overview of management capabilities and corresponding Solution Sets | 1.1.0 |
| 2024-10 | SA5#107 | S5-246062 | - | - | - | pCR TR 28.871 Add evaluation of potential solutions (schema retrieval support) | 1.1.0 |
| 2024-10 | SA5#107 | S5-246063 | - | - | - | Rel-19 pCR TR 28.871 Reliable notification transfer | 1.1.0 |
| 2024-10 | SA5#107 | S5-246064 | - | - | - | pCR TR 28.871 Alarm definition - filtering | 1.1.0 |
| 2024-10 | SA5#107 | S5-246065 | - | - | - | pCR TR 28.871 Update solution for Discovery of management capabilities of MnS supported by a MnS Producer | 1.1.0 |
| 2024-10 | SA5#107 | S5-246066 | - | - | - | pCR TR 28.871 Usage of MnS Registry NRM fragment for MnS Registry and Discovery | 1.1.0 |
| 2024-10 | SA5#107 | S5-246067 | - | - | - | pCR TR 28.871 Add use case, reqs, potential solutions and evaluation (management model decoupling) | 1.1.0 |