| 3GPP TR 28.876 V1.1.0 (2024-10) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on management aspect of RedCap feature (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	8

3.3	Abbreviations	8

4	Concepts and Background	8

5	Use Cases	8

5.1	Use Case#1: NG-RAN configuration to support RedCap and Non-RedCap UEs co-existence scenario	8

5.1.1	Description	8

5.1.2	Potential requirements	9

5.1.3	Potential Solutions	10

5.1.3.1	Potential solution #1	10

5.1.4	Evaluation of potential solutions	10

5.2	Use case #2: Monitoring EE KPI for RedCap service	10

5.2.1	Description	10

5.2.2	Potential requirements	11

5.3	Use case #3: Monitoring of RRC connection number for RedCap service	11

5.3.1	Description	11

5.3.2	Potential requirements	11

5.3.3	Potential Solutions	11

5.3.3.1	Potential solution #1	11

5.3.4	Evaluation of potential solutions	12

5.4	Use case #4: Monitoring of throughput for RedCap service	12

5.4.1	Description	12

5.4.2	Potential requirements	12

5.4.3	Potential Solutions	12

5.4.3.1	Potential solution #1	12

5.4.4	Evaluation of potential solutions	13

5.5	Use case #5: RedCap Information Exposure	13

5.5.1	Description	13

5.5.2	Potential requirements	13

5.5.3	Potential Solutions	13

5.5.3.1	Potential solution #1	13

5.5.4	Evaluation of potential solutions	14

5.6	Use case #6:  Performance evaluation on resource load of RedCap network service	14

5.6.1	Description	14

5.6.2	Potential requirements	14

5.6.3	Potential Solutions	14

5.6.3.1	Potential solution #1	14

5.X	Use case#A: title	15

5.X.1	Description	15

5.X.2	Potential requirements	15

5.X.3	Potential solutions	15

5.X.3.1	Solution1	15

5.X.3.2	Solution2	15

5.X.3.Y	Solutionx	15

5.X.4	Evaluation of potential solutions	15

6	Conclusions and recommendations	15

6.1	Use case #1: Multi-Initial BWP under RedCap and Non-RedCap UEs co-existence scenario	15

6.2	Use case #3: Monitoring of RRC connection number for RedCap service	16

6.3	Use case #4: Monitoring of throughput for RedCap service	16

6.4	Use case #5: RedCap Information Exposure	16

6.5	Use case #6: Performance evaluation on resource load of RedCap network service	16

Annex A: Change history	17

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document describes use cases, potential requirements, and solutions for the management aspects of RedCap feature.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[3]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[4]	3GPP TS 38.213: "NR; Physical layer procedures for control".

[5]	3GPP TR 38.865: "Study on further NR RedCap UE complexity reduction".

[6]	3GPP TR 38.875: "Study on support of reduced capability NR devices".

[7]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[8]	3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains".

[9]	3GPP TR 22.804: "Study on Communication for Automation in Vertical domains (CAV)".

[10]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[11]	3GPP TR 28.879: "Study on OAM for service management and exposure to external consumers".

[12]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[13]	3GPP TS 38.304: "User Equipment (UE) procedures in Idle mode and RRC Inactive state".

[14]	3GPP TS 38.306: "User Equipment (UE) radio access capabilities".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1]. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

RedCap UE: ref to 3GPP TS 38.331 [2], A UE with reduced capabilities as specified in clause 4.2.21.1 in TS 38.306 [14].

eRedCap UE: ref to 3GPP TS 38.331 [2], A UE with enhanced reduced capabilities as specified in clause 4.2.22.1 in TS 38.306 [14], ref to 3GPP TS 38.331 [2].

## 3.2	Symbols

Void.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1]. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

# 4	Concepts and Background

RedCap is a lightweight network access solution aiming at scenarios with relatively low cost, low energy consumption, and low data rate requirement, mainly applying to scenarios such as industrial sensors, video surveillance, wearable scenarios, smart grid, etc.

NOTE:	The term RedCap UE used in the present document refers to both RedCap UE and eRedCap UE defined in 3GPP TS 38.331 [2].

During Rel-17 and Rel-18, lots of normative works have been finished in RAN and CN to support the RedCap features in 3GPP system, bringing some new requirements in management aspects.

For example, access bandwidth of RedCap UEs is reduced from conventional UEs, so the management needs adapt accordingly. Besides, different types of RedCap UEs require different capabilities and features. In the aspect of performance, the key performance of RedCap service is different from the services such as eMBB, URLLC and MIoT (higher than MIoT but lower than URLLC and eMBB). In the aspect of energy consumption, 3GPP has introduced some complexity reduction features (such as bandwidth reduction and eDRX) for RedCap. These new features may have some impacts on the energy consumption of network which should be evaluated separately.

Hence, corresponding performance measurements, KPIs, feature management are supposed to be studied.

# 5	Use Cases

## 5.1	Use Case#1: NG-RAN configuration to support RedCap and Non-RedCap UEs co-existence scenario

### 5.1.1	Description

RedCap is a lightweight network access solution aiming at scenarios with low-cost, low-power, and low-speed requirements, mainly applying to industrial sensors, video surveillance, and wearable scenarios. Access bandwidth of RedCap UEs is different from conventional UEs, so management when bandwidth reducing is supposed to be studied. In clause 6.3.2 radio resource control information elements of 3GPP TS 38.331 [2], the DownlinkConfigCommon and UplinkConfigCommon provide initialDownlinkBWP-RedCap and initialUplinkBWP-RedCap parameters, respectively.

The IE DownlinkConfigCommon provides common downlink parameters of a cell in 3GPP TS 38.331 [2], as followed.

DownlinkConfigCommon field descriptions

frequencyInfoDL Basic parameters of a downlink carrier and transmission thereon.

initialDownlinkBWP The initial downlink BWP configuration for a serving cell. The network configures the locationAndBandwidth so that the initial downlink BWP contains the entire CORESET#0 of this serving cell in the frequency domain.

initialDownlinkBWP-RedCap If present, (e)RedCap UEs use this DL BWP instead of initialDownlinkBWP. If absent, (e)RedCap UEs use initialDownlinkBWP provided that it does not exceed the (e)RedCap UE maximum bandwidth (see 3GPP TS 38.331 [2], clause 5.2.2.4.2).

The IE UplinkConfigCommon provides common uplink parameters of a cell in 3GPP TS 38.331 [2], as followed.

UplinkConfigCommon field descriptions

frequencyInfoUL Absolute uplink frequency configuration and subcarrier specific virtual carriers.

initialUplinkBWP The initial uplink BWP configuration for a serving cell (see 3GPP TS 38.213 [4], clause 12).

initialUplinkBWP-RedCap If present, (e)RedCap UEs use this UL BWP instead of initialUplinkBWP. If absent, (e)RedCap UEs use initialUplinkBWP provided that it does not exceed the (e)RedCap UE maximum bandwidth (see 3GPP TS 38.331 [2], clause 5.2.2.4.2).

An initial BWP can be configured for RedCap UEs' initial access instead of initial BWP that may exceed the (e)RedCap UE maximum bandwidth. However, in 3GPP TS 28.541 [3], only two types of BWP are defined: INITIAL, OTHER, and the INITIAL-REDCAP type is missing. So some potential solutions need to be studied to support Multi-Initial BWP under RedCap and Non-RedCap UEs co-existence scenario.

SIB1 in 3GPP TS 38.331 [2] contains information relevant when evaluating if a UE is allowed to access a cell and defines the scheduling of other system information. It also contains radio resource configuration information that is common for all UEs and barring information applied to the unified access control. Following SIB1 IEs are provided for RedCap UEs in 3GPP TS 38.331 [2]

Following Cell Status is provided in 3GPP TS 38.304 [13]

Cell status and cell reservations are indicated in the MIB or SIB1 message as specified in TS 38.331 [2] by means of following fields:

-	cellBarredRedCap1Rx (IE type: "barred" or "not barred")
Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is common for all PLMNs and NPNs. This field is only applicable to RedCap UEs.

-	cellBarredRedCap2Rx (IE type: "barred" or "not barred")
Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is common for all PLMNs and NPNs. This field is only applicable to RedCap UEs.

Based on above information, a RedCap UE can treat the cell as barred (i.e. UE considers that the cell doesn’t not support RedCap) or as not barred (i.e. UE considers that the cell support RedCap).

In some scenarios, operator may want to prohibit RedCap UEs to access specific NR cells for a specified time to improve the network performance for non-RedCap UEs. After the specified time, operator will allow RedCap UEs to access specific NR cells again. So, the 3GPP management system should have the capability to allow operators to configure the NR cell to allow or prohibit RedCap UEs to access.

### 5.1.2	Potential requirements

REQ-NG-RAN_Config-1: The 3GPP management system should have the capability to support Multi-Initial BWP configuration to support RedCap and Non-RedCap UEs co-existence scenario.

REQ-NG-RAN_Config-2: The 3GPP management system should have the capability to configure initial BWP separately for RedCap UEs.

REQ-NG-RAN_Config-3: The 3GPP management system should have the capability to configure the NR cell to allow or prohibit RedCap UEs to access.

### 5.1.3	Potential Solutions

#### 5.1.3.1	Potential solution #1

To configure Multi-Initial BWP under RedCap and Non-RedCap UEs co-existence scenario, the existed isInitialBwp attribute can be modified and configured for RedCap, or other UEs.

1)	The potential NRM changes to support above could include:

The new "INITIAL-REDCAP" added to the allowedValues of isInitialBwp attribute of BWP IOC:

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| isInitialBwp | It identifies whether the object is used for initial or other BWP. allowedValues: INITIAL, INITIAL-REDCAP, OTHER | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

2)	A BWP can be either an initial BWP used for initial access, or other ("regular") BWP configured for relevant UEs that support the BWP's characteristics. An initial BWP can be configured for RedCap UEs, or other UEs.

To support the capability to configure the NR cell to allow or prohibit RedCap UEs to access, the NRCellDU IOC defiend in TS 28.541[3] needs to be enhanced by adding following attribute:

-	“redCapcellFlag”, this attribute represents whether the NR Cell is supporting RedCap UE accessing. The type of this attribute is Boolean. In case the value is True, the NR Cell is supporting RedCap UE accessing (i.e. allow RedCap UEs to access the NR Cell). Otherwise, the NR Cell is not supporting RedCap UE accessing (i.e., prohibit RedCap UEs to access the NR Celll).

### 5.1.4	Evaluation of potential solutions

The possible solution described in clause 5.1.3.1:

-	Enhances the allowedValues of isInitialBwp attribute of BWP IOC with a new allowedValue INITIAL-REDCAP indicating that an initial BWP can be configured for RedCap UEs.

-	Add “redCapcellFlag” in NRCellDU IOC to represent whether the NR Cell is supporting RedCap UE accessing.

The change is lightweight and it is backward compatible, therefore it is a feasible solution.

The attribute “redCapcellFlag” needs to be revisited during normative phase.

## 5.2	Use case #2: Monitoring EE KPI for RedCap service

### 5.2.1	Description

RedCap is a lightweight network access solution aiming at scenarios with low-cost, low-power, low-energy-consumption and low-speed requirements, mainly applying to industrial sensors, video surveillance, wearable scenarios.

As stated in 3GPP TS 28.554 [7], clause 6.7, the network Energy Efficiency (EE) KPI is related to the performance and the energy consumption of the network providing the specific service. When network delivers communication services for RedCap UEs, there has some impacts on the evaluation of EE:

-	In the aspect of performance, the key performance of RedCap service is different from the services such as eMBB, URLLC and MIoT (higher than MIoT but lower than URLLC and eMBB).

-	In the aspect of energy consumption, 3GPP has introduced some complexity reduction features (such as bandwidth reduction and eDRX) for RedCap. These new features may have some impacts on the energy consumption of network which should be evaluated separately.

However, 3GPP TS 28.554 [7] only provides EE KPIs for eMBB, URLLC and MIoT. How to evaluate the EE for RedCap service should be investigated.

### 5.2.2	Potential requirements

REQ-RedCap-Perf-EE-1: The 3GPP management system should have capability to evaluate the energy efficiency for RedCap service.

## 5.3	Use case #3: Monitoring of RRC connection number for RedCap service

### 5.3.1	Description

Industry sensors scenario is a typical use case of RedCap technology. It's desirable in this scenario to connect different kinds of sensors to 5G network as mentioned in 3GPP TR 38.875 [6]. According to 3GPP TS 22.104 [8] and 3GPP TR 22.804 [9], this use case has a requirement on UE density to ensure enough number of devices enjoying communication service.

RRC connection number is an indispensable performance measurements for 5G NR, indicating the number of UEs connected to gNB simultaneously. It can reflect NR performances to operators.

The current measurements related to RRC connection number specified in 3GPP TS 28.552 [10] are all performed without recognizing the UE type. Therefore, it is difficult to accurately determine the number of RedCap UEs connected to NR, especially when different type of UEs co-exist. Consequently, it is important to investigate how to define measurement for RRC connection number of RedCap UEs. It will assist operators in understanding the resource load brought by RedCap and enable dynamic resource allocation.

The measurements of inactive RRC connection number for RedCap UEs can help operators better understand the status of RedCap UEs in RRC inactive mode. Monitoring of inactive RRC connection number for RedCap UEs can help operators use this information for dynamic frequency resource allocation or load balance purpose.

Monitoring of RRC connection resuming for RedCap UEs, can help operators optimize connection recovery process and improve user experience. The success or failure of a RRC connection resuming directly impacts the quality level for delivering the service by the networks and user experience.

### 5.3.2	Potential requirements

REQ-RedCap_Perf_RRCNum-1: The 3GPP management system should have capability to provide measurements related to RRC connection number for RedCap UEs in NR network.

REQ-RedCap-Perf-RRCNum-2: The 3GPP management system should have capability to provide measurements related to inactive RRC connection number for RedCap UEs in NR network.

REQ-RedCap-Perf-RRCNum-3: The 3GPP management system should have capability to provide measurements related to RRC connection resuming for RedCap UEs in NR network.

### 5.3.3	Potential Solutions

#### 5.3.3.1	Potential solution #1

This solution proposes to reuse and enhance the existing measurements related to RRC connection number defined in 3GPP TS 28.552 [10] for this use case.

Current measurements about RRC connection number includes Mean number of RRC Connections, Max number of RRC Connections, Mean number of stored inactive RRC Connections, Max number of stored inactive RRC Connections, Number of RRC connection resuming attempts, Successful RRC connection resuming etc. In order to achieve the requirement in clause 5.3.2, most aspects of the existing measurements can be reused and some enhancements also need to be introduced.

In radio access procedure in 5G system, UEs (no matter non-Redcap UEs or RedCap UEs) need to report to gNB about the NR UE Radio Access Capability Parameter using UE-NR-Capability IE, which is specified in clause 6.3.3 in 3GPP TS 38.331 [2]. RedCapParameters is part of UE-NR-Capability IE which can be delivered by the UE capability inquiry process.

Based on the procedures above, gNB can be aware of whether the UE is RedCap or not when a UE tries to access to NG-RAN. Consequently, the measurements that performed after the inquiry of UE capability can be split into subcounters per UE type.

When there is more than one type of UEs (e.g. RedCap UEs, eMBB UEs) covered by a cell, the filter needs to be enhanced to optionally split the measurement into subcounters to represent the RRC connection number for RedCap UEs.

Take the Mean number of RRC Connection as an example:

-	Introduce a new filter which can be named as NewFilter, then RRC.ConnMean can be optionally split as RRC.ConnMean.NewFilter to represent RRC connection number for RedCap UEs when the value of NewFilter is RedCap.

The subcounters filtered by the new filter with the value of RedCap can be used as measurements of RedCap RRC connection number.

### 5.3.4	Evaluation of potential solutions

The possible solution described in clause 5.3.3.1 enhances the existing measurements related to RRC connection number defined in 3GPP TS 28.552 [10]. A new filter is introduced, which can be optionally used to separate the measurements into subcounters to represent RedCap UEs number. The change is lightweight and and largely reuses existing measurements.

In summary, the solution described in clause 5.3.3.1 is feasible.

## 5.4	Use case #4: Monitoring of throughput for RedCap service

### 5.4.1	Description

RedCap (Reduced Capability) represents a lightweight access technology within the 5G NR network, designed to satisfy the requirements for cost-effective, energy-efficient, and moderate-data-rate solutions. Some typical use cases, such as industrial sensors, video surveillance, wearables, etc., have been specified in 3GPP TR 38.865 [5], primarily applied in verticals to facilitate the digital transformation.

UE experienced data rate is an essential and important performance indicator for communication services. Compared to non-Redcap UEs, RedCap UEs have a lower cost and streamlined capabilities. For instance, in terms of average DL data rate which is measured by throughput, the requirement for legacy eMBB UEs is mostly more than 100 Mbps while that for RedCap UE is ranging from less than 2 Mbps to 50 Mbps. In co-existence scenario, average throughput is calculated combining the two types of UEs together in the current approach specified in 3GPP TS 28.552 [10]. The result may not reflect the network performance of RedCap accurately.

Consequently, it is crucial to investigate an approach for separately calculating the data rate of RedCap UEs and defining the throughput performance of networks that deliver RedCap services. This will ensure a more precise evaluation of the user experience for RedCap UEs.

### 5.4.2	Potential requirements

REQ-RedCap-Perf-Throughput-1: The 3GPP management system should have capability to provide measurements related to throughput for RedCap UEs in NR network.

### 5.4.3	Potential Solutions

#### 5.4.3.1	Potential solution #1

This solution proposes to reuse and enhance the existing measurements related to UE throughput defined in 3GPP TS 28.552 [10] for this use case.

Current measurements about UE throughput includes Average DL UE throughput in gNB, Distribution of DL UE throughput in gNB, Average UL UE throughput in gNB, Distribution of UL UE throughput in gNB, etc. In order to achieve the requirement in clause 5.4.2, most aspects of the existing measurements can be reused and some enhancements also need to be introduced.

In radio access procedure in 5G system, UEs (no matter legacy UEs or RedCap UEs) need to report to gNB about the NR UE Radio Access Capability Parameter using UE-NR-Capability IE, which is specified in clause 6.3.3 in 3GPP TS 38.331 [2]. RedCapParameters is part of UE-NR-Capability IE which can be delivered by the UE capability inquiry process.

Based on the procedures above, gNB can be aware of whether the UE is RedCap or not when a UE tries to access to NG-RAN. Consequently, the measurements that performed after the inquiry of UE capability can be separated for different types of UEs.

When there is more than one type of UEs (e.g. RedCap UEs, eMBB UEs) covered by a cell, the filter needs to be enhanced to optionally separate the measurement into subcounters to represent the RedCap UE throughput.

Take the Average DL UE throughput in gNB as an example:

-	Introduce a new filter which can be named as NewFilter, then DRB.UEThpDl can be optionally separate as DRB.UEThpDl.NewFilter to represent RedCap UE throughput when the value of NewFilter is RedCap.

The subcounters filtered by the new filter with the value of RedCap can be used as metrics of RedCap UE throughput.

### 5.4.4	Evaluation of potential solutions

The possible solution described in clause 5.4.3.1 enhances the existing measurements related to UE throughput in 3GPP TS 28.552 [10]. Measurements related to RedCap throughput will create a subcounter. The change is lightweight and largely reuses existing measurements.

In summary, the solution described in clause 5.4.3.1 is feasible.

## 5.5	Use case #5: RedCap Information Exposure

### 5.5.1	Description

RedCap is a lightweight network access solution aiming at scenarios with low-cost, low-power, low-energy-consumption and low-speed requirements, mainly applying to industrial sensors, video surveillance, wearable scenarios. To enabling the CSC to provide more customized and efficient services, the Communication Service Provider(CSP) needs to expose certain network information with accessed RedCap to the CSC. However, the specific information needs of CSC vary under different RedCap use cases and deployment scenarios.

For instance, in industrial sensor scenarios, the CSC (industrial enterprise) needs the CSP to monitor and expose network performance indicators related to RedCap (e.g. throughput, latency, packet loss rate), and dynamically adjust RedCap network configuration (e.g. bandwidth, resource allocation) based on production demands.

In video surveillance scenarios, the CSC (security company) also needs the CSP to monitor and expose network performance indicators related to RedCap and dynamically adjust network configuration based on surveillance demands. Detail on exposure will not be handled by this study, see 3GPP TR 28.879 [11] FS_MEXPO for general aspect of exposure mechanism.

### 5.5.2	Potential requirements

REQ-RedCap-Info-Exposure-1: The 3GPP management system may be capable of providing RedCap management data to the CSC based on different RedCap use cases and deployment scenarios.

### 5.5.3	Potential Solutions

#### 5.5.3.1	Potential solution #1

This solution proposes to reuse the general aspect of exposure mechanism in 3GPP TR 28.879 [11] for this use case.

NOTE:	The exposure mechanism is under study phase in study item FS_MEXPO (3GPP TR 28.879 [11]) and has not been finished, the detail will be discussed in FS_MEXPO instead of here.

### 5.5.4	Evaluation of potential solutions

This potential solution reuses the general aspect of exposure mechanism in study item FS_MEXPO (3GPP TR 28.879 [11]). No need to further investigated within this study item.

## 5.6	Use case #6:  Performance evaluation on resource load of RedCap network service

### 5.6.1	Description

The radio resource utilization measurements could provide operators the load information of the radio network during the measurement time period. Compared to non-Redcap UEs, RedCap UEs use fewer network resources (e.g. PRBs) relative to eMBB (enhanced mobile broadband) devices due to their reduced capabilities and lower data rate requirements. PRBs related measurements may include total PRB usage, PDSCH PRB Usage, PUSCH PRB Usage, etc.

In 3GPP TS 28.552 [10], radio resource utilization metrics are used to evaluate the performance of the entire network, but the current approach combines the resource utilization of RedCap UEs and non-RedCap UEs, which may not accurately reflect the performance of the RedCap service.

RedCap UEs typically have reduced capabilities and different performance characteristics compared to standard UEs.

Compared to non-Redcap UEs, RedCap UEs use fewer network resources (e.g. PRBs) related to eMBB (enhanced mobile broadband) devices. By distinguishing between the two types of UEs, network operators can better identify and troubleshoot specific network resource issues related to RedCap UEs and non-RedCap UEs, thus can allocate resources more efficiently and ensure that RedCap UEs receive adequate resource.

Therefore, it is essential to investigate an approach for separately calculating the resource load of RedCap UEs and defining the resource load performance of networks that deliver RedCap service. This will ensure a more precise evaluation of the user experience for RedCap UEs, thus make the operator to be aware of whether a cell has ever experienced high load or not in the monitoring period, and is a key input to network capacity planning and load balancing for RedCap service.

Similarly, to distinguish RedCap UEs and non-RedCap UEs for metric of the number of the active UEs in each cell is helpful for operators to know how many DRBs are running with buffered data per cell for different kinds of UEs.

Through the metrics of radio resource utilization and active users of RedCap UEs and non-RedCap UEs, operators can better evaluate the overall consumption of network resources of RedCap UEs to facilitate adjustment of related network policies.

### 5.6.2	Potential requirements

REQ-RedCap-Perf-Resource-1: The 3GPP management system should have capability to provide measurements related to resource load of RedCap service.

### 5.6.3	Potential Solutions

#### 5.6.3.1	Potential solution #1

This solution proposes to reuse and enhance the existing measurements related to radio resource utilization defined in 3GPP TS 28.552 [10] for this use case.

Current PRBs related measurements are specified in clause 5.1.1.2 in 3GPP TS 28.552 [10], including DL Total PRB Usage, UL Total PRB Usage, PRB Usage per SSB, etc. In order to achieve the requirement in clause 5.6.2, most aspects of the existing measurements can be reused and some enhancements also need to be introduced.

Also, clause 5.1.1.23 in 3GPP TS 28.552 [10] Number of Active UEs can also be reused to reflect the load of RedCap service including:

-	Active UEs in the DL per cell.

-	Active UEs in the UL per cell.

-	Active UEs per cell.

-	Etc.

Based on the procedures above, gNB can be aware of whether the UE is RedCap or not when a UE tries to access to NG-RAN. Consequently, the measurements that performed after the inquiry of UE capability can be split into subcounters per UE type.

When there is more than one type of UEs (e.g. RedCap UEs, eMBB UEs) covered by a cell, the measurements can be optionally split into subcounters to represent PRB number for RedCap UEs.

The related measurements will create a subcounter which can be named as per UE type.

Take the DL Total PRB Usage as an example:

-	RRU.PrbDl.UeType

Where UeType indicates the type of UEs (e.g. RedCap UEs, non-RedCap UE, etc.) and UeType the value of RedCap can be used as measurements of PRB number for RedCap UEs.

### 5.6.4	Evaluation of potential solutions

The possible solution described in clause 5.6.3.1 enhances the existing measurements related to radio resource load in 3GPP TS 28.552 [10]. Measurements related to radio resource load of RedCap service will create a subcounter. The change is lightweight and largely reuses existing measurements.

In summary, the solution described in clause 5.6.3.1 is feasible.

## 5.X	Use case#A: title

### 5.X.1	Description

### 5.X.2	Potential requirements

### 5.X.3	Potential solutions

#### 5.X.3.1	Solution1

#### 5.X.3.2	Solution2

#### 5.X.3.Y	Solutionx

### 5.X.4	Evaluation of potential solutions

# 6	Conclusions and recommendations

## 6.1	Use case #1: Multi-Initial BWP under RedCap and Non-RedCap UEs co-existence scenario

This use case identifies that access bandwidth of RedCap UEs is different from conventional UEs. An initial BWP can be configured for RedCap UEs' initial access instead of initial BWP that may exceed the (e)RedCap UE maximum bandwidth. In 3GPP TS 38.331 [2], the DownlinkConfigCommon and UplinkConfigCommon provide initialDownlinkBWP-RedCap and initialUplinkBWP-RedCap parameters, respectively. However, in 3GPP TS 28.541 [3], only two types of BWP are defined: INITIAL, OTHER, and the INITIAL-REDCAP type is missing. In order to address this issue, the potential solution proposes an approach to configure Multi-Initial BWP under RedCap and Non-RedCap UEs co-existence scenario, and the existed isInitialBwp attribute can be modified and configured for RedCap, or other UEs. Detailed description about this solution is in clause 5.1.3.

It is recommended to make some enhancements on management of RedCap related initial BWP according to this solution in the future normative work to satisfy the requirements for the access bandwidth of RedCap UEs.

## 6.2	Use case #3: Monitoring of RRC connection number for RedCap service

This use case identifies that existing cell-level measurements can hardly determine the number of RedCap UEs covered by 5G NR accurately, because the current measurements related to UE numbers in 3GPP TS 28.552 [10] are all performed without recognizing UE type. To address this issue, the potential solution proposes a new approach to enhance the existing measurements by separating the RedCap UEs. Detailed description about this solution is shown in clause 5.3.3.

It is recommended to make some enhancements on the measurements related to RRC number management according to this solution in the future normative work to satisfy the requirements for the evaluation of RedCap UE number and density.

## 6.3	Use case #4: Monitoring of throughput for RedCap service

This use case identifies that existing cell-level measurements related to throughput may not accurately reflect the performance of network delivering RedCap services, especially in co-existence scenarios. This is because these measurements combine all types of UEs together. To address this issue, the potential solution proposes a new approach to enhance the existing measurements by filtering the RedCap UEs. Detailed description about this solution is shown in clause 5.4.3.

It is recommended to make some enhancements on throughput performance management related to RedCap service according to this solution in the future normative work.

## 6.4	Use case #5: RedCap Information Exposure

This use case related mechanism is already under study in study item FS_MEXPO. This report does not recommend standardization of the potential solution.

## 6.5	Use case #6: Performance evaluation on resource load of RedCap network service

This use case identifies that existing cell-level measurements related to radio resource load may not accurately reflect the performance of network delivering RedCap services, especially in co-existence scenarios. This is because these measurements combine all types of UEs together. To address this issue, the potential solution proposes a new approach to enhance the existing measurements by filtering the RedCap UEs. Detailed description about this solution is shown in clause 5.6.3.

It is recommended to make some enhancements on radio resource load performance management related to RedCap service according to this solution in the future normative work.

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-04 | SA5#154 | TR 28.876-000 | - | - | - | Initial skeleton | 0.0.0 |
| 2024-04 | SA5#154 | S5-241681 | - | - | - | Add scope | 0.1.0 |
| 2024-04 | SA5#154 | S5-241684 | - | - | - | Add use case, requirements and potential solution for Multi-Initial BWP under RedCap and Non-RedCap UEs co-existence scenario | 0.1.0 |
| 2024-04 | SA5#154 | S5-241686 | - | - | - | Add use case requirements on EE KPI for RedCap .docx | 0.1.0 |
| 2024-04 | SA5#154 | S5-242123 | - | - | - | Add structure | 0.1.0 |
| 2024-04 | SA5#154 | S5-242124 | - | - | - | Add use case and potential requirement on metric of RedCap RRC connection number | 0.1.0 |
| 2024-04 | SA5#154 | S5-242125 | - | - | - | Add use case and potential requirement on throughput performance evaluation of RedCap | 0.1.0 |
| 2024-05 | SA5#155 | S5-242548 | - | - | - | Update the use cases and requirements | 0.2.0 |
| 2024-05 | SA5#155 | S5-243301 | - | - | - | Add use case and requirements on Inactive RedCap UEs | 0.2.0 |
| 2024-05 | SA5#155 | S5-243302 | - | - | - | Concepts and background | 0.2.0 |
| 2024-05 | SA5#155 | S5-243303 | - | - | - | Add potential solution for throughput performance evaluation of RedCap | 0.2.0 |
| 2024-05 | SA5#155 | S5-243304 | - | - | - | Add potential solution for metric of RedCap RRC connection number | 0.2.0 |
| 2024-05 | SA5#155 | S5-243306 | - | - | - | Add use case and potential requirements on RedCap information exposure | 0.2.0 |
| 2024-05 | SA5#155 | S5-243318 | - | - | - | Add use case and requirements on resource load of RedCap service | 0.2.0 |
| 2024-08 | SA5#156 | S5-243569 | - | - | - | Correction of some reference and editing errors | 0.3.0 |
| 2024-08 | SA5#156 | S5-243978 | - | - | - | Update Use case #3 Monitoring of RRC connection number for RedCap service | 0.3.0 |
| 2024-08 | SA5#156 | S5-244112 | - | - | - | Add evaluation, conclusion and recommendation for Use Case #1 | 0.3.0 |
| 2024-08 | SA5#156 | S5-244114 | - | - | - | Add evaluation, conclusion and recommendation for Use case #3 | 0.3.0 |
| 2024-08 | SA5#156 | S5-244119 | - | - | - | Update descriptions for TR 28.876 | 0.3.0 |
| 2024-08 | SA5#156 | S5-244812 | - | - | - | Update Use Case#1 Multi-Initial BWP under RedCap and Non-RedCap UEs co-existence scenario | 0.3.0 |
| 2024-08 | SA5#156 | S5-244814 | - | - | - | Add potential solutions on resource load of RedCap service | 0.3.0 |
| 2024-08 | SA5#156 | S5-244865 | - | - | - | Add evaluation, conclusion and recommendation for Use case #4 | 0.3.0 |
| 2024-08 | SA5#156 | S5-244866 | - | - | - | Add potential solution for RedCap Information Exposure | 0.3.0 |
| 2024-09 | SA#105 | SP-241131 |  |  |  | Presented at SA#105 for Information | 1.0.0 |
| 2024-09 |  |  |  |  |  | After editHelp cleanup | 1.0.1 |
| 2024-10 | SA5#157 | S5-245227 | - | - | - | Update some unclear descriptions | 1.1.0 |
| 2024-10 | SA5#157 | S5-246100 | - | - | - | Add evaluation and conclusions for use case #5 | 1.1.0 |
| 2024-10 | SA5#157 | S5-246101 | - | - | - | Update Use Case#1 NG-RAN configuration to support RedCap and Non-RedCap UEs co-existence scenario | 1.1.0 |
| 2024-10 | SA5#157 | S5-246102 | - | - | - | Add missing definitions and clarifications | 1.1.0 |
| 2024-10 | SA5#157 | S5-245724 | - | - | - | Update requirement and add potential solution for Inactive RedCap UEs | 1.1.0 |
| 2024-10 | SA5#157 | S5-245725 | - | - | - | Add conclusion and recommendation for resource load of RedCap service | 1.1.0 |