3GPP TSG CN WG1/ETSI SMG3 WPA	Tdoc N1-99259

Sophia Antipolis, France														                (Revision of Tdoc N1-99203)

April 21-29 1999

| CHANGE REQUEST No : |  |  |  |  |  | A651 R1 | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM / UMTS: |  |  |  | 04.08 |  | Version |  | 6.3.0 |  |  |  |
| Submitted to SMG | 29 |  | For approval |  | x | Without presentation ("non-strategic") |  |  |  |  |  |
| list plenary meeting or STC here |  | for information |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form. Filename: crf26_3.doc |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | x | Network | x |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | GPRS |
| --- | --- |

| Source: | Ericsson | Date: | 15-04-99 |
| --- | --- | --- | --- |

| Subject: | Paging with IMSI |
| --- | --- |

| Category: | F Correction | x | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 | x |
| and one release | C Functional modification of feature |  |  | Release 98 |  |
| only shall be | D Editorial modification |  |  | Release 99 |  |
| marked with an X) |  |  |  | UMTS |  |

| Reason for change: | This CR clarifies the Paging with IMSI procedure. |
| --- | --- |

| Clauses affected: | 4.7.9.1, 4.7.9.1.1 (new section), 4.7.9.1.2 (new section) |
| --- | --- |

| Other specs | Other releases of same spec |  | ® List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  | ® List of CRs: |  |
|  | MS test specifications / TBRs |  | ® List of CRs: |  |
|  | BSS test specifications |  | ® List of CRs: |  |
|  | O&M specifications |  | ® List of CRs: |  |

| Other comments: |  |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR

### 4.7.9	Paging procedure

#### 4.7.9.1	Paging for GPRS services

Paging for GPRS services is only permitted for mobiles which are currently attached for GPRS services.

Paging is used by the network to identify the cell the MS has currently selected, or to prompt the mobile to re-attach if necessary as a result of network failure.

##### 4.7.9.1.1 Paging for GPRS services using P-TMSI

The network shall initiate the paging procedure for GPRS services using P-TMSI when GMM signalling messages or user data is pending to be sent to the MS while the Mobile Reachable timer is running. The network may page only GPRS MSs which are GMM-REGISTERED and identified by a local P-TMSI.

To initiate the procedure the GMM entity requests the RR sublayer to start paging (see chapter 3 and GSM 04.60 [75]) and starts timer T3313. Upon reception of a paging indication, the MS shall respond to the paging (see GSM 04.07 [20] and GSM 03.60 [74]).

The network shall stop timer T3313 when a response is received from the MS and shall start the READY timer. When the timer T3313 expires the network may reinitiate paging.

##### 4.7.9.1.2 Paging for GPRS services using IMSI

Paging with IMSI is an abnormal procedure used for error recovery in the network. It shall never be done to initiate data or signalling message transfer.

The network shall only initiate paging using IMSI if the P-TMSI is not available due to a network failure.

To initiate the procedure the GMM entity in the network requests the RR sublayer to start paging (see chapter 3 and GSM 04.60 [75]) and starts timer T3313.

Upon reception of a paging indication, , the MS shall locally detach and deactivate any active PDP contexts. The local detach includes deleting any RAI, P-TMSI, P-TMSI signature and GPRS ciphering key sequence number stored, setting the GPRS update status to GU2 NOT UPDATED and  changing  state to GMM-DEREGISTERED.

After this he MS shall then perform a GPRS attach or combined GPRS attach procedure.

NOTE: After performing the attach, a mobile may activate PDP context(s) to replace any previously active PDP context(s)

The mobile does not response to the paging except with the Attach Request. Hence timer T3313 in the network will not be stopped and will always expire. When the timer T3313 expires the network should not reinitiate paging.

- 4.7.9.2	Paging for non-GPRS services

The network may initiate the paging procedure for non-GPRS services when the MS is IMSI attached for non-GPRS services. To initiate the procedure the GMM entity requests the RR sublayer to initiate paging (see chapter 3 and GSM 04.60 [75]) for non-GPRS services. The MS identity used for paging shall be the allocated TMSI if acknowledged by the MS, otherwise the IMSI.