ETSI SMG SMG3 WPA GPRS Ad-Hoc 	Tdoc N1-99413

Oslo, Norway

18-21 May 99

| CHANGE REQUEST No : |  |  |  |  | A061 |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM |  |  |  | 04.64 |  | Version: |  |  | 6.3.0 |  |  |  |
| Submitted to SMG | # 29 |  | for approval |  | x |  | without presentation ("non-strategic") |  |  |  |  |  |
| list SMG plenary meeting no. here |  | for information |  |  |  |  | with presentation ("strategic") |  |  |  | x |  |
| PT SMG CR cover form is available from: http://docbox.etsi.org/tech-org/smg/Document/smg/tools/CR_form/crf28_1.zip |  |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | x | Network | x |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | GPRS |
| --- | --- |

| Source: | Ericsson | Date: | 12 May 99 |
| --- | --- | --- | --- |

| Subject: | Differentiating between Layer 3 and LLC initiated procedures |
| --- | --- |

| Category: | F Correction | x | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 | x |
| and one release | C Functional modification of feature |  |  | Release 98 |  |
| only shall be | D Editorial modification |  |  | Release 99 |  |
| marked with an X) |  |  |  |  |  |

| Reason for change: | LLC’s behaviour following the receipt of an empty XID block from Layer 3 is not clear. This CR makes it clear that an LLE receiving an empty XID block from Layer 3 must include a zero length Layer 3 XID parameter to allow the receiving LLE to proceed correctly. |
| --- | --- |

| Clauses affected: | 6.4.1.6 |
| --- | --- |

| Other specs | Other releases of same spec |  | ® List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  | ® List of CRs: |  |
|  | MS test specifications / TBRs |  | ® List of CRs: |  |
|  | BSS test specifications |  | ® List of CRs: |  |
|  | O&M specifications |  | ® List of CRs: |  |

| Other comments: | If an originating LLE does not indicate whether the procedure it is executing is initiated by Layer 3 or not, the receiving LLE will be unable to determine the correct course of action. For example, an LLE receiving SABM must have an indication of whether the procedure is Layer 3 initiated to know whether to send LL-ESTABLISH-IND and wait for LL-ESTABLISH-RES (Figure 1) or to send UA immediately (Figure 2). The presence or absence of the Layer 3 XID field provides this indication. Figure 1: Layer 3-initiated ABM establishment procedure Figure 2: LLC-initiated ABM re-establishment procedure |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

#### 6.4.1.6	Exchange Identification (XID) command/response

This frame shall be used to negotiate and re-negotiate LLC layer parameters and layer-3 parameters. XID frames can be transmitted in ADM and ABM.

The negotiation procedure is one-step, i.e., one side shall start the process by sending an XID command, offering a certain set of parameters from the applicable parameter repertoire (see Table 6) the sending entity wants to negotiate, proposing values within the allowed range. In return, the other side shall send an XID response, either confirming these parameter values by returning the requested values, or offering higher or lower ones in their place. As an optimisation, parameters confirming the requested values may be omitted from the XID response. See Table 6 for sense of negotiation. This shall end the negotiation process.

Parameters that are not included in neither the XID command nor in the XID response, shall retain their current values.

The responding side may respond with parameters that were not included in the XID command. The parameters that were not included in the XID command shall in this case be treated as if the current values of the parameters were included in the XID command.

Both entities shall support the negotiated values, however under certain conditions one or more parameters may need to be re-negotiated (e.g., in the case of a change in SGSN).

XID frames shall always be used with the P/F bit set to 1.

Without any prior XID exchange, default values shall apply.

Negotiated XID parameters shall apply to the LLE identified by the DLCI of the XID frames used, except Version, Reset, and IOV-UI that applies to an LLME (i.e., a TLLI), and except Layer-3 Parameters that apply to the layer 3 above the LLE.

Table 6 lists the negotiable LLC layer parameters. Figure 11 shows the format of the XID parameter field.

Figure 11: XID parameter field format

A parameter item consists of one or two type/length octets followed by the value of that parameter. The XID Length (XL) bit indicates whether the Length field is 2 bits or 8 bits long. If XL is set to 0, then Length consists of 2 bits and type/length occupies one octet. If XL is set to 1 then Length consists of 8 bits and type/length occupies two octets. The length indicator gives the number of octets that the value actually occupies. The parameter items can be arranged in arbitrary order. The parameter items shall begin in the first octet of the XID information field and follow on contiguously.

Table 6: LLC layer parameter negotiation

| Parameter Name | Type | Length |  | Format (87654321) | Range | Units | Sense of Negotiation |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Version (LLC version number) | 0 | 1 |  | 0000bbbb | 0 through 15 | - | down |
| IOV-UI (ciphering Input offset value for UI frames), common for all SAPIs of a TLLI | 1 | 4 |  | bbbbbbbb bbbbbbbb bbbbbbbb bbbbbbbb | 0 through 232 - 1 | - | - |
| IOV-I (ciphering Input offset value for I frames), for the SAPI under negotiation | 2 | 4 |  | bbbbbbbb bbbbbbbb bbbbbbbb bbbbbbbb | 0 through 232 - 1 | - | - |
| T200 (retransmission time-out) | 3 | 2 |  | 0000bbbb bbbbbbbb | 1 through 4 095 | 0.1 seconds | up |
| N200 (maximum number of retransmissions) | 4 | 1 |  | 0000bbbb | 1 through 15 | - | up |
| N201-U (maximum information field length for U and UI frames) | 5 | 2 |  | 00000bbb bbbbbbbb | 140 through 1 520 | octets | down |
| N201-I (maximum information field length for I frames) | 6 | 2 |  | 00000bbb bbbbbbbb | 140 through 1 520 | octets | down |
| mD (I frame buffer size in the downlink direction) | 7 | 2 |  | 0bbbbbbb bbbbbbbb | 0 through 24 320 | 16 octets | down |
| mU (I frame buffer size in the uplink direction) | 8 | 2 |  | 0bbbbbbb bbbbbbbb | 0 through 24 320 | 16 octets | down |
| kD (window size in the downlink direction) | 9 | 1 |  | bbbbbbbb | 1 through 255 | frames | down |
| kU (window size in the uplink direction) | 10 | 1 |  | bbbbbbbb | 1 through 255 | frames | down |
| Layer-3 Parameters | 11 | Variable | See GSM 04.65 |  |  |  |  |
| Reset | 12 | 0 |  | - | - | - | - |
| - The Range for N201-U for SAPIs 1 and 7 is 270 through 1 520 octets. - All other Types and Ranges are reserved for future versions of the present document. - The length for Layer-3 Parameters shall be set equal to the number of octets received from layer 3. If an empty XID block is received from Layer 3, the LLE shall include a zero length Layer 3 XID parameter in the XID parameter field to allow the receiving LLE to distinguish between LLC and Layer 3 initiated procedures. |  |  |  |  |  |  |  |

Version shall not be negotiated while in ABM.

Reset shall only be negotiated with an XID frame, and only be transmitted in the downlink direction. If Reset is present in an XID frame, then it shall be the first XID parameter in the XID information field.

IOV-UI shall only be negotiated in ADM, and only before ciphering is enabled. IOV-I shall only be negotiated with SABM and UA frames. IOV-UI and IOV-I shall only be transmitted in the downlink direction.

T200, N200, and N201-U can be negotiated in ADM and ABM. The new values of T200 shall only apply to timers set after the negotiation has been completed.

N201-I, mD, mU, kD, and kU can be negotiated to any value in Range in ADM. In ABM, N201-I, mD, mU, kD, and kU can only be negotiated to the same or higher value as previously used.