ETSI STC SMG3 WPA GPRS Ad-Hoc	Tdoc N1-99447

Oslo, Norway	Revision of 3A99-094

18-21 May, 1999

| CHANGE REQUEST No : |  |  |  |  |  | A054r1 | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM / UMTS: |  |  |  | 04.64 |  | Version | 6.3.0 |  |  |  |
| Submitted to SMG | #29 |  | for approval |  | x | without presentation ("non-strategic") |  |  |  |  |
| list plenary meeting or STC here |  | for information |  |  |  | with presentation ("strategic") |  |  |  |  |
| PT SMG CR cover form. Filename: crf26_3.doc |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | x | Network | x |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | GPRS |
| --- | --- |

| Source: | Motorola | Date: | 14 May, 1999 |
| --- | --- | --- | --- |

| Subject: | Improving the user data loss-less inter-SGSN RA update |
| --- | --- |

| Category: | F Correction | x | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 | x |
| and one release | C Functional modification of feature |  |  | Release 98 |  |
| only shall be | D Editorial modification |  |  | Release 99 |  |
| marked with an X) |  |  |  | UMTS |  |

| Reason for change: | The cumbersome LLC V(R) transfer mechanism in the routeing area update procedure is replaced by the simpler SNDCP N-PDU number transfer mechanism. |
| --- | --- |

| Clauses affected: |  |
| --- | --- |

| Other specs | Other releases of same spec |  | ® List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  | ® List of CRs: | 03.60 A107, 04.07 A0xx, 04.08 A481, 04.65 A0yy, 09.60 A029 |
|  | MS test specifications / TBRs |  | ® List of CRs: |  |
|  | BSS test specifications |  | ® List of CRs: |  |
|  | O&M specifications |  | ® List of CRs: |  |

| Other comments: |  |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

### 7.1.2	Generic names

The generic name specifies the activity that the identified layer should perform. Table 1 lists the primitives defined in the present document.

Table 1: LLC layer service primitives

| Generic Name | Location |  | Type |  |  |  |  |  | Parameters |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MS | SGSN | REQ | IND |  | RES | CNF |  |  |  |
| LLGMM « LLM |  |  |  |  |  |  |  |  |  |  |
| LLGMM-ASSIGN | X | X | X |  |  |  |  |  | TLLI Old, TLLI New, Kc, Ciphering Algorithm |  |
| LLGMM-RESET |  | X | X |  |  |  | X |  | TLLI |  |
| LLGMM-TRIGGER | X |  | X |  |  |  |  |  | TLLI, Cause |  |
| LLGMM-SUSPEND | X |  | X |  |  |  |  |  | TLLI |  |
| LLGMM-SUSPEND |  | X | X |  |  |  |  |  | TLLI, Page |  |
| LLGMM-RESUME | X | X | X |  |  |  |  |  | TLLI |  |
| LLGMM-PAGE |  | X |  | X |  |  |  |  | TLLI |  |
| LLGMM-IOV |  | X | X |  |  |  | X |  | TLLI |  |
| LLGMM-STATUS | X | X |  | X |  |  |  |  | TLLI, Cause |  |
| GMM « LL, SNDCP « LL, and SMS « LL |  |  |  |  |  |  |  |  |  |  |
| LL-RESET | X | X |  | X |  |  |  |  | TLLI |  |
| LL-ESTABLISH | X | X | X |  |  |  |  |  | TLLI, XID Req |  |
| LL-ESTABLISH | X | X |  |  | X |  |  |  |  | TLLI, XID Req, N201-U, N201-I |
| LL-ESTABLISH | X | X |  |  |  | X |  |  |  | TLLI, XID Neg |
| LL-ESTABLISH | X | X |  |  |  |  |  | X |  | TLLI, XID Neg, N201-U, N201-I |
| LL-RELEASE | X | X | X |  |  |  |  |  |  | TLLI, Local |
| LL-RELEASE | X | X |  |  | X |  |  |  |  | TLLI, Cause |
| LL-RELEASE | X | X |  |  |  |  |  | X |  | TLLI |
| LL-XID | X | X | X |  |  |  |  |  |  | TLLI, XID Req |
| LL-XID | X | X |  |  | X |  |  |  |  | TLLI, XID Req, N201-U, N201-I |
| LL-XID | X | X |  |  |  | X |  |  |  | TLLI, XID Neg |
| LL-XID | X | X |  |  |  |  |  | X |  | TLLI, XID Neg, N201-U, N201-I |
| LL-DATA | X |  | X |  |  |  |  |  |  | TLLI, L3-PDU, Reference, QoS Parameters, Radio Priority |
| LL-DATA |  | X | X |  |  |  |  |  |  | TLLI, L3-PDU, Reference, QoS Parameters |
| LL-DATA | X | X |  |  | X |  |  |  |  | TLLI, L3-PDU |
| LL-DATA | X | X |  |  |  |  |  | X |  | TLLI, Reference |
| LL-UNITDATA | X |  | X |  |  |  |  |  |  | TLLI, L3-PDU, QoS Parameters, Radio Priority, Cipher |
| LL-UNITDATA |  | X | X |  |  |  |  |  |  | TLLI, L3-PDU, QoS Parameters, Cipher |
| LL-UNITDATA | X | X |  |  | X |  |  |  |  | TLLI, L3-PDU, Cipher |
| LL « RLC/MAC |  |  |  |  |  |  |  |  |  |  |
| GRR-DATA | X |  | X |  |  |  |  |  | TLLI, LL-PDU, SAPI, Cause, QoS Parameters, Radio Priority |  |
| GRR-DATA | X |  |  | X |  |  |  |  | TLLI, LL-PDU |  |
| GRR-UNITDATA | X |  | X |  |  |  |  |  | TLLI, LL-PDU, SAPI, QoS Parameters, Radio Priority |  |
| GRR-UNITDATA | X |  |  | X |  |  |  |  | TLLI, LL-PDU |  |
| LL « BSSGP |  |  |  |  |  |  |  |  |  |  |
| BSSGP-DL-UNITDATA |  | X | X |  |  |  |  |  | TLLI, LL-PDU, QoS Parameters, RLC Confirm, SAPI |  |
| BSSGP-UL-UNITDATA |  | X |  | X |  |  |  |  | TLLI, LL-PDU, Cell Id |  |

-

#### 7.2.1.7	LLGMM-IOV

LLGMM-IOV-REQ shall be used to order LLC in the SGSN to perform an XID negotiation of IOV-UI. The LLC layer shall randomly select the value of IOV-UI.

LLGMM-IOV-CNF shall be used to inform GMM in the SGSN that a successful XID negotiation of IOV-UI has been made.

#### 7.2.1.8	LLGMM-STATUS

LLGMM-STATUS-IND shall be used to inform GMM when an LLC error that cannot be corrected by the LLC layer has occurred.

#### 7.2.2.6	LL-UNITDATA

LL-UNITDATA-REQ shall be used to request the unconfirmed transmission of an L3-PDU to the peer. QoS Parameters in the SGSN includes precedence class, delay class, reliability class, and peak throughput. QoS Parameters in the MS includes peak throughput and reliability class. Reliability class indicates whether the UI frame carrying the L3-PDU shall be transmitted in protected or unprotected mode, and whether RLC/MAC acknowledged or unacknowledged mode shall be used. Radio Priority indicates the radio priority level to be used by RLC/MAC. Cipher indicates whether the UI frame shall be ciphered or not.

LL-UNITDATA-IND shall be used to deliver an L3-PDU received in a UI frame to layer 3. Cipher indicates whether the received UI frame was ciphered or not.