ETSI STC SMG3 WPA GPRS Ad-Hoc	Tdoc N1-99454

Oslo, Norway	Revision of Tdoc N1-99445

18-21 May, 1999

| CHANGE REQUEST No : |  |  |  |  | A107r2 |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM |  |  |  | 03.60 |  | Version: |  |  | 6.3.1 |  |  |  |
| Submitted to SMG | #29 |  | for approval |  | X |  | without presentation ("non-strategic") |  |  |  |  |  |
| list SMG plenary meeting no. here  |  | for information |  |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form is available from: http://docbox.etsi.org/tech-org/smg/Document/smg/tools/CR_form/crf28_1.zip |  |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | X | Network | X |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | GPRS |
| --- | --- |

| Source: | Motorola | Date: | 21 May, 1999 |
| --- | --- | --- | --- |

| Subject: | Improving the user data loss-less inter-SGSN RA update |
| --- | --- |

| Category: | F Correction | X | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 | X |
| and one release | C Functional modification of feature |  |  | Release 98 |  |
| only shall be | D Editorial modification |  |  | Release 99 |  |
|  |  |  |  | UMTS |  |
| marked with an X) |  |  |  |  |  |

| Reason for change: | At present, during inter-SGSN routeing area updates, data loss or duplication for PDP contexts requiring acknowledged transfer is avoided by the buffering of N-PDUs in SNDCP, the transfer of buffered N-PDUs from the old SGSN to the new SGSN, and the transfer of the LLC V(R) values between the MS, the old SGSN, and the new SGSN. There are some disadvantages about the V(R) transfer mechanism: - The LLC Reset XID parameter resets all LLC variables in the MS, including V(R). Therefore V(R) is not available when it is to be transfer to the new SGSN. - There is a mixing of protocol layers. SNDCP needs to understand V(S) and V(R) which are concepts in the LLC layer. With the introduction of the N-PDU number for acknowledged data transfer, it is possible to replace the V(R) transfer mechanism with a Receive N-PDU number transfer mechanism. The basic concept is that the Send N-PDU number and the Receive N-PDU number maintained in the MS and the SGSN shall not be reset even when an inter-SGSN RA update occurs. The new SGSN shall inherit these two parameters from the old SGSN. The Receive N-PDU number can then be transferred between the MS and the new SGSN in the Routeing Area Update Accept and Routeing Area Update Complete messages (see 04.08 CR A481) that will allow SNDCP in both the MS and the SGSN to delete any buffered N-PDUs that have been completely received. The changes required are summarized below: - The old SGSN shall stop assigning N-PDU numbers to the PDP contexts affected when an inter-SGSN RA update occurs. - The Send and Receive N-PDU number of every PDP context using acknowledged transfer shall be transferred from the old SGSN to the new SGSN in the SGSN Context Response message (see 09.60 CR A029). - Every buffered N-PDU shall be tunnelled from the old SGSN to the new SGSN together with its N-PDU number, if one has been assigned. - The new SGSN shall send its Receive N-PDU number to the MS in the Routeing Area Update Accept message, thus allowing the MS to delete the buffered N-PDUs that have been received by the old SGSN before the inter-SGSN RA update occurs. The MS shall send its Receive N-PDU number to the SGSN in the Routeing Area Update Complete message, thus allowing the SGSN to delete the buffered N-PDUs that have been received by the MS before the inter-SGSN RA update occurs. |
| --- | --- |

| Clauses affected: | 6.9.1.2.2, 6.9.1.3.2, 13.2, and 13.4. |
| --- | --- |

| Other specs | Other releases of same spec |  |  List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  |  List of CRs: | 04.07 A024, 04.08 A481, 04.64 A054, 04.65 A038, and 09.60 A029 |
|  | MS test specifications / TBRs |  |  List of CRs: |  |
|  | BSS test specifications |  |  List of CRs: |  |
|  | O&M specifications |  |  List of CRs: |  |

| Other comments: |  |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

##### 6.9.1.2.2	Inter SGSN Routeing Area Update

The Inter SGSN Routeing Area Update procedure is illustrated in Figure 1. Each step is explained in the following list.

Figure 1: Inter SGSN Routeing Area Update Procedure

1)	The MS sends a Routeing Area Update Request (old RAI, old P-TMSI Signature, Update Type) to the new SGSN. Update Type shall indicate RA update or periodic RA update. The BSS shall add the Cell Global Identity including the RAC and LAC of the cell where the message was received before passing the message to the SGSN.

2)	The new SGSN sends SGSN Context Request (old RAI, TLLI, old P-TMSI Signature, New SGSN Address) to the old SGSN to get the MM and PDP contexts for the MS. The old SGSN validates the old P-TMSI Signature and responds with an appropriate error cause if it does not match the value stored in the old SGSN. This should initiate the security functions in the new SGSN. If the security functions authenticate the MS correctly, the new SGSN shall send an SGSN Context Request (old RAI, TLLI, MS Validated, New SGSN Address) message to the old SGSN. MS Validated indicates that the new SGSN has authenticated the MS. If the old P-TMSI Signature was valid or if the new SGSN indicates that it has authenticated the MS, the old SGSN stops assigning SNDCP N-PDU numbers to downlink N-PDUs received, and responds with SGSN Context Response (MM Context, PDP Contexts). If the MS is not known in the old SGSN, the old SGSN responds with an appropriate error cause. The old SGSN stores New SGSN Address, to allow the old SGSN to forward data packets to the new SGSN. Each PDP Context includes the SNDCP Send N-PDU Number for the next downlink N-PDU to be sent in acknowledged mode to the MS, the SNDCP Receive N-PDU Number for the next uplink N-PDU to be received in acknowledged mode from the MS, the GTP sequence number for the next downlink N-PDU to be sent to the MS and the GTP sequence number for the next uplink N-PDU to be tunnelled to the GGSN. The old SGSN starts a timer and stops the transmission of N-PDUs to the MS.

3)	Security functions may be executed. These procedures are defined in subclause "Security Function". Ciphering mode shall be set if ciphering is supported.

4)	The new SGSN sends an SGSN Context Acknowledge message to the old SGSN. This informs the old SGSN that the new SGSN is ready to receive data packets belonging to the activated PDP contexts. The old SGSN marks in its context that the MSC/VLR association and the information in the GGSNs and the HLR are invalid. This triggers the MSC/VLR, the GGSNs, and the HLR to be updated if the MS initiates a routeing area update procedure back to the old SGSN before completing the ongoing routeing area update procedure. If the security functions do not authenticate the MS correctly, then the routing area update shall be rejected, and the new SGSN shall send a reject indication to the old SGSN. The old SGSN shall continue as if the SGSN Context Request was never received.

5)	The old SGSN duplicates the buffered N-PDUs and starts tunnelling them to the new SGSN. Additional N-PDUs received from the GGSN before the timer described in step 2 expires are also duplicated and tunnelled to the new SGSN. N-PDUs that were already sent to the MS in acknowledged mode and that are not yet acknowledged by the MS are tunnelled together with the SNDCP N-PDU number-. No N-PDUs shall be forwarded to the new SGSN after expiry of the timer described in step 2.

6)	The new SGSN sends Update PDP Context Request (new SGSN Address, TID, QoS Negotiated) to the GGSNs concerned. The GGSNs update their PDP context fields and return Update PDP Context Response (TID).

7)	The new SGSN informs the HLR of the change of SGSN by sending Update Location (SGSN Number, SGSN Address, IMSI) to the HLR.

8)	The HLR sends Cancel Location (IMSI, Cancellation Type) to the old SGSN with Cancellation Type set to Update Procedure. If the timer described in step 2 is not running, then the old SGSN removes the MM and PDP contexts. Otherwise, the contexts are removed only when the timer expires. This allows the old SGSN to complete the forwarding of N-PDUs. It also ensures that the MM and PDP contexts are kept in the old SGSN in case the MS initiates another inter SGSN routeing area update before completing the ongoing routeing area update to the new SGSN. The old SGSN acknowledges with Cancel Location Ack (IMSI).

9)	The HLR sends Insert Subscriber Data (IMSI, GPRS subscription data) to the new SGSN. The new SGSN validates the MS's presence in the (new) RA. If due to regional subscription restrictions the MS is not allowed to be attached in the RA, the SGSN rejects the Routeing Area Update Request with an appropriate cause, and may return an Insert Subscriber Data Ack (IMSI, SGSN Area Restricted) message to the HLR. If all checks are successful then the SGSN constructs an MM context for the MS and returns an Insert Subscriber Data Ack (IMSI) message to the HLR.

10)	The HLR acknowledges the Update Location by sending Update Location Ack (IMSI) to the new SGSN.

11)	The new SGSN validates the MS's presence in the new RA. If due to roaming restrictions the MS is not allowed to be attached in the SGSN, or if subscription checking fails, then the new SGSN rejects the routeing area update with an appropriate cause. If all checks are successful then the new SGSN constructs MM and PDP contexts for the MS. A logical link is established between the new SGSN and the MS. The new SGSN responds to the MS with Routeing Area Update Accept (P-TMSI, P-TMSI Signature, Receive N-PDU Number). Receive N-PDU Number contains the acknowledgements for each acknowledged-mode NSAPI used by the MS, thereby confirming all mobile-originated N-PDUs successfully transferred before the start of the update procedure.

12)	The MS acknowledges the new P-TMSI with a Routeing Area Update Complete (P-TMSI, Receive N-PDU Number). Receive N-PDU Number contains the acknowledgements for each acknowledged-mode NSAPI used by the MS, thereby confirming all mobile-terminated N-PDUs successfully transferred before the start of the update procedure. If Receive N-PDU Number confirms reception of N-PDUs that were forwarded from the old SGSN, then these N-PDUs shall be discarded by the new SGSN. LLC and SNDCP in the MS are reset.

In the case of a rejected routeing area update operation, due to regional subscription or roaming restrictions, the new SGSN shall not construct an MM context. A reject shall be returned to the MS with an appropriate cause. The MS shall not re-attempt a routeing area update to that RA. The RAI value shall be deleted when the MS is powered-up.

If the SGSN is unable to update the PDP context in one or more GGSNs, then the SGSN shall deactivate the corresponding PDP contexts as described in subclause "PDP Context Deactivation Initiated by SGSN Procedure". This shall not cause the SGSN to reject the routeing area update.

If the timer described in step 2 expires and no Cancel Location (IMSI) was received from the HLR, then the old SGSN shall stop forwarding N-PDUs to the new SGSN.

If the routeing area update procedure fails a maximum allowable number of times, or if the SGSN returns a Routeing Area Update Reject (Cause) message, the MS shall enter IDLE state.

##### 6.9.1.3.2	Combined Inter SGSN RA / LA Update

The Combined RA / LA Update (inter SGSN) procedure is illustrated in Figure 2. Each step is explained in the following list.

Figure 2: Combined RA / LA Update in the Case of Inter SGSN RA Update Procedure

1)	The MS sends a Routeing Area Update Request (old RAI, old P-TMSI Signature, Update Type) to the new SGSN. Update Type shall indicate combined RA / LA update, or, if the MS wants to perform an IMSI attach, combined RA / LA update with IMSI attach requested. The BSS shall add the Cell Global Identity including the RAC and LAC of the cell where the message was received before passing the message to the SGSN.

2)	The new SGSN sends SGSN Context Request (old RAI, TLLI, old P-TMSI Signature, New SGSN Address) to the old SGSN to get the MM and PDP contexts for the MS. The old SGSN validates the old P-TMSI Signature and responds with an appropriate error cause if it does not match the value stored in the old SGSN. This should initiate the security functions in the new SGSN. If the security functions authenticate the MS correctly, the new SGSN shall send an SGSN Context Request (old RAI, TLLI, MS Validated, New SGSN Address) message to the old SGSN. MS Validated indicates that the new SGSN has authenticated the MS. If the old P-TMSI Signature was valid or if the new SGSN indicates that it has authenticated the MS, the old SGSN stops assigning SNDCP N-PDU numbers to downlink N-PDUs received, and responds with SGSN Context Response (MM Context, PDP Contexts). If the MS is not known in the old SGSN, the old SGSN responds with an appropriate error cause. The old SGSN stores New SGSN Address until the old MM context is cancelled, to allow the old SGSN to forward data packets to the new SGSN. Each PDP Context includes the SNDCP Send N-PDU Number for the next downlink N-PDU to be sent in acknowledged mode to the MS, the SNDCP Receive N-PDU Number for the next uplink N-PDU to be received in acknowledged mode from the MS, the GTP sequence number for the next downlink N-PDU to be sent to the MS and the GTP sequence number for the next uplink N-PDU to be tunnelled to the GGSN. The old SGSN starts a timer and stops the downlink transfer.

3)	Security functions may be executed. These procedures are defined in subclause "Security Function". Ciphering mode shall be set if ciphering is supported.

4)	The new SGSN sends an SGSN Context Acknowledge message to the old SGSN. This informs the old SGSN that the new SGSN is ready to receive data packets belonging to the activated PDP contexts. The old SGSN marks in its context that the MSC/VLR association and the information in the GGSNs and the HLR are invalid. This triggers the MSC/VLR, the GGSNs, and the HLR to be updated if the MS initiates a routeing area update procedure back to the old SGSN before completing the ongoing routeing area update procedure. If the security functions do not authenticate the MS correctly, then the routing area update shall be rejected, and the new SGSN shall send a reject indication to the old SGSN. The old SGSN shall continue as if the SGSN Context Request was never received.

5)	The old SGSN duplicates the buffered N-PDUs and starts tunnelling them to the new SGSN. Additional N-PDUs received from the GGSN before the timer described in step 2 expires are also duplicated and tunnelled to the new SGSN. N-PDUs that were already sent to the MS in acknowledged mode and that are not yet acknowledged by the MS are tunnelled together with the SNDCP N-PDU number-. No N-PDUs shall be forwarded to the new SGSN after expiry of the timer described in step 2.

6)	The new SGSN sends Update PDP Context Request (new SGSN Address, TID, QoS Negotiated) to the GGSNs concerned. The GGSNs update their PDP context fields and return an Update PDP Context Response (TID).

7)	The new SGSN informs the HLR of the change of SGSN by sending Update Location (SGSN Number, SGSN Address, IMSI) to the HLR.

8)	The HLR sends Cancel Location (IMSI, Cancellation Type) to the old SGSN with Cancellation Type set to Update Procedure. If the timer described in step 2 is not running, then the old SGSN removes the MM and PDP contexts. Otherwise, the contexts are removed only when the timer expires. This allows the old SGSN to complete the forwarding of N-PDUs. It also ensures that the MM and PDP contexts are kept in the old SGSN in case the MS initiates another inter SGSN routeing area update before completing the ongoing routeing area update to the new SGSN. The old SGSN acknowledges with Cancel Location Ack (IMSI).

9)	The HLR sends Insert Subscriber Data (IMSI, GPRS subscription data) to the new SGSN. The new SGSN validates the MS's presence in the (new) RA. If due to regional subscription restrictions the MS is not allowed to be attached in the RA, the SGSN rejects the Routeing Area Upate Request with an appropriate cause, and may return an Insert Subscriber Data Ack (IMSI, SGSN Area Restricted) message to the HLR. If all checks are successful then the SGSN constructs an MM context for the MS and returns an Insert Subscriber Data Ack (IMSI) message to the HLR.

10)	The HLR acknowledges the Update Location by sending Update Location Ack (IMSI) to the new SGSN.

11)	If the association has to be established, if Update Type indicates combined RA / LA update with IMSI attach requested, or if the LA changed with the routeing area update, then the new SGSN sends a Location Update Request (new LAI, IMSI, SGSN Number, Location Update Type) to the VLR. Location Update Type shall indicate IMSI attach if Update Type in step 1 indicated combined RA / LA update with IMSI attach requested. Otherwise, Location Update Type shall indicate normal location update. The VLR number is translated from the RAI via a table in the SGSN. The SGSN starts the location update procedure towards the new MSC/VLR upon receipt of the first Insert Subscriber Data message from the HLR in step 9). The VLR creates or updates the association with the SGSN by storing SGSN Number.

12)	If the subscriber data in the VLR is marked as not confirmed by the HLR, the new VLR informs the HLR. The HLR cancels the old VLR and inserts subscriber data in the new VLR (this signalling is not modified from existing GSM signalling and is included here for illustrative purposes):

a)	The new VLR sends an Update Location (new VLR) to the HLR.

b)	The HLR cancels the data in the old VLR by sending Cancel Location (IMSI) to the old VLR.

c)	The old VLR acknowledges with Cancel Location Ack (IMSI).

d)	The HLR sends Insert Subscriber Data (IMSI, GSM subscriber data) to the new VLR.

e)	The new VLR acknowledges with Insert Subscriber Data Ack (IMSI).

f)	The HLR responds with Update Location Ack (IMSI) to the new VLR.

13)	The new VLR allocates a new TMSI and responds with Location Update Accept (VLR TMSI) to the SGSN. VLR TMSI is optional if the VLR has not changed.

14)	The new SGSN validates the MS's presence in the new RA. If due to roaming restrictions the MS is not allowed to be attached in the SGSN, or if subscription checking fails, then the SGSN rejects the routeing area update with an appropriate cause. If all checks are successful then the new SGSN establishes MM and PDP contexts for the MS. A logical link is established between the new SGSN and the MS. The new SGSN responds to the MS with Routeing Area Update Accept (P-TMSI, VLR TMSI, P-TMSI Signature, Receive N-PDU Number). Receive N-PDU Number contains the acknowledgements for each acknowledged-mode NSAPI used by the MS, thereby confirming all mobile-originated N-PDUs successfully transferred before the start of the update procedure.

15)	The MS confirms the reallocation of the TMSIs by sending Routeing Area Update Complete (P-TMSI, VLR TMSI, Receive N-PDU Number) to the SGSN. Receive N-PDU Number contains the acknowledgements for each acknowledged-mode NSAPI used by the MS, thereby confirming all mobile-terminated N-PDUs successfully transferred before the start of the update procedure. If Receive N-PDU Number confirms reception of N-PDUs that were forwarded from the old SGSN, then these N-PDUs shall be discarded by the new SGSN. LLC and SNDCP in the MS are reset.

16)	The new SGSN sends TMSI Reallocation Complete (VLR TMSI) to the new VLR if the VLR TMSI is confirmed by the MS.

In the case of a rejected routeing area update operation, due to regional subscription or roaming restrictions, the new SGSN shall not construct an MM context. A reject shall be returned to the MS with an appropriate cause. The MS shall not re-attempt a routeing area update to that RA. The RAI value shall be deleted when the MS is powered-up.

If the SGSN is unable to update the PDP context in one or more GGSNs, then the SGSN shall deactivate the corresponding PDP contexts as described in subclause "PDP Context Deactivation Initiated by SGSN Procedure". This shall not cause the SGSN to reject the routeing area update.

If the routeing area update procedure fails a maximum allowable number of times, or if the SGSN returns a Routeing Area Update Reject (Cause) message, the MS shall enter IDLE state.

If the timer described in step 2 expires and no Cancel Location (IMSI) was received from the HLR, then the old SGSN shall stop forwarding N-PDUs to the new SGSN.

If the Location Update Accept message indicates a reject, then this should be indicated to the MS, and the MS shall not access non-GPRS services until a successful location update is performed.

## 13.2	SGSN

SGSN maintains MM context and PDP context information for MSs in STANDBY and READY states. Table 1 shows the context fields for one MS.

Table 1: SGSN MM and PDP Contexts

| Field | Description |
| --- | --- |
| IMSI | IMSI is the main reference key. |
| MM State | Mobility management state, IDLE, STANDBY, or READY. |
| P-TMSI | Packet Temporary Mobile Subscriber Identity. |
| IMEI | International Mobile Equipment Identity |
| P-TMSI Signature | A signature used for identification checking purposes. |
| Routeing Area | Current routeing area. |
| Cell Identity | Current cell in READY state, last known cell in STANDBY or IDLE state. |
| Cell Identity Age | Time elapsed since the last LLC PDU was received from the MS at the SGSN. |
| VLR Number | The VLR number of the MSC/VLR currently serving this MS. |
| New SGSN Address | The IP address of the new SGSN where buffered and not sent N-PDUs should be forwarded to. |
| Authentication Triplets | Authentication and ciphering parameters. |
| Kc | Currently used ciphering key. |
| CKSN | Ciphering key sequence number of Kc. |
| Ciphering algorithm | Selected ciphering algorithm. |
| Radio Access Classmark | MS radio access capabilities. |
| SGSN Classmark | MS network capabilities. |
| DRX Parameters | Discontinuous reception parameters. |
| MNRG | Indicates whether activity from the MS shall be reported to the HLR. |
| NGAF | Indicates whether activity from the MS shall be reported to the MSC/VLR. |
| PPF | Indicates whether paging for GPRS and non-GPRS services can be initiated. |
| MSISDN | The basic MSISDN of the MS. |
| SMS Parameters | SMS-related parameters, e.g., operator-determined barring. |
| Recovery | Indicates if HLR or VLR is performing database recovery. |
| Radio Priority SMS | The RLC/MAC radio priority level for uplink SMS transmission. |
| Each MM context contains zero or more of the following PDP contexts: |  |
| PDP Context Identifier | Index of the PDP context. |
| PDP Type | PDP type, e.g., X.25 or IP. |
| PDP Address | PDP address, e.g., an X.121 address. |
| NSAPI | Network layer Service Access Point Identifier. |
| TI | Transaction Identifier. |
| PDP State | Packet data protocol state, INACTIVE or ACTIVE. |
| Access Point Name | The APN requested by the MS. |
| GGSN Address in Use | The IP address of the GGSN currently used by the activated PDP context. |
| VPLMN Address Allowed | Specifies whether the MS is allowed to use the APN in the domain of the HPLMN only, or additionally the APN in the domain of the VPLMN. |
| QoS Profile Subscribed | The quality of service profile subscribed. |
| QoS Profile Requested | The quality of service profile requested. |
| QoS Profile Negotiated | The quality of service profile negotiated. |
| Radio Priority | The RLC/MAC radio priority level for uplink user data transmission. |
| Send N-PDU Number | SNDCP sequence number of the next downlink N-PDU to be sent to the MS. |
| Receive N-PDU Number | SNDCP sequence number of the next uplink N-PDU expected from the MS. |
| SND | GTP sequence number of the next downlink N-PDU to be sent to the MS. |
| SNU | GTP sequence number of the next uplink N-PDU to be sent to the GGSN. |
| Charging Id | Charging identifier, identifies charging records generated by SGSN and GGSN. |
| Reordering Required | Specifies whether the SGSN shall reorder N-PDUs before delivering the N-PDUs to the MS. |

In case of anonymous access the SGSN maintains the MM context and PDP context information for MSs in READY state. Table 2 shows the context fields for one MS.

Table 2: SGSN MM and PDP Contexts for Anonymous Access

| Field | Description |
| --- | --- |
| A-TLLI | Auxiliary Temporary Logical Link Identity. |
| AA-TID | Anonymous Access Tunnel Identifier. |
| Routeing Area | Current routeing area. |
| Cell Identity | Current cell. |
| PDP Type | PDP type, e.g., X.25 or IP. |
| PDP Address | PDP address, e.g., an X.121 address. |
| NSAPI | Network layer Service Access Point Identifier. |
| TI | Transaction Identifier. |
| QoS Profile Negotiated | The quality of service profile negotiated. |
| QoS Profile Requested | The quality of service profile requested. |
| GGSN Address in Use | The IP address of the GGSN currently used. |
| Radio Priority | The RLC/MAC radio priority level for uplink user data transmission. |
| Send N-PDU Number | SNDCP sequence number of the next downlink N-PDU to be sent to the MS. |
| Receive N-PDU Number | SNDCP sequence number of the next uplink N-PDU expected from the MS. |
| SND | GTP sequence number of the next downlink N-PDU to be sent to the MS. |
| SNU | GTP sequence number of the next uplink N-PDU to be sent to the GGSN. |
| Charging Id | Charging identifier, identifies charging records generated by SGSN and GGSN. |
| Reordering Required | Specifies whether the SGSN shall reorder N-PDUs before delivering the N-PDUs to the MS. |

## 13.4	MS

Each GPRS MS maintains MM and PDP context information in IDLE, STANDBY and READY states. The information may be contained in the MS and the TE. Table 3 shows the MS context fields.

Table 3: MS MM and PDP Contexts

| Field | SIM | Description |
| --- | --- | --- |
| IMSI | X | International Mobile Subscriber Identity. |
| MM State |  | Mobility management state, IDLE, STANDBY, or READY. |
| P-TMSI | X | Packet Temporary Mobile Subscriber Identity. |
| P-TMSI Signature | X | A signature used for identification checking purposes. |
| Routeing Area | X | Current routeing area. |
| Cell Identity |  | Current cell. |
| Kc | X | Currently used ciphering key. |
| CKSN | X | Ciphering key sequence number of Kc. |
| Ciphering algorithm |  | Selected ciphering algorithm. |
| Classmark |  | MS classmark. |
| DRX Parameters |  | Discontinuous reception parameters. |
| Radio Priority SMS |  | The RLC/MAC radio priority level for uplink SMS transmission. |
| Each MM context contains zero or more of the following PDP contexts: |  |  |
| PDP Type |  | PDP type, e.g., X.25 or IP. |
| PDP Address |  | PDP address, e.g., an X.121 address. |
| PDP State |  | Packet data protocol state, INACTIVE or ACTIVE. |
| Dynamic Address Allowed |  | Specifies whether the MS is allowed to use a dynamic address. |
| NSAPI |  | Network layer Service Access Point Identifier. |
| TI |  | Transaction Identifier. |
| QoS Profile Requested |  | The quality of service profile requested. |
| QoS Profile Negotiated |  | The quality of service profile negotiated. |
| Radio Priority |  | The RLC/MAC radio priority level for uplink user data transmission. |
| Send N-PDU Number |  | SNDCP sequence number of the next uplink N-PDU to be sent to the SGSN. |
| Receive N-PDU Number |  | SNDCP sequence number of the next downlink N-PDU expected from the SGSN. |

The information marked with an "X" in Table 3:

-	shall be stored in the SIM if the connected SIM is GPRS-aware; and

-	may be stored in the ME after GPRS detach if the connected SIM is not GPRS-aware.

If the SIM is GPRS-aware, then the IMSI, P-TMSI, P-TMSI Signature, Routeing Area, Kc, and CKSN stored in the SIM shall be used when accessing the GPRS services.

If the SIM is not GPRS-aware, then the P-TMSI, P-TMSI Signature, Routeing Area, Kc, and CKSN stored in the ME shall be used if and only if the IMSI stored in the SIM is identical to the IMSI image maintained in the ME. If the IMSI stored in the SIM is different from the IMSI image in the ME, then the IMSI image in the ME shall not be used, and the MS shall identify itself with the IMSI stored in the SIM when performing a GPRS attach. IMSI, P-TMSI, P-TMSI Signature, Routeing Area, Kc, and CKSN may be stored in the ME after the GPRS attach has been successfully performed.

For anonymous access each GPRS MS maintains MM and PDP context information in READY state. The information may be contained in the ME and the TE. Table 4 shows the MS context fields.

Table 4: MS MM and PDP Contexts for Anonymous Access

| Field | Description |
| --- | --- |
| A-TLLI | Auxiliary Temporary Logical Link Identity. |
| Routeing Area | Current routeing area. |
| Cell Identity | Current cell. |
| PDP Type | PDP type, e.g., X.25 or IP. |
| PDP Address | PDP address, e.g., an X.121 address. |
| NSAPI | Network layer Service Access Point Identifier. |
| TI | Transaction Identifier. |
| QoS Profile Requested | The quality of service profile requested. |
| QoS Profile Negotiated | The quality of service profile negotiated. |
| Radio Priority | The RLC/MAC radio priority level for uplink user data transmission. |
| Send N-PDU Number | SNDCP sequence number of the next uplink N-PDU to be sent to the SGSN. |
| Receive N-PDU Number | SNDCP sequence number of the next downlink N-PDU expected from the SGSN. |