ETSI 3GPP TSG CN1/SMG3 WPA 	Tdoc N1-99485

Sophia Antipolis, France

May 31, June 01-04, 1999

| CHANGE REQUEST No : |  |  |  |  |  | A027 | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM / UMTS: |  |  |  | 09.18 |  | Version |  | 6.3.0 |  |  |  |
| Submitted to SMG | #29 |  | for approval |  | X | Without presentation ("non-strategic") |  |  |  |  |  |
| list plenary meeting or STC here |  | for information |  |  |  | With presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form. Filename: crf26_3.doc |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME |  | Network | X |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | GPRS |
| --- | --- |

| Source: | Alcatel | Date: | 990526 |
| --- | --- | --- | --- |

| Subject: | Load of the network in case of search procedure |
| --- | --- |

| Category: | F Correction | X | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 | X |
| and one release | C Functional modification of feature |  |  | Release 98 |  |
| only shall be | D Editorial modification |  |  | Release 99 |  |
| Marked with an X) |  |  |  | UMTS |  |

| Reason for change: | In GSM 09.18, in the Paging for non GPRS services procedure (when the MSC/VLR uses the SGSN to page the MS), it is stated in section 5.2.1 and 5.3 that: - “If the state of the association is Gs-NULL and the restoration indicator ‘Confirmed by Radio Contact’ is set to ‘false’, the VLR shall also perform a search procedure as specified in GSM 03.18.” - If the restoration indicator ‘SGSN-Reset’ at the SGSN is set to ‘true’ and if the BSSAP+ PAGING REQUEST message does not include the LAI IE, the SGSN may page in all the routeing areas served by the SGSN that are also served by the sending VLR. In these cases the paging messages are sent by the SGSN and the MSC/VLR in the same routing areas zones which results in a useless load of the network, especially if the MS responds to a PAGING message sent by the SGSN. Therefore, this CR adds the possibility for the MSC/VLR to trigger the search procedure only if the Paging procedure has failed through the SGSN. |
| --- | --- |

| Clauses affected: | 5.2.1 |
| --- | --- |

| Other specs | Other releases of same spec |  | ® List of CRs: |  |
| --- | --- | --- | --- | --- |
| Affected: | Other core specifications |  | ® List of CRs: |  |
|  | MS test specifications / TBRs |  | ® List of CRs: |  |
|  | BSS test specifications |  | ® List of CRs: |  |
|  | O&M specifications |  | ® List of CRs: |  |

| Other comments: |  |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

# 5	Paging for non-GPRS services procedure

## 5.1	General description

This procedure is used by the VLR to send a BSSAP+-PAGING-REQUEST message to an MS via the GPRS service. This procedure applies to MSs that are simultaneously IMSI attached for GPRS services and non-GPRS services. The procedure can be performed simultaneously with any other procedure at the Gs interface.

## 5.2	Procedures in the VLR

The VLR shall handle the timers, queuing and retransmission for sending the BSSAP+-PAGING-REQUEST message on the Gs interface in the same way that it handles the sending of a PAGING message on the A interface.

### 5.2.1	Paging Initiation

When a VLR has to page a GPRS MS it shall check whether the MSC has an SCCP connection for that MS. If no SCCP connection exists the VLR checks the state of the association to an SGSN and the value of the restoration indicators for that MS. The VLR sends BSSAP+-PAGING-REQUEST messages to the SGSN if the state of the association for the MS is Gs-ASSOCIATED, LA-UPDATE-PRESENT or if the state of the association is Gs-NULL and the ‘Confirmed by Radio Contact’ restoration indicator is set to ‘false’. The sending of the BSSAP+-PAGING-REQUEST message does not change the state of the association with  the SGSN.

If the ‘Confirmed by Radio Contact’ restoration indicator is set to ‘true’, the VLR shall include the Location area identifier IE into the BSSAP+-PAGING-REQUEST message, otherwise (i.e. after a VLR failure) the Location area identifier IE shall not be included. When sending the BSSAP+-PAGING-REQUEST message, the VLR shall start timer T5.

If the state of the association is Gs-NULL and the restoration indicator ‘Confirmed by Radio Contact’ is set to ‘false’, the VLR shall also perform a search procedure as specified in GSM 03.18 after the Paging procedure has failed through the SGSN.

### 5.2.2	Paging Response

The VLR stops the paging procedure on expiry of timer T5 or on receipt of an SCCP connection establishment containing the Initial L3 message from the MS via the A interface.

### 5.2.3	Paging Failure

On receipt of a BSSAP+-PAGING-REJECT message before the timer T5 expires, the VLR stops timer T5, the association is moved to the Gs-NULL state and within this state the association is marked with the contents of the Gs Cause IE.

### 5.2.4	MS unreachable

On receipt of a BSSAP+-MS-UNREACHABLE message before the timer T5 expires, the VLR stops timer T5 and the paging procedure for that paging request towards the SGSN is stopped. The state of the association at the VLR is not changed.