ETSI CN1/ SMG3-WPA	Tdoc N1-99-507

Sophia Antipolis,

1-4 June 1999

| CHANGE REQUEST No : |  |  |  |  |  | Axxx | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM / UMTS: |  |  |  | 04.08 |  | Version |  | 6.3.0 |  |  |  |
| Submitted to SMG |  |  | for approval |  |  | without presentation ("non-strategic") |  |  |  | x |  |
| list plenary meeting or STC here  |  | for information |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form. Filename: crf26_3.doc |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | X | Network | x |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | GPRS |
| --- | --- |

| Source: | Nortel Networks | Date: | 28 May 1999 |
| --- | --- | --- | --- |

| Subject: | Align PDP Address use in Activate PDP Context in GSM 04.08 with GSM 03.60 |
| --- | --- |

| Category: | F Correction | x | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 | X |
| and one release | C Functional modification of feature |  |  | Release 98 |  |
| only shall be | D Editorial modification |  |  | Release 99 |  |
| marked with an X) |  |  |  | UMTS |  |

| Reason for change: | This change request proposes to align the conditions for the presence of the PDP Address in the Activate PDP Context Request message as defined in GSM 04.08 with that specified in GSM 03.60. GSM 04.08 v6.3.0 section 6.1.3.1.2 states that in the case of PDP Context Activation by the network: “The ACTIVATE PDP CONTEXT REQUEST message sent by the MS in order to initiate the PDP context activation procedure shall contain the PDP address requested by the network in the REQUEST PDP CONTEXT ACTIVATION message.” No similar requirement is specified for the PDP Context Activation initiated by the mobile station. In fact GSM 03.60 section A.3 states “If no PDP type, no PDP address and no APN is sent by the MS, check if there is a single PDP context subscription record subscribed for this subscriber”. This implies that in the case of PDP Context Activation initiated by the mobile station the IE Requested PDP Address is optional. For this reason this CR proposes to make the IE Requested PDP Address, in the Activate PDP Context Request message, conditional. The conditions for the presence of this IE have been added to GSM 04.08 v6.3.0 section 9.5.1. |
| --- | --- |

| Clauses affected: | 9.5.1 |
| --- | --- |

| Other specs | Other releases of same spec |  |  List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  |  List of CRs: |  |
|  | MS test specifications / TBRs |  |  List of CRs: |  |
|  | BSS test specifications |  |  List of CRs: |  |
|  | O&M specifications |  |  List of CRs: |  |

| Other comments: |  |
| --- | --- |

### 9.5.1	Activate PDP context request

This message is sent by the MS to the network to request activation of a PDP context.	
See table 9.5.1/GSM 04.08.

Message type:	activate PDP context request

Significance:		global

Direction:			MS to network

Table 9.5.1/GSM 04.08: Activate PDP context request message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Protocol discriminator | Protocol discriminator 10.2 | M | V | 1/2 |
|  | Transaction identifier | Transaction identifier 10.3.2 | M | V | 1/2 |
|  | Activate PDP context request message identity | Message type 10.4 | M | V | 1 |
|  | NSAPI | Network service access point identifier 10.5.6.2 | M | V | 1 |
|  | Requested LLC SAPI | LLC service access point identifier 10.5.6.9 | M | V | 1 |
|  | Requested QoS | Quality of service 10.5.6.5 | M | V | 4 |
|  | Requested PDP address | Packet data protocol address 10.5.6.4 | C | LV | 3 - 19 |
| 28 | Access point name | Access point name 10.5.6.1 | O | TLV | 3 - 255 |
| 27 | Protocol configuration options | Protocol configuration options 10.5.6.3 | O | TLV | 3 - 255 |

#### 9.5.1.1 	Requested PDP address

This IE shall be included in the message when PDP Context Activation is requested by network.

This IE is optionally included in the message when the mobile station initiates PDP Context Activation (See GSM 03.60).

#### 9.5.1.2	Access point name

This IE is included in the message when the MS selects a specific external network to be connected to.

#### 9.5.1.3	Protocol configuration options

This IE is included in the message when the MS provides protocol configuration options for the external PDN.