ETSI 3GPP TSG CN1/SMG3 WPA 	Tdoc N1-99558

Sophia Antipolis, France

May 31, June 01-04, 1999

| CHANGE REQUEST No : |  |  |  |  |  | A026 rev1 | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM / UMTS: |  |  |  | 09.18 |  | Version |  | 6.3.0 |  |  |  |
| Submitted to SMG | #29 |  | for approval |  | X | Without presentation ("non-strategic") |  |  |  |  |  |
| list plenary meeting or STC here |  | for information |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form. Filename: crf26_3.doc |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME |  | Network | X |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | GPRS |
| --- | --- |

| Source: | Alcatel | Date: | 990526 |
| --- | --- | --- | --- |

| Subject: | TMSI requested by the MSC trough the Gs interface |
| --- | --- |

| Category: | F Correction | X | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 | X |
| and one release | C Functional modification of feature |  |  | Release 98 |  |
| only shall be | D Editorial modification |  |  | Release 99 |  |
| Marked with an X) |  |  |  | UMTS |  |

| Reason for change: | In GSM 04.08, it is possible for the MSC/VLR to request the TMSI from the MS in the Identity Request message (sections 9.2.10, 10.5.3.4). In addition, in GSM 03.60, section 6.3.5 (MS information procedure), it is written that: - "the VLR may perform the MS Information procedure via the SGSN" and that - "If the information requested is MS identity information (e.g., IMEI) that is not known by the SGSN but is known by the MS, then the SGSN shall interrogate the MS in a similar manner to that described in subclause "Identity Check Procedures"". In this case, the SGSN sends an Identity Request message to the MS. Therefore, this CR adds the possibility for the MSC/VLR to request and get the TMSI through the Gs interface. |
| --- | --- |

| Clauses affected: | 17.1.15, 18.4.12 |
| --- | --- |

| Other specs | Other releases of same spec |  | ® List of CRs: |  |
| --- | --- | --- | --- | --- |
| Affected: | Other core specifications | x | ® List of CRs: | CR A689 on GSM 04.08 |
|  | MS test specifications / TBRs |  | ® List of CRs: |  |
|  | BSS test specifications |  | ® List of CRs: |  |
|  | O&M specifications |  | ® List of CRs: |  |

| Other comments: | The meaning of the values of the Information requested IE has been changed in order to facilitate the description of the different possibilities. This CR is linked to the CR A689 on GSM 04.08 which introduces the possibility for the SGSN to request the TMSI to the MS. |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

### 17.1.14	BSSAP+-MS-INFORMATION-REQUEST message

This message is sent from the VLR to the SGSN to  request  information  associated with  the indicated IMSI. The type of information  requested is specified in the Information requested IE.

Table 17.1.14/GSM 09.18: BSSAP+-MS-INFORMATION-REQUEST message content

| Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- |
| Message type | Message type 18.2 | M | V | 1 |
| IMSI | IMSI 18.4.9 | M | TLV | 6-10 |
| Information requested | Information requested 18.4.12 | M | TLV | 3 |

### 17.1.15	BSSAP+-MS-INFORMATION-RESPONSE message

This message is sent from the SGSN to the VLR as a response to a previous BSSAP+- MS-INFORMATION -REQUEST message. (At least one of the requested identities shall be sent).

Table 17.1.15/GSM 09.18: BSSAP+-MS-INFORMATION-RESPONSE message content

| Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- |
| Message type | Message type 18.2 | M | V | 1 |
| IMSI | IMSI 18.4.9 | M | TLV | 6-10 |
| TMSI | TMSI 18.4.22 | O | TLV | 6 |
| PTMSI | PTMSI 18.4.19 | O | TLV | 6 |
| IMEI | IMEI 18.4.7 | O | TLV | 10 |
| IMEISV | IMEISV 18.4.8 | O | TLV | 10 |
| Cell global identity | Cell global identity 18.4.1 | O | TLV | 10 |
| Location information age | Location information age 18.4.14 | O | TLV | 4 |
| Mobile station state | Mobile station state 18.4.18 | O | TLV | 3 |

#### 17.1.15.1	IMEI

This information element should be included if it was requested in the BSSAP+-MS-INFORMATION-REQUEST message and if this information is obtainable.

#### 17.1.15.2	IMIESV

This information element should be included if it was requested in the BSSAP+-MS-INFORMATION-REQUEST message and if this information is obtainable.

#### 17.1.15.3	Cell global identity

Cell global identity where the MS was in the last radio contact.

This information element should be included if it was requested in the BSSAP+-MS-INFORMATION-REQUEST message and if this information is obtainable.

#### 17.1.15.4	Location information age

Time in minutes since the MS last established a radio transaction.

This information element should be included if it was requested in the BSSAP+-MS-INFORMATION-REQUEST message and if this information is obtainable.

#### 17.1.15.5	Mobile station state

This information element should be included in this message, irrespective of the information requested.

#### 17.1.15.6	TMSI

This information element should be included if it was requested in the BSSAP+-MS-INFORMATION-REQUEST message and if this information is obtainable.

*** Next Modification ***

### 18.4.12	Information requested

The Information requested IE is a TLV IE that indicates to the SGSN the type of information requested by the VLR. The coding of the V field is as follows.

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octet 1 | IEI |  |  |  |  |  |  |  |
| Octet 2 | Length indicator |  |  |  |  |  |  |  |
| Octet 3 | Information requested value |  |  |  |  |  |  |  |

Figure 18.4.12/GSM 09.18: Information requested IE

Table 18.4.12/GSM 09.18: Information requested IE value part

Information requested value (octet 3) Bits 8 7 6 5 4 3 2 1 0 0 0 0 0 0 0 0 Interpreted as Not supported in this version of the protocol. 0 0 0 0 0 0 0 1 PTMSI 0 0 0 0 0 0 1 0 IMEI 0 0 0 0 0 0 1 1 IMEISV 0 0 0 0 0 1 0 0 PTMSI and IMEI 0 0 0 0 0 1 0 1 PTMSI and IMEISV 0 0 0 0 0 1 1 0 IMEI and IMEISV 0 0 0 0 0 1 1 1 PTMSI, IMEI, and IMEISV 0 0 0 0 1 0 0 0 Mobile location information 0 0 0 0 1 0 0 1 TMSI 0 0 0 0 1 0 1 0 Interpreted as Not supported in this version of the protocol. To 1 1 1 1 1 1 1 1

NOTE:	The behaviour of the receiver in the case of a Not supported value is described in Sub-clause 14.3, Procedures in the SGSN.