ETSI 3GPP TSG CN1/SMG3 WPA 	Tdoc N1-99574

Sophia Antipolis, France																		rev of Tdoc N1-99489

May 31, June 01-04, 1999

| CHANGE REQUEST No : |  |  |  |  |  | A685 rev 1 | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM / UMTS: |  |  |  | 04.08 |  | Version |  | 6.3.0 |  |  |  |
| Submitted to SMG | #29 |  | for approval |  | X | without presentation ("non-strategic") |  |  |  |  |  |
| list plenary meeting or STC here |  | for information |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form. Filename: crf26_3.doc |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | x | Network | X |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | GPRS |
| --- | --- |

| Source: | Alcatel | Date: | 990603 |
| --- | --- | --- | --- |

| Subject: | TMSI requested by the MSC trough the Gs interface |
| --- | --- |

| Category: | F Correction | X | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 | X |
| and one release | C Functional modification of feature |  |  | Release 98 |  |
| only shall be | D Editorial modification |  |  | Release 99 |  |
| Marked with an X) |  |  |  | UMTS |  |

| Reason for change: | In GSM 04.08, it is possible for the MSC/VLR to request the TMSI from the MS in the Identity Request message (sections 9.2.10, 10.5.3.4). In addition, in GSM 03.60, section 6.3.5 (MS information procedure), it is written that: - "the VLR may perform the MS Information procedure via the SGSN" and that - "If the information requested is MS identity information (e.g., IMEI) that is not known by the SGSN but is known by the MS, then the SGSN shall interrogate the MS in a similar manner to that described in subclause "Identity Check Procedures"". In this case, the SGSN sends an Identity Request message to the MS. Therefore, this CR adds the possibility for the SGSN to request the TMSI to the MS. This can be used by the MSC/VLR to request and get the TMSI through the Gs interface |
| --- | --- |

| Clauses affected: | 10.5.5.9 |
| --- | --- |

| Other specs | Other releases of same spec |  | ® List of CRs: |  |
| --- | --- | --- | --- | --- |
| Affected: | Other core specifications | x | ® List of CRs: | CR A026 on GSM 09.18 |
|  | MS test specifications / TBRs |  | ® List of CRs: |  |
|  | BSS test specifications |  | ® List of CRs: |  |
|  | O&M specifications |  | ® List of CRs: |  |

| Other comments: | This CR is linked to the CR A026 on GSM 09.18 which introduces the possibility for the MSC/VLR to request the TMSI through the Gs interface. |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

## 9.4	GPRS Mobility Management Messages

### 9.4.12	Identity request

This message is sent by the network to the MS to request submission of the MS identity according to the specified identity type. See table 9.4.12/GSM 04.08.

Message type:	IDENTITY REQUEST

Significance:		dual

Direction:			network to MS

Table 9.4.12/GSM 04.08: IDENTITY REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Protocol discriminator | Protocol discriminator 10.2 | M | V | 1/2 |
|  | Skip indicator | Skip indicator 10.3.1 | M | V | 1/2 |
|  | Identity request message identity | Message type 10.4 | M | V | 1 |
|  | Identity type | Identity type 2 10.5.5.9 | M | V | 1/2 |
|  | Force to standby | Force to standby 10.5.5.7 | M | V | 1/2 |

### 9.4.13	Identity response

This message is sent by the MS to the network in response to an identity request message providing the requested identity. See table 9.4.13/GSM 04.08.

Message type:	IDENTITY RESPONSE

Significance:		dual

Direction:			MS to network

Table 9.4.13/GSM 04.08: IDENTITY RESPONSE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Protocol discriminator | Protocol discriminator 10.2 | M | V | 1/2 |
|  | Skip indicator | Skip indicator 10.3.1 | M | V | 1/2 |
|  | Identity response message identity | Message type 10.4 | M | V | 1 |
|  | Mobile identity | Mobile identity 10.5.1.4 | M | LV | 6 - 11 |

#### 10.5.5.9	Identity type 2

The purpose of the identity type 2 information element is to specify which identity is requested.

The identity type 2 is a type 1 information element.

The identity type 2 information element is coded as shown in figure 10.5.125/GSM 04.08 and table 10.5.142/GSM 04.08.

|  | 8 7 6 5 4 3 2 1 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Identity type 2 IEI | 0 spare | Type of identity | octet 1 |

Figure 10.5.125/GSM 04.08: Identity type 2 information element

Table 10.5.142/GSM 04.08: Identity type 2 information element

Type of identity (octet 1) Bits 3 2 1 0 0 1 IMSI 0 1 0 IMEI 0 1 1 IMEISV 1 0 0 TMSI All other values are interpreted as IMSI by this version of the protocol.