| 3GPP N2 Meeting |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Document |  |  |  |  |  |  |  |  |  | N2A000316 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Charleston, USA, 27-31 MAR 2000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | e.g. for 3GPP use the format TP-99xxx or for SMG, use the format P-99-xxx |  |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 23.078 |  |  |  |  |  | CR |  |  | 127 |  |  |  |  |  |  | Current Version: |  |  |  |  |  |  |  |  |  | 3.4.0 |  |  |  |  |
| GSM (AA.BB) or 3G (AA.BBB) specification number  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  CR number as allocated by MCC support team |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For submission to: |  |  |  |  |  |  |  |  | for approval |  |  |  |  |  |  |  |  |  | strategic |  |  |  |  |  |  |  |  |  |  |  |  |  |  | (for SMG |  |  |  |
| list expected approval meeting # here  |  |  |  |  |  | for information |  |  |  |  |  |  |  |  |  |  |  |  | non-strategic |  |  |  |  |  |  |  |  |  |  |  |  |  |  | use only) |  |  |  |
| Form: CR cover sheet, version 2 for 3GPP and SMG The latest version of this form is available from: ftp://ftp.3gpp.org/Information/CR-Form-v2.doc |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  |  |  | (U)SIM |  |  |  |  | ME |  |  |  |  | UTRAN / Radio |  |  |  |  |  |  |  |  |  |  |  | Core Network |  |  |  |  |  |  |  |  |
| (at least one should be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source: |  |  | Siemens |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  |  |  |  | 27 March 2000 |  |  |  |  |  |
| Subject: |  |  | GPRS reference number assigned by SCP |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item: |  |  | CAMEL Phase 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Category: | F Correction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release: |  |  |  |  |  | Phase 2 |  |  |  |  |  |  |
|  | A Corresponds to a correction in an earlier release |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 96 |  |  |  |  |  |  |
| (only one category | B Addition of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  | Release 97 |  |  |  |  |  |  |
| shall be marked | C Functional modification of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 98 |  |  |  |  |  |  |
| with an X) | D Editorial modification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 99 |  |  |  | x |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 00 |  |  |  |  |  |  |
| Reason for change: |  |  | Currently, GPRS reference number is allocated by the gprsSSF. When this number is handled by the gsmSCF, the gsmSCF has to map this number to the internal processing reference. Such a mapping procedure would be inevitable as long as only one reference number is allocated by one entity and used by the both ones. This CR proposes the procedure between the gprsSSF and the gsmSCF where each entity allocates its own reference number and, in the case of the information exchange, one entity also sends the another reference number which is solely used by the other end. Consequently, the current GPRS reference number is renamed to “GPRS reference number in gprsSSF”. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  | 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  | Other 3G core specifications |  |  |  |  |  |  |  |  |  |  |  | x |  List of CRs: |  |  |  |  |  |  | 29.078-060 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  | Other GSM core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | MS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | BSS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | O&M specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## 6.6	Description of information flows

### 6.6.1	gprsSSF to gsmSCF Information Flows

#### 6.6.1.1	Activity Test GPRS Ack

##### 6.6.1.1.1	Description

This IF is the response to the Activity Test GPRS.

##### 6.6.1.1.2	Information Elements

This IF contains no information elements.

#### 6.6.1.2		Apply Charging Report GPRS

##### 6.6.1.2.1	Description

This IF is used by the gprsSSF to report to the gsmSCF the information requested in the Apply Charging GPRS IF.  In addition, this IF is used to notify the gsmSCF of user initiated change in QoS.  Note that there are several possible QoS profiles defined by the combinations of the different QoS attributes as defined in 3G TS 23.060, see reference [11]. A PLMN may only support and charge on a limited subset of those QoS. It is recommended that changes in QoS are only reported in Apply Charging Report GPRS for those QoS profiles.

##### 6.6.1.2.2	Information Elements

The following information elements are used:

| Information element name | Required | Description |
| --- | --- | --- |
| Gprs Reference Number in gprsSSF | M | - This IE contains an identifier that is allocated by the gprsSSF and it is used to identify the gprsSSF instance taking care of GPRS session or PDP context. |
| Gprs Reference Number in gsmSCF | M | - This IE contains an identifier that is allocated by the gsmSCF and it is used to identify the gsmSCF instance taking care of GPRS session or PDP context. |
| Charging Result | M | This IE contains the charging information for the PDP provided by the gsmSSF. It is a choice between elapsed time and data volume. |
| Quality of Service | C | This IE identifies the QoS requested by the user and granted by the SGSN due to ‘Modify PDP Context request. This IE shall only be present if sending of the Apply Charging Report was triggered by a change in Quality of Service. |
| Active | M | This IE indicates if the GPRS session or PDP context is still established, or if it has been detached or deactivated. |
| PDP ID | C | This IE identifies the PDP context which the Apply Charging Report is applicable for. If not present the dialogue corresponds to the GPRS session or to one single PDP context. |

M	Mandatory (The IE shall always be sent)

C	Conditional (The IE shall be sent, if available)

#### 6.6.1.3	Entity Released GPRS

##### 6.6.1.3.1	Description

This IF is used by the gprsSSF to inform the gsmSCF at any phase that a GPRS session or PDP context has been terminated by the SGSN without reporting any EDP.

##### 6.6.1.3.2	Information Elements

The following information elements are used:

| Information element name | Required | Description |
| --- | --- | --- |
| Gprs Reference Number in gprsSSF | M | - This IE contains an identifier that is allocated by the gprsSSF and it is used to identify the gprsSSF instance taking care of GPRS session or PDP context. |
| Gprs Reference Number in gsmSCF | M | - This IE contains an identifier that is allocated by the gsmSCF and it is used to identify the gsmSCF instance taking care of GPRS session or PDP context. |
| GPRS Cause | M | This IE contains the Cause value indicating the reason for discontinuation of the PDP context. |
| PDP ID | M | This IE identifies the PDP context which has been terminated by the SGSN. |

M	Mandatory (The IE shall always be sent)

#### 6.6.1.4	Event Report GPRS

##### 6.6.1.4.1	Description

This IF is used to notify the gsmSCF of a GPRS event (e.g. Attach or Detach) previously requested by the gsmSCF in a Request Report GPRS Event IF.

##### 6.6.1.4.2	Information Elements

The following information elements are required:

| Information element name | Required | Description |
| --- | --- | --- |
| Gprs Reference Number in gprsSSF | M | - This IE contains an identifier that is allocated by the gprsSSF and it is used to identify the gprsSSF instance taking care of GPRS session or PDP context. |
| Gprs Reference Number in gsmSCF | M | - This IE contains an identifier that is allocated by the gsmSCF and it is used to identify the gsmSCF instance taking care of GPRS session or PDP context. |
| GPRS Event type | M | This IE specifies the type of event that is reported. |
| Misc GPRS Info | M | This IE indicates the DP type (EDP-N or EDP-R). |
| GPRS Event Specific Information | C | This IE contains information specific to the reported event, e.g. new routeing area in case of change of position or charging id in case of PDP Context Establishment Acknowledgement. |
| PDP ID | C | This IE identifies the PDP context, which the Report GPRS Event is applicable for. If not present the dialogue corresponds to the Attach/Detach FSM or to one single PDP context. |

M	Mandatory (The IE shall always be sent)

C	Conditional (The IE shall be sent, if available)

#### 6.6.1.5	Initial DP GPRS

##### 6.6.1.5.1	Description

This IF is generated by the gprsSSF when a trigger is detected at a DP in the GPRS state machines, to request instructions from the gsmSCF.

##### 6.6.1.5.2	Information Elements

The following information elements are required:

| Information element name | Required | Description |
| --- | --- | --- |
| Gprs Reference Number in gprsSSF | M | - This IE contains an identifier that is allocated by the gprsSSF and it is used to identify the gprsSSF instance taking care of GPRS session or PDP context. |
| ServiceKey | M | This IE indicates to the gsmSCF the requested CAMEL Service. It is used to address the required application/SLP within the gsmSCF. |
| GPRS Event Type | M | This IE indicates the armed GPRS DP event resulting in the Initial Data Event IF. |
| MSISDN | M | This IE contains the basic MSISDN of the MS. |
| IMSI | M | This IE identifies the mobile subscriber. |
| Time and Time zone | M | This IE contains the time that the gprsSSF was triggered, and the time zone the gprsSSF resides in. |
| GPRS MS Class | C | This IE contains the MS network and radio access capabilities. |
| PDP Type | C | This IE identifies the PDP Type, e.g. X.25 or IP. |
| Quality of Service | C | This IE identifies the QoS (subscribed, requested or negotiated). |
| Access Point Name | C | This IE identifies the address Access Point Name the MS has requested to connect to. |
| Routeing Area Identity | C | This IE contains the location information of the MS. |
| Charging ID | C | This IE contains the Charging ID received from the GGSN for the PDP context. |
| SGSN Capabilities | C | This IE specifies the capabilities of the SGSN node to support the CAMEL interwork, e.g. support of Advice of Charge. |

M	Mandatory (The IE shall always be sent)

C	Conditional (The IE shall be sent, if available)

### 6.6.2	gsmSCF to gprsSSF Information Flows

#### 6.6.2.1	Activity Test GPRS

##### 6.6.2.1.1	Description

This IF is used to check for the continued existence of a relationship between the gsmSCF and gprsSSF. If the relationship is still in existence, then the gprsSSF will respond. If no reply is received, then the gsmSCF will assume that the gprsSSF has failed in some way and will take the appropriate action.

##### 6.6.2.1.2	Information Elements

The following information elements are used:

| Information element name | Required | Description |
| --- | --- | --- |
| Gprs Reference Number in gprsSSF | M | - This IE contains an identifier that is allocated by the gprsSSF and it is used to identify the gprsSSF instance taking care of GPRS session or PDP context. |
| Gprs Reference Number in gsmSCF | M | - This IE contains an identifier that is allocated by the gsmSCF and it is used to identify the gsmSCF instance taking care of GPRS session or PDP context. |

M	Mandatory (The IE shall always be sent)

#### 6.6.2.2	Apply Charging GPRS

##### 6.6.2.2.1	Description

This IF is used for interacting from the gsmSCF with the gprsSSF charging mechanisms to control the charging of a GPRS session or PDP Context.

##### 6.6.2.2.2	Information Elements

| Information element name | Required | Description |
| --- | --- | --- |
| Gprs Reference Number in gprsSSF | M | - This IE contains an identifier that is allocated by the gprsSSF and it is used to identify the gprsSSF instance taking care of GPRS session or PDP context. |
| Gprs Reference Number in gsmSCF | M | - This IE contains an identifier that is allocated by the gsmSCF and it is used to identify the gsmSCF instance taking care of GPRS session or PDP context. |
| Charging Characteristics | M | This IE specifies the charging related information to be provided by the gsmSSF and the conditions on which this information has to be provided back to the gsmSCF. It is a choice between granted volume and granted time for the data transfer. |
| Tariff Switch Interval | O | This information element specifies the time duration until the next tariff switch occurrence. |
| PDP ID | C | This IE identifies the PDP context, which the Apply GPRS Charging is applicable for. If not present the dialogue corresponds to the GPRS session or to one single PDP context. |

M	Mandatory (The IE shall always be sent)

O	Optional (Service logic dependent).

C	Conditional (The IE shall be sent, if available)

#### 6.6.2.3	Cancel GPRS

##### 6.6.2.3.1	Description

This IF is used by the gsmSCF to request the gprsSSF to cancel all EDPs and reports.

##### 6.6.2.3.2	Information Elements

The following information elements are used:

| Information element name | Required | Description |
| --- | --- | --- |
| Gprs Reference Number in gprsSSF | M | - This IE contains an identifier that is allocated by the gprsSSF and it is used to identify the gprsSSF instance taking care of GPRS session or PDP context. |
| Gprs Reference Number in gsmSCF | M | - This IE contains an identifier that is allocated by the gsmSCF and it is used to identify the gsmSCF instance taking care of GPRS session or PDP context. |
| PDP ID | C | This IE identifies the PDP context which is to be cancelled. If not present the dialogue corresponds to the GPRS session or to one single PDP context. |

M	Mandatory (The IE shall always be sent)

C	Conditional (The IE shall be sent, if available)

#### 6.6.2.4	Connect GPRS

##### 6.6.2.4.1	Description

This IF is used by the gsmSCF to request the gprsSSF to modify the APN used when establishing a PDP Context.

##### 6.6.2.4.2	Information Elements

The following information elements are used:

| Information element name | Required | Description |
| --- | --- | --- |
| Access Point Name | M | This IE contains the Access Point Name to be used when establishing the PDP Context. |
| PDP Id | C | This IE identifies the PDP Context where the new Access Point Name shall be used. |

M	Mandatory (The IE shall always be sent)

#### 6.6.2.5	Continue GPRS

##### 6.6.2.5.1	Description

This information flow requests the gprsSSF to proceed with processing at the DP at which it previously suspended processing to await gsmSCF instructions. The gprsSSF completes DP processing, and continues processing (i.e., proceeds to the next point in the Attach/Detach FSM or PDP Context FSM) without substituting new data from the gsmSCF.

##### 6.6.2.5.2	Information Elements

The following information element is used:

| Information element name | Required | Description |
| --- | --- | --- |
| PDP ID | C | This IE identifies the PDP context which processing shall continue for. If not present the dialogue corresponds to the GPRS session or to one single PDP context. |

C	Conditional (The IE shall be sent, if available)

#### 6.6.2.6		Furnish Charging Information GPRS

##### 6.6.2.6.1	Description

This IF is used to request the gprsSSF to include information in the CAMEL specific logical call record.

The logical call record is created when FCI-GPRS is received and a logical call record for that FSM does not exist. For modelling purposes the logical call record is buffered in the gprsSSF. The gprsSSF completes logical call records as defined in the SDLs. Once the logical call record is completed, then its free format data is moved to the corresponding CDR and the logical call record is deleted.

In the SGSN there is a separate Logical call record for the attach/detach state model and for each PDP context.

The CSE can send multiple concatenated FCIs per Logical Call Record for completion. The total maximum of free format data is 160 octets per Logical Call Record. The 160 octets may be sent in one or more FCI operations. If there is non-completed free format data and new FCI operation(s) is/are received to overwrite the non-completed data, then the non-completed data is discarded and  the gsmSCF can send another 160 octets per CDR.

##### 6.6.2.6.2	Information Elements

The following information elements are used:

| Information element name | Required | Description |
| --- | --- | --- |
| Gprs Reference Number in gprsSSF | M | - This IE contains an identifier that is allocated by the gprsSSF and it is used to identify the gprsSSF instance taking care of GPRS session or PDP context. |
| Gprs Reference Number in gsmSCF | M | - This IE contains an identifier that is allocated by the gsmSCF and it is used to identify the gsmSCF instance taking care of GPRS session or PDP context. |
| FCI GPRS Billing Charging Characteristics | M | This IE is described in the next table. |

M	Mandatory (The IE shall always be sent)

FCI GPRS Billing Charging Characteristics contains the following information:

| Information element name | Required | Description |
| --- | --- | --- |
| FCIBCCCAMEL Sequence 1 | M | This IE is described in the next table. |

M	Mandatory (The IE shall always be sent)

FCIBCCCAMEL Sequence 1 contains the following information:

| Information element name | Required | Description |
| --- | --- | --- |
| Free Format Data | M | This IE is a free format data to be inserted in the CAMEL logical call record. |
| Append Free Format Data | O | This IE indicates that the gprsSSF shall append the free format data to the Logical call record. In the SGSN there is a separate Logical call record for the attach/detach state model and for each PDP context. - If this IE is present indicating “Append”, the gprsSSF shall append the free format data received in this IF to the free format data already present in the Logical call record for that GPRS session or PDP Context. - If this IE is absent or in value “Overwrite”, then the gprsSSF shall overwrite all free format data already present in the Logical call record for that GPRS session or PDP Context, by the free format data received in this IF. If no Logical call record exists yet for that GPRS session or PDP Context, then the gprsSSF shall ignore this IE. |
| PDP Id | C | This IE identifies the PDP context’s Logical call record to which the free format data shall be appended or overwritten. If not present, the free format data belong to a Logical call record for a GPRS session or a single PDP context for the dialogue. |

M	Mandatory (The IE shall always be sent)

O	Optimal (Service logic dependent)

C	Conditional (The IE shall be sent, if available)

#### 6.6.2.7	Release GPRS

##### 6.6.2.7.1	Description

This IF is used by the gsmSCF to tear down an existing GPRS session or PDP Context at any phase.

##### 6.6.2.7.2	Information Elements

The following information elements are used:

| Information element name | Required | Description |
| --- | --- | --- |
| Gprs Reference Number in gpsSSF | M | - This IE contains an identifier that is allocated by the gprsSSF and it is used to identify the gprsSSF instance taking care of GPRS session or PDP context. |
| Gprs Reference Number in gsmSCF | M | - This IE contains an identifier that is allocated by the gsmSCF and it is used to identify the gsmSCF instance taking care of GPRS session or PDP context. |
| GPRS Cause | M | This IE contains the Cause value indicating the reason for releasing the GPRS session or PDP context. |
| PDP ID | C | This IE identifies the PDP context which shall be released. If not present the dialogue corresponds to the GPRS session or to one single PDP context. |

M	Mandatory (The IE shall always be sent)

C	Conditional (The IE shall be sent, if available)

#### 6.6.2.8	Request Report GPRS Event

##### 6.6.2.8.1	Description

This IF is used to request the gprsSSF to monitor for an event and send a notification back to the gsmSCF when the event is detected (see Event Report Data).

##### 6.6.2.8.2	Information Elements

The following information elements are used:

| Information element name | Required | Description |
| --- | --- | --- |
| Gprs Reference Number in gprsSSF | M | - This IE contains an identifier that is allocated by the gprsSSF and it is used to identify the gprsSSF instance taking care of GPRS session or PDP context. |
| Gprs Reference Number in gsmSCF | M | - This IE contains an identifier that is allocated by the gsmSCF and it is used to identify the gsmSCF instance taking care of GPRS session or PDP context. |
| GPRS Event | M | This IE specifies the event or events of which a report is requested. |
| PDP ID | C | This IE identifies the PDP context, which the Request Report GPRS Event is applicable for. If not present the dialogue corresponds to the GPRS session or to one single PDP context. |

M	Mandatory (The IE shall always be sent)

C	Conditional (The IE shall be sent, if available)

Data Event contains the following information:

| Information element name | Required | Description |
| --- | --- | --- |
| GPRS Event type | M | This IE specifies the type of event of which a report is requested. |
| Monitor Mode | M | This IE indicates how the event shall be reported. |

M	Mandatory (The IE shall always be sent)

#### 6.6.2.9	Reset Timer GPRS

##### 6.6.2.9.1	Description

This IF is used to refresh the gprsSSF timer.

##### 6.6.2.9.2	Information Elements

The following information elements are required:

| Information element name | Required | Description |
| --- | --- | --- |
| Timer ID | M | This IE specifies the default value for the Tssf timer. |
| Timer Value | M | This IE specifies the value to which the timer Tssf shall be set. |
| PDP ID | C | This IE identifies the PDP context, which the Reset of the timer is applicable for. If not present the dialogue corresponds to the GPRS session or to one single PDP context. |

M	Mandatory (The IE shall always be sent)

C	Conditional (The IE shall be sent, if available)

#### 6.6.2.10	Send Charging Information GPRS

##### 6.6.2.10.1	Description

This IF is used to send e-parameters from the gsmSCF to the gprsSSF. If charge advice information is received from the gsmSCF, it shall replace the charge advice information which would be generated by the SGSN and inhibit any further generation of CAI by the SGSN. Further processing of the charge advice information by the SGSN shall be in accordance with the GSM Advice of Charge Supplementary Service.

NOTE: 	If charge advice information is received from the gsmSCF after charge information has been generated by the SGSN and sent to the MS, the behaviour of the service may be unpredictable or incorrect; the service designer should therefore ensure that the first set of charge advice information is sent to the gprsSSF before charge information is sent to the to the MS.

##### 6.6.2.10.2	Information Elements

The following information elements are required:

| Information element name | Required | Description |
| --- | --- | --- |
| Gprs Reference Number in gprsSSF | M | - This IE contains an identifier that is allocated by the gprsSSF and it is used to identify the gprsSSF instance taking care of GPRS session or PDP context. |
| Gprs Reference Number in gsmSCF | M | - This IE contains an identifier that is allocated by the gsmSCF and it is used to identify the gsmSCF instance taking care of GPRS session or PDP context. |
| SCI GPRS Billing ChargingCharacteristics | M | This IE defines the Advice Of Charge related information to be provided to the Mobile Station, if supported by the SGSN. |

M	Mandatory (The IE shall always be sent)

GPRS SCI Billing Charging Characteristics is defined as:

| Information element name | Required | Description |
| --- | --- | --- |
| AOC GPRS | M | This IE is sent after an Activate PDP Context Accept or Attach Accept has been received from the SGSN. This IE defines the Advice Of Charge related information to be provided to the Mobile Station, if supported by the SGSN. |
| PDP Id | C | This IE is included if the AoC is applicable to a PDP context. If not present the AoC is applicable to the GPRS session or for a single PDP context for the dialogue. |

M	Mandatory (The IE shall always be sent)

C	Conditional (The IE shall be sent, if available)

AOC GPRS is defined as:

| Information element name | Required | Description |
| --- | --- | --- |
| AOC Initial | M | This IE contains CAI elements as defined in 3G TS 22.024 [x]. |
| AOC Subsequent | O | See definition in the next table. |

M	Mandatory (The IE shall always be sent)

O	Optional (Service logic dependent)

AOC Subsequent is defined as:

| Information element name | Required | Description |
| --- | --- | --- |
| CAI Elements | M | This IE contains CAI elements as defined in 3G TS 22.024 [x]. |
| Tariff Switch Interval | O | This IE indicates the tariff switch time until the next tariff switch applies. |

M	Mandatory (The IE shall always be sent)

O	Optional (Service logic dependent)