3GPP TSG-CN WG2 Meeting #20	Tdoc N2-010804

Brighton, UK, 15th - 19th October  2001

| CR-Form-v4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| z | 23.078 |  |  |  | CR | 324 |  |  |  | z rev |  |  |  | 1 |  | z Current version: |  |  |  |  | 3.10.0 |  | z |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the z symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: z |  |  |  |  | (U)SIM |  |  | ME/UE |  |  |  |  | Radio Access Network |  |  |  |  |  |  |  |  | Core Network |  | X |
| Title: z |  | Indication of deletion of CSI in Notify Subscriber Data Change |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source: z |  | Lucent Technologies |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: z |  | CAMEL Phase 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: z |  | 15th October 2001 |  |  |  |  |  |
| Category: z |  | F (essential Correction) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release: z |  | R99 |  |  |  |  |  |
|  |  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: 2 (GSM Phase 2) R96 (Release 1996) R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) REL-4 (Release 4) REL-5 (Release 5) |  |  |  |  |  |  |
| Reason for change: z |  |  | The notify subscriber data change feature in CAMEL Phase 3 provides a facility for the HLR to notify the gsmSCF about changes in CSI, Call Forwarding, call baring data or ODB (provided that the notification flag is set). A notification is triggered when subscriber data changes. One process that triggers such a notification is subscriber data change by the administrator. A deletion of a CSI constitutes a change in subscriber data, but the current messages are unable to convey the fact that a CSI for a subscriber has been deleted. It is essential for the HLR to indicate the difference between when a CSI is deactivated and when it is deleted. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change: z |  |  | The change outlined in this document adds a parameter to the Notify Subscriber Data Change information between the HLR and the gsmSCF. This parameter, the SpecificCSIDeleted indicate which CSI has been deleted for a subscriber. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if z not approved: |  |  | An HLR would not be able to indicate to a gsmSCF that a particular CSI has been deleted. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: z |  |  | 10.3.2.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs z |  |  | X | Other core specifications z |  |  |  |  |  |  |  |  |  |  | 29.002 (CR#317) |  |  |  |  |  |  |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | O&M Specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments: z |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

- 10.3.2.3	Notify Subscriber Data Change

- 10.3.2.3.1	Description

This IF is used by the HLR to notify to the gsmSCF of the change of subscriber data.

- 10.3.2.3.2	Information Elements

The following information elements are required:

| Information element name | Required | Description |
| --- | --- | --- |
| IMSI | M | The IMSI is used to identify the subscriber. |
| MSISDN | M | The MSISDN is used to identify the subscriber. |
| Call Forwarding SS data | C | This IE is described in a table below. |
| Call Barring SS data | C | This IE is described in a table below. |
| Operator Determined Barring data | C | This IE is described in a table below. |
| CAMEL Subscription Information | C | This IE is described in a table below. |

M	Mandatory (The IE shall always be sent).

C	Conditional (The IE shall be sent, if  available)

Call Forwarding SS data contains the following information:

| Information element name | Required | Description |
| --- | --- | --- |
| SS Code | C | This IE indicates Call Forwarding supplementary service as defined in 3GPP TS 22.004 [25]. |
| Forwarding Feature List | C | See the table below. |
| Notification-to-CSE Flag | C | This IE indicates whether the gsmSCF is notified of a change of Call Forwarding SS data. |

C	Conditional (The IE shall be sent, if available)

Forwarding Feature List contains 1 to 32 items of the following information:

| Information element name | Required | Description |
| --- | --- | --- |
| Basic Service | C | See 3GPP TS 22.002 [24]. Also compound basic service codes can be used in this operation if the subscriber has used a compound code when modifying the SS (e.g. all bearer services compound code). |
| SS Status | C | See 3GPP TS 23.011 [26]. |
| Forwarded-to Number | C | See 3GPP TS 23.082 [27]. |
| Forwarded-to Subaddress | C | See 3GPP TS 23.082 [27]. |
| Subscription Options | C | See 3GPP TS 23.082 [27]. |
| No Reply Condition Time | C | See 3GPP TS 23.082 [27]. |

C	Conditional (The IE shall be sent, if available and applicable)

Call Barring SS data contains the following information:

| Information element name | Required | Description |
| --- | --- | --- |
| SS Code | C | This IE indicates Call Barring supplementary service as defined in 3GPP TS 22.004 [25]. |
| Call Barring Feature List | C | See the table below. |
| Password | C | See 3GPP TS 23.011 [26]. |
| Wrong password attempts counter | C | See 3GPP TS 23.011 [26]. |
| Notification-to-CSE flag | C | This IE indicates whether the gsmSCF is notified of a change of Call Barring SS data. |

C	Conditional (The IE shall be sent, if available)

Call Barring Feature List contains 1 to 32 items of  the following information:

| Information element name | Required | Description |
| --- | --- | --- |
| Basic Service | C | See 3GPP TS 22.002 [24]. Also compound basic service codes can be used in this operation if the subscriber has used a compound code when modifying the SS (e.g. all bearer services compound code). |
| SS Status | C | See 3GPP TS 23.011 [26]. |

C	Conditional (The IE shall be sent, if available and applicable)

Operator determined barring data contains the following information:

| Information element name | Required | Description |
| --- | --- | --- |
| ODB General Data | C | This IE indicates the set of subscribers features that the network operator or the service provider can regulate. |
| ODB HPLMN Specific Data | C | This IE indicates the set of subscribers features that the network operator or the service provider can regulate only when the subscriber is registered in the HPLMN. |
| Notification-to-CSE flag | C | This IE indicates whether the gsmSCF is notified of a change of ODB data. |

C	Conditional (The IE shall be sent, if available and applicable)

CAMEL Subscription Information contains the following information:

| Information element name | Required | Description |
| --- | --- | --- |
| O-CSI | C | See subclause 4.3.1. |
| D-CSI | C | See subclause 4.3.2. |
| T-CSI | C | See subclause 4.3.4. |
| VT-CSI | C | See subclause 4.3.5. |
| TIF-CSI | C | See subclause 4.3.6.2. |
| GPRS-CSI | C | See subclause 6.3.1. |
| SMS-CSI | C | See subclause 7.3.1. |
| SS-CSI | C | See subclause 8.2.1. |
| M-CSI | C | See subclause 9.2.1 |
| Specific CSI Deleted List | C1 | This IE indicates that one or more specific elements of CAMEL Subscription Information have been deleted from the HLR. It shall indicate any of the following : - O-CSI (with TDP criteria for O-CSI); - T-CSI (with TDP criteria for T-CSI); - TIF-CSI; - D-CSI; - VT-CSI with TDP criteria for VT-CSI; - GPRS-CSI; - SMS-CSI; - SS-CSI; - M-CSI. |

C	Conditional (The IE shall be sent, if it was modified)

C1	Conditional (The IE shall be sent, if CSI is/are deleted).