3GPP TSG-CN WG2  Meeting #21	Tdoc N2-010975

Cancun, Mexico, 26th – 30th November

| CR-Form-v3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 23.018 |  |  |  |  |  |  |  |  |  rev |  |  |  | - |  Current version: |  |  |  |  | 3.9.0 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  |  | (U)SIM |  |  | ME/UE |  |  |  | Radio Access Network |  |  |  |  |  |  |  | Core Network |  | x |
| Title:  |  | Corrections in the ATI mechanism description |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | France Telecom |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code:  |  | CAMEL 3 |  |  |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | 2001-11-25 |  |  |  |  |  |
| Category:  |  | F |  | Agreed by consensus |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | REL-99 |  |  |  |  |  |
|  |  | Use one of the following categories: F (essential correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: 2 (GSM Phase 2) R96 (Release 1996) R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) REL-4 (Release 4) REL-5 (Release 5) |  |  |  |  |  |  |
| Reason for change:  |  |  | The UMTS specific location information (SAI) has to be taken into account |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | The Active Location Retrieval procedure allows the gsmSCF to know the location of the MS with the accuracy of the cell ID for GSM or SAI for UMTS at every time it is possible. With the current R99 specification 23.018 (v 3.9.0), it is possible to retrieve the current SAI in UMTS in the following case : - there is no MS connexion. However, the RANAP message Location Reporting Control (see 25.413) allows the CN (SGSN or MSC) to retrieve the current SAI from the RNC in case an MS connexion exists. Therefore, we propose to correct the handling specified in the 23.018. We also have the following corrections in 23.018 : - § 7.2.3.4 : adding the SAI in "Location Information Received" - SDL 57 : adding the SAI case in the part concerning the interrogation to the SGSN (see § 6.3.6. in the 23.060 specification about the handling of the MS Information Request operation). - SDL 64 : allowing the MSC to send a "Location Reporting Control" to retrieve the SAI from the RNC. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  |  | Lack of clarity on requirements for ATI. Inconsistencies between 23.060 and 23.018 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  | 7.2.3. and 7.2.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | O&M Specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

How to create CRs using this form:

Comprehensive information and tips about how to create CRs can be found at: http://www.3gpp.org/3G_Specs/CRs.htm.  Below is a brief summary:

1)	Fill out the above form. The symbols above marked  contain pop-up help information about the field that they are closest to.

2)	Obtain the latest version for the release of the specification to which the change is proposed. Use the MS Word "revision marks"  feature (also known as "track changes") when making the changes. All 3GPP specifications can be downloaded from the 3GPP server under ftp://www.3gpp.org/specs/ For the latest version, look for the directory name with the latest date e.g. 2000-09 contains the specifications resulting from the September 2000 TSG meetings.

3)	With "track changes" disabled, paste the entire CR form (use CTRL-A to select it) into the specification just in front of the clause containing the first piece of changed text.  Delete those parts of the specification which are not relevant to the change request.

*** First Change ***

#### Process PSI_VLR

Sheet 1: the procedure Check_Parameters is specified in subclause 7.2.2.2. If the HLR requests neither location information nor subscriber state, the VLR treats this as a missing parameter.

#### 7.2.3.4	Procedure Retrieve_Location_Info_VLR

The variable Current location retrieved is used to indicate that the location information was obtained by paging the MS.

The test "Retrieve location info from SGSN" takes the "Yes" exit if:

-	the Gs interface is implemented; and

-	there is an association established between the MSC/VLR and the SGSN.

The stored location information consists of:

-	the service area ID (for UMTS) or cell ID (for GSM) of the cell in which the MS last established radio contact;

-	the location number, geodetic information and geographical information derived from the service area ID or cell ID if the VLR is capable of doing so (the mapping from service area ID or cell ID to location number is network-specific and outside the scope of the UMTS and GSM standards);

-	the age of the location information.

The received location information consists of:

- the service area ID (UMTS) or cell ID (GSM) received in the paging response message or in the MS Information Ack ;

- the location number, geodetic information and geographical information derived from the service area ID or cell ID if the VLR is capable of doing so (the mapping from service area ID or cell ID to location number is network-specific and outside the scope of the UMTS and GSM standards);

-	the age of the location information.

The output signal Send MS information towards the SGSN indicates that the required information is mobile location information.

The derivation of the location number, geodetic information and geographical information from the received service area ID or cell ID is a VLR operator option (the mapping from service area ID or cell ID to location number is network-specific and outside the scope of the UMTS and GSM standards).

The question box "Report upon change of service area" means that the MSC has performed the Location Reporting Control procedure with the Request Type IE set to "change of service area" [26]

Figure 57: Procedure Retrieve_Location_Info_VLR

*** Second Change ***

Figure 64: Procedure Current_Location_Page_MSC