3GPP TSG CN WG2 Meeting #31	N2-030536

Bangkok, Thailand, 27th – 31th October 2003

Title:	Contents of each CAMEL phase

Source:	CN2 Chairman

This is an informative document to summarise functionalities of each CAMEL phase.

# CAMEL phase 1:

## CAMEL phase 1 functionality:

| Functionality | Purpose / supports | Restrictions |
| --- | --- | --- |
| Triggering on MO and MT calls. | - Call screening - Simple VPN - Call Forwarding - Call Redirection - Call Rerouting - Homing of pre-paid calls to HPLMN | - No IN announcements supported - No charging operations - Limited set of detection points, e.g. the hunting services are not possible. - MT calls trigger in the GMSC only. - Not possible to activate Call Forwardings with VPN short codes. |
| Any Time Interrogation | - The SCP may whenever request HLR to provide subscriber status and/or location information. | - The location is not up-to-date unless the mobile has an active call ongoing. |

# CAMEL phase 2:

## CAMEL phase 2 functionality:

| Functionality | Purpose / supports | Restrictions |
| --- | --- | --- |
| Triggering on MO and MT calls. | - Call screening - Simple VPN - Call Forwarding - Call Redirection - Call Rerouting - In-band user interaction (DTMF) - Tones and announcements | - MT calls trigger in the GMSC only. |
| Charging operations | - Pre-paid (MO, MT, CF) - Free-phone - Premium rate - Personal Discount - Location Dependent discount - Reverse Charging |  |
| SCP provided e-parameters | - MO AoC | - Inter-vendor inter-working problems. |
| New detection points (Abandon, Busy, NotReachable, NoAnswer, RouteSelectFailure cases) | - Cleaner termination of relationship. - Better pre-paid service. - Hunting type of services. |  |
| Triggering criteria | - To reduce SCP & SSP load |  |
| Any Time Interrogation | - The SCP may whenever request HLR to provide subscriber status and/or location information. | The location is not up-to-date unless the mobile has an active call ongoing. |
| USSD communication between MS and SCP | - USSD call-back for pre-paid roaming - Control & enquiry of IN services |  |
| Control of north American carrier information | - Enhances pre-paid - SCP based carrier selection - Selecting correct long distance network for user interaction |  |
| Notification of supplementary service (ECT, CD and MPTY) invocation to SCP | - Fraud control |  |

# CAMEL phase 3:

## CAMEL phase 3 functionality:

| Functionality | Purpose / supports | Restrictions |
| --- | --- | --- |
| Triggering on MO and MT calls. | - Call screening - Simple VPN - Call Forwarding - Call Redirection - Call Rerouting - In-band user interaction (DTMF) - Tones and announcements |  |
| Charging operations | - Pre-paid (MO, MT, CF) - Free-phone - Premium rate - Personal Discount - Location Dependent discount - Reverse Charging |  |
| SCP provided e-parameters | - MO & MT AoC |  |
| CAMEL2 new detection points (Abandon, Busy, NotReachable, NoAnswer, RouteSelectFailure cases) | - Cleaner termination of relationship. - Better pre-paid service. - Hunting type of services. |  |
| CAMEL3 new trigger detection points. | - To trigger on demand only, e.g. for hunting services. |  |
| Triggering criteria | - To reduce SCP & SSP load |  |
| Any Time Interrogation. Enhanced with CAMEL3 current location retrieval. | - The SCP may whenever request HLR to provide subscriber status and/or location information. |  |
| USSD communication between MS and SCP | - USSD call-back for pre-paid roaming - Control & enquiry of IN services |  |
| Control of north American carrier information | - Enhances pre-paid - SCP based carrier selection - Selecting correct long distance network for user interaction |  |
| Notification of supplementary service (ECT, CD and MPTY) invocation to SCP | - Fraud control |  |
| Call Gapping | - To control SCP overload situations within HPLMN | For CS calls only. Not for SMS or GPRS. |
| MT call triggering in VMSC-B | - To control MT supplementary services (CW, hold, CF, CD, ECT, MPTY) - Pre-paid for MT air-time charge (1st minute free, e.g. in USA) |  |
| Abandon as an EDP-R | - Improves charging |  |
| Enhanced FreeFormatCharging data (40 octets -> 160 octets) | - Makes the SCP service logic easier |  |
| Reporting of MSRN/FTN to SCP | - Charging control of optimally routed calls |  |
| SCP-HLR interface: ATM/ATSI/NSDC | - Multiple Subscriber Profile phase 2 - SCP can control CF, barring supplementary services. - Off-line subscription control based on VPLMN / time-of-day - Virtual operators |  |
| Subscribed dialled services (D-CSI) | - HPLMN specific service numbers |  |
| Serving Network Dialled services (N-CSI) | - VPLMN specific service numbers |  |
| MO SMS control (CS+PS) | - Enhances pre-paid. - Enhances VPN | - No MT SMS control. - Not content based |
| Inter-working with GPRS | - GPRS pre-paid | - Not content based |
| CS Mobility management notifications | - SCP can monitor location / reachability |  |
| Service Interaction Indicators | - SCP can control inter-working with supplementary services (CW, hold, CF, CD, ECT, MPTY). |  |

# CAMEL phase 4:

## CAMEL phase 4 functionality in Release 5:

| Functionality | Purpose / supports | Restrictions |
| --- | --- | --- |
| Triggering on MO and MT calls. | - Call screening - Simple VPN - Call Forwarding - Call Redirection - Call Rerouting - In-band user interaction (DTMF) - Tones and announcements |  |
| Charging operations | - Pre-paid (MO, MT, CF) - Free-phone - Premium rate - Personal Discount - Location Dependent discount - Reverse Charging |  |
| SCP provided e-parameters | - MO & MT AoC |  |
| CAMEL2 new detection points (Abandon, Busy, NotReachable, NoAnswer, RouteSelectFailure cases) | - Cleaner termination of relationship. - Better pre-paid service. - Hunting type of services. |  |
| CAMEL3 new trigger detection points. | - To trigger on demand only, e.g. for hunting services. |  |
| Triggering criteria | - To reduce SCP & SSP load |  |
| Any Time Interrogation. Enhanced with CAMEL3 current location retrieval. CAMEL4 enhances this to PS domain. | - The SCP may whenever request HLR to provide subscriber status and/or location information. |  |
| USSD communication between MS and SCP | - USSD call-back for pre-paid roaming - Control & enquiry of IN services |  |
| Control of north American carrier information | - Enhances pre-paid - SCP based carrier selection - Selecting correct long distance network for user interaction |  |
| Notification of supplementary service (ECT, CD and MPTY) invocation to SCP | - Fraud control |  |
| Call Gapping | - To control SCP overload situations within HPLMN | For CS calls only. Not for SMS or GPRS. |
| MT call triggering in VMSC-B | - To control MT supplementary services (CW, hold, CF, CD, ECT, MPTY) - Pre-paid for MT air-time charge (1st minute free, e.g. in USA) |  |
| Abandon as an EDP-R | - Improves charging |  |
| Enhanced FreeFormatCharging data (40 octets -> 160 octets) | - Makes the SCP service logic easier |  |
| Reporting of MSRN/FTN to SCP | - Charging control of optimally routed calls |  |
| SCP-HLR interface: ATM/ATSI/NSDC | - Multiple Subscriber Profile phase 2 - SCP can control CF, barring supplementary services. - Off-line subscription control based on VPLMN / time-of-day - Virtual operators |  |
| Subscribed dialled services (D-CSI) | - HPLMN specific service numbers |  |
| Serving Network Dialled services (N-CSI) | - VPLMN specific service numbers |  |
| SMS control (CS+PS) | - Enhances pre-paid. - Enhances VPN | - Not content based |
| Inter-working with GPRS | - GPRS pre-paid | - Not content based |
| CS Mobility management notifications | - SCP can monitor location / reachability |  |
| Service Interaction Indicators | - SCP can control inter-working with supplementary services (CW, hold, CF, CD, ECT, MPTY). |  |
| Interactions with Optimal Routing | - SCP can control usage of OR |  |
| Call Party Handling | - Wake-up calls - Conference calling - Parallel hunting - Click-to dial - IN based CCBS |  |
| Mid call procedure for MO and MT calls | - Control CPH configuration during call. - Automatic call collect |  |
| CAMEL for IMS | - IMS pre-paid - Convergence to CS and GPRS services |  |
| Inclusion of flexible tone injection | - Enhancements of pre-paid warning tones - Various informative tones |  |
| Provision of location information of called subscriber (Alerting phase) | - The location of B-subscriber at the beginning of the call provided to service logic |  |
| Notification of GPRS mobility management to CSE | - SCP can monitor location / reachability in PS side |  |
| Inclusion of ODB data in the CSE-HLR interface | - Enhancement of MSP ph 2 |  |
| Location information during an ongoing call (Handover DP) | - Charging based on current location, for inter-PLMN and/or inter-system handovers |  |
| Transfer of the IMEI (with software version) to the CSE | - allow the gsmSCF to determine information about the capabilities of the ME |  |
| Partial implementation of CAMEL phase 4 | - A functional entity (VMSC, GMSC or SGSN) may support the complete CAMEL phase 4 functionality or, as a network option, it may support the complete CAMEL phase 3 functionality and a partial implementation of CAMEL phase 4. |  |

## CAMEL phase 4 additions in Release 6

| Enhancements of Dialled Services | - Full control of Dialled Services |  |
| --- | --- | --- |
| Full CAMEL4 prepay support for SCUDIF | - The capability to report basic service code changes during ongoing call. - Prepay service can work with SCUDIF so that correct rate is applied to CS call depending whether it is speech or multimedia. Applies to all call cases (MO, MT, CF). |  |
| Change of position procedure armed with criteria | - Technical enhancement to reduce the signalling load |  |