3GPP TSG-CN2 Meeting #31	N2-030537

Bangkok, Thailand, October 27 -31, 2003

Source:	China Mobile

Title:	Correction for data volume report in GPRS data volume threshold procedure

Agenda item:	CAMEL4

Document for:	Discussion and Decision

# Introduction

CSE uses threshold in AC GPRA to control the amount of data transmitted by and transmitted to the served subscribe or the used time. The VPLMN should report the amount of data or the used time to CSE in ACR GPRS.

As we know, data will always be transmitted in a package. It will scarcely happen that the transmitted volume is exactly reach the granted volume. in fact, the VPLMN will only know the threshold is reached when the volume transmitted exceed the grant volume.

# 2. Discussion

## 2.1. Caused problem

We can find a mismatch in the specification showed as below:

(in 22078,chapter 10.7)The following information shall be provided to the CSE if available:

-	Charge result (elapsed time or total amount of data transmitted)

(in 29078,chapter 13.2.1.1) -	maxTransferredVolume:

This parameter specifies the maximum volume that may be transferred before a ApplyChargingReportGPRS shall be sent to the gsmSCF.

According to the specification, if VPLMN reports the volume transmitted to CSE which is bigger than the granted volume, CSE will reject the ACR GPRS operation and the service will be terminated.

For an example, the granted volume is 10K, we suppose 9K volume have been transmitted and the next data package is 2K. If VPLMN reports 10K to CSE, the 1K volume left should be kept in a counter and reported in the next ACR GPRS. If VPLMN reports 11K to CSE, the CSE will reject the service with a reason of UnexpectedDataValue.

## 2.2. How to solve the problem

there are there way to solve the problem described as below:

-	VPLMN reports the granted volume to CSE when the transmitted volume exceeds the granted volume, and left the remainder in a counter;

-	VPLMN reports the transmitted volume to CSE, CSE accept the volume which is bigger than the granted volume and continue.

-	add a new parameter in ACR GPRS for the dispersion amount between transmitted volume and granted volume.

# 3. Proposal

China mobile suggests solution 1 which is VPLMN reports the granted volume to CSE when the transmitted volume exceeds the granted volume, and left the remainder in a counter.

If any of the solutuins accepted, we should modify the description of threshold reaching condition and the report method of transmitted volume in 22078, 23078, and 29078.

China mobile would like to make the CR accordingly.