3GPP TSG-CT WG4 Meeting #115e	C4-231072

# E-Meeting, 17th – 21st April 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.272 | CR | 0844 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Reachability Cause in immediate reports |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2023-4-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | When the MME reports UE reachability directly to the SCEF, the report can convey a reachability cause, indicating whether the UE is in connected mode or reachable for paging (see 29.128 clause 6.4.3). However, immediate reachability reports in S6a IDA response messages cannot convey such information. It is proposed to align immediate reports in S6a IDA with reports in T6a RIR wrt the possibility to report the reachability cause. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add Reachability-Cause AVP to MonitoringEvent-Report. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment between direct reports (MME to SCEF) and immediate (indirect) reports (MME to HSS to SCEF). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1, 7.3.196 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 7.3.1	General

The following table specifies the Diameter AVPs defined for the S6a/S6d interface protocol, the S7a/S7d interface protocol and the S13/S13' interface protocol, their AVP Code values, types, possible flag values and whether or not the AVP may be encrypted. The Vendor-ID header of all AVPs defined in this specification shall be set to 3GPP (10415).

For all AVPs which contain bit masks and are of the type Unsigned32, e.g., ULR-Flags, DSR-Flags, PUA-Flags, etc., bit 0 shall be the least significant bit. For example, to get the value of bit 0, a bit mask of 0x0001 should be used.

Table 7.3.1/1: S6a/S6d, S7a/S7d and S13/S13' specific DiameterAVPs

|  |  |  |  | AVP Flag rules |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Attribute Name | AVP Code | Clause defined | Value Type | Must | May | Should not | Must not | May Encr. |
| Subscription-Data | 1400 | 7.3.2 | Grouped | M, V |  |  |  | No |
| Terminal-Information | 1401 | 7.3.3 | Grouped | M, V |  |  |  | No |
| IMEI | 1402 | 7.3.4 | UTF8String | M, V |  |  |  | No |
| Software-Version | 1403 | 7.3.5 | UTF8String | M, V |  |  |  | No |
| QoS-Subscribed | 1404 | 7.3.77 | OctetString | M, V |  |  |  | No |
| ULR-Flags | 1405 | 7.3.7 | Unsigned32 | M, V |  |  |  | No |
| ULA-Flags | 1406 | 7.3.8 | Unsigned32 | M, V |  |  |  | No |
| Visited-PLMN-Id | 1407 | 7.3.9 | OctetString | M, V |  |  |  | No |
| Requested-EUTRAN-Authentication-Info | 1408 | 7.3.11 | Grouped | M, V |  |  |  | No |
| Requested-UTRAN-GERAN-Authentication-Info | 1409 | 7.3.12 | Grouped | M, V |  |  |  | No |
| Number-Of-Requested-Vectors | 1410 | 7.3.14 | Unsigned32 | M, V |  |  |  | No |
| Re-Synchronization-Info | 1411 | 7.3.15 | OctetString | M, V |  |  |  | No |
| Immediate-Response-Preferred | 1412 | 7.3.16 | Unsigned32 | M, V |  |  |  | No |
| Authentication-Info | 1413 | 7.3.17 | Grouped | M, V |  |  |  | No |
| E-UTRAN-Vector | 1414 | 7.3.18 | Grouped | M, V |  |  |  | No |
| UTRAN-Vector | 1415 | 7.3.19 | Grouped | M, V |  |  |  | No |
| GERAN-Vector | 1416 | 7.3.20 | Grouped | M, V |  |  |  | No |
| Network-Access-Mode | 1417 | 7.3.21 | Enumerated | M, V |  |  |  | No |
| HPLMN-ODB | 1418 | 7.3.22 | Unsigned32 | M, V |  |  |  | No |
| Item-Number | 1419 | 7.3.23 | Unsigned32 | M, V |  |  |  | No |
| Cancellation-Type | 1420 | 7.3.24 | Enumerated | M, V |  |  |  | No |
| DSR-Flags | 1421 | 7.3.25 | Unsigned32 | M, V |  |  |  | No |
| DSA-Flags | 1422 | 7.3.26 | Unsigned32 | M, V |  |  |  | No |
| Context-Identifier | 1423 | 7.3.27 | Unsigned32 | M, V |  |  |  | No |
| Subscriber-Status | 1424 | 7.3.29 | Enumerated | M, V |  |  |  | No |
| Operator-Determined-Barring | 1425 | 7.3.30 | Unsigned32 | M, V |  |  |  | No |
| Access-Restriction-Data | 1426 | 7.3.31 | Unsigned32 | M, V |  |  |  | No |
| APN-OI-Replacement | 1427 | 7.3.32 | UTF8String | M, V |  |  |  | No |
| All-APN-Configurations-Included-Indicator | 1428 | 7.3.33 | Enumerated | M, V |  |  |  | No |
| APN-Configuration-Profile | 1429 | 7.3.34 | Grouped | M, V |  |  |  | No |
| APN-Configuration | 1430 | 7.3.35 | Grouped | M, V |  |  |  | No |
| EPS-Subscribed-QoS-Profile | 1431 | 7.3.37 | Grouped | M, V |  |  |  | No |
| VPLMN-Dynamic-Address-Allowed | 1432 | 7.3.38 | Enumerated | M, V |  |  |  | No |
| STN-SR | 1433 | 7.3.39 | OctetString | M, V |  |  |  | No |
| Alert-Reason | 1434 | 7.3.83 | Enumerate | M, V |  |  |  | No |
| AMBR | 1435 | 7.3.41 | Grouped | M, V |  |  |  | No |
| CSG-Subscription-Data | 1436 | 7.3.78 | Grouped | M. V |  |  |  | No |
| CSG-Id | 1437 | 7.3.79 | Unsigned32 | M, V |  |  |  | No |
| PDN-GW-Allocation-Type | 1438 | 7.3.44 | Enumerated | M, V |  |  |  | No |
| Expiration-Date | 1439 | 7.3.80 | Time | M, V |  |  |  | No |
| RAT-Frequency-Selection-Priority-ID | 1440 | 7.3.46 | Unsigned32 | M, V |  |  |  | No |
| IDA-Flags | 1441 | 7.3.47 | Unsigned32 | M, V |  |  |  | No |
| PUA-Flags | 1442 | 7.3.48 | Unsigned32 | M, V |  |  |  | No |
| NOR-Flags | 1443 | 7.3.49 | Unsigned32 | M, V |  |  |  | No |
| User-Id | 1444 | 7.3.50 | UTF8String | V |  |  | M | No |
| Equipment-Status | 1445 | 7.3.51 | Enumerated | M, V |  |  |  | No |
| Regional-Subscription-Zone-Code | 1446 | 7.3.52 | OctetString | M, V |  |  |  | No |
| RAND | 1447 | 7.3.53 | OctetString | M, V |  |  |  | No |
| XRES | 1448 | 7.3.54 | OctetString | M, V |  |  |  | No |
| AUTN | 1449 | 7.3.55 | OctetString | M, V |  |  |  | No |
| KASME | 1450 | 7.3.56 | OctetString | M, V |  |  |  | No |
| Trace-Collection-Entity | 1452 | 7.3.98 | Address | M, V |  |  |  | No |
| Kc | 1453 | 7.3.59 | OctetString | M, V |  |  |  | No |
| SRES | 1454 | 7.3.60 | OctetString | M, V |  |  |  | No |
| PDN-Type | 1456 | 7.3.62 | Enumerated | M, V |  |  |  | No |
| Roaming-Restricted-Due-To-Unsupported-Feature | 1457 | 7.3.81 | Enumerated | M, V |  |  |  | No |
| Trace-Data | 1458 | 7.3.63 | Grouped | M, V |  |  |  | No |
| Trace-Reference | 1459 | 7.3.64 | OctetString | M, V |  |  |  | No |
| Trace-Depth | 1462 | 7.3.67 | Enumerated | M, V |  |  |  | No |
| Trace-NE-Type-List | 1463 | 7.3.68 | OctetString | M, V |  |  |  | No |
| Trace-Interface-List | 1464 | 7.3.69 | OctetString | M, V |  |  |  | No |
| Trace-Event-List | 1465 | 7.3.70 | OctetString | M, V |  |  |  | No |
| OMC-Id | 1466 | 7.3.71 | OctetString | M, V |  |  |  | No |
| GPRS-Subscription-Data | 1467 | 7.3.72 | Grouped | M, V |  |  |  | No |
| Complete-Data-List-Included-Indicator | 1468 | 7.3.73 | Enumerated | M, V |  |  |  | No |
| PDP-Context | 1469 | 7.3.74 | Grouped | M, V |  |  |  | No |
| PDP-Type | 1470 | 7.3.75 | OctetString | M, V |  |  |  | No |
| 3GPP2-MEID | 1471 | 7.3.6 | OctetString | M, V |  |  |  | No |
| Specific-APN-Info | 1472 | 7.3.82 | Grouped | M, V |  |  |  | No |
| LCS-Info | 1473 | 7.3.84 | Grouped | M, V |  |  |  | No |
| GMLC-Number | 1474 | 7.3.85 | OctetString | M, V |  |  |  | No |
| LCS-PrivacyException | 1475 | 7.3.86 | Grouped | M, V |  |  |  | No |
| SS-Code | 1476 | 7.3.87 | OctetString | M, V |  |  |  | No |
| SS-Status | 1477 | 7.3.88 | OctetString | M, V |  |  |  | No |
| Notification-To-UE-User | 1478 | 7.3.89 | Enumerated | M, V |  |  |  | No |
| External-Client | 1479 | 7.3.90 | Grouped | M, V |  |  |  | No |
| Client-Identity | 1480 | 7.3.91 | OctetString | M, V |  |  |  | No |
| GMLC-Restriction | 1481 | 7.3.92 | Enumerated | M, V |  |  |  | No |
| PLMN-Client | 1482 | 7.3.93 | Enumerated | M, V |  |  |  | No |
| Service-Type | 1483 | 7.3.94 | Grouped | M, V |  |  |  | No |
| ServiceTypeIdentity | 1484 | 7.3.95 | Unsigned32 | M, V |  |  |  | No |
| MO-LR | 1485 | 7.3.96 | Grouped | M, V |  |  |  | No |
| Teleservice-List | 1486 | 7.3.99 | Grouped | M, V |  |  |  | No |
| TS-Code | 1487 | 7.3.100 | OctetString | M, V |  |  |  | No |
| Call-Barring-Info | 1488 | 7.3.101 | Grouped | M, V |  |  |  | No |
| SGSN-Number | 1489 | 7.3.102 | OctetString | M, V |  |  |  | No |
| IDR-Flags | 1490 | 7.3.103 | Unsigned32 | M, V |  |  |  | No |
| ICS-Indicator | 1491 | 7.3.104 | Enumerated | V |  |  | M | No |
| IMS-Voice-Over-PS-Sessions-Supported | 1492 | 7.3.106 | Enumerated | V |  |  | M | No |
| Homogeneous-Support-of-IMS-Voice-Over-PS-Sessions | 1493 | 7.3.107 | Enumerated | V |  |  | M | No |
| Last-UE-Activity-Time | 1494 | 7.3.108 | Time | V |  |  | M | No |
| EPS-User-State | 1495 | 7.3.110 | Grouped | V |  |  | M | No |
| EPS-Location-Information | 1496 | 7.3.111 | Grouped | V |  |  | M | No |
| MME-User-State | 1497 | 7.3.112 | Grouped | V |  |  | M | No |
| SGSN-User-State | 1498 | 7.3.113 | Grouped | V |  |  | M | No |
| User-State | 1499 | 7.3.114 | Enumerated | V |  |  | M | No |
| MME-Location Information | 1600 | 7.3.115 | Grouped | V |  |  | M | No |
| SGSN-Location-Information | 1601 | 7.3.116 | Grouped | V |  |  | M | No |
| E-UTRAN-Cell-Global-Identity | 1602 | 7.3.117 | OctetString | V |  |  | M | No |
| Tracking-Area-Identity | 1603 | 7.3.118 | OctetString | V |  |  | M | No |
| Cell-Global-Identity | 1604 | 7.3.119 | OctetString | V |  |  | M | No |
| Routing-Area-Identity | 1605 | 7.3.120 | OctetString | V |  |  | M | No |
| Location-Area-Identity | 1606 | 7.3.121 | OctetString | V |  |  | M | No |
| Service-Area-Identity | 1607 | 7.3.122 | OctetString | V |  |  | M | No |
| Geographical-Information | 1608 | 7.3.123 | OctetString | V |  |  | M | No |
| Geodetic-Information | 1609 | 7.3.124 | OctetString | V |  |  | M | No |
| Current-Location-Retrieved | 1610 | 7.3.125 | Enumerated | V |  |  | M | No |
| Age-Of-Location-Information | 1611 | 7.3.126 | Unsigned32 | V |  |  | M | No |
| Active-APN | 1612 | 7.3.127 | Grouped | V |  |  | M | No |
| Error-Diagnostic | 1614 | 7.3.128 | Enumerated | V |  |  | M | No |
| Ext-PDP-Address | 1621 | 7.3.129 | Address | V |  |  | M | No |
| UE-SRVCC-Capability | 1615 | 7.3.130 | Enumerated | V |  |  | M | No |
| MPS-Priority | 1616 | 7.3.131 | Unsigned32 | V |  |  | M | No |
| VPLMN-LIPA-Allowed | 1617 | 7.3.132 | Enumerated | V |  |  | M | No |
| LIPA-Permission | 1618 | 7.3.133 | Enumerated | V |  |  | M | No |
| Subscribed-Periodic-RAU-TAU-Timer | 1619 | 7.3.134 | Unsigned32 | V |  |  | M | No |
| Ext-PDP-Type | 1620 | 7.3.75A | OctetString | V |  |  | M | No |
| SIPTO-Permission | 1613 | 7.3.135 | Enumerated | V |  |  | M | No |
| MDT-Configuration | 1622 | 7.3.136 | Grouped | V |  |  | M | No |
| Job-Type | 1623 | 7.3.137 | Enumerated | V |  |  | M | No |
| Area-Scope | 1624 | 7.3.138 | Grouped | V |  |  | M | No |
| List-Of-Measurements | 1625 | 7.3.139 | Unsigned32 | V |  |  | M | No |
| Reporting-Trigger | 1626 | 7.3.140 | Unsigned32 | V |  |  | M | No |
| Report-Interval | 1627 | 7.3.141 | Enumerated | V |  |  | M | No |
| Report-Amount | 1628 | 7.3.142 | Enumerated | V |  |  | M | No |
| Event-Threshold-RSRP | 1629 | 7.3.143 | Unsigned32 | V |  |  | M | No |
| Event-Threshold-RSRQ | 1630 | 7.3.144 | Unsigned32 | v |  |  | M | No |
| Logging-Interval | 1631 | 7.3.145 | Enumerated | V |  |  | M | No |
| Logging-Duration | 1632 | 7.3.146 | Enumerated | V |  |  | M | No |
| Relay-Node-Indicator | 1633 | 7.3.147 | Enumerated | V |  |  | M | No |
| MDT-User-Consent | 1634 | 7.3.148 | Enumerated | V |  |  | M | No |
| PUR-Flags | 1635 | 7.3.149 | Unsigned32 | V |  |  | M | No |
| Subscribed-VSRVCC | 1636 | 7.3.150 | Enumerated | V |  |  | M | No |
| Equivalent-PLMN-List | 1637 | 7.3.151 | Grouped | V |  |  | M | No |
| CLR-Flags | 1638 | 7.3.152 | Unsigned32 | V |  |  | M | No |
| UVR-Flags | 1639 | 7.3.153 | Unsigned32 | M, V |  |  |  | No |
| UVA-Flags | 1640 | 7.3.154 | Unsigned32 | M, V |  |  |  | No |
| VPLMN-CSG-Subscription-Data | 1641 | 7.3.155 | Grouped | M, V |  |  |  | No |
| Time-Zone | 1642 | 7.3.163 | UTF8String | V |  |  | M | No |
| A-MSISDN | 1643 | 7.3.157 | OctetString | V |  |  | M | No |
| MME-Number-for-MT-SMS | 1645 | 7.3.159 | OctetString | V |  |  | M | No |
| SMS-Register-Request | 1648 | 7.3.162 | Enumerated | V |  |  | M | No |
| Local-Time-Zone | 1649 | 7.3.156 | Grouped | V |  |  | M | No |
| Daylight-Saving-Time | 1650 | 7.3.164 | Enumerated | V |  |  | M | No |
| Subscription-Data-Flags | 1654 | 7.3.165 | Unsigned32 | V |  |  | M | No |
| Measurement-Period-LTE | 1655 | 7.3.166 | Enumerated | V |  |  | M | No |
| Measurement-Period-UMTS | 1656 | 7.3.167 | Enumerated | V |  |  | M | No |
| Collection-Period-RRM-LTE | 1657 | 7.3.168 | Enumerated | V |  |  | M | No |
| Collection-Period-RRM-UMTS | 1658 | 7.3.169 | Enumerated | V |  |  | M | No |
| Positioning-Method | 1659 | 7.3.170 | OctetString | V |  |  | M | No |
| Measurement-Quantity | 1660 | 7.3.171 | OctetString | V |  |  | M | No |
| Event-Threshold-Event-1F | 1661 | 7.3.172 | Integer32 | V |  |  | M | No |
| Event-Threshold-Event-1I | 1662 | 7.3.173 | Integer32 | V |  |  | M | No |
| Restoration-Priority | 1663 | 7.3.174 | Unsigned32 | V |  |  | M | No |
| SGs-MME-Identity | 1664 | 7.3.175 | UTF8String | V |  |  | M | No |
| SIPTO-Local-Network-Permission | 1665 | 7.3.176 | Unsigned32 | V |  |  | M | No |
| Coupled-Node-Diameter-ID | 1666 | 7.3.177 | DiameterIdentity | V |  |  | M | No |
| WLAN-offloadability | 1667 | 7.3.181 | Grouped | V |  |  | M | No |
| WLAN-offloadability-EUTRAN | 1668 | 7.3.182 | Unsigned32 | V |  |  | M | No |
| WLAN-offloadability-UTRAN | 1669 | 7.3.183 | Unsigned32 | V |  |  | M | No |
| Reset-ID | 1670 | 7.3.184 | OctetString | V |  |  | M | No |
| MDT-Allowed-PLMN-Id | 1671 | 7.3.185 | OctetString | V |  |  | M | No |
| Adjacent-PLMNs | 1672 | 7.3.186 | Grouped | V |  |  | M | No |
| Adjacent-Access-Restriction-Data | 1673 | 7.3.187 | Grouped | V |  |  | M | No |
| DL-Buffering-Suggested-Packet-Count | 1674 | 7.3.188 | Integer32 | V |  |  | M | No |
| IMSI-Group-Id | 1675 | 7.3.189 | Grouped | V |  |  | M | No |
| Group-Service-Id | 1676 | 7.3.190 | Unsigned32 | V |  |  | M | No |
| Group-PLMN-Id | 1677 | 7.3.191 | OctetString | V |  |  | M | No |
| Local-Group-Id | 1678 | 7.3.192 | OctetString | V |  |  | M | No |
| AIR-Flags | 1679 | 7.3.201 | Unsigned32 | V |  |  | M | No |
| UE-Usage-Type | 1680 | 7.3.202 | Unsigned32 | V |  |  | M | No |
| Non-IP-PDN-Type-Indicator | 1681 | 7.3.204 | Enumerated | V |  |  | M | No |
| Non-IP-Data-Delivery-Mechanism | 1682 | 7.3.205 | Unsigned32 | V |  |  | M | No |
| Additional-Context-ID | 1683 | 7.3.206 | Unsigned32 | V |  |  | M | No |
| SCEF-Realm | 1684 | 7.3.207 | DiameterIdentity | V |  |  | M | No |
| Subscription-Data-Deletion | 1685 | 7.3.208 | Grouped | V |  |  | M | No |
| Preferred-Data-Mode | 1686 | 7.3.209 | Unsigned32 | V |  |  | M | No |
| Emergency-Info | 1687 | 7.3.210 | Grouped | V |  |  | M | No |
| V2X-Subscription-Data | 1688 | 7.3.212 | Grouped | V |  |  | M | No |
| V2X-Permission | 1689 | 7.3.213 | Unsigned32 | V |  |  | M | No |
| PDN-Connection-Continuity | 1690 | 7.3.214 | Unsigned32 | V |  |  | M | No |
| eDRX-Cycle-Length | 1691 | 7.3.215 | Grouped | V |  |  | M | No |
| eDRX-Cycle-Length-Value | 1692 | 7.3.216 | OctetString | V |  |  | M | No |
| UE-PC5-AMBR | 1693 | 7.3.217 | Unsigned32 | V |  |  | M | No |
| MBSFN-Area | 1694 | 7.3.219 | Grouped | V |  |  | M | No |
| MBSFN-Area-ID | 1695 | 7.3.220 | Unsigned32 | V |  |  | M | No |
| Carrier-Frequency | 1696 | 7.3.221 | Unsigned32 | V |  |  | M | No |
| RDS-Indicator | 1697 | 7.3.222 | Enumerated | V |  |  | M | No |
| Service-Gap-Time | 1698 | 7.3.223 | Unsigned32 | V |  |  | M | No |
| Aerial-UE-Subscription-Information | 1699 | 7.3.224 | Unsigned32 | V |  |  | M | No |
| Broadcast-Location-Assistance-Data-Types | 1700 | 7.3.225 | Unsigned64 | V |  |  | M | No |
| Paging-Time-Window | 1701 | 7.3.226 | Grouped | V |  |  | M | No |
| Operation-Mode | 1702 | 7.3.227 | Unsigned32 | V |  |  | M | No |
| Paging-Time-Window-Length | 1703 | 7.3.228 | OctetString | V |  |  | M | No |
| Core-Network-Restrictions | 1704 | 7.3.230 | Unsigned32 | V |  |  | M | No |
| eDRX-Related-RAT | 1705 | 7.3.229 | Grouped | V |  |  | M | No |
| Interworking-5GS-Indicator | 1706 | 7.3.231 | Enumerated | V |  |  | M | No |
| Ethernet-PDN-Type-Indicator | 1707 | 7.3.232 | Enumerated | V |  |  | M | No |
| Subscribed-ARPI | 1708 | 7.3.233 | Unsigned32 | V |  |  | M | No |
| IAB-Operation-Permission | 1709 | 7.3.234 | Enumerated | V |  |  | M | No |
| V2X-Subscription-Data-Nr | 1710 | 7.3.235 | Grouped | V |  |  | M | No |
| UE-PC5-QoS | 1711 | 7.3.236 | Grouped | V |  |  | M | No |
| PC5-QoS-Flow | 1712 | 7.3.237 | Grouped | V |  |  | M | No |
| 5QI | 1713 | 7.3.238 | Integer32 | V |  |  | M | No |
| PC5-Flow-Bitrates | 1714 | 7.3.239 | Grouped | V |  |  | M | No |
| Guaranteed-Flow-Bitrates | 1715 | 7.3.240 | Integer32 | V |  |  | M | No |
| Maximum-Flow-Bitrates | 1716 | 7.3.241 | Integer32 | V |  |  | M | No |
| PC5-Range | 1717 | 7.3.242 | Integer32 | V |  |  | M | No |
| PC5-Link-AMBR | 1718 | 7.3.243 | Integer32 | V |  |  | M | No |
| Third-Context-Identifier | 1719 | 7.3.244 | Unsigned32 | V |  |  | M | No |
| NOTE 1: The AVP header bit denoted as "M", indicates whether support of the AVP is required. The AVP header bit denoted as "V", indicates whether the optional Vendor-ID field is present in the AVP header. For further details, see IETF RFC 6733 [61]. NOTE 2: If the M-bit is set for an AVP and the receiver does not understand the AVP, it shall return a rejection. If the M-bit is not set for an AVP, the receiver shall not return a rejection, whether or not it understands the AVP. If the receiver understands the AVP but the M-bit value does not match with the definition in this table, the receiver shall ignore the M-bit. |  |  |  |  |  |  |  |  |

The following table specifies the Diameter AVPs re-used by the S6a/S6d interface protocol from existing Diameter Applications, including a reference to their respective specifications and when needed, a short description of their use within S6a and S6d.

Any other AVPs from existing Diameter Applications, except for the AVPs from Diameter base protocol specified in IETF RFC 6733 [61], do not need to be supported. The AVPs from the Diameter base protocol specified in IETF RFC 6733 [61] are not included in table 7.3.1/2, but they may be re-used for the S6a/S6d protocol, the S7a/S7protocol and the S13/S13' protocol.

Table 7.3.1/2: S6a/S6d, S7a/S7d and S13/S13' re-used Diameter AVPs

| Attribute Name | Reference | Comments | M-bit |
| --- | --- | --- | --- |
| Service-Selection | IETF RFC 5778 [20] | See clause 7.3.36 |  |
| 3GPP-Charging-Characteristics | 3GPP TS 29.061 [21] | See 3GPP TS 32.251 [33] Annex A and 3GPP TS 32.298 [22] clause 5.1.2.2.7 This attribute holds the EPS PDN Connection Charging Characteristics data for an EPS APN Configuration, or the PDP context Charging Characteristics for GPRS PDP context, or the Subscribed Charging Characteristics data for the subscriber level 3GPP Charging Characteristics; refer to 3GPP TS 23.008 [30]. |  |
| Supported-Features | 3GPP TS 29.229 [9] |  |  |
| Feature-List-ID | 3GPP TS 29.229 [9] |  |  |
| Feature-List | 3GPP TS 29.229 [9] | See clause 7.3.10 |  |
| Served-Party-IP-Address | 3GPP TS 32.299 [8] | holds the PDN IP Address of the user |  |
| QoS-Class-Identifier | 3GPP TS 29.212 [10] |  |  |
| Allocation-Retention-Priority | 3GPP TS 29.212 [10] | See clause 7.3.40 |  |
| Priority-Level | 3GPP TS 29.212 [10] |  |  |
| Pre-emption-Capability | 3GPP TS 29.212 [10] |  |  |
| Pre-emption-Vulnerability | 3GPP TS 29.212 [10] |  |  |
| Max-Requested-Bandwidth-DL | 3GPP TS 29.214 [11] |  |  |
| Max-Requested-Bandwidth-UL | 3GPP TS 29.214 [11] |  |  |
| Extended-Max-Requested-BW-DL | 3GPP TS 29.214 [11] |  |  |
| Extended-Max-Requested-BW-UL | 3GPP TS 29.214 [11] |  |  |
| RAT-Type | 3GPP TS 29.212 [10] | See clause 7.3.13 | Must set |
| MSISDN | 3GPP TS 29.329 [25] |  |  |
| MIP6-Agent-Info | IETF RFC 5447 [26] |  |  |
| MIP-Home-Agent-Address | IETF RFC 4004 [27] |  |  |
| MIP-Home-Agent-Host | IETF RFC 4004 [27] |  |  |
| PDP-Address | 3GPP TS 32.299 [8] |  |  |
| Confidentiality-Key | 3GPP TS 29.229 [9] | See clause 7.3.57 |  |
| Integrity-Key | 3GPP TS 29.229 [9] | See clause 7.3.58 |  |
| Visited-Network-Identifier | 3GPP TS 29.229 [9] | See clause 7.3.105 | Must not set |
| GMLC-Address | 3GPP TS 29.173 [37] | See clause 7.3.109 | Must not set |
| User-CSG-Information | 3GPP TS 32.299 [8] |  | Must not set |
| ProSe-Subscription-Data | 3GPP TS 29.344 [49] | See clause 7.3.180 | Must not set |
| OC-Supported-Features | IETF RFC 7683 [50] | See clause 7.3.178 | Must not set |
| OC-OLR | IETF RFC 7683 [50] | See clause 7.3.179 | Must not set |
| SCEF-Reference-ID | 3GPP TS 29.336 [54] |  | Must not set |
| SCEF-ID | 3GPP TS 29.336 [54] |  | Must not set |
| AESE-Communication-Pattern | 3GPP TS 29.336 [54] | see clause 7.3.193 | Must not set |
| Communication-Pattern-set | 3GPP TS 29.336 [54] | see clause 7.3.194 | Must not set |
| Monitoring-Event-Configuration | 3GPP TS 29.336 [54] | See clause 7.3.195 | Must not set |
| Monitoring-Event-Report | 3GPP TS 29.336 [54] | See clause 7.3.196 | Must not set |
| UE-Reachability-Configuration | 3GPP TS 29.336 [54] | See clause 7.3.197 | Must not set |
| eNodeB-ID | 3GPP TS 29.217 [56] | See clause 7.3.198 | Must not set |
| SCEF-Reference-ID-for-Deletion | 3GPP TS 29.336 [54] |  | Must not set |
| Monitoring-Type | 3GPP TS 29.336 [54] |  | Must not set |
| Maximum-Number-of-Reports | 3GPP TS 29.336 [54] |  | Must not set |
| Monitoring-Duration | 3GPP TS 29.336 [54] |  | Must not set |
| Charged-Party | 3GPP TS 29.336 [54] |  | Must not set |
| Location-Information-Configuration | 3GPP TS 29.336 [54] |  | Must not set |
| Reachability-Type | 3GPP TS 29.336 [54] |  | Must not set |
| Maximum-Response-Time | 3GPP TS 29.336 [54] |  | Must not set |
| Reachability-Information | 3GPP TS 29.336 [54] |  | Must not set |
| Reachability-Cause | 3GPP TS 29.128 [63] |  | Must not set |
| Monitoring-Event-Config-Status | 3GPP TS 29.336 [54] |  | Must not set |
| Supported-Services | 3GPP TS 29.336 [54] | See clause 7.3.199 | Must not set |
| Supported-Monitoring-Events | 3GPP TS 29.336 [54] | See clause 7.3.200 | Must not set |
| DRMP | IETF RFC 7944 [57] | See clause 7.3.203 | Must not set |
| Reference-ID-Validity-Time | 3GPP TS 29.336 [54] |  | Must not set |
| Maximum-UE-Availability-Time | 3GPP TS 29.338 [48] | See clause 5.3.3.22 | Must not set |
| Emergency-Services | 3GPP TS 29.273 [59] |  |  |
| Load | IETF RFC 8583 [60] | See clause 7.3.211 | Must not set |
| Extended-eNodeB-ID | 3GPP TS 29.217 [56] | See clause 7.3.218 | Must not set |
| External-Identifier | 3GPP TS 29.336 [54] |  | Must not set |
| Loss-Of-Connectivity-Reason | 3GPP TS 29.336 [54] |  | Must not set |
| Active-Time | 3GPP TS 29.128 [63] |  | Must not set |
| Idle-Status-Indication | 3GPP TS 29.128 [63] |  | Must not set |
| MTC-Provider-Info | 3GPP TS 29.336 [54] |  | Must not set |
| Traffic-Profile | 3GPP TS 29.336 [54] |  | Must not set |
| PDN-Connectivity-Status-Configuration | 3GPP TS 29.336 [54] | See clause 7.3.195 | Must not set |
| PDN-Connectivity-Status-Report | 3GPP TS 29.336 [54] | See clause 7.3.196 | Must not set |
| Battery-Indicator | 3GPP TS 29.336 [54] |  | Battery-Indicator |
| SCEF-Reference-ID-Ext | 3GPP TS 29.336 [54] |  | Must not set |
| SCEF-Reference-ID-for-Deletion-Ext | 3GPP TS 29.336 [54] |  | Must not set |
| NOTE 1: The M-bit settings for re-used AVPs override those of the defining specifications that are referenced. Values include: "Must set", "Must not set". If the M-bit setting is blank, then the defining specification applies. NOTE 2: If the M-bit is set for an AVP and the receiver does not understand the AVP, it shall return a rejection. If the M-bit is not set for an AVP, the receiver shall not return a rejection, whether or not it understands the AVP. If the receiver understands the AVP but the M-bit value does not match with the definition in this table, the receiver shall ignore the M-bit. |  |  |  |

* * * Next Change * * * *

### 7.3.196	Monitoring-Event-Report

The Monitoring-Event-Report AVP is of type Grouped. It shall contain the Monitoring event report data. It is originally defined in 3GPP TS 29.336 [54].

For S6a/S6d interface, the Monitoring-Event-Report AVP format is specified as following:

AVP format:

Monitoring Event Report::= <AVP header: 3123 10415>

{ SCEF-Reference-ID }

[ SCEF-Reference-ID-Ext ]

[ SCEF-ID ]

[ Reachability-Information ]

[ Reachability-Cause ]

[ EPS-Location-Information ]

[ Monitoring-Type ]

[ Loss-Of-Connectivity-Reason ]

[ Idle-Status-Indication ]

[ Maximum-UE-Availability-Time ]

*[ PDN-Connectivity-Status-Report ]

*[ AVP ]

For S6a/S6d interface, when the Monitoring-Type AVP takes the value UE_REACHABILITY (1), the Reachability-Information AVP shall take the value REACHABLE_FOR_DATA (1), and the Reachability-Cause AVP may be present.

For S6a/S6d interface, when the Monitoring-Type AVP takes the value PDN_CONNECTIVITY_STATUS (10), the PDN-Connectivity-Status-Report AVP(s) shall contain the list of active PDNs, for the given APN provided in the monitoring event configuration, or for all APNs if no APN was provided; each PDN-Connectivity-Status-Report shall have the PDN-Connectivity-Status-Type set to value "CREATED (0)".

When the "Extended Reference IDs" feature is supported by the HSS and MME/SGSN, the SCEF-Reference-ID-Ext AVP shall be used insted of SCEF-Reference-ID; in such case, the required AVP "SCEF-Reference-ID" shall be included in the grouped AVP by the sender, but its content shall be discarded by the receiver.

* * * End of Change * * * *