GPP TSG-CT Meeting #100	CP-231200

# Taipei, 12th – 13th June 2023										Revision of C4-232521, C4-232260, C4-231431

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.505 | CR | 0472 | rev | 3 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on Multicast MBS group membership management parameters |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, ZTE, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei, ZTE, Ericsson |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As defined in TS 23.247, the Multicast MBS group membership management parameters is defined as below: Table 7.2.9-2: Multicast MBS group membership management parameters Parameters; Description. List of GPSI; List of multicast group members, each member is identified by GPSI.. External Group ID; Identifier for multicast MBS group.. And as defined in TS 23.502 the Multicast MBS group membership management parameters shall included in PP data types as below: Table 5.2.3.6.1-1: Parameter Provision data types Parameter Provision data type; Description. Expected UE Behaviour parameters; See clause 4.15.6.3. Network Configuration parameters; See clause 4.15.6.3a. 5G VN group configuration data; 5G VN Group Data and 5G VN Group membership management parameters for the 5G VN Group. See clause 4.15.6.3b and clause 4.15.6.3c.. Location Privacy Indication parameters; Location Privacy Indication parameters of the "LCS privacy" Data Subset of the Subscription Data. See clause 5.2.3.3.1 and clause 7.1 of TS 23.273 [51].. Enhanced Coverage Restriction Information; See clause 4.27.1 and clause 5.31.12 of TS 23.501 [2].. ECS Address Configuration Information; See clause 4.15.6.3d.. Multicast MBS group membership management parameters; See clause 7.2.9 of TS 23.247 [78].. MBS Session Authorization information; See clause 7.2.9 of TS 23.247 [78].. MBS Session Assistance Information; See clause 7.2.9a of TS 23.247 [78].. DNN and S-NSSAI specific Group Parameters; See clause 4.15.6.3.e.. It’s proposed to add Multicast MBS group membership management parameters reated resource to Nudr_DR. |
| Summary of change: |  | It’s proposed to add Multicast MBS group membership management parameters reated resource to Nudr_DR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with stage2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1, 5.2.xx(new), 5.2.yy(new), 5.2.zz(new), 5.2.aa(new), 5.4.1, 5.4.2.a1(new), 5.4.2.a2(new), A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This contribution introduces backward compatible new features to the OpenAPI file of Nudr_DR API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

5.2.1	Overview

# Figure 5.2.1-1: Resource URI sub-level structure for subscription data

# Figure 5.2.1-2: Resource URI sub-level structure for subscription data (cont.)

# Figure 5.2.1-3: Resource URI sub-level structure for subscription data (cont.)

Table 5.2.1-1 provides an overview of the resources, applicable HTTP methods and whether subscribe (implicit and explicit) to be notified about data change applies.

# Table 5.2.1-1: Resources and methods overview

| Resource name | Resource URI | Subscribe to be notified about data change supported | HTTP method | Description |
| --- | --- | --- | --- | --- |
| AuthenticationSubscription (Document) | /subscription-data/{ueId}/authentication-data/authentication-subscription | N | GET | Retrieve a UE's authentication subscription data |
|  |  |  | PATCH | Update a UE's authentication subscription data Updates shall be limited to the sequenceNumber attribute. Attempts to patch any other attribute shall be rejected by the UDR. |
| AuthenticationSoR (Document) | /subscription-data/{ueId}/ue-update-confirmation-data/sor-data | Y | PUT | Store UE's SOR acknowledgement information (SoR-XMAC-IUE) and "ME support of SOR-CMCI" |
|  |  |  | GET | Retrieve UE's SoR acknowledgement information, "ME support of SOR-CMCI" |
|  |  |  | PATCH | Update "ME support of SOR-CMCI" |
| AuthenticationUPU (Document) | /subscription-data/{ueId}/ue-update-confirmation-data/upu-data | Y | PUT | Store UE Parameter Update acknowledgement information (UPU-XMAC-IUE) |
|  |  |  | GET | Retrieve UE Parameter Update acknowledgement information |
| SubscribedSNSSAIs (Document) | /subscription-data/{ueId}/ue-update-confirmation-data/subscribed-snssais | Y | PUT | Store UE-acknowledgement info for change of subscribed S-NSSAIs |
|  |  |  | GET | Retrieve UE-acknowledgement info for change of subscribed S-NSSAIs |
| SubscribedCAG (Document) | /subscription-data/{ueId}/ue-update-confirmation-data/subscribed-cag | Y | PUT | Store UE-acknowledgement info for change of subscribed CAG |
|  |  |  | GET | Retrieve UE-acknowledgement info for change of subscribed CAG |
| AuthenticationStatus (Document) | /subscription-data/{ueId}/authentication-data/authentication-status | N | PUT | Store a UE's authentication status |
|  |  |  | GET | Retrieve a UE's authentication status |
|  |  |  | DELETE | Delete a UE's authentication status |
| IndividualAuthenticationStatus (Document) | /subscription-data/{ueId}/authentication-data/authentication-status/{servingNetworkName} | Y | PUT | When the feature "PerUePerSnAuthStatus" is supported, store a UE's Individual authentication status |
|  |  |  | GET | When the feature "PerUePerSnAuthStatus" is supported, retrieve a UE's Individual authentication status |
|  |  |  | DELETE | When the feature "PerUePerSnAuthStatus" is supported, delete a UE's Individual authentication status |
| ProvisionedData (Document) | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data | N | GET | Retrieve the UE's subscribed Provisioned Data |
| AccessAndMobilitySubscriptionData (Document) | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data/am-data | Y | GET | Retrieve the UE's subscribed Access and Mobility Data |
| SmfSelectionSubscriptionData (Document) | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data/smf-selection-subscription-data | Y | GET | Retrieve the UE's subscribed SMF Selection Data |
| SessionManagementSubscriptionData (Document) | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data/sm-data | Y | GET | Retrieve the UE's subscribed SM Subscription Data |
| ContextData (Document) | /subscription-data/{ueId}/context-data | N | GET | Retrieve the UE's context Data |
| Amf3GppAccessRegistration (Document) | /subscription-data/{ueId}/context-data/amf-3gpp-access | Y | PUT | Create and Update the AMF registration for 3GPP access |
|  |  |  | PATCH | Modify the AMF registration for 3GPP access |
|  |  |  | GET | Retrieve the AMF registration information for 3GPP access |
| AmfNon3GppAccessRegistration (Document) | /subscription-data/{ueId}/context-data/amf-non-3gpp-access | Y | PUT | Update the AMF registration for non 3GPP access |
|  |  |  | PATCH | Modify the AMF registration for non 3GPP access |
|  |  |  | GET | Retrieve the AMF registration information for non 3GPP access |
| SmfRegistrations (Store) | /subscription-data/{ueId}/context-data/smf-registrations | Y | GET | Retrieve the list of the SMF registrations |
| IndividualSmfRegistration (Document) | /subscription-data/{ueId}/context-data /smf-registrations/{pduSessionId} | Y | PUT | Store an individual SMF registration identified by PDU Session Id |
|  |  |  | DELETE | Delete an individual SMF registration |
|  |  |  | GET | Retrieve individual SMF registration |
|  |  |  | PATCH | Modify the individual SMF registration |
| OperatorSpecificData (Document) | /subscription-data/{ueId}/operator-specific-data | Y | GET | retrieve the operator specific subscription data of a UE |
|  |  |  | PATCH | modify the operator specific subscription data of a UE |
|  |  |  | PUT | create/update the operator specific subscription data of a UE |
|  |  |  | DELETE | delete the operator specific subscription data of a UE |
| OperatorDeterminedBarringData (Document) | /subscription-data/{ueId}/operator-determined-barring-data | Y | GET | Retrieve the UE's Operator Determined Barring |
| SMSManagementSubscriptionData (Document) | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data/sms-mng-data | Y | GET | Retrieve the UE's subscribed SMS management subscription data. |
| Smsf3GppAccessRegistration (Document) | /subscription-data/{ueId}/context-data /smsf-3gpp-access | Y | PUT | Create or Update the SMSF registration |
|  |  |  | DELETE | Delete the SMSF registration for 3GPP access |
|  |  |  | GET | Retrieve the SMSF registration information |
| SmsfNon3GppAccessRegistration (Document) | /subscription-data/{ueId}/context-data /smsf-non-3gpp-access | Y | PUT | Create or Update the SMSF registration for non 3GPP access |
|  |  |  | DELETE | Delete the SMSF registration for non 3GPP access |
|  |  |  | GET | Retrieve the SMSF registration information for non 3GPP access |
| IpSmGwRegistration (Document) | /subscription-data/{ueId}/context-data/ip-sm-gw | Y | PUT | Create or Update the IP-SM-GW registration |
|  |  |  | DELETE | Delete the IP-SM-GW registration |
|  |  |  | PATCH | Modify the IP-SM-GW registration |
|  |  |  | GET | Retrieve the IP-SM-GW registration information |
| MessageWaitingData (Document) | /subscription-data/{ueId}/context-data/mwd | Y | PUT | Create or Update the SMS Message Waiting Data |
|  |  |  | DELETE | Delete the SMS Message Waiting Data |
|  |  |  | PATCH | Modify the SMS Message Waiting Data |
|  |  |  | GET | Retrieve the SMS Message Waiting Data |
| SdmSubscriptions (Collection) | /subscription-data/{ueId}/context-data/sdm-subscriptions | N | GET | Retrieve SDM subscriptions |
|  |  |  | POST | Create an individual SDM subscription |
| IndividualSdmSubscription (Document) | /subscription-data/{ueId}/context-data/sdm-subscriptions/{subsId} | N | PUT | Update an individual SDM subscription |
|  |  |  | DELETE | Delete an individual SDM subscription |
|  |  |  | PATCH | Update an individual SDM Subscription |
|  |  |  | GET | Retrieve an individual SDM subscription |
| HssSdmSubscriptionInfo (Document) | /subscription-data/{ueId}/context-data/sdm-subscriptions/{subsId}/hss-sdm-subscriptions | N | PUT | Store information related to the Hss-SDM-Subscriptions |
|  |  |  | DELETE | Delete the Hss-SDM-subscriptions |
|  |  |  | GET | Retrieve Hss-SDM-subscriptions |
|  |  |  | PATCH | Update Hss-SDM-subscriptions |
| EeSubscriptions (Collection) | /subscription-data/{ueId}/context-data/ee-subscriptions | N | GET | Retrieve EE subscriptions |
|  |  |  | POST | Create an EE subscription |
| IndividualEeSubscription (Document) | /subscription-data/{ueId}/context-data/ee-subscriptions/{subsId} | N | PUT | Update an individual EE subscription |
|  |  |  | DELETE | Delete an individual EE subscription |
|  |  |  | PATCH | Update an individual EE subscription |
|  |  |  | GET | Retrieve an individual EE subscription |
| AmfSubscriptionInfo (Document) | /subscription-data/{ueId}/context-data/ee-subscriptions/{subsId}/amf-subscriptions | N | PUT | Store information related to the Amf-EE-Subscription response |
|  |  |  | DELETE | Delete the Amf-EE-subscriptions |
|  |  |  | GET | Retrieve AMF-subscriptions |
|  |  |  | PATCH | Update AMF-subscriptions |
| SmfSubscriptionInfo (Document) | /subscription-data/{ueId}/context-data/ee-subscriptions/{subsId}/smf-subscriptions | N | PUT | Store information related to the received Smf-EE-Subscription response |
|  |  |  | DELETE | Delete the Smf-EE-subscriptions |
|  |  |  | GET | Retrieve SMF-subscriptions |
|  |  |  | PATCH | Update SMF-subscriptions |
| HssSubscriptionInfo (Document) | /subscription-data/{ueId}/context-data/ee-subscriptions/{subsId}/hss-subscriptions | N | PUT | Store information related to the Hss-EE-Subscriptions |
|  |  |  | DELETE | Delete the Hss-EE-subscriptions |
|  |  |  | GET | Retrieve Hss-EE-subscriptions |
|  |  |  | PATCH | Update Hss-EE-subscriptions |
| EeProfileData (Document) | /subscription-data/{ueId}/ee-profile-data | Y | GET | Retrieve the UE's subscribed EE profile data. |
| ProvisionedParamenterData (Document) | /subscription-data/{ueId}/pp-data | y | PATCH | Update of provisioned parameters. |
|  |  |  | GET | Retrieves the UE's provisioned parameters. |
| PpProfileData (Document) | /subscription-data/{ueId}/pp-profile-data | Y | GET | Retrieve the UE's subscribed PP profile data. |
| ProvisionedParameterDataEntries (Store) | /subscription-data/{ueId}/pp-data-store | Y | GET | Retrieve a Provisioned Parameter Data of multiple Entries |
| ProvisionedParameterDataEntry (Document) | /subscription-data/{ueId}/pp-data-store/{afInstanceId} | Y | PUT | Create a Provisioned Parameter Data Entry |
|  |  |  | DELETE | Delete a Provisioned Parameter Data Entry |
|  |  |  | GET | Retrieve a Provisioned Parameter Data Entry |
| SMSSubscriptionData (Document) | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data/sms-data | Y | GET | Retrieve the UE's subscribed SMS subscription data. |
| SubscriptionDataSubscriptions (Collection) | /subscription-data/subs-to-notify | N | GET | Retrieve existing subscriptions |
|  |  |  | POST | Create a subscription, i.e. subscribe a node to receive notification for change of data |
|  |  |  | DELETE | Deletes multiple subscriptions for a given UE. |
| IndividualSubscriptionDataSubscription (Document) | /subscription-data/subs-to-notify/{subsId} | N | DELETE | Delete the subscription identified by {subsId}, i.e. unsubscribe a node to receive notification for change of data |
|  |  |  | PATCH | Update an individual Subscription to notification |
|  |  |  | GET | Retrieve an individual Subscription to notification |
| EeGroupSubscriptions (Collection) | /subscription-data/group-data/{ueGroupId}/ee-subscriptions | N | GET | Retrieve EE subscriptions for groups of UEs |
|  |  |  | POST | Create an EE subscription for groups of UEs |
| IndividualEeGroupSubscription (Document) | /subscription-data/group-data/{ueGroupId}/ee-subscriptions/{subsId} | N | PUT | Update an individual EE subscription for a group of UEs |
|  |  |  | DELETE | Delete an individual EE subscription for a group of UEs |
|  |  |  | PATCH | Update an individual EE subscription for a group of UEs |
|  |  |  | GET | Retrieve an individual EE subscription for a group of UEs |
| AmfGroupSubscriptionInfo (Document) | /subscription-data/group-data/{ueGroupId}/ee-subscriptions/{subsId}/amf-subscriptions | N | PUT | Store information related to the Amf-EE-Subscription response for a group of UEs |
|  |  |  | DELETE | Delete the Amf-EE-subscriptions for a group of UEs |
|  |  |  | GET | Retrieve AMF-subscriptions for a group of UEs |
|  |  |  | PATCH | Update AMF-subscriptions for a group of UEs |
| SmfGroupSubscriptionInfo (Document) | /subscription-data/group-data/{ueGroupId}/ee-subscriptions/{subsId}/smf-subscriptions | N | PUT | Store information related to the received Smf-EE-Subscription response for a group of UEs |
|  |  |  | DELETE | Delete the Smf-EE-subscriptions for a group of UEs |
|  |  |  | GET | Retrieve SMF-subscriptions for a group of UEs |
|  |  |  | PATCH | Update SMF-subscriptions for a group of UEs |
| HssGroupSubscriptionInfo (Document) | /subscription-data/group-data/{ueGroupId}/ee-subscriptions/{subsId}/hss-subscriptions | N | PUT | Store information related to the Hss-EE-Subscriptions for a group of UEs |
|  |  |  | DELETE | Delete the Hss-EE-subscriptions for a group of UEs |
|  |  |  | GET | Retrieve Hss-EE-subscriptions for a group of UEs |
|  |  |  | PATCH | Update Hss-EE-subscriptions for a group of UEs |
| EeGroupProfileData (Document) | /subscription-data/group-data/{ueGroupId}/ee-profile-data | Y | GET | Retrieve the subscribed EE profile data for a group of UEs. |
| TraceData (Document) | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data/trace-data | Y | GET | Retrieve the UE's trace configuration data |
| IdentityData (Document) | /subscription-data/{ueId}/identity-data | Y | GET | Retrieve identity data that corresponds to the provided ueId |
| SharedData (Collection) | /subscription-data/shared-data | N | GET | Retrieve shared data |
| IndividualSharedData (Document) | /subscription-data/shared-data/{sharedDataId} | Y | GET | Retrieve the individual Shared Data |
| GroupIdentifiers (Document) | /subscription-data/group-data/group-identifiers | Y | GET | Retrieve group identifiers and the UE identifiers belong to the group identifiers. |
| 5GvnGroups (Store) | /subscription-data/group-data/5g-vn-groups | Y | GET | Retrieve 5G VN Groups |
| Individual5GvnGroup (Document) | /subscription-data/group-data/5g-vn-groups/{externalGroupId} | Y | PUT | Create a 5G VN Group |
|  |  |  | PATCH | Update a 5G VN Group |
|  |  |  | DELETE | Delete a 5G VN Group |
|  |  |  | GET | Retrieve a 5G VN Group |
| 5GVnGroupsInternal (Document) | /subscription-data/group-data/5g-vn-groups/internal | Y | GET | Retrieve 5G VN Group Data based on Internal Group Identifier(s) |
| Pp5gVnGroupProfileData (Document) | /subscription-data/group-data/5g-vn-groups/pp-profile-data | Y | GET | Retrieve the UE's subscribed PP profile data for accessing 5G VN Groups service operations. |
| LcsPrivacySubscriptionData (Document) | /subscription-data/{ueId}/lcs-privacy-data | Y | GET | Retrieve the UE's subscribed LCS privacy Subscription Data |
| LcsMobileOriginatedSubscriptionData (Document) | /subscription-data/{ueId}/lcs-mo-data | Y | GET | Retrieve the UE's subscribed LCS Mobile Originated Subscription Data |
| NiddAuthorizationData (Document) | /subscription-data/{ueId}/nidd-authorization-data | Y | GET | Retrieve the UE's NIDD Authorization Data |
| ServiceSpecificAuthorizationData (Document) | /subscription-data/{ueId}/service-specific-authorization-data/{serviceType} | Y | GET | Retrieve the UE's Authorization Data for a specific service |
| CoverageRestrictionData (Document) | /subscription-data/{ueId}/coverage-restriction-data | Y | GET | Retrieve the UE's subscribed enhanced Coverage Restriction Data |
| Location (Document) | /subscription-data/{ueId}/context-data/location | Y | GET | Retrieve the UE's Location Information |
| V2xSubscriptionData (Document) | /subscription-data/{ueId}/v2x-data | Y | GET | Retrieve the UE's subscribed V2X Data |
| ProseSubscriptionData (Document) | /subscription-data/{ueId}/prose-data | Y | GET | Retrieve the UE's subscribed ProSe Service Data |
| 5MBSSubscriptionData (Document) | /subscription-data/{ueId}/5mbs-data | Y | GET | Retrieve the UE's 5MBS Subscription Data |
| UcSubscriptionData (Document) | /subscription-data/{ueId}/uc-data | Y | GET | Retrieve the UE's User Consent Data |
| LcsBroadcastAssistanceSubscriptionData (Document) | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data/lcs-bca-data | Y | GET | Retrieve the UE's subscribed LCS Broadcast Assistance subscription data |
| NiddAuthorizations (Document) | /subscription-data/{ueId}/context-data/nidd-authorizations | Y | PUT | Store information related to the NIDD Authorization |
|  |  |  | DELETE | Delete the NIDD Authorizations |
|  |  |  | GET | Retrieve NIDD Authorizations |
|  |  |  | PATCH | Update a specific NIDD Authorization |
| UeSubscriptionData (Document) | /subscription-data/{ueId} | N | GET | Retrieve data set(s) from the UE's subscription data |
| SpecificServiceAuthorizations (Document) | /subscription-data/{ueId}/context-data/service-specific-authorizations/{serviceType} | Y | PUT | Store information related to the service specific Authorization |
|  |  |  | DELETE | Delete the service specific Authorization |
|  |  |  | GET | Retrieve service specific Authorizations |
|  |  |  | PATCH | Update a service specific Authorization |
| RoamingInfo (Document) | /subscription-data/{ueId}/context-data/roaming-information | Y | GET | Retrieve the last 5GC/EPC common Roaming Information in the 3GPP access |
|  |  |  | PUT | Update or create the last 5GC/EPC common Roaming Information in the 3GPP access |
| PeiInfo (Document) | /subscription-data/{ueId}/context-data/ pei-info | Y | GET | Retrieve the PEI of the 5GC/EPC domains |
|  |  |  | PUT | Update or create the PEI of the 5GC/EPC domains |
| TimeSyncSubscriptionData (Document) | /subscription-data/{ueId}/time-sync-data | N | GET | Retrieve the UE's Time Synchronization Subscription Data |
| 5GMbsGroup (Store) | /subscription-data/group-data/mbs-group-membership | Y | GET | Retrieve 5G MBS Group |
| Individual5GmbsGroup (Document) | /subscription-data/group-data/mbs-group-membership/{externalGroupId} | Y | PUT | Create a 5G MBS Group |
|  |  |  | PATCH | Update a 5G MBS Group |
|  |  |  | DELETE | Delete a 5G MBS Group |
|  |  |  | GET | Retrieve a 5G MBS Group |
| 5GMbsGroupsInternal (Document) | /subscription-data/group-data/mbs-group-membership/internal | Y | GET | Retrieve 5G MBS Group Data based on Internal Group Identifier(s) |
| Pp5gMbsGroupProfileData (Document) | /subscription-data/group-data/mbs-group-membership/pp-profile-data | Y | GET | Retrieve the UE's subscribed PP profile data for accessing 5G MBS Group service operations. |

* * * Next Change * * * *

5.2.xx	Resource: 5GMbsGroup

5.2.xx.1	Description

This resource represents the store of 5G MBS Group data.

5.2.xx.2	Resource Definition

Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/group-data/mbs-group-membership

This resource shall support the resource URI variables defined in table 5.2.xx.2-1.

# Table 5.2.xx.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |

5.2.xx.3	Resource Standard Methods

5.2.xx.3.1	GET

This method shall support the URI query parameters specified in table 5.2.xx.3.1-1.

# Table 5.2.xx.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| gpsis | array(Gpsi) | O | 0..N | Only those group's data are retrieved of which at least one of the GPSIs is a member |

This method shall support the request data structures specified in table 5.2.xx.3.1-2 and the response data structures and response codes specified in table 5.2.xx.3.1-3.

# Table 5.2.xx.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.xx.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| map(MulticastMbsGroupMemb) | M | 0..N | 200 OK | Upon success, a response body containing the map (list of key-value pairs where ExtGroupId (see 3GPP TS 29.503 [6] serves as key) of MulticastMbsGroupMemb shall be returned. If the resource does not contain any keys, the response consists of an empty JSON object (empty map). |
| NOTE: In addition, common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

5.2.yy	Resource: Individual5GmbsGroup

5.2.yy.1	Description

This resource represents an individual 5G MBS Group.

This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [7]).

5.2.yy.2	Resource Definition

Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/group-data/mbs-group-membership/{externalGroupId}

This resource shall support the resource URI variables defined in table 5.2.yy.2-1.

# Table 5.2.yy.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |
| externalGroupId | Represents the External Identifier of the group |

5.2.yy.3	Resource Standard Methods

5.2.yy.3.1	PUT

This method shall support the URI query parameters specified in table 5.2.yy.3.1-1.

# Table 5.2.yy.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.yy.3.1-2 and the response data structures and response codes specified in table 5.2.yy.3.1-3.

# Table 5.2.yy.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MulticastMbsGroupMemb | M | 1 | See 3GPP TS 29.503 [6] |

# Table 5.2.yy.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MulticastMbsGroupMemb | M | 1 | 201 Created | Upon success, a representation of the created resource shall be returned. |
| ProblemDetails | O | 0..1 | 403 Forbidden | If the resource already exists, a response code of 403 Forbidden shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

5.2.yy.3.2	DELETE

This method shall support the URI query parameters specified in table 5.2.yy.3.2-1.

# Table 5.2.yy.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.yy.3.2-2 and the response data structures and response codes specified in table 5.2.19.3.2-3.

# Table 5.2.yy.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.yy.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

5.2.yy.3.3	PATCH

This method shall support the URI query parameters specified in table 5.2.yy.3.3-1.

# Table 5.2.yy.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [8] clause 6.6 |

This method shall support the request data structures specified in table 5.2.yy.3.3-2 and the response data structures and response codes specified in table 5.2.yy.3.3-3.

# Table 5.2.yy.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(PatchItem) | M | 1..N | Contains the delta data to the Individual 5GNvGroup |

# Table 5.2.yy.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon successful modification, there is no body in the response message. (NOTE 2) |
| PatchResult | M | 1 | 200 OK | Upon success, the execution report is returned. (NOTE 2) |
| ProblemDetails | O | 0..1 | 404 Not Found | If the resource corresponding to the ExternalGroupId can't be found, the "cause" attribute shall be set to: - GROUP_IDENTIFIER_NOT_FOUND |
|  | O | 0..1 | 403 Forbidden | If one or more attributes are not allowed to be modified according to e.g. policy or local configuration, then the invalidParams attribute shall contain the JSON pointers of attributes which are not allowed to be modified, and the cause attribute shall be set to "MODIFICATION_NOT_ALLOWED", see 3GPP TS 29.500 [8] table 5.2.7.2-1 |
| NOTE 1: In addition common data structures as listed in table 5.5-1 are supported. NOTE 2: If all the modification instructions in the PATCH request have been implemented, the UDR shall respond with 204 No Content response; if some of the modification instructions in the PATCH request have been discarded, and the NF service consumer has included in the supported-feature query parameter the "PatchReport" feature number, the UDR shall respond with PatchResult. |  |  |  |  |

5.2.yy.3.4	GET

This method shall support the URI query parameters specified in table 5.2.yy.3.4-1.

# Table 5.2.yy.3.4-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [8] clause 6.6 |

This method shall support the request data structures specified in table 5.2.yy.3.4-2 and the response data structures and response codes specified in table 5.2.yy.3.4-3.

# Table 5.2.yy.3.4-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.yy.3.4-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MulticastMbsGroupMemb | M | 1 | 200 OK | Upon success, a response body containing the group configuration of the individual 5G MBS Group shall be returned. |
| NOTE: In addition, common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

5.2.zz	Resource: 5GMbsGroupsInternal

5.2.zz.1	Description

This resource represents a read-only view of the collection resource of 5G MBS Group data, so it can be queried based on the Internal Group Id from NF Service Consumers having such Id as the ony way to identify a group (e.g. PCF).

5.2.zz.2	Resource Definition

Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/group-data/mbs-group-membership/internal

This resource shall support the resource URI variables defined in table 5.2.zz.2-1.

# Table 5.2.zz.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |

5.2.zz.3	Resource Standard Methods

5.2.zz.3.1	GET

This method shall support the URI query parameters specified in table 5.2.zz.3.1-1.

# Table 5.2.zz.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| internal-group-ids | array(GroupId) | M | 1..N | Return those 5G MBS groups whose Internal Group ID corresponds with any of the Group IDs in this list. |

This method shall support the request data structures specified in table 5.2.zz.3.1-2 and the response data structures and response codes specified in table 5.2.zz.3.1-3.

# Table 5.2.zz.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.zz.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| map(MulticastMbsGroupMemb) | M | 0..N | 200 OK | Upon success, a response body containing the map (list of key-value pairs where ExtGroupId (see 3GPP TS 29.503 [6] serves as key) of MulticastMbsGroupMemb shall be returned. If the resource does not contain any keys, the response consists of an empty JSON object (empty map). |

5.2.aa	Resource: Pp5gMbsGroupProfileData

5.2.aa.1	Description

This resource represents the Parameter Provision Profile Data for the 5G MBS Groups. It is queried by the UDM.

5.2.aa.2	Resource Definition

Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/group-data/mbs-group-membership/pp-profile-data

This resource shall support the resource URI variables defined in table 5.2.aa.2-1.

# Table 5.2.aa.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.1.1 |

5.2.aa.3	Resource Standard Methods

5.2.aa.3.1	GET

This method shall support the URI query parameters specified in table 5.2.aa.3.1-1.

# Table 5.2.aa.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| ext-group-ids | array(ExtGroupId) | O | 1..N | If included, this IE Contain a list of external MBS group identifiers for which the profiles are retrieved. |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [8] clause 6.6 |

This method shall support the request data structures specified in table 5.2.aa.3.1-2 and the response data structures and response codes specified in table 5.2.aa.3.1-3.

# Table 5.2.aa.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.aa.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| Pp5gMbsGroupProfileData | M | 1 | 200 OK | Upon success, a response body containing the Parameter Provision Profile for 5G MBS Groups. |
| NOTE: In addition, common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

* * * Next Change * * * *

5.4.1	General

This clause specifies the application data model supported by the API.

Table 5.4.1-1 specifies the data types defined for the Nudr service based interface protocol.

# Table 5.4.1-1: Nudr Subscriber Data specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| AuthenticationSubscription | 5.4.2.2 | A UE's authentication data |
| OperatorSpecificDataContainer | 5.4.2.3 | Container for operator specific data |
| SmfRegList | 5.4.2.4 | The list of all the SMF registrations of a UE |
| SubscriptionDataSubscriptions | 5.4.2.5 | A subscription to notifications. |
| DataChangeNotify | 5.4.2.6 | Container for data which have changed and notification was requested when changed. |
| IdentityData | 5.4.2.7 | Identity data corresponds to the provided ueId |
| ProvisionedDataSets | 5.4.2.8 | Contains the provisioned data sets |
| SorData | 5.4.2.9 | Used to store the status of the latest SOR data update |
| UpuData | 5.4.2.9A | Used to store the status of the latest UPU data update |
| NssaiAckData | 5.4.2.9B | Used to store the status of the latest NSSAI data update |
| CagAckData | 5.4.2.9C | Used to store the status of the latest CAG data update |
| AmfSubscriptionInfo | 5.4.2.19 | Information the UDR stores and retrieves related to active subscriptions at the AMF(s) |
| EeProfileData | 5.4.2.20 | Event Exposure Profile Data |
| ContextDataSets | 5.4.2.22 | Contains the context data sets |
| SequenceNumber | 5.4.2.23 | Contains the SQN |
| MessageWaitingData | 5.4.2.24 | Message Waiting Data list |
| SmscData | 5.4.2.25 | Addresses of SM-Service Center entities with SMS waiting to be delivered to the UE |
| SmfSubscriptionInfo | 5.4.2.26 | Information the UDR stores and retrieves related to active subscriptions at the SMF(s) |
| SmfSubscriptionItem | 5.4.2.27 | Contains info about a single SMF event subscription |
| MtcProvider | 5.4.2.28 | MTC provider information |
| HssSubscriptionInfo | 5.4.2.29 | Information the UDR stores related to active subscriptions at the HSS(s) |
| HssSubscriptionItem | 5.4.2.30 | Contains info about a single HSS event subscription |
| EeGroupProfileData | 5.4.2.31 | The Event Exposure Profile Data for a group of UEs |
| Pp5gVnGroupProfileData | 5.4.2.32 | The Parameter Provision Profile Data for 5G VN groups |
| PpProfileData | 5.4.2.33 | The Parameter Provision Profile Data |
| AllowedMtcProviderInfo | 5.4.2.34 | Allowed MTC Providers or AFs Information to provision parameters for UE. |
| GroupIdentifiers | 5.4.2.35 | External or Internal Group Identifier with a list of group members |
| AuthorizationData | 5.4.2.36 | Authorization Information |
| NiddAuthorizationInfo | 5.4.2.37 | Information the UDR stores and retrieves related to active NIDD authorizations |
| PpDataEntryList | 5.4.2.38 | A list of the Provisioned Parameters entries. |
| UeSubscribedDataSets | 5.4.2.39 | Contains the subscribed data sets of a UE |
| ServiceSpecificAuthorizationInfo | 5.4.2.40 | Information the UDR stores and retrieves related to active Service Specific authorizations |
| NfIdentifier | 5.4.2.41 | Network Function Identifier |
| EeSubscriptionExt | 5.4.2.42 | Extended Event Exposure Subscription |
| AdditionalEeSubsInfo | 5.4.2.43 | Additional EE-Subscription Information |
| ImmediateReport | 5.4.2.44 | Immediate Report |
| Pp5gMbsGroupProfileData | 5.4.2.a1 | The Parameter Provision Profile Data for 5G MBS groups |
| AllowedMbsInfo | 5.4.2.a2 | Allowed AFs Information to provision parameters for UE. |
| VarPlmnId | 5.4.3.2 | String identifying a PLMN ID or an SNPN ID. |
| AuthMethod | 5.4.3.3 | Contains the Authentication Method |
| ProvisionedDataSetName | 5.4.3.4 | The name of data set |
| ContextDataSetName | 5.4.3.6 | The name of context data set |
| SqnScheme | 5.4.3.7 | Scheme for generation of Sequence Numbers |
| Sign | 5.4.3.8 | Sign of the DIF value |
| UeUpdateStatus | 5.4.3.9 | Status of the procedure |
| PpDataType | 5.4.3.10 | The data type of the Provisioned Parameters |
| UeSubscribedDataSetName | 5.4.3.11 | The name of subscribed data set |

Table 5.4.1-2 specifies data types re-used by the Nudr service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nudr service based interface.

# Table 5.4.1-2: Nudr re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| AccessAndMobilitySubscriptionData | 3GPP TS 29.503 [6] | Access and Mobility Subscription Data |
| SmfSelectionSubscriptionData | 3GPP TS 29.503 [6] | SMF Selection Subscription Data |
| SmSubsData | 3GPP TS 29.503 [6] | Session Management Subscription Data |
| Amf3GppAccessRegistration | 3GPP TS 29.503 [6] | The complete set of information relevant to the AMF where the UE has registered via 3GPP access. |
| AmfNon3GppAccessRegistration | 3GPP TS 29.503 [6] | The complete set of information relevant to the AMF where the UE has registered via non 3GPP access. |
| SmfRegistration | 3GPP TS 29.503 [6] | The complete set of information relevant to an SMF serving the UE |
| SmsfRegistration | 3GPP TS 29.503 [6] | The complete set of information relevant to the SMSF serving the UE. |
| SmsManagementSubscriptionData | 3GPP TS 29.503 [6] | SMS Management Subscription Data |
| SupportedFeatures | 3GPP TS 29.571 [3] | Used to negotiate the applicability of the optional features |
| ProblemDetails | 3GPP TS 29.571 [3] | Detailed information about the status code. |
| PpData | 3GPP TS 29.503 [6] | Parameter Provision Data |
| PpDataEntry | 3GPP TS 29.503 [6] | Provisioned Parameter Data Entry |
| SmsSubscriptionData | 3GPP TS 29.503 [6] | SMS Subscription Data |
| PatchItem | 3GPP TS 29.571 [3] | Data structure used for JSON patch. |
| SdmSubscription | 3GPP TS 29.503 [6] | A subscription to notifications |
| EeSubscription | 3GPP TS 29.503 [6] | A subscription to Notifications of event exposure |
| Uri | 3GPP TS 29.571 [3] | URI |
| TraceData | 3GPP TS 29.571 [3] | Trace Data |
| Dnn | 3GPP TS 29.571 [3] | Data Network Name |
| Snssai | 3GPP TS 29.571 [3] | Single NSSAI |
| VarUeId | 3GPP TS 29.571 [3] | String represents the SUPI or GPSI. |
| NfInstanceId | 3GPP TS 29.571 [3] | NF Instance Identifier |
| NotifyItem | 3GPP TS 29.571 [3] | Common data type used for data change notification. |
| OdbData | 3GPP TS 29.571 [3] | Operator Determined Barring Data |
| EventType | 3GPP TS 29.503 [6] | Event Type |
| ExtGroupId | 3GPP TS 29.503 [6] | External Group Identifier |
| PduSessionId | 3GPP TS 29.571 [3] | PDU Session ID which identifies a PDU session |
| DateTime | 3GPP TS 29.571 [3] | Date Time |
| UpuMac | 3GPP TS 29.509 [15] | MAC value for protecting UPU procedure (UPU-MAC-IAUSF and UPU-MAC-IUE) |
| 5GVnGroupConfiguration | 3GPP TS 29.503 [6] | 5G VN Group Configuration data |
| PatchResult | 3GPP TS 29.571 [3] | Patch Result |
| SupportedFeatures | 3GPP TS 29.571 [3] | Supported Features |
| AppPortId | 3GPP TS 29.503 [6] | Application Port Identifier |
| LcsPrivacyData | 3GPP TS 29.503 [6] | LCS Privacy Subscription Data |
| LcsMoData | 3GPP TS 29.503 [6] | LCS Mobile Originated Subscription Data |
| MtcProviderInformation | 3GPP TS 29.571 [3] | An ID uniquely identifying MTC provider information |
| EnhancedCoverageRestrictionData | 3GPP TS 29.503 [6] | Enhanced Coverage Restriction Data |
| LocationInfo | 3GPP TS 29.503 [6] | Location Information |
| V2xSubscriptionData | 3GPP TS 29.503 [6] | V2X Subscription Data |
| E164Number | 3GPP TS 29.503 [6] | E164Number |
| NetworkNodeDiameterAddress | 3GPP TS 29.503 [6] | Diameter Address of an Network Node |
| IpSmGwRegistration | 3GPP TS 29.503 [6] | The complete set of information relevant to the IP-SM-GW where the UE has registered. |
| LcsBroadcastAssistanceTypesData | 3GPP TS 29.503 [6] | LCS Broadcast Assistance Data Types |
| ContextInfo | 3GPP TS 29.503 [6] | Contains the headers received by an NF. |
| NfGroupId | 3GPP TS 29.571 [3] | NF Group ID |
| ProseSubscriptionData | 3GPP TS 29.503 [6] | ProSe Service Subscription Data |
| PlmnId | 3GPP TS 29.571 [3] | PLMN ID |
| AppPortId | 3GPP TS 29.503 [6] | Application Port Id |
| AuthorizationInfo | 3GPP TS 29.503 [6] | Contains Authorization Information |
| MbsSubscriptionData | 3GPP TS 29.503 [6] | Contains 5MBS Subscription Data |
| ServiceType | 3GPP TS 29.503 [6] | Service Type |
| RoamingInfoUpdate | 3GPP TS 29.503 [6] | Roaming Information in EPC domain |
| UcSubscriptionData | 3GPP TS 29.503 [6] | User Consent Data |
| UcPurpose | 3GPP TS 29.503 [6] | User Consent Purpose |
| PeiUpdateInfo | 3GPP TS 29.503 [6] | PEI Update Information |
| SharedData | 3GPP TS 29.503 [6] | Shared Data |
| TimeSyncSubscriptionData | 3GPP TS 29.503 [6] | Contains Time Synchronization Subscription Data |

* * * Next Change * * * *

5.4.2.a1	Type: Pp5gMbsGroupProfileData

# Table 5.4.2.a1-1: Definition of type Pp5gMbsGroupProfileData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| allowedMbsInfos | map(array(AllowedMbsInfo)) | O | 1..N | A map (list of key-value pairs where ExtGroupId (see 3GPP TS 29.503 [6]) serves as key of a list of AllowedMbsInfos which include AF IDs that are allowed to create, update and delete a 5G MBS Group for the user using UDM ParameterProvision service. In addition to defined external group identifier, the key value "ALL" may be used to identify a map entry which contains a list of AllowedMbsInfos that are allowed operating all the external group identifiers. The absence of this IE indicates that all the AFs are authorized to create, update and delete any 5G MBS Group. |
| supportedFeatures | SupportedFeatures | O | 0..1 |  |

5.4.2.a2	Type: AllowedMbsInfo

# Table 5.4.2.a2-1: Definition of type AllowedMbsInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| afId | string | C | 0..1 | Indicates ID of the AF for Parameters Provision authorization. The string identifying the originating AF, formatted as described in the definition of type MonitoringConfiguration in 3GPP TS 29.503 [6]. |

* * * Next Change * * * *

A.2	Nudr_DataRepository API for Subscription Data

For the purpose of referencing entities in the Open API file defined in this Annex, it shall be assumed that this Open API file is contained in a physical file named "TS29505_Subscription_Data.yaml".

openapi: 3.0.0

info:

version: '-'

title: 'Unified Data Repository Service API file for subscription data'

description: |

Unified Data Repository Service (subscription data).

The API version is defined in 3GPP TS 29.504.

© 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

[…]

/subscription-data/{ueId}/time-sync-data:

get:

summary: Retrieves the time synchronization subscription data of a UE

operationId: QueryTimeSyncSubscriptionData

tags:

- TimeSyncSubscriptionData (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

- name: If-None-Match

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.2

schema:

type: string

- name: If-Modified-Since

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.3

schema:

type: string

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/TimeSyncSubscriptionData'

headers:

Cache-Control:

description: Cache-Control containing max-age, as described in RFC 7234, 5.2

schema:

type: string

ETag:

description: Entity Tag, containing a strong validator, as described in RFC 7232, 2.3

schema:

type: string

Last-Modified:

description: Timestamp for last modification of the resource, as described in RFC 7232, 2.2

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/subscription-data/group-data/mbs-group-membership:

get:

summary: Retrieves the data of a 5G MBS Group

operationId: Query5GmbsGroup

tags:

- 5G-MBS-Group (Store)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: gpsis

in: query

description: List of GPSIs

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

style: form

explode: false

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

type: object

description: A map(list of key-value pairs) where ExtGroupId serves as key

additionalProperties:

$ref: 'TS29503_Nudm_PP.yaml#/components/schemas/MulticastMbsGroupMemb'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/group-data/mbs-group-membership/{externalGroupId}:

put:

summary: Create an individual 5G MBS Grouop

operationId: Create5GmbsGroup

tags:

- MulticastMbsGroupMemb (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: externalGroupId

in: path

required: true

schema:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/ExtGroupId'

requestBody:

content:

application/json:

schema:

# $ref: 'TS29503_Nudm_PP.yaml#/components/schemas/MulticastMbsGroupMemb'

required: true

responses:

'201':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: 'TS29503_Nudm_PP.yaml#/components/schemas/MulticastMbsGroupMemb'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Deletes the 5GmbsGroup

operationId: Delete5GmbsGroup

tags:

- Delete 5GmbsGroup

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: externalGroupId

in: path

required: true

schema:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/ExtGroupId'

responses:

'204':

description: Expected response to a successful group removal

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: modify the 5GmbsGroup

operationId: Modify5GmbsGroup

tags:

- Modify 5GmbsGroup

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: externalGroupId

in: path

required: true

schema:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/ExtGroupId'

- name: supported-features

in: query

description: Features required to be supported by the target NF

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

requestBody:

content:

application/json-patch+json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchItem'

required: true

responses:

'204':

description: Expected response to a valid request

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchResult'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

description: modification is rejected

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

get:

summary: Retrieve a 5GmbsGroup

operationId: GetMulticastMbsGroupMemb

tags:

- Query MulticastMbsGroupMemb (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: externalGroupId

in: path

required: true

schema:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/ExtGroupId'

responses:

'200':

description: OK

content:

application/json:

schema:

$ref: 'TS29503_Nudm_PP.yaml#/components/schemas/MulticastMbsGroupMemb'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/group-data/mbs-group-membership/internal:

get:

summary: Retrieves the data of 5G MBS Group

operationId: Query5GMbsGroupInternal

tags:

- 5G-MBS-Groups-Internal (Document)

parameters:

- name: internal-group-ids

in: query

description: List of Internal Group IDs

required: true

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GroupId'

style: form

explode: false

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

type: object

description: A map (list of key-value pairs) where ExtGroupId serves as key

additionalProperties:

$ref: 'TS29503_Nudm_PP.yaml#/components/schemas/MulticastMbsGroupMemb'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/group-data/mbs-group-membership/pp-profile-data:

get:

summary: Retrieves the parameter provision profile data for 5G MBS Group

operationId: Query5GMbsGroupPPData

tags:

- Parameter Provision profile Data for 5G MBS Group(Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ext-group-ids

in: query

description: List of external MBS group identifiers

schema:

type: array

items:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/ExtGroupId'

minItems: 1

style: form

explode: false

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/Pp5gMbsGroupProfileData'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

schemas:

AuthenticationSubscription:

description: A UE's authentication data.

type: object

required:

- authenticationMethod

properties:

authenticationMethod:

$ref: '#/components/schemas/AuthMethod'

encPermanentKey:

type: string

protectionParameterId:

type: string

sequenceNumber:

$ref: '#/components/schemas/SequenceNumber'

authenticationManagementField:

type: string

pattern: '^[A-Fa-f0-9]{4}$'

algorithmId:

type: string

encOpcKey:

type: string

encTopcKey:

type: string

vectorGenerationInHss:

type: boolean

default: false

hssGroupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfGroupId'

n5gcAuthMethod:

$ref: '#/components/schemas/AuthMethod'

rgAuthenticationInd:

type: boolean

default: false

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

akmaAllowed:

type: boolean

default: false

routingId:

type: string

pattern: '^[0-9]{1,4}$'

[…]

ImmediateReport:

oneOf:

- $ref: '#/components/schemas/ProvisionedDataSets'

- type: array

items:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/SharedData'

minItems: 0

Pp5gMbsGroupProfileData:

type: object

properties:

allowedMbsInfos:

description: A map (list of key-value pairs where external VN group identifier serves as key) of AllowedMbsInfos lists. In addition to defined external MBS group identifier, the key value "ALL" may be used to identify a map entry which contains a list of AllowedMbsInfo that are allowed operating all the external group identifiers.

type: object

additionalProperties:

type: array

items:

$ref: '#/components/schemas/AllowedMbsInfo'

minItems: 1

minProperties: 1

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

AllowedMbsInfo:

type: object

properties:

afId:

type: string

* * * End of Changes * * * *