3GPP TSG-CT Meeting #100	CP-231202

# Taipei, 12th – 13th June 2023							Revision of C4-232618, C4-232524, C4-232128

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.559 | CR | 0010 | rev | 3 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add service Npkmf_ResolveRemoteUserId |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CATT, Huawei |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe |  |  |  |  |  | Date: |  |  | 2023-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Stage 2 has defined a new service for PKMF, as described in TS 33.503 clause 7.2.3. The function of this service is resolving Remote User ID (UP-PRUK ID) to SUPI. 7.2.3 Npkmf_ResolveRemoteUserId service 7.2.3.1 Npkmf_ResolveRemoteUserId_Get service operation Service operation name: Npkmf_ResolveRemoteUserId_Get Description: The NF consumer requests the PKMF to resolve the Remote User ID. Input, Required: Remote User ID (UP-PRUK ID). Input, Optional: HPLMN ID. Output, Required: SUPI. Output, Optional: None. This service is used in the following procedure: 6.3.3.2.2 PC5 security establishment for 5G ProSe UE-to-Network relay communication over User Plane … 8a. If the mapping of the Remote User ID and the 5G ProSe Remote UE's SUPI is not available in the SMF of the 5G ProSe UE-to-Network Relay, the SMF shall send a Resolve Remote User ID request towards the PKMF of the 5G ProSe UE-to-Network Relay in Npkmf_ResolveRemoteUserId_Get Request message, including the Remote User ID of the 5G ProSe Remote UE and the HPLMN ID of the 5G ProSe Remote UE if UP-PRUK ID used as Remote User ID is not in NAI format in the message. 8b. The 5G PKMF of the 5G ProSe UE-to-Network Relay forwards the Resolve Remote User ID request in Npkmf_ResolveRemoteUserId_Get Request message towards the 5G PKMF of the 5G ProSe Remote UE. The 5G PKMF of the 5G ProSe UE-to-Network Relay identifies the 5G PKMF address of the 5G ProSe Remote UE based on the UP-PRUK ID or HPLMN ID of the 5G ProSe Remote UE. 8c. The 5G PKMF of the 5G ProSe Remote UE shall send a Resolve Remote User ID response to the 5G PKMF of the 5G ProSe UE-to-Network Relay in Npkmf_ResolveRemoteUserId_Get Response message, including the SUPI of the 5G ProSe Remote UE in the message. 8d. The 5G PKMF of the 5G ProSe UE-to-Network Relay forwards the Npkmf_ResolveRemoteUserId_Get Response message including the SUPI to the SMF of the 5G ProSe UE-to-Network Relay. This CR adapts PKMF services to implement above requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1/ Add SMF/PKMF as consumer of PKMF service 2/ Define Npkmf_ResolveRemoteUserId Service 3/ Define Npkmf_ResolveRemoteUserId API 4/ Add Annex for Npkmf_ResolveRemoteUserId OpenAPI |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 2 requirement on Remote User ID resolving in the 5G ProSe UE-to-Network relay communication over User Plane procedure cannot be implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4, 5.1, 5.x (New), 6.x (New), A.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces new Npkmf_ResolveRemoteUserId OpenAPI file |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: Add Huawei as co-source Rev 2: Correct the editorial error (use hard spaces) Rev 3 (To CT#100): Correct the errors in OpenAPI (missing file extension.yaml in the ref field) |  |  |  |  |  |  |  |  |

* * * First Change * * *

# 4	Overview

The 5G ProSe Key Management Function (5G PKMF) is the logical function handling network related actions required for the key management and the security material for discovery of a 5G ProSe UE-to-Network Relay by a 5G ProSe Remote UE; and for establishing a secure PC5 communication link between a 5G ProSe Remote UE and 5G ProSe UE-to-Network Relay (see 3GPP TS 33.503 [4]).

Figure 4-1 provides the reference model (in service based interface representation and in reference point representation), with focus on the 5G PKMF:

Figure 4-1: Reference model – 5G PKMF

Editor's note:	The reference point between PKMF and SMF will be aligned with Stage 2.

The functionalities supported by the 5G PKMF are listed in clause 4.2.1.2 of 3GPP TS 33.503 [4].

* * * Next Change * * *

## 5.1	Introduction

The table 5.1-1 shows the PKMF Services and PKMF Service Operations:

Table 5.1-1: List of 5G PKMF Services

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Npkmf_PKMFKeyRequest | ProseKey | Request/Response | PKMF |
| Npkmf_ResolveRemoteUserId | Retrieve | Request/Response | SMF, 5G PKMF |

Table 5.1-2 summarizes the corresponding APIs defined for this specification.

Table 5.1-2: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | apiName | Annex |
| --- | --- | --- | --- | --- | --- |
| Npkmf_PKMFKeyRequest | 6.1 | PKMF Key Request Service | TS29559_Npkmf_PKMFKeyRequest.yaml | npkmf-keyrequest | A.2 |
| Npkmf_ResolveRemoteUserId | 6.x | PKMF Resolve Remote User ID Service | TS29559_Npkmf_UserId.yaml | npkmf-userid | A.x |

* * * Next Change * * *

## 5.x	Npkmf_ResolveRemoteUserId Service

### 5.x.1	Service Description

The Npkmf_ResolveRemoteUserId service enables a NF to request the PKMF to resolve Remote User ID (i.e., UP-PRUK ID) to SUPI.

### 5.x.2	Service Operations

#### 5.x.2.1	Introduction

The service operation defined for the Npkmf_ResolveRemoteUserId service is as follows:

-	Retrieve: It allows a consumer NF to get an user’s SUPI from Remote User ID.

#### 5.x.2.2	Retrieve

##### 5.x.2.2.1	General

The Retrieve service operation is used during the following procedure:

-	PC5 security establishment for 5G ProSe UE-to-Network relay communication over User Plane (see 3GPP TS 33.503 [4], clause 6.3.3.2.2)

The NF Service Consumer (e.g., SMF, PKMF) shall request the PKMF to get the SUPI of a 5G ProSe Remote UE as shown in Figure 5.x.2.2.1-1

Figure 5.x.2.2.1-1: Requesting the SUPI of a ProSe Remote UE

1.	The NF service consumer (e.g., SMF, PKMF) sends a POST request to the resource representing the resolve-id custom operation. The request body shall contain the UP-PRUK ID.

2a.	On success, the PKMF responds with "200 OK" with the message body containing the corresponding SUPI.

2b.	If there is no valid data, HTTP status code "404 Not Found" shall be returned including additional error information in the response body (in the "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the POST response body.

* * * Next Change * * *

## 6.x	Npkmf_ResolveRemoteUserId Service API

### 6.x.1	Introduction

The Npkmf_ResolveRemoteUserId service shall use the Npkmf_ResolveRemoteUserId API.

The API URI of the Npkmf_ResolveRemoteUserId API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests from the NF service consumer towards the NF service producer shall have the Resource URI structure defined in clause 4.4.1 of 3GPP TS 29.501 [6], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [6].

-	The <apiName> shall be "npkmf-userid".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 6.x.3.

### 6.x.2	Usage of HTTP

#### 6.x.2.1	General

HTTP/2, IETF RFC 7540 [8], shall be used as specified in clause 5 of 3GPP TS 29.500 [5].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [5].

The OpenAPI [7] specification of HTTP messages and content bodies for the Npkmf_ResolveRemoteUserId API is contained in Annex A.x.

#### 6.x.2.2	HTTP standard headers

##### 6.x.2.2.1	General

See clause 5.2.2 of 3GPP TS 29.500 [5] for the usage of HTTP standard headers.

##### 6.x.2.2.2	Content type

JSON, IETF RFC 8259 [9], shall be used as content type of the HTTP bodies specified in the present specification as specified in clause 5.4 of 3GPP TS 29.500 [5]. The use of the JSON format shall be signalled by the content type "application/json".

"Problem Details" JSON object shall be used to indicate additional details of the error in a HTTP response body and shall be signalled by the content type "application/problem+json", as defined in IETF RFC 7807 [10].

#### 6.x.2.3	HTTP custom headers

The mandatory HTTP custom header fields specified in clause 5.2.3.2 of 3GPP TS 29.500 [5] shall be supported, and the optional HTTP custom header fields specified in clause 5.2.3.3 of 3GPP TS 29.500 [5] may be supported.

### 6.x.3	Resources

#### 6.x.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.x.3.1-1 depicts the resource URIs structure for the Npkmf_ResolveRemoteUserId API.

Figure 6.x.3.1-1: Resource URI structure of the Npkmf_ResolveRemoteUserId API

Table 6.x.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.x.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |

### 6.x.4	Custom Operations without associated resources

#### 6.x.4.1	Overview

Table 6.x.4.1-1: Custom operations without associated resources

| Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- |
| resolve-id | POST | Resolve ProSe Remote User ID (i.e., UP-PRUK ID) to SUPI |

#### 6.x.4.2	Operation: resolve-id

##### 6.x.4.2.1	Description

This custom operation is used by the NF service consumer (e.g., SMF, PKMF) to request to resolve ProSe Remote User ID (i.e., UP-PRUK) to SUPI.

The URI of this custom operation is: {apiRoot}/npkmf-userid/<apiVersion>/resolve-id

##### 6.x.4.2.2	Operation Definition

This operation shall support the response data structures and response codes specified in tables 6.x.4.2.2-1 and 6.x.4.2.2-2.

Table 6.x.4.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ResolveRequest | M | 1 | Resolve Request data. |

Table 6.x.4.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ResolveResponse | M | 1 | 200 OK | Upon success, the response data contain the SUPI of the ProSe Remote UE. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - USER_NOT_FOUND See table 6.x.7.3-1 for the description of these errors. |
| NOTE 1: The manadatory HTTP error status code for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [5] also apply. NOTE 2: RedirectResponse may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [5]. |  |  |  |  |

Table 6.x.4.2.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same PKMF or PKMF (service) set. For the case, when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [5]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target PKMF (service) instance ID towards which the request is redirected |

Table 6.x.4.2.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same PKMF or PKMF (service) set. For the case, when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [5]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target PKMF (service) instance ID towards which the request is redirected |

### 6.x.5	Notifications

There is no notification defined for Npkmf_ResolveRemoteUserId service.

### 6.x.6	Data Model

#### 6.x.6.1	General

This clause specifies the application data model supported by the API.

Table 6.x.6.1-1 specifies the data types defined for the Npkmf_ResolveRemoteUserId service based interface protocol.

Table 6.x.6.1-1: Npkmf_ResolveRemoteUserId specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| ResolveRequest | 6.x.6.2.2 | Request Data |  |
| ResolveResponse | 6.x.6.2.3 | Response Data |  |

Table 6.x.6.1-2 specifies data types re-used by the Npkmf_ResolveRemoteUserId service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Npkmf_ResolveRemoteUserId service based interface.

Table 6.x.6.1-2: Npkmf_ResolveRemoteUserId re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| PrukId | 3GPP TS 29.559 | See clause 6.1.6.3.2 |  |
| PlmnId | 3GPP TS 29.571 [15] | PLMN ID |  |
| Supi | 3GPP TS 29.571 [15] | Subscription Permanent Identifier |  |

#### 6.x.6.2	Structured data types

##### 6.x.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 6.x.6.2.2	Type: ResolveRequest

Table 6.x.6.2.2-1: Definition of type ResolveRequest

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| upPrukId | PrukId | M | 1 | UP-PRUK ID of the ProSe Remote UE |  |
| plmnId | PlmnId | O | 0..1 | HPLMN ID of the 5G ProSe Remote UE |  |

##### 6.x.6.2.3	Type: ResolveResponse

Table 6.x.6.2.2-1: Definition of type ResolveResponse

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supi | Supi | M | 1 | The SUPI of the UE |  |

#### 6.x.6.3	Simple data types and enumerations

There are no simply data types and enumerations defined in Npkmf_ResolveRemoteUserId Service.

#### 6.x.6.4	Data types describing alternative data types or combinations of data types

There is no data type describing alternative data types or combinations of data types in Npkmf_ResolveRemoteUserId Service.

#### 6.x.6.5	Binary data

There is no binary data type in Npkmf_ResolveRemoteUserId Service.

### 6.x.7	Error Handling

#### 6.x.7.1	General

For the Npkmf_ResolveRemoteUserId API, HTTP error responses shall be supported as specified in clause 4.8 of 3GPP TS 29.501 [6]. Protocol errors and application errors specified in table 5.2.7.2-1 of 3GPP TS 29.500 [5] shall be supported for an HTTP method if the corresponding HTTP status codes are specified as mandatory for that HTTP method in table 5.2.7.1-1 of 3GPP TS 29.500 [5].

In addition, the requirements in the following clauses are applicable for the Npkmf_ResolveRemoteUserId API.

#### 6.x.7.2	Protocol Errors

No specific procedures for the Npkmf_ResolveRemoteUserId service are specified.

#### 6.x.7.3	Application Errors

The application errors defined for the Npkmf_ResolveRemoteUserId service are listed in Table 6.x.7.3-1.

Table 6.x.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| USER_NOT_FOUND | 404 Not Found | The provided subscriber identifier is not found. |

### 6.x.8	Feature negotiation

The optional features in table 6.x.8-1 are defined for the Npkmf_ResolveRemoteUserId API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [5].

Table 6.x.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 6.x.9	Security

As indicated in 3GPP TS 33.501 [8] and 3GPP TS 29.500 [5], the access to the Npkmf_ResolveRemoteUserId API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [9]), based on local configuration, using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [13]) plays the role of the authorization server.

If OAuth2 is used, an NF Service Consumer, prior to consuming services offered by the Npkmf_ResolveRemoteUserId API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [13], clause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Npkmf_ResolveRemoteUserId service.

The Npkmf_ResolveRemoteUserId API defines a single scope "npkmf-userid" for the entire service, and it does not define any additional scopes at resource or operation level.

### 6.x.10	HTTP redirection

An HTTP request may be redirected to a different 5G PKMF service instance, within the same 5G PKMF or a different 5G PKMF of an 5G PKMF set, e.g. when an 5G PKMF service instance is part of an 5G PKMF (service) set or when using indirect communications (see 3GPP TS 29.500 [5]).

An SCP that reselects a different 5G PKMF producer instance will return the NF Instance ID of the new 5G PKMF producer instance in the 3gpp-Sbi-Producer-Id header, as specified in clause 6.10.3.4 of 3GPP TS 29.500 [5].

If an 5G PKMF within an 5G PKMF set redirects a service request to a different 5G PKMF of the set using an 307 Temporary Redirect or 308 Permanent Redirect status code, the identity of the new 5G PKMF towards which the service request is redirected shall be indicated in the 3gpp-Sbi-Target-Nf-Id header of the 307 Temporary Redirect or 308 Permanent Redirect response as specified in clause 6.10.9.1 of 3GPP TS 29.500 [5].

* * * Next Change * * *

# A.x	Npkmf_ResolveRemoteUserId API

openapi: 3.0.0

info:

title: Npkmf_ResolveRemoteUserId

version: 1.0.0

description: |

PKMF Resolve Remote User Id Service.

© 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.559 V17.3.0; 5G System; 5G ProSe Anchor Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.559/

servers:

- url: '{apiRoot}/npkmf-userid/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- npkmf-userid

paths:

/resolve-id:

post:

summary: Retrieve the SUPI of the ProSe Remote UE

operationId: RetrieveSUPI

tags:

- SUPI Retrieval

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/ResolveRequest'

required: true

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/ResolveResponse'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

npkmf-userid: Access to the Npkmf_ResolveRemoteUserId API

schemas:

#

#  Structured Data Types

#

ResolveRequest:

description: Request Data

type: object

properties:

upPrukId:

$ref: 'TS29559_Npkmf_PKMFKeyRequest.yaml#/components/schemas/PrukId'

plmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

required:

- upPrukId

ResolveResponse:

description: Response Data

type: object

properties:

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

required:

- supi

#

#  Simple Data Types

#

#

#  Enumeration Data Types

#

* * * End of Changes * * *