3GPP TSG-CT Meeting #100	CP-231203

# Taipei, 12th – 13th June 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.549 | CR | 0157 | rev | 2 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | VAL service area support in the SS_LocationReporting API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Work item code: | SEAL_Ph3 |  |  |  |  |  | Date: |  |  | 2023-06-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clause 9.3.2.4 of 23.434 defines the support of the VAL service area functionality, i.e., the “Triggering criteria” IE states: “…It may include VAL service Area ID based criteria when this information flow is from VAL server to the location management server." Thus, the VAL service area support and the related triggering criteria shall be specified in 29.549. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR introduces the VAL service area support and the related triggering criteria in the SS_LocationReporting API. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Misalignment with stage 2 requirements |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.4.1, 7.1.1.4.2.2, 7.1.1.4.2.3, 7.1.1.4.2.4(new), 7.1.1.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | - This CR does not impact any OpenAPI file. |  |  |  |  |  |  |  |  |
| () |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | - In revision 2, clause 7.1.1.4.2.3 was updated to remove "(NOTE)" from description of the "accuracy" attribute. |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

* * * First Change * * * *

##### 7.1.1.4.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 6.2 apply to this API.

Table 7.1.1.4.1-1 specifies the data types defined specifically for the SS_LocationReporting API service.

Table 7.1.1.4.1-1: SS_LocationReporting API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| LocationReportConfiguration | 7.1.1.4.2.2 | Represents the location reporting configuration information. |  |
| LocationReportConfigurationPatch | 7.1.1.4.2.3 | Represents the requested modifications to the location reporting configuration information. Used to partially update Individual SEAL Location Reporting Configuration resource. | PatchUpdate |
| ValServAreaTriggCriteria | 7.1.1.4.2.4 | Represents the location reporting triggering criteria for the given VAL service area ID. | SEAL3_ValSrvArea |

Table 7.1.1.4.1-2 specifies data types re-used by the SS_LocationReporting API service.

Table 7.1.1.4.1-2: SS_LocationReporting API Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| Accuracy | 3GPP TS 29.122 [3] | Used to represent the desired level of accuracy of the requested location information. |  |
| DateTime | 3GPP TS 29.571 [21] | Used to represent the subscription duration. |  |
| DurationSec | 3GPP TS 29.571 [21] | Used to represent the time interval between successive location reports. |  |
| SupportedFeatures | 3GPP TS 29.571 [21] | Used to negotiate the applicability of optional features defined in table 7.1.1.6-1. |  |
| ValTargetUe | Clause 7.3.1.4.2.3 | Used to indicate either VAL User ID or VAL UE ID, to which location reporting applies. |  |  |

* * * Next Change * * * *

###### 7.1.1.4.2.2	Type: LocationReportConfiguration

Table 7.1.1.4.2.2-1: Definition of type LocationReportConfiguration

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valServerId | string | M | 1 | Represents the VAL server identifier. |  |
| valTgtUe | ValTargetUe | M | 1 | Represents the VAL User ID or VAL UE ID to which the location reporting applies. |  |
| immRep | boolean | O | 0..1 | Indication of immediate reporting. If included, when it is set to true it indicates immediate reporting of the subscribed events, if available. Otherwise, reporting will occur when the event is met. |  |
| monDur | DateTime | O | 0..1 | Represents the time at which the subscription ceases to exist (i.e the reporting trigger becomes invalid). If omitted, there is no time limit. |  |
| repPeriod | DurationSec | O | 0..1 | Indicates the time interval between successive location reports. |  |
| accuracy | Accuracy | O | 0..1 | Represents the desired level of accuracy of the requested location information. |  |
| valSrvAreaTrig | ValServAreaTriggCriteria | O | 0..1 | Represents the location report triggering criteria based on the VAL service area information. | SEAL3_ValSrvArea |
| suppFeat | SupportedFeatures | O | 0..1 | Used to negotiate the supported features of the API as defined in clause 7.1.1.6. This attribute shall be provided in the HTTP POST request and in the response of successful resource creation. |  |

Ediror's Note:	The definition and encoding of the "valSrvAreaTrig" attribute is FFS and pending feedback from stage 2.

Ediror's Note:	The applicability and presence conditions of the "repPeriod" and "accuracy" attributes when the "valSrvAreaTrig" attribute is present is FFS and pending feedback from stage 2.

* * * Next Change * * * *

###### 7.1.1.4.2.3	Type: LocationReportConfigurationPatch

Table 7.1.1.4.2.3-1: Definition of type LocationReportConfigurationPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valTgtUe | ValTargetUe | O | 1 | Represents the VAL User ID or VAL UE ID to which the location reporting applies. |  |
| monDur | DateTime | O | 0..1 | Represents the time at which the subscription ceases to exist (i.e the reporting trigger becomes invalid). If omitted, there is no time limit. |  |
| repPeriod | DurationSec | O | 0..1 | Indicates the time interval between successive location reports. |  |
| accuracy | Accuracy | O | 0..1 | Represents the desired level of accuracy of the requested location information. |  |
| valSrvAreaTrig | ValServAreaTriggCriteria | O | 0..1 | Represents the location report triggering criteria based on the VAL service area information. | SEAL3_ValSrvArea |

Editor's Note:	The definition and encoding of the "valSrvAreaTrig" attribute is FFS and pending feedback from stage 2.

Editor's Note:	The applicability and presence conditions of the "repPeriod" and "accuracy" attributes when the "valSrvAreaTrig" attribute is present is FFS and pending feedback from stage 2.

* * * Next Change * * * *

###### 7.1.1.4.2.4	Type: ValServAreaTriggCriteria

Table 7.1.1.4.2.4-1: Definition of type ValServAreaTriggCriteria

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valSvcAreaId | string | M | 1 | Represents the VAL service area. |  |

Editor's note:	The cardinality of the "valSvcAreaId" attribute is FFS and to be aligned based on stage-2 requirements.

Editor's note:	The full content of the ValServAreaTriggCriteria data type is FFS and pending feedback from stage 2.

* * * Next Change * * * *

#### 7.1.1.6	Feature negotiation

General feature negotiation procedures are defined in clause 6.8.

Table 7.1.1.6-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | PatchUpdate | Indicates the support of the PATCH method for updating an Individual SEAL Location Reporting Configuration resource. |
| 2 | SEAL3_ValSrvArea | Indicates the support of the VAL service area functionality. |

* * * End of changes * * * *