3GPP TSG-CT WG1 Meeting #142	C1-234044

Bratislava 22– 26 May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.571 | CR | 0026 | rev | 4 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Edit misaligned note |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-05-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As described below in TS 24.501, there are two conditions of Additional information IE in Location services message container: g) "Location services message container": 1) if the Additional information IE is not included in the UL NAS TRANSPORT message, the AMF shall provide the Payload container type and the content of the Payload container IE to the location services application; and 2) if the Additional information IE is included in the UL NAS TRANSPORT message, the AMF shall send the Payload container type and the content of the Payload container IE to an LMF associated with routing information included in the Additional information IE of the UL NAS TRANSPORT message. For the condition 2), if the Additional information IE is included in the UL NAS TRANSPORT message, the AMF shall send the routing information which is included in the Additional information IE associated with the Payload container type and the content of the Payload container IE to an LMF. Hence, when message signalling is transported in the Payload container, the optional Additional Information IE (if it is included) of the UL/DL NAS TRANSPORT message is used. However, these NOTE below in TS 24.571 are misaligned with the description in TS 24.501. 5.2.1.1.1: NOTE: The optional Additional Information IE of the DL/UL NAS Transport message is not used when the LCS Location Notification signalling is transported in the Payload container. 5.2.1.4.1: NOTE: The optional Additional Information IE of the DL/UL NAS Transport message is not used when the LCS CancelDeferredLocation signalling is transported in the Payload container. 5.2.2.1.1: NOTE: The optional Additional Information IE of the UL/DL NAS TRANSPORT message is not used when the MO-LR signaling is transported in the Payload container. Therefore, it is proposed to delete the misaligned notes in TS 24.571. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Edit the misaligned notes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Existing misaligned notes will cause confusions when dealing with cases where Additional information IE is included in UL NAS TRANSPORT message. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.1.1, 5.2.1.4.1, 5.2.2.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

5.2.1.1.1	General

The supplementary services Location Notification operation enables the AMF to trigger the end-user notification verification process on the UE using NAS signalling. The supplementary services messages are transported using the DL NAS Transport message and the UL NAS Transport message defined in 3GPP TS 24.501 [3]. UE Location Privacy Indication information may be included in Location Notification Return Result to indicate whether subsequent LCS requests will be allowed or disallowed by the UE. Figure 5.2.1.1.1.1 illustrates an example of the NAS signalling transport applicable to a 5GC-MT-LR procedure for immediate or deferred location.

Figure 5.2.1.1.1.1: NAS signalling transport for Supplementary Services Location Notification

NOTE:	The optional Additional Information IE of the DL/UL NAS Transport message is not included when the LCS Location Notification signalling is transported in the Payload container.

* * * Next Change * * * *

5.2.1.4.1	General

The supplementary services Cancel Deferred Location operation enables the AMF to cancel ongoing periodic or triggered location in a target UE using NAS signalling as described in 3GPP TS 23.273 [2] clause 6.3.3. The supplementary services Cancel Deferred Location messages are transported using the DL NAS Transport message and the UL NAS Transport message defined in 3GPP TS 24.501 [3]. Figure 5.2.1.4.1-1 illustrates an example of the NAS signalling transport.

Figure 5.2.1.4.1.1: NAS signalling transport for Cancel Deferred Location

NOTE:	The optional Additional Information IE of the DL/UL NAS Transport message is not included  when the LCS CancelDeferredLocation signalling is transported in the Payload container.

* * * Next Change * * * *

5.2.2.1.1	General

The supplementary services MO-LR operation enables the UE to launch MO positioning session or request location assistance data using NAS signaling. The NAS signaling are transported using the DL NAS Transport message and the Uplink NAS Transport message defined in 3GPP TS 24.501 [3]. Figure 5.2.2.1.1-1 illustrates an example of the NAS signaling transport for an MO-LR session.

Figure 5.2.2.1.1-1: NAS signaling transport for MO-LR

NOTE:	The optional Additional Information IE of the UL/DL NAS TRANSPORT message is not included  when the MO-LR signaling is transported in the Payload container.

* * * End of Changes * * * *