3GPP TSG-CT Meeting #100	CP-231250

Taipei, 12th – 13th June 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.523 | CR | 0089 | rev | 2 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of application detection event exposure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia, Nokia Shanghai Bell, China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Ericsson, Nokia, Nokia Shanghai Bell, China Mobile |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_ADEE |  |  |  |  |  | Date: |  |  | 2023-06-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TS 23.503 and TS 23.502 define application detection exposure mechanisms for bulk subscription from the PCF to the AF. Stage 3 specifications need to be updated accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | TS 29.523 is updated in order to: - Introduce new application detection events. - Introduce the AF as an NF Service Consumer of Npcf_EventExposure service. - Introduce the application identifier during the subscription of the Application Start event. - Allow the notification of the related events together with the IP address(es) of the impacted UEs and the detected application. - Define the corresponding feature. - Update OpenAPI specification accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing functionality. Misalignment with stage 2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.1; 4.1.2; 4.1.3.2; 4.2.2.1; 4.2.2.2; 4.2.4.2; 5.6.1; 5.6.2.2; 5.6.2.8; 5.6.3.3; 5.8; A.2. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces a backwards compatible feature in the Npcf_EventExposure OpenAPI specification. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | C3-232480 agreed in CT3#128 is updated to correct the name of the ApplicationId data type in the OpenAPI specification. |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

# *** 1st Change ***

### 4.1.1	Overview

The Policy Event Exposure Service, as defined in 3GPP TS 23.502 [3] and 3GPP TS 23.503 [4], is provided by the Policy Control Function (PCF).

This service:

-	allows NF service consumers to subscribe to, modify and unsubscribe from policy control events; and

-	notifies NF service consumers with a corresponding subscription about observed events on the PCF.

The types of observed events include:

-	PLMN identifier notification;

NOTE 1:	Within the PLMN identifier notification event the PLMN Identifier or SNPN Identifier where the UE is currently located is provided. The SNPN Identifier consists of the PLMN Identifier and the NID.

NOTE 2:	Mobility between non-equivalent SNPNs, and between SNPN and PLMN is not supported. When the UE is operating in SNPN access mode, the trigger reports changes of equivalent SNPNs.

-	access type change;

-	satellite backhaul category change;

-	service area coverage change;

-	successful or unsuccessful outcome of the UE Policy Delivery; and

-	application traffic detection events.

The target of the event reporting may include a group of UE(s) or any UE (i.e. all UEs). When an event occurs, to which the NF service consumer has subscribed, the PCF reports the requested information to the NF service consumer based on the event reporting information definition requested by the NF service consumer (see 3GPP TS 23.502 [3], clause 4.15.1).

# *** 2nd Change ***

### 4.1.2	Service Architecture

The 5G System Architecture is defined in 3GPP TS 23.501 [2]. The Policy and Charging related 5G architecture and signalling flows are also described in 3GPP TS 29.513 [8].

The Policy Event Exposure Service (Npcf_EventExposure) is part of the Npcf service-based interface exhibited by the Policy Control Function (PCF).

The known NF service consumers of the Npcf_EventExposure service are the Network Exposure Function (NEF) and the Application Function (AF).

The Npcf_EventExposure service is provided by the PCF and consumed by NF service consumers (e.g. NEF, AF), as shown in figure 4.1.2-1 for the SBI representation model and in figure 4.1.2-2 for reference point representation model.

Figure 4.1.2-1: Npcf_EventExposure service Architecture, SBI representation

Figure 4.1.2-2: Npcf_EventExposure service Architecture, reference point representation

NOTE:	The NWDAF and the DCCF can be consumers of the Npcf_EventExposure service to perform data collection. However, there is no data collected from the PCF by the NWDAF or the DCCF defined in this release of the specification.

# *** 3rd Change ***

#### 4.1.3.2	NF Service Consumers

As indicated in clause 4.1.2 above, the known NF service consumer of the Npcf_EventExposure service are the Network Exposure Function (NEF) and the Application Function (AF).

The Network Exposure Function (NEF) is a functional element that supports the following functionalities:

-	The NEF securely exposes network capabilities and events provided by 3GPP NFs to AF.

-	The NEF provides a means for the AF to securely provide information to 3GPP network and can authenticate, authorize and assist in throttling the AF.

-	The NEF translates the information received from the AF to the one sent to internal 3GPP NFs, and vice versa.

-	The NEF supports exposing information (collected from other 3GPP NFs) to the AF.

The Application Function (AF) is a functional element offering control to applications that require the policy and charging control of traffic plane resources; specific user plane paths for the requested traffic, the monitoring of the required service QoS, and/or specific QoS and alternative QoS profiles. The AF uses the Npcf_EventExposure service to receive exposed information from the 5GC network.

# *** 4th Change ***

#### 4.2.2.1	General

This service operation is used by an NF service consumer to explicitly subscribe for policy events notifications on a specified context for a group of UE(s) or any UE, or to modify an existing subscription.

The following are the types of events for which a subscription can be made:

-	PLMN identifier notification;

NOTE 1:	Within the PLMN identifier notification event the PLMN Identifier or SNPN Identifier where the UE is currently located is provided. The SNPN Identifier consists of the PLMN Identifier and the NID.

NOTE 2:	Mobility between non-equivalent SNPNs, and between SNPN and PLMN is not supported. When the UE is operating in SNPN access mode, the trigger reports changes of equivalent SNPNs.

-	change of Access Type;

-	when the feature "AMPoliciesEvents" is supported, change of Service Area Coverage;

-	when the feature "SatelliteBackhaul" is supported, satellite backhaul category change;

-	when the feature "DeliveryOutcome" is supported, UE Policy delivery outcome; and

-	when the feature "AppDetection" is supported, application traffic detection (Start/Stop) event notification.

The following procedures using the Npcf_EventExposure_Subscribe service operation are supported:

-	creating a new subscription;

-	modifying an existing subscription.

NOTE 3:	It is also possible to implicitly subscribe for policy events notifications for a single UE, for a group of UE(s) or any UE. Implicit subscription information is obtained from the UDR for application data. In this case, the PCF will use the callback URI provided by the AF to the UDR, see 3GPP TS 29.519 [24] for the details.

# *** 5th Change ***

#### 4.2.2.2	Creating a new subscription

Figure 4.2.2.2-1 illustrates the creation of a subscription.

Figure 4.2.2.2-1: Creation of a subscription

To subscribe to event notifications, the NF service consumer shall send an HTTP POST request with: "{apiRoot}/npcf-eventexposure/v1/subscriptions" as request URI as shown in figure 4.2.2.2-1, step 1, and the "PcEventExposureSubsc" data structure as request body.

The "PcEventExposureSubsc" data structure shall include:

-	identification of the policy events to subscribe as "eventSubs" attribute;

-	indication of the UEs to which the subscription applies via:

a)	identification of a group of UE(s) via a "groupId" attribute; or

b)	identification of any UE by omitting the "groupId" attribute;

-	a URI where to receive the requested notifications as "notifUri" attribute; and

-	a Notification Correlation Identifier assigned by the NF service consumer for the requested notifications as "notifId" attribute.

The "PcEventExposureSubsc" data structure may also include:

-	description of the event reporting information as "eventsRepInfo", which may include:

a)	event notification method (periodic, one time, on event detection) as "notifMethod" attribute;

b)	Maximum Number of Reports as "maxReportNbr" attribute;

c)	Monitoring Duration as "monDur" attribute;

d)	repetition period for periodic reporting as "repPeriod" attribute;

e)	immediate reporting indication as "immRep" attribute;

f)	sampling ratio as "sampRatio" attribute;

g)	group reporting guard time as "grpRepTime" attribute;

h)	partitioning criteria for partitioning the UEs before performing sampling as "partitionCriteria" attribute if the EneNA feature is supported; and/or

i)	a notification flag as "notifFlag" attribute if the EneNA feature is supported;

-	if the supported feature "ExtendedSessionInformation" is supported, to filter the AF sessions for which the policy event report shall occur, the identification of the services one or more AF sessions may belong to as "filterServices" attribute, which may include per service identification:

a)	a list of ethernet flows in the "servEthFlows" attribute; or

b)	a list of IP flows in the "servIpFlows" attribute; and/or

c)	an AF application identifier in the "afAppId" attribute;

-	to filter the DNNs for which the policy event report shall occur, the identification of the DNNs in the "filterDnns" attribute;

-	to filter the S-NSSAIs for which the policy event report shall occur, the identification of the S-NSSAIs in the "filterSnssais" attribute;

-	when the feature "EneNA" and/or "AppDetection" are supported, to filter the specific DNN and S-NSSAI combination list for which the policy event report shall occur, the identification of each combination within the "snssaiDnn" attribute; and

-	when the feature "AppDetection" is supported, to indicate the specific application(s) for which the policy event report shall occur, the application identifier(s) in the "appIds" attribute.

If the PCF cannot successfully fulfil the received HTTP POST request due to an internal PCF error or an error in the HTTP POST request, the PCF shall send an HTTP error response as specified in clause 5.7.

Upon successful reception of the HTTP POST request with "{apiRoot}/npcf-eventexposure/v1/subscriptions" as request URI and "PcEventExposureSubsc" data structure as request body, the PCF shall create a new "Individual Policy Events Subscription" resource, store the subscription and send a HTTP "201 Created" response as shown in figure 4.2.2.2-1, step 2. The PCF shall include in the "201 Created" response:

-	a Location header field; and

-	an "PcEventExposureSubsc" data type in the payload body.

The Location header field shall contain the URI of the created individual application session context resource i.e. "{apiRoot}/npcf-eventexposure/v1/subscriptions/{subscriptionId}".

The "PcEventExposureSubsc" data type payload body shall contain the representation of the created "Individual Policy Events Subscription".

When the "monDur" attribute is included in the response, it represents a server selected expiry time that is equal or less than a possible expiry time in the request.

When the "immRep" attribute set to true is included in the subscription and the subscribed policy control events are available:

-	if the feature "ERIR" is not supported, the PCF shall immediately notify the NF service consumer with the current available value(s) for the subscribed event(s) using the Npcf_EventExposure_Notify service operation, as described in clause 4.2.4.2.

-	if the feature "ERIR" is supported, the PCF shall immediately notify the NF service consumer with the current available value(s) for the subscribed event(s) within the HTTP "201 Created" response as shown in figure 4.2.2.2-1, step 2. The "PcEventExposureSubsc" data type shall include the corresponding event(s) notification within the "eventNotifs" attribute, as described in clause 4.2.4.2.

When the sampling ratio as the "sampRatio" attribute is included in the subscription without a "partitionCriteria" attribute, the PCF shall select a random subset of UEs among the target UEs according to the sampling ratio and only report the event(s) related to the selected subset of UEs. If the "partitionCriteria" attribute is additionally included, then the PCF shall first partition the UEs according to the value of the "partitionCriteria" attribute and then select a random subset of UEs from each partition according to the sampling ratio and only report the event(s) related to the selected subsets of UEs.

When the group reporting guard time as the "grpRepTime" attribute is included in the subscription, the PCF shall accumulate all the event reports for the target UEs until the group reporting guard time expires. Then the PCF shall notify the NF service consumer using the Npcf_EventExposure_Notify service operation, as described in clause 4.2.4.2.

When the "notifFlag" attribute is included and set to "DEACTIVATE" in the request, the PCF shall mute the event notification and store the available events.

# *** 6th Change ***

#### 4.2.4.2	Notification about subscribed events

Figure 4.2.4.2-1 illustrates the notification about subscribed events.

Figure 4.2.4.2-1: Notification about subscribed events

If the PCF observes policy control related event(s) for which an NF service consumer has subscribed explicitly as defined in clause 4.2.2 or implicitly when the subscription information is obtained from the UDR for application data, the PCF shall send an HTTP POST request as shown in figure 4.2.4.2-1, step 1, with the "{notifUri}" as request URI containing the value previously provided by the NF service consumer within the corresponding subscription or containing the callback URI provided by the AF to the UDR, and the "PcEventExposureNotif" data structure.

The "PcEventExposureNotif" data structure shall include:

-	The notification correlation ID provided by the NF service consumer during the subscription as "notifId" attribute or obtained from the UDR as specified in 3GPP TS 29.519 [24]; and

-	information about the observed event(s) within the "eventNotifs" attribute that shall contain for each observed event an "PcEventNotification" data structure that shall include:

1.	the Policy Control event as "event" attribute;

2.	for an access type change:

a)	new access type as "accType" attribute;

b)	the new RAT type as "ratType" attribute, if applicable for the notified access type; and

c)	if the "ATSSS" feature is supported:

i.	if it is the first access type report for a PDU session, and both, 3GPP and non-3GPP access information is available, the "addAccessInfo" attribute. The "addAccessInfo" attribute contains the additional access type information, where the access type is encoded in the "accessType" attribute, and the RAT type is encoded in the "ratType" attribute when applicable for the notified access type;

ii.	if it is a subsequent access type change report:

-	if a new access type is added to the MA PDU session, the "addAccessInfo" attribute with the added access type encoded in the "accessType" attribute, and the RAT type encoded in the "ratType" attribute when applicable for the notified access type;

-	if an access type is released in the MA PDU session, the "relAccessInfo" attribute with the released access type encoded in the "accessType" attribute, and the RAT type encoded in the "ratType" attribute when applicable for the notified access type; and

NOTE 1:	For a MA PDU session, if the "ATSSS" feature is not supported by the AF, the PCF includes the "accessType" attribute and the "ratType" attribute with a currently active combination of access type and RAT type (if applicable for the notified access type). When both 3GPP and non-3GPP accesses are available, the PCF includes the information corresponding to the 3GPP access.

d)	for EPC interworking scenarios, the ePDG address as "anGwAddr" attribute, if applicable for the notified access type;

3.	for a PLMN change:

a)	new network identity containing the PLMN Identifier or the SNPN Identifier in the "plmnId" attribute;

NOTE 2:	The SNPN Identifier consists of the PLMN Identifier and the NID.

NOTE 3:	Mobility between non-equivalent SNPNs, and between SNPN and PLMN is not supported. When the UE is operating in SNPN access mode, the trigger reports changes of equivalent SNPNs.

4.	when the feature "AMPoliciesEvents" is supported, for a service area coverage change, the new service area coverage in the "appliedCov" attribute, encoded as specified in 3GPP TS 29.534 [23], clause 4.2.7.4;

NOTE 4:	The service area coverage change event is met and the notification is triggered when the PCF determines the tracking areas where the service is allowed in relation to the NF consumer requested service area coverage. The actual service area coverage for the UE might be larger than the one reported with the service area coverage change event.

5.	when the feature "SatelliteBackhaul" is supported, for a satellite backhaul category change:

a)	the satellite backhaul category (i.e., GEO, MEO, LEO, or other satellite) or the indication of non-satellite backhaul category in the "satBackhaulCategory" attribute;

6.	when the feature "DeliveryOutcome" is supported, to report the unsuccessful outcome of the UE Policy Delivery related to the invocation of AF provisioned service parameters, the reason of failure within the "delivFailure" attribute;

7.	the identity of the affected UE in the "supi" attribute and, if available, in the "gpsi" attribute;

8.	the time at which the event was observed encoded as "timeStamp" attribute;

9.	if available, and if the feature "ExtendedSessionInformation" is supported, information about the PDU session involved in the reported event in the "pduSessInfo" attribute, that shall include:

a)	the S-NSSAI of the PDU session in the "snssai" attribute;

b)	the DNN of the PDU session in the "dnn" attribute; and

c)	the IPv4 address in the "ueIpv4" attribute and/or the IPv6 prefix in the "ueIpv6" attribute, or the Ethernet MAC address in the "ueMac" attribute; and

if the IPv4 address is included in the "ueIpv4" attribute, may include the IP domain in the "ipDomain" attribute;

10.	if available, and if the feature "ExtendedSessionInformation" is supported, information about the services involved in the reported event in the indicated PDU session in the "repServices" attribute, which may include per identified service:

a)	a list of Ethernet flows in the "servEthFlows" attribute which contains an impacted Ethernet flow number within the "flowNumber" attribute in each EthernetFlowInfo data structure; or

b)	a list of IP flows in the "servIpFlows" attribute which contains an impacted IP flow number within the "flowNumber" attribute in each IpFlowInfo data structure; and/or

c)	an AF application identifier in the "afAppId" attribute.

11.	for an application detection event and if the feature "AppDetection" is supported:

a)	the IPv4 address in the "ueIpv4" attribute and/or the IPv6 prefix in the "ueIpv6" attribute within the "pduSessInfo" attribute; and

b)	the application identifier for which the notification applies in the "appId" attribute.

If the NF service consumer cannot successfully fulfil the received HTTP POST request due to an internal error or an error in the HTTP POST request, the NF service consumer shall send an HTTP error response as specified in clause 5.7.

If the feature "ES3XX" is supported, and the NF service consumer determines the received HTTP POST request needs to be redirected, the NF service consumer shall send an HTTP redirect response as specified in clause 6.10.9 of 3GPP TS 29.500 [5].

Upon successful reception of the HTTP POST request with "{notifUri}" as request URI and a "PcEventExposureNotif" data structure as request body, the NF service consumer shall send a "204 No Content" HTTP response, as shown in figure 4.2.4.2-1, step 2, for a successful processing.

# *** 7th Change ***

### 5.6.1	General

This clause specifies the application data model supported by the API.

Table 5.6.1-1 specifies the data types defined for the Npcf_EventExposure service based interface protocol.

Table 5.6.1-1: Npcf_EventExposure specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| EthernetFlowInfo | 5.6.2.6 | Identification of an UL/DL ethernet flow. | ExtendedSessionInformation |
| IpFlowInfo | 5.6.2.7 | Identification of an UL/DL IP flow. | ExtendedSessionInformation |
| PcEvent | 5.6.3.3 | Policy Control Events. |  |
| PcEventExposureNotif | 5.6.2.3 | Describes notifications about Policy Control events that occurred in an Individual Policy Events Subscription resource. |  |
| PcEventExposureSubsc | 5.6.2.2 | Represents an Individual Policy Events Subscription resource. |  |
| PcEventNotification | 5.6.2.8 | Represents the information reported for a Policy Control event. |  |
| PduSessionInformation | 5.6.2.9 | Represents PDU session identification information. | ExtendedSessionInformation |
| ReportingInformation | 5.6.2.4 | Represents the type of reporting the subscription requires. |  |
| ServiceIdentification | 5.6.2.5 | Identification of the service to which the subscription applies. | ExtendedSessionInformation |
| SnssaiDnnCombination | 5.6.2.10 | Represents a combination of S-NSSAI and DNN(s). | EneNA |

Table 5.6.1-2 specifies data types re-used by the Npcf_EventExposure service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Npcf_EventExposure service based interface.

Table 5.6.1-2: Npcf_EventExposure re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| AccessType | 3GPP TS 29.571 [14] | Access Type. |  |
| AdditionalAccessInfo | 3GPP TS 29.512 [9] | Indicates the combination of additional Access Type and RAT Type for MA PDU session. | ATSSS |
| AfAppId | 3GPP TS 29.514 [12] | AF application Identifier. | ExtendedSessionInformation |
| AnGwAddress | 3GPP TS 29.514 [12] | Carries the control plane address of the EPC untrusted non-3GPP access network gateway. (NOTE 1) |  |
| ApplicationId | 3GPP TS 29.571 [14] | Application Identifier. | AppDetection |
| DateTime | 3GPP TS 29.571 [14] | Time stamp. |  |
| Dnn | 3GPP TS 29.571 [14] | Identifies a DNN. |  |
| DurationSec | 3GPP TS 29.571 [14] | Seconds of duration. |  |
| EthFlowDescription | 3GPP TS 29.514 [12] | Identifies an ethernet flow description. (NOTE 2) | ExtendedSessionInformation |
| Failure | 3GPP TS 29.522 [25] | Indicates the failure reason for an unsuccessful outcome of the UE Policy Delivery. | DeliveryOutcome |
| FlowDescription | 3GPP TS 29.514 [12] | Identifies an IP flow description. | ExtendedSessionInformation |
| Gpsi | 3GPP TS 29.571 [14] | Generic Public Subscription Identifier. |  |
| GroupId | 3GPP TS 29.571 [14] | Identifies a group of UEs. |  |
| MacAddr48 | 3GPP TS 29.571 [14] | Mac Address of the UE. | ExtendedSessionInformation |
| NotificationFlag | 3GPP TS 29.571 [14] | Notification flag. | EneNA |
| NotificationMethod | 3GPP TS 29.508 [15] | Represents the Notification Method. |  |
| PartitioningCriteria | 3GPP TS 29.571 [14] | Used to partition UEs before applying sampling. | EneNA |
| PlmnIdNid | 3GPP TS 29.571 [14] | Identifies the network: the PLMN Identifier or the SNPN Identifier. (NOTE 3) |  |
| RatType | 3GPP TS 29.571 [14] | RAT Type. |  |
| RedirectResponse | 3GPP TS 29.571 [14] | Contains redirection related information. | ES3XX |
| SamplingRatio | 3GPP TS 29.571 [14] | Sampling Ratio. |  |
| SatelliteBackhaulCategory | 3GPP TS 29.571 [14] | Indicates the satellite or non-satellite backhaul category. | SatelliteBackhaul |
| ServiceAreaCoverageInfo | 3GPP TS 29.534 [23] | Service area coverage in terms of tracking area codes and serving network. | AMPoliciesEvents |
| Snssai | 3GPP TS 29.571 [14] | Identifies a S-NSSAI. |  |
| Supi | 3GPP TS 29.571 [14] | Identifies the SUPI of the UE. |  |
| SupportedFeatures | 3GPP TS 29.571 [14] | Used to negotiate the applicability of the optional features defined in clause 5.8. |  |
| Uinteger | 3GPP TS 29.571 [14] | Unsigned integer. |  |
| NOTE 1: "AnGwAddress" data structure is only used to encode the ePDG address and is only applicable to the 5GS and EPC/E-UTRAN interworking scenario as defined in 3GPP TS 29.512 [9], Annex B. NOTE 2: In order to support a set of MAC addresses with a specific range in the traffic filter, feature MacAddressRange as specified in clause 5.8 shall be supported. NOTE 3: The SNPN Identifier consists of the PLMN Identifier and the NID. |  |  |  |

# *** 8th Change ***

#### 5.6.2.2	Type PcEventExposureSubsc

Table 5.6.2.2-1: Definition of type PcEventExposureSubsc

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eventSubs | array(PcEvent) | M | 1..N | Subscribed Policy Control events. |  |
| eventsRepInfo | ReportingInformation | O | 0..1 | Represents the reporting requirements of the subscription. |  |
| groupId | GroupId | C | 0..1 | Represents an internal group identifier and identifies a group of UEs. It shall be present when the subscription is targeting a Group of UE(s). |  |
| filterDnns | array(Dnn) | O | 1..N | Represents the DNNs for which the policy event report shall apply. Each DNN is a full DNN with both the Network Identifier and Operator Identifier, or a DNN with the Network Identifier only. If omitted it represents any DNN. |  |
| filterSnssais | array(Snssai) | O | 1..N | Represents the S-NSSAIs for which the policy event report shall apply. If omitted it represents any S-NSSAI. |  |
| snssaiDnn | array(SnssaiDnnCombination) | O | 1..N | Represents the combination list of S-NSSAI and DNN for which the policy event report shall apply. If omitted, it represents any combination. | EneNA, AppDetection |
| filterServices | array(ServiceIdentification) | O | 1..N | Represents the services for which the policy event report shall apply. If omitted, the policy event report shall apply for all the active services. | ExtendedSessionInformation |
| appIds | array(ApplicationId) | O | 1..N | Represents the applications for which the policy event report shall apply. It shall be provided for event "APPLICATION_START" or "APPLICATION_STOP". | AppDetection |
| notifUri | Uri | M | 1 | Notification URI for Policy Control event reporting. |  |
| notifId | string | M | 1 | Notification Correlation ID assigned by the NF service consumer. |  |
| eventNotifs | array(PcEventNotification) | C | 1..N | Represents the Policy Control Events to be reported in the Npcf_EvenExposure_Subscribe response. It shall be present in the resource creation/update response when the "eventsRepInfo" attribute includes the "immRep" attribute set to true. Otherwise, it shall be omitted. | ERIR |
| suppFeat | SupportedFeatures | C | 0..1 | This IE represents a list of Supported features used as described in clause 5.8. Shall be present in the HTTP POST request/response. (NOTE) |  |
| NOTE: In the HTTP request, it represents the set of features supported by the NF service consumer. In the HTTP response, it represents the set of features supported by both the NF service consumer and the PCF. |  |  |  |  |  |

# *** 9th Change ***

#### 5.6.2.8	Type PcEventNotification

Table 5.6.2.8-1: Definition of type PcEventNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| event | PcEvent | M | 1..N | Reported Policy Control event. |  |
| accType | AccessType | C | 0..1 | Access Type. It shall be included when the reported PcEvent is "AC_TY_CH". |  |
| addAccessInfo | AdditionalAccessInfo | O | 0..1 | Indicates the additional combination of Access Type and RAT Type available for MA PDU session. It may be present when the notified event is "AC_TY_CH" and the PDU session is a Multi-Access PDU session. | ATSSS |
| relAccessInfo | AdditionalAccessInfo | O | 0..1 | Indicates the release of a combination of Access Type and RAT Type available for MA PDU session. It may be present when the notified event is "AC_TY_CH" and the PDU session is a Multi-Access PDU session. | ATSSS |
| anGwAddr | AnGwAddress | O | 0..1 | ePDG address. It shall be included if applicable when the reported PcEvent is "AC_TY_CH". |  |
| ratType | RatType | O | 0..1 | RAT Type. It shall be included if applicable when the reported PcEvent is "AC_TY_CH". |  |
| plmnId | PlmnIdNid | C | 0..1 | PLMN Identifier or the SNPN Identifier. It shall be included when the reported PcEvent is "PLMN_CH". (NOTE) |  |
| appliedCov | ServiceAreaCoverageInfo | C | 0..1 | The list of applied allowed Tracking Areas for the serving network where the UE is camping. It shall be included when the reported PcEvent is "SAC_CH". | AMPoliciesEvents |
| supi | Supi | C | 0..1 | SUPI of the UE. It shall be present if available. |  |
| gpsi | Gpsi | O | 0..1 | Gpsi shall contain either an External Id or an MSISDN. |  |
| timeStamp | DateTime | M | 1 | Time at which the event is observed. |  |
| pduSessInfo | PduSessionInformation | O | 0..1 | Represents PDU session information related to the observed event. | ExtendedSessionInformation, AppDetection |
| appId | ApplicationId | O | 0..1 | Represents the detected application. | AppDetection |
| repServices | ServiceIdentification | O | 0..1 | Represents service information related to the observed event. | ExtendedSessionInformation |
| satBackhaulCategory | SatelliteBackhaulCategory | C | 0..1 | Indicates the satellite or non-satellite backhaul category of the PDU session. It shall be included when the reported PcEvent is "SAT_CATEGORY_CH". | SatelliteBackhaul |
| delivFailure | Failure | C | 0..1 | Indicates the failure reason for an unsuccessful outcome of the UE Policy Delivery. It shall be included when the reported PcEvent is "UNSUCCESS_UE_POL_DEL_SP". | DeliveryOutcome |
| NOTE: The SNPN Identifier consists of the PLMN Identifier and the NID. |  |  |  |  |  |

# *** 10th Change ***

#### 5.6.3.3	Enumeration: PcEvent

The enumeration PcEvent represents the policy control events that can be subscribed. It shall comply with the provisions defined in table 5.6.3.3-1.

Table 5.6.3.3-1: Enumeration PcEvent

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| AC_TY_CH | Access Type Change |  |
| PLMN_CH | PLMN Change |  |
| SAC_CH | Service Area Coverage change | AMPoliciesEvents |
| SAT_CATEGORY_CH | Indicates that a change between different satellite backhaul category, or non-satellite backhaul, has been detected. | SatelliteBackhaul |
| SUCCESS_UE_POL_DEL_SP | Indicates about the successful UE Policy delivery related to the invocation of AF provisioned service parameters. | DeliveryOutcome |
| UNSUCCESS_UE_POL_DEL_SP | Indicates about the unsuccessful UE Policy delivery related to the invocation of AF provisioned service parameters. | DeliveryOutcome |
| APPLICATION_START | The start of application traffic has been detected. | AppDetection |
| APPLICATION_STOP | The stop of application traffic has been detected. | AppDetection |

# *** 11th Change ***

## 5.8	Feature negotiation

The optional features in table 5.8-1 are defined for the Npcf_EventExposure API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [5].

Table 5.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | ExtendedSessionInformation | Indicates the support of additional session information in the subscription and report of policy control event. |
| 2 | MacAddressRange | Indicates the support of a set of MAC addresses with a specific range in the traffic filter. |
| 3 | ATSSS | Indicates the support of the report of the multiple access types of a MA PDU session. |
| 4 | ES3XX | Extended Support for 3xx redirections. This feature indicates the support of redirection for any service operation, according to Stateless NF procedures as specified in clauses 6.5.3.2 and 6.5.3.3 of 3GPP TS 29.500 [5] and according to HTTP redirection principles for indirect communication, as specified in clause 6.10.9 of 3GPP TS 29.500 [5]. |
| 5 | AMPoliciesEvents | Indicates the support of the report of changes of service area coverage for a UE. |
| 6 | EneNA | This feature indicates support for the enhancements of network data analytics requirements. |
| 7 | SatelliteBackhaul | Indicates the support of the report of the satellite or non-satellite backhaul category of the PDU session. |
| 8 | DeliveryOutcome | Indicates the support of notifications about the outcome of the UE Policy delivery related to the invocation of AF provisioned service parameters. |
| 9 | ERIR | Indicates the support of immediate report within the subscription response. |
| 10 | AppDetection | Indicates the support of Application Traffic Detection Event Exposure. |

# *** 12th Change ***

# A.2	Npcf_EventExposure API

openapi: 3.0.0

info:

version: 1.3.0-alpha.2

title: Npcf_EventExposure

description: |

PCF Event Exposure Service.

© 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.523 V18.1.0; 5G System; Policy Control Event Exposure Service; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.523/

servers:

- url: '{apiRoot}/npcf-eventexposure/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- npcf-eventexposure

paths:

/subscriptions:

post:

summary: Creates a new Individual Policy Control Events Subscription resource

operationId: PostPcEventExposureSubsc

tags:

- Policy Control Events Subscription (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PcEventExposureSubsc'

responses:

'201':

description: Success

content:

application/json:

schema:

$ref: '#/components/schemas/PcEventExposureSubsc'

headers:

Location:

description: >

Contains the URI of the created individual policy control events subscription

resource, according to the structure

{apiRoot}/npcf-eventexposure/v1/subscriptions/{subscriptionId}

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

PcEventNotification:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PcEventExposureNotif'

responses:

'204':

description: No Content, Notification was succesfull.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

get:

summary: "Reads an existing Individual Policy Control Events Subscription"

operationId: GetPcEventExposureSubsc

tags:

- Individual Policy Control Events Subscription (Document)

parameters:

- name: subscriptionId

in: path

description: Policy Control Event Subscription ID.

required: true

schema:

type: string

responses:

'200':

description: OK. Resource representation is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/PcEventExposureSubsc'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

put:

summary: "Modifies an existing Individual Policy Control Events Subscription "

operationId: PutPcEventExposureSubsc

tags:

- Individual Policy Control Events Subscription (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PcEventExposureSubsc'

parameters:

- name: subscriptionId

in: path

description: Policy Control Event Subscription ID.

required: true

schema:

type: string

responses:

'200':

description: OK. Resource was succesfully modified and representation is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/PcEventExposureSubsc'

'204':

description: No Content. Resource was succesfully modified.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: "Cancels an existing Individual Policy Control Events Subscription "

operationId: DeletePcEventExposureSubsc

tags:

- Individual Policy Control Events Subscription (Document)

parameters:

- name: subscriptionId

in: path

description: Policy Control Event Subscription ID.

required: true

schema:

type: string

responses:

'204':

description: No Content. Resource was succesfully deleted.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

npcf-eventexposure: Access to the Npcf_EventExposure API.

schemas:

PcEventExposureNotif:

description: >

Represents notifications about Policy Control events related to an Individual

Policy Events Subscription resource.

type: object

properties:

notifId:

type: string

eventNotifs:

type: array

items:

$ref: '#/components/schemas/PcEventNotification'

minItems: 1

required:

- notifId

- eventNotifs

PcEventExposureSubsc:

description: Represents an Individual Policy Events Subscription resource.

type: object

properties:

eventSubs:

type: array

items:

$ref: '#/components/schemas/PcEvent'

minItems: 1

eventsRepInfo:

$ref: '#/components/schemas/ReportingInformation'

groupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GroupId'

filterDnns:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

minItems: 1

filterSnssais:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

minItems: 1

snssaiDnns:

type: array

items:

$ref: '#/components/schemas/SnssaiDnnCombination'

minItems: 1

filterServices:

type: array

items:

$ref: '#/components/schemas/ServiceIdentification'

minItems: 1

appIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

minItems: 1

notifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

notifId:

type: string

eventNotifs:

type: array

items:

$ref: '#/components/schemas/PcEventNotification'

minItems: 1

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- eventSubs

- notifId

- notifUri

ReportingInformation:

description: Represents the type of reporting that the subscription requires.

type: object

properties:

immRep:

type: boolean

notifMethod:

$ref: 'TS29508_Nsmf_EventExposure.yaml#/components/schemas/NotificationMethod'

maxReportNbr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

monDur:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

repPeriod:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

sampRatio:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SamplingRatio'

partitionCriteria:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PartitioningCriteria'

minItems: 1

description: Criteria for partitioning the UEs before applying the sampling ratio.

grpRepTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

notifFlag:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NotificationFlag'

ServiceIdentification:

description: Identifies the service to which the subscription applies.

type: object

properties:

servEthFlows:

type: array

items:

$ref: '#/components/schemas/EthernetFlowInfo'

minItems: 1

servIpFlows:

type: array

items:

$ref: '#/components/schemas/IpFlowInfo'

minItems: 1

afAppId:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/AfAppId'

# All conditions in allOf must be met

allOf:

# First condition is that servEthFlows and servIpFlows are mutually exclusive

- not:

required: [servEthFlows, servIpFlows]

# Second condition is that at least one the servEthFlows, servIpFlows and afAppId shall be present

- anyOf:

- required: [servEthFlows]

- required: [servIpFlows]

- required: [afAppId]

EthernetFlowInfo:

description: Identifies an UL/DL ethernet flow.

type: object

properties:

ethFlows:

type: array

items:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/EthFlowDescription'

minItems: 1

maxItems: 2

flowNumber:

type: integer

required:

- flowNumber

IpFlowInfo:

description: Identifies an UL/DL IP flow.

type: object

properties:

ipFlows:

type: array

items:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/FlowDescription'

minItems: 1

maxItems: 2

flowNumber:

type: integer

required:

- flowNumber

PcEventNotification:

description: Represents the information reported for a Policy Control event.

type: object

properties:

event:

$ref: '#/components/schemas/PcEvent'

accType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

addAccessInfo:

$ref: 'TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/AdditionalAccessInfo'

relAccessInfo:

$ref: 'TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/AdditionalAccessInfo'

anGwAddr:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/AnGwAddress'

ratType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

plmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

satBackhaulCategory:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SatelliteBackhaulCategory'

appliedCov:

$ref: 'TS29534_Npcf_AMPolicyAuthorization.yaml#/components/schemas/ServiceAreaCoverageInfo'

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

timeStamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

pduSessionInfo:

$ref: '#/components/schemas/PduSessionInformation'

appId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

repServices:

$ref: '#/components/schemas/ServiceIdentification'

delivFailure:

$ref: 'TS29522_ServiceParameter.yaml#/components/schemas/Failure'

required:

- event

- timeStamp

PduSessionInformation:

description: Represents PDU session identification information.

type: object

properties:

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

ueIpv4:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

ueIpv6:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

ipDomain:

type: string

ueMac:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MacAddr48'

required:

- snssai

- dnn

oneOf:

- required: [ueMac]

- anyOf:

- required: [ueIpv4]

- required: [ueIpv6]

SnssaiDnnCombination:

description: Represents a combination of S-NSSAI and DNN(s).

type: object

properties:

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

dnns:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

minItems: 1

# Simple data types and Enumerations

PcEvent:

description: Represents the policy control events that can be subscribed.

anyOf:

- type: string

enum:

- AC_TY_CH

- PLMN_CH

- SAC_CH

- SAT_CATEGORY_CH

- SUCCESS_UE_POL_DEL_SP

- UNSUCCESS_UE_POL_DEL_SP

- APPLICATION_START

- APPLICATION_STOP

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

*** End of Changes ***