3GPP TSG-CT Meeting #100	CP-231323

# Taipei, 12th – 13th June 2023													was C3-231579 was C3-231227

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.549 | CR | 0149 | rev | 2 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Definition of the API data model part of the MBS resource management service operations |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei |  |  |  |  |  |  |  |  |
| Work item code: | SEAL_Ph3 |  |  |  |  |  | Date: |  | 2023-06-02 |
| Category: | B |  |  |  |  |  | Release: |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As specified in clause 14.3.4A, 5G MBS may be used to support VAL services provisioning and new service operations have been defined for this purpose under the SS_NetworkResourceAdaptation API, i.e. the "Request_Multicast/Broadcast_Resource", "Update_Multicast/Broadcast_Resource", "Delete_Multicast/Broadcast_Resource", "Activate_Multicast_Resource" and "Deactivate_Multicast_Resource" service operations. The stage 3 definition of this API in this specification needs hence to be updated to support these new service operations. |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes to: - Update the data model to support the above-mentioned new service operations under the SS_NetworkResourceAdaptation API. |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Stage 2 requirements for MBS resource management are not fully defined/supported in stage 3. |  |  |  |  |  |  |  |
| Clauses affected: |  |  | 7.4.1.4.1, 7.4.1.4.2.5, 7.4.1.4.2.11 (new clause), 7.4.1.4.2.12 (new clause), 7.4.1.4.2.13 (new clause), 7.4.1.4.2.14 (new clause), 7.4.1.4.2.15 (new clause), 7.4.1.4.2.16 (new clause), 7.4.1.4.2.17 (new clause), 7.4.1.4.2.18 (new clause), 7.4.1.4.2.19 (new clause), 7.4.1.4.3.2, 7.4.1.4.3.4 (new clause) |  |  |  |  |  |  |
|  |  |  | Y | N |  |  |  |  |  |
| Other specs |  |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |
| affected: |  |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |
| Other comments: |  |  | This CR does not impact the OpenAPI descriptions defined in this specification. |  |  |  |  |  |  |
| This CR's revision history: |  |  | Rev 2 (CT#100): - Changing the attribute name to "mbs5gSessionId" to avoid 3GPP Forge Lint tool errors. |  |  |  |  |  |  |

* * * * Start of changes * * * *

##### 7.4.1.4.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 6.2 apply to this API

Table 7.4.1.4.1-1 specifies the data types defined specifically for the SS_NetworkResourceAdaptation API service.

Table 7.4.1.4.1-1: SS_NetworkResourceAdaptation API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| DeliveryMode | 7.4.1.4.3.2 | Indicates the delivery mode. |  |
| MbsResource | 7.4.1.4.2.12 | Represents an MBS Resource. |  |
| MbsResourcePatch | 7.4.1.4.2.15 | Represents the parameters to request the modification of an MBS Resource. |  |
| MbsResourceReq | 7.4.1.4.2.11 | Represents the parameters to request the creation of an MBS Resource. |  |
| MbsResourceRespInfo | 7.4.1.4.2.13 | Represents NRM Server side information related to the MBS Resource. |  |
| MbsResourceResp | 7.4.1.4.2.14 | Represents a response to an MBS Resource creation/modification request. |  |
| MulticastSubscription | 7.4.1.4.2.2 | Represents a multicast subscription. |  |
| NetSysIndicator | 7.4.1.4.3.4 | Represents the network system indicator, i.e. 5GS, EPS or both. |  |
| NrmEvent | 7.4.1.4.3.3 | Indicates the NRM event. |  |
| NrmEventNotification | 7.4.1.4.2.5 | Represents a notification on an individual User Plane event. |  |
| ServiceAnnoucementMode | 7.4.1.4.3.1 | Indicates the service announcement mode. |  |
| StreamSpecification | 7.4.1.4.2.9 | Represents a stream specification that includes MAC addresses of the source and destination DS-TT ports. |  |
| TrafficSpecification | 7.4.1.4.2.10 | Represents the traffic class supported by the DS-TTs and available end-to-end maximum latency value. |  |
| TrafficSpecInformation | 7.4.1.4.2.7 | Represents the traffic class supported by the DS-TTs and available end-to-end latency value and Priority Code Point (PCP) value. |  |
| TscStreamAvailability | 7.4.1.4.2.8 | Represents the TSC stream availability information including the stream specification and list of traffic specifications. |  |
| TscStreamData | 7.4.1.4.2.6 | Represents the TSC stream data information. |  |
| UnicastSubscription | 7.4.1.4.2.3 | Represents a unicast subscription. |  |
| UserPlaneNotification | 7.4.1.4.2.4 | Represents a notification on User Plane events. |  |

Table 7.4.1.4.1-2 specifies data types re-used by the SS_NetworkResourceAdaptation API service.

Table 7.4.1.4.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| DateTime | 3GPP TS 29.571 [21] | Used to represent the subscription duration. |  |
| DurationSec | 3GPP TS 29.122 [3] | Used to represent the duration in seconds. |  |
| ExternalMbsServiceArea | 3GPP TS 29.571 [21] | Represents an MBS Service Area. |  |
| Ipv4Addr | 3GPP TS 29.571 [21] | Used to identify the IPv4 address. |  |
| Ipv6Addr | 3GPP TS 29.571 [21] | Used to identify the IPv6 address. |  |
| LocalMbmsInfo | 3GPP TS 29.486 [27] | Used to represent the local MBMS information. |  |
| MacAddr48 | 3GPP TS 29.571 [21] | Used to identify a MAC address. |  |
| MbmsLocArea | 3GPP TS 29.122 [3] | Used to indicate the requested area of the MBMS bearer. |  |
| MbsServiceInfo | 3GPP TS 29.571 [21] | Represents MBS Service Information. |  |
| MbsSessionId | 3GPP TS 29.571 [21] | Represents the identifier of an MBS Session. |  |
| Port | 3GPP TS 29.122 [3] | Used to identify the port. |  |
| SupportedFeatures | 3GPP TS 29.571 [21] | Used to negotiate the supported optional features of the API. |  |
| Uint32 | 3GPP TS 29.571 [21] | Represents an unsigned integer. |  |
| Uinteger | 3GPP TS 29.571 [21] | Represents an unsigned integer. |  |
| Uri | 3GPP TS 29.571 [21] | Used to indicate the notification URI. |  |
| ValTargetUe | Clause 7.3.1.4.2.3 | Used to identify either a VAL User ID or a VAL UE ID. |  |
| WebsockNotifConfig | 3GPP TS 29.122 [3] | Used to indicate the configuration parameters to set up notification delivery over Websocket protocol. |  |

* * * * Next changes * * * *

###### 7.4.1.4.2.5	Type: NrmEventNotification

Table 7.4.1.4.2.5-1: Definition of type NrmEventNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| event | NrmEvent | M | 1 | Event that is notified. |  |
| ts | DateTime | M | 1 | Time at which the event is observed. |  |
| deliveryMode | DeliveryMode | C | 0..1 | Indicates delivery of the user data to the UE(s) via unicast mode, EPS MBMS multicast mode, 5GS MBS multicast mode or 5GS MBS broadcast mode. This attribute shall be present if the "event" attribute is set to "UP_DELIVERY_MODE". |  |
| streamIds | array(string) | O | 1..N | Indicates the media streams (unicast or multicast) to be used. This attribute may be present if the "event" attribute is set to "UP_DELIVERY_MODE" and the NRM Server already has the streams available. |  |

* * * * Next changes * * * *

###### 7.4.1.4.2.11	Type: MBSResourceReq

Table 7.4.1.4.2.11-1: Definition of type MBSResourceReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mbsResource | MBSResource | M | 1 | Contains the parameters to request the creation of the MBS Resource. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Used to negotiate the applicability of optional features. This attribute shall be present only if feature negotiation needs to take place. |  |

* * * * Next changes * * * *

###### 7.4.1.4.2.12	Type: MBSResource

Table 7.4.1.4.2.12-1: Definition of type MBSResource

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valGroupId | string | C | 0..1 | Contains the identity of the VAL group to which the MBS Resource is related. (NOTE 1) |  |
| valUeIdsList | array(ValTargetUe) | C | 1..N | Contains the list of the identifier(s) of the VAL UE(s) to which the MBS Resource is related. (NOTE 1) |  |
| servAnmtMode | ServiceAnnoucementMode | M | 1 | Contain the MBS Service Announcement mode to be used, i.e. whether the MBS Service Announcement is delivered by the NRM Server or the VAL Server. |  |
| mbsResServInfo | MbsServiceInfo | M | 1 | Contains the MBS Service Information for the MBS Resource. |  |
| mbsResServiceArea | ExternalMbsServiceArea | O | 0..1 | Contains the Service Area of the MBS Resource. |  |
| notifUri | Uri | M | 1 | Contains the URI towards which the user plane delivery mode notifications shall be sent. |  |
| netSysInd | NetSysIndicator | O | 0..1 | Indicates whether the MBS Resource requires multicast/broadcast services from 5GS, EPS or both. |  |
| localMbmsInfo | LocalMbmsInfo | O | 0..1 | Contains the local MBMS information. This attribute may only be provided if the "netSysInd" attribute is set to "EPS" or "5GS_AND_EPS". (NOTE 2) |  |
| localMbmsActInd | boolean | O | 0..1 | Set to true by the service consumer to indicate that the local MBMS is activated. Set to false or omitted otherwise. (NOTE 2) |  |
| mbsResRespInfo | MBSResourceRespInfo | O | 0..1 | Represents the NRM Server side information related to the created MBS Resource. This attribute may only be present in the response to the request to create/modify/retrieve the MBS Resource. |  |
| NOTE 1: These attributes are mutually exclusive. Either one of them shall be present. NOTE 2: These attributes are mutually exclusive. Either one of them may be present. |  |  |  |  |  |

Editor's Note:	Whether the Service Area can also be provided in the form of a list of TAI(s) and/or a list cell ID(s) (i.e. using the MbsServiceArea data structure defined in clause 5.9.4.4 of 3GPP TS 29.571 [21] is FFS.

* * * * Next changes * * * *

###### 7.4.1.4.2.13	Type: MBSResourceRespInfo

Table 7.4.1.4.2.13-1: Definition of type MBSResourceRespInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mbs5gSessionId | MbsSessionId | O | 0..1 | Contains the identifier of the MBS session (i.e. TMGI and/or SSM, in addition to the NID in the case an SNPN). This attribute may not be present if the MBS Service Announcement is delivered by the NRM Server (i.e. the "servAnmtMode" attribute in the corresponding request is set to "NRM"). |  |
| mbmsBearerId | MbsSessionId | O | 0..1 | Contains the identifier of the MBMS bearer (i.e. TMGI). This attribute may not be present if the MBS Service Announcement is delivered by the NRM Server (i.e. the "servAnmtMode" attribute in the corresponding request is set to "NRM"). |  |
| upIpv4Addr | Ipv4Addr | C | 0..1 | Contains the user plane IPv4 address of the NRM Server to be used by the VAL Server for downlink packets delivery. (NOTE) |  |
| upIpv6Addr | Ipv6Addr | C | 0..1 | Contains the user plane IPv6 address of the NRM Server to be used by the VAL Server for downlink packets delivery. (NOTE) |  |
| upPortNum | Port | O | 0..1 | Contains the user plane UDP port number to be used by VAL Server for downlink packets delivery. |  |
| 5MbsInfo | ??? | O | 0..1 | Contains MBS related configuration information. This attribute may only be present if the network system used for multicast/broadcast services is 5GS or both 5GS and EPS. |  |
| epsMbmsInfo | ??? | O | 0..1 | Contains MBMS related configuration information. This attribute may only be present if the network system used for multicast/broadcast services is EPS or both 5GS and EPS. |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

Editor's Note:	The encoding of the "5MbsInfo" and "epsMbmsInfo" attributes is FFS.

Editor's Note:	Whether the MBS Session ID of the created MBS Session and/or the MBMS bearer ID of the created MBMS bearer may be returned to the VAL Server is FFS.

* * * * Next changes * * * *

###### 7.4.1.4.2.14	Type: MBSResourceResp

Table 7.4.1.4.2.14-1: Definition of type MBSResourceResp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mbsResource | MBSResource | M | 1 | Contains the the created MBS Resource. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Used to negotiate the applicability of optional features. This attribute shall be present only if feature negotiation is taking place (i.e. this attribute was present in the corresponding request). |  |

* * * * Next changes * * * *

###### 7.4.1.4.2.15	Type: MBSResourcePatch

Table 7.4.1.4.2.15-1: Definition of type MBSResourcePatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mbsResServInfo | MbsServiceInfo | O | 0..1 | Contains the updated MBS Service Information for the MBS Resource. |  |
| mbsResServiceArea | ExternalMbsServiceArea | O | 0..1 | Contains the updated Service Area of the MBS Resource. |  |
| notifUri | Uri | O | 0..1 | Contains the URI towards which the user plane delivery mode notifications shall be sent. |  |
| localMbmsInfo | LocalMbmsInfo | O | 0..1 | Contains the local MBMS information. This attribute may only be provided if the "netSysInd" attribute is set to "EPS" or "5GS_AND_EPS". (NOTE) |  |
| localMbmsActInd | boolean | O | 0..1 | Set to true by the service consumer to indicate that the local MBMS is activated. Set to false or omitted otherwise. (NOTE) |  |
| NOTE: These attributes are mutually exclusive. Either one of them may be present. |  |  |  |  |  |

Editor's Note:	Whether the Service Area can also be provided in the form of a list of TAI(s) and/or a list cell ID(s) (i.e. using the MbsServiceArea data structure defined in clause 5.9.4.4 of 3GPP TS 29.571 [21] is FFS.

* * * * Next changes * * * *

###### 7.4.1.4.3.2	Enumeration: DeliveryMode

Table 7.4.1.4.3.2-1: Enumeration DeliveryMode

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| UNICAST | Indicates Unicast delivery. |  |
| MULTICAST | Indicates EPS MBMS Multicast delivery. |  |
| MBS_MULTICAST | Indicates 5GS MBS Multicast delivery. |  |
| MBS_BROADCAST | Indicates 5GS MBS Broadcast delivery. |  |

* * * * Next changes * * * *

###### 7.4.1.4.3.4	Enumeration: NetSysIndicator

Table 7.4.1.4.3.4-1: Enumeration NetSysIndicator

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| 5GS | Indicates that the network system is 5GS. |  |
| EPS | Indicates that the network system is EPS. |  |
| 5GS_AND_EPS | Indicates that the network system is both 5GS and EPS. |  |

* * * * End of changes * * * *