3GPP TSG-CT Meeting #100	CP-231339

# Taipei, 12th – 13th June, 2023	(was C1-232379, C1-232961)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.558 | CR | 0031 | rev | 2 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EAS bundle information |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  | 2023-05-05 |
| Category: | B |  |  |  |  |  | Release: |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | SA6 has introduced EAS bundle information in TS 23.558 (S6-231074). As the EAS bundle information has been introduced there is need of update the AC Profile to supprot this new EAS bundle information. It is proposed to reference to the data type as defined in TS 29.558. |  |  |  |  |  |  |  |
| Summary of change: |  | Updates to the AC Profile. |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The stage-2 requirements on EAS bundle information are not supported in the specification which leads to misliagnment with stage 2. |  |  |  |  |  |  |  |
| Clauses affected: |  |  | 5.2.2.2.2, 5.2.2.3.2, 5.3.2.1, 6.2.5.1, 6.2.5.2.3, 6.2.7, 7.2.2.1, A.2 |  |  |  |  |  |  |
|  |  |  | Y | N |  |  |  |  |  |
| Other specs |  |  | X |  | Other core specifications |  |  | TS 29.558 CR0073 |  |
| affected: |  |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |
| Other comments: |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  | V1: cover sheet updated, changes to the Eees_EECRegistration API are now captured in the procedures as well. Name of the (re-)use data type changed to "EASBundleInfo". The "EdgeApp_2" feature added to the table 6.2.7-1. Editor’s notes added for the Eecs_ServiceProvisioning API and the Eees_EASDiscovery API. V2 (CT#100): Updated OpenAPI to resolve reference error to TS 29.558. |  |  |  |  |  |  |

* * * First Change * * * *

##### 5.2.2.2.2	EEC registering to EES using Eees_EECRegistration_Request operation

For an EEC to register at the EES, the EEC shall send an HTTP POST message to the EES on the "EEC Registrations" collection resource to create the resource associated to or representing the EEC. The body of the HTTP POST message shall include the EEC ID, may include UE identifier, AC Profile(s), proposed expiration time for the registration, EEC context ID obtained from a previous registration, the ACR scenario(s) supported by the EEC for service continuity, as specified in clause 6.2.2.2.3.1. If EEC context ID is included in the body of the HTTP POST message, it shall also include Source EES ID and Source EES Endpoint of the EES that provided EEC context ID.

Upon receiving the HTTP POST message from the EEC, the EES shall:

a)	Process the EEC registration request information;

b)	verify if the EEC is authorized to register itself at EES; and

c)	if the EEC is authorized to register with EES, then;

1)	if the AC Profile(s) is included in the HTTP POST message, the EES further determines whether the registered EAS(s) fulfils the requirements that were indicated in the AC Profile(s):

i)	if acSvcContSupp information is included in the AC Profile, the EEC, EES, and the matching EAS have to support ACRScenario indicated in the acSvcContSupp information; and

ii)	For each AC Profile, if eass information is included in the AC Profile, the EES identifies the matching EAS such that the matching EAS shall:

A)	be identified by the easId information; and

B)	suffice all information included in the minimumReqSvcKPIs information.

NOTE 1: With respect to expectedSvcKPIs information, it is up to the EES implementation on how to identifies the matching EAS.

iii)	For each AC Profile, if easBundleInfo information is included in the AC Profile, the EES identifies the matching list of EAS bundle to which the EAS identified by the by the easId information belongs.

When a matching EAS is identified, the EES determines that the corresponding requirements are fulfilled and are supported for the new resource.

When a matching EAS is not identified for even one AC profile, the EES shall reject the request message by sending an HTTP response to the EEC with a status code set to 404 Not Found and indicate the "RESOURCE_NOT_FOUND" error in the "cause" attribute of the "ProblemDetails" structure.

2)	if the received EEC registration request contains an EEC context ID, a source EES endpoint, the EES retrieves the EEC's context from the source EES according to the procedures specified in clause 5.10 of 3GPP TS 29.558 [4];

3)	the EES creates a new resource with the EEC registration information as specified in clause 6.2.2.1, and assigns and stores new EEC context ID;

i)	if the EES cannot reserve the necessary resources while meeting the capability requirements of the existing registered EECs, the EES shall determine the EEC Context information stale and send a failure response with a corresponding cause as specified in clause 6.2.2.2.3.1; and

ii)	Otherwise the EES shall return the EEC registration information in the response message. The response message may include expiration time to indicate when the EEC registration will automatically expire. The response message may include a newly assigned EEC context ID. The URI of the created resource shall be returned in the "Location" HTTP header. If the EEC registration request contains AC Profile(s), and the EES determines that the requirements indicated in the AC profile(s) cannot be fulfilled for some of the AC profile(s), the EES shall include "unfulfilledAcProfs" attribute containing the list of ACIDs of such AC Profile(s) and appropriate reasons as specified in clause 6.2.5.2.2.

The EEC stores the new EEC context ID and uses it when it registers with another EES.

If the expiration time is provided, then to maintain the registration, the EEC shall send a registration update request (as described in clause 5.2.2.3) prior to the expiration time. If a succesful registration update request is not received prior to the expiration time, then the EES shall treat the EEC as implicitly deregistered and remove the corresponding EEC registration resource.

* * * Next Change * * * *

##### 5.2.2.3.2	EEC updating registration information using Eees_EECRegistration_Update operation

To update the EEC registration information at the EES, the EEC shall send an HTTP PATCH request (for partial update) or HTTP PUT message (for fully replacement) to the EES on resource URI identifying the Individual EEC registration resource representation as specified in clause 6.2.2.3.3.3 for an HTTP PATCH message or in clause 6.2.2.3.3.1 for an HTTP PUT message.

The PATCH message includes the parameters (AC profiles or proposed expiry time) that need to be replaced in the existing registration information.

The PUT message shall replace all properties of the existing resource with the EEC registration information in the request. The value of the eecId provided during the EEC registration shall not be changed.

Upon receiving the HTTP PATCH or PUT message from the EEC, if the resource URI does not exist, the EES shall respond 404 Not Found error to the EEC. Otherwise, the EES shall:

a)	check the registration update message from the EEC to see if the EEC is authorized to modify the requested registration resource; and

b)	if the EEC is authorized to update the registration information and the eecId information in the request and the resource match, then the EES shall;

1)	if the AC Profile(s) is included in the HTTP PATCH or PUT message, the EES further determines whether the registered EAS(s) fulfils the requirements that were indicated in the AC Profile(s):

i)	if acSvcContSupp information is included in the AC Profile, the EEC, EES and the matching EAS have to support ACRScenario indicated in the acSvcContSupp information; and

ii)	For each AC Profile, if EAS(s) information is included in the AC Profile, the EES identifies the matching EAS such that the matching EAS shall:

A)	be identified by the easId information; and

B)	suffice all information included in the minimumReqSvcKPIs information.

NOTE 1:	With respect to expectedSvcKPIs information, it is up to the EES implementation on how to identifies the matching EAS.

iii)	For each AC Profile, if easBundleInfo information is included in the AC Profile, the EES identifies the matching list of EAS bundle to which the EAS identified by the by the easId information belongs.

When a matching EAS is identified for atleast one AC profile, the EES determines that the corresponding requirements are fulfilled and are supported and shall update the resource identified by Resource URI of the EEC registration information with the updated EEC registration information received in the HTTP PATCH or PUT request message.

2)	return the updated EEC registration information in the response. In the response message, the EES may send "200 OK" response code to provide an updated expiration time to indicate to the EEC when the updated registration will automatically expire. Otherwise, the EES sends "204 No Content" response code. If the EEC registration request contains AC Profile(s), and the EES determines that the requirements indicated in the AC profile(s) cannot be fulfilled for some of the AC profile(s), the EES shall include "unfulfilledAcProfs" attribute containing the list of ACIDs of such AC Profile(s) and appropriate reasons as specified in clause 6.2.5.2.2.

If the expiration time is provided, the EEC shall send a registration update request prior to the expiration time if the EEC wants to maintain the registration. If a successful registration update request is not received prior to the expiration time, the EES shall treat the EEC as implicitly de-registered and remove the corresponding EEC registration resource.

* * * Next Change * * * *

#### 5.3.2.1	Introduction

The service operations defined for Eees_EASDiscovery API are shown in the table 5.3.2.1-1.

Table 5.3.2.1-1: Operations of the Eees_EASDiscovery API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Eees_EASDiscovery_EasDiscRequest | This service operation is used by the EEC to request for one-time EAS discovery information. | EEC |
| Eees_EASDiscovery_Subscribe | This service operation is used by the EEC to request to subscribe to EAS discovery information reporting. | EEC |
| Eees_EASDiscovery_Notify | This service operation is used by the EES to notify a previously subscribed EEC on EAS discovery information. | EES |
| Eees_EASDiscovery_UpdateSubscription | This service operation is used by the EEC to update an existing subscription to EAS discovery information reporting. | EEC |
| Eees_EASDiscovery_Unsubscribe | This service operation is used by the EEC to delete an existing subscription to EAS discovery information reporting. | EEC |

Editor's note [CR#0031, EDGEAPP_Ph2]:	3GPP TS 23.558 [2] contains requirements on EAS bundle information initiated by the EEC. It is FFS which service operations of the Eecs_ServiceProvisoning API need to be updated.

* * * Next Change * * * *

#### 6.2.5.1	General

This clause specifies the application data model supported by the Eees_EECRegistration API.

Table 6.2.5.1-1 specifies the data types defined specifically for the Eees_EECRegistration API service.

Table 6.2.5.1-1: Eees_EECRegistration API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| EECRegistration | 6.2.5.2.2 | Describes the parameters to perform EEC Registration related operations |  |
| acProfile | 6.2.5.2.3 | Describes information about AC used to determine services and service characteristics required |  |
| EASDetail | 6.2.5.2.4 | Describes EAS along with service KPIs that serves the AC |  |
| acServiceKPIs | 6.2.5.2.5 | Describes the KPIs required by the AC in order to receive required services |  |

Table 6.2.5.1-2 specifies data types re-used by the Eees_EECRegistration API service.

Table 6.2.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| BitRate | 3GPP TS 29.571 [5] |  |  |
| DateTime | 3GPP TS 29.122 [3] |  |  |
| EndPoint | 3GPP TS 29.558 [4] |  |  |
| ScheduledCommunicationTime | 3GPP TS 29.122 [3] |  |  |
| LocationArea5G | 3GPP TS 29.122 [3] |  |  |
| Uinteger | 3GPP TS 29.571 [5] |  |  |
| DurationSec | 3GPP TS 29.122 [3] |  |  |
| SupportedFeatures | 3GPP TS 29.571 [5] |  |  |
| Gpsi | 3GPP TS 29.571 [5] |  |  |
| ACRScenario | 3GPP TS 29.558 [4] |  |  |
| easBundleInfo | 3GPP TS 29.558 [4] |  |  |

* * * Next Change * * * *

##### 6.2.5.2.3	Type: ACProfile

Table 6.2.5.2.3-1: Definition of type ACProfile

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| acId | string | M | 1 | Identity of the AC. |  |
| acType | string | O | 0..1 | The category or type of AC. |  |
| prefEcsps | array(string) | O | 1..N | Indicates to the ECS which ECSPs are preferred for the AC. The ECS may use this information in the selection of EESs. |  |
| acSchedule | ScheduledCommunicationTime | O | 0..1 | Indicates the expected operation schedule of the AC (e.g. time windows) |  |
| expAcGeoServArea | LocationArea5G | O | 0..1 | Indicates the expected location(s) (e.g. route) of the hosting UE during the AC's operation schedule. |  |
| acSvcContSupp | array(ACRScenario) | O | 0..1 | Indicates if service continuity support is required or not for the application. The ACR scenarios supported by the AC for service continuity. If this attribute is not present, then the AC does not support service continuity. |  |
| simInactTime | DurationSec | O | 0..1 | Indicates whether a simultaneous EAS connectivity in service continuity is required and the inactive time guidance for keeping connectivity towards the S-EAS. |  |
| eass | Array(EasDetail) | O | 1..N | Provides the list of EAS that serve the AC along with the service KPIs required by the AC |  |
| easBundleInfo | array(EASBundleInfo) | O | 1..N | Represents a list of EAS bundles to which the EAS (identified via the "easId" attribute) belongs. | EdgeApp_2 |

* * * Next Change * * * *

### 6.2.7	Feature negotiation

General feature negotiation procedures are described in clause 6.1. Table 6.2.7-1 lists the supported features for Eees_EECRegistration API.

Table 6.2.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | EdgeApp_2 | This feature indicates support of the enhancements for the Enabling Edge Applications. Within this feature the following enhacements are covered: - support of EAS bundle information. |

* * * Next Change * * * *

#### 7.2.2.1	Introduction

The service operation defined for Eecs_ServiceProvisioning API is shown in the table 7.2.2.1-1.

Table 7.2.2.1-1: Operations of the Eecs_ServiceProvisioning API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Eecs_ServiceProvisioning_Request | This service operation is used by the EEC to request for one-time service provisioning information. | EEC |
| Eecs_ServiceProvisioning_Subscribe | This service operation is used by the EEC to subscribe to ECS for reporting of service provisioning information. | EEC |
| Eecs_ServiceProvisioning_Notify | This service operation is used by the ECS to notify the EEC about the service provisioning information. | ECS |
| Eecs_ServiceProvisioning_UpdateSubscription | This service operation is used by the EEC to update its subscription at ECS for reporting of service provisioning information. | EEC |
| Eecs_ServiceProvisioning_Unsubscribe | This service operation is used by the EEC to remove its subscription from ECS for reporting of service provisioning information. | EEC |

Editor's note [CR#0031, EDGEAPP_Ph2]:	3GPP TS 23.558 [2] contains requirements on EAS bundle information received from the ECS. It is FFS which service operations of the Eecs_ServiceProvisoning API need to be updated.

* * * Next Change * * * *

# A.2	Eees_EECRegistration

openapi: 3.0.0

info:

title: Eees_EECRegistration

version: "1.1.0-alpha.1"

description: |

API for EEC registration.

© 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 24.558 V18.0.0 Enabling Edge Applications; Protocol specification.

url: 'https://www.3gpp.org/ftp/Specs/archive/24_series/24.558/'

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/eees-eecregistration/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.1 of 3GPP TS 24.558

paths:

/registrations:

post:

description: Create a new EEC registration at the EES.

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/EECRegistration'

responses:

'201':

description: Created (EEC information is registered successfully at EES).

content:

application/json:

schema:

$ref: '#/components/schemas/EECRegistration'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/registrations/{registrationId}:

put:

description: Update an existing EEC registration a the EES.

parameters:

- name: registrationId

in: path

description: Identifies an individual EEC registration

required: true

schema:

type: string

requestBody:

description: Parameters to replace the existing registration

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/EECRegistration'

responses:

'200':

description: OK (An individual EEC registration resource updated successfully).

content:

application/json:

schema:

$ref: '#/components/schemas/EECRegistration'

'204':

description: >

No Content (An individual EEC registration resource updated successfully).

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

description: Remove an existing ECC registration at EES.

parameters:

- name: registrationId

in: path

description: Identifies an individual EEC registration

required: true

schema:

type: string

responses:

'204':

description: >

No Content (An individual EEC registration resource deleted successfully).

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

description: partially update an existing EEC registration a the EES.

parameters:

- name: registrationId

in: path

description: Identifies an individual EEC registration

required: true

schema:

type: string

requestBody:

description: Parameters to replace the existing registration

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/EECRegistrationPatch'

responses:

'200':

description: OK (An individual EEC registration resource updated successfully).

content:

application/json:

schema:

$ref: '#/components/schemas/EECRegistration'

'204':

description: >

No Content (An individual EEC registration resource updated successfully).

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

EECRegistration:

description: Describes the parameters to perform EEC Registration related operations.

type: object

properties:

eecId:

type: string

description: Represents a unique identifier of the EEC.

ueId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

acProfs:

type: array

items:

$ref: '#/components/schemas/ACProfile'

description: Profiles of ACs for which the EEC provides edge enabling services.

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

eecSvcContSupp:

type: array

items:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ACRScenario'

description: Profiles of ACs for which the EEC provides edge enabling services.

eecCntxId:

type: string

description: Identifier of the EEC context obtained from a previous registration.

srcEesId:

type: string

description: Identifier of the EES that provided EEC context ID.

endPt:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

unfulfilledAcProfs:

$ref: '#/components/schemas/UnfulfilledAcProfile'

required:

- eecId

ACProfile:

description: ECS service provisioning response information.

type: object

properties:

acId:

type: string

description: Identity of the AC.

acType:

type: string

description: The category or type of AC.

prefEcsps:

type: array

items:

type: string

description: Indicates to the ECS which ECSPs are preferred for the AC.

acSchedule:

$ref: 'TS29122_CpProvisioning.yaml#/components/schemas/ScheduledCommunicationTime'

expAcGeoServArea:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

acSvcContSupp:

type: array

items:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ACRScenario'

description: Profiles of ACs for which the EEC provides edge enabling services.

simInactTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

eass:

type: array

items:

$ref: '#/components/schemas/EasDetail'

minItems: 1

description: List of EAS information.

easBundleInfo:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EASBundleInfo'

required:

- acId

EasDetail:

description: EAS details.

type: object

properties:

easId:

type: string

description: Application identifier of the EAS.

expectedSvcKPIs:

$ref: '#/components/schemas/ACServiceKPIs'

minimumReqSvcKPIs:

$ref: '#/components/schemas/ACServiceKPIs'

required:

- easId

ACServiceKPIs:

description: EAS details.

type: object

properties:

connBand:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

reqRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

respTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

avail:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

reqComp:

type: string

description: The compute resources required by the AC.

reqGrapComp:

type: string

description: The graphical compute resources required by the AC.

reqMem:

type: string

description: The memory resources required by the AC.

reqStrg:

type: string

description: The storage resources required by the AC.

EECRegistrationPatch:

description: Describes the parameters to perform EEC Registration update.

type: object

properties:

acProfs:

type: array

items:

$ref: '#/components/schemas/ACProfile'

description: Profiles of ACs for which the EEC provides edge enabling services.

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

unfulfilledAcProfs:

$ref: '#/components/schemas/UnfulfilledAcProfile'

UnfulfilledAcProfile:

description: Desrcibes AC Profile ID and reason sent by EES in EEC Register response.

type: object

properties:

acId:

type: string

description: The AC ID of a AC profile.

reason:

$ref: '#/components/schemas/UnfulfillACProfRsn'

UnfulfillACProfRsn:

anyOf:

- type: string

enum:

- EAS_NOT_AVAILABLE

- REQ_UNFULFILLED

description: represents reason for unfulfilled AC profile requirements.

* * * End of Changes * * * *