3GPP TSG-CT WG4 Meeting #117	C4-233745

# Göteborg, Sweden, 21st– 25th August 2023										Revision of C4-233476

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.562 | CR | 0134 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update OpenAPI for Scscf Registration and Scscf Restoration Information Resource URI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2023-08-23 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Figure 6.1.3.1-1 and Table 6.1.3.1-1 of TS 29.562 shows that Scscf Registration and Scscf Restoration Information Resource URI contain the IMS UE's identity (/{imsUeId}), which shall be an IMPI or an IMPU. But the OpenAPI definition in Annex A.2 uses IMPU (/{impu}). Both are not consistent. |  |  |  |  |  |  |  |  |
| Summary of change: |  | OpenAPI definition for Scscf Registration and Scscf Restoration Information Resource URI in Annex A.2 is not consistent with Figure 6.1.3.1-1 and Table 6.1.3.1-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent OpenAPI definition may lead to different implementations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible corrections to the OpenAPI file of Nhss_imsUECM API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * First Change * * * *

A.2	Nhss_imsUECM API

openapi: 3.0.0

info:

version: '1.2.0-alpha.1'

title: 'Nhss_imsUECM'

description: |

Nhss UE Context Management Service for IMS.

© 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.562 Home Subscriber Server (HSS) Services, version 18.0.0

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.562/'

servers:

- url: '{apiRoot}/nhss-ims-uecm/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501.

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-uecm

paths:

/{impu}/authorize:

post:

summary: >

Authorize IMS Identities to register in the network or establish multimedia sessions

and return CSCF location if it is stored

operationId: Authorize

tags:

- Authorize (Custom Operation)

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-uecm

- oAuth2ClientCredentials:

- nhss-ims-uecm

- nhss-ims-uecm:authorize:invoke

parameters:

- name: impu

in: path

description: Public identity of the user.

required: true

schema:

$ref: '#/components/schemas/Impu'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/AuthorizationRequest'

required: true

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/AuthorizationResponse'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

deregistrationeNotification:

'{request.body#/deregCallbackUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/DeregistrationData'

responses:

'204':

description: Successful Notification response

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/{imsUeId}/scscf-registration:

put:

summary: SCSCF registration information

operationId: SCSCF registration

tags:

- S-CSCF Registration (Document)

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-uecm

- oAuth2ClientCredentials:

- nhss-ims-uecm

- nhss-ims-uecm:registration:create

parameters:

- name: imsUeId

in: path

description: IMS Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/ScscfRegistration'

required: true

responses:

'201':

description: Created

content:

application/json:

schema:

$ref: '#/components/schemas/ScscfRegistration'

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure:

{apiRoot}/nhss-ims-uecm/v1/{imsUeId}/scscf-registration

required: true

schema:

type: string

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/ScscfRegistration'

'204':

description: No content

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

description: Forbidden

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ExtendedProblemDetails'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/{imsUeId}/scscf-registration/scscf-restoration-info:

put:

summary: Update the S-CSCF restoration information of the UE

operationId: UpdateScscfRestorationInfo

tags:

- S-CSCF Restoration Information (Document)

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-uecm

- oAuth2ClientCredentials:

- nhss-ims-uecm

- nhss-ims-uecm:restoration:modify

parameters:

- name: imsUeId

in: path

description: IMS Identity.

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/ScscfRestorationInfoRequest'

required: true

responses:

'201':

description: Created

content:

application/json:

schema:

$ref: '#/components/schemas/ScscfRestorationInfoResponse'

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure:

{apiRoot}/nhss-ims-uecm/v1/{imsUeId}/scscf-registration/scscf-restoration-info

required: true

schema:

type: string

'200':

description: OK

content:

application/json:

schema:

$ref: '#/components/schemas/ScscfRestorationInfoResponse'

'204':

description: No content

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

get:

summary: Retrieve the S-CSCF restoration information of the UE

operationId: GetScscfRestorationInfo

tags:

- S-CSCF Restoration Information (Document)

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-uecm

- oAuth2ClientCredentials:

- nhss-ims-uecm

- nhss-ims-uecm:restoration:read

parameters:

- name: impu

in: path

description: Public identity of the user.

required: true

schema:

$ref: '#/components/schemas/Impu'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/ScscfRestorationInfoResponse'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Delete the S-CSCF restoration information of the UE

operationId: DeleteScscfRestorationInfo

tags:

- S-CSCF Restoration Information (Document)

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-uecm

- oAuth2ClientCredentials:

- nhss-ims-uecm

- nhss-ims-uecm:restoration:modify

parameters:

- name: impu

in: path

description: Public identity of the user.

required: true

schema:

$ref: '#/components/schemas/Impu'

responses:

'204':

description: No content

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nhss-ims-uecm: Access to the Nhss IMS UE Context Management API

nhss-ims-uecm:authorize:invoke: Access to invoke the Authorize custom operation

nhss-ims-uecm:registration:create: Access to create the S-CSCF Registration resource

nhss-ims-uecm:restoration:read: Access to read the S-CSCF Restoration resource

nhss-ims-uecm:restoration:modify: >-

Access to create/update/delete the S-CSCF Restoration resource

schemas:

#

# COMPLEX TYPES:

#

AuthorizationRequest:

description: Ims authorization request data

type: object

required:

- authorizationType

properties:

impi:

$ref: '#/components/schemas/Impi'

authorizationType:

$ref: '#/components/schemas/AuthorizationType'

visitedNetworkIdentifier:

type: string

emergencyIndicator:

type: boolean

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

AuthorizationResponse:

description: Ims Registration authorization information result

type: object

properties:

authorizationResult:

$ref: '#/components/schemas/AuthorizationResult'

cscfServerName:

type: string

scscfSelectionAssistanceInfo:

$ref: 'TS29562_Nhss_imsSDM.yaml#/components/schemas/ScscfSelectionAssistanceInformation'

required:

- authorizationResult

oneOf:

- required:

- cscfServerName

- required:

- scscfSelectionAssistanceInfo

ScscfRegistration:

description: Scscf Registration

type: object

required:

- imsRegistrationType

- cscfServerName

properties:

impi:

$ref: '#/components/schemas/Impi'

imsRegistrationType:

$ref: '#/components/schemas/ImsRegistrationType'

cscfServerName:

type: string

scscfInstanceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

deregCallbackUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

associatedImpis:

type: array

items:

$ref: '#/components/schemas/Impi'

associatedRegisteredImpis:

type: array

items:

$ref: '#/components/schemas/Impi'

irsImpus:

type: array

items:

$ref: '#/components/schemas/Impu'

minItems: 1

uniqueItems: true

wildcardedPui:

$ref: '#/components/schemas/Impu'

looseRouteIndicator:

$ref: '#/components/schemas/LooseRouteIndication'

wildcardedPsi:

$ref: '#/components/schemas/Impu'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

multipleRegistrationIndicator:

type: boolean

pcscfRestorationIndicator:

type: boolean

default: false

scscfReselectionIndicator:

type: boolean

default: false

ExtendedProblemDetails:

description: >

Extension of the ProblemDetails data type, to include additional information

in an error response message

allOf:

- $ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

- $ref: '#/components/schemas/AdditionalInfo'

AdditionalInfo:

description: >

Additional information to be sent on error response messages, along with the

common content in ProblemDetails

type: object

properties:

scscfServerName:

type: string

ScscfRestorationInfo:

description: S-CSCF restoration information

type: object

properties:

userName:

$ref: '#/components/schemas/Impi'

restorationInfo:

type: array

items:

$ref: '#/components/schemas/RestorationInfo'

registrationTimeOut:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

sipAuthenticationScheme:

$ref: 'TS29562_Nhss_imsUEAU.yaml#/components/schemas/SipAuthenticationScheme'

ScscfRestorationInfoRequest:

description: S-CSCF restoration information request

type: object

properties:

scscfRestorationInfoRequest:

$ref: '#/components/schemas/ScscfRestorationInfo'

ScscfRestorationInfoResponse:

description: S-CSCF restoration information response

type: object

properties:

scscfRestorationInfoResponse:

type: array

items:

$ref: '#/components/schemas/ScscfRestorationInfo'

RestorationInfo:

description: >

The information relevant to a specific registration required for an S-CSCF

to handle the requests for a user

type: object

required:

- path

- contact

properties:

path:

type: string

contact:

type: string

initialCSeqSequenceNumber:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

callIdSipHeader:

type: string

uesubscriptionInfo:

$ref: '#/components/schemas/UeSubscriptionInfo'

pcscfSubscriptionInfo:

$ref: '#/components/schemas/PcscfSubscriptionInfo'

imsSdmSubscriptions:

description: >

A map (list of key-value pairs where subscriptionId serves as key) of ImsSdmSubscription

type: object

additionalProperties:

$ref: 'TS29562_Nhss_imsSDM.yaml#/components/schemas/ImsSdmSubscription'

UeSubscriptionInfo:

description: Subscription information of the UE for the SIP Registration State event

type: object

required:

- callIdSipHeader

- fromSipHeader

- toSipHeader

- recordRoute

- contact

properties:

callIdSipHeader:

type: string

fromSipHeader:

type: string

toSipHeader:

type: string

recordRoute:

type: string

contact:

type: string

PcscfSubscriptionInfo:

description: Subscription information of the P-CSCF for the SIP Registration State event

type: object

required:

- callIdSipHeader

- fromSipHeader

- toSipHeader

- contact

properties:

callIdSipHeader:

type: string

fromSipHeader:

type: string

toSipHeader:

type: string

contact:

type: string

DeregistrationData:

description: Data related to the de-registration information of a S-CSCF in HSS

type: object

required:

- deregReason

- impi

properties:

deregReason:

$ref: '#/components/schemas/DeregistrationReason'

impi:

$ref: '#/components/schemas/Impi'

associatedImpis:

type: array

items:

$ref: '#/components/schemas/Impi'

emergencyRegisteredIdentities:

type: array

items:

$ref: '#/components/schemas/EmergencyRegisteredIdentity'

minItems: 1

EmergencyRegisteredIdentity:

description: >

A pair of private and public user identities which have not been de-registered

due to emergency registration

type: object

required:

- impi

- impu

properties:

impi:

$ref: '#/components/schemas/Impi'

impu:

$ref: '#/components/schemas/Impu'

DeregistrationReason:

description: >

Contains the reason for the network initiated de-registration (including a reason code,

and a human-readable reason text)

type: object

required:

- reasonCode

- reasonText

properties:

reasonCode:

$ref: '#/components/schemas/DeregistrationReasonCode'

reasonText:

type: string

#

# SIMPLE TYPES

#

ImsUeId:

description: IMS UE Identity (IMPU or IMPI)

type: string

pattern: '^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}|impu-tel\:\+[0-9]{5,15}|impi-.+|.+)$'

Impu:

description: IMS Public Identity of the UE (sip URI or tel URI)

type: string

pattern: '^(sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}|tel\:\+[0-9]{5,15})$'

Impi:

description: IMS Private Identity of the UE

type: string

#

# ENUMS:

#

AuthorizationType:

description: Represents the type of authorization requested by the UE

anyOf:

- type: string

enum:

- REGISTRATION

- DEREGISTRATION

- type: string

AuthorizationResult:

description: Represents the details of the granted authorization to the UE

anyOf:

- type: string

enum:

- FIRST_REGISTRATION

- SUBSEQUENT_REGISTRATION

- type: string

ImsRegistrationType:

description: Represents the type of registration associated to the REGISTER request

anyOf:

- type: string

enum:

- INITIAL_REGISTRATION

- RE_REGISTRATION

- TIMEOUT_DEREGISTRATION

- USER_DEREGISTRATION

- ADMINISTRATIVE_DEREGISTRATION

- AUTHENTICATION_FAILURE

- AUTHENTICATION_TIMEOUT

- UNREGISTERED_USER

- type: string

LooseRouteIndication:

description: Indicates whether the loose routing mechanism is required to serve the user

anyOf:

- type: string

enum:

- LOOSE_ROUTE_NOT_REQUIRED

- LOOSE_ROUTE_REQUIRED

- type: string

DeregistrationReasonCode:

description: Indicates the reason for the network-initiated deregistration

anyOf:

- type: string

enum:

- PERMANENT_TERMINATION

- NEW_SERVER_ASSIGNED

- REMOVE_S-CSCF

- SERVER_CHANGE

- type: string

# * * * End of Changes * * * *